27.	Testing of the SIM/ME interface

The following sequence of tests confirms:

a)	the correct interpretation of data read from the SIM (Subscriber Identification Module) by the ME;

b)	the correct writing of data to the SIM by the ME;

c)	the initiation of appropriate procedures by the ME;

d)	low level protocols;

e)	electrical characteristics;

f)	physical characteristics.

All tests apply to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

A SIM simulator will be required as part of the SS. Alternatively, to perform the logical tests, SIMs programmed with specific data may be used. The SIM data is not defined within the initial conditions of the tests unless it differs from the default values defined below

Definition of default values for SIM/ME interface testing

A SIM containing the following default values is used for all tests of this section unless otherwise stated.

For each data item, the logical default values and the coding within the elementary files (EF) of the SIM follow.

Note :	Bx represents Byte x of the coding.

Note :	Unless otherwise defined, the coding values are hexadecimal.

EFIMSI (IMSI)

Logically:	246813579

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9

05	29	64	18	53	97	FF	FF	FF

EFLOCI (Location Information)

Logically:	LAI-MCC:	246

LAI-MNC:	81

TMSI:		'FF .. FF'

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11

FF	FF	FF	FF	42	F6	18	xx	xx	FF	00

EFKc (Ciphering Key Kc)

Logically:	Key Kc:		xx

Sequence No:	1

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9

xx	xx	xx	xx	xx	xx	xx	xx	01

EFACC (Access Control Class)

Logically:	One and only one access class from 0 - 9, e.g. class 7 for which the coding is '00 80'.

EFFPLMN (Forbidden PLMNs)

Logically:	PLMN1:	234 02 (MCC MNC)

PLMN2:	234 03

PLMN3:	234 04

PLMN4:	234 05

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12

32	F4	20	32	F4	30	32	F4	40	32	F4	50

EFSST (SIM Service Table)

Logically:	CHV1 disable function allocated and activated.

Abbreviated dialling numbers allocated and activated.

PLMN selector allocated and activated.

Fixed dialling numbers not activated.

Coding:	B1		B2		B3		B4

xx0x1111	0011xxxx	xxxxxxxx	0000xxxx (binary)

The coding of EFSST shall conform with the capabilities of the SIM used.

EFADN (Abbreviated Dialling Number)

Logically:

At least 10 records.

Record 1:

Length of alpha identifier:	32 characters

Alpha identifier:		"ABCDEFGHIJKLMNOPQRSTUVWXYZABCDEF"

Length of BCD number:	'03'

TON and NPI:		Telephony and Unknown

Dialled number:		123

CCI:				None

Ext1:				None

Coding:

Record 1:

B1	B2	B3	...	B32	B33	B34	B35	B36	B37

41	42	43	...	46	03	81	21	F3	FF

B38	B39	...	B46

FF	FF	...	FF

EFPhase

Logically:	Phase 2

Coding:	'02'

EFPLMNsel (PLMN Selector)

Logically:	1st PLMN:	234 01 (MCC MNC)

2nd PLMN:	234 02

3rd PLMN:	234 03

4th PLMN:	234 04

5th PLMN:	234 05

6th PLMN:	234 06

7th PLMN:	246 81

8th PLMN:	246 82

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B0	B10	B11	B12

32	F4	10	32	F4	20	32	F4	30	32	F4	40

B13	B14	B15	B16	B17	B18	B19	B20	B21	B22	B23	B24

32	F4	50	32	F4	60	42	F6	18	42	F6	28

CHV1 (PIN)

Logically:	2468

Coding:	B1	B2	B3	B4	B5	B6	B7	B8

32	34	36	38	FF	FF	FF	FF

CHV2 (PIN2)

Logically:	3579

Coding:	B1	B2	B3	B4	B5	B6	B7	B8

33	35	37	39	FF	FF	FF	FF

Unblock CHV1 (PUK)

Logically:	13243546

Coding:	B1	B2	B3	B4	B5	B6	B7	B8

31	33	32	34	33	35	34	36

Unblock CHV2 (PUK2)

Logically:	08978675

Coding:	B1	B2	B3	B4	B5	B6	B7	B8

30	38	39	37	38	36	37	35

Definition of FDN SIM

Some test cases require a different configuration than the one described above.  For that purpose a default FDN SIM is defined.  In general the values of the FDN SIM are identical to the default SIM, with the following exceptions:

EFSST (SIM Service Table)

Logically:	CHV1 disable function allocated and activated.

Abbreviated dialling numbers allocated and activated.

PLMN selector allocated and activated.

Fixed dialling numbers allocated and activated.

Advice of Charge allocated and activated.

Coding:	B1		B2		B3		B4

xx111111	0011xx11	xxxxxxxx	0000xxxx (binary)

The coding of EFSST shall conform with the capabilities of the SIM used.

EFFDN (Fixed Dialling Numbers)

Logically:

Record 1:

Length of alpha identifier:	6 characters

Alpha identifier:		"FDN111"

Length of BCD number:	'06'

TON and NPI:		Telephony and International

Dialled number:		+1357924680

CCI:				None

Ext1:				None

Coding for record 1:

B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12	B13

46	44	4E	31	31	31	06	91	31	75	29	64	08

B14	B15	B16	B17	B18	B19	B20

FF	FF	FF	FF	FF	FF	FF

Record 2:

Length of alpha identifier:	6 characters

Alpha identifier:		"FDN222"

Length of BCD number:	'04'

TON and NPI:		Telephony and Unknown

Dialled number:		+24680

CCI:				None

Ext1:				None

Coding for record 2:

B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12	B13

46	44	4E	32	32	32	04	81	42	86	F0	FF	FF

B14	B15	B16	B17	B18	B19	B20

FF	FF	FF	FF	FF	FF	FF

Record 3:

Length of alpha identifier:	6 characters

Alpha identifier:		"FDN333"

Length of BCD number:	'0B'

TON and NPI:		Telephony and International

Dialled number:		+12345678901234567890

CCI:				None

Ext1:				None

Coding for record 3:

B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12	B13

46	44	4E	33	33	33	0B	91	21	43	65	87	09

B14	B15	B16	B17	B18	B19	B20

21	43	65	87	09	FF	FF

27.1	MS identification by short IMSI

27.1.1	Definition and applicability

The IMSI is used for unique identification of the MS by a GSM network. The IMSI is stored in the SIM and read during the SIM/ME initialisation procedure.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.1.2	Conformance requirement

On the receipt of an IMMEDIATE ASSIGNMENT message the MS shall send PAGING RESPONSE containing the IMSI of the SIM.

TS GSM 11.11, clause 11.2.1 and 11.4.2, TS GSM 04.08, clause 10.5.1.4.

27.1.3	Test purpose

1)	To verify that the ME uses the IMSI of the SIM.

2)	To verify that the ME can handle an IMSI of less than the maximum length.

27.1.4	Method of test

27.1.4.1	Initial conditions

The SS transmits on the BCCH, with the following network parameters:

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	246/81/value in SIM

Access control:		unrestricted.

The default SIM is installed into the ME and the MS is powered on.

27.1.4.2	Procedure

a)	The SS sends PAGING REQUEST to the MS using the IMSI stored in the SIM.

b)	After receipt of a CHANNEL REQUEST from the MS, the SS sends IMMEDIATE ASSIGNMENT to the MS.

c)	After receipt of a PAGING RESPONSE from the MS, the SS sends CHANNEL RELEASE to the MS.

27.1.5	Test requirement

After step b) the MS shall send PAGING RESPONSE to the SS containing the IMSI stored in the SIM.

27.2	MS identification by short TMSI

27.2.1	Definition and applicability

The TMSI is temporarily used for identification of the MS by a GSM network. It will have been previously assigned by the network. The TMSI is stored in the SIM by the ME and read during the SIM/ME initialisation procedure.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.2.2	Conformance requirement

On the receipt of an IMMEDIATE ASSIGNMENT message the MS shall send PAGING RESPONSE containing the TMSI stored in the SIM.

TS GSM 11.11, clause 11.2.1 and 11.4.5, TS GSM 04.08, clause 10.5.1.4.

27.2.3	Test purpose

1)	To verify that the ME uses the TMSI stored in the SIM.

2)	To verify that the ME can handle a TMSI of less than maximum length.

27.2.4	Method of test

27.2.4.1	Initial conditions

The SS transmits on the BCCH, with the following network parameters:

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	246/81/value in SIM

Access control:		unrestricted.

The default SIM is used with the following exception:

EFLOCI (Location Information)

Logically:	LAI-MCC:	246

LAI-MNC:	81

TMSI:		'2143'

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11

00	00	21	43	42	F6	18	xx	xx	FF	00

The SIM is installed into the ME and the MS is powered on.

27.2.4.2	Procedure

a)	The SS sends PAGING REQUEST to the MS using the TMSI stored in the SIM.

b)	After receipt of a CHANNEL REQUEST from the MS, the SS sends IMMEDIATE ASSIGNMENT to the MS.

c)	After receipt of a PAGING RESPONSE from the MS, the SS sends CHANNEL RELEASE to the MS.

27.2.5	Test requirement

After step b) the MS shall send PAGING RESPONSE to the SS containing the TMSI stored in the SIM.

27.3	MS identification by long TMSI

27.3.1	Definition and applicability

The TMSI is temporarily used for identification of the MS by a GSM network. It will have been previously assigned by the network. The TMSI is stored in the SIM by the ME and read during the SIM/ME initialisation procedure.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.3.2	Conformance requirement

On the receipt of an IMMEDIATE ASSIGNMENT message the MS shall send PAGING RESPONSE containing the correct TMSI stored in the SIM.

TS GSM 11.11, clause 11.2.1 and 11.4.5, TS GSM 04.08, clause 10.5.1.4.

27.3.3	Test purpose

1)	To verify that the ME uses the TMSI stored in the SIM.

2)	To verify that the ME can handle a TMSI of maximum length.

3)	To verify that the ME does not respond to page requests containing a previous TMSI.

27.3.4	Method of test

27.3.4.1	Initial conditions

Prior to this test, the ME shall have been operated with a SIM containing TMSI '2143'. This may be achieved by executing the previous test (27.2) prior to this test. Only under this condition will test purpose 3) be verified.

The SS transmits on the BCCH, with the following network parameters:

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	246/81/value in SIM

Access control:		unrestricted.

The default SIM is used with the following exception:

EFLOCI (Location Information)

Logically:	LAI-MCC:	246

LAI-MNC:	81

TMSI:		'21430000'

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11

21	43	00	00	42	F6	18	xx	xx	FF	00

The SIM is installed into the ME and the MS is powered on.

27.3.4.2	Procedure

a)	The SS sends PAGING REQUEST to the MS using the TMSI '2143'.

b)	The SS sends PAGING REQUEST to the MS using the TMSI stored in the SIM.

c)	After receipt of a CHANNEL REQUEST from the MS, the SS sends IMMEDIATE ASSIGNMENT to the MS.

d)	After receipt of a PAGING RESPONSE from the MS, the SS sends CHANNEL RELEASE to the MS.

27.3.5	Test requirement

1)	After step a) the MS shall not respond to the PAGING REQUEST.

2)	After step c) the MS shall send PAGING RESPONSE to the SS containing the TMSI stored in the SIM.

27.4	MS identification by long IMSI, TMSI updating and cipher key sequence number assignment

27.4.1	Definition and applicability

The IMSI and TMSI are used for identification of the MS by a GSM network. They are read from the SIM during the SIM/ME initialisation procedure. Within the authentication procedure the network sends a cipher key sequence number to the MS. In addition the network may allocate a new TMSI to the MS. Cipher key sequence number and TMSI are stored in the SIM after call termination and/or at GSM session termination.

Test purpose 2) will only be verified if this test sequentially follows the previous test (27.3).

The test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.4.2	Conformance requirement

1.	On the receipt of an IMMEDIATE ASSIGNMENT message the MS shall send PAGING RESPONSE containing the correct IMSI stored in the SIM.

TS GSM 11.11, clauses 11.2.1 and 11.4.2, TS GSM 04.08, clause 10.5.1.4.

2.	After call termination the SIM shall contain the cipher key sequence number and TMSI received by the MS during the authentication and TMSI reallocation procedures.

TS GSM 11.11, clauses 11.2.2, 11.4.5 and 11.4.6, TS GSM 02.17, clause 6.1.

27.4.3	Test purpose

1)	To verify that the ME uses the IMSI stored in the SIM.

2)	To verify that the ME does not respond to page requests containing a previous IMSI.

3)	To verify that the ME can handle an IMSI of maximum length.

4)	To verify that the ME correctly updates the cipher key sequence number at call termination.

5)	To verify that the ME correctly updates the TMSI at call termination.

27.4.4	Method of test

27.4.4.1	Initial conditions

Prior to this test, the ME shall have been operated with a SIM containing IMSI '246813579'. This may be achieved by executing the previous test (27.3) prior to this test. Only under this condition will test purpose 2) be verified.

The SS transmits on the BCCH, with the following network parameters:

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	246/81/value in SIM

Access control:		unrestricted.

The default SIM is used with the following exception:

EFIMSI (IMSI)

Logically:	246811111111111

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9

08	29	64	18	11	11	11	11	11

The SIM is installed into the ME and the MS is powered on.

27.4.4.2	Procedure

a)	The SS sends PAGING REQUEST to the MS using the IMSI '246813579'.

b)	The SS sends PAGING REQUEST to the MS using the IMSI stored in the SIM.

c)	After receipt of a CHANNEL REQUEST from the MS, the SS sends IMMEDIATE ASSIGNMENT to the MS.

d)	After receipt of a PAGING RESPONSE from the MS, the SS sends AUTHENTICATION REQUEST to the MS containing cipher key sequence number set to binary 010.

e)	After receipt of AUTHENTICATION RESPONSE from the MS, the SS sends TMSI REALLOCATION to the MS containing TMSI '32547698'.

f)	Within 5 seconds after receipt of TMSI REALLOCATION COMPLETE from the MS, the SS sends CHANNEL RELEASE to the MS.

g)	To allow examination of the values in the SIM after call termination the MS shall not be soft powered down. If the test is performed with a SIM simulator, the simulation is stopped. If the test is performed with a SIM, the SIM is removed without soft powering down the MS. If this is not possible, the power supply of the ME is removed and then the SIM removed.

27.4.5	Test requirement

1)	After step a) the MS shall not respond to the PAGING REQUEST.

2)	After step c) the MS shall send PAGING RESPONSE to the SS containing the IMSI stored in the SIM.

3)	After step e) the MS shall send TMSI REALLOCATION COMPLETE to the SS.

4)	After step g) the SIM shall contain the following values:

EFLOCI (Location Information)

Logically:	LAI-MCC:	246

LAI-MNC:	81

TMSI:		'32547698'

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11

32	54	76	98	42	F6	18	xx	xx	xx	xx

EFKc (Ciphering Key Kc)

Logically:	Key Kc:		xx (result of the authentication algorithm)

Sequence No:	2

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9

xx	xx	xx	xx	xx	xx	xx	xx	02

27.5	Forbidden PLMNs, Location updating and undefined cipher key

27.5.1	Definition and applicability

A list of forbidden PLMNs stored in the SIM and providing storage for up to 4 entries is managed by the MS. In automatic PLMN selection mode the MS controls location updating attempts to appropriate networks with respect to this list of forbidden PLMNs. As a result of a location update reject with the cause "PLMN not allowed" the MS stores the PLMN which rejected the update request in the SIM.

After a location update, which is not followed by an authentication procedure, the cipher key sequence number indicates that the cipher key is undefined.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.5.2	Conformance requirement

1.	In automatic PLMN selection mode the MS shall only attempt a LOCATION UPDATE if it receives a BCCH containing a LAI that is not indicated in the EFFPLMN in the SIM.

TS GSM 02.11, clause 2.3, TS GSM 11.11, Clauses 11.2.1 and 11.4.8.

2.	After receipt of a LOCATION UPDATE REJECT message with the cause "PLMN not allowed" the ME shall update the EFFPLMN in the SIM.

TS GSM 02.11, clause 2.3, TS GSM 11.11, Clauses 11.2.1 and 11.4.8.

3.	After call termination the SIM shall contain the correct cipher key sequence number.

TS GSM 11.11, clauses 11.2.2, 11.4.5 and 11.4.6, TS GSM 02.17, clause 6.1.

4.	After call termination the SIM shall contain the correct TMSI and location information received by the MS.

TS GSM 11.11, clauses 11.2.2, 11.4.5 and 11.4.6, TS GSM 02.17, clause 6.1.

27.5.3	Test purpose

1)	To verify that in automatic PLMN selection mode the MS does not attempt to access PLMNs stored in EFFPLMN on the SIM.

2)	To verify that the EFFPLMN is correctly updated by the ME after receipt of a LOCATION UPDATE REJECT message with cause "PLMN not allowed".

3)	To verify that the EFKc has been correctly updated by the ME.

4)	To verify that the EFLOCI has been correctly updated by the ME.

27.5.4	Method of test

27.5.4.1	Initial conditions

The SS transmits on the BCCH, with the following network parameters:

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	234/02/0000

Access control:		unrestricted.

The default SIM is used with the following exception:

EFIMSI (IMSI)

Logically:	246811111111111

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9

08	29	64	18	11	11	11	11	11

EFLOCI (Location Information)

Logically:	LAI-MCC:	234

LAI-MNC:	01

TMSI:	'32547698'

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11

32	54	76	98	32	F4	10	00	00	FF	00

The SIM is installed into the ME and the MS is set to automatic PLMN selection mode.

EFKc (Ciphering Key Kc)

Logically:	Key Kc:		undefined

Sequence No:	2

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9

xx	xx	xx	xx	xx	xx	xx	xx	02

27.5.4.2	Procedure

a)	The MS is powered on.

b)	The SS stops all RF output on the BCCH for a long enough period of time to cause a cell reselection procedure in the MS. The BCCH is changed to contain:

LAI (MCC/MNC):	234/03

The SS then resumes RF output on the BCCH.

c)	The SS stops all RF output on the BCCH for a long enough period of time to cause a cell reselection procedure in the MS. The BCCH is changed to contain:

LAI (MCC/MNC):	234/04

The SS then resumes RF output on the BCCH.

d)	The SS stops all RF output on the BCCH for a long enough period of time to cause a cell reselection procedure in the MS. The BCCH is changed to contain:

LAI (MCC/MNC):	234/05

The SS then resumes RF output on the BCCH.

e)	The SS stops all RF output on the BCCH for a long enough period of time to cause a cell reselection procedure in the MS. The BCCH is changed to contain:

LAI (MCC/MNC):	234/01

The SS then resumes RF output on the BCCH.

f)	After receipt of a CHANNEL REQUEST from the MS, the SS sends IMMEDIATE ASSIGNMENT to the MS.

g)	After receipt of a LOCATION UPDATE REQUEST from the MS, the SS sends LOCATION UPDATE REJECT to the MS with cause "PLMN Not Allowed", followed by CHANNEL RELEASE.

The SS stops all RF output on the BCCH for a long enough period of time to cause a cell reselection procedure in the MS. The BCCH is changed to contain:

LAI (MCC/MNC):	234/06

The SS then resumes RF output on the BCCH.

h)	After receipt of a CHANNEL REQUEST from the MS, the SS sends IMMEDIATE ASSIGNMENT to the MS.

i)	After receipt of a LOCATION UPDATE REQUEST from the MS, the SS sends LOCATION UPDATE ACCEPT with:

LAI (MCC/MNC):	234/06

TMSI:		'43658709'

to the MS.

j)	After receipt of a TMSI REALLOCATION COMPLETE from the MS, the SS sends CHANNEL RELEASE to the MS.

k)	The MS is soft powered down.

27.5.5	Test requirement

1)	After each of the steps a) to d) the MS shall not attempt a LOCATION UPDATE.

2)	After step f) the MS shall send LOCATION UPDATE REQUEST to the SS.

3)	After step h) the MS shall send LOCATION UPDATE REQUEST to the SS.

4)	After step i) the MS shall respond with TMSI REALLOCATION COMPLETE.

5)	After step k) the SIM shall contain the following values:

EFLOCI (Location Information)

Logically:	LAI-MCC:	234

LAI-MNC:	06

TMSI:		'43658709'

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11

43	65	87	09	32	F4	60	xx	xx	xx	00

EFKc (Ciphering Key Kc)

Logically:	Key Kc:		xx

Sequence No:	7

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9

xx	xx	xx	xx	xx	xx	xx	xx	07

EFFPLMN (Forbidden PLMNs)

Logically:	PLMN1:	234 03 (MCC MNC)

PLMN2:	234 04

PLMN3:	234 05

PLMN4:	234 01

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12

32	F4	30	32	F4	40	32	F4	50	32	F4	10

27.6	MS updating forbidden PLMNs

27.6.1	Definition and applicability

A list of forbidden PLMNs stored in the SIM provides storage for up to 4 entries, and is managed by the MS. In automatic PLMN selection mode the MS controls location updating attempts to appropriate networks with respect to this list of forbidden PLMNs. As a result of a location update reject with the cause "PLMN not allowed" the MS stores the PLMN which rejected the update request in the SIM.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.6.2	Conformance requirement

After the receipt of a LOCATION UPDATE REJECT message with the cause "PLMN not allowed" the MS shall update the EFFPLMN in the SIM.

TS GSM 02.11, clause 3.2.2.4

27.6.3	Test purpose

To verify that the MS correctly updates the EFFPLMN, i.e fill up existing gaps in the elementary file before overwriting any existing entries.

27.6.4	Method of test

27.6.4.1	Initial conditions

The SS transmits on the BCCH, with the following network parameters:

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	234/03/value in SIM

Access control:		unrestricted.

The default SIM is used with the following exception:

EFFPLMN (Forbidden PLMNs)

Logically:	PLMN1:	234 02 (MCC MNC)

PLMN2:	empty

PLMN3:	234 04

PLMN4:	234 05

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12

32	F4	20	FF	FF	FF	32	F4	40	32	F4	50

The SIM is installed into the ME and the MS is set to automatic PLMN selection mode.

27.6.4.2	Procedure

a)	The MS is powered on.

b)	After receipt of a CHANNEL REQUEST from the MS, the SS sends IMMEDIATE ASSIGNMENT to the MS.

c)	After receipt of a LOCATION UPDATE REQUEST from the MS, the SS sends LOCATION UPDATE REJECT to the MS with the cause "PLMN not allowed", followed by CHANNEL RELEASE.

d)	The MS is soft powered down.

27.6.5	Test requirement

1)	After step b) the MS shall send LOCATION UPDATE REQUEST to the SS.

2)	After step d) the SIM shall contain:

EFFPLMN (Forbidden PLMNs)

Logically:	PLMN1:	234 02 (MCC MNC)

PLMN2:	234 03

PLMN3:	234 04

PLMN4:	234 05

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12

32	F4	20	32	F4	30	32	F4	40	32	F4	50

or

EFFPLMN (Forbidden PLMNs)

Logically:	PLMN1:	234 02 (MCC MNC)

PLMN2:	234 04

PLMN3:	234 05

PLMN4:	234 03

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12

32	F4	20	32	F4	40	32	F4	50	32	F4	30

27.7	MS deleting forbidden PLMNs

27.7.1	Definition and applicability

In manual PLMN selection mode the MS allows location update attempts to all available PLMNs, including forbidden PLMNs (as indicated by the forbidden PLMN list on the SIM). As a result of a successful location update procedure onto a PLMN which is in the forbidden PLMN list, the forbidden PLMN list is automatically updated by the MS.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.7.2	Conformance requirement

1.	In manual PLMN selection mode the MS shall be able to perform a LOCATION UPDATE attempt to a PLMN which is in the forbidden PLMN list.

TS GSM 02.11, clause 3.2.2.2.

2.	After receipt of LOCATION UPDATE ACCEPT the MS shall delete the forbidden PLMN from the forbidden PLMN list.

TS GSM 02.11, clause 3.2.2.4.

27.7.3	Test purpose

1)	To verify that in automatic PLMN selection mode the MS does not attempt to access PLMNs stored in EFFPLMN on the SIM.

2)	To verify that the MS is able to perform a LOCATION UPDATE on a forbidden PLMN in manual PLMN selection mode.

3)	To verify that the MS after a successful LOCATION UPDATE deletes the PLMN in the EFFPLMN on the SIM.

27.7.4	Method of test

27.7.4.1	Initial conditions

The SS transmits on the BCCH, with the following network parameters:

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	234/01/value in SIM

Access control:		unrestricted.

The default SIM is used with the following exception:

EFFPLMN (Forbidden PLMNs)

Logically:	PLMN1:	empty

PLMN2:	empty

PLMN3:	234 01 (MCC MNC)

PLMN4:	empty

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12

FF	FF	FF	FF	FF	FF	32	F4	10	FF	FF	FF

The SIM is installed into the ME and the MS is set to automatic PLMN selection mode.

27.7.4.2	Procedure

a)	The MS is powered on.

b)	PLMN with MCC/MNC of 234/01 is manually selected.

c)	After receipt of a CHANNEL REQUEST from the MS, the SS sends IMMEDIATE ASSIGNMENT to the MS.

d)	After receipt of a LOCATION UPDATE REQUEST from the MS, the SS sends LOCATION UPDATE ACCEPT with:

LAI (MCC/MNC):	234/01

TMSI:		'12345678'

to the MS.

e)	After receipt of TMSI REALLOCATION COMPLETE from the MS, the SS sends CHANNEL RELEASE.

f)	The MS is soft powered down.

27.7.5	Test requirement

1)	After step a) the MS shall not attempt a LOCATION UPDATE.

2)	After step c) the MS shall send LOCATION UPDATE REQUEST to the SS.

3)	After step d) the MS shall respond with TMSI REALLOCATION COMPLETE.

4)	After step f) the SIM shall contain the following values:

EFLOCI (Location Information)

Logically:	LAI-MCC:	234

LAI-MNC:	01

TMSI:		'12345678'

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11

12	34	56	78	32	F4	10	xx	xx	xx	00

EFFPLMN (Forbidden PLMNs)

Logically:	PLMN1:	empty

PLMN2:	empty

PLMN3:	empty

PLMN4:	empty

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12

FF	FF	FF	FF	FF	FF	FF	FF	FF	FF	FF	FF

27.8	MS updating the PLMN selector list

27.8.1	Definition and applicability

The PLMN selector list gives in priority order the preferred PLMNs on which the MS shall register. The list is stored on the SIM in the EFPLMNsel. Update and deletion of PLMNs may be performed by the subscriber.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.8.2	Conformance requirement

The MS shall correctly replace the selected PLMN in the PLMN selector list.

TS GSM 11.11, clause 11.5.5.

27.8.3	Test purpose

To verify that the MS correctly updates the EFPLMNsel.

27.8.4	Method of test

27.8.4.1	Initial conditions

No SS is required for this test.

The default SIM is used.

The SIM is installed into the ME and the MS is powered on.

27.8.4.2	Procedure

a)	The user shall initiate an MMI dependent procedure to change the second PLMN in the PLMN selector list to MCC/MNC of 567/01.

b)	The MS is soft powered down.

27.8.5	Test requirement

After step b) the SIM shall contain the following values:

EFPLMNsel (PLMN Selector)

Logically:	1st PLMN:	234 01 (MCC MNC)

2nd PLMN:	567 01

3rd PLMN:	234 03

4th PLMN:	234 04

5th PLMN:	234 05

6th PLMN:	234 06

7th PLMN:	246 81

8th PLMN:	246 82

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B0	B10	B11	B12

32	F4	10	65	F7	10	32	F4	30	32	F4	40

B13	B14	B15	B16	B17	B18	B19	B20	B21	B22	B23	B24

32	F4	50	32	F4	60	42	F6	18	42	F6	28

27.9	MS recognizing the priority order of the PLMN selector list

27.9.1	Definition and applicability

The PLMN selector list gives in priority order the preferred PLMNs on which the MS shall register. The list is stored on the SIM in the EFPLMNsel. Update and deletion of PLMNs may be performed by the subscriber by the use of the PIN.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.9.2	Conformance requirement

When registering onto a VPLMN the MS shall take into account the priority order of the PLMNs in the preferred list on the SIM.

TS GSM 02.11, clause 3.2.2.2.

27.9.3	Test purpose

To verify that the PLMN with the higher priority (defined by its position in EFPLMNsel) takes precedence over the PLMN with the lower priority when the MS performs a network selection.

27.9.4	Method of test

27.9.4.1	Initial conditions

The SS transmits on two BCCHs, with the following network parameters:

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	234/33/value in SIM

Access control:		unrestricted.

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	234/34/value in SIM

Access control:		unrestricted.

The default SIM is used with the following exception:

EFPLMNsel (PLMN Selector)

Logically:	1st PLMN:		234 01 (MCC MNC)

2nd PLMN:		234 02

.....			.....

.....			.....

32nd PLMN:		234 32

33rd PLMN:		234 34

34th PLMN:		234 33

Coding:	B1	B2	B3	B4	B5	B6

32	F4	10	32	F4	20

.....

.....

B94	b95	B96	B97	B98	B99	B100	B101	B102

32	F4	23	32	F4	43	32	F4	33

The SIM is installed into the ME and the MS is set to automatic PLMN selection mode.

27.9.4.2	Procedure

a)	The MS is powered on.

b)	After receipt of a CHANNEL REQUEST from the MS, the SS sends IMMEDIATE ASSIGNMENT to the MS.

c)	After receipt of a LOCATION UPDATE REQUEST from the MS, the SS sends LOCATION UPDATE ACCEPT with:

LAI (MCC/MNC):	234/34

TMSI:		'34567890'

to the MS

d)	After receipt of a TMSI REALLOCATION COMPLETE from the MS, the SS sends CHANNEL RELEASE to the MS.

e)	The MS is soft powered down.

27.9.5	Test requirement

1)	After step b) the MS shall send LOCATION UPDATE REQUEST containing an MCC/MNC of 234/34 to the SS.

2)	After step c) the MS shall respond with TMSI REALLOCATION COMPLETE.

3)	After step e) the SIM shall contain the following values:

EFLOCI (Location Information)

Logically:	LAI-MCC:	234

LAI-MNC:	34

TMSI:		'34567890'

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11

34	56	78	90	32	F4	43	xx	xx	xx	00

27.10	MS access control management

27.10.1	Definition and applicability

Access Control allows restriction of call access attempts. All mobile stations are assigned to a "low order class", and optionally (for priority uses) also to one or more "high order classes".

A "high order class" is only valid in the HPLMN or HPLMN country. Otherwise, the "low order class" is used.

The classes are programmed on the SIM. The network controls which classes at any time may be barred.

In addition, there is a separate mechanism for control of network access for emergency call attempts.

This test applies to GSM and DCS1800 MEs using ID-1 or Plug-in SIM.

27.10.2	Conformance requirement

1.	The ME shall read the access control value as part of the SIM/ME initialization procedure, and subsequently adopt this value.

TS GSM 11.11, clause 11.2.1

2.	If the MS is a member of at least one access class which corresponds to the permitted classes as signalled over the air interface, and the access class is applicable in the serving network, the MS may make call attempts. Otherwise call access attempts are not allowed.

If access class 10 is barred, MS of classes 0 - 9 and ME without SIMs shall not make emergency call attempts.

MS of classes 11 - 15 are not allowed to make emergency call attempts if access class 10 and the relevant access class(es) between 11 and 15 are barred. Otherwise, emergency call attempts are allowed irrespective of the conditions of access class 10.

All options are shown in Figure 27-1 and are referenced to the tests.

TS GSM 02.11, clauses 4.3 and 4.4.

27.10.3	Test purpose

1)	To verify that the ME reads the access control value as part of the SIM/ME initialisation procedure, and subsequently adopts this value.

2)	To verify that the MS controls its network access in accordance with its access control class and the conditions imposed by the serving network.

The tests verify ME performance for the following:

Tests (a) and (b)	No SIM in ME.

Tests (c) to (e)	MS with access class 0 to 9.

Test (f)		MS with access class 11 and 15 not in HPLMN, and

MS with access class 12,13 and 14 not in HPLMN country.

Test (g) and (h)	MS with access class 11 and 15 in HPLMN, and

MS with access class 12,13 and 14 in HPLMN country.

Each of the above are tested against all relevant combinations of access control and emergency call bits signalled by the network, as shown in Table 27-1.

27.10.4	Method of test

27.10.4.1	Initial conditions

The SS transmits on the BCCH, with the following network parameters:

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	see Table 27-1

Access control:		see Table 27-1

RACH:			see Table 27-1

A SIM is installed in the ME containing IMSI and access control values as given in Table 27-1 and the MS is powered on.

Note :	Depending on the initial value of the EFLOCI, the MS may perform a location update. This will be accepted by the SS.

Coding details

SIM IMSI: Data Field 6F 07

Value 246813579		Value 2468135x9

byte 1			05H				05H

byte 2			29H				29H

byte 3			64H				64H

byte 4			18H				18H

byte 5			53H				53H

byte 6			97H				9xH

byte 7			FFH				FFH

byte 8			FFH				FFH

byte 9			FFH				FFH

Access class: Data field 6F 78

See GSM 11.11.

NETWORK (SS)

RACH: As defined in GSM 04.08 section 10.5.2.29.

octet 1	01111000

octet 2	00001000

octet 3	}

octet 4	} as Table 27-1

27.10.4.2	Procedure

a)	Using the MMI or EMMI a normal call set-up is attempted.

b)	Using the MMI or EMMI an emergency call set-up is attempted.

c)	The test is repeated for each set of values in Table 27-1.

27.10.5	Requirement

After steps a) and b) the MS shall access the network, or shall make no access attempt, in accordance with Table 27-1.

Note :	For type approval, to limit testing, in tests (c, (d) and (e) it is only necessary that one of the access classes is tested.

Table 27-1

SIM	Network	Test result

IMSI	Access	RACH	BCCH/LAI	Normal	Emergency

class	octet 3	MCC	calls	calls

octet 4	MNC

TEST (a)

No SIM in ME		00000100	234	No	No

00000000	01

TEST (b)

No SIM in ME		00000000	234	No	Yes

00000000	01

TEST (c)

246813579	0	00000100	246	No	No

00000001	81

"	1	00000100	"	No	No

00000010

"	2	00000100	"	No	No

00000100

"	3	00000100	"	No	No

00001000

"	4	00000100	"	No	No

00010000

"	5	00000100	"	No	No

00100000

"	6	00000100	"	No	No

01000000

"	7	00000100	"	No	No

10000000

"	8	00000101	"	No	No

00000000

"	9	00000110	"	No	No

00000000

Table 27-1 (cont)

SIM	Network	Test result

IMSI	Access	RACH	BCCH/LAI	Normal	Emergency

class	octet 3	MNC	calls	calls

octet 4	MNC

TEST (d)

246813579	0	00000000	246	No	Yes

00000001	81

"	1	00000000	"	No	Yes

00000010

"	2	00000000	"	No	Yes

00000100

"	3	00000000	"	No	Yes

00001000

"	4	00000000	"	No	Yes

00010000

"	5	00000000	"	No	Yes

00100000

"	6	00000000	"	No	Yes

01000000

"	7	00000000	"	No	Yes

10000000

"	8	00000001	"	No	Yes

00000000

"	9	00000010	"	No	Yes

00000000

Table 27-1 (cont)

SIM	Network	Test result

IMSI	Access	RACH	BCCH/LAI	Normal	Emergency

class	octet 3	MNC	calls	calls

octet 4	MNC

TEST (e)

246813579	0	11111011	246	Yes	Yes

11111110	81

"	1	11111011	"	Yes	Yes

11111101

"	2	11111011	"	Yes	Yes

11111011

"	3	11111011	"	Yes	Yes

11110111

"	4	11111011	"	Yes	Yes

11101111

"	5	11111011	"	Yes	Yes

11011111

"	6	11111011	"	Yes	Yes

10111111

"	7	11111011	"	Yes	Yes

01111111

"	8	11111010	"	Yes	Yes

11111111

"	9	11111001	"	Yes	Yes

11111111

Table 27-1 (cont)

SIM	Network	Test result

IMSI	Access	RACH	BCCH/LAI	Normal	Emergency

class	octet 3	MNC	calls	calls

octet 4	MNC

TEST (f)

2468135x9	11 & x	00000111	246	No	No

11111111	82

"	"	00000011	"	No	Yes

11111111

"	"	00000000		Yes	Yes

00000000

Set 'x' to a random

value between 0 & 9

2468135x9	12 & x	00000111	234	No	No

11111111	01

"	"	00000011	"	No	Yes

11111111

"	"	00000000		Yes	Yes

00000000

Set 'x' to a random

value between 0 & 9

2468135x9	13 & x	00000111	234	No	No

11111111	01

"	"	00000011	"	No	Yes

11111111

"	"	00000000		Yes	Yes

00000000

Set 'x' to a random

value between 0 & 9

2468135x9	14 & x	00000111	234	No	No

11111111	01

"	"	00000011	"	No	Yes

11111111

"	"	00000000		Yes	Yes

00000000

Set 'x' to a random

value between 0 & 9

2468135x9	15 & x	00000111	246	No	No

11111111	82

"	"	00000011	"	No	Yes

11111111

"	"	00000000		Yes	Yes

00000000

Set 'x' to a random

value between 0 & 9

Table 27-1 (cont)

SIM	Network	Test Result

IMSI	Access	RACH	BCCH/LAI	Normal	Emergency

Class	octet 3	MNC	Calls	Calls

octet 4	MNC

TEST (g)

246813579	11 & x	00001100	246	No	Yes

11111111	81

"	"	00001000	"	No	Yes

11111111

246813579	12 & x	00010100	246	No	Yes

11111111	82

"	"	00010000	"	No	Yes

11111111

246813579	13 & x	00100100	246	No	Yes

11111111	82

"	"	00100000	"	No	Yes

11111111

246813579	14 & x	01000100	246	No	Yes

11111111	82

"	"	01000000	"	No	Yes

11111111

246813579	15 & x	10000100	246	No	Yes

11111111	81

"	"	10000000	"	No	Yes

11111111

Table 27-1 (cont)

SIM	Network	Test Result

IMSI	Access	RACH	BCCH/LAI	Normal	Emergency

Class	octet 3	MNC	Calls	Calls

octet 4	MNC

TEST (h)

246813579	11 & x	11110011	246	Yes	Yes

11111111	81

246813579	12 & x	11101011	246	Yes	Yes

11111111	82

246813579	13 & x	11011011	246	Yes	Yes

11111111	82

246813579	14 & x	10111011	246	Yes	Yes

11111111	82

246813579	15 & x	01111011	246	Yes	Yes

11111111	81

246813579	11 & x	11110111	246	Yes	Yes

11111111	81

246813579	12 & x	11101111	246	Yes	Yes

11111111	82

246813579	13 & x	11011111	246	Yes	Yes

11111111	82

246813579	14 & x	10111111	246	Yes	Yes

11111111	82

246813579	15 & x	01111111	246	Yes	Yes

11111111	81

.G.1110figs\FIG27-1.EPS;16.499 cm;24.698 cm;EPS

27.11	Exchange protocol tests

The list of SIM exchange protocol tests is divided into three subsections:

The first subsection deals with character transmission, which constitutes the basis for ME-SIM exchanges.

The remaining two subsections correspond to the two main parts of a transaction with a microprocessor card: the reset and the card answer then command processing. Both subsections describe the protocol tests to be run during each corresponding phase of the transaction.

27.11.1	Character transmission

27.11.1.1	Bit/character duration during the transmission from the ME to the SIM

27.11.1.1.1	Definition and applicability

Data is transmitted serially across the SIM/ME interface. Transmission can be either synchronous or asynchronous. To facilitate this, bit duration is defined in terms of elementary time units (etu). The ten bits that constitute a character are:

-	the start bit

-	eight data bits

-	the parity bit

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.11.1.1.2	Conformance requirement

The bit/character duration of the request sent from the ME shall be in the range specified.

TS GSM 11.11, section 5.8.

27.11.1.1.3	Test purpose

To verify the bit/character duration during the transmission from the ME to the SIM.

27.11.1.1.4	Method of test

27.11.1.1.4.1	Initial conditions

The ME is connected to the SIM simulator, and powered on.

27.11.1.1.4.2	Procedure

A character is transmitted from the ME to the SIM. The SIM simulator shall measure the bit/character duration of each of the ten bit elements in the request from the ME.

27.11.1.1.5	Test requirement

The bit/character duration of the request sent from the ME shall be in the range specified.

27.11.1.2	Bit/character duration during the transmission from the SIM simulator to the ME

27.11.1.2.1	Definition and applicability

Data is transmitted serially across the SIM/ME interface. Transmission can be either synchronous or asynchronous. To facilitate this, bit duration is defined in terms of elementary time units (etu). The ten bits that constitute a character are:

-	the start bit

-	eight data bits

-	the parity bit

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.11.1.2.2	Conformance requirement

Responses with maximum and minimum bir/character duration times shall be accepted by the ME.

TS GSM 11.11, clause 5.8.

27.11.1.2.3	Test purpose

To verify the acceptance of maximum and minimum bit/character duration during the transmission from the SIM to the ME.

27.11.1.2.4	Method of test

27.11.1.2.4.1	Initial conditions

The ME is connected to the SIM simulator, and powered on.

27.11.1.2.4.2	Procedure

The SIM simulator shall send responses with the maximum and minimum bit/character durations specified in GSM 11.11.

27.11.1.2.5	Test requirement

The ME shall accept the response and act accordingly.

27.11.1.3	Inter-character delay

27.11.1.3.1	Definition and applicability

The parameter N defines any extra guard-time required between transmission of characters.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.11.1.3.2	Conformance requirement

1)	The ME shall accept or reject the SIM according to the value N indicated in TC1.

2)	The ME shall accept characters with minimum and maximum inter-character delay transmitted from the SIM to the ME.

TS GSM 11.11, clause 5.8.

27.11.1.3.3	Test purpose

1)	To verify the correct adaption of the inter-character delay during the transmission from the ME to the SIM by evaluation of the character TC1 indicated in the ATR.

2)	To verify that the ME accepts the minimum and maximum inter-character delays during the transmission from the SIM to the ME.

27.11.1.3.4	Method of test

27.11.1.3.4.1	Initial conditions

The ME is connected to the SIM simulator, and powered on.

27.11.1.3.4.2	Procedure

a)	The requested inter-character delays from the ME to the SIM (or the SIM Simulator) are modified by changing the value of N in TC1:

a.1)	N = 0;

a.2)	N = Values other than 0 and 255.

b)	Verification is performed in the following two ways:

-	observation using the oscilloscope or logical analyzer,

-	non-acceptance by the SIM (or the SIM Simulator) of the byte transmitted too soon.

27.11.1.3.5	Test requirement

In step a.1) the ME shall work with the SIM.

In step a.2) the ME shall reject the SIM or repeat the reset at least twice more after receiving the Answer to Reset and then reject the SIM.

27.11.1.4	Error handling during the transmission from the ME to the SIM simulator

27.11.1.4.1	Definition and applicability

Error checking is done for each character transmitted by making use of the parity bit. If the SIM detects a parity error, an error signal is sent to the ME, and the ME retransmits that character.

This test applies to GSM and DCS1800 MEs using either ID-1 of Plug-in SIM.

27.11.1.4.2	Conformance requirement

Subsequent to Answer to Reset and the protocol type selection, the error detection and character repetition procedure specified in TS GSM 11.11 is mandatory for transmission on the basis of T = 0: on receipt of an error signal, the ME shall repeat the previously transmitted character.

TS GSM 11.11, clause 5.9.

27.11.1.4.3	Test purpose

To verify the error handling during the transmission from the ME to the SIM.

27.11.1.4.4	Method of test

27.11.1.4.4.1	Initial conditions

The ME is connected to the SIM simulator, and powered on.

27.11.1.4.4.2	Procedure

The SIM simulator shall transmit an error signal in response to a received character in accordance with ISO 7816-3, section 6.1.3.

27.11.1.4.5	Test requirement

The ME shall repeat the character in accordance with ISO 7816-3, section 6.1.3.

27.11.1.5	Error handling during transmission from the SIM simulator to the ME

27.11.1.5.1	Definition and applicability

Error checking is done for each character transmitted by making use of the parity bit. If the SIM detects a parity error, an error signal is sent to the ME, and the ME retransmits that character.

This test applies to GSM and DCS1800 MEs using either ID-1 of Plug-in SIM.

27.11.1.5.2	Conformance requirement

Subsequent to Answer to Reset and the protocol type selection, the error detection and character repetition procedure specified in TS GSM 11.11 is mandatory for transmission on the basis of T = 0: on receipt of a response with a parity error, the ME shall send an error signal and expect the previously transmitted character to be repeated.

TS GSM 11.11, clause 5.9.

27.11.1.5.3	Test purpose

To verify the error handling during the transmission from the SIM to the ME.

27.11.1.5.4	Method of test

27.11.1.5.4.1	Initial conditions

The ME is connected to the SIM simulator, and powered on.

27.11.1.5.4.2	Procedure

The SIM simulator shall send a response with a parity error and check that the ME performs error handling in accordance with ISO 7816-3, section 6.1.3.

27.11.1.5.5	Test requirement

The ME shall send an error signal in accordance with ISO 7816-3, section 6.1.3, and expect a repetition of the character. The ME shall correctly evaluate the character when repeated by the SIM simulator.

27.11.2	Answer to reset (RST)

27.11.2.1	Acceptance of SIMs with internal RST

27.11.2.1.1	Definition and applicability

Internal RST is one possible implementation of reset, and MEs must be able to accept SIMs with internal reset.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.11.2.1.2	Conformance requirement

The ME shall accept a SIM with internal reset by recognising the ATR within the specified time range.

TS GSM 11.11, clause 5.7.

ISO/IEC 7816-3: 1990, section 5.2.

27.11.2.1.3	Test purpose

To verify that the ME accepts a SIM with internal reset.

27.11.2.1.4	Method of test

27.11.2.1.4.1	Initial conditions

The ME is connected to the SIM (or SIM simulator) and powered on.

The SIM (or the SIM simulator) must be synchronized with the successive detection of Vcc and the clock signal in order to begin the reset routine.

27.11.2.1.4.2	Procedure

The SIM (or the SIM simulator) verifies that RST is at low status and starts its answer between (400/fi) s and (40000/fi) s after the clock signal has been detected.

Note :	fi is the initial frequency supplied by the ME.

27.11.2.1.5	Test requirement

The ME accepts the SIM with internal reset.

27.11.2.2	Acceptance of SIMs with active low RST

27.11.2.2.1	Definition and applicability

Active low RST is one possible implementation of reset, and MEs must be able to accept SIMs with active low reset.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.11.2.2.2	Conformance requirement

The ME shall accept a SIM with active low reset by putting the RST contact to state H. The signal timing shall be in accordance with the specification.

TS GSM 11.11, clause 5.7.

ISO/IEC 7816-3: 1990, section 5.2.

27.11.2.2.3	Test purpose

To verify that the ME accepts a SIM with active low reset.

27.11.2.2.4	Method of test

27.11.2.2.4.1	Initial conditions

The ME is connected to the SIM (or SIM simulator) and powered on.

The SIM (or the SIM simulator) must be synchronized with the successive detection of Vcc and the clock signal in order to begin the reset routine.

27.11.2.2.4.2	Procedure

The SIM simulator sends no internal reset and checks that the RST contact is put to state H after a minimum of (40,000/fi)s. The SIM simulator sends an answer to reset and verifies that the RST contact stays at this level for at least a further (40000/fi) s.

27.11.2.2.5	Test requirement

The ME shall accept the SIM with active low reset.

27.11.2.3	Characters of the answer to reset

27.11.2.3.1	Definition and applicability

When the SIM is reset, it sends up to 32 characters to the ME, containing information which must be interpreted by the ME to ascertain the transmission protocol to be used.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.11.2.3.2	Conformance requirement

1.	The ME shall adopt the data encoding convention and initial etu time defined in the initial character TS of the ATR.

TS GSM 11.11, clauses 5.6, 5.7 and 5.8.

2.	The ME shall be able to receive interface characters for other transmission protocols then T = 0, historical characters and a check byte, even if only T = 0 is used by the ME.

TS GSM 11.11, clause 5.7.1.

27.11.2.3.3	Test purpose

1.	To verify that the ME adopts the appropriate data encoding convention and initial elementary time unit (etu) defined in the initial character TS of the Answer to Reset.

2.	To verify that the ME accepts historical characters and the check digit.

27.11.2.3.4	Method of test

27.11.2.4.1	Initial conditions

The ME is connected to the SIM or SIM simulator.

27.11.2.4.2	Procedure

a)	The ME is powered on

b)	The SIM or SIM simulator sends an Answer to Reset with TS indicating direct convention coding, historical characters and a check digit.

c)	The ME is made to send further commands to the SIM (e.g by entering the PIN).

d)	The ME is switched off and on. This time the SIM or SIM simulator sends an Answer to Reset with TS indicating inverse convention coding.

e)	The ME is made to send further commands to the SIM (e.g by entering the PIN).

27.11.2.3.5	Test requirement

1.	After step b), the ME shall use this convention for all subsequent transmissions, and adopt the correct etu.

2.	After step d), the ME shall use this convention for all subsequent transmissions, and adopt the correct etu.

27.11.2.4	PTS procedure

27.11.2.4.1	Definition and applicability

The PTS procedure is required to select the standard transmission protocol if the SIM does not use this as a default.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.11.2.4.2	Conformance requirement

If the ME receives an Answer to Reset where TA1 is not equal to '11', it shall initiate the PTS procedure as defined in TS GSM 11.11.

TS GSM 11.11, clause 5.7.2.

27.11.2.4.3	Test purpose

To verify that ME uses the PTS procedure as specified in GSM 11.11.

27.11.2.4.4	Method of test

27.11.2.4.4.1	Initial conditions

The ME is connected to the SIM or SIM simulator.

27.11.2.4.4.2	Procedure

a)	The ME is powered on.

b)	The SIM (or the SIM simulator) gives an Answer to Reset with TA1 not equal to '11'.

27.11.2.4.5	Test requirement

After step b), the ME shall send to the SIM (or the SIM simulator) 'FF00FF'.

27.11.2.5	Reset repetition

27.11.2.5.1	Definition and applicability

If transmission errors result in the ATR being unitelligible to the ME, the ME performs the reset again. The minimum number of reset attempts is three.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.11.2.5.2	Conformance requirement

Following receipt of a wrong ATR, the ME shall perform a reset. The ME shall not reject the SIM until at least three consecutive wrong ATRs are received.

TS GSM 11.11, clause 5.9.

27.11.2.5.3	Test purpose

To verify that the ME repeats the reset procedure on receipt of a wrong ATR, and does not reject the SIM unless at least three consecutive wrong ATRs are received.

27.11.2.5.4	Method of test

27.11.2.5.4.1	Initial conditions

The ME is connected to the SIM simulator.

27.11.2.5.4.2	Procedure

a)	The ME is powered on.

b)	The SIM simulator sends a non understandable answer to reset to the ME. (e.g. a wrong TS byte), at each reset initiated by the ME.

27.11.2.5.5	Test requirement

1.	After step b), the ME shall repeat the reset.

2.	The ME shall not reject the SIM until at least three consecutive wrong answers to reset are received.

27.11.3	Command processing

27.11.3.1	Procedure bytes ACK

27.11.3.1.1	Definition and applicability

The byte ACK is sent from the SIM to the ME, and gives the ME an acknowledgement for the previous instruction, information concerning transfer of data, and the setting og Vpp.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.11.3.1.2	Conformance requirement

On the basis of protocol T = 0, the ME shall correctly use the different modes of data transmission defined in ISO 7816-3, section 8.2.2.1.

TS GSM 11.11, clause 5.7.

ISO/IEC 7616: 1990, section 8.2.2.1.

27.11.3.1.3	Test purpose

To verify that the ME uses correctly the different modes of data transmission.

27.11.3.1.4	Method of test

27.11.3.1.4.1	Initial conditions

The ME is connected to the SIM simulator and powered on.

27.11.3.1.4.2	Procedure

The ME is made to initiate a command which requires the transfer of data to the SIM. The SIM simulator  first answers ACK=INS or (INS+1) complemented and then changes ACK to INS or INS+1 during the transmission.

27.11.3.1.5	Test requirement

The command shall be executed correctly, i.e for the first part where ACK = INS or (INS + 1) complemented, each character of data is sent individually, and then the ME waits for a further ACK byte from the SIM simulator. When ACK is changed to INS or INS+1, the ME can then send all remaining data bytes together.

27.12	Evaluation of directory characteristics

27.12.1	Operating speed in authentication procedure

27.12.1.1	Definition and applicability

Authentication is performed by a GSM network on the SIM, by sending a random number to the SIM. The SIM then performs a calculation on the random number, and sends the result to the netwrok for verification.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.12.1.2	Conformance requirement

If bit b2 of the file characteristics is set to 1, then the ME shall provide a clock frequency of at least 13/4 MHz to enable the SIM to run the authentication process in the required time.

TS GSM 11.11, clause 5.4.

27.12.1.3	Test purpose

To verify that the authentication procedure is done with a frequency of at least 13/4 MHz if the bit b2 of the file characteristics (byte 1 of the directory characteristics) is set to 1.

27.12.1.4	Method of test

27.12.1.4.1	Initial conditions

System simulator:

1 cell, default parameters.

Mobile Equipment:

Connected to a SIM or SIM-simulator with bit b2 of the file characteristics set to 1.

ME is powered on.

27.12.1.4.2	Procedure

An authentication is made in the same way as in test [26.7.2. Authentication]. The MS is paged. After the MS has responded with a PAGING RESPONSE message to the SS, the SS initiates an authentication procedure, sending the MS the value RAND. During authentication, the SIM simulator checks the frequency of the clock supplied by the ME. Following the AUTHENTICATION RESPONSE from the MS, the SS sends CHANNEL RELEASE.

27.12.1.5	Test requirement

The frequency of the clock shall be at least 13/4 MHz during the authentication procedure.

27.12.2	Clock stop

27.12.2.1	Definition and applicability

The ME may switch off the clock signal to the SIM, but only if the SIM indicates that it supports this feature.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.12.2.2	Conformance requirement

1.	The ME shall not stop the clock, unless the requirements indicated in byte 1 of the file characteristics are met.

TS GSM 11.11, clauses 5.5 and 9.2.1.

2.	The ME shall wait at least five elementary time units after having received the last bit of the response before switching off the clock.

TS GSM 11.11, clause 5.5

27.12.2.3	Test purpose

1.	To verify that the clock is only switched off if requirements are met as indicated in the file characteristics (byte 1 of the directory characteristics).

2.	To verify that the ME waits at least 5 etu before switching off the clock.

27.12.2.4	Method of test

27.12.2.4.1	Initial conditions

The ME is connected to a SIM or SIM simulator.

27.12.2.4.2	Procedure

a)	A SIM (or a SIM simulator) is used with bits set as follows:

Bit b1		Bit b3		Bit b4

0		0		0

b)	The ME is powered on, and left on for several seconds after the ATR has been received.

c)	The ME is powered off, and a SIM (or SIM simulator) is used with bits set as follows:

Bit b1		Bit b3		Bit b4

0		1		0

d)	The ME is powered on, and left on for several seconds after the ATR has been received.

e)	The ME is powered off, and a SIM (or SIM simulator) is used with bits set as follows:

Bit b1		Bit b3		Bit b4

0		0		1

f)	The ME is powered on, and left on for several seconds after the ATR has been received.

g)	The ME is powered off, and a SIM (or SIM simulator) is used with bits set as follows:

Bit b1		Bit b3		Bit b4

1		0		0

h)	The ME is powered on, and left on for several seconds after the ATR has been received.

27.12.2.5	Test requirement

1.	After step b), the ME shall not switch off the clock.

2.	After step d), the ME shall not switch off the clock, unless at high level.

3.	After step f), the ME shall not switch off the clock, unless at low level.

4.	After step h), the ME shall not switch off the clock until at least five elementary time units have elapsed after having received the last bit of the response.

27.13	Mechanical tests

27.13.1	Contact pressure

27.13.1.1	Definition and applicability

The contacts of the card reader must exert a force to maintain a good electrical contact, but the force must not be excessive and damage the SIM. Only unembossed cards apply to Plug-in SIMs.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.13.1.2	Conformance requirement

A contact force may not be greater than 0.5 N per contact.

TS GSM 11.11, clause 4.3.3.

27.13.1.3	Test purpose

To verify that the contact pressure of each contacting element is not greater than 0.5 N when each of the following types of card is used:

i)	Unembossed

ii)	Embossed on the contact side

iii)	Embossed on the opposite side to the contacts.

Note :	Only type i) applies to the plug-in SIM.

27.13.1.4	Method of test

27.13.1.4.1	Initial conditions

The ME manufacturers shall provide a separate card reader (mechanical components) to make the measurement possible.

27.13.1.4.2	Procedure

The pressure of each contacting element is measured.

27.13.1.5	Test requirement

The contact pressure of each contacting element shall be not greater than 0.5 N.

27.13.2	Shape of contacts for IC card SIM card reader

27.13.2.1	Definition and applicability

The shape of the contacts is important to maintain a good electrical contact, nut not damage the SIM.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.13.2.2	Conformance requirement

The radius of curvature of the contacting elements shall be greater than or equal to 0.8 mm in the contact area of both axes.

TS GSM 11.11, clause 4.3.3.

27.13.2.3	Test purpose

To verify that the radius of curvature of the contacting elements is greater than or equal to 0.8 mm in the contact area on both axes.

27.13.2.4	Method of test

27.13.2.4.1	Initial conditions

The ME manufacturers shall provide a separate card reader (mechanical components) to make the measurement possible.

27.13.2.4.2	Procedure

The radius of curvature of the contacting elements is measured on both axes.

27.13.2.5	Test requirement

The radius of curvature of the contacting elements shall be greater than or equal to 0.8 mm in the contact area on both axes.

DCS1800 only - begin

27.14	DCS and GSM directories access

27.14.1	Definition and applicability

The TMSI is temporarily used for identification of the MS by a GSM network. It will have been previously assigned by the network. The TMSI is stored in the SIM by the ME and read during the SIM/ME initialisation procedure. If a SIM contains both a GSM and DCS1800 directory, a DCS1800 ME should select the TMSI value in the DCS directory.

This test applies to DCS1800 MEs using either ID-1 or Plug-in SIM.

27.14.2	Conformance requirement

1.	After SIM activation, a DCS1800 ME shall select the DCS1800 directory (dedicated file), and use elementary files within that directory.

TS GSM 11.11, clause 11.2.1.

2.	On receipt of an IMMEDIATE ASSIGNMENT message the MS shall send PAGING RESPONSE containing the TMSI stored in the SIM.

TS GSM 11.11, clauses 11.2.1 and 11.4.5.

TS GSM 04.08, clause 10.5.1.4.

27.14.3	Test purpose

To verify that a DCS1800 ME reads the DCS directory of a SIM which has a GSM and DCS directory.

The TMSI value read must be the DCS 1800 one.

27.14.4	Method of test

27.14.4.1	Initial conditions

The SS transmits on the BCCH, with the following network parameters:

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	246/81/value in SIM

Access Control:		unrestricted

The default SIM is used with the following exceptions:

GSM 900 directory:

EFLOCI (Location Information)

Logically:	LAI-MCC:	246

LAI-MNC:	81

TMSI:	'5678'

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11

00	00	56	78	42	F6	18	xx	xx	FF	00

DCS1800 directory:

EFLOCI (Location Information)

Logically:	LAI-MCC:	246

LAI-MNC:	81

TMSI:	'2143'

Coding:	B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11

00	00	21	43	42	F6	18	xx	xx	FF	00

The SIM is installed into the ME and the MS is powered on.

27.14.4.2	Procedure

a)	The SS sends PAGING REQUEST to the MS using the TMSIstored in the DCS1800 directory of the SIM.

b)	After receipt of a CHANNEL REQUEST from the MS, the SS sends IMMEDIATE ASSIGNMENT to the MS.

c)	After receipt of a PAGING RESPONSE from the MS, the SS sends CHANNEL RELEASE to the MS

27.14.5	Test requirement

After step b) the MS shall send PAGING RESPONSE to the SS containing the TMSI stored in the DCS1800 directory on the SIM.

27.15	GSM SIM in DCS1800 ME

27.15.1	Definition and applicability

As GSM 900 and DCS1800 SIMS are physically the same, the ME must discriminate between them logically.

This test applies to DCS1800 MEs using either ID-1 or Plug-in SIM.

27.15.2	Conformance requirement

After SIM activation, a DCS1800 ME shall select the DCS1800 directory (dedicated file), and use elementary files within that directory. If this is not successful, then operation shall not start.

TS GSM 11.11, clause 11.2.1

27.15.3	Test purpose

To verify that a DCS 1800 ME rejects a SIM  with a GSM directory only.

27.15.4	Method of test

27.15.4.1	Initial conditions

The ME is installed with a SIM with a GSM directory.

27.15.4.2	Procedure

The MS is powered on, and activates the SIM.

27.15.5	Test requirement

The MS shall send a status information indicating 'current directory inconsistent'.

DCS1800 only - end

27.16	Secret code usage

27.16.1	Entry of PIN

27.16.1.1	Definition and applicability

The PIN is a number used to authenticate the user to the SIM for security.  Entry of the correct PIN allows PIN-protected data to be accessed over the SIM-ME interface.

This test applies to all ME.

27.16.1.2	Conformance requirement

Following insertion of the SIM and switching on the MS, the ME shall check the state of the PIN.  If the PIN is enabled, the ME asks the user for PIN verification.

The VERIFY CHV function verifies the PIN presented by the ME to the SIM.

Reference:

TS GSM 02.30, section 4.6.1; GSM 11.11, sections 8.9, 9.2.9 and 11.3.1.

27.16.1.3	Test purpose

1.	To verify that the PIN verification procedure is performed by the ME correctly.

2.	To verify that the GSM basic public MMI string is supported.

27.16.1.4	Method of test

27.16.1.4.1	Initial conditions

The ME is connected to a SIM or SIM-simulator with the PIN enabled, and powered off.

The default SIM is used,

27.16.1.4.2	Procedure

a)	The ME is powered on.

b)	When the MS is in mode "PIN check" enter "2468#".

27.16.1.5	Requirement

1)	The ME shall send a VERIFY CHV command to the SIM, with CHV number = '01'.

2)	The MS shall give an indication "OK", following a successful execution of the command.

27.16.2	Change of PIN

27.16.2.1	Definition and applicibility

The PIN may be changed by the user, by entering the old and new PINs.  The length of the PIN is between 4 and 8 digits.

This test applies to all MEs.

27.16.2.2	Conformance requirement

The ME shall support the change of PIN procedure as defined in 02.30 and 11.11.

Reference

TS GSM 02.30, section 4.6.2; GSM 11.11, sections 8.10, 9.2.10 and 11.3.2.

27.16.2.3	Test purpose

1.	To verify that the PIN substitution procedure is performed correctly by the ME.

2.	To verify that the GSM basic public MMI string is supported.

27.16.2.4	Method of test

27.16.2.4.1	Initial conditions

The ME is connected to a SIM or SIM-simulator with the PIN enabled.

The default SIM is used.

The ME is powered-on, with the correct PIN entered.

27.16.2.4.2	Procedure

a)	Enter "**04*2468*01234567*01234567#".

b)	The MS is switched off and on.

c)	When the MS is in mode "PIN-check", the sequence "01234567#" is entered.

d)	The MS is switched off and on.

e)	When the MS is in mode "PIN check" enter "2468#".

27.16.2.5	Requirement

1)	After step a), the ME shall send a CHANGE CHV command to the SIM, with CHV number set to '01'.

2)	Following the successful execution of the command, the MS shall give an indication that the new PIN is accepted.

3)	After step c), the MS shall give an indication "OK".

4)	After step e), the MS shall give an indication that the entered PIN is not accepted.

27.16.3	Disabling the PIN

27.16.3.1	Definition and applicability

Entry of the PIN may be disabled by the user, depending on the service table of the SIM.  It is the responsibility of the ME to check the SIM service table.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM that support a feature to disable the PIN.

27.16.3.2	Conformance requirement

Disabling PIN is achieved through the DISABLE CHV command.  If the PIN disable function in the SIM service table is not allocated or activated, then the ME shall not attempt to disable the PIN.

Reference:

TS GSM 11.11, sections 8.11, 9.2.11, 10.2.7, 11 and 11.3.3,

27.16.3.3	Test purpose

To verify that the ME does not attempt to disable the PIN.

27.16.3.4	Method of test

27.16.3.4.1	Initial conditions

The ME is connected to the SIM simulator.

Elementary files in the SIM simulator shall be default, with the exception of:

EFSST (SIM Service Table)

Logically:	CHV1 disable function not activated.

Abbreviated dialling numbers allocated and activated.

PLMN selector allocated and activated.

Fixed dialling numbers not activated.

Coding:	B1			B2			B3			B4

xx0x110x		0011xxxx		xxxxxxxx		0000xxxx (binary)

The coding of EFSST shall conform with the capabilities of the SIM simulator.

The ME is powered on and a correct PIN entered.

27.16.3.4.2	Procedure

Using the ME's MMI procedure, an attempt is made to disable the PIN.

27.16.3.5	Requirement

The ME shall not send a DISABLE CHV command across the SIM/ME interface.

27.16.4	PUK entry

27.16.4.1	Definition and applicability

After three consecutive wrong entries of the PIN, the PIN becomes blocked.  The PUK is used to unblock the PIN.  This function may be performed whether or not the PIN is blocked.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM.

27.16.4.2	Conformance requirement

The ME shall support the procedure to unblock PIN using PUK, as defined in 02.30 and 11.11.

Reference:

TS GSM 02.30, section 4.6.3; GSM 11.11, sections 8.13, 9.2.13 and 11.3.5.

27.16.4.3	Test purpose

1.	To verify that the CHV unblocking procedure is performed correctly.

2.	To verify that the GSM basic public MMI string is supported.

27.16.4.4	Method of test

27.16.4.4.1	Initial conditions

The ME is connected to the SIM simulator.

The default SIM is used.

27.16.4.4.2	Procedure

a)	The ME is powered on.

b)	Enter "**05*13243546*1234*1234#"

c)	The ME is powered off and on.

d)	Enter the new PIN:  "1234".

e)	The ME is powered off and on.

f)	Enter a wrong PIN three times.

g)	Enter "**05*13243546*2468*2468#".

h)	The ME is powered off and on.

i)	Enter the new PIN:  "2468".

27.16.4.5	Requirements

1.	After step b), the ME shall send an UNBLOCK CHV command to the SIM, with CHV number = '00'.

2.	After step d), the ME shall indicate that the PIN has been accepted.

3.	After step f), the ME shall indicate that the PIN has been blocked.

4.	After step g), the ME shall send an UNBLOCK CHV command to the SIM, with CHV number = '00'.

5.	After step j), the ME shall indicate that the PIN has been accepted.

27.16.5	Entry of PIN2

27.16.5.1	Definition and applicability

PIN2 is a number used to authenticate the user to the SIM for security.  Entry of the correct PIN2 allows PIN2-protected data to be accessed over the SIM-ME interface.

This test applies to all ME that support a feature requiring entry of PIN2, such as AoC or FDN.

27.16.5.2	Conformance requirement

Where entry of PIN2 is necessary for security access, the ME shall indicate that PIN2 is to be entered.

The VERIFY CHV function verifies the PIN presented by the ME to the SIM.

Reference:

TS GSM 02.30, section 4.6.1; GSM 11.11, sections 8.9, 9.2.9, and 11.3.1.

27.16.5.3	Test purpose

To verify that entry of PIN2 is processed by the ME correctly.

27.16.5.4	Method of test

27.16.5.4.1	Initial conditions

The ME is connected to a SIM or SIM-simulator and powered on, with the correct PIN entered.

A default FDN SIM is used.

27.16.5.4.2	Procedure

a)	A feature is accessed which requires the entry of PIN2, eg resetting ACM for Advice of Charge, or changing a Fixed Dialling Number.

b)	The MMI is used to enter PIN2:  "3579".

27.16.5.5	Requirement

1)	After step b), the ME shall send a VERIFY CHV command to the SIM, with CHV number = '02'.

2)	Following the successful execution of the command, the MS shall give an indication that PIN2 was accepted.

27.16.6	Change of PIN2

27.16.6.1	Definition and applicibility

The PIN2 may be changed by the user, by entering the old and new PIN2s.  The length of the PIN is between 4 and 8 digits.

This test applies to all MEs that support PIN2.

27.16.6.2	Conformance requirement

The ME shall support the change of PIN2 procedure as defined in 02.30 and 11.11.

Reference

TS GSM 02.30, section 4.6.2; GSM 11.11, sections 8.10, 9.2.10 and 11.3.2.

27.16.6.3	Test purpose

1.	To verify that PIN2 substitution procedure is performed correctly by the ME.

2.	To verify that the GSM basic public MMI string is supported.

27.16.6.4	Method of test

27.16.6.4.1	Initial conditions

The ME is connected to a SIM or SIM-simulator.

The default FDN SIM is used, with PIN enabled.

The ME is powered on, with the correct PIN entered.

27.16.6.4.2	Procedure

a)	Enter "**042*3579*12345678*12345678#".

b)	The MS is switched off and on, and PIN entered:  "2468".

c)	Enter "**042*3579*12345678*12345678#".

d)	Enter "**042*12345678*3579*3579#".

27.16.6.5	Requirement

1)	After step a), the ME shall send a CHANGE CHV command to the SIM, with CHV number set to '02'.

2)	Following the successful execution of the command, the MS shall give an indication that the new PIN2 is accepted.

3)	After step c), the MS shall give an indication that the new PIN2 is not accepted.

4)	After step d), the MS shall give an indication that the new PIN2 is accepted.

27.16.7	PUK2 entry

27.16.7.1	Definition and applicability

After three consecutive wrong entries of PIN2, it becomes blocked.  PUK2 is used to unblock PIN2.  This function may be performed whether or not PIN2 is blocked.

This test applies to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM, that support PIN2.

27.16.7.2	Conformance requirement

The ME shall support the procedure to unblock PIN2 using PUK2, as defined in 02.30 and 11.11.

Reference:

TS GSM 02.30, section 4.6.3; GSM 11.11, sections 8.13, 9.2.13 and 11.3.5.

27.16.7.3	Test purpose

1.	To verify that the PUK2 unblock procedure is performed correctly by the ME.

2.	To verify that the GSM basic public MMI string is supported.

27.16.7.4	Method of test

27.16.7.4.1	Initial conditions

The ME is connected to the SIM simulator.

The default FDN SIM is used, with PIN enabled.

27.16.7.4.2	Procedure

a)	The ME is powered on and a correct PIN entered.

b)	Enter "**052*08978675*1234*1234#"

c)	The MS is powered off and on, and PIN entered:  "2468".

d)	A feature is selected requiring the entry of PIN2, and the new PIN2 "1234" is entered.

e)	A wrong PIN2 is entered three times.

f)	Enter "**052*08978675*3579*3579#".

g)	A feature is selected requiring the entry of PIN2, and the new PIN2 "3579" is entered.

27.16.7.5	Requirements

1.	After step b), the ME shall send an UNBLOCK CHV command to the SIM, with CHV number = '02'

2.	After step d), the ME shall send a VERIFY CHV command, with CHV number = '02'.  Following the successful execution of the command, the ME shall indicate that the PIN2 has been accepted.

3.	After step e), the ME shall indicate that PIN2 has been blocked.

4.	After step f), the ME shall send an UNBLOCK CHV command to the SIM, with CHV number = '02'

5.	After step g), the ME shall indicate that PIN2 has been accepted.

27.17	Abbreviated Dialling Numbers

27.17.1	Definition and applicability

Abbreviated Dialling Numbers contain subscriber number and supplementary service control strings.  They may also contain alpha identifiers.

This test applies to both GSM and DCS1800 MEs using either ID-1 or plug-in SIMs, that support ADN.

27.17.2	Conformance requirement

The ME shall be able to manage the storage and retrieval  of ADNs from the SIM, and set up calls to these numbers.

Reference:

GSM TS 02.07, section 3.1; GSM 11.11, section 11.5.1.

27.17.3	Test Purpose

To verify that the ME manages the storage and retrieval of ADNs from the SIM.

27.17.4	Method of Test

27.17.4.1	Initial conditions

Coding of elementary files in the SIM shall be as default, with the addition of:

EFADN (Abbreviated Dialling Number)

Logically:

At least 101 records.

Record 1:

Length of alpha identifier:		32 characters

Alpha identifier:			"ABCDEFGHIJKLMNOPQRSTUVWXYZABCDEF"

Length of BCD number:		'03'

TON and NPI:			Telephony and Unknown

Dialled number:			123

CCI:					None

Ext1:					None

Coding for record 1:

B1	B2	B3	...	B32	B33	B34	B35	B36	B37	B38	B39	...	B46

41	42	43	...	46	03	81	21	F3	FF	FF	FF	...	FF

The ME is installed with the default SIM or SIM simulator, and switched on.

27.17.4.2	Procedure

a)	The code "+123456789012345" is stored (entered) in the MS as abbreviated dialling number 7.

b)	The code "00112233" is stored (entered) in the MS as abbreviated dialling number 6.

c)	The code "**21*44556677#" is stored (entered) in the MS as abbreviated dialling number 101.

d)	Enter "7#".

e)	Enter "6#"

f)	Enter "101#"

g)	Enter "1#", and display the alpha identifier.

27.17.5	Requirements

1)	After step d), the number "+123456789012345" shall be displayed.

2)	After step e), the number "00112233" shall be displayed.

3)	After step f), the number "**21*44556677#" (or an equivalent representation) shall be displayed.

4)	After step g), the ME shall display at least part of the alpha identifier, and shall sustain normal operation.

27.18	MMI reaction to SIM status encoding

27.18.1	Definition and applicability

The SIM gives status information in response to instructions, as two-byte codes.  Some of these codes give valuable information to the user, and appropriate indication by the ME is mandatory.

This test applies to GSM and DCS1800 MEs using either ID-1 or plug-in SIMs.

27.18.2	Conformance requirement

It is mandatory to give the user an appropriate indication when any of the codes given below appear.

Reference

TS GSM 02.30, section 4.6.5.

27.18.3	Test purpose

To verify that the ME gives an appropriate indication to the user in response to status information return codes from the SIM.

27.18.4	Method of test

27.18.4.1	Initial conditions

The ME is connected to the SIM simulator.  All elementary files are coded as default.

The ME is powered on.

27.18.4.2	Procedure.

The SIM simulator is used to send the following error codes as reaction on an instruction from the ME:

-	9240	Memory Problem

-	9804	Access security policy not fulfilled or secret code rejected

-	9840	Secret code locked

-	6FXX	Technical problem with no diagnostic given as reaction on an instruction from the ME.

27.18.5	Requirement

For each error code, the ME shall give an appropriate MMI indication.

27.19	Electrical tests

Purpose of the test

Testing of electrical characteristics of the SIM/ME interface.

Whilst non-conformance in this area would be unlikely to cause difficulties to other users or the network (type approval criteria), significant deviations from the specifications (GSM 11.11 and ISO 7816) may damage the SIM. If an attempt is then made to use the SIM in a different ME, then its failure may reflect badly on both that ME and the network.

This sequence lists the electrical tests to be performed.

They include :

i) tests during activation and deactivation phases; and

ii) tests to be performed on each contact in both static and dynamic states: e.g. voltages, currents and signal characteristics.

However, due to the likely difficulty of accessing the terminals of the SIM/ME interface for the purposes of measurements, the ME manufacturer shall provide a test interface in accordance with section 36.5 for the purpose of conformance testing.

These tests are mandatory for all SIM or plug-in SIM/ME interfaces.

General measurement conventions

The measurement conventions are specified in ISO/IEC 7816-3 section 4.2.1.

27.19.1		Test of the Power Transition Phases

27.19.1.1	Phase preceding ME power on

27.19.1.1.1	Definition and applicability

When the mobile equipment is switched off, the contacts of the SIM/ME interface remain in an inactive state in order to prevent any damage to the SIM.

This test applies to GSM and DCS1800 MEs using either ID-1 or plug-in SIM.

27.19.1.1.2	Conformance Requirement

The residual voltage across the contacts of the SIM/ME interface (C1, C2, C3, C6, C7) shall not exceed +/- 0.4 Volts referenced to GND.

Reference

GSM TS 11.11, section 4.3.2

27.19.1.1.3	Test purpose

To verify that the residual voltage across the contacts of the SIM/ME interface (C1, C2, C3, C6, C7) is not greater than +/- 0.4 Volts referenced to GND.

27.19.1.1.4	Method of test

27.19.1.1.4.1	Initial condition

The ME is connected to a SIM Simulator.

The contact C1 (Vcc) of the SIM/ME interface is loaded with an impedance of 10 kOhm.

The other contacts (C2, C3, C6, C7) are loaded with an impedance of 50 kOhm.

27.19.1.1.4.2	Procedure

The residual voltage on each contact is measured.

27.19.1.1.5	Requirement

The residual voltage on each contact shall not exceed +/- 0.4 Volts referenced to GND.

27.19.1.2	Phase during SIM power on

27.19.1.2.1	Definition and applicability

When the mobile equipment is switched on, the contacts of the SIM/ME interface are activated in a defined sequence in order to prevent any damage to the SIM.

This test applies to GSM and DCS1800 MEs using either ID-1 or plug-in SIM.

27.19.1.2.2	Conformance Requirement

When the MS is soft powered on, the contacts of the SIM/ME interface shall be activated in the following order:

1 -	RST in state L,

2 -	Vcc powered,

3 -	I/O (ME) in reception mode,

4 -	Clock signal provided with a suitable and stable clock.

When Vpp is connected to Vcc, as allowed by GSM 11.11 (clause 5.3), then Vpp is activated together with Vcc, at the time of Vcc (step 2 in the sequence above).

Reference

GSM TS 11.11, section 4.3.2

27.19.1.2.3	Test purpose

To verify that the contacts of the SIM/ME interface are activated in the following order:

1 -	RST in state L,

2 -	Vcc powered,

3 -	I/O (ME) in reception mode,

4 -	Clock signal provided with a suitable and stable clock.

When Vpp is connected to Vcc, as allowed by GSM 11.11 (clause 5.3), then Vpp is activated together with Vcc, at the time of Vcc (step 2 in the sequence above).

27.19.1.2.4	Method of test

27.19.1.2.4.1	Initial condition

The ME is connected to a SIM Simulator.

27.19.1.2.4.2	Procedure

The MS is soft powered on.

The SIM/ME interface is monitored until it is fully activated (see ISO/IEC 7816-3 section 5.1).

27.19.1.2.5	Requirement

The contacts of the SIM/ME interface shall be activated in the following order:

1 -	RST in state L,

2 -	Vcc powered,

3 -	I/O (ME) in reception mode,

4 -	Clock signal provided with a suitable and stable clock.

When Vpp is connected to Vcc, as allowed by GSM 11.11 (clause 5.3), then Vpp is activated together with Vcc, at the time of Vcc (step 2 in the sequence above).

27.19.1.3	Phase during power off

27.19.1.3.1	Definition and applicability

When the mobile station is soft powered off, the contacts of the SIM/ME interface are deactivated in a defined sequence in order to prevent any damage to the SIM.

This test applies to GSM and DCS1800 MEs using either ID-1 or plug-in SIM.

27.19.1.3.2	Conformance Requirement

The contacts of the SIM/ME interface shall be deactivated in the following order:

1 -	RST at low status,

2 -	Clock stopped at low status,

3 -	I/O at status A,

4 -	Vcc inactive.

When Vpp is connected to Vcc, as allowed by GSM 11.11 (clause 5.3), then Vpp is deactivated together with Vcc, at the time of Vcc (step 5 in the sequence above).

Reference

GSM TS 11.11, section 4.3.2

27.19.1.3.3	Test purpose

To verify that the contacts of the SIM/ME interface become deactivated in the following order:

1 -	RST at low status,

2 -	Clock stopped at low status,

3 -	Vpp inactive (if Vpp was provided, See GSM 11.11 section 5.3.),

4 -	I/O at status A,

5 -	Vcc inactive.

When Vpp is connected to Vcc, as allowed by GSM 11.11 (clause 5.3), then Vpp is deactivated together with Vcc, at the time of Vcc (step 5 in the sequence above).

27.19.1.3.4	Method of test

27.19.1.3.4.1	Initial condition

The ME is connected to a SIM Simulator.

27.19.1.3.4.2	Procedure

The MS is soft powered off.

The SIM/ME interface is monitored until it is fully deactivated (See ISO/IEC 7816-3 section 5.4).

27.19.1.3.5	Requirement

The contacts of the SIM/ME interface shall be deactivated in the following order:

1 -	RST at low status,

2 -	Clock stopped at low status,

3 -	Vpp inactive (if Vpp was provided, See GSM 11.11 section 5.3),

4 -	I/O at status A,

5 -	Vcc inactive.

When Vpp is connected to Vcc, as allowed by GSM 11.11 (clause 5.3), then Vpp is deactivated together with Vcc, at the time of Vcc (step 5 in the sequence above).

27.19.2	Electrical tests on each ME contact

The electrical values, as given in the following table, must apply to all contacts, other than that under test, under both normal and extreme test conditions as defined in Annex 1, TC2.1 and TC2.2.

Contacts	Low level	High level	Max capacitive load

from test equipment

C1		V = 5V +/-10%

(Vcc)		I = 10 mA

C2	0V <= V <= 0.6V	4V <= V <= Vcc	30 pF

(RST)	I = -200mA	I = +20mA

C3	0V <= V <= 0.6V	4V <= V <= Vcc	30 pF

(CLK)	I = -200mA	I = +20mA

C5	0V	0V

(GND)

C6		5V +/-10%

(Vpp)

I	0V <= V <= 0.8V	0.7*Vcc <= V <= Vcc

I = -1mA	I = +/-20mA

C7			30 pF

(I/O) O	0V <= V <= 0.4V	3.8V <= V <= Vcc

I = 1mA	I = -20mA

Note :	Measurements of contacts voltage levels can be done at any time since the beginning of activation of the SIM and the end of deactivation of the SIM (ISO/IEC 7816-3 section 5.1).

Note :	The reference point of all measurements is the contact C5 (Ground).

Note :	Currents flowing into the SIM are considered positive.

27.19.2.1	Electrical tests on contact C1

C1 = Card power supply (Vcc)

27.19.2.1.1	Test 1

27.19.2.1.1.1	Definition and applicability

When the mobile station is activated, the supply voltage on the SIM/ME interface remains in the nominal range in order to ensure correct operation and preventing any damage to the SIM.

This test applies to GSM and DCS1800 MEs using either ID-1 or plug-in SIM.

27.19.2.1.1.2	Conformance Requirement

The voltage on contact C1 of the SIM/ME interface shall be 5V +/- 10% for Icc up to 10 mA.

Reference

GSM TS 11.11, section 5.1.

27.19.2.1.1.3	Test purpose

To verify that the voltage on contact C1 of the SIM/ME interface is held at 5V +/- 10%.

27.19.2.1.1.4	Method of test

27.19.2.1.1.4.1	Initial condition

The ME is connected to a SIM Simulator.

The MS is activated.

The remaining contacts of the SIM/ME interface are in nominal test conditions (See 11.10 section 27.19.2).

27.19.2.1.1.4.2	Procedure

The voltage of contact C1 (Vcc) of the SIM/ME interface is measured.

27.19.2.1.1.5	Requirement

The voltage on contact C1 of the SIM/ME interface shall be 5V +/- 10% for Icc up to 10 mA.

27.19.2.1.2	Test 2

27.19.2.1.2.1	Definition and applicability

When the mobile station is activated, the supply voltage on the SIM/ME interface shall be able to counteract spikes in the current consumption of the SIM up to a maximum charge of 40 nAs with no more than 400 ns duration and an amplitude of at most 200 mA, ensuring that the supply voltage stays in the specified range.

This test applies to GSM and DCS1800 MEs using either ID-1 or plug-in SIM.

27.19.2.1.2.2	Conformance Requirement

The voltage on contact C1 of the SIM/ME interface shall be 5V +/- 10% for spikes in the current consumption with a maximum charge of 40 nAs with no more than 400 ns duration and an amplitude of at most 200 mA.

Reference

GSM TS 11.11, section 5.2.

27.19.2.1.2.3	Test purpose

To verify that the voltage on contact C1 of the SIM/ME interface is 5V +/- 10% for spikes in the current consumption with a maximum charge of 40 nA with no more than 400 ns duration and an amplitude of at most 200 mA.

27.19.2.1.2.4	Method of test

27.19.2.1.2.4.1	Initial condition

The ME is connected to a SIM Simulator.

The MS is activated.

The remaining contacts of the SIM/ME interface are held in nominal test condition (See 11.10 section 27.19.2).

27.19.2.1.2.4.2	Procedure

The voltage on contact C1 of the SIM/ME interface is monitored and the following current spikes are applied:

1)	a single spike:

Max current 200 mA

Duration (including rise and fall times as specified in 11.40) 200 ns

2)	a single spike:

Max current 100 mA

Duration (including rise and fall times as defined in GSM 11.40) 400 ns.

3)	continuous spikes:

Max current 20 mA

Min current  0 mA

Duration 100 ns

Pause 100 ns

4)	continuous spikes:

Max current 20 mA

Min current  0 mA

Duration 400 ns

Pause 400 ns

5)	continuous spikes:

Max current 20 mA

Min current  5 mA

Duration 150 ns

Pause 300 ns

6)	Pseudo random spikes:

Max current 20 mA

Min current  5 mA

Duration 200 ns

200 spikes/second

7)	Pseudo random spikes:

Max current 20 mA

Min current  5 mA

Duration 200 ns

1000 spikes/second

27.19.2.1.2.5	Requirement

The voltage on contact C1 of the SIM/ME interface shall be 5V +/- 10%.

27.19.2.2	Electrical tests on contact C2

C2 = Reset (RST)

27.19.2.2.1	Definition and applicability

When the mobile station is activated, the voltage on contact C2 of the SIM/ME interface remain in the nominal range in order to ensure correct operation and preventing any damage to the SIM.

This test applies to GSM and DCS1800 MEs using either ID-1 or plug-in SIM.

27.19.2.2.2	Conformance Requirement

The voltage on contact C2 (RST) of the SIM/ME interface shall be between -0.3V and +0.6V for a current of -200 uA in low state and between 4V and Vcc + 0.3V for a current of +20uA in high state.

Reference

GSM TS 11.11, section 5.

27.19.2.2.3	Test purpose

To verify that the voltage on contact C2 (RST) of the SIM/ME interface remains between -0.3V and +0.6V for a current of -200 uA in low state and between 4V and Vcc + 0.3V for a current of +20uA in high state.

27.19.2.2.4	Method of test

27.19.2.2.4.1	Initial condition

The ME is connected to a SIM Simulator.

The MS is activated.

The remaining contacts of the SIM/ME interface are held in nominal test conditions (See 11.10 section 27.19.2).

27.19.2.2.4.2	Procedure

The voltage on contact C2 (RST) of the SIM/ME interface is measured.

27.19.2.2.5	Requirement

The voltage on contact C2 (RST) of the SIM/ME interface shall be between -0.3V and +0.6V for a current of -200 uA in low state and between 4V and Vcc + 0.3V for a current of +20uA in high state.

27.19.2.3		Electrical tests on contact C3

C3 = Clock (CLK)

27.19.2.3.1	Definition and applicability

When the mobile station is activated, the voltage, the rise/fall time of the signal, the clock cycle ratio and the frequency on contact C3 of the SIM/ME interface remain in the nominal range in order to ensure correct operation and preventing any damage to the SIM.

This test applies to GSM and DCS1800 MEs using either ID-1 or plug-in SIM.

27.19.2.3.2	Conformance Requirement

The voltage on contact C3 (CLK) of the SIM/ME interface shall be between -0.3V and +0.6V for a current of -200 uA in low state and between 4V and Vcc + 0.3V for a current of +20uA in high state.

The rise and the fall time of the clock signal shall not exceed 9% of the total time period and shall not exceed 0.5 us.

The cycle ratio of the clock signal shall be between 40% and 60% of the period, in steady state.

The frequency of the clock signal shall be between 1 MHz and 5 MHz.

Reference

GSM TS 11.11, section 5 and section 5.4.

27.19.2.3.3	Test purpose

To verify that the voltage on contact C3 (CLK) of the SIM/ME interface is between -0.3V and +0.6V for a current of -200 uA in low state and between 4V and Vcc + 0.3V for a current of +20uA in high state.

To verify that the rise and the fall time of the clock signal does not exceed 9% of the total time period and shall not exceed 0.5 us.

To verify that the cycle ratio of the clock signal is between 40% and 60% of the period, in steady state.

To verify that the frequency of the clock signal is between 1 MHz and 5 MHz.

27.19.2.3.4	Method of test

27.19.2.3.4.1	Initial condition

The ME is connected to a SIM Simulator.

The MS is activated.

The remaining contacts of the SIM/ME interface are held in nominal test conditions (See 11.10 section 27.19.2).

27.19.2.3.4.2	Procedure

The voltage, the rise/fall time, the clock cycle ratio and the frequency on contact C3 (CLK) of the SIM/ME interface are measured.

27.19.2.3.5	Requirement

The voltage on contact C3 (CLK) of the SIM/ME interface shall be between -0.3V and +0.6V for a current of -200 uA in low state and between 4V and Vcc + 0.3V for a current of +20uA in high state.

The rise and the fall time of the clock signal shall not exceed 9% of the total time period and shall not exceed 0.5 us.

The cycle ratio of the clock signal shall be between 40% and 60% of the period, in steady state.

The frequency of the clock signal shall be between 1 MHz and 5 MHz.

27.19.2.4		Electrical tests on contact C6

C6 = Programming Voltage (VPP)

27.19.2.4.1	Definition and applicability

The ME need not to provide any voltage on the contact C6 of the SIM/ME interface even if this contact is present. In order not to damage other chipcards which may need a programming voltage, this contact shall not be connected to ground. Contact C6 (VPP) also may be connected to VCC.

This test applies to GSM and DCS1800 MEs using either ID-1 or plug-in SIM.

27.19.2.4.2	Conformance Requirement

Contact C6 (VPP) of the SIM/ME interface shall not be connected to ground.

Reference

GSM TS 11.11, section 5.3.

27.19.2.4.3	Test purpose

To verify that contact C6 (VPP) of the SIM/ME interface is not connected to ground.

27.19.2.4.4	Method of test

27.19.2.4.4.1	Initial condition

The ME is powered off.

An OHM-Meter is connected across contact C6 and GND of the SIM/ME interface.

27.19.2.4.4.2	Procedure

The resistance is measured.

27.19.2.4.5	Requirement

The contact C6 (VPP) of the SIM/ME interface shall not be connected to ground.

27.19.2.5		Electrical tests on contact C7

C7 = Input - output (I/O)

27.19.2.5.1	Definition and applicability

When the mobile station is activated, the voltage and the rise/fall time of the signal on contact C7 of the SIM/ME interface is within the nominal range in order to ensure correct operation and preventing any damage to the SIM.

This test applies to GSM and DCS1800 MEs using either ID-1 or plug-in SIM.

27.19.2.5.2	Conformance Requirement

On the ME input:

The voltage on contact C7 (I/O) of the SIM/ME interface shall be between 0V and +0.4V for a current of +1 mA in low state and between 3.8V and Vcc + 0.5V for a current of -20uA in high state.

On the ME output:

The voltage on contact C7 (I/O) of the SIM/ME interface shall be between 0V and 0.8V for a current of -1 mA and between 0.7 * Vcc and Vcc + 0.5V for a current of +/- 20 uA in high state.

The rise time and the fall time of the I/O signal shall not exceed 1 us.

Reference

GSM TS 11.11, section 5.

27.19.2.5.3	Test purpose

To verify that the voltage on contact C7 (I/O) of the SIM/ME interface is between 0V and +0.4V for a current of +1 mA in low state and between 3.8V and Vcc + 0.5V for a current of -20uA in high state on the ME input.

To verify that the voltage on contact C7 (I/O) of the SIM/ME interface is between 0V and 0.8V for a current of -1 mA and between 0.7 * Vcc and Vcc + 0.5V for a current of +/- 20 uA in high state on the ME output.

To verify that the rise time and the fall time of the I/O signal not exceeds 1 us.

27.19.2.5.4	Method of test

27.19.2.5.4.1	Initial condition

The ME is connected to a SIM Simulator.

The MS is activated.

The remaining contacts of the SIM/ME interface are held in nominal test conditions (See 11.10 section 27.19.2).

27.19.2.5.4.2	Procedure

The voltage and the rise/fall time on contact C7 (I/O) of the SIM/ME interface are measured.

27.19.2.5.5	Requirement

The voltage on contact C7 (I/O) of the SIM/ME interface shall remain between 0V and +0.4V for a current of +1 mA in low state and between 3.8V and Vcc + 0.5V for a current of -20uA in high state on the ME input.

The voltage on contact C7 (I/O) of the SIM/ME interface shall remain between 0V and 0.8V for a current of -1 mA and between 0.7 * Vcc and Vcc + 0.5V for a current of +/- 20 uA in high state on the ME output.

The rise and the fall time of the I/O signal shall not exceed 1 us.

27.20	Fixed Number Dialling

27.20.1	ME and SIM with FDN activated

27.20.1.1	Definition and applicability

Fixed Number Dialling (FDN) is a service defined for the SIM. An activated FDN service results in call restrictions for the MS. Only directory numbers which are stored in the EFFDN may be dialled by the MS. The call restrictions are controlled by the ME. To ascertain the type of SIM and state of FDN the MS runs the FDN capability request procedure during SIM/ME initialization.

This test apply to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM and supporting the FDN service.

27.20.1.2	Conformance requirement

1.	Recognizing the state of the SIM (FDN enabled) the MS correctly performs the SIM initialisation procedure.

2.	The MS allows call set-up to a directory number as stored in EFFDN.

3.	The MS allows call set-up to a directory number as stored in EFFDN and extended by digits in the end.

4.	The MS does not allow call set-up to a directory number stored in EFFDN but with missing digits at the end.

5.	The MS does not allow call set-up to a directory number having no reference in EFFDN.

6.	The MS allows call set-up of an emergency call.

Reference:

GSM 11.11, clauses 10.2.7, 10.3.2, 11.2.1 and 11.5.1, GSM 02.07, clause 3.2.

27.20.1.3	Test purpose

1.	To verify that the ME as a result of the state of the SIM rehabilitates EFIMSI and EFLOCI during SIM/ME initialization procedure.

2.	To verify that the ME allows call set-up to a FDN number.

3.	To verify that the ME allows call set-up to a FDN number extended by some digits in the end.

4.	To verify that the ME rejects call set-up to a FDN number being corrupted.

5.	To verify that the ME rejects call set-up to number having no reference in EFFDN.

6.	To verify that the ME allows emergency call set-up.

27.20.1.4	Method of test

27.20.1.4.1	Initial conditions

The SS transmits on the BCCH, with the following network parameters

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	246/81/value in SIM

Access control:		unrestricted

The default FDN-SIM with FDN service enabled is installed into the ME and the MS is powered on.

27.20.1.4.2	Procedure

a)	Using the MMI a call set-up to the fixed dialling number 1 is attempted.

b)	Using the MMI a call set-up to the fixed dialling number 2 extended by '123' in the end is attempted.

c)	Using the MMI a call set-up to a number which is equal to the fixed dialling number 3 without the last digit is attempted, e.g. by recalling the fixed dialling number 3 and deleting the last digit (only in display).

d)	Using the MMI a call set-up to the number '1234567' is attempted.

e)	Using the MMI an emergency call set-up is attempted.

27.20.1.5	Test requirement

1)	After steps a) and b) the MS shall allow call set-up and send the requested number across the air interface.

2)	After steps c) and d) the MS shall prevent call set-up.

3)	After step e) the MS shall allow emergency call set-up and send the requested number across the air interface.

27.20.2	ME and SIM with FDN deactivated

27.20.2.1	Definition and applicability

Fixed Number Dialling (FDN) is a service defined for the SIM. An activated FDN service results in call restrictions for the MS. Only directory numbers which are stored in the EFFDN may be dialled by the MS. The call restrictions are controlled by the ME. To ascertain the type of SIM and state of FDN the MS runs the FDN capability request procedure during SIM/ME initialization. Deactivation of the service by the subscriber is possible under the control of PIN2 and switches the SIM into a 'normal', non restrictive SIM.

This test apply to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM and supporting the FDN service.

27.20.2.2	Conformance requirement

1.	Recognizing the state of the SIM (FDN disabled) the MS correctly performs the SIM initialisation procedure.

2.	The MS allows call set-up to a directory number as stored in EFFDN.

3.	The MS allows call set-up to a directory number as stored in EFADN.

4.	The MS allows call set-up to a directory number given in manually.

Reference:

GSM 11.11, clauses 10.2.7, 10.3.2, 11.2.1 and 11.5.1, GSM 02.07, clause 3.2.

27.20.2.3	Test purpose

1.	To verify that the ME as a result of the state of the SIM correctly performs the SIM/ME initialization procedure.

2.	To verify that the ME allows call set-up to a FDN number.

3.	To verify that the ME allows call set-up to a ADN number.

4.	To verify that the ME allows call set-up to manually given number.

27.20.2.4	Method of test

27.20.2.4.1	Initial conditions

The SS transmits on the BCCH, with the following network parameters

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	246/81/value in SIM

Access control:		unrestricted

The default FDN SIM with FDN service disabled is installed into the ME and the MS is powered on.

27.20.2.4.2	Procedure

a)	Using the MMI a call set-up to the fixed dialling number 1 is attempted.

b)	Using the MMI a call set-up to the abbreviated dialling number 1 is attempted.

c)	Using the MMI a call set-up to the number '1234567' is attempted.

27.20.2.5	Test requirement

After steps a), b) and c) the MS shall allow call set-up and send the requested number across the air interface.

27.20.3	Enabling, Disabling and Updating of FDN

27.20.3.1	Definition and applicability

FDN may be enabled and disabled by the subscriber under control of PIN2. Fixed dialling numbers are read with PIN and updated under control of PIN2.

This test apply to GSM and DCS1800 MEs using either ID-1 or Plug-in SIM and supporting the FDN service.

27.20.3.2	Conformance requirement

1.	Recognizing the state of the SIM (FDN enabled) the MS correctly performs the SIM initialisation procedure.

2.	The MS shall allow updating of EFFDN by the use of PIN2.

3.	The MS provides means to disable the FDN service by the use of PIN2.

4.	The MS shall allow the use of EFADN after disabling of FDN.

Reference:

GSM 11.11, clauses 10.2.7, 10.3.2, 11.2.1 and 11.5.1, GSM 02.07, clause 3.2.

27.20.3.3	Test purpose

1.	To verify that the ME as a result of the state of the SIM rehabilitates EFIMSI and EFLOCI during SIM/ME initialization procedure.

2.	To verify that the ME correctly performs the update of a number in EFFDN.

3.	To verify that the ME correctly disables FDN service.

4.	To verify that the ME recognizes disabling of FDN and allows access to EFADN.

27.20.3.4	Method of test

27.20.3.4.1	Initial conditions

The SS transmits on the BCCH, with the following network parameters

Attach/detach:		disabled

LAI (MCC/MNC/LAC):	246/81/value in SIM

Access control:		unrestricted

The default FDN SIM with FDN service enabled is installed into the ME and the MS is powered on.

27.20.3.4.2	Procedure

a)	Using the MMI the directory number '+876543210' is stored in EFFDN as fixed dialling number 1 (The alpha identifier is not changed).

b)	Using the MMI the FDN disabling procedure is performed. On request of the MS PIN2 is entered.

c)	Using the MMI a call set-up to the abbreviated dialling number 1 is attempted.

d)	The MS is soft-powered down.

27.20.3.5	Test requirement

1)	After step b) the MS shall indicate that FDN disabling procedure has been successful.

2)	After step c) the MS shall allow call set-up and send the requested number across the air interface.

3)	After step d) the SIM shall contain the following values:

EFFDN (Fixed Dialling Numbers)

Logically:

Record 1:

Length of alpha identifier:		6 characters

Alpha identifier:  		"FDN111"

Length of BCD number:		'06'

TON and NPI:			Telephony and International

Dialled number:			+876543210

CCI:				None

Ext1:				None

Coding for record 1:

B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12	B13

46	44	4E	31	31	31	06	91	78	56	34	12	F0

B14	B15	B16	B17	B18	B19	B20

FF	FF	FF	FF	FF	FF	FF

Record 2:

Length of alpha identifier:		6 characters

Alpha identifier:  		"FDN222"

Length of BCD number:		'04'

TON and NPI:			Telephony and Unknown

Dialled number:			+24680

CCI:				None

Ext1:				None

Coding for record 2:

B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12	B13

46	44	4E	32	32	32	04	81	42	86	F0	FF	FF

B14	B15	B16	B17	B18	B19	B20

FF	FF	FF	FF	FF	FF	FF

Record 3:

Length of alpha identifier:		6 characters

Alpha identifier:  		"FDN333"

Length of BCD number:		'0B'

TON and NPI:			Telephony and International

Dialled number:			+12345678901234567890

CCI:				None

Ext1:				None

Coding for record 3:

B1	B2	B3	B4	B5	B6	B7	B8	B9	B10	B11	B12	B13

46	44	4E	33	33	33	0B	91	21	43	65	87	09

B14	B15	B16	B17	B18	B19	B20

21	43	65	87	09	FF	FF

27.21	Short messages and cell broadcast

[TBD]

27.22	Phase identification

27.22.1	Definition and applicability

The phase of the SIM is indicated in the Elementary File EFPHASE.  This allows the ME to identify the phase of the SIM and adapt its functionality accordingly.

This test applies to GSM and DCS1800 MEs using either ID-1 or plug-in SIMs.

27.22.2	Conformance requirement

The phase of the card shall be determined as part of the initialisation procedure.

Reference:

TS GSM 11.11, sections 10.2.16 and 11.2.1.

27.22.3	Test Purpose

To verify that the ME requests the SIM phase as part of the initialisation procedure.

27.22.4	Method of test

27.22.4.1	Initial conditions

The ME is connected to the SIM simulator, and powered off.

The default values are used.

27.22.4.2	Procedure

a)	The mobile is powered on.

b)	The SIM simulator monitors the SIM initialisation procedure.

27.22.5	Test Requirement

The ME shall request the phase of the SIM as part of the initialisation procedure.

27.23	SIM presence detection

27.23.1	Definition and applicability

The presence of the SIM is an essential requirement for setting up and maintaining a call.  The ME detects the presence of the SIM electronically.

27.23.2	Conformance requirement

To ensure that the SIM has not been removed during a card session, the ME shall send STATUS commands at frequent intervals of no longer than 30s.  If the response data is not DFGSM, DFDCS1800 or DFTELECOM, then the call shall be terminated immediately.

Reference:

TS GSM 11.11, section 11.2.7.

27.23.3	Test purpose

1.	To verify that the ME sends STATUS messages at frequent intervals of no longer than 30s during a call.

2.	To verify that the ME clears a call immediately after receiving a wrong response to a STATUS command.

27.23.4	Method of test

27.23.4.1	Initial conditions

The ME is connected to the SIM-simulator.

All elementary files are coded as default.

27.23.4.2	Procedure

a)	A call is set up using the generic call setup.

b)	The SIM simulator monitors the time interval between STATUS commands sent by the ME.

c)	After 3 minutes, the call is cleared.

d)	A call is set up using the generic call setup.

e)	After one minute after the call was successfully set up, the SIM simulator responds to a STATUS command with the response data of the MF.

27.23.5	Requirements

1.	During step b), the time interval between STATUS commands shall not be longer than 30s.

2.	After step e), the ME shall clear the call immediately after receiving the wrong response to a STATUS command.

27.24	Advice of Charge

27.24.1	AoC not supported by SIM

27.24.1.1	Definition and applicability

If the ME under test supports Advice of Charge Charging, it shall still look at the capability of the SIM, before responding to any AoCC information from the network.

This test is applicable to all MEs supporting AoCC.

27.24.1.2	Conformance Requirement

1.	An MS not supporting AoCC and in the outgoing call / U4 call delivered state, on receipt of a CONNECT message containing AoCC information shall acknowledge the CONNECT message but ignore and not acknowledge the AoCC information sent within the CONNECT.

2.	An MS not supporting AoCC and in the outgoing call / U4 call delivered state, on receipt of a FACILITY message containing AoCC information shall ignore and not acknowledge the AoCC information sent within the FACILITY.

3.	An MS not supporting AoCC and in the incoming call / U9 call confirmed state, on receipt of a FACILITY message containing AoCC information shall ignore and not acknowledge the AoCC information sent within the FACILITY.

4.	An MS not supporting AoCC and in the U10 call active state, on receipt of a FACILITY message containing AoCC information, shall ignore and not acknowledge the AoCC information sent within the FACILITY.

References:	TS GSM 03.86, sections 1.2, 1.3, 2.2, 2.3; TS GSM 04.86, section 2.

27.24.1.3	Test Purpose

1.	To verify that an MS not supporting AoCC (where the ME does support AoCC but the SIM does not) and in the outgoing call / U4 call delivered state, on receipt of a CONNECT message containing AoCC information shall acknowledge the CONNECT message but ignore and not acknowledge the AoCC information sent within the CONNECT.

2.	To verify that an MS not supporting AoCC (where the ME does support AoCC but the SIM does not) and in the outgoing call / U4 call delivered state, on receipt of a FACILITY message containing AoCC information shall ignore and not acknowledge the AoCC information sent within the FACILITY.

3.	To verify that an MS not supporting AoCC (where the ME does support AoCC but the SIM does not) and in the incoming call / U9 call confirmed state, on receipt of a FACILITY message containing AoCC information shall ignore and not acknowledge the AoCC information sent within the FACILITY.

4.	To verify that an MS not supporting AoCC (where the ME does support AoCC but the SIM does not) and in the U10 call active state, on receipt of a FACILITY message containing AoCC information, shall ignore and not acknowledge the AoCC information sent within the FACILITY.

27.24.1.4	Method of Test

27.24.1.4.1	Initial conditions

The ME shall be installed with a SIM or SIM simulator, with all elementary files coded as for the default SIM, with the exception of:

EFSST (SIM Service Table)

Logically:	CHV1 disable function allocated and activated.

Abbreviated dialling numbers allocated and activated.

PLMN selector allocated and activated.

Fixed dialling numbers not activated.

AoC not activated.

Coding:	B1		B2		B3		B4

xx0x1111	0011xx0x	xxxxxxxx	0000xxxx (binary)

The coding of EFSST shall conform with the capabilities of the SIM used.

The generic call set up procedures are followed up to and including the reception, or transmission of the ALERTING message by the MS.

27.24.1.4.2	Procedure

a)	For an MO call in the U4 state the SS transmits CONNECT containing AoCC information.

b)	For an MO call in the U4 state the SS transmits FACILITY containing AoCC information.

c)	For an MO call in the U9 state the SS transmits FACILITY containing AoCC information.

d)	For an MO call in the U10 state the SS transmits FACILITY containing AoCC information.

27.24.1.5	Test Requirement

In all cases, the MS shall ignore the AoCC information sent to it in the Facility information elements as part of the CONNECT/FACILITY messages  and not send any AoCC information acknowledgement.  It shall be checked for 15 seconds that the MS does not transmit any AoCC information acknowledgement after the receipt of AoCC information.

27.24.2	Maximum frequency  of ACM updating

27.24.2.1	Definition and applicability

The ACM shall be updated at the end of every interval, where the interval length is given by parameter e2.  The ME shall update the ACM not more frequently than once every 5 seconds, even if the interval is less than 5 seconds.  More frequent updating may affect the SIM's read/write cycles.

This test applies to all ME supporting AoC.

27.24.2.2	Conformance requirement

The ACM shall be incremented when the CCM is incremented or once every 5 seconds, whichever is the longer period.

Reference:

TS GSM 02.24, section 4.3, part h.

27.24.2.3	Test Purpose

To verify that the interval between increments is  5 seconds.

27.24.2.4	Method of test

27.24.2.4.1	Initial conditions

The ME shall be connected to the SIM simulator, with all elementary files coded as default with the exception of:

EFSST (SIM Service Table)

Logically:	CHV1 disable function allocated and activated.

Abbreviated dialling numbers allocated and activated.

PLMN selector allocated and activated.

Fixed dialling numbers not activated.

AoC allocated and activated.

Coding:	B1		B2		B3		B4

xx0x1111	0011xx11	xxxxxxxx	0000xxxx (binary)

The coding of EFSST shall conform with the capabilities of the SIM used.

EFACM (Accumulated call meter)

Logically:	50 units

EFACMM (Accumulated call meter maximum)

Logically:	150 units

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

27.24.2.4.2	Procedure

a)	The MS is made to initiate a call. The call is established with  AOCC e-parameters sent in a Facility IE in the CONNECT message, as given below. The MS returns the AOCC acknowledgement within 1 second of the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent by the MS before or after the CONNECT ACKNOWLEDGE.

b)	The call is maintained for 90 seconds, then terminated by the SS.  During the call, the SIM-simulator monitors the time intervals between successive INCREMENT commands.

Maximum Duration of Test:

2 minutes

Expected Sequence:

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate a

call

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	CALL PROCEEDING

8	SS -> MS	ASSIGNMENT COMMAND

to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	SS -> MS	CONNECT	As default message

except contains Facility IE

with contents as indicated

in i below

Either A or B branch is

taken

A12	MS -> SS	CONNECT ACKNOWLEDGE

A13	MS -> SS	FACILITY	As default message

except contains Facility IE

with contents as indicated

in ii below

B12	MS -> SS	FACILITY	As default message

except contains Facility IE

with contents as indicated

in ii below

B13	MS -> SS	CONNECT ACKNOWLEDGE

14			call duration 90 seconds

after CAI information

sent by SS,

15	SS -> MS	DISCONNECT

16	MS -> SS	RELEASE

17	SS -> MS	RELEASE COMPLETE

18	SS -> MS	CHANNEL RELEASE	The main

signalling link is released.

Specific Message Contents:

i) FACILITY Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.1.3.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as below:

e-parameters

parameter		1	2	3	4	5	6	7

value		1	1	1	0	0	0	0

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.1.3.

27.24.2.5	Test Requirement

The MS shall send INCREMENT commands to the SIM every 5 seconds.

27.24.3	Call terminated when ACM greater than ACMM

27.24.3.1	Definition and applicability

ACMM gives the maximum value of ACM, at which the current calls shall be terminated and no further charged calls may be made (except emergency calls).

This test applies to all ME supporting AoCC.

27.24.3.2	Conformance requirement

ACM shall be incremented by the value of CCM.

If the ACMM is valid, and the ACM becomes equal to or exceeds the value of the ACMM, then all calls in progress, chargeable to the user, shall be terminated by the MS (except emergency calls).

Reference

TS GSM 02.24, section 4.3 part h and section 4.2.2.

27.24.3.3	Test purpose

1.	To verify that the ME increments the ACM by the correct number of units, even though this may take ACM above ACMM.

2.	To verify that the ME terminates the call.

27.24.3.4	Method of test

27.24.3.4.1	Initial conditions

The ME shall be connected to a SIM or the SIM simulator, with all elementary files coded as default with the exception of:

EFSST (SIM Service Table)

Logically:	CHV1 disable function allocated and activated.

Abbreviated dialling numbers allocated and activated.

PLMN selector allocated and activated.

Fixed dialling numbers not activated.

AoC allocated and activated.

Coding:	B1			B2			B3			B4

xx0x1111	0011xx11	xxxxxxxx	0000xxxx (binary)

The coding of EFSST shall conform with the capabilities of the SIM used.

EFACM (Accumulated call meter)

Logically:	80 units

EFACMM (Accumulated call meter maximum)

Logically:	94 units

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

27.24.3.4.2	Procedure

a)	The MS is made to initiate a call. The call is established with  AOCC e-parameters sent in a Facility IE in the CONNECT message, as given below. The MS returns the AOCC acknowledgement within 1 second of the CONNECT message.  It is an implementation option whether the AOCC acknowledge is sent by the MS before or after the CONNECT ACKNOWLEDGE.

b)	The call is maintained until cleared by the MS (after 20 seconds).

c)	The contents of ACM are checked.

Maximum Duration of Test:

2 minutes

Expected Sequence:

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate a 				call

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	CALL PROCEEDING

8	SS -> MS	ASSIGNMENT COMMAND

to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	SS -> MS	CONNECT	As default message

except contains Facility IE

with contents as indicated

in i below

Either A or B branch is

taken

A12	MS -> SS	CONNECT ACKNOWLEDGE

A13	MS -> SS	FACILITY	As default message

except contains Facility IE

with contents as indicated

in ii below

B12	MS -> SS	FACILITY	As default message

except contains Facility IE

with contents as indicated

in ii below

B13	MS -> SS	CONNECT ACKNOWLEDGE

14			call duration 20 seconds

after CAI information

sent by SS

15	MS -> SS	DISCONNECT

16	SS -> MS	RELEASE

17	MS -> SS	RELEASE COMPLETE

18	MS -> SS	CHANNEL RELEASE	The main

signalling link is released.

Specific Message Contents:

i) FACILITY Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.1.3.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as below:

e-parameters

parameter		1	2	3	4	5	6	7

value		10	10	1	0	0	0	0

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.1.3.

27.24.3.5	Test Requirement

1)	The MS shall terminate the call correctly after 20 seconds, when ACM exceeds ACMM.

2)	The value of ACM shall be 100 units.

27.24.4	Response codes of increase command

27.24.4.1	Definition and applicability

ACM has a maximum value in terms of coding, and an attempt by the ME to exceed that value by sending an INCREASE command shall result in an error message from the SIM.

This test applies to all MEs supporting AoCC.

27.24.4.2	Conformance Requirement

The ME shall perform the increasing procedure, sending the amount to be increased.

The running accumulated charge shall be stored in the ACM of the SIM.

Where this charge cannot be stored in the MS, use of the telecommunications service shall be prevented.

References:

TS GSM 11.11, section 11.5.3; 02.86, sections 2.2.1 and 2.1.

27.24.4.3	Test Purpose

To verify that the ME clears a charged call if the SIM indicates that the ACM has reached its maximum value.

27.24.4.4	Method of Test

27.24.4.4.1	Initial conditions

The ME shall be connected to the SIM simulator, with all elementary files coded as default with the exception of:

EFSST (SIM Service Table)

Logically:	CHV1 disable function allocated and activated.

Abbreviated dialling numbers allocated and activated.

PLMN selector allocated and activated.

Fixed dialling numbers not activated.

AoC allocated and activated.

Coding:	B1		B2		B3		B4

xx0x1111	0011xx11	xxxxxxxx	0000xxxx (binary)

The coding of EFSST shall conform with the capabilities of the SIM used.

EFACM (Accumulated call meter)

Logically:	(Maximum-10) units

EFACMM (Accumulated call meter maximum)

Logically:	(Maximum-2) units

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

27.24.4.4.2	Procedure

a)	The MS is made to initiate a call. The call is established with  AOCC e-parameters sent in a Facility IE in the CONNECT message, as given below. The MS returns the AOCC acknowledgement within 1 second of the CONNECT message.  It is an implementation option whether the AOCC acknowledge is sent by the MS before or after the CONNECT ACKNOWLEDGE.

b)	After an interval has elapsed, the ME  increments the ACM.  When an INCREASE command is received, the SIM-sim sends back the error '98 50'.

c)	Conditions are reset to those described in the initial conditions. Steps a) and b) of the test are repeated, except that the error code sent by the SIM simulator at step b) is now '6F xx'.

d)	Conditions are reset to those described in the initial conditions. Steps a) and b) of the test are repeated, except that the error code sent by the SIM simulator at step b) is now '92 50'.

Maximum Duration of Test:

3 minutes

Expected Sequence:

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate a 				call

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	CALL PROCEEDING

8	SS -> MS	ASSIGNMENT COMMAND

to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	SS -> MS	CONNECT	As default message

except contains Facility IE

with contents as indicated

in i below

Either A or B branch is

taken

A12	MS -> SS	CONNECT ACKNOWLEDGE

A13	MS -> SS	FACILITY	As default message

except contains Facility IE

with contents as indicated

in ii below

B12	MS -> SS	FACILITY	As default message

except contains Facility IE

with contents as indicated

in ii below

B13	MS -> SS	CONNECT ACKNOWLEDGE

14			call duration approx 10s

after CAI information

sent by SS

15	MS -> SS	DISCONNECT

16	SS -> MS	RELEASE

17	MS -> SS	RELEASE COMPLETE

18	MS -> SS	CHANNEL RELEASE	The main

signalling link is released.

Specific Message Contents:

i) FACILITY Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as below:

e-parameters

parameter		1	2	3	4	5	6	7

value		20	10	1	0	0	0	0

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

27.24.4.5	Test Requirement

In each of the three cases, as described in steps b), c) and d) of the procedure, the MS shall terminate the call correctly when it receives an indication from the SIM that the ACM cannot be incremented.