29.	Testing of bearer services

29.1	General

In GSM 07.01, section 2 the reference configurations for access to the data services of a GSM PLMN are described. For testing purposes only the following configurations will be considered:

-	MT2 configuration (Um- and R-interface available for testing)

-	MT0 configuration (only Um-interface available for testing) where it is possible to enable the MS to issue or accept a data call and send data over the Um-interface. A MT1 connected to an ISDN TE is considered as MT0.

For efficient testing it is essential that an MT0 configuration has some means to specifically activate every function towards the Um-interface the MS will perform during operation.

The correctness of the data bits transferred to the Um-interface cannot be tested in the MT0 configuration. However the correctness of the GSM 04.21 frames sent by the MS will be tested.

Testing the S-interface for the MT1 configuration is for further study.

For some tests it is of no importance whether the call is MO or MT. However, there might be MT0 configurations allowing the call to be established only from one side. In this case the appropriate actions shall be taken to establish the call.

In all other cases the data call shall be set up by the SS (i.e. MT) with an appropriate BCIE which is supported by the MS.

At the beginning of all tests the MS shall be in the idle updated state.

29.2	Testing of transparent data services

During all the tests the GSM 04.21 frames received as output of the channel coder in the SS shall be checked for correctness against GSM 04.21; this means checking that

-	S bits are coded as zeroes unless otherwise specified

-	the E bits have the correct value (for the synchronous services)

-	the data bits correctly include the start and stop bits (for the asynchronous services).

29.2.1	Verification of synchronisation

29.2.1.1	Definition and applicability

This test applies to MT2 and MT0 Mobile Stations supporting data services in transparent mode.

29.2.1.2	Conformance requirement

A Mobile Stations in MT2 configuration has to comply with all requirements whilst for MT0 configurations some of the requirements are not relevant. These restrictions are explicitely indicated in subclause 29.2.1.5.

29.2.1.3	Test Purpose

This test verifies the correct synchronization procedure of user data and status information which are mapped on modified CCITT V.110 frames (as per GSM 04.21).

As V-series interfaces are supported in full duplex mode, it will test the capability to synchronize these frames in either direction from the TAF to the IWF and vice versa.

29.2.1.4	Method of Test

The test shall be carried out under ideal radio conditions for all bearer services and user rates in transparent mode that are supported by the MS in case of mobile originated and terminated calls and in-call modification. The setting of Bearer Capability Information Elements in signalling messages sent to the MS by the SS must be supported by the MS for the bearer service(s) to be tested.

Note 1:	Since "steady state" is implementation dependent, there is no means to define a test "steady state detected". However, the whole testing procedure is limited to 1s. This includes an implizit upper time limit for the MS to detect a steady state. A MS failing this test is highly estimated to never detect a steady state under real radio conditions.

Note 2:	ti, as used in the description of the test procedures, are points of time not timers.

29.2.1.4.1	Procedure for Mobile Originated Calls

a)	The MS is connected to the System Simulator at the Um interface and to the MLTE using the appropriate R interface in case of MT2 only.

b)	The MLTE shall set the signalling lines of the R interface Ct 105, Ct 108.2 for V-series interface to ON in case of MT2 only.

c)	A mobile originated call shall be set up.

d)	At the reception of the SETUP message sent by the MS the SS shall send a CONNECT message and starts sending "1/OFF". t1 is at the completion of the CONNECT message.

e)	The reception of "1/OFF" at the SS side (see table 1) defines t2. t2 will be reset at the reception of again "1/OFF" after an interruption of continuous "1/OFF" pattern.

f)	The SS checks bits S1, S3, S6 and S8 of the modified CCITT V.110 frames (as described in TS GSM 04.21). Let t3 be the time when all four bits change from OFF to ON (i.e. if t<t3, (S1,S3,S6,S8) <> (0,0,0,0) and t>=t3,(S1,S3,S6,S8) = (0,0,0,0)).

29.2.1.4.2	Procedure for Mobile Terminated Calls

a)	The MS is connected to the System Simulator at the Um interface and to the MLTE using the appropriate R interface in case of MT2 only.

b)	The MLTE shall set the signalling lines of the R interface Ct. 105, Ct 108.2 for V-series interface to ON in case of MT2 only.

c)	A mobile terminated call shall be set up.

d)	At the reception of CONNECT the SS sends CONNACK. t1 is at the completion of the CONNACK message.

e)	The reception of "1/OFF" at the SS side (see table 1) defines t2. t2 will be reset at the reception of again "1/OFF" after an interruption of continuous "1/OFF" pattern.

f)	The SS checks bits S1, S3, S6 and S8 of the modified CCITT V.110 frames (as described in TS GSM 04.21). Let t3 be the time when all four bits change from OFF to ON (i.e. if t<t3, (S1,S3,S6,S8) <> (0,0,0,0) and t>=t3,(S1,S3,S6,S8) = (0,0,0,0)).

29.2.1.4.3	Procedure for In Call Modification

a)	The MS is connected to the System Simulator at the Um interface and to the MLTE using the appropriate R interface in case of MT2 only.

b)	The MLTE shall set the signalling lines of the R interface Ct. 105, Ct 108.2 for V-series interface to ON in case of MT2 only.

c)	a speech call shall be established with a SETUP message containing two bearer capabilities for speech and the bearer service to be tested.

d)	the MS shall start the ICM procedure with a bearer capability information element supporting the bearer service to be tested.

e)	at the reception of the MODIFY message sent by the MS the SS shall send a CHANNEL MODE MODIFY message.

f)	at the reception of the CHANNEL MODE MODIFY ACKNOWLEDGE message the SS shall send a MODIFY COMPLETE message. t1 is at the completion of the MODIFY COMPLETE message.

g)	the reception of "1/OFF" (see table 1) defines t2. t2 will be reset at the reception of again "1/OFF" after an interruption of continuous "1/OFF" pattern.

h)	the SS checks bits S1, S3, S6 and S8 of the modified CCITT V.110 frames (as described in TS GSM 04.21). Let t3 be the time when all four bits change from OFF to ON (i.e. if t<t3, (S1,S3,S6,S8) <> (0,0,0,0) and t>=t3,(S1,S3,S6,S8) = (0,0,0,0)).

29.2.1.5	Test Requirements

29.2.1.5.1	Test Reqirements for Mobile Originated Calls

1)	after step b) Cts 106, 107, 109 must be in the "OFF" condition, dataline 104 shall be set to "1".

2)	at t1 + 500 ms Ct 107 must still be in the "OFF" condition.

3)	between t1 + 500ms and t1 + 1000ms Ct 107 must switch to the "ON" condition. This indicates successful synchronization of TAF towards IFE.

4)	between t2 and t3 the SS must receive continuous "1/OFF" frames.

5)	the time between t2 and t3 must be more than 450ms.

6)	at t1 +1000ms the SS must check 04.21 frames sent by the MS with SA and SB bits (i.e. bits S1, S3, S4, S6, S8 and S9) set to "ON". This indicates successful synchronization of IFE towards TAF. At this point of time the whole synchronization procedure has been completed successfully.

Note:	Only requirements 4 to 6 apply to MT0.

29.2.1.5.2	Test Reqirements for Mobile Terminated Calls

1)	after step b) Cts 106, 107, 109 must be in the "OFF" condition, dataline 104 shall be set to "1".

2)	at t1 + 500 ms Ct 107 must still be in the "OFF" condition.

3)	between t1 + 500ms and t1 + 1000ms Ct 107 must switch to the "ON" condition. This indicates successful synchronization of TAF towards IFE.

4)	between t2 and t3 the SS must receive continuous "1/OFF" frames.

5)	the time between t2 and t3 must be more than 450ms.

6)	at t1 +1000ms the SS must check 04.21 frames sent by the MS with SA and SB bits (i.e. bits S1, S3, S4, S6, S8 and S9) set to "ON". This indicates successful synchronization of IFE towards TAF. At this point of time the whole synchronization procedure has been completed successfully.

Note:	Only requirements 4 to 6 apply to MT0.

29.2.1.5.3	Test Requirements for In Call Modification

1)	after step b) Cts 106, 107, 109 must be in the "OFF" condition, dataline 104 shall be set to "1".

2)	at t1 + 500 ms Ct 107 must still be in the "OFF" condition.

3)	between t1 + 500ms and t1 + 1000ms Ct 107 must switch to the "ON" condition. This indicates successful synchronization of TAF towards IFE.

4)	between t2 and t3 the SS must receive continuous "1/OFF" frames.

5)	the time between t2 and t3 must be more than 450ms.

6)	at t1 +1000ms the SS must check 04.21 frames sent by the MS with SA and SB bits (i.e. bits S1, S3, S4, S6, S8 and S9) set to "ON". This indicates successful synchronization of IFE towards TAF. At this point of time the whole synchronization procedure has been completed successfully.

Note:	Only requirements 4 to 6 apply to MT0.

Table 28-1: Definition of synchronization pattern "1/OFF"

Synch-Frame					Data-Frame

1	1	1	1	1	1	1		D1	D2	D3	D4	D5	D6	S1

1	1	1	1	1	1	1		D7	D8	D9	D10	D11	D12	X

1	1	1	1	1	1	1		D13	D14	D15	D16	D17	D18	S3

1	1	1	1	1	1	1		D19	D20	D21	D22	D23	D24	S4

1	1	1	1	1	1	1		E4	E5	E6	E7	D25	D26	S27

1	1	1	1	1	1	1		D28	D29	D30	S6	D31	D32	S33

1	1	1	1	1	1	1		D34	D35	D36	X	D37	D38	S39

1	1	1	1	1	1	1		D40	D41	D42	S8	D43	D44	S45

1	1	1	1					D46	D47	D48	S9

GSM 04.21 60 bits frame

Synch-Frame						Data-Frame

1	1	1	1	1	1	1	1		D1	D2	D3	S1	D4	D5	D6	X

1	1	1	1	1	1	1	1		D7	D8	D9	S3	D10	D11	D12	S4

1	1	1	1	1	1	1	1		E4	E5	E6	E7	D13	D14	D15	S6

1	1	1	1	1	1	1	1		D16	D17	D18	X	D19	D20	D21	S8

1	1	1	1	1					D22	D23	D24	S9

GSM 04.21 36 bits frame

29.2.2	Filtering of channel control information for Transparent BCs

29.2.2.1	Definition and Applicability

This test is only applicable to the MT2 configuration.

29.2.2.2	Conformance Requirement

An MS supporting data services shall decode and filter channel channel control information received over the Um-interface.

1	TS GSM 04.21, section 7

2	TS GSM 07.01, section 8.2.2

3	TS GSM 07.02, section 3.2.1 (for asynchronous bearer services only)

4	TS GSM 07.03, section 4.2.1 and 4.2.2 (for synchronous bearer services only)

29.2.2.3	Test Purpose

The purpose of this test is to verify the correct decoding and filtering of channel control information from the GSM 04.21 frames to the V.24/X.21 interface circuits. The tests apply after synchronisation has been completed.

29.2.2.4	Method of Test

The Test shall be carried out for all user data rates supported by the MS (see below) and the circuits CT106 (V.24) (interface circuit bit X) and CT109 (V.24) (interface circuit bit SB) and I (X.21) (S-bits). The test shall be carried out only for those frame formats and circuits which are supported by the MS. The test is to be repeated for all circuits.

Let T(ON-OFF) and T(OFF-ON) be the timers to integrate the ON-OFF and the OFF-ON transition respectedly for the circuit to be tested as stated in GSM 07.01, section 8.2.2.

Procedure:

a)	A data call shall be set up between the SS and the MS with a combination of BCIEs (see below) supported by the MS. The interface circuit bit(s) in the GSM 04.21 frame shall be set to OFF. The next step shall be entered 6 s after CT107 has been set to ON by the MS.

b)	The SS shall set the interface circuit bit(s) to continuously ON, wait half of T(OFF-ON) and then set the interface circuit bit(s) again to continuously OFF. The SS shall wait 6 s before entering the next step.

c)	The SS shall set the interface circuit bit(s) to continuously ON, wait twice T(OFF-ON) and then set the interface circuit bit(s) again to continuously OFF. The SS shall wait 6 s before entering the next step.

d)	The SS shall set interface circuit bit(s) to continuously ON and wait 6 s before entering the next step

e)	The SS shall set the interface circuit bit(s) to continuously OFF, wait half of T(ON-OFF) and then set the interface circuit bit(s) again to continuously ON. The SS shall wait 6 s before entering the next step.

f)	The SS shall set the interface circuit bit(s) to continuously OFF, wait twice T(ON-OFF) and then set the interface circuit bit(s) again to continuously ON. The SS shall wait 6 s before entering the next step.

29.2.2.5	Test Requirements

1)	after step a) the interface circuit at the R-interface shall be OFF.

2)	during step b) the interface circuit at the R-interface shall not change.

3)	during step c) the interface circuit at the R-interface shall change to ON and then again to OFF.

4)	after step d) the interface circuit at the R-interface shall be ON.

5)	during step e) the interface circuit at the R-interface shall not change.

6)	during step f) the interface circuit at the R-interface shall change to OFF and then again to ON.

29.2.2.6	BCIE

The following combinations shall be considered (ref. 07.01, Annex 2):

a)	User Rate = 9.6 kbit/s

b)	User Rate = 4.8 kbit/s

c)	User Rate = 2.4 kbit/s

d)	User Rate = 1.2 kbit/s

e)	User Rate = 1200/75 bit/s (only with asynchronous Bearer Services)

f)	User Rate = 300 bit/s (only with asynchronous Bearer Services)

The remaining parameters of the BCIE and the channel type (FR/HR) shall be set to a value supported by the MS.

29.2.3	Correct Terminal Compatibility Decision

[Editorial note: Signalling Group]

For further study.

29.2.4	Data Rate Adaptation for Synchronous Transparent Bearer Capabilities

29.2.4.1	Definition and Applicability

This test is applicable to the MT2 configuration and, in a restricted way, to MT0 configurations where the MS is able to send data over the Um-interface.

29.2.4.2	Conformance Requirement

An MS supporting synchronous transparent bearer capabilities shall perform data rate adaptation and support the frames at the Um-interface according to the following specifications:

1	TS GSM 04.21, section 5 and 7

29.2.4.3	Test Purpose

The purpose of these tests is to verify

-	that the format and the data bits of the GSM 04.21 frames sent by the MS are consistent with the data input and data rate at the R-interface and

-	that the data bits output by the MS at the R-interface are consistent with the received GSM 04.21 frames.

29.2.4.4	Method of Test

The Test shall be carried out for all possible user data rates which are supported by the MS (see below). The interface circuits CT105 and CT108 shall be set to the ON condition from the start.

Procedure:

a)	A data call shall be set up between the SS and the MS with a BCIE (see below) supported by the MS. The next step shall be entered immediately after TCH synchronisation has been completed at the SS side.

b)	The interface circuit bit(s) in the GSM 04.21 frame shall be set to ON and the SS shall start to transmit pseudo random data bits in the GSM 04.21 frames over the Um-interface to the MS.

c)	MT2 configuration: The SS shall input pseudo random data over the R-interface of the MS.

MT0 configuration: the transmission of data from the MS over the Um-interface shall be stimulated (if it does not start automatically).

d)	approximately 5 s after the data have been received by the SS over the Um-interface the test shall be stopped.

29.2.4.5	Test Requirements

1)	during the test no GSM 04.21 frame with incorrect format (i.e. format not compliant to GSM 04.21 (see conformance requirement) or not corresponding to the user data rate currently under test) shall have been received by the SS.

2)	only MT2 configuration: the user data stream input to the R-interface shall match bit-exactly the user data stream sent to the SS over the Um-interface and the user data stream output from the R-interface shall match bit-exactly the user data stream sent by the SS over the Um-interface.

29.2.4.6	BCIE

The following combinations shall be considered (ref. GSM 07.01, Annex 2):

a)	User Rate = 9.6 kbit/s

b)	User Rate = 4.8 kbit/s

c)	User Rate = 2.4 kbit/s

d)	User Rate = 1.2 kbit/s

The remaining parameters of the BCIE and the channel type (FR/HR) shall be set to a value supported by the MS.

29.2.5	Network Independent Clocking

For further study.

29.2.6	Asynchronous Transparent Bearer Capabilities

29.2.6.1	Data Rate Adaptation

29.2.6.1.1	Definition and Applicability

This test is applicable to the MT2 configuration and, in a restricted way, to the MT0 configuration where the MS is able to send data over the Um-interface and to use the Bearer Services for asynchronous data.

29.2.6.1.2	Conformance Requirement

An MS supporting asynchronous transparent bearer capabilities shall perform data rate adaptation and support the frames at the Um-interface according to the following specifications:

1	TS GSM 04.21, section 4.1, 4.4, 5 and 7

29.2.6.1.3	Test Purpose

The purpose of these tests is to verify the conversion between an asynchronous data stream at the R-interface and the GSM 04.21 frames at the Um-interface.

29.2.6.1.4	Method of Test

The Test shall be carried out for all possible user data rates which are supported by the MS in asynchronous mode (see below).

Procedure:

a)	A data call shall be set up between the SS and the MS with a BCIE (see below) supported by the MS. The next step shall be entered immediately after TCH synchronisation has been completed at the SS side.

b)	The interface circuit bit(s) in the GSM 04.21 frame shall be set to ON and the SS shall start to transmit pseudo random characters as described below to the MS.

c)	MT2 configuration: The SS shall input pseudo random characters as described below over the R-interface to the MS.

MT0 configuration: the transmission of data from the MS over the Um-interface shall be stimulated (if it does not start automatically).

d)	approximately 5 s after the data have been received by the SS over the Um-interface the test shall be stopped.

29.2.6.1.5	Test Requirements

1)	during the test no GSM 04.21 frame with incorrect format (i.e. format not compliant to GSM 04.21 (see conformance requirement) or not corresponding to the user data rate currently under test) shall have been received by the SS.

2)	MT2 configuration only: the user data stream input to the R-interface shall match character by character the user data stream sent to the SS over the Um-interface and the user data streams output from the R-interface shall match character by character the user data stream sent by the SS over the Um-interface.

29.2.6.1.6	Generation of the asynchronous pseudo random characters

Downlink direction:

The GSM 04.21 frames shall contain a bit stream which consists of repeating

-	a character which is generated pseudo randomly every time

-	n stop bits, where n is drawn pseudo randomly from the interval 1..15 every time.

Uplink direction:

The data stream at the R-interface consists of repeating

-	a character which is generated pseudo randomly every time

-	1 stop bit

-	1.13 +/- 1% bit frames (i.e. 1/nominal data rate) of stop polarity.

29.2.6.1.7	BCIE

Same as 29.2.3.

The Number of Data Bits per character (excl. parity) shall be 8. No parity bit shall be used. The Number of Stop Bits shall be 1. If the MS does not support these values different ones shall be chosen.

The remaining parameters of the BCIE shall and the channel type (FR/HR) be set to a value supported by the MS.

29.2.6.2	Passage of the Break Signal

29.2.6.2.1	Definition and Applicability

This test is applicable to the MT2 configuration only.

29.2.6.2.2	Conformance Requirement

An MS supporting asynchronous transparent bearer capabilities shall perform passage of the break signal in uplink and downlink direction according to:

1	TS GSM 04.21, section 4.1, 4.2, 4.4, 5 and 7

29.2.6.2.3	Test Purpose

The purpose of these tests is to verify the ability of the MS to transfer a Break Signal to the R-interface and vice versa.

29.2.6.2.4	Method of Test

The Test shall be carried out for all possible user data rates which are supported by the MS in asynchronous mode (see below).

Procedure:

a)	A data call shall be set up between the SS and the MS with a BCIE (see below) supported by the MS.The next step shall be entered immediately after TCH synchronisation has been completed at the SS side.

b)	The interface circuit bit(s) in the GSM 04.21 frame shall be set to ON.

c)	The SS shall send pseudo random characters with start and stop bit(s) (as selected by the BCIE) in the GSM 04.21 frames to the MS for approximately 5 s. Then it shall send the following bit sequence in the GSM 04.21 frames:

-	2M+3 bits of start polarity

-	2M bits of stop polarity

d)	The SS shall send pseudo random characters with start and stop bit(s) (as selected by the BCIE) in the GSM 04.21 frames to the MS for approximately 2 s. Then it shall send the following bit sequence in the GSM 04.21 frames:

-	for 1 s bits of start polarity

-	2M bits of stop polarity,

where M is as defined in GSM 04.21, section 4.2, and then again pseudo random characters as above.

e)	The SS shall input pseudo random characters with start and stop bit(s) (as selected by the BCIE) over the R-interface to the MS for approximately 2 s. Then it shall input the following bit sequence to the R-interface:

-	M bits of start polarity

-	2M bits of stop polarity

f)	The SS shall input pseudo random characters with start and stop bit(s) (as selected by the BCIE) over the R-interface to the MS for approximately 2 s. Then it shall input the following bit sequence to the R-interface:

-	for 1 s bits of start polarity

-	2M bits of stop polarity,

where M is as defined in GSM 04.21, section 4.2, and then again pseudo random characters as above.

g)	the test shall be stopped 2 s later.

29.2.6.2.5	Test Requirements

1)	during the test no GSM 04.21 frame with incorrect format (i.e. format not compliant to GSM 04.21 (see conformance requirement) or not corresponding to the user data rate currently under test) shall have been received by the SS.

2)	the user data stream sent over the Um-interface by the SS shall match character by character the user data stream output at the R-interface.

3)	the two Break Signals shall be detectable at the R-interface between the same characters as having been sent.

4)	the user data stream received over the Um-interface by the SS shall match character by character the user data stream input at the R-interface.

5)	the two Break Signals shall be detectable at the Um-interface between the same characters as having been input.

29.2.6.2.6	BCIE

Same as 29.2.2.6.

29.2.6.3	Overspeed/Underspeed Handling (Local Terminal)

29.2.6.3.1	Definition and Applicability

This test is applicable to the MT2 configuration only.

29.2.6.3.2	Conformance Requirement

An MS supporting asynchronous transparent bearer capabilities shall handle overspeed and underspeed of the local terminal according to:

1	TS GSM 04.21, section 4.1, 4.3, 4.4, 5 and 7

29.2.6.3.3	Test Purpose

The purpose of these tests is to verify the ability of the MS to deal with plesiochronous bit clocks in the MS and the TE in case of asynchronous Bearer Capabilities.

29.2.6.3.4	Method of Test

The Test shall be carried out for all possible user data rates supported by the MS in asynchronous mode (see below).

Procedure:

a)	A data call shall be set up between the SS and the MS with a BCIE (see below) supported by the MS. The next step shall be entered immediately after CT107 has been set to on by the MS.

b)	The interface circuit bit(s) in the GSM 04.21 frame shall be set to ON.

c)	The SS shall input continuously pseudo random characters with a bit clock of

-	nominal user data rate + 2.5% bit/s in case of less than 600 bit/s user data rate

-	nominal user data rate + 1 % in the other cases

(ref. GSM 04.21, section 4.3) to the R-interface of the MS for approximately 5 s.

d)	The SS shall input continuously pseudo random characters with a bit clock of nominal user rate - 2.5 % (ref. CCITT V.14, section 3) to the R-interface of the MS for approximately 5 s.

e)	the test shall be stopped.

29.2.6.3.5	Test Requirements

1)	during the test no GSM 04.21 frame with incorrect format (i.e. format not compliant to GSM 04.21 (see conformance requirement) or not corresponding to the user data rate currently under test) shall have been received by the SS.

2)	the user data stream input to the R-interface shall match character by character the user data stream sent to the SS over the Um-interface.

29.2.6.3.6	BCIE

Same as 29.2.2.6.

The Number of Data Bits per character (excl. parity) shall be 8. No parity bit shall be used. The Number of Stop Bits shall be 1. If the MS does not support these values different ones shall be chosen.

The remaining parameters of the BCIE and the channel type (FR/HR) shall be set to a value supported by the MS.

29.2.6.4	Overspeed/Underspeed Handling (Remote Terminal)

29.2.6.4.1	Definition and Applicability

This test is applicable to the MT2 configuration only.

29.2.6.4.2	Conformance Requirement

An MS supporting asynchronous transparent bearer capabilities shall handle overspeed and underspeed of the remote terminal (which shows in the structure of the GSM 04.21 frames received over the Um-interface) according to:

1	TS GSM 04.21, section 4.1, 4.3, 4.4, 5 and 7

29.2.6.4.3	Test Purpose

The purpose of these tests is to verify the ability of the MS to deal with plesiochronous bit clocks in the MS and the remote Terminal in case of asynchronous Bearer Capabilities.

The case of underspeed is covered by 29.6.1. The case of overspeed shall be tested as follows.

29.2.6.4.4	Method of Test

The Test shall be carried out for all possible user data rates supported by the MS in asynchronous mode (see below).

Procedure:

a)	A data call shall be set up between the SS and the MS with a BCIE (see below) supported by the MS. The next step shall be entered immediately after CT107 has been set to on by the MS.

b)	The interface circuit bit(s) in the GSM 04.21 frame shall be set to ON.

c)	The SS shall start sending pseudo random characters in the GSM 04.21 frames over the Um-interface with minimal number of stop bits between the characters and where one stop bit is ommitted every 8th character.

e)	the test shall be stopped 5 s later.

29.2.6.4.5	Test Requirements

1)	during the test no GSM 04.21 frame with incorrect format (i.e. format not compliant to GSM 04.21 (see conformance requirement) or not corresponding to the user data rate currently under test) shall have been received by the SS.

2)	the user data stream sent by the SS over the Um-interface shall match character by character the user data stream sent by the MS over the R-interface.

29.2.6.4.6	BCIE

Same as 29.2.6.3.6.

29.2.7	Interchange Circuit Mapping for Transparent Bearer Capabilities

29.2.7.1	Definition and Applicability

This test is applicable to the MT2 configuration only. However, it cannot be applied fully to MSs which support e.g.:

-	CT108/2 for releasing the call (e.g. support of CCITT V.25bis) and/or

-	similar use of the circuit C for CCITT X.21.

Therefore the test shall be applied only for those interchange circuits which do not influence Layer 3 signalling.

In case of circuit C the X.21-byte timing circuit B shall not be used.

29.2.7.2	Conformance Requirement

1)	An MS supporting transparent bearer capabilities with a V-series R-interface shall map the interchange circuits CT105 and CT108/2 to the GSM 04.21 frames sent over the Um-interface according to:

1.1	TS GSM 04.21, section 4.1, 4.4, 5 and 7

1.2	TS GSM 07.02, section 3.2.1

2)	An MS supporting transparent bearer capabilities with an X-series R-interface shall map the interchange circuit C to the GSM 04.21 frames sent over the Um-interface according to:

2.1	TS GSM 04.21, section 4.1, 4.4, 5 and 7

2.2	TS GSM 07.03, section 4.2.1 and 4.2.2

29.2.7.3	Test Purpose

The purpose of these tests is to verify the ability of the MS to correctly convey changes of the interface circuits at the R-interface to the GSM 04.21 frame sent over the Um-interface in case of Transparent Bearer Capabilities.

29.2.7.4	Method of Test

The Test shall be carried out for all user data rates supported by the MS (see below) and the circuits CT105 and CT108/2 (CCITT V.24) and C (CCITT X.21). The test shall be carried out only for those frame formats and circuits which are supported by the MS (exceptions see above).

Procedure:

a)	A data call shall be set up between the SS and the MS with a BCIE (see below) supported by the MS. The next step shall be entered immediately after CT107 has been set to ON by the MS.

b)	The interface circuit bit(s) in the GSM 04.21 frame shall be set to ON and the SS shall input continuously pseudo random data to the R-interface during the following steps. The SS shall wait for approximately 1 s before entering the next step.

c)	The SS shall set the interchange circuit at the R-interface to OFF and wait for 2 s.

d)	The SS shall again set the interchange circuit at the R-interface to ON.

e)	After further 2 s the test shall be stopped.

29.2.7.5	Test Requirements

1)	during the test no GSM 04.21 frame with incorrect format (i.e. format not compliant to GSM 04.21 (see conformance requirement) or not corresponding to the user data rate currently under test) shall have been received by the SS.

2)	the change of the interchange circuit signal level shall be indicated in the GSM 04.21 frames as required by GSM 04.21 and CCITT V.110 (i.e. OFF state shall start and end in the correct GSM 04.21 frame).

29.2.7.6	BCIE

Same as 29.2.2.5.

29.3	Testing of non transparent data services

For further study.