30.	Speech teleservices

Note 1	An MS may be either a handset MS, a handsfree MS or a combined handset and handsfree MS. The test description for handsfree operation, however, only covers the stability margin as no test method could be defined for the other parameter.

Note 2	Frequency settings in the following tests are taken from ISO 3, R10 series or R40 series or from table 2 of CCITT P.79. A departure from the nominal frequencies of + 5% below 240 Hz and + 2% at 240 Hz and above is accepted. Any sub-multiple of the sampling frequency of 8 kHz shall be avoided. In the case of 4 kHz the departure is restricted to -2%.

Note 3	The measurement accuracy for signal level is +/- 0.2 dB and for sound pressure +/- 0.6 dB.

30.1	Sending sensitivity/frequency response

30.1.1	Definition and applicability.

The sending sensitivity frequency response is, as a function of the input test tone frequency, the ratio expressed in dB between the output level, represented by the PCM bit stream at the Digital Audio Interface (DAI) and the input sound pressure in the artificial mouth required to obtain this.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.1.2	Conformance requirement

The sending sensitivity frequency response shall be within the mask given in ETS 300 085, subclause 6.2.1.1.

GSM 03.50; 3.8.1.1.

30.1.3	Test purpose

To verify that the sending sensitivity frequency response from the MRP to the DAI is within the mask given in ETS 300 085, subclause 6.2.1.1.

30.1.4	Method of test

30.1.4.1	Initial conditions

The SS sets DTX to "off".

The handset is mounted in the LRGP (see annex 1 of CCITT P.76) and the earpiece is sealed to the knife-edge of the artificial ear conforming to CCITT P.51.

A pure tone with a sound pressure of -4.7 dBPa is applied at the mouth reference point (MRP) as described in CCITT P.64 using an artificial mouth conforming to CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

30.1.4.2	Procedure

a)	The SS measures the output level represented by the PCM bit stream at the DAI (pin 23) at one-twelfth-octave intervals as given by the R40 series of preferred numbers in ISO 3 for frequencies from 100 Hz to 4000 Hz inclusive.

30.1.5	Test requirement

The sending sensitivity/frequency response (from MRP to the DAI) shall be within a mask given in Table 30.1. The mask can be drawn with straight lines between the breaking points in the table on a logarithmic (frequency) vs linear (dB sensitivity) scale.

All sensitivity levels are dB on an arbitrary scale.

Table 30.1

Frequency [Hz]	Upper Limit [dB]	Lower Limit [dB]

a =	100	-12

b =	200	0

c =	300	0	-12

d =	1000	0	-6

e =	2000	4	-6

f =	3000	4	-6

g =	3400	4	-9

h =	4000	0

30.2	Sending loudness rating

30.2.1	Definition and applicability

The Sending Loudness Rating (SLR) is a means of expressing the sending frequency response based on objective single tone measurements in a way which relates to how a speech signal would be perceived by a listener.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.2.2	Conformance requirement

The Sending Loudness Rating (SLR) shall be 8 +/- 3 dB.

GSM 03.50; 3.1.1

30.2.3	Test Purpose

To verify that the Sending Loudness Rating (SLR) is 8 +/- 3 dB.

30.2.4	Method of test

30.2.4.1	Initial conditions

The SS sets DTX to "off".

30.2.4.2	Procedure

a)	The SS measures the sending sensitivity  at each of the 14 frequencies listed in table 2 of CCITT P.79, bands 4 to 17.

b)	The sensitivity is expressed in terms of dBV/Pa and the SLR is calculated according to formula 4.19 b of CCITT P.79, over bands 4 to 17, using the sending weighting factors from table 2/CCITT P.79, adjusted according to table 3/CCITT P.79.

30.2.5	Test requirement

The SLR shall be 8 +/- 3 dB.

30.3	Receiving sensitivity/frequency response

30.3.1	Definition and applicability

The receiving sensitivity frequency response is, as a function of the input test tone frequency, the ratio expressed in dB between the output sound pressure in the artificial ear and the input level, represented by the PCM bit stream at the Digital Audio Interface (DAI), required to obtain this.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.3.2	Conformance requirement

The receiving sensitivity frequency response shall be within the mask given in ETS 300 085, subclause 6.2.1.2.

GSM 03.50; 3.8.1.2.

30.3.3	Test purpose

To verify that the receiving sensitivity frequency response from the DAI to the ERP is within the mask given in ETS 300 085, subclause 6.2.1.2.

30.3.4	Method of test

30.3.4.1	Initial conditions

The SS sets DTX to "off".

The handset is mounted in the LRGP (see annex 1 of CCITT P.76) and the earpiece is sealed to the knife-edge of the artificial ear conforming to CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

30.3.4.2	Procedure

a)	The SS sends a PCM bit stream equivalent to a pure tone level of -16 dBmO (see NET 33) over the DAI (pin 25) to the MS.

b)	The SS measures the sound pressure in the artificial ear (the ear reference point - ERP) at one-twelfth-octave intervals as given by the R40 series of preferred numbers in ISO 3 for frequencies from 100 Hz to 4000 Hz inclusive.

30.3.5	Test requirement

The receiving sensitivity/frequency response (from the DAI to the ERP) shall be within the mask given by Table 30.2. The mask can be drawn with straight lines between the breaking points in the following table on a logarithmic (frequency) vs linear (dB sensitivity) scale.

All sensitivity levels are dB on an arbitrary scale.

Table 30.2

Frequency [Hz]	Upper Limit [dB]	Lower Limit [dB]

a =	100	-12

b =	200	0

c =	300	2	-7

d =	1000	*)	-5

e =	2000	0	-5

f =	3000	2	-5

g =	3400	2	-10

h =	4000	2

*)	The limit at intermediate frequencies lies on a straight line drawn between the given values on a log (frequency) vs linear (dB) scale.

30.4	Receiving loudness rating

30.4.1	Definition and applicability

The Receiving Loudness Rating (RLR) is a means of expressing the receiving frequency response based on objective single tone measurements in a way which relates to how a speech signal would be perceived by a listener.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.4.2	Conformance requirement

1)	The Receiving Loudness Rating (RLR) shall be 2 +/- 3 dB.

If a user controlled receive volume control is provided the equipment shall meet this value for at least one setting of the control.

GSM 03.50; 3.1.1.

2)	If a user controlled receive volume control is provided the Receive Loudness Rating (RLR) shall be not less than -8 dB when the control is set to maximum.

GSM 03.50; 3.1.1.

30.4.3	Test purpose

1)	To verify that the Receiving Loudness Rating (RLR) is 2 +/- 3 dB.

2)	To verify that if a user controlled receive volume control is provided the Receive Loudness Rating (RLR) is not less than -8 dB when the control is set to maximum.

30.4.4	Method of test

30.4.4.1	Initial conditions

The SS sets DTX to "off".

30.4.4.2	Procedure

a)	The SS measures the receiving sensitivity at each of the 14 frequencies listed in table 2 of CCITT P.79, bands 4 to 17.

b)	The sensitivity is expressed in terms of dBPa/V and the RLR is calculated according to formula 4.19 c of CCITT P.79, over bands 4 to 17, using the receiving weighting factors from table 2/CCITT P.79, adjusted according to table 3/CCITT P.79. The artificial ear sensitivity must be corrected according to the real ear correction table 4 of CCITT P.79.

30.4.5	Test requirement

If no user controlled receive volume control is provided, the RLR shall be 2 +/- 3 dB, (later referred as the nominal value).

If a user controlled receive volume control is provided, the RLR shall meet this value for at least one setting of the control.

When the control is set to maximum the RLR shall not be less than (ie louder than) -8 dB.

30.5	Talker sidetone

30.5.1	Definition and applicability

The sidetone loudness ratings are a means of expressing the path loss from the artificial mouth to the artificial ear based on objective single tone measurements in a way that relates to how a speaker will perceive his own voice when speaking (talker sidetone, expressed by the sidetone masking rating - STMR), or how a listener will perceive the background noise picked up by the microphone (listener sidetone rating - LSTR).

Note 4	"Listener sidetone" testing is not included in this test specification.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.5.2	Conformance requirement

The Side Tone Masking Rating (STMR) shall be 13 +/- 5 dB.

GSM 03.50; 3.10.1.

30.5.3	Test purpose

To verify that the Side Tone Masking Rating (STMR) is 13 +/- 5 dB.

30.5.4	Method of test

30.5.4.1	Initial conditions

The SS sets DTX to "off".

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A".

The handset is mounted in the LRGP (see annex 1 of CCITT P.76) and the earpiece is sealed to the knife-edge of the artificial ear conforming to CCITT P.51.

30.5.4.2	Procedure

a)	The SS sends a PCM bit stream coded with the value No 1 over the DAI (pin 25).

b)	The SS applies a pure tone with a sound pressure of -4.7 dBPa at the mouth reference point as described in CCITT P.64 using an artificial mouth conforming to CCITT P 51.

c)	For each frequency given in table 2 of CCITT P.79, bands 4 to 17, the sound pressure in the artificial ear is measured.

d)	The sidetone path loss is expressed in dB and the STMR (in dB) is calculated from the formula 8.4 of CCITT P.79, using the weighting factors of column (3) in table 6/CCITT P.79 (unsealed), and values of LE in accordance with table 4/CCITT P.79.

30.5.5	Test requirement

The STMR shall be 13 +/- 5 dB.

Where a user controlled receive volume control is provided, the STMR shall meet the requirement given above at the setting where the RLR is equal to the nominal value.

30.6	Telephone acoustic coupling loss (TAL)

30.6.1	Echo return loss (ERL)

30.6.1.1	Definition and applicability

The echo return loss is the path loss from the input of the reference speech encoder of the SS to the output of the reference speech decoder of the SS.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.6.1.2	Conformance requirement

The Echo Return Loss from the input to the output of the reference speech codec in the SS shall be at least 46 dB.

GSM 03.50; 3.4.1.

30.6.1.3	Test purpose

To verify that the Echo Return Loss from the input to the output of the reference speech codec in the SS is at least 46 dB.

30.6.1.4	Method of test

30.6.1.4.1	Initial conditions

The SS sets DTX to "off".

The DAI of the MS is connected to the SS and is set to the operating mode "Normal operation".

The SS sets up a speech call according to the generic call set up..

The handset is mounted in the LRGP (see annex 1 of CCITT P.76) and the earpiece is sealed to the knife-edge of the artificial ear conforming to CCITT P.51.

Where a user controlled volume control is provided it is set to maximum.

30.6.1.4.2	Procedure

An analogue or digital signal generator delivering a signal equivalent to a pure tone level of 0 dBmO (see CCITT P.64 or 66 respectively) is connected to the corresponding analogue or digital input of the reference speech encoder of the SS.

Measurements are made at one-twelfth-octave intervals as given by the R40 series of preferred numbers in ISO 3 for frequencies from 300 Hz to 3400 Hz.

At each frequency the level or, for a digital signal, equivalent level at the reference speech decoder output of the SS is measured in the band 300 to 3400 Hz.

The echo return loss is calculated according to CCITT G.122.

Note 5	The normal operation of the speech codec results in modulation of the amplitude of pure tone signals at a frequency that depends upon the frequency of the tone. This makes the measurement of TAL with pure tones uncertain. Choice of test signal is therefore important but, whilst a speech-like signal may be more suitable, insufficient information is available for a definitive choice.

30.6.1.5	Test requirement

The echo return loss from the input to the output of the reference speech codec in the SS shall be at least 46 dB.

30.6.2	Stability margin

30.6.2.1	Definition and applicability

The receive-transmit stability margin is a measure of the gain that would have to be inserted between the go and return paths of the reference speech coder in the SS for oscillation to occur.

The requirements and this test apply to all types of GSM900 and DCS 1800 MS supporting speech.

30.6.2.2	Conformance requirement

The stability margin shall be at least 6 dB.

GSM 03.50; 3.2.

30.6.2.3	Test purpose

To verify that the stability margin is at least 6 dB.

30.6.2.4	Method of test

30.6.2.4.1	Initial conditions

The SS sets DTX to "off".

For handset operation the handset is placed on a hard plane surface with the transducers facing the surface.

For handsfree operation the test setup is shown in CCITT P.34 (Fig 3/CCITT P.34), but omitting the test table.

Where a user controlled volume control is provided it is set to maximum.

30.6.2.4.2	Procedure

a)	A gain equivalent to the minimum stability margin is inserted in the loop between the go and return paths of the reference speech coder in the SS and any acoustic echo control is enabled.

b)	A test signal according to CCITT O.131 is injected into the loop at the analogue or digital input of the reference speech codec of the SS and the stability is measured. The test signal has a level of -10 dBmO and a duration of 1 s.

30.6.2.5	Test requirement

The minimum stability margin shall be 6 dB and no audible oscillation shall be detected.

30.7	Distortion

30.7.1	Sending

30.7.1.1	Definition and applicability

The transmit signal to total distortion ratio is a measure of the linearity of the transmitter equipment (excluding the speech codec).

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.7.1.2	Conformance requirement

The ratio of signal to total distortion power in the sending direction measured at the DAI with psophometric filter shall be above the limits given in ETS 300 085, subclause 6.2.5.1.2.

GSM 03.50; 3.9.1.

30.7.1.3	Test purpose

To verify that the ratio of signal to total distortion power in the sending direction measured at the DAI with psophometric filter is above the limits given in ETS 300 085, subclause 6.2.5.1.2.

30.7.1.4	Method of test

30.7.1.4.1	Initial conditions

The SS sets DTX to "off".

The handset is mounted in the LRGP (see annex 1 of CCITT P.76) and the earpiece is sealed to the knife-edge of the artificial ear conforming to CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

30.7.1.4.2	Procedure

a)	A sine-wave signal with a frequency in the range 1004 Hz to 1025 Hz is applied to the MRP. The level of this signal is adjusted until the PCM bitstream at the DAI output (pin 23) corresponds to -10 dBmO. The level of the signal at the MRP is then the acoustic reference level (ARL).

b)	The test signal is applied at the following levels: -35, -30, -25, -20, -15, -10, -5, 0, 5, 10 dB relative to the ARL.

c)	The ratio of signal to total distortion power is measured at the DAI with the psophometric noise weighting (see CCITT G.714 and O.132) at each signal level.

Note 6	The measurement is carried out at sound pressures not exceeding +10 dBPa.

30.7.1.5	Test requirement

The ratio of signal to total distortion power measured at the DAI with the psophometric noise weighting (see table 4/CCITT G.223) shall be above the limits given in Table 30.3.

Table 30.3

dB relative to ARL	Level ratio

-35 dB	17.5 dB

-30 dB	22.5 dB

-20 dB	30.7 dB

-10 dB	33.3 dB

0 dB	33.7 dB

7 dB	31.7 dB

10 dB	25.5 dB

Limits for the signal to total distortion ratio (sending) when using the sine wave method.

Limits for intermediate levels are found by drawing a straight line between breaking points in a linear (dB signal level) vs linear (dB ratio) scale.

30.7.2	Receiving

30.7.2.1	Definition and applicability

The receive signal to total distortion ratio is a measure of the linearity in the receive equipment.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.7.2.2	Conformance requirement

The ratio of signal to total distortion power in the receiving direction measured at the ERP with psophometric filter shall be above the limits given in ETS 300 085 para. 6.2.5.2.2

GSM 03.50 para. 3.9.2

30.7.2.3	Test purpose

To verify that the ratio of signal to total distortion power in the receiving direction measured at the ERP with psophometric filter is above the limits given in ETS 300 085 para. 6.2.5.2.2

30.7.2.4	Method of test

30.7.2.4.1	Initial conditions

The SS sets DTX to "off".

The handset is mounted in the LRGP (see annex 1 of CCITT P.76) and the earpiece is sealed to the knife-edge of the artificial ear conforming to CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

30.7.2.4.2	Procedure

a)	The SS sends, via the DAI (Pin 25), a PCM bit stream simulating a sine-wave signal corresponding to CCITT O.132 at the following levels: -45, -40, -35, -30, -25, -20, -15, -10, -5, 0 dBmO.

b)	The ratio of signal to total distortion power is measured with the psophometric noise weighting in the artificial ear (see CCITT G.714) at each signal level.

c)	The measurement is carried out at sound pressures between -50 dBPa and +10 dBPa.

30.7.2.5	Test requirement

The ratio of signal to total distortion power measured at the artificial ear with the psophometric noise weighting (see table 4/CCITT G.223) shall be above the limits given in Table 30.4.

Table 30.4

Level at the digital audio interface	Level ratio

-45 dBmO	17.5 dB

-40 dBmO	22.5 dB

-30 dBmO	30.5 dB

-20 dBmO	33.0 dB

-10 dBmO	33.5 dB

-3 dBmO	31.2 dB

0 dBmO	25.5 dB

Limits for the signal to total distortion ratio (receiving) when using the sine wave method.

Limits for intermediate levels are found by drawing a straight line between breaking points in a linear (dB signal level) vs linear (dB ratio) scale.

30.8	Sidetone

30.8.1	Definition and applicability

The sidetone distortion expresses the linearity of the sidetone path in the handset.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.8.2	Conformance requirement

The third harmonic distortion of the sidetone shall not be greater than 10 %.

GSM 03.50 para. 3.10.2

30.8.3	Test purpose

To verify that the third harmonic distortion of the sidetone is not greater than 10 %.

30.8.4	Method of test

30.8.4.1	Initial conditions

The SS sets DTX to "off".

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A".

The handset shall is in the LRGP (see annex 1 of CCITT P.76) and the earpiece is sealed to the knife-edge of the artificial ear conforming to CCITT P.51.

30.8.4.2	Procedure

a)	The SS sends the PCM bit stream coded with the value No 1 over the DAI (pin 25) to the MS.

b)	An instrument capable of measuring the third harmonic distortion of signals with fundamental frequencies in the range 315 Hz to 1000 Hz is connected to the artificial ear.

c)	A pure-tone signal of -4.7 dBPa is applied at the mouth reference point at frequencies of 315 Hz, 500 Hz, and 1000 Hz. For each frequency the third harmonic distortion is measured in the artificial ear.

30.8.5	Test requirement

The third harmonic distortion generated shall not be greater than 10%.

30.9	Out-of-band signals

30.9.1	Sending

30.9.1.1	Definition and applicability

The discrimination against out-of-band input signals in the sending direction is a requirement on the in-band image frequencies created by any out-of-band input signals.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.9.1.2	Conformance requirement

The discrimination against out-of-band input signals in the sending direction shall be greater than the limit in ETS 300 085 para. 6.2.7.1

GSM 03.50 para. 3.11.1

30.9.1.3	Test purpose

To verify that the discrimination against out-of-band input signals in the sending direction is greater than the limit in ETS 300 085 para. 6.2.7.1

30.9.1.4	Method of test

30.9.1.4.1	Initial conditions

The SS sets DTX to "off".

The handset is mounted in the LRGP (see annex 1 of CCITT P.76) and the earpiece is sealed to the knife-edge of the artificial ear conforming to CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

30.9.1.4.2	Procedure

a)	A pure tone with a sound pressure of -4.7 dBPa is applied at the mouth reference point as described in CCITT P.64 using an artificial mouth conforming to CCITT P 51.

b)	For input signals at frequencies of 4.65, 5, 6, 6.5, 7, and 7.5 kHz, the level represented by the PCM bit stream at the DAI (Pin 23) of any image frequency is measured.

30.9.1.5	Test requirement

The level of any image frequency shall be below a reference obtained at 1 kHz by at least the amount as specified in Table 30.5.

Table 30.5

Applied sine-wave frequency	Limit (minimum)

4.6 kHz	30 dB

8 kHz	40 dB

Limits for the image frequency discrimination

The limit at intermediate frequencies lies on a straight line drawn between the given values on a log(frequency) vs linear(dB) scale.

30.9.2	Receiving

30.9.2.1	Definition and applicability

The discrimination against out-of-band signals in the receiving direction is a requirement on the out-of-band signals generated in the artificial ear from in-band input signals.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.9.2.2	Conformance requirement

The discrimination against out-of-band input signals in the receiving direction shall be greater than the limit in ETS 300 085  6.2.7.2

GSM 03.50 para. 3.11.2

30.9.2.3	Test purpose

To verify that the discrimination against out-of-band input signals in the receiving direction is greater than the limit in ETS 300 085 para. 6.2.7.2

30.9.2.4	Method of test

30.9.2.4.1	Initial conditions

The SS sets DTX to "off".

The handset is mounted in the LRGP (see annex 1 of CCITT P.76) and the earpiece is sealed to the knife-edge of the artificial ear conforming to CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

30.9.2.4.2	Procedure

a)	The SS sends over the DAI (pin 25) a PCM bit stream simulating a sine-wave signal with a level of 0 dBmO .

b)	For input signals at the nominal frequencies 500, 1000, 2000, and 3350 Hz (bearing in mind the restriction on sub-multiples of the sampling frequency) the level of any out-of-band signals at frequencies up to 8 kHz is measured in the artificial ear.

30.9.2.5	Test requirement

The level of out-of-band signals shall be lower than the in-band acoustic level obtained by a digital signal at 1 kHz set at the level specified in Table 30.6.

Table 30.6

Image signal frequency	Equivalent input signal level

4.6 kHz	-35 dBmO

8 kHz	-45 dBmO

Limits for the image frequency discrimination

The limit at intermediate frequencies lies on a straight line drawn between the given values on a log(frequency) vs linear(dB) scale.

30.10	Idle channel noise

30.10.1	Sending

30.10.1.1	Definition and applicability

The idle channel noise in the sending direction is the equivalent noise level produced at the DAI, when the mouth reference point is in a quiet environment.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.10.1.2	Conformance requirement

The idle noise in the sending direction shall not exceed - 64 dBm0p.

GSM 03.50 para. 3.6.1

30.10.1.3	Test purpose

To verify that the idle noise in the sending direction does not exceed - 64 dBm0p.

30.10.1.4	Method of test

30.10.1.4.1	Initial conditions

The SS sets DTX to "off".

The handset is mounted in the LRGP (see annex 1 of CCITT P.76) and the earpiece is sealed to the knife-edge of the artificial ear conforming to CCITT P.51 in a quiet environment (ambient noise less than 30 dBA).

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

30.10.1.4.2	Procedure

a)	The noise level represented by the PCM bit stream output at the DAI (pin 23) is measured with psophometric weighting according to CCITT G.223, table 4.

30.10.1.5	Test requirement

The noise produced by the MS in the sending direction shall not exceed -64 dBmOp.

30.10.2	Receiving

30.10.2.1	Definition and applicability

The idle channel noise in the receiving direction is the acoustic sound pressure in the artificial ear when the digital input signal at the DAI, is the PCM coded value No 1.

The requirements and this test apply to all types of GSM900 and DCS 1800 handset MS supporting speech.

30.10.2.2	Conformance requirement

1.	The idle noise in the receiving direction shall not exceed - 57 dBPa (A).

GSM 03.50 para. 3.6.2

2.	The idle noise in the receiving direction shall not exeed -54 dBPa(A).

GSM 03.50 para. 3.6.2

30.10.2.3	Test purpose

1.	To verify that the idle noise in the receiving direction does not exceed - 57 dBPa (A). (If a user controlled receive volume control is provided it shall be set to the position where RLR is equal to the nominal value).

2.	To verify that if a user controlled receive volume control is provided, the idle noise in the receiving direction does not exeed -54 dBPa(A) when the control is set to maximum.

30.10.2.4	Method of test

30.10.2.4.1	Initial conditions

The SS sets DTX to "off".

The handset is mounted in the LRGP (see annex 1 of  CCITT P.76) and the earpiece is sealed to the knife-edge of the artificial ear conforming to CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

30.10.2.4.2	Procedure

a)	The SS sends a PCM bit stream coded with the value No 1 over the DAI (Pin 25) to the MS.

b)	The level of the noise is measured in the artificial ear with any volume control set at the position at which the RLR is equal to the nominal value.

30.10.2.5	Test requirement

The measured noise generated by the MS shall not exceed -57 dBPa (A).

Where a volume control is provided, the measured noise shall also not exceed -54 dBPa (A) at the maximum setting of the volume control.