3.	Definitions, symbols and abbreviations

3.1	Mobile station configurations

3.1.1	Definition

A Mobile Station is the complete equipment configuration which may take part in a communication. However, this may not be the mobile station as it is offered to a test house for conformance testing.

In general, the Mobile Station, as it will be presented to a test house for conformance testing, is the station without all the additional terminal equipment. Such a piece of hardware is also called a Mobile Termination (MT), but in this ETS, the expression Mobile Station (MS) is used for any form of MS hardware as it is offered to the test house.

Mobile Termination

Interface: S, R, etc..

Terminal Equipment

During the tests, the interfaces of the "MT" shall be connected to a "System Simulator" (SS), which will also emulate the TE. For some tests, it may be necessary to establish a pre-configured setup of the MS. As an example: for reception of automatic fax group 3 to a fax machine on the R-interface, the MS needs configuration information about the presence of such a machine on that interface.

As an alternative, the terminal equipment may be physically integrated.

For a more detailed description of MS-configurations, see GSM 02.06.

3.1.2	Applicability of this specification

If a Mobile Station is equipped with a connector, to connect terminal equipment on an S or R interface, then testing of the Mobile Station may include testing of appropriate functioning to and from this connector.

This specification does not apply to terminal equipment which is to be connected to that connector (which constitutes a public interface), even if it is delivered with the MS.

3.1.3	Applicability of the individual tests

The applicability of each individual test is identified in the definition and applicability section of the test.

3.1.4	Applicability to terminal equipment

If an MS is delivered for conformance testing, and it contains physically integrated terminal equipment, then this specification applies to the complete Mobile Station including that terminal equipment.

This specification also applies to separate terminal equipment that is delivered for conformance testing with the Mobile Station. The MS is then tested as an MT0. In that case, the specific terminal equipment with which the Mobile Station is tested is documented in the test report.

3.2	Vocabulary

For definitions, abbreviations and acronyms, see GSM 01.04.

In addition, the following definitions are used in this technical specification:

"idle updated"

The mobile station is defined to be "idle updated" if the following three conditions are fulfilled:

-	its update status is U1 UPDATED (cf. TS GSM 04.08);

-	it is in the MM state MM-IDLE (cf. TS GSM 04.08);

-	it is in the RR idle mode (cf. TS GSM 04.08).

"idle not updated"

The mobile station is defined to be "idle not updated" if the following three conditions are fulfilled:

-	its update status is U2 NOT UPDATED (cf. TS GSM 04.08);

-	it is in the MM state MM-IDLE (cf. TS GSM 04.08);

-	it is in the RR idle mode (cf. TS GSM 04.08).

"arbitrary"

When it is said that for a test, a test purpose, a test group, or a test suite, which uses a certain parameter: the value of that parameter has to be chosen arbitrarily in a certain set of values, this means that

-	for each value in the set the MS is required to fulfil the requirements of the test, test purpose, test group, or  test suite, but that

-	the test, test purpose, test group, or  test suite is only performed for one value in the set, the selection of which is made by the test operator.

3.3	Conventions for mathematical notations

Some mathematical terms used throughout this ETS are shown below.

3.3.1	Mathematical signs

The "plus or minus" sign is expressed by "+/-".

The sign "multiplied by" is expressed by "*".

The sign "divided by" is expressed by "/", or the common division bar.

The sign "greater than or equal to" is expressed by ">=".

The sign "less than or equal to" is expressed by "<=".

3.3.2	Powers to the base 10

Powers to the base 10 are expressed by "10Ex", where x is the exponent, e.g. 10E-5, 10E6.

3.4	Conventions on electrical terms

3.4.1	RF input signal level

In general, the RF input signal level to the MS is expressed in terms of the received field strength E in dBmV/m (assuming a 0 dBi gain antenna). This is related to the power level P in dBm by the following formula (see GSM 05.05)

GSM900:	E (dBmV/m) = P (dBm) + 136.5 (calculated for a frequency of 925 MHz).

DCS1800:	E (dBmV/m) = P (dBm) + 142.3 (calculated for a frequency of 1795 MHz).

According to section [II.4.2.2.3], in all tests in which a handheld MS normally only equipped with integral antenna is the unit under test, the equivalent input signal level into a temporary test connector is determined from :

Ein 	=	Ereq + F

where:	Ein	= input signal level to a temporary antenna connector (dBmVemf);

Ereq	= signal level required by the test (dBmVemf);

F	= coupling factor (dB) at the respective ARFCN.

Since F has to be determined by each test house individually, Ein cannot be given as a figure in test procedures.

If the case of integral antenna is applicable, the input signal level, therefore, is expressed in the test procedures as:

Ereq dBmVemf(  ),

where the empty parenthesis is to be read as Ein.

Alternatively, the input signal level to the MS at the antenna connector can be expressed in dBmVemf(  ). This is related to the power level P in dBm by the following formula, assuming a 50 ohm antenna connector:

Input signal level (dBmVemf(  )) = P(dBm) + 113

3.4.2	Reference sensitivity level

In this ETS the term

Reference Sensitivity level ( )

is used to indicate that the SS establishes reference sensitivity level taking account of any loses associated with the RF connection to the MS.

3.4.3	Power level of fading signal

The power level of a fading signal is defined as the total signal level averaged over time.

3.5	Terms on test conditions

3.5.1	Radio test conditions

The radio propagation conditions refer to multipath propagation models of GSM 05.05.

They are expressed by typical profiles:

static

rural area	(RA),

hilly terrain	(HT),

urban area	(TU),

or for equalisation test	(EQ).

The non-static profiles are also related to typical speeds of movement of the MS expressed in km/h, e.g. TU1.5, TU3, TU50, HT100, EQ50.

In this ETS the following abbreviations are used:

Table 3.1

GSM 900	DCS 1800

RA	RA250	RA130

HT	HT100	HT100

TUhigh	TU50	TU50

TUlow	TU3	TU1.5

EQ	EQ50	EQ50

For tests using ARFCN ranges the following table shall be used.

Table 3.2

P-GSM 900	DCS 1800	E-GSM 900

Low ARFCN range	1 to 5	513 to 523	975 to 980

Mid ARFCN range	60 to 65	690 to 710	60 to 65

High ARFCN range	120 to 124	874 to 884	120 to 124

Note 1:	For definitions of P-GSM 900, DCS 1800 and E-GSM 900 refer to GSM 05.05.

Note 2:	In this ETS the term 'GSM 900' is used to cover the primary GSM band and the extended GSM band.