12.	Transceiver

12.1	Conducted spurious emissions

12.1.1	MS allocated a channel

12.1.1.1	Definition and applicability

Conducted spurious emissions, when the MS has been allocated a channel, are emissions from the antenna connector at frequencies other than those of the carrier and sidebands associated with normal modulation.

The requirements and this test apply to all types of GSM900 and DCS 1800 MS with a permanent antenna connector.

12.1.1.2	Conformance requirement

1.	The conducted spurious power emitted by the MS, when allocated a channel, shall be no more than the levels in Table 12.1.

1.1	Under normal voltage conditions; GSM 05.05, 4.3/4.3.3

1.2	Under extreme voltage conditions; GSM 05.05, 4.3/4.3.3/Annex D.2

Table 12.1

Frequency range	Power level in dBm

GSM 900	DCS 1800

9 kHz to	1 GHz	-36	-36

1 GHz to	12.75 GHz	-30

1 GHz to	1710 MHz		-30

1710 MHz to	1785 MHz		-36

1785 MHz to	12.75 GHz		-30

12.1.1.3	Test purpose

1.	To verify that conducted spurious emissions, in the frequency band 100 kHz to 12.75 GHz excluding the GSM 900 and DCS 1800 receive bands, from the MS when allocated a channel do not exceed the conformance requirements.

1.1 Under normal voltage conditions.

1.2 Under extreme voltage conditions.

Note 1	The band 9 - 100 kHz is not tested, because of test implementation problems.

12.1.1.4	Method of test

12.1.1.4.1	Initial conditions

A call is set up by the SS according to the generic call set up procedure on a channel in the Mid ARFCN range.

The SS commands the MS to loop back its channel decoder output to channel encoder input.

The SS sends Standard Test Signal C1.

The SS sets the MS to operate at its maximum output power.

12.1.1.4.2	Procedure

a)	Measurements are made in the frequency range 100 kHz to 12.75 GHz. Spurious emissions are measured at the connector of the transceiver, as the power level of any discrete signal, higher than the requirement in Table 12.1 minus 6 dB, delivered into a 50 Ohm load.

The measurement bandwidth based on a 5 pole synchronously tuned filter is according to Table 12.2. The power indication is the peak power detected by the measuring system.

The measurement on any frequency shall be performed for at least one TDMA frame period with the exception of the idle frame.

Note 2	This ensures that both the active times (MS transmitting) and the quiet times are measured.

b)	The test is repeated under extreme voltage test conditions ([Annex 1, TC2.2 and TC3]).

Table 12.2

Frequency range	Frequency offset	Filter bandwidth	Approx video

bandwidth

100 kHz to 50 MHz	-	10 kHz	30 kHz

50 to 500 MHz	-	100 kHz	300 kHz

500 MHz to 12.75 GHz,	0 to 10 MHz	100 kHz	300 kHz

>= 10 MHz	300 kHz	1 MHz

excl. relevant TX band:		>= 20 MHz	1 MHz	3 MHz

P-GSM: 890 to 915 MHz;		>= 30 MHz	3 MHz	3 MHz

E-GSM: 880 to 915 MHz;

DCS: 1710 to 1785 MHz,		(offset from edge

and the RX bands:		of relevant TX band)

925 to 960 MHz;

1805 to 1880 MHz.

relevant TX band:

P-GSM: 890 to 915 MHz		1.8 to 6.0 MHz	30 kHz	100 kHz

E-GSM: 880 to 915 MHz		> 6.0 MHz	100 kHz	300 kHz

DCS: 1710 to 1785 MHz		(offset from carrier)

Note 3	The frequency ranges 925 MHz to 960 MHz and 1805 MHz to 1880 MHz are excluded as these ranges are tested in section [II.3.4].

Note 4	The filter and video bandwidths, and frequency offsets are only correct for measurements on an MS transmitting on a channel in the Mid ARFCN range.

Note 5	Due to practical implementation, the video bandwidth is restricted to a maximum of 3 MHz.

12.1.1.5	Test requirement

The power of any spurious emission shall not exceed the levels given in Table 12.3.

Table 12.3

Frequency range	Power level in dBm

GSM 900	DCS 1800

100 kHz to	1 GHz	-36	-36

1 GHz to	12.75 GHz	-30

1 GHz to	1710 MHz		-30

1710 MHz to	1785 MHz		-36

1785 MHz to	12.75 GHz		-30

12.1.2	MS in idle mode

12.1.2.1	Definition and applicability

Conducted spurious emissions are any emissions from the antenna connector, when the MS is in idle mode.

The requirements and this test apply to all types of GSM 900 and DCS 1800 MS with a permanent antenna connector.

12.1.2.2	Conformance requirement

1.	The conducted spurious power emitted by the MS, when in idle mode, shall be no more than the levels in Table 12.4.

1.1	Under normal voltage conditions; GSM 05.05, 4.3/4.3.3

1.2	Under extreme voltage conditions; GSM 05.05, 4.3/4.3.3/Annex D.2

Table 12.4

Frequency range	Power level in dBm

9 kHz to	880 MHz	-57

880 MHz to	915 MHz	-59

915 MHz to	1000 MHz	-57

1 GHz to	1710 MHz	-47

1710 MHz to	1785 MHz	-53

1785 MHz to	12.75 GHz	-47

12.1.2.3	Test purpose

1.	To verify that conducted spurious emissions, in the frequency band 100 kHz to 12.75 GHz from the MS when in idle mode do not exceed the conformance requirements.

1.1 Under normal voltage conditions.

1.2 Under extreme voltage conditions.

Note 6	The band 9 - 100 kHz is not tested, because of test implementation problems.

12.1.2.4	Method of test

12.1.2.4.1	Initial conditions

The BCCH message content from the serving cell shall ensure that Periodic Location Updating is not used and that page mode is continuously set to Paging Reorganisation and BS_AG_BLKS_RES is set to 0 so that the MS receiver will operate continually.

The BCCH allocation shall either be empty or contain only the serving cell BCCH.

Note 7	This is to ensure that the receiver does not scan other ARFCN. Scanning other ARFCN could lead to a moving in frequency of the spurii and therefore to the possibility of either not measuring a spurious emission or measuring it more than once.

The MS is in MM state "idle, updated".

12.1.2.4.2	Procedure

a)	Measurements are made in the frequency range 100 kHz to 12.75 GHz. Spurious emissions are measured as the power level of any discrete signal, higher than the requirement in Table 12.4 minus 6 dB, delivered into a 50 Ohm load.

The measurement bandwidth based on a 5 pole synchronously tuned filter is set according to Table 12.5. The power indication is the peak power detected by the measuring system.

The measurement time on any frequency shall be such that it includes the time during which the MS receives a TDMA frame containing the paging channel.

Table 12.5

Frequency range	Filter bandwidth	Video bandwidth

100 kHz to 50 MHz	10 kHz	30 kHz

50 MHz to 12.75 GHz	100 kHz	300 kHz

b)	The test is repeated under extreme voltage test conditions ([Annex 1, TC2.2 and TC3]).

12.1.2.5	Test requirement

The power of any spurious emission shall not exceed the levels given in Table 12.6.

Table 12.6

Frequency range	Power level in dBm

100 kHz to	880 MHz	-57

880 MHz to	915 MHz	-59

915 MHz to	1000 MHz	-57

1 GHz to	1710 MHz	-47

1710 MHz to	1785 MHz	-53

1785 MHz to	12.75 GHz	-47

12.2	Radiated spurious emissions

This test is performed either on an outdoor test site, fulfilling the requirements of [GC4 of Annex 1], or in an anechoic shielded chamber, fulfilling the requirements of ([GC5 of Annex 1)]. Performing the measurement in the anechoic shielded chamber is preferred. The sample shall be placed at the specified height on the support.

Note 8	The test method described has been written for measurement in an anechoic shielded chamber. If an outdoor test site is used then additional precautions are necessary to ensure correct measurement. These measures are familiar to test houses which perform spurious emissions tests and are:

a) Raise/lower the test antenna through the specified height range during both the emission detection and substitution parts of the test.

b) Perform a qualitative pre-search in a shielded environment for test sites where the ambient RF environment can prevent the detection of spurious emissions which exceed the limit.

c) Detect emissions at a more sensitive threshold to that specified in section 12.2.1.4 to allow for destructive interference due to ground plane reflections at the test antenna search height.

12.2.1	MS allocated a channel

12.2.1.1	Definition and applicability

Radiated spurious emissions, when the MS has been allocated a channel, are any emissions radiated by the cabinet and structure of the mobile station, including all interconnecting cables.

This is also known as "cabinet radiation".

The requirements and this test apply to all types of GSM 900 and DCS 1800 MS.

12.2.1.2	Conformance requirement

1.	The radiated spurious power emitted by the MS, when allocated a channel, shall be no more than the levels in Table 12.7.

1.1	Under normal voltage conditions; GSM 05.05, 4.3/4.3.3

1.2	Under extreme voltage conditions; GSM 05.05, 4.3/4.3.3/Annex D.2

Table 12.7

Frequency range	Power level in dBm

GSM 900	DCS 1800

30 MHz to	1 GHz	-36	-36

1 GHz to	4 GHz	-30

1 GHz to	1710 MHz		-30

1710 MHz to	1785 MHz		-36

1785 MHz to	4 GHz		-30

12.2.1.3	Test purpose

1.	To verify that radiated spurious emissions from the MS when allocated a channel do not exceed the conformance requirements.

1.1 Under normal voltage conditions.

1.2 Under extreme voltage conditions.

12.2.1.4	Method of test

12.2.1.4.1	Initial conditions

A call is set up by the SS according to the generic call set up procedure on a channel in the Mid ARFCN range.

The SS commands the MS to loop back its channel decoder output to its channel encoder input.

The SS sends Standard Test Signal C1.

The SS sets the MS to operate at its maximum output power.

12.2.1.4.2	Procedure

a)	Initially the test antenna is closely coupled to the MS and any spurious emission radiated by the MS is detected by the test antenna and receiver in the range 30 MHz to 4 GHz.

Note 9	This is a qualitative step to identify the frequency and presence of spurious emissions which are to be measured in subsequent steps.

b)	The test antenna separation is set to the appropriate measurement distance and at each frequency at which an emission has been detected, the MS shall be rotated to obtain maximum response and the effective radiated power of the emission determined by a substitution measurement. In case of an anechoic shielded chamber pre-calibration may be used instead of a substitution measurement.

c)	The measurement bandwidth, based on a 5 pole synchronously tuned filter, is set according to Table 12.8. The power indication is the peak power detected by the measuring system.

The measurement on any frequency shall be performed for at least one TDMA frame period, with the exception of the idle frame.

Note 10	This ensures that both the active times (MS transmitting) and the quiet times are measured.

Note 11	For these filter bandwidths some difficulties may be experienced with noise floor above required measurement limit. This will depend on the gain of the test antenna, and adjustment of the measuring system bandwidth is permissible. Alternatively, for test frequencies above 900 MHz, the test antenna separation from the MS may be reduced to 1 metre.

d)	The measurements are repeated with the test antenna in the orthogonal polarisation plane.

e)	The test is repeated under extreme voltage test conditions (see [Annex 1, TC2.2]).

Table 12.8

Frequency range	Frequency offset	Filter bandwidth	Approx video

bandwidth

30 to 50 MHz	-	10 kHz	30 kHz

50 to 500 MHz	-	100 kHz	300 kHz

500 MHz to 4 GHz,	0 to 10 MHz	100 kHz	300 kHz

,	>= 10 MHz	300 kHz	1 MHz

excl. relevant TX band:		>= 20 MHz	1 MHz	3 MHz

P-GSM: 890 to 915 MHz;		>= 30 MHz	3 MHz	3 MHz

E-GSM: 880 to 915 MHz;

DCS: 1710 to 1785 MHz.		(offset from edge

of relevant TX band)

relevant TX band:

P-GSM: 890 to 915 MHz		1.8 to 6.0 MHz	30 kHz	100 kHz

E-GSM: 880 to 915 MHz		> 6.0 MHz	100 kHz	300 kHz

DCS: 1710 to 1785 MHz		(offset from carrier)

Note 12	The filter and video bandwidths, and frequency offsets are only correct for measurements on an MS transmitting on a channel in the Mid ARFCN range.

Note 13	Due to practical implementation of a SS, the video bandwidth is restricted to a maximum of 3 MHz.

12.2.1.5	Test requirement

The power of any spurious emission shall not exceed the levels given in Table 12.7.

12.2.2	MS in idle mode

12.2.2.1	Definition and applicability

Radiated spurious emissions, when the MS is in idle mode, are any emissions radiated by the cabinet and structure of the mobile station, including all interconnecting cables.

This is also known as "cabinet radiation".

The requirements and this test apply to all types of GSM 900 and DCS 1800 MS.

12.2.2.2	Conformance requirement

1.	The radiated spurious power emitted by the MS, when in idle mode, shall be no more than the levels in Table 12.9.

1.1	Under normal voltage conditions; GSM 05.05, 4.3/4.3.3

1.2	Under extreme voltage conditions; GSM 05.05, 4.3/4.3.3/Annex D.2

Table 12.9

Frequency range	Power level in dBm

30 MHz to	880 MHz	-57

880 MHz to	915 MHz	-59

915 MHz to	1000 MHz	-57

1 GHz to	1710 MHz	-47

1710 MHz to	1785 MHz	-53

1785 MHz to	4 GHz	-47

12.2.2.3	Test purpose

1.	To verify that radiated spurious emissions from the MS when in idle mode do not exceed the requirements.

1.1 Under normal voltage conditions.

1.2 Under extreme voltage conditions.

12.2.2.4	Method of test

12.2.2.4.1	Initial conditions

The BCCH message content from the serving cell shall ensure that Periodic Location Updating is not used and that page mode is continuously set to Paging Reorganisation and BS_AG_BLKS_RES is set to 0 so that the MS receiver will operate continually.

The BCCH allocation shall either be empty or contain only the serving cell BCCH.

Note 14	This is to ensure that the receiver does not scan other ARFCN. Scanning other ARFCN could lead to a moving in frequency of the spurii and therefore to the possibility of either not measuring a spurious emission or measuring it more than once.

The MS is in MM state "idle, updated".

12.2.2.4.2	Procedure

a)	Initially the test antenna is closely coupled to the MS and any spurious emission radiated by the MS are detected by the test antenna and receiver in the range 30 MHz to 4 GHz.

Note 15 	This is a qualitative step to identify the frequency and presence of spurious emissions which are to be measured in subsequent steps.

b)	The test antenna separation is set to the appropriate measurement distance and at each frequency at which a spurious emission has been detected the MS is rotated to obtain a maximum response. The effective radiated power of the emission is determined by a substitution measurement. In case of an anechoic shielded chamber pre-calibration may be used instead of a substitution measurement.

c)	The measurement bandwidth based on a 5 pole synchronously tuned filter shall be according to Table 12.10. The power indication is the peak power detected by the measuring system.

The measurement time on any frequency shall be such that it includes the time during which the MS receives a TDMA frame containing the paging channel.

Note 16	For these filter bandwidths some difficulties may be experienced with noise floor above required measurement limit. This will depend on the gain of the test antenna, and adjustment of the measuring system bandwidth is permissible.. Alternatively, for test frequencies above 900 MHz, the test antenna separation from the MS may be reduced to 1 metre.

Table 12.10

Frequency range	Filter bandwidth	Video bandwidth

30 MHz to 50 MHz	10 kHz	30 kHz

50 MHz to 4 GHz	100 kHz	300 kHz

d)	The measurements are repeated with the test antenna in the orthogonal polarisation plane.

e)	The test is repeated under extreme voltage test conditions (see [Annex 1, TC2.2]).

12.2.2.5	Test requirement

The power of any spurious emission shall not exceed the levels given in Table 12.9.