20	Cell selection and reselection tests

In the following paragraphs the general behaviour of the SS is described and some explanatory text given concerning the nature of the tests in this section.

The SS transmits one BCCH carrier per cell. These are referred to as carrier 1, carrier 2, etc. It is assumed that the SS can simultaneously transmit six BCCH carriers and monitor three RACH channels. In some cases, a test is performed in two stages in order that the requirements can be tested within the above constraints.

Unless otherwise stated in the method of test, in all of the tests of this section, the SS is continuously paging the MS on all carriers at the start of the test and does not respond to RACH requests from the MS. At every response to paging the SS stops paging on that carrier and starts or continues on all other carriers as indicated in the test description.

After the final procedural step has been completed, the SS waits [30] seconds and monitors for any false random accesses on the test RF carriers.

Since the conformance requirements of most of the tests in this section cannot be tested explicitly, testing is done implicitly by testing the MS behaviour from its responses to the SS.

The conformance requirements are direct extracts from the core recommendations. However, for each test only the relevant parts of the paragraphs/sentences in question have been included in the conformance requirements. This explains the use of "..." marks in the conformance requirements.

The absolute accuracy of the MS signal level measurements is assumed to be +/-6 dB. In the tests of this section, a difference of at least 8 dB is allowed for cases of discrimination between C1 or C2 values and 0.

The relative accuracy of the MS signal level measurements is assumed to be +/- [TBD] dB worst case under all signal conditions. A difference of at least [5] dB is allowed for cases of discrimination between C1 or C2 values on different carriers.

Note :	Random selection of BCCH carriers cannot be used because of the repeatability of tests.

The default values of the system information data fields are given in table 20.1. Differencies from the default values are given in the initial conditions of each test.

Table 20.1.: Default values of the system information fields

Parameter	GSM 04.08	Abbr.	Normal Setting

reference

Cell channel description	10.5.2.1	 -	Any values

MAX retrans	10.5.2.29	 -	1

TX-integer	10.5.2.29	 -	Any value

CELL_BAR_QUALIFY	10.5.2.35	CBQ	0

CELL_BAR_ACCESS	10.5.2.29	CBA	0 (not barred)

AC CN	10.5.2.29	AC	All 0

RE	10.5.2.29	RE	0 (re-establishment allowed)

NCC	10.5.2.2	NCC	Any value

Cell Identity	10.5.1.1	 -	Any value

MCC, MNC	10.5.1.3	PLMN	MS Home PLMN

LAC	10.5.1.3	LAC	1111 (Hex)

ATT	10.5.2.11	 -	0 (Attach/Detatch not allowed)

BS_AG_BLKS_RES	10.5.2.11	 -	Any values

T3212	10.5.2.11	 -	Any values

BS_PA_MFRMS	10.5.2.11	BPM	5 frames

Cell Options	10.5.2.3	 -	Any values

CELL_RESELECT_HYSTERESIS	10.5.2.4	CRH	4 dB

MS_TXPWR_MAX_CCH	10.5.2.4	MTMC	Max. output power of MS

RXLEV_ACCESS_MIN	10.5.2.4	RAM	-90 dBm

CELL_RESELECT_OFFSET	10.5.2.35	CRO	0

TEMPORARY_OFFSET	10.5.2.35	TO	0

PENALTY_TIME	10.5.2.35	PT	0

BA ARFCN	10.5.2.22	BA	All 0 except :

For GSM ARFCNs

3,9,18,25,41,43,49,50,54,58,

62,66,70,80,92,124

For DCS1800 ARFCNs

512,543,568,589,602,641,662,

683,696,711,732,754,794,851,

870,871,872,884

20.1	Cell selection

20.1.1	Definition and applicability

Cell selection is a process in which a MS, whenever a new PLMN is selected, attempts to find a suitable cell of that PLMN to camp on. Two methods of searching for a suitable cell are possible, normal cell selection and stored list cell selection. The process ensures that the MS is camped on a cell from which it can reliably decode downlink data and with which it has a high probability of communications on the uplink. Once the MS is camped on a cell, access to the network is allowed.

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.1.2	Conformance requirement

1.	The MS shall be able to select the correct (fourth strongest) cell and be able to respond to paging on that cell within 30 seconds when the three strongest cells are not suitable. This assumes ideal radio conditions; GSM 05.08, 6.1.

2.	There are various requirements that a cell must satisfy before an MS can perform normal camping on it:

2.1	(i)	It should be a cell of the selected PLMN

2.2	(ii)	It should not be "barred" (see Section 3.5.1)

2.3	(iv)	The radio path loss between MS and BTS must be below a threshold set by the PLMN operator. This is estimated as shown in Section 3.6.

Note :	Criteria (iii) is not applicable for Cell Selection

3.	Initially the MS looks for a cell which satisfies these 4 constraints ("suitable cell") by checking cells in descending order of received signal strength. If a suitable cell is found, the MS camps on it; GSM 03.22, 3.2.1.

4.	The MS shall be able to calculate correctly the path loss criterion parameter C1, used for cell selection and reselection; GSM 05.08, 6.4.

20.1.3	Test purpose

1.	To verify that the MS meets conformance requirement 1.

2.	To verify that:

2.1	The MS does not select a cell of a PLMN which is not the selected PLMN

2.2	The MS does not select a cell which is "barred".

2.3	The MS does not select a cell with C1<0.

3.	To verify that the MS selects suitable cells in descending order of received signal strength.

4.	To verify that the MS does not select a cell with C1<0.

20.1.4	Method of test

20.1.4.1	Initial conditions

Parameters changed from the default values in Table 20.1.

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBmV emf() / dBm )	48 / -65	38 / -75	43 / -70	33 / -80	28 / -85OFF	OFF

CBA	1	0	0	0	0

RXLEV_ACCESS_MIN

(dBm)	-90	-67	-90	-88	-98

MNC			001

NCC			02

C1	25	-8	20	8	13

C2	25	-8	20	8	13

The ARFCNs for the carriers are chosen from the BA ARFCNs in table 20. Adjacent carriers are separated by a minimum of three channels

The SIM is in the idle updated state in the default location area with a TMSI assigned.

20.1.4.2	Procedure

a)	The MS is switched on.

b)	After completion of the MS boot time declared by the manufacturer, the SS activates the carriers and monitors carriers 2, 4 and 5 for RA requests from the MS for 30 s.

c)	The MS is switched off

d)	The MS is swtched on.

e)	After completion of the MS boot time declared by the manufacturer, the SS activates the carriers and monitors carriers 1 and 3 for RA requests from the MS for 30 s.

20.1.5	Test requirements

1)	In step b), the MS shall respond on carrier 4 before a response if any, on carrier 5. There shall be no response from the MS on carrier 2.

2)	In step e), there shall be no response from the MS on either carrier 1 or carrier 3.

20.2	Cell selection with varying signal strength values

20.2.1	Definition and applicability

For definition see conformance requirement.

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.2.2	Conformance requirement

1.	The MS shall ...

1.1	take readings of received RF signal strength ... and calculate the received level average for each.

1.2	The averaging is based on at least five measurement samples per RF carrier spread over 3 to 5 s,

1.3	the measurement samples from the different RF carriers being spread evenly during this period;

GSM 05.08, 6.2.

2.	These quantities are termed the "receive level averages", shall be unweighted averages of the received signal strengths measured in dBm. GSM 05.08, 6.1

20.2.3	Test purpose

1.	To verify that:

1.1	The MS meets conformance requirement 1.1.

1.2	The MS meets conformance requirement 1.2.

1.3	The MS meets conformance requirement 1.3.

2	To verify that the MS meets conformance requirement 2.

[20.2.4	Method of test

20.2.4.1	Initial conditions

Parameters changed from the default values in Table 20.1.

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBmV emf() / dBm )	23 / -90	66 / -47	OFF	OFF	OFF	OFF

RXLEV_ACCESS_MIN

(dBmV emf() / dBm)	13 / -100	13 / -100

C1	10	53

The ARFCNs for the carriers are chosen from the BA ARFCNs in table 20. Adjacent carriers are separated by a minimum of three channels

20.2.4.2	Procedure

a)	The SS transmits on carriers 1 and 2. After 1.2 seconds carrier 2 reduces its transmit level to -97 dBm (16 dBmV emf( )). After a further 2.2 seconds, carrier 2 increases its transmit level again to -47 dBm (66 dBmV emf( )). Switching of carrier 2 continues with these levels and duty cycle until the end of the test.

b)	The MS is switched on

c)	The SS monitors all RA requests from MS on carriers 1 to 3

20.2.5	Test requirements

The MS shall respond only on carrier 2.

Note :	With the selected levels and duty cycle it can be guaranteed that a "good" MS passes the test even at the worst case situation, which happens with 3 s averaging time if 4 out of 5 samples hit the low signal level (result is an average level of 26 dBmV which is still 6 dB above carrier 1 level).

Note :	With the selected levels and duty cycle the probability that a "bad" MS (i.e. MS that averages over shorter period than 3 s) failes the test is maximised. However, it can not be guaranteed that all the MSs not fulfilling the conformance requirement of averaging or uniform sampling will fail this test.

Note :	The 50 dB change in the levels is needed to guarantee a good MS pass in all situations. However, it has to be verified that the SS is capable of doing a 50 dB step with a short settling time (compared to the sampling).]

20.3	Basic cell reselection

20.3.1	Definition and applicability

While camped on a cell of the selected PLMN the MS may need to select a different cell in order to fulfil the normal service state. This ensures that the MS is camped on a cell from which it can reliably decode downlink data and with which it has a high probability of communications on the uplink.

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.3.2	Conformance requirement

1.	While camped on a cell of the selected PLMN ("camped normally"), the MS may need to select a different cell ("normal cell reselection" state). The following events trigger a cell reselection:

1.1	(iii)	The cell camped on (current serving cell) has become barred.

1.2	(iv)	There is a better cell (in terms of the path loss criterion C2) in the same LA, or a much better cell in another LA of the selected PLMN (using the CRH parameter).

The MS will then reselect a new cell in order to fulfil the process goal.; GSM 03.22, 4.5.

Note :	Criterion (i) is tested in section 20.6 (Cell reselection when C1(serving cell) < 0 for 5 seconds

Note :	Criterion (ii) is tested section 20.15 (Downlink Signalling Failure test)

Note :	Criterion (v) is tested in section 20.4.

Note :	Criterion (vi) is tested in section 20.14 (location area not allowed test)

2.	There are various requirements that a cell must satisfy before an MS can perform normal camping on it:

2.1	(ii)	It should not be "barred"

2.2	(iv)	The radio path loss between MS and BTS must be below a threshold set by the PLMN operator. GSM 03.22, 3.2.1.

Note :	Criterion (i) is not relevant for cell reselection and for cell selection it is tested in section 20.1.

Note :	Criterion (iii) is tested in section 20.14. (location area not allowed for national roaming)

Note :	Criterion (iv) refers to the C1 parameter

3.	The MS shall be able to calculate correctly the path loss criterion parameter C2 used for cell reselection; GSM 05.08, 6.4.

4.	.... the MS shall calculate the value of ... C2 for the serving cell and recalculate ... C2 values for non serving cells (if necessary). The MS shall then check whether:

ii)	the calculated value of C2 for a nonserving suitable cell exceeds the value of C2 for the serving cell ..., except in the case of the new cell being in a different location area in which case the C2 value for the new cell shall exceed the C2 value of the serving cell by at least CELL_RESELECT_HYSTERESIS dB as defined by the BCCH data from the current serving cell, .... This indicates that it is a better cell; GSM 05.08, 6.6.2.

20.3.3	Test purpose

1.	To verify that:

1.1	 The MS meets conformance requirement 1.1.

1.2	The MS performs a cell reselection if the value of C2 for a non-serving suitable cell in the same LA exceeds the value of C2 for the serving cell

2.	To verify that:

2.1	The MS does not reselect a cell which is barred.

2.2	The MS does not reselect a cell which has a C1<0.

3.	To verify that the MS takes into account the CELL_RESELECT_OFFSET, TEMPORARY_OFFSET and PENALTY_TIME parameters when calculating C2.

4.	To verify that the MS takes into account the CELL_RESELECT_HYSTERESIS parameter when reselecting a cell in a different location area.

20.3.4	Method of test

20.3.4.1	Initial conditions

Parameters changed from the default values in Table 20.1.

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBmV emf() / dBm )	53 / -60	43 / -70	33 / -80	23 / -90	38 / -75	OFF

RXLEV_ACCESS_MIN

(dBm)	-80	-80	-90	-100	-67

Penalty time	11111	11111	50 s	70 s	30s

CRH	10 dB	10 dB	10 dB	10 dB	10 dB

CRO	20	5	5	10	10

Temporary offset	10	10	15	20	20

LAC				different

from other

carriers

C1	20	10	10	10	-8

C2	0	5	0 -> 15	0 -> 20	2 -> 12

The ARFCNs for the carriers are chosen from the BA ARFCNs in table 20. Adjacent carriers are separated by a minimum of three channels.

The SIM is in the idle updated state in the default location area with a TMSI assigned.

20.3.4.2	Procedure

a)	The MS is switched on.

b)	After completion of the boot time declared by the manufacturer, the SS activates the carriers. No paging messages are transmitted on carrier 1. The SS monitors carriers 1, 2 and 3 for RA requests from the MS for 60 s.

c)	The MS is switched off. The SS deactivates the carriers.

d)	The MS is switched on again.

e)	After completion of the boot time declared by the manufacturer, the SS activates the carriers. No paging messages are transmitted on either carrier 1 or carrier 2. The SS monitors carriers 1, 2 and 3 for RA requests from the MS for [90] s.

f)	The MS is switched off.

g)	The MS is switched on again.

h)	After completion of the boot time declared by the manufacturer, the SS activates the carriers. No paging messages are transmitted on either carriers 1, 2 or 3. The SS monitors carriers 4 and 5 for RA requests from the MS for [120] s.

20.3.5	Test requirements

1)	In step b), the first response from the MS shall be on carrier 2 within 42 s.

Note :	Carrier 1 has a higher signal level than carrier 2 but a lower C2, Therefore, initial cell selection should choose carrier 1, followed by reselection of carrier 2.

2)	In step e), the first response from the MS shall be on carrier 3 within [80] seconds.

3)	In step h), there shall be no response from the MS on either carrier 4 or carrier 5 within [110] seconds.

20.4	Cell reselection timings

20.4.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.4.2	Conformance requirement

1.	At least every 5 s the MS shall calculate the value of ... C2 for the serving cell and re-calculate ... C2 values for non serving cells (if necessary). The MS shall then check whether:...

1.1	ii)	The calculated value of C2 for a non-serving suitable cell exceeds the value of C2 for the serving cell for a period of 5 seconds, ...

1.2	In case ii) above, cell reselection shall not take place if there was a cell reselection within the previous 15 seconds.

1.3	Cell reselection for any other reason (see GSM 03.22) shall take place immediately, but the cell that the MS was camped on shall not be returned to within 5 seconds if another suitable cell can be found.;

GSM 05.08, 6.6.2.

20.4.3	Test purpose

1.	To verify that:

1.1	The MS does not perform a cell reselection when the C2 value for a non serving cell does not exceed the C2 value of the serving cell for a period of at least 5 seconds.

1.2	The MS meets conformance requirement 1.2.

1.3	When the MS performs an immediate cell reselection due to an unsuccessful random access attempt, the cell that the MS was camped onto is not returned to within 5 seconds.

20.4.4	Method of test

20.4.4.1	Initial conditions

Parameters changed from the default values in Table 20.1.

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBmV emf() / dBm )	56 / -57	46 / -67	46 / -67	OFF	OFF	OFF

RXLEV_ACCESS_MIN

(dBmV emf() / dBm)	29 / -84	33 / -80	38 / -75

Max. Retrans	00	00	00

C1	27	13	8

C2	27	13	8

The ARFCNs for the carriers are chosen from the BA ARFCNs in table 20. Adjacent carriers are separated by a minimum of three channels.

The SIM is in the idle updated state in the default location area with a TMSI assigned.

20.4.4.2	Procedure

a)	The MS is switched on.

b)	The SS monitors all RA requests from MS on carriers 1 and 2.

c)	After the SS has received an access on carrier 1, increase transmit level on carrier 2 to 70 dBmV emf( ) for 4 s so that C2 becomes 37, then drop level back to original.

Note :	With the selected RF level values and assuming a correct RF level sampling and averaging, the C2 of carrier 2 does not exceed that of carrier 1 for longer than about 4 s.

d)	The SS monitors all RA requests from MS on carriers 1 and 2.

e)	Increase transmit level on carrier 2 to 70 dBmV emf( ) so that C2 becomes 37 and wait for access from MS on carrier 2.

f)	As soon as an access is received on carrier 2, the transmit level of carrier 1 shall be increased to 76 dBmV emf( ) so that C2 becomes 43.

g)	The SS monitors all RA requests from MS on carriers 1 and 2, but shall not respond to any RA request on carrier 1. The SS shall always respond normally on carrier 2.

20.4.5	Test requirements

a)	The MS accesses on carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer.

b)	There should be no access on carrier 2 within 25 seconds. (5 s for rxlev averages, 5 s for calculation of C2 (absolute worst case scenario), 5 s for C2 check duration and 10 s for possible faulty access).

c)	The MS shall access on carrier 2 within 25 seconds of increasing the level in step (e).

d)	The MS should not access carrier 1 within 13 s (15 seconds for reselection and 2 seconds for possible differences in random access/paging timings) of the MS access on carrier 2.

e)	The MS shall then access carrier 1. (This access should probably be fairly soon after the expiry of the 15 s, probably safe to allow say 20s.)

f)	The MS shall not access on carrier 2 within 5 seconds of accessing on carrier 1 in test step (g).

20.5	Priority of cells

20.5.1	Definition and applicability

In general, cell prioritisation is a means of encouraging MSs to select some suitable cells in preference to others. Since the priority comparison is only between suitable cells, prioritisation does not affect coverage. Operators may prefer a certain type of cell not to be selected unless it is the only suitable type. For example, umbrella cells due to their large frequency reuse distance, or microcells because the MS could be travelling too fast for them.

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.5.2	Conformance requirement

1.	During cell selection a cell with low priority indication will only be selected if a suitable cell of normal priority cannot be found; GSM 03.22, 3.5.2.1.

2.	Table 1a. Parameters affecting cell priority for cell selection

CELL_BAR QUALIFY	CELL_BAR ACCESS	Cell selection priority	Status for cell reselection

0	0	normal	normal

0	1	barred	barred

1	0	low	normal (see note 2)

1	1	low	normal (see note 2)

GSM 05.08, Table 1.a

3.	If all the following conditions are met then the "Cell selection priority" and the "Status for cell reselection" shall be set to normal:

- the cell belongs to the MS HPLMN

- the MS is in cell test operation mode

- the CELL_BAR_ACCESS is set to "1"

- the CELL_BAR_QUALIFY is set to "0"

- the Access Control class 15 is barred

GSM 05.08, Table 1.a

20.5.3	Test purpose

1.	To verify that the MS meets conformance requirement 1.

2.	To verify that the MS meets conformance requirement 2.

3.	To verify that the MS meets conformance requirement 3.

20.5.4	Method of test

20.5.4.1	Initial conditions

The MS is in cell test operation mode

Note :	The cell test mode is a mode of operation defined in SIM administrative data field (see 11.11). It is used to set up a cell for test purposes without being used by normal subscribers.

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBmV emf() / dBm )	53 / -60	43 / -70	33 / -80	23 / -90	OFF	OFF

RXLEV_ACCESS_MIN

(dBmV emf() / dBm)	13 / -100	13 / -100	13 / -100	13 / -100

CRO	0	20	0	20

TO	40	0	0	20

PT	100 s (4)	11111	11111	60 s (2)

CBA	0	1	1	0

CBQ	1	1	0	0

Access class 15	barred	barred	barred	barred

C1	40	30	20	10

C2	0 -> 40	10	20	10 -> 30

20.5.4.2	Procedure

a)	The MS is switched on.

b)	The SS monitors all RA requests from MS on carriers 1 to 4.

20.5.5	Test requirements

a)	The MS shall access onto carrier 3 within 30 seconds of the boot up time of MS

b)	The MS shall access onto carrier 4 within 20 seconds of expiry of penalty time on carrier 4

c)	The MS shall access onto carrier 1 within 20 seconds of expiry of penalty time on carrier 1

d)	MS shall not access on carrier 2 during this test

Note :	The accesses to carriers 4 and 1 should not take much longer than 10 s after expiry of the penalty time (5 s for rxlev averages, 5 s for C2 check duration, 2.4 s for sync and BCCH data read on new carrier) 20 seconds is a safe time to wait.

20.6	Cell reselection when C1(serving cell) < 0 for 5 seconds

20.6.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.6.2	Conformance requirement

1.	At least every 5 s .... the MS shall calculate the value of C1...for the serving cell... The MS shall then check whether:

i)	The path loss criterion (C1) for current serving cell falls below zero for a period of 5 seconds. This indicates that the path loss to the cell has become too high; 05.08, 6.6.2.

2.	While camped on a cell of the selected PLMN ("camped normally"), the MS may need to select a different cell ("normal cell reselection" state). The following events trigger a cell reselection:

(i)	The path loss criterion parameter C1 (see Section 3.6) indicates that the path loss to the cell has become too high.; GSM 03.22, 4.5.

20.6.3	Test purpose

1.	To verify that the MS meets conformance requirement 1.

2.	To verify that the MS meets conformance requirement 2.

20.6.4	Method of test

20.6.4.1	Initial conditions

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBmV emf() / dBm )	49 / -64	28 / -85	OFF	OFF	OFF	OFF

RXLEV_ACCESS_MIN

(dBmV emf() / dBm)	33 / -80	18 / -95

C1	16	10

C2	16	10

Note :	This test assumes that the averaging of the serving cell RF level is a running average.

Note :	The RXLEV_ACCESS_MIN level for carrier 1 has to be above the RF level of carrier 2 to guarantee that the reselection is done because C1 of carrier 1 goes below zero and not because C2 of carrier 2 becomes greater than C2 of carrier 1.

20.6.4.2	Procedure

a)	The MS is switched on.

b)	After 30 seconds, SS reduces signal level on carrier 1 to -100 dBm / 13 dBmV emf() for 4 s. Then, raises back to -64 dBm / 49 dBmV emf().

c)	After a further 30 seconds, SS reduces signal level on carrier 1 to -100 dBm / 13 dBmV emf()

20.6.5	Test requirements

a)	The MS shall access on carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer. There shall be no access on carrier 2 within this time.

b)	There shall be no access on carrier 2 in the 30 seconds following the signal level changes in test step b).

c)	The MS shall access on carrier 2 within 20 seconds following the signal level changes in test step c).

Note :	The access should not take much longer than 10 s (5 s for rxlev averages, 5 s for C1 check duration on carrier 1, 2.4 s for sync and BCCH data read on carrier 2) 20 seconds is a safe time to wait.

20.7	Running average of the surrounding cell BCCH carrier signal levels

20.7.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.7.2	Conformance requirement

1.	Whilst in idle mode an MS shall continue to monitor all BCCH carriers as indicated by the BCCH allocation (BA - See table 1). A running average of received level in the preceding 5 to

Max. {5 , ((5 * N + 6) DIV 7) * BS_PA_MFRMS / 4}

seconds shall be maintained for each carrier in the BCCH allocation. N is the number of non-serving cell BCCH carriers in BA and the parameter BS_PA_MFRMS is defined in GSM 05.02; GSM 05.08, 6.6.1.

2.	The same number of measurement samples shall be taken for all non-serving cell BCCH carriers of the BA list, and the samples allocated to each carrier shall as far as possible be uniformly distributed over each evaluation period.; GSM 05.08, 6.6.1

20.7.3	Test purpose

1.	To verify that if the MS calculates a received level average (over 5 seconds) for a non-serving suitable cell which results in the value of C2 exceeding the value of C2 for the serving cell, then cell reselection takes place to the non-serving cell.

2.	To verify that by using suitable varying levels of signal strength for non serving cells, the MS samples on non serving cell BCCH carriers are as far as possible distributed uniformly over each evaluation period.

20.7.4	Method of test

20.7.4.1	Initial conditions

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBmV emf() / dBm )	40 / -73	33 / -80	OFF	OFF	OFF	OFF

RXLEV_ACCESS_MIN

(dBmV emf() / dBm)	30 / -83	33 / -80

N	5	5

BS_PA_MFRMS	5	5

C1	10	0

C2	10	0

Note :	N=5 and BS_PA_MFRMS=5 such that 5 s is the bigger value and hence the averaging limit

20.7.4.2	Procedure

a)	The MS is switched on

b)	After 30 seconds, the level of Carrier 2 is increased to -50 dBm for 10 seconds and shall then switch between -80 and -50 dBm every 5 seconds.

c)	The SS monitors all RA requests from MS on carriers 1 & 2

20.7.5	Test requirements

1.	The MS accesses on carrier 1 within 30 s.of the completion of the MS boot time as declared by the manufacturer.

2.	After the level of carrier 2 has been increased for the first time, the MS shall access on carrier 2 within 20 seconds

Note :	The access should not take much longer than 10 s (5 s for rxlev average, 5 s for C2 check duration, 2.4 s for sync and BCCH data read in carrier 2). 20 s is a safe  time to wait

3.	After accessing carrier 2 the MS shall not access again on carrier 1

20.8	Running average of the serving cell BCCH carrier signal level

ASK SMG2 IF THIS IS A RUNNING AVERAGE OR NOT??? the present test is not a running average test

20.8.1	Definition and applicability

The MS is required to monitor continuously the BCCH carrier signal level of the serving cell (and to compare it to the BCCH carrier signal levels of the non-serving cells) to guarantee that it is camped on the best suitable cell in terms of signal strength.

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.8.2	Conformance requirement

1.	For the serving cell, receive level measurement samples shall be taken at least for each paging block of the MS. The receive level average shall be a running average determined using samples collected over a period of 5 s or five consecutive paging blocks of that MS, whichever is the greater period. New receiving level average values shall be calculated as often as possible.; GSM 05.08, 6.6.1.

20.8.3	Test purpose

1.	To verify that if the BCCH carrier signal level average value of the serving cell changes so that the value of C2 for a non-serving suitable cell exceeds the value of C2 for the serving cell, the MS makes cell reselection.

20.8.4	Method of test

20.8.4.1	Initial conditions

Two BCCH carriers are established with the system information contents of Table x and Table y.

Parameters changed from Default values Table (table 20.1) are below,

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	58 / -55	48 / -65	OFF	OFF	OFF	OFF

RxLev_Access_Min

(dBmV emf() / dBm)	43 / -70	43 / -70

BS_BA_MFRMS	0	0

Note :	With BS_BA_MFRMS value 0, i.e. the minimum value the parameter can have, the averaging time of the MS becomes 5 s (see conformance requirement 1).

20.8.4.2	Procedure

a)	The MS is switched on

b)	The SS waits the MS to access on carrier 1 within 30 s.

Note :	The C1s of he two carriers are 15 for Carrier 1 and 5 for carrier 2.

c)	The SS starts to change the RF signal level of carrier 1 in 5 s periods as follows: -66 dBm for 2 s, -56 dBm for 3 s, -66 dBm for 2 s, -56 dBm for 3 s, -66 dBm for 2 s, etc.

Note :	In this step the C2 of carrier 1 is 10, and the C2 of carrier 2 is 5.

d)	The SS waits at least 25 s to check the the MS does not make access to carrier 2.

Note :	5 s for rxlev averages, 5 s for calculation of C2, 5 s for C2 check duration and 10 s for possible faulty access (25 s).

e)	The SS starts to change the RF signal level of carrier 1 in 5 s periods as follows: -81 dBm for 2 s, -71 dBm for 3 s, -81 dBm for 2 s, -71 dBm for 3 s, -81 dBm for 2 s, etc.

Note :	In this step the C2 of carrier 1 is -5 and the C2 of carrier 2 is 5.

f)	The SS waits the MS to access on carrier 2 within 25 s.

Note :	The access should not take much longer than 15 s (5 s for rxlev averages, 5 s for calculation of C2, 5 s for C2 to be lower for reselection, 2.4 s for sync and BCCH data read in carrier 2), 25 s is a safe time to wait.

20.8.5	Test requirement

The MS shall calculate correctly BCCH carrier signal level average values for the serving cell in steps d) and f) and act accordingly.

20.9	Updating the list of six strongest neighbour carriers and decoding the BCCH information of a new carrier on the list

20.9.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.9.2	Conformance requirement

1.	The list of the 6 strongest non-serving carriers shall be updated at least as often as duration of the running average defined for measurements on the BCCH allocation  and may be updated more frequently; GSM 05.08, 6.6.1.

2.	When the MS recognizes that a new BCCH carrier has become one of the 6 strongest, the BCCH data shall be decoded for the new carrier within 30 seconds; GSM 05.08, 6.6.1.

20.9.3	Test purpose

1.	To verify that MS meets conformance requirement 1.

2.	To verify that MS meets conformance requirement 2.

20.9.4	Method of test

20.9.4.1	Initial conditions

Six BCCH carriers are established with the system information contents of Table 20.1.

Parameters changed from Default values Table (table 20.1) are below,

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBmV emf() / dBm)	53 / -60	48 / -65	43 / -70	38 / -75	33 / -80	28 / -85

BA ARFCN	8 carriers	8 carriers	8 carriers	8 carriers	8 carriers	8 carriers

on BA list	on BA list	on BA list	on BA list	on BA list	on BA list

BPM	3	3	3	3	3	3

Note :	The combination of 8 carriers on the BA list and BS_PA_MFRMS = 3 gives a value 4.5 from the averaging formula (GSM 05.08 6.6.1), which being smaller than 5 s leads to averaging time of 5 seconds. Hence 5 seconds is also the updating time of the list of six strongest neighbour carriers.

Note :	The C1 (and C2) values of the carriers 1 to 6 are 30, 25, 20, 15, 10 and 5, correspondingly.

20.9.4.2	Procedure

a)	The MS is switched on.

b)	The SS waits the MS to access on carrier 1 (max 30 s)

c)	The SS switches on a seventh BCCH carrier whose signal level is 38 dBmV emf() / -75 dBm, and with the default system information contents except that RXLEV_ACCESS_MIN is -110 dBm.

Note :	The C2 value of carrier 7 is 35 due to the low RXLEV_ACCESS_MIN which is transmitted in the BCCH data.

c)	The SS monitors all RA requests from the MS on all carriers (max 55 s).

20.9.5	Test requirements

The MS shall access on carrier 7 within 55 s of switching carrier 7 on.

Note :	The access should not take much longer than 45 s (5 s to notice new strongest carrier in top 6 (because the updating time for six strongest is 5 s), 30 s to read BCCH, 5 s for calculation of C2, 5s for C2 to be lower for reselection, 2.4 s for sync and BCCH data read in carrier 2), 55 s is a safe time to wait.

20.10	Decoding the serving cell BCCH information

20.10.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.10.2	Conformance requirement

1.	The MS shall attempt to decode the full BCCH data of the serving cell at least every 30 seconds; GSM 05.08, 6.6.1.

Note :	Verification of cell reselection as implicitly tested here is performed in section 20.6

20.10.3	Test purpose

1.	To verify that the MS will decode the full BCCH data of the serving cell at least every 30 seconds  by changing the BCCH data of the serving cell  such that the value of C1 for the serving cell falls below zero and observing that the MS performs cell reselection at least 30 seconds plus the time allowed for cell reselection after the change of the BCCH data.

20.10.4	Method of test

20.10.4.1	Initial conditions

Two BCCH carriers are established with the system information contents of Table 20.1.

Parameters changed from Default values Table (table 20.1) are below,

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	38 / -75	33 / -80	OFF	OFF	OFF	OFF

Note :	The C1 (and C2) values of carriers 1 and 2 are 15 and 10, correspondingly.

20.10.4.2	Procedure

a)	The MS is switched on.

b)	The SS waits the MS to access on carrier 1 (max 30 s)

c)	The SS changes the RXLEV_ACCESS_MIN in the BCCH data of carrier 1 to be -67 dBm.

Note :	With the above change to RXLEV_ACCESS_MIN, the C1 of carrier 1 becomes -8.

d)	The SS monitors all RA requests from the MS on carriers 1 and 2 (max 50 s).

20.10.5	Test requirements

The MS shall access on carrier 2 within 50 s of the change of RXLEV_ACCESS_MIN on carrier 1.

Note :	The access should not take much longer than 40 s (30 s for decode of BCCH, 5 s for calculation of C1, 5 s for C1<0 for reselection, 2.4 s for sync and BCCH data read in carrier 2), 50 s is a safe time to wait.

20.11	Decoding the BCCH information of the neighbour carriers on the list of six strongest neighbour carriers

20.11.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.11.2	Conformance requirement

1.	The MS shall attempt to decode the BCCH data block that contains the parameters affecting cell reselection for each of the 6 strongest non─serving cell BCCH carriers at least every 5 minutes; GSM 05.08, 6.6.1.

Note :	Verification of cell reselection as implicitly tested here is performed in section 20.3

20.11.3	Test purpose

1.	To verify that the MS will decode the BCCH data block that contains the parameters affecting cell reselection for the non─serving cell BCCH carrier, which is in the list of six strongest neighbour cells, least every 5 minutes; by changing the BCCH data such that the value of C2 for that cell exceeds the value of C2 of the serving cell, and observing that the MS performs cell reselection within 5 minutes plus the time allowed for cell reselection after the change of the BCCH data, .

20.11.4	Method of test

20.11.4.1	Initial conditions

Two BCCH carriers are established with the system information contents of Table x and Table y.

Parameters changed from Default values Table (table 20.1) are below,

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	38 / -75	33 / -80	OFF	OFF	OFF	OFF

Note :	The C1 (and C2) values of carriers 1 and 2 are 15 and 10, correspondingly.

20.11.4.2	Procedure

a)	The MS is switched on.

b)	The SS waits for the MS to access on carrier 1 (max 30 s).

c)	The SS changes the RXLEV_ACCESS_MIN in the BCCH data of carrier 2 to be -100 dBm.

Note :	With the above change the C2 of carrier 2 becomes 20 whereas the C2 of carrier 1 stays at 15.

d)	The SS monitors all RA requests from the MS on carriers 1 and 2 (max 320 s).

20.11.5	Test requirements

The MS shall access on carrier 2 within 320 s of the change in the BCCH data of carrier 2.

Note :	The access should not take much longer than 310s (300 s for decode of BCCH of carrier 2, 5 s for calculation of C2, 5s for C2 of carrier 2 to be higher than C2 of carrier 1 for reselection, 2.4 s for sync and BCCH data read in carrier 2), 320 s is a safe time to wait.

20.12	Decoding the BSIC of the neighbour carriers on the list of six strongest neighbour carriers

20.12.1	Definition and applicability

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.12.2	Conformance requirement

1.	The MS shall attempt to check the BSIC for each of the 6 strongest non-serving cell BCCH carriers at least every 30 seconds, to confirm that it is monitoring the same cell. If a change of BSIC is detected then the carrier shall be treated as a new carrier and the BCCH data redetermined; GSM 05.08, 6.6.1.

Note :	Verification of cell reselection as implicitly tested here is performed in section 20.3

20.12.3	Test purpose

1.	To verify that the MS will check  the BSIC  of the non-serving cell, which is in the list of six strongest neighbour cells, by changing the BSIC and the BCCH data of the non-serving cell such that the value of C2 for that cell exceeds the value of C2 of the serving cell, and observing that the MS performs cell reselection within the time allowed to check the BSIC, redetermine the BCCH data and perform cell re-selection.

20.12.4	Method of test

20.12.4.1	Initial conditions

Two BCCH carriers are established with the system information contents of Table 20.1.

Parameters changed from Default values Table (table 20.1) are below,

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	38 / -75	33 / -80	OFF	OFF	OFF	OFF

Note :	The C1 (and C2) values of carriers 1 and 2 are 15 and 10, correspondingly.

20.12.4.2	Procedure

a)	The MS is switched on.

b)	The SS waits for the MS to access on carrier 1 (max 30 s).

c)	The SS changes the BSIC of carrier 2 by changing the Base Station Colour Code (BCC) part of the BSIC. The SS also changes the RXLEV_ACCESS_MIN in the BCCH data of carrier 2 to be -100 dBm.

Note :	It is left open how the BSIC is changed. A possible solution is to inverse the bits of the BCC part of the BSIC. In any case for the purpose of this test the Network Colour code (NCC) part of the BSIC shall not be changed.

Note :	With the above change to the BCCH data the C2 of carrier 2 becomes 20 whereas the C2 of carrier 1 stays at 15.

d)	The SS monitors all RA requests from the MS on carriers 1 and 2 (max 80 s).

20.12.5	Test requirements

The MS shall access on carrier 2 within 80 s of the change in the BSIC value (and BCCH data) of carrier 2.

Note :	The access should not take much longer than 70s (30 s for check of BSIC on carrier 2, 30 s for decode of BCCH of carrier 2, 5 s for calculation of C2, 5s for C2 of carrier 1 to be lower than C2 of carrier 2, 2.4 s for sync and BCCH data read on carrier 2), 80 s is a safe time to wait.

20.13	Emergency calls

20.13.1	Definition and applicability

This test is applicable for GSM 900 and DCS 1800 MSs supporting speech.

20.13.2	Conformance requirement

1.	When in a limited service state and if not camped on a cell, the MS shall monitor the signal strength of all 124 (for GSM), all 174 (for E-GSM) or all 374 (for DCS 1800) RF channels, and search for a BCCH carrier which has C1>0 and which is not barred. When such a carrier is found, the MS shall camp on that cell, irrespective of the PLMN identity.

2.	The MS shall perform cell reselection at least among the cells of the PLMN of the cell on which the MS has camped, according to the algorithm of GSM 03.22, 4.5 & 3.7, except that a zero value of CELL_RESELECT_HYSTERESIS shall be used; GSM 05.08, 6.8.

3.	When in a limited service state, the MS shall be able to initiate emergency calls; GSM 05.08, 6.8.

4.	When no SIM is present in the ME, emergency calls may be made; GSM 05.08 6.8

20.13.3	Test purpose

1	To verify conformance requirement 1, by verifying that conformance requirement 3 is fulfilled in cells where C1 > 0 and CBA = 0 as in test purpose 3 and 4.

2.	To verify that the MS, when performing  cell reselection limited service state ,  uses CELL_RESELECT_HYSTERESIS = 0 by verifying that conformance requirement 3 is fulfilled in cells to which cell re-selection will take place, given CRH = 0 through test purpose 3.

3.	To verify that the MS shall be able to initiate emergency calls when no suitable cells of the selected PLMN are available, but at least one acceptable cell is available

4.	To verify that the MS shall be able to initiate emergency calls without SIM card inserted (No IMSI).

20.13.4	Method of test

20.13.4.1	Initial conditions

Three BCCH carriers are established with the system information contents of Table 20.1.

Parameters changed from Default values Table (table 20.1) are below

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	38 / -75	33 / -80	33 / -80	OFF	OFF	OFF

RxLev_Access_Min

(dBmV emf() / dBm)	23 / -90	43 / -70	23 / -90

CELL_BAR_ACCESS	1 (barred)	0	0

MCC,MNC	forbidden	forbidden	forbidden

CELL_RESELECT_HYST	0	0	14 dB

C1	15	-10	10

Note :	All the BCCH carriers belong to the same PLMN, which is not the MS's home PLMN and is in the SIM's forbidden PLMN's list.

Note :	The MS should select carrier 3 since carrier 1 is barred and carrier 2 has C1<0.

No network selection shall be done during this test.

20.13.4.2	Procedure

a)	The MS is switched on.

b)	After 30 s an emergency call is initiated on the MS.

Note :	30 s for cell selection, 5 s for calculation of C2, 5s for C2 verification.

c)	The SS shall monitor the MS accesses on all carriers (max 10 s).

d)	The SS changes the CBA of carrier 1 to 0.

Note :	The MS should reselecet to carrier 1 because it should not take into account the CELL_RESELECT_HYST value of 14 but use 0 instead.

e)	After 320 s an emergency call is initiated on the MS.

Note :	Change of BCCH data in step d) will not be detected in up to 5 minutes (05.08 6.6.1). The access should not take much longer than 310s (300 s for decode of BCCH of carrier 2, 5 s for calculation of C2, 5s for C2 of carrier 2 to be higher than C2 of carrier 1 for reselection, 2.4 s for sync and BCCH data read in carrier 2), 320 s is a safe time to wait.

f)	The SS shall monitor the MS accesses on all carriers (max 10 s).

g)	The MS is switched off, the SIM is removed and carrier 1 is changed back to barred (CBA = 1).

h)	The MS is switched on.

i)	After 40 s an emergency call is initiated on the MS.

Note :	30 s for cell selection, 5 s for calculation of C2, 5s for C2 verification.

j)	The SS shall monitor the MS accesses on all carriers (max 10 s).

20.13.5	Test requirements

a)	The MS shall respond on carrier 3 in test step c).

b)	The MS shall respond on carrier 1 in test step f).

c)	The MS shall respond on carrier 3 in test step j).

20.14	Cell reselection due to MS rejection "LA not allowed"

20.14.1	Definition and applicability

While camped on a cell of the selected PLMN the MS may need to select a different cell in order to fulfil the normal service state. This ensures that the MS is camped on a cell from which it can reliably decode downlink data and with which it has a high probability of communications on the uplink.

The MS looks for suitable neighbour cells which satisfies 4 constraints including that It should not be in an LA which is in the list of "forbidden LAs for national roaming".

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.14.2	Conformance requirement

1.	In response to a registration attempt, when receiving an LU reject with cause value "LA not allowed", the MS stores this LAI in a list of "forbidden LAIs for regional provision of service", to prevent repeated attempts to access a cell of the forbidden LA, GSM 03.22 3.3.

2.	If the MS has received the cause 'LA not allowed', it shall ignore this fact when selecting a cell to camp on, i.e. it shall not reject a cell for camping on because that cell is part of a LA where this cause has been received, GSM 03.22 3.5.4.

3.	In response to a registration attempt, when receiving an LU reject with cause value "LA not allowed", the MS continues to perform normal cell-reselection, GSM 03.22 4.4.2

4.	A new LU attempt shall only be performed when a new LA (or new PLMN) is entered, GSM 03.22 3.3 & Figure 4.

5.	The list of "forbidden LAIs for regional provision of service"is deleted when the MS is switched off or the SIM is removed.GSM 03.22 3.3.

Note :	LA stands for "Location Area" and LU stands for "Location Update".

20.14.3	Test purpose

1.	To verify that if an LU is rejected with cause "LA not allowed"  that the LAI of that cell is written into a forbidden list which prevents the MS from performing LU  onto another cell in that LA.

2.	To verify that the MS will not reject a cell for camping on because that cell is part of a LA in the list of "forbidden LAIs for regional provision of service". This is verified indirectly ie no access is allowed. A direct verification is performed in section 26.7.4.2.3.

3.	To verify that the MS when receiving an LU reject with cause value "LA not allowed", the MS continues to perform normal cell-reselection:

Cell re-selection is triggered if there is a better cell (in terms of the path loss criterion C2 and the CRH parameter) in another LA of the selected PLMN, GSM 03.22 3.4 & 4.5.

4.	To verify that a new LU attempt will be performed when a new LA (or new PLMN) is entered, GSM 03.22 3.3 & Figure 4.

5.	To verify that if an LAI is in the  list of "forbidden LAIs for regional provision of service" of the MS,  after powering off and then on the MS a LU will be attempted in a LA with this LAI.

20.14.4	Method of test

20.14.4.1	Initial conditions

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBmV emf() / dBm )	63 / -50	53 / -60	63 / -50	53 / -60	OFF	OFF

RXLEV_ACCESS_MIN

(dBmV emf() / dBm)	23 / -90	23 / -90	43 / -70	43 / -70

CRH	0	0	0	0

LAC	H1111	H2222	H1111	H2222

ATT	1	1	1	1

C1	40	30	20	10

C2	40	30	20	10

20.14.4.2	Procedure

a)	The MS is switched on.

b)	The SS monitors all RA requests from MS on carriers 1 & 2

c)	When the MS performs an IMSI attach onto carrier 1, the SS shall reject it with cause "LA not allowed"

d)	The SS shall accept any LU on carrier 2.

e)	10 seconds after completion of the LU procedure on carrier 2, the SS shall reduce the level of carrier 2 to 13 dBmV emf() so that C1<-10.

f)	The SS monitors all RA requests from MS on carriers 1 to 4.

g)	After the MS has successfully location updated onto carrier 4, the MS shall be switched off and on.

h)	The SS monitors all RA requests from MS on carriers 1 to 4 for 30 seconds.

20.14.5	Test requirements

a)	The MS should respond on carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer.

b)	After LU reject, the MS shall then reselect and access onto Carrier 2 within 25 seconds.

c)	After reducing the level of carrier 2, the MS shall reselect and access onto carrier 4 within [25] seconds.

d)	After powering off and on the MS, it shall access onto carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer.

20.15	Downlink signalling failure

20.15.1	Definition and applicability

See conformance requirement.

20.15.2	Conformance requirement

The downlink signalling failure criterion is based on the downlink signalling failure counter DSC.

1.	When the MS camps on a cell, DSC shall be initialized to a value equal to the nearest integer to 90/N where N is the BS_PA_MFRMS parameter for that cell (see GSM 05.02).

2.	Thereafter, whenever the MS attempts to decode a message in its paging subchannel; if a message is successfully decoded DSC is increased by 1,(however never beyond the nearest integer to 90/N).

3.	Whenever the MS can not successfully decode a message in its paging subchannel the  DSC is decreased by 4.

4.	When DSC reaches 0, a downlink signalling failure shall be declared. A downlink signalling failure shall result in cell reselection, GSM 05.08, 6.5.

Note :	The network sends the paging subchannel for a given MS every BS_PA_MFRMS multiframes. The requirement for network transmission on the paging subchannel is specified in GSM 04.08. The MS is required to attempt to decode a message every time its paging subchannel is sent.

20.15.3	Test purpose

1.	To verify that the MS initializes the DSC counter in accordance with the conformance requirement. This is verified indirectly.

2.	To verify that whenever the MS successfully decodes a message on paging subchannel, the DSC is increased by 1, (however never beyond the nearest integer to 90/N). This is verified indirectly.

3	To verify that whenever the MS can not successfully decode a message on paging subchannel, the DSC decreased by 4. This is verified indirectly.

4.	To verify that when the DSC reaches 0, a downlink signalling failure shall be declared and the MS will perform cell reselection.

20.15.4	Method of test

20.15.4.1	Initial conditions

Two BCCH carriers are established with the system information contents of Table 20.1.

Parameters changed from Default values Table (table 20.1) are below

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	43 / -70	33 / -80	OFF	OFF	OFF	OFF

C1 = C2	20	10

Note :	The DSC counter will have a value 18 (90/5).

20.15.4.2	Procedure

a)	The MS is switched on.

b)	After 40 s the SS sends random data in four successive paging blocks to carrier 1 and then reverts to sending normal data.

Note :	Sending random, i.e. non-decodable data on four successive paging blocks should decrease the DSC to 2.

c)	The SS monitors all accesses on both carriers for 30 s.

d)	The SS sends random data in five successive paging blocks to carrier 1 and then reverts to sending normal data.

Note :	Sending random, data on five successive paging blocks should decrease the DSC to -2 and causes a cell reselection.

e)	The SS monitors all accesses on both carriers for 30 s.

20.15.5	Test requirements

a)	There shall be no access to carrier 2 at test step c).

b)	The MS shall access on carrier 2 at test step e).

20.16	Cell selection if no suitable cell found in 10 s

20.16.1	Definition and applicability

See conformance requirement.

20.16.2	Conformance requirement

If no suitable cell is found in cell reselection process within 10 seconds, the cell selection algorithm of GSM 03.22 shall be performed, GSM 05.08; 6.6.2.

20.16.3	Test purpose

1.	To verify that the MS fulfils the conformance requirement

20.16.4	Method of test

20.16.4.1	Initial conditions

One BCCH carrier is established with the system information contents of Table 20.1.

Parameters changed from Default values Table (table 20.1) are below

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF signal level

(dBmV emf() / dBm)	43 / -70	OFF	OFF	OFF	OFF	OFF

C1 = C2	20

20.16.4.2	Procedure

a)	The MS is switched on.

b)	After the SS has received an access on carrier 1 (max 30 s) the SS reduces the transmit level of carrier 1 to 13 dBmV emf() (so that C1 of carrier 1 becomes -10) and turns on a new carrier (carrier 2) at a level of 33 dBmV emf(). Carrier 2 shall not be in the MS BA list (i.e. it shall not be one of the carriers that MS has been monitoring after camped on carrier 1)

c)	The SS shall monitor all accesses on carriers 1 and 2 for 60 s.

Note :	The access on carrier 2 should not take longer than 50 s. (5 seconds to rxlev averages, 5 s for C1<0 duration, 10 s for searching another suitable cell, 30 s for cell selection), 60 s is a safe time to wait,

20.16.5	Test requirements

a)	The MS shall access on carrier 2 at test step c) within 60 s.

20.17	Cell reselection due to MS rejection "National Roaming not allowed in this LA"

20.17.1	Definition and applicability

While camped on a cell of the selected PLMN the MS may need to select a different cell in order to fulfil the normal service state. This ensures that the MS is camped on a cell from which it can reliably decode downlink data and with which it has a high probability of communications on the uplink.

The MS looks for suitable neighbour cells which satisfies 4 constraints including that It should not be in an LA which is in the list of "forbidden LAs for national roaming".

This test is applicable for all types of GSM 900 and DCS 1800 MSs.

20.17.2	Conformance requirement

1.	To prevent repeated attempts to have national roaming service on a not allowed LA, when the MS is informed that an LA is forbidden, the LA is added to a list of "forbidden LAs for national roaming" which is stored in the MS, GSM 03.22; 3.1..

2.	If the MS has received the cause "National Roaming not allowed in this LA", in response to a LU attempt, the Network Selection Procedure of 03.22 4.3.3.1 shall be started, GSM 03.22 4.3.3 L3.

3.	The MS can only perform camping on a suitable cell, which:

-	should not be in an LA which is in the list of "forbidden LAs for national roaming" GSM 03.22 3.2.1.

Note :	LA stands for "Location Area" and LU stands for "Location Update".

20.17.3	Test purpose

1.	To verify that if an LU is rejected with cause  "National Roaming not allowed in this LA", that the LAI of that cell is written into a forbidden list which prevents the MS from camping  onto  any cell in that LA.

2.	To verify conformance requirement 2. This is verified indirectly by test purpose 3, in that the new LA is accessed as part of cell selection, hence CRH is disregarded.

3.	To verify that if an LU is rejected, when attempting LU in a LA with LAI = LAI1, with cause "National Roaming not allowed in this LA" and only cells of the selected PLMN are available, the MS will only camp and attempt LU in any LA with LAI <> LAI1.

20.17.4	Method of test

20.17.4.1	Initial conditions

Parameters changed from Default values Table (table 20.1)

Parameter	Carrier 1	Carrier 2	Carrier 3	Carrier 4	Carrier 5	Carrier 6

RF Signal Level

(dBmV emf() / dBm )	63 / -50	53 / -60	63 / -50	53 / -60	OFF	OFF

RXLEV_ACCESS_MIN

(dBmV emf() / dBm)	23 / -90	23 / -90	43 / -70	43 / -70

MNC	MNC <> HPLMN	MNC <> HPLMN	MNC <> HPLMN	MNC <> HPLMN

MCC	MCC of HPLMN	MCC of HPLMN	MCC of HPLMN	MCC of HPLMN

CRH	0	0	0	0

LAC	H1111	H2222	H1111	H2222

ATT	1	1	1	1

C1	40	30	20	10

C2	40	30	20	10

20.17.4.2	Procedure

a)	The MS is switched on.

b)	The SS monitors all RA requests from MS on carriers 1 & 2

c)	When the MS performs an IMSI attach onto carrier 1, the SS shall reject it with cause "National Roaming not allowed in this LA".

d)	The SS shall accept any LU on carrier 2.

e)	10 seconds after completion of the LU procedure on carrier 2, the SS shall reduce the level of carrier 2 to 13 dBmV emf() so that C1<-10.

f)	The SS monitors all RA requests from MS on carriers 1 to 4.

g)	After the MS has successfully location updated onto carrier 4, the MS shall be switched off and on.

h)	The SS monitors all RA requests from MS on carriers 1 to 4 for 30 seconds.

20.17.5	Test requirements

a)	The MS should respond on carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer.

b)	After LU reject, the MS shall then reselect and access onto Carrier 2 within 25 seconds.

c)	After reducing the level of carrier 2, the MS shall reselect and access onto carrier 4 within [25] seconds.

d)	After powering off and on the MS, it shall access onto carrier 1 within 30 s of the completion of the MS boot time as declared by the manufacturer.