31.	Test of supplementary services

The general aspects of the specification of supplementary services at the layer 3 radio interface are given in GSM 04.10.

The formats and coding are given in GSM 04.80. If the value of a parameter of an uplink message (MS to network) is specified in a test, the implicit meaning is that it has to be checked; if the value is not specified, it is not to be checked unless otherwise stated.

Unless otherwise stated, the MS shall be in the idle updated state at the beginning of each test (including repetition of a test).

In each test, before the MS sends the first REGISTER message, a MM connection is established.

GSM 04.81 to 04.88 give the procedures used at the radio interface for normal operation, registration, erasure, activation, deactivation, invocation and interrogation of supplementary services.

The supplementary services are described in GSM 02.04 and 02.81 to 02.88.

Whenever activation via the standard MMI is mentioned, if the MS doesn't support it but supports a different procedure, this different procedure is used. In the supplementary services tests, only the applicable (see GSM 02.8x series) MMI service code groups (GSM 02.30 Annex 4), of the basic service code in GSM 09.02,which are supported by the MAP, will be used.

In all the following test cases, the MS will not be failed if it doesn't send a FACILITY information element in the initial REGISTER message, but in a subsequent FACILITY message (although the first case is the normal behaviour).

31.1	Number identification supplementary services

(Reserved).

31.2	Call offering supplementary services

31.2.1	Call forwarding supplementary services

Unless stated otherwise, the test procedures described in this section apply to all kinds of call forwarding supplementary services.

The following abbreviations are used:

CFU :	Call Forwarding Unconditional

CFB :	Call Forwarding on mobile subscriber Busy

CFNRy :	Call Forwarding on No Reply

CFNRc :	Call Forwarding on mobile subscriber Not Reachable

CFC :	Call Forwarding Conditional (common name for CFB, CFNRy and CFNRc)

CF :	Call Forwarding (common name for CFU, CFB, CFNRy and CFNRc).

These abbreviations are also used to represent the corresponding SS-Code; e.g. CFC is the SS-Code for all conditional forwarding services.

31.2.1.1	Registration

31.2.1.1.1	Registration accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message containing the following parameters (see GSM 04.82 section 1.2):

-	for "call forwarding unconditional",

"call forwarding on mobile subscriber busy" and

"call forwarding on mobile subscriber not reachable" :

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke =	Registration

Supplementary service code = CFU (or CFB, or CFNRc)

Forwarded to number

Basic service code.

-	for "call forwarding on no reply":

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke =	Registration

Supplementary service code = CFNRy

Forwarded to number

No reply condition time

Basic service code.

Note:	If basic service code is not included the forwarded to number applies to all basic services.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the RegisterSS operation with the following parameters with the same values as above:

-	Forwarded to number

-	No reply condition time	(CFNRy only)

-	Basic service code	(included if included in the invoke).

The invoke ID must be the same as in the invoke of the Register SS operation.

The test is repeated using different values for the SS-Code, forwarded to number, no reply condition time and basic service code and also with the basic service code not included. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS shall transmit successively a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user": (coded as 111), a CM SERVICE REQUEST with CM service type indicating "supplementary service activation" (coded as 1000) and then the REGISTER message containing the invoke of the RegisterSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.2.1.1.2	Registration rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case 31.2.1.1.1.

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE containing all parameter values found in the REGISTER message and additionally the cause for rejection.

The cause could be a return error (for instance "SS not available") or a reject (for instance mis-typed parameter).

RELEASE COMPLETE

Protocol discriminator.

Transaction identifier

Message type

Facility :	return error code

Or facility :	reject code.

For the return error and the reject the invoke ID must be the same as in the invoke of the Register SS operation.

The possible errors are:

-	IllegalSS-Operation

-	SS-ErrorStatus	with parameter SS-Status indicating:

-	Not provisioned, not registered and not active or

-	Provisioned, registered and active

-	SS-NotAvailable

-	SS-SpecificError

-	BearerServiceNotProvisioned

-	TeleserviceNotProvisioned

-	SS-SubscriptionViolation

-	SS-Incompatibility

-	UnexpectedDataValue

-	DataMissing

-	SystemFailure.

The possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

The test is repeated with different errors and problems and, when error is SS-ErrorStatus, with different SS-Status indications.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

31.2.1.2	Erasure by the subscriber

31.2.1.2.1	Erasure accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the facility information element containing an invoke of the EraseSS operation with the following parameters:

Facility invoke = Erasure

-	SS-Code		(CFU, CFB, CFNRy or CFNRc)

-	BasicServiceCode	(Optional).

If basic service code is not included erasure applies to all basic services.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the EraseSS operation with the following parameter with the same value as above:

Facility return result = Erasure

-	BasicServiceCode (included if included in the invoke).

The invoke ID must be the same as in the invoke of the EraseSS operation.

The test is repeated using different values for the SS-Code and BasicServiceCode and also with the BasicServiceCode not included. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the EraseSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.2.1.2.2	Erasure rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case 31.2.1.2.1.

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject, the invoke ID must be the same as in the invoke of the EraseSS operation.

If facility = return error

The possible errors are:

-	IllegalSS-Operation

-	SS-ErrorStatus	with parameter SS-Status indicating:

-	Not provisioned, not registered and not active or

-	Provisioned, not registered and not active

-	SS-SpecificError

-	SS-SubscriptionViolation

-	UnexpectedDataValue

-	DataMissing

-	SystemFailure.

If facility = reject

The possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

The test is repeated with different errors and problems and, when error is SS-ErrorStatus, with different SS-Status indications.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

31.2.1.3	Activation

Activation is not applicable to call forwarding services.

31.2.1.4	Deactivation

Deactivation causes no signalling on the radio path.

31.2.1.5	Invocation

Invocation is not applicable to the MS and causes no signalling on the radio path.

31.2.1.6	Interrogation

31.2.1.6.1	General data request

A)	Interrogation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message containing the supplementary service code for the specific call forwarding service to be tested.

REGISTER message

-	Protocol discriminator

-	Transaction identifier

-	Message type

-	Facility

invoke =	Interrogation

Supplementary service code = CFU or CFB, CFNRy, CFNRc.

As the basic service code is not included, interrogation applies to all basic services.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the InterrogateSS operation with zero, one or more than one triplet(s)/pair(s) of the following parameters:

-	in case of supplementary service code = CFNRy :

FACILITY :

Return Result = Interrogation

ForwardedToNumber	│

NoReplyConditionTime (CFNRy only)	│ 1st basic service

BasicServiceCode	│

Forwarded to number	│

No reply condition time	│ 2nd basic service

Basic service code	│

etc ...

-	in case of supplementary service code = CFU or CFB or CFNRc :

FACILITY :

Return result = Interrogation

Forwarded to number	│ 1st basic service

Basic service code	│

Forwarded to number	│

Basic service code	│ 2nd basic service

Forwarded to number	│

Basic service code	│ 3rd basic service

etc ...

The invoke ID must be the same as in the invoke of the InterrogateSS operation.

The test is repeated using different SS-Codes and different numbers of parameter triplets/pairs with different values. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the InterrogateSS operation with the expected parameter value.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

B)	Interrogation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the facility information element containing a return error or a reject with a problem.

For the return error and the reject, the invoke ID must be the same as in the invoke of the InterrogateSS operation.

If facility = return error

the possible errors are:

-	IllegalSS-Operation

-	SS-NotAvailable

-	SS-SpecificError

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If facility = reject

the possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.2.1.6.2	Specific data request

A)	Interrogation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a specific data request REGISTER containing the SS code for the specific call forwarding service and the basic service about which information is interrogated.

REGISTER message

-	Protocol discriminator

-	transaction identifier

-	message type

-	facility

Invoke =	Interrogation

Supplementary service code = CFU (or CFB or CFNRy, or CFNRc)

Basic service code.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the InterrogateSS operation with the following parameters:

-	in case of supplementary service code = CFNRy :

FACILITY

Return result =	Interrogation

Forwarded to number

No reply condition time

Basic service code.

-	in case of supplementary service code = CFU or CFB or CFNRc :

FACILITY

Return result =	Interrogation

Forwarded to number

Basic service code.

The invoke ID must be the same as in the invoke of the InterrogateSS operation.

The test is repeated using different SS-Codes and BasicServiceCodes. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

1)	For each case tested, the MS shall transmit the REGISTER message containing the invoke of the InterrogateSS operation with the expected parameter values.

2)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

B)	Interrogation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject, the invoke ID must be the same as in the invoke of the InterrorgateSS operation.

If facility = return error

the possible errors are:

-	IllegalSS-Operation

-	SS-NotAvailable

-	SS-SpecificError

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure

If facility = reject

the possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.2.1.7	Normal operation

31.2.1.7.1	Served mobile subscriber side

31.2.1.7.1.1	Notification of incoming call

This subscription option is only applicable to CFB and CFNRy.

Method of test:

a)	The MS is brought to the call state U7 of an incoming call.

b)	The system simulator transmits a FACILITY message with the facility information element containing an invoke of the NotifySS operation with the following parameters:

FACILITY

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke =	notification

Supplementary service code = CFNRy or CFB

SS notification = incoming call is forwarded

(call is forwarded indication to B subscriber).

c)	The MS is brought into the call state U10 of an incoming call.

d)	The system simulator transmits a FACILITY message with the facility information element containing an invoke of the NotifySS operation with the following parameters:

FACILITY

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke =	notification

Supplementary service code = CFB

SS notification = incoming call is forwarded

(call is forwarded indication to B subscriber).

e)	A STATUS ENQUIRY message is sent to the MS.

Requirements:

After steps b) and d), upon receipt of the FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

After step e), the MS shall send a STATUS message with CC-state U10.

31.2.1.7.1.2	Notification during outgoing call

a)	Method of test:

The MS is forced to place an outgoing call.

After having received a SETUP message and sent a CALL PROCEEDING message and after a TCH has been allocated the system simulator transmits an ALERTING message with the facility information element described below and then a STATUS ENQUIRY message

Facility invoke = notification

-	SS-Code (CFU or CFC)

-	SS-Status (indicating:

Provisioned, registered and active).

The test is repeated using different SS-Codes.

Requirements:

Upon receipt of the ALERTING message with the facility information element, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). On receipt of the STATUS ENQUIRY message the MS shall send a STATUS message with CC-state U4.

b)	Method of test

The MS is forced to place an outgoing call.

After having received a SETUP message and sent a CALL PROCEEDING message and after a TCH has been allocated the system simulator transmits an ALERTING message and then a CONNECT with the facility information element described below. After reception of a CONNECT ACKNOWLEDGE message the system simulator sends a STATUS ENQUIRY message.

Facility invoke = notification

-	SS-Code (CFU or CFC)

-	SS-Status (indicating:

Provisioned, registered and active).

The test is repeated using different SS-Codes.

Requirements:

Upon receipt of the CONNECT message with the facility information element, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). On receipt of the CONNECT message and then on receipt of the STATUS ENQUIRY message the MS shall send a STATUS message with CC-state U10.

c)	Method of test

The MS is forced to place an outgoing call.

After having received a SETUP message and sent a CALL PROCEEDING message and after a TCH has been allocated the system simulator transmits a FACILITY message with the facility information element described below. The system simulator then allocates a TCH and after the TCH has been allocated the system simulator sends a STATUS ENQUIRY message.

Facility invoke = notification

-	SS-Code (CFU or CFC)

-	SS-Status (indicating:

Provisioned, registered and active).

The test is repeated using different SS-Codes.

Requirements:

Upon receipt of the FACILITY message with the facility information element, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). After a TCH has been allocated the MS shall, on receipt of the STATUS ENQUIRY message send a STATUS message with CC-state U3.

31.2.1.7.2	Forwarded-to mobile subscriber side

Method of test:

An incoming call is given to the MS with the SETUP message with the facility information element containing an invoke of the NotifySS operation with the following parameters:

Facility invoke = notification

-	SS-Code (CFU, CFB, CFNRy, CFNRc or CF)

-	SS-Notification (indicating: call is forwarded i.e.

Call is forwarded indication to C-subscriber).

The test is repeated using different SS-Codes.

Requirements:

Upon receipt of the SETUP message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.2.1.7.3	Calling mobile subscriber side

Method of test:

A call is initiated by the MS. Upon receipt of the SETUP message the system simulator transmits a FACILITY message with the facility information element containing an invoke of the NotifySS operation with the following parameters:

Facility invoke = notification

-	SS-Code (CFU, CFB, CFNRy, CFNRc or CF)

-	SS-Notification (outgoing call has been forwarded i.e.

Call is forwarded indication to A-subscriber).

The test is repeated using different SS-Codes.

Requirements:

Upon receipt of the FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.2.2	Call transfer and mobile access hunting supplementary services

(Reserved).

31.3	Call completion supplementary services

(Reserved).

31.4	Multi-party supplementary services

(Reserved).

31.5	Community of interest supplementary services

(Reserved).

31.6	Charging supplementary services

The following Advice of Charge abbreviations are used in this section:

AOC		Advice of Charge

AOCC	Advice of Charge Charging

AOCI		Advice of Charge Information

ACM		Accumulated Charge Meter

ACMM	Accumulated Charge Meter Maximum

CCM		Current Call Meter

CAI		Charge Advice Information

CDUR	Chargeable Duration

The following other abbreviations are used:

IE 		Information Element

FIE		Facility Information Element

TCH		Traffic CHannel

General on Advice of Charge.

The purpose of these tests is to verify that the MS under test correctly performs procedures related to the AOC supplementary service.

The reasons for these test purposes are:

-	One example of a possible use for AOCC is in applications where the subscriber hires out a GSM ME and SIM to a user and bills the user according to the charge stored on the SIM at the end of the hire period. If a mobile station claims to support AOCC but does so incorrectly or not at all, this may cause the subscriber to mischarge the user of the hire phone. Hence an MS claiming to support AOCC must be shown to be reliable in that context.

-	Since AOCC offers the use of telecommunication services according to the charge stored independently in the MS, the AOCC service must not be susceptible to fraud at the MS end.

-	To ensure that a mobile station not claiming to support AOCC does not respond with a signal to the network indicating that it does. This could cause the network to allow the call to be placed without any charge being inserted on to the SIM.

Mobile originating and terminating speech AOC calls are tested and if supported, Call Hold and MultiParty calls. The type testing of data calls (i.e. those calls with a volume related charging component) will not be specified at the phase 2 stage but will be deferred to phase 2+ since the interaction of AOC with packet services is not yet defined in GSM 09.06 and GSM 09.07.

Tests are made on the ME-SIM interface to ensure that call charges are being correctly stored in the ACM field of the SIM in several situations. The AOC ACMM function is also tested for incoming and outgoing calls.

Basic service verification tests for the AOCC, Call Hold, and MultiParty supplementary services which have direct relevance to the testing of AOC have been added to GSM 11.10, sections 11.3, 11.4 and 11.5 respectively.

[Editorial Note: Certain error handling tests for AOC will be added to GSM 11.10 section 26.5.]

[Tests on certain SIM functions related to AOC will be added to GSM 11.10 section ?]

31.6.1	Advice of Charge Charging

31.6.1.1	AOC time related charging / MS originated call

Purpose:

1)	To verify that when the MS receives the AOCC parameters in a Facility IE which is contained in the CONNECT message and when a TCH has already been assigned, the MS returns a FACILITY message containing the acknowledgement within 1 second.

2)	To verify that when the MS receives the AOCC parameters in a Facility IE which is contained in a CONNECT message and when a TCH has already been assigned, the MS stores the correct value in the ACM field of the SIM.

3)	To verify that the when the call has no volume related component the MS ignores non-zero AOCC e5, e6 parameters sent to it.

Conformance Requirement(s):

1)	When the MS receives the AOCC parameters in a Facility IE which is contained in a CONNECT message and when a TCH has already been assigned, the MS shall return a FACILITY message containing the acknowledgement within 1 second.

2)	When the MS receives the AOCC parameters in a Facility IE which is contained in a CONNECT or FACILITY message and when a TCH has already been assigned, the MS shall store the correct value in the ACM field of the SIM.

3)	When the call has no volume related component the MS shall ignore non-zero AOC e5 and e6 parameters sent to it.

Reference(s):

Conformance requirement 1:	TS GSM 03.86, 04.13, 04.86.

Conformance requirement 2:	TS GSM 02.24

Conformance requirement 3:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent by the MS before or after the CONNECT ACKNOWLEDGE.

The SS sends the DISCONNECT y seconds after sending the CONNECT message containing the CAI. The MS shall have stored the correct amount on the SIM according to the e-parameters sent. The test is repeated for several different sets of e-parameters as defined below.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,...,5.

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM

1	MS 		The MS is made to initiate a call

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	CALL PROCEEDING

8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	SS -> MS	CONNECT	As default message except contains Facility IE 					with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE

A13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B13	MS -> SS	CONNECT ACKNOWLEDGE

14			call duration y seconds after CAI information 					sent by SS

15	SS -> MS	DISCONNECT

16	MS -> SS	RELEASE

17	SS -> MS	RELEASE COMPLETE

18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

19	MS		SIM contents checked (either via MMI or by 					removing the SIM and using SIM reader). ACM 					shall have been incremented correctly.

Notes:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

y is set to a constant value of 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as below:

k-value	e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

1	6	14	1	25	0	0	60	43	43

2	0	0	1	100	0	0	0	100	143

3	250	16	2	500	0	0	60	2000	2143

4	1	1	1	0	10	10	1	89 or 90	2232 or 2233

5	12.5	30	1	25	10	10	30	50 or 62.5	2294.5,

2295.5,

2282 or 2283

Values shown in table are in the format and have units as in GSM 02.24 section 3.

The ASN.1 description for each e-parameter allows integers in the range 0 to 8191 to be transmitted but some e-parameters have different actual ranges (eg e1 can take any value 0..819.1 with 0.1 resolution). The MS knows how to interpret the received parameter (eg received e1 refers to 10 times actual e1, see GSM 04.80 section 4.4.3). Therefore e1=12.5 would be sent to the MS as 125. The MS knows the value sent is 10 times the 'real' e1 and hence interprets the value as 12.5.

The non-zero e5 and e6 values for the k=4 and k=5 execution of the test are to check that the MS ignores the volume related parameters when carrying out time only related charging.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

31.6.1.2	AOC time related charging / MS terminated call

Purpose:

1)	To verify that when the MS receives certain AOCC e-parameters in a Facility IE which is contained in a FACILITY message sent after the CONNECT message and when a TCH has already been assigned, the MS returns a FACILITY message containing the acknowledgement within 1 second.

2)	To verify that when the MS receives the AOCC parameters in a Facility IE which is contained in a FACILITY message and when a TCH has already been assigned, the MS stores the correct value in the ACM field of the SIM.

Conformance Requirement(s):

1)	When the MS ceives the AOCC parameters in a Facility IE which is contained in a FACILITY message sent after the CONNECT message and when a TCH has already been assigned, the MS shall return a Facility message containing the acknowledgement within 1 second.

2)	When the MS receives the AOCC parameters in a Facility IE which is contained in a FACILITY message and when a TCH has already been assigned, the MS shall store the correct value in the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 03.86, 04.13, 04.86.

Conformance requirement 2:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The SS is made to initiate a call. The call is established and certain AOCC e-parameters are sent to the MS in a Facility IE contained within a FACILITY message. The MS shall return the AOCC acknowledgement within 1 second of the FACILITY message. It is an implementation option whether the AOCC e-parameters and the AOCC acknowledge are sent before or after the CONNECT ACKNOWLEDGE.

The SS sends the DISCONNECT y seconds after sending the FACILITY message. The MS shall have stored the correct amount on the SIM according to the e-parameters sent. The test is repeated for several different sets of e-parameters as defined below.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,...,5.

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM

1	SS 		The SS is made to initiate a call

2	SS -> MS	PAGING REQUEST

3	MS -> SS	CHANNEL REQUEST

4	SS -> MS	IMMEDIATE ASSIGNMENT

5	MS -> SS	PAGING RESPONSE

6	SS -> MS	SETUP

7	MS -> SS	CALL CONFIRMED

8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	MS -> SS	ALERTING

11	MS -> SS	CONNECT

12	SS -> MS	FACILITY	As default message except contains Facility IE 					with contents as indicated in i below

Either A or B branch is taken

A13	SS -> MS	CONNECT ACKNOWLEDGE

A14	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B14	SS -> MS	CONNECT ACKNOWLEDGE

15			call duration y seconds after CAI information 					sent by SS

16	SS -> MS	DISCONNECT

17	MS -> SS	RELEASE

18	SS -> MS	RELEASE COMPLETE

19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

20	MS		SIM contents checked (either via MMI or by 					removing the SIM and using SIM reader) ACM 					shall have been incremented correctly.

Notes:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

y is set to a constant value of 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as below:

k-value	e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

1	0	0	0	0	0	0	0	0	0

2	0	0	1	100	0	0	0	100	100

3	6	14	1	25	0	0	60	43	143

4	1	1	1	0	0	0	1	89 or 90	233 or 232

5	12.5	30	1	25	0	0	30	50 or 62.5	295.5,

294.5,

282 or 283

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

31.6.1.3	AOC volume related charging / MS originated call

Future addition at GSM Phase 2+ stage.

31.6.1.4	AOC volume related charging / MS terminated call

Future addition at GSM Phase 2+ stage.

31.6.1.5	Change in charging information during a call

Purpose:

1)	To verify that when the MS receives new AOCC parameters mid-way through a call in a Facility IE which is contained within a FACILITY message the MS returns a FACILITY message containing the acknowledgement within 1 second.

2)	To verify that when the MS receives new charging information mid-way through a call in the form of a Facility IE contained within a FACILITY message the MS correctly indicates the total charge considering both sets of charging information

Conformance Requirement(s):

1)	When the MS receives new AOCC parameters mid-way through a call in a Facility IE which is contained within a FACILITY message the MS shall return a FACILITY message containing the acknowledgement within 1 second.

2)	When the MS receives new charging information mid-way through a call in the form of a Facility IE contained within a FACILITY message the MS correctly indicates the total charge considering both sets of charging information

Reference(s):

Conformance requirement 1:	TS GSM 03.86, 04.13, 04.86.

Conformance requirement 2:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in a FACILITY message sent after the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of the FACILITY message. x seconds after sending the original CAI, new (and different) e-parameters are sent to the MS in a Facility IE contained within a FACILITY message. The MS shall return the AOCC acknowledge within 1 second of the FACILITY message.

The SS sends the DISCONNECT y seconds after sending the first CAI in the FACILITY message. The MS shall have stored the correct amount on the SIM according to the two sets of e-parameters sent and the times for the two parts of the call

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM

1	MS 		The MS is made to initiate a call

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	CALL PROCEEDING

8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	SS -> MS	CONNECT

Either A,B or C branch is taken

A12	SS -> MS	FACILITY	As default message except contains Facility IE 					with contents as indicated in i below

A13	MS -> SS	CONNECT ACKNOWLEDGE

A14	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B12	MS -> SS	CONNECT ACKNOWLEDGE

B13	SS -> MS	FACILITY	As default message except contains Facility IE 					with contents as indicated in i below

B14	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

C12	SS -> MS	FACILITY	As default message except contains Facility IE 					with contents as indicated in i below

C13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

C14	SS -> MS	CONNECT ACKNOWLEDGE

Second CAI sent x sec after first CAI

15	SS -> MS	FACILITY	As default message except contains Facility IE 					with contents as indicated in iii below

16	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

New CAI held in abeyance until CDUR has 					timed out present e2 value

17			Call duration y seconds after first CAI 						information sent by SS

18	SS -> MS	DISCONNECT

19	MS -> SS	RELEASE

20	SS -> MS	RELEASE COMPLETE

21	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

22	MS		SIM contents checked (either via MMI or by 					removing the SIM and using SIM reader) ACM 					shall have been incremented correctly

Notes:

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x, y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as defined below:

e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

10	28	1	10	0	0	60	(30)	(30)

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

iii) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x is set to a constant value of 80 seconds.

y is set to a constant value of 180 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

10	14	1	5	0	0	60	65	65

Values shown in table are in the format and have units as in GSM 02.24 section 3.

31.6.1.6	Different formats of charging information

Purpose:

1)	To verify that when the MS receives a Facility IE in which certain e-parameters are set to zero the total charge accumulated is the same as that when the same e-parameters are completely omitted from the Facility IE.

2)	To verify the operation of a shortened channel release procedure where the SS does not send DISCONNECT but only the RELEASE COMPLETE and CHANNEL RELEASE messages or just the CHANNEL RELEASE message.

Conformance Requirement(s):

1)	When the MS receives a Facility IE in which certain e-parameters are set to zero the total charge accumulated shall be the same as that when the same e-parameters are completely omitted from the Facility IE.

2)	The channel shall be correctly released when a shortened channel release procedure is used - the SS does not send DISCONNECT but only the RELEASE COMPLETE and CHANNEL RELEASE messages or just the CHANNEL RELEASE message.

Reference(s):

Conformance requirement 1:	TS GSM 02.24

Conformance requirement 2:	TS GSM 04.08

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

Part 1:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in a FACILITY message sent before the CONNECT message.

The SS sends the DISCONNECT y seconds after sending the FACILITY message containing the e-parameters. The MS shall have stored the correct amount on the SIM according to the e-parameters sent.

Part 2:

Part 1 is repeated twice with the e-parameters that were set to zero above now omitted completely from the Facility IE. The shortened release procedures are used. The MS shall have stored the correct amount on the SIM.

The results of parts 1 and 2 are compared. The value for the charge calculated by the MS shall be identical for parts 1 and 2.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,..3

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM

1	MS 		The MS is made to initiate a call

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	CALL PROCEEDING

8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	SS -> MS	FACILITY	As default message except contains Facility IE 					with contents as indicated in i below

Either A,B or C branch is taken

A12	SS -> MS	CONNECT

A13	MS -> SS	CONNECT ACKNOWLEDGE

A14	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B12	SS -> MS	CONNECT

B13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B14	MS -> SS	CONNECT ACKNOWLEDGE

C12	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

C13	SS -> MS	CONNECT

C14	MS -> SS	CONNECT ACKNOWLEDGE

15			call duration y seconds after CAI information 					sent by SS

Branch D,E and F shall be taken for k= 1,2 					and 3 respectively

D16	SS -> MS	DISCONNECT

D17	MS -> SS	RELEASE

D18	SS -> MS	RELEASE COMPLETE

D19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

E16	SS -> MS	RELEASE COMPLETE	Shortened channel release procedure

E17	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

F16	SS -> MS	CHANNEL RELEASE

20	MS		SIM contents checked (either via MMI or by 					removing the SIM and using SIM reader) ACM 					shall have been incremented correctly.

Notes:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

y is set to a constant value of 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value		e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

1	10	40	1	0	0	0	0	20		20

2	10	40	1	---------omitted---------	20		40

3	10	40	1	---------omitted---------	20		60

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

31.6.1.7	AOC on a Call Hold call

Purpose:

1)	To verify that when the MS invokes a Call Hold call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS returns a FACILITY message containing the AOCC acknowledgement within 1 second of transmission of each set of e-parameters.

2)	To verify that when the MS invokes a Call Hold call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM records the sum of all the charges for the services currently being used and hence that the ME inserts the correct charge in the ACM field of the SIM.

Conformance Requirement(s):

1)	When the MS invokes a Call Hold call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS shall return a FACILITY message containing the AOCC acknowledgement within 1 second of receiving each set of e-parameters.

2)	When the MS invokes a Call Hold call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM shall record the sum of all the charges for the services currently being used and hence the ME shall insert the correct charge in the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 04.13

Conformance requirement 2:	TS GSM 02.24, 04.83, 04.84, 04.86.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of transmission of the CONNECT message.

The call (call B) is then put on hold by sending a HOLD message from the MS to the SS. The SS shall reply with a HOLD ACKNOWLEDGE. The traffic channel is now available to originate another call.

The MS is made to initiate a second call (call C). The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement in a FACILITY message within 1 second of transmission of the CONNECT message.

The SS sends the DISCONNECT to the MS for call B x seconds after sending the call B CAI in the CONNECT/FACILITY message and the DISCONNECT for call C y seconds after sending the call C CAI in the CONNECT/FACILITY message. The MS shall have stored the correct amount on the SIM according to the two sets of e-parameters sent and call times x and y.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM

1	MS 		The MS is made to initiate a call

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	CALL PROCEEDING

8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	SS -> MS	CONNECT	As default message except contains Facility IE 					with contents as indicated in i below

Either D or E branch is taken

D12	MS -> SS	CONNECT ACKNOWLEDGE

D13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

E12	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

E13	MS -> SS	CONNECT ACKNOWLEDGE

14	MS -> SS	HOLD	Call B put on hold

15	SS -> MS	HOLD ACKNOWLEDGE

16	MS 		The MS is made to initiate a second call

17	MS -> SS	SETUP	TI arbitrary but different from existing TI

18	SS -> MS	CALL PROCEEDING

19	SS -> MS	ALERTING

20	SS -> MS	CONNECT	As default message except contains Facility IE 					with contents as indicated in i below

Either F or G branch is taken

F21	MS -> SS	CONNECT ACKNOWLEDGE

F22	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

G21	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

G22	MS -> SS	CONNECT ACKNOWLEDGE

23			Call durations x and y seconds after respective 					CAI information sent by SS

Either H or I branch is taken

For call C

H24	SS -> MS	DISCONNECT	y seconds after call C CAI sent

H25	MS -> SS	RELEASE

H26	SS -> MS	RELEASE COMPLETE

H27	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For call B

I24	SS -> MS	DISCONNECT	x seconds after call B CAI sent

I25	MS -> SS	RELEASE

I26	SS -> MS	RELEASE COMPLETE

I27	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

28	MS		SIM contents checked (either via MMI or by 					removing the SIM and using SIM reader) ACM 					shall have been incremented correctly.

Notes:

-	The value of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x set to 180s, y is set to 90s.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

Call		e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

B(time x)	7	40	1	0	0	0	0

C(time y)	13	40	1	0	0	0	0	54	54

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

31.6.1.8	AOC on a MultiParty call

Purpose:

1)	To verify that when the MS invokes a MultiParty call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS returns a FACILITY message containing the AOCC acknowledgement within 1 second of transmission of each set of e-parameters.

2)	To verify that when the MS originates a MultiParty call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM records the sum of all the charges for the services currently being used and hence the ME inserts the correct charge in the ACM field of the SIM.

Conformance Requirement(s):

1)	When the MS invokes a MultiParty call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS shall return a FACILITY message containing the AOCC acknowledgement within 1 second of receiving each set of e-parameters.

2)	When the MS originates a MultiParty call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM shall record the sum of all the charges for the services currently being used and hence the ME shall insert the correct charge in the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 04.13

Conformance requirement 2:	TS GSM 02.24, 04.83, 04.84, 04.86.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of transmission of the CONNECT message.

The call (call B) is then put on hold by sending a HOLD message from the MS to the SS. The SS shall reply with a HOLD ACKNOWLEDGE. The traffic channel is now available to originate another call.

The MS is made to initiate a second call (call C). The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of transmission of the CONNECT message.

The MS invokes the multiparty service by sending a FACILITY message to the SS containing the BuildMPTY request

The SS accepts the request and connects the MS with the other existing connections (active call C and held call B) and confirms with a FACILITY message.

The SS sends the DISCONNECT to the MS for call B x seconds after sending the call B CAI in the CONNECT message and the DISCONNECT for call C y seconds after sending the call C CAI in the CONNECT message. The MS shall have stored the correct amount on the SIM according to the two sets of e-parameters sent and call times x and y.

Maximum Duration of Test:

z minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM

1	MS 		The MS is made to initiate a call

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	CALL PROCEEDING

8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	SS -> MS	CONNECT	As default message except contains Facility IE 					with contents as indicated in i below

Either D or E branch is taken

D12	MS -> SS	CONNECT ACKNOWLEDGE

D13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

E12	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

E13	MS -> SS	CONNECT ACKNOWLEDGE

14	MS -> SS	HOLD	Call B put on hold

15	SS -> MS	HOLD ACKNOWLEDGE

16	MS 		The MS is made to initiate a second call

17	MS -> SS	SETUP	TI arbitrary but different from existing TI

18	SS -> MS	CALL PROCEEDING

19	SS -> MS	ALERTING

20	SS -> MS	CONNECT	As default message except contains Facility IE 					with contents as indicated in i below

Either F or G branch is taken

F21	MS -> SS	CONNECT ACKNOWLEDGE

F22	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

G21	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

G22	MS -> SS	CONNECT ACKNOWLEDGE

The MS is made to build a multiparty call

23	MS -> SS	FACILITY (TI A-B/A-C)	As default message except contains Facility IE 					with contents as indicated in iii below

24	SS -> MS	FACILITY (TI A-B/A-C)	As default message except contains Facility IE 					with contents as indicated in iv, below

25			Call durations x and y seconds after respective

CAI information sent by SS

Either H or I branch is taken

For call C

H26	SS -> MS	DISCONNECT	y seconds after call C CAI sent

H27	MS -> SS	RELEASE

H28	SS -> MS	RELEASE COMPLETE

H29	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For call B

I26	SS -> MS	DISCONNECT	x seconds after call B CAI sent

I27	MS -> SS	RELEASE

I28	SS -> MS	RELEASE COMPLETE

I29	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

30	MS		SIM contents checked (either via MMI or by 					removing the SIM and using SIM reader) ACM 					shall have been incremented correctly.

Notes:

-	A-B/A-C indicates a choice. the transaction identifier (TI) used must be that of the active call or the held call (ref. GSM 04.84).

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x set to 180s, y set to 90s.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

Call		e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

B(time x)	19	40	1	0	0	0	0

C(time y)	29	40	1	0	0	0	0	134		134

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

iii) FACILITY	Information Element with Invoke = BuildMPTY component type as defined in GSM 04.80 section 3.6.1 table 3.4.

The following abbreviations are used in the descriptions below:

U	Universal tag class

CS	Context Specific tag class

P	Primitive tag form

C	Constructed tag form

FIE	Facility Information Element

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00011100

Length of FIE contents	8	00001000

Component type tag 	CS/C/tag=1	10100001

Component length	6	00000110

Invoke ID tag	U/P/tag=2	00000010

Invoke ID length	1	00000001

Invoke ID	Arbitrary 	(00000000)			(1 octet)

Op-Code tag	From GSM 04.80	00000010

Op-Code length	1	00000001

Op-Code	Build MultiParty		00000001

operation						(local value 124)

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00111100

Length of FIE contents	5	00001001

Component type tag 	CS/C/tag=2	10100010

Component length	3	00000011

Invoke ID tag	U/P/Integer	00000010

Invoke ID length	1	00000001

Invoke ID	Same as used as	(00000000)			Invoke ID in Invoke FIE

31.6.2	Charge Storage

31.6.2.1	Removal of SIM during an active call

Purpose:

1)	To verify that when the SIM is removed from the ME during an active AOCC call the ME immediately terminates the call.

2)	To verify that when the SIM is removed during an active AOCC call the ME has written the total charge up to that point in the call to the ACM field of the SIM.

Conformance Requirement(s):

1)	When the SIM is removed from the ME during an active AOCC call the ME shall immediately terminate the call.

2)	When the SIM is removed during an active AOCC call midway through an AOC charging time interval (e7 or e2) the ME shall have written the total charge up to the point in the call where the charging time interval last expired to the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 02.17.

Conformance requirement 1:	TS GSM 02.24.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Removal of the SIM is possible without disconnection of power supply

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

This test is only performed if it is possible to remove SIM without disconnecting the power supply. If the battery pack must be removed to get at the SIM see 31.6.2.2.

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

If possible, without removing the power supply, the SIM is removed from the ME y seconds after the SS sends the CAI in the CONNECT/FACILITY message. The call shall be terminated immediately by the MS and the MS shall have stored the correct amount on the SIM according to the e-parameters sent.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM

1	MS 		The MS is made to initiate a call

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	CALL PROCEEDING

8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	SS -> MS	CONNECT	As default message except contains Facility IE 					with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE

A13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B13	MS -> SS	CONNECT ACKNOWLEDGE

14			SIM removed y seconds after CAI information 					sent by SS.

Either C,D,E or F branch is taken

C15	MS -> SS	DISCONNECT

C16	SS -> MS	RELEASE

C17	MS -> SS	RELEASE COMPLETE

C18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

D15	MS -> SS	RELEASE COMPLETE

D16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

E15	MS -> SS	Layer 2 DISC

E16	SS -> MS	UA

F15			No further messages are sent

19			SIM contents checked (by removing the SIM 					and using SIM reader). ACM shall have been 					incremented correctly.

Notes:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The SIM is removed after approximately y=90 seconds

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

10	55	1	10	0	0	10	30	30

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

31.6.2.2	Interruption of power supply during an active call

Purpose:

To verify that when the power supply of the MS is removed during an active AOCC call the ME has written the total charge up to that point in the call to the ACM field of the SIM.

Conformance Requirement(s):

When the power supply of the MS is removed during an active AOCC call midway through an AOC charging time interval (e7 or e2) the ME shall have written the total charge up to the point in the call where the charging time interval last expired to the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 02.24.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of removing power supply

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

The ME power is switched off by pressing power button on MMI during the active call. The call shall be terminated immediately and the MS shall have stored the correct amount on the SIM according to the e-parameters sent.

The test is repeated for ME power being lost by removal of battery pack. The call shall be terminated immediately and the MS shall have stored the correct amount on the SIM according to the e-parameters sent.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,2

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM

1	MS 		The MS is made to initiate a call

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	CALL PROCEEDING

8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	SS -> MS	CONNECT	As default message except contains Facility IE 					with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE

A13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B13	MS -> SS	CONNECT ACKNOWLEDGE

14			ME power interrupted y seconds after CAI 					information sent by SS by:

MMI power switch (for k=1)

Removing battery pack (for k=2)

Depending on the value of k 							C or D branch is taken

For k=1

C15	MS -> SS	DISCONNECT

C16	SS -> MS	RELEASE

C17	MS -> SS	RELEASE COMPLETE

C18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For k=2

D15	SS -> MS	RELEASE COMPLETE

D16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

19			SIM contents checked (by removing the SIM 					and using SIM reader). ACM shall have been 					incremented correctly.

Notes:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The ME power is removed after approximately y=90 seconds

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

10	55	1	10	0	0	10	30	30

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

31.6.2.3	MS going out of coverage during an active AOCC call

Note:	At present the core recommendations for AOCC in the case where the radio link is lost during an active call are vague. Does the mobile go on charging for a fixed period after radio link loss and continue as usual if radio link reestablishment occurs, or does charging stop? Input from people involved in charging for GSM is required.

Hence this test will be enhanced in the future when the requirements become clearer.

Purpose:

To verify that when the MS goes out of radio coverage area and an active call is dropped the ME has written the total charge up to that point in the call to the ACM field of the SIM.

Conformance Requirement(s):

When the MS goes out of radio coverage area during an active AOCC call midway through an AOC charging time interval (e7 or e2) and the call is dropped the ME shall have written the total charge up to the point in the call where the charging time interval last expired to the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 02.24.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

The cell simulated by the system simulator is then switched off to model the MS losing radio coverage. The MS shall have stored the correct amount on the SIM according to the e-parameters sent. [What happens on call reestablishment?]

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM

1	MS 		The MS is made to initiate a call

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	CALL PROCEEDING

8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	SS -> MS	CONNECT	As default message except contains Facility IE 					with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE

A13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B13	MS -> SS	CONNECT ACKNOWLEDGE

14			SS cell switched off y seconds after CAI 					information sent by SS

Failure of radio path, end of call (CEND) occurs 				and MS stops charging 								(ref. GSM 02.24 sec 2)

15			SIM contents checked (either via MMI or by 					removing the SIM and using SIM reader). ACM 					shall have been incremented correctly.

Notes:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The cell being simulated by the SS is switched off after approximately y=90 seconds

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value		e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

10	55	1	10	0	0	10	30	30

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

31.6.2.4	ACMM operation / Mobile Originating

Purpose:

1)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMM, any outgoing calls in progress for which a non-zero CAI exists are terminated by the ME with an appropriate indication given to the user.

2)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMM, the making of further calls that have a non-zero CAI is inhibited

3)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMM, the making of calls for which a zero CAI exists is uninhibited.

4)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMM, the making of emergency calls is uninhibited.

Conformance Requirement(s):

1)	When the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMM, any outgoing calls in progress for which a non-zero CAI exists shall be terminated by the ME with an appropriate indication given to the user.

2)	When the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMM, the making of further calls that have a non-zero CAI shall be inhibited

3)	When the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMM, the making of calls for which a zero CAI exists shall be uninhibited.

4)	When the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMM, the making of emergency calls shall be uninhibited.

Reference(s):

Conformance requirement 1:	TS GSM 02:24

Conformance requirement 2:	TS GSM 02.24

Conformance requirement 3:	TS GSM 02.24

Conformance requirement 4:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Type of user indication when ACMM exceeded

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

PIN 2 is entered into the MS allowing modification of both the ACM and ACMM fields on the SIM. The ACM is reset to zero and the ACMM is set to 2 units.

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC non-zero e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

After the ACM has been incremented to 2 units the call shall be terminated by the MS and an indication given to the user. The call duration is recorded and the ACM is checked to ensure it has been incremented to 2 units.

The MS is then made to attempt to originate an ordinary call to the MS for which a non-zero CAI exists for the calling party and shall be unsuccessful. The MS shall terminate the call with a DISCONNECT message.

The MS is then made to attempt to originate an ordinary call to the MS for which a zero CAI exists for the calling party and shall be successful.

The MS is then made to attempt to originate an emergency call and shall be successful.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,...,4.

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM (it shall be zero)

Depending on the value of k 							A or B branch is taken

For k= 1,2,3

A1	MS 		The MS is made to initiate an ordinary call

A2	MS -> SS	CHANNEL REQUEST

A3	SS -> MS	IMMEDIATE ASSIGNMENT

A4	MS -> SS	CM SERVICE REQUEST

A5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	SETUP

A7	SS -> MS	CALL PROCEEDING

A8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

A9	MS -> SS	ASSIGNMENT COMPLETE

A10	SS -> MS	ALERTING

For k=4

B1	MS 		The MS is made to initiate an emergency call 					(112 is entered)

B2	MS -> SS	CHANNEL REQUEST	Establishment cause is emergency call 						establishment

B3	SS -> MS	IMMEDIATE ASSIGNMENT

B4	MS -> SS	CM SERVICE REQUEST	CM service type IE indicates "emergency call 					establishment"

B5	SS -> MS	CM SERVICE ACCEPT

B6	MS -> SS	EMERGENCY SETUP

B7	SS -> MS	CALL PROCEEDING

B8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

B9	MS -> SS	ASSIGNMENT COMPLETE

B10	SS -> MS	ALERTING

11	SS -> MS	CONNECT	As default message except contains Facility IE 					with contents as indicated in i below

For k=1 or 3 either branch C or D is taken

C12	MS -> SS	CONNECT ACKNOWLEDGE

C13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

D12	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

D13	MS -> SS	CONNECT ACKNOWLEDGE

For k=2 either branch E,F,G,H,I or J is taken

E13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

E14	MS -> SS	DISCONNECT	MS terminates call

F13	MS -> SS	DISCONNECT	MS terminates call

F14	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

G13	MS -> SS	DISCONNECT	MS terminates call

DISCONNECT may or may not contain Facility 					IE with contents as indicated in ii below

H13	MS -> SS	CONNECT ACKNOWLEDGE

H14	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

H15	MS -> SS	DISCONNECT	MS terminates call

I13	MS -> SS	CONNECT ACKNOWLEDGE

I14	MS -> SS	DISCONNECT	MS terminates call

I15	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

J13	MS -> SS	CONNECT ACKNOWLEDGE

J13	MS -> SS	DISCONNECT	MS terminates call.

DISCONNECT may or may not contain Facility 					IE with contents as indicated in ii below

Depending on the value of k 							K,L or M branch is taken

For k=2

K15	SS -> MS	RELEASE

K16	MS -> SS	RELEASE COMPLETE	RELEASE COMPLETE may or may not contain 					Facility IE with contents as indicated in ii 					below

K17	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For k=1

L14			Record call duration, x seconds, after CAI 					information sent by SS until call is terminated 					(when ACM=ACMM)

L15	MS -> SS	DISCONNECT

L16	SS -> MS	RELEASE

L17	MS -> SS	RELEASE COMPLETE

L18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

L19			ACM checked (shall be 2 units)

For k=3 or 4

M14			Call duration y seconds after CAI information 					sent by SS

M15	MS -> SS	DISCONNECT

M16	SS -> MS	RELEASE

M17	MS -> SS	RELEASE COMPLETE

M18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

M19			ACM checked (shall be 2 units)

k=1 - Non zero CAI call attempted by MS and should succeed

k=2 - Non zero CAI call attempted by MS and should fail

k=3 - Zero CAI call attempted by MS and should succeed

k=4 - Emergency call attempted by MS and should succeed

Notes:

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x shall be 60 ± 2 seconds

y shall be set to 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

1	1	30	1	0	0	0	0	2	2

2	1	30	1	0	0	0	0	0	2

3	0	0	0	0	0	0	0	0	2

4	0	0	0	0	0	0	0	0	2

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

31.6.2.5	ACMM operation / Mobile Terminating

Purpose:

1)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMM, any mobile terminating calls in progress for which a non-zero CAI exists are terminated by the ME with an appropriate indication given to the user.

2)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMM, and an incoming call is received for which subsequently a non-zero CAI is received, then the call is terminated by the ME with an appropriate indication given to the user.

3)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMM, the receiving of calls for which the CAI is zero is uninhibited.

Conformance Requirement(s):

1)	When the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMM, any mobile terminating calls in progress for which a non-zero CAI exists shall be terminated by the ME with an appropriate indication given to the user.

2)	When the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMM, and an incoming call is received for which subsequently a non-zero CAI is received, the call shall be terminated by the ME with an appropriate indication given to the user.

3)	When the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMM, the receiving of calls for which the CAI is zero shall be uninhibited.

Reference(s):

Conformance requirement 1:	TS GSM 02:24.

Conformance requirement 2:	TS GSM 02.24

Conformance requirement 2:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Type of user indication when ACMM exceeded

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

PIN 2 is entered into the MS allowing modification of both the ACM and ACMM fields on the SIM. The ACM is reset to zero and the ACMM is set to 2 units.

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The SS is made to initiate a call. The call is established with certain non-zero AOCC e-parameters sent in a Facility IE contained in a FACILITY message sent before the CONNECT message. It is an implementation option whether the AOCC e-parameters and AOCC acknowledge are sent before or after the CONNECT ACKNOWLEDGE.

After the ACM has been incremented to 2 units the call shall be terminated by the MS and an indication given to the user. The ACM shall be checked to ensure that it has been incremented to 2 units.

The SS is then made to attempt to make an ordinary call to the MS for which a non-zero CAI exists for the called party and shall be unsuccessful. The MS shall terminate the call with a DISCONNECT message.

The SS is then made to attempt to make an ordinary call to the MS for which a zero CAI exists for the called party and shall be successful.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,...,3.

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM

1	SS 		The SS is made to initiate a call

2	SS -> MS	PAGING REQUEST

3	MS -> SS	CHANNEL REQUEST

4	SS -> MS	IMMEDIATE ASSIGNMENT

5	SS -> MS	PAGING RESPONSE

6	MS -> SS	SETUP

7	SS -> MS	CALL CONFIRMED

8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type

9	MS -> SS	ASSIGNMENT COMPLETE

10	SS -> MS	ALERTING

11	MS -> SS	CONNECT

12	SS -> MS	CONNECT ACKNOWLEDGE

For k=1 or 3 branch A is taken

A13	SS -> MS	FACILITY	As default message except contains Facility IE 					with contents as indicated in i below

A14	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

For k=2 either branch B,C or D is taken

B13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

B14	MS -> SS	DISCONNECT	MS terminates call

C13	MS -> SS	DISCONNECT	MS terminates call

C14	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

D13	MS -> SS	DISCONNECT	MS terminates call.

DISCONNECT may or may not contain Facility 					IE with contents as indicated in ii below

For k= 1, 2 and 3, D, E and F branch shall be 					taken respectively

For k=1

D15			Record call duration, x seconds, after CAI 					information sent by SS until call is terminated 					(when ACM=ACMM)

D16	MS -> SS	DISCONNECT	MS terminates call

D17	SS -> MS	RELEASE

D18	MS -> SS	RELEASE COMPLETE

D19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For k=2

E15	SS -> MS	RELEASE

E16	MS -> SS	RELEASE COMPLETE	RELEASE COMPLETE may or may not contain 					Facility IE with contents as indicated in ii 					below

E17	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For k=3

F15			Call duration y seconds after CAI information 					sent by SS

F16	MS -> SS	DISCONNECT	MS terminates call

F17	SS -> MS	RELEASE

F18	MS -> SS	RELEASE COMPLETE

F19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

20			ACM checked (should be 2 units)

k=1 - Non zero CAI call attempted to MS and should succeed

k=2 - Non zero CAI call attempted to MS and should fail

k=3 - Zero CAI call attempted to MS and should succeed

Notes:

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x shall be 60 ± 2 seconds

y shall be set to 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM

at call end	value 								increased by

1	2	3	4	5	6	7

1	1	30	1	0	0	0	0	2	2

2	1	30	1	0	0	0	0	0	2

3	0	0	0	0	0	0	0	0	2

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

31.6.3	Advice of Charge Information

[Text to be added once AOCC tests are completed.]

31.6.4	Default contents of messages

As default message contents in GSM 11.10 Section 26.8 "Tests related to circuit switched call control". These messages shall not contain SS version IEs.

Where indicated in specific tests CONNECT and FACILITY messages have Facility Information Elements as defined below.

Default contents of ForwardChargeAdvice Facility Information Elements

The following abbreviations are used in the descriptions below:

U	Universal tag class

CS	Context Specific tag class

P	Primitive tag form

C	Constructed tag form

FIE	Facility Information Element

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00011100

Length of FIE contents	43	00101011

Component type tag 	CS/C/tag=1	10100001

Component length	41	00101001

Invoke ID tag	U/P/tag=2	00000010

Invoke ID length	1	00000001

Invoke ID	Arbitrary 	(00000000)			(1 octet)

Op-Code tag	From GSM 04.80	00000010

Op-Code length	1	00000001

Op-Code	Forward Charge	01111101

Advice operation 					(local value 125)

Sequence Identifier	U/C/tag=16 	00110000			from CCITT X.208

Length Indicator	33	00100001

SS-Code Identifier	CS/P/tag=0	10000000

Length Indicator	1	00000001

SS-Code	AOCC SS-Code	01110010

Charging Info. identifier	CS/C/tag=1	10100001

Length Indicator	28	00011100

e1 Identifier	CS/P/tag=1	10000001

Length Indicator	2	00000010

e1	(2 Octets)	See e-parameter 			table in relevant test

e2 Identifier	CS/P/tag=2	10000010

Length Indicator	2	00000010

e2	(2 Octets)	See e-parameter 			table in relevant test

.	.	.

.	.	.

.	.	.

e7 Identifier	CS/P/tag=7	10000111

Length Indicator	2	00000010

e7	(2 Octets)	See e-parameter 			table in relevant test

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00111100

Length of FIE contents	5	00001001

Component type tag 	CS/C/tag=2	10100010

Component length	3	00000011

Invoke ID tag	U/P/Integer	00000010

Invoke ID length	1	00000001

Invoke ID	Same as used as	(00000000)			Invoke ID in Invoke FIE

31.7	Additional information transfer supplementary services

(Reserved).

31.8	Call restriction supplementary services

Unless stated otherwise, the test procedures in the following apply to all kinds of call restriction supplementary services.

The following abbreviations are used

BO	:	Barring of Outgoing calls

BAOC	:	Barring of All Outgoing Calls

BOIC	:	Barring of Outgoing International Calls

BOIC-exHC	:	Barring of Outgoing International Call EXcept those directed to the Home PLMN country

BI	:	Barring of Incoming calls

BAIC	:	Barring of All Incoming calls

BAIC-Roam	:	Barring of incoming when Roaming Outside the home PLMN country

B	:	Barring (common name for BAOC, BOIC, BOIC-exHC, BAIC and BAIC-Roam).

These abbreviations are also used to represent the corresponding SS-Code; e.g. B is the SS-Code for all call restriction (barring) services.

Note:	The password(s) to be used during tests of this section 31.8 may be randomly chosen - unless otherwise stated - in accordance with GSM 02.04 section 5.2.

31.8.1	Registration

This section allows the registration protocol for password registration to be tested. Both successful and unsuccessful outcomes are tested.

31.8.1.1	Registration accepted

General

The following steps shall be executed sequentially.

In the case of brand specific MMI, actions might have to be performed at the MS during the test (eg. entering new password).

Method of test, Step 1:

By means of either the standardized MMI functions as described in GSM 02.30 or the brand specific MMI, the MS is forced to send a REGISTER message with the facility information element containing an invoke of the RegisterPassword operation with the following parameter:

Facility invoke = Register password

-	SS-Code (B).

Upon receipt of the REGISTER message, the system simulator answers with the FACILITY message with the facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility invoke = Get password

-	GuidanceInfo	(indicating: enter password).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Requirement:

The MS shall transmit the REGISTER message with the expected parameter value.

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 2:

Upon receipt of the FACILITY message, the MS shall transmit a FACILITY message with the facility information element containing a return result of the GetPassword operation with the following parameter:

Facility return result = Get password

-	Password	(password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility invoke = Get password

GuidanceInfo	(indicating: Enter new password).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Requirements:

The MS shall transmit the FACILITY message with the expected parameter value.

Upon receipt of the FACILITY message the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 3:

Unless the standardized MMI functions of GSM 02.30 have been used, the new password is entered in the MS by means of procedures which are to be described by the manufacturer. The MS shall transmit a FACILITY message with the facility information element containing a return result of the GetPassword operation with the following parameter:

Facility return result

-	Password		(the new password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility invoke = Get password

-	GuidanceInfo	(indicating: Enter new password again).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Requirements:

The MS shall transmit the FACILITY message with the expected parameter value.

Upon receipt of the FACILITY message the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 4:

Unless the standardized MMI functions of GSM 02.30 have been used, the new password is entered again in the MS by means of procedures which are to be described by the manufacturer. The MS shall transmit a FACILITY message with the facility information element containing a return result of the GetPassword operation with the following parameter:

Facility return result = Get password

-	Password	(the new password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE message with the facility information element containing a return result of the RegisterPassword operation with the following parameter:

-	Password	(the new password).

The invoke ID must be the same as in the invoke of the RegisterPassword operation.

Requirements:

The MS shall transmit the FACILITY message with the expected parameter value.

Upon receipt of the RELEASE COMPLETE message the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.1.2	Registration rejected

31.8.1.2.1	Rejection after invoke of the RegisterPassword operation

Method of test:

The MS is forced to send a REGISTER message in the same way as described for case 31.8.1.1, Step 1.

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem. The test is repeated with all different errors and problems described below.

For the return error or the reject the invoke ID must be the same as in the invoke of the RegisterPassword operation.

If facility = return error

the possible errors are:

-	SS-SubscriptionViolation

-	UnexpectedDataValue

-	SystemFailure.

If facility = reject

the possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.1.2.2	Rejection after password check with negative result

Method of test:

The MS is forced to send a REGISTER message in the same way as described for case 31.8.1.1, Step 1, by using the standardized MMI as described in GSM 02.31.

Upon receipt of the REGISTER message, the system simulator answers with the FACILITY message with the facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility invoke = get password

-	GuidanceInfo	(indicating: enter password).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Upon receipt of the FACILITY message from the MS with the facility information element containing a return result of the GetPassword operation, the system simulator answers with the RELEASE COMPLETE message with the facility information element containing a return error with the following error:

Facility return error

-	NegativePasswordCheck.

The invoke ID must be the same as in the invoke of the RegisterPassword operation.

Requirements:

Upon receipt of the RELEASE COMPLETE message the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.1.2.3	Rejection after use of two different new passwords

Method of test:

The MS is forced to send a REGISTER message in the same way as described for case 31.8.1.1, Step 1. However, the new password and its repetition shall be different.

Upon receipt of the REGISTER message, the system simulator answers with the FACILITY message with the facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility invoke = Get password

-	GuidanceInfo	(indicating: Enter password).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues without interruption.

Upon receipt of the FACILITY message the MS shall transmit a FACILITY message with the facility information element containing a return result of the GetPassword operation with the following parameter:

Facility return result = Get password

-	Password	(the current password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility invoke = Get password

-	GuidanceInfo	(indicating: Enter new password).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues without interruption.

Upon receipt of the FACILITY message the MS shall transmit a FACILITY message with the facility information element containing a return result of the GetPassword operation with the following parameter:

Facility return result = Get password

-	Password	(the new password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility invoke = Get password

-	GuidanceInfo	(indicating: Enter new password again).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues without interruption.

Upon the receipt of the FACILITY message the MS shall transmit a FACILITY message with the facility information element containing a return result of the GetPassword operation with the following parameter:

Facility return result = Get password

-	Password	(a different new password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE message with the facility information element containing a return error with the following error:

Facility return error

-	PasswordRegistrationFailure.

This error contains the following parameter:

-	Diagnostic	(indicating: NewPasswordMismatch).

The invoke ID must be the same as in the invoke of the RegisterPassword operation.

Requirements:

Upon receipt of the RELEASE COMPLETE message the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.2	Erasure

Erasure causes no signalling on the radio path.

31.8.3	Activation

31.8.3.1 	Activation accepted

General

The following steps shall be executed sequentially.

Brand specific MMI functions which are described by the manufacturer and applicable to test Step 1 (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating test Step 1 while using these MMI functions.

This whole test case 31.8.3.1 shall be repeated using different values for the SS-Code and BasicServiceCode and also with the BasicServiceCode not included. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY. Further repetitions of test Step 1 for brand specific MMI functions are not required.

Method of test, Step 1:

By means of either the standardized MMI functions as described in GSM 02.30, or the brand specific MMI the MS is forced to send a REGISTER message with the facility information element containing an invoke of the ActivateSS operation with the following parameters:

REGISTER

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Invoke =	Activate SS

Supplementary service code (eg BAOC, BOIC, BOIC-exHC, BAIC,

BAIC-Roam)

Basic service code.

Note:	If basic service code is not included the supplementary service applies to all basic services.

Upon receipt of the REGISTER message, the system simulator answers with the FACILITY message with the facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility invoke = Get password

-	GuidanceInfo	(indicating: Enter password).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 2:

Unless the standardized MMI functions of GSM 02.30 have been used, the current password is entered in the MS by means of appropriate MMI functions (which are to be described by the manufacturer). The MS shall transmit a FACILITY message with the facility information element containing a return result of the GetPassword operation with the following parameter:

Facility return result = Get password

-	Password	(the current password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the ActivateSS operation with the following parameter with the same value as above:

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

- Return result:	Activate SS

Basic service code

(Included if included in the invoke).

The invoke ID must be the same as in the invoke of the ActivateSS operation.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the ActivateSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.3.2	Activation rejected

31.8.3.2.1	Rejection after invoke of ActivateSS operation

General

Brand specific MMI functions which are described by the manufacturer and applicable to the procedure under test (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating the test while using these MMI functions.

This test case 31.8.3.2.1 shall be repeated with different error and problem indications of the RELEASE COMPLETE message. Further repetitions of the test for brand specific MMI functions are not required.

Method of test:

By means of either the standardized MMI functions as described in GSM 02.30, or the brand specific MMI the MS is forced to send a REGISTER message with the facility information element containing an invoke of the ActivateSS operation with the following parameters:

Facility invoke = Activate SS

-	SS-Code	(BAOC, BOIC, BOIC-exHC, BAIC or BAIC-Roam)

-	BasicServiceCode	(Optional).

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error or the reject the invoke ID must be the same as in the invoke of the ActivateSS operation.

If facility = Return error

the possible errors are:

-	IllegalSS-Operation

-	SS-ErrorStatus	with parameter SS-Status indicating:

-	Not provisioned, not registered and not active or

-	Provisioned, not registered and not active or

-	Provisioned, registered and not active)

-	SS-NotAvailable

-	SS-SpecificError

-	BearerServiceNotProvisioned

-	TeleserviceNotProvisioned

-	SS-SubscriptionViolation

-	SS-Incompatibility

-	UnexpectedDataValue

-	DataMissing

-	SystemFailure.

If facility = Reject

the possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.3.2.2	Rejection after use of password procedure

General

The following steps shall be executed sequentially.

Brand specific MMI functions which are described by the manufacturer and applicable to test Step 1 (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating test Step 1 while using these MMI functions.

This test case 31.8.3.2.2 shall be repeated with different error and problem indications of the RELEASE COMPLETE message. Further repetitions of test Step 1 for brand specific MMI functions are not required.

Method of test, Step 1:

By means of either the standardized MMI functions as described in GSM 02.30, or the brand specific MMI the MS is forced to send a REGISTER message with the facility information element containing an invoke of the ActivateSS operation with the following parameters:

Facility invoke = Activate SS

-	SS-Code	(BAOC, BOIC, BOIC-exHC, BAIC or BAIC-Roam)

-	BasicServiceCode	(Optional).

Upon receipt of the REGISTER message, the system simulator answers with the FACILITY message with the facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility invoke = Get password

-	GuidanceInfo	(indicating: Enter password).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 2:

Unless the standardized MMI functions of GSM 02.30 have been used, the current password is entered in the MS by means of appropriate MMI functions (which are to be described by the manufacturer). The MS shall transmit a FACILITY message with the facility information element containing a return result of the GetPassword operation with the following parameter:

Facility return result = Get password

-	Password	(the current password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject, the invoke ID must be the same as in the invoke of the ActivateSS operation.

If facility = Return error

the possible errors are:

-	NegativePasswordCheck

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If facility = Reject

the possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Return result problems:

-	Unrecognized invoke ID

-	Mis-typed parameter.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the ActivateSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.4	Deactivation

31.8.4.1	Deactivation accepted

General

The following steps shall be executed sequentially.

Brand specific MMI functions which are described by the manufacturer and applicable to test Step 1 (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating test Step 1 while using these MMI functions.

This test case 31.8.4.1 shall be repeated using the 2 different values for the SS-Code and BasicServiceCode and also with the BasicServiceCode not included. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY. Further repetitions of test Step 1 for brand specific MMI functions are not required.

Method of test, Step 1:

By means of either the standardized MMI functions as described in GSM 02.30, or the brand specific MMI, the MS is forced to send a REGISTER message with the facility information element containing an invoke of the DeactivateSS operation with the following parameters:

REGISTER

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Invoke =	Deactivate SS

Supplementary service code (BO or BI)

Basic service code.

Note:	If Basic service code is not included the supplementary service applies to all basic services.

Upon receipt of the REGISTER message, the system simulator answers with the FACILITY message with the facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility invoke = Get password

-	GuidanceInfo	(indicating: Enter password).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 2:

Unless the standardized MMI functions of GSM 02.30 have been used, the current password is entered in the MS by means of appropriate MMI functions (which are to be described by the manufacturer). The MS shall transmit a FACILITY message with the facility information element containing a return result of the GetPassword operation with the following parameter:

Facility return result = Get password

-	Password	(the current password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the DeactivateSS operation with the following parameter with the same value as above:

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Return result = Deactivation

Basic service code (included if included in the invoke).

The invoke ID must be the same as in the invoke of the DeactivateSS operation.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the DeactivateSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.4.2	Deactivation rejected

31.8.4.2.1	Rejection after invoke of DeactivateSS operation

General

Brand specific MMI functions which are described by the manufacturer and applicable to the procedure under test (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating the test while using these MMI functions.

This test case 31.8.4.2.1 shall be repeated with different error and problem indications of the RELEASE COMPLETE message. Further repetitions of the test for brand specific MMI functions are not required.

Method of test:

By means of either the standardized MMI functions as described in GSM 02.30, or the brand specific MMI, the MS is forced to send a REGISTER message with the facility information element containing an invoke of the DeactivateSS operation with the following parameters:

Facility invoke = Deactivate SS

-	SS-Code (BO or BI)

-	BasicServiceCode (Optional).

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the DeactivateSS operation.

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Return error	= error code

Or reject	= problem code.

The possible errors are:

-	IllegalSS-Operation

-	SS-ErrorStatus	with parameter SS-Status indicating:

-	Not provisioned, not registered and not active or

-	Provisioned, not registered and not active or

-	Provisioned, registered and active

-	SS-NotAvailable

-	SS-SpecificError

-	BearerServiceNotProvisioned

-	TeleserviceNotProvisioned

-	SS-SubscriptionViolation

-	SS-Incompatibility

-	UnexpectedDataValue

-	DataMissing

-	SystemFailure.

The possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.4.2.2	Rejection after use of password procedure

General

The following steps shall be executed sequentially.

However, if the MS returns to the start of this test case 31.8.4.2.2 by sending the REGISTER message containing:

Facility invoke = Deactivate SS

then the system simulator shall repeat this test case 31.8.4.2.2.

Brand specific MMI functions which are described by the manufacturer and applicable to test Step 1 (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating test Step 1 while using these MMI functions.

This test case 31.8.4.2.2 shall be repeated with different error and problem indications of the RELEASE COMPLETE message. Further repetitions of the test for brand specific MMI functions are not required.

Method of test, Step 1:

By means of standardized MMI functions as described in GSM 02.30, including any password here, (and repeated for additional procedures which are to be described by the manufacturer) the MS is forced to send a REGISTER message with the facility information element containing an invoke of the DeactivateSS operation with the following parameters:

Facility invoke = Deactivate SS

-	SS-Code (BO or BI)

-	BasicServiceCode (Optional).

Upon receipt of the REGISTER message, the system simulator answers with the FACILITY message with the facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility invoke = Get password

-	GuidanceInfo	(indicating: Enter password).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 2:

Unless the standardized MMI functions of GSM 02.30 have been used, the current password is entered in the MS by means of appropriate MMI functions (which are to be described by the manufacturer). The MS shall transmit a FACILITY message with the facility information element containing a return result of the GetPassword operation with the following parameter:

Facility return result = Get password

-	Password	(the current password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject, the invoke ID must be the same as in the invoke of the DeactivateSS operation.

If facility = Return error

the possible errors are:

-	NegativePasswordCheck

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If facility = Reject

the possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Return result problems:

-	Unrecognized invoke ID

-	Mis-typed parameter.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the DeactivateSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.5	Invocation

Invocation causes no signalling on the radio path.

31.8.6	Interrogation

31.8.6.1	Interrogation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the facility information element containing an invoke of the InterrogateSS operation with the following parameter:

REGISTER

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke :	interrogation

SS code (BAOC, BOIC, BOIC-exHC, BAIC or BAIC-Roam).

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the InterrogateSS operation with zero, one or more than one number of the following parameters:

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Return result =	Interrogation

Basic service code n°1

Basic service code n°2

etc.

The invoke ID must be the same as in the invoke of the InterrogateSS operation.

The test is repeated using different values of the SS-Code and different numbers and values of the BasicServiceCode. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the InterrogateSS operation with the expected parameter value.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.6.2	Interrogation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case 31.8.6.1.

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the InterrogateSS operation. For the reject the invoke ID may be the same as in the invoke of the InterrogateSS operation but the invoke ID tag can also be universal null together with the invoke ID length = 0.

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Return error =	Error code

Or reject = problem code.

The possible errors are:

-	IllegalSS-Operation

-	SS-NotAvailable

-	SS-SpecificError

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

The possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.8.7	Normal operation

In case of barring of outgoing call the calling mobile receives information about the activation of supplementary services subscribed.

In case of barring of incoming call the calling mobile receives information about the activation of supplementary services subscribed by the other party (the mobile called).

Method of test:

The MS is forced to place an outgoing call.

Upon receipt of the SET UP message, the system simulator answers either with the negative acknowledgment RELEASE COMPLETE (to simulate a case where call barring is activated).

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke =	notification

SS code = BO or BI

SS status = activation indicator (indicating :

Provisioned, registered and active)

The test is repeated with BO then BI.

Requirements:

For each case tested, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.9	Handling of undefined (future) GSM supplementary services

The MMI procedures for handling of GSM supplementary services not yet defined are described in GSM 02.31.

The ProcessUnstructuredSsData operation to be used on the radio path for such services is defined in GSM 09.02 and 04.80.

When the MS detects a MMI procedure for a supplementary service where the coding of MMI and information elements is not yet specifically defined in GSM, it generates a REGISTER message with the facility information element containing an invoke of the Process Unstructured SS Data operation.

31.9.1	Test sequences

In sections 31.9.2 to 31.9.7 the test procedures for registration, erasure, activation, deactivation, interrogation and operations not yet defined in GSM 02.30 are described.

For registration, erasure, activation, deactivation and interrogation, the test sequence below is executed for a set of service codes NN(N) which have not yet been allocated for GSM supplementary services (see GSM 02.30 for service codes already specified).

This set of service codes is: 00, 000, 999 and two service codes randomly chosen in the range 01 to 99 and 001 to 998 (one service code in each range). In that random selection, service codes for which explicit tests are carried out in earlier tests of section 30 should not be selected.

Registration rejected

Registration accepted

Erasure rejected

Activation rejected

Activation accepted

Interrogation accepted

Interrogation rejected

Deactivation rejected

Deactivation accepted

Erasure accepted.

For operations not yet defined by GSM 02.30 the following test sequence is executed:

-	Operation accepted (see section 31.9.7)

-	Operation rejected (see section 31.9.7).

For each test in a test sequence the MS is forced to send a REGISTER message with the facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

-	SS-UserData.

The SS-UserData parameter will in an IA5String containing the keystrokes defined in the test case.

For registration, erasure, activation, interrogation and deactivation, each test in a test sequence characterized by the service code NN(N) shall be executed three times:

-	with no supplementary information

-	with one supplementary information field Si containing a value chosen at random

-	with two supplementary information fields Sia and Sib both containing values chosen at random.

Besides this, each test in a test sequence shall be repeated a number of times as described in sections 31.9.2 to 31.9.6.

31.9.2	Registration

31.9.2.1	Registration accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility invoke = Process unstructured SS data

-	SS-UserData.

The SS-UserData parameter shall, in an IA5String, contain the following digits and symbols:

**NN(N)#		(No supplementary information included)

or	**NN(N)*Si#	(One field of supplementary information included)

or	**NN(N)*Sia*Sib#	(Two fields of supplementary information included).

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility return result = Process unstructured SS data

-	SS-UserData	(optional).

The SS-UserData parameter may contain information in an IA5String.

The invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result. When SS-UserData is included in the return result the content of the IA5String is chosen at random. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). If IA5String information is included this shall be given to the user (in a way described by the manufacturer).

31.9.2.2	Registration rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case 31.9.2.1.

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If facility = Return error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If facility = Reject

The possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.9.3	Erasure

31.9.3.1	Erasure accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility invoke = Process unstructured SS data

-	SS-UserData.

The SS-UserData parameter shall, in an IA5String, contain the following digits and symbols:

##NN(N)#	(No supplementary information included)

or	##NN(N)*Si#	(One field of supplementary information included)

or	##NN(N)*Sia*Sib#	(Two fields of supplementary information included).

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility return result = Process unstructured SS data

-	SS-UserData	(Optional).

The SS-UserData parameter may contain information in an IA5String.

The invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result. When SS-UserData is included in the return result the content of the IA5String is chosen at random. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). If IA5String information is included this shall be given to the user (in a way described by the manufacturer).

31.9.3.2	Erasure rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case 31.9.3.1.

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If facility = Return error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If facility = Reject

The possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.9.4	Activation

31.9.4.1	Activation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility invoke = Process unstructured SS data

-	SS-UserData.

The SS-UserData parameter shall in an IA5String contain the following digits and symbols:

*NN(N)#		(No supplementary information included)

or	*NN(N)*Si#		(One field of supplementary information included)

or	*NN(N)*Sia*Sib#	(Two fields of supplementary information included).

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility result = Process unstructured SS data

-	SS-UserData	(optional).

The SS-UserData parameter may contain information in an IA5String.

The invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result. When SS-UserData is included in the return result the content of the IA5String is chosen at random. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). If IA5String information is included this shall be given to the user (in a way described by the manufacturer).

31.9.4.2	Activation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case 31.9.4.1.

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If facility = Return error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If facility = Reject

The possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.9.5	Deactivation

31.9.5.1	Deactivation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility invoke = Process unstructured SS data

-	SS-UserData.

The SS-UserData parameter shall, in an IA5String, contain the following digits and symbols:

#NN(N)#		(No supplementary information included)

or	#NN(N)*Si#	(One field of supplementary information included)

or	#NN(N)*Sia*Sib#	(Two fields of supplementary information included).

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility return result = Process unstructured SS data

-	SS-UserData	(optional).

The SS-UserData parameter may contain information in an IA5String.

The invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result. When SS-UserData is included in the return result the content of the IA5String is chosen at random. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). If IA5String information is included this shall be given to the user (in a way described by the manufacturer).

31.9.5.2	Deactivation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case 31.9.5.1.

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If facility = return error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If facility = reject

The possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.9.6	Interrogation

31.9.6.1	Interrogation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility invoke = Process unstructured SS data

-	SS-UserData.

The SS-UserData parameter shall, in an IA5String, contain the following digits and symbols:

*#NN(N)#		(No supplementary information included)

or	*#NN(N)*Si#	(One field of supplementary information included)

or	*#NN(N)*Sia*Sib#	(Two fields of supplementary information included).

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility return result = Process unstructured SS data

-	SS-UserData	(optional).

The SS-UserData parameter may contain information in an IA5String.

The invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result.

When SS-UserData is included in the return result the content of the IA5 string is chosen at random.

The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS shall transmit the REGISTER message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). If IA5String information is included this shall be given to the user (in a way described by the manufacturer).

31.9.6.2	Interrogation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case 31.9.6.1.

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If facility = Return error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If facility = Reject

The possible problems are:

General problems:

-	Unrecognized component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognized operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

31.9.7	Operations not yet specified in GSM 02.30 - General test of transparency of MMI inputs

31.9.7.1	Operation accepted

Method of test:

The following tests shall be performed within a call.

By means of a specified MMI keypad operation (defined below) the MS is forced to send a REGISTER or FACILTY message with the facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility invoke = ProcessUnstructuredSsData

-	SS-UserData.

The SS-UserData parameter shall in an IA5 string contain exactly the digits and symbols expressed on the mobile equipment keypad. One test for each of the below expressed keypad procedures shall be performed.

MMI key depressions	IA5 string contents

0 SEND (note 1)	0

1 SEND 	1

2 SEND	2

:		:

:		:

9 SEND	9

Note 1:	The keyword "SEND" refers to the SEND function as defined in GSM 02.31.

Upon receipt of the REGISTER or FACILITY message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility return result = ProcessUnstructuredSsData

-	SS-UserData (optional).

The SS-UserData parameter may contain information in an IA5 string.

The Invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result. When SS-UserData is included in the return result the content of the IA5 string is chosen at random. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS shall transmit the REGISTER or FACILITY message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). If IA5 information is included this shall be given to the user (in a way described by the manufacturer).

31.9.7.2	Operation rejected

Method of test:

The MS is forced to send the REGISTER or FACILITY message in the same way as described for case 31.9.7.1.

Upon receipt of the REGISTER or FACILITY message, the system simulator answers with the RELEASE COMPLETE message with the facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If facility = Return error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If facility = reject

The possible problems are:

General problems:

-	Unrecognised component

-	Mis-typed component

-	Badly structured component.

Invoke problems:

-	Unrecognised operation

-	Mis-typed parameter

-	Resource limitation

-	Initiating release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).