ANNEX 5:	TEST EQUIPMENT

A5.1	INTRODUCTION

A5.1.1	General

The test equipment is either an equipment or assembly of equipments which enables the tests described in this specification to be conducted.

This annex describes requirements for the test equipment which cannot be derived from and which are assumed in, the conformance test descriptions described in this specification.

Specifically stimulus setting and measurement uncertainties are defined.

A5.1.2	Test equipment terms

The term "System Simulator" (SS) is used to describe the complete suite of test equipment required to perform the tests in this specification when interacting with the following MS interfaces:

-	Antenna (Connector or radiated)

-	Accoustic

-	Data Port(s)

-	Power supply

-	DAI

Note :	To perform a sub-set of tests, the SS may be simplified accordingly.

The term "SIM simulator" is used to describe the test equipment required to interact with the SIM/ME interface.

A "test SIM" has the physical characteristics of a standard SIM card, (see GSM 11.11) with specific parameters defined in Annex 3.

A5.1.3	Confidence level

All uncertainty values stated in this annex are quoted for a Confidence Level of 95%.

A5.2.	Standard test signals

The Cx signals represent the wanted signals and the Ix signals represent the unwanted signals.

Signal C0	Unmodulated continuous carrier;

Signal C1	A standard GSM signal with the modulation derived by applying a data reversals signal to the input of a channel coder. The channel coder will depend on the test and the cypher mode shall be selectable by the test method. When using this signal in the non hopping mode, the unused seven time slots shall also contain dummy bursts, with power levels variable with respect to the used timeslot, see also 2.3.1.3.

Signal I0	Unmodulated continuous carrier;

Signal I1	A GMSK modulated carrier following the structure of the GSM signals, but with all modulating bits (including the midamble period) derived directly from a random or pseudo random data stream.

Signal I2	A standard GSM signal with valid midamble, different from C1. The data bits (including bits 58 and 59) shall be derived from a random or pseudo random data stream.

A5.3.	SS functional requirements

A5.3.1	Level setting range

It is assumed that the SS is capable of setting stimulus levels, at the MS interface, to those required in the test specification extended by the measurement uncertainty defined in this annex.

Note :	This ensures that the SS is able adequately to stimulate the MS performance at and just beyond the limit requirement under all conditions.

A5.3.2	Level Measurement / operation range

It is assumed that the SS is capable of performing measurements, within the uncertainty defined in this annex, over a level range, at the MS interface, as required in the test specification extended by the SS measurement uncertainty defined in this annex and extended by a further 3dB on the MS conformity requirement.

Note :	This ensures that the SS is able adequately to measure the MS performance at and just beyond the limit requirement under all conditions.

A5.3.3	MS power supply interface

Test DC power supply for MS:

Voltage setting uncertainty		< 1 %

Ripple					< 10 mV RMS, 50 mV peak to peak

Test AC power supply for MS:

Voltage setting uncertainty:		< 1 %.

A5.3.4	MS antenna interface

The SS is assumed to offer a nominal 50 ohm impedance to the MS

GSM/DCS bands	< 4 GHz	< 10 GHz	< 12.75 GHz

VSWR	<= 1.3	<= 2.0	<= 3.0	<= 3.5

A5.3.4.1	Uplink receiver error

The SS receiver should be capable of performing the tests as specified in GSM 11.10 without the addition of bit errors in excess of 1 in 10E7 due to the receiver performance when operated with a MS which meets the transmitter requirements of GSM 05.05.

Note :	This requirement is based on a minimum BER measurement of 1 in 10E5.

A5.3.4.2	Power versus Time measurements

Measurement uncertainty of peak transmitter carrier power:	+/- 1 dB

Measurement uncertainty of power level (relative to peak transmitter carrier power):

Power level				Measurement uncertainty

+ 6 dB to - 7 dB				+/-0.25 dB

- 7 dB to -20 dB				+/-1.0 dB

-20 dB to -32 dB				+/-2.0 dB

-32 dB to -45 dB				+/-5.0 dB

-45 dB to -71 dB				+/-1.0 dB

<-71 dB					+/-2.0 dB

Note :	Due to the method of measurement (downconversion to I/Q baseband / filtering / A/D conversion / postprocessing) several uncertainties occur. The sources are:

a)	absolute level uncertainty;

b)	filter ripple,

I/Q gain imbalance,

I/Q imperfect quadrature;

c)	A/D conversion (resolution),

I/Q offset.

Items under b) and c) affect the individual samples and can be observed as a "ripple" in the horizontal part of the power time mask.

Items under b) are uncertainties which are proportional to the signal measured.

Items under c) are constant amounts of uncertainty, independent of the signal measured.

The item a) moves the entire power time template up or down.

The uncertainties b) and c) are added to the measured signal as an uncorrelated interferer.

The above mentioned absolute measurement uncertainty refers to a). The table covers uncertainties b) and c).

Uncertainty of time measurement

The relative timing uncertainty of the transition point

-	bit 13 to 14 in the midamble (normal burst)

-	end of the sync sequence (access burst)

is +/- 1/8 bit.

Timing uncertainty of the measurement samples in the vertical part of the power time mask are displayed as marked fields in the Figure A5.3-1

.G.1110figs\FIGA3-1.EPS;16.499 cm;25.683 cm;EPS

Note :	With a real method of measurement one has to reckon on systematic measurement uncertainties in the vertical part of the power time template (Figures 13-2 & 13-3). The reason for this is that the measurement is conducted through a filter which has to fulfil different requirements simultaneously, requirements in the frequency domain and in the time domain as well. The time behaviour of the filter causes the above mentioned measurement uncertainty. It occurs clearly when measuring the falling edge of the power burst. The measurement uncertainty, which in principle delays the actual performance, depends on the filter characteristics and on the signal shape. At favourable signal shapes the uncertainty is negligible, however, at unfavourable signal shapes it consumes the marked area in Figure A5.3-1 (falling edge).

The underlying filter is:

type			inverse Chebycheff

passband			<= +/- 200 kHz

stopband (40 dB stop att.)	>= +/- 541.67 kHz.

To avoid aliasing with this filter the RF output spectrum must meet the requirements of section 13.4.

If the lowest limit line in the power time template is replaced by a -54 dBm line, measuring lower carrier powers, the area of measurement uncertainty is reduced equivalently.

The marked area in Figure A5.3-1 describes the systematic measurement uncertainty of the test equipment and does not widen the design requirements.

Uncertainties associated with 13.3.5 requirement b) (power control levels, adjacent steps):

Repeatability			+/- 0.3 dB

Linearity				+/- 0.03 dB/dB

Combined uncertainty is:		+/- ( 0.3 + 0.03 dB/dB ) dB

E.g. where the indicated value of the step size is 2.0 dB, the uncertainty is: +/- (0.3 + 0.06) dB = +/- 0.36 dB.

A5.3.4.3	Wideband selective power measurement

Power is to be measured selectively for spurious emissions without frequency hopping (ref: section 12.

Uncertainty conducted		100 kHz to 1GHz		+/- 1.5 dB

1 GHz to 12.75 GHz	+/- 3.0 dB

Uncertainty radiated		30 MHz to 4 GHz		+/- 6 dB

Note :	The uncertainties include the effect of a worst case reflection from the MS of 0.7 for out of band signals.

It is acceptable to use a band stop filter in spurious emission measurements of the transceiver in order to fulfil the above requirements.

A5.3.4.4	Inband selective power measurements

Power is to be measured selectively for output RF spectrum

The measurement is performed on a single frequency while the MS is frequency hopping (ref: section 13.3)

Uncertainty			< +/- 1.6 dB

Note :	The video signal of the spectrum analyser is "gated" such that the spectrum generated by at least 40 of the bits 87 to 132 of the burst is the only spectrum measured. This gating may be analogue or numerical, dependent upon the design of the spectrum analyser.

A5.3.4.5	Phase trajectory and frequency error measurements

Ref: Sections 13.1 & 13.2 for definitions and methods of measurement.

Phase measurement uncertainty	+/- 1 degree RMS

+/- 4 degrees for individual

phase measurement samples

The phase measurement uncertainties above apply during the useful bits.

Frequency measurement uncertainty:	+/- 5 Hz.

A5.3.4.6	RF delay measurements relative to nominal times

Range				-140 to +140 bit periods

Resolution			1/4 bit period

Uncertainty			+/- 1/8 bit period

A5.3.4.7	The Wanted signal or Traffic channel of serving cell

The Wanted signal is used in most of the specified RF measurements. The traffic channel of the serving cell is used in most of the signalling tests.

FREQUENCY:

Uncertainty:		< +/- 5*10E-9

PHASE:

Uncertainty:		< +/- 1 degree RMS and

< +/- 4 degrees peak(as defined in GSM 05.05)

LEVEL:

Uncertainty:		< +/- 1 dB in section 13, 14 except:-

< +/- 3 dB for test 14.2 radiated

< +/- 1.2 dB for test 14.6

< +/- 2.5 dB for all other tests.

Settling time:	< 10 us

DYNAMIC LEVEL SETTING:

The SS shall be able to switch from any power level to any other power level within the range of 30 dB on a timeslot per timeslot basis. This dynamic switching requirement only applicable for a single channel for a limited number of tests.

MODULATION:		GMSK (as specified in GSM 05.04)

SPURIOUS:

inchannel:		Covered by phase error

outchannel:

Noise Power, 1 Hz bandwidth:

<-100 dBc for >100kHz carrier offset,									<-110 dBc for >300kHz carrier offset,									<-121 dBc for >1500kHz carrier offset.

Non harmonics:	< -55 dBc for > 100kHz carrier offset,

< -68 dBc for >1500kHz carrier offset

FREQUENCY HOPPING:

The signal shall be capable of hopping according to the criteria of GSM 05.02. The timing of the frequency change shall be such that frequency transitions do not occur during the active timeslot of the MS.

A5.3.4.8	The First interfering signal or Traffic channel of the first adjacent cell

The First interfering signal is used in measurements of co-channel rejection, adjacent channel rejection and intermodulation rejection. The Traffic channel of the first adjacent cell is used in handover tests.

FREQUENCY:

Uncertainty:		< +/- 5*10E-9

PHASE:

Uncertainty:		< +/- 1 degree RMS and

< +/- 4 degrees peak(as defined in GSM 05.05)

LEVEL:

Uncertainty		< +/- 1 dB relative to the wanted signal for

test 13.2 and 14.5

< +/- 0.3 dB relative to the wanted signal for

test 14.4

< +/- 1 dB for test 14.6

< +/- 2.5 dB for all other tests

MODULATION:		GMSK (as specified in GSM 05.04)

The total relative single sideband power (noise + harmonics) in the frequency range 1.5 to 1.7 MHz offset from the nominal carrier frequency shall be less than -72 dBc.

SPURIOUS:

Inchannel:		Covered by phase error

Outchannel:

Noise Power, 1 Hz bandwidth:

<-100 dBc for > 100kHz carrier offset

<-110 dBc for > 300kHz carrier offset

<-127 dBc for >1500kHz carrier offset

non harmonics	< -55 dBc for > 100kHz carrier offset

< -68 dBc for >1500kHz carrier offset

FREQUENCY HOPPING:

The signal shall be capable of hopping according to the criteria of GSM 05.02. The timing of the frequency change shall be such that frequency transitions do not occur during the active timeslot of the MS.

A5.3.4.9	The Second interfering signal

The second interfering signal is used in the measurements of intermodulation rejection and blocking.

FREQUENCY:

Uncertainty:		< +/- 5*10E-9

LEVEL:

Uncertainty:		< +/- 1 dB for test 14.6

< +/- 1.5 dB relative to the wanted signal

for all other tests.

MODULATION:		Unmodulated

SPURIOUS:

Inchannel:		No requirements

Outchannel:

Noise Power, 1 Hz bandwidth:

<-135 dBc for > 500kHz carrier offset,

<-140 dBc for > 700kHz carrier offset,

<-150 dBc for >1500kHz carrier offset.

Non harmonics	< -79 dBc for > 500kHz carrier offset,

< -84 dBc for > 700kHz carrier offset,

< -94 dBc for >1500kHz carrier offset

Harmonically related spurii	<-40 dBc

A5.3.4.10	BCCH carriers of serving and adjacent cells

The BCCH of the serving cell is used for synchronising the MS and to send network information to the MS under test. The BCCH signals of the adjacent cells are used in the handover tests. The MS measures the RF-levels of the BCCHs of adjacent cells.

FREQUENCY:

Uncertainty:		< +/- 5*10E-9

PHASE:

Uncertainty:		< +/- 1 degree RMS and

< +/- 4 degrees peak(as defined in GSM 05.05)

LEVEL:

Uncertainty:		< 1 dB for test 13.2

< 2.5 dB for all other tests

< 0.6 dB relative to each other and to TCH

for test 21 over the range 65 dBmicroVoltemf

to 3 dBmicroVoltemf

< 1.2 dB relative to each other and to TCH for test 26.3

MODULATION:		GMSK (as specified in GSM 05.04)

SPURIOUS:

Inchannel:		Covered by phase error

Outchannel:

Noise Power, 1Hz bandwidth:

<-100 dBc for > 100kHz carrier offset

<-125 dBc for >1500kHz carrier offset

Non harmonics	< -55 dBc for > 100kHz carrier offset

< -72 dBc for >1500kHz carrier offset

A5.3.4.11	The wide frequency range signal

The wide frequency range signal is used in the measurements of spurious response.

FREQUENCY

Uncertainty:		< +/- 5*10E-9

LEVEL

Uncertainty:		< +/- 1.5 dB relative to the wanted signal for test 14.7

< +/- 1 dB error of substituted 'wanted signal'

MODULATION:		unmodulated

SPURIOUS in the MS receiving range

non harmonics:	< -94 dBc

Harmonically related spurii:	< -40 dBc

Noise:		< -4 dBuVemf equivalent at the MS receiver input

when measured in a 200 kHz bandwidth.

A5.3.4.12	The multipath fading function

The multipath fading function simulates the fading effects of a broadband radio channel in mobile radio communication

The propagation conditions are specified in GSM 05.05, annex 3.

The multipath fading function shall be performed only within a 5 MHz bandwidth during one test case.

A5.3.5	MS audio interface and DAI

A5.3.5.1	General uncertainties

Unless otherwise specified, the following uncertainties apply to the audio interface:--

Signal level measurement uncertainty:		+/- 0.2 dB

Sound presure measurement uncertainty:	+/- 0.6 dB

Frequency Measurement uncertainty:		+/- 0.1%

Stimulus frequency setting uncertainty:

Frequency settings are taken from ISO 3, R10 series or R40 series or from Table 2 of Rec. CCITT P.79. A departure from the nominal frequencies of +/- 5% below 240 Hz and +/-2% at 240 Hz and above is accepted.

In the case of 4 kHz the departure is restricted to -2%.

A5.3.5.2	Analog single test tone

Total distortion:				< 0.5 %

A5.3.5.3	Delay measurement between Um and DAI

The delay measurement between the Um interface of the MS and its DAI in both directions is described in section 32.5.

Uncertainty:				< +/- 0.1 ms

A5.4	SIM SIMULATOR FUNCTIONAL requirements

A5.4.1	General

The SIM simulator shall implement the functions of a SIM as described in GSM 02.17 and 11.11.

The Test Algorithm for authentication incorporated in the SIM Simulator shall operate as described in Annex 3

A5.4.2	Contacts C1, C2, C6, C7

A5.4.2.1	Default measurement / setting uncertainties

Unless stated otherwise below, the following uncertainties apply:

Voltage measurement uncertainty:	< +/- 50 mV

Voltage setting uncertainty:		< +/- 20 mV

Time measurement uncertainty:		< +/- 100 ns

A5.4.2.2	Contact C1

Single Current spike

Current setting uncertainty:		< +/- 2mA

Pulse width duration uncertainty:	< +/- 25 ns

Rise/Fall transition time uncertainty:	< +/- 5ns (0 to 100mA)

< +/- 10ns (100 to 300 mA)

Continuous / pseudo-random Spikes

Current uncertainty:			< +/- 0.5mA (Load 0 to 20 mA)

Pulse width & gap uncertainty:		< +/- 25 ns

Rise & fall time uncertainty:		< +/- 10 ns

A5.4.2.3	Contact C7

The Elementary Time Unit (etu) used in the sections below refer to the nominal bit duration on the I/O line, as defined in ISO 7816-3.

Rise & fall Time setting uncertainty:	< +/- 100 ns

Jitter measurement uncertainty:		< +/- 5*10E-3 etu

Jitter setting uncertainty:		< +/- 5*10E-3 etu

A5.4.3	Contact C3

Frequency measurement uncertainty:		< +/- 0.5 %

Voltage Measurement uncertainty:		< +/- 50 mV

Rise & fall time measurement uncertainty:	< +/- 5 ns

Duty cycle measurement uncertainty:		< +/- 2.5 %

A5.4.4	Definition of timing

It shall be possible to define all timings relative to the clock. The SIM simulator shall be able to calculate and to use the absolute values automatically, even if the ME changes the frequency during the communication.