Default tab should be 1.5cm

10	Generic call set up procedures

10.1	Generic call set-up procedure for mobile terminating speech calls

In the test procedures described in this I-ETS, unless otherwise stated in the test description, the Mobile Terminating Speech call set-up procedure shall be as described in this section.

NOTE :	In test cases where a fading profile is required, a different and appropriate ARFCN may be selected, for instance if the fading simulator bandwidth does not allow use of the default ARFCN.

10.1.1	Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

-	The MS shall be operated under normal test conditions (see Annex 1 TC.2.1)

-	The special Test-SIM (see Annex 4) shall be inserted.

-	The MS is "idle, updated", with a TMSI assigned and listening to the BCCH/CCCH of the active cell.

10.1.2	Definition of system information messages

The following parameters shall be coded into the system information messages. Parameters shall be coded according to GSM 04.08.

The RACH Control Parameters IE shall be the same in SYSTEM INFORMATION TYPE 1, TYPE 2, TYPE 3 and TYPE 4 messages.

The Location Area Identification IE, Cell Selection Parameters IE, and P1 bit shall be the same in SYSTEM INFORMATION TYPE 3 and TYPE 4 messages.

SYSTEM INFORMATION TYPE 1

Information Element			Value/remark

Cell channel description			Includes the hopping sequence ARFCNs,

if hopping is used

RACH control parameters

MAX RETRANS			Any Value

TX-INTEGER			Any Value

CELL BAR ACCESS			Not barred

CALL RE-ESTABLISHMENT		Not Allowed

EMERGENCY CALL			Allowed

ACCESS CONTROL CLASS		None Barred

(0...9, 11...15)

SI1 rest octets				Spare Octets

SYSTEM INFORMATION TYPE 2

Information Element			Value/remark

BCCH Frequency list			Indicates seven surrounding cells on any ARFCN

of the supported band, excluding ARFCNs in or

immediately adjacent to those specified in section 6.2

NCC permitted

NCC PERMITTED			e.g all NCCs permitted

RACH control parameters

MAX RETRANS			Any Value

TX-INTEGER			Any Value

CELL BAR ACCESS			Not barred

CALL RE-ESTABLISHMENT		Not Allowed

EMERGENCY CALL			Allowed

ACCESS CONTROL CLASS		None Barred

(0...9, 11...15)

SYSTEM INFORMATION TYPE 3

Information Element			Value/remark

Cell identity

CI VALUE				0001 hex (not relevant)

Location Area Identification

MCC					001  decimal (not relevant)

MNC					01   decimal (not relevant)

LAC					0001 hex (not relevant)

Control Channel Description

ATT (IMSI att/det)			MS shall not apply (not relevant)

BS-AG-BLKS-RES			0 blocks reserved (not relevant)

CCCH-CONF			Combined CCCH/SDCCH (not relevant)

BS-PA-MFRMS			5 multiframes (not relevant)

T3212					Infinite

Cell options

PWRC				power control not set

DTX					MS must not use DTX

RADIO LINK TIMEOUT		8

Cell selection parameters

CELL RESELECT HYSTERESIS	0 dB

MS-TXPWR-MAX-CCH		Max. output power of MS

RXLEV-ACCESS-MIN			-90 dBm

ACS					There are no additional cell parameters included

in SI7 and SI8

NECI					New establishment cause not supported

RACH control parameters

MAX RETRANS			Any Value

TX-INTEGER			Any Value

CELL BAR ACCESS			Not barred

CALL RE-ESTABLISHMENT		Not Allowed

EMERGENCY CALL			Allowed

ACCESS CONTROL CLASS		None Barred

(0...9, 11...15)

SI3 rest octets

P1					C2 parameters not present

SYSTEM INFORMATION TYPE 4

Information Element			Value/remark

Location Area Identification

MCC					001 decimal (not relevant)

MNC					01 decimal (not relevant)

LAC					0001 hex (not relevant)

Cell selection parameters

CELL RESELECT HYSTERESIS	0 dB

MS-TXPWR-MAX-CCH		Max. output power of MS

RXLEV-ACCESS-MIN			-90 dBm

RACH control parameters

MAX RETRANS			Any Value

TX-INTEGER			Any Value

CELL BAR ACCESS			Not barred

CALL RE-ESTABLISHMENT		Not Allowed

EMERGENCY CALL			Allowed

ACCESS CONTROL CLASS		None Barred

(0...9, 11...15)

CBCH Channel Description			Omitted

Channel type and TDMA offset	[TBD]

TN					[TBD]

TSC					[TBD]

H					[TBD]

MAIO					[TBD]

FB-NO				[TBD]

HSN					[TBD]

ARFCN				[TBD]

CBCH Mobile Allocation			Omitted

MA					[TBD]

SI4 rest octets

P1					C2 parameters not present

SYSTEM INFORMATION TYPE 5

Information Element			Value/remark

Neighbour cell description			As Cell Channel Description in SI 1

SYSTEM INFORMATION TYPE 6

Information Element			Value/remark

Cell identity

CI VALUE				0001 hex (not relevant)

Location Area Identification

MCC					001 decimal (not relevant)

MNC					01 decimal (not relevant)

LAC					0001 hex (not relevant)

Cell options

PWRC				power control not set

DTX					MS must not use DTX

RADIO LINK TIMEOUT		8

NCC permitted

NCC PERMITTED			e.g all NCCs permitted

10.1.3	Procedure

An MS terminating call on a TCH/FS shall be established under ideal radio conditions and with Timing advance set to 0, as follows:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging

subchannel

2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates

"answer to paging".

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.

All following

messages shall be sent enciphered.

9	SS		SS starts ciphering.

10	SS -> MS	SETUP	Message contains the signal IE

but not Bearer Capability IE

11	MS -> SS	CALL CONFIRMED	contains Bearer Capability IE

A12	MS -> SS	CONNECT

B12	MS -> SS	ALERTING

B13	MS		An alerting indication as defined

in an PICS/PIXIT statement

is given by the MS.

B14	MS		The MS is made to accept the call

in a way described in a

PICS/PIXIT statement.

B15	MS -> SS	CONNECT

16	SS -> MS	ASSIGNMENT COMMAND

17	MS -> SS	ASSIGNMENT COMPLETE

18	MS		The TCH is through connected in both

directions.

19	SS -> MS	CONNECT ACKNOWLEDGE

10.1.4	Specific message contents

PAGING REQUEST TYPE 1 (GSM 04.08, 9.1.22) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator					0000

Message Type

Page Mode				Normal Paging

Channel Needed				spare, any channel

Mobile Identity 1

Odd/even no of digits			As applicable for TMSI

Type of Identity			TMSI

Identity digits			As applicable

Mobile Identity 2				Omitted

P1 rest octets				Spare octets

IMMEDIATE ASSIGNMENT (GSM 04.08, 9.1.18) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator				0000

Message Type

Page Mode				Normal

Channel Description

Channel Type			SDCCH/SACCH 1(4)

Time slot number			zero

Training seq. code			same as BCCH

Hopping				No

ARFCN				ARFCN of the BCCH

Random Reference

Random access info			As in CHAN REQ

N51, N32, N26			As applicable

Timing Advance				0

Mobile allocation				length 0 due to hopping

IA rest octets				Spare octets

AUTHENTICATION REQUEST (GSM 04.08, 9.2.2) to the MS

Information Element			Value/remark

Protocol Discriminator			MM

Skip Indicator				0000

Message Type

Ciphering key seq. number			Arbitrary

Authent. parameter RAND			Arbitrary

CIPHERING MODE COMMAND (GSM 04.08, 9.1.9) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator				0000

Message Type

Ciphering mode setting			Start ciphering

Algorithm Identifier				Supported by the MS

Cipher Response				IMEISV shall not be included

SETUP (GSM 04.08, 9.3.23) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			SS orig.

Message Type

Signal						any non-reserved value

ASSIGNMENT COMMAND (GSM 04.08, 9.1.2) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator					0000

Message Type

Channel Description

Channel type			Bm + ACCHs

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

ARFCN				Default

Power level				Power control level 7

Channel mode				Speech full rate

CONNECT ACKNOWLEDGE (GSM 04.08, 9.3.6) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			SS orig.

Message Type

10.2	Generic call set-up procedure for mobile originating speech calls

In the test procedures described in this I-ETS, unless otherwise stated in the test description, the Mobile Originating Speech (MOC) call set-up procedure shall be as described in this clause.

NOTE :	In test cases where a fading profile is required, a different and appropriate ARFCN may be selected, for instance if the fading simulator bandwidth does not allow use of the default ARFCN.

10.2.1	Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

-	The MS shall be operated under normal test conditions (see [Annex 1 TC.2.1])

-	The special Test-SIM (see Annex 4) shall be inserted.

-	The MS is "idle, updated", with a TMSI assigned and listening to the BCCH/CCCH of the active cell.

10.2.2	Definition of system information messages

See clause 10.1.2.

10.2.3	Procedure

An MS originating call on a TCH/FS shall be established under ideal radio conditions and with Timing advance set to 0, as follows:

Step	Direction	Message	Comments

1	MS		"called number" entered

2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates

"originating call, NECI <> 1".

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.

All following messages shall

be sent enciphered.

9	SS		SS starts ciphering.

10	MS -> SS	SETUP

11	SS -> MS	CALL PROCEEDING

12	SS -> MS	ALERTING

13	MS		An alerting indication as defined in an

PICS/PIXIT statement is given by the MS.

14	SS -> MS	ASSIGNMENT COMMAND

15	MS -> SS	ASSIGNMENT COMPLETE

16	SS -> MS	CONNECT

17	MS -> SS	CONNECT ACKNOWLEDGE

18	MS		The TCH is through connected in both

directions

10.2.4	Specific message contents

IMMEDIATE ASSIGNMENT (GSM 04.08, 9.1.18) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator				0000

Message Type

Page Mode				Normal

Channel Description

Channel Type			SDCCH/SACCH 1(4)

Time slot number			zero

Training seq. code			same as BCCH

Hopping				No

ARFCN				ARFCN of the BCCH

Random Reference

Random access info			As in CHAN REQ

N51, N32, N26			As applicable

Timing Advance				0

Mobile allocation				length 0 due to hopping

IA rest octets				Spare octets

AUTHENTICATION REQUEST (GSM 04.08, 9.2.2) to the MS

Information Element			Value/remark

Protocol Discriminator			MM

Skip Indicator				0000

Message Type

Ciphering key seq. number			Arbitrary

Authent. parameter RAND			Arbitrary

CIPHERING MODE COMMAND (GSM 04.08, 9.1.9) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator				0000

Message Type

Ciphering mode setting			Start ciphering

Algorithm Identifier				Supported by the MS

Cipher Response				IMEISV shall not be included

CALL PROCEEDING (GSM 04.08, 9.3.3) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			As derived from SETUP

Message Type

Repeat Indicator				Omitted

Bearer Capability 1				Omitted

Bearer Capability 2				Omitted

Facility					Omitted

Progress Indicator				Omitted

ALERTING (GSM 04.08, 9.3.1) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			As derived from SETUP

Message Type

Facility					Omitted

Progress Indicator				Omitted

User-user				Omitted

ASSIGNMENT COMMAND (GSM 04.08, 9.1.2) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator				0000

Message Type

Channel Description

Channel type			Bm + ACCHs

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

ARFCN				Default

Power level				Power control level 7

Channel mode				Speech full rate

CONNECT (GSM 04.08, 9.3.5) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			As derived from SETUP

Message Type

Facility					Omitted

Progress Indicator				Omitted

Connected number				Omitted

Connected Subaddress			Omitted

User-user				Omitted

10.3	Generic call set-up procedure for mobile terminating data calls

In the test procedures described in this I-ETS, unless otherwise stated in the test description, the Mobile Terminating Data call set-up procedure shall be as described in this section.

NOTE :	In test cases where a fading profile is required, a different and appropriate ARFCN may be selected, for instance if the fading simulator bandwidth does not allow use of the default ARFCN.

10.3.1	Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

-	The MS shall be operated under normal test conditions (see Annex 1 TC.2.1)

-	The special Test-SIM (see Annex 4) shall be inserted.

-	The MS is "idle, updated", with a TMSI assigned and listening to the BCCH/CCCH of the active cell.

10.3.2	Definition of system information messages

See clause 10.1.2.

10.3.3	Procedure

An MS terminating call on a TCH shall be established under ideal radio conditions and with Timing advance set to 0, as follows:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging

subchannel

2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates

"answer to paging".

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.

All following

messages shall be sent enciphered.

9	SS		SS starts ciphering.

10	SS -> MS	SETUP	A call is set up according to the required

characteristics of the test procedure.

Bearer Capability and Signal IEs included

11	MS -> SS	CALL CONFIRMED	Bearer Capability shall or shall not be

included according to the rules

given in TS GSM 04.08 and 07.01.

A12	MS -> SS	CONNECT

B12	MS -> SS	ALERTING

B13	MS		An alerting indication as defined

in an PICS/PIXIT statement

is given by the MS.

B14	MS		The MS is made to accept the call

in a way described in a

PICS/PIXIT statement.

B15	MS -> SS	CONNECT

16	SS -> MS	ASSIGNMENT COMMAND

17	MS -> SS	ASSIGNMENT COMPLETE

18	MS		The TCH is through connected in both

directions.

19	SS -> MS	CONNECT ACKNOWLEDGE

10.3.4	Specific message contents

PAGING REQUEST TYPE 1 (GSM 04.08, 9.1.22) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator				0000

Message Type

Page Mode				Normal Paging

Channel Needed				spare, any channel

Mobile Identity 1

Odd/even no of digits			As applicable for TMSI

Type of Identity			TMSI

Identity digits			As applicable

Mobile Identity 2				Omitted

P1 rest octets				Spare octets

IMMEDIATE ASSIGNMENT (GSM 04.08, 9.1.18) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator				0000

Message Type

Page Mode				Normal

Channel Description

Channel Type			SDCCH/SACCH 1(4)

Time slot number			zero

Training seq. code			same as BCCH

Hopping				No

ARFCN				ARFCN of the BCCH

Random Reference

Random access info			As in CHAN REQ

N51, N32, N26			As applicable

Timing Advance				0

Mobile allocation				length 0 due to hopping

IA rest octets				Spare octets

AUTHENTICATION REQUEST (GSM 04.08, 9.2.2) to the MS

Information Element			Value/remark

Protocol Discriminator			MM

Skip Indicator				0000

Message Type

Ciphering key seq. number			Arbitrary

Authent. parameter RAND			Arbitrarily selected

CIPHERING MODE COMMAND (GSM 04.08, 9.1.9) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator				0000

Message Type

Ciphering mode setting			Start ciphering

Algorithm Identifier				Supported by the MS

Cipher Response				IMEISV shall not be included

SETUP (GSM 04.08, 9.3.23) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			SS orig.

Message Type

Bearer Capability

Radio Channel Requirement

Connection Element			T or NT and declared as supported by the MS

(Not "Both ...")

NIRR					No meaning

Other parameters			Declared as supported by the MS

Signal					any non-reserved value

ASSIGNMENT COMMAND (GSM 04.08, 9.1.2) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator				0000

Message Type

Channel Description

Channel type			Bm + ACCHs

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

ARFCN				Default

Power level				Power control level 7

Channel mode				Proper data rate,

according to BC-IE included in the Set-Up and

to the following table

Table 10-1:	Correspondence between User rate (UR) and Channel Mode (CM) for transparent (T) and non transparent (NT) connections

UR	9.6kbit/s	4.8kbit/s	2.4kbit/s	1.2kbit/s	1.2/0.075kbit/s	0.3kbit/s

CM T FR	12 FR	6 FR	3.6 FR	3.6 FR	3.6 FR	3.6 FR

CM T HR	n.a	6 HR	3.6 HR	3.6 HR	3.6 HR	3.6 HR

CM NT FR	12 FR	12 FR	12 FR	12 FR	12 FR	12 FR

CM NT HR	n.a	6 HR	6 HR	6 HR	6 HR	6 HR

CONNECT ACKNOWLEDGE (GSM 04.08, 9.3.6) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			SS orig.

Message Type

10.4	Generic call set-up procedure for mobile originating data calls

In the test procedures described in this I-ETS, unless otherwise stated in the test description, the Mobile Originating Data call set-up procedure shall be as described in this clause.

NOTE :	In test cases where a fading profile is required, a different and appropriate ARFCN may be selected, for instance if the fading simulator bandwidth does not allow use of the default ARFCN.

10.4.1	Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

-	The MS shall be operated under normal test conditions (see [Annex 1 TC.2.1])

-	The special Test-SIM (see Annex 4) shall be inserted.

-	The MS is "idle, updated", with a TMSI assigned and listening to the BCCH/CCCH of the active cell.

10.4.2	Definition of system information messages

See clause 10.1.2.

10.4.3	Procedure

An MS originating call on a TCH shall be established under ideal radio conditions and with Timing advance set to 0, as follows:

Step	Direction	Message	Comments

1	MS		"called number" entered

2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates

"originating call, NECI <> 1".

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.

5	SS -> MS	AUTHENTICATION REQ

6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.

7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending

the message.

8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.

All following messages shall

be sent enciphered.

9	SS		SS starts ciphering.

10	MS -> SS	SETUP

11	SS -> MS	CALL PROCEEDING

12	SS -> MS	ALERTING

13	MS		An alerting indication as defined in an

PICS/PIXIT statement is given by the MS.

14	SS -> MS	ASSIGNMENT COMMAND

15	MS -> SS	ASSIGNMENT COMPLETE

16	SS -> MS	CONNECT

17	MS -> SS	CONNECT ACKNOWLEDGE

18	MS		The TCH is through connected in both

directions

10.4.4	Specific message contents

IMMEDIATE ASSIGNMENT (GSM 04.08, 9.1.18) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator				0000

Message Type

Page Mode				Normal

Channel Description

Channel Type			SDCCH/SACCH 1(4)

Time slot number			zero

Training seq. code			same as BCCH

Hopping				No

ARFCN				ARFCN of the BCCH

Random Reference

Random access info			As in CHAN REQ

N51, N32, N26			As applicable

Timing Advance				0

Mobile allocation				length 0 due to hopping

IA rest octets				Spare octets

AUTHENTICATION REQUEST (GSM 04.08, 9.2.2) to the MS

Information Element			Value/remark

Protocol Discriminator			MM

Skip Indicator				0000

Message Type

Ciphering key seq. number			Arbitrary

Authent. parameter RAND			Arbitrarily selected.

CIPHERING MODE COMMAND (GSM 04.08, 9.1.9) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Skip Indicator				0000

Message Type

Ciphering mode setting			Start ciphering

Algorithm Identifier				Supported by the MS

Cipher Response				IMEISV shall not be included

CALL PROCEEDING (GSM 04.08, 9.3.3) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			As derived from SETUP

Message Type

Repeat Indicator				Present if and only if Bearer Capability 1 and

Bearer Capability 2 are present in this message

Bearer Capability 1				Present if negotiation of BC 1 or BC 2 necessary

(eg reception of "Both" for CE parameter in SETUP), else

omitted

Radio Channel Requirement		spare

Connection element			T (in case of "Both T (NT) preferred" received)

NIRR					No meaning

Other parameters			Same as sent by the MS in the SETUP, where applicable

Bearer Capability 2				Present if dual BC-IE received and negotiation of either BC 1 or BC 2

necessary, else omitted

Radio Channel Requirement		spare

Connection element			T in case of "Both, T (NT) preferred" in the SETUP

message else same as in the SETUP message

NIRR					No meaning

Other parameters			Same as sent by the MS in the SETUP, where applicable

Note: If both BC 1 and BC 2 are present, then one and only one of them shall indicate speech.

Facility					Omitted

Progress Indicator				Omitted

ALERTING (GSM 04.08, 9.3.1) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			As derived from SETUP

Message Type

Facility					Omitted

Progress Indicator				Omitted

User-user				Omitted

ASSIGNMENT COMMAND (GSM 04.08, 9.1.2) to the MS

Information Element			Value/remark

Protocol Discriminator			RR

Transaction Identifier			Not used

Message Type

Channel Description

Channel type			Bm + ACCHs

Time slot number			Arbitary

Training seq. code			Default

Hopping				No

FB no					Band no 0

ARFCN				Default

Power level				Power control level 7

Channel mode				If no negotiation took place:

-	Speech FR (resp. HR) if first BC IE in the

SETUP indicated speech FR (resp. HR),

-	Set according to the table below if first BC -IE

in the SETUP indicates data or fax

If negotiation took place

-	Speech FR (resp. HR) if first BC-IE in the CALL

PROCEEDING indicated speech FR (resp. HR)

-	Set according to the table below if first BC -IE in

the CALL PROCEEDING indicates data or fax

Table 10-2:	Correspondence between User rate (UR) and Channel Mode (CM) for transparent (T) and non transparent (NT) connections

UR	9.6kbit/s	4.8kbit/s	2.4kbit/s	1.2kbit/s	1.2/0.075kbit/s	0.3kbit/s

CM T FR	12FR	6 FR	3.6 FR	3.6 FR	3.6 FR	3.6 FR

CM T HR	n.a	6 HR	3.6 HR	3.6 HR	3.6 HR	3.6 HR

CM NT FR	12 FR	12 FR	12 FR	12 FR	12 FR	12 FR

CM NT HR	n.a	6 HR	6 HR	6 HR	6 HR	6 HR

CONNECT (GSM 04.08, 9.3.5) to the MS

Information Element			Value/remark

Protocol Discriminator			CC

Transaction Identifier			As derived from SETUP

Message Type

Facility					Omitted

Progress Indicator				Omitted

Connected number				Omitted

Connected Subaddress			Omitted

User-user				Omitted