26.3	Test of MS functions in idle mode

26.3.1	Initial conditions

The SIM shall contain a PLMN-Selector that contains only the HPLMN of the MS, and an empty forbidden PLMN list.

During the tests in 26.3.2 and 26.3.3, the following parameters apply:

RACH control parameters

In cells

GSM900:	1 to 7

DCS1800:	1 to 6:

Max retrans	= 01	2 retransmissions

Tx-integer	= 0111	(10) slots for spreading

CB, Cell Barred	= 0	access is allowed

RE	= 1	re-establishment not allowed

AC C00 to AC C15	= 0	access is not barred

In cell

GSM900:	8

DCS1800:	7:

Max retrans	= 01	2 retransmissions

Tx-integer	= 0111	(10) slots for spreading

CB, Cell Barred	= 1	access is not allowed

RE	= 1	re-establishment not allowed

AC C00 to AC C15	= 0	access is not barred

GSM900	DCS1800

Cell	PLMN	BA - ARFCN bit = 1	BA - ARFCN bit = 1

perm.

1	00000100	7	39	65	66	85	97	124	520	580	610	702	703	830	885

2	00000100	8	40	67	68	86	98	123	521	581	612	704	705	831	884

3	00000100	9	41	69	70	87	99	122	522	582	614	706	707	832	883

4	00000100	10	42	71	72	88	100	121	523	583	616	708	709	833	882

5	00000100	11	43	73	74	89	101	120	524	584	618	710	711	844	881

6	00000100	12	44	75	76	90	102	119	525	585	620	712	713	835	880

7	00000100	13	45	77	78	91	103	118	526	586	622	714	715	836	879

8	00000100	124

Location area identification

GSM900 only - begin

Cell	MCC1	MCC2	MCC3	MNC1	MNC2	LAC

1	0	0	2	0	F	x

2	0	0	3	2	F	x

3	0	0	4	3	F	x

4	0	0	5	4	F	x

5	0	0	6	5	F	x

6	0	0	7	6	F	x

7	0	0	8	7	F	x

8	0	0	1	0	1	x	The HPLMN of the MS

GSM900 only - end

DCS1800 only - begin

Cell	MCC1	MCC2	MCC3	MNC1	MNC2	LAC

1	0	0	2	0	F	x

2	0	0	3	2	F	x

3	0	0	4	3	F	x

4	0	0	5	4	F	x

5	0	0	6	5	F	x

6	0	0	7	6	F	x

7	0	0	1	0	1	x	The HPLMN of the MS

DCS1800 only - end

NOTE :	'x' denotes any value.

NOTE :	The MS representation of the MCC, MNC on the handset can be manufacturer dependant.

NOTE :	The NCC values of each cell must be different.

Control channel description and BS options

All

GSM900:	8 cells

DCS1800:	7 cells:

CELL_RESELECT_HYSTERESIS	= 010	4dB RXLEV hysteresis

MS_TXPWR_MAX_CCH	=	value corresponding to the maximum

available output power from MS

RXLEV_ACCESS_MIN	= 30

ATT	= 0	no IMSI attach and detach

DTX	= 0	no discontinuous transmission

BS_AG_BLKS_RES	= 1	1 block reserved for access grant

CCCH_CONF	= 001	1 SDCCH combined with the CCCH

RADIO_LINK_TIMEOUT	= 5	10 s timeout

BS_PA_MFRMS	= 010	4 multiframes periods for paging

T3212 timeout value	= H'00

GSM900	DCS1800

Cell	level	BCCH ARFCN	level	BCCH ARFCN

dBmVemf(  )		dBmVemf(  )

1	+65	1	+65	520

2	+63	7	+63	580

3	+61	39	+61	610

4	+55	65	+55	702

5	+59	66	+59	703

6	+57	85	+57	830

7	+55	97	+55	885

8	+53	124

NOTE :	The SIM should contain a PLMN-Selector that contains only the HPLMN of the MS, and an empty forbidden PLMN list.

26.3.2	MS indication of available PLMNs

26.3.2.1	Test purpose

To verify that a MS can present the available PLMNs to the user when asked to do so in manual mode according to the requirements of GSM 05.08 and 02.11.

26.3.2.2	Method of test

a)	The MS is switched on, equipped with a SIM containing default values except for those values listed under section 26.3.1 (initial conditions).

b)	The MS is put into manual network selection mode (see PIXIT).

26.3.2.3	Test requirements

1)	On entering manual network selection mode, the MS shall present a list of available PLMNs (MCC and MNC values, or any other valid indications, see PIXIT), within 2 minutes. The list shall include the MCC and MNC of

GSM900:	cells 1 to 7, but not of cell 8

DCS1800:	cells 1 to 6, but not of cell 7.

26.3.3	MS will send only if BSS is "on air"

26.3.3.1	Test purpose

To verify that the MS will not produce any RF transmission if no BSS is received.

26.3.3.2	Method of test

a)	The RF-signal for the BCCHs of

GSM900:	cell 1 to 8 is switched off

DCS1800:	cell 1 to 7 is switched off.

b)	The SS shall wait 20 s to allow the MS to detect the loss of cells.

c)	By MMI, an attempt to originate a call is made.

d)	By MMI, an attempt to originate an emergency call is made.

Step d) is only performed if the MS supports speech (see PICS/PIXIT statement).

26.3.3.3	Test requirements

1)	The MS must not give "service indication".

2)	In steps c) and d) the MS shall not produce any RF output.

26.3.4	Manual mode of PLMN selection

26.3.4.1	Conformance requirements

In manual mode, the MS can try to obtain normal service on any available VPLMN and it shall try to obtain normal service on a VPLMN if and only if the user makes a manual selection of this VPLMN.

Reference

GSM 03.22 section 3.1

26.3.4.2	Test purpose

To check that in manual mode the MS is able to obtain normal service on a PLMN which is neither the better nor a preferred PLMN and that it tries to obtain service on VPLMN if and only if the user selects it manually.

26.3.4.3	Method of test

Initial conditions

System Simulator:

2 cells, defaults parameter unless otherwise specified.

The SS transmits 2 BCCH carriers with the initial following parameters:

level (dBmV)

carrier A	PLMN 1	28

carrier B	PLMN 2	33

Mobile Station:

The MS is "idle updated" on PLMN1 (HPLMN) and is in manual mode.

The preferred PLMN list does not contain PLMN2, it contains PLMN 3.

Related PICS/PIXIT statement(s)

Description of the manual PLMN selector.

Foreseen final state of the MS

The MS is "idle updated"

Test procedure

For the different networks and during the whole test, "IMSI attach" flag is set in the BCCH data.

Carrier A is turned off. The MS does not attempt a location updating during 2 minutes.

Carrier A is turned back on with a different MCC-MNC (indicating PLMN 3) and with a higher level (38) than PLMN 2. The MS does not attempt a location updating during 2 minutes.

PLMN 2 is selected manually. The MS performs a location updating on PLMN 2. Carrier B is turned off. The MS does not attempt a location updating during 2 minutes.

Maximum duration of test

10 minutes

Expected Sequence

Step	Direction	Message	Comments

1	SS		carrier A is turned off

2			wait 2 min: the MS shall not send any

CHANNEL REQUEST messages during this

time

3	SS		carrier A is turned on with a different

MNC-NCC (PLMN3) and with a high level (38)

4			wait 2 min: the MS shall not send any

CHANNEL REQUEST messages during this

time

5	MS		PLMN 2 selected manually

6	MS -> SS	CHANNEL REQUEST

7	SS -> MS	IMMEDIATE ASSIGNMENT

8	MS -> SS	LOC UPD REQ	on carrier B

9	SS -> MS	LOC UPD ACCEPT

10	SS -> MS	CHANNEL RELEASE

11	SS		carrier B is turned off

12			wait 2 min: the MS shall not send any

CHANNEL REQUEST messages during this

time

Specific message contents

None