33	Mobile station features

GSM 02.07 defines mandatory and optional MS features. Their presence and appropriate functioning are verified by the following tests.

33.1	Entry and display of called number

33.1.1	Definition and applicability

The entry and display of a called number is the ability of a MS to correctly display and signal to the network the user required number.

The requirements and this test apply to all MS.

33.1.2	Conformance requirement

1)	The number of the called subscriber is included in the Called party BCD number Information element of the SETUP message for an outgoing call.

The "Display of Called number" shall be implemented in an MS where a human interface is provided.

2)	The "Numbering plan identification" is included in the Called party BCD number Information element of the SETUP message for an outgoing call.

An MS with MMI shall as default use the Numbering Plan Identification CCITT E164, unless otherwise indicated by the user.

3)	The "Type of number" is included in the Called party BCD number Information element of the SETUP message for an outgoing call.

An MS with MMI shall, if the "+" is not entered, and a number is entered, set the Type of Number to "unknown".

4)	The "Type of number" is included in the Called party BCD number Information element of the SETUP message for an outgoing call.

An MS with MMI shall, if the "+" is entered, and a number is entered, set the Type of Number to "International"

33.1.3	Test purpose

1)	To verify that an MS with human interface, in a SETUP message sent to originate a call, includes the same"Number digits" in the "Called party BCD number" of the SETUP message as displayed.

2)	To verify that an MS with MMI, when made to establish a call sends a SETUP message, which includes the "Numbering plan identification" in the "Called party BCD number" of the SETUP message for an outgoing call with the value "ISDN/telephony numbering plan (E.164/E.163)".

3)	To verify that an MS with MMI, when made to establish a call without use of the "+-key" function, sends a SETUP message, which includes the "Type of number" in the "Called party BCD number" of the SETUP message for an outgoing call with the value "unknown".

4)	To verify that an MS with MMI, implementing the "+-key" function, when made to establish a call with use of the "+-key" function, sends a SETUP message, which includes the "Type of number" in the "Called party BCD number" of the SETUP message for an outgoing call with the value "international number.

33.1.4	Method of test

33.1.4.1	Initial conditions

The MS is registered in a cell of the SS.

33.1.4.2	Procedure

a)	A number (not including "+ function") is entered and then a call is set up.

b)	After the SS has accepted the call the number displayed on the MS and the number received in the SS are compared.

c)	The NPI and TON are examined in the SS.

d)	Steps a) to c) are repeated, but in a), the number entered starts with the "+ function"

NOTE :	This test may also be performed automatically using the EMMI.

33.1.5	Test requirements

1)	In step b), both numbers shall be identical

2)	In step c), the NPI shall be "E164" and the TON shall be "unknown".

3)	In step d), the NPI shall be "E164" and the TON shall be "international".

33.2	Indication of call progress signals

33.2.1	Definition and applicability

33.2.2	Conformance requirement

33.2.3	Test purpose

33.2.4	Ringing tone

33.2.4.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to j).

33.2.4.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 7).

2)	The ringing tone characteristics shall be as follows:

Tone	Frequency	Tolerance	Type

Ringing tone	425 Hz	15 Hz	Periodic

tone on 1 s,

silence 4 s

33.2.5	Busy tone

33.2.5.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to h).

b)	The SS then sends message DISCONNECT with cause number 17.

Message: DISCONNECT	(GSM 04.08, 9.3.7) to the MS:

Information element	Comment

Protocol discriminator	CM

Transaction identifier	MS orig.

Message type

Cause

- Coding standard	GSM

- Location	User

- Cause value	#17 "user busy"

33.2.5.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 5).

2)	After the reception of DISCONNECT a busy tone shall be generated. The busy tone characteristics shall be as follows:

Tone	Frequency	Tolerance	Type

Busy tone	425 Hz	15 Hz	Periodic

tone on 500 ms,

silence 500 ms

33.2.6	Congestion tone

33.2.6.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to h).

b)	The SS then sends message DISCONNECT with cause number 42.

DISCONNECT message: As in 33.2.3.1 with cause value #42 "Switching equipment congestion" (0101010).

33.2.6.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 5).

2)	After the reception of DISCONNECT a congestion tone shall be generated.

3)	The congestion tone characteristics shall be as follows:

Tone	Frequency	Tolerance	Type

Congestion tone	425 Hz	15 Hz	Periodic

tone on 200 ms,

silence 200 ms

33.2.7	Authentication failure tone

33.2.7.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to e).

b)	After reception of message AUTHENTICATION RESPONSE the SS sends message AUTHENTICATION REJECT.

Message: AUTHENTICATION REJECT	(GSM 04.08, 9.2.1) to the MS:

Information element	Comment

Protocol discriminator	MM

Transaction identifier	not relevant

Message type

33.2.7.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 4).

2)	After reception of AUTHENTICATION REJECT a tone shall be generated indicating authentication failure.

3)	The authentication failure tone is the error/special information tone with characteristics as follows:

Tone	Frequency	Tolerance	Type

Error/Special	950 Hz	50 Hz	Triple tone

Information tone	1400 Hz	50 Hz	tones on 330 ms

1800 Hz	50 Hz	silence 1.0 s

33.2.8	Number unobtainable tone

33.2.8.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to h).

b)	The SS then sends message DISCONNECT with cause number 1.

DISCONNECT message: As in 33.2.3.1 with cause value #1 "Unassigned (unallocated) number" (0000001).

33.2.8.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 5).

2)	After reception of DISCONNECT a tone shall be generated indicating that the called number is unobtainable.

The number unobtainable tone is the error/special information tone with characteristics as in 33.2.5.2.

33.2.9	Call dropped tone

33.2.9.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to l). However, it shall be indicated in the system information messages that call re-establishment shall not be attempted (RACH control parameters).

b)	When the call has been established the SS stops transmitting on the TCH/SACCH.

33.2.9.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 8).

2)	After the radio link timeout period has expired a tone shall be generated indicating that the call has been dropped.

The call dropped tone characteristics shall be as follows:

Tone	Frequency	Tolerance	Type

Call dropped tone	425 Hz	15 Hz	Tone on 200 ms,

silence 200 ms

3 bursts of on/off

33.3	Network selection / indication

33.3.1	Definition and applicability

Network selection and indication is the ability of the MS to correctly select a network and display to the user in accordance with GSM 02.11.

The requirements and this test apply to all MS.

33.3.2	Conformance requirement

1)	Upon switching on, and when an IMSI is available, the MS shall select its Home PLMN and perform the cell selection procedure.

2)	If the Home PLMN is unavailable and the MS is in automatic PLMN selection mode, it shall attempt to select a suitable cell and accesses the PLMNs in turn, in the order of priority as stored in the SIM, when it looses radio coverage for its selected PLMN.

3)	The MS shall mark a PLMN, for which it has received a LOCATION UPDATING REJECT with the cause "PLMN not allowed", as forbidden on the SIM. The MS, when in automatic PLMN selection mode, shall not attempt to access a PLMN marked as forbidden on the SIM.

4)	No explicit conformance requirement. The requirement is implicit, because GSM 02.11 only describes selection of new PLMN for the following cases: 1) loss of radio coverage, and 2) search procedure at the user's request.

5)	If the MS loses radio coverage for its selected PLMN, and the MS is in automatic PLMN selection mode, it shall attempt to select its Home PLMN.

6)	The MS shall mark a PLMN, for which it has received a LOCATION UPDATING REJECT message with the cause "PLMN not allowed", as forbidden on the SIM. The MS, when in manual PLMN selection mode, shall be able to attempt to access a PLMN marked as forbidden on the SIM.

7)	If the Home PLMN is unavailable and the MS is in automatic PLMN selection mode, it shall attempt to select a suitable cell and accesses the PLMNs in turn, in the order of priority as stored in the SIM, upon switching on and when the IMSI is available.

8)	The MS shall delete a PLMN from the list of forbidden PLMNs, when a successful location update has been performed to that PLMN.

9)	Upon switching on, and when an IMSI is available, the MS shall select its Home PLMN and perform the cell selection procedure.

33.3.3	Test purpose

1)	To verify that the MS with SIM containing in the PLMN selector field at least one PLMN different from the Home PLMN, when in automatic PLMN selection mode, selects its Home PLMN, if available, upon switching on and when the IMSI is available. (This is verified by observation of the location updating procedure).

2)	To verify that the MS, when it looses radio coverage for its selected PLMN and in automatic PLMN selection mode, selects the PLMN with the highest priority among the PLMNs stored on the SIM, if the Home PLMN is unavailable.

3)	To verify that the MS, when in automatic PLMN selection mode with PLMN selector field containing a given PLMN, does not attempt to access that PLMN after having initiated a location updating procedure towards that network and having received a LOCATION UPDATING REJECT message with the cause "PLMN not allowed".

4)	To verify that the MS, when in automatic PLMN selection mode, does not select another PLMN if the selected PLMN is still available.

5)	To verify that the MS, when it looses radio coverage for its selected PLMN, and in automatic PLMN selection mode, selects its Home PLMN, if available.

6)	To verify that the MS, when in manual PLMN selection mode, is able to attempt an access to a PLMN for which it has previously received a LOCATION UPDATING REJECT with the cause "PLMN not allowed".

7)	To verify that the MS, upon switching on and when the IMSI is available, and in automatic PLMN selection mode, selects the PLMN with the highest priority among the PLMNs stored on the SIM, if the Home PLMN is unavailable.

8)	To verify that the MS again can perform automatic selection of a PLMN for which it has previously received a LOCATION UPDATING REJECT message with the cause "PLMN not allowed", when manual PLMN selection in the meantime has resulted in a LOCATION UPDATING ACCEPT message.

9)	To verify that the MS, when in manual PLMN selection mode, selects its Home PLMN, if available, upon switching on and when the IMSI is available.

33.3.4	Method of Test

Procedure 1: This procedure applies to both automatic and manual mode for PLMN selection.

Procedure 2: This procedure applies to the manual mode for PLMN selection.

33.3.4.1	Procedure 1

1a)	The MS is set up with a SIM which contains, in the "PLMN selector" data field, a list of 3 PLMN in the priority order PLMN2 (highest priority), PLMN3, PLMN4 (lowest priority). PLMN1 is the Home PLMN of the MS as defined in the IMSI. The "Forbidden PLMN" data field shall contain NULL values.

1b)	The SS transmits 4 BCCH carriers with the following parameters:

PLMN	Level dBmVemf( )

Carrier 1	PLMN1	28

Carrier 2	PLMN2	33

Carrier 3	PLMN3	38

Carrier 4	PLMN4	43

Each carrier has the "IMSI attach" (ATT) flag set in the BCCH data. (The purpose of this is to force the MS to do location updating whenever it is switched on, so that the SS can determine which PLMN has been selected).

The other system information parameters are as in table 33-1.

1c)	The MS is brought into the "on" condition with automatic selection mode active.

1d)	The SS checks that the MS sends a "location updating request" on carrier 1.

1e)	The SS sends a "location updating accept" message to the MS on carrier 1. After 5 s, the MS "selected PLMN indicator" is checked.

1f)	The SS switches off carriers 1 and 2.

1g)	The SS checks that the MS sends a "location updating request" on carrier 3.

NOTE :	Steps 1h) to 1j) test that the MS writes the forbidden PLMN to the correct location in the test SIM. The procedure will be invalid if the "forbidden PLMN" data field in the SIM does not contain NULL values at the start of the procedure.

1h)	The SS sends a "location updating reject" message on carrier 3 with the cause "PLMN not allowed".

1i)	The MS is turned off. The MS is brought into the "on" condition with automatic selection mode active.

1j)	The SS checks that the MS sends a "location updating request" on carrier 4. (PLMN3 should not be selected because it should have been written to the SIM "forbidden PLMN" data field in step 1h) above.)

1k)	The SS sends a "location updating accept" message on carrier 4. After 5 s, the MS "selected PLMN indicator" is checked.

1l)	Carriers 1 and 2 are turned on with the same parameters as in step 1b) above.

1m)	The SS checks that the MS does not send a "location updating request" on either carrier 1 or 2.

1n)	Carrier 4 is turned off. The SS then checks that the MS sends a "location updating request" on carrier 1.

1o)	The SS sends a "location updating accept" message on carrier 1. After 5 s, the MS "selected PLMN indicator" is checked.

1p)	Carrier 1 is turned off. After 30 s, the indication on the MS of available PLMN is checked.

1q)	PLMN3 is selected manually.

1r)	The SS checks that the MS sends a "location updating request" on carrier 3. (Although PLMN3 is in the "forbidden PLMN" list, it should still be able to be selected manually).

1s)	The SS sends a "location updating accept" message on carrier 3. After 5 s, the MS "selected PLMN indicator" is checked.

1t)	Carrier 3 is turned off. The "PLMN search procedure" is initiated manually. After 30 s, the indication on MS of available PLMN is checked.

1u)	The MS is turned off. Carrier 2 is turned off and carriers 3 and 4 are turned on with the parameters as in step 1b) above. The MS is brought into the "on" condition with automatic selection mode active.

1v)	The SS checks that the MS sends a "location updating request" on Carrier 3. (PLMN 3 should have been deleted from the SIM "forbidden PLMN" list in step 1u) above.)

1w)	The SS sends a "location updating accept" message on carrier 3. After 5 s, the MS "selected PLMN indicator" is checked.

1x)	Carrier 3 is turned off. The SS checks that the MS sends a "location updating request" on carrier 4.

1y)	The SS sends a "location updating reject" message on carrier 4 with the cause "location area not allowed".

1z)	The SS checks that the MS answers to the paging, with IMSI, to check that it camps on carrier 4 in accordance with GSM 05.08.

33.3.4.2	Requirements 1

Requirement 1.1) is mandatory for all MS. Requirements 1.2) and 1.3) only apply to MS with a human interface.

1.1)	The MS shall make a response as indicated in steps 1d), 1g), 1j), 1m), 1n), 1r), 1v), 1x) and 1z) above. In all cases, the MS shall respond within 30 s.

1.2)	The selected PLMN shall be indicated:

End of Step	1e)	1k)	1o)	1s)	1w)

PLMN indicated:	PLMN1	PLMN4	PLMN1	PLMN3	PLMN3

1.3)	The following PLMN shall be indicated as available:

Step 1p):	PLMN2,	PLMN3

Step 1t):	PLMN2

33.3.4.3	Procedure 2

2a)	The MS is set up with a SIM which contains NULL values in the "PLMN selector" data field. PLMN1 is the Home PLMN of the MS as defined in the IMSI. The "forbidden PLMN" data field shall contain PLMN3.

2b)	The SS transmits 4 BCCH carriers with the following parameters:

PLMN	Level dBmVemf(  )

Carrier 1	PLMN1	28

Carrier 2	PLMN2	33

Carrier 3	PLMN3	38

Carrier 4	PLMN4	43

Each carrier has the "IMSI attach" (ATT) flag set in the BCCH data. (The purpose of this is to force the MS to do location updating whenever it is switched on, so that the SS can determine which PLMN has been selected.)

The other system information parameters are as in table 33-1.

2c)	The MS is brought into the "on" condition with manual selection mode active.

2d)	The SS checks that the MS sends a "location updating request" on carrier 1.

33.3.4.4	Requirements 2

2.1)	The MS shall make a response as indicated in step 2d). The MS shall respond within 30 s.

Table 33-1 : Normal system information fields

Parameter	Reference in GSM 04.08	Abbreviation	Normal setting

Cell Channel Description	10.5.2.1	-	Any values

Max retrans	10.5.2.17	-	1

Tx-integer	10.5.2.17	-	Any value

CELL_BAR_ACCESS	10.5.2.17	CBA	0 (i.e. no barred)

AC CN	10.5.2.17	AC	All 0

RE	10.5.2.17	RE	0 (i.e. re-estab-

lishment allowed)

BA ARFCN	10.5.2.13	BA	One entry equal to the ARFCN of the carrier

NCC	10.5.2.15	NCC	Any value

Cell Identity	10.5.1.1	-	Any value

MCC, MNC	10.5.1.3	PLMN	Ref. 33.3.2, 1b) and 33.3.2, 2b)

LAC	10.5.1.3	LAC	1111 (Hex)

ATT, B_AG_BLKS_RES,T3212,	10.5.2.8	-	ATT = '1'

CCCH_CONF			Other parameters any values.

BS_PA_MFRMS	10.5.2.8	BPM	5 frames

Cell Options	10.5.2.3	-	Any values

CELL_RESELECT_HYSTERESIS	10.5.2.4	CRH	10 dB

MS_TXPWR_MAX_CCH	10.5.2.4	MTMC	Maximum RF output power of MS.

RXLEV_ACCESS_MIN	10.5.2.4	RAM	-95 dBm

33.4	Invalid and blocked PIN indicators

33.4.1	Definition and applicability

The requirements and this test apply to all MS.

33.4.2	Conformance requirement

33.4.3	Test purpose

33.4.4	Method of test

33.4.4.1	Initial conditions

The MS contains a SIM with the PIN enabled, and the SIM unblocking counter set to zero by previous presentation of the personal unblocking key.

33.4.4.2	Procedure

a)	The MS is switched on.

b)	Three wrong PIN are entered.

Activation may be either manual or via the EMMI.

33.4.5	Test requirements

For the first and second incorrect PIN the MS shall indicate that the PIN code has been rejected

For the third incorrect PIN the MS shall indicate that the PIN is blocked.

33.5	Service indicator

33.5.1	Definition and applicability

The requirements and this test apply to all MS.

33.5.2	Conformance requirement

33.5.3	Test purpose

33.5.4	Method of test

33.5.4.1	Initial conditions

a)	The MS is in idle mode, unregistered.

b)	The SS shall emulate perfect radio conditions so that the MS is able to register and to set up or receive a call.

33.5.4.2	Procedure

a)	The MS is brought in an active state by either switching it on or by inserting a SIM.

33.5.5	Test requirements

1)	The successful registration and the good condition shall be indicated by the MS indicator and by the SS.

33.6	Subscription identity management

33.6.1	Definition and applicability

Subscription identity management is the ability of the MS to prevent the establishment of MO (except MO emergency calls) and MT calls without a valid subscription.

The requirements and this test apply to all MS.

33.6.2	Conformance requirement

An MS can only be operated, if a valid IMSI is present.

33.6.3	Test purpose

To verify that an MS without SIM card will not accept an incoming call.

33.6.4	Method of test

33.6.4.1	Initial conditions

33.6.4.2	Procedure

a)	A call is set up.

b)	(Reserved).

c)	Either

(i)	The SIM is removed.

(ii)	Where this is not possible, the MS is powered down, the SIM is removed and the MS is powered on.

The SS observes whether or not the MS performs IMSI detach.

d)	An attempt to establish a MO call is made (not an emergency call).

e)	An attempt to establish a MT call is made.

33.6.5	Test requirements

1)	In step c(i), the MS shall perform an IMSI detach.

In step c(ii), the MS may perform an IMSI detach.

2)	In step d) the MS shall not attempt to set up a new call via the Um interface.

3)	In step e), the MS shall not attempt to set up a new call via the Um interface.

33.7	Barring of outgoing calls

33.7.1	Definition and applicability

The barring of outgoing calls is an optional feature. It is the ability of the MS to prevent all MO calls except emergency calls.

The requirements and this test apply to all MS supporting this feature.

33.7.2	Conformance requirement

An MS may have an optional facility to bar outgoing calls. Such barring facility shall not prevent the transmission on emergency calls.

33.7.3	Test purpose

To verify that an MS for which a local facility to bar outgoing calls has been declared as being implemented, is able to establish an emergency call if this facility is activated.

33.7.4	Method of test

33.7.4.1	Initial conditions

33.7.4.2	Procedure

a)	The local facility to bar outgoing calls is activated.

b)	Via MMI, the MS is actioned to establish an emergency call.

33.7.5	Test requirements

1)	The MS shall establish an emergency call.

33.8	Prevention of unauthorized calls

33.8.1	Definition and applicability

The prevention of unauthorized calls is an optional feature in the MS. It is the ability of the MS to prevent unauthorized use by using a key or keyword protection facility. When activated the MS does not prevent the establishment of except emergency calls.

The requirements and this test apply to all MS supporting this feature.

33.8.2	Conformance requirement

An MS may have an optional facility to prevent unauthorized use. Such facility shall not prevent the transmission on emergency calls.

33.8.3	Test purpose

To verify that an MS for which a local facility to prevent unauthorized use has been declared to be implemented, is able to establish an emergency call, if this facility is activated.

33.8.4	Method of test

33.8.4.1	Initial conditions

33.8.4.2	Procedure

a)	The local facility to restrict operation such that the MS can only be operated by using a key or a keyword is activated. The most restrictive situation is created.

b)	Via MMI, the MS is actioned to establish an emergency call.

33.8.5	Test requirements

1)	The MS shall establish an emergency call.