Tab set to 1.5cm

Annex 4 : Test SIM Parameters

A4.1.	Introduction

This section defines default parameters for programming the elementary files of the test SIM. The requirements of this annex do not apply to the SIM/ME tests of section 27.

A4.1.1	Definitions

"Test SIM card":

A SIM card supporting the test algorithm for authentication, programmed with the parameters defined in this section. The electrical, mechanical and environmental requirements of the test SIM card are specified in GSM 11.11.

"Test SIM":

Either a test SIM card or the SIM simulator programmed with the parameters defined in this section.

A4.1.2	Definition of the test algorithm for authentication

The following procedure employs bit wise modulo 2 addition ("XOR")

The following convention applies:

In all data transfer the most significant byte is the first byte to be sent; data is represented so that the left most bit is the most significant bit of the most significant byte.

Step 1:

XOR to the challenge RAND, a predefined number Ki, having the same bit length (128 bits) as RAND. The result RES1 of this is

RES1 = RAND XOR Ki

Step 2:

the most siginificant 32 bits of RES1 form SRES. the next 64 bits of RES1 form Kc. The remaining 32 bits are not used.

A4.2	Default Parameters for the test SIM

Ki:

The authentication key "Ki" will be chosen by the test house and will be non zero. The "Ki" value used by the SS will align with this value.

PIN Disabling

The PIN enabled / disabled flag will be set to "PIN Disabled". This ensures that when the Test SIM is inserted into a MS the user will not be prompted for PIN entry. This requires a specific card capability defined by the SIM service table (see section 2.9).

A4.3.	Default settings for the Elementary Files (EFs).

The format and coding of elementary files of the SIM are defined in GSM 11.11. The following sections define the default parameters to be programmed into each elementary file. Some files may be updated by the MS based on information received from the SS. These are identified in the following sections.

A4.3.1	EFICCID (ICC Identification)

The programming of this EF is a test house option.

A4.3.2	EFLP (Language preference)

The programming of this EF is a test house option.

A4.3.3	EFIMSI (IMSI)

The IMSI value will be chosen by the test house. The IMSI used by the SS will align this value.

File size:	9 bytes

Default values:	Byte 1 (DEC):	8

Bytes 2-9 (HEX):	09 10 10 ** ** ** ** **

"*" indicates any number between 0 and 9 subject to the restriction that IMSI mod 1000 (i.e bytes 7, 8 and 9) lies in one of the the following ranges:

063-125, 189-251, 315-377, 441-503, 567-629, 693-755, 819-881 or 945-999

NOTE :	This ensures that the MS can listen to the second CCCH when more than one basic physical channel is configured for the CCCH. This is necessary for the test of "paging re-organisation".

A4.3.4	EFKc (Ciphering key Kc)

File size:	9 Bytes

Default values (HEX):	Bytes 1-8:	Align with Kc used by SS

Byte 9:	Align with CKSN used by SS

The bytes within this elementary file may be updated by the MS as a result of a successful authentication attempt.

A4.3.5	EFPLMNsel (PLMN selector)

File size:			3n (n>/=8) bytes

Default values (HEX):	Bytes 1-3:		00 F1 10	(MCC, MNC) - Translates to 001, 01

Bytes 4-6:		FF FF FF	(MCC, MNC)

Bytes 7-9:		FF FF FF	(MCC, MNC)

Bytes 10-12:		FF FF FF	(MCC, MNC)

Bytes 13-15:		FF FF FF	(MCC, MNC)

Bytes 16-18:		FF FF FF	(MCC, MNC)

Bytes 19-21:		FF FF FF	(MCC, MNC)

Bytes 22-24:		FF FF FF	(MCC, MNC)

.			.		.

.			.		.

Bytes (3n-2)-3n:	FF FF FF	(MCC, MNC)

A4.3.6	EFHPLMN (HPLMN search period)

File size:			1 byte

Default value (HEX):	00 (no HPLMN search attempts)

A4.3.7	EFACMmax (ACM maximum value)

[TBD]

A4.3.8	EFSST (SIM service table)

Services will be allocated and activated as follows:

Service		Allocated	Activated

No. 1: CHV1 disable function	Yes	Yes

No. 2: Abbreviated Dialling numbers (ADN)	*	*

No. 3: Fixed dialling numbers (FDN)	*	*

No. 4: Short Message Storage (SMS)	Yes	Yes

No. 5: Advice of Charge (AoC)	[TBD]	*

No. 6: Capability Configuration Parameters (CCP)	*	*

No. 7: PLMN Selector	Yes	Yes

No. 8: Reserved for future use	-	-

No. 9: MSISDN		*	*

No. 10: Extension 1	*	*

No. 11: Extension 2	*	*

No. 12: SMS Parameters	*	*

No. 13: Last Dailled Number (LND)	*	*

No. 14: Cell Broadcast Message Identifier	*	*

NOTE:	"*" means that the programming of this bit is a test house option.

A4.3.9	EFACM (Accumulated call meter)

[TBD]

A4.3.10	EFPUCT (Price per unit and currency table)

[TBD]

A4.3.11	EFCBMI (Cell broadcast Message Identifier Selection)

[TBD]

A4.3.12	EFBCCH (Broadcast control channels)

File size:			16 Bytes

Default values (BIN):	Bytes 1-2:		11111111 11111111

Bytes 3-4:		11111111 11111111

Bytes 5-6:		11111111 11111111

Bytes 7-8:		11111111 11111111

Bytes 9-10:		11111111 11111111

Bytes 11-12:		11111111 11111111

Bytes 13-14:		11111111 11111111

Bytes 15-16:		11111111 11111111

This field may be updated dependent on the MS implementation.

A4.3.13	EFACC (Access control class)

File size:			2 Bytes

Default values (BIN):	Byte 1:		00000000

Byte 2:		********

The test house may set any single bit of byte 2 to "1". All remaining bits of byte 2 will be set to "0". This determines the access control class of the SIM.

A4.3.14	EFFPLMN (Forbidden PLMNs)

Length:			12 Bytes

Format (HEX):		Bytes 1-3:		FF FF FF

Bytes 4-6:		FF FF FF

Bytes 7-9:		FF FF FF

Bytes 10-12:		FF FF FF

This coding corresponds to an empty "forbidden PLMN list". The bytes within this file may be updated if a LOCATION UPDATE REJECT message is received by the MS with cause, "PLMN not allowed".

A4.3.15	EFLOCI (Location information)

File size:			11 Bytes

Default values:			Bytes 1-4 (HEX):	** ** ** ** (TMSI)

Bytes 5-9 (HEX):	** ** ** ** ** (LAI)

Byte 10 (HEX):	FF (Periodic LU Time = "the timer is not running")

Byte 11 (BIN):	00000000 (Location Update Status = "updated")

* indicates that the values will align with the values used by the SS.

Bytes in this file may be updated as a result of a location update attempt by the MS.

A4.3.16	EFAD (Administrative data)

File size:			3+X bytes

Default values			Byte 1:		10000000	- (type approval operations)					Byte 2:		11111111

Byte 3:		11111111

Bytes 4 - (3+X):	Reserved for future use.

A4.3.17	EFPhase (Phase identification)

File size:			1 byte

Default value (HEX):	02		Phase 2

A4.3.18	EFADN (Abbreviated dialling numbers)

[TBD]

A4.3.19	EFFDN (Fixed dialling numbers)

[TBD]

A4.3.20	EFSMS (Short messages)

Default:	Records 1-5	Byte 1:		00

Byte 2:		FF

Bytes 3-14:	FF FF FF FF FF FF FF FF FF FF FF FF

Bytes 15-26:	FF FF FF FF FF FF FF FF FF FF FF FF

Byte 27:	FF

Byte 28:	FF

Bytes 29-35:	FF FF FF FF FF FF FF

Byte 36:	FF

Bytes 37-176:	All Bytes set to FF

A4.3.21	EFCCP (Capability configuration parameters)

[TBD]

A4.3.22	EFMSISDN (MSISDN)

[TBD]

A4.3.23	EFSMSP (Short message service parameters)

[TBD]

A4.3.24	EFSMSS (SMS status)

File size: 2+X bytes

Byte 1:		* * * * * * * *

Byte 2 (BIN):	11111111

Bytes 3 - (2+X):	RFU

A4.3.25	EFEXT1 (Extension 1)

[TBD]

A4.3.26	EFEXT2 (Extension 2)

[TBD]