# 1	Scope

This Interim European Telecommunications Standard (I-ETS) describes the technical characteristics and methods of test for Mobile Stations (MS), for the Pan European digital cellular communications system and Personal Communication Systems (PCS) operating in the 900 MHz and 1800 MHz band (GSM900 and DCS1800), standardized by ETSI Technical Committee Special Mobile Group (SMG).

This I-ETS with methods of test is produced as a draft I-ETS. A subset of the tests is referenced in the GSM Common Technical Regulations (CTRs) and is used for regulatory conformance testing according to the EEC procedures for Telecommunications Terminal Equipment (TTE) type approval (EC Directive 91/263/EEC; also known as the "Terminal Directive" or "Second Phase Directive"). The remaining tests can be used to verify conformance with the GSM core technical specifications for those requirements that are not considered "essential" in the sense of the EC Directive 91/263/EEC (Article 4).

This I-ETS covers the minimum characteristics considered necessary in order to provide sufficient performance for mobile equipment and to prevent interference to other services or to other users, and to the PLMNs.

It does not necessarily include all the characteristics which may be required by a user or subscriber, nor does it necessarily represent the optimum performance achievable.

It applies to the public land mobile radio service in the GSM900 and DCS1800 systems, using constant envelope modulation and operating on radio frequencies in the 900 and 1800 MHz bands respectively with a channel separation of 200 kHz and carrying 8 full rate channels or 16 half rate channels per carrier according to the TDMA principle.

This I-ETS is part of the GSM-series of technical specifications. This I-ETS neither replaces any of the other GSM technical specifications or GSM related ETS, nor is it created to provide full understanding of (or parts of) the GSM900 and DCS1800 systems. This I-ETS lists the requirements, and provides the methods of test for testing a MS for conformance to the GSM standard.

For a full description of the system, reference should be made to all the GSM technical specifications or GSM related ETSs. Clause 2 provides a complete list of the GSM technical specifications, GSM related ETSs, and ETRs, on which this conformance test specifications is based.

This I-ETS applies to the unit which includes the hardware to establish a connection across the radio interface.

If there is a difference between this conformance test I-ETS, and any other GSM technical specification or GSM related ETS, then the other GSM technical specification or GSM related ETS shall prevail.

# 2	Normative references

This I-ETS incorporates, by dated and undated reference, provisions from other publications. These normative references are cited at the appropriate places in the text and the publications are listed hereafter. For dated references subsequent amendments to or revision of any of these publications apply to the requirements specified in this I-ETS, only when incorporated in it by amendment or revision. For undated references the latest edition of the publication referred to applies.

The following list contains the references to the core phase 2 GSM technical specifications to (part of) which conformance is explicitly tested in this I-ETS.

Number	Title

GSM

02.02	Bearer Services Supported by a GSM PLMN

02.03	Teleservices Supported by a GSM PLMN

02.04	Description of Supplementary Services

02.06	Types of Mobile Stations

02.07	Mobile Station Features

02.09	Security Aspects

02.11	Service Accessibility

02.16	International MS Equipment Identities

02.17	Subscriber Identity Modules, Functional Characteristics

02.30	Man-machine Interface of the Mobile Station

02.40	Procedures for Call Progress Indications

02.82	Call Offering Supplementary Services

02.88	Call Restriction Supplementary Services

03.03	Numbering, Addressing and Identification

03.05	Technical performance objectives

03.10	GSM PLMN Connection Types

03.13	Discontinuous Reception (DRX) in the GSM System

03.14	Support of DTMF via the GSM System

03.20	Security-related Network Functions

03.22	Functions related to MS in idle mode

03.40	Technical Realization Short Message Service Point-to-point

03.41	Technical Realization of Short Message Service Cell Broadcast

03.43	Technical Realization of Videotex

03.44	Support of Teletex in a GSM PLMN

03.45	Technical Realization of Facsimile Group 3 Service - transparent

03.46	Technical Realization of Facsimile Group 3 Service - non transparent

03.50	Transmission Planning Aspects of the Speech Service in the GSM PLMN System

04.01	MS-BSS Interface - General Aspects and Principles

04.02	GSM PLMN Access Reference Configuration

04.03	MS-BSS Interface : Channel Structures and Access Capabilities

04.04	MS-BSS Layer 1 - General Requirements

04.05	MS-BSS Data Link Layer - General Aspects

04.06	MS-BSS Data Link Layer Specification

04.07	Mobile Radio Interface Signalling Layer 3 - General Aspects

04.08	Mobile Radio Interface - Layer 3 Specification

04.10	Mobile Radio Interface Layer 3 - Supplementary Services Specification - General Aspects

04.11	Point-to-point Short Message Service Support on Mobile Radio Interface

04.12	Cell Broadcast Short Message Service Support on Mobile Radio Interface

04.21	Rate Adaptation on MS-BSS Interface

04.22	Radio Link Protocol for Data and Telematic Services on the MS-BSS Interface

04.80	Mobile Radio Interface Layer 3 - SS Specification - Formats and Coding

04.82	Mobile Radio Interface Layer 3 - Call Offering SS Specification

04.88	Mobile Radio Interface Layer 3 - Call Restriction SS Specification

05.01	Physical Layer on the Radio Path (General Description)

05.02	Multiplexing and Multiple Access on the Radio Path

05.03	Channel Coding

05.04	Modulation

05.05	Radio Transmission and Reception

05.08	Radio Subsystem Link Control

05.10	Radio Subsystem Synchronization

06.01	Speech Processing Functions : General Description

06.10	GSM Full Rate Speech Transcoding

06.11	Substitution and Muting of Lost Frames for Full-rate Speech Traffic Channels

06.12	Comfort Noise Aspects for Full Rate Speech Traffic Channels

06.31	Discontinuous Transmission (DTX) for Full Rate Speech Traffic Channels

06.32	Voice Activity Detection

07.01	General on Terminal Adaptation Functions for MSs

07.02	Terminal Adaptation Functions for Services Using Asynchronous Bearer Capabilities

07.03	Terminal Adaptation Functions for Services Using Synchronous Bearer Capabilities

09.02	Mobile Application Part Specification

11.11	Specification of the Internal Logical Organization of the SIM and its Interfaces

12.10	Maintenance Provisions for Operational Integrity of MSs]

The following list contains the references to the non-GSM technical specifications to (part of) which conformance is tested in this specification.

[RES 09.03

Others?]

# 3	Definitions, conventions, and applicability

For abbreviations and acronyms, see GSM 01.04.

## 3.1	Mobile station definition and configurations

In this I-ETS, a MS can be:

-	a vehicle mounted station;

-	a portable station;

-	a handheld station;

-	a vehicle mounted/portable station;

-	a vehicle mounted/handheld station.

A MS is the complete equipment configuration which may take part in a communication. However, this may not be the MS as it is offered to a test house for conformance testing.

In general, the MS, as it will be presented to a test house for conformance testing, is the station without all the additional Terminal Equipment (TE). Such a piece of hardware is also called a Mobile Termination (MT), but in this I-ETS, the expression MS is used for any form of MS hardware as it is offered to the test house.

| Mobile | Interfaces, reference points: S, R, etc.. | Terminal |
| --- | --- | --- |
| Termination |  | Equipment |

Figure 3-1

During the tests, the interfaces of the MT shall be connected to a System Simulator (SS), which will also emulate the TE. For some tests, it may be necessary to establish a pre-configured setup of the MS.

EXAMPLE:	For reception of automatic fax group 3 to a fax machine on the R-interface, the MS needs configuration information about the presence of such a machine on that interface.

As an alternative, the TE may be physically integrated.

For a more detailed description of MS-configurations, see GSM 02.06.

## 3.2	Applicability

### 3.2.1	Applicability of this specification

If a MS is equipped with a connector, to connect terminal equipment on an S or R interface, then testing of the MS may include testing of appropriate functioning to and from this connector.

This I-ETS does not apply to TE which is to be connected to that connector (which constitutes a public interface), even if it is delivered with the MS.

### 3.2.2	Applicability of the individual tests

The applicability of each individual test is identified in the definition and applicability subclause of the test.

### 3.2.3	Applicability to terminal equipment

If a MS is delivered for conformance testing, and it contains physically integrated TE, then this I-ETS applies to the complete MS including that TE.

This I-ETS also applies to separate TE that is delivered for conformance testing with the MS. The MS is then tested as an MT0. In that case, the specific TE with which the MS is tested is documented in the test report.

## 3.3	Definitions

The following definitions are used in this I-ETS:

idle updated:

the MS is defined to be "idle updated" if the following three conditions are fulfilled:

-	its update status is U1 UPDATED (cf. TS GSM 04.08);

-	it is in the MM state MM-IDLE (cf. TS GSM 04.08);

-	it is in the RR idle mode (cf. TS GSM 04.08).

idle not updated:

the MS is defined to be "idle not updated" if the following three conditions are fulfilled:

-	its update status is U2 NOT UPDATED (cf. TS GSM 04.08);

-	it is in the MM state MM-IDLE (cf. TS GSM 04.08);

-	it is in the RR idle mode (cf. TS GSM 04.08).

arbitrary:

if for a test, a test purpose, a test group, or a test suite, which uses a certain parameter the value of that parameter has to be chosen arbitrarily in a certain set of values, this means that:

-	for each value in the set the MS is required to fulfil the requirements of the test, test purpose, test group, or  test suite, but that

-	the test, test purpose, test group, or  test suite is only performed for one value in the set, the selection of which is made by the test operator.

## 3.4	Conventions for mathematical notations

For the purpose of this I-ETS mathematical terms used throughout this I-ETS are given in this subclause.

### 3.4.1	Mathematical signs

The "plus or minus" sign is expressed by "+/-".

The sign "multiplied by" is expressed by "*".

The sign "divided by" is expressed by "/", or the common division bar.

The sign "greater than or equal to" is expressed by ">=".

The sign "less than or equal to" is expressed by "<=".

### 3.4.2	Powers to the base 10

Powers to the base 10 are expressed by "10Ex", where x is the exponent, e.g. 10E-5, 10E6.

## 3.5	Conventions on electrical terms

### 3.5.1	Radio Frequency (RF) input signal level

In general, the RF input signal level to the MS is expressed in terms of the received field strength E in dBV/m (assuming a 0 dBi gain antenna). This is related to the power level P in dBm by the following formula (see GSM 05.05)

GSM900:	E (dBV/m) = P (dBm) + 136.5 (calculated for a frequency of 925 MHz).

DCS1800:	E (dBV/m) = P (dBm) + 142.3 (calculated for a frequency of 1795 MHz).

According to Annex 1 subclause A1.1.5.3, in all tests in which a handheld MS normally only equipped with integral antenna is the unit under test, the equivalent input signal level into a temporary test connector is determined from:

Ein 	=	Ereq + F

where:	Ein	= input signal level to a temporary antenna connector (dBVemf);

Ereq	= signal level required by the test (dBVemf);

F	= coupling factor (dB) at the respective ARFCN.

Since F has to be determined by each test house individually, Ein cannot be given as a figure in test procedures.

If the case of integral antenna is applicable, the input signal level is then expressed in the test procedures as:

Ereq dBVemf(  ),

where the empty parenthesis is to be read as Ein.

Alternatively, the input signal level to the MS at the antenna connector can be expressed in dBVemf(  ). This is related to the power level P in dBm by the following formula, assuming a 50 ohm antenna connector:

Input signal level (dBVemf(  )) = P(dBm) + 113

### 3.5.2	Reference sensitivity level

In this I-ETS the term:

Reference Sensitivity level ( )

is used to indicate that the SS establishes reference sensitivity level taking account of any loses associated with the RF connection to the MS.

### 3.5.3	Power level of fading signal

The power level of a fading signal is defined as the total signal level averaged over time.

## 3.6	Terms on test conditions

### 3.6.1	Radio test conditions

The radio propagation conditions refer to multipath propagation models of GSM 05.05.

They are expressed by typical profiles:

-	static

-	rural area	(RA);

-	hilly terrain	(HT);

-	urban area	(TU); or for

-	equalisation test	(EQ).

The non-static profiles are also related to typical speeds of movement of the MS expressed in km/h, e.g. TU1.5, TU3, TU50, HT100, EQ50.

In this I-ETS the following conventions are used:

Table 3.1

Term	for GSM900	for DCS1800
	represents	represents

RA	RA250	RA130
HT	HT100	HT100
TUhigh	TU50	TU50
TUlow	TU3	TU1.5
EQ	EQ50	EQ50

For tests using ARFCN ranges the following table shall be used.

Table 3.2

Term		P-GSM900	DCS1800	E-GSM900

Low ARFCN range	1 to 5	513 to 523	975 to 980
	Mid ARFCN range	60 to 65	690 to 710	60 to 65
	High ARFCN range	120 to 124	874 to 884	120 to 124

NOTE 1:	For definitions of P-GSM900, DCS1800 and E-GSM900 refer to GSM 05.05.

NOTE 2:	In this I-ETS the term 'GSM900' is used to cover the primary GSM band and the extended GSM band.

# 4	Test Equipment

## 4.1	Terms used to describe test equipment in this I-ETS

In order to perform MS conformity testing, the use of test equipment is necessary to provide the MS with stimulus signals and to analyse and record the resulting responses.

Throughout this I-ETS the term "System Simulator" is used to describe the suite of test equipment required to interact with the following MS interfaces:

-	antenna

-	acoustic

-	data port

-	power supply

-	DAI

The term "SIM simulator" is used to describe the test equipment required to interact with the SIM/ME interface.

A "test SIM" has the physical characteristics of a standard SIM card, (see 11.11) with specific parameters defined in Annex 4.

## 4.2	Functional requirements of test equipment

This I-ETS does not include a functional description of the test equipment required to perform the tests. These requirements should be deduced from the test descriptions and the information in Annex 5.

Annex 5 describes the requirements for the test equipment which cannot be derived from, and which are assumed in, the conformance test descriptions described in this I-ETS. Specifically, stimulus setting and measurement uncertainty requirements are defined in Annex 5.

# 5	Testing methodology in general (layers 1, 2, and 3)

## 5.1	Testing of optional functions and procedures

Any function or procedure which is optional, as indicated in this I-ETS , may be subject to a conformance test if it is implemented in the MS.

A declaration by the apparatus supplier (PICS/PIXIT) is used to determine whether an optional function/procedure has been implemented.

## 5.2	Test interfaces and facilities

The air interface (Um reference point) provides the main test interface for the purpose of performing conformance tests.

The SS layer 2 and layer 3 shall react with the MS on the air interface in accordance with the BSS requirements in the GSM 04.xx and 05.xx series recommendations, except where the description defines otherwise.

The provision of the following special conformance test facilities is mandatory where applicable:

-	support of special conformance test functions, which are enabled by the insertion of a dedicated SIM for testing (test-SIM);

-	provision of a Digital Audio Interface (only for MS which support speech services, or alternate speech/data services).

-	for equipment which does not have a permanent external 50 ohm connector, a temporary 50 ohm antenna connector shall be provided in accordance with the requirements of Annex 1 GC7.

-	for MS supporting diversity, or for any other reason having more than one RF connector (or temporary connector in the case of integral antenna MS) the manufacturer shall supply coupling and / or terminating devices so that the tests can be performed via a single transmit / receive RF connection.

Furthermore, an optional Electrical Man Machine Interface (EMMI), is specified.

These special conformance test facilities, with the exception of the temporary antenna connector, are described in subclause 36.1.

Actions at the user side of the equipment under test (i.e. at the man-machine Interface, at the S- or R- interface, at the SIM-interface, execution of higher layer processes in the case of data services) are used to invoke actions at layers 1, 2 and 3 of the Dm-channel protocol within the equipment under test.

## 5.3	Different protocol layers

The conformance tests for each layer of the Dm-channel protocol are specified separately and the test configuration(s) to be used in testing each layer is specified in the subclause of this I-ETS relating to the conformance tests for that layer.

## 5.4	Information to be provided by the apparatus supplier

The apparatus supplier shall provide two kinds of information:

-	information with respect to the protocol: Protocol Implementation Conformance Statement (PICS);

-	information with respect to the man machine interface: Protocol Implementation Extra Information required for Testing (PIXIT).

The complete list of the information to be provided by the apparatus supplier is a matter between the apparatus supplier and the test house but an example of the information to be supplied is given in informative Annex 3 of this I-ETS.

## 5.5	Definitions of transmit and receive times

The time a burst is received or transmitted is defined to be in the middle of the burst, i.e. transition from Bit Number BN74 to BN75 for all bursts except random access bursts, the middle of which is the transition from BN48 to BN49.

The reception/transmission time of speech or data blocks or a signalling frame (layer 2 and layer 3) is defined to be the reception/transmission time of the last burst containing part of the block or frame.

The start of a layer 2 or 3 frame is defined to be the time of the first burst containing part of the layer 2 or 3 frame. (The time of a burst is defined to be in the middle of the burst.)

The end of a layer 2 or 3 frame is defined to be the time of the last burst containig part of the layer 2 or 3 frame.

# 6	Reference test methods.

## 6.1	General

Annex 1 gives reference test conditions to be used throughout this I-ETS, unless otherwise specified. It consists of a part on general conditions, and a part on normal and extreme test conditions.

Unless otherwise specified, tests are run using the normal test conditions.

If a test is to be run using the extreme test conditions then this is identified in the test description.

For all tests, the MS is connected to the SS. This connection, unless otherwise specified, is to the permanent antenna connector for a MS which is equipped with one, or via the temporary antenna connector defined in Annex 1, GC7, for a MS with an integral antenna, and not normally having a means of connecting an external antenna.

## 6.2	Choice of frequencies in the frequency hopping mode

For the tests using frequency hopping, 38 frequencies are used over

P-GSM900:		a 21 MHz band

E-GSM900:		a [21] MHz band

DCS1800:		a 75 MHz band

Table 6.1: Hopping frequencies

ARFCN

P-GSM900	10, 14, 17, 18, 22, 24, 26, 30, 31, 34, 38, 42, 45, 46, 50, 52, 54, 58,
		59, 62, 66, 70, 73, 74, 78, 80, 82, 86, 87, 90, 94, 98, 101, 102, 106,
		108, 110, 114

E-GSM900	984, 988, 991, 992, 996, 998, 1000, 1004, 1005, 1008, 1012, 1016, 1019
		1020, 1022, 2, 6, 10, 14, 17, 18, 22, 24, 26, 30, 31, 34, 38, 42, 45, 46
		50, 52, 54, 58, 59, 62, 64

DCS1800	522, 539, 543, 556, 564, 573, 585, 590, 606, 607, 624, 627, 641, 648,
		658, 669, 675, 690, 692, 709, 711, 726, 732, 743, 753, 760, 774, 777,
		794, 795, 811, 816, 828, 837, 845, 858, 862, 879

NOTE:	The range of frequencies available during tests under simulated fading conditions is restricted by the fading simulator bandwidth.

## 6.3	"Ideal" radio conditions

In this I-ETS the following conditions are referenced by the term "ideal" radio conditions:

No multipath conditions

MS power control level:

GSM900:	7

DCS1800:	3

RF level to MS:	63 dBVemf( )	(not tests in subclause 14.4 or 18.1.4)

RF level to MS:	20 dB above reference sensitivity level ( )	(subclause 14.4)

RF level to MS:	28 dBVemf( )	(tests in subclause 18.1.4)

## 6.4	Standard test signals.

The standard test signals C0, C1, I0, I1 and I2 as used in this I-ETS, are defined in Annex 5.

# 7	Implicit testing

For some GSM features conformance is not verified explicitly in this I-ETS. This does not imply that correct functioning of these features is not essential, but that these are implicitly tested to a sufficient degree in other tests. Examples for implicitly tested features are frequency hopping and encryption.

It should be noted that for these features some aspects have to be and are explicitely tested, e.g. the ability to switch to frequency hopping or non-hopping, and the ability to change the encryption mode setting.

# 8	Measurement uncertainty

Annex 2 (normative) addresses the impact of measurement uncertainty on test results, and the relation between measurement result including uncertainty and the conformance requirement.

# 9	Format of tests.

In general the following basic format for tests is used:

*.*..*	Title

*.*..*.1		Definition and applicability

This sections provides, if necessary, a definition of the feature/function being tested and the applicability of the test to different MS (e.g speech only, data only etc).

*.*..*.2		Conformance requirement

This section details the core specification requirements being tested and includes any necessary core specification references.

*.*..*.3		Test purpose

This section details the purpose of the test.

*.*..*.4		Method of test

*.*..*.4.1		Initial conditions

If present this section defines the initial conditions to be established before running the test.

*.*..*.4.2		Procedure

This section details the test procedure.

*.*..*.5		Test requirements

This section details the conditions to be met for successful completion of the test.

[However for the Layer 3 tests, in general, a slightly modified format, as described below, is used:

*.*..*	Title

*.*..*.1		Conformance requirement

References

*.*..*.2		Test purpose

*.*..*.3		Method of test

Initial conditions

Related PICS/PIXIT statement

Foreseen final state of the MS

Test Procedure

Maximum duration of the test

Expected sequence]