## 26.6	Test of the elementary procedures for radio resource management

NOTE 1:	For SS implementor : if tests are concatenated, it is important that unused fields in IMMEDIATE ASSIGNMENT REJECT messages do not use Request References that relate to CHANNEL REQUEST messages recently transmitted by the MS.

### 26.6.1	Immediate assignment

The immediate assignment procedure is used by the network to establish a dedicated control channel for the MS and network to communicate the detail of the service requested. If the Mobile Station does not implement the procedure correctly, radio resources can be wasted as the Mobile Station might use the wrong channels.

#### 26.6.1.1	Immediate assignment / SDCCH or TCH assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.3 in phase 1 version.

-	Timing Advance is no longer specified]

##### 26.6.1.1.1	Conformance requirement

1.	Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on the SDCCH/8 described in the IMMEDIATE ASSIGNMENT message.

2.	Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on the TCH/FACCH described in the IMMEDIATE ASSIGNMENT message.

Reference(s)

GSM 04.08 section 3.3.1.2.1

##### 26.6.1.1.2	Test purpose

To verify that the MS can correctly set up a dedicated SDCCH control channel and that the MS can correctly set up a dedicated TCH/FACCH control channel.

##### 26.6.1.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, except that CCCH-CONF is set to '1 basic physical channel used for CCCH not combined with SDCCHs'.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statement(s)

Type of MS (P-GSM900 or EGSM or DCS1800).

Support rate(s) of TCH: TCH/F and/or TCH/H.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST message the SS assigns an SDCCH. The MS shall go to the correct channel and send a PAGING RESPONSE message. Then the SS initiates RR-release by sending a CHANNEL RELEASE message.

If TCH/F is supported by the MS, the test is repeated with the SS assigning a TCH/F.

If TCH/H is supported by the MS, the test is repeated with the SS assigning a TCH/H.

Maximum Duration of Test

6 seconds per value of the execution timer

Expected Sequence

This sequence is performed for execution counter, K = 1, 2, 3 (unless the TCH is not supported).

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	Channel Type : see below
	4	MS -> SS	PAGING RESPONSE	Shall be sent on the correct channel
	5	SS -> MS	CHANNEL RELEASE

Specific Message Contents

IMMEDIATE ASSIGNMENT

K=1, SDCCH test : Channel Type = SDCCH/8

K=2, TCH/F test : Channel Type = Bm + ACCHs

K=3, TCH/H test : Channel Type = Lm + ACCHs, subchannel randomly drawn

#### 26.6.1.2	Immediate assignment / extended assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.5 in phase 1 version.

-	two procedures have been concatenated to one test.

-	in the last part of the test the MS shall continue to send CHANNEL REQUESTs]

-

]

NOTE 2:	In these tests the SS must send the immediate assignment messages in due time to allow for the MS to receive them and send a PAGING RESPONSE rather than another random access. This applies to the whole of section 26.

##### 26.6.1.2.1	Conformance requirements

1.	The MS shall go to the allocated SDCCH/4 and send a PAGING RESPONSE message containing its identity and its classmark.

2.	The MS shall go to the allocated SDCCH/8 and send a PAGING RESPONSE message containing its identity and its classmark.

3.	The MS shall correctly identify its own assignment in either the Request Reference 1 or the Request Reference 2 information element in an extended assignment message.

4.	The MS shall only react to an Immediate Assignment which references one of the last 3 CHANNEL REQUEST messages from the MS.

References

GSM 04.08 section 3.3.

##### 26.6.1.2.2	Test purpose

To verify that the MS goes to the allocated SDCCH/4 and sends a PAGING RESPONSE message containing its identity and its classmark.

To verify that the MS goes to the allocated SDCCH/8 and sends a PAGING RESPONSE message containing its identity and its classmark.

To verify that the MS can correctly identify its own assignment in either the Request Reference 1 or the Request Reference 2 information element in an extended assignment message.

To verify that the MS only reacts to an Immediate Assignment which references one of the last 3 CHANNEL REQUEST messages from the MS.

##### 26.6.1.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-retrans is set to 7.

Mobile Station:

The MS is in the "idle, updated" state. with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with a TMSI allocated.

Test Procedure

In the first part of the test, the SS pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the n-th CHANNEL REQUEST message (n being randomly chosen by the SS from the set {1, 2 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message, which references one of the last 3 CHANNEL REQUEST messages from the MS. The MS shall then go to the correct channel and send a PAGING RESPONSE message. The SS will then release the channel.

In the second part of the test, the SS again pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the k-th CHANNEL REQUEST message (k being randomly chosen by the SS from the set {4, 5 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message which, instead of referencing one of the last 3 CHANNEL REQUEST messages from the MS, references an earlier CHANNEL REQUEST message. The MS shall then ignore the IMMEDIATE ASSIGNMENT EXTENDED message and continue to send CHANNEL REQUEST messages until the Max-Retrans value has been reached. Then a period of 7 seconds shall elapse in order to allow the MS to perform cell reselection (this allows for the time between the last CHANNEL REQUEST message and the beginning of cell reselection).

In the third part of the test, the CCCH_CONF of the SS is set to non-combined and the SS pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the r-th CHANNEL REQUEST message (r being randomly chosen by the SS from the set {4, 5 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message which, in the second request reference, references one of the last 3 CHANNEL REQUEST messages from the MS. The associated Channel Description allocates SDCCH(S) (S being randomly chosen by the SS from the set {0,1 ... 7}). The MS shall then go to the correct channel and send a PAGING RESPONSE message. The SS will then release the channel.

Maximum Duration of Test

90 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	n CHANNEL REQUESTs (n being
				randomly drawn from {1.... 8}
				are sent, all with Establ. Cause =
				"Answer to paging"
	3	MS -> SS	CHANNEL REQUEST
	4	SS -> MS	IMM ASSIGN EXT 	see note 3
	5	MS -> SS	PAGING RESPONSE
	6	SS -> MS	CHANNEL RELEASE
	7	--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	8	SS -> MS	PAG REQ TYPE 1
	9	MS -> SS	CHANNEL REQUEST	k CHANNEL REQUESTs (k being
				randomly drawn from the set {4, 5, 
				8}) are sent all with
	10	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	11	SS -> MS		IMM ASSIGN EXT	see note 4
	12	MS -> SS		CHANNEL REQUEST 8-k CHANNEL REQUESTs
				are sent, all with Establ. Cause = "Answer to
	13	MS -> SS	CHANNEL REQUEST	paging"
	14	SS		The SS verifies that the MS does not
				transmit any Layer 2 frames for at least
				3 seconds.
	15	 SS		The SS sets CCCH_CONF to
				non-combined
	16	 SS		The SS waits 40 seconds to
				allow the MS to perform
				cell reselection and to
				read the BCCH information
	17	SS -> MS	PAG REQ TYPE 1	
	18	MS -> SS	CHANNEL REQUEST	r CHANNEL REQUESTs (r being
				randomly drawn from {4, 5... 8}
				are sent, all with Establ. Cause =
				"Answer to paging"
	19	MS -> SS	CHANNEL REQUEST
	20	SS -> MS	IMM ASSIGN EXT 	see note 5
	21	MS -> SS	PAGING RESPONSE
	22	SS -> MS	CHANNEL RELEASE

Specific Message Contents

NOTE 3: 	The first Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {max (1,n-2) ... n}, its value being randomly drawn by the SS. The second Request Reference shall be different from any Request Reference the MS has generated in this test.

NOTE 4: 	The first Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {1 ... k-3}, its value being randomly drawn by the SS. The second Request Reference shall be different from any Request Reference the MS has generated in this test.

NOTE 5: 	The second Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {r-2, r-1, r}, its value being randomly drawn by the SS. The first Request Reference shall be different from any Request Reference the MS has generated in this test.

#### 26.6.1.3	Immediate assignment / assignment rejection

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.6 in phase 1 version.

-	two procedures have been concatenated to one test.

-	there is an IMMEDIATE ASSIGNMENT REJECT added at the end of the test to avoid a cell reselection.]

##### 26.6.1.3.1	Conformance requirements

1.	The MS shall respond to the Paging Request message by sending a Channel Request message with establishment cause set to "Answer to Paging". After the reception of IMMEDIATE ASSIGNMENT REJECT, the MS shall not transmit during the time indicated in the "Wait Indication" field of the IMMEDIATE ASSIGNMENT REJECT message, and then it shall answer to the new paging requests.

2.	After an assignment rejection, the MS shall perform a cell reselection (idle mode operation) and the MS shall not transmit unless a different cell is selected.

References

GSM 04.08, section 3.3.1.2.2.

##### 26.6.1.3.2	Test purpose

To verify that the MS can accept an Immediate Assignment Rejection.

To verify that the MS can respond to paging  after an Immediate Assignment Rejection is received on a different cell.

##### 26.6.1.3.3	Method of test

Initial Conditions

System Simulator:

2 cells with the same LAI, Max-Retrans is 7.

Mobile Station:

The MS is camped on cell A and is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The MS is camped on cell B and is in the "idle, updated" state, with a TMSI allocated.

Test Procedure

The SS pages the MS, which shall react by sending CHANNEL REQUESTs. Immediately after reception of the n-th CHANNEL REQUEST (n being an integer from the set {1, 2 ... 8}, randomly chosen by the SS) the SS sends an IMMEDIATE ASSIGNMENT REJECT message, which references one of the last 3 CHANNEL REQUESTs from the MS, and with the Wait Indication set to x seconds (x being an integer from the set {5, 6 ... 255}, randomly chosen by the SS). The SS continues to send paging messages for that mobile station in every block of the mobile station's paging subgroup for x+2 seconds. The MS shall only answer with a CHANNEL REQUEST after x seconds have elapsed.

The SS responds to this CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message with the Wait Indication set to 255 seconds.

Immediately afterwards the SS changes the power levels so the MS selects cell B. After 12 seconds have elapsed the SS pages the MS in cell B and the MS shall answer to this page. In order to avoid another cell reselection the SS then sends another IMMEDIATE ASSIGNMENT REJECT.

Maximum Duration of Test

5 minutes.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	n CHANNEL REQUESTs (n being
	.	.	.	randomly drawn from the set {1, 2
	.	.	.	 ... 8}) are sent, all with
	1+n	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	2+n	SS -> MS	IMM ASSIGN REJ	1st, 3rd and 4th Request
				References are different to all 
				n Request References received from
				the MS under test.
				2nd Request Reference : see note 6.
				2nd Wait Indication = x seconds (x
				being randomly drawn from the set
				{5 ,6 ... 255}
	3+n	SS -> MS	PAG REQ TYPE 1	Sent for more than x seconds
	.	.	.
	.	.	.
	k	SS -> MS	PAG REQ TYPE 1	between x and x+2 seconds (note 7)
	k+1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	k+2	SS -> MS	IMM ASSIGN REJ	1st, 2nd and 4th Request
				References are different to all 
				n Request References received from
				the MS.
				The 3rd Request Reference pertains
				to the last CHANNEL REQUEST
				sent by the MS.
				The 3rd Wait Indication is 255
				seconds
	k+3	--------	----------	Raise power level of cell B, lower
				power level of cell A until the MS
				selects cell B
	k+4	SS -> MS	PAG REQ TYPE 1	Sent once, 12 seconds after the change
				of levels
	k+5	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	k+6	SS -> MS	IMM ASSIGN REJ

NOTE 6:	The Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer from the set {max(1,n-2) ... n}, its value being randomly drawn by the SS.

NOTE 7:	the value of k is not important in this test.

Specific Message Contents

None.

#### 26.6.1.4	Immediate assignment / ignore assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.7 in phase 1 version.

-	two tests have been concatenated to one.

(* it is not clear that there were originally two tests : it is possible to do as one test. How have R+S implemented the FTA? *)

-	an IMMEDIATE ASSIGNMENT REJECT has been added to the end of the test in order to avoid a cell reselection

-	Timing Advance is no longer included]

##### 26.6.1.4.1	Conformance requirements

1.	An MS waiting for a response from the network, following the sending of a CHANNEL REQUEST, shall ignore an IMMEDIATE ASSIGNMENT message with a request reference containing a wrong frame number.

2.	An MS is waiting for an assignment of its own, shall ignore an IMMEDIATE ASSIGNMENT message with a request reference containing a wrong random access information.

References

GSM 04.08 section 3.3.1.2.1

##### 26.6.1.4.2	Test purpose

To verify that the MS ignores an assignment for another MS while it is waiting for an assignment of its own.

##### 26.6.1.4.3	Method of test

Initial Conditions

System Simulator:

1 cell

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS, which reacts with CHANNEL REQUESTs. The SS responds to the first CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT containing a wrong Request Reference (in the first run of the test the frame number is wrong, in the repetition it is the random access info that is wrong). It is verified for 2 seconds that the MS does not start signalling on the SDCCH. The MS shall ignore the assignment and send another CHANNEL REQUEST message. In order to avoid cell reselection the SS now answers with a correct IMMEDIATE ASSIGNMENT REJECT and repeats the test once.

Maximum Duration of Test

12 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Frame number in Request Reference
				is 2 too high. The MS shall not
				start signalling on the assigned
				SDCCH. This is verified for a
				period of 2 seconds
	4	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	5	SS -> MS	IMM ASSIGN REJ	
	6	SS		SS waits for 6 seconds
	7	SS -> MS	PAG REQ TYPE 1
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	SS -> MS	IMM ASSIGNMENT	Random access info in Request
				Reference is wrong. The MS shall
				not start signalling on the
				assigned SDCCH. This is verified
				for a period of 2 seconds
	10	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	11	SS -> MS	IMM ASSIGN REJ

Specific Message Contents

None.

#### 26.6.1.5	Immediate assignment after immediate assignment reject

##### 26.6.1.5.1	Conformance requirement

Following an IMMEDIATE ASSIGNMENT REJECT message, the MS shall listen for IMMEDIATE ASSIGNMENTS until T3126 expires.

Reference

GSM 04.08 section 3.3.1.3.2.

##### 26.6.1.5.2	Test purpose

To verify that the MS correctly responds to an IMMEDIATE ASSIGNMENT message sent after an IMMEDIATE ASSIGNMENT REJECT message.

##### 26.6.1.5.3	Method of test

Initial conditions

System Simulator

1 cell

CCCH-CONF is set to '1 basic physical channel used for CCCH not combined with SDCCHs'

Max retrans is set to 7

TX-integer is set to 7

Mobile Station

The MS is in "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

Type of MS (P-GSM900 or EGSM or DCS1800).

Forseen final state of the MS

"idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS, which shall react by sending CHANNEL REQUESTs. Immediately after reception of the third CHANNEL REQUEST the SS sends an IMMEDIATE ASSIGNMENT REJECT message which references the first CHANNEL REQUEST from the MS and has the Wait Indication IE set to 6 seconds.

Between 0.75 seconds and 1.25 seconds after sending the IMMEDIATE ASSIGNMENT REJECT message the SS sends an IMMEDIATE ASSIGNMENT message referencing the second CHANNEL REQUEST message, and assigning an SDCCH. The MS shall go to the correct channel and send a PAGING RESPONSE message. Then the SS initiates RR-release by sending a CHANNEL RELEASE message.

Maximum duration of test

10 seconds

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	first request
	3	MS -> SS	CHANNEL REQUEST	second request
	4	MS -> SS	CHANNEL REQUEST	third request
	5	SS -> MS	IMM ASSIGNMENT REJ	references the first request from MS, Wait
				Indication IE set to 6 seconds.
	6	SS -> MS	IMM ASSIGNMENT	references the second request from the MS
				Channel type set to SDCCH/8
				message sent between 0.75s and 1.25s after the
				completion of step 5
	7	MS -> SS	PAGING RESPONSE	shall be sent on the correct channel
	8	SS -> MS	CHANNEL RELEASE

Specific message contents

None.

### 26.6.2	Test of paging

The Paging procedure is used by the network to cause the Mobile Station to establish a radio connection. Normally the Mobile Station listens to its paging subchannel, but this can be modified by the use of different page modes. The correct implementation of the paging procedure in the Mobile Station is essential for the basic establishment of a connection.

#### 26.6.2.1	Normal paging

##### 26.6.2.1.1	Paging / normal / type 1

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.3 in phase 1 version.

-	6 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.

]

(* step 5 has been changed for phase 2 imsi response to imsi paging.*)

(* steps 25 and 26 may be deleted as ignoring IMEI pages is difficult to 'justify'.*)

###### 26.6.2.1.1.1	Conformance requirements

1.	The MS shall respond correctly to various PAGING REQUEST TYPE 1 messages, when the page mode is set to normal paging, in the following cases:

1.1	The MS is addressed with its IMSI in the first Mobile Identity field. The optional Mobile Identity field is not present.

1.2.	The MS is addressed with its TMSI in the first Mobile Identity field. The optional Mobile Identity field specifies an IMSI different from that of the MS.

1.3.	The first Mobile Identity field specifies a TMSI different from that of the MS. The optional Mobile Identity field addresses the MS by its IMSI.

1.4	The first Mobile Identity field specifies a TMSI different from that of the MS. The optional Mobile Identity field contains the correct TMSI of the MS.

2.	An MS shall ignore PAGING REQUEST TYPE 1 messages with incorrect information, when the page mode is set to normal paging, in the following case:

2.1	The MS is addressed with its TMSI in the first Mobile Identity field, but the type of identity in this field is set to "No Identity". The optional Mobile Identity field is not present.

References

GSM 04.08 section 3.3.2, GSM 05.02 section 6.5.

###### 26.6.2.1.1.2	Test purpose

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 1 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested. It is tested that the MS responds with the same type of identity that is used in the PAGING REQUEST TYPE 1 message. It is tested that the MS ignores fill paging.

###### 26.6.2.1.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS 5 times with different PAGING REQUEST TYPE 1 messages on the paging subchannel which corresponds to the MS's IMSI.

In the first 4 cases, where the MS is addressed by its IMSI or its TMSI, the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

In the last case, it is tested that the MS does not answer to paging that does not address it.

Maximum Duration of Test

2 min

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains IMSI of
				MS, 2nd Mobile Ident not present.
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to 
				the message received in step 3
	5	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.
	6	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	7	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of
				MS, 2nd Mobile Ident contains IMSI
				of another MS.
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 9
	11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	12	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	13	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of
				another MS, 2nd Mobile Ident
				contains IMSI of MS.
	14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 15
	17	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.
	18	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	19	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of
				another MS, 2nd Mobile Ident
				contains TMSI of MS.
	20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 21
	23	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	24	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection

	25	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of
				MS but with type of identity set
				to "no identity", second Mobile
				Ident not present.

	26	--------	--------	During 1 second, the SS checks
				that the MS does not produce any
				Layer 2 frames.

(* In steps 26 and 28, it may be sufficient to check for no layer 3 messages rather than checking for no RF-transmission.*)

Specific Message Contents

None.

##### 26.6.2.1.2	Paging / normal / type 2

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.5 in phase 1 version.

-	6 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.]

(* imsi response to imsi paging is included.*)

(* imei paging test may not be 'justified' *)

###### 26.6.2.1.2.1	Conformance requirements

1.	The MS shall respond correctly (by sending CHANNEL REQUEST messages with an Establishment Cause set to "Answer to Paging") to various PAGING REQUEST TYPE 2 messages, when the page mode is set to normal paging, in the following cases:

1.1	The MS is addressed in the first TMSI field.

1.2	The MS is addressed in the second TMSI field.

1.3	The MS is addressed in the optional Mobile Identity field with its TMSI.

1.4	The MS is addressed in the optional Mobile Identity field with its IMSI.

2.	The MS shall ignore PAGING REQUEST TYPE 2 messages with incorrect information, when the page mode is set to normal paging, in the following case:

2.1	The MS is addressed in the optional Mobile Identity field with its TMSI, but the type of identity in this field is set to "No Identity".

References

GSM 04.08 section 3.3.2.

###### 26.6.2.1.2.2	Test purpose

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 2 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested. It is tested that the MS responds with the same type of identity that is used in the PAGING REQUEST TYPE 2 message. It is tested that the MS ignores a PAGING REQUEST TYPE 2 message that does not address it.

###### 26.6.2.1.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS 5 times with different PAGING REQUEST TYPE 2 messages on the paging subchannel which corresponds to the MS's IMSI.

In the first 4 cases, where the MS is addressed by its IMSI or by its TMSI, the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second request by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

In the last case, it is tested that the MS does not answer to paging that does not address it.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses MS, 2nd TMSI
				addresses another MS, Mobile
				Identity IE not present
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 3
	5	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	6	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	7	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd
				TMSI addresses MS, Mobile Identity
				IE not present.
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 9
	11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	12	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	13	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd
				TMSI addresses another MS, Mobile
				Identity IE contains TMSI of MS.
	14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 15
	17	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	18	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	19	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd
				TMSI addresses another MS, Mobile
				Identity IE contains IMSI of MS.
	20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 21
	23	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.
	24	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection

	25	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd
				TMSI addresses another MS, Mobile
				Identity IE contains TMSI of MS
				but with type of identity set to
				"no identity".

	26	--------	--------	During 1 second, the SS checks
				that the MS does not produce any
				Layer 2 frames.

(* RF transmission ==> L3 messages ?*)

Specific Message Contents

None.

##### 26.6.2.1.3	Paging / normal / type 3

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.7 in phase 1 version.

-	4 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.]

###### 26.6.2.1.3.1	Conformance requirements

An MS shall respond correctly to various PAGING REQUEST TYPE 3 messages, when the page mode is set to normal paging. The MS shall send CHANNEL REQUEST messages, with an Establishment Cause set to "Answer to Paging", until the network answers. The number of CHANNEL REQUEST messages shall be limited by the parameter Max-retrans. After the assignment procedure, the MS shall send a PAGING RESPONSE message on the channel assigned by the network.

References

GSM 04.08 section 3.3.2.

###### 26.6.2.1.3.2	Test purpose

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 3 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested.

###### 26.6.2.1.3.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS 4 times with different PAGING REQUEST TYPE 3 messages on the paging subchannel which corresponds to the MS's IMSI.

In all the cases the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second request by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 3	1st TMSI addresses MS; 2nd, 3rd
				and 4th TMSIs address other MSs.
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 3
	5	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	6	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	7	SS -> MS	PAG REQ TYPE 3	2nd TMSI addresses MS; 1st, 3rd
				and 4th TMSIs address other MSs.
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 9
	11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	12	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	13	SS -> MS	PAG REQ TYPE 3	3rd TMSI addresses MS; 1st, 2nd
				and 4th TMSIs address other MSs.
	14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 15
	17	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	18	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	19	SS -> MS	PAG REQ TYPE 3	4th TMSI addresses MS; 1st, 2nd
				and 3rd TMSIs address other MSs.
	20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 21
	23	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	24	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None.

#### 26.6.2.2	Paging / extended

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	two procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.

-	Instead of specifying a specific page mode , the mode is now chosen arbitrarily.*)

-	It is specified that in step 6 the random reference is not one used by the MS.]

##### 26.6.2.2.1	Conformance requirements

1.	The MS shall operate in the extended page mode when this is ordered by the network in a PAGING REQUEST TYPE 1 message not addressing the MS but on the paging subchannel which corresponds to the MS's identity.

2.	The MS shall operate in the extended page mode when this is ordered by the network in a PAGING REQUEST TYPE 2 message not addressing the MS but on the paging subchannel which corresponds to the MS's identity.

3.	The MS shall operate in the extended page mode when this is ordered by the network in a PAGING REQUEST TYPE 3 message not addressing the MS but on the paging subchannel which corresponds to the MS's identity.

4.	The MS shall operate in the extended page mode when this is ordered by the network in an IMMEDIATE ASSIGNMENT message on the paging subchannel which corresponds to the MS's identity.

5.	The MS shall operate in the extended page mode when this is ordered by the network in an IMMEDIATE ASSIGNMENT EXTENDED message on the paging subchannel which corresponds to the MS's identity.

References

GSM 04.08 section 3.3.2.1; GSM 05.02, section 6.5

##### 26.6.2.2.2	Test purpose

To test that the MS is operating in the extended page mode when this is ordered by the SS in either a PAGING REQUEST message or an IMMEDIATE ASSIGNMENT message.

##### 26.6.2.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS sends a PAGING REQUEST TYPE 1 message not addressing the MS under test but on the paging subchannel which corresponds to the MS's identity. The page mode is set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying an arbitrarily chosen page mode and addressing the MS by its TMSI. The MS shall respond to the last page by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

The SS then sends an IMMEDIATE ASSIGNMENT on the paging subchannel which corresponds to the MS's identity. The random reference is different to those used by the Mobile Station in the last two CHANNEL REQUEST messages. (Phase 2 requires a Mobile Station to react on an IMMEDIATE ASSIGNMENT after a rejection.) The page mode is again set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 2 message specifying an arbitrarily chosen page mode and addressing the MS by its TMSI. The MS shall respond with CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

The SS then sends an IMMEDIATE ASSIGNMENT EXTENDED on the paging subchannel which corresponds to the MS's identity. The random references are different to those used by the Mobile Station in the last three CHANNEL REQUEST messages. The page mode is again set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 3 message specifying an arbitrarily chosen page mode and addressing the MS by its TMSI. The MS shall respond with CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

The SS then sends a PAGING REQUEST TYPE 3 message not addressing the MS under test but on the paging subchannel which corresponds to the MS's identity. The page mode is set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 2 message specifying an arbitrarily chosen page mode and addressing the MS by its IMSI. The MS shall respond to the last page by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

The SS then sends a PAGING REQUEST TYPE 2 message not addressing the MS under test but on the paging subchannel which corresponds to the MS's identity. The page mode is set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying an arbitrarily chosen page mode and addressing the MS by its IMSI. The MS shall respond to the last page by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

[editorial note : If it is an essential feature that the MS ignores paging on other subchannels, later, a test shall be elaborated, which tests that the MS does not take into account the paging reorganisation (only page modes on the MS's own paging subchannel shall be taken into account). However it seems not reasonable to forbid the MS listening to e

Maximum Duration of Test

10 sec

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	Mobile Ident : IMSI of another MS,
				page mode = "extended paging"
	2	SS -> MS	PAG REQ TYPE 1	Sent in the next but one paging
				subblock. Page mode 
				is arbitrarily chosen
				Mobile Ident : TMSI of the MS.
	3	MS -> SS	CHANNEL REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMM ASSIGN REJ	Wait indication = 5 seconds
				page mode = normal
	6	SS		SS waits for 5 seconds
		SS -> MS	IMM ASSIGNMENT	Sent in the paging subblock of
				MS under test. Page mode =
				"extended paging", Request
				reference chosen arbitrarily by
				the SS, but different from all
				references used earlier in this
				test sequence
	8	SS -> MS	PAG REQ TYPE 1	Sent in the next but one paging
				subblock. Page mode is arbitrarily
				chosen. Mobile Ident : TMSI of
				the MS.
	9	MS -> SS	CHANNEL REQUEST	
	10	MS -> SS	CHANNEL REQUEST	
	11	SS -> MS	IMM ASSIGN REJ	Wait indication = 5 seconds
				page mode = normal
	12	SS	--	SS waits for 5 seconds
	13	SS -> MS	IMM ASSIGNMENT EXT	Sent in the paging subblock of
				MS under test. Page mode =
				"extended paging", Request
				references chosen arbitrarily by
				the SS, but different from all
				references used earlier in this
				test sequence
	14	SS -> MS	PAG REQ TYPE 3	Sent in the next but one paging
				subblock. Page mode is arbitrarily
				chosen. Mobile Ident : TMSI of
				the MS.
	15	MS -> SS	CHANNEL REQUEST	
	16	MS -> SS	CHANNEL REQUEST	
	17	SS -> MS	IMM ASSIGN REJ	Wait indication = 5 seconds
				page mode = normal
	18	SS	--	SS waits for 5 seconds
	19	SS -> MS	PAG REQ TYPE 3	Sent in the paging subblock of
				MS under test. Page mode =
				"extended paging", 
	20	SS -> MS	PAG REQ TYPE 2	Sent in the next but one paging
				subblock. Page mode is arbitrarily
				chosen. Mobile Ident : IMSI of
				the MS.
	21	MS -> SS	CHANNEL REQUEST	
	22	MS -> SS	CHANNEL REQUEST	
	23	SS -> MS	IMM ASSIGN REJ	Wait indication = 5 seconds
				page mode = normal
	24	SS	--	SS waits for 5 seconds

25	SS -> MS	PAG REQ TYPE 2	Sent in the paging subblock of
				MS under test. Page mode =
				"extended paging", 
	26	SS -> MS	PAG REQ TYPE 1	Sent in the next but one paging
				subblock. Page mode is arbitrarily
				chosen. Mobile Ident : IMSI of
				the MS.
	27	MS -> SS	CHANNEL REQUEST	
	28	MS -> SS	CHANNEL REQUEST	
	29	SS -> MS	IMM ASSIGN REJ	Wait indication = 5 seconds

Specific Message Contents

None.

#### 26.6.2.3	Paging / reorganisation

##### 26.6.2.3.1	Paging / reorganisation / procedure 1

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.3 in phase 1 version.

-	the paging parameters are chosen   arbitrarily by the SS

-	type of identity in the first PAGING REQUEST TYPE 1 is no longer specified as IMSI.]

(* half of this test can probably be deleted -- however as these are the 2nd and 4th quarters it would disrupt the SS !!! *)

###### 26.6.2.3.1.1	Conformance requirements

1.	An MS, after reception of a message with page mode set to "paging reorganisation", shall answer to paging messages (with page mode set to "normal paging") sent on its old CCCH in paging blocks which do not belong to the MS's paging sub-channel.

2.	When the network changes the paging group of the MS by modifying BCCH parameters (CCCH-CONF set to "1 basic physical channel used for CCCH combined with SDCCH", and BS-AG-BLKS-RES set to "2 blocks reserved for access grant"), the MS shall calculate its new paging group and answer to paging messages on its new paging subchannel.

3.	When the network changes the paging group of the MS by modifying BCCH parameters (CCCH-CONF set to "2 basic physical channels used for CCCH, not combined with SDCCHs" and BS-AG-BLKS-RES set to "2 blocks reserved for access grant"), the MS shall calculate its new paging group and answer to paging messages on its new paging subchannel.

References

GSM 04.08 section 3.3.2.1; GSM 05.02, section 6.5.

###### 26.6.2.3.1.2	Test purpose

To test that the MS correctly determines its new paging subchannel when the CCCH structure is changed from non-combined to combined.

###### 26.6.2.3.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, CCCH-CONF set to "1 basic physical channel used for CCCH, not combined with SDCCHs", a legal combination of BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS, with the exception that BS-PA-MFRMS shall not be set to 9.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated. The IMSI of the MS is from a defined/default range that ensures its paging channel changes when the broadcast parameters are changed.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS sends an IMMEDIATE ASSIGNMENT EXTENDED message on the MS's paging subchannel, with the page mode element set to "paging reorganisation" and Request References that do not pertain to the MS. Before the MS's original paging subchannel re-occurs, the SS pages it on the CCCH corresponding to the Mobile Station's IMSI with a PAGING REQUEST TYPE 2 message (page mode = normal paging) containing the MS's TMSI in some paging block which does not belong to the Mobile Station's paging subchannel. The MS shall respond by sending CHANNEL REQUESTs.The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection) on an arbitrarily selected paging subchannel.

Then the SS starts sending messages (PAGING REQUEST TYPE 1 or PAGING REQUEST TYPE 2 or PAGING REQUEST TYPE 3 or IMMEDIATE ASSIGNMENT or IMMEDIATE ASSIGNMENT EXTENDED or IMMEDIATE ASSIGNMENT REJECT) with page mode set to "paging reorganization" on all paging subchannels.

After 5 seconds (to ensure T3126 expires) the SS pages the MS with its TMSI on an arbitrarily selected paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

The SS changes the paging parameters.

Then the SS sets the page mode to "normal paging".

The SS then waits for the duration of five 51-TDMA multiframes (4 to allow the MS to read all the system information type 1, 2, 3, and 4 messages on the BCCH, and one to calculate the new paging group). Not before 5 seconds after the last IMMEDIATE ASSIGNMENT REJECT message addressing the MS (to ensure T3126 expires), the MS is paged with a PAGING REQUEST TYPE 1 on its new paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection) and then waits 5 seconds (to ensure T3126 expires).

Then the MS is paged with a PAGING REQUEST TYPE 2 on its new paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test

60 sec

Expected Sequence

This sequence is performed for execution counter, K =1, 2.

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN EXT	Sent on the MS's paging
				channel. Page mode set to "paging
				reorganisation". Request Reference
				not pertaining to the MS.
	2	SS -> MS	PAG REQ TYPE 2	Sent before the MS's original
				paging subchannel re-occurs, but 
				later than the next paging
				block of that CCCH.
	3	MS -> SS	CHANNEL REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.

	6	--------	--------	All L3 messages sent on any paging
				subchannel are paging fill
				frames specify "paging re
				organization.

	7	SS -> MS	PAG REQ TYPE 2	Sent on an arbitrarily 
				selected paging subchannel
				Page mode "paging reorganisation"
				Not sent before 5 seconds after step 5.
	8	MS -> SS	CHANNEL REQUEST	
	9	MS -> SS	CHANNEL REQUEST	
	10	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.

	11			Change of paging
				parameters in SYS INFO 3  as
				described below for K=1, 2
	12			The SS waits until it has
				sent all system information
				messages (page mode is still
				paging reorganisation).

	13	--------	--------	All L3 messages sent on any paging
				subchannel specify "normal paging 
	14	--------	--------	Wait 3 seconds
	15	SS -> MS	PAG REQ TYPE 1	Sent on the new paging subchannel						of the MS.
				Not sent before 5 seconds after step 10.
	16	MS -> SS	CHANNEL REQUEST	
	17	MS -> SS	CHANNEL REQUEST	
	18	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.
	19	SS -> MS	PAG REQ TYPE 2	Sent on the new paging subchannel					of the MS.
				Not sent before 5 seconds after step 18.
	20	MS -> SS	CHANNEL REQUEST	
	21	MS -> SS	CHANNEL REQUEST	
	22	SS -> MS	IMM ASSIGN REJ

Specific Message Contents

For execution counter K :

K=1:

SYSTEM INFORMATION TYPE 3 shall have the Control Channel Description IE changed to:

CCCH-CONF	"1 basic physical channel used for CCCH, combined with SDCCHs"

BS-AG-BLKS-RES	2

BS-PA-MFRMS	9

K=2:

SYSTEM INFORMATION TYPE 3 shall have the Control Channel Description IE changed to:

CCCH-CONF	"2 basic physical channel used for CCCH, not combined with SDCCHs"

BS-AG-BLKS-RES	2

BS-PA-MFRMS	9

##### 26.6.2.3.2	Paging / reorganisation / procedure 2

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.3 in phase 1 version.

-	It is stated clearly that the paging parameters are chosen randomly by the SS.]

###### 26.6.2.3.2.1	Conformance requirement

An MS, after reception of a message with page mode set to "paging reorganisation", shall answer to paging messages (with page mode set to "normal paging") sent in a former Access Grant block.

References

GSM 04.08 section 3.3.2.1.

###### 26.6.2.3.2.2	Test purpose

To test that the MS is operating in the "paging reorganisation" page mode when this is ordered by the SS and the MS is paged in its former access grant channel.

###### 26.6.2.3.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 1, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated. The IMSI of the MS is from a defined\default range that ensures its paging channel changes when the broadcast parameters are changed.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS sends an IMMEDIATE ASSIGNMENT EXTENDED on the MS's paging subchannel, with the page mode element set to "paging reorganisation". The MS is then paged immediately in a former Access Grant block with a PAGING REQUEST TYPE 2 message. The MS shall respond by sending CHANNEL REQUESTs.The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test

5 sec

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN EXT	Page mode set to "paging
				reorganisation"
	2	SS -> MS	PAG REQ TYPE 2	Sent in a former access grant
				block.
	3	MS -> SS	CHANNEL REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMM ASSIGN REJ

Specific Message Contents

None.

#### 26.6.2.4	Paging / same as before

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	the paging parameters are chosen randomly by the SS.]

##### 26.6.2.4.1	Conformance requirements

An MS, after first receiving a message with page mode set to "extended paging" and then another message with page mode set to "same as before", shall remember the page mode from the previous message and answer to paging messages in the next but one paging sub block.

References

GSM 04.08 section 3.3.2.1;  GSM 05.02, section 6.5.

##### 26.6.2.4.2	Test purpose

To test that the MS remembers the page mode from the previous paging request message.

##### 26.6.2.4.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS sends an IMMEDIATE ASSIGNMENT REJECT on the MS's paging subchannel, with the page mode element set to "extended paging". In the next but one subblock on the same CCCH, nothing addresses the MS. When the MS's specific paging subchannel reoccurs, a PAGING REQUEST TYPE 3 is sent, not addressing the MS under test and with page mode set to "same as before". In the next but one subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying paging reorganisation and addressing the MS. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test

10 sec

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN REJ	Page mode set to "extended paging"
	2	SS -> MS	XXXX	In the next but one subblock
				nothing addresses the MS
	3	SS -> MS	PAG REQ TYPE 3	This is sent in the next paging
				subblock on the MS's specific
				paging subchannel. The page mode
				is set to "same as before", and the MS
				under test is not addressed.
	4	SS -> MS	PAG REQ TYPE 1	The MS is addressed in this 'next
				but one subblock'. Page mode set
				to "paging reorganisation"
	5	MS -> SS	CHANNEL REQUEST	
	6	MS -> SS	CHANNEL REQUEST	
	7	SS -> MS	IMM ASSIGN REJ

Specific Message Contents

None.

#### 26.6.2.5	Paging / multislot CCCH

##### 26.6.2.5.1	Conformance requirements

The MS shall respond correctly to a PAGING REQUEST TYPE 1 message, when the page mode is set to normal paging, when a multislot CCCH is used and the MS is addressed with its IMSI in the first Mobile Identity field, the optional Mobile Identity field being not present.

References

GSM 04.08 section 3.3.2, GSM 05.02 section 6.5.

##### 26.6.2.5.2	Test purpose

1)	To test that the MS is able to determine its CCCH group and paging group correctly in the case of a CCCH configuration on more than one timeslot when it is paged on a timeslot other than 0. The MS is addressed with a PAGING REQUEST TYPE 1 message when the page mode is set to normal paging. The MS is paged with its IMSI in the 1st Mobile Identity field, the optional Mobile Identity field being not present, is the only way of addressing tested.

2)	To test that in such conditions the MS answers to the paging message on the timeslot on which the paging message was sent.

##### 26.6.2.5.3	Method of test

Initial Conditions

System Simulator:

1 cell, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily under the following constraint :

CCCH-CONF is in the set :

-	2 basic physical channels used for CCCH, not combined with SDCCHs

-	3 basic physical channels used for CCCH, not combined with SDCCHs

-	4 basic physical channels used for CCCH, not combined with SDCCHs

Mobile Station :

The IMSI last 3 digits are so that the CCCH-GROUP of the MS under test is other than 0. According to section 6.5.2 of recommendation 05.02, this means that :

(IMSI mod 1000) mod (BS_CC_CHANS X N) is greater or equal to N, where N = BS_PA_MFRMS X (9 - BS_AG_BLKS_RES).

The MS is in the "idle, updated" state.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle updated".

Test Procedure

The SS pages the MS once with a PAGING REQUEST TYPE 1 message on the timeslot and paging subchannel which correspond to the MS's IMSI.

The MS shall send the CHANNEL REQUEST on the same timeslot as the paging message.

The SS sends an IMMEDIATE ASSIGNMENT on the same timeslot as the paging message.

Maximum Duration of Test

10 s

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains IMSI of
				MS, 2nd Mobile Ident not present.
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging".
				on the same timeslot as the paging
				message
	3	SS -> MS	IMM ASSIGNMENT	on the same timeslot as the paging
				message
	4	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.
	5	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None.

### 26.6.3	Test of measurement report

When an RR-connection exists, the MS shall send measurement reports. These reports contain reception characteristics from serving and neighbouring cells. The measurement report procedure is described in section 3.4.1.2 of 04.08.

NOTE 8:	The capability to calculate RxLev and RxQual is tested in sections 15 and 16. In this test only the signalling aspect is verified.

#### 26.6.3.1	Measurement / no neighbours

This test applies to both GSM900 and DCS1800 mobile stations.

##### 26.6.3.1.1	Conformance requirements

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH block and the measurement valid indication shall be set to valid (0) within the second block at the latest.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.6.3.1.2	Test purpose

To test that, when the SS gives absolutely no information about neighbouring cells, the MS does not report on neighbouring cells.

##### 26.6.3.1.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BCC	ARFCN	ARFCN	Cell Identity

(GSM900)	(DCS1800)

Serving, S1	-60	1	3	002	514	0001H

Neighbour, N1	-85	1	5	008	530	0002H

Neighbour, N2	-80	1	7	014	602	0003H

Neighbour, N3	-75	1	1	020	665	0004H

Neighbour, N4	-55	1	3	026	762	0005H

Neighbour, N5	-50	1	5	032	686	0006H

Neighbour, N6	-45	1	7	038	549	0007H

Neighbour, N7	-40	1	1	044	810	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed twice.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 (on the second iteration of the test the SS also sends SYSTEM INFORMATION TYPE 5bis) on the SACCH. The BA is indicated as empty. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that no measurement results have been obtained.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, k = 1, 2.

Since when k = 1, SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT (and when k = 2 an additional SYSTEM INFORMATION TYPE 5bis is included) are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

GSM900 begin:

SYSTEM INFORMATION TYPE 5:

Information Element				value/remark

Neighbour Cells Description

- Format Identifier			bit map 0

- BCCH Allocation Sequence number	1

- BCCH Allocation ARFCN		No channels belong to the BCCH

allocation

- EXT IND				k = 1. Information Element carries the
				complete BA.

k = 2. Information Element carries only a

part of the BA.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element				value/remark

Protocol Descriminator			RR management

Mesage Type				Sys Info 5bis.

Neighbour Cells Description

- Format 				1024 range

- EXT IND				k = 2.   Information Element carries only a part
				of the BA.

- W(i)					Only channel 500 belongs to the BCCH
				allocation.

MEASUREMENT REPORT:

Information Element				value/remark

Protocol Discriminator			RR Management

Transaction Identifier				0000

Message Type				MEASUREMENT REPORT

Measurement Results

BA-used				1

DTX-used				DTX was not used

RXLEV-FULL-SERVING-CELL		See note 9

RXLEV-SUB-SERVING-CELL		See note 9

MEAS-VALID				See note 10

RXQUAL-FULL-SERVING-CELL		See note 9

RXQUAL-SUB-SERVING-CELL		See note 9

NO-NCELL-M				No neighbour cell measurement result, or

Neighbour cell information not available

for serving cell

RXLEV-NCELL-1			00 0000

BCCH-FREQ-NCELL-1			 0 0000

BSIC-NCELL-1				00 0000

.     .     .				.       .

RXLEV-NCELL-6			00 0000

BCCH-FREQ-NCELL-6			 0 0000

BSIC-NCELL-6				00 0000

GSM900 end:

DCS1800 begin:

SYSTEM INFORMATION TYPE 5:

Information Element				value/remark

Protocol Descriminator			RR management

Mesage Type				Sys Info 5.

Neighbour Cells Description

- Format 				1024 range

- EXT IND				k = 1. Information Element carries the

complete BA.

k = 2. Information Element carries only a part

of the BA.

- W(i)					null.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element				value/remark

Protocol Descriminator			RR management

Mesage Type				Sys Info 5bis.

Neighbour Cells Description

- Format 				1024 range

- EXT IND				k = 2.   Information Element carries only a part
				of the BA.

- W(i)					null.

SYSTEM INFORMATION TYPE 6:

Information Element				value/ remark

Protocol Discriminator			RR Management

Message Type				sys info 6

Cell Identity					default

LAI						default

Cell Options

- Power Control Indicator		Power Control Indicator is set

- DTX Indicator			MS shall not use DTX

- Radio-Link-Timeout		default

PLMN permitted				only NCC 1 permitted

MEASUREMENT REPORT:

Information Element				value/remark

Protocol Discriminator			RR Management

Transaction Identifier				0000

Message Type				MEASUREMENT REPORT

Measurement Results

BA-used				1

DTX-used				DTX was not used

RXLEV-FULL-SERVING-CELL		See note 9

RXLEV-SUB-SERVING-CELL		See note 9

MEAS-VALID				See note 10

RXQUAL-FULL-SERVING-CELL		See note 9

RXQUAL-SUB-SERVING-CELL		See note 9

NO-NCELL-M				No neighbour cell measurement result, or

Neighbour cell information not available for

serving cell

RXLEV-NCELL-1			00 0000

BCCH-FREQ-NCELL-1			 0 0000

BSIC-NCELL-1				00 0000

.     .     .				.       .

.     .     .				.       .

RXLEV-NCELL-6			00 0000

BCCH-FREQ-NCELL-6			 0 0000

BSIC-NCELL-6				00 0000

DSC1800 end:

NOTE 9:	The actual values are not checked.

NOTE 10:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable -- once the MS is in U10 then this can be checked immediately.]

#### 26.6.3.2	Measurement / all neighbours present

This test applies to both GSM900 and DCS1800 mobile stations.

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.3.3 in phase 1 version

-	one test has been splitted into 4]

##### 26.6.3.2.1	Conformance requirements

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the values in the MEASUREMENT REPORT message shall contain measurement results for the 6 strongest BCCH carriers with known and allowed NCC part of BSIC.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.6.3.2.2	Test purpose

To test that, when the SS gives information about neighbouring cells, the MS reports appropriate results.

##### 26.6.3.2.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BSCC	ARFCN	ARFCN	Cell identity

(GSM900)	(DCS1800)

Serving, S1	-60	1	3	002	514	0001H

Neighbour, N1	-85	1	5	008	530	0002H

Neighbour, N2	-80	1	7	014	602	0003H

Neighbour, N3	-75	1	1	020	665	0004H

Neighbour, N4	-55	1	3	026	762	0005H

Neighbour, N5	-50	1	5	032	686	0006H

Neighbour, N6	-45	1	7	038	549	0007H

Neighbour, N7	-40	1	1	044	810	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed twice.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 (on the second iteration of the test the SS also sends SYSTEM INFORMATION TYPE 5bis) on the SACCH. All 8 of the BCCHs "on air" are indicated in the BA. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest carriers have been obtained.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, k = 1, 2.

Since when k = 1, SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT (and when k = 2 an additional SYSTEM INFORMATION TYPE 5bis is included) are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

GSM900 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Neighbour Cells Description

Format Identifier		bit map 0

BCCH Allocation Sequence	1

BCCH Allocation ARFCN	The channel numbers 2, 4, 5, 6, 7, 8, 9,

10, 11, 12, 14, 16, 17, 18, 19, 20, 21, 22,

23, 24, 25, 26, 32, 33, 34, 35, 36, 37,

38, 39, 40 and 44 belong to the BCCH

allocation.

- EXT IND			k = 1. Information Element carries complete

BA.

k = 2. Information Element carries only a

part of the BA.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part

of the BA.

- W(i)				Channel 0 and 800 belong to the BCCH

allocation.

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 11

RXLEV-SUB-SERVING-CELL	See note 11

MEAS-VALID			See note 12

RXQUAL-FULL-SERVING-CELL	See note 11

RXQUAL-SUB-SERVING-CELL	See note 11

NO-NCELL-M			6 neighbour cell measurement results

RXLEV-NCELL-1		See note 11

BCCH-FREQ-NCELL-1		Shall not correspond to N1 or N2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 11

BCCH-FREQ-NCELL-2		Shall not correspond to N1 or N2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 11

BCCH-FREQ-NCELL-3		Shall not correspond to N1 or N2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 11

BCCH-FREQ-NCELL-4		Shall not correspond to N1 or N2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		See note 11

BCCH-FREQ-NCELL-5		Shall not correspond to N1 or N2

BSIC-NCELL-5			Corresponds to that of BCCH-FREQ-NCELL-5

RXLEV-NCELL-6		See note 11

BCCH-FREQ-NCELL-6		Shall not correspond to N1 or N2

BSIC-NCELL-6			Corresponds to that of BCCH-FREQ-NCELL-6

GSM900 end:

DCS1800 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5.

Neighbour Cells Description

- Format			1024 range

- EXT IND			k = 1. Information Element carries complete

BA.

k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 1. Non null for ARFCN 514, 530, 549, 602,
			665, 686, 762, 810.

k = 2.  Non null for ARFCN 549, 602, 665, 686,
			810.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 2. Non null ARFCN 20, 514, 530, 549, 762.

SYSTEM INFORMATION TYPE 6:

Information Element			value/ remark

Protocol Discriminator		RR Management

Message Type			sys info 6

Cell Identity				default

LAI					default

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall not use DTX

- Radio-Link-Timeout	default

PLMN permitted			only NCC 1 permitted

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 11

RXLEV-SUB-SERVING-CELL	See note 11

MEAS-VALID			See note 12

RXQUAL-FULL-SERVING-CELL	See note 11

RXQUAL-SUB-SERVING-CELL	See note 11

NO-NCELL-M			6 neighbour cell measurement results

RXLEV-NCELL-1		See note 11

BCCH-FREQ-NCELL-1		Shall not correspond to N1 or N2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 11

BCCH-FREQ-NCELL-2		Shall not correspond to N1 or N2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 11

BCCH-FREQ-NCELL-3		Shall not correspond to N1 or N2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 11

BCCH-FREQ-NCELL-4		Shall not correspond to N1 or N2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		See note 11

BCCH-FREQ-NCELL-5		Shall not correspond to N1 or N2

BSIC-NCELL-5			Corresponds to that of BCCH-FREQ-NCELL-5

RXLEV-NCELL-6		See note 11

BCCH-FREQ-NCELL-6		Shall not correspond to N1 or N2

BSIC-NCELL-6			Corresponds to that of BCCH-FREQ-NCELL-6

DCS1800 end:

NOTE 11:	These actual values are not checked.

NOTE 12:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

#### 26.6.3.3	Measurement / barred cells and non-permitted NCCs

This test applies to both GSM900 and DCS1800 mobile stations.

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	one test has been splitted into 4]

##### 26.6.3.3.1	Conformance requirements

The MS shall continuously send MEASUREMENT REPORTs on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the messages shall contain measurement results only for the 4 BCCH carriers on which the MS is allowed to report.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.6.3.3.2	Test purpose

To test that, when a combination of normal neighbours, barred cells and non-permitted NCCs is "on air", the MS reports only on normal neighbours.

##### 26.6.3.3.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BCC	ARFCN	ARFCN	Cell identity

(GSM900)	(DCS1800)

Serving, S1	-60	1	3	002	514	0001H

Neighbour, N1	-85	1	5	008	530	0002H

Neighbour, N2	-80	1	7	014	602	0003H

Neighbour, N3	-75	2	1	020	665	0004H

Neighbour, N4	-55	3	3	026	762	0005H

Neighbour, N5	-50	4	5	032	686	0006H

Neighbour, N6	-45	1	7	038	549	0007H

Neighbour, N7	-40	1	1	044	810	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

NOTE 13:	The BA sent in SYSTEM INFORMATION TYPE 5 does not include N1, N4 and N5. N1 may be the case of a barred cell, N3 simulates the case where another operator is transmitting on the same frequency (e.g. in border areas), N4 & N5 simulate the case where other operators are transmitting on other frequencies.

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed twice.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 (on the second iteration of the test the SS also sends SYSTEM INFORMATION TYPE 5bis) on the SACCH. 5 of the 8 BCCHs "on air" are indicated in the BA. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 4 strongest permitted carriers have been obtained (one of the carriers in the BA belongs to a non-permitted NCC).

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, k = 1, 2.

Since when k = 1 SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT (and when k = 2 an additional SYSTEM INFORMATION TYPE 5bis is included) messages are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

GSM900 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Neighbour Cells Description

Format Identifier		bit map 0

BCCH Allocation Sequence	1

BCCH Allocation ARFCN	only channel numbers 2, 14, 20, 38, and

44 belong to the BCCH allocation.

- EXT IND			k = 1. Information Element carries complete BA.

k = 2. Information Element carries only a part
			of the BA.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part of
			the BA.

- W(i)				Channel 0 and 800 belong to the BCCH
			allocation.

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 14

RXLEV-SUB-SERVING-CELL	See note 14

MEAS-VALID			See note 15

RXQUAL-FULL-SERVING-CELL	See note 14

RXQUAL-SUB-SERVING-CELL	See note 14

NO-NCELL-M			4 neighbour cell measurement results

RXLEV-NCELL-1		See note 14

BCCH-FREQ-NCELL-1		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 14

BCCH-FREQ-NCELL-2		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 14

BCCH-FREQ-NCELL-3		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 14

BCCH-FREQ-NCELL-4		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		00 0000

BCCH-FREQ-NCELL-5		 0 0000

BSIC-NCELL-5			00 0000

RXLEV-NCELL-6		00 0000

BCCH-FREQ-NCELL-6		 0 0000

BSIC-NCELL-6			00 0000

GSM900 end:

DCS1800 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5.

Neighbour Cells Description

- Format			1024 range

- EXT IND			k = 1. Information Element carries complete BA.

k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 1. Non null for ARFCN 514, 549, 602,
			665, 810.

k = 2.  Non null for ARFCN 549, 602, 810.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 2. Non null ARFCN 514, 665.

SYSTEM INFORMATION TYPE 6:

Information Element			value/ remark

Protocol Discriminator		RR Management

Message Type			sys info 6

Cell Identity				default

LAI					default

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall not use DTX

- Radio-Link-Timeout	default

PLMN permitted			only NCC 1 permitted

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 14

RXLEV-SUB-SERVING-CELL	See note 14

MEAS-VALID			See note 15

RXQUAL-FULL-SERVING-CELL	See note 14

RXQUAL-SUB-SERVING-CELL	See note 14

NO-NCELL-M			4 neighbour cell measurement results

RXLEV-NCELL-1		See note 14

BCCH-FREQ-NCELL-1		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 14

BCCH-FREQ-NCELL-2		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 14

BCCH-FREQ-NCELL-3		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 14

BCCH-FREQ-NCELL-4		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		00 00000

BCCH-FREQ-NCELL-5		0 0000

BSIC-NCELL-5			00 0000

RXLEV-NCELL-6		00 0000

BCCH-FREQ-NCELL-6		0 0000

BSIC-NCELL-6			00 0000

DCS1800 end:

NOTE 14:	These actual values are not checked.

NOTE 15:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

#### 26.6.3.4	Measurement / DTX

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.3.3 in phase 1 version

-	one test have been splitted into 4

-	Handover is no more specified]

This test applies to both GSM900 and DCS1800 mobile stations.

##### 26.6.3.4.1	Conformance requirements

After the sending of the HANDOVER COMPLETE, the MS shall continuously send measurement reports in every SACCH blocks, the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the order of values in the MEASUREMENT REPORT message shall contain measurement results for the 6 strongest BCCH carriers among those monitored by the MS. Further, in a quiet environment, the DTX-USED field shall be set by the MS to "DTX used".

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.6.3.4.2	Test purpose

To test that, in the case of the MS using DTX and the SS indicating that power control is in use, the MS reports appropriate results.

##### 26.6.3.4.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BCC	ARFCN	ARFCN	Cell identity

(GSM900)	(DCS1800)

Serving, S1	-60	1	3	002	514	0001H

Neighbour, N1	-85	1	5	008	530	0002H

Neighbour, N2	-80	1	7	014	602	0003H

Neighbour, N3	-75	1	1	020	665	0004H

Neighbour, N4	-55	1	3	026	762	0005H

Neighbour, N5	-50	1	5	032	686	0006H

Neighbour, N6	-45	1	7	038	549	0007H

Neighbour, N7	-40	1	1	044	810	0008H

In the serving cell, the DTX indicator is set to "MS shall use discontinous transmission"

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Mobile Station:

The MS is in the active state of a call (U10).

The MS has just completed a handover into the serving cell, S1.

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Support for transparent data services only: yes/no.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed twice.

With the MS having a call in progress on an arbitrary cell, the MS is handed over to cell S1. On cell S1, the SS sends SYSTEM INFORMATION TYPE 5 & 6 (on the second iteration of the test the SS also sends SYSTEM INFORMATION TYPE 5bis) on the SACCH with all 8 of the BCCHs "on air" indicated in the BA. Cell S1 also indicates that DTX shall be used. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest carriers have been obtained and that DTX has been used. (The MS is positioned in an environment free from acoustic noise.)

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed twice for execution counter, k = 1, 2.

Since when k = 1, SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT (and when k = 2 an additional SYSTEM INFORMATION TYPE 5bis is included) messages are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

GSM900 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Neighbour Cells Description

- Format Identifier		bit map 0

- BCCH Allocation Sequence	1

- BCCH Allocation ARFCN	only channel numbers 2, 8, 14, 20, 26,
			32, 38, and 44 belong to the BCCH allocation.

- EXT IND			k = 1. Information Element carries the
			complete BA.

k = 2. Information Element carries only a part

of the BA.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part
			of the BA.

- W(i)				Only channel 500 belongs to the BCCH
			allocation.

SYSTEM INFORMATION TYPE 6:

Information Element			value/remark

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall use DTX

- Radio-Link-Timeout		8

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

- BA-used			0

- DTX-used			DTX was used (note 18)

- RXLEV-FULL-SERVING-CELL	See note 16

- RXLEV-SUB-SERVING-CELL	See note 16

- MEAS-VALID			See note 17

- RXQUAL-FULL-SERVING-CELL	See note 16

- RXQUAL-SUB-SERVING-CELL	See note 16

- NO-NCELL-M			6 neighbour cell measurement results

- RXLEV-NCELL-1		See note 16

- BCCH-FREQ-NCELL-1		Shall not correspond to N1 or N2

- BSIC-NCELL-1		Corresponds to that of BCCH-FREQ-NCELL-1

- RXLEV-NCELL-2		See note 16

- BCCH-FREQ-NCELL-2		Shall not correspond to N1 or N2

- BSIC-NCELL-2		Corresponds to that of BCCH-FREQ-NCELL-2

- RXLEV-NCELL-3		See note 16

- BCCH-FREQ-NCELL-3		Shall not correspond to N1 or N2

- BSIC-NCELL-3		Corresponds to that of BCCH-FREQ-NCELL-3

- RXLEV-NCELL-4		See note 16

- BCCH-FREQ-NCELL-4		Shall not correspond to N1 or N2

- BSIC-NCELL-4		Corresponds to that of BCCH-FREQ-NCELL-4

- RXLEV-NCELL-5		See note 16

- BCCH-FREQ-NCELL-5		Shall not correspond to N1 or N2

- BSIC-NCELL-5		Corresponds to that of BCCH-FREQ_NCELL-5

- RXLEV-NCELL-6		See note 16

- BCCH-FREQ-NCELL-6		Shall not correspond to N1 or N2

- BSIC-NCELL-6		Corresponds to that of BCCH-FREQ-NCELL-6

GSM900 end:

DCS1800 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5.

Neighbour Cells Description

- Format			1024 range

- EXT IND			k = 1. Information Element carries complete BA.

k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 1. Non null for ARFCN 514, 530, 549, 602
				665, 686, 762, 810.

k = 2.  Non null for ARFCN 549, 602, 665, 686,
				810.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 2. Non null ARFCN 514, 530, 762.

SYSTEM INFORMATION TYPE 6:

Information Element			value/ remark

Protocol Discriminator		RR Management

Message Type			sys info 6

Cell Identity				default

LAI					default

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall use DTX

- Radio-Link-Timeout	default

PLMN permitted			only NCC 1 permitted

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was used (see note 18)

RXLEV-FULL-SERVING-CELL	See note 16

RXLEV-SUB-SERVING-CELL	See note 16

MEAS-VALID			See note 17

RXQUAL-FULL-SERVING-CELL	See note 16

RXQUAL-SUB-SERVING-CELL	See note 16

NO-NCELL-M			6 neighbour cell measurement results

RXLEV-NCELL-1		See note 16

BCCH-FREQ-NCELL-1		Shall not correspond to N1 or N2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 16

BCCH-FREQ-NCELL-2		Shall not correspond to N1 or N2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 16

BCCH-FREQ-NCELL-3		Shall not correspond to N1 or N2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 16

BCCH-FREQ-NCELL-4		Shall not correspond to N1 or N2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		See note 16

BCCH-FREQ-NCELL-5		Shall not correspond to N1 or N2

BSIC-NCELL-5			Corresponds to that of BCCH-FREQ-NCELL-5

RXLEV-NCELL-6		See note 16

BCCH-FREQ-NCELL-6		Shall not correspond to N1 or N2

BSIC-NCELL-6			Corresponds to that of BCCH-FREQ-NCELL-6

DCS1800 end:

NOTE 16:	These actual values are not checked.

NOTE 17:	The Measurement Valid Indication shall be set to valid within the second SACCH block after the HANDOVER COMPLETE message at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

NOTE 18:	For an MS that only supports transparent data services, the value of DTX-used shall not be checked.

#### 26.6.3.5	Measurement / Frequency Formats

This test applies to both GSM900 and DCS1800 mobiles stations.

##### 26.6.3.5.1	Conformance Requirement

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the values in the MEASUREMENT REPORT message shall contain measurement results for the cells on which the mobile is allowed to report.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.6.3.5.2	Test Purpose

To test that, when the SS gives information about neighbouring cells, the MS reports appropriate results.

##### 26.6.3.5.3	Method of test

Initial Conditions

System Simulator:

2 cells with the following settings:

Transmitter	Level	NCC	BCC	ARFCN	ARFCN	Cell Identity

(GSM900)	(DCS1800)

Serving, S1	-60	1	3	002	715	0001H

Neighbour, N1	-85	1	5	008	815	0002H

With the exception of the Cell Allocation, the rest of the parameters for both cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed three times.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5, 5bis & 6 on the SACCH. Both of the BCCHs "on air" are indicated in the BA. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that no measurement results have been obtained.

For each iteration of the test the frequency format of the BA list contained in the System Information 5 and 5bis message shall change according to the specific message contents.

Maximum Duration Of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, K = 1, 2, 3.

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION 5bis, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

GSM900 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Neighbour Cells Description

Format Identifier			Bit Map 0.

BCCH Allocation Sequence		1

BCCH Allocation ARFCN		Channel numbers 2, 6 and 8  belong to the

BCCH allocation.

- EXT IND			Information Element carries only a part

of the BA.

SYSTEM INFORMATION TYPE 5bis :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

Format Identifier			K = 1. Range 1024 Format

K = 2. Range 512 Format

K = 3. Variable Bit Map.

- EXT IND			Information Element carries only a part

of the BA.

- W(i)				K = 1,2. 500, 530, 595, 965, 1000, 715, 815, 0

K = 3.    965, 1000, 0, 2, 6

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 19

RXLEV-SUB-SERVING-CELL	See note 19

MEAS-VALID			See note 20

RXQUAL-FULL-SERVING-CELL	See note 19

RXQUAL-SUB-SERVING-CELL	See note 19

NO-NCELL-M			2 neighbour cell measurement result

RXLEV-NCELL-1		See note 19

BCCH-FREQ-NCELL-1		Shall correspond to S1 or N1, ie, it shall be 0
			or 2.

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 19

BCCH-FREQ-NCELL-2		Shall correspond to S1 or N1, i.e. it shall be 0
			or 2.

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		00 0000

BCCH-FREQ-NCELL-3		 0 0000

BSIC-NCELL-3			00 0000

.     .     .			.       .

.     .     .			.       .

RXLEV-NCELL-6		00 0000

BCCH-FREQ-NCELL-6		 0 0000

BSIC-NCELL-6			00 0000

GSM900 end:

DCS1800 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5.

Neighbour Cells Description

- Format 			K = 1. Range 1024 Format

K = 2. Variable Bit Map

K = 3. Range 128 Format

- EXT IND			Information Element carries only a
			part of the BA.

- W(i)				K = 1. Non null for 500, 530, 595, 715,
			815, 965, 1000, 0

K = 2. Non null for 965, 1000, 0, 2, 6, 8

K = 3. Non null for 695, 715, 800

SYSTEM INFORMATION TYPE 5bis

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			K = 1. Bit Map 0.

K = 2. Range 512 Format 							K = 3. Range 256 Format

- EXT IND			Information Element carries only a part

of the BA.

- W(i) / BCCH Allocation		K = 1. Non null for 2, 6, 8

K = 2. Non null for 500, 530, 595, 715, 815, 965

K = 3. Non Null for 815, 965, 1000, 0, 2, 6

SYSTEM INFORMATION TYPE 6:

Information Element			value/ remark

Protocol Discriminator		RR Management

Message Type			sys info 6

Cell Identity				default

LAI					default

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall not use DTX

- Radio-Link-Timeout	default

PLMN permitted			only NCC 1 permitted

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 19

RXLEV-SUB-SERVING-CELL	See note 19

MEAS-VALID			See note 20

RXQUAL-FULL-SERVING-CELL	See note 19

RXQUAL-SUB-SERVING-CELL	See note 19

NO-NCELL-M			2 neighbour cell measurement results

RXLEV-NCELL-1		See note 19

BCCH-FREQ-NCELL-1		K= 1. Shall correspond to S1 or N1, ie,  it shall
				be 6 or 7

K= 2. Shall correspond to S1 or N1, ie,  it shall
				be 6 or 7

K= 3. Shall correspond to S1 or N1, ie,  it shall
				be 3 or 5

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 19

BCCH-FREQ-NCELL-2		K= 1. Shall correspond to S1 or N1, ie,  it shall
				be 6 or 7

K= 2. Shall correspond to S1 or N1, ie,  it shall
				be 6 or 7

K= 3. Shall correspond to S1 or N1, ie,  it shall
				be 3 or 5

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		00 0000

BCCH-FREQ-NCELL-3		 0 0000

BSIC-NCELL-3			00 0000

.     .     .			.       .

.     .     .			.       .

RXLEV-NCELL-6		00 0000

BCCH-FREQ-NCELL-6		 0 0000

BSIC-NCELL-6			00 0000

NOTE 19:	These actual values are not checked.

NOTE 20:	The Measurement Valid Indication shall be set to valid within the second SACCH block at  the latest.

### 26.6.4	Test of the channel assignment procedure

An intracell change of channel can be requested by upper layers in order to change the channel type, or it may be initiated by the RR-sublayer, e.g. for an intra cell handover. This change is performed using the channel assignment procedure. If the procedure is incorrectly implemented in the MS, the establishment and maintenance of connections is endangered. This applies for the successful case and for the assignment failure: the MS's correct return to the old channel after assignment failure is a necessary part of the GSM system design.

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The phase 2 CR from France Telecom is not yet included.??

- The succesful procedure is not tested for SDCCH-only MSes.

- Mobile allocation needs further description; was the cell allocation present in SYSTEM INFO? (for 6.4.1)

]

#### 26.6.4.1	Dedicated assignment / successful case

[Dissimilarities to 11.10 phase 1 - editorial notes :

- This section was part of II.5.3.6.4.2 in phase 1 version; for non-halfrate MSes the test was the same with the exceptions noted below; for F+H-MSes, the test is different, because the TCH/H cases are tested at the end, not in the middle, and both hopping and non-hopping are checked.

-	N(SD) is used instead of V(SD)

- In phase 1 it was required that the MS uses the commanded power level on the assigned channels; in phase 2 it is checked that the MS reports that level.

- A postamble has been added.

]

This test is only applicable to an MS supporting a TCH.

##### 26.6.4.1.1	Conformance requirements

1.	Upon receipt of the ASSIGNMENT COMMAND message, the mobile station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the main signalling links).

2.	MM-messages and CM-messages using SAPI=0 sent from the mobile station to the network can be duplicated by the data link layer in the following case:

a channel change of dedicated channels is required (assignment or handover procedure) and the last layer 2 frame has not been acknowledged by the peer data link layer before the mobile station leaves the old channel.

In this case, the mobile station does not know whether the network has received the message correctly. Therefore, the mobile station has to send the message again after the new dedicated channel is established.

3.	An ASSIGNMENT COMMAND message may indicate a frequency change in progress, with a starting time and possibly alternative channel descriptions.

In the case of the reception of an ASSIGNMENT COMMAND message which contains only the description of a channel to be used after the starting time, and if the starting time has not already elapsed,the mobile station shall wait up to the starting time before accessing the channel.

4.	The MS shall establish the link with the power level specified in the ASSIGNMENT COMMAND message.

The MS shall confirm the power control level that it is currently employing in the uplink SACCH L1 header. The indicated value shall be the power control level actually used by the mobile for the last burst of the previous SACCH period.

5.	The MS shall apply the hopping frequencies specified in ASSIGNMENT COMMAND message in the Mobile Allocation IE or the Frequency List IE at the time of accessing the new channel using the last received Cell Allocation.

6.	After receipt of the ASSIGNMENT COMMAND the MS shall perform the assignment and return an ASSIGNMENT COMPLETE without undue delay.

References

1, 3, 5.	GSM 04.08, subclause 3.4.3.

2.	GSM 04.08, subclause 3.1.4.3.

4.	GSM 04.08, subclause 3.4.3; GSM 05.08, subclause 4.2.

6.	GSM 04.13, subclause 5.2.4.

##### 26.6.4.1.2	Test purpose

1.	To verify that upon receipt of an ASSIGNMENT COMMAND, the MS switches to the channel defined in the ASSIGNMENT COMMAND, establishes the link and sends an ASSIGNMENT COMPLETE message. This is tested for an MS supporting TCH in the special cases of a transition

1.1	from non-hopping SDCCH to hopping TCH/F using a different timeslot

1.2	from hopping TCH/F to non-hopping TCH/F using a different timeslot

1.3	from non-hopping TCH/F to non-hopping TCH/F using a different timeslot

1.4	from non-hopping TCH/F to hopping TCH/H using a different timeslot; this test purpose is only applicable if the MS supports TCH/H

1.5	from hopping TCH/H to non-hopping TCH/H using a different timeslot; this test purpose is only applicable if the MS supports TCH/H

1.6	from non-hopping TCH/H to hopping TCH/F using a different timeslot; this test purpose is only applicable if the MS supports TCH/H.

2.	To verify that an MS supporting TCH, having sent an MM- or CM message which was not acknowledged on L2 before the channel assignment procedure was initiated and before the MS has left the old channel, repeats that message after completion of the assignment procedure without incrementing N(SD). This is tested in the special case of MM message AUTHENICATION RESPONSE.

3.	To verify that, if an MS supporting TCH has received an ASSIGNMENT COMMAND message which contains only the description of a channel to be used after the starting time, and if the starting time has not already elapsed, the mobile station shall wait up to the starting time before accessing the channel.

4.	To verify that an MS supporting TCH, having received an ASSIGNMENT COMMAND, having sent an SABM frame to establish the main signalling link on the assigned channel, reports the power level specified in the ASSIGNMENT COMMAND message, in the uplink SACCH L1 header of the SACCH message sent in the SACCH period following the transmission of the SABM frame.

5.	To verify that an MS supporting TCH, having received an ASSIGNMENT COMMAND, is able in the case of frequency hopping to decode the Mobile Allocation and Frequency List IEs correctly and applies the specified frequencies using the correct Cell Allocation.

6.	To verify that after receipt of the ASSIGNMENT COMMAND the MS returns an ASSIGNMENT COMPLETE without undue delay.

##### 26.6.4.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters except:

GSM900:

BCCH ARFCN =20

Throughout the test, the CA broadcast in System Information 1 is (10, 17, 20, 26, 34, 42, 45, 46, 52, 59)

Note that the actual CA of the cell contains other frequencies.

DCS1800:

BCCH ARFCN =747

Throughout the test, the CA broadcast in System Information 1 is (734, 741, 747, 754, 759, 766, 773, 775, 779, 782)

Note that the actual CA of the cell contains other frequencies.

Mobile Station:

The MS is in the "idle, updated" state with a TMSI allocated.

Related PICS/PIXIT Statements

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	The supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS and allocates an SDCCH. Then 2 different channels are assigned with ASSIGNMENT COMMANDs. Each time the MS shall switch to the assigned channel, establish the link and send an ASSIGNMENT COMPLETE message.

Then the SS sends a AUTHENTICATION REQUEST message. The MS shall answer with an AUTHENTICATION RESPONSE message, which is not acknowledged on L2 by the SS. Immediately after the AUTHENTICATION RESPONSE message is received, the SS sends an ASSIGNMENT COMMAND. The MS shall switch to the assigned channel, establish the link with the commanded power level and send as ASSIGNMENT COMPLETE message. Then MS shall repeat the AUTHENTICATION RESPONSE message, with the same N(SD) value.

Then the SS sends an ASSIGNMENT COMMAND, which includes a Starting Time IE. The MS shall react as specified above, but this shall be done at the time specified in Starting Time IE.

For an MS not supporting TCH/H, the SS initiates the channel release procedure and the test ends here. For an MS supporting TCH/H, the channel assignment procedure is performed another three times, with half rate channels involved, and again it is checked that the MS correctly completes the procedures, before the SS initiates the channel release procedure.

Maximum Duration of Test

30 s

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGN	See specific message contents
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	ASSIGN COMMAND	See specific message contents
	6	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after 
				establishment of the main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 5.
	7	SS		The SS checks that the MS
				reports the requested power
				level in the layer 1 header
				of the SACCH message
				that is sent in the first SACCH
				muliframe following the SABM.
	8	SS -> MS	ASSIGN COMMAND	See specific message contents.
	9	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after 
				establishment of the main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 8.
	10	SS -> MS	AUTHENT REQUEST	
	11	MS -> SS	AUTHENT RESPONSE	This message is not L2
				acknowledged by the SS
	12	SS -> MS	ASSIGN COMMAND	See specific message contents
	13	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after 
				establishment of the main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 12.
	14	MS -> SS	AUTHENT RESPONSE	N(SD) shall be the same as in
				step 10
	15	SS -> MS	ASSIGN COMMAND	See specific message contents
	16	MS -> SS	SABM	Sent on the assigned channel.
				The frames containing the SABM shall
				have frame numbers x with 
				0 <=  (x - starting time) mod 42432 <= 17
	17	SS -> MS	UA
	18	MS -> SS	ASSIGN COMPLETE

A			This test part is performed
				if the MS doesn't support
				TCH/H (see PICS/PIXIT)
	A19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

B			This test part is performed
				if the MS supports
				TCH/H (see PICS/PIXIT)
	B19	SS -> MS	ASSIGN COMMAND	See specific message contents
	B20	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel
				after establishment of the
				main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 19.
	B21 	SS -> MS	ASSIGN COMMAND	See specific message contents
	B22	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel
				after establishment of the
				main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 22.
	B23	SS -> MS	ASSIGN COMMAND	See specific message contents
	B24	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel
				after establishment of the
				main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 23.
	B25	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents

GSM900 begin:

Step 3

IMMEDIATE ASSIGNMENT

As default message contents except

Channel Description

- Channel Type	SDCCH/8

TDMA offset	Chosen arbitrarily

- Timeslot Number	N, chosen arbitrarily

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	the ARFCN of the BCCH carrier

Step 5

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+1) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1 to 63)

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Frequency list IE	Not included

Channel Mode

- Mode	Signalling

Mobile Allocation	Indicates all of the CA (broadcast on the BCCH) except for the BCCH carrier.

Starting Time	Not included

Step 8

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+3) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	the ARFCN of the BCCH carrier

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	A non-signalling mode
		arbitrarily selected from the full
		rate capabilities declared for the MS

Frequency list IE	Not Included

Cell Channel Description	Bit map zero encodes (45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114)

Mobile Allocation	Not included

Starting Time	Not included

Step 12

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+4) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1 to 63)

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	Not included (thus the CA from 
		step 8 is used to decode the MA)

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Indicates frequencies (45, 46, 73, 74, 75, 76, 108, 114)

Starting Time	Not included

Step 15

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+5) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	10

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	signalling

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Not included

Starting Time	indicates (current frame
		number + 100 frames) mod 42432

Step 19

ASSIGNMENT COMMAND:

Channel Description

- Channel Type	TCH/H
	        TDMA offset	Chosen arbitrarily

- Timeslot Number	(N+6) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Frequency List IE.

- HSN	0

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	A non-signalling mode
		arbitrarily selected from the half
		rate capabilities declared for the MS

Cell Channel Description	Not included

Frequency list IE	Uses bit map 0 to indicate (10, 34, 52, 73, 108, 114)

Mobile Allocation	Not included

Starting Time	Not included

Step 21

ASSIGNMENT COMMAND:

Channel Description

- Channel Type	TCH/H

TDMA offset	Chosen arbitrarily

- Timeslot Number	(N+7) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	Chosen arbitrarily, but not the BCCH carrier

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	Not included

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Not included

Starting Time	Not incldued

Step 23

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+1) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1

to 63

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Cell Channel Description	Uses bit map 0 to indicate (17, 20)

Frequency list IE	Not included

Mobile Allocation	Indicates ARFCN 17 only

Starting Time	Not included

GSM900 end:

DCS1800 begin:

Step 3

IMMEDIATE ASSIGNMENT

As default message contents except

Channel Description

- Channel Type	SDCCH/8

TDMA offset	Chosen arbitrarily

- Timeslot Number	N, chosen arbitrarily

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	the ARFCN of the BCCH carrier

Step 5

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+1) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1 to 63)

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Frequency list IE	Not included

Channel Mode

- Mode	Signalling

Mobile Allocation	Indicates all of the CA (broadcast on the BCCH) except for the BCCH carrier.

Starting Time	Not included

Step 8

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+3) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	the ARFCN of the BCCH carrier

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	A non-signalling mode
		arbitrarily selected from the full
		rate capabilities declared for the MS

Frequency list IE	Not Included

Cell Channel Description	Use Range 128 to encode (773, 775, 779, 782, 791, 798, 829, 832, 844)

Mobile Allocation	Not included

Starting Time	Not included

Step 12

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+4) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1 to 63)

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	Not included (thus the CA from 
		step 8 is used to decode the MA)

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Indicates frequencies (773, 775, 779, 829, 832, 844)

Starting Time	Not included

Step 15

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+5) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	734

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	signalling

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Not included

Starting Time	indicates (current frame
		number + 100 frames) mod 42432

Step 19

ASSIGNMENT COMMAND:

Channel Description

- Channel Type	TCH/H
	        TDMA offset	Chosen arbitrarily

- Timeslot Number	(N+6) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Frequency List IE.

- HSN	0

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	A non-signalling mode
		arbitrarily selected from the half
		rate capabilities declared for the MS

Cell Channel Description	Not included

Frequency list IE	Uses Range 1024 to indicate (734, 741, 759, 766, 773, 832, 844)

Mobile Allocation	Not included

Starting Time	Not included

Step 21

ASSIGNMENT COMMAND:

Channel Description

- Channel Type	TCH/H

TDMA offset	Chosen arbitrarily

- Timeslot Number	(N+7) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	Chosen arbitrarily, but not the BCCH carrier

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	Not included

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Not included

Starting Time	Not incldued

Step 23

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+1) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1

to 63

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Cell Channel Description	Uses Variable Range Format to indicate (741, 747)

Frequency list IE	Not included

Mobile Allocation	Indicates ARFCN 741 only

Starting Time	Not included

DCS1800 end:

#### 26.6.4.2	Dedicated assignment / failure

##### 26.6.4.2.1	Dedicated assignment / failure / failure during active state

This test is only applicable to an MS supporting the call control protocol.

[ dissimilarities to 11.10 phase 1 - editorial notes :

- For an MS supporting TCH/H, the test is repeated for TCH/H.

- Instead of checking the power level, it is checked which power level the MS specifies in the L1 header.

]

###### 26.6.4.2.1.1	Conformance requirements

On the mobile station side, if a lower layer failure happens on the new channel before the ASSIGNMENT COMPLETE message has been sent, the mobile station deactivates the new channels, reactivates the old channels, reconnects the TCHs if any and triggers the establishment of the main signalling link. It then sends a ASSIGNMENT FAILURE message, cause "protocol error unspecified" on the main DCCH and resumes the normal operation, as if no assignment attempt had occurred. The operational parameters (e.g. ciphering mode) when returning on the old channel are those applied before the procedure.

References

GSM 04.08 section 3.4.3.3.

GSM 05.08 section 4.2.

GSM 05.05 section 4.1.1.

###### 26.6.4.2.1.2	Test purpose

To test that, when the MS fails to seize the new channel, the MS reactivates the old channel, reporting use of the last power level used on the old channel.

This is tested in the special cases of a transition:

- from TCH/F to hopping TCH/F in state U10 if the MS supports TCH/F and call control

- from TCH/H to hopping TCH/H in state U10 if the MS supports TCH/H and call control

###### 26.6.4.2.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters. The SS orders the MS to use a power level P. Where P is a power level within the range supported by the Type of MS.

Mobile Station:

The MS is in the active state (U10) of a mobile terminated call.

Related PICS/PIXIT Statements

- Support for TCH/F.

- Support for TCH/H.

- Support for state U10 of the Call Control protocol.

- Type of MS (P-GSM900 or EGSM or DCS1800).

- Power Class of MS.

Foreseen Final State of the MS

The active state (U10) of a mobile terminated call.

Test Procedure

The MS is in the active state (U10) of a mobile terminated call. The SS sends an ASSIGNMENT COMMAND allocating a new TCH/F, but does not activate the new channel. It is checked that the MS  triggers the establishment of the main signalling link on the old channel and then sends an ASSIGNMENT FAILURE.

Maximum Duration of Test

30 s

Expected Sequence

The test is repeated for execution counter k = 1, 2 if the MS supports TCH/H.

Step	Direction	Message	Comments

1	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, if k = 1,
				Channel Type = TCH/H, if k = 2.
				Power level specified in power command
				is different to P, again where P is a power
				level within the range supported by the Type
				of MS.
				The MS attempts (and fails) to establish a
				signalling link on the new channel.
	2			The MS re-establishes the signalling link on
				the old channel.
	3	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"
	4	SS		The SS checks that the MS
				reports power level P
				in the L1 header of the SACCH message
				that is sent in the first SACCH
				multiframe following the SABM.

Specific Message Contents

None

##### 26.6.4.2.2	Dedicated assignment / failure / general case

[ dissimilarities to 11.10 phase 1 - editorial notes :

New test.

]

###### 26.6.4.2.2.1	Conformance requirements

On the mobile station side, if a lower layer failure happens on the new channel before the ASSIGNMENT COMPLETE message has been sent, the mobile station deactivates the new channels, reactivates the old channels, reconnects the TCHs if any and triggers the establishment of the main signalling link. It then sends a ASSIGNMENT FAILURE message, cause "protocol error unspecified" on the main DCCH and resumes the normal operation, as if no assignment attempt had occurred. The operational parameters (e.g. ciphering mode) when returning on the old channel are those applied before the procedure.

References

GSM 04.08 section 3.4.3.3.

###### 26.6.4.2.2.2	Test purpose

To test that, when the MS fails to seize the new channel, the MS reactivates the old channel.

This is tested in the special cases of a transition:

-	from SDCCH to hopping TCH/F; this test part is only applicable if the MS supports TCH/F.

-	from non-hopping SDCCH to hopping TCH/H; this test part is only applicable if the MS supports TCH/H.

-	from hopping TCH/F to hopping TCH/H; this test part is only applicable if the MS supports TCH/H.

NOTE 21:	26.6.8.4 contains the case of an assignment failure SDCCH -> SDCCH.

###### 26.6.4.2.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT Statements

-	Support for TCH/F.

-	Support for TCH/H.

-	Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The MS is "idle updated".

Test Procedure

A mobile terminated RR connection is established on an SDCCH. The SS sends an ASSIGNMENT COMMAND message allocating a hopping TCH/F, but does not activate the assigned channels. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel and trigger the establishment of the main signalling link on the old channel. Then the MS shall send an ASSIGNMENT FAILURE.

For an MS not supporting TCH/H, the SS initiates the channel release procedure and the test ends here. For an MS supporting TCH/H, the test sequence is repeated another two times, with half rate channels involved, and again it is checked that the MS correctly returns to the old channels, before the SS initiates the channel release procedure.

Maximum Duration of Test

30 s

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	Channel Type : SDCCH
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, hopping.
				The MS attempts (and fails) to establish a
				signalling link on the new channel.
	6			The MS reestablishes the signalling link on
				the old channel.
	7	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"

A			This test part is performed 
				if the MS does not support TCH/H
	A8	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

B			This test part is performed 
				if the MS supports TCH/H.
	B8	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/H, hopping.
				The MS attempts (and fails) to establish a
				signalling link on the new channel.
	B9			The MS reestablishes the signalling link on
				the old channel.
	B10	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"
	B11	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, hopping.
	B12	MS -> SS	ASSIGN COMPLETE	Sent on the assigned channel after 
				establishment of the main signalling link.
	B13	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/H, hopping.
				The MS attempts (and fails) to establish a
				signalling link on the new channel.
	B14			The MS reestablishes the signalling link on
				the old channel.
	B15	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"
	B16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents

None.

### 26.6.5	Test of handover

With the Handover procedure, it is possible to completely alter the channels allocated to a MS. This makes it possible in particular to switch a call in progress from one cell to another. The procedure is always initiated by the network and with the MS in a dedicated mode.

Sections 26.6.5.1 - 26.6.5.4 contain generic test procedures to be used for executing successful Handover tests. Table 1 contains a summary of the different combinations of parameters which have to be tested, together with a reference to the appropriate generic test procedure. If a test uses a channel rate which the MS under test does not support, the test shall be skipped.

Table 1

From	To	Timing	Start	Syn	State	Section	Exec
		Adv.	Time	?	of call	26.	Counter

TCH/F, no FH	TCH/F, no FH	20	none	no	U10	6.5.1	1
TCH/F, no FH	TCH/F, FH	arbitrary	none	no	U10	6.5.1	2
TCH/F, FH	TCH/F, no FH	20	1.1s	no	U10	6.5.1	3
TCH/H, FH	TCH/H, no FH	20	none	no	U10	6.5.1	6
TCH/H, no FH	TCH/H, FH	arbitrary	none	no	U10	6.5.1	7
TCH/H, FH	TCH/H, FH	20	1.1s	no	U10	6.5.1	5
TCH/F, no FH	TCH/H, FH	arbitrary	none	no	U10	6.5.1	4
TCH/H, FH	TCH/F, no FH	arbitrary	none	no	U10	6.5.1	8
SDCCH/4, no FH	TCH/F, FH	20	none	no	estab	6.5.2	1
SDCCH/4, no FH	TCH/H, FH	20	none	no	estab	6.5.2	2
SDCCH/4, no FH	SDCCH/8, FH	20	none	no	estab	6.5.2	3
SDCCH/8, no FH	SDCCH/8, FH	arbitrary	none	no	estab	6.5.2	4
TCH/F, no FH	TCH/H, no FH	20	none	no	estab	6.5.2	5
TCH/H, FH	TCH/F, FH	20	none	no	estab	6.5.2	6
TCH/F, FH	TCH/F, FH	arbitrary	none	no	estab	6.5.2	7
SDCCH/8, FH	TCH/F, no FH	20	none	no	estab	6.5.2	8
SDCCH/8, no FH	TCH/F, FH	20	none	no	estab	6.5.2	9
SDCCH/8, no FH	TCH/H, FH	arbitrary	none	no	estab	6.5.2	10
TCH/F, FH	TCH/F, no FH	2k+y	none	yes	U10	6.5.3	1
TCH/H, FH	TCH/H, no FH	2k+y	none	yes	U10	6.5.3	2
SDCCH/8, FH	SDCCH/8, FH	2k+y	none	yes	estab	6.5.4	1
SDCCH/8, FH	SDCCH/4, no FH	2k+y	1.1s	yes	estab	6.5.4	2
TCH/F, no FH	TCH/F, FH	2k+y	none	yes	estab	6.5.4	3
SDCCH/8, no FH	TCH/F, no FH	2k+y	none	yes	estab	6.5.4	4

Table 2

TCH/FS	TCH/HS	SDCCH

n	10-20	5-10	2-5

In addition to the successfull case of Handover, 2 unsuccessful cases shall be tested. These tests are described in 26.6.5.8 and 26.6.5.9.

#### 26.6.5.1	Handover / successful / active call / non-synchronized

##### 26.6.5.1.1	Conformance requirements

The MS shall correctly apply the handover procedure in the non-synchronized case when a call is in progress and when handover is performed from a TCH/F without frequency hopping towards a TCH/F without frequency hopping.

The MS shall correctly apply the handover procedure in the non-synchronized case when a call is in progress and when handover is performed from a TCH/H without frequency hopping to a TCH/H with frequency hopping. This does not apply to MSs not supporting TCH/H.

References

GSM 04.08 sections 3.4.4 and 9.1.15

GSM 04.13 section 5.2.6.2.

##### 26.6.5.1.2	Test purpose

To test that when the MS is ordered to make a non-synchronized handover it continuously sends access bursts on the main DCCH until it receives a PHYSICAL INFORMATION message from the SS. To test that the MS correctly handles the values of any Starting Time IE in the HANDOVER COMMAND message in the case when none of the information elements referring to before the starting time are present. To test that the MS correctly handles the Timing Advance IE in the PHYSICAL INFORMATION message. To test that the MS activates the new channel correctly and transmits the HANDOVER COMPLETE message without undue delay.

##### 26.6.5.1.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B with same LAI, default parameters except:

GSM900 :

Cell A has :

BCCH ARFCN = 20

Cell Allocation = (10, 17, 20, 26, 34, 42, 45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114)

Cell B has :

BCCH ARFCN = 40

Cell Allocation = (14, 18, 22, 24, 30, 31, 38, 40, 60, 66, 73, 74, 75, 76, 108, 114)

The frame numbers of cells A and B shall be different by 100.

The timebase of cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

DCS1800 :

Cell A has :

BCCH ARFCN = 747

Cell Allocation = ( 734, 741, 747, 754, 759, 762, 766, 767, 773, 775, 779, 782, 791, 798, 829, 832, 844 )

Cell B has :

BCCH ARFCN = 764

Cell Allocation = ( 739, 743, 746, 749, 756, 758, 761, 764, 771, 779, 782, 791, 798, 829, 832, 844 )

The Cell Allocation of both Cell A and Cell B shall be coded using range 256 format.

The frame numbers of cells A and B shall be different by 100.

The timebase of cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

Mobile Station:

The MS is in the active state (U10) of a call on cell A.

[editorial note : since not all MSs support such high power levels, it might be better to choose e

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Support for speech : yes/no

supported radio interface rates : 12kbps, 6kbps, 3.6kbps

Type of Mobile Station ( P-GSM900 or EGSM or DCS1800 ).

Foreseen Final State of the MS

The active state (U10) of a call on cell A.

Test Procedure

This procedure is repeated for execution counter M = 1 to 8. (See table 1).

The MS is in the active state (U10) of a call. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall (at the time defined by the Starting Time information element, if included in the message) begin to send access bursts on the new DCCH of the target cell. The SS observes the access bursts and after receiving n (n being randomly drawn between values according to table 2 of section 26.6.5) access bursts, the SS sends one PHYSICAL INFORMATION message with a Timing Advance as specified in table 1 of section 26.6.5. The MS shall activate the channel in sending and receiving mode. The MS shall establish a signalling link. The MS shall be ready to transmit a HANDOVER COMPLETE message, before 'x' MS after the end of the PHYSICAL INFORMATION message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in TS GSM 04.13. The value of 'x' depends upon the target channel and is specified in the specific message contents section.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for an execution counter M = 1, 2, 3 for an MS which only supports TCH/F.

This sequence is performed for an execution counter M = 1, 2.. 8 for an MS which supports TCH/F and H

Step	Direction	Message	Comments

0	MS -> SS		The MS and SS are in the active state of a call
				on the channel described below.

	1	SS -> MS	HANDOVER COMMAND	See Specific message contents
	2	MS -> SS	HANDOVER ACCESS	Repeated on every burst of the uplink main
				DCCH until reception of PHYSICAL
				INFORMATION. Handover Reference as
				included in the HANDOVER COMMAND.  If 
				the HANDOVER COMMAND includes a 
				starting time IE then the first HANDOVER 
				ACCESS message shall be transmitted in 
				the indicated frame (unless the indicated 
				frame is not used by that channel, in which 
				case the next frame used by that channel 
				shall be used)
	3	SS -> MS	PHYSICAL INFO	Sent after reception of n HANDOVER
				ACCESS messages. See specific message
				contents.
	4	MS -> SS	SABM	Sent without information field
	5	SS -> MS	UA
	6	MS -> SS	HANDOVER COMPLETE	The message shall be ready to be 						transmitted before 'x' ms after the 						completion of step 3.
	7	MS -> SS		The MS and SS are in the active state of a 
				call on the channel described below.

Specific Message Contents For Mobiles Supporting Speech

For M = 1 :

P-GSM900

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non hopping mode on cell B.

For M = 2 :

GSM900

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			20

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Chosen arbitrarily, but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Channel Sequence IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Synchronisation Indication IE is not included.

Channel Mode IE is not included.

Frequency Channel Sequence after time

- Frequency Channel Sequence		Allocates the following 12

frequencies ( 10, 17, 20, 26,

59, 66, 73, 74, 75, 76,108,

114 )

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			747

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Chosen arbitrarily, but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Short List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Synchronisation Indication IE is not included.

Channel Mode IE is not included.

Frequency Short List after time

- Frequency List				Use Range 256 to encode

the following 9 frequencies :

( 747, 775, 779, 782, 791, 798, 829, 832, 844 )

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in hopping mode on cell A.

For M = 3 :

GSM900

Step 0 : The MS and SS are using a full rate TCH in hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Out of range timing advance shall

trigger a handover failure procedure.

Starting Time				Indicates the frame number of cell B

that will occur approximately 1.1

seconds ( 238 frames have

elapsed ) after  the HANDOVER

COMMAND is sent by cell A.

Mode of first channel			Speech full rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a full rate TCH in hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Out of range timing advance shall

trigger a handover failure procedure.

Starting Time				Indicates the frame number of cell B

that will occur approximately 1.1

seconds ( 238 frames have

elapsed ) after  the HANDOVER

COMMAND is sent by cell A.

Mode of first channel			Speech full rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

For M = 4 :

GSM900

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			20

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Mobile Allocation.

- HSN					zero ( this gives cyclic hopping )

Synchronisation Indication  IE is not included.

Cell Channel Description			Use bit map 0 to encode the

complete CA of Cell A

Mobile Allocation after time			Indicates all of the CA of cell A

except for the BCCH frequency.

Mode of First Channel			Speech half rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			747

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Mobile Allocation.

- HSN					zero ( this gives cyclic hopping )

Synchronisation Indication  IE is not included.

Cell Channel Description			Use Range 512 to encode the

complete CA of Cell A

Mobile Allocation after time			Indicates all of the CA of cell A

except for the BCCH frequency.

Mode of First Channel			Speech half rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell A.

For M = 5 :

GSM900

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily, but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency List after time

- Frequency List				use bit map 0 to allocates

the following 12 frequencies :

( 14, 18, 22, 24, 60, 66, 73, 74, 75, 76,108, 114 )

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Starting Time				Indicates the frame number of cell B.

that will occur approximately 1.1

seconds ( 238 frames have

elapsed ) after  the HANDOVER

COMMAND is sent by cell A.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency List after time

- Frequency List				Use Range 1024 to allocate the

following 12 frequencies :

(749, 758, 761, 764, 771, 779, 782, 791, 798, 829, 832, 844 )

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Starting Time				Indicates the frame number of cell B

that will occur approximately 1.1

seconds ( 238 frames have

elapsed ) after  the HANDOVER

COMMAND is sent by cell A.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell B.

For M = 6 :

GSM900

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			20

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell A.

Synchronisation Indication IE not included.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in non-hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			747

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell A.

Synchronisation Indication IE not included.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in non-hopping mode on cell A.

For M = 7 :

GSM900

Step 0 : The MS and SS are using a half rate TCH in non-hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Channel Sequence IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency Channel Sequence after time

- Frequency Channel Sequence		Allocates the following 8

frequencies :

( 40, 66, 73, 74, 75, 76,108, 114 )

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of first channel			speech half rate.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a half rate TCH in non-hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency List after time

- Frequency List				Use Variable Bit Map to encode the

following 8 frequencies :

( 764, 779, 782, 791, 798, 829, 832, 844 )

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of first channel			speech half rate.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell B.

For M = 8 :

GSM900

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			20

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF channel

- ARFCN					20

Synchronisation Indication IE not included.

Mode of First Channel			Speech full rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non-hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			747

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF channel

- ARFCN					747

Synchronisation Indication IE not included.

Mode of First Channel			Speech full rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non-hopping mode on cell A.

Specific Message Contents For Mobiles not Supporting Speech

If the mobile station supports half rate, then the 12 kbps radio interface rate is not used for this test. With this restriction, the radio interface rate is selected arbitrarily from those support.

The message contents shall be the same for the declared type of Mobile Station ( either P-GSM900 or DCS1800 ) supporting speech , except for :

M = 3 and 8 :

HANDOVER COMMAND

Information Element				value/remarks

Mode of first channel			The full rate radio interface

rate that is in use.

For M = 4 and 7 :

HANDOVER COMMAND

Information Element				value/remarks

Mode of first channel			The same radio interface

rate as M = 3, but for a half

rate channel.

#### 26.6.5.2	Handover / successful / call under establishment / non-synchronized

##### 26.6.5.2.1	Conformance requirements

The MS shall correctly apply the handover procedure from SDCCH/8, TCH/F or TCH/H with or without frequency hopping to SDCCH/8, TCH/F or TCH/H with or without frequency hopping in the non-synchronized case during call establishment. The mobile shall correctly apply the handover procedures from non frequency hopping SDCCH/4 to SDCCH/8, TCH/F or TCH/H with or without frequency hopping. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

References

GSM 04.08, sections 3.1.4.2, 3.4.4 and 9.1.15

GSM 04.13, section 5.2.6.2

##### 26.6.5.2.2	Test purpose

To test that when the MS is ordered to make a non-synchronized handover, it continuously sends access bursts on the main DCCH until it receives a PHYSICAL INFORMATION message from the SS. To test that the MS correctly takes the values of the Timing Advance information element in the PHYSICAL INFORMATION message into account. To test that the MS activates the new channel correctly and transmits the HANDOVER COMPLETE message without undue delay. To test that the MS correctly retransmitsLayer 3 MM or CC messages, that were not acknowledged by Layer 2 before the Handover, after completion of the Handover.

##### 26.6.5.2.3	Method of test

Initial Conditions

System Simulator:

2 cells A and B with same LAI, default parameters, except:

GSM900 :

Cell A has :

BCCH ARFCN = 20

Cell Allocation = (10, 17, 20, 26, 34, 42, 45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114)

PLMN colour code, NCC = as defaults.

BS colour code, BCC = as defaults.

Cell B has :

BCCH ARFCN = 40

Cell Allocation = (14, 18, 22, 24, 30, 31, 38, 40, 60, 66, 73, 74, 75, 76, 108, 114)

PLMN colour code, NCC = 3.

BS colour code, BCC = 0.

Both cells send SYSTEM INFORMATION TYPE 1 messages containing the complete Cell Allocation of the cell, using bit map 0 format.

The timebase of Cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

For execution counter M = 1, 2 and 3 a combined CCH/SDCCH is used.

For execution counter M = 4 to 10 a non combined SDCCH is used.

DCS1800 :

Cell A has :

BCCH ARFCN = 747

Cell Allocation = (734, 741, 747, 754, 759, 762, 766, 767, 773, 775, 779, 782, 791, 	798, 829, 	832, 844)

PLMN colour code, NCC = as defaults.

BS colour code, BCC = as defaults.

Cell B has :

BCCH ARFCN = 764

Cell Allocation = ( 739, 743, 746, 749, 756, 758, 761, 764, 771, 779, 782, 791, 798, 	829, 832, 	844)

PLMN colour code, NCC = 3.

BS colour code, BCC = 0.

Both cells send SYSTEM INFORMATION TYPE 1 messages containing the complete Cell Allocation of the cell, using Range 512 format.

The timebase of Cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

For execution counter M = 1, 2 and 3 a combined CCH/SDCCH is used.

For execution counter M = 4 to 10 a non combined SDCCH is used.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Supported speech and data rates.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated" with TMSI allocated and camped on cell B.

Test Procedure

This procedure is repeated for execution counter M = 1, 2 .. 10 ( See Table 1 )

A Mobile Originating Call is initiated on Cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND message, ordering the MS to switch to cell B. The MS shall then begin to send access bursts on the new DCCH to cell B. The SS observes the access bursts and after receiving n (n being randomly drawn between values according to table 2 of section 26.6.5) access bursts, the SS sends one PHYSICAL INFORMATION message with a Timing Advance as specified in table 1 of section 26.6.5. The MS shall activate the channel in sending and receiving mode. The MS shall establish a signalling link. The MS shall be ready to transmit a HANDOVER COMPLETE message before x ms after the end of the PHYSICAL INFORMATION message, but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

The term 'ready to transmit' is defined in TS GSM 04.13. The value of 'x' depends upon the target channel and is specified in the specific message contents section.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

The sequence is performed for execution counter M = 1, 2..10 (unless a particular TCH is not supported).

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated on cell A.
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call,
				NECI not set to 1
	3	SS -> MS	IMM ASSIGNMENT	See specific message contents.
	  4	MS -> SS	CM SERVICE REQUEST	CM Service Type = Mobile Origina-
				ting Call Establishment.
	  5	SS -> MS	CIPH MODE COMMAND
	  6	MS -> SS	CIPH MODE COMPLETE
	  7	MS -> SS	SETUP	Last L2 frame not acknowledged by
				the SS.
	 8	SS -> MS	HANDOVER COMMAND	See specific message contents.
	 9	MS -> SS	HANDOVER ACCESS	Repeated on every burst of the uplink main 
				DCCH until reception of PHYSICAL
				INFORMATION. Handover Reference as
				included in the HANDOVER COMMAND
	 10	SS -> MS	PHYSICAL INFO	Sent after reception of n HANDOVER
				ACCESS message. Timing Advance as
				specified in table 1 of section
				26.6.5.
	11	MS -> SS	SABM	Sent without information field
	12	SS -> MS	UA
	 13	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted 
				before 'x' ms after the completion of step 10.
	 14	MS -> SS	SETUP	Same N(SD) as in step 7.
	 15	SS -> MS	CHANNEL RELEASE

Specific Message Contents For Mobiles Supporting Speech

M = 1

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Short List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Synchronisation IE is not included.

Frequency Short List after time

- Frequency Short List			Use Range 128 to encode the

following 11 frequencies :

( 756, 758, 761, 771, 779, 782, 791, 798, 829, 832, 844 )

Mode of the First Channel			Speech full rate

Step 13 : 'x' = 500

For GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Channel Sequence IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Synchronisation IE is not included.

Frequency Channel Sequence, after time

- Frequency List				Allocates the following 15

frequencies :

(14, 18, 22, 24, 30, 31, 38, 60, 66, 73, 74, 75, 76, 108, 114 )

Mode of the First Channel			Speech full rate.

Step 13 : 'x' = 500

M = 2

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set ( 1,

2.. 63)

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Cell Channel Description			Use Range 512 to encode the

following frequencies :

( 761, 764, 771, 779, 782, 791, 798, 829, 832 )

Mobile Allocation after time			Indicates  ( 791, 798, 829 ) only )

Mode of First Channel			Speech Half rate

Step 13 : 'x' = 750

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set ( 1,

2.. 63)

Synchronisation Indication IE is not included

Cell Channel Description			uses bit map 0 to encode :

{40, 66, 73, 74, 75, 76, 108, 114} only

Mobile Allocation after time		indicates channel {73, 74, 75} only.

Channel Mode IE

speech half rate

Step 13 : 'x' = 750

M = 3

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Short List IE.

- HSN					Zero ( this gives cyclic hopping )

Synchronisation Indication IE not included.

Frequency Short List after time

- Frequency Short List			Use Range 128 to encode the

following 2 frequencies ( 746, 779 )

Mode of First Channel			Signalling Only

Step 13 : 'x' = 1500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN				Chosen arbitrarily from the set

{1, 2, ..., 63}.

Synchronisation Indication IE is not included	Frequency list after time

- Frequency List			uses bit map 0 to allocate the

following 15 frequencies

{14, 18, 22, 24, 30, 31, 38, 60, 66, 73, 74, 75, 76, 108, 114}

Channel Mode IE

signalling only

Step 13 : 'x' = 1500

M = 4

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

- Timeslot number				Arbitrary value, but not zero.

- ARFCN					747

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					zero ( this gives cyclic hopping )

Frequency List after time

- Frequency List				Use Range 1024 to encode the

complete CA of Cell B

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Channel Mode IE is not included.

Step 13 : 'x' = 1500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

- Timeslot number				Arbitrary value, but not zero.

- ARFCN					20

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value, but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Channel Sequence IE.

Frequency Channel Sequence, after time

- Frequency channel sequence	Allocates the following 14

frequencies :

{14, 18, 22, 24, 30, 40, 31, 38, 60, 66, 73, 74, 75, 76, 108}

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Channel Mode IE is not included

Step 13 : 'x' = 1500

M = 5

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

- Channel Type				TCH/F + ACCH's

- Timeslot number				Arbitrary value but not zero.

- Hopping Channel			Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/H + ACCH's

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of cell B.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Signalling only

Step 13 : 'x' = 750

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

- Channel Type				TCH/F + ACCH's

- Timeslot number				Arbitrary value, but not zero.

- Hopping Channel			Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/H + ACCH's

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of cell B.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Signalling only

Step 13 : 'x' = 750

M = 6

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation

- Length					3 octets

- Contents				Indicates all of the CA of cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency List after time

- Frequency List				Use Range 256 to encode the

following 9 frequencies :

(746, 749, 756, 761, 764, 798, 829, 832,844)

Synchronisation Indication IE not included.

Channel Mode				Signalling Only

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation

- Length					3 octets

- Contents				Indicates all of the CA of cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Cell Channel Description IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Cell Channel Description			uses bit map 0 to encode the

complete CA of cell B.

Synchronisation Indication IE not included.

Channel Mode				Signalling Only

Step 13 : 'x' = 500

M = 7 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Zero

Mobile Allocation

- Length					3 octets

- Contents				Indicates only one frequency ( 844 )

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Cell Channel Description			Use Variable bit map to encode the

complete CA of cell B.

Mobile Allocation

Indicates all of the CA of cell B

except for the following three

frequencies : ( 764, 832 and 844 )

Mode of First channel			Speech full rate

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Zero

Mobile Allocation

- Length					3 octets

- Contents				Indicates only one frequency ( 114 )

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Synchronisation Indication IE is not included:

Frequency list after time.

- Frequency List IE		uses bit map 0 to allocate the

following 16 frequencies

{14, 18, 22, 24, 30, 31, 38, 40, 60, 66, 73, 74, 75, 76, 108, 114}

Mode of First channel			Speech full rate

Step 13 : 'x' = 500

M = 8 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation

- Length					3octets.

- Contents				Indicates only three frequencies :

( 773, 775, 779 ).

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell B, but

not the

BCCH carrier of Cell B.

Synchronisation Indication IE not included.

Mode of First Channel			Full rate speech

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value, but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation

- Length					3 octets.

- Contents				Indicates only three frequencies :

( 73, 74, 75 ).

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell B, but

not the

BCCH carrier of Cell B.

Mode of First Channel			Full rate speech

Step 13 : 'x' = 500

M = 9 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value, but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Short List IE.

- HSN					Zero ( this gives cyclic hopping )

Frequency Short List after time

- Frequency Short List			Use Range 256 to encode the

following 3 frequencies :

( 764, 779, 782 )

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Channel mode				Speech full rate

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code		3

- Base Station Colour Code		0

- BCCH Carrier Number		40

Channel Description

- Channel Type			TCH/F + ACCHs

- Timeslot number			zero

- Training Sequence Code		Chosen arbitrarily

- Hopping			RF hopping channel

- MAIO				Chosen arbitrarily from the set {0, 1}.

- HSN				Chosen arbitrarily from the set

{1, 2,..., 63}.

Synchronisation Indication IE is not included

Channel Mode IE

speech full rate

Frequency Channel Sequence, after time.

- Frequency Channel Sequence  IE	allocates the following two

frequencies {14, 114}

Channel mode				Speech full rate

Step 13 : 'x' = 500

M = 10 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value, but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Frequency List IE.

- HSN					Zero ( this gives cyclic hopping )

Frequency List after time

- Frequency List				Use Variable Bit Map to encode the

following 15 frequencies :

(739, 743,746,749, 756, 758, 764, 771, 779, 782, 791, 798, 829, 832, 844)

Synchronisation Indication IE is not included.

Channel mode				Speech half rate

Step 13 : 'x' = 750

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code		3

- Base Station Colour Code		0

- BCCH Carrier Number		40

Channel Description

- Channel Type			TCH/H + ACCHs

- TDMA offset			Chosen arbitrarily

- Timeslot number			Chosen arbitrarily but not zero

- Training Sequence Code		Chosen arbitrarily

- Hopping			RF hopping channel

- MAIO				Chosen arbitrarily from the set {0, 1}

- HSN				Chosen arbitrarily from the set

{1, 2,..., 63}.

Synchronisation Indication IE is not included

Frequency Channel Sequence, after time

- Frequency Channel Sequence IE	allocates the following two

frequencies {40, 114}

Channel Mode IE				speech half rate

Step 13 : 'x' = 750

Specific Message Contents For Mobiles not Supporting Speech

The message contents shall be the same for the declared type of Mobile Station ( either P-GSM900 or DCS1800 ) supporting speech , except for :

M = 1, 7, 8 and 9 :

HANDOVER COMMAND

Information Element				value/remarks

Mode of first channel			Arbitrary from those supported ( 12,

6, 3.6 kbps )

For M =  2 and 10  :

HANDOVER COMMAND

Information Element				value/remarks

Mode of first channel				Arbitrary from those supported

(6, 3.6 kbps )

#### 26.6.5.3	Handover / successful / active call / finely synchronized

##### 26.6.5.3.1	Conformance requirements

The MS shall correctly apply the handover procedure from TCH/F with frequency hopping to TCH/F without frequency hopping in the finely synchronized case when a call is in progress.

The MS shall correctly apply the handover procedure from TCH/H with frequency hopping to TCH/H without frequency hopping in the finely synchronized case when a call is in progress. This requirement does not apply to MSs not supporting TCH/H.

References

GSM 04.08 section 3.4.4, 9.1.14, 9.1.15 and 9.1.16.

GSM 04.13 section 5.2.6

GSM 05.05 section 4.1.1.

GSM 05.10, section 6.6

##### 26.6.5.3.2	Test purpose

To test that when the MS is ordered to make a finely synchronized handover to a synchronized cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly, taking into account the value of any Starting Time information element, power command and correctly calculating the timing advance to use. To test the MS activates the new channel correctly and transmits the HANDOVER COMPLETE message without undue delay.

##### 26.6.5.3.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI, default parameters, except:

The BCCH of cell A is sent k bit periods before the BCCH of cell B. The timing advance in cell A sent to the MS is y bit periods. k and y are selected such that 0 < 2k+y < 60

P-GSM900 :

Cell B has BCCH ARFCN = 40

DCS1800 :

Cell B has BCCH ARFCN = 764

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). The MS is using a power level P. Where P is a power level within the supported range of that type of MS.

[editorial note : since not all MSs support such high power levels, it might be better to choose e

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F and/or TCH/H.

Support for state U10 of the Call Control protocol.

Type of Mobile Station ( P-GSM900 or EGSM or DCS1800 ).

Power class of Mobile Station.

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell B).

Test Procedure

This procedure is repeated for execution counter M = 1 to 2. ( See Table 1 )

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall (at the time specified in the Starting Time information element, if included) send 4 access bursts, in 4 successive slots on the new DCCH to cell B. Then the MS shall establish a signalling link indicating the correct Timing Advance and power level and send a HANDOVER COMPLETE message.

The MS shall be 'ready to transmit' a HANDOVER COMPLETE message before 'x' ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in TS GSM 04.13. The value of 'x' depends upon the target channel and is specified in the specific message contents section.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter M = 1 for an MS which only supports TCH/F

This sequence is performed for execution counter M = 1 to 2 for an MS which supports TCH/F and H

Step	Direction	Message	Comments

0	MS -> SS		M = 1, The MS and SS are using a full rate 
				TCH in hopping mode on cell A.
				M = 2, The MS and SS are using a half rate 
				TCH in hopping mode on cell A.

	1	SS -> MS	HANDOVER COMMAND	See Specific Message Contents
	2	MS -> SS	HANDOVER ACCESS	See specific message contents. Four messages 
	3	MS -> SS	HANDOVER ACCESS	are transmitted to Cell B in 4 successive slots 
	4	MS -> SS	HANDOVER ACCESS	on the new DCCH.
	5	MS -> SS	HANDOVER ACCESS	
	6	MS -> SS	SABM	Sent without information field
	7	SS -> MS	UA
	8	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted 
				before 'x' ms after the completion of step 1. 
				See specific message contents.
	9	SS		The header of the next uplink SACCH is 
				examined and the Timing Advance and Power 
				Level indications are examined. The correct 
				timing advance shall be indicated. The power 
				level indication shall indicate the power 
				level used in the handover command.
	10	MS, SS		M =1, The MS and SS are using a full rate 
				TCH in non-hopping mode on cell B
				M = 2, The MS and SS are using a half rate 
				TCH in non-hopping mode on cell B.

Specific Message Contents

M = 1 :

GSM900

HANDOVER COMMAND

Information Element				value/remark

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel type				TCH/F + ACCHs

- Timeslot Number			Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					40

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by that type of MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step  8 : x = 650 ms

DCS1800

HANDOVER COMMAND

Information Element				value/remark

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel type				TCH/F + ACCHs

- Timeslot Number			Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					764

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by that type of MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step  8 : x = 650 ms

M = 2 :

GSM900

HANDOVER COMMAND

Information Element				value/remark

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot Number			Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					40

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by that type of MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Speech half rate

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step  8 : x = 900 ms

DCS1800

HANDOVER COMMAND

Information Element				value/remark

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot Number			Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					764

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by that type of MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Speech half rate

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step  8 : x = 900 ms

#### 26.6.5.4	Handover / successful / call under establishment / finely synchronized

##### 26.6.5.4.1	Conformance requirements

The MS shall correctly apply the handover procedure from SDCCH/8 or TCH/F with or without frequency hopping to SDCCH4, SDCCH/8 or TCH/F with or without frequency hopping in the finely synchronized case, during call establishment.

If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

References

GSM 05.10, section 6.6

GSM 04.08, section 3.4.4

GSM 04.08, section 3.1.4.2

GSM 04.13, section 5.2.6

##### 26.6.5.4.2	Test purpose

To test that when the MS is ordered to make a finely synchronized handover to a synchronized cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly, taking into account the value of any Starting Time information element, power command and correctly calculating the timing advance to use. To test that the MS correctly retransmits Layer 3 MM or CC messages that were not acknowledged by Layer 2 before the Handover, after completion of the Handover. To verify the MS transmits the HANDOVER COMPLETE message without undue delay.

##### 26.6.5.4.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI, default parameters, except:

The BCCH of cell A is sent k bit periods before the BCCH of cell B. The timing advance in cell A sent to the MS is y bit periods. k and y are selected such that 0 < 2k + y < 60

The frame numbers of cells A and B shall be different by 100.

GSM900 :

Cell A has :

BCCH ARFCN = 20

Cell Allocation = (10, 17, 20, 26, 34, 42, 45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114)

Cell B has :

BCCH ARFCN = 40

Cell Allocation = (14, 18, 22, 24, 30, 31, 38, 40, 60, 66, 73, 74, 75, 76, 108, 114)

DCS1800 :

Cell A has :

BCCH ARFCN = 747

Cell Allocation = (734, 741, 747, 754, 759, 762, 766, 767, 773, 775, 779, 782, 791, 	798, 829, 	832, 844 )

Cell B has :

BCCH ARFCN = 764

Cell Allocation = (739, 743, 746, 749, 756, 758, 761, 764, 771, 779, 782, 791, 798, 	829, 832, 	844 )

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A. The MS is using a power level P, where P is a power level within the supported range of that type of MS.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of Mobile Station ( P-GSM900 or EGSM or DCS1800 ).

Power class of Mobile Station.

Foreseen Final State of the MS

"Idle, updated" with TMSI allocated and camped on cell B.

Test Procedure

This procedure is repeated for execution counter M = 1, 2, 3, 4 ( See Table 1 )

A Mobile Originating Call is initiated on cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND message, ordering the MS to switch to cell B. The MS shall then (at the time specified in the Starting Time information element, if included) send 4 access bursts, in successive slots on the new DCCH to cell B. Then the MS shall establish a signalling link indicating the correct timing advance and power level and send a HANDOVER COMPLETE message. The MS shall be 'ready to transmit' the HANDOVER COMPLETE message before 'x' ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

The term 'ready to transmit' is defined in TS GSM 04.13. The value 'x' depends upon the target channel and is specified in the specific message contents section.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This procedure is repeated for execution counter M = 1, 2, 3, 4 (unless a particular TCH is not supported)

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call,
				NECI not set to 1"
	3	SS -> MS	IMM ASSIGNMENT	See Specific Message contents.
	4	MS -> SS	CM SERVICE REQUEST	CM Service Type = Mobile Origina-
				ting Call Establishment.
	5	SS -> MS	CIPH MODE COMMAND
	6	MS -> SS	CIPH MODE COMPLETE
	7	MS -> SS	SETUP	Last L2 frame not acknowledged by
				the SS.
	8	SS -> MS	HANDOVER COMMAND	See Specific Message Contents.
	9	MS -> SS	HANDOVER ACCESS	
	10	MS -> SS	HANDOVER ACCESS	
	11	MS -> SS	HANDOVER ACCESS	See Specific message contents. Four
	12	MS -> SS	HANDOVER ACCESS	Messages are transmitted to cell B in 4 
				successive slots on the new DCCH. If  the 
				HANDOVER COMMAND message includes a 
				starting time IE then the first HANDOVER 
				ACCESS message shall be transmitted in the 
				indicated frame ( unless the starting time has 
				elapsed ).
	13	MS -> SS	SABM	Sent without information field
	14	SS -> MS	UA
	15	MS -> SS	HANDOVER COMPLETE	The message shall be ready to be transmitted
				before 'x' ms after the completion of step 8.
	16	SS		The header of the next uplink SACCH is 
				examined and the Timing Advance and Power 
				Level indications are examined. The correct 
				timing advance shall be indicated. The power 
				level indication shall indicate the power level 
				used in the handover command.
	17	MS -> SS	SETUP	Same N(SD) as in step 7
	18	SS -> MS	CHANNEL RELEASE

Specific Message Contents

M = 1 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation				Indicates all of the CA of cell A

except for the BCCH frequency

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Zero ( this gives cyclic hopping )

Frequency List after time

- Frequency List				Use Range 512 to encode the

complete CA of Cell B.

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Signalling only

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  1500 ms

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation				Indicates all of the CA of cell A

except for the BCCH frequency

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Zero ( this gives cyclic hopping )

Frequency List after time

- Frequency List				Use Bit Map 0 to encode the

complete CA of Cell B.

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Signalling only

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  1500 ms

M = 2 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Zero ( this gives cyclic hopping ).

Mobile Allocation				Indicates all of the CA of cell A

except for the following 2 frequencies:

( 747 and 767 )

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				SDCCH/4

- TDMA offset				Chosen arbitrarily

- Timeslot number				zero

- Training Sequence Code			same as the BCCH

- Hopping				Single RF Channel

- ARFCN					764

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Starting Time				Indicates the frame number of cell B

that will occur approximately 1.1

seconds after the HANDOVER

COMMAND message is sent by cell A.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  1500 ms

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Zero ( this gives cyclic hopping ).

Mobile Allocation				Indicates all of the CA of cell A

except for the following 2 frequencies:

( 20 and 52 )

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				SDCCH/4

- TDMA offset				Chosen arbitrarily

- Timeslot number				zero

- Training Sequence Code			same as the BCCH

- Hopping				Single RF Channel

- ARFCN					40

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Starting Time				Indicates the frame number of cell B

that will occur approximately 1.1

seconds after the HANDOVER

COMMAND message is sent by cell A.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  1500 ms

M = 3 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					747

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Short list IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency Short List after time

- Frequency Short List			Use Range 128 to encode the following 3

frequencies : (758, 761, 771)

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  650 ms

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					20

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Channel Sequence IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency  Channel Sequence after time

- Frequency Channel Sequence

Indicates (66, 75, 76, 108 )

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  650 ms

M = 4 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				As default message contents

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell A.

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell B.

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Channel Mode				Full rate speech

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  650 ms

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				As default message contents

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell A.

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell B.

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Channel Mode				Full rate speech

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  650 ms

#### 26.6.5.5	Pre-synchronized handovers

##### 26.6.5.5.1	Handover / successful / active call / pre-synchronized / Timing Advance IE not included

If an MS does not implement the pre-synchronized handover procedure correctly then calls may fail.

###### 26.6.5.5.1.1	Conformance requirements

1	The MS shall correctly apply the handover procedure from TCH/F without frequency hopping to TCH/F without frequency hopping in the pre-synchronized case when a call is active.

2	When the Timing Advance information element is not included in the HANDOVER COMMAND, the MS shall access the new cell with the default timing advance of 1 bit period.

3	The MS shall be ready to transmit the HANDOVER COMPLETE message within 650 ms of the end of the HANDOVER COMMAND message.

References

Conformance requirement 1: GSM 04.08 section 3.4.4.

Conformance requirement 2: GSM 05.10, section 6.6.

Conformance requirement 3: GSM 04.13, section 5.2.6.1.

###### 26.6.5.5.1.2	Test purpose

To verify that when the MS is ordered to make a pre-synchronized handover to another cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly and correctly calculates the time to transmit.

###### 26.6.5.5.1.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B. k is arbitrarily selected.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A) using a full rate TCH in non-hopping mode.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell B) using a full rate TCH in non-hopping mode.

Test Procedure

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND for a pre-synchronised handover without the Timing Advance IE on the main DCCH. The MS shall send 4 access bursts, at the commanded power level, in 4 successive slots of the new DCCH to cell B with a Timing Advance of zero. Then the MS shall establish a signalling link using a Timing Advance of one and send a HANDOVER COMPLETE message. The MS shall be ready to transmit the HANDOVER COMPLETE message before 650 ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in GSM 04.13.

Maximum Duration of Test

5 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	See specific message contents below.
	2	MS -> SS	HANDOVER ACCESS	Handover Reference as included in the
	3	MS -> SS	HANDOVER ACCESS	HANDOVER COMMAND.
	4	MS -> SS	HANDOVER ACCESS	
	5	MS -> SS	HANDOVER ACCESS	
	6	MS -> SS	SABM	Sent without information field.
	7	SS -> MS	UA
	8	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted
				before 650 ms after the completion of step 1.
	9	SS	-	The SS checks that the timing
				advance reported in the layer 1
				header of the SACCH message
				that is sent in the first
				SACCH multiframe following the
				SABM is 1 bit period.

Specific Message Contents

HANDOVER COMMAND

As default message contents, except:

Synchronisation Indication			pre-synchronized; ROT=0; NCI=0.

##### 26.6.5.5.2	Handover / successful / call being established / pre-synchronized / timing advance IE is included / reporting of observed time difference requested.

If an MS does not implement the pre-synchronized handover procedure correctly then calls may fail.

If an MS does not report the observed time difference between cells correctly then pseudo synchronised handovers might not be possible for any MS.

###### 26.6.5.5.2.1	Conformance requirements

1	The MS shall correctly apply the handover procedure from an SDCCH/4 to a TCH/F without frequency hopping in the pre-synchronized case while a call is being established.

2	If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

3	When the Timing Advance information element is included in the HANDOVER COMMAND, the MS shall access the new cell with the timing advance included in the Timing Advance IE.

4	The MS shall be ready to transmit the HANDOVER COMPLETE message within 650 ms of the end of the HANDOVER COMMAND message.

5	When requested to do so in the HANDOVER COMMAND message, the MS shall return the Mobile Time Difference IE in the HANDOVER COMPLETE message indicating the sum of the observed time difference between the cells and the timing advance used on the old cell.

References

Conformance requirement 1: GSM 04.08, section 3.4.4.

Conformance requirement 2: GSM 04.08, section 3.1.4.3.

Conformance requirement 3: GSM 05.10, section 6.6.

Conformance requirement 4: GSM 04.13, section 5.2.6.1.

Conformance requirement 5: GSM 04.08, section 10.5.2.39.

###### 26.6.5.5.2.2	Test purpose

To test that when the MS is ordered to make a pre-synchronized handover to another cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly and correctly calculates the time to transmit. To test that the MS correctly retransmits Layer 3 MM or CC messages that were not acknowledged by Layer 2 before the Handover, after completion of the Handover. To test that the MS correctly reports on the time difference between the cells.

###### 26.6.5.5.2.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated" with TMSI allocated and camped on cell B.

Test Procedure

A Mobile Originating Call is initiated. The SS sends an IMMEDIATE ASSIGNMENT message allocating an SDCCH/4. The MS is commanded to use a timing advance of y bit periods on cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND, ordering the MS to switch to cell B. The MS shall then send 4 access bursts, at the commanded power level, in 4 successive slots of the new DCCH to cell B. Then the MS shall establish a signalling link using the correct timing advance and send a HANDOVER COMPLETE message. The MS shall be ready to transmit the HANDOVER COMPLETE message before 650 ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in GSM 04.13.

The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test

20 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMM ASSIGNMENT	to an SDCCH/4.
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS -> MS	CIPH MODE COMMAND
	6	MS -> SS	CIPH MODE COMPLETE
	7	MS -> SS	SETUP	Last L2 frame not acknowledged by the SS.
	8	SS -> MS	HANDOVER COMMAND	See specific message contents below.
	9	MS -> SS	HANDOVER ACCESS	Handover Reference as included in the
	10	MS -> SS	HANDOVER ACCESS	HANDOVER COMMAND
	11	MS -> SS	HANDOVER ACCESS	
	12	MS -> SS	HANDOVER ACCESS	
	13	MS -> SS	SABM	Sent without information field.
	14	SS -> MS	UA
	15	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted
				before 650 ms after the completion of step 8.
				Shall include the Mobile Time Difference IE
				with value (2k+y) mod 127 500 half bit
				periods. A tolerance of ±2 half bit periods is
				allowed.
	16	MS -> SS	SETUP	Same N(SD) as in step 7
	17	SS	-	The SS checks that the timing
				advance reported in the layer 1
				header of the SACCH message
				that is sent in the first
				SACCH multiframe following the
				SABM is 9 bit periods.
	18	SS -> MS	CHANNEL RELEASE

Specific Message Contents

HANDOVER COMMAND

As default message contents, except:

Synchronisation Indication			pre-synchronized; ROT=1; NCI=0.

Timing Advance				9 bit periods.

#### 26.6.5.6	Handover / successful / active call / pseudo synchronized

This test only applies to MSs that claim to support the pseudo synchronized handover procedure. If MSs that claim to support this procedure do not correctly implement it, then calls may fail.

##### 26.6.5.6.1	Conformance requirements

1	The MS shall correctly apply the handover procedure from TCH/F without frequency hopping to TCH/F without frequency hopping in the pseudo synchronized case when a call is in progress.

2	The MS shall access the new cell with the correct timing advance.

3	The MS shall be ready to transmit the HANDOVER COMPLETE message within 650 ms of the end of the HANDOVER COMMAND message.

4	When requested to do so in the HANDOVER COMMAND message, the MS shall return the Mobile Time Difference IE in the HANDOVER COMPLETE message indicating the sum of the observed time difference between the cells and the timing advance used on the old cell.

References

Conformance requirement 1: GSM 04.08 section 3.4.4.

Conformance requirement 2: GSM 05.10, section 6.6.

Conformance requirement 3: GSM 04.13, section 5.2.6.1.

Conformance requirement 4: GSM 04.08, section 10.5.2.39.

##### 26.6.5.6.2	Test purpose

To test that when the MS is ordered to make a pseudo synchronized handover to another cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly and correctly calculates the time to transmit. To test that the MS correctly reports the time difference between the cells.

##### 26.6.5.6.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B. k is arbitrarily selected.

The MS is being commanded to use a timing advance of y bit periods on cell A, where y is arbitrarily selected from the set {11, 12, ... ,62}.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A) using a full rate TCH in non-hopping mode.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Support for pseudo synchronisation.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell B) using a full rate TCH in non-hopping mode.

Test Procedure

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND for a pseudo-synchronised handover with the Real Time Difference IE included. The Time Difference value is set to (2k+10) modulo 256. The MS shall send 4 access bursts, at the commanded power level, in 4 successive slots of the new DCCH to cell B with a Timing Advance of zero. Then the MS shall establish a signalling link using a Timing Advance of (y-10) bit periods and send a HANDOVER COMPLETE message. The MS shall be ready to transmit the HANDOVER COMPLETE message before 650 ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in GSM 04.13.

Maximum Duration of Test

5 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	See specific message contents below.
	2	MS -> SS	HANDOVER ACCESS	Handover Reference as included in the
	3	MS -> SS	HANDOVER ACCESS	HANDOVER COMMAND.
	4	MS -> SS	HANDOVER ACCESS	
	5	MS -> SS	HANDOVER ACCESS	
	6	MS -> SS	SABM	Sent without information field
	7	SS -> MS	UA
	8	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted
				before 650 ms after the completion of step 1.
				Shall include the Mobile Time Difference IE with
				value (2k+y) mod 127 500 half bit periods. A
				tolerance of ±2 half bit periods is allowed.
	9	SS	-	The SS checks that the timing
				advance reported in the layer 1
				header of the SACCH message
				that is sent in the first
				SACCH multiframe following the
				SABM is (y-10) bit periods.

Specific Message Contents

HANDOVER COMMAND

As default message contents, except:

Synchronisation Indication			pseudo-synchronized; ROT=1; NCI=0.

Time Difference				(2k+10) mod 256.

#### 26.6.5.7	Handover / successful / active call / non-synchronized / reporting of observed time difference requested.

If an MS does not report the observed time difference between cells correctly then pseudo synchronised handovers might not be possible for any MS.

##### 26.6.5.7.1	Conformance requirements

1	The MS shall correctly apply the handover procedure from a TCH/F without frequency hopping to a TCH/F without frequency hopping in the non-synchronized case while a call is active.

2	When requested to do so in the HANDOVER COMMAND message, the MS shall return the Mobile Time Difference IE in the HANDOVER COMPLETE message indicating the sum of the observed time difference between the cells and the timing advance used on the old cell.

References

Conformance requirement 1: GSM 04.08, section 3.4.4.

Conformance requirement 2: GSM 04.08, section 10.5.2.39.

##### 26.6.5.7.2	Test purpose

To verify that when the MS is ordered to make a non-synchronized handover to another cell and is ordered to report on the time difference between the cells, that it does so correctly.

##### 26.6.5.7.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B with default parameters except the LAI of cell B has MNC = 02 decimal, MCC = 315 decimal, and LAC = 5344 H.

The BCCH of cell A is sent k bit periods before the BCCH of cell B.

The MS is commanded to use a timing advance of y bit periods on cell A.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A) using a full rate TCH in non-hopping mode.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell B) using a full rate TCH in non-hopping mode.

Test Procedure

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The HANDOVER COMMAND includes a Synchronisation Indication IE that instructs the MS to supply the observed time difference between the cells. The MS shall begin to send access bursts on the new DCCH to cell B and the SS sends one PHYSICAL INFORMATION message. The MS shall activate the channel in sending and receiving mode and establish a signalling link using the correct timing advance. The MS shall transmit a HANDOVER COMPLETE message containing the Mobile Time Difference IE with a correct value.

Maximum Duration of Test

5 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	See specific message contents below.
	2	MS -> SS	HANDOVER ACCESS	Repeated on every burst of the uplink main
				DCCH until reception of PHYSICAL
				INFORMATION. Handover Reference as
				included in the HANDOVER COMMAND.
	3	SS -> MS	PHYSICAL INFORMATION	
	4	MS -> SS	SABM	Sent without information field
	5	SS -> MS	UA
	6	MS -> SS	HANDOVER COMPLETE	Shall include the Mobile Time Difference IE
				with value (2k+y) mod 127 500 half bit
				periods. A tolerance of ±2 half bit periods is
				allowed.

Specific Message Contents

HANDOVER COMMAND

as default message contents, except:

Synchronisation Indication			'not synchronised'; ROT=1; NCI=0.

#### 26.6.5.8	Handover / layer 3 failure

##### 26.6.5.8.1	Conformance requirements

The MS shall return to the old channel in the case of an handover failure caused by the non reception of the PHYSICAL INFORMATION message. On the old channel the MS shall use the Power Level that it was previously using on that channel.

References

GSM 04.08 section 3.4.4.

##### 26.6.5.8.2	Test purpose

To verify the function of timer T3124 and the contents in the message HANDOVER FAILURE and in the layer 1 header on the SACCH.

##### 26.6.5.8.3	Method of test

Initial Conditions

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). Used power level is the maximum supported by the MS.

[editorial note : since not all MSs support such high power levels, it might be better to choose e

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell A). Used power level is the maximum supported by the MS.

Test Procedure

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND with Power Command set to 8 on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B. The SS activates the SACCH, but does not send PHYSICAL INFORMATION (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

0	SS	-	The SS records the power level
				in the layer 1 header of the
				last SACCH message sent by the
				MS before step 1.
	1	SS -> MS	HANDOVER COMMAND	Channel description : non-hopping,
				full rate
				Power Command : 8. Synchronisation
				Indication : non synchronized.
	2	MS -> SS	HANDOVER ACCESS	Several messages are sent, all
				with correct Handover References.
	3	MS -> SS	HANDOVER FAILURE	Sent on old channel,
				RR cause value ="Abnormal release,
				unspecified", "Abnormal release,
				channel unacceptable", "Abnormal
				release, timer expired" or
				"Abnormal release, no activity on
				the radio path".
				Shall be sent
				within 3 seconds from the trans-
				mission of HANDOVER COMMAND.
	4	SS	-	The SS checks that the power
				level reported in the layer 1
				header of the SACCH message
				that is sent in the first
				SACCH multiframe following the
				SABM is the same as in step 0.

Specific Message Contents

None

#### 26.6.5.9	Handover / layer 1 failure

##### 26.6.5.9.1	Conformance requirements

The MS shalll return to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell. On the old channel the MS shall use the Power Level that it was previously using on that channel.

References

GSM 04.08 section 3.4.4.

##### 26.6.5.9.2	Test purpose

To verify the function of timer T3124 and the contents in the message HANDOVER FAILURE and in the layer 1 header on the SACCH.

##### 26.6.5.9.3	Method of test

Initial Conditions

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). Used power level is the maximum supported by the MS.

[editorial note : since not all MSs support such high power levels, it might be better to choose e

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell A). Used power level is the maximum supported by the MS.

Test Procedure

The MS is in the active state (u10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B. With the exception of normal BCCH signalling, the SS does not transmit anything on cell B (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

0	SS	-	The SS records the power level
				in the layer 1 header of the
				last SACCH message sent by the
				MS before step 1.
	  1	SS -> MS	HANDOVER COMMAND	Channel description : non-hopping,
				full rate. Synchronisation
				Indication : non synchronized.
	  2	MS -> SS	HANDOVER ACCESS	Several messages are sent, all
				with correct Handover References.
	  3	MS -> SS	HANDOVER FAILURE	Sent on old channel,
				RR cause value ="Abnormal release,
				unspecified", "Abnormal release,
				channel unacceptable", "Abnormal
				release, timer expired" or
				"Abnormal release, no activity on
				the radio path".
				Shall be sent
				within 3 seconds from the trans-
				mission of HANDOVER COMMAND.
	4	SS	-	The SS checks that the power
				level reported in the layer 1
				header of the SACCH message
				that is sent in the first
				SACCH multiframe following the
				SABM is the same as in step 0.

Specific Message Contents

None

### 26.6.6	Test of frequency redefinition

The Frequency Redefinition procedure is used by the network to change the frequencies and hopping sequences of the allocated channels.

#### 26.6.6.1	Frequency redefinition

##### 26.6.6.1.1	Conformance requirements

An MS, after receiving a FREQUENCY REDEFINITION message, shall start using the new frequencies and hopping sequence in the correct time slot when the MS is allocated a dedicated channel.

The behaviour described in the test purpose is applied for each combination of the value T(k) (k = 1,2,3) and for each supported dedicated channel type.

References

GSM 04.08 sections 3.4.5, 9.1.13 and 10.5.2.13.

##### 26.6.6.1.2	Test purpose

To verify that the MS, after receiving a Frequency Redefinition message, starts using the new frequencies and hopping sequence at the time indicated in the message.

##### 26.6.6.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs. The cell allocation is set to CAPGSM(1) or CADCS(1), depending on the band of operation of the Mobile Station ( See PICS/PIXIT), before each execution of this test.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F and/or TCH/H.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

Test parameters:

P-GSM900 :

caPGSM(1) is set to 64.

A random subset CAGSM(1) of the set {1,...,124} containing caGSM(1) elements is drawn.

An element B of the set CAGSM(1) is randomly drawn.

A random value caPGSM(2) in the range 20,...,63 is drawn.

A random subset CAGSM(2) of the set {1,...,124} with caGSM(2) elements and containing B is drawn.

A random value caGSM(3) in the range 4,...,19 is drawn.

A random subset CAGSM(3) of the set {1,...,124} with caGSM(3) elements and containing B is drawn.

For j = 1,2,3, values maGSM(j) in the range j,...,caGSM(j)-1 and values MAIOGSM(j) in the range 0,...,maGSM(j)-1 are randomly drawn.

Subsets MAGSM(j) of CAGSM(j) not containing B and having ma(j) elements are randomly drawn.

DCS1800 :

caDCS(1) is set to 64.

A random subset CADCS(1) of the set {700,...,812} containing caDCS(1) elements is drawn.

An element B of the set CADCS(1) is randomly drawn. CADCS(1) is then coded using the Variable Bit Map coding scheme.

A random value caDCS(2) in the range 17,...,63 is drawn.

A random subset CADCS(2) of the set {700,...,812} with caDCS(2) elements and containing B is drawn. CADCS(2) is then coded using the Variable Bit Map coding scheme.

A random value caDCS(3) in the range 4,...,16 is drawn.

A random subset CADCS(3) of the set {700,...,812} with caDCS(3) elements and containing B is drawn. CADCS(3) is then coded according to the specific message contents.

For j = 1,2,3, values maDCS(j) in the range j,...,caDCS(j)-1 and values MAIODCS(j) in the range 0,...,maDCS(j)-1 are randomly drawn.

Subsets MADCS(j) of CADCS(j) not containing B and having maDCS(j) elements are randomly drawn.

DCS1800 and GSM900

Let T(1) = 91, T(2) = 42000.

A random value T(3) in the range 92,...,29999 is drawn.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a dedicated channel (TCH/F, TCH/H or SDCCH, as defined by the execution counter R). Then the SS sends a FREQUENCY REDEFINITION message, which modifies the frequencies/hopping sequence to be used by the MS. The MS shall then, at the TDMA frame defined by the contents of the "Starting Time" information element , use the new frequencies/hopping sequence. (The value of T(2) ensures that the MS believes the Starting Time has passed and so the MS shall start transmitting immediately. The range for T(3) ensures that the MS has to wait until the designated frame before starting transmission on the new frequencies.)

The verification is performed at the RF burst level. The MS transmits the standard test signal C1 (Annex 5), and for the TCH case, the SS checks the received pattern with the expected pattern. For the SDCCH case the MS transmits fill frames, and the SS checks for each burst whether the burst is transmitted at the right frequency.

[editorial note : how is the MS made to transmit the C1 signal ? By means of loop-back ? ]

Maximum Duration of Test

3 * (number of supported channels * T(3) + 7)

Expected Sequence

This sequence is performed for every combination of execution counters K = 1,2,3 and R = 1,2,3:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	  2	MS -> SS	CHANNEL REQUEST	
	  3	SS -> MS	IMM ASSIGNMENT	
	  4	MS -> SS	PAGING RESPONSE
	  5	--------	--------	The SS checks that the MS is
				transmitting on the correct
				frequencies without delay.
	  6	SS -> MS	FREQUENCY REDEF	See description 1 below.
	  7	--------	--------	The SS checks that the MS is
				transmitting on the correct
				frequencies and that the transmissions
				started in the correct frame.
	  8	SS -> MS	FREQUENCY REDEF	See description 2 below
	  9	--------	--------	The SS checks that the MS is
				transmitting on the correct
				frequencies and that the transmissions
				started in the correct frame.
	 10	SS -> MS	CHANNEL RELEASE

For	K=1, R = 1,2,3		T(K) = T(1);

K=2, R = 1,2,3		T(K) = T(2);

K=3, R = 1,2,3		T(K) = T(3).

Specific Message Contents

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

L2 pseudo length					value dependent on the length of the

Mobile Allocation and thus on the number

of channels in CAPGSM(1).

Channel Description

Channel type				SDCCH/8 arbitrary offset, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs arbitrary offset, for R=3

Timeslot number				arbitrarily selected by

TSC						arbitrarily selected

Hopping channel				RF hopping channel

MAIO						MAIOPGSM(1)

HSN						0

Request reference					corresponds to the Channel Request

Timing advance					30 bit periods

Mobile Allocation					corresponds to set MAPGSM(1)

Starting Time					not present

IA rest octets					all bits are set to spare

FREQUENCY REDEFINITION (Description 1)

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIOPGSM(2)

HSN						0

Mobile Allocation					corresponds to set MAPGSM(2)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(K)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					corresponds to set CAPGSM(2) with 'Format ID'

set to "bit map 0"

FREQUENCY REDEFINITION (Description 2)

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs, for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIOPGSM(3)

HSN						0

Mobile Allocation					corresponds to set MAPGSM(3)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(K)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					corresponds to set CAPGSM(3) with 'Format ID'

set to "bit map 0"

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

L2 pseudo length					value dependent on the length of the

Mobile Allocation and thus on the number

of channels in CADCS(1).

Channel Description

Channel type				SDCCH/8 arbitrary offset, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs arbitrary offset, for R=3

Timeslot number				arbitrarily selected

TSC						arbitrarily selected

Hopping channel				RF hopping channel

MAIO						MAIODCS(1)

HSN						0

Request reference					corresponds to the Channel Request

Timing advance					30 bit periods

Mobile Allocation					corresponds to set MADCS(1)

Starting Time					not present

IA rest octets					all bits are set to spare

FREQUENCY REDEFINITION (Description 1)

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIODCS(2)

HSN						0

Mobile Allocation					corresponds to set MADCS(2)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(K)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					K = 1 - corresponds to set CADCS(2) with

'Format ID' set to " Variable Bit Map"

K = 2- corresponds to set CADCS(2) with

'Format ID' set to " Variable Bit Map "

K = 3 - corresponds to set CADCS(2) with

'Format ID' set to " Variable Bit Map "

FREQUENCY REDEFINITION (Description 2)

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs, for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIODCS(3)

HSN						0

Mobile Allocation					corresponds to set MADCS(3)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(K)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					K = 1 - corresponds to set CADCS(3) with

'Format ID' set to "Range 1024"

K = 2- corresponds to set CADCS(3) with

'Format ID' set to "Range 256"

K = 3 - corresponds to set CADCS(3) with

'Format ID' set to "Range 512"

### 26.6.7	Test of the channel mode modify procedure

The channel mode modify procedure allows the network to request the MS to change the channel mode for one channel. If the mobile station doesn't correctly respond to the CHANNEL MODE MODIFY message (with a positive acknowledgement if the new channel mode is supported, with a negative acknowledgement if the new channel mode is not supported), the network may try to repeat the procedure, release the connection, or continue to wait for the acknowledgement (the maximum time resulting from layer two re-transmissions and MS reaction time being around 5 seconds).

#### 26.6.7.1	Test of the channel mode modify procedure / full rate

This test is only applicable to an MS supporting TCH/F.

##### 26.6.7.1.1	Conformance requirement

When the MS has received the CHANNEL MODE MODIFY message, the mobile station changes the mode for the indicated channel and then replies by a CHANNEL MODE MODIFY ACKNOWLEDGE message indicating the new channel mode.

If the mobile station does not support the indicated mode, it shall retain the old mode and return the associated channel mode information in the CHANNEL MODE MODIFY ACKNOWLEDGE message.

References

TS GSM 04.08, subclauses 3.4.6.2 and 3.4.6.3

##### 26.6.7.1.2	Test purpose

To verify that the MS, in an RR connected state, acknowledges a CHANNEL MODE MODIFY message by sending a CHANNEL MODE MODIFY ACKNOWLEDGEMENT message specifying and switching to the correct mode.

-	the new mode if that mode is supported

-	the old mode if the new mode is not supported.

This shall be verified for the channel modes

-	signalling only

-	speech full rate

-	data 9.6 Kb/s

-	data 4.8 Kb/s full rate

-	data 2.4 Kb/s full rate.

##### 26.6.7.1.3	Method of test

Initial Conditions

System Simulator:

1 cells, default parameters.

Mobile Station:

The MS is "idle updated", with TMSI allocated.

Related PICS/PIXIT Statements

-	Type of MS (P-GSM900 or EGSM or DCS1800).

-	The MS supports TCH/F (Y/N)

-	Bearer Capabilities supported by the MS

-	Channel modes supported by the MS

*	MS supports speech full rate (p1 = Y/N)

*	MS supports data 9.6 Kb/s (p2 = Y/N)

*	MS supports data 4.8 Kb/s full rate (p3 = Y/N)

*	MS supports data 2.4 Kb/s full rate (p4 = Y/N)

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test procedure

A Mobile Terminated call is initiated , however following the Channel Request received from the Mobile Station, the SS sends an Immediate Assignment to the MS commanding it to go to a TCH/F. This sets the Channel Mode automatically to "Signalling Only".

The SS then sends a series of CHANNEL MODE MODIFY messages to the MS. Each time it is checked that the MS responds with a CHANNEL MODE MODIFY ACKNOWLEDGE message specifying

-	the channel mode that has been specified in the CHANNEL MODE MODIFY message, if the MS supports that mode (this mode then becomes the "channel mode in use")

-	the channel mode that was in use when the CHANNEL MODE MODIFY message has been received, if the MS does not support the channel mode specified in the CHANNEL MODE MODIFY message.

Maximum Duration of Test

30 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS->MS	PAGING REQUEST TYPE 1	Sent on correct paging subchannel
	2	MS->SS	CHANNEL REQUEST	Establishment cause indicates "answer to
				paging"
	3	SS->MS	IMMEDIATE ASSIGNMENT	Assignment to a non hopping TCH/F
	4	SS->MS	CHANNEL MODE MODIFY	
 	5	MS->SS	CHAN MODE MOD ACK	
 	6	SS->MS	CHANNEL MODE MODIFY	
 	7	MS->SS	CHAN MODE MOD ACK	
	8	SS->MS	CHANNEL MODE MODIFY	
 	9	MS->SS	CHAN MODE MOD ACK	
 	10	SS->MS	CHANNEL MODE MODIFY	
	11	MS->SS	CHAN MODE MOD ACK	
 	12	SS->MS	CHANNEL MODE MODIFY	
	13	MS->SS	CHAN MODE MOD ACK	
	14	SS->MS	CHANNEL RELEASE

Specific Message Contents

CHANNEL MODE MODIFY

Information Element		value/remark

Channel description		describes the already assigned dedicated

channel

Channel mode

Mode			in step 4:	speech full rate

in step 6:	data 9.6 Kb/s

in step 8:	data 4.8 Kb/s full rate

in step 10:	data 2.4 Kb/s full rate

in step 12	signalling only

CHANNEL MODE MODIFY ACKNOWLEDGE

Channel mode

Mode			in step 2:	signalling only

in step 4:	if p2 = Y: data 9.6 Kb/s

if p2 = N: signalling only

in step 6:	if p3 = Y: data 4.8 Kb/s full rate

if p3 = N: same as in step 4

in step 8:	if p4 = Y: data 2.4 Kb/s full rate

if p4 = N: same as in step 6

in step 10:	if p1 = Y: speech full rate

if p1 = N: same as in step 8

#### 26.6.7.2	Test of the channel mode modify procedure / half rate

This test is only applicable to an MS supporting TCH/H.

##### 26.6.7.2.1	Conformance requirement

When the MS has received the CHANNEL MODE MODIFY message, the mobile station changes the mode for the indicated channel and then replies by a CHANNEL MODE MODIFY ACKNOWLEDGE message indicating the new channel mode.

If the mobile station does not support the indicated mode, it shall retain the old mode and return the associated channel mode information in the CHANNEL MODE MODIFY ACKNOWLEDGE message.

References

TS GSM 04.08, subclauses 3.4.6.2 and 3.4.6.3

##### 26.6.7.2.2	Test purpose

To verify that the MS, in an RR connected state, acknowledges a CHANNEL MODE MODIFY message by sending a CHANNEL MODE MODIFY ACKNOWLEDGEMENT message specifying and switches to the correct mode.

-	the new mode if that mode is supported

-	the old mode if the new mode is not supported.

This shall be verified for the channel modes

-	signalling only

-	speech half rate

-	data 4.8 Kb/s half rate

-	data 2.4 Kb/s half rate

##### 26.6.7.2.3	Method of test

Initial Conditions

System Simulator:

1 cells, default parameters.

Mobile Station:

The MS is "idle updated", with TMSI allocated

Related PICS/PIXIT Statements

-	Type of MS (P-GSM900 or EGSM or DCS1800).

-	The MS supports TCH/H (Y/N)

-	Bearer Capabilities supported by the MS

-	Channel modes supported by the MS

*	MS supports speech half rate (q1 = Y/N)

*	MS supports data 4.8 Kb/s half rate (q2 = Y/N)

*	MS supports data 2.4 Kb/s half rate (q3 = Y/N)

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test procedure

A Mobile Terminated call is initiated , however following the Channel Request received from the Mobile Station, the SS sends an Immediate Assignment to the MS commanding it to go to a TCH/H. This sets the Channel Mode automatically to "Signalling Only".

The SS then sends a series of CHANNEL MODE MODIFY messages to the MS. Each time it is checked that the MS  responds with a CHANNEL MODE MODIFY ACKNOWLEDGE message specifying

-	the channel mode that has been specified in the CHANNEL MODE MODIFY message, if the MS supports that mode (this mode then becomes the "channel mode in use")

-	the channel mode that was in use when the CHANNEL MODE MODIFY message has been received, if the MS does not support the channel mode specified in the CHANNEL MODE MODIFY message.

Maximum Duration of Test

30 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS->MS	PAGING REQUEST TYPE 1	Sent on the correct paging subchannel
	2	MS->SS	CHANNEL REQUEST	Establishment cause indicates "answer to
				paging"
	3	SS->MS	IMMEDIATE ASSIGNMENT	Assignment to a non hopping TCH/H
	4	SS->MS	CHANNEL MODE MODIFY	
 	5	MS->SS	CHAN MODE MOD ACK	
 	6	SS->MS	CHANNEL MODE MODIFY	
 	7	MS->SS	CHAN MODE MOD ACK	
	8	SS->MS	CHANNEL MODE MODIFY	
 	9	MS->SS	CHAN MODE MOD ACK	
 	10	SS->MS	CHANNEL MODE MODIFY	
 	11	MS->SS	CHAN MODE MOD ACK	
 	12	SS->MS	CHANNEL RELEASE

Specific Message Contents

CHANNEL MODE MODIFY

Information Element				value/remark

Channel description				describes the already assigned

dedicated channel

Channel mode

Mode			in step 4: 	signalling only

in step 6: 	data 4.8 Kb/s half rate

in step 8: 	data 2.4 Kb/s half rate

in step 10: 	speech half rate

CHANNEL MODE MODIFY ACKNOWLEDGE

Information Element				value/remark

Channel mode

Mode			in step 5:	if q1 = Y: Speech half rate

if q1 = N: signalling only

in step 7:	if q2 = Y: data 4.8 Kb/s half rate

if q2 = N: same as in step 5

in step 9:	if q3 = Y: data 2.4 Kb/s half rate

if q3 = N: same as in step 7

in step 11:	if q1 = Y: signalling only

if q1 = N: same as in step 6.

### 26.6.8	Test of ciphering mode setting

The Ciphering Mode Setting Procedure can be used by the network to trigger the start and stop of stream ciphering.

The SS shall start and synchronize ciphering and deciphering according to GSM 03.20. The bitstream shall be generated by algorithm A5 (A5/1 or A5/2 as defined by the test case) using the encryption key Kc.

#### 26.6.8.1	Ciphering mode / start ciphering

##### 26.6.8.1.1	Conformance requirements

1.	When the MS receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS starts ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field;

-	the MS responds with a CIPHERING MODE COMPLETE message in ciphered mode;

-	the ciphering uses the cipher key determined during the authentication procedure.

2.	The MS responds to the AUTHENTICATION REQUEST message with an AUTHENTICATION RESPONSE message and continues to use the ciphering key obtained from the previous authentication procedure.

References

GSM 04.08 section 3.4.7.

##### 26.6.8.1.2	Test purpose

To verify that the MS starts ciphering when it receives a CIPHERING MODE COMMAND message with Cipher Mode Setting = "Start Ciphering". To verify that it continues to use the old cipher key after it receives an AUTHENTICATION REQUEST whilst in ciphered mode.

##### 26.6.8.1.3	Method of test

Initial Conditions

System Simulator:

1 cells, Radio-Link-Timeout set to 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Support for state U10 of the Call Control protocol.

Supported encryption algorithms: A5/1 and/or A5/2

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends an AUTHENTICATION REQUEST and the MS shall answer with AUTHENTICATION RESPONSE. Then the SS sends a CIPHERING MODE COMMAND, ordering the MS to start ciphering with an algorithm supported by the MS. After transmission of this command the SS starts deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in ciphered mode using the cipher key determined during the authentication procedure, and continue to establish the call with a SETUP message. After reception of the CIPHERING MODE COMPLETE the SS starts enciphering.

The SS then sends another AUTHENTICATION REQUEST and the MS shall respond with an AUTHENTICATION RESPONSE. The MS shall continue to use the old cipher key.

Finally the SS sends a CHANNEL RELEASE to end the test.

[editorial note : In the old 11.10 it was required that the MS should store the new key, but that was not tested. Should it be tested in phase 2 ? ]

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, K=1, 2.

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating Call"
				NECI not set to 1
	  2	SS -> MS	IMM ASSIGNMENT
	  3	MS -> SS	CM SERVICE REQUEST
	  4	SS -> MS	AUTHENT REQUEST
	  5	MS -> SS	AUTHENT RESPONSE
	  6	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "Start
				Ciphering; cipher with algorithm A5/K".
				The SS starts deciphering.
	  7	MS -> SS	CIPH MODE COMPLETE	Sent in ciphered mode using
				the cipher key determined in between
				steps 4&5. The SS start enciphering
	  8	MS -> SS	SETUP
	  9	SS -> MS	AUTHENT REQUEST	Determines a new cipher key
	 10	MS -> SS	AUTHENT RESPONSE	Sent in ciphered mode using the
				cipher key determined in between steps
				4&5.
	 11	SS -> MS	CHANNEL RELEASE

Specific Message Contents

CIPHERING MODE COMMAND

For	k = 1, A5/k = A5/1

For	k = 2, A5/k = A5/2

#### 26.6.8.2	Ciphering mode / no ciphering

##### 26.6.8.2.1	Conformance requirements

When the MS receives a CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "no ciphering" the MS shall respond in non ciphered mode with a CIPHERING MODE COMPLETE message.

When the CIPHERING MODE COMMAND with Ciphering Mode Setting information element set to "no ciphering" is received as a response to a CM SERVICE REQUEST, the MS shall continue the establishment of the CM service.

References

GSM 04.08 section 3.4.7.

##### 26.6.8.2.2	Test purpose

To verify that the MS does not start ciphering when it receives a CIPHERING MODE COMMAND message with Cipher Mode Setting = "No Ciphering".

##### 26.6.8.2.3	Method of test

Initial Conditions

System Simulator:

1 cells, Radio-Link-Timeout set to 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends an AUTHENTICATION REQUEST and the MS shall answer with an AUTHENTICATION RESPONSE. Then the SS sends a CIPHERING MODE COMMAND, ordering the MS not to start ciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in non-ciphered mode and continue to establish the call with a SETUP message.

Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call;
				NECI not equal to 1
	  2	SS -> MS	IMM ASSIGNMENT
	  3	MS -> SS	CM SERVICE REQUEST
	  4	SS -> MS	AUTHENT REQUEST
	  5	MS -> SS	AUTHENT RESPONSE
	  6	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering"
	  7	MS -> SS	CIPH MODE COMPLETE	Sent in non-ciphered mode
	  8	MS -> SS	SETUP
	 11	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

#### 26.6.8.3	Ciphering mode / old cipher key

##### 26.6.8.3.1	Conformance requirements

When the MS receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS starts ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field. Also;

-	the MS responds with a CIPHERING MODE COMPLETE message in the correct ciphered mode;

-	the ciphering shall use the previously stored cipher key

-	in the case of a mobile originating speech caal, the MS shall send a SETUP message after the completion of the ciphering procedure.

References

GSM 04.08 section 3.4.7.

##### 26.6.8.3.2	Test purpose

To verify that the MS uses the stored cipher key when it receives a CIPHERING MODE COMMAND without a preceding authentication procedure.

##### 26.6.8.3.3	Method of test

Initial Conditions

System Simulator:

1 cells, Radio-Link-Timeout = 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and a known cipher key stored.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Support for state U10 of the Call Control protocol.

Supported encyrption algorithms: A5/1 and/or A5/2.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends a CIPHERING MODE COMMAND, ordering the MS to start ciphering with a supported algorithm. After transmission of this command the SS starts deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in the commanded ciphered mode using the stored cipher key and continue to establish the call with a SETUP message. After reception of the CIPHERING MODE COMPLETE the SS starts enciphering.

Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating Call:
				NECI not equal to 1
	  2	SS -> MS	IMM ASSIGNMENT
	  3	MS -> SS	CM SERVICE REQUEST
	  4	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "Start
				Ciphering", algorithm arbitrarily selected
				from those supported by the MS. The SS starts
				deciphering.
	  5	MS -> SS	CIPH MODE COMPLETE	Sent in commanded ciphered mode with
				the stored cipher key. The SS
				starts enciphering.
	  6	MS -> SS	SETUP
	  7	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

#### 26.6.8.4	Ciphering mode / change of mode, algorithm and key

Networks can be implemented that do not have the same ciphering algorithms on all base stations. In such networks changes of algorithms and ciphering mode may occur and calls will fail if MSs incorrectly handle commands or use an incorrect cipher key.

##### 26.6.8.4.1	Conformance requirements

1	When the MS in the "not ciphered" mode, receives a CIPHERING MODE COMMAND message with the Ciphering Mode Setting information element set to "start ciphering", the MS shall load the cipher key stored in the SIM into the ME, use this key to start ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field and, respond with a CIPHERING MODE COMPLETE message.

2	If the last timeslot of the message block containing a CIPHERING MODE COMMAND message occurs at time T, then the MS shall be ready to transmit the CIPHERING MODE COMPLETE message before T+500ms.

3	When the MS receives an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the assignment, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key.

4	When the MS receives a HANDOVER COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the handover, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key

5	When the MS in the "ciphered" mode receives a CIPHERING MODE COMMAND message with Cipher Mode Setting IE set to "no ciphering", the MS shall load the cipher key stored in the SIM into the ME, load the cipher key stored in the SIM into the ME, stop ciphering and deciphering and, respond with a CIPHERING MODE COMPLETE message.

6	When the MS receives an AUTHENTICATION REQUEST message, it shall process the challenge information and send back an AUTHENTICATION RESPONSE message to the network. The new ciphering key calculated from the challenge information shall overwrite the previous one and be stored on the SIM before the AUTHENTICATION RESPONSE message is transmitted. The ciphering key stored in the SIM shall be loaded in to the ME when any valid CIPHERING MODE COMMAND is received.

7	When the MS in the not ciphered mode receives a CIPHERING MODE COMMAND message with Cipher Mode Setting IE set to "no ciphering", the MS shall load the cipher key stored in the SIM into the ME, stop ciphering and deciphering and, respond with a CIPHERING MODE COMPLETE message.

8	If a handover fails then the operational parameters used when returning to the old channel are those applied before the HANDOVER COMMAND message was received.

9	If an assignment fails then the operational parameters used when returning to the old channel are those applied before the ASSIGNMENT COMMAND message was received.

References

Conformance requirement 1: GSM 04.08 sections 3.4.7.2 and 4.3.2.2.

Conformance requirement 2: GSM 04.13 section 5.2.7.

Conformance requirement 3: GSM 04.08 section 3.4.3.1.

Conformance requirement 4: GSM 04.08 section 3.4.4.1.

Conformance requirement 5: GSM 04.08 sections 3.4.7.2 and 4.3.2.2.

Conformance requirement 6: GSM 04.08 section 4.3.2.2.

Conformance requirement 7: GSM 04.08 section 3.4.7.2.

Conformance requirement 8: GSM 04.08 section 3.4.4.4.

Conformance requirement 9: GSM 04.08 section 3.4.3.3.

##### 26.6.8.4.2	Test purpose

1	To verify that when the MS is in the "not ciphered" mode and receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS uses the cipher key stored in the SIM to start ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field and that the MS responds with a CIPHERING MODE COMPLETE message.

2	To verify that the MS is ready to transmit the CIPHERING MODE COMPLETE message before 500ms after the end of the CIPHERING MODE COMMAND message.

3	To verify that when the MS receives an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the assignment, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key.

4	To verify that when the MS receives a HANDOVER COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the handover, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key

5	To verify that when the MS is in the "ciphered" mode and receives the CIPHERING MODE COMMAND message with Cipher Mode Setting IE set to "no ciphering", the MS loads the cipher key stored in the SIM into the ME, stops ciphering and deciphering and, responds with a CIPHERING MODE COMPLETE message.

6	To verify that the MS responds to an AUTHENTICATION REQUEST message with an AUTHENTICATION RESPONSE message and continues to use the cipher key obtained from the previous authentication procedure.

7	To verify that when the MS is in the "not ciphered" mode and receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "no ciphering", the does not start ciphering or deciphering, but does respond with a CIPHERING MODE COMPLETE message.

8	To verify that when the MS receives a HANDOVER COMMAND message and the handover fails, the MS sends a HANDOVER FAILURE message on the old channel using the old ciphering mode and (if ciphered) the old algorithm and old key.

9	To verify that when the MS receives an ASSIGNMENT COMMAND message and the assignment fails, the MS sends an ASSIGNMENT FAILURE message on the old channel using the old ciphering mode and (if ciphered) the old algorithm and old key.

##### 26.6.8.4.3	Method of test

Initial Conditions

System Simulator:

1 cell with default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and a known cipher key, K, stored in the SIM.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Supported ciphering algorithms: A5/1 and/or A5/2.

Power class of mobile station

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged. The MS shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a PAGING RESPONSE message.

The SS sends a CIPHERING MODE COMMAND, ordering the MS to start ciphering with a supported algorithm. After transmission of this command the SS starts deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in the commanded ciphered mode using the cipher key K. After reception of the CIPHERING MODE COMPLETE the SS starts enciphering. The MS shall be ready to transmit the CIPHERING MODE COMPLETE message before 500 ms after the end of the CIPHERING MODE COMMAND message.

The term 'ready to transmit' is defined in GSM 04.13.

The SS sends an AUTHENTICATION REQUEST message to the MS. Cipher key L is calculated. The MS shall send an AUTHENTICATION RESPONSE message to the SS.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "no ciphering". The MS shall transmit the HANDOVER COMPLETE on the commanded channel in non ciphered mode.

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "start ciphering". The MS shall start transmitting on the commanded channel using the commanded algorithm and cipher key K. The MS shall transmit the ASSIGNMENT COMPLETE message.

The SS sends a CIPHERING MODE COMMAND, ordering the MS to stop ciphering. After transmission of this command the SS stops deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in non ciphered mode. After reception of the CIPHERING MODE COMPLETE the SS stops enciphering.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "start ciphering". The MS shall transmit the HANDOVER COMPLETE on the commanded channel in ciphered mode using cipher key L and command algorithm.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "start ciphering" and the algorithm identifier indicating the algorithm currently in use. The MS shall transmit the HANDOVER COMPLETE on the commanded channel in ciphered mode using the same algorithm as before the handover.

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "no ciphering". The MS shall start transmitting on the commanded channel in non-ciphered mode. The MS shall transmit the ASSIGNMENT COMPLETE message.

The SS sends a CIPHERING MODE COMMAND, containing a Cipher Mode Setting IE set to "no ciphering". The MS shall respond with a CIPHERING MODE COMPLETE message.

The SS sends an AUTHENTICATION REQUEST message to the MS. Cipher key M is calculated. The MS shall send an AUTHENTICATION RESPONSE message to the SS.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "start ciphering". The MS shall transmit the HANDOVER COMPLETE on the commanded channel using the commanded algorithm and cipher key L.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "no ciphering". The SS does not activate the commanded channel. The MS's transmissions on the new channel need not be monitored. The MS shall transmit the HANDOVER FAILURE message on the 'old' channel using the 'old' algorithm and cipher key L and commanded algorithm.

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "start ciphering". The SS does not activate the commanded channel. The MS's transmissions on the new channel need not be monitored. The MS shall transmit the ASSIGNMENT FAILURE message on the 'old' channel using the 'old' algorithm and cipher key L.

If the MS only supports one ciphering algorithm then the SS sends a CHANNEL RELEASE message.

If the MS supports more than one ciphering algorithm then the following steps are performed:

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "start ciphering" and the Algorithm Identifier indicating a different supported algorithm to the one in use. The MS shall start transmitting on the commanded channel using the commanded algorithm. The MS shall transmit the ASSIGNMENT COMPLETE message.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) containing a Cipher Mode Setting IE set to "start ciphering" and the Algorithm Identifier indicating a different supported algorithm to the one in use. The MS shall transmit the HANDOVER COMPLETE on the commanded channel using the commanded algorithm.

The SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test

3 minutes.

Expected Sequence

For MSs that only support one ciphering algorithm, the SS shall use step 57A. For MSs that support more than one ciphering algorithm, the SS shall use step 57B and the subsequent steps.

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMM ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "Start Ciphering",
				algorithm arbitrarily selected from those
				supported by the MS. The SS starts 
				deciphering with the selected algorithm.
	  6	MS -> SS	CIPH MODE COMPLETE	Sent in ciphered mode using key 'K', the
				stored cipher key, and the commanded 
				algorithm. This message shall be ready to
				be transmitted before 500 ms after the
				completion of step 5.
  	7	SS	-	The SS starts enciphering using key 'K'.
	  8	SS -> MS	AUTHENT REQUEST	Contains a new Ciphering Key Sequence
				Number which is associated with the new
				cipher key, 'L'.
	  9	MS -> SS	AUTHENT RESPONSE
	 10	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"No Ciphering".
 	11	SS		The SS activates the new channel without
				ciphering.
 	12	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are
 	13	MS -> SS	HANDOVER ACCESS	sent on the new channel in non ciphered
 	14	MS -> SS	HANDOVER ACCESS	mode.
 	15	MS -> SS	HANDOVER ACCESS
 	16	MS -> SS	HANDOVER COMPLETE	Sent in non ciphered mode on the new
				channel.
 	17	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering", with algorithm arbitrarily
				selected from those supported by the MS.
 	18	SS		The SS activates the new channel with
				enciphering and deciphering enabled and using
				cipher key 'K'.
 	19	MS -> SS	ASSIGNMENT COMPLETE	Sent on the new channel in ciphered mode
				using key 'K' and the commanded algorithm.
	 20	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering". The
				SS starts receiving in non ciphered mode.
 	21	MS -> SS	CIPH MODE COMPLETE	Sent in non ciphered mode. This message shall
				be ready to be transmitted before 500 ms after
				the completion of step 20.
 	22	SS	-	The SS starts transmitting in non ciphered
				mode.
 	23	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering", with algorithm 'X' arbitrarily
				selected from those supported by the MS.
 	24	SS		The SS activates the new channel with
				enciphering and deciphering enabled and using
				cipher key 'L'.
 	25	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are
 	26	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered
 	27	MS -> SS	HANDOVER ACCESS	mode.
 	28	MS -> SS	HANDOVER ACCESS
 	29	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode
				using key 'L' and algorithm 'X'.
 	30	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering", with algorithm identifier
				set to 'X'.

31	SS		The SS activates the new channel with
				enciphering and deciphering enabled.
 	32	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are
 	33	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered
 	34	MS -> SS	HANDOVER ACCESS	mode.
 	35	MS -> SS	HANDOVER ACCESS
 	36	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode
				using algorithm 'X'.
 	37	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to
				"No Ciphering".
 	38	SS		The SS activates the new channel without
				ciphering.
 	39	MS -> SS	ASSIGNMENT COMPLETE	Sent in non-ciphered mode on the new
				channel.
	 40	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering". 
 	41	MS -> SS	CIPH MODE COMPLETE	Sent in non ciphered mode. This message shall
				be ready to be transmitted before 500 ms after
				the completion of step 40.
	 42	SS -> MS	AUTHENT REQUEST	Contains a new Ciphering Key Sequence
				Number which is associated with the new
				cipher key, 'M'.
	 43	MS -> SS	AUTHENT RESPONSE
 	44	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering", with algorithm 'Y' arbitrarily
				selected from those supported by the MS.
 	45	SS		The SS activates the new channel with
				enciphering and deciphering enabled and using
				cipher key 'L'.
 	46	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are
 	47	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered
 	48	MS -> SS	HANDOVER ACCESS	mode.
 	49	MS -> SS	HANDOVER ACCESS
 	50	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode
				using key 'L'
 	51	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"No Ciphering".
 	52	SS, MS		The SS does not activate the new channel
				The MS's transmissions on the new channel
				need not be monitored.
 	53	MS -> SS	HANDOVER FAILURE	sent on old channel using algorithm 'Y' and
				key 'L'.
 	54	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to
				"No Ciphering".
 	55	SS, MS		The SS does not activate the new channel
				The MS's transmissions on the new channel
				need not be monitored.
 	56	MS -> SS	ASSIGNMENT FAILURE	sent on old channel using algorithm 'Y' and
				key 'L'.
	57A	SS -> MS	CHANNEL RELEASE
	57B	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering", with algorithm 'Z' arbitrarily
				selected from those supported by the MS but 
				different to algorithm 'Y'.
	58B	SS		The SS activates the new channel with
				enciphering and deciphering enabled.
	59B	MS -> SS	ASSIGNMENT COMPLETE	Sent on the new channel in ciphered mode
				using key 'L' and algorithm 'Z'.
	60B	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering" and the algorithm identifier
				set to 'Y'.

61B	SS		The SS activates the new channel with
				enciphering and deciphering enabled.
	62B	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are
	63B	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered
	64B	MS -> SS	HANDOVER ACCESS	mode.
	65B	MS -> SS	HANDOVER ACCESS
	66B	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode
				using key 'L' and algorithm 'Y'.
	67B	SS -> MS	CHANNEL RELEASE

Specific Message Contents

ASSIGNMENT COMMAND

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101110

Channel Description

-	Channel Type				SDCCH/4

-		TDMA offset			Chosen arbitrarily, but different to the one in use.

-	Timeslot Number			Timeslot zero

-	Training Sequence Code			5 (same as the BCC)

-	Hopping				Single RF channel

-	ARFCN				Channel number 30

Power Command

-	Power level				Chosen arbitrarily but

within the range supported by the MS.

Cipher Mode Setting				As specified above.

All other information elements:			Not present

HANDOVER COMMAND

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Cell Description

-	Network Colour Code			1

-	Base station Colour Code		5

-	BCCH Carrier Number			Channel Number 30

Channel Description

-	Channel Type				SDCCH/4

-		TDMA offset			Chosen arbitrarily, but different to the one in use.

-	Timeslot Number			Timeslot zero

-	Training Sequence Code			5 (same as the BCC)

-	Hopping				Single RF channel

-	ARFCN				Channel Number 30

Handover Reference

-	Handover Reference Value		Chosen arbitrarily .

Power Command

-	Power level				Chosen arbitrarily, but

within the range supported by the MS

Synchronisation Indication

-	Synchronisation indication		synchronized

-	Report observed time difference		not included

-	Normal cell indication			out of range ignored

Cipher Mode setting				As specified above.

All other information elements:			Not present

CIPHER MODE COMMAND

Information Element				value/remark

As default message contents, except :

Cipher Mode Setting

- Algorithm Identifier				As specified above

- Cipher Mode Set				As specified above

DCS1800 :

ASSIGNMENT COMMAND

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101110

Channel

-	Channel Type				SDCCH/4

-		TDMA offset			Chosen arbitrarily, but different to the one in

use.

-	Timeslot Number			Timeslot zero

-	Training Sequence Code			5 (same as the BCC)

-	Hopping				Single RF channel

-	ARFCN				Channel number 650

Power Command

-	Power level				Chosen arbitrarily, but within the range

supported by the MS.

Cipher Mode Setting				As specified above.

All other information elements:			Not present

HANDOVER COMMAND

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Cell Description

-	Network Colour Code			1

-	Base station Colour Code		5

-	BCCH Carrier Number			Channel Number 650

Channel Description

-	Channel Type				SDCCH/4

-		TDMA offset				Chosen arbitrarily, but different to the one in

use.

-	Timeslot Number			Timeslot zero

-	Training Sequence Code			5 (same as the BCC)

-	Hopping				Single RF channel

-	ARFCN				Channel Number 650

Handover Reference

-	Handover Reference Value		Chosen arbitrarily.

Power Command

-	Power level				Chosen arbitrarily, but within the range

supported by the MS.

Synchronisation Indication

-	Synchronisation indication		synchronized

-	Report observed time difference		not included

-	Normal cell indication			out of range ignored

Cipher Mode setting				As specified above.

All other information elements:			Not present

#### 26.6.8.5	Ciphering mode / IMEISV request

If the MS does not supply the IMEISV when requested, the network will not know whether or not the MS is type approved, ie. whether or not it has passed any tests.

If the MS supplies its IMEISV when not requested, this may cause calls to systematically fail.

##### 26.6.8.5.1	Conformance requirements

1.	When the MS receives the CIPHERING MODE COMMAND message with Cipher Response bit set to 'IMEISV shall be included', the MS shall include the IMEISV in the Mobile Identity IE in the CIPHERING MODE COMPLETE message.

2.	When the MS receives the CIPHERING MODE COMMAND message with Cipher Response bit set to 'IMEISV shall not be included', the MS shall not include the Mobile Identity IE in the CIPHERING MODE COMPLETE message.

References

Conformance requirement 1: GSM 04.08 sections 3.4.7.2 and 9.1.10.1.

Conformance requirement 2: GSM 04.08 section 9.1.10.1.

##### 26.6.8.5.2	Test purpose

To verify that the MS supplies its IMEISV in the CIPHERING MODE COMPLETE message when it receives a CIPHERING MODE COMMAND message with a Cipher Response bit set to 'IMEISV shall be included'.

To verify that the MS does not supply any Mobile Identity IE in the CIPHERING MODE COMPLETE message when it receives a CIPHERING MODE COMMAND message with a Cipher Response bit set to 'IMEISV shall not be included'.

##### 26.6.8.5.3	Method of test

Initial Conditions

System Simulator:

1 cell.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

IMEISV of the MS

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a PAGING RESPONSE. Then the SS sends a CIPHERING MODE COMMAND indicating 'No ciphering' and with the Cipher Response bit set to 'IMEISV shall not be included'. The MS shall respond with a CIPHERING MODE COMPLETE message that does not include the Mobile Identity IE.

Then the SS sends a CIPHERING MODE COMMAND indicating 'No ciphering' and with the Cipher Response bit set to 'IMEISV shall be included'. The MS shall respond with a CIPHERING MODE COMPLETE message that carries the IMEISV in the Mobile Identity IE.

Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test

10 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering".
				Cipher Response = "IMEISV shall not be
				included".
	6	MS -> SS	CIPH MODE COMPLETE	Shall not include Mobile Identity IE.
	7	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering".
				Cipher Response = "IMEISV shall be included".
	8	MS -> SS	CIPH MODE COMPLETE	Shall include one Mobile Identity IE carrying
				the MS's IMEISV.
	9	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None.

### 26.6.9	Test of additional assignment

The Additional Assignment procedure is used to change an MS's channel configuration from Lm + ACCH to Lm + Lm + ACCH. It is therefore only relevant to those mobiles which perform such an operation.

No tests are specified at the moment.

### 26.6.10	Test of partial release

The Partial Release procedure is used to change an MS's channel configuration from Lm + Lm + ACCH to Lm + ACCH. It is therefore only relevant to those mobiles which perform such an operation.

No tests are specified at the moment.

### 26.6.11	Test of classmark

#### 26.6.11.1	Classmark change

This procedure allows the MS to indicate to the network that a change in the classmark (e.g. due to addition of power amplification) has taken place.

##### 26.6.11.1.1	Conformance requirements

If the RF power capability of the MS is changed during a call, this change shall be signalled to the network.

If the RF power capability of the MS is changed in idle mode, the up to date RF power capability shall be signalled to the network during RR connection establishment.

References

GSM 04.08 section 3.4.10.

##### 26.6.11.1.2	Test purpose

To verify that if the RF power capability or any other capability indicated in a Classmark IE of the MS is changed during a call, the change is communicated on the DCCH to the network.

To verify that if the RF power capability or any other capability indicated in a Classmark IE of the MS is changed in idle mode, the out of date capabilities are not communicated to the network during RR connection establishment.

##### 26.6.11.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and with no additional power amplification applied.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Support for more than one RF power class: yes/no.

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated and with no additional power amplification applied.

Test Procedure

With the MS in idle mode, the RF power capability shall be changed by the addition of power amplification, after which the MS is made to originate a call. The new RF power capability shall be included in the CM SERVICE REQUEST message. After the call has reached the Call Control state U10, the RF power capability of the MS is changed by removal of the additional power amplification. The MS shall send a CLASSMARK CHANGE message indicating the new RF power capability. The RF power capability is then changed by adding the power amplification. The MS shall again send a CLASSMARK CHANGE message indicating the new RF power capability. The call is then released by the SS.

With the MS in idle mode, the power amplification is removed. The SS then pages the MS, which in the PAGING RESPONSE message shall indicate the correct RF power capability.

Finally the SS transmits a CHANNEL RELEASE to end the test.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

1	--------	--------	Add power amplification
	2	--------	--------	The MS shall be made to
				originate a call
	3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call"
				NECI not equal to one
	4	SS -> MS	IMM ASSIGNMENT
	5	MS -> SS	CM SERVICE REQUEST	The "Mobile Station Classmark 2"
				IE shall indicate the new RF power
				capability
	6	SS -> MS	CM SERVICE ACCEPT
	7	MS -> SS	SETUP
	8	SS -> MS	CALL PROCEEDING
	9	SS -> MS	ALERT
	10	SS -> MS	ASSIGNMENT COMMAND
	11	MS -> SS	ASSIGNMENT COMPLETE
	12	SS -> MS	CONNECT
	13	MS -> SS	CONNECT ACKNOWLEDGE
	14	--------	--------	Remove the power amplification
	15	MS -> SS	CLASSMARK CHANGE	The "Mobile Station Classmark 2"
				IE shall indicate the new power
				capability
	16	--------	--------	Add power amplification
	17	MS -> SS	CLASSMARK CHANGE	The "Mobile Station Classmark 2"
				IE shall indicate the new power
				capability
	18	SS -> MS	CHANNEL RELEASE
	19	--------	--------	Remove the power amplification
		--------	--------	The SS waits 12 seconds to allow the
				MS to perform cell reselection
	20	SS -> MS	PAG REQ TYPE 1
	21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	22	SS -> MS	IMM ASSIGNMENT
	23	MS -> SS	PAGING RESPONSE	The "Mobile Station Classmark 2"
				IE shall indicate the new power
				capability
	24	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

#### 26.6.11.2	Classmark interrogation

This procedure allows the network to request the MS to supply all its classmark information to the network.

Networks may systematically use this procedure (eg. during location updating) and, it if is incorrectly implemented in the MS, the basic connection establishment procedure may systematically fail.

##### 26.6.11.2.1	Conformance requirements

On receipt of a CLASSMARK ENQUIRY message, the MS sends a CLASSMARK CHANGE message to the network containing the Mobile Station Classmark 2 information element and depending upon the contents of this information element, possibly the Mobile Station Classmark 3 information element.

References

GSM 04.08 sections 3.4.11 and 9.1.11.1.

GSM 04.13 [version 1.2.0, section 5.2.9]

##### 26.6.11.2.2	Test purpose

To verify that if the network requests the MS to supply all its classmark information then this information is communicated on the DCCH to the network.

##### 26.6.11.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

"Idle, updated", with TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Contents of Mobile Station Classmark 2 information element.

Existence of Mobile Station Classmark 3 information element: yes/no.

Contents of Mobile Station Classmark 3 information element.

Switch off button: yes/no.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is switched off (or has its power removed).

The SS then sets the IMSI attach-detach flag in the SYSTEM INFORMATION messages so that the MS shall perform a location update when switched on.

The MS is switched on (or its power is re-applied). The MS then initiates a location update attempt. After the mobile has sent the LOCATION UPDATING REQUEST message, the SS transmits a CLASSMARK ENQUIRY message. The MS shall be ready to transmit the CLASSMARK CHANGE message before 300 ms after the end of the CLASSMARK ENQUIRY message.

The term 'ready to transmit' is defined in GSM 04.13.

Then the SS transmits a LOCATION UPDATING ACCEPT message that does not contain a Mobile Identity IE.

Maximum Duration of Test

2 minutes.

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is switched off (or has its power
				removed).
  	2	SS		IMSI attach-detach flag changed.
  	3	MS		The MS is switched on (or its power
				is re-applied).
	  4	MS -> SS	CHANNEL REQUEST
	  5	SS -> MS	IMMEDIATE ASSIGNMENT
	  6	MS -> SS	LOCATION UPDATING REQ	
	  7	SS -> MS	CLASSMARK ENQUIRY
	  8	MS -> SS	CLASSMARK CHANGE	Contents as defined for default message.
				This message shall be ready to be transmitted
				before 300 ms after the completion of step 7.
	  9	SS -> MS	LOCATION UPDATING ACC
	 10	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

Default Message Contents

Contents of LOCATION UPDATING ACCEPT message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					00000010

Location Area Identification

-	Mobile Country Code			001 decimal

-	Mobile Network Code			01 decimal

-	Location Area Code			0001H

Mobile Identity					Not present

Follow on proceed					Not present

### 26.6.12	Test of channel release

The purpose of this procedure is to deactivate the dedicated channels in use. When the channels are released, the MS returns to the CCCH configuration, idle mode.

#### 26.6.12.1	Channel release / SDCCH

##### 26.6.12.1.1	Conformance requirements

After the acknowledgement of the Layer 2 disconnection by the network, the MS shall not produce any further RF-transmission.

References

GSM 04.08 section 3.5.1.

##### 26.6.12.1.2	Test purpose

To verify that the MS is able to correctly release an SDCCH after having received a CHANNEL RELEASE message.

##### 26.6.12.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall initiate a Layer 2 disconnection process on the main signalling link. After the acknowledgement of the Layer 2 disconnection by the SS, the MS shall stop transmission of Layer 2 messages. This is verified for 3 seconds. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test

20 seconds

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.
	6	MS -> SS	DISC
	7	SS -> MS	UA
		--------	--------	The SS verifies for 3 seconds that the
				MS does not produce any Layer 2 messages
		--------	--------	The SS waits 12 seconds to allow the
				MS to perform cell reselection
	8	SS -> MS	PAG REQ TYPE 1
	9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	10	SS -> MS	IMM ASSIGNMENT	
	11	MS -> SS	PAGING RESPONSE
	12	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

#### 26.6.12.2	Channel release / SDCCH - no L2 ACK

##### 26.6.12.2.1	Conformance requirements

After the expiry of timer T3110 the MS shall not produce any further RF-transmission.

References

GSM 04.08 section 3.5.1.

##### 26.6.12.2.2	Test purpose

To verify that the MS is able to correctly release a SDCCH after having received a CHANNEL RELEASE message, even if the SS does not L2 acknowledge the L2 DISC frame.

##### 26.6.12.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall send at least 2 L2 DISC frames. The SS does not acknowledge any of the L2 DISC frames. After 2 seconds, the SS verifies for 3 seconds that the MS has stopped transmission of Layer 2 messages. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test

25 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.
	6	MS - > SS	DISC	The MS shall send at least 2 L2
				DISC frames, to which the SS does
				not respond. After a period of 2
				seconds, the SS verifies for 3
				seconds that the MS does not
				produce any further Layer 2 messages
		--------	--------	The SS waits 12 seconds to allow the
				MS to perform cell reselection
	7	SS -> MS	PAG REQ TYPE 1
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	SS -> MS	IMM ASSIGNMENT	
	10	MS -> SS	PAGING RESPONSE
	11	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None.

#### 26.6.12.3	Channel release / TCH-F

##### 26.6.12.3.1	Conformance requirements

After the acknowledgement of the Layer 2 disconnection by the network the MS shall not produce any further RF-transmission.

References

GSM 04.08 section 3.5.1.

##### 26.6.12.3.2	Test purpose

To verify that the MS is able to correctly release a full-rate TCH after having received a CHANNEL RELEASE message.

##### 26.6.12.3.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Supported rate of TCH: TCH/F.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall initiate a Layer 2 disconnection process on the main signalling link. After the acknowledgement of the Layer 2 disconnection by the SS, the MS shall stop transmission of Layer 2 messages. This is verified for 3 seconds. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test

20 seconds

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Channel Type = "Bm + ACCHs"
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.
	6	MS -> SS	DISC
	7	SS -> MS	UA
		--------	--------	The SS verifies for 3 seconds that the
				MS does not produce any Layer 2 messages
		--------	--------	The SS waits 12 seconds to allow the
				MS to perform cell reselection
	8	SS -> MS	PAG REQ TYPE 1
	9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	10	SS -> MS	IMM ASSIGNMENT	
	11	MS -> SS	PAGING RESPONSE
	12	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

#### 26.6.12.4	Channel release / TCH-F - no L2 ACK

##### 26.6.12.4.1	Conformance requirements

After the expiry of timer T3110 the MS shall not produce any further RF-transmission.

References

GSM 04.08 section 3.5.1.

##### 26.6.12.4.2	Test purpose

To verify that the MS is able to correctly release a TCH/F after having received a CHANNEL RELEASE message, even if the SS does not L2 acknowledge the L2 DISC frame.

##### 26.6.12.4.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Supported rate of TCH: TCH/F.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message (with cause "abnormal release, unspecified"), after which the MS shall send at least 2 L2 DISC frames. The SS does not acknowledge any of the L2 DISC frames. After 2 seconds, the SS verifies for 3 seconds that the MS has stopped transmission of Layer 2 messages. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test

25 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Channel Type = "Bm + ACCHs"
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CHANNEL RELEASE	Cause value = "Abnormal release,
				unspecified".
	6	MS -> SS	DISC	The MS shall sned at
				least 2 L2 DISC frames, to which
				the SS does not respond. After a
				period of 2 seconds, the SS
				verifies for 3 seconds that the MS
				does not produce any further
				Layer 2 messages
		--------	--------	The SS waits 12 seconds to allow the
				MS to perform cell reselection
	7	SS -> MS	PAG REQ TYPE 1
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8
	10	MS -> SS	PAGING RESPONSE
	11	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None.

### 26.6.13	Test of starting time

The Assignment, Handover and immediate assignment procedures can include a delayed change of frequency list, MAIO and HSN. This series of tests checks the behavior of the Mobile Station when receiving channel allocation messages with a starting time and channel description for both before and after the starting time. Tests checking the phase 1 usage of the starting time (that is without a channel description for before the time) are included in the series related to immediate assignment, dedicated assignment and handover.

#### 26.6.13.1	Dedicated assignment with starting time / successful case / time not elapsed

##### 26.6.13.1.1	Conformance requirement

A Mobile Station receiving an ASSIGNMENT COMMAND message with a starting time and channel descriptions for both after and before the starting time, and ready to access before the indicated time has elapsed, shall perform the assignment on the channels as described for before the starting time and shall start using the new frequency parameters (frequencies and hopping sequence, or single frequency) in the correct time slot indicated by the starting time.

The Mobile Station shall accept the ASSIGNMENT COMMAND message for different message formatting, differing by the information elements used to describe frequency lists.

Reference(s)

GSM 04.08 sub-clauses 3.4.3.1, 9.1.2.

##### 26.6.13.1.2	Test purpose

To verify that the MS, after receiving an ASSIGNMENT COMMAND message with a starting time and channel  descriptions both for before and after the starting time, and ready to access before the indicated time, performs correctly the assignment using the description for before the time, and eventually starts using the frequency parameters  for after the time at the time indicated in the message.

##### 26.6.13.1.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a hopping SDCCH. Then the SS sends an ASSIGNMENT COMMAND message allocating a channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions (hopping case) for both before and after the starting time, as detailed in the "specific message contents" clause. The indicated time is such that the Mobile Station is ready to access before that time. The Mobile Station then accesses the channel as described for before the starting time. The MS shall eventually, at the TDMA frame defined by the contents of the "Starting Time" information element of the ASSIGNMENT COMMAND message, use the new frequency parameters . The verification is performed at the RF burst level.

Test parameters:

T1 is chosen arbitrarily to be between T0+60 and T0+100 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

Maximum duration of test

45 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	ASSIGN COMMAND	See specific message contents
	6	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel (before time
				parameters) after establishment of the main
				signalling link.
	7	--------	Time T1	The SS checks that the MS is transmitting
				now on the correct frequencies (after time
				parameters) and that the transmission started
				in the correct frame.
	8	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the ASSIGNMENT COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

ASSIGNMENT COMMAND

Information element			Value/remark

Channel Description, after time

Channel Type and TDMA offset	Chosen arbitrarily among that supported by the
				Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Channel Mode

Mode					Arbitrarily selected from capabilities declared for

the MS for the chosen type of channel

Mobile Allocation, after time		Chosen arbitrarily, at least one frequency.  In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T1

Channel Description, before time

Channel Type and TDMA offset	Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency and different

from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.2	Dedicated assignment with starting time / successful case / time elapsed

##### 26.6.13.2.1	Conformance requirement

A Mobile Station receiving an ASSIGNMENT COMMAND message with a starting time and channel descriptions for both after and before the starting time, and ready to access after the indicated time has elapsed, shall perform the assignment on the channels as described for after the starting time.

The Mobile Station shall accept the ASSIGNMENT COMMAND message for different message formattings, differing by the information elements used to describe frequency lists.

Reference(s):

GSM 04.08 sub-clauses 3.4.3.1, 9.1.2.

##### 26.6.13.2.2	Test purpose

To verify that the MS, after receiving an ASSIGNMENT REQUEST message with a starting time and channel  descriptions both for before and after the starting time, performs correctly the assignment using the frequency parameters  for after the time if the indicated time has already elapsed when the Mobile Station is ready to transmit.

##### 26.6.13.2.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a hopping SDCCH. Then the SS sends an ASSIGNMENT COMMAND message allocating a channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions (hopping case) for both before and after the starting time, as detailed in the "specific message contents" clause. The indicated time is such that the Mobile Station is ready to access only after that time. The Mobile Station then accesses the channel as described for after the starting time. The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

Maximum duration of test

45 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Hopping channel
	3	SS -> MS	IMM ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	ASSIGN COMMAND	See specific message contents
	6	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel (after time
				parameters) after establishment of the main
				signalling link.
	7	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the ASSIGNMENT COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

ASSIGNMENT COMMAND

Information element			Value/remark

Channel Description, after time

Channel Type and TDMA offset	Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time		Chosen arbitrarily, at least one frequency. In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T1

Channel Description, before time

Channel Type and TDMA offset	Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency and different

from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.3	Dedicated assignment with starting time and frequency redefinition / failure case / time not elapsed

##### 26.6.13.3.1	Conformance requirement

An MS, after receiving a FREQUENCY REDEFINITION message, shall keep the provided information until the time is elapsed. The Mobile Station must accept an intervening dedicated assignment, and, in case of failure of this assignment resulting in a return to the old channel before the time indicated in the FREQUENCY REDEFINITION message, shall return on the old channel with the frequency parameters in use at the moment of the reception of the FREQUENCY REDEFINITION message, and shall eventually start using the new frequency parameters in the correct time slot indicated by the starting time of the FREQUENCY REDEFINITION message.

Reference(s):

GSM 04.08 sub-clauses 3.4.3, 3.4.5.

##### 26.6.13.3.2	Test purpose

To verify that the MS, after receiving a FREQUENCY REDEFINITION and then an ASSIGNMENT REQUEST message with a starting time and channel  descriptions both for before and after the starting time, failing the assignment and returning on the old channel, and ready to access before the time indicated in the FREQUENCY REDEFINITION, resumes transmission on the channels used at the time of the reception of the FREQUENCY REDEFINITION message and eventually starts using the new frequency parameters  at the time indicated in the FREQUENCY REDEFINITION message.

##### 26.6.13.3.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported). Then the SS sends a FREQUENCY REDEFINITION message (starting time T1), which modifies the frequency parameters to be used by the MS. Then the SS sends an ASSIGNMENT COMMAND message, with a starting time (T2, possibly distinct from T1) and channel descriptions for both before and after the starting time. The channels and hopping sequences so allocated are distinct from those used and from those described by the FREQUENCY REDEFINITION message. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel and trigger the establishment of the main signalling link on the old channel. Then the MS shall send an ASSIGNMENT FAILURE message. Time T1 is chosen so it is reached only after the sending of the ASSIGNMENT FAILURE message. The MS shall eventually, at the TDMA frame defined by the contents of the "Starting Time" information element of the FREQUENCY REDEFINITION message, use the new frequency parameters. The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5000 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

T2 is set to T0+4000 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

Maximum duration of test

180 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	FREQUENCY REDEF
	6	SS -> MS	ASSIGNMENT COMMAND	Hopping channel, type among possible, 
				signalling mode.
	7	MS -> SS	ASSIGNMENT FAILURE	Sent on the correct channel (original
				parameters) after establishment of the main
				signalling link.
	8	--------	Time T1	The SS checks that the MS is transmitting
				now on the correct frequencies (parameters of
				the FREQUENCY REDEFINITION message)
				and that the transmissions started in the
				correct frame.
	9	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the ASSIGNMENT COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

FREQUENCY REDEFINITION

Information element			Value/remark

Channel Description

Channel Type and TDMA offset	Same as in IMMEDIATE ASSIGNMENT

Timeslot Number			Same as in IMMEDIATE ASSIGNMENT

Training Sequence Code		Same as in IMMEDIATE ASSIGNMENT

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different than those of the

IMMEDIATE ASSIGNMENT message

Mobile Allocation				Chosen arbitrarily, at least two frequencies, different

than those of the IMMEDIATE ASSIGNMENT message

Starting Time				T1

ASSIGNMENT COMMAND:

Information element			Value/remark

Channel Description, after time

Channel Type and TDMA offset	Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time		Chosen arbitrarily, at least two frequency

Starting Time				T2

Channel Description, before time

Channel Type and TDMA offset	Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least two frequencies, different

from "Mobile Allocation, after time"

#### 26.6.13.4	Dedicated assignment with starting time and frequency redefinition / failure case / time elapsed

##### 26.6.13.4.1	Conformance requirement

An MS, after receiving a FREQUENCY REDEFINITION message, shall keep the provided information until the time is elapsed. The Mobile Station must accept an intervening dedicated assignment, and, in case of failure of this assignment resulting in a return to the old channel after the time indicated in the FREQUENCY REDEFINITION message, shall return on the old channel with the frequency parameters indicated in the FREQUENCY REDEFINITION message.

Reference(s):

GSM 04.08 sub-clauses 3.4.3, 3.4.5.

##### 26.6.13.4.2	Test purpose

To verify that the MS, after receiving a FREQUENCY REDEFINITION and then an ASSIGNMENT REQUEST message with a starting time and channel  descriptions both for before and after the starting time, failing the assignment and returning on the old channel, and ready to access after the time indicated in the FREQUENCY REDEFINITION, resumes transmission using the new frequency parameters indicated in the FREQUENCY REDEFINITION message.

##### 26.6.13.4.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported). Then the SS sends a FREQUENCY REDEFINITION message (starting time T1), which modifies the frequency parameters to be used by the MS. Then the SS sends an ASSIGNMENT COMMAND message, with a starting time (T2) and channel descriptions for both before and after the starting time. Time T1 is chosen so it is reached after the sending of the ASSIGNMENT COMMAND message, but before the return on the old channel. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel with the new frequency parameters as indicated by the FREQUENCY REDEFINITION message, and trigger the establishment of the main signalling link on the old channel. Then the MS shall send an ASSIGNMENT FAILURE message. The verification is performed at the RF burst level.

Test parameters:

T2 is set to T0+5000 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

T1 is set to T0+10 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

Maximum duration of test

180 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	FREQUENCY REDEF
	6	SS -> MS	ASSIGNMENT COMMAND	Hopping channel, type among possible, 
				signalling mode.
	7	MS -> SS	ASSIGNMENT FAILURE	Sent on the correct channel (parameters
				from the FREQUENCY REDEF message)
				after establishment of the main signalling link.
	8	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the ASSIGNMENT COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

FREQUENCY REDEFINITION

Information element			Value/remark

Channel Description

Channel Type and TDMA offset		Same as in IMMEDIATE ASSIGNMENT

Timeslot Number			Same as in IMMEDIATE ASSIGNMENT

Training Sequence Code		Same as in IMMEDIATE ASSIGNMENT

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different than those of the

IMMEDIATE ASSIGNMENT message

Mobile Allocation				Chosen arbitrarily, at least two frequencies, different

than those of the IMMEDIATE ASSIGNMENT

message

Starting Time				T1

ASSIGNMENT COMMAND

Information element			Value/remark

Channel Description, after time

Channel Type and TDMA offset		Chosen arbitrarily among that supported by the
				Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time		Chosen arbitrarily, at least two frequencies

Starting Time				T2

Channel Description, before time

Channel Type and TDMA offset	Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least two frequencies, different

from "Mobile Allocation, after time"

#### 26.6.13.5	Handover with starting time / successful case / time not elapsed

##### 26.6.13.5.1	Conformance requirement

A Mobile Station receiving an HANDOVER COMMAND message with a starting time and channel descriptions for both after and before the starting time, and ready to access before the indicated time has elapsed, shall perform the handover on the channels as described for before the starting time and shall, if specified, the frequency list, MAIO and HSN  in the correct time slot indicated by the starting time.

The Mobile Station shall accept the HANDOVER  COMMAND message for different message formatting, differing by the information elements used to describe frequency lists.

Reference(s):

GSM 04.08 sub-clauses 3.4.4.1, 9.1.15.

##### 26.6.13.5.2	Test purpose

To verify that the MS, after receiving a HANDOVER COMMAND message with a starting time and channel  descriptions both for before and after the starting time, and ready to access before the indicated time, performs correctly the handover using the description for before the time, and then starts using the frequency parameters  for after the time at the time indicated in the message.

##### 26.6.13.5.3	Method of test

Initial condition(s)

System Simulator:

2 cells, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated, and camped on cell B.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a hopping SDCCH. After the SS has received measurements concerning cell B, the SS sends a HANDOVER COMMAND  message allocating a channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions (hopping case) for both before and after the starting time , as detailed in the "specific message contents" clause. The indicated time is such that the Mobile Station is ready to access before that time. The Mobile Station then accesses the channel as described for before the starting time. The MS shall eventually, at the TDMA frame defined by the contents of the "Starting Time" information element of the HANDOVER COMMAND message, use the new frequency parameters. The verification is performed at the RF burst level.

Test parameters:

T1 is chosen arbitrarily to be between T0+60 and T0+100 (mod 42432), where T0 is the frame number at which the first burst of the HANDOVER COMMAND message is sent.

Maximum duration of test

120 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	HANDOVER COMMAND	See specific message contents
	6	MS -> SS	HANDOVER COMPLETE	Sent on the correct channel (before time
				parameters) after establishment of the main
				signalling link.
	7	--------	Time T1	The SS checks that the MS is transmitting
				now on the correct frequencies (after time
				parameters) and that the transmissions
				started in the correct frame.
	8	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the HANDOVER COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

HANDOVER COMMAND:

Information element			Value/remark

Cell Description				As for cell B

Channel Description, after time

Channel Type and TDMA offset	Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Synchronisation indication			Non synchronised

Cell Channel Description			As for cell B

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time		Chosen arbitrarily, at least one frequency. In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T1

Channel Description, before time

Channel Type and TDMA offset	Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency, different

from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.6	Handover with starting time / successful case / time elapsed

##### 26.6.13.6.1	Conformance requirement

A Mobile Station receiving a HANDOVER COMMAND message with a starting time and channel descriptions for both after and before the starting time, and ready to access after the indicated time has elapsed, shall perform the handover on the channels as described for after the starting time.

The Mobile Station shall accept the HANDOVER COMMAND message for different message formattings, differing by the information elements used to describe frequency lists.

Reference(s):

GSM 04.08 sub-clauses 3.4.4.1, 9.1.15.

##### 26.6.13.6.2	Test purpose:

To verify that the MS, after receiving a HANDOVER COMMAND message with a starting time and channel  descriptions both for before and after the starting time, and ready to transmit after the indicated time, performs correctly the handover using the frequency parameters  for after the time.

##### 26.6.13.6.3	Method of test

Initial condition(s)

System Simulator:

2 cells, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated, and camped on cell A

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated, and camped on cell B.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a hopping SDCCH. After the SS has received measurements concerning cell B, the SS sends a HANDOVER COMMAND message allocating a channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions (hopping case) for both before and after the starting time., as detailed in the "specific message contents" clause. The indicated time is such that the Mobile Station is ready to access only after that time. The Mobile Station then accesses the channel as described for after the starting time.  The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5 (mod 42432), where T0 is the frame number at which the first burst of the HANDOVER COMMAND message is sent.

Maximum duration of test

120 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	HANDOVER COMMAND	See specific message contents
	6	MS -> SS	HANDOVER COMPLETE	Sent on the correct channel (after time
				parameters) after establishment of the main
				signalling link.
	7	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the HANDOVER COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

The contents of the HANDOVER COMMAND message sent at step 5 depends on the nature of the channel after the time. Two cases are possible.

HANDOVER COMMAND:

Information element			Value/remark

Cell Description				As for cell B

Channel Description, after time

Channel Type and TDMA offset	Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Synchronisation indication			Non synchronised

Cell Channel Description			As for cell B

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time		Chosen arbitrarily, at least one frequency. In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T1

Channel Description, before time

Channel Type and TDMA offset	Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency and different						from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.7	Handover with starting time and frequency redefinition / failure case / time not elapsed

##### 26.6.13.7.1	Conformance requirement

An MS, after receiving a FREQUENCY REDEFINITION message, shall keep the provided information until the time is elapsed. The Mobile Station must accept an intervening handover, and, in case of failure of this handover resulting in a return to the old channel before the time indicated in the FREQUENCY REDEFINITION message, shall return on the old channel with the frequency parameters in use at the moment of the reception of the FREQUENCY REDEFINITION message, and shall eventually start using the new frequency parameters in the correct time slot indicated by the starting time of the FREQUENCY REDEFINITION message.

Reference(s):

GSM 04.08 sub-clauses 3.4.4, 3.4.5.

##### 26.6.13.7.2	Test purpose:

To verify that the MS, after receiving a FREQUENCY REDEFINITION and then a HANDOVER COMMAND message with a starting time and channel  descriptions both for before and after the starting time, failing the handover, and ready to access on the old channel before the time indicated in the FREQUENCY REDEFINITION, resumes transmission on the channels used at the time of the reception of the FREQUENCY REDEFINITION message and eventually starts using the new frequency parameters  at the time indicated in the FREQUENCY REDEFINITION message.

##### 26.6.13.7.3	Method of test

Initial condition(s)

System Simulator:

2 cells, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated, and camped on cell A.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated, and camped on cell A.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported). Then the SS sends a FREQUENCY REDEFINITION message (starting time T1), which modifies the frequency parameters to be used by the MS. Then the SS sends a HANDOVER COMMAND message, with a starting time (T2, possibly distinct from T1) and channel descriptions for both before and after the starting time. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel and trigger the establishment of the main signalling link on the old channel. Then the MS shall send a HANDOVER FAILURE message. Time T1 is chosen so it is reached only after the sending of the HANDOVER FAILURE message. The MS shall eventually, at the TDMA frame defined by the contents of the "Starting Time" information element of the FREQUENCY REDEFINITION message, use the new frequency parameters. The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5000 (mod 42432), where T0 is the frame number at which the first burst of the HANDOVER COMMAND message is sent.

T2 is set to T0+4000 (mod 42432), where T0 is the frame number at which the first burst of the HANDOVER COMMAND message is sent.

Maximum duration of test

180 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	FREQUENCY REDEF
	6	SS -> MS	HANDOVER COMMAND	Hopping channel, type among possible, 
				signalling mode.
	7	MS -> SS	HANDOVER FAILURE	Sent on the correct channel (original
				parameters) after establishment of the main
				signalling link.
	8	--------	Time T1	The SS checks that the MS is
				transmitting now on the correct
				frequencies (parameters of the
				FREQUENCY REDEFINITION message) and 
				that the transmissions
				started in the correct frame.
	9	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the HANDOVER COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

FREQUENCY REDEFINITION

Information element			Value/remark

Channel Description

Channel Type and TDMA offset		Same as in IMMEDIATE ASSIGNMENT

Timeslot Number			Same as in IMMEDIATE ASSIGNMENT

Training Sequence Code		Same as in IMMEDIATE ASSIGNMENT

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different than those of

the IMMEDIATE ASSIGNMENT message

Mobile Allocation				Chosen arbitrarily, at least two frequencies, different

than those of the IMMEDIATE ASSIGNMENT

message

Starting Time				T1

HANDOVER COMMAND:

Information element			Value/remark

Cell Description				As for cell B

Channel Description, after time

Channel Type and TDMA offset		Chosen arbitrarily among that supported by the
					Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Synchronisation indication			Non synchronised

Cell Channel Description			As for cell B

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time		Chosen arbitrarily, at least one frequency. In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T2

Channel Description, before time

Channel Type and TDMA offset	Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency and different

from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.8	Handover with starting time and frequency redefinition / failure case / time elapsed

##### 26.6.13.8.1	Conformance requirement

An MS, after receiving a FREQUENCY REDEFINITION message, shall keep the provided information until the time is elapsed. The Mobile Station must accept an intervening handover, and, in case of failure of this handover resulting in a return to the old channel after the time indicated in the FREQUENCY REDEFINITION message, shall return on the old channel with the frequency parameters indicated in the FREQUENCY REDEFINITION message.

Reference(s):

GSM 04.08 sub-clauses 3.4.4, 3.4.5.

##### 26.6.13.8.2	Test purpose:

To verify that the MS, after receiving a FREQUENCY REDEFINITION and then a HANDOVER COMMAND message with a starting time and channel descriptions both for before and after the starting time, failing the handover and returning on the old channel, and ready to access after the time indicated in the FREQUENCY REDEFINITION, resumes transmission using the new frequency parameters indicated in the FREQUENCY REDEFINITION message.

##### 26.6.13.8.3	Method of test

Initial condition(s)

System Simulator:

2 cells, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated, and camped on cell A.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated, and camped on cell A.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported). Then the SS sends a FREQUENCY REDEFINITION message (starting time T1), which modifies the frequency parameters to be used by the MS. Then the SS sends a HANDOVER COMMAND message, with a starting time (T2, possibly distinct from T1) and channel descriptions for both before and after the starting time. Time T1 is chosen so it is reached after the sending of the HANDOVER COMMAND message, but before the return on the old channel. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel with the new frequency parameters as indicated by the FREQUENCY REDEFINITION message, and trigger the establishment of the main signalling link on the old channel. Then the MS shall send a HANDOVER FAILURE message. The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5000 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

T2 is set to T0+10 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

Maximum duration of test

180 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	FREQUENCY REDEF
	6	SS -> MS	HANDOVER COMMAND	Hopping channel, type among possible, 
				signalling mode.
	7	MS -> SS	HANDOVER FAILURE	Sent on the correct channel (parameters
				from the FREQUENCY REDEF message)
				after establishment of the main signalling link.
	8	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the HANDOVER COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

FREQUENCY REDEFINITION

Information element			Value/remark

Channel Description

Channel Type and TDMA offset		Same as in IMMEDIATE ASSIGNMENT

Timeslot Number			Same as in IMMEDIATE ASSIGNMENT

Training Sequence Code		Same as in IMMEDIATE ASSIGNMENT

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different than those of

the IMMEDIATE ASSIGNMENT message

Mobile Allocation				Chosen arbitrarily, at least two frequencies, different

than those of the IMMEDIATE ASSIGNMENT

message

Starting Time				T1

HANDOVER COMMAND

Information element			Value/remark

Cell Description				As for cell B

Channel Description, after time

Channel Type and TDMA offset		Chosen arbitrarily among that supported by the
				Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Synchronisation indication			Non synchronised

Cell Channel Description			As for cell B

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time		Chosen arbitrarily, at least one frequency. In case of
					an MA with just one frequency, the frequency should
					be different from the BCCH carrier.

Starting Time				T1

Channel Description, before time

Channel Type and TDMA offset	Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency and different
					from "Mobile Allocation, after time". In case of an MA
					with just one frequency, the frequency should be
					different from the BCCH carrier.

#### 26.6.13.9	Immediate assignment with starting time / successful case / time not elapsed

##### 26.6.13.9.1	Conformance requirement

A Mobile Station receiving an IMMEDIATE ASSIGNMENT message with a starting time and channel descriptions for both after and before the starting time, and ready to access before the indicated time has elapsed, shall perform the assignment on the channels as described for before the starting time and shall start using the new frequencies and hopping sequence in the correct time slot when the MS is allocated a dedicated channel.

Reference(s):

GSM 04.08 sub-clauses 3.3.1.3.1, 9.1.18.

##### 26.6.13.9.2	Test purpose

To verify that the MS, after receiving an IMMEDIATE ASSIGNMENT message with a starting time and channel  descriptions both for before and after the starting time, and ready to access before the indicated time, performs correctly the assignment using the description for before the time, and then starts using the frequency parameters  for after the time at the time indicated in the message.

##### 26.6.13.9.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator sends an IMMEDIATE ASSIGNMENT message allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions for both before and after the starting time. The indicated time is such that the Mobile Station is ready to access before that time. The Mobile Station then accesses the channel as described for before the starting time. The MS shall eventually, at the TDMA frame defined by the contents of the "Starting Time" information element of the IMMEDIATE ASSIGNMENT message, use the new frequency parameters. The verification is performed at the RF burst level.

Test parameters:

T1 is chosen arbitrarily to be between T0+60 and T0+100 (mod 42432), where T0 is the frame number at which the first burst of the IMMEDIATE ASSIGNMENT COMMAND message is sent.

Maximum duration of test

45 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	See specific message contents
	4	MS -> SS	PAGING RESPONSE.
	5	--------	Time T1	The SS checks that the MS is
				transmitting now on the correct
				frequencies (after time parameters) and 
				that the transmissions
				started in the correct frame.
	6	SS -> MS	CHANNEL RELEASE

Specific message contents

IMMEDIATE ASSIGNMENT COMMAND

Information element			Value/remark

Page Mode				Normal

Channel Description

Channel Type and TDMA offset	Chosen arbitrarily among that supported by the
				Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Timing Advance				As needed

Mobile Allocation (after time)		Chosen arbitrarily, at least one frequency. In case of
					an MA with just one frequency, the frequency should
					be different from the BCCH carrier.

Starting Time				T1

IA Rest Octet

MAIO					Chosen arbitrarily, different from "after time"

Mobile Allocation (before time)		Chosen arbitrarily, at least one frequency and different
					from "Mobile Allocation, after time". In case of an MA
					with just one frequency, the frequency should be
					different from the BCCH carrier.

#### 26.6.13.10	Immediate assignment with starting time / successful case / time elapsed

##### 26.6.13.10.1	Conformance requirement

A Mobile Station receiving an IMMEDIATE ASSIGNMENT message with a starting time and channel descriptions for both after and before the starting time, and ready to access after the indicated time has elapsed, shall perform the assignment on the channels as described for after the starting time.

Reference(s):

GSM 04.08 sub-clauses 3.3.1.3.1, 9.1.18.

26.6.13.10.2	Test purpose

To verify that the MS, after receiving an IMMEDIATE ASSIGNMENT message with a starting time and channel  descriptions both for before and after the starting time, performs correctly the assignment using the frequencies and hopping sequence for after the time if the indicated time has already elapsed when the Mobile Station is ready to transmit.

##### 26.6.13.10.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	Type of MS (P-GSM900 or EGSM or DCS1800).

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator sends an IMMEDIATE ASSIGNMENT message allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions for both before and after the starting time. The indicated time is such that the Mobile Station is ready to access only after that time. The Mobile Station then accesses the channel as described for after the starting time. The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5 (mod 42432), where T0 is the frame number at which the first burst of the IMMEDIATE ASSIGNMENT COMMAND message is sent.

Maximum duration of test

45 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	See specific message contents
	4	MS -> SS	PAGING RESPONSE.	The SS checks that the MS is
				transmitting now on the correct
				frequencies (after time parameters).
	5	SS -> MS	CHANNEL RELEASE

Specific message contents

IMMEDIATE ASSIGNMENT COMMAND:

Information element		Value/remark

Page Mode			Normal

Channel Description		Channel Type and TDMA offset
			Chosen arbitrarily among that supported by the
			Mobile Station

Timeslot Number		Chosen arbitrarily

Training Sequence Code	Chosen arbitrarily

Hopping			Yes

Hopping parameters		Chosen arbitrarily

Timing Advance			As needed

Mobile Allocation (after time)	Chosen arbitrarily, at least one frequency. In case of
				an MA with just one frequency, the frequency should
				be different from the BCCH carrier.

Starting Time			T1

IA Rest Octet

MAIO				Chosen arbitrarily, different from "after time"

Mobile Allocation (before time)	Chosen arbitrarily, at least one frequency and
				different from "Mobile Allocation, after time". In case of
				an MA with just one frequency, the frequency should
				be different from the BCCH carrier.

### 26.6.14	Default contents of GSM900 layer 3 messages for RR tests

This section contains the default values of GSM900 L3 messages, which unless indicated otherwise in section 26.6 shall be transmitted by the system simulator and which are required to be received from the GSM900 MS under test. These values are used in order to be consistent with the phase 2 version of 26.6.

The neccessary L3 messages are listed in alphabetic order, with the exception of the SYSTEM INFORMATION messages, where it is the information elements which are listed in alphabetic order (this is because some information elements occur in several SYSTEM INFORMATION types).

In this section, decimal values are normally used. However, sometimes a hexadecimal value, indicated by an 'H', or a binary value, indicated by a 'B' is used.

Default SYSTEM INFORMATION:

NOTE 22:	SYSTEM INFORMATION 2 bis, SYSTEM INFORMATION 5 bis, SYSTEM INFORMATION 7, and SYSTEM INFORMATION 8 messages are not used.

Cell A

Contents of information elements in SYSTEM INFORMATION TYPE 1 to 6 messages for cell A.

(CBCH) Channel Description			Not present

(CBCH) Mobile Allocation				Not present

Cell Channel Description

-	Format identifier			bit map 0

-	Cell Allocation ARFCN			Channel Numbers 20, 30, 50 and 70.

Cell Identity

-	Cell Identity Value			0001H

Cell Options

-	Power Control Indicator			Power Control Indicator is not set

-	DTX Indicator				MS shall not use DTX

-	Radio-Link-Timeout			8 SACCH blocks

Cell Selection Parameters

-	Cell-Reselect-Hysteresis			12 dB

-	MX-TXPWR-MAX-CCH			Minimum level

-	ACS					No addition cell parameters are present in

SYSTEM INFORMATION messages 7 and 8.

-	NECI					New establishment causes not supported

-	RXLEV-ACCESS-MIN			Minimum level

Control Channel Description

-	Attach-Detach allowed			No Attach/Detach

-	BS-AG-BLKS-RES			0 blocks reserved for access grant

-	CCCH-CONF				1 basic physical channel used for

CCCH, combined with SDCCHs

-	BS-PA-MFRMS			5 multiframe periods for transmission of

paging messages

-	T3212 Timeout value			Infinite

L2 pseudo length

-	System information 1			21

-	System information 2			22

-	System information 3			18

-	System information 4			12

Location Area Identification

-	Mobile Country Code			001 decimal

-	Mobile Network Code			01 decimal

-	Location Area Code			0001H

Message Type

-	System information 1			00011001

-	System information 2			00011010

-	System information 3			00011011

-	System information 4			00011100

-	System information 5			00011101

-	System information 6			00011110

Neighbour Cells Description

-	Format identifier			bit map 0

-	BCCH Allocation Sequence		0

-	BCCH Allocation ARFCN		Channels numbers 10, 20, 80, 90,

100, 110 and 120

-	EXT-IND				This IE carries the complete BA

NCC Permitted					0000 0010

RACH Control Parameters

-	Max Retrans				Max 1 retrans

-	Tx-integer				5 slots used

-	Cell Barred for Access			Cell is not barred

-	Call Reestablishment Allowed		Not allowed

-	Access Control Class			Access is not barred

-	Emergency Call allowed			Yes

SI 1 rest octets					Not used (all bits are set to spare)

SI 2 rest octets					Not used (all bits are set to spare)

SI 3 rest octets					Not used (all bits are set to spare)

SI 4 rest octets					Not used (all bits are set to spare)

Default settings for cell A:

Downlink input level				63 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power

class

Propagation profile				static

BCCH/CCCH carrier number			20

Cell B

The contents of SYSTEM INFORMATION TYPE 1 to 6 messages for cell B are identical to those of cell A with the following exceptions:

Cell Channel Description

-	Format Identifier			Bit map 0

-	Cell Allocation ARFCN			Channel Number 10

NOTE 23:	This IE needs modification when used in handover tests which command the MS to go to a frequency hopping channel in cell B.

Cell Identity

-	Cell Identity Value			0002H

Default settings for cell B:

Downlink input level				53 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power

class

Propagation profile				static

BCCH/CCCH carrier number			10

Contents of ALERTING message (SS to MS):

Protocol Discriminator				Call Control,

Transaction Identifier

TI value					As used in the SETUP message

TI flag					1 (destination side)

Message Type					00000001

All other information elements			Not present

Contents of ASSIGNMENT COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101110

Channel Description

-	Channel Type and TDMA offset		Bm + ACCHs

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code			Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Channel number 30

Power Command

-	Power level				Chosen arbitrarily by the test house

All other information elements			Not present

Contents of ASSIGNMENT COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101001

RR Cause

-	RR Cause Value			Normal event

Contents of ASSIGNMENT FAILURE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101111

RR Cause

-	RR Cause Value			Depending on test

Contents of AUTHENTICATION REQUEST message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					00010010

Ciphering Key Sequence Number

-	Key Sequence				Chosen randomly by the test house

from the range 0 to 6

Authentication Parameter RAND

-	RAND value				Chosen arbitrarily by the test house

Contents of AUTHENTICATION RESPONSE message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					0X010100

Other information element(s)			Not checked

Contents of CALL PROCEEDING message:

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag					1 (destination side)

Message Type					00000010

All other information elements			Not present

Contents of CHANNEL MODE MODIFY message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00010000

Channel Description

-	Channel Type and TDMA offset		Depending on test

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code			Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Channel number 30

Channel Mode

-	Mode					Depending on test

Contents of CHANNEL MODE MODIFY ACKNOWLEDGE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00010111

Channel Description

-	Channel Type and TDMA offset		Depending on test

-	Timeslot Number			Same as in the CHANNEL MODE MODIFY

message

-	Training Sequence Code			Same as in the CHANNEL MODE MODIFY

message

-	Hopping				Single RF channel

-	Frequency Band			Band number 0

-	ARFCN				Channel number 30

Channel Mode

-	Mode					Same as in the CHANNEL MODE MODIFY

message

Contents of CHANNEL RELEASE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00001101

RR Cause

-	RR Cause Value			Normal event

Contents of CHANNEL REQUEST message

Establishment Cause				Not checked

Random Reference				Not checked

Contents of CIPHERING MODE COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00110101

Cipher Mode Setting

-	algorithm identifier			cipher with A5/1

-	SC					Start ciphering

Cipher Response					IMEI shall not be included

Contents of CIPHERING MODE COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00110010

Mobile Identity					Not present

Contents of the CLASSMARK CHANGE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					000100110

Mobile Station Classmark 2			See PICS/PIXIT

Mobile Station Classmark 3			For presence and contents see PICS/PIXIT

Contents of CM SERVICE ACCEPT message:

Protocol Discriminator				Mobility Management

Skip Indicator					0000

Message Type					00100001

Contents of CM SERVICE REQUEST message

Protocol Discriminator				Mobility Management

Skip Indicator					0000

Message Type					0X100100

Other information elements				Not checked

Contents of CONNECT message (SS to MS)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag					1

Message Type					00000111

All other information elements			Not present

Contents of CONNECT ACKNOWLEDGE message (MS to SS)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag					0

Message Type					0X001111

Contents of HANDOVER ACCESS message:

Handover Reference				Equal to the value included in

the Handover Command message

Contents of HANDOVER COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Cell Description

-	Network Colour Code			1

-	Base station Colour Code		5

-	BCCH Carrier Number			Set to the BCCH carrier number of cell B.

(one of 10, 20, 80, 90, 100, 110 or 120)

Channel Description

-	Channel Type and TDMA offset		Bm + ACCHs

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code			Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Chosen randomly by the test house from those 
					supported on the target cell

Handover Reference

-	Handover Reference Value		Chosen arbitrarily by the test house.

Power Command

-	Power level				Chosen arbitrarily by the test house

All other information elements			Not present

Contents of HANDOVER COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101100

RR cause					Normal event

Time difference					Not present

Contents of HANDOVER FAILURE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101000

RR cause					Dependent on the test

Contents of IMMEDIATE ASSIGNMENT message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the IA rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111111

Page Mode

-	Page Mode				Normal Paging

Channel Description

-	Channel Type and TDMA offset		For non-combined CCCH/SDCCH (see initial

conditions), SDCCH/8, with subchannel

chosen randomly by

the testhouse

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

-	Timeslot Number			For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

-	Training Sequence Code			For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as

the BCC).

-	Hopping				Single RF channel

-	ARFCN				For non-combined CCCH/SDCCH (see initial

conditions), Channel number 30;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 20.

Request Reference				Pertaining to last Channel Request

sent by the MS

Timing Advance

-	Timing advance value			30 bit periods

Mobile Allocation

-	Length				0

Starting Time					Not present

IA rest octets					Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT EXTENDED message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the IAX rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 18.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111001

Page Mode

-	Page Mode				Normal Paging

Channel Description 1

- Channel Type and TDMA offset		For non-combined CCCH/SDCCH (see test

conditions), SDCCH/8, with subchannel

chosen randomly by the test house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

- Timeslot Number			For non-combined CCCH/SDCCH (see test

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

- Training Sequence Code		For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as the

BCC).

- Hopping				Single RF channel

- ARFCN				For non-combined CCCH/SDCCH (see initial

conditions),Channel number 30;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 20.

Request Reference 1				Pertaining to last Channel Request sent

by the MS

Timing Advance 1

-	Timing advance value			Chosen arbitrarily by the test house

Channel Description 2

-	Channel Type and TDMA offset		Same channel type as in Channel

Description 1, but different TDMA offset to that
					in Channel Description 1.

-	Timeslot Number			equal to the value in Channel Description 1.

-	Training Sequence Code			equal to the value in Channel Description 1.

-	Hopping				Single RF channel

-	ARFCN				equal to the value in Channel Description 1.

Request Reference 2				Not pertaining to any Channel Requests sent

by the MS.

Timing Advance 2

-	Timing advance value			Chosen arbitrarily by the test house

Mobile Allocation

-	Length				0

Starting Time					Not present

IAX rest octets					Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT REJECT message:

L2 pseudo length					19

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111010

Page Mode

-	Page Mode				Normal Paging

Request Reference				Pertaining to last Channel Request sent

by the MS

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

IAR rest octets					Not used (all bits set to spare)

Contents of LOCATION UPDATING REQUEST message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					0X001000

Other information elements		Not checked

Contents of PAGING REQUEST TYPE 1 message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the P1 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 9.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100001

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel			any channel

Mobile Identity 1

-	odd/even indication			Even

-	Type of Identity			TMSI

-	Identity Digits				TMSI previously allocated to MS

Mobile Identity 2					Not present

P1 rest octets					Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 2 message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the P2 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100010

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel			any channel

Mobile Identity 1

-	TMSI value				TMSI previously allocated to MS

Mobile Identity 2

-	TMSI value				TMSI not allocated to MS

Mobile Identity					Not present

P2 rest octets					Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 3 message:

L2 pseudo length					19

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100100

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel			any channel

Mobile identity 1

-	TMSI value				TMSI previously allocated to MS

Mobile identity 2

-	TMSI value				TMSI not allocated to MS

Mobile identity 3

-	TMSI value				TMSI not allocated to MS

Mobile identity 4

-	TMSI value				TMSI not allocated to MS

P3 rest octets					Not used (all bits set to spare)

Contents of PAGING RESPONSE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100111

Ciphering Key Sequence Number

-	Key Sequence				Key sequence number previously allocated

to MS, or '111' if no key is available.

Mobile Station Classmark 2

Mobile Identity

-	odd/even indication			Even

-	Type of identity			TMSI

-	Identity Digits				TMSI previously allocated to MS

Contents of PHYSICAL INFORMATION message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101101

Timing advance					20 bit periods

Contents of SETUP message; (MS to SS; for full rate speech)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					any value from the set {0, ..., 6}

TI flag					0

Message Type					0X000101

Other information elements				Not checked

Contents of SETUP message; (SS to MS; for full rate speech)

(* This message is not used in this direction in the RR tests- yet! It is put into hidden text in case a new test needs it.*)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					chosen randomly by the test house from the

set {0, ..., 6}

TI flag						0

Message Type					00000101

BC repeat indicator				Not present

Bearer capability 1

IEI						04h

length						01h

information transfer capability		speech

transfer mode				circuit mode

coding standard				GSM standardized coding

radio channel requirement			full rate channel

Signal							[Not present]

All other information elements			Not present

### 26.6.15	Default contents of DCS1800 layer 3 messages for RR tests

This section contains the default values of DCS1800 L3 messages, which unless indicated otherwise in section 26.6 shall be transmitted by the system simulator and which are required to be received from the DCS1800 MS under test. These values are used in order to be consistent with the phase 2 version of 26.6.

The neccessary L3 messages are listed in alphabetic order, with the exception of the SYSTEM INFORMATION messages, where it is the information elements which are listed in alphabetic order (this is because some information elements occur in several SYSTEM INFORMATION types).

In this section, decimal values are normally used. However, sometimes a hexadecimal value, indicated by an 'H', or a binary value, indicated by a 'B' is used.

Default SYSTEM INFORMATION:

NOTE 23:	SYSTEM INFORMATION 2 bis, SYSTEM INFORMATION 7, and SYSTEM INFORMATION 8 messages are not used.

SYSTEM INFORMATION 5 bis is not sent as a default message. For those tests which require SYSTEM INFORMATION 5 bis see the specific message contents for that test

Cell A

Contents of information elements in SYSTEM INFORMATION TYPE 1 to 6 messages for cell A.

(CBCH) Channel Description		Not present

(CBCH) Mobile Allocation			Not present

Cell Channel Description

-	Format identifier		Range 512

-	Cell Allocation ARFCN		Channel Numbers, 590, 650, 750 and 850

Cell Identity

-	Cell Identity Value		0001H

Cell Options

-	Power Control Indicator		Power Control Indicator is not set, 0

-	DTX Indicator			MS shall not use DTX

-	Radio-Link-Timeout		8 SACCH blocks

Cell Selection Parameters

-	Cell-Reselect-Hysteresis		12 dB

-	MX-TXPWR-MAX-CCH		Minimum level

-	ACS				No addition cell parameters are present in

SYSTEM INFORMATION messages 7 and 8.

-	NECI				New establishment causes not supported

-	RXLEV-ACCESS-MIN		Minimum level

Control Channel Description

-	Attach-Detach allowed		MS shall not apply

-	BS-AG-BLKS-RES		0 blocks reserved for access grant

-	CCCH-CONF			1 basic physical channel used for

CCCH, combined with SDCCHs

-	BS-PA-MFRMS			5 multiframe periods for transmission of

paging messages

-	T3212 Timeout value		Infinite

L2 pseudo length

-	System information 1		21

-	System information 2		22

-	System information 3		18

-	System information 4		12

Location Area Identification

-	Mobile Country Code		001 decimal

-	Mobile Network Code		01 decimal

-	Location Area Code		0001H

Message Type

-	System information 1		00011001

-	System information 2		00011010

-	System information 3		00011011

-	System information 4		00011100

-	System information 5		00011101

-	System information 6		00011110

Neighbour Cells Description

-	Format identifier		Range 512

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers, 520, 590, 600, 700, 780,

810, 870

-	EXT-IND			This IE carries the complete BA. EXT-IND is 0.

NCC Permitted				0000 0010

RACH Control Parameters

-	Max Retrans			Max 1 retrans

-	Tx-integer			5 slots used

-	Cell Barred for Access		Cell is not barred

-	Call Reestablishment Allowed	Not Allowed

-	Access Control Class		Access is not barred

-	Emergency Call allowed		Yes

SI 1 rest octets				Not used (all bits are set to spare)

SI 2 rest octets				Not used (all bits are set to spare)

SI 3 rest octets				Not used (all bits are set to spare)

SI 4 rest octets				Not used (all bits are set to spare)

Default settings for cell A:

Downlink input level			63 dBmicroVolt emf()

Uplink output power			minimum supported by the MS's power

class

Propagation profile			static

BCCH/CCCH carrier number		ARFN 590

Cell B

The contents of SYSTEM INFORMATION TYPE 1 to 6 messages for cell B are identical to those of cell A with the following exceptions:

Cell Channel Description

-	Format Identifier		Range 512

-	Cell Allocation ARFCN		Channel Number 520

NOTE 23:	This IE needs modification when used in handover tests which command the MS to go to a frequency hopping channel in cell B.

Cell Identity

-	Cell Identity Value		0002H

Default settings for cell B:

Downlink input level			53 dBmicroVolt emf()

Uplink output power			minimum supported by the MS's power

class

Propagation profile			static

BCCH/CCCH carrier number		10

Contents of ALERTING message (SS to MS):

Protocol Discriminator			Call Control,

Transaction Identifier

TI value				As used in the SETUP message

TI flag				1 (destination side)

Message Type				00000001

All other information elements		Not present

Contents of ASSIGNMENT COMMAND message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101110

Channel Description

-	Channel Type and TDMA offset	Bm + ACCHs

-	Timeslot Number		Chosen randomly by the test house

-	Training Sequence Code		Chosen randomly by the test house

-	Hopping			Single RF channel

-	ARFCN			Channel number 650

Power Command

-	Power level			Chosen arbitrarily by the test house

All other information elements		Not present

Contents of ASSIGNMENT COMPLETE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101001

RR Cause

-	RR Cause Value		Normal event

Contents of ASSIGNMENT FAILURE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101111

RR Cause

-	RR Cause Value		Depending on test

Contents of AUTHENTICATION REQUEST message:

Protocol Discriminator			MM message

Skip Indicator				0000

Message Type				00010010

Ciphering Key Sequence Number

-	Key Sequence			Chosen randomly by the test house

from the range 0 to 6

Authentication Parameter RAND

-	RAND value			Chosen arbitrarily by the test house

Contents of AUTHENTICATION RESPONSE message:

Protocol Discriminator			MM message

Skip Indicator				0000

Message Type				0X010100

Other information element(s)		Not checked

Contents of CALL PROCEEDING message:

Protocol Discriminator			Call Control

Transaction Identifier

TI value				As used in the SETUP message

TI flag				1 (destination side)

Message Type				00000010

All other information elements		Not present

Contents of CHANNEL MODE MODIFY message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00010000

Channel Description

-	Channel Type and TDMA offset	Depending on test

-	Timeslot Number		Chosen randomly by the test house

-	Training Sequence Code		Chosen randomly by the test house

-	Hopping			Single RF channel

-	ARFCN			Channel number 650

Channel Mode

-	Mode				Depending on test

Contents of CHANNEL MODE MODIFY ACKNOWLEDGE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00010111

Channel Description

-	Channel Type and TDMA offset	Depending on test

-	Timeslot Number		Same as in the CHANNEL MODE MODIFY

message

-	Training Sequence Code		Same as in the CHANNEL MODE MODIFY

message

-	Hopping			Single RF channel

-	Frequency Band		Band number 0

-	ARFCN			Channel number 650

Channel Mode

-	Mode				Same as in the CHANNEL MODE MODIFY

message

Contents of CHANNEL RELEASE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00001101

RR Cause

-	RR Cause Value		Normal event

Contents of CHANNEL REQUEST message

Establishment Cause			Not checked

Random Reference			Not checked

Contents of CIPHERING MODE COMMAND message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00110101

Cipher Mode Setting

-	algorithm identifier		cipher with A5/1

-	SC				Start ciphering

Cipher Response				IMEI shall not be included

Contents of CIPHERING MODE COMPLETE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00110010

Mobile Identity				Not present

Contents of the CLASSMARK CHANGE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				000100110

Mobile Station Classmark 2

- RF Power Capability		See PICS/PIXIT.

- Frequency Capability		Set to 0.

Mobile Station Classmark 3		For presence and contents see PICS/PIXIT

Contents of CM SERVICE ACCEPT message:

Protocol Discriminator			Mobility Management

Skip Indicator				0000

Message Type				00100001

Contents of CM SERVICE REQUEST message

Protocol Discriminator			Mobility Management

Skip Indicator				0000

Message Type				0X100100

Other information elements			Not checked

Contents of CONNECT message (SS to MS)

Protocol Discriminator			Call Control

Transaction Identifier

TI value				As used in the SETUP message

TI flag				1

Message Type				00000111

All other information elements		Not present

Contents of CONNECT ACKNOWLEDGE message (MS to SS)

Protocol Discriminator			Call Control

Transaction Identifier

TI value				As used in the SETUP message

TI flag				0

Message Type				0X001111

Contents of HANDOVER ACCESS message:

Handover Reference			Equal to the value included in

the Handover Command message

Contents of HANDOVER COMMAND message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101011

Cell Description

-	Network Colour Code		1

-	Base station Colour Code	5

-	BCCH Carrier Number		Set to the BCCH carrier number of cell B.

(one of 520, 590, 600, 700, 780, 810 or 870 )

Channel Description

-	Channel Type and TDMA offset	Bm + ACCHs

-	Timeslot Number		Chosen randomly by the test house

-	Training Sequence Code		Chosen randomly by the test house

-	Hopping			Single RF channel

-	ARFCN			Chosen randomly by the test house from those 
				supported on the target cell

Handover Reference

-	Handover Reference Value	Chosen arbitrarily by the test house.

Power Command

-	Power level			Chosen arbitrarily by the test house

All other information elements		Not present

Contents of HANDOVER COMPLETE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101100

RR cause				Normal event

Time difference				Not present

Contents of HANDOVER FAILURE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101000

RR cause				Dependent on the test

Contents of IMMEDIATE ASSIGNMENT message:

L2 pseudo length				This is the sum of the lengths of all the

information elements present in the

message except for the IA rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00111111

Page Mode

-	Page Mode			Normal Paging

Channel Description

-	Channel Type and TDMA offset	For non-combined CCCH/SDCCH (see initial

conditions), SDCCH/8, with subchannel

chosen randomly by

the testhouse

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

-	Timeslot Number		For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

-	Training Sequence Code		For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as

the BCC).

-	Hopping			Single RF channel

-	ARFCN			For non-combined CCCH/SDCCH (see initial

conditions), Channel number 650;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 590.

Request Reference			Pertaining to last Channel Request

sent by the MS

Timing Advance

-	Timing advance value		30 bit periods

Mobile Allocation

-	Length			0

Starting Time				Not present

IA rest octets				Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT EXTENDED message:

L2 pseudo length				This is the sum of the lengths of all the

information elements present in the

message except for the IAX rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 18.

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00111001

Page Mode

-	Page Mode			Normal Paging

Channel Description 1

- Channel Type and TDMA offset	For non-combined CCCH/SDCCH (see test

conditions), SDCCH/8, with subchannel

chosen randomly by the test house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

- Timeslot Number		For non-combined CCCH/SDCCH (see test

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

- Training Sequence Code	For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as the

BCC).

- Hopping			Single RF channel

- ARFCN			For non-combined CCCH/SDCCH (see initial

conditions),Channel number 650;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 590.

Request Reference 1			Pertaining to last Channel Request sent

by the MS

Timing Advance 1

-	Timing advance value		Chosen arbitrarily by the test house

Channel Description 2

-	Channel Type and TDMA offset	Same channel type as in Channel

Description 1, but different TDMA offset to that
				in Channel Description 1.

-	Timeslot Number		equal to the value in Channel Description 1.

-	Training Sequence Code		equal to the value in Channel Description 1.

-	Hopping			Single RF channel

-	ARFCN			equal to the value in Channel Description 1.

Request Reference 2			Not pertaining to any Channel Requests sent

by the MS.

Timing Advance 2

-	Timing advance value		Chosen arbitrarily by the test house

Mobile Allocation

-	Length			0

Starting Time				Not present

IAX rest octets				Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT REJECT message:

L2 pseudo length				19

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00111010

Page Mode

-	Page Mode			Normal Paging

Request Reference			Pertaining to last Channel Request sent

by the MS

Wait Indication				0 seconds

Request Reference			Not pertaining to the MS under test

Wait Indication				0 seconds

Request Reference			Not pertaining to the MS under test

Wait Indication				0 seconds

Request Reference			Not pertaining to the MS under test

Wait Indication				0 seconds

IAR rest octets				Not used (all bits set to spare)

Contents of LOCATION UPDATING REQUEST message:

Protocol Discriminator			MM message

Skip Indicator				0000

Message Type				0X001000

Other information elements			Not checked

Contents of PAGING REQUEST TYPE 1 message:

L2 pseudo length				This is the sum of the lengths of all the

information elements present in the

message except for the P1 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 9.

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00100001

Page Mode

-	Page Mode			Normal Paging

Channels needed

-	first channel			any channel

-	second channel		any channel

Mobile Identity 1

-	odd/even indication		Even

-	Type of Identity		TMSI

-	Identity Digits			TMSI previously allocated to MS

Mobile Identity 2				Not present

P1 rest octets				Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 2 message:

L2 pseudo length				This is the sum of the lengths of all the

information elements present in the

message except for the P2 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00100010

Page Mode

-	Page Mode			Normal Paging

Channels needed

-	first channel			any channel

-	second channel		any channel

Mobile Identity 1

-	TMSI value			TMSI previously allocated to MS

Mobile Identity 2

-	TMSI value			TMSI not allocated to MS

Mobile Identity				Not present

P2 rest octets				Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 3 message:

L2 pseudo length				19

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00100100

Page Mode

-	Page Mode			Normal Paging

Channels needed

-	first channel			any channel

-	second channel		any channel

Mobile identity 1

-	TMSI value			TMSI previously allocated to MS

Mobile identity 2

-	TMSI value			TMSI not allocated to MS

Mobile identity 3

-	TMSI value			TMSI not allocated to MS

Mobile identity 4

-	TMSI value			TMSI not allocated to MS

P3 rest octets				Not used (all bits set to spare)

Contents of PAGING RESPONSE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00100111

Ciphering Key Sequence Number

-	Key Sequence			Key sequence number previously allocated

to MS, or '111' if no key is available.

Mobile Station Classmark 2

Mobile Identity

-	odd/even indication		Even

-	Type of identity		TMSI

-	Identity Digits			TMSI previously allocated to MS

Contents of PHYSICAL INFORMATION message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101101

Timing advance				20 bit periods

Contents of SETUP message; (MS to SS; for full rate speech)

Protocol Discriminator			Call Control

Transaction Identifier

TI value				any value from the set {0, ..., 6}

TI flag				0

Message Type				0X000101

Other information elements			Not checked

Contents of SETUP message; (SS to MS; for full rate speech)

(* This message is not used in this direction in the RR tests- yet! It is put into hidden text in case a new test needs it.*)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					chosen randomly by the test house from the

set {0, ..., 6}

TI flag						0

Message Type					00000101

BC repeat indicator				Not present

Bearer capability 1

IEI						04h

length						01h

information transfer capability		speech

transfer mode				circuit mode

coding standard				GSM standardized coding

radio channel requirement			full rate channel

Signal							[Not present]

All other information elements			Not present

Specific message contents

IMMEDIATE ASSIGNMENT COMMAND:

Information element			Value/remark

Page Mode				Normal

Channel Description			Channel Type and TDMA offset

Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Timing Advance				As needed

Mobile Allocation (after time)		Chosen arbitrarily, at least one frequency. In

case of an MA with just one frequency, the

frequency should be different from the BCCH

carrier.

Starting Time				T1

IA Rest Octet

MAIO					Chosen arbitrarily, different from "after time"

Mobile Allocation (before time)		Chosen arbitrarily, at least one frequency

and different from "Mobile Allocation, after

time". In case of an MA with just one frequency,

the frequency should be different from the BCCH

carrier.