## 26.7	Elementary procedures of mobility management

The tests are based on GSM 04.08 and GSM 03.03.

In this section, when the expected sequence require that "a mobile originated CM connection is attempted", it shall be for a service other than emergency call.

In this section, a initial CM message is either a SETUP message, a REGISTER message or a CP-DATA message (in that case the acknowledged mode of operation on SAPI 3 will have be established and this message will be sent on SAPI 3).

### 26.7.1	TMSI reallocation

The intention of the TMSI Reallocation procedure is to assign a new temporary identity for the MS. If the message is not understood by the MS, the network could not establish a link to the MS. As this is a common MM procedure, it can be initiated at any time.

[Editorial Note:	- it is not checked that this procedure can be initiated at any time

]

#### 26.7.1.1	Conformance requirement

1)	A Mobile Station shall acknowledge a new TMSI when explicitly allocated during a location updating procedure or an incoming call.

2)	The TMSI shall be updated on the SIM when the Mobile Station is correctly deactivated in accordance with the manufacturer's instructions.

3)	A Mobile Station shall answer paging with this TMSI and includes it in the Paging Response message.

Reference(s):

GSM 04.08 section 4.3.1, GSM 03.03 section 2, GSM 02.17 section 6.1.

#### 26.7.1.2	Test purpose

To verify that the MS is able to receive and acknowledge a new TMSI by means of an explicit TMSI reallocation procedure.

To verify that the MS has stored the TMSI in a non-volatile memory.

The implicit reallocation procedure is tested in section 26.7.4.1.

#### 26.7.1.3	Method of test

Initial conditions

System Simulator:

Two cells A and B, belonging to different location areas a and b, default parameters.

Mobile Station:

The MS has valid TMSI (= TMSI1), CKSN, Kc. It is "idle updated" on cell B.

Related PICS/PIXIT statement(s)

Switch off button Yes/No

Way to bring the MS into service

Foreseen final state of the MS

The MS has a valid TMSI (= TMSI1), CKSN, Kc. It is "idle updated" on cell A.

Test Procedure

The MS is paged in cell B and the ciphering mode is established. An explicit TMSI reallocation procedure is performed. The channel is released. The MS is switched off and then its power supply is interrupted for 10 seconds. The power supply is resumed and then the MS is switched on. The system simulator checks, by paging, whether the MS has stored the received TMSI.

The MS is made to select cell A. A normal location updating procedure is performed in cell A. An explicit TMSI reallocation procedure is performed and then the location updating procedure is accepted by the SS. The system simulator checks, by paging, whether the MS has stored the allocated TMSI.

Maximum duration of test

2 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B

	1	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI1
	2	MS -> SS	CHANNEL REQUEST	Establishment Cause: Answer to paging.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	CIPH MODE COMMAND	The SS starts deciphering.
	6	MS -> SS	CIPH MODE COMPLETE	The SS starts enciphering.
	7	SS -> MS	TMSI REALLOCATION CMD	"Mobile identity"= new TMSI 
					(TMSI2) different from TMSI 1.
	8	MS -> SS	TMSI REALLOCATION CMP	
	9	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
					the SS waits for the disconnection 
					of the main signalling link.
	10	MS		If possible (see PICS), the MS is switched off
	10a	MS		The power supply is interrupted for
					10 seconds.
	11	MS		The MS is brought into service
	12	SS		The SS waits an amount of time 
					which is enough to guarantee that 
					the MS is in service (listening to 
					its paging subchannel)
	13	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI2.
	14	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to
					paging.
	15	SS -> MS	IMMEDIATE ASSIGNMENT	
	16	MS -> SS	PAGING RESPONSE	"Mobile identity" =TMSI2.
	17	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

The following messsages are sent and shall be
				received on cell A

	18	SS		The RF level of cell B is lowered 
				until the MS selects cell A. The RF 
				level of cell B is set sufficiently 
				low to ensure that cell B is not 
				suitable as defined in GSM 05.08 
				section 6.6.2
	19	MS -> SS	CHANNEL REQUEST	Establishment cause:
					Location updating
	20	SS -> MS	IMMEDIATE ASSIGNMENT	
	21	MS -> SS	LOCATION UPDATING REQ	location updating type= normal,
				"ciphering key sequence number"= 
				CKSN, LAI= b, "mobile identity"= 
				TMSI2.
	22	SS -> MS	TMSI REALLOCATION CMD	TMSI = TMSI1
	23	MS -> SS	TMSI REALLOCATION CMP	
	24	SS -> MS	LOCATION UPDATING ACC	This message does not contain the 
				optional Mobile Identity field.
	25	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link. The SS waits an 
				amount of time which is enough to
				guarantee that the MS is "idle updated" on cell A
	26	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the new TMSI (= TMSI1).
	27	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Answer to paging.
	28	SS -> MS	IMMEDIATE ASSIGNMENT	
	29	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the new TMSI (= TMSI1).
	30	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

### 26.7.2	Authentication

The purpose of this procedure is to verify the user identity. A correct response is essential to guarantee the establishment of the connection. If not, the connection will drop.

The SS shall be able to handle vectors of Kc, RAND, and SRES in a similar way as the MSC/BSS entities. The SS shall incorporate a test algorithm for generating SRES and Kc from RAND and Ki which operates as described in Annex 4.

#### 26.7.2.1	Authentication accepted

##### 26.7.2.1.1	Conformance requirement

1)	A Mobile Station shall correctly respond to an Authentication Request message by sending an Authentication Response message with the SRES information field set to the same value as the one produced by the authentication algorithm in the network.

2)	A Mobile Station shall indicate in a Paging Response message the ciphering key sequence number which was allocated to it through the authentication procedure.

Reference(s)

GSM 04.08 section 4.3.2, GSM 03.03 section 2

##### 26.7.2.1.2	Test purpose

1)	To check that a Mobile Station correctly responds to an Authentication Request message by sending an Authentication Response message with the SRES information field set to the same value as the one produced by the authentication algorithm in the network.

2)	To check that a Mobile Station indicates in a Paging Response message the ciphering key sequence number which was allocated to it through the authentication procedure.

##### 26.7.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has valid TMSI, CKSN (CKSN1), Kc. It is "idle updated" on the cell.

Related PICS/PIXIT statement(s)

None

Foreseen final state of the MS

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on the cell.

Test Procedure

The MS is paged. After the MS has sent a PAGING RESPONSE message to the SS, the SS initiates an authentication procedure and checks the value SRES sent by the MS in the AUTHENTICATION RESPONSE message. The channel is released. The MS is paged and the SS checks the value of the ciphering key sequence number sent by the MS in the PAGING RESPONSE message.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	 2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer
				to paging.
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	PAGING RESPONSE	CKSN= CKSN1
	 5	SS -> MS	AUTHENT REQUEST	The SS initiates authentication
				with CKSN2 different from CKSN1
	 6	MS -> SS	AUTHENT RESPONSE	"Auth.parameter SRES" IE shall
				be bit exact with the value as
				produced by the authentication
				algorithm.
	 7	SS -> MS	CHANNEL RELEASE	After the sending of this
		 		message, the SS waits for the
				disconnection of the main
				signalling link. The SS waits an 
				amount of time which is enough to 
				guarantee that the MS is in 
				service.

8	SS -> MS	PAGING REQUEST TYPE 1
	 9	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer
				to paging.
	 10	SS -> MS	IMMEDIATE ASSIGNMENT	
	 11	MS -> SS	PAGING RESPONSE	"Ciphering key sequence number" 
				shall be the same as the value that 
				was sent in the last AUTHENTICATION 
				REQUEST message (= CKSN2).
	12	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

#### 26.7.2.2	Authentication rejected

##### 26.7.2.2.1	Conformance requirement

1)	After reception of an Authentication Reject message the Mobile Station shall:

1.1	not perform normal location updating

1.2	not perform periodic location updating

1.3	not respond to paging with TMSI

1.4	reject any request from CM entity for MM connection except for emergency call

1.5	not perform IMSI detach if deactivated

2)	After reception of an Authentication Reject message the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a CHANNEL REQUEST message with the establishment cause set to "emergency call" and include an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	After reception of an Authentication Reject message the Mobile Station shall delete the stored LAI, CKSN and TMSI.

Reference(s)

GSM 04.08 sections 4.3.2.5.

##### 26.7.2.2.2	Test purpose

1)	To check that ,after reception of an Authentication Reject message, the Mobile Station :

1.1	does not perform normal location updating

1.2	does not perform periodic location updating

1.3	does not respond to paging with TMSI

1.4	rejects any request from CM entity for MM connection except for emergency call

1.5	does not perform IMSI detach if deactivated

2)	To check that, after reception of an Authentication Reject message the Mobile Station, if it supports speech, accepts a request for an emergency call by sending a CHANNEL REQUEST message with the establishment cause set to "emergency call" and includes an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	To check that, after reception of an Authentication Reject message and after having been deactivated and reactivated, the MS performs location updating using its IMSI as mobile identity and indicates deleted LAI and CKSN.

##### 26.7.2.2.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has valid TMSI, CKSN (CKSN2) and Kc. It is "idle updated" on cell B.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

Foreseen final state of the MS

The MS has valid TMSI, CKSN (CKSN1) and Kc. It is "idle updated" on cell A.

Test procedure

The SS rejects an authentication. The channel is released. The SS checks that the MS has entered the state MM-IDLE substate NO IMSI, i.e. does not perform normal location updating, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if powered down.

Maximum duration of test

10 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	SS -> MS	PAGING REQUEST TYPE 1
	 2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer
				to paging.
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	PAGING RESPONSE	"Ciphering key sequence number" 
				shall be the same as the value that 
				was sent in the last AUTHENTICATION 
				REQUEST message (= CKSN2).
	 5	SS -> MS	AUTHENT REQUEST
	 6	MS -> SS	AUTHENT RESPONSE
	 7	SS -> MS	AUTHENT REJECT
	 8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

9	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell B.
				"Mobile identity" IE contains
				TMSI.
	 10	   MS		The MS shall ignore this
				message. This is verified
				during 3 seconds.

11	   SS		The SS waits for at least for 15 s
	 12	   MS		A MO CM connection is attempted.
	 13	   MS		The MS shall not initiate an RR
				connection establishment on cell A or
				cell B. This is checked during 3 seconds

14	   MS		If the MS supports speech (see PICS),
				an emergency call is attempted.
	 15	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Emergency call.
	 16	SS -> MS	IMMEDIATE ASSIGNMENT	
	 17	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency call
				establishment. "Mobile identity" : type
				of identity is set to IMEI
	 18	SS -> MS	CM SERVICE ACCEPT	
	 19	MS -> SS	EMERGENCY SETUP
	 20	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.
	 21	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be received on cell A

22	   SS		The RF levels are changed
				to make the MS reselect
				the cell A.
	 23	   MS		The MS performs cell
				reselection according to
				procedure as specified in GSM
				05.08 (this however is not checked 
				until step 29). The MS shall not initiate
				an RR connection establishment on cell A
				or on cell B.

24	   SS		The SS waits at least 7 minutes
				for a possible periodic
				updating.
	 25 	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

26	   MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	 27	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

28	MS		Depending on what has been 
				performed in step 26 the MS is 
				brought back to operation.
	 29	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating.
	 30	SS -> MS	IMMEDIATE ASSIGNMENT	
	 31	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"Mobile Identity"= IMSI,
				"LAI"= deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
	 32	SS -> MS	AUTHENT REQUEST	"CKSN"= CKSN1
	 33	MS -> SS	AUTHENT RESPONSE	
	 34	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity"= TMSI
	 35	MS -> SS	TMSI REALLOCATION CMP
	 36	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

### 26.7.3	Identification

The purpose of this procedure is to check that the MS gives its identity as requested by the network. If this procedure does not work, it will not be possible for the network to rely on the identity claimed by the MS.

#### 26.7.3.1	General Identification

##### 26.7.3.1.1	Conformance requirement

1)	When requested by the network the Mobile Station shall send its IMSI.

2)	When requested by the network the Mobile Station shall send the TMSI which it was previously allocated.

3)	When requested by the network the Mobile Station shall send its IMEI as stored in the Mobile Equipement.

4)	When requested by the network the Mobile Station shall send its IMEISV as stored in the Mobile Equipment.

Reference(s)

GSM 04.08 sections 4.3.3.

##### 26.7.3.1.2	Test purpose

1)	To verify that the MS sends identity information as requested by the system in the following cases: IMSI and TMSI are requested in non-ciphered mode, IMEI is requested in ciphered mode.

2)	To verify that the MS sends its IMEI, when requested to do so, in non-ciphered mode.

3)	To verify that the MS sends its IMEISV, when requested to do so, in non-ciphered mode.

##### 26.7.3.1.3	Method of test

###### 26.7.3.1.3.1	Identification / test 1

Initial conditions

System Simulator:

1 cell, default values

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on the cell.

Related PICS/PIXIT statement(s)

IMEI of the ME

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on the cell.

Test Procedure

The SS requests identity information from the MS :

-	IMSI in non ciphering mode,

-	allocated TMSI in non ciphering mode,

-	IMEI in ciphering mode.

Maximum duration of test

30 seconds

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	
	 2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to
				paging.
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	PAGING RESPONSE	
	 5	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMSI.
	 6	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the IMSI of the MS.

7	SS -> MS	IDENTITY REQUEST	"Identity type" IE is TMSI.
	 8	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the allocated TMSI of the MS.

9	SS -> MS	CIPH MODE COMMAND
	10	MS -> SS	CIPH MODE COMPLETE
	11	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEI.
	12	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the IMEI stored in the Mobile
				Equipment.
	13	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

###### 26.7.3.1.3.2	Identification / test 2

Initial conditions

System Simulator:

1 cell, default values

Mobile Station:

The MS has a valid TMSI. It is in "idle updated".

Related PICS/PIXIT statement(s)

IMEI of the ME

IMEISV of the ME

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

The SS requests identity information from the MS :

-	IMEI in non ciphering mode.

-	IMEISV in non ciphering mode.

Maximum duration of test

30 seconds

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to
				paging.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEI.
	6	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the IMEI of the MS.
	7	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEISV
	8	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the IMEISV of the MS.

9	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

#### 26.7.3.2	Handling of IMSI shorter than the maximum length

##### 26.7.3.2.1	Conformance requirement

The MS shall be capable of handling an IMSI that is not of the maximum length.

Reference(s)

GSM 04.08 clause 10.5.1.4

##### 26.7.3.2.2	Test purpose

To check that the MS behaves correctly when activated with an IMSI of length less than the maximum length.

In this condition, the MS shall:

-	perform location updating

-	answer to paging with IMSI

-	give the correct IMSI when asked by an IDENTITY REQUEST

-	attempt CM connection establishment when requested to

-	attempt call re-establishment when needed

-	attempt IMSI detach when needed

-	erase its TMSI when the IMSI is sent by the network in a LOCATION UPDATING ACCEPT or a TMSI REALLOCATION COMMAND message.

##### 26.7.3.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default values

Mobile Station:

The MS has no valid TMSI.

It is "idle updated".

The IMSI has the value 001011234.

Related PICS/PIXIT statement(s)

On/Off switch - Yes/No

Foreseen final state of MS

The MS has no valid TMSI. It is in "idle, updated".

Test Procedure

The MS is paged with its IMSI. The MS shall answer to paging and include the correct IMSI in the PAGING RESPONSE message. During call establishment, the SS asks for the IMSI of the MS. The MS shall answer by an IDENTITY RESPONSE message including the correct IMSI. During the active phase of the call, the SS stops sending valid SACCH frames. The MS performs call re-establishment. The MS shall include the correct IMSI in the CM RE-ESTABLISHMENT message. a TMSI REALLOCATION COMMAND including a TMSI is sent to the MS. The MS acknowledges this message. The call is release.

The MS is paged with its TMSI. The MS shall answer to paging and includes its TMSI in the PAGING RESPONSE message. During call establishment, the SS sends a TMSI REALLOCATION COMMAND including the IMSI to the MS. The MS shall acknowledge this message. The MS shall erase its TMSI. The call is released.

The MS is switched off or powered down. The MS performs IMSI detach. The MS shall include the correct IMSI in the IMSI DETACH message.

The MS is switched on or powered on. The MS performs IMSI attach. The MS shall include the correct IMSI in the LOCATION UPDATING REQUEST message. A TMSI is allocated to the MS.

The LAC of the cell is changed. The MS performs location updating. The SS includes the IMSI in the LOCATION UPDATING ACCEPT message.

A mobile originated CM connection is attempted. The MS shall include the correct IMSI in the CM SERVICE REQUEST message.

Maximum duration of test

5 minutes.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	"mobile identity 1" contains IMSI of MS
	2	MS -> SS	CHANNEL REQUEST	Establishment cause: Answer to paging
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE	"mobile identity" contains the IMSI of the MS
	5	SS -> MS	IDENTITY REQUEST	"identity type" IE is IMSI
	6	MS -> SS	IDENTITY RESPONSE	"mobile identity" IE contains the IMSI of the MS
	7			The call is established using the sequence of the
				generic terminating call set-up procedure.

8	SS		The SS stops sending valid SACCH frames
	9	MS -> SS	CHANNEL REQUEST
	10	SS -> MS	IMMEDIATE ASSIGNMENT
	11	MS -> SS	CM RE-ESTAB REQUEST	"mobile identity" IE contains IMSI of the MS
	12	SS -> MS	TMSI REALLOC COMMAND	"mobile identity" contains a TMSI
	13	MS -> SS	TMSI REALLOC COMPLETE
	14	SS -> MS	CHANNEL RELEASE

15	SS -> MS	CHANNEL RELEASE	After sending this message, the SS waits for the
				disconnection of the main signalling link.

16	SS -> MS	PAGING REQUEST TYPE 1	"mobile identity 1" contains TMSI of MS
	17	MS -> SS	CHANNEL REQUEST	Establishment cause: Answer to paging
	18	SS -> MS	IMMEDIATE ASSIGNMENT
	19	MS -> SS	PAGING RESPONSE	"mobile identity" contains the TMSI of the MS
	20	SS -> MS	AUTHENTICATION REQUEST
	21	MS -> SS	AUTHENTICATION RESPONSE
	22	SS -> MS	TMSI REALLOC COMMAND	"mobile identity" contains a IMSI of MS
	23	MS -> SS	TMSI REALLOC COMPLETE
	24	SS -> MS	CHANNEL RELEASE

25	MS		If possible (see PICS) the MS is switched off,
				otherwise the MS is powered down.
	26	MS -> SS	CHANNEL REQUEST	If the MS was switched off it
				performs IMSI detach
	27	SS -> MS	IMMEDIATE ASSIGNMENT
	28	MS -> SS	IMSI DETACH	"mobile identity" contains IMSI of MS
	29	SS -> MS	CHANNEL RELEASE

30	MS		The MS is activated
	31	MS -> SS	CHANNEL REQUEST
	32	SS -> MS	IMMEDIATE ASSIGNMENT
	33	MS -> SS	LOCATION UPDATING REQ	"mobile identity" contains IMSI of MS
	34	SS -> MS	LOCATION UPDATING ACC	"mobile identity" contains a TMSI
	35	MS -> SS	TMSI REALLOC COMPLETE
	36	SS -> MS	CHANNEL RELEASE

37	SS		The SS changes the LAC of the cell
	38	SS		The SS waits for at least 30 seconds
	39	MS -> SS	CHANNEL REQUEST
	40	SS -> MS	IMMEDIATE ASSIGNMENT
	41	MS -> SS	LOCATION UPDATING REQ	"mobile identity" contains TMSI of the MS
	42	SS -> MS	LOCATION UPDATING ACC	"mobile identity" contains IMSI of the MS
	43	SS -> MS	CHANNEL RELEASE

44	MS		a mobile originated CM connection is attempted
	45	MS -> SS	CHANNEL REQUEST
	46	SS -> MS	IMMEDIATE ASSIGNMENT
	47	MS -> SS	CM SERVICE REQUEST	"mobile identity" contains IMSI of the MS
	48	SS -> MS	CHANNEL RELEASE

Specific message contents

None

### 26.7.4	Location updating

This procedure is used to register the MS in the network. If it is not performed correctly, no call can be established.

#### 26.7.4.1	Location updating / accepted

##### 26.7.4.1.1	Conformance requirement

1.

1.1	If the network accepts a location updating from the Mobile Station and reallocates a TMSI in the Location Updating Accept message the Mobile Station shall acknowledge the reception of the new TMSI.

1.2	The Mobile Station shall answer to paging with this TMSI and include it in a Paging Response message.

2	If the network accepts a location updating from the Mobile Station and the Location Updating Accept message contains neither TMSI nor IMSI, the Mobile Station shall answer to paging when addressed with the last allocated TMSI and include it in the Paging Response message.

3.

3.1	If the network accepts a location updating from the Mobile Station by use of a Location Updating Accept message containing the IMSI of the Mobile Station, the Mobile Station shall not answer paging with the last allocated TMSI.

3.2	The Mobile Station shall still answer paging with IMSI.

Reference(s)

GSM 04.08 section 4.4.4.6.

##### 26.7.4.1.2	Test purpose

To test the behaviour of the MS if the network accepts the location updating of the MS.

For the network response three different cases are identified:

1)	TMSI is allocated,

2)	Location updating accept contains neither TMSI nor IMSI,

3)	Location updating accept contains IMSI.

##### 26.7.4.1.3	Method of test

Initial conditions:

System Simulator:

Two cells, A and B, belonging to different location areas with location area identification a and b of the same PLMN.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has a valid TMSI (=TMSI1) and CKSN (=CKSN1). It is "idle updated" on cell A.

Related PICS/PIXIT statement(s)

None.

Foreseen final state of the MS

The MS has no valid TMSI. It has valid CKSN and Kc. It is "idle, updated" on cell B.

Test Procedure

The MS is made to select cell B. A normal location updating with TMSI reallocation is performed in cell B. The channel is released. The SS checks, by paging, that the MS has stored the newly allocated TMSI. The channel is released. The MS is made to select cell A. A normal location updating is performed in cell A. The LOCATION UPDATING ACCEPT message contains neither IMSI nor TMSI. The SS checks, by paging, that the MS has kept the old TMSI. The channel is released. The MS is made to select cell B. A normal location updating is performed in cell B. The LOCATION UPDATING ACCEPT message contains an IMSI. The SS checks, by paging, that the MS has deleted its TMSI and responds to paging with IMSI.

Maximum duration of test

4 minutes

Expected sequence

Step	Direction	Message	Comments

1	   SS		The RF level of cell A is
				lowered until the MS selects cell B.
	 2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating.
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= CKSN1, "location area
				identification"= a, "mobile station 
				classmark 1" as given by the PICS 
				and "mobile identity"= TMSI1.
	 5	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= new TMSI 
				(=TMSI2), LAI= b.
	 6	MS -> SS	TMSI REALLOCATION CMP	
	 7	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link. The SS waits an 
				amount of time which is enough to 
				guarantee that the MS is in service.

8	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the new TMSI (= TMSI2).
	 9	MS -> SS	CHANNEL REQUEST	
	 10	SS -> MS	IMMEDIATE ASSIGNMENT	
	 11	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the new TMSI (= TMSI2).
	 12	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main signalling link.

13	   SS		The RF level of cell B is
				lowered until the MS selects
				cell A.
	 14	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 15	SS -> MS	IMMEDIATE ASSIGNMENT	
	 16	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= CKSN1, "location area
				identification"= b, "mobile station 
				classmark 1" as given by the PICS 
				and "mobile identity"= TMSI2.
	 17	SS -> MS	LOCATION UPDATING ACC	"Mobile identity" IE not included
	 18	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link. The SS waits an 
				amount of time which is enough to 
				guarantee that the MS is in 
				service.

19	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the TMSI (= TMSI2).
	 20	MS -> SS	CHANNEL REQUEST	
	 21	SS -> MS	IMMEDIATE ASSIGNMENT	
	 22	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the TMSI (=TMSI2).
	 23	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

24	   SS		The RF level of cell A is
				lowered until the MS selects
				cell B.
	 25	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 26	SS -> MS	IMMEDIATE ASSIGNMENT	
	 27	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= CKSN1, "location area
				identification"= a, "mobile station 
				classmark 1" as given by the PICS 
				and "mobile identity"= TMSI2.
	 28	SS -> MS	LOCATION UPDATING ACC	"Mobile identity" IE contains
				IMSI.
	 29	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
				the SS waits for the disconnection 
				of the main signalling link. The SS
				waits an amount of time which is
				enough to guarantee that the MS is
				in service.

30	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the old TMSI (= TMSI2).
	 31	   MS		The MS shall ignore this
				message. This is checked during 5 
				seconds.

32	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the IMSI.
	 33	MS -> SS	CHANNEL REQUEST	
	 34	SS -> MS	IMMEDIATE ASSIGNMENT	
	 35	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the IMSI.
	 36	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

#### 26.7.4.2	Location updating / rejected

##### 26.7.4.2.1	Location updating / rejected / IMSI invalid

###### 26.7.4.2.1.1	Conformance requirement

1)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station shall:

1.1	not perform normal location updating

1.2	not perform periodic location updating

1.3	not respond to paging with IMSI

1.4	not respond to paging with TMSI

1.5	reject any request from CM entity for MM connection other than for emergency call

1.6	not perform IMSI detach if powered down

2)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a Channel Request message with the establishment cause set to "emergency call" and include an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station shall delete the stored LAI, CKSN and TMSI.

Reference(s)

GSM 04.08 sections 4.4.4.7.

###### 26.7.4.2.1.2	Test purpose

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "IMSI unknown in HLR", "illegal MS" or "Illegal ME".

###### 26.7.4.2.1.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas of the same PLMN.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support for speech Yes/No

Foreseen final state of the MS

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

Test Procedure

The SS rejects a normal location updating with the cause value "IMSI unknown in HLR". The channel is released. The SS checks that the MS has entered the state MM-IDLE and the substate NO IMSI, i.e. does not perform normal location updating when a new cell of the same or another PLMN is entered, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if powered down.

The test is repeated with cause value "Illegal MS" and with cause value "Illegal ME".

Maximum duration of test

25 minutes

Expected sequence

The sequence is executed for execution counter k = 1, 2, 3

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B.

	1	SS		The RF level of cell A is
				lowered until the MS selects
				cell B.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "IMSI
				unknown in HLR" for k = 1,
				"Illegal MS" for k = 2,
				"Illegal ME" for k = 3.
	 6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell A.
	7	SS		The RF levels are then changed
				again to make the MS reselect
				the cell A.
	8	MS		The MS performs cell
				reselection according to
				procedure as specified in GSM
				05.08 (this however is not checked 
				until step 18). The MS shall not initiate
				an RR connection establishment on cell A
				or on cell B.

9	   SS		The SS waits at least 7 minutes
				for a possible periodic
				updating.
	 10 	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B

11	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell A.
				"Mobile identity" IE contains
				IMSI.
	 12	   MS		The MS shall ignore this
				message. This is verified
				during 3 seconds.

13	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell A.
				"Mobile identity" IE contains
				TMSI.
	 14	   MS		The MS shall ignore this
				message. This is verified
				during 3 seconds.

15	   MS		A MO CM connection is attempted.
	 16	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds

17	   MS		An emergency call is attempted.
	 18	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Emergency call. This message is 
				sent in cell A
	 19	SS -> MS	IMMEDIATE ASSIGNMENT	
	 20	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency
				call establishment. "Mobile identity" : type
				of identity is set to IMEI
	 21	SS -> MS	CM SERVICE ACCEPT	
	 22	MS -> SS	EMERGENCY SETUP
	 23	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.
	 24	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

25	   MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	 26	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

27	MS		Depending on what has been 
				performed in step 25 the MS is 
				brought back to operation.
	 28	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 29	SS -> MS	IMMEDIATE ASSIGNMENT	
	 30	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available, "mobile 
				station classmark 1" as given by 
				the PICS, "Mobile Identity"= IMSI, 
				"LAI"= deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
	 31	SS -> MS	AUTHENT REQUEST	"CKSN"= CKSN1
	 32	MS -> SS	AUTHENT RESPONSE	
	 33	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity"= TMSI
	 32	MS -> SS	TMSI REALLOCATION CMP
	 33	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

##### 26.7.4.2.2	Location updating / rejected / PLMN not allowed

###### 26.7.4.2.2.1	Conformance requirement

1)	If the network reject a location updating from the Mobile Station with the cause "PLMN not allowed" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not perform IMSI detach when switched off,

1.3	not perform IMSI attach when switched on in the same location area,

1.4	not perform normal location updating when in the same PLMN and when that PLMN is not selected manually.

1.5	reject any request from CM entity for MM connection other than for emergency call

2)	If the network rejects a location updating from the Mobile Station with the cause "PLMN not allowed" the Mobile Station shall:

2.1	perform normal location updating when a new PLMN is entered

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

3)	If the network rejects a location updating from the Mobile Station with the cause "PLMN not allowed" and if after that the PLMN from which this rejection was received, is manually selected, the Mobile Station shall perform a normal location updating procedure.

Reference(s)

GSM 04.08 section 4.4.4.7

###### 26.7.4.2.2.2	Test purpose

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "PLMN not allowed".

###### 26.7.4.2.2.3	Method of test

###### 26.7.4.2.2.3.1	Location updating / rejected / PLMN not allowed / test 1

Initial conditions

System Simulator:

One cell: C, belonging to PLMN1.

Two cells : A and B, belonging to different location areas a and b and belonging to PLMN2. PLMN2 is different from HPLMN and from PLMN1.

IMSI attach/detach is allowed in cells A and B but not in cell C.

The T3212 timeout value is 1/10 hour in cells A and B.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell C.

The MS is in manual mode for PLMN selection.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

The MS is automatically in automatic mode after switch on Yes/No

Support for speech Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell C.The MS is in automatic mode for PLMN selection.

Test Procedure

The SS rejects a normal location updating with the cause value "PLMN not allowed". The channel is released. The SS checks that the MS does not perform periodic updating, does not perform IMSI detach, does not perform IMSI attach if activated in the same location area, rejects any request for CM connection establishment other than emergency call, accepts a request for an emergency call and performs normal location updating only when a new PLMN is entered.

Maximum duration of test

12 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B.

	1	MS		The MS is switched off (or power is removed).
	2	SS		The SS activates cells A and B and

deactivates cell C.
				Cell B has a level higher by
				at least 5 dB than cell A
	3	MS		The MS is switched on. (or power is reappliesd)
				If necessary the MS is put in manual selection
				mode. The MS shall
				offer the new PLMN as available to 
				the user. The PLMN is manually selected.
	 4	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 5	SS -> MS	IMMEDIATE ASSIGNMENT	
	 6	MS -> SS	LOCATION UPDATING REQ	
	 7	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= PLMN not allowed.
	 8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

9	   SS		The SS waits for a possible
				periodic updating for 7 minutes.
	 10	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

11	   MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	 12	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

13	   MS		Depending on what has been 
				performed in step 11 the MS is 
				brought back to operation. The MS is
				not made to select PLMN 2.
	 14	   MS		The MS shall not initiate an RR
				connection establishment. This is checked
				during 3 seconds.

The following message are sent and shall be
				received on cell A.

	 15	   SS		The RF level of cell B is
				lowered to make the MS reselect cell A.
	 16	   MS		No access to the network shall
				be registered by the SS within one minute.

17	   MS		An emergency call is attempted.
	 18	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Emergency call
	 19	SS -> MS	IMMEDIATE ASSIGNMENT	
	 20	MS -> SS	CM SERVICE REQUEST	"CM service type"= Emergency call
				establishment.
	 21	SS -> MS	CM SERVICE ACCEPT	
	 22	MS -> SS	EMERGENCY SETUP
	 23	SS -> MS	RELEASE COMPLETE	Cause IE : "unassigned number".
	 24	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

25	   MS		A MO CM connection is attempted.
	 26	   MS		The MS shall not initiate an RR connection
				establishment. This is checked
				during 3 seconds

The following messages are sent and shall be
				received on cell C

	 27	   MS		The MS is switched off.
	 28	   SS		The SS activates cell C
				and deactivates cells A and B
	 29	   MS		The MS is switched on. If necessary
				the MS is placed into the automatic mode.
	 30	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 31	SS -> MS	IMMEDIATE ASSIGNMENT	
	 32	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"LAI"= deleted LAI (the MCC and MNC hold
				the values of PLMN2, the LAC is coded FFFE)
				"mobile identity"= IMSI
	 33	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= TMSI.
	 34	MS -> SS	TMSI REALLOCATION CMP	
	 35	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

###### 26.7.4.2.2.3.2	Location updating / rejected / PLMN not allowed / test 2

Initial conditions

System Simulator:

One cell C, belonging to PLMN1.

Two cells A and B, belonging to different location areas a and b and belonging to PLMN2. PLMN2 is different from HPLMN.

IMSI attach/detach is allowed in cells A and B but not in cell C.

The T3212 timeout value is 1/10 hour in cells A and B.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell C.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

The MS is automatically in automatic mode after switch on Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle, updated"on cell C.

The MS is in automatic mode for PLMN selection.

Test Procedure

The SS rejects a normal location updating with the cause value "PLMN not allowed". The channel is released. Then the PLMN from which this rejection was received is manually selected and the SS checks that a normal location updating is performed.

Maximum duration of test

5 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B

	 1	MS		The MS is switched off (or power is removed).
	 2	SS		The SS activates cells A and B and
				deactivates cell C.
				Cell B has a level higher by at least
				5 dB than cell A
	3	MS		The MS is switched on (or power is reapplied).
	3a	MS		If the MS is in manual mode, it shall 
				offer the new PLMN as available to 
				the user. In this case the PLMN is manually 
				selected.
	 4	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 5	SS -> MS	IMMEDIATE ASSIGNMENT	
	 6	MS -> SS	LOCATION UPDATING REQ	
	 7	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= PLMN not
				allowed.
	 8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

9	   MS		The MS is made to search for
				PLMNs and the PLMN indicated 
				by the SS is manually selected
	 10	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 11	SS -> MS	IMMEDIATE ASSIGNMENT	
	 12	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"LAI"= deleted LAI (the MCC and MNC hold
				the values of PLMN2, the LAC is coded FFFE)
				"mobile identity"= IMSI

	 13	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell C.

 14	   MS		The MS is switched off.
	 15	   SS		The SS activates cell C
				and deactivates cells A and B
	 16	   MS		The MS is switched on.
				If necessary, the MS is put
				into the automatic mode.
	 17	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 18	SS -> MS	IMMEDIATE ASSIGNMENT	
	 19	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"LAI"= deleted LAI (the MCC and MNC hold
				the values of PLMN2, the LAC is coded FFFE)
				"mobile identity"= IMSI
	 20	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= TMSI.
	 21	MS -> SS	TMSI REALLOCATION CMP	
	 22	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

##### 26.7.4.2.3	Location updating / rejected / location area not allowed

###### 26.7.4.2.3.1	Conformance requirement

1)	If the network rejects a location updating from the Mobile Station with the cause "Location Area not allowed" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not respond to paging with TMSI,

1.3	reject any request from CM entity for MM connection other than for emergency call,

1.4	not perform IMSI detach.

2)	If the network rejects a location updating from the Mobile Station with the cause "Location Area not allowed" the Mobile Station shall:

2.1	perform normal location updating when a new location area is entered,

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

2.3	delete the list of forbidden LAs after switch off (power off).

Reference(s)

GSM 04.08 sections 4.4.4.7

###### 26.7.4.2.3.2	Test purpose

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "Location Area not allowed".

To test that the MS deletes the list of forbidden LAs after switch off (power off).

###### 26.7.4.2.3.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell A.

Related PICS/PIXIT statement(s)

Switch off on button Yes/No

Method to clear the list of forbidden location areas periodically.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell A.

Test Procedure

The SS rejects a normal location updating with the cause value "Location Area not allowed". The channel is released. The SS checks that the MS does not perform periodic updating, does not respond to paging with TMSI, rejects any requests from CM entities for MM-connections except emergency calls, does not perform IMSI detach, performs normal location updating when a new location area is entered and deletes the list of forbidden LAs when switched off.

Different types of MS may use different methods to periodically clear the list of forbidden location areas (eg. every day at 12am). If the list is cleared while the test is being run, it may be necessary to re-run the test.

Maximum duration of test

12 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B

	 1	   SS		The RF level of cell A is
				lowered so that cell B is selected, while keeping
				the C1 and C2 of cell A greater than 10.
	 2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	LOCATION UPDATING REQ	
	 5	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= "Location
				Area not allowed".
	 6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

7	   SS		The SS waits for a possible
				location updating for 7 minutes.
	 8	   MS		The MS shall not initiate an
				RR-connection establishment either on
				cell A or cell B.

9	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell B.
				"Mobile identity"= TMSI.
	 10	   MS		The MS shall ignore this message.
				This is checked during 3 seconds.

11	   MS		A MO CM connection is attempted.
	 12	   MS		The MS shall not initiate an RR 	connection
				establishment on cell A or cell B.
				This is checked during 3 seconds

13	   MS		An emergency call is attempted.
	 14	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Emergency call.
	 15	SS -> MS	IMMEDIATE ASSIGNMENT	
	 16	MS -> SS	CM SERVICE REQUEST	"CM service type":
				Emergency call establishment.
	 17	SS -> MS	CM SERVICE ACCEPT	
	 18	MS -> SS	EMERGENCY SETUP
	 19	SS -> MS	RELEASE COMPLETE	Cause: "unassigned number".
	 20	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

21	   MS		If possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	 22	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B (check
				for IMSI detach)
				This is checked during 3 seconds.

23	MS		Depending on what has been 
				performed in step 21 the MS is 
				brought back to operation.
	 24	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 25	SS -> MS	IMMEDIATE ASSIGNMENT	
	 26	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"LAI"= deleted LAI, "mobile 
				identity"= IMSI (This checks the
				deletion of the forbidden lists)
 	27	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= "Location
				Area not allowed".
	28	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell A

	 29	   SS		The RF level of cell B is
				lowered until the MS selects
				cell A.
	 30	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 31	SS -> MS	IMMEDIATE ASSIGNMENT	
	 32	MS -> SS	LOCATION UPDATING REQ	
	 33	SS -> MS	AUTHENT REQUEST	
	 34	MS -> SS	AUTHENT RESPONSE
	 35	SS -> MS	LOCATION UPDATING ACC	Mobile identity= TMSI
	 36	MS -> SS	TMSI REALLOCATION CMP
	 37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

##### 26.7.4.2.4	Location updating / rejected / national roaming

###### 26.7.4.2.4.1	Conformance requirement

1)	If the network rejects a location updating from the Mobile Station with the cause "National roaming not allowed in this area" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not respond to paging with TMSI,

1.3	reject any request from CM entity for MM connection other than for emergency call,

1.4	not perform IMSI detach.

2)	If the network rejects a location updating from the Mobile Station with the cause "National roaming not allowed in this area" the Mobile Station shall:

2.1	perform normal location updating when a new location area is entered,

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

2.3	periodically search for its HPLMN. [editorial note : no test purpose checks that]

3)	The mobile station shall reset the list of "forbidden location areas for national roaming" when powered down or when the SIM is removed.

4)	The MS shall be capable of storing at leat 6 entries in the list of "forbidden forbidden location areas for national roaming".

Reference(s)

GSM 04.08 section 4.4.4.7

###### 26.7.4.2.4.2	Test purposes

Test purpose 1

To test that on receipt of a rejection using the National Roaming cause code, the MS ceases trying to update on that cell, that this situation continues for at least one periodic location interval period, and that the corresponding list is re-set by powering down the MS.

Test purpose 2

To test that if no cell is available, the MS does not answer to paging with TMSI, rejects a request from CM entity other than for emergency calls.

Test purpose 3

To test that at least 6 entries can be held in the list of "forbidden location areas for national roaming" (the requirement in TS GSM 04.08 is to store at least 10 entries. This is not fully tested by the third procedure).

Test purpose 4

To test that if a cell of the Home PLMN is available then the MS returns to it in preference to any other available cell.

Test purpose 5

To test that if the SIM is removed the list of "forbidden location areas for national roaming"is cleared.

###### 26.7.4.2.4.3	Method of test

Initial conditions

The initial conditions shall be met before each of the different procedures.

System Simulator:

For procedures 1, 2, 3 and 5: Two cells A and B, belonging to different location areas of the same PLMN with LAI a and b. The MCC of that PLMN is the same as that of the HPLMN. The MNC of that PLMN is different from that of the HPLMN.

For procedure 4: three cells A, B, C of the same PLMN which is not the HPLMN with 3 different location area codes. Cells should differ in signal strength by 10 dB with cell A being the strongest and cell C the weakest. There should be a 20 dB range between A and C. A should be set to a level of - 40 dBm.

IMSI attach/detach is allowed in every cell.

The T3212 timeout value is 1/10 hour in every cell.

Mobile Station:

Procedures 1, 2, 3 and 5 : The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell B.

Procedure 4 : The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

The list of "forbidden location areas for national roaming" shall be empty (this may be achieved by either removing the SIM or switching the MS OFF then ON or powering down the MS depending on PICS).

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

Method to clear the list of location areas for national roaming periodically.

The MS is automatically in automatic mode after switch on Yes/No

Foreseen final state of the MS

Procedures 1 and 5 : The MS has no valid TMSI and no CKSN. It is "idle updated" on cell A.

Procedure 2 and 3: The MS has no valid TMSI and no CKSN. It is in the 'limited service' state on cell A.

Procedure 4 : The MS has no valid TMSI and no CKSN. It is "idle updated" on cell C.

Test Procedures

Procedure 1 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not perform periodic location updating procedure. The MS is turned off and then on. The SS checks that the MS performs location updating on the cell on which its location update request had been rejected (this checks that the LA is not the forbidden list after switch on). This procedure is performed another time but the deletion of the list is checked while removing the SIM (instead of turning off the MS).

Procedure 2 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not answer to a paging message with TMSI, rejects a request from CM entity but supports an emergency call.

Procedure 3 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". This is done for 6 different location areas. Then the SS checks that the MS does not attempt to begin a location updating procedure on the non-allowed location areas.

Procedure 4 :

The SS accepts a normal location updating on a cell not belonging to the HPLMN. Then when the MS attemps to perform a periodic location updating to this cell, the SS rejects this location updating with the cause value "National Roaming not allowed in this area". Two cells are then available, one belonging to the HPLMN but with the weakest level. It is checked that the MS returns to its HPLMN.

Procedure 5 : If SIM removal is possible without powering down :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not perform periodic location updating procedure. The SIM is removed and inserted in the MS. The SS checks that the MS performs location updating on the cell on which its location update request had been rejected (this checks that the LA is not the forbidden list after switch on).

Different types of MS may use different methods to periodically clear the list of forbidden areas (eg. every day at 12am) for national roaming. If the list is cleared while the test is being run, it may be necessary to re-run the test.

Maximum duration of test

Procedures 1 and 5: 12 minutes each

Procedure 2: 6 minutes

Procedure 3: 17 minutes

Procedure 4: 16 minutes

Expected sequence

The following procedure is used during the test:

Change_LAI (x):

-	the purpose of this procedure is to change the value of Location Area Identifier of cell x

-	The Location Area Identifier of cell x shall be changed. The code shall be chosen arbitrarily but shall be different from any previously used in this procedure. The code shall have the same MCC as the Home PLMN and shall not have the same MNC as the Home PLMN.

Procedure 1

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A.

	1	SS		The RF level of cell B is
					lowered until cell B is no
				more suitable and the the MS selects cell A.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
					Roaming not allowed in this 
					location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

7	SS		The SS waits at least 7 minutes
					for a possible location updating
	8 	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

9	MS		If possible (see PICS) the MS is 
				switched off. Otherwise 
				if possible the power is removed.
	10	MS		Depending on what has been 
				performed in step 9 the MS is 
				brought back to operation and placed
				in a automatic mode.
	11	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	12	SS -> MS	IMMEDIATE ASSIGNMENT	
	13	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= normal
	14	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	15	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main signalling link.

Procedure 2

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The RF level of cell B is
				lowered until the MS selects
				cell A. The level of cell B
				shall be such that cell B is
				suitable for cell selection.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message is sent on 
				cell A.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National"
				Roaming not allowed in this 
				location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell B

	7	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	8	SS -> MS	IMMEDIATE ASSIGNMENT	
	9	MS -> SS	LOCATION UPDATING REQ	
	10	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	11	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	12	SS		The SS waits for a possible 
				location updating 
				procedure on both cells A and B for 
				2 minutes.
	13	MS		The MS shall not initiate an RR connection 
				establishment  on  cell A or on cell B
				within 2 minutes after the end of step 11

14	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI.This message is
				sent on cell A and on cell B
	15	  MS		The MS shall not initiate an RR connection
				on cell A or on cell B. This is checked
				during 3 seconds.

16	   MS		A MO CM connection is attempted.
	17	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

The following messages are sent and shall be
				received on cell A

				Steps 20 to 27 are performed if the MS
				supports speech.

	18	MS		An emergency call is attempted.
	19	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Emergency call.
	20	SS -> MS	IMMEDIATE ASSIGNMENT	
	21	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency
				call establishment.
	22	SS -> MS	CM SERVICE ACCEPT	
	23	MS -> SS	EMERGENCY SETUP
	24	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.
	25	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Procedure 3

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The RF level of cell B is
				lowered until the MS selects
				cell A. The level of cell B
				shall be such that cell B is
				suitable for cell selection.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : 
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell B

	9	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	10	SS -> MS	IMMEDIATE ASSIGNMENT	
	11	MS -> SS	LOCATION UPDATING REQ	
	12	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	13	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	14	SS		Change_LAI (A) within 5 seconds 
				after step 12

The following messages are sent and shall be
				received on cell A

	17	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	18	SS -> MS	IMMEDIATE ASSIGNMENT	
	19	MS -> SS	LOCATION UPDATING REQ	
	20	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	21	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	22	SS		Change_LAI (B) within 5 seconds 
				after step 20

The following messages are sent and shall be
				received on cell B

	25	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	26	SS -> MS	IMMEDIATE ASSIGNMENT	
	27	MS -> SS	LOCATION UPDATING REQ	
	28	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	29	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	30	SS		Change_LAI (A) within 5 seconds 
				after step 28

The following messages are sent and shall be
				received on cell A

	33	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	34	SS -> MS	IMMEDIATE ASSIGNMENT	
	35	MS -> SS	LOCATION UPDATING REQ	
	36	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	38	SS		Change_LAI (B) within 5 seconds 
				after step 36

The following messages are sent and shall be
				received on cell B

	41	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	42	SS -> MS	IMMEDIATE ASSIGNMENT	
	43	MS -> SS	LOCATION UPDATING REQ	
	44	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	45	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	46	SS		The SS waits for a possible 
				location updating 
				procedure on both cells A and B for 
				7 minutes.
	47	MS		The MS shall not initiate an RR connection 
				establishment on cell A or on cell B
				within 7 minutes after the end of step 45

Procedure 4

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The SS waits for a 
					periodic location updating 
					procedure on cell A for 7 minutes 
					after the initial conditions have 
					been established.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= periodic
	5	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

	7	SS		The location area identity of cell 
					C shall be changed to that of a 
					location area in the Home PLMN

	8	SS		The SS waits for a  
					periodic location updating 
					procedure on cell A for 
					7 minutes.
	9	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
					This message is sent on 
					cell A within 7 minutes after the end
					of step 6.
	10	SS -> MS	IMMEDIATE ASSIGNMENT	
	11	MS -> SS	LOCATION UPDATING REQ	"Location updating type" = periodic
	12	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
					Roaming not allowed in this 
					location area"
	13	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

The following messages are sent and shall be
				received on cell C

	16	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	17	SS -> MS	IMMEDIATE ASSIGNMENT	
	18	MS -> SS	LOCATION UPDATING REQ	
	19	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	20	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

Procedure 5

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The RF level of cell B is
					lowered until the MS selects cell A.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
					Roaming not allowed in this 
					location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

7	SS		The SS waits at least 7 minutes
					for a possible location updating
	8 	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

9	MS		The SIM is removed
	10	MS		The SIM is inserted into the ME.
	11	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	12	SS -> MS	IMMEDIATE ASSIGNMENT	
	13	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= normal
	14	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	15	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

Specific message contents:

None.

#### 26.7.4.3	Location updating / abnormal cases

##### 26.7.4.3.1	Location updating / abnormal cases / random access fails

###### 26.7.4.3.1.1	Conformance requirement

If during the RR connection establishment phase of a normal location updating procedure, channel requests are not answered by the network, the Mobile Station shall:

1.	send (Max-Retrans+1) Channel Request messages

2.	not try to establish a connection during a period of T3213

3.	then perform a normal location updating procedure as it is still necessary.

4.	not repeat the complete procedure if the original cause of the location updating procedure has disappeared.

Reference(s):

GSM 04.08 section 4.4.4.9 and GSM 05.08 section 6.6.2

###### 26.7.4.3.1.2	Test purpose

To verify that when during the RR connection establishment phase of a location updating procedure, channel requests are not answered by the network, after expiry of T3213 (= 4s in Phase 2) the complete procedure is repeated if still necessary.

###### 26.7.4.3.1.3	Method of test

Initial conditions

System Simulator:

Two cells: A and B of the same PLMN, belonging to different location areas with LAI a and b.

The RF power level of cell B is higher than the one of cell A.

IMSI attach/detach is not allowed in both cells.

The T3212 timeout value is set to infinite in both cells.

Mobile Station:

The MS has a valid TMSI. It is "Idle updated" on cell B.

Related PICS/PIXIT statement(s)

None

Foreseen final state of the MS

The MS is "Idle updated" on cell A.

Test Procedure

The SS causes a random access failure in the MS during a normal location updating procedure. After the expiry of T3213 and when the cell reselection procedure is finished the MS will try to restart the normal location updating procedure.

The test is repeated but the original cause of the location updating procedure has disappeared. The SS then checks that the MS will not restart the location updating procedure.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	2	MS -> SS	CHANNEL REQUEST	Establishment cause:
					Location updating
				This message is sent by the MS 
				(Max_Retrans + 1) times
	3	   SS		The SS waits for 3 seconds
	4	   MS		The MS shall not send any layer 3
				message during this time.
	5	MS -> SS	CHANNEL REQUEST	Establishment cause:
					Location updating
				The time difference between this message
				and the last CHANNEL REQUEST sent in
				step 2 shall be in the range 4 s - 6 s.
	6	SS -> MS	IMMEDIATE ASSIGNMENT	
	7	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b,
				mobile station classmark 1 and
				mobile identity = TMSI.
	8	SS -> MS	LOCATION UPDATING ACC	Optional IE Mobile Identity not
				included
	9	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell B

	10	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	11	MS -> SS	CHANNEL REQUEST	Establishment cause:
					Location updating
				This message is sent by the MS 
				(Max_Retrans + 1) times
	12	SS		Immediately after the end of
				step 11 the RF level of cell A is
				set sufficiently high to ensure
				that cell A is suitable as defined
				in GSM 05.08 section 6.6.2
	13	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 6 seconds.

Specific message contents:

None.

##### 26.7.4.3.2	Location updating / abnormal cases / attempt counter less or equal to 4, LAI different

[Editorial Note:

- it is not checked that the MS does not perform periodic location updating and IMSI attach procedures

]

###### 26.7.4.3.2.1	Conformance requirement

1)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure, if the attempt counter is smaller than 4 and after expiry of T3211, the Mobile Station shall resend its Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "normal location updating".

2)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall:

2.1	not answer to paging with the previously allocated TMSI

2.2	not perform the IMSI detach procedure, when switched off.

3)	When a failure such as case e) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure and when an emergency call establishment is requested by the user the Mobile Station, if it supports speech, shall send a CM Service Request message with CM Service Type IE set to "emergency call establishment", CKSN IE set to "no key available" and Mobile Identity IE set to its IMSI and after acceptance by the network it shall send an Emergency Setup message.

4)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall use a request from CM entity other than emergency call as a trigger for a normal location updating procedure and shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

5)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall answer to paging with IMSI and shall send a Paging Response message with CKSN IE set to "no key available" and Mobile Identity IE set to its IMSI.

6)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall perform a normal location updating procedure as soon as it enters a new cell.

References

GSM 04.08 section 4.4.4.2 and 4.4.4.9 and GSM 05.08 section 6.6.2

###### 26.7.4.3.2.2	Test purpose

To verify that the MS performs normal location updating procedures when its attempt counter is smaller than 4.

To check that the MS does not perform the IMSI detach procedure when "idle not updated".

To verify that when "idle not updated" the MS can perform an emergency call.

To verify that when "idle not updated" the MS uses requests from CM layer other than emergency call as triggering of a normal location updating procedure.

To verify that the MS performs a normal location updating procedure if it enters a new cell while being "idle not updated".

###### 26.7.4.3.2.3	Method of test

Initial conditions

System Simulator:

Two cells: A and B of the same PLMN, belonging to different location areas with LAI a and b.

ATT flag shall be set to IMSI attach/detach allowed.

Mobile Station:

The MS is "idle updated" on cell A. A valid CKSN value is stored in the SIM and is noted "initial CKSN". A TMSI is allocated.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Foreseen final state of the MS

The MS is "Idle updated" on cell A with a valid CKSN and a TMSI.

Test Procedure

The MS is made to perform a normal location updating procedure. Four types of failure cases are triggered:

-	sending of a Location Updating Reject with cause randomly chosen between all defined cause values except 2, 3, 6, 11, 12 and 13 (which trigger a different action) (case g of GSM 04.08 section 4.4.4.9),

-	RR-connection failure (case d),

-	sending of a CHANNEL RELEASE message before the normal end of the procedure (case f),

-	T3210 timeout (case e).

As there is no stored LAI or the stored LAI is different from the broadcast LAI, and the attempt counter in the MS shall be lower than 4, the MS enters the state MM-IDLE and substate ATTEMPTING TO UPDATE and waits for T3211 seconds before trying again a location updating procedure.

Then the behaviour of the MS in the MM-IDLE ATTEMPTING TO UPDATE SERVICE state is checked, that is:

-	not answer to paging with TMSI,

-	not perform an IMSI detach procedure,

-	support request for emergency call,

-	use requests from CM layer other than emergency call as triggering of a normal location updating procedure,

-	perform normal location updating procedure when a new cell is entered.

Maximum duration of test

6 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	2	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	5	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value
				randomly chosen:
				* in Table 10.44 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being 
				excluded
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	7	MS		The MS shall not initiate an RR connection 
				establishment on cell A or on cell B
				 during T3211 seconds at least after
				the channel release.
	8	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	9	SS -> MS	IMMEDIATE ASSIGNMENT
	10	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.

	11	SS		The SS stops any RF transmission on 
				the dedicated channel. The SS waits 
				until there is no more SACCH frames 
				in the uplink direction

	12	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				 during T3211+RadioLinkTimeout seconds at 
				least after the channel release.
	13	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	14	SS -> MS	IMMEDIATE ASSIGNMENT

	15	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	16	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	17	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				 during T3211 seconds at least after
				the channel release.
	18	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	19	SS -> MS	IMMEDIATE ASSIGNMENT
	20	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	21	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	22	MS -> SS	AUTHENT RESPONSE
	23	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	24	MS -> SS	TMSI REALLOCATION CMP
	25	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle updated" in cell B

The following messages are sent and shall be received on cell A

26	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	27	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	28	SS -> MS	IMMEDIATE ASSIGNMENT	
	29	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	30	SS		chooses randomly one among the 
				three following steps: (5 and 6) or 
				11 or 16
	31	SS		performs the chosen step
	32	SS -> MS	PAGING REQUEST TYPE 1	Mobile identity = old TMSI of the 
				MS.
				This message is sent continuously to
				the MS during 8 seconds 
	33	SS		The SS checks that there is no 
				answer from the MS during 12 
				seconds
	33a	SS		If during steps 33 and 34 the MS 
				attempts to perform a location 
				updating procedure the SS will 
				perform steps 30 and 31 and then 
				continue the procedure.
	34	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) mobile switch off 
				is performed. Otherwise the power 
				is removed.
	35	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 30 seconds.
	36	MS		Depending on what has been 
				performed in step 34 the MS is 
				brought back to operation.
	37	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	38	SS -> MS	IMMEDIATE ASSIGNMENT
	39	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	40	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	41	MS -> SS	AUTHENT RESPONSE
	42	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	43	MS -> SS	TMSI REALLOCATION CMP
	44	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle updated" in cell A

45	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	46	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	47	SS -> MS	IMMEDIATE ASSIGNMENT	
	48	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	49	SS -> MS	AUTHENT REQUEST	
	50	MS->SS	AUTHENT RESPONSE	steps 49 and 50 are performed N 
				times. N shall be chosen in such a 
				way that T3210 expires.
	51	SS		The SS checks that there is no more 
				activity from the MS on the 
				channel.
	52	MS		The MS is made to perform an 
				emergency call
	53	MS -> SS	CHANNEL REQUEST	Establishment cause: Emergency call
	54	SS -> MS	IMMEDIATE ASSIGNMENT	
	55	MS -> SS	CM SERVICE REQUEST	CM service type = Emergency call 
				establishment; CKSN = no key 
				available; Mobile Identity = IMSI
	56	SS -> MS	CM SERVICE ACCEPT
	57	MS -> SS	EMERGENCY SETUP
	58	SS -> MS	RELEASE COMPLETE	Cause = unassigned number
	59	SS -> MS	CHANNEL RELEASE
	60	MS -> SS	CHANNEL REQUEST	Establishment cause : Location updating
				The SS will wait at most 15 
				seconds for this message
	61	SS -> MS	IMMEDIATE ASSIGNMENT
	62	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	63	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	64	MS -> SS	AUTHENT RESPONSE
	65	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	66	MS -> SS	TMSI REALLOCATION CMP
	67	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle updated" in cell B

68	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	69	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	70	SS -> MS	IMMEDIATE ASSIGNMENT	
	71	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	72	SS		chooses randomly one among the 
				three following steps: (5 and 6), 
				11 or 16
	73	SS		performs the chosen step
	74	MS		A MO CM connection is attempted
				before T3211 expiry
	75	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	76	SS -> MS	IMMEDIATE ASSIGNMENT
	77	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	78	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	79	MS -> SS	TMSI REALLOCATION CMP
	80	SS -> MS	CHANNEL RELEASE	
				Steps 81 to 84 are optional as the 
				MS may have memorized the request for
				CM connection attempt
				Wait 10 s to decide whether to go directly to
				step 85.
	81	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Not checked
	82	SS -> MS	IMMEDIATE ASSIGNMENT
	83	MS -> SS	CM SERVICE REQUEST	CKSN= 
				no key available,
				Mobile identity= TMSI
	84	SS -> MS	CHANNEL RELEASE	After the sending of this message,
				the SS waits for the disconnection of
				the main signalling link.
				MS is now "idle updated" in cell A

85	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	86	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	87	SS -> MS	IMMEDIATE ASSIGNMENT	
	88	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	89	SS		chooses randomly one among the 
				three following steps: (5 and 6)
				or 11 or 16
	90	SS		performs the chosen step and notes 
				the time when the CHANNEL RELEASE 
				message is sent
	91	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	92	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
				The time interval between the last 
				CHANNEL RELEASE message and this 
				message shall be less than 
				0.8*T3211
	93	SS -> MS	IMMEDIATE ASSIGNMENT	
	94	MS -> SS	LOCATION UPDATING REQ	location updating type= normal, 
				CKSN= no key available , 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	95	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	96	MS -> SS	AUTHENT RESPONSE
	97	SS -> MS	LOCATION UPDATING ACC	Mobile identity= TMSI
	98	MS -> SS	TMSI REALLOCATION CMP
	99	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "Idle, updated" in cell A

Specific message contents

None.

##### 26.7.4.3.3	Location updating / abnormal cases / attempt counter equal to 4

[Editorial Note:

- it is not tested that the MS does not answer to paging with TMSI

- in phase 1 Procedure 2 ended with "the MS is switched off" and it did not appear in the requirement that the MS would send an IMSI detach indication: it has been added and now the MS is brought back to operation at the end of the test

]

###### 26.7.4.3.3.1	Conformance requirement

1)	When four failures such as cases d) to g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure the Mobile Station shall:

1.1	not perform normal location updating procedures.

1.2	perform periodic location updating after T3212 expiry by sending a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "periodic updating".

1.3	if the periodic location updating was unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

2)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure the Mobile Station, if it supports speech, shall be able to perform an emergency call i.e. the Mobile Station is able to send a CM Service Request message with the CM Service Type IE set to "emergency call establishment", CKSN IE set to "no key is available" and Mobile Identity IE set to its IMSI and then send an Emergency Setup message.

3)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure

3.1	the Mobile Station shall use a request from CM entity for MM connection for a service other than emergency call as a trigger for a normal location updating procedure and shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

3.2	after a location updating triggered by a request from the CM layer which was .unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

4)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure

4.1	the Mobile Station shall perform a normal location updating procedure if it enters a new cell.

4.2	if this location updating is unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

References

GSM 04.08 section 4.4.4.9 and GSM 05.08 section 6.6.2

###### 26.7.4.3.3.2	Test purpose

To verify that the MS performs only periodic location updating procedures, that it does not perform normal location updating procedures when its attempt counter has reached value 4 and that the MS reset its attempt counter after a timer T3212 expiry.

To verify that the MS still follows the MM-IDLE ATTEMPTING TO UPDATE state requirements after its attempt counter has reached value 4.

To verify that the attempt counter is reset in the cases where it has to be done.

###### 26.7.4.3.3.3	Method of test

Initial conditions

System Simulator:

Two cells: A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

T3212 is set to 6 minutes.

Mobile Station:

The MS is "Idle updated" on cell B with a valid CKSN and a TMSI.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

Foreseen final state of the MS

The MS is "Idle updated" on cell A with a valid CKSN and a TMSI.

Test Procedure

The MS is made to perform a normal location updating. The SS triggers a failure in this procedure. After T3211 expiry the MS will try again the location updating procedure. The SS triggers again a failure. This is done again 2 times. At this point the attempt counter shall be equal to 4. It is then checked that T3212 has been started and that at its expiry the MS will try a periodic location updating procedure. It is verified that the MS has reset its attempt counter after timer T3212 expiry.

Then it is checked that, when the attempt counter has reached the value of 4, the MS is in the MM-IDLE state and ATTEMPTING TO UPDATE substate, that is:

-	not perform an IMSI detach procedure,

-	support request for emergency call,

-	use requests from CM layer other than emergency call as triggering of a normal location updating procedure,

-	perform normal location updating procedure when a new cell is entered.

Maximum duration of test

20 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be 
				received on cell A
	1	MS		The RF level of cell B is lowered
				until the MS selec_its cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	2	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	5	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value
				randomly chosen:
				* in Table 10.44 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being
				excluded
	6	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	7	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B 
				within T3211 + RadioLinkTimeOut
				seconds after the SS stops
				RF transmission
	8	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	9	SS -> MS	IMMEDIATE ASSIGNMENT
	10	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.

	11	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there is no more SACCH in the
				uplink
	12	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at least after the
				channel release
	13	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	14	SS -> MS	IMMEDIATE ASSIGNMENT
	15	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	16	SS -> MS	AUTHENT REQUEST

17	MS -> SS	AUTHENT RESPONSE	these steps (16 and 17) are 
				performed N times.
				N shall be chosen in such a way 
				that T3210 expires.
	18	MS		The MS shall cease transmission and then
				shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at least after the
				expiry of T3210
	19	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	20	SS -> MS	IMMEDIATE ASSIGNMENT

	21	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	22	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	23	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3212 (tolerance 
				-15s; 45s) at
				least after the channel release.
	24	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	25	SS -> MS	IMMEDIATE ASSIGNMENT	
	26	MS -> SS	LOCATION UPDATING REQ	location updating type : not checked
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	27	SS -> MS	LOCATION UPDATING REJ	IE Reject cause = #17 "network 
				failure"
	28	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	29	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	30	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	31	SS -> MS	IMMEDIATE ASSIGNMENT	
	32	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	33	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	34	MS -> SS	AUTHENT RESPONSE
	35	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	36	MS -> SS	TMSI REALLOCATION CMP
	37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle, updated" in cell A

38	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	39	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	40	SS -> MS	IMMEDIATE ASSIGNMENT	
	41	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	42	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value
				randomly chosen:
				* in Table 10.44 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being
				excluded
	43	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	44	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	45	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	46	SS -> MS	IMMEDIATE ASSIGNMENT
	47	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.

	48	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there is no more SACCH in the
				uplink
	48a	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				within T3211 + RadioLinkTimeOut seconds 
				after the SS stops RF Transmission.
	49	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	50	SS -> MS	IMMEDIATE ASSIGNMENT
	51	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	52	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	53	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.

54	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	55	SS -> MS	IMMEDIATE ASSIGNMENT	
	56	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	57	SS		chooses randomly one among the 
				three following steps: (42 and 
				43) or 48 or 52
	58	SS		performs the chosen step 
	59	MS		If the MS supports speech, it is made to perform
				an emergency call
	60	MS -> SS	CHANNEL REQUEST	Establishment cause: Emergency call
	61	SS -> MS	IMMEDIATE ASSIGNMENT	
	62	MS -> SS	CM SERVICE REQUEST	CM service type = Emergency call 
				establishment; CKSN = no key 
				available; Mobile Identity = IMSI
	63	SS -> MS	CM SERVICE ACCEPT
	64	MS -> SS	EMERGENCY SETUP
	65	SS -> MS	RELEASE COMPLETE	Cause = unassigned number
	66	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	67	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	68	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.
	69	MS		Depending on what has been 
				performed in step 67 the MS is 
				brought back to operation.
	70	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	71	SS -> MS	IMMEDIATE ASSIGNMENT
	72	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	73	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	74	MS -> SS	AUTHENT RESPONSE
	75	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	76	MS -> SS	TMSI REALLOCATION CMP
	77	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle, updated" in cell B

78	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	79	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	80	SS -> MS	IMMEDIATE ASSIGNMENT	
	81	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	82	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value
				randomly chosen:
				* in Table 10.44 of GSM 04.08, 
				causes #2, #3, #6, #11 #12, and #13 being
				excluded
	83	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	84	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	85	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	86	SS -> MS	IMMEDIATE ASSIGNMENT
	87	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.

	88	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there is no more SACCH in the
				uplink
	88a	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				within T3211 +RadioLinkTimeout
				seconds after the SS stops RF transmission
				.
	89	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	90	SS -> MS	IMMEDIATE ASSIGNMENT
	91	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	92	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	93	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.

94	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	95	SS -> MS	IMMEDIATE ASSIGNMENT	
	96	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	97	SS		chooses randomly one among the 
				three following steps: (42 and 
				43) or 48 or 52
	98	SS		performs the chosen step 

	99	MS		A MO CM connection is attempted
	100	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	101	SS -> MS	IMMEDIATE ASSIGNMENT
	102	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	103	SS		chooses randomly one among the 
				three following steps: (42 and 
				43) or 48 or 52
	104	SS		performs the chosen step 
	105	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	106	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	107	SS -> MS	IMMEDIATE ASSIGNMENT
	108	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	109	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	110	MS -> SS	AUTHENT RESPONSE
	111	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	112	MS -> SS	TMSI REALLOCATION CMP
	113	SS -> MS	CHANNEL RELEASE	MS is now "idle, updated" in cell A
				The MS may or may not have memorized the
				request for CM connection. The steps 114 to 
				117a are therefore optional for the MS.
				The SS waits 10 second whether to decide to
				go directly to step 118
	114	MS -> SS	CHANNEL REQUEST
	115	SS -> MS	IMMEDIATE ASSIGNMENT
	116	MS -> SS	CM SERVICE REQUEST	CKSN= initial value,
				Mobile identity= TMSI
	117	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	117a	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection of the main
				signalling link.

118	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.

119	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	120	SS -> MS	IMMEDIATE ASSIGNMENT	
	121	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	122	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value
				randomly chosen:
				* in Table 10.44 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being 
				excluded
	123	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	124	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	125	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	126	SS -> MS	IMMEDIATE ASSIGNMENT
	127	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.

	128	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there is no more SACCH in the
				uplink
	129	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				within T3211 + RadioLinkTimeOut seconds 
				after the SS stops RF transmission.

	130	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	131	SS -> MS	IMMEDIATE ASSIGNMENT
	132	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	133	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	134	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.

135	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	136	SS -> MS	IMMEDIATE ASSIGNMENT

137	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	138	SS		chooses randomly one among the 
				three following steps: (42 and 
				43) or 48 or 52
	139	SS		performs the chosen step 
	140	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	141	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	142	SS -> MS	IMMEDIATE ASSIGNMENT	
	143	MS -> SS	LOCATION UPDATING REQ	location updating type= normal, 
				CKSN= no key available , 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	144	SS		chooses randomly one among the 
				three following steps: (42 and 
				43) or 48 or 52
	145	SS		performs the chosen step 
	146	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				until T3211 + RadioLinkTimeout seconds 
				after the SS stops RF transmission.
	147	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	148	SS -> MS	IMMEDIATE ASSIGNMENT
	149	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	150	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	151	MS -> SS	AUTHENT RESPONSE
	152	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	153	MS -> SS	TMSI REALLOCATION CMP
	154	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle, updated" in cell A

Specific message contents

None.

##### 26.7.4.3.4	Location updating / abnormal cases / attempt counter less or equal to 4, stored LAI equal to broadcast LAI

[Editorial Note:

- In steps 14 and 34 time to check there is no location updating procedure is reduced from 0.5*T3212 (= 3 minutes) to 2*T3211 (=30 seconds). This time is put to check that T3211 has been stopped with the successful MM establishment. This will reduce the test time.

]

###### 26.7.4.3.4.1	Conformance requirement

1)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a periodic location updating procedure (the broadcast LAI is equal to the stored LAI),

1.1	the Mobile Station shall be able to establish an MM connection i.e. send a Channel Request and then a CM Service Request message, CKSN and LAI set to those which have been allocated to the Mobile Station, Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station

1.2	then the Mobile Station shall not attempt a location updating procedure.

2)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during an IMSI attach procedure (the broadcast LAI is equal to the stored LAI).

2.1	the Mobile Station shall be able to establish an MM connection i.e. send a Channel Request and then a CM Service Request message, CKSN and LAI set to those which have been allocated to the Mobile Station, Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station

2.2	then the Mobile Station shall not attempt a location updating procedure.

3)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a periodic location updating procedure and the attempt counter is smaller than 4 the Mobile Station shall send, after T3211 expiry, a Location Updating Request message with the Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station, CKSN IE and LAI set to those which have been allocated to the Mobile Station and the Location Updating type set to "periodic updating".

When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a periodic location updating procedure) after T3212 expiry it shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "normal".

4)	When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a periodic location updating procedure) it shall use a request for a CM connection other than emergency call as a trigger for a location updating procedure.

5)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during an IMSI attach procedure and the attempt counter is smaller than 4 the Mobile Station shall send, after T3211 expiry, a Location Updating Request message with the Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station, CKSN IE and LAI set to those which have been allocated to the Mobile Station and the Location Updating type set to "IMSI attach".

When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during an IMSI attach procedure) after T3212 expiry it shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "normal".

6)	When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during an IMSI attach procedure) it shall use a request for a CM connection other than emergency call as a trigger for a location updating procedure.

References

GSM 04.08 section 4.4.4.9

###### 26.7.4.3.4.2	Test purpose

To verify that in the case when the attempt counter is smaller than 4 and the broadcast LAI is equal to the stored LAI, the MS is in the MM-IDLE state and NORMAL SERVICE substate. To verify that timer T3211 is stopped after a MM connection establishment.

To verify that the MS uses the T3211 timer. and that it enters the MM-IDLE state and NORMAL SERVICE substate when its attempt counter reaches value 4 even in the case where the stored LAI is equal to the broadcast LAI.

###### 26.7.4.3.4.3	Method of test

Initial conditions

System Simulator:

One cell: B, belonging to location area b.

IMSI attach/detach is allowed.

T3212 is set to 6 minutes.

Mobile Station:

The MS is "Idle updated" on cell B with a valid CKSN and a TMSI.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Foreseen final state of the MS

The MS is "idle updated" on cell B with a valid CKSN and a TMSI.

Test Procedure

A failure during the periodic location updating is triggered: as the broadcast LAI is equal to the stored LAI, the MS is still in the MM-IDLE state and NORMAL SERVICE substate and timer T3211 is started. A CM connection other than for emergency call is attempted. It is checked that this is possible and that T3211 is stopped. Same test is performed with a failure during an IMSI attach procedure.

Then failures are triggered during the periodic location updating to let the attempt counter to reach the value of 4. The MS shall enter the MM-IDLE LIMITED SERVICE state and delete any TMSI, stored LAI, ciphering key sequence number and ciphering key. When the attempt counter reaches the value of 4, timer T3212 shall be started. At timer T3212 expiry a location updating procedure is started. A request for CM connection other for than emergency call shall trigger a location updating procedure.

Same tests are performed when the failures are triggered during an IMSI attach procedure.

Maximum duration of test

40 minutes

Expected sequence

Step	Direction	Message	Comments

1	SS		The SS shall wait at most T3212 + 45 seconds
	2	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	5	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	6	SS		performs the chosen step
	7	MS		A MO CM connection is attempted
	8	MS -> SS	CHANNEL REQUEST
	9	SS -> MS	IMMEDIATE ASSIGNMENT
	10	MS -> SS	CM SERVICE REQUEST	CKSN= initial CKSN, Mobile Identity= TMSI
	11	SS -> MS	CM SERVICE ACCEPT
	12	MS -> SS	An initial CM message
	13	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	14	SS		The MS shall not initiate an RR connection
				establishment. This is checked during 2*T3211

15	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.

Steps 16 to 20 are optional
	16	MS -> SS	CHANNEL REQUEST	
	17	SS -> MS	IMMEDIATE ASSIGNMENT	
	18	MS -> SS	IMSI DETACH INDICATION
	19	SS -> MS	CHANNEL RELEASE
	20	MS		Depending on what has been 
				performed in step 15 the MS is 
				brought back to operation.

21	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	22	SS -> MS	IMMEDIATE ASSIGNMENT	
	23	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	24	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	25	SS		performs the chosen step
	26	MS		A MO CM connection is attempted
	27	MS -> SS	CHANNEL REQUEST
	28	SS -> MS	IMMEDIATE ASSIGNMENT
	29	MS -> SS	CM SERVICE REQUEST	CKSN= initial CKSN, Mobile Identity= TMSI
	30	SS -> MS	CIPH MODE COMMAND
	31	MS -> SS	CIPH MODE COMPLETE
	32	MS -> SS	An initial CM message
	33	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection
				of the main signalling link
	34	SS		The MS shall not initiate an RR connection
				establishment. This is checked during 2*T3211 
				MS is "idle, updated" in cell B.

35	SS		The SS shall wait at most T3212 + 15 seconds
	36	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	37	SS -> MS	IMMEDIATE ASSIGNMENT	
	38	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	39	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	40	SS		performs the chosen step
	41	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	42	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	43	SS -> MS	IMMEDIATE ASSIGNMENT	
	44	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	45	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	46	SS		performs the chosen step
	47	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	48	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	49	SS -> MS	IMMEDIATE ASSIGNMENT	
	50	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	51	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	52	SS		performs the chosen step
	53	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	54	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	55	SS -> MS	IMMEDIATE ASSIGNMENT	
	56	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

57	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	58	SS		performs the chosen step
	59	MS		The MS shall not initiate an RR connection
				establishment during T3212 - 15 seconds at 
				least after the channel release.
	60	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	61	SS -> MS	IMMEDIATE ASSIGNMENT	
	62	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic 
				or normal (see Note 1), 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	63	SS -> MS	AUTHENT REQUEST
	64	MS -> SS	AUTHENT RESPONSE
	65	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI
	66	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	67	MS		The MS shall no initiate an RR connection
				establishment earlier than T3212 - 15 seconds
				after the transmission of the
				CHANNEL RELEASE in step 66
	68	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	69	SS -> MS	IMMEDIATE ASSIGNMENT	
	70	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	71	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	72	SS		performs the chosen step
	73	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	74	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	75	SS -> MS	IMMEDIATE ASSIGNMENT	
	76	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	77	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	78	SS		performs the chosen step
	79	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.

80	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	81	SS -> MS	IMMEDIATE ASSIGNMENT	
	82	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	83	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	84	SS		performs the chosen step
	85	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	86	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	87	SS -> MS	IMMEDIATE ASSIGNMENT	
	88	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	89	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	90	SS		performs the chosen step
	91	MS		A MO CM connection is attempted
	92	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	93	SS -> MS	IMMEDIATE ASSIGNMENT	
	94	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	95	SS -> MS	LOCATION UPDATING ACC	IE mobile identity = TMSI
	96	MS -> SS	TMSI REALLOCATION CMP
	97	SS -> MS	CHANNEL RELEASE

Steps 98 to 102 are optional
				Wait 10 s to decide whether to go directly to
				step 103.
	98	MS -> SS	CHANNEL REQUEST
	99	SS -> MS	IMMEDIATE ASSIGNMENT
	100	MS -> SS	CM SERVICE REQUEST	CKSN = no key available,
				Mobile identity= TMSI
	101	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	102	SS -> MS	CHANNEL RELEASE

103	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.

Steps 104 to 107 are optional
104	MS -> SS	CHANNEL REQUEST	
	105	SS -> MS	IMMEDIATE ASSIGNMENT	
	106	MS -> SS	IMSI DETACH INDICATION
	107	SS -> MS	CHANNEL RELEASE

	108	MS		Depending on what has been 
				performed in step 107 the MS is 
				brought back to operation.
	109	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	110	SS -> MS	IMMEDIATE ASSIGNMENT	
	111	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	112	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	113	SS		performs the chosen step
	114	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	115	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	116	SS -> MS	IMMEDIATE ASSIGNMENT	
	117	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	118	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	119	SS		performs the chosen step
	120	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	121	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	122	SS -> MS	IMMEDIATE ASSIGNMENT	
	123	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	124	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	125	SS		performs the chosen step
	126	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	127	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating

128	SS -> MS	IMMEDIATE ASSIGNMENT	
	129	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	130	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	131	SS		performs the chosen step
	132	MS		The MS shall not initiate an RR connection
				establishment during T3212 - 15 seconds at 
				least after the channel release.
	133	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	134	SS -> MS	IMMEDIATE ASSIGNMENT	
	135	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic 
				or normal or IMSI attach (see 
				Note 2), CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	136	SS -> MS	AUTHENT REQUEST
	137	MS -> SS	AUTHENT RESPONSE
	138	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI
	139	MS -> SS	TMSI REALLOCATION CMP
	140	SS -> MS	CHANNEL RELEASE

141	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.

Steps 142 to 145 are optional
	142	MS -> SS	CHANNEL REQUEST	
	143	SS -> MS	IMMEDIATE ASSIGNMENT	
	144	MS -> SS	IMSI DETACH INDICATION
	145	SS -> MS	CHANNEL RELEASE

146	MS		Depending on what has been 
				performed in step 144 the MS is 
				brought back to operation.
	147	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	148	SS -> MS	IMMEDIATE ASSIGNMENT	
	149	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	150	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	151	SS		performs the chosen step
	152	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	153	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	154	SS -> MS	IMMEDIATE ASSIGNMENT	
	155	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	156	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	157	SS		performs the chosen step
	158	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	159	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	160	SS -> MS	IMMEDIATE ASSIGNMENT	
	161	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	162	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	163	SS		performs the chosen step
	164	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	165	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	166	SS -> MS	IMMEDIATE ASSIGNMENT	
	167	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

168	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	169	SS		performs the chosen step
	170	MS		The MS is made to perform a MO call
	171	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	172	SS -> MS	IMMEDIATE ASSIGNMENT	
	173	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	174	SS -> MS	AUTHENT REQUEST
	175	MS -> SS	AUTHENT RESPONSE
	176	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI
	177	MS -> SS	TMSI REALLOCATION CMP
	178	SS -> MS	CHANNEL RELEASE

Steps 179 to 184 are optional
	179	MS	A MO CM connection is atempted
	180	MS -> SS	CHANNEL REQUEST
	181	SS -> MS	IMMEDIATE ASSIGNMENT
	182	MS -> SS	CM SERVICE REQUEST	CKSN = initial value,
				Mobile identity= TMSI
	183	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	184	SS -> MS	CHANNEL RELEASE

NOTE 1:	the MS can include both types of Location updating. As T3212 expires it can be a periodic location updating procedure and as there is no stored LAI it can be a normal one.

NOTE 2:	same problem as in note 1. Three types of location updating procedures should be allowed.

Specific message contents

None.

#### 26.7.4.4	Location updating / release / expiry of T3240

[ editorial notes :

- no conformance requirement

- new tests may be needed]

##### 26.7.4.4.1	Conformance requirement

The mobile station receiving a LOCATION UPDATING REJECT message shall start T3240: it shall abort the RR connection at the expiry of timer T3240.

References

GSM 04.08 section 4.4.4.8, 11.2.

##### 26.7.4.4.2	Test purpose

To verify that the MS aborts the RR-connection at the expiry of timer T3240.

##### 26.7.4.4.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell A.

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS is "idle updated" on cell B

Test Procedure

A normal location updating procedure is performed. The RR-connection is not released by the SS within the timer T3240. It is checked that the MS aborts the RR-connection.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	SS		The RF level of cell A is
				lowered until the MS selects
				cell B.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING ACC	
	6	SS		The SS waits T3240 expiry.
	7	MS		The MS shall abort the RR
				connection (disconnection of layer 2).

Specific message contents

None.

#### 26.7.4.5	Location updating / periodic

##### 26.7.4.5.1	Location updating / periodic spread

###### 26.7.4.5.1.1	Conformance requirement

1)	The Mobile Stations shall perform spreading of the time before performing a periodic location updating when the location updating timer value is reduced.

2)	The Mobile Station shall reset timer T3212 when the Mobile Station is deactivated, and shall start with a value between zero and the broadcasted value when reactivated in the same cell, IMSI attach being forbidden.

3)	When activated the Mobile Station shall start timer T3212 with a value randomly drawn in the allowed range.

NOTE 3:	This conformance requirement is not covered by a test purpose. It is intended to be covered by a manufacturer declaration.

References

GSM 04.08 section 4.4.2.

###### 26.7.4.5.1.2	Test purpose

1)	To check that when the location updating timer is reduced, the timer running in the MS is started with a value depending on the current timer value and the new broadcasted T3212 value.

2)	To verify that when the MS is reactivated in the same cell (as the one in which it was deactivated), IMSI attach being forbidden, the MS starts the timer T3212 with a value between zero and the broadcasted value

NOTE 4:	It is not tested that the value is random.

###### 26.7.4.5.1.3	Method of test

Initial conditions

System Simulator:

One cell, T3212 is set to 30 minutes.

IMSI attach is allowed in the cell.

Mobile Station:

The MS is deactivated. The stored MCC, MNC and LAC correspond to the broacasted values. The stored update status is "updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated. It performs IMSI attach. 3 minutes after the end of the IMSI attach procedure, the value of T3212 is set to 6 minutes. The MS shall perform periodic location updating 6 minutes after the end of the IMSI attach procedure.

Then, the IMSI attach/detach is forbidden. T3212 is still set to 6 minutes.

The MS is deactivated. The MS is reactivated. It is checked that the MS performs a periodic location updating during the 6 minutes following activation.

Maximum duration of test

20 minutes

Expected sequence

Step	Direction	Message	Comments

1	   MS		The MS is activated
	 2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	LOCATION UPDATING REQ	"location updating type": IMSI attach
	 5	SS -> MS	LOCATION UPDATING ACC	
	 6	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
					the SS waits for the disconnection 
					of the main signalling link.

7	   SS		3 minutes after step 6 the value of T3212
				is set to 6 minutes
	 8	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
				This message shall be sent
				by the MS between 5minutes 45s and
				6minutes 15s after step 6.
	 9	SS -> MS	IMMEDIATE ASSIGNMENT	
	 10	MS -> SS	LOCATION UPDATING REQ
				"location updating type": periodic updating
	 11	SS -> MS	LOCATION UPDATING ACC	
	 12	SS -> MS	CHANNEL RELEASE
				After the sending of this message, 
					the SS waits for the disconnection 
					of the main signalling link.

13	   SS		IMSI attach/detach is not allowed.
	 14	   MS		The MS is deactivated
	 15	   MS		The MS is activated
	 16	   SS		The SS waits until the periodic
				location updating.
	 17	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall
				arrive during the 7 minutes 
				following the MS activation.
	 18	SS -> MS	IMMEDIATE ASSIGNMENT	
	 19	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	 20	SS -> MS	LOCATION UPDATING ACC	
	 21	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.4.5.2	Location updating / periodic normal / test 1

###### 26.7.4.5.2.1	Conformance requirement

1	The Mobile Station shall stop and reset the timer T3212 of the periodic location updating procedure when the first MM message is received or ciphering mode setting is completed in the case of MM connection establishment.

2	The Mobile Station shall stop and reset the timer T3212 of the periodic location updating procedure when the Mobile Station has responded to paging and thereafter has received the first correct L3 message that is not an RR message.

References

GSM 04.08 section 4.4.2.

###### 26.7.4.5.2.2	Test purpose

To verify that the MS stops and resets the timer T3212 of the periodic location updating procedure when :

-	the first MM-message is received in the case of MM-connection establishment, ciphering mode being not set,

-	the MS has responded to paging and the first correct L3 message that is not an RR message is received.

NOTE 5:	T3212 is stopped when the MM-idle state is left and restarted when the MM sublayer returns to that state, substate NORMAL SERVICE or ATTEMPTING TO UPDATE. As a consequence, the exact time when T3212 is reset between those two events cannot be tested.

###### 26.7.4.5.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

IMSI attach/detach is not allowed.

The T3212 timeout value is 2/10 hour .

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

An MS originated MM connection is established and cleared. The channel is released. It is checked that the MS performs a periodic location updating 12 minutes after the release of the channel.

One minute after the periodic location updating, the MS is paged, it sends a CHANNEL REQUEST message and the SS responds with an IMMEDIATE ASSIGNMENT message, a call is established and then cleared. It is checked that the MS performs a periodic location updating 12 minutes after the release of the link.

Maximum duration of test

30 minutes.

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	7	SS		The SS waits until the periodic
				location updating.
	8	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall
				arrive between 11 minutes 45 s and
				12 minutes 15 s after
				the last release of the RR
				connection by the SS.
	9	SS -> MS	IMMEDIATE ASSIGNMENT	
	10	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	11	SS -> MS	LOCATION UPDATING ACC	
	12	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

13	   SS		The SS waits 1 minute.
	14	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity"= IMSI.
	15	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Answer to paging.
	16	SS -> MS	IMMEDIATE ASSIGNMENT	
	17	MS -> SS	PAGING RESPONSE	
	18	SS -> MS	AUTHENTICATION REQ
	19	MS -> SS	AUTHENTIC RESPONSE
	20	SS - MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	21	   SS		The SS waits until the periodic
				location updating.
	22	MS -> SS	CHANNEL REQUEST
				"Establishment cause" : Location updating
				This message shall
				arrive between 11 minutes 45 s and
				12 minutes 15 s after
				the last release of the RR
				connection by the SS.
	23	SS -> MS	IMMEDIATE ASSIGNMENT	
	24	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	25	SS -> MS	LOCATION UPDATING ACC	
	26	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.4.5.3	Location updating / periodic normal / test 2

###### 26.7.4.5.3.1	Conformance requirement

When a LOCATION UPDATING ACCEPT or a LOCATION UPDATING REJECT message is received, the timer T3212 is stopped and reset and the Mobile Station shall perform a periodic location updating after T3212 expiry.

References

GSM 04.08 section 4.4.2.

###### 26.7.4.5.3.2	Test purpose

To verify that the MS stops and resets the timer T3212 of the periodic location updating procedure when a LOCATION UPDATING ACCEPT message is received.

NOTE 6:	T3212 is stopped when the MM-idle state is left and restarted when the MM sublayer returns to that state, substate NORMAL SERVICE or ATTEMPTING TO UPDATE. As a consequence, the exact time when T3212 is reset between those two events cannot be tested.

###### 26.7.4.5.3.3	Method of test

Initial conditions

System Simulator:

2 cells, IMSI attach/detach is allowed in both cells.

T3212 is set to 6 minutes.

Mobile Station:

The MS has a valid TMSI. It is "idle updated"an cell A.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell B.

Test procedure

A normal location updating is performed. The channel is released. One minute later, the MS is deactivated, then reactivated in the same cell. It is checked that the MS performs an IMSI attach and a periodic location updating 6 minutes after the IMSI attach.

Maximum duration of test

20 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be 
				received on cell B.
	1	SS		The RF level of cell A is 
				lowered until the MS selects
				cell B.
	2	MS -> SS	CHANNEL REQUEST	"establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal
	5	SS -> MS	LOCATION UPDATING ACC	
	6	SS ->MS	CHANNEL RELEASE	After the sending of this message,
				the SS waits for the disconnection
				of the main signalling link.
	 7	   SS		The SS waits until the periodic
				location updating.
	 8	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall arrive between 5 minutes 45s
				and 6 minutes 15 s
				after the last release of the RR
				connection by the SS.
	 9	SS -> MS	IMMEDIATE ASSIGNMENT	
	 10	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	 11	SS -> MS	LOCATION UPDATING ACC	
	 12	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

13	   MS		If possible (see PICS) SIM 
				removal is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
				steps 14 to 17 may be performed or not
				depending on the action made in step 13
	 14	MS -> SS	CHANNEL REQUEST	
	 15	SS -> MS	IMMEDIATE ASSIGNMENT	
	 16	MS -> SS	IMSI DETACH INDICATION	
	 17	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	 18	   MS		Depending on what has been 
				performed in step 13 the MS is 
				brought back to operation.
	 19	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 20	SS -> MS	IMMEDIATE ASSIGNMENT	
	 21	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= IMSI 
				attach.
	 22	SS -> MS	LOCATION UPDATING ACC	
	 23	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	 24	   SS		The SS waits until the periodic
				location updating.
	 25	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall
				arrive between 5 minutes 45 s and 6 minutes 15s
				after the last release of the RR
				connection by the SS.
	 26	SS -> MS	IMMEDIATE ASSIGNMENT	
	 27	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	 28	SS -> MS	LOCATION UPDATING ACC	
	 29	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.4.5.4	Location updating / periodic HPLMN search

###### 26.7.4.5.4.1	Location updating / periodic HPLMN search / MS waits time T

###### 26.7.4.5.4.1.1	Conformance requirement

When in automatic mode and roaming in the home country, the MS shall make an attempt to access the HPLMN, if the MS is on the VPLMN at time T after since the last attempt.

NOTE 7:	This test is not intended to test every value in the range 6 minutes to 8 hours or the default of 30 minutes, but is intended to check that the mobile is capable of using the value stored on the SIM.

References

GSM 02.11 section 3.2.2.5.2

GSM 03.22 section 4.4.3.3

###### 26.7.4.5.4.1.2	Test purpose

To verify that when a cell of the HPLMN becomes available, following the successful location request on the VPLMN of the home country and after the first search the mobile has failed to find its HPLMN, that the MS shall perform a location update request on the HPLMN after time T. Were T is the HPLMN Search Period stored in the SIM.

###### 26.7.4.5.4.1.3	Method of test

Initial conditions

System Simulator :

Two cells A and B, belonging to different location areas with location identification a and b. Cell A shall be a cell of the HPLMN and Cell B shall be a cell of the VPLMN with a Country Code the same as that of Cell A. Initially Cell A shall not be broadcasting. IMSI attach/detach is not allowed on either cell.

Mobile Station :

The MS is switched off. The HPLMN Search Period on the SIM shall be set to 6 minutes. The location area information on the SIM is "deleted".

Related PICS/PIXIT statements

Switch on/off button	Yes/No.

Foreseen final state of the MS

The MS is "idle updated" on Cell A.

Test Procedure

Only Cell B shall be broadcasting. The MS shall be switched on either by using the Power Switch or by applying power. A normal location updating is performed on Cell B. Cell A shall be made available after 8 minutes, thus ensuring the MS fails to find the HPLMN during its first attempt. It is verified that the MS performs a location update request on Cell A, within 6 minutes after broadcasting of Cell A.

Maximum duration of test

17 minutes.

Expected sequence

Step	Direction	Message	Contents

The following messages shall be sent and
				received on Cell B.
	1	MS		The MS is switched on by either using the
				Power Switch or by applying power.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATE REQ	"Location Update Type" : Normal.
	5	SS -> MS	LOCATION UPDATE ACC
	6	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.
				The SS waits a period of 8 minutes, this
				allowing the MS to make its first
				periodic search

8	SS		Cell A is made available.
				Within 8 minutes after step 8 the following
				messages shall be sent and received on Cell A
	9	MS -> SS	CHANNEL REQUEST	"Establishment cause": Location updating
	10	SS -> MS	IMMEDIATE ASSIGNEMENT
	11	MS -> SS	LOCATION UPDATE REQ	"Location Update Type": normal
	12	SS -> MS	LOCATION UPDATE ACC
	13	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

Specific message contents

None.

###### 26.7.4.5.4.2	Location updating / periodic HPLMN search / MS in manual mode

###### 26.7.4.5.4.2.1	Conformance requirement

The periodic attempts shall only be performed if in automatic mode when the MS is roaming in its home country.

References

GSM 02.11 section 3.2.2.5.2

GSM 03.22 section 4.4.3.3

###### 26.7.4.5.4.2.2	Test purpose

To verify that no HPLMN Search is performed when the MS is not in automatic mode

###### 26.7.4.5.4.2.3	Method of test

Initial conditions

System Simulator :

Two cells A and B, belonging to different location areas with location identification a and b. Cell A shall be a cell of the HPLMN and Cell B shall be a cell of the VPLMN with a Country Code the same as that of Cell A. Initially Cell A shall not be broadcasting. IMSI attach/detach is not allowed on either cell.

Mobile Station :

The MS is switched off. The HPLMN Search Period on the SIM shall be set to 6 minutes. The location area information on the SIM is "deleted".

Related PICS/PIXIT statements

Switch on/off button	Yes/No.

Foreseen final state of the MS

The MS is "idle updated" on Cell B.

Test Procedure

Only Cell B shall be broadcasting. The MS shall be switched on either by using the Power Switch or by applying power. A normal location updating is performed on Cell B. The MS is forced into manual selection mode. Cell A is made available. It is verified that the MS does not attempt to perform a location update on Cell A.

Maximum duration of test

7 minutes.

Expected sequence

Step	Direction	Message	Contents

The following messages shall be sent and
				received on Cell B.
	1	MS		The MS is switched on by either using the
				Power Switch or by applying power.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATE REQ	"Location Update Type" : Normal.
	5	SS -> MS	LOCATION UPDATE ACC
	6	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

8	MS		The MS is forced into manual selection mode.
	9	SS		Cell A is made available.
	10	SS		The SS waits a period of 7 minutes. During this
				time no messages shall be received on Cell A.

Specific message contents

None.

###### 26.7.4.5.4.3	Location updating / periodic HPLMN search / MS waits at least two minutes and at most T minutes

###### 26.7.4.5.4.3.1	Conformance requirement

After switch on, the MS waits at least 2 minutes and at most T minutes before the first HPLMN Search is attempted.

References

GSM 02.11 section 3.2.2.5.2

GSM 03.22 section 4.4.3.3

###### 26.7.4.5.4.3.2	Test purpose

To verify that the MS waits at least 2 minutes and at most T minutes before attempting its first HPLMN Search.

###### 26.7.4.5.4.3.3	Method of test

Initial Conditions

System Simulator :

Two cells A and B, belonging to different location areas with location identification a and b. Cell A shall be a cell of the HPLMN and Cell B shall be a cell of the VPLMN with a Country Code the same as that of Cell A. Initially Cell A shall not be broadcasting. IMSI attach/detach is not allowed on either cell.

Mobile Station :

The MS is switched off. The HPLMN Search Period on the SIM shall be set to 6 minutes. The location area information on the SIM is "deleted".

Related PICS/PIXIT statements

Switch on/off button	Yes/No.

Foreseen final state of the MS

The MS is "idle updated" on Cell A.

Test Procedure

Only Cell B shall be broadcasting. The MS shall be switched on either by using the Power Switch or by applying power. A normal location updating is performed on Cell B. Cell A is made available. It is verified that the MS attempts to perform a location update on Cell A, after at least 2 minutes and at most T minutes have passed following power on.

Maximum duration of test

8 minutes.

Expected sequence

Step	Direction	Message	Contents

The following messages shall be sent and
				received on Cell B.
	1	MS		The MS is switched on by either using the
				Power Switch or by applying power.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATE REQ	"Location Update Type" : Normal.
	5	SS -> MS	LOCATION UPDATE ACC
	6	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

8	SS		Cell A is made available.
	9	SS		The SS waits a period of 2 minutes after the MS
				is switched on. During this time no messages
				shall be received on Cell A.

				The following messages shall be sent and
				received on cell A.
				Within T minutes after the MS is switched on
				the following messages shall be sent and
				received on cell A.
	10	MS -> SS	CHANNEL REQUEST	"Establishment cause": Location updating
				This message shall be sent between 2 and 7
				minutes after step 1
	11	SS -> MS	IMMEDIATE ASSIGNEMENT
	12	MS -> SS	LOCATION UPDATE REQ	"Location Update Type": normal
	13	SS -> MS	LOCATION UPDATE ACC
	14	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

Specific message contents

None.

#### 26.7.4.6	Location updating / interworking of attach and periodic

###### 26.7.4.6.1	Conformance requirement

1)	If the Mobile Station is in service state NO CELL AVAILABLE, LIMITED SERVICE, PLMN SEARCH or PLMN SEARCH-NORMAL SERVICE when the timer expires the location updating procedure is delayed until this service state is left.

2)	The T3212 timeout value shall not be changed in the NO CELL AVAILBLE, LIMITED SERVICE, PLMN SEARCH and PLMN SEARCH-NORMAL SERVICE states.

3)	If the selected cell is in the location area where the mobile station is registered and IMSI ATTACH is not required and timer T3212 has not expired, then the state is NORMAL SERVICE.

References

1	GSM 04.08 section 4.4.2.

2	GSM 04.08 section 4.4.2.

3	GSM 04.08 section 4.2.1.1

###### 26.7.4.6.2	Test purpose

1)	To check that if the PLU timer expires while the MS is out of coverage, the MS informs the network of its return to coverage.

2)	To check that the PLU timer is not disturbed by cells of forbidden PLMNs.

3)	To check that if the PLU timer does not expire while out of coverage and if the mobile returns to the LA where it is updated, the mobile does not inform the network of its return to coverage.

###### 26.7.4.6.3	Method of test

Initial conditions

System Simulator:

Two cells, a and b, of different PLMNs.

T3212 is set to 12 minutes on cell a.

T3212 is set to 6 minutes on cell b.

IMSI attach is allowed in both cells.

Mobile Station:

The MS is deactivated. The PLMN of cell b is entered in the SIM's forbidden PLMN list.

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS is "idle updated". The PLMN of cell b is entered in the SIM's forbidden PLMN list.

Test procedure

The MS is activated and placed in automatic network selection mode. It performs IMSI attach. 1 minute after the end of the IMSI attach procedure, cell a is switched off. The MS shall not location update on cell b. 8 minutes after the end of the IMSI attach procedure, cell a is switched on. The MS shall not location update on cell a before 11.75 minutes after the end of the IMSI attach procedure. The MS shall perform a periodic location update on cell a between 11.75 minutes and 12.25 minutes after the end of the IMSI attach procedure.

3 minutes after the end of the periodic location updating procedure, cell a is switched off. The MS shall not location update on cell b. 14 minutes after the end of the periodic location updating procedure, cell a is switched on and cell b is switched off. The MS shall perform a location update on cell a before 17 minutes after the end of the periodic location updating procedure.

Maximum duration of test

35 minutes

Expected sequence

Step	Direction	Message	Comments

1	MS		The MS is activated in automatic 
				network selection mode
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	"location updating type": IMSI attach
	5	SS -> MS	LOCATION UPDATING ACC	
	6	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
				the SS waits for the disconnection 
				of the main signalling link.
	7	SS		1 minute after step 6, cell a is switched off
	8	SS		8 minutes after step 6, cell a is switched on
	9	MS -> SS	CHANNEL REQUEST	This message shall be sent
				by the MS between 11 minutes 45s and
				12 minutes 15s after step 6.
	10	SS -> MS	IMMEDIATE ASSIGNMENT	
	11	MS -> SS	LOCATION UPDATING REQ
				"location updating type": periodic updating
	12	SS -> MS	LOCATION UPDATING ACC	
	13	SS -> MS	CHANNEL RELEASE
				After the sending of this message, 
				the SS waits for the disconnection 
				of the main signalling link.
	14	SS		3 minutes after step 13, cell a is switched off
	15	SS		14 minutes after step 13, cell a is switched on
				and cell b is switched off.
	16	MS -> SS	CHANNEL REQUEST	This message shall
				be sent by the MS before 17 minutes
				after step 13.
	17	SS -> MS	IMMEDIATE ASSIGNMENT	
	18	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	19	SS -> MS	LOCATION UPDATING ACC	
	22	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

### 26.7.5	MM connection

#### 26.7.5.1	Introduction

[tbd]

#### 26.7.5.2	MM connection / establishment with cipher

##### 26.7.5.2.1	Conformance requirement

1)	The Mobile Station shall be able to correctly set up an MM connection in a Mobile Originating CM connection attemptand send a CM Service Request message with CKSN information element as stored in the SIM and Mobile Identity information element set to the TMSI

2)	The Mobile Station shall be able to interpret cipher mode setting as acceptance of its CM service request i.e. send a CM message.

References

GSM 04.08 sections 4.5.1.1.

##### 26.7.5.2.2	Test purpose

To verify that the MS can correctly set up an MM connection in an origination and interpret cipher mode setting as acceptance of its CM service request.

##### 26.7.5.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has valid TMSI, CKSN. It is "idle updated".

Test Procedure

A mobile originating CM connection is initiated. After the MS has sent the CM SERVICE REQUEST message to the SS, an authentication procedure and a ciphering mode setting procedure are performed. Then, the MS sends a CM message and the SS clears the call and releases the channel.

Maximum duration of test

One minute

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM onnection is attempted
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST	
	 5	SS -> MS	AUTHENT REQUEST	
	 6	MS -> SS	AUTHENT RESPONSE	
	 7	SS -> MS	CIPH MODE COMMAND	The SS starts deciphering.
	 8	MS -> SS	CIPH MODE COMPLETE	The SS starts enciphering.

A9	MS -> SS	SETUP
	A10	SS -> MS	RELEASE COMPLETE	"Cause" IE : "unassigned
				number".

B9	MS -> SS	REGISTER
	B10	SS -> MS	RELEASE COMPLETE

C9	MS -> SS	CP-DATA
	C10	SS -> MS	CP-ACK
	C11	SS -> MS	CP-DATA
	C12	MS -> SS	CP-ACK

13	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.3	MM connection / establishment without cipher

##### 26.7.5.3.1	Conformance requirement

Upon reception of the CM SERVICE ACCEPT message, the MS shall send a CM message.

References

GSM 04.08 sections 4.5.1.1.

##### 26.7.5.3.2	Test purpose

To verify that the MS can correctly set up an MM connection in an originating CM connection establishment when ciphering mode setting is not required.

##### 26.7.5.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. The MM-connection is established without invoking the ciphering mode setting procedure.

Then, the MS sends a CM message and the SS releases the channel.

Maximum duration of test

one minute

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST	
	 5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	SETUP

B6	MS -> SS	REGISTER

C6	MS -> SS	CP-DATA
	C7	SS -> MS	CP-ACK
	C8	SS -> MS	CP-DATA
	C9	MS -> SS	CP-ACK

10	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.4	MM connection / establishment rejected

##### 26.7.5.4.1	Conformance requirement

Upon reception of a CM SERVICE REJECT message, the MS shall not send any layer 3 message, start timer T3240 and enter the "wait for network command" state.

References

GSM 04.08 sections 4.5.1.1.

##### 26.7.5.4.2	Test purpose

To verify that the MS does not send a layer 3 message when the service request is rejected by the SS.

##### 26.7.5.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI; It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS responds with a CM SERVICE REJECT message with reject cause "requested service option not subscribed". It is checked that the MS does not send a layer 3 message.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attemped
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST	
	 5	SS -> MS	CM SERVICE REJECT	"Reject cause" IE : "requested
				service option not subscribed".
	 6	SS		The MS shall not send a layer 3
				message. This is checked during
				5 seconds.
	 7	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.5	MM connection / establishment rejected cause 4

##### 26.7.5.5.1	Conformance requirement

1)	The Mobile Station shall be able to correctly set up an MM connection in a Mobile Originating CM connection attempt and send a CM Service Request message with CKSN information element as stored in the SIM and Mobile Identity information element set to the TMSI

2)	The Mobile Station, when receiving a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR" shall wait for the network to release the RR connection.

3)	The Mobile Station shall then be able to perform a location updating procedure.

References

GSM 04.08 sections 4.5.1.1.

##### 26.7.5.5.2	Test purpose

To verify that the MS can correctly accept a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR".

##### 26.7.5.5.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has valid TMSI, CKSN. It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS responds with a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR". On receipt of this message, the MS shall delete any TMSI, LAI, cipher key and cipher key sequence number. The channel is released. It is checked that the MS performs a normal location updating procedure.

Maximum duration of test

One minute

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST	
	 5	SS -> MS	CM SERVICE REJECT	"Reject cause"= "IMSI
				unknown in VLR"
	 6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	 7	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 8	SS -> MS	IMMEDIATE ASSIGNMENT	
	 9	MS -> SS	LOCATION UPDATING REQ	"Ciphering key sequence number"=
				"No key is available".
				"Mobile identity"= IMSI.
				"Location area identification"=
				deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE).
	10	SS -> MS	AUTHENT REQUEST	
	11	MS -> SS	AUTHENT RESPONSE
	12	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= new TMSI.
	13	MS -> SS	TMSI REALLOCATION CMP	
	14	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.6	MM connection / expiry T3230

[ editorial notes :

- no conformance requirement

]

##### 26.7.5.6.1	Conformance requirement

At T3230 expiry (i.e. no response is given but an RR connection is available) the MM connection establishment shall be aborted.

References

GSM 04.08 sections 4.5.1.2 and 11.2.

##### 26.7.5.6.2	Test purpose

To verify that at T3230 expiry, the MS aborts the MM-connection establishment.

##### 26.7.5.6.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS waits for expiry of timer T3230. It is checked that the MS does not send a layer 3 message but waits for the release of the RR-connection.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted.
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST	
	 5	   SS		The SS waits for expiry of
				timer T3230.
	 6	SS -> MS	CM SERVICE ACCEPT	
	 7	MS -> SS	MM-STATUS	"Reject cause " IE is "message
				not compatible with the call
				state or not implemented".
	 8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.7	MM connection / abortion by the network

##### 26.7.5.7.1	MM connection / abortion by the network / cause #6

###### 26.7.5.7.1.1	Conformance requirement

1)	Upon reception of an ABORT message, the MS shall release any ongoing MM connection and enter the "wait for network command" state.

2)	If the cause in the ABORT message was cause #6, the Mobile Station shall:

2.1	not perform normal location updating

2.2	not perform periodic location updating

2.3	not respond to paging with TMSI

2.4	reject any request for Mobile Originating call establishment except Emergency call

2.5	not perform IMSI detach if deactivated

3)	After reception of an ABORT message with cause #6, the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a Channel Request message with the establishment cause set to "emergency call".

4)	After reception of an ABORT message with cause #6, the Mobile Station shall delete the stored LAI, CKSN and TMSI.

Reference(s)

GSM 04.08 section 4.3.5

###### 26.7.5.7.1.2	Test purpose

To check that upon reception of an ABORT message with cause #6 during call establishment :

-	the MS does not send any layer 3 message.

-	after reception of an ABORT message and after having been deactivated and reactivated, the MS performs location updating using its IMSI as mobile identity and indicates deleted LAI and CKSN.

-	the MS does not perform location updating, does not answer to paging with TMSI, rejects any request for mobile originating call except emergency call, does not perform IMSI detach.

-	the MS accepts a request for emergency call.

###### 26.7.5.7.1.3	Method of test

Initial Conditions

System Simulator:

2 cells, default parameters.

Mobile Station:

The MS has a valid TMSI, CKSN and Kc. It is "idle updated" on cell B.

Related PICS/PIXIT Statement(s)

SIM removal possible without powering down Yes/No.

Switch off on button Yes/No

Support of speech Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell A.

Test procedure

A mobile originating CM connection is attempted. Upon reception of the AUTHENTICATION RESPONSE message, the SS sends an ABORT message with cause #6. The SS waits for 5 seconds. The MS shall not send any layer 3 message. The SS releases the RR connection.

The SS checks that the MS has entered the state MM-IDLE substate NO IMSI, i.e. does not perform normal location updating, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if deactivated.

Maximum Duration Of Test

10 minutes.

Expected Sequence

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	MS		A mobile originating CM connection is attempted
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST
	 5	SS -> MS	AUTHENT REQUEST
	 6	MS -> SS	AUTHENT RESPONSE
	 7	SS -> MS	ABORT	"reject cause" = #6
	 8	SS		The SS waits for 5 seconds
	 9	MS		The MS shall not send any layer 3 message
				during that time
	 10	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be received on cell A

11	   SS		The RF levels are changed
				to make the MS reselect cell A.
	 12	   MS		The MS performs cell reselection
				according to procedure as specified in GSM
				05.08 (this however is not checked 
				until step 22). The MS shall not initiate
				an RR connection establishment on
				cell A or on cell B.

13	   SS		The SS waits at least 7 minutes
				for a possible periodic
				updating.
	 14 	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

15	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				TMSI.
	 16	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is verified during 3 seconds.

17	   MS		A MO CM connection is attempted.
	 18	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds

19	   MS		If the MS supports speech (see PICS),
				an emergency call is attempted.
	 20	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Emergency call.
	 21	SS -> MS	IMMEDIATE ASSIGNMENT	
	 22	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency
				call establishment.
	 23	SS -> MS	CM SERVICE ACCEPT	
	 24	MS -> SS	EMERGENCY SETUP
	 25	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.
	 26	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

27	   MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	 28	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

29	MS		Depending on what has been 
				performed in step 29 the MS is 
				brought back to operation.
	 30	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 31	SS -> MS	IMMEDIATE ASSIGNMENT	
	 32	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"Mobile Identity"= IMSI,
				"LAI"= deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
	 33	SS -> MS	AUTHENT REQUEST	"CKSN"= CKSN1
	 34	MS -> SS	AUTHENT RESPONSE	
	 35	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity"= TMSI
	 36	MS -> SS	TMSI REALLOCATION CMP
	 37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.5.7.2	MM connection / abortion by the network / cause not equal to #6

###### 26.7.5.7.2.1	Conformance requirement

Upon reception of an ABORT message, the MS shall release any ongoing MM connection and enter the "wait for network command" state.

Reference(s)

GSM 04.08 section 4.3.5

###### 26.7.5.7.2.2	Test purpose

To check that when multiple MM connections are established, the MS releases all MM connections upon reception of an ABORT message, in the case when the two MM connections are established for a mobile terminating call and a non call related supplementary service operation.

###### 26.7.5.7.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in state U10 of a mobile terminating call

Related PICS/PIXIT Statement(s)

The MS supports a non call related supplementary service operation during an active call Yes/No.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated"

Test procedure

A non call related supplementary service operation is attempted at the MS. Upon reception of the REGISTER message, the SS sends an ABORT message with cause # 17. The SS sends a DISCONNECT using the TI of the mobile terminating call. The MS shall send a RELEASE COMPLETE message with the PD and TI of the DISCONNECT message and with cause #81. The SS releases the RR connection.

Maximum Duration Of Test

15 seconds.

Expected Sequence

This procedure is performed if the MS supports non call related supplementary service operation

Step	Direction	Message	Comments

1	MS		A non call related supplementary
				service operation is attempted
				at the MS
	 2	MS -> SS	CM SERVICE REQUEST
	 3	SS -> MS	CM SERVICE ACCEPT
	 4	MS -> SS	REGISTER
	 5	SS -> MS	ABORT	"reject cause" = #17
	 6	SS -> MS	DISCONNECT	with the TI of the
				mobile terminating call
	 7	MS -> SS	RELEASE COMPLETE	"cause"= #81. Same PD and TI
				as the DISCONNECT message
	 8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.8	MM connection / follow-on request pending

##### 26.7.5.8.1	MM connection / follow-on request pending / test 1

###### 26.7.5.8.1.1	Conformance requirement

The MS shall not attempt to establish a new MM connection after location updating on the same RR connection if not allowed by the network.

Reference(s)

GSM 04.08 section 4.4.4.6

###### 26.7.5.8.1.2	Test purpose

To check that when the network does not include the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has a CM application request pending does not attempt to establish a new MM connection on that RR connection.

###### 26.7.5.8.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS has a valid TMSI and is deactivated.

Related PICS/PIXIT Statement(s)

none

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated and a CM connection is immediately attempted. The MS performs location updating. The SS does not include the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for at least 8 seconds. The MS shall not send any layer 3 message for 8 seconds.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated and CM connection 
				is immediately attempted.
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATING REQ
	5	SS -> MS	LOCATION UPDATING ACC	folllow on proceed IE not included
	6	SS		The SS wait for at least 8 seconds
	7	MS		The MS shall not send any layer 3
				message for 8 seconds after reception
				of the LOCATION UPDATING ACCEPT
				message.
	8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.5.8.2	MM connection / follow-on request pending / test 2

###### 26.7.5.8.2.1	Conformance requirement

A MS supporting the follow-on request procedure and having a CM connection request pending shall correctly establish an MM connection following a location update when allowed by the network.

Reference(s)

GSM 04.08 section 4.4.4.6

###### 26.7.5.8.2.2	Test purpose

To check that when the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that supports the follow on request procedure and that has a CM application request pending establishes successfully a new MM connection on that RR connection.

###### 26.7.5.8.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS has a valid TMSI and is deactivated.

Related PICS/PIXIT Statement(s)

MS supports the follow on request procedure   Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated and a CM connection is immediately attempted. The MS performs location updating. The SS includes the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for at least 8 seconds.

If the MS supports the follow on request procedure :

The MS shall send a CM SERVICE REQUEST. Upon reception of that message, the SS sends a CM SERVICE ACCEPT message. The MS shall send an initial CM message. Upon reception of that message, the SS releases the RR connection.

If the MS does not support the follow on request procedure :

The MS shall not send any layer 3 message for 8 seconds.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated and an CM connection
				is immediately attempted.
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT
	 4	MS -> SS	LOCATION UPDATING REQ
	 5	SS -> MS	LOCATION UPDATING ACC	folllow on proceed IE included

If the MS supports the follow on
				request procedure (see PICS) steps A6 to A8
				are performed, otherwise steps B6 to B7
				are performed

A6	MS -> SS	CM-SERVICE REQUEST
	A7	SS ->MS	CM-SERVICE ACCEPT
	A8	MS -> SS	An initial CM message

B6	SS		The SS wait for at least 8 seconds
	B7	MS		The MS shall not send any layer 3
				message for 8 seconds after reception
				of the LOCATION UPDATING ACCEPT
				message.

9	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

none

##### 26.7.5.8.3	MM connection / follow-on request pending / test 3

###### 26.7.5.8.3.1	Conformance requirement

1)	The MS shall not set the follow on request bit in a LOCATION UPDATING REQUEST message if no MM connection request is pending.

2)	When the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has no CM application request pending shall not attempt to establish a new MM connection on that RR connection.

3)	The MS shall correctly handle a CM connection established by the network on the RR connection that was used for the location updating procedure.

Reference(s)

GSM 04.08 section 4.4.4.6

###### 26.7.5.8.3.2	Test purpose

1)	To check that a MS that has no CM application request pending sets the Follow-On-Request bit to No follow-on request pending in a LOCATION UPDATING REQUEST message.

2)	To check that when the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has no CM application request pending does not attempt to establish a new MM connection on that RR connection.

3)	To check that the MS accepts establishment by the network of a new MM connection on the existing RR connection.

###### 26.7.5.8.3.3	Method of test

Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS has a valid TMSI and is deactivated.

Related PICS/PIXIT Statement(s)

Supported services on TCH

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated. The MS performs location updating. The MS shall set the FOR bit to No follow-on request pending in the LOCATION UPDATING REQUEST message. The SS includes the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for 5 seconds. The MS shall not send any layer 3 message for 5 seconds. The SS sends a SETUP message to the MS requesting a basic service supported by the MS. The MS shall send either a CALL CONFIRMED message if it supports a service on TCH or a RELEASE COMPLETE with cause #88.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATING REQ	"Location updating type" : the FOR bit
				is set to No follow-on request pending
	5	SS -> MS	LOCATION UPDATING ACC	follow on proceed IE is included
	6	   SS		The SS wait for 5 seconds
	7	   MS		The MS shall not send any layer 3
				message for 5 seconds after reception
				of the LOCATION UPDATING ACCEPT
				message.
	8	SS -> MS	SETUP

If the MS supports a basic service on TCH
	 A9	MS -> SS	CALL CONFIRMED

If the MS does not support any basic
				service on TCH
	B9	MS -> SS	RELEASE COMPLETE	cause #88

10	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

none

### 26.7.6	Default contents of messages

Default contents SYSTEM INFORMATION messages and default settings

For cell A and B 		For GSM use 26.6.14

For DCS use 26.6.15

Cell C

The contents of SYSTEM INFORMATION TYPE 1 to 6 messages for cell C are identical to those of cell A with the following exceptions:

Cell Channel Description

-	Format Identifier			Bit map 0 for GSM

Range 512 for DCS

-	Cell Allocation ARFCN			Channel Number 30 for GSM,

Channel Number 700 for DCS,

Cell Identity

-	Cell Identity Value			0003H

Default settings for cell C:

Downlink input level				53 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power

class

Propagation profile				static

BCCH/CCCH carrier number			30 for GSM

700 for DCS

ABORT

Information element					Value/remark
Reject cause						Depending on the test one of either :

#6 	- Illegal ME

#17 	- Network Failure.

AUTHENTICATION REQUEST

Information element					Value/remark
Cipher Key Sequence Number				Arbitrary

Authentication parameter RAND 				Arbitrarily chosen by the test house

AUTHENTICATION RESPONSE

Information element	Value/remark
Authentication parameter SRES	As applicable

AUTHENTICATION REJECT

Information element	Value/remark
None but message head

CHANNEL RELEASE

Information element	Value/remark
RR cause	Normal release

CIPHERING MODE COMMAND

Information element	Value/remark
Cipher mode setting	Start ciphering
Cipher Response	IMEI must not be included

CM RE-ESTABLISHMENT REQUEST

Information element					Value/remark

Cipher Key Sequence Number				According to SIM contents

Mobile station classmark 2					See PICS/PIXIT
Mobile Identity						IMSI of MS under test

Location area identification					As in section 26.1.1

CM SERVICE ACCEPT

Information element	Value/remark
None but message head	Omitted

CM SERVICE REQUEST

Information element	Value/remark

CM service type	Mobile originating call establishment
	unless otherwise specified in test.

Ciphering key sequence number	According to SIM contents
Mobile station classmark 2	See PICS/PIXIT
Mobile identity	TMSI of the MS under test

CM SERVICE REJECT

Information element	Value/remark
Reject cause	Depending on test

IDENTITY REQUEST

Information element	Value/remark
Identity type	Depending on test
Spare half octet	0000

IDENTITY RESPONSE

Information element	Value/remark
Mobile identity	Depending on test

IMMEDIATE ASSIGNMENT

Information element	Value/remark
L2 pseudo length
Page mode	Normal Paging
Spare half octet	0000
Channel description
- Channel type and TDMA offset	SDCCH/4 or SDCCH/8
- Time slot number	Arbitrary legal value
- Subsequent fields of the Channel description IE depend upn the Type of MS under test (either DCS1800 or GSM), as specified in section 26.1.1
Request reference
- Random access information	As received from MS
- N51,N32,N26	Corresponding to the frame in which the Channel Request was sent
Timing advance	0
Mobile allocation	Empty (L=0)
Starting time	Omitted
IA rest octets	all bits set to spare

IMSI DETACH INDICATION

Information element	Value/remark
Mobile station classmark 1	See PICS/PIXIT
Mobile identity	TMSI of the MS under test

LOCATION UPDATING ACCEPT

Information element	Value/remark
Location area identification	As in section 26.1.1
Mobile identity	Omitted
Follow on proceed	Omitted

LOCATION UPDATING REJECT

Information element					Value/remark
Reject cause						As specified in test

LOCATION UPDATING REQUEST

Information element					Value/remark
Location updating type					Normal location updating

Cipher Key Sequence Number				According to SIM contents

Location area identification					As in section 26.1.1
Mobile station classmark					See PICS/PIXIT

Mobile identity						TMSI of the MS

PAGING REQUEST TYPE 1

Information element	Value/remark
L2 pseudo length
Page Mode	Normal paging
Channels needed
- mobile 1	"any channel"
- mobile 2	spare
Mobile identity 1	TMSI of MS under test
Mobile identity 2	Omitted
P1 rest octets	All bits set to spare

PAGING RESPONSE

Information element	Value/remark
Ciphering key sequence number	According to SIM contents
Spare half octet	0000
Mobile station classmark 2	See PICS/PIXIT
Mobile identity	TMSI of the MS under test

TMSI REALLOCATION COMMAND

Information element	Value/remark
Location area identification	As in section 26.1.1
Mobile identity	TMSI of the MS under test

TMSI REALLOCATION COMPLETE

Information element	Value/remark
None but message head	omitted