## 26.9	Structured procedures

Abbreviations for message names used in this section:

AUTHENTICATION REQ	=	AUTHENTICATION REQUEST

AUTHENTICATION RESP	=	AUTHENTICATION RESPONSE

CIPHER MODE COMMAND	=	CIPHERING MODE COMMAND

CIPHER MODE COMPLETE	=	CIPHERING MODE COMPLETE

### 26.9.1	Structured procedures / general

The purpose of these tests is to verify that the MS performs certain elementary procedures of the RR, MM, and CC protocol correctly within a structured procedure. The term "structured procedure" is defined in TS GSM 04.08, section 7, where also examples of structured procedures are given.

The reason for this test purposes is twofold:

-	The behaviour of the MS in an elementary procedure may depend on the preamble which precedes the elementary procedure.

-	Structured procedures tested in this section are used in other parts of this Technical Specification as preambles to establish the initial conditions for other tests; correct behaviour of an implementation under test in a preamble is essential for the validity of a test.

Mobile originating and terminating calls are tested in cases of both early and late assignment of the traffic channel; in one of the cases call release initiated by the network is tested, in another one, call release initiated by the MS. The tests in this section only cover the successful outcome of elementary procedures (i.e. they do not deal with abnormal cases).

The emergency call service is tested in the following cases:

-	emergency call initiated in the idle, updated state with authentication and ciphering, for all supported speech rates,

-	emergency call initiated in the idle, no IMSI state (hence without authentication and without ciphering), the network accepting the call, for one supported speech rates,

-	emergency call initiated in the idle, no IMSI state (hence without authentication and without ciphering), the network rejecting the call, for one supported speech rate.

These tests on emergency calls are only applicable to an MS supporting speech.

For an MS supporting speech the test procedures in 26.9.2, 26.9.3, 26.9.4 and 26.9.5 are performed for speech (teleservice 11, telephony), once for each supported rate (full rate/half rate).

For an MS not supporting speech but supporting at least one teleservice, for each of the test procedures in 26.9.2, 26.9.3, 26.9.4, and 26.9.5 and each supported rate (full rate/half rate) a teleservice supported by the MS (see PICS/PIXIT statement) is chosen, and the test is performed corresponding to that teleservice (note that this teleservice is never a dual service).

In cases where a mobile originated call for the tested teleservice can be initiated both

-	via the MMI and

-	via the R or S interface,

procedure 26.9.2 shall be performed when initiating the mobile originated call via the MMI and procedure 26.9.3 shall be performed when initiating the mobile originated call via an appropriate interface (R or S).

### 26.9.2	Structured procedures / MS originated call / early assignment

#### 26.9.2.1	Conformance requirements

1)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call, if it provides a human interface, shall display the dialled number.

2)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call for a selected teleservice that is supported by the MS, shall start to initiate the immediate assignment procedure by sending a CHANNEL REQUEST message with correct establishment cause.

3)	Subsequently after establishment of an MM connection, the MS shall send a SETUP message with correct parameters.

4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

5)	Subsequently when the network initiates call clearing by sending a DISCONNECT message, the MS shall proceed to release the call by sending a RELEASE message.

6)	On receipt of a CHANNEL RELEASE message, the MS shall disconnect the main signalling link.

References

Conformance requirement 1:	TS GSM 02.07.

Conformance requirement 2:	TS GSM 04.08 section 3.3.1.

Conformance requirement 3:	TS GSM 04.08 section 5.2.1.1.

Conformance requirement 4:	TS GSM 04.08 section 5.2.1.6.

Conformance requirement 5:	TS GSM 04.08 section 5.4.4.

Conformance requirement 6:	TS GSM 04.08, section 3.5.1.

#### 26.9.2.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, displays the dialled number in the way described in a PICS/PIXIT statement.

2)	To verify that the MS in MM state "idle, updated" and in RR idle mode, with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

3)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed the authentication and cipher mode setting procedures, the MS sends a SETUP message with correct parameters.

4)	To verify that subsequently, after receipt of a CALL PROCEEDING message and of an ASSIGNMENT COMMAND message allocating an appropriate TCH, after having completed the traffic channel early assignment procedure by replying with the ASSIGNMENT COMPLETE message, after receipt of an ALERTING message and a CONNECT message, the MS returns a CONNECT ACKNOWLEDGE message.

5)	To verify that subsequently the MS has attached the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

6)	To verify that subsequently upon the network initiating call clearing by sending a DISCONNECT message, the MS proceed to release the call with RELEASE.

7)	To verify that subsequently, on receipt of a RELEASE COMPLETE message followed by a CHANNEL RELEASE message, the MS disconnects the main signalling link.

These test purposes are tested for all rates supported by the MS (full rate/half rate).

#### 26.9.2.3	Method of test

Related PICS/PIXIT Statements

- 	Supported rates (full rate/half rate)

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	SS version

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" is entered
  	2	MS		If p1 = Y, the MS must display the
				called number in the way defined in
				PICS/PIXIT.
	  3	MS -> SS	CHANNEL REQUEST	Establishment cause is "originating
				call and the network does not
				set the NECI bit to 1".
	  4	SS -> MS	IMMEDIATE ASSIGNMENT	
	  5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.
	  6	SS -> MS	AUTHENTICATION REQ
	  7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.
				All following messages
				shall be sent enciphered.
	 10	SS		SS starts ciphering.
	 11	MS -> SS	SETUP
	 12	SS -> MS	CALL PROCEEDING
			
				
				
	 13	SS -> MS	ASSIGNMENT COMMAND
	 14	MS -> SS	ASSIGNMENT COMPLETE
	15	SS -> MS	ALERTING
	16	MS		Depending on the PICS, an alerting indication
				is given
	 17	SS -> MS	CONNECT
	 18	MS -> SS	CONNECT ACKNOWLEDGE
	 19	MS		The appropriate bearer channel is
				through connected in both directions.
	 20	SS -> MS	DISCONNECT
	 21	MS -> SS	RELEASE
	 22	SS -> MS	RELEASE COMPLETE
	 23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

### 26.9.3	Structured procedures / MS originated call / late assignment

#### 26.9.3.1	Conformance requirement

1)	An MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS , shall start to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

2)	Upon receipt of the ASSIGNMENT COMMAND message, the Mobile Station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the data links). After the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message, specifying cause 'normal event', to the network on the main DCCH.

3,4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

References

Conformance requirement 1:	TS GSM 04.08 section 3.3.1.

Conformance requirement 2:	TS GSM 04.08 section 3.4.3.1, 3.4.3.2

Conformance requirement 3:	TS GSM 04.08 section 5.2.1.6.

#### 26.9.3.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message.

2)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed authentication and cipher mode setting procedures, after having sent a SETUP message, after having received a CALL PROCEEDING message followed by an ALERTING message and an ASSIGNMENT COMMAND message allocating an appropriate TCH, the MS sends an ASSIGNMENT COMPLETE message.

3)	To verify that subsequently, after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message returns a CONNECT ACKNOWLEDGE message.

4)	To verify that after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message attaches the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

These test purposes are tested for all rates supported by the MS (full rate/half rate).

#### 26.9.3.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS has a MO call in state U10, "active".

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with late assignment.

Maximum Duration of Test

30 seconds

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" is entered
	  2	MS		
	  3	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"originating call and the
				network does not set the NECI bit to 1".
	  4	SS -> MS	IMMEDIATE ASSIGNMENT	
	  5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.
	  6	SS -> MS	AUTHENTICATION REQ
	  7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 10	SS		SS starts ciphering.
	 11	MS -> SS	SETUP
	 12	SS -> MS	CALL PROCEEDING
	 13	SS -> MS	ALERTING
	 14	MS		Depending on the PICS, an alerting
				indication is given.
	 15	SS -> MS	ASSIGNMENT COMMAND
	 16	MS -> SS	ASSIGNMENT COMPLETE
	 17	SS -> MS	CONNECT
	 18	MS -> SS	CONNECT ACKNOWLEDGE
	 19	MS		The appropriate bearer channel is
				through connected in both directions.

Specific Message Contents:

None

### 26.9.4	Structured procedures / MS terminated call / early assignment

#### 26.9.4.1	Conformance requirements

1)	The MS shall acknowledge the SETUP message with a CALL CONFIRMED message, if compatibility checking was successful, the MS is not busy, and the user does not refuse the call.

2,3)	The MS on acceptance of the call sends a CONNECT, otherwise user alerting is initiated (the MS sends an ALERTING message and generates an alerting indication). ASSIGNMENT COMMAND is answered by ASSIGNMENT COMPLETE.

4)	An MS indicates acceptance of a MT call by sending CONNECT.

5)

For speech calls:

The mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment shall be delayed until such a resource becomes available.

For data calls:

The mobile station shall attach the user connection when receiving the CONNECT ACKNOWLEDGE message from the network.

6)	The MS initiates call clearing of an active call by sending a DISCONNECT message.

7)	The MS in this phase of call release, upon receipt of a RELEASE message, shall return a RELEASE COMPLETE message.

8)	Subsequently the MS, upon receipt of a CHANNEL RELEASE message, shall disconnect the main signalling link.

References

Conformance requirement 1:		TS GSM 04.08, section 5.2.2.3.1.

Conformance requirements 2,3:	TS GSM 04.08, section 5.2.2.3.2, 3.4.3.1.

Conformance requirement 4:		TS GSM 04.08 section 5.2.2.5.

Conformance requirement 5:		TS GSM 04.08, section 5.2.2.6 and 5.2.2.9.

Conformance requirements 6,7,8:	TS GSM 04.08, section 5.4.

#### 26.9.4.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, after being paged by the network on the correct paging subchannel, after initiating the immediate assignment procedure by sending the CHANNEL REQUEST message, after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after having sent a PAGING RESPONSE message on the allocated SDCCH, after having performed successful authentication and cipher mode setting procedures, after receipt of a SETUP message not containing a signal information element, returns a CALL CONFIRMED message.

2)	To verify that subsequently, the SS sending an ASSIGNMENT COMMAND message, the MS successfully continues a mobile terminating call establishment with early assignment of traffic channel

a)	by replying to the ASSIGNMENT command with an ASSIGNMENT COMPLETE message, and

b)	by continuing the call establishment by either sending one or two CONNECT messages (with equal N(SD)) or sending an ALERTING message, steps a) and b) being performed in any permitted interleaving.

3)	To verify that if after sending a CALL PROCEEDING message, the MS sends an ALERTING message during MTC establishment with early assignment, it generates an alerting indication.

4)	To verify that if an ALERTING had been sent, subsequently, when the user accepts the call (possibly internal action as declared in PICS/PIXIT statement), the MS returns a CONNECT message.

5)	To verify that the MS:

-	if the call is a speech call: after sending the CONNECT message has through connected the TCH in both directions (this is checked by verifying that after transmission of the first L2 frame containing the (complete) CONNECT message, the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

-	if the call is a data call: after receipt of a subsequent CONNECT ACKNOWLEDGE message through connects the TCH in both directions (this is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT ACKNOWLEDGE message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

6)	To verify that subsequently, the MS can initiate call clearing by sending a DISCONNECT message.

7)	To verify that the MS in this phase of call release, upon receipt of a RELEASE message, returns a RELEASE COMPLETE message.

8)	To verify that subsequently the MS, upon receipt of a CHANNEL RELEASE message, disconnects the main signalling link.

These test purposes are tested for all rates supported by the MS (full rate/half rate).

#### 26.9.4.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Immediate connect supported (Y/N)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged and the resulting call is established. Having reached the active state, the MS is made to clear the call.

Maximum Duration of Test

1 minute

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging
				subchannel
	  2	MS -> SS	CHANNEL REQUEST	
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	
	  4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  5	SS -> MS	AUTHENTICATION REQ
	  6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	  9	SS		SS starts ciphering.
	 10	SS -> MS	SETUP	Message does not contain the 
				signal IE.
	 11	MS -> SS	CALL CONFIRMED	
	 12	SS -> MS	ASSIGNMENT COMMAND

A13	MS -> SS	ASSIGNMENT COMPLETE	During step 12 or 
				between step 12 and step 13
				the MS may send a CONNECT message
				on the old channel.
	A14	MS -> SS	CONNECT	If during step 12 or 
				between step 12 and step A13
				the MS has send a CONNECT message
				on the old channel, both CONNECT
				messages must have equal N(SD).

B13	MS -> SS	ASSIGNMENT COMPLETE	Sent on the new channel.
	B14	MS -> SS	ALERTING	
	B15	MS		An alerting indication as defined 
				in an PICS/PIXIT statement 
				is given by the MS.
	B16	MS		The MS is made to accept the call
				in the way described in a
				PICS/PIXIT statement.
	B17	MS -> SS	CONNECT

18	MS		If the call is a speech call,
				the TCH shall be through connected 
				in both directions.
 19	SS -> MS	CONNECT ACKNOWLEDGE
	 20	MS		If the call is a data call, 
				the TCH shall be through
				connected in both
				directions.
	21	MS		The MS is made to release the call.
	22	MS -> SS	DISCONNECT
	23	SS -> MS	RELEASE
	24	MS -> SS	RELEASE COMPLETE
	25	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

### 26.9.5	Structured procedures / MS terminated call / late assignment

#### 26.9.5.1	Conformance requirement

TP1,TP2:	The MS shall acknowledge the SETUP message with a CALL CONFIRMED message, if compatibility checking was successful, the MS is not busy, and the user does not refuse the call. The MS on acceptance of the call sends a CONNECT, otherwise user alerting is initiated.

TP3:	The MS indicates acceptance of a call by sending a CONNECT message.

TP4:	ASSIGNMENT COMMAND is answered by ASSIGNMENT COMPLETE.

TP5:

For speech calls:

The mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment shall be delayed until such a resource becomes available.

For data calls:

The mobile station shall attach the user connection when receiving the CONNECT ACKNOWLEDGE message from the network.

Requirement reference:

Conformance requirements 1, 2, 3:		TS GSM 04.08, sections 5.2.2.3.1, 5.2.2.3.2, 5.2.2.5.

Conformance requirement 4:		TS GSM 04.08, section 3.4.3.1.

Conformance requirement 5:		TS GSM 04.08, section 5.2.2.9.

#### 26.9.5.2	Test purpose

1)	To verify that the MS in "Idle, Updated" state with a TMSI assigned, after being paged by the network on the correct paging subchannel, after initiating the immediate assignment procedure by sending the CHANNEL REQUEST message, after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after having established the main signalling link, after having sent a PAGING RESPONSE message, after having performed successful authentication and cipher mode setting procedures, after receipt of a SETUP message containing a signal information element, returns a CALL CONFIRMED message followed by

-	an ALERTING message

-	or a CONNECT message.

2)	To verify that in the situation of test purpose 1, if the MS sends an ALERTING message, the MS generates an alerting indication in the way described in a PICS/PIXIT statement.

3)	To verify that subsequently the MS, if it had not yet sent a CONNECT message, upon acceptance of the call, sends a CONNECT message.

4)	To verify that subsequently after receipt of an ASSIGNMENT COMMAND, the MS sends an ASSIGNMENT COMPLETE message.

5)	To verify that subsequently the MS

-	if the call is a speech call: after sending the ASSIGNMENT COMPLETE message has through connected the TCH in both directions (this is checked by verifying that after transmission of the first L2 frame containing the (complete) ASSIGNMENT COMPLETE message, the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

-	if the call is a data call: after receipt of a subsequent CONNECT ACKNOWLEDGE message through connects the TCH in both directions (this is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT ACKNOWLEDGE message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

These test purposes are tested for all rates supported by the MS (full rate/half rate).

#### 26.9.5.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Immediate connect supported (Y/N)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

CC state U10-call active.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged and a MT call is established with late assignment (after CONNECT).

Maximum Duration of Test

40 seconds

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging
				subchannel
	  2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates
				"answer to paging".
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	
	  4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  5	SS -> MS	AUTHENTICATION REQ
	  6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. 
				All following
				messages shall be sent enciphered.
	  9	SS		SS starts ciphering.
	 10	SS -> MS	SETUP	Message contains the signal IE.
	 11	MS -> SS	CALL CONFIRMED

A12	MS -> SS	CONNECT

B12	MS -> SS	ALERTING	
	B13	MS		An alerting indication as defined 
				in an PICS/PIXIT statement 
				is given by the MS.
	B14	MS		The MS is made to accept the call
				in the way described in a
				PICS/PIXIT statement.
	B15	MS -> SS	CONNECT

16	SS -> MS	ASSIGNMENT COMMAND	
	 17	MS -> SS	ASSIGNMENT COMPLETE	
	 18	MS		If the call is a speech call,
				the TCH shall be through connected
				in both directions.
 	19	SS -> MS	CONNECT ACKNOWLEDGE
	 20	MS		If the call is a data call, 
				the MS shall 
				through connect the TCH
				in both directions.

Specific Message Contents:

None

### 26.9.6	Structured procedures / emergency call

Emergency call establishment can be initiated by an MS whether location updating has been successful or not and whether a SIM is inserted into the MS or not; but only if the MS is equipped for speech.

If the procedures tested in this section are not correctly implemented in the MS, establishment, maintenance and clearing of connections might fail in the essential case of emergency calls.

The tests of this section are only applicable to an MS supporting speech.

#### 26.9.6.1	Structured procedures / emergency call / idle updated

##### 26.9.6.1.1	Structured procedures / emergency call / idle updated / preferred channel rate

26.9.6.1.1.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct CKSN and TMSI , with CM Service Type "emergency call establishment".

3)	Authentication and cipher mode setting shall be performed successfully.

4)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

5),6)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

7)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

8)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 3.4.7, TS GSM 04.08 section 4.3.2.

For conformance requirement 4:

TS GSM 04.08, section 5.2.1.1.

For conformance requirement 5 and 6:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 7:

TS GSM 04.08, sections 5.2.1.6 and 5.1.3.

For conformance requirement 8:

TS GSM 04.08, section 5.4.

26.9.6.1.1.2	Test purpose

1)	To verify that an MS supporting speech in the MM state "idle, updated", when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message specifying the correct CKSN and TMSI, with CM Service Type "emergency call establishment" .

3)	To verify that authentication and cipher mode setting are performed successfully.

4)	To verify that after cipher mode setting acceptance by the SS, the MS sends an EMERGENCY SETUP message.

5)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure of an appropriate speech traffic channel, which, if the MS supports both TCH/FS and TCH/HS, is at the preferred rate, the MS performs correctly that assignment procedure.

6)	To verify subsequent correct performance of a connect procedure.

7)	To verify that subsequently the MS has through connected the TCH in both directions.

8)	To verify that the call is cleared correctly.

26.9.6.1.1.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Supported rate for speech: (p1 = F/H, F).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The MS is made to initiate an emergency call. The call is established with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	  3	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				emergency call establishment.
	  4	SS -> MS	IMMEDIATE ASSIGNMENT	
	  5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
	  6	SS -> MS	AUTHENTICATION REQ
	  7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 10	SS		SS starts ciphering.
	 11	MS -> SS	EMERGENCY SETUP	If p1 = F/H, the message must contain
				one bearer capability IE indicating 
				in the radio channel requirement field
				"dual rate/half rate preferred" or 
				"dual rate/full rate preferred". 

				If p1 = F, the message must either
				contain no bearer capability IE or
				contain one bearer capability IE 
				indicating in the 
				radio channel requirement field 
				"full rate channel".
	 12	SS -> MS	CALL PROCEEDING
	 13	SS -> MS	ALERTING
	 14	SS -> MS	ASSIGNMENT COMMAND	The rate of the channel is that one
				indicated by the EMERGENCY SETUP
				message, if that message did not 
				offer a choice, and the rate is the
				preferred one else.
	 15	MS -> SS	ASSIGNMENT COMPLETE
	 16	SS -> MS	CONNECT
	 17	MS -> SS	CONNECT ACKNOWLEDGE
	 18	MS		The TCH is
				through connected in both directions.
	 19	SS -> MS	DISCONNECT
	 20	MS -> SS	RELEASE
	 21	SS -> MS	RELEASE COMPLETE
	 23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

##### 26.9.6.1.2	Structured procedures / emergency call / idle updated, non-preferred channel rate

The test is performed if the MS supports both TCH/HS and TCH/FS see PICS/PIXIT statement).

It is identical to the test in 26.9.6.1.1 except that in step 14 the assigned TCH has the non-preferred rate.

#### 26.9.6.2	Structured procedures / emergency call / idle, no IMSI

##### 26.9.6.2.1	Structured procedures / emergency call / idle, no IMSI / accept case

26.9.6.2.1.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

4),5)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

6)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

7)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 5.2.1.1.

For conformance requirements 4 and 5:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 6:

TS GSM 04.08, sections 5.2.1.6 and 5.1.3.

For conformance requirement 7:

TS GSM 04.08, section 5.4.

26.9.6.2.1.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message in which the cipher key sequence number IE indicates "no key is available", the CM service type IE indicates "emergency number establishment", and the mobile identity IE specifies the IMEI of the MS.

3)	To verify that after receipt of a CM SERVICE ACCEPT message from the SS, the MS sends an EMERGENCY SETUP message.

4)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure of an appropriate speech traffic channel, which, if the MS supports both TCH/FS and TCH/HS, is at the preferred rate, the MS performs correctly that assignment procedure.

5)	To verify subsequent correct performance of a connect procedure.

6)	To verify that subsequently the MS has through connected the TCH in both directions.

7)	To verify that the call is cleared correctly.

26.9.6.2.1.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	  3	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"emergency call".
	  4	SS -> MS	IMMEDIATE ASSIGNMENT	
	  5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
				The mobile identity IE specifies the
				IMEI of the MS.
				The cipher key sequence number IE
				indicates "no key is available".
				The mobile station classmark IE is as
				specified by the manufacturer in a
				PICS/PIXIT statement.
	  4	SS -> MS	CM SERVICE ACCEPT	
	 11	MS -> SS	EMERGENCY SETUP	
	 12	SS -> MS	CALL PROCEEDING
	 13	SS -> MS	ALERTING
	 14	SS -> MS	ASSIGNMENT COMMAND	The rate of the channel is one
				indicated by the EMERGENCY SETUP
				message.
	 15	MS -> SS	ASSIGNMENT COMPLETE
	 16	SS -> MS	CONNECT
	 17	MS -> SS	CONNECT ACKNOWLEDGE
	 18	MS		The TCH is through connected in both
				directions.
	 19	SS -> MS	DISCONNECT
	 20	MS -> SS	RELEASE
	 21	SS -> MS	RELEASE COMPLETE
	 23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

##### 26.9.6.2.2	Structured procedures / emergency call / idle, no IMSI / reject case

26.9.6.2.2.1	Conformance requirement

1)	The MS in the "idle, no IMSI" state (no SIM inserted), after the number 112 has been entered, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	In the situation at the end of test purpose 2, when the MS receives a CM SERVICE REJECT message, it shall abandon the emergency call.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4

For conformance requirement 3:

TS GSM 04.08 section 3.4.7, TS GSM 04.08 section 4.5.1.1.

26.9.6.2.2.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message in which the cipher key sequence number IE indicates "no key is available", the CM service type IE indicates "emergency call establishment", and the mobile identity IE specifies the IMEI of the MS.

3)	To verify that after receipt of a CM SERVICE REJECT message from the SS, the MS abandons the emergency call establishment.

26.9.6.2.2.3	Method of test

Related PICS/PIXIT statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. The SS responds to the CM SERVICE REQUEST from the MS with a CM SERVICE REJECT message specifying in the reject cause IE the reject cause value "IMEI not accepted". The SS then verifies for during 5 seconds that the MS does not send a layer 3 message. Then the call is cleared by the SS. The SS verifies during 20 seconds after disconnection of the main signalling link that the MS does not initiate an RR connection establishment.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	  3	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"emergency call".
	  4	SS -> MS	IMMEDIATE ASSIGNMENT	
	  5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
				The mobile identity IE specifies the
				IMEI of the MS.
				The cipher key sequence number IE
				indicates "no key is available".
				The mobile station classmark IE is as
				specified by the manufacturer in a
				PICS/PIXIT statement.
	  4	SS -> MS	CM SERVICE REJECT	the reject cause IE specifies 
				reject cause value #5, 
				"IMEI not accepted".
	  5	SS		During 5 seconds, the SS verifies
				that the MS does not send L3 messages.
	  6	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	  7	SS		During 20 seconds, the SS verifies
				that the MS does not initiate an 
				RR connection establishment.

Specific Message Contents:

-

### 26.9.7	Default contents of messages

ALERTING (mobile station to network direction)

Information element			Value/remark

Facility					Omitted

User-user 				Omitted

SS version				Omitted

ALERTING (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

User-user 				Omitted

ASSIGNMENT COMMAND

Information element			Value/remark

Description of the first channel		describes non-hopping Bm+ACCHs or Lm+ACCHs as

appropriate for the test

Power Command				As in section 26.1.1

Frequency list				Omitted

Cell channel description			Omitted

Mode of the first channel 			appropriate for on bearer capability chosen for the test

Description of the second channel		Omitted

Mode of the second channel			Omitted

Mobile allocation				Omitted

Starting time				Omitted

Cipher mode setting			Omitted

ASSIGNMENT COMPLETE

Information element			Value/remark

RR cause				normal event

AUTHENTICATION REQUEST

Information element			Value/remark

Ciphering key sequence number		Arbitrary

Spare half octet				(spare bits)

Authentication parameter RAND		Arbitrary

AUTHENTICATION RESPONSE

Information element			Value/remark

Authentication parameter SRES		Correct for given SRES

CALL CONFIRMED

Information element			Value/remark

Repeat indicator				Omitted

Bearer capability 1 			Omitted if the MS only supports TCH/F. Included if the

MS supports both TCH/F and TCH/H; in that case, the

"radio channel requirement" field specifies "dual

rate/half rate preferred" or "dual rate/full rate

preferred" or "half rate channel", all other parameters

are same as in the bearer capability 1 IE of the

received SETUP message.

Bearer capability 2			Omitted

Cause					Omitted

CALL PROCEEDING

Information element			Value/remark

Repeat Indicator				Omitted

Bearer Capability 1			Omitted if the SETUP message did not specify in the

bearer capability 1 IE a connection element value

"both, transparent preferred" or "both, non-

transparent preferred". Otherwise included; in that

case the connection element specifies the value that is

appropriate for the selected teleservice (either value

"transparent" or value "non transparent (RLP)"), all

other parameters are same as in the bearer capability

1 IE of the received SETUP message.

Bearer Capability 2			Omitted

Facility					Omitted

Progress indicator				Omitted

CHANNEL RELEASE

Information element			Value/remark

RR cause  				Normal event

CHANNEL REQUEST

Information element			Value/remark

Establishment cause 			Answer to paging (100)

Random reference				Arbitrary value of 5 bits length

CIPHERING MODE COMMAND

Information element			Value/remark

Cipher mode setting

algorithm identifier			indicates a supported algorithm

SC					Start ciphering

Cipher response

CR					IMEI must not be included

CIPHER MODE COMPLETE

Information element			Value/remark

Mobile equipment identity			Omitted

CM SERVICE ACCEPT

Information element			Value/remark

none but message head

CM SERVICE REQUEST

Information element			Value/remark

CM service type				Mobile originating call

establishment or packet mode

connection establishment

Ciphering key sequence number		CKSN of the MS

Mobile station classmark 2			as given by PICS.

Mobile identity				TMSI of MS

CONNECT (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

Connected number			Omitted

Connected subaddress			Omitted

User-user				Omitted

CONNECT (mobile station to network direction)

Information element			Value/remark

Facility					Omitted

Connected subaddress			Omitted

User-user				Omitted

SS version				Omitted

CONNECT ACKNOWLEDGE

Information element			Value/remark

none but message head

DISCONNECT (network to mobile station direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

Progress indicator				Omitted

User-user 				Omitted

DISCONNECT (mobile station to network direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

User-user 				Omitted

SS version				Omitted

IMMEDIATE ASSIGNMENT

Information element			Value/remark

Page mode 				Normal paging

Channel description			describes a valid SDCCH+SACCH

in non-hopping mode

Request reference

Random access information		As received from MS

N51, N32, N26			Corresponding to frame number

of the CHANNEL REQUEST

Timing advance				Arbitrary

Mobile allocation				Empty (L=0)

Starting time				Omitted

PAGING REQUEST TYPE 1

Information element			Value/remark

L2 pseudo length				L2 pseudo length of the message

Page Mode				Normal Paging

Channels needed for Mobiles 1 and 2

channel (first)			any channel

channel (second)			any channel

Mobile identity 1				TMSI of MS under test

Mobile identity 2				Omitted

P1 rest octets				(spare octets)

PAGING RESPONSE

Information element			Value/remark

Ciphering key sequence number		Value assigned to MS in the initial conditions

Spare half octet				(spare bits)

Mobile station classmark 2			as given by PICS

Mobile identity				specifies TMSI of MS

RELEASE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Second cause				Omitted

Facility					Omitted

User-user				Omitted

RELEASE (mobile station to network direction)

Information element			Value/remark

Cause					Omitted

Second cause				Omitted

Facility					Omitted

User-user				Omitted

SS version				Omitted

RELEASE COMPLETE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Facility					Omitted

User-user				Omitted

RELEASE COMPLETE (mobile station to network direction)

Information element			Value/remark

Cause					Omitted

Facility					Omitted

User-user				Omitted

SS version				Omitted

SETUP (MS to SS)

Information element			Value/remark

BC Repeat indicator			Omitted

Bearer capability 1				Appropriate for the teleservice selected for the test

Bearer capability 2				Omitted

Facility					Omitted

Calling party subaddress			Omitted

Called party BCD number			As entered

Called party subaddress			Omitted

LLC repeat indicator			Omitted

Low layer compatibility I			Appropriate for teleservice selected for the test

Low layer compatibility II			Omitted

HLC repeat indicator			Omitted

High layer compatibility i			Appropriate for teleservice selected for the test

High layer compatibility ii			Omitted

User-user				Omitted

SS version				Omitted

CLIR suppression				Omitted

SETUP (SS to MS)

Information element			Value/remark

BC repeat indicator			Omitted

Bearer capability 1				Appropriate for teleservice selected for the test

Bearer capability 2				Omitted

Facility					Omitted

Progress indicator				Omitted

Signal					Omitted

Calling party BCD number			Omitted

Calling party subaddress			Omitted

Called party BCD number			Omitted

Called party subaddress			Omitted

LLC repeat indicator			Appropriate for the teleservice selected for the test

Low layer compatibility I			Appropriate for teleservice selected for the test

Low layer compatibility II			Appropriate for the teleservice selected for the test

HLC repeat indicator			Appropriate for the teleservice selected for the test

High layer compatibility i			Appropriate for the teleservice selected for the test

High layer compatibility ii			Appropriate for the teleservice selected for the test

User-user				Omitted

## 26.10	E-GSM signalling

### 26.10.1	E-GSM signalling / general considerations

Section 26.10 only applies to E-GSM mobile stations.

Conformance requirements of section 26 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26 under the described GSM900 conditions.

The purpose of this extra section is to test the different procedures which may be impacted when some channel uses E-GSM frequencie(s).

### 26.10.2	E-GSM signalling / RR

#### 26.10.2.1	E-GSM signalling / RR / Measurement

This test applies E-GSM mobile stations.

Conformance requirements of section 26.6.3 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.3 under the described GSM900 conditions.

The purpose of this extra section is to test the reporting of mesurements in the case where cells use E-GSM frequencie(s). Several coding formats may be used by the network in the SYSTEM INFORMATION message.

26.10.2.1.1	Conformance requirements

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the values in the MEASUREMENT REPORT message shall contain measurement results for the 6 th strongest cells on which the mobile is asked to report.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.10.2.1.2	Test purpose

To test that, when the SS gives information about neighbouring cells, the MS reports appropriate results.

##### 26.10.2.1.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BSCC	ARFCN	Cell identity

Serving, S1	-60	1	3	002	0001H

Neighbour, N1	-85	1	5	990	0002H

Neighbour, N2	-80	1	7	1005	0003H

Neighbour, N3	-75	1	1	000	0004H

Neighbour, N4	-55	1	3	026	0005H

Neighbour, N5	-50	1	5	1020	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	1003	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed 6 times.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5, 5bis & 6  on the SACCH. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest present carriers of the supported band have been obtained.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, c= 1 to 6.

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT  are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Protocol Descriminator		RR management

Message Type			Sys Info 5.

Neighbour Cells Description

- EXT IND			Information Element carries only a part of the BA.

- BA-IND			0

for c=1, use range 128

to encode the following frequencies :

(26, 38)

for c=2, use range 256

to encode the following frequencies

(990, 1003, 1005)

for c=3, use range 512

to encode the following frequencies

(520, 990, 1003, 1005, 1020)

for c=4, use range 1024

to encode the following frequencies

(0, 26, 38, 990, 1003, 1005)

for c=5, use variable Bitmap

to encode the following frequencies

(0, 26, 38)

for c=6, use Bitmap 0

to encode the following frequencies

(26)

SYSTEM INFORMATION TYPE 5bis:

Information Element			value/remark

Protocol Descriminator		RR management

Message Type			Sys Info 5bis.

Neighbour Cells Description

- EXT IND			Information Element carries only a part of the BA.

- BA-IND			0

for c=1, use range 512

to encode the following frequencies :

(520, 990, 1003, 1005, 1020)

for c=2, use range 128

to encode the following frequencies

(0, 26, 38)

for c=3, use range 256

to encode the following frequencies

(0, 26, 38)

for c=4, use range 1024

to encode the following frequencies

(520, 1000, 1020)

for c=5, use variable Bitmap

to encode the following frequencies

(884, 990, 1003, 1005)

for c=6, use range 512

to encode the following frequencies

(520, 990, 1003)

SYSTEM INFORMATION TYPE 6:

Information Element			value/ remark

Protocol Discriminator		RR Management

Message Type			sys info 6

Cell Identity				default

LAI					default

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall not use DTX

- Radio-Link-Timeout	default

PLMN permitted			only NCC 1 permitted

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 1

RXLEV-SUB-SERVING-CELL	See note 1

MEAS-VALID			See note 3

RXQUAL-FULL-SERVING-CELL	See note 1

RXQUAL-SUB-SERVING-CELL	See note 1

NO-NCELL-M			n (see note 2)

RXLEV-NCELL-1		See note 1

BCCH-FREQ-NCELL-1		See note 2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 1

BCCH-FREQ-NCELL-2		See note 2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 1

BCCH-FREQ-NCELL-3		See note 2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 1

BCCH-FREQ-NCELL-4		See note 2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		See note 1

BCCH-FREQ-NCELL-5		See note 2

BSIC-NCELL-5			Corresponds to that of BCCH-FREQ-NCELL-5

RXLEV-NCELL-6		See note 1

BCCH-FREQ-NCELL-6		See note 2

BSIC-NCELL-6			Corresponds to that of BCCH-FREQ-NCELL-6

NOTE 1:	These actual values are not checked.

NOTE 2 :	ARFCN 2 is the serving cell carrier.

c=1 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 1020, n=6

c=2 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0, 1020 stronger than 1005 but not broadcasted in SYS INFO), n=6

c=3 report on ARFCNs  2, 20, 26, 38, 1003, 1005, 1020, 0 (990 less strong, 520 DCS), n=6

c=4 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0 (1000 less strong, 520 DCS, 1020 not broadcasted in SYS INFO), n=6

c=5 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0 (884 DCS), n=6

c=6 report on 2, 26, 990, 1003,  n=6

NOTE 3:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

#### 26.10.2.2	E-GSM signalling / RR / Immediate assignment

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.1 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.1 under the described GSM900 conditions.

The purpose of this extra section is to test the immediate assignment procedure in the case where the target channel is E-GSM frequency.

##### 26.10.2.2.1	Conformance requirement

Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on a supported channel described in the IMMEDIATE ASSIGNMENT message, using some E-GSM frequencie.

Reference(s)

GSM 04.08 section 3.3.1.2.1

##### 26.10.2.2.2	Test purpose

To verify that the MS can correctly set up a dedicated control channel when E-GSM frequencies are used.

This tested for a SDCCH/8

##### 26.10.2.2.3	Method of test

Initial Conditions

System Simulator:

1 cell,

CCCH-CONF is set to '1 basic physical channel used for CCCH not combined with SDCCHs'.

SYTEM INFORMATION type 1 message contains the following frequencies in the Cell Channel Description IE: 0, 30, 40, 66, 80, 1005, 1010, 1015 (use range 1024 to encode)

BCCH carrier number 1015

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statement(s)

none

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

This test procedure is performed twice.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST message the SS assigns an SDCCH channel using some E-GSM frequencies. The MS shall go to the correct channel and send a PAGING RESPONSE message. Then the SS initiates RR-release by sending a CHANNEL RELEASE message.

Maximum Duration of Test

15 seconds

Expected Sequence

The sequence is performed for execution counter k=1,2

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	cause "answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Channel Type : see below
	4	MS -> SS	PAGING RESPONSE	Shall be sent on the correct channel
	5	SS -> MS	CHANNEL RELEASE

Specific Message Contents

IMMEDIATE ASSIGNMENT

as default except :

Information element				remark/value

Channel description

- Channel Type 			SDCCH/8

- Timeslot number			arbitrary but not zero

- Training sequence code			arbitrary

- Hopping channel			k=1 Single RF

k=2	RF hopping channel

- Channel Selector			k=1  ARFCN=1015

k=2 MAIO=arbitrarily chosen

HSN arbitrary chosen from the set (1,..63)

Mobile allocation				k=1 empty

k=2 indicates the following frequencies

(0, 80, 1005, 1010)

#### 26.10.2.3	E-GSM signalling / RR / channel assignment procedure

This test is only applicable to an E-GSM mobile station.

Conformance requirements of section 26.6.4 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.4 under the described GSM900 conditions.

The purpose of this extra section is to test the assignment in the case where the allocated channel uses E-GSM frequencie(s) : in the case of frequency hopping several coding formats may be used by the network in the ASSIGNMENT COMMAND message.

##### 26.10.2.3.1	Conformance requirements

1.	Upon receipt of the ASSIGNMENT COMMAND message, the mobile station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the main signalling links).

2.	The MS shall apply the hopping frequencies specified in ASSIGNMENT COMMAND message in the mobile allocation or frequency list or frequency short list at the time accessing the new channel.

References

GSM 04.08, subclause  3.4.3

GSM 04.08 subclause 9.1.2

##### 26.10.2.3.2	Test purpose

1.	To verify that upon receipt of an ASSIGNMENT COMMAND, the MS switches to the channel defined in the ASSIGNMENT COMMAND, establishes the link and sends an ASSIGNMENT COMPLETE message.

2.	To verify that an MS, having received an ASSIGNMENT COMMAND, is able in case of frequency hopping to decode the mobile allocation and frequency list correctly and applies the specified frequencies.

##### 26.10.2.3.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state with a TMSI allocated.

Related PICS/PIXIT Statements

-	TCH supported (Y/N)

-	The supported channel mode(s) need to be declared.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The test procedure is performed 2 times.

The SS pages the MS and allocates an SDCCH. Then a channel is assigned with ASSIGNMENT COMMAND. Each time the MS shall switch to the assigned channel, establish the link and send an ASSIGNMENT COMPLETE message.

The SS initiates the channel release procedure.

Maximum Duration of Test

3 min

Expected Sequence

The test sequence is performed for execution counter k=1 to 2

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGN	Timeslot Number = n.
	4	MS -> SS	PAGING RESPONSE

5	SS -> MS	ASSIGN COMMAND	See specific message contents
				Timeslot Number = (n+1) mod 8
	6	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after 
				establishment of the main signalling link.
				Steps 5 and 6 are repeated cmax times, where 
				cmax is the number of frequency formats allowed
				for each value of k.
				Use repetition counter c:
				See specific message content.

7	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents

ASSIGNMENT COMMAND:

Information element			value/remark

Protocol Discriminator			RR

Skip indicator				0000

Message type				ASSIGNMENT COMMAND

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO					arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Power Command

- Power level			Arbitrarily chosen

For k=1

Cell Channel Description IE is not included

Frequency list				for c=1, use range 128

to encode the following frequencies :

(1005, 1010, 1015)

for c=2, use range 256

to encode the following frequencies

(73, 74, 75, 76, 77)

for c=3, use range 512

to encode the following frequencies

(980, 981, 982, 983)

for c=4, use range 1024

to encode the following frequencies

(30, 40, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 991, 992, 993, 994, 1015)

with an encoding origin set to 900

for c=6, use Bitmap 0

to encode the following frequencies

(20, 40, 66)

For k=2

Cell Channel Description 			for c=1, use range 128

to encode the following 13 frequencies :

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=2, use range 256

to encode the following 13 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108,

114, 115)

for c=3, use range 512

to encode the following 13 frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=4, use range 1024

to encode the following 8 frequencies

(0, 30, 40, 66, 80, 1005, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=6, use Bitmap 0

to encode the following 12 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114)

For k = 2

Mobile Allocation				indicates the following the frequencies :

for c=1 (1005, 1010, 1015)

for c=2 (73, 74, 75, 76, 77)

for c=3 (980, 981, 982, 983)

for c=4 (0, 30, 40, 1010, 15)

for c=5 (990, 991, 992, 993, 994)

for c=6 (20, 40, 66)

Mode of the First channel			speech full rate for TCH/F

except if speech is not supported :

arbitrary from those supported

Starting Time				not included

#### 26.10.2.4	E-GSM signalling / RR / Handover

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.5 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.5 under the described GSM900 conditions.

The purpose of this extra section is to test the handover in the case where the target channel uses E-GSM frequencie(s) : in the case of frequency hopping several coding formats may be used by the network in the HANDOVER COMMAND message.

##### 26.10.2.4.1	E-GSM signalling / RR / Handover / Successful handover

26.10.2.4.1.1	Conformance requirements

The MS shall correctly apply the handover procedure from a channel without frequency hopping in the primary band to a channel with frequency hopping using P-GSM and E-GSM frequencies whatever the coding used for the frequency hopping description.

References

GSM 04.08, section 3.4.4 and 9.1.15

26.10.2.4.1.2	Test purpose

To check that the MS correctly performs a non-synchronized handover, from a non hopping primary band SDCCH to a hopping TCH or SDCCH using E-GSM frequencies, whatever the coding used for the hopping sequence description and that it activates the new channel correctly.

This is tested in the following case :

E-GSM signalling / Handover / successful / call under establishment / non-synchronized /

- from SDCCH/8 to TCH/F if the MS supports a TCH

- from SDCCH/8 to SDCCH/8 if not

26.10.2.4.1.3	Method of test

Initial Conditions

System Simulator:

2 cells A and B with same LAI, default parameters

except for Cell A : the broadcasted Cell Channel Description in SYSTEM INFORMATION type 1 message contains the following frequencies : 20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114 (encoded using Bit Map 0 format)

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Related PICS/PIXIT Statements

Support of TCH (Y/N)

Support of speech (Y/N)

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

"Idle, updated" with TMSI allocated and camped on cell B.

Test Procedure

The test procedure is performed 18 times.

A Mobile Originating Call is initiated on cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND message, ordering the MS to switch to cell B.

The MS shall then begin to send access bursts on the new channel, a TCH/F if supported (see PICS/PIXIT) or an SDCCH if not, to cell B.

The SS observes the access bursts. After receiving n access bursts, n being randomly drawn between 10 and 20 for the TCH case, 2 and 5 for the SDCCH (see table 2 of section 26.6.5) , the SS sends one PHYSICAL INFORMATION message with a Timing Advance of 20 (see table 1 of section 26.6.5).

The MS shall activate the channel in sending and receiving mode. Then the MS shall establish a signalling link using the correct timing advance. The MS shall be ready to transmit a HANDOVER COMPLETE message before x ms after the end of the PHYSICAL INFORMATION message but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

The term 'ready to transmit' is specified in TS GSM 04.13. The value of 'x' depends upon the target channel:

case SDCCH/8 x = 750

case TCH/F x=500

Maximum Duration of Test

10 min

Expected Sequence

The sequence is performed for execution counter k=1 to 3 and c=1 to 6

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated on cell A
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call,
				NECI not set to 1
	3	SS -> MS	IMM ASSIGNMENT	see specific message contents
				
	  4	MS -> SS	CM SERVICE REQUEST	CM Service Type = Mobile Origina-
				ting Call Establishment.
	  5	SS -> MS	CIPH MODE COMMAND
	  6	MS -> SS	CIPH MODE COMPLETE
	  7	MS -> SS	SETUP	Last L2 frame not acknowledged by
				the SS.
	 8	SS -> MS	HANDOVER COMMAND	 see specific message contents
	 9	MS -> SS	HANDOVER ACCESS	Repeated  on every burst
				 of the uplink main DCCH until reception
				 of PHYSICAL	INFORMATION.
				 Handover Reference as
				included in the HANDOVER COMMAND
	 10	SS -> MS	PHYSICAL INFO	Sent after reception of n HANDOVER
				ACCESS. Timing Advance : 20
	11	MS -> SS	SABM	Sent without information field
	12	SS -> MS	UA
	 13	MS -> SS	HANDOVER COMPLETE	This message shall be ready
				 to be transmitted before 'x' ms
				 after the completion of step 10
	 14	MS -> SS	SETUP	Same N(SD) as in step 7.
	 15	SS -> MS	CHANNEL RELEASE

Specific Message Contents

IMMEDIATE ASSIGNMENT

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		20

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Mobile Allocation				indicates the following frequencies :

(20, 66, 78)

HANDOVER COMMAND

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		40

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Synchronisation Indication IE is not included

For k = 1,

Cell Channel DescriptionIE is not included

Frequency short list			for c=1, use range 128

to encode the following frequencies :

(1005, 1010, 1015)

for c=2, use range 256

to encode the following frequencies

(73, 74, 75, 76, 77)

for c=3, use range 512

to encode the following frequencies

(980, 981, 982, 983)

for c=4, use range 1024

to encode the following frequencies

(30, 40, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 991, 992, 993, 994, 1015)

For k=2

Cell Channel Description IE is not included

Frequency list				for c=1, use range 128

to encode the following frequencies :

(1005, 1010, 1015)

for c=2, use range 256

to encode the following frequencies

(73, 74, 75, 76, 77)

for c=3, use range 512

to encode the following frequencies

(980, 981, 982, 983)

for c=4, use range 1024

to encode the following frequencies

(30, 40, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 991, 992, 993, 994, 1015)

for c=6, use Bitmap 0

to encode the following frequencies

(20, 40, 66)

For k=3

Cell Channel Description 			for c=1, use range 128

to encode the following 13 frequencies :

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=2, use range 256

to encode the following 13 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108,

114, 115)

for c=3, use range 512

to encode the following 13 frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=4, use range 1024

to encode the following 8 frequencies

(0, 30, 40, 66, 80, 520, 975, 1005, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=6, use Bitmap 0

to encode the following 12 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114)

For k = 3

Mobile Allocation				indicates the following the frequencies :

for c=1 (1005, 1010, 1015)

for c=2 (73, 74, 75, 76, 77)

for c=3 (980, 981, 982, 983)

for c=4 (30, 40, 1010, 1015)

for c=5 (990, 991, 992, 993, 994)

for c=6 (20, 40, 66)

Mode of the First channel			speech full rate for TCH/F

except if speech is not supported :

arbitrary from those supported for SDCCH/8

##### 26.10.2.4.2	E-GSM signalling / RR / Handover / layer 1 failure

##### 26.10.2.4.2.1	Conformance requirements

During a handover from a channel in the E-GSM band to a channel in the P-GSM band, or the contrary, the MS shall correctly return to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell.

References

GSM 04.08 section 3.4.4.

26.10.2.4.2.2	Test purpose

To check  that the MS correctly returns to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell, even if the origin is in the P-GSM band and the targetin the E-GSM band.

26.10.2.4.2.3	Method of test

Initial Conditions

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call on a P-GSM channel of cell A. power level = 10

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/FS

Supported mode(s).

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell A), used power level 10.

Test Procedure

The MS is in the active state (U10) of a call on a P-GSM channel of cell A (used power level 10). The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B (power level 12). With the exception of normal BCCH signalling, the SS does not transmit anything on cell B (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test

1 min

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	to an E-GSM channel
				see specific message contents
	  2	MS -> SS	HANDOVER ACCESS	Several messages are sent, all
				with correct Handover References.
	  3	MS -> SS	HANDOVER FAILURE	Sent on old channel,
				RR cause value ="Abnormal release,
				unspecified", "Abnormal release,
				channel unacceptable", "Abnormal
				release, timer expired" or
				"Abnormal release, no activity on
				the radio path". Layer 1 header
				has the same power level as the
				layer 1 header in step 1.
				Shall be sent
				within 3 seconds from the trans-
				mission of HANDOVER COMMAND.

Specific Message Contents

HANDOVER COMMAND

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		40

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				chosen randomly from the set

(1,2...63)

Synchronisation Indication IE is not included

Cell Channel DescriptionIE is not included

Frequency short list			use range 128

to encode the following frequencies :

(1005, 1010, 1015)

Mode of the first channel 			Full rate speech if supported

If not, arbitrary from those supported except signalling.

#### 26.10.2.5	E-GSM signalling / RR / Frequency Redefinition

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.6 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.6 under the described GSM900 conditions.

The purpose of this extra section is to test the frequency redefinition procedure in the case where the target channel uses E-GSM frequencie(s) : in the case of frequency hopping several coding formats may be used by the network in the FREQUENCY REDEFINITION message.

##### 26.10.2.5.1	Conformance requirements

1) 	To verify that the MS, after receiving a FREQUENCY REDEFINITION message, correctly starts using the new frequencies, and hopping sequence when some E-GSM frequencies are used.

2) 	The last received Cell Channel Description information element is used to decode the Mobile Allocation IE received on the FREQUENCY REDEFINITION message.

References

GSM 04.08 3.4.5 and 9.1.13.

26.10.2.5.2	Test purpose

1)	To verify that the MS, after receiving a FREQUENCY REDEFINITION message, starts using the new frequencies and hopping sequence when some E-GSM frequencies are used.

2)	To check that the last received Cell Channel Description information element is used to decode the Mobile Aloocation IE received in the FREQUENCY REDEFINITION message.

##### 26.10.2.5.3	Method of test

Initial conditions

System Simulator:

1 cell; default parameters.

Mobile Station:

The MS is in "idle, updated" state with TMSI allocated.

Related PICS/PIXIT statement(s)

- Support of a TCH (Y/N)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The test procedure is performed six times.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a TCH/F if supported otherwise a SDCCH/8. Then the SS sends to MS a FREQUENCY REDEFINITION. The MS shall then use the new frequencies/hopping sequence.

Maximum duration of test

3 min

Expected sequence

The sequence is performed for execution counter c=1 to 6

Step	Direction	Message	Comments

1	SS->MS	PAGE REQ TYPE 1
	2	MS->SS	CHANNEL REQUEST

3	SS->MS	IMMEDIATE ASSIGN	Channel type=TCH/F if a TCH is supported 
				otherwise, Channel type=SDCCH/8. 
	4	MS->SS	PAGING RESPONSE	Sent on the correct channel after 
				establishment of the main signalling link
	5	SS->MS	FREQUENCY REDEFINITION	See specific message contents.
	6	-----	------------------	The SS checks that the MS is transmitting on 
				the correct frequencies.

7	SS->MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents

FREQUENCY REDEFINITION

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		40

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Cell Channel Description 			for c=1, use range 128

to encode the following 13 frequencies :

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=2, use range 256

to encode the following 13 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108,

114, 115)

for c=3, use range 512

to encode the following 13 frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=4, use range 1024

to encode the following 8 frequencies

(0, 30, 40, 66, 80, 1005, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=6, not present

(the mobile station will use the last Cell Channel

Description IE received, i.e. the one broadcast in the

SYSTEM INFORMATION TYPE 1 message).

Mobile Allocation				indicates the following frequencies :

for c=1 (1005, 1010, 1015)

for c=2 (73, 74, 75, 76, 77)

for c=3 (980, 981, 982, 983)

for c=4 (30, 40, 1010, 1015)

for c=5 (990, 991, 992, 993, 994)

for c=6 (30, 50, 70)

Starting time				indicates

(current frame number + 100 frames) mod 42432

### 26.10.3	E-GSM signalling / Structured procedure

Conformance requirements of section 26.9 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.9 under the described GSM900 conditions.

The purpose of these tests is to verify that the MS performs certain elementary procedures of the RR, MM, and CC protocol correctly within a structured procedure. This tested here in an E-GSM context to evaluate the global behaviour of an E-GSM mobile station using its specific frequency capability.

One mobile originated call and one emergency call are performed.

In one of the different cases the different following situations occur : early and late assignments, call release initiated by the network and by the MS.

#### 26.10.3.1	E-GSM signalling / Structured procedure / Mobile originated call

##### 26.10.3.1.1	Conformance requirement

1)	An MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS , shall start to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

2)	Upon receipt of the ASSIGNMENT COMMAND message, the Mobile Station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the data links). After the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message, specifying cause 'normal event', to the network on the main DCCH.

3,4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

References

Conformance requirement 1:	TS GSM 04.08 section 3.3.1.

Conformance requirement 2:	TS GSM 04.08 section 3.4.3.1, 3.4.3.2

Conformance requirement 3:	TS GSM 04.08 section 3.4.3.

Conformance requirement 4:	TS GSM 04.08 section 5.2.1.1.6.

Conformance requirement 5:	TS GSM 02.07.

Conformance requirement 6:	TS GSM 04.08 sections 5.2.1.1.6 and 5.1.3.

##### 26.10.3.1.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message.

2)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed authentication and cipher mode setting procedures, after having sent a SETUP message, after having received a CALL PROCEEDING message followed by an ALERTING message and an ASSIGNMENT COMMAND message allocating an appropriate TCH, the MS sends an ASSIGNMENT COMPLETE message.

3)	To verify that subsequently, after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message returns a CONNECT ACKNOWLEDGE message.

4)	To verify that after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message attaches the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

##### 26.10.3.1.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM state “idle updated”.

Test procedure

The following test is performed for one teleservice supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with late assignment. The release of the call is initiated by the MS.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" is entered
	  3	MS		
	  4	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"originating call and the
				network does not set the NECI bit to 1".
	  5	SS -> MS	IMMEDIATE ASSIGNMENT	
	  6	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.
	  7	SS -> MS	AUTHENTICATION REQ
	  8	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  9	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  10	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 11	SS		SS starts ciphering.
	 12	MS -> SS	SETUP
	 13	SS -> MS	CALL PROCEEDING
	 14	SS -> MS	ALERTING
	 15	MS		Depending on the PICS, an alerting
				indication is given.
	 16	SS -> MS	ASSIGNMENT COMMAND
	 17	MS -> SS	ASSIGNMENT COMPLETE
	 18	SS -> MS	CONNECT
	 19	MS -> SS	CONNECT ACKNOWLEDGE
	 20	MS		The appropriate bearer channel is
				through connected in both directions.
	 21	MS		If the call is a data call, 
				the TCH shall be through
				connected in both
				directions.
	22	MS		The MS is made to release the call.
	23	MS -> SS	DISCONNECT
	24	SS -> MS	RELEASE
	25	MS -> SS	RELEASE COMPLETE
	26	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

#### 26.10.3.2	E-GSM signalling / Structured procedures / emergency call

Emergency call establishment can be initiated by an MS whether location updating has been successful or not and whether a SIM is inserted into the MS or not; but only if the MS is equipped for speech.

This section is only applicable to an MS supporting speech.

##### 26.10.3.2.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

4),5)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

6)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

7)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 5.2.1.2.

For conformance requirements 4 and 5:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 6:

TS GSM 04.08, sections 5.2.1.1.6 and 5.1.3.

For conformance requirement 7:

TS GSM 04.08, section 5.4.

##### 26.10.3.2.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message specifying the correct CKSN and TMSI, with CM Service Type "emergency call establishment".

3)	To verify that after receipt of a CM SERVICE ACCEPT message from the SS, the MS sends an EMERGENCY SETUP message.

4)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure, the MS performs correctly that assignment procedure.

5)	To verify subsequent correct performance of a connect procedure.

6)	To verify that subsequently the MS has through connected the TCH in both directions.

7)	To verify that the call is cleared correctly.

This is tested in the following case :

Structured procedures / emergency call / idle, no IMSI / accept case

##### 26.10.3.2.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters except :

BCCH ARFCN = 990

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	  2	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"emergency call".
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	
	  4	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
				The mobile identity IE specifies the
				IMEI of the MS.
				The cipher key sequence number IE
				indicates "no key is available".
				The mobile station classmark IE is as
				specified by the manufacturer in a
				PICS/PIXIT statement.
	  5	SS -> MS	CM SERVICE ACCEPT	
	 6	MS -> SS	EMERGENCY SETUP	
	 7	SS -> MS	CALL PROCEEDING
			
	8	SS -> MS	ASSIGNMENT COMMAND	The rate of the channel is one
				indicated by the EMERGENCY SETUP
				message.
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	 11	SS -> MS	CONNECT
	 12	MS -> SS	CONNECT ACKNOWLEDGE
	 13	MS		The TCH is through connected in both
				directions.
	 14	SS -> MS	DISCONNECT
	 15	MS -> SS	RELEASE
	 16	SS -> MS	RELEASE COMPLETE
	 17	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

#### 26.10.3.3	Default contents of messages

same as in 26.9.7 except for the following :

ASSIGNMENT COMMAND

Information element			Value/remark

Description of the first channel		describes non-hopping Bm+ACCHs or Lm+ACCHs as

appropriate for the test

ARFCN = 990

Power Command

-					Chosen arbitrarily but within the range supported by the

MS.

Frequency list				Omitted

Cell channel description			Omitted

Mode of the first channel 			appropriate for on bearer capability chosen for the test

Description of the second channel		Omitted

Mode of the second channel			Omitted

Mobile allocation				Omitted

Starting time				Omitted

Cipher mode setting			Omitted

IMMEDIATE ASSIGNMENT

Information element			Value/remark

Page mode 				Normal paging

Channel description			describes a valid SDCCH+SACCH

in non-hopping mode

ARFCN = 40

Request reference

Random access information		As received from MS

N51, N32, N26			Corresponding to frame number

of the CHANNEL REQUEST

Timing advance				Arbitrary

Mobile allocation				Empty (L=0)

Starting time				Omitted

### 26.10.4	E-GSM signalling / Default message contents

Default SYSTEM INFORMATION:

NOTE :	SYSTEM INFORMATION 7, and SYSTEM INFORMATION 8 messages are not used.

SYSTEM INFORMATION type 1 message

Information elements				Value/Remark

Cell Channel Description

For Cell A

-	Format identifier			Bit Map 0

-	Cell Allocation ARFCN			Channel Numbers 20, 30, 50 and 70.

For Cell B

-	Format identifier			Bit Map 0

-	Cell Allocation ARFCN			Channel Numbers 10, 12, 40, 60, 62, 63

RACH Control parameters				see below

SI1 Rest octets					see below

SYSTEM INFORMATION type 2 message

Information elements				Value/Remark

BCCH frequency list

For cell A

-	Format identifier			bit map 0

-	BCCH Allocation Sequence		0

-	BCCH Allocation ARFCN			Channels numbers 10, 20, 80, 90,

100, 110 and 120

-	EXT-IND				This IE does not carry the complete BA

For cell B

-	Format identifier			bit map 0

-	BCCH Allocation Sequence		0

-	BCCH Allocation ARFCN			Channels numbers 10, 36, 40, 114, 118

-	EXT-IND				This IE does not carry the complete BA

NCC permitted					see below

RACH control parameters				see below

SYSTEM INFORMATION type 2bis message

Information elements				Value/Remark

Extended BCCH frequency list

For cell A

-	Format identifier			range 256

-	BCCH Allocation Sequence		0

-	BCCH Allocation ARFCN			988, 990, 1003

-	EXT-IND				This IE does not carry the complete BA

For cell B

-	Format identifier			range 128

-	BCCH Allocation Sequence		0

-	BCCH Allocation ARFCN			Channels numbers 1005, 1010, 1015

-	EXT-IND				This IE does not carry the complete BA

RACH control parameters				see below

SI 2bis rest octets					see below

SYSTEM INFORMATION type 3 message

Information elements				Value/Remark

Cell identity					see below

LAI							see below

Control channel description				see below

Cell options					see below

Cell Selection parameters				see below

RACH control parameter				see below

SI3 Rest octets 					see below

SYSTEM INFORMATION type 4 message

Information elements				Value/Remark

LAI							see below

Cell selection parameters				see below

RACH control parameters 				see below

CBCH Channel Description				see below

CBCH MA					see below

SI4 Rest octets					see below

SYSTEM INFORMATION type 5 message

Information elements				Value/Remark

BCCH frequency list

For cell A

-	Format identifier			bit map 0

-	BCCH Allocation Sequence		0

-	BCCH Allocation ARFCN			Channels numbers 10, 20, 80, 90,

100, 110 and 120

-	EXT-IND				This IE does not carry the complete BA

For cell B

-	Format identifier			Bit map 0

-	BCCH Allocation Sequence		0

-	BCCH Allocation ARFCN			Channels numbers 10, 36, 40, 114, 118

-	EXT-IND				This IE does not carry the complete BA

SYSTEM INFORMATION type 5bis message

Information elements				Value/Remark

Extension of BCCH frequency list description

For cell A

-	Format identifier			Range 256

-	BCCH Allocation Sequence		0

-	BCCH Allocation ARFCN			Channels numbers 988, 990, 1003

-	EXT-IND				This IE does not carry the complete BA

For cell B

-	Format identifier			range 128

-	BCCH Allocation Sequence		0

-	BCCH Allocation ARFCN			Channels numbers 1005, 1010, 1015

-	EXT-IND				This IE does not carry the complete BA

SYSTEM INFORMATION type 6 message

Information elements				Value/Remark

Cell identity 					see below

LAI							see below

Cell options 					see below

NCC permitted					see below

Common contents of information elements in SYSTEM INFORMATION TYPE 1 to 6 messages.

(CBCH) Channel Description			Not present

(CBCH) Mobile Allocation				Not present

Cell Identity

-	Cell Identity Value			0001H for cell A,

0002H for cell B

Cell Options

-	Power Control Indicator			Power Control Indicator is not set

-	DTX Indicator				MS shall not use DTX

-	Radio-Link-Timeout			8 SACCH blocks

Cell Selection Parameters

-	Cell-Reselect-Hysteresis			12 dB

-	MX-TXPWR-MAX-CCH			Minimum level

-	ACS					No addition cell parameters are present in

SYSTEM INFORMATION messages 7 and 8.

-	NECI					New establishment causes not supported

-	RXLEV-ACCESS-MIN			Minimum level

Control Channel Description

-	Attach-Detach allowed			No Attach/Detach

-	BS-AG-BLKS-RES			0 blocks reserved for access grant

-	CCCH-CONF				1 basic physical channel used for

CCCH, combined with SDCCHs

-	BS-PA-MFRMS			5 multiframe periods for transmission of

paging messages

-	T3212 Timeout value			Infinite

L2 pseudo length

-	System information 1			21

-	System information 2			22

-	System information 2bis			21

-	System information 3			18

-	System information 4			12

Location Area Identification

-	Mobile Country Code			001 decimal

-	Mobile Network Code			01 decimal

-	Location Area Code			0001H

Message Type

-	System information 1			00011001

-	System information 2			00011010

-	System information 2bis			00000010

-	System information 3			00011011

-	System information 4			00011100

-	System information 5			00011101

-	System information 5bis			00000101

-	System information 6			00011110

NCC Permitted					0000 0010

RACH Control Parameters

-	Max Retrans				Max 1 retrans

-	Tx-integer				5 slots used

-	Cell Barred for Access			Cell is not barred

-	Call Reestablishment Allowed		Not allowed

-	Access Control Class			Access is not barred

-	Emergency Call allowed			Yes

SI 1 rest octets					Not used (all bits are set to spare)

SI 2 rest octets					Not used (all bits are set to spare)

SI 3 rest octets					Not used (all bits are set to spare)

SI 4 rest octets					Not used (all bits are set to spare)

Default settings for cell A:

Downlink input level				63 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power class

Propagation profile				static

BCCH/CCCH carrier number			20

Default settings for cell B:

Downlink input level				53 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power  class

Propagation profile				static

BCCH/CCCH carrier number			10

Default message contents for other messages

For sections 26.10.1 to 26.10.2.5 			same as in 26.6.14

For section 26.10.3				same as in 26.9.7