# 31	Test of supplementary services

The general aspects of the specification of supplementary services at the layer 3 radio interface are given in GSM 04.10.

The formats and coding are given in GSM 04.80. If the value of a parameter of an uplink message (MS to network) is specified in a test, the implicit meaning is that it has to be checked; if the value is not specified, it is not to be checked unless otherwise stated.

Unless otherwise stated, the MS shall be in the idle updated state at the beginning of each test (including repetition of a test).

In each test, before the MS sends the first REGISTER message, a MM connection is established.

GSM 04.81 to 04.88 give the procedures used at the radio interface for normal operation, registration, erasure, activation, deactivation, invocation and interrogation of supplementary services.

The supplementary services are described in GSM 02.04 and 02.81 to 02.88.

Whenever activation via the standard MMI is mentioned, if the MS doesn't support it but supports a different procedure, this different procedure is used. In the supplementary services tests, only the applicable (see GSM 02.8x series) MMI service code groups (GSM 02.30 Annex 4), of the basic service code in GSM 09.02,which are supported by the MAP, will be used.

In all the following test cases, the MS will not be failed if it doesn't send a FACILITY information element in the initial REGISTER message, but in a subsequent FACILITY message (although the first case is the normal behaviour).

## 31.1	Number identification supplementary services

(Reserved).

## 31.2	Call offering supplementary services

## 31.2	Call offering supplementary services

The following abbreviations are used:

CFU :	Call Forwarding Unconditional

CFB :	Call Forwarding on mobile subscriber Busy

CFNRy :	Call Forwarding on No Reply

CFNRc :	Call Forwarding on mobile subscriber Not Reachable

CFC :	Call Forwarding Conditional (common name for CFB, CFNRy and CFNRc)

CF :	Call Forwarding (common name for CFU, CFB, CFNRy and CFNRc).

NOTE:	These abbreviations are also used to represent the corresponding SS-Code; e.g. CFC is the SS-Code for all conditional forwarding services.

### 31.2.1	Call forwarding supplementary services

#### 31.2.1.1	Registration

##### 31.2.1.1.1	Registration accepted

###### 31.2.1.1.1.1	Conformance requirements

For registration of any type of call forwarding with any parameters, the MS shall transmit successively

1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

3)	and then the REGISTER message containing a facility IE (or an empty REGISTER followed by a FACILITY message containing the facility IE) that includes an invoke of the RegisterSS operation with  parameter values according to the user's request (MMI action).

4)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.2.1.1.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for registration of call forwarding in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for registration of call forwarding in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the RegisterSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of call forwarding.

4)	To check that upon receipt of the result of the operation (in a RELEASE COMPLETE or a FACILITY message), the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for :

a)	CFNRy, for basic service group speech the result of the operation being sent in a FACILITY message.

b)	CFU, for basic service group all facsimile, the result of the operation being sent in a RELEASE COMPLETE message.

###### 31.2.1.1.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of CFNRy for Telephony, to a number arbitrarily selected and with a no reply time value arbitrarily selected.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing the return result of the RegisterSS operation.

The SS transaction is released and the dedicated channel is released.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests registration of CFU for all facsimile, to a number arbitrarily selected.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the RegisterSS operation.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of call forwarding service 
				for CFNRy (Telephony)
	2	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed 
				with an SDCCH" 
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation" 
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator : 
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	CNFRySS operation Return_result
	9	SS -> MS	RELEASE COMPLETE
	10	SS -> MS	CHANNEL RELEASE

11	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CFU (all facsimile)
	12	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed 
				with an SDCCH" 
	13	SS -> MS	IMMEDIATE ASSIGNMENT
	14	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation" 
	15	SS -> MS	CM SERVICE ACCEPT

A16	MS -> SS	REGISTER

B16	MS -> SS	REGISTER 	protocol discriminator : 
				"non call related SS message",
				containing no SSoperation (invoke)
	B17	MS -> SS	FACILITY

18	SS -> MS	RELEASE COMPLETE	RegisterSS operation Return result
	19	MS		provide correct MMI user indication
	20	SS -> MS	CHANNEL RELEASE

Specific message content

step A6 or B7 - CFNRy

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Registration

Supplementary service code = CFNRy

Forwarded to number : as selected,

No reply condition time : as selected,

Basic service code : TeleService (Telephony), no Bearerservice present.

step 8 - CFU

-	protocol discriminator : non call related SS message

-	transaction identifier : same as at step A6 or B6

-	message type : FACILITY

-	facility

Return Result = Registration

Supplementary service code = CFNRy

Forwarded to number

No Reply condition time

Basic service code : TeleService (Telephony), no Bearerservice present.

step A16 or B17 - CFU

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Registration

Supplementary service code = CFU

Forwarded to number : as selected,

Basic service code : no bearer service present, teleservice (all facsimile).

step 18 - CFU

-	protocol discriminator : non call related SS message

-	transaction identifier : same as at step A16 or B16

-	message type : RELEASE COMPLETE

-	facility

Return Result = Registration

Supplementary service code = CFU

Forwarded to number

Basic service code : TeleService (all facsimile), no Bearerservice present.

##### 31.2.1.1.2	Registration rejected

###### 31.2.1.1.2.1	Conformance requirements

1)	A transaction of any kind being already established, for registration of any type of call forwarding with any parameters, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the RegisterSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.2.1.1.2.2	Test purpose

1)	To check that the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of registration of call forwarding, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the RegisterSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of call forwarding.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

These checks are performed with a call transaction already established for :

a)	CFB, for all asynchronous services, the RELEASE COMPLETE message being sent with a facility IE containing a return_error(error) where error is "Bearer Service not provisionned".

b)	CF, for all facsimile, the RELEASE COMPLETE message being sent with a facility IE containing a reject(invoke_problem) where invoke_problem is "resource limitation".

###### 31.2.1.1.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI),  the user requests registration of CFB for all asynchronous services, to a number arbitrarily selected.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing a Return_error(error : BearerService not provisioned) of the RegisterSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests registration of CF for all facsimile, to a number arbitrarily selected.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing a reject(invoke_problem : resource limitation) of the RegisterSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CFB (all asynchronous services)
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation" 
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	RegisterSS operation Return_error
	7	SS -> MS	STATUS ENQUIRY
	8	MS -> SS	STATUS	CCstate U10

9	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CF (all facsimile)
	10	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	11	SS -> MS	CM SERVICE ACCEPT

A12	MS -> SS	REGISTER
	B12	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B13	MS -> SS	FACILITY

14	SS -> MS	RELEASE COMPLETE 	RegisterSS operation Reject
	15	MS		provide correct MMI user indication
	16	SS -> MS	STATUS ENQUIRY
	17	MS -> SS	STATUS	CCstate U10

Specific message content

step A4 or B5 - CFB

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = Registration

Supplementary service code = CFB

Forwarded to number : as selected,

Basic service code : Bearer Service (all asynchronous services), no teleservice present

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : BearerService not provisioned

For the return error the invoke ID must be the same as in the invoke of the RegisterSS operation.

step A12 or B13 - CF

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B12

-	message type : REGISTER (A12), FACILITY (B13)

-	facility

invoke = Registration

Supplementary service code = CF

Forwarded to number : as selected,

Basic service code : no bearer service present, teleservice (all facsimile)).

step 14 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : resource limitation

For the reject the invoke ID must be the same as in the invoke of the Register SS operation.

#### 31.2.1.2	Erasure by the subscriber

##### 31.2.1.2.1	Erasure accepted

###### 31.2.1.2.1.1	Conformance requirements

For erasure of any type of call forwarding, the MS shall transmit successively

1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

3)	and then the REGISTER message containing a facility that includes an invoke of the EraseSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values according to the user request (MMI action).

4)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.2.1.2.1.2	Test purpose

1)	To check that the MS correctly requests supplementary service transaction for erasure of call forwarding in CHANNEL REQUEST message.

2)	To check that the MS correctly requests supplementary service transaction for erasure of call forwarding in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the EraseSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for erasure of call forwarding.

4)	To check that upon receipt of the result of the operation (in a RELEASE COMPLETE or a FACILITY message), the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for :

a)	CFC, for basic service group all facsimile, the result of the operation being sent in a FACILITY message.

b)	CFNRc, for all basic service groups, the result of the operation being sent in a RELEASE COMPLETE message.

###### 31.2.1.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests erasure of CFC for all facsimile.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with a FACILITY message with the facility information element containing a return result of the EraseSS operation.

The SS transaction is released and the dedicated channel is released.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests erasure of CFNRc for all basic service groups.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the EraseSS operation.

The dedicated channel is released.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				erasure of call forwarding
				for CFC (all facsimile)
	2	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	EraseSS operation Return_result
	9	SS -> MS	RELEASE COMPLETE
	10	SS -> MS	CHANNEL RELEASE

11	MS 		The MS is made to initiate
 				a erasure of call forwarding service
				for CFNRc (all basic services)
	12	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed
				with an SDCCH"
	13	SS -> MS	IMMEDIATE ASSIGNMENT
	14	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	15	SS -> MS	CM SERVICE ACCEPT

A16	MS -> SS	REGISTER

B16	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B17	MS -> SS	FACILITY

18	SS -> MS	RELEASE COMPLETE	EraseSS operation Return result
	19	MS		provide correct MMI user indication
	20	SS -> MS	CHANNEL RELEASE

Specific message content

step A6 or B7 - CFC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Erasure

Supplementary service code = CFC

Basic service code : no Bearer Service, teleservice (all facsimile).

step A16 or B17 - CFNRc

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Erasure

Supplementary service code = CFNRc

Basic service code : no bearer service present, no teleservice present.

##### 31.2.1.2.2	Erasure rejected

###### 31.2.1.2.2.1	Conformance requirements

1)	A transaction of any kind being already established, for erasure of any type of call forwarding with any parameters, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the EraseSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.2.1.2.2.2	Test purpose

1)	To check that the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of erasure of call forwarding, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the EraseSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for erasure of call forwarding.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

These checks are performed with a call transaction already established for :

a)	CFU, for Telephony, the RELEASE COMPLETE message being sent with a facility IE containing a return_error(error) where error is "Teleservice not provisioned".

b)	CFNRy, for all facsimile, the RELEASE COMPLETE message being sent with a facility IE containing a reject(invoke_problem) where invoke_problem is "resource limitation".

###### 31.2.1.2.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests erasure of CFU for Telephony.

Upon receipt of the operation (in a REGISTER or a FACILITY message),the system simulator answers with the FACILITY message with the Facility information element containing a Return_error(error : TeleService not provisioned) of the EraseSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests erasure of CFNRy for all facsimile.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing a reject(invoke_problem : resource limitation) of the EraseSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a erasure of call forwarding service
				for CFU (speech)
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT
	A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	EraseSS operation Return_error
	7	SS -> MS	STATUS ENQUIRY
	8	MS -> SS	STATUS	CCstate U10

9	MS 		The MS is made to initiate
 				a erasure of call forwarding service
				for CFNRy (all facsimile)
	10	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	11	SS -> MS	CM SERVICE ACCEPT

A12	MS -> SS	REGISTER
	B12	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B13	MS -> SS	FACILITY

14	SS -> MS	RELEASE COMPLETE 	EraseSS operation Reject
	15	MS		provide correct MMI user indication
	16	SS -> MS	STATUS ENQUIRY
	17	MS -> SS	STATUS	CCstate U10

Specific message content

step A4 or B5 - CFU

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = Erasure

Supplementary service code = CFU

Basic service code : Tele Service (Telephony), no Bearerservice present

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : TeleService not provisioned

For the return error the invoke ID must be the same as in the invoke of the EraseSS operation.

step A12 or B13 - CFNRy

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B12

-	message type : REGISTER (A12), FACILITY (B13)

-	facility

invoke = Erasure

Supplementary service code = CFNRy

Basic service code : no bearer service present, teleservice (all facsimile)).

step 14 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : resource limitation

For the reject the invoke ID must be the same as in the invoke of the EraseSS operation.

#### 31.2.1.3	Activation

##### 31.2.1.3.1	Conformance requirements

For activation of any type of call forwarding with any parameters, the MS shall transmit successively

1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

3)	and then the REGISTER message containing a facility IE that includes an invoke of the ActivateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

4) Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

##### 31.2.1.3.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for activation of call forwarding in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for activation of call forwarding in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for activation of call forwarding.

4)	To check that upon receipt of the result of the operation (in a RELEASE COMPLETE or a FACILITY message), the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for :

a)	CF, for basic service group "all synchrounous services" the result of the operation being sent in a FACILITY message.

b)	CFU, for all basic service groups, the result of the operation being sent in a RELEASE COMPLETE message.

##### 31.2.1.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests activation of CF for all synchronous services.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing the return result of the ActivateSS operation.

The SS transaction is released and the dedicated channel is released.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests activation of CFU for all basic service groups.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the ActivateSS operation.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a activation of call forwarding service
				for CF (all synchronous services)
	2	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	ActivateSS operation Return_result
	9	SS -> MS	RELEASE COMPLETE
	10	SS -> MS	CHANNEL RELEASE

11	MS 		The MS is made to initiate
 				a activation of call forwarding service
				for CFU (all basic service groups)
	12	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	13	SS -> MS	IMMEDIATE ASSIGNMENT
	14	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	15	SS -> MS	CM SERVICE ACCEPT

A16	MS -> SS	REGISTER

B16	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B17	MS -> SS	FACILITY

18	SS -> MS	RELEASE COMPLETE	ActivateSS operation Return result
	19	MS		provide correct MMI user indication
	20	SS -> MS	CHANNEL RELEASE

Specific message contents :

step A6 or B7 - CF

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Activation

Supplementary service code = CF

Basic service code : Bearer Service (all synchronous services), no teleservice present.

step A16 or B17 - CFU

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Activation

Supplementary service code = CFU

Basic service code : no bearer service present, no teleservice present.

#### 31.2.1.4	Deactivation

##### 31.2.1.4.1	Conformance requirements

For deactivation of any type of call forwarding with any parameters, the MS shall transmit successively

1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

3)	and then the REGISTER message containing a facility IE that includes an invoke of the DeactivateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

4) Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

##### 31.2.1.4.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for deactivation of call forwarding in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for deactivation of call forwarding in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for deactivation of call forwarding.

4)	To check that upon receipt of the result of the operation (in a RELEASE COMPLETE or a FACILITY message), the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for :

a)	CFC, for basic service group speech the result of the operation being sent in a FACILITY message.

b)	CFNRc, for basic service group all facsimile, the result of the operation being sent in a RELEASE COMPLETE message.

##### 31.2.1.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests deactivation of CFC for speech.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing the return result of the DeactivateSS operation.

The SS transaction is released and the dedicated channel is released.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests deactivation of CFNRc for all facsimile.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the DeactivateSS operation.

The dedicated channel is released.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a decativation of call forwarding service
				for CFC (speech)
	2	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	DeactivateSS operation Return_result
	9	SS -> MS	RELEASE COMPLETE
	10	SS -> MS	CHANNEL RELEASE

11	MS 		The MS is made to initiate
 				a deactivation of call forwarding service
				for CFNRc (all facsimile)
	12	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed
				with an SDCCH"
	13	SS -> MS	IMMEDIATE ASSIGNMENT
	14	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	15	SS -> MS	CM SERVICE ACCEPT

A16	MS -> SS	REGISTER

B16	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B17	MS -> SS	FACILITY

18	SS -> MS	RELEASE COMPLETE	DeactivateSS operation Return result
	19	MS		provide correct MMI user indication
	20	SS -> MS	CHANNEL RELEASE

Specific message content

step A6 or B7 - CFC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Deactivation

Supplementary service code = CFC

Basic service code : Bearer Service (speech), no teleservice present.

step A16 or B17 - CFNRc

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Deactivation

Supplementary service code = CFNRc

Basic service code : no bearer service present, teleservice (all facsimile).

#### 31.2.1.5	Invocation

Invocation is not applicable to the MS and causes no signalling on the radio path.

#### 31.2.1.6	Interrogation

##### 31.2.1.6.1	Interrogation accepted

###### 31.2.1.6.1.1	Conformance requirements

For interrogation of any specific call forwarding service (not applicable to a group of services) with any parameters, the MS shall transmit successively

1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

3)	and then the REGISTER message containing a facility IE that includes an invoke of the InterrogateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

4) Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.2.1.6.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for interrogation of a specific call forwarding in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for interrogation of call forwarding in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for interrogation of call forwarding.

4)	To check that upon receipt of the result of the operation (in a RELEASE COMPLETE or a FACILITY message), the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for :

a)	CFB, for all basic service groups, the result of the operation being sent in a FACILITY message.

b)	CFC, for basic service group speech, the result of the operation being sent in a RELEASE COMPLETE message.

###### 31.2.1.6.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests interrogation of CFB for all basic service groups.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing the return result of the InterrogateSS operation.

The SS transaction is released and the dedicated channel is released.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests interrogation of CFC for speech.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the InterrogateSS operation.

The dedicated channel is released.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CFB (all)
	2	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	InterrogateSS operation Return_result
	9	SS -> MS	RELEASE COMPLETE
	10	SS -> MS	CHANNEL RELEASE

11	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CFC (speech)
	12	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed
				with an SDCCH"
	13	SS -> MS	IMMEDIATE ASSIGNMENT
	14	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	15	SS -> MS	CM SERVICE ACCEPT

A16	MS -> SS	REGISTER

B16	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B17	MS -> SS	FACILITY

18	SS -> MS	RELEASE COMPLETE	InterrogateSS operation Return result
	19	MS		provide correct MMI user indication
	20	SS -> MS	CHANNEL RELEASE

Specific message content

step A6 or B7 - CFB

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Interrogation

Supplementary service code = CFB

Basic service code : no Bearer Service present, no teleservice present.

step A16 or B17 - CFC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Interrogation

Supplementary service code = CFC

Basic service code : no bearer service present, teleservice (speech).

##### 31.2.1.6.2	Interrogation rejected

###### 31.2.1.6.2.1	Conformance requirements

1)	A transaction of any kind being already established, for interrogation of any specific call forwarding service with any parameters, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.2.1.6.2.2	Test purpose

1)	To check that the MS correctly requests the establishemnt of a parallel MM transaction for supplementary service transaction of interrogation of a specific call forwarding service, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for interrogation of call forwarding.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

These checks are performed with a call transaction already established for :

a)	CFNRc, for all basic service group, the RELEASE COMPLETE message being sent with a facility IE containing a return_error(error) where error is "SS not available".

b)	CFB, for all facsimile, the RELEASE COMPLETE message being sent with a facility IE containing a reject(invoke_problem) where invoke_problem is "resource limitation".

###### 31.2.1.6.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests interrogation of CFNRc for all basic service groups.

Upon receipt of the operation (in a REGISTER or a FACILITY message),the system simulator answers with the FACILITY message with the Facility information element containing a Return_error(error : SS not available) of the InterrogateSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests interrogation of CFB for all facsimile.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message (same PD and TI that in the REGISTER message) with the Facility information element containing a reject(invoke_problem : resource limitation) of the InterrogateSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CFNRc (all)
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS ->SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	InterrogateSS operation Return_error
	7	SS -> MS	STATUS ENQUIRY
	8	MS -> SS	STATUS	CCstate U10
	9	MS 		The MS is made to initiate
 				a interrogtaion of call forwarding service
				for CFB (all facsimile)
	10	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	11	SS -> MS	CM SERVICE ACCEPT

A12	MS -> SS	REGISTER
	B12	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B13	MS ->SS	FACILITY

14	SS -> MS	RELEASE COMPLETE 	InterrogateSS operation Reject
	15	MS		provide correct MMI user indication
	16	SS -> MS	STATUS ENQUIRY
	17	MS -> SS	STATUS	CCstate U10

Specific message content

step A4 or B5 - CFNRc

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = Interrogation

Supplementary service code = CFNRc

Basic service code :no Bearer Service present, no teleservice present

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : SS not available

For the return error the invoke ID must be the same as in the invoke of the InterrogateSS operation.

step A12 or B13 - CFB

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B12

-	message type : REGISTER (A12), FACILITY (B13)

-	facility

invoke = Interrogation

Supplementary service code = CFB

Basic service code : no bearer service present, teleservice (all facsimile)).

step 14 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : resource limitation

For the reject the invoke ID must be the same as in the invoke of the InterrogateSS operation.

#### 31.2.1.7	Normal operation

##### 31.2.1.7.1	Served mobile subscriber side

###### 31.2.1.7.1.1	Notification during an incoming call

This subscription option is only applicable to CFB and CFNRy.

31.2.1.7.1.1.1	Conformance requirements

1)	During a call transaction, call establishment or not, upon receipt of a FACILITY message notifying an incoming call, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

2)	If a call transaction is being established or is already established when the notification of incoming call is received, the receipt of the notification has no effect on its state.

References

1)	GSM 02.30 section 4.5

2)	GSM 04.82 section 1

31.2.1.7.1.1.2	Test purpose

1)	To check that, in state U7 or U10, upon receipt of a FACILITY message notifying an incoming call, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

2)	To check that when the notification of incoming call is received while the MS is in CC state U7 and U10 of another incoming call, it has no effect on its state.

These checks are performed in the case of CFB.

31.2.1.7.1.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

The MS is brought to the call state U7 of an incoming call.

The system simulator transmits a FACILITY message with the facility information element containing an invoke of the NotifySS operation for CFB.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U7.

The MS is brought into the call state U10 of an incoming call.

The system simulator transmits a FACILITY message with the facility information element containing an invoke of the NotifySS operation for CFB.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

The transaction and the channel are released by the SS.

Maximum duration of test

1 mn

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING
	2	MS -> SS	CHANNEL REQUEST	with establisment cause
				"answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	SETUP
	6	MS -> SS	CALL CONFIRMED
	7	SS -> MS	ASS COMMAND
	8	MS -> SS	ASS COMPLETE
	9	SS -> MS	ALERTING
	10	SS -> MS	FACILITY	(invoke NotifySS)
	11	SS -> MS 	STATUS ENQUIRY
	12	MS -> SS	STATUS	(U7)
	13	MS -> SS	CONNECT	MS off hook
	14	SS -> MS	CONNECT ACK
	15 	SS -> MS	FACILITY	(invoke NotifySS)
	16	SS -> MS	STATUS ENQUIRY
	17	MS -> SS	STATUS	(U10)
	18	SS -> MS 	RELEASE COMPLETE
	19	SS -> MS	CHANNEL RELEASE

Specific message contents

Steps 10 and 15

-	protocol discriminator : CC

-	transaction identifier : same as for the call transaction already established

-	message type : FACILITY

-	facility

invoke = notification

Supplementary service code = CFB

SS notification = incoming call is forwarded

(call is forwarded indication to B subscriber).

###### 31.2.1.7.1.2	Notification during an outgoing call

31.2.1.7.1.2.1	Conformance requirements

1)	As an outgoing call is being established, if the ALERTING message is received with the facility information element containing an SS notification (for CFU or CFC), the MS shall correctly reach CC state U4.

2)	As an outgoing call is being established, if the ALERTING message is received with the facility information element containing an SS notification (for CFU or CFC), the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

3)	As an outgoing call is being established, if the CONNECT message is received with the facility information element containing an SS notification (for CFU or CFC), the MS shall normally send a CONNECT ACK message and enter CC state U10.

4)	As an outgoing call is being established, if the CONNECT message is received with the facility information element containing an SS notification (for CFU or CFC), the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.82, T.S. GSM 04.80 section 3.6

2)	GSM 04.82, T.S. GSM 04.80 section 3.6

31.2.1.7.1.2.2	Test purpose

1)	To check that when an outgoing call is being established, if the ALERTING message is received with the facility information element containing an SS notification, the MS correctly reaches CC state U4. This is tested for CFU.

2)	As an outgoing call is being established, if the ALERTING message is received with the facility information element containing an SS notification, the MS provides the appropriate user indication (which is to be described by the manufacturer). This is tested for CFU.

3)	As an outgoing call is being established, if the CONNECT message is received with the facility information element containing an SS notification, the MS normally sends a CONNECT ACK message and enter CC state U10. This is tested for CFC.

4)	As an outgoing call is being established, if the CONNECT message is received with the facility information element containaining an SS notification (for CFU or CFC), the MS provides the appropriate user indication (which is to be described by the manufacturer). This is tested for CFC.

31.2.1.7.1.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

The MS is made to place an outgoing call.

After having received a SETUP message and sent a CALL PROCEEDING message and after a TCH has been allocated the system simulator transmits an ALERTING message with the facility information element containing a notification

The system simulator send then a STATUS ENQUIRY message. On receipt of the STATUS ENQUIRY message the MS shall send a STATUS message with CC-state U4.

After that, the system simulator transmits a CONNECT with the facility information element containing a notification

After reception of a CONNECT ACKNOWLEDGE message the system simulator sends a STATUS ENQUIRY message.

The MS shall respond with a STATUS message indicating CC-state U10.

The transaction and the channel are released by the SS.

Maximum duration of test

1 mn

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST
	2	SS -> MS	IMMEDIATE ASSIGNMENT
	3	MS -> SS	CM SERVICE REQUEST	"mobile originating call establishment"
	4	SS -> MS	CM SERVICE ACCEPT
	5	MS -> SS	SETUP
	6	SS -> MS	CALL PROCEEDING
	7	SS -> MS	ASS COMMAND
	8	MS -> SS	ASS COMPLETE
	9	SS -> MS	ALERTING	 containing facility IE
	10	SS -> MS 	STATUS ENQUIRY
	11	MS -> SS	STATUS	(U4)
	12	SS -> MS	CONNECT	containing facility IE
	13	MS -> SS	CONNECT ACK
	14	SS -> MS 	STATUS ENQUIRY
	15	MS -> SS	STATUS	(U10)
	16	SS -> MS 	RELEASE COMPLETE
	17	SS -> MS	CHANNEL RELEASE

Specific message contents

At step 9 -

Facility invoke = notification

-	SS-Code (CFU)

-	SS-Status (indicating:

Provisioned, registered and active).

At step 12 -

Facility invoke = notification

-	SS-Code (CFC)

-	SS-Status (indicating:

Provisioned, registered and active).

##### 31.2.1.7.2	Forwarded-to mobile subscriber side

###### 31.2.1.7.2.1	Conformance requirements

1)	Upon receipt of the SETUP message containing a notification indication that the call is a forwarded one (with any SS code except CFC), the MS shall correctly continue call establishment and enter CC state U6.

2)	Upon receipt of the SETUP message containing a notification indication that the call is a forwarded one, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.82

2)	GSM O2.30 section 4.5

###### 31.2.1.7.2.2	Test purpose

1)	To check that, upon receipt of the SETUP message containing a notification indication that the call is a forwarded one, the MS correctly continues call establishment and enters CC state U6.

2)	Upon receipt of the SETUP message containing a notification indication that the call is a forwarded one, the MS provides the appropriate user indication (which is to be described by the manufacturer).

###### 31.2.1.7.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

An incoming call is given to the MS with the SETUP message with the facility information element containing an invoke of the NotifySS operation with the indication that the call is forwarded.

Then the network sends a STATUS ENQUIRY message : the MS responds indicating CC state U6.

The transaction and the channel are released by the SS.

Maximum duration of test

1 mn

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING
	2	MS -> SS	CHANNEL REQUEST	with establisment cause
				"answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	SETUP	containing the notification that
				 the call is a forwarded one
	6	SS -> MS 	STATUS ENQUIRY
	7	MS -> SS	STATUS	(U6)
	8	SS -> MS 	RELEASE COMPLETE
	9	SS -> MS	CHANNEL RELEASE

Specific message contents

at step 6 -

-	protocol discriminator : CC

-	transaction identifier :

-	message type : SETUP

-	facility

invoke = notification

-	SS-Code (CFU, CFB, CFNRy, CFNRc or CF)

-	SS-Notification (indicating: call is forwarded i.e.

Call is forwarded indication to C-subscriber).

### 31.2.2	Call transfer and mobile access hunting supplementary services

(Reserved).

## 31.3	Call completion supplementary services

(Reserved).

## 31.4	Multi-party supplementary services

(Reserved).

## 31.5	Community of interest supplementary services

(Reserved).

## 31.6	Charging supplementary services

The following Advice of Charge abbreviations are used in this section:

AOC		Advice of Charge

AOCC	Advice of Charge Charging

AOCI		Advice of Charge Information

ACM		Accumulated Charge Meter

ACMmax	Accumulated Charge Meter Maximum

CCM		Current Call Meter

CAI		Charge Advice Information

CDUR	Chargeable Duration

The following other abbreviations are used:

IE 		Information Element

FIE		Facility Information Element

TCH		Traffic CHannel

General on Advice of Charge.

The purpose of these tests is to verify that the MS under test correctly performs procedures related to the AOC supplementary service.

The reasons for these test purposes are:

-	One example of a possible use for AOCC is in applications where the subscriber hires out a GSM ME and SIM to a user and bills the user according to the charge stored on the SIM at the end of the hire period. If a mobile station claims to support AOCC but does so incorrectly or not at all, this may cause the subscriber to mischarge the user of the hire phone. Hence an MS claiming to support AOCC must be shown to be reliable in that context.

-	Since AOCC offers the use of telecommunication services according to the charge stored independently in the MS, the AOCC service must not be susceptible to fraud at the MS end.

-	To ensure that a mobile station not claiming to support AOCC does not respond with a signal to the network indicating that it does. This could cause the network to allow the call to be placed without any charge being inserted on to the SIM.

Mobile originating and terminating speech AOC calls are tested and if supported, Call Hold and Multi-party calls. The type testing of data calls (i.e. those calls with a volume related charging component) will not be specified at the phase 2 stage but will be deferred to phase 2+ since the interaction of AOC with packet services is not yet defined in GSM 09.06 and GSM 09.07.

Tests are made on the ME-SIM interface to ensure that call charges are being correctly stored in the ACM field of the SIM in several situations. The AOC ACMmax function is also tested for incoming and outgoing calls.

Basic service verification tests for the AOCC, Call Hold, and Multi-party supplementary services which have direct relevance to the testing of AOC have been added to GSM 11.10, sections 11.3, 11.4 and 11.5 respectively.

[Editorial Note: Certain error handling tests for AOC will be added to GSM 11.10 section 26.5.]

[Tests on certain SIM functions related to AOC will be added to GSM 11.10 section ?]

### 31.6.1	Advice of Charge Charging

#### 31.6.1.1	AOC time related charging / MS originated call

Purpose:

1)	To verify that when the MS receives the AOCC parameters in a Facility IE which is contained in the CONNECT message and when a TCH has already been assigned, the MS returns a FACILITY message containing the acknowledgement within 1 second.

2)	To verify that when the MS receives the AOCC parameters in a Facility IE which is contained in a CONNECT message and when a TCH has already been assigned, the MS stores the correct value in the ACM field of the SIM.

3)	To verify that the when the call has no volume related component the MS ignores non-zero AOCC e5, e6 parameters sent to it.

Conformance Requirement(s):

1)	When the MS receives the AOCC parameters in a Facility IE which is contained in a CONNECT message and when a TCH has already been assigned, the MS shall return a FACILITY message containing the acknowledgement within 1 second.

2)	When the MS receives the AOCC parameters in a Facility IE which is contained in a CONNECT or FACILITY message and when a TCH has already been assigned, the MS shall store the correct value in the ACM field of the SIM.

3)	When the call has no volume related component the MS shall ignore non-zero AOC e5 and e6 parameters sent to it.

Reference(s):

Conformance requirement 1:	TS GSM 03.86, 04.13, 04.86.

Conformance requirement 2:	TS GSM 02.24

Conformance requirement 3:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent by the MS before or after the CONNECT ACKNOWLEDGE.

The SS sends the DISCONNECT y seconds after sending the CONNECT message containing the CAI. The MS shall have stored the correct amount on the SIM according to the e-parameters sent. The test is repeated for several different sets of e-parameters as defined below.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,...,5.

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE
	A13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	B13	MS -> SS	CONNECT ACKNOWLEDGE

14			call duration y seconds after CAI information 
				sent by SS
	15	SS -> MS	DISCONNECT
	16	MS -> SS	RELEASE
	17	SS -> MS	RELEASE COMPLETE
	18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	19	MS		SIM contents checked (either via MMI or by 
				removing the SIM and using SIM reader). ACM 
				shall have been incremented correctly.

NOTE :

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

y is set to a constant value of 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as below:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

1	6	14	1	25	0	0	60	43	43
		2	0	0	1	100	0	0	0	100	143
		3	250	16	2	500	0	0	60	2000	2143
		4	1	1	1	0	10	10	1	89 or 90	2232 or 2233
		5	12.5	30	1	25	10	10	30	50 or 62.5	2295,
											2296,
											2282 or 2283

Values shown in table are in the format and have units as in GSM 02.24 section 3.

The ASN.1 description for each e-parameter allows integers in the range 0 to 8191 to be transmitted but some e-parameters have different actual ranges (eg e1 can take any value 0..819.1 with 0.1 resolution). The MS knows how to interpret the received parameter (eg received e1 refers to 10 times actual e1, see GSM 04.80 section 4.4.3). Therefore e1=12.5 would be sent to the MS as 125. The MS knows the value sent is 10 times the 'real' e1 and hence interprets the value as 12.5.

The non-zero e5 and e6 values for the k=4 and k=5 execution of the test are to check that the MS ignores the volume related parameters when carrying out time only related charging.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.1.2	AOC time related charging / MS terminated call

Purpose:

1)	To verify that when the MS receives certain AOCC e-parameters in a Facility IE which is contained in a FACILITY message sent after the CONNECT message and when a TCH has already been assigned, the MS returns a FACILITY message containing the acknowledgement within 1 second.

2)	To verify that when the MS receives the AOCC parameters in a Facility IE which is contained in a FACILITY message and when a TCH has already been assigned, the MS stores the correct value in the ACM field of the SIM.

Conformance Requirement(s):

1)	When the MS ceives the AOCC parameters in a Facility IE which is contained in a FACILITY message sent after the CONNECT message and when a TCH has already been assigned, the MS shall return a Facility message containing the acknowledgement within 1 second.

2)	When the MS receives the AOCC parameters in a Facility IE which is contained in a FACILITY message and when a TCH has already been assigned, the MS shall store the correct value in the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 03.86, 04.13, 04.86.

Conformance requirement 2:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The SS is made to initiate a call. The call is established and certain AOCC e-parameters are sent to the MS in a Facility IE contained within a FACILITY message. The MS shall return the AOCC acknowledgement within 1 second of the FACILITY message. It is an implementation option whether the AOCC e-parameters and the AOCC acknowledge are sent before or after the CONNECT ACKNOWLEDGE.

The SS sends the DISCONNECT y seconds after sending the FACILITY message. The MS shall have stored the correct amount on the SIM according to the e-parameters sent. The test is repeated for several different sets of e-parameters as defined below.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,...,5.

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM
	1	SS 		The SS is made to initiate a call
	2	SS -> MS	PAGING REQUEST
	 3	MS -> SS	CHANNEL REQUEST
	4	SS -> MS	IMMEDIATE ASSIGNMENT
	5	MS -> SS	PAGING RESPONSE
	6	SS -> MS	SETUP
	7	MS -> SS	CALL CONFIRMED
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	 9	MS -> SS	ASSIGNMENT COMPLETE
	10	MS -> SS	ALERTING
	11	MS -> SS	CONNECT
	12	SS -> MS	FACILITY	As default message except contains Facility IE 					with contents as indicated in i below

Either A or B branch is taken

A13	SS -> MS	CONNECT ACKNOWLEDGE
	 A14	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

B13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	 B14	SS -> MS	CONNECT ACKNOWLEDGE

15			call duration y seconds after CAI information 
				sent by SS
	 16	SS -> MS	DISCONNECT
	 17	MS -> SS	RELEASE
	 18	SS -> MS	RELEASE COMPLETE
	 19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	 20	MS		SIM contents checked (either via MMI or by 
				removing the SIM and using SIM reader) ACM 
				shall have been incremented correctly.

NOTE :

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

y is set to a constant value of 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as below:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

1	0	0	0	0	0	0	0	0	0
		2	0	0	1	100	0	0	0	100	100
		3	6	14	1	25	0	0	60	43	143
		4	1	1	1	0	0	0	1	89 or 90	233 or 232
		5	12.5	30	1	25	0	0	30	50 or 62.5	296,
											295,
											282 or 283

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.1.3	AOC volume related charging / MS originated call

Future addition at GSM Phase 2+ stage.

#### 31.6.1.4	AOC volume related charging / MS terminated call

Future addition at GSM Phase 2+ stage.

#### 31.6.1.5	Change in charging information during a call

Purpose:

1)	To verify that when the MS receives new AOCC parameters mid-way through a call in a Facility IE which is contained within a FACILITY message the MS returns a FACILITY message containing the acknowledgement within 1 second.

2)	To verify that when the MS receives new charging information mid-way through a call in the form of a Facility IE contained within a FACILITY message the MS correctly indicates the total charge considering both sets of charging information

Conformance Requirement(s):

1)	When the MS receives new AOCC parameters mid-way through a call in a Facility IE which is contained within a FACILITY message the MS shall return a FACILITY message containing the acknowledgement within 1 second.

2)	When the MS receives new charging information mid-way through a call in the form of a Facility IE contained within a FACILITY message the MS correctly indicates the total charge considering both sets of charging information

Reference(s):

Conformance requirement 1:	TS GSM 03.86, 04.13, 04.86.

Conformance requirement 2:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in a FACILITY message sent after the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of the FACILITY message. x seconds after sending the original CAI, new (and different) e-parameters are sent to the MS in a Facility IE contained within a FACILITY message. The MS shall return the AOCC acknowledge within 1 second of the FACILITY message.

The SS sends the DISCONNECT y seconds after sending the first CAI in the FACILITY message. The MS shall have stored the correct amount on the SIM according to the two sets of e-parameters sent and the times for the two parts of the call

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT

Either A,B or C branch is taken

A12	SS -> MS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in i below
	 A13	MS -> SS	CONNECT ACKNOWLEDGE
	 A14	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

B12	MS -> SS	CONNECT ACKNOWLEDGE
	 B13	SS -> MS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in i below
	 B14	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

C12	SS -> MS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in i below
	 C13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	 C14	SS -> MS	CONNECT ACKNOWLEDGE

Second CAI sent x sec after first CAI
	15	SS -> MS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in iii below
	16	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
				New CAI held in abeyance until CDUR has 
				timed out present e2 value
	 17			Call duration y seconds after first CAI 
				information sent by SS
	 18	SS -> MS	DISCONNECT
	 19	MS -> SS	RELEASE
	 20	SS -> MS	RELEASE COMPLETE
	 21	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	 22	MS		SIM contents checked (either via MMI or by 
				removing the SIM and using SIM reader) ACM 
				shall have been incremented correctly

NOTE :

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x, y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as defined below:

e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

10	28	1	10	0	0	60	(30)	(30)

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

iii) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x is set to a constant value of 80 seconds.

y is set to a constant value of 180 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

e-parameter		CCM total	Step 0 ACM
					at call end	value 				increased by

1	2	3	4	5	6	7

10	14	1	5	0	0	60	65	65

Values shown in table are in the format and have units as in GSM 02.24 section 3.

#### 31.6.1.6	Different formats of charging information

Purpose:

1)	To verify that when the MS receives a Facility IE in which certain e-parameters are set to zero the total charge accumulated is the same as that when the same e-parameters are completely omitted from the Facility IE.

2)	To verify the operation of a shortened channel release procedure where the SS does not send DISCONNECT but only the RELEASE COMPLETE and CHANNEL RELEASE messages or just the CHANNEL RELEASE message.

Conformance Requirement(s):

1)	When the MS receives a Facility IE in which certain e-parameters are set to zero the total charge accumulated shall be the same as that when the same e-parameters are completely omitted from the Facility IE.

2)	The channel shall be correctly released when a shortened channel release procedure is used - the SS does not send DISCONNECT but only the RELEASE COMPLETE and CHANNEL RELEASE messages or just the CHANNEL RELEASE message.

Reference(s):

Conformance requirement 1:	TS GSM 02.24

Conformance requirement 2:	TS GSM 04.08

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

Part 1:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in a FACILITY message sent before the CONNECT message.

The SS sends the DISCONNECT y seconds after sending the FACILITY message containing the e-parameters. The MS shall have stored the correct amount on the SIM according to the e-parameters sent.

Part 2:

Part 1 is repeated twice with the e-parameters that were set to zero above now omitted completely from the Facility IE. The shortened release procedures are used. The MS shall have stored the correct amount on the SIM.

The results of parts 1 and 2 are compared. The value for the charge calculated by the MS shall be identical for parts 1 and 2.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,..3

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in i below

Either A,B or C branch is taken

A12	SS -> MS	CONNECT
	 A13	MS -> SS	CONNECT ACKNOWLEDGE
	 A14	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

B12	SS -> MS	CONNECT
	 B13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	 B14	MS -> SS	CONNECT ACKNOWLEDGE

C12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	 C13	SS -> MS	CONNECT
	 C14	MS -> SS	CONNECT ACKNOWLEDGE

15			call duration y seconds after CAI information 
				sent by SS

Branch D,E and F shall be taken for k= 1,2 
				and 3 respectively

D16	SS -> MS	DISCONNECT
	 D17	MS -> SS	RELEASE
	 D18	SS -> MS	RELEASE COMPLETE
	 D19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

E16	SS -> MS	RELEASE COMPLETE	Shortened channel release procedure
	 E17	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

F16	SS -> MS	CHANNEL RELEASE

20	MS		SIM contents checked (either via MMI or by 					removing the SIM and using SIM reader) ACM 					shall have been incremented correctly.

NOTE :

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

y is set to a constant value of 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value		e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

1	10	40	1	0	0	0	0	20		20

2	10	40	1	---------omitted---------	20		40

3	10	40	1	---------omitted---------	20		60

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.1.7	AOC on a Call Hold call

Purpose:

1)	To verify that when the MS invokes a Call Hold call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS returns a FACILITY message containing the AOCC acknowledgement within 1 second of transmission of each set of e-parameters.

2)	To verify that when the MS invokes a Call Hold call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM records the sum of all the charges for the services currently being used and hence that the ME inserts the correct charge in the ACM field of the SIM.

Conformance Requirement(s):

1)	When the MS invokes a Call Hold call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS shall return a FACILITY message containing the AOCC acknowledgement within 1 second of receiving each set of e-parameters.

2)	When the MS invokes a Call Hold call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM shall record the sum of all the charges for the services currently being used and hence the ME shall insert the correct charge in the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 04.13

Conformance requirement 2:	TS GSM 02.24, 04.83, 04.84, 04.86.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of transmission of the CONNECT message.

The call (call B) is then put on hold by sending a HOLD message from the MS to the SS. The SS shall reply with a HOLD ACKNOWLEDGE. The traffic channel is now available to originate another call.

The MS is made to initiate a second call (call C). The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement in a FACILITY message within 1 second of transmission of the CONNECT message.

The SS sends the DISCONNECT to the MS for call B x seconds after sending the call B CAI in the CONNECT/FACILITY message and the DISCONNECT for call C y seconds after sending the call C CAI in the CONNECT/FACILITY message. The MS shall have stored the correct amount on the SIM according to the two sets of e-parameters sent and call times x and y.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either D or E branch is taken

D12	MS -> SS	CONNECT ACKNOWLEDGE
	 D13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

E12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	 E13	MS -> SS	CONNECT ACKNOWLEDGE

14	MS -> SS	HOLD	Call B put on hold
	15	SS -> MS	HOLD ACKNOWLEDGE
	16	MS 		The MS is made to initiate a second call
	17	MS -> SS	SETUP	TI arbitrary but different from existing TI
	18	SS -> MS	CALL PROCEEDING
	19	SS -> MS	ALERTING
	20	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either F or G branch is taken

F21	MS -> SS	CONNECT ACKNOWLEDGE
	 F22	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

G21	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	 G22	MS -> SS	CONNECT ACKNOWLEDGE

23			Call durations x and y seconds after respective 
				CAI information sent by SS

Either H or I branch is taken

For call C
	 H24	SS -> MS	DISCONNECT	y seconds after call C CAI sent
	 H25	MS -> SS	RELEASE
	 H26	SS -> MS	RELEASE COMPLETE
	 H27	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For call B
	 I24	SS -> MS	DISCONNECT	x seconds after call B CAI sent
	 I25	MS -> SS	RELEASE
	 I26	SS -> MS	RELEASE COMPLETE
	 I27	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

28	MS		SIM contents checked (either via MMI or by 
				removing the SIM and using SIM reader) ACM 
				shall have been incremented correctly.

NOTE :

-	The value of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x set to 180s, y is set to 90s.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

Call		e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

B(time x)	7	40	1	0	0	0	0

C(time y)	13	40	1	0	0	0	0	54	54

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.1.8	AOC on a Multi-party call

Purpose:

1)	To verify that when the MS invokes a Multi-party call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS returns a FACILITY message containing the AOCC acknowledgement within 1 second of transmission of each set of e-parameters.

2)	To verify that when the MS originates a Multi-party call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM records the sum of all the charges for the services currently being used and hence the ME inserts the correct charge in the ACM field of the SIM.

Conformance Requirement(s):

1)	When the MS invokes a Multi-party call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS shall return a FACILITY message containing the AOCC acknowledgement within 1 second of receiving each set of e-parameters.

2)	When the MS originates a Multi-party call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM shall record the sum of all the charges for the services currently being used and hence the ME shall insert the correct charge in the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 04.13

Conformance requirement 2:	TS GSM 02.24, 04.83, 04.84, 04.86.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of transmission of the CONNECT message.

The call (call B) is then put on hold by sending a HOLD message from the MS to the SS. The SS shall reply with a HOLD ACKNOWLEDGE. The traffic channel is now available to originate another call.

The MS is made to initiate a second call (call C). The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of transmission of the CONNECT message.

The MS invokes the multi-party service by sending a FACILITY message to the SS containing the BuildMPTY request

The SS accepts the request and connects the MS with the other existing connections (active call C and held call B) and confirms with a FACILITY message.

The SS sends the DISCONNECT to the MS for call B x seconds after sending the call B CAI in the CONNECT message and the DISCONNECT for call C y seconds after sending the call C CAI in the CONNECT message. The MS shall have stored the correct amount on the SIM according to the two sets of e-parameters sent and call times x and y.

Maximum Duration of Test:

z minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either D or E branch is taken

D12	MS -> SS	CONNECT ACKNOWLEDGE
	 D13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

E12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	 E13	MS -> SS	CONNECT ACKNOWLEDGE

14	MS -> SS	HOLD	Call B put on hold
	15	SS -> MS	HOLD ACKNOWLEDGE
	16	MS 		The MS is made to initiate a second call
	17	MS -> SS	SETUP	TI arbitrary but different from existing TI
	18	SS -> MS	CALL PROCEEDING
	19	SS -> MS	ALERTING
	20	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either F or G branch is taken

F21	MS -> SS	CONNECT ACKNOWLEDGE
	 F22	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

G21	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	 G22	MS -> SS	CONNECT ACKNOWLEDGE

The MS is made to build a multi-party call
	23	MS -> SS	FACILITY (TI A-B/A-C)	As default message except contains Facility IE 
				with contents as indicated in iii below
	24	SS -> MS	FACILITY (TI A-B/A-C)	As default message except contains Facility IE 
				with contents as indicated in iv, below

	25			Call durations x and y seconds after respective
				CAI information sent by SS

Either H or I branch is taken

For call C
	 H26	SS -> MS	DISCONNECT	y seconds after call C CAI sent
	 H27	MS -> SS	RELEASE
	 H28	SS -> MS	RELEASE COMPLETE
	 H29	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For call B
	 I26	SS -> MS	DISCONNECT	x seconds after call B CAI sent
	 I27	MS -> SS	RELEASE
	 I28	SS -> MS	RELEASE COMPLETE
	 I29	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

30	MS		SIM contents checked (either via MMI or by 
				removing the SIM and using SIM reader) ACM 
				shall have been incremented correctly.

NOTE :

-	A-B/A-C indicates a choice. the transaction identifier (TI) used must be that of the active call or the held call (ref. GSM 04.84).

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x set to 180s, y set to 90s.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

Call		e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

B(time x)	19	40	1	0	0	0	0

C(time y)	29	40	1	0	0	0	0	134		134

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

iii) FACILITY	Information Element with Invoke = BuildMPTY component type as defined in GSM 04.80 section 3.6.1 table 3.4.

The following abbreviations are used in the descriptions below:

U	Universal tag class

CS	Context Specific tag class

P	Primitive tag form

C	Constructed tag form

FIE	Facility Information Element

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00011100

Length of FIE contents	8	00001000

Component type tag 	CS/C/tag=1	10100001

Component length	6	00000110

Invoke ID tag	U/P/tag=2	00000010

Invoke ID length	1	00000001

Invoke ID	Arbitrary 	(00000000)			(1 octet)

Op-Code tag	From GSM 04.80	00000010

Op-Code length	1	00000001

Op-Code	Build Multi-party		00000001
	operation						(local value 124)

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00111100

Length of FIE contents	5	00001001

Component type tag 	CS/C/tag=2	10100010

Component length	3	00000011

Invoke ID tag	U/P/Integer	00000010

Invoke ID length	1	00000001

Invoke ID	Same as used as	(00000000)			Invoke ID in Invoke FIE

### 31.6.2	Charge Storage

#### 31.6.2.1	Removal of SIM during an active call

Purpose:

1)	To verify that when the SIM is removed from the ME during an active AOCC call the ME immediately terminates the call.

2)	To verify that when the SIM is removed during an active AOCC call the ME has written the total charge up to that point in the call to the ACM field of the SIM.

Conformance Requirement(s):

1)	When the SIM is removed from the ME during an active AOCC call the ME shall immediately terminate the call.

2)	When the SIM is removed during an active AOCC call midway through an AOC charging time interval (e7 or e2) the ME shall have written the total charge up to the point in the call where the charging time interval last expired to the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 02.17.

Conformance requirement 1:	TS GSM 02.24.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Removal of the SIM is possible without disconnection of power supply

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

This test is only performed if it is possible to remove SIM without disconnecting the power supply. If the battery pack must be removed to get at the SIM see 31.6.2.2.

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

If possible, without removing the power supply, the SIM is removed from the ME y seconds after the SS sends the CAI in the CONNECT/FACILITY message. The call shall be terminated immediately by the MS and the MS shall have stored the correct amount on the SIM according to the e-parameters sent.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE
	 A13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	 B13	MS -> SS	CONNECT ACKNOWLEDGE

14			SIM removed y seconds after CAI information 
				sent by SS.

Either C,D,E or F branch is taken

C15	MS -> SS	DISCONNECT
	 C16	SS -> MS	RELEASE
	 C17	MS -> SS	RELEASE COMPLETE
	 C18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

D15	MS -> SS	RELEASE COMPLETE
	 D16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

E15	MS -> SS	Layer 2 DISC
	 E16	SS -> MS	UA

F15			No further messages are sent

19			SIM contents checked (by removing the SIM 
				and using SIM reader). ACM shall have been 
				incremented correctly.

NOTE :

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The SIM is removed after approximately y=90 seconds

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

10	55	1	10	0	0	10	30	30

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.2.2	Interruption of power supply during an active call

Purpose:

To verify that when the power supply of the MS is removed during an active AOCC call the ME has written the total charge up to that point in the call to the ACM field of the SIM.

Conformance Requirement(s):

When the power supply of the MS is removed during an active AOCC call midway through an AOC charging time interval (e7 or e2) the ME shall have written the total charge up to the point in the call where the charging time interval last expired to the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 02.24.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of removing power supply

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is switched off.

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

The ME power is switched off by pressing power button on MMI during the active call. The call shall be terminated immediately and the MS shall have stored the correct amount on the SIM according to the e-parameters sent.

The test is repeated for ME power being lost by removal of battery pack. The call shall be terminated immediately and the MS shall have stored the correct amount on the SIM according to the e-parameters sent.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,2

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE
				with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE
	 A13	MS -> SS	FACILITY	As default message except contains Facility IE
				with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	 B13	MS -> SS	CONNECT ACKNOWLEDGE

14			ME power interrupted y seconds after CAI 
				information sent by SS by:
				MMI power switch (for k=1)
				Removing battery pack (for k=2)
				Depending on the value of k
				C or D branch is taken
	15			SIM contents checked (by removing the
				SIM and using SIM reader). ACM shall have
				been incremented correctly.

NOTE :

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The ME power is removed after approximately y=90 seconds

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

10	55	1	10	0	0	10	30	30

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.2.3	MS going out of coverage during an active AOCC call

NOTE :	At present the core recommendations for AOCC in the case where the radio link is lost during an active call are vague. Does the mobile go on charging for a fixed period after radio link loss and continue as usual if radio link reestablishment occurs, or does charging stop? Input from people involved in charging for GSM is required.

Hence this test will be enhanced in the future when the requirements become clearer.

Purpose:

To verify that when the MS goes out of radio coverage area and an active call is dropped the ME has written the total charge up to that point in the call to the ACM field of the SIM.

Conformance Requirement(s):

When the MS goes out of radio coverage area during an active AOCC call midway through an AOC charging time interval (e7 or e2) and the call is dropped the ME shall have written the total charge up to the point in the call where the charging time interval last expired to the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 02.24.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

The cell simulated by the system simulator is then switched off to model the MS losing radio coverage. The MS shall have stored the correct amount on the SIM according to the e-parameters sent. [What happens on call reestablishment?]

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE
	 A13	MS -> SS	FACILITY	As default message except contains Facility IE
				with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	 B13	MS -> SS	CONNECT ACKNOWLEDGE

14			SS cell switched off y seconds after CAI 
				information sent by SS
				Failure of radio path, end of call (CEND) occurs
				and MS stops charging (ref. GSM 02.24 sec 2)
	15			SIM contents checked (either via MMI or by
				removing the SIM and using SIM reader). ACM
				shall have been incremented correctly.

NOTE :

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The cell being simulated by the SS is switched off after approximately y=90 seconds

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

10	55	1	10	0	0	10	30	30

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.2.4	ACMmax operation / Mobile Originating

Purpose:

1)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, any outgoing calls in progress for which a non-zero CAI exists are terminated by the ME with an appropriate indication given to the user.

2)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the making of non-emergency calls is inhibited

3)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the making of emergency calls is uninhibited.

Conformance Requirement(s):

1)	When the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, any outgoing calls in progress for which a non-zero CAI exists shall be terminated by the ME with an appropriate indication given to the user.

2)	When the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the making of further non-emergency calls shall be inhibited

3)	When the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the making of emergency calls shall be uninhibited.

Reference(s):

Conformance requirement 1:	TS GSM 02:24

Conformance requirement 2:	TS GSM 02.24

Conformance requirement 3:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Type of user indication when ACMmax exceeded

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

PIN 2 is entered into the MS allowing modification of both the ACM and ACMmax fields on the SIM. The ACM is reset to zero and the ACMmax is set to 2 units.

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC non-zero e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

After the ACM has been incremented to 2 units the call shall be terminated by the MS and an indication given to the user. The call duration is recorded and the ACM is checked to ensure it has been incremented to 2 units.

The MS is then made to attempt to originate an ordinary call to the MS for which a non-zero CAI exists for the calling party and shall be unsuccessful. The MS shall not send a CHANNEL REQUEST for that call.

The MS is then made to attempt to originate an emergency call and shall be successful.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,...,3.

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM (it shall be zero)

For k= 1
	A1	MS 		The MS is made to initiate an ordinary call
	A2	MS -> SS	CHANNEL REQUEST
	A3	SS -> MS	IMMEDIATE ASSIGNMENT
	A4	MS -> SS	CM SERVICE REQUEST
	A5	SS -> MS	CM SERVICE ACCEPT
	A6	MS -> SS	SETUP
	A7	SS -> MS	CALL PROCEEDING
	A8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	A9	MS -> SS	ASSIGNMENT COMPLETE
	 A10	SS -> MS	ALERTING

For k=4
	B1	MS 		The MS is made to initiate an emergency call 					(112 is entered)
	B2	MS -> SS	CHANNEL REQUEST	Establishment cause is emergency call 						establishment
	B3	SS -> MS	IMMEDIATE ASSIGNMENT
	B4	MS -> SS	CM SERVICE REQUEST	CM service type IE indicates "emergency call 					establishment"
	B5	SS -> MS	CM SERVICE ACCEPT
	B6	MS -> SS	EMERGENCY SETUP
	B7	SS -> MS	CALL PROCEEDING
	B8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	B9	MS -> SS	ASSIGNMENT COMPLETE
	 B10	SS -> MS	ALERTING
	B11	SS -> MS	CONNECT	As default message except contains Facility IE 					with contents as indicated in i below
				For k=1 or 3 either branch C or D is taken

C12	MS -> SS	CONNECT ACKNOWLEDGE
	 C13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

D12	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below
	 D13	MS -> SS	CONNECT ACKNOWLEDGE

For k=1
	 L14			Record call duration, x seconds, after CAI 					information sent by SS until call is terminated 					(when ACM=ACMmax)
	 L15	MS -> SS	DISCONNECT
	 L16	SS -> MS	RELEASE
	 L17	MS -> SS	RELEASE COMPLETE
	 L18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	 L19			ACM checked (shall be 2 units)

For k=3
	 M14			Call duration y seconds after CAI information 					sent by SS
	 M15	MS -> SS	DISCONNECT
	 M16	SS -> MS	RELEASE
	 M17	MS -> SS	RELEASE COMPLETE
	 M18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	 M19			ACM checked (shall be 2 units)

k=1 - Non zero CAI call attempted by MS and should succeed

k=2 - Non zero CAI call attempted by MS and should fail

k=3 - Emergency call attempted by MS and should succeed

NOTE :

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x shall be 60 ± 2 seconds

y shall be set to 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

1	1	30	1	0	0	0	0	2	2
		2	1	30	1	0	0	0	0	0	2
		3	0	0	0	0	0	0	0	0	2

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.2.5	ACMmax operation / Mobile Terminating

Purpose:

1)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, any mobile terminating calls in progress for which a non-zero CAI exists are terminated by the ME with an appropriate indication given to the user.

2)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, and an incoming call is received for which subsequently a non-zero CAI is received, then the call is terminated by the ME with an appropriate indication given to the user.

3)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the receiving of calls for which the CAI is zero is uninhibited.

Conformance Requirement(s):

1)	When the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, any mobile terminating calls in progress for which a non-zero CAI exists shall be terminated by the ME with an appropriate indication given to the user.

2)	When the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, and an incoming call is received for which subsequently a non-zero CAI is received, the call shall be terminated by the ME with an appropriate indication given to the user.

3)	When the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the receiving of calls for which the CAI is zero shall be uninhibited.

Reference(s):

Conformance requirement 1:	TS GSM 02:24.

Conformance requirement 2:	TS GSM 02.24

Conformance requirement 2:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Type of user indication when ACMmax exceeded

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

PIN 2 is entered into the MS allowing modification of both the ACM and ACMmax fields on the SIM. The ACM is reset to zero and the ACMmax is set to 2 units.

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The SS is made to initiate a call. The call is established with certain non-zero AOCC e-parameters sent in a Facility IE contained in a FACILITY message sent before the CONNECT message. It is an implementation option whether the AOCC e-parameters and AOCC acknowledge are sent before or after the CONNECT ACKNOWLEDGE.

After the ACM has been incremented to 2 units the call shall be terminated by the MS and an indication given to the user. The ACM shall be checked to ensure that it has been incremented to 2 units.

The SS is then made to attempt to make an ordinary call to the MS for which a non-zero CAI exists for the called party and shall be unsuccessful. The MS shall terminate the call with a DISCONNECT message.

The SS is then made to attempt to make an ordinary call to the MS for which a zero CAI exists for the called party and shall be successful.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,...,3.

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM
	1	SS 		The SS is made to initiate a call
	2	SS -> MS	PAGING REQUEST
	3	MS -> SS	CHANNEL REQUEST
	4	SS -> MS	IMMEDIATE ASSIGNMENT
	5	SS -> MS	PAGING RESPONSE
	6	MS -> SS	SETUP
	7	SS -> MS	CALL CONFIRMED
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	MS -> SS	CONNECT
	12	SS -> MS	CONNECT ACKNOWLEDGE

For k=1 or 3 branch A is taken

A13	SS -> MS	FACILITY	As default message except contains Facility IE 					with contents as indicated in i below
	 A14	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

For k=2 either branch B,C or D is taken

B13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below
	 B14	MS -> SS	DISCONNECT	MS terminates call

C13	MS -> SS	DISCONNECT	MS terminates call
	 C14	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

D13	MS -> SS	DISCONNECT	MS terminates call.
				DISCONNECT may or may not contain Facility 					IE with contents as indicated in ii below

For k= 1, 2 and 3, D, E and F branch shall be 					taken respectively

For k=1
	 D15			Record call duration, x seconds, after CAI 					information sent by SS until call is terminated 					(when ACM=ACMmax)
	 D16	MS -> SS	DISCONNECT	MS terminates call
	 D17	SS -> MS	RELEASE
	 D18	MS -> SS	RELEASE COMPLETE
	 D19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For k=2
	 E15	SS -> MS	RELEASE
	 E16	MS -> SS	RELEASE COMPLETE	RELEASE COMPLETE may or may not contain 					Facility IE with contents as indicated in ii 					below
	 E17	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For k=3
	 F15			Call duration y seconds after CAI information 					sent by SS
	 F16	MS -> SS	DISCONNECT	MS terminates call
	 F17	SS -> MS	RELEASE
	 F18	MS -> SS	RELEASE COMPLETE
	 F19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

20			ACM checked (should be 2 units)

k=1 - Non zero CAI call attempted to MS and should succeed

k=2 - Non zero CAI call attempted to MS and should fail

k=3 - Zero CAI call attempted to MS and should succeed

NOTE :

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x shall be 60 ± 2 seconds

y shall be set to 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

1	1	30	1	0	0	0	0	2	2
		2	1	30	1	0	0	0	0	0	2
		3	0	0	0	0	0	0	0	0	2

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

### 31.6.3	Advice of Charge Information

[Text to be added once AOCC tests are completed.]

### 31.6.4	Default contents of messages

As default message contents in GSM 11.10 Section 26.8 "Tests related to circuit switched call control". These messages shall not contain SS version IEs.

Where indicated in specific tests CONNECT and FACILITY messages have Facility Information Elements as defined below.

Default contents of ForwardChargeAdvice Facility Information Elements

The following abbreviations are used in the descriptions below:

U	Universal tag class

CS	Context Specific tag class

P	Primitive tag form

C	Constructed tag form

FIE	Facility Information Element

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00011100

Length of FIE contents	43	00101011

Component type tag 	CS/C/tag=1	10100001

Component length	41	00101001

Invoke ID tag	U/P/tag=2	00000010

Invoke ID length	1	00000001

Invoke ID	Arbitrary 	(00000000)			(1 octet)

Op-Code tag	From GSM 04.80	00000010

Op-Code length	1	00000001

Op-Code	Forward Charge	01111101
	Advice operation 					(local value 125)

Sequence Identifier	U/C/tag=16 	00110000			from CCITT X.208

Length Indicator	33	00100001

SS-Code Identifier	CS/P/tag=0	10000000

Length Indicator	1	00000001

SS-Code	AOCC SS-Code	01110010

Charging Info. identifier	CS/C/tag=1	10100001

Length Indicator	28	00011100

e1 Identifier	CS/P/tag=1	10000001

Length Indicator	2	00000010

e1	(2 Octets)	See e-parameter 			table in relevant test

e2 Identifier	CS/P/tag=2	10000010

Length Indicator	2	00000010

e2	(2 Octets)	See e-parameter 			table in relevant test

.	.	.

.	.	.

.	.	.

e7 Identifier	CS/P/tag=7	10000111

Length Indicator	2	00000010

e7	(2 Octets)	See e-parameter 			table in relevant test

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00111100

Length of FIE contents	5	00001001

Component type tag 	CS/C/tag=2	10100010

Component length	3	00000011

Invoke ID tag	U/P/Integer	00000010

Invoke ID length	1	00000001

Invoke ID	Same as used as	(00000000)		                                          Invoke ID in Invoke FIE

## 31.7	Additional information transfer supplementary services

(Reserved).

## 31.8	Call restriction supplementary services

The following abbreviations are used

BO :	Barring of Outgoing calls

BAOC :	Barring of All Outgoing Calls

BOIC :	Barring of Outgoing International Calls

BOICExHC :	Barring of Outgoing International Call EXcept those directed to the Home PLMN country

BI :	Barring of Incoming calls

BAIC :	Barring of All Incoming calls

BICRoam :	Barring of Incoming when Roaming outside the home PLMN country

B :	Barring (common name for BAOC, BOIC, BOICExHC, BAIC and BICRoam).

These abbreviations are also used to represent the corresponding SS-Code; e.g. B is the SS-Code for all barring services.

NOTE:	The password(s) to be used during tests of this section 31.8 may be randomly chosen - unless otherwise stated - in accordance with GSM 02.04 section 5.2.

### 31.8.1	Registration of a password

#### 31.8.1.1	Registration accepted

##### 31.8.1.1.1	Conformance requirements

1)	For registration of a password for all barring services, the MS shall transmit successively

1.1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

1.2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

1.3)	and then the REGISTER message containing a facility IE (or an empty REGISTER followed by a FACILITY message containing the facility IE) that includes an invoke of the RegisterPassword operation with parameter values according to the user's request (MMI action).

2)	Upon receipt of the different FACILITY messages part of the procedure of registration of a password for all barring services, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

3)	The MS shall be able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

4)	Upon receipt of the result of the procedure, contained in RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1.1)	GSM 04.08 sections 3.3.1.2, 9.1.9

1.2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

1.3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

2)	GSM 02.30 section 4.5

3)	GSM 04.80

4)	GSM 02.30

##### 31.8.1.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for registration of a password for all barring services in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for registration of a password for all barring services in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of a password for all barring services.

4)	To check that upon receipt of the different FACILITY messages part of the procedure of registration of a password for all barring services, the MS provides the appropriate user indication (as described by the manufacturer).

5)	To check that the MS is able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

6)	To check that upon receipt of the result of the procedure, contained in RELEASE COMPLETE or FACILITY message, the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for:

all barring services, the result of the operation being sent in a RELEASE COMPLETE message.

##### 31.8.1.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of a new password for all barring services.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the old password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring a new password.

Then, by means of appropriate MMI functions, the user gives a new password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring again the new password.

Then, by means of appropriate MMI functions, the user gives again the new password.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the RegisterPassword operation.

Maximum duration of test

3 minutes

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of a password for all call
				barring services
	2	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	Invoke = GetPassword (password)
	9	MS		provide correct MMI user. indication.
				A password is entered.
	10	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	11	SS -> MS	FACILITY 	Invoke = GetPassword (new password)
	12	MS		provide correct MMI user indication
				A password is entered.
	13	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	14	SS -> MS	FACILITY 	Invoke = GetPassword (new password again)
	15	MS		provide correct MMI user indication
				A password is entered.
	16	MS -> SS	FACILITY 	 GetPassword operation ReturnResult
	17	SS -> MS	RELEASE COMPLETE	RegisterPassword operation ReturnResult
	18	SS -> MS	CHANNEL RELEASE

Specific message content

step A6 or B7 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = RegisterPassword

Supplementary service code = B

steps 8, 11, and 14 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : FACILITY

-	facility

invoke = GetPassword

Supplementary service code = B

Guidance info : Password (step 8), new password (step 11), new password again (step 14)

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

#### 31.8.1.2	Registration rejected

##### 31.8.1.2.1	Rejection after invoke of the RegisterPassword operation

###### 31.8.1.2.1.1	Conformance requirements

1)	A transaction of any kind being already established, for registration of a password for all call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.8.1.2.1.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of registration of a password for all call restriction services, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of a password for all call restriction services.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for:

all call restriction services, the RELEASE COMPLETE message being sent at the beginning of the procedure with a facility IE containing a return_error(error) where error is "SS subscription violation".

###### 31.8.1.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of a new password for all call restriction services.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the RELEASE COMPLETE (PD and TI of the SS transaction) message with the Facility information element containing a Return_error(error : SS subscription violation) of the RegisterPassword operation.

Upon receipt of the FACILITY message, the system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

2 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a registration of a new password
				 for all call restriction services
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	RegisterPassword operation Return_error
	7	MS		provide correct MMI user indication
	8	SS -> MS	STATUS ENQUIRY
	9	MS -> SS	STATUS	CCstate U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = RegisterPassword

Supplementary service code = all call restrictions

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility:

return error code : SS subscription violation

For the return error the invoke ID must be the same as in the invoke of the RegisterPassword operation.

##### 31.8.1.2.2	Rejection after password check with negative result

###### 31.8.1.2.2.1	Conformance requirements

1)	A transaction of any kind being already established, for registration of a password for all call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.8.1.2.2.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of registration of a password for all call restriction services, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of a password for all call restriction services.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for :

all call restriction services, the RELEASE COMPLETE message being sent at the end of the procedure with a facility IE containing a return_error(error) where error is "NegativePasswordCheck".

###### 31.8.1.2.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of a new password for all call restriction services.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the old password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the new password.

Then, by means of appropriate MMI functions, the user gives a new password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring again the new password.

Then, by means of appropriate MMI functions, the user gives again the new password.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE message (PD and TI of the SS transaction) with the Facility information element containing a Return_error(error : NegativePasswordCheck) of the RegisterPassword operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

5 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of a new password
				 for all call restriction services
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	FACILITY 	Invoke = GetPassword (password)
	7	MS		provide correct MMI user indication
				A password is entered.
	8	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	9	SS -> MS	FACILITY 	Invoke = GetPassword (new password)
	10	MS		provide correct MMI user indication
				A password is entered.
	11	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	12	SS -> MS	FACILITY 	Invoke = GetPassword (new password again)
	13	MS		provide correct MMI user indication
				A password is entered.
	14	MS -> SS	FACILITY 	 GetPassword operation ReturnResult
	15	SS -> MS	RELEASE COMPLETE	Register Password operation ReturnError
	16	MS		provide correct MMI user indication
	17	SS -> MS	STATUS ENQUIRY
	18	MS -> SS	STATUS	CCstate U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = RegisterPassword

Supplementary service code = B

step 15 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility:

error code : NegativePasswordCheck

For the reject the invoke ID must be the same as in the invoke of the Registerpassword operation.

steps 6, 9, 11 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type : FACILITY

-	facility

invoke = GetPassword

Supplementary service code = B

Guidance info : Password (step 6), new password (step 9), new password again (step 11)

##### 31.8.1.2.3	Rejection after new password mismatch

###### 31.8.1.2.3.1	Conformance requirements

1)	A transaction of any kind being already established, for registration of a password for all call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.8.1.2.3.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of registration of a password for all call restriction services, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of a password for all call restriction services.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for :

all call restriction services, the RELEASE COMPLETE message being sent at the end of the procedure with a facility IE containing a return_error(error) where error is "PasswordRegistrationFailure" with diagnostic "new password mismatch".

###### 31.8.1.2.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of a new password for all call restriction services.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the old password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the new password.

Then, by means of appropriate MMI functions, the user gives a new password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring again the new password.

Then, by means of appropriate MMI functions, the user gives again a new password.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE message (PD and TI of the SS transaction) with the Facility information element containing a Return_error(error : PasswordRegistrationFailure, parameter : NewPasswordMismatch) of the RegisterPassword operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of a new password
				 for all call restriction services
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	FACILITY 	Invoke = GetPassword (password)
	7	MS		provide correct MMI user indication
				A password is entered.
	8	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	9	SS -> MS	FACILITY 	Invoke = GetPassword (new password)
	10	MS		provide correct MMI user indication
				A password is entered.
	11	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	12	SS -> MS	FACILITY 	Invoke = GetPassword (new password again)
	13	MS		provide correct MMI user indication
				A password is entered.
	14	MS -> SS	FACILITY 	 GetPassword operation ReturnResult
	15	SS -> MS	RELEASE COMPLETE	Register Password operation ReturnError
	16	MS		provide correct MMI user indication
	17	SS -> MS	STATUS ENQUIRY
	18	MS -> SS	STATUS	CCstate U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke =	RegisterPassword

Supplementary service code = B

step 15 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility:

error code : PasswordRegistrationFailure

parameter : NewPasswordMismatch

For the reject the invoke ID must be the same as in the invoke of the Registerpassword operation.

steps 6, 9, 11 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type : FACILITY

-	facility

invoke =	GetPassword

Supplementary service code = 	B

Guidance info : Password (step 6), new password (step 9), new password again (step 11)

### 31.8.2	Erasure

Not applicable

### 31.8.3	Activation

#### 31.8.3.1	Activation accepted

##### 31.8.3.1.1	Conformance requirements

1)	For activation of any specific call restriction service with any parameters, the MS shall transmit successively

1.1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

1.2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

1.3)	and then the REGISTER message containing a facility IE that includes an invoke of the ActivateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

2) Upon receipt of FACILITY message requiring the password, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

3) The MS shall be able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

4) Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

##### 31.8.3.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for activation of a specific call restriction service in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for activation of call restriction service in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for activation of a specific call restriction service.

4)	To check that upon receipt of FACILITY message requiring the password, the MS provides the appropriate user indication (as described by the manufacturer).

5)	To check that the MS is able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

6)	To check that upon receipt of the RELEASE COMPLETE or FACILITY message, the MS provides the appropriate user indication (which is to be described by the manufacturer).

These checks are done for:

a)	BAOC, for basic service group "all synchrounous services" the result of the operation being sent in a FACILITY message.

b)	BICRoam, for all basic service groups, the result of the operation being sent in a RELEASE COMPLETE message.

##### 31.8.3.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests activation of BAOC, for basic service group "all synchrounous services".

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing the Return_result of the ActivateSS operation.

The SS transaction and the dedicated channel are released.

Then again, by means of appropriate MMI functions, the user requests activation of BICRoam, for all basic service groups.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing the Return_result of the ActivateSS operation.

The dedicated channel is released.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a activation of BAOC(all synchronous services)
	2	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	GetPassword
	9	MS		provide correct MMI user indication
				A password is entered.
	10	MS -> SS	FACILITY 	Getpassword operation Return Result
	11	SS -> MS	FACILITY 	ActivateSS operation Return_result
	12	SS -> MS	RELEASE COMPLETE
	13	SS -> MS	CHANNEL RELEASE

14	MS 		The MS is made to initiate an activation
				of BICRoam(all basic service groups),
	15	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	16	SS -> MS	IMMEDIATE ASSIGNMENT
	17	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	18	SS -> MS	CM SERVICE ACCEPT

A19	MS -> SS	REGISTER

B19	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B20	MS -> SS	FACILITY

21	SS -> MS	FACILITY 	GetPassword
	22	MS		provide correct MMI user indication
				A password is entered.
	23	MS -> SS	FACILITY 	Getpassword operation Return Result
	24	SS -> MS	RELEASE COMPLETE	ActivateSS operation Return result
	25	MS		provide correct MMI user indication
	26	SS -> MS	CHANNEL RELEASE

Specific message contents

step A6 or B7 - BAOC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Activation

Supplementary service code = BAOC

Basic service code : Bearer Service (all synchronous services), no teleservice present.

step A16 or B17 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B19

-	message type : REGISTER (A16), FACILITY (B20)

-	facility

invoke = Activation

Supplementary service code = BICRoam

Basic service code : no bearer service present, no teleservice present.

steps 8, 21 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type : FACILITY

-	facility

invoke = GetPassword

Supplementary service code = all call restrictions

Guidance info : Password

#### 31.8.3.2	Activation rejected

##### 31.8.3.2.1	Rejection after invoke of ActivateSS operation

###### 31.8.3.2.1.1	Conformance requirements

1)	A transaction of any kind being already established, for activation of one specific barring services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.8.3.2.1.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of specific call barring service, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for specific call barring service.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for :

BOIC, the RELEASE COMPLETE messagebeing sent at the beginning of the procedure with a facility IE containing a return_error(error) where error is "SS subscription violation".

###### 31.8.3.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests activation of BOIC.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the RELEASE COMPLETE (PD and TI of the SS transaction) message with the Facility information element containing a Return_error(error : SS subscription violation) of the ActivateSS operation.

Upon receipt of the FACILITY message, the system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

2 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of a new password
				 for BOIC
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	RegisterPassword operation Return_error
	7	MS		provide correct MMI user indication
	8	SS -> MS	STATUS ENQUIRY
	9	MS -> SS	STATUS	CCstate U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke =	ActivateSS

Supplementary service code = BOIC

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : SS subscription violation

For the return error the invoke ID must be the same as in the invoke of the ActivateSS operation.

##### 31.8.3.2.2	Rejection after use of password procedure

###### 31.8.3.2.2.1	Conformance requirements

1)	A transaction of any kind being already established, for activation of any specific call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.8.3.2.2.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of activation of one specific call restriction service, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for activation of one specific call restriction service.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for :

BAIC, the RELEASE COMPLETE message being sent at the end of the procedure with a facility IE containing a return_error(error) where error is "NegativePasswordCheck".

###### 31.8.3.2.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of a new password for all call restriction services.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator sends RELEASE COMPLETE message (PD and TI of the SS transaction) with the Facility information element containing a Return_error(error : NegativePasswordCheck) of the GetPassword operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of BAIC
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	FACILITY 	Invoke = GetPassword (password)
	7	MS		provide correct MMI user indication
				A password is entered.
	8	SS -> MS	FACILITY	GetPassword operation Return Result
	9	SS -> MS	RELEASE COMPLETE	Register Password operation ReturnError
	10	MS		provide correct MMI user indication
	11	SS -> MS	STATUS ENQUIRY
	12	MS -> SS	STATUS	CCstate U10

Specific message content

step A4 or B5 - BAIC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke =	Activation

Supplementary service code = BAIC

step 6 - All Barring services

-	protocol discriminator: non call related SS message

-	transaction identifier: in FACILITY same as previous REGISTER message

-	message type: FACILITY

-	facility

invoke = GetPassword

Supplementary service code = B

Guidance info: Password (step 6)

step 9 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : NegativePasswordCheck

For the reject the invoke ID must be the same as in the invoke of the GetPassword operation.

### 31.8.4	Deactivation

#### 31.8.4.1	Deactivation accepted

##### 31.8.4.1.1	Conformance requirements

1)	For deactivation of any group of call restriction services with any parameters, the MS shall transmit successively

1.1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

1.2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

1.3)	and then the REGISTER message containing a facility IE that includes an invoke of the DeactivateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

2)	Upon receipt of FACILITY message requiring the password, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

3)	The MS shall be able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

3)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

##### 31.8.4.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for deactivation of a group of call barring services in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for deactivation of a group of call barring services in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for deactivation of a goup of call restriction services.

4)	To check that upon receipt of FACILITY message requiring the password, the MS provides the appropriate user indication (as described by the manufacturer).

5)	To check that the MS is able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

6)	To check that upon receipt of the RELEASE COMPLETE or FACILITY message, the MS provides the appropriate user indication (which is to be described by the manufacturer).

These checks are done for:

a)	all restrictions, for basic service group "speech" the result of the operation being sent in a FACILITY message.

b)	barring of outgoing calls, for all facsimile, the result of the operation being sent in a RELEASE COMPLETE message.

##### 31.8.4.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests deactivation of all restrictions, for speech.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing the Return_result of the DeactivateSS operation.

The SS transaction and the dedicated channel are released.

Then again, by means of appropriate MMI functions, the user requests activation of barring of outgoing calls, for all facsimile.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing the Return_result of the DeactivateSS operation.

The dedicated channel is released.

Maximum duration of test

5 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a deactivation of all call restrictions(speech)
	2	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	GetPassword
	9	MS		provide correct MMI user indication
				A password is entered.
	10	MS -> SS	FACILITY 	Getpassword operation Return Result
	11	SS -> MS	FACILITY 	DeactivateSS operation Return_result
	12	SS -> MS	RELEASE COMPLETE
	13	SS -> MS	CHANNEL RELEASE

14	MS 		The MS is made to initiate
				a deactivation of barring of outgoing
				calls(all facsimile),
	15	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	16	SS -> MS	IMMEDIATE ASSIGNMENT
	17	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	18	SS -> MS	CM SERVICE ACCEPT

A19	MS -> SS	REGISTER

B19	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B20	MS -> SS	FACILITY

21	SS -> MS	FACILITY 	GetPassword
	22	MS		provide correct MMI user indication
				A password is entered.
	23	MS -> SS	FACILITY 	Getpassword operation Return Result
	24	SS -> MS	RELEASE COMPLETE	DeactivateSS operation Return result
	25	MS		provide correct MMI user indication
	26	SS -> MS	CHANNEL RELEASE

Specific message contents

step A6 or B7 - all call restrictions:

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Deactivation

Supplementary service code = B

Basic service code : no Bearer Service present, teleservice : speech.

step A16 or B17 - barring of outgoing calls,

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Deactivation

Supplementary service code = BO

Basic service code : no bearer service present, teleservice : all facsimile.

steps 8, 21 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type : FACILITY

-	facility

invoke = GetPassword

Supplementary service code = all call restrictions

Guidance info : Password

#### 31.8.4.2	Deactivation rejected

##### 31.8.4.2.1	Rejection after invoke of DeactivateSS operation

###### 31.8.4.2.1.1	Conformance requirements

1)	A transaction of any kind being already established, for deactivation of a group of call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.8.4.2.1.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of a group of call barring services, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for a group of call barring services.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

These checks are performed with a call transaction already established for :

BOIC, the RELEASE COMPLETE messagebeing sent at the beginning of the procedure with a facility IE containing a return_error(error) where error is "SS subscription violation".

###### 31.8.4.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests deactivation of incoming calls.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the RELEASE COMPLETE (PD and TI of the SS transaction) message with the Facility information element containing a Return_error(error : SS subscription violation) of the DeactivateSS operation.

Upon receipt of the FACILITY message, the system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

30 s

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a deactivation
				 for bi
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	DeactivateSS operation Return_error
	7	MS		provide correct MMI user indication
	8	SS -> MS	STATUS ENQUIRY
	9	MS -> SS	STATUS	CCstate U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = DeactivateSS

Supplementary service code = bi

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : SS subscription violation

For the return error the invoke ID must be the same as in the invoke of the DeactivateSS operation.

##### 31.8.4.2.2	Rejection after use of password procedure

###### 31.8.4.2.2.1	Conformance requirements

1)	A transaction of any kind being already established, for deactivation of a group of call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

###### 31.8.4.2.2.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of deactivation of a group of call restriction services, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for deactivation of a group of call restriction service.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for :

BOICExHC, the RELEASE COMPLETE message being sent at the end of the procedure with a facility IE containing a return_error(error) where error is "NegativePasswordCheck".

###### 31.8.4.2.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests deactivation of a group of call restriction services.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the DeactivateSS operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator sends RELEASE COMPLETE message (PD and TI of the SS transaction) with the Facility information element containing a Return_error(error : NegativePasswordCheck) of the GetPassword operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a deactivation of BoicExHC
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	FACILITY 	Invoke = GetPassword (password)
	7	MS		provide correct MMI user indication
				A password is entered.
	8	SS -> MS	RELEASE COMPLETE	Register Password operation ReturnError
	9	MS		provide correct MMI user indication
	10	SS -> MS	STATUS ENQUIRY
	11	MS -> SS	STATUS	CCstate U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = DeactivateSS

Supplementary service code = BOICExHC

step 11 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : NegativePasswordCheck

For the reject the invoke ID must be the same as in the invoke of the DeactivateSS operation.

step 6 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type :FACILITY

-	facility

invoke = GetPassword

Supplementary service code =

Guidance info : Password (step 6)

### 31.8.5	Invocation

Invocation is not applicable.

### 31.8.6	Interrogation

#### 31.8.6.1	Interrogation accepted

##### 31.8.6.1.1	Conformance requirements

1)	For interrogation of any specific call restriction service with any parameters, the MS shall transmit successively

1.1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

1.2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

1.3)	and then the REGISTER message containing a facility IE that includes an invoke of the InterrogateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

2)	Upon receipt of FACILITY message requiring the password, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

3)	The MS shall be able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

4)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

##### 31.8.6.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for interrogation of a specific call barring service in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for interrogation of a call barring service in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for interrogation of one call restriction service.

4)	To check that upon receipt of FACILITY message requiring the password, the MS provides the appropriate user indication (as described by the manufacturer).

5)	To check that the MS is able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

6)	To check that upon receipt of the RELEASE COMPLETE or FACILITY message, the MS provides the appropriate user indication (which is to be described by the manufacturer).

These checks are done for :

a)	BAIC, the result of the operation being a Basic Service code sent in a FACILITY message.

b)	BOICExHC, the result of the operation being a SS-status sent in a RELEASE COMPLETE message.

##### 31.8.6.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests interrogation of BAIC.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing the Return_result (basic service) of the InterrogateSS operation.

The SS transaction and the dedicated channel are released.

Then again, by means of appropriate MMI functions, the user requests activation of BOICExHC.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing the Return_result(SS-status) of the InterrogateSS operation.

The dedicated channel is released.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a interrogation of BAIC
	2	MS -> SS	CHANNEL REQUEST	with establisment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	GetPassword
	9	MS		provide correct MMI user indication
				A password is entered.
	10	MS -> SS	FACILITY 	Getpassword operation Return Result
	11	SS -> MS	FACILITY 	InterrogateSS operation Return_result
	12	SS -> MS	RELEASE COMPLETE
	13	SS -> MS	CHANNEL RELEASE

14	MS 		The MS is made to initiate
 				a interrogation of call forwarding service
				for BOICExHC,
	15	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	16	SS -> MS	IMMEDIATE ASSIGNMENT
	17	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	18	SS -> MS	CM SERVICE ACCEPT

A19	MS -> SS	REGISTER

B19	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B20	MS -> SS	FACILITY

21	SS -> MS	FACILITY 	GetPassword
	22	MS		provide correct MMI user indication
				A password is entered.
	23	MS -> SS	FACILITY 	Getpassword operation Return Result
	24	SS -> MS	RELEASE COMPLETE	InterrogateSS operation Return result
	25	MS		provide correct MMI user indication
	26	SS -> MS	CHANNEL RELEASE

Specific message contents

step A6 or B7 - BOIC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Interrogation

Supplementary service code = BAIC

step A16 or B17 - BOICExHC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Interrogation

Supplementary service code = BOICExHC

steps 8, 21 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type : FACILITY

-	facility

invoke = GetPassword

Supplementary service code = B

Guidance info : Password

#### 31.8.6.2	Interrogation rejected

##### 31.8.6.2.1	Conformance requirements

1)	A transaction of any kind being already established, for interrogation of any specific call barring with any parameters, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

##### 31.8.6.2.2	Test purpose

1)	To check that the MS correctly requests the establishemnt of a parallel MM transaction for supplementary service transaction of interrogation of a specific call barring service message, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for interrogation of call barring.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

These checks are performed with a call transaction already established for :

a)	BICRoam, the RELEASE COMPLETE message being sent with a facility IE containing a return_error(error) where error is "SS not available".

b)	BOIC, the RELEASE COMPLETE message being sent with a facility IE containing a reject(invoke_problem) where invoke_problem is "resource limitation".

##### 31.8.6.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests interrogation of BICRoam.

Upon receipt of the operation (in a REGISTER or a FACILITY message),the system simulator answers with the FACILITY message with the Facility information element containing a Return_error(error : SS not available) of the InterrogateSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Then again, by means of appropriate MMI functions, the user requests interrogation of BOIC.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message (same PD and TI that in the REGISTER message) with the Facility information element containing a reject(invoke_problem : resource limitation) of the InterrogateSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a interrogation of call barring service
				for BICRoam
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	FACILITY

6	SS -> MS	RELEASE COMPLETE 	InterrogateSS operation Return_error
	7	SS -> MS	STATUS ENQUIRY
	8	MS -> SS	STATUS	CCstate U10

9	MS 		The MS is made to initiate
				a interrogation of call barring service
				for BOIC
	10	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	11	SS -> MS	CM SERVICE ACCEPT

A12	MS -> SS	REGISTER
	B12	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B13	FACILITY

14	SS -> MS	RELEASE COMPLETE 	(SS)
	15	MS		provide correct MMI user indication
	16	SS -> MS	STATUS ENQUIRY
	17	MS -> SS	STATUS	CCstate U10

Specific message content

step A4 or B5 - BICRoam

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = Interrogation

Supplementary service code = BICRoam

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : SS not available

For the return error the invoke ID must be the same as in the invoke of the InterrogateSS operation.

step A12 or B13 - BOIC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A12), FACILITY (B13)

-	facility

invoke = Interrogation

Supplementary service code = BOIC

step 14 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : resource limitation

For the reject the invoke ID must be the same as in the invoke of the InterrogateSS operation.

### 31.8.7	Normal operation

In case of barring of outgoing call the calling mobile receives information about the activation of supplementary services subscribed.

In case of barring of incoming call the calling mobile receives information about the activation of supplementary services subscribed by the other party (the mobile called).

#### 31.8.7.1	Conformance requirements

Upon receipt of the RELEASE COMPLETE message the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

#### 31.8.7.2	Test purpose

To check that upon receipt of the RELEASE COMPLETE message the MS provides the appropriate user indication (as described by the manufacturer).

This is tested in the case of barring of incoming calls.

#### 31.8.7.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

The MS is paged.

Upon receipt of the SETUP message, the system simulator answers either with the negative acknowledgment RELEASE COMPLETE (to simulate a case where call barring is activated).

Maximum duration of test

1 mn

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	SETUP
	6	SS -> MS 	RELEASE COMPLETE	provide correct MMI user indication
	7	SS -> MS	CHANNEL RELEASE

Specific message content

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke = notification

SS code = BI

SS status = activation indicator (indicating :

Provisioned, registered and active)

## 31.9	Handling of undefined (future) GSM supplementary services

### 31.9.1	Mobile station initiated Unstructured supplementary service data operation

#### 31.9.1.1	ProcessUnstructuredSS-request/accepted

Conformance requirements

1)	The mobile station shall invoke an USSD request by sending a REGISTER message to the network containing a ProcessUnstructuredSS-Request invoke component. This message will contain the alphabet indicator set to "SMS default alphabet" and the langage indicator set to "language unspecified". The ussd-string parameter shall contain the following digits and symbols depending on the operation initiated :

Activation			*NN(N)# (no supplementary information included)

*NN(N)*SI# (one field of supplementary infor. included)

*NN(N)*SIA*SIB# (two fiels of supplementary infor. included)

Deactivation			#NN(N)#

#NN(N)*SI#

#NN(N)*SIA*SIB#

Interrogation		*#NN(N)#

*#NN(N)*SI#

*#NN(N)*SIA*SIB#

Registration			**NN(N)#

**NN(N)*SI#

**NN(N)*SIA*SIB#

Erasure			##NN(N)#

##NN(N)*SI#

##NN(N)*SIA*SIB#

Operations not yet		see conformance requirement 2)

defined in GSM 02.30

NN(N) features a set of service codes which have not yet been allocated for GSM supplementary services (see GSM 02.30 for service codes already specified).

N is a digit within 1..9 and SI, SIA, SIB strings of characters.

2) 	Concerning operations which are not yet specified in GSM 02.30, the MS shall proceed as follows:

The entry of 1 or 2 characters defined in the GSM 03.38 default alphabet followed by "SEND" shall be interpreted by the MS as an USSD request unless the MS is not engaged in a call and the first of the two character entry followed by "SEND" is a "1".

3)	For supplementary service procedures independent of any call, the initiating side shall  establish a MM-connection between the network and the mobile station according to the rules given in TS GSM 4.08.

4)	Within a call, the MS shall transmit a USSD request from the user if any. See TS GSM 04.07 and 04.08 for the handling of multiple MM connections.

5)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall display the information contained to the user in a way described by the manufacturer.

References

Conformance requirement 1 : TS GSM 04.90 subclause 6.1, TS GSM 2.30 subclause 4.5.2, and TS GSM 02.90 subclause 4.1.1

Conformance requirement 2 : TS GSM 02.30 subclause 4.5.3.2

Conformance requirement 3 : TS GSM 4.10 subclause 3.2.1

Conformance requirement 4 : TS GSM 4.08 subclause 4.5.1.1

Conformance requirement 5 : TS GSM 3.90 subclause 6.2.1

Test Purpose

1)	To verify that the mobile station  invokes an USSD request by sending a REGISTER message to the network containing a ProcessUnstructuredSS-Request invoke component. This message will contain the alphabet indicator set to "SMS default alphabet" and the langage indicator set to "language unspecified". The ussd-string parameter shall contain the following digits and symbols depending on the operation initiated :

Activation			*NN(N)# (no supplementary information included)

*NN(N)*SI# (one field of supplementary infor. included)

*NN(N)*SIA*SIB# (two fiels of supplementary infor. included)

Deactivation			#NN(N)#

#NN(N)*SI#

#NN(N)*SIA*SIB#

Interrogation		*#NN(N)#

*#NN(N)*SI#

*#NN(N)*SIA*SIB#

Registration			**NN(N)#

**NN(N)*SI#

**NN(N)*SIA*SIB#

Erasure			##NN(N)#

##NN(N)*SI#

##NN(N)*SIA*SIB#

Operations not yet		see 2)

defined in GSM 02.30

NN(N) features a set of service codes which have not yet been allocated for GSM supplementary services (see GSM 02.30 for service codes already specified).

N is a digit within 1..9 and SI, SIA, SIB strings of characters.

2) 	To check that the entry of 1 or 2 characters defined in the GSM 03.38 default alphabet followed by "SEND" shall be interpreted by the MS as an USSD request unless the MS is not engaged in a call and the first of the two character entry followed by "SEND" is a "1".

3)	To verify that, for supplementary service procedures independent of any call, the initiating side must establish a MM-connection between the network and the mobile station according to the rules given in TS GSM 4.07 and 4.08.

4)	To verify that, within a call the MS shall transmit a USSD request if any. See TS GSM 04.07 and 04.08 for the handling of multiple MM connections.

5)	To check that upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall display the information contained to the user in a way described by the manufacturer.

Related PICS/PIXIT Statements

- support of USSD

- Description of the user's commands and of display of USSD

- Full rate supported

- Supported teleservices

- Support of active state of the call control protocol (U10)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle updated".

Foreseen final state of the MS

MM-state "idle updated".

Test Procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI) the mobile is made to initiate an Unstructured SS data operation. The mobile first establishes a MM-connection with the SS. Then, a REGISTER message is sent to the SS. Upon receipt of this message, the system simulator answers with the RELEASE COMPLETE or FACILITY message. If the FACILITY message is used, the mobile station sends a RELEASE COMPLETE message. Then  a CHANNEL RELEASE message is sent to the MS to release the main signalling link.

The mobile station is forced to originate a call. After the MS has received a CONNECT ACKNOWLEDGE, MMI keys are depressed on the mobile in order to initiate an Unstructured SS data operation. Then a REGISTER message is sent to the SS. Upon receipt of this message, the system simulator answers with the RELEASE COMPLETE or FACILITY message. If the FACILITY message is used, the mobile station sends a RELEASE COMPLETE message.  Finally, the main signalling link is released by transferring a CHANNEL RELEASE message to the MS.

Expected Sequence

The sequence is executed for execution counters c=1,..,17 and k=1,2.

Counter c  determines the ussd-string selected by the user and sent by the MS (see specific message contents concerning the REGISTER message).

Counter k   determines if the FACILITY message sent by the network includes information to the user (see specific message contents on FACILITY message).

Step	Direction	Message	Comments

1	MS		The user presses appropriate MMI keys to
				initiate the desired unstructured SS data
				operation.

2	MS -> SS	CHANNEL REQUEST	with establishment cause set to "Other
				procedures which can be completed with an
				SDCCH".
	3	SS -> MS	IMM. ASSIGN
	4	MS -> SS	CM SERVICE REQUEST 	"Supplementary service activation".
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	REGISTER	The SS checks that the content of this message
				matches specific message content i).

either A or B branch is taken by the SS. Each of
				one will have to be run through at least once
				during the test.

A7	SS -> MS	FACILITY	See specific message contents ii).It is checked
				that ussd string,if any, is displayed by the MS
				in a way described by the manufacturer.
	A8	MS -> SS	RELEASE COMPLETE	END key is pressed on the mobile keypad. It
				terminates the transaction used to initiate
				the desired undefined supplementary service
				data operation.
				See specific message contents iii).

B7	SS -> MS	RELEASE COMPLETE	It terminates the transaction used to initiate
				the desired unstructured SS data operation.It
				is checked that the ussd string, if any, is
				displayed by the MS in a way described by the
				manufacturer. See iii).

9	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

10	MS -> SS	CHANNEL REQUEST	with establishment cause related to mobile
				originating call.
	11	SS -> MS	IMMEDIATE ASSIGNMENT	a TCH/F channel is assigned to the MS.
	12	MS -> SS	CM SERVICE REQUEST	"mobile originating call"
	13	SS -> MS	CM SERVICE ACCEPT
	14	MS -> SS	SETUP
	15	SS -> MS	CALL PROCEEDING
	16	SS -> MS	ALERTING
	17	SS -> MS	CONNECT
	18	MS -> SS	CONNECT ACKNOWLEDGE

19			Specific MMI keys are depressed to initiate the
				desired undefined SS service operation. DTMF
				signalling may occur.

20	MS -> SS	CM SERVICE REQUEST	"Supplementary service activation".
	21	SS -> MS	CM SERVICE ACCEPT
	22	MS -> SS	REGISTER	The MS starts the transaction on the radio
				interface. The SS checks that REGISTER
				matches i).

either A or B branch is taken by the SS. Each
				branch will have to be run through at least
				once during the test.

A23	SS -> MS	FACILITY	See specific message contents ii).It is checked
				that the ussd string, if any, is displayed by the
				MS in a way described by the manufacturer.
	A24	MS -> SS	RELEASE COMPLETE	It terminates the transaction used to initiate the
				desired undefined SS data operation. See
				specific message contents iii).

B23	SS -> MS	RELEASE COMPLETE	It terminates the transaction used to activate
				the desired undefined SS data operation.
				See specific message contents iii).

24	SS -> MS	DISCONNECT
	25	MS -> SS	RELEASE
	26	SS -> MS	RELEASE COMPLETE	See message contents iv).
	27	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific message contents

i) REGISTER message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier

Register message type			As GSM 4.80

Facility Information Element		See below

SS version indicator				As specified in 04.80

Facility Information Element with Invoke = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.3

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						Arbitrary (1 octet)

Operation Code tag				From GSM 04.80

Operation Code length			1

Operation Code					ProcessUnstructuredSS-Request

ussd-dataCodingScheme			1 octet long.Alphabet indicator set to "default alphabet".
								Language indicator set to "undefined"

ussd-string						c=1, *60# (Activation  with no supplementary information
								included)
								c=2, *201*35# (Activation with one field of supplementary
								information included)
								c=3, *70*635*562# ( Activation with two fields of supplementary
								information included)
								c=4, #60# (Deactivation  with no supplementary information
								included)
								c=5, #201*35# (Deactivation  with one field of supplementary
								information included)
								c=6, #70*635*562# (Deactivation  with two fields of
								supplementary information included)
								c=7, *#60# (Interrogation  with no supplementary information
								included)
								c=8, *#201*35# (Interrogation  with one field of supplementary
								information included)
								c=9, *#70*635*562# (Interrogation  with two fields of
								supplementary information included)
								c=10, **60# (Registration  with no supplementary
								information included)
								c=11, **201*35# (Registration  with one field of
								supplementary information included)
								c=12, **70*635*562# (Registration  with two fields of
								supplementary information included)
								c=13, ##60# (Erasure with no supplementary
								information included)
								c=14, ##201*35# (Erasure with one field of
								supplementary information included)
								c=15, ##70*635*562# (Erasure  with two fields of
								supplementary information included)
								c=16,7 (related to conformance requirement  2)
								c=17, 26 (related to conformance requirement  2)

ii) Facility message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier				same as REGISTER transaction identifier

Facility message type				As GSM 4.80

Facility Information Element		See below

Facility Information Element with Return Result = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.4.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the ProcessUnstructuredSS-
								Request

Sequence Identifier				k=1, field omitted
								k=2, From 04.80

Sequence length					k=1, field omitted
								k=2,

Operation Code tag				k=1, field omitted
								k=2, From GSM 04.80

Operation Code length			k=1, field omitted
								k=2, 1

Operation Code					k=1, field omitted
								k=2, ProcessUnstructuredSS-Request

ussd-dataCodingScheme			k=1, field omitted
								k=2, 1 octet long.Alphabet indicator set to "default
								alphabet". Language indicator set to "undefined"

ussd-string						k=1, field omitted
								k=2, chosen at random

iii) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier				same as REGISTER transaction identifier

Release Complete message type	As GSM 4.80

Cause							for step A8, cause set to user normal call clearing
								otherwise, omitted.

Facility Information Element		See ii)

iv) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call control)"

Transaction identifier				same as current call.

Release Complete message type	As GSM04.08

#### 31.9.1.2	ProcessUnstructuredSS-request/cross phase compatibility and error handling

Conformance requirements

1)	If a mobile initiated USSD request using protocole version 2 is rejected by the network, and the reason for the rejection is indicated either by the problem code "unrecognized operation" or a cause "facility rejected", the mobile station shall assume that the network only supports protocole version 1 of USSD operations. The mobile station shall re-attempt the request by using the appropriate protocole version 1 USSD operation without a SS version indicator if the unstructured data entered by the user can be coded as an IA5 string.

2)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). If  ussd-String information is included this shall be given to the user (in a way described by the manufacturer).

References

Conformance requirement1:TS GSM 4.90 subclause 6.2.1

Conformance requirement2:TS GSM 3.90 subclause

Test Purpose

1) 	To verify that If a mobile initiated USSD request using protocole version 2 is rejected by the network, and the reason for the rejection is indicated either by the problem code "unrecognized operation" or a cause "facility rejected", the mobile station shall assume that the network only supports protocole version 1 of USSD operations. The mobile station shall re-attempt the request by using the appropriate protocole version 1 USSD operation without a SS version indicator if the unstructured data entered by the user can be coded as an IA5 string.

2)	To check that, upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). If  ussd-String information is included this shall be given to the user (in a way described by the manufacturer).

Related PICS/PIXIT Statements

- support of USSD

- Full rate supported

- Supported teleservices

- Support of active state of the call control protocol (U10)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle updated".

Foreseen final state of the MS

MM-state "idle updated".

Test Procedure

Appropriate MMI keys are depressed on the mobile in order to activate an USSD service. The mobile first establishes a MM-connection with the SS. Then, a REGISTER message is sent to the SS. Upon receipt of this message, the system simulator answers with the RELEASE COMPLETE message with the cause element set to "Facility rejected" or the problem code set to "unrecognized operation". Then the SS checks that the MS re-attemps the request by using a REGISTER message containing an invoke of the ProcessUnstructuredData operation. The SS answers with a normal RELEASE COMPLETE to terminate the transaction. Finally the main signalling link is released by transferring to the MS a CHANNEL RELEASE.

Appropriate MMI keys are depressed on the mobile in order to activate an USSD service. The mobile first establishes a MM-connection with the SS. Then, a REGISTER message is sent to the SS. Upon receipt of this message, the system simulator answers with the RELEASE COMPLETE message with the cause element set to "Facility rejected", the main signalling link is release. Then the SS checks that the MS re-establishes a MM-connection and re-attemps the request by using a REGISTER message containing an invoke of the ProcessUnstructuredData operation. Then the SS answers with a normal RELEASE COMPLETE to terminate the transaction. Finally the main signalling link is released by transferring to the MS a CHANNEL RELEASE.

The mobile station is forced to originate a call. After the SS has sent a CONNECT ACKNOWLEDGE, MMI keys are depressed on the mobile in order to activate an USSD service. A REGISTER message is sent to the SS. Upon receipt of this message, the system simulator answers with the RELEASE COMPLETE message containing a return error with an error or a reject with a problem. Then the radio link is release by transferring to the MS a CHANNEL RELEASE message. This subtest is repeated with different errors and problems in the RELEASE COMPLETE message.

Expected Sequence

Step	Direction	Message	Comments

1	MS		The user presses appropriate MMI keys to
				initiate the desired undefined supplementary
				service operation.

2	MS -> SS	CHANNEL REQUEST	with establishment cause
				set to "Other procedures which can be
				completed with an SDCCH".
	3	SS -> MS	IMM. ASSIGN
	4	MS -> SS	CM SERVICE REQUEST 	"Supplementary service activation".
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	REGISTER	See specific message contents i).Operation
				code set to "ProcessUnstructuredSS-Request"
	7	SS -> MS	 RELEASE COMPLETE	See specific message contents ii).
	8	MS -> SS	CM SERVICE REQUEST	"Supplementary service activation".
	9		SS -> MS	CM SERVICE ACCEPT
	10	MS -> SS	REGISTER	Operation code is set to
				"ProcessUnstructuredSS-Data".See specific
				message content i).
	11	MS -> SS	RELEASE COMPLETE	normal release of the transaction. See iiia).
	12	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

13	MS		The user presses appropriate MMI keys to
				initiate the desired undefined supplementary
				service.

14	MS -> SS	CHANNEL REQUEST	with establishment cause
				set to "Other procedures which can be
				completed with an SDCCH".
	15	SS -> MS	IMM. ASSIGN
	16	MS -> SS	CM SERVICE REQUEST 	"Supplementary service activation".
	17	SS -> MS	CM SERVICE ACCEPT
	18	MS -> SS	REGISTER	See specific message contents i).Operation
				code is set to "ProcessUnstructureSS-
				Request".See i).
	19	MS -> SS	RELEASE COMPLETE	see iii).cause is set to "operation rejected"
	20	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	21	MS -> SS	CHANNEL REQUEST	with establishment cause
				set to "Other procedures which can be
				completed with an SDCCH".
	22	SS -> MS	IMM. ASSIGN
	23	MS -> SS	CM SERVICE REQUEST 	"Supplementary service activation".
	24	SS -> MS	CM SERVICE ACCEPT
	25	MS -> SS	REGISTER	Operation code is
				"ProcessUnstructuredSSdata".See specific
				message contents i).
	26	SS -> MS	RELEASE COMPLETE	normal release of the transaction. See iiia).
	27	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For k=1 to 10, go through steps  28 to 
				45. Counter k deals with different kinds of
				general and invoke problems in step  41.

28	MS -> SS	CHANNEL REQUEST	with establishment cause related to mobile
				originating call.
	29	SS -> MS	IMMEDIATE ASSIGNMENT	a TCH/F channel is assigned to the MS.
	30	MS -> SS	CM SERVICE REQUEST	"mobile originating call"
	31	SS -> MS	CM SERVICE ACCEPT
	32	MS -> SS	SETUP
	33	SS -> MS	CALL PROCEEDING
	34	SS -> MS	ALERTING
	35	SS -> MS	CONNECT
	36	MS -> SS	CONNECT ACKNOWLEDGE

37			Specific MMI keys are depressed to initiate
				the desired undefined supplementary service.
				DTMF signalling may occur.

38	MS -> SS	CM SERVICE REQUEST	"supplementary service activation"
	39	SS -> MS	CM SERVICE ACCEPT
	40	MS -> SS	REGISTER	The MS starts the transaction on the radio
				interface. See specific message contents i).The
				operation code is set to
				"ProcessUnstructuredSS-Request".

41	SS -> MS	RELEASE COMPLETE	different errors and problems are sent. See
				specific message contents iiib) and iiic).
	42	SS -> MS	DISCONNECT
	43	MS -> SS	RELEASE
	44	SS -> MS	RELEASE COMPLETE	See message contents iv).
	45	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific message contents

i) REGISTER message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier

Register message type			As GSM 4.80

Facility Information Element		See below

SS version indicator				As specified in GSM 04.80 for version 2 protocol
								Always omitted for version 1 protocol

Facility Information Element with Invoke = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.3

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						Arbitrary (1 octet)

Operation Code tag				From GSM 04.80

Operation Code length			1

Operation Code					ProcessUnstructuredSS-Request (for version 2 protocol)
								ProcessUnstructuredSS-Data (for version 1 protocol)

ussd-dataCodingScheme			1 octet long.Alphabet indicator set to "default alphabet".
								Language indicator set to "undefined"

ussd-string						*70*635*562#

ii)  RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier				same as REGISTER transaction identifier

Facility message type				As GSM 4.80

Facility Information Element		See below

Facility Information Element with Reject = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.6.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag			Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the ProcessUnstructuredSS-
								Request

Problem Code tag				As GSM 04.80

Problem Code length

General Problem code			Unrecognized operation

iii) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier				same as REGISTER transaction identifier

Release complete message type	As GSM 4.80

cause							for step 17, cause is set to "facility rejected" and FIE is
								omitted.
								for steps 9, 23, 37 this field is omitted.

Facility Information Element		for step 23 see iiia).
								for steps 9, 37 see iiib) and iic).

iiia)	Facility Information Element with Return Result = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.4.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the ProcessUnstructuredSS-
								Request

iiib)	Facility Information Element with Return Error = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.5.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the ProcessUnstructuredSS-
								Request

Error Code tag					As GSM 04.80

Error Code length

Error Code						k=1, system failure
								k=2, data missing
								k=3, unknown alphabet
								k=4, unexpected data value

iiic)	Facility Information Element with Reject = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.6.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the ProcessUnstructuredSS-
								Request

Problem Code tag				As GSM 04.80

Problem Code length

General Problem code			k=5, Unrecognized component
								k=6, Mistyped component
								k=7, Badly structured component
Invoke Problem code				k=8, Mistyped parameter
								k=9, Resource limitation
								k=10, Initiating release

iv) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call control)"

Transaction identifier				same as current call.

Release Complete message type	As GSM04.08

#### 31.9.2	Network initiated unstructured supplementary service operations

#### 31.9.2.1	UnstructuredSS-Notify/accepted

Conformance requirements

1)	For a USSD notification, the MS shall display the text provided and await user input. The MS shall acknowledge the operation by sending a FACILITY message containing an empty result component to the network.

2)	The MS shall include alphabet and language indicators in the response to the network. The alphabet indicator shall indicate "SMS default alphabet". The language indicator shall indicate "language unspecified".

3)	At any stage while the MS is registered with a network, the network may send an unstructured string to the MS. So, the MS shall be able to process the operation during a call or out of a call.

References

conformance requirement 1: TS GSM 4.90 subclause 5.2.1

conformance requirement 2: TS GSM 2.90 subclause 4.2.2

conformance requirement 3: TS GSM 2.90 SUBCLAUSE 4.2.1

Test Purpose

1)	To verify that for a USSD notification, the MS shall display the text provided and await user input. If the user enters a response, the MS shall acknowledge the operation by sending a FACILITY message containnig an empty result component to the network.

2)	To verify that the MS shall include alphabet and language indicators in the response to the network. The alphabet indicator shall indicate "SMS default alphabet". The language indicator shall indicate "language unspecified".

3)	To check that the MS shall be able to process the operation during a call or out of a call.

Related PICS/PIXIT Statements

- support of USSD

- Full rate supported

- Supported teleservices

- Support of active state of the call control protocol (U10)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle updated".

Foreseen final state of the MS

MM-state "idle updated".

Test Procedure

-	The MS is paged and a RR-connection is established. Then, the SS sends a REGISTER message containing a facility information element with operation code set to ProcessUnstructuredSS-Notify. The user checks that the ussd string sent by the SS is correctly displayed by the MS. The MS has to send a FACILITY message with an empty return result component.

-	Then the SS originates a call to the MS. When the MS is in the U10 state, the SS releases the transaction identifier concerning USSD transaction by sending a RELEASE COMPLETE. The SS initiates a new ussd transaction by sending a REGISTER message containing a facility information element with operation code set to UnstructuredSS-Notify. The user checks that the ussd string sent by the SS is correctly displayed by the MS. The MS has to send a FACILITY message with an empty return result component. Finally the SS releases both MM connections and the radio link.

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	a SDCCH is allocated to the MS.
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	REGISTER	initiation of the transaction concerning the
				USSD Request operation. See i).
	6			The MS has to display the USSD string sent
				from the network.
	7	MS -> SS	FACILITY	signalling message sent by the MS as the
				response. The SS checks that it matches ii).
	8	SS -> MS	SETUP	incoming call.
	9	MS -> SS	CALL CONFIRMED
	10	MS -> SS	ALERTING
	11	MS -> SS	CONNECT
	12	SS -> MS	ASSIGNMENT COMMAND	a TCH is allocated to the MS.
	13	MS -> SS	ASSIGNMENT COMPLETE
	14 	SS -> MS	CONNECT ACKNOWLEDGE
	15	SS -> MS	RELEASE COMPLETE	this message releases the transaction
				concerning the USSD operation.See iii).
	16	SS -> MS	REGISTER	initiation of a USS request operation during a
				call. See specific message contents i).
	17			The MS has to display the USSD string
				received from the SS.
	18	MS -> SS	FACILITY	The SS checks that this message matches ii).
	19	SS -> MS	RELEASE COMPLETE	It releases he transaction identifier concerning
				the ussd operation.
	20	SS -> MS	DISCONNECT	call release initiation. see iv).
	21	MS -> SS	RELEASE
	22	SS -> MS	RELEASE COMPLETE
	23	SS -> MS	CHANNEL RELEASE

Specific message contents

i) REGISTER message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier

Register message type			As GSM 4.80

Facility Information Element		See below

Facility Information Element with Invoke = UnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.3

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						Arbitrary (1 octet)

Operation Code tag				From GSM 04.80

Operation Code length			1

Operation Code					UnstructuredSS-Notify

ussd-dataCodingScheme			1 octet long.Alphabet indicator set to "default alphabet".
								Language indicator set to "undefined"

ussd-string						 arbitrary chosen by the SS

ii) Facility message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier				same as REGISTER transaction identifier

Facility message type				As GSM 4.80

Facility Information Element		See below

Facility Information Element with Return Result = UnstructuredSS-Notify component type as defined in GSM 4.80 section 3.6.1 table 3.4.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the UnstructuredSS-Notify

iii) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier				same as REGISTER transaction identifier

Release Complete message type	As GSM 4.80

Facility Information Element		omitted

iv) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call control)"

Transaction identifier				same as current call.

Release Complete message type	As GSM04.08

31.9.2.2	UnstructuredSS-Notify/rejected on user busy

Conformance requirements

1)	When the mobile station receives an USSD operation in parallel to any call independent supplementary transaction, it shall respond with a return error component in a RELEASE COMPLETE message, containing the "USSD-Busy"error.

References

conformance requirement 1: TS GSM 4.90 subclause 5.1.1

Test Purpose

1)	To verify that when the mobile station receives an USSD operation in parallel to any call independent supplementary transaction, it responds with a return error component in a RELEASE COMPLETE message, containing the "USSD-Busy" error.

Related PICS/PIXIT Statements

- support of USSD

- Full rate supported

- Supported teleservices

- Support of active state of the call control protocol (U10)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle updated".

Foreseen final state of the MS

MM-state "idle updated".

Test Procedure

-	The MS is paged and a RR-connection is established. Then, the SS sends a REGISTER message containing a facility information element with operation code set to UnstructuredSS-Notify. The SS initiates an other USSD transaction by sending a REGISTER message with transaction identifier different form the previous one. The SS checks that the MS answers with a RELEASE COMPLETE message with an error component set to "USSD busy". Then the SS releases the radio link.

Test Procedure

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	a SDCCH is allocated to the MS.
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	REGISTER	initiation of the transaction concerning the
				USSD notification operation. See i).
	6			The MS has to display the USSD string sent
				from the SS.
	7		MS -> SS	FACILITY	See ii).
	8	SS -> MS	REGISTER	initiation of an other USSD request. The
				transaction identifier is different from that in
				step 5.See i).
	9	MS -> SS	RELEASE COMPLETE	error set to "USSD busy".Transaction initiated
				in step 6 by the network is rejected by the
				MS.See iii).
	10	SS -> MS	RELEASE COMPLETE 	normal release of transaction initiated in step
				5.See iii).
	11	SS -> MS	CHANNEL RELEASE	release of the main signalling link.

Specific message contents

i) REGISTER message

see paragraph 31.9.2..1

ii) FACILITY message

see paragraph 31.9.2..1

iii) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier				same as REGISTER transaction identifier

Release complete message type	As GSM 4.80

Facility Information Element		for step  9 see iiib).
								for step  10, see iiia).

iiia)	Facility Information Element with Return Result = UnstructuredSS-Notify component type as defined in GSM 4.80 section 3.6.1 table 3.4.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the UnstructuredSS-Notify

iiib)	Facility Information Element with Return Error = UnstructuredSS-Notify component type as defined in GSM 4.80 section 3.6.1 table 3.5.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the UnstructuredSS-Notify

Error Code tag					As GSM 04.80

Error Code length

Error Code						USSD Busy

#### 31.9.2.3	UnstructuredSS-Request/accepted

Conformance requirements

1)	For a USSD request, the MS shall display the text provided and await user input. if the user enters a response, the MS shall return the response to the network, maintaining the transaction.

2)	The MS shall include alphabet and language indicators in the response to the network. The alphabet indicator shall indicate "SMS default alphabet". The language indicator shall indicate "language unspecified".

3)	At any stage while the MS is registered with a network, the network may send an unstructured string to the MS. So, the MS shall be able to process the operation during a call or out of a call.

References

conformance requirement 1: TS GSM 3.90 subclause 5.2.5

conformance requirement 2: TS GSM 2.90 subclause 4.2.2

conformance requirement 3: TS GSM 2.90 subclause 4.2.1

Test Purpose

1)	To test that, for a USSD request, the MS shall display the text provided and await user input. if the user enters a response, the MS shall return the response to the network, maintaining the transaction.

2)	To verify that the MS shall include alphabet and language indicators in the response to the network. The alphabet indicator shall indicate "SMS default alphabet". The language indicator shall indicate "language unspecified".

3)	To check that the MS shall be able to process the operation during a call or out of a call.

Related PICS/PIXIT Statements

- support of USSD

- Full rate supported

- Supported teleservices

- Support of active state of the call control protocol (U10)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle updated".

Foreseen final state of the MS

MM-state "idle updated".

Test Procedure

-	The MS is paged and a RR-connection is established. Then, the SS sends a REGISTER message containing a facility information element with operation code set to ProcessUnstructuredSS-Request. The user checks that the ussd string sent by the SS is correctly displayed by the MS and answers the request by depressing MMI keys. Then the MS has to send a FACILITY message with ussd string exactly containing the digits and symbols expressed on the mobile equipment keypad.

-	Then the SS originates a call to the MS. When the MS is in the U10 state, the SS releases the transaction identifier concerning USSD transaction by sending a RELEASE COMPLETE. The SS initiates a new ussd transaction by sending a REGISTER message containing a facility information element with operation code set to UnstructuredSS-Request. The user checks that the ussd string sent by the SS is correctly displayed by the MS and answers the request by depressing MMI keys. Then the MS has to send a FACILITY message with ussd string exactly containing the digits and symbols expressed on the mobile equipment keypad. Finally the SS releases both MM connections and the radio link.

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	a SDCCH is allocated to the MS.
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	REGISTER	initiation of the transaction concerning the
				USSD Request operation. See i).
	6			The MS has to display the USSD string sent
				from the network and waits for the user
				response. By depressing MMI keys followed
				by SEND the user answers
	7	MS -> SS	FACILITY	signalling message sent by the MS as the
				response. See ii).
	8	SS -> MS	SETUP	incoming call.
	9	MS -> SS	CALL CONFIRMED
	10	MS -> SS	ALERTING
	11	MS -> SS	CONNECT
	12	SS -> MS	ASSIGNMENT COMMAND	a TCH is allocated to the MS.
	13	MS -> SS	ASSIGNMENT COMPLETE
	14 	SS -> MS	CONNECT ACKNOWLEDGE
	15	SS -> MS	RELEASE COMPLETE	this message releases the transaction
				concerning the USSD operation.See iii).
	16	SS -> MS	REGISTER	initiation of a USS request operation during a
				call. See specific message contents i).
	17			The MS has to display the USSD string
				received from the SS. By depressing MMI
				keys followed by SEND, the user answers.
				DTMF signalling may occur.
	18	MS -> SS	FACILITY	See ii).
	19	SS -> MS	RELEASE COMPLETE	It releases the transaction identifier concerning 
				the ussd operation.
	20	SS -> MS	DISCONNECT	call release initiation. see iv).
	21	MS -> SS	RELEASE
	22	SS -> MS	RELEASE COMPLETE
	23	SS -> MS	CHANNEL RELEASE

Specific message contents

i) REGISTER message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier

Register message type			As GSM 4.80

Facility Information Element		See below

Facility Information Element with Invoke = UnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.3

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						Arbitrary (1 octet)

Operation Code tag				From GSM 04.80

Operation Code length			1

Operation Code					UnstructuredSS-Request

ussd-dataCodingScheme			1 octet long.Alphabet indicator set to "default alphabet".
								Language indicator set to "undefined"

ussd-string						arbitrary chosen by the SS

ii) Facility message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier				same as REGISTER transaction identifier

Facility message type				As GSM 4.80

Facility Information Element		See below

Facility Information Element with Return Result = UnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.4.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the UnstructuredSS-Request

Sequence Identifier

Sequence length

Operation Code tag				1

Operation Code length			1

Operation Code					UnstructuredSS-Request

ussd-dataCodingScheme			1 octet long.Alphabet indicator set to "default
								alphabet". Language indicator set to "undefined"

ussd-string						contains exactly the digits and symbols expressed on the
								mobile equipment keypad.

iii) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call indepedent)"

Transaction identifier				same as REGISTER transaction identifier

Release Complete message type	As GSM 4.80

Facility Information Element		omitted

iv) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call control)"

Transaction identifier				same as current call.

Release Complete message type	As GSM04.08

## 31.10	MMI input for USSD

### 31.10.1	Conformance requirements

If the MS cannot interpret the MMI input as a defined GSM Supplementary Services, SIM control procedure or MS manufacturer-defined procedure, and if the MMI input is in the form:

"entry of any characters defined in the GSM 03.38 Default Alphabet (up to the maximum defined in GSM 04.80) followed by #SEND"

or

"entry of 1 or 2 characters defined in the GSM 03.38 Default Alphabet followed by SEND"

then it shall be interpreted by the MS as Unstructured SS Data and sent transparently towards the network, unless the MS is not engaged in a call and the first digit of the 2 character entry, followed by SEND, is a "1". In this case the MS shall transmit this as a call-setup request.

References

GSM 02.30 section 4.5.3

### 31.10.2	Test purpose

To check that the entry of 2 digits in the form 1X (X in the set 0,...9) followed by SEND is accepted by the mobile station in idle mode as a normal call establishment for the 1X number. It is checked that the MS sends a CHANNEL REQUEST, sends CM SERVICE REQUEST message for mobile originated call (after having received an IMMEDIATE  ASSIGNMENT), and then sends the SETUP message containing the 1X phone number as called number (after having received the CM SERVICE ACCEPT message).

#### 31.10.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station

The MS is "idle updated";

Related PICS/PIXIT statement(s)

Foreseen final state of the MS :

The MS is "idle updated

Test procedure

The user requests call establishment successively for every 2 digit phone number of the 1X form (X in the set 0,...9) allowing ten seconds between each attempts.

Maximum duration of test

3 min

Expected Sequence

The following sequence is executed for execution counter k = 1 to 10.

Step	Direction	Message	Comments

1	MS		The MS is made to initiate call establishment
				for phone number 1X (where X = k-1)
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				related to mobile originating call
	3	SS -> MS	IMM. ASSIGN
	4	MS -> SS	CM SERVICE REQUEST 	"mobile originating call"
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP	called BCD number is 1X
	7	SS -> MS	RELEASE COMPLETE
	8			Wait for 10 seconds

Specific message contents

None