# 26	Testing of layer 3 functions

Ref: GSM 04.08

NOTE :	The tests on functioning of the elementary procedures in the MS are grouped as the description of those procedures in GSM 04.08. However, the test procedures are carried out in an order which is more logic for the purpose of testing.

## 26.1	Default conditions and structured sequence of tests

### 26.1.1	Default test conditions during layer 3 tests

During tests in section 26 the following default test conditions shall apply if not otherwise stated within the test description. In the table below, decimal values are normally used. Sometimes a hexadecimal value, indicated with a 'H', or a binary value, indicated with a 'B' is given.

GSM900	DCS1800

General signalling conditions for all carriers

Cyphering	yes	yes

General RF-conditions for all carriers

Frequency hopping mode	Non-hopping	Non-hopping
Propagation profile	Static	Static
Downlink Input Level	63 dBVemf(  )	63 dBVemf(  )
Uplink output power	Minimum according	Minimum according
	to MS power class	to MS power class

Serving cell, BCCH/CCCH carrier

Channel ARFCN	20	590
Alternative channels	40 or 60	690 or 830

Serving cell, Traffic channel, SDCCH

Channel ARFCN	30	650
Alternative channels	50 or 70	750 or 850
Power Control Indicator	0	0

Neighbouring cells BCCH/CCCH carriers

Channel ARFCN	10, 80, 90, 100, 110, 120	520, 600, 700, 780, 810, 870
Alternative channels	15, 85, 95, 105, 115, 122	530, 610, 710, 790, 820, 880
Input level	53 dBVemf(  )	53 dBVemf(  )

Network dependent parameters

Cell identity	0001H	0001H
Mobile country code, MCC	001 (decimal)	001 (decimal)
Mobile network code, MNC	01 (decimal)	01 (decimal)
Location area code, LAC	0001H	0001H
Frequency List	Bit Map 0	Range 512
BCCH allocation sequence	0	0
number(BA-IND)
Cell Channel Descriptor	Bit Map 0	Range 512
PLMN colour code, NCC	1	1
BS colour code, BCC	5	5
SMS Cell Broadcast	not active	not active
DTX	MS must not use	MS must not use
IMSI Attach-detach	MS shall not apply	MS shall not apply
CCCH_CONF	1 basic physical channel for	1 basic physical channel for
	CCCH combined with SDCCH	CCCH combined with SDCCH
BS_AG_BLKS_RES	0 blocks reserved	0 blocks reserved
BS_PA_MFRMS	5 paging subgroups	5 paging subgroups

GSM900	DCS1800

CELL_BAR_ACCESS	(not barred)	(not barred)
Call-reestablishment (RE)	(allowed)	(allowed)
Emergency Call allowed	allowed	same
Access Control Class	allowed	same
(AC) (0..9, 11..15)

Network dependent timers
Radio_Link_Timeout	8	8
T3212 Periodic	Infinite	Infinite
updating in decihours

Access control parameters
Max retrans	1	1
Tx-integer, nr. of slots	5	5
CELL_RESELECT_HYSTERESIS	12 dB	12 dB
MS_TXPWR_MAX_CCH	minimum level	minimum level
RXLEV_ACCESS_MIN	minimum	minimum
NECI	New establishment causes	same
	are not supported
ACS (ADDITIONAL RESELECTION	No additional cell parameters	same
PARAM IND)	are present in SI messages
	7 and 8
P1 and C2 parameters	C2 parameters not present	same
POI and POWER OFFSET	N/A	POWER OFFSET Parameter not
		present.

These informations are provided by system information 1, 2, 3 and 4 messages.

The system information elements which are broadcast on the SACCH during the dedicated mode should be consistent with those sent on the BCCH when the MS was in idle mode prior to the channel request.

### 26.1.2	Structured sequence of the tests

The tests shall be performed in the order as indicated in the following table.

The validity of the tests depends upon the results of the tests performed before.

Channel request (basic test)	RR	26.2.1

Immediate assignment	RR	26.6.1

IMSI attach/detach (basic)	RR	26.2.2

Paging	RR	26.6.2

Test of the mobile station functions in idle mode	RR	26.3

Frequency redefinition	RR	26.6.6

Measurement report (incl. system info not idle)	RR	26.6.3

Authentication	MM	26.7.2

Cipher mode setting	RR	26.6.8

Identification	MM	26.7.3

Sequenced MM/CC message transfer	..	26.2.3

Channel release	RR	26.6.12

Location updating	MM	26.7.4

TMSI reallocation	MM	26.7.1

Classmark change	RR	26.6.11

Call controll (verification on CC state diagram)	CC	26.8.1

Call rearrangement	CC	26.8.* missing

DTMF information transfer	CC	26.8.4

Handover	RR	26.6.5

Additional assignment	RR	26.6.9

Partial release	RR	26.6.10

Re-establishment	CC	26.8.3

Dedicated channel assignment (during calls)	RR	26.6.4

Emergency call establishment	CC	26.8.2

Transmission mode change	RR	26.6.7

Mobility management connection establishment	MM	26.7.5

Test of Layer 3 error handling		26.5

User to user signalling	CC	26.8.5

Testing of structured procedures		26.9

E-GSM signalling		26.10

### 26.1.3	General rules for message parameters

The following rules concerning message parameters apply to section 26:

1)	Those values of parameters which are a consequence of the context of a test and which are not specific to that test need not be defined.

2)	If the value of a parameter of an uplink message (MS to Network) is specified in a test, the implicit meaning is that it has to be checked; if the value is not specified, it is not to be checked unless stated otherwise.

3)	An optional field or optional Information Element of a downlink message (Network to MS), the presence of which is not a consequence of a test description, shall be absent in that test.

4)	If an optional field or Information Element is not indicated for the uplink (MS to Network) - unless specified otherwise -, it may be included or not.

5)	The Protocol Discriminator, Transaction Identifier and Message Type of all uplink messages have to be checked.

### 26.1.4	General rules for layer 3 testing

Unless otherwise specified, before the SS pages the MS, the MS must be given the necessary time to be able to receive paging (see section 20.1).

### 26.1.5	Format of layer 3 test descriptions

In sections 26.2, 26.5, 26.6, 26.7, 26.9 and 26.10 a rigourous description technique is used which is defined here.

For every test, a subsection titled "method of test" and a subsection titled "expected sequence" define the exact test steps and the verifications to be performed in the test. These sections are normative and give requirements for the MS behaviour. The information of both subsections applies.

For the message contents further normative requirements for the MS behaviour are defined in the following parts which apply in the following order (starting with the highest) on basis of the general rules of 26.1.3:

-	specifications in the "method of test" and "expected sequence" subsections;

-	specifications in the subsection titled "specific message contents";

-	specifications in the subsection "default message contents" at the end of the relevant sections 26.5, 26.6, 26.7, 26.9 or 26.10;

-	specifications of default conditions in 26.1.

The relevant section may contain the definition of abbreviations of L3 message names that are used in that section.

In many cases, a test description contains an introductory subsection explaining the background of the relevant procedures and explaining why the tests of that description are essential.

For every test, test purposes are given. In general conformance testing methodology, the correspondence between test purposes and test cases can be n to m: To one test purpose more than one test case may correspond (e.g. different test cases checking data variations); also a test case may serve more than one test purpose. In some contexts a structure of conformance test descriptions is advisable which specifies in one part (non-duplicated) test purposes with references to corresponding test suites serving the test purposes, in another part test suites realizing the test purposes; this structuring is especially usefull for gaining completeness and avoiding duplications. In this specification, however, it is preferred to group descriptions by test cases. The reasons are:

-	The structure is more sought to assist the test execution and evaluation than test development. It must be easy to determine why a wrong behaviour leads to a verdict.

-	The structure is to be close to TS 11.10 phase 1.

For every test purpose of a test, a conformance requirement is given.

For each conformance requirement in a test description, references to core specifications are given.

For every test, the related PICS/PIXIT statements that are necessary for performing the test are given.

For every test, initial conditions for both the System Simulator and the Mobile Station are given. Unless otherwise specified, these initial conditions apply together with the default conditions of 26.1, the initial conditions of the test prevailing over the default conditions of 26.1.

For every test, the foreseen final state of the MS after the test and the maximum duration of the test are specified. These parts are non-normative and do not contain a description of verifications to be performed. The contained information might be used for sequencing different tests and for the decision when a test is to be interrupted.

The expected sequence specifies the actions in numbered steps in a tabular form. In the column "direction", "SS -> MS" denotes a message sent from the SS to the MS, "MS -> SS" denotes a message sent from the MS to the SS, "SS" denotes an action at the SS, "MS" denotes an action at the MS (e.g. interaction with the user or higher layers). The column "message" defines the L3 messages to be sent or expected by the SS. In the "comments" column, further normative information is to be found, e.g. message parameters. In some cases, different alternative behaviours are possible in a test. Then test steps in alternative  sequences are numbered as:

"A n", "A n+1",...,"A n+k"

"B n", "B n+1",...,"B n+l"

"C n", "C n+1",...,"C n+m"

etc (n,m,l,k integers > 0)

and  step numbering of a re-unified sequence resumes with the lowest of n+k+1, n+l+1, n+m+1.

In some cases the test steps of a test are to be repeated. Then an execution counter is introduced for the test.

## 26.2	Initial tests

### 26.2.1	Channel request

The random access procedure is used by the MS to ask for resources to the network. If it is not performed correctly, the MS could prevent other MSs from obtaining resources, or the network could be overloaded if the MS does not respect the duration between 2 CHANNEL REQUEST messages.

#### 26.2.1.1	Channel request / initial time

##### 26.2.1.1.1	Conformance requirement

1)	The MS shall start the initial access procedure at the latest 0.7 second after reception of the paging message.

2)	The MS shall spread the initial CHANNEL REQUEST with equal probability on the correct number of time slots.

Reference(s)

GSM 04.08 section 3.3.1.2 and 3.3.2.2

##### 26.2.1.1.2	Test purpose

1)	To verifiy that the MS answers to a PAGING message by sending a CHANNEL REQUEST message within 0.7 seconds after reception of the PAGING message.

2	To verify that the MS does not always use the same delay between reception of paging message and sending of the CHANNEL REQUEST message. If an MS uses a fixed delay, there is a high probability that different MSs of the same product series use the same delay. There would then be a high risk of collision.

##### 26.2.1.1.3	Method of test

Initial Conditions

System Simulator :

1 cell, Tx-Integer = 5. The CCCH is either combined or not with SDCCH. This is arbitrarily chosen.

Mobile Station :

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT Statement(s)

None.

Foreseen Final State of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

Specific test parameters :

K = 200.

The MS is paged. The SS measures and stores the number of CCCH RACH slots between the sending of the PAGING REQUEST message and the reception of the CHANNEL REQUEST from the MS, excluding the slots containing the messages themselves. The SS sends an IMMEDIATE ASSIGNMENT REJECT. The sequence is performed K times.

Maximum Duration of Test

30 mn.

Between two consecutive executions (for k and k+1), the SS must wait for an amount of time which is enough to guarantee that the MS is in service (listening to its paging subchannel).

Expected Sequence

The sequence is executed for execution counter k = 1, ... , K.

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	"Mobile Identity" IE contains 
					the TMSI allocated to the MS.
	2	SS		The SS measures the number f
					of CCCH RACH slots between the 
					sending of PAGING REQUEST message 
					and the reception of a CHANNEL 
					REQUEST message from the MS.
	3	SS		The SS stores f.
					f(k) shall be lower than 500/4.615+8
					if the CCCH is not combined or
					lower than 61+8 if the CCCH is
					combined with SDCCH.
	4	MS -> SS	CHANNEL REQUEST	"Establishment Cause" = Answer to 
					paging.
	5	SS -> MS	IMMEDIATE ASS REJECT	the first "request reference" corresponds
					to the CHANNEL REQUEST sent by the MS.

NOTE :	The test limit has been computed to give a confidence of [99.74 %] that a unit which follows the requirements will pass. The number of samples (200) has been chosen to get a good compromise between the test time and the risk of passing a bad unit.

##### 26.2.1.1.4	Test requirements

S(n) = CARD {k | f(k) = n}

The following requirements shall be met:

S(n) <= 41 for all n.

NOTE :	CARD {k | f(k) = n} is mathematical notation for the number of times that f(k) equals n.

#### 26.2.1.2	Channel request / repetition time

##### 26.2.1.2.1	Conformance requirement

1)	The MS shall spread retransmissions of a CHANNEL REQUEST message, with equal probability on Tx-Integer timeslots and with the correct delay after the reception of the PAGING REQUEST.

2)	The MS shall not retransmit another CHANNEL REQUEST message when Max-retrans is reached.

Reference(s)

GSM 04.08 section 3.3.1.2

##### 26.2.1.2.2	Test purpose

1)	To verify that the MS spreads retransmission of a CHANNEL REQUEST message with equal probability on Tx-Integer time slots and correctly applies the fixed delay when the following conditions apply:

-	the CCCH is combined or not combined with SDCCHs;

-	the maximum number of retransmissions is equal to one of the following values: 1, 2, 4, 7;

-	Tx-Integer is put to any of the allowed values among those which are greater or equal to 6.

2)	To verify that the MS retransmits exactly Max_Retrans times a CHANNEL REQUEST message if the network never responds to the CHANNEL REQUEST message.

##### 26.2.1.2.3	Method of test

Initial Conditions

System Simulator :

1 cell.

Tx-Integer is randomly chosen in the set {6, 7, 8, 9, 10, 11, 12, 14, 16, 20, 25, 32, 50}.

Max_Retrans is randomly chosen in the set {1,2,4,7}.

Mobile Station :

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT Statement(s)

None.

Foreseen Final State of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

Specific test parameters

K equals the upper rounded value of 230/Max_Retrans.

m equals the upper rounded value of 0.5*Tx-Integer.

Counter M = 0.

Parameter S : according to table 3.1/GSM 04.08 (this parameter depend on the value chosen for Tx-Integer).

N0 = max (8, Tx-Integer)

The MS is paged. The MS sends a CHANNEL REQUEST message. The MS retransmits CHANNEL REQUEST messages Max_Retrans times. The SS measures the number of CCCH RACH slots f(i,k) between the moment where a CHANNEL REQUEST message has been received, and the reception of the following CHANNEL REQUEST message, excluding the slots containing the messages themselves. The SS updates the counter M. The SS does not answer to the CHANNEL REQUEST messages Max_Retrans times. After the last CHANNEL REQUEST message in every sequence where k is lower than K, the SS sends an IMMEDIATE ASSIGNMENT REJECT . In the last sequence (k = K), the SS does not respond to the MS. The MS shall not send any other CHANNEL REQUEST message.

Maximum Duration of Test

The execution of one sequence (for one value k) : 10s.

Between two consecutive executions (for k and k+1), the SS must wait for an amount of time which is enough to guarantee that the MS is in service (listening to its paging subchannel).

Expected Sequence

The sequence is executed for execution counter k = 1, ... , K for each of the 2 test cases.

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	"Mobile Identity"= TMSI of the MS.
	2	MS -> SS	CHANNEL REQUEST	"Establishment Cause" = Answer to 
					paging.

Steps 3, 4, 5 are executed for 
					execution counter
					i = 1,...,Max_Retrans.
	3	MS -> SS	CHANNEL REQUEST	"Establishment Cause" = Answer to 
					paging.
	4	SS		The SS measures the number f(i,k) 
					of CCCH RACH slots between :
					- the moment where the last CHANNEL 
					REQUEST message has been received 
					and
					- the reception of the new CHANNEL 
					REQUEST message from the MS,
					excluding the slots containing 
					the messages themselves.
					f(i,k) shall be in the 
					set {S,S+1,...,S+T-1}
	5	SS		If f(i,k)-S >= m, M = M+1

Depending on the value of k, step 
					A6 or B6 is performed :
	A6	SS -> MS	IMMEDIATE ASSIGN REJ	k < K
					The third "Request Reference" IE 
					corresponds to the last CHANNEL 
					REQUEST message received.
					The third "Wait Indication" IE 
					specifies 0 second.
					Other fields do not address the MS 
					under test.
	B6	SS		k = K
					The SS checks that the MS sends no 
					more CHANNEL REQUEST messages. This 
					is verified during 3 seconds.

7	SS		M / (K * Max_Retrans) shall be
					inside the following interval :
					[0.8 - m/Tx-Integer ; 
					1.2 - m/Tx-Integer]

NOTE :	The confidence interval in step 7, and the number of samples are chosen in such a way that the possibility of non accepting a correct MS is less than [0.26%].

#### 26.2.1.3	Channel request / random reference

##### 26.2.1.3.1	Conformance requirement

A CHANNEL REQUEST message sent by the MS shall include a random reference randomly drawn from a uniform probability distribution for every new transmission.

Reference(s)

GSM 04.08 section 3.3.1.2

##### 26.2.1.3.2	Test purpose

To verify that an MS produces different random references for a CHANNEL REQUEST. If a MS always produces the same random reference, it makes possible that different MSs of the same product series produce the same random reference.

##### 26.2.1.3.3	Method of test

Initial Conditions

System Simulator :

1 cell, CCCH not combined with SDCCH.

Mobile Station :

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT Statement(s)

None.

Foreseen Final State of the MS

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR idle-mode.

Test Procedure

Specific test parameters:

K = 7.

D = 4.

The SS sends a PAGING REQUEST message. The SS stores the "Random Reference" r(k) contained as a parameter in the CHANNEL REQUEST message sent by the MS. This sequence is performed K times, and it is verified that the MS produces different values r(k).

Maximum Duration of Test

6 mn

Between two consecutive executions (for k and k+1), the SS must wait for an amount of time which is enough to guarantee that the MS is in service (listening to its paging subchannel).

Expected Sequence

The sequence is executed for execution counter k = 1, ..., K.

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	2	MS -> SS	CHANNEL REQUEST	"Establishment Cause" = Answer to paging.
	3	SS		The SS stores the "Random 
					Reference" contained in the CHANNEL 
					REQUEST message.

##### 26.2.1.3.4	Test requirements

At least D values of r(1),...,r(k) shall be different.

NOTE :	D has been computed such that the probability of refusing a correct MS is less than [0.027%].

### 26.2.2	IMSI detach and IMSI attach

The IMSI detach/attach procedures are used to indicate to the network that the MS is deactivated/activated. Theses procedures are allowed or not by the network (ATT flag set to "MSs in the cell shall apply IMSI attach and detach procedure" or "MSs in the cell are not allowed to apply IMSI attach and detach procedure").

If the IMSI attach procedure does not work correctly then the network would in certain situations not try to establish Mobile Terminating call even if the MS is "idle updated".

If an MS performs an unwanted IMSI detach procedure or does not perform IMSI detach when required, network resources are wasted.

#### 26.2.2.1	Conformance requirement

1)	When the Attach-detach flag in the Control Channel Description of the System Information Type 3 indicates "MSs in the cell are not allowed to apply IMSI attach and detach procedure", the MS shall not perform the IMSI detach procedure upon deactivation.

2)	When the Attach-detach flag in the Control Channel Description of the System Information Type 3 indicates "MSs in the cell are not allowed to apply IMSI attach and detach procedure", the MS shall not perform the IMSI attach procedure upon activation.

3)	The MS shall not perform the IMSI detach procedure if the Subscriber Identity Module is removed when the Attach-detach flag in the Control Channel Description of the System Information Type 3 indicates "MSs in the cell are not allowed to apply IMSI attach and detach procedure".

4)	The MS shall not perform the IMSI attach procedure if the Subscriber Identity Module is inserted, when the Attach-detach flag in the Control Channel Description of the System Information Type 3 indicates "MSs in the cell are not allowed to apply IMSI attach and detach procedure".

5)	The MS shall correctly perform the IMSI detach procedure, upon switch off, when it is required by the network to do so.

6)	The MS shall correctly perform the IMSI attach procedure upon switch on when the IMSI attach procedure is required by the network. The MS shall correctly acknowlege the implicit TMSI reallocation procedure, which is part of this IMSI attach procedure, this means that the MS shall send a TMSI REALLOCATION COMPLETE message.

7)	The MS shall correctly perform the IMSI detach procedure upon SIM removal when it is required by the network to do so.

8)	The MS shall correctly perform the IMSI attach procedure, following SIM insertion and switch on when the IMSI attach procedure is required by the network. The MS shall correctly acknowlege the implicit TMSI reallocation procedure which is part of this IMSI attach procedure. This means that the MS shall send a TMSI REALLOCATION COMPLETE message.

Reference(s):

GSM 02.07, normative annex B, section B1.17.

GSM 04.08 section 4.4.3 and 4.4.4

#### 26.2.2.2	Test purpose

1)	To verify that the MS correctly performs IMSI detach/attach procedures when it is required by the network and upon deactivation/activation or SIM removal/insertion and does not perform these procedures when not required.

2)	To verify that the mobile station acknowledges a re-allocated TMSI during IMSI attach.

#### 26.2.2.3	Method of test

Initial Conditions

System Simulator :

1 cell,default parameters.

For procedures 1 and 2 ATT flag is set to "MSs in the cell are not allowed to apply IMSI attach and detach procedure".

For procedures 3 and 4 ATT flag is set to "MSs in the cell should apply IMSI attach and detach procedure".

Mobile Station :

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT Statement(s)

a:	SIM removal possible without powering down	Yes / No.

b:	On/off switch				Yes / No.

c:	IMSI detach after SIM removal			Yes / No.

d:	IMSI detach after powering down		Yes / No.

Foreseen Final State of the MS

The MS has a valid TMSI which may be different from the initial one. It is "idle updated".

Test Procedure

The SS indicates that IMSI detach/attach is not allowed. If possible the MS is switched off, then switched on, otherwise it is powered down then powered on (see b in PICS). The SS checks that the MS does not perform IMSI detach/attach procedures. If possible (if a = Yes, see PICS), the SIM is removed, then the SIM is inserted. The SS checks that the MS does not perform IMSI detach/attach procedures.The SS indicates now, that IMSI detach/attach is allowed. If possible (if b = Yes, see PICS) the MS is switched off, otherwise it is powered down (if d = Yes, see PICS). The MS initiates an IMSI detach procedure. Then depending on what has been performed before, the MS is switched on or powered on. It initiates an IMSI attach procedure. The location updating procedure contains an implicit TMSI reallocation. The SIM is removed. If (a = yes and c = yes) or (a = no and d = yes) the MS initiates an IMSI detach procedure. Then the SIM is inserted, it initiates an IMSI attach procedure, the location updating procedure contains an implicit TMSI reallocation.

The SS indicates that IMSI detach/attach is not allowed. If possible the MS is switched off, then switched on, otherwise it is powered down then powered on (see b in PICS). The SS checks that the MS does not perform IMSI detach/attach procedures. If possible (if a = Yes, see PICS), the SIM is removed, then the SIM is inserted. The SS checks that the MS does not perform IMSI detach/attach procedures.The SS indicates now, that IMSI detach/attach is allowed. If possible (if b = Yes, see PICS) the MS is switched off, otherwise it is powered down (if d = Yes, see PICS). The MS initiates an IMSI detach procedure. Then depending on what has been performed before, the MS is switched on or powered on. It initiates an IMSI attach procedure. The location updating procedure contains an implicit TMSI reallocation. The SIM is removed. If (a = yes and c = yes) or (a = no and d = yes) the MS initiates an IMSI detach procedure. Then the SIM is inserted, it initiates an IMSI attach procedure, the location updating procedure contains an implicit TMSI reallocation.

Maximum Duration of Test

4 mn

Expected Sequence

Procedure 1

Step	Direction	Message	Comments

1	MS		If possible the MS is switched off 
					(see b in PICS), otherwise the MS 
					is powered down. 
	2				The MS shall not initiate the IMSI
					detach procedure. This is checked 
					by the SS during 5 seconds.
	3	MS		Depending on what has been 
					performed in step 1, the MS is 
					brought back to operation.
	4				The MS shall not initiate an IMSI 
					attach procedure. This is checked 
					by the SS during 30 seconds.

Procedure 2

1	MS		If possible (a = Yes, see PICS), 
					the SIM is removed from the MS.
	2				The MS shall not initiate the IMSI 
					detach procedure. This is checked 
					by the SS during 5 seconds.
	3	MS		The SIM is inserted in the MS. 
	4				The MS shall not initiate an IMSI 
					attach procedure. This is checked
					by the SS during 30 seconds.

Procedures 3

1	MS		The MS is switched off, or powered 
					down, depending on value b in the 
					PICS file. If b = Yes or d = 
					Yes the MS initiates an IMSI 
					detach procedure (steps A2, A3, A4, 
					A5), otherwise the SS goes straight to step
					6.

A2	MS -> SS	CHANNEL REQUEST
	A3	SS -> MS	IMMEDIATE ASSIGNMENT
	A4	MS -> SS	IMSI DETACH INDICATION
	A5	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
					the SS waits the disconnection of 
					the main signalling link.

6	MS		Depending on what has been 
					performed in step 1, the MS is 
					brought back to operation.
	7	MS -> SS	CHANNEL REQUEST	The MS initiates an IMSI attach 
					procedure.
	8	SS -> MS	IMMEDIATE ASSIGNMENT
	9	MS -> SS	LOCATION UPDATING REQ	"Location Updating Type" = IMSI 
					attach.
	10	SS -> MS	LOCATION UPDATING ACC	The SS allocates a new TMSI
	11	MS -> SS	TMSI REALLOCATION COMP
	12	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
					the SS waits the disconnection of 
					the main signalling link.

Specific message contents :

SYSTEM INFORMATION TYPE 3 message :

Information Element				value/remark

Control Channel Description

- Attach/Detach allowed		MS shall apply IMSI attach and

detach procedures.

Procedures 4

1	MS		The SIM is removed from the MS .
				If (a = Yes and c= Yes) or (a = no and 
				d =yes) in PICS, the 
					MS initiates an IMSI detach 
					procedure (steps A2, A3, A4, A5), otherwise
					the SS goes straight to step 6.

A2	MS -> SS	CHANNEL REQUEST
	A3	SS -> MS	IMMEDIATE ASSIGNMENT
	A4	MS -> SS	IMSI DETACH INDICATION
	A5	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
					the SS waits the disconnection of 
					the main signalling link.

6	MS		The SIM is inserted in the MS.
	7	MS -> SS	CHANNEL REQUEST	The MS initiates a IMSI attach 
					procedure.
	8	SS -> MS	IMMEDIATE ASSIGNMENT
	9	MS -> SS	LOCATION UPDATING REQ	"Location Updating Type" = IMSI 
					attach.
	10	SS -> MS	LOCATION UPDATING ACC	The SS allocates a new TMSI
	11	MS -> SS	TMSI REALLOCATION COMP
	12	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
					the SS waits the disconnection of 
					the main signalling link.

Specific message contents :

SYSTEM INFORMATION TYPE 3 message :

Information Element				value/remark

Control Channel Description

- Attach/Detach allowed		MS shall apply IMSI attach and

detach procedures.

### 26.2.3	Sequenced MM / CC message transfer

The RR sublayer of the MS shall have an associated send state variable V(SD) for sending MM and CM messages. This send state variable variable has been introduced to avoid the duplication of MM and CM messages. It is useful for the network after a handover or a change of channel to identify duplicated messages.

If the MS started V(SD) with 1 instead of 0 the network would incorrectly diagnose loss of message.

If the MS later on does not handle correctly incrementation of V(SD) the network would not be able to continue the dialogue.

#### 26.2.3.1	Conformance requirement

The MS shall implement correctly the "send state variable V(SD)" ("Send duplicated"), included in transmitted MM and CM messages.

Reference(s)

GSM 04.08 section 3.1.4.3

#### 26.2.3.2	Test purpose

To verify that V(SD) is correctly set to 0 at the beginning of the establishment of the first RR connection and to verify that the MS handles correctly this variable in the special case of IDENTITY REQUEST messages, which are MM messages.

#### 26.2.3.3	Method of test

Initial Conditions

System Simulator :

1 cell,default parameters.

Mobile Station :

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT Statement(s)

None.

Foreseen Final State of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

The MS is paged. After reception of the PAGING RESPONSE message from the MS, the SS sends an IDENTITY REQUEST message. The MS sends an IDENTITY RESPONSE message where N(SD) = 0. The SS repeats its IDENTITY REQUEST message 10 times. The MS transmits IDENTITY RESPONSE message with the value 1 and 0 in the N(SD) field alternately.

Maximum Duration of Test

1 mn

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	IDENTITY REQUEST
	6	MS -> SS	IDENTITY RESPONSE	N(SD) = 0
	7	SS -> MS	IDENTITY REQUEST
					Steps 7, 8, 9 and 10 are repeated 5
					times.
	8	MS -> SS	IDENTITY RESPONSE	N(SD) = 1.
	9	SS -> MS	IDENTITY REQUEST
	10	MS -> SS	IDENTITY RESPONSE	N(SD) = 0.
	11	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
					the SS waits the disconnection of 
					the main signalling link.

### 26.2.4	Establishment cause

The establishment cause set by the MS in the CHANNEL REQUEST message shall be consistent with the requested service or function, with the capabilities of the MS and with the indications given by the network.

If the MS uses a wrong establishment cause, the network might assign an unapropriate or incompatible resource.

In the case of Emergency call a wrong priority might be used.

If a reserved value is used, the network may discard the channel request.

#### 26.2.4.1	Conformance requirements

In the CHANNEL REQUEST message, the MS shall include an establishment cause which correspond to the establishment cause given by the MM sublayer and the broadcasted NECI value, or which correspond to one of the establishment causes "answer to paging" given by the RR entity in response to a PAGING REQUEST message including the Channel Needed information.

Reference(s)

GSM 04.08 section 3.3.1.2

#### 26.2.4.2	Test purpose

To verify that the establishment cause sent by the MS in the Max-Retrans+1 CHANNEL REQUEST messages is consistent with the requested service, with the capabilities of the MS and with the indications of the network in the following cases :

1)	If the MS supports a service on a traffic channel :

when the NECI bit is set to 0 and call re-establishment is attempted and the call was established on TCH/H if the MS supports a service on half rate channel or on TCH/F otherwise.

2)	If the MS supports a service on half rate channel :

when the NECI bit is set to 1 and call re-establishment is attempted and the call was established on TCH/H.

3)	If the MS supports speech :

3.1	when the NECI bit is set to 0 and a speech call is attempted.

3.2	when the NECI bit is set to 1 and a speech call is attempted.

4)	If the MS supports a data service :

4.1	when the NECI bit is set to 0 and a data call is attempted.

4.2	when the NECI bit is set to 1 and a data call is attempted for a service supported on half rate channel (if the MS does not support any data call on half rate channel any data service is used).

5)

5.1	when the NECI bit is set to 0 and the MS is paged with the paging indication set to "any channel".

5.2	when the NECI bit is set to 0 and the MS is paged with the paging indication set to "SDCCH".

5.3	when the NECI bit is set to 0 and the MS is paged with the paging indication set to "TCH/F".

5.4	when the NECI bit is set to 0 and the MS is paged with the paging indication set to "TCH/H or TCH/F".

6)

6.1	when the NECI bit is set to 0 and IMSI attach is attempted.

6.2	when the NECI bit is set to 0 and normal location updating is attempted.

6.3	when the NECI bit is set to 0 and periodic location updating is attempted.

6.4	when the NECI bit is set to 0 and IMSI detach is attempted.

7)	If the MS supports a non call related supplementary service operation :

when the NECI bit is set to 0 and a supplementary service operation is attempted at the MS.

8)	If the MS supports SMS/PP MO :

when the NECI bit is set to 0 and a mobile originated short message service transaction is attempted.

NOTE :	To verify that when the MS supports speech and an emergency call is attempted and the NECI bit is set to 0, then the MS sends a CHANNEL REQUEST message with an establishment cause consistent with the requested service, with the capabilities of the MS and with the indications of the network is done in test 26.9.6.1.1 test purpose 1.

#### 26.2.4.3	Method of test

Initial Conditions

System Simulator :

for all procedures : 1 cell, Max-Retrans = 7 slots. The NECI bit is set to 0.

Mobile Station :

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT Statement(s)

a:	MS supports speech on TCH/F				Yes / No

b:	MS supports speech on TCH/H				Yes / No

c:	MS supports data on TCH/F				Yes / No

d:	MS supports data on TCH/H				Yes / No

e:	MS only supports SDCCH				Yes / No

f:	MS supports a supplementary service operation		Yes / No

g:	MS supports SMS/PP MO				Yes / No

h:	On/Off switch					Yes / No

Note : in the above PICS, data and speech refer to the Radio Resource Channel Mode.

Foreseen Final State of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedures

NOTE :	If the procedures are chained, the SS shall ensure that at the beginnig of each procedure, the initial conditions are reached and that the MS had enough time to decode the broadcasted parameters.

Procedure 1

If the MS supports a service on a traffic channel:

A call is established on TCH/H if the MS supports a service on half rate channel or on TCH/F otherwise. The SS stops transmission on the SACCH. The MS attemps call reestablishment. The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "110".

Procedure 2

If the MS supports a service on half rate channel:

The NECI bit is set to 1. A call is established on TCH/H. The SS stops transmission on the SACCH. The MS attemps call reestablishment. The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "011010".

Procedure 3

If the MS supports speech:

A speech call is attempted. The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "111". The NECI bit is set to 1. A speech call is attempted. The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "111" if the MS does not support speech on half rate channel or "0100" if the MS supports speech on half rate channel.

Procedure 4

If the MS supports a data service:

A data call is attempted. The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "111". The NECI bit is set to 1. A data call is attempted for a service supported on half rate channel (if the MS does not support any data call on half rate channel any data service is used). The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "111"if the MS does not support a data service on half rate channel or "0101" if the MS supports a data service on half rate channel.

Procedure 5

The MS is paged with the paging indication set to "any channel". The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "100". The SS waits for a time sufficient for the MS to be "idle updated". The MS is paged with the paging indication set to "SDCCH". The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "0001". The SS waits for a time sufficient for the MS to be "idle updated". The MS is paged with he paging indication set to "TCH/F". The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "100" if the MS capability is full rate only, "0010" if the MS capability is dual rate and "0001" if the MS capability is SDCCH only. The SS waits for a time sufficient for the MS to be "idle updated". The MS is paged with the paging indication set to "TCH/H or TCH/F". The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "100" if the MS capability is full rate only, "0011" if the MS capability is dual rate and "0001" if the MS capability is SDCCH only.

Procedure 6

The MS is switched off or powered off. Then system information messages are altered so that IMSI attach/detach is allowed in the cell. The MS is switched on or powered on. The MS performs IMSI attach. The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the values "0000". The IMSI attach procedure is followed. The location area code of the cell is changed, T3212 is set to 1 deci-hour. The MS performs a location updating. The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the values "0000". The location updating procedure is followed. The SS waits for at least 7 minutes. The MS performs a periodic updating. The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the values "0000". The location updating procedure is followed. The MS is switched off or powered off. If the MS has an On/off switch (see PICS), it attempts IMSI detach. The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "0001".

Procedure 7

If the MS supports a non call related supplementary service operation:

A supplementary service operation is attempted at the MS. The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "0001".

Procedure 8

If the MS supports SMS/PP MO:

A mobile originated short message service transaction is attempted. The SS does not answer to Max-Retrans CHANNEL REQUEST messages and answers to the next CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message. The SS checks that all CHANNEL REQUEST messages contain an establishment cause with the value "0001".

Maximum Duration of Test

For procedures 1, 2, 3, 4, 5, 7 and 8: 5 minutes, including 1 minute for any necessary operator actions.

For procedure 6: 10 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Procedure 1

This procedure is performed if the MS supports a service on a traffic channel

Step	Direction	Message	Comments

1			a call is established on TCH/H if the MS
				supports a service on half rate channel
				or on TCH/F otherwise. The generic call
				setup procedure is used.
	2	SS		the SS stops transmission on the SACCH.
	3	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "110"
	4	SS -> MS	IMM ASSIGNMENT REJECT

Procedure 2

This procedure is performed if the MS supports a service on half rate channel

Step	Direction	Message	Comments

1	SS		The NECI bit is set to 1
	2			a call is established on TCH/H. The
				generic call setup procedure is used.
	3	SS		the SS stops transmission on the SACCH.
	4	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "011010"
	5	SS -> MS	IMM ASSIGNMENT REJECT

Procedure 3

This procedure is performed if the MS supports speech

Step	Direction	Message	Comments

1	MS		a speech call is attempted
	2	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "111"
	3	SS -> MS	IMM ASSIGNMENT REJECT

4	SS		The NECI bit is set to 1
	5	SS		The SS waits for 30 s
	6	MS		a speech call is attempted
	7	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "0100" if
				the MS supports speech on half rate
				or set to "111" otherwise
	8	SS -> MS	IMM ASSIGNMENT REJECT

Procedure 4

This procedure is performed if the MS supports a data service

Step	Direction	Message	Comments

1	MS		a data call is attempted
	2	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "111"
	3	SS -> MS	IMM ASSIGNMENT REJECT

4	SS		The NECI bit is set to 1
	5	SS		The SS waits for 30 s
	6	MS		a data call is attempted for a
				service supported by the MS on half
				rate (for any data service if the MS
				does not support any data service on
				half rate)
	7	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "0101" if
				the MS supports a data service on
				half rate or set to "111" otherwise
	8	SS -> MS	IMM ASSIGNMENT REJECT

Procedure 5

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	paging indication = any channel
	2	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "100"
	3	SS -> MS	IMM ASSIGNMENT REJECT

4	SS		The SS waits for 5 seconds
	5	SS -> MS	PAGING REQUEST TYPE 1	paging indication = SDCCH
	6	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "0001"
	7	SS -> MS	IMM ASSIGNMENT REJECT

8	SS		The SS waits for 5 seconds
	9	SS -> MS	PAGING REQUEST TYPE 1	paging indication = TCH/F
	10	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "100" if
				the MS capability is full rate only
				or "0010" if the MS capability
				is dual rate or "0001" if the MS
				capability is SDCCH only
	11	SS -> MS	IMM ASSIGNMENT REJECT

12	SS		The SS waits for 5 seconds
	13	SS -> MS	PAGING REQUEST TYPE 1	paging indication = TCH/H or TCH/F
	14	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "100" if
				the MS capability is full rate only
				or "0011" if the MS capability
				is dual rate or "0001" if the MS
				capability is SDCCH only
	15	SS -> MS	IMM ASSIGNMENT REJECT

Procedure 6

Step	Direction	Message	Comments

1	MS		The MS is switched off or powered down
	2	SS		IMSI attach/detach is set to "MSs in the
				cell shall apply IMSI attach and detach
				procedure"
	3	MS		The MS is switched on or powered on
	4	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "0000"
	5	SS -> MS	IMMEDIATE ASSIGNMENT
	6	MS -> SS	LOC UPDATING REQUEST	"location updating type" = IMSI attach
	7	SS -> MS	LOC UPDATING ACCEPT	with no mobile identity
	8	SS -> MS	CHANNEL RELEASE

9	SS		the LAC of the cell is changed and T3212
				is set to 6 minutes
	10	SS		the SS waits at least 30 seconds
	11	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "0000"
	12	SS -> MS	IMMEDIATE ASSIGNMENT
	13	MS -> SS	LOC UPDATING REQUEST	"location updating type" = Normal
				location updating
	14	SS -> MS	LOC UPDATING ACCEPT	with no mobile identity
	15	SS -> MS	CHANNEL RELEASE

16	SS		the SS waits at least 7 minutes
	17	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "0000"
	18	SS -> MS	IMMEDIATE ASSIGNMENT
	19	MS -> SS	LOC UPDATING REQUEST	"location updating type" = Periodic updating
	20	SS -> MS	LOC UPDATING ACCEPT	with no mobile identity
	21	SS -> MS	CHANNEL RELEASE

22	MS		If possible (see PICS), the MS is switched
				off, otherwise it is powered down
	23	MS		If the MS was switched off it attempts
				IMSI detach
	24	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "0001"
	25	SS -> MS	IMM ASSIGNMENT REJECT

Procedure 7

This procedure is performed if the MS supports a non call related supplemetary service operation

Step	Direction	Message	Comments

1	MS		a non call related supplementary
				service operation is attempted
	2	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "0001"
	3	SS -> MS	IMM ASSIGNMENT REJECT

Procedure 8

This procedure is performed if the MS supports SMS/PP MO

Step	Direction	Message	Comments

1	MS		a mobile originated short message
				service transaction is attempted
	2	MS -> SS	8 CHANNEL REQUEST	all messages have
				establishment cause set to "0001"
	3	SS -> MS	IMM ASSIGNMENT REJECT

## 26.3	Test of MS functions in idle mode

### 26.3.1	Initial conditions

The SIM shall contain a PLMN-Selector that contains only the HPLMN of the MS, and an empty forbidden PLMN list.

Related PICS/PIXIT statements

Type of mobile station (P-GSM900 or E-GSM900 or DCS1800)

During the tests in 26.3.2 and 26.3.3, the following parameters apply according to the above PICS/PIXIT statement:

RACH control parameters

In cells

GSM900:	1 to 7

DCS1800:	1 to 6:

Max retrans	= 01	2 retransmissions

Tx-integer	= 0111	(10) slots for spreading

CB, Cell Barred	= 0	access is allowed

RE	= 1	re-establishment not allowed

AC C00 to AC C15	= 0	access is not barred

In cell

GSM900:	8

DCS1800:	7:

Max retrans	= 01	2 retransmissions

Tx-integer	= 0111	(10) slots for spreading

CB, Cell Barred	= 1	access is not allowed

RE	= 1	re-establishment not allowed

AC C00 to AC C15	= 0	access is not barred

GSM900	DCS1800

Cell	PLMN	BA - ARFCN bit = 1	BA - ARFCN bit = 1
	perm.

1	00000100	7	39	65	66	85	97	124	520	580	610	702	703	830	885
2	00000100	8	40	67	68	86	98	123	521	581	612	704	705	831	884
3	00000100	9	41	69	70	87	99	122	522	582	614	706	707	832	883
4	00000100	10	42	71	72	88	100	121	523	583	616	708	709	833	882
5	00000100	11	43	73	74	89	101	120	524	584	618	710	711	844	881
6	00000100	12	44	75	76	90	102	119	525	585	620	712	713	835	880
7	00000100	13	45	77	78	91	103	118	526	586	622	714	715	836	879
8	00000100	124

Location area identification

GSM900 only - begin

Cell	MCC1	MCC2	MCC3	MNC1	MNC2	LAC

1	0	0	2	0	F	x
	2	0	0	3	2	F	x
	3	0	0	4	3	F	x
	4	0	0	5	4	F	x
	5	0	0	6	5	F	x
	6	0	0	7	6	F	x
	7	0	0	8	7	F	x
	8	0	0	1	0	1	x	The HPLMN of the MS

GSM900 only - end

DCS1800 only - begin

Cell	MCC1	MCC2	MCC3	MNC1	MNC2	LAC

1	0	0	2	0	F	x
	2	0	0	3	2	F	x
	3	0	0	4	3	F	x
	4	0	0	5	4	F	x
	5	0	0	6	5	F	x
	6	0	0	7	6	F	x
	7	0	0	1	0	1	x	The HPLMN of the MS

DCS1800 only - end

NOTE :	'x' denotes any value.

NOTE :	The MS representation of the MCC, MNC on the handset can be manufacturer dependant.

NOTE :	The NCC values of each cell must be different.

Control channel description and BS options

All

GSM900:	8 cells

DCS1800:	7 cells:

CELL_RESELECT_HYSTERESIS	= 010	4dB RXLEV hysteresis

MS_TXPWR_MAX_CCH	=	value corresponding to the maximum
		available output power from MS

RXLEV_ACCESS_MIN	= 30

ATT	= 0	no IMSI attach and detach

DTX	= 0	no discontinuous transmission

BS_AG_BLKS_RES	= 1	1 block reserved for access grant

CCCH_CONF	= 001	1 SDCCH combined with the CCCH

RADIO_LINK_TIMEOUT	= 5	10 s timeout

BS_PA_MFRMS	= 010	4 multiframes periods for paging

T3212 timeout value	= H'00

GSM900	DCS1800

Cell	level	BCCH ARFCN	level	BCCH ARFCN
		dBVemf(  )		dBVemf(  )

1	+65	1	+65	520
	2	+63	7	+63	580
	3	+61	39	+61	610
	4	+55	65	+55	702
	5	+59	66	+59	703
	6	+57	85	+57	830
	7	+55	97	+55	885
	8	+53	124

For testing an E-GSM Mobile station (see PICS/PIXIT), the BCCH ARFCN of call 7 shall be 985 (instead of 97).

NOTE :	The SIM should contain a PLMN-Selector that contains only the HPLMN of the MS, and an empty forbidden PLMN list.

### 26.3.2	MS indication of available PLMNs

#### 26.3.2.1	Test purpose

To verify that a MS can present the available PLMNs to the user when asked to do so in manual mode according to the requirements of GSM 05.08 and 02.11.

#### 26.3.2.2	Method of test

a)	The MS is switched on, equipped with a SIM containing default values except for those values listed under section 26.3.1 (initial conditions).

b)	The MS is put into manual network selection mode (see PIXIT).

#### 26.3.2.3	Test requirements

1)	On entering manual network selection mode, the MS shall present a list of available PLMNs (MCC and MNC values, or any other valid indications, see PIXIT), within 2 minutes. The list shall include the MCC and MNC of

GSM900:	cells 1 to 7, but not of cell 8

DCS1800:	cells 1 to 6, but not of cell 7.

### 26.3.3	MS will send only if BSS is "on air"

#### 26.3.3.1	Test purpose

To verify that the MS will not produce any RF transmission if no BSS is received.

#### 26.3.3.2	Method of test

a)	The RF-signal for the BCCHs of

GSM900:	cell 1 to 8 is switched off

DCS1800:	cell 1 to 7 is switched off.

b)	The SS shall wait 20 s to allow the MS to detect the loss of cells.

c)	By MMI, an attempt to originate a call is made.

d)	By MMI, an attempt to originate an emergency call is made.

Step d) is only performed if the MS supports speech (see PICS/PIXIT statement).

#### 26.3.3.3	Test requirements

1)	The MS must not give "service indication".

2)	In steps c) and d) the MS shall not produce any RF output.

### 26.3.4	Manual mode of PLMN selection

#### 26.3.4.1	Conformance requirements

In manual mode, the MS can try to obtain normal service on any available VPLMN and it shall try to obtain normal service on a VPLMN if and only if the user makes a manual selection of this VPLMN.

Reference

GSM 03.22 section 3.1

#### 26.3.4.2	Test purpose

To check that in manual mode the MS is able to obtain normal service on a PLMN which is neither the better nor a preferred PLMN and that it tries to obtain service on VPLMN if and only if the user selects it manually.

#### 26.3.4.3	Method of test

Initial conditions

System Simulator:

2 cells, defaults parameter unless otherwise specified.

The SS transmits 2 BCCH carriers in the supported band of the mobile station with the initial following parameters:

level (dBV)

carrier A	PLMN 1	28

carrier B	PLMN 2	33

Mobile Station:

The MS is "idle updated" on PLMN1 (HPLMN) and is in manual mode.

The preferred PLMN list does not contain PLMN2, it contains PLMN 3.

Related PICS/PIXIT statement(s)

Description of the manual PLMN selector.

Foreseen final state of the MS

The MS is "idle updated"

Test procedure

For the different networks and during the whole test, "IMSI attach" flag is set in the BCCH data.

Carrier A is turned off. The MS does not attempt a location updating during 2 minutes.

Carrier A is turned back on with a different MCC-MNC (indicating PLMN 3) and with a higher level (38) than PLMN 2. The MS does not attempt a location updating during 2 minutes.

PLMN 2 is selected manually. The MS performs a location updating on PLMN 2. Carrier B is turned off. The MS does not attempt a location updating during 2 minutes.

Maximum duration of test

10 minutes

Expected Sequence

Step	Direction	Message	Comments

1	SS		carrier A is turned off
	2			wait 2 min: the MS shall not send any
				CHANNEL REQUEST messages during this
				time

3	SS		carrier A is turned on with a different 
				MNC-NCC (PLMN3) and with a high level (38)
	4			wait 2 min: the MS shall not send any
				CHANNEL REQUEST messages during this
				time

5	MS		PLMN 2 selected manually
	6	MS -> SS	CHANNEL REQUEST
	7	SS -> MS	IMMEDIATE ASSIGNMENT
	8	MS -> SS	LOC UPD REQ	on carrier B
	9	SS -> MS	LOC UPD ACCEPT
	10	SS -> MS	CHANNEL RELEASE

11	SS		carrier B is turned off
	12			wait 2 min: the MS shall not send any
				CHANNEL REQUEST messages during this
				time

Specific message contents

None

## 26.4	Lower layer failures in layer 3 testing

### 26.4.1	Introduction

The text in this section is intended to develop a standardized way of creating lower layer failures whilst testing the performance of Layer 3 signalling.

There are two groups of lower layer failures:

1)	Detected by analysis of reception at Layer 1 (GSM 05.08, 04.08),

2)	Data link layer failures.

References

GSM 04.08, 04.06, 05.08

### 26.4.2	Layer 1 reception failures

The absence of reception of correct frames on the SACCH until the S counter reaches value 0 will be interpreted as a Layer 1 failure.

### 26.4.3	Data link layer failures

Many kinds of error cases can be caused in Layer 2. For example too many "T200 - timeout/retrying" - pairs.

NOTE :	All types of data link failures are indicated similarly to the RR layer (Release Indication).

NOTE :	All types of L1 failures are indicated similarly to each layer (Abort Indication, Error Indication).

### 26.4.4	Lower layer failures, used for the tests in clause 25

For L3 testing different lower layer failures are performed:

1)	T100 timeout in Layer 1.

2)	Too many T200 timeouts consecutively in Layer 2.

## 26.5	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions

Abbreviations for message names used in this section:

CIPH MOD CMD	=	CIPHERING MODE COMMAND

IMM ASSIGNMENT	=	IMMEDIATE ASSIGNMENT

IMM ASS REJ	=	IMMEDIATE ASSIGNMENT REJECT

LOC UPD REQ	=	LOCATION UPDATING REQUEST

LOC UPD ACC	=	LOCATION UPDATING ACCEPT

PAG REQ TYPE 1	=	PAGING REQUEST TYPE 1

### 26.5.1	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unknown protocol discriminator

An MS ignores messages with unknown protocol discriminator. This allows for the introduction of new messages which will be ignored by MS of earlier phases.

#### 26.5.1.1	Conformance requirements

If the mobile station receives a standard L3 message with a protocol discriminator different from those specified in Table 9.2/GSM 04.07, the mobile station shall ignore the message.

References

TS GSM 04.07, section 11.2.1.

#### 26.5.1.2	Test purpose

To verify that a MS supporting TCH and the call control protocol ignores a message containing an undefined protocol discriminator in the special case of a message coded otherwise like a CC STATUS ENQUIRY message received by the MS having a mobile terminating call in CC-state U10, "active".

#### 26.5.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has been paged and an RR connection has been established.

If the MS supports the call control protocol, the test may alternatively be performed with the MS having a mobile terminating call in the CC-state U10, "active".

Related PICS/PIXIT statements

-	Call control protocol supported for at least one BC (Y/N).

Foreseen Final State of the MS

Same as in the initial conditions.

Test Procedure

The SS sends a message to the MS which is coded like a CC STATUS ENQUIRY message relating to the active call except for the fact that the protocol discriminator of the message is undefined.

Maximum duration of test

11 seconds.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	UNKNOWN MESSAGE
	2	SS		The SS waits between 5 and 10 seconds
				verifying during this
				period that the MS does not
				send a L3 message on the main
				signalling link.

Specific message contents

UNKNOWN MESSAGE

Information element			Value/remark

Protocol discriminator			0000

TI flag				transaction originated by SS

TI value				TI value of the active call if the test is performed in state

U10 otherwise the value is arbitary.

Message Type			H'34

### 26.5.2	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / TI and skip indicator

#### 26.5.2.1	TI and skip indicator / RR

[Dissimilarities to 11.10 phase 1 - editorial notes :

- It is now specified how long and on which channel the SS verifies "no answer".

]

The MS ignores RR messages with skip indicator different to 0. This allows for the introduction of new RR messages which will be ignored by MS of earlier phases, especially on the downlink CCCH and BCCH.

##### 26.5.2.1.1	TI and skip indicator / RR / Idle Mode

###### 26.5.2.1.1.1	Conformance requirements

A radio resource message received with skip indicator different from 0000 shall be ignored.

Reference(s):

TS GSM 04.08, section 10.3.1.

###### 26.5.2.1.1.2	Test purpose

To verify that the MS ignores an RR message with skip indicator different from H'0 in the special case of a PAGING REQUEST TYPE 1 message received in the MM-state "idle, updated" and in RR-idle mode.

###### 26.5.2.1.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Related PICS/PIXIT statements

None.

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Test Procedure

For every binary value x in the range 0001 - 0110 (binary) and for binary value x = 1000, the following procedure is performed: The SS sends a PAGING REQUEST TYPE 1 message to the MS with skip indicator set to x. It is verified that the MS doesn't answer to the paging request message.

Maximum duration of test

5 seconds for each execution.

Expected sequence

The sequence is executed for execution counter k = 1,2,3,4,5,6,8.

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	The value of the 
				skip indicator IE is the binary 
				encoding of k.
	2	SS 		During 3 seconds the 
				SS verifies that the MS does not
				send any message on the RACH.

Specific message contents

None.

##### 26.5.2.1.2	TI and skip indicator / RR / RR-Connection established

###### 26.5.2.1.2.1	Conformance requirements

A radio resource message received with skip indicator different from H'0 shall be ignored.

Reference(s):

TS GSM 04.08, section 10.3.1.

###### 26.5.2.1.2.2	Test purpose

To verify that the MS ignores RR messages with skip indicator different from H'0 in the case of a message being received during the RR-connection establishment in the MM-state "idle, updated" / "wait for network command" and in RR-connected mode.

###### 26.5.2.1.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters, max retrans = 2.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Related PICS/PIXIT statements

None.

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Test Procedure

The SS sends a PAGING REQUEST TYPE 1 message to the MS with skip indicator set to H'0. The first CHANNEL REQUEST message will be answered with an IMMEDIATE ASSIGNMENT addressing the MS but with skip indicator set to H'1. Transmission of the second CHANNEL REQUEST message verifies that the MS has ignored the IMMEDIATE ASSIGNMENT message.

The second CHANNEL REQUEST message is answered by an IMMEDIATE ASSIGNMENT REJECT message addressing the MS but with skip indicator set to H'2 and a reject time set to 255 seconds.  Transmission of the third CHANNEL REQUEST message verifies that the MS has ignored the IMMEDIATE ASSIGNMENT REJECT message.

The third CHANNEL REQUEST message from the MS will be answered with a correct IMMEDIATE ASSIGNMENT addressing the MS and having skip indicator set to H'0.

In the RR-Connected mode messages such as CIPHERING MODE COMMAND, HANDOVER COMMAND, ASSIGNMENT COMMAND and CHANNEL RELEASE are sent with the skip indicator <> H'0 and it is checked that the MS does not take any action on these commands.

Maximum duration of test

40 seconds.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQ TYPE 1	The value of the 
				skip indicator IE is H'0
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	skip indicator set to H'1
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMM ASSIGNMENT REJ	skip indicator = H'2, reject time = 255
				seconds
	6	MS -> SS	CHANNEL REQUEST	Cause, answer to paging
	7	SS -> MS	IMMEDIATE ASSIGNMENT	skip indicator = H'0
	8	MS -> SS	PAGING RESPONSE	RR connection established
	9	SS -> MS	AUTH REQUEST
	10	MS -> SS	AUTH RESPONSE
	11	SS -> MS	CIPHERING MODE CMD	skip indicator = H'3
	12	SS		the SS neither starts ciphering nor deciphering
	13	SS -> MS	IDENTITY REQUEST	with IMSI requested
	14	MS -> SS	IDENTITY RESPONSE	to check the MS still uses unciphered mode
	15	SS -> MS	ASSIGNMENT COMMAND	skip indicator = H'4
	16	SS		SS checks no SABM is sent by the MS on the
				new channel
	17	SS -> MS	HANDOVER COMMAND	skip indicator = H'5
	18	SS		During 3 seconds the SS verifies that the MS
				does not send a handover failure or RR-Status
				messsage on the old channel
	19	SS -> MS	CHANNEL RELEASE	skip indicator = H'6
	20	SS -> MS	IDENTITY REQUEST	with IMSI requested
	21	MS -> SS	IDENTITY RESPONSE	to check the RR connection is still established
	22	SS -> MS	CHANNEL RELEASE	skip indicator = H'0
	23	SS		The SS checks that the layer 2 connection is
				released

Specific message contents

None.

#### 26.5.2.2	TI and skip indicator / MM

The MS ignores MM messages with skip indicator different to 0. This allows for the introduction of new MM messages which will be ignored by MS of earlier phases.

##### 26.5.2.2.1	Conformance requirements

A mobility management message received with skip indicator different from 0000 shall be ignored.

References

TS GSM 04.08, section 10.3.1.

##### 26.5.2.2.2	Test purpose

To verify that the MS ignores an MM message with skip indicator different from H'0 in the special case of an MS supporting the call control protocol and an IDENTITY REQUEST message received in the active state of a mobile terminating call.

##### 26.5.2.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a mobile terminating call in CC-state U10, "active", or alternatively, the MS has been paged and an RR connection has been established.

Related PICS/PIXIT statements

Call control protocol supported (Y/N).

Foreseen Final State of the MS

Same as in the initial conditions.

Test Procedure

For every binary value x in the range 0001 - 0110 and for the binary value x = 1000, the following procedure is performed: The SS sends an IDENTITY REQUEST message to the MS with skip indicator set to x. It is verified during 5 seconds that the MS doesn't answer to the IDENTITY REQUEST message.

Maximum duration of test

15 seconds.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'1.
	2	SS 		The SS starts verifying that the 
				MS does not send any L3 message
				on the main signalling link.
				This verification continues
				until step 16 of this test
				sequence.
	3	SS		The SS waits 1 second.
	4	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'2.
	5	SS		The SS waits 1 second.
	6	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'3.
	7	SS		The SS waits 1 second.
	8	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'4.
	9	SS		The SS waits 1 second.
	10	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'5.
	11	SS		The SS waits 1 second.
	12	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'6.
	13	SS		The SS waits 1 second.
	14	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'8.
	15	SS		The SS waits 5 seconds.
	16	SS		The SS stops verifying that the 
				MS does not send any L3 message
				on the main signalling link.

Specific message contents

None.

#### 26.5.2.3	TI and skip indicator / CC

[Dissimilarities to 11.10 phase 1 - editorial notes :

- Test at the moment only applicable to MS supporting call control.

Action points: 

Define in 04.08 when to use which location in cause IE, and that in specified cases the coding standard of GSM must be used; and that the MS shall not include octet 3a if coding standard is GSM (whereas for the receiver other rules apply).

Correct 8.3 of 04.08:

c) When a SETUP message is received specifying a transaction identifier which is not recognized as relating to an active call or to a call in progress, and with a transaction identifier flag incorrectly set to "1", this message shall be ignored.

Define in 11.10 that for indicated IEs the complete coding must be correct. Esp. IEI and length.

]

##### 26.5.2.3.1	Conformance requirements

a)	Whenever any call control message except SETUP or RELEASE COMPLETE is received specifying a transaction identifier with a value different from 111, which is not recognized as relating to an active call or to a call in progress, the receiving entity shall send a RELEASE COMPLETE message with cause value #81 "invalid transaction identifier value" using the received transaction identifier value and remain in the Null state.

b1)	When a RELEASE COMPLETE message is received specifying a transaction identifier with a value different from 111, which is not recognized as relating to an active call or to a call in progress, the MM-connection associated with that transaction identifier shall be released.

b2)	When a SETUP message is received with a transaction identifier flag set to "1", this message shall be ignored.

b3)	When a SETUP message is received specifying a transaction identifier which is recognized as relating to an active call or to a call in progress, this SETUP message shall be ignored.

c)	When a CC message with a TI value = 111 is received, this message shall be ignored.

References

TS GSM 04.08, section 8.3.

##### 26.5.2.3.2	Test purpose

a)	To verify that the MS having a mobile terminating call in CC-state U10, "active", on receipt of a DISCONNECT message which includes a transaction identifier with a value different from 111, which is not recognized as relating to an active call or a call in progress, sends a RELEASE COMPLETE message with cause value #81 and refering to the latter TI without changing the state of the active call (this is verified by use of the status enquiry procedure).

b)	To verify that the MS having a mobile terminating call in CC-state U10, "active", on receipt of a

b1)	RELEASE COMPLETE message which includes a transaction identifier with a value different from 111, which is not recognized as relating to an active call or a call in progress, or a

b2)	SETUP message with TI flag refering to a transaction originated by the MS (in the special case where the TI value is equal to the TI value relating to the active call), or a

b3)	SETUP message with TI refering to the active call,

ignores that message without changing the state of the active call (this is verified by use of the status enquiry procedure).

c)	To verify that the MS ignores a CCmessage with a TI value of 111.

The test is only applicable to an MS supporting the call control protocol for at least one BC.

##### 26.5.2.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a mobile terminating call in CC-state U10, "active". No other call is active or in progress.

Related PICS/PIXIT statements

Call control protocol supported for at least one BC (Y/N).

Foreseen Final State of the MS

The MS has a mobile terminating call in CC-state U10, "active". No other call is active or in progress.

Test Procedure

The SS sends a DISCONNECT message to the MS with a TI not relating to the active call. The MS shall respond with a RELEASE COMPLETE message including cause value #81 and specifying the same transaction. By means of the status enquiry procedure the SS checks that the CC-state of the active call did not change.

Then the SS sends the following call control messages to the MS;

-	a RELEASE COMPLETE message, where the TI does not refer to the active call,

-	a SETUP message with TI flag set to 1,

-	a SETUP message with TI refering to the active call.

-	a DISCONNECT messagewith a TI value of 111.

Each time the SS verifies that the MS does not respond to the message and each time the SS verifies by means of the status enquiry procedure that the CC-state of the active call has not been changed.

Maximum duration of test

40 seconds.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	TI flag = 0; TI does not refer
				to the active call.
	2	MS -> SS	RELEASE COMPLETE	TI flag = 1; TI value is equal
				to TI value received in step 1;
				Cause IE indicates cause value
				#81.
	3	SS -> MS	STATUS ENQUIRY	TI refers to the active call.
	4	MS -> SS	STATUS	TI refers to the active call;
				Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U10
	5	SS -> MS	RELEASE COMPLETE	TI flag = 0; TI does not refer
				to the active call.
	6	SS		The SS verifies during 5 seconds 
				that the MS does not send any L3 
				message on the main signalling 
				link.
	7	SS -> MS	STATUS ENQUIRY	TI refers to the active call.
	8	MS -> SS	STATUS	TI refers to the active call;
				Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U10
	9	SS -> MS	SETUP	TI flag = 1; TI value is equal
				to TI value of the active call.
	10	SS		The SS verifies during 5 seconds 
				that the MS does not send any L3 
				message on the main signalling 
				link.
	11	SS -> MS	STATUS ENQUIRY	TI refers to the active call.
	12	MS -> SS	STATUS	TI refers to the active call;
				Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U10
	13	SS -> MS	SETUP	TI flag = 0; TI refers
				to the active call.
	14	SS		The SS verifies during 5 seconds 
				that the MS does not send any L3 
				message on the main signalling 
				link.
	15	SS -> MS	STATUS ENQUIRY	TI refers to the active call.
	16	MS -> SS	STATUS	TI refers to the active call;
				Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U10
	17	SS -> MS	DISCONNECT	TI flag = 0; TI value is 111.
	18	SS		The SS verifies during 5 seconds that the MS
				does not send any L3 message on the main
				signalling link.
	19	SS -> MS	STATUS ENQUIRY	TI refers to the active call.
	20	MS -> SS	STATUS	TI refers to the active call;
				Cause IE indicates cause value #30.
				Call state IE indicates state U10

Specific message contents

None.

### 26.5.3	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / undefined or unexpected message type

#### 26.5.3.1	Undefined or unexpected message type / undefined message type / CC

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The MS is no more allowed to use cause value #98 in the first STATUS message.

- It is clarified how the undefined message type may look like.

- The test is only applicable to MSes supporting the CC protocol.

]

##### 26.5.3.1.1	Conformance requirements

If the Mobile Station receives a message with message type not defined for the PD, it shall ignore the message except for the fact that, if an RR-connection exists, it returns a status message (STATUS, RR-STATUS or MM-STATUS depending on the protocol discriminator) with cause value #97 "message type non-existent or not implemented".

References

TS GSM 04.08, section 8.4; TS GSM 04.07, section 11.2.4.

##### 26.5.3.1.2	Test purpose

To verify that a MS supporting the call control protocol for at least one BC, having a mobile terminating call in CC-state U10, "active", on receipt of a message with CC protocol discriminator and an arbitrary undefined message type, returns a STATUS message with cause value #97 to the peer CC entity without changing the state of the active call (this is verified by use of the status enquiry procedure.)

##### 26.5.3.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a mobile terminating call in CC-state U10, "active".

Related PICS/PIXIT statements

Call control protocol supported for at least one BC (Y/N).

Foreseen Final State of the MS

The MS has a mobile terminating call in CC-state U10, "active".

Test Procedure

The SS sends a message to the MS the PD of which refers to call control, the TI of which refers to the active call, and the message type of which is undefined in the call control protocol (however bit 7 of the message type is "0"). The SS then checks that the MS responds with a STATUS message specifying cause value #97. The SS then sends a STATUS ENQUIRY message to the MS and verifies that the MS responds with a STATUS message specifying cause value #30 and call state U10, "active".

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	see comments	PD = "call control; call
				related SS messages"

				TI is that of the active call

				Message type is undefined for
				call control, bit 7 of the
				message type is "0"
				

	2	MS -> SS	STATUS	Cause IE indicates cause value
				#97.
	3	SS -> MS	STATUS ENQUIRY	
				
	4	MS -> SS	STATUS	TI refers to the active call;
				Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U10

Specific message contents

None.

#### 26.5.3.2	Undefined or unexpected message type / undefined message type / MM

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The MS is no more allowed to use cause value #98 in the MM-STATUS message.

- The test is only applicable to MSes supporting the CC protocol. For other MSes, a test has to be added.

]

##### 26.5.3.2.1	Conformance requirements

If the Mobile Station receives a message with message type not defined for the PD, it shall ignore the message except for the fact that, if an RR-connection exists, it returns a status message (STATUS, RR-STATUS or MM-STATUS depending on the protocol discriminator) with cause value #97 "message type non-existent or not implemented".

References

TS GSM 04.08, section 8.4

##### 26.5.3.2.2	Test purpose

To verify that a MS supporting the call control protocol for at least one BC, having a mobile terminating call in CC-state U10, "active", on receipt of a message with MM protocol discriminator and message type undefined for the mobility management protocol, returns an MM-STATUS message with reject cause value #97 without changing the state of the active call (this is verified by use of the status enquiry procedure.) This is tested in the special case where the CC TI has value 0 (so that it has the same encoding as the skip indicator when sent from the SS) and where the message type has the same encoding as DISCONNECT in CC.

##### 26.5.3.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a mobile terminating call in CC-state U10, "active". The TI of that mobile terminating call has value 0.

Related PICS/PIXIT statements

Call control protocol supported for at least one BC (Y/N).

Foreseen Final State of the MS

The MS has a mobile terminating call in CC-state U10, "active".

Test Procedure

The SS sends a message to the MS the PD of which refers to mobility management, the skip indicator of which is "0000", and the message type of which is "0010 0101". The SS then checks that the MS responds with an MM-STATUS message specifying reject cause value #97. The SS then sends a STATUS ENQUIRY message to the MS and verifies that the MS responds with a STATUS message specifying cause #30 and call state U10, "active".

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	see comments	PD = "mobility management
				messages"

				Skip indicator = "0000"

				Message type = "0010 0101"
				rest of the message is
					H'02
					H'E0
					H'90

	2	MS -> SS	MM-STATUS	Reject cause IE indicates 
				reject cause value #97.

	3	SS -> MS	STATUS ENQUIRY	
				
	4	MS -> SS	STATUS	TI refers to the active call;
				Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U10

Specific message contents

None.

#### 26.5.3.3	Undefined or unexpected message type / undefined message type / RR

##### 26.5.3.3.1	Conformance requirements

If the Mobile Station receives a message with message type not defined for the PD, it shall ignore the message except for the fact that, if an RR-connection exists, it returns a status message (STATUS, RR-STATUS or MM-STATUS depending on the protocol discriminator) with cause value #97 "message type non-existent or not implemented".

Reference(s)

TS GSM 04.08, section 8.4

##### 26.5.3.3.2	Test purpose

To verify that an MS in RR connected mode on receipt of a message with RR protocol discriminator and message type undefined for the RR protocol, returns an RR-STATUS message with reject cause value #97 without changing its state (this is checked by observing that the MS does not send L3 messages.)

##### 26.5.3.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has been paged and an RR connection has been established.

Related PICS/PIXIT statement(s)

- The MS supports at least one bearer capability (p = Y/N)

Forseen Final State of the MS

The MS is in "idle updated" state.

Test Procedure

The SS sends a message to the MS the PD of which refers to radio resources management, the skip indicator of which is "0000", and the message type of which is "0010 1010". The SS then checks that the MS responds with an RR-STATUS message specifying reject cause value #97. The SS then verifies during 5 seconds that the MS does not send a L3 message on the main signalling link but continues sending L2 fill frames on the main signalling link. Then the SS sends a SETUP message to the MS. This message specifies a BC that is supported by the MS, if there exists any; if the MS does not support any BC, the SETUP message specifies an arbitrary BC. The SS then verifies that the MS responds with a CALL CONFIRMED message if the SETUP had specified a BC supported by the MS, and that the MMS responds with a RELEASE COMPLETE message otherwise. Then the SS sends a CHANNEL RELEASE to the MS and waits for the disconnection of the main signalling link.

Maximum duration of test

15 s.

Expected sequence

Step	Direction	Message	Comments

1	SS->MS	see comments	PD = "radio resources
				management messages"

				Skip indicator = "0000"

				Message type = "0010 0101"
				rest of the message is
					H'02
					H'E0
					H'90

	 2	MS->SS	RR-STATUS	RR cause IE indicates 
				RR cause value #97.

	 3	SS		During 5 seconds the SS
				verifies that the MS does not
				send a L3 message on the main
				signalling link but still
				continues to send L2 fill
				frames on the main signalling
				link.
 4	SS->MS	SETUP  	If the MS supports at least 
				one BC (p = Y), the SETUP
				specifies a bearer capability
				supported by the MS. 
				Otherwise (p = N) the SETUP
				message specifies any bearer
				capability.

A5	MS->SS	CALL CONFIRMED	This message shall be sent by
				the MS if p = Y.

B5	MS->SS	RELEASE COMPLETE	This message shall be sent by
				the MS if p = N.

6	SS->MS	CHANNEL RELEASE	The SS waits for disconnection
				of the main signalling link.

Specific message contents

None.

#### 26.5.3.4	Undefined or unexpected message type / unexpected message type / CC

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The MS is no more allowed to use cause value #97 in the first STATUS message.

- The test is only applicable to MSes supporting the CC protocol.

- Tests for inopportune messages in other protocols are missing.

]

##### 26.5.3.4.1	Conformance requirements

If the Mobile Station receives a message not consistent with the protocol state, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns a status message (STATUS, RR-STATUS or MM-STATUS depending on the protocol discriminator) with cause value #98 "Message type not compatible with protocol state".

References

TS GSM 04.08, section 8.4

##### 26.5.3.4.2	Test purpose

To verify that a MS supporting the call control protocol for at least one BC, having a call in CC-state U10, "active", on receipt of an inopportune CC message, returns a STATUS message with reject cause value #98 without changing the state of the active call (this is verified by use of the status enquiry procedure.) This is tested in the special case where the inopportune CC message is a CALL PROCEEDING message relating to the active call.

##### 26.5.3.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call in CC-state U10, "active".

Related PICS/PIXIT statements

Call control protocol supported for at least one BC (Y/N).

Foreseen Final State of the MS

The MS has a call in CC-state U10, "active".

Test Procedure

The SS sends a CALL PROCEEDING message to the MS. The SS then checks that the MS responds with a STATUS message specifying reject cause value #98. The SS then sends a STATUS ENQUIRY message to the MS and verifies that the MS responds with a STATUS message specifying cause #30 and call state U10, "active".

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CALL PROCEEDING	

	2	MS -> SS	STATUS	Cause IE indicates cause 
				value #98.
				
	3	SS -> MS	STATUS ENQUIRY	
				
	4	MS -> SS	STATUS	TI refers to the active call;
				Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U10

Specific message contents

None.

### 26.5.4	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unforeseen information elements in the non-imperative message part

[Dissimilarities to 11.10 phase 1 - editorial notes :

- Strange section 26.5.4.1 b) should be amended or deleted in phase 1!.

- Tests are missing that when repetition of information elements is specified, only the contents of specified repeated information elements shall be handled, and if the limit on repetition of information elements is exceeded, the contents of information elements appearing first up to the limit of repetitions shall be handled and all subsequent repetitions of the information element shall be ignored.

]

#### 26.5.4.1	Unforeseen information elements in the non-imperative message part / duplicated information elements

[Dissimilarities to 11.10 phase 1 - editorial notes :

- Tests are missing for the case of a repeated IE with format T or TV. This is important because the decoding mechanism is unclear.

- The verification of no response to the first paging is now specified in seconds rather than in terms of a time our value.

- After return to idle mode the time guaranteeing the MS becoming pageable is given explicitely.

]

##### 26.5.4.1.1	Conformance requirements

If an information element with format T, TV, or TLV is repeated in a message in which repetition of the information element is not specified, only the contents of the information element appearing first shall be handled and all subsequent repetitions of the information element shall be ignored.

References

TS GSM 04.08, section 8.6.3.

##### 26.5.4.1.2	Test purpose

To verify that the MS ignores an unforeseen second occurrence of an information element with format T, TV, or TLV in the special case of the mobile identity IE which has format TLV in the LOCATION UPDATING ACCEPT message.

##### 26.5.4.1.3	Method of test

Initial conditions

System Simulator:

2 cells A and B belonging to different location areas, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode, listening to the BCCH/CCCH of cell A. It has a valid TMSI.

Related PICS/PIXIT statements

None

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode, listening to the BCCH/CCCH of cell B. It does not have a valid TMSI.

Test Procedure

The RF level of cell A is lowered until the MS selects cell B (according to the cell-reselection procedures of TS GSM 05.08). The MS shall establish an RR connection and initiate the normal location updating procedure (using TMSI). The SS responds to the location update request with the LOCATION UPDATING ACCEPT message containing the mobile identity IE specifying the IMSI of the MS followed by an additional mobile identity IE specifying the TMSI that was assigned to the MS in the initial conditions (i.e. duplication of information element).

The SS then pages the MS using the PAGING REQUEST TYPE 1 message including the TMSI which was previously used in the LOCATION UPDATE ACCEPT message. The SS then verifies during 5 seconds that the MS doesn't answer to paging. The SS then pages the MS with its IMSI. The SS verifies that the MS responds on cell B by initiating the immediate assignment procedure using the CHANNEL REQUEST message.

Maximum duration of test

20 s.

Expected sequence

During 3 seconds the SS verifies that the MS does not send any message on the RACH.

Step	Direction	Message	Comments

1	SS 		The Rf level of cell A is
				lowered until the MS selects
				cell B.

	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOC UPD REQ	Mobile identity IE specifies
				the TMSI of the MS.

	5	SS -> MS	LOC UPD ACC	(see below)
	6	SS -> MS	CHANNEL RELEASE
	7	SS		The SS waits at least 5 s to
				give the MS time to become
				pageable

	8	SS -> MS	PAG REQ TYPE 1	Mobile identity 1 IE specifies
				the TMSI of the MS. 
				Mobile identity 2 is omitted.
	9	SS		The SS waits at least 5 seconds
		 		During that period the 
				SS verifies that the MS does not
				send any message on the RACH.
	10	SS -> MS	PAG REQ TYPE 1	Mobile identity 1 IE specifies
				the IMSI of the MS. 
				Mobile identity 2 is omitted.
	11	MS -> SS	CHANNEL REQUEST	Establishment cause = answer
				to paging.
	12	SS -> MS	IMM ASS REJ

Specific message contents

LOCATION UPDATING ACCEPT

Information element			value/remark

location area identification		LAI of cell B

Mobile identity 			coded TLV, specifies the IMSI of the MS

Type of identity				IMSI

Odd/even indication		corresponding to IMSI

Identity digit 1 etc.		corresponding to IMSI

Mobile identity (duplication)		coded TLV

Type of identity				TMSI of the MS

Odd/even indication		corresponding to TMSI

Identity digit 1 etc.		corresponding to TMSI

### 26.5.5.	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / non-semantical mandatory IE errors

#### 26.5.5.1.	Non-semantical mandatory IE errors / RR

##### 26.5.5.1.1	Non-semantical mandatory IE errors / RR / missing mandatory IE error

###### 26.5.5.1.1.1	Non-semantical mandatory IE errors / RR / missing mandatory IE error / special case

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The verification method of the MS releasing the RR connection is clarified: it consists in observing a L2 DISC frame.

- The test was numbered 26.5.5.1 procedure/requirement (i) inphase 1.

- Paging is now specified to use PAGING REQUEST TYPE 1 (hough this has no impact on the test).

]

The MS shall accept a CHANNEL RELEASE message whether it contains an RR cause or not. This allows for the shortening of the message in the future.

###### 26.5.5.1.1.1.1	Conformance requirements

When on receipt of a message a "missing mandatory IE" error is diagnosed the MS shall proceed as follows: If the message is a CHANNEL RELEASE message, the actions taken shall be the same as specified for a normal RR-connection release.

References

TS GSM 04.08, section 8.5.

###### 26.5.5.1.1.1.2	Test purpose

To verify that the MS in RR connected mode releases the connection upon receipt of a CHANNEL RELEASE message with missing RR cause (which is "mandatory" in that message).

###### 26.5.5.1.1.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Related PICS/PIXIT statements

None

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Test Procedure

A mobile terminating RR connection is established. Then the SS sends a CHANNEL RELEASE message in which the RR cause IE is missing. It is verified that the MS releases the main signalling link by sending a L2 DISC frame. The main signalling link release is then completed.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	

	2	MS -> SS	CHANNEL REQUEST	

	3	SS -> MS	IMM ASSIGNMENT	

	4	MS -> SS	PAGING RESPONSE	

	5	SS -> MS	CHANNEL RELEASE	The mandatory RR cause IE is
				missing (the message consists
				only of protocol discriminator,
				skip indicator, and 
				message type).
	6	MS -> SS		The main signalling link is
				released (this is observed by a
				L2 DISC frame sent from the MS
				to the SS).

Specific message contents

None.

###### 26.5.5.1.1.2	Non-semantical mandatory IE errors / RR / missing mandatory IE error / general case

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was numbered 26.5.5.1 procedure/requirement (ii) in phase 1.

- The RR cause value reported by the MS is 96 instead of 100 in phase 2.

]

In the general case, the MS has to report an RR message with missing mandatory IE bu the use of an RR-STATUS message, but otherwise to ignore it. This is  a recovery mechanism for unforeseen states.

###### 26.5.5.1.1.2.1	Conformance requirements

When on receipt of a message a "missing mandatory IE" error is diagnosed the MS shall proceed as follows: If the message is not one of the messages listed in sections 8.5.1, 8.5.2, and 8.5.3 of TS GSM 04.08, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns a status message (STATUS, RR-STATUS or MM-STATUS depending on the protocol discriminator) with cause value #96 "invalid mandatory information".

References

TS GSM 04.08, section 8.5.

###### 26.5.5.1.1.2.2	Test purpose

To verify that the MS in RR connected mode ignores a ciphering mode command message in which the ciphering mode setting IE and cipher response IE are missing except for the fact that it returns a RR-STATUS message.

###### 26.5.5.1.1.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Related PICS/PIXIT statements

None

Foreseen Final State of the MS

The MS is in RR-connected mode.

Test Procedure

A mobile terminating RR connection is established. Then the SS sends a ciphering mode command message in which the ciphering mode setting IE and cipher response IE are messing. The SS verifies that the MS does not start ciphering and returns a RR-STATUS message.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	

	2	MS -> SS	CHANNEL REQUEST	

	3	SS -> MS	IMM ASSIGNMENT	

	4	MS -> SS	PAGING RESPONSE	

	5	SS -> MS	CIPH MOD CMD	The mandatory
				ciphering mode setting IE and
				cipher response IE are missing.
	6	MS -> SS	RR-STATUS	RR cause IE specifies 
				RR cause value #96.

Specific message contents

None.

##### 26.5.5.1.2	Non-semantical mandatory IE errors / RR / comprehension required

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was numbered 26.5.6.1.3 procedure/requirement 1 in phase 1.

- Test is at the moment only applicable to MSes supporting the CC protocol. For other MSes a test should be added.

- The MS must report cause value #96 in ph. 2.

]

###### 26.5.5.1.2.1	Conformance requirements

When an RR message containing an IE unknown in the message, but encoded as "comprehension required" (see subclause 10.5 / TS GSM 04.08) is received, the MS shall proceed as follows: When the message is not one of the messages listed in TS GSM 04.08 sections 8.5.1, 8.5.2, and 8.5.3, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns a RR-STATUS message with cause value #96 "invalid mandatory information".

References

TS GSM 04.08, section 8.5.

###### 26.5.5.1.2.2	Test purpose

To verify that the MS having an RR-connection established ignores a HANDOVER COMMAND message containing in the non-imperative part an IE encoded as comprehension required except for the fact that it returns a RR-STATUS message with cause # 96 "invalid mandatory information".

###### 26.5.5.1.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has an MT call in state U10, "active"; or alternatively, the MS has been paged and an RR-connection has been established.

Related PICS/PIXIT statements

MS supports call control protocol for at least one BC (Y/N).

Foreseen Final State of the MS

As in the initial conditions.

Test Procedure

The SS sends a HANDOVER command message containing in the non-imperative part an IE encoded as comprehension required. The SS verifies that the MS returns a RR-STATUS message with cause value #96 without changing the dedicated channel.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	See below.
	2	MS -> SS	RR-STATUS	Sent on the old channel.
				RR cause IE specifies 
				RR cause value #96.

Specific message contents

HANDOVER COMMAND

Information element			value/remark

cell description			as required

channel description			as required

handover reference			as required

power command			as required

comprehension required IEI		0000	0000

length				0000	0001

unrecognised IE contents		xxxx	xxxx

#### 26.5.5.2	Non-semantical mandatory IE errors / MM

The MS shall ignore MM messages with syntactically incorrect mandatory IE. This allows to use reserved values in later phases.

##### 26.5.5.2.1	Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE

Test 26.5.5.2.1 is only applicable for an MS supporting at least one BC, whereas test 26.5.5.2.2 is applicable to all types of MS.

###### 26.5.5.2.1.1	Conformance requirements

When an MM message containing a syntactically incorrect mandatory IE is received, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns a MM-STATUS message with cause value #96 "invalid mandatory information".

References

TS GSM 04.08, section 8.5.

###### 26.5.5.2.1.2	Test purpose

To verify that an MS supporting at least one BC, having a CC entity in state U10, "active", ignores an MM message with syntactically incorrect IE except for the fact that it sends an MM-STATUS message with reject cause #96. This is tested in the special case of an IDENTITY REQUEST message in which the (mandatory) identity type IE specifies a reserved value for the type of identity; that the MS otherwise ignores the message is checked by means of the status enquiry procedure.

###### 26.5.5.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a mobile terminating call in the CC-state U10, "active".

Related PICS/PIXIT statements

MS supports call control protocol for at least one BC (Y/N)

Foreseen Final State of the MS

The MS has a mobile terminating call in the CC-state U10, "active".

Test Procedure

The SS sends an IDENTITY REQUEST message in which the (mandatory) identity type IE specifies a reserved value for the type of identity. The SS verifies that the MS returns an MM-STATUS message specifying cause value #96 but does not change its state (this is verified by use of the status enquiry procedure).

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	IDENTITY REQUEST	The identity type IE is encoded
				as "1111" (so that the type of
				identity contains the reserved
				value "111").
	2	MS -> SS	MM STATUS	Reject cause IE indicates 
				reject cause value #96.

	3	SS -> MS	STATUS ENQUIRY	TI refers to the active call.
	4	MS -> SS	STATUS	TI refers to the active call;
				Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U10.

Specific message contents

None.

##### 26.5.5.2.2	Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was is new in phase 2.

- For an MS supporting at least one BC, test 26.5.5.2.1 is used instead of this test.

]

Test 26.5.5.2.1 is only applicable for an MS supporting at least one BC, whereas this test (26.5.5.2.2) is applicable to all types of MS.

###### 26.5.5.2.2.1	Conformance requirement(s)

When an MM message containing a syntactically incorrect mandatory IE is received, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns an MM-STATUS message with cause value #96 "invalid mandatory information".

Reference(s)

TS GSM 04.08, section 8.5.

###### 26.5.5.2.2.2	Test purpose

To verify that an MS having been paged and having an RR connection established ignores an MM message with syntactically incorrect IE except for the fact that it sends an MM-STATUS message with reject cause #96. This is tested in the special case of an IDENTITY REQUEST message in which the (mandatory) identity type IE specifies a reserved value for the type of identity; the fact that the MS otherwise ignores the message is checked by testing that it answers as usual to an incoming SETUP message.

###### 26.5.5.2.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has been paged; an RR connection has been established.

The MS has a valid TMSI.

Related PICS/PIXIT statements

MS supports call control protocol for at least one BC (p=Y/N)

Foreseen final state of the MS

The MS is in the MM-state 'idle updated' listening to the BCCH/CCCH of the cell. It has a valid TMSI.

Test Procedure

The SS sends an IDENTITY REQUEST message in which the (mandatory) identity type IE specifies a reserved value for the type of identity. The SS verifies that the MS returns an MM-STATUS message specifying cause value #96 but does not change its state; this is verified as follows:

The SS sends a SETUP message to the MS. This message specifies a BC that is supported by the MS, if there exists any; if the MS does not support any BC, the SETUP message specifies an arbitrary BC. The SS then verifies that the MS responds with a CALL CONFIRMED message if the SETUP had specified a BC supported by the MS, and that the MS responds with a RELEASE COMPLETE message otherwise.

Then the SS sends a CHANNEL RELEASE to the MS and waits for the disconnection of the main signalling link.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	IDENTITY REQUEST	The identity type IE is encoded
				as "1111" (so that the type of
				identity contains the reserved
				value "111").
	2	MS -> SS	MM STATUS	Reject cause IE indicates 
				reject cause value #96.

3	SS -> MS	SETUP  	If the MS supports at least 
				one BC (p = Y), the SETUP
				specifies a bearer capability
				supported by the MS. 
				Otherwise (p = N) the SETUP
				message specifies any bearer
				capability.

A4	MS -> SS	CALL CONFIRMED	This message shall be sent by
				the MS if p = Y.

B4	MS -> SS	RELEASE COMPLETE	This message shall be sent by
				the MS if p = N.

5	SS -> MS	CHANNEL RELEASE	The SS waits for disconnection
				of the main signalling link.

Specific message contents

None.

##### 26.5.5.2.3	Non-semantical mandatory IE errors / MM / comprehension required

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.2, proc/req 1 in ph.1.

- Reject cause to be reported is 96 in ph 2.

]

The "comprehension required" mechanism allows for the introduction of essential new information elements into messages, such that a message is ignored and a report is sent if the new information element is not understood.

###### 26.5.5.2.3.1	Conformance requirements

When an MM message containing an IE unknown in the message, but encoded as "comprehension required" (see subclause 10.5 / GSM 04.08) is received, the MS shall ignore the message except for the fact that, if an RR-connection exists, it returns an MM-STATUS message with cause value #96 "invalid mandatory information".

References

TS GSM 04.08, section 8.5.

###### 26.5.5.2.3.2	Test purpose

To verify that the MS on receipt of an MM message containing an IE unknown in the message, but encoded as "comprehension required" ignores the message except for the fact that it returns an MM-STATUS message with cause value #96 "invalid mandatory information"; this in the special case of the MM message being a LOCATION UPDATING ACCEPT responding to a LOCATION UPDATING REQUEST from the MS.

###### 26.5.5.2.3.3	Method of test

Initial conditions

System Simulator:

The SS simulates two cells, A and B, belonging to different location areas, default parameters.

Mobile Station:

The MS is in the MM-state 'idle,updated' listening to the BCCH/CCCH of cell A. It has a valid TMSI.

Related PICS/PIXIT statements

None

Foreseen Final State of the MS

The MS is in the MM-state 'idle,updated' listening to the BCCH/CCCH of cell B. It has a valid TMSI.

Test Procedure

The Rf level of cell A is lowered until the MS selects cell B. The SS verifies that the MS establishes an RR connection and performs the normal location updating procedure using its TMSI. The SS responds to the location updating request with the LOCATION UPDATING ACCEPT message containing an optional information element coded as "comprehension required". The SS verifies that the MS returns the MM-STATUS message with cause #96 in response to the LOCATION UPDATING ACCEPT. The SS then waits ofr the MS to abort the RR-connection. The SS verifies that the MS establishes a new RR connection and starts a new location updating procedure.

On receipt of the new LOCATION UPDATING REQUEST, the SS sends a correctly coded LOCATION UPDATING ACCEPT allocating a new TMSI.

The SS verifies that the MS sends a TMSI REALLOCATION COMPLETE message. The SS then initiates the RR connection release.

Maximum duration of test

30 s.

Expected sequence

Step	Direction	Message	Comments

1	SS		The RF level of cell A is
				lowered until the MS selects
				cell B.
	2	MS -> SS	CHANNEL REQUEST	

	3	SS -> MS	IMM ASSIGNMENT	
	4	MS -> SS	LOC UPD REQ	The mobile identity IE specifies
				the TMSI of the MS.
	5	SS -> MS	LOC UPD ACC	See below.
	6	MS -> SS	MM-STATUS	Reject cause IE specifies reject
				cause value #96.
	7	MS		The MS aborts the RR connection
				(it initiates release of L2 on
				SAPI 0.
	8	MS -> SS	CHANNEL REQUEST	

	9	SS -> MS	IMM ASSIGNMENT	
	10	MS -> SS	LOC UPD REQ	The mobile identity IE specifies
				the IMSI of the MS.
	11	SS -> MS	LOC UPD ACC	see below
	12	MS -> SS	TMSI REALLOCATION COMPLETE
	13	SS -> MS	CHANNEL RELEASE	The RR connection is released.

Specific message contents

LOCATION UPDATING ACCEPT - first occurrence

Information element			value/remark

Location area identification		LAI of cell B

Comprehension required IEI		0000 0000

length				1

unrecognised IE contents		xxxx xxxx (arbitrary octet)

LOCATION UPDATING ACCEPT - second occurrence

Information element			value/remark

Location area identification		specifies LAI of cell B

Mobile Identity			specifies a TMSI

#### 26.5.5.3	Non-semantical mandatory IE errors / CC

##### 26.5.5.3.1	Non-semantical mandatory IE errors / CC / missing mandatory IE

[Dissimilarities to 11.10 phase 1 - editorial notes :

Test in proc/req (i) does no more make sense in phase 2.

- A possible new test would be a comprehension reqired IE.

]

###### 26.5.5.3.1.1	Non-semantical mandatory IE errors / CC / missing mandatory IE / disconnect message

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was 26.5.5.3 proc/req (ii) in ph. 1.

- The test is only applicable to an MS supporting the CC protocol.

- The reject cause to be reported by the MS is #96 in ph. 2, not #100.

- At the end of the test the MS is now brought to a stable state.

]

###### 26.5.5.3.1.1.1	Conformance requirements

When on receipt of a message a "missing mandatory IE" error is diagnosed, the MS shall proceed as follows: If the message is a DISCONNECT message, a RELEASE message shall be returned with cause value # 96 "invalid mandatory information" and normal call clearing applies.

References

TS GSM 04.08, section 8.5.

###### 26.5.5.3.1.1.2	Test purpose

To verify that the MS having an MT call in state U10, "active", on receipt of a DISCONNECT message in which the mandatory cause IE is missing shall return a RELEASE message with cause value #96 "invalid mandatory information".

###### 26.5.5.3.1.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has an MT call in the CC-state U10, "active".

Related PICS/PIXIT statements

MS supports call control protocol for at least one BC (Y/N).

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Test Procedure

The SS sends a DISCONNECT message in which the (mandatory) cause IE is missing. The SS verifies that the MS returns a RELEASE message specifying cause value #96. The SS then sends a RELEASE COMPLETE message and performs the RR connection release.

Maximum duration of test

15 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	The mandatory cause IE 
				is missing.
	2	MS -> SS	RELEASE	The cause IE indicates
				cause value #96

	3	SS -> MS	RELEASE COMPLETE	

	4	SS -> MS	CHANNEL RELEASE	The RR connection is released.

Specific message contents

None.

###### 26.5.5.3.1.2	Non-semantical mandatory IE errors / CC / missing mandatory IE / general case

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was 26.5.5.3 proc/req (iv) in ph. 1.

- The test is only applicable to an MS supporting the CC protocol.

- The reject cause to be reported by the MS is #96 in ph. 2, not #100.

]

###### 26.5.5.3.1.2.1	Conformance requirements

When on receipt of a message a "missing mandatory IE" error is diagnosed, the MS shall proceed as follows: If the message is not a SETUP, RELEASE, DISCONNECT, RELEASE COMPLETE, HOLD REJECT or RETRIEVE REJECT message, it shall ignore the message except for the fact that it returns a STATUS message specifying cause value #96.

References

TS GSM 04.08, section 8.5.

###### 26.5.5.3.1.2.2	Test purpose

To verify that the MS having an MT call in state U10, "active", on receipt of a STATUS message in which the mandatory cause IE and call state IE are missing shall ignore the message except for the fact that it return a STATUS message with cause value #96 "invalid mandatory information" (that the MS doesn't change state is checked by use of the status enquiry procedure).

###### 26.5.5.3.1.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has an MT call in the CC-state U10, "active".

Related PICS/PIXIT statements

MS supports call control protocol for at least one BC (Y/N).

Foreseen Final State of the MS

The MS has an MT call in the CC-state U10, "active".

Test Procedure

The SS sends a a STATUS message in which the mandatory cause IE and call state IE are missing. The SS verifies that the MS returns a STATUS message with cause value #96 "invalid mandatory information". Then the SS sends a STATUS ENQUIRY message and checks that the MS returns a STATUS message indicating cause value #30 and call state U10, "active".

Maximum duration of test

15 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	STATUS	The mandatory cause IE and
				call state IE are missing.
	2	MS -> SS	STATUS	The cause IE indicates
				cause value #96

	3	SS -> MS	STATUS ENQUIRY	TI refers to the active call.
	4	MS -> SS	STATUS	TI refers to the active call;
				Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U10

Specific message contents

None.

##### 26.5.5.3.2	Non-semantical mandatory IE errors / CC / comprehension required

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.3, proc/req 2 in ph.1.

- The behaviour of the MS has been changed: it must now report cause value #96.

]

###### 26.5.5.3.2.1	Conformance requirements

When a CC message containing an IE unknown in the message, but encoded as "comprehension required" (see TS GSM 04.08, section 10.5) is received, the MS shall proceed as follows: When the message is not one of the messages listed in TS GSM 04.08 sections 8.5.1, 8.5.2, and 8.5.3, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns a STATUS message with cause value #96 "invalid mandatory information".

References

TS GSM 04.08, sections 8.5 and 10.5.

###### 26.5.5.3.2.2	Test purpose

To verify that an MS supporting the call control protocol for at least one BC having a call control entity in state U3 ignores a CONNECT message containing in the non-imperative part an IE encoded as comprehension required except for the fact that it returns a STATUS message with cause value #96 "invalid mandatory information".

###### 26.5.5.3.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call contol entity in CC state U3.

Related PICS/PIXIT statements

MS supports call control protocol for at least one BC (Y/N).

Foreseen Final State of the MS

The MS has a call contol entity in CC state U3.

Test Procedure

The SS sends a CONNECT message containing an optional information element coded as 'comprehension required'. The SS verifies that the MS returns a STATUS message specifying cause value #96 "invalid mandatory information". The SS checks by use of the status enquiry procedure that the MS did not change the state.

Maximum duration of test

5 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CONNECT	See below.
	2	MS -> SS	STATUS	TI refers to the call in
				progress; cause IE indicates
				cause value #96.
	3	SS -> MS	STATUS ENQUIRY	TI refers to the call in
				progress.
	4	MS -> SS	STATUS	TI refers to the call in
				progress;
				Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U3.

Specific message contents

CONNECT

Information element				value/remark

Unknown IEI				0000 0000

length					1

unknown IE contents			xxxx xxxx (arbitrary octet)

### 26.5.6	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unknown IE, comprehension not required

[Dissimilarities to 11.10 phase 1 - editorial notes :

]

#### 26.5.6.1	Unknown information elements in the non-imperative message part / MM

##### 26.5.6.1.1	Unknown IE, comprehension not required / MM / IE unknown in the protocol

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.2, proc/req 2 in ph.1. It was intended to be run after proc/req 1 of the same section.

- The behaviour of the MS has been changed: no more MM-STATUS message.

- It is not nice that the SS causes the MS abort the RR connection.

- The MS is brought to the idle state at the end.

- The ph. 1 test must be corrected so that the unknown IE has bit 8 = 0 and is undefined in 04.08. For phase 2 it is sufficient that the IE has bit 8 = 0 and is undefined in the message.

]

###### 26.5.6.1.1.1	Conformance requirements

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

References

TS GSM 04.08, sections 8.6.1, 8.6.2 and 10.5.

###### 26.5.6.1.1.2	Test purpose

To verify that the MS on receipt of an MM message containing an IE unknown in the message and unknown in the MM protocol which is not  encoded as "comprehension required" ignores that IE; this in the special case of the MM message being a LOCATION UPDATING ACCEPT responding to a LOCATION UPDATING REQUEST from the MS.

###### 26.5.6.1.1.3	Method of test

Initial conditions

System Simulator:

The SS simulates two cells, A and B, belonging to different location areas, default parameters.

Mobile Station:

The MS is in the MM-state 'idle,updated' listening to the BCCH/CCCH of cell B. It has a valid TMSI.

Related PICS/PIXIT statements

None

Foreseen Final State of the MS

The MS is in the MM-state 'idle, updated' listening to the BCCH/CCCH of cell A. It has a valid TMSI.

Test Procedure

The RF level of cell B is lowered until the MS selects cell A. The SS verifies that the MS establishes an RR connection and performs the normal location updating procedure using its TMSI. The SS responds to the location updating request with the LOCATION UPDATING ACCEPT message containing an optional information element not coded as 'comprehension required' the IE of which is unknown in the MM protocol. The LOCATION UPDATING ACCEPT message contains a new TMSI in the mobile identity IE which is placed after the unknown IE. The MS shall send the TMSI REALLOCATION COMPLETE message.

Maximum duration of test

20 s.

Expected sequence

Step	Direction	Message	Comments

1	SS		The RF level of cell B is
				lowered until the MS selects
				cell A.
	2	MS -> SS	CHANNEL REQUEST	

	3	SS -> MS	IMM ASSIGNMENT	
	4	MS -> SS	LOC UPD REQ	The mobile identity IE specifies
				the TMSI of the MS.
	5	SS -> MS	LOC UPD ACC	See below.
	6	MS -> SS	TMSI REALLOCATION CMP
	7	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific message contents

LOCATION UPDATING ACCEPT

Information element				value/remark

Location area identification			LAI of cell A

Unknown IEI				 1010 xxx0 (where x is arbitrary)

Mobile Identity IEI

length					5

Type of identity				TMSI

Identity				4 octets of 'new' TMSI

##### 26.5.6.1.2	Unknown IE, comprehension not required / MM / IE unknown in the message

[Dissimilarities to 11.10 phase 1 - editorial notes :

- New test. Is identical to II.5.3.5.6.1.1 except for the unknown IEI in the LOCATION UPDATING message.

]

###### 26.5.6.1.2.1	Conformance requirements

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

References

TS GSM 04.08, sections 8.6.1, 8.6.2 and 10.5.

###### 26.5.6.1.2.2	Test purpose

To verify that the MS on receipt of an MM message containing an IE unknown in the message, but known in the MM protocol, which is not encoded as "comprehension required" ignores that IE; this in the special case of the MM message being a LOCATION UPDATING ACCEPT responding to a LOCATION UPDATING REQUEST from the MS.

###### 26.5.6.1.2.3	Method of test

Initial conditions

System Simulator:

The SS simulates two cells, A and B, belonging to different location areas, default parameters.

Mobile Station:

The MS is in the MM-state 'idle, updated' listening to the BCCH/CCCH of cell B. It has a valid TMSI.

Related PICS/PIXIT statements

None

Foreseen Final State of the MS

The MS is in the MM-state 'idle, updated' listening to the BCCH/CCCH of cell A. It has a valid TMSI.

Test Procedure

The RF level of cell B is lowered until the MS selects cell A. The SS verifies that the MS establishes an RR connection and performs the normal location updating procedure using its TMSI. The SS responds to the location updating request with the LOCATION UPDATING ACCEPT message containing an optional information element not coded as 'comprehension required' the IEI of which is unknown in the message but is used as the location area identification IEI in other messages of the MM protocol. The LOCATION UPDATING ACCEPT message contains a new TMSI in the mobile identity IE which is placed after the unknown IE. The MS shall send the TMSI REALLOCATION COMPLETE message.

Maximum duration of test

20 s.

Expected sequence

Step	Direction	Message	Comments

1	SS		The RF level of cell B is
				lowered until the MS selects
				cell A.
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMM ASSIGNMENT	
	4	MS -> SS	LOC UPD REQ	The mobile identity IE specifies
				the TMSI of the MS.
	5	SS -> MS	LOC UPD ACC	See below.
	6	MS -> SS	TMSI REALLOCATION CMP
	7	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific message contents

LOCATION UPDATING ACCEPT

Information element				value/remark

Location area identification			LAI of cell A

Unknown IEI				0001 0011

length					2

unknown IE contents			xxxx xxxx

xxxx xxxx (2 arbitrary octets)

Mobile Identity IEI

length					5

Type of identity				TMSI

Identity				4 octets of 'new' TMSI

#### 26.5.6.2	Unknown information elements in the non-imperative message part / CC

##### 26.5.6.2.1	Unknown information elements in the non-imperative message part / CC / Call establishment

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.3, proc/req 1 in ph.1.

- The behaviour of the MS has been changed: no more STATUS message.

- The ph. 1 test must be corrected so that the unknown IE has bit 8 = 0 and is undefined in 04.08. For phase 2 it is sufficient that the IE has bit 8 = 0 and is undefined in the message.

]

###### 26.5.6.2.1.1	Conformance requirements

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

References

TS GSM 04.08, section 8.6.1.

###### 26.5.6.2.1.2	Test purpose

To verify that an MS supporting the CC protocol for at least one BC receiving a CC message containing an IE unknown in the message which is not encoded as "comprehension required" ignores that IE; this in the special case of the CC message being a CALL PROCEEDING message received by the MS in state U1.

###### 26.5.6.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call control entity in CC state U1.

Related PICS/PIXIT statements

MS supports call control protocol for at least one BC (Y/N).

Foreseen Final State of the MS

The MS has a call control entity in CC state U3.

Test Procedure

The SS sends a CALL PROCEEDING message containing an optional information element not coded as 'comprehension required' the IEI of which is unknown in the message, but used for a called party BCD number IE in other messages of the protocol. The SS verifies by use of the status enquiry procedure that the MS did not change the state.

Maximum duration of test

30 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CALL PROCEEDING	See below.
	2	SS -> MS	STATUS ENQUIRY	TI refers to the call in
				progress.
	3	MS -> SS	STATUS	TI refers to the active call;
				Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U3.

Specific message contents

CALL PROCEEDING

Information element			value/remark

Unknown IEI			0101 1110

length				1

unknown IE contents		xxxx xxxx (arbitrary octet)

##### 26.5.6.2.2	Unknown information elements in the non-imperative message part / CC / disconnect

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.3, proc/req 3 in ph.1.

- The behaviour of the MS has been changed: no more cause #99 required.

- The ph. 1 test must be corrected so that the unknown IE has bit 8 = 0 and is undefined in 04.08. For phase 2 it is sufficient that the IE has bit 8 = 0 and is undefined in the message.

]

###### 26.5.6.2.2.1	Conformance requirements

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

References

TS GSM 04.08, section 8.6.1.

###### 26.5.6.2.2.2	Test purpose

To verify that an MS supporting the CC protocol for at least one BC receiving a CC message containing an IE unknown in the message which is not encoded as "comprehension required" ignores that IE; this in the special case of a DISCONNECT message received by the MS in state U10.

###### 26.5.6.2.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call control entity in CC state U10.

Related PICS/PIXIT statements

MS supports call control protocol for at least one BC (Y/N).

Foreseen Final State of the MS

The MS has a call control entity in CC state U19.

Test Procedure

The SS sends a DISCONNECT message containing an optional information element not coded as 'comprehension required' the IEI of which is unknown in the message, but used for a connected number IE in other messages of the protocol. The SS verifies that the MS responds with a RELEASE message; the SS verifies by use of the status enquiry procedure that the MS has entered state U19.

Maximum duration of test

5 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	See below.
	2	MS -> SS	RELEASE
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	Cause IE indicates cause value
				#30.
				Call state IE indicates state
				U19.

Specific message contents

DISCONNECT

Information element			value/remark

Unknown IEI			0100 1100

length				1

unknown IE contents		xxxx xxxx (arbitrary octet)

##### 26.5.6.2.3	Unknown information elements in the non-imperative message part / CC / release

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.3, proc/req 4 in ph.1.

- The behaviour of the MS has been changed: no more cause #99 required.

- The ph. 1 test must be corrected so that the unknown IE has bit 8 = 0 and is undefined in 04.08. For phase 2 it is sufficient that the IE has bit 8 = 0 and is undefined in the message.

]

###### 26.5.6.2.3.1	Conformance requirements

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

References

TS GSM 04.08, section 8.6.1.

###### 26.5.6.2.3.2	Test purpose

To verify that an MS supporting the CC protocol for at least one BC receiving a CC message containing an IE unknown in the message which is not encoded as "comprehension required" ignores that IE; this in the special case of a RELEASE message received by the MS having sent in state U10 a DISCONNECT message.

###### 26.5.6.2.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call control entity in CC state U10.

Related PICS/PIXIT statements

MS supports call control protocol for at least one BC (Y/N).

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Test Procedure

The MS is made to send a DISCONNECT message. The SS responds with a RELEASE message containing an optional information element not coded as 'comprehension required' the IEI of which is unknown in the message, but used for a high layer compatability IE in other messages of the protocol. The SS verifies that the MS responds with a RELEASE COMPLETE message; the SS then releases the RR connection.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	MS		The MS is made to initiate call
				clearing.
	2	MS -> SS	DISCONNECT	

	3	SS -> MS	RELEASE	See below.

	4	MS -> SS	RELEASE COMPLETE
	5	SS -> MS	CHANNEL RELEASE	The RR connection is released.

Specific message contents

RELEASE

Information element			value/remark

Unknown IEI			0111 1101

length				1

unknown IE contents		1 arbitrary octet

##### 26.5.6.2.4	Unknown information elements in the non-imperative message part / CC / release complete

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.3, proc/req 5 in ph.1.

- The ph. 1 test must be corrected so that the unknown IE has bit 8 = 0 and is undefined in 04.08. For phase 2 it is sufficient that the IE has bit 8 = 0 and is undefined in the message.

]

###### 26.5.6.2.4.1	Conformance requirements

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

References

TS GSM 04.08, section 8.6.1.

###### 26.5.6.2.4.2	Test purpose

To verify that an MS supporting the CC protocol for at least one BC receiving a CC message containing an IE unknown in the message which is not encoded as "comprehension required" ignores that IE; this in the special case of a RELEASE COMPLETE message received by the MS in state U19.

###### 26.5.6.2.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call control entity in CC state U10.

Related PICS/PIXIT statements

MS supports call control protocol for at least one BC (Y/N).

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Test Procedure

The SS sends a DISCONNECT message. The SS verifies that the MS responds with a RELEASE message. The SS answers with a RELEASE COMPLETE  message containing an optional information element not coded as 'comprehension required' the IEI of which is unknown in the message, but used for an auxiliary states IE in other messages of the protocol. The SS verifies that the MS releases the link after some time.

Maximum duration of test

20 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	

	2	MS -> SS	RELEASE	

	3	SS -> MS	RELEASE COMPLETE	See below.
	4	MS		The MS aborts the RR connection
				(it initiates release of L2 on
				SAPI 0)

Specific message contents

RELEASE COMPLETE

Information element			value/remark

Unknown IEI			0010 0100

length				1

unknown IE contents		1 arbitrary octet

#### 26.5.6.3	Unknown IE in the non-imperative message part, comprehension not required / RR

##### 26.5.6.3.1	Conformance requirements

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

References

TS GSM 04.08, sections 8.6.1, 8.6.2 and 10.5.

##### 26.5.6.3.2	Test purpose

To verify that the MS ignores an IE which is unknown in a message for Radio Resource Management in the special cases of CIPHERING MODE COMMAND, ASSIGNMENT COMMAND and CHANNEL RELEASE.

##### 26.5.6.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state 'idle,updated' and in the RR-idle mode. It has a valid TMSI.

Related PICS/PIXIT statements

Supported frequency bands, PGSM or EGSM or DCS1800.

Foreseen Final State of the MS

The MS is in the MM-state 'idle, updated' and in the RR-idle mode. It has a valid TMSI.

Test Procedure

In the normal call establishment the CIPHERING MODE COMMAND and ASSIGNMENT COMMAND contain additional IEs unknown in the message which are not encoded as "comprehension required", and therefore should be ignored by the MS. After sending an ASSIGNMENT COMPLETE, the subsequent CHANNEL RELEASE received by the MS also contains an IE unknown in a message which is not encoded as "comprehension required". The MS should ignore this IE.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CIPHERING MODE COMMAND	See specific message contents
	6	MS -> SS	CIPHERING MODE COMPLETE	
	7	SS -> MS	ASSIGNMENT COMMAND	See specific message contents
	8	MS -> SS	ASSIGNMENT COMPLETE	On the dedicated channel
	9	SS -> MS	CHANNEL RELEASE	See specific message contents
	10	SS		The SS checks the release of the main
				signalling link at layer 2 level.

Specific message contents

None

Step 5:	CIPHERING MODE COMMAND

Cipher mode setting

-	algorithm identifier	cipher with A5/1

-	SC				start ciphering

Cipher Response				IMEI shall not be included

Unknown IE (type 2)				1001 0010

(attempts to look similar to the cipher response and cipher mode setting IEs)

Step 7:	ASSIGNMENT COMMAND

Channel Description

Channel Type			TCH/F + ACCHs

Timeslot number		arbitrarily selected, but not zero

Training sequence code		arbitrarily selected

Hopping			RF hopping channel

MAIO				0

HSN				0

Power Command			arbitrarily selected

First Unknown IE (Type 2)		1101 1010

Cell Channel Description		For PGSM and EGSM mobiles, bit map 0

encodes ARFCNs 30 and 50. For DCS1800

mobiles, the variable bit map format

encodes ARFCNs 650 and 750.

Second Unknown IE (Type 4)

-	IEI			0110 1001

-	length 			2

-	contents			xxxx xxxx

xxxx xxxx, where x is arbitrarily coded.

Mobile Allocation			For PGSM and EGSM mobiles, indicates ARFCN 50,

only. For DCS1800 mobiles, indicates ARFCN 750,

only.

Step 9:	CHANNEL RELEASE

RR Cause				normal event

Unknown IE (type 4)

-	IEI			0110 0010

-	length 			5

-	contents			xxxx xxxx

xxxx xxxx

xxxx xxxx

xxxx xxxx

xxxx xxxx, where x is arbitrarily coded.

### 26.5.7	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / spare bits

#### 26.5.7.1	Spare bits / RR

##### 26.5.7.1.1	Spare bits / RR / paging channel

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.5.3.6.1.1 in ph.1.

- The postamble now brings the MS back to idle mode.

- In ph 1 the corresponding test should be modified so that the unknown IEI begins with 0 and is undefined in 04.08 ph1. A choice would be 0011 1111. In ph2 an IEI beginning with 0 but used in RR is more interesting, eg. 01100100.

- The test in ph 2 is modified to send at least one non-default octet in the P1 rest octets IE.

]

###### 26.5.7.1.1.1	Conformance requirements

The MS shall ignore the value of spare bits.

References

TS GSM 04.08, section 10.5.

###### 26.5.7.1.1.2	Test purpose

To verify that the MS in the MM-state "idle, updated" and in RR-idle mode ignores the value of spare bits in the special case of the spare bits occurring in the P1 Rest Octets IE of a PAGING REQUEST TYPE 1 message. That the spare bits are ignored is checked by adressing the MS in that PAGING REQUEST message and verifying that the MS responds to that paging.

###### 26.5.7.1.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Related PICS/PIXIT statements

None.

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Test Procedure

The SS sends a PAGING REQUEST TYPE 1 message containing at least one octet in the P1 rest octets IE that is different from 0010 1011.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	See below.
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASS REJ

Specific message contents

PAGING REQUEST TYPE 1

Information element			Value/remark

L2 pseudo length			k+3 where k is the sum of the length of

the mobile identity 1 IE

Page Mode				Normal paging

Channels needed for Mobiles 1 and 2

Channel (first)			Any channel

Channel (second)		(spare)

Mobile identity 1			IMSI or TMSI of MS under test

Mobile identity 2			Omitted

P1 rest octets			not all octets are "0010 1011"

##### 26.5.7.1.2	Spare bits / RR / BCCH

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The postamble now brings the MS back to idle mode.

- The ph 1 test should be modified: only one octet but in both SI3 and 4!! (Only in SI 4 is not enough. Leading 1 in 1111 1111 means type 1 or 2.) Still better: delete test in ph. 1 and replace by a test where L2 fill frames are sent on the BCCH.

- The ph 2 test is based on the "rest octet scheme".

]

###### 26.5.7.1.2.1	Conformance requirements

The MS shall ignore the value of spare bits.

References

TS GSM 04.08, section 10.5.

###### 26.5.7.1.2.2	Test purpose

To verify that the MS in the MM-state "idle, updated" and in RR-idle mode ignores the value of spare bits in the special case where these spare bits are contained in the SI3 and SI4 messages. That the MS ignores the value of the spare bits is checked by changing the LAI in those message and observing the MS initiating a location update though the spare bits do not all have the default value.

###### 26.5.7.1.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Related PICS/PIXIT statements

None.

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Test Procedure

The SS simulates a BCCH where continuously for at least 30 seconds at least one octet of the SI3 Rest Octets IE in all SYSTEM INFORMATION TYPE 3 messages and at least one octet of the SI4 Rest Octets IE in all SYSTEM INFORMATION TYPE 4 messages is different from 0010 1011 and the location area identification IE denotes a location area different from the current location area held by the MS. The SS verifies that the MS sends a CHANNEL REQUEST message on the RACH including the establishment cause "location updating". The SS responds with an IMMEDIATE ASSIGNMENT REJECT message.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS		The SS starts sending modified
				SYSTEM INFORMATION TYPE 3 and
				SYSTEM INFORMATION TYPE 4
				messages (as defined
				below) continuously for at least
				30 seconds on the BCCH.
	2	MS -> SS	CHANNEL REQUEST	Establishment cause = "location
				updating (SDCCH needed). This message may be
				received during the 30 seconds.
	3	SS -> MS	IMM ASS REJ

Specific message contents

SYSTEM INFORMATION TYPE 3

Information element			value/remark

L2 pseudo length			18

cell identity				as required

location area identification		denoting a new location area

control channel description		as required, but with the spare bits

arbitrarily selected and at least one spare bit set to 1.

cell options				as required, but with (spare) bit 8 set to 1

cell selection parameters		as required

RACH control parameters		as required

SI3 rest octets			at least one octet is different from "0010 1011"

SYSTEM INFORMATION TYPE 4

Information element			value/remark

L2 pseudo length			12

location area identification		denoting a new location area

cell selection parameters		as required

RACH control parameters		as required

SI4 rest octets			at least one octet is different from "0010 1011"

##### 26.5.7.1.3	Spare bits / RR / AGCH

###### 26.5.7.1.3.1	Conformance requirements

The MS shall ignore the value of spare bits.

References

TS GSM 04.08, section 10.5.

###### 26.5.7.1.3.2	Test purpose

To verify that the MS in the MM-state "idle, updated" and in RR-idle mode ignores the value of spare bits in the special case of the spare bits occurring in the Page Mode IE, the Spare Half Octet IE, the Channel Description IE, the Timing Advance IE, the IA Rest Octet IE, and in the IAR Rest Octet IE.

###### 26.5.7.1.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Related PICS/PIXIT statements

None.

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Test Procedure

The SS sends an IMMEDIATE ASSIGNMENT message containing arbitary spare bits in the Page Mode IE, in the Spare Half Octet IE, in the Channel Description IE, in the Timing Advance IE, and in the IA Rest Octet IE.

It is checked that the MS answers on the dedicated channel with a PAGING RESPONSE message and releases the main signalling link after a CHANNEL RELEASE message.

After a new paging of the MS an IMMEDIATE ASSIGNMENT REJECT is sent to test the spare bits in the IAR Rest Octet IE.

The MS is then paged again to check the idle state.

Maximum duration of test

20 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	Addressing the MS under test
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	see below
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CHANNEL RELEASE
	6	SS		The SS checks that the MS releases the main
				signalling link and waits 10 seconds for a
				cell reselection of the MS
	7	SS -> MS	PAGING REQUEST	Addressing the MS under test
	8	MS -> SS	CHANNEL REQUEST
	9	SS -> MS	IMM ASS REJECT	normal, waiting time = 0, except the IAR Rest
				Octet IE (see below)
	10	SS		The SS waits six seconds
	11	SS -> MS	PAGING REQUEST	Addressing the MS under test
	12	MS -> SS	CHANNEL REQUEST	To check that the MS has reached the idle state
				after the IMM ASS REJECT

Specific message contents

IMMEDIATE ASSIGNMENT

Information element		Value/remark

L2 pseudo length		sum of the length of all IE except L2 pseudo length and

IA Rest Octets

Protocol Discriminator	RR

Skip Indicator		0000

Message Type		Immediate Assignment

Page mode			xx00 (where 'xx' is arbitrary, with at least 1 bit set to 1)

Spare half octet		xxxx (where 'xxxx' is arbitrary, with at least 1 bit set to 1)

Channel description		normal, no hopping, the two spare bits before

ARFCN are chosen arbitrarily with at least one bit set to 1.

Request reference		normal (derived from the CHANNEL REQUEST)

Timing advance		xx00 0000 (where 'xx' is arbitrary, with at least 1 bit set to 1)

Mobile allocation		chosen so that, together with the channel description

Length		0

IA rest octets

first octet		00xx xxxx (where 'xx xxxx' is arbitrary but different to 10

1011)

other octets		xxxx xxxx (where 'xxxx xxxx' is arbitrary but different to 0010

1011)

IMMEDIATE ASSIGNMENT REJECT

Information element		Value/remark

L2 pseudo length		19

Page mode			normal

Spare half octet		xxxx (where 'xxxx' is arbitrary, with at least 1 bit is set to 1)

Request reference 1		addressing the MS under test

Wait indication 1		0 seconds

.

.	Other Request References and Wait Indications arbitrary

.

IAR rest octets

Octet 1 to 3		xxxx xxxx (where 'xxxx xxxx' is arbitrary but different to

0010 1011)

##### 26.5.7.1.4	Spare bits / RR / Connected Mode

###### 26.5.7.1.4.1	Conformance requirements

The MS shall ignore the value of spare bits.

References

TS GSM 04.08, section 10.5.

###### 26.5.7.1.4.2	Test purpose

To verify that the MS in the MM-state "MM-Connection active" and in RR-Connected mode ignores the value of spare bits in the special case of the spare bits occurring in the Cell Channel Description IE and in the Power Command IE.

###### 26.5.7.1.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters, except:

PGSM and EGSM mobiles are assigned to ARFCN 124 in step 10.

DCS1800 mobiles are assigned to ARFCN 801 in step 10.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Related PICS/PIXIT statements

Mobile's frequency capabilities, PGSM or EGSM or DCS1800.

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Test Procedure

In the procedure of a normal call establishment the ASSIGNMENT COMMAND will be modified to test the spare bits in the Cell Channel Description IE and in the Power Command IE.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	Addressing the MS under test
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CIPHER MODE COMMAND
	6	MS -> SS	CIPHER MODE COMPLETE
	7	SS -> MS	SETUP
	8	MS -> SS	CALL CONFIRMED

A9	MS -> SS	ALERTING

B9	MS ->SS	CONNECT

10	SS -> MS	ASSIGNMENT COMMAND	see below
	11	MS -> SS	ASSIGNMENT COMPLETE	on the dedicated channel
	12	SS -> MS	CHANNEL RELEASE
	13	SS		The SS checks that the MS release the main
				signalling link

Specific message contents

ASSIGNMENT COMMAND

For PGSM and EGSM mobiles

Information element			Value/remark

Channel Description			normal, hopping HSN=63, MAIO=0

Power Command			xxx0 0111 (where 'xxx' is arbitrary, with at least 1 bit set

to 1)

Cell Channel Description

octet 2			00xx 1000 (where 'xx' is arbitrary, with at least 1 bit

set to

1)

octet 3 to 17 (inclusive)		all bits set to zero

Mobile Allocation			indicates ARFCN 124 only

For DCS1800 mobiles

Information element			Value/remark

Channel Description			normal, hopping, HSN=63, MAIO=0

Power Command			xxx0 0111 (where 'xxx' is arbitrary, with at least 1 bit

set to 1)

Cell Channel Description

octet 2			10xx 111? (where 'xx' is arbitrary, with at least 1 bit

set to 1). Bit 1 of octet 2 and all of octets 3 to 17

(inclusive) indicate ARFCN 801 only (using the

variable bit map format).

Mobile Allocation			indicates ARFCN 801 only

#### 26.5.7.2	Spare bits / MM

##### 26.5.7.2.1	Conformance requirements

The MS shall ignore the value of spare bits.

References

TS GSM 04.08, section 10.5.

##### 26.5.7.2.2	Test purpose

To verify that the MS in the MM-state "wait net cmd" and in RR-Connected mode ignores the value of spare bits in the special case of the spare bits occurring in the Cipher Key Seq. Number IE or in the Identity Type IE.

##### 26.5.7.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Related PICS/PIXIT statements

None.

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Test Procedure

After the establishment of the RR-connection, in the AUTHENTIFICATION REQUEST message the spare bits of the Ciphering Key Sequence Number and of the Spare Half Octet IE will be randomly chosen. The spare bits of the Identity Type IE and the Spare Half Octet IE in the IDENTITY REQUEST message will also be chosen arbitrarily.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	Addressing the MS under test
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	AUTHENTIFICATION REQ	see below
	6	MS -> SS	AUTHENTIFICATION RES
	7	SS -> MS	IDENTITY REQUEST	see below
	8	MS -> SS	IDENTITY RESPONSE	with the right TMSI
	9	SS -> MS	CHANNEL RELEASE
	10	SS		The SS checks that the MS release the main
				signalling link

Specific message contents

AUTHENTIFICATION REQ

Information element			Value/remark

Ciphering Key Sequence Number	x000 (where 'x' is set to 1)

Spare Half Octet			xxxx (where 'xxxx' is arbitrary, with at least 1 bit set

to 1)

Auth. Parameter RAND		standard value

IDENTITY REQ

Information element			Value/remark

Identity Type			x100 (where 'x' is set to 1)

Spare Half Octet			xxxx (where 'xxxx' is arbitrary, with at least 1 bit set

to 1)

#### 26.5.7.3	Spare bits / CC

##### 26.5.7.3.1	Conformance requirements

The MS shall ignore the value of spare bits.

References

TS GSM 04.08, section 10.5.

##### 26.5.7.3.2	Test purpose

To verify that the MS in the MM-state "connection established" and in RR-Connected mode ignores the value of spare bits in the special case of the spare bits occurring in the Calling Party BCD Number IE, Calling Party Subaddress IE, Called Party Subaddress IE, Cause IE and Progress Indicator IEs.

##### 26.5.7.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Related PICS/PIXIT statements

None.

Foreseen Final State of the MS

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Test Procedure

After the establishment of the MM-connection, in the SETUP message the spare bits of the Calling Party BCD Number, Calling Party Subaddress and Called Party Subaddress will be arbitrarily chosen and also in the DISCONNECT message the spare bits of the Progress Indicator IE and of the Cause IE will be arbitrarily chosen.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	Addressing the MS under test
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	AUTHENTIFICATION REQ
	6	MS -> SS	AUTHENTIFICATION RES
	7	SS -> MS	CIPHER MODE COMMAND
	8	MS -> SS	CIPHER MODE COMPLETE
	9	SS -> MS	SETUP	see below
	10	MS -> SS	CALL CONFIRMED

A11	MS -> SS	CONNECT
	B11	MS -> SS	ALERTING

B12	MS -> SS	CONNECT

13	SS -> MS	ASSIGNMENT COMMAND
	14	MS -> SS	ASSIGNMENT COMPLETE
	15	SS -> MS	CONNECT ACKNOWLEDGE
	16	SS -> MS	DISCONNECT	see below
	17	SS -> MS	STATUS ENQUIRY
	18	MS -> SS	STATUS	with actual call state U12
	19	SS -> MS	RELEASE
	20	MS -> SS	RELEASE COMPLETE
	21	SS -> MS	CHANNEL RELEASE

Specific message contents

SETUP

Information element			Value/remark

Calling Party BCD Number

IEI

length				3

octet 3			0000 0000

octet 3a			100x xx00 (where 'x' is chosen arbitrarily, with at

least one bit set to 1)

octet 4			0000 0001

Calling Party Subaddress

IEI

length				3

octet 3			1000 0xxx (where 'x' is chosen arbitrarily, with at

least one bit set to 1)

octet 4			0101 0000 (AFI: request IA5 charater)

octet 5			0000 0001

Called Party Subaddress

IEI

length				3

octet 3			1000 0xxx (where 'x' is chosen arbitrarily, with at

least one bit set to 1)

octet 4			0101 0000 (AFI: request IA5 character)

octet 5			0000 0001

DISCONNECT

Information element			Value/remark

Cause

Length			2

octet 3			111x 0000 (where 'x' is set to 1)

octet 4			1000 0001

Progress Indicator

IEI

Length			2

octet 3			111x 000 (where 'x' is set to 1)

progress description		8 (in band info now available)

### 26.5.8	Default contents of messages

Default requirements for messages that are not mentioned in this section are given in section 26.8.4.

CHANNEL RELEASE

Information element			Value/remark

RR cause				Normal event

CHANNEL REQUEST

DISCONNECT (SS -> MS)

Information element			Value/remark

Cause

Coding standard		Standard defined for the GSM PLMNS

Location			user

Cause value				#16

IDENTITY REQUEST

Information element			Value/remark

Identity type				Depending on test

Spare half octet			0000

IMMEDIATE ASSIGNMENT

Information element			Value/remark

L2 pseudo length   			n, where n is the L2 pseudo length of the message

Page mode				arbitrary

Spare half octet			0000

Channel description			a valid description of an SDCCH + SACCH

Request reference			Corresponding to the last CHANNEL REQUEST

received from the MS

Timing advance			arbitrary

Mobile allocation			chosen so that, together with the channel description

IE, it describes a valid SDCCH + SACCH

Starting time			Omitted

IA rest octets			m octets, each coded as H'2B, where m = 22 - n

IMMEDIATE ASSIGNMENT REJECT

Information element			Value/remark

L2 pseudo length   			19

Page mode				arbitrary

Spare half octet			0000

Request reference 1			corresponding to the last CHANNEL REQUEST

received from the MS

Wait indication 1			0 seconds

Request reference 2			arbitrary

Wait indication 2			0 seconds

Request reference 3			arbitrary

Wait indication 3			0 seconds

Request reference 4			arbitrary

Wait indication 4			0 seconds

IA rest octets			3 octets, each coded as H'2B

PAGING REQUEST TYPE 1

Information element			Value/remark

L2 pseudo length   			n where n is the sum of the mobile

identity 1 IE and 3

Page Mode				Normal paging

Channels needed for Mobiles 1 and 2

Channel (first)			Any channel

Channel (second)		(spare)

Mobile identity 1			IMSI or TMSI of MS under test

Mobile identity 2			Omitted

P1 rest octets			m octets, each coded as H'2B, where m = 22 - n

PAGING RESPONSE

RELEASE COMPLETE (MS -> SS)

No default requirements defined for this message.

RELEASE COMPLETE (SS -> MS)

Information element			Value/remark

Cause

Coding standard		Standard defined for the GSM PLMNS

Location			user

Cause value				#16

STATUS (MS -> SS)

Information element			Value/remark

Cause

Length			length of cause IE

Coding standard		Standard defined for the GSM PLMNS

Location			user

Cause value				as defined in test

Call state				as defined in test

STATUS ENQUIRY (SS -> MS)

Information element			Value/remark

Transaction identifier			relating to the active call

## 26.6	Test of the elementary procedures for radio resource management

NOTE 1:	For SS implementor : if tests are concatenated, it is important that unused fields in IMMEDIATE ASSIGNMENT REJECT messages do not use Request References that relate to CHANNEL REQUEST messages recently transmitted by the MS.

### 26.6.1	Immediate assignment

The immediate assignment procedure is used by the network to establish a dedicated control channel for the MS and network to communicate the detail of the service requested. If the Mobile Station does not implement the procedure correctly, radio resources can be wasted as the Mobile Station might use the wrong channels.

#### 26.6.1.1	Immediate assignment / SDCCH or TCH assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.3 in phase 1 version.

-	Timing Advance is no longer specified]

##### 26.6.1.1.1	Conformance requirement

1.	Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on the SDCCH/8 described in the IMMEDIATE ASSIGNMENT message.

2.	Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on the TCH/FACCH described in the IMMEDIATE ASSIGNMENT message.

Reference(s)

GSM 04.08 section 3.3.1.2.1

##### 26.6.1.1.2	Test purpose

To verify that the MS can correctly set up a dedicated SDCCH control channel and that the MS can correctly set up a dedicated TCH/FACCH control channel.

##### 26.6.1.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, except that CCCH-CONF is set to '1 basic physical channel used for CCCH not combined with SDCCHs'.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statement(s)

Type of MS (P-GSM900 or EGSM or DCS1800).

Support rate(s) of TCH: TCH/F and/or TCH/H.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST message the SS assigns an SDCCH. The MS shall go to the correct channel and send a PAGING RESPONSE message. Then the SS initiates RR-release by sending a CHANNEL RELEASE message.

If TCH/F is supported by the MS, the test is repeated with the SS assigning a TCH/F.

If TCH/H is supported by the MS, the test is repeated with the SS assigning a TCH/H.

Maximum Duration of Test

6 seconds per value of the execution timer

Expected Sequence

This sequence is performed for execution counter, K = 1, 2, 3 (unless the TCH is not supported).

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	Channel Type : see below
	4	MS -> SS	PAGING RESPONSE	Shall be sent on the correct channel
	5	SS -> MS	CHANNEL RELEASE

Specific Message Contents

IMMEDIATE ASSIGNMENT

K=1, SDCCH test : Channel Type = SDCCH/8

K=2, TCH/F test : Channel Type = Bm + ACCHs

K=3, TCH/H test : Channel Type = Lm + ACCHs, subchannel randomly drawn

#### 26.6.1.2	Immediate assignment / extended assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.5 in phase 1 version.

-	two procedures have been concatenated to one test.

-	in the last part of the test the MS shall continue to send CHANNEL REQUESTs]

-

]

NOTE 2:	In these tests the SS must send the immediate assignment messages in due time to allow for the MS to receive them and send a PAGING RESPONSE rather than another random access. This applies to the whole of section 26.

##### 26.6.1.2.1	Conformance requirements

1.	The MS shall go to the allocated SDCCH/4 and send a PAGING RESPONSE message containing its identity and its classmark.

2.	The MS shall go to the allocated SDCCH/8 and send a PAGING RESPONSE message containing its identity and its classmark.

3.	The MS shall correctly identify its own assignment in either the Request Reference 1 or the Request Reference 2 information element in an extended assignment message.

4.	The MS shall only react to an Immediate Assignment which references one of the last 3 CHANNEL REQUEST messages from the MS.

References

GSM 04.08 section 3.3.

##### 26.6.1.2.2	Test purpose

To verify that the MS goes to the allocated SDCCH/4 and sends a PAGING RESPONSE message containing its identity and its classmark.

To verify that the MS goes to the allocated SDCCH/8 and sends a PAGING RESPONSE message containing its identity and its classmark.

To verify that the MS can correctly identify its own assignment in either the Request Reference 1 or the Request Reference 2 information element in an extended assignment message.

To verify that the MS only reacts to an Immediate Assignment which references one of the last 3 CHANNEL REQUEST messages from the MS.

##### 26.6.1.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-retrans is set to 7.

Mobile Station:

The MS is in the "idle, updated" state. with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with a TMSI allocated.

Test Procedure

In the first part of the test, the SS pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the n-th CHANNEL REQUEST message (n being randomly chosen by the SS from the set {1, 2 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message, which references one of the last 3 CHANNEL REQUEST messages from the MS. The MS shall then go to the correct channel and send a PAGING RESPONSE message. The SS will then release the channel.

In the second part of the test, the SS again pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the k-th CHANNEL REQUEST message (k being randomly chosen by the SS from the set {4, 5 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message which, instead of referencing one of the last 3 CHANNEL REQUEST messages from the MS, references an earlier CHANNEL REQUEST message. The MS shall then ignore the IMMEDIATE ASSIGNMENT EXTENDED message and continue to send CHANNEL REQUEST messages until the Max-Retrans value has been reached. Then a period of 7 seconds shall elapse in order to allow the MS to perform cell reselection (this allows for the time between the last CHANNEL REQUEST message and the beginning of cell reselection).

In the third part of the test, the CCCH_CONF of the SS is set to non-combined and the SS pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the r-th CHANNEL REQUEST message (r being randomly chosen by the SS from the set {4, 5 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message which, in the second request reference, references one of the last 3 CHANNEL REQUEST messages from the MS. The associated Channel Description allocates SDCCH(S) (S being randomly chosen by the SS from the set {0,1 ... 7}). The MS shall then go to the correct channel and send a PAGING RESPONSE message. The SS will then release the channel.

Maximum Duration of Test

90 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	n CHANNEL REQUESTs (n being
				randomly drawn from {1.... 8}
				are sent, all with Establ. Cause =
				"Answer to paging"
	3	MS -> SS	CHANNEL REQUEST
	4	SS -> MS	IMM ASSIGN EXT 	see note 3
	5	MS -> SS	PAGING RESPONSE
	6	SS -> MS	CHANNEL RELEASE
	7	--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	8	SS -> MS	PAG REQ TYPE 1
	9	MS -> SS	CHANNEL REQUEST	k CHANNEL REQUESTs (k being
				randomly drawn from the set {4, 5, 
				8}) are sent all with
	10	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	11	SS -> MS		IMM ASSIGN EXT	see note 4
	12	MS -> SS		CHANNEL REQUEST 8-k CHANNEL REQUESTs
				are sent, all with Establ. Cause = "Answer to
	13	MS -> SS	CHANNEL REQUEST	paging"
	14	SS		The SS verifies that the MS does not
				transmit any Layer 2 frames for at least
				3 seconds.
	15	 SS		The SS sets CCCH_CONF to
				non-combined
	16	 SS		The SS waits 40 seconds to
				allow the MS to perform
				cell reselection and to
				read the BCCH information
	17	SS -> MS	PAG REQ TYPE 1	
	18	MS -> SS	CHANNEL REQUEST	r CHANNEL REQUESTs (r being
				randomly drawn from {4, 5... 8}
				are sent, all with Establ. Cause =
				"Answer to paging"
	19	MS -> SS	CHANNEL REQUEST
	20	SS -> MS	IMM ASSIGN EXT 	see note 5
	21	MS -> SS	PAGING RESPONSE
	22	SS -> MS	CHANNEL RELEASE

Specific Message Contents

NOTE 3: 	The first Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {max (1,n-2) ... n}, its value being randomly drawn by the SS. The second Request Reference shall be different from any Request Reference the MS has generated in this test.

NOTE 4: 	The first Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {1 ... k-3}, its value being randomly drawn by the SS. The second Request Reference shall be different from any Request Reference the MS has generated in this test.

NOTE 5: 	The second Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {r-2, r-1, r}, its value being randomly drawn by the SS. The first Request Reference shall be different from any Request Reference the MS has generated in this test.

#### 26.6.1.3	Immediate assignment / assignment rejection

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.6 in phase 1 version.

-	two procedures have been concatenated to one test.

-	there is an IMMEDIATE ASSIGNMENT REJECT added at the end of the test to avoid a cell reselection.]

##### 26.6.1.3.1	Conformance requirements

1.	The MS shall respond to the Paging Request message by sending a Channel Request message with establishment cause set to "Answer to Paging". After the reception of IMMEDIATE ASSIGNMENT REJECT, the MS shall not transmit during the time indicated in the "Wait Indication" field of the IMMEDIATE ASSIGNMENT REJECT message, and then it shall answer to the new paging requests.

2.	After an assignment rejection, the MS shall perform a cell reselection (idle mode operation) and the MS shall not transmit unless a different cell is selected.

References

GSM 04.08, section 3.3.1.2.2.

##### 26.6.1.3.2	Test purpose

To verify that the MS can accept an Immediate Assignment Rejection.

To verify that the MS can respond to paging  after an Immediate Assignment Rejection is received on a different cell.

##### 26.6.1.3.3	Method of test

Initial Conditions

System Simulator:

2 cells with the same LAI, Max-Retrans is 7.

Mobile Station:

The MS is camped on cell A and is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The MS is camped on cell B and is in the "idle, updated" state, with a TMSI allocated.

Test Procedure

The SS pages the MS, which shall react by sending CHANNEL REQUESTs. Immediately after reception of the n-th CHANNEL REQUEST (n being an integer from the set {1, 2 ... 8}, randomly chosen by the SS) the SS sends an IMMEDIATE ASSIGNMENT REJECT message, which references one of the last 3 CHANNEL REQUESTs from the MS, and with the Wait Indication set to x seconds (x being an integer from the set {5, 6 ... 255}, randomly chosen by the SS). The SS continues to send paging messages for that mobile station in every block of the mobile station's paging subgroup for x+2 seconds. The MS shall only answer with a CHANNEL REQUEST after x seconds have elapsed.

The SS responds to this CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message with the Wait Indication set to 255 seconds.

Immediately afterwards the SS changes the power levels so the MS selects cell B. After 12 seconds have elapsed the SS pages the MS in cell B and the MS shall answer to this page. In order to avoid another cell reselection the SS then sends another IMMEDIATE ASSIGNMENT REJECT.

Maximum Duration of Test

5 minutes.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	n CHANNEL REQUESTs (n being
	.	.	.	randomly drawn from the set {1, 2
	.	.	.	 ... 8}) are sent, all with
	1+n	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	2+n	SS -> MS	IMM ASSIGN REJ	1st, 3rd and 4th Request
				References are different to all 
				n Request References received from
				the MS under test.
				2nd Request Reference : see note 6.
				2nd Wait Indication = x seconds (x
				being randomly drawn from the set
				{5 ,6 ... 255}
	3+n	SS -> MS	PAG REQ TYPE 1	Sent for more than x seconds
	.	.	.
	.	.	.
	k	SS -> MS	PAG REQ TYPE 1	between x and x+2 seconds (note 7)
	k+1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	k+2	SS -> MS	IMM ASSIGN REJ	1st, 2nd and 4th Request
				References are different to all 
				n Request References received from
				the MS.
				The 3rd Request Reference pertains
				to the last CHANNEL REQUEST
				sent by the MS.
				The 3rd Wait Indication is 255
				seconds
	k+3	--------	----------	Raise power level of cell B, lower
				power level of cell A until the MS
				selects cell B
	k+4	SS -> MS	PAG REQ TYPE 1	Sent once, 12 seconds after the change
				of levels
	k+5	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	k+6	SS -> MS	IMM ASSIGN REJ

NOTE 6:	The Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer from the set {max(1,n-2) ... n}, its value being randomly drawn by the SS.

NOTE 7:	the value of k is not important in this test.

Specific Message Contents

None.

#### 26.6.1.4	Immediate assignment / ignore assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.7 in phase 1 version.

-	two tests have been concatenated to one.

(* it is not clear that there were originally two tests : it is possible to do as one test. How have R+S implemented the FTA? *)

-	an IMMEDIATE ASSIGNMENT REJECT has been added to the end of the test in order to avoid a cell reselection

-	Timing Advance is no longer included]

##### 26.6.1.4.1	Conformance requirements

1.	An MS waiting for a response from the network, following the sending of a CHANNEL REQUEST, shall ignore an IMMEDIATE ASSIGNMENT message with a request reference containing a wrong frame number.

2.	An MS is waiting for an assignment of its own, shall ignore an IMMEDIATE ASSIGNMENT message with a request reference containing a wrong random access information.

References

GSM 04.08 section 3.3.1.2.1

##### 26.6.1.4.2	Test purpose

To verify that the MS ignores an assignment for another MS while it is waiting for an assignment of its own.

##### 26.6.1.4.3	Method of test

Initial Conditions

System Simulator:

1 cell

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS, which reacts with CHANNEL REQUESTs. The SS responds to the first CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT containing a wrong Request Reference (in the first run of the test the frame number is wrong, in the repetition it is the random access info that is wrong). It is verified for 2 seconds that the MS does not start signalling on the SDCCH. The MS shall ignore the assignment and send another CHANNEL REQUEST message. In order to avoid cell reselection the SS now answers with a correct IMMEDIATE ASSIGNMENT REJECT and repeats the test once.

Maximum Duration of Test

12 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Frame number in Request Reference
				is 2 too high. The MS shall not
				start signalling on the assigned
				SDCCH. This is verified for a
				period of 2 seconds
	4	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	5	SS -> MS	IMM ASSIGN REJ	
	6	SS		SS waits for 6 seconds
	7	SS -> MS	PAG REQ TYPE 1
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	SS -> MS	IMM ASSIGNMENT	Random access info in Request
				Reference is wrong. The MS shall
				not start signalling on the
				assigned SDCCH. This is verified
				for a period of 2 seconds
	10	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	11	SS -> MS	IMM ASSIGN REJ

Specific Message Contents

None.

#### 26.6.1.5	Immediate assignment after immediate assignment reject

##### 26.6.1.5.1	Conformance requirement

Following an IMMEDIATE ASSIGNMENT REJECT message, the MS shall listen for IMMEDIATE ASSIGNMENTS until T3126 expires.

Reference

GSM 04.08 section 3.3.1.3.2.

##### 26.6.1.5.2	Test purpose

To verify that the MS correctly responds to an IMMEDIATE ASSIGNMENT message sent after an IMMEDIATE ASSIGNMENT REJECT message.

##### 26.6.1.5.3	Method of test

Initial conditions

System Simulator

1 cell

CCCH-CONF is set to '1 basic physical channel used for CCCH not combined with SDCCHs'

Max retrans is set to 7

TX-integer is set to 7

Mobile Station

The MS is in "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

Type of MS (P-GSM900 or EGSM or DCS1800).

Forseen final state of the MS

"idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS, which shall react by sending CHANNEL REQUESTs. Immediately after reception of the third CHANNEL REQUEST the SS sends an IMMEDIATE ASSIGNMENT REJECT message which references the first CHANNEL REQUEST from the MS and has the Wait Indication IE set to 6 seconds.

Between 0.75 seconds and 1.25 seconds after sending the IMMEDIATE ASSIGNMENT REJECT message the SS sends an IMMEDIATE ASSIGNMENT message referencing the second CHANNEL REQUEST message, and assigning an SDCCH. The MS shall go to the correct channel and send a PAGING RESPONSE message. Then the SS initiates RR-release by sending a CHANNEL RELEASE message.

Maximum duration of test

10 seconds

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	first request
	3	MS -> SS	CHANNEL REQUEST	second request
	4	MS -> SS	CHANNEL REQUEST	third request
	5	SS -> MS	IMM ASSIGNMENT REJ	references the first request from MS, Wait
				Indication IE set to 6 seconds.
	6	SS -> MS	IMM ASSIGNMENT	references the second request from the MS
				Channel type set to SDCCH/8
				message sent between 0.75s and 1.25s after the
				completion of step 5
	7	MS -> SS	PAGING RESPONSE	shall be sent on the correct channel
	8	SS -> MS	CHANNEL RELEASE

Specific message contents

None.

### 26.6.2	Test of paging

The Paging procedure is used by the network to cause the Mobile Station to establish a radio connection. Normally the Mobile Station listens to its paging subchannel, but this can be modified by the use of different page modes. The correct implementation of the paging procedure in the Mobile Station is essential for the basic establishment of a connection.

#### 26.6.2.1	Normal paging

##### 26.6.2.1.1	Paging / normal / type 1

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.3 in phase 1 version.

-	6 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.

]

(* step 5 has been changed for phase 2 imsi response to imsi paging.*)

(* steps 25 and 26 may be deleted as ignoring IMEI pages is difficult to 'justify'.*)

###### 26.6.2.1.1.1	Conformance requirements

1.	The MS shall respond correctly to various PAGING REQUEST TYPE 1 messages, when the page mode is set to normal paging, in the following cases:

1.1	The MS is addressed with its IMSI in the first Mobile Identity field. The optional Mobile Identity field is not present.

1.2.	The MS is addressed with its TMSI in the first Mobile Identity field. The optional Mobile Identity field specifies an IMSI different from that of the MS.

1.3.	The first Mobile Identity field specifies a TMSI different from that of the MS. The optional Mobile Identity field addresses the MS by its IMSI.

1.4	The first Mobile Identity field specifies a TMSI different from that of the MS. The optional Mobile Identity field contains the correct TMSI of the MS.

2.	An MS shall ignore PAGING REQUEST TYPE 1 messages with incorrect information, when the page mode is set to normal paging, in the following case:

2.1	The MS is addressed with its TMSI in the first Mobile Identity field, but the type of identity in this field is set to "No Identity". The optional Mobile Identity field is not present.

References

GSM 04.08 section 3.3.2, GSM 05.02 section 6.5.

###### 26.6.2.1.1.2	Test purpose

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 1 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested. It is tested that the MS responds with the same type of identity that is used in the PAGING REQUEST TYPE 1 message. It is tested that the MS ignores fill paging.

###### 26.6.2.1.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS 5 times with different PAGING REQUEST TYPE 1 messages on the paging subchannel which corresponds to the MS's IMSI.

In the first 4 cases, where the MS is addressed by its IMSI or its TMSI, the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

In the last case, it is tested that the MS does not answer to paging that does not address it.

Maximum Duration of Test

2 min

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains IMSI of
				MS, 2nd Mobile Ident not present.
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to 
				the message received in step 3
	5	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.
	6	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	7	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of
				MS, 2nd Mobile Ident contains IMSI
				of another MS.
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 9
	11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	12	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	13	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of
				another MS, 2nd Mobile Ident
				contains IMSI of MS.
	14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 15
	17	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.
	18	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	19	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of
				another MS, 2nd Mobile Ident
				contains TMSI of MS.
	20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 21
	23	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	24	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection

	25	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of
				MS but with type of identity set
				to "no identity", second Mobile
				Ident not present.

	26	--------	--------	During 1 second, the SS checks
				that the MS does not produce any
				Layer 2 frames.

(* In steps 26 and 28, it may be sufficient to check for no layer 3 messages rather than checking for no RF-transmission.*)

Specific Message Contents

None.

##### 26.6.2.1.2	Paging / normal / type 2

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.5 in phase 1 version.

-	6 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.]

(* imsi response to imsi paging is included.*)

(* imei paging test may not be 'justified' *)

###### 26.6.2.1.2.1	Conformance requirements

1.	The MS shall respond correctly (by sending CHANNEL REQUEST messages with an Establishment Cause set to "Answer to Paging") to various PAGING REQUEST TYPE 2 messages, when the page mode is set to normal paging, in the following cases:

1.1	The MS is addressed in the first TMSI field.

1.2	The MS is addressed in the second TMSI field.

1.3	The MS is addressed in the optional Mobile Identity field with its TMSI.

1.4	The MS is addressed in the optional Mobile Identity field with its IMSI.

2.	The MS shall ignore PAGING REQUEST TYPE 2 messages with incorrect information, when the page mode is set to normal paging, in the following case:

2.1	The MS is addressed in the optional Mobile Identity field with its TMSI, but the type of identity in this field is set to "No Identity".

References

GSM 04.08 section 3.3.2.

###### 26.6.2.1.2.2	Test purpose

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 2 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested. It is tested that the MS responds with the same type of identity that is used in the PAGING REQUEST TYPE 2 message. It is tested that the MS ignores a PAGING REQUEST TYPE 2 message that does not address it.

###### 26.6.2.1.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS 5 times with different PAGING REQUEST TYPE 2 messages on the paging subchannel which corresponds to the MS's IMSI.

In the first 4 cases, where the MS is addressed by its IMSI or by its TMSI, the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second request by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

In the last case, it is tested that the MS does not answer to paging that does not address it.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses MS, 2nd TMSI
				addresses another MS, Mobile
				Identity IE not present
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 3
	5	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	6	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	7	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd
				TMSI addresses MS, Mobile Identity
				IE not present.
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 9
	11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	12	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	13	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd
				TMSI addresses another MS, Mobile
				Identity IE contains TMSI of MS.
	14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 15
	17	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	18	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	19	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd
				TMSI addresses another MS, Mobile
				Identity IE contains IMSI of MS.
	20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 21
	23	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.
	24	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection

	25	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd
				TMSI addresses another MS, Mobile
				Identity IE contains TMSI of MS
				but with type of identity set to
				"no identity".

	26	--------	--------	During 1 second, the SS checks
				that the MS does not produce any
				Layer 2 frames.

(* RF transmission ==> L3 messages ?*)

Specific Message Contents

None.

##### 26.6.2.1.3	Paging / normal / type 3

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.7 in phase 1 version.

-	4 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.]

###### 26.6.2.1.3.1	Conformance requirements

An MS shall respond correctly to various PAGING REQUEST TYPE 3 messages, when the page mode is set to normal paging. The MS shall send CHANNEL REQUEST messages, with an Establishment Cause set to "Answer to Paging", until the network answers. The number of CHANNEL REQUEST messages shall be limited by the parameter Max-retrans. After the assignment procedure, the MS shall send a PAGING RESPONSE message on the channel assigned by the network.

References

GSM 04.08 section 3.3.2.

###### 26.6.2.1.3.2	Test purpose

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 3 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested.

###### 26.6.2.1.3.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS 4 times with different PAGING REQUEST TYPE 3 messages on the paging subchannel which corresponds to the MS's IMSI.

In all the cases the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second request by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 3	1st TMSI addresses MS; 2nd, 3rd
				and 4th TMSIs address other MSs.
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 3
	5	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	6	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	7	SS -> MS	PAG REQ TYPE 3	2nd TMSI addresses MS; 1st, 3rd
				and 4th TMSIs address other MSs.
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 9
	11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	12	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	13	SS -> MS	PAG REQ TYPE 3	3rd TMSI addresses MS; 1st, 2nd
				and 4th TMSIs address other MSs.
	14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 15
	17	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	18	SS -> MS	CHANNEL RELEASE
		--------	--------	The SS waits 12 seconds to allow
				the MS to perform cell reselection
	19	SS -> MS	PAG REQ TYPE 3	4th TMSI addresses MS; 1st, 2nd
				and 3rd TMSIs address other MSs.
	20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to
				the message received in step 21
	23	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.
	24	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None.

#### 26.6.2.2	Paging / extended

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	two procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.

-	Instead of specifying a specific page mode , the mode is now chosen arbitrarily.*)

-	It is specified that in step 6 the random reference is not one used by the MS.]

##### 26.6.2.2.1	Conformance requirements

1.	The MS shall operate in the extended page mode when this is ordered by the network in a PAGING REQUEST TYPE 1 message not addressing the MS but on the paging subchannel which corresponds to the MS's identity.

2.	The MS shall operate in the extended page mode when this is ordered by the network in a PAGING REQUEST TYPE 2 message not addressing the MS but on the paging subchannel which corresponds to the MS's identity.

3.	The MS shall operate in the extended page mode when this is ordered by the network in a PAGING REQUEST TYPE 3 message not addressing the MS but on the paging subchannel which corresponds to the MS's identity.

4.	The MS shall operate in the extended page mode when this is ordered by the network in an IMMEDIATE ASSIGNMENT message on the paging subchannel which corresponds to the MS's identity.

5.	The MS shall operate in the extended page mode when this is ordered by the network in an IMMEDIATE ASSIGNMENT EXTENDED message on the paging subchannel which corresponds to the MS's identity.

References

GSM 04.08 section 3.3.2.1; GSM 05.02, section 6.5

##### 26.6.2.2.2	Test purpose

To test that the MS is operating in the extended page mode when this is ordered by the SS in either a PAGING REQUEST message or an IMMEDIATE ASSIGNMENT message.

##### 26.6.2.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS sends a PAGING REQUEST TYPE 1 message not addressing the MS under test but on the paging subchannel which corresponds to the MS's identity. The page mode is set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying an arbitrarily chosen page mode and addressing the MS by its TMSI. The MS shall respond to the last page by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

The SS then sends an IMMEDIATE ASSIGNMENT on the paging subchannel which corresponds to the MS's identity. The random reference is different to those used by the Mobile Station in the last two CHANNEL REQUEST messages. (Phase 2 requires a Mobile Station to react on an IMMEDIATE ASSIGNMENT after a rejection.) The page mode is again set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 2 message specifying an arbitrarily chosen page mode and addressing the MS by its TMSI. The MS shall respond with CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

The SS then sends an IMMEDIATE ASSIGNMENT EXTENDED on the paging subchannel which corresponds to the MS's identity. The random references are different to those used by the Mobile Station in the last three CHANNEL REQUEST messages. The page mode is again set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 3 message specifying an arbitrarily chosen page mode and addressing the MS by its TMSI. The MS shall respond with CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

The SS then sends a PAGING REQUEST TYPE 3 message not addressing the MS under test but on the paging subchannel which corresponds to the MS's identity. The page mode is set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 2 message specifying an arbitrarily chosen page mode and addressing the MS by its IMSI. The MS shall respond to the last page by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

The SS then sends a PAGING REQUEST TYPE 2 message not addressing the MS under test but on the paging subchannel which corresponds to the MS's identity. The page mode is set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying an arbitrarily chosen page mode and addressing the MS by its IMSI. The MS shall respond to the last page by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

[editorial note : If it is an essential feature that the MS ignores paging on other subchannels, later, a test shall be elaborated, which tests that the MS does not take into account the paging reorganisation (only page modes on the MS's own paging subchannel shall be taken into account). However it seems not reasonable to forbid the MS listening to e

Maximum Duration of Test

10 sec

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	Mobile Ident : IMSI of another MS,
				page mode = "extended paging"
	2	SS -> MS	PAG REQ TYPE 1	Sent in the next but one paging
				subblock. Page mode 
				is arbitrarily chosen
				Mobile Ident : TMSI of the MS.
	3	MS -> SS	CHANNEL REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMM ASSIGN REJ	Wait indication = 5 seconds
				page mode = normal
	6	SS		SS waits for 5 seconds
		SS -> MS	IMM ASSIGNMENT	Sent in the paging subblock of
				MS under test. Page mode =
				"extended paging", Request
				reference chosen arbitrarily by
				the SS, but different from all
				references used earlier in this
				test sequence
	8	SS -> MS	PAG REQ TYPE 1	Sent in the next but one paging
				subblock. Page mode is arbitrarily
				chosen. Mobile Ident : TMSI of
				the MS.
	9	MS -> SS	CHANNEL REQUEST	
	10	MS -> SS	CHANNEL REQUEST	
	11	SS -> MS	IMM ASSIGN REJ	Wait indication = 5 seconds
				page mode = normal
	12	SS	--	SS waits for 5 seconds
	13	SS -> MS	IMM ASSIGNMENT EXT	Sent in the paging subblock of
				MS under test. Page mode =
				"extended paging", Request
				references chosen arbitrarily by
				the SS, but different from all
				references used earlier in this
				test sequence
	14	SS -> MS	PAG REQ TYPE 3	Sent in the next but one paging
				subblock. Page mode is arbitrarily
				chosen. Mobile Ident : TMSI of
				the MS.
	15	MS -> SS	CHANNEL REQUEST	
	16	MS -> SS	CHANNEL REQUEST	
	17	SS -> MS	IMM ASSIGN REJ	Wait indication = 5 seconds
				page mode = normal
	18	SS	--	SS waits for 5 seconds
	19	SS -> MS	PAG REQ TYPE 3	Sent in the paging subblock of
				MS under test. Page mode =
				"extended paging", 
	20	SS -> MS	PAG REQ TYPE 2	Sent in the next but one paging
				subblock. Page mode is arbitrarily
				chosen. Mobile Ident : IMSI of
				the MS.
	21	MS -> SS	CHANNEL REQUEST	
	22	MS -> SS	CHANNEL REQUEST	
	23	SS -> MS	IMM ASSIGN REJ	Wait indication = 5 seconds
				page mode = normal
	24	SS	--	SS waits for 5 seconds

25	SS -> MS	PAG REQ TYPE 2	Sent in the paging subblock of
				MS under test. Page mode =
				"extended paging", 
	26	SS -> MS	PAG REQ TYPE 1	Sent in the next but one paging
				subblock. Page mode is arbitrarily
				chosen. Mobile Ident : IMSI of
				the MS.
	27	MS -> SS	CHANNEL REQUEST	
	28	MS -> SS	CHANNEL REQUEST	
	29	SS -> MS	IMM ASSIGN REJ	Wait indication = 5 seconds

Specific Message Contents

None.

#### 26.6.2.3	Paging / reorganisation

##### 26.6.2.3.1	Paging / reorganisation / procedure 1

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.3 in phase 1 version.

-	the paging parameters are chosen   arbitrarily by the SS

-	type of identity in the first PAGING REQUEST TYPE 1 is no longer specified as IMSI.]

(* half of this test can probably be deleted -- however as these are the 2nd and 4th quarters it would disrupt the SS !!! *)

###### 26.6.2.3.1.1	Conformance requirements

1.	An MS, after reception of a message with page mode set to "paging reorganisation", shall answer to paging messages (with page mode set to "normal paging") sent on its old CCCH in paging blocks which do not belong to the MS's paging sub-channel.

2.	When the network changes the paging group of the MS by modifying BCCH parameters (CCCH-CONF set to "1 basic physical channel used for CCCH combined with SDCCH", and BS-AG-BLKS-RES set to "2 blocks reserved for access grant"), the MS shall calculate its new paging group and answer to paging messages on its new paging subchannel.

3.	When the network changes the paging group of the MS by modifying BCCH parameters (CCCH-CONF set to "2 basic physical channels used for CCCH, not combined with SDCCHs" and BS-AG-BLKS-RES set to "2 blocks reserved for access grant"), the MS shall calculate its new paging group and answer to paging messages on its new paging subchannel.

References

GSM 04.08 section 3.3.2.1; GSM 05.02, section 6.5.

###### 26.6.2.3.1.2	Test purpose

To test that the MS correctly determines its new paging subchannel when the CCCH structure is changed from non-combined to combined.

###### 26.6.2.3.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, CCCH-CONF set to "1 basic physical channel used for CCCH, not combined with SDCCHs", a legal combination of BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS, with the exception that BS-PA-MFRMS shall not be set to 9.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated. The IMSI of the MS is from a defined/default range that ensures its paging channel changes when the broadcast parameters are changed.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS sends an IMMEDIATE ASSIGNMENT EXTENDED message on the MS's paging subchannel, with the page mode element set to "paging reorganisation" and Request References that do not pertain to the MS. Before the MS's original paging subchannel re-occurs, the SS pages it on the CCCH corresponding to the Mobile Station's IMSI with a PAGING REQUEST TYPE 2 message (page mode = normal paging) containing the MS's TMSI in some paging block which does not belong to the Mobile Station's paging subchannel. The MS shall respond by sending CHANNEL REQUESTs.The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection) on an arbitrarily selected paging subchannel.

Then the SS starts sending messages (PAGING REQUEST TYPE 1 or PAGING REQUEST TYPE 2 or PAGING REQUEST TYPE 3 or IMMEDIATE ASSIGNMENT or IMMEDIATE ASSIGNMENT EXTENDED or IMMEDIATE ASSIGNMENT REJECT) with page mode set to "paging reorganization" on all paging subchannels.

After 5 seconds (to ensure T3126 expires) the SS pages the MS with its TMSI on an arbitrarily selected paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

The SS changes the paging parameters.

Then the SS sets the page mode to "normal paging".

The SS then waits for the duration of five 51-TDMA multiframes (4 to allow the MS to read all the system information type 1, 2, 3, and 4 messages on the BCCH, and one to calculate the new paging group). Not before 5 seconds after the last IMMEDIATE ASSIGNMENT REJECT message addressing the MS (to ensure T3126 expires), the MS is paged with a PAGING REQUEST TYPE 1 on its new paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection) and then waits 5 seconds (to ensure T3126 expires).

Then the MS is paged with a PAGING REQUEST TYPE 2 on its new paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test

60 sec

Expected Sequence

This sequence is performed for execution counter, K =1, 2.

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN EXT	Sent on the MS's paging
				channel. Page mode set to "paging
				reorganisation". Request Reference
				not pertaining to the MS.
	2	SS -> MS	PAG REQ TYPE 2	Sent before the MS's original
				paging subchannel re-occurs, but 
				later than the next paging
				block of that CCCH.
	3	MS -> SS	CHANNEL REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.

	6	--------	--------	All L3 messages sent on any paging
				subchannel are paging fill
				frames specify "paging re
				organization.

	7	SS -> MS	PAG REQ TYPE 2	Sent on an arbitrarily 
				selected paging subchannel
				Page mode "paging reorganisation"
				Not sent before 5 seconds after step 5.
	8	MS -> SS	CHANNEL REQUEST	
	9	MS -> SS	CHANNEL REQUEST	
	10	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.

	11			Change of paging
				parameters in SYS INFO 3  as
				described below for K=1, 2
	12			The SS waits until it has
				sent all system information
				messages (page mode is still
				paging reorganisation).

	13	--------	--------	All L3 messages sent on any paging
				subchannel specify "normal paging 
	14	--------	--------	Wait 3 seconds
	15	SS -> MS	PAG REQ TYPE 1	Sent on the new paging subchannel						of the MS.
				Not sent before 5 seconds after step 10.
	16	MS -> SS	CHANNEL REQUEST	
	17	MS -> SS	CHANNEL REQUEST	
	18	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.
	19	SS -> MS	PAG REQ TYPE 2	Sent on the new paging subchannel					of the MS.
				Not sent before 5 seconds after step 18.
	20	MS -> SS	CHANNEL REQUEST	
	21	MS -> SS	CHANNEL REQUEST	
	22	SS -> MS	IMM ASSIGN REJ

Specific Message Contents

For execution counter K :

K=1:

SYSTEM INFORMATION TYPE 3 shall have the Control Channel Description IE changed to:

CCCH-CONF	"1 basic physical channel used for CCCH, combined with SDCCHs"

BS-AG-BLKS-RES	2

BS-PA-MFRMS	9

K=2:

SYSTEM INFORMATION TYPE 3 shall have the Control Channel Description IE changed to:

CCCH-CONF	"2 basic physical channel used for CCCH, not combined with SDCCHs"

BS-AG-BLKS-RES	2

BS-PA-MFRMS	9

##### 26.6.2.3.2	Paging / reorganisation / procedure 2

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.3 in phase 1 version.

-	It is stated clearly that the paging parameters are chosen randomly by the SS.]

###### 26.6.2.3.2.1	Conformance requirement

An MS, after reception of a message with page mode set to "paging reorganisation", shall answer to paging messages (with page mode set to "normal paging") sent in a former Access Grant block.

References

GSM 04.08 section 3.3.2.1.

###### 26.6.2.3.2.2	Test purpose

To test that the MS is operating in the "paging reorganisation" page mode when this is ordered by the SS and the MS is paged in its former access grant channel.

###### 26.6.2.3.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 1, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated. The IMSI of the MS is from a defined\default range that ensures its paging channel changes when the broadcast parameters are changed.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS sends an IMMEDIATE ASSIGNMENT EXTENDED on the MS's paging subchannel, with the page mode element set to "paging reorganisation". The MS is then paged immediately in a former Access Grant block with a PAGING REQUEST TYPE 2 message. The MS shall respond by sending CHANNEL REQUESTs.The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test

5 sec

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN EXT	Page mode set to "paging
				reorganisation"
	2	SS -> MS	PAG REQ TYPE 2	Sent in a former access grant
				block.
	3	MS -> SS	CHANNEL REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMM ASSIGN REJ

Specific Message Contents

None.

#### 26.6.2.4	Paging / same as before

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	the paging parameters are chosen randomly by the SS.]

##### 26.6.2.4.1	Conformance requirements

An MS, after first receiving a message with page mode set to "extended paging" and then another message with page mode set to "same as before", shall remember the page mode from the previous message and answer to paging messages in the next but one paging sub block.

References

GSM 04.08 section 3.3.2.1;  GSM 05.02, section 6.5.

##### 26.6.2.4.2	Test purpose

To test that the MS remembers the page mode from the previous paging request message.

##### 26.6.2.4.3	Method of test

Initial Conditions

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS sends an IMMEDIATE ASSIGNMENT REJECT on the MS's paging subchannel, with the page mode element set to "extended paging". In the next but one subblock on the same CCCH, nothing addresses the MS. When the MS's specific paging subchannel reoccurs, a PAGING REQUEST TYPE 3 is sent, not addressing the MS under test and with page mode set to "same as before". In the next but one subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying paging reorganisation and addressing the MS. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test

10 sec

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN REJ	Page mode set to "extended paging"
	2	SS -> MS	XXXX	In the next but one subblock
				nothing addresses the MS
	3	SS -> MS	PAG REQ TYPE 3	This is sent in the next paging
				subblock on the MS's specific
				paging subchannel. The page mode
				is set to "same as before", and the MS
				under test is not addressed.
	4	SS -> MS	PAG REQ TYPE 1	The MS is addressed in this 'next
				but one subblock'. Page mode set
				to "paging reorganisation"
	5	MS -> SS	CHANNEL REQUEST	
	6	MS -> SS	CHANNEL REQUEST	
	7	SS -> MS	IMM ASSIGN REJ

Specific Message Contents

None.

#### 26.6.2.5	Paging / multislot CCCH

##### 26.6.2.5.1	Conformance requirements

The MS shall respond correctly to a PAGING REQUEST TYPE 1 message, when the page mode is set to normal paging, when a multislot CCCH is used and the MS is addressed with its IMSI in the first Mobile Identity field, the optional Mobile Identity field being not present.

References

GSM 04.08 section 3.3.2, GSM 05.02 section 6.5.

##### 26.6.2.5.2	Test purpose

1)	To test that the MS is able to determine its CCCH group and paging group correctly in the case of a CCCH configuration on more than one timeslot when it is paged on a timeslot other than 0. The MS is addressed with a PAGING REQUEST TYPE 1 message when the page mode is set to normal paging. The MS is paged with its IMSI in the 1st Mobile Identity field, the optional Mobile Identity field being not present, is the only way of addressing tested.

2)	To test that in such conditions the MS answers to the paging message on the timeslot on which the paging message was sent.

##### 26.6.2.5.3	Method of test

Initial Conditions

System Simulator:

1 cell, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily under the following constraint :

CCCH-CONF is in the set :

-	2 basic physical channels used for CCCH, not combined with SDCCHs

-	3 basic physical channels used for CCCH, not combined with SDCCHs

-	4 basic physical channels used for CCCH, not combined with SDCCHs

Mobile Station :

The IMSI last 3 digits are so that the CCCH-GROUP of the MS under test is other than 0. According to section 6.5.2 of recommendation 05.02, this means that :

(IMSI mod 1000) mod (BS_CC_CHANS X N) is greater or equal to N, where N = BS_PA_MFRMS X (9 - BS_AG_BLKS_RES).

The MS is in the "idle, updated" state.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle updated".

Test Procedure

The SS pages the MS once with a PAGING REQUEST TYPE 1 message on the timeslot and paging subchannel which correspond to the MS's IMSI.

The MS shall send the CHANNEL REQUEST on the same timeslot as the paging message.

The SS sends an IMMEDIATE ASSIGNMENT on the same timeslot as the paging message.

Maximum Duration of Test

10 s

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains IMSI of
				MS, 2nd Mobile Ident not present.
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging".
				on the same timeslot as the paging
				message
	3	SS -> MS	IMM ASSIGNMENT	on the same timeslot as the paging
				message
	4	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.
	5	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None.

### 26.6.3	Test of measurement report

When an RR-connection exists, the MS shall send measurement reports. These reports contain reception characteristics from serving and neighbouring cells. The measurement report procedure is described in section 3.4.1.2 of 04.08.

NOTE 8:	The capability to calculate RxLev and RxQual is tested in sections 15 and 16. In this test only the signalling aspect is verified.

#### 26.6.3.1	Measurement / no neighbours

This test applies to both GSM900 and DCS1800 mobile stations.

##### 26.6.3.1.1	Conformance requirements

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH block and the measurement valid indication shall be set to valid (0) within the second block at the latest.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.6.3.1.2	Test purpose

To test that, when the SS gives absolutely no information about neighbouring cells, the MS does not report on neighbouring cells.

##### 26.6.3.1.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BCC	ARFCN	ARFCN	Cell Identity

(GSM900)	(DCS1800)

Serving, S1	-60	1	3	002	514	0001H

Neighbour, N1	-85	1	5	008	530	0002H

Neighbour, N2	-80	1	7	014	602	0003H

Neighbour, N3	-75	1	1	020	665	0004H

Neighbour, N4	-55	1	3	026	762	0005H

Neighbour, N5	-50	1	5	032	686	0006H

Neighbour, N6	-45	1	7	038	549	0007H

Neighbour, N7	-40	1	1	044	810	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed twice.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 (on the second iteration of the test the SS also sends SYSTEM INFORMATION TYPE 5bis) on the SACCH. The BA is indicated as empty. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that no measurement results have been obtained.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, k = 1, 2.

Since when k = 1, SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT (and when k = 2 an additional SYSTEM INFORMATION TYPE 5bis is included) are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

GSM900 begin:

SYSTEM INFORMATION TYPE 5:

Information Element				value/remark

Neighbour Cells Description

- Format Identifier			bit map 0

- BCCH Allocation Sequence number	1

- BCCH Allocation ARFCN		No channels belong to the BCCH

allocation

- EXT IND				k = 1. Information Element carries the

complete BA.

k = 2. Information Element carries only a

part of the BA.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element				value/remark

Protocol Descriminator			RR management

Mesage Type				Sys Info 5bis.

Neighbour Cells Description

- Format 				1024 range

- EXT IND				k = 2.   Information Element carries only a part

of the BA.

- W(i)					Only channel 500 belongs to the BCCH

allocation.

MEASUREMENT REPORT:

Information Element				value/remark

Protocol Discriminator			RR Management

Transaction Identifier				0000

Message Type				MEASUREMENT REPORT

Measurement Results

BA-used				1

DTX-used				DTX was not used

RXLEV-FULL-SERVING-CELL		See note 9

RXLEV-SUB-SERVING-CELL		See note 9

MEAS-VALID				See note 10

RXQUAL-FULL-SERVING-CELL		See note 9

RXQUAL-SUB-SERVING-CELL		See note 9

NO-NCELL-M				No neighbour cell measurement result, or

Neighbour cell information not available

for serving cell

RXLEV-NCELL-1			00 0000

BCCH-FREQ-NCELL-1			 0 0000

BSIC-NCELL-1				00 0000

.     .     .				.       .

RXLEV-NCELL-6			00 0000

BCCH-FREQ-NCELL-6			 0 0000

BSIC-NCELL-6				00 0000

GSM900 end:

DCS1800 begin:

SYSTEM INFORMATION TYPE 5:

Information Element				value/remark

Protocol Descriminator			RR management

Mesage Type				Sys Info 5.

Neighbour Cells Description

- Format 				1024 range

- EXT IND				k = 1. Information Element carries the

complete BA.

k = 2. Information Element carries only a part

of the BA.

- W(i)					null.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element				value/remark

Protocol Descriminator			RR management

Mesage Type				Sys Info 5bis.

Neighbour Cells Description

- Format 				1024 range

- EXT IND				k = 2.   Information Element carries only a part

of the BA.

- W(i)					null.

SYSTEM INFORMATION TYPE 6:

Information Element				value/ remark

Protocol Discriminator			RR Management

Message Type				sys info 6

Cell Identity					default

LAI						default

Cell Options

- Power Control Indicator		Power Control Indicator is set

- DTX Indicator			MS shall not use DTX

- Radio-Link-Timeout		default

PLMN permitted				only NCC 1 permitted

MEASUREMENT REPORT:

Information Element				value/remark

Protocol Discriminator			RR Management

Transaction Identifier				0000

Message Type				MEASUREMENT REPORT

Measurement Results

BA-used				1

DTX-used				DTX was not used

RXLEV-FULL-SERVING-CELL		See note 9

RXLEV-SUB-SERVING-CELL		See note 9

MEAS-VALID				See note 10

RXQUAL-FULL-SERVING-CELL		See note 9

RXQUAL-SUB-SERVING-CELL		See note 9

NO-NCELL-M				No neighbour cell measurement result, or

Neighbour cell information not available for

serving cell

RXLEV-NCELL-1			00 0000

BCCH-FREQ-NCELL-1			 0 0000

BSIC-NCELL-1				00 0000

.     .     .				.       .

.     .     .				.       .

RXLEV-NCELL-6			00 0000

BCCH-FREQ-NCELL-6			 0 0000

BSIC-NCELL-6				00 0000

DSC1800 end:

NOTE 9:	The actual values are not checked.

NOTE 10:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable -- once the MS is in U10 then this can be checked immediately.]

#### 26.6.3.2	Measurement / all neighbours present

This test applies to both GSM900 and DCS1800 mobile stations.

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.3.3 in phase 1 version

-	one test has been splitted into 4]

##### 26.6.3.2.1	Conformance requirements

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the values in the MEASUREMENT REPORT message shall contain measurement results for the 6 strongest BCCH carriers with known and allowed NCC part of BSIC.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.6.3.2.2	Test purpose

To test that, when the SS gives information about neighbouring cells, the MS reports appropriate results.

##### 26.6.3.2.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BSCC	ARFCN	ARFCN	Cell identity

(GSM900)	(DCS1800)

Serving, S1	-60	1	3	002	514	0001H

Neighbour, N1	-85	1	5	008	530	0002H

Neighbour, N2	-80	1	7	014	602	0003H

Neighbour, N3	-75	1	1	020	665	0004H

Neighbour, N4	-55	1	3	026	762	0005H

Neighbour, N5	-50	1	5	032	686	0006H

Neighbour, N6	-45	1	7	038	549	0007H

Neighbour, N7	-40	1	1	044	810	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed twice.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 (on the second iteration of the test the SS also sends SYSTEM INFORMATION TYPE 5bis) on the SACCH. All 8 of the BCCHs "on air" are indicated in the BA. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest carriers have been obtained.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, k = 1, 2.

Since when k = 1, SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT (and when k = 2 an additional SYSTEM INFORMATION TYPE 5bis is included) are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

GSM900 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Neighbour Cells Description

Format Identifier		bit map 0

BCCH Allocation Sequence	1

BCCH Allocation ARFCN		The channel numbers 2, 4, 5, 6, 7, 8, 9,

10, 11, 12, 14, 16, 17, 18, 19, 20, 21, 22,

23, 24, 25, 26, 32, 33, 34, 35, 36, 37,

38, 39, 40 and 44 belong to the BCCH

allocation.

- EXT IND			k = 1. Information Element carries complete

BA.

k = 2. Information Element carries only a

part of the BA.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part

of the BA.

- W(i)				Channel 0 and 800 belong to the BCCH

allocation.

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 11

RXLEV-SUB-SERVING-CELL	See note 11

MEAS-VALID			See note 12

RXQUAL-FULL-SERVING-CELL	See note 11

RXQUAL-SUB-SERVING-CELL	See note 11

NO-NCELL-M			6 neighbour cell measurement results

RXLEV-NCELL-1		See note 11

BCCH-FREQ-NCELL-1		Shall not correspond to N1 or N2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 11

BCCH-FREQ-NCELL-2		Shall not correspond to N1 or N2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 11

BCCH-FREQ-NCELL-3		Shall not correspond to N1 or N2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 11

BCCH-FREQ-NCELL-4		Shall not correspond to N1 or N2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		See note 11

BCCH-FREQ-NCELL-5		Shall not correspond to N1 or N2

BSIC-NCELL-5			Corresponds to that of BCCH-FREQ-NCELL-5

RXLEV-NCELL-6		See note 11

BCCH-FREQ-NCELL-6		Shall not correspond to N1 or N2

BSIC-NCELL-6			Corresponds to that of BCCH-FREQ-NCELL-6

GSM900 end:

DCS1800 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5.

Neighbour Cells Description

- Format			1024 range

- EXT IND			k = 1. Information Element carries complete

BA.

k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 1. Non null for ARFCN 514, 530, 549, 602,

665, 686, 762, 810.

k = 2.  Non null for ARFCN 549, 602, 665, 686,

810.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 2. Non null ARFCN 20, 514, 530, 549, 762.

SYSTEM INFORMATION TYPE 6:

Information Element			value/ remark

Protocol Discriminator		RR Management

Message Type			sys info 6

Cell Identity				default

LAI					default

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall not use DTX

- Radio-Link-Timeout	default

PLMN permitted			only NCC 1 permitted

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 11

RXLEV-SUB-SERVING-CELL	See note 11

MEAS-VALID			See note 12

RXQUAL-FULL-SERVING-CELL	See note 11

RXQUAL-SUB-SERVING-CELL	See note 11

NO-NCELL-M			6 neighbour cell measurement results

RXLEV-NCELL-1		See note 11

BCCH-FREQ-NCELL-1		Shall not correspond to N1 or N2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 11

BCCH-FREQ-NCELL-2		Shall not correspond to N1 or N2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 11

BCCH-FREQ-NCELL-3		Shall not correspond to N1 or N2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 11

BCCH-FREQ-NCELL-4		Shall not correspond to N1 or N2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		See note 11

BCCH-FREQ-NCELL-5		Shall not correspond to N1 or N2

BSIC-NCELL-5			Corresponds to that of BCCH-FREQ-NCELL-5

RXLEV-NCELL-6		See note 11

BCCH-FREQ-NCELL-6		Shall not correspond to N1 or N2

BSIC-NCELL-6			Corresponds to that of BCCH-FREQ-NCELL-6

DCS1800 end:

NOTE 11:	These actual values are not checked.

NOTE 12:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

#### 26.6.3.3	Measurement / barred cells and non-permitted NCCs

This test applies to both GSM900 and DCS1800 mobile stations.

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	one test has been splitted into 4]

##### 26.6.3.3.1	Conformance requirements

The MS shall continuously send MEASUREMENT REPORTs on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the messages shall contain measurement results only for the 4 BCCH carriers on which the MS is allowed to report.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.6.3.3.2	Test purpose

To test that, when a combination of normal neighbours, barred cells and non-permitted NCCs is "on air", the MS reports only on normal neighbours.

##### 26.6.3.3.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BCC	ARFCN	ARFCN	Cell identity

(GSM900)	(DCS1800)

Serving, S1	-60	1	3	002	514	0001H

Neighbour, N1	-85	1	5	008	530	0002H

Neighbour, N2	-80	1	7	014	602	0003H

Neighbour, N3	-75	2	1	020	665	0004H

Neighbour, N4	-55	3	3	026	762	0005H

Neighbour, N5	-50	4	5	032	686	0006H

Neighbour, N6	-45	1	7	038	549	0007H

Neighbour, N7	-40	1	1	044	810	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

NOTE 13:	The BA sent in SYSTEM INFORMATION TYPE 5 does not include N1, N4 and N5. N1 may be the case of a barred cell, N3 simulates the case where another operator is transmitting on the same frequency (e.g. in border areas), N4 & N5 simulate the case where other operators are transmitting on other frequencies.

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed twice.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 (on the second iteration of the test the SS also sends SYSTEM INFORMATION TYPE 5bis) on the SACCH. 5 of the 8 BCCHs "on air" are indicated in the BA. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 4 strongest permitted carriers have been obtained (one of the carriers in the BA belongs to a non-permitted NCC).

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, k = 1, 2.

Since when k = 1 SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT (and when k = 2 an additional SYSTEM INFORMATION TYPE 5bis is included) messages are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

GSM900 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Neighbour Cells Description

Format Identifier		bit map 0

BCCH Allocation Sequence	1

BCCH Allocation ARFCN		only channel numbers 2, 14, 20, 38, and

44 belong to the BCCH allocation.

- EXT IND			k = 1. Information Element carries complete BA.

k = 2. Information Element carries only a part

of the BA.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part of

the BA.

- W(i)				Channel 0 and 800 belong to the BCCH

allocation.

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 14

RXLEV-SUB-SERVING-CELL	See note 14

MEAS-VALID			See note 15

RXQUAL-FULL-SERVING-CELL	See note 14

RXQUAL-SUB-SERVING-CELL	See note 14

NO-NCELL-M			4 neighbour cell measurement results

RXLEV-NCELL-1		See note 14

BCCH-FREQ-NCELL-1		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 14

BCCH-FREQ-NCELL-2		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 14

BCCH-FREQ-NCELL-3		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 14

BCCH-FREQ-NCELL-4		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		00 0000

BCCH-FREQ-NCELL-5		 0 0000

BSIC-NCELL-5			00 0000

RXLEV-NCELL-6		00 0000

BCCH-FREQ-NCELL-6		 0 0000

BSIC-NCELL-6			00 0000

GSM900 end:

DCS1800 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5.

Neighbour Cells Description

- Format			1024 range

- EXT IND			k = 1. Information Element carries complete BA.

k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 1. Non null for ARFCN 514, 549, 602,

665, 810.

k = 2.  Non null for ARFCN 549, 602, 810.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 2. Non null ARFCN 514, 665.

SYSTEM INFORMATION TYPE 6:

Information Element			value/ remark

Protocol Discriminator		RR Management

Message Type			sys info 6

Cell Identity				default

LAI					default

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall not use DTX

- Radio-Link-Timeout	default

PLMN permitted			only NCC 1 permitted

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 14

RXLEV-SUB-SERVING-CELL	See note 14

MEAS-VALID			See note 15

RXQUAL-FULL-SERVING-CELL	See note 14

RXQUAL-SUB-SERVING-CELL	See note 14

NO-NCELL-M			4 neighbour cell measurement results

RXLEV-NCELL-1		See note 14

BCCH-FREQ-NCELL-1		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 14

BCCH-FREQ-NCELL-2		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 14

BCCH-FREQ-NCELL-3		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 14

BCCH-FREQ-NCELL-4		Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		00 00000

BCCH-FREQ-NCELL-5		0 0000

BSIC-NCELL-5			00 0000

RXLEV-NCELL-6		00 0000

BCCH-FREQ-NCELL-6		0 0000

BSIC-NCELL-6			00 0000

DCS1800 end:

NOTE 14:	These actual values are not checked.

NOTE 15:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

#### 26.6.3.4	Measurement / DTX

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.3.3 in phase 1 version

-	one test have been splitted into 4

-	Handover is no more specified]

This test applies to both GSM900 and DCS1800 mobile stations.

##### 26.6.3.4.1	Conformance requirements

After the sending of the HANDOVER COMPLETE, the MS shall continuously send measurement reports in every SACCH blocks, the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the order of values in the MEASUREMENT REPORT message shall contain measurement results for the 6 strongest BCCH carriers among those monitored by the MS. Further, in a quiet environment, the DTX-USED field shall be set by the MS to "DTX used".

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.6.3.4.2	Test purpose

To test that, in the case of the MS using DTX and the SS indicating that power control is in use, the MS reports appropriate results.

##### 26.6.3.4.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BCC	ARFCN	ARFCN	Cell identity

(GSM900)	(DCS1800)

Serving, S1	-60	1	3	002	514	0001H

Neighbour, N1	-85	1	5	008	530	0002H

Neighbour, N2	-80	1	7	014	602	0003H

Neighbour, N3	-75	1	1	020	665	0004H

Neighbour, N4	-55	1	3	026	762	0005H

Neighbour, N5	-50	1	5	032	686	0006H

Neighbour, N6	-45	1	7	038	549	0007H

Neighbour, N7	-40	1	1	044	810	0008H

In the serving cell, the DTX indicator is set to "MS shall use discontinous transmission"

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Mobile Station:

The MS is in the active state of a call (U10).

The MS has just completed a handover into the serving cell, S1.

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Support for transparent data services only: yes/no.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed twice.

With the MS having a call in progress on an arbitrary cell, the MS is handed over to cell S1. On cell S1, the SS sends SYSTEM INFORMATION TYPE 5 & 6 (on the second iteration of the test the SS also sends SYSTEM INFORMATION TYPE 5bis) on the SACCH with all 8 of the BCCHs "on air" indicated in the BA. Cell S1 also indicates that DTX shall be used. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest carriers have been obtained and that DTX has been used. (The MS is positioned in an environment free from acoustic noise.)

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed twice for execution counter, k = 1, 2.

Since when k = 1, SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT (and when k = 2 an additional SYSTEM INFORMATION TYPE 5bis is included) messages are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

GSM900 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Neighbour Cells Description

- Format Identifier		bit map 0

- BCCH Allocation Sequence	1

- BCCH Allocation ARFCN	only channel numbers 2, 8, 14, 20, 26,

32, 38, and 44 belong to the BCCH allocation.

- EXT IND			k = 1. Information Element carries the

complete BA.

k = 2. Information Element carries only a part

of the BA.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part

of the BA.

- W(i)				Only channel 500 belongs to the BCCH

allocation.

SYSTEM INFORMATION TYPE 6:

Information Element			value/remark

Cell Options

- Power Control Indicator		Power Control Indicator is set

- DTX Indicator			MS shall use DTX

- Radio-Link-Timeout		8

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

- BA-used			0

- DTX-used			DTX was used (note 18)

- RXLEV-FULL-SERVING-CELL	See note 16

- RXLEV-SUB-SERVING-CELL	See note 16

- MEAS-VALID			See note 17

- RXQUAL-FULL-SERVING-CELL	See note 16

- RXQUAL-SUB-SERVING-CELL	See note 16

- NO-NCELL-M			6 neighbour cell measurement results

- RXLEV-NCELL-1		See note 16

- BCCH-FREQ-NCELL-1		Shall not correspond to N1 or N2

- BSIC-NCELL-1		Corresponds to that of BCCH-FREQ-NCELL-1

- RXLEV-NCELL-2		See note 16

- BCCH-FREQ-NCELL-2		Shall not correspond to N1 or N2

- BSIC-NCELL-2		Corresponds to that of BCCH-FREQ-NCELL-2

- RXLEV-NCELL-3		See note 16

- BCCH-FREQ-NCELL-3		Shall not correspond to N1 or N2

- BSIC-NCELL-3		Corresponds to that of BCCH-FREQ-NCELL-3

- RXLEV-NCELL-4		See note 16

- BCCH-FREQ-NCELL-4		Shall not correspond to N1 or N2

- BSIC-NCELL-4		Corresponds to that of BCCH-FREQ-NCELL-4

- RXLEV-NCELL-5		See note 16

- BCCH-FREQ-NCELL-5		Shall not correspond to N1 or N2

- BSIC-NCELL-5		Corresponds to that of BCCH-FREQ_NCELL-5

- RXLEV-NCELL-6		See note 16

- BCCH-FREQ-NCELL-6		Shall not correspond to N1 or N2

- BSIC-NCELL-6		Corresponds to that of BCCH-FREQ-NCELL-6

GSM900 end:

DCS1800 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5.

Neighbour Cells Description

- Format			1024 range

- EXT IND			k = 1. Information Element carries complete BA.

k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 1. Non null for ARFCN 514, 530, 549, 602

665, 686, 762, 810.

k = 2.  Non null for ARFCN 549, 602, 665, 686,

810.

SYSTEM INFORMATION TYPE 5bis ( Sent only when k = 2 ) :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			1024 range

- EXT IND			k = 2. Information Element carries only a part

of the BA.

- W(i)				k = 2. Non null ARFCN 514, 530, 762.

SYSTEM INFORMATION TYPE 6:

Information Element			value/ remark

Protocol Discriminator		RR Management

Message Type			sys info 6

Cell Identity				default

LAI					default

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall use DTX

- Radio-Link-Timeout	default

PLMN permitted			only NCC 1 permitted

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was used (see note 18)

RXLEV-FULL-SERVING-CELL	See note 16

RXLEV-SUB-SERVING-CELL	See note 16

MEAS-VALID			See note 17

RXQUAL-FULL-SERVING-CELL	See note 16

RXQUAL-SUB-SERVING-CELL	See note 16

NO-NCELL-M			6 neighbour cell measurement results

RXLEV-NCELL-1		See note 16

BCCH-FREQ-NCELL-1		Shall not correspond to N1 or N2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 16

BCCH-FREQ-NCELL-2		Shall not correspond to N1 or N2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 16

BCCH-FREQ-NCELL-3		Shall not correspond to N1 or N2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 16

BCCH-FREQ-NCELL-4		Shall not correspond to N1 or N2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		See note 16

BCCH-FREQ-NCELL-5		Shall not correspond to N1 or N2

BSIC-NCELL-5			Corresponds to that of BCCH-FREQ-NCELL-5

RXLEV-NCELL-6		See note 16

BCCH-FREQ-NCELL-6		Shall not correspond to N1 or N2

BSIC-NCELL-6			Corresponds to that of BCCH-FREQ-NCELL-6

DCS1800 end:

NOTE 16:	These actual values are not checked.

NOTE 17:	The Measurement Valid Indication shall be set to valid within the second SACCH block after the HANDOVER COMPLETE message at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

NOTE 18:	For an MS that only supports transparent data services, the value of DTX-used shall not be checked.

#### 26.6.3.5	Measurement / Frequency Formats

This test applies to both GSM900 and DCS1800 mobiles stations.

##### 26.6.3.5.1	Conformance Requirement

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the values in the MEASUREMENT REPORT message shall contain measurement results for the cells on which the mobile is allowed to report.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.6.3.5.2	Test Purpose

To test that, when the SS gives information about neighbouring cells, the MS reports appropriate results.

##### 26.6.3.5.3	Method of test

Initial Conditions

System Simulator:

2 cells with the following settings:

Transmitter	Level	NCC	BCC	ARFCN	ARFCN	Cell Identity

(GSM900)	(DCS1800)

Serving, S1	-60	1	3	002	715	0001H

Neighbour, N1	-85	1	5	008	815	0002H

With the exception of the Cell Allocation, the rest of the parameters for both cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed three times.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5, 5bis & 6 on the SACCH. Both of the BCCHs "on air" are indicated in the BA. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that no measurement results have been obtained.

For each iteration of the test the frequency format of the BA list contained in the System Information 5 and 5bis message shall change according to the specific message contents.

Maximum Duration Of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, K = 1, 2, 3.

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION 5bis, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

GSM900 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Neighbour Cells Description

Format Identifier			Bit Map 0.

BCCH Allocation Sequence		1

BCCH Allocation ARFCN		Channel numbers 2, 6 and 8  belong to the

BCCH allocation.

- EXT IND			Information Element carries only a part

of the BA.

SYSTEM INFORMATION TYPE 5bis :

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

Format Identifier			K = 1. Range 1024 Format

K = 2. Range 512 Format

K = 3. Variable Bit Map.

- EXT IND			Information Element carries only a part

of the BA.

- W(i)				K = 1,2. 500, 530, 595, 965, 1000, 715, 815, 0

K = 3.    965, 1000, 0, 2, 6

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 19

RXLEV-SUB-SERVING-CELL	See note 19

MEAS-VALID			See note 20

RXQUAL-FULL-SERVING-CELL	See note 19

RXQUAL-SUB-SERVING-CELL	See note 19

NO-NCELL-M			2 neighbour cell measurement result

RXLEV-NCELL-1		See note 19

BCCH-FREQ-NCELL-1		Shall correspond to S1 or N1, ie, it shall be 0

or 2.

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 19

BCCH-FREQ-NCELL-2		Shall correspond to S1 or N1, i.e. it shall be 0

or 2.

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		00 0000

BCCH-FREQ-NCELL-3		 0 0000

BSIC-NCELL-3			00 0000

.     .     .			.       .

.     .     .			.       .

RXLEV-NCELL-6		00 0000

BCCH-FREQ-NCELL-6		 0 0000

BSIC-NCELL-6			00 0000

GSM900 end:

DCS1800 begin:

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5.

Neighbour Cells Description

- Format 			K = 1. Range 1024 Format

K = 2. Variable Bit Map

K = 3. Range 128 Format

- EXT IND			Information Element carries only a

part of the BA.

- W(i)				K = 1. Non null for 500, 530, 595, 715,

815, 965, 1000, 0

K = 2. Non null for 965, 1000, 0, 2, 6, 8

K = 3. Non null for 695, 715, 800

SYSTEM INFORMATION TYPE 5bis

Information Element			value/remark

Protocol Descriminator		RR management

Mesage Type			Sys Info 5bis.

Neighbour Cells Description

- Format 			K = 1. Bit Map 0.

K = 2. Range 512 Format

K = 3. Range 256 Format

- EXT IND			Information Element carries only a part

of the BA.

- W(i) / BCCH Allocation		K = 1. Non null for 2, 6, 8

K = 2. Non null for 500, 530, 595, 715, 815, 965

K = 3. Non Null for 815, 965, 1000, 0, 2, 6

SYSTEM INFORMATION TYPE 6:

Information Element			value/ remark

Protocol Discriminator		RR Management

Message Type			sys info 6

Cell Identity				default

LAI					default

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall not use DTX

- Radio-Link-Timeout	default

PLMN permitted			only NCC 1 permitted

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 19

RXLEV-SUB-SERVING-CELL	See note 19

MEAS-VALID			See note 20

RXQUAL-FULL-SERVING-CELL	See note 19

RXQUAL-SUB-SERVING-CELL	See note 19

NO-NCELL-M			2 neighbour cell measurement results

RXLEV-NCELL-1		See note 19

BCCH-FREQ-NCELL-1		K= 1. Shall correspond to S1 or N1, ie,  it shall

be 6 or 7

K= 2. Shall correspond to S1 or N1, ie,  it shall

be 6 or 7

K= 3. Shall correspond to S1 or N1, ie,  it shall

be 3 or 5

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 19

BCCH-FREQ-NCELL-2		K= 1. Shall correspond to S1 or N1, ie,  it shall

be 6 or 7

K= 2. Shall correspond to S1 or N1, ie,  it shall

be 6 or 7

K= 3. Shall correspond to S1 or N1, ie,  it shall

be 3 or 5

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		00 0000

BCCH-FREQ-NCELL-3		 0 0000

BSIC-NCELL-3			00 0000

.     .     .			.       .

.     .     .			.       .

RXLEV-NCELL-6		00 0000

BCCH-FREQ-NCELL-6		 0 0000

BSIC-NCELL-6			00 0000

NOTE 19:	These actual values are not checked.

NOTE 20:	The Measurement Valid Indication shall be set to valid within the second SACCH block at  the latest.

### 26.6.4	Test of the channel assignment procedure

An intracell change of channel can be requested by upper layers in order to change the channel type, or it may be initiated by the RR-sublayer, e.g. for an intra cell handover. This change is performed using the channel assignment procedure. If the procedure is incorrectly implemented in the MS, the establishment and maintenance of connections is endangered. This applies for the successful case and for the assignment failure: the MS's correct return to the old channel after assignment failure is a necessary part of the GSM system design.

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The phase 2 CR from France Telecom is not yet included.??

- The succesful procedure is not tested for SDCCH-only MSes.

- Mobile allocation needs further description; was the cell allocation present in SYSTEM INFO? (for 6.4.1)

]

#### 26.6.4.1	Dedicated assignment / successful case

[Dissimilarities to 11.10 phase 1 - editorial notes :

- This section was part of II.5.3.6.4.2 in phase 1 version; for non-halfrate MSes the test was the same with the exceptions noted below; for F+H-MSes, the test is different, because the TCH/H cases are tested at the end, not in the middle, and both hopping and non-hopping are checked.

-	N(SD) is used instead of V(SD)

- In phase 1 it was required that the MS uses the commanded power level on the assigned channels; in phase 2 it is checked that the MS reports that level.

- A postamble has been added.

]

This test is only applicable to an MS supporting a TCH.

##### 26.6.4.1.1	Conformance requirements

1.	Upon receipt of the ASSIGNMENT COMMAND message, the mobile station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the main signalling links).

2.	MM-messages and CM-messages using SAPI=0 sent from the mobile station to the network can be duplicated by the data link layer in the following case:

a channel change of dedicated channels is required (assignment or handover procedure) and the last layer 2 frame has not been acknowledged by the peer data link layer before the mobile station leaves the old channel.

In this case, the mobile station does not know whether the network has received the message correctly. Therefore, the mobile station has to send the message again after the new dedicated channel is established.

3.	An ASSIGNMENT COMMAND message may indicate a frequency change in progress, with a starting time and possibly alternative channel descriptions.

In the case of the reception of an ASSIGNMENT COMMAND message which contains only the description of a channel to be used after the starting time, and if the starting time has not already elapsed,the mobile station shall wait up to the starting time before accessing the channel.

4.	The MS shall establish the link with the power level specified in the ASSIGNMENT COMMAND message.

The MS shall confirm the power control level that it is currently employing in the uplink SACCH L1 header. The indicated value shall be the power control level actually used by the mobile for the last burst of the previous SACCH period.

5.	The MS shall apply the hopping frequencies specified in ASSIGNMENT COMMAND message in the Mobile Allocation IE or the Frequency List IE at the time of accessing the new channel using the last received Cell Allocation.

6.	After receipt of the ASSIGNMENT COMMAND the MS shall perform the assignment and return an ASSIGNMENT COMPLETE without undue delay.

References

1, 3, 5.	GSM 04.08, subclause 3.4.3.

2.	GSM 04.08, subclause 3.1.4.3.

4.	GSM 04.08, subclause 3.4.3; GSM 05.08, subclause 4.2.

6.	GSM 04.13, subclause 5.2.4.

##### 26.6.4.1.2	Test purpose

1.	To verify that upon receipt of an ASSIGNMENT COMMAND, the MS switches to the channel defined in the ASSIGNMENT COMMAND, establishes the link and sends an ASSIGNMENT COMPLETE message. This is tested for an MS supporting TCH in the special cases of a transition

1.1	from non-hopping SDCCH to hopping TCH/F using a different timeslot

1.2	from hopping TCH/F to non-hopping TCH/F using a different timeslot

1.3	from non-hopping TCH/F to non-hopping TCH/F using a different timeslot

1.4	from non-hopping TCH/F to hopping TCH/H using a different timeslot; this test purpose is only applicable if the MS supports TCH/H

1.5	from hopping TCH/H to non-hopping TCH/H using a different timeslot; this test purpose is only applicable if the MS supports TCH/H

1.6	from non-hopping TCH/H to hopping TCH/F using a different timeslot; this test purpose is only applicable if the MS supports TCH/H.

2.	To verify that an MS supporting TCH, having sent an MM- or CM message which was not acknowledged on L2 before the channel assignment procedure was initiated and before the MS has left the old channel, repeats that message after completion of the assignment procedure without incrementing N(SD). This is tested in the special case of MM message AUTHENICATION RESPONSE.

3.	To verify that, if an MS supporting TCH has received an ASSIGNMENT COMMAND message which contains only the description of a channel to be used after the starting time, and if the starting time has not already elapsed, the mobile station shall wait up to the starting time before accessing the channel.

4.	To verify that an MS supporting TCH, having received an ASSIGNMENT COMMAND, having sent an SABM frame to establish the main signalling link on the assigned channel, reports the power level specified in the ASSIGNMENT COMMAND message, in the uplink SACCH L1 header of the SACCH message sent in the SACCH period following the transmission of the SABM frame.

5.	To verify that an MS supporting TCH, having received an ASSIGNMENT COMMAND, is able in the case of frequency hopping to decode the Mobile Allocation and Frequency List IEs correctly and applies the specified frequencies using the correct Cell Allocation.

6.	To verify that after receipt of the ASSIGNMENT COMMAND the MS returns an ASSIGNMENT COMPLETE without undue delay.

##### 26.6.4.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters except:

GSM900:

BCCH ARFCN =20

Throughout the test, the CA broadcast in System Information 1 is (10, 17, 20, 26, 34, 42, 45, 46, 52, 59)

Note that the actual CA of the cell contains other frequencies.

DCS1800:

BCCH ARFCN =747

Throughout the test, the CA broadcast in System Information 1 is (734, 741, 747, 754, 759, 766, 773, 775, 779, 782)

Note that the actual CA of the cell contains other frequencies.

Mobile Station:

The MS is in the "idle, updated" state with a TMSI allocated.

Related PICS/PIXIT Statements

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	The supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The SS pages the MS and allocates an SDCCH. Then 2 different channels are assigned with ASSIGNMENT COMMANDs. Each time the MS shall switch to the assigned channel, establish the link and send an ASSIGNMENT COMPLETE message.

Then the SS sends a AUTHENTICATION REQUEST message. The MS shall answer with an AUTHENTICATION RESPONSE message, which is not acknowledged on L2 by the SS. Immediately after the AUTHENTICATION RESPONSE message is received, the SS sends an ASSIGNMENT COMMAND. The MS shall switch to the assigned channel, establish the link with the commanded power level and send as ASSIGNMENT COMPLETE message. Then MS shall repeat the AUTHENTICATION RESPONSE message, with the same N(SD) value.

Then the SS sends an ASSIGNMENT COMMAND, which includes a Starting Time IE. The MS shall react as specified above, but this shall be done at the time specified in Starting Time IE.

For an MS not supporting TCH/H, the SS initiates the channel release procedure and the test ends here. For an MS supporting TCH/H, the channel assignment procedure is performed another three times, with half rate channels involved, and again it is checked that the MS correctly completes the procedures, before the SS initiates the channel release procedure.

Maximum Duration of Test

30 s

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGN	See specific message contents
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	ASSIGN COMMAND	See specific message contents
	6	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after 
				establishment of the main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 5.
	7	SS		The SS checks that the MS
				reports the requested power
				level in the layer 1 header
				of the SACCH message
				that is sent in the first SACCH
				muliframe following the SABM.
	8	SS -> MS	ASSIGN COMMAND	See specific message contents.
	9	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after 
				establishment of the main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 8.
	10	SS -> MS	AUTHENT REQUEST	
	11	MS -> SS	AUTHENT RESPONSE	This message is not L2
				acknowledged by the SS
	12	SS -> MS	ASSIGN COMMAND	See specific message contents
	13	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after 
				establishment of the main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 12.
	14	MS -> SS	AUTHENT RESPONSE	N(SD) shall be the same as in
				step 10
	15	SS -> MS	ASSIGN COMMAND	See specific message contents
	16	MS -> SS	SABM	Sent on the assigned channel.
				The frames containing the SABM shall
				have frame numbers x with 
				0 <=  (x - starting time) mod 42432 <= 17
	17	SS -> MS	UA
	18	MS -> SS	ASSIGN COMPLETE

A			This test part is performed
				if the MS doesn't support
				TCH/H (see PICS/PIXIT)
	A19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

B			This test part is performed
				if the MS supports
				TCH/H (see PICS/PIXIT)
	B19	SS -> MS	ASSIGN COMMAND	See specific message contents
	B20	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel
				after establishment of the
				main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 19.
	B21 	SS -> MS	ASSIGN COMMAND	See specific message contents
	B22	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel
				after establishment of the
				main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 22.
	B23	SS -> MS	ASSIGN COMMAND	See specific message contents
	B24	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel
				after establishment of the
				main signalling link.
				This message shall be ready to be
				transmitted before 600 ms after
				the completion of step 23.
	B25	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents

GSM900 begin:

Step 3

IMMEDIATE ASSIGNMENT

As default message contents except

Channel Description

- Channel Type	SDCCH/8

TDMA offset	Chosen arbitrarily

- Timeslot Number	N, chosen arbitrarily

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	the ARFCN of the BCCH carrier

Step 5

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+1) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1 to 63)

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Frequency list IE	Not included

Channel Mode

- Mode	Signalling

Mobile Allocation	Indicates all of the CA (broadcast on the BCCH) except for the BCCH carrier.

Starting Time	Not included

Step 8

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+3) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	the ARFCN of the BCCH carrier

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	A non-signalling mode
		arbitrarily selected from the full
		rate capabilities declared for the MS

Frequency list IE	Not Included

Cell Channel Description	Bit map zero encodes (45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114)

Mobile Allocation	Not included

Starting Time	Not included

Step 12

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+4) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1 to 63)

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	Not included (thus the CA from 
		step 8 is used to decode the MA)

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Indicates frequencies (45, 46, 73, 74, 75, 76, 108, 114)

Starting Time	Not included

Step 15

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+5) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	10

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	signalling

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Not included

Starting Time	indicates (current frame
		number + 100 frames) mod 42432

Step 19

ASSIGNMENT COMMAND:

Channel Description

- Channel Type	TCH/H
	        TDMA offset	Chosen arbitrarily

- Timeslot Number	(N+6) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Frequency List IE.

- HSN	0

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	A non-signalling mode
		arbitrarily selected from the half
		rate capabilities declared for the MS

Cell Channel Description	Not included

Frequency list IE	Uses bit map 0 to indicate (10, 34, 52, 73, 108, 114)

Mobile Allocation	Not included

Starting Time	Not included

Step 21

ASSIGNMENT COMMAND:

Channel Description

- Channel Type	TCH/H

TDMA offset	Chosen arbitrarily

- Timeslot Number	(N+7) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	Chosen arbitrarily, but not the BCCH carrier

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	Not included

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Not included

Starting Time	Not incldued

Step 23

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+1) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1

to 63

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Cell Channel Description	Uses bit map 0 to indicate (17, 20)

Frequency list IE	Not included

Mobile Allocation	Indicates ARFCN 17 only

Starting Time	Not included

GSM900 end:

DCS1800 begin:

Step 3

IMMEDIATE ASSIGNMENT

As default message contents except

Channel Description

- Channel Type	SDCCH/8

TDMA offset	Chosen arbitrarily

- Timeslot Number	N, chosen arbitrarily

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	the ARFCN of the BCCH carrier

Step 5

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+1) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1 to 63)

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Frequency list IE	Not included

Channel Mode

- Mode	Signalling

Mobile Allocation	Indicates all of the CA (broadcast on the BCCH) except for the BCCH carrier.

Starting Time	Not included

Step 8

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+3) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	the ARFCN of the BCCH carrier

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	A non-signalling mode
		arbitrarily selected from the full
		rate capabilities declared for the MS

Frequency list IE	Not Included

Cell Channel Description	Use Range 128 to encode (773, 775, 779, 782, 791, 798, 829, 832, 844)

Mobile Allocation	Not included

Starting Time	Not included

Step 12

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+4) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1 to 63)

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	Not included (thus the CA from 
		step 8 is used to decode the MA)

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Indicates frequencies (773, 775, 779, 829, 832, 844)

Starting Time	Not included

Step 15

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+5) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	734

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	signalling

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Not included

Starting Time	indicates (current frame
		number + 100 frames) mod 42432

Step 19

ASSIGNMENT COMMAND:

Channel Description

- Channel Type	TCH/H
	        TDMA offset	Chosen arbitrarily

- Timeslot Number	(N+6) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Frequency List IE.

- HSN	0

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	A non-signalling mode
		arbitrarily selected from the half
		rate capabilities declared for the MS

Cell Channel Description	Not included

Frequency list IE	Uses Range 1024 to indicate (734, 741, 759, 766, 773, 832, 844)

Mobile Allocation	Not included

Starting Time	Not included

Step 21

ASSIGNMENT COMMAND:

Channel Description

- Channel Type	TCH/H

TDMA offset	Chosen arbitrarily

- Timeslot Number	(N+7) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	Single RF Channel

- ARFCN	Chosen arbitrarily, but not the BCCH carrier

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Channel Mode	Not included

Frequency list IE	Not included

Cell Channel Description	Not included

Mobile Allocation	Not included

Starting Time	Not incldued

Step 23

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	TCH/F

- Timeslot Number	(N+1) mod 8

- Training Sequence Code	Chosen arbitrarily

- Hopping	RF hopping channel

- MAIO	Chosen arbitrarily from the set (0, 1

to N-1) where N is the number of

frequencies in the Mobile Allocation IE.

- HSN	Chosen arbitrarily from the set (1

to 63

Power Command

- Power level	Chosen arbitrarily but with a changed value.

Cell Channel Description	Uses Variable Range Format to indicate (741, 747)

Frequency list IE	Not included

Mobile Allocation	Indicates ARFCN 741 only

Starting Time	Not included

DCS1800 end:

#### 26.6.4.2	Dedicated assignment / failure

##### 26.6.4.2.1	Dedicated assignment / failure / failure during active state

This test is only applicable to an MS supporting the call control protocol.

[ dissimilarities to 11.10 phase 1 - editorial notes :

- For an MS supporting TCH/H, the test is repeated for TCH/H.

- Instead of checking the power level, it is checked which power level the MS specifies in the L1 header.

]

###### 26.6.4.2.1.1	Conformance requirements

On the mobile station side, if a lower layer failure happens on the new channel before the ASSIGNMENT COMPLETE message has been sent, the mobile station deactivates the new channels, reactivates the old channels, reconnects the TCHs if any and triggers the establishment of the main signalling link. It then sends a ASSIGNMENT FAILURE message, cause "protocol error unspecified" on the main DCCH and resumes the normal operation, as if no assignment attempt had occurred. The operational parameters (e.g. ciphering mode) when returning on the old channel are those applied before the procedure.

References

GSM 04.08 section 3.4.3.3.

GSM 05.08 section 4.2.

GSM 05.05 section 4.1.1.

###### 26.6.4.2.1.2	Test purpose

To test that, when the MS fails to seize the new channel, the MS reactivates the old channel, reporting use of the last power level used on the old channel.

This is tested in the special cases of a transition:

- from TCH/F to hopping TCH/F in state U10 if the MS supports TCH/F and call control

- from TCH/H to hopping TCH/H in state U10 if the MS supports TCH/H and call control

###### 26.6.4.2.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters. The SS orders the MS to use a power level P. Where P is a power level within the range supported by the Type of MS.

Mobile Station:

The MS is in the active state (U10) of a mobile terminated call.

Related PICS/PIXIT Statements

- Support for TCH/F.

- Support for TCH/H.

- Support for state U10 of the Call Control protocol.

- Type of MS (P-GSM900 or EGSM or DCS1800).

- Power Class of MS.

Foreseen Final State of the MS

The active state (U10) of a mobile terminated call.

Test Procedure

The MS is in the active state (U10) of a mobile terminated call. The SS sends an ASSIGNMENT COMMAND allocating a new TCH/F, but does not activate the new channel. It is checked that the MS  triggers the establishment of the main signalling link on the old channel and then sends an ASSIGNMENT FAILURE.

Maximum Duration of Test

30 s

Expected Sequence

The test is repeated for execution counter k = 1, 2 if the MS supports TCH/H.

Step	Direction	Message	Comments

1	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, if k = 1,
				Channel Type = TCH/H, if k = 2.
				Power level specified in power command
				is different to P, again where P is a power
				level within the range supported by the Type
				of MS.
				The MS attempts (and fails) to establish a
				signalling link on the new channel.
	2			The MS re-establishes the signalling link on
				the old channel.
	3	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"
	4	SS		The SS checks that the MS
				reports power level P
				in the L1 header of the SACCH message
				that is sent in the first SACCH
				multiframe following the SABM.

Specific Message Contents

None

##### 26.6.4.2.2	Dedicated assignment / failure / general case

[ dissimilarities to 11.10 phase 1 - editorial notes :

New test.

]

###### 26.6.4.2.2.1	Conformance requirements

On the mobile station side, if a lower layer failure happens on the new channel before the ASSIGNMENT COMPLETE message has been sent, the mobile station deactivates the new channels, reactivates the old channels, reconnects the TCHs if any and triggers the establishment of the main signalling link. It then sends a ASSIGNMENT FAILURE message, cause "protocol error unspecified" on the main DCCH and resumes the normal operation, as if no assignment attempt had occurred. The operational parameters (e.g. ciphering mode) when returning on the old channel are those applied before the procedure.

References

GSM 04.08 section 3.4.3.3.

###### 26.6.4.2.2.2	Test purpose

To test that, when the MS fails to seize the new channel, the MS reactivates the old channel.

This is tested in the special cases of a transition:

-	from SDCCH to hopping TCH/F; this test part is only applicable if the MS supports TCH/F.

-	from non-hopping SDCCH to hopping TCH/H; this test part is only applicable if the MS supports TCH/H.

-	from hopping TCH/F to hopping TCH/H; this test part is only applicable if the MS supports TCH/H.

NOTE 21:	26.6.8.4 contains the case of an assignment failure SDCCH -> SDCCH.

###### 26.6.4.2.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT Statements

-	Support for TCH/F.

-	Support for TCH/H.

-	Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The MS is "idle updated".

Test Procedure

A mobile terminated RR connection is established on an SDCCH. The SS sends an ASSIGNMENT COMMAND message allocating a hopping TCH/F, but does not activate the assigned channels. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel and trigger the establishment of the main signalling link on the old channel. Then the MS shall send an ASSIGNMENT FAILURE.

For an MS not supporting TCH/H, the SS initiates the channel release procedure and the test ends here. For an MS supporting TCH/H, the test sequence is repeated another two times, with half rate channels involved, and again it is checked that the MS correctly returns to the old channels, before the SS initiates the channel release procedure.

Maximum Duration of Test

30 s

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	Channel Type : SDCCH
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, hopping.
				The MS attempts (and fails) to establish a
				signalling link on the new channel.
	6			The MS reestablishes the signalling link on
				the old channel.
	7	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"

A			This test part is performed 
				if the MS does not support TCH/H
	A8	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

B			This test part is performed 
				if the MS supports TCH/H.
	B8	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/H, hopping.
				The MS attempts (and fails) to establish a
				signalling link on the new channel.
	B9			The MS reestablishes the signalling link on
				the old channel.
	B10	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"
	B11	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, hopping.
	B12	MS -> SS	ASSIGN COMPLETE	Sent on the assigned channel after 
				establishment of the main signalling link.
	B13	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/H, hopping.
				The MS attempts (and fails) to establish a
				signalling link on the new channel.
	B14			The MS reestablishes the signalling link on
				the old channel.
	B15	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"
	B16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents

None.

### 26.6.5	Test of handover

With the Handover procedure, it is possible to completely alter the channels allocated to a MS. This makes it possible in particular to switch a call in progress from one cell to another. The procedure is always initiated by the network and with the MS in a dedicated mode.

Sections 26.6.5.1 - 26.6.5.4 contain generic test procedures to be used for executing successful Handover tests. Table 1 contains a summary of the different combinations of parameters which have to be tested, together with a reference to the appropriate generic test procedure. If a test uses a channel rate which the MS under test does not support, the test shall be skipped.

Table 1

From	To	Timing	Start	Syn	State	Section	Exec
		Adv.	Time	?	of call	26.	Counter

TCH/F, no FH	TCH/F, no FH	20	none	no	U10	6.5.1	1
TCH/F, no FH	TCH/F, FH	arbitrary	none	no	U10	6.5.1	2
TCH/F, FH	TCH/F, no FH	20	1.1s	no	U10	6.5.1	3
TCH/H, FH	TCH/H, no FH	20	none	no	U10	6.5.1	6
TCH/H, no FH	TCH/H, FH	arbitrary	none	no	U10	6.5.1	7
TCH/H, FH	TCH/H, FH	20	1.1s	no	U10	6.5.1	5
TCH/F, no FH	TCH/H, FH	arbitrary	none	no	U10	6.5.1	4
TCH/H, FH	TCH/F, no FH	arbitrary	none	no	U10	6.5.1	8
SDCCH/4, no FH	TCH/F, FH	20	none	no	estab	6.5.2	1
SDCCH/4, no FH	TCH/H, FH	20	none	no	estab	6.5.2	2
SDCCH/4, no FH	SDCCH/8, FH	20	none	no	estab	6.5.2	3
SDCCH/8, no FH	SDCCH/8, FH	arbitrary	none	no	estab	6.5.2	4
TCH/F, no FH	TCH/H, no FH	20	none	no	estab	6.5.2	5
TCH/H, FH	TCH/F, FH	20	none	no	estab	6.5.2	6
TCH/F, FH	TCH/F, FH	arbitrary	none	no	estab	6.5.2	7
SDCCH/8, FH	TCH/F, no FH	20	none	no	estab	6.5.2	8
SDCCH/8, no FH	TCH/F, FH	20	none	no	estab	6.5.2	9
SDCCH/8, no FH	TCH/H, FH	arbitrary	none	no	estab	6.5.2	10
TCH/F, FH	TCH/F, no FH	2k+y	none	yes	U10	6.5.3	1
TCH/H, FH	TCH/H, no FH	2k+y	none	yes	U10	6.5.3	2
SDCCH/8, FH	SDCCH/8, FH	2k+y	none	yes	estab	6.5.4	1
SDCCH/8, FH	SDCCH/4, no FH	2k+y	1.1s	yes	estab	6.5.4	2
TCH/F, no FH	TCH/F, FH	2k+y	none	yes	estab	6.5.4	3
SDCCH/8, no FH	TCH/F, no FH	2k+y	none	yes	estab	6.5.4	4

Table 2

TCH/FS	TCH/HS	SDCCH

n	10-20	5-10	2-5

In addition to the successfull case of Handover, 2 unsuccessful cases shall be tested. These tests are described in 26.6.5.8 and 26.6.5.9.

#### 26.6.5.1	Handover / successful / active call / non-synchronized

##### 26.6.5.1.1	Conformance requirements

The MS shall correctly apply the handover procedure in the non-synchronized case when a call is in progress and when handover is performed from a TCH/F without frequency hopping towards a TCH/F without frequency hopping.

The MS shall correctly apply the handover procedure in the non-synchronized case when a call is in progress and when handover is performed from a TCH/H without frequency hopping to a TCH/H with frequency hopping. This does not apply to MSs not supporting TCH/H.

References

GSM 04.08 sections 3.4.4 and 9.1.15

GSM 04.13 section 5.2.6.2.

##### 26.6.5.1.2	Test purpose

To test that when the MS is ordered to make a non-synchronized handover it continuously sends access bursts on the main DCCH until it receives a PHYSICAL INFORMATION message from the SS. To test that the MS correctly handles the values of any Starting Time IE in the HANDOVER COMMAND message in the case when none of the information elements referring to before the starting time are present. To test that the MS correctly handles the Timing Advance IE in the PHYSICAL INFORMATION message. To test that the MS activates the new channel correctly and transmits the HANDOVER COMPLETE message without undue delay.

##### 26.6.5.1.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B with same LAI, default parameters except:

GSM900 :

Cell A has :

BCCH ARFCN = 20

Cell Allocation = (10, 17, 20, 26, 34, 42, 45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114)

Cell B has :

BCCH ARFCN = 40

Cell Allocation = (14, 18, 22, 24, 30, 31, 38, 40, 60, 66, 73, 74, 75, 76, 108, 114)

The frame numbers of cells A and B shall be different by 100.

The timebase of cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

DCS1800 :

Cell A has :

BCCH ARFCN = 747

Cell Allocation = ( 734, 741, 747, 754, 759, 762, 766, 767, 773, 775, 779, 782, 791, 798, 829, 832, 844 )

Cell B has :

BCCH ARFCN = 764

Cell Allocation = ( 739, 743, 746, 749, 756, 758, 761, 764, 771, 779, 782, 791, 798, 829, 832, 844 )

The Cell Allocation of both Cell A and Cell B shall be coded using range 256 format.

The frame numbers of cells A and B shall be different by 100.

The timebase of cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

Mobile Station:

The MS is in the active state (U10) of a call on cell A.

[editorial note : since not all MSs support such high power levels, it might be better to choose e

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Support for speech : yes/no

supported radio interface rates : 12kbps, 6kbps, 3.6kbps

Type of Mobile Station ( P-GSM900 or EGSM or DCS1800 ).

Foreseen Final State of the MS

The active state (U10) of a call on cell A.

Test Procedure

This procedure is repeated for execution counter M = 1 to 8. (See table 1).

The MS is in the active state (U10) of a call. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall (at the time defined by the Starting Time information element, if included in the message) begin to send access bursts on the new DCCH of the target cell. The SS observes the access bursts and after receiving n (n being randomly drawn between values according to table 2 of section 26.6.5) access bursts, the SS sends one PHYSICAL INFORMATION message with a Timing Advance as specified in table 1 of section 26.6.5. The MS shall activate the channel in sending and receiving mode. The MS shall establish a signalling link. The MS shall be ready to transmit a HANDOVER COMPLETE message, before 'x' MS after the end of the PHYSICAL INFORMATION message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in TS GSM 04.13. The value of 'x' depends upon the target channel and is specified in the specific message contents section.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for an execution counter M = 1, 2, 3 for an MS which only supports TCH/F.

This sequence is performed for an execution counter M = 1, 2.. 8 for an MS which supports TCH/F and H

Step	Direction	Message	Comments

0	MS -> SS		The MS and SS are in the active state of a call
				on the channel described below.

	1	SS -> MS	HANDOVER COMMAND	See Specific message contents
	2	MS -> SS	HANDOVER ACCESS	Repeated on every burst of the uplink main
				DCCH until reception of PHYSICAL
				INFORMATION. Handover Reference as
				included in the HANDOVER COMMAND.  If 
				the HANDOVER COMMAND includes a 
				starting time IE then the first HANDOVER 
				ACCESS message shall be transmitted in 
				the indicated frame (unless the indicated 
				frame is not used by that channel, in which 
				case the next frame used by that channel 
				shall be used)
	3	SS -> MS	PHYSICAL INFO	Sent after reception of n HANDOVER
				ACCESS messages. See specific message
				contents.
	4	MS -> SS	SABM	Sent without information field
	5	SS -> MS	UA
	6	MS -> SS	HANDOVER COMPLETE	The message shall be ready to be 						transmitted before 'x' ms after the 						completion of step 3.
	7	MS -> SS		The MS and SS are in the active state of a 
				call on the channel described below.

Specific Message Contents For Mobiles Supporting Speech

For M = 1 :

P-GSM900

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non hopping mode on cell B.

For M = 2 :

GSM900

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			20

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Chosen arbitrarily, but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Channel Sequence IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Synchronisation Indication IE is not included.

Channel Mode IE is not included.

Frequency Channel Sequence after time

- Frequency Channel Sequence		Allocates the following 12

frequencies ( 10, 17, 20, 26,

59, 66, 73, 74, 75, 76,108,

114 )

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			747

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Chosen arbitrarily, but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Short List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Synchronisation Indication IE is not included.

Channel Mode IE is not included.

Frequency Short List after time

- Frequency List				Use Range 256 to encode

the following 9 frequencies :

( 747, 775, 779, 782, 791, 798, 829, 832, 844 )

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in hopping mode on cell A.

For M = 3 :

GSM900

Step 0 : The MS and SS are using a full rate TCH in hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Out of range timing advance shall

trigger a handover failure procedure.

Starting Time				Indicates the frame number of cell B

that will occur approximately 1.1

seconds ( 238 frames have

elapsed ) after  the HANDOVER

COMMAND is sent by cell A.

Mode of first channel				Speech full rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a full rate TCH in hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Out of range timing advance shall

trigger a handover failure procedure.

Starting Time				Indicates the frame number of cell B

that will occur approximately 1.1

seconds ( 238 frames have

elapsed ) after  the HANDOVER

COMMAND is sent by cell A.

Mode of first channel				Speech full rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

For M = 4 :

GSM900

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			20

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Mobile Allocation.

- HSN					zero ( this gives cyclic hopping )

Synchronisation Indication  IE is not included.

Cell Channel Description			Use bit map 0 to encode the

complete CA of Cell A

Mobile Allocation after time			Indicates all of the CA of cell A

except for the BCCH frequency.

Mode of First Channel			Speech half rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a full rate TCH in non-hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			747

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Mobile Allocation.

- HSN					zero ( this gives cyclic hopping )

Synchronisation Indication  IE is not included.

Cell Channel Description			Use Range 512 to encode the

complete CA of Cell A

Mobile Allocation after time			Indicates all of the CA of cell A

except for the BCCH frequency.

Mode of First Channel			Speech half rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell A.

For M = 5 :

GSM900

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily, but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency List after time

- Frequency List				use bit map 0 to allocates

the following 12 frequencies :

( 14, 18, 22, 24, 60, 66, 73, 74, 75, 76,108, 114 )

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Starting Time				Indicates the frame number of cell B.

that will occur approximately 1.1

seconds ( 238 frames have

elapsed ) after  the HANDOVER

COMMAND is sent by cell A.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency List after time

- Frequency List				Use Range 1024 to allocate the

following 12 frequencies :

(749, 758, 761, 764, 771, 779, 782, 791, 798, 829, 832, 844 )

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Starting Time				Indicates the frame number of cell B

that will occur approximately 1.1

seconds ( 238 frames have

elapsed ) after  the HANDOVER

COMMAND is sent by cell A.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell B.

For M = 6 :

GSM900

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			20

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell A.

Synchronisation Indication IE not included.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in non-hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			747

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell A.

Synchronisation Indication IE not included.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in non-hopping mode on cell A.

For M = 7 :

GSM900

Step 0 : The MS and SS are using a half rate TCH in non-hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Channel Sequence IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency Channel Sequence after time

- Frequency Channel Sequence		Allocates the following 8

frequencies :

( 40, 66, 73, 74, 75, 76,108, 114 )

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of first channel				speech half rate.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a half rate TCH in non-hopping mode on cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency List after time

- Frequency List				Use Variable Bit Map to encode the

following 8 frequencies :

( 764, 779, 782, 791, 798, 829, 832, 844 )

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of first channel			speech half rate.

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH in hopping mode on cell B.

For M = 8 :

GSM900

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			20

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF channel

- ARFCN					20

Synchronisation Indication IE not included.

Mode of First Channel			Speech full rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non-hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a half rate TCH in hopping mode on cell B.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			747

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Chosen arbitrarily but not Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF channel

- ARFCN					747

Synchronisation Indication IE not included.

Mode of First Channel			Speech full rate

PHYSICAL INFORMATION

Information Element				value/remarks

As default message contents, except :

Timing advance				Arbitrarily selected but different to

default value.

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH in non-hopping mode on cell A.

Specific Message Contents For Mobiles not Supporting Speech

If the mobile station supports half rate, then the 12 kbps radio interface rate is not used for this test. With this restriction, the radio interface rate is selected arbitrarily from those support.

The message contents shall be the same for the declared type of Mobile Station ( either P-GSM900 or DCS1800 ) supporting speech , except for :

M = 3 and 8 :

HANDOVER COMMAND

Information Element				value/remarks

Mode of first channel				The full rate radio interface

rate that is in use.

For M = 4 and 7 :

HANDOVER COMMAND

Information Element				value/remarks

Mode of first channel				The same radio interface

rate as M = 3, but for a half

rate channel.

#### 26.6.5.2	Handover / successful / call under establishment / non-synchronized

##### 26.6.5.2.1	Conformance requirements

The MS shall correctly apply the handover procedure from SDCCH/8, TCH/F or TCH/H with or without frequency hopping to SDCCH/8, TCH/F or TCH/H with or without frequency hopping in the non-synchronized case during call establishment. The mobile shall correctly apply the handover procedures from non frequency hopping SDCCH/4 to SDCCH/8, TCH/F or TCH/H with or without frequency hopping. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

References

GSM 04.08, sections 3.1.4.2, 3.4.4 and 9.1.15

GSM 04.13, section 5.2.6.2

##### 26.6.5.2.2	Test purpose

To test that when the MS is ordered to make a non-synchronized handover, it continuously sends access bursts on the main DCCH until it receives a PHYSICAL INFORMATION message from the SS. To test that the MS correctly takes the values of the Timing Advance information element in the PHYSICAL INFORMATION message into account. To test that the MS activates the new channel correctly and transmits the HANDOVER COMPLETE message without undue delay. To test that the MS correctly retransmitsLayer 3 MM or CC messages, that were not acknowledged by Layer 2 before the Handover, after completion of the Handover.

##### 26.6.5.2.3	Method of test

Initial Conditions

System Simulator:

2 cells A and B with same LAI, default parameters, except:

GSM900 :

Cell A has :

BCCH ARFCN = 20

Cell Allocation = (10, 17, 20, 26, 34, 42, 45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114)

PLMN colour code, NCC = as defaults.

BS colour code, BCC = as defaults.

Cell B has :

BCCH ARFCN = 40

Cell Allocation = (14, 18, 22, 24, 30, 31, 38, 40, 60, 66, 73, 74, 75, 76, 108, 114)

PLMN colour code, NCC = 3.

BS colour code, BCC = 0.

Both cells send SYSTEM INFORMATION TYPE 1 messages containing the complete Cell Allocation of the cell, using bit map 0 format.

The timebase of Cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

For execution counter M = 1, 2 and 3 a combined CCH/SDCCH is used.

For execution counter M = 4 to 10 a non combined SDCCH is used.

DCS1800 :

Cell A has :

BCCH ARFCN = 747

Cell Allocation = (734, 741, 747, 754, 759, 762, 766, 767, 773, 775, 779, 782, 791, 	798, 829, 	832, 844)

PLMN colour code, NCC = as defaults.

BS colour code, BCC = as defaults.

Cell B has :

BCCH ARFCN = 764

Cell Allocation = ( 739, 743, 746, 749, 756, 758, 761, 764, 771, 779, 782, 791, 798, 	829, 832, 	844)

PLMN colour code, NCC = 3.

BS colour code, BCC = 0.

Both cells send SYSTEM INFORMATION TYPE 1 messages containing the complete Cell Allocation of the cell, using Range 512 format.

The timebase of Cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

For execution counter M = 1, 2 and 3 a combined CCH/SDCCH is used.

For execution counter M = 4 to 10 a non combined SDCCH is used.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Supported speech and data rates.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated" with TMSI allocated and camped on cell B.

Test Procedure

This procedure is repeated for execution counter M = 1, 2 .. 10 ( See Table 1 )

A Mobile Originating Call is initiated on Cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND message, ordering the MS to switch to cell B. The MS shall then begin to send access bursts on the new DCCH to cell B. The SS observes the access bursts and after receiving n (n being randomly drawn between values according to table 2 of section 26.6.5) access bursts, the SS sends one PHYSICAL INFORMATION message with a Timing Advance as specified in table 1 of section 26.6.5. The MS shall activate the channel in sending and receiving mode. The MS shall establish a signalling link. The MS shall be ready to transmit a HANDOVER COMPLETE message before x ms after the end of the PHYSICAL INFORMATION message, but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

The term 'ready to transmit' is defined in TS GSM 04.13. The value of 'x' depends upon the target channel and is specified in the specific message contents section.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

The sequence is performed for execution counter M = 1, 2..10 (unless a particular TCH is not supported).

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated on cell A.
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call,
				NECI not set to 1
	3	SS -> MS	IMM ASSIGNMENT	See specific message contents.
	  4	MS -> SS	CM SERVICE REQUEST	CM Service Type = Mobile Origina-
				ting Call Establishment.
	  5	SS -> MS	CIPH MODE COMMAND
	  6	MS -> SS	CIPH MODE COMPLETE
	  7	MS -> SS	SETUP	Last L2 frame not acknowledged by
				the SS.
	 8	SS -> MS	HANDOVER COMMAND	See specific message contents.
	 9	MS -> SS	HANDOVER ACCESS	Repeated on every burst of the uplink main 
				DCCH until reception of PHYSICAL
				INFORMATION. Handover Reference as
				included in the HANDOVER COMMAND
	 10	SS -> MS	PHYSICAL INFO	Sent after reception of n HANDOVER
				ACCESS message. Timing Advance as
				specified in table 1 of section
				26.6.5.
	11	MS -> SS	SABM	Sent without information field
	12	SS -> MS	UA
	 13	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted 
				before 'x' ms after the completion of step 10.
	 14	MS -> SS	SETUP	Same N(SD) as in step 7.
	 15	SS -> MS	CHANNEL RELEASE

Specific Message Contents For Mobiles Supporting Speech

M = 1

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Short List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Synchronisation IE is not included.

Frequency Short List after time

- Frequency Short List			Use Range 128 to encode the

following 11 frequencies :

( 756, 758, 761, 771, 779, 782, 791, 798, 829, 832, 844 )

Mode of the First Channel			Speech full rate

Step 13 : 'x' = 500

For GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Channel Sequence IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Synchronisation IE is not included.

Frequency Channel Sequence, after time

- Frequency List				Allocates the following 15

frequencies :

(14, 18, 22, 24, 30, 31, 38, 60, 66, 73, 74, 75, 76, 108, 114 )

Mode of the First Channel			Speech full rate.

Step 13 : 'x' = 500

M = 2

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set ( 1,

2.. 63)

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Cell Channel Description			Use Range 512 to encode the

following frequencies :

( 761, 764, 771, 779, 782, 791, 798, 829, 832 )

Mobile Allocation after time			Indicates  ( 791, 798, 829 ) only )

Mode of First Channel			Speech Half rate

Step 13 : 'x' = 750

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Chosen arbitrarily

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set ( 1,

2.. 63)

Synchronisation Indication IE is not included

Cell Channel Description			uses bit map 0 to encode :

{40, 66, 73, 74, 75, 76, 108, 114} only

Mobile Allocation after time			indicates channel {73, 74, 75} only.

Channel Mode IE

speech half rate

Step 13 : 'x' = 750

M = 3

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Short List IE.

- HSN					Zero ( this gives cyclic hopping )

Synchronisation Indication IE not included.

Frequency Short List after time

- Frequency Short List			Use Range 128 to encode the

following 2 frequencies ( 746, 779 )

Mode of First Channel			Signalling Only

Step 13 : 'x' = 1500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN				Chosen arbitrarily from the set

{1, 2, ..., 63}.

Synchronisation Indication IE is not included	Frequency list after time

- Frequency List			uses bit map 0 to allocate the

following 15 frequencies

{14, 18, 22, 24, 30, 31, 38, 60, 66, 73, 74, 75, 76, 108, 114}

Channel Mode IE

signalling only

Step 13 : 'x' = 1500

M = 4

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

- Timeslot number				Arbitrary value, but not zero.

- ARFCN					747

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					zero ( this gives cyclic hopping )

Frequency List after time

- Frequency List				Use Range 1024 to encode the

complete CA of Cell B

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Channel Mode IE is not included.

Step 13 : 'x' = 1500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

- Timeslot number				Arbitrary value, but not zero.

- ARFCN					20

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value, but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Channel Sequence IE.

Frequency Channel Sequence, after time

- Frequency channel sequence	Allocates the following 14

frequencies :

{14, 18, 22, 24, 30, 40, 31, 38, 60, 66, 73, 74, 75, 76, 108}

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Channel Mode IE is not included

Step 13 : 'x' = 1500

M = 5

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

- Channel Type				TCH/F + ACCH's

- Timeslot number				Arbitrary value but not zero.

- Hopping Channel				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/H + ACCH's

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of cell B.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Signalling only

Step 13 : 'x' = 750

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

- Channel Type				TCH/F + ACCH's

- Timeslot number				Arbitrary value, but not zero.

- Hopping Channel				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/H + ACCH's

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of cell B.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Signalling only

Step 13 : 'x' = 750

M = 6

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation

- Length					3 octets

- Contents				Indicates all of the CA of cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency List after time

- Frequency List				Use Range 256 to encode the

following 9 frequencies :

(746, 749, 756, 761, 764, 798, 829, 832,844)

Synchronisation Indication IE not included.

Channel Mode				Signalling Only

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation

- Length					3 octets

- Contents				Indicates all of the CA of cell A.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Cell Channel Description IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Cell Channel Description			uses bit map 0 to encode the

complete CA of cell B.

Synchronisation Indication IE not included.

Channel Mode				Signalling Only

Step 13 : 'x' = 500

M = 7 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Zero

Mobile Allocation

- Length					3 octets

- Contents				Indicates only one frequency ( 844 )

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Cell Channel Description			Use Variable bit map to encode the

complete CA of cell B.

Mobile Allocation

Indicates all of the CA of cell B

except for the following three

frequencies : ( 764, 832 and 844 )

Mode of First channel				Speech full rate

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Zero

Mobile Allocation

- Length					3 octets

- Contents				Indicates only one frequency ( 114 )

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Synchronisation Indication IE is not included:

Frequency list after time.

- Frequency List IE			uses bit map 0 to allocate the

following 16 frequencies

{14, 18, 22, 24, 30, 31, 38, 40, 60, 66, 73, 74, 75, 76, 108, 114}

Mode of First channel				Speech full rate

Step 13 : 'x' = 500

M = 8 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation

- Length					3octets.

- Contents				Indicates only three frequencies :

( 773, 775, 779 ).

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell B, but

not the

BCCH carrier of Cell B.

Synchronisation Indication IE not included.

Mode of First Channel			Full rate speech

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

L2 pseudo length				14 octets ( 11 + contents of the MA )

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value, but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation

- Length					3 octets.

- Contents				Indicates only three frequencies :

( 73, 74, 75 ).

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell B, but

not the

BCCH carrier of Cell B.

Mode of First Channel			Full rate speech

Step 13 : 'x' = 500

M = 9 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value, but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Short List IE.

- HSN					Zero ( this gives cyclic hopping )

Frequency Short List after time

- Frequency Short List			Use Range 256 to encode the

following 3 frequencies :

( 764, 779, 782 )

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Non synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Channel mode				Speech full rate

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code		3

- Base Station Colour Code		0

- BCCH Carrier Number		40

Channel Description

- Channel Type			TCH/F + ACCHs

- Timeslot number			zero

- Training Sequence Code		Chosen arbitrarily

- Hopping			RF hopping channel

- MAIO				Chosen arbitrarily from the set {0, 1}.

- HSN				Chosen arbitrarily from the set

{1, 2,..., 63}.

Synchronisation Indication IE is not included

Channel Mode IE

speech full rate

Frequency Channel Sequence, after time.

- Frequency Channel Sequence  IE	allocates the following two

frequencies {14, 114}

Channel mode				Speech full rate

Step 13 : 'x' = 500

M = 10 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code			3

- Base Station Colour Code			0

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value, but not zero.

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Frequency List IE.

- HSN					Zero ( this gives cyclic hopping )

Frequency List after time

- Frequency List				Use Variable Bit Map to encode the

following 15 frequencies :

(739, 743,746,749, 756, 758, 764, 771, 779, 782, 791, 798, 829, 832, 844)

Synchronisation Indication IE is not included.

Channel mode				Speech half rate

Step 13 : 'x' = 750

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents.

HANDOVER COMMAND

Information Element				value/remarks

As default message contents, except :

Cell Description

- Network Colour Code		3

- Base Station Colour Code		0

- BCCH Carrier Number		40

Channel Description

- Channel Type			TCH/H + ACCHs

- TDMA offset			Chosen arbitrarily

- Timeslot number			Chosen arbitrarily but not zero

- Training Sequence Code		Chosen arbitrarily

- Hopping			RF hopping channel

- MAIO				Chosen arbitrarily from the set {0, 1}

- HSN				Chosen arbitrarily from the set

{1, 2,..., 63}.

Synchronisation Indication IE is not included

Frequency Channel Sequence, after time

- Frequency Channel Sequence IE	allocates the following two

frequencies {40, 114}

Channel Mode IE				speech half rate

Step 13 : 'x' = 750

Specific Message Contents For Mobiles not Supporting Speech

The message contents shall be the same for the declared type of Mobile Station ( either P-GSM900 or DCS1800 ) supporting speech , except for :

M = 1, 7, 8 and 9 :

HANDOVER COMMAND

Information Element				value/remarks

Mode of first channel				Arbitrary from those supported ( 12,

6, 3.6 kbps )

For M =  2 and 10  :

HANDOVER COMMAND

Information Element				value/remarks

Mode of first channel					Arbitrary from those supported

(6, 3.6 kbps )

#### 26.6.5.3	Handover / successful / active call / finely synchronized

##### 26.6.5.3.1	Conformance requirements

The MS shall correctly apply the handover procedure from TCH/F with frequency hopping to TCH/F without frequency hopping in the finely synchronized case when a call is in progress.

The MS shall correctly apply the handover procedure from TCH/H with frequency hopping to TCH/H without frequency hopping in the finely synchronized case when a call is in progress. This requirement does not apply to MSs not supporting TCH/H.

References

GSM 04.08 section 3.4.4, 9.1.14, 9.1.15 and 9.1.16.

GSM 04.13 section 5.2.6

GSM 05.05 section 4.1.1.

GSM 05.10, section 6.6

##### 26.6.5.3.2	Test purpose

To test that when the MS is ordered to make a finely synchronized handover to a synchronized cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly, taking into account the value of any Starting Time information element, power command and correctly calculating the timing advance to use. To test the MS activates the new channel correctly and transmits the HANDOVER COMPLETE message without undue delay.

##### 26.6.5.3.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI, default parameters, except:

The BCCH of cell A is sent k bit periods before the BCCH of cell B. The timing advance in cell A sent to the MS is y bit periods. k and y are selected such that 0 < 2k+y < 60

P-GSM900 :

Cell B has BCCH ARFCN = 40

DCS1800 :

Cell B has BCCH ARFCN = 764

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). The MS is using a power level P. Where P is a power level within the supported range of that type of MS.

[editorial note : since not all MSs support such high power levels, it might be better to choose e

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F and/or TCH/H.

Support for state U10 of the Call Control protocol.

Type of Mobile Station ( P-GSM900 or EGSM or DCS1800 ).

Power class of Mobile Station.

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell B).

Test Procedure

This procedure is repeated for execution counter M = 1 to 2. ( See Table 1 )

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall (at the time specified in the Starting Time information element, if included) send 4 access bursts, in 4 successive slots on the new DCCH to cell B. Then the MS shall establish a signalling link indicating the correct Timing Advance and power level and send a HANDOVER COMPLETE message.

The MS shall be 'ready to transmit' a HANDOVER COMPLETE message before 'x' ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in TS GSM 04.13. The value of 'x' depends upon the target channel and is specified in the specific message contents section.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter M = 1 for an MS which only supports TCH/F

This sequence is performed for execution counter M = 1 to 2 for an MS which supports TCH/F and H

Step	Direction	Message	Comments

0	MS -> SS		M = 1, The MS and SS are using a full rate 
				TCH in hopping mode on cell A.
				M = 2, The MS and SS are using a half rate 
				TCH in hopping mode on cell A.

	1	SS -> MS	HANDOVER COMMAND	See Specific Message Contents
	2	MS -> SS	HANDOVER ACCESS	See specific message contents. Four messages 
	3	MS -> SS	HANDOVER ACCESS	are transmitted to Cell B in 4 successive slots 
	4	MS -> SS	HANDOVER ACCESS	on the new DCCH.
	5	MS -> SS	HANDOVER ACCESS	
	6	MS -> SS	SABM	Sent without information field
	7	SS -> MS	UA
	8	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted 
				before 'x' ms after the completion of step 1. 
				See specific message contents.
	9	SS		The header of the next uplink SACCH is 
				examined and the Timing Advance and Power 
				Level indications are examined. The correct 
				timing advance shall be indicated. The power 
				level indication shall indicate the power 
				level used in the handover command.
	10	MS, SS		M =1, The MS and SS are using a full rate 
				TCH in non-hopping mode on cell B
				M = 2, The MS and SS are using a half rate 
				TCH in non-hopping mode on cell B.

Specific Message Contents

M = 1 :

GSM900

HANDOVER COMMAND

Information Element				value/remark

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel type				TCH/F + ACCHs

- Timeslot Number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					40

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by that type of MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step  8 : x = 650 ms

DCS1800

HANDOVER COMMAND

Information Element				value/remark

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel type				TCH/F + ACCHs

- Timeslot Number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					764

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by that type of MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step  8 : x = 650 ms

M = 2 :

GSM900

HANDOVER COMMAND

Information Element				value/remark

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot Number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					40

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by that type of MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Speech half rate

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step  8 : x = 900 ms

DCS1800

HANDOVER COMMAND

Information Element				value/remark

As default message contents, except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel type				TCH/H + ACCHs

- TDMA offset				Chosen arbitrarily

- Timeslot Number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					764

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by that type of MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Speech half rate

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step  8 : x = 900 ms

#### 26.6.5.4	Handover / successful / call under establishment / finely synchronized

##### 26.6.5.4.1	Conformance requirements

The MS shall correctly apply the handover procedure from SDCCH/8 or TCH/F with or without frequency hopping to SDCCH4, SDCCH/8 or TCH/F with or without frequency hopping in the finely synchronized case, during call establishment.

If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

References

GSM 05.10, section 6.6

GSM 04.08, section 3.4.4

GSM 04.08, section 3.1.4.2

GSM 04.13, section 5.2.6

##### 26.6.5.4.2	Test purpose

To test that when the MS is ordered to make a finely synchronized handover to a synchronized cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly, taking into account the value of any Starting Time information element, power command and correctly calculating the timing advance to use. To test that the MS correctly retransmits Layer 3 MM or CC messages that were not acknowledged by Layer 2 before the Handover, after completion of the Handover. To verify the MS transmits the HANDOVER COMPLETE message without undue delay.

##### 26.6.5.4.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI, default parameters, except:

The BCCH of cell A is sent k bit periods before the BCCH of cell B. The timing advance in cell A sent to the MS is y bit periods. k and y are selected such that 0 < 2k + y < 60

The frame numbers of cells A and B shall be different by 100.

GSM900 :

Cell A has :

BCCH ARFCN = 20

Cell Allocation = (10, 17, 20, 26, 34, 42, 45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114)

Cell B has :

BCCH ARFCN = 40

Cell Allocation = (14, 18, 22, 24, 30, 31, 38, 40, 60, 66, 73, 74, 75, 76, 108, 114)

DCS1800 :

Cell A has :

BCCH ARFCN = 747

Cell Allocation = (734, 741, 747, 754, 759, 762, 766, 767, 773, 775, 779, 782, 791, 	798, 829, 	832, 844 )

Cell B has :

BCCH ARFCN = 764

Cell Allocation = (739, 743, 746, 749, 756, 758, 761, 764, 771, 779, 782, 791, 798, 	829, 832, 	844 )

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A. The MS is using a power level P, where P is a power level within the supported range of that type of MS.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of Mobile Station ( P-GSM900 or EGSM or DCS1800 ).

Power class of Mobile Station.

Foreseen Final State of the MS

"Idle, updated" with TMSI allocated and camped on cell B.

Test Procedure

This procedure is repeated for execution counter M = 1, 2, 3, 4 ( See Table 1 )

A Mobile Originating Call is initiated on cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND message, ordering the MS to switch to cell B. The MS shall then (at the time specified in the Starting Time information element, if included) send 4 access bursts, in successive slots on the new DCCH to cell B. Then the MS shall establish a signalling link indicating the correct timing advance and power level and send a HANDOVER COMPLETE message. The MS shall be 'ready to transmit' the HANDOVER COMPLETE message before 'x' ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

The term 'ready to transmit' is defined in TS GSM 04.13. The value 'x' depends upon the target channel and is specified in the specific message contents section.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This procedure is repeated for execution counter M = 1, 2, 3, 4 (unless a particular TCH is not supported)

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call,
				NECI not set to 1"
	3	SS -> MS	IMM ASSIGNMENT	See Specific Message contents.
	4	MS -> SS	CM SERVICE REQUEST	CM Service Type = Mobile Origina-
				ting Call Establishment.
	5	SS -> MS	CIPH MODE COMMAND
	6	MS -> SS	CIPH MODE COMPLETE
	7	MS -> SS	SETUP	Last L2 frame not acknowledged by
				the SS.
	8	SS -> MS	HANDOVER COMMAND	See Specific Message Contents.
	9	MS -> SS	HANDOVER ACCESS	
	10	MS -> SS	HANDOVER ACCESS	
	11	MS -> SS	HANDOVER ACCESS	See Specific message contents. Four
	12	MS -> SS	HANDOVER ACCESS	Messages are transmitted to cell B in 4 
				successive slots on the new DCCH. If  the 
				HANDOVER COMMAND message includes a 
				starting time IE then the first HANDOVER 
				ACCESS message shall be transmitted in the 
				indicated frame ( unless the starting time has 
				elapsed ).
	13	MS -> SS	SABM	Sent without information field
	14	SS -> MS	UA
	15	MS -> SS	HANDOVER COMPLETE	The message shall be ready to be transmitted
				before 'x' ms after the completion of step 8.
	16	SS		The header of the next uplink SACCH is 
				examined and the Timing Advance and Power 
				Level indications are examined. The correct 
				timing advance shall be indicated. The power 
				level indication shall indicate the power level 
				used in the handover command.
	17	MS -> SS	SETUP	Same N(SD) as in step 7
	18	SS -> MS	CHANNEL RELEASE

Specific Message Contents

M = 1 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation				Indicates all of the CA of cell A

except for the BCCH frequency

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Zero ( this gives cyclic hopping )

Frequency List after time

- Frequency List				Use Range 512 to encode the

complete CA of Cell B.

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Signalling only

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  1500 ms

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Mobile Allocation				Indicates all of the CA of cell A

except for the BCCH frequency

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				Chosen arbitrarily

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency List IE.

- HSN					Zero ( this gives cyclic hopping )

Frequency List after time

- Frequency List				Use Bit Map 0 to encode the

complete CA of Cell B.

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Mode of First Channel			Signalling only

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  1500 ms

M = 2 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Zero ( this gives cyclic hopping ).

Mobile Allocation				Indicates all of the CA of cell A

except for the following 2 frequencies:

( 747 and 767 )

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				SDCCH/4

- TDMA offset				Chosen arbitrarily

- Timeslot number				zero

- Training Sequence Code			same as the BCCH

- Hopping				Single RF Channel

- ARFCN					764

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Starting Time				Indicates the frame number of cell B

that will occur approximately 1.1

seconds after the HANDOVER

COMMAND message is sent by cell A.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  1500 ms

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the Mobile

Allocation.

- HSN					Zero ( this gives cyclic hopping ).

Mobile Allocation				Indicates all of the CA of cell A

except for the following 2 frequencies:

( 20 and 52 )

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				SDCCH/4

- TDMA offset				Chosen arbitrarily

- Timeslot number				zero

- Training Sequence Code			same as the BCCH

- Hopping				Single RF Channel

- ARFCN					40

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Starting Time				Indicates the frame number of cell B

that will occur approximately 1.1

seconds after the HANDOVER

COMMAND message is sent by cell A.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  1500 ms

M = 3 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					747

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Short list IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency Short List after time

- Frequency Short List			Use Range 128 to encode the following 3

frequencies : (758, 761, 771)

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  650 ms

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					20

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				RF hopping channel.

- MAIO					Chosen arbitrarily from the set ( 0, 1

to N-1 ), where N is the number of

frequencies encoded in the

Frequency Channel Sequence IE.

- HSN					Chosen arbitrarily from the set

( 1,2,..63 )

Frequency  Channel Sequence after time

- Frequency Channel Sequence

Indicates (66, 75, 76, 108 )

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication		'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  650 ms

M = 4 :

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				As default message contents

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell A.

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			764

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell B.

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Channel Mode				Full rate speech

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  650 ms

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

As default message contents except :

Channel Description

Channel Description

- Channel Type				SDCCH/8

- TDMA offset				As default message contents

- Timeslot number				As default message contents

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell A.

HANDOVER COMMAND

Information Element				value/remark

As default message contents except :

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			40

Channel Description

- Channel Type				TCH/F + ACCHs

- Timeslot number				Arbitrary value, but not zero

- Training Sequence Code			Chosen arbitrarily

- Hopping				Single RF Channel.

- ARFCN					Chosen arbitrarily from the Cell

Allocation of Cell B.

Handover Reference

- Value					Chosen arbitrarily from the range ( 0,

1..255 )

Power command

- Power Level				Arbitrarily chosen, but different to the

one already in use and within the

range supported by the MS.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

Channel Mode				Full rate speech

HANDOVER ACCESS

Information Element				value/remark

As default message contents except :

Handover Reference

- Value					Same as HANDOVER COMMAND

Step 15 : x =  650 ms

#### 26.6.5.5	Pre-synchronized handovers

##### 26.6.5.5.1	Handover / successful / active call / pre-synchronized / Timing Advance IE not included

If an MS does not implement the pre-synchronized handover procedure correctly then calls may fail.

###### 26.6.5.5.1.1	Conformance requirements

1	The MS shall correctly apply the handover procedure from TCH/F without frequency hopping to TCH/F without frequency hopping in the pre-synchronized case when a call is active.

2	When the Timing Advance information element is not included in the HANDOVER COMMAND, the MS shall access the new cell with the default timing advance of 1 bit period.

3	The MS shall be ready to transmit the HANDOVER COMPLETE message within 650 ms of the end of the HANDOVER COMMAND message.

References

Conformance requirement 1: GSM 04.08 section 3.4.4.

Conformance requirement 2: GSM 05.10, section 6.6.

Conformance requirement 3: GSM 04.13, section 5.2.6.1.

###### 26.6.5.5.1.2	Test purpose

To verify that when the MS is ordered to make a pre-synchronized handover to another cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly and correctly calculates the time to transmit.

###### 26.6.5.5.1.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B. k is arbitrarily selected.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A) using a full rate TCH in non-hopping mode.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell B) using a full rate TCH in non-hopping mode.

Test Procedure

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND for a pre-synchronised handover without the Timing Advance IE on the main DCCH. The MS shall send 4 access bursts, at the commanded power level, in 4 successive slots of the new DCCH to cell B with a Timing Advance of zero. Then the MS shall establish a signalling link using a Timing Advance of one and send a HANDOVER COMPLETE message. The MS shall be ready to transmit the HANDOVER COMPLETE message before 650 ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in GSM 04.13.

Maximum Duration of Test

5 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	See specific message contents below.
	2	MS -> SS	HANDOVER ACCESS	Handover Reference as included in the
	3	MS -> SS	HANDOVER ACCESS	HANDOVER COMMAND.
	4	MS -> SS	HANDOVER ACCESS	
	5	MS -> SS	HANDOVER ACCESS	
	6	MS -> SS	SABM	Sent without information field.
	7	SS -> MS	UA
	8	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted
				before 650 ms after the completion of step 1.
	9	SS	-	The SS checks that the timing
				advance reported in the layer 1
				header of the SACCH message
				that is sent in the first
				SACCH multiframe following the
				SABM is 1 bit period.

Specific Message Contents

HANDOVER COMMAND

As default message contents, except:

Synchronisation Indication			pre-synchronized; ROT=0; NCI=0.

##### 26.6.5.5.2	Handover / successful / call being established / pre-synchronized / timing advance IE is included / reporting of observed time difference requested.

If an MS does not implement the pre-synchronized handover procedure correctly then calls may fail.

If an MS does not report the observed time difference between cells correctly then pseudo synchronised handovers might not be possible for any MS.

###### 26.6.5.5.2.1	Conformance requirements

1	The MS shall correctly apply the handover procedure from an SDCCH/4 to a TCH/F without frequency hopping in the pre-synchronized case while a call is being established.

2	If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

3	When the Timing Advance information element is included in the HANDOVER COMMAND, the MS shall access the new cell with the timing advance included in the Timing Advance IE.

4	The MS shall be ready to transmit the HANDOVER COMPLETE message within 650 ms of the end of the HANDOVER COMMAND message.

5	When requested to do so in the HANDOVER COMMAND message, the MS shall return the Mobile Time Difference IE in the HANDOVER COMPLETE message indicating the sum of the observed time difference between the cells and the timing advance used on the old cell.

References

Conformance requirement 1: GSM 04.08, section 3.4.4.

Conformance requirement 2: GSM 04.08, section 3.1.4.3.

Conformance requirement 3: GSM 05.10, section 6.6.

Conformance requirement 4: GSM 04.13, section 5.2.6.1.

Conformance requirement 5: GSM 04.08, section 10.5.2.39.

###### 26.6.5.5.2.2	Test purpose

To test that when the MS is ordered to make a pre-synchronized handover to another cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly and correctly calculates the time to transmit. To test that the MS correctly retransmits Layer 3 MM or CC messages that were not acknowledged by Layer 2 before the Handover, after completion of the Handover. To test that the MS correctly reports on the time difference between the cells.

###### 26.6.5.5.2.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated" with TMSI allocated and camped on cell B.

Test Procedure

A Mobile Originating Call is initiated. The SS sends an IMMEDIATE ASSIGNMENT message allocating an SDCCH/4. The MS is commanded to use a timing advance of y bit periods on cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND, ordering the MS to switch to cell B. The MS shall then send 4 access bursts, at the commanded power level, in 4 successive slots of the new DCCH to cell B. Then the MS shall establish a signalling link using the correct timing advance and send a HANDOVER COMPLETE message. The MS shall be ready to transmit the HANDOVER COMPLETE message before 650 ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in GSM 04.13.

The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test

20 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMM ASSIGNMENT	to an SDCCH/4.
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS -> MS	CIPH MODE COMMAND
	6	MS -> SS	CIPH MODE COMPLETE
	7	MS -> SS	SETUP	Last L2 frame not acknowledged by the SS.
	8	SS -> MS	HANDOVER COMMAND	See specific message contents below.
	9	MS -> SS	HANDOVER ACCESS	Handover Reference as included in the
	10	MS -> SS	HANDOVER ACCESS	HANDOVER COMMAND
	11	MS -> SS	HANDOVER ACCESS	
	12	MS -> SS	HANDOVER ACCESS	
	13	MS -> SS	SABM	Sent without information field.
	14	SS -> MS	UA
	15	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted
				before 650 ms after the completion of step 8.
				Shall include the Mobile Time Difference IE
				with value (2k+y) mod 127 500 half bit
				periods. A tolerance of ±2 half bit periods is
				allowed.
	16	MS -> SS	SETUP	Same N(SD) as in step 7
	17	SS	-	The SS checks that the timing
				advance reported in the layer 1
				header of the SACCH message
				that is sent in the first
				SACCH multiframe following the
				SABM is 9 bit periods.
	18	SS -> MS	CHANNEL RELEASE

Specific Message Contents

HANDOVER COMMAND

As default message contents, except:

Synchronisation Indication			pre-synchronized; ROT=1; NCI=0.

Timing Advance				9 bit periods.

#### 26.6.5.6	Handover / successful / active call / pseudo synchronized

This test only applies to MSs that claim to support the pseudo synchronized handover procedure. If MSs that claim to support this procedure do not correctly implement it, then calls may fail.

##### 26.6.5.6.1	Conformance requirements

1	The MS shall correctly apply the handover procedure from TCH/F without frequency hopping to TCH/F without frequency hopping in the pseudo synchronized case when a call is in progress.

2	The MS shall access the new cell with the correct timing advance.

3	The MS shall be ready to transmit the HANDOVER COMPLETE message within 650 ms of the end of the HANDOVER COMMAND message.

4	When requested to do so in the HANDOVER COMMAND message, the MS shall return the Mobile Time Difference IE in the HANDOVER COMPLETE message indicating the sum of the observed time difference between the cells and the timing advance used on the old cell.

References

Conformance requirement 1: GSM 04.08 section 3.4.4.

Conformance requirement 2: GSM 05.10, section 6.6.

Conformance requirement 3: GSM 04.13, section 5.2.6.1.

Conformance requirement 4: GSM 04.08, section 10.5.2.39.

##### 26.6.5.6.2	Test purpose

To test that when the MS is ordered to make a pseudo synchronized handover to another cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly and correctly calculates the time to transmit. To test that the MS correctly reports the time difference between the cells.

##### 26.6.5.6.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B. k is arbitrarily selected.

The MS is being commanded to use a timing advance of y bit periods on cell A, where y is arbitrarily selected from the set {11, 12, ... ,62}.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A) using a full rate TCH in non-hopping mode.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Support for pseudo synchronisation.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell B) using a full rate TCH in non-hopping mode.

Test Procedure

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND for a pseudo-synchronised handover with the Real Time Difference IE included. The Time Difference value is set to (2k+10) modulo 256. The MS shall send 4 access bursts, at the commanded power level, in 4 successive slots of the new DCCH to cell B with a Timing Advance of zero. Then the MS shall establish a signalling link using a Timing Advance of (y-10) bit periods and send a HANDOVER COMPLETE message. The MS shall be ready to transmit the HANDOVER COMPLETE message before 650 ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in GSM 04.13.

Maximum Duration of Test

5 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	See specific message contents below.
	2	MS -> SS	HANDOVER ACCESS	Handover Reference as included in the
	3	MS -> SS	HANDOVER ACCESS	HANDOVER COMMAND.
	4	MS -> SS	HANDOVER ACCESS	
	5	MS -> SS	HANDOVER ACCESS	
	6	MS -> SS	SABM	Sent without information field
	7	SS -> MS	UA
	8	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted
				before 650 ms after the completion of step 1.
				Shall include the Mobile Time Difference IE with
				value (2k+y) mod 127 500 half bit periods. A
				tolerance of ±2 half bit periods is allowed.
	9	SS	-	The SS checks that the timing
				advance reported in the layer 1
				header of the SACCH message
				that is sent in the first
				SACCH multiframe following the
				SABM is (y-10) bit periods.

Specific Message Contents

HANDOVER COMMAND

As default message contents, except:

Synchronisation Indication			pseudo-synchronized; ROT=1; NCI=0.

Time Difference				(2k+10) mod 256.

#### 26.6.5.7	Handover / successful / active call / non-synchronized / reporting of observed time difference requested.

If an MS does not report the observed time difference between cells correctly then pseudo synchronised handovers might not be possible for any MS.

##### 26.6.5.7.1	Conformance requirements

1	The MS shall correctly apply the handover procedure from a TCH/F without frequency hopping to a TCH/F without frequency hopping in the non-synchronized case while a call is active.

2	When requested to do so in the HANDOVER COMMAND message, the MS shall return the Mobile Time Difference IE in the HANDOVER COMPLETE message indicating the sum of the observed time difference between the cells and the timing advance used on the old cell.

References

Conformance requirement 1: GSM 04.08, section 3.4.4.

Conformance requirement 2: GSM 04.08, section 10.5.2.39.

##### 26.6.5.7.2	Test purpose

To verify that when the MS is ordered to make a non-synchronized handover to another cell and is ordered to report on the time difference between the cells, that it does so correctly.

##### 26.6.5.7.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B with default parameters except the LAI of cell B has MNC = 02 decimal, MCC = 315 decimal, and LAC = 5344 H.

The BCCH of cell A is sent k bit periods before the BCCH of cell B.

The MS is commanded to use a timing advance of y bit periods on cell A.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A) using a full rate TCH in non-hopping mode.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell B) using a full rate TCH in non-hopping mode.

Test Procedure

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The HANDOVER COMMAND includes a Synchronisation Indication IE that instructs the MS to supply the observed time difference between the cells. The MS shall begin to send access bursts on the new DCCH to cell B and the SS sends one PHYSICAL INFORMATION message. The MS shall activate the channel in sending and receiving mode and establish a signalling link using the correct timing advance. The MS shall transmit a HANDOVER COMPLETE message containing the Mobile Time Difference IE with a correct value.

Maximum Duration of Test

5 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	See specific message contents below.
	2	MS -> SS	HANDOVER ACCESS	Repeated on every burst of the uplink main
				DCCH until reception of PHYSICAL
				INFORMATION. Handover Reference as
				included in the HANDOVER COMMAND.
	3	SS -> MS	PHYSICAL INFORMATION	
	4	MS -> SS	SABM	Sent without information field
	5	SS -> MS	UA
	6	MS -> SS	HANDOVER COMPLETE	Shall include the Mobile Time Difference IE
				with value (2k+y) mod 127 500 half bit
				periods. A tolerance of ±2 half bit periods is
				allowed.

Specific Message Contents

HANDOVER COMMAND

as default message contents, except:

Synchronisation Indication			'not synchronised'; ROT=1; NCI=0.

#### 26.6.5.8	Handover / layer 3 failure

##### 26.6.5.8.1	Conformance requirements

The MS shall return to the old channel in the case of an handover failure caused by the non reception of the PHYSICAL INFORMATION message. On the old channel the MS shall use the Power Level that it was previously using on that channel.

References

GSM 04.08 section 3.4.4.

##### 26.6.5.8.2	Test purpose

To verify the function of timer T3124 and the contents in the message HANDOVER FAILURE and in the layer 1 header on the SACCH.

##### 26.6.5.8.3	Method of test

Initial Conditions

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). Used power level is the maximum supported by the MS.

[editorial note : since not all MSs support such high power levels, it might be better to choose e

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell A). Used power level is the maximum supported by the MS.

Test Procedure

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND with Power Command set to 8 on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B. The SS activates the SACCH, but does not send PHYSICAL INFORMATION (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

0	SS	-	The SS records the power level
				in the layer 1 header of the
				last SACCH message sent by the
				MS before step 1.
	1	SS -> MS	HANDOVER COMMAND	Channel description : non-hopping,
				full rate
				Power Command : 8. Synchronisation
				Indication : non synchronized.
	2	MS -> SS	HANDOVER ACCESS	Several messages are sent, all
				with correct Handover References.
	3	MS -> SS	HANDOVER FAILURE	Sent on old channel,
				RR cause value ="Abnormal release,
				unspecified", "Abnormal release,
				channel unacceptable", "Abnormal
				release, timer expired" or
				"Abnormal release, no activity on
				the radio path".
				Shall be sent
				within 3 seconds from the trans-
				mission of HANDOVER COMMAND.
	4	SS	-	The SS checks that the power
				level reported in the layer 1
				header of the SACCH message
				that is sent in the first
				SACCH multiframe following the
				SABM is the same as in step 0.

Specific Message Contents

None

#### 26.6.5.9	Handover / layer 1 failure

##### 26.6.5.9.1	Conformance requirements

The MS shalll return to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell. On the old channel the MS shall use the Power Level that it was previously using on that channel.

References

GSM 04.08 section 3.4.4.

##### 26.6.5.9.2	Test purpose

To verify the function of timer T3124 and the contents in the message HANDOVER FAILURE and in the layer 1 header on the SACCH.

##### 26.6.5.9.3	Method of test

Initial Conditions

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). Used power level is the maximum supported by the MS.

[editorial note : since not all MSs support such high power levels, it might be better to choose e

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F.

Support for state U10 of the Call Control protocol.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell A). Used power level is the maximum supported by the MS.

Test Procedure

The MS is in the active state (u10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B. With the exception of normal BCCH signalling, the SS does not transmit anything on cell B (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

0	SS	-	The SS records the power level
				in the layer 1 header of the
				last SACCH message sent by the
				MS before step 1.
	  1	SS -> MS	HANDOVER COMMAND	Channel description : non-hopping,
				full rate. Synchronisation
				Indication : non synchronized.
	  2	MS -> SS	HANDOVER ACCESS	Several messages are sent, all
				with correct Handover References.
	  3	MS -> SS	HANDOVER FAILURE	Sent on old channel,
				RR cause value ="Abnormal release,
				unspecified", "Abnormal release,
				channel unacceptable", "Abnormal
				release, timer expired" or
				"Abnormal release, no activity on
				the radio path".
				Shall be sent
				within 3 seconds from the trans-
				mission of HANDOVER COMMAND.
	4	SS	-	The SS checks that the power
				level reported in the layer 1
				header of the SACCH message
				that is sent in the first
				SACCH multiframe following the
				SABM is the same as in step 0.

Specific Message Contents

None

### 26.6.6	Test of frequency redefinition

The Frequency Redefinition procedure is used by the network to change the frequencies and hopping sequences of the allocated channels.

#### 26.6.6.1	Frequency redefinition

##### 26.6.6.1.1	Conformance requirements

An MS, after receiving a FREQUENCY REDEFINITION message, shall start using the new frequencies and hopping sequence in the correct time slot when the MS is allocated a dedicated channel.

The behaviour described in the test purpose is applied for each combination of the value T(k) (k = 1,2,3) and for each supported dedicated channel type.

References

GSM 04.08 sections 3.4.5, 9.1.13 and 10.5.2.13.

##### 26.6.6.1.2	Test purpose

To verify that the MS, after receiving a Frequency Redefinition message, starts using the new frequencies and hopping sequence at the time indicated in the message.

##### 26.6.6.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs. The cell allocation is set to CAPGSM(1) or CADCS(1), depending on the band of operation of the Mobile Station ( See PICS/PIXIT), before each execution of this test.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F and/or TCH/H.

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

Test parameters:

P-GSM900 :

caPGSM(1) is set to 64.

A random subset CAGSM(1) of the set {1,...,124} containing caGSM(1) elements is drawn.

An element B of the set CAGSM(1) is randomly drawn.

A random value caPGSM(2) in the range 20,...,63 is drawn.

A random subset CAGSM(2) of the set {1,...,124} with caGSM(2) elements and containing B is drawn.

A random value caGSM(3) in the range 4,...,19 is drawn.

A random subset CAGSM(3) of the set {1,...,124} with caGSM(3) elements and containing B is drawn.

For j = 1,2,3, values maGSM(j) in the range j,...,caGSM(j)-1 and values MAIOGSM(j) in the range 0,...,maGSM(j)-1 are randomly drawn.

Subsets MAGSM(j) of CAGSM(j) not containing B and having ma(j) elements are randomly drawn.

DCS1800 :

caDCS(1) is set to 64.

A random subset CADCS(1) of the set {700,...,812} containing caDCS(1) elements is drawn.

An element B of the set CADCS(1) is randomly drawn. CADCS(1) is then coded using the Variable Bit Map coding scheme.

A random value caDCS(2) in the range 17,...,63 is drawn.

A random subset CADCS(2) of the set {700,...,812} with caDCS(2) elements and containing B is drawn. CADCS(2) is then coded using the Variable Bit Map coding scheme.

A random value caDCS(3) in the range 4,...,16 is drawn.

A random subset CADCS(3) of the set {700,...,812} with caDCS(3) elements and containing B is drawn. CADCS(3) is then coded according to the specific message contents.

For j = 1,2,3, values maDCS(j) in the range j,...,caDCS(j)-1 and values MAIODCS(j) in the range 0,...,maDCS(j)-1 are randomly drawn.

Subsets MADCS(j) of CADCS(j) not containing B and having maDCS(j) elements are randomly drawn.

DCS1800 and GSM900

Let T(1) = 91, T(2) = 42000.

A random value T(3) in the range 92,...,29999 is drawn.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a dedicated channel (TCH/F, TCH/H or SDCCH, as defined by the execution counter R). Then the SS sends a FREQUENCY REDEFINITION message, which modifies the frequencies/hopping sequence to be used by the MS. The MS shall then, at the TDMA frame defined by the contents of the "Starting Time" information element , use the new frequencies/hopping sequence. (The value of T(2) ensures that the MS believes the Starting Time has passed and so the MS shall start transmitting immediately. The range for T(3) ensures that the MS has to wait until the designated frame before starting transmission on the new frequencies.)

The verification is performed at the RF burst level. The MS transmits the standard test signal C1 (Annex 5), and for the TCH case, the SS checks the received pattern with the expected pattern. For the SDCCH case the MS transmits fill frames, and the SS checks for each burst whether the burst is transmitted at the right frequency.

[editorial note : how is the MS made to transmit the C1 signal ? By means of loop-back ? ]

Maximum Duration of Test

3 * (number of supported channels * T(3) + 7)

Expected Sequence

This sequence is performed for every combination of execution counters K = 1,2,3 and R = 1,2,3:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	  2	MS -> SS	CHANNEL REQUEST	
	  3	SS -> MS	IMM ASSIGNMENT	
	  4	MS -> SS	PAGING RESPONSE
	  5	--------	--------	The SS checks that the MS is
				transmitting on the correct
				frequencies without delay.
	  6	SS -> MS	FREQUENCY REDEF	See description 1 below.
	  7	--------	--------	The SS checks that the MS is
				transmitting on the correct
				frequencies and that the transmissions
				started in the correct frame.
	  8	SS -> MS	FREQUENCY REDEF	See description 2 below
	  9	--------	--------	The SS checks that the MS is
				transmitting on the correct
				frequencies and that the transmissions
				started in the correct frame.
	 10	SS -> MS	CHANNEL RELEASE

For	K=1, R = 1,2,3		T(K) = T(1);

K=2, R = 1,2,3		T(K) = T(2);

K=3, R = 1,2,3		T(K) = T(3).

Specific Message Contents

GSM900 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

L2 pseudo length					value dependent on the length of the

Mobile Allocation and thus on the number

of channels in CAPGSM(1).

Channel Description

Channel type				SDCCH/8 arbitrary offset, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs arbitrary offset, for R=3

Timeslot number				arbitrarily selected by

TSC						arbitrarily selected

Hopping channel				RF hopping channel

MAIO						MAIOPGSM(1)

HSN						0

Request reference					corresponds to the Channel Request

Timing advance					30 bit periods

Mobile Allocation					corresponds to set MAPGSM(1)

Starting Time					not present

IA rest octets					all bits are set to spare

FREQUENCY REDEFINITION (Description 1)

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIOPGSM(2)

HSN						0

Mobile Allocation					corresponds to set MAPGSM(2)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(K)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					corresponds to set CAPGSM(2) with 'Format ID'

set to "bit map 0"

FREQUENCY REDEFINITION (Description 2)

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs, for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIOPGSM(3)

HSN						0

Mobile Allocation					corresponds to set MAPGSM(3)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(K)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					corresponds to set CAPGSM(3) with 'Format ID'

set to "bit map 0"

DCS1800 :

IMMEDIATE ASSIGNMENT

Information Element				value/remark

L2 pseudo length					value dependent on the length of the

Mobile Allocation and thus on the number

of channels in CADCS(1).

Channel Description

Channel type				SDCCH/8 arbitrary offset, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs arbitrary offset, for R=3

Timeslot number				arbitrarily selected

TSC						arbitrarily selected

Hopping channel				RF hopping channel

MAIO						MAIODCS(1)

HSN						0

Request reference					corresponds to the Channel Request

Timing advance					30 bit periods

Mobile Allocation					corresponds to set MADCS(1)

Starting Time					not present

IA rest octets					all bits are set to spare

FREQUENCY REDEFINITION (Description 1)

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIODCS(2)

HSN						0

Mobile Allocation					corresponds to set MADCS(2)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(K)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					K = 1 - corresponds to set CADCS(2) with

'Format ID' set to " Variable Bit Map"

K = 2- corresponds to set CADCS(2) with

'Format ID' set to " Variable Bit Map "

K = 3 - corresponds to set CADCS(2) with

'Format ID' set to " Variable Bit Map "

FREQUENCY REDEFINITION (Description 2)

Information Element				value/remark

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs, for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIODCS(3)

HSN						0

Mobile Allocation					corresponds to set MADCS(3)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(K)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					K = 1 - corresponds to set CADCS(3) with

'Format ID' set to "Range 1024"

K = 2- corresponds to set CADCS(3) with

'Format ID' set to "Range 256"

K = 3 - corresponds to set CADCS(3) with

'Format ID' set to "Range 512"

### 26.6.7	Test of the channel mode modify procedure

The channel mode modify procedure allows the network to request the MS to change the channel mode for one channel. If the mobile station doesn't correctly respond to the CHANNEL MODE MODIFY message (with a positive acknowledgement if the new channel mode is supported, with a negative acknowledgement if the new channel mode is not supported), the network may try to repeat the procedure, release the connection, or continue to wait for the acknowledgement (the maximum time resulting from layer two re-transmissions and MS reaction time being around 5 seconds).

#### 26.6.7.1	Test of the channel mode modify procedure / full rate

This test is only applicable to an MS supporting TCH/F.

##### 26.6.7.1.1	Conformance requirement

When the MS has received the CHANNEL MODE MODIFY message, the mobile station changes the mode for the indicated channel and then replies by a CHANNEL MODE MODIFY ACKNOWLEDGE message indicating the new channel mode.

If the mobile station does not support the indicated mode, it shall retain the old mode and return the associated channel mode information in the CHANNEL MODE MODIFY ACKNOWLEDGE message.

References

TS GSM 04.08, subclauses 3.4.6.2 and 3.4.6.3

##### 26.6.7.1.2	Test purpose

To verify that the MS, in an RR connected state, acknowledges a CHANNEL MODE MODIFY message by sending a CHANNEL MODE MODIFY ACKNOWLEDGEMENT message specifying and switching to the correct mode.

-	the new mode if that mode is supported

-	the old mode if the new mode is not supported.

This shall be verified for the channel modes

-	signalling only

-	speech full rate

-	data 9.6 Kb/s

-	data 4.8 Kb/s full rate

-	data 2.4 Kb/s full rate.

##### 26.6.7.1.3	Method of test

Initial Conditions

System Simulator:

1 cells, default parameters.

Mobile Station:

The MS is "idle updated", with TMSI allocated.

Related PICS/PIXIT Statements

-	Type of MS (P-GSM900 or EGSM or DCS1800).

-	The MS supports TCH/F (Y/N)

-	Bearer Capabilities supported by the MS

-	Channel modes supported by the MS

*	MS supports speech full rate (p1 = Y/N)

*	MS supports data 9.6 Kb/s (p2 = Y/N)

*	MS supports data 4.8 Kb/s full rate (p3 = Y/N)

*	MS supports data 2.4 Kb/s full rate (p4 = Y/N)

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test procedure

A Mobile Terminated call is initiated , however following the Channel Request received from the Mobile Station, the SS sends an Immediate Assignment to the MS commanding it to go to a TCH/F. This sets the Channel Mode automatically to "Signalling Only".

The SS then sends a series of CHANNEL MODE MODIFY messages to the MS. Each time it is checked that the MS responds with a CHANNEL MODE MODIFY ACKNOWLEDGE message specifying

-	the channel mode that has been specified in the CHANNEL MODE MODIFY message, if the MS supports that mode (this mode then becomes the "channel mode in use")

-	the channel mode that was in use when the CHANNEL MODE MODIFY message has been received, if the MS does not support the channel mode specified in the CHANNEL MODE MODIFY message.

Maximum Duration of Test

30 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS->MS	PAGING REQUEST TYPE 1	Sent on correct paging subchannel
	2	MS->SS	CHANNEL REQUEST	Establishment cause indicates "answer to
				paging"
	3	SS->MS	IMMEDIATE ASSIGNMENT	Assignment to a non hopping TCH/F
	4	SS->MS	CHANNEL MODE MODIFY	
 	5	MS->SS	CHAN MODE MOD ACK	
 	6	SS->MS	CHANNEL MODE MODIFY	
 	7	MS->SS	CHAN MODE MOD ACK	
	8	SS->MS	CHANNEL MODE MODIFY	
 	9	MS->SS	CHAN MODE MOD ACK	
 	10	SS->MS	CHANNEL MODE MODIFY	
	11	MS->SS	CHAN MODE MOD ACK	
 	12	SS->MS	CHANNEL MODE MODIFY	
	13	MS->SS	CHAN MODE MOD ACK	
	14	SS->MS	CHANNEL RELEASE

Specific Message Contents

CHANNEL MODE MODIFY

Information Element				value/remark

Channel description				describes the already assigned dedicated

channel

Channel mode

Mode			in step 4:		speech full rate

in step 6:		data 9.6 Kb/s

in step 8:		data 4.8 Kb/s full rate

in step 10:	data 2.4 Kb/s full rate

in step 12	signalling only

CHANNEL MODE MODIFY ACKNOWLEDGE

Channel mode

Mode			in step 2:		signalling only

in step 4:		if p2 = Y: data 9.6 Kb/s

if p2 = N: signalling only

in step 6:		if p3 = Y: data 4.8 Kb/s full rate

if p3 = N: same as in step 4

in step 8:		if p4 = Y: data 2.4 Kb/s full rate

if p4 = N: same as in step 6

in step 10:	if p1 = Y: speech full rate

if p1 = N: same as in step 8

#### 26.6.7.2	Test of the channel mode modify procedure / half rate

This test is only applicable to an MS supporting TCH/H.

##### 26.6.7.2.1	Conformance requirement

When the MS has received the CHANNEL MODE MODIFY message, the mobile station changes the mode for the indicated channel and then replies by a CHANNEL MODE MODIFY ACKNOWLEDGE message indicating the new channel mode.

If the mobile station does not support the indicated mode, it shall retain the old mode and return the associated channel mode information in the CHANNEL MODE MODIFY ACKNOWLEDGE message.

References

TS GSM 04.08, subclauses 3.4.6.2 and 3.4.6.3

##### 26.6.7.2.2	Test purpose

To verify that the MS, in an RR connected state, acknowledges a CHANNEL MODE MODIFY message by sending a CHANNEL MODE MODIFY ACKNOWLEDGEMENT message specifying and switches to the correct mode.

-	the new mode if that mode is supported

-	the old mode if the new mode is not supported.

This shall be verified for the channel modes

-	signalling only

-	speech half rate

-	data 4.8 Kb/s half rate

-	data 2.4 Kb/s half rate

##### 26.6.7.2.3	Method of test

Initial Conditions

System Simulator:

1 cells, default parameters.

Mobile Station:

The MS is "idle updated", with TMSI allocated

Related PICS/PIXIT Statements

-	Type of MS (P-GSM900 or EGSM or DCS1800).

-	The MS supports TCH/H (Y/N)

-	Bearer Capabilities supported by the MS

-	Channel modes supported by the MS

*	MS supports speech half rate (q1 = Y/N)

*	MS supports data 4.8 Kb/s half rate (q2 = Y/N)

*	MS supports data 2.4 Kb/s half rate (q3 = Y/N)

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test procedure

A Mobile Terminated call is initiated , however following the Channel Request received from the Mobile Station, the SS sends an Immediate Assignment to the MS commanding it to go to a TCH/H. This sets the Channel Mode automatically to "Signalling Only".

The SS then sends a series of CHANNEL MODE MODIFY messages to the MS. Each time it is checked that the MS  responds with a CHANNEL MODE MODIFY ACKNOWLEDGE message specifying

-	the channel mode that has been specified in the CHANNEL MODE MODIFY message, if the MS supports that mode (this mode then becomes the "channel mode in use")

-	the channel mode that was in use when the CHANNEL MODE MODIFY message has been received, if the MS does not support the channel mode specified in the CHANNEL MODE MODIFY message.

Maximum Duration of Test

30 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS->MS	PAGING REQUEST TYPE 1	Sent on the correct paging subchannel
	2	MS->SS	CHANNEL REQUEST	Establishment cause indicates "answer to
				paging"
	3	SS->MS	IMMEDIATE ASSIGNMENT	Assignment to a non hopping TCH/H
	4	SS->MS	CHANNEL MODE MODIFY	
 	5	MS->SS	CHAN MODE MOD ACK	
 	6	SS->MS	CHANNEL MODE MODIFY	
 	7	MS->SS	CHAN MODE MOD ACK	
	8	SS->MS	CHANNEL MODE MODIFY	
 	9	MS->SS	CHAN MODE MOD ACK	
 	10	SS->MS	CHANNEL MODE MODIFY	
 	11	MS->SS	CHAN MODE MOD ACK	
 	12	SS->MS	CHANNEL RELEASE

Specific Message Contents

CHANNEL MODE MODIFY

Information Element				value/remark

Channel description				describes the already assigned

dedicated channel

Channel mode

Mode			in step 4: 	signalling only

in step 6: 	data 4.8 Kb/s half rate

in step 8: 	data 2.4 Kb/s half rate

in step 10: 	speech half rate

CHANNEL MODE MODIFY ACKNOWLEDGE

Information Element				value/remark

Channel mode

Mode			in step 5:		if q1 = Y: Speech half rate

if q1 = N: signalling only

in step 7:		if q2 = Y: data 4.8 Kb/s half rate

if q2 = N: same as in step 5

in step 9:		if q3 = Y: data 2.4 Kb/s half rate

if q3 = N: same as in step 7

in step 11:	if q1 = Y: signalling only

if q1 = N: same as in step 6.

### 26.6.8	Test of ciphering mode setting

The Ciphering Mode Setting Procedure can be used by the network to trigger the start and stop of stream ciphering.

The SS shall start and synchronize ciphering and deciphering according to GSM 03.20. The bitstream shall be generated by algorithm A5 (A5/1 or A5/2 as defined by the test case) using the encryption key Kc.

#### 26.6.8.1	Ciphering mode / start ciphering

##### 26.6.8.1.1	Conformance requirements

1.	When the MS receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS starts ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field;

-	the MS responds with a CIPHERING MODE COMPLETE message in ciphered mode;

-	the ciphering uses the cipher key determined during the authentication procedure.

2.	The MS responds to the AUTHENTICATION REQUEST message with an AUTHENTICATION RESPONSE message and continues to use the ciphering key obtained from the previous authentication procedure.

References

GSM 04.08 section 3.4.7.

##### 26.6.8.1.2	Test purpose

To verify that the MS starts ciphering when it receives a CIPHERING MODE COMMAND message with Cipher Mode Setting = "Start Ciphering". To verify that it continues to use the old cipher key after it receives an AUTHENTICATION REQUEST whilst in ciphered mode.

##### 26.6.8.1.3	Method of test

Initial Conditions

System Simulator:

1 cells, Radio-Link-Timeout set to 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Support for state U10 of the Call Control protocol.

Supported encryption algorithms: A5/1 and/or A5/2

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends an AUTHENTICATION REQUEST and the MS shall answer with AUTHENTICATION RESPONSE. Then the SS sends a CIPHERING MODE COMMAND, ordering the MS to start ciphering with an algorithm supported by the MS. After transmission of this command the SS starts deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in ciphered mode using the cipher key determined during the authentication procedure, and continue to establish the call with a SETUP message. After reception of the CIPHERING MODE COMPLETE the SS starts enciphering.

The SS then sends another AUTHENTICATION REQUEST and the MS shall respond with an AUTHENTICATION RESPONSE. The MS shall continue to use the old cipher key.

Finally the SS sends a CHANNEL RELEASE to end the test.

[editorial note : In the old 11.10 it was required that the MS should store the new key, but that was not tested. Should it be tested in phase 2 ? ]

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, K=1, 2.

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating Call"
				NECI not set to 1
	  2	SS -> MS	IMM ASSIGNMENT
	  3	MS -> SS	CM SERVICE REQUEST
	  4	SS -> MS	AUTHENT REQUEST
	  5	MS -> SS	AUTHENT RESPONSE
	  6	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "Start
				Ciphering; cipher with algorithm A5/K".
				The SS starts deciphering.
	  7	MS -> SS	CIPH MODE COMPLETE	Sent in ciphered mode using
				the cipher key determined in between
				steps 4&5. The SS start enciphering
	  8	MS -> SS	SETUP
	  9	SS -> MS	AUTHENT REQUEST	Determines a new cipher key
	 10	MS -> SS	AUTHENT RESPONSE	Sent in ciphered mode using the
				cipher key determined in between steps
				4&5.
	 11	SS -> MS	CHANNEL RELEASE

Specific Message Contents

CIPHERING MODE COMMAND

For	k = 1, A5/k = A5/1

For	k = 2, A5/k = A5/2

#### 26.6.8.2	Ciphering mode / no ciphering

##### 26.6.8.2.1	Conformance requirements

When the MS receives a CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "no ciphering" the MS shall respond in non ciphered mode with a CIPHERING MODE COMPLETE message.

When the CIPHERING MODE COMMAND with Ciphering Mode Setting information element set to "no ciphering" is received as a response to a CM SERVICE REQUEST, the MS shall continue the establishment of the CM service.

References

GSM 04.08 section 3.4.7.

##### 26.6.8.2.2	Test purpose

To verify that the MS does not start ciphering when it receives a CIPHERING MODE COMMAND message with Cipher Mode Setting = "No Ciphering".

##### 26.6.8.2.3	Method of test

Initial Conditions

System Simulator:

1 cells, Radio-Link-Timeout set to 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends an AUTHENTICATION REQUEST and the MS shall answer with an AUTHENTICATION RESPONSE. Then the SS sends a CIPHERING MODE COMMAND, ordering the MS not to start ciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in non-ciphered mode and continue to establish the call with a SETUP message.

Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call;
				NECI not equal to 1
	  2	SS -> MS	IMM ASSIGNMENT
	  3	MS -> SS	CM SERVICE REQUEST
	  4	SS -> MS	AUTHENT REQUEST
	  5	MS -> SS	AUTHENT RESPONSE
	  6	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering"
	  7	MS -> SS	CIPH MODE COMPLETE	Sent in non-ciphered mode
	  8	MS -> SS	SETUP
	 11	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

#### 26.6.8.3	Ciphering mode / old cipher key

##### 26.6.8.3.1	Conformance requirements

When the MS receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS starts ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field. Also;

-	the MS responds with a CIPHERING MODE COMPLETE message in the correct ciphered mode;

-	the ciphering shall use the previously stored cipher key

-	in the case of a mobile originating speech caal, the MS shall send a SETUP message after the completion of the ciphering procedure.

References

GSM 04.08 section 3.4.7.

##### 26.6.8.3.2	Test purpose

To verify that the MS uses the stored cipher key when it receives a CIPHERING MODE COMMAND without a preceding authentication procedure.

##### 26.6.8.3.3	Method of test

Initial Conditions

System Simulator:

1 cells, Radio-Link-Timeout = 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and a known cipher key stored.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Support for state U10 of the Call Control protocol.

Supported encyrption algorithms: A5/1 and/or A5/2.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends a CIPHERING MODE COMMAND, ordering the MS to start ciphering with a supported algorithm. After transmission of this command the SS starts deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in the commanded ciphered mode using the stored cipher key and continue to establish the call with a SETUP message. After reception of the CIPHERING MODE COMPLETE the SS starts enciphering.

Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating Call:
				NECI not equal to 1
	  2	SS -> MS	IMM ASSIGNMENT
	  3	MS -> SS	CM SERVICE REQUEST
	  4	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "Start
				Ciphering", algorithm arbitrarily selected
				from those supported by the MS. The SS starts
				deciphering.
	  5	MS -> SS	CIPH MODE COMPLETE	Sent in commanded ciphered mode with
				the stored cipher key. The SS
				starts enciphering.
	  6	MS -> SS	SETUP
	  7	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

#### 26.6.8.4	Ciphering mode / change of mode, algorithm and key

Networks can be implemented that do not have the same ciphering algorithms on all base stations. In such networks changes of algorithms and ciphering mode may occur and calls will fail if MSs incorrectly handle commands or use an incorrect cipher key.

##### 26.6.8.4.1	Conformance requirements

1	When the MS in the "not ciphered" mode, receives a CIPHERING MODE COMMAND message with the Ciphering Mode Setting information element set to "start ciphering", the MS shall load the cipher key stored in the SIM into the ME, use this key to start ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field and, respond with a CIPHERING MODE COMPLETE message.

2	If the last timeslot of the message block containing a CIPHERING MODE COMMAND message occurs at time T, then the MS shall be ready to transmit the CIPHERING MODE COMPLETE message before T+500ms.

3	When the MS receives an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the assignment, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key.

4	When the MS receives a HANDOVER COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the handover, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key

5	When the MS in the "ciphered" mode receives a CIPHERING MODE COMMAND message with Cipher Mode Setting IE set to "no ciphering", the MS shall load the cipher key stored in the SIM into the ME, load the cipher key stored in the SIM into the ME, stop ciphering and deciphering and, respond with a CIPHERING MODE COMPLETE message.

6	When the MS receives an AUTHENTICATION REQUEST message, it shall process the challenge information and send back an AUTHENTICATION RESPONSE message to the network. The new ciphering key calculated from the challenge information shall overwrite the previous one and be stored on the SIM before the AUTHENTICATION RESPONSE message is transmitted. The ciphering key stored in the SIM shall be loaded in to the ME when any valid CIPHERING MODE COMMAND is received.

7	When the MS in the not ciphered mode receives a CIPHERING MODE COMMAND message with Cipher Mode Setting IE set to "no ciphering", the MS shall load the cipher key stored in the SIM into the ME, stop ciphering and deciphering and, respond with a CIPHERING MODE COMPLETE message.

8	If a handover fails then the operational parameters used when returning to the old channel are those applied before the HANDOVER COMMAND message was received.

9	If an assignment fails then the operational parameters used when returning to the old channel are those applied before the ASSIGNMENT COMMAND message was received.

References

Conformance requirement 1: GSM 04.08 sections 3.4.7.2 and 4.3.2.2.

Conformance requirement 2: GSM 04.13 section 5.2.7.

Conformance requirement 3: GSM 04.08 section 3.4.3.1.

Conformance requirement 4: GSM 04.08 section 3.4.4.1.

Conformance requirement 5: GSM 04.08 sections 3.4.7.2 and 4.3.2.2.

Conformance requirement 6: GSM 04.08 section 4.3.2.2.

Conformance requirement 7: GSM 04.08 section 3.4.7.2.

Conformance requirement 8: GSM 04.08 section 3.4.4.4.

Conformance requirement 9: GSM 04.08 section 3.4.3.3.

##### 26.6.8.4.2	Test purpose

1	To verify that when the MS is in the "not ciphered" mode and receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS uses the cipher key stored in the SIM to start ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field and that the MS responds with a CIPHERING MODE COMPLETE message.

2	To verify that the MS is ready to transmit the CIPHERING MODE COMPLETE message before 500ms after the end of the CIPHERING MODE COMMAND message.

3	To verify that when the MS receives an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the assignment, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key.

4	To verify that when the MS receives a HANDOVER COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the handover, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key

5	To verify that when the MS is in the "ciphered" mode and receives the CIPHERING MODE COMMAND message with Cipher Mode Setting IE set to "no ciphering", the MS loads the cipher key stored in the SIM into the ME, stops ciphering and deciphering and, responds with a CIPHERING MODE COMPLETE message.

6	To verify that the MS responds to an AUTHENTICATION REQUEST message with an AUTHENTICATION RESPONSE message and continues to use the cipher key obtained from the previous authentication procedure.

7	To verify that when the MS is in the "not ciphered" mode and receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "no ciphering", the does not start ciphering or deciphering, but does respond with a CIPHERING MODE COMPLETE message.

8	To verify that when the MS receives a HANDOVER COMMAND message and the handover fails, the MS sends a HANDOVER FAILURE message on the old channel using the old ciphering mode and (if ciphered) the old algorithm and old key.

9	To verify that when the MS receives an ASSIGNMENT COMMAND message and the assignment fails, the MS sends an ASSIGNMENT FAILURE message on the old channel using the old ciphering mode and (if ciphered) the old algorithm and old key.

##### 26.6.8.4.3	Method of test

Initial Conditions

System Simulator:

1 cell with default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and a known cipher key, K, stored in the SIM.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Supported ciphering algorithms: A5/1 and/or A5/2.

Power class of mobile station

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged. The MS shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a PAGING RESPONSE message.

The SS sends a CIPHERING MODE COMMAND, ordering the MS to start ciphering with a supported algorithm. After transmission of this command the SS starts deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in the commanded ciphered mode using the cipher key K. After reception of the CIPHERING MODE COMPLETE the SS starts enciphering. The MS shall be ready to transmit the CIPHERING MODE COMPLETE message before 500 ms after the end of the CIPHERING MODE COMMAND message.

The term 'ready to transmit' is defined in GSM 04.13.

The SS sends an AUTHENTICATION REQUEST message to the MS. Cipher key L is calculated. The MS shall send an AUTHENTICATION RESPONSE message to the SS.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "no ciphering". The MS shall transmit the HANDOVER COMPLETE on the commanded channel in non ciphered mode.

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "start ciphering". The MS shall start transmitting on the commanded channel using the commanded algorithm and cipher key K. The MS shall transmit the ASSIGNMENT COMPLETE message.

The SS sends a CIPHERING MODE COMMAND, ordering the MS to stop ciphering. After transmission of this command the SS stops deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in non ciphered mode. After reception of the CIPHERING MODE COMPLETE the SS stops enciphering.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "start ciphering". The MS shall transmit the HANDOVER COMPLETE on the commanded channel in ciphered mode using cipher key L and command algorithm.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "start ciphering" and the algorithm identifier indicating the algorithm currently in use. The MS shall transmit the HANDOVER COMPLETE on the commanded channel in ciphered mode using the same algorithm as before the handover.

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "no ciphering". The MS shall start transmitting on the commanded channel in non-ciphered mode. The MS shall transmit the ASSIGNMENT COMPLETE message.

The SS sends a CIPHERING MODE COMMAND, containing a Cipher Mode Setting IE set to "no ciphering". The MS shall respond with a CIPHERING MODE COMPLETE message.

The SS sends an AUTHENTICATION REQUEST message to the MS. Cipher key M is calculated. The MS shall send an AUTHENTICATION RESPONSE message to the SS.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "start ciphering". The MS shall transmit the HANDOVER COMPLETE on the commanded channel using the commanded algorithm and cipher key L.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "no ciphering". The SS does not activate the commanded channel. The MS's transmissions on the new channel need not be monitored. The MS shall transmit the HANDOVER FAILURE message on the 'old' channel using the 'old' algorithm and cipher key L and commanded algorithm.

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "start ciphering". The SS does not activate the commanded channel. The MS's transmissions on the new channel need not be monitored. The MS shall transmit the ASSIGNMENT FAILURE message on the 'old' channel using the 'old' algorithm and cipher key L.

If the MS only supports one ciphering algorithm then the SS sends a CHANNEL RELEASE message.

If the MS supports more than one ciphering algorithm then the following steps are performed:

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "start ciphering" and the Algorithm Identifier indicating a different supported algorithm to the one in use. The MS shall start transmitting on the commanded channel using the commanded algorithm. The MS shall transmit the ASSIGNMENT COMPLETE message.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) containing a Cipher Mode Setting IE set to "start ciphering" and the Algorithm Identifier indicating a different supported algorithm to the one in use. The MS shall transmit the HANDOVER COMPLETE on the commanded channel using the commanded algorithm.

The SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test

3 minutes.

Expected Sequence

For MSs that only support one ciphering algorithm, the SS shall use step 57A. For MSs that support more than one ciphering algorithm, the SS shall use step 57B and the subsequent steps.

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMM ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "Start Ciphering",
				algorithm arbitrarily selected from those
				supported by the MS. The SS starts 
				deciphering with the selected algorithm.
	  6	MS -> SS	CIPH MODE COMPLETE	Sent in ciphered mode using key 'K', the
				stored cipher key, and the commanded 
				algorithm. This message shall be ready to
				be transmitted before 500 ms after the
				completion of step 5.
  	7	SS	-	The SS starts enciphering using key 'K'.
	  8	SS -> MS	AUTHENT REQUEST	Contains a new Ciphering Key Sequence
				Number which is associated with the new
				cipher key, 'L'.
	  9	MS -> SS	AUTHENT RESPONSE
	 10	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"No Ciphering".
 	11	SS		The SS activates the new channel without
				ciphering.
 	12	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are
 	13	MS -> SS	HANDOVER ACCESS	sent on the new channel in non ciphered
 	14	MS -> SS	HANDOVER ACCESS	mode.
 	15	MS -> SS	HANDOVER ACCESS
 	16	MS -> SS	HANDOVER COMPLETE	Sent in non ciphered mode on the new
				channel.
 	17	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering", with algorithm arbitrarily
				selected from those supported by the MS.
 	18	SS		The SS activates the new channel with
				enciphering and deciphering enabled and using
				cipher key 'K'.
 	19	MS -> SS	ASSIGNMENT COMPLETE	Sent on the new channel in ciphered mode
				using key 'K' and the commanded algorithm.
	 20	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering". The
				SS starts receiving in non ciphered mode.
 	21	MS -> SS	CIPH MODE COMPLETE	Sent in non ciphered mode. This message shall
				be ready to be transmitted before 500 ms after
				the completion of step 20.
 	22	SS	-	The SS starts transmitting in non ciphered
				mode.
 	23	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering", with algorithm 'X' arbitrarily
				selected from those supported by the MS.
 	24	SS		The SS activates the new channel with
				enciphering and deciphering enabled and using
				cipher key 'L'.
 	25	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are
 	26	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered
 	27	MS -> SS	HANDOVER ACCESS	mode.
 	28	MS -> SS	HANDOVER ACCESS
 	29	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode
				using key 'L' and algorithm 'X'.
 	30	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering", with algorithm identifier
				set to 'X'.

31	SS		The SS activates the new channel with
				enciphering and deciphering enabled.
 	32	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are
 	33	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered
 	34	MS -> SS	HANDOVER ACCESS	mode.
 	35	MS -> SS	HANDOVER ACCESS
 	36	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode
				using algorithm 'X'.
 	37	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to
				"No Ciphering".
 	38	SS		The SS activates the new channel without
				ciphering.
 	39	MS -> SS	ASSIGNMENT COMPLETE	Sent in non-ciphered mode on the new
				channel.
	 40	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering". 
 	41	MS -> SS	CIPH MODE COMPLETE	Sent in non ciphered mode. This message shall
				be ready to be transmitted before 500 ms after
				the completion of step 40.
	 42	SS -> MS	AUTHENT REQUEST	Contains a new Ciphering Key Sequence
				Number which is associated with the new
				cipher key, 'M'.
	 43	MS -> SS	AUTHENT RESPONSE
 	44	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering", with algorithm 'Y' arbitrarily
				selected from those supported by the MS.
 	45	SS		The SS activates the new channel with
				enciphering and deciphering enabled and using
				cipher key 'L'.
 	46	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are
 	47	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered
 	48	MS -> SS	HANDOVER ACCESS	mode.
 	49	MS -> SS	HANDOVER ACCESS
 	50	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode
				using key 'L'
 	51	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"No Ciphering".
 	52	SS, MS		The SS does not activate the new channel
				The MS's transmissions on the new channel
				need not be monitored.
 	53	MS -> SS	HANDOVER FAILURE	sent on old channel using algorithm 'Y' and
				key 'L'.
 	54	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to
				"No Ciphering".
 	55	SS, MS		The SS does not activate the new channel
				The MS's transmissions on the new channel
				need not be monitored.
 	56	MS -> SS	ASSIGNMENT FAILURE	sent on old channel using algorithm 'Y' and
				key 'L'.
	57A	SS -> MS	CHANNEL RELEASE
	57B	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering", with algorithm 'Z' arbitrarily
				selected from those supported by the MS but 
				different to algorithm 'Y'.
	58B	SS		The SS activates the new channel with
				enciphering and deciphering enabled.
	59B	MS -> SS	ASSIGNMENT COMPLETE	Sent on the new channel in ciphered mode
				using key 'L' and algorithm 'Z'.
	60B	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to
				"Start Ciphering" and the algorithm identifier
				set to 'Y'.

61B	SS		The SS activates the new channel with
				enciphering and deciphering enabled.
	62B	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are
	63B	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered
	64B	MS -> SS	HANDOVER ACCESS	mode.
	65B	MS -> SS	HANDOVER ACCESS
	66B	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode
				using key 'L' and algorithm 'Y'.
	67B	SS -> MS	CHANNEL RELEASE

Specific Message Contents

ASSIGNMENT COMMAND

Information Element			value/remark

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101110

Channel Description

-	Channel Type			SDCCH/4

-		TDMA offset		Chosen arbitrarily, but different to the one in use.

-	Timeslot Number		Timeslot zero

-	Training Sequence Code		5 (same as the BCC)

-	Hopping			Single RF channel

-	ARFCN			Channel number 30

Power Command

-	Power level			Chosen arbitrarily but

within the range supported by the MS.

Cipher Mode Setting			As specified above.

All other information elements:		Not present

HANDOVER COMMAND

Information Element			value/remark

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101011

Cell Description

-	Network Colour Code		1

-	Base station Colour Code	5

-	BCCH Carrier Number		Channel Number 30

Channel Description

-	Channel Type			SDCCH/4

-		TDMA offset		Chosen arbitrarily, but different to the one in use.

-	Timeslot Number		Timeslot zero

-	Training Sequence Code		5 (same as the BCC)

-	Hopping			Single RF channel

-	ARFCN			Channel Number 30

Handover Reference

-	Handover Reference Value	Chosen arbitrarily .

Power Command

-	Power level			Chosen arbitrarily, but

within the range supported by the MS

Synchronisation Indication

-	Synchronisation indication	synchronized

-	Report observed time difference	not included

-	Normal cell indication		out of range ignored

Cipher Mode setting			As specified above.

All other information elements:		Not present

CIPHER MODE COMMAND

Information Element			value/remark

As default message contents, except :

Cipher Mode Setting

- Algorithm Identifier			As specified above

- Cipher Mode Set			As specified above

DCS1800 :

ASSIGNMENT COMMAND

Information Element			value/remark

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101110

Channel

-	Channel Type			SDCCH/4

-		TDMA offset		Chosen arbitrarily, but different to the one in

use.

-	Timeslot Number		Timeslot zero

-	Training Sequence Code		5 (same as the BCC)

-	Hopping			Single RF channel

-	ARFCN			Channel number 650

Power Command

-	Power level			Chosen arbitrarily, but within the range

supported by the MS.

Cipher Mode Setting			As specified above.

All other information elements:		Not present

HANDOVER COMMAND

Information Element			value/remark

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101011

Cell Description

-	Network Colour Code		1

-	Base station Colour Code	5

-	BCCH Carrier Number		Channel Number 650

Channel Description

-	Channel Type			SDCCH/4

-		TDMA offset			Chosen arbitrarily, but different to the one in

use.

-	Timeslot Number		Timeslot zero

-	Training Sequence Code		5 (same as the BCC)

-	Hopping			Single RF channel

-	ARFCN			Channel Number 650

Handover Reference

-	Handover Reference Value	Chosen arbitrarily.

Power Command

-	Power level			Chosen arbitrarily, but within the range

supported by the MS.

Synchronisation Indication

-	Synchronisation indication	synchronized

-	Report observed time difference	not included

-	Normal cell indication		out of range ignored

Cipher Mode setting			As specified above.

All other information elements:		Not present

#### 26.6.8.5	Ciphering mode / IMEISV request

If the MS does not supply the IMEISV when requested, the network will not know whether or not the MS is type approved, ie. whether or not it has passed any tests.

If the MS supplies its IMEISV when not requested, this may cause calls to systematically fail.

##### 26.6.8.5.1	Conformance requirements

1.	When the MS receives the CIPHERING MODE COMMAND message with Cipher Response bit set to 'IMEISV shall be included', the MS shall include the IMEISV in the Mobile Identity IE in the CIPHERING MODE COMPLETE message.

2.	When the MS receives the CIPHERING MODE COMMAND message with Cipher Response bit set to 'IMEISV shall not be included', the MS shall not include the Mobile Identity IE in the CIPHERING MODE COMPLETE message.

References

Conformance requirement 1: GSM 04.08 sections 3.4.7.2 and 9.1.10.1.

Conformance requirement 2: GSM 04.08 section 9.1.10.1.

##### 26.6.8.5.2	Test purpose

To verify that the MS supplies its IMEISV in the CIPHERING MODE COMPLETE message when it receives a CIPHERING MODE COMMAND message with a Cipher Response bit set to 'IMEISV shall be included'.

To verify that the MS does not supply any Mobile Identity IE in the CIPHERING MODE COMPLETE message when it receives a CIPHERING MODE COMMAND message with a Cipher Response bit set to 'IMEISV shall not be included'.

##### 26.6.8.5.3	Method of test

Initial Conditions

System Simulator:

1 cell.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

IMEISV of the MS

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a PAGING RESPONSE. Then the SS sends a CIPHERING MODE COMMAND indicating 'No ciphering' and with the Cipher Response bit set to 'IMEISV shall not be included'. The MS shall respond with a CIPHERING MODE COMPLETE message that does not include the Mobile Identity IE.

Then the SS sends a CIPHERING MODE COMMAND indicating 'No ciphering' and with the Cipher Response bit set to 'IMEISV shall be included'. The MS shall respond with a CIPHERING MODE COMPLETE message that carries the IMEISV in the Mobile Identity IE.

Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test

10 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering".
				Cipher Response = "IMEISV shall not be
				included".
	6	MS -> SS	CIPH MODE COMPLETE	Shall not include Mobile Identity IE.
	7	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering".
				Cipher Response = "IMEISV shall be included".
	8	MS -> SS	CIPH MODE COMPLETE	Shall include one Mobile Identity IE carrying
				the MS's IMEISV.
	9	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None.

### 26.6.9	Test of additional assignment

The Additional Assignment procedure is used to change an MS's channel configuration from Lm + ACCH to Lm + Lm + ACCH. It is therefore only relevant to those mobiles which perform such an operation.

No tests are specified at the moment.

### 26.6.10	Test of partial release

The Partial Release procedure is used to change an MS's channel configuration from Lm + Lm + ACCH to Lm + ACCH. It is therefore only relevant to those mobiles which perform such an operation.

No tests are specified at the moment.

### 26.6.11	Test of classmark

#### 26.6.11.1	Classmark change

This procedure allows the MS to indicate to the network that a change in the classmark (e.g. due to addition of power amplification) has taken place.

##### 26.6.11.1.1	Conformance requirements

If the RF power capability of the MS is changed during a call, this change shall be signalled to the network.

If the RF power capability of the MS is changed in idle mode, the up to date RF power capability shall be signalled to the network during RR connection establishment.

References

GSM 04.08 section 3.4.10.

##### 26.6.11.1.2	Test purpose

To verify that if the RF power capability or any other capability indicated in a Classmark IE of the MS is changed during a call, the change is communicated on the DCCH to the network.

To verify that if the RF power capability or any other capability indicated in a Classmark IE of the MS is changed in idle mode, the out of date capabilities are not communicated to the network during RR connection establishment.

##### 26.6.11.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and with no additional power amplification applied.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Support for more than one RF power class: yes/no.

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated and with no additional power amplification applied.

Test Procedure

With the MS in idle mode, the RF power capability shall be changed by the addition of power amplification, after which the MS is made to originate a call. The new RF power capability shall be included in the CM SERVICE REQUEST message. After the call has reached the Call Control state U10, the RF power capability of the MS is changed by removal of the additional power amplification. The MS shall send a CLASSMARK CHANGE message indicating the new RF power capability. The RF power capability is then changed by adding the power amplification. The MS shall again send a CLASSMARK CHANGE message indicating the new RF power capability. The call is then released by the SS.

With the MS in idle mode, the power amplification is removed. The SS then pages the MS, which in the PAGING RESPONSE message shall indicate the correct RF power capability.

Finally the SS transmits a CHANNEL RELEASE to end the test.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

Step	Direction	Message	Comments

1	--------	--------	Add power amplification
	2	--------	--------	The MS shall be made to
				originate a call
	3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call"
				NECI not equal to one
	4	SS -> MS	IMM ASSIGNMENT
	5	MS -> SS	CM SERVICE REQUEST	The "Mobile Station Classmark 2"
				IE shall indicate the new RF power
				capability
	6	SS -> MS	CM SERVICE ACCEPT
	7	MS -> SS	SETUP
	8	SS -> MS	CALL PROCEEDING
	9	SS -> MS	ALERT
	10	SS -> MS	ASSIGNMENT COMMAND
	11	MS -> SS	ASSIGNMENT COMPLETE
	12	SS -> MS	CONNECT
	13	MS -> SS	CONNECT ACKNOWLEDGE
	14	--------	--------	Remove the power amplification
	15	MS -> SS	CLASSMARK CHANGE	The "Mobile Station Classmark 2"
				IE shall indicate the new power
				capability
	16	--------	--------	Add power amplification
	17	MS -> SS	CLASSMARK CHANGE	The "Mobile Station Classmark 2"
				IE shall indicate the new power
				capability
	18	SS -> MS	CHANNEL RELEASE
	19	--------	--------	Remove the power amplification
		--------	--------	The SS waits 12 seconds to allow the
				MS to perform cell reselection
	20	SS -> MS	PAG REQ TYPE 1
	21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	22	SS -> MS	IMM ASSIGNMENT
	23	MS -> SS	PAGING RESPONSE	The "Mobile Station Classmark 2"
				IE shall indicate the new power
				capability
	24	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

#### 26.6.11.2	Classmark interrogation

This procedure allows the network to request the MS to supply all its classmark information to the network.

Networks may systematically use this procedure (eg. during location updating) and, it if is incorrectly implemented in the MS, the basic connection establishment procedure may systematically fail.

##### 26.6.11.2.1	Conformance requirements

On receipt of a CLASSMARK ENQUIRY message, the MS sends a CLASSMARK CHANGE message to the network containing the Mobile Station Classmark 2 information element and depending upon the contents of this information element, possibly the Mobile Station Classmark 3 information element.

References

GSM 04.08 sections 3.4.11 and 9.1.11.1.

GSM 04.13 [version 1.2.0, section 5.2.9]

##### 26.6.11.2.2	Test purpose

To verify that if the network requests the MS to supply all its classmark information then this information is communicated on the DCCH to the network.

##### 26.6.11.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

"Idle, updated", with TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Contents of Mobile Station Classmark 2 information element.

Existence of Mobile Station Classmark 3 information element: yes/no.

Contents of Mobile Station Classmark 3 information element.

Switch off button: yes/no.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is switched off (or has its power removed).

The SS then sets the IMSI attach-detach flag in the SYSTEM INFORMATION messages so that the MS shall perform a location update when switched on.

The MS is switched on (or its power is re-applied). The MS then initiates a location update attempt. After the mobile has sent the LOCATION UPDATING REQUEST message, the SS transmits a CLASSMARK ENQUIRY message. The MS shall be ready to transmit the CLASSMARK CHANGE message before 300 ms after the end of the CLASSMARK ENQUIRY message.

The term 'ready to transmit' is defined in GSM 04.13.

Then the SS transmits a LOCATION UPDATING ACCEPT message that does not contain a Mobile Identity IE.

Maximum Duration of Test

2 minutes.

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is switched off (or has its power
				removed).
  	2	SS		IMSI attach-detach flag changed.
  	3	MS		The MS is switched on (or its power
				is re-applied).
	  4	MS -> SS	CHANNEL REQUEST
	  5	SS -> MS	IMMEDIATE ASSIGNMENT
	  6	MS -> SS	LOCATION UPDATING REQ	
	  7	SS -> MS	CLASSMARK ENQUIRY
	  8	MS -> SS	CLASSMARK CHANGE	Contents as defined for default message.
				This message shall be ready to be transmitted
				before 300 ms after the completion of step 7.
	  9	SS -> MS	LOCATION UPDATING ACC
	 10	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

Default Message Contents

Contents of LOCATION UPDATING ACCEPT message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					00000010

Location Area Identification

-	Mobile Country Code			001 decimal

-	Mobile Network Code			01 decimal

-	Location Area Code			0001H

Mobile Identity					Not present

Follow on proceed					Not present

### 26.6.12	Test of channel release

The purpose of this procedure is to deactivate the dedicated channels in use. When the channels are released, the MS returns to the CCCH configuration, idle mode.

#### 26.6.12.1	Channel release / SDCCH

##### 26.6.12.1.1	Conformance requirements

After the acknowledgement of the Layer 2 disconnection by the network, the MS shall not produce any further RF-transmission.

References

GSM 04.08 section 3.5.1.

##### 26.6.12.1.2	Test purpose

To verify that the MS is able to correctly release an SDCCH after having received a CHANNEL RELEASE message.

##### 26.6.12.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall initiate a Layer 2 disconnection process on the main signalling link. After the acknowledgement of the Layer 2 disconnection by the SS, the MS shall stop transmission of Layer 2 messages. This is verified for 3 seconds. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test

20 seconds

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.
	6	MS -> SS	DISC
	7	SS -> MS	UA
		--------	--------	The SS verifies for 3 seconds that the
				MS does not produce any Layer 2 messages
		--------	--------	The SS waits 12 seconds to allow the
				MS to perform cell reselection
	8	SS -> MS	PAG REQ TYPE 1
	9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	10	SS -> MS	IMM ASSIGNMENT	
	11	MS -> SS	PAGING RESPONSE
	12	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

#### 26.6.12.2	Channel release / SDCCH - no L2 ACK

##### 26.6.12.2.1	Conformance requirements

After the expiry of timer T3110 the MS shall not produce any further RF-transmission.

References

GSM 04.08 section 3.5.1.

##### 26.6.12.2.2	Test purpose

To verify that the MS is able to correctly release a SDCCH after having received a CHANNEL RELEASE message, even if the SS does not L2 acknowledge the L2 DISC frame.

##### 26.6.12.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall send at least 2 L2 DISC frames. The SS does not acknowledge any of the L2 DISC frames. After 2 seconds, the SS verifies for 3 seconds that the MS has stopped transmission of Layer 2 messages. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test

25 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.
	6	MS - > SS	DISC	The MS shall send at least 2 L2
				DISC frames, to which the SS does
				not respond. After a period of 2
				seconds, the SS verifies for 3
				seconds that the MS does not
				produce any further Layer 2 messages
		--------	--------	The SS waits 12 seconds to allow the
				MS to perform cell reselection
	7	SS -> MS	PAG REQ TYPE 1
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	SS -> MS	IMM ASSIGNMENT	
	10	MS -> SS	PAGING RESPONSE
	11	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None.

#### 26.6.12.3	Channel release / TCH-F

##### 26.6.12.3.1	Conformance requirements

After the acknowledgement of the Layer 2 disconnection by the network the MS shall not produce any further RF-transmission.

References

GSM 04.08 section 3.5.1.

##### 26.6.12.3.2	Test purpose

To verify that the MS is able to correctly release a full-rate TCH after having received a CHANNEL RELEASE message.

##### 26.6.12.3.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Supported rate of TCH: TCH/F.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall initiate a Layer 2 disconnection process on the main signalling link. After the acknowledgement of the Layer 2 disconnection by the SS, the MS shall stop transmission of Layer 2 messages. This is verified for 3 seconds. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test

20 seconds

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Channel Type = "Bm + ACCHs"
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.
	6	MS -> SS	DISC
	7	SS -> MS	UA
		--------	--------	The SS verifies for 3 seconds that the
				MS does not produce any Layer 2 messages
		--------	--------	The SS waits 12 seconds to allow the
				MS to perform cell reselection
	8	SS -> MS	PAG REQ TYPE 1
	9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	10	SS -> MS	IMM ASSIGNMENT	
	11	MS -> SS	PAGING RESPONSE
	12	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None

#### 26.6.12.4	Channel release / TCH-F - no L2 ACK

##### 26.6.12.4.1	Conformance requirements

After the expiry of timer T3110 the MS shall not produce any further RF-transmission.

References

GSM 04.08 section 3.5.1.

##### 26.6.12.4.2	Test purpose

To verify that the MS is able to correctly release a TCH/F after having received a CHANNEL RELEASE message, even if the SS does not L2 acknowledge the L2 DISC frame.

##### 26.6.12.4.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statements

Type of MS (P-GSM900 or EGSM or DCS1800).

Supported rate of TCH: TCH/F.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message (with cause "abnormal release, unspecified"), after which the MS shall send at least 2 L2 DISC frames. The SS does not acknowledge any of the L2 DISC frames. After 2 seconds, the SS verifies for 3 seconds that the MS has stopped transmission of Layer 2 messages. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test

25 seconds.

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Channel Type = "Bm + ACCHs"
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	CHANNEL RELEASE	Cause value = "Abnormal release,
				unspecified".
	6	MS -> SS	DISC	The MS shall sned at
				least 2 L2 DISC frames, to which
				the SS does not respond. After a
				period of 2 seconds, the SS
				verifies for 3 seconds that the MS
				does not produce any further
				Layer 2 messages
		--------	--------	The SS waits 12 seconds to allow the
				MS to perform cell reselection
	7	SS -> MS	PAG REQ TYPE 1
	8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"
	9	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8
	10	MS -> SS	PAGING RESPONSE
	11	SS -> MS	CHANNEL RELEASE

Specific Message Contents

None.

### 26.6.13	Test of starting time

The Assignment, Handover and immediate assignment procedures can include a delayed change of frequency list, MAIO and HSN. This series of tests checks the behavior of the Mobile Station when receiving channel allocation messages with a starting time and channel description for both before and after the starting time. Tests checking the phase 1 usage of the starting time (that is without a channel description for before the time) are included in the series related to immediate assignment, dedicated assignment and handover.

#### 26.6.13.1	Dedicated assignment with starting time / successful case / time not elapsed

##### 26.6.13.1.1	Conformance requirement

A Mobile Station receiving an ASSIGNMENT COMMAND message with a starting time and channel descriptions for both after and before the starting time, and ready to access before the indicated time has elapsed, shall perform the assignment on the channels as described for before the starting time and shall start using the new frequency parameters (frequencies and hopping sequence, or single frequency) in the correct time slot indicated by the starting time.

The Mobile Station shall accept the ASSIGNMENT COMMAND message for different message formatting, differing by the information elements used to describe frequency lists.

Reference(s)

GSM 04.08 sub-clauses 3.4.3.1, 9.1.2.

##### 26.6.13.1.2	Test purpose

To verify that the MS, after receiving an ASSIGNMENT COMMAND message with a starting time and channel  descriptions both for before and after the starting time, and ready to access before the indicated time, performs correctly the assignment using the description for before the time, and eventually starts using the frequency parameters  for after the time at the time indicated in the message.

##### 26.6.13.1.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a hopping SDCCH. Then the SS sends an ASSIGNMENT COMMAND message allocating a channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions (hopping case) for both before and after the starting time, as detailed in the "specific message contents" clause. The indicated time is such that the Mobile Station is ready to access before that time. The Mobile Station then accesses the channel as described for before the starting time. The MS shall eventually, at the TDMA frame defined by the contents of the "Starting Time" information element of the ASSIGNMENT COMMAND message, use the new frequency parameters . The verification is performed at the RF burst level.

Test parameters:

T1 is chosen arbitrarily to be between T0+60 and T0+100 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

Maximum duration of test

45 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	ASSIGN COMMAND	See specific message contents
	6	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel (before time
				parameters) after establishment of the main
				signalling link.
	7	--------	Time T1	The SS checks that the MS is transmitting
				now on the correct frequencies (after time
				parameters) and that the transmission started
				in the correct frame.
	8	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the ASSIGNMENT COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

ASSIGNMENT COMMAND

Information element			Value/remark

Channel Description, after time

Channel Type and TDMA offset		Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Channel Mode

Mode					Arbitrarily selected from capabilities declared for

the MS for the chosen type of channel

Mobile Allocation, after time			Chosen arbitrarily, at least one frequency.  In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T1

Channel Description, before time

Channel Type and TDMA offset		Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency and different

from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.2	Dedicated assignment with starting time / successful case / time elapsed

##### 26.6.13.2.1	Conformance requirement

A Mobile Station receiving an ASSIGNMENT COMMAND message with a starting time and channel descriptions for both after and before the starting time, and ready to access after the indicated time has elapsed, shall perform the assignment on the channels as described for after the starting time.

The Mobile Station shall accept the ASSIGNMENT COMMAND message for different message formattings, differing by the information elements used to describe frequency lists.

Reference(s):

GSM 04.08 sub-clauses 3.4.3.1, 9.1.2.

##### 26.6.13.2.2	Test purpose

To verify that the MS, after receiving an ASSIGNMENT REQUEST message with a starting time and channel  descriptions both for before and after the starting time, performs correctly the assignment using the frequency parameters  for after the time if the indicated time has already elapsed when the Mobile Station is ready to transmit.

##### 26.6.13.2.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a hopping SDCCH. Then the SS sends an ASSIGNMENT COMMAND message allocating a channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions (hopping case) for both before and after the starting time, as detailed in the "specific message contents" clause. The indicated time is such that the Mobile Station is ready to access only after that time. The Mobile Station then accesses the channel as described for after the starting time. The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

Maximum duration of test

45 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Hopping channel
	3	SS -> MS	IMM ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	ASSIGN COMMAND	See specific message contents
	6	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel (after time
				parameters) after establishment of the main
				signalling link.
	7	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the ASSIGNMENT COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

ASSIGNMENT COMMAND

Information element			Value/remark

Channel Description, after time

Channel Type and TDMA offset		Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time			Chosen arbitrarily, at least one frequency. In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T1

Channel Description, before time

Channel Type and TDMA offset		Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency and different

from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.3	Dedicated assignment with starting time and frequency redefinition / failure case / time not elapsed

##### 26.6.13.3.1	Conformance requirement

An MS, after receiving a FREQUENCY REDEFINITION message, shall keep the provided information until the time is elapsed. The Mobile Station must accept an intervening dedicated assignment, and, in case of failure of this assignment resulting in a return to the old channel before the time indicated in the FREQUENCY REDEFINITION message, shall return on the old channel with the frequency parameters in use at the moment of the reception of the FREQUENCY REDEFINITION message, and shall eventually start using the new frequency parameters in the correct time slot indicated by the starting time of the FREQUENCY REDEFINITION message.

Reference(s):

GSM 04.08 sub-clauses 3.4.3, 3.4.5.

##### 26.6.13.3.2	Test purpose

To verify that the MS, after receiving a FREQUENCY REDEFINITION and then an ASSIGNMENT REQUEST message with a starting time and channel  descriptions both for before and after the starting time, failing the assignment and returning on the old channel, and ready to access before the time indicated in the FREQUENCY REDEFINITION, resumes transmission on the channels used at the time of the reception of the FREQUENCY REDEFINITION message and eventually starts using the new frequency parameters  at the time indicated in the FREQUENCY REDEFINITION message.

##### 26.6.13.3.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported). Then the SS sends a FREQUENCY REDEFINITION message (starting time T1), which modifies the frequency parameters to be used by the MS. Then the SS sends an ASSIGNMENT COMMAND message, with a starting time (T2, possibly distinct from T1) and channel descriptions for both before and after the starting time. The channels and hopping sequences so allocated are distinct from those used and from those described by the FREQUENCY REDEFINITION message. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel and trigger the establishment of the main signalling link on the old channel. Then the MS shall send an ASSIGNMENT FAILURE message. Time T1 is chosen so it is reached only after the sending of the ASSIGNMENT FAILURE message. The MS shall eventually, at the TDMA frame defined by the contents of the "Starting Time" information element of the FREQUENCY REDEFINITION message, use the new frequency parameters. The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5000 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

T2 is set to T0+4000 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

Maximum duration of test

180 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	FREQUENCY REDEF
	6	SS -> MS	ASSIGNMENT COMMAND	Hopping channel, type among possible, 
				signalling mode.
	7	MS -> SS	ASSIGNMENT FAILURE	Sent on the correct channel (original
				parameters) after establishment of the main
				signalling link.
	8	--------	Time T1	The SS checks that the MS is transmitting
				now on the correct frequencies (parameters of
				the FREQUENCY REDEFINITION message)
				and that the transmissions started in the
				correct frame.
	9	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the ASSIGNMENT COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

FREQUENCY REDEFINITION

Information element			Value/remark

Channel Description

Channel Type and TDMA offset		Same as in IMMEDIATE ASSIGNMENT

Timeslot Number			Same as in IMMEDIATE ASSIGNMENT

Training Sequence Code		Same as in IMMEDIATE ASSIGNMENT

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different than those of the

IMMEDIATE ASSIGNMENT message

Mobile Allocation				Chosen arbitrarily, at least two frequencies, different

than those of the IMMEDIATE ASSIGNMENT message

Starting Time				T1

ASSIGNMENT COMMAND:

Information element			Value/remark

Channel Description, after time

Channel Type and TDMA offset		Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time			Chosen arbitrarily, at least two frequency

Starting Time				T2

Channel Description, before time

Channel Type and TDMA offset		Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least two frequencies, different

from "Mobile Allocation, after time"

#### 26.6.13.4	Dedicated assignment with starting time and frequency redefinition / failure case / time elapsed

##### 26.6.13.4.1	Conformance requirement

An MS, after receiving a FREQUENCY REDEFINITION message, shall keep the provided information until the time is elapsed. The Mobile Station must accept an intervening dedicated assignment, and, in case of failure of this assignment resulting in a return to the old channel after the time indicated in the FREQUENCY REDEFINITION message, shall return on the old channel with the frequency parameters indicated in the FREQUENCY REDEFINITION message.

Reference(s):

GSM 04.08 sub-clauses 3.4.3, 3.4.5.

##### 26.6.13.4.2	Test purpose

To verify that the MS, after receiving a FREQUENCY REDEFINITION and then an ASSIGNMENT REQUEST message with a starting time and channel  descriptions both for before and after the starting time, failing the assignment and returning on the old channel, and ready to access after the time indicated in the FREQUENCY REDEFINITION, resumes transmission using the new frequency parameters indicated in the FREQUENCY REDEFINITION message.

##### 26.6.13.4.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported). Then the SS sends a FREQUENCY REDEFINITION message (starting time T1), which modifies the frequency parameters to be used by the MS. Then the SS sends an ASSIGNMENT COMMAND message, with a starting time (T2) and channel descriptions for both before and after the starting time. Time T1 is chosen so it is reached after the sending of the ASSIGNMENT COMMAND message, but before the return on the old channel. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel with the new frequency parameters as indicated by the FREQUENCY REDEFINITION message, and trigger the establishment of the main signalling link on the old channel. Then the MS shall send an ASSIGNMENT FAILURE message. The verification is performed at the RF burst level.

Test parameters:

T2 is set to T0+5000 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

T1 is set to T0+10 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

Maximum duration of test

180 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	FREQUENCY REDEF
	6	SS -> MS	ASSIGNMENT COMMAND	Hopping channel, type among possible, 
				signalling mode.
	7	MS -> SS	ASSIGNMENT FAILURE	Sent on the correct channel (parameters
				from the FREQUENCY REDEF message)
				after establishment of the main signalling link.
	8	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the ASSIGNMENT COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

FREQUENCY REDEFINITION

Information element			Value/remark

Channel Description

Channel Type and TDMA offset		Same as in IMMEDIATE ASSIGNMENT

Timeslot Number			Same as in IMMEDIATE ASSIGNMENT

Training Sequence Code		Same as in IMMEDIATE ASSIGNMENT

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different than those of the

IMMEDIATE ASSIGNMENT message

Mobile Allocation				Chosen arbitrarily, at least two frequencies, different

than those of the IMMEDIATE ASSIGNMENT

message

Starting Time				T1

ASSIGNMENT COMMAND

Information element			Value/remark

Channel Description, after time

Channel Type and TDMA offset		Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time			Chosen arbitrarily, at least two frequencies

Starting Time				T2

Channel Description, before time

Channel Type and TDMA offset		Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least two frequencies, different

from "Mobile Allocation, after time"

#### 26.6.13.5	Handover with starting time / successful case / time not elapsed

##### 26.6.13.5.1	Conformance requirement

A Mobile Station receiving an HANDOVER COMMAND message with a starting time and channel descriptions for both after and before the starting time, and ready to access before the indicated time has elapsed, shall perform the handover on the channels as described for before the starting time and shall, if specified, the frequency list, MAIO and HSN  in the correct time slot indicated by the starting time.

The Mobile Station shall accept the HANDOVER  COMMAND message for different message formatting, differing by the information elements used to describe frequency lists.

Reference(s):

GSM 04.08 sub-clauses 3.4.4.1, 9.1.15.

##### 26.6.13.5.2	Test purpose

To verify that the MS, after receiving a HANDOVER COMMAND message with a starting time and channel  descriptions both for before and after the starting time, and ready to access before the indicated time, performs correctly the handover using the description for before the time, and then starts using the frequency parameters  for after the time at the time indicated in the message.

##### 26.6.13.5.3	Method of test

Initial condition(s)

System Simulator:

2 cells, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated, and camped on cell B.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a hopping SDCCH. After the SS has received measurements concerning cell B, the SS sends a HANDOVER COMMAND  message allocating a channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions (hopping case) for both before and after the starting time , as detailed in the "specific message contents" clause. The indicated time is such that the Mobile Station is ready to access before that time. The Mobile Station then accesses the channel as described for before the starting time. The MS shall eventually, at the TDMA frame defined by the contents of the "Starting Time" information element of the HANDOVER COMMAND message, use the new frequency parameters. The verification is performed at the RF burst level.

Test parameters:

T1 is chosen arbitrarily to be between T0+60 and T0+100 (mod 42432), where T0 is the frame number at which the first burst of the HANDOVER COMMAND message is sent.

Maximum duration of test

120 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	HANDOVER COMMAND	See specific message contents
	6	MS -> SS	HANDOVER COMPLETE	Sent on the correct channel (before time
				parameters) after establishment of the main
				signalling link.
	7	--------	Time T1	The SS checks that the MS is transmitting
				now on the correct frequencies (after time
				parameters) and that the transmissions
				started in the correct frame.
	8	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the HANDOVER COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

HANDOVER COMMAND:

Information element			Value/remark

Cell Description				As for cell B

Channel Description, after time

Channel Type and TDMA offset		Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Synchronisation indication			Non synchronised

Cell Channel Description			As for cell B

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time			Chosen arbitrarily, at least one frequency. In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T1

Channel Description, before time

Channel Type and TDMA offset		Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency, different

from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.6	Handover with starting time / successful case / time elapsed

##### 26.6.13.6.1	Conformance requirement

A Mobile Station receiving a HANDOVER COMMAND message with a starting time and channel descriptions for both after and before the starting time, and ready to access after the indicated time has elapsed, shall perform the handover on the channels as described for after the starting time.

The Mobile Station shall accept the HANDOVER COMMAND message for different message formattings, differing by the information elements used to describe frequency lists.

Reference(s):

GSM 04.08 sub-clauses 3.4.4.1, 9.1.15.

##### 26.6.13.6.2	Test purpose:

To verify that the MS, after receiving a HANDOVER COMMAND message with a starting time and channel  descriptions both for before and after the starting time, and ready to transmit after the indicated time, performs correctly the handover using the frequency parameters  for after the time.

##### 26.6.13.6.3	Method of test

Initial condition(s)

System Simulator:

2 cells, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated, and camped on cell A

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated, and camped on cell B.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a hopping SDCCH. After the SS has received measurements concerning cell B, the SS sends a HANDOVER COMMAND message allocating a channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions (hopping case) for both before and after the starting time., as detailed in the "specific message contents" clause. The indicated time is such that the Mobile Station is ready to access only after that time. The Mobile Station then accesses the channel as described for after the starting time.  The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5 (mod 42432), where T0 is the frame number at which the first burst of the HANDOVER COMMAND message is sent.

Maximum duration of test

120 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	HANDOVER COMMAND	See specific message contents
	6	MS -> SS	HANDOVER COMPLETE	Sent on the correct channel (after time
				parameters) after establishment of the main
				signalling link.
	7	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the HANDOVER COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

The contents of the HANDOVER COMMAND message sent at step 5 depends on the nature of the channel after the time. Two cases are possible.

HANDOVER COMMAND:

Information element			Value/remark

Cell Description				As for cell B

Channel Description, after time

Channel Type and TDMA offset		Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Synchronisation indication			Non synchronised

Cell Channel Description			As for cell B

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time			Chosen arbitrarily, at least one frequency. In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T1

Channel Description, before time

Channel Type and TDMA offset		Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency and different						from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.7	Handover with starting time and frequency redefinition / failure case / time not elapsed

##### 26.6.13.7.1	Conformance requirement

An MS, after receiving a FREQUENCY REDEFINITION message, shall keep the provided information until the time is elapsed. The Mobile Station must accept an intervening handover, and, in case of failure of this handover resulting in a return to the old channel before the time indicated in the FREQUENCY REDEFINITION message, shall return on the old channel with the frequency parameters in use at the moment of the reception of the FREQUENCY REDEFINITION message, and shall eventually start using the new frequency parameters in the correct time slot indicated by the starting time of the FREQUENCY REDEFINITION message.

Reference(s):

GSM 04.08 sub-clauses 3.4.4, 3.4.5.

##### 26.6.13.7.2	Test purpose:

To verify that the MS, after receiving a FREQUENCY REDEFINITION and then a HANDOVER COMMAND message with a starting time and channel  descriptions both for before and after the starting time, failing the handover, and ready to access on the old channel before the time indicated in the FREQUENCY REDEFINITION, resumes transmission on the channels used at the time of the reception of the FREQUENCY REDEFINITION message and eventually starts using the new frequency parameters  at the time indicated in the FREQUENCY REDEFINITION message.

##### 26.6.13.7.3	Method of test

Initial condition(s)

System Simulator:

2 cells, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated, and camped on cell A.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated, and camped on cell A.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported). Then the SS sends a FREQUENCY REDEFINITION message (starting time T1), which modifies the frequency parameters to be used by the MS. Then the SS sends a HANDOVER COMMAND message, with a starting time (T2, possibly distinct from T1) and channel descriptions for both before and after the starting time. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel and trigger the establishment of the main signalling link on the old channel. Then the MS shall send a HANDOVER FAILURE message. Time T1 is chosen so it is reached only after the sending of the HANDOVER FAILURE message. The MS shall eventually, at the TDMA frame defined by the contents of the "Starting Time" information element of the FREQUENCY REDEFINITION message, use the new frequency parameters. The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5000 (mod 42432), where T0 is the frame number at which the first burst of the HANDOVER COMMAND message is sent.

T2 is set to T0+4000 (mod 42432), where T0 is the frame number at which the first burst of the HANDOVER COMMAND message is sent.

Maximum duration of test

180 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	FREQUENCY REDEF
	6	SS -> MS	HANDOVER COMMAND	Hopping channel, type among possible, 
				signalling mode.
	7	MS -> SS	HANDOVER FAILURE	Sent on the correct channel (original
				parameters) after establishment of the main
				signalling link.
	8	--------	Time T1	The SS checks that the MS is
				transmitting now on the correct
				frequencies (parameters of the
				FREQUENCY REDEFINITION message) and 
				that the transmissions
				started in the correct frame.
	9	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the HANDOVER COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

FREQUENCY REDEFINITION

Information element			Value/remark

Channel Description

Channel Type and TDMA offset		Same as in IMMEDIATE ASSIGNMENT

Timeslot Number			Same as in IMMEDIATE ASSIGNMENT

Training Sequence Code		Same as in IMMEDIATE ASSIGNMENT

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different than those of

the IMMEDIATE ASSIGNMENT message

Mobile Allocation				Chosen arbitrarily, at least two frequencies, different

than those of the IMMEDIATE ASSIGNMENT

message

Starting Time				T1

HANDOVER COMMAND:

Information element			Value/remark

Cell Description				As for cell B

Channel Description, after time

Channel Type and TDMA offset		Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Synchronisation indication			Non synchronised

Cell Channel Description			As for cell B

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time			Chosen arbitrarily, at least one frequency. In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T2

Channel Description, before time

Channel Type and TDMA offset		Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency and different

from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.8	Handover with starting time and frequency redefinition / failure case / time elapsed

##### 26.6.13.8.1	Conformance requirement

An MS, after receiving a FREQUENCY REDEFINITION message, shall keep the provided information until the time is elapsed. The Mobile Station must accept an intervening handover, and, in case of failure of this handover resulting in a return to the old channel after the time indicated in the FREQUENCY REDEFINITION message, shall return on the old channel with the frequency parameters indicated in the FREQUENCY REDEFINITION message.

Reference(s):

GSM 04.08 sub-clauses 3.4.4, 3.4.5.

##### 26.6.13.8.2	Test purpose:

To verify that the MS, after receiving a FREQUENCY REDEFINITION and then a HANDOVER COMMAND message with a starting time and channel descriptions both for before and after the starting time, failing the handover and returning on the old channel, and ready to access after the time indicated in the FREQUENCY REDEFINITION, resumes transmission using the new frequency parameters indicated in the FREQUENCY REDEFINITION message.

##### 26.6.13.8.3	Method of test

Initial condition(s)

System Simulator:

2 cells, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated, and camped on cell A.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated, and camped on cell A.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported). Then the SS sends a FREQUENCY REDEFINITION message (starting time T1), which modifies the frequency parameters to be used by the MS. Then the SS sends a HANDOVER COMMAND message, with a starting time (T2, possibly distinct from T1) and channel descriptions for both before and after the starting time. Time T1 is chosen so it is reached after the sending of the HANDOVER COMMAND message, but before the return on the old channel. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel with the new frequency parameters as indicated by the FREQUENCY REDEFINITION message, and trigger the establishment of the main signalling link on the old channel. Then the MS shall send a HANDOVER FAILURE message. The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5000 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

T2 is set to T0+10 (mod 42432), where T0 is the frame number at which the first burst of the ASSIGNMENT COMMAND message is sent.

Maximum duration of test

180 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	Hopping channel
	4	MS -> SS	PAGING RESPONSE.
	5	SS -> MS	FREQUENCY REDEF
	6	SS -> MS	HANDOVER COMMAND	Hopping channel, type among possible, 
				signalling mode.
	7	MS -> SS	HANDOVER FAILURE	Sent on the correct channel (parameters
				from the FREQUENCY REDEF message)
				after establishment of the main signalling link.
	8	SS -> MS	CHANNEL RELEASE

NOTE:	the sending of the HANDOVER COMMAND message can be delayed as needed to simplify the computing of T1.

Specific message contents

FREQUENCY REDEFINITION

Information element			Value/remark

Channel Description

Channel Type and TDMA offset		Same as in IMMEDIATE ASSIGNMENT

Timeslot Number			Same as in IMMEDIATE ASSIGNMENT

Training Sequence Code		Same as in IMMEDIATE ASSIGNMENT

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different than those of

the IMMEDIATE ASSIGNMENT message

Mobile Allocation				Chosen arbitrarily, at least two frequencies, different

than those of the IMMEDIATE ASSIGNMENT

message

Starting Time				T1

HANDOVER COMMAND

Information element			Value/remark

Cell Description				As for cell B

Channel Description, after time

Channel Type and TDMA offset		Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Power Command

Power level				Chosen arbitrarily .

Synchronisation indication			Non synchronised

Cell Channel Description			As for cell B

Channel Mode

Mode					Arbitrarily selected from capabilities declared for the

MS for the chosen type of channel

Mobile Allocation, after time			Chosen arbitrarily, at least one frequency. In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T1

Channel Description, before time

Channel Type and TDMA offset		Same as after time

Timeslot Number			Same as after time

Training Sequence Code		Same as after time

Hopping				Yes

Hopping parameters			Chosen arbitrarily, different from "after time"

Mobile Allocation, before time		Chosen arbitrarily, at least one frequency and different

from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.9	Immediate assignment with starting time / successful case / time not elapsed

##### 26.6.13.9.1	Conformance requirement

A Mobile Station receiving an IMMEDIATE ASSIGNMENT message with a starting time and channel descriptions for both after and before the starting time, and ready to access before the indicated time has elapsed, shall perform the assignment on the channels as described for before the starting time and shall start using the new frequencies and hopping sequence in the correct time slot when the MS is allocated a dedicated channel.

Reference(s):

GSM 04.08 sub-clauses 3.3.1.3.1, 9.1.18.

##### 26.6.13.9.2	Test purpose

To verify that the MS, after receiving an IMMEDIATE ASSIGNMENT message with a starting time and channel  descriptions both for before and after the starting time, and ready to access before the indicated time, performs correctly the assignment using the description for before the time, and then starts using the frequency parameters  for after the time at the time indicated in the message.

##### 26.6.13.9.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	Supported frequencies (P-GSM or E-GSM or DCS1800)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator sends an IMMEDIATE ASSIGNMENT message allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions for both before and after the starting time. The indicated time is such that the Mobile Station is ready to access before that time. The Mobile Station then accesses the channel as described for before the starting time. The MS shall eventually, at the TDMA frame defined by the contents of the "Starting Time" information element of the IMMEDIATE ASSIGNMENT message, use the new frequency parameters. The verification is performed at the RF burst level.

Test parameters:

T1 is chosen arbitrarily to be between T0+60 and T0+100 (mod 42432), where T0 is the frame number at which the first burst of the IMMEDIATE ASSIGNMENT COMMAND message is sent.

Maximum duration of test

45 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	See specific message contents
	4	MS -> SS	PAGING RESPONSE.
	5	--------	Time T1	The SS checks that the MS is
				transmitting now on the correct
				frequencies (after time parameters) and 
				that the transmissions
				started in the correct frame.
	6	SS -> MS	CHANNEL RELEASE

Specific message contents

IMMEDIATE ASSIGNMENT COMMAND

Information element			Value/remark

Page Mode				Normal

Channel Description

Channel Type and TDMA offset	Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Timing Advance				As needed

Mobile Allocation (after time)		Chosen arbitrarily, at least one frequency. In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time				T1

IA Rest Octet

MAIO					Chosen arbitrarily, different from "after time"

Mobile Allocation (before time)		Chosen arbitrarily, at least one frequency and different

from "Mobile Allocation, after time". In case of an MA

with just one frequency, the frequency should be

different from the BCCH carrier.

#### 26.6.13.10	Immediate assignment with starting time / successful case / time elapsed

##### 26.6.13.10.1	Conformance requirement

A Mobile Station receiving an IMMEDIATE ASSIGNMENT message with a starting time and channel descriptions for both after and before the starting time, and ready to access after the indicated time has elapsed, shall perform the assignment on the channels as described for after the starting time.

Reference(s):

GSM 04.08 sub-clauses 3.3.1.3.1, 9.1.18.

26.6.13.10.2	Test purpose

To verify that the MS, after receiving an IMMEDIATE ASSIGNMENT message with a starting time and channel  descriptions both for before and after the starting time, performs correctly the assignment using the frequencies and hopping sequence for after the time if the indicated time has already elapsed when the Mobile Station is ready to transmit.

##### 26.6.13.10.3	Method of test

Initial condition(s)

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT statement(s)

-	Type of MS (P-GSM900 or EGSM or DCS1800).

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator sends an IMMEDIATE ASSIGNMENT message allocating a hopping channel (TCH/F, TCH/H or SDCCH, randomly chosen among the channels supported), with a starting time and channel descriptions for both before and after the starting time. The indicated time is such that the Mobile Station is ready to access only after that time. The Mobile Station then accesses the channel as described for after the starting time. The verification is performed at the RF burst level.

Test parameters:

T1 is set to T0+5 (mod 42432), where T0 is the frame number at which the first burst of the IMMEDIATE ASSIGNMENT COMMAND message is sent.

Maximum duration of test

45 sec

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMM ASSIGNMENT	See specific message contents
	4	MS -> SS	PAGING RESPONSE.	The SS checks that the MS is
				transmitting now on the correct
				frequencies (after time parameters).
	5	SS -> MS	CHANNEL RELEASE

Specific message contents

IMMEDIATE ASSIGNMENT COMMAND:

Information element		Value/remark

Page Mode			Normal

Channel Description		Channel Type and TDMA offset

Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number		Chosen arbitrarily

Training Sequence Code	Chosen arbitrarily

Hopping			Yes

Hopping parameters		Chosen arbitrarily

Timing Advance			As needed

Mobile Allocation (after time)	Chosen arbitrarily, at least one frequency. In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

Starting Time			T1

IA Rest Octet

MAIO				Chosen arbitrarily, different from "after time"

Mobile Allocation (before time)	Chosen arbitrarily, at least one frequency and

different from "Mobile Allocation, after time". In case of

an MA with just one frequency, the frequency should

be different from the BCCH carrier.

### 26.6.14	Default contents of GSM900 layer 3 messages for RR tests

This section contains the default values of GSM900 L3 messages, which unless indicated otherwise in section 26.6 shall be transmitted by the system simulator and which are required to be received from the GSM900 MS under test. These values are used in order to be consistent with the phase 2 version of 26.6.

The neccessary L3 messages are listed in alphabetic order, with the exception of the SYSTEM INFORMATION messages, where it is the information elements which are listed in alphabetic order (this is because some information elements occur in several SYSTEM INFORMATION types).

In this section, decimal values are normally used. However, sometimes a hexadecimal value, indicated by an 'H', or a binary value, indicated by a 'B' is used.

Default SYSTEM INFORMATION:

NOTE 22:	SYSTEM INFORMATION 2 bis, SYSTEM INFORMATION 5 bis, SYSTEM INFORMATION 7, and SYSTEM INFORMATION 8 messages are not used.

Cell A

Contents of information elements in SYSTEM INFORMATION TYPE 1 to 6 messages for cell A.

(CBCH) Channel Description		Not present

(CBCH) Mobile Allocation			Not present

Cell Channel Description

-	Format identifier		bit map 0

-	Cell Allocation ARFCN		Channel Numbers 20, 30, 50 and 70.

Cell Identity

-	Cell Identity Value		0001H

Cell Options

-	Power Control Indicator		Power Control Indicator is not set

-	DTX Indicator			MS shall not use DTX

-	Radio-Link-Timeout		8 SACCH blocks

Cell Selection Parameters

-	Cell-Reselect-Hysteresis		12 dB

-	MX-TXPWR-MAX-CCH		Minimum level

-	ACS				No addition cell parameters are present in

SYSTEM INFORMATION messages 7 and 8.

-	NECI				New establishment causes not supported

-	RXLEV-ACCESS-MIN		Minimum level

Control Channel Description

-	Attach-Detach allowed		No Attach/Detach

-	BS-AG-BLKS-RES		0 blocks reserved for access grant

-	CCCH-CONF			1 basic physical channel used for

CCCH, combined with SDCCHs

-	BS-PA-MFRMS		5 multiframe periods for transmission of

paging messages

-	T3212 Timeout value		Infinite

L2 pseudo length

-	System information 1		21

-	System information 2		22

-	System information 3		18

-	System information 4		12

Location Area Identification

-	Mobile Country Code		001 decimal

-	Mobile Network Code		01 decimal

-	Location Area Code		0001H

Message Type

-	System information 1		00011001

-	System information 2		00011010

-	System information 3		00011011

-	System information 4		00011100

-	System information 5		00011101

-	System information 6		00011110

Neighbour Cells Description

-	Format identifier		bit map 0

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 10, 20, 80, 90, 100, 110 and 120

-	EXT-IND			This IE carries the complete BA

NCC Permitted				0000 0010

RACH Control Parameters

-	Max Retrans			Max 1 retrans

-	Tx-integer			5 slots used

-	Cell Barred for Access		Cell is not barred

-	Call Reestablishment Allowed	Not allowed

-	Access Control Class		Access is not barred

-	Emergency Call allowed		Yes

SI 1 rest octets				Not used (all bits are set to spare)

SI 2 rest octets				Not used (all bits are set to spare)

SI 3 rest octets				Not used (all bits are set to spare)

SI 4 rest octets				Not used (all bits are set to spare)

Default settings for cell A:

Downlink input level			63 dBmicroVolt emf

Uplink output power			minimum supported by the MS's power

class

Propagation profile			static

BCCH/CCCH carrier number		20

Cell B

The contents of SYSTEM INFORMATION TYPE 1 to 6 messages for cell B are identical to those of cell A with the following exceptions:

Cell Channel Description

-	Format Identifier		Bit map 0

-	Cell Allocation ARFCN		Channel Number 10

NOTE 23:	This IE needs modification when used in handover tests which command the MS to go to a frequency hopping channel in cell B.

Cell Identity

-	Cell Identity Value		0002H

Default settings for cell B:

Downlink input level				53 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power

class

Propagation profile				static

BCCH/CCCH carrier number			10

Contents of ALERTING message (SS to MS):

Protocol Discriminator				Call Control,

Transaction Identifier

TI value					As used in the SETUP message

TI flag					1 (destination side)

Message Type					00000001

All other information elements			Not present

Contents of ASSIGNMENT COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101110

Channel Description

-	Channel Type and TDMA offset		Bm + ACCHs

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code			Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Channel number 30

Power Command

-	Power level				Chosen arbitrarily by the test house

All other information elements			Not present

Contents of ASSIGNMENT COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101001

RR Cause

-	RR Cause Value			Normal event

Contents of ASSIGNMENT FAILURE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101111

RR Cause

-	RR Cause Value			Depending on test

Contents of AUTHENTICATION REQUEST message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					00010010

Ciphering Key Sequence Number

-	Key Sequence				Chosen randomly by the test house

from the range 0 to 6

Authentication Parameter RAND

-	RAND value				Chosen arbitrarily by the test house

Contents of AUTHENTICATION RESPONSE message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					0X010100

Other information element(s)			Not checked

Contents of CALL PROCEEDING message:

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag					1 (destination side)

Message Type					00000010

All other information elements			Not present

Contents of CHANNEL MODE MODIFY message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00010000

Channel Description

-	Channel Type and TDMA offset		Depending on test

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code			Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Channel number 30

Channel Mode

-	Mode					Depending on test

Contents of CHANNEL MODE MODIFY ACKNOWLEDGE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00010111

Channel Description

-	Channel Type and TDMA offset		Depending on test

-	Timeslot Number			Same as in the CHANNEL MODE MODIFY

message

-	Training Sequence Code			Same as in the CHANNEL MODE MODIFY

message

-	Hopping				Single RF channel

-	Frequency Band			Band number 0

-	ARFCN				Channel number 30

Channel Mode

-	Mode					Same as in the CHANNEL MODE MODIFY

message

Contents of CHANNEL RELEASE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00001101

RR Cause

-	RR Cause Value			Normal event

Contents of CHANNEL REQUEST message

Establishment Cause				Not checked

Random Reference				Not checked

Contents of CIPHERING MODE COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00110101

Cipher Mode Setting

-	algorithm identifier			cipher with A5/1

-	SC					Start ciphering

Cipher Response					IMEI shall not be included

Contents of CIPHERING MODE COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00110010

Mobile Identity					Not present

Contents of the CLASSMARK CHANGE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					000100110

Mobile Station Classmark 2				See PICS/PIXIT

Mobile Station Classmark 3				For presence and contents see PICS/PIXIT

Contents of CM SERVICE ACCEPT message:

Protocol Discriminator				Mobility Management

Skip Indicator					0000

Message Type					00100001

Contents of CM SERVICE REQUEST message

Protocol Discriminator				Mobility Management

Skip Indicator					0000

Message Type					0X100100

Other information elements				Not checked

Contents of CONNECT message (SS to MS)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag					1

Message Type					00000111

All other information elements			Not present

Contents of CONNECT ACKNOWLEDGE message (MS to SS)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag					0

Message Type					0X001111

Contents of HANDOVER ACCESS message:

Handover Reference				Equal to the value included in

the Handover Command message

Contents of HANDOVER COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Cell Description

-	Network Colour Code			1

-	Base station Colour Code		5

-	BCCH Carrier Number			Set to the BCCH carrier number of cell B.

(one of 10, 20, 80, 90, 100, 110 or 120)

Channel Description

-	Channel Type and TDMA offset		Bm + ACCHs

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code			Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Chosen randomly by the test house from those

supported on the target cell

Handover Reference

-	Handover Reference Value		Chosen arbitrarily by the test house.

Power Command

-	Power level				Chosen arbitrarily by the test house

All other information elements			Not present

Contents of HANDOVER COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101100

RR cause					Normal event

Time difference					Not present

Contents of HANDOVER FAILURE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101000

RR cause					Dependent on the test

Contents of IMMEDIATE ASSIGNMENT message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the IA rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111111

Page Mode

-	Page Mode				Normal Paging

Channel Description

-	Channel Type and TDMA offset		For non-combined CCCH/SDCCH (see initial

conditions), SDCCH/8, with subchannel

chosen randomly by

the testhouse

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

-	Timeslot Number			For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

-	Training Sequence Code			For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as

the BCC).

-	Hopping				Single RF channel

-	ARFCN				For non-combined CCCH/SDCCH (see initial

conditions), Channel number 30;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 20.

Request Reference				Pertaining to last Channel Request

sent by the MS

Timing Advance

-	Timing advance value			30 bit periods

Mobile Allocation

-	Length				0

Starting Time					Not present

IA rest octets					Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT EXTENDED message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the IAX rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 18.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111001

Page Mode

-	Page Mode				Normal Paging

Channel Description 1

- Channel Type and TDMA offset		For non-combined CCCH/SDCCH (see test

conditions), SDCCH/8, with subchannel

chosen randomly by the test house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

- Timeslot Number			For non-combined CCCH/SDCCH (see test

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

- Training Sequence Code		For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as the

BCC).

- Hopping				Single RF channel

- ARFCN				For non-combined CCCH/SDCCH (see initial

conditions),Channel number 30;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 20.

Request Reference 1				Pertaining to last Channel Request sent

by the MS

Timing Advance 1

-	Timing advance value			Chosen arbitrarily by the test house

Channel Description 2

-	Channel Type and TDMA offset		Same channel type as in Channel

Description 1, but different TDMA offset to that

in Channel Description 1.

-	Timeslot Number			equal to the value in Channel Description 1.

-	Training Sequence Code			equal to the value in Channel Description 1.

-	Hopping				Single RF channel

-	ARFCN				equal to the value in Channel Description 1.

Request Reference 2				Not pertaining to any Channel Requests sent

by the MS.

Timing Advance 2

-	Timing advance value			Chosen arbitrarily by the test house

Mobile Allocation

-	Length				0

Starting Time					Not present

IAX rest octets					Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT REJECT message:

L2 pseudo length					19

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111010

Page Mode

-	Page Mode				Normal Paging

Request Reference				Pertaining to last Channel Request sent

by the MS

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

IAR rest octets					Not used (all bits set to spare)

Contents of LOCATION UPDATING REQUEST message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					0X001000

Other information elements				Not checked

Contents of PAGING REQUEST TYPE 1 message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the P1 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 9.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100001

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel				any channel

Mobile Identity 1

-	odd/even indication			Even

-	Type of Identity				TMSI

-	Identity Digits				TMSI previously allocated to MS

Mobile Identity 2					Not present

P1 rest octets					Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 2 message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the P2 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100010

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel				any channel

Mobile Identity 1

-	TMSI value				TMSI previously allocated to MS

Mobile Identity 2

-	TMSI value				TMSI not allocated to MS

Mobile Identity					Not present

P2 rest octets					Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 3 message:

L2 pseudo length					19

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100100

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel			any channel

Mobile identity 1

-	TMSI value				TMSI previously allocated to MS

Mobile identity 2

-	TMSI value				TMSI not allocated to MS

Mobile identity 3

-	TMSI value				TMSI not allocated to MS

Mobile identity 4

-	TMSI value				TMSI not allocated to MS

P3 rest octets					Not used (all bits set to spare)

Contents of PAGING RESPONSE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100111

Ciphering Key Sequence Number

-	Key Sequence				Key sequence number previously allocated

to MS, or '111' if no key is available.

Mobile Station Classmark 2

Mobile Identity

-	odd/even indication			Even

-	Type of identity				TMSI

-	Identity Digits				TMSI previously allocated to MS

Contents of PHYSICAL INFORMATION message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101101

Timing advance					20 bit periods

Contents of SETUP message; (MS to SS; for full rate speech)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					any value from the set {0, ..., 6}

TI flag					0

Message Type					0X000101

Other information elements				Not checked

Contents of SETUP message; (SS to MS; for full rate speech)

(* This message is not used in this direction in the RR tests- yet! It is put into hidden text in case a new test needs it.*)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					chosen randomly by the test house from the

set {0, ..., 6}

TI flag						0

Message Type					00000101

BC repeat indicator				Not present

Bearer capability 1

IEI						04h

length						01h

information transfer capability		speech

transfer mode				circuit mode

coding standard				GSM standardized coding

radio channel requirement			full rate channel

Signal							[Not present]

All other information elements			Not present

### 26.6.15	Default contents of DCS1800 layer 3 messages for RR tests

This section contains the default values of DCS1800 L3 messages, which unless indicated otherwise in section 26.6 shall be transmitted by the system simulator and which are required to be received from the DCS1800 MS under test. These values are used in order to be consistent with the phase 2 version of 26.6.

The neccessary L3 messages are listed in alphabetic order, with the exception of the SYSTEM INFORMATION messages, where it is the information elements which are listed in alphabetic order (this is because some information elements occur in several SYSTEM INFORMATION types).

In this section, decimal values are normally used. However, sometimes a hexadecimal value, indicated by an 'H', or a binary value, indicated by a 'B' is used.

Default SYSTEM INFORMATION:

NOTE 23:	SYSTEM INFORMATION 2 bis, SYSTEM INFORMATION 7, and SYSTEM INFORMATION 8 messages are not used.

SYSTEM INFORMATION 5 bis is not sent as a default message. For those tests which require SYSTEM INFORMATION 5 bis see the specific message contents for that test

Cell A

Contents of information elements in SYSTEM INFORMATION TYPE 1 to 6 messages for cell A.

(CBCH) Channel Description		Not present

(CBCH) Mobile Allocation			Not present

Cell Channel Description

-	Format identifier		Range 512

-	Cell Allocation ARFCN		Channel Numbers, 590, 650, 750 and 850

Cell Identity

-	Cell Identity Value		0001H

Cell Options

-	Power Control Indicator		Power Control Indicator is not set, 0

-	DTX Indicator			MS shall not use DTX

-	Radio-Link-Timeout		8 SACCH blocks

Cell Selection Parameters

-	Cell-Reselect-Hysteresis		12 dB

-	MX-TXPWR-MAX-CCH		Minimum level

-	ACS				No addition cell parameters are present in

SYSTEM INFORMATION messages 7 and 8.

-	NECI				New establishment causes not supported

-	RXLEV-ACCESS-MIN		Minimum level

Control Channel Description

-	Attach-Detach allowed		MS shall not apply

-	BS-AG-BLKS-RES		0 blocks reserved for access grant

-	CCCH-CONF			1 basic physical channel used for

CCCH, combined with SDCCHs

-	BS-PA-MFRMS		5 multiframe periods for transmission of

paging messages

-	T3212 Timeout value		Infinite

L2 pseudo length

-	System information 1		21

-	System information 2		22

-	System information 3		18

-	System information 4		12

Location Area Identification

-	Mobile Country Code		001 decimal

-	Mobile Network Code		01 decimal

-	Location Area Code		0001H

Message Type

-	System information 1		00011001

-	System information 2		00011010

-	System information 3		00011011

-	System information 4		00011100

-	System information 5		00011101

-	System information 6		00011110

Neighbour Cells Description

-	Format identifier		Range 512

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers, 520, 590, 600, 700, 780,

810, 870

-	EXT-IND			This IE carries the complete BA. EXT-IND is 0.

NCC Permitted				0000 0010

RACH Control Parameters

-	Max Retrans			Max 1 retrans

-	Tx-integer			5 slots used

-	Cell Barred for Access		Cell is not barred

-	Call Reestablishment Allowed	Not Allowed

-	Access Control Class		Access is not barred

-	Emergency Call allowed		Yes

SI 1 rest octets				Not used (all bits are set to spare)

SI 2 rest octets				Not used (all bits are set to spare)

SI 3 rest octets				Not used (all bits are set to spare)

SI 4 rest octets				Not used (all bits are set to spare)

Default settings for cell A:

Downlink input level			63 dBmicroVolt emf()

Uplink output power			minimum supported by the MS's power

class

Propagation profile			static

BCCH/CCCH carrier number		ARFN 590

Cell B

The contents of SYSTEM INFORMATION TYPE 1 to 6 messages for cell B are identical to those of cell A with the following exceptions:

Cell Channel Description

-	Format Identifier		Range 512

-	Cell Allocation ARFCN		Channel Number 520

NOTE 23:	This IE needs modification when used in handover tests which command the MS to go to a frequency hopping channel in cell B.

Cell Identity

-	Cell Identity Value		0002H

Default settings for cell B:

Downlink input level			53 dBmicroVolt emf()

Uplink output power			minimum supported by the MS's power

class

Propagation profile			static

BCCH/CCCH carrier number		10

Contents of ALERTING message (SS to MS):

Protocol Discriminator			Call Control,

Transaction Identifier

TI value				As used in the SETUP message

TI flag				1 (destination side)

Message Type				00000001

All other information elements		Not present

Contents of ASSIGNMENT COMMAND message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101110

Channel Description

-	Channel Type and TDMA offset	Bm + ACCHs

-	Timeslot Number		Chosen randomly by the test house

-	Training Sequence Code		Chosen randomly by the test house

-	Hopping			Single RF channel

-	ARFCN			Channel number 650

Power Command

-	Power level			Chosen arbitrarily by the test house

All other information elements		Not present

Contents of ASSIGNMENT COMPLETE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101001

RR Cause

-	RR Cause Value		Normal event

Contents of ASSIGNMENT FAILURE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101111

RR Cause

-	RR Cause Value		Depending on test

Contents of AUTHENTICATION REQUEST message:

Protocol Discriminator			MM message

Skip Indicator				0000

Message Type				00010010

Ciphering Key Sequence Number

-	Key Sequence			Chosen randomly by the test house

from the range 0 to 6

Authentication Parameter RAND

-	RAND value			Chosen arbitrarily by the test house

Contents of AUTHENTICATION RESPONSE message:

Protocol Discriminator			MM message

Skip Indicator				0000

Message Type				0X010100

Other information element(s)		Not checked

Contents of CALL PROCEEDING message:

Protocol Discriminator			Call Control

Transaction Identifier

TI value				As used in the SETUP message

TI flag				1 (destination side)

Message Type				00000010

All other information elements		Not present

Contents of CHANNEL MODE MODIFY message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00010000

Channel Description

-	Channel Type and TDMA offset	Depending on test

-	Timeslot Number		Chosen randomly by the test house

-	Training Sequence Code		Chosen randomly by the test house

-	Hopping			Single RF channel

-	ARFCN			Channel number 650

Channel Mode

-	Mode				Depending on test

Contents of CHANNEL MODE MODIFY ACKNOWLEDGE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00010111

Channel Description

-	Channel Type and TDMA offset	Depending on test

-	Timeslot Number		Same as in the CHANNEL MODE MODIFY

message

-	Training Sequence Code		Same as in the CHANNEL MODE MODIFY

message

-	Hopping			Single RF channel

-	Frequency Band		Band number 0

-	ARFCN			Channel number 650

Channel Mode

-	Mode				Same as in the CHANNEL MODE MODIFY

message

Contents of CHANNEL RELEASE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00001101

RR Cause

-	RR Cause Value		Normal event

Contents of CHANNEL REQUEST message

Establishment Cause			Not checked

Random Reference			Not checked

Contents of CIPHERING MODE COMMAND message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00110101

Cipher Mode Setting

-	algorithm identifier		cipher with A5/1

-	SC				Start ciphering

Cipher Response				IMEI shall not be included

Contents of CIPHERING MODE COMPLETE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00110010

Mobile Identity				Not present

Contents of the CLASSMARK CHANGE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				000100110

Mobile Station Classmark 2

- RF Power Capability			See PICS/PIXIT.

- Frequency Capability		Set to 0.

Mobile Station Classmark 3			For presence and contents see PICS/PIXIT

Contents of CM SERVICE ACCEPT message:

Protocol Discriminator			Mobility Management

Skip Indicator				0000

Message Type				00100001

Contents of CM SERVICE REQUEST message

Protocol Discriminator			Mobility Management

Skip Indicator				0000

Message Type				0X100100

Other information elements			Not checked

Contents of CONNECT message (SS to MS)

Protocol Discriminator			Call Control

Transaction Identifier

TI value				As used in the SETUP message

TI flag				1

Message Type				00000111

All other information elements		Not present

Contents of CONNECT ACKNOWLEDGE message (MS to SS)

Protocol Discriminator			Call Control

Transaction Identifier

TI value				As used in the SETUP message

TI flag				0

Message Type				0X001111

Contents of HANDOVER ACCESS message:

Handover Reference			Equal to the value included in

the Handover Command message

Contents of HANDOVER COMMAND message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101011

Cell Description

-	Network Colour Code		1

-	Base station Colour Code	5

-	BCCH Carrier Number		Set to the BCCH carrier number of cell B.

(one of 520, 590, 600, 700, 780, 810 or 870 )

Channel Description

-	Channel Type and TDMA offset	Bm + ACCHs

-	Timeslot Number		Chosen randomly by the test house

-	Training Sequence Code		Chosen randomly by the test house

-	Hopping			Single RF channel

-	ARFCN			Chosen randomly by the test house from those

supported on the target cell

Handover Reference

-	Handover Reference Value	Chosen arbitrarily by the test house.

Power Command

-	Power level			Chosen arbitrarily by the test house

All other information elements		Not present

Contents of HANDOVER COMPLETE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101100

RR cause				Normal event

Time difference				Not present

Contents of HANDOVER FAILURE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101000

RR cause				Dependent on the test

Contents of IMMEDIATE ASSIGNMENT message:

L2 pseudo length				This is the sum of the lengths of all the

information elements present in the

message except for the IA rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00111111

Page Mode

-	Page Mode			Normal Paging

Channel Description

-	Channel Type and TDMA offset	For non-combined CCCH/SDCCH (see initial

conditions), SDCCH/8, with subchannel

chosen randomly by

the testhouse

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

-	Timeslot Number		For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

-	Training Sequence Code		For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as

the BCC).

-	Hopping			Single RF channel

-	ARFCN			For non-combined CCCH/SDCCH (see initial

conditions), Channel number 650;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 590.

Request Reference			Pertaining to last Channel Request

sent by the MS

Timing Advance

-	Timing advance value		30 bit periods

Mobile Allocation

-	Length			0

Starting Time				Not present

IA rest octets				Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT EXTENDED message:

L2 pseudo length				This is the sum of the lengths of all the

information elements present in the

message except for the IAX rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 18.

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00111001

Page Mode

-	Page Mode			Normal Paging

Channel Description 1

- Channel Type and TDMA offset	For non-combined CCCH/SDCCH (see test

conditions), SDCCH/8, with subchannel

chosen randomly by the test house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

- Timeslot Number		For non-combined CCCH/SDCCH (see test

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

- Training Sequence Code	For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as the

BCC).

- Hopping			Single RF channel

- ARFCN			For non-combined CCCH/SDCCH (see initial

conditions),Channel number 650;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 590.

Request Reference 1			Pertaining to last Channel Request sent

by the MS

Timing Advance 1

-	Timing advance value		Chosen arbitrarily by the test house

Channel Description 2

-	Channel Type and TDMA offset	Same channel type as in Channel

Description 1, but different TDMA offset to that

in Channel Description 1.

-	Timeslot Number		equal to the value in Channel Description 1.

-	Training Sequence Code		equal to the value in Channel Description 1.

-	Hopping			Single RF channel

-	ARFCN			equal to the value in Channel Description 1.

Request Reference 2			Not pertaining to any Channel Requests sent

by the MS.

Timing Advance 2

-	Timing advance value		Chosen arbitrarily by the test house

Mobile Allocation

-	Length			0

Starting Time				Not present

IAX rest octets				Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT REJECT message:

L2 pseudo length				19

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00111010

Page Mode

-	Page Mode			Normal Paging

Request Reference			Pertaining to last Channel Request sent

by the MS

Wait Indication				0 seconds

Request Reference			Not pertaining to the MS under test

Wait Indication				0 seconds

Request Reference			Not pertaining to the MS under test

Wait Indication				0 seconds

Request Reference			Not pertaining to the MS under test

Wait Indication				0 seconds

IAR rest octets				Not used (all bits set to spare)

Contents of LOCATION UPDATING REQUEST message:

Protocol Discriminator			MM message

Skip Indicator				0000

Message Type				0X001000

Other information elements			Not checked

Contents of PAGING REQUEST TYPE 1 message:

L2 pseudo length				This is the sum of the lengths of all the

information elements present in the

message except for the P1 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 9.

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00100001

Page Mode

-	Page Mode			Normal Paging

Channels needed

-	first channel			any channel

-	second channel			any channel

Mobile Identity 1

-	odd/even indication		Even

-	Type of Identity			TMSI

-	Identity Digits			TMSI previously allocated to MS

Mobile Identity 2				Not present

P1 rest octets				Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 2 message:

L2 pseudo length				This is the sum of the lengths of all the

information elements present in the

message except for the P2 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00100010

Page Mode

-	Page Mode			Normal Paging

Channels needed

-	first channel			any channel

-	second channel			any channel

Mobile Identity 1

-	TMSI value			TMSI previously allocated to MS

Mobile Identity 2

-	TMSI value			TMSI not allocated to MS

Mobile Identity				Not present

P2 rest octets				Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 3 message:

L2 pseudo length				19

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00100100

Page Mode

-	Page Mode			Normal Paging

Channels needed

-	first channel			any channel

-	second channel			any channel

Mobile identity 1

-	TMSI value			TMSI previously allocated to MS

Mobile identity 2

-	TMSI value			TMSI not allocated to MS

Mobile identity 3

-	TMSI value			TMSI not allocated to MS

Mobile identity 4

-	TMSI value			TMSI not allocated to MS

P3 rest octets				Not used (all bits set to spare)

Contents of PAGING RESPONSE message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00100111

Ciphering Key Sequence Number

-	Key Sequence			Key sequence number previously allocated

to MS, or '111' if no key is available.

Mobile Station Classmark 2

Mobile Identity

-	odd/even indication		Even

-	Type of identity			TMSI

-	Identity Digits			TMSI previously allocated to MS

Contents of PHYSICAL INFORMATION message:

Protocol Discriminator			RR Management

Skip Indicator				0000

Message Type				00101101

Timing advance				20 bit periods

Contents of SETUP message; (MS to SS; for full rate speech)

Protocol Discriminator			Call Control

Transaction Identifier

TI value				any value from the set {0, ..., 6}

TI flag				0

Message Type				0X000101

Other information elements			Not checked

Contents of SETUP message; (SS to MS; for full rate speech)

(* This message is not used in this direction in the RR tests- yet! It is put into hidden text in case a new test needs it.*)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					chosen randomly by the test house from the

set {0, ..., 6}

TI flag						0

Message Type					00000101

BC repeat indicator				Not present

Bearer capability 1

IEI						04h

length						01h

information transfer capability		speech

transfer mode				circuit mode

coding standard				GSM standardized coding

radio channel requirement			full rate channel

Signal							[Not present]

All other information elements			Not present

Specific message contents

IMMEDIATE ASSIGNMENT COMMAND:

Information element			Value/remark

Page Mode				Normal

Channel Description			Channel Type and TDMA offset

Chosen arbitrarily among that supported by the

Mobile Station

Timeslot Number			Chosen arbitrarily

Training Sequence Code		Chosen arbitrarily

Hopping				Yes

Hopping parameters			Chosen arbitrarily

Timing Advance				As needed

Mobile Allocation (after time)		Chosen arbitrarily, at least one frequency. In

case of an MA with just one frequency, the

frequency should be different from the BCCH

carrier.

Starting Time				T1

IA Rest Octet

MAIO					Chosen arbitrarily, different from "after time"

Mobile Allocation (before time)		Chosen arbitrarily, at least one frequency

and different from "Mobile Allocation, after

time". In case of an MA with just one frequency,

the frequency should be different from the BCCH

carrier.

## 26.7	Elementary procedures of mobility management

The tests are based on GSM 04.08 and GSM 03.03.

In this section, when the expected sequence require that "a mobile originated CM connection is attempted", it shall be for a service other than emergency call.

In this section, a initial CM message is either a SETUP message, a REGISTER message or a CP-DATA message (in that case the acknowledged mode of operation on SAPI 3 will have be established and this message will be sent on SAPI 3).

### 26.7.1	TMSI reallocation

The intention of the TMSI Reallocation procedure is to assign a new temporary identity for the MS. If the message is not understood by the MS, the network could not establish a link to the MS. As this is a common MM procedure, it can be initiated at any time.

[Editorial Note:	- it is not checked that this procedure can be initiated at any time

]

#### 26.7.1.1	Conformance requirement

1)	A Mobile Station shall acknowledge a new TMSI when explicitly allocated during a location updating procedure or an incoming call.

2)	The TMSI shall be updated on the SIM when the Mobile Station is correctly deactivated in accordance with the manufacturer's instructions.

3)	A Mobile Station shall answer paging with this TMSI and includes it in the Paging Response message.

Reference(s):

GSM 04.08 section 4.3.1, GSM 03.03 section 2, GSM 02.17 section 6.1.

#### 26.7.1.2	Test purpose

To verify that the MS is able to receive and acknowledge a new TMSI by means of an explicit TMSI reallocation procedure.

To verify that the MS has stored the TMSI in a non-volatile memory.

The implicit reallocation procedure is tested in section 26.7.4.1.

#### 26.7.1.3	Method of test

Initial conditions

System Simulator:

Two cells A and B, belonging to different location areas a and b, default parameters.

Mobile Station:

The MS has valid TMSI (= TMSI1), CKSN, Kc. It is "idle updated" on cell B.

Related PICS/PIXIT statement(s)

Switch off button Yes/No

Way to bring the MS into service

Foreseen final state of the MS

The MS has a valid TMSI (= TMSI1), CKSN, Kc. It is "idle updated" on cell A.

Test Procedure

The MS is paged in cell B and the ciphering mode is established. An explicit TMSI reallocation procedure is performed. The channel is released. The MS is switched off and then its power supply is interrupted for 10 seconds. The power supply is resumed and then the MS is switched on. The system simulator checks, by paging, whether the MS has stored the received TMSI.

The MS is made to select cell A. A normal location updating procedure is performed in cell A. An explicit TMSI reallocation procedure is performed and then the location updating procedure is accepted by the SS. The system simulator checks, by paging, whether the MS has stored the allocated TMSI.

Maximum duration of test

2 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B

	1	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI1
	2	MS -> SS	CHANNEL REQUEST	Establishment Cause: Answer to paging.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	CIPH MODE COMMAND	The SS starts deciphering.
	6	MS -> SS	CIPH MODE COMPLETE	The SS starts enciphering.
	7	SS -> MS	TMSI REALLOCATION CMD	"Mobile identity"= new TMSI 
					(TMSI2) different from TMSI 1.
	8	MS -> SS	TMSI REALLOCATION CMP	
	9	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
					the SS waits for the disconnection 
					of the main signalling link.
	10	MS		If possible (see PICS), the MS is switched off
	10a	MS		The power supply is interrupted for
					10 seconds.
	11	MS		The MS is brought into service
	12	SS		The SS waits an amount of time 
					which is enough to guarantee that 
					the MS is in service (listening to 
					its paging subchannel)
	13	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI2.
	14	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to
					paging.
	15	SS -> MS	IMMEDIATE ASSIGNMENT	
	16	MS -> SS	PAGING RESPONSE	"Mobile identity" =TMSI2.
	17	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

The following messsages are sent and shall be
				received on cell A

	18	SS		The RF level of cell B is lowered 
				until the MS selects cell A. The RF 
				level of cell B is set sufficiently 
				low to ensure that cell B is not 
				suitable as defined in GSM 05.08 
				section 6.6.2
	19	MS -> SS	CHANNEL REQUEST	Establishment cause:
					Location updating
	20	SS -> MS	IMMEDIATE ASSIGNMENT	
	21	MS -> SS	LOCATION UPDATING REQ	location updating type= normal,
				"ciphering key sequence number"= 
				CKSN, LAI= b, "mobile identity"= 
				TMSI2.
	22	SS -> MS	TMSI REALLOCATION CMD	TMSI = TMSI1
	23	MS -> SS	TMSI REALLOCATION CMP	
	24	SS -> MS	LOCATION UPDATING ACC	This message does not contain the 
				optional Mobile Identity field.
	25	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link. The SS waits an 
				amount of time which is enough to
				guarantee that the MS is "idle updated" on cell A
	26	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the new TMSI (= TMSI1).
	27	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Answer to paging.
	28	SS -> MS	IMMEDIATE ASSIGNMENT	
	29	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the new TMSI (= TMSI1).
	30	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

### 26.7.2	Authentication

The purpose of this procedure is to verify the user identity. A correct response is essential to guarantee the establishment of the connection. If not, the connection will drop.

The SS shall be able to handle vectors of Kc, RAND, and SRES in a similar way as the MSC/BSS entities. The SS shall incorporate a test algorithm for generating SRES and Kc from RAND and Ki which operates as described in Annex 4.

#### 26.7.2.1	Authentication accepted

##### 26.7.2.1.1	Conformance requirement

1)	A Mobile Station shall correctly respond to an Authentication Request message by sending an Authentication Response message with the SRES information field set to the same value as the one produced by the authentication algorithm in the network.

2)	A Mobile Station shall indicate in a Paging Response message the ciphering key sequence number which was allocated to it through the authentication procedure.

Reference(s)

GSM 04.08 section 4.3.2, GSM 03.03 section 2

##### 26.7.2.1.2	Test purpose

1)	To check that a Mobile Station correctly responds to an Authentication Request message by sending an Authentication Response message with the SRES information field set to the same value as the one produced by the authentication algorithm in the network.

2)	To check that a Mobile Station indicates in a Paging Response message the ciphering key sequence number which was allocated to it through the authentication procedure.

##### 26.7.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has valid TMSI, CKSN (CKSN1), Kc. It is "idle updated" on the cell.

Related PICS/PIXIT statement(s)

None

Foreseen final state of the MS

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on the cell.

Test Procedure

The MS is paged. After the MS has sent a PAGING RESPONSE message to the SS, the SS initiates an authentication procedure and checks the value SRES sent by the MS in the AUTHENTICATION RESPONSE message. The channel is released. The MS is paged and the SS checks the value of the ciphering key sequence number sent by the MS in the PAGING RESPONSE message.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	 2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer
				to paging.
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	PAGING RESPONSE	CKSN= CKSN1
	 5	SS -> MS	AUTHENT REQUEST	The SS initiates authentication
				with CKSN2 different from CKSN1
	 6	MS -> SS	AUTHENT RESPONSE	"Auth.parameter SRES" IE shall
				be bit exact with the value as
				produced by the authentication
				algorithm.
	 7	SS -> MS	CHANNEL RELEASE	After the sending of this
		 		message, the SS waits for the
				disconnection of the main
				signalling link. The SS waits an 
				amount of time which is enough to 
				guarantee that the MS is in 
				service.

8	SS -> MS	PAGING REQUEST TYPE 1
	 9	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer
				to paging.
	 10	SS -> MS	IMMEDIATE ASSIGNMENT	
	 11	MS -> SS	PAGING RESPONSE	"Ciphering key sequence number" 
				shall be the same as the value that 
				was sent in the last AUTHENTICATION 
				REQUEST message (= CKSN2).
	12	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

#### 26.7.2.2	Authentication rejected

##### 26.7.2.2.1	Conformance requirement

1)	After reception of an Authentication Reject message the Mobile Station shall:

1.1	not perform normal location updating

1.2	not perform periodic location updating

1.3	not respond to paging with TMSI

1.4	reject any request from CM entity for MM connection except for emergency call

1.5	not perform IMSI detach if deactivated

2)	After reception of an Authentication Reject message the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a CHANNEL REQUEST message with the establishment cause set to "emergency call" and include an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	After reception of an Authentication Reject message the Mobile Station shall delete the stored LAI, CKSN and TMSI.

Reference(s)

GSM 04.08 sections 4.3.2.5.

##### 26.7.2.2.2	Test purpose

1)	To check that ,after reception of an Authentication Reject message, the Mobile Station :

1.1	does not perform normal location updating

1.2	does not perform periodic location updating

1.3	does not respond to paging with TMSI

1.4	rejects any request from CM entity for MM connection except for emergency call

1.5	does not perform IMSI detach if deactivated

2)	To check that, after reception of an Authentication Reject message the Mobile Station, if it supports speech, accepts a request for an emergency call by sending a CHANNEL REQUEST message with the establishment cause set to "emergency call" and includes an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	To check that, after reception of an Authentication Reject message and after having been deactivated and reactivated, the MS performs location updating using its IMSI as mobile identity and indicates deleted LAI and CKSN.

##### 26.7.2.2.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has valid TMSI, CKSN (CKSN2) and Kc. It is "idle updated" on cell B.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

Foreseen final state of the MS

The MS has valid TMSI, CKSN (CKSN1) and Kc. It is "idle updated" on cell A.

Test procedure

The SS rejects an authentication. The channel is released. The SS checks that the MS has entered the state MM-IDLE substate NO IMSI, i.e. does not perform normal location updating, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if powered down.

Maximum duration of test

10 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	SS -> MS	PAGING REQUEST TYPE 1
	 2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer
				to paging.
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	PAGING RESPONSE	"Ciphering key sequence number" 
				shall be the same as the value that 
				was sent in the last AUTHENTICATION 
				REQUEST message (= CKSN2).
	 5	SS -> MS	AUTHENT REQUEST
	 6	MS -> SS	AUTHENT RESPONSE
	 7	SS -> MS	AUTHENT REJECT
	 8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

9	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell B.
				"Mobile identity" IE contains
				TMSI.
	 10	   MS		The MS shall ignore this
				message. This is verified
				during 3 seconds.

11	   SS		The SS waits for at least for 15 s
	 12	   MS		A MO CM connection is attempted.
	 13	   MS		The MS shall not initiate an RR
				connection establishment on cell A or
				cell B. This is checked during 3 seconds

14	   MS		If the MS supports speech (see PICS),
				an emergency call is attempted.
	 15	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Emergency call.
	 16	SS -> MS	IMMEDIATE ASSIGNMENT	
	 17	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency call
				establishment. "Mobile identity" : type
				of identity is set to IMEI
	 18	SS -> MS	CM SERVICE ACCEPT	
	 19	MS -> SS	EMERGENCY SETUP
	 20	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.
	 21	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be received on cell A

22	   SS		The RF levels are changed
				to make the MS reselect
				the cell A.
	 23	   MS		The MS performs cell
				reselection according to
				procedure as specified in GSM
				05.08 (this however is not checked 
				until step 29). The MS shall not initiate
				an RR connection establishment on cell A
				or on cell B.

24	   SS		The SS waits at least 7 minutes
				for a possible periodic
				updating.
	 25 	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

26	   MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	 27	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

28	MS		Depending on what has been 
				performed in step 26 the MS is 
				brought back to operation.
	 29	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating.
	 30	SS -> MS	IMMEDIATE ASSIGNMENT	
	 31	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"Mobile Identity"= IMSI,
				"LAI"= deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
	 32	SS -> MS	AUTHENT REQUEST	"CKSN"= CKSN1
	 33	MS -> SS	AUTHENT RESPONSE	
	 34	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity"= TMSI
	 35	MS -> SS	TMSI REALLOCATION CMP
	 36	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

### 26.7.3	Identification

The purpose of this procedure is to check that the MS gives its identity as requested by the network. If this procedure does not work, it will not be possible for the network to rely on the identity claimed by the MS.

#### 26.7.3.1	General Identification

##### 26.7.3.1.1	Conformance requirement

1)	When requested by the network the Mobile Station shall send its IMSI.

2)	When requested by the network the Mobile Station shall send the TMSI which it was previously allocated.

3)	When requested by the network the Mobile Station shall send its IMEI as stored in the Mobile Equipement.

4)	When requested by the network the Mobile Station shall send its IMEISV as stored in the Mobile Equipment.

Reference(s)

GSM 04.08 sections 4.3.3.

##### 26.7.3.1.2	Test purpose

1)	To verify that the MS sends identity information as requested by the system in the following cases: IMSI and TMSI are requested in non-ciphered mode, IMEI is requested in ciphered mode.

2)	To verify that the MS sends its IMEI, when requested to do so, in non-ciphered mode.

3)	To verify that the MS sends its IMEISV, when requested to do so, in non-ciphered mode.

##### 26.7.3.1.3	Method of test

###### 26.7.3.1.3.1	Identification / test 1

Initial conditions

System Simulator:

1 cell, default values

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on the cell.

Related PICS/PIXIT statement(s)

IMEI of the ME

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on the cell.

Test Procedure

The SS requests identity information from the MS :

-	IMSI in non ciphering mode,

-	allocated TMSI in non ciphering mode,

-	IMEI in ciphering mode.

Maximum duration of test

30 seconds

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	
	 2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to
				paging.
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	PAGING RESPONSE	
	 5	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMSI.
	 6	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the IMSI of the MS.

7	SS -> MS	IDENTITY REQUEST	"Identity type" IE is TMSI.
	 8	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the allocated TMSI of the MS.

9	SS -> MS	CIPH MODE COMMAND
	10	MS -> SS	CIPH MODE COMPLETE
	11	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEI.
	12	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the IMEI stored in the Mobile
				Equipment.
	13	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

###### 26.7.3.1.3.2	Identification / test 2

Initial conditions

System Simulator:

1 cell, default values

Mobile Station:

The MS has a valid TMSI. It is in "idle updated".

Related PICS/PIXIT statement(s)

IMEI of the ME

IMEISV of the ME

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

The SS requests identity information from the MS :

-	IMEI in non ciphering mode.

-	IMEISV in non ciphering mode.

Maximum duration of test

30 seconds

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to
				paging.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEI.
	6	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the IMEI of the MS.
	7	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEISV
	8	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the IMEISV of the MS.

9	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

#### 26.7.3.2	Handling of IMSI shorter than the maximum length

##### 26.7.3.2.1	Conformance requirement

The MS shall be capable of handling an IMSI that is not of the maximum length.

Reference(s)

GSM 04.08 clause 10.5.1.4

##### 26.7.3.2.2	Test purpose

To check that the MS behaves correctly when activated with an IMSI of length less than the maximum length.

In this condition, the MS shall:

-	perform location updating

-	answer to paging with IMSI

-	give the correct IMSI when asked by an IDENTITY REQUEST

-	attempt CM connection establishment when requested to

-	attempt call re-establishment when needed

-	attempt IMSI detach when needed

-	erase its TMSI when the IMSI is sent by the network in a LOCATION UPDATING ACCEPT or a TMSI REALLOCATION COMMAND message.

##### 26.7.3.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default values

Mobile Station:

The MS has no valid TMSI.

It is "idle updated".

The IMSI has the value 001011234.

Related PICS/PIXIT statement(s)

On/Off switch - Yes/No

Foreseen final state of MS

The MS has no valid TMSI. It is in "idle, updated".

Test Procedure

The MS is paged with its IMSI. The MS shall answer to paging and include the correct IMSI in the PAGING RESPONSE message. During call establishment, the SS asks for the IMSI of the MS. The MS shall answer by an IDENTITY RESPONSE message including the correct IMSI. During the active phase of the call, the SS stops sending valid SACCH frames. The MS performs call re-establishment. The MS shall include the correct IMSI in the CM RE-ESTABLISHMENT message. a TMSI REALLOCATION COMMAND including a TMSI is sent to the MS. The MS acknowledges this message. The call is release.

The MS is paged with its TMSI. The MS shall answer to paging and includes its TMSI in the PAGING RESPONSE message. During call establishment, the SS sends a TMSI REALLOCATION COMMAND including the IMSI to the MS. The MS shall acknowledge this message. The MS shall erase its TMSI. The call is released.

The MS is switched off or powered down. The MS performs IMSI detach. The MS shall include the correct IMSI in the IMSI DETACH message.

The MS is switched on or powered on. The MS performs IMSI attach. The MS shall include the correct IMSI in the LOCATION UPDATING REQUEST message. A TMSI is allocated to the MS.

The LAC of the cell is changed. The MS performs location updating. The SS includes the IMSI in the LOCATION UPDATING ACCEPT message.

A mobile originated CM connection is attempted. The MS shall include the correct IMSI in the CM SERVICE REQUEST message.

Maximum duration of test

5 minutes.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	"mobile identity 1" contains IMSI of MS
	2	MS -> SS	CHANNEL REQUEST	Establishment cause: Answer to paging
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE	"mobile identity" contains the IMSI of the MS
	5	SS -> MS	IDENTITY REQUEST	"identity type" IE is IMSI
	6	MS -> SS	IDENTITY RESPONSE	"mobile identity" IE contains the IMSI of the MS
	7			The call is established using the sequence of the
				generic terminating call set-up procedure.

8	SS		The SS stops sending valid SACCH frames
	9	MS -> SS	CHANNEL REQUEST
	10	SS -> MS	IMMEDIATE ASSIGNMENT
	11	MS -> SS	CM RE-ESTAB REQUEST	"mobile identity" IE contains IMSI of the MS
	12	SS -> MS	TMSI REALLOC COMMAND	"mobile identity" contains a TMSI
	13	MS -> SS	TMSI REALLOC COMPLETE
	14	SS -> MS	CHANNEL RELEASE

15	SS -> MS	CHANNEL RELEASE	After sending this message, the SS waits for the
				disconnection of the main signalling link.

16	SS -> MS	PAGING REQUEST TYPE 1	"mobile identity 1" contains TMSI of MS
	17	MS -> SS	CHANNEL REQUEST	Establishment cause: Answer to paging
	18	SS -> MS	IMMEDIATE ASSIGNMENT
	19	MS -> SS	PAGING RESPONSE	"mobile identity" contains the TMSI of the MS
	20	SS -> MS	AUTHENTICATION REQUEST
	21	MS -> SS	AUTHENTICATION RESPONSE
	22	SS -> MS	TMSI REALLOC COMMAND	"mobile identity" contains a IMSI of MS
	23	MS -> SS	TMSI REALLOC COMPLETE
	24	SS -> MS	CHANNEL RELEASE

25	MS		If possible (see PICS) the MS is switched off,
				otherwise the MS is powered down.
	26	MS -> SS	CHANNEL REQUEST	If the MS was switched off it
				performs IMSI detach
	27	SS -> MS	IMMEDIATE ASSIGNMENT
	28	MS -> SS	IMSI DETACH	"mobile identity" contains IMSI of MS
	29	SS -> MS	CHANNEL RELEASE

30	MS		The MS is activated
	31	MS -> SS	CHANNEL REQUEST
	32	SS -> MS	IMMEDIATE ASSIGNMENT
	33	MS -> SS	LOCATION UPDATING REQ	"mobile identity" contains IMSI of MS
	34	SS -> MS	LOCATION UPDATING ACC	"mobile identity" contains a TMSI
	35	MS -> SS	TMSI REALLOC COMPLETE
	36	SS -> MS	CHANNEL RELEASE

37	SS		The SS changes the LAC of the cell
	38	SS		The SS waits for at least 30 seconds
	39	MS -> SS	CHANNEL REQUEST
	40	SS -> MS	IMMEDIATE ASSIGNMENT
	41	MS -> SS	LOCATION UPDATING REQ	"mobile identity" contains TMSI of the MS
	42	SS -> MS	LOCATION UPDATING ACC	"mobile identity" contains IMSI of the MS
	43	SS -> MS	CHANNEL RELEASE

44	MS		a mobile originated CM connection is attempted
	45	MS -> SS	CHANNEL REQUEST
	46	SS -> MS	IMMEDIATE ASSIGNMENT
	47	MS -> SS	CM SERVICE REQUEST	"mobile identity" contains IMSI of the MS
	48	SS -> MS	CHANNEL RELEASE

Specific message contents

None

### 26.7.4	Location updating

This procedure is used to register the MS in the network. If it is not performed correctly, no call can be established.

#### 26.7.4.1	Location updating / accepted

##### 26.7.4.1.1	Conformance requirement

1.

1.1	If the network accepts a location updating from the Mobile Station and reallocates a TMSI in the Location Updating Accept message the Mobile Station shall acknowledge the reception of the new TMSI.

1.2	The Mobile Station shall answer to paging with this TMSI and include it in a Paging Response message.

2	If the network accepts a location updating from the Mobile Station and the Location Updating Accept message contains neither TMSI nor IMSI, the Mobile Station shall answer to paging when addressed with the last allocated TMSI and include it in the Paging Response message.

3.

3.1	If the network accepts a location updating from the Mobile Station by use of a Location Updating Accept message containing the IMSI of the Mobile Station, the Mobile Station shall not answer paging with the last allocated TMSI.

3.2	The Mobile Station shall still answer paging with IMSI.

Reference(s)

GSM 04.08 section 4.4.4.6.

##### 26.7.4.1.2	Test purpose

To test the behaviour of the MS if the network accepts the location updating of the MS.

For the network response three different cases are identified:

1)	TMSI is allocated,

2)	Location updating accept contains neither TMSI nor IMSI,

3)	Location updating accept contains IMSI.

##### 26.7.4.1.3	Method of test

Initial conditions:

System Simulator:

Two cells, A and B, belonging to different location areas with location area identification a and b of the same PLMN.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has a valid TMSI (=TMSI1) and CKSN (=CKSN1). It is "idle updated" on cell A.

Related PICS/PIXIT statement(s)

None.

Foreseen final state of the MS

The MS has no valid TMSI. It has valid CKSN and Kc. It is "idle, updated" on cell B.

Test Procedure

The MS is made to select cell B. A normal location updating with TMSI reallocation is performed in cell B. The channel is released. The SS checks, by paging, that the MS has stored the newly allocated TMSI. The channel is released. The MS is made to select cell A. A normal location updating is performed in cell A. The LOCATION UPDATING ACCEPT message contains neither IMSI nor TMSI. The SS checks, by paging, that the MS has kept the old TMSI. The channel is released. The MS is made to select cell B. A normal location updating is performed in cell B. The LOCATION UPDATING ACCEPT message contains an IMSI. The SS checks, by paging, that the MS has deleted its TMSI and responds to paging with IMSI.

Maximum duration of test

4 minutes

Expected sequence

Step	Direction	Message	Comments

1	   SS		The RF level of cell A is
				lowered until the MS selects cell B.
	 2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating.
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= CKSN1, "location area
				identification"= a, "mobile station 
				classmark 1" as given by the PICS 
				and "mobile identity"= TMSI1.
	 5	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= new TMSI 
				(=TMSI2), LAI= b.
	 6	MS -> SS	TMSI REALLOCATION CMP	
	 7	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link. The SS waits an 
				amount of time which is enough to 
				guarantee that the MS is in service.

8	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the new TMSI (= TMSI2).
	 9	MS -> SS	CHANNEL REQUEST	
	 10	SS -> MS	IMMEDIATE ASSIGNMENT	
	 11	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the new TMSI (= TMSI2).
	 12	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main signalling link.

13	   SS		The RF level of cell B is
				lowered until the MS selects
				cell A.
	 14	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 15	SS -> MS	IMMEDIATE ASSIGNMENT	
	 16	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= CKSN1, "location area
				identification"= b, "mobile station 
				classmark 1" as given by the PICS 
				and "mobile identity"= TMSI2.
	 17	SS -> MS	LOCATION UPDATING ACC	"Mobile identity" IE not included
	 18	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link. The SS waits an 
				amount of time which is enough to 
				guarantee that the MS is in 
				service.

19	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the TMSI (= TMSI2).
	 20	MS -> SS	CHANNEL REQUEST	
	 21	SS -> MS	IMMEDIATE ASSIGNMENT	
	 22	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the TMSI (=TMSI2).
	 23	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

24	   SS		The RF level of cell A is
				lowered until the MS selects
				cell B.
	 25	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 26	SS -> MS	IMMEDIATE ASSIGNMENT	
	 27	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= CKSN1, "location area
				identification"= a, "mobile station 
				classmark 1" as given by the PICS 
				and "mobile identity"= TMSI2.
	 28	SS -> MS	LOCATION UPDATING ACC	"Mobile identity" IE contains
				IMSI.
	 29	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
				the SS waits for the disconnection 
				of the main signalling link. The SS
				waits an amount of time which is
				enough to guarantee that the MS is
				in service.

30	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the old TMSI (= TMSI2).
	 31	   MS		The MS shall ignore this
				message. This is checked during 5 
				seconds.

32	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the IMSI.
	 33	MS -> SS	CHANNEL REQUEST	
	 34	SS -> MS	IMMEDIATE ASSIGNMENT	
	 35	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the IMSI.
	 36	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

#### 26.7.4.2	Location updating / rejected

##### 26.7.4.2.1	Location updating / rejected / IMSI invalid

###### 26.7.4.2.1.1	Conformance requirement

1)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station shall:

1.1	not perform normal location updating

1.2	not perform periodic location updating

1.3	not respond to paging with IMSI

1.4	not respond to paging with TMSI

1.5	reject any request from CM entity for MM connection other than for emergency call

1.6	not perform IMSI detach if powered down

2)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a Channel Request message with the establishment cause set to "emergency call" and include an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station shall delete the stored LAI, CKSN and TMSI.

Reference(s)

GSM 04.08 sections 4.4.4.7.

###### 26.7.4.2.1.2	Test purpose

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "IMSI unknown in HLR", "illegal MS" or "Illegal ME".

###### 26.7.4.2.1.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas of the same PLMN.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support for speech Yes/No

Foreseen final state of the MS

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

Test Procedure

The SS rejects a normal location updating with the cause value "IMSI unknown in HLR". The channel is released. The SS checks that the MS has entered the state MM-IDLE and the substate NO IMSI, i.e. does not perform normal location updating when a new cell of the same or another PLMN is entered, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if powered down.

The test is repeated with cause value "Illegal MS" and with cause value "Illegal ME".

Maximum duration of test

25 minutes

Expected sequence

The sequence is executed for execution counter k = 1, 2, 3

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B.

	1	SS		The RF level of cell A is
				lowered until the MS selects
				cell B.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "IMSI
				unknown in HLR" for k = 1,
				"Illegal MS" for k = 2,
				"Illegal ME" for k = 3.
	 6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell A.
	7	SS		The RF levels are then changed
				again to make the MS reselect
				the cell A.
	8	MS		The MS performs cell
				reselection according to
				procedure as specified in GSM
				05.08 (this however is not checked 
				until step 18). The MS shall not initiate
				an RR connection establishment on cell A
				or on cell B.

9	   SS		The SS waits at least 7 minutes
				for a possible periodic
				updating.
	 10 	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B

11	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell A.
				"Mobile identity" IE contains
				IMSI.
	 12	   MS		The MS shall ignore this
				message. This is verified
				during 3 seconds.

13	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell A.
				"Mobile identity" IE contains
				TMSI.
	 14	   MS		The MS shall ignore this
				message. This is verified
				during 3 seconds.

15	   MS		A MO CM connection is attempted.
	 16	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds

17	   MS		An emergency call is attempted.
	 18	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Emergency call. This message is 
				sent in cell A
	 19	SS -> MS	IMMEDIATE ASSIGNMENT	
	 20	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency
				call establishment. "Mobile identity" : type
				of identity is set to IMEI
	 21	SS -> MS	CM SERVICE ACCEPT	
	 22	MS -> SS	EMERGENCY SETUP
	 23	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.
	 24	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

25	   MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	 26	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

27	MS		Depending on what has been 
				performed in step 25 the MS is 
				brought back to operation.
	 28	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 29	SS -> MS	IMMEDIATE ASSIGNMENT	
	 30	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available, "mobile 
				station classmark 1" as given by 
				the PICS, "Mobile Identity"= IMSI, 
				"LAI"= deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
	 31	SS -> MS	AUTHENT REQUEST	"CKSN"= CKSN1
	 32	MS -> SS	AUTHENT RESPONSE	
	 33	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity"= TMSI
	 32	MS -> SS	TMSI REALLOCATION CMP
	 33	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

##### 26.7.4.2.2	Location updating / rejected / PLMN not allowed

###### 26.7.4.2.2.1	Conformance requirement

1)	If the network reject a location updating from the Mobile Station with the cause "PLMN not allowed" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not perform IMSI detach when switched off,

1.3	not perform IMSI attach when switched on in the same location area,

1.4	not perform normal location updating when in the same PLMN and when that PLMN is not selected manually.

1.5	reject any request from CM entity for MM connection other than for emergency call

2)	If the network rejects a location updating from the Mobile Station with the cause "PLMN not allowed" the Mobile Station shall:

2.1	perform normal location updating when a new PLMN is entered

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

3)	If the network rejects a location updating from the Mobile Station with the cause "PLMN not allowed" and if after that the PLMN from which this rejection was received, is manually selected, the Mobile Station shall perform a normal location updating procedure.

Reference(s)

GSM 04.08 section 4.4.4.7

###### 26.7.4.2.2.2	Test purpose

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "PLMN not allowed".

###### 26.7.4.2.2.3	Method of test

###### 26.7.4.2.2.3.1	Location updating / rejected / PLMN not allowed / test 1

Initial conditions

System Simulator:

One cell: C, belonging to PLMN1.

Two cells : A and B, belonging to different location areas a and b and belonging to PLMN2. PLMN2 is different from HPLMN and from PLMN1.

IMSI attach/detach is allowed in cells A and B but not in cell C.

The T3212 timeout value is 1/10 hour in cells A and B.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell C.

The MS is in manual mode for PLMN selection.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

The MS is automatically in automatic mode after switch on Yes/No

Support for speech Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell C.The MS is in automatic mode for PLMN selection.

Test Procedure

The SS rejects a normal location updating with the cause value "PLMN not allowed". The channel is released. The SS checks that the MS does not perform periodic updating, does not perform IMSI detach, does not perform IMSI attach if activated in the same location area, rejects any request for CM connection establishment other than emergency call, accepts a request for an emergency call and performs normal location updating only when a new PLMN is entered.

Maximum duration of test

12 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B.

	1	MS		The MS is switched off (or power is removed).
	2	SS		The SS activates cells A and B and

deactivates cell C.
				Cell B has a level higher by
				at least 5 dB than cell A
	3	MS		The MS is switched on. (or power is reappliesd)
				If necessary the MS is put in manual selection
				mode. The MS shall
				offer the new PLMN as available to 
				the user. The PLMN is manually selected.
	 4	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 5	SS -> MS	IMMEDIATE ASSIGNMENT	
	 6	MS -> SS	LOCATION UPDATING REQ	
	 7	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= PLMN not allowed.
	 8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

9	   SS		The SS waits for a possible
				periodic updating for 7 minutes.
	 10	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

11	   MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	 12	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

13	   MS		Depending on what has been 
				performed in step 11 the MS is 
				brought back to operation. The MS is
				not made to select PLMN 2.
	 14	   MS		The MS shall not initiate an RR
				connection establishment. This is checked
				during 3 seconds.

The following message are sent and shall be
				received on cell A.

	 15	   SS		The RF level of cell B is
				lowered to make the MS reselect cell A.
	 16	   MS		No access to the network shall
				be registered by the SS within one minute.

17	   MS		An emergency call is attempted.
	 18	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Emergency call
	 19	SS -> MS	IMMEDIATE ASSIGNMENT	
	 20	MS -> SS	CM SERVICE REQUEST	"CM service type"= Emergency call
				establishment.
	 21	SS -> MS	CM SERVICE ACCEPT	
	 22	MS -> SS	EMERGENCY SETUP
	 23	SS -> MS	RELEASE COMPLETE	Cause IE : "unassigned number".
	 24	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

25	   MS		A MO CM connection is attempted.
	 26	   MS		The MS shall not initiate an RR connection
				establishment. This is checked
				during 3 seconds

The following messages are sent and shall be
				received on cell C

	 27	   MS		The MS is switched off.
	 28	   SS		The SS activates cell C
				and deactivates cells A and B
	 29	   MS		The MS is switched on. If necessary
				the MS is placed into the automatic mode.
	 30	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 31	SS -> MS	IMMEDIATE ASSIGNMENT	
	 32	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"LAI"= deleted LAI (the MCC and MNC hold
				the values of PLMN2, the LAC is coded FFFE)
				"mobile identity"= IMSI
	 33	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= TMSI.
	 34	MS -> SS	TMSI REALLOCATION CMP	
	 35	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

###### 26.7.4.2.2.3.2	Location updating / rejected / PLMN not allowed / test 2

Initial conditions

System Simulator:

One cell C, belonging to PLMN1.

Two cells A and B, belonging to different location areas a and b and belonging to PLMN2. PLMN2 is different from HPLMN.

IMSI attach/detach is allowed in cells A and B but not in cell C.

The T3212 timeout value is 1/10 hour in cells A and B.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell C.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

The MS is automatically in automatic mode after switch on Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle, updated"on cell C.

The MS is in automatic mode for PLMN selection.

Test Procedure

The SS rejects a normal location updating with the cause value "PLMN not allowed". The channel is released. Then the PLMN from which this rejection was received is manually selected and the SS checks that a normal location updating is performed.

Maximum duration of test

5 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B

	 1	MS		The MS is switched off (or power is removed).
	 2	SS		The SS activates cells A and B and
				deactivates cell C.
				Cell B has a level higher by at least
				5 dB than cell A
	3	MS		The MS is switched on (or power is reapplied).
	3a	MS		If the MS is in manual mode, it shall 
				offer the new PLMN as available to 
				the user. In this case the PLMN is manually 
				selected.
	 4	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 5	SS -> MS	IMMEDIATE ASSIGNMENT	
	 6	MS -> SS	LOCATION UPDATING REQ	
	 7	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= PLMN not
				allowed.
	 8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

9	   MS		The MS is made to search for
				PLMNs and the PLMN indicated 
				by the SS is manually selected
	 10	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 11	SS -> MS	IMMEDIATE ASSIGNMENT	
	 12	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"LAI"= deleted LAI (the MCC and MNC hold
				the values of PLMN2, the LAC is coded FFFE)
				"mobile identity"= IMSI

	 13	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell C.

 14	   MS		The MS is switched off.
	 15	   SS		The SS activates cell C
				and deactivates cells A and B
	 16	   MS		The MS is switched on.
				If necessary, the MS is put
				into the automatic mode.
	 17	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 18	SS -> MS	IMMEDIATE ASSIGNMENT	
	 19	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"LAI"= deleted LAI (the MCC and MNC hold
				the values of PLMN2, the LAC is coded FFFE)
				"mobile identity"= IMSI
	 20	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= TMSI.
	 21	MS -> SS	TMSI REALLOCATION CMP	
	 22	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

##### 26.7.4.2.3	Location updating / rejected / location area not allowed

###### 26.7.4.2.3.1	Conformance requirement

1)	If the network rejects a location updating from the Mobile Station with the cause "Location Area not allowed" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not respond to paging with TMSI,

1.3	reject any request from CM entity for MM connection other than for emergency call,

1.4	not perform IMSI detach.

2)	If the network rejects a location updating from the Mobile Station with the cause "Location Area not allowed" the Mobile Station shall:

2.1	perform normal location updating when a new location area is entered,

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

2.3	delete the list of forbidden LAs after switch off (power off).

Reference(s)

GSM 04.08 sections 4.4.4.7

###### 26.7.4.2.3.2	Test purpose

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "Location Area not allowed".

To test that the MS deletes the list of forbidden LAs after switch off (power off).

###### 26.7.4.2.3.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell A.

Related PICS/PIXIT statement(s)

Switch off on button Yes/No

Method to clear the list of forbidden location areas periodically.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell A.

Test Procedure

The SS rejects a normal location updating with the cause value "Location Area not allowed". The channel is released. The SS checks that the MS does not perform periodic updating, does not respond to paging with TMSI, rejects any requests from CM entities for MM-connections except emergency calls, does not perform IMSI detach, performs normal location updating when a new location area is entered and deletes the list of forbidden LAs when switched off.

Different types of MS may use different methods to periodically clear the list of forbidden location areas (eg. every day at 12am). If the list is cleared while the test is being run, it may be necessary to re-run the test.

Maximum duration of test

12 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B

	 1	   SS		The RF level of cell A is
				lowered so that cell B is selected, while keeping
				the C1 and C2 of cell A greater than 10.
	 2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	LOCATION UPDATING REQ	
	 5	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= "Location
				Area not allowed".
	 6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

7	   SS		The SS waits for a possible
				location updating for 7 minutes.
	 8	   MS		The MS shall not initiate an
				RR-connection establishment either on
				cell A or cell B.

9	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell B.
				"Mobile identity"= TMSI.
	 10	   MS		The MS shall ignore this message.
				This is checked during 3 seconds.

11	   MS		A MO CM connection is attempted.
	 12	   MS		The MS shall not initiate an RR 	connection
				establishment on cell A or cell B.
				This is checked during 3 seconds

13	   MS		An emergency call is attempted.
	 14	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Emergency call.
	 15	SS -> MS	IMMEDIATE ASSIGNMENT	
	 16	MS -> SS	CM SERVICE REQUEST	"CM service type":
				Emergency call establishment.
	 17	SS -> MS	CM SERVICE ACCEPT	
	 18	MS -> SS	EMERGENCY SETUP
	 19	SS -> MS	RELEASE COMPLETE	Cause: "unassigned number".
	 20	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

21	   MS		If possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	 22	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B (check
				for IMSI detach)
				This is checked during 3 seconds.

23	MS		Depending on what has been 
				performed in step 21 the MS is 
				brought back to operation.
	 24	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 25	SS -> MS	IMMEDIATE ASSIGNMENT	
	 26	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"LAI"= deleted LAI, "mobile 
				identity"= IMSI (This checks the
				deletion of the forbidden lists)
 	27	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= "Location
				Area not allowed".
	28	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell A

	 29	   SS		The RF level of cell B is
				lowered until the MS selects
				cell A.
	 30	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 31	SS -> MS	IMMEDIATE ASSIGNMENT	
	 32	MS -> SS	LOCATION UPDATING REQ	
	 33	SS -> MS	AUTHENT REQUEST	
	 34	MS -> SS	AUTHENT RESPONSE
	 35	SS -> MS	LOCATION UPDATING ACC	Mobile identity= TMSI
	 36	MS -> SS	TMSI REALLOCATION CMP
	 37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

##### 26.7.4.2.4	Location updating / rejected / national roaming

###### 26.7.4.2.4.1	Conformance requirement

1)	If the network rejects a location updating from the Mobile Station with the cause "National roaming not allowed in this area" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not respond to paging with TMSI,

1.3	reject any request from CM entity for MM connection other than for emergency call,

1.4	not perform IMSI detach.

2)	If the network rejects a location updating from the Mobile Station with the cause "National roaming not allowed in this area" the Mobile Station shall:

2.1	perform normal location updating when a new location area is entered,

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

2.3	periodically search for its HPLMN. [editorial note : no test purpose checks that]

3)	The mobile station shall reset the list of "forbidden location areas for national roaming" when powered down or when the SIM is removed.

4)	The MS shall be capable of storing at leat 6 entries in the list of "forbidden forbidden location areas for national roaming".

Reference(s)

GSM 04.08 section 4.4.4.7

###### 26.7.4.2.4.2	Test purposes

Test purpose 1

To test that on receipt of a rejection using the National Roaming cause code, the MS ceases trying to update on that cell, that this situation continues for at least one periodic location interval period, and that the corresponding list is re-set by powering down the MS.

Test purpose 2

To test that if no cell is available, the MS does not answer to paging with TMSI, rejects a request from CM entity other than for emergency calls.

Test purpose 3

To test that at least 6 entries can be held in the list of "forbidden location areas for national roaming" (the requirement in TS GSM 04.08 is to store at least 10 entries. This is not fully tested by the third procedure).

Test purpose 4

To test that if a cell of the Home PLMN is available then the MS returns to it in preference to any other available cell.

Test purpose 5

To test that if the SIM is removed the list of "forbidden location areas for national roaming"is cleared.

###### 26.7.4.2.4.3	Method of test

Initial conditions

The initial conditions shall be met before each of the different procedures.

System Simulator:

For procedures 1, 2, 3 and 5: Two cells A and B, belonging to different location areas of the same PLMN with LAI a and b. The MCC of that PLMN is the same as that of the HPLMN. The MNC of that PLMN is different from that of the HPLMN.

For procedure 4: three cells A, B, C of the same PLMN which is not the HPLMN with 3 different location area codes. Cells should differ in signal strength by 10 dB with cell A being the strongest and cell C the weakest. There should be a 20 dB range between A and C. A should be set to a level of - 40 dBm.

IMSI attach/detach is allowed in every cell.

The T3212 timeout value is 1/10 hour in every cell.

Mobile Station:

Procedures 1, 2, 3 and 5 : The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell B.

Procedure 4 : The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

The list of "forbidden location areas for national roaming" shall be empty (this may be achieved by either removing the SIM or switching the MS OFF then ON or powering down the MS depending on PICS).

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

Method to clear the list of location areas for national roaming periodically.

The MS is automatically in automatic mode after switch on Yes/No

Foreseen final state of the MS

Procedures 1 and 5 : The MS has no valid TMSI and no CKSN. It is "idle updated" on cell A.

Procedure 2 and 3: The MS has no valid TMSI and no CKSN. It is in the 'limited service' state on cell A.

Procedure 4 : The MS has no valid TMSI and no CKSN. It is "idle updated" on cell C.

Test Procedures

Procedure 1 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not perform periodic location updating procedure. The MS is turned off and then on. The SS checks that the MS performs location updating on the cell on which its location update request had been rejected (this checks that the LA is not the forbidden list after switch on). This procedure is performed another time but the deletion of the list is checked while removing the SIM (instead of turning off the MS).

Procedure 2 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not answer to a paging message with TMSI, rejects a request from CM entity but supports an emergency call.

Procedure 3 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". This is done for 6 different location areas. Then the SS checks that the MS does not attempt to begin a location updating procedure on the non-allowed location areas.

Procedure 4 :

The SS accepts a normal location updating on a cell not belonging to the HPLMN. Then when the MS attemps to perform a periodic location updating to this cell, the SS rejects this location updating with the cause value "National Roaming not allowed in this area". Two cells are then available, one belonging to the HPLMN but with the weakest level. It is checked that the MS returns to its HPLMN.

Procedure 5 : If SIM removal is possible without powering down :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not perform periodic location updating procedure. The SIM is removed and inserted in the MS. The SS checks that the MS performs location updating on the cell on which its location update request had been rejected (this checks that the LA is not the forbidden list after switch on).

Different types of MS may use different methods to periodically clear the list of forbidden areas (eg. every day at 12am) for national roaming. If the list is cleared while the test is being run, it may be necessary to re-run the test.

Maximum duration of test

Procedures 1 and 5: 12 minutes each

Procedure 2: 6 minutes

Procedure 3: 17 minutes

Procedure 4: 16 minutes

Expected sequence

The following procedure is used during the test:

Change_LAI (x):

-	the purpose of this procedure is to change the value of Location Area Identifier of cell x

-	The Location Area Identifier of cell x shall be changed. The code shall be chosen arbitrarily but shall be different from any previously used in this procedure. The code shall have the same MCC as the Home PLMN and shall not have the same MNC as the Home PLMN.

Procedure 1

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A.

	1	SS		The RF level of cell B is
					lowered until cell B is no
				more suitable and the the MS selects cell A.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
					Roaming not allowed in this 
					location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

7	SS		The SS waits at least 7 minutes
					for a possible location updating
	8 	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

9	MS		If possible (see PICS) the MS is 
				switched off. Otherwise 
				if possible the power is removed.
	10	MS		Depending on what has been 
				performed in step 9 the MS is 
				brought back to operation and placed
				in a automatic mode.
	11	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	12	SS -> MS	IMMEDIATE ASSIGNMENT	
	13	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= normal
	14	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	15	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main signalling link.

Procedure 2

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The RF level of cell B is
				lowered until the MS selects
				cell A. The level of cell B
				shall be such that cell B is
				suitable for cell selection.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message is sent on 
				cell A.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National"
				Roaming not allowed in this 
				location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell B

	7	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	8	SS -> MS	IMMEDIATE ASSIGNMENT	
	9	MS -> SS	LOCATION UPDATING REQ	
	10	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	11	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	12	SS		The SS waits for a possible 
				location updating 
				procedure on both cells A and B for 
				2 minutes.
	13	MS		The MS shall not initiate an RR connection 
				establishment  on  cell A or on cell B
				within 2 minutes after the end of step 11

14	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI.This message is
				sent on cell A and on cell B
	15	  MS		The MS shall not initiate an RR connection
				on cell A or on cell B. This is checked
				during 3 seconds.

16	   MS		A MO CM connection is attempted.
	17	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

The following messages are sent and shall be
				received on cell A

				Steps 20 to 27 are performed if the MS
				supports speech.

	18	MS		An emergency call is attempted.
	19	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Emergency call.
	20	SS -> MS	IMMEDIATE ASSIGNMENT	
	21	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency
				call establishment.
	22	SS -> MS	CM SERVICE ACCEPT	
	23	MS -> SS	EMERGENCY SETUP
	24	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.
	25	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Procedure 3

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The RF level of cell B is
				lowered until the MS selects
				cell A. The level of cell B
				shall be such that cell B is
				suitable for cell selection.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : 
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell B

	9	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	10	SS -> MS	IMMEDIATE ASSIGNMENT	
	11	MS -> SS	LOCATION UPDATING REQ	
	12	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	13	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	14	SS		Change_LAI (A) within 5 seconds 
				after step 12

The following messages are sent and shall be
				received on cell A

	17	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	18	SS -> MS	IMMEDIATE ASSIGNMENT	
	19	MS -> SS	LOCATION UPDATING REQ	
	20	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	21	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	22	SS		Change_LAI (B) within 5 seconds 
				after step 20

The following messages are sent and shall be
				received on cell B

	25	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	26	SS -> MS	IMMEDIATE ASSIGNMENT	
	27	MS -> SS	LOCATION UPDATING REQ	
	28	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	29	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	30	SS		Change_LAI (A) within 5 seconds 
				after step 28

The following messages are sent and shall be
				received on cell A

	33	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	34	SS -> MS	IMMEDIATE ASSIGNMENT	
	35	MS -> SS	LOCATION UPDATING REQ	
	36	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	38	SS		Change_LAI (B) within 5 seconds 
				after step 36

The following messages are sent and shall be
				received on cell B

	41	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	42	SS -> MS	IMMEDIATE ASSIGNMENT	
	43	MS -> SS	LOCATION UPDATING REQ	
	44	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	45	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	46	SS		The SS waits for a possible 
				location updating 
				procedure on both cells A and B for 
				7 minutes.
	47	MS		The MS shall not initiate an RR connection 
				establishment on cell A or on cell B
				within 7 minutes after the end of step 45

Procedure 4

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The SS waits for a 
					periodic location updating 
					procedure on cell A for 7 minutes 
					after the initial conditions have 
					been established.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= periodic
	5	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

	7	SS		The location area identity of cell 
					C shall be changed to that of a 
					location area in the Home PLMN

	8	SS		The SS waits for a  
					periodic location updating 
					procedure on cell A for 
					7 minutes.
	9	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
					This message is sent on 
					cell A within 7 minutes after the end
					of step 6.
	10	SS -> MS	IMMEDIATE ASSIGNMENT	
	11	MS -> SS	LOCATION UPDATING REQ	"Location updating type" = periodic
	12	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
					Roaming not allowed in this 
					location area"
	13	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

The following messages are sent and shall be
				received on cell C

	16	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	17	SS -> MS	IMMEDIATE ASSIGNMENT	
	18	MS -> SS	LOCATION UPDATING REQ	
	19	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	20	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

Procedure 5

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The RF level of cell B is
					lowered until the MS selects cell A.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
					Roaming not allowed in this 
					location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

7	SS		The SS waits at least 7 minutes
					for a possible location updating
	8 	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

9	MS		The SIM is removed
	10	MS		The SIM is inserted into the ME.
	11	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	12	SS -> MS	IMMEDIATE ASSIGNMENT	
	13	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= normal
	14	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	15	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

Specific message contents:

None.

#### 26.7.4.3	Location updating / abnormal cases

##### 26.7.4.3.1	Location updating / abnormal cases / random access fails

###### 26.7.4.3.1.1	Conformance requirement

If during the RR connection establishment phase of a normal location updating procedure, channel requests are not answered by the network, the Mobile Station shall:

1.	send (Max-Retrans+1) Channel Request messages

2.	not try to establish a connection during a period of T3213

3.	then perform a normal location updating procedure as it is still necessary.

4.	not repeat the complete procedure if the original cause of the location updating procedure has disappeared.

Reference(s):

GSM 04.08 section 4.4.4.9 and GSM 05.08 section 6.6.2

###### 26.7.4.3.1.2	Test purpose

To verify that when during the RR connection establishment phase of a location updating procedure, channel requests are not answered by the network, after expiry of T3213 (= 4s in Phase 2) the complete procedure is repeated if still necessary.

###### 26.7.4.3.1.3	Method of test

Initial conditions

System Simulator:

Two cells: A and B of the same PLMN, belonging to different location areas with LAI a and b.

The RF power level of cell B is higher than the one of cell A.

IMSI attach/detach is not allowed in both cells.

The T3212 timeout value is set to infinite in both cells.

Mobile Station:

The MS has a valid TMSI. It is "Idle updated" on cell B.

Related PICS/PIXIT statement(s)

None

Foreseen final state of the MS

The MS is "Idle updated" on cell A.

Test Procedure

The SS causes a random access failure in the MS during a normal location updating procedure. After the expiry of T3213 and when the cell reselection procedure is finished the MS will try to restart the normal location updating procedure.

The test is repeated but the original cause of the location updating procedure has disappeared. The SS then checks that the MS will not restart the location updating procedure.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	2	MS -> SS	CHANNEL REQUEST	Establishment cause:
					Location updating
				This message is sent by the MS 
				(Max_Retrans + 1) times
	3	   SS		The SS waits for 3 seconds
	4	   MS		The MS shall not send any layer 3
				message during this time.
	5	MS -> SS	CHANNEL REQUEST	Establishment cause:
					Location updating
				The time difference between this message
				and the last CHANNEL REQUEST sent in
				step 2 shall be in the range 4 s - 6 s.
	6	SS -> MS	IMMEDIATE ASSIGNMENT	
	7	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b,
				mobile station classmark 1 and
				mobile identity = TMSI.
	8	SS -> MS	LOCATION UPDATING ACC	Optional IE Mobile Identity not
				included
	9	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell B

	10	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	11	MS -> SS	CHANNEL REQUEST	Establishment cause:
					Location updating
				This message is sent by the MS 
				(Max_Retrans + 1) times
	12	SS		Immediately after the end of
				step 11 the RF level of cell A is
				set sufficiently high to ensure
				that cell A is suitable as defined
				in GSM 05.08 section 6.6.2
	13	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 6 seconds.

Specific message contents:

None.

##### 26.7.4.3.2	Location updating / abnormal cases / attempt counter less or equal to 4, LAI different

[Editorial Note:

- it is not checked that the MS does not perform periodic location updating and IMSI attach procedures

]

###### 26.7.4.3.2.1	Conformance requirement

1)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure, if the attempt counter is smaller than 4 and after expiry of T3211, the Mobile Station shall resend its Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "normal location updating".

2)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall:

2.1	not answer to paging with the previously allocated TMSI

2.2	not perform the IMSI detach procedure, when switched off.

3)	When a failure such as case e) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure and when an emergency call establishment is requested by the user the Mobile Station, if it supports speech, shall send a CM Service Request message with CM Service Type IE set to "emergency call establishment", CKSN IE set to "no key available" and Mobile Identity IE set to its IMSI and after acceptance by the network it shall send an Emergency Setup message.

4)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall use a request from CM entity other than emergency call as a trigger for a normal location updating procedure and shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

5)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall answer to paging with IMSI and shall send a Paging Response message with CKSN IE set to "no key available" and Mobile Identity IE set to its IMSI.

6)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall perform a normal location updating procedure as soon as it enters a new cell.

References

GSM 04.08 section 4.4.4.2 and 4.4.4.9 and GSM 05.08 section 6.6.2

###### 26.7.4.3.2.2	Test purpose

To verify that the MS performs normal location updating procedures when its attempt counter is smaller than 4.

To check that the MS does not perform the IMSI detach procedure when "idle not updated".

To verify that when "idle not updated" the MS can perform an emergency call.

To verify that when "idle not updated" the MS uses requests from CM layer other than emergency call as triggering of a normal location updating procedure.

To verify that the MS performs a normal location updating procedure if it enters a new cell while being "idle not updated".

###### 26.7.4.3.2.3	Method of test

Initial conditions

System Simulator:

Two cells: A and B of the same PLMN, belonging to different location areas with LAI a and b.

ATT flag shall be set to IMSI attach/detach allowed.

Mobile Station:

The MS is "idle updated" on cell A. A valid CKSN value is stored in the SIM and is noted "initial CKSN". A TMSI is allocated.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Foreseen final state of the MS

The MS is "Idle updated" on cell A with a valid CKSN and a TMSI.

Test Procedure

The MS is made to perform a normal location updating procedure. Four types of failure cases are triggered:

-	sending of a Location Updating Reject with cause randomly chosen between all defined cause values except 2, 3, 6, 11, 12 and 13 (which trigger a different action) (case g of GSM 04.08 section 4.4.4.9),

-	RR-connection failure (case d),

-	sending of a CHANNEL RELEASE message before the normal end of the procedure (case f),

-	T3210 timeout (case e).

As there is no stored LAI or the stored LAI is different from the broadcast LAI, and the attempt counter in the MS shall be lower than 4, the MS enters the state MM-IDLE and substate ATTEMPTING TO UPDATE and waits for T3211 seconds before trying again a location updating procedure.

Then the behaviour of the MS in the MM-IDLE ATTEMPTING TO UPDATE SERVICE state is checked, that is:

-	not answer to paging with TMSI,

-	not perform an IMSI detach procedure,

-	support request for emergency call,

-	use requests from CM layer other than emergency call as triggering of a normal location updating procedure,

-	perform normal location updating procedure when a new cell is entered.

Maximum duration of test

6 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	2	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	5	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value
				randomly chosen:
				* in Table 10.44 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being 
				excluded
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	7	MS		The MS shall not initiate an RR connection 
				establishment on cell A or on cell B
				 during T3211 seconds at least after
				the channel release.
	8	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	9	SS -> MS	IMMEDIATE ASSIGNMENT
	10	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.

	11	SS		The SS stops any RF transmission on 
				the dedicated channel. The SS waits 
				until there is no more SACCH frames 
				in the uplink direction

	12	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				 during T3211+RadioLinkTimeout seconds at 
				least after the channel release.
	13	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	14	SS -> MS	IMMEDIATE ASSIGNMENT

	15	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	16	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	17	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				 during T3211 seconds at least after
				the channel release.
	18	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	19	SS -> MS	IMMEDIATE ASSIGNMENT
	20	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	21	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	22	MS -> SS	AUTHENT RESPONSE
	23	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	24	MS -> SS	TMSI REALLOCATION CMP
	25	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle updated" in cell B

The following messages are sent and shall be received on cell A

26	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	27	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	28	SS -> MS	IMMEDIATE ASSIGNMENT	
	29	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	30	SS		chooses randomly one among the 
				three following steps: (5 and 6) or 
				11 or 16
	31	SS		performs the chosen step
	32	SS -> MS	PAGING REQUEST TYPE 1	Mobile identity = old TMSI of the 
				MS.
				This message is sent continuously to
				the MS during 8 seconds 
	33	SS		The SS checks that there is no 
				answer from the MS during 12 
				seconds
	33a	SS		If during steps 33 and 34 the MS 
				attempts to perform a location 
				updating procedure the SS will 
				perform steps 30 and 31 and then 
				continue the procedure.
	34	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) mobile switch off 
				is performed. Otherwise the power 
				is removed.
	35	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 30 seconds.
	36	MS		Depending on what has been 
				performed in step 34 the MS is 
				brought back to operation.
	37	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	38	SS -> MS	IMMEDIATE ASSIGNMENT
	39	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	40	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	41	MS -> SS	AUTHENT RESPONSE
	42	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	43	MS -> SS	TMSI REALLOCATION CMP
	44	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle updated" in cell A

45	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	46	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	47	SS -> MS	IMMEDIATE ASSIGNMENT	
	48	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	49	SS -> MS	AUTHENT REQUEST	
	50	MS->SS	AUTHENT RESPONSE	steps 49 and 50 are performed N 
				times. N shall be chosen in such a 
				way that T3210 expires.
	51	SS		The SS checks that there is no more 
				activity from the MS on the 
				channel.
	52	MS		The MS is made to perform an 
				emergency call
	53	MS -> SS	CHANNEL REQUEST	Establishment cause: Emergency call
	54	SS -> MS	IMMEDIATE ASSIGNMENT	
	55	MS -> SS	CM SERVICE REQUEST	CM service type = Emergency call 
				establishment; CKSN = no key 
				available; Mobile Identity = IMSI
	56	SS -> MS	CM SERVICE ACCEPT
	57	MS -> SS	EMERGENCY SETUP
	58	SS -> MS	RELEASE COMPLETE	Cause = unassigned number
	59	SS -> MS	CHANNEL RELEASE
	60	MS -> SS	CHANNEL REQUEST	Establishment cause : Location updating
				The SS will wait at most 15 
				seconds for this message
	61	SS -> MS	IMMEDIATE ASSIGNMENT
	62	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	63	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	64	MS -> SS	AUTHENT RESPONSE
	65	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	66	MS -> SS	TMSI REALLOCATION CMP
	67	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle updated" in cell B

68	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	69	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	70	SS -> MS	IMMEDIATE ASSIGNMENT	
	71	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	72	SS		chooses randomly one among the 
				three following steps: (5 and 6), 
				11 or 16
	73	SS		performs the chosen step
	74	MS		A MO CM connection is attempted
				before T3211 expiry
	75	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	76	SS -> MS	IMMEDIATE ASSIGNMENT
	77	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	78	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	79	MS -> SS	TMSI REALLOCATION CMP
	80	SS -> MS	CHANNEL RELEASE	
				Steps 81 to 84 are optional as the 
				MS may have memorized the request for
				CM connection attempt
				Wait 10 s to decide whether to go directly to
				step 85.
	81	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Not checked
	82	SS -> MS	IMMEDIATE ASSIGNMENT
	83	MS -> SS	CM SERVICE REQUEST	CKSN= 
				no key available,
				Mobile identity= TMSI
	84	SS -> MS	CHANNEL RELEASE	After the sending of this message,
				the SS waits for the disconnection of
				the main signalling link.
				MS is now "idle updated" in cell A

85	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	86	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	87	SS -> MS	IMMEDIATE ASSIGNMENT	
	88	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	89	SS		chooses randomly one among the 
				three following steps: (5 and 6)
				or 11 or 16
	90	SS		performs the chosen step and notes 
				the time when the CHANNEL RELEASE 
				message is sent
	91	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	92	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
				The time interval between the last 
				CHANNEL RELEASE message and this 
				message shall be less than 
				0.8*T3211
	93	SS -> MS	IMMEDIATE ASSIGNMENT	
	94	MS -> SS	LOCATION UPDATING REQ	location updating type= normal, 
				CKSN= no key available , 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	95	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	96	MS -> SS	AUTHENT RESPONSE
	97	SS -> MS	LOCATION UPDATING ACC	Mobile identity= TMSI
	98	MS -> SS	TMSI REALLOCATION CMP
	99	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "Idle, updated" in cell A

Specific message contents

None.

##### 26.7.4.3.3	Location updating / abnormal cases / attempt counter equal to 4

[Editorial Note:

- it is not tested that the MS does not answer to paging with TMSI

- in phase 1 Procedure 2 ended with "the MS is switched off" and it did not appear in the requirement that the MS would send an IMSI detach indication: it has been added and now the MS is brought back to operation at the end of the test

]

###### 26.7.4.3.3.1	Conformance requirement

1)	When four failures such as cases d) to g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure the Mobile Station shall:

1.1	not perform normal location updating procedures.

1.2	perform periodic location updating after T3212 expiry by sending a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "periodic updating".

1.3	if the periodic location updating was unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

2)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure the Mobile Station, if it supports speech, shall be able to perform an emergency call i.e. the Mobile Station is able to send a CM Service Request message with the CM Service Type IE set to "emergency call establishment", CKSN IE set to "no key is available" and Mobile Identity IE set to its IMSI and then send an Emergency Setup message.

3)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure

3.1	the Mobile Station shall use a request from CM entity for MM connection for a service other than emergency call as a trigger for a normal location updating procedure and shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

3.2	after a location updating triggered by a request from the CM layer which was .unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

4)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure

4.1	the Mobile Station shall perform a normal location updating procedure if it enters a new cell.

4.2	if this location updating is unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

References

GSM 04.08 section 4.4.4.9 and GSM 05.08 section 6.6.2

###### 26.7.4.3.3.2	Test purpose

To verify that the MS performs only periodic location updating procedures, that it does not perform normal location updating procedures when its attempt counter has reached value 4 and that the MS reset its attempt counter after a timer T3212 expiry.

To verify that the MS still follows the MM-IDLE ATTEMPTING TO UPDATE state requirements after its attempt counter has reached value 4.

To verify that the attempt counter is reset in the cases where it has to be done.

###### 26.7.4.3.3.3	Method of test

Initial conditions

System Simulator:

Two cells: A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

T3212 is set to 6 minutes.

Mobile Station:

The MS is "Idle updated" on cell B with a valid CKSN and a TMSI.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

Foreseen final state of the MS

The MS is "Idle updated" on cell A with a valid CKSN and a TMSI.

Test Procedure

The MS is made to perform a normal location updating. The SS triggers a failure in this procedure. After T3211 expiry the MS will try again the location updating procedure. The SS triggers again a failure. This is done again 2 times. At this point the attempt counter shall be equal to 4. It is then checked that T3212 has been started and that at its expiry the MS will try a periodic location updating procedure. It is verified that the MS has reset its attempt counter after timer T3212 expiry.

Then it is checked that, when the attempt counter has reached the value of 4, the MS is in the MM-IDLE state and ATTEMPTING TO UPDATE substate, that is:

-	not perform an IMSI detach procedure,

-	support request for emergency call,

-	use requests from CM layer other than emergency call as triggering of a normal location updating procedure,

-	perform normal location updating procedure when a new cell is entered.

Maximum duration of test

20 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be 
				received on cell A
	1	MS		The RF level of cell B is lowered
				until the MS selec_its cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	2	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	5	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value
				randomly chosen:
				* in Table 10.44 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being
				excluded
	6	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	7	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B 
				within T3211 + RadioLinkTimeOut
				seconds after the SS stops
				RF transmission
	8	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	9	SS -> MS	IMMEDIATE ASSIGNMENT
	10	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.

	11	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there is no more SACCH in the
				uplink
	12	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at least after the
				channel release
	13	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	14	SS -> MS	IMMEDIATE ASSIGNMENT
	15	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	16	SS -> MS	AUTHENT REQUEST

17	MS -> SS	AUTHENT RESPONSE	these steps (16 and 17) are 
				performed N times.
				N shall be chosen in such a way 
				that T3210 expires.
	18	MS		The MS shall cease transmission and then
				shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at least after the
				expiry of T3210
	19	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	20	SS -> MS	IMMEDIATE ASSIGNMENT

	21	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	22	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	23	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3212 (tolerance 
				-15s; 45s) at
				least after the channel release.
	24	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	25	SS -> MS	IMMEDIATE ASSIGNMENT	
	26	MS -> SS	LOCATION UPDATING REQ	location updating type : not checked
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	27	SS -> MS	LOCATION UPDATING REJ	IE Reject cause = #17 "network 
				failure"
	28	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	29	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	30	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	31	SS -> MS	IMMEDIATE ASSIGNMENT	
	32	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	33	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	34	MS -> SS	AUTHENT RESPONSE
	35	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	36	MS -> SS	TMSI REALLOCATION CMP
	37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle, updated" in cell A

38	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	39	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	40	SS -> MS	IMMEDIATE ASSIGNMENT	
	41	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	42	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value
				randomly chosen:
				* in Table 10.44 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being
				excluded
	43	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	44	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	45	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	46	SS -> MS	IMMEDIATE ASSIGNMENT
	47	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.

	48	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there is no more SACCH in the
				uplink
	48a	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				within T3211 + RadioLinkTimeOut seconds 
				after the SS stops RF Transmission.
	49	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	50	SS -> MS	IMMEDIATE ASSIGNMENT
	51	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	52	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	53	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.

54	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	55	SS -> MS	IMMEDIATE ASSIGNMENT	
	56	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	57	SS		chooses randomly one among the 
				three following steps: (42 and 
				43) or 48 or 52
	58	SS		performs the chosen step 
	59	MS		If the MS supports speech, it is made to perform
				an emergency call
	60	MS -> SS	CHANNEL REQUEST	Establishment cause: Emergency call
	61	SS -> MS	IMMEDIATE ASSIGNMENT	
	62	MS -> SS	CM SERVICE REQUEST	CM service type = Emergency call 
				establishment; CKSN = no key 
				available; Mobile Identity = IMSI
	63	SS -> MS	CM SERVICE ACCEPT
	64	MS -> SS	EMERGENCY SETUP
	65	SS -> MS	RELEASE COMPLETE	Cause = unassigned number
	66	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	67	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	68	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.
	69	MS		Depending on what has been 
				performed in step 67 the MS is 
				brought back to operation.
	70	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	71	SS -> MS	IMMEDIATE ASSIGNMENT
	72	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	73	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	74	MS -> SS	AUTHENT RESPONSE
	75	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	76	MS -> SS	TMSI REALLOCATION CMP
	77	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle, updated" in cell B

78	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	79	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	80	SS -> MS	IMMEDIATE ASSIGNMENT	
	81	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	82	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value
				randomly chosen:
				* in Table 10.44 of GSM 04.08, 
				causes #2, #3, #6, #11 #12, and #13 being
				excluded
	83	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	84	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	85	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	86	SS -> MS	IMMEDIATE ASSIGNMENT
	87	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.

	88	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there is no more SACCH in the
				uplink
	88a	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				within T3211 +RadioLinkTimeout
				seconds after the SS stops RF transmission
				.
	89	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	90	SS -> MS	IMMEDIATE ASSIGNMENT
	91	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	92	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	93	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.

94	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	95	SS -> MS	IMMEDIATE ASSIGNMENT	
	96	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	97	SS		chooses randomly one among the 
				three following steps: (42 and 
				43) or 48 or 52
	98	SS		performs the chosen step 

	99	MS		A MO CM connection is attempted
	100	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	101	SS -> MS	IMMEDIATE ASSIGNMENT
	102	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	103	SS		chooses randomly one among the 
				three following steps: (42 and 
				43) or 48 or 52
	104	SS		performs the chosen step 
	105	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	106	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	107	SS -> MS	IMMEDIATE ASSIGNMENT
	108	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	109	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	110	MS -> SS	AUTHENT RESPONSE
	111	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	112	MS -> SS	TMSI REALLOCATION CMP
	113	SS -> MS	CHANNEL RELEASE	MS is now "idle, updated" in cell A
				The MS may or may not have memorized the
				request for CM connection. The steps 114 to 
				117a are therefore optional for the MS.
				The SS waits 10 second whether to decide to
				go directly to step 118
	114	MS -> SS	CHANNEL REQUEST
	115	SS -> MS	IMMEDIATE ASSIGNMENT
	116	MS -> SS	CM SERVICE REQUEST	CKSN= initial value,
				Mobile identity= TMSI
	117	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	117a	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection of the main
				signalling link.

118	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.

119	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	120	SS -> MS	IMMEDIATE ASSIGNMENT	
	121	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	122	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value
				randomly chosen:
				* in Table 10.44 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being 
				excluded
	123	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	124	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	125	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	126	SS -> MS	IMMEDIATE ASSIGNMENT
	127	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.

	128	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there is no more SACCH in the
				uplink
	129	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				within T3211 + RadioLinkTimeOut seconds 
				after the SS stops RF transmission.

	130	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	131	SS -> MS	IMMEDIATE ASSIGNMENT
	132	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	133	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	134	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.

135	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	136	SS -> MS	IMMEDIATE ASSIGNMENT

137	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	138	SS		chooses randomly one among the 
				three following steps: (42 and 
				43) or 48 or 52
	139	SS		performs the chosen step 
	140	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	141	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	142	SS -> MS	IMMEDIATE ASSIGNMENT	
	143	MS -> SS	LOCATION UPDATING REQ	location updating type= normal, 
				CKSN= no key available , 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	144	SS		chooses randomly one among the 
				three following steps: (42 and 
				43) or 48 or 52
	145	SS		performs the chosen step 
	146	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				until T3211 + RadioLinkTimeout seconds 
				after the SS stops RF transmission.
	147	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	148	SS -> MS	IMMEDIATE ASSIGNMENT
	149	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity =IMSI.
	150	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	151	MS -> SS	AUTHENT RESPONSE
	152	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	153	MS -> SS	TMSI REALLOCATION CMP
	154	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle, updated" in cell A

Specific message contents

None.

##### 26.7.4.3.4	Location updating / abnormal cases / attempt counter less or equal to 4, stored LAI equal to broadcast LAI

[Editorial Note:

- In steps 14 and 34 time to check there is no location updating procedure is reduced from 0.5*T3212 (= 3 minutes) to 2*T3211 (=30 seconds). This time is put to check that T3211 has been stopped with the successful MM establishment. This will reduce the test time.

]

###### 26.7.4.3.4.1	Conformance requirement

1)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a periodic location updating procedure (the broadcast LAI is equal to the stored LAI),

1.1	the Mobile Station shall be able to establish an MM connection i.e. send a Channel Request and then a CM Service Request message, CKSN and LAI set to those which have been allocated to the Mobile Station, Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station

1.2	then the Mobile Station shall not attempt a location updating procedure.

2)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during an IMSI attach procedure (the broadcast LAI is equal to the stored LAI).

2.1	the Mobile Station shall be able to establish an MM connection i.e. send a Channel Request and then a CM Service Request message, CKSN and LAI set to those which have been allocated to the Mobile Station, Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station

2.2	then the Mobile Station shall not attempt a location updating procedure.

3)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a periodic location updating procedure and the attempt counter is smaller than 4 the Mobile Station shall send, after T3211 expiry, a Location Updating Request message with the Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station, CKSN IE and LAI set to those which have been allocated to the Mobile Station and the Location Updating type set to "periodic updating".

When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a periodic location updating procedure) after T3212 expiry it shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "normal".

4)	When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a periodic location updating procedure) it shall use a request for a CM connection other than emergency call as a trigger for a location updating procedure.

5)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during an IMSI attach procedure and the attempt counter is smaller than 4 the Mobile Station shall send, after T3211 expiry, a Location Updating Request message with the Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station, CKSN IE and LAI set to those which have been allocated to the Mobile Station and the Location Updating type set to "IMSI attach".

When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during an IMSI attach procedure) after T3212 expiry it shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "normal".

6)	When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during an IMSI attach procedure) it shall use a request for a CM connection other than emergency call as a trigger for a location updating procedure.

References

GSM 04.08 section 4.4.4.9

###### 26.7.4.3.4.2	Test purpose

To verify that in the case when the attempt counter is smaller than 4 and the broadcast LAI is equal to the stored LAI, the MS is in the MM-IDLE state and NORMAL SERVICE substate. To verify that timer T3211 is stopped after a MM connection establishment.

To verify that the MS uses the T3211 timer. and that it enters the MM-IDLE state and NORMAL SERVICE substate when its attempt counter reaches value 4 even in the case where the stored LAI is equal to the broadcast LAI.

###### 26.7.4.3.4.3	Method of test

Initial conditions

System Simulator:

One cell: B, belonging to location area b.

IMSI attach/detach is allowed.

T3212 is set to 6 minutes.

Mobile Station:

The MS is "Idle updated" on cell B with a valid CKSN and a TMSI.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Foreseen final state of the MS

The MS is "idle updated" on cell B with a valid CKSN and a TMSI.

Test Procedure

A failure during the periodic location updating is triggered: as the broadcast LAI is equal to the stored LAI, the MS is still in the MM-IDLE state and NORMAL SERVICE substate and timer T3211 is started. A CM connection other than for emergency call is attempted. It is checked that this is possible and that T3211 is stopped. Same test is performed with a failure during an IMSI attach procedure.

Then failures are triggered during the periodic location updating to let the attempt counter to reach the value of 4. The MS shall enter the MM-IDLE LIMITED SERVICE state and delete any TMSI, stored LAI, ciphering key sequence number and ciphering key. When the attempt counter reaches the value of 4, timer T3212 shall be started. At timer T3212 expiry a location updating procedure is started. A request for CM connection other for than emergency call shall trigger a location updating procedure.

Same tests are performed when the failures are triggered during an IMSI attach procedure.

Maximum duration of test

40 minutes

Expected sequence

Step	Direction	Message	Comments

1	SS		The SS shall wait at most T3212 + 45 seconds
	2	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	5	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	6	SS		performs the chosen step
	7	MS		A MO CM connection is attempted
	8	MS -> SS	CHANNEL REQUEST
	9	SS -> MS	IMMEDIATE ASSIGNMENT
	10	MS -> SS	CM SERVICE REQUEST	CKSN= initial CKSN, Mobile Identity= TMSI
	11	SS -> MS	CM SERVICE ACCEPT
	12	MS -> SS	An initial CM message
	13	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	14	SS		The MS shall not initiate an RR connection
				establishment. This is checked during 2*T3211

15	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.

Steps 16 to 20 are optional
	16	MS -> SS	CHANNEL REQUEST	
	17	SS -> MS	IMMEDIATE ASSIGNMENT	
	18	MS -> SS	IMSI DETACH INDICATION
	19	SS -> MS	CHANNEL RELEASE
	20	MS		Depending on what has been 
				performed in step 15 the MS is 
				brought back to operation.

21	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	22	SS -> MS	IMMEDIATE ASSIGNMENT	
	23	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	24	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	25	SS		performs the chosen step
	26	MS		A MO CM connection is attempted
	27	MS -> SS	CHANNEL REQUEST
	28	SS -> MS	IMMEDIATE ASSIGNMENT
	29	MS -> SS	CM SERVICE REQUEST	CKSN= initial CKSN, Mobile Identity= TMSI
	30	SS -> MS	CIPH MODE COMMAND
	31	MS -> SS	CIPH MODE COMPLETE
	32	MS -> SS	An initial CM message
	33	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection
				of the main signalling link
	34	SS		The MS shall not initiate an RR connection
				establishment. This is checked during 2*T3211 
				MS is "idle, updated" in cell B.

35	SS		The SS shall wait at most T3212 + 15 seconds
	36	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	37	SS -> MS	IMMEDIATE ASSIGNMENT	
	38	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	39	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	40	SS		performs the chosen step
	41	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	42	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	43	SS -> MS	IMMEDIATE ASSIGNMENT	
	44	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	45	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	46	SS		performs the chosen step
	47	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	48	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	49	SS -> MS	IMMEDIATE ASSIGNMENT	
	50	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	51	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	52	SS		performs the chosen step
	53	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	54	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	55	SS -> MS	IMMEDIATE ASSIGNMENT	
	56	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

57	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	58	SS		performs the chosen step
	59	MS		The MS shall not initiate an RR connection
				establishment during T3212 - 15 seconds at 
				least after the channel release.
	60	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	61	SS -> MS	IMMEDIATE ASSIGNMENT	
	62	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic 
				or normal (see Note 1), 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	63	SS -> MS	AUTHENT REQUEST
	64	MS -> SS	AUTHENT RESPONSE
	65	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI
	66	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	67	MS		The MS shall no initiate an RR connection
				establishment earlier than T3212 - 15 seconds
				after the transmission of the
				CHANNEL RELEASE in step 66
	68	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	69	SS -> MS	IMMEDIATE ASSIGNMENT	
	70	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	71	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	72	SS		performs the chosen step
	73	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	74	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	75	SS -> MS	IMMEDIATE ASSIGNMENT	
	76	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	77	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	78	SS		performs the chosen step
	79	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.

80	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	81	SS -> MS	IMMEDIATE ASSIGNMENT	
	82	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	83	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	84	SS		performs the chosen step
	85	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	86	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	87	SS -> MS	IMMEDIATE ASSIGNMENT	
	88	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	89	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	90	SS		performs the chosen step
	91	MS		A MO CM connection is attempted
	92	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	93	SS -> MS	IMMEDIATE ASSIGNMENT	
	94	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	95	SS -> MS	LOCATION UPDATING ACC	IE mobile identity = TMSI
	96	MS -> SS	TMSI REALLOCATION CMP
	97	SS -> MS	CHANNEL RELEASE

Steps 98 to 102 are optional
				Wait 10 s to decide whether to go directly to
				step 103.
	98	MS -> SS	CHANNEL REQUEST
	99	SS -> MS	IMMEDIATE ASSIGNMENT
	100	MS -> SS	CM SERVICE REQUEST	CKSN = no key available,
				Mobile identity= TMSI
	101	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	102	SS -> MS	CHANNEL RELEASE

103	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.

Steps 104 to 107 are optional
104	MS -> SS	CHANNEL REQUEST	
	105	SS -> MS	IMMEDIATE ASSIGNMENT	
	106	MS -> SS	IMSI DETACH INDICATION
	107	SS -> MS	CHANNEL RELEASE

	108	MS		Depending on what has been 
				performed in step 107 the MS is 
				brought back to operation.
	109	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	110	SS -> MS	IMMEDIATE ASSIGNMENT	
	111	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	112	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	113	SS		performs the chosen step
	114	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	115	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	116	SS -> MS	IMMEDIATE ASSIGNMENT	
	117	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	118	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	119	SS		performs the chosen step
	120	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	121	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	122	SS -> MS	IMMEDIATE ASSIGNMENT	
	123	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	124	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	125	SS		performs the chosen step
	126	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	127	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating

128	SS -> MS	IMMEDIATE ASSIGNMENT	
	129	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	130	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	131	SS		performs the chosen step
	132	MS		The MS shall not initiate an RR connection
				establishment during T3212 - 15 seconds at 
				least after the channel release.
	133	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	134	SS -> MS	IMMEDIATE ASSIGNMENT	
	135	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic 
				or normal or IMSI attach (see 
				Note 2), CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	136	SS -> MS	AUTHENT REQUEST
	137	MS -> SS	AUTHENT RESPONSE
	138	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI
	139	MS -> SS	TMSI REALLOCATION CMP
	140	SS -> MS	CHANNEL RELEASE

141	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.

Steps 142 to 145 are optional
	142	MS -> SS	CHANNEL REQUEST	
	143	SS -> MS	IMMEDIATE ASSIGNMENT	
	144	MS -> SS	IMSI DETACH INDICATION
	145	SS -> MS	CHANNEL RELEASE

146	MS		Depending on what has been 
				performed in step 144 the MS is 
				brought back to operation.
	147	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	148	SS -> MS	IMMEDIATE ASSIGNMENT	
	149	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	150	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	151	SS		performs the chosen step
	152	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	153	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	154	SS -> MS	IMMEDIATE ASSIGNMENT	
	155	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	156	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	157	SS		performs the chosen step
	158	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	159	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	160	SS -> MS	IMMEDIATE ASSIGNMENT	
	161	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	162	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	163	SS		performs the chosen step
	164	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	165	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	166	SS -> MS	IMMEDIATE ASSIGNMENT	
	167	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

168	SS		chooses randomly one among the 
				three following choices: (5 and 6) 
				or 11 or 16 of 26.7.4.3.2
	169	SS		performs the chosen step
	170	MS		The MS is made to perform a MO call
	171	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	172	SS -> MS	IMMEDIATE ASSIGNMENT	
	173	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	174	SS -> MS	AUTHENT REQUEST
	175	MS -> SS	AUTHENT RESPONSE
	176	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI
	177	MS -> SS	TMSI REALLOCATION CMP
	178	SS -> MS	CHANNEL RELEASE

Steps 179 to 184 are optional
	179	MS	A MO CM connection is atempted
	180	MS -> SS	CHANNEL REQUEST
	181	SS -> MS	IMMEDIATE ASSIGNMENT
	182	MS -> SS	CM SERVICE REQUEST	CKSN = initial value,
				Mobile identity= TMSI
	183	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	184	SS -> MS	CHANNEL RELEASE

NOTE 1:	the MS can include both types of Location updating. As T3212 expires it can be a periodic location updating procedure and as there is no stored LAI it can be a normal one.

NOTE 2:	same problem as in note 1. Three types of location updating procedures should be allowed.

Specific message contents

None.

#### 26.7.4.4	Location updating / release / expiry of T3240

[ editorial notes :

- no conformance requirement

- new tests may be needed]

##### 26.7.4.4.1	Conformance requirement

The mobile station receiving a LOCATION UPDATING REJECT message shall start T3240: it shall abort the RR connection at the expiry of timer T3240.

References

GSM 04.08 section 4.4.4.8, 11.2.

##### 26.7.4.4.2	Test purpose

To verify that the MS aborts the RR-connection at the expiry of timer T3240.

##### 26.7.4.4.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell A.

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS is "idle updated" on cell B

Test Procedure

A normal location updating procedure is performed. The RR-connection is not released by the SS within the timer T3240. It is checked that the MS aborts the RR-connection.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	SS		The RF level of cell A is
				lowered until the MS selects
				cell B.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING ACC	
	6	SS		The SS waits T3240 expiry.
	7	MS		The MS shall abort the RR
				connection (disconnection of layer 2).

Specific message contents

None.

#### 26.7.4.5	Location updating / periodic

##### 26.7.4.5.1	Location updating / periodic spread

###### 26.7.4.5.1.1	Conformance requirement

1)	The Mobile Stations shall perform spreading of the time before performing a periodic location updating when the location updating timer value is reduced.

2)	The Mobile Station shall reset timer T3212 when the Mobile Station is deactivated, and shall start with a value between zero and the broadcasted value when reactivated in the same cell, IMSI attach being forbidden.

3)	When activated the Mobile Station shall start timer T3212 with a value randomly drawn in the allowed range.

NOTE 3:	This conformance requirement is not covered by a test purpose. It is intended to be covered by a manufacturer declaration.

References

GSM 04.08 section 4.4.2.

###### 26.7.4.5.1.2	Test purpose

1)	To check that when the location updating timer is reduced, the timer running in the MS is started with a value depending on the current timer value and the new broadcasted T3212 value.

2)	To verify that when the MS is reactivated in the same cell (as the one in which it was deactivated), IMSI attach being forbidden, the MS starts the timer T3212 with a value between zero and the broadcasted value

NOTE 4:	It is not tested that the value is random.

###### 26.7.4.5.1.3	Method of test

Initial conditions

System Simulator:

One cell, T3212 is set to 30 minutes.

IMSI attach is allowed in the cell.

Mobile Station:

The MS is deactivated. The stored MCC, MNC and LAC correspond to the broacasted values. The stored update status is "updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated. It performs IMSI attach. 3 minutes after the end of the IMSI attach procedure, the value of T3212 is set to 6 minutes. The MS shall perform periodic location updating 6 minutes after the end of the IMSI attach procedure.

Then, the IMSI attach/detach is forbidden. T3212 is still set to 6 minutes.

The MS is deactivated. The MS is reactivated. It is checked that the MS performs a periodic location updating during the 6 minutes following activation.

Maximum duration of test

20 minutes

Expected sequence

Step	Direction	Message	Comments

1	   MS		The MS is activated
	 2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	LOCATION UPDATING REQ	"location updating type": IMSI attach
	 5	SS -> MS	LOCATION UPDATING ACC	
	 6	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
					the SS waits for the disconnection 
					of the main signalling link.

7	   SS		3 minutes after step 6 the value of T3212
				is set to 6 minutes
	 8	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
				This message shall be sent
				by the MS between 5minutes 45s and
				6minutes 15s after step 6.
	 9	SS -> MS	IMMEDIATE ASSIGNMENT	
	 10	MS -> SS	LOCATION UPDATING REQ
				"location updating type": periodic updating
	 11	SS -> MS	LOCATION UPDATING ACC	
	 12	SS -> MS	CHANNEL RELEASE
				After the sending of this message, 
					the SS waits for the disconnection 
					of the main signalling link.

13	   SS		IMSI attach/detach is not allowed.
	 14	   MS		The MS is deactivated
	 15	   MS		The MS is activated
	 16	   SS		The SS waits until the periodic
				location updating.
	 17	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall
				arrive during the 7 minutes 
				following the MS activation.
	 18	SS -> MS	IMMEDIATE ASSIGNMENT	
	 19	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	 20	SS -> MS	LOCATION UPDATING ACC	
	 21	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.4.5.2	Location updating / periodic normal / test 1

###### 26.7.4.5.2.1	Conformance requirement

1	The Mobile Station shall stop and reset the timer T3212 of the periodic location updating procedure when the first MM message is received or ciphering mode setting is completed in the case of MM connection establishment.

2	The Mobile Station shall stop and reset the timer T3212 of the periodic location updating procedure when the Mobile Station has responded to paging and thereafter has received the first correct L3 message that is not an RR message.

References

GSM 04.08 section 4.4.2.

###### 26.7.4.5.2.2	Test purpose

To verify that the MS stops and resets the timer T3212 of the periodic location updating procedure when :

-	the first MM-message is received in the case of MM-connection establishment, ciphering mode being not set,

-	the MS has responded to paging and the first correct L3 message that is not an RR message is received.

NOTE 5:	T3212 is stopped when the MM-idle state is left and restarted when the MM sublayer returns to that state, substate NORMAL SERVICE or ATTEMPTING TO UPDATE. As a consequence, the exact time when T3212 is reset between those two events cannot be tested.

###### 26.7.4.5.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

IMSI attach/detach is not allowed.

The T3212 timeout value is 2/10 hour .

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

An MS originated MM connection is established and cleared. The channel is released. It is checked that the MS performs a periodic location updating 12 minutes after the release of the channel.

One minute after the periodic location updating, the MS is paged, it sends a CHANNEL REQUEST message and the SS responds with an IMMEDIATE ASSIGNMENT message, a call is established and then cleared. It is checked that the MS performs a periodic location updating 12 minutes after the release of the link.

Maximum duration of test

30 minutes.

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	7	SS		The SS waits until the periodic
				location updating.
	8	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall
				arrive between 11 minutes 45 s and
				12 minutes 15 s after
				the last release of the RR
				connection by the SS.
	9	SS -> MS	IMMEDIATE ASSIGNMENT	
	10	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	11	SS -> MS	LOCATION UPDATING ACC	
	12	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

13	   SS		The SS waits 1 minute.
	14	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity"= IMSI.
	15	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Answer to paging.
	16	SS -> MS	IMMEDIATE ASSIGNMENT	
	17	MS -> SS	PAGING RESPONSE	
	18	SS -> MS	AUTHENTICATION REQ
	19	MS -> SS	AUTHENTIC RESPONSE
	20	SS - MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	21	   SS		The SS waits until the periodic
				location updating.
	22	MS -> SS	CHANNEL REQUEST
				"Establishment cause" : Location updating
				This message shall
				arrive between 11 minutes 45 s and
				12 minutes 15 s after
				the last release of the RR
				connection by the SS.
	23	SS -> MS	IMMEDIATE ASSIGNMENT	
	24	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	25	SS -> MS	LOCATION UPDATING ACC	
	26	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.4.5.3	Location updating / periodic normal / test 2

###### 26.7.4.5.3.1	Conformance requirement

When a LOCATION UPDATING ACCEPT or a LOCATION UPDATING REJECT message is received, the timer T3212 is stopped and reset and the Mobile Station shall perform a periodic location updating after T3212 expiry.

References

GSM 04.08 section 4.4.2.

###### 26.7.4.5.3.2	Test purpose

To verify that the MS stops and resets the timer T3212 of the periodic location updating procedure when a LOCATION UPDATING ACCEPT message is received.

NOTE 6:	T3212 is stopped when the MM-idle state is left and restarted when the MM sublayer returns to that state, substate NORMAL SERVICE or ATTEMPTING TO UPDATE. As a consequence, the exact time when T3212 is reset between those two events cannot be tested.

###### 26.7.4.5.3.3	Method of test

Initial conditions

System Simulator:

2 cells, IMSI attach/detach is allowed in both cells.

T3212 is set to 6 minutes.

Mobile Station:

The MS has a valid TMSI. It is "idle updated"an cell A.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell B.

Test procedure

A normal location updating is performed. The channel is released. One minute later, the MS is deactivated, then reactivated in the same cell. It is checked that the MS performs an IMSI attach and a periodic location updating 6 minutes after the IMSI attach.

Maximum duration of test

20 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be 
				received on cell B.
	1	SS		The RF level of cell A is 
				lowered until the MS selects
				cell B.
	2	MS -> SS	CHANNEL REQUEST	"establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal
	5	SS -> MS	LOCATION UPDATING ACC	
	6	SS ->MS	CHANNEL RELEASE	After the sending of this message,
				the SS waits for the disconnection
				of the main signalling link.
	 7	   SS		The SS waits until the periodic
				location updating.
	 8	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall arrive between 5 minutes 45s
				and 6 minutes 15 s
				after the last release of the RR
				connection by the SS.
	 9	SS -> MS	IMMEDIATE ASSIGNMENT	
	 10	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	 11	SS -> MS	LOCATION UPDATING ACC	
	 12	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

13	   MS		If possible (see PICS) SIM 
				removal is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
				steps 14 to 17 may be performed or not
				depending on the action made in step 13
	 14	MS -> SS	CHANNEL REQUEST	
	 15	SS -> MS	IMMEDIATE ASSIGNMENT	
	 16	MS -> SS	IMSI DETACH INDICATION	
	 17	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	 18	   MS		Depending on what has been 
				performed in step 13 the MS is 
				brought back to operation.
	 19	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 20	SS -> MS	IMMEDIATE ASSIGNMENT	
	 21	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= IMSI 
				attach.
	 22	SS -> MS	LOCATION UPDATING ACC	
	 23	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	 24	   SS		The SS waits until the periodic
				location updating.
	 25	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall
				arrive between 5 minutes 45 s and 6 minutes 15s
				after the last release of the RR
				connection by the SS.
	 26	SS -> MS	IMMEDIATE ASSIGNMENT	
	 27	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	 28	SS -> MS	LOCATION UPDATING ACC	
	 29	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.4.5.4	Location updating / periodic HPLMN search

###### 26.7.4.5.4.1	Location updating / periodic HPLMN search / MS waits time T

###### 26.7.4.5.4.1.1	Conformance requirement

When in automatic mode and roaming in the home country, the MS shall make an attempt to access the HPLMN, if the MS is on the VPLMN at time T after since the last attempt.

NOTE 7:	This test is not intended to test every value in the range 6 minutes to 8 hours or the default of 30 minutes, but is intended to check that the mobile is capable of using the value stored on the SIM.

References

GSM 02.11 section 3.2.2.5.2

GSM 03.22 section 4.4.3.3

###### 26.7.4.5.4.1.2	Test purpose

To verify that when a cell of the HPLMN becomes available, following the successful location request on the VPLMN of the home country and after the first search the mobile has failed to find its HPLMN, that the MS shall perform a location update request on the HPLMN after time T. Were T is the HPLMN Search Period stored in the SIM.

###### 26.7.4.5.4.1.3	Method of test

Initial conditions

System Simulator :

Two cells A and B, belonging to different location areas with location identification a and b. Cell A shall be a cell of the HPLMN and Cell B shall be a cell of the VPLMN with a Country Code the same as that of Cell A. Initially Cell A shall not be broadcasting. IMSI attach/detach is not allowed on either cell.

Mobile Station :

The MS is switched off. The HPLMN Search Period on the SIM shall be set to 6 minutes. The location area information on the SIM is "deleted".

Related PICS/PIXIT statements

Switch on/off button	Yes/No.

Foreseen final state of the MS

The MS is "idle updated" on Cell A.

Test Procedure

Only Cell B shall be broadcasting. The MS shall be switched on either by using the Power Switch or by applying power. A normal location updating is performed on Cell B. Cell A shall be made available after 8 minutes, thus ensuring the MS fails to find the HPLMN during its first attempt. It is verified that the MS performs a location update request on Cell A, within 6 minutes after broadcasting of Cell A.

Maximum duration of test

17 minutes.

Expected sequence

Step	Direction	Message	Contents

The following messages shall be sent and
				received on Cell B.
	1	MS		The MS is switched on by either using the
				Power Switch or by applying power.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATE REQ	"Location Update Type" : Normal.
	5	SS -> MS	LOCATION UPDATE ACC
	6	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.
				The SS waits a period of 8 minutes, this
				allowing the MS to make its first
				periodic search

8	SS		Cell A is made available.
				Within 8 minutes after step 8 the following
				messages shall be sent and received on Cell A
	9	MS -> SS	CHANNEL REQUEST	"Establishment cause": Location updating
	10	SS -> MS	IMMEDIATE ASSIGNEMENT
	11	MS -> SS	LOCATION UPDATE REQ	"Location Update Type": normal
	12	SS -> MS	LOCATION UPDATE ACC
	13	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

Specific message contents

None.

###### 26.7.4.5.4.2	Location updating / periodic HPLMN search / MS in manual mode

###### 26.7.4.5.4.2.1	Conformance requirement

The periodic attempts shall only be performed if in automatic mode when the MS is roaming in its home country.

References

GSM 02.11 section 3.2.2.5.2

GSM 03.22 section 4.4.3.3

###### 26.7.4.5.4.2.2	Test purpose

To verify that no HPLMN Search is performed when the MS is not in automatic mode

###### 26.7.4.5.4.2.3	Method of test

Initial conditions

System Simulator :

Two cells A and B, belonging to different location areas with location identification a and b. Cell A shall be a cell of the HPLMN and Cell B shall be a cell of the VPLMN with a Country Code the same as that of Cell A. Initially Cell A shall not be broadcasting. IMSI attach/detach is not allowed on either cell.

Mobile Station :

The MS is switched off. The HPLMN Search Period on the SIM shall be set to 6 minutes. The location area information on the SIM is "deleted".

Related PICS/PIXIT statements

Switch on/off button	Yes/No.

Foreseen final state of the MS

The MS is "idle updated" on Cell B.

Test Procedure

Only Cell B shall be broadcasting. The MS shall be switched on either by using the Power Switch or by applying power. A normal location updating is performed on Cell B. The MS is forced into manual selection mode. Cell A is made available. It is verified that the MS does not attempt to perform a location update on Cell A.

Maximum duration of test

7 minutes.

Expected sequence

Step	Direction	Message	Contents

The following messages shall be sent and
				received on Cell B.
	1	MS		The MS is switched on by either using the
				Power Switch or by applying power.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATE REQ	"Location Update Type" : Normal.
	5	SS -> MS	LOCATION UPDATE ACC
	6	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

8	MS		The MS is forced into manual selection mode.
	9	SS		Cell A is made available.
	10	SS		The SS waits a period of 7 minutes. During this
				time no messages shall be received on Cell A.

Specific message contents

None.

###### 26.7.4.5.4.3	Location updating / periodic HPLMN search / MS waits at least two minutes and at most T minutes

###### 26.7.4.5.4.3.1	Conformance requirement

After switch on, the MS waits at least 2 minutes and at most T minutes before the first HPLMN Search is attempted.

References

GSM 02.11 section 3.2.2.5.2

GSM 03.22 section 4.4.3.3

###### 26.7.4.5.4.3.2	Test purpose

To verify that the MS waits at least 2 minutes and at most T minutes before attempting its first HPLMN Search.

###### 26.7.4.5.4.3.3	Method of test

Initial Conditions

System Simulator :

Two cells A and B, belonging to different location areas with location identification a and b. Cell A shall be a cell of the HPLMN and Cell B shall be a cell of the VPLMN with a Country Code the same as that of Cell A. Initially Cell A shall not be broadcasting. IMSI attach/detach is not allowed on either cell.

Mobile Station :

The MS is switched off. The HPLMN Search Period on the SIM shall be set to 6 minutes. The location area information on the SIM is "deleted".

Related PICS/PIXIT statements

Switch on/off button	Yes/No.

Foreseen final state of the MS

The MS is "idle updated" on Cell A.

Test Procedure

Only Cell B shall be broadcasting. The MS shall be switched on either by using the Power Switch or by applying power. A normal location updating is performed on Cell B. Cell A is made available. It is verified that the MS attempts to perform a location update on Cell A, after at least 2 minutes and at most T minutes have passed following power on.

Maximum duration of test

8 minutes.

Expected sequence

Step	Direction	Message	Contents

The following messages shall be sent and
				received on Cell B.
	1	MS		The MS is switched on by either using the
				Power Switch or by applying power.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATE REQ	"Location Update Type" : Normal.
	5	SS -> MS	LOCATION UPDATE ACC
	6	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

8	SS		Cell A is made available.
	9	SS		The SS waits a period of 2 minutes after the MS
				is switched on. During this time no messages
				shall be received on Cell A.

				The following messages shall be sent and
				received on cell A.
				Within T minutes after the MS is switched on
				the following messages shall be sent and
				received on cell A.
	10	MS -> SS	CHANNEL REQUEST	"Establishment cause": Location updating
				This message shall be sent between 2 and 7
				minutes after step 1
	11	SS -> MS	IMMEDIATE ASSIGNEMENT
	12	MS -> SS	LOCATION UPDATE REQ	"Location Update Type": normal
	13	SS -> MS	LOCATION UPDATE ACC
	14	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

Specific message contents

None.

#### 26.7.4.6	Location updating / interworking of attach and periodic

###### 26.7.4.6.1	Conformance requirement

1)	If the Mobile Station is in service state NO CELL AVAILABLE, LIMITED SERVICE, PLMN SEARCH or PLMN SEARCH-NORMAL SERVICE when the timer expires the location updating procedure is delayed until this service state is left.

2)	The T3212 timeout value shall not be changed in the NO CELL AVAILBLE, LIMITED SERVICE, PLMN SEARCH and PLMN SEARCH-NORMAL SERVICE states.

3)	If the selected cell is in the location area where the mobile station is registered and IMSI ATTACH is not required and timer T3212 has not expired, then the state is NORMAL SERVICE.

References

1	GSM 04.08 section 4.4.2.

2	GSM 04.08 section 4.4.2.

3	GSM 04.08 section 4.2.1.1

###### 26.7.4.6.2	Test purpose

1)	To check that if the PLU timer expires while the MS is out of coverage, the MS informs the network of its return to coverage.

2)	To check that the PLU timer is not disturbed by cells of forbidden PLMNs.

3)	To check that if the PLU timer does not expire while out of coverage and if the mobile returns to the LA where it is updated, the mobile does not inform the network of its return to coverage.

###### 26.7.4.6.3	Method of test

Initial conditions

System Simulator:

Two cells, a and b, of different PLMNs.

T3212 is set to 12 minutes on cell a.

T3212 is set to 6 minutes on cell b.

IMSI attach is allowed in both cells.

Mobile Station:

The MS is deactivated. The PLMN of cell b is entered in the SIM's forbidden PLMN list.

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS is "idle updated". The PLMN of cell b is entered in the SIM's forbidden PLMN list.

Test procedure

The MS is activated and placed in automatic network selection mode. It performs IMSI attach. 1 minute after the end of the IMSI attach procedure, cell a is switched off. The MS shall not location update on cell b. 8 minutes after the end of the IMSI attach procedure, cell a is switched on. The MS shall not location update on cell a before 11.75 minutes after the end of the IMSI attach procedure. The MS shall perform a periodic location update on cell a between 11.75 minutes and 12.25 minutes after the end of the IMSI attach procedure.

3 minutes after the end of the periodic location updating procedure, cell a is switched off. The MS shall not location update on cell b. 14 minutes after the end of the periodic location updating procedure, cell a is switched on and cell b is switched off. The MS shall perform a location update on cell a before 17 minutes after the end of the periodic location updating procedure.

Maximum duration of test

35 minutes

Expected sequence

Step	Direction	Message	Comments

1	MS		The MS is activated in automatic 
				network selection mode
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	"location updating type": IMSI attach
	5	SS -> MS	LOCATION UPDATING ACC	
	6	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
				the SS waits for the disconnection 
				of the main signalling link.
	7	SS		1 minute after step 6, cell a is switched off
	8	SS		8 minutes after step 6, cell a is switched on
	9	MS -> SS	CHANNEL REQUEST	This message shall be sent
				by the MS between 11 minutes 45s and
				12 minutes 15s after step 6.
	10	SS -> MS	IMMEDIATE ASSIGNMENT	
	11	MS -> SS	LOCATION UPDATING REQ
				"location updating type": periodic updating
	12	SS -> MS	LOCATION UPDATING ACC	
	13	SS -> MS	CHANNEL RELEASE
				After the sending of this message, 
				the SS waits for the disconnection 
				of the main signalling link.
	14	SS		3 minutes after step 13, cell a is switched off
	15	SS		14 minutes after step 13, cell a is switched on
				and cell b is switched off.
	16	MS -> SS	CHANNEL REQUEST	This message shall
				be sent by the MS before 17 minutes
				after step 13.
	17	SS -> MS	IMMEDIATE ASSIGNMENT	
	18	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic
	19	SS -> MS	LOCATION UPDATING ACC	
	22	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

### 26.7.5	MM connection

#### 26.7.5.1	Introduction

[tbd]

#### 26.7.5.2	MM connection / establishment with cipher

##### 26.7.5.2.1	Conformance requirement

1)	The Mobile Station shall be able to correctly set up an MM connection in a Mobile Originating CM connection attemptand send a CM Service Request message with CKSN information element as stored in the SIM and Mobile Identity information element set to the TMSI

2)	The Mobile Station shall be able to interpret cipher mode setting as acceptance of its CM service request i.e. send a CM message.

References

GSM 04.08 sections 4.5.1.1.

##### 26.7.5.2.2	Test purpose

To verify that the MS can correctly set up an MM connection in an origination and interpret cipher mode setting as acceptance of its CM service request.

##### 26.7.5.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has valid TMSI, CKSN. It is "idle updated".

Test Procedure

A mobile originating CM connection is initiated. After the MS has sent the CM SERVICE REQUEST message to the SS, an authentication procedure and a ciphering mode setting procedure are performed. Then, the MS sends a CM message and the SS clears the call and releases the channel.

Maximum duration of test

One minute

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM onnection is attempted
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST	
	 5	SS -> MS	AUTHENT REQUEST	
	 6	MS -> SS	AUTHENT RESPONSE	
	 7	SS -> MS	CIPH MODE COMMAND	The SS starts deciphering.
	 8	MS -> SS	CIPH MODE COMPLETE	The SS starts enciphering.

A9	MS -> SS	SETUP
	A10	SS -> MS	RELEASE COMPLETE	"Cause" IE : "unassigned
				number".

B9	MS -> SS	REGISTER
	B10	SS -> MS	RELEASE COMPLETE

C9	MS -> SS	CP-DATA
	C10	SS -> MS	CP-ACK
	C11	SS -> MS	CP-DATA
	C12	MS -> SS	CP-ACK

13	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.3	MM connection / establishment without cipher

##### 26.7.5.3.1	Conformance requirement

Upon reception of the CM SERVICE ACCEPT message, the MS shall send a CM message.

References

GSM 04.08 sections 4.5.1.1.

##### 26.7.5.3.2	Test purpose

To verify that the MS can correctly set up an MM connection in an originating CM connection establishment when ciphering mode setting is not required.

##### 26.7.5.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. The MM-connection is established without invoking the ciphering mode setting procedure.

Then, the MS sends a CM message and the SS releases the channel.

Maximum duration of test

one minute

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST	
	 5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	SETUP

B6	MS -> SS	REGISTER

C6	MS -> SS	CP-DATA
	C7	SS -> MS	CP-ACK
	C8	SS -> MS	CP-DATA
	C9	MS -> SS	CP-ACK

10	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.4	MM connection / establishment rejected

##### 26.7.5.4.1	Conformance requirement

Upon reception of a CM SERVICE REJECT message, the MS shall not send any layer 3 message, start timer T3240 and enter the "wait for network command" state.

References

GSM 04.08 sections 4.5.1.1.

##### 26.7.5.4.2	Test purpose

To verify that the MS does not send a layer 3 message when the service request is rejected by the SS.

##### 26.7.5.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI; It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS responds with a CM SERVICE REJECT message with reject cause "requested service option not subscribed". It is checked that the MS does not send a layer 3 message.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attemped
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST	
	 5	SS -> MS	CM SERVICE REJECT	"Reject cause" IE : "requested
				service option not subscribed".
	 6	SS		The MS shall not send a layer 3
				message. This is checked during
				5 seconds.
	 7	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.5	MM connection / establishment rejected cause 4

##### 26.7.5.5.1	Conformance requirement

1)	The Mobile Station shall be able to correctly set up an MM connection in a Mobile Originating CM connection attempt and send a CM Service Request message with CKSN information element as stored in the SIM and Mobile Identity information element set to the TMSI

2)	The Mobile Station, when receiving a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR" shall wait for the network to release the RR connection.

3)	The Mobile Station shall then be able to perform a location updating procedure.

References

GSM 04.08 sections 4.5.1.1.

##### 26.7.5.5.2	Test purpose

To verify that the MS can correctly accept a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR".

##### 26.7.5.5.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has valid TMSI, CKSN. It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS responds with a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR". On receipt of this message, the MS shall delete any TMSI, LAI, cipher key and cipher key sequence number. The channel is released. It is checked that the MS performs a normal location updating procedure.

Maximum duration of test

One minute

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST	
	 5	SS -> MS	CM SERVICE REJECT	"Reject cause"= "IMSI
				unknown in VLR"
	 6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	 7	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 8	SS -> MS	IMMEDIATE ASSIGNMENT	
	 9	MS -> SS	LOCATION UPDATING REQ	"Ciphering key sequence number"=
				"No key is available".
				"Mobile identity"= IMSI.
				"Location area identification"=
				deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE).
	10	SS -> MS	AUTHENT REQUEST	
	11	MS -> SS	AUTHENT RESPONSE
	12	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= new TMSI.
	13	MS -> SS	TMSI REALLOCATION CMP	
	14	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.6	MM connection / expiry T3230

[ editorial notes :

- no conformance requirement

]

##### 26.7.5.6.1	Conformance requirement

At T3230 expiry (i.e. no response is given but an RR connection is available) the MM connection establishment shall be aborted.

References

GSM 04.08 sections 4.5.1.2 and 11.2.

##### 26.7.5.6.2	Test purpose

To verify that at T3230 expiry, the MS aborts the MM-connection establishment.

##### 26.7.5.6.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS waits for expiry of timer T3230. It is checked that the MS does not send a layer 3 message but waits for the release of the RR-connection.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted.
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST	
	 5	   SS		The SS waits for expiry of
				timer T3230.
	 6	SS -> MS	CM SERVICE ACCEPT	
	 7	MS -> SS	MM-STATUS	"Reject cause " IE is "message
				not compatible with the call
				state or not implemented".
	 8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.7	MM connection / abortion by the network

##### 26.7.5.7.1	MM connection / abortion by the network / cause #6

###### 26.7.5.7.1.1	Conformance requirement

1)	Upon reception of an ABORT message, the MS shall release any ongoing MM connection and enter the "wait for network command" state.

2)	If the cause in the ABORT message was cause #6, the Mobile Station shall:

2.1	not perform normal location updating

2.2	not perform periodic location updating

2.3	not respond to paging with TMSI

2.4	reject any request for Mobile Originating call establishment except Emergency call

2.5	not perform IMSI detach if deactivated

3)	After reception of an ABORT message with cause #6, the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a Channel Request message with the establishment cause set to "emergency call".

4)	After reception of an ABORT message with cause #6, the Mobile Station shall delete the stored LAI, CKSN and TMSI.

Reference(s)

GSM 04.08 section 4.3.5

###### 26.7.5.7.1.2	Test purpose

To check that upon reception of an ABORT message with cause #6 during call establishment :

-	the MS does not send any layer 3 message.

-	after reception of an ABORT message and after having been deactivated and reactivated, the MS performs location updating using its IMSI as mobile identity and indicates deleted LAI and CKSN.

-	the MS does not perform location updating, does not answer to paging with TMSI, rejects any request for mobile originating call except emergency call, does not perform IMSI detach.

-	the MS accepts a request for emergency call.

###### 26.7.5.7.1.3	Method of test

Initial Conditions

System Simulator:

2 cells, default parameters.

Mobile Station:

The MS has a valid TMSI, CKSN and Kc. It is "idle updated" on cell B.

Related PICS/PIXIT Statement(s)

SIM removal possible without powering down Yes/No.

Switch off on button Yes/No

Support of speech Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell A.

Test procedure

A mobile originating CM connection is attempted. Upon reception of the AUTHENTICATION RESPONSE message, the SS sends an ABORT message with cause #6. The SS waits for 5 seconds. The MS shall not send any layer 3 message. The SS releases the RR connection.

The SS checks that the MS has entered the state MM-IDLE substate NO IMSI, i.e. does not perform normal location updating, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if deactivated.

Maximum Duration Of Test

10 minutes.

Expected Sequence

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	MS		A mobile originating CM connection is attempted
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST
	 5	SS -> MS	AUTHENT REQUEST
	 6	MS -> SS	AUTHENT RESPONSE
	 7	SS -> MS	ABORT	"reject cause" = #6
	 8	SS		The SS waits for 5 seconds
	 9	MS		The MS shall not send any layer 3 message
				during that time
	 10	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be received on cell A

11	   SS		The RF levels are changed
				to make the MS reselect cell A.
	 12	   MS		The MS performs cell reselection
				according to procedure as specified in GSM
				05.08 (this however is not checked 
				until step 22). The MS shall not initiate
				an RR connection establishment on
				cell A or on cell B.

13	   SS		The SS waits at least 7 minutes
				for a possible periodic
				updating.
	 14 	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

15	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				TMSI.
	 16	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is verified during 3 seconds.

17	   MS		A MO CM connection is attempted.
	 18	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds

19	   MS		If the MS supports speech (see PICS),
				an emergency call is attempted.
	 20	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Emergency call.
	 21	SS -> MS	IMMEDIATE ASSIGNMENT	
	 22	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency
				call establishment.
	 23	SS -> MS	CM SERVICE ACCEPT	
	 24	MS -> SS	EMERGENCY SETUP
	 25	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.
	 26	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

27	   MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	 28	   MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

29	MS		Depending on what has been 
				performed in step 29 the MS is 
				brought back to operation.
	 30	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	 31	SS -> MS	IMMEDIATE ASSIGNMENT	
	 32	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,
				"CKSN"= no key available,
				"Mobile Identity"= IMSI,
				"LAI"= deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
	 33	SS -> MS	AUTHENT REQUEST	"CKSN"= CKSN1
	 34	MS -> SS	AUTHENT RESPONSE	
	 35	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity"= TMSI
	 36	MS -> SS	TMSI REALLOCATION CMP
	 37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.5.7.2	MM connection / abortion by the network / cause not equal to #6

###### 26.7.5.7.2.1	Conformance requirement

Upon reception of an ABORT message, the MS shall release any ongoing MM connection and enter the "wait for network command" state.

Reference(s)

GSM 04.08 section 4.3.5

###### 26.7.5.7.2.2	Test purpose

To check that when multiple MM connections are established, the MS releases all MM connections upon reception of an ABORT message, in the case when the two MM connections are established for a mobile terminating call and a non call related supplementary service operation.

###### 26.7.5.7.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in state U10 of a mobile terminating call

Related PICS/PIXIT Statement(s)

The MS supports a non call related supplementary service operation during an active call Yes/No.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated"

Test procedure

A non call related supplementary service operation is attempted at the MS. Upon reception of the REGISTER message, the SS sends an ABORT message with cause # 17. The SS sends a DISCONNECT using the TI of the mobile terminating call. The MS shall send a RELEASE COMPLETE message with the PD and TI of the DISCONNECT message and with cause #81. The SS releases the RR connection.

Maximum Duration Of Test

15 seconds.

Expected Sequence

This procedure is performed if the MS supports non call related supplementary service operation

Step	Direction	Message	Comments

1	MS		A non call related supplementary
				service operation is attempted
				at the MS
	 2	MS -> SS	CM SERVICE REQUEST
	 3	SS -> MS	CM SERVICE ACCEPT
	 4	MS -> SS	REGISTER
	 5	SS -> MS	ABORT	"reject cause" = #17
	 6	SS -> MS	DISCONNECT	with the TI of the
				mobile terminating call
	 7	MS -> SS	RELEASE COMPLETE	"cause"= #81. Same PD and TI
				as the DISCONNECT message
	 8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.8	MM connection / follow-on request pending

##### 26.7.5.8.1	MM connection / follow-on request pending / test 1

###### 26.7.5.8.1.1	Conformance requirement

The MS shall not attempt to establish a new MM connection after location updating on the same RR connection if not allowed by the network.

Reference(s)

GSM 04.08 section 4.4.4.6

###### 26.7.5.8.1.2	Test purpose

To check that when the network does not include the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has a CM application request pending does not attempt to establish a new MM connection on that RR connection.

###### 26.7.5.8.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS has a valid TMSI and is deactivated.

Related PICS/PIXIT Statement(s)

none

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated and a CM connection is immediately attempted. The MS performs location updating. The SS does not include the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for at least 8 seconds. The MS shall not send any layer 3 message for 8 seconds.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated and CM connection 
				is immediately attempted.
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATING REQ
	5	SS -> MS	LOCATION UPDATING ACC	folllow on proceed IE not included
	6	SS		The SS wait for at least 8 seconds
	7	MS		The MS shall not send any layer 3
				message for 8 seconds after reception
				of the LOCATION UPDATING ACCEPT
				message.
	8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.5.8.2	MM connection / follow-on request pending / test 2

###### 26.7.5.8.2.1	Conformance requirement

A MS supporting the follow-on request procedure and having a CM connection request pending shall correctly establish an MM connection following a location update when allowed by the network.

Reference(s)

GSM 04.08 section 4.4.4.6

###### 26.7.5.8.2.2	Test purpose

To check that when the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that supports the follow on request procedure and that has a CM application request pending establishes successfully a new MM connection on that RR connection.

###### 26.7.5.8.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS has a valid TMSI and is deactivated.

Related PICS/PIXIT Statement(s)

MS supports the follow on request procedure   Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated and a CM connection is immediately attempted. The MS performs location updating. The SS includes the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for at least 8 seconds.

If the MS supports the follow on request procedure :

The MS shall send a CM SERVICE REQUEST. Upon reception of that message, the SS sends a CM SERVICE ACCEPT message. The MS shall send an initial CM message. Upon reception of that message, the SS releases the RR connection.

If the MS does not support the follow on request procedure :

The MS shall not send any layer 3 message for 8 seconds.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated and an CM connection
				is immediately attempted.
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT
	 4	MS -> SS	LOCATION UPDATING REQ
	 5	SS -> MS	LOCATION UPDATING ACC	folllow on proceed IE included

If the MS supports the follow on
				request procedure (see PICS) steps A6 to A8
				are performed, otherwise steps B6 to B7
				are performed

A6	MS -> SS	CM-SERVICE REQUEST
	A7	SS ->MS	CM-SERVICE ACCEPT
	A8	MS -> SS	An initial CM message

B6	SS		The SS wait for at least 8 seconds
	B7	MS		The MS shall not send any layer 3
				message for 8 seconds after reception
				of the LOCATION UPDATING ACCEPT
				message.

9	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

none

##### 26.7.5.8.3	MM connection / follow-on request pending / test 3

###### 26.7.5.8.3.1	Conformance requirement

1)	The MS shall not set the follow on request bit in a LOCATION UPDATING REQUEST message if no MM connection request is pending.

2)	When the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has no CM application request pending shall not attempt to establish a new MM connection on that RR connection.

3)	The MS shall correctly handle a CM connection established by the network on the RR connection that was used for the location updating procedure.

Reference(s)

GSM 04.08 section 4.4.4.6

###### 26.7.5.8.3.2	Test purpose

1)	To check that a MS that has no CM application request pending sets the Follow-On-Request bit to No follow-on request pending in a LOCATION UPDATING REQUEST message.

2)	To check that when the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has no CM application request pending does not attempt to establish a new MM connection on that RR connection.

3)	To check that the MS accepts establishment by the network of a new MM connection on the existing RR connection.

###### 26.7.5.8.3.3	Method of test

Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS has a valid TMSI and is deactivated.

Related PICS/PIXIT Statement(s)

Supported services on TCH

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated. The MS performs location updating. The MS shall set the FOR bit to No follow-on request pending in the LOCATION UPDATING REQUEST message. The SS includes the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for 5 seconds. The MS shall not send any layer 3 message for 5 seconds. The SS sends a SETUP message to the MS requesting a basic service supported by the MS. The MS shall send either a CALL CONFIRMED message if it supports a service on TCH or a RELEASE COMPLETE with cause #88.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATING REQ	"Location updating type" : the FOR bit
				is set to No follow-on request pending
	5	SS -> MS	LOCATION UPDATING ACC	follow on proceed IE is included
	6	   SS		The SS wait for 5 seconds
	7	   MS		The MS shall not send any layer 3
				message for 5 seconds after reception
				of the LOCATION UPDATING ACCEPT
				message.
	8	SS -> MS	SETUP

If the MS supports a basic service on TCH
	 A9	MS -> SS	CALL CONFIRMED

If the MS does not support any basic
				service on TCH
	B9	MS -> SS	RELEASE COMPLETE	cause #88

10	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

none

### 26.7.6	Default contents of messages

Default contents SYSTEM INFORMATION messages and default settings

For cell A and B 		For GSM use 26.6.14

For DCS use 26.6.15

Cell C

The contents of SYSTEM INFORMATION TYPE 1 to 6 messages for cell C are identical to those of cell A with the following exceptions:

Cell Channel Description

-	Format Identifier			Bit map 0 for GSM

Range 512 for DCS

-	Cell Allocation ARFCN			Channel Number 30 for GSM,

Channel Number 700 for DCS,

Cell Identity

-	Cell Identity Value			0003H

Default settings for cell C:

Downlink input level				53 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power

class

Propagation profile				static

BCCH/CCCH carrier number			30 for GSM

700 for DCS

ABORT

Information element					Value/remark
Reject cause						Depending on the test one of either :

#6 	- Illegal ME

#17 	- Network Failure.

AUTHENTICATION REQUEST

Information element					Value/remark
Cipher Key Sequence Number				Arbitrary

Authentication parameter RAND 				Arbitrarily chosen by the test house

AUTHENTICATION RESPONSE

Information element	Value/remark
Authentication parameter SRES	As applicable

AUTHENTICATION REJECT

Information element	Value/remark
None but message head

CHANNEL RELEASE

Information element	Value/remark
RR cause	Normal release

CIPHERING MODE COMMAND

Information element	Value/remark
Cipher mode setting	Start ciphering
Cipher Response	IMEI must not be included

CM RE-ESTABLISHMENT REQUEST

Information element					Value/remark

Cipher Key Sequence Number				According to SIM contents

Mobile station classmark 2					See PICS/PIXIT
Mobile Identity						IMSI of MS under test

Location area identification					As in section 26.1.1

CM SERVICE ACCEPT

Information element	Value/remark
None but message head	Omitted

CM SERVICE REQUEST

Information element	Value/remark

CM service type	Mobile originating call establishment
	unless otherwise specified in test.

Ciphering key sequence number	According to SIM contents
Mobile station classmark 2	See PICS/PIXIT
Mobile identity	TMSI of the MS under test

CM SERVICE REJECT

Information element	Value/remark
Reject cause	Depending on test

IDENTITY REQUEST

Information element	Value/remark
Identity type	Depending on test
Spare half octet	0000

IDENTITY RESPONSE

Information element	Value/remark
Mobile identity	Depending on test

IMMEDIATE ASSIGNMENT

Information element	Value/remark
L2 pseudo length
Page mode	Normal Paging
Spare half octet	0000
Channel description
- Channel type and TDMA offset	SDCCH/4 or SDCCH/8
- Time slot number	Arbitrary legal value
- Subsequent fields of the Channel description IE depend upn the Type of MS under test (either DCS1800 or GSM), as specified in section 26.1.1
Request reference
- Random access information	As received from MS
- N51,N32,N26	Corresponding to the frame in which the Channel Request was sent
Timing advance	0
Mobile allocation	Empty (L=0)
Starting time	Omitted
IA rest octets	all bits set to spare

IMSI DETACH INDICATION

Information element	Value/remark
Mobile station classmark 1	See PICS/PIXIT
Mobile identity	TMSI of the MS under test

LOCATION UPDATING ACCEPT

Information element	Value/remark
Location area identification	As in section 26.1.1
Mobile identity	Omitted
Follow on proceed	Omitted

LOCATION UPDATING REJECT

Information element					Value/remark
Reject cause						As specified in test

LOCATION UPDATING REQUEST

Information element					Value/remark
Location updating type					Normal location updating

Cipher Key Sequence Number				According to SIM contents

Location area identification					As in section 26.1.1
Mobile station classmark					See PICS/PIXIT

Mobile identity						TMSI of the MS

PAGING REQUEST TYPE 1

Information element	Value/remark
L2 pseudo length
Page Mode	Normal paging
Channels needed
- mobile 1	"any channel"
- mobile 2	spare
Mobile identity 1	TMSI of MS under test
Mobile identity 2	Omitted
P1 rest octets	All bits set to spare

PAGING RESPONSE

Information element	Value/remark
Ciphering key sequence number	According to SIM contents
Spare half octet	0000
Mobile station classmark 2	See PICS/PIXIT
Mobile identity	TMSI of the MS under test

TMSI REALLOCATION COMMAND

Information element	Value/remark
Location area identification	As in section 26.1.1
Mobile identity	TMSI of the MS under test

TMSI REALLOCATION COMPLETE

Information element	Value/remark
None but message head	omitted

## 26.8	Tests related to circuit switched call control

### 26.8.1	Circuit switched Call Control (CC) state machine verification

#### 26.8.1.1	General on CC state machine verification

The principle of checking the call control functions consists in the validation of each call control identified state.

State U0 as an initial state is not verified in the tests of 26.8.1.2 (establishment of an outgoing call).

State U0.1 is never verified.

The steps to be followed within each performed test are:

-	bring the MS into the required state,

-	trigger the tested event,

-	check the MS response and new state.

In sections 26.8.1.2 and 26.8.1.3 different tables are defined to bring the MS into the required initial state. The exact table to be chosen is specified individually in section "Initial conditions" of "Method of test" for each test case.

For each test, unless otherwise specified, a circuit switched basic service among those supported by the MS but excluding the emergency call teleservice shall be chosen arbitrarily, and the test shall be performed according to that basic service. If the only circuit switched basic service supported by the mobile is emergency call, then the incoming call tests shall not be performed and the other call control tests shall be performed with the EMERGENCY SETUP message replacing the SETUP message.

The initial states are to be checked through STATUS ENQUIRY messages sent by the SS, when feasible. This is not explicitly stated in the tables of expected sequences of signalling messages. The checking of final states are explicitly included into the expected sequences of signalling messages.

The following postamble may be used by the SS to bring MS back to idle mode in those test cases, in which it is not already included into expected sequence of signalling messages:

Table 26.8.1.1/1: A postamble to bring the MS back to idle mode.

Step	Direction	Message	Comments

n	SS -> MS	CHANNEL RELEASE	
	n+1	MS		the MS shall release the main
				signalling link (DISC/UA)

The postamble has not been included into the all of the tests in order to leave an option to concatenate the procedures in the future by using a final state of a test case as an initial state to another one.

For the special case of U0, the state is checked by sending STATUS ENQUIRY message with all possible values of transaction identifier (seven values) as U0 is the only state in which for every TI the MS will answer with release complete with cause #81. If U0 is to be verified when no RR connection exists, first a mobile terminating radio connection must be established.

The MS responses are either call management messages received by the SS or lower layers functions activated within the MS or MMI actions (e.g. the buzzing of an alerting tone).

A timeout within the MS is triggered by the SS when it does not answer back an MS expected response.

The test sequences may be split in 3 main groups:

-	establishment and release of an outgoing call;

-	establishment and release of an incoming call;

-	in-call functions.

Remark on verification of transient states:

Some call control states of the mobile station may be transient, depending on implementation, configuration of the MS and previous messages (see Annex 3, subclause 3.1.6).

If a test starts in a transient state, then the test is executed without verification of the starting state.

##### 26.8.1.1.1	List of abbreviations

The following abbreviations are used throughout the section 26.8.1.

AUTH REQUEST			=	AUTHENTICATION REQUEST

AUTH RESPONSE			=	AUTHENTICATION RESPONSE

CM-REESTAB REQUEST		=	CM-REESTABLISHMENT REQUEST

CHANNEL MODE MOD ACK	=	CHANNEL MODE MODIFY ACKNOWLEDGE

CIPH MODE COMMAND		=	CIPHERING MODE COMMAND

CIPH MODE COMPLETE		=	CIPHERING MODE COMPLETE

START DTMF ACK		=	START DTMF ACKNOWLEDGE

STOP DTMF ACK			=	STOP DTMF ACKNOWLEDGE

#### 26.8.1.2	Establishment of an outgoing call

Initial conditions

As a minimum requirement the MS is updated and has been given a TMSI, a ciphering key and cipher key sequence number, and the layer 2, RR and MM functionalities have been verified.

There are as many CM initial conditions as states to be checked.

The tables below describe message exchanges which bring the MS in the requested initial states.

A state may be taken as initial only when all the states which lead to this initial states have been validated. The order followed in the test procedure will be U0, U0.1, U1, U3, U4, U10, U12, U19, U11 as seen in the table underneath.

The MS is brought again in the initial state starting with U0 at each new test performed.

Table 26.8.1.2/1: Establishment of an outgoing call, procedure 1 (late assignment)

Step	Direction	Message	Comments/actions/next state

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call
	2	SS -> MS	IMMEDIATE ASSIGNMENT	SDCCH, U0
	3	MS -> SS	CM-SERVICE REQUEST	U0.1
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	U1
	7	SS -> MS	CALL PROCEEDING	U3
	8	SS -> MS	ALERTING	U4
	9	SS -> MS	ASSIGNMENT COMMAND	TCH
	10	MS -> SS	ASSIGNMENT COMPLETE	
	11	SS -> MS	CONNECT	
	12	MS -> SS	CONNECT ACKNOWLEDGE	U10

A13	SS -> MS	DISCONNECT	U12 (note 1)

B13	SS -> MS	DISCONNECT	U12 (note 2)
	B14	MS -> SS	RELEASE	U19

C13			MMI action, terminate call
	C14	MS -> SS	DISCONNECT	U11

NOTE 1:	The Progress Indicator IE with progress description #8 "in band information or appropriate pattern now available" is included.

NOTE 2:	The Progress Indication IE is not included.

Table 26.8.1.2/2: Establishment of an outgoing call, procedure 2

Step	Direction	Message	Comments/actions/next state

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call
	2	SS -> MS	IMMEDIATE ASSIGNMENT	TCH, U0
	3	MS -> SS	CM-SERVICE REQUEST	U0.1
	4	SS -> MS	CHANNEL MODE MODIFY	(note 3)
	5	MS -> SS	CHANNEL MODE MOD ACK	
	6	SS -> MS	CIPH MODE COMMAND	
	7	MS -> SS	CIPH MODE COMPLETE	
	8	MS -> SS	SETUP	U1
	9	SS -> MS	CALL PROCEEDING	U3
	10	SS -> SS	ALERTING	U4
	11	SS -> MS	CONNECT	
	12	MS -> SS	CONNECT ACKNOWLEDGE	U10

A13	SS -> MS	DISCONNECT	U12 (note 4)

B13	SS -> MS	DISCONNECT	U12 (note 5)
	B14	MS -> SS	RELEASE	U19

C13			MMI action, terminate call
	C14	MS -> SS	DISCONNECT	U11

NOTE 3:	Assigned channel is appropriate for the chosen bearer capability (see 26.8.1).

NOTE 4:	The Progress Indicator IE with progress description #8 "in band information or appropriate pattern now available' is included.

NOTE 5:	The Progress Indicator IE is not included.

Table 26.8.1.2/3: Establishment of an outgoing call, procedure 3

Step	Direction	Message	Comments/actions/next state

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call
	2	SS -> MS	IMMEDIATE ASSIGNMENT	SDCCH, U0
	3	MS -> SS	CM-SERVICE REQUEST	U0.1
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	U1
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	U3
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> MS	ALERTING	U4
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACKNOWLEDGE	U10

A15	SS -> MS	DISCONNECT	U12 (note 6)

B15	SS -> MS	DISCONNECT	U12 (note 7)
	B16	MS -> SS	RELEASE	U19

C15			MMI action, terminate call
	C16	MS -> SS	DISCONNECT	U11

NOTE 6:	The Progress Indicator IE with progress description #8 "in band information or appropriate pattern now available" is included.

NOTE 7:	The Progress indicator IE is not included.

Table 26.8.1.2/4: Establishment of an outgoing call, procedure 4

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call
	2	SS -> MS	IMMEDIATE ASSIGNMENT	TCH, U0
	3	MS -> SS	CM-SERVICE REQUEST	U0.1
	4	SS -> MS	IDENTITY REQUEST	
	5	MS -> SS	IDENTITY RESPONSE	
	6	SS -> MS	CIPH MODE COMMAND	
	7	MS -> SS	CIPH MODE COMPLETE	
	8	MS -> SS	SETUP	U1
	9	SS -> MS	CHANNEL MODE MODIFY	(note 8)
	10	MS -> SS	CHANNEL MODE MOD ACK	
	11	SS -> MS	CALL PROCEEDING	U3
	12	SS -> MS	ALERTING	U4
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACKNOWLEDGE	U10

A15	SS -> MS	DISCONNECT	U12 (note 9)

B15	SS -> MS	DISCONNECT	U12 (note 10)
	B16	MS -> SS	RELEASE	U19

C15			MMI action, terminate call
	C16	MS -> SS	DISCONNECT	U11

NOTE 8:	Assigned channel is appropriate for the chosen bearer capability (see 26.8.1).

NOTE 9:	The Progress Indicator IE with progress description #8 "in band information or appropriate pattern now available" is included.

NOTE 10:	The Progress Indicator IE is not included.

##### 26.8.1.2.1	Outgoing call / U0 null state

###### 26.8.1.2.1.1	Outgoing call / U0 null state / MM connection requested

26.8.1.2.1.1.1	Definition and applicability

The call control entity of the Mobile Station requests the MM-sublayer to establish a mobile originating MM-connection. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.1.1.2	Conformance requirement

1)	Upon initiation of an outgoing basic call by user the MS shall initiate establishment of an MM connection, using as first MM message a CM-SERVICE REQUEST message with CM service type "Mobile originating call establishment or packet mode connection establishment".

References

TS GSM 04.08 section 5.2.1.1, TS GSM 04.08 section 4.5.1.1, TS GSM 04.08 section 3.3.1.

26.8.1.2.1.1.3	Test purpose

To verify that upon initiation of an outgoing basic call by user the MS initiates establishment of an MM connection, using as first MM message a CM-SERVICE REQUEST message with CM service type "Mobile originating call establishment or packet mode connection establishment".

26.8.1.2.1.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

U0, null.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. When the SS receives CM-SERVICE REQUEST, the contents of it shall be checked, and also the state of the call control entity shall be checked by STATUS ENQUIRY with all the relevant transaction identifiers.

Maximum duration of test

30 s

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call
	2	SS -> MS	IMMEDIATE ASSIGNMENT	TCH
	3	MS -> SS	CM-SERVICE REQUEST	verify the type of call which is asked
				for "basic" or "emergency by the MS
	4	SS -> MS	STATUS ENQUIRY	
	5	MS -> SS	RELEASE COMPLETE	with cause 81# (invalid TI value)
	6	SS		repeat steps 4-5 to cover all the
				transaction identifiers from 000 ...110
	7	SS -> MS	CHANNEL RELEASE	
	8	MS		the MS shall release the main signalling
				link (DISC/UA)

Specific message contents:

None.

##### 26.8.1.2.2	Outgoing call / U0.1 MM connection pending

###### 26.8.1.2.2.1	Outgoing call / U0.1 MM connection pending / CM service rejected

26.8.1.2.2.1.1	Definition and applicability

A request for MM connection is rejected by the SS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.2.1.2	Conformance requirement

Upon receiving indication of an MM-connection establishment being rejected, CC entity should inform upper layer of this rejection.

References

TS GSM 04.08, section 4.5.1.1 , TS GSM 04.07, section 6.2.2.

26.8.1.2.2.1.3	Test purpose

To verify that a CC entity of the MS in CC-state U0.1, "MM-connection pending", upon the MS receiving a CM SERVICE REJECT message, returns to CC state U0, "Null".

26.8.1.2.2.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U0.1 by using table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. When the SS receives CM-SERVICE REQUEST, the contents of it shall be checked. The SS rejects it by CM-SERVICE REJECT. Then the SS will check the state of the MS by using STATUS ENQUIRY with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CM-SERVICE REJECT	
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	RELEASE COMPLETE	cause shall be 81# (invalid TI value)
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000 ...110
	5	SS -> MS	CHANNEL RELEASE	
	6	MS		the MS shall release the main signalling 						link (DISC/UA)

Specific message contents:

None.

###### 26.8.1.2.2.2	Outgoing call / U0.1 MM connection pending / CM service accepted

26.8.1.2.2.2.1	Definition and applicability

A CM request is accepted for the MM-connection by the SS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.2.2.2	Conformance requirement

A CC entity of the MS in CC-state U0.1, "MM-connection pending", upon the MS receiving a CM SERVICE ACCEPT message, shall send a SETUP message specifying the Called party BCD number that was entered into the MS and then enter CC state U1, "Call initiated".

References

TS GSM 04.08, section 4.5.1.1, TS GSM 04.08, section 5.2.1.1.

26.8.1.2.2.2.3	Test purpose

To verify that a CC entity of the MS in CC-state U0.1, "MM-connection pending", upon the MS receiving a CM SERVICE ACCEPT message, sends a SETUP message specifying the Called party BCD number that was entered into the MS and then enters CC state U1, "Call initiated".

26.8.1.2.2.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U0.1 by using table 26.8.1.2/1.

Foreseen final state of the MS

U1, call initiated.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. When the MS is requesting a MM-connection, the SS will indicate acceptance by sending a CM-SERVICE ACCEPT message. The MS shall respond with SETUP. Then the SS will check the state of the call control entity by STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CM-SERVICE ACCEPT	
	2	MS -> SS	SETUP	with called party BCD number
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause shall be 30# (response to enq.)
				and state U1 call initiated

Specific message contents:

None.

###### 26.8.1.2.2.3	Outgoing call / U0.1 MM connection pending / lower layer failure

26.8.1.2.2.3.1	Definition and applicability

The call control entity of the MS being in the state, U0.1, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.2.3.2	Conformance requirement

1)	Upon a lower layer failure the MS releases the MM connection in progress and returns to idle mode. In that state no call exists, and the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

References

TS GSM 04.08, section 4.5.1.2, TS GSM 04.08, section 5.2.1.1., TS GSM 04.08 section 5.5.3.2. and TS GSM 04.08 section 8.3.

26.8.1.2.2.3.3	Test purpose

To verify that after the MS with a CC entity in state U0.1, "MM-connection pending", has detected a lower layer failure and has returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.2.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U0.1 by using table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. When the MS has sent a CM-SERVICE REQUEST message, the SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1		SS	SS generates lower layer failure
	2		SS	SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause shall be 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000 ...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

##### 26.8.1.2.3	Outgoing call / U1 call initiated

###### 26.8.1.2.3.1	Outgoing call / U1 call initiated / receiving CALL PROCEEDING

26.8.1.2.3.1.1	Definition and applicability

The call control entity of the MS being in the state, U1, a CALL PRECEEDING message is sent by the SS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.1.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CALL PROCEEDING message, shall enter CC state U3, "Mobile originating call proceeding".

References

TS GSM 04.08, section 5.2.1.1, 5.2.1.2, 5.2.1.3.

26.8.1.2.3.1.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CALL PROCEEDING message, enters CC state U3, "Mobile originating call proceeding".

26.8.1.2.3.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/2.

Foreseen final state of the MS

U3, Mobile originating call proceeding.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a CALL PROCEEDING message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U3.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CALL PROCEEDING	tone generation not mandatory
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U3

Specific message contents:

None.

###### 26.8.1.2.3.2	Outgoing call / U1 call initiated / rejecting with RELEASE COMPLETE

26.8.1.2.3.2.1	Definition and applicability

The call contrl entity of the MS being in the state, U1, the call is rejected by a RELEASE COMPLETE message sent by the SS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.2.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a RELEASE COMPLETE message with valid cause value, shall enter CC state U0, "Null".

2)	On returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

3)	On releasing the MM-connection, the MS shall wait for  MM layer release initiated by the network.

References

Conformance requirement 1:	TS GSM 04.08, section 5.4.2, TS GSM 04.08, section 5.4.4.

Conformance requirement 2:	TS GSM 04.08, section 5.5.3.2.

Conformance requirement 3:	TS GSM 04.08, section 5.4.4.3, TS GSM 04.08, section 4.5.3, TS GSM 04.08, section 3.5.1.

26.8.1.2.3.2.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a RELEASE COMPLETE message with valid cause value, enters CC state U0, "Null".

2)	To verify that in returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

3)	To verify that in releasing the MM-connection, the MS shall wait for MM layer release initiated by SS.

26.8.1.2.3.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a RELEASE COMPLETE message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE COMPLETE	See specific message content below.
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000...110
	5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

RELEASE COMPLETE

1) With a valid cause value among:

related to numbering,

#1 unallocated number

#3 no route to destination

#22 number changed

#28 invalid number format

related to bearer capabilities,

#8 operator determined barring

#57 bearer capability not authorized

#58 bearer capability not presently available

#63 service or option not available

#65 bearer service not implemented

#34 no circuit/channel available (call queueing).

###### 26.8.1.2.3.3	Outgoing call / U1 call initiated / T303 expiry

26.8.1.2.3.3.1	Definition and applicability

The call control entity of the MS being in the state, U1, if no response is then received from the SS, timer T303 expires at the MS side. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.3.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon expiry of T303 shall send a DISCONNECT message to its peer entity and enter state U11, "Disconnect request".

References

TS GSM 04.08, section 5.2.1.1, TS GSM 04.08, section 5.4.

26.8.1.2.3.3.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon expiry of T303 (accuracy +/-20% between reception of CM-SERVICE REQUEST and DISCONNECT by SS) sends a DISCONNECT message to its peer entity and enters state U11, "Disconnect request".

26.8.1.2.3.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/2.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

1 minute.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. When T303 expiries at the MS, the MS shall send DISCONNECT. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS waits for T303 expiry
	2	MS -> SS	DISCONNECT	Shall be transmitted between 24 s and
				36 s after the CM SERVICE REQUEST.
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, status U11

Specific message contents:

None.

###### 26.8.1.2.3.4	Outgoing call / U1 call initiated / lower layer failure

26.8.1.2.3.4.1	Definition and applicability

The call control entity of the MS being in the state, U1, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.4.2	Conformance requirement

Upon a lower layer failure MM informs the relevant CM entities that the MM connection has been interrupted. As call re-establishment is not allowed, the CC entity must perform a local release. The MS returns to idle mode. In that state no call exists, and the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

References

TS GSM 04.08, section 4.5.2.3, TS GSM 04.08, section 5.2.1.1, TS GSM 04.08 section 5.5.3.2.

26.8.1.2.3.4.3	Test purpose

To verify that after the MS with a CC entity in state U1 "Call initiated", has detected a lower layer failure and has returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.3.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U1. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	
	11	MS		the MS shall release the main signalling
				link (DISC/UA)

Specific message contents:

None.

###### 26.8.1.2.3.5	Outgoing call / U1 call initiated / receiving ALERTING

26.8.1.2.3.5.1	Definition and applicability

The call control entity of the MS being in the state, U1, an ALERTING message is sent to the MS as a indication that a call is being alerted at a called end. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.5.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of an ALERTING message, shall enter CC state U4, "Call delivered".

References

TS GSM 04.08, section 5.2.1.1.

26.8.1.2.3.5.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of an ALERTING message, enters CC state U4, "Call delivered".

26.8.1.2.3.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/4.

Foreseen final state of the MS

U4, call delivered.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends an ALERTING message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U4, call delivered.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ALERTING
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

###### 26.8.1.2.3.6	Outgoing call / U1 call initiated / entering state U10

26.8.1.2.3.6.1	Definition and applicability

The call control entity of the MS being in the state, U1, a CONNECT message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.6.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CONNECT message, shall send a CONNECT ACKNOWLEDGE message to its peer entity and enter CC state U10, "Active".

References

TS GSM 04.08, section 5.2.1.1, TS GSM 04.08, section 5.2.1.6.

26.8.1.2.3.6.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CONNECT message, sends a CONNECT ACKNOWLEDGE message to its peer entity and enters CC state U10, "Active".

26.8.1.2.3.6.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/4.

Foreseen final state of the MS

U10, call active.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a CONNECT message to the MS. The MS shall respond by sending a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U10, active.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CONNECT	
	2	MS -> SS	CONNECT ACKNOWLEDGE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

###### 26.8.1.2.3.7	Outgoing call / U1 call initiated / unknown message received

26.8.1.2.3.7.1	Definition and applicability

The call control entity of the MS being in the state, U1, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.7.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a message with message type not defined for the protocol discriminator from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.2.3.7.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a message with message type not defined for the protocol discriminator unknown message from its peer entity returns a STATUS message.

26.8.1.2.3.7.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/1.

Foreseen final state of the MS

U1, call initiated.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U1
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U1

Specific message contents:

None.

##### 26.8.1.2.4	Outgoing call / U3 MS originating call proceeding

###### 26.8.1.2.4.1	Outgoing call / U3 MS originating call proceeding / ALERTING received

26.8.1.2.4.1.1	Definition and applicability

The call control entity of the MS being in the state, U3, an ALERTING message is sent to the MS as a indication that a call is being alerted at a called end. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a ALERTING message shall enter CC-state U4, "Call Delivered".

References

TS GSM 04.08 section 5.2.1.5.

26.8.1.2.4.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a ALERTING message enters CC-state U4, "Call Delivered".

26.8.1.2.4.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U4, call delivered.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends an ALERTING message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U4, call delivered.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ALERTING
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

###### 26.8.1.2.4.2	Outgoing call / U3 MS originating call proceeding / CONNECT received

26.8.1.2.4.2.1	Definition and applicability

The call control entity of the MS being in the state, U3, a CONNECT message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a CONNECT message shall return a "CONNECT ACKNOWLEDGE" message to its peer entity and enter the CC state U10, "Active".

2)	The MS shall then stop any locally generated indication.

References

Conformance requirement 1:	TS GSM 04.08 section 5.2.1.6.

Conformance requirement 2:	TS GSM 04.08 section 5.2.1.6.

26.8.1.2.4.2.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a CONNECT message returns a "CONNECT ACKNOWLEDGE" message to its peer entity and enters the CC state U10, "Active".

2)	To verify that the MS stops locally generated indication, if any.

26.8.1.2.4.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U10, active.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a CONNECT message to the MS. The MS shall respond by sending a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U10, active.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CONNECT	the MS shall stop tone generation, if any
	2	MS -> SS	CONNECT ACKNOWLEDGE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

###### 26.8.1.2.4.3	Outgoing call / U3 MS originating call proceeding / PROGRESS received without in band information

26.8.1.2.4.3.1	Definition and applicability

The call control entity of the MS being in the state, U3, a PROGRESS message is received by the MS. The PROGRESS message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message with valid cause values shall stay in CC-state U3.

2)	After receipt of the PROGRESS message timer T310 shall be stopped.

References

Conformance requirement 1:	TS GSM 04.08 section 5.2.1.4.

Conformance requirement 2:	TS GSM 04.08 section 11.3.

26.8.1.2.4.3.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message with valid cause values stays in CC-state U3.

2)	To verify that after receipt of the PROGRESS message timer T310 is stopped.

26.8.1.2.4.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U3, mobile originating call proceeding.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a PROGRESS message not containing indication of in-band information availability to the MS. The SS checks that the MS has stopped T310, ie at T310 timeout no DISCONNECT message is sent by the MS. Then the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PROGRESS	(note 11)
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U3
	4	SS		SS waits at least 45 seconds and checks
				no DISCONNECT is sent by the MS
	5	SS -> MS	STATUS ENQUIRY	
	6	MS -> SS	STATUS	cause 30#, state U3

NOTE 11:	Tested with a valid cause value among:

#4 call has returned to PLMN/ISDN

#32 call is end-to-end PLMN/ISDN or

any value in the set #(21-127)

Specific message contents:

None.

###### 26.8.1.2.4.4	Outgoing call / U3 MS originating call proceeding / PROGRESS with in band information

26.8.1.2.4.4.1	Definition and applicability

The call control entity of the MS being in the state, U3, a PROGRESS message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message indicating in-band announcement shall through-connect the traffic channel  for speech, if TCH is in a speech mode. If TCH is not in speech mode, the MS shall not through-connect the TCH.

2)	After receipt of the PROGRESS message, T310 shall be stopped.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.1.3, TS GSM 04.08 section 5.2.1.4., TS GSM 04.08 section 5.2.1.9, TS GSM 04.08 section 5.5.1., TS GSM 04.08 section 11.3.

26.8.1.2.4.4.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message indicating in-band announcement through-connects the traffic channel for speech, if TCH is in speech mode. If TCH is not in a speech mode, the MS does not through-connect the TCH.

2	To verify that after receipt of the PROGRESS message, T310 is stopped.

26.8.1.2.4.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U3, mobile originating call proceeding.

Maximum duration of test

1 minute.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a PROGRESS message containing indication of in-band information availability to the MS. The SS checks that if channel mode is speech, the TCH shall be through connected. If channel mode is not speech, the TCH shall not be through connected. Also the SS checks that the MS has stopped T310, ie at T310 timeout no DISCONNECT message is sent by the MS. Then the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PROGRESS 1)	the MS shall stop all the CC timers (note 12),
				if channel mode is speech, the TCH shall be
				through connected. If channel mode is not
				speech, the TCH shall not be through
				connected.
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U3
	4	SS		SS waits at least 45 seconds and checks
				no DISCONNECT is sent by the MS
	5	SS -> MS	STATUS ENQUIRY	
	6	MS -> SS	STATUS	cause 30#, state U3
	7	SS		If the channel mode is speech
				the SS will check that the user connection
				for speech is attached
				(both downlink and uplink)

Specific message contents:

NOTE 12:	Tested with a valid cause value among:

#1 call is not end to end PLMN/ISDN

#2 destination address is non PLMN/ISDN

#3 originating address is non PLMN/ISDN

#8 in band information or appropriate pattern now available or any value in the set #(6-20).

###### 26.8.1.2.4.5	Outgoing call / U3 MS originating call proceeding / DISCONNECT with in band tones

26.8.1.2.4.5.1	Definition and applicability

The call control entity of the MS being in the state, U3, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT with progress indicator #8, shall through-connect the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

References

TS GSM 04.08 section 5.2.1.4., TS GSM 04.08 section 5.4.4.

26.8.1.2.4.5.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT with progress indicator #8 through-connects the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS sends a RELEASE message.

26.8.1.2.4.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U12, disconnect indication.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. The SS checks that if channel mode is speech, the TCH shall be through connected and the MS enters state U12, disconnect indication. If channel mode is not speech, the TCH shall not be through connected and the MS shall enter state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 13)

TCH in speech mode:
	B2	SS		the SS will check that the audio path
				for in band tones is attached.
	B3	SS -> MS	STATUS ENQUIRY	
	B4	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:
	C2	MS -> SS	RELEASE	
	C3	SS -> MS	STATUS ENQUIRY	
	C4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 13:	the cause value:

#8 in band information or appropriate pattern now available.

###### 26.8.1.2.4.6	Outgoing call / U3 MS originating call proceeding / DISCONNECT without in band tones

26.8.1.2.4.6.1	Definition and applicability

The call control entity of the MS being in the state, U3, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.6.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT without progress indicator shall return a RELEASE message and enter the CC-state U19, "Release Request"

References

TS GSM 04.08 section 5.4.4.

26.8.1.2.4.6.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT without progress indicator returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.4.6.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.4.7	Outgoing call / U3 MS originating call proceeding / RELEASE received

26.8.1.2.4.7.1	Definition and applicability

The call control entity of the MS being in the state, U3, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a RELEASE will return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	The MS on returning to the idle mode shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

3)	On releasing the MM-connection, the MS shall wait for  MM layer release initiated by the network.

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.2., TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

Conformance requirement 3:	TS GSM 04.08, section 5.4.4.3, TS GSM 04.08, section 4.5.3, TS GSM 04.08, section 3.5.1.

26.8.1.2.4.7.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a RELEASE will return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

3)	To verify that in releasing the MM-connection, the MS shall wait for  MM layer release initiated by SS.

26.8.1.2.4.7.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.4.8	Outgoing call / U3 MS originating call proceeding / termination requested by the user

26.8.1.2.4.8.1	Definition and applicability

The call contrl entity of the MS being in the state, U3, the user requests to terminate the call. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.8.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.2.4.8.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.2.4.8.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/3.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The user requests termination of the call. The MS shall send a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1			MMI action, terminate call
	2	MS -> SS	DISCONNECT	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.2.4.9	Outgoing call / U3 MS originating call proceeding / traffic channel allocation

26.8.1.2.4.9.1	Definition and applicability

The call control entity of the MS being in the state, U3, a traffic channel assignment procedure is performed. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.9.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.1.9.

26.8.1.2.4.9.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", when allocated a traffic channel by the network performing the assignment procedure, performs a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.2.4.9.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/3.

Foreseen final state of the MS

U3, mobile originating call proceeding.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	(TCH) the MS shall perform L2 establishment
				on the FACCH
	2	MS -> SS	ASSIGNMENT COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U3

Specific message contents:

None.

###### 26.8.1.2.4.10	Outgoing call / U3 MS originating call proceeding / timer T310 timeout

26.8.1.2.4.10.1	Definition and applicability

The call contrl entity of the MS being in the state, U3, if no response is then received from the SS, timer T310 expires at the MS side. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.10.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" shall, upon expiry of timer T310, and not before, initiate call release by sending DISCONNECT and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.08 section 5.2.1.3./Abnormal case, TS GSM 04.08 section 5.4.3, TS GSM 04.08 section 11.3.

26.8.1.2.4.10.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" will, upon expiry of timer T310 (accuracy minus 2%, plus 50%), initiate call release by sending DISCONNECT and enter the CC-state U11, "Disconnect Request".

26.8.1.2.4.10.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/3.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The T310 expiries at the MS and the MS shall send DISCONNECT. The SS checks timer T310 accuracy and that the CC entity has entered the state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1	SS		the SS waits for T310 timeout
	2	MS -> SS	DISCONNECT	check the timer T310 accuracy
				(minus 2% to plus 50%)
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.2.4.11	Outgoing call / U3 MS originating call proceeding / lower layer failure

26.8.1.2.4.11.1	Definition and applicability

The call control entity of the MS being in the state, U3, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.11.2	Conformance requirement

1)	If a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" has detected a lower layer failure and has returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1.

26.8.1.2.4.11.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" having detected a lower layer failure and having returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.4.11.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U3. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 18-19 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.4.12	Outgoing call / U3 MS originating call proceeding / unknown message received

26.8.1.2.4.12.1	Definition and applicability

The call control entity of the MS being in the state, U3, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.12.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.5.

26.8.1.2.4.12.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" having received an unknown message from its peer entity returns a STATUS message.

26.8.1.2.4.12.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/1.

Foreseen final state of the MS

U3, mobile originating call proceeding.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U3
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U3

Specific message contents:

None.

###### 26.8.1.2.4.13	Outgoing call / U3 MS originating call proceeding / Internal alerting indication

26.8.1.2.4.13.1	Definition and applicability

The call control entity of the MS being in the state, U3, an ALERTING message is sent to the MS when the user connection is not attached to the radio path. This test is applicable for any equipment supporting mobile originated circuit switched basic service for telephony.

26.8.1.2.4.13.2	Conformance requirement

1)	When the call control entity of the MS in the "mobile originating call proceeding" state receives an ALERTING message then it shall enter "call delivered" state and, for speech calls, if the user connection is not attached to the radio path, the MS shall internally generate an alerting indication.

References

TS GSM 04.08 section 5.2.1.5.

26.8.1.2.4.13.3	Test purpose

When the call control entity of the MS in the "mobile originating call proceeding" state receives an ALERTING message then it enters "call delivered" state and, for speech calls, if the user connection is not attached to the radio path, the MS generates internally an alerting indication.

26.8.1.2.4.13.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

-	way to give internally generated alerting indication for outgoing calls

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/1.

Foreseen final state of the MS

U4, call delivered.

Maximum duration of test

30 s.

Test procedure

The SS sends an ALERTING message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U4, call delivered. Also it is checked that the MS generates internally alerting indication to the user in the way described in the PICX/PIXIT statements.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ALERTING	the MS shall generate an alerting indication
				to the user in the way described in the
				PICS/PIXIT statements
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

##### 26.8.1.2.5	Outgoing call / U4 call delivered

###### 26.8.1.2.5.1	Outgoing call / U4 call delivered / CONNECT received

26.8.1.2.5.1.1	Definition and applicability

The call control entity of the MS being in the state, U4, a CONNECT message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the CONNECT message shall return a CONNECT ACKNOWLEDGE to its peer entity and enter the CC-state U10, "Active".

References

TS GSM 04.08 section 5.2.1.6.

26.8.1.2.5.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the CONNECT message returns a CONNECT ACKNOWLEDGE to its peer entity and enters the CC-state U10, "Active".

26.8.1.2.5.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/3.

Foreseen final state of the MS

U10, active.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a CONNECT message to the MS. The MS shall respond by sending a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U10, active.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CONNECT	
	2	MS -> SS	CONNECT ACKNOWLEDGE	MS stops alerting, if applicable
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

###### 26.8.1.2.5.2	Outgoing call / U4 call delivered / termination requested by the user

26.8.1.2.5.2.1	Definition and applicability

The call control entity of the MS being in the state, U4, the user requests to terminate the call. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2.,TS GSM 04.08 section 5.4.3.

26.8.1.2.5.2.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.2.5.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/3.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The user requests termination of the call. The MS shall send a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1			MMI action, terminate call
	2	MS -> SS	DISCONNECT	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.2.5.3	Outgoing call / U4 call delivered / DISCONNECT with in band tones

26.8.1.2.5.3.1	Definition and applicability

The call control entity of the MS being in the state, U4, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered" shall, upon receipt of a DISCONNECT with a progress indicator indicating in-band information, shall through-connect the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

References

TS GSM 04.08 section 3.4.3, TS GSM 04.08 section 5.4.4.1., TS GSM 04.08 section 5.5.1., TS GSM 04.08 section 5.2.1.9.

26.8.1.2.5.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of a DISCONNECT with a progress indicator indicating in-band information, through-connects the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

26.8.1.2.5.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS

U12, disconnect indication.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. The SS checks that if channel mode is MO telephony, the TCH shall be through connected and the MS enters state U12, disconnect indication. If channel mode is not speech, the TCH shall not be through connected and the MS shall enter state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 14)

TCH in speech mode:
	A2	SS		the SS will check that the audio path
				for in band tones is attached.
	A3	SS -> MS	STATUS ENQUIRY	
	A4	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:
	B2	MS -> SS	RELEASE	
	B3	SS -> MS	STATUS ENQUIRY	
	B4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 14:	the Progress Indicator, Progress Description:

#8 in band information or appropriate pattern now available.

###### 26.8.1.2.5.4	Outgoing call / U4 call delivered / DISCONNECT without in band tones

26.8.1.2.5.4.1	Definition and applicability

The call control entity of the MS being in the state, U4, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of a DISCONNECT without progress indicator, shall return a RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.4.

26.8.1.2.5.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of a DISCONNECT without progress indicator, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.5.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.5.5	Outgoing call / U4 call delivered / RELEASE received

26.8.1.2.5.5.1	Definition and applicability

The call control entity of the MS being in the state, U4, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the RELEASE message shall respond with the RELEASE COMPLETE message and enter the CC-state U0, "Null".

2)	The MS on returning to idle mode shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.2., TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.2.5.5.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the RELEASE message will respond with the RELEASE COMPLETE message and enter the CC-state U0, "Null"

2)	To verify that the MS on returning the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.5.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 19-20 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.5.6	Outgoing call / U4 call delivered / lower layer failure

26.8.1.2.5.6.1	Definition and applicability

The call control entity of the MS being in the state, U4, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.6.2	Conformance requirement

1)	When CC-entity of the MS in CC-state U4, "Call Delivered" has detected a lower layer failure and has returned to idle mode, the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1.

26.8.1.2.5.6.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered" having detected a lower layer failure and has returned to idle mode, the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.5.6.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U4. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.5.7	Outgoing call / U4 call delivered / traffic channel allocation

26.8.1.2.5.7.1	Definition and applicability

The call control entity of the MS being in the state, U4, a traffic channel assignment procedure is performed. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.1.9.

26.8.1.2.5.7.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.2.5.7.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/1.

Foreseen final state of the MS

U4, call delivered.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH, the MS shall perform L2 establishment
				on the FACCH
	2	MS -> SS	ASSIGNMENT COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

###### 26.8.1.2.5.8	Outgoing call / U4 call delivered / unknown message received

26.8.1.2.5.8.1	Definition and applicability

The call control entity of the MS being in the state, U4, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.8.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.2.5.8.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.2.5.8.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/4.

Foreseen final state of the MS

U4, call delivered.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U4
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

##### 26.8.1.2.6	U10 call active

###### 26.8.1.2.6.1	U10 call active / termination requested by the user

26.8.1.2.6.1.1	Definition and applicability

The call contrl entity of the MS being in the state, U10, the user requests to terminate the call. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.2.6.1.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U10, "Call Active", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.2.6.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/1.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The user requests termination of the call. The MS shall send a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1			MMI action, terminate call
	2	MS -> SS	DISCONNECT	U11
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.2.6.2	U10 call active / RELEASE received

26.8.1.2.6.2.1	Definition and applicability

The call control entity of the MS being in the state, U10, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon receipt of the RELEASE shall respond with the RELEASE COMPLETE message and enter the CC-state U0, "Null"

2)	When the MS returns to the idle mode it shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null"

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.2., TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.2.6.2.3	Test purpose

1)	To verify that the a CC-entity of the MS in CC-state U10, "Call Active", upon receive of the RELEASE will respond with the RELEASE COMPLETE message and enter the CC-state U0, "Null"

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null"

26.8.1.2.6.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	the MS starts T3240
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA)

Specific message contents:

None.

###### 26.8.1.2.6.3	U10 call active / DISCONNECT with in band tones

26.8.1.2.6.3.1	Definition and applicability

The call control entity of the MS being in the state, U10, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message with a Progress Indicator indicating in-band information, shall through-connect the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

References

TS GSM 04.08 section 5.4.4.1., TS GSM 04.08 section 5.5.1.

26.8.1.2.6.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message with a Progress Indicator indicating in-band information, through-connects the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS sends a RELEASE message.

26.8.1.2.6.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/2.

Foreseen final state of the MS

U12, disconnect indication.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. The SS checks that if channel mode is speech, the TCH shall be through connected and the MS enters state U12, disconnect indication. If channel mode is not speech, the TCH shall not be through connected and the MS enters state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 15)

TCH in speech mode:
	A2	SS		the SS will check that the audio path
				for in band tones is attached.
	A3	SS -> MS	STATUS ENQUIRY	
	A4	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:
	B2	MS -> SS	RELEASE	
	B3	SS -> MS	STATUS ENQUIRY	
	B4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 15:	the Progress Indicator, Progress Description:

#8 in band information or appropriate pattern now available.

###### 26.8.1.2.6.4	U10 call active / DISCONNECT without in band tones

26.8.1.2.6.4.1	Definition and applicability

The call control entity of the MS being in the state, U10, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message without progress indicator, shall return a RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.4.

26.8.1.2.6.4.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message without progress indicator, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.6.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/2.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.6.5	U10 call active / RELEASE COMPLETE received

26.8.1.2.6.5.1	Definition and applicability

The call control entity of the MS being in the state, U10, the call is cleared by a RELEASE COMPLETE message sent by the SS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.5.2	Conformance requirement

1)	A CC entity of the MS in CC-state U10, "active", upon receipt of a RELEASE COMPLETE message with valid cause value, shall enter CC state U0, "Null".

2)	On returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08, section 5.4.2, TS GSM 04.08, section 5.4.4.

Conformance requirement 2:	TS GSM 04.08, section 5.4.4.3.

26.8.1.2.6.5.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U10, "Call active" upon receipt of a RELEASE COMPLETE message with valid cause value, enters CC state U0, "Null".

2)	To verify that in returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.6.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

30 s.

Test procedure

The SS sends a RELEASE COMPLETE message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE COMPLETE	note 16
	2	SS -> MS	STATUS ENQUIRY	note 17
	3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value), 
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000...110
	5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

NOTE 16:	With the cause value chosen arbitrarily.

NOTE 17:	TI flag has the value indicating the MS as a originator of the call.

###### 26.8.1.2.6.6	U10 call active / SETUP received

26.8.1.2.6.6.1	Definition and applicability

If the MS does not react correctly when receiving a SETUP message on a new Transaction Identifier during an active call, the active call may be lost.

This test is applicable for all GSM900 and DCS1800 MS supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.6.2	Conformance requirement

1)	A Mobile Station that has a call established when receiving a SETUP message shall respond either with a CALL CONFIRMED message or a RELEASE COMPLETE message, both with cause #17 "user busy".

2)	The call control state of the existing transaction shall not be affected by the incoming SETUP message.

Reference(s):

Conformance requirement 1:	TS GSM 04.08, section 5.2.2.3.1.

Conformance requirement 2:	TS GSM 04.08, section 5.1.1.

26.8.1.2.6.6.3	Test purpose

1)	To verify that a Mobile Station that has a call established and receives a SETUP message answers either with a CALL CONFIRMED message with cause "user busy" if it supports call waiting, or with a RELEASE COMPLETE message with cause "user busy" otherwise.

2)	To verify that after having sent this message, the MS is still in state U10 for the established call.

26.8.1.2.6.6.4	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is idle updated with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/14.

Related PICS/PIXIT statement(s)

- supported MO circuit switched basic services

- support of call waiting Y/N

Foreseen final state of the MS

U10, call active.

Maximum duration of test

30 s.

Test Procedure

The MS has a mobile originated call in the U10 state.

The SS sends a SETUP message to the MS (with signal IE indicating "call waiting tone on").

If the MS does not support call waiting it shall answer by a RELEASE COMPLETE message.

If the MS supports call waiting it shall answer by a CALL CONFIRMED message followed by an ALERTING. The second transaction is then released by the SS with a RELEASE COMPLETE message.

In both cases the SS checks by using the status enquiry procedure that the CC entity of the MS is still in state U10, active call for the original call.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	SETUP	this message establishes a second transaction
				The TI value shall be the same as the
				one that is in use for the MO call.
				The TI flag shall have the value
				specified for an MT call.

if the MS does not support call waiting
	A2	MS -> SS	RELEASE COMPLETE	with cause user busy"
				with the TI of the second transaction

if the MS supports call waiting
	B2	MS -> SS	CALL CONFIRMED	with cause user busy"
				with the TI of the second transaction
	B3	MS -> SS	ALERTING	with the TI of the second transaction
	B4	SS -> MS	RELEASE COMPLETE	with the TI of the second transaction

5	SS -> MS	STATUS ENQUIRY	with the TI of the original transaction
	6	MS -> SS	STATUS	cause 30#, state U10
				with the TI of the original transaction

NOTE:	The Transaction Identifier of the second transaction shall be different from the one of the already established transaction.

Specific message contents

SETUP message contains a Signal IE with value "call waiting tone on" (H'07).

##### 26.8.1.2.7	U11 disconnect request

###### 26.8.1.2.7.1	U11 disconnect request / clear collision

26.8.1.2.7.1.1	Definition and applicability

The call control entity of the MS being in the state, U11, a DISCONNECT message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.7.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of a DISCONNECT message, shall return to its peer entity the RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.5.

26.8.1.2.7.1.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of a DISCONNECT message, returns to its peer entity the RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.7.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/3.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. The SS sends a DISCONNECT message to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.7.2	U11 disconnect request / RELEASE received

26.8.1.2.7.2.1	Definition and applicability

The call control entity of the MS being in the state, U11, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.7.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of the RELEASE message shall return RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.3.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.2.7.2.3	Test purpose

1)	To verify that the a CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of the RELEASE message shall return RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.7.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/3.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.7.3	U11 disconnect request / timer T305 timeout

26.8.1.2.7.3.1	Definition and applicability

The call control entity of the MS being in the state, U11, if no response is then received from the SS, timer T305 expires at the MS side. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.7.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request" shall on expiry of T305, proceed with the connection release procedure by sending the RELEASE message to its peer entity and shall enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.3., TS GSM 04.08 section 11.3.

26.8.1.2.7.3.3	Test purpose

To verify that the CC-entity of the MS in CC-state U11, "Disconnect Request" shall on expiry of T305 (accuracy +/- 10%), proceed with the connection release procedure by sending the RELEASE message to its peer entity and enters the CC-state U19, "Release Request".

26.8.1.2.7.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/3.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

1 minute.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. Then T305 expires at the MS and the MS shall send a RELEASE message. The SS checks timer T305 accuracy and that the CC entity has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS waits until T305 expires at the MS
	2	MS -> SS	RELEASE	SS checks the time between DISCONNECT
				and RELEASE (note 18),
				(T305 +/- 10%)
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 18:	With the same cause value as originally contained in the DISCONNECT message. An additional cause information element (#102 recovery on timer expiry) may be included.

###### 26.8.1.2.7.4	U11 disconnect request / lower layer failure

26.8.1.2.7.4.1	Definition and applicability

The call control entity of the MS being in the state, U11, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.7.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request" having detected a lower layer failure shall return to the idle mode. The CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1., TS GSM 04.08 section 8.3.

26.8.1.2.7.4.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U11, "Disconnect Request" having detected a lower layer failure returns to the idle mode. The CC entities relating to the seven mobile originating transaction identifiers are thus in state U0, "Null".

26.8.1.2.7.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U11. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.7.5	U11 disconnect request / unknown message received

26.8.1.2.7.5.1	Definition and applicability

The call control entity of the MS being in the state, U4, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.7.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U100, "Call Delivered", having received an unknown message from its peer rntty shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.2.7.5.3	Test purpose

To verify that a CC-entity of the MS in CC-stste U11, "Call Delivered", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.2.7.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/4.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U11
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

##### 26.8.1.2.8	U12 disconnect indication

###### 26.8.1.2.8.1	U12 disconnect indication / call releasing requested by the user

26.8.1.2.8.1.1	Definition and applicability

The call control entity of the MS being in the state, U12, the user requests to terminate the call. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U12, "Disconnect Indication" being in network initiated call release phase, shall, upon receiving a call release request from the user send a RELEASE to its peer entity and enter CC-state U19, "Release Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.4.

26.8.1.2.8.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication" being in network initiated call release phase, shall, upon receiving a call release request from the user sends a RELEASE to its peer entity and enters CC-state U19, "Release Request"

26.8.1.2.8.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using Option A of table 26.8.1.2/1.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U12. The user requests termination of the call. The MS shall send a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1			MMI action, "on hook"
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.8.2	U12 disconnect indication / RELEASE received

26.8.1.2.8.2.1	Definition and applicability

The call control entity of the MS being in the state, U12, a RELEASE message is received by the MS. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U12, "Disconnect Indication", upon receipt of a RELEASE message shall return to its peer entity the RELEASE COMPLETE message and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.2

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.2.8.2.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication", upon receipt of a RELEASE message returns to its peer entity the RELEASE COMPLETE message and enters the CC-state U0, "Null"

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.8.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using Option A of table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U12. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.8.3	U12 disconnect indication / lower layer failure

26.8.1.2.8.3.1	Definition and applicability

The call control entity of the MS being in the state, U12, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U12, "Disconnect Indication" having detected a lower layer failure shall return to idle mode. The CC-entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

References

TS GSM 04.08 section 4.5.3.2., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1., TS GSM 04.08 section 8.3.

26.8.1.2.8.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication" having detected a lower layer failure returns to idle mode. The CC-entities relating to the seven mobile originating transaction identifiers are thus in state U0, "Null".

26.8.1.2.8.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using Option A of table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U12. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.8.4	U12 disconnect indication / unknown message received

26.8.1.2.8.4.1	Definition and applicability

The call control entity of the MS being in the state, U12, an unknown message is received by the MS. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.4.2	Conformance requirement

A CC-entity of the MS in CC-state U12, "Disconnect Indication" having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.2.8.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication" having received an unknown message from its peer entity returns a STATUS message.

26.8.1.2.8.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using Option A of table 26.8.1.2/3.

Foreseen final state of the MS

U12, disconnect indication.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U12. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U12
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U12

Specific message contents:

None.

##### 26.8.1.2.9	Outgoing call / U19 release request

###### 26.8.1.2.9.1	Outgoing call / U19 release request / timer T308 timeout

26.8.1.2.9.1.1	Definition and applicability

The call control entity of the MS being in the state, U19, if no response is then received from the SS, timer T308 expires at the MS side. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.9.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request" will, upon the first expiry of timer T308 send the RELEASE message to its peer entity and remain in the CC-state U19.

References

TS GSM 04.08 section 5.4.4.3.1, TS GSM 04.08 section 11.3.

26.8.1.2.9.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request" will, upon the first expiry of timer T308 (accuracy +/- 10%) send the RELEASE message to its peer entity and remain in the CC-state U19.

26.8.1.2.9.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/4.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. When T308 expiries at the MS, the MS shall send a RELEASE message. The SS checks timer T308 accuracy and that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS waits until T308 at the MS
	2	MS -> SS	RELEASE	SS checks the time between the two
				RELEASE messages (T308 +/- 10%)
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.9.2	Outgoing call / U19 release request / 2nd timer T308 timeout

26.8.1.2.9.2.1	Definition and applicability

The call control entity of the MS being in the state, U19, if no response is then received after timer T308 has expired two times in success at the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.9.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request", upon the 2nd expiry of the timer T308, shall enter the CC-state U0, "Null".

2)	Subsequently the MS shall proceed with releasing the MM-connection and enter the idle mode with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.3.1., TS GSM 04.08 section 11.3.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.2.9.2.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U19, "Release Request", upon the 2nd expiry of the timer T308, enters the CC-state U0, "Null".

2)	To verify that subsequently the MS proceeds with releasing the MM-connection and enters the idle mode with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

26.8.1.2.9.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

2 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. The SS allows T308 expiry at the MS, and the MS shall repeat sending the RELEASE message and start timer T308 again. The SS allows again T308 expiry at the MS. The MS shall abort the RR-connection (DISC/UA). The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS waits until T308 expiry at the MS
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19
	5	SS		SS waits until the second T308 expiry
				at the MS
	6	SS		SS waits T3240 expiry at the MS
	7	MS		the main signalling link shall be
				released by the MS (L2: DISC/UA).
	8	SS		SS waits 10 s for the MS to return to
				listening to paging
	9	SS -> MS	PAGING REQUEST	
	10	MS -> SS	CHANNEL REQUEST	
	11	SS -> MS	IMMEDIATE ASSIGNMENT	
	12	MS -> SS	PAGING RESPONSE	
	13	SS -> MS	STATUS ENQUIRY	
	14	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	15	SS		repeat steps 13-14 to cover all the
				transaction identifiers from 000...110
	16	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.9.3	Outgoing call / U19 release request / RELEASE received

26.8.1.2.9.3.1	Definition and applicability

The call control entity of the MS being in the state, U19, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.9.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE, shall release the MM-connection and enter the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

References

TS GSM 04.08 section 5.4.5., TS GSM 04.08 section 11.3, TS GSM 04.08 section 5.5.3.2.

26.8.1.2.9.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE, shall release the MM-connection and enters the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

26.8.1.2.9.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. The SS sends a RELEASE message to the MS. The MS shall release the MM-connection. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	(note 19)
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000...110
	5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

NOTE 19:	With the same cause number as originally contained in DISC and optional cause #102 recovery on timer expiry.

###### 26.8.1.2.9.4	Outgoing call / U19 release request / RELEASE COMPLETE received

26.8.1.2.9.4.1	Definition and applicability

The call control entity of the MS being in the state, U19, a RELEASE COMPLETE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.9.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE COMPLETE, shall release the MM-connection and enter the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

References

TS GSM 04.08 section 5.4.4.3., TS GSM 04.08 section 4.5.3, TS GSM 04.08 section 8.3.

26.8.1.2.9.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE COMPLETE, shall release the MM-connection and enters the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

26.8.1.2.9.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. The SS sends a RELEASE COMPLETE message to the MS. The MS shall release the MM-connection. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE COMPLETE	
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000...110
	5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.9.5	Outgoing call /  U19 release request / lower layer failure

26.8.1.2.9.5.1	Definition and applicability

The call control entity of the MS being in the state, U19, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.9.5.2	Conformance requirement

A CC-entity of the MS in CC-state U19, "Release Request", having detected a lower layer failure, shall return to the idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.2.9.5.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request", having detected a lower layer failure, returns to the idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.9.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U19. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

#### 26.8.1.3	Establishment of an incoming call / Initial conditions

The tables below describe message exchanges which bring the MS in the requested initial states in case of an incoming call.

A state may be taken as initial only when all the states which lead to this initial states have been validated. The order will be U0, U6, U9, U7, U8, U10, U26 etc. as in the following tables.

Table 26.8.1.3/1: Establishment of an incoming call, procedure 1

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, SDCCH
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	AUTH REQUEST	
	6	MS -> SS	AUTH RESPONSE	
	7	SS -> MS	CIPH MODE COMMAND	
	8	MS -> SS	CIPH MODE COMPLETE	
	9	SS -> MS	SETUP	U6, (note 20)
	10	MS -> SS	CALL CONFIRMED	U9

A11	MS -> SS	CONNECT	U8, p = Y, (note 21)

B11	MS -> SS	ALERTING	U7, p = N, (note 21)
	B12	MS		(note 22)
	B13	MS -> SS	CONNECT	U8

14	SS -> MS	ASSIGNMENT COMMAND	TCH
	15	MS -> SS	ASSIGNMENT COMPLETE	
	16	SS -> MS	CONNECT ACKNOWLEDGE	U10

NOTE 20:	With signal information included in the SETUP message.

NOTE 21:	The MS is supporting immeditate connect (p = Y/N). See PICS/PIXIT statement.

NOTE 22:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

Table 26.8.1.3/2: Establishment of an incoming call, procedure 2

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, SDCCH
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	CIPH MODE COMMAND	
	6	MS -> SS	CIPH MODE COMPLETE	
	7	SS -> MS	SETUP	U6, (note 23)
	8	MS -> SS	CALL CONFIRMED	U9

A9	MS -> SS	CONNECT	U8, p = Y, (note 24)
	A10	SS -> MS	ASSIGNMENT COMMAND	TCH
	A11	MS -> SS	ASSIGNMENT COMPLETE

B9	MS -> SS	ALERTING	U7, p = N, (note 24)
	B10	SS -> MS	ASSIGNMENT COMMAND	TCH
	B11	MS -> SS	ASSIGNMENT COMPLETE	
	B12	MS		(note 25)
	B13	MS -> SS	CONNECT	U8

14	SS -> MS	AUTH REQUEST	
	15	MS -> SS	AUTH RESPONSE	
	16	SS -> MS	CONNECT ACKNOWLEDGE	U10

NOTE 23:	With signal information included in the SETUP message.

NOTE 24:	The MS is supporting immediate connect (p = Y/N). See PICS/PIXIT statement.

NOTE 25:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

Table 26.8.1.3/3: Establishment of an incoming call, procedure 3

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, TCH
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	AUTH REQUEST	
	6	MS -> SS	AUTH RESPONSE	
	7	SS -> MS	CIPH MODE COMMAND	
	8	MS -> SS	CIPH MODE COMPLETE	
	9	SS -> MS	CHANNEL MODE MODIFY	(note 26)
	10	MS -> SS	CHANNEL MODE MOD ACK	
	11	SS -> MS	SETUP	U6, (note 27)
	12	MS -> SS	CALL CONFIRMED	U9

A13	MS -> SS	CONNECT	U8, p = Y, (note 28)

B13	MS -> SS	ALERTING	U7, p = N, (note 28)
	B14	MS		(note 29)
	B15	MS -> SS	CONNECT	U8

16	SS -> MS	CONNECT ACKNOWLEDGE	U10

NOTE 26:	Assigned channel is appropriate for the chosen mobile originated circuit switched basic service.

NOTE 27:	With signal information included in the SETUP message.

NOTE 28:	The MS is supporting immeditate connect (p = Y/N). See PICS/PIXIT statement.

NOTE 29:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

Table 26.8.1.3/4: Establishment of an incoming call, procedure 4

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, SDCCH
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	CIPH MODE COMMAND	
	6	MS -> SS	CIPH MODE COMPLETE	
	7	SS -> MS	SETUP	U6, (note 30)
	8	MS -> SS	CALL CONFIRMED	U9
	9	SS -> MS	ASSIGNMENT COMMAND	TCH
	10	MS -> SS	ASSIGNMENT COMPLETE

A11	MS -> SS	CONNECT	U8, p = Y, (note 31)

B11	MS -> SS	ALERTING	U7, p = N, (note 31)
	B12	MS		(note 32)
	B13	MS -> SS	CONNECT	U8

14	SS -> MS	AUTH REQUEST	
	15	MS -> SS	AUTH RESPONSE	
	16	SS -> MS	CONNECT ACKNOWLEDGE	U10

NOTE 30:	The signal information element is not included in the SETUP message.

NOTE 31:	The MS is supporting immeditate connect (p = Y/N). See PICS/PIXIT statement.

NOTE 32:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

##### 26.8.1.3.1	Incoming call / U0 null state

###### 26.8.1.3.1.1	Incoming call / U0 null state / SETUP received with a non supported bearer capability

26.8.1.3.1.1.1	Definition and applicability

The call control entity of the MS being in the state, U0, a SETUP message is received with only one bearer capability and this bearer capability is not supported by the MS. This test is applicable for all equipment.

26.8.1.3.1.1.2	Conformance requirement

1)	A CC entity of the MS, upon receipt of SETUP containing one bearer capability and this bearer capability is not supported, shall return a RELEASE COMPLETE with correct cause value to its peer entity and return to the idle mode. The CC-entities relating to the seven mobile terminating transaction identifiers shall be in the state U0,"Null".

References

TS GSM 04.08 section 5.2.2.2., TS GSM 04.08 Annex B.

26.8.1.3.1.1.3	Test purpose

To verify that a CC entity of the MS, upon receipt of SETUP containing one bearer capability and this bearer capability is not supported, returns a RELEASE COMPLETE with correct cause value to its peer entity, and returns to the idle mode. To verify that the CC-entities relating to the seven mobile terminating transaction identifiers are then in the state U0,"Null".

26.8.1.3.1.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

U0, null.

Maximum duration of test

30 s.

Test procedure

A mobile terminated call is initiated. The MS receives a SETUP message that contains a bearer capability not supported by the MS. The MS returns a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity is still in the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	SS sends paging
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	(SDCCH)	
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	AUTH REQUEST	
	6	MS -> SS	AUTH RESPONSE	
	7	SS -> MS	CIPH MODE COMMAND	
	8	MS -> SS	CIPH MODE COMPLETE	
	9	SS -> MS	SETUP	(note 33)
	10	MS -> SS	RELEASE COMPLETE	(note 34)
	11	SS -> MS	STATUS ENQUIRY
	12	MS ->SS	RELEASE COMPLETE	Cause 81# (invalid TI value).
	13	SS		Repeat steps 11-12 to cover all the
				transaction identifiers from 000... 110.

Specific message contents:

NOTE 33:	With one bearer capability and that bearer capability is not supported by the MS

NOTE 34:	With cause #88 incompatible destination.

##### 26.8.1.3.2.	Incoming call / U6 call present

###### 26.8.1.3.2.1	Incoming call / U6 call present / automatic call rejection

26.8.1.3.2.1.1	Definition and applicability

Although the state U6 is transient, the ability to refuse a call (automatically) in this state is tested, if it is implemented at the MS. The test is applicable for those equipments described above supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.2.1.2	Conformance requirement

1)	A CC entity of the MS in CC-state U6, "Call Present", upon receipt of a rejection indication of the incoming call from the user, send RELEASE COMPLETE with the appropriate cause value to its peer entity and enter the CC-state U0, "Null". The CC entities relating to the seven mobile terminating transaction identifiers shall be in state U0, "Null".

References

TS GSM 11.10, Annex B (for PICS/PIXIT statement), TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.2.2.3.1., TS GSM 04.08 section 5.5.3.2, TS GSM 04.08 section 8.3.

26.8.1.3.2.1.3	Test purpose

To verify that a CC entity of the MS in CC-state U6, "Call Present", shall upon receipt of a rejection indication of the incoming call from the user, shall send RELEASE COMPLETE with the appropriate cause value to its peer entity and enter the CC-state U0, "Null". The CC entities relating to the seven mobile terminating transaction identifiers are then in state U0, "Null".

26.8.1.3.2.1.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	the MS supports an ability to refuse a call after receipt of a SETUP message

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U6 by using table 26.8.1.3/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The call control entiry of the MS is brought to the state U6 (Note: The state U6 is not checked, since it is not stable). The MS is made to refuse the call (the refusal may require some preliminary preparations in order to achieve refusal at this point). The MS shall send a RELEASE COMPLETE message and enter a call control state U0. The SS checks by using the status enquiry procedure that the CC entity has entered the state state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1			the MS is made to refuse the call
	2	MS -> SS	RELEASE COMPLETE	(note 35)
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

NOTE 35:	With cause value #21 call rejected.

##### 26.8.1.3.3	Incoming call / U9 mobile terminating call confirmed

###### 26.8.1.3.3.1	Incoming call / U9 mobile terminating call confirmed / alerting or immediate connecting

26.8.1.3.3.1.1	Definition and applicability

The call control entity of the MS having entered the state, U9, with signal information received in the preceeding SETUP message, the subsequent behaviour of the MS is tested. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.3.1.2	Conformance requirement

1)	A CC entity in CC-state U9, "MS Terminating Call Confirmed", (if signalled by the network in previous SETUP message that it may alert) shall either send a ALERTING message to its peer entity and enter state U7, or send a CONNECT message to its peer entity and enter U8.

References

TS GSM 04.08 section 5.2.2.3.2., TS GSM 11.10, Annex 3 - section 2.2.

26.8.1.3.3.1.3	Test purpose

To verify that a CC entity in CC-state U9, "MS Terminating Call Confirmed", (if signalled by the network in previous SETUP message that it may alert) will either send a ALERTING message to its peer entity and enter state U7, or send a CONNECT message to its peer entity and enter U8.

26.8.1.3.3.1.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/2.

Foreseen final state of the MS

-	U8, connect request, if the MS supports immediate connect for the selected basic service

-	otherwise U7, call received

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9 by using a SETUP message containing signalling information element. (The state U9 is not a stable state in this case, and consequently it is not checked as an initial state.) If the MS supports immediate connect for the selected basic service (p = Y), it sends a CONNECT message and enters the state U8, connect request. Otherwise (p = N) the MS sends an ALERTING message and enters the state U7, call receiving. The SS checks by using the status enquiry procedure that the CC entity has entered its state as described.

Expected sequence

Step	Direction	Message	Comments

A11	MS -> SS	CONNECT	p = Y
	A12	SS -> MS	STATUS ENQUIRY	
	A13	MS -> SS	STATUS	cause 30#, state U8

B11	MS -> SS	ALERTING	p = N
	B12	SS -> MS	STATUS ENQUIRY	
	B13	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

###### 26.8.1.3.3.2	Incoming call / U9 mobile terminating call confirmed / TCH assignment

26.8.1.3.3.2.1	Definition and applicability

The call control entity of the MS being in the state, U9, an assignment procedure is performed for traffic channel. This test is applicable for any equipment supporting at least one MT circuit switched basic service, for which immediate connect is not used.

26.8.1.3.3.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH, send a ALERTING message and enter state U7.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.2.7., TS GSM 04.08 section 5.2.2.3.2.

26.8.1.3.3.2.3	Test purpose

To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", when allocated a traffic channel by the network performing the assignment procedure, performs a layer 2 establishment on the FACCH, sends a ALERTING message and enters state U7.

26.8.1.3.3.2.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U9, mobile terminating call confirmed.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9 (by using a SETUP message not containing the signal information element). The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The MS sends an ALERTING message and enters state U7, call received. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH, an appropriate non-signalling mode
	2	MS		the MS shall establish L2 link
	3	MS -> SS	ASSIGNMENT COMPLETE	
	4	MS -> SS	ALERTING	
	5	SS -> MS	STATUS ENQUIRY	
	6	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

###### 26.8.1.3.3.3	Incoming call / U9 mobile terminating call confirmed / termination requested by the user

26.8.1.3.3.3.1	Definition and applicability

The call control entity of the MS being in the state, U9, the user requests for releasing of the call. This test is applicable for any equipment supporting at least one MT circuit switched basic service for which immediate connection is not used and, in addition to this, the facility to send a DISCONNECT message in state U9.

26.8.1.3.3.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.3.3.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.3.3.3.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

-	the MS supports user requested call clearing in the state U9

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9 (by using a SETUP message not containing the signal information element). Then the user requests termination of the call, if possible. The MS sends a DISCONNECT message and enters state U11, disconnect request. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence

Step	Direction	Message	Comments

1			the MS is made to clear the call
	2	MS -> SS	DISCONNECT	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.3.3.4	Incoming call / U9 mobile terminating call confirmed / DISCONNECT received

26.8.1.3.3.4.1	Definition and applicability

The call control entity of the MS being in the state, U9, a DISCONNECT message is received by the MS. This test is applicable for any equipment supporting at least one MT circuit switched basic service, for which immediate connect is not used.

26.8.1.3.3.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a DISCONNECT shall return a RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.4.

26.8.1.3.3.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a DISCONNECT returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.3.3.4.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9. The SS sends a DISCONNECT message to the MS. The MS responds by sending a RELEASE message and enters state U19, release request. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.3.3.5	Incoming call / U9 mobile terminating call confirmed / RELEASE received

26.8.1.3.3.5.1	Definition and applicability

The call control entity of the MS being in the state, U9, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one MT circuit switched basic service, for which immediate connect is not used.

26.8.1.3.3.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a RELEASE shall return a RELEASE COMPLETE and enter the CC-state U0, "Null".

TS GSM 04.08 section 5.4.4.

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile terminating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.3.3.5.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a RELEASE will return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile terminating transaction identifiers are in CC-state U0, "Null".

26.8.1.3.3.5.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9. The SS sends a RELEASE message to the MS. The MS responds by sending a RELEASE COMPLETE message and enters state U0, null. The SS verifies by using the status enquiry procedure that the MS has entered the correct state with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.3.6	Incoming call / U9 mobile terminating call confirmed / lower layer failure

26.8.1.3.3.6.1	Definition and applicability

The call control entity of the MS being in the state, U9, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one MT circuit switched basic service, for which immediate connect is not used.

26.8.1.3.3.6.2	Conformance requirement

1)	A CC entity of the MS in CC-state U9, "MS Terminating Call Confirmed", having detected a lower layer failure shall return to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3

26.8.1.3.3.6.3	Test purpose

To verify that a CC entity of the MS in CC-state U9, "MS Terminating Call Confirmed", having detected a lower layer failure returns to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

26.8.1.3.3.6.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The MS is brought to the state U9. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.3.7	Incoming call / U9 mobile terminating call confirmed / unknown message received

26.8.1.3.3.7.1	Definition and applicability

The call control entity of the MS being in the state, U9, an unknown message is received by the MS. This test is applicable for any equipment supporting at least MT circuit switched basic service, for which immediate connect is not used.

26.8.1.3.3.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed" having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.3.3.7.3	Test purpose

To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed" having received an unknown message from its peer entity returns a STATUS message.

26.8.1.3.3.7.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U9, mobile terminating call proceeding.

Maximum duration of test

30 s.

Test procedure

A MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U9
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U9

Specific message contents:

None.

##### 26.8.1.3.4.	Incoming call / U7 call received

###### 26.8.1.3.4.1	Incoming call / U7 call received / call accepted

26.8.1.3.4.1.1	Definition and applicability

The call control entity of the MS being in the state, U7, a user accepts the incoming call. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.1.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon a user accepting the incoming call, shall send a CONNECT message to its peer entity and enter the CC-state U8, "Connect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.2.2.5.

26.8.1.3.4.1.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", upon a user accepting the incoming call, shall send a CONNECT message to its peer entity and enter the CC-state U8, "Connect Request"

26.8.1.3.4.1.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/3.

Foreseen final state of the MS

U8, connect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The user accepts the incoming call. The MS sends a CONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered state U8, connect request.

Expected sequence

Step	Direction	Message	Comments

1			the MS is made to accept the call by the user
	2	MS -> SS	CONNECT	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U8

Specific message contents:

None.

###### 26.8.1.3.4.2	Incoming call / U7 call received / termination requested by the user

26.8.1.3.4.2.1	Definition and applicability

The call control entity of the MS being in the state, U7, a user requests to terminate incoming call. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.2.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.3.4.2.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.3.4.2.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/3.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The user initiates clearing the incoming call. The MS sends a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1			the MS is made to terminate/reject the call
	2	MS -> SS	DISCONNECT	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.3.4.3	Incoming call / U7 call received / DISCONNECT received

26.8.1.3.4.3.1	Definition and applicability

The call control entity of the MS being in the state, U7, a DISCONNECT message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.3.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon receipt of a DISCONNECT with a progress indicator indicating in-band information from network, if a TCH was not assigned, shall return a RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.4.

26.8.1.3.4.3.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", upon receipt of a DISCONNECT with a progress indicator indicating in-band information from network, if a TCH was not assigned, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.3.4.3.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/1.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends a DISCONNECT message. The MS responds with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity has entered state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 36)
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 36:	With a progress indicator indicating in-band information; Progress Indicator, Progress Description #8.

###### 26.8.1.3.4.4	Incoming call / U7 call received / RELEASE received

26.8.1.3.4.4.1	Definition and applicability

The call control entity of the MS being in the state, U7, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.4.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon receipt of a RELEASE shall return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile terminating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.3.4.4.3	Test purpose

1)	To verify that a CC entity of a MS in CC-state U7, "Call Received", upon receipt of a RELEASE will return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile terminating transaction identifiers are in CC-state U0, "Null".

26.8.1.3.4.4.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends a RELEASE message. The MS responds with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered state U0, null, with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.4.5	Incoming call / U7 call received / lower layer failure

26.8.1.3.4.5.1	Definition and applicability

The call control entity of the MS being in the state, U7, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.5.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", having detected a lower layer failure shall return to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.3.4.5.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", having detected a lower layer failure returns to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

26.8.1.3.4.5.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The MS is brought to the state U7. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.4.6	Incoming call / U7 call received / unknown message received

26.8.1.3.4.6.1	Definition and applicability

The call control entity of the MS being in the state, U7, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.6.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.3.4.6.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.3.4.6.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/3.

Foreseen final state of the MS

U7, call received.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U7
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

###### 26.8.1.3.4.7	Incoming call / U7 call received / TCH assignment

26.8.1.3.4.7.1	Definition and applicability

The call control entity of the MS being in the state, U7, an assignment procedure is performed for traffic channel. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.7.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.2.7.

26.8.1.3.4.7.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.3.4.7.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/1.

Foreseen final state of the MS

U7, call received.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS verifies by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH
	2	MS		the MS shall establish L2 link
	3	MS -> SS	ASSIGNMENT COMPLETE	
	4	SS -> MS	STATUS ENQUIRY	
	5	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

###### 26.8.1.3.4.8	Incoming call / U7 call received / RELEASE COMPLETE received

26.8.1.3.4.8.1	Definition and applicability

The call control entity of the MS being in the state, U7, the call is cleared by a RELEASE COMPLETE message sent by the SS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service, for which immediate connect is not used.

26.8.1.3.4.8.2	Conformance requirement

1)	A CC entity of the MS in CC-state U7, "call received", upon receipt of a RELEASE COMPLETE message with valid cause value, shall enter CC state U0, "Null".

2)	On returning to idle mode, the CC entities relating to the seven mobile terminating transaction identifiers shall be in state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08, section 5.4.2, TS GSM 04.08, section 5.4.4.

Conformance requirement 2:	TS GSM 04.08, section 5.4.4.3.

26.8.1.3.4.8.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U7, "Call received", upon receipt of a RELEASE COMPLETE message with valid cause value, enters CC state U0, "Null".

2)	To verify that in returning to idle mode, the CC entities relating to the seven mobile terminating transaction identifiers are in state U0, "Null".

26.8.1.3.4.8.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services.

MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected service is telephony. If necessary, the MS is configured for that basic service. The a mobile terminated call is initiated. the CCentity of the MS is brought to U7. The SS sends a RELEASE COMPLETE message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE COMPLETE	note 37
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value), note 38
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000...110
	5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

NOTE 37:	With the cause value chosen arbitrarily.

NOTE 38:	TI flag has the value indicating the SS as a originator of the call.

##### 26.8.1.3.5	Incoming call / U8 connect request

###### 26.8.1.3.5.1	Incoming call / U8 connect request / CONNECT acknowledged

26.8.1.3.5.1.1	Definition and applicability

The call control entity of the MS being in the state, U8, a CONNECT ACKNOWLEDGE message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.1.2	Conformance requirement

A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of CONNECT ACKNOWLEDGE shall enter the CC-state U10, "Call Active".

References

TS GSM 04.08 section 5.2.2.6.

26.8.1.3.5.1.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of CONNECT ACKNOWLEDGE shall enter the CC-state U10, "Call Active".

26.8.1.3.5.1.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/2.

Foreseen final state of the MS

U10, active.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8 (if the MS uses immediate connection for the selected basic service then p = Y, otherwise p = N). The SS sends a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered state U10, active.

Expected sequence

Step	Direction	Message	Comments

A1	SS -> MS	ASSIGNMENT COMMAND	p = Y
	A2	MS -> SS	ASSIGNMENT COMPLETE

3	SS -> MS	CONNECT ACKNOWLEDGE	
	4	SS -> MS	STATUS ENQUIRY	
	5	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

###### 26.8.1.3.5.2	Incoming call / U8 connect request / timer T313 timeout

26.8.1.3.5.2.1	Definition and applicability

The call control entity of the MS being in the state, U8, if no response is then received from the SS, timer T313 expires at the MS side. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.2.2	Conformance requirement

A CC entity of a MS in CC-state U8, "Connect Request", having waited for a reasonable length of time (eg. expiry of timer T313) without receiving the appropriate protocol message to complete the incoming call, shall initiate the clearing of that incoming call by sending the CC message DISCONNECT and enter the CC-state U11, "Disconnect Request".

If an MS disconnects too early then, in the case of very late assignment of a traffic channel, systematic waste of radio resources may occur.

References

TS GSM 04.08 section 5.2.2.6., TS GSM 04.08 section 5.4.3.

26.8.1.3.5.2.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", having waited for a reasonable length of time (eg. expiry of timer T313) without receiving the appropriate protocol message to complete the incoming call, shall initiate the clearing of that incoming call by sending the CC message DISCONNECT and enter the CC-state U11, "Disconnect Request"

26.8.1.3.5.2.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/2.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

45 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8 (if the MS uses immediate connection for the selected basic service then p = Y, otherwise p = N). The T313 expires at the MS and the MS sends a DISCONNECT message and enters state U11, disconnect request. The SS checks by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence

Step	Direction	Message	Comments

A1	SS -> MS	ASSIGNMENT COMMAND	p = Y
	A2	MS -> SS	ASSIGNMENT COMPLETE

3	MS -> SS	DISCONNECT	Shall not be sent before 15 seconds after entry
				into state U8. But, shall be sent before 1.1 * T313
				after entry into state U8.
	4	SS -> MS	STATUS ENQUIRY
	5	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.3.5.3	Incoming call / U8 connect request / termination requested by the user

26.8.1.3.5.3.1	Definition and applicability

The call control entity of the MS being in the state, U10, the user requests for releasing of the call. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.3.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.3., TS GSM 11.10, Annex 3 - section 2.2.

26.8.1.3.5.3.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.3.5.3.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/2.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8 (if the MS uses immediate connection for the selected basic service then p = Y, otherwise p = N). Then the user requests termination of the call. The MS sends a DISCONNECT message and enters state U11, disconnect request. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence

Step	Direction	Message	Comments

A1	SS -> MS	ASSIGNMENT COMMAND	p = Y
	A2	MS -> SS	ASSIGNMENT COMPLETE

3			the user requests to clear the call
	4	MS -> SS	DISCONNECT	
	5	SS -> MS	STATUS ENQUIRY	
	6	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.3.5.4	Incoming call / U8 connect request / DISCONNECT received with in-band information

26.8.1.3.5.4.1	Definition and applicability

The call control entity of the MS being in the state, U8, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.4.2	Conformance requirement

A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT with progress indicator #8 shall enter CC-state U12, if the traffic channel is in speech mode. If the TCH is not in speech mode, the MS shall send a RELEASE message and enter CC-state U19.

References

TS GSM 04.08 section 5.4.4., TS GSM 04.08 section 5.5.1.

26.8.1.3.5.4.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT with progress indicator #8 enters CC-state U12, if the traffic channel is in speech mode, and that the MS sends a RELEASE message and enters CC-state U19 if the TCH is not in speech mode.

26.8.1.3.5.4.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN. The MS is brought into the state U8 by using table 26.8.1.3/3.

Foreseen final state of the MS

U12, disconnect indication or U19 depending on the bearer capabilities.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. If channel mode is speech, the MS enters state U12, disconnect indication. If channel mode is not speech, the MS sends a RELEASE message and enters state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 39)

TCH in speech mode:
	A2	SS -> MS	STATUS ENQUIRY	
	A3	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:
	B2	MS -> SS	RELEASE	
	B3	SS -> MS	STATUS ENQUIRY	
	B4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 39:	With a progress indicator indicating in-band information; Progress Indicator, Progress description #8.

###### 26.8.1.3.5.5	Incoming call / U8 connect request / DISCONNECT received without in-band information

26.8.1.3.5.5.1	Definition and applicability

The call control entity of the MS being in the state, U8, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.5.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT without progress indicator, shall return a RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.4., TS GSM 04.08 section 5.4.4.2.

26.8.1.3.5.5.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT without progress indicator, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.3.5.5.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/3.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 40)
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 40:	Without a progress indicator indicating in-band information

###### 26.8.1.3.5.6	Incoming call / U8 connect request / RELEASE received

26.8.1.3.5.6.1	Definition and applicability

The call control entity of the MS being in the state, U8, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.6.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a RELEASE shall return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile terminating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.3.5.6.3	Test purpose

1)	To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a RELEASE will return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile terminating transaction identifiers are in CC-state U0, "Null".

26.8.1.3.5.6.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/3.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends a RELEASE message. The MS responds with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered state U0, null, with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.5.7	Incoming call / U8 connect request / lower layer failure

26.8.1.3.5.7.1	Definition and applicability

The call control entity of the MS being in the state, U8, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.7.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", having detected a lower layer failure shall return to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.3.5.7.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", having detected a lower layer failure returns to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

26.8.1.3.5.7.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The MS is brought to the state U8. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.5.8	Incoming call / U8 connect request / TCH assignment

26.8.1.3.5.8.1	Definition and applicability

The call control entity of the MS being in the state, U8, an assignment procedure is performed for traffic channel. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.8.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.2.7.

26.8.1.3.5.8.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.3.5.8.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/1.

Foreseen final state of the MS

U8, connect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS verifies by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH
	2	MS -> SS	ASSIGNMENT COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U8

Specific message contents:

None.

###### 26.8.1.3.5.9	Incoming call / U8 connect request / unknown message received

26.8.1.3.5.9.1	Definition and applicability

The call control entity of the MS being in the state, U8, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.9.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.3.5.9.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.3.5.9.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/1.

Foreseen final state of the MS

U8, connect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U8
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U8

Specific message contents:

None.

#### 26.8.1.4	In call fuctions

##### 26.8.1.4.1	In-call functions / DTMF information transfer

###### 26.8.1.4.1.1	In-call functions / DTMF information transfer / basic procedures

26.8.1.4.1.1.1	Definition and applicability

Dual Tone Multi Frequency (DTMF) is an inband one out of four plus one out of four signalling system primarily used from terminal instruments in telecommunication networks.

The support of DTMF is only permitted when a bearer capability for speech is in use or during the speech phase of alternate speech/data and alternate speech/facsimile teleservices.

26.8.1.4.1.1.2	Conformance requirement

1)	An MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone, shall send a START DTMF message on the correct DCCH.

References

TS GSM 04.08, 5.3.3

2)	An MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone (the corresponding IA5 character being selected from among the ones supported), shall send a START DTMF message specifying the correct IA5 character in the "keypad information" field of the keypad facility information element.

2.1	TS GSM 04.08, 5.3.3

26.8.1.4.1.1.3	Test purpose

1)	To verify that an MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone, sends a START DTMF message on the correct DCCH.

2)	To verify that an MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone (the corresponding IA5 character being selected from among the ones supported), sends a START DTMF message specifying the correct IA5 character in the "keypad information" field of the keypad facility information element.

26.8.1.4.1.1.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	supported character set (e.g. 0-9, #, *, A, B, C, D)

-	if and how DTMF tone is indicated to the user

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC-state "active".

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

1 min.

Test procedure

The MS being in the call active state, a user causes a DTMF tone to be generated e.g. by depression of a key in the MS. A DTMF digit corresponding to the digit indicated by the user is sent in a START DTMF message by the MS. The SS will return a START DTMF ACK message to the MS. This acknowledgement may be used in the MS to generate an indication as a feedback for a successful transmission. Then the user indicates that the DTMF sending should cease e.g. by releasing the key. The MS will send a STOP DTMF message to the network which is acknowledged with STOP DTMF ACK by the SS.

The sequence described above is repeated for each of the applicable characters 0-9, #, *, A, B, C, and D.

Then a case of rejecting a DTMF tone is tested and the state of the MS is verified.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	START DTMF	the user causes DTMF tone to be generated
		SS		the SS will verify that the transmitted
				information corresponds to the digit pressed
	2	SS -> MS	START DTMF ACK	possible indication of a DTMF tone depending
				the PICS/PIXIT statements
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10
	5	MS -> SS	STOP DTMF	
	6	SS -> MS	STOP DTMF ACK	the DTMF tone indication shall be stopped
	7			the steps 1-4 shall be repeated for each of the
				applicable characters 0-9, #, *, A, B, C, D.
	8	SS -> MS	STATUS ENQUIRY	
	9	MS -> SS	STATUS	cause 30#, state U10
	10	MS -> SS	START DTMF	
	11	SS -> MS	START DTMF REJECT	
	12	SS -> MS	STATUS ENQUIRY	
	13	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None

##### 26.8.1.4.2	In-call functions / user notification

User notification procedure allows the network to notify a MS of any call-related event during the "active" state of a call. It also may allow a MS to notify the remote user of any appropriate call-related event during the "active" state of a call by sending a NOTIFY message containing a notification indicator to the network. No state change occurs at any of the interface sides during this procedure.

###### 26.8.1.4.2.1	In-call functions / User notification / MS terminated

26.8.1.4.2.1.1	Definition and applicability

This is a case for testing user notification procedure terminated by the mobile station. The test is applicable for those equipments supporting at least one circuit switched basic service.

26.8.1.4.2.1.2	Conformance requirement

1)	A CC entity of a MS in CC-state U10, "active", upon receiving of a NOTIFY message shall remain in the active state.

References

TS GSM 04.08, 5.3.1.

26.8.1.4.2.1.3	Test purpose

To verify that a CC entity of a MS in CC-state U10, "active", upon receiving of a NOTIFY message remains in the active state.

26.8.1.4.2.1.4	Method of test

Related PICS/PIXIT statements

-	supported circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC-state "active".

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

10 s.

Test procedure

The MS being in the call active state, the SS will send a NOTIFY message to the MS. The state of the MS is checked after that.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	NOTIFY	
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

##### 26.8.1.4.3	In-call functions / channel changes

The two following test cases are for testing some elementary radio resource level procedures during an active state of a call to ensure call maintenance also during physical channel changes.

###### 26.8.1.4.3.1	In-call functions / channel changes / a successful channel change in active state/ Handover and Assignment Command

26.8.1.4.3.1.1	Definition and applicability

This is a case to test a change of a physical channel during active state of a call. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.4.3.1.2	Conformance requirement

1)	The MS being in the call active state after having successful completed a channel assignment or a handover command, shall remain in the call active state.

References

TS GSM 04.08, section 5.3.4.3.2, TS GSM 04.08, section 3.4.6.

26.8.1.4.3.1.3	Test purpose

To verify that the MS being in the call active state after having successful completed a channel assignment or having completed a handover command remains in the call active state.

26.8.1.4.3.1.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

- 	Type of MS ( P-GSM900 or EGSM or DCS1800 )

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC-state "active".

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

10 s.

Test procedure

The SS initiates a call to the Mobile Station, using an arbitrarily chosen MT circuit switched basic service ( see section 10 for generic call set up procedures )

The MS being in the call active state, th SS initiated channel assignment procedure causing an intracell change of channel by sending ASSIGNMENT COMMAND message to the MS. The MS performs channel assignment procedure and after the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message. The state of the MS is then checked.

The SS then initiates a Finely Synchronised handover intra cell procedure. On the successful completion of this procedure the state of the MS is checked.

Expected sequence

Step	Direction	Message	Comments

0			Generic call set up procedure defined in 
				sections 10.1 and 10.3, depending on choice
				of Bearer Capability.
	1	SS -> MS	ASSIGNMENT COMMAND	
	2	MS -> SS	ASSIGNMENT COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10
	5	SS -> MS	HANDOVER COMMAND	See Specific message contents.
	6	MS -> SS	HANDOVER ACCESS	Four HANDOVER ACCESS

7	MS -> SS	HANDOVER ACCESS
	8	MS -> SS	HANDOVER ACCESS
	9	MS -> SS	HANDOVER ACCESS
	10	MS -> SS	HANDOVER COMPLETE
	11	SS -> MS	STATUS ENQUIRY	
	12	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

ASSIGNMENT COMMAND

Information Element					value/remark

Channel Description

As used in Assignment Command when setting up the call, except :

- Timeslot Number					Arbitrary value, but different to

originally used.

HANDOVER COMMAND

Information Element					value/remark

Cell Description

- Network Colour Code				1

- Base Station Colour Code				5

- BCCH Carrier Number				P-GSM900 - ARFCN  30

DCS1800  - ARFCN 650

Channel Description

As used in Assignment Command when setting up the call, except :

- Timeslot Number					Arbitrary value, but different to

originally used.

Synchronisation Indication

- Report Observed Time Difference			Shall not be included.

- Synchronisation Indication				'Synchronised'.

- Normal Cell Indication				Ignore out of range timing advance.

STATUS

Information Element					value/remark

cause						#30, statue U10.

###### 26.8.1.4.3.2	In-call functions / channel changes / an unsuccessful channel change in active mode/ Handover and Assignment Command

26.8.1.4.3.2.1	Definition and applicability

This is a case to test an unsuccessful change of a physical channel during active state of a call. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.4.3.2.2	Conformance requirement

1)	The MS, when returning to the old channel after handover or Assignment failure and having established the link, shall remain in the call active state.

References

TS GSM 04.08, section 5.3.4.3.

26.8.1.4.3.2.3	Test purpose

To verify that the MS, when returning to the old channel after handover or Assignment failure and correctly establishing the link, will remain in the call active state.

26.8.1.4.3.2.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	Type of Mobile Station ( P-GSM900 or EGSM or DCS1800 )

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC-state "active".

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

30 s.

Test procedure

The SS initiates a call to the Mobile Station, using an arbitrarily chosen circuit switched basic service ( see section 10 for generic call set up procedures )

The MS being in the call active state, the SS initiates non synchronised handover procedure to cell B. The MS begins to send access bursts on the new DCCH. The SS activates the SACCH, but does not send a PHYSICAL INFORMATION MESSAGE, thus causing timer T3124 to timeout. Then the MS shall return back to the old channel and re-establish the signalling link on cell A and send a HANDOVER FAILURE message. The state of the MS is then checked.

The SS sends an Assignment command message allocating a hopping TCH/F, but does not activate the assigned channel. The MS shall attempt try to activate the new channel ( this is not verified ) and shall then reactivate the 'old' channel and trigger the establishment of the main signalling link on the old channel. The MS shall send an ASSIGNMENT FAILURE message. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

0			Generic call set up procedure defined in 
				sections 10.1 and 10.3, depending on choice 
				of Bearer Capability.
	1	SS -> MS	HANDOVER COMMAND	
	2	MS -> SS	HANDOVER ACCESS	Several messages
				are sent, all with the handover reference sent
				in the HANDOVER COMMAND message.
	3	MS -> SS	HANDOVER FAILURE	
	4	SS -> MS	STATUS ENQUIRY	
	5	MS -> SS	STATUS	cause 30#, state U10
	6 	SS -> MS	ASSIGNMENT COMMAND	Channel type = TCH/F, hopping. The MS
					attempts and fails to establish a signalling link
				on the new channel.
	7			The MS re-establishes the signalling link on the
				'old' channel.

8	MS -> SS	ASSIGNMENT FAILURE	RR cause value = "protocol error unspecified"

9	SS -> MS	STATUS ENQUIRY
	10	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

ASSIGNMENT FAILURE

Information Element				value/remark

RR cause					"protocol error unspecified"

HANDOVER FAILURE

Information Element				value/remark

RR cause					Not checked, as tested elsewhere.

STATUS

Information Element				value/remark

cause					#30, statue U10.

##### 26.8.1.4.4	In-call functions / MS terminated in-call modification

###### 26.8.1.4.4.1	In-call functions / MS terminated in-call modification / modify when new mode is not supported

26.8.1.4.4.1.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the new mode is not supported (and consequently not one of those negotiated and agreed during the establishment phase of the call). This test is applicable for any equipment supporting at least one circuit switched basic service.

26.8.1.4.4.1.2	Conformance requirement

1)	In the case that the MS supports the network originated in-call modification procedure, the MS after having received a MODIFY message with a new mode which is not the actual one and cannot be supported by the MS shall reject it by sending a MODIFY REJECT message or a STATUS message.

2)	In the case that the MS does not support the network originated in-call modification procedure, the MS shall, when receiving a MODIFY message, treat the message as unknown and respond with a STATUS message.

References

1)	TS GSM 04.08, sections 5.3.4.3.4.2 and 5.3.4.4.

2)	TS GSM 04.08, section 5.3.4.

26.8.1.4.4.1.3	Test purpose

1)	To verify that an MS supporting the network originated in-call modification procedure, after having received a MODIFY message with a new mode which is not the actual one and cannot be supported by the MS, rejects it by sending a MODIFY REJECT.

2)	To verify that an MS not supporting the newtork originated in-call modification procedure, after having received a MODIFY message, responds with a STATUS message.

26.8.1.4.4.1.4	Method of test

Related PICS/PIXIT statements

-	supported circuit switched basic services

-	the MS supports the network originated in-call modification procedure (p = Yes/No).

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC-state "active".

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

10 s.

Test procedure

The MS being in the call active state, the SS initiatiates in-call modification procedure by sending a MODIFY message with new mode different from actual mode and one of those not supported by the MS. The MS either returns a MODIFY REJECT message with the old bearer capability or a STATUS message with reject cause #97, depending on the PICS/PIXIT statement. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	MODIFY	with new mode different from actual one

2a	MS -> SS	MODIFY REJECT	with the old call mode included OR, p = Yes

2b	MS -> SS	STATUS	cause #97, state U10, p = No

3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

##### 26.8.1.4.5	In-call functions / MS originated in-call modification

###### 26.8.1.4.5.1	In-call functions / MS originated in-call modification / a successful case of modifying

26.8.1.4.5.1.1	Definition and applicability

This test is to test a successful case of in-call modification, which is triggered by the calling tone identification (CNG) received by the MS. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.1.2	Conformance requirement

1)	The procedure shall be initiated by the MS in the "active" state of the call. It shall send a MODIFY message including the new mode to be changed to; and enter the "mobile originating modify" state. The new mode given in the MODIFY message shall be one of those already negotiated and agreed during the establishment phase of the call. The MS shall stop sending Bm-channel information according to the old mode and enter the state U26 "Mobile Originating Modify".

2)	Upon receipt of the MODIFY COMPLETE message the MS shall start sending channel information according to the new call mode and enter the "active" state.

References

Conformance requirement 1:	TS GSM 04.08, section 5.3.4.3.1.

Conformance requirement 2:	TS GSM 04.08, section 5.3.4.3.2.

26.8.1.4.5.1.3	Test purpose

1)	To verify that the procedure is initiated by the MS in the "active" state of the call. It sends a MODIFY message including the new mode to be changed to; and enters the "mobile originating modify" state. The new mode given in the MODIFY message is one of those already negotiated and agreed during the establishment phase of the call. The MODIFY originating side stops sending Bm-channel information.

2)	To verify that upon receipt of the MODIFY COMPLETE message the MS starts sending channel information according to the new call mode and enters the "active" state.

26.8.1.4.5.1.4	Method of test

Related PICS/PIXIT statements

-	a way to activate a dual mode call

-	a way to activate in-call modification

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with the new mode to the SS and the state of the MS is checked. The channel mode is modified with the CMM message including the appropriate channel mode for the new service. The SS then returns a MODIFY COMPLETE message. The state of the MS is then checked.

NOTE 41:	ICM can be initiated by manual intervention at the MS.

Expected sequence

Step	Direction	Message	Comments

1	MS		The MS is made to iniate a dual mode call
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	SDCCH
	4	MS -> SS	CM-SERVICE REQUEST	
	5	SS -> MS	CIPH MODE COMMAND	
	6	MS -> SS	CIPH MODE COMPLETE	
	7	MS -> SS	SETUP	as specified in specific message contents
	8	SS -> MS	AUTH REQUEST	
	9	MS -> SS	AUTH RESPONSE	
	10	SS -> MS	CALL PROCEEDING	as specified in specific message contents
	11	SS -> MS	ASSIGNMENT COMMAND	channel mode: see section 10.4
	12	MS -> SS	ASSIGNMENT COMPLETE	
	13	SS -> MS	ALERTING	
	14	SS -> MS	CONNECT	
	15	MS -> SS	CONNECT ACK	
	16	MS -> SS	MODIFY	as specified in specific message contents
	17	SS -> MS	STATUS ENQUIRY	
	18	MS -> SS	STATUS	cause 30#, state U26
	19	SS -> MS	CMM	as specified in specific message contents
	20	MS -> SS	CMM ACKNOWLEDGE
	21	SS -> MS	MODIFY COMPLETE	contains the new mode as bearer capability
	22	SS		allow at least 2 seconds for the MS to adapt
				for the new mode
	23	SS -> MS	STATUS ENQUIRY	
	24	MS -> SS	STATUS	cause 30#, state U10
	25	SS		verify that the MS stops sending Bm channel
				information according to the old mode

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.2	In-call functions / MS originated in-call modification / modify rejected

26.8.1.4.5.2.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the in-call modification is rejected. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.2.2	Conformance requirement

1)	Upon receipt of the MODIFY REJECT message with the old bearer capability the MS shall: resume sending Bm-channel information according to the present call mode; resume interpreting received Bm-channel information according to the present call mode; and enter the "active" state.

References

TS GSM 04.08, section 5.3.4.3.4.1.

26.8.1.4.5.2.3	Test purpose

To verify that upon receipt of the MODIFY REJECT message with the old bearer capability the MS resumes sending Bm-channel information according to the present call mode; resumes interpreting received Bm-channel information according to the present call mode; and enters the "active" state.

26.8.1.4.5.2.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS returns a MODIFY REJECT message. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode
				call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> SS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	MODIFY REJECT	with cause #58 bearer capability not
				available and with old bearer capabilities
	17	SS -> MS	STATUS ENQUIRY	
	18	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.3	In-call functions / MS originated in-call modification / an abnormal case of acceptance

26.8.1.4.5.3.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the in-call modification is accepted incorrectly. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.3.2	Conformance requirement

1)	Upon receipt of the MODIFY COMPLETE message indicating a call mode which does not correspond to the requested one the MS shall discard it and take no action.

References

TS GSM 04.08, section 5.3.4.4.

26.8.1.4.5.3.3	Test purpose

To verify that upon receipt of the MODIFY COMPLETE message indicating a call mode which does not correspond to the requested one the MS discards it and takes no action.

26.8.1.4.5.3.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U26 "Mobile Originating Modify".

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS returns a MODIFY COMPLETE message specifying a mode that does not correspond to the requested one. It will be verified then that the MS shall not take any action and the state of the MS will be checked.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode
				call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> SS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	MODIFY COMPLETE	with a mode that does not correspond
				to the requested one
	17	SS -> MS	STATUS ENQUIRY	
	18	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.4	In-call functions / MS originated in-call modification / an abnormal case of rejection

26.8.1.4.5.4.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the in-call modification is rejected incorrectly. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.4.2	Conformance requirement

1)	Upon receipt of the MODIFY REJECT message indicating a call mode which does not correspond to the actual one the MS shall discard it and take no action.

References

TS GSM 04.08, section 5.3.4.4.

26.8.1.4.5.4.3	Test purpose

To verify that upon receipt of the MODIFY REJECT message indicating a call mode which does not correspond to the actual one the MS discards it and takes no action.

26.8.1.4.5.4.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U26 "Mobile Originating Modify".

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS returns a MODIFY REJECT message specifying a mode that does not correspond to the actual one. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode
				call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> SS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	MODIFY REJECT	with a mode that does not correspond
				to the actual one
	17	SS -> MS	STATUS ENQUIRY	
	18	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.5	In-call functions / MS originated in-call modification / timeout of timer T323

26.8.1.4.5.5.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which timer T323 expires in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.5.2	Conformance requirement

1)	Upon expiration of T323 the MS shall initiate the procedures for call clearing with cause #102 "recovery on timer expiry".

References

TS GSM 04.08, section 5.3.4.3.4.3.

26.8.1.4.5.5.3	Test purpose

To verify that upon expiration of T323 (accuracy +/- 10%) the MS shall initiate the procedures for call clearing with cause #102 "recovery on timer expiry".

26.8.1.4.5.5.4	Method of test

Related PICS/PIXIT statements

-	supported circuit switched basic services

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U11 "disconnect request".

Maximum duration of test

1 minute.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS does not respond until timer T323 expires at the MS. The MS is expected to respond with a DISCONNECT message. The SS checks timer T323 accuracy between emission of MODIFY and reception of DISCONNECT messages, the state of the MS and a cause value from the DISCONNECT message.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode
				call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> MS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS		the SS waits for the timer
				T323 expiry
	17	MS -> SS	DISCONNECT	cause value #102, the SS checks timer T323
				accuracy (+/- 10 %) between  MODIFY and
				 DISCONNECT messages
	18	SS -> MS	STATUS ENQUIRY	
	19	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.6	In-call functions / MS originated in-call modification / a successful channel change in state mobile originating modify

26.8.1.4.5.6.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which a change of a physical channel occurs in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.6.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U26, "Mobile Originating Modify", after successful completion of a channel assignment procedure or channel mode modify procedure shall remain in the call state U26.

2)	Upon receipt of the MODIFY COMPLETE message the MS shall start sending channel information according to the new call mode and enter the "active" state.

References

1)	TS GSM 04.08, section 5.3.4.3.2, TS GSM 04.08, section 3.4.6.

2)	TS GSM 04.08, section 5.3.4.3.2.

26.8.1.4.5.6.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U26, "Mobile Originating Modify", after successful completion of a channel assignment procedure procedure remains in the call state U26.

2)	To verify that upon receipt of the MODIFY COMPLETE message the MS starts sending channel information according to the new call mode and enters the "active" state.

26.8.1.4.5.6.4	Method of test

Related PICS/PIXIT statements

-	supported circuit switched basic services

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U10, active

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with a new mode to the SS. The SS does not respond immediately, but performs channel assignment procedure including the appropriate channel mode for the new service. The state of the MS is then checked. The SS then returns a MODIFY COMPLETE message. The state of the MS is checked finally.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode
				call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> MS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	ASSIGNMENT COMMAND	channel mode implied by the MODIFY
				message
	17	MS -> SS	ASSIGNMENT COMPLETE	
	18	SS -> MS	STATUS ENQUIRY	
	19	MS -> SS	STATUS	cause 30#, state U26
	20	SS -> MS	MODIFY COMPLETE
	21	SS -> MS	STATUS ENQUIRY
	22	MS -> SS	STATUS	cause	30#, state U10

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.7	In-call functions / MS originated in-call modification / an unsuccessful channel change in state mobile originating modify

26.8.1.4.5.7.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which an unsuccessful change of a physical channel occurs in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U26, "Mobile Originating Modify", when returning to the old channel after handover failure and having established the link, shall remain in the call state U26.

References

TS GSM 04.08, section 5.3.4.3.2.

26.8.1.4.5.7.3	Test purpose

To verify that a CC-entity of the MS in CC-state U26, "Mobile Originating Modify", when returning to the old channel after handover failure and having established the link, remains in the call state U26.

26.8.1.4.5.7.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U26, mobile originating modify.

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with a new mode to the SS. The SS initiates handover procedure. When the MS tries to establish the main signalling link, it is prohibited by the SS. Then the MS shall return back to the old channel and re-establish correctly the link. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode
				call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> SS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	HANDOVER COMMAND	
	17	MS -> SS	HANDOVER ACCESS	the SS does not respond
	18	MS -> SS	HANDOVER FAILURE	after the MS has re-established the main
				signalling link in the old channel
	19	SS -> MS	STATUS ENQUIRY	
	20	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.8	In-call functions / MS originated in-call modification / unknown message received

26.8.1.4.5.8.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which an unknown message is received in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.8.2	Conformance requirement

A CC entity of a MS in CC-state U26, "Mobile Originating Modify", having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08, section 8.4

26.8.1.4.5.8.3	Test purpose

To verify that a CC entity of a MS in CC-state U26, "Mobile Originating Modify", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.4.5.8.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U26, mobile originating modify.

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with a new mode to the SS. The SS sends a message with message type not defined for the protocol discriminator. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode
				call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> SS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	unknown message	message type not defined for PD
	17	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.9	In-call functions / MS originated in-call modification / a release complete received

###### 26.8.1.4.5.9.1	Definition and applicability

The call control entity of the MS being in the state, U26, the call is cleared by a RELEASE COMPLETE message sent by the SS. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech / Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech / Group 3 fax).

###### 26.8.1.4.5.9.2	Conformance requirement

1)	A CC entity of the MS in CC-state U26, "mobile originating modify", upon receipt of a RELEASE COMPLETE message with valid cause value, shall enter CC state U0, "Null".

2)	On returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

Reference(s)

Conformance requirement 1: GSM 04.08 section 5.4.2, GSM 04.08 section 5.4.4

Conformance requirement 2: GSM 04.08 section 5.4.4.3

###### 26.8.1.4.5.9.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U26, "mobile originating modify", upon receipt of a RELEASE COMPLETE message with valid cause value, enters CC state U0, "Null".

2)	To verify that on returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

###### 26.8.1.4.5.9.4	Method of test

Related PICS/PIXIT statements

-	a way to activate a dual mode call

-	a way to activate in-call modification

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Maximum duration of test

30 seconds.

Test Procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with the new mode to the SS and the state of the MS is checked. The SS sends a RELEASE COMPLETE message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

NOTE:	ICM can be initiated by manual intervention at the MS.

Expected sequence

Step	Direction	Message	Comments

1	MS		The MS is made to initiate a dual mode call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	SDCCH
	4	MS -> SS	CM-SERVICE REQUEST
	5	SS -> MS	CIPH MODE COMMAND
	6	MS -> SS	CIPH MODE COMPLETE
	7	MS -> SS	SETUP	as specified in specific message contents
	8	SS -> MS	AUTH REQUEST
	9	MS -> SS	AUTH RESPONSE
	10	SS -> MS	CALL PROCEEDING	as specified in specific message contents
	11	SS -> MS	ASSIGNMENT COMMAND	channel mode: see section 10.4
	12	MS -> SS	ASSIGNMENT COMPLETE
	13	SS -> MS	ALERTING
	14	SS -> MS	CONNECT
	15	MS -> SS	CONNECT ACK
	16	MS -> SS	MODIFY	as specified in specific message contents
	17	SS -> MS	STATUS ENQUIRY
	18	MS -> SS	STATUS	cause #30, state U26
	19	SS -> MS	RELEASE COMPLETE
	20	SS -> MS	STATUS ENQUIRY
	21	MS -> SS	RELEASE COMPLETE	cause #81 (invalid TI value)
	22	SS		repeat steps 20 - 21 to cover all the
				transaction identifiers from 000 ... 110
	23	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA)

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.10	In-call functions/MS originated in-call modification/contents of some of the messages

The following messages are used for testing in-call modification procedures, test cases 26.8.1.4.5.*, as default messages for those ones defined below. If any other values are defined in the expected sequence of the actual test cases, those values take precedence over the ones defined hereafter.

SETUP (MS to SS)

Information element			Value/remark

BC Repeat indicator

Repeat indication			Sequential, if BS81 is being tested, otherwise

circular for successive selection

Bearer capability 1				Appropriate for the teleservice/Bearer Service

selected as an initial call mode

Bearer capability 2				Appropriate for the teleservice/Bearer Service

to be selected as a new call mode

Facility					Omitted

Calling party subaddress			Omitted

Called party BCD number			As entered

Called party subaddress			Omitted

LLC repeat indicator			The same repeat indication as the one for BC.

Present if and only if LLC I and LLC II are present

Low layer compatibility I			See note 42

Low layer compatibility II			See note 42

HLC repeat indicator			The same repeat indication as the one for BC.

Present if and only if HLC i and HLC ii are present.

High layer compatibility i			See note 42

High layer compatibility ii			See note 42

User-user				Omitted

SS version				Omitted

CLIR suppression				Omitted

NOTE 42:	HLC/LLC may or may not be present. The contents of HLC/LLC are not verified. If LLC I is present then LLC II shall be present. If HLC i is present then HLC ii shall be present.

CALL PROCEEDING

If the MS offers a choice in a SETUP message with respect to its bearer capabilities (this choice is restricted to the connection element), the bearer capabilities 1 and 2 and BC repeat indicator must all be present in this message. Otherwise, all three IEs are omitted.

Information element			Value/remark

Repeat Indicator				See above

Repeat indication			As received in the SETUP message

Bearer Capability 1			Same as in section 10.4

Bearer Capability 2			Same as in section 10.4

Facility					Omitted

Progress indicator				Omitted

MODIFY

Information element			Value/remark

Bearer capability				If the bearer capability IEs were present in the

CALL PROCEEDING message, then as it was

specified in the bearer capability 2 of the

CALL PROCEEDING message. Otherwise as

in the bearer capability 2 of the

SETUP message.

Reverse Call Setup Direction		Presence and value not checked

Low layer compatibility			See note 43

High layer compatibility			See note 43

NOTE 43:	HLC (LLC) shall be included if the HLC (LLC) was included in the SETUP message. The contents of LLC/HLC are not verified.

MODIFY COMPLETE

Information element			Value/remark

Bearer capability				If the bearer capability IEs were present in the

CALL PROCEEDING message, then as it was

specified in the bearer capability 2 of the

CALL PROCEEDING message. Otherwise as

in the bearer capability 2 of the

SETUP message.

Reverse Call Setup Direction		Same as in MODIFY

Low layer compatibility			See note 44

High layer compatibility			See note 44

NOTE 44:	HLC (LLC) shall be included if the HLC (LLC) was included in the SETUP message. The contents of LLC/HLC are not verified.

MODIFY REJECT

Information element			Value/remark

Bearer capability				If the bearer capability IEs were present in the

CALL PROCEEDING message, then as it was

specified in the bearer capability 1 of the

CALL PROCEEDING message. Otherwise as

in the bearer capability 1 of the

SETUP message.

Cause					#58 "bearer capability not presently available".

Low layer compatibility			See note 44a

High layer compatibility			See note 44a

NOTE 44a:	HLC (LLC) shall be included if the HLC (LLC) was included in the SETUP message. The contents of LLC/HLC are not verified.

CHANNEL MODE MODIFY

Information element			Value/remark

Channel description			describes non-hopping Bm+ACCHs or

Lm+ACCHs as appropriate for the test

Channel Mode				appropriate for the BC in the MODIFY

CHANNEL MODE MODIFY ACKNOWLEDGE

Information element			Value/remark

Channel description			as sent by the SS in the corresponding

CHANNEL MODE MODIFY message

Channel mode				as sent by the SS in the corresponding

CHANNEL MODE MODIFY message

### 26.8.2	Call Re-establishment

#### 26.8.2.1	Call Re-establishment/call present, re-establishment allowed

##### 26.8.2.1.1	Definition and applicability

This is to test a successful case of a call re-establishment procedure. This test is applicable for any equipment supporting at least one bearer capability. If the MS does not perform call re-establishment procedure correctly, the network will waste resources.

##### 26.8.2.1.2	Conformance requirement

1)	If the call is in the "active" state or "mobile originating modify" state, the indication from MM that re-establishment is possible shall cause call control to request re-establishment from the MM-connection, suspend any further message to be sent and await the completion of the re-establishment procedure.

2)	When the call control entity is notified that the MM-connection is reestablished, it shall then resume the transmission of possibly suspended messages and resume user data exchange when an appropriate channel is available.

References

1)	TS GSM 04.08, subclauses 4.5.1.6 and 5.5.4.2.

2)	TS GSM 04.08, subclauses 4.5.1.6 and 5.5.4.3.

##### 26.8.2.1.3	Test purpose

The purpose of this test is to verify that the MS can correctly perform a call re-establishment procedure.

##### 26.8.2.1.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

Initial conditions

System Simulator:

The SS simulates cells A and B. The LAC of cell A is different from the LAC of cell B. The PLMN identities of cell A and B are equal.

Cell B is not barred, the RACH control parameters information element sent in SYSTEM INFORMATION TYPE 1 to 4 messages of cell A and B specifies 'call reestablishment allowed in the cell', the NCC of cell B is indicated as permitted in the PLMN permitted information element of SYSTEM INFORMATION TYPE 2 and 6 messages of cell A. Cell B is indicated as a neighbour cell of cell A in SYSTEM INFORMATION TYPE 2 and 5 messages of cell A. Cell reselect hysteresis parameter of cell A is set to zero.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN on cell A.

Foreseen final state of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Maximum duration of test

1 minute.

Test procedure

The MS is brought to active state by using procedure 26.9.2, "structured procedures, MS originated call, early assignment". The RF level of cell A is lowered so that cell B is to be selected (when the MS performs re-establishment after radiolink failure), while keeping the C1 and C2 of cell A greater than zero. SS waits for at least 5 seconds. Then the SS stops transmission on the TCH/SACCH. The MS shall re-establish the call on cell B using a CM-REESTABLISHMENT message. The SS performs ciphering mode setting and assignment procedures. The MS shall through-connect the appropriate bearer channel. Then, the call is cleared by the SS.

Expected sequence

Step	Direction	Message	Comments

1			Steps 1-19 of test case 26.9.2 are performed
				(the appropriate bearer channel is through
				connected in both directions in TCH)
	2	SS		The RF level of cell A is lowered. The SS
				waits at least 5 seconds. The SS stops
				transmission on the TCH/SACCH.
	3	MS -> SS	CHANNEL REQUEST	this is sent on cell B. Establ. cause shall be
				"call re-establishment; TCH/F was in use,..."
	4	SS -> MS	IMMEDIATE ASSIGNMENT	
	5	MS -> SS	CM-REESTAB REQUEST	note specific message contents
	6	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	7	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.
				All following messages
				shall be sent enciphered.
	8	SS		SS starts ciphering.
	9	SS -> MS	ASSIGNMENT COMMAND	
	10	MS -> SS	ASSIGNMENT COMPLETE	
	11	MS		The appropriate bearer channel is
				through connected in both directions.
	12	SS -> MS	DISCONNECT	with cause value "Normal"
	13	MS -> SS	RELEASE	
	14	SS -> MS	RELEASE COMPLETE	
	15	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific message contents:

CM-REESTABLISHMENT REQUEST

Information element			Value/remark

Protocol discriminator			Mobility Management

Skip indicator				Encoded as zeroes

Message type				CM-REESTABLISHMENT REQUEST

Ciphering key sequence number		The CKSN which the MS was allocated

in step 6 of the procedure of section 26.9.2.

Spare half octet				zero

Mobile station classmark 2			as declared in the PICS/PIXIT

Mobile identity				The TMSI that the MS is having initially

Location area identification			Corresponding the LAI of cell A

#### 26.8.2.2	Call Re-establishment/call present, re-establishment not allowed

##### 26.8.2.2.1	Definition and applicability

This is to test a special case of a call re-establishment, in which it is not allowed for a MS to attempt re-establishment of a call. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

##### 26.8.2.2.2	Conformance requirement

When a lower layer failure occurs while an MM-connection is active, if a cell allowing call re-establishment is not available, the MS shall release the MM-connection and shall not attempt call re-establishment.

References

TS GSM 04.08, subclauses 4.5.1.6 and 5.5.4.

##### 26.8.2.2.3	Test purpose

The purpose of this test is to verify that the MS does not attempt call re-establishment when it is not allowed to take place because of the unavailability of a cell allowing call re-establishment.

##### 26.8.2.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

The SS simulates cell A.

Cell A is not barred, the NCC of cell A is indicated as permitted in the PLMN permitted information element of SYSTEM INFORMATION TYPE 2 and 6 messages. The RE field of the RACH control parameters information element broadcast in messages SYSTEM INFORMATION TYPE 1, 2, 3 and 4 of cell A are set to "call reestablishment not allowed in the cell".

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Maximum duration of test

1 minute.

Test procedure

The MS is brought to active state by using procedure 26.9.2, "structured procedures, MS originated call, early assignment". The SS stops transmission on the TCH/SACCH. The MS shall not require re-establishment of the call.

Expected sequence

Step	Direction	Message	Comments

1			Steps 1-19 of test case 26.9.2 are performed
				(the appropriate bearer channel is through
				connected in both directions in TCH)
	  2	SS		the SS stops transmission on the TCH/SACCH
  	3	MS		the MS shall not attempt re-establishment
				on cell A. This is checked for 30
				seconds after the radiolink failure.

Specific message contents:

None.

#### 26.8.2.3	Call Re-establishment/call under establishment, transmission stopped

##### 26.8.2.3.1	Definition and applicability

This is to test a special case of a call re-establishment, in which it is not allowed for a MS to attempt re-establishment of a call, since the call hasn't been established yet. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

##### 26.8.2.3.2	Conformance requirement

When a lower layer failure occurs while an MM-connection is active, if the state of the call control entity is not "active", the MS shall release the MM-connection and shall not attempt call re-establishment.

References

TS GSM 04.08, subclauses 4.5.1.6 and 5.5.4.2.

##### 26.8.2.3.3	Test purpose

The purpose of this test is to verify that the MS does not attempt call re-establishment when it is not allowed to take place because of the call control state.

##### 26.8.2.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic serices

Initial conditions

System Simulator:

The SS simulates cell A.

Cell A is not barred, the RACH control parameters information element sent in SYSTEM INFORMATION TYPE 1 to 4 messages of cell A specifies 'call reestablishment allowed in the cell', the NCC of cell A is indicated as permitted in the PLMN permitted information element of SYSTEM INFORMATION TYPE 2 and 6 messages.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Maximum duration of test

1 minute.

Test procedure

The call control entity of the MS is brought to state U4, "call delivered" by using initial part of procedure 26.9.2, "structured procedures, MS originated call, early assignment". The SS stops transmission on the TCH/SACCH. The MS shall not require re-establishment of the call on cell A.

Expected sequence

Step	Direction	Message	Comments

1			the MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS -> MS	AUTHENTICATION REQ	
	6	MS -> SS	AUTHENTICATION RESP	
	7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	8	MS -> SS	CIPHER MODE COMPLETE	
				All following messages
				shall be sent enciphered.
	9	SS		SS starts ciphering.
	10	MS -> SS	SETUP	
	11	SS -> MS	CALL PROCEEDING	
	12	SS -> MS	ALERTING	
	13	SS		the SS stops transmission on the TCH/SACCH
	14	MS		the MS shall not attempt re-establishment 
				on cell A. This is checked for 30
				seconds after the radiolink failure.

Specific message contents:

None.

### 26.8.3	User to user signalling

#### 26.8.3.1		Definition and applicability

The 'user to user' information element is used to convey information between the mobile user and a remote ISDN user. This test is therefore applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

NOTE 45:	There is no test for an MS originating call including a 'user-user' information element since it is not a mandatory MS feature.

#### 26.8.3.2	Conformance requirement

The inclusion of the 'user-user' information element in downlink call control messages shall cause no adverse effects on the operation of the MS.

References

GSM 04.08 sections 5.2.2, 9.3.7, 9.3.23.1 and 10.5.4.25

#### 26.8.3.3	Purpose of the test

The purpose of this test is to verify that inclusion of the 'user-user' information element in either of the down link messages, SETUP or DISCONNECT causes no adverse effects on the operation of the MS.

#### 26.8.3.4	Method of test

Related PICS/PIXIT statement(s)

Supported MT circuit switched basic services.

Support of user-user information element, and details of suitable codings

Initial conditions.

System Simulator :

The SS simulates 1 cell, with default parameters.

Mobile Station :

The MS is in MM-state "idle updated", with a valid TMSI and CKSN.

Foreseen final state of the MS

The MS is in MM-state "idle updated", with a valid TMSI and CKSN.

Maximum duration of test

2 minutes

Test procedure

The SS attempts to set up a mobile terminated call, with one of the supported circuit switched basic services which has been arbitrarily chosen, using one of the generic call set up procedures,( either speech or data ) as specified in section 10. The default SETUP message contents are modified to include the user-user Information Element. The MS shall not respond adversely to the inclusion of the user-user information element.

After 30 seconds the SS sends a DISCONNECT message, again the MS shall not respond adversely to the inclusion of the user-user information element, but shall continue to clear down the call normally.

Expected sequence

Step	Direction	Message	Comments

1			Generic Call Setup procedure defined in 
				sections in 10.1 or 10.3, depending on choice 
				of Bearer Capability. The SETUP message in
				either case contains the user-user IE, see
				Specific message contents.

	2			The SS waits 30 seconds.

	3	SS -> MS	DISCONNECT	Message contains the user-user IE, see
				Specific message contents
	4	MS -> SS	RELEASE	As defined in section 26.8.4
	5	SS_-> MS	RELEASE COMPLETE	As defined in section 26.8.4
	6	SS_-> MS	CHANNEL RELEASE	As defined in section 26.8.4

Specific message contents:

SETUP

Information Element			value/remark

As default message contents as defined in the Generic Call setup procedures section 10.1 or 10.3 except :

Bearer Capability				Bearer capability arbitrarily chosen from

those supported by the Mobile Station under test.

user-user

- length				Length of user-user contents ( note 46 )

- PD				IA5 characters ( note 46 )

- user-user 				The following string coded in IA5 characters :

"Call Setup" ( note 46 )

DISCONNECT

Information Element			value/remark

As default message contents as defined in section 26.8.4, except :

user-user

- length				Length of user-user contents ( note 46 )

- PD				IA5 characters ( note 46 )

- user-user 				The following string coded in IA5 characters :

"Call Disconnect" ( note 46 )

NOTE 46:	The codings above are for example only. For the case of an MS which supports 'user-user' signalling it may be necessary to add meaning to the data fields, see PICS/PIXIT statement(s).

### 26.8.4	Default contents of message

ALERTING (mobile station to network direction)

No default requirements defined for this message.

ALERTING (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

User-user				Omitted

ASSIGNMENT COMMAND

Information element			Value/remark

Description of the first channel		describes non-hopping Bm+ACCHs or

Lm+ACCHs as appropriate for the test

Power Command				As in section 6.3

Frequency list				Omitted

Cell channel description			Omitted

Mode of the first channel 			appropriate for the bearer capability chosen forthe test

Description of the second channel		Omitted

Mode of the second channel			Omitted

Mobile allocation				Omitted

Starting time				Omitted

Cipher mode setting			Omitted

ASSIGNMENT COMPLETE

Information element			Value/remark

RR cause				not checked

AUTHENTICATION REQUEST

Information element			Value/remark

Ciphering key sequence number		Arbitrary excluding 111B

Spare half octet				(spare bits)

Authentication parameter RAND		Arbitrary

AUTHENTICATION RESPONSE

Information element			Value/remark

Authentication parameter SRES		not checked

CALL CONFIRMED

No default requirements defined for this message.

CALL PROCEEDING

Information element			Value/remark

Repeat Indicator				Omitted

Bearer Capability 1			Omitted if the SETUP message did not specify

in the bearer capability 1 IE a connection

element value "both, transparent preferred" or

"both, non-transparent preferred". Otherwise

included; in that case the connection element

specifies the value that is appropriate for the

selected basic service (either value

"transparent" or value "non transparent (RLP)"),

all other parameters are same as in the bearer

capability 1 IE of the received

SETUP message.

Bearer Capability 2				Omitted

Facility					Omitted

Progress indicator				Omitted

CHANNEL MODE MODIFY

Information element			Value/remark

Channel description			describes non-hopping Bm+ACCHs or

Lm+ACCHs as appropriate for the test

Channel mode				appropriate for the bearer capability chosen for

the test

CHANNEL MODE MODIFY ACKNOWLEDGE

Information element			Value/remark

Channel description			as sent by the SS in the corresponding

CHANNEL MODE MODIFY message

Channel mode				as sent by the SS in the corresponding

CHANNEL MODE MODIFY message

CHANNEL RELEASE

Information element			Value/remark

RR cause  				Normal event

CHANNEL REQUEST

Information element			Value/remark

Establishment cause 			If in response to paging,

then '100'; if a mobile originating call,

then '111'

Random reference				Arbitrary value of 5 bits length

CIPHERING MODE COMMAND

Information element			Value/remark

Cipher mode setting

algorithm identifier			indicates a supported algorithm

SC					Start ciphering

Cipher response

CR					IMEI must not be included

CIPHERING MODE COMPLETE

No default requirements defined for this message.

CM-SERVICE ACCEPT

No default values defined for this message.

CM-SERVICE REJECT

Information element			Value/remark

Reject cause				Service or option not available, unspecified

CM-SERVICE REQUEST

No default requirements defined for this message.

CONNECT (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

Connected number				Omitted

Connected subaddress			Omitted

User-user				Omitted

CONNECT (mobile station to network direction)

No default requirements defined for this message.

CONNECT ACKNOWLEDGE

No default requirements defined for this message.

DISCONNECT (network to mobile station direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

Progress indicator				Omitted

User-user 				Omitted

DISCONNECT (mobile station to network direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

User-user 				Omitted

SS version				Omitted

HANDOVER ACCESS

No default requirements defined for this message.

HANDOVER COMMAND

Information element			Value/remark

Cell Description				a BCCH frequency, which is one of the

neighbour cells

Description of the first channel		describes non-hopping Bm+ACCHs or

Lm+ACCHs as appropriate for the test

Handover Reference			an arbitrary value

Power Command				as in 6.3

Synchronisation indication			Omitted

Frequency short list			Omitted

Frequency List				Omitted

Cell Channel Description			Omitted

Channel Mode				Omitted

Channel Description			Omitted

Channel Mode 2				Omitted

Frequency Channel Sequence		Omitted

Mobile Allocation				Omitted

Starting Time				Omitted

Real time difference			Omitted

Timing advance				Omitted

Cipher Mode setting			Omitted

HANDOVER FAILURE

No default requirements defined for this message.

IMMEDIATE ASSIGNMENT

Information element			Value/remark

Page mode 				Normal paging

Channel description			describes a valid SDCCH+SACCH

in non-hopping mode

Request reference

Random access information		As received from MS

T1', T2, T3				Corresponding to frame number

of the CHANNEL REQUEST

Timing advance				corresponding the timing difference between

the MS and the SS

Mobile allocation				Empty (L=0)

Starting time				Omitted

MODIFY

No default values defined for this message.

MODIFY COMPLETE

No default requirements defined for this message.

MODIFY REJECT

No default values defined for this message.

NOTIFY (network to mobile station direction)

Information element			Value/remark

Notification indicator			one of the valid values chosen arbitrarily

PAGING REQUEST TYPE 1

Information element			Value/remark

L2 pseudo length				L2 pseudo length of the message

Page Mode				Normal Paging

Channels needed for Mobiles 1 and 2

channel (first)			any channel

channel (second)			any channel

Mobile identity 1				TMSI of MS under test

Mobile identity 2				Omitted

P1 rest octets				(spare octets)

PAGING RESPONSE

No default requirements defined for this message.

PROGRESS

No default values defined for this message.

RELEASE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Second cause				Omitted

Facility					Omitted

User-user				Omitted

RELEASE (mobile station to network direction)

No default requirements defined for this message.

RELEASE COMPLETE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Facility					Omitted

User-user				Omitted

RELEASE COMPLETE (mobile station to network direction)

No default requirements defined for this message.

SETUP (mobile station to network direction)

Information element			Value/remark

BC Repeat indicator			Omitted

Bearer capability 1			Appropriate for the basic service selected for

the test

Bearer capability 2			Omitted

Facility					Omitted

Calling party subaddress			Omitted

Called party BCD number			As entered

Called party subaddress			Omitted

LLC repeat indicator			Omitted

Low layer compatibility I			Appropriate for the basic service selected for

the test

Low layer compatibility II			Omitted

HLC repeat indicator			Omitted

High layer compatibility i			Appropriate for the basic service selected for

the test

High layer compatibility ii			Omitted

User-user				Omitted

SS version				Omitted

CLIR suppression				Omitted

SETUP (network to mobile station direction)

Information element			Value/remark

BC repeat indicator			Omitted

Bearer capability 1			Appropriate for the basic service selected for

the test

Bearer capability 2			Omitted

Facility					Omitted

Progress indicator				Omitted

Signal					Any defined value as described for Signal IE

in TS GSM 04.08

Calling party BCD number			Omitted

Calling party subaddress			Omitted

Called party BCD number			Omitted

Called party subaddress			Omitted

LLC repeat indicator			Omitted

Low layer compatibility I			Appropriate for the basic service selected for

the test

Low layer compatibility II			Omitted

HLC repeat indicator			Omitted

High layer compatibility i			Appropriate for the basic service selected for

the test

High layer compatibility ii			Omitted

User-user				Omitted

START DTMF

No default requirements defined for this message.

START DTMF ACKNOWLEDGE

Information element			Value/remark

Keypad facility				corresponding to the DTMF digit indicated in

the START DTMF message

START DTMF REJECT

Information element			Value/remark

Cause					value "Resources unavailable, unspecified"

STATUS

Information element			Value/remark

Cause					Value "Response to STATUS ENQUIRY"

Call state				Specified separately in each test case

Auxiliary states				Omitted

STATUS ENQUIRY

No default values defined for this message, except that when this message is used to check that "all the transaction identifiers from 000 to 110" are in the null state, the TI flag shall take the value '1' in mobile originating call tests and shall take the value '0' in mobile terminating call tests.

STOP DTMF

No default requirements defined for this message.

STOP DTMF ACKNOWLEDGE

No default values defined for this message.

Unknown Message

Protocol Discriminator			Call Control; Call Related SS

Transaction Identifier			same as in use in the test

Message Type				0000 0100

## 26.9	Structured procedures

Abbreviations for message names used in this section:

AUTHENTICATION REQ	=	AUTHENTICATION REQUEST

AUTHENTICATION RESP	=	AUTHENTICATION RESPONSE

CIPHER MODE COMMAND	=	CIPHERING MODE COMMAND

CIPHER MODE COMPLETE	=	CIPHERING MODE COMPLETE

### 26.9.1	Structured procedures / general

The purpose of these tests is to verify that the MS performs certain elementary procedures of the RR, MM, and CC protocol correctly within a structured procedure. The term "structured procedure" is defined in TS GSM 04.08, section 7, where also examples of structured procedures are given.

The reason for this test purposes is twofold:

-	The behaviour of the MS in an elementary procedure may depend on the preamble which precedes the elementary procedure.

-	Structured procedures tested in this section are used in other parts of this Technical Specification as preambles to establish the initial conditions for other tests; correct behaviour of an implementation under test in a preamble is essential for the validity of a test.

Mobile originating and terminating calls are tested in cases of both early and late assignment of the traffic channel; in one of the cases call release initiated by the network is tested, in another one, call release initiated by the MS. The tests in this section only cover the successful outcome of elementary procedures (i.e. they do not deal with abnormal cases).

The emergency call service is tested in the following cases:

-	emergency call initiated in the idle, updated state with authentication and ciphering, for all supported speech rates,

-	emergency call initiated in the idle, no IMSI state (hence without authentication and without ciphering), the network accepting the call, for one supported speech rates,

-	emergency call initiated in the idle, no IMSI state (hence without authentication and without ciphering), the network rejecting the call, for one supported speech rate.

These tests on emergency calls are only applicable to an MS supporting speech.

For an MS supporting speech the test procedures in 26.9.2, 26.9.3, 26.9.4 and 26.9.5 are performed for speech (teleservice 11, telephony), once for each supported rate (full rate/half rate).

For an MS not supporting speech but supporting at least one teleservice, for each of the test procedures in 26.9.2, 26.9.3, 26.9.4, and 26.9.5 and each supported rate (full rate/half rate) a teleservice supported by the MS (see PICS/PIXIT statement) is chosen, and the test is performed corresponding to that teleservice (note that this teleservice is never a dual service).

In cases where a mobile originated call for the tested teleservice can be initiated both

-	via the MMI and

-	via the R or S interface,

procedure 26.9.2 shall be performed when initiating the mobile originated call via the MMI and procedure 26.9.3 shall be performed when initiating the mobile originated call via an appropriate interface (R or S).

### 26.9.2	Structured procedures / MS originated call / early assignment

#### 26.9.2.1	Conformance requirements

1)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call, if it provides a human interface, shall display the dialled number.

2)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call for a selected teleservice that is supported by the MS, shall start to initiate the immediate assignment procedure by sending a CHANNEL REQUEST message with correct establishment cause.

3)	Subsequently after establishment of an MM connection, the MS shall send a SETUP message with correct parameters.

4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

5)	Subsequently when the network initiates call clearing by sending a DISCONNECT message, the MS shall proceed to release the call by sending a RELEASE message.

6)	On receipt of a CHANNEL RELEASE message, the MS shall disconnect the main signalling link.

References

Conformance requirement 1:	TS GSM 02.07.

Conformance requirement 2:	TS GSM 04.08 section 3.3.1.

Conformance requirement 3:	TS GSM 04.08 section 5.2.1.1.

Conformance requirement 4:	TS GSM 04.08 section 5.2.1.6.

Conformance requirement 5:	TS GSM 04.08 section 5.4.4.

Conformance requirement 6:	TS GSM 04.08, section 3.5.1.

#### 26.9.2.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, displays the dialled number in the way described in a PICS/PIXIT statement.

2)	To verify that the MS in MM state "idle, updated" and in RR idle mode, with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

3)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed the authentication and cipher mode setting procedures, the MS sends a SETUP message with correct parameters.

4)	To verify that subsequently, after receipt of a CALL PROCEEDING message and of an ASSIGNMENT COMMAND message allocating an appropriate TCH, after having completed the traffic channel early assignment procedure by replying with the ASSIGNMENT COMPLETE message, after receipt of an ALERTING message and a CONNECT message, the MS returns a CONNECT ACKNOWLEDGE message.

5)	To verify that subsequently the MS has attached the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

6)	To verify that subsequently upon the network initiating call clearing by sending a DISCONNECT message, the MS proceed to release the call with RELEASE.

7)	To verify that subsequently, on receipt of a RELEASE COMPLETE message followed by a CHANNEL RELEASE message, the MS disconnects the main signalling link.

These test purposes are tested for all rates supported by the MS (full rate/half rate).

#### 26.9.2.3	Method of test

Related PICS/PIXIT Statements

- 	Supported rates (full rate/half rate)

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	SS version

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" is entered
  	2	MS		If p1 = Y, the MS must display the
				called number in the way defined in
				PICS/PIXIT.
	  3	MS -> SS	CHANNEL REQUEST	Establishment cause is "originating
				call and the network does not
				set the NECI bit to 1".
	  4	SS -> MS	IMMEDIATE ASSIGNMENT	
	  5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.
	  6	SS -> MS	AUTHENTICATION REQ
	  7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.
				All following messages
				shall be sent enciphered.
	 10	SS		SS starts ciphering.
	 11	MS -> SS	SETUP
	 12	SS -> MS	CALL PROCEEDING
			
				
				
	 13	SS -> MS	ASSIGNMENT COMMAND
	 14	MS -> SS	ASSIGNMENT COMPLETE
	15	SS -> MS	ALERTING
	16	MS		Depending on the PICS, an alerting indication
				is given
	 17	SS -> MS	CONNECT
	 18	MS -> SS	CONNECT ACKNOWLEDGE
	 19	MS		The appropriate bearer channel is
				through connected in both directions.
	 20	SS -> MS	DISCONNECT
	 21	MS -> SS	RELEASE
	 22	SS -> MS	RELEASE COMPLETE
	 23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

### 26.9.3	Structured procedures / MS originated call / late assignment

#### 26.9.3.1	Conformance requirement

1)	An MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS , shall start to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

2)	Upon receipt of the ASSIGNMENT COMMAND message, the Mobile Station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the data links). After the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message, specifying cause 'normal event', to the network on the main DCCH.

3,4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

References

Conformance requirement 1:	TS GSM 04.08 section 3.3.1.

Conformance requirement 2:	TS GSM 04.08 section 3.4.3.1, 3.4.3.2

Conformance requirement 3:	TS GSM 04.08 section 5.2.1.6.

#### 26.9.3.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message.

2)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed authentication and cipher mode setting procedures, after having sent a SETUP message, after having received a CALL PROCEEDING message followed by an ALERTING message and an ASSIGNMENT COMMAND message allocating an appropriate TCH, the MS sends an ASSIGNMENT COMPLETE message.

3)	To verify that subsequently, after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message returns a CONNECT ACKNOWLEDGE message.

4)	To verify that after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message attaches the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

These test purposes are tested for all rates supported by the MS (full rate/half rate).

#### 26.9.3.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS has a MO call in state U10, "active".

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with late assignment.

Maximum Duration of Test

30 seconds

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" is entered
	  2	MS		
	  3	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"originating call and the
				network does not set the NECI bit to 1".
	  4	SS -> MS	IMMEDIATE ASSIGNMENT	
	  5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.
	  6	SS -> MS	AUTHENTICATION REQ
	  7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 10	SS		SS starts ciphering.
	 11	MS -> SS	SETUP
	 12	SS -> MS	CALL PROCEEDING
	 13	SS -> MS	ALERTING
	 14	MS		Depending on the PICS, an alerting
				indication is given.
	 15	SS -> MS	ASSIGNMENT COMMAND
	 16	MS -> SS	ASSIGNMENT COMPLETE
	 17	SS -> MS	CONNECT
	 18	MS -> SS	CONNECT ACKNOWLEDGE
	 19	MS		The appropriate bearer channel is
				through connected in both directions.

Specific Message Contents:

None

### 26.9.4	Structured procedures / MS terminated call / early assignment

#### 26.9.4.1	Conformance requirements

1)	The MS shall acknowledge the SETUP message with a CALL CONFIRMED message, if compatibility checking was successful, the MS is not busy, and the user does not refuse the call.

2,3)	The MS on acceptance of the call sends a CONNECT, otherwise user alerting is initiated (the MS sends an ALERTING message and generates an alerting indication). ASSIGNMENT COMMAND is answered by ASSIGNMENT COMPLETE.

4)	An MS indicates acceptance of a MT call by sending CONNECT.

5)

For speech calls:

The mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment shall be delayed until such a resource becomes available.

For data calls:

The mobile station shall attach the user connection when receiving the CONNECT ACKNOWLEDGE message from the network.

6)	The MS initiates call clearing of an active call by sending a DISCONNECT message.

7)	The MS in this phase of call release, upon receipt of a RELEASE message, shall return a RELEASE COMPLETE message.

8)	Subsequently the MS, upon receipt of a CHANNEL RELEASE message, shall disconnect the main signalling link.

References

Conformance requirement 1:		TS GSM 04.08, section 5.2.2.3.1.

Conformance requirements 2,3:	TS GSM 04.08, section 5.2.2.3.2, 3.4.3.1.

Conformance requirement 4:		TS GSM 04.08 section 5.2.2.5.

Conformance requirement 5:		TS GSM 04.08, section 5.2.2.6 and 5.2.2.9.

Conformance requirements 6,7,8:	TS GSM 04.08, section 5.4.

#### 26.9.4.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, after being paged by the network on the correct paging subchannel, after initiating the immediate assignment procedure by sending the CHANNEL REQUEST message, after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after having sent a PAGING RESPONSE message on the allocated SDCCH, after having performed successful authentication and cipher mode setting procedures, after receipt of a SETUP message not containing a signal information element, returns a CALL CONFIRMED message.

2)	To verify that subsequently, the SS sending an ASSIGNMENT COMMAND message, the MS successfully continues a mobile terminating call establishment with early assignment of traffic channel

a)	by replying to the ASSIGNMENT command with an ASSIGNMENT COMPLETE message, and

b)	by continuing the call establishment by either sending one or two CONNECT messages (with equal N(SD)) or sending an ALERTING message, steps a) and b) being performed in any permitted interleaving.

3)	To verify that if after sending a CALL PROCEEDING message, the MS sends an ALERTING message during MTC establishment with early assignment, it generates an alerting indication.

4)	To verify that if an ALERTING had been sent, subsequently, when the user accepts the call (possibly internal action as declared in PICS/PIXIT statement), the MS returns a CONNECT message.

5)	To verify that the MS:

-	if the call is a speech call: after sending the CONNECT message has through connected the TCH in both directions (this is checked by verifying that after transmission of the first L2 frame containing the (complete) CONNECT message, the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

-	if the call is a data call: after receipt of a subsequent CONNECT ACKNOWLEDGE message through connects the TCH in both directions (this is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT ACKNOWLEDGE message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

6)	To verify that subsequently, the MS can initiate call clearing by sending a DISCONNECT message.

7)	To verify that the MS in this phase of call release, upon receipt of a RELEASE message, returns a RELEASE COMPLETE message.

8)	To verify that subsequently the MS, upon receipt of a CHANNEL RELEASE message, disconnects the main signalling link.

These test purposes are tested for all rates supported by the MS (full rate/half rate).

#### 26.9.4.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Immediate connect supported (Y/N)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged and the resulting call is established. Having reached the active state, the MS is made to clear the call.

Maximum Duration of Test

1 minute

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging
				subchannel
	  2	MS -> SS	CHANNEL REQUEST	
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	
	  4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  5	SS -> MS	AUTHENTICATION REQ
	  6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	  9	SS		SS starts ciphering.
	 10	SS -> MS	SETUP	Message does not contain the 
				signal IE.
	 11	MS -> SS	CALL CONFIRMED	
	 12	SS -> MS	ASSIGNMENT COMMAND

A13	MS -> SS	ASSIGNMENT COMPLETE	During step 12 or 
				between step 12 and step 13
				the MS may send a CONNECT message
				on the old channel.
	A14	MS -> SS	CONNECT	If during step 12 or 
				between step 12 and step A13
				the MS has send a CONNECT message
				on the old channel, both CONNECT
				messages must have equal N(SD).

B13	MS -> SS	ASSIGNMENT COMPLETE	Sent on the new channel.
	B14	MS -> SS	ALERTING	
	B15	MS		An alerting indication as defined 
				in an PICS/PIXIT statement 
				is given by the MS.
	B16	MS		The MS is made to accept the call
				in the way described in a
				PICS/PIXIT statement.
	B17	MS -> SS	CONNECT

18	MS		If the call is a speech call,
				the TCH shall be through connected 
				in both directions.
 19	SS -> MS	CONNECT ACKNOWLEDGE
	 20	MS		If the call is a data call, 
				the TCH shall be through
				connected in both
				directions.
	21	MS		The MS is made to release the call.
	22	MS -> SS	DISCONNECT
	23	SS -> MS	RELEASE
	24	MS -> SS	RELEASE COMPLETE
	25	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

### 26.9.5	Structured procedures / MS terminated call / late assignment

#### 26.9.5.1	Conformance requirement

TP1,TP2:	The MS shall acknowledge the SETUP message with a CALL CONFIRMED message, if compatibility checking was successful, the MS is not busy, and the user does not refuse the call. The MS on acceptance of the call sends a CONNECT, otherwise user alerting is initiated.

TP3:	The MS indicates acceptance of a call by sending a CONNECT message.

TP4:	ASSIGNMENT COMMAND is answered by ASSIGNMENT COMPLETE.

TP5:

For speech calls:

The mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment shall be delayed until such a resource becomes available.

For data calls:

The mobile station shall attach the user connection when receiving the CONNECT ACKNOWLEDGE message from the network.

Requirement reference:

Conformance requirements 1, 2, 3:		TS GSM 04.08, sections 5.2.2.3.1, 5.2.2.3.2, 5.2.2.5.

Conformance requirement 4:		TS GSM 04.08, section 3.4.3.1.

Conformance requirement 5:		TS GSM 04.08, section 5.2.2.9.

#### 26.9.5.2	Test purpose

1)	To verify that the MS in "Idle, Updated" state with a TMSI assigned, after being paged by the network on the correct paging subchannel, after initiating the immediate assignment procedure by sending the CHANNEL REQUEST message, after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after having established the main signalling link, after having sent a PAGING RESPONSE message, after having performed successful authentication and cipher mode setting procedures, after receipt of a SETUP message containing a signal information element, returns a CALL CONFIRMED message followed by

-	an ALERTING message

-	or a CONNECT message.

2)	To verify that in the situation of test purpose 1, if the MS sends an ALERTING message, the MS generates an alerting indication in the way described in a PICS/PIXIT statement.

3)	To verify that subsequently the MS, if it had not yet sent a CONNECT message, upon acceptance of the call, sends a CONNECT message.

4)	To verify that subsequently after receipt of an ASSIGNMENT COMMAND, the MS sends an ASSIGNMENT COMPLETE message.

5)	To verify that subsequently the MS

-	if the call is a speech call: after sending the ASSIGNMENT COMPLETE message has through connected the TCH in both directions (this is checked by verifying that after transmission of the first L2 frame containing the (complete) ASSIGNMENT COMPLETE message, the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

-	if the call is a data call: after receipt of a subsequent CONNECT ACKNOWLEDGE message through connects the TCH in both directions (this is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT ACKNOWLEDGE message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

These test purposes are tested for all rates supported by the MS (full rate/half rate).

#### 26.9.5.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Immediate connect supported (Y/N)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

CC state U10-call active.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged and a MT call is established with late assignment (after CONNECT).

Maximum Duration of Test

40 seconds

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging
				subchannel
	  2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates
				"answer to paging".
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	
	  4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	  5	SS -> MS	AUTHENTICATION REQ
	  6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. 
				All following
				messages shall be sent enciphered.
	  9	SS		SS starts ciphering.
	 10	SS -> MS	SETUP	Message contains the signal IE.
	 11	MS -> SS	CALL CONFIRMED

A12	MS -> SS	CONNECT

B12	MS -> SS	ALERTING	
	B13	MS		An alerting indication as defined 
				in an PICS/PIXIT statement 
				is given by the MS.
	B14	MS		The MS is made to accept the call
				in the way described in a
				PICS/PIXIT statement.
	B15	MS -> SS	CONNECT

16	SS -> MS	ASSIGNMENT COMMAND	
	 17	MS -> SS	ASSIGNMENT COMPLETE	
	 18	MS		If the call is a speech call,
				the TCH shall be through connected
				in both directions.
 	19	SS -> MS	CONNECT ACKNOWLEDGE
	 20	MS		If the call is a data call, 
				the MS shall 
				through connect the TCH
				in both directions.

Specific Message Contents:

None

### 26.9.6	Structured procedures / emergency call

Emergency call establishment can be initiated by an MS whether location updating has been successful or not and whether a SIM is inserted into the MS or not; but only if the MS is equipped for speech.

If the procedures tested in this section are not correctly implemented in the MS, establishment, maintenance and clearing of connections might fail in the essential case of emergency calls.

The tests of this section are only applicable to an MS supporting speech.

#### 26.9.6.1	Structured procedures / emergency call / idle updated

##### 26.9.6.1.1	Structured procedures / emergency call / idle updated / preferred channel rate

26.9.6.1.1.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct CKSN and TMSI , with CM Service Type "emergency call establishment".

3)	Authentication and cipher mode setting shall be performed successfully.

4)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

5),6)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

7)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

8)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 3.4.7, TS GSM 04.08 section 4.3.2.

For conformance requirement 4:

TS GSM 04.08, section 5.2.1.1.

For conformance requirement 5 and 6:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 7:

TS GSM 04.08, sections 5.2.1.6 and 5.1.3.

For conformance requirement 8:

TS GSM 04.08, section 5.4.

26.9.6.1.1.2	Test purpose

1)	To verify that an MS supporting speech in the MM state "idle, updated", when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message specifying the correct CKSN and TMSI, with CM Service Type "emergency call establishment" .

3)	To verify that authentication and cipher mode setting are performed successfully.

4)	To verify that after cipher mode setting acceptance by the SS, the MS sends an EMERGENCY SETUP message.

5)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure of an appropriate speech traffic channel, which, if the MS supports both TCH/FS and TCH/HS, is at the preferred rate, the MS performs correctly that assignment procedure.

6)	To verify subsequent correct performance of a connect procedure.

7)	To verify that subsequently the MS has through connected the TCH in both directions.

8)	To verify that the call is cleared correctly.

26.9.6.1.1.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Supported rate for speech: (p1 = F/H, F).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The MS is made to initiate an emergency call. The call is established with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	  3	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				emergency call establishment.
	  4	SS -> MS	IMMEDIATE ASSIGNMENT	
	  5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
	  6	SS -> MS	AUTHENTICATION REQ
	  7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 10	SS		SS starts ciphering.
	 11	MS -> SS	EMERGENCY SETUP	If p1 = F/H, the message must contain
				one bearer capability IE indicating 
				in the radio channel requirement field
				"dual rate/half rate preferred" or 
				"dual rate/full rate preferred". 

				If p1 = F, the message must either
				contain no bearer capability IE or
				contain one bearer capability IE 
				indicating in the 
				radio channel requirement field 
				"full rate channel".
	 12	SS -> MS	CALL PROCEEDING
	 13	SS -> MS	ALERTING
	 14	SS -> MS	ASSIGNMENT COMMAND	The rate of the channel is that one
				indicated by the EMERGENCY SETUP
				message, if that message did not 
				offer a choice, and the rate is the
				preferred one else.
	 15	MS -> SS	ASSIGNMENT COMPLETE
	 16	SS -> MS	CONNECT
	 17	MS -> SS	CONNECT ACKNOWLEDGE
	 18	MS		The TCH is
				through connected in both directions.
	 19	SS -> MS	DISCONNECT
	 20	MS -> SS	RELEASE
	 21	SS -> MS	RELEASE COMPLETE
	 23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

##### 26.9.6.1.2	Structured procedures / emergency call / idle updated, non-preferred channel rate

The test is performed if the MS supports both TCH/HS and TCH/FS see PICS/PIXIT statement).

It is identical to the test in 26.9.6.1.1 except that in step 14 the assigned TCH has the non-preferred rate.

#### 26.9.6.2	Structured procedures / emergency call / idle, no IMSI

##### 26.9.6.2.1	Structured procedures / emergency call / idle, no IMSI / accept case

26.9.6.2.1.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

4),5)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

6)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

7)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 5.2.1.1.

For conformance requirements 4 and 5:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 6:

TS GSM 04.08, sections 5.2.1.6 and 5.1.3.

For conformance requirement 7:

TS GSM 04.08, section 5.4.

26.9.6.2.1.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message in which the cipher key sequence number IE indicates "no key is available", the CM service type IE indicates "emergency number establishment", and the mobile identity IE specifies the IMEI of the MS.

3)	To verify that after receipt of a CM SERVICE ACCEPT message from the SS, the MS sends an EMERGENCY SETUP message.

4)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure of an appropriate speech traffic channel, which, if the MS supports both TCH/FS and TCH/HS, is at the preferred rate, the MS performs correctly that assignment procedure.

5)	To verify subsequent correct performance of a connect procedure.

6)	To verify that subsequently the MS has through connected the TCH in both directions.

7)	To verify that the call is cleared correctly.

26.9.6.2.1.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	  3	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"emergency call".
	  4	SS -> MS	IMMEDIATE ASSIGNMENT	
	  5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
				The mobile identity IE specifies the
				IMEI of the MS.
				The cipher key sequence number IE
				indicates "no key is available".
				The mobile station classmark IE is as
				specified by the manufacturer in a
				PICS/PIXIT statement.
	  4	SS -> MS	CM SERVICE ACCEPT	
	 11	MS -> SS	EMERGENCY SETUP	
	 12	SS -> MS	CALL PROCEEDING
	 13	SS -> MS	ALERTING
	 14	SS -> MS	ASSIGNMENT COMMAND	The rate of the channel is one
				indicated by the EMERGENCY SETUP
				message.
	 15	MS -> SS	ASSIGNMENT COMPLETE
	 16	SS -> MS	CONNECT
	 17	MS -> SS	CONNECT ACKNOWLEDGE
	 18	MS		The TCH is through connected in both
				directions.
	 19	SS -> MS	DISCONNECT
	 20	MS -> SS	RELEASE
	 21	SS -> MS	RELEASE COMPLETE
	 23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

##### 26.9.6.2.2	Structured procedures / emergency call / idle, no IMSI / reject case

26.9.6.2.2.1	Conformance requirement

1)	The MS in the "idle, no IMSI" state (no SIM inserted), after the number 112 has been entered, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	In the situation at the end of test purpose 2, when the MS receives a CM SERVICE REJECT message, it shall abandon the emergency call.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4

For conformance requirement 3:

TS GSM 04.08 section 3.4.7, TS GSM 04.08 section 4.5.1.1.

26.9.6.2.2.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message in which the cipher key sequence number IE indicates "no key is available", the CM service type IE indicates "emergency call establishment", and the mobile identity IE specifies the IMEI of the MS.

3)	To verify that after receipt of a CM SERVICE REJECT message from the SS, the MS abandons the emergency call establishment.

26.9.6.2.2.3	Method of test

Related PICS/PIXIT statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. The SS responds to the CM SERVICE REQUEST from the MS with a CM SERVICE REJECT message specifying in the reject cause IE the reject cause value "IMEI not accepted". The SS then verifies for during 5 seconds that the MS does not send a layer 3 message. Then the call is cleared by the SS. The SS verifies during 20 seconds after disconnection of the main signalling link that the MS does not initiate an RR connection establishment.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	  3	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"emergency call".
	  4	SS -> MS	IMMEDIATE ASSIGNMENT	
	  5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
				The mobile identity IE specifies the
				IMEI of the MS.
				The cipher key sequence number IE
				indicates "no key is available".
				The mobile station classmark IE is as
				specified by the manufacturer in a
				PICS/PIXIT statement.
	  4	SS -> MS	CM SERVICE REJECT	the reject cause IE specifies 
				reject cause value #5, 
				"IMEI not accepted".
	  5	SS		During 5 seconds, the SS verifies
				that the MS does not send L3 messages.
	  6	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	  7	SS		During 20 seconds, the SS verifies
				that the MS does not initiate an 
				RR connection establishment.

Specific Message Contents:

-

### 26.9.7	Default contents of messages

ALERTING (mobile station to network direction)

Information element			Value/remark

Facility					Omitted

User-user 				Omitted

SS version				Omitted

ALERTING (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

User-user 				Omitted

ASSIGNMENT COMMAND

Information element			Value/remark

Description of the first channel		describes non-hopping Bm+ACCHs or Lm+ACCHs as

appropriate for the test

Power Command				As in section 26.1.1

Frequency list				Omitted

Cell channel description			Omitted

Mode of the first channel 			appropriate for on bearer capability chosen for the test

Description of the second channel		Omitted

Mode of the second channel			Omitted

Mobile allocation				Omitted

Starting time				Omitted

Cipher mode setting			Omitted

ASSIGNMENT COMPLETE

Information element			Value/remark

RR cause				normal event

AUTHENTICATION REQUEST

Information element			Value/remark

Ciphering key sequence number		Arbitrary

Spare half octet				(spare bits)

Authentication parameter RAND		Arbitrary

AUTHENTICATION RESPONSE

Information element			Value/remark

Authentication parameter SRES		Correct for given SRES

CALL CONFIRMED

Information element			Value/remark

Repeat indicator				Omitted

Bearer capability 1 			Omitted if the MS only supports TCH/F. Included if the

MS supports both TCH/F and TCH/H; in that case, the

"radio channel requirement" field specifies "dual

rate/half rate preferred" or "dual rate/full rate

preferred" or "half rate channel", all other parameters

are same as in the bearer capability 1 IE of the

received SETUP message.

Bearer capability 2			Omitted

Cause					Omitted

CALL PROCEEDING

Information element			Value/remark

Repeat Indicator				Omitted

Bearer Capability 1			Omitted if the SETUP message did not specify in the

bearer capability 1 IE a connection element value

"both, transparent preferred" or "both, non-

transparent preferred". Otherwise included; in that

case the connection element specifies the value that is

appropriate for the selected teleservice (either value

"transparent" or value "non transparent (RLP)"), all

other parameters are same as in the bearer capability

1 IE of the received SETUP message.

Bearer Capability 2			Omitted

Facility					Omitted

Progress indicator				Omitted

CHANNEL RELEASE

Information element			Value/remark

RR cause  				Normal event

CHANNEL REQUEST

Information element			Value/remark

Establishment cause 			Answer to paging (100)

Random reference				Arbitrary value of 5 bits length

CIPHERING MODE COMMAND

Information element			Value/remark

Cipher mode setting

algorithm identifier			indicates a supported algorithm

SC					Start ciphering

Cipher response

CR					IMEI must not be included

CIPHER MODE COMPLETE

Information element			Value/remark

Mobile equipment identity			Omitted

CM SERVICE ACCEPT

Information element			Value/remark

none but message head

CM SERVICE REQUEST

Information element			Value/remark

CM service type				Mobile originating call

establishment or packet mode

connection establishment

Ciphering key sequence number		CKSN of the MS

Mobile station classmark 2			as given by PICS.

Mobile identity				TMSI of MS

CONNECT (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

Connected number			Omitted

Connected subaddress			Omitted

User-user				Omitted

CONNECT (mobile station to network direction)

Information element			Value/remark

Facility					Omitted

Connected subaddress			Omitted

User-user				Omitted

SS version				Omitted

CONNECT ACKNOWLEDGE

Information element			Value/remark

none but message head

DISCONNECT (network to mobile station direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

Progress indicator				Omitted

User-user 				Omitted

DISCONNECT (mobile station to network direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

User-user 				Omitted

SS version				Omitted

IMMEDIATE ASSIGNMENT

Information element			Value/remark

Page mode 				Normal paging

Channel description			describes a valid SDCCH+SACCH

in non-hopping mode

Request reference

Random access information		As received from MS

N51, N32, N26			Corresponding to frame number

of the CHANNEL REQUEST

Timing advance				Arbitrary

Mobile allocation				Empty (L=0)

Starting time				Omitted

PAGING REQUEST TYPE 1

Information element			Value/remark

L2 pseudo length				L2 pseudo length of the message

Page Mode				Normal Paging

Channels needed for Mobiles 1 and 2

channel (first)			any channel

channel (second)			any channel

Mobile identity 1				TMSI of MS under test

Mobile identity 2				Omitted

P1 rest octets				(spare octets)

PAGING RESPONSE

Information element			Value/remark

Ciphering key sequence number		Value assigned to MS in the initial conditions

Spare half octet				(spare bits)

Mobile station classmark 2			as given by PICS

Mobile identity				specifies TMSI of MS

RELEASE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Second cause				Omitted

Facility					Omitted

User-user				Omitted

RELEASE (mobile station to network direction)

Information element			Value/remark

Cause					Omitted

Second cause				Omitted

Facility					Omitted

User-user				Omitted

SS version				Omitted

RELEASE COMPLETE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Facility					Omitted

User-user				Omitted

RELEASE COMPLETE (mobile station to network direction)

Information element			Value/remark

Cause					Omitted

Facility					Omitted

User-user				Omitted

SS version				Omitted

SETUP (MS to SS)

Information element			Value/remark

BC Repeat indicator			Omitted

Bearer capability 1				Appropriate for the teleservice selected for the test

Bearer capability 2				Omitted

Facility					Omitted

Calling party subaddress			Omitted

Called party BCD number			As entered

Called party subaddress			Omitted

LLC repeat indicator			Omitted

Low layer compatibility I			Appropriate for teleservice selected for the test

Low layer compatibility II			Omitted

HLC repeat indicator			Omitted

High layer compatibility i			Appropriate for teleservice selected for the test

High layer compatibility ii			Omitted

User-user				Omitted

SS version				Omitted

CLIR suppression				Omitted

SETUP (SS to MS)

Information element			Value/remark

BC repeat indicator			Omitted

Bearer capability 1				Appropriate for teleservice selected for the test

Bearer capability 2				Omitted

Facility					Omitted

Progress indicator				Omitted

Signal					Omitted

Calling party BCD number			Omitted

Calling party subaddress			Omitted

Called party BCD number			Omitted

Called party subaddress			Omitted

LLC repeat indicator			Appropriate for the teleservice selected for the test

Low layer compatibility I			Appropriate for teleservice selected for the test

Low layer compatibility II			Appropriate for the teleservice selected for the test

HLC repeat indicator			Appropriate for the teleservice selected for the test

High layer compatibility i			Appropriate for the teleservice selected for the test

High layer compatibility ii			Appropriate for the teleservice selected for the test

User-user				Omitted

## 26.10	E-GSM signalling

### 26.10.1	E-GSM signalling / general considerations

Section 26.10 only applies to E-GSM mobile stations.

Conformance requirements of section 26 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26 under the described GSM900 conditions.

The purpose of this extra section is to test the different procedures which may be impacted when some channel uses E-GSM frequencie(s).

### 26.10.2	E-GSM signalling / RR

#### 26.10.2.1	E-GSM signalling / RR / Measurement

This test applies E-GSM mobile stations.

Conformance requirements of section 26.6.3 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.3 under the described GSM900 conditions.

The purpose of this extra section is to test the reporting of mesurements in the case where cells use E-GSM frequencie(s). Several coding formats may be used by the network in the SYSTEM INFORMATION message.

26.10.2.1.1	Conformance requirements

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the values in the MEASUREMENT REPORT message shall contain measurement results for the 6 th strongest cells on which the mobile is asked to report.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

##### 26.10.2.1.2	Test purpose

To test that, when the SS gives information about neighbouring cells, the MS reports appropriate results.

##### 26.10.2.1.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BSCC	ARFCN	Cell identity

Serving, S1	-60	1	3	002	0001H

Neighbour, N1	-85	1	5	990	0002H

Neighbour, N2	-80	1	7	1005	0003H

Neighbour, N3	-75	1	1	000	0004H

Neighbour, N4	-55	1	3	026	0005H

Neighbour, N5	-50	1	5	1020	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	1003	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed 6 times.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5, 5bis & 6  on the SACCH. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest present carriers of the supported band have been obtained.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, c= 1 to 6.

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT  are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Protocol Descriminator		RR management

Message Type			Sys Info 5.

Neighbour Cells Description

- EXT IND			Information Element carries only a part of the BA.

- BA-IND			0

for c=1, use range 128

to encode the following frequencies :

(26, 38)

for c=2, use range 256

to encode the following frequencies

(990, 1003, 1005)

for c=3, use range 512

to encode the following frequencies

(520, 990, 1003, 1005, 1020)

for c=4, use range 1024

to encode the following frequencies

(0, 26, 38, 990, 1003, 1005)

for c=5, use variable Bitmap

to encode the following frequencies

(0, 26, 38)

for c=6, use Bitmap 0

to encode the following frequencies

(26)

SYSTEM INFORMATION TYPE 5bis:

Information Element			value/remark

Protocol Descriminator		RR management

Message Type			Sys Info 5bis.

Neighbour Cells Description

- EXT IND			Information Element carries only a part of the BA.

- BA-IND			0

for c=1, use range 512

to encode the following frequencies :

(520, 990, 1003, 1005, 1020)

for c=2, use range 128

to encode the following frequencies

(0, 26, 38)

for c=3, use range 256

to encode the following frequencies

(0, 26, 38)

for c=4, use range 1024

to encode the following frequencies

(520, 1000, 1020)

for c=5, use variable Bitmap

to encode the following frequencies

(884, 990, 1003, 1005)

for c=6, use range 512

to encode the following frequencies

(520, 990, 1003)

SYSTEM INFORMATION TYPE 6:

Information Element			value/ remark

Protocol Discriminator		RR Management

Message Type			sys info 6

Cell Identity				default

LAI					default

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall not use DTX

- Radio-Link-Timeout	default

PLMN permitted			only NCC 1 permitted

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 1

RXLEV-SUB-SERVING-CELL	See note 1

MEAS-VALID			See note 3

RXQUAL-FULL-SERVING-CELL	See note 1

RXQUAL-SUB-SERVING-CELL	See note 1

NO-NCELL-M			n (see note 2)

RXLEV-NCELL-1		See note 1

BCCH-FREQ-NCELL-1		See note 2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 1

BCCH-FREQ-NCELL-2		See note 2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 1

BCCH-FREQ-NCELL-3		See note 2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 1

BCCH-FREQ-NCELL-4		See note 2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		See note 1

BCCH-FREQ-NCELL-5		See note 2

BSIC-NCELL-5			Corresponds to that of BCCH-FREQ-NCELL-5

RXLEV-NCELL-6		See note 1

BCCH-FREQ-NCELL-6		See note 2

BSIC-NCELL-6			Corresponds to that of BCCH-FREQ-NCELL-6

NOTE 1:	These actual values are not checked.

NOTE 2 :	ARFCN 2 is the serving cell carrier.

c=1 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 1020, n=6

c=2 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0, 1020 stronger than 1005 but not broadcasted in SYS INFO), n=6

c=3 report on ARFCNs  2, 20, 26, 38, 1003, 1005, 1020, 0 (990 less strong, 520 DCS), n=6

c=4 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0 (1000 less strong, 520 DCS, 1020 not broadcasted in SYS INFO), n=6

c=5 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0 (884 DCS), n=6

c=6 report on 2, 26, 990, 1003,  n=6

NOTE 3:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

#### 26.10.2.2	E-GSM signalling / RR / Immediate assignment

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.1 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.1 under the described GSM900 conditions.

The purpose of this extra section is to test the immediate assignment procedure in the case where the target channel is E-GSM frequency.

##### 26.10.2.2.1	Conformance requirement

Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on a supported channel described in the IMMEDIATE ASSIGNMENT message, using some E-GSM frequencie.

Reference(s)

GSM 04.08 section 3.3.1.2.1

##### 26.10.2.2.2	Test purpose

To verify that the MS can correctly set up a dedicated control channel when E-GSM frequencies are used.

This tested for a SDCCH/8

##### 26.10.2.2.3	Method of test

Initial Conditions

System Simulator:

1 cell,

CCCH-CONF is set to '1 basic physical channel used for CCCH not combined with SDCCHs'.

SYTEM INFORMATION type 1 message contains the following frequencies in the Cell Channel Description IE: 0, 30, 40, 66, 80, 1005, 1010, 1015 (use range 1024 to encode)

BCCH carrier number 1015

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statement(s)

none

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

This test procedure is performed twice.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST message the SS assigns an SDCCH channel using some E-GSM frequencies. The MS shall go to the correct channel and send a PAGING RESPONSE message. Then the SS initiates RR-release by sending a CHANNEL RELEASE message.

Maximum Duration of Test

15 seconds

Expected Sequence

The sequence is performed for execution counter k=1,2

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	cause "answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Channel Type : see below
	4	MS -> SS	PAGING RESPONSE	Shall be sent on the correct channel
	5	SS -> MS	CHANNEL RELEASE

Specific Message Contents

IMMEDIATE ASSIGNMENT

as default except :

Information element				remark/value

Channel description

- Channel Type 			SDCCH/8

- Timeslot number			arbitrary but not zero

- Training sequence code			arbitrary

- Hopping channel			k=1 Single RF

k=2	RF hopping channel

- Channel Selector			k=1  ARFCN=1015

k=2 MAIO=arbitrarily chosen

HSN arbitrary chosen from the set (1,..63)

Mobile allocation				k=1 empty

k=2 indicates the following frequencies

(0, 80, 1005, 1010)

#### 26.10.2.3	E-GSM signalling / RR / channel assignment procedure

This test is only applicable to an E-GSM mobile station.

Conformance requirements of section 26.6.4 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.4 under the described GSM900 conditions.

The purpose of this extra section is to test the assignment in the case where the allocated channel uses E-GSM frequencie(s) : in the case of frequency hopping several coding formats may be used by the network in the ASSIGNMENT COMMAND message.

##### 26.10.2.3.1	Conformance requirements

1.	Upon receipt of the ASSIGNMENT COMMAND message, the mobile station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the main signalling links).

2.	The MS shall apply the hopping frequencies specified in ASSIGNMENT COMMAND message in the mobile allocation or frequency list or frequency short list at the time accessing the new channel.

References

GSM 04.08, subclause  3.4.3

GSM 04.08 subclause 9.1.2

##### 26.10.2.3.2	Test purpose

1.	To verify that upon receipt of an ASSIGNMENT COMMAND, the MS switches to the channel defined in the ASSIGNMENT COMMAND, establishes the link and sends an ASSIGNMENT COMPLETE message.

2.	To verify that an MS, having received an ASSIGNMENT COMMAND, is able in case of frequency hopping to decode the mobile allocation and frequency list correctly and applies the specified frequencies.

##### 26.10.2.3.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state with a TMSI allocated.

Related PICS/PIXIT Statements

-	TCH supported (Y/N)

-	The supported channel mode(s) need to be declared.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The test procedure is performed 2 times.

The SS pages the MS and allocates an SDCCH. Then a channel is assigned with ASSIGNMENT COMMAND. Each time the MS shall switch to the assigned channel, establish the link and send an ASSIGNMENT COMPLETE message.

The SS initiates the channel release procedure.

Maximum Duration of Test

3 min

Expected Sequence

The test sequence is performed for execution counter k=1 to 2

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGN	Timeslot Number = n.
	4	MS -> SS	PAGING RESPONSE

5	SS -> MS	ASSIGN COMMAND	See specific message contents
				Timeslot Number = (n+1) mod 8
	6	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after 
				establishment of the main signalling link.
				Steps 5 and 6 are repeated cmax times, where 
				cmax is the number of frequency formats allowed
				for each value of k.
				Use repetition counter c:
				See specific message content.

7	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents

ASSIGNMENT COMMAND:

Information element			value/remark

Protocol Discriminator			RR

Skip indicator				0000

Message type				ASSIGNMENT COMMAND

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Power Command

- Power level			Arbitrarily chosen

For k=1

Cell Channel Description IE is not included

Frequency list				for c=1, use range 128

to encode the following frequencies :

(1005, 1010, 1015)

for c=2, use range 256

to encode the following frequencies

(73, 74, 75, 76, 77)

for c=3, use range 512

to encode the following frequencies

(980, 981, 982, 983)

for c=4, use range 1024

to encode the following frequencies

(30, 40, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 991, 992, 993, 994, 1015)

with an encoding origin set to 900

for c=6, use Bitmap 0

to encode the following frequencies

(20, 40, 66)

For k=2

Cell Channel Description 			for c=1, use range 128

to encode the following 13 frequencies :

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=2, use range 256

to encode the following 13 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108,

114, 115)

for c=3, use range 512

to encode the following 13 frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=4, use range 1024

to encode the following 8 frequencies

(0, 30, 40, 66, 80, 1005, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=6, use Bitmap 0

to encode the following 12 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114)

For k = 2

Mobile Allocation				indicates the following the frequencies :

for c=1 (1005, 1010, 1015)

for c=2 (73, 74, 75, 76, 77)

for c=3 (980, 981, 982, 983)

for c=4 (0, 30, 40, 1010, 15)

for c=5 (990, 991, 992, 993, 994)

for c=6 (20, 40, 66)

Mode of the First channel			speech full rate for TCH/F

except if speech is not supported :

arbitrary from those supported

Starting Time				not included

#### 26.10.2.4	E-GSM signalling / RR / Handover

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.5 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.5 under the described GSM900 conditions.

The purpose of this extra section is to test the handover in the case where the target channel uses E-GSM frequencie(s) : in the case of frequency hopping several coding formats may be used by the network in the HANDOVER COMMAND message.

##### 26.10.2.4.1	E-GSM signalling / RR / Handover / Successful handover

26.10.2.4.1.1	Conformance requirements

The MS shall correctly apply the handover procedure from a channel without frequency hopping in the primary band to a channel with frequency hopping using P-GSM and E-GSM frequencies whatever the coding used for the frequency hopping description.

References

GSM 04.08, section 3.4.4 and 9.1.15

26.10.2.4.1.2	Test purpose

To check that the MS correctly performs a non-synchronized handover, from a non hopping primary band SDCCH to a hopping TCH or SDCCH using E-GSM frequencies, whatever the coding used for the hopping sequence description and that it activates the new channel correctly.

This is tested in the following case :

E-GSM signalling / Handover / successful / call under establishment / non-synchronized /

- from SDCCH/8 to TCH/F if the MS supports a TCH

- from SDCCH/8 to SDCCH/8 if not

26.10.2.4.1.3	Method of test

Initial Conditions

System Simulator:

2 cells A and B with same LAI, default parameters

except for Cell A : the broadcasted Cell Channel Description in SYSTEM INFORMATION type 1 message contains the following frequencies : 20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114 (encoded using Bit Map 0 format)

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Related PICS/PIXIT Statements

Support of TCH (Y/N)

Support of speech (Y/N)

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

"Idle, updated" with TMSI allocated and camped on cell B.

Test Procedure

The test procedure is performed 18 times.

A Mobile Originating Call is initiated on cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND message, ordering the MS to switch to cell B.

The MS shall then begin to send access bursts on the new channel, a TCH/F if supported (see PICS/PIXIT) or an SDCCH if not, to cell B.

The SS observes the access bursts. After receiving n access bursts, n being randomly drawn between 10 and 20 for the TCH case, 2 and 5 for the SDCCH (see table 2 of section 26.6.5) , the SS sends one PHYSICAL INFORMATION message with a Timing Advance of 20 (see table 1 of section 26.6.5).

The MS shall activate the channel in sending and receiving mode. Then the MS shall establish a signalling link using the correct timing advance. The MS shall be ready to transmit a HANDOVER COMPLETE message before x ms after the end of the PHYSICAL INFORMATION message but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

The term 'ready to transmit' is specified in TS GSM 04.13. The value of 'x' depends upon the target channel:

case SDCCH/8 x = 750

case TCH/F x=500

Maximum Duration of Test

10 min

Expected Sequence

The sequence is performed for execution counter k=1 to 3 and c=1 to 6

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated on cell A
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call,
				NECI not set to 1
	3	SS -> MS	IMM ASSIGNMENT	see specific message contents
				
	  4	MS -> SS	CM SERVICE REQUEST	CM Service Type = Mobile Origina-
				ting Call Establishment.
	  5	SS -> MS	CIPH MODE COMMAND
	  6	MS -> SS	CIPH MODE COMPLETE
	  7	MS -> SS	SETUP	Last L2 frame not acknowledged by
				the SS.
	 8	SS -> MS	HANDOVER COMMAND	 see specific message contents
	 9	MS -> SS	HANDOVER ACCESS	Repeated  on every burst
				 of the uplink main DCCH until reception
				 of PHYSICAL	INFORMATION.
				 Handover Reference as
				included in the HANDOVER COMMAND
	 10	SS -> MS	PHYSICAL INFO	Sent after reception of n HANDOVER
				ACCESS. Timing Advance : 20
	11	MS -> SS	SABM	Sent without information field
	12	SS -> MS	UA
	 13	MS -> SS	HANDOVER COMPLETE	This message shall be ready
				 to be transmitted before 'x' ms
				 after the completion of step 10
	 14	MS -> SS	SETUP	Same N(SD) as in step 7.
	 15	SS -> MS	CHANNEL RELEASE

Specific Message Contents

IMMEDIATE ASSIGNMENT

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		20

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Mobile Allocation				indicates the following frequencies :

(20, 66, 78)

HANDOVER COMMAND

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		40

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Synchronisation Indication IE is not included

For k = 1,

Cell Channel DescriptionIE is not included

Frequency short list			for c=1, use range 128

to encode the following frequencies :

(1005, 1010, 1015)

for c=2, use range 256

to encode the following frequencies

(73, 74, 75, 76, 77)

for c=3, use range 512

to encode the following frequencies

(980, 981, 982, 983)

for c=4, use range 1024

to encode the following frequencies

(30, 40, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 991, 992, 993, 994, 1015)

For k=2

Cell Channel Description IE is not included

Frequency list				for c=1, use range 128

to encode the following frequencies :

(1005, 1010, 1015)

for c=2, use range 256

to encode the following frequencies

(73, 74, 75, 76, 77)

for c=3, use range 512

to encode the following frequencies

(980, 981, 982, 983)

for c=4, use range 1024

to encode the following frequencies

(30, 40, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 991, 992, 993, 994, 1015)

for c=6, use Bitmap 0

to encode the following frequencies

(20, 40, 66)

For k=3

Cell Channel Description 			for c=1, use range 128

to encode the following 13 frequencies :

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=2, use range 256

to encode the following 13 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108,

114, 115)

for c=3, use range 512

to encode the following 13 frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=4, use range 1024

to encode the following 8 frequencies

(0, 30, 40, 66, 80, 520, 975, 1005, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=6, use Bitmap 0

to encode the following 12 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114)

For k = 3

Mobile Allocation				indicates the following the frequencies :

for c=1 (1005, 1010, 1015)

for c=2 (73, 74, 75, 76, 77)

for c=3 (980, 981, 982, 983)

for c=4 (30, 40, 1010, 1015)

for c=5 (990, 991, 992, 993, 994)

for c=6 (20, 40, 66)

Mode of the First channel			speech full rate for TCH/F

except if speech is not supported :

arbitrary from those supported for SDCCH/8

##### 26.10.2.4.2	E-GSM signalling / RR / Handover / layer 1 failure

##### 26.10.2.4.2.1	Conformance requirements

During a handover from a channel in the E-GSM band to a channel in the P-GSM band, or the contrary, the MS shall correctly return to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell.

References

GSM 04.08 section 3.4.4.

26.10.2.4.2.2	Test purpose

To check  that the MS correctly returns to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell, even if the origin is in the P-GSM band and the targetin the E-GSM band.

26.10.2.4.2.3	Method of test

Initial Conditions

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call on a P-GSM channel of cell A. power level = 10

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/FS

Supported mode(s).

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell A), used power level 10.

Test Procedure

The MS is in the active state (U10) of a call on a P-GSM channel of cell A (used power level 10). The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B (power level 12). With the exception of normal BCCH signalling, the SS does not transmit anything on cell B (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test

1 min

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	to an E-GSM channel
				see specific message contents
	  2	MS -> SS	HANDOVER ACCESS	Several messages are sent, all
				with correct Handover References.
	  3	MS -> SS	HANDOVER FAILURE	Sent on old channel,
				RR cause value ="Abnormal release,
				unspecified", "Abnormal release,
				channel unacceptable", "Abnormal
				release, timer expired" or
				"Abnormal release, no activity on
				the radio path". Layer 1 header
				has the same power level as the
				layer 1 header in step 1.
				Shall be sent
				within 3 seconds from the trans-
				mission of HANDOVER COMMAND.

Specific Message Contents

HANDOVER COMMAND

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		40

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				chosen randomly from the set

(1,2...63)

Synchronisation Indication IE is not included

Cell Channel DescriptionIE is not included

Frequency short list			use range 128

to encode the following frequencies :

(1005, 1010, 1015)

Mode of the first channel 			Full rate speech if supported

If not, arbitrary from those supported except signalling.

#### 26.10.2.5	E-GSM signalling / RR / Frequency Redefinition

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.6 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.6 under the described GSM900 conditions.

The purpose of this extra section is to test the frequency redefinition procedure in the case where the target channel uses E-GSM frequencie(s) : in the case of frequency hopping several coding formats may be used by the network in the FREQUENCY REDEFINITION message.

##### 26.10.2.5.1	Conformance requirements

1) 	To verify that the MS, after receiving a FREQUENCY REDEFINITION message, correctly starts using the new frequencies, and hopping sequence when some E-GSM frequencies are used.

2) 	The last received Cell Channel Description information element is used to decode the Mobile Allocation IE received on the FREQUENCY REDEFINITION message.

References

GSM 04.08 3.4.5 and 9.1.13.

26.10.2.5.2	Test purpose

1)	To verify that the MS, after receiving a FREQUENCY REDEFINITION message, starts using the new frequencies and hopping sequence when some E-GSM frequencies are used.

2)	To check that the last received Cell Channel Description information element is used to decode the Mobile Aloocation IE received in the FREQUENCY REDEFINITION message.

##### 26.10.2.5.3	Method of test

Initial conditions

System Simulator:

1 cell; default parameters.

Mobile Station:

The MS is in "idle, updated" state with TMSI allocated.

Related PICS/PIXIT statement(s)

- Support of a TCH (Y/N)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The test procedure is performed six times.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a TCH/F if supported otherwise a SDCCH/8. Then the SS sends to MS a FREQUENCY REDEFINITION. The MS shall then use the new frequencies/hopping sequence.

Maximum duration of test

3 min

Expected sequence

The sequence is performed for execution counter c=1 to 6

Step	Direction	Message	Comments

1	SS->MS	PAGE REQ TYPE 1
	2	MS->SS	CHANNEL REQUEST

3	SS->MS	IMMEDIATE ASSIGN	Channel type=TCH/F if a TCH is supported 
				otherwise, Channel type=SDCCH/8. 
	4	MS->SS	PAGING RESPONSE	Sent on the correct channel after 
				establishment of the main signalling link
	5	SS->MS	FREQUENCY REDEFINITION	See specific message contents.
	6	-----	------------------	The SS checks that the MS is transmitting on 
				the correct frequencies.

7	SS->MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents

FREQUENCY REDEFINITION

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		40

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Cell Channel Description 			for c=1, use range 128

to encode the following 13 frequencies :

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=2, use range 256

to encode the following 13 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108,

114, 115)

for c=3, use range 512

to encode the following 13 frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=4, use range 1024

to encode the following 8 frequencies

(0, 30, 40, 66, 80, 1005, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=6, not present

(the mobile station will use the last Cell Channel

Description IE received, i.e. the one broadcast in the

SYSTEM INFORMATION TYPE 1 message).

Mobile Allocation				indicates the following frequencies :

for c=1 (1005, 1010, 1015)

for c=2 (73, 74, 75, 76, 77)

for c=3 (980, 981, 982, 983)

for c=4 (30, 40, 1010, 1015)

for c=5 (990, 991, 992, 993, 994)

for c=6 (30, 50, 70)

Starting time				indicates

(current frame number + 100 frames) mod 42432

### 26.10.3	E-GSM signalling / Structured procedure

Conformance requirements of section 26.9 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.9 under the described GSM900 conditions.

The purpose of these tests is to verify that the MS performs certain elementary procedures of the RR, MM, and CC protocol correctly within a structured procedure. This tested here in an E-GSM context to evaluate the global behaviour of an E-GSM mobile station using its specific frequency capability.

One mobile originated call and one emergency call are performed.

In one of the different cases the different following situations occur : early and late assignments, call release initiated by the network and by the MS.

#### 26.10.3.1	E-GSM signalling / Structured procedure / Mobile originated call

##### 26.10.3.1.1	Conformance requirement

1)	An MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS , shall start to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

2)	Upon receipt of the ASSIGNMENT COMMAND message, the Mobile Station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the data links). After the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message, specifying cause 'normal event', to the network on the main DCCH.

3,4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

References

Conformance requirement 1:	TS GSM 04.08 section 3.3.1.

Conformance requirement 2:	TS GSM 04.08 section 3.4.3.1, 3.4.3.2

Conformance requirement 3:	TS GSM 04.08 section 3.4.3.

Conformance requirement 4:	TS GSM 04.08 section 5.2.1.1.6.

Conformance requirement 5:	TS GSM 02.07.

Conformance requirement 6:	TS GSM 04.08 sections 5.2.1.1.6 and 5.1.3.

##### 26.10.3.1.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message.

2)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed authentication and cipher mode setting procedures, after having sent a SETUP message, after having received a CALL PROCEEDING message followed by an ALERTING message and an ASSIGNMENT COMMAND message allocating an appropriate TCH, the MS sends an ASSIGNMENT COMPLETE message.

3)	To verify that subsequently, after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message returns a CONNECT ACKNOWLEDGE message.

4)	To verify that after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message attaches the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

##### 26.10.3.1.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM state “idle updated”.

Test procedure

The following test is performed for one teleservice supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with late assignment. The release of the call is initiated by the MS.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" is entered
	  3	MS		
	  4	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"originating call and the
				network does not set the NECI bit to 1".
	  5	SS -> MS	IMMEDIATE ASSIGNMENT	
	  6	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.
	  7	SS -> MS	AUTHENTICATION REQ
	  8	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	  9	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	  10	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	 11	SS		SS starts ciphering.
	 12	MS -> SS	SETUP
	 13	SS -> MS	CALL PROCEEDING
	 14	SS -> MS	ALERTING
	 15	MS		Depending on the PICS, an alerting
				indication is given.
	 16	SS -> MS	ASSIGNMENT COMMAND
	 17	MS -> SS	ASSIGNMENT COMPLETE
	 18	SS -> MS	CONNECT
	 19	MS -> SS	CONNECT ACKNOWLEDGE
	 20	MS		The appropriate bearer channel is
				through connected in both directions.
	 21	MS		If the call is a data call, 
				the TCH shall be through
				connected in both
				directions.
	22	MS		The MS is made to release the call.
	23	MS -> SS	DISCONNECT
	24	SS -> MS	RELEASE
	25	MS -> SS	RELEASE COMPLETE
	26	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

#### 26.10.3.2	E-GSM signalling / Structured procedures / emergency call

Emergency call establishment can be initiated by an MS whether location updating has been successful or not and whether a SIM is inserted into the MS or not; but only if the MS is equipped for speech.

This section is only applicable to an MS supporting speech.

##### 26.10.3.2.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

4),5)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

6)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

7)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 5.2.1.2.

For conformance requirements 4 and 5:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 6:

TS GSM 04.08, sections 5.2.1.1.6 and 5.1.3.

For conformance requirement 7:

TS GSM 04.08, section 5.4.

##### 26.10.3.2.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message specifying the correct CKSN and TMSI, with CM Service Type "emergency call establishment".

3)	To verify that after receipt of a CM SERVICE ACCEPT message from the SS, the MS sends an EMERGENCY SETUP message.

4)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure, the MS performs correctly that assignment procedure.

5)	To verify subsequent correct performance of a connect procedure.

6)	To verify that subsequently the MS has through connected the TCH in both directions.

7)	To verify that the call is cleared correctly.

This is tested in the following case :

Structured procedures / emergency call / idle, no IMSI / accept case

##### 26.10.3.2.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters except :

BCCH ARFCN = 990

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	  2	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"emergency call".
	  3	SS -> MS	IMMEDIATE ASSIGNMENT	
	  4	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
				The mobile identity IE specifies the
				IMEI of the MS.
				The cipher key sequence number IE
				indicates "no key is available".
				The mobile station classmark IE is as
				specified by the manufacturer in a
				PICS/PIXIT statement.
	  5	SS -> MS	CM SERVICE ACCEPT	
	 6	MS -> SS	EMERGENCY SETUP	
	 7	SS -> MS	CALL PROCEEDING
			
	8	SS -> MS	ASSIGNMENT COMMAND	The rate of the channel is one
				indicated by the EMERGENCY SETUP
				message.
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	 11	SS -> MS	CONNECT
	 12	MS -> SS	CONNECT ACKNOWLEDGE
	 13	MS		The TCH is through connected in both
				directions.
	 14	SS -> MS	DISCONNECT
	 15	MS -> SS	RELEASE
	 16	SS -> MS	RELEASE COMPLETE
	 17	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

#### 26.10.3.3	Default contents of messages

same as in 26.9.7 except for the following :

ASSIGNMENT COMMAND

Information element			Value/remark

Description of the first channel		describes non-hopping Bm+ACCHs or Lm+ACCHs as

appropriate for the test

ARFCN = 990

Power Command

-					Chosen arbitrarily but within the range supported by the

MS.

Frequency list				Omitted

Cell channel description			Omitted

Mode of the first channel 			appropriate for on bearer capability chosen for the test

Description of the second channel		Omitted

Mode of the second channel			Omitted

Mobile allocation				Omitted

Starting time				Omitted

Cipher mode setting			Omitted

IMMEDIATE ASSIGNMENT

Information element			Value/remark

Page mode 				Normal paging

Channel description			describes a valid SDCCH+SACCH

in non-hopping mode

ARFCN = 40

Request reference

Random access information		As received from MS

N51, N32, N26			Corresponding to frame number

of the CHANNEL REQUEST

Timing advance				Arbitrary

Mobile allocation				Empty (L=0)

Starting time				Omitted

### 26.10.4	E-GSM signalling / Default message contents

Default SYSTEM INFORMATION:

NOTE :	SYSTEM INFORMATION 7, and SYSTEM INFORMATION 8 messages are not used.

SYSTEM INFORMATION type 1 message

Information elements			Value/Remark

Cell Channel Description

For Cell A

-	Format identifier		Bit Map 0

-	Cell Allocation ARFCN		Channel Numbers 20, 30, 50 and 70.

For Cell B

-	Format identifier		Bit Map 0

-	Cell Allocation ARFCN		Channel Numbers 10, 12, 40, 60, 62, 63

RACH Control parameters			see below

SI1 Rest octets				see below

SYSTEM INFORMATION type 2 message

Information elements			Value/Remark

BCCH frequency list

For cell A

-	Format identifier		bit map 0

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 10, 20, 80, 90,

100, 110 and 120

-	EXT-IND			This IE does not carry the complete BA

For cell B

-	Format identifier		bit map 0

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 10, 36, 40, 114, 118

-	EXT-IND			This IE does not carry the complete BA

NCC permitted				see below

RACH control parameters			see below

SYSTEM INFORMATION type 2bis message

Information elements			Value/Remark

Extended BCCH frequency list

For cell A

-	Format identifier		range 256

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		988, 990, 1003

-	EXT-IND			This IE does not carry the complete BA

For cell B

-	Format identifier		range 128

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 1005, 1010, 1015

-	EXT-IND			This IE does not carry the complete BA

RACH control parameters			see below

SI 2bis rest octets				see below

SYSTEM INFORMATION type 3 message

Information elements			Value/Remark

Cell identity				see below

LAI						see below

Control channel description			see below

Cell options				see below

Cell Selection parameters			see below

RACH control parameter			see below

SI3 Rest octets 				see below

SYSTEM INFORMATION type 4 message

Information elements			Value/Remark

LAI						see below

Cell selection parameters			see below

RACH control parameters 			see below

CBCH Channel Description			see below

CBCH MA				see below

SI4 Rest octets				see below

SYSTEM INFORMATION type 5 message

Information elements			Value/Remark

BCCH frequency list

For cell A

-	Format identifier		bit map 0

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 10, 20, 80, 90,

100, 110 and 120

-	EXT-IND			This IE does not carry the complete BA

For cell B

-	Format identifier		Bit map 0

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 10, 36, 40, 114, 118

-	EXT-IND			This IE does not carry the complete BA

SYSTEM INFORMATION type 5bis message

Information elements			Value/Remark

Extension of BCCH frequency list description

For cell A

-	Format identifier		Range 256

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 988, 990, 1003

-	EXT-IND			This IE does not carry the complete BA

For cell B

-	Format identifier		range 128

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 1005, 1010, 1015

-	EXT-IND			This IE does not carry the complete BA

SYSTEM INFORMATION type 6 message

Information elements			Value/Remark

Cell identity 				see below

LAI						see below

Cell options 				see below

NCC permitted				see below

Common contents of information elements in SYSTEM INFORMATION TYPE 1 to 6 messages.

(CBCH) Channel Description		Not present

(CBCH) Mobile Allocation			Not present

Cell Identity

-	Cell Identity Value		0001H for cell A,

0002H for cell B

Cell Options

-	Power Control Indicator		Power Control Indicator is not set

-	DTX Indicator			MS shall not use DTX

-	Radio-Link-Timeout		8 SACCH blocks

Cell Selection Parameters

-	Cell-Reselect-Hysteresis		12 dB

-	MX-TXPWR-MAX-CCH		Minimum level

-	ACS				No addition cell parameters are present in

SYSTEM INFORMATION messages 7 and 8.

-	NECI				New establishment causes not supported

-	RXLEV-ACCESS-MIN		Minimum level

Control Channel Description

-	Attach-Detach allowed		No Attach/Detach

-	BS-AG-BLKS-RES		0 blocks reserved for access grant

-	CCCH-CONF			1 basic physical channel used for

CCCH, combined with SDCCHs

-	BS-PA-MFRMS		5 multiframe periods for transmission of

paging messages

-	T3212 Timeout value		Infinite

L2 pseudo length

-	System information 1		21

-	System information 2		22

-	System information 2bis		21

-	System information 3		18

-	System information 4		12

Location Area Identification

-	Mobile Country Code		001 decimal

-	Mobile Network Code		01 decimal

-	Location Area Code		0001H

Message Type

-	System information 1		00011001

-	System information 2		00011010

-	System information 2bis		00000010

-	System information 3		00011011

-	System information 4		00011100

-	System information 5		00011101

-	System information 5bis		00000101

-	System information 6		00011110

NCC Permitted				0000 0010

RACH Control Parameters

-	Max Retrans			Max 1 retrans

-	Tx-integer			5 slots used

-	Cell Barred for Access		Cell is not barred

-	Call Reestablishment Allowed	Not allowed

-	Access Control Class		Access is not barred

-	Emergency Call allowed		Yes

SI 1 rest octets				Not used (all bits are set to spare)

SI 2 rest octets				Not used (all bits are set to spare)

SI 3 rest octets				Not used (all bits are set to spare)

SI 4 rest octets				Not used (all bits are set to spare)

Default settings for cell A:

Downlink input level			63 dBmicroVolt emf

Uplink output power			minimum supported by the MS's power class

Propagation profile			static

BCCH/CCCH carrier number		20

Default settings for cell B:

Downlink input level			53 dBmicroVolt emf

Uplink output power			minimum supported by the MS's power  class

Propagation profile			static

BCCH/CCCH carrier number		10

Default message contents for other messages

For sections 26.10.1 to 26.10.2.5 		same as in 26.6.14

For section 26.10.3			same as in 26.9.7