## 26.7	Elementary procedures of mobility management

The tests are based on GSM 04.08 and GSM 03.03.

In this section, when the expected sequence require that "a mobile originated CM connection is attempted", it shall be for a service other than emergency call.

In this section, a initial CM message is either a SETUP message, a REGISTER message or a CP-DATA message (in that case the acknowledged mode of operation on SAPI 3 will have be established and this message will be sent on SAPI 3).

### 26.7.1	TMSI reallocation

The intention of the TMSI Reallocation procedure is to assign a new temporary identity for the MS. If the message is not understood by the MS, the network could not establish a link to the MS. As this is a common MM procedure, it can be initiated at any time.

26.7.1.1	Conformance requirement

1)	A Mobile Station shall acknowledge a new TMSI when explicitly allocated during a location updating procedure or an incoming call.

2)	The TMSI shall be updated on the SIM when the Mobile Station is correctly deactivated in accordance with the manufacturer's instructions.

3)	A Mobile Station shall answer paging with this TMSI and includes it in the Paging Response message.

Reference(s):

GSM 04.08 section 4.3.1, GSM 03.03 section 2, GSM 02.17 section 6.1.

26.7.1.2	Test purpose

To verify that the MS is able to receive and acknowledge a new TMSI by means of an explicit TMSI reallocation procedure.

To verify that the MS has stored the TMSI in a non-volatile memory.

The implicit reallocation procedure is tested in section 26.7.4.1.

26.7.1.3	Method of test

Initial conditions

System Simulator:

Two cells A and B, belonging to different location areas a and b, default parameters.

Mobile Station:

The MS has valid TMSI (= TMSI1), CKSN, Kc. It is "idle updated" on cell B.

Related PICS/PIXIT statement(s)

Switch off button Yes/No

Way to bring the MS into service

Foreseen final state of the MS

The MS has a valid TMSI (= TMSI1), CKSN, Kc. It is "idle updated" on cell A.

Test Procedure

The MS is paged in cell B and the ciphering mode is established. An explicit TMSI reallocation procedure is performed. The channel is released. The MS is switched off and then its power supply is interrupted for 10 seconds. The power supply is resumed and then the MS is switched on. The system simulator checks, by paging, whether the MS has stored the received TMSI.

The MS is made to select cell A. A normal location updating procedure is performed in cell A. An explicit TMSI reallocation procedure is performed and then the location updating procedure is accepted by the SS. The system simulator checks, by paging, whether the MS has stored the allocated TMSI.

Maximum duration of test

2 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B
	1	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI1
	2	MS -> SS	CHANNEL REQUEST	Establishment Cause: Answer to paging.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	CIPH MODE COMMAND	The SS starts deciphering.
	6	MS -> SS	CIPH MODE COMPLETE	The SS starts enciphering.
	7	SS -> MS	TMSI REALLOCATION CMD	"Mobile identity” = new TMSI 
					(TMSI2) different from TMSI 1.
	8	MS -> SS	TMSI REALLOCATION CMP	
	9	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits 
					for the disconnection of the main signalling link.
	10	MS		If possible (see PICS), the MS is switched off
	10a	MS		The power supply is interrupted for 10 seconds.
	11	MS		The MS is brought into service
	12	SS		The SS waits an amount of time which is enough 
					to guarantee that the MS is in service (listening 
					to its paging subchannel)
	13	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI2.
	14	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to paging.
	15	SS -> MS	IMMEDIATE ASSIGNMENT	
	16	MS -> SS	PAGING RESPONSE	"Mobile identity" =TMSI2.
	17	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits for 
					the disconnection of the main signalling link.
				The following messages are sent and shall be
				received on cell A
	18	SS		The RF level of cell B is lowered until the MS 
				selects cell A. The RF level of cell B is set 
				sufficiently low to ensure that cell B is not suitable 
				as defined in GSM 05.08 section 6.6.2
	19	MS -> SS	CHANNEL REQUEST	Establishment cause: Location updating
	20	SS -> MS	IMMEDIATE ASSIGNMENT	
	21	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				"ciphering key sequence number” = CKSN, 
				LAI = b, "mobile identity” = TMSI2.
	22	SS -> MS	TMSI REALLOCATION CMD	TMSI = TMSI1
	23	MS -> SS	TMSI REALLOCATION CMP	
	24	SS -> MS	LOCATION UPDATING ACC	This message does not contain the 
				optional Mobile Identity field.
	25	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits for 
				the disconnection of the main signalling link. 
				The SS waits an  
				amount of time which is enough to 
				guarantee that the MS is "idle updated" on cell A
	26	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the new TMSI (= TMSI1).
	27	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Answer to paging.
	28	SS -> MS	IMMEDIATE ASSIGNMENT	
	29	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the new TMSI (= TMSI1).
	30	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

### 26.7.2	Authentication

The purpose of this procedure is to verify the user identity. A correct response is essential to guarantee the establishment of the connection. If not, the connection will drop.

The SS shall be able to handle vectors of Kc, RAND, and SRES in a similar way as the MSC/BSS entities. The SS shall incorporate a test algorithm for generating SRES and Kc from RAND and Ki which operates as described in Annex 4.

#### 26.7.2.1	Authentication accepted

26.7.2.1.1	Conformance requirement

1)	A Mobile Station shall correctly respond to an Authentication Request message by sending an Authentication Response message with the SRES information field set to the same value as the one produced by the authentication algorithm in the network.

2)	A Mobile Station shall indicate in a Paging Response message the ciphering key sequence number which was allocated to it through the authentication procedure.

Reference(s)

GSM 04.08 section 4.3.2, GSM 03.03 section 2

26.7.2.1.2	Test purpose

1)	To check that a Mobile Station correctly responds to an Authentication Request message by sending an Authentication Response message with the SRES information field set to the same value as the one produced by the authentication algorithm in the network.

2)	To check that a Mobile Station indicates in a Paging Response message the ciphering key sequence number which was allocated to it through the authentication procedure.

26.7.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has valid TMSI, CKSN (CKSN1), Kc. It is "idle updated" on the cell.

Related PICS/PIXIT statement(s)

None

Foreseen final state of the MS

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on the cell.

Test Procedure

The MS is paged. After the MS has sent a PAGING RESPONSE message to the SS, the SS initiates an authentication procedure and checks the value SRES sent by the MS in the AUTHENTICATION RESPONSE message. The channel is released. The MS is paged and the SS checks the value of the ciphering key sequence number sent by the MS in the PAGING RESPONSE message.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to paging.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE	CKSN = CKSN1
	5	SS -> MS	AUTHENT REQUEST	The SS initiates authentication with CKSN2 
				different from CKSN1
	6	MS -> SS	AUTHENT RESPONSE	"Auth. parameter SRES" IE shall be bit exact with 
				the value as produced by the authentication
				algorithm.
	7	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits for 
				the disconnection of the main signalling link. The 
				SS waits an amount of time which is enough to 
				guarantee that the MS is in service.

8	SS -> MS	PAGING REQUEST TYPE 1
	9	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to paging.
	10	SS -> MS	IMMEDIATE ASSIGNMENT	
	11	MS -> SS	PAGING RESPONSE	"Ciphering key sequence number" shall be the 
				same as the value that was sent in the last 
				AUTHENTICATION REQUEST message 
				(= CKSN2).
	12	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits for 
				the disconnection of the main signalling link.

Specific message contents:

None.

#### 26.7.2.2	Authentication rejected

26.7.2.2.1	Conformance requirement

1)	After reception of an Authentication Reject message the Mobile Station shall:

1.1	not perform normal location updating

1.2	not perform periodic location updating

1.3	not respond to paging with TMSI

1.4	reject any request from CM entity for MM connection except for emergency call

1.5	not perform IMSI detach if deactivated

2)	After reception of an Authentication Reject message the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a CHANNEL REQUEST message with the establishment cause set to "emergency call" and include an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	After reception of an Authentication Reject message the Mobile Station shall delete the stored LAI, CKSN and TMSI.

Reference(s)

GSM 04.08 sections 4.3.2.5.

26.7.2.2.2	Test purpose

1)	To check that ,after reception of an Authentication Reject message, the Mobile Station :

1.1	does not perform normal location updating

1.2	does not perform periodic location updating

1.3	does not respond to paging with TMSI

1.4	rejects any request from CM entity for MM connection except for emergency call

1.5	does not perform IMSI detach if deactivated

2)	To check that, after reception of an Authentication Reject message the Mobile Station, if it supports speech, accepts a request for an emergency call by sending a CHANNEL REQUEST message with the establishment cause set to "emergency call" and includes an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	To check that, after reception of an Authentication Reject message and after having been deactivated and reactivated, the MS performs location updating using its IMSI as mobile identity and indicates deleted LAI and CKSN.

26.7.2.2.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has valid TMSI, CKSN (CKSN2) and Kc. It is "idle updated" on cell B.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

Foreseen final state of the MS

The MS has valid TMSI, CKSN (CKSN1) and Kc. It is "idle updated" on cell A.

Test procedure

The SS rejects an authentication. The channel is released. The SS checks that the MS has entered the state MM-IDLE substate NO IMSI, i.e. does not perform normal location updating, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if powered down.

Maximum duration of test

10 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	SS -> MS	PAGING REQUEST TYPE 1
	2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to paging.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE	"Ciphering key sequence number" shall be the 
				same as the value that was sent in the last 
				AUTHENTICATION REQUEST message 
				(= CKSN2).
	5	SS -> MS	AUTHENT REQUEST
	6	MS -> SS	AUTHENT RESPONSE
	7	SS -> MS	AUTHENT REJECT
	8	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits for 
				the disconnection of the main signalling link.

9	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell B. "Mobile identity" IE 
				contains TMSI.
	10	MS		The MS shall ignore this message. This is verified 
				during 3 seconds.

11	SS		The SS waits for at least for 15 s
	12	MS		A MO CM connection is attempted.
	13	MS		The MS shall not initiate an RR connection 
				establishment on cell A or cell B. This is checked 
				during 3 seconds

14	MS		If the MS supports speech (see PICS),
				an emergency call is attempted.
	15	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Emergency call.
	16	SS -> MS	IMMEDIATE ASSIGNMENT	
	17	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency call establishment. 
				"Mobile identity" : type of identity is set to IMEI
	18	SS -> MS	CM SERVICE ACCEPT	
	19	MS -> SS	EMERGENCY SETUP
	20	SS -> MS	RELEASE COMPLETE	"Cause” = unassigned number.
	21	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits for 
				the disconnection of the main signalling link.

The following messages are sent and shall be received on cell A

22	SS		The RF levels are changed to make the MS 
				reselect the cell A.
	23	MS		The MS performs cell reselection according to 
				procedure as specified in GSM 05.08 (this 
				however is not checked until step 29). The MS 
				shall not initiate an RR connection establishment 
				on cell A or on cell B.

24	SS		The SS waits at least 7 minutes for a possible 
				periodic updating.
	25 	MS		The MS shall not initiate an RR connection 
				establishment on cell A or on cell B.

26	MS		If possible (see PICS) SIM detachment is 
				performed. Otherwise if possible (see PICS) switch 
				off is performed. Otherwise the power is removed.
	27	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

28	MS		Depending on what has been performed in step 26 
				the MS is brought back to operation.
	29	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating.
	30	SS -> MS	IMMEDIATE ASSIGNMENT	
	31	MS -> SS	LOCATION UPDATING REQ	"location updating type” = normal,
				"CKSN” = no key available,
				"Mobile Identity” = IMSI,
				"LAI” = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
	32	SS -> MS	AUTHENT REQUEST	"CKSN” = CKSN1
	33	MS -> SS	AUTHENT RESPONSE	
	34	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity” = TMSI
	35	MS -> SS	TMSI REALLOCATION CMP
	36	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits for 
				the disconnection of the main signalling link.

Specific message contents

None.

### 26.7.3	Identification

The purpose of this procedure is to check that the MS gives its identity as requested by the network. If this procedure does not work, it will not be possible for the network to rely on the identity claimed by the MS.

#### 26.7.3.1	General Identification

26.7.3.1.1	Conformance requirement

1)	When requested by the network the Mobile Station shall send its IMSI.

2)	When requested by the network the Mobile Station shall send the TMSI which it was previously allocated.

3)	When requested by the network the Mobile Station shall send its IMEI as stored in the Mobile Equipment.

4)	When requested by the network the Mobile Station shall send its IMEISV as stored in the Mobile Equipment.

Reference(s)

GSM 04.08 sections 4.3.3.

26.7.3.1.2	Test purpose

1)	To verify that the MS sends identity information as requested by the system in the following cases: IMSI and TMSI are requested in non-ciphered mode, IMEI is requested in ciphered mode.

2)	To verify that the MS sends its IMEI, when requested to do so, in non-ciphered mode.

3)	To verify that the MS sends its IMEISV, when requested to do so, in non-ciphered mode.

26.7.3.1.3	Method of test

26.7.3.1.3.1	Identification / test 1

Initial conditions

System Simulator:

1 cell, default values

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on the cell.

Related PICS/PIXIT statement(s)

IMEI of the ME

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on the cell.

Test Procedure

The SS requests identity information from the MS :

-	IMSI in non ciphering mode,

-	allocated TMSI in non ciphering mode,

-	IMEI in ciphering mode.

Maximum duration of test

30 seconds

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to paging.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMSI.
	6	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies the IMSI of the MS.

7	SS -> MS	IDENTITY REQUEST	"Identity type" IE is TMSI.
	8	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies the allocated TMSI 
				of the MS.

9	SS -> MS	CIPH MODE COMMAND
	10	MS -> SS	CIPH MODE COMPLETE
	11	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEI.
	12	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies the IMEI stored in 
				the Mobile Equipment. 
	13	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits for 
				the disconnection of the main signalling link.

Specific message contents:

None.

26.7.3.1.3.2	Identification / test 2

Initial conditions

System Simulator:

1 cell, default values

Mobile Station:

The MS has a valid TMSI. It is in "idle updated".

Related PICS/PIXIT statement(s)

IMEI of the ME

IMEISV of the ME

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

The SS requests identity information from the MS :

-	IMEI in non ciphering mode.

-	IMEISV in non ciphering mode.

Maximum duration of test

30 seconds

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to
				paging.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEI.
	6	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the IMEI of the MS.
	7	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEISV
	8	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies
				the IMEISV of the MS.

9	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

#### 26.7.3.2	Handling of IMSI shorter than the maximum length

26.7.3.2.1	Conformance requirement

The MS shall be capable of handling an IMSI that is not of the maximum length.

Reference(s)

GSM 04.08 clause 10.5.1.4

26.7.3.2.2	Test purpose

To check that the MS behaves correctly when activated with an IMSI of length less than the maximum length.

In this condition, the MS shall:

-	perform location updating

-	answer to paging with IMSI

-	give the correct IMSI when asked by an IDENTITY REQUEST

-	attempt CM connection establishment when requested to

-	attempt call re-establishment when needed

-	attempt IMSI detach when needed

-	erase its TMSI when the IMSI is sent by the network in a LOCATION UPDATING ACCEPT or a TMSI REALLOCATION COMMAND message.

26.7.3.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default values

IMSI attach/detach bit set to ‘1’

Mobile Station:

The MS has no valid TMSI.

It is "idle updated".

The IMSI has the value 001011234.

Related PICS/PIXIT statement(s)

On/Off switch - Yes/No

Foreseen final state of MS

The MS has no valid TMSI. It is in "idle, updated".

Test Procedure

The MS is paged with its IMSI. The MS shall answer to paging and include the correct IMSI in the PAGING RESPONSE message. During call establishment, the SS asks for the IMSI of the MS. The MS shall answer by an IDENTITY RESPONSE message including the correct IMSI. During the active phase of the call, the SS stops sending valid SACCH frames. The MS performs call re-establishment. The MS shall include the correct IMSI in the CM RE-ESTABLISHMENT message. a TMSI REALLOCATION COMMAND including a TMSI is sent to the MS. The MS acknowledges this message. The call is release.

The MS is paged with its TMSI. The MS shall answer to paging and includes its TMSI in the PAGING RESPONSE message. During call establishment, the SS sends a TMSI REALLOCATION COMMAND including the IMSI to the MS. The MS shall acknowledge this message. The MS shall erase its TMSI. The call is released.

The MS is switched off or powered down. The MS performs IMSI detach. The MS shall include the correct IMSI in the IMSI DETACH message.

The MS is switched on or powered on. The MS performs IMSI attach. The MS shall include the correct IMSI in the LOCATION UPDATING REQUEST message. A TMSI is allocated to the MS.

The LAC of the cell is changed. The MS performs location updating. The SS includes the IMSI in the LOCATION UPDATING ACCEPT message.

A mobile originated CM connection is attempted. The MS shall include the correct IMSI in the CM SERVICE REQUEST message.

Maximum duration of test

5 minutes.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	"mobile identity 1" contains IMSI of MS
	2	MS -> SS	CHANNEL REQUEST	Establishment cause: Answer to paging
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE	"mobile identity" contains the IMSI of the MS
	5	SS -> MS	IDENTITY REQUEST	"identity type" IE is IMSI
	6	MS -> SS	IDENTITY RESPONSE	"mobile identity" IE contains the IMSI of the MS
	7			The call is established using the sequence of the
				generic terminating call set-up procedure.

8	SS		The SS stops sending valid SACCH frames
	9	MS -> SS	CHANNEL REQUEST
	10	SS -> MS	IMMEDIATE ASSIGNMENT
	11	MS -> SS	CM RE-ESTAB REQUEST	"mobile identity" IE contains IMSI of the MS
	12	SS -> MS	TMSI REALLOC COMMAND	"mobile identity" contains a TMSI
	13	MS -> SS	TMSI REALLOC COMPLETE

14	SS -> MS	CHANNEL RELEASE	After sending this message, the SS waits for the
				disconnection of the main signalling link.

15	SS -> MS	PAGING REQUEST TYPE 1	"mobile identity 1" contains TMSI of MS
	16	MS -> SS	CHANNEL REQUEST	Establishment cause: Answer to paging
	17	SS -> MS	IMMEDIATE ASSIGNMENT
	18	MS -> SS	PAGING RESPONSE	"mobile identity" contains the TMSI of the MS
	19	SS -> MS	AUTHENTICATION REQUEST
	20	MS -> SS	AUTHENTICATION RESPONSE
	21	SS -> MS	TMSI REALLOC COMMAND	"mobile identity" contains a IMSI of MS
	22	MS -> SS	TMSI REALLOC COMPLETE
	23	SS -> MS	CHANNEL RELEASE

24	MS		If possible (see PICS) the MS is switched off,
				otherwise the MS is powered down.
	25	MS -> SS	CHANNEL REQUEST	If the MS was switched off it
				performs IMSI detach
	26	SS -> MS	IMMEDIATE ASSIGNMENT
	27	MS -> SS	IMSI DETACH	"mobile identity" contains IMSI of MS
	28	SS -> MS	CHANNEL RELEASE

29	MS		The MS is activated
	30	MS -> SS	CHANNEL REQUEST
	31	SS -> MS	IMMEDIATE ASSIGNMENT
	32	MS -> SS	LOCATION UPDATING REQ	"mobile identity" contains IMSI of MS
	33	SS -> MS	LOCATION UPDATING ACC	"mobile identity" contains a TMSI
	34	MS -> SS	TMSI REALLOC COMPLETE
	35	SS -> MS	CHANNEL RELEASE

36	SS		The SS changes the LAC of the cell
	37	SS		The SS waits for at least 30 seconds
	38	MS -> SS	CHANNEL REQUEST
	39	SS -> MS	IMMEDIATE ASSIGNMENT
	40	MS -> SS	LOCATION UPDATING REQ	"mobile identity" contains TMSI of the MS
	41	SS -> MS	LOCATION UPDATING ACC	"mobile identity" contains IMSI of the MS
	42	SS -> MS	CHANNEL RELEASE

43	MS		a mobile originated CM connection is attempted
	44	MS -> SS	CHANNEL REQUEST
	45	SS -> MS	IMMEDIATE ASSIGNMENT
	46	MS -> SS	CM SERVICE REQUEST	"mobile identity" contains IMSI of the MS
	47	SS -> MS	CHANNEL RELEASE

Specific message contents

None

### 26.7.4	Location updating

This procedure is used to register the MS in the network. If it is not performed correctly, no call can be established.

#### 26.7.4.1	Location updating / accepted

26.7.4.1.1	Conformance requirement

1.

1.1	If the network accepts a location updating from the Mobile Station and reallocates a TMSI in the Location Updating Accept message the Mobile Station shall acknowledge the reception of the new TMSI.

1.2	The Mobile Station shall answer to paging with this TMSI and include it in a Paging Response message.

2	If the network accepts a location updating from the Mobile Station and the Location Updating Accept message contains neither TMSI nor IMSI, the Mobile Station shall answer to paging when addressed with the last allocated TMSI and include it in the Paging Response message.

3.

3.1	If the network accepts a location updating from the Mobile Station by use of a Location Updating Accept message containing the IMSI of the Mobile Station, the Mobile Station shall not answer paging with the last allocated TMSI.

3.2	The Mobile Station shall still answer paging with IMSI.

Reference(s)

GSM 04.08 section 4.4.4.6.

26.7.4.1.2	Test purpose

To test the behaviour of the MS if the network accepts the location updating of the MS.

For the network response three different cases are identified:

1)	TMSI is allocated,

2)	Location updating accept contains neither TMSI nor IMSI,

3)	Location updating accept contains IMSI.

26.7.4.1.3	Method of test

Initial conditions:

System Simulator:

Two cells, A and B, belonging to different location areas with location area identification a and b of the same PLMN.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has a valid TMSI (=TMSI1) and CKSN (=CKSN1). It is "idle updated" on cell A.

Related PICS/PIXIT statement(s)

None.

Foreseen final state of the MS

The MS has no valid TMSI. It has valid CKSN and Kc. It is "idle, updated" on cell B.

Test Procedure

The MS is made to select cell B. A normal location updating with TMSI reallocation is performed in cell B. The channel is released. The SS checks, by paging, that the MS has stored the newly allocated TMSI. The channel is released. The MS is made to select cell A. A normal location updating is performed in cell A. The LOCATION UPDATING ACCEPT message contains neither IMSI nor TMSI. The SS checks, by paging, that the MS has kept the old TMSI. The channel is released. The MS is made to select cell B. A normal location updating is performed in cell B. The LOCATION UPDATING ACCEPT message contains an IMSI. The SS checks, by paging, that the MS has deleted its TMSI and responds to paging with IMSI.

Maximum duration of test

4 minutes

Expected sequence

Step	Direction	Message	Comments

1	SS		The RF level of cell A is
				lowered until the MS selects cell B.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	"location updating type” = normal,
				"CKSN” = CKSN1, "location area
				identification” = a, "mobile station 
				classmark 1" as given by the PICS 
				and "mobile identity” = TMSI1.
	5	SS -> MS	LOCATION UPDATING ACC	"Mobile identity” = new TMSI 
				(=TMSI2), LAI = b.
	6	MS -> SS	TMSI REALLOCATION CMP	
	7	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link. The SS waits an 
				amount of time which is enough to 
				guarantee that the MS is in service.

8	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the new TMSI (= TMSI2).
	9	MS -> SS	CHANNEL REQUEST	
	10	SS -> MS	IMMEDIATE ASSIGNMENT	
	11	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the new TMSI (= TMSI2).
	12	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main signalling link.

13	SS		The RF level of cell B is
				lowered until the MS selects
				cell A.
	14	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	15	SS -> MS	IMMEDIATE ASSIGNMENT	
	16	MS -> SS	LOCATION UPDATING REQ	"location updating type” = normal,
				"CKSN” = CKSN1, "location area
				identification” = b, "mobile station 
				classmark 1" as given by the PICS 
				and "mobile identity” = TMSI2.
	17	SS -> MS	LOCATION UPDATING ACC	"Mobile identity" IE not included
	18	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link. The SS waits an 
				amount of time which is enough to 
				guarantee that the MS is in 
				service.

19	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the TMSI (= TMSI2).
	20	MS -> SS	CHANNEL REQUEST	
	21	SS -> MS	IMMEDIATE ASSIGNMENT	
	22	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the TMSI (=TMSI2).
	23	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

24	SS		The RF level of cell A is
				lowered until the MS selects
				cell B.
	25	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	26	SS -> MS	IMMEDIATE ASSIGNMENT	
	27	MS -> SS	LOCATION UPDATING REQ	"location updating type” = normal,
				"CKSN” = CKSN1, "location area
				identification” = a, "mobile station 
				classmark 1" as given by the PICS 
				and "mobile identity” = TMSI2.
	28	SS -> MS	LOCATION UPDATING ACC	"Mobile identity" IE contains
				IMSI.
	29	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
				the SS waits for the disconnection 
				of the main signalling link. The SS
				waits an amount of time which is
				enough to guarantee that the MS is
				in service.

30	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the old TMSI (= TMSI2).
	31	MS		The MS shall ignore this
				message. This is checked during 5 
				seconds.

32	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				the IMSI.
	33	MS -> SS	CHANNEL REQUEST	
	34	SS -> MS	IMMEDIATE ASSIGNMENT	
	35	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains
				the IMSI.
	36	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

#### 26.7.4.2	Location updating / rejected

##### 26.7.4.2.1	Location updating / rejected / IMSI invalid

26.7.4.2.1.1	Conformance requirement

1)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station shall:

1.1	not perform normal location updating

1.2	not perform periodic location updating

1.3	not respond to paging with IMSI

1.4	not respond to paging with TMSI

1.5	reject any request from CM entity for MM connection other than for emergency call

1.6	not perform IMSI detach if powered down

2)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a Channel Request message with the establishment cause set to "emergency call" and include an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station shall delete the stored LAI, CKSN and TMSI.

Reference(s)

GSM 04.08 sections 4.4.4.7.

26.7.4.2.1.2	Test purpose

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "IMSI unknown in HLR", "illegal MS" or "Illegal ME".

26.7.4.2.1.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas of the same PLMN.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support for speech Yes/No

Foreseen final state of the MS

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

Test Procedure

The SS rejects a normal location updating with the cause value "IMSI unknown in HLR". The channel is released. The SS checks that the MS has entered the state MM-IDLE and the substate NO IMSI, i.e. does not perform normal location updating when a new cell of the same or another PLMN is entered, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if powered down.

The test is repeated with cause value "Illegal MS" and with cause value "Illegal ME".

Maximum duration of test

25 minutes

Expected sequence

The sequence is executed for execution counter k = 1, 2, 3

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B.

	1	SS		The RF level of cell A is
				lowered until the MS selects
				cell B.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "IMSI
				unknown in HLR" for k = 1,
				"Illegal MS" for k = 2,
				"Illegal ME" for k = 3.
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell A.
	7	SS		The RF levels are then changed
				again to make the MS reselect
				the cell A.
	8	MS		The MS performs cell
				reselection according to
				procedure as specified in GSM
				05.08 (this however is not checked 
				until step 18). The MS shall not initiate
				an RR connection establishment on cell A
				or on cell B.

9	SS		The SS waits at least 7 minutes
				for a possible periodic
				updating.
	10 	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B

11	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell A.
				"Mobile identity" IE contains
				IMSI.
	12	MS		The MS shall ignore this
				message. This is verified
				during 3 seconds.

13	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell A.
				"Mobile identity" IE contains
				TMSI.
	14	MS		The MS shall ignore this
				message. This is verified
				during 3 seconds.

15	MS		A MO CM connection is attempted.
	16	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds

17	MS		An emergency call is attempted.
	18	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Emergency call. This message is 
				sent in cell A
	19	SS -> MS	IMMEDIATE ASSIGNMENT	
	20	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency
				call establishment. "Mobile identity" : type
				of identity is set to IMEI
	21	SS -> MS	CM SERVICE ACCEPT	
	22	MS -> SS	EMERGENCY SETUP
	23	SS -> MS	RELEASE COMPLETE	"Cause” = unassigned number.
	24	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

25	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	26	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

27	MS		Depending on what has been 
				performed in step 25 the MS is 
				brought back to operation.
	28	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	29	SS -> MS	IMMEDIATE ASSIGNMENT	
	30	MS -> SS	LOCATION UPDATING REQ	"location updating type” = normal,
				"CKSN” = no key available, "mobile 
				station classmark 1" as given by 
				the PICS, "Mobile Identity” = IMSI, 
				"LAI” = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
	31	SS -> MS	AUTHENT REQUEST	"CKSN” = CKSN1
	32	MS -> SS	AUTHENT RESPONSE	
	33	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity” = TMSI
	32	MS -> SS	TMSI REALLOCATION CMP
	33	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

##### 26.7.4.2.2	Location updating / rejected / PLMN not allowed

26.7.4.2.2.1	Conformance requirement

1)	If the network reject a location updating from the Mobile Station with the cause "PLMN not allowed" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not perform IMSI detach when switched off,

1.3	not perform IMSI attach when switched on in the same location area,

1.4	not perform normal location updating when in the same PLMN and when that PLMN is not selected manually.

1.5	reject any request from CM entity for MM connection other than for emergency call

2)	If the network rejects a location updating from the Mobile Station with the cause "PLMN not allowed" the Mobile Station shall:

2.1	perform normal location updating when a new PLMN is entered

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

3)	If the network rejects a location updating from the Mobile Station with the cause "PLMN not allowed" and if after that the PLMN from which this rejection was received, is manually selected, the Mobile Station shall perform a normal location updating procedure.

Reference(s)

GSM 04.08 section 4.4.4.7

26.7.4.2.2.2	Test purpose

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "PLMN not allowed".

26.7.4.2.2.3	Method of test

26.7.4.2.2.3.1	Location updating / rejected / PLMN not allowed / test 1

Initial conditions

System Simulator:

One cell: C, belonging to PLMN1.

Two cells : A and B, belonging to different location areas a and b and belonging to PLMN2. PLMN2 is different from HPLMN and from PLMN1.

IMSI attach/detach is allowed in cells A and B but not in cell C.

The T3212 timeout value is 1/10 hour in cells A and B.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell C.

The MS is in manual mode for PLMN selection.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

The MS is automatically in automatic mode after switch on Yes/No

Support for speech Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell C. The MS is in automatic mode for PLMN selection.

Test Procedure

The SS rejects a normal location updating with the cause value "PLMN not allowed". The channel is released. The SS checks that the MS does not perform periodic updating, does not perform IMSI detach, does not perform IMSI attach if activated in the same location area, rejects any request for CM connection establishment other than emergency call, accepts a request for an emergency call and performs normal location updating only when a new PLMN is entered.

Maximum duration of test

12 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B.

	1	MS		The MS is switched off (or power is removed).
	2	SS		The SS activates cells A and B and

deactivates cell C.
				Cell B has a level higher by
				at least 5 dB than cell A
	3	MS		The MS is switched on. (or power is reapplied)
				If necessary the MS is put in manual selection
				mode. The MS shall
				offer the new PLMN as available to 
				the user. The PLMN is manually selected.
	4	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	LOCATION UPDATING REQ	
	7	SS -> MS	LOCATION UPDATING REJ	"Reject cause” = PLMN not allowed.
	8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

9	SS		The SS waits for a possible
				periodic updating for 7 minutes.
	10	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

11	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	12	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

13	MS		Depending on what has been 
				performed in step 11 the MS is 
				brought back to operation. The MS is
				not made to select PLMN 2.
	14	MS		The MS shall not initiate an RR
				connection establishment. This is checked
				during 3 seconds.

The following message are sent and shall be
				received on cell A.

	15	SS		The RF level of cell B is
				lowered to make the MS reselect cell A.
	16	MS		No access to the network shall
				be registered by the SS within one minute.

17	MS		An emergency call is attempted.
	18	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Emergency call
	19	SS -> MS	IMMEDIATE ASSIGNMENT	
	20	MS -> SS	CM SERVICE REQUEST	"CM service type” = Emergency call
				establishment.
	21	SS -> MS	CM SERVICE ACCEPT	
	22	MS -> SS	EMERGENCY SETUP
	23	SS -> MS	RELEASE COMPLETE	Cause IE : "unassigned number".
	24	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

25	MS		A MO CM connection is attempted.
	26	MS		The MS shall not initiate an RR connection
				establishment. This is checked
				during 3 seconds

The following messages are sent and shall be
				received on cell C

	27	MS		The MS is switched off.
	28	SS		The SS activates cell C
				and deactivates cells A and B
	29	MS		The MS is switched on. If necessary
				the MS is placed into the automatic mode.
	30	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	31	SS -> MS	IMMEDIATE ASSIGNMENT	
	32	MS -> SS	LOCATION UPDATING REQ	"location updating type” = normal,
				"CKSN” = no key available,
				"LAI” = deleted LAI (the MCC and MNC hold
				the values of PLMN2, the LAC is coded FFFE)
				"mobile identity” = IMSI
	33	SS -> MS	LOCATION UPDATING ACC	"Mobile identity” = TMSI.
	34	MS -> SS	TMSI REALLOCATION CMP	
	35	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

26.7.4.2.2.3.2	Location updating / rejected / PLMN not allowed / test 2

Initial conditions

System Simulator:

One cell C, belonging to PLMN1.

Two cells A and B, belonging to different location areas a and b and belonging to PLMN2. PLMN2 is different from HPLMN.

IMSI attach/detach is allowed in cells A and B but not in cell C.

The T3212 timeout value is 1/10 hour in cells A and B.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell C.

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

The MS is automatically in automatic mode after switch on Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle, updated” on cell C.

The MS is in automatic mode for PLMN selection.

Test Procedure

The SS rejects a normal location updating with the cause value "PLMN not allowed". The channel is released. Then the PLMN from which this rejection was received is manually selected and the SS checks that a normal location updating is performed.

Maximum duration of test

5 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B
	1	MS		The MS is switched off (or power is removed).
	2	SS		The SS activates cells A and B and deactivates 
				cell C. Cell B has a level higher by at least 5 dB 
				than cell A
	3	MS		The MS is switched on (or power is reapplied).
	3a	MS		If the MS is in manual mode, it shall 
				offer the new PLMN as available to 
				the user. In this case the PLMN is manually 
				selected.
	4	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	LOCATION UPDATING REQ	
	7	SS -> MS	LOCATION UPDATING REJ	"Reject cause” = PLMN not
				allowed.
	8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

9	MS		The MS is made to search for
				PLMNs and the PLMN indicated 
				by the SS is manually selected
	10	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	11	SS -> MS	IMMEDIATE ASSIGNMENT	
	12	MS -> SS	LOCATION UPDATING REQ	"location updating type” = normal,
				"CKSN” = no key available,
				"LAI” = deleted LAI (the MCC and MNC hold
				the values of PLMN2, the LAC is coded FFFE)
				"mobile identity” = IMSI

	13	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell C.

	14	MS		The MS is switched off.
	15	SS		The SS activates cell C
				and deactivates cells A and B
	16	MS		The MS is switched on.
				If necessary, the MS is put
				into the automatic mode.
	17	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	18	SS -> MS	IMMEDIATE ASSIGNMENT	
	19	MS -> SS	LOCATION UPDATING REQ	"location updating type” = normal,
				"CKSN” = no key available,
				"LAI” = deleted LAI (the MCC and MNC hold
				the values of PLMN2, the LAC is coded FFFE)
				"mobile identity” = IMSI
	20	SS -> MS	LOCATION UPDATING ACC	"Mobile identity” = TMSI.
	21	MS -> SS	TMSI REALLOCATION CMP	
	22	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

##### 26.7.4.2.3	Location updating / rejected / location area not allowed

26.7.4.2.3.1	Conformance requirement

1)	If the network rejects a location updating from the Mobile Station with the cause "Location Area not allowed" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not respond to paging with TMSI,

1.3	reject any request from CM entity for MM connection other than for emergency call,

1.4	not perform IMSI detach.

2)	If the network rejects a location updating from the Mobile Station with the cause "Location Area not allowed" the Mobile Station shall:

2.1	perform normal location updating when a new location area is entered,

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

2.3	delete the list of forbidden LAs after switch off (power off).

Reference(s)

GSM 04.08 sections 4.4.4.7

26.7.4.2.3.2	Test purpose

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "Location Area not allowed".

To test that the MS deletes the list of forbidden LAs after switch off (power off).

26.7.4.2.3.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell A.

Related PICS/PIXIT statement(s)

Switch off on button Yes/No

Method to clear the list of forbidden location areas periodically.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell A.

Test Procedure

The SS rejects a normal location updating with the cause value "Location Area not allowed". The channel is released. The SS checks that the MS does not perform periodic updating, does not respond to paging with TMSI, rejects any requests from CM entities for MM-connections except emergency calls, does not perform IMSI detach, performs normal location updating when a new location area is entered and deletes the list of forbidden LAs when switched off.

Different types of MS may use different methods to periodically clear the list of forbidden location areas (e.g. every day at 12am). If the list is cleared while the test is being run, it may be necessary to re-run the test.

Maximum duration of test

12 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell B

	1	SS		The RF level of cell A is
				lowered so that cell B is selected, while keeping
				the C1 and C2 of cell A greater than 10.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause” = "Location
				Area not allowed".
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

7	SS		The SS waits for a possible
				location updating for 7 minutes.
	8	MS		The MS shall not initiate an
				RR-connection establishment either on
				cell A or cell B.

9	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell B.
				"Mobile identity” = TMSI.
	10	MS		The MS shall ignore this message.
				This is checked during 3 seconds.

11	MS		A MO CM connection is attempted.
	12	MS		The MS shall not initiate an RR 	connection
				establishment on cell A or cell B.
				This is checked during 3 seconds

13	MS		An emergency call is attempted.
	14	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Emergency call.
	15	SS -> MS	IMMEDIATE ASSIGNMENT	
	16	MS -> SS	CM SERVICE REQUEST	"CM service type":
				Emergency call establishment.
	17	SS -> MS	CM SERVICE ACCEPT	
	18	MS -> SS	EMERGENCY SETUP
	19	SS -> MS	RELEASE COMPLETE	Cause: "unassigned number".
	20	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

21	MS		If possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	22	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B (check
				for IMSI detach)
				This is checked during 3 seconds.

23	MS		Depending on what has been 
				performed in step 21 the MS is 
				brought back to operation.
	24	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	25	SS -> MS	IMMEDIATE ASSIGNMENT	
	26	MS -> SS	LOCATION UPDATING REQ	"location updating type” = normal,
				"CKSN” = no key available,
				"LAI” = deleted LAI, "mobile 
				identity” = IMSI (This checks the
				deletion of the forbidden lists)
 	27	SS -> MS	LOCATION UPDATING REJ	"Reject cause” = "Location
				Area not allowed".
	28	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell A

	29	SS		The RF level of cell B is
				lowered until the MS selects
				cell A.
	30	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	31	SS -> MS	IMMEDIATE ASSIGNMENT	
	32	MS -> SS	LOCATION UPDATING REQ	
	33	SS -> MS	AUTHENT REQUEST	
	34	MS -> SS	AUTHENT RESPONSE
	35	SS -> MS	LOCATION UPDATING ACC	Mobile identity = TMSI
	36	MS -> SS	TMSI REALLOCATION CMP
	37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents:

None.

##### 26.7.4.2.4	Location updating / rejected / national roaming

26.7.4.2.4.1	Conformance requirement

1)	If the network rejects a location updating from the Mobile Station with the cause "National roaming not allowed in this area" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not respond to paging with TMSI,

1.3	reject any request from CM entity for MM connection other than for emergency call,

1.4	not perform IMSI detach.

2)	If the network rejects a location updating from the Mobile Station with the cause "National roaming not allowed in this area" the Mobile Station shall:

2.1	perform normal location updating when a new location area is entered,

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

2.3	periodically search for its HPLMN.

3)	The mobile station shall reset the list of "forbidden location areas for national roaming" when powered down or when the SIM is removed.

4)	The MS shall be capable of storing at least 6 entries in the list of "forbidden location areas for national roaming".

Reference(s)

GSM 04.08 section 4.4.4.7

26.7.4.2.4.2	Test purposes

Test purpose 1

To test that on receipt of a rejection using the National Roaming cause code, the MS ceases trying to update on that cell, that this situation continues for at least one periodic location interval period, and that the corresponding list is re-set by powering down the MS.

Test purpose 2

To test that if no cell is available, the MS does not answer to paging with TMSI, rejects a request from CM entity other than for emergency calls.

Test purpose 3

To test that at least 6 entries can be held in the list of "forbidden location areas for national roaming" (the requirement in TS GSM 04.08 is to store at least 10 entries. This is not fully tested by the third procedure).

Test purpose 4

To test that if a cell of the Home PLMN is available then the MS returns to it in preference to any other available cell.

Test purpose 5

To test that if the SIM is removed the list of "forbidden location areas for national roaming” is cleared.

26.7.4.2.4.3	Method of test

Initial conditions

The initial conditions shall be met before each of the different procedures.

System Simulator:

For procedures 1, 2, 3 and 5: Two cells A and B, belonging to different location areas of the same PLMN with LAI a and b. The MCC of that PLMN is the same as that of the HPLMN. The MNC of that PLMN is different from that of the HPLMN.

For procedure 4: three cells A, B, C of the same PLMN which is not the HPLMN with 3 different location area codes. Cells should differ in signal strength by 10 dB with cell A being the strongest and cell C the weakest. There should be a 20 dB range between A and C. A should be set to a level of - 40 dBm.

IMSI attach/detach is allowed in every cell.

The T3212 timeout value is 1/10 hour in every cell.

Mobile Station:

Procedures 1, 2, 3 and 5 : The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell B.

Procedure 4 : The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

The list of "forbidden location areas for national roaming" shall be empty (this may be achieved by either removing the SIM or switching the MS OFF then ON or powering down the MS depending on PICS).

Related PICS/PIXIT statement(s)

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

Method to clear the list of location areas for national roaming periodically.

The MS is automatically in automatic mode after switch on Yes/No

Foreseen final state of the MS

Procedures 1 and 5 : The MS has no valid TMSI and no CKSN. It is "idle updated" on cell A.

Procedure 2 and 3: The MS has no valid TMSI and no CKSN. It is in the 'limited service' state on cell A.

Procedure 4 : The MS has no valid TMSI and no CKSN. It is "idle updated" on cell C.

Test Procedures

Procedure 1 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not perform periodic location updating procedure. The MS is turned off and then on. The SS checks that the MS performs location updating on the cell on which its location update request had been rejected (this checks that the LA is not the forbidden list after switch on). This procedure is performed another time but the deletion of the list is checked while removing the SIM (instead of turning off the MS).

Procedure 2 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not answer to a paging message with TMSI, rejects a request from CM entity but supports an emergency call.

Procedure 3 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". This is done for 6 different location areas. Then the SS checks that the MS does not attempt to begin a location updating procedure on the non-allowed location areas.

Procedure 4 :

The SS accepts a normal location updating on a cell not belonging to the HPLMN. Then when the MS attempts to perform a periodic location updating to this cell, the SS rejects this location updating with the cause value "National Roaming not allowed in this area". Two cells are then available, one belonging to the HPLMN but with the weakest level. It is checked that the MS returns to its HPLMN.

Procedure 5 : If SIM removal is possible without powering down :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not perform periodic location updating procedure. The SIM is removed and inserted in the MS. The SS checks that the MS performs location updating on the cell on which its location update request had been rejected (this checks that the LA is not the forbidden list after switch on).

Different types of MS may use different methods to periodically clear the list of forbidden areas (e.g. every day at 12am) for national roaming. If the list is cleared while the test is being run, it may be necessary to re-run the test.

Maximum duration of test

Procedures 1 and 5: 12 minutes each

Procedure 2: 6 minutes

Procedure 3: 17 minutes

Procedure 4: 16 minutes

Expected sequence

The following procedure is used during the test:

Change_LAI (x):

-	the purpose of this procedure is to change the value of Location Area Identifier of cell x

-	The Location Area Identifier of cell x shall be changed. The code shall be chosen arbitrarily but shall be different from any previously used in this procedure. The code shall have the same MCC as the Home PLMN and shall not have the same MNC as the Home PLMN.

Procedure 1

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A.

	1	SS		The RF level of cell B is
					lowered until cell B is no
				more suitable and the MS selects cell A.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
					Roaming not allowed in this 
					location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

7	SS		The SS waits at least 7 minutes
					for a possible location updating
	8 	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

9	MS		If possible (see PICS) the MS is 
				switched off. Otherwise 
				if possible the power is removed.
	10	MS		Depending on what has been 
				performed in step 9 the MS is 
				brought back to operation and placed
				in a automatic mode.
	11	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	12	SS -> MS	IMMEDIATE ASSIGNMENT	
	13	MS -> SS	LOCATION UPDATING REQ	Location Updating Type = normal
	14	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	15	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main signalling link.

Procedure 2

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The RF level of cell B is lowered until the MS 
				selects cell A. The level of cell B shall be such that 
				cell B is suitable for cell selection.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message is sent on  cell A.
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National"
				Roaming not allowed in this 
				location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell B
	7	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	8	SS -> MS	IMMEDIATE ASSIGNMENT	
	9	MS -> SS	LOCATION UPDATING REQ	
	10	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National  Roaming not 
				allowed in this location area"
	11	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits for 
				the disconnection of the main signalling link.

12	SS		The SS waits for a possible  location updating  
				procedure on both cells A and B for  2 minutes.
	13	MS		The MS shall not initiate an RR connection 
				establishment  on  cell A or on cell B
				within 2 minutes after the end of step 11

14	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI. This message is
				sent on cell A and on cell B
	15	MS		The MS shall not initiate an RR connection on cell 
				A or on cell B. This is checked during 3 seconds.

16	MS		A MO CM connection is attempted.
	17	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

The following messages are sent and shall be
				received on cell A
				Steps 20 to 27 are performed if the MS
				supports speech.
	18	MS		An emergency call is attempted.
	19	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
	20	SS -> MS	IMMEDIATE ASSIGNMENT	
	21	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency call establishment.
	22	SS -> MS	CM SERVICE ACCEPT	
	23	MS -> SS	EMERGENCY SETUP
	24	SS -> MS	RELEASE COMPLETE	"Cause” = unassigned number.
	25	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits for 
				the disconnection of the main signalling link.

Procedure 3

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The RF level of cell B is
				lowered until the MS selects
				cell A. The level of cell B
				shall be such that cell B is
				suitable for cell selection.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : 
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell B

	9	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	10	SS -> MS	IMMEDIATE ASSIGNMENT	
	11	MS -> SS	LOCATION UPDATING REQ	
	12	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	13	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	14	SS		Change_LAI (A) within 5 seconds 
				after step 12

The following messages are sent and shall be
				received on cell A

	17	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	18	SS -> MS	IMMEDIATE ASSIGNMENT	
	19	MS -> SS	LOCATION UPDATING REQ	
	20	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	21	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	22	SS		Change_LAI (B) within 5 seconds 
				after step 20

The following messages are sent and shall be
				received on cell B

	25	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	26	SS -> MS	IMMEDIATE ASSIGNMENT	
	27	MS -> SS	LOCATION UPDATING REQ	
	28	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	29	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	30	SS		Change_LAI (A) within 5 seconds 
				after step 28

The following messages are sent and shall be
				received on cell A

	33	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	34	SS -> MS	IMMEDIATE ASSIGNMENT	
	35	MS -> SS	LOCATION UPDATING REQ	
	36	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	38	SS		Change_LAI (B) within 5 seconds 
				after step 36

The following messages are sent and shall be
				received on cell B

	41	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	42	SS -> MS	IMMEDIATE ASSIGNMENT	
	43	MS -> SS	LOCATION UPDATING REQ	
	44	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
				Roaming not allowed in this 
				location area"
	45	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	46	SS		The SS waits for a possible 
				location updating 
				procedure on both cells A and B for 
				7 minutes.
	47	MS		The MS shall not initiate an RR connection 
				establishment on cell A or on cell B
				within 7 minutes after the end of step 45

Procedure 4

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The SS waits for a 
					periodic location updating 
					procedure on cell A for 7 minutes 
					after the initial conditions have 
					been established.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	Location Updating Type = periodic
	5	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

	7	SS		The location area identity of cell 
					C shall be changed to that of a 
					location area in the Home PLMN

	8	SS		The SS waits for a  
					periodic location updating 
					procedure on cell A for 
					7 minutes.
	9	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
					This message is sent on 
					cell A within 7 minutes after the end
					of step 6.
	10	SS -> MS	IMMEDIATE ASSIGNMENT	
	11	MS -> SS	LOCATION UPDATING REQ	"Location updating type" = periodic
	12	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
					Roaming not allowed in this 
					location area"
	13	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

The following messages are sent and shall be
				received on cell C

	16	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	17	SS -> MS	IMMEDIATE ASSIGNMENT	
	18	MS -> SS	LOCATION UPDATING REQ	
	19	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	20	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

Procedure 5

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	SS		The RF level of cell B is
					lowered until the MS selects cell A.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National 
					Roaming not allowed in this 
					location area"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

7	SS		The SS waits at least 7 minutes
					for a possible location updating
	8 	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

9	MS		The SIM is removed
	10	MS		The SIM is inserted into the ME.
	11	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	12	SS -> MS	IMMEDIATE ASSIGNMENT	
	13	MS -> SS	LOCATION UPDATING REQ	Location Updating Type = normal
	14	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present
	15	SS -> MS	CHANNEL RELEASE	After the sending of this
					message, the SS waits for the
					disconnection of the main
					signalling link.

Specific message contents:

None.

#### 26.7.4.3	Location updating / abnormal cases

##### 26.7.4.3.1	Location updating / abnormal cases / random access fails

26.7.4.3.1.1	Conformance requirement

If during the RR connection establishment phase of a normal location updating procedure, channel requests are not answered by the network, the Mobile Station shall:

1.	send (Max-Retrans+1) Channel Request messages

2.	not try to establish a connection during a period of T3213

3.	then perform a normal location updating procedure as it is still necessary.

4.	not repeat the complete procedure if the original cause of the location updating procedure has disappeared.

Reference(s):

GSM 04.08 section 4.4.4.9 and GSM 05.08 section 6.6.2

26.7.4.3.1.2	Test purpose

To verify that when during the RR connection establishment phase of a location updating procedure, channel requests are not answered by the network, after expiry of T3213 (= 4s in Phase 2) the complete procedure is repeated if still necessary.

26.7.4.3.1.3	Method of test

Initial conditions

System Simulator:

Two cells: A and B of the same PLMN, belonging to different location areas with LAI a and b.

The RF power level of cell B is higher than the one of cell A.

IMSI attach/detach is not allowed in both cells.

The T3212 timeout value is set to infinite in both cells.

Mobile Station:

The MS has a valid TMSI, CKSN and Kc. It is "Idle updated" on cell B.

Related PICS/PIXIT statement(s)

None

Foreseen final state of the MS

The MS is "Idle updated" on cell A.

Test Procedure

The SS causes a random access failure in the MS during a normal location updating procedure. After the expiry of T3213 and when the cell reselection procedure is finished the MS will try to restart the normal location updating procedure.

The test is repeated but the original cause of the location updating procedure has disappeared. The SS then checks that the MS will not restart the location updating procedure.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be
				received on cell A

	1	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	2	MS -> SS	CHANNEL REQUEST	Establishment cause:
					Location updating
				This message is sent by the MS 
				(Max_Retrans + 1) times
	3	SS		The SS waits for 3 seconds
	4	MS		The MS shall not send any layer 3
				message during this time.
	5	MS -> SS	CHANNEL REQUEST	Establishment cause:
					Location updating
				The time difference between this message
				and the last CHANNEL REQUEST sent in
				step 2 shall be in the range 4 s - 6 s.
	6	SS -> MS	IMMEDIATE ASSIGNMENT	
	7	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b,
				mobile station classmark 1 and
				mobile identity = TMSI.
	8	SS -> MS	LOCATION UPDATING ACC	Optional IE Mobile Identity not
				included
	9	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be
				received on cell B

	10	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	11	MS -> SS	CHANNEL REQUEST	Establishment cause:
					Location updating
				This message is sent by the MS 
				(Max_Retrans + 1) times
	12	SS		Immediately after the end of
				step 11 the RF level of cell A is
				set sufficiently high to ensure
				that cell A is suitable as defined
				in GSM 05.08 section 6.6.2
	13	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 6 seconds.

Specific message contents:

None.

##### 26.7.4.3.2	Location updating / abnormal cases / attempt counter less or equal to 4, LAI different

26.7.4.3.2.1	Conformance requirement

1)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure, if the attempt counter is smaller than 4 and after expiry of T3211, the Mobile Station shall resend its Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "normal location updating".

2)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall:

2.1	not answer to paging with the previously allocated TMSI

2.2	not perform the IMSI detach procedure, when switched off.

3)	When a failure such as case e) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure and when an emergency call establishment is requested by the user the Mobile Station, if it supports speech, shall send a CM Service Request message with CM Service Type IE set to "emergency call establishment", CKSN IE set to "no key available" and Mobile Identity IE set to its IMSI and after acceptance by the network it shall send an Emergency Setup message.

4)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall use a request from CM entity other than emergency call as a trigger for a normal location updating procedure and shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

5)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall answer to paging with IMSI and shall send a Paging Response message with CKSN IE set to "no key available" and Mobile Identity IE set to its IMSI.

6)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall perform a normal location updating procedure as soon as it enters a new cell.

References

GSM 04.08 section 4.4.4.2 and 4.4.4.9 and GSM 05.08 section 6.6.2

26.7.4.3.2.2	Test purpose

To verify that the MS performs normal location updating procedures when its attempt counter is smaller than 4.

To check that the MS does not perform the IMSI detach procedure when "idle not updated".

To verify that when "idle not updated" the MS can perform an emergency call.

To verify that when "idle not updated" the MS uses requests from CM layer other than emergency call as triggering of a normal location updating procedure.

To verify that the MS performs a normal location updating procedure if it enters a new cell while being "idle not updated".

26.7.4.3.2.3	Method of test

Initial conditions

System Simulator:

Two cells: A and B of the same PLMN, belonging to different location areas with LAI a and b.

ATT flag shall be set to IMSI attach/detach allowed.

Mobile Station:

The MS is "idle updated" on cell A. A valid CKSN value is stored in the SIM and is noted "initial CKSN". A TMSI is allocated.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Foreseen final state of the MS

The MS is "Idle updated" on cell A with a valid CKSN and a TMSI.

Test Procedure

The MS is made to perform a normal location updating procedure. Four types of failure cases are triggered:

-	sending of a Location Updating Reject with cause randomly chosen between all defined cause values except 2, 3, 6, 11, 12 and 13 (which trigger a different action) (case g of GSM 04.08 section 4.4.4.9),

-	RR-connection failure (case d),

-	sending of a CHANNEL RELEASE message before the normal end of the procedure (case f),

-	T3210 timeout (case e).

As there is no stored LAI or the stored LAI is different from the broadcast LAI, and the attempt counter in the MS shall be lower than 4, the MS enters the state MM-IDLE and substate ATTEMPTING TO UPDATE and waits for T3211 seconds before trying again a location updating procedure.

Then the behaviour of the MS in the MM-IDLE ATTEMPTING TO UPDATE SERVICE state is checked, that is:

-	not answer to paging with TMSI,

-	not perform an IMSI detach procedure,

-	support request for emergency call,

-	use requests from CM layer other than emergency call as triggering of a normal location updating procedure,

-	perform normal location updating procedure when a new cell is entered.

Maximum duration of test

6 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	2	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	5	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value
				arbitrarily chosen:
				* in Table 10.66 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being 
				excluded
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	7	MS		The MS shall not initiate an RR connection 
				establishment on cell A or on cell B
				 during T3211 seconds at least after
				the channel release.
	8	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	9	SS -> MS	IMMEDIATE ASSIGNMENT
	10	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.

	11	SS		The SS stops any RF transmission on 
				the dedicated channel. The SS waits 
				until there are no more SACCH frames 
				in the uplink direction. This release connection is
				done within 8 SACCH blocks and the MS starts
				T3211

	12	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				 during T3211.
	13	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	14	SS -> MS	IMMEDIATE ASSIGNMENT

15	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	16	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

	17	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				 during T3211 seconds at least after
				the channel release.
	18	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	19	SS -> MS	IMMEDIATE ASSIGNMENT
	20	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	21	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	22	MS -> SS	AUTHENT RESPONSE
	23	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	24	MS -> SS	TMSI REALLOCATION CMP
	25	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle updated" in cell B

The following messages are sent and shall be received on cell A

26	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	27	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	28	SS -> MS	IMMEDIATE ASSIGNMENT	
	29	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	30	SS		performs step 5 with reject cause 100
	31	SS -> MS	PAGING REQUEST TYPE 1	Mobile identity = old TMSI of the 
				MS.
				This message is sent continuously to
				the MS during 8 seconds

32	SS		The SS checks that there is no 
				answer from the MS during 12 
				seconds
	33	SS		If during steps 31 and 32 the MS 
				attempts to perform a location 
				updating procedure the SS will 
				perform step 30 and then 
				continue the procedure.
	34	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) mobile switch off 
				is performed. Otherwise the power 
				is removed.
	35	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 30 seconds.
	36	MS		Depending on what has been 
				performed in step 34 the MS is 
				brought back to operation.
	37	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	38	SS -> MS	IMMEDIATE ASSIGNMENT
	39	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	40	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	41	MS -> SS	AUTHENT RESPONSE
	42	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	43	MS -> SS	TMSI REALLOCATION CMP
	44	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle updated" in cell A
	45	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	46	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	47	SS -> MS	IMMEDIATE ASSIGNMENT	
	48	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	49	SS -> MS	AUTHENT REQUEST	
	50	MS->SS	AUTHENT RESPONSE	steps 49 and 50 are performed N 
				times. N shall be chosen in such a 
				way that T3210 expires.
	51	SS		The SS checks that there is no more 
				activity from the MS on the 
				channel.
	52	MS		The MS is made to perform an 
				emergency call

53	MS -> SS	CHANNEL REQUEST	Establishment cause: Emergency call
	54	SS -> MS	IMMEDIATE ASSIGNMENT	
	55	MS -> SS	CM SERVICE REQUEST	CM service type = Emergency call 
				establishment; CKSN = no key 
				available; Mobile Identity = IMSI
	56	SS -> MS	CM SERVICE ACCEPT
	57	MS -> SS	EMERGENCY SETUP
	58	SS -> MS	RELEASE COMPLETE	Cause = unassigned number
	59	SS -> MS	CHANNEL RELEASE
	60	MS -> SS	CHANNEL REQUEST	Establishment cause : Location updating
				The SS will wait at most 15 
				seconds for this message
	61	SS -> MS	IMMEDIATE ASSIGNMENT
	62	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	63	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	64	MS -> SS	AUTHENT RESPONSE
	65	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	66	MS -> SS	TMSI REALLOCATION CMP
	67	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle updated" in cell B

68	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	69	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	70	SS -> MS	IMMEDIATE ASSIGNMENT	
	71	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	72	SS		performs step 11
	73	MS		A MO CM connection is attempted
				before T3211 expiry
	74	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	75	SS -> MS	IMMEDIATE ASSIGNMENT
	76	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	77	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	78	MS -> SS	TMSI REALLOCATION CMP
	79	SS -> MS	CHANNEL RELEASE	
				Steps 80 to 83 are optional as the 
				MS may have memorised the request for
				CM connection attempt
				Wait 10 s to decide whether to go directly to
				step 84.
	80	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Not checked
	81	SS -> MS	IMMEDIATE ASSIGNMENT
	82	MS -> SS	CM SERVICE REQUEST	CKSN = 
				no key available,
				Mobile identity = TMSI
	83	SS -> MS	CHANNEL RELEASE	After the sending of this message,
				the SS waits for the disconnection of
				the main signalling link.
				MS is now "idle updated" in cell A
	84	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	85	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	86	SS -> MS	IMMEDIATE ASSIGNMENT	
	87	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	88	SS		performs step 16 and notes 
				the time when the CHANNEL RELEASE 
				message is sent

89	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	90	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
				The time interval between the last 
				CHANNEL RELEASE message and this 
				message shall be less than 
				0.8*T3211
	91	SS -> MS	IMMEDIATE ASSIGNMENT	
	92	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available , 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	93	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	94	MS -> SS	AUTHENT RESPONSE
	95	SS -> MS	LOCATION UPDATING ACC	Mobile identity = TMSI
	96	MS -> SS	TMSI REALLOCATION CMP
	97	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "Idle, updated" in cell A

Specific message contents

None.

##### 26.7.4.3.3	Location updating / abnormal cases / attempt counter equal to 4

26.7.4.3.3.1	Conformance requirement

1)	When four failures such as cases d) to g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure the Mobile Station shall:

1.1	not perform normal location updating procedures.

1.2	perform periodic location updating after T3212 expiry by sending a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "periodic updating".

1.3	if the periodic location updating was unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

2)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure the Mobile Station, if it supports speech, shall be able to perform an emergency call i.e. the Mobile Station is able to send a CM Service Request message with the CM Service Type IE set to "emergency call establishment", CKSN IE set to "no key is available" and Mobile Identity IE set to its IMSI and then send an Emergency Setup message.

3)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure

3.1	the Mobile Station shall use a request from CM entity for MM connection for a service other than emergency call as a trigger for a normal location updating procedure and shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

3.2	after a location updating triggered by a request from the CM layer which was .unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

4)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure

4.1	the Mobile Station shall perform a normal location updating procedure if it enters a new cell.

4.2	if this location updating is unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

References

GSM 04.08 section 4.4.4.9 and GSM 05.08 section 6.6.2

26.7.4.3.3.2	Test purpose

To verify that the MS performs only periodic location updating procedures, that it does not perform normal location updating procedures when its attempt counter has reached value 4 and that the MS reset its attempt counter after a timer T3212 expiry.

To verify that the MS still follows the MM-IDLE ATTEMPTING TO UPDATE state requirements after its attempt counter has reached value 4.

To verify that the attempt counter is reset in the cases where it has to be done.

26.7.4.3.3.3	Method of test

Initial conditions

System Simulator:

Two cells: A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

T3212 is set to 6 minutes.

Mobile Station:

The MS is "Idle updated" on cell B with a valid CKSN and a TMSI.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

Foreseen final state of the MS

The MS is "Idle updated" on cell A with a valid CKSN and a TMSI.

Test Procedure

The MS is made to perform a normal location updating. The SS triggers a failure in this procedure. After T3211 expiry the MS will try again the location updating procedure. The SS triggers again a failure. This is done again 2 times. At this point the attempt counter shall be equal to 4. It is then checked that T3212 has been started and that at its expiry the MS will try a periodic location updating procedure. It is verified that the MS has reset its attempt counter after timer T3212 expiry.

Then it is checked that, when the attempt counter has reached the value of 4, the MS is in the MM-IDLE state and ATTEMPTING TO UPDATE substate, that is:

-	not perform an IMSI detach procedure,

-	support request for emergency call,

-	use requests from CM layer other than emergency call as triggering of a normal location updating procedure,

-	perform normal location updating procedure when a new cell is entered.

Maximum duration of test

20 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be 
				received on cell A
	1	MS		The RF level of cell B is lowered
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	2	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	5	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to #22
				* in Table 10.66 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being
				excluded
	6	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	7	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B 
				within T3211 + RadioLinkTimeOut
				seconds after the SS stops
				RF transmission
	8	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	9	SS -> MS	IMMEDIATE ASSIGNMENT
	10	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.

	11	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there are no more SACCH in the
				uplink
	12	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at least after the
				channel release
	13	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	14	SS -> MS	IMMEDIATE ASSIGNMENT
	15	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	16	SS -> MS	AUTHENT REQUEST

17	MS -> SS	AUTHENT RESPONSE	these steps (16 and 17) are 
				performed N times.
				N shall be chosen in such a way 
				that T3210 expires.
	18	MS		The MS shall cease transmission and then
				shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at least after the
				expiry of T3210
	19	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	20	SS -> MS	IMMEDIATE ASSIGNMENT

	21	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	22	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	23	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3212 (tolerance 
				-15s; 45s) at
				least after the channel release.
	24	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	25	SS -> MS	IMMEDIATE ASSIGNMENT	
	26	MS -> SS	LOCATION UPDATING REQ	location updating type : ‘periodic location update’ 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	27	SS -> MS	LOCATION UPDATING REJ	IE Reject cause = #17 "network 
				failure"
	28	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	29	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	30	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	31	SS -> MS	IMMEDIATE ASSIGNMENT	
	32	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	33	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	34	MS -> SS	AUTHENT RESPONSE
	35	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	36	MS -> SS	TMSI REALLOCATION CMP

37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle, updated" in cell A
	38	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	39	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	40	SS -> MS	IMMEDIATE ASSIGNMENT	
	41	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	42	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to #42
				* in Table 10.66 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being
				excluded
	43	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	44	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	45	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	46	SS -> MS	IMMEDIATE ASSIGNMENT
	47	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.

	48	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there is no more SACCH in the
				uplink
	48a	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				within T3211 + RadioLinkTimeOut seconds 
				after the SS stops RF Transmission.
	49	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	50	SS -> MS	IMMEDIATE ASSIGNMENT
	51	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	52	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

53	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	54	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	55	SS -> MS	IMMEDIATE ASSIGNMENT	
	56	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	57	SS		performs step 42 with cause  #38 
	58	MS		If the MS supports speech, it is made to perform
				an emergency call
	59	MS -> SS	CHANNEL REQUEST	Establishment cause: Emergency call
	60	SS -> MS	IMMEDIATE ASSIGNMENT	
	61	MS -> SS	CM SERVICE REQUEST	CM service type = Emergency call 
				establishment; CKSN = no key 
				available; Mobile Identity = IMSI
	62	SS -> MS	CM SERVICE ACCEPT
	63	MS -> SS	EMERGENCY SETUP
	64	SS -> MS	RELEASE COMPLETE	Cause = unassigned number
	65	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	66	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	67	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.
	68	MS		Depending on what has been 
				performed in step 66 the MS is 
				brought back to operation.
	69	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	70	SS -> MS	IMMEDIATE ASSIGNMENT
	71	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	72	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	73	MS -> SS	AUTHENT RESPONSE
	74	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	75	MS -> SS	TMSI REALLOCATION CMP
	76	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle, updated" in cell B

77	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	78	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	79	SS -> MS	IMMEDIATE ASSIGNMENT	
	80	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	81	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to #38
				* in Table 10.66 of GSM 04.08, 
				causes #2, #3, #6, #11, #12, and #13 being
				excluded
	82	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	83	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	84	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	85	SS -> MS	IMMEDIATE ASSIGNMENT
	86	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.

	87	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there is no more SACCH in the
				uplink
	88	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				within T3211 +RadioLinkTimeout
				seconds after the SS stops RF transmission.
	89	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	90	SS -> MS	IMMEDIATE ASSIGNMENT
	91	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	92	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	93	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.

94	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	95	SS -> MS	IMMEDIATE ASSIGNMENT	
	96	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	97	SS		performs step 48

	98	MS		A MO CM connection is attempted
	99	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	100	SS -> MS	IMMEDIATE ASSIGNMENT
	101	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	102	SS		performs step 52 
	103	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	104	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	105	SS -> MS	IMMEDIATE ASSIGNMENT
	106	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	107	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	108	MS -> SS	AUTHENT RESPONSE
	109	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	110	MS -> SS	TMSI REALLOCATION CMP
	111	SS -> MS	CHANNEL RELEASE	MS is now "idle, updated" in cell A
				The MS may or may not have memorised the
				request for CM connection. The steps 112 to 
				116 are therefore optional for the MS.
				The SS waits 10 second whether to decide to
				go directly to step 117
	112	MS -> SS	CHANNEL REQUEST
	113	SS -> MS	IMMEDIATE ASSIGNMENT
	114	MS -> SS	CM SERVICE REQUEST	CKSN = initial value,
				Mobile identity = TMSI
	115	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	116	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection of the main
				signalling link.

117	MS		The RF level of cell A is lowered 
				until the MS selects cell B.
				The RF level of cell A is set 
				sufficiently low to ensure that 
				cell A is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	118	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	119	SS -> MS	IMMEDIATE ASSIGNMENT	
	120	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = initial value, 
				LAI = a, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	121	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to #38
				* in Table 10.66 of GSM 04.08, 
				causes #2, #3, #6, #11, #12 and #13 being 
				excluded
	122	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	123	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.
	124	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	125	SS -> MS	IMMEDIATE ASSIGNMENT
	126	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.

	127	SS		The SS stops any RF transmission on 
				the dedicated channel and waits
				until there is no more SACCH in the
				uplink
	128	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				within T3211 + RadioLinkTimeOut seconds 
				after the SS stops RF transmission.

	129	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	130	SS -> MS	IMMEDIATE ASSIGNMENT
	131	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	132	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link

	133	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				during T3211 seconds at
				least after the channel release.

134	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	135	SS -> MS	IMMEDIATE ASSIGNMENT	
	136	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	137	SS		performs step 42 
	138	MS		The RF level of cell B is lowered 
				until the MS selects cell A.
				The RF level of cell B is set 
				sufficiently low to ensure that 
				cell B is not suitable as defined 
				in GSM 05.08 section 6.6.2.
	139	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	140	SS -> MS	IMMEDIATE ASSIGNMENT	
	141	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available , 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	142	SS		performs the step 48 
	143	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B
				until T3211 + RadioLinkTimeout seconds 
				after the SS stops RF transmission.
	1447	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	145	SS -> MS	IMMEDIATE ASSIGNMENT
	146	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE),
				Mobile Identity = IMSI.
	147	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN
	148	MS -> SS	AUTHENT RESPONSE
	149	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI
	150	MS -> SS	TMSI REALLOCATION CMP
	151	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
				MS is now "idle, updated" in cell A

Specific message contents

None.

##### 26.7.4.3.4	Location updating / abnormal cases / attempt counter less or equal to 4, stored LAI equal to broadcast LAI

26.7.4.3.4.1	Conformance requirement

1)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a periodic location updating procedure (the broadcast LAI is equal to the stored LAI),

1.1	the Mobile Station shall be able to establish an MM connection i.e. send a Channel Request and then a CM Service Request message, CKSN and LAI set to those which have been allocated to the Mobile Station, Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station

1.2	then the Mobile Station shall not attempt a location updating procedure.

2)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during an IMSI attach procedure (the broadcast LAI is equal to the stored LAI).

2.1	the Mobile Station shall be able to establish an MM connection i.e. send a Channel Request and then a CM Service Request message, CKSN and LAI set to those which have been allocated to the Mobile Station, Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station

2.2	then the Mobile Station shall not attempt a location updating procedure.

3)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a periodic location updating procedure and the attempt counter is smaller than 4 the Mobile Station shall send, after T3211 expiry, a Location Updating Request message with the Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station, CKSN IE and LAI set to those which have been allocated to the Mobile Station and the Location Updating type set to "periodic updating".

When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a periodic location updating procedure) after T3212 expiry it shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "normal".

4)	When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a periodic location updating procedure) it shall use a request for a CM connection other than emergency call as a trigger for a location updating procedure.

5)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during an IMSI attach procedure and the attempt counter is smaller than 4 the Mobile Station shall send, after T3211 expiry, a Location Updating Request message with the Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station, CKSN IE and LAI set to those which have been allocated to the Mobile Station and the Location Updating type set to "IMSI attach".

When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during an IMSI attach procedure) after T3212 expiry it shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "normal".

6)	When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during an IMSI attach procedure) it shall use a request for a CM connection other than emergency call as a trigger for a location updating procedure.

References

GSM 04.08 section 4.4.4.9

26.7.4.3.4.2	Test purpose

To verify that in the case when the attempt counter is smaller than 4 and the broadcast LAI is equal to the stored LAI, the MS is in the MM-IDLE state and NORMAL SERVICE substate. To verify that timer T3211 is stopped after a MM connection establishment.

To verify that the MS uses the T3211 timer. and that it enters the MM-IDLE state and NORMAL SERVICE substate when its attempt counter reaches value 4 even in the case where the stored LAI is equal to the broadcast LAI.

26.7.4.3.4.3	Method of test

Initial conditions

System Simulator:

One cell: B, belonging to location area b.

IMSI attach/detach is allowed.

T3212 is set to 6 minutes.

Mobile Station:

The MS is "Idle updated" on cell B with a valid CKSN and a TMSI.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Foreseen final state of the MS

The MS is "idle updated" on cell B with a valid CKSN and a TMSI.

Test Procedure

A failure during the periodic location updating is triggered: as the broadcast LAI is equal to the stored LAI, the MS is still in the MM-IDLE state and NORMAL SERVICE substate and timer T3211 is started. A CM connection other than for emergency call is attempted. It is checked that this is possible and that T3211 is stopped. Same test is performed with a failure during an IMSI attach procedure.

Then failures are triggered during the periodic location updating to let the attempt counter to reach the value of 4. The MS shall enter the MM-IDLE LIMITED SERVICE state and delete any TMSI, stored LAI, ciphering key sequence number and ciphering key. When the attempt counter reaches the value of 4, timer T3212 shall be started. At timer T3212 expiry a location updating procedure is started. A request for CM connection other for than emergency call shall trigger a location updating procedure.

Same tests are performed when the failures are triggered during an IMSI attach procedure.

Maximum duration of test

40 minutes

Expected sequence

Step	Direction	Message	Comments

1	SS		The SS shall wait at most T3212 + 45 seconds
	2	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	5	SS		performs step 5, of 26.7.4.3.2 with cause #17 
	6	MS		A MO CM connection is attempted
	7	MS -> SS	CHANNEL REQUEST
	8	SS -> MS	IMMEDIATE ASSIGNMENT
	9	MS -> SS	CM SERVICE REQUEST	CKSN = initial CKSN, Mobile Identity = TMSI
	10	SS -> MS	CM SERVICE ACCEPT
	11	MS -> SS	An initial CM message
	12	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	13	SS		The MS shall not initiate an RR connection
				establishment. This is checked during 2*T3211

14	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.

Steps 15 to 19 are optional
	15	MS -> SS	CHANNEL REQUEST	
	16	SS -> MS	IMMEDIATE ASSIGNMENT	
	17	MS -> SS	IMSI DETACH INDICATION
	18	SS -> MS	CHANNEL RELEASE
	19	MS		Depending on what has been 
				performed in step 14 the MS is 
				brought back to operation.
	20	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	21	SS -> MS	IMMEDIATE ASSIGNMENT	
	22	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	23	SS		performs step 11 of 26.7.4.3.2
	24	MS		A MO CM connection is attempted
	25	MS -> SS	CHANNEL REQUEST
	26	SS -> MS	IMMEDIATE ASSIGNMENT
	27	MS -> SS	CM SERVICE REQUEST	CKSN = initial CKSN, Mobile Identity = TMSI
	28	SS -> MS	CIPH MODE COMMAND
	29	MS -> SS	CIPH MODE COMPLETE
	30	MS -> SS	An initial CM message
	31	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection
				of the main signalling link

20	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	21	SS -> MS	IMMEDIATE ASSIGNMENT	
	22	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	23	SS		performs step 11 of 26.7.4.3.2
	24	MS		A MO CM connection is attempted
	25	MS -> SS	CHANNEL REQUEST
	26	SS -> MS	IMMEDIATE ASSIGNMENT
	27	MS -> SS	CM SERVICE REQUEST	CKSN = initial CKSN, Mobile Identity = TMSI
	28	SS -> MS	CIPH MODE COMMAND
	29	MS -> SS	CIPH MODE COMPLETE
	30	MS -> SS	An initial CM message
	31	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection
				of the main signalling link
	32	SS		The MS shall not initiate an RR connection
				establishment. This is checked during 2*T3211 
				MS is "idle, updated" in cell B.
	33	SS		The SS shall wait at most T3212 + 15 seconds
	34	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	35	SS -> MS	IMMEDIATE ASSIGNMENT	
	36	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	37	SS		performs step 16 of 26.7.4.3.2
	38	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	39	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	40	SS -> MS	IMMEDIATE ASSIGNMENT	
	41	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	42	SS		performs step 5 of 26.7.4.3.2 with cause #17
	43	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	44	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	45	SS -> MS	IMMEDIATE ASSIGNMENT	
	46	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	47	SS		performs step 11 of 26.7.4.3.2
	48	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.

49	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	50	SS -> MS	IMMEDIATE ASSIGNMENT	
	51	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

52	SS		performs step 16 of 26.7.4.3.2
	53	MS		The MS shall not initiate an RR connection
				establishment during T3212 - 15 seconds at 
				least after the channel release.
	54	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	55	SS -> MS	IMMEDIATE ASSIGNMENT	
	56	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic 
				or normal (see Note 1), 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	57	SS -> MS	AUTHENT REQUEST
	58	MS -> SS	AUTHENT RESPONSE
	59	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = TMSI
	60	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection 
				of the main signalling link
	61	MS		The MS shall no initiate an RR connection
				establishment earlier than T3212 - 15 seconds
				after the transmission of the
				CHANNEL RELEASE in step 62
	62	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	63	SS -> MS	IMMEDIATE ASSIGNMENT	
	64	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	65	SS		performs step 5 of 26.7.4.3.2 with cause #17
	66	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	67	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	68	SS -> MS	IMMEDIATE ASSIGNMENT	
	69	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	70	SS		performs step 11 of 26.7.4.3.2
	71	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.

72	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	73	SS -> MS	IMMEDIATE ASSIGNMENT	
	74	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	75	SS		performs step 16 of 26.7.4.3.2
	76	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	77	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	78	SS -> MS	IMMEDIATE ASSIGNMENT	
	79	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic, 
				CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.

	90	SS		performs step 5 of 26.7.4.3.2 with cause #17
	81	MS		A MO CM connection is attempted
	82	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	83	SS -> MS	IMMEDIATE ASSIGNMENT	
	84	MS -> SS	LOCATION UPDATING REQ	location updating type = normal, 
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	85	SS -> MS	LOCATION UPDATING ACC	IE mobile identity = TMSI
	86	MS -> SS	TMSI REALLOCATION CMP
	87	SS -> MS	CHANNEL RELEASE

Steps 88 to 92 are optional
				Wait 10 s to decide whether to go directly to
				step 93.
	88	MS -> SS	CHANNEL REQUEST
	9899	SS -> MS	IMMEDIATE ASSIGNMENT
	90	MS -> SS	CM SERVICE REQUEST	CKSN = no key available,
				Mobile identity = TMSI
	91	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	92	SS -> MS	CHANNEL RELEASE

93	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.

Steps 94 to 97 are optional
	94	MS -> SS	CHANNEL REQUEST	
	95	SS -> MS	IMMEDIATE ASSIGNMENT	
	96	MS -> SS	IMSI DETACH INDICATION
	97	SS -> MS	CHANNEL RELEASE

98	MS		Depending on what has been 
				performed in step 97 the MS is 
				brought back to operation.
	99	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	100	SS -> MS	IMMEDIATE ASSIGNMENT	
	101	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	102	SS		performs step 11 of 26.7.4.3.2
	103	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	104	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	105	SS -> MS	IMMEDIATE ASSIGNMENT	
	106	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	107	SS		performs the chosen step
	108	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	109	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	110	SS -> MS	IMMEDIATE ASSIGNMENT	
	111	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	112	SS		performs the chosen step
	113	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	114	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	115	SS -> MS	IMMEDIATE ASSIGNMENT	
	116	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	117	SS		performs step 11 of 26.7.4.3.2
	118	MS		The MS shall not initiate an RR connection
				establishment during T3212 - 15 seconds at 
				least after the channel release.
	119	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	120	SS -> MS	IMMEDIATE ASSIGNMENT

121	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic 
				or normal or IMSI attach (see 
				Note 2), CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	122	SS -> MS	AUTHENT REQUEST
	123	MS -> SS	AUTHENT RESPONSE
	124	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = TMSI
	125	MS -> SS	TMSI REALLOCATION CMP
	126	SS -> MS	CHANNEL RELEASE

127	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.

Steps 128 to 131 are optional
	128	MS -> SS	CHANNEL REQUEST	
	129	SS -> MS	IMMEDIATE ASSIGNMENT	
	130	MS -> SS	IMSI DETACH INDICATION
	131	SS -> MS	CHANNEL RELEASE

132	MS		Depending on what has been 
				performed in step 130 the MS is 
				brought back to operation.
	133	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	134	SS -> MS	IMMEDIATE ASSIGNMENT	
	135	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	136	SS		performs step 16 of 26.7.4.3.2
	137	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.

138	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	139	SS -> MS	IMMEDIATE ASSIGNMENT	
	140	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	141	SS		performs step 5 of 26.7.4.3.2 with cause #17
	142	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	143	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	144	SS -> MS	IMMEDIATE ASSIGNMENT

145	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	146	SS		performs step 11 of 26.7.4.3.2
	147	MS		The MS shall not initiate an RR connection
				establishment during T3211 at 
				least after the channel release.
	148	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	149	SS -> MS	IMMEDIATE ASSIGNMENT	
	150	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI 
				attach, CKSN = initial value, 
				LAI = b, mobile station classmark 1
				as given by the PICS and
				mobile identity = TMSI.
	151	SS		performs step 16 of 26.7.4.3.2
	152	MS		The MS is made to perform a MO call
	153	MS -> SS	CHANNEL REQUEST	Establishment cause :
					Location updating
	154	SS -> MS	IMMEDIATE ASSIGNMENT	
	155	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,
				CKSN = no key available, 
				LAI = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
				mobile station
				classmark 1 as given by the PICS 
				and mobile identity = IMSI.
	156	SS -> MS	AUTHENT REQUEST
	157	MS -> SS	AUTHENT RESPONSE
	158	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = TMSI
	159	MS -> SS	TMSI REALLOCATION CMP
	160	SS -> MS	CHANNEL RELEASE

Steps 161 to 166 are optional
	161	MS		An MO CM connection is attempted
	162	MS -> SS	CHANNEL REQUEST
	163	SS -> MS	IMMEDIATE ASSIGNMENT
	164	MS -> SS	CM SERVICE REQUEST	CKSN = initial value,
				Mobile identity = TMSI
	165	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	166	SS -> MS	CHANNEL RELEASE

NOTE 1:	the MS can include both types of Location updating. As T3212 expires it can be a periodic location updating procedure and as there is no stored LAI it can be a normal one.

NOTE 2:	same problem as in note 1. Three types of location updating procedures should be allowed.

Specific message contents

None.

#### 26.7.4.4	Location updating / release / expiry of T3240

26.7.4.4.1	Conformance requirement

The mobile station receiving a LOCATION UPDATING REJECT message shall start T3240: it shall abort the RR connection at the expiry of timer T3240.

References

GSM 04.08 section 4.4.4.8, 11.2.

26.7.4.4.2	Test purpose

To verify that the MS aborts the RR-connection at the expiry of timer T3240.

26.7.4.4.3	Method of test

Initial conditions

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell A.

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS is "idle updated" on cell B

Test Procedure

A normal location updating procedure is performed. The RR-connection is not released by the SS within the timer T3240. It is checked that the MS aborts the RR-connection.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	SS		The RF level of cell A is
				lowered until the MS selects
				cell B.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	
	5	SS -> MS	LOCATION UPDATING ACC	
	6	SS		The SS waits T3240 expiry.
	7	MS		The MS shall abort the RR
				connection (disconnection of layer 2).

Specific message contents

None.

#### 26.7.4.5	Location updating / periodic

##### 26.7.4.5.1	Location updating / periodic spread

26.7.4.5.1.1	Conformance requirement

1)	The Mobile Stations shall perform spreading of the time before performing a periodic location updating when the location updating timer value is reduced.

2)	The Mobile Station shall reset timer T3212 when the Mobile Station is deactivated, and shall start with a value between zero and the broadcasted value when reactivated in the same cell, IMSI attach being forbidden.

3)	When activated the Mobile Station shall start timer T3212 with a value randomly drawn in the allowed range.

NOTE 3:	This conformance requirement is not covered by a test purpose. It is intended to be covered by a manufacturer declaration.

References

GSM 04.08 section 4.4.2.

26.7.4.5.1.2	Test purpose

1)	To check that when the location updating timer is reduced, the timer running in the MS is started with a value depending on the current timer value and the new broadcasted T3212 value.

2)	To verify that when the MS is reactivated in the same cell (as the one in which it was deactivated), IMSI attach being forbidden, the MS starts the timer T3212 with a value between zero and the broadcasted value

NOTE 4:	It is not tested that the value is random.

26.7.4.5.1.3	Method of test

Initial conditions

System Simulator:

One cell, T3212 is set to 30 minutes.

IMSI attach is allowed in the cell.

Mobile Station:

The MS is deactivated. The stored MCC, MNC and LAC correspond to the broadcasted values. The stored update status is "updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated. It performs IMSI attach. 3 minutes after the end of the IMSI attach procedure, the value of T3212 is set to 6 minutes. The MS shall perform periodic location updating 6 minutes after the end of the IMSI attach procedure.

Then, the IMSI attach/detach is forbidden. T3212 is still set to 6 minutes.

The MS is deactivated. The MS is reactivated. It is checked that the MS performs a periodic location updating during the 6 minutes following activation.

Maximum duration of test

20 minutes

Expected sequence

Step	Direction	Message	Comments

1	MS		The MS is activated
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	"location updating type": IMSI attach
	5	SS -> MS	LOCATION UPDATING ACC	
	6	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
					the SS waits for the disconnection 
					of the main signalling link.

7	SS		3 minutes after step 6 the value of T3212
				is set to 6 minutes
	8	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
				This message shall be sent
				by the MS between 5minutes 45s and
				6minutes 15s after step 6.
	9	SS -> MS	IMMEDIATE ASSIGNMENT	
	10	MS -> SS	LOCATION UPDATING REQ
				"location updating type": periodic updating
	11	SS -> MS	LOCATION UPDATING ACC	
	12	SS -> MS	CHANNEL RELEASE
				After the sending of this message, 
					the SS waits for the disconnection 
					of the main signalling link.

13	SS		IMSI attach/detach is not allowed.
	14	MS		The MS is deactivated
	15	MS		The MS is activated
	16	SS		The SS waits until the periodic
				location updating.
	17	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall
				arrive during the 7 minutes 
				following the MS activation.
	18	SS -> MS	IMMEDIATE ASSIGNMENT	
	19	MS -> SS	LOCATION UPDATING REQ	"Location updating type” = periodic
	20	SS -> MS	LOCATION UPDATING ACC	
	21	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.4.5.2	Location updating / periodic normal / test 1

26.7.4.5.2.1	Conformance requirement

1	The Mobile Station shall stop and reset the timer T3212 of the periodic location updating procedure when the first MM message is received or ciphering mode setting is completed in the case of MM connection establishment.

2	The Mobile Station shall stop and reset the timer T3212 of the periodic location updating procedure when the Mobile Station has responded to paging and thereafter has received the first correct L3 message that is not an RR message.

References

GSM 04.08 section 4.4.2.

26.7.4.5.2.2	Test purpose

To verify that the MS stops and resets the timer T3212 of the periodic location updating procedure when :

-	the first MM-message is received in the case of MM-connection establishment, ciphering mode being not set,

-	the MS has responded to paging and the first correct L3 message that is not an RR message is received.

NOTE 5:	T3212 is stopped when the MM-idle state is left and restarted when the MM sublayer returns to that state, substate NORMAL SERVICE or ATTEMPTING TO UPDATE. As a consequence, the exact time when T3212 is reset between those two events cannot be tested.

26.7.4.5.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

IMSI attach/detach is not allowed.

The T3212 timeout value is 2/10 hour .

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

An MS originated MM connection is established and cleared. The channel is released. It is checked that the MS performs a periodic location updating 12 minutes after the release of the channel.

One minute after the periodic location updating, the MS is paged, it sends a CHANNEL REQUEST message and the SS responds with an IMMEDIATE ASSIGNMENT message, a call is established and then cleared. It is checked that the MS performs a periodic location updating 12 minutes after the release of the link.

Maximum duration of test

30 minutes.

Expected sequence

Step	Direction	Message	Comments

1	MS		A MO CM connection is attempted
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS -> MS	CM SERVICE REJECT	cause #17 (network failure)
	6	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits 
				for the disconnection of the main signalling link.
	7	SS		The SS waits until the periodic
				location updating.
	8	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall
				arrive between 11 minutes 45 s and
				12 minutes 15 s after
				the last release of the RR
				connection by the SS.
	9	SS -> MS	IMMEDIATE ASSIGNMENT	
	10	MS -> SS	LOCATION UPDATING REQ	"Location updating type” = periodic
	11	SS -> MS	LOCATION UPDATING ACC	
	12	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

13	SS		The SS waits 1 minute.
	14	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity” = IMSI.
	15	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
				Answer to paging.
	16	SS -> MS	IMMEDIATE ASSIGNMENT	
	17	MS -> SS	PAGING RESPONSE	
	18	SS -> MS	AUTHENTICATION REQ
	19	MS -> SS	AUTHENTIC RESPONSE
	20	SS - MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	21	SS		The SS waits until the periodic
				location updating.
	22	MS -> SS	CHANNEL REQUEST
				"Establishment cause" : Location updating
				This message shall
				arrive between 11 minutes 45 s and
				12 minutes 15 s after
				the last release of the RR
				connection by the SS.
	23	SS -> MS	IMMEDIATE ASSIGNMENT	
	24	MS -> SS	LOCATION UPDATING REQ	"Location updating type” = periodic
	25	SS -> MS	LOCATION UPDATING ACC	
	26	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.4.5.3	Location updating / periodic normal / test 2

26.7.4.5.3.1	Conformance requirement

When a LOCATION UPDATING ACCEPT or a LOCATION UPDATING REJECT message is received, the timer T3212 is stopped and reset and the Mobile Station shall perform a periodic location updating after T3212 expiry.

References

GSM 04.08 section 4.4.2.

26.7.4.5.3.2	Test purpose

To verify that the MS stops and resets the timer T3212 of the periodic location updating procedure when a LOCATION UPDATING ACCEPT message is received.

NOTE 6:	T3212 is stopped when the MM-idle state is left and restarted when the MM sublayer returns to that state, substate NORMAL SERVICE or ATTEMPTING TO UPDATE. As a consequence, the exact time when T3212 is reset between those two events cannot be tested.

26.7.4.5.3.3	Method of test

Initial conditions

System Simulator:

2 cells, IMSI attach/detach is allowed in both cells.

T3212 is set to 6 minutes.

Mobile Station:

The MS has a valid TMSI. It is "idle updated” on cell A.

Related PICS/PIXIT statements

SIM removal possible without powering down Yes/No

Switch off on button yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell B.

Test procedure

A normal location updating is performed. The channel is released. One minute later, the MS is deactivated, then reactivated in the same cell. It is checked that the MS performs an IMSI attach and a periodic location updating 6 minutes after the IMSI attach.

Maximum duration of test

20 minutes

Expected sequence

Step	Direction	Message	Comments

The following messages are sent and shall be 
				received on cell B.
	1	SS		The RF level of cell A is 
				lowered until the MS selects
				cell B.
	2	MS -> SS	CHANNEL REQUEST	"establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	"location updating type” = normal
	5	SS -> MS	LOCATION UPDATING ACC	
	6	SS ->MS	CHANNEL RELEASE	After the sending of this message,
				the SS waits for the disconnection
				of the main signalling link.
	7	SS		The SS waits until the periodic
				location updating.
	8	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall arrive between 5 minutes 45s
				and 6 minutes 15 s
				after the last release of the RR
				connection by the SS.
	9	SS -> MS	IMMEDIATE ASSIGNMENT	
	10	MS -> SS	LOCATION UPDATING REQ	"Location updating type” = periodic
	11	SS -> MS	LOCATION UPDATING ACC	
	12	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	13	MS		If possible (see PICS) SIM 
				removal is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
				steps 14 to 17 may be performed or not
				depending on the action made in step 13
	14	MS -> SS	CHANNEL REQUEST	
	15	SS -> MS	IMMEDIATE ASSIGNMENT	
	16	MS -> SS	IMSI DETACH INDICATION	
	17	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	18	MS		Depending on what has been 
				performed in step 13 the MS is 
				brought back to operation.
	19	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	20	SS -> MS	IMMEDIATE ASSIGNMENT	
	21	MS -> SS	LOCATION UPDATING REQ	"Location updating type” = IMSI 
				attach.
	22	SS -> MS	LOCATION UPDATING ACC	
	23	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	24	SS		The SS waits until the periodic
				location updating.

25	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
				This message shall
				arrive between 5 minutes 45 s and 6 minutes 15s
				after the last release of the RR
				connection by the SS.
	26	SS -> MS	IMMEDIATE ASSIGNMENT	
	27	MS -> SS	LOCATION UPDATING REQ	"Location updating type” = periodic
	28	SS -> MS	LOCATION UPDATING ACC	
	29	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.4.5.4	Location updating / periodic HPLMN search

###### 26.7.4.5.4.1	Location updating / periodic HPLMN search / MS waits time T

26.7.4.5.4.1.1	Conformance requirement

When in automatic mode and roaming in the home country, the MS shall make an attempt to access the HPLMN, if the MS is on the VPLMN at time T after since the last attempt.

NOTE 7:	This test is not intended to test every value in the range 6 minutes to 8 hours or the default of 30 minutes, but is intended to check that the mobile is capable of using the value stored on the SIM.

References

GSM 02.11 section 3.2.2.5.2

GSM 03.22 section 4.4.3.3

26.7.4.5.4.1.2	Test purpose

To verify that when a cell of the HPLMN becomes available, following the successful location request on the VPLMN of the home country and after the first search the mobile has failed to find its HPLMN, that the MS shall perform a location update request on the HPLMN after time T. Were T is the HPLMN Search Period stored in the SIM.

26.7.4.5.4.1.3	Method of test

Initial conditions

System Simulator :

Two cells A and B, belonging to different location areas with location identification a and b. Cell A shall be a cell of the HPLMN and Cell B shall be a cell of the VPLMN with a Country Code the same as that of Cell A. Initially Cell A shall not be broadcasting. IMSI attach/detach is not allowed on either cell.

Mobile Station :

The MS is switched off. The HPLMN Search Period on the SIM shall be set to 6 minutes. The location area information on the SIM is "deleted".

Related PICS/PIXIT statements

Switch on/off button	Yes/No.

Foreseen final state of the MS

The MS is "idle updated" on Cell A.

Test Procedure

Only Cell B shall be broadcasting. The MS shall be switched on either by using the Power Switch or by applying power. A normal location updating is performed on Cell B. Cell A shall be made available after 8 minutes, thus ensuring the MS fails to find the HPLMN during its first attempt. It is verified that the MS performs a location update request on Cell A, within 6 minutes after broadcasting of Cell A.

Maximum duration of test

17 minutes.

Expected sequence

Step	Direction	Message	Contents

The following messages shall be sent and
				received on Cell B.
	1	MS		The MS is switched on by either using the
				Power Switch or by applying power.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATE REQ	"Location Update Type" : Normal.
	5	SS -> MS	LOCATION UPDATE ACC
	6	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.
				The SS waits a period of 8 minutes, this
				allowing the MS to make its first
				periodic search

8	SS		Cell A is made available.
				Within 8 minutes after step 8 the following
				messages shall be sent and received on Cell A
	9	MS -> SS	CHANNEL REQUEST	"Establishment cause": Location updating
	10	SS -> MS	IMMEDIATE ASSIGNMENT
	11	MS -> SS	LOCATION UPDATE REQ	"Location Update Type": normal
	12	SS -> MS	LOCATION UPDATE ACC
	13	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

Specific message contents

None.

###### 26.7.4.5.4.2	Location updating / periodic HPLMN search / MS in manual mode

26.7.4.5.4.2.1	Conformance requirement

The periodic attempts shall only be performed if in automatic mode when the MS is roaming in its home country.

References

GSM 02.11 section 3.2.2.5.2

GSM 03.22 section 4.4.3.3

26.7.4.5.4.2.2	Test purpose

To verify that no HPLMN Search is performed when the MS is not in automatic mode

26.7.4.5.4.2.3	Method of test

Initial conditions

System Simulator :

Two cells A and B, belonging to different location areas with location identification a and b. Cell A shall be a cell of the HPLMN and Cell B shall be a cell of the VPLMN with a Country Code the same as that of Cell A. Initially Cell A shall not be broadcasting. IMSI attach/detach is not allowed on either cell.

Mobile Station :

The MS is switched off. The HPLMN Search Period on the SIM shall be set to 6 minutes. The location area information on the SIM is "deleted".

Related PICS/PIXIT statements

Switch on/off button	Yes/No.

Foreseen final state of the MS

The MS is "idle updated" on Cell B.

Test Procedure

Only Cell B shall be broadcasting. The MS shall be switched on either by using the Power Switch or by applying power. A normal location updating is performed on Cell B. The MS is forced into manual selection mode. Cell A is made available. It is verified that the MS does not attempt to perform a location update on Cell A.

Maximum duration of test

7 minutes.

Expected sequence

Step	Direction	Message	Contents

The following messages shall be sent and
				received on Cell B.
	1	MS		The MS is switched on by either using the
				Power Switch or by applying power.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATE REQ	"Location Update Type" : Normal.
	5	SS -> MS	LOCATION UPDATE ACC
	6	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

8	MS		The MS is forced into manual selection mode.
	9	SS		Cell A is made available.
	10	SS		The SS waits a period of 7 minutes. During this
				time no messages shall be received on Cell A.

Specific message contents

None.

###### 26.7.4.5.4.3	Location updating / periodic HPLMN search / MS waits at least two minutes and at most T minutes

26.7.4.5.4.3.1	Conformance requirement

After switch on, the MS waits at least 2 minutes and at most T minutes before the first HPLMN Search is attempted.

References

GSM 02.11 section 3.2.2.5.2

GSM 03.22 section 4.4.3.3

26.7.4.5.4.3.2	Test purpose

To verify that the MS waits at least 2 minutes and at most T minutes before attempting its first HPLMN Search.

26.7.4.5.4.3.3	Method of test

Initial Conditions

System Simulator :

Two cells A and B, belonging to different location areas with location identification a and b. Cell A shall be a cell of the HPLMN and Cell B shall be a cell of the VPLMN with a Country Code the same as that of Cell A. Initially Cell A shall not be broadcasting. IMSI attach/detach is not allowed on either cell.

Mobile Station :

The MS is switched off. The HPLMN Search Period on the SIM shall be set to 6 minutes. The location area information on the SIM is "deleted".

Related PICS/PIXIT statements

Switch on/off button	Yes/No.

Foreseen final state of the MS

The MS is "idle updated" on Cell A.

Test Procedure

Only Cell B shall be broadcasting. The MS shall be switched on either by using the Power Switch or by applying power. A normal location updating is performed on Cell B. Cell A is made available. It is verified that the MS attempts to perform a location update on Cell A, after at least 2 minutes and at most T minutes have passed following power on.

Maximum duration of test

8 minutes.

Expected sequence

Step	Direction	Message	Contents

The following messages shall be sent and
				received on Cell B.
	1	MS		The MS is switched on by either using the
				Power Switch or by applying power.
	2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Location updating
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATE REQ	"Location Update Type" : Normal.
	5	SS -> MS	LOCATION UPDATE ACC
	6	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

8	SS		Cell A is made available.
	9	SS		The SS waits a period of 2 minutes after the MS
				is switched on. During this time no messages
				shall be received on Cell A.

				The following messages shall be sent and
				received on cell A.
				Within T minutes after the MS is switched on
				the following messages shall be sent and
				received on cell A.
	10	MS -> SS	CHANNEL REQUEST	"Establishment cause": Location updating
				This message shall be sent between 2 and 7
				minutes after step 1
	11	SS -> MS	IMMEDIATE ASSIGNMENT
	12	MS -> SS	LOCATION UPDATE REQ	"Location Update Type": normal
	13	SS -> MS	LOCATION UPDATE ACC
	14	SS -> MS	CHANNEL RELEASE	After sending this message the SS waits for the
				disconnection of the main signalling link.

Specific message contents

None.

#### 26.7.4.6	Location updating / interworking of attach and periodic

26.7.4.6.1	Conformance requirement

1)	If the Mobile Station is in service state NO CELL AVAILABLE, LIMITED SERVICE, PLMN SEARCH or PLMN SEARCH-NORMAL SERVICE when the timer expires the location updating procedure is delayed until this service state is left.

2)	The T3212 timeout value shall not be changed in the NO CELL AVAILABLE, LIMITED SERVICE, PLMN SEARCH and PLMN SEARCH-NORMAL SERVICE states.

3)	If the selected cell is in the location area where the mobile station is registered and IMSI ATTACH is not required and timer T3212 has not expired, then the state is NORMAL SERVICE.

References

1	GSM 04.08 section 4.4.2.

2	GSM 04.08 section 4.4.2.

3	GSM 04.08 section 4.2.1.1

26.7.4.6.2	Test purpose

1)	To check that if the PLU timer expires while the MS is out of coverage, the MS informs the network of its return to coverage.

2)	To check that the PLU timer is not disturbed by cells of forbidden PLMNs.

3)	To check that if the PLU timer does not expire while out of coverage and if the mobile returns to the LA where it is updated, the mobile does not inform the network of its return to coverage.

26.7.4.6.3	Method of test

Initial conditions

System Simulator:

Two cells, a and b, of different PLMNs.

T3212 is set to 12 minutes on cell a.

T3212 is set to 6 minutes on cell b.

IMSI attach is allowed in both cells.

Mobile Station:

The MS is deactivated. The PLMN of cell b is entered in the SIM's forbidden PLMN list.

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS is "idle updated". The PLMN of cell b is entered in the SIM's forbidden PLMN list.

Test procedure

The MS is activated and placed in automatic network selection mode. It performs IMSI attach. 1 minute after the end of the IMSI attach procedure, cell a is switched off. The MS shall not location update on cell b. 8 minutes after the end of the IMSI attach procedure, cell a is switched on. The MS shall not location update on cell a before 11.75 minutes after the end of the IMSI attach procedure. The MS shall perform a periodic location update on cell a between 11.75 minutes and 12.25 minutes after the end of the IMSI attach procedure.

3 minutes after the end of the periodic location updating procedure, cell a is switched off. The MS shall not location update on cell b. 14 minutes after the end of the periodic location updating procedure, cell a is switched on and cell b is switched off. The MS shall perform a location update on cell a before 17 minutes after the end of the periodic location updating procedure.

Maximum duration of test

35 minutes

Expected sequence

Step	Direction	Message	Comments

1	MS		The MS is activated in automatic 
				network selection mode
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	LOCATION UPDATING REQ	"location updating type": IMSI attach
	5	SS -> MS	LOCATION UPDATING ACC	
	6	SS -> MS	CHANNEL RELEASE	After the sending of this message, 
				the SS waits for the disconnection 
				of the main signalling link.
	7	SS		1 minute after step 6, cell a is switched off
	8	SS		8 minutes after step 6, cell a is switched on
	9	MS -> SS	CHANNEL REQUEST	This message shall be sent
				by the MS between 11 minutes 45s and
				12 minutes 15s after step 6.
	10	SS -> MS	IMMEDIATE ASSIGNMENT	
	11	MS -> SS	LOCATION UPDATING REQ
				"location updating type": periodic updating
	12	SS -> MS	LOCATION UPDATING ACC	
	13	SS -> MS	CHANNEL RELEASE
				After the sending of this message, 
				the SS waits for the disconnection 
				of the main signalling link.
	14	SS		3 minutes after step 13, cell a is switched off
	15	SS		14 minutes after step 13, cell a is switched on
				and cell b is switched off.
	16	MS -> SS	CHANNEL REQUEST	This message shall
				be sent by the MS before 17 minutes
				after step 13.
	17	SS -> MS	IMMEDIATE ASSIGNMENT	
	18	MS -> SS	LOCATION UPDATING REQ	"Location updating type” = periodic
	19	SS -> MS	LOCATION UPDATING ACC	
	22	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

### 26.7.5	MM connection

#### 26.7.5.1	Introduction

[tbd]

#### 26.7.5.2	MM connection / establishment with cipher

26.7.5.2.1	Conformance requirement

1)	The Mobile Station shall be able to correctly set up an MM connection in a Mobile Originating CM connection attempt and send a CM Service Request message with CKSN information element as stored in the SIM and Mobile Identity information element set to the TMSI

2)	The Mobile Station shall be able to interpret cipher mode setting as acceptance of its CM service request i.e. send a CM message.

References

GSM 04.08 sections 4.5.1.1.

26.7.5.2.2	Test purpose

To verify that the MS can correctly set up an MM connection in an origination and interpret cipher mode setting as acceptance of its CM service request.

26.7.5.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has valid TMSI, CKSN. It is "idle updated".

Test Procedure

A mobile originating CM connection is initiated. After the MS has sent the CM SERVICE REQUEST message to the SS, an authentication procedure and a ciphering mode setting procedure are performed. Then, the MS sends a CM message and the SS clears the call and releases the channel.

Maximum duration of test

One minute

Expected sequence

Step	Direction	Message	Comments

1	MS		A MO CM connection is attempted
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS -> MS	AUTHENT REQUEST	
	6	MS -> SS	AUTHENT RESPONSE	
	7	SS -> MS	CIPH MODE COMMAND	The SS starts deciphering.
	8	MS -> SS	CIPH MODE COMPLETE	The SS starts enciphering.

A9	MS -> SS	SETUP
	A10	SS -> MS	RELEASE COMPLETE	"Cause" IE : "unassigned number".

B9	MS -> SS	REGISTER
	B10	SS -> MS	RELEASE COMPLETE

C9	MS -> SS	CP-DATA
	C10	SS -> MS	CP-ACK
	C11	SS -> MS	CP-DATA
	C12	MS -> SS	CP-ACK

13	SS -> MS	CHANNEL RELEASE	After the sending of this message, the SS waits for 
				the disconnection of the main signalling link.

Specific message contents

None.

#### 26.7.5.3	MM connection / establishment without cipher

26.7.5.3.1	Conformance requirement

Upon reception of the CM SERVICE ACCEPT message, the MS shall send a CM message.

References

GSM 04.08 sections 4.5.1.1.

26.7.5.3.2	Test purpose

To verify that the MS can correctly set up an MM connection in an originating CM connection establishment when ciphering mode setting is not required.

26.7.5.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. The MM-connection is established without invoking the ciphering mode setting procedure.

Then, the MS sends a CM message and the SS releases the channel.

Maximum duration of test

one minute

Expected sequence

Step	Direction	Message	Comments

1	MS		A MO CM connection is attempted
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	SETUP

B6	MS -> SS	REGISTER

C6	MS -> SS	CP-DATA
	C7	SS -> MS	CP-ACK
	C8	SS -> MS	CP-DATA
	C9	MS -> SS	CP-ACK

10	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.4	MM connection / establishment rejected

26.7.5.4.1	Conformance requirement

Upon reception of a CM SERVICE REJECT message, the MS shall not send any layer 3 message, start timer T3240 and enter the "wait for network command" state.

References

GSM 04.08 sections 4.5.1.1.

26.7.5.4.2	Test purpose

To verify that the MS does not send a layer 3 message when the service request is rejected by the SS.

26.7.5.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI; It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS responds with a CM SERVICE REJECT message with reject cause "requested service option not subscribed". It is checked that the MS does not send a layer 3 message.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	MS		A MO CM connection is attempted
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS -> MS	CM SERVICE REJECT	"Reject cause" IE : "requested
				service option not subscribed".
	6	SS		The MS shall not send a layer 3
				message. This is checked during
				5 seconds.
	7	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.5	MM connection / establishment rejected cause 4

26.7.5.5.1	Conformance requirement

1)	The Mobile Station shall be able to correctly set up an MM connection in a Mobile Originating CM connection attempt and send a CM Service Request message with CKSN information element as stored in the SIM and Mobile Identity information element set to the TMSI

2)	The Mobile Station, when receiving a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR" shall wait for the network to release the RR connection.

3)	The Mobile Station shall then be able to perform a location updating procedure.

References

GSM 04.08 sections 4.5.1.1.

26.7.5.5.2	Test purpose

To verify that the MS can correctly accept a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR".

26.7.5.5.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has valid TMSI, CKSN. It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS responds with a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR". On receipt of this message, the MS shall delete any TMSI, LAI, cipher key and cipher key sequence number. The channel is released. It is checked that the MS performs a normal location updating procedure.

Maximum duration of test

One minute

Expected sequence

Step	Direction	Message	Comments

1	MS		A MO CM connection is attempted
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS -> MS	CM SERVICE REJECT	"Reject cause” = "IMSI
				unknown in VLR"
	6	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.
	7	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	8	SS -> MS	IMMEDIATE ASSIGNMENT	
	9	MS -> SS	LOCATION UPDATING REQ	"Ciphering key sequence number” =
				"No key is available".
				"Mobile identity” = IMSI.
				"Location area identification” =
				deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE).
	10	SS -> MS	AUTHENT REQUEST	
	11	MS -> SS	AUTHENT RESPONSE
	12	SS -> MS	LOCATION UPDATING ACC	"Mobile identity” = new TMSI.
	13	MS -> SS	TMSI REALLOCATION CMP	
	14	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.6	MM connection / expiry T3230

26.7.5.6.1	Conformance requirement

At T3230 expiry (i.e. no response is given but an RR connection is available) the MM connection establishment shall be aborted.

References

GSM 04.08 sections 4.5.1.2 and 11.2.

26.7.5.6.2	Test purpose

To verify that at T3230 expiry, the MS aborts the MM-connection establishment.

26.7.5.6.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Related PICS/PIXIT statements

None.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test Procedure

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS waits for expiry of timer T3230. It is checked that the MS does not send a layer 3 message but waits for the release of the RR-connection.

Maximum duration of test

1 minute

Expected sequence

Step	Direction	Message	Comments

1	MS		A MO CM connection is attempted.
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS		The SS waits for expiry of
				timer T3230.
	6	SS -> MS	CM SERVICE ACCEPT	
	7	MS -> SS	MM-STATUS	"Reject cause " IE is "message
				not compatible with the call
				state or not implemented".
	8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.7	MM connection / abortion by the network

##### 26.7.5.7.1	MM connection / abortion by the network / cause #6

26.7.5.7.1.1	Conformance requirement

1)	Upon reception of an ABORT message, the MS shall release any ongoing MM connection and enter the "wait for network command" state.

2)	If the cause in the ABORT message was cause #6, the Mobile Station shall:

2.1	not perform normal location updating

2.2	not perform periodic location updating

2.3	not respond to paging with TMSI

2.4	reject any request for Mobile Originating call establishment except Emergency call

2.5	not perform IMSI detach if deactivated

3)	After reception of an ABORT message with cause #6, the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a Channel Request message with the establishment cause set to "emergency call".

4)	After reception of an ABORT message with cause #6, the Mobile Station shall delete the stored LAI, CKSN and TMSI.

Reference(s)

GSM 04.08 section 4.3.5

26.7.5.7.1.2	Test purpose

To check that upon reception of an ABORT message with cause #6 during call establishment :

-	the MS does not send any layer 3 message.

-	after reception of an ABORT message and after having been deactivated and reactivated, the MS performs location updating using its IMSI as mobile identity and indicates deleted LAI and CKSN.

-	the MS does not perform location updating, does not answer to paging with TMSI, rejects any request for mobile originating call except emergency call, does not perform IMSI detach.

-	the MS accepts a request for emergency call.

26.7.5.7.1.3	Method of test

Initial Conditions

System Simulator:

2 cells, default parameters.

Mobile Station:

The MS has a valid TMSI, CKSN and Kc. It is "idle updated" on cell B.

Related PICS/PIXIT Statement(s)

SIM removal possible without powering down Yes/No.

Switch off on button Yes/No

Support of speech Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated" on cell A.

Test procedure

A mobile originating CM connection is attempted. Upon reception of the AUTHENTICATION RESPONSE message, the SS sends an ABORT message with cause #6. The SS waits for 5 seconds. The MS shall not send any layer 3 message. The SS releases the RR connection.

The SS checks that the MS has entered the state MM-IDLE substate NO IMSI, i.e. does not perform normal location updating, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if deactivated.

Maximum Duration Of Test

10 minutes.

Expected Sequence

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	MS		A mobile originating CM connection is attempted
	 2	MS -> SS	CHANNEL REQUEST
	 3	SS -> MS	IMMEDIATE ASSIGNMENT	
	 4	MS -> SS	CM SERVICE REQUEST
	 5	SS -> MS	AUTHENT REQUEST
	 6	MS -> SS	AUTHENT RESPONSE
	 7	SS -> MS	ABORT	"reject cause" = #6
	 8	SS		The SS waits for 5 seconds
	 9	MS		The MS shall not send any layer 3 message
				during that time
	 10	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

The following messages are sent and shall be received on cell A

11	   SS		The RF levels are changed
				to make the MS reselect cell A.
	 12	   MS		The MS performs cell reselection
				according to procedure as specified in GSM
				05.08 (this however is not checked 
				until step 22). The MS shall not initiate
				an RR connection establishment on
				cell A or on cell B.

13	SS		The SS waits at least 7 minutes
				for a possible periodic
				updating.
	14 	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.

15	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains
				TMSI.
	16	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is verified during 3 seconds.

17	MS		A MO CM connection is attempted.
	18	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds

19	MS		If the MS supports speech (see PICS),
				an emergency call is attempted.
	20	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Emergency call.
	21	SS -> MS	IMMEDIATE ASSIGNMENT	
	22	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency
				call establishment.
	23	SS -> MS	CM SERVICE ACCEPT	
	24	MS -> SS	EMERGENCY SETUP
	25	SS -> MS	RELEASE COMPLETE	"Cause” = unassigned number.
	26	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

27	MS		If possible (see PICS) SIM 
				detachment is performed. Otherwise 
				if possible (see PICS) switch off 
				is performed. Otherwise the power 
				is removed.
	28	MS		The MS shall not initiate an RR connection
				establishment on cell A or on cell B.
				This is checked during 3 seconds.

29	MS		Depending on what has been 
				performed in step 29 the MS is 
				brought back to operation.
	30	MS -> SS	CHANNEL REQUEST	"Establishment cause" :
					Location updating
	31	SS -> MS	IMMEDIATE ASSIGNMENT	
	32	MS -> SS	LOCATION UPDATING REQ	"location updating type” = normal,
				"CKSN” = no key available,
				"Mobile Identity” = IMSI,
				"LAI” = deleted LAI (the MCC and MNC hold
				the previous values, the LAC is coded FFFE)
	33	SS -> MS	AUTHENT REQUEST	"CKSN” = CKSN1
	34	MS -> SS	AUTHENT RESPONSE	
	35	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity” = TMSI
	36	MS -> SS	TMSI REALLOCATION CMP
	37	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.5.7.2	MM connection / abortion by the network / cause not equal to #6

26.7.5.7.2.1	Conformance requirement

Upon reception of an ABORT message, the MS shall release any ongoing MM connection and enter the "wait for network command" state.

Reference(s)

GSM 04.08 section 4.3.5

26.7.5.7.2.2	Test purpose

To check that when multiple MM connections are established, the MS releases all MM connections upon reception of an ABORT message, in the case when the two MM connections are established for a mobile terminating call and a non call related supplementary service operation.

26.7.5.7.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in state U10 of a mobile terminating call

Related PICS/PIXIT Statement(s)

The MS supports a non call related supplementary service operation during an active call Yes/No.

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated"

Test procedure

A non call related supplementary service operation is attempted at the MS. Upon reception of the REGISTER message, the SS sends an ABORT message with cause # 17. The SS sends a DISCONNECT using the TI of the mobile terminating call. The MS shall send a RELEASE COMPLETE message with the PD and TI of the DISCONNECT message and with cause #81. The SS releases the RR connection.

Maximum Duration Of Test

15 seconds.

Expected Sequence

This procedure is performed if the MS supports non call related supplementary service operation

Step	Direction	Message	Comments

1	MS		A non call related supplementary
				service operation is attempted
				at the MS
	2	MS -> SS	CM SERVICE REQUEST
	3	SS -> MS	CM SERVICE ACCEPT
	4	MS -> SS	REGISTER
	5	SS -> MS	ABORT	"reject cause" = #17
	6	SS -> MS	DISCONNECT	with the TI of the
				mobile terminating call
	7	MS -> SS	RELEASE COMPLETE	"cause” = #81. Same PD and TI
				as the DISCONNECT message
	8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

#### 26.7.5.8	MM connection / follow-on request pending

##### 26.7.5.8.1	MM connection / follow-on request pending / test 1

26.7.5.8.1.1	Conformance requirement

The MS shall not attempt to establish a new MM connection after location updating on the same RR connection if not allowed by the network.

Reference(s)

GSM 04.08 section 4.4.4.6

26.7.5.8.1.2	Test purpose

To check that when the network does not include the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has a CM application request pending does not attempt to establish a new MM connection on that RR connection.

26.7.5.8.1.3	Method of test

Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS has a valid TMSI and is deactivated.

Related PICS/PIXIT Statement(s)

none

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated and a CM connection is immediately attempted. The MS performs location updating. The SS does not include the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for at least 8 seconds. The MS shall not send any layer 3 message for 8 seconds.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated and CM connection 
				is immediately attempted.
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATING REQ
	5	SS -> MS	LOCATION UPDATING ACC	follow on proceed IE not included
	6	SS		The SS wait for at least 8 seconds
	7	MS		The MS shall not send any layer 3
				message for 8 seconds after reception
				of the LOCATION UPDATING ACCEPT
				message.
	8	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

None.

##### 26.7.5.8.2	MM connection / follow-on request pending / test 2

26.7.5.8.2.1	Conformance requirement

A MS supporting the follow-on request procedure and having a CM connection request pending shall correctly establish an MM connection following a location update when allowed by the network.

Reference(s)

GSM 04.08 section 4.4.4.6

26.7.5.8.2.2	Test purpose

To check that when the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that supports the follow on request procedure and that has a CM application request pending establishes successfully a new MM connection on that RR connection.

26.7.5.8.2.3	Method of test

Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS has a valid TMSI and is deactivated.

Related PICS/PIXIT Statement(s)

MS supports the follow on request procedure   Yes/No

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated and a CM connection is immediately attempted. The MS performs location updating. The SS includes the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for at least 8 seconds.

If the MS supports the follow on request procedure :

The MS shall send a CM SERVICE REQUEST. Upon reception of that message, the SS sends a CM SERVICE ACCEPT message. The MS shall send an initial CM message. Upon reception of that message, the SS releases the RR connection.

If the MS does not support the follow on request procedure :

The MS shall not send any layer 3 message for 8 seconds.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated and an CM connection
				is immediately attempted.
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATING REQ
	5	SS -> MS	LOCATION UPDATING ACC	follow on proceed IE included

If the MS supports the follow on
				request procedure (see PICS) steps A6 to A8
				are performed, otherwise steps B6 to B7
				are performed

A6	MS -> SS	CM-SERVICE REQUEST
	A7	SS ->MS	CM-SERVICE ACCEPT
	A8	MS -> SS	An initial CM message

B6	SS		The SS wait for at least 8 seconds
	B7	MS		The MS shall not send any layer 3
				message for 8 seconds after reception
				of the LOCATION UPDATING ACCEPT
				message.

9	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

none

##### 26.7.5.8.3	MM connection / follow-on request pending / test 3

26.7.5.8.3.1	Conformance requirement

1)	The MS shall not set the follow on request bit in a LOCATION UPDATING REQUEST message if no MM connection request is pending.

2)	When the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has no CM application request pending shall not attempt to establish a new MM connection on that RR connection.

3)	The MS shall correctly handle a CM connection established by the network on the RR connection that was used for the location updating procedure.

Reference(s)

GSM 04.08 section 4.4.4.6

26.7.5.8.3.2	Test purpose

1)	To check that a MS that has no CM application request pending sets the Follow-On-Request bit to No follow-on request pending in a LOCATION UPDATING REQUEST message.

2)	To check that when the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has no CM application request pending does not attempt to establish a new MM connection on that RR connection.

3)	To check that the MS accepts establishment by the network of a new MM connection on the existing RR connection.

26.7.5.8.3.3	Method of test

Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS has a valid TMSI and is deactivated.

Related PICS/PIXIT Statement(s)

Supported services on TCH

Foreseen final state of the MS

The MS has a valid TMSI. It is "idle updated".

Test procedure

The MS is activated. The MS performs location updating. The MS shall set the FOR bit to No follow-on request pending in the LOCATION UPDATING REQUEST message. The SS includes the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for 5 seconds. The MS shall not send any layer 3 message for 5 seconds. The SS sends a SETUP message to the MS requesting a basic service supported by the MS. The MS shall send either a CALL CONFIRMED message if it supports a service on TCH or a RELEASE COMPLETE with cause #88.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	LOCATION UPDATING REQ	"Location updating type" : the FOR bit
				is set to No follow-on request pending
	5	SS -> MS	LOCATION UPDATING ACC	follow on proceed IE is included
	6	SS		The SS wait for 5 seconds
	7	MS		The MS shall not send any layer 3
				message for 5 seconds after reception
				of the LOCATION UPDATING ACCEPT
				message.
	8	SS -> MS	SETUP

If the MS supports a basic service on TCH
	A9	MS -> SS	CALL CONFIRMED

If the MS does not support any basic
				service on TCH
	B9	MS -> SS	RELEASE COMPLETE	cause #88

10	SS -> MS	CHANNEL RELEASE	After the sending of this
				message, the SS waits for the
				disconnection of the main
				signalling link.

Specific message contents

none

### 26.7.6	Default contents of messages

Default contents SYSTEM INFORMATION messages and default settings

For cell A and B 		For GSM use 26.6.14

For DCS use 26.6.15

Cell C

The contents of SYSTEM INFORMATION TYPE 1 to 6 messages for cell C are identical to those of cell A with the following exceptions:

Cell Channel Description

-	Format Identifier			Bit map 0 for GSM

Range 512 for DCS

-	Cell Allocation ARFCN			Channel Number 30 for GSM,

Channel Number 700 for DCS,

Cell Identity

-	Cell Identity Value			0003H

Default settings for cell C:

Downlink input level				53 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power

class

Propagation profile				static

BCCH/CCCH carrier number			30 for GSM

700 for DCS

ABORT

Information element					Value/remark
Reject cause						Depending on the test one of either :

#6 	- Illegal ME

#17 	- Network Failure.

AUTHENTICATION REQUEST

Information element					Value/remark
Cipher Key Sequence Number				Arbitrary

Authentication parameter RAND 				Arbitrarily chosen by the test house

AUTHENTICATION RESPONSE

Information element	Value/remark
Authentication parameter SRES	As applicable

AUTHENTICATION REJECT

Information element	Value/remark
None but message head

CHANNEL RELEASE

Information element	Value/remark
RR cause	Normal release

CIPHERING MODE COMMAND

Information element	Value/remark
Cipher mode setting	Start ciphering
Cipher Response	IMEI must not be included

CM RE-ESTABLISHMENT REQUEST

Information element					Value/remark

Cipher Key Sequence Number				According to SIM contents

Mobile station classmark 2					See PICS/PIXIT
Mobile Identity						IMSI of MS under test

Location area identification					As in section 26.1.1

CM SERVICE ACCEPT

Information element	Value/remark
None but message head	Omitted

CM SERVICE REQUEST

Information element	Value/remark

CM service type	Mobile originating call establishment
	unless otherwise specified in test.

Ciphering key sequence number	According to SIM contents
Mobile station classmark 2	See PICS/PIXIT
Mobile identity	TMSI of the MS under test

CM SERVICE REJECT

Information element	Value/remark
Reject cause	Depending on test

IDENTITY REQUEST

Information element	Value/remark
Identity type	Depending on test
Spare half octet	0000

IDENTITY RESPONSE

Information element	Value/remark
Mobile identity	Depending on test

IMMEDIATE ASSIGNMENT

Information element	Value/remark
L2 pseudo length
Page mode	Normal Paging
Spare half octet	0000
Channel description
- Channel type and TDMA offset	SDCCH/4 or SDCCH/8
- Time slot number	Arbitrary legal value
- Subsequent fields of the Channel description IE depend upon the Type of MS under test (either DCS1800 or GSM), as specified in section 26.1.1
Request reference
- Random access information	As received from MS
- N51,N32,N26	Corresponding to the frame in which the Channel Request was sent
Timing advance	0
Mobile allocation	Empty (L=0)
Starting time	Omitted
IA rest octets	all bits set to spare

IMSI DETACH INDICATION

Information element	Value/remark
Mobile station classmark 1	See PICS/PIXIT
Mobile identity	TMSI of the MS under test

LOCATION UPDATING ACCEPT

Information element	Value/remark
Location area identification	As in section 26.1.1
Mobile identity	Omitted
Follow on proceed	Omitted

LOCATION UPDATING REJECT

Information element					Value/remark
Reject cause						As specified in test

LOCATION UPDATING REQUEST

Information element					Value/remark
Location updating type					Normal location updating

Cipher Key Sequence Number				According to SIM contents

Location area identification					As in section 26.1.1
Mobile station classmark					See PICS/PIXIT

Mobile identity						TMSI of the MS

PAGING REQUEST TYPE 1

Information element	Value/remark
L2 pseudo length
Page Mode	Normal paging
Channels needed
- mobile 1	"any channel"
- mobile 2	spare
Mobile identity 1	TMSI of MS under test
Mobile identity 2	Omitted
P1 rest octets	All bits set to spare

PAGING RESPONSE

Information element	Value/remark
Ciphering key sequence number	According to SIM contents
Spare half octet	0000
Mobile station classmark 2	See PICS/PIXIT
Mobile identity	TMSI of the MS under test

TMSI REALLOCATION COMMAND

Information element	Value/remark
Location area identification	As in section 26.1.1
Mobile identity	TMSI of the MS under test

TMSI REALLOCATION COMPLETE

Information element	Value/remark
None but message head	omitted

## 26.8	Tests related to circuit switched call control

### 26.8.1	Circuit switched Call Control (CC) state machine verification

#### 26.8.1.1	General on CC state machine verification

The principle of checking the call control functions consists in the validation of each call control identified state.

State U0 as an initial state is not verified in the tests of 26.8.1.2 (establishment of an outgoing call).

State U0.1 is never verified.

The steps to be followed within each performed test are:

-	bring the MS into the required state,

-	trigger the tested event,

-	check the MS response and new state.

In sections 26.8.1.2 and 26.8.1.3 different tables are defined to bring the MS into the required initial state. The exact table to be chosen is specified individually in section "Initial conditions" of "Method of test" for each test case.

For each test, unless otherwise specified, a circuit switched basic service among those supported by the MS but excluding the emergency call teleservice shall be chosen arbitrarily, and the test shall be performed according to that basic service. If the only circuit switched basic service supported by the mobile is emergency call, then the incoming call tests shall not be performed and the other call control tests shall be performed with the EMERGENCY SETUP message replacing the SETUP message.

The initial states are to be checked through STATUS ENQUIRY messages sent by the SS, when feasible. This is not explicitly stated in the tables of expected sequences of signalling messages. The checking of final states are explicitly included into the expected sequences of signalling messages.

The following postamble may be used by the SS to bring MS back to idle mode in those test cases, in which it is not already included into expected sequence of signalling messages:

Table 26.8.1.1/1: A postamble to bring the MS back to idle mode.

Step	Direction	Message	Comments

n	SS -> MS	CHANNEL RELEASE	
	n+1	MS		the MS shall release the main
				signalling link (DISC/UA)

The postamble has not been included into the all of the tests in order to leave an option to concatenate the procedures in the future by using a final state of a test case as an initial state to another one.

For the special case of U0, the state is checked by sending STATUS ENQUIRY message with all possible values of transaction identifier (seven values) as U0 is the only state in which for every TI the MS will answer with release complete with cause #81. If U0 is to be verified when no RR connection exists, first a mobile terminating radio connection must be established.

The MS responses are either call management messages received by the SS or lower layers functions activated within the MS or MMI actions (e.g. the buzzing of an alerting tone).

A timeout within the MS is triggered by the SS when it does not answer back an MS expected response.

The test sequences may be split in 3 main groups:

-	establishment and release of an outgoing call;

-	establishment and release of an incoming call;

-	in-call functions.

Remark on verification of transient states:

Some call control states of the mobile station may be transient, depending on implementation, configuration of the MS and previous messages (see Annex 3, subclause 3.1.6).

If a test starts in a transient state, then the test is executed without verification of the starting state.

##### 26.8.1.1.1	List of abbreviations

The following abbreviations are used throughout the section 26.8.1.

AUTH REQUEST			=	AUTHENTICATION REQUEST

AUTH RESPONSE			=	AUTHENTICATION RESPONSE

CM-REESTAB REQUEST		=	CM-REESTABLISHMENT REQUEST

CHANNEL MODE MOD ACK	=	CHANNEL MODE MODIFY ACKNOWLEDGE

CIPH MODE COMMAND		=	CIPHERING MODE COMMAND

CIPH MODE COMPLETE		=	CIPHERING MODE COMPLETE

START DTMF ACK		=	START DTMF ACKNOWLEDGE

STOP DTMF ACK			=	STOP DTMF ACKNOWLEDGE

#### 26.8.1.2	Establishment of an outgoing call

Initial conditions

As a minimum requirement the MS is updated and has been given a TMSI, a ciphering key and cipher key sequence number, and the layer 2, RR and MM functionalities have been verified.

There are as many CM initial conditions as states to be checked.

The tables below describe message exchanges which bring the MS in the requested initial states.

A state may be taken as initial only when all the states which lead to this initial states have been validated. The order followed in the test procedure will be U0, U0.1, U1, U3, U4, U10, U12, U19, U11 as seen in the table underneath.

The MS is brought again in the initial state starting with U0 at each new test performed.

Table 26.8.1.2/1: Establishment of an outgoing call, procedure 1 (late assignment)

Step	Direction	Message	Comments/actions/next state

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call
	2	SS -> MS	IMMEDIATE ASSIGNMENT	SDCCH, U0
	3	MS -> SS	CM-SERVICE REQUEST	U0.1
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	U1
	7	SS -> MS	CALL PROCEEDING	U3
	8	SS -> MS	ALERTING	U4
	9	SS -> MS	ASSIGNMENT COMMAND	TCH
	10	MS -> SS	ASSIGNMENT COMPLETE	
	11	SS -> MS	CONNECT	
	12	MS -> SS	CONNECT ACKNOWLEDGE	U10

A13	SS -> MS	DISCONNECT	U12 (note 1)

B13	SS -> MS	DISCONNECT	U12 (note 2)
	B14	MS -> SS	RELEASE	U19

C13			MMI action, terminate call
	C14	MS -> SS	DISCONNECT	U11

NOTE 1:	The Progress Indicator IE with progress description #8 "in band information or appropriate pattern now available" is included.

NOTE 2:	The Progress Indication IE is not included.

Table 26.8.1.2/2: Establishment of an outgoing call, procedure 2

Step	Direction	Message	Comments/actions/next state

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call
	2	SS -> MS	IMMEDIATE ASSIGNMENT	TCH, U0
	3	MS -> SS	CM-SERVICE REQUEST	U0.1
	4	SS -> MS	CHANNEL MODE MODIFY	(note 3)
	5	MS -> SS	CHANNEL MODE MOD ACK	
	6	SS -> MS	CIPH MODE COMMAND	
	7	MS -> SS	CIPH MODE COMPLETE	
	8	MS -> SS	SETUP	U1
	9	SS -> MS	CALL PROCEEDING	U3
	10	SS -> SS	ALERTING	U4
	11	SS -> MS	CONNECT	
	12	MS -> SS	CONNECT ACKNOWLEDGE	U10

A13	SS -> MS	DISCONNECT	U12 (note 4)

B13	SS -> MS	DISCONNECT	U12 (note 5)
	B14	MS -> SS	RELEASE	U19

C13			MMI action, terminate call
	C14	MS -> SS	DISCONNECT	U11

NOTE 3:	Assigned channel is appropriate for the chosen bearer capability (see 26.8.1).

NOTE 4:	The Progress Indicator IE with progress description #8 "in band information or appropriate pattern now available' is included.

NOTE 5:	The Progress Indicator IE is not included.

Table 26.8.1.2/3: Establishment of an outgoing call, procedure 3

Step	Direction	Message	Comments/actions/next state

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call
	2	SS -> MS	IMMEDIATE ASSIGNMENT	SDCCH, U0
	3	MS -> SS	CM-SERVICE REQUEST	U0.1
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	U1
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	U3
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> MS	ALERTING	U4
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACKNOWLEDGE	U10

A15	SS -> MS	DISCONNECT	U12 (note 6)

B15	SS -> MS	DISCONNECT	U12 (note 7)
	B16	MS -> SS	RELEASE	U19

C15			MMI action, terminate call
	C16	MS -> SS	DISCONNECT	U11

NOTE 6:	The Progress Indicator IE with progress description #8 "in band information or appropriate pattern now available" is included.

NOTE 7:	The Progress indicator IE is not included.

Table 26.8.1.2/4: Establishment of an outgoing call, procedure 4

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call
	2	SS -> MS	IMMEDIATE ASSIGNMENT	TCH, U0
	3	MS -> SS	CM-SERVICE REQUEST	U0.1
	4	SS -> MS	IDENTITY REQUEST	
	5	MS -> SS	IDENTITY RESPONSE	
	6	SS -> MS	CIPH MODE COMMAND	
	7	MS -> SS	CIPH MODE COMPLETE	
	8	MS -> SS	SETUP	U1
	9	SS -> MS	CHANNEL MODE MODIFY	(note 8)
	10	MS -> SS	CHANNEL MODE MOD ACK	
	11	SS -> MS	CALL PROCEEDING	U3
	12	SS -> MS	ALERTING	U4
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACKNOWLEDGE	U10

A15	SS -> MS	DISCONNECT	U12 (note 9)

B15	SS -> MS	DISCONNECT	U12 (note 10)
	B16	MS -> SS	RELEASE	U19

C15			MMI action, terminate call
	C16	MS -> SS	DISCONNECT	U11

NOTE 8:	Assigned channel is appropriate for the chosen bearer capability (see 26.8.1).

NOTE 9:	The Progress Indicator IE with progress description #8 "in band information or appropriate pattern now available" is included.

NOTE 10:	The Progress Indicator IE is not included.

##### 26.8.1.2.1	Outgoing call / U0 null state

###### 26.8.1.2.1.1	Outgoing call / U0 null state / MM connection requested

26.8.1.2.1.1.1	Definition and applicability

The call control entity of the Mobile Station requests the MM-sublayer to establish a mobile originating MM-connection. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.1.1.2	Conformance requirement

1)	Upon initiation of an outgoing basic call by user the MS shall initiate establishment of an MM connection, using as first MM message a CM-SERVICE REQUEST message with CM service type "Mobile originating call establishment or packet mode connection establishment".

References

TS GSM 04.08 section 5.2.1.1, TS GSM 04.08 section 4.5.1.1, TS GSM 04.08 section 3.3.1.

26.8.1.2.1.1.3	Test purpose

To verify that upon initiation of an outgoing basic call by user the MS initiates establishment of an MM connection, using as first MM message a CM-SERVICE REQUEST message with CM service type "Mobile originating call establishment or packet mode connection establishment".

26.8.1.2.1.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

U0, null.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. When the SS receives CM-SERVICE REQUEST, the contents of it shall be checked, and also the state of the call control entity shall be checked by STATUS ENQUIRY with all the relevant transaction identifiers.

Maximum duration of test

30 s

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call
	2	SS -> MS	IMMEDIATE ASSIGNMENT	TCH
	3	MS -> SS	CM-SERVICE REQUEST	verify the type of call which is asked
				for "basic" or "emergency by the MS
	4	SS -> MS	STATUS ENQUIRY	
	5	MS -> SS	RELEASE COMPLETE	with cause 81# (invalid TI value)
	6	SS		repeat steps 4-5 to cover all the
				transaction identifiers from 000 ...110
	7	SS -> MS	CHANNEL RELEASE	
	8	MS		the MS shall release the main signalling
				link (DISC/UA)

Specific message contents:

None.

##### 26.8.1.2.2	Outgoing call / U0.1 MM connection pending

###### 26.8.1.2.2.1	Outgoing call / U0.1 MM connection pending / CM service rejected

26.8.1.2.2.1.1	Definition and applicability

A request for MM connection is rejected by the SS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.2.1.2	Conformance requirement

Upon receiving indication of an MM-connection establishment being rejected, CC entity should inform upper layer of this rejection.

References

TS GSM 04.08, section 4.5.1.1 , TS GSM 04.07, section 6.2.2.

26.8.1.2.2.1.3	Test purpose

To verify that a CC entity of the MS in CC-state U0.1, "MM-connection pending", upon the MS receiving a CM SERVICE REJECT message, returns to CC state U0, "Null".

26.8.1.2.2.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U0.1 by using table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. When the SS receives CM-SERVICE REQUEST, the contents of it shall be checked. The SS rejects it by CM-SERVICE REJECT. Then the SS will check the state of the MS by using STATUS ENQUIRY with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CM-SERVICE REJECT	
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	RELEASE COMPLETE	cause shall be 81# (invalid TI value)
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000 ...110
	5	SS -> MS	CHANNEL RELEASE	
	6	MS		the MS shall release the main signalling 						link (DISC/UA)

Specific message contents:

None.

###### 26.8.1.2.2.2	Outgoing call / U0.1 MM connection pending / CM service accepted

26.8.1.2.2.2.1	Definition and applicability

A CM request is accepted for the MM-connection by the SS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.2.2.2	Conformance requirement

A CC entity of the MS in CC-state U0.1, "MM-connection pending", upon the MS receiving a CM SERVICE ACCEPT message, shall send a SETUP message specifying the Called party BCD number that was entered into the MS and then enter CC state U1, "Call initiated".

References

TS GSM 04.08, section 4.5.1.1, TS GSM 04.08, section 5.2.1.1.

26.8.1.2.2.2.3	Test purpose

To verify that a CC entity of the MS in CC-state U0.1, "MM-connection pending", upon the MS receiving a CM SERVICE ACCEPT message, sends a SETUP message specifying the Called party BCD number that was entered into the MS and then enters CC state U1, "Call initiated".

26.8.1.2.2.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U0.1 by using table 26.8.1.2/1.

Foreseen final state of the MS

U1, call initiated.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. When the MS is requesting a MM-connection, the SS will indicate acceptance by sending a CM-SERVICE ACCEPT message. The MS shall respond with SETUP. Then the SS will check the state of the call control entity by STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CM-SERVICE ACCEPT	
	2	MS -> SS	SETUP	with called party BCD number
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause shall be 30# (response to enq.)
				and state U1 call initiated

Specific message contents:

None.

###### 26.8.1.2.2.3	Outgoing call / U0.1 MM connection pending / lower layer failure

26.8.1.2.2.3.1	Definition and applicability

The call control entity of the MS being in the state, U0.1, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.2.3.2	Conformance requirement

1)	Upon a lower layer failure the MS releases the MM connection in progress and returns to idle mode. In that state no call exists, and the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

References

TS GSM 04.08, section 4.5.1.2, TS GSM 04.08, section 5.2.1.1., TS GSM 04.08 section 5.5.3.2. and TS GSM 04.08 section 8.3.

26.8.1.2.2.3.3	Test purpose

To verify that after the MS with a CC entity in state U0.1, "MM-connection pending", has detected a lower layer failure and has returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.2.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U0.1 by using table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. When the MS has sent a CM-SERVICE REQUEST message, the SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1		SS	SS generates lower layer failure
	2		SS	SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause shall be 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000 ...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

##### 26.8.1.2.3	Outgoing call / U1 call initiated

###### 26.8.1.2.3.1	Outgoing call / U1 call initiated / receiving CALL PROCEEDING

26.8.1.2.3.1.1	Definition and applicability

The call control entity of the MS being in the state, U1, a CALL PROCEEDING message is sent by the SS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.1.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CALL PROCEEDING message, shall enter CC state U3, "Mobile originating call proceeding".

References

TS GSM 04.08, section 5.2.1.1, 5.2.1.2, 5.2.1.3.

26.8.1.2.3.1.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CALL PROCEEDING message, enters CC state U3, "Mobile originating call proceeding".

26.8.1.2.3.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/2.

Foreseen final state of the MS

U3, Mobile originating call proceeding.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a CALL PROCEEDING message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U3.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CALL PROCEEDING	tone generation not mandatory
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U3

Specific message contents:

None.

###### 26.8.1.2.3.2	Outgoing call / U1 call initiated / rejecting with RELEASE COMPLETE

26.8.1.2.3.2.1	Definition and applicability

The call control entity of the MS being in the state, U1, the call is rejected by a RELEASE COMPLETE message sent by the SS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.2.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a RELEASE COMPLETE message with valid cause value, shall enter CC state U0, "Null".

2)	On returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

3)	On releasing the MM-connection, the MS shall wait for  MM layer release initiated by the network.

References

Conformance requirement 1:	TS GSM 04.08, section 5.4.2, TS GSM 04.08, section 5.4.4.

Conformance requirement 2:	TS GSM 04.08, section 5.5.3.2.

Conformance requirement 3:	TS GSM 04.08, section 5.4.4.3, TS GSM 04.08, section 4.5.3, TS 					GSM 04.08, section 3.5.1.

26.8.1.2.3.2.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a RELEASE COMPLETE message with valid cause value, enters CC state U0, "Null".

2)	To verify that in returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

3)	To verify that in releasing the MM-connection, the MS shall wait for MM layer release initiated by SS.

26.8.1.2.3.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a RELEASE COMPLETE message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE COMPLETE	See specific message content below.
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000...110
	5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

RELEASE COMPLETE

1) With a valid cause value among:

related to numbering,

#1 unallocated number

#3 no route to destination

#22 number changed

#28 invalid number format

related to bearer capabilities,

#8 operator determined barring

#57 bearer capability not authorized

#58 bearer capability not presently available

#63 service or option not available

#65 bearer service not implemented

#34 no circuit/channel available (call queuing).

###### 26.8.1.2.3.3	Outgoing call / U1 call initiated / T303 expiry

26.8.1.2.3.3.1	Definition and applicability

The call control entity of the MS being in the state, U1, if no response is then received from the SS, timer T303 expires at the MS side. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.3.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon expiry of T303 shall send a DISCONNECT message to its peer entity and enter state U11, "Disconnect request".

References

TS GSM 04.08, section 5.2.1.1, TS GSM 04.08, section 5.4.

26.8.1.2.3.3.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon expiry of T303 (accuracy +/-20% between reception of CM-SERVICE REQUEST and DISCONNECT by SS) sends a DISCONNECT message to its peer entity and enters state U11, "Disconnect request".

26.8.1.2.3.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/2.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

1 minute.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. When T303 expires at the MS, the MS shall send DISCONNECT. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS waits for T303 expiry
	2	MS -> SS	DISCONNECT	Shall be transmitted between 24 s and
				36 s after the CM SERVICE REQUEST.
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, status U11

Specific message contents:

None.

###### 26.8.1.2.3.4	Outgoing call / U1 call initiated / lower layer failure

26.8.1.2.3.4.1	Definition and applicability

The call control entity of the MS being in the state, U1, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.4.2	Conformance requirement

Upon a lower layer failure MM informs the relevant CM entities that the MM connection has been interrupted. As call re-establishment is not allowed, the CC entity must perform a local release. The MS returns to idle mode. In that state no call exists, and the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

References

TS GSM 04.08, section 4.5.2.3, TS GSM 04.08, section 5.2.1.1, TS GSM 04.08 section 5.5.3.2.

26.8.1.2.3.4.3	Test purpose

To verify that after the MS with a CC entity in state U1 "Call initiated", has detected a lower layer failure and has returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.3.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U1. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	
	11	MS		the MS shall release the main signalling
				link (DISC/UA)

Specific message contents:

None.

###### 26.8.1.2.3.5	Outgoing call / U1 call initiated / receiving ALERTING

26.8.1.2.3.5.1	Definition and applicability

The call control entity of the MS being in the state, U1, an ALERTING message is sent to the MS as a indication that a call is being alerted at a called end. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.5.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of an ALERTING message, shall enter CC state U4, "Call delivered".

References

TS GSM 04.08, section 5.2.1.1.

26.8.1.2.3.5.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of an ALERTING message, enters CC state U4, "Call delivered".

26.8.1.2.3.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/4.

Foreseen final state of the MS

U4, call delivered.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends an ALERTING message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U4, call delivered.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ALERTING
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

###### 26.8.1.2.3.6	Outgoing call / U1 call initiated / entering state U10

26.8.1.2.3.6.1	Definition and applicability

The call control entity of the MS being in the state, U1, a CONNECT message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.6.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CONNECT message, shall send a CONNECT ACKNOWLEDGE message to its peer entity and enter CC state U10, "Active".

References

TS GSM 04.08, section 5.2.1.1, TS GSM 04.08, section 5.2.1.6.

26.8.1.2.3.6.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CONNECT message, sends a CONNECT ACKNOWLEDGE message to its peer entity and enters CC state U10, "Active".

26.8.1.2.3.6.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/4.

Foreseen final state of the MS

U10, call active.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a CONNECT message to the MS. The MS shall respond by sending a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U10, active.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CONNECT	
	2	MS -> SS	CONNECT ACKNOWLEDGE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

###### 26.8.1.2.3.7	Outgoing call / U1 call initiated / unknown message received

26.8.1.2.3.7.1	Definition and applicability

The call control entity of the MS being in the state, U1, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.3.7.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a message with message type not defined for the protocol discriminator from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.2.3.7.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a message with message type not defined for the protocol discriminator unknown message from its peer entity returns a STATUS message.

26.8.1.2.3.7.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/1.

Foreseen final state of the MS

U1, call initiated.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U1
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U1

Specific message contents:

None.

##### 26.8.1.2.4	Outgoing call / U3 MS originating call proceeding

###### 26.8.1.2.4.1	Outgoing call / U3 MS originating call proceeding / ALERTING received

26.8.1.2.4.1.1	Definition and applicability

The call control entity of the MS being in the state, U3, an ALERTING message is sent to the MS as a indication that a call is being alerted at a called end. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a ALERTING message shall enter CC-state U4, "Call Delivered".

References

TS GSM 04.08 section 5.2.1.5.

26.8.1.2.4.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a ALERTING message enters CC-state U4, "Call Delivered".

26.8.1.2.4.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U4, call delivered.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends an ALERTING message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U4, call delivered.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ALERTING
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

###### 26.8.1.2.4.2	Outgoing call / U3 MS originating call proceeding / CONNECT received

26.8.1.2.4.2.1	Definition and applicability

The call control entity of the MS being in the state, U3, a CONNECT message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a CONNECT message shall return a "CONNECT ACKNOWLEDGE" message to its peer entity and enter the CC state U10, "Active".

2)	The MS shall then stop any locally generated indication.

References

Conformance requirement 1:	TS GSM 04.08 section 5.2.1.6.

Conformance requirement 2:	TS GSM 04.08 section 5.2.1.6.

26.8.1.2.4.2.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a CONNECT message returns a "CONNECT ACKNOWLEDGE" message to its peer entity and enters the CC state U10, "Active".

2)	To verify that the MS stops locally generated indication, if any.

26.8.1.2.4.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U10, active.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a CONNECT message to the MS. The MS shall respond by sending a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U10, active.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CONNECT	the MS shall stop tone generation, if any
	2	MS -> SS	CONNECT ACKNOWLEDGE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

###### 26.8.1.2.4.3	Outgoing call / U3 MS originating call proceeding / PROGRESS received without in band information

26.8.1.2.4.3.1	Definition and applicability

The call control entity of the MS being in the state, U3, a PROGRESS message is received by the MS. The PROGRESS message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message with valid cause values shall stay in CC-state U3.

2)	After receipt of the PROGRESS message timer T310 shall be stopped.

References

Conformance requirement 1:	TS GSM 04.08 section 5.2.1.4.

Conformance requirement 2:	TS GSM 04.08 section 11.3.

26.8.1.2.4.3.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message with valid cause values stays in CC-state U3.

2)	To verify that after receipt of the PROGRESS message timer T310 is stopped.

26.8.1.2.4.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U3, mobile originating call proceeding.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a PROGRESS message not containing indication of in-band information availability to the MS. The SS checks that the MS has stopped T310, i.e. at T310 timeout no DISCONNECT message is sent by the MS. Then the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PROGRESS	(note 11)
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U3
	4	SS		SS waits at least 45 seconds and checks
				no DISCONNECT is sent by the MS
	5	SS -> MS	STATUS ENQUIRY	
	6	MS -> SS	STATUS	cause 30#, state U3

NOTE 11:	Tested with a valid cause value among:

#4 call has returned to PLMN/ISDN

#32 call is end-to-end PLMN/ISDN or

any value in the set #(21-127)

Specific message contents:

None.

###### 26.8.1.2.4.4	Outgoing call / U3 MS originating call proceeding / PROGRESS with in band information

26.8.1.2.4.4.1	Definition and applicability

The call control entity of the MS being in the state, U3, a PROGRESS message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message indicating in-band announcement shall through-connect the traffic channel  for speech, if TCH is in a speech mode. If TCH is not in speech mode, the MS shall not through-connect the TCH.

2)	After receipt of the PROGRESS message, T310 shall be stopped.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.1.3, TS GSM 04.08 section 5.2.1.4., TS GSM 04.08 section 5.2.1.9, TS GSM 04.08 section 5.5.1., TS GSM 04.08 section 11.3.

26.8.1.2.4.4.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message indicating in-band announcement through-connects the traffic channel for speech, if TCH is in speech mode. If TCH is not in a speech mode, the MS does not through-connect the TCH.

2)	To verify that after receipt of the PROGRESS message, T310 is stopped.

26.8.1.2.4.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U3, mobile originating call proceeding.

Maximum duration of test

1 minute.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a PROGRESS message containing indication of in-band information availability to the MS. The SS checks that if channel mode is speech, the TCH shall be through connected. If channel mode is not speech, the TCH shall not be through connected. Also the SS checks that the MS has stopped T310, i.e. at T310 timeout no DISCONNECT message is sent by the MS. Then the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PROGRESS 1)	the MS shall stop all the CC timers (note 12),
				if channel mode is speech, the TCH shall be
				through connected. If channel mode is not
				speech, the TCH shall not be through
				connected.
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U3
	4	SS		SS waits at least 45 seconds and checks
				no DISCONNECT is sent by the MS
	5	SS -> MS	STATUS ENQUIRY	
	6	MS -> SS	STATUS	cause 30#, state U3
	7	SS		If the channel mode is speech
				the SS will check that the user connection
				for speech is attached
				(both downlink and uplink)

Specific message contents:

NOTE 12:	Tested with a valid cause value among:

#1 call is not end to end PLMN/ISDN

#2 destination address is non PLMN/ISDN

#3 originating address is non PLMN/ISDN

#8 in band information or appropriate pattern now available or any value in the set #(6-20).

###### 26.8.1.2.4.5	Outgoing call / U3 MS originating call proceeding / DISCONNECT with in band tones

26.8.1.2.4.5.1	Definition and applicability

The call control entity of the MS being in the state, U3, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT with progress indicator #8, shall through-connect the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

References

TS GSM 04.08 section 5.2.1.4., TS GSM 04.08 section 5.4.4.

26.8.1.2.4.5.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT with progress indicator #8 through-connects the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS sends a RELEASE message.

26.8.1.2.4.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U12, disconnect indication.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. The SS checks that if channel mode is speech, the TCH shall be through connected and the MS enters state U12, disconnect indication. If channel mode is not speech, the TCH shall not be through connected and the MS shall enter state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 13)

TCH in speech mode:
	B2	SS		the SS will check that the audio path
				for in band tones is attached.
	B3	SS -> MS	STATUS ENQUIRY	
	B4	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:
	C2	MS -> SS	RELEASE	
	C3	SS -> MS	STATUS ENQUIRY	
	C4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 13:	the cause value:

#8 in band information or appropriate pattern now available.

###### 26.8.1.2.4.6	Outgoing call / U3 MS originating call proceeding / DISCONNECT without in band tones

26.8.1.2.4.6.1	Definition and applicability

The call control entity of the MS being in the state, U3, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.6.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT without progress indicator shall return a RELEASE message and enter the CC-state U19, "Release Request"

References

TS GSM 04.08 section 5.4.4.

26.8.1.2.4.6.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT without progress indicator returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.4.6.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.4.7	Outgoing call / U3 MS originating call proceeding / RELEASE received

26.8.1.2.4.7.1	Definition and applicability

The call control entity of the MS being in the state, U3, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a RELEASE will return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	The MS on returning to the idle mode shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

3)	On releasing the MM-connection, the MS shall wait for  MM layer release initiated by the network.

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.2., TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

Conformance requirement 3:	TS GSM 04.08, section 5.4.4.3, TS GSM 04.08, section 4.5.3, TS 					GSM 04.08, section 3.5.1.

26.8.1.2.4.7.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a RELEASE will return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

3)	To verify that in releasing the MM-connection, the MS shall wait for  MM layer release initiated by SS.

26.8.1.2.4.7.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.4.8	Outgoing call / U3 MS originating call proceeding / termination requested by the user

26.8.1.2.4.8.1	Definition and applicability

The call control entity of the MS being in the state, U3, the user requests to terminate the call. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.8.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.2.4.8.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.2.4.8.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/3.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The user requests termination of the call. The MS shall send a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1			MMI action, terminate call
	2	MS -> SS	DISCONNECT	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.2.4.9	Outgoing call / U3 MS originating call proceeding / traffic channel allocation

26.8.1.2.4.9.1	Definition and applicability

The call control entity of the MS being in the state, U3, a traffic channel assignment procedure is performed. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.9.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.1.9.

26.8.1.2.4.9.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", when allocated a traffic channel by the network performing the assignment procedure, performs a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.2.4.9.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/3.

Foreseen final state of the MS

U3, mobile originating call proceeding.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	(TCH) the MS shall perform L2 establishment
				on the FACCH
	2	MS -> SS	ASSIGNMENT COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U3

Specific message contents:

None.

###### 26.8.1.2.4.10	Outgoing call / U3 MS originating call proceeding / timer T310 timeout

26.8.1.2.4.10.1	Definition and applicability

The call control entity of the MS being in the state, U3, if no response is then received from the SS, timer T310 expires at the MS side. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.10.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" shall, upon expiry of timer T310, and not before, initiate call release by sending DISCONNECT and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.08 section 5.2.1.3./Abnormal case, TS GSM 04.08 section 5.4.3, TS GSM 04.08 section 11.3.

26.8.1.2.4.10.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" will, upon expiry of timer T310 (accuracy minus 2%, plus 50%), initiate call release by sending DISCONNECT and enter the CC-state U11, "Disconnect Request".

26.8.1.2.4.10.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/3.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The T310 expires at the MS and the MS shall send DISCONNECT. The SS checks timer T310 accuracy and that the CC entity has entered the state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1	SS		the SS waits for T310 timeout
	2	MS -> SS	DISCONNECT	check the timer T310 accuracy
				(minus 2% to plus 50%)
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.2.4.11	Outgoing call / U3 MS originating call proceeding / lower layer failure

26.8.1.2.4.11.1	Definition and applicability

The call control entity of the MS being in the state, U3, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.11.2	Conformance requirement

1)	If a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" has detected a lower layer failure and has returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1.

26.8.1.2.4.11.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" having detected a lower layer failure and having returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.4.11.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U3. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 18-19 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.4.12	Outgoing call / U3 MS originating call proceeding / unknown message received

26.8.1.2.4.12.1	Definition and applicability

The call control entity of the MS being in the state, U3, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.4.12.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.5.

26.8.1.2.4.12.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" having received an unknown message from its peer entity returns a STATUS message.

26.8.1.2.4.12.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/1.

Foreseen final state of the MS

U3, mobile originating call proceeding.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U3
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U3

Specific message contents:

None.

###### 26.8.1.2.4.13	Outgoing call / U3 MS originating call proceeding / Internal alerting indication

26.8.1.2.4.13.1	Definition and applicability

The call control entity of the MS being in the state, U3, an ALERTING message is sent to the MS when the user connection is not attached to the radio path. This test is applicable for any equipment supporting mobile originated circuit switched basic service for telephony.

26.8.1.2.4.13.2	Conformance requirement

1)	When the call control entity of the MS in the "mobile originating call proceeding" state receives an ALERTING message then it shall enter "call delivered" state and, for speech calls, if the user connection is not attached to the radio path, the MS shall internally generate an alerting indication.

References

TS GSM 04.08 section 5.2.1.5.

26.8.1.2.4.13.3	Test purpose

When the call control entity of the MS in the "mobile originating call proceeding" state receives an ALERTING message then it enters "call delivered" state and, for speech calls, if the user connection is not attached to the radio path, the MS generates internally an alerting indication.

26.8.1.2.4.13.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

-	way to give internally generated alerting indication for outgoing calls

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/1.

Foreseen final state of the MS

U4, call delivered.

Maximum duration of test

30 s.

Test procedure

The SS sends an ALERTING message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U4, call delivered. Also it is checked that the MS generates internally alerting indication to the user in the way described in the PICX/PIXIT statements.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ALERTING	the MS shall generate an alerting indication
				to the user in the way described in the
				PICS/PIXIT statements
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

##### 26.8.1.2.5	Outgoing call / U4 call delivered

###### 26.8.1.2.5.1	Outgoing call / U4 call delivered / CONNECT received

26.8.1.2.5.1.1	Definition and applicability

The call control entity of the MS being in the state, U4, a CONNECT message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the CONNECT message shall return a CONNECT ACKNOWLEDGE to its peer entity and enter the CC-state U10, "Active".

References

TS GSM 04.08 section 5.2.1.6.

26.8.1.2.5.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the CONNECT message returns a CONNECT ACKNOWLEDGE to its peer entity and enters the CC-state U10, "Active".

26.8.1.2.5.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/3.

Foreseen final state of the MS

U10, active.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a CONNECT message to the MS. The MS shall respond by sending a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U10, active.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	CONNECT	
	2	MS -> SS	CONNECT ACKNOWLEDGE	MS stops alerting, if applicable
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

###### 26.8.1.2.5.2	Outgoing call / U4 call delivered / termination requested by the user

26.8.1.2.5.2.1	Definition and applicability

The call control entity of the MS being in the state, U4, the user requests to terminate the call. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2.,TS GSM 04.08 section 5.4.3.

26.8.1.2.5.2.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.2.5.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/3.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The user requests termination of the call. The MS shall send a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1			MMI action, terminate call
	2	MS -> SS	DISCONNECT	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.2.5.3	Outgoing call / U4 call delivered / DISCONNECT with in band tones

26.8.1.2.5.3.1	Definition and applicability

The call control entity of the MS being in the state, U4, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered" shall, upon receipt of a DISCONNECT with a progress indicator indicating in-band information, shall through-connect the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

References

TS GSM 04.08 section 3.4.3, TS GSM 04.08 section 5.4.4.1., TS GSM 04.08 section 5.5.1., TS GSM 04.08 section 5.2.1.9.

26.8.1.2.5.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of a DISCONNECT with a progress indicator indicating in-band information, through-connects the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

26.8.1.2.5.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS

U12, disconnect indication.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. The SS checks that if channel mode is MO telephony, the TCH shall be through connected and the MS enters state U12, disconnect indication. If channel mode is not speech, the TCH shall not be through connected and the MS shall enter state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 14)

TCH in speech mode:
	A2	SS		the SS will check that the audio path
				for in band tones is attached.
	A3	SS -> MS	STATUS ENQUIRY	
	A4	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:
	B2	MS -> SS	RELEASE	
	B3	SS -> MS	STATUS ENQUIRY	
	B4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 14:	the Progress Indicator, Progress Description:

#8 in band information or appropriate pattern now available.

###### 26.8.1.2.5.4	Outgoing call / U4 call delivered / DISCONNECT without in band tones

26.8.1.2.5.4.1	Definition and applicability

The call control entity of the MS being in the state, U4, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of a DISCONNECT without progress indicator, shall return a RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.4.

26.8.1.2.5.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of a DISCONNECT without progress indicator, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.5.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.5.5	Outgoing call / U4 call delivered / RELEASE received

26.8.1.2.5.5.1	Definition and applicability

The call control entity of the MS being in the state, U4, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the RELEASE message shall respond with the RELEASE COMPLETE message and enter the CC-state U0, "Null".

2)	The MS on returning to idle mode shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.2., TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.2.5.5.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the RELEASE message will respond with the RELEASE COMPLETE message and enter the CC-state U0, "Null"

2)	To verify that the MS on returning the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.5.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 19-20 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.5.6	Outgoing call / U4 call delivered / lower layer failure

26.8.1.2.5.6.1	Definition and applicability

The call control entity of the MS being in the state, U4, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.6.2	Conformance requirement

1)	When CC-entity of the MS in CC-state U4, "Call Delivered" has detected a lower layer failure and has returned to idle mode, the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1.

26.8.1.2.5.6.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered" having detected a lower layer failure and has returned to idle mode, the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.5.6.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U4. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.5.7	Outgoing call / U4 call delivered / traffic channel allocation

26.8.1.2.5.7.1	Definition and applicability

The call control entity of the MS being in the state, U4, a traffic channel assignment procedure is performed. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.1.9.

26.8.1.2.5.7.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.2.5.7.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/1.

Foreseen final state of the MS

U4, call delivered.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH, the MS shall perform L2 establishment
				on the FACCH
	2	MS -> SS	ASSIGNMENT COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

###### 26.8.1.2.5.8	Outgoing call / U4 call delivered / unknown message received

26.8.1.2.5.8.1	Definition and applicability

The call control entity of the MS being in the state, U4, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.5.8.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.2.5.8.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.2.5.8.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/4.

Foreseen final state of the MS

U4, call delivered.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U4
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

##### 26.8.1.2.6	U10 call active

###### 26.8.1.2.6.1	U10 call active / termination requested by the user

26.8.1.2.6.1.1	Definition and applicability

The call control entity of the MS being in the state, U10, the user requests to terminate the call. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.2.6.1.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U10, "Call Active", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.2.6.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/1.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The user requests termination of the call. The MS shall send a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1			MMI action, terminate call
	2	MS -> SS	DISCONNECT	U11
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.2.6.2	U10 call active / RELEASE received

26.8.1.2.6.2.1	Definition and applicability

The call control entity of the MS being in the state, U10, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon receipt of the RELEASE shall respond with the RELEASE COMPLETE message and enter the CC-state U0, "Null"

2)	When the MS returns to the idle mode it shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null"

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.2., TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.2.6.2.3	Test purpose

1)	To verify that the a CC-entity of the MS in CC-state U10, "Call Active", upon receive of the RELEASE will respond with the RELEASE COMPLETE message and enter the CC-state U0, "Null"

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null"

26.8.1.2.6.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	the MS starts T3240
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA)

Specific message contents:

None.

###### 26.8.1.2.6.3	U10 call active / DISCONNECT with in band tones

26.8.1.2.6.3.1	Definition and applicability

The call control entity of the MS being in the state, U10, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message with a Progress Indicator indicating in-band information, shall through-connect the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

References

TS GSM 04.08 section 5.4.4.1., TS GSM 04.08 section 5.5.1.

26.8.1.2.6.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message with a Progress Indicator indicating in-band information, through-connects the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS sends a RELEASE message.

26.8.1.2.6.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/2.

Foreseen final state of the MS

U12, disconnect indication.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. The SS checks that if channel mode is speech, the TCH shall be through connected and the MS enters state U12, disconnect indication. If channel mode is not speech, the TCH shall not be through connected and the MS enters state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 15)

TCH in speech mode:
	A2	SS		the SS will check that the audio path
				for in band tones is attached.
	A3	SS -> MS	STATUS ENQUIRY	
	A4	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:
	B2	MS -> SS	RELEASE	
	B3	SS -> MS	STATUS ENQUIRY	
	B4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 15:	the Progress Indicator, Progress Description:

#8 in band information or appropriate pattern now available.

###### 26.8.1.2.6.4	U10 call active / DISCONNECT without in band tones

26.8.1.2.6.4.1	Definition and applicability

The call control entity of the MS being in the state, U10, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message without progress indicator, shall return a RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.4.

26.8.1.2.6.4.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message without progress indicator, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.6.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/2.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.6.5	U10 call active / RELEASE COMPLETE received

26.8.1.2.6.5.1	Definition and applicability

The call control entity of the MS being in the state, U10, the call is cleared by a RELEASE COMPLETE message sent by the SS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.5.2	Conformance requirement

1)	A CC entity of the MS in CC-state U10, "active", upon receipt of a RELEASE COMPLETE message with valid cause value, shall enter CC state U0, "Null".

2)	On returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08, section 5.4.2, TS GSM 04.08, section 5.4.4.

Conformance requirement 2:	TS GSM 04.08, section 5.4.4.3.

26.8.1.2.6.5.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U10, "Call active" upon receipt of a RELEASE COMPLETE message with valid cause value, enters CC state U0, "Null".

2)	To verify that in returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.6.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

30 s.

Test procedure

The SS sends a RELEASE COMPLETE message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE COMPLETE	note 16
	2	SS -> MS	STATUS ENQUIRY	note 17
	3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value), 
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000...110
	5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

NOTE 16:	With the cause value chosen arbitrarily.

NOTE 17:	TI flag has the value indicating the MS as a originator of the call.

###### 26.8.1.2.6.6	U10 call active / SETUP received

26.8.1.2.6.6.1	Definition and applicability

If the MS does not react correctly when receiving a SETUP message on a new Transaction Identifier during an active call, the active call may be lost.

This test is applicable for all GSM900 and DCS1800 MS supporting at least one mobile originated circuit switched basic service.

26.8.1.2.6.6.2	Conformance requirement

1)	A Mobile Station that has a call established when receiving a SETUP message shall respond either with a CALL CONFIRMED message or a RELEASE COMPLETE message, both with cause #17 "user busy".

2)	The call control state of the existing transaction shall not be affected by the incoming SETUP message.

Reference(s):

Conformance requirement 1:	TS GSM 04.08, section 5.2.2.3.1.

Conformance requirement 2:	TS GSM 04.08, section 5.1.1.

26.8.1.2.6.6.3	Test purpose

1)	To verify that a Mobile Station that has a call established and receives a SETUP message answers either with a CALL CONFIRMED message with cause "user busy" if it supports call waiting, or with a RELEASE COMPLETE message with cause "user busy" otherwise.

2)	To verify that after having sent this message, the MS is still in state U10 for the established call.

26.8.1.2.6.6.4	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is idle updated with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/14.

Related PICS/PIXIT statement(s)

- supported MO circuit switched basic services

- support of call waiting Y/N

Foreseen final state of the MS

U10, call active.

Maximum duration of test

30 s.

Test Procedure

The MS has a mobile originated call in the U10 state.

The SS sends a SETUP message to the MS (with signal IE indicating "call waiting tone on").

If the MS does not support call waiting it shall answer by a RELEASE COMPLETE message.

If the MS supports call waiting it shall answer by a CALL CONFIRMED message followed by an ALERTING. The second transaction is then released by the SS with a RELEASE COMPLETE message.

In both cases the SS checks by using the status enquiry procedure that the CC entity of the MS is still in state U10, active call for the original call.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	SETUP	this message establishes a second transaction
				The TI value shall be the same as the
				one that is in use for the MO call.
				The TI flag shall have the value
				specified for an MT call.

if the MS does not support call waiting
	A2	MS -> SS	RELEASE COMPLETE	with cause user busy"
				with the TI of the second transaction

if the MS supports call waiting
	B2	MS -> SS	CALL CONFIRMED	with cause user busy"
				with the TI of the second transaction
	B3	MS -> SS	ALERTING	with the TI of the second transaction
	B4	SS -> MS	RELEASE COMPLETE	with the TI of the second transaction

5	SS -> MS	STATUS ENQUIRY	with the TI of the original transaction
	6	MS -> SS	STATUS	cause 30#, state U10
				with the TI of the original transaction

NOTE:	The Transaction Identifier of the second transaction shall be different from the one of the already established transaction.

Specific message contents

SETUP message contains a Signal IE with value "call waiting tone on" (H'07).

##### 26.8.1.2.7	U11 disconnect request

###### 26.8.1.2.7.1	U11 disconnect request / clear collision

26.8.1.2.7.1.1	Definition and applicability

The call control entity of the MS being in the state, U11, a DISCONNECT message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.7.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of a DISCONNECT message, shall return to its peer entity the RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.5.

26.8.1.2.7.1.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of a DISCONNECT message, returns to its peer entity the RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.7.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/3.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. The SS sends a DISCONNECT message to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.7.2	U11 disconnect request / RELEASE received

26.8.1.2.7.2.1	Definition and applicability

The call control entity of the MS being in the state, U11, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.7.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of the RELEASE message shall return RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.3.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.2.7.2.3	Test purpose

1)	To verify that the a CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of the RELEASE message shall return RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.7.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/3.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.7.3	U11 disconnect request / timer T305 timeout

26.8.1.2.7.3.1	Definition and applicability

The call control entity of the MS being in the state, U11, if no response is then received from the SS, timer T305 expires at the MS side. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.7.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request" shall on expiry of T305, proceed with the connection release procedure by sending the RELEASE message to its peer entity and shall enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.3., TS GSM 04.08 section 11.3.

26.8.1.2.7.3.3	Test purpose

To verify that the CC-entity of the MS in CC-state U11, "Disconnect Request" shall on expiry of T305 (accuracy +/- 10%), proceed with the connection release procedure by sending the RELEASE message to its peer entity and enters the CC-state U19, "Release Request".

26.8.1.2.7.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/3.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

1 minute.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. Then T305 expires at the MS and the MS shall send a RELEASE message. The SS checks timer T305 accuracy and that the CC entity has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS waits until T305 expires at the MS
	2	MS -> SS	RELEASE	SS checks the time between DISCONNECT
				and RELEASE (note 18),
				(T305 +/- 10%)
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 18:	With the same cause value as originally contained in the DISCONNECT message. An additional cause information element (#102 recovery on timer expiry) may be included.

###### 26.8.1.2.7.4	U11 disconnect request / lower layer failure

26.8.1.2.7.4.1	Definition and applicability

The call control entity of the MS being in the state, U11, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.7.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request" having detected a lower layer failure shall return to the idle mode. The CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1., TS GSM 04.08 section 8.3.

26.8.1.2.7.4.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U11, "Disconnect Request" having detected a lower layer failure returns to the idle mode. The CC entities relating to the seven mobile originating transaction identifiers are thus in state U0, "Null".

26.8.1.2.7.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U11. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.7.5	U11 disconnect request / unknown message received

26.8.1.2.7.5.1	Definition and applicability

The call control entity of the MS being in the state, U4, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.7.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U100, "Call Delivered", having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.2.7.5.3	Test purpose

To verify that a CC-entity of the MS in CC-state U11, "Call Delivered", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.2.7.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/4.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U11
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

##### 26.8.1.2.8	U12 disconnect indication

###### 26.8.1.2.8.1	U12 disconnect indication / call releasing requested by the user

26.8.1.2.8.1.1	Definition and applicability

The call control entity of the MS being in the state, U12, the user requests to terminate the call. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U12, "Disconnect Indication" being in network initiated call release phase, shall, upon receiving a call release request from the user send a RELEASE to its peer entity and enter CC-state U19, "Release Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.4.

26.8.1.2.8.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication" being in network initiated call release phase, shall, upon receiving a call release request from the user sends a RELEASE to its peer entity and enters CC-state U19, "Release Request"

26.8.1.2.8.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using Option A of table 26.8.1.2/1.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U12. The user requests termination of the call. The MS shall send a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1			MMI action, "on hook"
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.8.2	U12 disconnect indication / RELEASE received

26.8.1.2.8.2.1	Definition and applicability

The call control entity of the MS being in the state, U12, a RELEASE message is received by the MS. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U12, "Disconnect Indication", upon receipt of a RELEASE message shall return to its peer entity the RELEASE COMPLETE message and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.2

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.2.8.2.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication", upon receipt of a RELEASE message returns to its peer entity the RELEASE COMPLETE message and enters the CC-state U0, "Null"

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.8.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using Option A of table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U12. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.8.3	U12 disconnect indication / lower layer failure

26.8.1.2.8.3.1	Definition and applicability

The call control entity of the MS being in the state, U12, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U12, "Disconnect Indication" having detected a lower layer failure shall return to idle mode. The CC-entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

References

TS GSM 04.08 section 4.5.3.2., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1., TS GSM 04.08 section 8.3.

26.8.1.2.8.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication" having detected a lower layer failure returns to idle mode. The CC-entities relating to the seven mobile originating transaction identifiers are thus in state U0, "Null".

26.8.1.2.8.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using Option A of table 26.8.1.2/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U12. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.8.4	U12 disconnect indication / unknown message received

26.8.1.2.8.4.1	Definition and applicability

The call control entity of the MS being in the state, U12, an unknown message is received by the MS. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.4.2	Conformance requirement

A CC-entity of the MS in CC-state U12, "Disconnect Indication" having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.2.8.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication" having received an unknown message from its peer entity returns a STATUS message.

26.8.1.2.8.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using Option A of table 26.8.1.2/3.

Foreseen final state of the MS

U12, disconnect indication.

Maximum duration of test

30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U12. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U12
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U12

Specific message contents:

None.

##### 26.8.1.2.9	Outgoing call / U19 release request

###### 26.8.1.2.9.1	Outgoing call / U19 release request / timer T308 timeout

26.8.1.2.9.1.1	Definition and applicability

The call control entity of the MS being in the state, U19, if no response is then received from the SS, timer T308 expires at the MS side. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.9.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request" will, upon the first expiry of timer T308 send the RELEASE message to its peer entity and remain in the CC-state U19.

References

TS GSM 04.08 section 5.4.4.3.1, TS GSM 04.08 section 11.3.

26.8.1.2.9.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request" will, upon the first expiry of timer T308 (accuracy +/- 10%) send the RELEASE message to its peer entity and remain in the CC-state U19.

26.8.1.2.9.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/4.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

1 min.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. When T308 expires at the MS, the MS shall send a RELEASE message. The SS checks timer T308 accuracy and that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS waits until T308 at the MS
	2	MS -> SS	RELEASE	SS checks the time between the two
				RELEASE messages (T308 +/- 10%)
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.2.9.2	Outgoing call / U19 release request / 2nd timer T308 timeout

26.8.1.2.9.2.1	Definition and applicability

The call control entity of the MS being in the state, U19, if no response is then received after timer T308 has expired two times in success at the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.9.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request", upon the 2nd expiry of the timer T308, shall enter the CC-state U0, "Null".

2)	Subsequently the MS shall proceed with releasing the MM-connection and enter the idle mode with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.3.1., TS GSM 04.08 section 11.3.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.2.9.2.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U19, "Release Request", upon the 2nd expiry of the timer T308, enters the CC-state U0, "Null".

2)	To verify that subsequently the MS proceeds with releasing the MM-connection and enters the idle mode with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

26.8.1.2.9.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

2 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. The SS allows T308 expiry at the MS, and the MS shall repeat sending the RELEASE message and start timer T308 again. The SS allows again T308 expiry at the MS. The MS shall abort the RR-connection (DISC/UA). The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS waits until T308 expiry at the MS
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19
	5	SS		SS waits until the second T308 expiry
				at the MS
	6	SS		SS waits T3240 expiry at the MS
	7	MS		the main signalling link shall be
				released by the MS (L2: DISC/UA).
	8	SS		SS waits 10 s for the MS to return to
				listening to paging
	9	SS -> MS	PAGING REQUEST	
	10	MS -> SS	CHANNEL REQUEST	
	11	SS -> MS	IMMEDIATE ASSIGNMENT	
	12	MS -> SS	PAGING RESPONSE	
	13	SS -> MS	STATUS ENQUIRY	
	14	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	15	SS		repeat steps 13-14 to cover all the
				transaction identifiers from 000...110
	16	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.9.3	Outgoing call / U19 release request / RELEASE received

26.8.1.2.9.3.1	Definition and applicability

The call control entity of the MS being in the state, U19, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.9.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE, shall release the MM-connection and enter the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

References

TS GSM 04.08 section 5.4.5., TS GSM 04.08 section 11.3, TS GSM 04.08 section 5.5.3.2.

26.8.1.2.9.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE, shall release the MM-connection and enters the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

26.8.1.2.9.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. The SS sends a RELEASE message to the MS. The MS shall release the MM-connection. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	(note 19)
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000...110
	5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

NOTE 19:	With the same cause number as originally contained in DISC and optional cause #102 recovery on timer expiry.

###### 26.8.1.2.9.4	Outgoing call / U19 release request / RELEASE COMPLETE received

26.8.1.2.9.4.1	Definition and applicability

The call control entity of the MS being in the state, U19, a RELEASE COMPLETE message is received by the MS. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.9.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE COMPLETE, shall release the MM-connection and enter the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

References

TS GSM 04.08 section 5.4.4.3., TS GSM 04.08 section 4.5.3, TS GSM 04.08 section 8.3.

26.8.1.2.9.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE COMPLETE, shall release the MM-connection and enters the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

26.8.1.2.9.4.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. The SS sends a RELEASE COMPLETE message to the MS. The MS shall release the MM-connection. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE COMPLETE	
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000...110
	5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.2.9.5	Outgoing call /  U19 release request / lower layer failure

26.8.1.2.9.5.1	Definition and applicability

The call control entity of the MS being in the state, U19, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.1.2.9.5.2	Conformance requirement

A CC-entity of the MS in CC-state U19, "Release Request", having detected a lower layer failure, shall return to the idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.2.9.5.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request", having detected a lower layer failure, returns to the idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.9.5.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MO circuit switched basic service is selected that is supported by the MS; if the MS supports MO telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then, the MS is made to initiate a call. The MS is brought to the state U19. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

#### 26.8.1.3	Establishment of an incoming call / Initial conditions

The tables below describe message exchanges which bring the MS in the requested initial states in case of an incoming call.

A state may be taken as initial only when all the states which lead to this initial states have been validated. The order will be U0, U6, U9, U7, U8, U10, U26 etc. as in the following tables.

Table 26.8.1.3/1: Establishment of an incoming call, procedure 1

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, SDCCH
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	AUTH REQUEST	
	6	MS -> SS	AUTH RESPONSE	
	7	SS -> MS	CIPH MODE COMMAND	
	8	MS -> SS	CIPH MODE COMPLETE	
	9	SS -> MS	SETUP	U6, (note 20)
	10	MS -> SS	CALL CONFIRMED	U9

A11	MS -> SS	CONNECT	U8, p = Y, (note 21)

B11	MS -> SS	ALERTING	U7, p = N, (note 21)
	B12	MS		(note 22)
	B13	MS -> SS	CONNECT	U8

14	SS -> MS	ASSIGNMENT COMMAND	TCH
	15	MS -> SS	ASSIGNMENT COMPLETE	
	16	SS -> MS	CONNECT ACKNOWLEDGE	U10

NOTE 20:	With signal information included in the SETUP message.

NOTE 21:	The MS is supporting immediate connect (p = Y/N). See PICS/PIXIT statement.

NOTE 22:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

Table 26.8.1.3/2: Establishment of an incoming call, procedure 2

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, SDCCH
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	CIPH MODE COMMAND	
	6	MS -> SS	CIPH MODE COMPLETE	
	7	SS -> MS	SETUP	U6, (note 23)
	8	MS -> SS	CALL CONFIRMED	U9

A9	MS -> SS	CONNECT	U8, p = Y, (note 24)
	A10	SS -> MS	ASSIGNMENT COMMAND	TCH
	A11	MS -> SS	ASSIGNMENT COMPLETE

B9	MS -> SS	ALERTING	U7, p = N, (note 24)
	B10	SS -> MS	ASSIGNMENT COMMAND	TCH
	B11	MS -> SS	ASSIGNMENT COMPLETE	
	B12	MS		(note 25)
	B13	MS -> SS	CONNECT	U8

14	SS -> MS	AUTH REQUEST	
	15	MS -> SS	AUTH RESPONSE	
	16	SS -> MS	CONNECT ACKNOWLEDGE	U10

NOTE 23:	With signal information included in the SETUP message.

NOTE 24:	The MS is supporting immediate connect (p = Y/N). See PICS/PIXIT statement.

NOTE 25:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

Table 26.8.1.3/3: Establishment of an incoming call, procedure 3

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, TCH
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	AUTH REQUEST	
	6	MS -> SS	AUTH RESPONSE	
	7	SS -> MS	CIPH MODE COMMAND	
	8	MS -> SS	CIPH MODE COMPLETE	
	9	SS -> MS	CHANNEL MODE MODIFY	(note 26)
	10	MS -> SS	CHANNEL MODE MOD ACK	
	11	SS -> MS	SETUP	U6, (note 27)
	12	MS -> SS	CALL CONFIRMED	U9

A13	MS -> SS	CONNECT	U8, p = Y, (note 28)

B13	MS -> SS	ALERTING	U7, p = N, (note 28)
	B14	MS		(note 29)
	B15	MS -> SS	CONNECT	U8

16	SS -> MS	CONNECT ACKNOWLEDGE	U10

NOTE 26:	Assigned channel is appropriate for the chosen mobile originated circuit switched basic service.

NOTE 27:	With signal information included in the SETUP message.

NOTE 28:	The MS is supporting immediate connect (p = Y/N). See PICS/PIXIT statement.

NOTE 29:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

Table 26.8.1.3/4: Establishment of an incoming call, procedure 4

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, SDCCH
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	CIPH MODE COMMAND	
	6	MS -> SS	CIPH MODE COMPLETE	
	7	SS -> MS	SETUP	U6, (note 30)
	8	MS -> SS	CALL CONFIRMED	U9
	9	SS -> MS	ASSIGNMENT COMMAND	TCH
	10	MS -> SS	ASSIGNMENT COMPLETE

A11	MS -> SS	CONNECT	U8, p = Y, (note 31)

B11	MS -> SS	ALERTING	U7, p = N, (note 31)
	B12	MS		(note 32)
	B13	MS -> SS	CONNECT	U8

14	SS -> MS	AUTH REQUEST	
	15	MS -> SS	AUTH RESPONSE	
	16	SS -> MS	CONNECT ACKNOWLEDGE	U10

NOTE 30:	The signal information element is not included in the SETUP message.

NOTE 31:	The MS is supporting immediate connect (p = Y/N). See PICS/PIXIT statement.

NOTE 32:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

##### 26.8.1.3.1	Incoming call / U0 null state

###### 26.8.1.3.1.1	Incoming call / U0 null state / SETUP received with a non supported bearer capability

26.8.1.3.1.1.1	Definition and applicability

The call control entity of the MS being in the state, U0, a SETUP message is received with only one bearer capability and this bearer capability is not supported by the MS. This test is applicable for all equipment.

26.8.1.3.1.1.2	Conformance requirement

1)	A CC entity of the MS, upon receipt of SETUP containing one bearer capability and this bearer capability is not supported, shall return a RELEASE COMPLETE with correct cause value to its peer entity and return to the idle mode. The CC-entities relating to the seven mobile terminating transaction identifiers shall be in the state U0,"Null".

References

TS GSM 04.08 section 5.2.2.2., TS GSM 04.08 Annex B.

26.8.1.3.1.1.3	Test purpose

To verify that a CC entity of the MS, upon receipt of SETUP containing one bearer capability and this bearer capability is not supported, returns a RELEASE COMPLETE with correct cause value to its peer entity, and returns to the idle mode. To verify that the CC-entities relating to the seven mobile terminating transaction identifiers are then in the state U0,"Null".

26.8.1.3.1.1.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

U0, null.

Maximum duration of test

30 s.

Test procedure

A mobile terminated call is initiated. The MS receives a SETUP message that contains a bearer capability not supported by the MS. The MS returns a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity is still in the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	SS sends paging
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	(SDCCH)	
	4	MS -> SS	PAGING RESPONSE	
	5	SS -> MS	AUTH REQUEST	
	6	MS -> SS	AUTH RESPONSE	
	7	SS -> MS	CIPH MODE COMMAND	
	8	MS -> SS	CIPH MODE COMPLETE	
	9	SS -> MS	SETUP	(note 33)
	10	MS -> SS	RELEASE COMPLETE	(note 34)
	11	SS -> MS	STATUS ENQUIRY
	12	MS ->SS	RELEASE COMPLETE	Cause 81# (invalid TI value).
	13	SS		Repeat steps 11-12 to cover all the
				transaction identifiers from 000... 110.

Specific message contents:

NOTE 33:	With one bearer capability and that bearer capability is not supported by the MS

NOTE 34:	With cause #88 incompatible destination.

##### 26.8.1.3.2.	Incoming call / U6 call present

###### 26.8.1.3.2.1	Incoming call / U6 call present / automatic call rejection

26.8.1.3.2.1.1	Definition and applicability

Although the state U6 is transient, the ability to refuse a call (automatically) in this state is tested, if it is implemented at the MS. The test is applicable for those equipments described above supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.2.1.2	Conformance requirement

1)	A CC entity of the MS in CC-state U6, "Call Present", upon receipt of a rejection indication of the incoming call from the user, send RELEASE COMPLETE with the appropriate cause value to its peer entity and enter the CC-state U0, "Null". The CC entities relating to the seven mobile terminating transaction identifiers shall be in state U0, "Null".

References

TS GSM 11.10, Annex B (for PICS/PIXIT statement), TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.2.2.3.1., TS GSM 04.08 section 5.5.3.2, TS GSM 04.08 section 8.3.

26.8.1.3.2.1.3	Test purpose

To verify that a CC entity of the MS in CC-state U6, "Call Present", shall upon receipt of a rejection indication of the incoming call from the user, shall send RELEASE COMPLETE with the appropriate cause value to its peer entity and enter the CC-state U0, "Null". The CC entities relating to the seven mobile terminating transaction identifiers are then in state U0, "Null".

26.8.1.3.2.1.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	the MS supports an ability to refuse a call after receipt of a SETUP message

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U6 by using table 26.8.1.3/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The call control entire of the MS is brought to the state U6 (Note: The state U6 is not checked, since it is not stable). The MS is made to refuse the call (the refusal may require some preliminary preparations in order to achieve refusal at this point). The MS shall send a RELEASE COMPLETE message and enter a call control state U0. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1			the MS is made to refuse the call
	2	MS -> SS	RELEASE COMPLETE	(note 35)
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

NOTE 35:	With cause value #21 call rejected.

##### 26.8.1.3.3	Incoming call / U9 mobile terminating call confirmed

###### 26.8.1.3.3.1	Incoming call / U9 mobile terminating call confirmed / alerting or immediate connecting

26.8.1.3.3.1.1	Definition and applicability

The call control entity of the MS having entered the state, U9, with signal information received in the preceding SETUP message, the subsequent behaviour of the MS is tested. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.3.1.2	Conformance requirement

1)	A CC entity in CC-state U9, "MS Terminating Call Confirmed", (if signalled by the network in previous SETUP message that it may alert) shall either send a ALERTING message to its peer entity and enter state U7, or send a CONNECT message to its peer entity and enter U8.

References

TS GSM 04.08 section 5.2.2.3.2., TS GSM 11.10, Annex 3 - section 2.2.

26.8.1.3.3.1.3	Test purpose

To verify that a CC entity in CC-state U9, "MS Terminating Call Confirmed", (if signalled by the network in previous SETUP message that it may alert) will either send a ALERTING message to its peer entity and enter state U7, or send a CONNECT message to its peer entity and enter U8.

26.8.1.3.3.1.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/2.

Foreseen final state of the MS

-	U8, connect request, if the MS supports immediate connect for the selected basic service

-	otherwise U7, call received

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9 by using a SETUP message containing signalling information element. (The state U9 is not a stable state in this case, and consequently it is not checked as an initial state.) If the MS supports immediate connect for the selected basic service (p = Y), it sends a CONNECT message and enters the state U8, connect request. Otherwise (p = N) the MS sends an ALERTING message and enters the state U7, call receiving. The SS checks by using the status enquiry procedure that the CC entity has entered its state as described.

Expected sequence

Step	Direction	Message	Comments

A11	MS -> SS	CONNECT	p = Y
	A12	SS -> MS	STATUS ENQUIRY	
	A13	MS -> SS	STATUS	cause 30#, state U8

B11	MS -> SS	ALERTING	p = N
	B12	SS -> MS	STATUS ENQUIRY	
	B13	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

###### 26.8.1.3.3.2	Incoming call / U9 mobile terminating call confirmed / TCH assignment

26.8.1.3.3.2.1	Definition and applicability

The call control entity of the MS being in the state, U9, an assignment procedure is performed for traffic channel. This test is applicable for any equipment supporting at least one MT circuit switched basic service, for which immediate connect is not used.

26.8.1.3.3.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH, send a ALERTING message and enter state U7.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.2.7., TS GSM 04.08 section 5.2.2.3.2.

26.8.1.3.3.2.3	Test purpose

To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", when allocated a traffic channel by the network performing the assignment procedure, performs a layer 2 establishment on the FACCH, sends a ALERTING message and enters state U7.

26.8.1.3.3.2.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U9, mobile terminating call confirmed.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9 (by using a SETUP message not containing the signal information element). The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The MS sends an ALERTING message and enters state U7, call received. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH, an appropriate non-signalling mode
	2	MS		the MS shall establish L2 link
	3	MS -> SS	ASSIGNMENT COMPLETE	
	4	MS -> SS	ALERTING	
	5	SS -> MS	STATUS ENQUIRY	
	6	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

###### 26.8.1.3.3.3	Incoming call / U9 mobile terminating call confirmed / termination requested by the user

26.8.1.3.3.3.1	Definition and applicability

The call control entity of the MS being in the state, U9, the user requests for releasing of the call. This test is applicable for any equipment supporting at least one MT circuit switched basic service for which immediate connection is not used and, in addition to this, the facility to send a DISCONNECT message in state U9.

26.8.1.3.3.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.3.3.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.3.3.3.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

-	the MS supports user requested call clearing in the state U9

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9 (by using a SETUP message not containing the signal information element). Then the user requests termination of the call, if possible. The MS sends a DISCONNECT message and enters state U11, disconnect request. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence

Step	Direction	Message	Comments

1			the MS is made to clear the call
	2	MS -> SS	DISCONNECT	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.3.3.4	Incoming call / U9 mobile terminating call confirmed / DISCONNECT received

26.8.1.3.3.4.1	Definition and applicability

The call control entity of the MS being in the state, U9, a DISCONNECT message is received by the MS. This test is applicable for any equipment supporting at least one MT circuit switched basic service, for which immediate connect is not used.

26.8.1.3.3.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a DISCONNECT shall return a RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.4.

26.8.1.3.3.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a DISCONNECT returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.3.3.4.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9. The SS sends a DISCONNECT message to the MS. The MS responds by sending a RELEASE message and enters state U19, release request. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

###### 26.8.1.3.3.5	Incoming call / U9 mobile terminating call confirmed / RELEASE received

26.8.1.3.3.5.1	Definition and applicability

The call control entity of the MS being in the state, U9, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one MT circuit switched basic service, for which immediate connect is not used.

26.8.1.3.3.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a RELEASE shall return a RELEASE COMPLETE and enter the CC-state U0, "Null".

TS GSM 04.08 section 5.4.4.

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile terminating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS 
				      GSM 04.08 section 8.3.

26.8.1.3.3.5.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a RELEASE will return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile terminating transaction identifiers are in CC-state U0, "Null".

26.8.1.3.3.5.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9. The SS sends a RELEASE message to the MS. The MS responds by sending a RELEASE COMPLETE message and enters state U0, null. The SS verifies by using the status enquiry procedure that the MS has entered the correct state with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.3.6	Incoming call / U9 mobile terminating call confirmed / lower layer failure

26.8.1.3.3.6.1	Definition and applicability

The call control entity of the MS being in the state, U9, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one MT circuit switched basic service, for which immediate connect is not used.

26.8.1.3.3.6.2	Conformance requirement

1)	A CC entity of the MS in CC-state U9, "MS Terminating Call Confirmed", having detected a lower layer failure shall return to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3

26.8.1.3.3.6.3	Test purpose

To verify that a CC entity of the MS in CC-state U9, "MS Terminating Call Confirmed", having detected a lower layer failure returns to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

26.8.1.3.3.6.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The MS is brought to the state U9. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.3.7	Incoming call / U9 mobile terminating call confirmed / unknown message received

26.8.1.3.3.7.1	Definition and applicability

The call control entity of the MS being in the state, U9, an unknown message is received by the MS. This test is applicable for any equipment supporting at least MT circuit switched basic service, for which immediate connect is not used.

26.8.1.3.3.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed" having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.3.3.7.3	Test purpose

To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed" having received an unknown message from its peer entity returns a STATUS message.

26.8.1.3.3.7.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS

U9, mobile terminating call proceeding.

Maximum duration of test

30 s.

Test procedure

A MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U9
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U9

Specific message contents:

None.

##### 26.8.1.3.4.	Incoming call / U7 call received

###### 26.8.1.3.4.1	Incoming call / U7 call received / call accepted

26.8.1.3.4.1.1	Definition and applicability

The call control entity of the MS being in the state, U7, a user accepts the incoming call. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.1.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon a user accepting the incoming call, shall send a CONNECT message to its peer entity and enter the CC-state U8, "Connect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.2.2.5.

26.8.1.3.4.1.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", upon a user accepting the incoming call, shall send a CONNECT message to its peer entity and enter the CC-state U8, "Connect Request"

26.8.1.3.4.1.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/3.

Foreseen final state of the MS

U8, connect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The user accepts the incoming call. The MS sends a CONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered state U8, connect request.

Expected sequence

Step	Direction	Message	Comments

1			the MS is made to accept the call by the user
	2	MS -> SS	CONNECT	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U8

Specific message contents:

None.

###### 26.8.1.3.4.2	Incoming call / U7 call received / termination requested by the user

26.8.1.3.4.2.1	Definition and applicability

The call control entity of the MS being in the state, U7, a user requests to terminate incoming call. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.2.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.3.4.2.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.3.4.2.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/3.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The user initiates clearing the incoming call. The MS sends a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered state U11, disconnect request.

Expected sequence

Step	Direction	Message	Comments

1			the MS is made to terminate/reject the call
	2	MS -> SS	DISCONNECT	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.3.4.3	Incoming call / U7 call received / DISCONNECT received

26.8.1.3.4.3.1	Definition and applicability

The call control entity of the MS being in the state, U7, a DISCONNECT message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.3.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon receipt of a DISCONNECT with a progress indicator indicating in-band information from network, if a TCH was not assigned, shall return a RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.4.

26.8.1.3.4.3.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", upon receipt of a DISCONNECT with a progress indicator indicating in-band information from network, if a TCH was not assigned, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.3.4.3.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/1.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends a DISCONNECT message. The MS responds with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity has entered state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 36)
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 36:	With a progress indicator indicating in-band information; Progress Indicator, Progress Description #8.

###### 26.8.1.3.4.4	Incoming call / U7 call received / RELEASE received

26.8.1.3.4.4.1	Definition and applicability

The call control entity of the MS being in the state, U7, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.4.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon receipt of a RELEASE shall return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile terminating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.3.4.4.3	Test purpose

1)	To verify that a CC entity of a MS in CC-state U7, "Call Received", upon receipt of a RELEASE will return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile terminating transaction identifiers are in CC-state U0, "Null".

26.8.1.3.4.4.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends a RELEASE message. The MS responds with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered state U0, null, with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.4.5	Incoming call / U7 call received / lower layer failure

26.8.1.3.4.5.1	Definition and applicability

The call control entity of the MS being in the state, U7, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.5.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", having detected a lower layer failure shall return to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.3.4.5.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", having detected a lower layer failure returns to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

26.8.1.3.4.5.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/2.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The MS is brought to the state U7. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.4.6	Incoming call / U7 call received / unknown message received

26.8.1.3.4.6.1	Definition and applicability

The call control entity of the MS being in the state, U7, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.6.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.3.4.6.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.3.4.6.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/3.

Foreseen final state of the MS

U7, call received.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U7
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

###### 26.8.1.3.4.7	Incoming call / U7 call received / TCH assignment

26.8.1.3.4.7.1	Definition and applicability

The call control entity of the MS being in the state, U7, an assignment procedure is performed for traffic channel. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service for which immediate connect is not used.

26.8.1.3.4.7.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.2.7.

26.8.1.3.4.7.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.3.4.7.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/1.

Foreseen final state of the MS

U7, call received.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected basic service is telephony. If necessary, the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS verifies by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH
	2	MS		the MS shall establish L2 link
	3	MS -> SS	ASSIGNMENT COMPLETE	
	4	SS -> MS	STATUS ENQUIRY	
	5	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

###### 26.8.1.3.4.8	Incoming call / U7 call received / RELEASE COMPLETE received

26.8.1.3.4.8.1	Definition and applicability

The call control entity of the MS being in the state, U7, the call is cleared by a RELEASE COMPLETE message sent by the SS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service, for which immediate connect is not used.

26.8.1.3.4.8.2	Conformance requirement

1)	A CC entity of the MS in CC-state U7, "call received", upon receipt of a RELEASE COMPLETE message with valid cause value, shall enter CC state U0, "Null".

2)	On returning to idle mode, the CC entities relating to the seven mobile terminating transaction identifiers shall be in state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08, section 5.4.2, TS GSM 04.08, section 5.4.4.

Conformance requirement 2:	TS GSM 04.08, section 5.4.4.3.

26.8.1.3.4.8.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U7, "Call received", upon receipt of a RELEASE COMPLETE message with valid cause value, enters CC state U0, "Null".

2)	To verify that in returning to idle mode, the CC entities relating to the seven mobile terminating transaction identifiers are in state U0, "Null".

26.8.1.3.4.8.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services.

MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS and for which the MS does not use immediate connection; if the MS supports MT telephony without immediate connection, the selected service is telephony. If necessary, the MS is configured for that basic service. The a mobile terminated call is initiated. the CC entity of the MS is brought to U7. The SS sends a RELEASE COMPLETE message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE COMPLETE	note 37
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value), note 38
	4	SS		repeat steps 2-3 to cover all the
				transaction identifiers from 000...110
	5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

NOTE 37:	With the cause value chosen arbitrarily.

NOTE 38:	TI flag has the value indicating the SS as a originator of the call.

##### 26.8.1.3.5	Incoming call / U8 connect request

###### 26.8.1.3.5.1	Incoming call / U8 connect request / CONNECT acknowledged

26.8.1.3.5.1.1	Definition and applicability

The call control entity of the MS being in the state, U8, a CONNECT ACKNOWLEDGE message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.1.2	Conformance requirement

A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of CONNECT ACKNOWLEDGE shall enter the CC-state U10, "Call Active".

References

TS GSM 04.08 section 5.2.2.6.

26.8.1.3.5.1.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of CONNECT ACKNOWLEDGE shall enter the CC-state U10, "Call Active".

26.8.1.3.5.1.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/2.

Foreseen final state of the MS

U10, active.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8 (if the MS uses immediate connection for the selected basic service then p = Y, otherwise p = N). The SS sends a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered state U10, active.

Expected sequence

Step	Direction	Message	Comments

A1	SS -> MS	ASSIGNMENT COMMAND	p = Y
	A2	MS -> SS	ASSIGNMENT COMPLETE

3	SS -> MS	CONNECT ACKNOWLEDGE	
	4	SS -> MS	STATUS ENQUIRY	
	5	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

###### 26.8.1.3.5.2	Incoming call / U8 connect request / timer T313 timeout

26.8.1.3.5.2.1	Definition and applicability

The call control entity of the MS being in the state, U8, if no response is then received from the SS, timer T313 expires at the MS side. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.2.2	Conformance requirement

A CC entity of a MS in CC-state U8, "Connect Request", having waited for a reasonable length of time (e.g. expiry of timer T313) without receiving the appropriate protocol message to complete the incoming call, shall initiate the clearing of that incoming call by sending the CC message DISCONNECT and enter the CC-state U11, "Disconnect Request".

If an MS disconnects too early then, in the case of very late assignment of a traffic channel, systematic waste of radio resources may occur.

References

TS GSM 04.08 section 5.2.2.6., TS GSM 04.08 section 5.4.3.

26.8.1.3.5.2.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", having waited for a reasonable length of time (e.g. expiry of timer T313) without receiving the appropriate protocol message to complete the incoming call, shall initiate the clearing of that incoming call by sending the CC message DISCONNECT and enter the CC-state U11, "Disconnect Request"

26.8.1.3.5.2.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/2.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

45 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8 (if the MS uses immediate connection for the selected basic service then p = Y, otherwise p = N). The T313 expires at the MS and the MS sends a DISCONNECT message and enters state U11, disconnect request. The SS checks by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence

Step	Direction	Message	Comments

A1	SS -> MS	ASSIGNMENT COMMAND	p = Y
	A2	MS -> SS	ASSIGNMENT COMPLETE

3	MS -> SS	DISCONNECT	Shall not be sent before 15 seconds after entry
				into state U8. But, shall be sent before 1.1 * T313
				after entry into state U8.
	4	SS -> MS	STATUS ENQUIRY
	5	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.3.5.3	Incoming call / U8 connect request / termination requested by the user

26.8.1.3.5.3.1	Definition and applicability

The call control entity of the MS being in the state, U10, the user requests for releasing of the call. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.3.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

References

TS GSM 04.07 section 6.2.2., TS GSM 04.08 section 5.4.3., TS GSM 11.10, Annex 3 - section 2.2.

26.8.1.3.5.3.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.3.5.3.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	MT circuit switched basic services for which immediate connect is not used.

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/2.

Foreseen final state of the MS

U11, disconnect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8 (if the MS uses immediate connection for the selected basic service then p = Y, otherwise p = N). Then the user requests termination of the call. The MS sends a DISCONNECT message and enters state U11, disconnect request. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence

Step	Direction	Message	Comments

A1	SS -> MS	ASSIGNMENT COMMAND	p = Y
	A2	MS -> SS	ASSIGNMENT COMPLETE

3			the user requests to clear the call
	4	MS -> SS	DISCONNECT	
	5	SS -> MS	STATUS ENQUIRY	
	6	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

###### 26.8.1.3.5.4	Incoming call / U8 connect request / DISCONNECT received with in-band information

26.8.1.3.5.4.1	Definition and applicability

The call control entity of the MS being in the state, U8, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.4.2	Conformance requirement

A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT with progress indicator #8 shall enter CC-state U12, if the traffic channel is in speech mode. If the TCH is not in speech mode, the MS shall send a RELEASE message and enter CC-state U19.

References

TS GSM 04.08 section 5.4.4., TS GSM 04.08 section 5.5.1.

26.8.1.3.5.4.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT with progress indicator #8 enters CC-state U12, if the traffic channel is in speech mode, and that the MS sends a RELEASE message and enters CC-state U19 if the TCH is not in speech mode.

26.8.1.3.5.4.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN. The MS is brought into the state U8 by using table 26.8.1.3/3.

Foreseen final state of the MS

U12, disconnect indication or U19 depending on the bearer capabilities.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. If channel mode is speech, the MS enters state U12, disconnect indication. If channel mode is not speech, the MS sends a RELEASE message and enters state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 39)

TCH in speech mode:
	A2	SS -> MS	STATUS ENQUIRY	
	A3	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:
	B2	MS -> SS	RELEASE	
	B3	SS -> MS	STATUS ENQUIRY	
	B4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 39:	With a progress indicator indicating in-band information; Progress Indicator, Progress description #8.

###### 26.8.1.3.5.5	Incoming call / U8 connect request / DISCONNECT received without in-band information

26.8.1.3.5.5.1	Definition and applicability

The call control entity of the MS being in the state, U8, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.5.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT without progress indicator, shall return a RELEASE message and enter the CC-state U19, "Release Request".

References

TS GSM 04.08 section 5.4.4., TS GSM 04.08 section 5.4.4.2.

26.8.1.3.5.5.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT without progress indicator, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.3.5.5.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/3.

Foreseen final state of the MS

U19, release request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	(note 40)
	2	MS -> SS	RELEASE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

NOTE 40:	Without a progress indicator indicating in-band information

###### 26.8.1.3.5.6	Incoming call / U8 connect request / RELEASE received

26.8.1.3.5.6.1	Definition and applicability

The call control entity of the MS being in the state, U8, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.6.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a RELEASE shall return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile terminating transaction identifiers shall be in CC-state U0, "Null".

References

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.3.5.6.3	Test purpose

1)	To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a RELEASE will return a RELEASE COMPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile terminating transaction identifiers are in CC-state U0, "Null".

26.8.1.3.5.6.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/3.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends a RELEASE message. The MS responds with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered state U0, null, with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	RELEASE	with cause "Normal, unspecified"
	2	MS -> SS	RELEASE COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	5	SS		repeat steps 3-4 to cover all the
				transaction identifiers from 000...110
	6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.5.7	Incoming call / U8 connect request / lower layer failure

26.8.1.3.5.7.1	Definition and applicability

The call control entity of the MS being in the state, U8, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.7.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", having detected a lower layer failure shall return to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

References

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.3.5.7.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", having detected a lower layer failure returns to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

26.8.1.3.5.7.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/1.

Foreseen final state of the MS

U0, null.

Maximum duration of test

1 min 30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The MS is brought to the state U8. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure
	2	SS		SS waits 20 s for the MS to return to
				listening to paging
	3	SS -> MS	PAGING REQUEST	
	4	MS -> SS	CHANNEL REQUEST	
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	PAGING RESPONSE	
	7	SS -> MS	STATUS ENQUIRY	
	8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)
	9	SS		repeat steps 7-8 to cover all the
				transaction identifiers from 000...110
	10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA).

Specific message contents:

None.

###### 26.8.1.3.5.8	Incoming call / U8 connect request / TCH assignment

26.8.1.3.5.8.1	Definition and applicability

The call control entity of the MS being in the state, U8, an assignment procedure is performed for traffic channel. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.8.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

References

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.2.7.

26.8.1.3.5.8.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.3.5.8.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/1.

Foreseen final state of the MS

U8, connect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS verifies by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH
	2	MS -> SS	ASSIGNMENT COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U8

Specific message contents:

None.

###### 26.8.1.3.5.9	Incoming call / U8 connect request / unknown message received

26.8.1.3.5.9.1	Definition and applicability

The call control entity of the MS being in the state, U8, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.3.5.9.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08 section 8.4.

26.8.1.3.5.9.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.3.5.9.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/1.

Foreseen final state of the MS

U8, connect request.

Maximum duration of test

30 s.

Test procedure

An MT circuit switched basic service is selected that is supported by the MS; if the MS supports MT telephony, the selected basic service is telephony. If necessary the MS is configured for that basic service. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD
	2	MS -> SS	STATUS	cause 97#, state U8
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U8

Specific message contents:

None.

#### 26.8.1.4	In call functions

##### 26.8.1.4.1	In-call functions / DTMF information transfer

###### 26.8.1.4.1.1	In-call functions / DTMF information transfer / basic procedures

26.8.1.4.1.1.1	Definition and applicability

Dual Tone Multi Frequency (DTMF) is an inband one out of four plus one out of four signalling system primarily used from terminal instruments in telecommunication networks.

The support of DTMF is only permitted when a bearer capability for speech is in use or during the speech phase of alternate speech/data and alternate speech/facsimile teleservices.

26.8.1.4.1.1.2	Conformance requirement

1)	An MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone, shall send a START DTMF message on the correct DCCH.

References

TS GSM 04.08, 5.3.3

2)	An MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone (the corresponding IA5 character being selected from among the ones supported), shall send a START DTMF message specifying the correct IA5 character in the "keypad information" field of the keypad facility information element.

2.1	TS GSM 04.08, 5.3.3

26.8.1.4.1.1.3	Test purpose

1)	To verify that an MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone, sends a START DTMF message on the correct DCCH.

2)	To verify that an MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone (the corresponding IA5 character being selected from among the ones supported), sends a START DTMF message specifying the correct IA5 character in the "keypad information" field of the keypad facility information element.

26.8.1.4.1.1.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	supported character set (e.g. 0-9, #, *, A, B, C, D)

-	if and how DTMF tone is indicated to the user

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC-state "active".

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

1 min.

Test procedure

The MS being in the call active state, a user causes a DTMF tone to be generated e.g. by depression of a key in the MS. A DTMF digit corresponding to the digit indicated by the user is sent in a START DTMF message by the MS. The SS will return a START DTMF ACK message to the MS. This acknowledgement may be used in the MS to generate an indication as a feedback for a successful transmission. Then the user indicates that the DTMF sending should cease e.g. by releasing the key. The MS will send a STOP DTMF message to the network which is acknowledged with STOP DTMF ACK by the SS.

The sequence described above is repeated for each of the applicable characters 0-9, #, *, A, B, C, and D.

Then a case of rejecting a DTMF tone is tested and the state of the MS is verified.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	START DTMF	the user causes DTMF tone to be generated
		SS		the SS will verify that the transmitted
				information corresponds to the digit pressed
	2	SS -> MS	START DTMF ACK	possible indication of a DTMF tone depending
				the PICS/PIXIT statements
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10
	5	MS -> SS	STOP DTMF	
	6	SS -> MS	STOP DTMF ACK	the DTMF tone indication shall be stopped
	7			the steps 1-4 shall be repeated for each of the
				applicable characters 0-9, #, *, A, B, C, D.
	8	SS -> MS	STATUS ENQUIRY	
	9	MS -> SS	STATUS	cause 30#, state U10
	10	MS -> SS	START DTMF	
	11	SS -> MS	START DTMF REJECT	
	12	SS -> MS	STATUS ENQUIRY	
	13	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None

##### 26.8.1.4.2	In-call functions / user notification

User notification procedure allows the network to notify a MS of any call-related event during the "active" state of a call. It also may allow a MS to notify the remote user of any appropriate call-related event during the "active" state of a call by sending a NOTIFY message containing a notification indicator to the network. No state change occurs at any of the interface sides during this procedure.

###### 26.8.1.4.2.1	In-call functions / User notification / MS terminated

26.8.1.4.2.1.1	Definition and applicability

This is a case for testing user notification procedure terminated by the mobile station. The test is applicable for those equipments supporting at least one circuit switched basic service.

26.8.1.4.2.1.2	Conformance requirement

1)	A CC entity of a MS in CC-state U10, "active", upon receiving of a NOTIFY message shall remain in the active state.

References

TS GSM 04.08, 5.3.1.

26.8.1.4.2.1.3	Test purpose

To verify that a CC entity of a MS in CC-state U10, "active", upon receiving of a NOTIFY message remains in the active state.

26.8.1.4.2.1.4	Method of test

Related PICS/PIXIT statements

-	supported circuit switched basic services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC-state "active".

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

10 s.

Test procedure

The MS being in the call active state, the SS will send a NOTIFY message to the MS. The state of the MS is checked after that.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	NOTIFY	
	2	SS -> MS	STATUS ENQUIRY	
	3	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

##### 26.8.1.4.3	In-call functions / channel changes

The two following test cases are for testing some elementary radio resource level procedures during an active state of a call to ensure call maintenance also during physical channel changes.

###### 26.8.1.4.3.1	In-call functions / channel changes / a successful channel change in active state/ Handover and Assignment Command

26.8.1.4.3.1.1	Definition and applicability

This is a case to test a change of a physical channel during active state of a call. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.4.3.1.2	Conformance requirement

1)	The MS being in the call active state after having successful completed a channel assignment or a handover command, shall remain in the call active state.

References

TS GSM 04.08, section 5.3.4.3.2, TS GSM 04.08, section 3.4.6.

26.8.1.4.3.1.3	Test purpose

To verify that the MS being in the call active state after having successful completed a channel assignment or having completed a handover command remains in the call active state.

26.8.1.4.3.1.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

- 	Type of MS ( P-GSM900 or EGSM or DCS1800 )

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC-state "active".

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

10 s.

Test procedure

The SS initiates a call to the Mobile Station, using an arbitrarily chosen MT circuit switched basic service ( see section 10 for generic call set up procedures )

The MS being in the call active state, th SS initiated channel assignment procedure causing an intracell change of channel by sending ASSIGNMENT COMMAND message to the MS. The MS performs channel assignment procedure and after the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message. The state of the MS is then checked.

The SS then initiates a Finely Synchronised handover intra cell procedure. On the successful completion of this procedure the state of the MS is checked.

Expected sequence

Step	Direction	Message	Comments

0			Generic call set up procedure defined in 
				sections 10.1 and 10.3, depending on choice
				of Bearer Capability.
	1	SS -> MS	ASSIGNMENT COMMAND	
	2	MS -> SS	ASSIGNMENT COMPLETE	
	3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10
	5	SS -> MS	HANDOVER COMMAND	See Specific message contents.
	6	MS -> SS	HANDOVER ACCESS	Four HANDOVER ACCESS
	7	MS -> SS	HANDOVER ACCESS
	8	MS -> SS	HANDOVER ACCESS
	9	MS -> SS	HANDOVER ACCESS
	10	MS -> SS	HANDOVER COMPLETE
	11	SS -> MS	STATUS ENQUIRY	
	12	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

ASSIGNMENT COMMAND

Information Element				value/remark

Channel Description

As used in Assignment Command when setting up the call, except :

- Timeslot Number				Arbitrary value, but different to

originally used.

HANDOVER COMMAND

Information Element				value/remark

Cell Description

- Network Colour Code			1

- Base Station Colour Code			5

- BCCH Carrier Number			P-GSM900 - ARFCN  30

DCS1800  - ARFCN 650

Channel Description

As used in Assignment Command when setting up the call, except :

- Timeslot Number				Arbitrary value, but different to

originally used.

Synchronisation Indication

- Report Observed Time Difference		Shall not be included.

- Synchronisation Indication			'Synchronised'.

- Normal Cell Indication			Ignore out of range timing advance.

STATUS

Information Element				value/remark

cause					#30, statue U10.

###### 26.8.1.4.3.2	In-call functions / channel changes / an unsuccessful channel change in active mode/ Handover and Assignment Command

26.8.1.4.3.2.1	Definition and applicability

This is a case to test an unsuccessful change of a physical channel during active state of a call. This test is applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

26.8.1.4.3.2.2	Conformance requirement

1)	The MS, when returning to the old channel after handover or Assignment failure and having established the link, shall remain in the call active state.

References

TS GSM 04.08, section 5.3.4.3.

26.8.1.4.3.2.3	Test purpose

To verify that the MS, when returning to the old channel after handover or Assignment failure and correctly establishing the link, will remain in the call active state.

26.8.1.4.3.2.4	Method of test

Related PICS/PIXIT statements

-	supported MT circuit switched basic services

-	Type of Mobile Station ( P-GSM900 or EGSM or DCS1800 )

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC-state "active".

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

30 s.

Test procedure

The SS initiates a call to the Mobile Station, using an arbitrarily chosen circuit switched basic service ( see section 10 for generic call set up procedures )

The MS being in the call active state, the SS initiates non synchronised handover procedure to cell B. The MS begins to send access bursts on the new DCCH. The SS activates the SACCH, but does not send a PHYSICAL INFORMATION MESSAGE, thus causing timer T3124 to timeout. Then the MS shall return back to the old channel and re-establish the signalling link on cell A and send a HANDOVER FAILURE message. The state of the MS is then checked.

The SS sends an Assignment command message allocating a hopping TCH/F, but does not activate the assigned channel. The MS shall attempt try to activate the new channel ( this is not verified ) and shall then reactivate the 'old' channel and trigger the establishment of the main signalling link on the old channel. The MS shall send an ASSIGNMENT FAILURE message. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

0			Generic call set up procedure defined in 
				sections 10.1 and 10.3, depending on choice 
				of Bearer Capability.
	1	SS -> MS	HANDOVER COMMAND	
	2	MS -> SS	HANDOVER ACCESS	Several messages
				are sent, all with the handover reference sent
				in the HANDOVER COMMAND message.
	3	MS -> SS	HANDOVER FAILURE	
	4	SS -> MS	STATUS ENQUIRY	
	5	MS -> SS	STATUS	cause 30#, state U10
	6 	SS -> MS	ASSIGNMENT COMMAND	Channel type = TCH/F, hopping. The MS
				attempts and fails to establish a signalling link
				on the new channel.
	7			The MS re-establishes the signalling link on the
				'old' channel.
	8	MS -> SS	ASSIGNMENT FAILURE	RR cause value = "protocol error unspecified"
	9	SS -> MS	STATUS ENQUIRY
	10	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

ASSIGNMENT FAILURE

Information Element				value/remark

RR cause					"protocol error unspecified"

HANDOVER FAILURE

Information Element				value/remark

RR cause					Not checked, as tested elsewhere.

STATUS

Information Element				value/remark

cause					#30, statue U10.

##### 26.8.1.4.4	In-call functions / MS terminated in-call modification

###### 26.8.1.4.4.1	In-call functions / MS terminated in-call modification / modify when new mode is not supported

26.8.1.4.4.1.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the new mode is not supported (and consequently not one of those negotiated and agreed during the establishment phase of the call). This test is applicable for any equipment supporting at least one circuit switched basic service.

26.8.1.4.4.1.2	Conformance requirement

1)	In the case that the MS supports the network originated in-call modification procedure, the MS after having received a MODIFY message with a new mode which is not the actual one and cannot be supported by the MS shall reject it by sending a MODIFY REJECT message or a STATUS message.

2)	In the case that the MS does not support the network originated in-call modification procedure, the MS shall, when receiving a MODIFY message, treat the message as unknown and respond with a STATUS message.

References

1)	TS GSM 04.08, sections 5.3.4.3.4.2 and 5.3.4.4.

2)	TS GSM 04.08, section 5.3.4.

26.8.1.4.4.1.3	Test purpose

1)	To verify that an MS supporting the network originated in-call modification procedure, after having received a MODIFY message with a new mode which is not the actual one and cannot be supported by the MS, rejects it by sending a MODIFY REJECT.

2)	To verify that an MS not supporting the network originated in-call modification procedure, after having received a MODIFY message, responds with a STATUS message.

26.8.1.4.4.1.4	Method of test

Related PICS/PIXIT statements

-	supported circuit switched basic services

-	the MS supports the network originated in-call modification procedure (p = Yes/No).

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC-state "active".

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

10 s.

Test procedure

The MS being in the call active state, the SS initiates in-call modification procedure by sending a MODIFY message with new mode different from actual mode and one of those not supported by the MS. The MS either returns a MODIFY REJECT message with the old bearer capability or a STATUS message with reject cause #97, depending on the PICS/PIXIT statement. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	MODIFY	with new mode different from actual one

2a	MS -> SS	MODIFY REJECT	with the old call mode included OR, p = Yes

2b	MS -> SS	STATUS	cause #97, state U10, p = No

3	SS -> MS	STATUS ENQUIRY	
	4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

##### 26.8.1.4.5	In-call functions / MS originated in-call modification

###### 26.8.1.4.5.1	In-call functions / MS originated in-call modification / a successful case of modifying

26.8.1.4.5.1.1	Definition and applicability

This test is to test a successful case of in-call modification, which is triggered by the calling tone identification (CNG) received by the MS. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.1.2	Conformance requirement

1)	The procedure shall be initiated by the MS in the "active" state of the call. It shall send a MODIFY message including the new mode to be changed to; and enter the "mobile originating modify" state. The new mode given in the MODIFY message shall be one of those already negotiated and agreed during the establishment phase of the call. The MS shall stop sending Bm-channel information according to the old mode and enter the state U26 "Mobile Originating Modify".

2)	Upon receipt of the MODIFY COMPLETE message the MS shall start sending channel information according to the new call mode and enter the "active" state.

References

Conformance requirement 1:	TS GSM 04.08, section 5.3.4.3.1.

Conformance requirement 2:	TS GSM 04.08, section 5.3.4.3.2.

26.8.1.4.5.1.3	Test purpose

1)	To verify that the procedure is initiated by the MS in the "active" state of the call. It sends a MODIFY message including the new mode to be changed to; and enters the "mobile originating modify" state. The new mode given in the MODIFY message is one of those already negotiated and agreed during the establishment phase of the call. The MODIFY originating side stops sending Bm-channel information.

2)	To verify that upon receipt of the MODIFY COMPLETE message the MS starts sending channel information according to the new call mode and enters the "active" state.

26.8.1.4.5.1.4	Method of test

Related PICS/PIXIT statements

-	a way to activate a dual mode call

-	a way to activate in-call modification

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with the new mode to the SS and the state of the MS is checked. The channel mode is modified with the CMM message including the appropriate channel mode for the new service. The SS then returns a MODIFY COMPLETE message. The state of the MS is then checked.

NOTE 41:	ICM can be initiated by manual intervention at the MS.

Expected sequence

Step	Direction	Message	Comments

1	MS		The MS is made to initiate a dual mode call
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	SDCCH
	4	MS -> SS	CM-SERVICE REQUEST	
	5	SS -> MS	CIPH MODE COMMAND	
	6	MS -> SS	CIPH MODE COMPLETE	
	7	MS -> SS	SETUP	as specified in specific message contents
	8	SS -> MS	AUTH REQUEST	
	9	MS -> SS	AUTH RESPONSE	
	10	SS -> MS	CALL PROCEEDING	as specified in specific message contents
	11	SS -> MS	ASSIGNMENT COMMAND	channel mode: see section 10.4
	12	MS -> SS	ASSIGNMENT COMPLETE	
	13	SS -> MS	ALERTING	
	14	SS -> MS	CONNECT	
	15	MS -> SS	CONNECT ACK	
	16	MS -> SS	MODIFY	as specified in specific message contents
	17	SS -> MS	STATUS ENQUIRY	
	18	MS -> SS	STATUS	cause 30#, state U26
	19	SS -> MS	CMM	as specified in specific message contents
	20	MS -> SS	CMM ACKNOWLEDGE
	21	SS -> MS	MODIFY COMPLETE	contains the new mode as bearer capability
	22	SS		allow at least 2 seconds for the MS to adapt
				for the new mode
	23	SS -> MS	STATUS ENQUIRY	
	24	MS -> SS	STATUS	cause 30#, state U10
	25	SS		verify that the MS stops sending Bm channel
				information according to the old mode

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.2	In-call functions / MS originated in-call modification / modify rejected

26.8.1.4.5.2.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the in-call modification is rejected. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.2.2	Conformance requirement

1)	Upon receipt of the MODIFY REJECT message with the old bearer capability the MS shall: resume sending Bm-channel information according to the present call mode; resume interpreting received Bm-channel information according to the present call mode; and enter the "active" state.

References

TS GSM 04.08, section 5.3.4.3.4.1.

26.8.1.4.5.2.3	Test purpose

To verify that upon receipt of the MODIFY REJECT message with the old bearer capability the MS resumes sending Bm-channel information according to the present call mode; resumes interpreting received Bm-channel information according to the present call mode; and enters the "active" state.

26.8.1.4.5.2.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state "active".

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS returns a MODIFY REJECT message. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode
				call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> SS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	MODIFY REJECT	with cause #58 bearer capability not
				available and with old bearer capabilities
	17	SS -> MS	STATUS ENQUIRY	
	18	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.3	In-call functions / MS originated in-call modification / an abnormal case of acceptance

26.8.1.4.5.3.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the in-call modification is accepted incorrectly. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.3.2	Conformance requirement

1)	Upon receipt of the MODIFY COMPLETE message indicating a call mode which does not correspond to the requested one the MS shall discard it and take no action.

References

TS GSM 04.08, section 5.3.4.4.

26.8.1.4.5.3.3	Test purpose

To verify that upon receipt of the MODIFY COMPLETE message indicating a call mode which does not correspond to the requested one the MS discards it and takes no action.

26.8.1.4.5.3.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U26 "Mobile Originating Modify".

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS returns a MODIFY COMPLETE message specifying a mode that does not correspond to the requested one. It will be verified then that the MS shall not take any action and the state of the MS will be checked.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> SS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	MODIFY COMPLETE	with a mode that does not correspond
				to the requested one
	17	SS -> MS	STATUS ENQUIRY	
	18	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.4	In-call functions / MS originated in-call modification / an abnormal case of rejection

26.8.1.4.5.4.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the in-call modification is rejected incorrectly. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.4.2	Conformance requirement

1)	Upon receipt of the MODIFY REJECT message indicating a call mode which does not correspond to the actual one the MS shall discard it and take no action.

References

TS GSM 04.08, section 5.3.4.4.

26.8.1.4.5.4.3	Test purpose

To verify that upon receipt of the MODIFY REJECT message indicating a call mode which does not correspond to the actual one the MS discards it and takes no action.

26.8.1.4.5.4.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U26 "Mobile Originating Modify".

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS returns a MODIFY REJECT message specifying a mode that does not correspond to the actual one. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> SS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	MODIFY REJECT	with a mode that does not correspond
				to the actual one
	17	SS -> MS	STATUS ENQUIRY	
	18	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.5	In-call functions / MS originated in-call modification / timeout of timer T323

26.8.1.4.5.5.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which timer T323 expires in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.5.2	Conformance requirement

1)	Upon expiration of T323 the MS shall initiate the procedures for call clearing with cause #102 "recovery on timer expiry".

References

TS GSM 04.08, section 5.3.4.3.4.3.

26.8.1.4.5.5.3	Test purpose

To verify that upon expiration of T323 (accuracy +/- 10%) the MS shall initiate the procedures for call clearing with cause #102 "recovery on timer expiry".

26.8.1.4.5.5.4	Method of test

Related PICS/PIXIT statements

-	supported circuit switched basic services

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U11 "disconnect request".

Maximum duration of test

1 minute.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS does not respond until timer T323 expires at the MS. The MS is expected to respond with a DISCONNECT message. The SS checks timer T323 accuracy between emission of MODIFY and reception of DISCONNECT messages, the state of the MS and a cause value from the DISCONNECT message.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> MS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS		the SS waits for the timer
				T323 expiry
	17	MS -> SS	DISCONNECT	cause value #102, the SS checks timer T323
				accuracy (+/- 10 %) between  MODIFY and
				DISCONNECT messages
	18	SS -> MS	STATUS ENQUIRY	
	19	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.6	In-call functions / MS originated in-call modification / a successful channel change in state mobile originating modify

26.8.1.4.5.6.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which a change of a physical channel occurs in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.6.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U26, "Mobile Originating Modify", after successful completion of a channel assignment procedure or channel mode modify procedure shall remain in the call state U26.

2)	Upon receipt of the MODIFY COMPLETE message the MS shall start sending channel information according to the new call mode and enter the "active" state.

References

1)	TS GSM 04.08, section 5.3.4.3.2, TS GSM 04.08, section 3.4.6.

2)	TS GSM 04.08, section 5.3.4.3.2.

26.8.1.4.5.6.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U26, "Mobile Originating Modify", after successful completion of a channel assignment procedure remains in the call state U26.

2)	To verify that upon receipt of the MODIFY COMPLETE message the MS starts sending channel information according to the new call mode and enters the "active" state.

26.8.1.4.5.6.4	Method of test

Related PICS/PIXIT statements

-	supported circuit switched basic services

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U10, active

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with a new mode to the SS. The SS does not respond immediately, but performs channel assignment procedure including the appropriate channel mode for the new service. The state of the MS is then checked. The SS then returns a MODIFY COMPLETE message. The state of the MS is checked finally.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> MS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	ASSIGNMENT COMMAND	channel mode implied by the MODIFY
				message
	17	MS -> SS	ASSIGNMENT COMPLETE	
	18	SS -> MS	STATUS ENQUIRY	
	19	MS -> SS	STATUS	cause 30#, state U26
	20	SS -> MS	MODIFY COMPLETE
	21	SS -> MS	STATUS ENQUIRY
	22	MS -> SS	STATUS	cause	30#, state U10

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.7	In-call functions / MS originated in-call modification / an unsuccessful channel change in state mobile originating modify

26.8.1.4.5.7.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which an unsuccessful change of a physical channel occurs in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U26, "Mobile Originating Modify", when returning to the old channel after handover failure and having established the link, shall remain in the call state U26.

References

TS GSM 04.08, section 5.3.4.3.2.

26.8.1.4.5.7.3	Test purpose

To verify that a CC-entity of the MS in CC-state U26, "Mobile Originating Modify", when returning to the old channel after handover failure and having established the link, remains in the call state U26.

26.8.1.4.5.7.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U26, mobile originating modify.

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with a new mode to the SS. The SS initiates handover procedure. When the MS tries to establish the main signalling link, it is prohibited by the SS. Then the MS shall return back to the old channel and re-establish correctly the link. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> SS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	HANDOVER COMMAND	
	17	MS -> SS	HANDOVER ACCESS	the SS does not respond
	18	MS -> SS	HANDOVER FAILURE	after the MS has re-established the main
				signalling link in the old channel
	19	SS -> MS	STATUS ENQUIRY	
	20	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.8	In-call functions / MS originated in-call modification / unknown message received

26.8.1.4.5.8.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which an unknown message is received in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.8.2	Conformance requirement

A CC entity of a MS in CC-state U26, "Mobile Originating Modify", having received an unknown message from its peer entity shall return a STATUS message.

References

TS GSM 04.08, section 8.4

26.8.1.4.5.8.3	Test purpose

To verify that a CC entity of a MS in CC-state U26, "Mobile Originating Modify", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.4.5.8.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

CC-state U26, mobile originating modify.

Maximum duration of test

10 s.

Test procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with a new mode to the SS. The SS sends a message with message type not defined for the protocol discriminator. The state of the MS is then checked.

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call
	2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH
	3	MS -> SS	CM-SERVICE REQUEST	
	4	SS -> MS	CIPH MODE COMMAND	
	5	MS -> SS	CIPH MODE COMPLETE	
	6	MS -> SS	SETUP	as specified in specific message contents
	7	SS -> MS	AUTH REQUEST	
	8	MS -> SS	AUTH RESPONSE	
	9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode call
	10	SS -> MS	ASSIGNMENT COMMAND	TCH
	11	MS -> SS	ASSIGNMENT COMPLETE	
	12	SS -> SS	ALERTING	
	13	SS -> MS	CONNECT	
	14	MS -> SS	CONNECT ACK	
	15	MS -> SS	MODIFY	MMI action to change the mode
	16	SS -> MS	unknown message	message type not defined for PD
	17	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.9	In-call functions / MS originated in-call modification / a release complete received

26.8.1.4.5.9.1	Definition and applicability

The call control entity of the MS being in the state, U26, the call is cleared by a RELEASE COMPLETE message sent by the SS. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech / Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech / Group 3 fax).

26.8.1.4.5.9.2	Conformance requirement

1)	A CC entity of the MS in CC-state U26, "mobile originating modify", upon receipt of a RELEASE COMPLETE message with valid cause value, shall enter CC state U0, "Null".

2)	On returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

Reference(s)

Conformance requirement 1: GSM 04.08 section 5.4.2, GSM 04.08 section 5.4.4

Conformance requirement 2: GSM 04.08 section 5.4.4.3

26.8.1.4.5.9.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U26, "mobile originating modify", upon receipt of a RELEASE COMPLETE message with valid cause value, enters CC state U0, "Null".

2)	To verify that on returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.4.5.9.4	Method of test

Related PICS/PIXIT statements

-	a way to activate a dual mode call

-	a way to activate in-call modification

-	support of dual bearer capability services

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Maximum duration of test

30 seconds.

Test Procedure

The MS initiates a call for one of the supported dual mode services. The MS being in the call active state, in-call modification procedure is initiated for the selected service from the MS side. The MS shall send a MODIFY message with the new mode to the SS and the state of the MS is checked. The SS sends a RELEASE COMPLETE message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

NOTE:	ICM can be initiated by manual intervention at the MS.

Expected sequence

Step	Direction	Message	Comments

1	MS		The MS is made to initiate a dual mode call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	SDCCH
	4	MS -> SS	CM-SERVICE REQUEST
	5	SS -> MS	CIPH MODE COMMAND
	6	MS -> SS	CIPH MODE COMPLETE
	7	MS -> SS	SETUP	as specified in specific message contents
	8	SS -> MS	AUTH REQUEST
	9	MS -> SS	AUTH RESPONSE
	10	SS -> MS	CALL PROCEEDING	as specified in specific message contents
	11	SS -> MS	ASSIGNMENT COMMAND	channel mode: see section 10.4
	12	MS -> SS	ASSIGNMENT COMPLETE
	13	SS -> MS	ALERTING
	14	SS -> MS	CONNECT
	15	MS -> SS	CONNECT ACK
	16	MS -> SS	MODIFY	as specified in specific message contents
	17	SS -> MS	STATUS ENQUIRY
	18	MS -> SS	STATUS	cause #30, state U26
	19	SS -> MS	RELEASE COMPLETE
	20	SS -> MS	STATUS ENQUIRY
	21	MS -> SS	RELEASE COMPLETE	cause #81 (invalid TI value)
	22	SS		repeat steps 20 - 21 to cover all the
				transaction identifiers from 000 ... 110
	23	SS -> MS	CHANNEL RELEASE	the main signalling link shall be
				released by the MS (L2: DISC/UA)

Specific message contents:

As specified in 26.8.1.4.5.10.

###### 26.8.1.4.5.10	In-call functions/MS originated in-call modification/contents of some of the messages

The following messages are used for testing in-call modification procedures, test cases 26.8.1.4.5.*, as default messages for those ones defined below. If any other values are defined in the expected sequence of the actual test cases, those values take precedence over the ones defined hereafter.

SETUP (MS to SS)

Information element			Value/remark

BC Repeat indicator

Repeat indication			Sequential, if BS81 is being tested, otherwise

circular for successive selection

Bearer capability 1				Appropriate for the teleservice/Bearer Service

selected as an initial call mode

Bearer capability 2				Appropriate for the teleservice/Bearer Service

to be selected as a new call mode

Facility					Omitted

Calling party subaddress			Omitted

Called party BCD number			As entered

Called party subaddress			Omitted

LLC repeat indicator			The same repeat indication as the one for BC.

Present if and only if LLC I and LLC II are present

Low layer compatibility I			See note 42

Low layer compatibility II			See note 42

HLC repeat indicator			The same repeat indication as the one for BC.

Present if and only if HLC i and HLC ii are present.

High layer compatibility i			See note 42

High layer compatibility ii			See note 42

User-user				Omitted

SS version				Omitted

CLIR suppression				Omitted

NOTE 42:	HLC/LLC may or may not be present. The contents of HLC/LLC are not verified. If LLC I is present then LLC II shall be present. If HLC i is present then HLC ii shall be present.

CALL PROCEEDING

If the MS offers a choice in a SETUP message with respect to its bearer capabilities (this choice is restricted to the connection element), the bearer capabilities 1 and 2 and BC repeat indicator must all be present in this message. Otherwise, all three IEs are omitted.

Information element			Value/remark

Repeat Indicator				See above

Repeat indication			As received in the SETUP message

Bearer Capability 1			Same as in section 10.4

Bearer Capability 2			Same as in section 10.4

Facility					Omitted

Progress indicator				Omitted

MODIFY

Information element			Value/remark

Bearer capability				If the bearer capability IEs were present in the

CALL PROCEEDING message, then as it was

specified in the bearer capability 2 of the

CALL PROCEEDING message. Otherwise as

in the bearer capability 2 of the

SETUP message.

Reverse Call Setup Direction		Presence and value not checked

Low layer compatibility			See note 43

High layer compatibility			See note 43

NOTE 43:	HLC (LLC) shall be included if the HLC (LLC) was included in the SETUP message. The contents of LLC/HLC are not verified.

MODIFY COMPLETE

Information element			Value/remark

Bearer capability				If the bearer capability IEs were present in the

CALL PROCEEDING message, then as it was

specified in the bearer capability 2 of the

CALL PROCEEDING message. Otherwise as

in the bearer capability 2 of the

SETUP message.

Reverse Call Setup Direction		Same as in MODIFY

Low layer compatibility			See note 44

High layer compatibility			See note 44

NOTE 44:	HLC (LLC) shall be included if the HLC (LLC) was included in the SETUP message. The contents of LLC/HLC are not verified.

MODIFY REJECT

Information element			Value/remark

Bearer capability				If the bearer capability IEs were present in the

CALL PROCEEDING message, then as it was

specified in the bearer capability 1 of the

CALL PROCEEDING message. Otherwise as

in the bearer capability 1 of the

SETUP message.

Cause					#58 "bearer capability not presently available".

Low layer compatibility			See note 44a

High layer compatibility			See note 44a

NOTE 44a:	HLC (LLC) shall be included if the HLC (LLC) was included in the SETUP message. The contents of LLC/HLC are not verified.

CHANNEL MODE MODIFY

Information element			Value/remark

Channel description			describes non-hopping Bm+ACCHs or

Lm+ACCHs as appropriate for the test

Channel Mode				appropriate for the BC in the MODIFY

CHANNEL MODE MODIFY ACKNOWLEDGE

Information element			Value/remark

Channel description			as sent by the SS in the corresponding

CHANNEL MODE MODIFY message

Channel mode				as sent by the SS in the corresponding

CHANNEL MODE MODIFY message

### 26.8.2	Call Re-establishment

#### 26.8.2.1	Call Re-establishment/call present, re-establishment allowed

26.8.2.1.1	Definition and applicability

This is to test a successful case of a call re-establishment procedure. This test is applicable for any equipment supporting at least one bearer capability. If the MS does not perform call re-establishment procedure correctly, the network will waste resources.

26.8.2.1.2	Conformance requirement

1)	If the call is in the "active" state or "mobile originating modify" state, the indication from MM that re-establishment is possible shall cause call control to request re-establishment from the MM-connection, suspend any further message to be sent and await the completion of the re-establishment procedure.

2)	When the call control entity is notified that the MM-connection is reestablished, it shall then resume the transmission of possibly suspended messages and resume user data exchange when an appropriate channel is available.

References

1)	TS GSM 04.08, subclauses 4.5.1.6 and 5.5.4.2.

2)	TS GSM 04.08, subclauses 4.5.1.6 and 5.5.4.3.

26.8.2.1.3	Test purpose

The purpose of this test is to verify that the MS can correctly perform a call re-establishment procedure.

26.8.2.1.4	Method of test

Related PICS/PIXIT statements

-	supported teleservices

Initial conditions

System Simulator:

The SS simulates cells A and B. The LAC of cell A is different from the LAC of cell B. The PLMN identities of cell A and B are equal.

The call re-establishment parameter concerning cell A is set to an arbitrary value.

Cell B is not barred, the RACH control parameters information element sent in SYSTEM INFORMATION TYPE 1 to 4 messages of cell A and B specifies 'call reestablishment allowed in the cell', the NCC of cell B is indicated as permitted in the PLMN permitted information element of SYSTEM INFORMATION TYPE 2 and 6 messages of cell A. Cell B is indicated as a neighbour cell of cell A in SYSTEM INFORMATION TYPE 2 and 5 messages of cell A. Cell reselect hysteresis parameter of cell A is set to zero.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN on cell A.

Foreseen final state of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Maximum duration of test

1 minute.

Test procedure

The MS is brought to active state by using procedure 26.9.2, "structured procedures, MS originated call, early assignment". The RF level of cell A is lowered so that cell B is to be selected (when the MS performs re-establishment after radio link failure), while keeping the C1 and C2 of cell A greater than zero. SS waits for at least 5 seconds. Then the SS stops transmission on the TCH/SACCH. The MS shall re-establish the call on cell B using a CM-REESTABLISHMENT message. The SS performs ciphering mode setting and assignment procedures. The MS shall through-connect the appropriate bearer channel. Then, the call is cleared by the SS.

Expected sequence

Step	Direction	Message	Comments

1			Steps 1-19 of test case 26.9.2 are performed
				(the appropriate bearer channel is through
				connected in both directions in TCH)
	2	SS		The RF level of cell A is lowered. The SS
				waits at least 5 seconds. The SS stops
				transmission on the TCH/SACCH.
	3	MS -> SS	CHANNEL REQUEST	this is sent on cell B. Establ. cause shall be
				"call re-establishment; TCH/F was in use,..."
	4	SS -> MS	IMMEDIATE ASSIGNMENT	
	5	MS -> SS	CM-REESTAB REQUEST	note specific message contents
	6	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	7	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.
				All following messages
				shall be sent enciphered.
	8	SS		SS starts ciphering.
	9	SS -> MS	ASSIGNMENT COMMAND	
	10	MS -> SS	ASSIGNMENT COMPLETE	
	11	MS		The appropriate bearer channel is
				through connected in both directions.
	12	SS -> MS	DISCONNECT	with cause value "Normal"
	13	MS -> SS	RELEASE	
	14	SS -> MS	RELEASE COMPLETE	
	15	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific message contents:

CM-REESTABLISHMENT REQUEST

Information element			Value/remark

Protocol discriminator			Mobility Management

Skip indicator				Encoded as zeroes

Message type				CM-REESTABLISHMENT REQUEST

Ciphering key sequence number		The CKSN which the MS was allocated

in step 6 of the procedure of section 26.9.2.

Spare half octet				zero

Mobile station classmark 2			as declared in the PICS/PIXIT

Mobile identity				The TMSI that the MS is having initially

Location area identification			Corresponding the LAI of cell A

#### 26.8.2.2	Call Re-establishment/call present, re-establishment not allowed

26.8.2.2.1	Definition and applicability

This is to test a special case of a call re-establishment, in which it is not allowed for a MS to attempt re-establishment of a call. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.2.2.2	Conformance requirement

When a lower layer failure occurs while an MM-connection is active, if a cell allowing call re-establishment is not available, the MS shall release the MM-connection and shall not attempt call re-establishment.

References

TS GSM 04.08, subclauses 4.5.1.6 and 5.5.4.

26.8.2.2.3	Test purpose

The purpose of this test is to verify that the MS does not attempt call re-establishment when it is not allowed to take place because of the unavailability of a cell allowing call re-establishment.

26.8.2.2.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

The SS simulates cell A.

Cell A is not barred, the NCC of cell A is indicated as permitted in the PLMN permitted information element of SYSTEM INFORMATION TYPE 2 and 6 messages. The RE field of the RACH control parameters information element broadcast in messages SYSTEM INFORMATION TYPE 1, 2, 3 and 4 of cell A are set to "call reestablishment not allowed in the cell".

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Maximum duration of test

1 minute.

Test procedure

The MS is brought to active state by using procedure 26.9.2, "structured procedures, MS originated call, early assignment". The SS stops transmission on the TCH/SACCH. The MS shall not require re-establishment of the call.

Expected sequence

Step	Direction	Message	Comments

1			Steps 1-19 of test case 26.9.2 are performed
				(the appropriate bearer channel is through
				connected in both directions in TCH)
	2	SS		the SS stops transmission on the TCH/SACCH
  	3	MS		the MS shall not attempt re-establishment
				on cell A. This is checked for 30
				seconds after the radio link failure.

Specific message contents:

None.

#### 26.8.2.3	Call Re-establishment/call under establishment, transmission stopped

26.8.2.3.1	Definition and applicability

This is to test a special case of a call re-establishment, in which it is not allowed for a MS to attempt re-establishment of a call, since the call hasn't been established yet. This test is applicable for any equipment supporting at least one mobile originated circuit switched basic service.

26.8.2.3.2	Conformance requirement

When a lower layer failure occurs while an MM-connection is active, if the state of the call control entity is not "active", the MS shall release the MM-connection and shall not attempt call re-establishment.

References

TS GSM 04.08, subclauses 4.5.1.6 and 5.5.4.2.

26.8.2.3.3	Test purpose

The purpose of this test is to verify that the MS does not attempt call re-establishment when it is not allowed to take place because of the call control state.

26.8.2.3.4	Method of test

Related PICS/PIXIT statements

-	supported MO circuit switched basic services

Initial conditions

System Simulator:

The SS simulates cell A.

Cell A is not barred, the RACH control parameters information element sent in SYSTEM INFORMATION TYPE 1 to 4 messages of cell A specifies 'call reestablishment allowed in the cell', the NCC of cell A is indicated as permitted in the PLMN permitted information element of SYSTEM INFORMATION TYPE 2 and 6 messages.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Maximum duration of test

1 minute.

Test procedure

The call control entity of the MS is brought to state U4, "call delivered" by using initial part of procedure 26.9.2, "structured procedures, MS originated call, early assignment". The SS stops transmission on the TCH/SACCH. The MS shall not require re-establishment of the call on cell A.

Expected sequence

Step	Direction	Message	Comments

1			the MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	CM SERVICE REQUEST	
	5	SS -> MS	AUTHENTICATION REQ	
	6	MS -> SS	AUTHENTICATION RESP	
	7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	8	MS -> SS	CIPHER MODE COMPLETE	
				All following messages
				shall be sent enciphered.
	9	SS		SS starts ciphering.
	10	MS -> SS	SETUP	
	11	SS -> MS	CALL PROCEEDING	
	12	SS -> MS	ALERTING	
	13	SS		the SS stops transmission on the TCH/SACCH
	14	MS		the MS shall not attempt re-establishment 
				on cell A. This is checked for 30
				seconds after the radio link failure.

Specific message contents:

None.

### 26.8.3	User to user signalling

26.8.3.1		Definition and applicability

The 'user to user' information element is used to convey information between the mobile user and a remote ISDN user. This test is therefore applicable for any equipment supporting at least one mobile terminating circuit switched basic service.

NOTE 45:	There is no test for an MS originating call including a 'user-user' information element since it is not a mandatory MS feature.

26.8.3.2	Conformance requirement

The inclusion of the 'user-user' information element in downlink call control messages shall cause no adverse effects on the operation of the MS.

References

GSM 04.08 sections 5.2.2, 9.3.7, 9.3.23.1 and 10.5.4.25

26.8.3.3	Purpose of the test

The purpose of this test is to verify that inclusion of the 'user-user' information element in either of the down link messages, SETUP or DISCONNECT causes no adverse effects on the operation of the MS.

26.8.3.4	Method of test

Related PICS/PIXIT statement(s)

Supported MT circuit switched basic services.

Support of user-user information element, and details of suitable codings

Initial conditions.

System Simulator :

The SS simulates 1 cell, with default parameters.

Mobile Station :

The MS is in MM-state "idle updated", with a valid TMSI and CKSN.

Foreseen final state of the MS

The MS is in MM-state "idle updated", with a valid TMSI and CKSN.

Maximum duration of test

2 minutes

Test procedure

The SS attempts to set up a mobile terminated call, with one of the supported circuit switched basic services which has been arbitrarily chosen, using one of the generic call set up procedures,( either speech or data ) as specified in section 10. The default SETUP message contents are modified to include the user-user Information Element. The MS shall not respond adversely to the inclusion of the user-user information element.

After 30 seconds the SS sends a DISCONNECT message, again the MS shall not respond adversely to the inclusion of the user-user information element, but shall continue to clear down the call normally.

Expected sequence

Step	Direction	Message	Comments

1			Generic Call Setup procedure defined in 
				sections in 10.1 or 10.3, depending on choice 
				of Bearer Capability. The SETUP message in
				either case contains the user-user IE, see
				Specific message contents.
	2			The SS waits 30 seconds.
	3	SS -> MS	DISCONNECT	Message contains the user-user IE, see
				Specific message contents
	4	MS -> SS	RELEASE	As defined in section 26.8.4
	5	SS-> MS	RELEASE COMPLETE	As defined in section 26.8.4
	6	SS-> MS	CHANNEL RELEASE	As defined in section 26.8.4

Specific message contents:

SETUP

Information Element			value/remark

As default message contents as defined in the Generic Call setup procedures section 10.1 or 10.3 except :

Bearer Capability				Bearer capability arbitrarily chosen from

those supported by the Mobile Station under test.

user-user

- length				Length of user-user contents ( note 46 )

- PD				IA5 characters ( note 46 )

- user-user 				The following string coded in IA5 characters :

"Call Setup" ( note 46 )

DISCONNECT

Information Element			value/remark

As default message contents as defined in section 26.8.4, except :

user-user

- length				Length of user-user contents ( note 46 )

- PD				IA5 characters ( note 46 )

- user-user 				The following string coded in IA5 characters :

"Call Disconnect" ( note 46 )

NOTE 46:	The codings above are for example only. For the case of an MS which supports 'user-user' signalling it may be necessary to add meaning to the data fields, see PICS/PIXIT statement(s).

### 26.8.4	Default contents of message

ALERTING (mobile station to network direction)

No default requirements defined for this message.

ALERTING (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

User-user				Omitted

ASSIGNMENT COMMAND

Information element			Value/remark

Description of the first channel		describes non-hopping Bm+ACCHs or

Lm+ACCHs as appropriate for the test

Power Command				As in section 6.3

Frequency list				Omitted

Cell channel description			Omitted

Mode of the first channel 			appropriate for the bearer capability chosen for the test

Description of the second channel		Omitted

Mode of the second channel			Omitted

Mobile allocation				Omitted

Starting time				Omitted

Cipher mode setting			Omitted

ASSIGNMENT COMPLETE

Information element			Value/remark

RR cause				not checked

AUTHENTICATION REQUEST

Information element			Value/remark

Ciphering key sequence number		Arbitrary excluding 111B

Spare half octet				(spare bits)

Authentication parameter RAND		Arbitrary

AUTHENTICATION RESPONSE

Information element			Value/remark

Authentication parameter SRES		not checked

CALL CONFIRMED

No default requirements defined for this message.

CALL PROCEEDING

Information element			Value/remark

Repeat Indicator				Omitted

Bearer Capability 1			Omitted if the SETUP message did not specify

in the bearer capability 1 IE a connection

element value "both, transparent preferred" or

"both, non-transparent preferred". Otherwise

included; in that case the connection element

specifies the value that is appropriate for the

selected basic service (either value

"transparent" or value "non transparent (RLP)"),

all other parameters are same as in the bearer

capability 1 IE of the received

SETUP message.

Bearer Capability 2				Omitted

Facility					Omitted

Progress indicator				Omitted

CHANNEL MODE MODIFY

Information element			Value/remark

Channel description			describes non-hopping Bm+ACCHs or

Lm+ACCHs as appropriate for the test

Channel mode				appropriate for the bearer capability chosen for

the test

CHANNEL MODE MODIFY ACKNOWLEDGE

Information element			Value/remark

Channel description			as sent by the SS in the corresponding

CHANNEL MODE MODIFY message

Channel mode				as sent by the SS in the corresponding

CHANNEL MODE MODIFY message

CHANNEL RELEASE

Information element			Value/remark

RR cause  				Normal event

CHANNEL REQUEST

Information element			Value/remark

Establishment cause 			If in response to paging,

then '100'; if a mobile originating call,

then '111'

Random reference				Arbitrary value of 5 bits length

CIPHERING MODE COMMAND

Information element			Value/remark

Cipher mode setting

algorithm identifier			indicates a supported algorithm

SC					Start ciphering

Cipher response

CR					IMEI must not be included

CIPHERING MODE COMPLETE

No default requirements defined for this message.

CM-SERVICE ACCEPT

No default values defined for this message.

CM-SERVICE REJECT

Information element			Value/remark

Reject cause				Service or option not available, unspecified

CM-SERVICE REQUEST

No default requirements defined for this message.

CONNECT (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

Connected number				Omitted

Connected subaddress			Omitted

User-user				Omitted

CONNECT (mobile station to network direction)

No default requirements defined for this message.

CONNECT ACKNOWLEDGE

No default requirements defined for this message.

DISCONNECT (network to mobile station direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

Progress indicator				Omitted

User-user 				Omitted

DISCONNECT (mobile station to network direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

User-user 				Omitted

SS version				Omitted

HANDOVER ACCESS

No default requirements defined for this message.

HANDOVER COMMAND

Information element			Value/remark

Cell Description				a BCCH frequency, which is one of the

neighbour cells

Description of the first channel		describes non-hopping Bm+ACCHs or

Lm+ACCHs as appropriate for the test

Handover Reference			an arbitrary value

Power Command				as in 6.3

Synchronisation indication			Omitted

Frequency short list			Omitted

Frequency List				Omitted

Cell Channel Description			Omitted

Channel Mode				Omitted

Channel Description			Omitted

Channel Mode 2				Omitted

Frequency Channel Sequence		Omitted

Mobile Allocation				Omitted

Starting Time				Omitted

Real time difference			Omitted

Timing advance				Omitted

Cipher Mode setting			Omitted

HANDOVER FAILURE

No default requirements defined for this message.

IMMEDIATE ASSIGNMENT

Information element			Value/remark

Page mode 				Normal paging

Channel description			describes a valid SDCCH+SACCH

in non-hopping mode

Request reference

Random access information		As received from MS

T1', T2, T3				Corresponding to frame number

of the CHANNEL REQUEST

Timing advance				corresponding the timing difference between

the MS and the SS

Mobile allocation				Empty (L=0)

Starting time				Omitted

MODIFY

No default values defined for this message.

MODIFY COMPLETE

No default requirements defined for this message.

MODIFY REJECT

No default values defined for this message.

NOTIFY (network to mobile station direction)

Information element			Value/remark

Notification indicator			one of the valid values chosen arbitrarily

PAGING REQUEST TYPE 1

Information element			Value/remark

L2 pseudo length				L2 pseudo length of the message

Page Mode				Normal Paging

Channels needed for Mobiles 1 and 2

channel (first)			any channel

channel (second)			any channel

Mobile identity 1				TMSI of MS under test

Mobile identity 2				Omitted

P1 rest octets				(spare octets)

PAGING RESPONSE

No default requirements defined for this message.

PROGRESS

No default values defined for this message.

RELEASE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Second cause				Omitted

Facility					Omitted

User-user				Omitted

RELEASE (mobile station to network direction)

No default requirements defined for this message.

RELEASE COMPLETE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Facility					Omitted

User-user				Omitted

RELEASE COMPLETE (mobile station to network direction)

No default requirements defined for this message.

SETUP (mobile station to network direction)

Information element			Value/remark

BC Repeat indicator			Omitted

Bearer capability 1			Appropriate for the basic service selected for

the test

Bearer capability 2			Omitted

Facility					Omitted

Calling party subaddress			Omitted

Called party BCD number			As entered

Called party subaddress			Omitted

LLC repeat indicator			Omitted

Low layer compatibility I			Appropriate for the basic service selected for

the test

Low layer compatibility II			Omitted

HLC repeat indicator			Omitted

High layer compatibility i			Appropriate for the basic service selected for

the test

High layer compatibility ii			Omitted

User-user				Omitted

SS version				Omitted

CLIR suppression				Omitted

SETUP (network to mobile station direction)

Information element			Value/remark

BC repeat indicator			Omitted

Bearer capability 1			Appropriate for the basic service selected for

the test

Bearer capability 2			Omitted

Facility					Omitted

Progress indicator				Omitted

Signal					Any defined value as described for Signal IE

in TS GSM 04.08

Calling party BCD number			Omitted

Calling party subaddress			Omitted

Called party BCD number			Omitted

Called party subaddress			Omitted

LLC repeat indicator			Omitted

Low layer compatibility I			Appropriate for the basic service selected for

the test

Low layer compatibility II			Omitted

HLC repeat indicator			Omitted

High layer compatibility i			Appropriate for the basic service selected for

the test

High layer compatibility ii			Omitted

User-user				Omitted

START DTMF

No default requirements defined for this message.

START DTMF ACKNOWLEDGE

Information element			Value/remark

Keypad facility				corresponding to the DTMF digit indicated in

the START DTMF message

START DTMF REJECT

Information element			Value/remark

Cause					value "Resources unavailable, unspecified"

STATUS

Information element			Value/remark

Cause					Value "Response to STATUS ENQUIRY"

Call state				Specified separately in each test case

Auxiliary states				Omitted

STATUS ENQUIRY

No default values defined for this message, except that when this message is used to check that "all the transaction identifiers from 000 to 110" are in the null state, the TI flag shall take the value '1' in mobile originating call tests and shall take the value '0' in mobile terminating call tests.

STOP DTMF

No default requirements defined for this message.

STOP DTMF ACKNOWLEDGE

No default values defined for this message.

Unknown Message

Protocol Discriminator			Call Control; Call Related SS

Transaction Identifier			same as in use in the test

Message Type				0000 0100

## 26.9	Structured procedures

Abbreviations for message names used in this section:

AUTHENTICATION REQ	=	AUTHENTICATION REQUEST

AUTHENTICATION RESP	=	AUTHENTICATION RESPONSE

CIPHER MODE COMMAND	=	CIPHERING MODE COMMAND

CIPHER MODE COMPLETE	=	CIPHERING MODE COMPLETE

### 26.9.1	Structured procedures / general

The purpose of these tests is to verify that the MS performs certain elementary procedures of the RR, MM, and CC protocol correctly within a structured procedure. The term "structured procedure" is defined in TS GSM 04.08, section 7, where also examples of structured procedures are given.

The reason for this test purposes is twofold:

-	The behaviour of the MS in an elementary procedure may depend on the preamble which precedes the elementary procedure.

-	Structured procedures tested in this section are used in other parts of this Technical Specification as preambles to establish the initial conditions for other tests; correct behaviour of an implementation under test in a preamble is essential for the validity of a test.

Mobile originating and terminating calls are tested in cases of both early and late assignment of the traffic channel; in one of the cases call release initiated by the network is tested, in another one, call release initiated by the MS. The tests in this section only cover the successful outcome of elementary procedures (i.e. they do not deal with abnormal cases).

The emergency call service is tested in the following cases:

-	emergency call initiated in the idle, updated state with authentication and ciphering, for all supported speech rates,

-	emergency call initiated in the idle, no IMSI state (hence without authentication and without ciphering), the network accepting the call, for one supported speech rates,

-	emergency call initiated in the idle, no IMSI state (hence without authentication and without ciphering), the network rejecting the call, for one supported speech rate.

These tests on emergency calls are only applicable to an MS supporting speech.

For an MS supporting speech the test procedures in 26.9.2, 26.9.3, 26.9.4 and 26.9.5 are performed for speech (teleservice 11, telephony), once for each supported rate (full rate/half rate).

For an MS not supporting speech but supporting at least one teleservice, for each of the test procedures in 26.9.2, 26.9.3, 26.9.4, and 26.9.5 and each supported rate (full rate/half rate) a teleservice supported by the MS (see PICS/PIXIT statement) is chosen, and the test is performed corresponding to that teleservice (note that this teleservice is never a dual service).

In cases where a mobile originated call for the tested teleservice can be initiated both

-	via the MMI and

-	via the R or S interface,

procedure 26.9.2 shall be performed when initiating the mobile originated call via the MMI and procedure 26.9.3 shall be performed when initiating the mobile originated call via an appropriate interface (R or S).

### 26.9.2	Structured procedures / MS originated call / early assignment

26.9.2.1	Conformance requirements

1)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call, if it provides a human interface, shall display the dialled number.

2)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call for a selected teleservice that is supported by the MS, shall start to initiate the immediate assignment procedure by sending a CHANNEL REQUEST message with correct establishment cause.

3)	Subsequently after establishment of an MM connection, the MS shall send a SETUP message with correct parameters.

4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

5)	Subsequently when the network initiates call clearing by sending a DISCONNECT message, the MS shall proceed to release the call by sending a RELEASE message.

6)	On receipt of a CHANNEL RELEASE message, the MS shall disconnect the main signalling link.

References

Conformance requirement 1:	TS GSM 02.07.

Conformance requirement 2:	TS GSM 04.08 section 3.3.1.

Conformance requirement 3:	TS GSM 04.08 section 5.2.1.1.

Conformance requirement 4:	TS GSM 04.08 section 5.2.1.6.

Conformance requirement 5:	TS GSM 04.08 section 5.4.4.

Conformance requirement 6:	TS GSM 04.08, section 3.5.1.

26.9.2.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, displays the dialled number in the way described in a PICS/PIXIT statement.

2)	To verify that the MS in MM state "idle, updated" and in RR idle mode, with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

3)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed the authentication and cipher mode setting procedures, the MS sends a SETUP message with correct parameters.

4)	To verify that subsequently, after receipt of a CALL PROCEEDING message and of an ASSIGNMENT COMMAND message allocating an appropriate TCH, after having completed the traffic channel early assignment procedure by replying with the ASSIGNMENT COMPLETE message, after receipt of an ALERTING message and a CONNECT message, the MS returns a CONNECT ACKNOWLEDGE message.

5)	To verify that subsequently the MS has attached the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

6)	To verify that subsequently upon the network initiating call clearing by sending a DISCONNECT message, the MS proceed to release the call with RELEASE.

7)	To verify that subsequently, on receipt of a RELEASE COMPLETE message followed by a CHANNEL RELEASE message, the MS disconnects the main signalling link.

These test purposes are tested for all rates supported by the MS (full rate/half rate).

26.9.2.3	Method of test

Related PICS/PIXIT Statements

- 	Supported rates (full rate/half rate)

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	SS version

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" is entered
  	2	MS		If p1 = Y, the MS must display the called number 
				in the way defined in PICS/PIXIT.
	3	MS -> SS	CHANNEL REQUEST	Establishment cause is "originating call and the 
				network does not set the NECI bit to 1".
	4	SS -> MS	IMMEDIATE ASSIGNMENT	
	5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.
	6	SS -> MS	AUTHENTICATION REQ
	7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered.
				All following messages shall be sent enciphered.
	10	SS		SS starts ciphering.
	11	MS -> SS	SETUP
	12	SS -> MS	CALL PROCEEDING
	13	SS -> MS	ASSIGNMENT COMMAND
	14	MS -> SS	ASSIGNMENT COMPLETE
	15	SS -> MS	ALERTING
	16	MS		Depending on the PICS, an alerting indication
				is given
	17	SS -> MS	CONNECT
	18	MS -> SS	CONNECT ACKNOWLEDGE
	19	MS		The appropriate bearer channel is
				through connected in both directions.
	20	SS -> MS	DISCONNECT
	21	MS -> SS	RELEASE
	22	SS -> MS	RELEASE COMPLETE
	23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

### 26.9.3	Structured procedures / MS originated call / late assignment

26.9.3.1	Conformance requirement

1)	An MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS , shall start to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

2)	Upon receipt of the ASSIGNMENT COMMAND message, the Mobile Station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the data links). After the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message, specifying cause 'normal event', to the network on the main DCCH.

3,4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

References

Conformance requirement 1:	TS GSM 04.08 section 3.3.1.

Conformance requirement 2:	TS GSM 04.08 section 3.4.3.1, 3.4.3.2

Conformance requirement 3:	TS GSM 04.08 section 5.2.1.6.

26.9.3.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message.

2)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed authentication and cipher mode setting procedures, after having sent a SETUP message, after having received a CALL PROCEEDING message followed by an ALERTING message and an ASSIGNMENT COMMAND message allocating an appropriate TCH, the MS sends an ASSIGNMENT COMPLETE message.

3)	To verify that subsequently, after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message returns a CONNECT ACKNOWLEDGE message.

4)	To verify that after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message attaches the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

These test purposes are tested for all rates supported by the MS (full rate/half rate).

26.9.3.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS has a MO call in state U10, "active".

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with late assignment.

Maximum Duration of Test

30 seconds

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" is entered
	2	MS		
	3	MS -> SS	CHANNEL REQUEST	Establishment cause is "originating call and the
				network does not set the NECI bit to 1".
	4	SS -> MS	IMMEDIATE ASSIGNMENT	
	5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.
	6	SS -> MS	AUTHENTICATION REQ
	7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	10	SS		SS starts ciphering.
	11	MS -> SS	SETUP
	12	SS -> MS	CALL PROCEEDING
	13	SS -> MS	ALERTING
	14	MS		Depending on the PICS, an alerting
				indication is given.
	15	SS -> MS	ASSIGNMENT COMMAND
	16	MS -> SS	ASSIGNMENT COMPLETE
	17	SS -> MS	CONNECT
	18	MS -> SS	CONNECT ACKNOWLEDGE
	19	MS		The appropriate bearer channel is
				through connected in both directions.

Specific Message Contents:

None

### 26.9.4	Structured procedures / MS terminated call / early assignment

26.9.4.1	Conformance requirements

1)	The MS shall acknowledge the SETUP message with a CALL CONFIRMED message, if compatibility checking was successful, the MS is not busy, and the user does not refuse the call.

2,3)	The MS on acceptance of the call sends a CONNECT, otherwise user alerting is initiated (the MS sends an ALERTING message and generates an alerting indication). ASSIGNMENT COMMAND is answered by ASSIGNMENT COMPLETE.

4)	An MS indicates acceptance of a MT call by sending CONNECT.

5)

For speech calls:

The mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment shall be delayed until such a resource becomes available.

For data calls:

The mobile station shall attach the user connection when receiving the CONNECT ACKNOWLEDGE message from the network.

6)	The MS initiates call clearing of an active call by sending a DISCONNECT message.

7)	The MS in this phase of call release, upon receipt of a RELEASE message, shall return a RELEASE COMPLETE message.

8)	Subsequently the MS, upon receipt of a CHANNEL RELEASE message, shall disconnect the main signalling link.

References

Conformance requirement 1:		TS GSM 04.08, section 5.2.2.3.1.

Conformance requirements 2,3:	TS GSM 04.08, section 5.2.2.3.2, 3.4.3.1.

Conformance requirement 4:		TS GSM 04.08 section 5.2.2.5.

Conformance requirement 5:		TS GSM 04.08, section 5.2.2.6 and 5.2.2.9.

Conformance requirements 6,7,8:	TS GSM 04.08, section 5.4.

26.9.4.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, after being paged by the network on the correct paging subchannel, after initiating the immediate assignment procedure by sending the CHANNEL REQUEST message, after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after having sent a PAGING RESPONSE message on the allocated SDCCH, after having performed successful authentication and cipher mode setting procedures, after receipt of a SETUP message not containing a signal information element, returns a CALL CONFIRMED message.

2)	To verify that subsequently, the SS sending an ASSIGNMENT COMMAND message, the MS successfully continues a mobile terminating call establishment with early assignment of traffic channel

a)	by replying to the ASSIGNMENT command with an ASSIGNMENT COMPLETE message, and

b)	by continuing the call establishment by either sending one or two CONNECT messages (with equal N(SD)) or sending an ALERTING message, steps a) and b) being performed in any permitted interleaving.

3)	To verify that if after sending a CALL PROCEEDING message, the MS sends an ALERTING message during MTC establishment with early assignment, it generates an alerting indication.

4)	To verify that if an ALERTING had been sent, subsequently, when the user accepts the call (possibly internal action as declared in PICS/PIXIT statement), the MS returns a CONNECT message.

5)	To verify that the MS:

-	if the call is a speech call: after sending the CONNECT message has through connected the TCH in both directions (this is checked by verifying that after transmission of the first L2 frame containing the (complete) CONNECT message, the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

-	if the call is a data call: after receipt of a subsequent CONNECT ACKNOWLEDGE message through connects the TCH in both directions (this is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT ACKNOWLEDGE message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

6)	To verify that subsequently, the MS can initiate call clearing by sending a DISCONNECT message.

7)	To verify that the MS in this phase of call release, upon receipt of a RELEASE message, returns a RELEASE COMPLETE message.

8)	To verify that subsequently the MS, upon receipt of a CHANNEL RELEASE message, disconnects the main signalling link.

These test purposes are tested for all rates supported by the MS (full rate/half rate).

26.9.4.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Immediate connect supported (Y/N)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged and the resulting call is established. Having reached the active state, the MS is made to clear the call.

Maximum Duration of Test

1 minute

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging subchannel
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	5	SS -> MS	AUTHENTICATION REQ
	6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	9	SS		SS starts ciphering.
	10	SS -> MS	SETUP	Message does not contain the  signal IE.
	11	MS -> SS	CALL CONFIRMED	
	12	SS -> MS	ASSIGNMENT COMMAND

A13	MS -> SS	ASSIGNMENT COMPLETE	During step 12 or  between step 12 and step 13
				the MS may send a CONNECT message
				on the old channel.
	A14	MS -> SS	CONNECT	If during step 12 or  between step 12 and step A13
				the MS has send a CONNECT message on the old 
				channel, both CONNECT messages must have 
				equal N(SD).

B13	MS -> SS	ASSIGNMENT COMPLETE	Sent on the new channel.
	B14	MS -> SS	ALERTING	
	B15	MS		An alerting indication as defined  in an PICS/PIXIT 
				statement  is given by the MS.
	B16	MS		The MS is made to accept the call in the way 
				described in a PICS/PIXIT statement.		
	B17	MS -> SS	CONNECT

18	MS		If the call is a speech call,
				the TCH shall be through connected 
				in both directions.
 19	SS -> MS	CONNECT ACKNOWLEDGE
	20	MS		If the call is a data call, 
				the TCH shall be through
				connected in both
				directions.
	21	MS		The MS is made to release the call.
	22	MS -> SS	DISCONNECT
	23	SS -> MS	RELEASE
	24	MS -> SS	RELEASE COMPLETE
	25	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

### 26.9.5	Structured procedures / MS terminated call / late assignment

26.9.5.1	Conformance requirement

TP1,TP2:	The MS shall acknowledge the SETUP message with a CALL CONFIRMED message, if compatibility checking was successful, the MS is not busy, and the user does not refuse the call. The MS on acceptance of the call sends a CONNECT, otherwise user alerting is initiated.

TP3:	The MS indicates acceptance of a call by sending a CONNECT message.

TP4:	ASSIGNMENT COMMAND is answered by ASSIGNMENT COMPLETE.

TP5:

For speech calls:

The mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment shall be delayed until such a resource becomes available.

For data calls:

The mobile station shall attach the user connection when receiving the CONNECT ACKNOWLEDGE message from the network.

Requirement reference:

Conformance requirements 1, 2, 3:		TS GSM 04.08, sections 5.2.2.3.1, 5.2.2.3.2, 5.2.2.5.

Conformance requirement 4:		TS GSM 04.08, section 3.4.3.1.

Conformance requirement 5:		TS GSM 04.08, section 5.2.2.9.

26.9.5.2	Test purpose

1)	To verify that the MS in "Idle, Updated" state with a TMSI assigned, after being paged by the network on the correct paging subchannel, after initiating the immediate assignment procedure by sending the CHANNEL REQUEST message, after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after having established the main signalling link, after having sent a PAGING RESPONSE message, after having performed successful authentication and cipher mode setting procedures, after receipt of a SETUP message containing a signal information element, returns a CALL CONFIRMED message followed by

-	an ALERTING message

-	or a CONNECT message.

2)	To verify that in the situation of test purpose 1, if the MS sends an ALERTING message, the MS generates an alerting indication in the way described in a PICS/PIXIT statement.

3)	To verify that subsequently the MS, if it had not yet sent a CONNECT message, upon acceptance of the call, sends a CONNECT message.

4)	To verify that subsequently after receipt of an ASSIGNMENT COMMAND, the MS sends an ASSIGNMENT COMPLETE message.

5)	To verify that subsequently the MS

-	if the call is a speech call: after sending the ASSIGNMENT COMPLETE message has through connected the TCH in both directions (this is checked by verifying that after transmission of the first L2 frame containing the (complete) ASSIGNMENT COMPLETE message, the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

-	if the call is a data call: after receipt of a subsequent CONNECT ACKNOWLEDGE message through connects the TCH in both directions (this is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT ACKNOWLEDGE message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

These test purposes are tested for all rates supported by the MS (full rate/half rate).

26.9.5.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Immediate connect supported (Y/N)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

CC state U10-call active.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged and a MT call is established with late assignment (after CONNECT).

Maximum Duration of Test

40 seconds

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	Sent on the correct paging
				subchannel
	2	MS -> SS	CHANNEL REQUEST	Establishment cause indicates
				"answer to paging".
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	5	SS -> MS	AUTHENTICATION REQ
	6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. 
				All following
				messages shall be sent enciphered.
	9	SS		SS starts ciphering.
	10	SS -> MS	SETUP	Message contains the signal IE.
	11	MS -> SS	CALL CONFIRMED

A12	MS -> SS	CONNECT

B12	MS -> SS	ALERTING	
	B13	MS		An alerting indication as defined 
				in an PICS/PIXIT statement 
				is given by the MS.
	B14	MS		The MS is made to accept the call
				in the way described in a
				PICS/PIXIT statement.
	B15	MS -> SS	CONNECT

16	SS -> MS	ASSIGNMENT COMMAND	
	17	MS -> SS	ASSIGNMENT COMPLETE	
	18	MS		If the call is a speech call,
				the TCH shall be through connected
				in both directions.
 	19	SS -> MS	CONNECT ACKNOWLEDGE
	20	MS		If the call is a data call, 
				the MS shall 
				through connect the TCH
				in both directions.

Specific Message Contents:

None

### 26.9.6	Structured procedures / emergency call

Emergency call establishment can be initiated by an MS whether location updating has been successful or not and whether a SIM is inserted into the MS or not; but only if the MS is equipped for speech.

If the procedures tested in this section are not correctly implemented in the MS, establishment, maintenance and clearing of connections might fail in the essential case of emergency calls.

The tests of this section are only applicable to an MS supporting speech.

#### 26.9.6.1	Structured procedures / emergency call / idle updated

##### 26.9.6.1.1	Structured procedures / emergency call / idle updated / preferred channel rate

26.9.6.1.1.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct CKSN and TMSI , with CM Service Type "emergency call establishment".

3)	Authentication and cipher mode setting shall be performed successfully.

4)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

5),6)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

7)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

8)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 3.4.7, TS GSM 04.08 section 4.3.2.

For conformance requirement 4:

TS GSM 04.08, section 5.2.1.1.

For conformance requirement 5 and 6:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 7:

TS GSM 04.08, sections 5.2.1.6 and 5.1.3.

For conformance requirement 8:

TS GSM 04.08, section 5.4.

26.9.6.1.1.2	Test purpose

1)	To verify that an MS supporting speech in the MM state "idle, updated", when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message specifying the correct CKSN and TMSI, with CM Service Type "emergency call establishment" .

3)	To verify that authentication and cipher mode setting are performed successfully.

4)	To verify that after cipher mode setting acceptance by the SS, the MS sends an EMERGENCY SETUP message.

5)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure of an appropriate speech traffic channel, which, if the MS supports both TCH/FS and TCH/HS, is at the preferred rate, the MS performs correctly that assignment procedure.

6)	To verify subsequent correct performance of a connect procedure.

7)	To verify that subsequently the MS has through connected the TCH in both directions.

8)	To verify that the call is cleared correctly.

26.9.6.1.1.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Supported rate for speech: (p1 = F/H, F).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The MS is made to initiate an emergency call. The call is established with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	3	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				emergency call establishment.
	4	SS -> MS	IMMEDIATE ASSIGNMENT	
	5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
	6	SS -> MS	AUTHENTICATION REQ
	7	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	8	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	9	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	10	SS		SS starts ciphering.
	11	MS -> SS	EMERGENCY SETUP	If p1 = F/H, the message must contain
				one bearer capability IE indicating 
				in the radio channel requirement field
				"dual rate/half rate preferred" or 
				"dual rate/full rate preferred". 

				If p1 = F, the message must either
				contain no bearer capability IE or
				contain one bearer capability IE 
				indicating in the 
				radio channel requirement field 
				"full rate channel".
	12	SS -> MS	CALL PROCEEDING
	13	SS -> MS	ALERTING
	14	SS -> MS	ASSIGNMENT COMMAND	The rate of the channel is that one
				indicated by the EMERGENCY SETUP
				message, if that message did not 
				offer a choice, and the rate is the
				preferred one else.
	15	MS -> SS	ASSIGNMENT COMPLETE
	16	SS -> MS	CONNECT
	17	MS -> SS	CONNECT ACKNOWLEDGE
	18	MS		The TCH is
				through connected in both directions.
	19	SS -> MS	DISCONNECT
	20	MS -> SS	RELEASE
	21	SS -> MS	RELEASE COMPLETE
	23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

##### 26.9.6.1.2	Structured procedures / emergency call / idle updated, non-preferred channel rate

The test is performed if the MS supports both TCH/HS and TCH/FS see PICS/PIXIT statement).

It is identical to the test in 26.9.6.1.1 except that in step 14 the assigned TCH has the non-preferred rate.

#### 26.9.6.2	Structured procedures / emergency call / idle, no IMSI

##### 26.9.6.2.1	Structured procedures / emergency call / idle, no IMSI / accept case

26.9.6.2.1.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

4),5)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

6)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

7)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 5.2.1.1.

For conformance requirements 4 and 5:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 6:

TS GSM 04.08, sections 5.2.1.6 and 5.1.3.

For conformance requirement 7:

TS GSM 04.08, section 5.4.

26.9.6.2.1.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message in which the cipher key sequence number IE indicates "no key is available", the CM service type IE indicates "emergency number establishment", and the mobile identity IE specifies the IMEI of the MS.

3)	To verify that after receipt of a CM SERVICE ACCEPT message from the SS, the MS sends an EMERGENCY SETUP message.

4)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure of an appropriate speech traffic channel, which, if the MS supports both TCH/FS and TCH/HS, is at the preferred rate, the MS performs correctly that assignment procedure.

5)	To verify subsequent correct performance of a connect procedure.

6)	To verify that subsequently the MS has through connected the TCH in both directions.

7)	To verify that the call is cleared correctly.

26.9.6.2.1.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	3	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"emergency call".
	4	SS -> MS	IMMEDIATE ASSIGNMENT	
	5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
				The mobile identity IE specifies the
				IMEI of the MS.
				The cipher key sequence number IE
				indicates "no key is available".
				The mobile station classmark IE is as
				specified by the manufacturer in a
				PICS/PIXIT statement.
	4	SS -> MS	CM SERVICE ACCEPT	
	11	MS -> SS	EMERGENCY SETUP	
	12	SS -> MS	CALL PROCEEDING
	13	SS -> MS	ALERTING
	14	SS -> MS	ASSIGNMENT COMMAND	The rate of the channel is one
				indicated by the EMERGENCY SETUP
				message.
	15	MS -> SS	ASSIGNMENT COMPLETE
	16	SS -> MS	CONNECT
	17	MS -> SS	CONNECT ACKNOWLEDGE
	18	MS		The TCH is through connected in both
				directions.
	19	SS -> MS	DISCONNECT
	20	MS -> SS	RELEASE
	21	SS -> MS	RELEASE COMPLETE
	23	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

##### 26.9.6.2.2	Structured procedures / emergency call / idle, no IMSI / reject case

26.9.6.2.2.1	Conformance requirement

1)	The MS in the "idle, no IMSI" state (no SIM inserted), after the number 112 has been entered, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	In the situation at the end of test purpose 2, when the MS receives a CM SERVICE REJECT message, it shall abandon the emergency call.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4

For conformance requirement 3:

TS GSM 04.08 section 3.4.7, TS GSM 04.08 section 4.5.1.1.

26.9.6.2.2.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message in which the cipher key sequence number IE indicates "no key is available", the CM service type IE indicates "emergency call establishment", and the mobile identity IE specifies the IMEI of the MS.

3)	To verify that after receipt of a CM SERVICE REJECT message from the SS, the MS abandons the emergency call establishment.

26.9.6.2.2.3	Method of test

Related PICS/PIXIT statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. The SS responds to the CM SERVICE REQUEST from the MS with a CM SERVICE REJECT message specifying in the reject cause IE the reject cause value "IMEI not accepted". The SS then verifies for during 5 seconds that the MS does not send a layer 3 message. Then the call is cleared by the SS. The SS verifies during 20 seconds after disconnection of the main signalling link that the MS does not initiate an RR connection establishment.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	3	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"emergency call".
	4	SS -> MS	IMMEDIATE ASSIGNMENT	
	5	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
				The mobile identity IE specifies the
				IMEI of the MS.
				The cipher key sequence number IE
				indicates "no key is available".
				The mobile station classmark IE is as
				specified by the manufacturer in a
				PICS/PIXIT statement.
	4	SS -> MS	CM SERVICE REJECT	the reject cause IE specifies 
				reject cause value #5, 
				"IMEI not accepted".
	5	SS		During 5 seconds, the SS verifies
				that the MS does not send L3 messages.
	6	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	7	SS		During 20 seconds, the SS verifies
				that the MS does not initiate an 
				RR connection establishment.

Specific Message Contents:

-

### 26.9.7	Default contents of messages

ALERTING (mobile station to network direction)

Information element			Value/remark

Facility					Omitted

User-user 				Omitted

SS version				Omitted

ALERTING (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

User-user 				Omitted

ASSIGNMENT COMMAND

Information element			Value/remark

Description of the first channel		describes non-hopping Bm+ACCHs or Lm+ACCHs as

appropriate for the test

Power Command				As in section 26.1.1

Frequency list				Omitted

Cell channel description			Omitted

Mode of the first channel 			appropriate for on bearer capability chosen for the test

Description of the second channel		Omitted

Mode of the second channel			Omitted

Mobile allocation				Omitted

Starting time				Omitted

Cipher mode setting			Omitted

ASSIGNMENT COMPLETE

Information element			Value/remark

RR cause				normal event

AUTHENTICATION REQUEST

Information element			Value/remark

Ciphering key sequence number		Arbitrary

Spare half octet				(spare bits)

Authentication parameter RAND		Arbitrary

AUTHENTICATION RESPONSE

Information element			Value/remark

Authentication parameter SRES		Correct for given SRES

CALL CONFIRMED

Information element			Value/remark

Repeat indicator				Omitted

Bearer capability 1 			Omitted if the MS only supports TCH/F. Included if the

MS supports both TCH/F and TCH/H; in that case, the

"radio channel requirement" field specifies "dual

rate/half rate preferred" or "dual rate/full rate

preferred" or "half rate channel", all other parameters

are same as in the bearer capability 1 IE of the

received SETUP message.

Bearer capability 2				Omitted

Cause					Omitted

CALL PROCEEDING

Information element			Value/remark

Repeat Indicator				Omitted

Bearer Capability 1			Omitted if the SETUP message did not specify in the

bearer capability 1 IE a connection element value

"both, transparent preferred" or "both, non-

transparent preferred". Otherwise included; in that

case the connection element specifies the value that is

appropriate for the selected teleservice (either value

"transparent" or value "non transparent (RLP)"), all

other parameters are same as in the bearer capability

1 IE of the received SETUP message.

Bearer Capability 2			Omitted

Facility					Omitted

Progress indicator				Omitted

CHANNEL RELEASE

Information element			Value/remark

RR cause  				Normal event

CHANNEL REQUEST

Information element			Value/remark

Establishment cause 			Answer to paging (100)

Random reference				Arbitrary value of 5 bits length

CIPHERING MODE COMMAND

Information element			Value/remark

Cipher mode setting

algorithm identifier			indicates a supported algorithm

SC					Start ciphering

Cipher response

CR					IMEI must not be included

CIPHER MODE COMPLETE

Information element			Value/remark

Mobile equipment identity			Omitted

CM SERVICE ACCEPT

Information element			Value/remark

none but message head

CM SERVICE REQUEST

Information element			Value/remark

CM service type				Mobile originating call

establishment or packet mode

connection establishment

Ciphering key sequence number		CKSN of the MS

Mobile station classmark 2			as given by PICS.

Mobile identity				TMSI of MS

CONNECT (network to mobile station direction)

Information element			Value/remark

Facility					Omitted

Progress indicator				Omitted

Connected number			Omitted

Connected subaddress			Omitted

User-user				Omitted

CONNECT (mobile station to network direction)

Information element			Value/remark

Facility					Omitted

Connected subaddress			Omitted

User-user				Omitted

SS version				Omitted

CONNECT ACKNOWLEDGE

Information element			Value/remark

none but message head

DISCONNECT (network to mobile station direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

Progress indicator				Omitted

User-user 				Omitted

DISCONNECT (mobile station to network direction)

Information element			Value/remark

Cause

Coding standard			GSM

Location				User

Cause value 			Normal clearing

Facility					Omitted

User-user 				Omitted

SS version				Omitted

IMMEDIATE ASSIGNMENT

Information element			Value/remark

Page mode 				Normal paging

Channel description			describes a valid SDCCH+SACCH

in non-hopping mode

Request reference

Random access information		As received from MS

N51, N32, N26			Corresponding to frame number

of the CHANNEL REQUEST

Timing advance				Arbitrary

Mobile allocation				Empty (L=0)

Starting time				Omitted

PAGING REQUEST TYPE 1

Information element			Value/remark

L2 pseudo length				L2 pseudo length of the message

Page Mode				Normal Paging

Channels needed for Mobiles 1 and 2

channel (first)			any channel

channel (second)			any channel

Mobile identity 1				TMSI of MS under test

Mobile identity 2				Omitted

P1 rest octets				(spare octets)

PAGING RESPONSE

Information element			Value/remark

Ciphering key sequence number		Value assigned to MS in the initial conditions

Spare half octet				(spare bits)

Mobile station classmark 2			as given by PICS

Mobile identity				specifies TMSI of MS

RELEASE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Second cause				Omitted

Facility					Omitted

User-user				Omitted

RELEASE (mobile station to network direction)

Information element			Value/remark

Cause					Omitted

Second cause				Omitted

Facility					Omitted

User-user				Omitted

SS version				Omitted

RELEASE COMPLETE (network to mobile station direction)

Information element			Value/remark

Cause					Omitted

Facility					Omitted

User-user				Omitted

RELEASE COMPLETE (mobile station to network direction)

Information element			Value/remark

Cause					Omitted

Facility					Omitted

User-user				Omitted

SS version				Omitted

SETUP (MS to SS)

Information element			Value/remark

BC Repeat indicator			Omitted

Bearer capability 1				Appropriate for the teleservice selected for the test

Bearer capability 2				Omitted

Facility					Omitted

Calling party subaddress			Omitted

Called party BCD number			As entered

Called party subaddress			Omitted

LLC repeat indicator			Omitted

Low layer compatibility I			Appropriate for teleservice selected for the test

Low layer compatibility II			Omitted

HLC repeat indicator			Omitted

High layer compatibility i			Appropriate for teleservice selected for the test

High layer compatibility ii			Omitted

User-user				Omitted

SS version				Omitted

CLIR suppression				Omitted

SETUP (SS to MS)

Information element			Value/remark

BC repeat indicator			Omitted

Bearer capability 1				Appropriate for teleservice selected for the test

Bearer capability 2				Omitted

Facility					Omitted

Progress indicator				Omitted

Signal					Omitted

Calling party BCD number			Omitted

Calling party subaddress			Omitted

Called party BCD number			Omitted

Called party subaddress			Omitted

LLC repeat indicator			Appropriate for the teleservice selected for the test

Low layer compatibility I			Appropriate for teleservice selected for the test

Low layer compatibility II			Appropriate for the teleservice selected for the test

HLC repeat indicator			Appropriate for the teleservice selected for the test

High layer compatibility i			Appropriate for the teleservice selected for the test

High layer compatibility ii			Appropriate for the teleservice selected for the test

User-user				Omitted

## 26.10	E-GSM signalling

### 26.10.1	E-GSM signalling / general considerations

Section 26.10 only applies to E-GSM mobile stations.

Conformance requirements of section 26 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26 under the described GSM900 conditions.

The purpose of this extra section is to test the different procedures which may be impacted when some channel uses E-GSM frequency(s).

### 26.10.2	E-GSM signalling / RR

#### 26.10.2.1	E-GSM signalling / RR / Measurement

This test applies E-GSM mobile stations.

Conformance requirements of section 26.6.3 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.3 under the described GSM900 conditions.

The purpose of this extra section is to test the reporting of measurements in the case where cells use E-GSM frequency(s). Several coding formats may be used by the network in the SYSTEM INFORMATION message.

26.10.2.1.1	Conformance requirements

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the values in the MEASUREMENT REPORT message shall contain measurement results for the 6 th strongest cells on which the mobile is asked to report.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

26.10.2.1.2	Test purpose

To test that, when the SS gives information about neighbouring cells, the MS reports appropriate results.

26.10.2.1.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BSCC	ARFCN	Cell identity

Serving, S1	-60	1	3	002	0001H

Neighbour, N1	-85	1	5	990	0002H

Neighbour, N2	-80	1	7	1005	0003H

Neighbour, N3	-75	1	1	000	0004H

Neighbour, N4	-55	1	3	026	0005H

Neighbour, N5	-50	1	5	1020	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	1003	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed 6 times.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5, 5bis & 6  on the SACCH. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest present carriers of the supported band have been obtained.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, c= 1 to 6.

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT  are sent continuously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

SYSTEM INFORMATION TYPE 5:

Information Element			value/remark

Protocol Discriminator		RR management

Message Type			Sys Info 5.

Neighbour Cells Description

- EXT IND			Information Element carries only a part of the BA.

- BA-IND			0

for c=1, use range 128

to encode the following frequencies :

(26, 38)

for c=2, use range 256

to encode the following frequencies

(990, 1003, 1005)

for c=3, use range 512

to encode the following frequencies

(520, 990, 1003, 1005, 1020)

for c=4, use range 1024

to encode the following frequencies

(0, 26, 38, 990, 1003, 1005)

for c=5, use variable Bitmap

to encode the following frequencies

(0, 26, 38)

for c=6, use Bitmap 0

to encode the following frequencies

(26)

SYSTEM INFORMATION TYPE 5bis:

Information Element			value/remark

Protocol Discriminator		RR management

Message Type			Sys Info 5bis.

Neighbour Cells Description

- EXT IND			Information Element carries only a part of the BA.

- BA-IND			0

for c=1, use range 512

to encode the following frequencies :

(520, 990, 1003, 1005, 1020)

for c=2, use range 128

to encode the following frequencies

(0, 26, 38)

for c=3, use range 256

to encode the following frequencies

(0, 26, 38)

for c=4, use range 1024

to encode the following frequencies

(520, 1000, 1020)

for c=5, use variable Bitmap

to encode the following frequencies

(884, 990, 1003, 1005)

for c=6, use range 512

to encode the following frequencies

(520, 990, 1003)

SYSTEM INFORMATION TYPE 6:

Information Element			value/ remark

Protocol Discriminator		RR Management

Message Type			sys info 6

Cell Identity				default

LAI					default

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall not use DTX

- Radio-Link-Timeout	default

PLMN permitted			only NCC 1 permitted

MEASUREMENT REPORT:

Information Element			value/remark

Protocol Discriminator		RR Management

Transaction Identifier			0000

Message Type			MEASUREMENT REPORT

Measurement Results

BA-used			1

DTX-used			DTX was not used

RXLEV-FULL-SERVING-CELL	See note 1

RXLEV-SUB-SERVING-CELL	See note 1

MEAS-VALID			See note 3

RXQUAL-FULL-SERVING-CELL	See note 1

RXQUAL-SUB-SERVING-CELL	See note 1

NO-NCELL-M			n (see note 2)

RXLEV-NCELL-1		See note 1

BCCH-FREQ-NCELL-1		See note 2

BSIC-NCELL-1			Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2		See note 1

BCCH-FREQ-NCELL-2		See note 2

BSIC-NCELL-2			Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3		See note 1

BCCH-FREQ-NCELL-3		See note 2

BSIC-NCELL-3			Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4		See note 1

BCCH-FREQ-NCELL-4		See note 2

BSIC-NCELL-4			Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5		See note 1

BCCH-FREQ-NCELL-5		See note 2

BSIC-NCELL-5			Corresponds to that of BCCH-FREQ-NCELL-5

RXLEV-NCELL-6		See note 1

BCCH-FREQ-NCELL-6		See note 2

BSIC-NCELL-6			Corresponds to that of BCCH-FREQ-NCELL-6

NOTE 1:	These actual values are not checked.

NOTE 2 :	ARFCN 2 is the serving cell carrier.

c=1 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 1020, n=6

c=2 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0, 1020 stronger than 1005 but not broadcasted in SYS INFO), n=6

c=3 report on ARFCNs  2, 20, 26, 38, 1003, 1005, 1020, 0 (990 less strong, 520 DCS), n=6

c=4 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0 (1000 less strong, 520 DCS, 1020 not broadcasted in SYS INFO), n=6

c=5 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0 (884 DCS), n=6

c=6 report on 2, 26, 990, 1003,  n=6

NOTE 3:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

#### 26.10.2.2	E-GSM signalling / RR / Immediate assignment

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.1 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.1 under the described GSM900 conditions.

The purpose of this extra section is to test the immediate assignment procedure in the case where the target channel is E-GSM frequency.

26.10.2.2.1	Conformance requirement

Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on a supported channel described in the IMMEDIATE ASSIGNMENT message, using some E-GSM frequency.

Reference(s)

GSM 04.08 section 3.3.1.2.1

26.10.2.2.2	Test purpose

To verify that the MS can correctly set up a dedicated control channel when E-GSM frequencies are used.

This tested for a SDCCH/8

26.10.2.2.3	Method of test

Initial Conditions

System Simulator:

1 cell,

CCCH-CONF is set to '1 basic physical channel used for CCCH not combined with SDCCHs'.

SYSTEM INFORMATION type 1 message contains the following frequencies in the Cell Channel Description IE: 0, 30, 40, 66, 80, 1005, 1010, 1015 (use range 1024 to encode)

BCCH carrier number 1015

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statement(s)

none

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

This test procedure is performed twice.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST message the SS assigns an SDCCH channel using some E-GSM frequencies. The MS shall go to the correct channel and send a PAGING RESPONSE message. Then the SS initiates RR-release by sending a CHANNEL RELEASE message.

Maximum Duration of Test

15 seconds

Expected Sequence

The sequence is performed for execution counter k=1,2

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	cause "answer to paging"
	3	SS -> MS	IMM ASSIGNMENT	Channel Type : see below
	4	MS -> SS	PAGING RESPONSE	Shall be sent on the correct channel
	5	SS -> MS	CHANNEL RELEASE

Specific Message Contents

IMMEDIATE ASSIGNMENT

as default except :

Information element				remark/value

Channel description

- Channel Type 			SDCCH/8

- Timeslot number			arbitrary but not zero

- Training sequence code			arbitrary

- Hopping channel			k=1 Single RF

k=2	RF hopping channel

- Channel Selector			k=1  ARFCN=1015

k=2 MAIO = arbitrarily chosen

HSN arbitrary chosen from the set (1,..63)

Mobile allocation				k=1 empty

k=2 indicates the following frequencies

(0, 80, 1005, 1010)

#### 26.10.2.3	E-GSM signalling / RR / channel assignment procedure

This test is only applicable to an E-GSM mobile station.

Conformance requirements of section 26.6.4 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.4 under the described GSM900 conditions.

The purpose of this extra section is to test the assignment in the case where the allocated channel uses E-GSM frequency(s) : in the case of frequency hopping several coding formats may be used by the network in the ASSIGNMENT COMMAND message.

26.10.2.3.1	Conformance requirements

1.	Upon receipt of the ASSIGNMENT COMMAND message, the mobile station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the main signalling links).

2.	The MS shall apply the hopping frequencies specified in ASSIGNMENT COMMAND message in the mobile allocation or frequency list or frequency short list at the time accessing the new channel.

References

GSM 04.08, subclause  3.4.3

GSM 04.08 subclause 9.1.2

26.10.2.3.2	Test purpose

1.	To verify that upon receipt of an ASSIGNMENT COMMAND, the MS switches to the channel defined in the ASSIGNMENT COMMAND, establishes the link and sends an ASSIGNMENT COMPLETE message.

2.	To verify that an MS, having received an ASSIGNMENT COMMAND, is able in case of frequency hopping to decode the mobile allocation and frequency list correctly and applies the specified frequencies.

26.10.2.3.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state with a TMSI allocated.

Related PICS/PIXIT Statements

-	TCH supported (Y/N)

-	The supported channel mode(s) need to be declared.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The test procedure is performed 2 times.

The SS pages the MS and allocates an SDCCH. Then a channel is assigned with ASSIGNMENT COMMAND. Each time the MS shall switch to the assigned channel, establish the link and send an ASSIGNMENT COMPLETE message.

The SS initiates the channel release procedure.

Maximum Duration of Test

3 min

Expected Sequence

The test sequence is performed for execution counter k=1 to 2

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	
	2	MS -> SS	CHANNEL REQUEST	
	3	SS -> MS	IMMEDIATE ASSIGN	Timeslot Number = n.
	4	MS -> SS	PAGING RESPONSE

5	SS -> MS	ASSIGN COMMAND	See specific message contents
				Timeslot Number = (n+1) mod 8
	6	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after 
				establishment of the main signalling link.
				Steps 5 and 6 are repeated cmax times, where 
				cmax is the number of frequency formats allowed
				for each value of k.
				Use repetition counter c:
				See specific message content.

7	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents

ASSIGNMENT COMMAND:

Information element			value/remark

Protocol Discriminator			RR

Skip indicator				0000

Message type				ASSIGNMENT COMMAND

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Power Command

- Power level			Arbitrarily chosen

For k=1

Cell Channel Description IE is not included

Frequency list				for c=1, use range 128

to encode the following frequencies :

(1005, 1010, 1015)

for c=2, use range 256

to encode the following frequencies

(73, 74, 75, 76, 77)

for c=3, use range 512

to encode the following frequencies

(980, 981, 982, 983)

for c=4, use range 1024

to encode the following frequencies

(30, 40, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 991, 992, 993, 994, 1015)

with an encoding origin set to 900

for c=6, use Bitmap 0

to encode the following frequencies

(20, 40, 66)

For k=2

Cell Channel Description 			for c=1, use range 128

to encode the following 13 frequencies :

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=2, use range 256

to encode the following 13 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108,

114, 115)

for c=3, use range 512

to encode the following 13 frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=4, use range 1024

to encode the following 8 frequencies

(0, 30, 40, 66, 80, 1005, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=6, use Bitmap 0

to encode the following 12 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114)

For k = 2

Mobile Allocation				indicates the following the frequencies :

for c=1 (1005, 1010, 1015)

for c=2 (73, 74, 75, 76, 77)

for c=3 (980, 981, 982, 983)

for c=4 (0, 30, 40, 1010, 15)

for c=5 (990, 991, 992, 993, 994)

for c=6 (20, 40, 66)

Mode of the First channel			speech full rate for TCH/F

except if speech is not supported :

arbitrary from those supported

Starting Time				not included

#### 26.10.2.4	E-GSM signalling / RR / Handover

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.5 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.5 under the described GSM900 conditions.

The purpose of this extra section is to test the handover in the case where the target channel uses E-GSM frequency(s) : in the case of frequency hopping several coding formats may be used by the network in the HANDOVER COMMAND message.

##### 26.10.2.4.1	E-GSM signalling / RR / Handover / Successful handover

26.10.2.4.1.1	Conformance requirements

The MS shall correctly apply the handover procedure from a channel without frequency hopping in the primary band to a channel with frequency hopping using P-GSM and E-GSM frequencies whatever the coding used for the frequency hopping description.

References

GSM 04.08, section 3.4.4 and 9.1.15

26.10.2.4.1.2	Test purpose

To check that the MS correctly performs a non-synchronized handover, from a non hopping primary band SDCCH to a hopping TCH or SDCCH using E-GSM frequencies, whatever the coding used for the hopping sequence description and that it activates the new channel correctly.

This is tested in the following case :

E-GSM signalling / Handover / successful / call under establishment / non-synchronized /

- from SDCCH/8 to TCH/F if the MS supports a TCH

- from SDCCH/8 to SDCCH/8 if not

26.10.2.4.1.3	Method of test

Initial Conditions

System Simulator:

2 cells A and B with same LAI, default parameters

except for Cell A : the broadcasted Cell Channel Description in SYSTEM INFORMATION type 1 message contains the following frequencies : 20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114 (encoded using Bit Map 0 format)

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Related PICS/PIXIT Statements

Support of TCH (Y/N)

Support of speech (Y/N)

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

"Idle, updated" with TMSI allocated and camped on cell B.

Test Procedure

The test procedure is performed 18 times.

A Mobile Originating Call is initiated on cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND message, ordering the MS to switch to cell B.

The MS shall then begin to send access bursts on the new channel, a TCH/F if supported (see PICS/PIXIT) or an SDCCH if not, to cell B.

The SS observes the access bursts. After receiving n access bursts, n being randomly drawn between 10 and 20 for the TCH case, 2 and 5 for the SDCCH (see table 2 of section 26.6.5) , the SS sends one PHYSICAL INFORMATION message with a Timing Advance of 20 (see table 1 of section 26.6.5).

The MS shall activate the channel in sending and receiving mode. Then the MS shall establish a signalling link using the correct timing advance. The MS shall be ready to transmit a HANDOVER COMPLETE message before x ms after the end of the PHYSICAL INFORMATION message but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

The term 'ready to transmit' is specified in TS GSM 04.13. The value of 'x' depends upon the target channel:

case SDCCH/8 x = 750

case TCH/F x=500

Maximum Duration of Test

10 min

Expected Sequence

The sequence is performed for execution counter k=1 to 3 and c=1 to 6

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated on cell A
	2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call,
				NECI not set to 1
	3	SS -> MS	IMM ASSIGNMENT	see specific message contents
	4	MS -> SS	CM SERVICE REQUEST	CM Service Type = Mobile Origina-
				ting Call Establishment.
	5	SS -> MS	CIPH MODE COMMAND
	6	MS -> SS	CIPH MODE COMPLETE
	7	MS -> SS	SETUP	Last L2 frame not acknowledged by
				the SS.
	8	SS -> MS	HANDOVER COMMAND	see specific message contents
	9	MS -> SS	HANDOVER ACCESS	Repeated  on every burst
				of the uplink main DCCH until reception
				of PHYSICAL	INFORMATION.
				Handover Reference as
				included in the HANDOVER COMMAND
	10	SS -> MS	PHYSICAL INFO	Sent after reception of n HANDOVER
				ACCESS. Timing Advance : 20
	11	MS -> SS	SABM	Sent without information field
	12	SS -> MS	UA
	13	MS -> SS	HANDOVER COMPLETE	This message shall be ready
				to be transmitted before 'x' ms
				after the completion of step 10
	14	MS -> SS	SETUP	Same N(SD) as in step 7.
	15	SS -> MS	CHANNEL RELEASE

Specific Message Contents

IMMEDIATE ASSIGNMENT

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		20

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Mobile Allocation				indicates the following frequencies :

(20, 66, 78)

HANDOVER COMMAND

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		40

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Synchronisation Indication IE is not included

For k = 1,

Cell Channel Description IE is not included

Frequency short list			for c=1, use range 128

to encode the following frequencies :

(1005, 1010, 1015)

for c=2, use range 256

to encode the following frequencies

(73, 74, 75, 76, 77)

for c=3, use range 512

to encode the following frequencies

(980, 981, 982, 983)

for c=4, use range 1024

to encode the following frequencies

(30, 40, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 991, 992, 993, 994, 1015)

For k=2

Cell Channel Description IE is not included

Frequency list				for c=1, use range 128

to encode the following frequencies :

(1005, 1010, 1015)

for c=2, use range 256

to encode the following frequencies

(73, 74, 75, 76, 77)

for c=3, use range 512

to encode the following frequencies

(980, 981, 982, 983)

for c=4, use range 1024

to encode the following frequencies

(30, 40, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 991, 992, 993, 994, 1015)

for c=6, use Bitmap 0

to encode the following frequencies

(20, 40, 66)

For k=3

Cell Channel Description 			for c=1, use range 128

to encode the following 13 frequencies :

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=2, use range 256

to encode the following 13 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108,

114, 115)

for c=3, use range 512

to encode the following 13 frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=4, use range 1024

to encode the following 8 frequencies

(0, 30, 40, 66, 80, 520, 975, 1005, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=6, use Bitmap 0

to encode the following 12 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114)

For k = 3

Mobile Allocation				indicates the following the frequencies :

for c=1 (1005, 1010, 1015)

for c=2 (73, 74, 75, 76, 77)

for c=3 (980, 981, 982, 983)

for c=4 (30, 40, 1010, 1015)

for c=5 (990, 991, 992, 993, 994)

for c=6 (20, 40, 66)

Mode of the First channel			speech full rate for TCH/F

except if speech is not supported :

arbitrary from those supported for SDCCH/8

##### 26.10.2.4.2	E-GSM signalling / RR / Handover / layer 1 failure

26.10.2.4.2.1	Conformance requirements

During a handover from a channel in the E-GSM band to a channel in the P-GSM band, or the contrary, the MS shall correctly return to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell.

References

GSM 04.08 section 3.4.4.

26.10.2.4.2.2	Test purpose

To check  that the MS correctly returns to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell, even if the origin is in the P-GSM band and the target in the E-GSM band.

26.10.2.4.2.3	Method of test

Initial Conditions

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call on a P-GSM channel of cell A. power level = 10

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/FS

Supported mode(s).

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell A), used power level 10.

Test Procedure

The MS is in the active state (U10) of a call on a P-GSM channel of cell A (used power level 10). The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B (power level 12). With the exception of normal BCCH signalling, the SS does not transmit anything on cell B (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test

1 min

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	to an E-GSM channel
				see specific message contents
	2	MS -> SS	HANDOVER ACCESS	Several messages are sent, all
				with correct Handover References.
	3	MS -> SS	HANDOVER FAILURE	Sent on old channel,
				RR cause value = “Abnormal release,
				unspecified", "Abnormal release,
				channel unacceptable", "Abnormal
				release, timer expired" or
				"Abnormal release, no activity on
				the radio path". Layer 1 header
				has the same power level as the
				layer 1 header in step 1.
				Shall be sent
				within 3 seconds from the trans-
				mission of HANDOVER COMMAND.

Specific Message Contents

HANDOVER COMMAND

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		40

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				chosen randomly from the set

(1,2...63)

Synchronisation Indication IE is not included

Cell Channel Description IE is not included

Frequency short list			use range 128

to encode the following frequencies :

(1005, 1010, 1015)

Mode of the first channel 			Full rate speech if supported

If not, arbitrary from those supported except signalling.

#### 26.10.2.5	E-GSM signalling / RR / Frequency Redefinition

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.6 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.6 under the described GSM900 conditions.

The purpose of this extra section is to test the frequency redefinition procedure in the case where the target channel uses E-GSM frequency(s) : in the case of frequency hopping several coding formats may be used by the network in the FREQUENCY REDEFINITION message.

26.10.2.5.1	Conformance requirements

1) 	To verify that the MS, after receiving a FREQUENCY REDEFINITION message, correctly starts using the new frequencies, and hopping sequence when some E-GSM frequencies are used.

2) 	The last received Cell Channel Description information element is used to decode the Mobile Allocation IE received on the FREQUENCY REDEFINITION message.

References

GSM 04.08 3.4.5 and 9.1.13.

26.10.2.5.2	Test purpose

1)	To verify that the MS, after receiving a FREQUENCY REDEFINITION message, starts using the new frequencies and hopping sequence when some E-GSM frequencies are used.

2)	To check that the last received Cell Channel Description information element is used to decode the Mobile Allocation IE received in the FREQUENCY REDEFINITION message.

26.10.2.5.3	Method of test

Initial conditions

System Simulator:

1 cell; default parameters.

Mobile Station:

The MS is in "idle, updated" state with TMSI allocated.

Related PICS/PIXIT statement(s)

- Support of a TCH (Y/N)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The test procedure is performed six times.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a TCH/F if supported otherwise a SDCCH/8. Then the SS sends to MS a FREQUENCY REDEFINITION. The MS shall then use the new frequencies/hopping sequence.

Maximum duration of test

3 min

Expected sequence

The sequence is performed for execution counter c=1 to 6

Step	Direction	Message	Comments

1	SS->MS	PAGE REQ TYPE 1
	2	MS->SS	CHANNEL REQUEST

3	SS->MS	IMMEDIATE ASSIGN	Channel type = TCH/F if a TCH is supported 
				otherwise, Channel type=SDCCH/8. 
	4	MS->SS	PAGING RESPONSE	Sent on the correct channel after 
				establishment of the main signalling link
	5	SS->MS	FREQUENCY REDEFINITION	See specific message contents.
	6	-----	------------------	The SS checks that the MS is transmitting on 
				the correct frequencies.

7	SS->MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents

FREQUENCY REDEFINITION

Information Element			Value/remark

As default message contents, except :

Cell Description

- BCCH carrier number		40

Channel Description

- Channel type			TCH/F + ACCHs if supported by the MS

or SDCCH/8 if not

- Timeslot number 			arbitrary but not zero

- Training sequence code 		chosen arbitrarily

- Hopping 				RF hopping channel

- MAIO				arbitrary

- HSN				arbitrarily chosen from the set

(1,2...63)

Cell Channel Description 			for c=1, use range 128

to encode the following 13 frequencies :

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=2, use range 256

to encode the following 13 frequencies

(20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108,

114, 115)

for c=3, use range 512

to encode the following 13 frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=4, use range 1024

to encode the following 8 frequencies

(0, 30, 40, 66, 80, 1005, 1010, 1015)

for c=5, use variable Bitmap

to encode the following frequencies

(980, 981, 982, 983, 990, 991, 992, 993, 994,

1000, 1005, 1010, 1015)

for c=6, not present

(the mobile station will use the last Cell Channel

Description IE received, i.e. the one broadcast in the

SYSTEM INFORMATION TYPE 1 message).

Mobile Allocation				indicates the following frequencies :

for c=1 (1005, 1010, 1015)

for c=2 (73, 74, 75, 76, 77)

for c=3 (980, 981, 982, 983)

for c=4 (30, 40, 1010, 1015)

for c=5 (990, 991, 992, 993, 994)

for c=6 (30, 50, 70)

Starting time				indicates

(current frame number + 100 frames) mod 42432

### 26.10.3	E-GSM signalling / Structured procedure

Conformance requirements of section 26.9 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.9 under the described GSM900 conditions.

The purpose of these tests is to verify that the MS performs certain elementary procedures of the RR, MM, and CC protocol correctly within a structured procedure. This tested here in an E-GSM context to evaluate the global behaviour of an E-GSM mobile station using its specific frequency capability.

One mobile originated call and one emergency call are performed.

In one of the different cases the different following situations occur : early and late assignments, call release initiated by the network and by the MS.

#### 26.10.3.1	E-GSM signalling / Structured procedure / Mobile originated call

26.10.3.1.1	Conformance requirement

1)	An MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS , shall start to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

2)	Upon receipt of the ASSIGNMENT COMMAND message, the Mobile Station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the data links). After the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message, specifying cause 'normal event', to the network on the main DCCH.

3,4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

References

Conformance requirement 1:	TS GSM 04.08 section 3.3.1.

Conformance requirement 2:	TS GSM 04.08 section 3.4.3.1, 3.4.3.2

Conformance requirement 3:	TS GSM 04.08 section 3.4.3.

Conformance requirement 4:	TS GSM 04.08 section 5.2.1.1.6.

Conformance requirement 5:	TS GSM 02.07.

Conformance requirement 6:	TS GSM 04.08 sections 5.2.1.1.6 and 5.1.3.

26.10.3.1.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message.

2)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed authentication and cipher mode setting procedures, after having sent a SETUP message, after having received a CALL PROCEEDING message followed by an ALERTING message and an ASSIGNMENT COMMAND message allocating an appropriate TCH, the MS sends an ASSIGNMENT COMPLETE message.

3)	To verify that subsequently, after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message returns a CONNECT ACKNOWLEDGE message.

4)	To verify that after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message attaches the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

26.10.3.1.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM state “idle updated”.

Test procedure

The following test is performed for one teleservice supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with late assignment. The release of the call is initiated by the MS.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" is entered
	3	MS		
	4	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"originating call and the
				network does not set the NECI bit to 1".
	5	SS -> MS	IMMEDIATE ASSIGNMENT	
	6	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM.
	7	SS -> MS	AUTHENTICATION REQ
	8	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	9	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending
				the message.
	10	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	11	SS		SS starts ciphering.
	12	MS -> SS	SETUP
	13	SS -> MS	CALL PROCEEDING
	14	SS -> MS	ALERTING
	15	MS		Depending on the PICS, an alerting
				indication is given.
	16	SS -> MS	ASSIGNMENT COMMAND
	17	MS -> SS	ASSIGNMENT COMPLETE
	18	SS -> MS	CONNECT
	19	MS -> SS	CONNECT ACKNOWLEDGE
	20	MS		The appropriate bearer channel is
				through connected in both directions.
	21	MS		If the call is a data call, 
				the TCH shall be through
				connected in both
				directions.
	22	MS		The MS is made to release the call.
	23	MS -> SS	DISCONNECT
	24	SS -> MS	RELEASE
	25	MS -> SS	RELEASE COMPLETE
	26	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

#### 26.10.3.2	E-GSM signalling / Structured procedures / emergency call

Emergency call establishment can be initiated by an MS whether location updating has been successful or not and whether a SIM is inserted into the MS or not; but only if the MS is equipped for speech.

This section is only applicable to an MS supporting speech.

26.10.3.2.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

4),5)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

6)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

7)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 5.2.1.2.

For conformance requirements 4 and 5:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 6:

TS GSM 04.08, sections 5.2.1.1.6 and 5.1.3.

For conformance requirement 7:

TS GSM 04.08, section 5.4.

26.10.3.2.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message specifying the correct CKSN and TMSI, with CM Service Type "emergency call establishment".

3)	To verify that after receipt of a CM SERVICE ACCEPT message from the SS, the MS sends an EMERGENCY SETUP message.

4)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure, the MS performs correctly that assignment procedure.

5)	To verify subsequent correct performance of a connect procedure.

6)	To verify that subsequently the MS has through connected the TCH in both directions.

7)	To verify that the call is cleared correctly.

This is tested in the following case :

Structured procedures / emergency call / idle, no IMSI / accept case

26.10.3.2.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters except :

BCCH ARFCN = 990

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute.

Expected Sequence

Step	Direction	Message	Comments

1	MS 		The "called number" 112 is entered
	2	MS -> SS	CHANNEL REQUEST	Establishment cause is 
				"emergency call".
	3	SS -> MS	IMMEDIATE ASSIGNMENT	
	4	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. The 
				CM service type IE indicates
				"emergency call establishment".
				The mobile identity IE specifies the
				IMEI of the MS.
				The cipher key sequence number IE
				indicates "no key is available".
				The mobile station classmark IE is as
				specified by the manufacturer in a
				PICS/PIXIT statement.
	5	SS -> MS	CM SERVICE ACCEPT	
	6	MS -> SS	EMERGENCY SETUP	
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	The rate of the channel is one
				indicated by the EMERGENCY SETUP
				message.
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT
	12	MS -> SS	CONNECT ACKNOWLEDGE
	13	MS		The TCH is through connected in both
				directions.
	14	SS -> MS	DISCONNECT
	15	MS -> SS	RELEASE
	16	SS -> MS	RELEASE COMPLETE
	17	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None

#### 26.10.3.3	Default contents of messages

same as in 26.9.7 except for the following :

ASSIGNMENT COMMAND

Information element			Value/remark

Description of the first channel		describes non-hopping Bm+ACCHs or Lm+ACCHs as

appropriate for the test

ARFCN = 990

Power Command

-					Chosen arbitrarily but within the range supported by the

MS.

Frequency list				Omitted

Cell channel description			Omitted

Mode of the first channel 			appropriate for on bearer capability chosen for the test

Description of the second channel		Omitted

Mode of the second channel			Omitted

Mobile allocation				Omitted

Starting time				Omitted

Cipher mode setting			Omitted

IMMEDIATE ASSIGNMENT

Information element			Value/remark

Page mode 				Normal paging

Channel description			describes a valid SDCCH+SACCH

in non-hopping mode

ARFCN = 40

Request reference

Random access information		As received from MS

N51, N32, N26			Corresponding to frame number

of the CHANNEL REQUEST

Timing advance				Arbitrary

Mobile allocation				Empty (L=0)

Starting time				Omitted

### 26.10.4	E-GSM signalling / Default message contents

Default SYSTEM INFORMATION:

NOTE :	SYSTEM INFORMATION 7, and SYSTEM INFORMATION 8 messages are not used.

SYSTEM INFORMATION type 1 message

Information elements			Value/Remark

Cell Channel Description

For Cell A

-	Format identifier		Bit Map 0

-	Cell Allocation ARFCN		Channel Numbers 20, 30, 50 and 70.

For Cell B

-	Format identifier		Bit Map 0

-	Cell Allocation ARFCN		Channel Numbers 10, 12, 40, 60, 62, 63

RACH Control parameters			see below

SI1 Rest octets				see below

SYSTEM INFORMATION type 2 message

Information elements			Value/Remark

BCCH frequency list

For cell A

-	Format identifier		bit map 0

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 10, 20, 80, 90,

100, 110 and 120

-	EXT-IND			This IE does not carry the complete BA

For cell B

-	Format identifier		bit map 0

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 10, 36, 40, 114, 118

-	EXT-IND			This IE does not carry the complete BA

NCC permitted				see below

RACH control parameters			see below

SYSTEM INFORMATION type 2bis message

Information elements			Value/Remark

Extended BCCH frequency list

For cell A

-	Format identifier		range 256

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		988, 990, 1003

-	EXT-IND			This IE does not carry the complete BA

For cell B

-	Format identifier		range 128

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 1005, 1010, 1015

-	EXT-IND			This IE does not carry the complete BA

RACH control parameters			see below

SI 2bis rest octets				see below

SYSTEM INFORMATION type 3 message

Information elements			Value/Remark

Cell identity				see below

LAI						see below

Control channel description			see below

Cell options				see below

Cell Selection parameters			see below

RACH control parameter			see below

SI3 Rest octets 				see below

SYSTEM INFORMATION type 4 message

Information elements			Value/Remark

LAI						see below

Cell selection parameters			see below

RACH control parameters 			see below

CBCH Channel Description			see below

CBCH MA				see below

SI4 Rest octets				see below

SYSTEM INFORMATION type 5 message

Information elements			Value/Remark

BCCH frequency list

For cell A

-	Format identifier		bit map 0

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 10, 20, 80, 90,

100, 110 and 120

-	EXT-IND			This IE does not carry the complete BA

For cell B

-	Format identifier		Bit map 0

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 10, 36, 40, 114, 118

-	EXT-IND			This IE does not carry the complete BA

SYSTEM INFORMATION type 5bis message

Information elements			Value/Remark

Extension of BCCH frequency list description

For cell A

-	Format identifier		Range 256

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 988, 990, 1003

-	EXT-IND			This IE does not carry the complete BA

For cell B

-	Format identifier		range 128

-	BCCH Allocation Sequence	0

-	BCCH Allocation ARFCN		Channels numbers 1005, 1010, 1015

-	EXT-IND			This IE does not carry the complete BA

SYSTEM INFORMATION type 6 message

Information elements			Value/Remark

Cell identity 				see below

LAI						see below

Cell options 				see below

NCC permitted				see below

Common contents of information elements in SYSTEM INFORMATION TYPE 1 to 6 messages.

(CBCH) Channel Description		Not present

(CBCH) Mobile Allocation			Not present

Cell Identity

-	Cell Identity Value		0001H for cell A,

0002H for cell B

Cell Options

-	Power Control Indicator		Power Control Indicator is not set

-	DTX Indicator			MS shall not use DTX

-	Radio-Link-Timeout		8 SACCH blocks

Cell Selection Parameters

-	Cell-Reselect-Hysteresis		12 dB

-	MX-TXPWR-MAX-CCH		Minimum level

-	ACS				No addition cell parameters are present in

SYSTEM INFORMATION messages 7 and 8.

-	NECI				New establishment causes not supported

-	RXLEV-ACCESS-MIN		Minimum level

Control Channel Description

-	Attach-Detach allowed		No Attach/Detach

-	BS-AG-BLKS-RES		0 blocks reserved for access grant

-	CCCH-CONF			1 basic physical channel used for

CCCH, combined with SDCCHs

-	BS-PA-MFRMS		5 multiframe periods for transmission of

paging messages

-	T3212 Timeout value		Infinite

L2 pseudo length

-	System information 1		21

-	System information 2		22

-	System information 2bis		21

-	System information 3		18

-	System information 4		12

Location Area Identification

-	Mobile Country Code		001 decimal

-	Mobile Network Code		01 decimal

-	Location Area Code		0001H

Message Type

-	System information 1		00011001

-	System information 2		00011010

-	System information 2bis		00000010

-	System information 3		00011011

-	System information 4		00011100

-	System information 5		00011101

-	System information 5bis		00000101

-	System information 6		00011110

NCC Permitted				0000 0010

RACH Control Parameters

-	Max Retrans			Max 1 retrans

-	Tx-integer			5 slots used

-	Cell Barred for Access		Cell is not barred

-	Call Reestablishment Allowed	Not allowed

-	Access Control Class		Access is not barred

-	Emergency Call allowed		Yes

SI 1 rest octets				Not used (all bits are set to spare)

SI 2 rest octets				Not used (all bits are set to spare)

SI 3 rest octets				Not used (all bits are set to spare)

SI 4 rest octets				Not used (all bits are set to spare)

Default settings for cell A:

Downlink input level			63 dBmicroVolt emf

Uplink output power			minimum supported by the MS's power class

Propagation profile			static

BCCH/CCCH carrier number		20

Default settings for cell B:

Downlink input level			53 dBmicroVolt emf

Uplink output power			minimum supported by the MS's power  class

Propagation profile			static

BCCH/CCCH carrier number		10

Default message contents for other messages

For sections 26.10.1 to 26.10.2.5 		same as in 26.6.14

For section 26.10.3			same as in 26.9.7