# 31	Test of supplementary services

The general aspects of the specification of supplementary services at the layer 3 radio interface are given in GSM 04.10.

The formats and coding are given in GSM 04.80. If the value of a parameter of an uplink message (MS to network) is specified in a test, the implicit meaning is that it has to be checked; if the value is not specified, it is not to be checked unless otherwise stated.

Unless otherwise stated, the MS shall be in the idle updated state at the beginning of each test (including repetition of a test).

In each test, before the MS sends the first REGISTER message, a MM connection is established.

GSM 04.81 to 04.88 give the procedures used at the radio interface for normal operation, registration, erasure, activation, deactivation, invocation and interrogation of supplementary services.

The supplementary services are described in GSM 02.04 and 02.81 to 02.88.

Whenever activation via the standard MMI is mentioned, if the MS doesn't support it but supports a different procedure, this different procedure is used. In the supplementary services tests, only the applicable (see GSM 02.8x series) MMI service code groups (GSM 02.30 Annex 4), of the basic service code in GSM 09.02,which are supported by the MAP, will be used.

In all the following test cases, the MS will not be failed if it doesn't send a FACILITY information element in the initial REGISTER message, but in a subsequent FACILITY message (although the first case is the normal behaviour).

## 31.1	Number identification supplementary services

(Reserved).

## 31.2	Call offering supplementary services

The following abbreviations are used:

CFU :	Call Forwarding Unconditional

CFB :	Call Forwarding on mobile subscriber Busy

CFNRy :	Call Forwarding on No Reply

CFNRc :	Call Forwarding on mobile subscriber Not Reachable

CFC :	Call Forwarding Conditional (common name for CFB, CFNRy and CFNRc)

CF :	Call Forwarding (common name for CFU, CFB, CFNRy and CFNRc).

NOTE:	These abbreviations are also used to represent the corresponding SS-Code; e.g. CFC is the SS-Code for all conditional forwarding services.

### 31.2.1	Call forwarding supplementary services

#### 31.2.1.1	Registration

##### 31.2.1.1.1	Registration accepted

31.2.1.1.1.1	Conformance requirements

For registration of any type of call forwarding with any parameters, the MS shall transmit successively

1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

3)	and then the REGISTER message containing a facility IE (or an empty REGISTER followed by a FACILITY message containing the facility IE) that includes an invoke of the RegisterSS operation with  parameter values according to the user's request (MMI action).

4)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.2.1.1.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for registration of call forwarding in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for registration of call forwarding in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the RegisterSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of call forwarding.

4)	To check that upon receipt of the result of the operation (in a RELEASE COMPLETE or a FACILITY message), the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for :

a)	CFNRy, for basic service group speech the result of the operation being sent in a FACILITY message.

b)	CFU, for basic service group all facsimile, the result of the operation being sent in a RELEASE COMPLETE message.

31.2.1.1.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of CFNRy for Speech, to a number arbitrarily selected and with a no reply time value arbitrarily selected.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing the return result of the RegisterSS operation.

The SS transaction is released and the dedicated channel is released.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests registration of CFU for all facsimile, to a number arbitrarily selected.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the RegisterSS operation.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of call forwarding service 
				for CFNRy (Speech)
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed 
				with an SDCCH" 
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation" 
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator : 
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	CNFRySS operation Return_result
	9	SS -> MS	RELEASE COMPLETE
	10	SS -> MS	CHANNEL RELEASE

11	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CFU (all facsimile)
	12	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed 
				with an SDCCH" 
	13	SS -> MS	IMMEDIATE ASSIGNMENT
	14	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation" 
	15	SS -> MS	CM SERVICE ACCEPT

A16	MS -> SS	REGISTER

B16	MS -> SS	REGISTER 	protocol discriminator : 
				"non call related SS message",
				containing no SSoperation (invoke)
	B17	MS -> SS	FACILITY

18	SS -> MS	RELEASE COMPLETE	RegisterSS operation Return result
	19	MS		provide correct MMI user indication
	20	SS -> MS	CHANNEL RELEASE

Specific message content

step A6 or B7 - CFNRy

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Registration

Supplementary service code = CFNRy

Forwarded to number : as selected,

No reply condition time : as selected,

Basic service code : TeleService (AllSpeechTransmissionServices), no Bearerservice present.

step 8 - CFU

-	protocol discriminator : non call related SS message

-	transaction identifier : same as at step A6 or B6

-	message type : FACILITY

-	facility

Return Result = Registration

Supplementary service code = CFNRy

Forwarded to number

No Reply condition time

Basic service code : TeleService (AllSpeechTransmissionServices), no Bearerservice present.

step A16 or B17 - CFU

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Registration

Supplementary service code = CFU

Forwarded to number : as selected,

Basic service code : no bearer service present, teleservice (all facsimile).

step 18 - CFU

-	protocol discriminator : non call related SS message

-	transaction identifier : same as at step A16 or B16

-	message type : RELEASE COMPLETE

-	facility

Return Result = Registration

Supplementary service code = CFU

Forwarded to number

Basic service code : TeleService (all facsimile), no Bearerservice present.

##### 31.2.1.1.2	Registration rejected

31.2.1.1.2.1	Conformance requirements

1)	A transaction of any kind being already established, for registration of any type of call forwarding with any parameters, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the RegisterSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.2.1.1.2.2	Test purpose

1)	To check that the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of registration of call forwarding, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the RegisterSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of call forwarding.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

These checks are performed with a call transaction already established for :

a)	CFB, for all asynchronous services, the RELEASE COMPLETE message being sent with a facility IE containing a return_error(error) where error is "Bearer Service not provisioned".

b)	CF, for all facsimile, the RELEASE COMPLETE message being sent with a facility IE containing a reject(invoke_problem) where invoke_problem is "resource limitation".

31.2.1.1.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI),  the user requests registration of CFB for all asynchronous services, to a number arbitrarily selected.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing a Return_error(error : BearerService not provisioned) of the RegisterSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests registration of CF for all facsimile, to a number arbitrarily selected.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing a reject(invoke_problem : resource limitation) of the RegisterSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CFB (all asynchronous services)
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation" 
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	RegisterSS operation Return_error
	7	SS -> MS	STATUS ENQUIRY
	8	MS -> SS	STATUS	CC state U10

9	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CF (all facsimile)
	10	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	11	SS -> MS	CM SERVICE ACCEPT

A12	MS -> SS	REGISTER
	B12	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B13	MS -> SS	FACILITY

14	SS -> MS	RELEASE COMPLETE 	RegisterSS operation Reject
	15	MS		provide correct MMI user indication
	16	SS -> MS	STATUS ENQUIRY
	17	MS -> SS	STATUS	CC state U10

Specific message content

step A4 or B5 - CFB

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = Registration

Supplementary service code = CFB

Forwarded to number : as selected,

Basic service code : Bearer Service (all asynchronous services), no teleservice present

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : BearerService not provisioned

For the return error the invoke ID must be the same as in the invoke of the RegisterSS operation.

step A12 or B13 - CF

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B12

-	message type : REGISTER (A12), FACILITY (B13)

-	facility

invoke = Registration

Supplementary service code = CF

Forwarded to number : as selected,

Basic service code : no bearer service present, teleservice (all facsimile)).

step 14 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : resource limitation

For the reject the invoke ID must be the same as in the invoke of the Register SS operation.

#### 31.2.1.2	Erasure by the subscriber

##### 31.2.1.2.1	Erasure accepted

31.2.1.2.1.1	Conformance requirements

For erasure of any type of call forwarding, the MS shall transmit successively

1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

3)	and then the REGISTER message containing a facility that includes an invoke of the EraseSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values according to the user request (MMI action).

4)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.2.1.2.1.2	Test purpose

1)	To check that the MS correctly requests supplementary service transaction for erasure of call forwarding in CHANNEL REQUEST message.

2)	To check that the MS correctly requests supplementary service transaction for erasure of call forwarding in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the EraseSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for erasure of call forwarding.

4)	To check that upon receipt of the result of the operation (in a RELEASE COMPLETE or a FACILITY message), the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for :

a)	CFC, for basic service group all facsimile, the result of the operation being sent in a FACILITY message.

b)	CFNRc, for all basic service groups, the result of the operation being sent in a RELEASE COMPLETE message.

31.2.1.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests erasure of CFC for all facsimile.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with a FACILITY message with the facility information element containing a return result of the EraseSS operation.

The SS transaction is released and the dedicated channel is released.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests erasure of CFNRc for all basic service groups.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the EraseSS operation.

The dedicated channel is released.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				erasure of call forwarding
				for CFC (all facsimile)
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	EraseSS operation Return_result
	9	SS -> MS	RELEASE COMPLETE
	10	SS -> MS	CHANNEL RELEASE

11	MS 		The MS is made to initiate
 				a erasure of call forwarding service
				for CFNRc (all basic services)
	12	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	13	SS -> MS	IMMEDIATE ASSIGNMENT
	14	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	15	SS -> MS	CM SERVICE ACCEPT

A16	MS -> SS	REGISTER

B16	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B17	MS -> SS	FACILITY

18	SS -> MS	RELEASE COMPLETE	EraseSS operation Return result
	19	MS		provide correct MMI user indication
	20	SS -> MS	CHANNEL RELEASE

Specific message content

step A6 or B7 - CFC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Erasure

Supplementary service code = CFC

Basic service code : no Bearer Service, teleservice (all facsimile).

step A16 or B17 - CFNRc

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Erasure

Supplementary service code = CFNRc

Basic service code : no bearer service present, no teleservice present.

##### 31.2.1.2.2	Erasure rejected

31.2.1.2.2.1	Conformance requirements

1)	A transaction of any kind being already established, for erasure of any type of call forwarding with any parameters, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the EraseSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.2.1.2.2.2	Test purpose

1)	To check that the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of erasure of call forwarding, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the EraseSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for erasure of call forwarding.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

These checks are performed with a call transaction already established for :

a)	CFU, for Speech, the RELEASE COMPLETE message being sent with a facility IE containing a return_error(error) where error is "Teleservice not provisioned".

b)	CFNRy, for all facsimile, the RELEASE COMPLETE message being sent with a facility IE containing a reject(invoke_problem) where invoke_problem is "resource limitation".

31.2.1.2.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests erasure of CFU for Speech.

Upon receipt of the operation (in a REGISTER or a FACILITY message),the system simulator answers with the FACILITY message with the Facility information element containing a Return_error(error : TeleService not provisioned) of the EraseSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests erasure of CFNRy for all facsimile.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing a reject(invoke_problem : resource limitation) of the EraseSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a erasure of call forwarding service
				for CFU (speech)
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT
	A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	EraseSS operation Return_error
	7	SS -> MS	STATUS ENQUIRY
	8	MS -> SS	STATUS	CC state U10

9	MS 		The MS is made to initiate
 				a erasure of call forwarding service
				for CFNRy (all facsimile)
	10	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	11	SS -> MS	CM SERVICE ACCEPT

A12	MS -> SS	REGISTER
	B12	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B13	MS -> SS	FACILITY

14	SS -> MS	RELEASE COMPLETE 	EraseSS operation Reject
	15	MS		provide correct MMI user indication
	16	SS -> MS	STATUS ENQUIRY
	17	MS -> SS	STATUS	CC state U10

Specific message content

step A4 or B5 - CFU

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = Erasure

Supplementary service code = CFU

Basic service code : Tele Service (AllSpeechTransmissionServices), no Bearerservice present

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : TeleService not provisioned

For the return error the invoke ID must be the same as in the invoke of the EraseSS operation.

step A12 or B13 - CFNRy

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B12

-	message type : REGISTER (A12), FACILITY (B13)

-	facility

invoke = Erasure

Supplementary service code = CFNRy

Basic service code : no bearer service present, teleservice (all facsimile)).

step 14 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : resource limitation

For the reject the invoke ID must be the same as in the invoke of the EraseSS operation.

#### 31.2.1.3	Activation

31.2.1.3.1	Conformance requirements

For activation of any type of call forwarding with any parameters, the MS shall transmit successively

1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

3)	and then the REGISTER message containing a facility IE that includes an invoke of the ActivateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

4) Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.2.1.3.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for activation of call forwarding in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for activation of call forwarding in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for activation of call forwarding.

4)	To check that upon receipt of the result of the operation (in a RELEASE COMPLETE or a FACILITY message), the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for :

a)	CF, for basic service group "all synchronous services" the result of the operation being sent in a FACILITY message.

b)	CFU, for all basic service groups, the result of the operation being sent in a RELEASE COMPLETE message.

31.2.1.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests activation of CF for all synchronous services.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing the return result of the ActivateSS operation.

The SS transaction is released and the dedicated channel is released.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests activation of CFU for all basic service groups.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the ActivateSS operation.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a activation of call forwarding service
				for CF (all synchronous services)
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	ActivateSS operation Return_result
	9	SS -> MS	RELEASE COMPLETE
	10	SS -> MS	CHANNEL RELEASE

11	MS 		The MS is made to initiate
 				a activation of call forwarding service
				for CFU (all basic service groups)
	12	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	13	SS -> MS	IMMEDIATE ASSIGNMENT
	14	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	15	SS -> MS	CM SERVICE ACCEPT

A16	MS -> SS	REGISTER

B16	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B17	MS -> SS	FACILITY

18	SS -> MS	RELEASE COMPLETE	ActivateSS operation Return result
	19	MS		provide correct MMI user indication
	20	SS -> MS	CHANNEL RELEASE

Specific message contents :

step A6 or B7 - CF

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Activation

Supplementary service code = CF

Basic service code : Bearer Service (all synchronous services), no teleservice present.

step A16 or B17 - CFU

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Activation

Supplementary service code = CFU

Basic service code : no bearer service present, no teleservice present.

#### 31.2.1.4	Deactivation

31.2.1.4.1	Conformance requirements

For deactivation of any type of call forwarding with any parameters, the MS shall transmit successively

1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

3)	and then the REGISTER message containing a facility IE that includes an invoke of the DeactivateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

4) Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.2.1.4.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for deactivation of call forwarding in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for deactivation of call forwarding in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for deactivation of call forwarding.

4)	To check that upon receipt of the result of the operation (in a RELEASE COMPLETE or a FACILITY message), the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for :

a)	CFC, for basic service group speech the result of the operation being sent in a FACILITY message.

b)	CFNRc, for basic service group all facsimile, the result of the operation being sent in a RELEASE COMPLETE message.

31.2.1.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests deactivation of CFC for speech.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing the return result of the DeactivateSS operation.

The SS transaction is released and the dedicated channel is released.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests deactivation of CFNRc for all facsimile.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the DeactivateSS operation.

The dedicated channel is released.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a deactivation of call forwarding service
				for CFC (speech)
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	DeactivateSS operation Return_result
	9	SS -> MS	RELEASE COMPLETE
	10	SS -> MS	CHANNEL RELEASE

11	MS 		The MS is made to initiate
 				a deactivation of call forwarding service
				for CFNRc (all facsimile)
	12	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	13	SS -> MS	IMMEDIATE ASSIGNMENT
	14	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	15	SS -> MS	CM SERVICE ACCEPT

A16	MS -> SS	REGISTER

B16	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B17	MS -> SS	FACILITY

18	SS -> MS	RELEASE COMPLETE	DeactivateSS operation Return result
	19	MS		provide correct MMI user indication
	20	SS -> MS	CHANNEL RELEASE

Specific message content

step A6 or B7 - CFC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Deactivation

Supplementary service code = CFC

Basic service code : Bearer Service (speech), no teleservice present.

step A16 or B17 - CFNRc

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Deactivation

Supplementary service code = CFNRc

Basic service code : no bearer service present, teleservice (all facsimile).

#### 31.2.1.5	Invocation

Invocation is not applicable to the MS and causes no signalling on the radio path.

#### 31.2.1.6	Interrogation

##### 31.2.1.6.1	Interrogation accepted

31.2.1.6.1.1	Conformance requirements

For interrogation of any specific call forwarding service (not applicable to a group of services) with any parameters, the MS shall transmit successively

1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

3)	and then the REGISTER message containing a facility IE that includes an invoke of the InterrogateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

4) Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.2.1.6.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for interrogation of a specific call forwarding in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for interrogation of call forwarding in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for interrogation of call forwarding.

4)	To check that upon receipt of the result of the operation (in a RELEASE COMPLETE or a FACILITY message), the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for :

a)	CFB, for all basic service groups, the result of the operation being sent in a FACILITY message.

b)	CFNRy, for basic service group speech, the result of the operation being sent in a RELEASE COMPLETE message.

31.2.1.6.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests interrogation of CFB for all basic service groups.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing the return result of the InterrogateSS operation.

The SS transaction is released and the dedicated channel is released.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests interrogation of CFNRy for speech.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the InterrogateSS operation.

The dedicated channel is released.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CFB (all)
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	InterrogateSS operation Return_result
	9	SS -> MS	RELEASE COMPLETE
	10	SS -> MS	CHANNEL RELEASE

11	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CFNRy(speech)
	12	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	13	SS -> MS	IMMEDIATE ASSIGNMENT
	14	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	15	SS -> MS	CM SERVICE ACCEPT

A16	MS -> SS	REGISTER

B16	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B17	MS -> SS	FACILITY

18	SS -> MS	RELEASE COMPLETE	InterrogateSS operation Return result
	19	MS		provide correct MMI user indication
	20	SS -> MS	CHANNEL RELEASE

Specific message content

step A6 or B7 - CFB

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Interrogation

Supplementary service code = CFB

Basic service code : no Bearer Service present, no teleservice present.

step A16 or B17 - CFNRy

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Interrogation

Supplementary service code = CFNRy

Basic service code : no bearer service present, teleservice (speech).

##### 31.2.1.6.2	Interrogation rejected

31.2.1.6.2.1	Conformance requirements

1)	A transaction of any kind being already established, for interrogation of any specific call forwarding service with any parameters, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.82, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.2.1.6.2.2	Test purpose

1)	To check that the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of interrogation of a specific call forwarding service, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for interrogation of call forwarding.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

These checks are performed with a call transaction already established for :

a)	CFNRc, for all basic service group, the RELEASE COMPLETE message being sent with a facility IE containing a return_error(error) where error is "SS not available".

b)	CFB, for all facsimile, the RELEASE COMPLETE message being sent with a facility IE containing a reject(invoke_problem) where invoke_problem is "resource limitation".

31.2.1.6.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests interrogation of CFNRc for all basic service groups.

Upon receipt of the operation (in a REGISTER or a FACILITY message),the system simulator answers with the FACILITY message with the Facility information element containing a Return_error(error : SS not available) of the InterrogateSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Then again, by means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30, the user requests interrogation of CFB for all facsimile.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message (same PD and TI that in the REGISTER message) with the Facility information element containing a reject(invoke_problem : resource limitation) of the InterrogateSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of call forwarding service
				for CFNRc (all)
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS ->SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	InterrogateSS operation Return_error
	7	SS -> MS	STATUS ENQUIRY
	8	MS -> SS	STATUS	CC state U10
	9	MS 		The MS is made to initiate
 				a interrogation of call forwarding service
				for CFB (all facsimile)
	10	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	11	SS -> MS	CM SERVICE ACCEPT

A12	MS -> SS	REGISTER
	B12	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B13	MS ->SS	FACILITY

14	SS -> MS	RELEASE COMPLETE 	InterrogateSS operation Reject
	15	MS		provide correct MMI user indication
	16	SS -> MS	STATUS ENQUIRY
	17	MS -> SS	STATUS	CC state U10

Specific message content

step A4 or B5 - CFNRc

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = Interrogation

Supplementary service code = CFNRc

Basic service code :no Bearer Service present, no teleservice present

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : SS not available

For the return error the invoke ID must be the same as in the invoke of the InterrogateSS operation.

step A12 or B13 - CFB

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B12

-	message type : REGISTER (A12), FACILITY (B13)

-	facility

invoke = Interrogation

Supplementary service code = CFB

Basic service code : no bearer service present, teleservice (all facsimile)).

step 14 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : resource limitation

For the reject the invoke ID must be the same as in the invoke of the InterrogateSS operation.

#### 31.2.1.7	Normal operation

##### 31.2.1.7.1	Served mobile subscriber side

###### 31.2.1.7.1.1	Notification during an incoming call

This subscription option is only applicable to CFB and CFNRy.

31.2.1.7.1.1.1	Conformance requirements

1)	During a call transaction, call establishment or not, upon receipt of a FACILITY message notifying an incoming call, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

2)	If a call transaction is being established or is already established when the notification of incoming call is received, the receipt of the notification has no effect on its state.

References

1)	GSM 02.30 section 4.5

2)	GSM 04.82 section 1

31.2.1.7.1.1.2	Test purpose

1)	To check that, in state U7 or U10, upon receipt of a FACILITY message notifying an incoming call, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

2)	To check that when the notification of incoming call is received while the MS is in CC state U7 and U10 of another incoming call, it has no effect on its state.

These checks are performed in the case of CFB.

31.2.1.7.1.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

The MS is brought to the call state U7 of an incoming call.

The system simulator transmits a FACILITY message with the facility information element containing an invoke of the NotifySS operation for CFB.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U7.

The MS is brought into the call state U10 of an incoming call.

The system simulator transmits a FACILITY message with the facility information element containing an invoke of the NotifySS operation for CFB.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

The transaction and the channel are released by the SS.

Maximum duration of test

1 mn

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	SETUP
	6	MS -> SS	CALL CONFIRMED
	7	SS -> MS	ASS COMMAND
	8	MS -> SS	ASS COMPLETE
	9	MS -> SS	ALERTING
	10	SS -> MS	FACILITY	(invoke NotifySS)
	11	SS -> MS 	STATUS ENQUIRY
	12	MS -> SS	STATUS	(U7)
	13	MS -> SS	CONNECT	MS off hook
	14	SS -> MS	CONNECT ACK
	15 	SS -> MS	FACILITY	(invoke NotifySS)
	16	SS -> MS	STATUS ENQUIRY
	17	MS -> SS	STATUS	(U10)
	18	SS -> MS 	RELEASE COMPLETE
	19	SS -> MS	CHANNEL RELEASE

Specific message contents

Steps 10 and 15

-	protocol discriminator : CC

-	transaction identifier : same as for the call transaction already established

-	message type : FACILITY

-	facility

invoke = notification

Supplementary service code = CFB

SS notification = incoming call is forwarded

(call is forwarded indication to B subscriber).

###### 31.2.1.7.1.2	Notification during an outgoing call

31.2.1.7.1.2.1	Conformance requirements

1)	As an outgoing call is being established, if the ALERTING message is received with the facility information element containing an SS notification (for CFU or CFC), the MS shall correctly reach CC state U4.

2)	As an outgoing call is being established, if the ALERTING message is received with the facility information element containing an SS notification (for CFU or CFC), the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

3)	As an outgoing call is being established, if the CONNECT message is received with the facility information element containing an SS notification (for CFU or CFC), the MS shall normally send a CONNECT ACK message and enter CC state U10.

4)	As an outgoing call is being established, if the CONNECT message is received with the facility information element containing an SS notification (for CFU or CFC), the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.82, TS GSM 04.80 section 3.6

2)	GSM 04.82, TBS. GSM 04.80 section 3.6

31.2.1.7.1.2.2	Test purpose

1)	To check that when an outgoing call is being established, if the ALERTING message is received with the facility information element containing an SS notification, the MS correctly reaches CC state U4. This is tested for CFU.

2)	As an outgoing call is being established, if the ALERTING message is received with the facility information element containing an SS notification, the MS provides the appropriate user indication (which is to be described by the manufacturer). This is tested for CFU.

3)	As an outgoing call is being established, if the CONNECT message is received with the facility information element containing an SS notification, the MS normally sends a CONNECT ACK message and enter CC state U10. This is tested for CFC.

4)	As an outgoing call is being established, if the CONNECT message is received with the facility information element containing an SS notification (for CFU or CFC), the MS provides the appropriate user indication (which is to be described by the manufacturer). This is tested for CFC.

31.2.1.7.1.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

The MS is made to place an outgoing call.

After having received a SETUP message and sent a CALL PROCEEDING message and after a TCH has been allocated the system simulator transmits an ALERTING message with the facility information element containing a notification

The system simulator send then a STATUS ENQUIRY message. On receipt of the STATUS ENQUIRY message the MS shall send a STATUS message with CC-state U4.

After that, the system simulator transmits a CONNECT with the facility information element containing a notification

After reception of a CONNECT ACKNOWLEDGE message the system simulator sends a STATUS ENQUIRY message.

The MS shall respond with a STATUS message indicating CC-state U10.

The transaction and the channel are released by the SS.

Maximum duration of test

1 mn

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST
	2	SS -> MS	IMMEDIATE ASSIGNMENT
	3	MS -> SS	CM SERVICE REQUEST	"mobile originating call establishment"
	4	SS -> MS	CM SERVICE ACCEPT
	5	MS -> SS	SETUP
	6	SS -> MS	CALL PROCEEDING
	7	SS -> MS	ASS COMMAND
	8	MS -> SS	ASS COMPLETE
	9	SS -> MS	ALERTING	containing facility IE
	10	SS -> MS 	STATUS ENQUIRY
	11	MS -> SS	STATUS	(U4)
	12	SS -> MS	CONNECT	containing facility IE
	13	MS -> SS	CONNECT ACK
	14	SS -> MS 	STATUS ENQUIRY
	15	MS -> SS	STATUS	(U10)
	16	SS -> MS 	RELEASE COMPLETE
	17	SS -> MS	CHANNEL RELEASE

Specific message contents

At step 9 -

Facility invoke = notification

-	SS-Code (CFU)

-	SS-Status (indicating:

Provisioned, registered and active).

At step 12 -

Facility invoke = notification

-	SS-Code (CFC)

-	SS-Status (indicating:

Provisioned, registered and active).

##### 31.2.1.7.2	Forwarded-to mobile subscriber side

31.2.1.7.2.1	Conformance requirements

1)	Upon receipt of the SETUP message containing a notification indication that the call is a forwarded one (with any SS code except CFC), the MS shall correctly continue call establishment and enter CC state U6.

2)	Upon receipt of the SETUP message containing a notification indication that the call is a forwarded one, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.82

2)	GSM O2.30 section 4.5

31.2.1.7.2.2	Test purpose

1)	To check that, upon receipt of the SETUP message containing a notification indication that the call is a forwarded one, the MS correctly continues call establishment and enters CC state U6.

2)	Upon receipt of the SETUP message containing a notification indication that the call is a forwarded one, the MS provides the appropriate user indication (which is to be described by the manufacturer).

31.2.1.7.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call forwarding

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

An incoming call is given to the MS with the SETUP message with the facility information element containing an invoke of the NotifySS operation with the indication that the call is forwarded.

Then the network sends a STATUS ENQUIRY message : the MS responds indicating CC state U6.

The transaction and the channel are released by the SS.

Maximum duration of test

1 mn

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	SETUP	containing the notification that
				the call is a forwarded one
	6	SS -> MS 	STATUS ENQUIRY
	7	MS -> SS	STATUS	(U6)
	8	SS -> MS 	RELEASE COMPLETE
	9	SS -> MS	CHANNEL RELEASE

Specific message contents

at step 6 -

-	protocol discriminator : CC

-	transaction identifier :

-	message type : SETUP

-	facility

invoke = notification

-	SS-Code (CFU, CFB, CFNRy, CFNRc or CF)

-	SS-Notification (indicating: call is forwarded i.e.

Call is forwarded indication to C-subscriber).

### 31.2.2	Call transfer and mobile access hunting supplementary services

(Reserved).

## 31.3	Call completion supplementary services

(Reserved).

## 31.4	Multi-party supplementary services

(Reserved).

## 31.5	Community of interest supplementary services

(Reserved).

## 31.6	Charging supplementary services

The following Advice of Charge abbreviations are used in this section:

AOC		Advice of Charge

AOCC	Advice of Charge Charging

AOCI		Advice of Charge Information

ACM		Accumulated Charge Meter

ACMmax	Accumulated Charge Meter Maximum

CCM		Current Call Meter

CAI		Charge Advice Information

CDUR	Chargeable Duration

The following other abbreviations are used:

IE 		Information Element

FIE		Facility Information Element

TCH		Traffic CHannel

General on Advice of Charge.

The purpose of these tests is to verify that the MS under test correctly performs procedures related to the AOC supplementary service.

The reasons for these test purposes are:

-	One example of a possible use for AOCC is in applications where the subscriber hires out a GSM ME and SIM to a user and bills the user according to the charge stored on the SIM at the end of the hire period. If a mobile station claims to support AOCC but does so incorrectly or not at all, this may cause the subscriber to mischarge the user of the hire phone. Hence an MS claiming to support AOCC must be shown to be reliable in that context.

-	Since AOCC offers the use of telecommunication services according to the charge stored independently in the MS, the AOCC service must not be susceptible to fraud at the MS end.

-	To ensure that a mobile station not claiming to support AOCC does not respond with a signal to the network indicating that it does. This could cause the network to allow the call to be placed without any charge being inserted on to the SIM.

Mobile originating and terminating speech AOC calls are tested and if supported, Call Hold and Multi-party calls. The type testing of data calls (i.e. those calls with a volume related charging component) will not be specified at the phase 2 stage but will be deferred to phase 2+ since the interaction of AOC with packet services is not yet defined in GSM 09.06 and GSM 09.07.

Tests are made on the ME-SIM interface to ensure that call charges are being correctly stored in the ACM field of the SIM in several situations. The AOC ACMmax function is also tested for incoming and outgoing calls.

Basic service verification tests for the AOCC, Call Hold, and Multi-party supplementary services which have direct relevance to the testing of AOC have been added to GSM 11.10, sections 11.3, 11.4 and 11.5 respectively.

### 31.6.1	Advice of Charge Charging

#### 31.6.1.1	AOC time related charging / MS originated call

Purpose:

1)	To verify that when the MS receives the AOCC parameters in a Facility IE which is contained in the CONNECT message and when a TCH has already been assigned, the MS returns a FACILITY message containing the acknowledgement within 1 second.

2)	To verify that when the MS receives the AOCC parameters in a Facility IE which is contained in a CONNECT message and when a TCH has already been assigned, the MS stores the correct value in the ACM field of the SIM.

3)	To verify that the when the call has no volume related component the MS ignores non-zero AOCC e5, e6 parameters sent to it.

Conformance Requirement(s):

1)	When the MS receives the AOCC parameters in a Facility IE which is contained in a CONNECT message and when a TCH has already been assigned, the MS shall return a FACILITY message containing the acknowledgement within 1 second.

2)	When the MS receives the AOCC parameters in a Facility IE which is contained in a CONNECT or FACILITY message and when a TCH has already been assigned, the MS shall store the correct value in the ACM field of the SIM.

3)	When the call has no volume related component the MS shall ignore non-zero AOC e5 and e6 parameters sent to it.

Reference(s):

Conformance requirement 1:	TS GSM 03.86, 04.13, 04.86.

Conformance requirement 2:	TS GSM 02.24

Conformance requirement 3:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent by the MS before or after the CONNECT ACKNOWLEDGE.

The SS sends the DISCONNECT y seconds after sending the CONNECT message containing the CAI. The MS shall have stored the correct amount on the SIM according to the e-parameters sent. The test is repeated for several different sets of e-parameters as defined below.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,...,5.

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE
	A13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	B13	MS -> SS	CONNECT ACKNOWLEDGE

14			call duration y seconds after CAI information 
				sent by SS
	15	SS -> MS	DISCONNECT
	16	MS -> SS	RELEASE
	17	SS -> MS	RELEASE COMPLETE
	18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	19	MS		SIM contents checked (either via MMI or by 
				removing the SIM and using SIM reader). ACM 
				shall have been incremented correctly.

NOTE 1:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

y is set to a constant value of 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as below:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 
						increased by

1	2	3	4	5	6	7

1	6	14	1	25	0	0	60	43	43
		2	0	0	1	100	0	0	0	100	143
		3	250	16	2	500	0	0	60	2000	2143
		4	1	1	1	0	10	10	1	89 or 90	2232 or 2233
		5	12.5	30	1	25	10	10	30	50 or 62.5	2295,
											2296,
											2282 or 2283

Values shown in table are in the format and have units as in GSM 02.24 section 3.

The ASN.1 description for each e-parameter allows integers in the range 0 to 8191 to be transmitted but some e-parameters have different actual ranges (e.g. e1 can take any value 0..819.1 with 0.1 resolution). The MS knows how to interpret the received parameter (e.g. received e1 refers to 10 times actual e1, see GSM 04.80 section 4.4.3). Therefore e1=12.5 would be sent to the MS as 125. The MS knows the value sent is 10 times the 'real' e1 and hence interprets the value as 12.5.

The non-zero e5 and e6 values for the k=4 and k=5 execution of the test are to check that the MS ignores the volume related parameters when carrying out time only related charging.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.1.2	AOC time related charging / MS terminated call

Purpose:

1)	To verify that when the MS receives certain AOCC e-parameters in a Facility IE which is contained in a FACILITY message sent after the CONNECT message and when a TCH has already been assigned, the MS returns a FACILITY message containing the acknowledgement within 1 second.

2)	To verify that when the MS receives the AOCC parameters in a Facility IE which is contained in a FACILITY message and when a TCH has already been assigned, the MS stores the correct value in the ACM field of the SIM.

Conformance Requirement(s):

1)	When the MS receives the AOCC parameters in a Facility IE which is contained in a FACILITY message sent after the CONNECT message and when a TCH has already been assigned, the MS shall return a Facility message containing the acknowledgement within 1 second.

2)	When the MS receives the AOCC parameters in a Facility IE which is contained in a FACILITY message and when a TCH has already been assigned, the MS shall store the correct value in the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 03.86, 04.13, 04.86.

Conformance requirement 2:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The SS is made to initiate a call. The call is established and certain AOCC e-parameters are sent to the MS in a Facility IE contained within a FACILITY message. The MS shall return the AOCC acknowledgement within 1 second of the FACILITY message. It is an implementation option whether the AOCC e-parameters and the AOCC acknowledge are sent before or after the CONNECT ACKNOWLEDGE.

The SS sends the DISCONNECT y seconds after sending the FACILITY message. The MS shall have stored the correct amount on the SIM according to the e-parameters sent. The test is repeated for several different sets of e-parameters as defined below.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,...,5.

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM
	1	SS 		The SS is made to initiate a call
	2	SS -> MS	PAGING REQUEST
	3	MS -> SS	CHANNEL REQUEST
	4	SS -> MS	IMMEDIATE ASSIGNMENT
	5	MS -> SS	PAGING RESPONSE
	6	SS -> MS	SETUP
	7	MS -> SS	CALL CONFIRMED
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	MS -> SS	ALERTING
	11	MS -> SS	CONNECT
	12	SS -> MS	FACILITY	As default message except contains Facility IE 					with contents as indicated in i below

Either A or B branch is taken

A13	SS -> MS	CONNECT ACKNOWLEDGE
	A14	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

B13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	B14	SS -> MS	CONNECT ACKNOWLEDGE

15			call duration y seconds after CAI information 
				sent by SS
	16	SS -> MS	DISCONNECT
	17	MS -> SS	RELEASE
	18	SS -> MS	RELEASE COMPLETE
	19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	20	MS		SIM contents checked (either via MMI or by 
				removing the SIM and using SIM reader) ACM 
				shall have been incremented correctly.

NOTE 2:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

y is set to a constant value of 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as below:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

1	0	0	0	0	0	0	0	0	0
		2	0	0	1	100	0	0	0	100	100
		3	6	14	1	25	0	0	60	43	143
		4	1	1	1	0	0	0	1	89 or 90	233 or 232
		5	12.5	30	1	25	0	0	30	50 or 62.5	296,
											295,
											282 or 283

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.1.3	AOC volume related charging / MS originated call

Future addition at GSM Phase 2+ stage.

#### 31.6.1.4	AOC volume related charging / MS terminated call

Future addition at GSM Phase 2+ stage.

#### 31.6.1.5	Change in charging information during a call

Purpose:

1)	To verify that when the MS receives new AOCC parameters mid-way through a call in a Facility IE which is contained within a FACILITY message the MS returns a FACILITY message containing the acknowledgement within 1 second.

2)	To verify that when the MS receives new charging information mid-way through a call in the form of a Facility IE contained within a FACILITY message the MS correctly indicates the total charge considering both sets of charging information

Conformance Requirement(s):

1)	When the MS receives new AOCC parameters mid-way through a call in a Facility IE which is contained within a FACILITY message the MS shall return a FACILITY message containing the acknowledgement within 1 second.

2)	When the MS receives new charging information mid-way through a call in the form of a Facility IE contained within a FACILITY message the MS correctly indicates the total charge considering both sets of charging information

Reference(s):

Conformance requirement 1:	TS GSM 03.86, 04.13, 04.86.

Conformance requirement 2:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in a FACILITY message sent after the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of the FACILITY message. x seconds after sending the original CAI, new (and different) e-parameters are sent to the MS in a Facility IE contained within a FACILITY message. The MS shall return the AOCC acknowledge within 1 second of the FACILITY message.

The SS sends the DISCONNECT y seconds after sending the first CAI in the FACILITY message. The MS shall have stored the correct amount on the SIM according to the two sets of e-parameters sent and the times for the two parts of the call

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT

Either A, B or C branch is taken

A12	SS -> MS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in i below
	A13	MS -> SS	CONNECT ACKNOWLEDGE
	A14	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

B12	MS -> SS	CONNECT ACKNOWLEDGE
	B13	SS -> MS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in i below
	B14	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

C12	SS -> MS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in i below
	C13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	C14	SS -> MS	CONNECT ACKNOWLEDGE

Second CAI sent x sec after first CAI
	15	SS -> MS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in iii below
	16	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
				New CAI held in abeyance until CDUR has 
				timed out present e2 value
	17			Call duration y seconds after first CAI 
				information sent by SS
	18	SS -> MS	DISCONNECT
	19	MS -> SS	RELEASE
	20	SS -> MS	RELEASE COMPLETE
	21	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	22	MS		SIM contents checked (either via MMI or by 
				removing the SIM and using SIM reader) ACM 
				shall have been incremented correctly

NOTE 3:

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x, y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set as defined below:

e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

10	28	1	10	0	0	60	(30)	(30)

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

iii) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x is set to a constant value of 80 seconds.

y is set to a constant value of 180 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

e-parameter		CCM total	Step 0 ACM
					at call end	value 
						increased by

1	2	3	4	5	6	7

10	14	1	5	0	0	60	65	65

Values shown in table are in the format and have units as in GSM 02.24 section 3.

#### 31.6.1.6	Different formats of charging information

Purpose:

1)	To verify that when the MS receives a Facility IE in which certain e-parameters are set to zero the total charge accumulated is the same as that when the same e-parameters are completely omitted from the Facility IE.

2)	To verify the operation of a shortened channel release procedure where the SS does not send DISCONNECT but only the RELEASE COMPLETE and CHANNEL RELEASE messages or just the CHANNEL RELEASE message.

Conformance Requirement(s):

1)	When the MS receives a Facility IE in which certain e-parameters are set to zero the total charge accumulated shall be the same as that when the same e-parameters are completely omitted from the Facility IE.

2)	The channel shall be correctly released when a shortened channel release procedure is used - the SS does not send DISCONNECT but only the RELEASE COMPLETE and CHANNEL RELEASE messages or just the CHANNEL RELEASE message.

Reference(s):

Conformance requirement 1:	TS GSM 02.24

Conformance requirement 2:	TS GSM 04.08

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

Part 1:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in a FACILITY message sent before the CONNECT message.

The SS sends the DISCONNECT y seconds after sending the FACILITY message containing the e-parameters. The MS shall have stored the correct amount on the SIM according to the e-parameters sent.

Part 2:

Part 1 is repeated twice with the e-parameters that were set to zero above now omitted completely from the Facility IE. The shortened release procedures are used. The MS shall have stored the correct amount on the SIM.

The results of parts 1 and 2 are compared. The value for the charge calculated by the MS shall be identical for parts 1 and 2.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,..3

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in i below

Either A, B or C branch is taken

A12	SS -> MS	CONNECT
	A13	MS -> SS	CONNECT ACKNOWLEDGE
	A14	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

B12	SS -> MS	CONNECT
	B13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	B14	MS -> SS	CONNECT ACKNOWLEDGE

C12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	C13	SS -> MS	CONNECT
	C14	MS -> SS	CONNECT ACKNOWLEDGE

15			call duration y seconds after CAI information 
				sent by SS

Branch D, E and F shall be taken for k= 1,2 
				and 3 respectively

D16	SS -> MS	DISCONNECT
	D17	MS -> SS	RELEASE
	D18	SS -> MS	RELEASE COMPLETE
	D19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

E16	SS -> MS	RELEASE COMPLETE	Shortened channel release procedure
	E17	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

F16	SS -> MS	CHANNEL RELEASE

20	MS		SIM contents checked (either via MMI or by 					removing the SIM and using SIM reader) ACM 					shall have been incremented correctly.

NOTE 4:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

y is set to a constant value of 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value		e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

1	10	40	1	0	0	0	0	20		20

2	10	40	1	---------omitted---------	20		40

3	10	40	1	---------omitted---------	20		60

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.1.7	AOC on a Call Hold call

Purpose:

1)	To verify that when the MS invokes a Call Hold call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS returns a FACILITY message containing the AOCC acknowledgement within 1 second of transmission of each set of e-parameters.

2)	To verify that when the MS invokes a Call Hold call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM records the sum of all the charges for the services currently being used and hence that the ME inserts the correct charge in the ACM field of the SIM.

Conformance Requirement(s):

1)	When the MS invokes a Call Hold call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS shall return a FACILITY message containing the AOCC acknowledgement within 1 second of receiving each set of e-parameters.

2)	When the MS invokes a Call Hold call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM shall record the sum of all the charges for the services currently being used and hence the ME shall insert the correct charge in the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 04.13

Conformance requirement 2:	TS GSM 02.24, 04.83, 04.84, 04.86.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of transmission of the CONNECT message.

The call (call B) is then put on hold by sending a HOLD message from the MS to the SS. The SS shall reply with a HOLD ACKNOWLEDGE. The traffic channel is now available to originate another call.

The MS is made to initiate a second call (call C). The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement in a FACILITY message within 1 second of transmission of the CONNECT message.

The SS sends the DISCONNECT to the MS for call B x seconds after sending the call B CAI in the CONNECT/FACILITY message and the DISCONNECT for call C y seconds after sending the call C CAI in the CONNECT/FACILITY message. The MS shall have stored the correct amount on the SIM according to the two sets of e-parameters sent and call times x and y.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 					ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either D or E branch is taken

D12	MS -> SS	CONNECT ACKNOWLEDGE
	D13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

E12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	E13	MS -> SS	CONNECT ACKNOWLEDGE

14	MS -> SS	HOLD	Call B put on hold
	15	SS -> MS	HOLD ACKNOWLEDGE
	16	MS 		The MS is made to initiate a second call
	17	MS -> SS	SETUP	TI arbitrary but different from existing TI
	18	SS -> MS	CALL PROCEEDING
	19	SS -> MS	ALERTING
	20	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either F or G branch is taken

F21	MS -> SS	CONNECT ACKNOWLEDGE
	F22	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

G21	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	G22	MS -> SS	CONNECT ACKNOWLEDGE

23			Call durations x and y seconds after respective 
				CAI information sent by SS

Branch H and I branch are taken, the
				sequence depending on the durations x and y

For call C
	H24	SS -> MS	DISCONNECT	y seconds after call C CAI sent
	H25	MS -> SS	RELEASE
	H26	SS -> MS	RELEASE COMPLETE

For call B
	I24	SS -> MS	DISCONNECT	x seconds after call B CAI sent
	I25	MS -> SS	RELEASE
	I26	SS -> MS	RELEASE COMPLETE

27	SS -> MS	CHANNEL RELEASE	The main signalling link is released
	28	MS		SIM contents checked (either via MMI or by 
				removing the SIM and using SIM reader) ACM 
				shall have been incremented correctly.

NOTE 5:

-	The value of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x set to 180s, y is set to 90s.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

Call		e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

B(time x)	7	40	1	0	0	0	0

C(time y)	13	40	1	0	0	0	0	54	54

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.1.8	AOC on a Multi-party call

Purpose:

1)	To verify that when the MS invokes a Multi-party call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS returns a FACILITY message containing the AOCC acknowledgement within 1 second of transmission of each set of e-parameters.

2)	To verify that when the MS originates a Multi-party call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM records the sum of all the charges for the services currently being used and hence the ME inserts the correct charge in the ACM field of the SIM.

Conformance Requirement(s):

1)	When the MS invokes a Multi-party call and hence receives Facility IEs containing AOCC e-parameters for each chargeable call the MS shall return a FACILITY message containing the AOCC acknowledgement within 1 second of receiving each set of e-parameters.

2)	When the MS originates a Multi-party call and hence receives Facility IEs containing CAI elements for each chargeable call the CCM shall record the sum of all the charges for the services currently being used and hence the ME shall insert the correct charge in the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 04.13

Conformance requirement 2:	TS GSM 02.24, 04.83, 04.84, 04.86.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of transmission of the CONNECT message.

The call (call B) is then put on hold by sending a HOLD message from the MS to the SS. The SS shall reply with a HOLD ACKNOWLEDGE. The traffic channel is now available to originate another call.

The MS is made to initiate a second call (call C). The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. The MS shall return the AOCC acknowledgement within 1 second of transmission of the CONNECT message.

The MS invokes the multi-party service by sending a FACILITY message to the SS containing the BuildMPTY request

The SS accepts the request and connects the MS with the other existing connections (active call C and held call B) and confirms with a FACILITY message.

The SS sends the DISCONNECT to the MS for call B x seconds after sending the call B CAI in the CONNECT message and the DISCONNECT for call C y seconds after sending the call C CAI in the CONNECT message. The MS shall have stored the correct amount on the SIM according to the two sets of e-parameters sent and call times x and y.

Maximum Duration of Test:

z minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either D or E branch is taken

D12	MS -> SS	CONNECT ACKNOWLEDGE
	D13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

E12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	E13	MS -> SS	CONNECT ACKNOWLEDGE

14	MS -> SS	HOLD	Call B put on hold
	15	SS -> MS	HOLD ACKNOWLEDGE
	16	MS 		The MS is made to initiate a second call
	17	MS -> SS	SETUP	TI arbitrary but different from existing TI
	18	SS -> MS	CALL PROCEEDING
	19	SS -> MS	ALERTING
	20	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either F or G branch is taken

F21	MS -> SS	CONNECT ACKNOWLEDGE
	F22	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

G21	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	G22	MS -> SS	CONNECT ACKNOWLEDGE

The MS is made to build a multi-party call
	23	MS -> SS	FACILITY (TI A-B/A-C)	As default message except contains Facility IE 
				with contents as indicated in iii below
	24	SS -> MS	FACILITY (TI A-B/A-C)	As default message except contains Facility IE 
				with contents as indicated in iv, below

	25			Call durations x and y seconds after respective
				CAI information sent by SS

Branch H and branch I are taken,
				the sequence depending on the durations x and y

For call C
	H26	SS -> MS	DISCONNECT	y seconds after call C CAI sent
	H27	MS -> SS	RELEASE
	H28	SS -> MS	RELEASE COMPLETE

For call B
	I26	SS -> MS	DISCONNECT	x seconds after call B CAI sent
	I27	MS -> SS	RELEASE
	I28	SS -> MS	RELEASE COMPLETE

29	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	30	MS		SIM contents checked (either via MMI or by 
				removing the SIM and using SIM reader) ACM 
				shall have been incremented correctly.

NOTE 6:

-	A-B/A-C indicates a choice. the transaction identifier (TI) used must be that of the active call or the held call (ref. GSM 04.84).

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x set to 180s, y set to 90s.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

Call		e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

B(time x)	19	40	1	0	0	0	0

C(time y)	29	40	1	0	0	0	0	134		134

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

iii) FACILITY	Information Element with Invoke = BuildMPTY component type as defined in GSM 04.80 section 3.6.1 table 3.4.

The following abbreviations are used in the descriptions below:

U	Universal tag class

CS	Context Specific tag class

P	Primitive tag form

C	Constructed tag form

FIE	Facility Information Element

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00011100

Length of FIE contents	8	00001000

Component type tag 	CS/C/tag=1	10100001

Component length	6	00000110

Invoke ID tag	U/P/tag=2	00000010

Invoke ID length	1	00000001

Invoke ID	Arbitrary 	(00000000)			(1 octet)

Op-Code tag	From GSM 04.80	00000010

Op-Code length	1	00000001

Op-Code	Build Multi-party		00000001
	operation						(local value 124)

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00111100

Length of FIE contents	5	00001001

Component type tag 	CS/C/tag=2	10100010

Component length	3	00000011

Invoke ID tag	U/P/Integer	00000010

Invoke ID length	1	00000001

Invoke ID	Same as used as	(00000000)			Invoke ID in Invoke FIE

### 31.6.2	Charge Storage

#### 31.6.2.1	Removal of SIM during an active call

Purpose:

1)	To verify that when the SIM is removed from the ME during an active AOCC call the ME immediately terminates the call.

2)	To verify that when the SIM is removed during an active AOCC call the ME has written the total charge up to that point in the call to the ACM field of the SIM.

Conformance Requirement(s):

1)	When the SIM is removed from the ME during an active AOCC call the ME shall immediately terminate the call.

2)	When the SIM is removed during an active AOCC call midway through an AOC charging time interval (e7 or e2) the ME shall have written the total charge up to the point in the call where the charging time interval last expired to the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 02.17.

Conformance requirement 1:	TS GSM 02.24.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Removal of the SIM is possible without disconnection of power supply

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

This test is only performed if it is possible to remove SIM without disconnecting the power supply. If the battery pack must be removed to get at the SIM see 31.6.2.2.

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

If possible, without removing the power supply, the SIM is removed from the ME y seconds after the SS sends the CAI in the CONNECT/FACILITY message. The call shall be terminated immediately by the MS and the MS shall have stored the correct amount on the SIM according to the e-parameters sent.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE
	A13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	B13	MS -> SS	CONNECT ACKNOWLEDGE

14			SIM removed y seconds after CAI information 
				sent by SS.

Either C, D, E or F branch is taken

C15	MS -> SS	DISCONNECT
	C16	SS -> MS	RELEASE
	C17	MS -> SS	RELEASE COMPLETE
	C18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

D15	MS -> SS	RELEASE COMPLETE
	D16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

E15	MS -> SS	Layer 2 DISC
	E16	SS -> MS	UA

F15			No further messages are sent

19			SIM contents checked (by removing the SIM 
				and using SIM reader). ACM shall have been 
				incremented correctly.

NOTE 7:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The SIM is removed after approximately y=90 seconds

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

10	55	1	10	0	0	10	30	30

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.2.2	Interruption of power supply during an active call

Purpose:

To verify that when the power supply of the MS is removed during an active AOCC call the ME has written the total charge up to that point in the call to the ACM field of the SIM.

Conformance Requirement(s):

When the power supply of the MS is removed during an active AOCC call midway through an AOC charging time interval (e7 or e2) the ME shall have written the total charge up to the point in the call where the charging time interval last expired to the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 02.24.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of removing power supply

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is switched off.

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

The ME power is switched off by pressing power button on MMI during the active call. The call shall be terminated immediately and the MS shall have stored the correct amount on the SIM according to the e-parameters sent.

The test is repeated for ME power being lost by removal of battery pack. The call shall be terminated immediately and the MS shall have stored the correct amount on the SIM according to the e-parameters sent.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,2

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE
				with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE
	A13	MS -> SS	FACILITY	As default message except contains Facility IE
				with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	B13	MS -> SS	CONNECT ACKNOWLEDGE

14			ME power interrupted y seconds after CAI 
				information sent by SS by:
				MMI power switch (for k=1)
				Removing battery pack (for k=2)
				Depending on the value of k
				C or D branch is taken
	15			SIM contents checked (by removing the
				SIM and using SIM reader). ACM shall have
				been incremented correctly.

NOTE 8:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The ME power is removed after approximately y=90 seconds

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

10	55	1	10	0	0	10	30	30

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.2.3	MS going out of coverage during an active AOCC call

NOTE 9:	At present the core recommendations for AOCC in the case where the radio link is lost during an active call are vague. Does the mobile go on charging for a fixed period after radio link loss and continue as usual if radio link reestablishment occurs, or does charging stop? Input from people involved in charging for GSM is required.

Hence this test will be enhanced in the future when the requirements become clearer.

Purpose:

To verify that when the MS goes out of radio coverage area and an active call is dropped the ME has written the total charge up to that point in the call to the ACM field of the SIM.

Conformance Requirement(s):

When the MS goes out of radio coverage area during an active AOCC call midway through an AOC charging time interval (e7 or e2) and the call is dropped the ME shall have written the total charge up to the point in the call where the charging time interval last expired to the ACM field of the SIM.

Reference(s):

Conformance requirement 1:	TS GSM 02.24.

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

The cell simulated by the system simulator is then switched off to model the MS losing radio coverage. The MS shall have stored the correct amount on the SIM according to the e-parameters sent. [What happens on call reestablishment?]

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	MS 		The MS is made to initiate a call
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP
	7	SS -> MS	CALL PROCEEDING
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	SS -> MS	ALERTING
	11	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in i below

Either A or B branch is taken

A12	MS -> SS	CONNECT ACKNOWLEDGE
	A13	MS -> SS	FACILITY	As default message except contains Facility IE
				with contents as indicated in ii below

B12	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	B13	MS -> SS	CONNECT ACKNOWLEDGE

14			SS cell switched off y seconds after CAI 
				information sent by SS
				Failure of radio path, end of call (CEND) occurs
				and MS stops charging (ref. GSM 02.24 sec 2)
	15			SIM contents checked (either via MMI or by
				removing the SIM and using SIM reader). ACM
				shall have been incremented correctly.

NOTE 10:

-	The value of y is given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

The cell being simulated by the SS is switched off after approximately y=90 seconds

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

10	55	1	10	0	0	10	30	30

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.2.4	ACMmax operation / Mobile Originating

Purpose:

1)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, any outgoing calls in progress for which a non-zero CAI exists are terminated by the ME with an appropriate indication given to the user.

2)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the making of non-emergency calls is inhibited

3)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the making of emergency calls is uninhibited.

Conformance Requirement(s):

1)	When the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, any outgoing calls in progress for which a non-zero CAI exists shall be terminated by the ME with an appropriate indication given to the user.

2)	When the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the making of further non-emergency calls shall be inhibited

3)	When the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the making of emergency calls shall be uninhibited.

Reference(s):

Conformance requirement 1:	TS GSM 02:24

Conformance requirement 2:	TS GSM 02.24

Conformance requirement 3:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Type of user indication when ACMmax exceeded

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

PIN 2 is entered into the MS allowing modification of both the ACM and ACMmax fields on the SIM. The ACM is reset to zero and the ACMmax is set to 2 units.

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The MS is made to initiate a call. The call is established with certain AOCC non-zero e-parameters sent in a Facility IE in the CONNECT message. It is an implementation option whether the AOCC acknowledge is sent before or after the CONNECT ACKNOWLEDGE.

After the ACM has been incremented to 2 units the call shall be terminated by the MS and an indication given to the user. The call duration is recorded and the ACM is checked to ensure it has been incremented to 2 units.

The MS is then made to attempt to originate an ordinary call to the MS for which a non-zero CAI exists for the calling party and shall be unsuccessful. The MS shall not send a CHANNEL REQUEST for that call.

The MS is then made to attempt to originate an emergency call and shall be successful.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM (it shall be zero)

For k= 1
	A1	MS 		The MS is made to initiate an ordinary call
	A2	MS -> SS	CHANNEL REQUEST
	A3	SS -> MS	IMMEDIATE ASSIGNMENT
	A4	MS -> SS	CM SERVICE REQUEST
	A5	SS -> MS	CM SERVICE ACCEPT
	A6	MS -> SS	SETUP
	A7	SS -> MS	CALL PROCEEDING
	A8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	A9	MS -> SS	ASSIGNMENT COMPLETE
	A10	SS -> MS	ALERTING
	A11	SS -> MS	CONNECT	As default message except contains Facility IE 
				with contents as indicated in I below

For k = 3, 4
	B1	MS 		The MS is made to initiate an emergency call 					(112 is entered)
	B2	MS -> SS	CHANNEL REQUEST	Establishment cause is emergency call 						establishment
	B3	SS -> MS	IMMEDIATE ASSIGNMENT
	B4	MS -> SS	CM SERVICE REQUEST	CM service type IE indicates "emergency call 					establishment"
	B5	SS -> MS	CM SERVICE ACCEPT
	B6	MS -> SS	EMERGENCY SETUP
	B7	SS -> MS	CALL PROCEEDING
	B8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	B9	MS -> SS	ASSIGNMENT COMPLETE
	B10	SS -> MS	ALERTING
	B11	SS -> MS	CONNECT	As default message except contains Facility IE 					with contents as indicated in i below
				For k=1 or 3 either branch C or D is taken

C12	MS -> SS	CONNECT ACKNOWLEDGE
	C13	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below

D12	MS -> SS	FACILITY	As default message except contains Facility IE 					with contents as indicated in ii below
	D13	MS -> SS	CONNECT ACKNOWLEDGE

For k=1
	L14			Record call duration, x seconds, after CAI 					information sent by SS until call is terminated 					(when ACM = ACMmax)
	L15	MS -> SS	DISCONNECT
	L16	SS -> MS	RELEASE
	L17	MS -> SS	RELEASE COMPLETE
	L18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	L19			ACM checked (shall be 2 units)

For k=3
	M14			Call duration y seconds after CAI information 					sent by SS
	M15	MS -> SS	DISCONNECT
	M16	SS -> MS	RELEASE
	M17	MS -> SS	RELEASE COMPLETE
	M18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	M19			ACM checked (shall be 2 units)

k=1 - Non zero CAI call attempted by MS and should succeed

k=2 - Non zero CAI call attempted by MS and should fail

k=3 - Emergency call attempted by MS and should succeed

NOTE 11:

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x shall be 60 ± 2 seconds

y shall be set to 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

1	1	30	1	0	0	0	0	2	2
		2	1	30	1	0	0	0	0	0	2
		3	0	0	0	0	0	0	0	0	2

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

#### 31.6.2.5	ACMmax operation / Mobile Terminating

Purpose:

1)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, any mobile terminating calls in progress for which a non-zero CAI exists are terminated by the ME with an appropriate indication given to the user.

2)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, and an incoming call is received for which subsequently a non-zero CAI is received, then the call is terminated by the ME with an appropriate indication given to the user.

3)	To verify that when the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the receiving of calls for which the CAI is zero is uninhibited.

Conformance Requirement(s):

1)	When the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, any mobile terminating calls in progress for which a non-zero CAI exists shall be terminated by the ME with an appropriate indication given to the user.

2)	When the value stored in the ACM becomes equal to or exceeds its maximum value, the ACMmax, and an incoming call is received for which subsequently a non-zero CAI is received, the call shall be terminated by the ME with an appropriate indication given to the user.

3)	When the value stored in the ACM becomes equal to or exceeds its maximum limit, the ACMmax, the receiving of calls for which the CAI is zero shall be uninhibited.

Reference(s):

Conformance requirement 1:	TS GSM 02:24.

Conformance requirement 2:	TS GSM 02.24

Conformance requirement 2:	TS GSM 02.24

Related PICS/PIXIT Statement(s):

-	Support of AOCC

-	Supported rates (full rate/half rate)

-	Method of reading ACM from the SIM via the ME

-	Type of user indication when ACMmax exceeded

-	Supported teleservices

-	Support for active state of the call control protocol (U10)

Initial Conditions:

System Simulator:

1 cell, default parameters, IMSI attach/detach disabled.

Mobile Station:

The MS is in MM-state "idle, updated".

PIN 2 is entered into the MS allowing modification of both the ACM and ACMmax fields on the SIM. The ACM is reset to zero and the ACMmax is set to 2 units.

Foreseen Final State of the MS:

The MS is in MM-state "idle, updated".

Method of Test:

The SS is made to initiate a call. The call is established with certain non-zero AOCC e-parameters sent in a Facility IE contained in a FACILITY message sent before the CONNECT message. It is an implementation option whether the AOCC e-parameters and AOCC acknowledge are sent before or after the CONNECT ACKNOWLEDGE.

After the ACM has been incremented to 2 units the call shall be terminated by the MS and an indication given to the user. The ACM shall be checked to ensure that it has been incremented to 2 units.

The SS is then made to attempt to make an ordinary call to the MS for which a non-zero CAI exists for the called party and shall be unsuccessful. The MS shall terminate the call with a DISCONNECT message.

The SS is then made to attempt to make an ordinary call to the MS for which a zero CAI exists for the called party and shall be successful.

Maximum Duration of Test:

[z] minutes

Expected Sequence:

The sequence step 1-20 is executed for execution counter k = 1,...,3.

Step	Direction	Message	Comments

0			At start of test only, read and note value of 
				ACM on SIM
	1	SS 		The SS is made to initiate a call
	2	SS -> MS	PAGING REQUEST
	3	MS -> SS	CHANNEL REQUEST
	4	SS -> MS	IMMEDIATE ASSIGNMENT
	5	MS -> SS	PAGING RESPONSE
	6	SS -> MS	SETUP
	7	MS -> SS	CALL CONFIRMED
	8	SS -> MS	ASSIGNMENT COMMAND	to a supported channel type
	9	MS -> SS	ASSIGNMENT COMPLETE
	10	MS -> SS	ALERTING
	11	MS -> SS	CONNECT
	12	SS -> MS	CONNECT ACKNOWLEDGE

For k=1 or 3 branch A is taken

A13	SS -> MS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in i below
	A14	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

For k=2 branch B, and then either branch C,
				D or E is taken

B13	SS -> MS	FACILITY	As default message except contains Facility IE
				with contents as indicated in i below

C13	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below
	C14	MS -> SS	DISCONNECT	MS terminates call

D13	MS -> SS	DISCONNECT	MS terminates call
	D14	MS -> SS	FACILITY	As default message except contains Facility IE 
				with contents as indicated in ii below

E13	MS -> SS	DISCONNECT	MS terminates call.
				DISCONNECT may or may not contain Facility 
				IE with contents as indicated in ii below

For k= 1, 2 and 3, F, G and  branch shall be 
				taken respectively

For k=1
	F15			Record call duration, x seconds, after CAI  
				information sent by SS until call is terminated 
				(when ACM = ACMmax)
	F16	MS -> SS	DISCONNECT	MS terminates call
	F17	SS -> MS	RELEASE
	F18	MS -> SS	RELEASE COMPLETE
	F19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For k=2
	G15	SS -> MS	RELEASE
	G16	MS -> SS	RELEASE COMPLETE	RELEASE COMPLETE may or may not contain 
				Facility IE with contents as indicated in ii below
	G17	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For k=3
	H15			Call duration y seconds after CAI information 
				sent by SS
	H16	MS -> SS	DISCONNECT	MS terminates call
	H17	SS -> MS	RELEASE
	H18	MS -> SS	RELEASE COMPLETE
	H19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

20			ACM checked (should be 2 units)

k=1 - Non zero CAI call attempted to MS and should succeed

k=2 - Non zero CAI call attempted to MS and should fail

k=3 - Zero CAI call attempted to MS and should succeed

NOTE 12:

-	The values of x and y are given below.

-	The correct value of the ACM is found from the general AOC formula given in GSM 02.24 section 4 with CDUR = x,y seconds and e parameters as defined below.

Specific Message Contents:

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

For ASN.1 description see default message contents in section 31.6.4.

x shall be 60 ± 2 seconds

y shall be set to 90 seconds.

The values of the e-parameters within the parameter part of the Facility Information Element shall be set to:

k-value	e-parameter		CCM total	Step 0 ACM
					at call end	value 								increased by

1	2	3	4	5	6	7

1	1	30	1	0	0	0	0	2	2
		2	1	30	1	0	0	0	0	0	2
		3	0	0	0	0	0	0	0	0	2

Values shown in table are in the format and have units as in GSM 02.24 section 3.

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

For ASN.1 description see default message contents in section 31.6.4.

### 31.6.3	Advice of Charge Information

### 31.6.4	Default contents of messages

As default message contents in GSM 11.10 Section 26.8 "Tests related to circuit switched call control". These messages shall not contain SS version IEs.

Where indicated in specific tests CONNECT and FACILITY messages have Facility Information Elements as defined below.

Default contents of ForwardChargeAdvice Facility Information Elements

The following abbreviations are used in the descriptions below:

U	Universal tag class

CS	Context Specific tag class

P	Primitive tag form

C	Constructed tag form

FIE	Facility Information Element

i) FACILITY	Information Element with Invoke = ForwardChargeInformation component type as defined in GSM 04.80 section 3.6.1 table 3.3.

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00011100

Length of FIE contents	43	00101011

Component type tag 	CS/C/tag=1	10100001

Component length	41	00101001

Invoke ID tag	U/P/tag=2	00000010

Invoke ID length	1	00000001

Invoke ID	Arbitrary 	(00000000)			(1 octet)

Op-Code tag	From GSM 04.80	00000010

Op-Code length	1	00000001

Op-Code	Forward Charge	01111101
	Advice operation 					(local value 125)

Sequence Identifier	U/C/tag=16 	00110000			from CCITT X.208

Length Indicator	33	00100001

SS-Code Identifier	CS/P/tag=0	10000000

Length Indicator	1	00000001

SS-Code	AOCC SS-Code	01110010

Charging Info. identifier	CS/C/tag=1	10100001

Length Indicator	28	00011100

e1 Identifier	CS/P/tag=1	10000001

Length Indicator	2	00000010

e1	(2 Octets)	See e-parameter 			table in relevant test

e2 Identifier	CS/P/tag=2	10000010

Length Indicator	2	00000010

e2	(2 Octets)	See e-parameter 			table in relevant test

.	.	.

.	.	.

.	.	.

e7 Identifier	CS/P/tag=7	10000111

Length Indicator	2	00000010

e7	(2 Octets)	See e-parameter 			table in relevant test

ii) FACILITY	Information Element with Return Result component type as defined in GSM 04.80 section 3.6.1 table 3.4.

Contents	Value/remark	Coding

Facility IE Identifier	As GSM 04.80	00111100

Length of FIE contents	5	00001001

Component type tag 	CS/C/tag=2	10100010

Component length	3	00000011

Invoke ID tag	U/P/Integer	00000010

Invoke ID length	1	00000001

Invoke ID	Same as used as	(00000000)		                                          Invoke ID in Invoke FIE

## 31.7	Additional information transfer supplementary services

(Reserved).

## 31.8	Call restriction supplementary services

The following abbreviations are used

BO :	Barring of Outgoing calls

BAOC :	Barring of All Outgoing Calls

BOIC :	Barring of Outgoing International Calls

BOICExHC :	Barring of Outgoing International Call EXcept those directed to the Home PLMN country

BI :	Barring of Incoming calls

BAIC :	Barring of All Incoming calls

BICRoam :	Barring of Incoming when Roaming outside the home PLMN country

B :	Barring (common name for BAOC, BOIC, BOICExHC, BAIC and BICRoam).

These abbreviations are also used to represent the corresponding SS-Code; e.g. B is the SS-Code for all barring services.

NOTE:	The password(s) to be used during tests of this section 31.8 may be randomly chosen - unless otherwise stated - in accordance with GSM 02.04 section 5.2.

### 31.8.1	Registration of a password

#### 31.8.1.1	Registration accepted

31.8.1.1.1	Conformance requirements

1)	For registration of a password for all barring services, the MS shall transmit successively

1.1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

1.2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

1.3)	and then the REGISTER message containing a facility IE (or an empty REGISTER followed by a FACILITY message containing the facility IE) that includes an invoke of the RegisterPassword operation with parameter values according to the user's request (MMI action).

2)	Upon receipt of the different FACILITY messages part of the procedure of registration of a password for all barring services, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

3)	The MS shall be able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

4)	Upon receipt of the result of the procedure, contained in RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1.1)	GSM 04.08 sections 3.3.1.2, 9.1.9

1.2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

1.3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

2)	GSM 02.30 section 4.5

3)	GSM 04.80

4)	GSM 02.30

31.8.1.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for registration of a password for all barring services in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for registration of a password for all barring services in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of a password for all barring services.

4)	To check that upon receipt of the different FACILITY messages part of the procedure of registration of a password for all barring services, the MS provides the appropriate user indication (as described by the manufacturer).

5)	To check that the MS is able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

6)	To check that upon receipt of the result of the procedure, contained in RELEASE COMPLETE or FACILITY message, the MS provides the appropriate user indication (as described by the manufacturer).

These checks are done for:

all barring services, the result of the operation being sent in a RELEASE COMPLETE message.

31.8.1.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of a new password for all barring services.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the old password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring a new password.

Then, by means of appropriate MMI functions, the user gives a new password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring again the new password.

Then, by means of appropriate MMI functions, the user gives again the new password.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing the return result of the RegisterPassword operation.

Maximum duration of test

3 minutes

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of a password for all call
				barring services
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	Invoke = GetPassword (password)
	9	MS		provide correct MMI user. indication.
				A password is entered.
	10	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	11	SS -> MS	FACILITY 	Invoke = GetPassword (new password)
	12	MS		provide correct MMI user indication
				A password is entered.
	13	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	14	SS -> MS	FACILITY 	Invoke = GetPassword (new password again)
	15	MS		provide correct MMI user indication
				A password is entered.
	16	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	17	SS -> MS	RELEASE COMPLETE	RegisterPassword operation ReturnResult
	18	SS -> MS	CHANNEL RELEASE

Specific message content

step A6 or B7 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = RegisterPassword

Supplementary service code = B

steps 8, 11, and 14 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : FACILITY

-	facility

invoke = GetPassword

Supplementary service code = B

Guidance info : Password (step 8), new password (step 11), new password again (step 14)

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

#### 31.8.1.2	Registration rejected

##### 31.8.1.2.1	Rejection after invoke of the RegisterPassword operation

31.8.1.2.1.1	Conformance requirements

1)	A transaction of any kind being already established, for registration of a password for all call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.8.1.2.1.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of registration of a password for all call restriction services, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of a password for all call restriction services.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for:

all call restriction services, the RELEASE COMPLETE message being sent at the beginning of the procedure with a facility IE containing a return_error(error) where error is "SS subscription violation".

31.8.1.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statement(s)

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of a new password for all call restriction services.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the RELEASE COMPLETE (PD and TI of the SS transaction) message with the Facility information element containing a Return_error(error : SS subscription violation) of the RegisterPassword operation.

Upon receipt of the FACILITY message, the system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

2 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a registration of a new password
				for all call restriction services
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	RegisterPassword operation Return_error
	7	MS		provide correct MMI user indication
	8	SS -> MS	STATUS ENQUIRY
	9	MS -> SS	STATUS	CC staTE U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = RegisterPassword

Supplementary service code = all call restrictions

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility:

return error code : SS subscription violation

For the return error the invoke ID must be the same as in the invoke of the RegisterPassword operation.

##### 31.8.1.2.2	Rejection after password check with negative result

31.8.1.2.2.1	Conformance requirements

1)	A transaction of any kind being already established, for registration of a password for all call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6, GSM 04.10 section 4.2.2, GSM 03.11 section 3

4)	GSM 02.30 section 4.5

31.8.1.2.2.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of registration of a password for all call restriction services, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of a password for all call restriction services.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for :

all call restriction services, the RELEASE COMPLETE message being sent at the end of the procedure with a facility IE containing a return_error(error) where error is "NegativePasswordCheck".

31.8.1.2.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

WPA > 3

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of a new password for all call restriction services. By means of appropriate MMI functions the user enters the old and new passwords.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the old password.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE message (PD and TI of the SS transaction) with the Facility information element containing a Return_error(error : NegativePasswordCheck) of the RegisterPassword operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

5 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of a new password
				for all call restriction services. The old and new
				passwords are entered.
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	FACILITY 	Invoke = GetPassword (password)
	7	SS -> MS	RELEASE COMPLETE	Register Password operation ReturnError
	8	MS		provide correct MMI user indication
	9	SS -> MS	STATUS ENQUIRY
	10	MS -> SS	STATUS	CC staTE U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = RegisterPassword

Supplementary service code = B

step 7 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility:

error code : NegativePasswordCheck

For the reject the invoke ID must be the same as in the invoke of the Registerpassword operation.

step 6 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type : FACILITY

-	facility

invoke = GetPassword

Supplementary service code = B

Guidance info : Password (step 1)

##### 31.8.1.2.3	Rejection after new password mismatch

31.8.1.2.3.1	Conformance requirements

1)	A transaction of any kind being already established, for registration of a password for all call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.8.1.2.3.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of registration of a password for all call restriction services, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the RegisterPassword operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for registration of a password for all call restriction services.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for :

all call restriction services, the RELEASE COMPLETE message being sent at the end of the procedure with a facility IE containing a return_error(error) where error is "PasswordRegistrationFailure" with diagnostic "new password mismatch".

31.8.1.2.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of a new password for all call restriction services.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the old password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the new password.

Then, by means of appropriate MMI functions, the user gives a new password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring again the new password.

Then, by means of appropriate MMI functions, the user gives again a new password.

Upon receipt of the FACILITY message, the system simulator answers with the RELEASE COMPLETE message (PD and TI of the SS transaction) with the Facility information element containing a Return_error(error : PasswordRegistrationFailure, parameter : NewPasswordMismatch) of the RegisterPassword operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of a new password
				for all call restriction services
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	FACILITY 	Invoke = GetPassword (password)
	7	MS		provide correct MMI user indication
				A password is entered.
	8	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	9	SS -> MS	FACILITY 	Invoke = GetPassword (new password)
	10	MS		provide correct MMI user indication
				A password is entered.
	11	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	12	SS -> MS	FACILITY 	Invoke = GetPassword (new password again)
	13	MS		provide correct MMI user indication
				A password is entered.
	14	MS -> SS	FACILITY 	GetPassword operation ReturnResult
	15	SS -> MS	RELEASE COMPLETE	Register Password operation ReturnError
	16	MS		provide correct MMI user indication
	17	SS -> MS	STATUS ENQUIRY
	18	MS -> SS	STATUS	CC staTE U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke =	RegisterPassword

Supplementary service code = B

step 15 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility:

error code : PasswordRegistrationFailure

parameter : NewPasswordMismatch

For the reject the invoke ID must be the same as in the invoke of the Registerpassword operation.

steps 6, 9, 11 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type : FACILITY

-	facility

invoke =	GetPassword

Supplementary service code = 	B

Guidance info : Password (step 6), new password (step 9), new password again (step 11)

### 31.8.2	Erasure

Not applicable

### 31.8.3	Activation

#### 31.8.3.1	Activation accepted

31.8.3.1.1	Conformance requirements

1)	For activation of any specific call restriction service with any parameters, the MS shall transmit successively

1.1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

1.2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

1.3)	and then the REGISTER message containing a facility IE that includes an invoke of the ActivateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

2) Upon receipt of FACILITY message requiring the password, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

3) The MS shall be able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

4) Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.8.3.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for activation of a specific call restriction service in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for activation of call restriction service in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for activation of a specific call restriction service.

4)	To check that upon receipt of FACILITY message requiring the password, the MS provides the appropriate user indication (as described by the manufacturer).

5)	To check that the MS is able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

6)	To check that upon receipt of the RELEASE COMPLETE or FACILITY message, the MS provides the appropriate user indication (which is to be described by the manufacturer).

These checks are done for:

a)	BAOC, for basic service group "all synchronous services" the result of the operation being sent in a FACILITY message.

b)	BICRoam, for all basic service groups, the result of the operation being sent in a RELEASE COMPLETE message.

31.8.3.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests activation of BAOC, for basic service group "all synchronous services".

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing the Return_result of the ActivateSS operation.

The SS transaction and the dedicated channel are released.

Then again, by means of appropriate MMI functions, the user requests activation of BICRoam, for all basic service groups.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing the Return_result of the ActivateSS operation.

The dedicated channel is released.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a activation of BAOC(all synchronous services)
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	GetPassword
	9	MS		provide correct MMI user indication
				A password is entered.
	10	MS -> SS	FACILITY 	Getpassword operation Return Result
	11	SS -> MS	FACILITY 	ActivateSS operation Return_result
	12	SS -> MS	RELEASE COMPLETE
	13	SS -> MS	CHANNEL RELEASE

14	MS 		The MS is made to initiate an activation
				of BICRoam(all basic service groups),
	15	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	16	SS -> MS	IMMEDIATE ASSIGNMENT
	17	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	18	SS -> MS	CM SERVICE ACCEPT

A19	MS -> SS	REGISTER

B19	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B20	MS -> SS	FACILITY

21	SS -> MS	FACILITY 	GetPassword
	22	MS		provide correct MMI user indication
				A password is entered.
	23	MS -> SS	FACILITY 	Getpassword operation Return Result
	24	SS -> MS	RELEASE COMPLETE	ActivateSS operation Return result
	25	MS		provide correct MMI user indication
	26	SS -> MS	CHANNEL RELEASE

Specific message contents

step A6 or B7 - BAOC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Activation

Supplementary service code = BAOC

Basic service code : Bearer Service (all synchronous services), no teleservice present.

step A16 or B17 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B19

-	message type : REGISTER (A16), FACILITY (B20)

-	facility

invoke = Activation

Supplementary service code = BICRoam

Basic service code : no bearer service present, no teleservice present.

steps 8, 21 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type : FACILITY

-	facility

invoke = GetPassword

Supplementary service code = all call restrictions

Guidance info : Password

#### 31.8.3.2	Activation rejected

##### 31.8.3.2.1	Rejection after invoke of ActivateSS operation

31.8.3.2.1.1	Conformance requirements

1)	A transaction of any kind being already established, for activation of one specific barring services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.8.3.2.1.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of specific call barring service, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for specific call barring service.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for :

BOIC, the RELEASE COMPLETE message being sent at the beginning of the procedure with a facility IE containing a return_error(error) where error is "SS subscription violation".

31.8.3.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests activation of BOIC.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the RELEASE COMPLETE (PD and TI of the SS transaction) message with the Facility information element containing a Return_error(error : SS subscription violation) of the ActivateSS operation.

Upon receipt of the FACILITY message, the system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

2 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of a new password
				for BOIC
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	RegisterPassword operation Return_error
	7	MS		provide correct MMI user indication
	8	SS -> MS	STATUS ENQUIRY
	9	MS -> SS	STATUS	CC state U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke =	ActivateSS

Supplementary service code = BOIC

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : SS subscription violation

For the return error the invoke ID must be the same as in the invoke of the ActivateSS operation.

##### 31.8.3.2.2	Rejection after use of password procedure

31.8.3.2.2.1	Conformance requirements

1)	A transaction of any kind being already established, for activation of any specific call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.8.3.2.2.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of activation of one specific call restriction service, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the ActivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for activation of one specific call restriction service.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for :

BAIC, the RELEASE COMPLETE message being sent at the end of the procedure with a facility IE containing a return_error(error) where error is "NegativePasswordCheck".

31.8.3.2.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests registration of a new password for all call restriction services.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator sends RELEASE COMPLETE message (PD and TI of the SS transaction) with the Facility information element containing a Return_error(error : NegativePasswordCheck) of the GetPassword operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
 				a registration of BAIC
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	FACILITY 	Invoke = GetPassword (password)
	7	MS		provide correct MMI user indication
				A password is entered.
	8	SS -> MS	FACILITY	GetPassword operation Return Result
	9	SS -> MS	RELEASE COMPLETE	Register Password operation ReturnError
	10	MS		provide correct MMI user indication
	11	SS -> MS	STATUS ENQUIRY
	12	MS -> SS	STATUS	CC state U10

Specific message content

step A4 or B5 - BAIC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke =	Activation

Supplementary service code = BAIC

step 6 - All Barring services

-	protocol discriminator: non call related SS message

-	transaction identifier: in FACILITY same as previous REGISTER message

-	message type: FACILITY

-	facility

invoke = GetPassword

Supplementary service code = B

Guidance info: Password (step 6)

step 9 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : NegativePasswordCheck

For the reject the invoke ID must be the same as in the invoke of the GetPassword operation.

### 31.8.4	Deactivation

#### 31.8.4.1	Deactivation accepted

31.8.4.1.1	Conformance requirements

1)	For deactivation of any group of call restriction services with any parameters, the MS shall transmit successively

1.1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

1.2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

1.3)	and then the REGISTER message containing a facility IE that includes an invoke of the DeactivateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

2)	Upon receipt of FACILITY message requiring the password, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

3)	The MS shall be able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

3)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.8.4.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for deactivation of a group of call barring services in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for deactivation of a group of call barring services in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for deactivation of a group of call restriction services.

4)	To check that upon receipt of FACILITY message requiring the password, the MS provides the appropriate user indication (as described by the manufacturer).

5)	To check that the MS is able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

6)	To check that upon receipt of the RELEASE COMPLETE or FACILITY message, the MS provides the appropriate user indication (which is to be described by the manufacturer).

These checks are done for:

a)	all restrictions, for basic service group "speech" the result of the operation being sent in a FACILITY message.

b)	barring of outgoing calls, for all facsimile, the result of the operation being sent in a RELEASE COMPLETE message.

31.8.4.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests deactivation of all restrictions, for speech.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing the Return_result of the DeactivateSS operation.

The SS transaction and the dedicated channel are released.

Then again, by means of appropriate MMI functions, the user requests activation of barring of outgoing calls, for all facsimile.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing the Return_result of the DeactivateSS operation.

The dedicated channel is released.

Maximum duration of test

5 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a deactivation of all call restrictions(speech)
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	GetPassword
	9	MS		provide correct MMI user indication
				A password is entered.
	10	MS -> SS	FACILITY 	Getpassword operation Return Result
	11	SS -> MS	FACILITY 	DeactivateSS operation Return_result
	12	SS -> MS	RELEASE COMPLETE
	13	SS -> MS	CHANNEL RELEASE

14	MS 		The MS is made to initiate
				a deactivation of barring of outgoing
				calls(all facsimile),
	15	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	16	SS -> MS	IMMEDIATE ASSIGNMENT
	17	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	18	SS -> MS	CM SERVICE ACCEPT

A19	MS -> SS	REGISTER

B19	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B20	MS -> SS	FACILITY

21	SS -> MS	FACILITY 	GetPassword
	22	MS		provide correct MMI user indication
				A password is entered.
	23	MS -> SS	FACILITY 	Getpassword operation Return Result
	24	SS -> MS	RELEASE COMPLETE	DeactivateSS operation Return result
	25	MS		provide correct MMI user indication
	26	SS -> MS	CHANNEL RELEASE

Specific message contents

step A6 or B7 - all call restrictions:

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Deactivation

Supplementary service code = B

Basic service code : no Bearer Service present, teleservice : speech.

step A16 or B17 - barring of outgoing calls,

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Deactivation

Supplementary service code = BO

Basic service code : no bearer service present, teleservice : all facsimile.

steps 8, 21 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type : FACILITY

-	facility

invoke = GetPassword

Supplementary service code = all call restrictions

Guidance info : Password

#### 31.8.4.2	Deactivation rejected

##### 31.8.4.2.1	Rejection after invoke of DeactivateSS operation

31.8.4.2.1.1	Conformance requirements

1)	A transaction of any kind being already established, for deactivation of a group of call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.8.4.2.1.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of a group of call barring services, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for a group of call barring services.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

These checks are performed with a call transaction already established for :

BOIC, the RELEASE COMPLETE message being sent at the beginning of the procedure with a facility IE containing a return_error(error) where error is "SS subscription violation".

31.8.4.2.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests deactivation of incoming calls.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the RELEASE COMPLETE (PD and TI of the SS transaction) message with the Facility information element containing a Return_error(error : SS subscription violation) of the DeactivateSS operation.

Upon receipt of the FACILITY message, the system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

30 s

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a deactivation
				for bi
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	RELEASE COMPLETE 	DeactivateSS operation Return_error
	7	MS		provide correct MMI user indication
	8	SS -> MS	STATUS ENQUIRY
	9	MS -> SS	STATUS	CC state U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = DeactivateSS

Supplementary service code = bi

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : SS subscription violation

For the return error the invoke ID must be the same as in the invoke of the DeactivateSS operation.

##### 31.8.4.2.2	Rejection after use of password procedure

31.8.4.2.2.1	Conformance requirements

1)	A transaction of any kind being already established, for deactivation of a group of call restriction services, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.8.4.2.2.2	Test purpose

1)	To check that, when a call transaction is already established, the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of deactivation of a group of call restriction services, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the DeactivateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for deactivation of a group of call restriction service.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

Those checks are performed with a call transaction already established for :

BOICExHC, the RELEASE COMPLETE message being sent at the end of the procedure with a facility IE containing a return_error(error) where error is "NegativePasswordCheck".

31.8.4.2.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests deactivation of a group of call restriction services.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the DeactivateSS operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator sends RELEASE COMPLETE message (PD and TI of the SS transaction) with the Facility information element containing a Return_error(error : NegativePasswordCheck) of the GetPassword operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a deactivation of BoicExHC
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	MS -> SS	FACILITY

6	SS -> MS	FACILITY 	Invoke = GetPassword (password)
	7	MS		provide correct MMI user indication
				A password is entered.
	8	SS -> MS	RELEASE COMPLETE	Register Password operation ReturnError
	9	MS		provide correct MMI user indication
	10	SS -> MS	STATUS ENQUIRY
	11	MS -> SS	STATUS	CC state U10

Specific message content

step A4 or B5 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B4

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = DeactivateSS

Supplementary service code = BOICExHC

step 11 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : NegativePasswordCheck

For the reject the invoke ID must be the same as in the invoke of the DeactivateSS operation.

step 6 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type :FACILITY

-	facility

invoke = GetPassword

Supplementary service code =

Guidance info : Password (step 6)

### 31.8.5	Invocation

Invocation is not applicable.

### 31.8.6	Interrogation

#### 31.8.6.1	Interrogation accepted

31.8.6.1.1	Conformance requirements

1)	For interrogation of any specific call restriction service with any parameters, the MS shall transmit successively

1.1)	a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user",

1.2)	a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

1.3)	and then the REGISTER message containing a facility IE that includes an invoke of the InterrogateSS operation (or an "empty" REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

2)	Upon receipt of FACILITY message requiring the password, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

3)	The MS shall be able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

4)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.8.6.1.2	Test purpose

1)	To check that the MS correctly requests a supplementary service transaction for interrogation of a specific call barring service in CHANNEL REQUEST message.

2)	To check that the MS correctly requests a supplementary service transaction for interrogation of a call barring service in the subsequent CM-SERVICE REQUEST.

3)	To check that the MS sends a REGISTER message containing the invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for interrogation of one call restriction service.

4)	To check that upon receipt of FACILITY message requiring the password, the MS provides the appropriate user indication (as described by the manufacturer).

5)	To check that the MS is able to send a password by sending a FACILITY message in accordance to the user request (MMI actions).

6)	To check that upon receipt of the RELEASE COMPLETE or FACILITY message, the MS provides the appropriate user indication (which is to be described by the manufacturer).

These checks are done for :

a)	BAIC, the result of the operation being a Basic Service code sent in a FACILITY message.

b)	BOICExHC, the result of the operation being a SS-status sent in a RELEASE COMPLETE message.

31.8.6.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests interrogation of BAIC.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with the FACILITY message with the Facility information element containing the Return_result (basic service) of the InterrogateSS operation.

The SS transaction and the dedicated channel are released.

Then again, by means of appropriate MMI functions, the user requests activation of BOICExHC.

Upon receipt of the operation (in a REGISTER or a FACILITY message), the system simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation requiring the current password.

Then, by means of appropriate MMI functions, the user gives a password.

Upon receipt of the FACILITY message, the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing the Return_result(SS-status) of the InterrogateSS operation.

The dedicated channel is released.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a interrogation of BAIC
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	3	SS -> MS	IMMEDIATE ASSIGNMENT
	4	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	5	SS -> MS	CM SERVICE ACCEPT

A6	MS -> SS	REGISTER

B6	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B7	MS -> SS	FACILITY

8	SS -> MS	FACILITY 	GetPassword
	9	MS		provide correct MMI user indication
				A password is entered.
	10	MS -> SS	FACILITY 	Getpassword operation Return Result
	11	SS -> MS	FACILITY 	InterrogateSS operation Return_result
	12	SS -> MS	RELEASE COMPLETE
	13	SS -> MS	CHANNEL RELEASE

14	MS 		The MS is made to initiate
 				a interrogation of call forwarding service
				for BOICExHC,
	15	MS -> SS	CHANNEL REQUEST	with establishment cause
				"Other procedures which can be completed
				with an SDCCH"
	16	SS -> MS	IMMEDIATE ASSIGNMENT
	17	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	18	SS -> MS	CM SERVICE ACCEPT

A19	MS -> SS	REGISTER

B19	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B20	MS -> SS	FACILITY

21	SS -> MS	FACILITY 	GetPassword
	22	MS		provide correct MMI user indication
				A password is entered.
	23	MS -> SS	FACILITY 	Getpassword operation Return Result
	24	SS -> MS	RELEASE COMPLETE	InterrogateSS operation Return result
	25	MS		provide correct MMI user indication
	26	SS -> MS	CHANNEL RELEASE

Specific message contents

step A6 or B7 - BOIC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A6), FACILITY (B7)

-	facility

invoke = Interrogation

Supplementary service code = BAIC

step A16 or B17 - BOICExHC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A16), FACILITY (B17)

-	facility

invoke = Interrogation

Supplementary service code = BOICExHC

steps 8, 21 -

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as previous REGISTER message

-	message type : FACILITY

-	facility

invoke = GetPassword

Supplementary service code = B

Guidance info : Password

#### 31.8.6.2	Interrogation rejected

31.8.6.2.1	Conformance requirements

1)	A transaction of any kind being already established, for interrogation of any specific call barring with any parameters, the MS shall establish a parallel MM transaction, sending a CM SERVICE REQUEST with CM service type indicating "supplementary service activation",

2)	Then the MS shall send a REGISTER message related to the present SS transaction containing a facility IE that includes an invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with parameter values according to the user's request (MMI action).

3)	Upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the SS transaction shall be released but the first transaction shall remain unaffected.

4)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the Manufacturer).

References

1)	GSM 04.08 sections 3.3.1.2, 9.1.9

2)	GSM 04.08 sections 4.5.1.1, 9.2.9, 10.5.33

3)	GSM 04.88, GSM 04.80 sections 2.3, 3.6

4)	GSM 02.30 section 4.5

31.8.6.2.2	Test purpose

1)	To check that the MS correctly requests the establishment of a parallel MM transaction for supplementary service transaction of interrogation of a specific call barring service message, sending a CM-SERVICE REQUEST.

2)	To check that the MS sends a REGISTER message containing the invoke of the InterrogateSS operation (or an empty REGISTER followed by a FACILITY message containing the invoke) with the expected parameter values for interrogation of call barring.

3)	To check that upon receipt of the RELEASE COMPLETE message related to the present SS transaction, the first transaction remains unaffected.

4)	To check that upon receipt of the RELEASE COMPLETE message, the MS provides the appropriate user indication (as described by the Manufacturer).

These checks are performed with a call transaction already established for :

a)	BICRoam, the RELEASE COMPLETE message being sent with a facility IE containing a return_error(error) where error is "SS not available".

b)	BOIC, the RELEASE COMPLETE message being sent with a facility IE containing a reject(invoke_problem) where invoke_problem is "resource limitation".

31.8.6.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in CC state U10.

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is in CC state U10.

Test procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI), the user requests interrogation of BICRoam.

Upon receipt of the operation (in a REGISTER or a FACILITY message),the system simulator answers with the FACILITY message with the Facility information element containing a Return_error(error : SS not available) of the InterrogateSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Then again, by means of appropriate MMI functions, the user requests interrogation of BOIC.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE message (same PD and TI that in the REGISTER message) with the Facility information element containing a reject(invoke_problem : resource limitation) of the InterrogateSS operation.

The system simulator sends STATUS ENQUIRY, the MS responds with STATUS message indicating CC state U10.

Maximum duration of test

3 mn

Expected sequence

Step	Direction	Message	Comments

1	MS 		The MS is made to initiate
				a interrogation of call barring service
				for BICRoam
	2	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	3	SS -> MS	CM SERVICE ACCEPT

A4	MS -> SS	REGISTER

B4	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B5	FACILITY

6	SS -> MS	RELEASE COMPLETE 	InterrogateSS operation Return_error
	7	SS -> MS	STATUS ENQUIRY
	8	MS -> SS	STATUS	CC state U10

9	MS 		The MS is made to initiate
				a interrogation of call barring service
				for BOIC
	10	MS -> SS	CM SERVICE REQUEST	cause : "supplementary service activation"
	11	SS -> MS	CM SERVICE ACCEPT

A12	MS -> SS	REGISTER
	B12	MS -> SS	REGISTER 	protocol discriminator :
				"non call related SS message",
				containing no SSoperation (invoke)
	B13	FACILITY

14	SS -> MS	RELEASE COMPLETE 	(SS)
	15	MS		provide correct MMI user indication
	16	SS -> MS	STATUS ENQUIRY
	17	MS -> SS	STATUS	CC state U10

Specific message content

step A4 or B5 - BICRoam

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B6

-	message type : REGISTER (A4), FACILITY (B5)

-	facility

invoke = Interrogation

Supplementary service code = BICRoam

step 6 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

return error code : SS not available

For the return error the invoke ID must be the same as in the invoke of the InterrogateSS operation.

step A12 or B13 - BOIC

-	protocol discriminator : non call related SS message

-	transaction identifier : in FACILITY same as at step B16

-	message type : REGISTER (A12), FACILITY (B13)

-	facility

invoke = Interrogation

Supplementary service code = BOIC

step 14 -

-	protocol discriminator: non call related SS message

-	transaction identifier : same TI as previous REGISTER message

-	message type : RELEASE COMPLETE

-	facility :

reject code : resource limitation

For the reject the invoke ID must be the same as in the invoke of the InterrogateSS operation.

### 31.8.7	Normal operation

In case of barring of outgoing call the calling mobile receives information about the activation of supplementary services subscribed.

In case of barring of incoming call the calling mobile receives information about the activation of supplementary services subscribed by the other party (the mobile called).

31.8.7.1	Conformance requirements

Upon receipt of the RELEASE COMPLETE message the MS shall provide the appropriate user indication (which is to be described by the manufacturer).

References

31.8.7.2	Test purpose

To check that upon receipt of the RELEASE COMPLETE message the MS provides the appropriate user indication (as described by the manufacturer).

This is tested in the case of an MS making a call to a mobile with incoming calls barred.

31.8.7.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Related PICS/PIXIT statements

Description of the user's commands and of display of the answers from the network for call barring

Foreseen final state of the MS

The MS is "idle updated".

Test procedure

The MS is made to initiate a call.

Upon receipt of the SETUP message, the system simulator answers with the negative acknowledgement RELEASE COMPLETE (to simulate a case where call barring is activated).

Maximum duration of test

1 mn

Expected sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	
	2	SS -> MS	IMMEDIATE ASSIGNMENT
	3	MS -> SS	CM SERVICE REQUEST
	4	SS -> MS	CM SERVICE ACCEPT
	5	MS -> SS	SETUP
	6	SS -> MS 	RELEASE COMPLETE	provide correct MMI user indication
	7	SS -> MS	CHANNEL RELEASE

Specific message content

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke = notification

SS code = BI

SS status = activation indicator (indicating :

Provisioned, registered and active)

## 31.9	Handling of undefined (future) GSM supplementary services

### 31.9.1	Mobile station initiated Unstructured supplementary service data operation

#### 31.9.1.1	ProcessUnstructuredSS-request/accepted

Conformance requirements

1)	The mobile station shall invoke an USSD request by sending a REGISTER message to the network containing a ProcessUnstructuredSS-Request invoke component. This message will contain the alphabet indicator set to "SMS default alphabet" and the language indicator set to "language unspecified". The ussd-string parameter shall contain the following digits and symbols depending on the operation initiated :

Activation			*NN(N)# (no supplementary information included)

*NN(N)*SI# (one field of supplementary infor. included)

*NN(N)*SIA*SIB# (two fields of supplementary infor. included)

Deactivation			#NN(N)#

#NN(N)*SI#

#NN(N)*SIA*SIB#

Interrogation		*#NN(N)#

*#NN(N)*SI#

*#NN(N)*SIA*SIB#

Registration			**NN(N)#

**NN(N)*SI#

**NN(N)*SIA*SIB#

Erasure			##NN(N)#

##NN(N)*SI#

##NN(N)*SIA*SIB#

Operations not yet		see conformance requirement 2)

defined in GSM 02.30

NN(N) features a set of service codes which have not yet been allocated for GSM supplementary services (see GSM 02.30 for service codes already specified).

N is a digit within 1..9 and SI, SIA, SIB strings of characters.

2) 	Concerning operations which are not yet specified in GSM 02.30, the MS shall proceed as follows:

The entry of 1 or 2 characters defined in the GSM 03.38 default alphabet followed by "SEND" shall be interpreted by the MS as an USSD request unless the MS is not engaged in a call and the first of the two character entry followed by "SEND" is a "1".

3)	For supplementary service procedures independent of any call, the initiating side shall  establish a MM-connection between the network and the mobile station according to the rules given in TS GSM 4.08.

4)	Within a call, the MS shall transmit a USSD request from the user if any. See TS GSM 04.07 and 04.08 for the handling of multiple MM connections.

5)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall display the information contained to the user in a way described by the manufacturer.

References

Conformance requirement 1 : TS GSM 04.90 subclause 6.1, TS GSM 02.30 subclause 4.5.2, and TS GSM 02.90 subclause 4.1.1

Conformance requirement 2 : TS GSM 02.30 subclause 4.5.3.2

Conformance requirement 3 : TS GSM 04.10 subclause 3.2.1

Conformance requirement 4 : TS GSM 04.08 subclause 4.5.1.1

Conformance requirement 5 : TS GSM 03.90 subclause 6.2.1

Test Purpose

1)	To verify that the mobile station  invokes an USSD request by sending a REGISTER message to the network containing a ProcessUnstructuredSS-Request invoke component. This message will contain the alphabet indicator set to "SMS default alphabet" and the language indicator set to "language unspecified". The ussd-string parameter shall contain the following digits and symbols depending on the operation initiated :

Activation			*NN(N)# (no supplementary information included)

*NN(N)*SI# (one field of supplementary infor. included)

*NN(N)*SIA*SIB# (two fields of supplementary infor. included)

Deactivation			#NN(N)#

#NN(N)*SI#

#NN(N)*SIA*SIB#

Interrogation		*#NN(N)#

*#NN(N)*SI#

*#NN(N)*SIA*SIB#

Registration			**NN(N)#

**NN(N)*SI#

**NN(N)*SIA*SIB#

Erasure			##NN(N)#

##NN(N)*SI#

##NN(N)*SIA*SIB#

Operations not yet		see 2)

defined in GSM 02.30

NN(N) features a set of service codes which have not yet been allocated for GSM supplementary services (see GSM 02.30 for service codes already specified).

N is a digit within 1..9 and SI, SIA, SIB strings of characters.

2) 	To check that the entry of 1 or 2 characters defined in the GSM 03.38 default alphabet followed by "SEND" shall be interpreted by the MS as an USSD request unless the MS is not engaged in a call and the first of the two character entry followed by "SEND" is a "1".

3)	To verify that, for supplementary service procedures independent of any call, the initiating side must establish a MM-connection between the network and the mobile station according to the rules given in TS GSM 4.07 and 4.08.

4)	To verify that, within a call the MS shall transmit a USSD request if any. See TS GSM 04.07 and 04.08 for the handling of multiple MM connections.

5)	To check that upon receipt of the RELEASE COMPLETE or FACILITY message, the MS shall display the information contained to the user in a way described by the manufacturer.

Related PICS/PIXIT Statements

- support of USSD

- Description of the user's commands and of display of USSD

- Full rate supported

- Supported teleservices

- Support of active state of the call control protocol (U10)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle updated".

Foreseen final state of the MS

MM-state "idle updated".

Test Procedure

By means of appropriate MMI functions (using either 02.30 or manufacturer defined MMI) the mobile is made to initiate an Unstructured SS data operation. The mobile first establishes a MM-connection with the SS. Then, a REGISTER message is sent to the SS. Upon receipt of this message, the system simulator answers with the RELEASE COMPLETE or FACILITY message. If the FACILITY message is used, the mobile station sends a RELEASE COMPLETE message. Then  a CHANNEL RELEASE message is sent to the MS to release the main signalling link.

The mobile station is forced to originate a call. After the MS has received a CONNECT ACKNOWLEDGE, MMI keys are depressed on the mobile in order to initiate an Unstructured SS data operation. Then a REGISTER message is sent to the SS. Upon receipt of this message, the system simulator answers with the RELEASE COMPLETE or FACILITY message. If the FACILITY message is used, the mobile station sends a RELEASE COMPLETE message.  Finally, the main signalling link is released by transferring a CHANNEL RELEASE message to the MS.

Expected Sequence

The sequence is executed for execution counters c=1,..,17 and k=1,2.

Counter c  determines the ussd-string selected by the user and sent by the MS (see specific message contents concerning the REGISTER message).

Counter k   determines if the FACILITY message sent by the network includes information to the user (see specific message contents on FACILITY message).

Step	Direction	Message	Comments

1	MS		The user presses appropriate MMI keys to
				initiate the desired unstructured SS data
				operation.

2	MS -> SS	CHANNEL REQUEST	with establishment cause set to "Other
				procedures which can be completed with an
				SDCCH".
	3	SS -> MS	IMM. ASSIGN
	4	MS -> SS	CM SERVICE REQUEST 	"Supplementary service activation".
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	REGISTER	The SS checks that the content of this message
				matches specific message content i).

either A or B branch is taken by the SS. Each of
				one will have to be run through at least once
				during the test.

A7	SS -> MS	FACILITY	See specific message contents ii).It is checked
				that ussd string, if any, is displayed by the MS
				in a way described by the manufacturer.
	A8	MS -> SS	RELEASE COMPLETE	END key is pressed on the mobile keypad. It
				terminates the transaction used to initiate
				the desired undefined supplementary service
				data operation.
				See specific message contents iii).

B7	SS -> MS	RELEASE COMPLETE	It terminates the transaction used to initiate
				the desired unstructured SS data operation. It
				is checked that the ussd string, if any, is
				displayed by the MS in a way described by the
				manufacturer. See iii).

9	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

10	MS -> SS	CHANNEL REQUEST	with establishment cause related to mobile
				originating call.
	11	SS -> MS	IMMEDIATE ASSIGNMENT	a TCH/F channel is assigned to the MS.
	12	MS -> SS	CM SERVICE REQUEST	"mobile originating call"
	13	SS -> MS	CM SERVICE ACCEPT
	14	MS -> SS	SETUP
	15	SS -> MS	CALL PROCEEDING
	16	SS -> MS	ALERTING
	17	SS -> MS	CONNECT
	18	MS -> SS	CONNECT ACKNOWLEDGE

19			Specific MMI keys are depressed to initiate the
				desired undefined SS service operation. DTMF
				signalling may occur.

20	MS -> SS	CM SERVICE REQUEST	"Supplementary service activation".
	21	SS -> MS	CM SERVICE ACCEPT
	22	MS -> SS	REGISTER	The MS starts the transaction on the radio
				interface. The SS checks that REGISTER
				matches i).

either A or B branch is taken by the SS. Each
				branch will have to be run through at least
				once during the test.

A23	SS -> MS	FACILITY	See specific message contents ii).It is checked
				that the ussd string, if any, is displayed by the
				MS in a way described by the manufacturer.
	A24	MS -> SS	RELEASE COMPLETE	It terminates the transaction used to initiate the
				desired undefined SS data operation. See
				specific message contents iii).

B23	SS -> MS	RELEASE COMPLETE	It terminates the transaction used to activate
				the desired undefined SS data operation.
				See specific message contents iii).

24	SS -> MS	DISCONNECT
	25	MS -> SS	RELEASE
	26	SS -> MS	RELEASE COMPLETE	See message contents iv).
	27	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific message contents

i) REGISTER message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier

Register message type			As GSM 4.80

Facility Information Element		See below

SS version indicator				As specified in 04.80

Facility Information Element with Invoke = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.3

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						Arbitrary (1 octet)

Operation Code tag				From GSM 04.80

Operation Code length			1

Operation Code					ProcessUnstructuredSS-Request

ussd-dataCodingScheme			1 octet long. Alphabet indicator set to "default alphabet".
								Language indicator set to "undefined"

ussd-string						c=1, *60# (Activation  with no supplementary information
								included)
								c=2, *201*35# (Activation with one field of supplementary
								information included)
								c=3, *70*635*562# ( Activation with two fields of supplementary
								information included)
								c=4, #60# (Deactivation  with no supplementary information
								included)
								c=5, #201*35# (Deactivation  with one field of supplementary
								information included)
								c=6, #70*635*562# (Deactivation  with two fields of
								supplementary information included)
								c=7, *#60# (Interrogation  with no supplementary information
								included)
								c=8, *#201*35# (Interrogation  with one field of supplementary
								information included)
								c=9, *#70*635*562# (Interrogation  with two fields of
								supplementary information included)
								c=10, **60# (Registration  with no supplementary
								information included)
								c=11, **201*35# (Registration  with one field of
								supplementary information included)
								c=12, **70*635*562# (Registration  with two fields of
								supplementary information included)
								c=13, ##60# (Erasure with no supplementary
								information included)
								c=14, ##201*35# (Erasure with one field of
								supplementary information included)
								c=15, ##70*635*562# (Erasure  with two fields of
								supplementary information included)
								c=16,7 (related to conformance requirement  2)
								c=17, 26 (related to conformance requirement  2)

ii) Facility message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier				The transaction value is the same as the REGISTER
								transaction value, but the transaction flag is different

Facility message type				As GSM 4.80

Facility Information Element		See below

Facility Information Element with Invoke = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.3.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the ProcessUnstructuredSS-
								Request

Operation Code tag				k=1, field omitted
								k=2, From GSM 04.80

Operation Code length			k=1, field omitted
								k=2, 1

Operation Code					k=1, field omitted
								k=2, UnstructuredSS-Request

ussd-dataCodingScheme			k=1, field omitted
								k=2, 1 octet long. Alphabet indicator set to "default
								alphabet". Language indicator set to "undefined"

ussd-string						k=1, field omitted
								k=2, chosen at random

iii) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier				the transaction value is the same as REGISTER transaction 
								value but the transaction flag is different

Release Complete message type	As GSM 4.80

Cause							for step A8, cause set to user normal call clearing
								otherwise, omitted.

Facility Information Element		See below

Facility information element with Return Result = ProcessUnstructuredSS-Request component type as defined in GSM 04.80 section 3.6.1 table 3.4.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of IE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of the ussd-string

Invoke ID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the 
								ProcessUnstructuredSS-Request

Sequence Identifier				k = 1, field omitted
								k = 2, From 04.80

Operation Code tag				k = 1, field omitted
								k = 2, From 04.80

Operation Code length			k = 1, field omitted
								k = 2, 1

Operation Code 					k = 1, field omitted
								k = 2, ProcessUnstructuredSS-Request

ussd-dataCodingScheme			k = 1, field omitted
								k = 2, 1 octet long. Alphabet indicator set to “default alphabet”
								Language indicator set to “undefined”

ussd-string						k = 1, field omitted
								k = 2, chosen at random

iv) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call control)"

Transaction identifier				same as current call.

Release Complete message type	As GSM04.08

#### 31.9.1.2	ProcessUnstructuredSS-request/cross phase compatibility and error handling

Conformance requirements

1)	If a mobile initiated USSD request using protocol version 2 is rejected by the network, and the reason for the rejection is indicated either by the problem code "unrecognized operation" or a cause "facility rejected", the mobile station shall assume that the network only supports protocol version 1 of USSD operations. The mobile station shall re-attempt the request by using the appropriate protocol version 1 USSD operation without a SS version indicator if the unstructured data entered by the user can be coded as an IA5 string.

2)	Upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). If  ussd-String information is included this shall be given to the user (in a way described by the manufacturer).

References

Conformance requirement1:TS GSM 4.90 subclause 6.2.1

Conformance requirement2:TS GSM 3.90 subclause

Test Purpose

1) 	To verify that If a mobile initiated USSD request using protocol version 2 is rejected by the network, and the reason for the rejection is indicated either by the problem code "unrecognized operation" or a cause "facility rejected", the mobile station shall assume that the network only supports protocol version 1 of USSD operations. The mobile station shall re-attempt the request by using the appropriate protocol version 1 USSD operation without a SS version indicator if the unstructured data entered by the user can be coded as an IA5 string.

2)	To check that, upon receipt of the RELEASE COMPLETE message, the MS shall provide the appropriate user indication (which is to be described by the manufacturer). If  ussd-String information is included this shall be given to the user (in a way described by the manufacturer).

Related PICS/PIXIT Statements

- support of USSD

- Full rate supported

- Supported teleservices

- Support of active state of the call control protocol (U10)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle updated".

Foreseen final state of the MS

MM-state "idle updated".

Test Procedure

Appropriate MMI keys are depressed on the mobile in order to activate an USSD service. The mobile first establishes a MM-connection with the SS. Then, a REGISTER message is sent to the SS. Upon receipt of this message, the system simulator answers with the RELEASE COMPLETE message with the cause element set to "Facility rejected" or the problem code set to "unrecognized operation". Then the SS checks that the MS re-attempts the request by using a REGISTER message containing an invoke of the ProcessUnstructuredData operation. The SS answers with a normal RELEASE COMPLETE to terminate the transaction. Finally the main signalling link is released by transferring to the MS a CHANNEL RELEASE.

Appropriate MMI keys are depressed on the mobile in order to activate an USSD service. The mobile first establishes a MM-connection with the SS. Then, a REGISTER message is sent to the SS. Upon receipt of this message, the system simulator answers with the RELEASE COMPLETE message with the cause element set to "Facility rejected", the main signalling link is release. Then the SS checks that the MS re-establishes a MM-connection and re-attempts the request by using a REGISTER message containing an invoke of the ProcessUnstructuredData operation. Then the SS answers with a normal RELEASE COMPLETE to terminate the transaction. Finally the main signalling link is released by transferring to the MS a CHANNEL RELEASE.

The mobile station is forced to originate a call. After the SS has sent a CONNECT ACKNOWLEDGE, MMI keys are depressed on the mobile in order to activate an USSD service. A REGISTER message is sent to the SS. Upon receipt of this message, the system simulator answers with the RELEASE COMPLETE message containing a return error with an error or a reject with a problem. Then the radio link is release by transferring to the MS a CHANNEL RELEASE message. This subtest is repeated with different errors and problems in the RELEASE COMPLETE message.

Expected Sequence

Step	Direction	Message	Comments

1	MS		The user presses appropriate MMI keys to
				initiate the desired undefined supplementary
				service operation.

2	MS -> SS	CHANNEL REQUEST	with establishment cause
				set to "Other procedures which can be
				completed with an SDCCH".
	3	SS -> MS	IMM. ASSIGN
	4	MS -> SS	CM SERVICE REQUEST 	"Supplementary service activation".
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	REGISTER	See specific message contents i).Operation
				code set to "ProcessUnstructuredSS-Request"
	7	SS -> MS	RELEASE COMPLETE	See specific message contents ii).
	8	MS -> SS	CM SERVICE REQUEST	"Supplementary service activation".
	9	SS -> MS	CM SERVICE ACCEPT
	10	MS -> SS	REGISTER	Operation code is set to
				"ProcessUnstructuredSS-Data”. See specific
				message content i).
	11	MS -> SS	RELEASE COMPLETE	normal release of the transaction. See iiia).
	12	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

13	MS		The user presses appropriate MMI keys to
				initiate the desired undefined supplementary
				service.

14	MS -> SS	CHANNEL REQUEST	with establishment cause
				set to "Other procedures which can be
				completed with an SDCCH".
	15	SS -> MS	IMM. ASSIGN
	16	MS -> SS	CM SERVICE REQUEST 	"Supplementary service activation".
	17	SS -> MS	CM SERVICE ACCEPT
	18	MS -> SS	REGISTER	See specific message contents i).Operation
				code is set to "ProcessUnstructureSS-
				Request”. See i).
	19	MS -> SS	RELEASE COMPLETE	see iii).cause is set to "operation rejected"
	20	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	21	MS -> SS	CHANNEL REQUEST	with establishment cause
				set to "Other procedures which can be
				completed with an SDCCH".
	22	SS -> MS	IMM. ASSIGN
	23	MS -> SS	CM SERVICE REQUEST 	"Supplementary service activation".
	24	SS -> MS	CM SERVICE ACCEPT
	25	MS -> SS	REGISTER	Operation code is
				"ProcessUnstructuredSSdata”. See specific
				message contents i).
	26	SS -> MS	RELEASE COMPLETE	normal release of the transaction. See iiia).
	27	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

For k=1 to 10, go through steps  28 to 
				45. Counter k deals with different kinds of
				general and invoke problems in step  41.

28	MS -> SS	CHANNEL REQUEST	with establishment cause related to mobile
				originating call.
	29	SS -> MS	IMMEDIATE ASSIGNMENT	a TCH/F channel is assigned to the MS.
	30	MS -> SS	CM SERVICE REQUEST	"mobile originating call"
	31	SS -> MS	CM SERVICE ACCEPT
	32	MS -> SS	SETUP
	33	SS -> MS	CALL PROCEEDING
	34	SS -> MS	ALERTING
	35	SS -> MS	CONNECT
	36	MS -> SS	CONNECT ACKNOWLEDGE

37			Specific MMI keys are depressed to initiate
				the desired undefined supplementary service.
				DTMF signalling may occur.

38	MS -> SS	CM SERVICE REQUEST	"supplementary service activation"
	39	SS -> MS	CM SERVICE ACCEPT
	40	MS -> SS	REGISTER	The MS starts the transaction on the radio
				interface. See specific message contents i).The
				operation code is set to
				"ProcessUnstructuredSS-Request".

41	SS -> MS	RELEASE COMPLETE	different errors and problems are sent. See
				specific message contents iiib) and iiic).
	42	SS -> MS	DISCONNECT
	43	MS -> SS	RELEASE
	44	SS -> MS	RELEASE COMPLETE	See message contents iv).
	45	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific message contents

i) REGISTER message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier

Register message type			As GSM 4.80

Facility Information Element		See below

SS version indicator				As specified in GSM 04.80 for version 2 protocol
								Always omitted for version 1 protocol

For steps 6, 18 and 40, Facility Information Element with Invoke = ProcessUnstructuredSS-Request (for version 2 protocol) component type as defined in GSM 4.80 section 3.6.1 table 3.3

For steps 10 and 25, Facility Information Element with Invoke = ProcessUnstructuredSS-Data (for version 1 protocol) as defined in GSM 4.80 section 3.6.1 table 3.3

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						Arbitrary (1 octet)

Operation Code tag				From GSM 04.80

Operation Code length			1

Operation Code					ProcessUnstructuredSS-Request (for version 2 protocol)
								ProcessUnstructuredSS-Data (for version 1 protocol)

ussd-dataCodingScheme			1 octet long. Alphabet indicator set to "default alphabet".
								Language indicator set to "undefined"

ussd-string						*70*635*562#

ii)  RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier				the transaction value is the same as the REGISTER
								transaction value but the transaction flag is different

Facility message type				As GSM 4.80

Facility Information Element		See below

Facility Information Element with Reject = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.6.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Reject from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the ProcessUnstructuredSS-
								Request

Problem Code tag				As GSM 04.80

Problem Code length

General Problem code			Unrecognized operation

iii) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier				the transaction value is the same as the REGISTER
								transaction value but the transaction flag is different

Release complete message type	As GSM 4.80

cause							for step 19, cause is set to "facility rejected" and FIE is
								omitted.
								for steps 11, 26, 41 this field is omitted.

Facility Information Element		for step 11 and 26 see iiia).
								for step 41 see iiib) and iic).
								For step 19 this field is omitted.

iiia)	For steps 6, 18 and 40, Facility Information Element with Return Result = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.4.

For steps 10 and 25, Facility Information Element with Return Result = ProcessUnstructuredSS-Data component type as defined in GSM 4.80 section 3.6.1 table 3.4.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the ProcessUnstructuredSS-
								Request

iiib)	Facility Information Element with Return Error = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.5.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Return Error from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the ProcessUnstructuredSS-
								Request

Error Code tag					As GSM 04.80

Error Code length

Error Code						k=1, system failure
								k=2, data missing
								k=3, unknown alphabet
								k=4, unexpected data value

iiic)	Facility Information Element with Reject = ProcessUnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.6.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Reject from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the ProcessUnstructuredSS-
								Request

Problem Code tag				As GSM 04.80

Problem Code length

General Problem code			k=5, Unrecognized component
								k=6, Mistyped component
								k=7, Badly structured component
Invoke Problem code				k=8, Mistyped parameter
								k=9, Resource limitation
								k=10, Initiating release

iv) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call control)"

Transaction identifier				same as current call.

Release Complete message type	As GSM04.08

### 31.9.2	Network initiated unstructured supplementary service operations

#### 31.9.2.1	UnstructuredSS-Notify/accepted

Conformance requirements

1)	For a USSD notification, the MS shall display the text provided and await user input. The MS shall acknowledge the operation by sending a FACILITY message containing an empty result component to the network.

2)	The MS shall include alphabet and language indicators in the response to the network. The alphabet indicator shall indicate "SMS default alphabet". The language indicator shall indicate "language unspecified".

3)	At any stage while the MS is registered with a network, the network may send an unstructured string to the MS. So, the MS shall be able to process the operation during a call or out of a call.

References

conformance requirement 1: TS GSM 4.90 subclause 5.2.1

conformance requirement 2: TS GSM 2.90 subclause 4.2.2

conformance requirement 3: TS GSM 2.90 SUBCLAUSE 4.2.1

Test Purpose

1)	To verify that for a USSD notification, the MS shall display the text provided and await user input. If the user enters a response, the MS shall acknowledge the operation by sending a FACILITY message containing an empty result component to the network.

2)	To verify that the MS shall include alphabet and language indicators in the response to the network. The alphabet indicator shall indicate "SMS default alphabet". The language indicator shall indicate "language unspecified".

3)	To check that the MS shall be able to process the operation during a call or out of a call.

Related PICS/PIXIT Statements

- support of USSD

- Full rate supported

- Supported teleservices

- Support of active state of the call control protocol (U10)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle updated".

Foreseen final state of the MS

MM-state "idle updated".

Test Procedure

-	The MS is paged and a RR-connection is established. Then, the SS sends a REGISTER message containing a facility information element with operation code set to ProcessUnstructuredSS-Notify. The user checks that the ussd string sent by the SS is correctly displayed by the MS. The MS has to send a FACILITY message with an empty return result component.

-	Then the SS originates a call to the MS. When the MS is in the U10 state, the SS releases the transaction identifier concerning USSD transaction by sending a RELEASE COMPLETE. The SS initiates a new ussd transaction by sending a REGISTER message containing a facility information element with operation code set to UnstructuredSS-Notify. The user checks that the ussd string sent by the SS is correctly displayed by the MS. The MS has to send a FACILITY message with an empty return result component. Finally the SS releases both MM connections and the radio link.

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	a SDCCH is allocated to the MS.
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	REGISTER	initiation of the transaction concerning the
				USSD notification operation. See i).
	6			The MS has to display the USSD string sent
				from the network.
	7	MS -> SS	FACILITY	signalling message sent by the MS as the
				response. The SS checks that it matches ii).
	8	SS -> MS	SETUP	incoming call.
	9	MS -> SS	CALL CONFIRMED
	10	MS -> SS	ALERTING
	11	MS -> SS	CONNECT
	12	SS -> MS	ASSIGNMENT COMMAND	a TCH is allocated to the MS.
	13	MS -> SS	ASSIGNMENT COMPLETE
	14 	SS -> MS	CONNECT ACKNOWLEDGE
	15	SS -> MS	RELEASE COMPLETE	this message releases the transaction
				concerning the USSD operation. See iii).
	16	SS -> MS	REGISTER	initiation of a USS request operation during a
				call. See specific message contents i).
	17			The MS has to display the USSD string
				received from the SS.
	18	MS -> SS	FACILITY	The SS checks that this message matches ii).
	19	SS -> MS	RELEASE COMPLETE	It releases he transaction identifier concerning
				the ussd operation.
	20	SS -> MS	DISCONNECT	call release initiation. see iv).
	21	MS -> SS	RELEASE
	22	SS -> MS	RELEASE COMPLETE
	23	SS -> MS	CHANNEL RELEASE

Specific message contents

i) REGISTER message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier

Register message type			As GSM 4.80

Facility Information Element		See below

Facility Information Element with Invoke = UnstructuredSS-Notify component type as defined in GSM 04.90, clause 5.2.1

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						Arbitrary (1 octet)

Operation Code tag				From GSM 04.80

Operation Code length			1

Operation Code					UnstructuredSS-Notify

ussd-dataCodingScheme			1 octet long. Alphabet indicator set to "default alphabet".
								Language indicator set to "undefined"

ussd-string						 arbitrary chosen by the SS

ii) Facility message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier				the transaction value is the same as REGISTER transaction
								value but the transaction flag is different

Facility message type				As GSM 4.80

Facility Information Element		See below

Facility Information Element with Return Result = empty result component according to GSM 04.90, clause 5.2.1, figure 5.3.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Return Result from GSM 04.80

Component length

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the UnstructuredSS-Notify

iii) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier				the transaction value is the same as REGISTER transaction
								flag is different

Release Complete message type	As GSM 4.80

Facility Information Element		omitted

iv) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call control)"

Transaction identifier				same as current call.

Release Complete message type	As GSM04.08

31.9.2.2	UnstructuredSS-Notify/rejected on user busy

Conformance requirements

1)	When the mobile station receives an USSD operation in parallel to any call independent supplementary transaction, it shall respond with a return error component in a RELEASE COMPLETE message, containing the "USSD-Busy” error.

References

conformance requirement 1: TS GSM 4.90 subclause 5.1.1

Test Purpose

1)	To verify that when the mobile station receives an USSD operation in parallel to any call independent supplementary transaction, it responds with a return error component in a RELEASE COMPLETE message, containing the "USSD-Busy" error.

Related PICS/PIXIT Statements

- support of USSD

- Full rate supported

- Supported teleservices

- Support of active state of the call control protocol (U10)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle updated".

Foreseen final state of the MS

MM-state "idle updated".

Test Procedure

-	The MS is paged and a RR-connection is established. Then, the SS sends a REGISTER message containing a facility information element with operation code set to UnstructuredSS-Notify. The SS initiates an other USSD transaction by sending a REGISTER message with transaction identifier different form the previous one. The SS checks that the MS answers with a RELEASE COMPLETE message with an error component set to "USSD busy". Then the SS releases the radio link.

Test Procedure

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	a SDCCH is allocated to the MS.
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	REGISTER	initiation of the transaction concerning the
				USSD notification operation. See i).
	6			The MS has to display the USSD string sent
				from the SS.
	7	MS -> SS	FACILITY	See ii).
	8	SS -> MS	REGISTER	initiation of an other USSD notification. The
				transaction identifier is different from that in
				step 5.See i).
	9	MS -> SS	RELEASE COMPLETE	error set to "USSD busy”. Transaction initiated
				in step 6 by the network is rejected by the
				MS. See iii).
	10	SS -> MS	RELEASE COMPLETE 	normal release of transaction initiated in step
				5.See iii).
	11	SS -> MS	CHANNEL RELEASE	release of the main signalling link.

Specific message contents

i) REGISTER message

see paragraph 31.9.2..1

ii) FACILITY message

see paragraph 31.9.2..1

iii) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier				for step 10, same as REGISTER transaction identifier
								for step 9, the transaction value is the same as REGISTER
								transaction value but the transaction flag is different.

Release complete message type	As GSM 4.80

Facility Information Element		for step  9 see iiib).
								for step  10, see iiia).

iiia)	Facility Information Element with Return Result = UnstructuredSS-Notify component type as defined in GSM 4.80 section 3.6.1 table 3.4.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Return Result from GSM 04.80

Component length

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the UnstructuredSS-Notify in step 5

iiib)	Facility Information Element with Return Error = UnstructuredSS-Notify component type as defined in GSM 4.80 section 3.6.1 table 3.5.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Return Result from GSM 04.80

Component length

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the UnstructuredSS-Notify in step 8

Error Code tag					As GSM 04.80

Error Code length

Error Code						USSD Busy

#### 31.9.2.3	UnstructuredSS-Request/accepted

Conformance requirements

1)	For a USSD request, the MS shall display the text provided and await user input. if the user enters a response, the MS shall return the response to the network, maintaining the transaction.

2)	The MS shall include alphabet and language indicators in the response to the network. The alphabet indicator shall indicate "SMS default alphabet". The language indicator shall indicate "language unspecified".

3)	At any stage while the MS is registered with a network, the network may send an unstructured string to the MS. So, the MS shall be able to process the operation during a call or out of a call.

References

conformance requirement 1: TS GSM 3.90 subclause 5.2.5

conformance requirement 2: TS GSM 2.90 subclause 4.2.2

conformance requirement 3: TS GSM 2.90 subclause 4.2.1

Test Purpose

1)	To test that, for a USSD request, the MS shall display the text provided and await user input. if the user enters a response, the MS shall return the response to the network, maintaining the transaction.

2)	To verify that the MS shall include alphabet and language indicators in the response to the network. The alphabet indicator shall indicate "SMS default alphabet". The language indicator shall indicate "language unspecified".

3)	To check that the MS shall be able to process the operation during a call or out of a call.

Related PICS/PIXIT Statements

- support of USSD

- Full rate supported

- Supported teleservices

- Support of active state of the call control protocol (U10)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle updated".

Foreseen final state of the MS

MM-state "idle updated".

Test Procedure

-	The MS is paged and a RR-connection is established. Then, the SS sends a REGISTER message containing a facility information element with operation code set to ProcessUnstructuredSS-Request. The user checks that the ussd string sent by the SS is correctly displayed by the MS and answers the request by depressing MMI keys. Then the MS has to send a FACILITY message with ussd string exactly containing the digits and symbols expressed on the mobile equipment keypad.

-	Then the SS originates a call to the MS. When the MS is in the U10 state, the SS releases the transaction identifier concerning USSD transaction by sending a RELEASE COMPLETE. The SS initiates a new ussd transaction by sending a REGISTER message containing a facility information element with operation code set to UnstructuredSS-Request. The user checks that the ussd string sent by the SS is correctly displayed by the MS and answers the request by depressing MMI keys. Then the MS has to send a FACILITY message with ussd string exactly containing the digits and symbols expressed on the mobile equipment keypad. Finally the SS releases both MM connections and the radio link.

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	a SDCCH is allocated to the MS.
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	REGISTER	initiation of the transaction concerning the
				USSD Request operation. See i).
	6			The MS has to display the USSD string sent
				from the network and waits for the user
				response. By depressing MMI keys followed
				by SEND the user answers
	7	MS -> SS	FACILITY	signalling message sent by the MS as the
				response. See ii).
	8	SS -> MS	SETUP	incoming call.
	9	MS -> SS	CALL CONFIRMED
	10	MS -> SS	ALERTING
	11	MS -> SS	CONNECT
	12	SS -> MS	ASSIGNMENT COMMAND	a TCH is allocated to the MS.
	13	MS -> SS	ASSIGNMENT COMPLETE
	14 	SS -> MS	CONNECT ACKNOWLEDGE
	15	SS -> MS	RELEASE COMPLETE	this message releases the transaction
				concerning the USSD operation. See iii).
	16	SS -> MS	REGISTER	initiation of a USS request operation during a
				call. See specific message contents i).
	17			The MS has to display the USSD string
				received from the SS. By depressing MMI
				keys followed by SEND, the user answers.
				DTMF signalling may occur.
	18	MS -> SS	FACILITY	See ii).
	19	SS -> MS	RELEASE COMPLETE	It releases the transaction identifier concerning 
				the ussd operation.
	20	SS -> MS	DISCONNECT	
	21	MS -> SS	RELEASE
	22	SS -> MS	RELEASE COMPLETE	call release initiation. see iv).
	23	SS -> MS	CHANNEL RELEASE

Specific message contents

i) REGISTER message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier

Register message type			As GSM 4.80

Facility Information Element		See below

Facility Information Element with Invoke = UnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.3

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Invoke from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						Arbitrary (1 octet)

Operation Code tag				From GSM 04.80

Operation Code length			1

Operation Code					UnstructuredSS-Request

ussd-dataCodingScheme			1 octet long. Alphabet indicator set to "default alphabet".
								Language indicator set to "undefined"

ussd-string						arbitrary chosen by the SS

ii) Facility message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier				the transaction identifier value is the same as REGISTER 
								transaction value but the transaction flag is different.

Facility message type				As GSM 4.80

Facility Information Element		See below

Facility Information Element with Return Result = UnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.4.

Contents						Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Return Result from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the UnstructuredSS-Request

Sequence Identifier

Sequence length

Operation Code tag				1

Operation Code length			1

Operation Code					UnstructuredSS-Request

ussd-dataCodingScheme			1 octet long. Alphabet indicator set to "default
								alphabet". Language indicator set to "undefined"

ussd-string						contains exactly the digits and symbols expressed on the
								mobile equipment keypad.

iii) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier				same as REGISTER transaction identifier

Release Complete message type	As GSM 4.80

Facility Information Element		omitted

iv) RELEASE COMPLETE message

Contents						Value/remark

Protocol Discriminator				set to "Supplementary service (call control)"

Transaction identifier				same as current call.

Release Complete message type	As GSM04.08

31.9.2.4	UnstructuredSS-Request/rejected on user busy

Conformance requirements

1)	When the mobile station receives an USSD operation in parallel to any call independent supplementary transaction, it shall respond with a return error component in a RELEASE COMPLETE message, containing the "USSD-Busy” error.

References

conformance requirement 1: TS GSM 4.90 subclause 5.1.1

Test Purpose

1)	To verify that when the mobile station receives an USSD operation in parallel to any call independent supplementary transaction, it responds with a return error component in a RELEASE COMPLETE message, containing the "USSD-Busy" error.

Related PICS/PIXIT Statements

- support of USSD

- Full rate supported

- Supported teleservices

- Support of active state of the call control protocol (U10)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle updated".

Foreseen final state of the MS

MM-state "idle updated".

Test Procedure

-	The MS is paged and a RR-connection is established. Then, the SS sends a REGISTER message containing a facility information element with operation code set to UnstructuredSS-Request . The SS initiates an other USSD transaction by sending a REGISTER message with transaction identifier different form the previous one. The SS checks that the MS answers with a RELEASE COMPLETE message with an error component set to "USSD busy". Then the SS releases the radio link.

Test Procedure

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1
	2	MS -> SS	CHANNEL REQUEST
	3	SS -> MS	IMMEDIATE ASSIGNMENT	a SDCCH is allocated to the MS.
	4	MS -> SS	PAGING RESPONSE
	5	SS -> MS	REGISTER	initiation of the transaction concerning the 
				USSD Request operation. See i).
	6			The MS has to display the USSD string sent 
				from the network and waits for the user 
				response. By depressing MMI keys followed 
				by SEND the user answers.
	7	MS -> SS	FACILITY	signalling message sent by the MS as the 
				response to the request. See ii). 
	8	SS -> MS	REGISTER	initiation of an other USSD request. The 
				transaction identifier is different from that in 
				step 5.See i).
	9	MS -> SS	RELEASE COMPLETE	error set to "USSD busy”. Transaction initiated 
				in step 6 by the network is rejected by the 
				MS. See iii). 
	10	SS -> MS	RELEASE COMPLETE 	normal release of transaction initiated in step 
				5. See iii).
	11	SS -> MS	CHANNEL RELEASE	release of the main signalling link.

Specific message contents

i) REGISTER message

see paragraph  31.9.2.3

ii) FACILITY message

see paragraph  31.9.2.3

iii) RELEASE COMPLETE message

Contents					Value/remark

Protocol Discriminator				set to "Supplementary service (call independent)"

Transaction identifier				the transaction identifier value is the same as REGISTER 
								transaction value but the transaction flag is different.

Release complete message type	As GSM 4.80

Facility Information Element		for step  9 see iiib).													for step10 see iiia).

iiia)	Facility Information Element with Return Result = UnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.4.

Contents					Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Return Result from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the UnstructuredSS-Request

iiib)	Facility Information Element with Return Error = UnstructuredSS-Request component type as defined in GSM 4.80 section 3.6.1 table 3.5.

Contents					Value/remark

Facility IE identifier				As GSM 04.08

Length of FIE contents

Component type tag				Return Error from GSM 04.80

Component length				depending on the length of ussd-string

InvokeID tag						From 04.80

Invoke ID length					1

Invoke ID						The same as the invoke of the UnstructuredSS-Request

Error Code tag					As GSM 04.80

Error Code length

Error Code						USSD Busy

## 31.10	MMI input for USSD

31.10.1	Conformance requirements

If the MS cannot interpret the MMI input as a defined GSM Supplementary Services, SIM control procedure or MS manufacturer-defined procedure, and if the MMI input is in the form:

"entry of any characters defined in the GSM 03.38 Default Alphabet (up to the maximum defined in GSM 04.80) followed by #SEND"

or

"entry of 1 or 2 characters defined in the GSM 03.38 Default Alphabet followed by SEND"

then it shall be interpreted by the MS as Unstructured SS Data and sent transparently towards the network, unless the MS is not engaged in a call and the first digit of the 2 character entry, followed by SEND, is a "1". In this case the MS shall transmit this as a call-setup request.

References

GSM 02.30 section 4.5.3

31.10.2	Test purpose

To check that the entry of 2 digits in the form 1X (X in the set 0,...9) followed by SEND is accepted by the mobile station in idle mode as a normal call establishment for the 1X number. It is checked that the MS sends a CHANNEL REQUEST, sends CM SERVICE REQUEST message for mobile originated call (after having received an IMMEDIATE  ASSIGNMENT), and then sends the SETUP message containing the 1X phone number as called number (after having received the CM SERVICE ACCEPT message).

31.10.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station

The MS is "idle updated";

Related PICS/PIXIT statement(s)

Foreseen final state of the MS :

The MS is "idle updated

Test procedure

The user requests call establishment successively for every 2 digit phone number of the 1X form (X in the set 0,...9) allowing ten seconds between each attempts.

Maximum duration of test

3 min

Expected Sequence

The following sequence is executed for execution counter k = 1 to 10.

Step	Direction	Message	Comments

1	MS		The MS is made to initiate call establishment
				for phone number 1X (where X = k-1)
	2	MS -> SS	CHANNEL REQUEST	with establishment cause
				related to mobile originating call
	3	SS -> MS	IMM. ASSIGN
	4	MS -> SS	CM SERVICE REQUEST 	"mobile originating call"
	5	SS -> MS	CM SERVICE ACCEPT
	6	MS -> SS	SETUP	called BCD number is 1X
	7	SS -> MS	RELEASE COMPLETE
	8			Wait for 10 seconds

Specific message contents

None

## 31.11	Specific message contents and ASN.1 codings

Introduction

In this section a mixed form of ASN.1 coding has been used in ASN.1 components within the messages.

Some components use the indefinite form of coding, and some use the short definite form.

An example of a FACILITY message using indefinite form of coding is described below .

The same message using short definite form of coding is described in Test 31.2.1.1 Step A8 of this section.

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 34 | 22 |
| Component type tag | Return Result | A2 |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence tag |  | 30 |
| Sequence length | indefinite | 80 |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | RegisterSS | 0A |
| SS-Information |  |  |
| Forwarding Info tag | Tag=A0 | A0 |
| Forwarding Info length | indefinite | 80 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFNRy | 2A |
| Forwarding Feature List | Seq. | 30 |
| Length Indicator | indefinite | 80 |
| Forwarding Feature | Seq. | 30 |
| Forwarding Feature length | indefinite | 80 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | AllSpeechTransmission | 10 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Prov., Registered, Active | 07 |
| Forwarded To Number identifier | Tag=85 | 85 |
| ISDN-AddressString length | 4 | 04 |
| AddressString type | International Number | 91 |
| AddressString | TBCD-String : 431234 | 34 21 43 |
| NoReplyConditionTime | Tag=87 | 87 |
| Length Indicator | 1 | 01 |
| NoReplyConditionTime | 5 | 05 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.2.1.1.1. Registration accepted

MMI sequence : **61*00431234*11*5#

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 26 | 20 |
| Component type tag | Invoke | A1 (1) |
| Component length | 24 | 18 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | RegisterSS | 0A |
| RegisterSS-Arg | Seq. | 30 (1) |
| RegisterSS-Arg length | 15 | 1F |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFNRy | 2A |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | AllSpeechTransmission | 10 |
| Forwarded To Number identifier | Tag=84 | 84 |
| AddressString length | 4 | 04 |
| AddressString type | International Number | 91 |
| AddressString | TBCD-String : 431234 | 43 21 43 |
| NoReplyConditionTime tag | Tag=85 | 85 |
| NoReplyConditionTime length | 1 | 01 |
| NoReplyConditionTime | 5 | 05 |

Note 1 : This component may use the indefinite form.

Step A8 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 34 | 22 |
| Component type tag | Return Result | A2 (1) |
| Component length | 32 | 20 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence tag |  | 30 (1) |
| Sequence length | 27 | 1B |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | RegisterSS | 0A |
| SS-Information |  |  |
| Forwarding Info tag | Tag=A0 | A0 (1) |
| Forwarding Info length | 22 | 16 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFNRy | 2A |
| Forwarding Feature List | Seq. | 30 (1) |
| Length Indicator | 17 | 11 |
| Forwarding Feature | Seq. | 30 (1) |
| Forwarding Feature length | 15 | 0F |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | AllSpeechTransmission | 10 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Prov., Registered, Active | 07 |
| Forwarded To Number identifier | Tag=85 | 85 |
| ISDN-AddressString length | 4 | 04 |
| AddressString type | International Number | 91 |
| AddressString | TBCD-String : 431234 | 34 21 43 |
| NoReplyConditionTime | Tag=87 | 87 |
| Length Indicator | 1 | 01 |
| NoReplyConditionTime | 5 | 05 |

MMI sequence : **21*00431234*13#

Step A16/B17 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 22 | 16 |
| Component type tag | Invoke | A1 (1) |
| Component length | 20 | 14 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | RegisterSS | 0A |
| RegisterSS-Arg | Seq. | 30 (1) |
| RegisterSS-Arg length | 12 | 0c |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFU | 21 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | All Facsimile Services | 60 |
| Forwarded To Number identifier | Tag=84 | 84 |
| AddressString length | 4 | 04 |
| AddressString type | International Number | 91 |
| AddressString | TBCD-String : 431234 | 34 21 43 |

Step A18 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 41 | 29 |
| Component type tag | Return Result | A2 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence tag |  | 30 (1) |
| Sequence length | indefinite | 80 |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | RegisterSS | 0A |
| SS-Information |  |  |
| Forwarding Info tag | Tag=A0 | A0 (1) |
| Forwarding Info length | indefinite | 80 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFU | 21 |
| Forwarding Feature List | Seq. | 30 (1) |
| Length indicator | indefinite | 80 |
| Forwarding Feature | Seq. | 30 (1) |
| Forwarding Feature length | indefinite | 80 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | All Facsimile Services | 60 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Prov., Registered, Active | 07 |
| Forwarded To Number identifier | Tag=85 | 85 |
| ISDN-AddressString length | 4 | 04 |
| AddressString type | International Number | 91 |
| AddressString | TBCD-String : 431234 | 34 21 43 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.2.1.1.2. Registration rejected

MMI sequence : **67*00431234*21#

Step A4/B5 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 22 | 16 |
| Component type tag | Invoke | A1 (1) |
| Component length | 20 | 14 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | RegisterSS | 0A |
| RegisterSS-Arg | Seq. | 30 (1) |
| RegisterSS-Arg length | 11 | 0B |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFB | 29 |
| Basic Service Code identifier | BearerserviceCode | 82 |
| Bearerservice length | 1 | 01 |
| Bearerservice code | AllAsynchronousServices | 60 |
| Forwarded To Number identifier | Tag=84 | 84 |
| AddressString length | 4 | 04 |
| AddressString type | International Number | 91 |
| AddressString | TBCD-String : 431234 | 34 21 43 |

Step  6   : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 8 | 08 |
| Component type tag | Return Error | A3 (1) |
| Component length | 6 | 06 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Error Code tag |  | 02 |
| Error Code length | 1 | 01 |
| Error Code | BearerServiceNot Provisioned | 0A |

MMI sequence : **002*00431234*13#

Step A12/B13 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 21 | 15 |
| Component type tag | Invoke | A1 (1) |
| Component length | 19 | 13 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | RegisterSS | 0A |
| RegisterSS-Arg |  | 30 (1) |
| RegisterSS-Arg length | 11 | 0B |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CF | 20 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | All Facsimile Services | 60 |
| Forwarded To Number identifier | Tag=84 | 84 |
| AddressString length | 3 | 06 |
| AddressString type | International Number | 91 |
| AddressString | TBCD-String : 1234 | 34 21 43 |

Step 14 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 8 | 08 |
| Component type tag | Reject | A4 (1) |
| Component length | 6 | 06 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Invoke Problem tag | Tag=81 | 81 |
| Invoke Problem length | 1 | 01 |
| Invoke Problem code | Resource limitation | 03 |

Test 31.2.1.2.1. Erasure accepted

MMI sequence : ##004*13#

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Invoke | A1 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | EraseSS | 0B |
| SS-ForBS | Seq. | 30 (1) |
| SS-ForBS length | 6 | 06 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFC | 28 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | All Facsimile Services | 60 |

Step 8 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 31 | 1F |
| Component type tag | Return Result | A2 (1) |
| Component length | 29 | 1D |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence tag |  | 30 (1) |
| Sequence length | 24 | 18 |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | EraseSS | 0B |
| SS-Information |  |  |
| Forwarding Info tag | Tag=A0 | A0 (1) |
| Forwarding Info length | indefinite | 80 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFC | 28 |
| Forwarding Feature List | Seq. | 30 (1) |
| Length Indicator | indefinite | 80 |
| Forwarding Feature | Seq. | 30 (1) |
| Forwarding Feature length | indefinite | 80 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | All Facsimile Services | 60 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Provisioned | 04 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

MMI sequence : ##62#

Step A16/B17 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0D |
| Component type tag | Invoke | A1 (1) |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | EraseSS | 0B |
| SS-ForBS | Seq. | 30 (1) |
| SS-ForBS length | 3 | 03 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFNRc | 2B |

Step 18 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 22 | 16 |
| Component type tag | Return Result | A2 (1) |
| Component length | 18 | 14 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence tag |  | 30 (1) |
| Sequence length | 15 | 0F |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | EraseSS | 0B |
| SS-Information |  |  |
| Forwarding Info tag | Tag=A0 | A0 (1) |
| Forwarding Info length | 10 | 0A |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFNRc | 2B |
| Forwarding Feature List | Seq. | 30 (1) |
| Length Indicator | 5 | 05 |
| Forwarding Feature | Seq. | 30 (1) |
| Forwarding Feature length | 3 | 03 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Provisioned | 04 |

Test 31.2.1.2.2. Erasure rejected

MMI sequence : ##21*11#

Step A4/B5 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Invoke | A1 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | EraseSS | 0B |
| SS-ForBS |  | 30 (1) |
| SS-ForBS length | 6 | 06 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFU | 21 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | AllSpeechTransmission | 10 |

Step  6   : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 8 | 08 |
| Component type tag | Return Error | A3 (1) |
| Component length | 6 | 06 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Error Code tag | Tag=2 | 02 |
| Error Code length | 1 | 01 |
| Error Code | TeleserviceNotProvisioned | 0B |

MMI sequence : ##61*13#

Step A12/B13 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Invoke | A1 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | EraseSS | 0B |
| SS-ForBS |  | 30 (1) |
| SS-ForBS length | 6 | 06 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFNRy | 2A |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | All Facsimile Services | 60 |

Step 14: SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 10 | 0A |
| Component type tag | Reject | A4 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Invoke Problem tag |  | 81 |
| Invoke Problem length | 1 | 01 |
| Invoke Problem | Resource limitation | 03 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.2.1.3. Activation

MMI sequence : *002*22#

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Invoke | A1 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | ActivateSS | 0C |
| SS-ForBS |  | 30 (1) |
| SS-ForBS length | 6 | 06 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CF | 20 |
| Basic Service Code identifier | BearerserviceCode | 82 |
| Bearerservice length | 1 | 01 |
| Bearerservice code | AllSynchronousServices | 68 |

Step A8 :SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 29 | 1D |
| Component type tag | Return Result | A2 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence tag |  | 30 (1) |
| Sequence length | 20 | 14 |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | ActivateSS | 0C |
| SS-Information |  |  |
| Forwarding Info tag | Tag=A0 | A0 (1) |
| Forwarding Info length | indefinite | 80 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CF | 20 |
| Forwarding Feature List | Seq. | 30 (1) |
| Length Indicator | 8 | 08 |
| Forwarding Feature | Seq. | 30 (1) |
| Forwarding Feature length | 6 | 06 |
| Basic Service Code identifier | BearerserviceCode | 82 |
| Bearerservice length | 1 | 01 |
| Bearerservice code | AllSynchronousServices | 68 |
| SS-Status | Tag=4 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Prov., Registered, Active | 07 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

MMI sequence : *21#

Step A16/B17 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0D |
| Component type tag | Invoke | A1 (1) |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | ActivateSS | 0C |
| SS-ForBS | Seq. | 30 (1) |
| SS-ForBS length | 3 | 06 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFU | 21 |

Step 18 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 26 | 1A |
| Component type tag | Return Result | A2 (1) |
| Component length | 24 | 18 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence tag |  | 30 (1) |
| Sequence length | indefinite | 80 |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | ActivateSS | 0C |
| SS-Information |  |  |
| Forwarding Info tag | Tag=A0 | A0 (1) |
| Forwarding Info length | indefinite | 80 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFU | 21 |
| Forwarding Feature List | Seq. | 30 (1) |
| Length Indicator | 5 | 05 |
| Forwarding Feature | Seq. | 30 (1) |
| Forwarding Feature length | 3 | 03 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Prov. Registered, Active | 07 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.2.1.4. Deactivation

MMI sequence : #004*11#

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Invoke | A1 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | DeactivateSS | 0D |
| SS-ForBS | Seq. | 30 (1) |
| SS-ForBS length | 6 | 06 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFC | 28 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | AllSpeechTransmission | 10 |

Step  8 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 29 | 1D |
| Component type tag | Return Result | A2 (1) |
| Component length | 27 | 1B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence tag |  | 30 (1) |
| Sequence length | 22 | 16 |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | DeactivateSS | 0D |
| SS-Information |  |  |
| Forwarding Info tag | Tag=A0 | A0 (1) |
| Forwarding Info length | indefinite | 80 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFC | 28 |
| Forwarding Feature List | Seq. | 30 (1) |
| Length Indicator | indefinite | 80 |
| Forwarding Feature | Seq. | 30 (1) |
| Forwarding Feature length | 6 | 06 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | AllSpeechTransmission | 10 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Provisioned, Registered | 06 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

MMI sequence : #62*13#

Step A16/B17 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Same header as Step 6 |  |  |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFNRc | 2B |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | All Facsimile Services | 60 |

Step 18 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 27 | 1B |
| Component type tag | Return Result | A2 (1) |
| Component length | 25 | 19 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence tag |  | 30 (1) |
| Sequence length | 20 | 14 |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | DeactivateSS | 0D |
| SS-Information |  |  |
| Forwarding Info tag | Tag=A0 | A0 (1) |
| Forwarding Info length | 15 | 0F |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFNRc | 2B |
| Forwarding Feature List | Seq. | 30 (1) |
| Length Indicator | 10 | 0A |
| Forwarding Feature | Seq. | 30 (1) |
| Forwarding Feature length | indefinite | 80 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | All Facsimile Services | 60 |
| SS-Status | Tag=4 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Provisioned, Registered | 06 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.2.1.6.1. Interrogation accepted

MMI sequence : *#67#

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0E |
| Component type tag | Invoke | A1 (1) |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | InterrogateSS | 0E |
| SS-ForBS |  | 30 (1) |
| SS-ForBS length | 3 | 03 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFB | 29 |

Step 8 : SS -> MS FACILITY

| Contents | Value/Remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0D |
| Component type tag | Return Result | A2 (1) |
| Component length | 11 | 0B |
| Invoke ID tag | Tag=02 | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | 03 |
| Sequence tag |  | 30 (1) |
| Sequence length | 6 | 06 |
| Operation code tag | Tag=02 | 02 |
| Operation code length | 1 | 01 |
| Operation code | InterrogateSS | 0E |
| InterrogateSS-Res | Choice |  |
| SS-Status | Tag=80 | 80 |
| SS-Status length | 1 | 01 |
| SS-Status | Provisioned | 04 |

MMI sequence : *#61*11#

Step A16/B17 : MS -> SS REGISTER/FACILITY

| Contents | Value/Remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Invoke | A1 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | InterrogateSS | 0E |
| SS-ForBS | Seq | 30 (1) |
| SS-ForBS length | 6 | 06 |
| SS code tag | Tag=4 | 04 |
| SS code length | 1 | 01 |
| SS code | CFNRy | 28 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | AllSpeechTransmission | 10 |

Step 18 : SS -> MS RELEASE COMPLETE

| Contents | Value/Remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 26 | 20 |
| Component type tag | Return Result | A2 (1) |
| Component length | 24 | 18 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | 03 |
| Sequence tag |  | 30 (1) |
| Sequence length | 19 | 13 |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | InterrogateSS | 0E |
| InterrogateSS-Res | Choice |  |
| Forwarding Feature List | Seq. | A3 (1) |
| length indicator | 14 | 0E |
| Forwarding Feature tag | Seq. | 30 (1) |
| Forwarding Feature length | 12 | 0C |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | AllSpeechTransmission | 10 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Prov. Registered Active | 07 |
| Forwarded To Number Identifier | Tag=85 | 85 |
| ISDN-AddressString length | 4 | 03 |
| AddressString type | International Number | 91 |
| AddressString | TBCD-String : 431234 | 34 21 43 |

Test 31.2.1.6.2. Interrogation rejected

MMI sequence : *#62#

Step A4/B5 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0D |
| Component type tag | Invoke | A1 (1) |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | InterrogateSS | 0E |
| SS-ForBS |  | 30 (1) |
| SS-ForBS length | 3 | 03 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | CFNRc | 2B |

Step  6   : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 8 | 08 |
| Component type tag | Return Error | A3 (1) |
| Component length | 6 | 06 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Error Code tag |  | 02 |
| Error Code length | 1 | 01 |
| Error Code | SS-NotAvailable | 12 |

MMI sequence : *#67*13#

Step A12/B13 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Invoke | A1 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | InterrogateSS | 0E |
| SS-ForBS | Seq | 30 (1) |
| SS-ForBS length | 6 | 06 |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | CFB: CF on MS Busy | 29 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | AllFacsimileServices | 60 |

Step 14 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 8 | 08 |
| Component type tag | Reject | A4 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Problem code tag | Tag=81 | 81 |
| Problem code length | 1 | 01 |
| Invoke problem | Resource limitation | 03 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.2.1.7.1.1. Notification during an incoming call

Step 10 and 15 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 18 | 12 |
| Component type tag | Invoke | A1 (1) |
| Component length | 16 | 10 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | NotifySS | 10 |
| SS-ForBS |  | 30 (1) |
| SS-ForBS length | indefinite | 80 |
| SS-Code tag | Tag=81 | 81 |
| SS-Code length | 1 | 01 |
| SS-Code | CFB | 29 |
| SS-Notification tag | Tag=85 | 85 |
| SS-Notification length | 1 | 01 |
| SS-Notification | Incoming call forwarded | 02 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.2.1.7.1.2. Notification during an outgoing call

Step  9  : SS -> MS ALERTING

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 18 | 12 |
| Component type tag | Invoke | A1 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | 00 |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | NotifySS | 10 |
| NotifySS-Arg tag | Seq. | 30 (1) |
| NotifyBS-Arg length | 6 | 06 |
| SS code tag | Tag=81 | 81 |
| SS code length | 1 | 01 |
| SS code | CFU: Forw Unconditional | 21 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Prov. Registered Active | 07 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Step 10  : SS -> MS CONNECT

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 18 | 12 |
| Component type tag | Invoke | A1 (1) |
| Component length | 16 | 10 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | 01 |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | NotifySS | 10 |
| NotifySS-Arg tag | Seq. | 30 (1) |
| NotifyBS-Arg length | indefinite | 80 |
| SS code tag | Tag=81 | 81 |
| SS code length | 1 | 01 |
| SS code | CFC: Conditional Forw. | 28 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Prov. Registered Active | 07 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.2.1.7.2. Forwarded-to mobile subscriber side

Step  5  : SS -> MS SETUP

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 18 | 12 |
| Component type tag | Invoke | A1 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | 00 |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | NotifySS | 10 |
| NotifySS-Arg tag | Seq. | 30 (1) |
| NotifyBS-Arg length | 6 | 06 |
| SS code tag | Tag=81 | 81 |
| SS code length | 1 | 01 |
| SS code | CFNRc MS not reachable | 2B |
| SS-Notification tag | Tag=85 | 85 |
| SS-Notification length | 1 | 01 |
| SS-Notification | Forwarded call | 01 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.6.1.1. AOC time related charging/MS originated call

k=1 Step 11 : SS -> MS CONNECT

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 45 | 2D |
| Component type tag | Invoke | A1 (1) |
| Component length | 43 | 2B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ForwardChargeAdvice | 7D |
| ForwardChargeAdviceArg | Seq. | 30 (1) |
| ForwardChargeAdviceArg length | indefinite | 80 |
| SS code tag | Tag=80 | 80 |
| SS code length | 1 | 01 |
| SS code | AoC-Charging | 72 |
| Charging Information | Tag=a1 | A1 (1) |
| Charging Information length | 28 | 1C |
| e1 tag | Tag=81 | 81 |
| e1 length | 2 | 02 |
| e1 | Value = 6.0 | 00 3C |
| e2 tag | Tag=82 | 82 |
| e2 length | 2 | 02 |
| e2 | Value = 14.0 | 00 8C |
| e3 tag | Tag=83 | 83 |
| e3 length | 2 | 02 |
| e3 | Value = 1.0 | 00 64 |
| e4 tag | Tag=84 | 84 |
| e4 length | 2 | 02 |
| e4 | Value = 25.0 | 00 FA |
| e5 tag | Tag=85 | 85 |
| e5 length | 2 | 02 |
| e5 | Value = 0.0 | 00 00 |
| e6 tag | Tag=86 | 86 |
| e6 length | 2 | 02 |
| e6 | Value = 0.0 | 00 00 |
| e7 tag | Tag=87 | 87 |
| e7 length | 2 | 02 |
| e7 | Value = 60.0 | 02 58 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Step A13/B12 : MS -> SS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 5 | 05 |
| Component type tag | Return Result | A2 (1) |
| Component length | 3 | 03 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |

k=1...5 : e-parameters

|  | e-parameter values |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| k-value | e1 | e2 | e3 | e4 | e5 | e6 | e7 |
| 1 | 6 | 14 | 1 | 25 | 0 | 0 | 60 |
| 2 | 0 | 0 | 1 | 100 | 0 | 0 | 0 |
| 3 | 250 | 16 | 2 | 500 | 0 | 0 | 60 |
| 4 | 1 | 1 | 1 | 0 | 10 | 10 | 1 |
| 5 | 12.5 | 30 | 1 | 25 | 10 | 10 | 30 |

|  | e-parameter coding |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| k-value | e1 | e2 | e3 | e4 | e5 | e6 | e7 |
| 1 | 00 3C | 00 8C | 00 64 | 00 FA | 00 00 | 00 00 | 02 58 |
| 2 | 00 00 | 00 00 | 00 64 | 03 E8 | 00 00 | 00 00 | 00 00 |
| 3 | 09 C4 | 00 A0 | 00 C8 | 13 88 | 00 00 | 00 00 | 02 58 |
| 4 | 00 0A | 00 0A | 00 64 | 00 00 | 00 64 | 00 0A | 00 0A |
| 5 | 00 7D | 01 2C | 00 64 | 00 FA | 00 64 | 00 0A | 01 2C |

Test 31.6.1.2. AOC time related charging/MS terminated call

k=1...5 Step 12 : SS -> MS FACILITY e-parameters

|  | e-parameter values |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| k-value | e1 | e2 | e3 | e4 | e5 | e6 | e7 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 1 | 100 | 0 | 0 | 0 |
| 3 | 6 | 14 | 1 | 25 | 0 | 0 | 60 |
| 4 | 1 | 1 | 1 | 0 | 0 | 0 | 1 |
| 5 | 12.5 | 30 | 1 | 25 | 0 | 0 | 30 |

|  | e-parameter coding |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| k-value | e1 | e2 | e3 | e4 | e5 | e6 | e7 |
| 1 | 00 00 | 00 00 | 00 00 | 00 00 | 00 00 | 00 00 | 00 00 |
| 2 | 00 00 | 00 00 | 00 64 | 03 E8 | 00 00 | 00 00 | 00 00 |
| 3 | 00 3C | 00 8C | 00 64 | 00 FA | 00 00 | 00 00 | 02 58 |
| 4 | 00 0A | 00 0A | 00 64 | 00 00 | 00 00 | 00 00 | 00 0A |
| 5 | 00 7D | 01 2C | 00 64 | 00 FA | 00 00 | 00 00 | 01 2C |

Test 31.6.1.5. Change in charging information during a call

Step A12 : SS -> MS FACILITY (initial CAI message)

| Contents | Value/remark | Coding |
| --- | --- | --- |
| CAI header element |  |  |
| e1 tag | Tag=81 | 81 |
| e1 length | 2 | 02 |
| e1 | Value = 10.0 | 00 64 |
| e2 tag | Tag=82 | 82 |
| e2 length | 2 | 02 |
| e2 | Value = 28.0 | 01 18 |
| e3 tag | Tag=83 | 83 |
| e3 length | 2 | 02 |
| e3 | Value = 1.0 | 00 64 |
| e4 tag | Tag=84 | 84 |
| e4 length | 2 | 02 |
| e4 | Value = 10.0 | 00 64 |
| e5 tag | Tag=85 | 85 |
| e5 length | 2 | 02 |
| e5 | Value = 0.0 | 00 00 |
| e6 tag | Tag=86 | 86 |
| e6 length | 2 | 02 |
| e6 | Value = 0.0 | 00 00 |
| e7 tag | Tag=87 | 87 |
| e7 length | 2 | 02 |
| e7 | Value = 60.0 | 02 58 |

Step A12 : SS -> MS FACILITY (subsequent CAI message)

| Contents | Value/remark | Coding |
| --- | --- | --- |
| CAI header element |  |  |
| e1 | Value = 10.0 | 00 64 |
| e2 | Value = 14.0 | 00 8C |
| e3 | Value = 1.0 | 00 64 |
| e4 | Value = 5.0 | 00 32 |
| e5 | Value = 0.0 | 00 00 |
| e6 | Value = 0.0 | 00 00 |
| e7 | Value = 60.0 | 02 58 |

Test 31.6.1.6. Different formats of charging information

k=1 SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| CAI header element |  |  |
| e1 | Value = 10.0 | 00 64 |
| e2 | Value = 40.0 | 01 90 |
| e3 | Value = 1.0 | 00 64 |
| e4 | Value = 0.0 | 00 00 |
| e5 | Value = 0.0 | 00 00 |
| e6 | Value = 0.0 | 00 00 |
| e7 | Value = 0.0 | 00 00 |

k=2 SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 29 | 1D |
| Component type tag | Invoke | A1 (1) |
| Component length | 27 | 1B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | forwardChargeAdvice | 7D |
| ForwardChargeAdviceArg | Seq. | 30 (1) |
| ForwardChargeAdviceArg length | 19 | 13 |
| SS code tag | Tag=80 | 80 |
| SS code length | 1 | 01 |
| SS code | AoC-Charging | 72 |
| Charging Information | Tag=a1 | A1 (1) |
| Charging Information length | indefinite | 80 |
| e1 tag | Tag=81 | 81 |
| e1 length | 2 | 02 |
| e1 | Value = 10.0 | 00 64 |
| e2 tag | Tag=82 | 82 |
| e2 length | 2 | 02 |
| e2 | Value = 40.0 | 01 90 |
| e3 tag | Tag=83 | 83 |
| e3 length | 2 | 02 |
| e3 | Value = 1.0 | 00 64 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.6.1.7. AOC on a Call Hold call

Step 11 : SS -> MS CONNECT

| Contents | Value/remark | Coding |
| --- | --- | --- |
| CAI header element |  |  |
| e1 | Value = 7.0 | 00 46 |
| e2 | Value = 40.0 | 01 90 |
| e3 | Value = 1.0 | 00 64 |
| e4 | Value = 0.0 | 00 00 |
| e5 | Value = 0.0 | 00 00 |
| e6 | Value = 0.0 | 00 00 |
| e7 | Value = 0.0 | 00 00 |

Step 20 : SS -> MS CONNECT

| Contents | Value/remark | Coding |
| --- | --- | --- |
| CAI header element |  |  |
| e1 | Value = 13.0 | 00 82 |
| e2 | Value = 40.0 | 01 90 |
| e3 | Value = 1.0 | 00 64 |
| e4 | Value = 0.0 | 00 00 |
| e5 | Value = 0.0 | 00 00 |
| e6 | Value = 0.0 | 00 00 |
| e7 | Value = 0.0 | 00 00 |

Test 31.6.1.8. AOC on a Multi-party call

Step 11 & 20 : SS -> MS CONNECT

|  | e-parameter values |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| k-value | e1 | e2 | e3 | e4 | e5 | e6 | e7 |
| 1 | 19 | 40 | 1 | 0 | 0 | 0 | 0 |
| 2 | 29 | 40 | 1 | 0 | 0 | 0 | 0 |

|  | e-parameter coding |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| k-value | e1 | e2 | e3 | e4 | e5 | e6 | e7 |
| 1 | 00 BE | 01 90 | 00 64 | 00 00 | 00 00 | 00 00 | 00 00 |
| 2 | 01 22 | 01 90 | 00 64 | 00 00 | 00 00 | 00 00 | 00 00 |

Step 23 : MS -> SS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 8 | 08 |
| Component type tag | Invoke | A1 (1) |
| Component length | 6 | 06 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | BuildMPTY | 7C |

Step 24 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 7 | 07 |
| Component type tag | Return Result | A2 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.6.2. Charge Storage

31.6.2.1 Removal of SIM during an active call

and 31.6.2.2 Interruption of power supply during an active call

and 31.6.2.3 MS going out of coverage during an active AOCC call

Step A11 : SS -> MS CONNECT

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 45 | 2D |
| Component type tag | Invoke | A1 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | forwardChargeAdvice | 7D |
| ForwardChargeAdviceArg | Seq. | 30 (1) |
| ForwardChargeAdviceArg length | 33 | 21 |
| SS code tag | Tag=80 | 80 |
| SS code length | 1 | 01 |
| SS code | AoC-Charging | 72 |
| Charging Information | Tag=a1 | A1 (1) |
| Charging Information length | 28 | 1C |
| e1 tag | Tag=81 | 81 |
| e1 length | 2 | 02 |
| e1 | Value = 10.0 | 00 64 |
| e2 tag | Tag=82 | 82 |
| e2 length | 2 | 02 |
| e2 | Value = 55.0 | 02 26 |
| e3 tag | Tag=83 | 83 |
| e3 length | 2 | 02 |
| e3 | Value = 1.0 | 00 64 |
| e4 tag | Tag=84 | 84 |
| e4 length | 2 | 02 |
| e4 | Value = 10.0 | 00 64 |
| e5 tag | Tag=85 | 85 |
| e5 length | 2 | 02 |
| e5 | Value = 0.0 | 00 00 |
| e6 tag | Tag=86 | 86 |
| e6 length | 2 | 02 |
| e6 | Value = 0.0 | 00 00 |
| e7 tag | Tag=87 | 87 |
| e7 length | 2 | 02 |
| e7 | Value = 10.0 | 00 64 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.6.2.4. ACMmax operation/Mobile Originating

k=1   Step A11 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 47 | 2F |
| Component type tag | Invoke | A1 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ForwardChargeAdvice | 7D |
| ForwardChargeAdviceArg | Seq. | 30 (1) |
| ForwardChargeAdviceArg length | 33 | 21 |
| SS code tag | Tag=80 | 80 |
| SS code length | 1 | 01 |
| SS code | AoC-Charging | 72 |
| Charging Information | Tag=a1 | A1 (1) |
| Charging Information length | 28 | 1C |
| e1 tag | Tag=81 | 81 |
| e1 length | 2 | 02 |
| e1 | Value = 1.0 | 00 0A |
| e2 tag | Tag=82 | 82 |
| e2 length | 2 | 02 |
| e2 | Value = 30.0 | 01 2C |
| e3 tag | Tag=83 | 83 |
| e3 length | 2 | 02 |
| e3 | Value = 1.0 | 00 64 |
| e4 tag | Tag=84 | 84 |
| e4 length | 2 | 02 |
| e4 | Value = 0.0 | 00 00 |
| e5 tag | Tag=85 | 85 |
| e5 length | 2 | 02 |
| e5 | Value = 0.0 | 00 00 |
| e6 tag | Tag=86 | 86 |
| e6 length | 2 | 02 |
| e6 | Value = 0.0 | 00 00 |
| e7 tag | Tag=87 | 87 |
| e7 length | 2 | 02 |
| e7 | Value = 0.0 | 00 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

k=2  Step A11 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 47 | 2F |
| Component type tag | Invoke | A1 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ForwardChargeAdvice | 7D |
| ForwardChargeAdviceArg | Seq. | 30 (1) |
| ForwardChargeAdviceArg length | 33 | 21 |
| SS code tag | Tag=80 | 80 |
| SS code length | 1 | 01 |
| SS code | AoC-Charging | 72 |
| Charging Information | Tag=a1 | A1 (1) |
| Charging Information length | 28 | 1C |
| e1 tag | Tag=81 | 81 |
| e1 length | 2 | 02 |
| e1 | Value = 0.0 | 00 00 |
| e2 tag | Tag=82 | 82 |
| e2 length | 2 | 02 |
| e2 | Value = 0.0 | 00 00 |
| e3 tag | Tag=83 | 83 |
| e3 length | 2 | 02 |
| e3 | Value = 0.0 | 00 00 |
| e4 tag | Tag=84 | 84 |
| e4 length | 2 | 02 |
| e4 | Value = 0.0 | 00 00 |
| e5 tag | Tag=85 | 85 |
| e5 length | 2 | 02 |
| e5 | Value = 0.0 | 00 00 |
| e6 tag | Tag=86 | 86 |
| e6 length | 2 | 02 |
| e6 | Value = 0.0 | 00 00 |
| e7 tag | Tag=87 | 87 |
| e7 length | 2 | 02 |
| e7 | Value = 0.0 | 00 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.6.2.5 ACMmax operation/Mobile Terminating

k=1  Step A13 : SS -> MS FACILITY, k=2 Step B13 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 47 | 2F |
| Component type tag | Invoke | A1 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ForwardChargeAdvice | 7D |
| ForwardChargeAdviceArg | Seq. | 30 (1) |
| ForwardChargeAdviceArg length | 33 | 21 |
| SS code tag | Tag=80 | 80 |
| SS code length | 1 | 01 |
| SS code | AoC-Charging | 72 |
| Charging Information | Tag=a1 | A1 (1) |
| Charging Information length | 28 | 1C |
| e1 tag | Tag=81 | 81 |
| e1 length | 2 | 02 |
| e1 | Value = 1.0 | 00 0A |
| e2 tag | Tag=82 | 82 |
| e2 length | 2 | 02 |
| e2 | Value = 30.0 | 01 2C |
| e3 tag | Tag=83 | 83 |
| e3 length | 2 | 02 |
| e3 | Value = 1.0 | 00 64 |
| e4 tag | Tag=84 | 84 |
| e4 length | 2 | 02 |
| e4 | Value = 0.0 | 00 00 |
| e5 tag | Tag=85 | 85 |
| e5 length | 2 | 02 |
| e5 | Value = 0.0 | 00 00 |
| e6 tag | Tag=86 | 86 |
| e6 length | 2 | 02 |
| e6 | Value = 0.0 | 00 00 |
| e7 tag | Tag=87 | 87 |
| e7 length | 2 | 02 |
| e7 | Value = 0.0 | 00 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

k=3  Step A13 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 47 | 2F |
| Component type tag | Invoke | A1 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ForwardChargeAdvice | 7D |
| ForwardChargeAdviceArg | Seq. | 30 (1) |
| ForwardChargeAdviceArg length | 33 | 21 |
| SS code tag | Tag=80 | 80 |
| SS code length | 1 | 01 |
| SS code | AoC-Charging | 72 |
| Charging Information | Tag=a1 | A1 (1) |
| Charging Information length | 28 | 1C |
| e1 tag | Tag=81 | 81 |
| e1 length | 2 | 02 |
| e1 | Value = 0.0 | 00 00 |
| e2 tag | Tag=82 | 82 |
| e2 length | 2 | 02 |
| e2 | Value = 0.0 | 00 00 |
| e3 tag | Tag=83 | 83 |
| e3 length | 2 | 02 |
| e3 | Value = 0.0 | 00 00 |
| e4 tag | Tag=84 | 84 |
| e4 length | 2 | 02 |
| e4 | Value = 0.0 | 00 00 |
| e5 tag | Tag=85 | 85 |
| e5 length | 2 | 02 |
| e5 | Value = 0.0 | 00 00 |
| e6 tag | Tag=86 | 86 |
| e6 length | 2 | 02 |
| e6 | Value = 0.0 | 00 00 |
| e7 tag | Tag=87 | 87 |
| e7 length | 2 | 02 |
| e7 | Value = 0.0 | 00 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.8.1.1. Registration of a password accepted

MMI sequence :**03*330*1234*9876*9876#

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 11 | 0B |
| Component type tag | Invoke | A1 (1) |
| Component length | 9 | 09 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | RegisterPasswordSS | 11 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | B: All barring services | 90 |

Step 8 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 14 | 0E |
| Component type tag | Invoke | A1 (1) |
| Component length | 12 | 0C |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Linked ID tag |  | 80 |
| Linked ID length | 1 | 01 |
| Linked ID | As received | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | GetPasswordSS | 12 |
| Guidance information | Enumerated | 0A |
| Guidance length | 1 | 01 |
| Guidance | enter password | 00 |

Step 10 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Return Result | A2 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |

| Sequence tag |  | 30 (1) |
| --- | --- | --- |
| Sequence length | 9 | 09 |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | GetPasswordSS | 12 |
| Password tag | Numeric String | 12 |
| Password length | 4 | 04 |
| Password | Password = 1234 | 31 32 33 34 |

Step 11 & 13 : FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Guidance | Enter New Password | 01 |

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Password | Password = 9876 | 39 38 37 36 |

Step 14 & 16 : FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Guidance | EnterNewPasswordAgain | 02 |

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Password | Password = 9876 | 39 38 37 36 |

Step 17 : SS -> MS RELEASE COMPLETE

| Contents |  | Value/remark | Coding |
| --- | --- | --- | --- |
| Length of FIE contents |  | 20 | 14 |
| Component type tag |  | Return Result | A2 (1) |
| Component length |  | indefinite | 80 |
| Invoke ID tag |  |  | 02 |
| Invoke ID length |  | 1 | 01 |
| Invoke ID |  | As received | -- |
| Sequence tag |  |  | 30 (1) |
| Sequence length |  | indefinite | 80 |
| Operation Code tag |  |  | 02 |
| Operation Code length |  | 1 | 01 |
| Operation Code |  | RegisterPasswordSS | 11 |
| Password tag |  | Numeric String | 12 |
| Password length |  | 4 | 04 |
| Password |  | Password = 1234 | 31 32 33 34 |
|  | End-Of-Content Tag | 0 | 00 |
|  | Length Indicator | 0 | 00 |
|  | End-Of-Content Tag | 0 | 00 |
|  | Length Indicator | 0 | 00 |

Test 31.8.1.2. Registration of a password rejected

MMI sequence :**03*330*1234*9876*987X#

Test 31.8.1.2.1. Rejection after invoke of the RegisterPassword operation

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 11 | 0B |
| Component type tag | Invoke | A1 (1) |
| Component length | 9 | 09 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | RegisterPasswordSS | 11 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | B : All barring services | 90 |

Step  6   : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 8 | 08 |
| Component type tag | Return Error | A3 (1) |
| Component length | 6 | 06 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Error Code tag | Tag=2 | 02 |
| Error Code length | 1 | 01 |
| Error Code | SS subscription violation | 13 |

Test 31.8.1.2.2. Rejection after password check with negative result

Step 7 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 10 | 0A |
| Component type tag | Return Error | A3 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Error Code tag | Tag=2 | 02 |
| Error Code length | 1 | 01 |
| Error Code | Negative Password Check | 26 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.8.1.2.3. Rejection after new password mismatch

Step 15 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 11 | 0B |
| Component type tag | Return Error | A3 (1) |
| Component length | 9 | 09 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Error Code tag | Tag=2 | 02 |
| Error Code length | 1 | 01 |
| Error Code | PW-Registration Failure | 25 |
| pw-Registration Failure cause | Tag=04 | 04 (2) |
| pw-Registration Failure length | 1 | 01 |
| pw-Registration Failure | New Password Mismatch | 02 |

NOTE 2 : This element is described in Rec. 04.80 4.3.2.12, but there is no ASN.1 description in chapter 4.5. Description given in the informative annex A should be put at the end of chapter 4.5.

Test 31.8.3.1. Activation accepted

MMI sequence : *33*22#

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Invoke | A1 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary (01) | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ActivateSS | 0C |
| SS-ForBS | Seq. | 30 (1) |
| SS-ForBS length | 6 | 06 |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | BAOC | 92 |
| Basic Service Code identifier | BearerserviceCode | 82 |
| BearerService length | 1 | 01 |
| BearerService code | AllSynchronousServices | 68 |

Step  8 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 14 | 0E |
| Component type tag | Invoke | A1 (1) |
| Component length | 12 | 0C |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary (02) | -- |
| Linked ID tag |  | 80 |
| Linked ID length | 1 | 01 |
| Linked ID | As received (01) | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | Getpassword | 12 |
| Guidance tag | Enumerated | 0a |
| Guidance length | 1 | 01 |
| Guidance | Enter Password | 00 |

Step 10 : MS -> SS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Return Result | A2 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received (02) | -- |
| Sequence tag |  | 30 (1) |
| Sequence length | 9 | 09 |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | Getpassword | 12 |
| Password tag | NumericString | 12 |
| Password length | 4 | 04 |
| Password | Password = 1234 | 31 32 33 34 |

Step 11 : SS -> MS FACILITY

This message is coded to give a complete answer to the MS request. Shorter message can also be used (see last paragraph)

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 29 | 1D |
| Component type tag | Return Result | A2 (1) |
| Component length | 27 | 1B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received (01) | -- |
| Sequence tag |  | 30 (1) |
| Sequence length | indefinite | 80 |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ActivateSS | 0C |
| SS information |  |  |
| CallBarringInfo | Tag=a1 | A1 (1) |
| CallBarringInfo length | indefinite | 80 |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | BAOC | 92 |
| CallBarringFeature List | Seq. | 30 (1) |
| Length indicator | 8 | 08 |
| CallBarring Feature tag | Seq. | 30 (1) |
| CallBarring Feature length | 6 | 06 |
| Basic Service Code identifier | BearerserviceCode | 82 |
| BearerService length | 1 | 01 |
| BearerService code | AllSynchronousServices | 68 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Prov. Registered Active | 07 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

MMI sequence : *351#

Step A19/B20 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0D |
| Component type tag | Invoke | A1 (1) |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ActivateSS | 0C |
| SS-ForBS | Seq | 30 (1) |
| SS-ForBS length | 3 | 03 |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | BICRoam | 9B |

Step 24 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 26 | 1A |
| Component type tag | Return Result | A2 (1) |
| Component length | 24 | 18 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence identifier |  | 30 (1) |
| Sequence length | 19 | 13 |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ActivateSS | 0C |
| SS-Information |  |  |
| CallBarringInfo | Tag=a1 | A1 (1) |
| CallBarringInfo length | 14 | 0E |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | BICRoam | 9B |
| CallBarringFeature List | Seq. | 30 (1) |
| Length indicator | indefinite | 80 |
| CallBarring Feature tag | Seq. | 30 (1) |
| CallBarring Feature length | indefinite | 80 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Prov. Registered Active | 07 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Step 11 & 24 : short messages

Step 11 and 24 messages can be coded using this shorter form:

Message containing only the "Return result"

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 5 | 05 |
| Component type tag | Return Result | A2 (1) |
| Component length | 3 | 03 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |

Message containing the "Return result" and the "Operation Code"

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 12 | 0C |
| Component type tag | Return Result | A2 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence identifier |  | 30 (1) |
| Sequence length | 3 | 03 |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ActivateSS | 0C |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.8.3.2.1. Rejection after invoke of ActivateSS operation

MMI sequence : *331#

Step A4/B5 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0D |
| Component type tag | Invoke | A1 (1) |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ActivateSS | 0C |
| SS-ForBS | Seq | 30 (1) |
| SS-ForBS length | 3 | 03 |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | BOIC | 93 |

Step  6 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 8 | 08 |
| Component type tag | Return Error | A3 (1) |
| Component length | 6 | 06 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Error code tag |  | 02 |
| Error code length | 1 | 01 |
| Error code | SS Subscription Violation | 13 |

Test 31.8.3.2.2. Rejection after use of password procedure

MMI sequence : *35#

Step A4/B5 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0D |
| Component type tag | Invoke | A1 (1) |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | ActivateSS | 0C |
| SS-ForBS | Seq | 30 (1) |
| SS-ForBS length | 3 | 03 |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | BAIC | 9A |

Step  9 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 10 | 0A |
| Component type tag | Return Error | A3 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Error code tag |  | 02 |
| Error code length | 1 | 01 |
| Error code | NegativePasswordCheck | 26 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.8.4.1. Deactivation accepted

MMI sequence : *330*11#

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0D |
| Component type tag | Invoke | A1 (1) |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | DeactivateSS | 0D |
| SS-ForBS | Seq | 30 (1) |
| SS-ForBS length | 3 | 03 |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | B : All barring services | 90 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | AllSpeechTransmission | 10 |

Step  11  : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 20 | 14 |
| Component type tag | Return Result | A2 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence identifier |  | 30 (1) |
| Sequence length | 12 | 0C |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | DeactivateSS | 0D |
| SS-Information |  |  |
| CallBarringInfo | Tag=a1 | A1 (1) |
| CallBarringInfo length | 7 | 07 |
| CallBarringFeature List | Seq. | 30 (1) |
| Length indicator | 5 | 05 |
| CallBarring Feature tag | Seq. | 30 (1) |
| CallBarring Feature length | 3 | 03 |
| Basic Service Code Identifier | TeleserviceCode | 83 |
| Teleservice Length | 1 | 01 |
| Teleservice code | AllSpeechTransmission | 10 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

MMI sequence : #333*13#

Step A19/B20 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Invoke | A1 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | DeactivateSS | 0D |
| SS-ForBS | Seq | 30 (1) |
| SS-ForBS length | 6 | 06 |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | BO | 91 |
| Basic Service Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | AllFacsimileServices | 60 |

Step A24 : MS -> SS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 20 | 14 |
| Component type tag | Return Result | A2 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence identifier |  | 30 (1) |
| Sequence length | 12 | 0C |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | DeactivateSS | 0D |
| SS-Information |  |  |
| CallBarringInfo | Tag=a1 | A1 (1) |
| CallBarringInfo length | 7 | 07 |
| CallBarringFeature List | Seq. | 30 (1) |
| Length indicator | 5 | 05 |
| CallBarring Feature tag | Seq. | 30 (1) |
| CallBarring Feature length | 3 | 03 |
| Basic Service Code Identifier | TeleserviceCode | 83 |
| Teleservice Length | 1 | 01 |
| Teleservice code | AllFacsimileServices | 60 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.8.4.2.1. Deactivation rejected after invoke operation

MMI sequence : *#353#

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0D |
| Component type tag | Invoke | A1 (1) |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | DeactivateSS | 0D |
| SS-ForBS | Seq | 30 (1) |
| SS-ForBS length | 3 | 03 |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | BI | 99 |

Step  8 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 10 | 0A |
| Component type tag | Return Error | A3 (1) |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Error code tag |  | 02 |
| Error code length | 1 | 01 |
| Error code | SS Subscription Violation | 13 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

Test 31.8.4.2.2. Deactivation rejection after password operation

MMI sequence : *#332#

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0D |
| Component type tag | Invoke | A1 (1) |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | DeactivateSS | 0D |
| SS-ForBS | Seq | 30 (1) |
| SS-ForBS length | 3 | 03 |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | BOICExHome | 94 |

Step  8 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 8 | 08 |
| Component type tag | Return Error | A3 (1) |
| Component length | 6 | 06 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Error code tag |  | 02 |
| Error code length | 1 | 01 |
| Error code | NegativePasswordCheck | 26 |

Test 31.8.6.1. Interrogation accepted

MMI sequence : *#35#

Step A6/B7 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 13 | 0D |
| Component type tag | Invoke | A1 (1) |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | InterrogateSS | 0E |
| SS-ForBS | Seq | 30 (1) |
| SS-ForBS length | 3 | 03 |
| SS code tag | Tag=04 | 04 |
| SS code length | 1 | 01 |
| SS code | BAIC | 9A |

Step 11 : SS -> MS FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE Contents | 15 | 0F |
| Component type tag | Return Result | A2 |
| Component length | 13 | 0D |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence tag |  | 30 |
| Sequence length | 8 | 08 |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | InterrogateSS | 0E |
| InterrogateSS-Res | Choice |  |
| BasicServiceGroupList tag | Seq. | A2 |
| BasicServiceGroupList length | 3 | 03 |
| BasicService Code identifier | TeleserviceCode | 83 |
| Teleservice length | 1 | 01 |
| Teleservice code | Telephony | 11 |

MMI sequence : *#332#

Step A19/A20 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE Contents | 13 | 0D |
| Component type tag | Invoke | A1 |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | InterrogateSS | 0E |
| SS-ForBS | Seq | 30 |
| SS-ForBS length | 3 | 03 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | BOICExHome | 94 |

Step A24 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE Contents | 13 | 0D |
| Component type tag | Return Result | A2 |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Sequence tag |  | 30 |
| Sequence length | 6 | 06 |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | InterrogateSS | 0E |
| InterrogateSS-Res | Choice |  |
| SS-Status | Tag=80 | 80 |
| SS-Status length | 1 | 01 |
| SS-Status | Prov., Registered, Active | 07 |

Test 31.8.6.2. Interrogation rejected

MMI sequence : *#351#

Step A4/B5 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE Contents | 13 | 0D |
| Component type tag | Invoke | A1 |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | InterrogateSS | 0E |
| SS-ForBS | Seq | 30 |
| SS-ForBS length | 3 | 03 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | BICRoam | 9B |

Step A6 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE Contents | 8 | 08 |
| Component type tag | Return Error | A3 |
| Component length | indefinite | 80 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Error code tag |  | 02 |
| Error code length | 1 | 01 |
| Error code | SS_NotAvailable | 12 |
| End-Of-Content Tag | 0 | 00 |
| Length Indicator | 0 | 00 |

MMI sequence : *#331#

Step A12/B13 : MS -> SS REGISTER/FACILITY

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE Contents | 13 | 0D |
| Component type tag | Invoke | A1 |
| Component length | 11 | 0B |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | InterrogateSS | 0E |
| SS-ForBS | Seq | 30 |
| SS-ForBS length | 3 | 03 |
| SS-Code tag | Tag=4 | 04 |
| SS-Code length | 1 | 01 |
| SS-Code | BOIC | 93 |

Step A14 : SS -> MS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE Contents | 8 | 08 |
| Component type tag | Reject | A4 |
| Component length | 6 | 06 |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | As received | -- |
| Problem code tag | Tag=81 | 81 |
| Problem code length | 1 | 01 |
| Invoke problem code | Resource limitation | 03 |

Test 31.8.7. Normal operation

Incoming call

Step A6 : MS -> SS RELEASE COMPLETE

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 16 | 10 |
| Component type tag | Invoke | A1 (1) |
| Component length | 14 | 0E |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation code tag |  | 02 |
| Operation code length | 1 | 01 |
| Operation code | NotifySS | 10 |
| NotifySS-Arg tag | Seq. | 30 (1) |
| NotifySS-Arg length | 6 | 06 |
| SS code tag | Tag=81 | 81 |
| SS code length | 1 | 01 |
| SS code | BI | 99 |
| SS-Status | Tag=84 | 84 |
| SS-Status length | 1 | 01 |
| SS-Status code | Prov. Registered Active | 07 |

Test 31.9. Registration accepted of a USSD

MMI sequence : **00#

| Contents | Value/remark | Coding |
| --- | --- | --- |
| Length of FIE contents | 15 | 0F |
| Component type tag | Invoke | A1 (1) |
| Component length | 13 | 0D |
| Invoke ID tag |  | 02 |
| Invoke ID length | 1 | 01 |
| Invoke ID | Arbitrary | -- |
| Operation Code tag |  | 02 |
| Operation Code length | 1 | 01 |
| Operation Code | ProcessUSSDSS | 0A |
| IA5 String tag | Tag=16 | 16 |
| IA5 String length | 5 | 05 |
| Content | **00# | 2A 2A 30 30 23 |

# 32	Testing of speech transcoding functions

The test sequences for speech transcoding and DTX tests, both for input and required output, are defined in GSM 06.10, section 5, and GSM 06.32, section 4 for the full rate speech codec. For the half rate speech codec the test sequences are defined in GSM 06.20, section 5 and GSM 06.42 section 7. They are available on floppy disks in IBM/AT MS-DOS format from ETSI publications department.

The Digital Audio Interface (DAI) is described in 36.4.

NOTE 1:	For a definition of the term "traffic frame" used in this chapter, refer to GSM 06.32 and GSM 06.42.

## 32.1	Full Rate Downlink speech transcoding

32.1.1	Definition and applicability

Downlink speech transcoding transforms the 13 kbit/s net bit stream obtained by channel decoding the incoming bit stream from the air interface to 13 bit linear PCM.

The requirements and this test apply to all MS supporting TCH/FS.

32.1.2	Conformance requirement

The output bit stream from the speech transcoder shall be continuous and bit by bit exactly the same as the predefined output sequence (SEQ01.OUT, SEQ03.OUT, SEQ04.OUT and SEQ05.OUT).

GSM 06.01, 2; GSM 06.10, 5.2/5.2.2

32.1.3	Test purpose

To verify that the speech transcoding of the MS can transform all predefined sequences (SEQ01.OUT, SEQ03.OUT, SEQ04.OUT and SEQ05.OUT) at 13 kbits/s level to 104 kbit/s (13 bit linear PCM at 8 kHz) level correctly.

32.1.4	Method of test

32.1.4.1	Initial conditions

DTX is off.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech decoder/DTX functions (downlink)"

32.1.4.2	Procedure

a)	The SS resets the speech decoder of the MS via the DAI.

b)	The SS sends test sequence SEQ01.COD at 13 kbit/s to the MS via the air interface after passing it through the SS channel encoder.

NOTE 2:	These test sequence files contain 16 bit words for all speech encoded parameters and are justified as described in GSM 06.10 table 5.1. 76 words are used as input in a period of 20 ms.

c)	The SS records the 104 kbit/s output bit stream from the MS on the digital audio interface.

d)	The test is repeated using the test sequences SEQ03.COD, SEQ04.COD and SEQ05.COD.

32.1.5	Test requirements

The bit stream output shall be continuous and bit by bit exactly the same as the sequence given in the files SEQ01.OUT, SEQ03.OUT, SEQ04.OUT and SEQ05.OUT.

NOTE 3:	These files contain 16 bit words of 13 bit linear PCM left justified.

## 32.2	Full Rate Downlink receiver DTX functions

32.2.1	Definition and applicability

The DTX receiver functions consist of a SID frame detector, comfort noise generator functions and lost frame substitution and muting functions.

The requirements and this test apply to all MS supporting TCH/FS.

32.2.2	Conformance requirement

1)	The output level of the decoder has to be constant for an input signal consisting of identical speech frames.

GSM 06.10

2/3)	When, after the first lost speech frame subsequent speech frames are lost, a muting technique shall be used that will gradually decrease the output level, resulting in the silencing of the output after a maximum of 320 ms. Speech frames with the FACCH flag set provoke a Bad Frame Indication (BFI = 1) and are hence regarded as lost speech frames.

GSM 06.01, 6; GSM 06.11, 2.1/2.2/3 for requirement 2 (first part).

GSM 06.01, 6; GSM 06.11, 2.1/2.2/3; GSM 06.31, 1.2.2/3.1.1 for requirement 3 (second part).

4/5)	A valid SID-frame followed by a sequence of lost speech frames shall result in comfort noise generation with constant block amplitude parameters. Speech frames with the FACCH flag set provoke a Bad Frame Indication (BFI = 1) and are hence regarded as lost speech frames.

GSM 06.01, 3/5; GSM 06.12, 3/3.1;

GSM 06.31, 1.2.2/3.1/3.1.1/3.1.2 for requirement 4 (first part).

GSM 06.01, 3/5/6; GSM 06.11, 2.1/2.2; GSM 06.12, 3/3.1;

GSM 06.31, 1.2.2/3.1/3.1.1/3.1.2 for requirement 5 (second part).

6/7)	An invalid SID-frame followed by a sequence of lost speech frames shall result in comfort noise generation, using the set of parameters from the last valid SID-frame. Speech frames with the FACCH flag set provoke a Bad Frame Indication (BFI = 1) and are hence regarded as lost speech frames.

GSM 06.01, 3/5; GSM 06.12, 3/3.1;

GSM 06.31, 1.2.2/3.1/3.1.1/3.1.2 for requirement 6 (first part).

GSM 06.01, 3/5/6; GSM 06.11, 2.1/2.2; GSM 06.12, 3/3.1;

GSM 06.31, 1.2.2/3.1/3.1.1/3.1.2 for requirement 7 (second part).

8)	The energy of the output signal is controlled by the block amplitude parameter, xmaxc.

GSM 06.10, 3.1.20/3.1.21/3.2.1;

9/10)	The first SID-frame that is expected and not received shall be substituted by the last valid SID-frame and the procedure for valid SID-frames shall be applied. For the second lost SID-frame, a muting technique shall be used that will gradually decrease the output level, resulting in silencing the output after a maximum of 320 ms. Speech frames with the FACCH flag set provoke a Bad Frame Indication (BFI = 1) and are hence regarded as lost speech frames.

GSM 05.08, 8.3; GSM 06.01, 6; GSM 06.11, 2.3/2.4;

GSM 06.31, 1.2.2/3.1.1/3.1.2.

32.2.3	Test purpose

1)	To verify that the signal energy at the output of the decoder is constant with a tolerance of +/- 3 dB if a sequence of identical speech frames is applied at the receiver input.

2)	To verify that the muting function of the receiver is within the required limits if a sequence of lost speech frames is applied at the receiver input.

3)	To verify that the muting function of the receiver is within the required limits if a sequence of speech frames with the FACCH flag set is applied at the receiver input.

4)	To verify the function of comfort noise generation when a valid SID-frame is received followed by a sequence of lost speech frames. The signal energy at the output of the decoder shall be constant with a tolerance of +/- 3 dB.

5)	To verify the function of comfort noise generation when a valid SID-frame is received followed by a sequence of speech frames with the FACCH flag set. The signal energy at the output of the decoder shall be constant with a tolerance of +/- 3 dB.

6)	To verify the function of comfort noise generation when an invalid SID-frame is received followed by a sequence of lost speech frames. The signal energy at the output of the decoder shall be constant with a tolerance of +/- 3 dB.

7)	To verify the function of comfort noise generation when an invalid SID-frame is received followed by a sequence of speech frames with the FACCH flag set. The signal energy at the output of the decoder shall be constant with a tolerance of +/- 3 dB.

8)	To verify that the signal energy at the output of the decoder depends on the block amplitude xmaxc of the input frames if a sequence of speech frames is applied to the decoder. The signal energy at the output of the decoder shall be constant with a tolerance of +/- 3 dB.

9)	To verify the SID-frame substitution and muting functions on the comfort noise, if two consecutive expected SID-frames are lost with the other frames being lost speech frames.

10)	To verify the SID-frame substitution and muting functions on the comfort noise, if two consecutive expected SID-frames are lost with the other frames being speech frames with the FACCH flag set.

32.2.4	Method of test

32.2.4.1	Initial conditions

Uplink DTX is off.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech decoder/DTX functions (downlink)"

32.2.4.2	Procedure

a)	The SS transmits coded "speech" traffic frames on the air interface after passing them trough the SS channel encoder. They contain a special test signal at 13 kbit/s as defined below. All traffic frames are identical with the exception of some frames which are SID frames as defined in GSM 06.32.

b)	The energy of the PCM signal is evaluated (as a mean square average) at the digital audio interface of the MS at 104 kbit/s level (13 bit, 8 kHz linear PCM) and recorded for each block of 20 ms synchronized to the 20 ms speech frame structure.

c)	The SS transmission of the TDMA frames of the TCH/FS on the air interface is ramped "on" and "off" on a traffic frame by traffic frame basis, taking into account the block diagonal interleaving scheme defined in GSM 05.03. The first traffic frame in step 1 occurs one frame after the window of the SACCH multiframe (TDMA frame 60 modulo 104), allocated for the SID frame (see GSM 05.02 and 05.08). The SACCH will also be transmitted.

NOTE 4:	8 timeslots in 8 consecutive TCH/FS TDMA frames are seen as one traffic frame, and the next traffic frame starts in the middle of the previous one (i.e. after 4 TDMA frames of the previous one) due to the block diagonal interleaving scheme defined in GSM 05.03.

d)	The special test frame is an encoded "speech" traffic frame of 260 bits obtained from white Gaussian noise band limited to 300 - 3400 Hz. When repeated, the special test frame results in a humming sound with a fairly constant level when decoded, and is defined in table 1.

Table 1: Table of special test traffic frame for receiver DTX tests

Encoded parameter:	Value:

LARc(1)	38
LARc(2)	42
LARc(3)	24
LARc(4)	20
LARc(5)	10
LARc(6)	9
LARc(7)	5
LARc(8)	3

Sub-block no:

0	1	2	3
Grid position (Mc)	1	3	2	0
Block amplitude (xmaxc)	40	40	40	40
LTP gain (Bc)	0	0	0	0
LTP lag (Nc)	40	120	40	120
RPE pulses (xmc)
- pulse no 1	4	6	6	6
- pulse no 2	4	5	4	3
- pulse no 3	2	1	3	4
- pulse no 4	6	2	1	3
- pulse no 5	3	6	4	1
- pulse no 6	5	1	6	3
- pulse no 7	5	2	5	5
- pulse no 8	5	6	2	1
- pulse no 9	1	3	4	4
- pulse no 10	3	2	4	3
- pulse no 11	5	5	4	5
- pulse no 12	6	1	2	2
- pulse no 13	1	3	4	3

NOTE 5:	The signal energy of the decoded special test frame is controlled with the block amplitude parameter (xmaxc). Reducing xmaxc from 40 to 32 reduces the signal energy by 6 dB, and reducing xmaxc from 40 to 24 reduces the signal energy by 12 dB.

e)	The sequence of traffic frames on the air interface is as follows:

e.1)	23 test frames "on".

e.2)	20 frames "off".

e.3)	20 test frames "on".

e.4)	1 SID frame followed by 6 frames "off", another identical SID frame and 23 frames "off". Except for the SID codeword, the SID frames are identical to the test frame.

e.5)	1 different SID frame, however with 2 to 15 errors inserted in the SID codeword, followed by 23 frames "off".

e.6)	20 test frames "on", but with the block amplitude parameter xmaxc = 24.

e.7)	1 SID frame followed by 50 frames "off". Except for the SID codeword, the SID frames are identical to the test frame.

e.8)	The whole test is repeated, but the frames "off" are replaced by frames "on" with the FACCH flag set.

32.2.5	Test requirements

1)	In step e.1), the signal energy shall be fairly constant within +/- 3 dB.

2)	In step e.2), the signal energy shall decrease to less than -60 dBm within 17 frames.

3)	In step e.4), comfort noise shall be generated. The same requirements as in step e.1) apply.

4)	In step e.5), the same requirements as in step e.4) apply.

5)	In step e.6), the same requirements as in step e.1) apply. However, the signal energy shall be 12 dB lower.

6)	In step e.7), the signal energy shall be fairly constant within +/- 3 dB for 28 frames. Then the signal energy shall decrease to less than -60 dBm within 16 frames.

7)	In step e.8), the same requirements as in all previous steps apply.

## 32.3	Full Rate Uplink speech transcoding

32.3.1	Definition and applicability

Uplink speech transcoding transforms 13 bit linear PCM to the 13 kbit/s net bit stream. This net bit stream is to be channel encoded for transmission on the air interface.

The requirements and this test apply to all MS supporting TCH/FS.

32.3.2	Conformance requirement

The output bit stream from the speech transcoder shall be bit by bit exactly the same as the predefined output sequence (SEQ01.OUT, SEQ02.OUT, SEQ03.OUT and SEQ04.OUT).

GSM 06.01, 2; GSM 06.10, 5.2/5.2.1

32.3.3	Test purpose

To verify that the speech transcoder on the MS can transform all predefined sequences (SEQ01.INP, SEQ02.INP, SEQ03.INP and SEQ04.INP) at 104 kbit/s (13 bit linear PCM at 8 kHz) level to 13 kbit/s level correctly.

32.3.4	Method of test

32.3.4.1	Initial conditions

Uplink DTX is off.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech encoder/DTX functions (uplink)"

32.3.4.2	Procedure

a)	The SS resets the speech decoder on the MS (see 36.4)

b)	The SS sends a test sequence SEQ01.INP to the MS at 104 kbit/s level via the digital audio interface.

NOTE 6:	These files contain 16 bit words for 13 bit linear PCM left justified. See also GSM 06.10 table 5.1.

c)	The SS records the 13 kbit/s output bit stream obtained by channel decoding the incoming bit stream from the air interface.

d)	The test is repeated using the test sequences SEQ02.INP, SEQ03.INP and SEQ04.INP.

32.3.5	Test requirements

The bit stream output shall be bit by bit exactly the same as the sequence given in the files SEQ01.COD, SEQ02.COD, SEQ03.COD and SEQ04.COD.

NOTE 7:	These files contain 16 bit words of all the 76 parameters in a speech frame justified as in GSM 06.10 table 5.1. 76 codewords shall occur in a frame of 20 ms.

## 32.4	Full Rate Uplink transmitter DTX functions

32.4.1	Definition and applicability

The VAD/DTX transmitter functions consist of a Voice Activity Detector (VAD) and a surrounding Discontinuous Transmission (DTX) system introducing additional "speech" traffic frames on the air interface compared to those the VAD itself would classify as speech frames containing real speech. The additional traffic frames on the air are introduced due to:

1)	A "hangover" period at the end of speech bursts in order to be certain that the traffic frames contain only noise and to evaluate the background acoustic noise characteristics when no real speech is present.

2)	Special traffic frames (SID frames) added on the air at regular intervals containing only the evaluated background acoustic noise characteristics. These frames are used for generation of comfort noise in speaker silence periods on the receiving side.

The requirements and this test apply to all MS supporting TCH/FS.

32.4.2	Conformance requirement

The MS VAD and DTX function allows only those frames to be transmitted that are either marked with SP = 1 or that are properly positioned SID-frames.

GSM 05.08, 8.3; GSM 06.01, 3/4; GSM 06.31, 2.1/2.1.1/2.1.2; GSM 06.32, 1/2/2.1/2.2.8

32.4.3	Test purpose

To verify that the combination of VAD and DTX operates correctly.

32.4.4	Method of test

32.4.4.1	Initial conditions

A call is set up on a TCH/FS according to the generic call set-up procedure

Uplink DTX is on.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech encoder/DTX functions (uplink)"

32.4.4.2	Procedure.

a)	The SS sends a test sequence SPEC_A1.INP of PCM samples, which are grouped into frames of 20 ms synchronized to the TDMA and traffic frame structure on the air interface, on the digital audio interface in the MS at 104 kbits/s (13 bit, 8 kHz linear PCM).

The start of the test sequences is synchronized with the radio transmission on the air interface so that the first traffic frame on the air occurs just after the traffic frame allocated for the SID frame (TDMA frame 56 modulo 104, see GSM 05.02 and 05.08).

NOTE 8:	8 timeslots in 8 consecutive TCH/FS TDMA frames are seen as one traffic frame and the next traffic frame starts in the middle of the previous one (i.e. after 4 TDMA frames of the previous one) due to the block diagonal interleaving scheme defined in GSM 05.03.

b)	The SS detects whether or not there is any power transmitted over the radio path on a timeslot basis excluding SACCH frames. The speech frame by speech frame on/off transmission (on = 1) is recorded.

c)	The test is repeated for all test sequences *.INP described in GSM 06.32 section 4.

32.4.5	Test requirements

1)	In step b), the traffic frame on/off sequence recorded shall be bit exact like the sequence of SP flags stored as bit 15 of LAR(2) on the respective reference files *.COD described in GSM 06.32, with the following exceptions:

1.1)	The occurrence of a SID frame in its allowed window within the SACCH multiframe as defined in GSM 05.08.

1.2)	The occurrence of a SID frame after 1 or more real speech frames consecutively transmitted on the air.

## 32.5	Full Rate Speech channel transmission delay

32.5.1	Definition and applicability

The total transmission delay within the various elements of a GSM system are specified as round trip delays. For the MS this would be equivalent to applying an RF equivalent of a speech signal to the MS receiver, closing an acoustic path from the ERP to the MRP, detecting the corresponding RF signal at the MS transmitter output and measuring the time interval between the signal originally fed to the MS receiver and that transmitted by the MS transmitter.

This simple approach cannot be demonstrated to be accurate due to the inherent non linear characteristic of the speech transcoder. The overall delay therefore is split into four identifiable and measurable delays. The delays are respectively:

-	the downlink delay from RF input to DAI output,

-	DAI output to ERP,

-	MRP to DAI output, and

-	DAI to uplink RF output.

Each delay is defined and its method of test described in the following sections.

The requirements and this test apply to all MS supporting TCH/FS.

32.5.2	Conformance requirement

The overall speech channel transmission delay shall be less than 143.9 ms.

GSM 03.50, section 3.3.6.1

32.5.3	Test purpose

To verify that the round trip delay, of a speech channel for a MS, which consists of the sum of:

-	the downlink delay from RF input to DAI output,

-	DAI output to ERP,

-	MRP to DAI output, and

-	DAI to uplink RF output

meets the requirements when using the predefined test sequences SEQ01.COD, SEQ03.COD, SEQ04.COD and SEQ05.COD.

### 32.5.4	Downlink processing delay

32.5.4.1	Definition and applicability

The downlink processing delay is the delay from the first bit of a speech block transmitted from the RF output of the SS up to the last bit of the corresponding speech block received at the DAI on the output of the speech transcoder.

The requirements and this test apply to all MS supporting TCH/FS.

32.5.4.2	Method of test

32.5.4.2.1	Initial conditions

DTX is off.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech decoder/DTX functions (downlink)"

32.5.4.2.2	Procedure

a)	The test set up is that described in 32.1 for downlink speech transcoding.

b)	The SS transmits one of the test patterns SEQ01.COD, SEQ03.COD, SEQ04.COD or SEQ05.COD to the MS.

c)	The SS measures for each speech block it transmits the time between the first bit at the air interface and the last bit of that speech block on the DAI. This difference is the delay measured.

d)	Step c) is repeated 20 times and the maximum delay measured in ms is the downlink processing delay TDP.

NOTE 9:	This is to account for the fact that the processing time may not be constant.

### 32.5.5	Downlink coding delay

32.5.5.1	Definition and applicability

The downlink coding delay is defined as the delay between the digital representation of an acoustic signal on the DAI and the corresponding acoustic signal at the ERP.

The requirements and this test apply to all MS supporting TCH/FS.

32.5.5.2	Method of test

32.5.5.2.1	Initial conditions

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A".

The handset is mounted in the LRGP (see annex 1 of CCITT recommendation P.76) and the earpiece is sealed to the knife edge of the artificial ear conforming to CCITT recommendation P.51.

32.5.5.2.2	Procedure

a)	The SS generates on the DAI a digital representation of a sine wave with a frequency of 1000 Hz.

b)	The SS measures the "phase shift" 1, in the range of 0 to 360 degrees, between the equivalent sine wave generated at the DAI and the sine wave at the input to the artificial ear.

c)	The frequency is increased to 1100 Hz and the resulting phase shift 2 noted.

d)	The downlink coding delay TDC is calculated from either:

TDC = (2 - 1)/36 ms	for 2 > 1

or

TDC = (2 + 360 - 1)/36 ms	for 2 < 1

### 32.5.6	Uplink processing delay

32.5.6.1	Definition and applicability

The uplink processing delay is the delay from the first bit of a speech block on the DAI to the last bit of that speech block being transmitted on the air interface of the MS.

The requirements and this test apply to all MS supporting TCH/FS.

32.5.6.2	Method of test

32.5.6.2.1	Initial conditions

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech encoder/DTX functions (uplink)".

32.5.6.2.2	Procedure

a)	The test set up is that described in 32.3 for uplink speech transcoding.

b)	The SS sends one of the test patterns SEQ01.INP, SEQ03.INP, SEQ04.INP or SEQ05.INP to the DAI of the MS.

c)	The SS measures the time between the first bit on the DAI, and the last transmitted bit of the block at the air interface for each speech block the SS sends on the DAI. This time difference is the delay measured.

d)	Step c) is repeated 20 times. The maximum delay measured in ms is the uplink coding delay TUP.

NOTE 10:	This is to account for the fact that the processing time may not be constant.

### 32.5.7	Uplink coding delay

32.5.7.1	Definition and applicability

The uplink coding delay is defined as the delay between an acoustic signal at the MRP and the digital representation of that signal on the DAI.

The requirements and this test apply to all MS supporting TCH/FS.

32.5.7.2	Method of test

32.5.7.2.1	Initial conditions

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A".

The handset is mounted in the LRGP (see annex 1 of CCITT recommendation P.76) and the earpiece is sealed to the knife edge of the artificial ear conforming to CCITT recommendation P.51.

32.5.7.2.2	Procedure

a)	The SS generates an acoustic signal at the artificial mouth of the LRGP, being a pure sine wave with a frequency of 1000 Hz.

b)	The SS measures the "phase shift" 1, in the range of 0 to 360 degrees, between the signal at the MRP and its digital representation on the DAI.

c)	The SS set the generated frequency to 1100 Hz and measures the resulting phase shift 2.

d)	The uplink coding delay TUC is calculated from either:

TUC = (2 - 1)/36 ms	for 2 > 1

or

TUC = (2 + 360 - 1)/36 ms	for 2 < 1

32.5.8	Test requirement

The sum of the delays {TDP + TDC + TUP + TUC} shall be less than 144.9 ms.

NOTE 11:	This limit includes an allowance of 4*0.25 ms delay from the DAI to the MS transmission path.

NOTE 12:	No allowances have been made for any delays within the measurement system. These must either be calibrated out or subtracted from the individual delays before performing the sum above.

## 32.6	Half Rate Downlink speech transcoding

32.6.1	Definition and applicability

Downlink speech transcoding transforms the 5.6 kbit/s net bit stream obtained by channel decoding the incoming bit stream from the air interface to 104 kbit/s (13 bit linear PCM at 8 kHz) level.

The requirements and this test apply to all MS supporting TCH/HS.

32.6.2	Conformance requirement:

The output bit stream from the speech transcoder shall be continuous and bit by bit exactly the same as the predefined output sequences contained in SEQ01.OUT, SEQ02.OUT, SEQ03.OUT and SEQ04.OUT.

GSM 06.02, 5; GSM 06.20

32.6.3	Test purpose:

To verify that the speech transcoder of the MS can transform all the predefined sequences (SEQ01.DEC, SEQ02.DEC, SEQ03.DEC and SEQ04.DEC) at 5.6 kbit/s level to 104 kbit/s (13 bit linear PCM at 8 kHz) level correctly.

32.6.4	Method of test

32.6.4.1	Initial conditions

Uplink DTX is off.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech decoder/DTX functions (downlink)"

Frequency hopping is on, where the BCCH carrier is part of the hopping sequence. Frequency hopping shall be performed over four carriers using random frequency hopping. Downlink power control shall be activated and a difference of 30 dB between the level of the BCCH carrier and the other carriers adjusted.

NOTE 13:	Frequency hopping is used to ensure that the MS can cope with the reception of bursts (on the BCCH carrier) that have a power level that is different from the rest of the bursts.

32.6.4.2	Procedure

a)	The SS sends a reset pulse to the MS on the digital audio interface. This reset pulse will start the clock output of the MS at 104 kHz (pin 24 of the DAI).

b)	The SS sends test sequence SEQ01.DEC at 5.6 kbit/s to the MS via the air interface after passing it through the SS channel encoder. The speech decoder of the MS is reset by the special reset sequence which is at the beginning of the test sequence.

c)	The SS records the 104 kbit/s output bit stream from the MS on the digital audio interface. The recording shall be triggered by the reception of the encoder homing frame. The encoder homing frame itself shall not be recorded.

d)	The test is repeated using test sequences SEQ02.DEC, SEQ03.DEC and SEQ04.DEC.

32.6.5	Test requirement

The bit stream output shall be continuous and bit by bit exactly the same as the sequence describing the speech data contained in the files SEQ01.OUT, SEQ02.OUT, SEQ03.OUT and SEQ04.OUT. The two encoder homing frames at the beginning of each test sequence *.OUT shall be disregarded for this comparison.

## 32.7	Half Rate Downlink receiver DTX functions

32.7.1	Definition and applicability

The DTX receiver functions consist of a SID frame detector, comfort noise generator functions and lost frame substitution and muting functions.

The requirements and this test apply to all MS supporting TCH/HS.

32.7.2	Conformance requirement

The output bit stream from the speech transcoder shall be continuous and bit by bit exactly the same as the predefined output sequences contained in DTX*.OUT described in GSM 06.07 subclause 7.

GSM 06.02, 6/8; GSM 06.22; GSM 06.41

32.7.3	Test purpose

To verify that the MS generates comfort noise correctly.

32.7.4	Method of test

32.7.4.1	Initial conditions

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech decoder/DTX functions (downlink)"

Frequency Hopping is on, where the BCCH carrier is part of the hopping sequence. Frequency Hopping shall be done over four carriers using random Frequency Hopping.

NOTE 14:	Frequency Hopping is used to ensure that the MS can cope with the reception of dummy bursts (on the BCCH frequency) during DTX.

32.7.4.2	Procedure

a)	The SS sends a reset pulse to the MS on the digital audio interface. This reset pulse will start the clock output of the MS at 104 kHz (pin 24 of the DAI).

b)	The SS sends test sequence DTX01.DEC at 5.6 kbit/s to the MS via the air interface after passing it through the SS channel encoder. The speech decoder of the MS will be reset by the special reset sequence which is at the beginning of the test sequence.

c)	The SS transmission of the TDMA frames of the TCH/HS on the air interface is ramped "on" and "off" on a traffic frame by traffic frame basis, taking into account the block diagonal interleaving scheme defined in GSM 05.03. The first traffic frame in step b occurs one frame after the window of the SACCH multiframe (TDMA frame 0 or 52 modulo 104 for subchannel 0 and TDMA frame 1 or 53 modulo 104 for subchannel 1), allocated for the SID frame (see GSM 05.02 and 05.08). The SACCH will also be transmitted.

d)	The information whether to ramp the transmitter of the SS "on" or "off" is derived from the sequence of SP-flags contained in the file DTX01.COD (see file format description in GSM 06.07 subclause 5 for the position of the SP-flag).

e)	The SS records the 104 kbit/s output bit stream from the MS on the digital audio interface. The recording shall be triggered by the reception of the encoder homing frame. The encoder homing frame itself is not recorded.

f)	The test is repeated using test sequences *.DEC described in GSM 06.07 subclause 7.

32.7.5	Test requirement

The bit stream output shall be continuous and bit by bit exactly the same as the sequence describing the speech data contained in the files DTX*.OUT described in GSM 06.07 subclause 7. The two encoder homing frames at the beginning of each test sequence *.OUT shall be disregarded for this comparison.

## 32.8	Half Rate Uplink speech transcoding

32.8.1	Definition and applicability

Uplink speech transcoding transforms 104 kbit/s (13 bit linear PCM at 8 kHz) level to the 5.6 kbit/s net bit stream. This net bit stream is to be channel encoded for transmission on the air interface.

The requirements and this test applies to all MS supporting TCH/HS.

32.8.2	Conformance requirement

The output bit stream from the speech transcoder shall be bit by bit exactly the same as the predefined sequences contained in SEQ01.COD, SEQ02.COD and SEQ03.COD described in GSM 06.07 subclause 6.

GSM 06.02, 5; GSM 06.20

32.8.3	Test purpose

To verify that the speech transcoder of the MS can transform all the predefined sequences SEQ01.INP, SEQ02.INP and SEQ03.INP at 104 kbit/s (13 bit linear PCM at 8 kHz) level to 5.6 kbit/s level correctly.

32.8.4	Method of test

32.8.4.1	Initial conditions

Uplink DTX is off.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech encoder/DTX functions (uplink)"

Frequency hopping is on.

32.8.4.2	Procedure

a)	The SS sends a reset pulse to the MS on the digital audio interface. This reset pulse will start the clock output of the MS at 104 kHz (pin 24 of the DAI).

b)	The SS synchronizes the input of the test sequences via the digital audio interface to the framing of the MS in the uplink. This can be done in two steps as follows:

b.1)	The SS sends to the MS at 104 kbit/s level via the digital audio interface 13 triplets of input frames, each triplet consisting of 480 samples. The 480 samples of one triplet shall all be identical. The 13 bits of one sample shall all be set to "zero" except for one which is set to "one". The position of the bit within the 13 bits of a sample that is set to "one" shall vary in such a way, that all possible 13 positions are exercised within the 13 triplets of input frames. An example for such a sequence is given in test sequence BITSYNC.INP described in GSM 06.07 subclause 8. The SS records the 5.6 kbit/s output bit stream obtained by channel decoding the incoming bit stream from the air interface. As soon as the decoder homing frame is detected at the output, the framing of the MS with respect to the 13 bit long input words is known by looking at the corresponding input frame that has caused the decoder homing frame at the output.

NOTE 15:	The encoder homing frame consists of 160 identical samples, each 13 bit long left justified, with the least significant bit set to "one" and all other bits set to "zero" (0008 hex). The speech encoder will go to its predefined home state at the end of the first received encoder homing frame. Consecutive encoder homing frames will produce the decoder homing frame at the output of the speech encoder.

b.2)	Synchronized to the 13 bit framing of the MS, the SS now sends test sequence SEQSYNC.INP described in GSM 06.07 subclause 8 to the MS at 104 kbit/s level via the digital audio interface. The SS records the 5.6 kbit/s output bit stream obtained by channel decoding the incoming bit stream from the air interface. The recording shall be triggered by the reception of the decoder homing frame. By comparing the first recorded frame that is not a decoder homing frame with the 160 possible output frames contained in sequences SYNC*.COD, the offset of the input to the 20 ms framing of the MS is known.

c)	Synchronized to the 20 ms framing of the MS, the SS sends a test sequence SEQ01.INP to the MS at 104 kbit/s level via the digital audio interface. The speech encoder of the MS is reset by the special homing sequence which is at the beginning of the test sequence.

d)	The SS records the 5.6 kbit/s output bit stream obtained by channel decoding the incoming bit stream from the air interface. The recording shall be triggered by the reception of the decoder homing frame. The decoder homing frame itself is not recorded.

e)	The test is repeated using test sequences SEQ02.INP and SEQ03.INP.

32.8.5	Test requirements

The bit stream output shall be bit by bit exactly the same as the sequences describing the speech parameters contained in the files SEQ01.COD, SEQ02.COD and SEQ03.COD. The two decoder homing frames at the beginning of each test sequence *.COD shall be disregarded for this comparison.

## 32.9	Half Rate Uplink transmitter DTX functions

32.9.1	Definition and applicability

The VAD/DTX transmitter functions consist of a Voice Activity Detector (VAD) that inhibits the transmitter during speech pauses, and a surrounding Discontinuous Transmission (DTX) system introducing Silence Descriptor (SID) frames on the air interface.

The requirements and this test apply to all MS supporting TCH/HS.

32.9.2	Conformance requirement

The MS VAD and DTX function allow only those frames to be transmitted that are either properly positioned SID-frames, SACCH-frames or frames marked with SP-flag = 1.

For the transmitted frames, the output bit stream from the speech transcoder shall be bit by bit exactly the same as the predefined sequences contained in DTX*.COD described in GSM 06.07 subclause 6.

GSM 05.08, 8.3; GSM 06.02, 6/7; GSM 06.41; GSM 06.42

32.9.3	Test purpose

To verify that the combination of VAD and DTX operates correctly.

32.9.4	Method of test

32.9.4.1	Initial conditions

Uplink DTX is on.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech encoder/DTX functions (uplink)".

Frequency Hopping is on.

32.9.4.2	Procedure

a)	The SS sends a reset pulse to the MS on the digital audio interface. This reset pulse will start the clock output of the MS at 104 kHz (pin 24 of the DAI).

b)	The SS synchronizes the input of the test sequences via the digital audio interface to the framing of the MS in the uplink. This can be done in two steps as follows:

b.1)	The SS sends to the MS at 104 kbit/s level via the digital audio interface 13 triplets of input frames, each triplet consisting of 480 samples. The 480 samples of one triplet shall all be identical. The 13 bits of one sample shall all be set to "zero" except for one which is set to "one". The position of the bit within the 13 bits of a sample that is set to "one" shall vary in such a way, that all possible 13 positions are exercised within the 13 triplets of input frames. An example for such a sequence is given in test sequence BITSYNC.INP described in GSM 06.07 subclause 8. The SS records the 5.6 kbit/s output bit stream obtained by channel decoding the incoming bit stream from the air interface. As soon as the decoder homing frame is detected at the output, the framing of the MS with respect to the 13 bit long input words is known by looking at the corresponding input frame that has caused the decoder homing frame at the output.

NOTE 16:	The encoder homing frame consists of 160 identical samples, each 13 bit long left justified, with the least significant bit set to "one" and all other bits set to "zero" (0008 hex). The speech encoder will go to its predefined home state at the end of the first received encoder homing frame. Consecutive encoder homing frames will produce the decoder homing frame at the output of the speech encoder.

b.2)	Synchronized to the 13 bit framing of the MS, the SS now sends test sequence SEQSYNC.INP  described in GSM 06.07 subclause 8 to the MS at 104 kbit/s level via the digital audio interface. The SS records the 5.6 kbit/s output bit stream obtained by channel decoding the incoming bit stream from the air interface. The recording shall be triggered by the reception of the decoder homing frame. By comparing the first recorded frame that is not a decoder homing frame with the 160 possible output frames contained in sequences SYNC*.COD, the offset of the input to the 20 ms framing of the MS is known.

c)	The SS sends test sequence DTX01.INP of PCM samples  described in GSM 06.07 subclause 7 on the digital audio interface in the MS at 104 kbit/s (13 bit linear PCM at 8 kHz). The speech encoder of the MS will be reset by the special homing sequence which is at the beginning of the test sequence.

d)	The start of the test sequence is synchronized with the radio transmission on the air interface so that the first traffic frame on the air caused by the first encoder homing frame in the test sequence occurs just after the traffic frame allocated for the SID frame (TDMA frame 0 or 52 modulo 104 for subchannel 0 and TDMA frame 1 or 53 modulo 104 for subchannel 1), allocated for the SID frame (see GSM 05.02 and 05.08).

e)	The SS detects whether or not there is any power transmitted over the radio path on a time slot basis excluding SACCH frames. The speech frame by speech frame on/off transmission (on = 1) is calculated and recorded. The recording shall be triggered by the reception of the decoder homing frame. The flag marking the  decoder homing frame itself is not recorded.

f)	The SS records the 5.6 kbit/s output bit stream obtained by channel decoding the incoming bit stream from the air interface. The recording shall be triggered by the reception of the decoder homing frame. The decoder homing frame itself is not recorded.

g)	The test is repeated for all test sequences DTX*.INP described in GSM 06.07 subclause 7.

32.9.5	Test requirements

1)	The bit stream recorded in step e) shall be continuous and bit by bit exactly the same as the sequence of SP-flags contained in the files DTX*.COD (see file format description in GSM 06.07 subclause 5 for the position of the SP-flag), except for the bits marking those frames that are SID frames scheduled for transmission according to GSM 06.41. The first two frames in the reference files *.COD shall be disregarded for this comparison.

2)	The bit stream recorded in step f) shall be continuous and bit by bit exactly the same as the sequence describing the speech parameters contained in the files *.COD described in GSM 06.07 subclause 7, except for the bits of the speech frames marked with SP-flag=0. The two decoder homing frames at the beginning of each test sequence *.COD shall be disregarded for this comparison.

## 32.10	Half Rate Speech channel transmission delay

32.10.1	Definition and applicability

The total transmission delay within the various elements of a GSM system are specified as round trip delays. For the MS this would be equivalent to applying an RF equivalent of a speech signal to the MS receiver, closing an acoustic path from the ERP to the MRP, detecting the corresponding RF signal at the MS transmitter output and measuring the time interval between the signal originally fed to the MS receiver and that transmitted by the MS transmitter.

This simple approach cannot be demonstrated to be accurate due to the inherent non linear characteristic of the speech transcoder. The overall delay therefore is split into four identifiable and measurable delays. The delays are respectively:

-	the downlink delay from RF input to DAI output,

-	DAI output to ERP,

-	MRP to DAI output, and

-	DAI to uplink RF output.

Each delay is defined and its method of test described in the following sections.

The requirements and this test applies to all MS supporting TCH/HS.

32.10.2	Conformance requirement

The overall speech channel transmission delay shall be less than 143.9 ms.

GSM 03.50 section 3.3.6.2

32.10.3	Test purpose

To verify that the round trip delay of a speech channel for a MS which consists of the sum of

-	the downlink delay from RF input to DAI output,

-	DAI output to ERP,

-	MRP to DAI output, and

-	DAI to uplink RF output

meets the requirements when using the predefined test sequences SEQ01.INP and SEQ01.DEC.

### 32.10.4	Downlink processing delay

32.10.4.1	Definition and applicability

The downlink processing delay is the delay from the first bit of a speech block transmitted from the RF output of the SS up to the last bit of the corresponding speech block received at the DAI on the output of the speech transcoder.

The requirements and this test apply to all MS supporting TCH/HS.

32.10.4.2	Method of test

32.10.4.2.1	Initial conditions

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech decoder/DTX functions (downlink)"

32.10.4.2.2	Procedure

a)	The test set up is that described in section 32.6.4.2 for downlink speech transcoding.

b)	The SS transmits the test pattern SEQ01.DEC described in GSM 06.07 subclause 6 to the MS.

c)	The SS measures for each speech block it transmits the time between the first bit at the air interface and the last bit of that speech block on the DAI. This time difference is the delay measured.

d)	Step c) is repeated 20 times and the maximum delay measured in ms is the downlink processing delay TDP.

NOTE 17:	This is to account for the fact that the processing time may not be constant.

### 32.10.5	Downlink coding delay

32.10.5.1	Definition and applicability

The downlink coding delay is defined as the delay between the digital representation of an acoustic signal on the DAI and the corresponding acoustic signal at the ERP.

The requirements and this test applies to all MS supporting TCH/HS.

32.10.5.2	Method of test

32.10.5.2.1	Initial conditions

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A".

The handset is mounted in the LRGP (see annex 1 of CCITT recommendation P.76) and the earpiece is sealed to the knife edge of the artificial ear conforming to CCITT recommendation P.51.

32.10.5.2.2	Procedure

a)	The SS generates on the DAI a digital representation of a sine wave with a frequency of 1000 Hz.

b)	The SS measures the "phase shift" 1, in the range of 0 to 360 degrees, between the equivalent sine wave generated at the DAI and the sine wave at the input to the artificial ear.

c)	The frequency is increased to 1100 Hz and the resulting phase shift 2 noted.

d)	The downlink coding delay TDC is calculated from either:

TDC = (2 - 1) ms/36 	for 2 > 1

or

TDC = (2 + 360 - 1) ms/36 	for 2 < 1

### 32.10.6	Uplink processing delay

32.10.6.1	Definition and applicability

The uplink processing delay is the delay from the first bit of a speech block on the DAI to the last bit of that speech block being transmitted on the air interface of the MS.

The requirements and this test apply to all MS supporting TCH/HS.

32.10.6.2		Method of test

32.10.6.2.1	Initial conditions

The DAI of the MS is connected to the SS and is set to the operating mode "Test of speech encoder/DTX functions (uplink)"

32.10.6.2.2	Procedure

a)	The test set up is that described in section 32.8.4.2 for uplink speech transcoding.

b)	The SS sends one of the test patterns SEQ01.INP described in GSM 06.07 subclause 6 to the DAI of the MS.

c)	The SS measures the time between the first bit on the DAI, and the last transmitted bit of the block at the air interface for each speech block the SS sends on the DAI. This time difference is the delay measured.

d)	Step c) is repeated 20 times. The maximum delay measured in ms is the uplink coding delay TUP.

NOTE 18:	This is to account for the fact that the processing time may not be constant.

### 32.10.7	Uplink coding delay

32.10.7.1	 Definition and applicability

The uplink coding delay is defined as the delay between an acoustic signal at the MRP and the digital representation of that signal on the DAI.

The requirements and this test applies to all MS supporting TCH/HS.

32.10.7.2	Method of test

32.10.7.2.1	Initial conditions

The handset is mounted in the LRGP (see annex 1 of CCITT recommendation P.76) and the earpiece is sealed to the knife edge of the artificial ear conforming to CCITT recommendation P.51.

32.10.7.2.2	Procedure

a)	The SS generates an acoustic signal at the artificial mouth of the LRGP, being a pure sine wave with a frequency of 1000 Hz.

b)	The SS measures the "phase shift" 1, in the range of 0 to 360 degrees, between the signal at the MRP and its digital representation on the DAI.

c)	The SS sets the generated frequency to 1100 Hz, and measures the resulting phase shift 2.

d)	The uplink coding delay TUC is calculated from either:

TDC = (2 - 1) ms/36 	for 2 > 1

or

TDC = (2 + 360 - 1) ms/36 	for 2 < 1

32.10.8	Test requirement

The sum of the delays  TDP, TDC, TUP, and TUC shall be less than 144.9 ms.

NOTE 19:	This limit includes an allowance of 4*0.25 ms delay from the DAI to the MS transmission path.

## 32.11	Intra cell channel change from a TCH/HS to a TCH/FS

32.11.1	Definition and applicability

Dual rate MSs support an intra cell channel change from a TCH/HS to a TCH/FS by switching the Speech and channel codec used from HR to FR.

The requirements and this test apply to all MS supporting TCH/HS.

32.11.2	Conformance requirement:

1)	When commanded to perform an intra cell channel change from a TCH/HS to a TCH/FS, the MS shall switch channels from HR to FR. The maximum time allowed for the MS to perform this switch in rates is 20 ms.

GSM 05.10, section 6.8

2)	For an intra cell channel change, the time between the end of the last complete speech frame sent on the old channel and the time the MS is ready to transmit on the new channel shall be less than 20 ms.

GSM 05.10, section 6.8

32.11.3	Test purpose:

1)	To verify that the MS encodes speech correctly after performing an intra cell channel change from a TCH/HS to a TCH/FS.

2)	To verify that the MS, when commanded to perform an intracell channel change to a new ARFCN and/or timeslot number within the same cell, if the starting time is not used in the ASSIGNMENT COMMAND, is ready to transmit on the new channel within 20 ms of the last complete speech frame sent on the old channel.

32.11.4	Method of test

32.11.4.1	Initial conditions

Uplink DTX is off.

The SS sets up a call according to the generic call set up procedure on a HR channel in the low ARFCN range on timeslot 1.

32.11.4.2	Procedure

a)	The SS records the sequence of BFI flags obtained by channel decoding the incoming bit stream from the uplink air interface using the HR channel decoder on the old channel and at the same time  records the sequence of BFI flags obtained by channel decoding the incoming bit stream from the uplink air interface using the FR channel decoder on the channel to which the channel change will take place.

b)	The SS sends an ASSIGNMENT_COMMAND to the MS allocating a FR channel in the high ARFCN range on timeslot 2, and with a power command of 7. These old and new carriers have a relative frequency tolerance of 0, and a relative timing tolerance of 1/4 bit.

c)	The time at which the sequence of BFI flags at the output of the HR channel decoder performs the first transition from 0 to 1 is registered (t1).

d)	The time values at which the sequence of BFI flags at the output of the FR channel decoder performs transitions from 1 to 0 are registered. The time t2 is defined as the time where the BFI flag at the output of the FR channel decoder toggles from 1 to 0 due to a correctly received speech traffic frame received at the channel decoder. Transitions due to the occurrence of an ASSIGNMENT COMPLETE frame or an SABM frame after the reception of good speech frames shall not be considered. If the first frame sent on the new traffic channel was an SABM frame, t2 is defined as the time the BFI flag toggles from 1 to 0 due to a correctly received speech traffic frame after the reception of th SABM frame.

NOTE 20:	There shall be an allowance of at maximum two transitions for this BFI flag from 0 to 1 and back to zero again after t2. These transitions are caused by the SABM frame if it was not the first frame to be sent on the new TCH, or the ASSIGNMENT_COMPLETE frame, or both. Since both frames are FACCH frames, each would cause exactly one BFI=1 indications.

e)	The time difference Dt = t2 - t1 shall be calculated.

32.11.5	Test requirement

1)	The last transition of the BFI flag at the output of the FR channel decoder from 1 to 0 shall be followed by a sequence of at least 50 zeroes, interrupted by at maximum two transitions to 1, each interruption containing exactly one BFI=1 flag, caused by the SABM or the ASSIGNMENT_COMPLETE frames.

2)	The calculated time difference Dt shall not exceed 56.1 ms. If the first frame sent on the new channel was an SABM frame, the calculated time difference Dt shall not exceed 76.1 ms.

NOTE 21:	The BFI of the old channel will toggle from 0 to 1 only four frames after the reception of the last bit of the speech frame sent on the old channel. The requirement time of 20 ms, at which the MS shall be ready to transmit, will expire at just over 4 frames after the sending of the last bit on the old channel.  The next frame could be an IDLE frame and the MS would then transmit in the following frame. This equates to 2 frames starting from the time the BFI flag on the old channel toggles to 1, so in the worst case, including the 1/4 bit relative timing tolerance between the carriers, the MS may take 9.3 ms before starting transmissions on the new channel. The MS might need to wait up to three frames before it can start transmission of a new speech frame in its allowed starting frame. Therefore, two more frames have to be allowed (since the IDLE frame was already taken into account, only two further frames need to be allowed, meaning an extra 9.3 ms). The first frame on the TCH could belong to an SABM frame, which will provoke a BFI at the decoder output (20 ms). Additionally, 37.5 ms will be needed due to the interleaving until the last bit of the first speech frame on the new channel is received. The next frame has to be the first one of a decodable speech frame (BFI = 0).

## 32.12	Intra cell channel change from a TCH/FS to a TCH/HS

32.12.1	Definition and applicability

Dual rate MSs support an intra cell channel change from a TCH/FS to a TCH/HS by switching the Speech and channel codec used from FR to HR.

The requirements and this test apply to all MS supporting TCH/HS.

32.12.2	Conformance requirement:

1)	When commanded to perform an intra cell channel change from a TCH/FS to a TCH/HS, the MS shall switch channels from FR to HR.

GSM 05.10 section 6.8.

2)	For an intra cell channel change, the time between the end of the last complete speech frame sent on the old channel and the time the MS is ready to transmit on the new channel shall be less than 20 ms.

GSM 05.10, section 6.8

32.12.3	Test purpose:

1)	To verify that the MS encodes speech correctly after performing an intra cell channel change from a TCH/FS to a TCH/HS.

2)	To verify that the MS, when commanded to perform an intra cell channel change to a new ARFCN and/or new timeslot number within the same cell, if the starting time is not used in the ASSIGNMENT COMMAND, is ready to transmit on the new channel within 20 ms of the last complete speech frame sent on an old channel.

32.12.4	Method of test

32.12.4.1	Initial conditions

Uplink DTX is off.

The SS sets up a call according to the generic call set up procedure on a FR channel in the low ARFCN range on timeslot 1.

32.12.4.2	Procedure

a)	The SS records the sequence of BFI flags obtained by  channel decoding the incoming bit stream from the uplink air interface using the FR channel decoder on the old channel and at the same time the sequence of BFI flags obtained by channel decoding the incoming bit stream from the uplink air interface using the HR channel decoder on the channel to which the channel change will take place.

b)	The SS sends an ASSIGNMENT_COMMAND to the MS allocating a HR channel in the high ARFCN range on timeslot 2, and with a power command of 7. These old and new carriers have a relative frequency tolerance of 0, and a relative timing tolerance of 1/4 bit.

c)	The time at which the sequence of BFI flags at the output of the FR channel decoder performs the first transition from 0 to 1 is registered (t1).

d)	The time values at which the sequence of BFI flags at the output of the HR channel decoder performs transitions from 1 to 0 are registered. The time t2  is defined as the time where the BFI flag at the output of the FR channel decoder toggles from 1 to 0 due to a correctly encoded speech traffic frame received at the channel decoder. Transitions due to the occurrence of an ASSIGNMENT COMPLETE frame or and SABM frame after the reception of good speech frames shall not be considered. If the first frame sent on the new traffic channel was an SABM frame, t2 is defined as the time the BFI flag toggles from 1 to 0 due to a correctly received speech traffic frame after the reception of the SABM frame.

NOTE 22:	There shall be an allowance of at maximum two transitions for this BFI flag from 0 to 1 and back to zero again after t2. These transitions are caused by the SABM frame if it was not the first frame to be sent on the new TCH, or the ASSIGNMENT_COMPLETE frame, or both. Since both frames are FACCH frames, each would cause exactly two BFI=1 indications.

e)	The time difference Dt = t2 - t1 shall be calculated.

32.12.5	Test requirement

1)	The last transition of the BFI flag at the output of the HR channel decoder from 1 to 0 shall be followed by a sequence of at least 50 zeroes, interrupted by at maximum two transitions to 1, each interruption containing exactly two BFI=1 flags, caused by the SABM or the ASSIGNMENT_COMPLETE frames.

2)	The calculated time difference Dt shall not exceed 51.4 ms. if the first frame sent on the new channel was an SABM frame, the calculated time difference Dt shall not exceed 91.4 ms.

NOTE 23:	The BFI of the old channel will toggle from 0 to 1 only 4 frames after the reception of the last bit of the last speech frame sent on the old channel. The requirement time of 20 ms, at which the MS shall be ready to transmit, will expire at just over 4 frames after the sending of the last bit on the old channel. The next frame could belong to the other subchannel where the MS is not allowed to transmit and the MS would then transmit in the following frame. The next frame could then be an IDLE frame and the MS would then transmit in the following frame of its subchannel. This equates to 4 frames starting from the time the BFI flag on the old channel toggles to 1, so in the worst case, including the 1/4 bit relative timing tolerance between the carriers, the MS may take 18.5 ms before starting transmissions on the new channel. The first frame on the TCH could belong to an SABM frame, which will provoke 2 BFIs at the decoder output (40 ms). Additionally, 32.9 ms will be needed due to the interleaving until the last bit of the first speech frame on the new channel is received. The next frame has to be the first one of a decodable speech frame (BFI = 0).

# 33	Mobile station features

GSM 02.07 defines mandatory and optional MS features. Their presence and appropriate functioning are verified by the following tests.

## 33.1	Entry and display of called number

33.1.1	Definition and applicability

The entry and display of a called number is the ability of a MS to correctly display and signal to the network the user required number.

The requirements and this test apply to all MS.

33.1.2	Conformance requirement

1)	The number of the called subscriber is included in the Called party BCD number Information element of the SETUP message for an outgoing call.

The "Display of Called number" shall be implemented in an MS where a human interface is provided.

2)	The "Numbering plan identification" is included in the Called party BCD number Information element of the SETUP message for an outgoing call.

An MS with MMI shall as default use the Numbering Plan Identification CCITT E164, unless otherwise indicated by the user.

3)	The "Type of number" is included in the Called party BCD number Information element of the SETUP message for an outgoing call.

An MS with MMI shall, if the "+" is not entered, and a number is entered, set the Type of Number to "unknown".

4)	The "Type of number" is included in the Called party BCD number Information element of the SETUP message for an outgoing call.

An MS with MMI shall, if the "+" is entered, and a number is entered, set the Type of Number to "International"

33.1.3	Test purpose

1)	To verify that an MS with human interface, in a SETUP message sent to originate a call, includes the same “Number digits" in the "Called party BCD number" of the SETUP message as displayed.

2)	To verify that an MS with MMI, when made to establish a call sends a SETUP message, which includes the "Numbering plan identification" in the "Called party BCD number" of the SETUP message for an outgoing call with the value "ISDN/telephony numbering plan (E.164/E.163)".

3)	To verify that an MS with MMI, when made to establish a call without use of the "+-key" function, sends a SETUP message, which includes the "Type of number" in the "Called party BCD number" of the SETUP message for an outgoing call with the value "unknown".

4)	To verify that an MS with MMI, implementing the "+-key" function, when made to establish a call with use of the "+-key" function, sends a SETUP message, which includes the "Type of number" in the "Called party BCD number" of the SETUP message for an outgoing call with the value "international number.

33.1.4	Method of test

33.1.4.1	Initial conditions

The MS is registered in a cell of the SS.

33.1.4.2	Procedure

a)	A number (not including "+ function") is entered and then a call is set up.

b)	After the SS has accepted the call the number displayed on the MS and the number received in the SS are compared.

c)	The NPI and TON are examined in the SS.

d)	Steps a) to c) are repeated, but in a), the number entered starts with the "+ function"

NOTE 1:	This test may also be performed automatically using the EMMI.

33.1.5	Test requirements

1)	In step b), both numbers shall be identical

2)	In step c), the NPI shall be "E164" and the TON shall be "unknown".

3)	In step d), the NPI shall be "E164" and the TON shall be "international".

## 33.2	Indication of call progress signals

33.2.1	Definition and applicability

33.2.2	Conformance requirement

33.2.3	Test purpose

### 33.2.4	Ringing tone

33.2.4.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to j).

33.2.4.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 7).

2)	The ringing tone characteristics shall be as follows:

Tone	Frequency	Tolerance	Type

Ringing tone	425 Hz	15 Hz	Periodic
			tone on 1 s,
			silence 4 s

### 33.2.5	Busy tone

33.2.5.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to h).

b)	The SS then sends message DISCONNECT with cause number 17.

Message: DISCONNECT	(GSM 04.08, 9.3.7) to the MS:

Information element	Comment

Protocol discriminator	CM

Transaction identifier	MS orig.

Message type

Cause

- Coding standard	GSM

- Location	User

- Cause value	#17 "user busy"

33.2.5.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 5).

2)	After the reception of DISCONNECT a busy tone shall be generated. The busy tone characteristics shall be as follows:

Tone	Frequency	Tolerance	Type

Busy tone	425 Hz	15 Hz	Periodic
			tone on 500 ms,
			silence 500 ms

### 33.2.6	Congestion tone

33.2.6.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to h).

b)	The SS then sends message DISCONNECT with cause number 42.

DISCONNECT message: As in 33.2.3.1 with cause value #42 "Switching equipment congestion" (0101010).

33.2.6.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 5).

2)	After the reception of DISCONNECT a congestion tone shall be generated.

3)	The congestion tone characteristics shall be as follows:

Tone	Frequency	Tolerance	Type

Congestion tone	425 Hz	15 Hz	Periodic
			tone on 200 ms,
			silence 200 ms

### 33.2.7	Authentication failure tone

33.2.7.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to e).

b)	After reception of message AUTHENTICATION RESPONSE the SS sends message AUTHENTICATION REJECT.

Message: AUTHENTICATION REJECT	(GSM 04.08, 9.2.1) to the MS:

Information element	Comment

Protocol discriminator	MM

Transaction identifier	not relevant

Message type

33.2.7.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 4).

2)	After reception of AUTHENTICATION REJECT a tone shall be generated indicating authentication failure.

3)	The authentication failure tone is the error/special information tone with characteristics as follows:

Tone	Frequency	Tolerance	Type

Error/Special	950 Hz	50 Hz	Triple tone
Information tone	1400 Hz	50 Hz	tones on 330 ms
	1800 Hz	50 Hz	silence 1.0 s

### 33.2.8	Number unobtainable tone

33.2.8.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to h).

b)	The SS then sends message DISCONNECT with cause number 1.

DISCONNECT message: As in 33.2.3.1 with cause value #1 "Unassigned (unallocated) number" (0000001).

33.2.8.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 5).

2)	After reception of DISCONNECT a tone shall be generated indicating that the called number is unobtainable.

The number unobtainable tone is the error/special information tone with characteristics as in 33.2.5.2.

### 33.2.9	Call dropped tone

33.2.9.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to l). However, it shall be indicated in the system information messages that call re-establishment shall not be attempted (RACH control parameters).

b)	When the call has been established the SS stops transmitting on the TCH/SACCH.

33.2.9.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 8).

2)	After the radio link timeout period has expired a tone shall be generated indicating that the call has been dropped.

The call dropped tone characteristics shall be as follows:

Tone	Frequency	Tolerance	Type

Call dropped tone	425 Hz	15 Hz	Tone on 200 ms,
			silence 200 ms
			3 bursts of on/off

## 33.3	Network selection / indication

33.3.1	Definition and applicability

Network selection and indication is the ability of the MS to correctly select a network and display to the user in accordance with GSM 02.11.

The requirements and this test apply to all MS.

33.3.2	Conformance requirement

1)	Upon switching on, and when an IMSI is available, the MS shall select its Home PLMN and perform the cell selection procedure.

2)	If the Home PLMN is unavailable and the MS is in automatic PLMN selection mode, it shall attempt to select a suitable cell and accesses the PLMNs in turn, in the order of priority as stored in the SIM, when it looses radio coverage for its selected PLMN.

3)	The MS shall mark a PLMN, for which it has received a LOCATION UPDATING REJECT with the cause "PLMN not allowed", as forbidden on the SIM. The MS, when in automatic PLMN selection mode, shall not attempt to access a PLMN marked as forbidden on the SIM.

4)	No explicit conformance requirement. The requirement is implicit, because GSM 02.11 only describes selection of new PLMN for the following cases: 1) loss of radio coverage, and 2) search procedure at the user's request.

5)	If the MS loses radio coverage for its selected PLMN, and the MS is in automatic PLMN selection mode, it shall attempt to select its Home PLMN.

6)	The MS shall mark a PLMN, for which it has received a LOCATION UPDATING REJECT message with the cause "PLMN not allowed", as forbidden on the SIM. The MS, when in manual PLMN selection mode, shall be able to attempt to access a PLMN marked as forbidden on the SIM.

7)	If the Home PLMN is unavailable and the MS is in automatic PLMN selection mode, it shall attempt to select a suitable cell and accesses the PLMNs in turn, in the order of priority as stored in the SIM, upon switching on and when the IMSI is available.

8)	The MS shall delete a PLMN from the list of forbidden PLMNs, when a successful location update has been performed to that PLMN.

9)	Upon switching on, and when an IMSI is available, the MS shall select its Home PLMN and perform the cell selection procedure.

33.3.3	Test purpose

1)	To verify that the MS with SIM containing in the PLMN selector field at least one PLMN different from the Home PLMN, when in automatic PLMN selection mode, selects its Home PLMN, if available, upon switching on and when the IMSI is available. (This is verified by observation of the location updating procedure).

2)	To verify that the MS, when it looses radio coverage for its selected PLMN and in automatic PLMN selection mode, selects the PLMN with the highest priority among the PLMNs stored on the SIM, if the Home PLMN is unavailable.

3)	To verify that the MS, when in automatic PLMN selection mode with PLMN selector field containing a given PLMN, does not attempt to access that PLMN after having initiated a location updating procedure towards that network and having received a LOCATION UPDATING REJECT message with the cause "PLMN not allowed".

4)	To verify that the MS, when in automatic PLMN selection mode, does not select another PLMN if the selected PLMN is still available.

5)	To verify that the MS, when it looses radio coverage for its selected PLMN, and in automatic PLMN selection mode, selects its Home PLMN, if available.

6)	To verify that the MS, when in manual PLMN selection mode, is able to attempt an access to a PLMN for which it has previously received a LOCATION UPDATING REJECT with the cause "PLMN not allowed".

7)	To verify that the MS, upon switching on and when the IMSI is available, and in automatic PLMN selection mode, selects the PLMN with the highest priority among the PLMNs stored on the SIM, if the Home PLMN is unavailable.

8)	To verify that the MS again can perform automatic selection of a PLMN for which it has previously received a LOCATION UPDATING REJECT message with the cause "PLMN not allowed", when manual PLMN selection in the meantime has resulted in a LOCATION UPDATING ACCEPT message.

9)	To verify that the MS, when in manual PLMN selection mode, selects its Home PLMN, if available, upon switching on and when the IMSI is available.

33.3.4	Method of Test

Procedure 1: This procedure applies to both automatic and manual mode for PLMN selection.

Procedure 2: This procedure applies to the manual mode for PLMN selection.

33.3.4.1	Procedure 1

1a)	The MS is set up with a SIM which contains, in the "PLMN selector" data field, a list of 3 PLMN in the priority order PLMN2 (highest priority), PLMN3, PLMN4 (lowest priority). PLMN1 is the Home PLMN of the MS as defined in the IMSI. The "Forbidden PLMN" data field shall contain NULL values.

1b)	The SS transmits 4 BCCH carriers with the following parameters:

PLMN	Level dBVemf( )

Carrier 1	PLMN1	28

Carrier 2	PLMN2	33

Carrier 3	PLMN3	38

Carrier 4	PLMN4	43

Each carrier has the "IMSI attach" (ATT) flag set in the BCCH data. (The purpose of this is to force the MS to do location updating whenever it is switched on, so that the SS can determine which PLMN has been selected).

The other system information parameters are as in table 33-1.

1c)	The MS is brought into the "on" condition with automatic selection mode active.

1d)	The SS checks that the MS sends a "location updating request" on carrier 1.

1e)	The SS sends a "location updating accept" message to the MS on carrier 1. After 5 s, the MS "selected PLMN indicator" is checked.

1f)	The SS switches off carriers 1 and 2.

1g)	The SS checks that the MS sends a "location updating request" on carrier 3.

NOTE 2:	Steps 1h) to 1j) test that the MS writes the forbidden PLMN to the correct location in the test SIM. The procedure will be invalid if the "forbidden PLMN" data field in the SIM does not contain NULL values at the start of the procedure.

1h)	The SS sends a "location updating reject" message on carrier 3 with the cause "PLMN not allowed".

1i)	The MS is turned off. The MS is brought into the "on" condition with automatic selection mode active.

1j)	The SS checks that the MS sends a "location updating request" on carrier 4. (PLMN3 should not be selected because it should have been written to the SIM "forbidden PLMN" data field in step 1h) above.)

1k)	The SS sends a "location updating accept" message on carrier 4. After 5 s, the MS "selected PLMN indicator" is checked.

1l)	Carriers 1 and 2 are turned on with the same parameters as in step 1b) above.

1m)	The SS checks that the MS does not send a "location updating request" on either carrier 1 or 2.

1n)	Carrier 4 is turned off. The SS then checks that the MS sends a "location updating request" on carrier 1.

1o)	The SS sends a "location updating accept" message on carrier 1. After 5 s, the MS "selected PLMN indicator" is checked.

1p)	Carrier 1 is turned off. After 30 s, the indication on the MS of available PLMN is checked.

1q)	PLMN3 is selected manually.

1r)	The SS checks that the MS sends a "location updating request" on carrier 3. (Although PLMN3 is in the "forbidden PLMN" list, it should still be able to be selected manually).

1s)	The SS sends a "location updating accept" message on carrier 3. After 5 s, the MS "selected PLMN indicator" is checked.

1t)	Carrier 3 is turned off. The "PLMN search procedure" is initiated manually. After 30 s, the indication on MS of available PLMN is checked.

1u)	The MS is turned off. Carrier 2 is turned off and carriers 3 and 4 are turned on with the parameters as in step 1b) above. The MS is brought into the "on" condition with automatic selection mode active.

1v)	The SS checks that the MS sends a "location updating request" on Carrier 3. (PLMN 3 should have been deleted from the SIM "forbidden PLMN" list in step 1u) above.)

1w)	The SS sends a "location updating accept" message on carrier 3. After 5 s, the MS "selected PLMN indicator" is checked.

1x)	Carrier 3 is turned off. The SS checks that the MS sends a "location updating request" on carrier 4.

1y)	The SS sends a "location updating reject" message on carrier 4 with the cause "location area not allowed".

1z)	The SS checks that the MS answers to the paging, with IMSI, to check that it camps on carrier 4 in accordance with GSM 05.08.

33.3.4.2	Requirements 1

Requirement 1.1) is mandatory for all MS. Requirements 1.2) and 1.3) only apply to MS with a human interface.

1.1)	The MS shall make a response as indicated in steps 1d), 1g), 1j), 1m), 1n), 1r), 1v), 1x) and 1z) above. In all cases, the MS shall respond within 30 s.

1.2)	The selected PLMN shall be indicated:

End of Step	1e)	1k)	1o)	1s)	1w)

PLMN indicated:	PLMN1	PLMN4	PLMN1	PLMN3	PLMN3

1.3)	The following PLMN shall be indicated as available:

Step 1p):	PLMN2,	PLMN3

Step 1t):	PLMN2

33.3.4.3	Procedure 2

2a)	The MS is set up with a SIM which contains NULL values in the "PLMN selector" data field. PLMN1 is the Home PLMN of the MS as defined in the IMSI. The "forbidden PLMN" data field shall contain PLMN3.

2b)	The SS transmits 4 BCCH carriers with the following parameters:

PLMN	Level dBVemf(  )

Carrier 1	PLMN1	28

Carrier 2	PLMN2	33

Carrier 3	PLMN3	38

Carrier 4	PLMN4	43

Each carrier has the "IMSI attach" (ATT) flag set in the BCCH data. (The purpose of this is to force the MS to do location updating whenever it is switched on, so that the SS can determine which PLMN has been selected.)

The other system information parameters are as in table 33-1.

2c)	The MS is brought into the "on" condition with manual selection mode active.

2d)	The SS checks that the MS sends a "location updating request" on carrier 1.

33.3.4.4	Requirements 2

2.1)	The MS shall make a response as indicated in step 2d). The MS shall respond within 30 s.

Table 33-1 : Normal system information fields

Parameter	Reference in GSM 04.08	Abbreviation	Normal setting

Cell Channel Description	10.5.2.1	-	Any values

Max retrans	10.5.2.17	-	1

Tx-integer	10.5.2.17	-	Any value

CELL_BAR_ACCESS	10.5.2.17	CBA	0 (i.e. no barred)

AC CN	10.5.2.17	AC	All 0

RE	10.5.2.17	RE	0 (i.e. re-estab-
			lishment allowed)

BA ARFCN	10.5.2.13	BA	One entry equal to 
			the ARFCN of the
			 carrier

NCC	10.5.2.15	NCC	Any value

Cell Identity	10.5.1.1	-	Any value

MCC, MNC	10.5.1.3	PLMN	Ref. 33.3.2, 1b) 
			and 33.3.2, 2b)

LAC	10.5.1.3	LAC	1111 (Hex)

ATT, B_AG_BLKS_RES,T3212,	10.5.2.8	-	ATT = '1'

CCCH_CONF			Other parameters 
			any values.

BS_PA_MFRMS	10.5.2.8	BPM	5 frames

Cell Options	10.5.2.3	-	Any values

CELL_RESELECT_HYSTERESIS	10.5.2.4	CRH	10 dB

MS_TXPWR_MAX_CCH	10.5.2.4	MTMC	Maximum RF 
			output power of 
			MS.

RXLEV_ACCESS_MIN	10.5.2.4	RAM	-95 dBm

## 33.4	Invalid and blocked PIN indicators

33.4.1	Definition and applicability

The requirements and this test apply to all MS.

33.4.2	Conformance requirement

33.4.3	Test purpose

33.4.4	Method of test

33.4.4.1	Initial conditions

The MS contains a SIM with the PIN enabled, and the SIM unblocking counter set to zero by previous presentation of the personal unblocking key.

33.4.4.2	Procedure

a)	The MS is switched on.

b)	Three wrong PIN are entered.

Activation may be either manual or via the EMMI.

33.4.5	Test requirements

For the first and second incorrect PIN the MS shall indicate that the PIN code has been rejected

For the third incorrect PIN the MS shall indicate that the PIN is blocked.

## 33.5	Service indicator

33.5.1	Definition and applicability

The requirements and this test apply to all MS.

33.5.2	Conformance requirement

33.5.3	Test purpose

33.5.4	Method of test

33.5.4.1	Initial conditions

a)	The MS is in idle mode, unregistered.

b)	The SS shall emulate perfect radio conditions so that the MS is able to register and to set up or receive a call.

33.5.4.2	Procedure

a)	The MS is brought in an active state by either switching it on or by inserting a SIM.

33.5.5	Test requirements

1)	The successful registration and the good condition shall be indicated by the MS indicator and by the SS.

## 33.6	Subscription identity management

33.6.1	Definition and applicability

Subscription identity management is the ability of the MS to prevent the establishment of MO (except MO emergency calls) and MT calls without a valid subscription.

The requirements and this test apply to all MS.

33.6.2	Conformance requirement

An MS can only be operated, if a valid IMSI is present.

33.6.3	Test purpose

To verify that an MS without SIM card will not accept an incoming call.

33.6.4	Method of test

33.6.4.1	Initial conditions

33.6.4.2	Procedure

a)	A call is set up.

b)	(Reserved).

c)	Either

(i)	The SIM is removed.

(ii)	Where this is not possible, the MS is powered down, the SIM is removed and the MS is powered on.

The SS observes whether or not the MS performs IMSI detach.

d)	An attempt to establish a MO call is made (not an emergency call).

e)	An attempt to establish a MT call is made.

33.6.5	Test requirements

1)	In step c(i), the MS shall perform an IMSI detach.

In step c(ii), the MS may perform an IMSI detach.

2)	In step d) the MS shall not attempt to set up a new call via the Um interface.

3)	In step e), the MS shall not attempt to set up a new call via the Um interface.

## 33.7	Barring of outgoing calls

33.7.1	Definition and applicability

The barring of outgoing calls is an optional feature. It is the ability of the MS to prevent all MO calls except emergency calls.

The requirements and this test apply to all MS supporting this feature.

33.7.2	Conformance requirement

An MS may have an optional facility to bar outgoing calls. Such barring facility shall not prevent the transmission on emergency calls.

33.7.3	Test purpose

To verify that an MS for which a local facility to bar outgoing calls has been declared as being implemented, is able to establish an emergency call if this facility is activated.

33.7.4	Method of test

33.7.4.1	Initial conditions

33.7.4.2	Procedure

a)	The local facility to bar outgoing calls is activated.

b)	Via MMI, the MS is actioned to establish an emergency call.

33.7.5	Test requirements

1)	The MS shall establish an emergency call.

## 33.8	Prevention of unauthorized calls

33.8.1	Definition and applicability

The prevention of unauthorized calls is an optional feature in the MS. It is the ability of the MS to prevent unauthorized use by using a key or keyword protection facility. When activated the MS does not prevent the establishment of except emergency calls.

The requirements and this test apply to all MS supporting this feature.

33.8.2	Conformance requirement

An MS may have an optional facility to prevent unauthorized use. Such facility shall not prevent the transmission on emergency calls.

33.8.3	Test purpose

To verify that an MS for which a local facility to prevent unauthorized use has been declared to be implemented, is able to establish an emergency call, if this facility is activated.

33.8.4	Method of test

33.8.4.1	Initial conditions

33.8.4.2	Procedure

a)	The local facility to restrict operation such that the MS can only be operated by using a key or a keyword is activated. The most restrictive situation is created.

b)	Via MMI, the MS is actioned to establish an emergency call.

33.8.5	Test requirements

1)	The MS shall establish an emergency call.

# 34	Short message service (SMS)

Ref : GSM 03.40, 04.11 (point to point)
GSM 03.41, 04.12 (cell broadcast)

## 34.1	General

The purpose of these tests is to verify that the MS can handle GSM functions when submitting or receiving Short Messages (SM) between MS and a short message service centre as described in GSM 03.40.

The procedures are based upon services provided by the Mobility Management (MM) sublayer which is not tested in this case.

The SMS comprises three basic services. The SMS point to point services shall work in an active MS at any time independent of whether or not there is a speech or data call in progress. The SMS cell broadcast service only works when the MS is in idle mode.

Since the timer TC1M currently is not standardised, the value of TC1M shall be declared by the manufacturer (to be used in 34.2.1 and 34.2.2).

The manufacturer shall declare whether SMS messages are stored in the SIM and/or the ME. This shall be referred to as the SMS message store in the following tests.

Unless otherwise stated default message contents from section 26.6.14 applies for GSM900 and default message contents from section 26.6.15 applies for DCS1800.

## 34.2	Short message service point to point

### 34.2.1	SMS mobile terminated

34.2.1.1 	Conformance requirements

An active MS shall be able to receive short message TPDU (SMS-DELIVER) at any time, independently of whether or not there is a speech or data call in progress. A report will always be returned to the SC, confirming that the MS has received the short message.

Reference

GSM 03.40; 3.1

34.2.1.2 	Test purpose

To verify the ability of a MS to receive and decode the SMS where provided for the point to point service.

34.2.1.3 	Method of test

Initial Conditions

System simulator:

1 cell, default parameters.

Mobile Station:

The MS shall be in "Idle, updated" state.

The SMS message storage shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME

Support for call control state U10

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The SS initiates the transmission of a short message using a paging request. Upon response of the MS to the paging the SS assigns an SDCCH, authenticates the MS and activates ciphering. Then the SS establishes SAPI 3 by sending a SABM frame with SAPI 3 on the SDCCH.

When a UA frame (SAPI 3) is received in response, the SS sends a CP-DATA message. The information element of the CP-DATA message will be RP-DATA RPDU (SMS DELIVER TPDU).

b)	The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

c)	The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

d)	Steps a), b) and c) are repeated but the first CP-DATA message from the MS is not acknowledged. The second CP-DATA message from the MS is acknowledged by a CP-ACK within a time TC1M.

e)	Steps a) and b) are repeated. The SS is configured not to send CP-ACK. Then maximum 3 CP-DATA retransmissions may occur. After a duration of TC1M + 5 seconds after the last CP-DATA retransmission the SS then initiates the channel release. The 5 seconds is the appropriate time to wait to verify that the MS does not send more than the maximum allowed (3) CP-DATA retransmissions.

f)	The SMS message store shall be cleared manually by the operator.

g)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH.

When a UA frame (SAPI-3) is received in response, the SS sends a CP-DATA message. The information element of the CP-DATA message will be RP-DATA RPDU (SMS DELIVER TPDU). The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

h)	The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release. The SMS message store shall be cleared manually by the operator.

i)	Steps g) and h) are repeated but the first CP-DATA message from the MS is not acknowledged. The second CP-DATA message from the MS is acknowledged by a CP-ACK within a time TC1M.

j)	Step g) is repeated. The SS is configured not to send CP-ACK. Then maximum 3 CP-DATA retransmissions may occur. After a duration of TC1M + 15 seconds  after the last CP-DATA retransmission the SS initiates the channel release.  The 15 seconds is the appropriate time to wait to verify that the MS does not send more than the maximum allowed (3) CP-DATA retransmissions (during a call in progress).

k)	A data or speech call is established on a TCH with the SS and the state U10 of call control shall be entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH. After the UA response on SAPI-3, the speech call is cleared by the SS with a disconnect message. (The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS).

The SS sends a CP-DATA RPDU (SMS DELIVER TPDU) message. The information element of the CP-DATA message is RP-DATA.

The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

The SMS message store shall be cleared manually by the operator.

l)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH. After the UA response on SAPI-3, the SS waits 15 seconds. During this time the speech call shall be cleared from the MS. (The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS).

The SS sends a CP-DATA message. The information element of the CP-DATA message is RP-DATA RPDU (SMS DELIVER TPDU).

The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

The SMS message store shall be cleared manually by the operator.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST		
	2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	5	SS -> MS	AUTHENTICATION REQ
	6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	9	SS		SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 	MS shall respond to SABM in step 10
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	13	SS 		Waits max 25 seconds for CP-ACK
	14	MS -> SS	CP-ACK	
	15	SS 		Waits max 60 seconds for RP-ACK RPDU
	16	MS -> SS	CP-DATA	Contains RP-ACK RPDU
	17	SS -> MS	CP-ACK	Within TC1M after step 16, no further 
				CP-DATA messages
	18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	19	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	20	SS -> MS	PAGING REQUEST
	21	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	22	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	23	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	24	SS -> MS	AUTHENTICATION REQ
	25	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	26	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	27	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	28	SS		SS starts ciphering.
	29	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	30	MS -> SS	UA (SAPI=3)	The MS shall respond to the SABM 
	31	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	32	SS 		Waits max 25 seconds for CP-ACK
	33	MS -> SS	CP-ACK
	34	SS 		Waits max 60 seconds for RP-ACK RPDU
	35	MS -> SS	CP-DATA	First CP-DATA from MS, contains RP-ACK
				RPDU
	36	SS		First CP-DATA message not acknowledged by
				SS
	37	MS -> SS	CP-DATA	Retransmitted CP-DATA from MS within 
				TC1M, after step 35, contains RP-ACK RPDU
	38	SS -> MS	CP-ACK	Second CP-DATA message is acknowledged
				within TC1M after step 37, no further 
				CP-DATA messages
	39	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

40	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	41	SS -> MS	PAGING REQUEST
	42	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	43	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	44	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	45	SS -> MS	AUTHENTICATION REQ
	46	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	47	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	48	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	49	SS		SS starts ciphering.
	50	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	51	MS -> SS	UA (SAPI=3) 	The MS shall respond to the SABM
	52	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	53	SS 		Waits max 25 seconds for CP-ACK
	54	MS -> SS	CP-ACK
	55	SS 		Waits max 60 seconds for RP-ACK RPDU
	56	MS -> SS	CP-DATA	Contains RP-ACK RPDU
	57	SS		First CP-DATA message not acknowledged by
				SS
	58	MS -> SS	CP-DATA	Retransmitted CP-DATA from MS within twice
				TC1M after step 56, contains RP-ACK RPDU
	59	SS 		Retransmitted CP-DATA message not 
				acknowledged by SS
	60	MS		Depending upon the maximum number of
				CP-DATA retransmissions implemented, step 58 
				and 59 may be repeated. 
	61	SS -> MS	CHANNEL RELEASE	The main signalling link is released after a
				duration of TC1M + 5 seconds after the last
				CP-DATA retransmission..
	62	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	63	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	64	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	65	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM
	66	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	67	SS		Waits max 25 seconds for CP-ACK
	68	MS -> SS	CP-ACK
	69	SS		Waits max 60 seconds for RP-ACK RPDU
	70	MS -> SS	CP-DATA	Contains RP-ACK RPDU
	71	SS -> MS	CP-ACK	Within TC1M after step 70, no further
				CP-DATA messages
	72	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	73	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	74	MS		Clear the SMS message store

75	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	76	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH	77	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM
	78	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	79	SS		Waits max 25 seconds for CP-ACK
	80	MS -> SS	CP-ACK
	81	SS		Waits max 60 seconds for RP-ACK RPDU
	82	MS -> SS	CP-DATA	First CP-DATA from MS, contains RP-ACK
				RPDU
	83	SS		First CP-DATA message not acknowledged by
				SS
	84	MS -> SS	CP-DATA	Retransmitted CP-DATA message within twice
				TC1M after step 82, contains RP-ACK RPDU
	85	SS -> MS	CP-ACK	Second CP-DATA message is acknowledged
				within TC1M after step 84, no further 
				CP-DATA messages
	86	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	87	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	88	MS		Clear the SMS message store
	89	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	90	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	91	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM
	92	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	93	SS		Waits max 25 seconds for CP-ACK
	94	MS -> SS	CP-ACK
	95	SS		Waits max 60 seconds for RP-ACK RPDU
	96	MS -> SS	CP-DATA	First CP-DATA from MS, contains RP-ACK
				RPDU
	97	SS		First CP-DATA message not acknowledged by
				SS
	98	MS -> SS	CP-DATA	Transmitted CP-DATA message within 
				TC1M after step 96, contains RP-ACK RPDU
	99	SS 		Retransmitted CP-DATA message not 
				acknowledged by SS
	100	MS		Depending on the maximum number of CP-DATA
				retransmissions implemented, step 98-99 may be
				repeated. The maximum number of 
				retransmissions may however not exceed 
				three. 
 	101	SS -> MS	CHANNEL RELEASE	The main signalling link is released after a 
				duration of TC1M + 15 seconds after the last 
				CP-DATA retransmission.
	102	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	103	MS		Clear the SMS message store
	104	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	105	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	106	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM

107	SS -> MS	DISCONNECT	The speech call is cleared by the SS. The call
				clearing is continued on the FACCH in parallel
				to the following exchange of messages related
				to SMS.
	108	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	109	SS		Waits max 25 seconds for CP-ACK
	110	MS -> SS	CP-ACK
	111	SS		Waits max 60 seconds for RP-ACK RPDU
	112	MS -> SS	CP-DATA	Contains RP-ACK RPDU
	113	SS -> MS	CP-ACK	Within TC1M after step 112, no further 
				CP-DATA 
	114	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	115	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	116	MS		Clear the SMS message store
	117	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	118	SS -> MS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	119	MS -> SS 	UA (SAPI=3)	The MS shall respond to the SABM
	120	SS		Waits 15 seconds
	121	MS		During the SS wait time in step 120 the
				speech call shall be cleared from the MS. The
				call clearing is continued on the FACCH in
				parallel to the following exchange of messages
				related to SMS.
	122	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	123	SS		Waits max 25 seconds for CP-ACK
	124	MS -> SS	CP-ACK
	125	SS		Waits max 60 seconds for RP-ACK RPDU
	126	MS -> SS	CP-DATA	Contains RP-ACK RPDU
	127	SS -> MS	CP-ACK	Within TC1M after step 126, no further 
				CP-DATA 
	128	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	129	MS		The MS shall indicate that an SM has arrived.
				If the MS provides the functionality to display
				MT messages, it is checked that the correct
				message is displayed
	130	MS		Clear the SMS message store

NOTE 13:	Time values for SS wait time are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS DELIVER TPDU

Information element	Comment	Value

TP-MTI 	SMS DELIVER	‘00’B

TP-MMS 	more messages are waiting in SC	‘0’B

TP-RP	no reply path	‘0’B

TP-UDHI	TP-UD contains only the SM	‘0’B

TP-SRI	no status report returned	‘0’B

TP-OA 	an international number coded E.164

TP-PID 	default	‘00000000’B

TP-DCS 	default alphabet	‘00000000’B

TP-SCTS 	any legal value (cf. GSM 03.40)

TP-UDL 		160

TP-UD (140 octets)	text of message (160 characters)

NOTE 24:	The 160 characters shall include at least one occurrence of each character in the default alphabet (see GSM 03.40 annex 2).

### 34.2.2	SMS mobile originated

34.2.2.1	Conformance requirements

An active MS shall be able to submit short message TPDU (SMS-SUBMIT) at any time, independently of whether or not there is a speech or data call in progress.

Reference

GSM 03.40; 3.1

34.2.2.2	Test purpose

To verify that the MS is able to correctly send a short message where the SMS is provided for the point to point service. The test also verifies that the MS is capable of simultaneously receive a network originated SM whilst sending a mobile originated SM.

34.2.2.3	Method of test

Initial Conditions

System simulator:

1 cell, default parameters.

Mobile Station:

The MS shall be in "Idle, updated" state.

The SMS message storage shall be empty.

Related PICS/PIXIT Statements

Support for Short message MO/PP

Description of the basic procedures to display a mobile originated short message

Support for state U10 of call control

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME.

Maximum length (characters) of a mobile originated short message.

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The MS shall be set up to send a SM to the SS. The SS responds to the channel request message by allocating an SDCCH. The SS answers correctly to the SABM on SAPI 0 and then performs the authentication and ciphering procedures.

b)	The SS responds with a UA frame SAPI-3 to the MS.

c)	The SS responds to the CP-DATA containing RP-DATA RPDU (SMS SUBMIT TPDU) from the MS with a CP-ACK message within TC1M followed by a CP-DATA message containing the correct RP-ACK RPDU. The SS waits a maximum of 25 seconds for the CP-ACK message.

d)	The SS sends a channel release message to the MS.

e)	Steps a) and b) are repeated. The SS is configured not to send the CP-ACK message. Then maximum 3 CP-DATA retransmissions may occur. After a duration of TC1M + 5 seconds after the last CP-DATA retransmission the SS initiates channel release. The 5 seconds is the appropriate time to wait to verify that the MS does not send more than the maximum CP-DATA retransmissions.

f)	Steps a) and b) are repeated. On receipt of the CP-DATA from the MS the SS sends a CP-ERROR message within TC1M containing a "Network Failure" cause. Then the SS initiates channel release.

g)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The MS is setup to send an SM to the SS. After the reception of the CM SERVICE REQUEST, the SS sends a CM SERVICE ACCEPT message. The SS responds with a UA frame SAPI-3 to the SABM with SAPI-3 received from the MS.

h)	The SS responds to the CP-DATA containing RP-DATA RPDU (SMS SUBMIT TPDU) from the MS with a CP-ACK message within TC1M followed by a CP-DATA message containing the correct RP-ACK RPDU. The SS waits a maximum of 25 seconds for the CP-ACK message. Then the SS sends a channel release message to the MS.

i)	Step g) is repeated. The SS is configured not to send the CP-ACK message. Then maximum 3 CP-DATA retransmissions may occur. After a duration of TC1M + 15 seconds after the last CP-DATA retransmission the SS initiates channel release. The 15 seconds is the appropriate time to wait to verify that the MS does not send more than the maximum CP-DATA retransmissions (during a call in progress).

j)	The SS is configured to receive a mobile originated SM. Steps a) and b) are repeated and, using the end of the CP-DATA message from the MS as a trigger, the SS sends a SM to the MS. In this case a new transaction identifier shall be used in the CP messages of SMS mobile terminated.

k)	The MS is set up to send an SM to the SS. On receipt of the CM SERVICE REQUEST the SS sends a CM SERVICE REJECT message with the reject cause set to "Service Option not supported" or "Service Option temporarily out of order". After 5 seconds the SS initiates channel release.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establishment cause is “Other procedures which
				can be completed with an SDDCH” NECI = 0
	2	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	3	MS -> SS	CM SERVICE REQ	Message is contained in SABM on SAPI 0. CM
				service type set to "Short message transfer"
	4	SS -> MS	AUTHENTICATION REQ
	5	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	6	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	7	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	8	SS		SS starts ciphering.
	9	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	10	SS -> MS	UA (SAPI=3) 
	11	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	12	SS -> MS	CP-ACK	Sent within TC1M after step 11
	13	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	14	SS		Waits max 25 seconds for CP-ACK
	15	MS -> SS	CP-ACK
	16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	17	MS -> SS	DISC (SAPI=0)	MS shall respond to channel release with a layer 2
				DISC frame with SAPI 0
	18	MS -> SS	CHANNEL REQUEST	Establishment cause is “Other procedures which
				can be completed with an SDDCH” (NECI=0)
	19	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	20	MS -> SS	CM SERVICE REQ	Message is contained in SABM on SAPI 0.
	21	SS -> MS	AUTHENTICATION REQ
	22	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	23	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	24	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	25	SS		SS starts ciphering.
	26	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	27	SS -> MS	UA (SAPI=3) 
	28	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	29	SS	 	SS configured not to send CP-ACK
	30	MS -> SS	CP-DATA	Retransmitted CP-DATA message within TC1M 
				after step
				28
	31	MS		Depending on the maximum number of CP-DATA
				retransmissions implemented, step 30 may be 
				repeated. The maximum number of 
				retransmissions may however not exceed 	
				three. 
	32	SS -> MS	CHANNEL RELEASE	The main signalling link is released after a
				duration of TC1M + 5 seconds after the last 
				CP-DATA retransmission.
	32a	MS -> SS	DISC (SAPI = 0)	MS shall respond to channel release with a layer 2
				DISC frame with SAPI 0.
	33	MS -> SS	CHANNEL REQUEST	Establishment cause is “Other procedures which
				can be completed with an SDDCH” (NECI=0) 
	34	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

35	MS -> SS	CM SERVICE REQ	Message is contained in SABM. CM service
				type set to "short message transfer"
	36	SS -> MS	AUTHENTICATION REQ
	37	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	38	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	39	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	40	SS		SS starts ciphering.
	41	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	42	SS -> MS	UA (SAPI=3) 
	43	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	44	SS -> MS	CP-ERROR	Sent within TC1M containing "Network
				Failure" cause.
 45	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	45a	MS -> SS	DISC (SAPI = 0)	MS shall respond to channel release with a layer 2
				DISC frame with SAPI 0.
	46	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	47	MS		The MS is set up to send an SM
	48	MS -> SS	CM SERVICE REQ	Sent in a layer 2 frame on the FACCH. CM
				service type set to "short message transfer"
	49	SS -> MS	CM SERVICE ACCEPT
	50	MS -> SS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	51	SS -> MS 	UA (SAPI=3)
	52	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	53	SS -> MS	CP-ACK	Sent within TC1M after step 52
	54	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	55	SS		Waits max 25 seconds for CP-ACK
	56	MS -> SS	CP-ACK
	57	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	58	MS -> SS	DISC (SAPI =0)	The MS shall respond to channel release with a 
				layer 2 DISC frame with SAPI 0.
	59	SS		A data or speech call is established on a TCH
				and the state U10 of call control is entered.
	60	MS -> SS	CM SERVICE REQ	Sent in a layer 2 frame on the FACCH. CM
				service type set to "short message transfer"
	61	SS -> MS	CM SERVICE ACCEPT
	62	MS -> SS	SABM (SAPI=3)	Sent on SACCH associated with the TCH
	63	SS -> MS 	UA (SAPI=3)
	64	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	65	SS		SS configured not to send CP-ACK
	66	MS -> SS	CP-DATA	Transmitted CP-DATA message TC1M after step
				64
	67	MS		Depending on the maximum number of CP-DATA
				retransmissions implemented, step 66 may be
				repeated. The maximum number of 
				retransmissions may however not exceed 
				three.
	68	SS -> MS	CHANNEL RELEASE	The main signalling link is released after a
				duration of TC1m + 15 seconds after the last 
				CP-DATA retransmission.
	69	MS -> SS	DISC (SAPI =0)	The MS shall respond to channel release
	70	MS -> SS	CHANNEL REQUEST	Establishment cause is “Other procedures which
				can be completed with an SDDCH” (NECI=0)
	71	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH

72	MS -> SS	CM SERVICE REQ	Message is contained in SABM. CM service
				type set to "short message transfer"
	73	SS -> MS	AUTHENTICATION REQ
	74	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	75	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	76	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	77	SS		SS starts ciphering.
	78	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	79	SS -> MS	UA (SAPI=3) 
	80	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	81	SS		The SS sends an SM to the MS triggered by
				the end of the CP-DATA message from the MS
	82	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
	83	MS		The MS shall correctly receive the SM and
				indicate that a message has arrived. If the MS
				provides the functionality to display MT
				messages, it is checked that the correct
				message is displayed. In the MO case the MS
				shall send the CP-ACK message with
				transaction identifier assigned to this transfer.
				In the MT case the MS shall send a CP-ACK
				message and a CP-DATA message containing
				the RP-ACK RPDU. The transaction identifier
				shall be the same as chosen by the SS for the
				MT transfer. 


	84	MS -> SS	CHANNEL REQUEST	Establishment cause is “Other procedures which
				can be completed with an SDDCH” (NECI=0)
	85	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	86	MS -> SS	CM SERVICE REQ	Message is contained in SABM. CM service
				type set to "short message transfer"
	87	SS -> MS	CM SERVICE REJ	Reject cause set to "Service Option not
				supported" or "Service Option temporarily out
				of order"
	88	MS		The MS shall not establish SAPI-3
	89	SS -> MS	CHANNEL RELEASE	Sent 5 seconds after CM SERVICE REJ

NOTE 35:	Time values for SS wait times are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS SUBMIT TPDU

Information element	Comment	Value

TP-MTI 	SMS SUBMIT	‘01’B

TP-VPF 	not checked

TP-RP	no reply path	‘0’B

TP-UDHI	not checked

TP-SRR	not checked

TP-MR	not checked

TP-RD	not checked

TP-DA 	not checked (an E164 number)

TP-PID 	default	‘00000000’B

TP-DCS 	default alphabet	‘00000000’B

TP-VP 	not checked

TP-UDL 	as applicable

TP-UD (140 octets max)	maximum number of characters

(text of message) as defined

by the manufacturer (see PICS/PIXIT)

### 34.2.3	Test of memory full condition and memory available notification:

The Memory Available Notification provides a means for the MS to notify the network that it has memory available to receive one or more short messages. The SMS status field in the SIM contains status information on the "memory available" notification flag.

34.2.3.1	Conformance requirement

1.	When a mobile terminated message is Class 2, the MS shall ensure that the message has been transferred to the SMS data field in the SIM before sending an acknowledgement to the SC. The MS shall return a protocol error message if the short message cannot be stored in the SIM and there is other short message storage available in the MS. If all the short message storage in the MS is already in use, the MS shall return "memory capability exceeded".

2.	When the MS rejects a short message due to lack of available memory capability the need to transfer notification shall be stored in the SIM.

3.	If the memory capability becomes available because memory is cleared, the value of the memory capability exceeded notification flag in the SIM is read. If the flag is set, the MS notifies the network that memory capability is now available. After a positive acknowledgement from the network, the ME unsets the memory capability exceeded notification flag in the SIM.

References

GSM 03.40, 9.2.3.10, GSM 03.38, 4.

GSM 03.40, 10.3 (operation 14)

GSM 03.40, 10.3 (operation 14)

34.2.3.2	Test purpose

1.	To verify that the MS sends the correct acknowledgement when its memory in the SIM becomes full.

2.	To verify that the MS sends the correct acknowledgement when its memory in the ME and the SIM becomes full, and sets the "memory exceeded" notification flag in the SIM.

3.	To verify that the MS performs the "memory available" procedure when its message store becomes available for receiving short messages, and only at this moment.

34.2.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The SMS message storage shall be empty.

The MS shall be connected to the SIM simulator. The following shall be present in the SIM 	simulator:

-	EFSMS with at least one record

-	EFSMSstatus, with SMS "Memory Cap. Exceed" notification flag set to "memory 	available"

-	Service no. 4 (SMS) in EFSST set to allocated and activated

For storing of Class 1 Short Messages the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

Whether SMS messages are stored in the SIM and/or the ME

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	step a) of 34.2.5.3 (test of Class 2 Short Messages) is repeated until the MS sends a negative acknowledgement (RP-ERROR). The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been set on the SIM.

b)	a Class 1 Short Message is sent to the MS.

c)	step b) is repeated until the MS sends a negative acknowledgement (RP-ERROR). The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been set on the SIM.

d)	a Short Message is sent to the MS with the DCS field of the SMS-DELIVER TPDU set to 0.

e)	the SS prompts the operator to read a short message and to remove it from the message store of the MS.

f)	the SS waits for a CHANNEL REQUEST from the MS, and sends an IMMEDIATE ASSIGNMENT allocating an SDCCH.

g)	the SS answers correctly to the SABM on SAPI 0.

h)	the SS answers correctly to the SABM on SAPI 3.

i)	the SS answers to the RP-SMMA from the MS with a CP-DATA containing a RP-ACK RPDU.

j)	after the MS has acknowledged the CP-DATA with a CP-ACK, the SS releases the channel with a CHANNEL RELEASE message. The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been unset on the SIM.

k)	step e) is repeated.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST
	2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	5	SS -> MS	AUTHENTICATION REQ
	6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	9	SS		SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU), Class 2 Short Message
	13	SS 		Waits max 25 seconds for CP-ACK
	14	MS -> SS	CP-ACK	
	15	SS 		Waits max 60 seconds for RP-ACK RPDU
	16	MS -> SS	CP-DATA	Contains RP-ACK RPDU
	17	SS -> MS	CP-ACK	Within TC1M after step 16
	18	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
				Step 1-18 is repeated until MS sends a
				negative acknowledgement (RP-ERROR) in step
				16. The RP-ERROR RPDU cause field
				shall be "Protocol error, unspecified" if there is
				message capability in the ME, or "Memory
				capability exceeded" if there is no message 
				capability in the ME. If the total memory store 
				of the MS is full, the ME shall set the 
				"memory capability exceeded" notification
				flag on the SIM.
	19	SS -> MS	PAGING REQUEST
	20	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	21	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	22	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	23	SS -> MS	AUTHENTICATION REQ
	24	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	25	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the 
				message.
	26	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following 
				messages shall be sent enciphered.
	27	SS		SS starts ciphering.
	28	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	29	MS -> SS	UA (SAPI=3) 
	30	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU), Class 1 Short Message
	31	SS 		Waits max 25 seconds for CP-ACK
	32	MS -> SS	CP-ACK
	33	SS 		Waits max 60 seconds for RP-ACK RPDU	34	MS -> SS	CP-DATA	Shall contain RP-ACK RPDU if there is memory
				capability in the ME. If not it shall contain 
				RP-ERROR RPDU which cause field shall be 
				"memory capability exceeded". If the total
				memory store of the MS now becomes full at
				this step, the ME shall set the
				"memory cap. exceed" notification flag on the SIM.
	35	SS -> MS	CP-ACK	Within TC1M after step 34

36	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	
				Step 19-36 is repeated until the MS sends an
				RP-ERROR. The SIM simulator shall indicate if
				the "memory capability exceeded" notification
				flag has been set on the SIM.
	37	SS -> MS	PAGING REQUEST
	38	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	39	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	40	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	41	SS -> MS	AUTHENTICATION REQ
	42	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	43	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	44	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	45	SS		SS starts ciphering.
	46	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	47	MS -> SS	UA (SAPI=3) 
	48	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU) with TP-DCS set to 0
	49	SS 		Waits max 25 seconds for CP-ACK
	50	MS -> SS	CP-ACK
	51	SS 		Waits max 60 seconds for RP-ACK RPDU
	52	MS -> SS	CP-DATA	Shall contain RP-ERROR RPDU with error
				cause "memory capability exceeded".
	53	SS -> MS	CP-ACK	Within TC1M after step 52
	54 	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	55	SS		Prompts the operator to read a short message
				and to remove it from the message store of
				the MS.
	57	MS -> SS	CHANNEL REQUEST	Establishment cause "Other services which
				can be completed with an SDCCH" (NECI=0).
	58	SS -> MS	IMMEDIATE ASSIGNMENT	SS allocates an SDCCH
	59	MS -> SS	CM SERVICE REQUEST	Message is contained in SABM. CM service
				type information element is set to "Short
				message transfer".
	60	SS -> MS	CM SERVICE ACCEPT (UA)	SAPI 0
	61	MS -> SS	SABM (SAPI=3)	MS shall establish SAPI 3
	62	SS -> MS	UA (SAPI=3)
	63	MS -> SS	CP-DATA	Contains RP-SMMA RPDU
	64	SS -> MS	CP-ACK
	65	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	66	MS -> SS	CP-ACK	Acknowledge of CP-DATA containing the 
				RP-ACK RPDU. The ME shall unset the 
				"memory capability exceeded" notification flag
				on the SIM.
	67	SS -> MS	CHANNEL RELEASE	The main signalling link is released. The SIM 
				simulator shall indicate if the "memory
				capability exceeded" notification flag has been
				unset on the SIM.
	68	SS		Prompts the operator to read a short message and 
				to remove it from the message store of the MS.
	69	MS		Shall not attempt to send a RP-SMMA RPDU.
				This is verified by checking that the MS does
				not send a CHANNEL REQUEST message with
				the establishment cause "Other services
				which can be completed with an SDCCH"

NOTE 46:	Time values for SS wait time are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS-DELIVER TPDU in step 12

Information element	Comment	Value

TP-MTI 	SMS DELIVER	‘00’B

TP-MMS 	more messages are waiting in SC	‘0’B

TP-RP	no reply path	‘0’B

TP-UDHI	TP-UD contains only the SM	‘0’B

TP-SRI	no status report returned	‘0’B

TP-OA 	an international number coded E.164

TP-PID 	default	‘00000000’B

TP-DCS 	default alphabet, class 2	‘11110010’B

TP-SCTS 	any legal value (cf. GSM 03.40)

TP-UDL 		160

TP-UD (140 octets)	text of message (160 characters)

SMS-DELIVER TPDU in step 30

same as in step 12 except:

SMS-DELIVER TPDU in step 48

same as in step 12 except

TP-DCS 	default alphabet	‘00000000’B

### 34.2.4	Test of the status report capabilities and of SMS-COMMAND:

This test applies to MSs which support the status report capabilities.

34.2.4.1	Conformance requirement

The SMS offers the SC the capabilities of informing the MS of the status of a previously sent mobile originated short message. This is achieved by the SC returning a status report TPDU (SMS-STATUS-REPORT) to the originating MS.

SMS-COMMAND enables an MS to invoke an operation at the SC.

The MS shall increment TP-MR by 1 for each SMS-SUBMIT or SMS-COMMAND being submitted.

References

GSM 03.40; 3.2.9

GSM 03.40; 9.2.3.6

34.2.4.2	Test purpose

1)	To verify that the MS is able to accept a SMS-STATUS-REPORT TPDU.

2)	To verify that the MS is able to use the SMS-COMMAND functionality correctly and sends an SMS-COMMAND TPDU with the correct TP-Message-Reference.

34.2.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

Related PICS/PIXIT Statements

Support of SMS MO/PP and MT/PP

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The MS is made to send a Mobile Originated short message as in steps a) to d) of test 34.2.2 	(SMS Mobile originated).

b)	The SS establishes a data link on SAPI-3 with the MS, then sends a CP-DATA message containing a RP-DATA RPDU itself containing an SMS-STATUS-REPORT TPDU.

c)	The SS sends a CHANNEL RELEASE message.

d)	The MS is made to send an SMS-COMMAND message enquiring about the previously submitted short message.

e)	The SS responds to the MS so as to enable it to establish a data link on SAPI-3 on an SDCCH.

f)	The SS acknowledges the CP-DATA message from the MS with a CP-ACK followed by a CP-DATA message containing an RP-ACK RPDU

g)	After receiving the CP-ACK from the MS, the SS releases the channel by using a CHANNEL RELEASE message.

h)	The MS is made to send an SMS-COMMAND message requiring to delete the previously submitted short message.

i)	steps e) to g) are repeated.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establishment cause is "Other procedures
				which can be completed with an SDCCH " 
				(NECI=0)
	2	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	3	MS -> SS	CM SERVICE REQ	Message is contained in SABM.
	4	SS -> MS	AUTHENTICATION REQ
	5	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	6	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	7	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	8	SS		SS starts ciphering.
	9	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	10	SS -> MS	UA (SAPI=3) 
	11	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT
				TPDU)
	12	SS -> MS	CP-ACK	Sent within TC1M after step 11
	13	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	14	SS		Waits max 25 seconds for CP-ACK
	15	MS -> SS	CP-ACK
	16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.
	17	SS -> MS	PAGING REQUEST
	18	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	19	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	20	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	22	SS -> MS	AUTHENTICATION REQ
	23	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	24	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	25	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	26	SS		SS starts ciphering.
	27	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	28	MS -> SS	UA (SAPI=3) 
	29	SS -> MS	CP-DATA	Contains RP-DATA RPDU 
				(SMS-STATUS-REPORT TPDU)
	30	MS -> SS	CP-ACK
	31	MS -> SS 	CP-DATA	Contains RP-ACK RPDU
	32	SS ->MS	CP-ACK
	33	SS -> MS	CHANNEL RELEASE
	34	MS		The MS is made to send an SMS-COMMAND 
				message enquiring about the previously
				submitted SM	
	35	MS -> SS	CHANNEL REQUEST	Establishment cause "Other services which
				can be completed with an SDCCH".
	36  	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	37	MS -> SS	CM SERVICE REQ	Message is contained in SABM.
	38	SS -> MS	AUTHENTICATION REQ
	39	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	40	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	41	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	42	SS		SS starts ciphering.
	43	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	44	SS -> MS	UA (SAPI=3)

45	MS -> SS	CP-DATA	Contains RP-DATA RPDU 
				(SMS-COMMAND TPDU) which shall contain
				the correct TP-MR
	46	SS -> MS	CP-ACK
	47	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	48	MS -> SS	CP-ACK
	49 	SS -> MS	CHANNEL RELEASE
	50	MS	The MS is made to send an SMS-COMMAND 
					message requiring to delete the previously
				submitted SM.
	51	MS -> SS	CHANNEL REQUEST	Establishment cause "Other services which
				can be completed with an SDCCH".
	52	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	53	MS -> SS	CM SERVICE REQ	Message is contained in SABM.
	54	SS -> MS	AUTHENTICATION REQ
	55	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	56	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	57	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	58	SS		SS starts ciphering.
	59	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	60	SS -> MS	UA (SAPI=3) 
	61	MS -> SS	CP-DATA	Contains RP-DATA RPDU 
				(SMS-COMMAND TPDU) which shall contain
				the correct TP-MR
	62	SS -> MS	CP-ACK
	63	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	64	MS -> SS	CP-ACK
	65 	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

SMS SUBMIT TPDU

Information element	Comment	Value

TP-MTI 	SMS SUBMIT	‘01’B

TP-VPF 	not checked

TP-RP	no reply path	‘0’B

TP-UDHI	not checked

TP-SRR	status report is requested	‘1’B

TP-MR	not checked

TP-RD	not checked

TP-DA 	not checked (an E164 number)

TP-PID 	 default	‘00000000’B

TP-DCS 	default alphabet	‘00000000’B

TP-VP 	not checked

TP-UDL 	as applicable

TP-UD (140 octets max)	maximum number of characters

SMS-STATUS-REPORT TPDU (SS to MS in step 29):

Information element	Comment	Value

TP-MTI	SMS-STATUS-REPORT	‘10’B

TP-MR	same as previous SMS-SUBMIT

TP-MMS	no more messages	‘1’B

TP-SRQ	result of SMS-SUBMIT	‘0’B

TP-RA	same as the Destination address of the SMS-SUBMIT

TP-SCTS	any legal value (cf. GSM 03.40)

TP-DT	any legal value (cf. GSM 03.40)

TP-ST	SM received	‘00000000’B

first SMS-COMMAND TPDU (MS to SS in step 44)

Information element	Comment	Value

TP-MTI	SMS-COMMAND	‘10’B

TP-MR	TP-MR in previous SMS-SUBMIT plus "1"

TP-SRR	status report requested (03.40  9.2.3.19)	‘1’B

TP-PID	default	‘00000000’B

TP-CT	Enquiry relating to previously submitted 
	          short message	‘00000000’B

TP-MN	not checked (TP-MR in previous SMS-SUBMIT)

TP-DA	not checked (an E164 number)

TP-CDL	not checked

TP-CD	not checked

second SMS-COMMAND TPDU (MS to SS in step 60)

Information element	Comment	Value

TP-MTI	SMS-COMMAND	‘10’B

TP-MR	TP-MR in previous SMS-COMMAND plus "1"

TP-SRR	status report not requested	‘0’B

TP-PID	default	‘00000000’B

TP-CT	Delete previously submitted short message	
		’00000010’B

TP-MN	not checked (TP-MR in previous SMS-SUBMIT)

TP-DA	not checked (an E164 number)

TP-CDL	not checked

TP-CD	not checked

### 34.2.5	Test of message class 0 to 3

The tests under this section only apply to a MS capable of displaying short messages (see PICS/PIXIT).

#### 34.2.5.1	Short message class 0

34.2.5.1.1	Conformance requirement

When a mobile terminated message is class 0 and the MS has the capability of displaying short messages, the MS shall display the message immediately and send an acknowledgement to the SC when the message has successfully reached the MS irrespective of whether there is memory available in the SIM or ME. The message shall not be automatically stored in the SIM or ME.

References

GSM 03.38, 4.

34.2.5.1.2	Test purpose

To verify that the MS will accept and display but not store a class 0 message, and that it will accept and display a class 0 message if its message store is full.

NOTE 57:	failure of this test in a mobile could cause it to reject a class 0 message when its SMS memory becomes full. This could lead to unwanted repetitions between the MS and the service centre.

34.2.5.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME.

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The SS sends a class 0 message by using the method described in step a) of section 34.2.1 but with the TPDU described in this section.

b)	The MS message store shall be filled (for example by using the method of 34.2.3 test of the memory available notification) with the same SMS-DELIVER TPDU except that TP-DCS is set to class 1.

c)	The SS sends a class 0 message as in step a).

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST
	2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	5	SS -> MS	AUTHENTICATION REQ
	6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	9	SS		SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU),
				Class 0 Short Message
	13	MS -> SS	CP-ACK
	14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	15	SS -> MS	CP-ACK
	16	SS -> MS	CHANNEL RELEASE
	17	MS		The content of the short message shall be
				displayed by the ME. The MS shall not store the
				the message. This can be checked by verifying 
				that it is impossible to retrieve any short messages
				from the MS message store.
	18	SS		The MS message store shall be filled
				(for example by
				using the method of 34.2.3) with Class 1
				SMS-DELIVER TPDU. 
	19	SS -> MS	PAGING REQUEST	
	20	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	21	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	22	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	23	SS -> MS	AUTHENTICATION REQ
	24	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	25	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	26	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	27	SS		SS starts ciphering.
	28	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	29	MS -> SS	UA (SAPI=3) 
	30	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
			TPDU),
				Class 0 Short Message
	31	MS -> SS	CP-ACK
	32	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	33	SS -> MS	CP-ACK
	34	SS -> MS	CHANNEL RELEASE
	35	MS		The content of the short
				message shall be displayed
				by the ME.

Specific Message Contents:

SMS-DELIVER TPDU (containing a class 0 message) (SS to MS):

Information element	Comment	Value

TP-MTI	SMS-DELIVER	‘00’B

TP-MMS	more messages are waiting in SC	‘0’B

TP-RP	no reply path	‘0’B

TP-UDHI	TP-UD contains only the SM	‘0’B

TP-SRI	no status report returned	0

TP-OA	an international number coded E.164

TP-PID	default	‘00000000’B

TP-DCS	default alphabet, class 0	“1111 0000’B

TP-SCTS	any legal value (cf. GSM 03.40)

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters)

SMS-DELIVER TPDU (containing a class 1 message to fill the MS message store) (SS to MS):

Information element	Comment	Value

TP-MTI	SMS-DELIVER	‘00’B

TP-MMS	more messages are waiting in SC	‘0’B

TP-RP	no reply path	‘0’B

TP-UDHI	TP-UD contains only the SM	‘0’B

TP-SRI	no status report returned	0

TP-OA	an international number coded E.164

TP-PID	default	‘00000000’B

TP-DCS	default alphabet, class 1	‘1111 0001’B

TP-SCTS	any legal value (cf. GSM 03.40)

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters)

#### 34.2.5.2	Test of class 1 short messages

This test shall apply to MSs which support

-	storing of received Class 1 Short Messages and

-	displaying of stored Short Messages.

34.2.5.2.1	Conformance requirement

When a mobile terminated message is class 1, the MS shall send an acknowledgement to the SC when the message has successfully reached the MS and can be stored, either in the ME or in the SIM.

References

GSM 03.38, 4.

34.2.5.2.2	Test purpose

This procedure verifies that the MS acts correctly on receiving a class 1 message, i.e. that it stores the message in the ME or SIM and sends an acknowledgement (at RP and CP-Layer).

34.2.5.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

For storing of class 1 Short Messages, the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message of class 1 to the MS as specified in section 34.2.1, step a).

b)	the Short Message is recalled (e.g. by means of the MMI).

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST		
	2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	5	SS -> MS	AUTHENTICATION REQ
	6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	9	SS		SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU), Class 1 Short Message
	13	MS -> SS	CP-ACK
	14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	15	SS -> MS	CP-ACK
	16	SS -> MS	CHANNEL RELEASE
	17	MS		The short message shall be recalled
				and displayed at the MS.

Specific Message Contents:

SMS-DELIVER TPDU (containing a class 1 message) (SS to MS):

Information element	Comment	Value

TP-MTI	SMS-DELIVER	‘00’B

TP-MMS	more messages are waiting in SC	‘0’B

TP-RP	no reply path	‘0’B

TP-UDHI	TP-UD contains only the SM	‘0’B

TP-SRI	no status report returned	0

TP-OA	an international number coded E.164

TP-PID	default	‘00000000’B

TP-DCS	default alphabet, class 1	‘1111 0001’B

TP-SCTS	any legal value (cf. GSM 03.40)

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters)

#### 34.2.5.3	Test of class 2 short messages

34.2.5.3.1	Definition and applicability

Class 2 Short Messages are defined as SIM specific, and the MS shall ensure that a message of this class is stored on the SIM.

This test shall apply to MSs which support

-	storing of received Class 2 Short Messages in the SIM and

-	displaying of Short Messages stored in the SIM.

34.2.5.3.2	Conformance requirement

When a mobile terminated message is Class 2, the MS shall ensure that the message has been correctly  transferred to the SMS data field in the SIM before sending an acknowledgement to the SC. The MS shall return a "protocol error, unspecified" error message if the short message cannot be stored in the SIM and there is other short message storage available at the MS.

Reference(s)

GSM 03.40, 9.2.3.10; GSM 03.38, 4. GSM 11.11, 10.3.3

34.2.5.3.3	Test purpose

This procedure verifies that the MS acts correctly on receiving a class 2 message, i.e. that it stores the message correctly in the SIM, and if this is not possible, returns a protocol error message to the network.

34.2.5.3.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The ME message store shall be empty.

The ME shall be connected to the SIM simulator. The following shall be present in the SIM 	simulator:

-	EFSMS with at least two free records and one full record

-	EFSMSstatus, with SMS "Memory Cap. Exceed" notification flag set to "memory 	available"

-	Service no. 4 (SMS) in EFSST set to allocated and activated

For storing of Class 1 Short Messages the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message of class 2 to the MS as specified in section 34.2.1, step b).

b)	following an attempt by the ME to store the short message in a free record of EFSMS in the SIM, the SIM simulator returns the status response "OK" ('90 00').

c)	step a) is repeated.

d)	following an attempt by the ME to store the short message in a free record of EFSMS in the SIM, the SIM simulator returns the status response "memory problem" ('92 40').

e)	the SIM simulator indicates if an attempt was made in steps a) and c) to store the messages and if the messages are stored according to the requirement.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	5	SS -> MS	AUTHENTICATION REQ
	6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	9	SS		SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU), Class 2 Short Message
	13	MS -> SS	CP-ACK
	14	ME		The ME shall correctly store the short message 
				in a free record of EFSMS in the SIM, i.e.
				-	the ME shall use a free record
				-	the first byte of the record
					shall indicate "message received by MS
					from network"
				-	the TS-Service-Centre-Address shall be
					correctly stored
				-	the TPDU shall be identical
					to that sent by the SS
				-	bytes following the TPDU
					shall be set to 'FF'
	15	SIM		The SIM simulator returns the status response 
				"OK" ('90 00'). The SIM simulator shall indicate 
				if an attempt was made by the ME to store the
				short message in the SIM.
	16	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	17	SS -> MS	CHANNEL RELEASE 
	18	SS -> MS	PAGING REQUEST		
	19	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	20	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	21	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	22	SS -> MS	AUTHENTICATION REQ
	23	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	24	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the 
				message.
	25	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	26	SS		SS starts ciphering.
	27	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	28	MS -> SS	UA (SAPI=3) 
	29	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU), Class 2 Short Message

30	MS -> SS	CP-ACK
	31	ME		The ME shall attempt to store the short 
				message in a free record of EFSMS in the SIM.
	32	SIM		The SIM simulator returns the status response 
				"memory problem" ('92 40'). The SIM simulator 
				shall indicate if an attempt was made by the ME 
				to store the short message in the SIM.
	33	MS -> SS 	CP-DATA	Contains RP-ERROR RPDU with error cause
				"protocol error, unspecified".
	34	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

SMS-DELIVER TPDU (containing a class 2 message) (SS to MS):

Information element	Comment	Value

TP-MTI	SMS-DELIVER	‘00’B

TP-MMS	more messages are waiting in SC	‘0’B

TP-RP	no reply path	‘0’B

TP-UDHI	TP-UD contains only the SM	‘0’B

TP-SRI	no status report returned	0

TP-OA	an international number coded E.164

TP-PID	default	‘00000000’B

TP-DCS	default alphabet, class 2	‘1111 0010’B

TP-SCTS	any legal value (cf. GSM 03.40)

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters)

#### 34.2.5.4	Test of class 3 short messages

For further study

### 34.2.6	Test of short message type 0

For further study

### 34.2.7	Test of the replace mechanism for SM type 1-7

34.2.7.1	Definition and applicability

This test shall apply to MSs which support

-	Replace Short Messages and

-	display of received Short Messages.

34.2.7.2	Conformance requirement

On receipt of a short message, the MS shall check to see if the associated Protocol Identifier contains a Replace Short Message Type code. If such a code is present, then the MS will check the associated SC address (RP-OA) and originating address (TP-OA) and replace any existing stored message having the same Protocol Identifier code, SC address and originating address with the new short message.

Reference(s)

GSM 03.40; 9.2.3.9

34.2.7.3	Test purpose

This procedure verifies the correct implementation of the replace mechanism for Replace Short Messages.

34.2.7.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	two different numbers n and m are drawn randomly between 1 and 7.  Two different addresses for TP-Originating-Address (TPOA1 and TPOA2) are drawn.  Two different addresses for RP-Originating-Address (RPOA1 and RPOA2) are drawn.

b)	the SS delivers a short message to the MS as specified in section 34.2.1 step a).  In the SMS-DELIVER TPDU, the TP-Protocol-Identifier parameter is "Replace Short Message Type n", the TP-Originating-Address is TPOA1, and the RP-Originating-Address is RPOA1.

c)	step b) is repeated but with a different TP-Originating-Address (TPOA2), and different contents of TP-User-Data in the SMS-DELIVER TPDU.  The other parameters are the same as in step b).

d)	step c) is repeated but with RPOA2 in the RP-Originated-Address, and contents of TP-User-Data different from the former two messages.  The other parameters are the same as in step c).

e)	step d) is repeated but with the TP-Protocol-Identifier equal to "Replace Short Message Type m", and contents of TP-User-Data different from the former three messages.  The other parameters are the same as in step d).

f)	step e) is repeated but the contents of TP-User-Data are different from that used in step e).

g)	the SS prompts the operator to display the Short Messages stored in the MS.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	5	SS -> MS	AUTHENTICATION REQ
	6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	9	SS		SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
 				TPDU)
				TP-PID is "Replace Short Message
				Type n", TP-OA is TPOA1 and
 				RP-OA is RPOA1
	13	MS -> SS	CP-ACK
	14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	15	SS -> MS	CHANNEL RELEASE 


	16	SS -> MS	PAGING REQUEST	
	17	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	18	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	19	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	20	SS -> MS	AUTHENTICATION REQ
	21	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	22	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	23	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	24	SS		SS starts ciphering.
	25	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	26	MS -> SS	UA (SAPI=3) 
	27	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)
				TP-PID is "Replace Short Message Type n", 
				TP-OA is TPOA2 and RP-OA is RPOA1, 
				TP-UD different from step 12
	28	MS -> SS	CP-ACK
	29	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	30	SS -> MS	CHANNEL RELEASE 


	31	SS -> MS	PAGING REQUEST	
	32	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	33	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	34	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	35	SS -> MS	AUTHENTICATION REQ
	36	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	37	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	38	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	39	SS		SS starts ciphering.
	40	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3

41	MS -> SS	UA (SAPI=3) 
	42	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)
				TP-PID is "Replace Short Message Type n",
				TP-OA is TPOA2 and RP-OA is RPOA2, TP-UD
				different from step 12 and 27
	43	MS -> SS	CP-ACK
	44	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	45	SS -> MS	CHANNEL RELEASE 


	46	SS -> MS	PAGING REQUEST
	47	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	48	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	49	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	50	SS -> MS	AUTHENTICATION REQ
	51	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	52	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	53	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	54	SS		SS starts ciphering.
	55	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	56	MS -> SS	UA (SAPI=3) 
	57	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU) TP-PID is "Replace Short Message
				Type m", TP-OA is TPOA2 and RP-OA is
				RPOA2, TP-UD different 
				from step 12, 27 and 42
	58	MS -> SS	CP-ACK
	59	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	60	SS -> MS	CHANNEL RELEASE 

	61	SS -> MS	PAGING REQUEST	
	62	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	63	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	64	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	65	SS -> MS	AUTHENTICATION REQ
	66	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	67	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	68	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	69	SS		SS starts ciphering.
	70	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	71	MS -> SS	UA (SAPI=3) 
	72	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER
				TPDU)
				TP-PID is "Replace Short Message
 				Type m", TP-OA is TPOA2 and RP-OA is RPOA2, 
				TP-UD different from step 57
	73	MS -> SS	CP-ACK
	74	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	75	SS -> MS	CHANNEL RELEASE 

	76	SS		Prompts the operator to display the
				Short Messages stored in the MS. Only the
				Short Messages delivered in step 12, 27, 42 and
				72 shall be retrievable and displayed

Specific Message Contents:

SMS-DELIVER TPDU

Information element	Comment	Value

TP-MTI	SMS-DELIVER	‘00’B

TP-MMS	no more messages are waiting in SC	‘1’B

TP-RP	no Reply Path	‘0’B

TP-UDHI	TP-UD contains only the SM	‘0’B

TP-SRI	no Status Report returned	‘0’B

TP-OA	an international number coded E.164 (see test method 	description)

TP-PID	binary 01000xxx, xxx represents n resp. m (see test method 	description)

TP-DCS	default alphabet	‘00000000’B

TP-SCTS	the time when the message was submitted according to 
	GSM 03.40

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters) (see test method 	description)

### 34.2.8	Test of the reply path scheme

34.2.8.1	Definition and applicability

This test applies to MSs which support

-	reply procedures (the class of MSs for which this is mandatory is described in GSM 03.40, Annex 4)

-	displaying of received Short Messages and

-	submitting Short Messages.

Steps b) and d) are only executed for MSs which support storing of Short messages.

34.2.8.2	Conformance requirement

When a replying MS receives an original mobile terminated short message it has:

originating SME = TP-Originating Address in the SMS-DELIVER TPDU

original SC = RP-Originating Address in the RP-MT-DATA

When submitting the reply mobile originated short message, the replying MS should use parameters as follows:

TP-Destination Address in SMS-SUBMIT TPDU = originating SME

RP-Destination Address in RP-MO-DATA = original SC

Reference(s)

GSM 03.40 Annex 4, sect. 5, 6.

34.2.8.3	Test purpose

This procedure verifies that the MS is able to send a Reply Short Message back to the correct originating SME even if in the meantime it receives another Short Message.

34.2.8.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Support for Short message MO/PP

Description of the basic procedures to display a mobile terminated short message

Description of the basic procedures to send a mobile originated short message

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message as specified in section 34.2.1, step b) with TP-Reply-Path set to 1.

b)	step a) is repeated but with

-	different TP-Originating-Address for the originating SME

-	different RP-Originating-Address for the original SC and

-	different message contents TP-User-Data.

c)	one of the two Short Messages is displayed (e.g. by means of the MMI) and the Reply Short Message is submitted (e.g. by means of the MMI).

d)	step c) is repeated for the other Short Message.

Maximum Duration of Test

-

Expected Sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	
	2	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	3	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	4	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	5	SS -> MS	AUTHENTICATION REQ
	6	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	7	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	8	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages shall be sent enciphered.
	9	SS		SS starts ciphering.
	10	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	11	MS -> SS	UA (SAPI=3) 
	12	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)
				TP-RP set to 1
	13	MS -> SS	CP-ACK	Sent within TC1M after step 12
	14	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	14A	SS -> MS	CP-ACK
	15	SS -> MS	CHANNEL RELEASE 
	16	SS -> MS	PAGING REQUEST	
	17	MS -> SS	CHANNEL REQUEST	Establishment cause is "Answer to paging"
	18	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	19	MS -> SS	PAGING RESPONSE	Message is contained in SABM.
	20	SS -> MS	AUTHENTICATION REQ
	21	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	22	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the
				message.
	23	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following
				messages
				shall be sent enciphered.
	24	SS		SS starts ciphering.
	25	SS -> MS	SABM (SAPI=3)	SS establishes SAPI 3
	26	MS -> SS	UA (SAPI=3) 
	27	SS -> MS	CP-DATA	Contains RP-DATA RPDU (SMS DELIVER TPDU)
				TP-OA, RP-OA and TP-UD different from step 12
	28	MS -> SS	CP-ACK	Sent within TC1M after step 12
	29	MS -> SS 	CP-DATA	Contains RP-ACK RPDU. 
	29A	SS -> MS 	CP-ACK
	30	SS -> MS	CHANNEL RELEASE 
	31	MS		One of the two Short Messages is displayed
				and the Reply Short Message is submitted.
	32	MS -> SS	CHANNEL REQUEST	
	33	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	34	MS -> SS	CM SERVICE REQ	Message is contained in SABM.
	35	SS -> MS	AUTHENTICATION REQ
	36	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	37	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the message.
	38	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following messages
				shall be sent enciphered.
	39	SS		SS starts ciphering.
	40	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	41	SS -> MS	UA (SAPI=3) 
	42	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT TPDU)
				RP-DA = RP-OA corresponding to the
				message displayed
				TP-DA = TP-OA corresponding to the
				message displayed

43	SS -> MS	CP-ACK	Sent within TC1M after step 42
	44	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	45	SS		Waits max 25 seconds for CP-ACK
	46	MS -> SS	CP-ACK	
	47	SS -> MS	CHANNEL RELEASE	The main signalling link is released.


	48	MS		The other Short Message is displayed and the 
				Reply Short Message is submitted.
	49	MS -> SS	CHANNEL REQUEST	
	50	SS -> MS	IMMEDIATE ASSIGNMENT	SS assigns an SDCCH
	51	MS -> SS	CM SERVICE REQ	Message is contained in SABM.
	52	SS -> MS	AUTHENTICATION REQ
	53	MS -> SS	AUTHENTICATION RESP	SRES specifies correct value.
	54	SS -> MS	CIPHER MODE COMMAND	SS starts deciphering after sending the 
				message.
	55	MS -> SS	CIPHER MODE COMPLETE	Shall be sent enciphered. All following 
				messages shall be sent enciphered.
	56	SS		SS starts ciphering.
	57	MS -> SS	SABM (SAPI=3)	MS establishes SAPI 3
	58	SS -> MS	UA (SAPI=3) 
	59	MS -> SS	CP-DATA	Contains RP-DATA RPDU (SMS SUBMIT TPDU)
				RP-DA = RP-OA corresponding to the
				Message displayed
				TP-DA = TP-OA corresponding to the
				message displayed
	60	SS -> MS	CP-ACK	Sent within TC1M after step 59
	61	SS -> MS	CP-DATA	Contains RP-ACK RPDU
	62	SS		Waits max 25 seconds for CP-ACK
	63	MS -> SS	CP-ACK	
	64	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

SMS-DELIVER TPDU

Information element	Comment	Value

TP-MTI	SMS-DELIVER	‘00’B

TP-MMS	no more messages are waiting in SC	‘1’B

TP-RP	Reply Path exists	‘1’B

TP-UDHI	TP-UD contains only the SM	‘0’B

TP-SRI	no Status Report returned	‘0’B

TP-OA	an international number coded E.164 (see test method 	description)

TP-PID	default	‘00000000’B

TP-DCS	default alphabet	‘00000000’B

TP-SCTS	the time when the message was submitted according to 	GSM 03.40

TP-UDL		160

TP-UD (140 octets)	text of message (160 characters) (see test method 	description)

## 34.3	Short message service cell broadcast

This test only applies to MSs which support SMS-Cell Broadcast.

34.3.1	Conformance requirements

The MS is responsible for recombination of the four blocks received via the radio path into a single block which constitutes the cell broadcast short message.

In idle mode, the MS listens to the BCCH and to the paging sub-channel for the paging group it belongs to. The MS is required to receive and analyse the paging messages and immediate assignment messages sent on the paging subchannel corresponding to its paging subgroup.

Reference

GSM 03.41; 8

GSM 04.08; 3.2.1, 3.3.2.1

34.3.2	Test purpose

This test verifies that an MS is able to receive SMS-CB messages.

This test verifies that an MS is able to respond to a paging requested during the transmission of a cell broadcast short message.

34.3.3	Test method

Initial conditions

System Simulator:

1 cell, default parameters

The SS provides a BCCH/CCCH to support the MS in idle mode.

Periodic location updating is disabled.

Mobile Station:

The MS shall be in the idle updated state.

Related PICS/PIXIT Statements

Support for short message transmission cell broadcast

Description of the basic procedures to display a cell broadcasted short message

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	Three Cell Broadcast (CB) messages are sent by the SS on the CBCH with serial numbers 0,1,1.

b)	Step a) is repeated, but the SS pages the MS during the transmission of the second CB message.

The MS shall respond to the page.

Maximum Duration of Test

-

Expected Sequence

Since the SMS-CB messages are sent continuously, a table is not applicable in this test.

Specific Message Contents:

Cell broadcast test message content

Information element	Comment	Value

Serial Number

- Geographical scope		‘00’B

- Message code	see test procedure	‘0000000000’B

or ‘0000000001’B

- Update number		as applicable

Message identifier		‘0’B

Data Coding Scheme	Default alphabet, English	‘00000000’B

Page parameter		‘0001 0001’B

Contents of message	93 user characters

using 93 different characters

of default 7 bit

coded alphabet

SYSTEM INFORMATION TYPE 4

As default except:

Information element				Value/remark

CBCH Channel Description

- Channel type and TDMA offset		SDDCH/4 + SACCH/C4 or CBCH (SDDCH/4)

- Timeslot number				time slot zero

- Training sequence code			5 (same as BCC)

- Hopping channel				Single RF channel

-Channel selector				Channel number 20 (for GSM900 MS)

Channel number 590 (for DCS1800 MS)

CBCH Mobile Allocation				Empty

## 34.4		Default message contents:

CP-DATA (including RP-DATA  SS->MS)

Protocol Discriminator				SMS messages (‘1001’B)

Transaction Identifier

TI value					any value from the set {0, ...., 6}

TI flag					0

Message type					00000001

Cp-User data

length

RP-DATA				max 255 octets

RP-Message Type			001 (RP-DATA  SS->MS)

RP-Message Reference		see 04.11  8.2.3

RP-Originator Address		see 04.11  8.2.5.1

RP-Destination Address		length indicator set to 0

RP-User Data

length

TP-DATA		max 238 octets

CP-DATA (including RP-DATA  MS->SS)

Protocol Discriminator				SMS messages (‘1001’B)

Transaction Identifier

TI value					any value from the set {0, ...., 6}

TI flag					0

Message type					00000001

Cp-User data

length

RP-DATA				max 255 octets

RP-Message Type			000 (RP-DATA  MS->SS)

RP-Message Reference		see 04.11  8.2.3

RP-Originator Address		length indicator set to 0

RP-Destination Address		see 04.11  8.2.5.2

RP-User Data

length

TP-DATA		max 238 octets

CP-DATA (including RP-ACK  MS->SS)

Protocol Discriminator				SMS messages (‘1001’B)

Transaction Identifier

TI value

TI fllag

Message type					00000001

Cp-User data

length

RP-ACK

RP-Message Type			010 (RP-ACK  MS->SS)

RP-Message Reference		see 04.11  8.2.3

CP-DATA (including RP-ACK  SS->MS)

Protocol Discriminator				SMS messages (‘1001’B)

Transaction Identifier

TI value

TI flag

Message type					00000001

Cp-User data

length

RP-ACK

RP-Message Type			011 (RP-ACK  SS->MS)

RP-Message Reference		see 04.11  8.2.3

CP-DATA (including RP-ERROR  MS->SS)

Protocol Discriminator				SMS messages (‘1001’B)

Transaction Identifier

TI value

TI flag

Message type					00000001

Cp-User data

length

RP-ERROR

RP-Message Type			100 (RP-ERROR  MS->SS)

RP-Message Reference		see 04.11  8.2.3

CP-DATA (including RP-ERROR  SS->MS)

Protocol Discriminator				SMS messages (‘1001’B)

Transaction Identifier

TI value

TI flag

Message type					00000001

Cp-User data

length

RP-ERROR

RP-Message Type			101 (RP-ERROR  SS->MS)

RP-Message Reference		see 04.11  8.2.3

CP-DATA (including RP-SMMA  MS->SS)

Protocol Discriminator				SMS messages (‘1001’B)

Transaction Identifier

TI value

TI flag

Message type					00000001

Cp-User data

length

RP-SMMA

RP-Message Type			110 (RP-SMMA  MS->SS)

RP-Message Reference		see 04.11  8.2.3

CP-ACK

Protocol Discriminator				SMS messages (‘1001’B)

Transaction Identifier

TI value

TI flag

Message type					00000100

CP-ERROR

Protocol Discriminator				SMS messages (‘1001’B)

Transaction Identifier

TI value

TI flag

Message type					00010000

Cp-Cause

Cause value				see 04.11  8.1.4.2

# 35	Low battery voltage detection

35.1	Definition and applicability

Low battery or shutdown voltage detection is used to trigger inhibition of all RF transmission before the MS supply voltage reaches a level where effective use of the radio frequency spectrum is no longer guaranteed.

The requirements and this test apply to all types of GSM900 and DCS1800 MS.

35.2	Conformance requirement

1.	The MS shall not make ineffective use of the radio frequency spectrum. In no case shall the MS exceed the transmitted levels as defined in GSM 05.05 for extreme operation.

GSM 05.05, Annex D.2.2

2.	The MS shall inhibit all RF transmission when the power supply voltage is below the manufacturer declared approximate shutdown voltage.

GSM 05.05, Annex D.2.2

35.3	Test purpose

1.	To verify that the MS does not make ineffective use of the RF spectrum.

2.	To verify that the MS inhibits all RF transmission when the battery voltage falls below the manufacturer declared shutdown level.

35.4	Method of test

35.4.1	Initial conditions

The SS transmits a BCCH with a location updating time set to 0.1 hours.

The SS sends a paging request message to the MS.

The MS responds with a channel request message.

The SS sends an immediate assignment message establishing an SDCCH.

35.4.2	Procedure

a)	The SS gradually reduces the power supply voltage until the MS ceases the production of RF output.

The RF output spectrum shall be monitored for any anomalies while the supply voltage is being reduced.

NOTE 1:	The declared approximate shutdown voltage gives an indication of the voltage where the MS will cease RF output.

NOTE 2:	If any anomalies occur, then additional testing using the transmitter tests at the voltage where the anomaly occurred is performed to determine in an objective manner, whether or not the conformance requirement is met.

c)	After 7 minutes, the SS sends a paging message to the MS.

d)	The SS observes whether or not the MS produces any RF output.

This measurement is performed over the relevant  transmit band.

The spectrum analyser is set to:

Bandwidth: 3 MHz

Peak Hold

e)	The SS modifies the location area of the BCCH.

f)	For 7 minutes, the SS observes whether or not the MS produces any RF output.

NOTE 3:	It is anticipated that the MS might attempt location updating.

g)	The MS is switched off and on.

h)	The SS pages the MS.

i)	The SS observes whether or not the MS produces any RF output.

35.5	Test requirement

1.	In step a) no anomalies shall occur.

2.	In step a), the MS shall cease the production of RF output.

3.	In steps d), f) and i), the MS shall not produce any RF output above -30 dBm.

# 36	Individual equipment type requirements and interworking - special conformance testing functions

## 36.1	General

This chapter specifies those ME functions which are required for conformance testing purposes only. However, except for the Electrical Man Machine Interface (EMMI), they are required for every mobile station.

For conformance tests, functions are activated via the radio interface, test SIM or dedicated pins. These functions must be capable of being activated when a test SIM is present but must not function with any other (e.g. network) SIM present. In this state, the MS must be able to perform all functions specified in this MS conformity specification; in addition however, the special conformance testing functions must be operational.

The special conformance testing functions of the ME are enabled by use of a dedicated Subscriber Identity Module (test SIM, see Annex 4). SIM, in general, is described in GSM 11.11. The ME recognizes the test SIM by the Administrative Data Field.

## 36.2	Activation and deactivation of special test functions in the MS

These functions can be activated and deactivated from a SS by sending appropriate layer 3 commands to the MS. The protocol discriminator to be used is defined in GSM 04.08, section 10.2.

The layer 3 commands are sent on the DCCH. On layer 2, SAPI 0 is used in acknowledged mode.

Apart from sending the appropriate deactivation command to the MS the functions can be deactivated by switching off the MS or removing the test SIM.

The following test functions can be activated (and deactivated):

-	TCH Loop,

-	TCH Burst-by-Burst Loop,

-	Electrical MMI,

-	Test via DAI.

The TCH loops and the test via DAI are test functions which are mutually exclusive.

### 36.2.1	Internal test loops

A number of internal test loops are required providing access to isolated functions of the MS without introducing new physical interfaces just for the reason of type approval testing. Fig 36-1 shows a functional block diagram of a reference MS containing the different test loops.

NOTE 1:	It should be emphasized that these test loops only describe the functional behaviour of the MS with respect to its external interfaces; physical implementation of the loops is completely left open to the manufacturer.

A particular loop is activated in an MS by transmitting the appropriate command message to the MS.

Figure 36-1: Test loops in the MS

#### 36.2.1.1	TCH loop

Purpose:

To establish a transparent loop for TCH blocks. A TCH must be active between SS and MS. The TCH may be full or half rate, speech or data of any rate specified in the GSM system.

Six types of TCH loop back are defined.

The first (A) includes the signalling of erased frames and is used to determine Frame Erasure Ratio (FER) and Residual Bit Error Ratio (RBER) for speech TCH and Bit Error Ratio (BER) for any data TCH.

The second type (B) is required to determine Class II bit error ratio for the speech TCH.

With the third loop (C) the 114 information bits of each TCH burst (excluding stealing flags) prior to applying benefit of the channel decoder, but after decryption, shall be transmitted in an uplink burst. (Equivalent error rate to TCH/FS Class II). All that is received shall be re-transmitted regardless of the state of the received midamble. The midamble in the uplink bursts shall be the normal midamble used by the MS. SACCH and idle bursts are not looped back.

The fourth loop (D) includes the signalling of erased frames and unreliable frames and is used to determine Unreliable Frame Ratio (UFR) and Residual Bit Error Ratio (RBER) for TCH/HS.

The fifth loop (E) includes the signalling of erased SID frames and is used to determine Erased SID Frame Rate (ESIDR) and Residual Bit Error Ratio (RBER) for TCH/HS.

The sixth loop (F) includes the signalling of erased valid SID frames and is used to determine Erased Valid SID Frame Rate (EVSIDR) and Residual Bit Error Ratio (RBER) for TCH/HS.

NOTE 2:	Measurement of TCH/FS chip BER is approximately five times faster using loop C rather than loop B.

##### 36.2.1.1.1	TCH loop including signalling of erased frames (A)

Procedure:

The SS orders the MS to close its TCH loop by transmitting a CLOSE_TCH_LOOP_CMD message, specifying the TCH to be looped and that erased frames are to be signalled by the MS. The SS then starts timer TT01.

If no TCH is active, or any test loop is already closed, the MS shall ignore any CLOSE_TCH_LOOP_CMD message.

If a TCH is active, the MS shall close its TCH loop for the TCH specified and send back to the SS a CLOSE_TCH_LOOP_ACK message. Upon reception of that message the SS stops timer TT01.

After the MS has closed its TCH loop, every good speech frame or any user data frame received by the MS on the specified TCH (downlink) shall be taken from the output of the channel decoder, input to the channel encoder and transmitted on the same TCH (uplink).

If the channel decoder detects a bad speech frame or if the MS decodes the stealing flags as indicating an FACCH frame, then this shall be signalled to the SS by setting the input frame to the channel encoder to zero's, and transmitting on the TCH (uplink).The FACCH channel shall operate normally.

##### 36.2.1.1.2	Speech TCH loop without signalling of erased frames (B)

Procedure

The SS orders the MS to close its TCH loop by transmitting a CLOSE_TCH_LOOP_CMD message, specifying the TCH to be looped. The SS then starts timer TT01.

If no TCH is active or any test loop is already closed, the MS shall ignore any CLOSE_TCH_LOOP_CMD message.

If a TCH is active, the MS shall close its TCH loop for the TCH specified and send back to the SS a CLOSE_TCH_LOOP_ACK message. Upon reception of that message the SS stops timer TT01.

After the MS has closed its TCH loop, any speech frame received by the MS on the specified TCH (downlink) shall be taken from the output of the channel decoder, input to the channel encoder, and transmitted on the same TCH (uplink).

The SS should avoid using the FACCH downlink in this situation until the test is complete.

##### 36.2.1.1.3	TCH burst-by-burst loop (C)

Applicability:	The test loop shall be implemented by all ME, supporting any TCH.

Procedure

Establishment and clearing of the loop is performed at ideal radio conditions.

Establishment:

-	The establishment shall be commanded by transmitting a CLOSE_TCH_LOOP_CMD message. The SS then starts timer TT01. This command shall be acknowledged by the MS with a CLOSE_TCH_LOOP_ACK message. Upon receipt of that message the SS stops timer TT01. The MS shall establish the loop within one reporting period [SACCH-block = 104 frames] from the sending of the CLOSE_TCH_LOOP_ACK.

-	If no TCH is active or any test loop is already closed, the MS shall ignore any CLOSE_TCH_LOOP_CMD message.

Operation:

-	The round trip delay (RTD), which is the number of frames between the reception of one burst at the MS, and the transmission of the same burst (on the uplink) shall be less than 26 TDMA frames. The actual value shall be declared for the implementation to be tested.

NOTE 3:	The RTD can be as long as required to receive the number of interleaved burst for the relevant TCH.

##### 36.2.1.1.4	TCH loop including signalling of erased frames and unreliable frames (D)

Procedure:

The SS orders the MS to close its TCH loop by transmitting a CLOSE_TCH_LOOP_CMD message, specifying the TCH to be looped and that erased frames and unreliable frames are to be signalled by the MS. The SS then starts timer TT01.

If no TCH is active, or any test loop is already closed, the MS shall ignore any CLOSE_TCH_LOOP_CMD message.

If a TCH is active, the MS shall close its TCH loop for the TCH specified and send back to the SS a CLOSE_TCH_LOOP_ACK message. Upon reception of that message the SS stops timer TT01.

After the MS has closed its TCH loop, every reliable speech frame (UFI = 0) received by the MS on the specified TCH/HS (downlink) shall be taken from the output of the channel decoder, input to the channel encoder and transmitted on the same TCH (uplink).

If the channel decoder detects a bad speech frame or an unreliable frame (BFI = 1 or UFI = 1) or if the MS decodes the stealing flags as indicating an FACCH frame, then this shall be signalled to the SS by setting the input frame to the channel encoder to zero's, and transmitting on the TCH/HS (uplink).The FACCH channel shall operate normally.

##### 36.2.1.1.5	TCH loop including signalling of erased SID frames (E)

Procedure:

The SS orders the MS to close its TCH loop by transmitting a CLOSE_TCH_LOOP_CMD message, specifying the TCH to be looped and that erased SID frames are to be signalled by the MS. The SS then starts timer TT01.

If no TCH is active, or any test loop is already closed, the MS shall ignore any CLOSE_TCH_LOOP_CMD message.

If a TCH is active, the MS shall close its TCH loop for the TCH specified and send back to the SS a CLOSE_TCH_LOOP_ACK message. Upon reception of that message the SS stops timer TT01.

After the MS has closed its TCH loop, every valid SID frame (SID = 2) or invalid SID frame (SID = 1) received by the MS on the specified TCH/HS (downlink), shall be taken from the output of the channel decoder, input to the channel encoder and transmitted on the same TCH/HS (uplink).

If the channel decoder detects an erased SID frame (SID = 0), then this shall be signalled to the SS, by setting the input frame to the channel encoder to zero's, and transmitting on the TCH/HS (uplink).

If the MS decodes the stealing flags as indicating an FACCH frame, then this shall be signalled to the SS by setting the input frame to the channel encoder to zero's, and transmitting on the TCH/HS (uplink).The FACCH channel shall operate normally.

##### 36.2.1.1.6	TCH loop including signalling of erased valid SID frames (F)

Procedure:

The SS orders the MS to close its TCH loop by transmitting a CLOSE_TCH_LOOP_CMD message, specifying the TCH to be looped and that erased valid SID frames are to be signalled by the MS. The SS then starts timer TT01.

If no TCH is active, or any test loop is already closed, the MS shall ignore any CLOSE_TCH_LOOP_CMD message.

If a TCH is active, the MS shall close its TCH loop for the TCH specified and send back to the SS a CLOSE_TCH_LOOP_ACK message. Upon reception of that message the SS stops timer TT01.

After the MS has closed its TCH loop, every valid SID frame (SID = 2 and BFI = 0) received by the MS on the specified TCH/HS (downlink), shall be taken from the output of the channel decoder, input to the channel encoder and transmitted on the same TCH/HS (uplink).

If the channel decoder detects an erased valid SID frame (SID = 1) or (SID = 0) or ((BFI or UFI) = 1)), then this shall be signalled to the SS by setting the input frame to the channel encoder to zero's, and transmitting on the TCH/HS (uplink).

If the MS decodes the stealing flags as indicating an FACCH frame, then this shall be signalled to the SS by setting the input frame to the channel encoder to zero's, and transmitting on the TCH/HS (uplink).The FACCH channel shall operate normally.

##### 36.2.1.1.7	Deactivating TCH loops

The SS orders the MS to open any TCH loop by transmitting an OPEN_LOOP_CMD message.

If no loop is closed the MS shall ignore any OPEN_LOOP_CMD message.

If a TCH is looped, the MS shall open the loop.

If the loop opened was type C, the MS shall send an OPEN_LOOP_CMD message to the SS with bit 0 of the optional acknowledgement element set to 1.

All channels shall be open for normal use again.

##### 36.2.1.1.8	Additional non-mandatory operating characteristics

In order to optimise the speed and flexibility of mobile manufacturing and repair, the following non-mandatory characteristics of the test loops are suggested:

The normal FACCH downlink and uplink functions should ideally be maintained when the test loop is closed. In particular, channel assignments or handovers, and call termination from either the mobile or the base station simulator.

Following an assignment or handover, the loop should not open if it was closed prior to the handover.

Following call dropping or deliberate call termination, the loop should be re-opened.

The loopback functions should ideally operate with or without (i.e. no SIM) the test SIM present, but should not operate with a network SIM present.

Audio muting should be enabled when the loop is closed.

### 36.2.2	Activating and deactivating EMMI

Activating EMMI requires the presence of a test SIM. EMMI shall be activated by any of the following:

-	switching on the MS

-	inserting a test SIM

-	layer 3 message on the radio interface (ACT_EMMI_CMD).

When the MS is ready to receive frames, it shall send one XON message.

EMMI shall be deactivated by any of the following:

-	switching off the MS

-	removing the test SIM

-	layer 3 message on the radio interface (DEACT_EMMI).

NOTE 4:	No XOF shall be sent after deactivation.

The L3 message used on the radio interface to activate the EMMI is the activation command ACT_EMMI_CMD (see 36.2.4.4). This message has to be acknowledged by the message ACT_EMMI_ACK on the radio interface sent by the MS (see 36.2.4.5).

For deactivation of the EMMI in the MS through the radio interface, the message DEACT_EMMI is defined in 36.2.4.6. An acknowledgement of this message is not required.

### 36.2.3	Activating and deactivating DAI tests

Purpose: to determine the routing of speech data (DAI or internal, i.e. normal mode) and which device is being tested (speech transcoder / DTX functions or A/D & D/A).

Prerequisites:	a dedicated channel must be established if the manufacturer has stated that the DAI is activated by means of the layer 3 message.

Procedure:	the SS sends a TEST_INTERFACE message if the manufacturer has stated that the DAI is activated by means of the layer 3 message or applies the appropriate control signal on the DAI if the manufacturer has declared that the DAI is activated this way.

When the test mode is established i.e. speech data comes from test interface, each new test function overrides the previous one.

### 36.2.4	Message definitions and contents

NOTE 5:	For all messages the protocol discriminator (PD) 111 (binary) is used.

NOTE 6:	For definition of "Presence" and "Format", see GSM 04.07 sections 11.4 and 11.1.1.

#### 36.2.4.1	CLOSE_TCH_LOOP_CMD

| Information element | Reference | Type | Length |
| --- | --- | --- | --- |
| Protocol discriminator | GSM 04.08 sect. 10.2 | MF |  |
| Transaction identifier | GSM 04.08 sect. 10.3 | MF | 2 |
| Message type |  | MF |  |
| Sub-channel |  | MF | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | octet 1 |

and Sub-channel is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | B | A | Z | Y | X | octet 1 |

X = 0	If there is only one TCH active (so there is no choice) or if sub-channel 0 of two half rate channels is to be looped.

X = 1	If sub-channel 1 of two half rate channels is to be used.

Y = 0	If the looped TCH is a speech channel then the frame erasure is to be signalled, type A.

Y = 1	If the looped TCH is a speech channel then frame erasure is not signalled, type B.

Z = 0	The type of the loop is determined by the value Y.

Z = 1	The Burst-by-Burst loop is activated, type C. The value of Y is disregarded.

A = 0 and B = 0	The loop is determined by the values Z, Y and X.

A = 1 and B = 0	If the looped TCH is a half rate speech channel then frame erasure and unreliable frames have to be signalled, type D. The values of Y and Z are disregarded.

A = 0 and B = 1	If the looped TCH is a half rate channel sending SID frames then SID frame erasure is to be signalled, type E. The values of Y and Z are disregarded.

A = 1 and B = 1	If the looped TCH is a half rate channel sending SID frames then valid SID frame erasure is to be signalled, type F. The values of Y and Z are disregarded.

#### 36.2.4.2	CLOSE_TCH_LOOP_ACK

Information Element	Reference	Presence	Format	Length

Protocol discriminator	GSM 04.08 sect. 10.2	M	V	1/2

Transaction identifier	GSM 04.08 sect. 10.3	M	V	1/2

Message type		M	V	1

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | octet 1 |

#### 36.2.4.3	OPEN_LOOP_CMD

NOTE 7:	OPEN_LOOP_ACKNOWLEDGE message is not mandatory for loops A and B.

Information Element	Reference	Presence	Format	Length

Protocol discriminator	GSM 04.08 sect. 10.2	M	V	1/2

Transaction identifier	GSM 04.08 sect. 10.3	M	V	1/2

Message type		M	V	1

Acknowledge		O	V	1

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | octet 1 |

where Acknowledge is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sp | Sp | Sp | Sp | Sp | Sp | Sp | 1 | octet 1 |

#### 36.2.4.4	Command for the activation of the EMMI, ACT_EMMI_CMD

Information Element	Reference	Presence	Format	Length

Protocol discriminator	GSM 04.08 sect. 10.2	M	V	1/2

Transaction identifier	GSM 04.08 sect. 10.3	M	V	1/2

Message type		M	V	1

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | octet 1 |

#### 36.2.4.5	Acknowledge of the activation of the EMMI, ACT_EMMI_ACK

Information Element	Reference	Presence	Format	Length

Protocol discriminator	GSM 04.08 sect. 10.2	M	V	1/2

Transaction identifier	GSM 04.08 sect. 10.3	M	V	1/2

Message type		M	V	1

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | octet 1 |

#### 36.2.4.6	Deactivation of the EMMI, DEACT_EMMI

Information Element	Reference	Presence	Format	Length

Protocol discriminator	GSM 04.08 sect. 10.2	M	V	1/2

Transaction identifier	GSM 04.08 sect. 10.3	M	V	1/2

Message type		M	V	1

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | octet 1 |

#### 36.2.4.7	Test_Interface

Information Element	Reference	Presence	Format	Length

Protocol discriminator	GSM 04.08 sect. 10.2	M	V	1/2

Transaction identifier	GSM 04.08 sect. 10.3	M	V	1/2

Message type		M	V	1

Tested device		M	V	1

Where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | octet 1 |

and Tested device is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | X | X | X | octet 1 |

Tested devices:

000 = normal operation (no tested device via DAI)

001 = test of speech decoder / DTX functions (downlink)

010 = test of speech encoder / DTX functions (uplink)

100 = test of acoustic devices and A/D & D/A.

All other values are reserved.

### 36.2.5	Timer values

TT01:	To be started when a CLOSE_TCH_LOOP_CMD is sent. To be stopped when the corresponding CLOSE_TCH_LOOP_ACK is received.

Recommended value: [2.5 seconds].

## 36.3	Electrical Man Machine Interface (EMMI)

### 36.3.1	Use of the EMMI

Conformity tests of mobile stations are made using the system simulator specified in Annex 4.

Test signals are sent on the Um interface, and actions of the MS are registered. The Electrical Man Machine Interface (EMMI) is a half duplex communication link between the SS and the MS by which it is possible to automatically register the status, indications and performance of the MS.

It is also possible to simulate actions normally made by the user on the keyboard of the MS.

### 36.3.2	Formal aspects

i)	The EMMI is optional for the ME.

ii)	The EMMI is mandatory for the SS.

iii)	If the EMMI is to be used in conformance testing of an MS, it shall be possible to connect the SS to a connector on the MS, or to an adapter connected to the same MS. If an adapter is to be used, it shall be provided by the manufacturer.

iv)	If the MS fulfils the requirements performed with the use of an EMMI, the MS is regarded as having passed that test.

v)	If the MS is rejected in a test performed with EMMI, the test shall be repeated on the same mobile with the device carrying the EMMI to the MS removed. The MS shall be regarded as fulfilling the requirements, if it then passes the test.

vi)	When using the EMMI, the MS does not necessarily conform to the RF requirements. Therefore, tests concerning Rx and Tx parameters on MS with integral antenna and cabinet radiation tests for all types of MS will never be performed with the use of the EMMI.

### 36.3.3	Layered structure of the interface

The definition of the EMMI is divided into three different layers. On layer 1 the use of a 25-pole socket with standard electrical characteristics for serial communication is defined. On layer 2, an extremely simple frame oriented protocol is defined. On layer 3, messages for control and verification of functions and indications are defined. Each layer is defined independently of surrounding layers, and is therefore easy to replace.

The EMMI protocol structure takes into account that the SS only sends and receives layer 3 frames when the corresponding step within a test case is to be performed.

### 36.3.4	Terminology

EMMI	Electrical Man Machine Interface

MI	Message Identifier

ME	Mobile Equipment

MS	Mobile Station

SS	System Simulator

Frame	Used on layer 2 to transfer messages to and from layer 3

Message	Information on layer 3

### 36.3.5	Description of the EMMI

#### 36.3.5.1	EMMI, Layer 1

##### 36.3.5.1.1	Mechanical and electrical characteristics

If implemented, the EMMI interface shall use the same connector as the Digital Audio Interface (DAI), described in section 36.4.3.

The pin assignments for the EMMI shall be as follows:

Pin	Function	Source

2	Transmitted data	SS
	3	Received data	MS
	7	EMMI signal ground

The electrical characteristics of the interface shall be as given in section 36.4.3.2.

##### 36.3.5.1.2	Transmission and reception characteristics

The EMMI uses asynchronous serial data transmission with 1 start bit (S), 8 data bits (D1 to D8), no parity and 1 stop bit (E).

S	D1	D2	D3	D4	D5	D6	D7	D8	E

Table 3 : Use of start and stop bits

The conditions on start and stop characters are defined in REC CCITT V.1.

The transmission rates are: 600, 1200, 2400, 4800, 9600 bits per second. The ME shall support at least one of these speeds.

The SS will adapt its rate (manually or by MMI) to this ME rate.

#### 36.3.5.2	EMMI, layer 2

##### 36.3.5.2.1	General structure

On layer 2, frames are used to carry data from higher layers. Frames consist of one or several octets. One frame with variable length is used to carry data from higher layers, and four other frames with the length of one character is used to control the flow of frames.

##### 36.3.5.2.2	Control frames

Special frames have been assigned to control the flow of frames on layer 2. They are only one octet long.

Abbr.	Meaning	Dec.value	Bit pattern

ACK	Acknowledge	06	0000 0110
	NAK	Not Acknowledge	21	0001 0101
	XON	Resume sending data	17	0001 0001
	XOF	Stop sending data	19	0001 0011

Table 4 : Characters for flow control

##### 36.3.5.2.3	Frame structure

Two octets, called characters, indicate start and stop of I-frames.

Abbr.	Meaning	Dec.value	Bit pattern

STX	Start of data	02	0000 0010
	ETX	End of data	03	0000 0011

Table 5 : Start and stop characters

Information from higher layers are transferred in I-frames with the following structure.

Field name	No of octet	Value	Start at octet no	Note

Start	1	Character STX	1	-
Length	1	Length of data	2	a1)
Data	0-255	Content of data	3	a2)
Check	1	Error detection	Length+2	a3)
End	1	Character ETX	Length+3

NOTES:

a1)	Length: The total number of data octets in the data field of the frame is calculated. The value shall be in the range of (0..255 decimal). The corresponding binary value is put into the length field.

a2)	Data: Data to and from higher layers are in the form of octets (groups of 8 digital bits).

a3)	Check: Longitudinal checksum is created by exclusive OR on all characters starting with the Start field and ending with the last octet before the Check field. The value, one octet, is mapped into the Check field.

##### 36.3.5.2.4	Flow of I-frames on layer 2

###### 36.3.5.2.4.1	Transmission of frames

A multiple frame starts with the first octet being the character STX and stops with the last octet with content ETX. The time between the start bits of two consecutive octets shall not be longer than T22.

| octet n |  | octet n + 1 |
| --- | --- | --- |
| < T22 > |  |  |

The time between two frames is measured from the start bit of the last octet of the nth frame to the start bit of the first octet of the n+1st frame. The time between two consecutive frames shall not be less than T23.

| STX |  | frame n |  |  | ETX |  |  | STX |  | frame n+1 |  |  | ETX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | < T23 > |  |  |  |  |  |  |  |  |

##### 36.3.5.2.4.2	Reception of frames

Start of a frame is defined as (more than T22*2 since previous reception of octet) AND (reception of STX).

End of a frame is defined as (reception of ETX in octet number (length+2)) OR (more than T22*2 since last reception of octet).

##### 36.3.5.2.4.3	Use of ACK and NAK on receiving side

ACK is used by the MS to acknowledge a frame on receiving side if all the following conditions are fulfilled. Otherwise NAK is used.

1)	The content of Start field is STX.

2)	The content of Check field corresponds to the exclusive OR sum of previous octets in the frame.

3)	The content of the last octet is ETX.

NAK is used by the SS to request retransmission of a frame. Otherwise ACK is used.

NOTE 8:	NAK shall not be used for layer 3 errors, if the layer 2 frame is correct. If the meaning of a layer 3 message is undefined or not performable, then the problem is solved with layer 3 messages.

##### 36.3.5.2.4.4	Use of XON and XOF

XON and XOF are used for flow control of layer 2.

XOF is sent if the unit (MS or SS) due to internal processing is not capable of receiving a following frame when a frame is being received.

XON is sent if XOF has been sent previously, but the reason for that XOF-transmission no longer exists.

##### 36.3.5.2.4.5	Parameters on layer 2

Bit rate	Value of T22	Value of T23

600	25.0 ms	58.3 ms
	1200	12.5 ms	29.2 ms
	2400	6.3 ms	14.6 ms
	4800	3.1 ms	7.3 ms
	9600	1.6 ms	3.6 ms

Table 7: Timer values on layer 2

#### 36.3.5.3	EMMI, layer 3

##### 36.3.5.3.1	Message structure

Messages are used on layer 3. They are defined by Message Identifiers (MI) in the range of (0..255). The message identifier is always the first, and often the only, octet of the message.

MI	Use

0-49	Not used.

50-179	General messages. All undefined values reserved for further evolution of the EMMI.

180-209	ME-type dependent blocks, may be used by the SS as a sender or receiver, if so requested by the manufacturer. Undefined values available for the manufacturer.

210-239	ME-type dependent blocks, never to be used by the SS in conformance testing. Undefined values available for the manufacturer.

240-255	Reserved for L3 error handling. All undefined values reserved for further evolution of the EMMI.

NOTE 9:	Layer 2 is transparent, but to avoid unnecessary interference from layer 3, MI with the same value as control frames on layer 2 are not used.

Table 8 : Use of message identifiers

Most of the messages, especially in the direction SS - MS contain only one octet, the message indicator. Some of the messages, especially in the direction MS - SS are quite long.

NOTE 10:	If the interface is limited only to the minimum required for automatic conformity testing with the electrical man machine interface, then the included L3 messages should be RQTI, KEYS, BEL1, BEL0, HOK1, HOK0, BCAP and RSTI.

Table 9: Block types

MI			Source

value	Abbr.	Meaning	MS	SS

051	VOL1	Increase volume (***)		X
	052	VOL0	Decrease volume (***)		X

	053	RQTS	Request for table, status		X
	054	RQTI	Request for table, indication		X
	055	RQPL	Request for power level		X

	056	RQBE	Request for bell status		X
	057	RQSM	Request for short message		X

	058	KEYS	Perform keystroke sequence		X

	060	BEL1	Indication user alert on	X
	061	BEL0	Indication user alert off	X

	064	HOK1	Hook on		X
	065	HOK0	Hook off		X

	070	BCAP	Selection of bearer capability		X

	080	STPO	Set power level		X

	091	RSTS	Response table, status	X
	092	RSTI	Response table, indication	X
	093	RSPO	Response, power level	X

	101	RXSM	Received short message	X
	102	RXSN	No short message received	X

	240	ER00	Internal malfunction detected	X
	241	ER01	L3 message not recognized	X	X
	242	ER02	L3 message not performable	X

	255	RESE	Perform hardware and software reset		X

NOTE 11:	Functioning of this should be verified, as the volume control in the ME might be of another type (non-incremental).

##### 36.3.5.3.2	Definition of messages

Messages are defined in the order of the value of the message identifier.

051 VOL1	Increase volume

052 VOL0	Decrease volume

Increase/decrease volume in the loudspeaker by one step.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



053 RQTS	Request for table, status

054 RQTI	Request for table, indication

055 RQPL	Request for power level

On receiving a request message RQxy, the corresponding table RSxy shall be sent.

8	7	6	5	4	3	2	1	Octet



                       	1	Message identifier



056 RQBE	Request for bell status

On receiving a request message RQBE, the internal user alert status (BEL1 or BEL0) shall be sent. BEL1 and BEL0 will indicate whether the ringing or alert procedure has been activated or not. (See also the definition of BEL1 and BEL0).

8	7	6	5	4	3	2	1	Octet



                       	1	Message identifier



057 RQSM	Request for short message

In response to the RQSM request the MS shall send either the short message type using the message RXSM or, in case of no short message received, the message RXSN.

8	7	6	5	4	3	2	1	Octet



                       	1	Message identifier



058 KEYS	Perform keystroke sequence

Perform the MS function related to the received keystroke sequence.

8	7	6	5	4	3	2	1	Octet



                       	1	Message identifier



                       	2	First key



                       



                       	N	Key number N-1



The possible keystroke sequences are based on the basic public man machine interface as defined in GSM 02.30. There exists a minimum set of key characters.

The codes associated with these characters are defined as follows:

Key	Code (decimal)

#	35

*	42

+	43

0	48

1	49

2	50

3	51

4	52

5	53

6	54

7	55

8	56

9	57

END (function)	18

SEND (function)	20

060 BEL1	Indication user alert on

061 BEL0	Indication user alert off

Indication user alert shall indicate, on request of the SS, the internal status of the alert or ringing procedure. For this purpose, the SS shall send the RQBE (request for bell status) message to the MS.

BEL1 shall indicate that the alert procedure is active.

BEL0 shall indicate that the alert procedure is not active.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



064 HOK1	Hook on

065 HOK0	Hook off

Control of the hook. The hook on/off command shall action the normal procedure associated with physically lifting the handset and replacing it whatever that maybe.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



070 BCAP	Selection of bearer capability

The EMMI BCAP message shall change the default bearer capability on all future calls made via the EMMI "perform keystroke sequence" message. The new default bearer capability shall be in effect until the ME is switched off or until the ME receives another EMMI BCAP message.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



	2	Bearer capability

		(see Note below)









NOTE 12:	The field "bearer capability" in the BCAP message is mandatory and is coded bit for bit exactly as the "bearer capability" information element as described in GSM 04.08 section 10.5.4.4, beginning with octet 2 (length of the bearer capability contents). Because the "bearer capability" is mandatory, the first byte of the field shall be the length of the bearer capability content and not the bearer capability information element identifier.

080 STPO	Set power level

Used to control the Tx power level of the ME.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



	2	Power level

		(see Note below)

NOTE 13:	The power level is defined as the 2nd octet of the power command information element in GSM 04.08.

091 RSTS	Response table, status

Response table status is sent as an answer to the corresponding request.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



	2	Status field

		(see Note below)





NOTE 14:

F1

0 0 0 0	Spare

F2	(1=yes, 0=no)

Bit 4	L2 link on SACCH established

Bit 3	Speech connection on TCH establ

Bit 2	Listening to BCCH

Bit 1	SDCCH established

F3

Bit 8	Frequency hopping (yes/no)

Bit 7-1	ARFCN of BCCH of serving cell

092 RSTI	Response table, indication

Response table indication send as an answer to the corresponding request.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



	2	Indication field

		(see Note below)

NOTE 15:

F1

0000000	Spare

F2	(yes=1, no=0)

Bit 1	Service indication On (yes/no)

093 RSPO	Response, power level

Response power level is sent as an answer to the corresponding request.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



	2	Power level

		(see Note below)

NOTE 16:	The power level is defined as the 2nd octet of the power command information element in GSM 04.08.

101 RXSM	Received short message

The message RXSM has to be sent by the MS in response to the RQSM request if a short message has been received.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



	2	SM-field octet 1

		(see note below)





	N	SM field octet N-1



NOTE 17:	The SM field is defined as the content of the data field short message storage defined in GSM 11.11 paragraph 3.5.1, excluding byte 1, as follows:

1	 byte	NULL value

12	 bytes	TP Originating Address

12	 bytes	TS Service Centre Address

1	 byte	TP Protocol Identifier

1	 byte	TP Data Coding Scheme

7	 bytes	TP Service Centre Time Stamp

1	 byte	TP User Data Length

up to 140	 bytes	TP User Data.

If the length of the address for TP and for TS service centre is less than 12 bytes, the remaining bytes are filled with null values.

102 RXSN	No short message received

The message RXSN has to be sent by the MS in response to the RQSM request if no short message has been received.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



240 ER00	Internal malfunction detected

Used to indicate to the SS that the MS has discovered an internal error. This error message is to be handled in the same manner as the layer 3 error messages (only to be sent to the SS in response to a Layer 3 message received at the MS).

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



	2	Error indicator

		(see Note below)

NOTE 18:	The error indicator is defined by the manufacturer of the ME.

241 ER01	L3 message not recognized

Used by the receiver to indicate to the sender that the message was correctly received, but ignored due to the fact that it was not understandable (e.g. a KEYS message with one or many values which are not part of the standard set of keys for the MSE). The message to the MS implies a request for re-sending the latest message sent.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



242 ER02	L3 message not performable

Used by the receiver to indicate to the sender that the message was understood, but not performable due to intentional lack of functionality in the MS (e.g. as answer to a CALL message containing a type of service which the MS is not able to use) or to a message requesting a change to a state already existent.

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



255 RESE	Perform HW and SW reset

The MS shall perform a total reset. The MS shall behave as if it has been switched "off" and "on".

8	7	6	5	4	3	2	1	Octet



	1	Message identifier



## 36.4	Digital audio interface

### 36.4.1	General

A special interface is required in order to perform the bit exact test of the speech coder/decoder and to test the SLR/RLR performance of the analogue and acoustic devices. It shall be possible to insert and extract speech data in both the transmit and receive directions. The interruption of the normal speech data paths can be commanded either by a layer 3 message over the air interface or by special control lines in the test interface. The MS need react to only one of these command methods. The manufacturer shall state which method is to be used.

### 36.4.2	Formal aspects

It shall be possible to connect the SS to the ME or to an adapter connected to the ME. If an adapter is to be used, it shall be provided by the manufacturer of the ME.

When using the DAI, the MS does not necessarily conform to all RF requirements.

When the DAI is connected the MS shall be able to correctly send and receive on a TCH and associated channels under ideal radio conditions.

NOTE 19:	Prior to tests of the speech coder, other functional entities involved in the tests, such as the channel codec or RF units must have been verified successfully.

### 36.4.3	Hardware aspect of the interface

The data exchanged on the interface are 13 bit linear PCM at 8000 samples per second, which, in order to keep the pin count low, are transferred on a duplex set of serial lines at 104 kbit/s.

One additional line resets the speech transcoder and the speech A/D and D/A functions. Two lines control the data flow direction and working mode of the interface, one mode being normal operation of the MS. These lines are controlled by the SS. Another line, controlled by the MS, clocks the data as required or available.

This is described in detail below.

#### 36.4.3.1	Mechanical characteristics of the interface

The interface shall use a 25-pin DSUB socket, detailed in the ISO 2110 document. The ME shall use a female part.

The manufacturer may provide this interface on an external test "adapter".

The pin assignment of the connector shall be as follows:

Pin	Use	Function	Source

1		Chassis ground	ME
	2-3	EMMI	signals
	4-6	Not used
	7	EMMI	Signal ground
	8-10	Not used
	11	DAI	Test control 1	SS
	12	DAI	Signal ground
	13	DAI	Test control 2	SS
	14-21	Not used
	22	DAI	Reset	SS
	23	DAI	Data	ME
	24	DAI	Data clock (104 kHz)	ME
	25	DAI	Data	SS

NOTE 20:	The EMMI interface is optional and is described in section 36.3.

#### 36.4.3.2	Electrical characteristics of the interface

The state of a signal pin is defined by the voltage (V) between the pin and its associated ground as follows:

Logical state	Voltage v

0 or "LOW" or "ON"	0 V	<	v	<	+ 0,8 V
	1 or "HIGH" or "OFF"	+ 3,5 V	<	v	<	+ 5 V
	undefined	+ 0,8 V	<	v	<	+ 3,5 V
	forbidden	v < 0 V,				 v > + 5 V

#### 36.4.3.3	Timing characteristics of the interface

The following timing applies:

Parameter	Value

Clock frequency	104 kHz +/- 20 ppm
Duty cycle	40 to 60 %
Clock rising edge time	< 1 microsecond
Clock falling edge time	< 1 microsecond
Reset pulse duration	>= 4 millisecond

|<-------------------- 9,6 s ------------------->|

|                                                 |

logical state   |                        |<--- 4,8 s +/-1 s --->|

1 or high       |                        |                        |

---------\  |<--- 4,8 s +/-1 s --->|  /------------------\  |

..............\.|........................|./....................\.|......90%

|\|                        |/|                     \|

..............|.\......................../.|......................\......50%

|  \                      /  |                       \

..............|...\..................../...|........................\....10%

|   |\                  /|   |                         \

|   | \----------------/ |   |                          \------

|   |                    |   |              logical state ^

|<>|                    |<>|               0 or  low 

falling                   rising

edge < 1s                edge < 1s

Data shall be stable during the period between 3 microseconds before and 1 microsecond after the rising edge of the clock (50% level).

### 36.4.4	Logical interface

The reset signal is active low.

The data consists of 13 bit words in two's complement format, with the most significant bit transmitted first.

Data are read in by the MS or SS at the rising edge and are output by the SS or MS at the falling edge of the clock, as defined in figure 36-2.

The clock signal is high when inactive.

The two test control lines determine the routing of the speech data (DAI or internal, i.e. normal mode) and which device is being tested (speech transcoder/DTX functions or A/D & D/A) as follows:

Test control line	Function

1	2

Low	Low	Normal operation
	Low	High	Test of speech decoder / DTX functions (downlink)
	High	Low	Test of speech encoder / DTX functions (uplink)
	High	High	Test of acoustic devices and A/D & D/A

The same test setup may be achieved by the layer 3 TEST_INTERFACE message (see 36.2.3 and 36.2.4.7).

### 36.4.5	Functionality of the DAI

To initiate a test, the SS shall apply the appropriate test control signals or send the appropriate layer 3 messages and then, more than 1 second later, apply a reset pulse.

Upon release of the reset pulse, the MS subsequently starts the test by issuing clock pulses when data are required or are ready.

When testing uplink speech transcoding or DTX functions, the first falling clock edge shall request from the SS the first bit of the speech samples to be encoded, the transmission of which shall start at the next earliest possible interleaved block TDMA frame (as defined in GSM 05.02) after the release of the reset pulse.

When testing downlink speech transcoding or DTX functions, the first falling clock edge shall output to the SS the first bit of the speech samples decoded from the first interleaved block TDMA frames, the reception of which is completed subsequently to the release of the reset pulse.

The MS speech transcoders shall be reset by the end of the reset pulse, whenever it occurs, whilst the DAI is in one of the active states (Test of speech decoder / DTX functions (downlink), Test of speech encoder / DTX functions (uplink), Test of acoustic devices and A/D and D/A).

Figure 36-2 / GSM 11.10 : DAI Timing

## 36.5	SIM/ME test interface

### 36.5.1	General

A special interface is required in order to perform the tests of the SIM/ME interface.

### 36.5.2	Formal aspects

It shall be possible to connect the SIM simulator to the ME. If an adapter is to be used, it shall be provided by the manufacturer of the ME.

When using the SIM simulator, the ME does not necessarily conform to all RF requirements.

When the SIM simulator is connected the ME shall be able to correctly send and receive on a TCH and associated channels under ideal radio conditions (according to GC3 of Annex 1).

### 36.5.3	Hardware and logical aspects of the interface

The signals on this interface are specified in GSM 11.11.

### 36.5.4	Mechanical characteristics of the interface

The interface of the SIM-simulator offers two connection types :

i)	a paddle which is inserted into the ME under test in place of an IC card SIM, and connects with wires to the measuring equipment. The dimensions of the paddle are shown in Figure 36-3.

ii)	a connector with leads of length 12cm, terminated directly at the contacting elements inside the ME under test.

For ME which use the plug in SIM, or when the paddle cannot be inserted due to constraints imposed by the ME design, then the ME manufacturer shall, for the purpose of testing the SIM/ME interface only, provide the ME with the connector of type ii) and the leads attached directly to the contacting elements. This connector with the flying leads shall be delivered by the test house.

Figure 36-3 / GSM 11.10 : Dimensions of full size paddle.

Annex 1 (Normative): 	Reference test methods

# A1.1	General Conditions (GC)

## A1.1.1	Outdoor test site and general arrangements for measurements involving the use of radiated fields (GC4)

The outdoor test site shall be on a reasonably level surface or ground. At one point on the site a ground plane of at least 5 metres diameter shall be provided. In the middle of this ground plane a non-conducting support capable of rotation through 360 degrees in the horizontal plane shall be used to support the test sample at 1.5 metres above the ground plane.

The test site shall be large enough to allow the erection of a measuring or transmitting antenna at a distance of half a wavelength or at least 3 metres whichever is the greater. Sufficient precautions shall be taken to ensure that reflections from extraneous objects adjacent to the site and ground reflections do not degrade the measurement results.

The test antenna is used to detect the radiation from both the test sample and the substitution antenna, when the site is used for radiation measurements. Where necessary the substitution antenna is used as a transmitting antenna, when the site is used for the measurement of receiver characteristics. This antenna is mounted on a support such as to allow the antenna to be used in either the horizontal or vertical polarization and for the height of its centre above ground to be varied over the range 1 to 4 metres. Preferably test antennas with pronounced directivity should be used. The size of the test antenna along the measurement axis shall not exceed 20% of the measuring distance.

For radiation measurements the test antenna is connected to a test receiver capable of being tuned to any frequency under investigation and of measuring accurately the relative levels of signals at its input. When necessary (for receiver measurements) the test receiver is replaced by a signal source.

The substitution antenna shall be a half wave dipole, resonant at the frequency under consideration, or a shortened dipole, or (in the range 1 to 4 GHz) a horn radiator. Antennas other than a half wave dipole shall have been calibrated to the half wave dipole. The centre of this antenna shall coincide with the reference point of the test sample it has replaced. This reference point shall be the volume centre of the sample when its antenna is mounted inside the cabinet, or the point where an external antenna is connected to the cabinet. The distance between the lower extremity of the dipole and the ground shall be at least 30 cm.

The substitution antenna shall be connected to a calibrated signal generator when the site is used for radiation measurements and to a calibrated measuring receiver when the site is used for measurements of receiver characteristics. The signal generator and the receiver shall be operating at the frequencies under investigation and shall be connected to the antenna through suitable matching and balancing network.

## A1.1.2	Anechoic shielded chamber (GC5)

As an alternative to the above mentioned outdoor test site an indoor test site, being a well shielded anechoic chamber simulating free space environment may be used. If such a chamber is used, this shall be recorded in the test report.

NOTE 21:	The anechoic shielded chamber is the preferred test site for testing to this specification.

The measurement site may be an electrically shielded anechoic chamber being 10 m long, 5 m broad and 5 m high. Walls and ceiling should be coated with RF absorbers of 1 m height. The ground should be covered with absorbing material 1 m thick able to carry test equipment and operators. A measuring distance of 3 to 5 m in the long middle axis of the chamber can be used for measurements up to at least 10 GHz.

The test antenna, test receiver, substitution antenna and calibrated signal generator are used in a way similar to that of the outdoor test site method with the exception that, because the floor absorbers reject floor reflections, the antenna height need not be changed and shall be at the same height as the test sample. In the range between 30 MHz and 100 MHz some additional calibration may be necessary.

## A1.1.3	Temporary antenna connector (GC7)

If the MS to be tested does not normally have a permanent external 50 ohm connector then for test purposes only it may be modified to fit a temporary 50 ohm antenna connector.

The permanent integral antenna shall be used for measurement of:

-Transmitter effective radiated power (section 13.3)

-Radiated spurious emissions (section 12)

For tests in the MS Receive band (925-960 MHz):- The temporary antenna coupling factor is determined using the procedure defined in Annex 1, 1.1.5. When using the temporary antenna connector, the temporary antenna coupling factor needs to be taken into consideration when determining a stimulus or measured level in the receive band.

For tests in the MS Transmit band (880-915 MHz):- The temporary antenna coupling factor is determined using the procedure defined in 13.3.4.2 When using the temporary antenna connector, the temporary antenna coupling factor needs to be taken into consideration when determining a stimulus or measured level in the transmit band.

For frequencies outside the GSM bands (880-915 MHz and 925-960 MHz) the temporary antenna coupling factor is assumed to be 0 dB.

NOTE 22:	The uncertainty in the determined value of the temporary antenna coupling factor is directly related to the uncertainty of the field strength value measured in 13.3.4.2 step n) and Annex 1, 1.1.5.2 (approximately +/- [3dB]). By mutual agreement, between the MS manufacturer and the testing authority, a value of 0 dB for the temporary antenna coupling factor could be used.

NOTE 23:	The accommodation of the uncertainty in the temporary antenna coupling factor in the MS receive band (925-960 MHz) for the tests in section 14 is for further study.

NOTE 24:	The uncertainty in the temporary antenna coupling factor in the MS transmit band (880-915 MHz) can be accommodated with appropriate adjustment of the measured levels by the uncertainty.

Testing must be performed in the following order to ensure that all the free field measurements are performed before the MS is modified.

-	Section 12.1.2.

-	Sections Annex 1, 1.1.5.1 and Annex 1, 1.1.5.2

-	Section 13.3.4.2 (during this step the MS is modified)

-	Section Annex 1, 1.1.5.3

-	All remaining tests of sections 12, 13, 14, 15, 16, 17, 18, 19, 20, 21 and 22

## A1.1.4	Temporary antenna connector characteristics

The method of connection of the temporary connector shall allow secure and repeatable connections to be made to the device under test.

The antenna connector shall present a nominal 50 ohm impedance over the GSM receive and transmit frequency ranges. The maximum loss within the frequency range 100 kHz to 12.75 GHz shall be less than 1 dB.

The connection circuitry shall be maximally broadband and shall contain no non-linear or active devices.

The characteristics of the connector shall not be significantly affected by temperatures in the range -25 to +60 degrees Celsius.

## A1.1.5	Calibration of the temporary antenna connector

For equipments fitted with an integral antenna and not provided with a permanent means for connection to an external antenna a calibration procedure is required to allow subsequent measurements to be performed on the temporary antenna connector.

Once calibrated this temporary antenna connector enables all receiver test procedures to be identical for equipments with an integral antenna and for equipments with an antenna connector.

The calibration procedure shall be carried out at three frequencies, namely an ARFCN in the low mid and high ARFCN ranges. The procedure consists of three distinct stages as follows :

1)	Establish the MS antenna radiation pattern for the three selected frequencies.

2)	Calibrate the test range (or anechoic shielded chamber) for the conditions needed in 1).

3)	Determine the temporary antenna connector coupling factor.

### A1.1.5.1	Antenna radiation pattern

a)	The MS shall be in the anechoic shielded chamber, or on an outdoor test site, on an isolated support in a vertical position at an orientation specified by the manufacturer. This position is the 0 degree position.

A test antenna, connected to the SS shall be in the anechoic shielded chamber, or on the outdoor test site, at a distance of at least 3 metres from the MS.

b)	A call shall be originated by the SS to the MS on a frequency in the low ARFCN range. The MS shall be made to answer the call. The SS shall command the MS to maximum transmit power.

c)	The SS shall, using estimated parameters for the outdoor test site or anechoic shielded chamber, set its output level 'E' [see Fig. Ann 1-1 to give an MS receiver input level of approximately 32 dBVemf. This corresponds to a field strength of 55.5 dBV/m at the MS position. The signal shall be the Standard Test Signal C1.

NOTE 25:	The absolute value of the received signal level is not critical. The value suggested however will ensure that the MS receiver is operating essentially error free, yet is low enough to avoid any non linear effects in the receiver.

d)	The SS shall use the RXLEV message from the MS to determine a measure of the received field strength. The procedure detailed in the flow chart of Figure Annex 1-1 shall now be followed.

FIGURE  Annex 1-1 / GSM 11.10

The signal level from the SS that just results in the transition from RXLEVa to RXLEVb shall be recorded as Ei.

NOTE 26:	The actual values of RXLEVa and RXLEVb will need to be recorded, because this transition will be used as the reference point for all further stages of the calibration procedure.

e)	Step d) shall be repeated after the MS has been rotated by n * 45 degrees in the horizontal plane. Ensuring that the same RXLEV transition is used, the signal levels from the SS shall be recorded as Ein.

f)	Calculate the effective mean signal level from the RMS value of the eight signal levels obtained in d) and e) above by using the following formula :

g)	Repeat steps b) to f), except in step b) use an ARFCN in the mid ARFCN range to obtain a mean signal level E2. Ensure the same RXLEV transition is used.

h)	Repeat steps b) to f), except in step b) use an ARFCN in the high ARFCN range to obtain a mean signal level E3.

Ensure the same RXLEV transition is used.

### A1.1.5.2	Test range calibration

The objective of this step is to determine the actual field strength at the MS corresponding to the three signal levels E1, E2 and E3 established in Annex 1, 1.1.5.1. The following procedure shall be used :

a)	Replace the MS by a calibrated reception antenna connected to a measuring receiver.

b)	For each frequency used in Annex 1, 1.1.5.1 measure the field strength Efr corresponding to the respective signal levels Er determined in steps f), g) and h) of Annex 1, 1.1.5.1 record these values as Ef1, Ef2, Ef3.

### A1.1.5.3	Temporary antenna connector coupling factor

The coupling factor of the temporary antenna connector is the relationship expressed in dB, between the output signal of the SS and the effective receiver input signal for the MS.

The test sample MS is modified to fit a temporary antenna connector in accordance with Annex 1, 1.1.3. Or alternatively a second MS shall be provided, fitted with such a temporary antenna connector.

NOTE 27:	If only one MS is supplied for testing, the tests of radiated spurious emissions (transmit and receive) and receiver sensitivity shall be performed before the MS is modified to accept a temporary antenna connector.

The calibration procedure shall be as follows :

a)	The MS temporary connector is connected to the output of the SS.

b)	A call shall be originated by the SS to the MS using a frequency in the low ARFCN range. The MS shall be made to answer the call. The SS shall command the MS to maximum transmit power, non hopping encrypted mode.

c)	The SS shall, using the procedures of Annex 1, 1.1.5.1, adjust its output signal level to determine the RXLEVa to RXLEVb transition. This signal level shall be recorded as Ec1.

d)	Repeat steps b) and c) for frequencies in the mid ARFCN range and the high ARFCN range. Record the RXLEV transitions as Ec2 and Ec3 respectively.

e)	The temporary antenna connector coupling factor F is then calculated from:

where Kn = conversion factor of an isotropic antenna expressed as V at the frequency V/m corresponding to the ARCFN used.

f)	The mean antenna coupling factor Fm to be used for measurements requiring hopping shall be calculated from the RMS value of all parameters in e) as follows :

g)	In all tests in which a MS with integral antenna is the unit under test, the signal level at the temporary antenna connector is determined from :

Ein 	  =	Ereq + F

where:	Ein   = signal level at coupling device (dBVemf)

Ereq  = signal level required by the test (dBVemf)

F     = coupling factor at the respective ARFCN (dB)

This is indicated in the test procedures as Ereq, dBVemf(   ), where the empty parenthesis is to be read as Ein.

For frequencies not in the receive band or the transmit band, 0dBi antenna gain shall be assumed.]

# A1.2	Normal and extreme Test Conditions (TC)

## A1.2.1	Power sources and ambient temperatures (TC2)

During type approval tests the power source of the equipment shall be replaced by a test power source, capable of producing normal and extreme test voltages as specified in sections 1.2.2 and 1.2.3. The internal impedance of the test power source shall be low enough for its effect on the test results to be negligible. For the purpose of tests, the voltage of the power source shall be measured at the input terminals of the equipment. If the equipment is provided with a permanently connected power cable, the test voltage shall be that measured at the point of connection of the power cable to the equipment. In equipment with incorporated batteries the test power source shall be applied as close to the battery terminals as practicable.

During tests the power source voltages shall be maintained within a tolerance of +/-3% relative to the voltage at the beginning of each test.

## A1.2.2	Normal test conditions (TC2.1)

The normal temperature and humidity conditions for tests shall be any convenient combination of temperature and humidity within the following ranges:

Temperature : +15 °C to +35 °C (degrees Celsius)

Relative humidity : 20% to 75%

NOTE 28:	When it is impracticable to carry out the tests under the conditions stated above, the actual temperature and relative humidity during the tests shall be recorded in the test report.

The normal test voltage for equipment to be connected to the mains shall be the nominal mains voltage. For the purpose of these specifications, the nominal voltage shall be the declared voltage or any of the declared voltages for which the equipment was designed. The frequency of the test power source corresponding to the mains shall be within 1 Hz of the nominal mains frequency.

When the radio equipment is intended for operation from the usual types of regulated lead-acid battery power source of vehicles, the normal test voltage shall be 1.1 times the nominal voltage of the battery (6 volts, 12 volts etc.).

For operation from other power sources or types of battery (primary or secondary) the normal test voltage shall be that declared by the equipment manufacturer.

## A1.2.3	Extreme test conditions (TC2.2)

For tests under extreme test conditions the 4 combinations of extreme voltages and extreme temperatures in Table 1.1 shall be applied.

Table 1.1

1	2	3	4

Temperature	High	High	Low	Low
Voltage	High	Low	High	Low

For tests at extreme ambient temperatures measurements shall be made at the temperatures given in Table 1.2, following the testing procedures given in IEC publications 68-2-1 and 68-2-2 for the low and high temperature tests.

For tests at the high temperature, after thermal balance has been achieved, the MS is switched on in the transmit condition (non DTX) for a period of one minute followed by 4 minutes in the idle mode (non DRX) after which the MS shall meet the specified requirements.

For tests at the low temperature, after thermal balance has been achieved, the MS is switched to the idle mode (non DRX) for a period of one minute after which the MS shall meet the specified requirements.

Table 1.2

Temperature (degrees Celsius)

Low	High

Handheld	-10	+55

Vehicular or	-20	+55
Portable

For tests at extreme voltages measurements shall be made at the lower and higher extreme voltages as declared by the MS manufacturer. For MS that can be operated from one or more of the power sources listed below, the lower extreme voltage shall not be higher, and the higher extreme voltage shall not be lower than that specified in Table 1.3.

Table 1.3

Voltage (relative to nominal)

Lower extreme	Higher extreme	Normal cond.

Power source:

AC mains	0.9	1.1	1.0

Regulated lead
acid battery	0.9	1.3	1.1

Non regulated
batteries:

 Leclanché/
 lithium	0.85	1.0	1.0
 
 mercury/
 nickel cadmium	0.9	1.0	1.0

## A1.2.4	Vibration requirements (TC4)

When the MS is to be tested under vibration, then random vibration is used, where the acceleration spectral densities (ASD) and the frequency ranges of GSM 05.05 [Annex D.2.3] apply. These are given in Table 1.4.

Table 1.4

frequency in Hz	ASD in m2/s3

5- 20	0.96
	20 - 500 	0.96 at 20 Hz, thereafter -3 dB / octave

The test shall be performed as described in IEC publication 68-2-36.

Annex 2 : 	Measurement uncertainty

# A2.1	Introduction

Measurement uncertainty arises from a number of causes and is often classified into random and systematic uncertainty. The former results from the combined effects of a number of small independent random variables in the measuring system, while the latter can be the result of factors such as instrument calibration, measurement conditions (e.g. temperature), etc. The combined effect is to produce a range of values within which the true value of a measured result actually lies. Thus the value of the quantity to be measured can be expressed as a probability distribution and the usual mathematical techniques applied to the analysis.

An appreciation of these factors is of value in understanding the general problem and in ensuring that the smallest uncertainties are achieved in a particular test apparatus. For testing of GSM mobile stations a system simulator is used and the various circuits of this equipment and the associated measuring equipment contribute to the total uncertainty in any measurement. These elements have been identified and the estimations are set out in Annex 5 at a confidence level of 95%.

Many of the uncertainties arise in the RF area where cables, connectors and switches with differing attenuation, VSWR, mismatch loss, etc, occur between the measuring point and the MS antenna terminal. These RF paths are systematically changed during testing and this causes further uncertainty. Also the input impedance of a mobile under test will not necessarily be known in advance which gives rise to possible additional mismatch loss, and the impedance at 12.5 GHz will almost certainly not be the same as at 900 MHz.

Given that the overall uncertainty is composed of several factors and that the random combination of a number of distributions tends towards a Gaussian distribution advantage can be taken of the relatively simple mathematical analysis that this affords.

The RF tests set out in GSM 11.10, sections 12, 13, 14 and 15 through 22, audio tests in section 30.1 and the stimulus setting levels in other tests, are the tests that are most susceptible to measurement uncertainty due to the factors described.

# A2.2	Definitions

Some terms are defined here for the purpose of this Annex.

Measurand

A quantity subjected to measurement.

Uncertainty

An estimate characterising the range of values within which the true value of a measurand lies.

Confidence level

The probability that the true value of the measurand lies within the range of values bounded by the uncertainty.

Design limits

The range of values within which a measurand should lie to ensure satisfactory operation.

# A2.3	Effect of uncertainty upon the perceived design tolerance

Suppose that a design parameter has the value 10 dB with the design tolerances of +/- 2 dB and that the tester is perfect and has absolute precision, i.e. no uncertainty, then on test any indicated value between 8 dB and 12 dB will be acceptable; this can be said with complete confidence.

Suppose, however, that the tester is real and has an uncertainty of, say, +/- 1 dB when indicating any value within its range. At an indicated value of 8 dB, it can be stated that the true value of the measurand lies somewhere in the range 7 dB to 9 dB (i.e. 8 dB +/- 1 dB); similarly at an indicated value of 12 dB the true value of the measurand lies between 11 dB and 13 dB (i.e. 12 dB +/- 1 dB).

For an indicated value at the centre of the design range, in this example 10 dB, the true value of the measurand lies in the range 9 to 11 dB, which is entirely satisfactory. For some indicated values which lie off-centre the true value of the measurand still lies in the range 8 to 12 dB. For indicated values at the extremes of the design parameter, however, there is a significant probability that the true value of the measurand lies outside the design tolerances. There is, therefore, a finite probability that for an indicated value within design tolerance the true value actually lies outside the design parameter, e.g. in the lower sub-range 7 to 8 dB, or in the upper sub-range 12 to 13 dB.

Therefore for a pass result anywhere in the indicated range 8 dB to 12 dB the true value of the measurand lies between 7 dB and 13 dB. The indicated value will, however, be a single value not a range of values and for any single indicated value the uncertainty is +/- 1 dB.

2.4	Confidence level

The confidence level can be specified for a given magnitude of uncertainty 'u' expressed in terms of the standard deviation 's' of the distribution. The nature of the distribution of the uncertainties gives a clue as to this probability. For a Gaussian distribution with a standard deviation 's' the following, well known, standard results apply.

Estimated confidence probability	Uncertainty +/- u equivalent to

95.0 %	+/-1.96 s

95.5 %	+/-2.0 s

99.0 %	+/-2.58 s

99.7 %	+/-3.0 s

For example, where the estimated confidence probability is 95 % the true value of the measurand lies within a range of +/-1.96 s from the mean. Thus in the example above if the uncertainty +/- u is +/-1 dB, at the 95 % confidence level, then there is a 95 % probability that the true value of the measurand lies within +/-1 dB of the indicated value.

# A2.5	Result of a test

From the foregoing it will be apparent that the result of a test can have several outcomes. The measurand may lie inside or outside the design limits. Where a result lies well within the design limits there is a high probability that the true value of the measurand also lies within the design limits. Where a result is exactly coincident with a design limit, there is still a significant probability that the true value of the measurand lies within the tolerance range but there is also a significant probability that the true value actually lies outside the range.

Where an indicated value lies outside the design limits but within the uncertainty range there is a high probability that the true value of the measurand also lies outside the design limits. However, there is a probability that the true value lies within the design limits. Depending upon the exact indicated value and its relation to the design limits, this probability can range from nearly 50 % for an indicated value just outside the design limits, to about 2.5 % for an indicated value at the edge of the design limits extended by the uncertainty; should this situation occur it should be noted.

Thus when a result lies close enough to a design limit for the distribution due to the measurement uncertainty to straddle that limit it is clear that the true value could lie on the other side of that limit. This poses a problem in interpretation and some rules have been formulated for the avoidance of doubt.

The result of a test shall be recorded in the test report in the following way:

1	Result within design limits.

2	Result outside the design limits.

If within the uncertainty this fact shall be stated.

NOTE 1:	Result 1 includes the case of exact coincidence with a limit.

# A2.6	Receiver tests

Receiver tests present a particular difficulty due both to the statistical nature of the tests and to the rapid rate of change of performance with respect to stimulus level.

A very small change in stimulus level (e.g. within the setting error) can drastically affect the outcome of a test. Also the number of transmission errors due to the test propagation conditions must be statistically significant and at low error rates this impacts upon the duration of the test. These factors are catered for in the test method while table Ann. 2-1 shows the effect of the uncertainties of the applied stimulus level (column headed "Test limit with Uncert.") on the error rate. The values in this column are the estimated error rates at a stimulus level equal to level specified for the particular test minus the estimated uncertainty.

The outcome of each test shall be reported as described in Section 2.5 of this Annex.

GSM900 only - begin

Table Ann.2-1 : Effect of uncertainties on "Max-events"

Type of	Type of	Propagation/	Test limit	Test limit	Minimum No
test	channel	frequency	FER/BER %	FER/BER %	of
		conditions		with uncert.	samples

BFI	TCH/FS	Static	0.244	same	82000
					
Sensitivity	TCH/FS	Static/FH	0.122	same	164000
''	TCH/FS Class Ib	Static/FH	0.410	same	20000000
''	TCH/FS Class II	Static/FH	2.439	same	8200
''	TCH/FS	TUhigh/NoFH	6.742	14.000	8900
''	TCH/FS Class Ib	TUhigh/NoFH	0.420	0.900	1000000
''	TCH/FS Class I	TUhigh/NoFH	8.333	9.000	120000
''	TCH/FS Class II	HT/NoFH	9.333	10.000	60000
''	TCH/FS Class II	RA/NoFH	7.500	8.000	24000
''	FACCH/F	TUhigh/NoFH	8.961	14.000	6696
''	FACCH/H	TUhigh/NoFH	7.728	12.000	7764
''	TCH/F9.6&H4.8	HT/No FH	0.778	2.000	180000
''	TCH/F4.8	HT/NoFH	0.011	0.040	5350000
''	TCH/F2.4	HT/NoFH	0.001	0.004	11900000
''	TCH/H2.4	HT/NoFH	0.011	0.040	5350000
					
Input Level	TCH/FS Class II	Static<-40dBm	0.012	same	1640000
range	TCH/FS Class II	Static<-15dBm	0.122	same	164000
	TCH/FS Class II	EQ	3.250	same	120000
					
Co-channel	TCH/FS	TUlow/NoFH	24.000	25.100	25000
rejection	TCH/FS Class Ib	TUlow/NoFH	2.091	2.180	3300000
''	TCH/FS Class II	TUlow/NoFH	4.300	4.470	2000000
''	TCH/FS	TUhigh/FH	3.371	4.000	17800
''	TCH/FS Class Ib	TUhigh/FH	0.215	0.300	2000000
''	TCH/FS Class II	TUhigh/FH	8.333	9.000	1200000
''	FACCH/F	TUlow/NoFH	24.000	25.100	25000
''	FACCH/H	TUlow/NoFH	24.000	25.100	25000
''	TCH/F9.6 or H4.8	TUhigh/FH	0.336	0.424	178500
''	TCH/F4.8	TUhigh/FH	0.011	same	5350000
''	TCH/F2.4	TUhigh/FH	0.001	same	11900000
''	TCH/H2.4	TUhigh/FH	0.011	same	5350000
					
Adjacent	TCH/FS	TUhigh/NoFH	6.742	12.000	8900
channel	TCH/FS Class Ib	TUhigh/NoFH	0.420	0.800	1000000
200 kHz	TCH/FS Class II	TUhigh/NoFH	8.333	10.000	600000
''	FACCH/F	TUhigh/NoFH	10.640	16.900	5639
					
Adjacent	TCH/FS	TUhigh/NoFH	11.461	18.710	8900
channel	TCH/FS Class Ib	TUhigh/NoFH	0.756	1.670	1000000
400 kHz	TCH/FS Class II	TUhigh/NoFH	9.167	10.660	600000
''	FACCH/F	TUhigh/NoFH	19.152	30.500	3133
					
Intermod.	TCH/FS Class II	Static	2.439	same	8200
''	FACCH	TUhigh/NoFH	8.961	14.000	6696
					
Blocking &	TCH/FS Class II	Static	2.439	same	8200
spurious resp.	FACCH	TUhigh/NoFH	8.961	14.000	6696

GSM900 only - end

DCS1800 only - begin

Table Ann.2-1 : Effect of uncertainties on "Max-events"

Type of	Type of	Propagation/	Test limit	Test limit	Minimum No
test	channel	frequency	FER/BER %	FER/BER %	of
		conditions		with uncert.	samples

BFI	TCH/FS	Static	0.244	same	82000

Sensitivity	TCH/FS	Static/FH	0.122	same	164000
''	TCH/FS Class Ib	Static/FH	0.410	same	20000000
''	TCH/FS Class II	Static/FH	2.439	same	8200
''	TCH/FS	TUhigh/NoFH	4.478	9.000	13400
''	TCH/FS Class Ib	TUhigh/NoFH	0.320	0.700	1500000
''	TCH/FS Class II	TUhigh/NoFH	8.333	9.000	120000
''	TCH/FS Class II	HT/NoFH	9.333	10.000	30000
''	TCH/FS Class II	RA/NoFH	7.500	8.000	24000
''	FACCH/F	TUhigh/NoFH	4.368	7.970	13736
''	FACCH/H	TUhigh/NoFH	7.752	10.800	7440
''	TCH/F9.6&H4.8HT	HT/NoFH	0.784	2.000	76500
''	TCH/F4.8	HT/NoFH	0.011	0.040	5350000
''	TCH/F2.4	HT/NoFH	0.001	0.004	11900000

Input Level	TCH/FS Class II	Static<-23dBm	0.122	same	164000
range	TCH/FS Class II	Static<-40dBm	0.012	same	1640000
	TCH/FS ClassII	EQ	3.250	same	60000

Co-channel	TCH/FS	TUlow/NoFH	24.000	25.100	25000
rejection	TCH/FS Class Ib	TUlow/NoFH	2.091	2.180	3300000
''	TCH/FS Class II	TUlow/NoFH	4.300	4.470	2000000
''	TCH/FS	TUhigh/FH	3.371	4.000	17800
''	TCH/FS Class Ib	TUhigh/FH	0.215	0.300	2000000
''	TCH/FS Class II	TUhigh/FH	8.333	9.000	1200000
''	FACCH/F	TUlow/NoFH	24.000	25.100	25000
''	FACCH/H	TUlow/NoFH	24.000	25.100	25000
''	TCH/F9.6 or H4.8	TUhigh/FH	0.336	0.424	178500
''	TCH/F4.8	TUhigh/FH	0.011	same	5350000
''	TCH/F2.4	TUhigh/FH	0.001	same	11900000
''	TCH/H2.4	TUhigh/FH	0.011	same	5350000

Adjacent	TCH/FS	TUhigh/NoFH	3.371	6.740	17800
channel	TCH/FS Class Ib	TUhigh/NoFH	0.270	0.540	2000000
200 kHz	TCH/FS Class II	TUhigh/NoFH	8.333	9.580	1200000
''	FACCH/F	TUhigh/NoFH	3.808	6.950	15756

Adjacent	TCH/FS	TUhigh/NoFH	5.714	10.140	10500
channel	TCH/FS Class Ib	TUhigh/NoFH	0.448	0.780	1200000
400 kHz	TCH/FS Class II	TUhigh/NoFH	9.167	10.760	720000
''	FACCH/F	TUhigh/NoFH	6.832	13.160	8782

Intermod.	TCH/FS Class II	Static	2.439	same	8200
''	FACCH	TUhigh/NoFH	4.368	7.970	13736

Blocking &	TCH/FS Class II	Static	2.439	same	8200
spurious resp.	FACCH	TUhigh/NoFH	4.368	7.970	13736

DCS1800 only - end

# A2.7	Pass / fail criteria

The outcomes of these tests might be interpreted in terms of pass / fail but it is not the function of the system simulator (SS) to do this interpretation. The SS reports the results of the tests.

Annex 3 : 	Protocol implementation information

General

The list of PICS and PIXIT gives all the information needed to perform the tests described in GSM 11.10.

# A3.1	Protocol Implementation Conformance Statement (PICS)

For the points listed the manufacturer has the choice between different solutions in implementation. The manufacturer has to describe his choice if there is any consequence for the tests.

## A3.1.1	LAPDm protocol (GSM 04.05 and 04.06)

### A3.1.1.1	Simplified protocol - GSM 04.06 section 6

Statement about the choice made by the manufacturer.

### A3.1.1.2	Management of SAPI = 3 - GSM 04.11 section 2.3

Statement about the handling of SAPI = 3 on the data link layer chosen by the manufacturer.

## A3.1.2	Mobility management

### A3.1.2.1	IMSI detach initiation by the MS - GSM 04.08 section 4.3.4.1

During a location updating, if an IMSI detach has to be performed (SIM or power off), the IMSI detach can be delayed until the location updating is finished, or can be omitted.

### A3.1.2.2	IMSI detach completion by the MS - GSM 04.08 section 4.3.4.3

The MS should delay the local release of the channel to allow a normal release from the network after a detach by power off command, if possible.

If not possible the RR sub-layer on the MS side should be aborted without waiting for something from the network.

### A3.1.2.3	MM specific procedures - GSM 04.08 section 4.4 and 4.5.1.1

During the lifetime of an MM specific procedure, if an MM connection establishment is required by a CM-entity, this request will either be rejected or delayed until the running MM specific procedure is terminated and, provided that the network has not sent a "follow-on proceed" indication, the RR connection is released.

If the LOCATION UPDATING REQUEST message has not been sent, the mobile station may include a "follow-on request" indicator in the message. The mobile station shall then delay the request until the MM specific procedure is completed, when it may be given the opportunity by the network to use the RR connection.

### A3.1.2.4	Receiving an MM STATUS message - GSM 04.08 section 4.6

If the MM-entity of the Mobile Station receives a MM-STATUS message no state transition and no specific action shall be taken as seen from the radio interface, i.e. local actions are possible.

## A3.1.3	Call control

### A3.1.3.1	Status enquiry procedures - GSM 04.08 section 5.5.3.1

The MS may send a STATUS ENQUIRY and take the appropriate actions based on the answer (STATUS) of the network.

### A3.1.3.2	Receiving a STATUS message by a CC entity - GSM 04.08 section 5.5.3.2

The determination of which CC states are incompatible between the MS and the network is left as an implementation decision except in some particular cases.

### A3.1.3.3	Called side compatibility checking - GSM 04.08 annex B.3

Compatibility checking can be performed in various ways from the viewpoint of execution order and information to be checked, e.g. first DDI number, sub-address and then compatibility or vice versa.

### A3.1.3.4	Disconnect on incoming call

The mobile equipment may or may not offer the possibility to disconnect an incoming call:

a)	after having confirmed an incoming call, but before alerting.

b)	after alerting, but before connecting.

02.30 (5.2.3) allows the combination of SEND and END function in one key.

## A3.1.4	Layer 1

### A3.1.4.1	Optional storage of BCCH carrier information - GSM 05.08 section 6.3

The MS may include optional storage of BCCH carrier information. For instance, the MS may store the BCCH carriers in use by the PLMN accessed when it was last active in the GSM network, or it may store BCCH carriers for more than one PLMN.

## A3.1.5	Autocalling - (ref: GSM 02.07, Annex 1)

Cause number 27 implemented in:

-	category 2 (preferred);

-	category 3.

## A3.1.6	Transient states

The following call control states may be transient in the mobile station:

State U6

State U6 may be transient if the mobile station is not configured to support explicit refusal of an incoming call by the (human or non-human) user (e.g. via  a terminal interface) before call confirmation.

If U6 is transient, there is an internal transition

CALL CONFIRMED

U6 	> U9

or an internal transition

RELEASE COMPLETE

U6 	> U0.

State U7

State U7 is transient if the implementation allows for automatic connect after an implementation specific time T.

If U7 is transient, there is an internal transition:

after T, CONNECT

U7 	> U8

State U9

State U9 is not transient if:

- the implementation does not support immediate connect

- an appropriate TCH is not yet assigned

- the signalling element has not been present in the SETUP

If the implementation supports immediate connect, there is an internal transition:

CONNECT

U9 	> U8.

If the appropriate TCH is available or the signalling element was present in SETUP, there is an internal transition:

ALERTING

U9 	> U7.

State U12

U12 is a stable state, if an appropriate speech traffic channel is connected and progress indicator #8 was present in the DISCONNECT message. Otherwise U12 is transient, and there is an internal transition:

# A3.2	Protocol Implementation Extra Information for Testing (PIXIT)

## A3.2.0	Introduction

Some of the features listed below are mandatory, others are not ; but in any case for each feature implemented the manufacturer must provide information to enable regulatory testing to be conducted.

## A3.2.1	Basic characteristics

### A3.2.1.1	Type of antenna

-	Integrated without a connector.

-	Position for normal use (if integrated without a connector).

-	With a connector allowing the connection of an external antenna.

If with a connector, declare in band impedance.

### A3.2.1.2	Power supply

-	Type of battery (if any).

-	Type of power supply.

-	Nominal voltage(s).

-	End-point voltage(s) of battery(s) (if any).

-	Details of MS shut-down voltage.

### A3.2.1.3	Power class of the MS

-	Different class declared.

-	Class mark change : description of the means to change the RF power capabilities.

### A3.2.1.4	Channel modes supported

Speech full rate

Speech half rate

Data 12 kbit/s full rate T/NT

Data 6 kbit/s full rate T/NT

Data 6 kbit/s half rate T/NT

Data 3.6 kbit/s full rate T

Data 3.6 kbit/s half rate T

### A3.2.1.5	Teleservices supported

11)	Telephony

12)	Emergency calls

21)	Short message MT/PP

22)	Short message MO/PP

23)	Short message transmission cell broadcast

61)	Alternate speech and facsimile group 3 T/NT

62)	Automatic facsimile group 3 T/NT

### A3.2.1.6	Supplementary services supported

Call forwarding.

Call restriction.

Handling of undefined GSM Supplementary Services.

### A3.2.1.7	Bearer services supported

21 )	Data circuit Duplex asynchronous	300 bit/s	T/NT

22 )	Data circuit Duplex asynchronous	1200 bit/s	T/NT

23 )	Data circuit Duplex asynchronous	1200/75 bit/s	T/NT

24 )	Data circuit Duplex asynchronous	2400 bit/s	T/NT

25 )	Data circuit Duplex asynchronous	4800 bit/s	T/NT

26 )	Data circuit Duplex asynchronous	9600 bit/s	T/NT

31 )	Data circuit Duplex synchronous	1200 bit/s	T

32 )	Data circuit Duplex synchronous	2400 bit/s	T/NT

33 )	Data circuit Duplex synchronous	4800 bit/s	T/NT

34 )	Data circuit Duplex synchronous	9600 bit/s	T/NT

41 )	PAD Access circuit asynchronous	300 bit/s	T/NT

42 )	PAD Access circuit asynchronous	1200 bit/s	T/NT

43 )	PAD Access circuit asynchronous	1200/75 bit/s	T/NT

44 )	PAD Access circuit asynchronous	2400 bit/s	T/NT

45 )	PAD Access circuit asynchronous	4800 bit/s	T/NT

46 )	PAD Access circuit asynchronous	9600 bit/s	T/NT

51 )	Data Packet Duplex synchronous	2400 bit/s	NT

52 )	Data Packet Duplex synchronous	4800 bit/s	NT

53 )	Data Packet Duplex synchronous	9600 bit/s	NT

61 )	Alternate Speech/Date

(here Data offers the same service

as bearer services 21-34 with "3.1kHz" information transfer capability)

81 )	Speech followed by Data

(here Data offers the same service

as bearer services 21-34 with "3.1kHz" information transfer capability)

### A3.2.1.8	SIM removal

-	Removal of the SIM is possible without disconnection of the power supply (Y/N).

### A3.2.1.9	Classmark

The coding of Mobile station classmark 1, 2, and 3 and the fact whether and under which conditions the classmark 3 information element is included in a CLASSMARK CHANGE message, has to be declared by the manufacturer. The declaration has to fulfil the following requirements:

-	Mobile station classmark 1: Bits 4, 5, and 8 of the first (and only) octet of the value part of the information element shall be coded as "0". The "Revision level" and "RF power capability" field shall specify the value that is correct for the MS.

-	Mobile station classmark 2: Bits 4, 5, and 8 of the first octet, bits 2, 3, and 8 of the second octet, bits 3 to 7 of the third octet of the value part of the information element shall be coded as "0". The "Revision level" field, "RF power capability" field, "PS capability" field, "SS Screening indicator" field, "SM capability" field, "Frequency capability" field, "Classmark 3" field, "A5/2 algorithm supported" field, and "A5/3 algorithm supported" field shall specify the value that is correct for the MS.

-	Mobile station classmark 3: Bits 5 to 8 of the first octet of the value part of the information element shall be coded as "0". If the value part contains more octets, they shall be coded as "0000 0000". The "A5/4 algorithm supported" field, "A5/5 algorithm supported" field "A5/6 algorithm supported" field, and "A5/7 algorithm supported" field shall specify the value that is correct for the MS (that is, they shall be set to "0").

NOTE 1:	The requirements to the classmark may be subject to changes. That is why test cases are expected to verify the manufacturer's declaration, whereas the correctness of the manufacturer's declaration is to be verified "off line".

### A3.2.1.10	Type of SIM/ME interface (ref. GSM 11.11 and GSM 11.12)

-	5V SIM/ME interface (5V only ME)

-	3V SIM/ME interface (3V only ME)

-	5V/3V SIM/ME interface (3V technology ME)

## A3.2.2	Man machine interface

### A3.2.2.1	Mobile station features

-	Description of manual entry and display of a called number.

-	Description of the basic way to send a call manually.

-	Description of the basic way to take a call manually.

-	Description of the basic way to end a call manually.

-	Description of the basic way to send an emergency call manually.

-	Description of the basic way to send DTMF manually.

-	Description of the manual PLMN selector.

-	Description of the automatic PLMN selector.

-	Description of the indication of the country.

-	Description of the indication of the available PLMN.

-	Description of the indication of the automatic registration to a PLMN.

-	Description of the service indicator.

-	Description of the management of the SIM by the user:

.	keying PIN and changing PIN,

.	indication of acceptance or rejection of keyed PIN,

.	indication of blocked SIM,

.	indication of successful unblocking of the SIM,

.	storing an abbreviated number,

.	displaying an abbreviated number.

-	Description of the selection of the hands free.

-	Description of the volume control.

-	Description of local barring of outgoing calls.

-	Description of prevention of unauthorized calls.

-	Description of the auto calling management:

.	selection of the auto calling,

.	indication that the call failed and a re-try is attempted,

.	indication that the call finally failed.

-	Description of the way in which the MS generates an MS originated NOTIFY, if possible. This feature may or may not be supported by the MS.

NOTE 2:	All the above description could be extracted from the user's manual.

### A3.2.2.2	Short message service

-	Description of the basic procedures to send a mobile originated short message.

-	Description of the basic procedures to display a mobile terminated short message.

-	Description of the basic procedures to display a cell broadcasted short message.

-	The value of the timer TC1M.

-	Whether SMS messages are stored in the SIM and/or the ME.

-	Maximum length (characters) of a mobile originated short message.

### A3.2.2.3	Supplementary services

#### A3.2.2.3.1	Call forwarding

-	Description of the user's commands and of the display of the answers from the network for:

registration,

erasure,

activation,

deactivation,

interrogation,

specific data request.

-	Description of the display of:

.	notification of an incoming call to the "served" mobile or the "forwarded to" mobile,

.	notification during out-going call,

.	information to the calling mobile.

#### A3.2.2.3.2	Call restriction

-	Description of the user's commands and the display of the answers from the network for:

registration,

change of the password,

activation,

deactivation,

interrogation.

-	Description of the display of the indication of call barring.

#### A3.2.2.3.3	Handling of (undefined) GSM supplementary services

-	Description of the user's commands and the display of the answer from the network.

## A3.2.3	Electrical Man Machine Interface (EMMI)

### A3.2.3.1	Methods supported for activation/deactivation of EMMI

-	all possibilities specified in GSM 11.10, III.1.2.2;

-	all possibilities specified in GSM 11.10, III.1.2.2, except activation by inserting a test SIM (when the ME is already switched on);

-	activation/deactivation only via layer 3 messages on the radio interface according to GSM 11.10, III.1.2.2.

### A3.2.3.2	Transmission rate supported by the ME on the EMMI

### A3.2.3.3	Layer 3 messages supported on the EMMI

-	Layer 3 messages as specified in GSM 11.10, III.1.3.5.3.2, except: (followed by the list of messages not supported);

-	others than defined in GSM 11.10 part III table 9.

### A3.2.3.4	Keystroke sequence messages

Non standard keystroke sequences to be used on the EMMI (in line with GSM 11.10, III.1.3.5.3.2):

-	related to tests of the mobile station features (GSM 11.10, II.16)

-	related to testing of the ME/SIM interface (GSM 11.10, II.8)

-	related to tests of autocalling restrictions (GSM 11.10, II.9)

-	related to tests of supplementary services (GSM 11.10, II.12)

-	related to tests of data services (GSM 11.10, II.10, II.14, II.15)

-	related to tests of short message service (GSM 11.10, II.11.2.2.2)

-	related to other tests.

### A3.2.3.5	Internal malfunction detected messages

List of the error indicators provided.

## A3.2.4	Digital Audio Interface (DAI)

Description of the speech data routing

-	via the control lines

-	or via the test interface message.

## A3.2.5	Characteristics related to bearer services or teleservices

### A3.2.5.1	Access interface

Description of the access interface to connect the DTE (e.g. V series (V.24, V.28), X series, two wire analogue interface for use with fax group 3, I.420 (S-reference point).

In case of a proprietary interface to a DTE (non standard), description of this interface (hardware and software).

In case of a non standard connector provide a mechanical adapter.

### A3.2.5.2	Configuration of the MT

Description of the configuration information to be selected in the MT to connect a terminal equipment to the mobile termination.

Description of the (different) configuration(s) of the MT for each bearer service and each teleservice supported, with the range or value for the parameters and the configuration procedure.

For the purpose of test of MOC, the manufacturer shall describe precisely how it is possible to put the MT in the different configurations to generate the capability information of the Mobile according to section 3.2.5.3, and  described as supported by the MS.

For the purpose of test of MTC, the manufacturer shall describe how to verify the correct selection by the MT of the required function with regard to the capability information as described below, especially using the messages at the Um interface if there is no R or S interface available (case MTO). The description shall be made for every combination of the parameter value valid for the MT.

### A3.2.5.3	Capability information

Description of the capability information, related to supported bearer services

-	bearer capabilities,

-	higher layer capabilities,

-	lower layer capabilities,

The manufacturer shall describe for every capability the associated terminal functions and their characteristics.

### A3.2.5.4	Subaddress or DDI number

Subaddress or a DDI number of the MT.

Procedure to allocate or change DDI number or subaddress, if possible.

### A3.2.5.5	User to user signalling

Description of the function and the user's access to it.

### A3.2.5.6	Data call set-up and data call clearing

For each implemented transparent and non-transparent data service:

-	Description of the data call establishment mechanism

-	Terminal initiated (CT108) (if possible),

-	MT (MMI/EMMI) initiated.

-	Description of DCE provided information (MT to TE), if any.

-	Declaration of optimal function and procedure, services supported by the MT.

-	Description of the data call clearing mechanism

-	Terminal initiated (CT109) (if possible),

-	MT (MMI/EMMI) initiated.

-	Description of DCE provided information (MT to TE) related to a mobile or network initiated call clearing, if any.

### A3.2.5.7	Characteristics of non-transparent data services

Description of Radio Link Protocol (RLP) features supported

Description of flow control mechanism

-	INBAND (XON/XOFF)

-	OUTBAND COPnoFICt (CT105 and CT106)

### A3.2.5.8	Possible ways of setting-up a call from either an external interface or internally

Describe in detail all possible ways a call can be initiated from the MS or a connected terminal.

### A3.2.5.9	Application layer causing automatic call termination.

State whether the call termination facility can be disabled and if so, describe in detail how.

### A3.2.5.10	Call re-establishment for MS not supporting speech

Applicability of call re-establishment.

## A3.2.6	International mobile station equipment identity

IMEI of the MS.

## A3.2.7	Receiver intermediate frequencies

Flo - Local Oscillator frequency applied to first receiver mixer.

IF1 ... IFn - intermediate frequencies.

Annex 4: 	Test SIM Parameters

# A4.1	Introduction

This section defines default parameters for programming the elementary files of the test SIM. The requirements of this annex do not apply to the SIM/ME tests of section 27.

## A4.1.1	Definitions

"Test SIM card":

A SIM card supporting the test algorithm for authentication, programmed with the parameters defined in this section. The electrical, mechanical and environmental requirements of the test SIM card are specified in GSM 11.11.

"Test SIM":

Either a test SIM card or the SIM simulator programmed with the parameters defined in this section.

## A4.1.2	Definition of the test algorithm for authentication

The following procedure employs bit wise modulo 2 addition ("XOR")

The following convention applies:

In all data transfer the most significant byte is the first byte to be sent; data is represented so that the left most bit is the most significant bit of the most significant byte.

Step 1:

XOR to the challenge RAND, a predefined number Ki, having the same bit length (128 bits) as RAND. The result RES1 of this is

RES1 = RAND XOR Ki

Step 2:

The most significant 32 bits of RES1 form SRES. The next 64 bits of RES1 form Kc. The remaining 32 bits are not used.

# A4.2	Default Parameters for the test SIM

Ki:

The authentication key "Ki" will be chosen by the test house and will be non zero. The "Ki" value used by the SS will align with this value.

PIN Disabling

The PIN enabled / disabled flag will be set to "PIN Disabled". This ensures that when the Test SIM is inserted into a MS the user will not be prompted for PIN entry. This requires a specific card capability defined by the SIM service table (see section 2.9).

# A4.3.	Default settings for the Elementary Files (EFs).

The format and coding of elementary files of the SIM are defined in GSM 11.11. The following sections define the default parameters to be programmed into each elementary file. Some files may be updated by the MS based on information received from the SS. These are identified in the following sections.

## A4.3.1	EFICCID (ICC Identification)

The programming of this EF is a test house option.

## A4.3.2	EFLP (Language preference)

The programming of this EF is a test house option.

## A4.3.3	EFIMSI (IMSI)

The IMSI value will be chosen by the test house. The IMSI used by the SS will align this value.

File size:	9 bytes

Default values:	Byte 1 (DEC):	8

Bytes 2-9 (HEX):	09 10 10 ** ** ** ** **

"*" indicates any number between 0 and 9 subject to the restriction that IMSI mod 1000 (i.e. bytes 7, 8 and 9) lies in one of the following ranges:

063-125, 189-251, 315-377, 441-503, 567-629, 693-755, 819-881 or 945-999

NOTE 1:	This ensures that the MS can listen to the second CCCH when more than one basic physical channel is configured for the CCCH. This is necessary for the test of "paging re-organisation".

## A4.3.4	EFKc (Ciphering key Kc)

File size:	9 Bytes

Default values (HEX):	Bytes 1-8:	Align with Kc used by SS

Byte 9:	07

Byte 9 is set to 07 to indicate that there is no key available at the start of a test.

The bytes within this elementary file may be updated by the MS as a result of a successful authentication attempt.

## A4.3.5	EFPLMNsel (PLMN selector)

File size:			102 bytes

Default values (HEX):	Bytes 1-3:		32 F4 10	(MCC, MNC) - Translates to 234, 01

Bytes 4-6:		32 F4 20	(MCC, MNC)

Bytes 7-9:		32 F4 30	(MCC, MNC)

....

....

....

Bytes 94-96:		32 F4 23	(MCC, MNC)

Bytes 97-99:		32 F4 33	(MCC, MNC)

Bytes 100-102:		32 F4 43	(MCC, MNC)

34 PLMNs are shown coded above since this is the largest number required for a test - see section 27.9.4.1. It is necessary to take this into account since the SIM cards must be dimensioned to cope with this number of records.

## A4.3.6	EFHPLMN (HPLMN search period)

File size:			1 byte

Default value (HEX):	00 (no HPLMN search attempts)

## A4.3.7	EFACMmax (ACM maximum value)

File size:				3 bytes

Default:		Byte 1:		00

Byte 2:		00

Byte 3:		00

The above translates to:	“Not valid”.

## A4.3.8	EFSST (SIM service table)

Services will be allocated and activated as follows:

Service		Allocated	Activated

No. 1: CHV1 disable function	Yes	Yes

No. 2: Abbreviated Dialling numbers (ADN)	Yes	Yes

No. 3: Fixed dialling numbers (FDN)	Yes	Optional

No. 4: Short Message Storage (SMS)	Yes	Yes

No. 5: Advice of Charge (AoC)	Yes	Yes

No. 6: Capability Configuration Parameters (CCP)	Yes	Yes

No. 7: PLMN Selector	Yes	Yes

No. 8: Reserved for future use	No	No

No. 9: MSISDN		Optional	Optional

No. 10: Extension 1	Yes	Optional

No. 11: Extension 2	Yes	Optional

No. 12: SMS Parameters	Yes	Yes

No. 13: Last Dialled Number (LND)	Yes	Yes

No. 14: Cell Broadcast Message Identifier	Yes	Yes

No. 15: Group identifier Level 1	Yes	Optional

No. 16: Group identifier Level 2	Yes	Optional

## A4.3.9	EFACM (Accumulated call meter)

File size:				3 bytes

Default:		Byte 1:		00

Byte 2:		00

Byte 3:		00

The above translates to:	“Not yet implemented”.

## A4.3.10	EFPUCT (Price per unit and currency table)

File size:				5 bytes

Default:		Byte 1-3:		FF

Byte 4-5:		00

## A4.3.11	EFCBMI (Cell broadcast Message Identifier Selection)

The programming of this EF is a test house option.

The file size is 2n bytes, where n is the number of Cell broadcast message identifier records - each record defining a type of Cell Broadcast message which may be accessed by the MS. Care should be taken when dimensioning the SIM to take into account the number of Cell Broadcast message identifier records required.

## A4.3.12	EFBCCH (Broadcast control channels)

File size:			16 Bytes

Default values (BIN):	Bytes 1-2:		11111111 11111111

Bytes 3-4:		11111111 11111111

Bytes 5-6:		11111111 11111111

Bytes 7-8:		11111111 11111111

Bytes 9-10:		11111111 11111111

Bytes 11-12:		11111111 11111111

Bytes 13-14:		11111111 11111111

Bytes 15-16:		11111111 11111111

This field may be updated dependent on the MS implementation.

## A4.3.13	EFACC (Access control class)

File size:			2 Bytes

Default values (BIN):	Byte 1:		00000000

Byte 2:		********

The test house may set any single bit of byte 2 to "1". All remaining bits of byte 2 will be set to "0". This determines the access control class of the SIM.

## A4.3.14	EFFPLMN (Forbidden PLMNs)

Length:			12 Bytes

Format (HEX):		Bytes 1-3:		FF FF FF

Bytes 4-6:		FF FF FF

Bytes 7-9:		FF FF FF

Bytes 10-12:		FF FF FF

This coding corresponds to an empty "forbidden PLMN list". The bytes within this file may be updated if a LOCATION UPDATE REJECT message is received by the MS with cause, "PLMN not allowed".

## A4.3.15	EFLOCI (Location information)

File size:			11 Bytes

Default values:			Bytes 1-4 (HEX):	FF FF FF FF (TMSI)

Bytes 5-9 (HEX):	42 F6 18 FF FE (LAI)

Byte 10 (HEX):	FF (Periodic LU Time = "the timer is not running")

Byte 11 (BIN):	00000001 (Location Update Status = "not updated")

Bytes 5-9:	LAI-MCC = 246 (bytes 5-6) and LAI-MNC = 81 (byte 7) are frequently used in section27. The LAC (bytes 8-9) is set to “FF FE” since this, in conjunction with byte 11 setting of “01”, is used to ensure that the MS performs a location update at the beginning of a test.

Bytes in this file (e.g. TMSI in bytes 1-4) may be updated as a result of a location update attempt by the MS.

## A4.3.16	EFAD (Administrative data)

File size:			3 bytes

Default values			Byte 1:		10000000	- (type approval operations)

Byte 2:		11111111

Byte 3:		11111111

## A4.3.17	EFPhase (Phase identification)

File size:			1 byte

Default value (HEX):	02		Phase 2

## A4.3.18	EFADN (Abbreviated dialling numbers)

The programming of this EF is a test house option. It should be noted that sufficient space should be provided on the SIM card for 101 records - see section 27.15.4.1.

## A4.3.19	EFFDN (Fixed dialling numbers)

Optional.

## A4.3.20	EFSMS (Short messages)

Default:	Records 1-5	Byte 1:		00

Byte 2:		FF

Bytes 3-14:	FF FF FF FF FF FF FF FF FF FF FF FF

Bytes 15-26:	FF FF FF FF FF FF FF FF FF FF FF FF

Byte 27:	FF

Byte 28:	FF

Bytes 29-35:	FF FF FF FF FF FF FF

Byte 36:	FF

Bytes 37-176:	All Bytes set to FF

## A4.3.21	EFCCP (Capability configuration parameters)

File size:			14 bytes

Default values			Byte 1:		04

Byte 2:		01

Byte 3:		A0

Bytes 4-14:	FF

The above translates to:	“Full rate, GSM Standardised coding, circuit mode and speech”.

## A4.3.22	EFMSISDN (MSISDN)

Optional

## A4.3.23	EFSMSP (Short message service parameters)

The programming of this EF is a test house option.

Each record size is 28+Y bytes, where Y is the number of bytes in the Alpha Identifier. Care should be taken when dimensioning the SIM to take into account the number of Short message service parameter records required.

## A4.3.24	EFSMSS (SMS status)

File size: 2 bytes

Byte 1:		00

Byte 2 (BIN):	11111111

The above translates to:

(a)	Last Mobile Originated Short Message had a TP Message Reference parameter of “00”.

(b)	SMS Memory Capacity Exceeded, Notification Flag unset: memory capacity available.

## A4.3.25	EFEXT1 (Extension 1)

Optional

## A4.3.26	EFEXT2 (Extension 2)

Optional

Annex 5: 	Test equipment

# A5.1	Introduction

## A5.1.1	General

The test equipment is either an equipment or assembly of equipments which enables the tests described in this specification to be conducted.

This annex describes requirements for the test equipment which cannot be derived from and which are assumed in, the conformance test descriptions described in this specification.

Specifically stimulus setting and measurement uncertainties are defined.

## A5.1.2	Test equipment terms

The term "System Simulator" (SS) is used to describe the complete suite of test equipment required to perform the tests in this specification when interacting with the following MS interfaces:

-	Antenna (Connector or radiated)

-	Acoustic

-	Data Port(s)

-	Power supply

-	DAI

NOTE 1:	To perform a sub-set of tests, the SS may be simplified accordingly.

The term "SIM simulator" is used to describe the test equipment required to interact with the SIM/ME interface.

A "test SIM" has the physical characteristics of a standard SIM card, (see GSM 11.11) with specific parameters defined in Annex 3.

## A5.1.3	Confidence level

All uncertainty values stated in this annex are quoted for a Confidence Level of 95%.

# A5.2.	Standard test signals

The Cx signals represent the wanted signals and the Ix signals represent the unwanted signals.

Signal C0	Unmodulated continuous carrier;

Signal C1	A standard GSM signal with the modulation derived by applying a data reversals signal to the input of a channel coder. The channel coder will depend on the test and the cipher mode shall be selectable by the test method. When using this signal in the non hopping mode, the unused seven time slots shall also contain dummy bursts, with power levels variable with respect to the used timeslot, see also 2.3.1.3.

Signal I0	Unmodulated continuous carrier;

Signal I1	A GMSK modulated carrier following the structure of the GSM signals, but with all modulating bits (including the midamble period) derived directly from a random or pseudo random data stream.

Signal I2	A standard GSM signal with valid midamble, different from C1. The data bits (including bits 58 and 59) shall be derived from a random or pseudo random data stream.

# A5.3.	SS functional requirements

## A5.3.1	Level setting range

It is assumed that the SS is capable of setting stimulus levels, at the MS interface, to those required in the test specification extended by the measurement uncertainty defined in this annex.

NOTE 2:	This ensures that the SS is able adequately to stimulate the MS performance at and just beyond the limit requirement under all conditions.

## A5.3.2	Level Measurement / operation range

It is assumed that the SS is capable of performing measurements, within the uncertainty defined in this annex, over a level range, at the MS interface, as required in the test specification extended by the SS measurement uncertainty defined in this annex and extended by a further 3dB on the MS conformity requirement.

NOTE 3:	This ensures that the SS is able adequately to measure the MS performance at and just beyond the limit requirement under all conditions.

## A5.3.3	MS power supply interface

Test DC power supply for MS:

Voltage setting uncertainty		< 1 %

Ripple				< 10 mV RMS, 50 mV peak to peak

Test AC power supply for MS:

Voltage setting uncertainty:		< 1 %.

## A5.3.4	MS antenna interface

The SS is assumed to offer a nominal 50 ohm impedance to the MS

GSM/DCS bands	< 4 GHz	< 10 GHz	< 12.75 GHz

VSWR		<= 1.3	<= 2.0	<= 3.0	<= 3.5

### A5.3.4.1	Uplink receiver error

The SS receiver should be capable of performing the tests as specified in GSM 11.10 without the addition of bit errors in excess of 1 in 10E7 due to the receiver performance when operated with a MS which meets the transmitter requirements of GSM 05.05.

NOTE 4:	This requirement is based on a minimum BER measurement of 1 in 10E5.

### A5.3.4.2	Power versus time measurements

Measurement uncertainty of peak transmitter carrier power:	+/- 1 dB

Measurement uncertainty of power level (relative to peak transmitter carrier power):

Power level				Measurement uncertainty

+ 6 dB to - 7 dB				+/-0.25 dB

- 7 dB to -20 dB				+/-1.0 dB

-20 dB to -32 dB				+/-2.0 dB

-32 dB to -45 dB				+/-5.0 dB

-45 dB to -71 dB				+/-1.0 dB

<-71 dB					+/-2.0 dB

NOTE 5:	Due to the method of measurement (downconversion to I/Q baseband / filtering / A/D conversion / postprocessing) several uncertainties occur. The sources are:

a)	absolute level uncertainty;

b)	filter ripple,	
	I/Q gain imbalance,	
	I/Q imperfect quadrature;	

c)	A/D conversion (resolution),	
	I/Q offset.

Items under b) and c) affect the individual samples and can be observed as a "ripple" in the horizontal part of the power time mask.

Items under b) are uncertainties which are proportional to the signal measured.

Items under c) are constant amounts of uncertainty, independent of the signal measured.

The item a) moves the entire power time template up or down.

The uncertainties b) and c) are added to the measured signal as an uncorrelated interferer.

The above mentioned absolute measurement uncertainty refers to a). The table covers uncertainties b) and c).

Uncertainty of time measurement

The relative timing uncertainty of the transition point

-	bit 13 to 14 in the midamble (normal burst)

-	end of the sync sequence (access burst)

is +/- 1/8 bit.

Timing uncertainty of the measurement samples in the vertical part of the power time mask are displayed as marked fields in the Figure A5.3-1

Figure A5.3-1 Time Measurement Uncertainty for the Power Time Mask

NOTE 6:	With a real method of measurement one has to reckon on systematic measurement uncertainties in the vertical part of the power time template (Figures 13-2 & 13-3). The reason for this is that the measurement is conducted through a filter which has to fulfil different requirements simultaneously, requirements in the frequency domain and in the time domain as well. The time behaviour of the filter causes the above mentioned measurement uncertainty. It occurs clearly when measuring the falling edge of the power burst. The measurement uncertainty, which in principle delays the actual performance, depends on the filter characteristics and on the signal shape. At favourable signal shapes the uncertainty is negligible, however, at unfavourable signal shapes it consumes the marked area in Figure A5.3-1 (falling edge).

The underlying filter is:

type			inverse Chebycheff

passband			<= +/- 200 kHz

stopband (40 dB stop att.)	>= +/- 541.67 kHz.

To avoid aliasing with this filter the RF output spectrum must meet the requirements of section 13.4.

If the lowest limit line in the power time template is replaced by a -54 dBm line, measuring lower carrier powers, the area of measurement uncertainty is reduced equivalently.

The marked area in Figure A5.3-1 describes the systematic measurement uncertainty of the test equipment and does not widen the design requirements.

Uncertainties associated with 13.3.5 requirement b) (power control levels, adjacent steps):

Repeatability			+/- 0.3 dB

Linearity				+/- 0.03 dB/dB

Combined uncertainty is:		+/- ( 0.3 + 0.03 dB/dB ) dB

E.g. where the indicated value of the step size is 2.0 dB, the uncertainty is: +/- (0.3 + 0.06) dB = +/- 0.36 dB.

### A5.3.4.3	Wideband selective power measurement

Power is to be measured selectively for spurious emissions without frequency hopping (ref: section 12.

Uncertainty conducted		100 kHz to 1GHz		+/- 1.5 dB

1 GHz to 12.75 GHz	+/- 3.0 dB

Uncertainty radiated		30 MHz to 4 GHz		+/- 6 dB

NOTE 7:	The uncertainties include the effect of a worst case reflection from the MS of 0.7 for out of band signals.

It is acceptable to use a band stop filter in spurious emission measurements of the transceiver in order to fulfil the above requirements.

### A5.3.4.4	Inband selective power measurements

Power is to be measured selectively for output RF spectrum

The measurement is performed on a single frequency while the MS is frequency hopping (ref: section 13.3)

Uncertainty			< +/- 1.6 dB

NOTE 8:	The video signal of the spectrum analyser is "gated" such that the spectrum generated by at least 40 of the bits 87 to 132 of the burst is the only spectrum measured. This gating may be analogue or numerical, dependent upon the design of the spectrum analyser.

### A5.3.4.5	Phase trajectory and frequency error measurements

Ref: Sections 13.1 & 13.2 for definitions and methods of measurement.

Phase measurement uncertainty	+/- 1 degree RMS

+/- 4 degrees for individual

phase measurement samples

The phase measurement uncertainties above apply during the useful bits.

Frequency measurement uncertainty:	+/- 5 Hz.

### A5.3.4.6	RF delay measurements relative to nominal times

Range				-140 to +140 bit periods

Resolution			1/4 bit period

Uncertainty			+/- 1/8 bit period

### A5.3.4.7	The wanted signal or traffic channel of serving cell

The Wanted signal is used in most of the specified RF measurements. The traffic channel of the serving cell is used in most of the signalling tests.

FREQUENCY:

Uncertainty:		< +/- 5*10E-9

PHASE:

Uncertainty:		< +/- 1 degree RMS and

< +/- 4 degrees peak(as defined in GSM 05.05)

LEVEL:

Uncertainty:		< +/- 1 dB in section 13, 14 except:-

< +/- 3 dB for test 14.2 radiated

< +/- 1.2 dB for test 14.6

< +/- 2.5 dB for all other tests.

Settling time:	< 10 us

DYNAMIC LEVEL SETTING:

The SS shall be able to switch from any power level to any other power level within the range of 30 dB on a timeslot per timeslot basis. This dynamic switching requirement only applicable for a single channel for a limited number of tests.

MODULATION:		GMSK (as specified in GSM 05.04)

SPURIOUS:

inchannel:		Covered by phase error

outchannel:

Noise Power, 1 Hz bandwidth:

<-100 dBc for >100kHz carrier offset,									<-110 dBc for >300kHz carrier offset,									<-121 dBc for >1500kHz carrier offset.

Non harmonics:	< -55 dBc for > 100kHz carrier offset,

< -68 dBc for >1500kHz carrier offset

FREQUENCY HOPPING:

The signal shall be capable of hopping according to the criteria of GSM 05.02. The timing of the frequency change shall be such that frequency transitions do not occur during the active timeslot of the MS.

### A5.3.4.8	The first interfering signal or traffic channel of the first adjacent cell

The First interfering signal is used in measurements of co-channel rejection, adjacent channel rejection and intermodulation rejection. The Traffic channel of the first adjacent cell is used in handover tests.

FREQUENCY:

Uncertainty:		< +/- 5*10E-9

PHASE:

Uncertainty:		< +/- 1 degree RMS and

< +/- 4 degrees peak(as defined in GSM 05.05)

LEVEL:

Uncertainty		< +/- 1 dB relative to the wanted signal for

test 13.2 and 14.5

< +/- 0.3 dB relative to the wanted signal for

test 14.4

< +/- 1 dB for test 14.6

< +/- 2.5 dB for all other tests

MODULATION:		GMSK (as specified in GSM 05.04)

The total relative single sideband power (noise + harmonics) in the frequency range 1.5 to 1.7 MHz offset from the nominal carrier frequency shall be less than -72 dBc.

SPURIOUS:

Inchannel:		Covered by phase error

Outchannel:

Noise Power, 1 Hz bandwidth:

<-100 dBc for > 100kHz carrier offset

<-110 dBc for > 300kHz carrier offset

<-127 dBc for >1500kHz carrier offset

non harmonics	< -55 dBc for > 100kHz carrier offset

< -68 dBc for >1500kHz carrier offset

FREQUENCY HOPPING:

The signal shall be capable of hopping according to the criteria of GSM 05.02. The timing of the frequency change shall be such that frequency transitions do not occur during the active timeslot of the MS.

### A5.3.4.9	The second interfering signal

The second interfering signal is used in the measurements of intermodulation rejection and blocking.

FREQUENCY:

Uncertainty:		< +/- 5*10E-9

LEVEL:

Uncertainty:		< +/- 1 dB for test 14.6

< +/- 1.5 dB relative to the wanted signal

for all other tests.

MODULATION:		Unmodulated

SPURIOUS:

Inchannel:		No requirements

Outchannel:

Noise Power, 1 Hz bandwidth:

<-135 dBc for > 500kHz carrier offset,

<-140 dBc for > 700kHz carrier offset,

<-150 dBc for >1500kHz carrier offset.

Non harmonics	< -79 dBc for > 500kHz carrier offset,

< -84 dBc for > 700kHz carrier offset,

< -94 dBc for >1500kHz carrier offset

Harmonically related spurii	<-40 dBc

### A5.3.4.10	BCCH carriers of serving and adjacent cells

The BCCH of the serving cell is used for synchronising the MS and to send network information to the MS under test. The BCCH signals of the adjacent cells are used in the handover tests. The MS measures the RF-levels of the BCCHs of adjacent cells.

FREQUENCY:

Uncertainty:		< +/- 5*10E-9

PHASE:

Uncertainty:		< +/- 1 degree RMS and

< +/- 4 degrees peak(as defined in GSM 05.05)

LEVEL:

Uncertainty:		< 1 dB for test 13.2

< 2.5 dB for all other tests

< 0.6 dB relative to each other and to TCH

for test 21 over the range 65 dBmicroVoltemf

to 3 dBmicroVoltemf

< 1.2 dB relative to each other and to TCH for test 26.3

MODULATION:		GMSK (as specified in GSM 05.04)

SPURIOUS:

Inchannel:		Covered by phase error

Outchannel:

Noise Power, 1Hz bandwidth:

<-100 dBc for > 100kHz carrier offset

<-125 dBc for >1500kHz carrier offset

Non harmonics	< -55 dBc for > 100kHz carrier offset

< -72 dBc for >1500kHz carrier offset

### A5.3.4.11	The wide frequency range signal

The wide frequency range signal is used in the measurements of spurious response.

FREQUENCY

Uncertainty:		< +/- 5*10E-9

LEVEL

Uncertainty:		< +/- 1.5 dB relative to the wanted signal for test 14.7

< +/- 1 dB error of substituted 'wanted signal'

MODULATION:		unmodulated

SPURIOUS in the MS receiving range

non harmonics:	< -94 dBc

Harmonically related spurii:	< -40 dBc

Noise:		< -4 dBuVemf equivalent at the MS receiver input

when measured in a 200 kHz bandwidth.

### A5.3.4.12	The multipath fading function

The multipath fading function simulates the fading effects of a broadband radio channel in mobile radio communication

The propagation conditions are specified in GSM 05.05, annex 3.

The multipath fading function shall be performed only within a 5 MHz bandwidth during one test case.

## A5.3.5	MS audio interface and DAI

### A5.3.5.1	General uncertainties

Unless otherwise specified, the following uncertainties apply to the audio interface:--

Signal level measurement uncertainty:		+/- 0.2 dB

Sound pressure measurement uncertainty:	+/- 0.6 dB

Frequency Measurement uncertainty:		+/- 0.1%

Stimulus frequency setting uncertainty:

Frequency settings are taken from ISO 3, R10 series or R40 series or from Table 2 of Rec. CCITT P.79. A departure from the nominal frequencies of +/- 5% below 240 Hz and +/-2% at 240 Hz and above is accepted.

In the case of 4 kHz the departure is restricted to -2%.

### A5.3.5.2	Analogue single test tone

Total distortion:				< 0.5 %

### A5.3.5.3	Delay measurement between Um and DAI

The delay measurement between the Um interface of the MS and its DAI in both directions is described in section 32.5.

Uncertainty:				< +/- 0.1 ms

# A5.4	SIM simulator functional requirements

## A5.4.1	General

The SIM simulator shall implement the functions of a SIM as described in GSM 02.17 and 11.11.

The Test Algorithm for authentication incorporated in the SIM Simulator shall operate as described in Annex 3

## A5.4.2	Contacts C1, C2, C6, C7

### A5.4.2.1	Default measurement / setting uncertainties

Unless stated otherwise below, the following uncertainties apply:

Voltage measurement uncertainty:	< +/- 50 mV

Voltage setting uncertainty:		< +/- 20 mV

Time measurement uncertainty:		< +/- 100 ns

### A5.4.2.2	Contact C1

Continuous Spikes

Current Load Amplitude			0 mA - 20 mA

Adjustable Step Size		1 mA

Uncertainty			< +/- 1 mA

Additional Current Offset		0 mA - 5 mA

Adjustable Step Size		1 mA

Uncertainty			< +/- 1 mA

Pulse Width				100 ns - 500 ns

Adjustable Step Size		50 ns

Uncertainty			< +/- 25 ns

Rise and Fall Time			<= 50 ns

Pause Width				100 ns - 500 ns

Adjustable Step Size		50 ns

Uncertainty			< +/- 25 ns

Random Spikes

Current Load Amplitude			50 mA - 200 mA

Adjustable Step Size			1 mA

Uncertainty				< +/- 1 mA

Additional Current Offset		0 mA - 5 mA

Adjustable Step Size		1 mA

Uncertainty			< +/- 0.1 mA

Pulse Width				100 ns - 500 ns

Adjustable Step Size		50 ns

Uncertainty			< +/- 25 ns

Pause Width				0.1 ms - 500 ms, randomly varied

Adjustable Step Size		0.1 ms

Uncertainty			<+/- 0.1 ms

### A5.4.2.3	Contact C7

The Elementary Time Unit (etu) used in the sections below refer to the nominal bit duration on the I/O line, as defined in ISO 7816-3.

Rise & fall Time setting uncertainty:	< +/- 100 ns

Jitter measurement uncertainty:		< +/- 5*10E-3 etu

Jitter setting uncertainty:		< +/- 5*10E-3 etu

## A5.4.3	Contact C3

Frequency measurement uncertainty:		< +/- 0.5 %

Voltage Measurement uncertainty:		< +/- 50 mV

Rise & fall time measurement uncertainty:	< +/- 5 ns

Duty cycle measurement uncertainty:		< +/- 2.5 %

## A5.4.4	Definition of timing

It shall be possible to define all timings relative to the clock. The SIM simulator shall be able to calculate and to use the absolute values automatically, even if the ME changes the frequency during the communication.

# History

| Document history |  |
| --- | --- |
| November 1994 | Public Enquiry PE 73: 1994-11-07 to 1995-03-03 (I-ETS 300 607) |
| November 1995 | Vote V 92: 1995-11-27 to 1996-01-19 |