## 26.9	Structured procedures

Abbreviations for message names used in this section:

AUTHENTICATION REQ	=	AUTHENTICATION REQUEST

AUTHENTICATION RESP	=	AUTHENTICATION RESPONSE

CIPHER MODE COMMAND	=	CIPHERING MODE COMMAND

CIPHER MODE COMPLETE	=	CIPHERING MODE COMPLETE

### 26.9.1	Structured procedures / general

The purpose of these tests is to verify that the MS performs certain elementary procedures of the RR, MM, and CC protocol correctly within a structured procedure. The term "structured procedure" is defined in TS GSM 04.08, section 7, where also examples of structured procedures are given.

The reason for this test purposes is twofold:

-	The behaviour of the MS in an elementary procedure may depend on the preamble which precedes the elementary procedure.

-	Structured procedures tested in this section are used in other parts of this Technical Specification as preambles to establish the initial conditions for other tests; correct behaviour of an implementation under test in a preamble is essential for the validity of a test.

Mobile originating and terminating calls are tested in cases of both early and late assignment of the traffic channel; in one of the cases call release initiated by the network is tested, in another one, call release initiated by the MS. The tests in this section only cover the successful outcome of elementary procedures (i.e. they do not deal with abnormal cases).

The emergency call service is tested in the following cases:

-	emergency call initiated in the idle, updated state with authentication and ciphering, for all supported speech rates,

-	emergency call initiated in the idle, no IMSI state (hence without authentication and without ciphering), the network accepting the call, for one supported speech rates,

-	emergency call initiated in the idle, no IMSI state (hence without authentication and without ciphering), the network rejecting the call, for one supported speech rate.

These tests on emergency calls are only applicable to an MS supporting speech.

For an MS supporting speech the test procedures in 26.9.2, 26.9.3, 26.9.4 and 26.9.5 are performed for speech (teleservice 11, telephony), once for each supported rate (full rate/half rate).

For an MS not supporting speech but supporting at least one teleservice, for each of the test procedures in 26.9.2, 26.9.3, 26.9.4, and 26.9.5 and each supported rate (full rate/half rate) a teleservice supported by the MS (see PICS/PIXIT statement) is chosen, and the test is performed corresponding to that teleservice (note that this teleservice is never a dual service).

In cases where a mobile originated call for the tested teleservice can be initiated both

-	via the MMI and

-	via the R or S interface,

procedure 26.9.2 shall be performed when initiating the mobile originated call via the MMI and procedure 26.9.3 shall be performed when initiating the mobile originated call via an appropriate interface (R or S).

### 26.9.2	Structured procedures / MS originated call / early assignment

26.9.2.1	Conformance requirements

1)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call, if it provides a human interface, shall display the dialled number.

2)	An MS in MM state "idle, updated" and in RR idle mode, when made to initiate a call for a selected teleservice that is supported by the MS, shall start to initiate the immediate assignment procedure by sending a CHANNEL REQUEST message with correct establishment cause.

3)	Subsequently after establishment of an MM connection, the MS shall send a SETUP message with correct parameters.

4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

5)	Subsequently when the network initiates call clearing by sending a DISCONNECT message, the MS shall proceed to release the call by sending a RELEASE message.

6)	On receipt of a CHANNEL RELEASE message, the MS shall disconnect the main signalling link.

References

Conformance requirement 1:	TS GSM 02.07.

Conformance requirement 2:	TS GSM 04.08 section 3.3.1.

Conformance requirement 3:	TS GSM 04.08 section 5.2.1.1.

Conformance requirement 4:	TS GSM 04.08 section 5.2.1.6.

Conformance requirement 5:	TS GSM 04.08 section 5.4.4.

Conformance requirement 6:	TS GSM 04.08, section 3.5.1.

26.9.2.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, displays the dialled number in the way described in a PICS/PIXIT statement.

2)	To verify that the MS in MM state "idle, updated" and in RR idle mode, with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

3)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed the authentication and cipher mode setting procedures, the MS sends a SETUP message with correct parameters.

4)	To verify that subsequently, after receipt of a CALL PROCEEDING message and of an ASSIGNMENT COMMAND message allocating an appropriate TCH, after having completed the traffic channel early assignment procedure by replying with the ASSIGNMENT COMPLETE message, after receipt of an ALERTING message and a CONNECT message, the MS returns a CONNECT ACKNOWLEDGE message.

5)	To verify that subsequently the MS has attached the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

6)	To verify that subsequently upon the network initiating call clearing by sending a DISCONNECT message, the MS proceed to release the call with RELEASE.

7)	To verify that subsequently, on receipt of a RELEASE COMPLETE message followed by a CHANNEL RELEASE message, the MS disconnects the main signalling link.

These test purposes are tested for all rates supported by the MS (full rate/half rate).

26.9.2.3	Method of test

Related PICS/PIXIT Statements

- 	Supported rates (full rate/half rate)

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	SS version

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The "called number" is entered |
| 2 | MS |  | If p1 = Y, the MS must display the called number in the way defined in PICS/PIXIT. |
| 3 | MS -> SS | CHANNEL REQUEST | Establishment cause is "originating call and the network does not set the NECI bit to 1". |
| 4 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 5 | MS -> SS | CM SERVICE REQUEST | Message is contained in SABM. |
| 6 | SS -> MS | AUTHENTICATION REQ |  |
| 7 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 8 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 9 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 10 | SS |  | SS starts ciphering. |
| 11 | MS -> SS | SETUP |  |
| 12 | SS -> MS | CALL PROCEEDING |  |
| 13 | SS -> MS | ASSIGNMENT COMMAND |  |
| 14 | MS -> SS | ASSIGNMENT COMPLETE |  |
| 15 | SS -> MS | ALERTING |  |
| 16 | MS |  | Depending on the PICS, an alerting indication is given |
| 17 | SS -> MS | CONNECT |  |
| 18 | MS -> SS | CONNECT ACKNOWLEDGE |  |
| 19 | MS |  | The appropriate bearer channel is through connected in both directions. |
| 20 | SS -> MS | DISCONNECT |  |
| 21 | MS -> SS | RELEASE |  |
| 22 | SS -> MS | RELEASE COMPLETE |  |
| 23 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents:

None

### 26.9.3	Structured procedures / MS originated call / late assignment

26.9.3.1	Conformance requirement

1)	An MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS , shall start to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

2)	Upon receipt of the ASSIGNMENT COMMAND message, the Mobile Station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the data links). After the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message, specifying cause 'normal event', to the network on the main DCCH.

3,4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

References

Conformance requirement 1:	TS GSM 04.08 section 3.3.1.

Conformance requirement 2:	TS GSM 04.08 section 3.4.3.1, 3.4.3.2

Conformance requirement 3:	TS GSM 04.08 section 5.2.1.6.

26.9.3.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message.

2)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed authentication and cipher mode setting procedures, after having sent a SETUP message, after having received a CALL PROCEEDING message followed by an ALERTING message and an ASSIGNMENT COMMAND message allocating an appropriate TCH, the MS sends an ASSIGNMENT COMPLETE message.

3)	To verify that subsequently, after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message returns a CONNECT ACKNOWLEDGE message.

4)	To verify that after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message attaches the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

These test purposes are tested for all rates supported by the MS (full rate/half rate).

26.9.3.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS has a MO call in state U10, "active".

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with late assignment.

Maximum Duration of Test

30 seconds

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The "called number" is entered |
| 2 | MS |  |  |
| 3 | MS -> SS | CHANNEL REQUEST | Establishment cause is "originating call and the network does not set the NECI bit to 1". |
| 4 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 5 | MS -> SS | CM SERVICE REQUEST | Message is contained in SABM. |
| 6 | SS -> MS | AUTHENTICATION REQ |  |
| 7 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 8 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 9 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 10 | SS |  | SS starts ciphering. |
| 11 | MS -> SS | SETUP |  |
| 12 | SS -> MS | CALL PROCEEDING |  |
| 13 | SS -> MS | ALERTING |  |
| 14 | MS |  | Depending on the PICS, an alerting indication is given. |
| 15 | SS -> MS | ASSIGNMENT COMMAND |  |
| 16 | MS -> SS | ASSIGNMENT COMPLETE |  |
| 17 | SS -> MS | CONNECT |  |
| 18 | MS -> SS | CONNECT ACKNOWLEDGE |  |
| 19 | MS |  | The appropriate bearer channel is through connected in both directions. |

Specific Message Contents:

None

### 26.9.4	Structured procedures / MS terminated call / early assignment

26.9.4.1	Conformance requirements

1)	The MS shall acknowledge the SETUP message with a CALL CONFIRMED message, if compatibility checking was successful, the MS is not busy, and the user does not refuse the call.

2,3)	The MS on acceptance of the call sends a CONNECT, otherwise user alerting is initiated (the MS sends an ALERTING message and generates an alerting indication). ASSIGNMENT COMMAND is answered by ASSIGNMENT COMPLETE.

4)	An MS indicates acceptance of a MT call by sending CONNECT.

5)

For speech calls:

The mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment shall be delayed until such a resource becomes available.

For data calls:

The mobile station shall attach the user connection when receiving the CONNECT ACKNOWLEDGE message from the network.

6)	The MS initiates call clearing of an active call by sending a DISCONNECT message.

7)	The MS in this phase of call release, upon receipt of a RELEASE message, shall return a RELEASE COMPLETE message.

8)	Subsequently the MS, upon receipt of a CHANNEL RELEASE message, shall disconnect the main signalling link.

References

Conformance requirement 1:		TS GSM 04.08, section 5.2.2.3.1.

Conformance requirements 2,3:	TS GSM 04.08, section 5.2.2.3.2, 3.4.3.1.

Conformance requirement 4:		TS GSM 04.08 section 5.2.2.5.

Conformance requirement 5:		TS GSM 04.08, section 5.2.2.6 and 5.2.2.9.

Conformance requirements 6,7,8:	TS GSM 04.08, section 5.4.

26.9.4.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, after being paged by the network on the correct paging subchannel, after initiating the immediate assignment procedure by sending the CHANNEL REQUEST message, after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after having sent a PAGING RESPONSE message on the allocated SDCCH, after having performed successful authentication and cipher mode setting procedures, after receipt of a SETUP message not containing a signal information element, returns a CALL CONFIRMED message.

2)	To verify that subsequently, the SS sending an ASSIGNMENT COMMAND message, the MS successfully continues a mobile terminating call establishment with early assignment of traffic channel

a)	by replying to the ASSIGNMENT command with an ASSIGNMENT COMPLETE message, and

b)	by continuing the call establishment by either sending one or two CONNECT messages (with equal N(SD)) or sending an ALERTING message, steps a) and b) being performed in any permitted interleaving.

3)	To verify that if after sending a CALL PROCEEDING message, the MS sends an ALERTING message during MTC establishment with early assignment, it generates an alerting indication.

4)	To verify that if an ALERTING had been sent, subsequently, when the user accepts the call (possibly internal action as declared in PICS/PIXIT statement), the MS returns a CONNECT message.

5)	To verify that the MS:

-	if the call is a speech call: after sending the CONNECT message has through connected the TCH in both directions (this is checked by verifying that after transmission of the first L2 frame containing the (complete) CONNECT message, the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

-	if the call is a data call: after receipt of a subsequent CONNECT ACKNOWLEDGE message through connects the TCH in both directions (this is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT ACKNOWLEDGE message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

6)	To verify that subsequently, the MS can initiate call clearing by sending a DISCONNECT message.

7)	To verify that the MS in this phase of call release, upon receipt of a RELEASE message, returns a RELEASE COMPLETE message.

8)	To verify that subsequently the MS, upon receipt of a CHANNEL RELEASE message, disconnects the main signalling link.

These test purposes are tested for all rates supported by the MS (full rate/half rate).

26.9.4.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Immediate connect supported (Y/N)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged and the resulting call is established. Having reached the active state, the MS is made to clear the call.

Maximum Duration of Test

1 minute

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAGING REQUEST TYPE 1 | Sent on the correct paging subchannel |
| 2 | MS -> SS | CHANNEL REQUEST |  |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 4 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 5 | SS -> MS | AUTHENTICATION REQ |  |
| 6 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 7 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 8 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 9 | SS |  | SS starts ciphering. |
| 10 | SS -> MS | SETUP | Message does not contain the signal IE. |
| 11 | MS -> SS | CALL CONFIRMED |  |
| 12 | SS -> MS | ASSIGNMENT COMMAND |  |
| A13 | MS -> SS | ASSIGNMENT COMPLETE | During step 12 or between step 12 and step 13 the MS may send a CONNECT message on the old channel. |
| A14 | MS -> SS | CONNECT | If during step 12 or between step 12 and step A13 the MS has send a CONNECT message on the old channel, both CONNECT messages must have equal N(SD). |
| B13 | MS -> SS | ASSIGNMENT COMPLETE | Sent on the new channel. |
| B14 | MS -> SS | ALERTING |  |
| B15 | MS |  | An alerting indication as defined in an PICS/PIXIT statement is given by the MS. |
| B16 | MS |  | The MS is made to accept the call in the way described in a PICS/PIXIT statement. |
| B17 | MS -> SS | CONNECT |  |
| 18 | MS |  | If the call is a speech call, the TCH shall be through connected in both directions. |
| 19 | SS -> MS | CONNECT ACKNOWLEDGE |  |
| 20 | MS |  | If the call is a data call, the TCH shall be through connected in both directions. |
| 21 | MS |  | The MS is made to release the call. |
| 22 | MS -> SS | DISCONNECT |  |
| 23 | SS -> MS | RELEASE |  |
| 24 | MS -> SS | RELEASE COMPLETE |  |
| 25 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents:

None

### 26.9.5	Structured procedures / MS terminated call / late assignment

26.9.5.1	Conformance requirement

TP1,TP2:	The MS shall acknowledge the SETUP message with a CALL CONFIRMED message, if compatibility checking was successful, the MS is not busy, and the user does not refuse the call. The MS on acceptance of the call sends a CONNECT, otherwise user alerting is initiated.

TP3:	The MS indicates acceptance of a call by sending a CONNECT message.

TP4:	ASSIGNMENT COMMAND is answered by ASSIGNMENT COMPLETE.

TP5:

For speech calls:

The mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment shall be delayed until such a resource becomes available.

For data calls:

The mobile station shall attach the user connection when receiving the CONNECT ACKNOWLEDGE message from the network.

Requirement reference:

Conformance requirements 1, 2, 3:	TS GSM 04.08, sections 5.2.2.3.1, 5.2.2.3.2, 5.2.2.5.

Conformance requirement 4:		TS GSM 04.08, section 3.4.3.1.

Conformance requirement 5:		TS GSM 04.08, section 5.2.2.9.

26.9.5.2	Test purpose

1)	To verify that the MS in "Idle, Updated" state with a TMSI assigned, after being paged by the network on the correct paging subchannel, after initiating the immediate assignment procedure by sending the CHANNEL REQUEST message, after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after having established the main signalling link, after having sent a PAGING RESPONSE message, after having performed successful authentication and cipher mode setting procedures, after receipt of a SETUP message containing a signal information element, returns a CALL CONFIRMED message followed by

-	an ALERTING message

-	or a CONNECT message.

2)	To verify that in the situation of test purpose 1, if the MS sends an ALERTING message, the MS generates an alerting indication in the way described in a PICS/PIXIT statement.

3)	To verify that subsequently the MS, if it had not yet sent a CONNECT message, upon acceptance of the call, sends a CONNECT message.

4)	To verify that subsequently after receipt of an ASSIGNMENT COMMAND, the MS sends an ASSIGNMENT COMPLETE message.

5)	To verify that subsequently the MS

-	if the call is a speech call: after sending the ASSIGNMENT COMPLETE message has through connected the TCH in both directions (this is checked by verifying that after transmission of the first L2 frame containing the (complete) ASSIGNMENT COMPLETE message, the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

-	if the call is a data call: after receipt of a subsequent CONNECT ACKNOWLEDGE message through connects the TCH in both directions (this is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT ACKNOWLEDGE message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

These test purposes are tested for all rates supported by the MS (full rate/half rate).

26.9.5.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Immediate connect supported (Y/N)

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

CC state U10-call active.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged and a MT call is established with late assignment (after CONNECT).

Maximum Duration of Test

40 seconds

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAGING REQUEST TYPE 1 | Sent on the correct paging subchannel |
| 2 | MS -> SS | CHANNEL REQUEST | Establishment cause indicates "answer to paging". |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 4 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 5 | SS -> MS | AUTHENTICATION REQ |  |
| 6 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 7 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 8 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 9 | SS |  | SS starts ciphering. |
| 10 | SS -> MS | SETUP | Message contains the signal IE. |
| 11 | MS -> SS | CALL CONFIRMED |  |
| A12 | MS -> SS | CONNECT |  |
| B12 | MS -> SS | ALERTING |  |
| B13 | MS |  | An alerting indication as defined in an PICS/PIXIT statement is given by the MS. |
| B14 | MS |  | The MS is made to accept the call in the way described in a PICS/PIXIT statement. |
| B15 | MS -> SS | CONNECT |  |
| 16 | SS -> MS | ASSIGNMENT COMMAND |  |
| 17 | MS -> SS | ASSIGNMENT COMPLETE |  |
| 18 | MS |  | If the call is a speech call, the TCH shall be through connected in both directions. |
| 19 | SS -> MS | CONNECT ACKNOWLEDGE |  |
| 20 | MS |  | If the call is a data call, the MS shall through connect the TCH in both directions. |

Specific Message Contents:

None

### 26.9.6	Structured procedures / emergency call

Emergency call establishment can be initiated by an MS whether location updating has been successful or not and whether a SIM is inserted into the MS or not; but only if the MS is equipped for speech.

If the procedures tested in this section are not correctly implemented in the MS, establishment, maintenance and clearing of connections might fail in the essential case of emergency calls.

The tests of this section are only applicable to an MS supporting speech.

#### 26.9.6.1	Structured procedures / emergency call / idle updated

##### 26.9.6.1.1	Structured procedures / emergency call / idle updated / preferred channel rate

26.9.6.1.1.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct CKSN and TMSI , with CM Service Type "emergency call establishment".

3)	Authentication and cipher mode setting shall be performed successfully.

4)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

5),6)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

7)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

8)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 3.4.7, TS GSM 04.08 section 4.3.2.

For conformance requirement 4:

TS GSM 04.08, section 5.2.1.1.

For conformance requirement 5 and 6:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 7:

TS GSM 04.08, sections 5.2.1.6 and 5.1.3.

For conformance requirement 8:

TS GSM 04.08, section 5.4.

26.9.6.1.1.2	Test purpose

1)	To verify that an MS supporting speech in the MM state "idle, updated", when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message specifying the correct CKSN and TMSI, with CM Service Type "emergency call establishment" .

3)	To verify that authentication and cipher mode setting are performed successfully.

4)	To verify that after cipher mode setting acceptance by the SS, the MS sends an EMERGENCY SETUP message.

5)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure of an appropriate speech traffic channel, which, if the MS supports both TCH/FS and TCH/HS, is at the preferred rate, the MS performs correctly that assignment procedure.

6)	To verify subsequent correct performance of a connect procedure.

7)	To verify that subsequently the MS has through connected the TCH in both directions.

8)	To verify that the call is cleared correctly.

26.9.6.1.1.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Supported rate for speech: (p1 = F/H, F).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The MS is made to initiate an emergency call. The call is established with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The "called number" 112 is entered |
| 3 | MS -> SS | CHANNEL REQUEST | Establishment cause is emergency call establishment. |
| 4 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 5 | MS -> SS | CM SERVICE REQUEST | Message is contained in SABM. The CM service type IE indicates "emergency call establishment". |
| 6 | SS -> MS | AUTHENTICATION REQ |  |
| 7 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 8 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 9 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 10 | SS |  | SS starts ciphering. |
| 11 | MS -> SS | EMERGENCY SETUP | If p1 = F/H, the message must contain one bearer capability IE indicating in the radio channel requirement field "dual rate/half rate preferred" or "dual rate/full rate preferred". If p1 = F, the message must either contain no bearer capability IE or contain one bearer capability IE indicating in the radio channel requirement field "full rate channel". |
| 12 | SS -> MS | CALL PROCEEDING |  |
| 13 | SS -> MS | ALERTING |  |
| 14 | SS -> MS | ASSIGNMENT COMMAND | The rate of the channel is that one indicated by the EMERGENCY SETUP message, if that message did not offer a choice, and the rate is the preferred one else. |
| 15 | MS -> SS | ASSIGNMENT COMPLETE |  |
| 16 | SS -> MS | CONNECT |  |
| 17 | MS -> SS | CONNECT ACKNOWLEDGE |  |
| 18 | MS |  | The TCH is through connected in both directions. |
| 19 | SS -> MS | DISCONNECT |  |
| 20 | MS -> SS | RELEASE |  |
| 21 | SS -> MS | RELEASE COMPLETE |  |
| 23 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents:

None

##### 26.9.6.1.2	Structured procedures / emergency call / idle updated, non-preferred channel rate

The test is performed if the MS supports both TCH/HS and TCH/FS see PICS/PIXIT statement).

It is identical to the test in 26.9.6.1.1 except that in step 14 the assigned TCH has the non-preferred rate.

#### 26.9.6.2	Structured procedures / emergency call / idle, no IMSI

##### 26.9.6.2.1	Structured procedures / emergency call / idle, no IMSI / accept case

26.9.6.2.1.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

4),5)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

6)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

7)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 5.2.1.1.

For conformance requirements 4 and 5:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 6:

TS GSM 04.08, sections 5.2.1.6 and 5.1.3.

For conformance requirement 7:

TS GSM 04.08, section 5.4.

26.9.6.2.1.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message in which the cipher key sequence number IE indicates "no key is available", the CM service type IE indicates "emergency number establishment", and the mobile identity IE specifies the IMEI of the MS.

3)	To verify that after receipt of a CM SERVICE ACCEPT message from the SS, the MS sends an EMERGENCY SETUP message.

4)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure of an appropriate speech traffic channel, which, if the MS supports both TCH/FS and TCH/HS, is at the preferred rate, the MS performs correctly that assignment procedure.

5)	To verify subsequent correct performance of a connect procedure.

6)	To verify that subsequently the MS has through connected the TCH in both directions.

7)	To verify that the call is cleared correctly.

26.9.6.2.1.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute.

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The "called number" 112 is entered |
| 3 | MS -> SS | CHANNEL REQUEST | Establishment cause is "emergency call". |
| 4 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 5 | MS -> SS | CM SERVICE REQUEST | Message is contained in SABM. The CM service type IE indicates "emergency call establishment". The mobile identity IE specifies the IMEI of the MS. The cipher key sequence number IE indicates "no key is available". The mobile station classmark IE is as specified by the manufacturer in a PICS/PIXIT statement. |
| 4 | SS -> MS | CM SERVICE ACCEPT |  |
| 11 | MS -> SS | EMERGENCY SETUP |  |
| 12 | SS -> MS | CALL PROCEEDING |  |
| 13 | SS -> MS | ALERTING |  |
| 14 | SS -> MS | ASSIGNMENT COMMAND | The rate of the channel is one indicated by the EMERGENCY SETUP message. |
| 15 | MS -> SS | ASSIGNMENT COMPLETE |  |
| 16 | SS -> MS | CONNECT |  |
| 17 | MS -> SS | CONNECT ACKNOWLEDGE |  |
| 18 | MS |  | The TCH is through connected in both directions. |
| 19 | SS -> MS | DISCONNECT |  |
| 20 | MS -> SS | RELEASE |  |
| 21 | SS -> MS | RELEASE COMPLETE |  |
| 23 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents:

None

##### 26.9.6.2.2	Structured procedures / emergency call / idle, no IMSI / reject case

26.9.6.2.2.1	Conformance requirement

1)	The MS in the "idle, no IMSI" state (no SIM inserted), after the number 112 has been entered, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	In the situation at the end of test purpose 2, when the MS receives a CM SERVICE REJECT message, it shall abandon the emergency call.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4

For conformance requirement 3:

TS GSM 04.08 section 3.4.7, TS GSM 04.08 section 4.5.1.1.

26.9.6.2.2.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message in which the cipher key sequence number IE indicates "no key is available", the CM service type IE indicates "emergency call establishment", and the mobile identity IE specifies the IMEI of the MS.

3)	To verify that after receipt of a CM SERVICE REJECT message from the SS, the MS abandons the emergency call establishment.

26.9.6.2.2.3	Method of test

Related PICS/PIXIT statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. The SS responds to the CM SERVICE REQUEST from the MS with a CM SERVICE REJECT message specifying in the reject cause IE the reject cause value "IMEI not accepted". The SS then verifies for during 5 seconds that the MS does not send a layer 3 message. Then the call is cleared by the SS. The SS verifies during 20 seconds after disconnection of the main signalling link that the MS does not initiate an RR connection establishment.

Maximum Duration of Test

1 minute.

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The "called number" 112 is entered |
| 3 | MS -> SS | CHANNEL REQUEST | Establishment cause is "emergency call". |
| 4 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 5 | MS -> SS | CM SERVICE REQUEST | Message is contained in SABM. The CM service type IE indicates "emergency call establishment". The mobile identity IE specifies the IMEI of the MS. The cipher key sequence number IE indicates "no key is available". The mobile station classmark IE is as specified by the manufacturer in a PICS/PIXIT statement. |
| 4 | SS -> MS | CM SERVICE REJECT | the reject cause IE specifies reject cause value #5, "IMEI not accepted". |
| 5 | SS |  | During 5 seconds, the SS verifies that the MS does not send L3 messages. |
| 6 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 7 | SS |  | During 20 seconds, the SS verifies that the MS does not initiate an RR connection establishment. |

Specific Message Contents:

-

### 26.9.7	Default contents of messages

ALERTING (mobile station to network direction)

| Information element | Value/remark |
| --- | --- |
| Facility | Omitted |
| User-user | Omitted |
| SS version | Omitted |

ALERTING (network to mobile station direction)

| Information element | Value/remark |
| --- | --- |
| Facility | Omitted |
| Progress indicator | Omitted |
| User-user | Omitted |

ASSIGNMENT COMMAND

| Information element | Value/remark |
| --- | --- |
| Description of the first channel | describes non-hopping Bm+ACCHs or Lm+ACCHs as appropriate for the test |
| Power Command | As in section 26.1.1 |
| Frequency list | Omitted |
| Cell channel description | Omitted |
| Mode of the first channel | appropriate for on bearer capability chosen for the test |
| Description of the second channel | Omitted |
| Mode of the second channel | Omitted |
| Mobile allocation | Omitted |
| Starting time | Omitted |
| Cipher mode setting | Omitted |

ASSIGNMENT COMPLETE

| Information element | Value/remark |
| --- | --- |
| RR cause | normal event |

AUTHENTICATION REQUEST

| Information element | Value/remark |
| --- | --- |
| Ciphering key sequence number | Arbitrary |
| Spare half octet | (spare bits) |
| Authentication parameter RAND | Arbitrary |

AUTHENTICATION RESPONSE

| Information element | Value/remark |
| --- | --- |
| Authentication parameter SRES | Correct for given SRES |

CALL CONFIRMED

| Information element | Value/remark |
| --- | --- |
| Repeat indicator | Omitted |
| Bearer capability 1 | Omitted if the MS only supports TCH/F. Included if the MS supports both TCH/F and TCH/H; in that case, the "radio channel requirement" field specifies "dual rate/half rate preferred" or "dual rate/full rate preferred" or "half rate channel", all other parameters are same as in the bearer capability 1 IE of the received SETUP message. |
| Bearer capability 2 | Omitted |
| Cause | Omitted |

CALL PROCEEDING

| Information element | Value/remark |
| --- | --- |
| Repeat Indicator | Omitted |
| Bearer Capability 1 | Omitted if the SETUP message did not specify in the bearer capability 1 IE a connection element value "both, transparent preferred" or "both, non- transparent preferred". Otherwise included; in that case the connection element specifies the value that is appropriate for the selected teleservice (either value "transparent" or value "non transparent (RLP)"), all other parameters are same as in the bearer capability 1 IE of the received SETUP message. |
| Bearer Capability 2 | Omitted |
| Facility | Omitted |
| Progress indicator | Omitted |

CHANNEL RELEASE

| Information element | Value/remark |
| --- | --- |
| RR cause | Normal event |

CHANNEL REQUEST

| Information element | Value/remark |
| --- | --- |
| Establishment cause | Answer to paging (100) |
| Random reference | Arbitrary value of 5 bits length |

CIPHERING MODE COMMAND

| Information element | Value/remark |
| --- | --- |
| Cipher mode setting |  |
| algorithm identifier | indicates a supported algorithm |
| SC | Start ciphering |
| Cipher response |  |
| CR | IMEI must not be included |

CIPHER MODE COMPLETE

| Information element | Value/remark |
| --- | --- |
| Mobile equipment identity | Omitted |

CM SERVICE ACCEPT

| Information element | Value/remark |
| --- | --- |
| none but message head |  |

CM SERVICE REQUEST

| Information element | Value/remark |
| --- | --- |
| CM service type | Mobile originating call establishment or packet mode connection establishment |
| Ciphering key sequence number | CKSN of the MS |
| Mobile station classmark 2 | as given by PICS. |
| Mobile identity | TMSI of MS |

CONNECT (network to mobile station direction)

| Information element | Value/remark |
| --- | --- |
| Facility | Omitted |
| Progress indicator | Omitted |
| Connected number | Omitted |
| Connected subaddress | Omitted |
| User-user | Omitted |

CONNECT (mobile station to network direction)

| Information element | Value/remark |
| --- | --- |
| Facility | Omitted |
| Connected subaddress | Omitted |
| User-user | Omitted |
| SS version | Omitted |

CONNECT ACKNOWLEDGE

| Information element | Value/remark |
| --- | --- |
| none but message head |  |

DISCONNECT (network to mobile station direction)

| Information element | Value/remark |
| --- | --- |
| Cause |  |
| Coding standard | GSM |
| Location | User |
| Cause value | Normal clearing |
| Facility | Omitted |
| Progress indicator | Omitted |
| User-user | Omitted |

DISCONNECT (mobile station to network direction)

| Information element | Value/remark |
| --- | --- |
| Cause |  |
| Coding standard | GSM |
| Location | User |
| Cause value | Normal clearing |
| Facility | Omitted |
| User-user | Omitted |
| SS version | Omitted |

IMMEDIATE ASSIGNMENT

| Information element | Value/remark |
| --- | --- |
| Page mode | Normal paging |
| Channel description | describes a valid SDCCH+SACCH in non-hopping mode |
| Request reference |  |
| Random access information | As received from MS |
| N51, N32, N26 | Corresponding to frame number of the CHANNEL REQUEST |
| Timing advance | Arbitrary |
| Mobile allocation | Empty (L=0) |
| Starting time | Omitted |

PAGING REQUEST TYPE 1

| Information element | Value/remark |
| --- | --- |
| L2 pseudo length | L2 pseudo length of the message |
| Page Mode | Normal Paging |
| Channels needed for Mobiles 1 and 2 |  |
| channel (first) | any channel |
| channel (second) | any channel |
| Mobile identity 1 | TMSI of MS under test |
| Mobile identity 2 | Omitted |
| P1 rest octets | (spare octets) |

PAGING RESPONSE

| Information element | Value/remark |
| --- | --- |
| Ciphering key sequence number | Value assigned to MS in the initial conditions |
| Spare half octet | (spare bits) |
| Mobile station classmark 2 | as given by PICS |
| Mobile identity | specifies TMSI of MS |

RELEASE (network to mobile station direction)

| Information element | Value/remark |
| --- | --- |
| Cause | Omitted |
| Second cause | Omitted |
| Facility | Omitted |
| User-user | Omitted |

RELEASE (mobile station to network direction)

| Information element | Value/remark |
| --- | --- |
| Cause | Omitted |
| Second cause | Omitted |
| Facility | Omitted |
| User-user | Omitted |
| SS version | Omitted |

RELEASE COMPLETE (network to mobile station direction)

| Information element | Value/remark |
| --- | --- |
| Cause | Omitted |
| Facility | Omitted |
| User-user | Omitted |

RELEASE COMPLETE (mobile station to network direction)

| Information element | Value/remark |
| --- | --- |
| Cause | Omitted |
| Facility | Omitted |
| User-user | Omitted |
| SS version | Omitted |

SETUP (MS to SS)

| Information element | Value/remark |
| --- | --- |
| BC Repeat indicator | Omitted |
| Bearer capability 1 | Appropriate for the teleservice selected for the test |
| Bearer capability 2 | Omitted |
| Facility | Omitted |
| Calling party subaddress | Omitted |
| Called party BCD number | As entered |
| Called party subaddress | Omitted |
| LLC repeat indicator | Omitted |
| Low layer compatibility I | Appropriate for teleservice selected for the test |
| Low layer compatibility II | Omitted |
| HLC repeat indicator | Omitted |
| High layer compatibility i | Appropriate for teleservice selected for the test |
| High layer compatibility ii | Omitted |
| User-user | Omitted |
| SS version | Omitted |
| CLIR suppression | Omitted |
| CC Capabilities | present, shall indicate support for DTMF as per section 5.5.7 of TS GSM 04.08 |

SETUP (SS to MS)

| Information element | Value/remark |
| --- | --- |
| BC repeat indicator | Omitted |
| Bearer capability 1 | Appropriate for teleservice selected for the test |
| Bearer capability 2 | Omitted |
| Facility | Omitted |
| Progress indicator | Omitted |
| Signal | Omitted |
| Calling party BCD number | Omitted |
| Calling party subaddress | Omitted |
| Called party BCD number | Omitted |
| Called party subaddress | Omitted |
| LLC repeat indicator | Appropriate for the teleservice selected for the test |
| Low layer compatibility I | Appropriate for teleservice selected for the test |
| Low layer compatibility II | Appropriate for the teleservice selected for the test |
| HLC repeat indicator | Appropriate for the teleservice selected for the test |
| High layer compatibility i | Appropriate for the teleservice selected for the test |
| High layer compatibility ii | Appropriate for the teleservice selected for the test |
| User-user | Omitted |