## 26.10	E-GSM signalling

### 26.10.1	E-GSM signalling / general considerations

Section 26.10 only applies to E-GSM mobile stations.

Conformance requirements of section 26 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26 under the described GSM900 conditions.

The purpose of this extra section is to test the different procedures which may be impacted when some channel uses E-GSM frequency(s).

### 26.10.2	E-GSM signalling / RR

#### 26.10.2.1	E-GSM signalling / RR / Measurement

This test applies E-GSM mobile stations.

Conformance requirements of section 26.6.3 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.3 under the described GSM900 conditions.

The purpose of this extra section is to test the reporting of measurements in the case where cells use E-GSM frequency(s). Several coding formats may be used by the network in the SYSTEM INFORMATION message.

26.10.2.1.1	Conformance requirements

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the values in the MEASUREMENT REPORT message shall contain measurement results for the 6 th strongest cells on which the mobile is asked to report.

References

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

26.10.2.1.2	Test purpose

To test that, when the SS gives information about neighbouring cells, the MS reports appropriate results.

26.10.2.1.3	Method of test

Initial Conditions

System Simulator:

8 cells with the following settings:

Transmitter	Level	NCC	BSCC	ARFCN	Cell identity

Serving, S1	-60	1	3	002	0001H

Neighbour, N1	-85	1	5	990	0002H

Neighbour, N2	-80	1	7	1005	0003H

Neighbour, N3	-75	1	1	000	0004H

Neighbour, N4	-55	1	3	026	0005H

Neighbour, N5	-50	1	5	1020	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	1003	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

Mobile Station:

The MS is in the active state of a call (U10).

Related PICS/PIXIT Statements

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

Active state of a call (U10).

Test Procedure

This test procedure is performed 6 times.

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5, 5bis & 6  on the SACCH. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest present carriers of the supported band have been obtained.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for execution counter, c= 1 to 6.

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT  are sent continuously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents

SYSTEM INFORMATION TYPE 5:

| Information Element | value/remark |
| --- | --- |
| Protocol Discriminator | RR management |
| Message Type | Sys Info 5. |
| Neighbour Cells Description |  |
| - EXT IND | Information Element carries only a part of the BA. |
| - BA-IND | 0 for c=1, use range 128 to encode the following frequencies : (26, 38) for c=2, use range 256 to encode the following frequencies (990, 1003, 1005) for c=3, use range 512 to encode the following frequencies (520, 990, 1003, 1005, 1020) for c=4, use range 1024 to encode the following frequencies (0, 26, 38, 990, 1003, 1005) for c=5, use variable Bitmap to encode the following frequencies (0, 26, 38) for c=6, use Bitmap 0 to encode the following frequencies (26) |

SYSTEM INFORMATION TYPE 5bis:

| Information Element | value/remark |
| --- | --- |
| Protocol Discriminator | RR management |
| Message Type | Sys Info 5bis. |
| Neighbour Cells Description |  |
| - EXT IND | Information Element carries only a part of the BA. |
| - BA-IND | 0 for c=1, use range 512 to encode the following frequencies : (520, 990, 1003, 1005, 1020) for c=2, use range 128 to encode the following frequencies (0, 26, 38) for c=3, use range 256 to encode the following frequencies (0, 26, 38) for c=4, use range 1024 to encode the following frequencies (520, 1000, 1020) for c=5, use variable Bitmap to encode the following frequencies (884, 990, 1003, 1005) for c=6, use range 512 to encode the following frequencies (520, 990, 1003) |

SYSTEM INFORMATION TYPE 6:

| Information Element | value/ remark |
| --- | --- |
| Protocol Discriminator | RR Management |
| Message Type | sys info 6 |
| Cell Identity | default |
| LAI | default |
| Cell Options |  |
| - Power Control Indicator | Power Control Indicator is set |
| - DTX Indicator | MS shall not use DTX |
| - Radio-Link-Timeout | default |
| PLMN permitted | only NCC 1 permitted |

MEASUREMENT REPORT:

| Information Element | value/remark |
| --- | --- |
| Protocol Discriminator | RR Management |
| Transaction Identifier | 0000 |
| Message Type | MEASUREMENT REPORT |
| Measurement Results |  |
| BA-used | 1 |
| DTX-used | DTX was not used |
| RXLEV-FULL-SERVING-CELL | See note 1 |
| RXLEV-SUB-SERVING-CELL | See note 1 |
| MEAS-VALID | See note 3 |
| RXQUAL-FULL-SERVING-CELL | See note 1 |
| RXQUAL-SUB-SERVING-CELL | See note 1 |
| NO-NCELL-M | n (see note 2) |
| RXLEV-NCELL-1 | See note 1 |
| BCCH-FREQ-NCELL-1 | See note 2 |
| BSIC-NCELL-1 | Corresponds to that of BCCH-FREQ-NCELL-1 |
| RXLEV-NCELL-2 | See note 1 |
| BCCH-FREQ-NCELL-2 | See note 2 |
| BSIC-NCELL-2 | Corresponds to that of BCCH-FREQ-NCELL-2 |
| RXLEV-NCELL-3 | See note 1 |
| BCCH-FREQ-NCELL-3 | See note 2 |
| BSIC-NCELL-3 | Corresponds to that of BCCH-FREQ-NCELL-3 |
| RXLEV-NCELL-4 | See note 1 |
| BCCH-FREQ-NCELL-4 | See note 2 |
| BSIC-NCELL-4 | Corresponds to that of BCCH-FREQ-NCELL-4 |
| RXLEV-NCELL-5 | See note 1 |
| BCCH-FREQ-NCELL-5 | See note 2 |
| BSIC-NCELL-5 | Corresponds to that of BCCH-FREQ-NCELL-5 |
| RXLEV-NCELL-6 | See note 1 |
| BCCH-FREQ-NCELL-6 | See note 2 |
| BSIC-NCELL-6 | Corresponds to that of BCCH-FREQ-NCELL-6 |

NOTE 1:	These actual values are not checked.

NOTE 2 :	ARFCN 2 is the serving cell carrier.

c=1 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 1020, n=6

c=2 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0, 1020 stronger than 1005 but not broadcasted in SYS INFO), n=6

c=3 report on ARFCNs  2, 20, 26, 38, 1003, 1005, 1020, 0 (990 less strong, 520 DCS), n=6

c=4 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0 (1000 less strong, 520 DCS, 1020 not broadcasted in SYS INFO), n=6

c=5 report on ARFCNs 2, 26, 38, 990, 1003, 1005, 0 (884 DCS), n=6

c=6 report on 2, 26, 990, 1003,  n=6

NOTE 3:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

#### 26.10.2.2	E-GSM signalling / RR / Immediate assignment

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.1 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.1 under the described GSM900 conditions.

The purpose of this extra section is to test the immediate assignment procedure in the case where the target channel is E-GSM frequency.

26.10.2.2.1	Conformance requirement

Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on a supported channel described in the IMMEDIATE ASSIGNMENT message, using some E-GSM frequency.

Reference(s)

GSM 04.08 section 3.3.1.2.1

26.10.2.2.2	Test purpose

To verify that the MS can correctly set up a dedicated control channel when E-GSM frequencies are used.

This tested for a SDCCH/8

26.10.2.2.3	Method of test

Initial Conditions

System Simulator:

1 cell,

CCCH-CONF is set to '1 basic physical channel used for CCCH not combined with SDCCHs'.

SYSTEM INFORMATION type 1 message contains the following frequencies in the Cell Channel Description IE: 0, 30, 40, 66, 80, 1005, 1010, 1015 (use range 1024 to encode)

BCCH carrier number 1015

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Related PICS/PIXIT Statement(s)

none

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

This test procedure is performed twice.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST message the SS assigns an SDCCH channel using some E-GSM frequencies. The MS shall go to the correct channel and send a PAGING RESPONSE message. Then the SS initiates RR-release by sending a CHANNEL RELEASE message.

Maximum Duration of Test

15 seconds

Expected Sequence

The sequence is performed for execution counter k=1,2

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAG REQ TYPE 1 |  |
| 2 | MS -> SS | CHANNEL REQUEST | cause "answer to paging" |
| 3 | SS -> MS | IMM ASSIGNMENT | Channel Type : see below |
| 4 | MS -> SS | PAGING RESPONSE | Shall be sent on the correct channel |
| 5 | SS -> MS | CHANNEL RELEASE |  |

Specific Message Contents

IMMEDIATE ASSIGNMENT

as default except :

| Information element | remark/value |
| --- | --- |
| Channel description |  |
| - Channel Type | SDCCH/8 |
| - Timeslot number | arbitrary but not zero |
| - Training sequence code | arbitrary |
| - Hopping channel | k=1 Single RF k=2 RF hopping channel |
| - Channel Selector | k=1 ARFCN=1015 k=2 MAIO = arbitrarily chosen HSN arbitrary chosen from the set (1,..63) |
| Mobile allocation | k=1 empty k=2 indicates the following frequencies (0, 80, 1005, 1010) |

#### 26.10.2.3	E-GSM signalling / RR / channel assignment procedure

This test is only applicable to an E-GSM mobile station.

Conformance requirements of section 26.6.4 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.4 under the described GSM900 conditions.

The purpose of this extra section is to test the assignment in the case where the allocated channel uses E-GSM frequency(s) : in the case of frequency hopping several coding formats may be used by the network in the ASSIGNMENT COMMAND message.

26.10.2.3.1	Conformance requirements

1.	Upon receipt of the ASSIGNMENT COMMAND message, the mobile station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the main signalling links).

2.	The MS shall apply the hopping frequencies specified in ASSIGNMENT COMMAND message in the mobile allocation or frequency list or frequency short list at the time accessing the new channel.

References

GSM 04.08, subclause  3.4.3

GSM 04.08 subclause 9.1.2

26.10.2.3.2	Test purpose

1.	To verify that upon receipt of an ASSIGNMENT COMMAND, the MS switches to the channel defined in the ASSIGNMENT COMMAND, establishes the link and sends an ASSIGNMENT COMPLETE message.

2.	To verify that an MS, having received an ASSIGNMENT COMMAND, is able in case of frequency hopping to decode the mobile allocation and frequency list correctly and applies the specified frequencies.

26.10.2.3.3	Method of test

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state with a TMSI allocated.

Related PICS/PIXIT Statements

-	TCH supported (Y/N)

-	The supported channel mode(s) need to be declared.

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test Procedure

The test procedure is performed 2 times.

The SS pages the MS and allocates an SDCCH. Then a channel is assigned with ASSIGNMENT COMMAND. Each time the MS shall switch to the assigned channel, establish the link and send an ASSIGNMENT COMPLETE message.

The SS initiates the channel release procedure.

Maximum Duration of Test

3 min

Expected Sequence

The test sequence is performed for execution counter k=1 to 2

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAG REQ TYPE 1 |  |
| 2 | MS -> SS | CHANNEL REQUEST |  |
| 3 | SS -> MS | IMMEDIATE ASSIGN | Timeslot Number = n. |
| 4 | MS -> SS | PAGING RESPONSE |  |
| 5 | SS -> MS | ASSIGN COMMAND | See specific message contents Timeslot Number = (n+1) mod 8 |
| 6 | MS -> SS | ASSIGN COMPLETE | Sent on the correct channel after establishment of the main signalling link. Steps 5 and 6 are repeated cmax times, where cmax is the number of frequency formats allowed for each value of k. Use repetition counter c: See specific message content. |
| 7 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents

ASSIGNMENT COMMAND:

| Information element | value/remark |
| --- | --- |
| Protocol Discriminator | RR |
| Skip indicator | 0000 |
| Message type | ASSIGNMENT COMMAND |
| Channel Description |  |
| - Channel type | TCH/F + ACCHs if supported by the MS or SDCCH/8 if not |
| - Timeslot number | arbitrary |
| - Training sequence code | chosen arbitrarily |
| - Hopping | RF hopping channel |
| - MAIO | arbitrary |
| - HSN | arbitrarily chosen from the set (1,2...63) |
| Power Command |  |
| - Power level | Arbitrarily chosen |
| For k=1 |  |
| Cell Channel Description IE is not included |  |
| Frequency list | for c=1, use range 128 to encode the following frequencies : (1005, 1010, 1015) for c=2, use range 256 to encode the following frequencies (73, 74, 75, 76, 77) for c=3, use range 512 to encode the following frequencies (980, 981, 982, 983) for c=4, use range 1024 to encode the following frequencies (30, 40, 1010, 1015) for c=5, use variable Bitmap to encode the following frequencies (980, 991, 992, 993, 994, 1015) with an encoding origin set to 900 for c=6, use Bitmap 0 to encode the following frequencies (20, 40, 66) |
| For k=2 |  |
| Cell Channel Description | for c=1, use range 128 to encode the following 13 frequencies : (980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015) for c=2, use range 256 to encode the following 13 frequencies (20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114, 115) for c=3, use range 512 to encode the following 13 frequencies (980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015) for c=4, use range 1024 to encode the following 8 frequencies (0, 30, 40, 66, 80, 1005, 1010, 1015) for c=5, use variable Bitmap to encode the following frequencies (980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015) for c=6, use Bitmap 0 to encode the following 12 frequencies (20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114) |
| For k = 2 |  |
| Mobile Allocation | indicates the following the frequencies : for c=1 (1005, 1010, 1015) for c=2 (73, 74, 75, 76, 77) for c=3 (980, 981, 982, 983) for c=4 (0, 30, 40, 1010, 15) for c=5 (990, 991, 992, 993, 994) for c=6 (20, 40, 66) |
| Mode of the First channel | speech full rate for TCH/F except if speech is not supported : arbitrary from those supported |
| Starting Time | not included |

#### 26.10.2.4	E-GSM signalling / RR / Handover

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.5 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.5 under the described GSM900 conditions.

The purpose of this extra section is to test the handover in the case where the target channel uses E-GSM frequency(s) : in the case of frequency hopping several coding formats may be used by the network in the HANDOVER COMMAND message.

##### 26.10.2.4.1	E-GSM signalling / RR / Handover / Successful handover

26.10.2.4.1.1	Conformance requirements

The MS shall correctly apply the handover procedure from a channel without frequency hopping in the primary band to a channel with frequency hopping using P-GSM and E-GSM frequencies whatever the coding used for the frequency hopping description.

References

GSM 04.08, section 3.4.4 and 9.1.15

26.10.2.4.1.2	Test purpose

To check that the MS correctly performs a non-synchronized handover, from a non hopping primary band SDCCH to a hopping TCH or SDCCH using E-GSM frequencies, whatever the coding used for the hopping sequence description and that it activates the new channel correctly.

This is tested in the following case :

E-GSM signalling / Handover / successful / call under establishment / non-synchronized /

- from SDCCH/8 to TCH/F if the MS supports a TCH

- from SDCCH/8 to SDCCH/8 if not

26.10.2.4.1.3	Method of test

Initial Conditions

System Simulator:

2 cells A and B with same LAI, default parameters

except for Cell A : the broadcasted Cell Channel Description in SYSTEM INFORMATION type 1 message contains the following frequencies : 20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114 (encoded using Bit Map 0 format)

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Related PICS/PIXIT Statements

Support of TCH (Y/N)

Support of speech (Y/N)

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

"Idle, updated" with TMSI allocated and camped on cell B.

Test Procedure

The test procedure is performed 18 times.

A Mobile Originating Call is initiated on cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND message, ordering the MS to switch to cell B.

The MS shall then begin to send access bursts on the new channel, a TCH/F if supported (see PICS/PIXIT) or an SDCCH if not, to cell B.

The SS observes the access bursts. After receiving n access bursts, n being randomly drawn between 10 and 20 for the TCH case, 2 and 5 for the SDCCH (see table 2 of section 26.6.5) , the SS sends one PHYSICAL INFORMATION message with a Timing Advance of 20 (see table 1 of section 26.6.5).

The MS shall activate the channel in sending and receiving mode. Then the MS shall establish a signalling link using the correct timing advance. The MS shall be ready to transmit a HANDOVER COMPLETE message before x ms after the end of the PHYSICAL INFORMATION message but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

The term 'ready to transmit' is specified in TS GSM 04.13. The value of 'x' depends upon the target channel:

case SDCCH/8 x = 750

case TCH/F x=500

Maximum Duration of Test

10 min

Expected Sequence

The sequence is performed for execution counter k=1 to 3 and c=1 to 6

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | -------- | -------- | A MO call is initiated on cell A |
| 2 | MS -> SS | CHANNEL REQUEST | Establ. Cause = "Originating call, NECI not set to 1 |
| 3 | SS -> MS | IMM ASSIGNMENT | see specific message contents |
| 4 | MS -> SS | CM SERVICE REQUEST | CM Service Type = Mobile Origina- ting Call Establishment. |
| 5 | SS -> MS | CIPH MODE COMMAND |  |
| 6 | MS -> SS | CIPH MODE COMPLETE |  |
| 7 | MS -> SS | SETUP | Last L2 frame not acknowledged by the SS. |
| 8 | SS -> MS | HANDOVER COMMAND | see specific message contents |
| 9 | MS -> SS | HANDOVER ACCESS | Repeated on every burst of the uplink main DCCH until reception of PHYSICAL INFORMATION. Handover Reference as included in the HANDOVER COMMAND |
| 10 | SS -> MS | PHYSICAL INFO | Sent after reception of n HANDOVER ACCESS. Timing Advance : 20 |
| 11 | MS -> SS | SABM | Sent without information field |
| 12 | SS -> MS | UA |  |
| 13 | MS -> SS | HANDOVER COMPLETE | This message shall be ready to be transmitted before 'x' ms after the completion of step 10 |
| 14 | MS -> SS | SETUP | Same N(SD) as in step 7. |
| 15 | SS -> MS | CHANNEL RELEASE |  |

Specific Message Contents

IMMEDIATE ASSIGNMENT

| Information Element | Value/remark |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - BCCH carrier number | 20 |
| Channel Description |  |
| - Channel type | TCH/F + ACCHs if supported by the MS or SDCCH/8 if not |
| - Timeslot number | arbitrary but not zero |
| - Training sequence code | chosen arbitrarily |
| - Hopping | RF hopping channel |
| - MAIO | arbitrary |
| - HSN | arbitrarily chosen from the set (1,2...63) |
| Mobile Allocation | indicates the following frequencies : (20, 66, 78) |

HANDOVER COMMAND

| Information Element | Value/remark |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - BCCH carrier number | 40 |
| Channel Description |  |
| - Channel type | TCH/F + ACCHs if supported by the MS or SDCCH/8 if not |
| - Timeslot number | arbitrary but not zero |
| - Training sequence code | chosen arbitrarily |
| - Hopping | RF hopping channel |
| - MAIO | arbitrary |
| - HSN | arbitrarily chosen from the set (1,2...63) |
| Synchronisation Indication IE is not included |  |
| For k = 1, |  |
| Cell Channel Description IE is not included |  |
| Frequency short list | for c=1, use range 128 to encode the following frequencies : (1005, 1010, 1015) for c=2, use range 256 to encode the following frequencies (73, 74, 75, 76, 77) for c=3, use range 512 to encode the following frequencies (980, 981, 982, 983) for c=4, use range 1024 to encode the following frequencies (30, 40, 1010, 1015) for c=5, use variable Bitmap to encode the following frequencies (980, 991, 992, 993, 994, 1015) |
| For k=2 |  |
| Cell Channel Description IE is not included |  |
| Frequency list | for c=1, use range 128 to encode the following frequencies : (1005, 1010, 1015) for c=2, use range 256 to encode the following frequencies (73, 74, 75, 76, 77) for c=3, use range 512 to encode the following frequencies (980, 981, 982, 983) for c=4, use range 1024 to encode the following frequencies (30, 40, 1010, 1015) for c=5, use variable Bitmap to encode the following frequencies (980, 991, 992, 993, 994, 1015) for c=6, use Bitmap 0 to encode the following frequencies (20, 40, 66) |
| For k=3 |  |
| Cell Channel Description | for c=1, use range 128 to encode the following 13 frequencies : (980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015) for c=2, use range 256 to encode the following 13 frequencies (20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114, 115) for c=3, use range 512 to encode the following 13 frequencies (980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015) for c=4, use range 1024 to encode the following 8 frequencies (0, 30, 40, 66, 80, 520, 975, 1005, 1010, 1015) for c=5, use variable Bitmap to encode the following frequencies (980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015) for c=6, use Bitmap 0 to encode the following 12 frequencies (20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114) |
| For k = 3 |  |
| Mobile Allocation | indicates the following the frequencies : for c=1 (1005, 1010, 1015) for c=2 (73, 74, 75, 76, 77) for c=3 (980, 981, 982, 983) for c=4 (30, 40, 1010, 1015) for c=5 (990, 991, 992, 993, 994) for c=6 (20, 40, 66) |
| Mode of the First channel | speech full rate for TCH/F except if speech is not supported : arbitrary from those supported for SDCCH/8 |

##### 26.10.2.4.2	E-GSM signalling / RR / Handover / layer 1 failure

26.10.2.4.2.1	Conformance requirements

During a handover from a channel in the E-GSM band to a channel in the P-GSM band, or the contrary, the MS shall correctly return to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell.

References

GSM 04.08 section 3.4.4.

26.10.2.4.2.2	Test purpose

To check  that the MS correctly returns to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell, even if the origin is in the P-GSM band and the target in the E-GSM band.

26.10.2.4.2.3	Method of test

Initial Conditions

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call on a P-GSM channel of cell A. power level = 10

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/FS

Supported mode(s).

Support for state U10 of the Call Control protocol.

Foreseen Final State of the MS

The active state (U10) of a mobile call (on cell A), used power level 10.

Test Procedure

The MS is in the active state (U10) of a call on a P-GSM channel of cell A (used power level 10). The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B (power level 12). With the exception of normal BCCH signalling, the SS does not transmit anything on cell B (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test

1 min

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | HANDOVER COMMAND | to an E-GSM channel see specific message contents |
| 2 | MS -> SS | HANDOVER ACCESS | Several messages are sent, all with correct Handover References. |
| 3 | MS -> SS | HANDOVER FAILURE | Sent on old channel, RR cause value = “Abnormal release, unspecified", "Abnormal release, channel unacceptable", "Abnormal release, timer expired", "Abnormal release, no activity on the radio path" or "Protocol error unspecified". Layer 1 header has the same power level as the layer 1 header in step 1. Shall be sent within 3 seconds from the transmission of HANDOVER COMMAND. |

Specific Message Contents

HANDOVER COMMAND

| Information Element | Value/remark |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - BCCH carrier number | 40 |
| Channel Description |  |
| - Channel type | TCH/F + ACCHs if supported by the MS |
| - Timeslot number | arbitrary but not zero |
| - Training sequence code | chosen arbitrarily |
| - Hopping | RF hopping channel |
| - MAIO | arbitrary |
| - HSN | chosen randomly from the set (1,2...63) |
| Synchronisation Indication IE is not included |  |
| Cell Channel Description IE is not included |  |
| Frequency short list | use range 128 to encode the following frequencies : (1005, 1010, 1015) |
| Mode of the first channel | Full rate speech if supported If not, arbitrary from those supported except signalling. |

#### 26.10.2.5	E-GSM signalling / RR / Frequency Redefinition

This section only applies to E-GSM mobile stations.

Conformance requirements of section 26.6.6 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.6.6 under the described GSM900 conditions.

The purpose of this extra section is to test the frequency redefinition procedure in the case where the target channel uses E-GSM frequency(s) : in the case of frequency hopping several coding formats may be used by the network in the FREQUENCY REDEFINITION message.

26.10.2.5.1	Conformance requirements

1) 	To verify that the MS, after receiving a FREQUENCY REDEFINITION message, correctly starts using the new frequencies, and hopping sequence when some E-GSM frequencies are used.

2) 	The last received Cell Channel Description information element is used to decode the Mobile Allocation IE received on the FREQUENCY REDEFINITION message.

References

GSM 04.08 3.4.5 and 9.1.13.

26.10.2.5.2	Test purpose

1)	To verify that the MS, after receiving a FREQUENCY REDEFINITION message, starts using the new frequencies and hopping sequence when some E-GSM frequencies are used.

2)	To check that the last received Cell Channel Description information element is used to decode the Mobile Allocation IE received in the FREQUENCY REDEFINITION message.

26.10.2.5.3	Method of test

Initial conditions

System Simulator:

1 cell; default parameters.

Mobile Station:

The MS is in "idle, updated" state with TMSI allocated.

Related PICS/PIXIT statement(s)

- Support of a TCH (Y/N)

Foreseen final state of the MS

"Idle, updated", with TMSI allocated.

Test procedure

The test procedure is performed six times.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a TCH/F if supported otherwise a SDCCH/8. Then the SS sends to MS a FREQUENCY REDEFINITION. The MS shall then use the new frequencies/hopping sequence.

Maximum duration of test

3 min

Expected sequence

The sequence is performed for execution counter c=1 to 6

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS->MS | PAGE REQ TYPE 1 |  |
| 2 | MS->SS | CHANNEL REQUEST |  |
| 3 | SS->MS | IMMEDIATE ASSIGN | Channel type = TCH/F if a TCH is supported otherwise, Channel type=SDCCH/8. |
| 4 | MS->SS | PAGING RESPONSE | Sent on the correct channel after establishment of the main signalling link |
| 5 | SS->MS | FREQUENCY REDEFINITION | See specific message contents. |
| 6 | ----- | ------------------ | The SS checks that the MS is transmitting on the correct frequencies. |
| 7 | SS->MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents

FREQUENCY REDEFINITION

| Information Element | Value/remark |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - BCCH carrier number | 40 |
| Channel Description |  |
| - Channel type | TCH/F + ACCHs if supported by the MS or SDCCH/8 if not |
| - Timeslot number | arbitrary but not zero |
| - Training sequence code | chosen arbitrarily |
| - Hopping | RF hopping channel |
| - MAIO | arbitrary |
| - HSN | arbitrarily chosen from the set (1,2...63) |
| Cell Channel Description | for c=1, use range 128 to encode the following 13 frequencies : (980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015) for c=2, use range 256 to encode the following 13 frequencies (20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114, 115) for c=3, use range 512 to encode the following 13 frequencies (980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015) for c=4, use range 1024 to encode the following 8 frequencies (0, 30, 40, 66, 80, 1005, 1010, 1015) for c=5, use variable Bitmap to encode the following frequencies (980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015) for c=6, not present (the mobile station will use the last Cell Channel Description IE received, i.e. the one broadcast in the SYSTEM INFORMATION TYPE 1 message). |
| Mobile Allocation | indicates the following frequencies : for c=1 (1005, 1010, 1015) for c=2 (73, 74, 75, 76, 77) for c=3 (980, 981, 982, 983) for c=4 (30, 40, 1010, 1015) for c=5 (990, 991, 992, 993, 994) for c=6 (30, 50, 70) |
| Starting time | indicates (current frame number + 100 frames) mod 42432 |

### 26.10.3	E-GSM signalling / Structured procedure

Conformance requirements of section 26.9 fully apply to any mobile station (P-GSM, E-GSM or DCS) in the whole supported band of the mobile station.

Besides, as an E-GSM mobile station shall support the P-GSM band, it shall pass successfully every test of section 26.9 under the described GSM900 conditions.

The purpose of these tests is to verify that the MS performs certain elementary procedures of the RR, MM, and CC protocol correctly within a structured procedure. This tested here in an E-GSM context to evaluate the global behaviour of an E-GSM mobile station using its specific frequency capability.

One mobile originated call and one emergency call are performed.

In one of the different cases the different following situations occur : early and late assignments, call release initiated by the network and by the MS.

#### 26.10.3.1	E-GSM signalling / Structured procedure / Mobile originated call

26.10.3.1.1	Conformance requirement

1)	An MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS , shall start to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message with correct establishment cause.

2)	Upon receipt of the ASSIGNMENT COMMAND message, the Mobile Station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the data links). After the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message, specifying cause 'normal event', to the network on the main DCCH.

3,4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

References

Conformance requirement 1:	TS GSM 04.08 section 3.3.1.

Conformance requirement 2:	TS GSM 04.08 section 3.4.3.1, 3.4.3.2

Conformance requirement 3:	TS GSM 04.08 section 3.4.3.

Conformance requirement 4:	TS GSM 04.08 section 5.2.1.1.6.

Conformance requirement 5:	TS GSM 02.07.

Conformance requirement 6:	TS GSM 04.08 sections 5.2.1.1.6 and 5.1.3.

26.10.3.1.2	Test purpose

1)	To verify that the MS in MM state "idle, updated" and in RR idle mode with a TMSI assigned, when made to initiate a call for a selected teleservice that is supported by the MS as declared in a PICS/PIXIT statement, starts to initiate an immediate assignment procedure by sending the CHANNEL REQUEST message.

2)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed authentication and cipher mode setting procedures, after having sent a SETUP message, after having received a CALL PROCEEDING message followed by an ALERTING message and an ASSIGNMENT COMMAND message allocating an appropriate TCH, the MS sends an ASSIGNMENT COMPLETE message.

3)	To verify that subsequently, after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message returns a CONNECT ACKNOWLEDGE message.

4)	To verify that after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message attaches the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

26.10.3.1.3	Method of test

Related PICS/PIXIT statements

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM state “idle updated”.

Test procedure

The following test is performed for one teleservice supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is made to initiate a call. The call is established with late assignment. The release of the call is initiated by the MS.

Maximum Duration of Test

1 minute.

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The "called number" is entered |
| 3 | MS |  |  |
| 4 | MS -> SS | CHANNEL REQUEST | Establishment cause is "originating call and the network does not set the NECI bit to 1". |
| 5 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 6 | MS -> SS | CM SERVICE REQUEST | Message is contained in SABM. |
| 7 | SS -> MS | AUTHENTICATION REQ |  |
| 8 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 9 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 10 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 11 | SS |  | SS starts ciphering. |
| 12 | MS -> SS | SETUP |  |
| 13 | SS -> MS | CALL PROCEEDING |  |
| 14 | SS -> MS | ALERTING |  |
| 15 | MS |  | Depending on the PICS, an alerting indication is given. |
| 16 | SS -> MS | ASSIGNMENT COMMAND |  |
| 17 | MS -> SS | ASSIGNMENT COMPLETE |  |
| 18 | SS -> MS | CONNECT |  |
| 19 | MS -> SS | CONNECT ACKNOWLEDGE |  |
| 20 | MS |  | The appropriate bearer channel is through connected in both directions. |
| 21 | MS |  | If the call is a data call, the TCH shall be through connected in both directions. |
| 22 | MS |  | The MS is made to release the call. |
| 23 | MS -> SS | DISCONNECT |  |
| 24 | SS -> MS | RELEASE |  |
| 25 | MS -> SS | RELEASE COMPLETE |  |
| 26 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents:

None

#### 26.10.3.2	E-GSM signalling / Structured procedures / emergency call

Emergency call establishment can be initiated by an MS whether location updating has been successful or not and whether a SIM is inserted into the MS or not; but only if the MS is equipped for speech.

This section is only applicable to an MS supporting speech.

26.10.3.2.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct IMEI and a non-available CKSN, with CM Service Type "emergency call establishment".

3)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

4),5)	The emergency call shall be correctly established. The assignment procedure shall be correctly performed.

6)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

7)	The call shall be cleared correctly.

Requirement Reference:

For conformance requirement 1 and 2:

TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4.

For conformance requirement 3:

TS GSM 04.08, section 5.2.1.2.

For conformance requirements 4 and 5:

TS GSM 04.08, sections 5.2.1.1 and 3.4.3.

For conformance requirement 6:

TS GSM 04.08, sections 5.2.1.1.6 and 5.1.3.

For conformance requirement 7:

TS GSM 04.08, section 5.4.

26.10.3.2.2	Test purpose

1)	To verify that the MS in the "idle, no IMSI" state (no SIM inserted) when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message specifying the correct CKSN and TMSI, with CM Service Type "emergency call establishment".

3)	To verify that after receipt of a CM SERVICE ACCEPT message from the SS, the MS sends an EMERGENCY SETUP message.

4)	To verify that subsequently, the SS having sent a CALL PROCEEDING message and then an ALERT message and having initiated the assignment procedure, the MS performs correctly that assignment procedure.

5)	To verify subsequent correct performance of a connect procedure.

6)	To verify that subsequently the MS has through connected the TCH in both directions.

7)	To verify that the call is cleared correctly.

This is tested in the following case :

Structured procedures / emergency call / idle, no IMSI / accept case

26.10.3.2.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Classmark.

Initial Conditions

System Simulator:

1 cell, default parameters except :

BCCH ARFCN = 990

Mobile Station:

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Foreseen Final State of the MS

The MS is in MM-state "idle, no IMSI", no SIM inserted.

Test procedure

The MS is made to initiate an emergency call. The call is established without authentication, without ciphering, with early assignment. Having reached the active state, the call is cleared by the SS.

Maximum Duration of Test

1 minute.

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The "called number" 112 is entered |
| 2 | MS -> SS | CHANNEL REQUEST | Establishment cause is "emergency call". |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 4 | MS -> SS | CM SERVICE REQUEST | Message is contained in SABM. The CM service type IE indicates "emergency call establishment". The mobile identity IE specifies the IMEI of the MS. The cipher key sequence number IE indicates "no key is available". The mobile station classmark IE is as specified by the manufacturer in a PICS/PIXIT statement. |
| 5 | SS -> MS | CM SERVICE ACCEPT |  |
| 6 | MS -> SS | EMERGENCY SETUP |  |
| 7 | SS -> MS | CALL PROCEEDING |  |
| 8 | SS -> MS | ASSIGNMENT COMMAND | The rate of the channel is one indicated by the EMERGENCY SETUP message. |
| 9 | MS -> SS | ASSIGNMENT COMPLETE |  |
| 10 | SS -> MS | ALERTING |  |
| 11 | SS -> MS | CONNECT |  |
| 12 | MS -> SS | CONNECT ACKNOWLEDGE |  |
| 13 | MS |  | The TCH is through connected in both directions. |
| 14 | SS -> MS | DISCONNECT |  |
| 15 | MS -> SS | RELEASE |  |
| 16 | SS -> MS | RELEASE COMPLETE |  |
| 17 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents:

None

#### 26.10.3.3	Default contents of messages

same as in 26.9.7 except for the following :

ASSIGNMENT COMMAND

| Information element | Value/remark |
| --- | --- |
| Description of the first channel | describes non-hopping Bm+ACCHs or Lm+ACCHs as appropriate for the test ARFCN = 990 |
| Power Command |  |
| - | Chosen arbitrarily but within the range supported by the MS. |
| Frequency list | Omitted |
| Cell channel description | Omitted |
| Mode of the first channel | appropriate for on bearer capability chosen for the test |
| Description of the second channel | Omitted |
| Mode of the second channel | Omitted |
| Mobile allocation | Omitted |
| Starting time | Omitted |
| Cipher mode setting | Omitted |

IMMEDIATE ASSIGNMENT

| Information element | Value/remark |
| --- | --- |
| Page mode | Normal paging |
| Channel description | describes a valid SDCCH+SACCH in non-hopping mode ARFCN = 40 |
| Request reference |  |
| Random access information | As received from MS |
| N51, N32, N26 | Corresponding to frame number of the CHANNEL REQUEST |
| Timing advance | Arbitrary |
| Mobile allocation | Empty (L=0) |
| Starting time | Omitted |

### 26.10.4	E-GSM signalling / Default message contents

Default SYSTEM INFORMATION:

NOTE :	SYSTEM INFORMATION 7, and SYSTEM INFORMATION 8 messages are not used.

SYSTEM INFORMATION type 1 message

| Information elements | Value/Remark |
| --- | --- |
| Cell Channel Description |  |
| For Cell A |  |
| - Format identifier | Bit Map 0 |
| - Cell Allocation ARFCN | Channel Numbers 20, 30, 50 and 70. |
| For Cell B |  |
| - Format identifier | Bit Map 0 |
| - Cell Allocation ARFCN | Channel Numbers 10, 12, 40, 60, 62, 63 |
| RACH Control parameters | see below |
| SI1 Rest octets | see below |

SYSTEM INFORMATION type 2 message

| Information elements | Value/Remark |
| --- | --- |
| BCCH frequency list |  |
| For cell A |  |
| - Format identifier | bit map 0 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | Channels numbers 10, 20, 80, 90, 100, 110 and 120 |
| - EXT-IND | This IE does not carry the complete BA |
| For cell B |  |
| - Format identifier | bit map 0 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | Channels numbers 10, 36, 40, 114, 118 |
| - EXT-IND | This IE does not carry the complete BA |
| NCC permitted | see below |
| RACH control parameters | see below |

SYSTEM INFORMATION type 2bis message

| Information elements | Value/Remark |
| --- | --- |
| Extended BCCH frequency list |  |
| For cell A |  |
| - Format identifier | range 256 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | 988, 990, 1003 |
| - EXT-IND | This IE does not carry the complete BA |
| For cell B |  |
| - Format identifier | range 128 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | Channels numbers 1005, 1010, 1015 |
| - EXT-IND | This IE does not carry the complete BA |
| RACH control parameters | see below |
| SI 2bis rest octets | see below |

SYSTEM INFORMATION type 3 message

| Information elements | Value/Remark |
| --- | --- |
| Cell identity | see below |
| LAI | see below |
| Control channel description | see below |
| Cell options | see below |
| Cell Selection parameters | see below |
| RACH control parameter | see below |
| SI3 Rest octets | see below |

SYSTEM INFORMATION type 4 message

| Information elements | Value/Remark |
| --- | --- |
| LAI | see below |
| Cell selection parameters | see below |
| RACH control parameters | see below |
| CBCH Channel Description | see below |
| CBCH MA | see below |
| SI4 Rest octets | see below |

SYSTEM INFORMATION type 5 message

| Information elements | Value/Remark |
| --- | --- |
| BCCH frequency list |  |
| For cell A |  |
| - Format identifier | bit map 0 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | Channels numbers 10, 20, 80, 90, 100, 110 and 120 |
| - EXT-IND | This IE does not carry the complete BA |
| For cell B |  |
| - Format identifier | Bit map 0 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | Channels numbers 10, 36, 40, 114, 118 |
| - EXT-IND | This IE does not carry the complete BA |

SYSTEM INFORMATION type 5bis message

| Information elements | Value/Remark |
| --- | --- |
| Extension of BCCH frequency list description |  |
| For cell A |  |
| - Format identifier | Range 256 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | Channels numbers 988, 990, 1003 |
| - EXT-IND | This IE does not carry the complete BA |
| For cell B |  |
| - Format identifier | range 128 |
| - BCCH Allocation Sequence | 0 |
| - BCCH Allocation ARFCN | Channels numbers 1005, 1010, 1015 |
| - EXT-IND | This IE does not carry the complete BA |

SYSTEM INFORMATION type 6 message

| Information elements | Value/Remark |
| --- | --- |
| Cell identity | see below |
| LAI | see below |
| Cell options | see below |
| NCC permitted | see below |

Common contents of information elements in SYSTEM INFORMATION TYPE 1 to 6 messages.

| (CBCH) Channel Description | Not present |
| --- | --- |
| (CBCH) Mobile Allocation | Not present |
| Cell Identity |  |
| - Cell Identity Value | 0001H for cell A, 0002H for cell B |
| Cell Options |  |
| - Power Control Indicator | Power Control Indicator is not set |
| - DTX Indicator | MS shall not use DTX |
| - Radio-Link-Timeout | 8 SACCH blocks |
| Cell Selection Parameters |  |
| - Cell-Reselect-Hysteresis | 12 dB |
| - MX-TXPWR-MAX-CCH | Minimum level |
| - ACS | No addition cell parameters are present in SYSTEM INFORMATION messages 7 and 8. |
| - NECI | New establishment causes not supported |
| - RXLEV-ACCESS-MIN | Minimum level |
| Control Channel Description |  |
| - Attach-Detach allowed | No Attach/Detach |
| - BS-AG-BLKS-RES | 0 blocks reserved for access grant |
| - CCCH-CONF | 1 basic physical channel used for CCCH, combined with SDCCHs |
| - BS-PA-MFRMS | 5 multiframe periods for transmission of paging messages |
| - T3212 Timeout value | Infinite |
| L2 pseudo length |  |
| - System information 1 | 21 |
| - System information 2 | 22 |
| - System information 2bis | 21 |
| - System information 3 | 18 |
| - System information 4 | 12 |
| Location Area Identification |  |
| - Mobile Country Code | 001 decimal |
| - Mobile Network Code | 01 decimal |
| - Location Area Code | 0001H |
| Message Type |  |
| - System information 1 | 00011001 |
| - System information 2 | 00011010 |
| - System information 2bis | 00000010 |
| - System information 3 | 00011011 |
| - System information 4 | 00011100 |
| - System information 5 | 00011101 |
| - System information 5bis | 00000101 |
| - System information 6 | 00011110 |
| NCC Permitted | 0000 0010 |
| RACH Control Parameters |  |
| - Max Retrans | Max 1 retrans |
| - Tx-integer | 5 slots used |
| - Cell Barred for Access | Cell is not barred |
| - Call Reestablishment Allowed | Not allowed |
| - Access Control Class | Access is not barred |
| - Emergency Call allowed | Yes |
| SI 1 rest octets | Not used (all bits are set to spare) |
| SI 2 rest octets | Not used (all bits are set to spare) |
| SI 3 rest octets | Not used (all bits are set to spare) |
| SI 4 rest octets | Not used (all bits are set to spare) |

Default settings for cell A:

| Downlink input level | 63 dBmicroVolt emf |
| --- | --- |
| Uplink output power | minimum supported by the MS's power class |
| Propagation profile | static |
| BCCH/CCCH carrier number | 20 |

Default settings for cell B:

| Downlink input level | 53 dBmicroVolt emf |
| --- | --- |
| Uplink output power | minimum supported by the MS's power class |
| Propagation profile | static |
| BCCH/CCCH carrier number | 10 |

Default message contents for other messages

| For sections 26.10.1 to 26.10.2.5 | same as in 26.6.14 |
| --- | --- |
| For section 26.10.3 | same as in 26.9.7 |