# 34	Short message service (SMS)

Ref : GSM 03.40, 04.11 (point to point)
GSM 03.41, 04.12 (cell broadcast)

## 34.1	General

The purpose of these tests is to verify that the MS can handle GSM functions when submitting or receiving Short Messages (SM) between MS and a short message service centre as described in GSM 03.40.

The procedures are based upon services provided by the Mobility Management (MM) sublayer which is not tested in this case.

The SMS comprises three basic services. The SMS point to point services shall work in an active MS at any time independent of whether or not there is a speech or data call in progress. The SMS cell broadcast service only works when the MS is in idle mode.

Since the timer TC1M currently is not standardised, the value of TC1M shall be declared by the manufacturer (to be used in 34.2.1 and 34.2.2).

The manufacturer shall declare whether SMS messages are stored in the SIM and/or the ME. This shall be referred to as the SMS message store in the following tests.

Unless otherwise stated default message contents from section 26.6.14 applies for GSM900 and default message contents from section 26.6.15 applies for DCS1800.

## 34.2	Short message service point to point

### 34.2.1	SMS mobile terminated

34.2.1.1 	Conformance requirements

An active MS shall be able to receive short message TPDU (SMS-DELIVER) at any time, independently of whether or not there is a speech or data call in progress. A report will always be returned to the SC, confirming that the MS has received the short message.

Reference

GSM 03.40; 3.1

34.2.1.2 	Test purpose

To verify the ability of a MS to receive and decode the SMS where provided for the point to point service.

34.2.1.3 	Method of test

Initial Conditions

System simulator:

1 cell, default parameters.

Mobile Station:

The MS shall be in "Idle, updated" state.

The SMS message storage shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME

Support for call control state U10

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The SS initiates the transmission of a short message using a paging request. Upon response of the MS to the paging the SS assigns an SDCCH, authenticates the MS and activates ciphering. Then the SS establishes SAPI 3 by sending a SABM frame with SAPI 3 on the SDCCH.

When a UA frame (SAPI 3) is received in response, the SS sends a CP-DATA message. The information element of the CP-DATA message will be RP-DATA RPDU (SMS DELIVER TPDU).

b)	The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

c)	The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

d)	Steps a), b) and c) are repeated but the first CP-DATA message from the MS is not acknowledged. The second CP-DATA message from the MS is acknowledged by a CP-ACK within a time TC1M.

e)	Steps a) and b) are repeated. The SS is configured not to send CP-ACK. Then maximum 3 CP-DATA retransmissions may occur. After a duration of TC1M + 5 seconds after the last CP-DATA retransmission the SS then initiates the channel release. The 5 seconds is the appropriate time to wait to verify that the MS does not send more than the maximum allowed (3) CP-DATA retransmissions.

f)	The SMS message store shall be cleared manually by the operator.

g)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH.

When a UA frame (SAPI-3) is received in response, the SS sends a CP-DATA message. The information element of the CP-DATA message will be RP-DATA RPDU (SMS DELIVER TPDU). The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

h)	The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release. The SMS message store shall be cleared manually by the operator.

i)	Steps g) and h) are repeated but the first CP-DATA message from the MS is not acknowledged. The second CP-DATA message from the MS is acknowledged by a CP-ACK within a time TC1M.

j)	Step g) is repeated. The SS is configured not to send CP-ACK. Then maximum 3 CP-DATA retransmissions may occur. After a duration of TC1M + 15 seconds  after the last CP-DATA retransmission the SS initiates the channel release.  The 15 seconds is the appropriate time to wait to verify that the MS does not send more than the maximum allowed (3) CP-DATA retransmissions (during a call in progress).

k)	A data or speech call is established on a TCH with the SS and the state U10 of call control shall be entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH. After the UA response on SAPI-3, the speech call is cleared by the SS with a disconnect message. (The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS).

The SS sends a CP-DATA RPDU (SMS DELIVER TPDU) message. The information element of the CP-DATA message is RP-DATA.

The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

The SMS message store shall be cleared manually by the operator.

l)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH. After the UA response on SAPI-3, the SS waits 15 seconds. During this time the speech call shall be cleared from the MS. (The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS).

The SS sends a CP-DATA message. The information element of the CP-DATA message is RP-DATA RPDU (SMS DELIVER TPDU).

The SS waits a maximum of 25 seconds for the CP-ACK message and then a maximum of 60 seconds for the CP-DATA message containing the RP-ACK RPDU.

The SS sends a CP-ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

The SMS message store shall be cleared manually by the operator.

Maximum Duration of Test

-

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAGING REQUEST |  |
| 2 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 4 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 5 | SS -> MS | AUTHENTICATION REQ |  |
| 6 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 7 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 8 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 9 | SS |  | SS starts ciphering. |
| 10 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 11 | MS -> SS | UA (SAPI=3) | MS shall respond to SABM in step 10 |
| 12 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) |
| 13 | SS |  | Waits max 25 seconds for CP-ACK |
| 14 | MS -> SS | CP-ACK |  |
| 15 | SS |  | Waits max 60 seconds for RP-ACK RPDU |
| 16 | MS -> SS | CP-DATA | Contains RP-ACK RPDU |
| 17 | SS -> MS | CP-ACK | Within TC1M after step 16, no further CP-DATA messages |
| 18 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 19 | MS |  | The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed |
| 20 | SS -> MS | PAGING REQUEST |  |
| 21 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 22 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 23 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 24 | SS -> MS | AUTHENTICATION REQ |  |
| 25 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 26 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 27 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 28 | SS |  | SS starts ciphering. |
| 29 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 30 | MS -> SS | UA (SAPI=3) | The MS shall respond to the SABM |
| 31 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) |
| 32 | SS |  | Waits max 25 seconds for CP-ACK |
| 33 | MS -> SS | CP-ACK |  |
| 34 | SS |  | Waits max 60 seconds for RP-ACK RPDU |
| 35 | MS -> SS | CP-DATA | First CP-DATA from MS, contains RP-ACK RPDU |
| 36 | SS |  | First CP-DATA message not acknowledged by SS |
| 37 | MS -> SS | CP-DATA | Retransmitted CP-DATA from MS within TC1M, after step 35, contains RP-ACK RPDU |
| 38 | SS -> MS | CP-ACK | Second CP-DATA message is acknowledged within TC1M after step 37, no further CP-DATA messages |
| 39 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 40 | MS |  | The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed |
| 41 | SS -> MS | PAGING REQUEST |  |
| 42 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 43 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 44 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 45 | SS -> MS | AUTHENTICATION REQ |  |
| 46 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 47 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 48 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 49 | SS |  | SS starts ciphering. |
| 50 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 51 | MS -> SS | UA (SAPI=3) | The MS shall respond to the SABM |
| 52 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) |
| 53 | SS |  | Waits max 25 seconds for CP-ACK |
| 54 | MS -> SS | CP-ACK |  |
| 55 | SS |  | Waits max 60 seconds for RP-ACK RPDU |
| 56 | MS -> SS | CP-DATA | Contains RP-ACK RPDU |
| 57 | SS |  | First CP-DATA message not acknowledged by SS |
| 58 | MS -> SS | CP-DATA | Retransmitted CP-DATA from MS within twice TC1M after step 56, contains RP-ACK RPDU |
| 59 | SS |  | Retransmitted CP-DATA message not acknowledged by SS |
| 60 | MS |  | Depending upon the maximum number of CP-DATA retransmissions implemented, step 58 and 59 may be repeated. |
| 61 | SS -> MS | CHANNEL RELEASE | The main signalling link is released after a duration of TC1M + 5 seconds after the last CP-DATA retransmission.. |
| 62 | MS |  | The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed |
| 63 | SS |  | A data or speech call is established on a TCH and the state U10 of call control is entered. |
| 64 | SS -> MS | SABM (SAPI=3) | Sent on SACCH associated with the TCH |
| 65 | MS -> SS | UA (SAPI=3) | The MS shall respond to the SABM |
| 66 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) |
| 67 | SS |  | Waits max 25 seconds for CP-ACK |
| 68 | MS -> SS | CP-ACK |  |
| 69 | SS |  | Waits max 60 seconds for RP-ACK RPDU |
| 70 | MS -> SS | CP-DATA | Contains RP-ACK RPDU |
| 71 | SS -> MS | CP-ACK | Within TC1M after step 70, no further CP-DATA messages |
| 72 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 73 | MS |  | The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed |
| 74 | MS |  | Clear the SMS message store |
| 75 | SS |  | A data or speech call is established on a TCH and the state U10 of call control is entered. |
| 76 | SS -> MS | SABM (SAPI=3) | Sent on SACCH associated with the TCH |
| 77 | MS -> SS | UA (SAPI=3) | The MS shall respond to the SABM |
| 78 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) |
| 79 | SS |  | Waits max 25 seconds for CP-ACK |
| 80 | MS -> SS | CP-ACK |  |
| 81 | SS |  | Waits max 60 seconds for RP-ACK RPDU |
| 82 | MS -> SS | CP-DATA | First CP-DATA from MS, contains RP-ACK RPDU |
| 83 | SS |  | First CP-DATA message not acknowledged by SS |
| 84 | MS -> SS | CP-DATA | Retransmitted CP-DATA message within twice TC1M after step 82, contains RP-ACK RPDU |
| 85 | SS -> MS | CP-ACK | Second CP-DATA message is acknowledged within TC1M after step 84, no further CP-DATA messages |
| 86 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 87 | MS |  | The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed |
| 88 | MS |  | Clear the SMS message store |
| 89 | SS |  | A data or speech call is established on a TCH and the state U10 of call control is entered. |
| 90 | SS -> MS | SABM (SAPI=3) | Sent on SACCH associated with the TCH |
| 91 | MS -> SS | UA (SAPI=3) | The MS shall respond to the SABM |
| 92 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) |
| 93 | SS |  | Waits max 25 seconds for CP-ACK |
| 94 | MS -> SS | CP-ACK |  |
| 95 | SS |  | Waits max 60 seconds for RP-ACK RPDU |
| 96 | MS -> SS | CP-DATA | First CP-DATA from MS, contains RP-ACK RPDU |
| 97 | SS |  | First CP-DATA message not acknowledged by SS |
| 98 | MS -> SS | CP-DATA | Transmitted CP-DATA message within TC1M after step 96, contains RP-ACK RPDU |
| 99 | SS |  | Retransmitted CP-DATA message not acknowledged by SS |
| 100 | MS |  | Depending on the maximum number of CP-DATA retransmissions implemented, step 98-99 may be repeated. The maximum number of retransmissions may however not exceed three. |
| 101 | SS -> MS | CHANNEL RELEASE | The main signalling link is released after a duration of TC1M + 15 seconds after the last CP-DATA retransmission. |
| 102 | MS |  | The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed |
| 103 | MS |  | Clear the SMS message store |
| 104 | SS |  | A data or speech call is established on a TCH and the state U10 of call control is entered. |
| 105 | SS -> MS | SABM (SAPI=3) | Sent on SACCH associated with the TCH |
| 106 | MS -> SS | UA (SAPI=3) | The MS shall respond to the SABM |
| 107 | SS -> MS | DISCONNECT | The speech call is cleared by the SS. The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS. |
| 108 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) |
| 109 | SS |  | Waits max 25 seconds for CP-ACK |
| 110 | MS -> SS | CP-ACK |  |
| 111 | SS |  | Waits max 60 seconds for RP-ACK RPDU |
| 112 | MS -> SS | CP-DATA | Contains RP-ACK RPDU |
| 113 | SS -> MS | CP-ACK | Within TC1M after step 112, no further CP-DATA |
| 114 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 115 | MS |  | The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed |
| 116 | MS |  | Clear the SMS message store |
| 117 | SS |  | A data or speech call is established on a TCH and the state U10 of call control is entered. |
| 118 | SS -> MS | SABM (SAPI=3) | Sent on SACCH associated with the TCH |
| 119 | MS -> SS | UA (SAPI=3) | The MS shall respond to the SABM |
| 120 | SS |  | Waits 15 seconds |
| 121 | MS |  | During the SS wait time in step 120 the speech call shall be cleared from the MS. The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS. |
| 122 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) |
| 123 | SS |  | Waits max 25 seconds for CP-ACK |
| 124 | MS -> SS | CP-ACK |  |
| 125 | SS |  | Waits max 60 seconds for RP-ACK RPDU |
| 126 | MS -> SS | CP-DATA | Contains RP-ACK RPDU |
| 127 | SS -> MS | CP-ACK | Within TC1M after step 126, no further CP-DATA |
| 128 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 129 | MS |  | The MS shall indicate that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed |
| 130 | MS |  | Clear the SMS message store |

NOTE:	Time values for SS wait time are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS DELIVER TPDU

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS DELIVER ‘00’B |
| TP-MMS | more messages are waiting in SC ‘0’B |
| TP-RP | no reply path ‘0’B |
| TP-UDHI | TP-UD contains only the SM ‘0’B |
| TP-SRI | no status report returned ‘0’B |
| TP-OA | an international number coded E.164 |
| TP-PID | default ‘00000000’B |
| TP-DCS | default alphabet ‘00000000’B |
| TP-SCTS | any legal value (cf. GSM 03.40) |
| TP-UDL | 160 |
| TP-UD (140 octets) | text of message (160 characters) |

NOTE:	The 160 characters shall include at least one occurrence of each character in the default alphabet (see GSM 03.40 annex 2).

### 34.2.2	SMS mobile originated

34.2.2.1	Conformance requirements

An active MS shall be able to submit short message TPDU (SMS-SUBMIT) at any time, independently of whether or not there is a speech or data call in progress.

Reference

GSM 03.40; 3.1

34.2.2.2	Test purpose

To verify that the MS is able to correctly send a short message where the SMS is provided for the point to point service. The test also verifies that the MS is capable of simultaneously receive a network originated SM whilst sending a mobile originated SM.

34.2.2.3	Method of test

Initial Conditions

System simulator:

1 cell, default parameters.

Mobile Station:

The MS shall be in "Idle, updated" state.

The SMS message storage shall be empty.

Related PICS/PIXIT Statements

Support for Short message MO/PP

Description of the basic procedures to display a mobile originated short message

Support for state U10 of call control

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME.

Maximum length (characters) of a mobile originated short message.

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The MS shall be set up to send a SM to the SS. The SS responds to the channel request message by allocating an SDCCH. The SS answers correctly to the SABM on SAPI 0 and then performs the authentication and ciphering procedures.

b)	The SS responds with a UA frame SAPI-3 to the MS.

c)	The SS responds to the CP-DATA containing RP-DATA RPDU (SMS SUBMIT TPDU) from the MS with a CP-ACK message within TC1M followed by a CP-DATA message containing the correct RP-ACK RPDU. The SS waits a maximum of 25 seconds for the CP-ACK message.

d)	The SS sends a channel release message to the MS.

e)	Steps a) and b) are repeated. The SS is configured not to send the CP-ACK message. Then maximum 3 CP-DATA retransmissions may occur. After a duration of TC1M + 5 seconds after the last CP-DATA retransmission the SS initiates channel release. The 5 seconds is the appropriate time to wait to verify that the MS does not send more than the maximum CP-DATA retransmissions.

f)	Steps a) and b) are repeated. On receipt of the CP-DATA from the MS the SS sends a CP-ERROR message within TC1M containing a "Network Failure" cause. Then the SS initiates channel release.

g)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The MS is setup to send an SM to the SS. After the reception of the CM SERVICE REQUEST, the SS sends a CM SERVICE ACCEPT message. The SS responds with a UA frame SAPI-3 to the SABM with SAPI-3 received from the MS.

h)	The SS responds to the CP-DATA containing RP-DATA RPDU (SMS SUBMIT TPDU) from the MS with a CP-ACK message within TC1M followed by a CP-DATA message containing the correct RP-ACK RPDU. The SS waits a maximum of 25 seconds for the CP-ACK message. Then the SS sends a channel release message to the MS.

i)	Step g) is repeated. The SS is configured not to send the CP-ACK message. Then maximum 3 CP-DATA retransmissions may occur. After a duration of TC1M + 15 seconds after the last CP-DATA retransmission the SS initiates channel release. The 15 seconds is the appropriate time to wait to verify that the MS does not send more than the maximum CP-DATA retransmissions (during a call in progress).

j)	The SS is configured to receive a mobile originated SM. Steps a) and b) are repeated and, using the end of the CP-DATA message from the MS as a trigger, the SS sends a SM to the MS. In this case a new transaction identifier shall be used in the CP messages of SMS mobile terminated.

k)	The MS is set up to send an SM to the SS. On receipt of the CM SERVICE REQUEST the SS sends a CM SERVICE REJECT message with the reject cause set to "Service Option not supported" or "Service Option temporarily out of order". After 5 seconds the SS initiates channel release.

Maximum Duration of Test

-

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS -> SS | CHANNEL REQUEST | Establishment cause is “Other procedures which can be completed with an SDDCH” NECI = 0 |
| 2 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 3 | MS -> SS | CM SERVICE REQ | Message is contained in SABM on SAPI 0. CM service type set to "Short message transfer" |
| 4 | SS -> MS | AUTHENTICATION REQ |  |
| 5 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 6 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 7 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 8 | SS |  | SS starts ciphering. |
| 9 | MS -> SS | SABM (SAPI=3) | MS establishes SAPI 3 |
| 10 | SS -> MS | UA (SAPI=3) |  |
| 11 | MS -> SS | CP-DATA | Contains RP-DATA RPDU (SMS SUBMIT TPDU) |
| 12 | SS -> MS | CP-ACK | Sent within TC1M after step 11 |
| 13 | SS -> MS | CP-DATA | Contains RP-ACK RPDU |
| 14 | SS |  | Waits max 25 seconds for CP-ACK |
| 15 | MS -> SS | CP-ACK |  |
| 16 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 17 | MS -> SS | DISC (SAPI=0) | MS shall respond to channel release with a layer 2 DISC frame with SAPI 0 |
| 18 | MS -> SS | CHANNEL REQUEST | Establishment cause is “Other procedures which can be completed with an SDDCH” (NECI=0) |
| 19 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 20 | MS -> SS | CM SERVICE REQ | Message is contained in SABM on SAPI 0. |
| 21 | SS -> MS | AUTHENTICATION REQ |  |
| 22 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 23 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 24 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 25 | SS |  | SS starts ciphering. |
| 26 | MS -> SS | SABM (SAPI=3) | MS establishes SAPI 3 |
| 27 | SS -> MS | UA (SAPI=3) |  |
| 28 | MS -> SS | CP-DATA | Contains RP-DATA RPDU (SMS SUBMIT TPDU) |
| 29 | SS |  | SS configured not to send CP-ACK |
| 30 | MS -> SS | CP-DATA | Retransmitted CP-DATA message within TC1M after step 28 |
| 31 | MS |  | Depending on the maximum number of CP-DATA retransmissions implemented, step 30 may be repeated. The maximum number of retransmissions may however not exceed three. |
| 32 | SS -> MS | CHANNEL RELEASE | The main signalling link is released after a duration of TC1M + 5 seconds after the last CP-DATA retransmission. |
| 32a | MS -> SS | DISC (SAPI = 0) | MS shall respond to channel release with a layer 2 DISC frame with SAPI 0. |
| 33 | MS -> SS | CHANNEL REQUEST | Establishment cause is “Other procedures which can be completed with an SDDCH” (NECI=0) |
| 34 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 35 | MS -> SS | CM SERVICE REQ | Message is contained in SABM. CM service type set to "short message transfer" |
| 36 | SS -> MS | AUTHENTICATION REQ |  |
| 37 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 38 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 39 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 40 | SS |  | SS starts ciphering. |
| 41 | MS -> SS | SABM (SAPI=3) | MS establishes SAPI 3 |
| 42 | SS -> MS | UA (SAPI=3) |  |
| 43 | MS -> SS | CP-DATA | Contains RP-DATA RPDU (SMS SUBMIT TPDU) |
| 44 | SS -> MS | CP-ERROR | Sent within TC1M containing "Network Failure" cause. |
| 45 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 45a | MS -> SS | DISC (SAPI = 0) | MS shall respond to channel release with a layer 2 DISC frame with SAPI 0. |
| 46 | SS |  | A data or speech call is established on a TCH and the state U10 of call control is entered. |
| 47 | MS |  | The MS is set up to send an SM |
| 48 | MS -> SS | CM SERVICE REQ | Sent in a layer 2 frame on the FACCH. CM service type set to "short message transfer" |
| 49 | SS -> MS | CM SERVICE ACCEPT |  |
| 50 | MS -> SS | SABM (SAPI=3) | Sent on SACCH associated with the TCH |
| 51 | SS -> MS | UA (SAPI=3) |  |
| 52 | MS -> SS | CP-DATA | Contains RP-DATA RPDU (SMS SUBMIT TPDU) |
| 53 | SS -> MS | CP-ACK | Sent within TC1M after step 52 |
| 54 | SS -> MS | CP-DATA | Contains RP-ACK RPDU |
| 55 | SS |  | Waits max 25 seconds for CP-ACK |
| 56 | MS -> SS | CP-ACK |  |
| 57 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 58 | MS -> SS | DISC (SAPI =0) | The MS shall respond to channel release with a layer 2 DISC frame with SAPI 0. |
| 59 | SS |  | A data or speech call is established on a TCH and the state U10 of call control is entered. |
| 60 | MS -> SS | CM SERVICE REQ | Sent in a layer 2 frame on the FACCH. CM service type set to "short message transfer" |
| 61 | SS -> MS | CM SERVICE ACCEPT |  |
| 62 | MS -> SS | SABM (SAPI=3) | Sent on SACCH associated with the TCH |
| 63 | SS -> MS | UA (SAPI=3) |  |
| 64 | MS -> SS | CP-DATA | Contains RP-DATA RPDU (SMS SUBMIT TPDU) |
| 65 | SS |  | SS configured not to send CP-ACK |
| 66 | MS -> SS | CP-DATA | Transmitted CP-DATA message TC1M after step 64 |
| 67 | MS |  | Depending on the maximum number of CP-DATA retransmissions implemented, step 66 may be repeated. The maximum number of retransmissions may however not exceed three. |
| 68 | SS -> MS | CHANNEL RELEASE | The main signalling link is released after a duration of TC1m + 15 seconds after the last CP-DATA retransmission. |
| 69 | MS -> SS | DISC (SAPI =0) | The MS shall respond to channel release |
| 70 | MS -> SS | CHANNEL REQUEST | Establishment cause is “Other procedures which can be completed with an SDDCH” (NECI=0) |
| 71 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 72 | MS -> SS | CM SERVICE REQ | Message is contained in SABM. CM service type set to "short message transfer" |
| 73 | SS -> MS | AUTHENTICATION REQ |  |
| 74 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 75 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 76 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 77 | SS |  | SS starts ciphering. |
| 78 | MS -> SS | SABM (SAPI=3) | MS establishes SAPI 3 |
| 79 | SS -> MS | UA (SAPI=3) |  |
| 80 | MS -> SS | CP-DATA | Contains RP-DATA RPDU (SMS SUBMIT TPDU) |
| 81 | SS |  | The SS sends an SM to the MS triggered by the end of the CP-DATA message from the MS |
| 82 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) |
| 83 | MS |  | The MS shall correctly receive the SM and indicate that a message has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed. In the MO case the MS shall send the CP-ACK message with transaction identifier assigned to this transfer. In the MT case the MS shall send a CP-ACK message and a CP-DATA message containing the RP-ACK RPDU. The transaction identifier shall be the same as chosen by the SS for the MT transfer. |
| 84 | MS -> SS | CHANNEL REQUEST | Establishment cause is “Other procedures which can be completed with an SDDCH” (NECI=0) |
| 85 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 86 | MS -> SS | CM SERVICE REQ | Message is contained in SABM. CM service type set to "short message transfer" |
| 87 | SS -> MS | CM SERVICE REJ | Reject cause set to "Service Option not supported" or "Service Option temporarily out of order" |
| 88 | MS |  | The MS shall not establish SAPI-3 |
| 89 | SS -> MS | CHANNEL RELEASE | Sent 5 seconds after CM SERVICE REJ |

NOTE:	Time values for SS wait times are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS SUBMIT TPDU

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS SUBMIT ‘01’B |
| TP-VPF | not checked |
| TP-RP | no reply path ‘0’B |
| TP-UDHI | not checked |
| TP-SRR | not checked |
| TP-MR | not checked |
| TP-RD | not checked |
| TP-DA | not checked (an E164 number) |
| TP-PID | default ‘00000000’B |
| TP-DCS | default alphabet ‘00000000’B |
| TP-VP | not checked |
| TP-UDL | as applicable |
| TP-UD (140 octets max) | maximum number of characters (text of message) as defined by the manufacturer (see PICS/PIXIT) |

### 34.2.3	Test of memory full condition and memory available notification:

The Memory Available Notification provides a means for the MS to notify the network that it has memory available to receive one or more short messages. The SMS status field in the SIM contains status information on the "memory available" notification flag.

34.2.3.1	Conformance requirement

1.	When a mobile terminated message is Class 2, the MS shall ensure that the message has been transferred to the SMS data field in the SIM before sending an acknowledgement to the SC. The MS shall return a protocol error message if the short message cannot be stored in the SIM and there is other short message storage available in the MS. If all the short message storage in the MS is already in use, the MS shall return "memory capability exceeded".

2.	When the MS rejects a short message due to lack of available memory capability the need to transfer notification shall be stored in the SIM.

3.	If the memory capability becomes available because memory is cleared, the value of the memory capability exceeded notification flag in the SIM is read. If the flag is set, the MS notifies the network that memory capability is now available. After a positive acknowledgement from the network, the ME unsets the memory capability exceeded notification flag in the SIM.

References

GSM 03.40, 9.2.3.10, GSM 03.38, 4.

GSM 03.40, 10.3 (operation 14)

GSM 03.40, 10.3 (operation 14)

34.2.3.2	Test purpose

1.	To verify that the MS sends the correct acknowledgement when its memory in the SIM becomes full.

2.	To verify that the MS sends the correct acknowledgement when its memory in the ME and the SIM becomes full, and sets the "memory exceeded" notification flag in the SIM.

3.	To verify that the MS performs the "memory available" procedure when its message store becomes available for receiving short messages, and only at this moment.

34.2.3.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The SMS message storage shall be empty.

The MS shall be connected to the SIM simulator. The following shall be present in the SIM 	simulator:

-	EFSMS with at least one record

-	EFSMSstatus, with SMS "Memory Cap. Exceed" notification flag set to "memory 	available"

-	Service no. 4 (SMS) in EFSST set to allocated and activated

For storing of Class 1 Short Messages the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

Whether SMS messages are stored in the SIM and/or the ME

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	step a) of 34.2.5.3 (test of Class 2 Short Messages) is repeated until the MS sends a negative acknowledgement (RP-ERROR). The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been set on the SIM.

b)	a Class 1 Short Message is sent to the MS.

c)	step b) is repeated until the MS sends a negative acknowledgement (RP-ERROR). The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been set on the SIM.

d)	a Short Message is sent to the MS with the DCS field of the SMS-DELIVER TPDU set to 0.

e)	the SS prompts the operator to read a short message and to remove it from the message store of the MS.

f)	the SS waits for a CHANNEL REQUEST from the MS, and sends an IMMEDIATE ASSIGNMENT allocating an SDCCH.

g)	the SS answers correctly to the SABM on SAPI 0.

h)	the SS answers correctly to the SABM on SAPI 3.

i)	the SS answers to the RP-SMMA from the MS with a CP-DATA containing a RP-ACK RPDU.

j)	after the MS has acknowledged the CP-DATA with a CP-ACK, the SS releases the channel with a CHANNEL RELEASE message. The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been unset on the SIM.

k)	step e) is repeated.

Maximum Duration of Test

-

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAGING REQUEST |  |
| 2 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 4 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 5 | SS -> MS | AUTHENTICATION REQ |  |
| 6 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 7 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 8 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 9 | SS |  | SS starts ciphering. |
| 10 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 11 | MS -> SS | UA (SAPI=3) |  |
| 12 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 2 Short Message |
| 13 | SS |  | Waits max 25 seconds for CP-ACK |
| 14 | MS -> SS | CP-ACK |  |
| 15 | SS |  | Waits max 60 seconds for RP-ACK RPDU |
| 16 | MS -> SS | CP-DATA | Contains RP-ACK RPDU |
| 17 | SS -> MS | CP-ACK | Within TC1M after step 16 |
| 18 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. Step 1-18 is repeated until MS sends a negative acknowledgement (RP-ERROR) in step 16. The RP-ERROR RPDU cause field shall be "Protocol error, unspecified" if there is message capability in the ME, or "Memory capability exceeded" if there is no message capability in the ME. If the total memory store of the MS is full, the ME shall set the "memory capability exceeded" notification flag on the SIM. |
| 19 | SS -> MS | PAGING REQUEST |  |
| 20 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 21 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 22 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 23 | SS -> MS | AUTHENTICATION REQ |  |
| 24 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 25 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 26 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 27 | SS |  | SS starts ciphering. |
| 28 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 29 | MS -> SS | UA (SAPI=3) |  |
| 30 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 1 Short Message |
| 31 | SS |  | Waits max 25 seconds for CP-ACK |
| 32 | MS -> SS | CP-ACK |  |
| 33 | SS |  | Waits max 60 seconds for RP-ACK RPDU |
| 34 | MS -> SS | CP-DATA | Shall contain RP-ACK RPDU if there is memory capability in the ME. If not it shall contain RP-ERROR RPDU which cause field shall be "memory capability exceeded". If the total memory store of the MS now becomes full at this step, the ME shall set the "memory cap. exceed" notification flag on the SIM. |
| 35 | SS -> MS | CP-ACK | Within TC1M after step 34 |
| 36 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. Step 19-36 is repeated until the MS sends an RP-ERROR. The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been set on the SIM. |
| 37 | SS -> MS | PAGING REQUEST |  |
| 38 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 39 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 40 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 41 | SS -> MS | AUTHENTICATION REQ |  |
| 42 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 43 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 44 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 45 | SS |  | SS starts ciphering. |
| 46 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 47 | MS -> SS | UA (SAPI=3) |  |
| 48 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) with TP-DCS set to 0 |
| 49 | SS |  | Waits max 25 seconds for CP-ACK |
| 50 | MS -> SS | CP-ACK |  |
| 51 | SS |  | Waits max 60 seconds for RP-ACK RPDU |
| 52 | MS -> SS | CP-DATA | Shall contain RP-ERROR RPDU with error cause "memory capability exceeded". |
| 53 | SS -> MS | CP-ACK | Within TC1M after step 52 |
| 54 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 55 | SS |  | Prompts the operator to read a short message and to remove it from the message store of the MS. |
| 57 | MS -> SS | CHANNEL REQUEST | Establishment cause "Other services which can be completed with an SDCCH" (NECI=0). |
| 58 | SS -> MS | IMMEDIATE ASSIGNMENT | SS allocates an SDCCH |
| 59 | MS -> SS | CM SERVICE REQUEST | Message is contained in SABM. CM service type information element is set to "Short message transfer". |
| 60 | SS -> MS | CM SERVICE ACCEPT (UA) | SAPI 0 |
| 61 | MS -> SS | SABM (SAPI=3) | MS shall establish SAPI 3 |
| 62 | SS -> MS | UA (SAPI=3) |  |
| 63 | MS -> SS | CP-DATA | Contains RP-SMMA RPDU |
| 64 | SS -> MS | CP-ACK |  |
| 65 | SS -> MS | CP-DATA | Contains RP-ACK RPDU |
| 66 | MS -> SS | CP-ACK | Acknowledge of CP-DATA containing the RP-ACK RPDU. The ME shall unset the "memory capability exceeded" notification flag on the SIM. |
| 67 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. The SIM simulator shall indicate if the "memory capability exceeded" notification flag has been unset on the SIM. |
| 68 | SS |  | Prompts the operator to read a short message and to remove it from the message store of the MS. |
| 69 | MS |  | Shall not attempt to send a RP-SMMA RPDU. This is verified by checking that the MS does not send a CHANNEL REQUEST message with the establishment cause "Other services which can be completed with an SDCCH" |

NOTE:	Time values for SS wait time are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Specific Message Contents:

SMS-DELIVER TPDU in step 12

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS DELIVER ‘00’B |
| TP-MMS | more messages are waiting in SC ‘0’B |
| TP-RP | no reply path ‘0’B |
| TP-UDHI | TP-UD contains only the SM ‘0’B |
| TP-SRI | no status report returned ‘0’B |
| TP-OA | an international number coded E.164 |
| TP-PID | default ‘00000000’B |
| TP-DCS | default alphabet, class 2 ‘11110010’B |
| TP-SCTS | any legal value (cf. GSM 03.40) |
| TP-UDL | 160 |
| TP-UD (140 octets) | text of message (160 characters) |

SMS-DELIVER TPDU in step 30

same as in step 12 except:

SMS-DELIVER TPDU in step 48

same as in step 12 except

| TP-DCS | default alphabet ‘00000000’B |
| --- | --- |

### 34.2.4	Test of the status report capabilities and of SMS-COMMAND:

This test applies to MSs which support the status report capabilities.

34.2.4.1	Conformance requirement

The SMS offers the SC the capabilities of informing the MS of the status of a previously sent mobile originated short message. This is achieved by the SC returning a status report TPDU (SMS-STATUS-REPORT) to the originating MS.

SMS-COMMAND enables an MS to invoke an operation at the SC.

The MS shall increment TP-MR by 1 for each SMS-SUBMIT or SMS-COMMAND being submitted.

References

GSM 03.40; 3.2.9

GSM 03.40; 9.2.3.6

34.2.4.2	Test purpose

1)	To verify that the MS is able to accept a SMS-STATUS-REPORT TPDU.

2)	To verify that the MS is able to use the SMS-COMMAND functionality correctly and sends an SMS-COMMAND TPDU with the correct TP-Message-Reference.

34.2.4.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

Related PICS/PIXIT Statements

Support of SMS MO/PP and MT/PP

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The MS is made to send a Mobile Originated short message as in steps a) to d) of test 34.2.2 	(SMS Mobile originated).

b)	The SS establishes a data link on SAPI-3 with the MS, then sends a CP-DATA message containing a RP-DATA RPDU itself containing an SMS-STATUS-REPORT TPDU.

c)	The SS sends a CHANNEL RELEASE message.

d)	The MS is made to send an SMS-COMMAND message enquiring about the previously submitted short message.

e)	The SS responds to the MS so as to enable it to establish a data link on SAPI-3 on an SDCCH.

f)	The SS acknowledges the CP-DATA message from the MS with a CP-ACK followed by a CP-DATA message containing an RP-ACK RPDU

g)	After receiving the CP-ACK from the MS, the SS releases the channel by using a CHANNEL RELEASE message.

h)	The MS is made to send an SMS-COMMAND message requiring to delete the previously submitted short message.

i)	steps e) to g) are repeated.

Maximum Duration of Test

-

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Other procedures which can be completed with an SDCCH " (NECI=0) |
| 2 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 3 | MS -> SS | CM SERVICE REQ | Message is contained in SABM. |
| 4 | SS -> MS | AUTHENTICATION REQ |  |
| 5 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 6 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 7 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 8 | SS |  | SS starts ciphering. |
| 9 | MS -> SS | SABM (SAPI=3) | MS establishes SAPI 3 |
| 10 | SS -> MS | UA (SAPI=3) |  |
| 11 | MS -> SS | CP-DATA | Contains RP-DATA RPDU (SMS SUBMIT TPDU) |
| 12 | SS -> MS | CP-ACK | Sent within TC1M after step 11 |
| 13 | SS -> MS | CP-DATA | Contains RP-ACK RPDU |
| 14 | SS |  | Waits max 25 seconds for CP-ACK |
| 15 | MS -> SS | CP-ACK |  |
| 16 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 17 | SS -> MS | PAGING REQUEST |  |
| 18 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 19 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 20 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 22 | SS -> MS | AUTHENTICATION REQ |  |
| 23 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 24 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 25 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 26 | SS |  | SS starts ciphering. |
| 27 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 28 | MS -> SS | UA (SAPI=3) |  |
| 29 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS-STATUS-REPORT TPDU) |
| 30 | MS -> SS | CP-ACK |  |
| 31 | MS -> SS | CP-DATA | Contains RP-ACK RPDU |
| 32 | SS ->MS | CP-ACK |  |
| 33 | SS -> MS | CHANNEL RELEASE |  |
| 34 | MS |  | The MS is made to send an SMS-COMMAND message enquiring about the previously submitted SM |
| 35 | MS -> SS | CHANNEL REQUEST | Establishment cause "Other services which can be completed with an SDCCH". |
| 36 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 37 | MS -> SS | CM SERVICE REQ | Message is contained in SABM. |
| 38 | SS -> MS | AUTHENTICATION REQ |  |
| 39 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 40 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 41 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 42 | SS |  | SS starts ciphering. |
| 43 | MS -> SS | SABM (SAPI=3) | MS establishes SAPI 3 |
| 44 | SS -> MS | UA (SAPI=3) |  |
| 45 | MS -> SS | CP-DATA | Contains RP-DATA RPDU (SMS-COMMAND TPDU) which shall contain the correct TP-MR |
| 46 | SS -> MS | CP-ACK |  |
| 47 | SS -> MS | CP-DATA | Contains RP-ACK RPDU |
| 48 | MS -> SS | CP-ACK |  |
| 49 | SS -> MS | CHANNEL RELEASE |  |
| 50 | MS | The MS is made to send an SMS-COMMAND | message requiring to delete the previously submitted SM. |
| 51 | MS -> SS | CHANNEL REQUEST | Establishment cause "Other services which can be completed with an SDCCH". |
| 52 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 53 | MS -> SS | CM SERVICE REQ | Message is contained in SABM. |
| 54 | SS -> MS | AUTHENTICATION REQ |  |
| 55 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 56 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 57 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 58 | SS |  | SS starts ciphering. |
| 59 | MS -> SS | SABM (SAPI=3) | MS establishes SAPI 3 |
| 60 | SS -> MS | UA (SAPI=3) |  |
| 61 | MS -> SS | CP-DATA | Contains RP-DATA RPDU (SMS-COMMAND TPDU) which shall contain the correct TP-MR |
| 62 | SS -> MS | CP-ACK |  |
| 63 | SS -> MS | CP-DATA | Contains RP-ACK RPDU |
| 64 | MS -> SS | CP-ACK |  |
| 65 | SS -> MS | CHANNEL RELEASE |  |

Specific Message Contents:

SMS SUBMIT TPDU

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS SUBMIT ‘01’B |
| TP-VPF | not checked |
| TP-RP | no reply path ‘0’B |
| TP-UDHI | not checked |
| TP-SRR | status report is requested ‘1’B |
| TP-MR | not checked |
| TP-RD | not checked |
| TP-DA | not checked (an E164 number) |
| TP-PID | default ‘00000000’B |
| TP-DCS | default alphabet ‘00000000’B |
| TP-VP | not checked |
| TP-UDL | as applicable |
| TP-UD (140 octets max) | maximum number of characters |

SMS-STATUS-REPORT TPDU (SS to MS in step 29):

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS-STATUS-REPORT ‘10’B |
| TP-MR | same as previous SMS-SUBMIT |
| TP-MMS | no more messages ‘1’B |
| TP-SRQ | result of SMS-SUBMIT ‘0’B |
| TP-RA | same as the Destination address of the SMS-SUBMIT |
| TP-SCTS | any legal value (cf. GSM 03.40) |
| TP-DT | any legal value (cf. GSM 03.40) |
| TP-ST | SM received ‘00000000’B |

first SMS-COMMAND TPDU (MS to SS in step 44)

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS-COMMAND ‘10’B |
| TP-MR | TP-MR in previous SMS-SUBMIT plus "1" |
| TP-SRR | status report requested (03.40 9.2.3.19) ‘1’B |
| TP-PID | default ‘00000000’B |
| TP-CT | Enquiry relating to previously submitted short message ‘00000000’B |
| TP-MN | not checked (TP-MR in previous SMS-SUBMIT) |
| TP-DA | not checked (an E164 number) |
| TP-CDL | not checked |
| TP-CD | not checked |

second SMS-COMMAND TPDU (MS to SS in step 60)

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS-COMMAND ‘10’B |
| TP-MR | TP-MR in previous SMS-COMMAND plus "1" |
| TP-SRR | status report not requested ‘0’B |
| TP-PID | default ‘00000000’B |
| TP-CT | Delete previously submitted short message ’00000010’B |
| TP-MN | not checked (TP-MR in previous SMS-SUBMIT) |
| TP-DA | not checked (an E164 number) |
| TP-CDL | not checked |
| TP-CD | not checked |

### 34.2.5	Test of message class 0 to 3

The tests under this section only apply to a MS capable of displaying short messages (see PICS/PIXIT).

#### 34.2.5.1	Short message class 0

34.2.5.1.1	Conformance requirement

When a mobile terminated message is class 0 and the MS has the capability of displaying short messages, the MS shall display the message immediately and send an acknowledgement to the SC when the message has successfully reached the MS irrespective of whether there is memory available in the SIM or ME. The message shall not be automatically stored in the SIM or ME.

References

GSM 03.38, 4.

34.2.5.1.2	Test purpose

To verify that the MS will accept and display but not store a class 0 message, and that it will accept and display a class 0 message if its message store is full.

NOTE:	failure of this test in a mobile could cause it to reject a class 0 message when its SMS memory becomes full. This could lead to unwanted repetitions between the MS and the service centre.

34.2.5.1.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME.

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	The SS sends a class 0 message by using the method described in step a) of section 34.2.1 but with the TPDU described in this section.

b)	The MS message store shall be filled (for example by using the method of 34.2.3 test of the memory available notification) with the same SMS-DELIVER TPDU except that TP-DCS is set to class 1.

c)	The SS sends a class 0 message as in step a).

Maximum Duration of Test

-

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAGING REQUEST |  |
| 2 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 4 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 5 | SS -> MS | AUTHENTICATION REQ |  |
| 6 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 7 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 8 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 9 | SS |  | SS starts ciphering. |
| 10 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 11 | MS -> SS | UA (SAPI=3) |  |
| 12 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 0 Short Message |
| 13 | MS -> SS | CP-ACK |  |
| 14 | MS -> SS | CP-DATA | Contains RP-ACK RPDU. |
| 15 | SS -> MS | CP-ACK |  |
| 16 | SS -> MS | CHANNEL RELEASE |  |
| 17 | MS |  | The content of the short message shall be displayed by the ME. The MS shall not store the the message. This can be checked by verifying that it is impossible to retrieve any short messages from the MS message store. |
| 18 | SS |  | The MS message store shall be filled (for example by using the method of 34.2.3) with Class 1 SMS-DELIVER TPDU. |
| 19 | SS -> MS | PAGING REQUEST |  |
| 20 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 21 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 22 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 23 | SS -> MS | AUTHENTICATION REQ |  |
| 24 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 25 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 26 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 27 | SS |  | SS starts ciphering. |
| 28 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 29 | MS -> SS | UA (SAPI=3) |  |
| 30 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER |
|  |  | TPDU), | Class 0 Short Message |
| 31 | MS -> SS | CP-ACK |  |
| 32 | MS -> SS | CP-DATA | Contains RP-ACK RPDU. |
| 33 | SS -> MS | CP-ACK |  |
| 34 | SS -> MS | CHANNEL RELEASE |  |
| 35 | MS |  | The content of the short message shall be displayed by the ME. |

Specific Message Contents:

SMS-DELIVER TPDU (containing a class 0 message) (SS to MS):

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS-DELIVER ‘00’B |
| TP-MMS | more messages are waiting in SC ‘0’B |
| TP-RP | no reply path ‘0’B |
| TP-UDHI | TP-UD contains only the SM ‘0’B |
| TP-SRI | no status report returned 0 |
| TP-OA | an international number coded E.164 |
| TP-PID | default ‘00000000’B |
| TP-DCS | default alphabet, class 0 “1111 0000’B |
| TP-SCTS | any legal value (cf. GSM 03.40) |
| TP-UDL | 160 |
| TP-UD (140 octets) | text of message (160 characters) |

SMS-DELIVER TPDU (containing a class 1 message to fill the MS message store) (SS to MS):

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS-DELIVER ‘00’B |
| TP-MMS | more messages are waiting in SC ‘0’B |
| TP-RP | no reply path ‘0’B |
| TP-UDHI | TP-UD contains only the SM ‘0’B |
| TP-SRI | no status report returned 0 |
| TP-OA | an international number coded E.164 |
| TP-PID | default ‘00000000’B |
| TP-DCS | default alphabet, class 1 ‘1111 0001’B |
| TP-SCTS | any legal value (cf. GSM 03.40) |
| TP-UDL | 160 |
| TP-UD (140 octets) | text of message (160 characters) |

#### 34.2.5.2	Test of class 1 short messages

This test shall apply to MSs which support

-	storing of received Class 1 Short Messages and

-	displaying of stored Short Messages.

34.2.5.2.1	Conformance requirement

When a mobile terminated message is class 1, the MS shall send an acknowledgement to the SC when the message has successfully reached the MS and can be stored, either in the ME or in the SIM.

References

GSM 03.38, 4.

34.2.5.2.2	Test purpose

This procedure verifies that the MS acts correctly on receiving a class 1 message, i.e. that it stores the message in the ME or SIM and sends an acknowledgement (at RP and CP-Layer).

34.2.5.2.3	Method of test

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

For storing of class 1 Short Messages, the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message of class 1 to the MS as specified in section 34.2.1, step a).

b)	the Short Message is recalled (e.g. by means of the MMI).

Maximum Duration of Test

-

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAGING REQUEST |  |
| 2 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 4 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 5 | SS -> MS | AUTHENTICATION REQ |  |
| 6 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 7 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 8 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 9 | SS |  | SS starts ciphering. |
| 10 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 11 | MS -> SS | UA (SAPI=3) |  |
| 12 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 1 Short Message |
| 13 | MS -> SS | CP-ACK |  |
| 14 | MS -> SS | CP-DATA | Contains RP-ACK RPDU. |
| 15 | SS -> MS | CP-ACK |  |
| 16 | SS -> MS | CHANNEL RELEASE |  |
| 17 | MS |  | The short message shall be recalled and displayed at the MS. |

Specific Message Contents:

SMS-DELIVER TPDU (containing a class 1 message) (SS to MS):

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS-DELIVER ‘00’B |
| TP-MMS | more messages are waiting in SC ‘0’B |
| TP-RP | no reply path ‘0’B |
| TP-UDHI | TP-UD contains only the SM ‘0’B |
| TP-SRI | no status report returned 0 |
| TP-OA | an international number coded E.164 |
| TP-PID | default ‘00000000’B |
| TP-DCS | default alphabet, class 1 ‘1111 0001’B |
| TP-SCTS | any legal value (cf. GSM 03.40) |
| TP-UDL | 160 |
| TP-UD (140 octets) | text of message (160 characters) |

#### 34.2.5.3	Test of class 2 short messages

34.2.5.3.1	Definition and applicability

Class 2 Short Messages are defined as SIM specific, and the MS shall ensure that a message of this class is stored on the SIM.

This test shall apply to MSs which support

-	storing of received Class 2 Short Messages in the SIM and

-	displaying of Short Messages stored in the SIM.

34.2.5.3.2	Conformance requirement

When a mobile terminated message is Class 2, the MS shall ensure that the message has been correctly  transferred to the SMS data field in the SIM before sending an acknowledgement to the SC. The MS shall return a "protocol error, unspecified" error message if the short message cannot be stored in the SIM and there is other short message storage available at the MS.

Reference(s)

GSM 03.40, 9.2.3.10; GSM 03.38, 4. GSM 11.11, 10.3.3

34.2.5.3.3	Test purpose

This procedure verifies that the MS acts correctly on receiving a class 2 message, i.e. that it stores the message correctly in the SIM, and if this is not possible, returns a protocol error message to the network.

34.2.5.3.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The ME message store shall be empty.

The ME shall be connected to the SIM simulator. The following shall be present in the SIM 	simulator:

-	EFSMS with at least two free records and one full record

-	EFSMSstatus, with SMS "Memory Cap. Exceed" notification flag set to "memory 	available"

-	Service no. 4 (SMS) in EFSST set to allocated and activated

For storing of Class 1 Short Messages the MS shall be set up to store Short Messages in the ME memory (by way of MMI, as described in PICS/PIXIT statement).

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Whether SMS messages are stored in the SIM and/or the ME

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message of class 2 to the MS as specified in section 34.2.1, step b).

b)	following an attempt by the ME to store the short message in a free record of EFSMS in the SIM, the SIM simulator returns the status response "OK" ('90 00').

c)	step a) is repeated.

d)	following an attempt by the ME to store the short message in a free record of EFSMS in the SIM, the SIM simulator returns the status response "memory problem" ('92 40').

e)	the SIM simulator indicates if an attempt was made in steps a) and c) to store the messages and if the messages are stored according to the requirement.

Maximum Duration of Test

-

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAGING REQUEST |  |
| 2 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 4 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 5 | SS -> MS | AUTHENTICATION REQ |  |
| 6 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 7 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 8 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 9 | SS |  | SS starts ciphering. |
| 10 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 11 | MS -> SS | UA (SAPI=3) |  |
| 12 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 2 Short Message |
| 13 | MS -> SS | CP-ACK |  |
| 14 | ME |  | The ME shall correctly store the short message in a free record of EFSMS in the SIM, i.e. - the ME shall use a free record - the first byte of the record shall indicate "message received by MS from network" - the TS-Service-Centre-Address shall be correctly stored - the TPDU shall be identical to that sent by the SS - bytes following the TPDU shall be set to 'FF' |
| 15 | SIM |  | The SIM simulator returns the status response "OK" ('90 00'). The SIM simulator shall indicate if an attempt was made by the ME to store the short message in the SIM. |
| 16 | MS -> SS | CP-DATA | Contains RP-ACK RPDU. |
| 17 | SS -> MS | CHANNEL RELEASE |  |
| 18 | SS -> MS | PAGING REQUEST |  |
| 19 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 20 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 21 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 22 | SS -> MS | AUTHENTICATION REQ |  |
| 23 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 24 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 25 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 26 | SS |  | SS starts ciphering. |
| 27 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 28 | MS -> SS | UA (SAPI=3) |  |
| 29 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU), Class 2 Short Message |
| 30 | MS -> SS | CP-ACK |  |
| 31 | ME |  | The ME shall attempt to store the short message in a free record of EFSMS in the SIM. |
| 32 | SIM |  | The SIM simulator returns the status response "memory problem" ('92 40'). The SIM simulator shall indicate if an attempt was made by the ME to store the short message in the SIM. |
| 33 | MS -> SS | CP-DATA | Contains RP-ERROR RPDU with error cause "protocol error, unspecified". |
| 34 | SS -> MS | CHANNEL RELEASE |  |

Specific Message Contents:

SMS-DELIVER TPDU (containing a class 2 message) (SS to MS):

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS-DELIVER ‘00’B |
| TP-MMS | more messages are waiting in SC ‘0’B |
| TP-RP | no reply path ‘0’B |
| TP-UDHI | TP-UD contains only the SM ‘0’B |
| TP-SRI | no status report returned 0 |
| TP-OA | an international number coded E.164 |
| TP-PID | default ‘00000000’B |
| TP-DCS | default alphabet, class 2 ‘1111 0010’B |
| TP-SCTS | any legal value (cf. GSM 03.40) |
| TP-UDL | 160 |
| TP-UD (140 octets) | text of message (160 characters) |

#### 34.2.5.4	Test of class 3 short messages

For further study

### 34.2.6	Test of short message type 0

For further study

### 34.2.7	Test of the replace mechanism for SM type 1-7

34.2.7.1	Definition and applicability

This test shall apply to MSs which support

-	Replace Short Messages and

-	display of received Short Messages.

34.2.7.2	Conformance requirement

On receipt of a short message, the MS shall check to see if the associated Protocol Identifier contains a Replace Short Message Type code. If such a code is present, then the MS will check the associated SC address (RP-OA) and originating address (TP-OA) and replace any existing stored message having the same Protocol Identifier code, SC address and originating address with the new short message.

Reference(s)

GSM 03.40; 9.2.3.9

34.2.7.3	Test purpose

This procedure verifies the correct implementation of the replace mechanism for Replace Short Messages.

34.2.7.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Description of the basic procedures to display a mobile terminated short message

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	two different numbers n and m are drawn randomly between 1 and 7.  Two different addresses for TP-Originating-Address (TPOA1 and TPOA2) are drawn.  Two different addresses for RP-Originating-Address (RPOA1 and RPOA2) are drawn.

b)	the SS delivers a short message to the MS as specified in section 34.2.1 step a).  In the SMS-DELIVER TPDU, the TP-Protocol-Identifier parameter is "Replace Short Message Type n", the TP-Originating-Address is TPOA1, and the RP-Originating-Address is RPOA1.

c)	step b) is repeated but with a different TP-Originating-Address (TPOA2), and different contents of TP-User-Data in the SMS-DELIVER TPDU.  The other parameters are the same as in step b).

d)	step c) is repeated but with RPOA2 in the RP-Originated-Address, and contents of TP-User-Data different from the former two messages.  The other parameters are the same as in step c).

e)	step d) is repeated but with the TP-Protocol-Identifier equal to "Replace Short Message Type m", and contents of TP-User-Data different from the former three messages.  The other parameters are the same as in step d).

f)	step e) is repeated but the contents of TP-User-Data are different from that used in step e).

g)	the SS prompts the operator to display the Short Messages stored in the MS.

Maximum Duration of Test

-

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAGING REQUEST |  |
| 2 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 4 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 5 | SS -> MS | AUTHENTICATION REQ |  |
| 6 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 7 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 8 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 9 | SS |  | SS starts ciphering. |
| 10 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 11 | MS -> SS | UA (SAPI=3) |  |
| 12 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-PID is "Replace Short Message Type n", TP-OA is TPOA1 and RP-OA is RPOA1 |
| 13 | MS -> SS | CP-ACK |  |
| 14 | MS -> SS | CP-DATA | Contains RP-ACK RPDU. |
| 15 | SS -> MS | CHANNEL RELEASE |  |
| 16 | SS -> MS | PAGING REQUEST |  |
| 17 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 18 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 19 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 20 | SS -> MS | AUTHENTICATION REQ |  |
| 21 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 22 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 23 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 24 | SS |  | SS starts ciphering. |
| 25 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 26 | MS -> SS | UA (SAPI=3) |  |
| 27 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-PID is "Replace Short Message Type n", TP-OA is TPOA2 and RP-OA is RPOA1, TP-UD different from step 12 |
| 28 | MS -> SS | CP-ACK |  |
| 29 | MS -> SS | CP-DATA | Contains RP-ACK RPDU. |
| 30 | SS -> MS | CHANNEL RELEASE |  |
| 31 | SS -> MS | PAGING REQUEST |  |
| 32 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 33 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 34 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 35 | SS -> MS | AUTHENTICATION REQ |  |
| 36 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 37 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 38 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 39 | SS |  | SS starts ciphering. |
| 40 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 41 | MS -> SS | UA (SAPI=3) |  |
| 42 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-PID is "Replace Short Message Type n", TP-OA is TPOA2 and RP-OA is RPOA2, TP-UD different from step 12 and 27 |
| 43 | MS -> SS | CP-ACK |  |
| 44 | MS -> SS | CP-DATA | Contains RP-ACK RPDU. |
| 45 | SS -> MS | CHANNEL RELEASE |  |
| 46 | SS -> MS | PAGING REQUEST |  |
| 47 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 48 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 49 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 50 | SS -> MS | AUTHENTICATION REQ |  |
| 51 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 52 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 53 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 54 | SS |  | SS starts ciphering. |
| 55 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 56 | MS -> SS | UA (SAPI=3) |  |
| 57 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-PID is "Replace Short Message Type m", TP-OA is TPOA2 and RP-OA is RPOA2, TP-UD different from step 12, 27 and 42 |
| 58 | MS -> SS | CP-ACK |  |
| 59 | MS -> SS | CP-DATA | Contains RP-ACK RPDU. |
| 60 | SS -> MS | CHANNEL RELEASE |  |
| 61 | SS -> MS | PAGING REQUEST |  |
| 62 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 63 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 64 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 65 | SS -> MS | AUTHENTICATION REQ |  |
| 66 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 67 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 68 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 69 | SS |  | SS starts ciphering. |
| 70 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 71 | MS -> SS | UA (SAPI=3) |  |
| 72 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-PID is "Replace Short Message Type m", TP-OA is TPOA2 and RP-OA is RPOA2, TP-UD different from step 57 |
| 73 | MS -> SS | CP-ACK |  |
| 74 | MS -> SS | CP-DATA | Contains RP-ACK RPDU. |
| 75 | SS -> MS | CHANNEL RELEASE |  |
| 76 | SS |  | Prompts the operator to display the Short Messages stored in the MS. Only the Short Messages delivered in step 12, 27, 42 and 72 shall be retrievable and displayed |

Specific Message Contents:

SMS-DELIVER TPDU

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS-DELIVER ‘00’B |
| TP-MMS | no more messages are waiting in SC ‘1’B |
| TP-RP | no Reply Path ‘0’B |
| TP-UDHI | TP-UD contains only the SM ‘0’B |
| TP-SRI | no Status Report returned ‘0’B |
| TP-OA | an international number coded E.164 (see test method description) |
| TP-PID | binary 01000xxx, xxx represents n resp. m (see test method description) |
| TP-DCS | default alphabet ‘00000000’B |
| TP-SCTS | the time when the message was submitted according to GSM 03.40 |
| TP-UDL | 160 |
| TP-UD (140 octets) | text of message (160 characters) (see test method description) |

### 34.2.8	Test of the reply path scheme

34.2.8.1	Definition and applicability

This test applies to MSs which support

-	reply procedures (the class of MSs for which this is mandatory is described in GSM 03.40, Annex 4)

-	displaying of received Short Messages and

-	submitting Short Messages.

Steps b) and d) are only executed for MSs which support storing of Short messages.

34.2.8.2	Conformance requirement

When a replying MS receives an original mobile terminated short message it has:

originating SME = TP-Originating Address in the SMS-DELIVER TPDU

original SC = RP-Originating Address in the RP-MT-DATA

When submitting the reply mobile originated short message, the replying MS should use parameters as follows:

TP-Destination Address in SMS-SUBMIT TPDU = originating SME

RP-Destination Address in RP-MO-DATA = original SC

Reference(s)

GSM 03.40 Annex 4, sect. 5, 6.

34.2.8.3	Test purpose

This procedure verifies that the MS is able to send a Reply Short Message back to the correct originating SME even if in the meantime it receives another Short Message.

34.2.8.4	Test method

Initial conditions

System Simulator:

1 cell, default parameters

Mobile Station:

The MS shall be in the idle updated state.

The MS message store shall be empty.

Related PICS/PIXIT Statements

Support for Short message MT/PP

Support for Short message MO/PP

Description of the basic procedures to display a mobile terminated short message

Description of the basic procedures to send a mobile originated short message

The value of timer TC1M

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	the SS delivers a Short Message as specified in section 34.2.1, step b) with TP-Reply-Path set to 1.

b)	step a) is repeated but with

-	different TP-Originating-Address for the originating SME

-	different RP-Originating-Address for the original SC and

-	different message contents TP-User-Data.

c)	one of the two Short Messages is displayed (e.g. by means of the MMI) and the Reply Short Message is submitted (e.g. by means of the MMI).

d)	step c) is repeated for the other Short Message.

Maximum Duration of Test

-

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAGING REQUEST |  |
| 2 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 4 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 5 | SS -> MS | AUTHENTICATION REQ |  |
| 6 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 7 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 8 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 9 | SS |  | SS starts ciphering. |
| 10 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 11 | MS -> SS | UA (SAPI=3) |  |
| 12 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-RP set to 1 |
| 13 | MS -> SS | CP-ACK | Sent within TC1M after step 12 |
| 14 | MS -> SS | CP-DATA | Contains RP-ACK RPDU. |
| 14A | SS -> MS | CP-ACK |  |
| 15 | SS -> MS | CHANNEL RELEASE |  |
| 16 | SS -> MS | PAGING REQUEST |  |
| 17 | MS -> SS | CHANNEL REQUEST | Establishment cause is "Answer to paging" |
| 18 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 19 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 20 | SS -> MS | AUTHENTICATION REQ |  |
| 21 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 22 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 23 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 24 | SS |  | SS starts ciphering. |
| 25 | SS -> MS | SABM (SAPI=3) | SS establishes SAPI 3 |
| 26 | MS -> SS | UA (SAPI=3) |  |
| 27 | SS -> MS | CP-DATA | Contains RP-DATA RPDU (SMS DELIVER TPDU) TP-OA, RP-OA and TP-UD different from step 12 |
| 28 | MS -> SS | CP-ACK | Sent within TC1M after step 12 |
| 29 | MS -> SS | CP-DATA | Contains RP-ACK RPDU. |
| 29A | SS -> MS | CP-ACK |  |
| 30 | SS -> MS | CHANNEL RELEASE |  |
| 31 | MS |  | One of the two Short Messages is displayed and the Reply Short Message is submitted. |
| 32 | MS -> SS | CHANNEL REQUEST |  |
| 33 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 34 | MS -> SS | CM SERVICE REQ | Message is contained in SABM. |
| 35 | SS -> MS | AUTHENTICATION REQ |  |
| 36 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 37 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 38 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 39 | SS |  | SS starts ciphering. |
| 40 | MS -> SS | SABM (SAPI=3) | MS establishes SAPI 3 |
| 41 | SS -> MS | UA (SAPI=3) |  |
| 42 | MS -> SS | CP-DATA | Contains RP-DATA RPDU (SMS SUBMIT TPDU) RP-DA = RP-OA corresponding to the message displayed TP-DA = TP-OA corresponding to the message displayed |
| 43 | SS -> MS | CP-ACK | Sent within TC1M after step 42 |
| 44 | SS -> MS | CP-DATA | Contains RP-ACK RPDU |
| 45 | SS |  | Waits max 25 seconds for CP-ACK |
| 46 | MS -> SS | CP-ACK |  |
| 47 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |
| 48 | MS |  | The other Short Message is displayed and the Reply Short Message is submitted. |
| 49 | MS -> SS | CHANNEL REQUEST |  |
| 50 | SS -> MS | IMMEDIATE ASSIGNMENT | SS assigns an SDCCH |
| 51 | MS -> SS | CM SERVICE REQ | Message is contained in SABM. |
| 52 | SS -> MS | AUTHENTICATION REQ |  |
| 53 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 54 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 55 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 56 | SS |  | SS starts ciphering. |
| 57 | MS -> SS | SABM (SAPI=3) | MS establishes SAPI 3 |
| 58 | SS -> MS | UA (SAPI=3) |  |
| 59 | MS -> SS | CP-DATA | Contains RP-DATA RPDU (SMS SUBMIT TPDU) RP-DA = RP-OA corresponding to the Message displayed TP-DA = TP-OA corresponding to the message displayed |
| 60 | SS -> MS | CP-ACK | Sent within TC1M after step 59 |
| 61 | SS -> MS | CP-DATA | Contains RP-ACK RPDU |
| 62 | SS |  | Waits max 25 seconds for CP-ACK |
| 63 | MS -> SS | CP-ACK |  |
| 64 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents:

SMS-DELIVER TPDU

| Information element | Comment Value |
| --- | --- |
| TP-MTI | SMS-DELIVER ‘00’B |
| TP-MMS | no more messages are waiting in SC ‘1’B |
| TP-RP | Reply Path exists ‘1’B |
| TP-UDHI | TP-UD contains only the SM ‘0’B |
| TP-SRI | no Status Report returned ‘0’B |
| TP-OA | an international number coded E.164 (see test method description) |
| TP-PID | default ‘00000000’B |
| TP-DCS | default alphabet ‘00000000’B |
| TP-SCTS | the time when the message was submitted according to GSM 03.40 |
| TP-UDL | 160 |
| TP-UD (140 octets) | text of message (160 characters) (see test method description) |

## 34.3	Short message service cell broadcast

This test applies to all MSs.

34.3.1	Conformance requirements

If the MS supports SMS-CB, it is responsible for recombination of the four blocks received via the radio path into a single block which constitutes the cell broadcast short message.

In idle mode, the MS listens to the BCCH and to the paging sub-channel for the paging group it belongs to. The MS is required to receive and analyse the paging messages and immediate assignment messages sent on the paging subchannel corresponding to its paging subgroup.

Reference

GSM 03.41; 8

GSM 04.08; 3.2.1, 3.3.2.1

34.3.2	Test purpose

This test verifies that an MS supporting SMS-CB is able to receive SMS-CB messages.

This test verifies that an MS is able to respond to a paging requested during the transmission of a cell broadcast short message.

34.3.3	Test method

Initial conditions

System Simulator:

1 cell, default parameters

The SS provides a BCCH/CCCH to support the MS in idle mode.

Periodic location updating is disabled.

Mobile Station:

The MS shall be in the idle updated state.

Related PICS/PIXIT Statements

Support for short message transmission cell broadcast

Description of the basic procedures to display a cell broadcasted short message

Foreseen Final State of MS

Idle, updated

Test Procedure

a)	Three Cell Broadcast (CB) messages are sent by the SS on the CBCH with serial numbers 0,1,1.

b)	Step a) is repeated, but the SS pages the MS during the transmission of the second CB message.

The MS shall respond to the page.

Maximum Duration of Test

-

Expected Sequence

Since the SMS-CB messages are sent continuously, a table is not applicable in this test.

Specific Message Contents:

Cell broadcast test message content

| Information element | Comment Value |
| --- | --- |
| Serial Number |  |
| - Geographical scope | ‘00’B |
| - Message code | see test procedure ‘0000000000’B or ‘0000000001’B |
| - Update number | as applicable |
| Message identifier | ‘0’B |
| Data Coding Scheme | Default alphabet, English ‘00000000’B |
| Page parameter | ‘0001 0001’B |
| Contents of message | 93 user characters using 93 different characters of default 7 bit coded alphabet |

SYSTEM INFORMATION TYPE 4

As default except:

| Information element | Value/remark |
| --- | --- |
| CBCH Channel Description |  |
| - Channel type and TDMA offset | SDDCH/4 + SACCH/C4 or CBCH (SDDCH/4) |
| - Timeslot number | time slot zero |
| - Training sequence code | 5 (same as BCC) |
| - Hopping channel | Single RF channel |
| -Channel selector | Channel number 20 (for GSM900 MS) Channel number 590 (for DCS1800 MS) |
| CBCH Mobile Allocation | Empty |

## 34.4		Default message contents:

CP-DATA (including RP-DATA  SS->MS)

| Transaction Identifier |  |
| --- | --- |
| TI value | any value from the set {0, ...., 6} |
| TI flag | 0 |
| Message type | 00000001 |
| Cp-User data |  |
| length |  |
| RP-DATA | max 255 octets |
| RP-Message Type | 001 (RP-DATA SS->MS) |
| RP-Message Reference | see 04.11 8.2.3 |
| RP-Originator Address | see 04.11 8.2.5.1 |
| RP-Destination Address | length indicator set to 0 |
| RP-User Data |  |
| length |  |
| TP-DATA | max 238 octets |

CP-DATA (including RP-DATA  MS->SS)

| Protocol Discriminator | SMS messages (‘1001’B) |
| --- | --- |
| Transaction Identifier |  |
| TI value | any value from the set {0, ...., 6} |
| TI flag | 0 |
| Message type | 00000001 |
| Cp-User data |  |
| length |  |
| RP-DATA | max 255 octets |
| RP-Message Type | 000 (RP-DATA MS->SS) |
| RP-Message Reference | see 04.11 8.2.3 |
| RP-Originator Address | length indicator set to 0 |
| RP-Destination Address | see 04.11 8.2.5.2 |
| RP-User Data |  |
| length |  |
| TP-DATA | max 238 octets |

CP-DATA (including RP-ACK  MS->SS)

| Protocol Discriminator | SMS messages (‘1001’B) |
| --- | --- |
| Transaction Identifier |  |
| TI value |  |
| TI fllag |  |
| Message type | 00000001 |
| Cp-User data |  |
| length |  |
| RP-ACK |  |
| RP-Message Type | 010 (RP-ACK MS->SS) |
| RP-Message Reference | see 04.11 8.2.3 |

CP-DATA (including RP-ACK  SS->MS)

| Protocol Discriminator | SMS messages (‘1001’B) |
| --- | --- |
| Transaction Identifier |  |
| TI value |  |
| TI flag |  |
| Message type | 00000001 |
| Cp-User data |  |
| length |  |
| RP-ACK |  |
| RP-Message Type | 011 (RP-ACK SS->MS) |
| RP-Message Reference | see 04.11 8.2.3 |

CP-DATA (including RP-ERROR  MS->SS)

| Protocol Discriminator | SMS messages (‘1001’B) |
| --- | --- |
| Transaction Identifier |  |
| TI value |  |
| TI flag |  |
| Message type | 00000001 |
| Cp-User data |  |
| length |  |
| RP-ERROR |  |
| RP-Message Type | 100 (RP-ERROR MS->SS) |
| RP-Message Reference | see 04.11 8.2.3 |

CP-DATA (including RP-ERROR  SS->MS)

| Protocol Discriminator | SMS messages (‘1001’B) |
| --- | --- |
| Transaction Identifier |  |
| TI value |  |
| TI flag |  |
| Message type | 00000001 |
| Cp-User data |  |
| length |  |
| RP-ERROR |  |
| RP-Message Type | 101 (RP-ERROR SS->MS) |
| RP-Message Reference | see 04.11 8.2.3 |

CP-DATA (including RP-SMMA  MS->SS)

| Protocol Discriminator | SMS messages (‘1001’B) |
| --- | --- |
| Transaction Identifier |  |
| TI value |  |
| TI flag |  |
| Message type | 00000001 |
| Cp-User data |  |
| length |  |
| RP-SMMA |  |
| RP-Message Type | 110 (RP-SMMA MS->SS) |
| RP-Message Reference | see 04.11 8.2.3 |

CP-ACK

| Protocol Discriminator | SMS messages (‘1001’B) |
| --- | --- |
| Transaction Identifier |  |
| TI value |  |
| TI flag |  |
| Message type | 00000100 |

CP-ERROR

| Protocol Discriminator | SMS messages (‘1001’B) |
| --- | --- |
| Transaction Identifier |  |
| TI value |  |
| TI flag |  |
| Message type | 00010000 |
| Cp-Cause |  |
| Cause value | see 04.11 8.1.4.2 |