II.3	TRANSMITTER

Ref GSM 05.05-DCS

II.3.1	Phase Error and Frequency Error

The GSM radio subsystem uses a GMSK modulation system with an equivalent pre modulation gaussian filter having a bandwidth defined by B * T = 0.3, where B is the filter bandwidth and T is the modulation symbol time.

In order to measure the accuracy of the modulation process a sampled measurement of the transmitted phase trajectory is obtained. This is compared with the theoretically expected phase trajectory. The regression line of the difference between the expected trajectory and the measured trajectory is an indication of the frequency error, whilst the departure of the phase differences from this trajectory is a measure of the phase error.

II.3.1.1	Method of Measurement

a)  The MS is connected to the SS. This connection will be direct for a MS having an antenna connector or via an antenna coupling device for a MS with an integral antenna and not having means of connecting an external antenna.

b)  The SS shall originate a call to the MS and the MS shall be made to answer the call. The SS shall command the MS to hopping mode. The hopping bandwidth shall be according to GC1 of Annex 1. The SS shall activate ciphering mode.

Note: 	Ciphering mode is active during the test to give a pseudo-random bitstream to the modulator

c)  The SS shall command the MS to complete the traffic channel loop back (channel decoder output to channel encoder input).  The  SS shall generate Standard Test Signal C1.

d)  For each transmitted burst, the receiving section of the SS shall capture the signal as a series of phase samples over the period of the burst.  These samples shall be evenly distributed over the duration of the burst with a minimum sampling rate of 2/T, where T is the modulation symbol period.  The received phase trajectory is then represented by this array of at least 294 samples.

e)  The SS must then calculate, from the known bit pattern and the formal definition of the modulator contained in GSM 05.04, the expected phase trajectory.

f)  From d) and e) the phase trajectory error is calculated, and a linear regression line plotted through this phase trajectory error.  The slope of this regression line is the frequency error of the mobile transmitter relative to the simulator reference.  The difference between the regression line and the individual sample points is the phase error of that point.

f.1)  The sampled array of at least 294 phase measurements is represented by the vector φm = φm(0)...φm(n), where the number of samples in the array n+1 >= 294.

f.2)  The calculated array, at the corresponding sampling instants, is represented by the vector φc = φc(0)...φc(n).

f.3)  The error array is represented by the vector:

φe = {φm(0) - φc(0)}.........{φm(n) - φc(n)} = φe(0)...φe(n).

f.4)  The corresponding sample numbers form a vector t = t(0)...t(n).

f.5)  Plot a scattergram with the abscissa represented by the linear values of the vector t and the ordinate by the corresponding value of φe.

f.6)  This scattergram can now be considered as a graph of the equation  φe = k * t, and by regression theory:

j=n

Σ  t(j) * φe(j)

j=0

k =  ─────────────────

j=n

Σ  t(j)²

j=0

f.7)  The frequency error is given by k/(360 * τ), where τ is the sampling interval in seconds and all phase samples are measured in degrees.

f.8)  The individual phase errors from the regression line are given by

φe(j) - k*t(j).

f.9)  The RMS value φE of the phase errors is given by:

┌	┐1/2

│ j=n	│

│  Σ {φe(j) - k*t(j)}2	│

φE(RMS) =	│ j=0	│

│ ────────────────	│

│        n+1	│

└	┘

g)  Steps d) to f) shall be repeated for 20 bursts, not necessarily contiguous.

h)  The SS shall instruct the MS to its maximum power level, all other conditions remaining constant. Steps d) to g) shall be repeated.

i)  The SS shall instruct the MS to power level 15 10 for class 1 MS or 13 for class 2 MS, all other conditions remaining constant. Steps d) to g) shall be repeated.

j)  If the MS has an antenna connector it shall be hard mounted on a vibration table and vibrated at the following frequency/amplitudes:

Frequency	ASD (Acceleration Spectral Density)

10 Hz -  20 Hz	0.005g2/Hz

20 Hz - 150 Hz	0.005g2/Hz at 20 Hz, thereafter -3dB/Octave

This equates to a total RMS vibration in the range 10Hz to 150 Hz of 0.5g.

During the vibration steps a) to i) shall be repeated.

Note:	It is considered to be impracticable to carry out a vibration test for a Mobile Station within its antenna coupling device. Mobile Stations that only have an integral antenna and are not supplied with any accessory that provides an alternative RF connection, are not subject to the vibration test for practical reasons.

k)  The MS shall be repositioned on the vibration table in the two orthogonal planes to the plane used in step j). For each of the orthogonal planes step j) shall be repeated.

l)  The MS is placed in a climatic test chamber and steps d) to i) are repeated for the following combinations of extreme test voltages and ambient temperatures (see Annex 1, TC2.2 and TC3):

Temp       Hi  Hi  Lo  Lo

Voltage    Lo  Hi  Lo  Hi

Note:	The series of samples taken to determine the phase trajectory could also be used, with different post-processing, to determine the transmitter burst characteristics of II.3.3. Although described independently, it is at the discretion of the test house whether to combine the tests of II.3.1 and II.3.3, giving both answers from single sets of captured data.

II.3.1.2	Limits

II.3.1.2.1	Frequency Error

The frequency error, derived in step f.7), for all measured bursts shall be less than 10E-7.

II.3.1.2.2	Phase Error

For every burst, the RMS phase error from  step f.9)  shall  be  less  than 5 degrees.  An individual phase error  from  step f.8)  shall  not  exceed 20 degrees.

II.3.2	Frequency Error under Multipath & Interference Conditions

Ref.	05.10 clause 6

The Mobile Station is required to maintain frequency synchronization with the received signal under conditions of Doppler shift, multipath reception and interference.

By using the techniques of section II.3.1 this section checks the correct functioning of this feature under various conditions.

N.B.:	Rec. GSM 05.10-DCS requires that frequency synchronization should be maintained for input signals 3 dB below reference sensitivity. Due to the Radio Link Failure counter this test condition can not be established. Hence all tests in this section are conducted at reference sensitivity level.

II.3.2.1	Method of Measurement

a)  Set up call on a traffic channel in the range ARFCN 60690-65710. Set the SS fading function to RA250130 with a MS input level of 202dBµVemf(...) on the serving cell BCCH. The SS shall capture the first transmission burst from the MS during call establishment.

The SS shall also set up six adjacent cell BCCH's having signal levels in the range 1419dBµVemf(  ) to 60dBµVemf(  ). The exact level and ARFCN of each signal is not critical, but some simple rules need to be observed.

- No adjacent cell BCCH shall be set on ARFCN close to the serving cell BCCH or TCH (say five channels separated) if it is set to maximum level.

- Of the six adjacent cell BCCH's, twoone should be located near the each band edges.

The amplitude and ARFCN of all six could with benefit be varied continuously throughout the test, but observing the simple rules given here.

b)  The SS shall set the serving cell BCCH and TCH to 1113 dBµVemf(  ) for handheld MS's and 9 dBµVemf(  ) for all other MS's.

Wait 30 seconds for the MS to stabilize to these conditions.

c)  The SS shall capture a subsequent burst from the traffic channel in the manner described in II.3.1.1. Throughout the test the SS shall monitor the status message from the MS to verify that the adjacent cell BCCH's are being correctly measured by the MS.

d)  Due to the very low signal level at the MS receiver input the MS receiver is liable to error.

The "looped back" bits are therefore also liable to error, and hence the SS does not know the expected bit sequence.

From the received burst from the MS transmitter the SS must demodulate the signal to derive (error free) the transmitter burst bit pattern.  Using this bit pattern the SS can calculate the expected phase trajectory according to the definition within Recommendation 05.04.

e)  From (c) and (d) the phase error trajectory is determined and the regression line calculated.  This regression line is used to determine the frequency error for the single burst.

f)  Steps (c) to (e) are repeated for 5 traffic channel bursts spaced over a period of not less than 20 seconds.

g)  The SS shall release the call. The SS increases the level of the serving cell BCCH to 2022 dBuVemf( ) and sets up a call on the traffic channel with an MS input level of 2022 dBuVemf( ) , with the fading function set to HT100. The SS shall capture the first transmitted burst from the MS during call establishment.

h)  Repeat steps (b) - (f).

i)  The SS shall release the call. The SS increases the level of the serving cell BCCH to 2022 dBuVemf( ) and sets up a call on the traffic channel with an MS input level of 2022 dBuVemf( ), with the fading function set to TU50. The SS shall capture the first transmitted burst from the MS during call establishment.

j)  Repeat steps (b) - (f).

k)  The SS shall release the call. The SS increases the level of the serving cell BCCH to 2830 dBuVemf( ) and sets up a call on the traffic channel with an MS input level of 2830 dBuVemf( ) , with each channel applied to the MS via the SS fading function set at TU31.5. The SS shall capture the first transmitted burst from the MS during call establishment.

l)  The SS shall now apply to the MS two independent interfering signals on the same nominal carrier frequencies as the active channels. These interfering signals shall be at a level of 1921 dBuVemf( ) each via a fading function set to TU31.5 and shall be modulated with random data, including the midamble period.

m)  Wait 100 seconds for the MS to stabilize to these conditions.

n)  Repeat steps c) to f), except that at step (f) the measurement period must be extended to 200 seconds and the number of measurements increased to 20

o)  Steps (a) - (n) are repeated for ARFCN of 1513 - 5523 and 120874 - 124884.

p)  Repeat steps (i) and (j) for the combinations of extreme voltage and temperature (see Annex 1, TC 2.2 and TC 3) shown below:

Temp.	Hi  Hi  Lo  Lo

Volt.	Lo  Hi  Lo  Hi

II.3.2.2	Limits

For all test conditions specified in II.3.2.1 the frequency error, with reference to the SS carrier frequency, for every burst shall be less than the values shown in the table below.

Propagation	Permitted

Condition	Frequency Error

RA250130	+/- 300400  Hz

HT100	+/- 175350  Hz

TU50	+/- 135260  Hz

TU31.5	+/-  95180  Hz

II.3.3.	Peak transmitter carrier power & burst timing

ref: 05.05-DCS

II.3.3.1	Definition

The transmitter carrier power is the power delivered to an artificial antenna or radiated by the MS and its integral antenna.

The peak transmitter carrier power is the average value of the transmitter carrier power over the time that the useful information bits of one burst are transmitted.

The peak received transmitter carrier power is the peak transmitter carrier power, attenuated by propagation and as received by the SS during measurements on equipment with an integral antenna.

The peak transmitter carrier power is defined for each power control level at which a MS may operate.

The power class (the class of maximum peak transmitter carrier power) shall be stated by the manufacturer.

The power control level is a parameter which indicates a specific value for the peak transmitter carrier power.

The transmit burst under reference conditions should be timed to occur 3 burst periods (Timing Advance = 0) after the corresponding received burst.  The timings are referenced to the transition from bit 13 to bit 14 of the Training Sequence ("midamble") before differential decoding

Note: 	Within the current standard,also an ME implementation with a deviation of the bursts by 1/2 bit prior to the defiend timing reference is acceptable. In this case the manufacturer has to notify the test house. For type testing to the current standard, this is taken into account by considering the uncertainty margins within the SS as if extended in front of the rising and falling edge each by 1/2 bit (1.84 us) as shown in Figure App.3-1 of GSM 11.40-DCS, Appendix 3. In caes of such an ME failing the test, the test house will have to revise the test result of the SS to assess the appropriate verdict to be given to the Test Report.

II.3.3.2	Methods of measurement

Two methods of measurement are described, separately for:

1) equipment fitted with an antenna connector and for

2) equipment fitted with an integral antenna, and which cannot be connected to an external antenna.

II.3.3.2.1	Method of measurement for equipment with an antenna connector

a)  The MS shall be connected to the SS.  A call shall be originated by the SS to the MS and answered by the MS.  The call shall be on a radio frequency in the range 60690 - 65710 (ARFCN), power control level set to Max power and timing advance 0.

│

+4 dB │        ┌───┐

+1.0 dB │        │   └─────────────/ /──────────────┐

0 dB │        │                                  │

-1.0 dB │        │   ┌─────────────/ /──────────┐   │

│        │   │                          │   │

-6 dB │     ┌──┘   │                          │   └──┐

│     │      │                          │      │

-30 dB │ ┌───┘      │                          │      └───┐

│ │          │                          │          │

│ │          │                          │          │

-70 dB │─┘          │                          │          └---

│            │<-------- (147 bits) ---->│

└──────────────────────────/ /──────────────────────-

10  8   10     7056/13=542,8 microsec.10   8  10

Fig.II.3.3. Power/time template. For a transmitter carrier power lower than 3423 dBm, where the level of -70 dB is lower than - 3647 dBm, the -70 dB shall be replaced by a value which is equivalent to - 3647 dBm.

b)  Measure Peak Transmitter Carrier Power

Using a sampling power measurement method with a sampling rate of at least 2/T, where T is bit duration, capture a representation of the MS transmit burst's  amplitude and timing. From the array of samples the SS identifies the centre of the useful 147 transmitted bits, i.e. the transition from bit 13 to bit 14 of the midamble, as the timing reference.  This enables the peak transmitted carrier power over the 147 useful bits to be calculated and used as the 0dB reference.

c)  Measure Burst Timing

The transition point identified in (b) above is referred to the corresponding transition in the MS received burst.

d)  Match Power/Time Template

The array of power samples shall be tested for a fit within the template of Fig. II.3.3.  The centre of the template must be centred on the measured transition from bit 13 to bit 14 of the midamble of the measured burst.

e)  The SS shall then command the MS to each of the 1611 power control levels applicable to the MS power class(see table II.3.3) and steps (b) - (d) shall be repeated.

f) The SS shall command the MS to the maximum power control level and steps b) to d) shall be repeated for ARFCN in the ranges 1 to 5, 30 to 35, 90 to 95 and 120 to 124512 to 527, 605 to 620, 791-806, 869 to 884..

│

+4 dB │        ┌───┐

+1.0 dB │        │   └─────────────/ /─────────────┐

0 dB │        │                                 │

-1.0 dB │        │   ┌─────────────/ /─────────┐   │

│        │   │                         │   │

-6 dB │     ┌──┘   │                         │   └──┐

│     │      │                         │      │

-30 dB │ ┌───┘      │                         │      └───┐

│ │          │                         │          │

│ │          │                         │          │

-70 dB │─┘          │                         │          └────

│            │<-------- (87 bits) ---->│

└──────────────────────────/ /-------------------------

10  8   10 4176/13=321,2 microsec.  10   8    10

Fig. II.3.4: Power/Time Template for Access Burst. For a transmitter carrier power lower than 3423 dBm, where the level of -70 dB is lower than - 3647 dBm, the -70 dB shall be replaced by a value which is equivalent to -3647 dBm.

g)  The SS shall cause the MS to generate an Access Burst on an ARFCN in the range 60690 - 65710.  The SS shall capture this burst as described in (b) above.  However, in this case the SS needs to locate the centre of the useful bits of the burst by identifying the transition from the last bit of the sync sequence.  The center of the burst as the timing reference is then five data bits prior to this point.

h)  Determine the peak transmitted carrier power of the useful 87 bits of the burst, the 87 bits being symmetrically disposed about the centre point derived in (g). This gives the 0dB reference.

i)  Determine the time of the centre point derived in (g) with respect to the MS received data on the common control channel.

j)  The captured burst shall fit the template of Fig. II.3.4 with the centre of the template horizontal axis located on the centre located in (g) above.

k)  The SS shall modify the control data on the serving cell BCCH in order to limit the MS transmit power on the Access burst to power control level 10 (+2310 dBm). Steps (g) to (j) shall be repeated.

l)  Tests (a) to (k) shall be repeated for the following extreme test conditions (see Annex 1, TC2.2 ans TC3) except that for step e) only power control levels 10 and 15 the minimum power control level and the level 10 dB above that shall be tested:

Temp.	Hi	Hi  	Lo  	Lo

Volt.	Lo	Hi  	Lo  	Hi

II.3.3.2.2	Method of measurement for equipment with an integral antenna

NOTE: If the MS is equipped with a connector, such that the antenna can be disconnected and the SS be connected directly, then the method of clause II.3.3.2.1 shall be applied.

The MS shall be in the anechoic shielded chamber (clause A1.1.2.5) or on the test site, on an isolated support, in the position for normal use.

A test antenna, connected to the SS, shall be in the anechoic shielded chamber or on the test site, at a distance of at least 3 metres from the MS

Note: The test method described has been written for measurement in an anechoic shielded chamber. If an outdoor test site is used, then it is additionally necessary to raise/lower the test antenna through the specified height range to maximise the received power levels from both the test sample and the substitution antenna

a)  A call shall be originated by the SS to the MS and the MS shall be made to answer the call. The call shall be on an  ARFCN, in the range 60690 - 65710, power control level set at MS maximum power, and timing advance 0.

b)  Measure Peak Received Transmitter Carrier Power

Using a sampling power measurement method with a sampling rate of at least 2/T, where T is bit duration, capture a representation of the MS transmit burst's  amplitude and timing. From the array of samples the SS identifies the centre of the useful 147 transmitted bits, i.e. the transition from bit 13 to bit 14 of the mid amble, as the timing reference.  This enables the peak received transmitter carrier power over the 147 useful bits to be calculated and used as the 0dB reference.

c)  Match Power/Time Template

The array of power samples shall be tested for a fit within the template of Fig.  II.3.3.  The centre of the template must be centred on the measured transition from bit 13 to bit 14 of the mid amble of the measured burst.

d)  Measure Burst Timing

The transition point identified in (b) above is referred to the corresponding transition in the MS received burst.

e)  The SS shall then command the MS to each of the 1611 power control levels applicable to the MS power class(see table II.3.3) and steps (b) - (d) shall be repeated.

f)  The SS shall command the MS to the maximum power control level and steps b) to d) shall be repeated for ARFCN in the ranges 1 to 5, 30 to 35, 90 to 95 and 120 to 124512 to 527, 605 to 620, 791-806, 869 to 884.

g)  The SS shall cause the MS to generate an access burst on an ARFCN in  the range 60690 - 65710. The SS shall capture this burst as described in (b) above. However, in this case the SS needs to locate the centre of the useful bits of the burst by identifying the transition from the last bit of the sync sequence. The center of the burst , as the timing reference is then five data bits prior to this point.

h)  Determine the peak received transmitter carrier power of the useful 87 bits of the burst, the 87 bits being symmetrically disposed about the centre point derived in (g).

i)  The captured burst shall fit the template of Fig. II.3.4 with the centre of the template horizontal axis located on the centre located in (g) above.

j)  Determine the time of the centre point derived in (g) with respect to the MS received data on the common control channel.

k)  The SS shall modify the control data on the serving cell BCCH in order to limit the transmit power on the Access Burst to power control level 10(+2310dBm). Steps (g) to (j) shall be repeated.

l)  The MS shall be rotated by n*45° for all values of n in the range 1-7. At each rotation step (b) shall be repeated for all channels of steps (f) and (a) at the maximum power control level..

m)  The MS shall be replaced by a half-wave dipole, resonating at the centre frequency of the transmit band (902.5 1747.5 MHz), connected to an RF generator.

n)  The output power of the RF generator shall be adjusted to reproduce the peak received transmitter power recorded in steps (e) ( maximum power control level ) (f) and (l). For each indication the power, delivered by the generator (in Watts) to the half wave dipole, shall be recorded. These values shall be ordered in a matrix where the columns of the matrix represent the eight orientations of the MS and the rows of the matrix represent the 5 channels specified in steps(a) and (b). Record values in the form Pnc where n = MS rotation and c = channel number.

For each channel number used compute:

n=7

Pac (Watts into dipole) =   0.125  *  Σ  Pnc

n=0

from which: Pac (Tx dBm) = 10log10(Pac) + 30 + 2.15

o)  The difference for each of the five channels between the actual peak transmitter carrier power level averaged over the 8 measurement orientations determined in step (n) and the relative peak transmitter carrier power level for the maximum power control level derived in steps (e) and (f) is used to relate the relative measurement results of steps (e), (f), (g) and (k) to actual peak transmitter carrier powers for all measured power control levels, which can be checked against the requirements of Table II.3.3.

Note:	The next steps are to determine the peak transmitter carrier power under extreme test conditions. Basically the procedure is such that:

-	the power/time template is tested in the "normal" way

-	the radiated power is measured by measuring the difference with respect to the radiation under normal test conditions.

p)  The MS is placed in the climatic test chamber and is linked to the SS via an antenna coupling device.

q)  Under normal test conditions steps (a) - (k) shall be repeated. The peak received power level shall be noted again for all power control levels and the access burst on every frequency used except that for step e) only power control levels 10 and 15 the minimum power control level and the level 10 dB above the minimum shall be tested..

Note:	The values noted here are related to the peak transmitter carrier power level under normal test conditions, which are known after step (o). Therefore frequency dependent calibration factors that account for the effects of the antenna coupling device can be determined.

r)  Steps (a) to (k) shall be repeated for the following combinations of extreme test conditions (see Annex 1, TC2.2 and TC3) except that for step e) only power control levels 10 and 15 the minimum power control level and the level 10 dB above the minimum  shall be tested :

Temperature:	Hi   	Hi   	Lo   	Lo.

Test voltage:	Hi   	Lo   	Hi   	Lo.

s)  The peak transmitter carrier power under extreme test conditions is calculated for each power control level and for every frequency used by adding the calibration factor. i.e. the difference, obtained in the readings of step (o) with respect to the reading obtained in step (q) to the values obtained in step (n).

II.3.3.3	Requirements

a)  The peak transmitter carrier power, under normal and under extreme test conditions, at each frequency and for each power class shall be within the tolerances as shown in table II.3.3.

POWER	POWER	PEAK TRANSMITTED	TOLERANCES

CLASS	CONTROL	CARRIER POWER

LEVEL

1  2  3  4  5		dBm	N.T.C.	E.T.C.

X	 0 	4330 	+/-2.0dB 	+/-2.5dB

X	 1 	4128 	+/-3dB	+/-4dB

X  X	 2 	3926 	+/-3dB*) 	+/-4dB*)

X  X  X	 3 	3724 	+/-3dB*) 	+/-4dB*)

X  X  X	 4 	3522 	+/-3dB	+/-4dB

X  X  X  X	 5 	3320 	+/-3dB 	+/-4dB

X  X  X  X	 6 	3118 	+/-3dB	+/-4dB

X  X  X  X  X	 7 	2916 	+/-3dB 	+/-4dB

X  X  X  X  X	 8 	2714 	+/-3dB	+/-4dB

X  X  X  X  X	 9 	2512 	+/-34dB  	+/-45dB

X  X  X  X  X	10 	2310 	+/-34dB  	+/-45dB

X  X  X  X  X	11 	21 8 	+/-34dB  	+/-45dB

X  X  X  X  X	12 	19 6 	+/-34dB  	+/-45dB

X  X  X  X  X	13 	17 4 	+/-34dB  	+/-45dB

X  X  X  X  X	14 	15	+/-3dB	+/-4dB

X  X  X  X  X	15 	13	+/-3dB	+/-4dB

*)  When the power control level corresponds to the power class of the mobile station, then the maximum tolerances shall be 2.0 dB under normal test conditions and 2.5 dB under extreme test conditions.

TABLE II.3.3   Peak transmitted carrier power for different power classes.

b)  The difference of the peak transmitted carrier power for two adjacent power control levels (measured at the same frequency, under the same combination of test conditions), shall not be less than 0.5 dB and not be more than 3.5 dB, and the power control steps must form a monotonic sequence.

c)  The power/time curve shall be within the limits of the power time template of figure II.3.3 at each frequency, at each power control level and under every combination of normal and extreme test conditions.

d)  All the power control levels, for the power class of the MS as stated by the manufacturer, from the maximum power control level down to (including) power control level 15 level 10 (10 dBm) for class 1 or 13 (4 dBm)for class 2 up to the maximum peak power corresponding to the class of the particular mobile station shall be implemented in the MS

e)  When the transmitter is commanded to a power control level with a peak transmitted carrier power exceeding the peak transmitter carrier power corresponding to the power class of the MS as stated by the manufacturer, then the transmitter carrier power shall be within the tolerances for the highest peak transmitted carrier power corresponding to the power class as stated by the manufacturer.

f)  The centre of the transmitted burst as defined by the transition of bits 13/14 of the midamble shall be 3 timeslot periods (1731 microsecond) +± 1/2 bit period (±1.83.69 microsecond) after the corresponding received burst.

g)  The access burst shall fit the power time template of Fig. II.3.4 for all conditions of test.

h)  The centre of the transmitted access burst as defined by the bit transition described in II.3.3.1 shall be an integer number of timeslot periods less 30 bit periods relative to any CCCH midamble center. The permitted tolerance is +± 1 bit period (+±3.69 microseconds).

II.3.4	Output RF Spectrum

Ref: GSM 05.05-DCS section 4.3

II.3.4.1	Definition

The output RF power spectrum is the radio frequency spectrum, produced by the MS due to the effect of modulation and power ramping, on frequency bands adjacent to the nominal carrier frequency of the MS.

II.3.4.2	Modulation and switching transients

II.3.4.2.1	Method of Measurement

a) If the Mobile Station is equipped with an antenna connector, then the MS is connected to the SS and is operated under normal test conditions.

If the MS is equipped with an integral antenna, and it cannot be operated via an antenna connector, then the MS  is coupled to the SS via an antenna coupling device (Annex 1, GC6), and is operated under normal test conditions.

b) A call shall be originated by the SS to the MS. The MS shall be made to answer the call.

The MS shall be commanded into the hopping mode. The hopping pattern shall include only three channels, namely one with an ARFCN in the range 1 to 5 513 to 523, a second one with an ARFCN in the range 60 to 65 690 to 710 and the third one with an ARFCN in the range 120 to 124. 874 to 884.

Range 512 format of frequency list (GSM 04.08-DCS section 10.5.2.9a.4) shall be used to address the mobile allocation used for frequency hopping.

Note:	Although the measurement is made whilst the MS is in hopping mode, each measurement is on one single channel.

When averaging is in use during frequency hopping mode, the averaging shall only include bursts transmitted when the hopping carrier corresponds to the nominal carrier of the measurement.

c) The SS commands the MS to create the loop back facility from the receiver decoder output to the transmitter encoder input.

d) The MS is operated in the encrypted mode.

e) The SS shall send Standard Test Signal C1 to the MS at a level of 23dBmicroVolt emf(   ). The transmitter signal received from the MS shall be applied to a spectrum analysis function within the SS. The centre frequency of the spectrum analysis function shall be tuned to the hop pattern channel with an ARFCN in the range 60 to 65690 to 710.

f) The MS shall be commanded to the MS Max Power level. The settings of the spectrum analyser shall be adjusted as follows:

Zero frequency scan

Resolution bandwidth:	 30 kHz

Video bandwidth     :	 30 kHz

Video averaging     :	may be used, depending on the implementation of the

test

The video signal of the spectrum analyser is "gated" such that the spectrum generated by at least 40 of the bits 87 to 132 of the burst is the only spectrum measured. This gating may be analogue or numerical, dependent upon the design of the spectrum analyser.

Only measurements during transmitted bursts on the nominal carrier of the measurement shall be included. The spectrum analyser shall average over the gated period and over 200 such bursts, using numerical and/or video averaging.

g) The power level shall be measured at the following frequencies:

FT

FT + 100 kHz			FT - 100 kHz

FT + 200 kHz			FT - 200 kHz

FT + 250 kHz			FT - 250 kHz

FT + 200 kHz * N			FT - 200 kHz * N

where N = 2, 3, 4, 5, 6, 7, 8 and 9

and  FT = RF channel nominal centre frequency.

g1) The analyser settings shall be adjusted to

Zero frequency scan

Resolution bandwidth		100 kHz

Video bandwidth		100 kHz

Video averaging

The power level measurement shall be continued beyond the range of the step g) measurements and extending over the whole MS Tx band.

1710.2 + 0.2*(n-512) MHz (512<= n <= 885)

For each frequency the averaging shall be done for 3 separate groups of bursts, each group corresponding to when the transmitter is on one of the 3 hop frequencies. The averaging shall be over 50 transmitted bursts per group.

h) The measurement at frequencies up to FT +/- 1800 kHz shall be repeated for the minimum MS power control level 15.

i) The analyser settings are adjusted to:

Zero frequency scan

Resolution bandwidth:	 30 kHz

Video bandwidth     :	100 kHz

Peak hold

j) The MS shall be commanded to powermaximum power. level 0. The spectrum analyser power levels shall be measured at the following frequencies:

FT + 400 kHz	FT - 400 kHz

FT + 600 kHz	FT - 600 kHz

FT + 1.2 MHz	FT - 1.2 MHz

FT + 1.8 MHz	FT - 1.8 MHz

where FT = RF channel nominal centre frequency.

The duration of each measurement (at each frequency) shall be such as to cover at least 10 burst transmissions.

k) Step j) shall be repeated for the minimum power control level.s 7 and 11.

l)  The spectrum analysis function shall be tuned to the hop pattern channel with an ARFCN in the range513 to 523. Steps f) to g1) and i) to j) shall be repeated except that at step j) the MS shall be commanded to minimum power level.

m)  The spectrum analysis function shall be tuned to the hop pattern channel with an ARFCN in the range 120 to 124.874 to 884. Steps f) to g1) and i) to j) shall be repeated except that at step j) the MS shall be commanded to minimum power level.

n) The MS shall be commanded to full power and steps e) and f) shall be repeated, except that the System Simulator set to the conditions of e) and g1). T the power level shall be measured over the range 935 to 960 MHz in 30 kHz steps,at the following frequencies

1805.2 + 0.2*(n-512) 	(512<= n <=885)

935.2 + ).2*(n-1) 	(1<= n <=124)

and with the spectrum analyser set to a resolution bandwidth of 100kHz and a video bandwidth of 300100 kHz. and at each step,For each frequency the averaging shall be done forin 3 separate groups of bursts, each group corresponding to when the transmitter is on one of the 3 hop frequencies. The averaging shall be over at least 50 transmitted bursts per group.

o) The MS is placed in a climatic test chamber (for extreme test conditions see Annex 1, TC).  If the MS cannot be operated via an antenna connector, then the antenna coupling device is also placed in the test chamber.

p) Steps  e) to g1) and i) to j) shall be repeated - except that at step i) the MS shall be commanded to power level 11 - under the following combinations of extreme test voltages and ambient temperatures (Annex 1, TC2.2 and TC3):

Temp.		Hi  Hi  Lo  Lo

Volt.		Hi  Lo  Hi  Lo

II.3.4.2.2	Limits

For the modulation sidebands of steps g) and h), the power level relative to a measurement in 30 kHz on the carrier frequency shall not exceed the values shown in Table II.3.4.2. Offsets from the carrier frequency are shown in kHz.

power	power	Maximum level, relative to measurement on the carrier

control	level	frequency (dB) at the listed frequency offsets from the nominal

level	(dBm)	carrier frequency (kHz)

0 	 100 	200 	250 	       400	>=600

ant.	int. 	ant.	int.

con.	ant. 	con.	ant.

0	 43 	0 	+0.5 	-30 	-33 	-60	-70

2	 39 	0 	+0.5 	-30 	-33 	-60	-66

3	 37 	0 	+0.5 	-30 	-33 	-60 	-58	-64	-58

>= 5	<= 33 	0 	+0.5 	-30 	-33 	-60 	-54	-60 	-54

ant. con.   = for MS with antenna connector

int. ant.   = for MS with integral antenna

power 	 	 measurement bandwidth

level

(dBm) 	 		 30 kHz 			100 kHz

100 	200 	250 	400 	600 to 	>1800 to	>6000

1800 	 6000

30 	+0.5 	-30 	-33 	-60 	-60	 -65	 -75

28 	+0.5 	-30 	-33 	-60 	-60	 -63	 -73

26 	+0.5 	-30 	-33 	-60 	-60	 -61	 -71

<= 24 	+0.5 	-30 	-33 	-60 	-60	 -59	 -69

TABLE II.3.4.2

For all power levels tested tThe maximum power measured at step n) in the band 935 to 960 MHz shall be shall be no more than -77 dBm and in the band 1805 to 1880 MHz shall be no more than -71 dBm except in up to 5 measurements where a level of up to -36 dBm is permitted in each set of measurements grouped by the hopping frequencies.

-76 dBm for class 1 MSs, and

-84 dBm for class 2, 3, 4 or 5 MSs.

Note 1:	For each value of FT in the combined frequency range formed both by:

1) Step g) measurements in the range FT-1800 kHz to FT-600 kHz and FT+600 kHz to FT+1800kHz,

and

2) Step g1) measurements in the range FT-6000 kHz to FT-1800 kHz and FT+1800 kHz to FT+6000 kHz.

Up to a total of 3 measurements in the combined set of measurements grouped by the hopping frequencies can be up to -36 dBm.

Note 1 :	For each value of FT, 1 measurement in the combined range FT + 400 kHz to FT + 1800 kHz, and FT - 400 kHz to FT - 1800 kHz may be at -36 dBm.

In the range 935 to 960 MHz, within each group of bursts, measurements within 5 GSM RF channels may be at -36 dBm.

Note 2:	MSs of power class 1 will test power control levels 0 and 15,class 2 will test power control levels 2 and 15, class 3 will test power control levels 3 and 15, class 4 will test power control levels 5 and 15 and class 5 will test power control levels 7 and 15.

Note 2 :	Up to 12 measurements (in each set of measurements grouped by the hopping frequencies) above 6 MHz offset from the nominal carrier frequency to the edge of the transmit band in test g1) may be at a level up to -36 dBm.

Note 3 :	As a result of GSM 05.05-DCS section 4.2.1 note iii, relaxations of the measurements in steps g and h are possible where :

1) For offsets between 100 kHz and 600 kHz if the table indicates that measurements performed in a 30kHz bandwidth relative to a measurement in 30 kHz on the carrier would result in an absolute level measured in 30 kHz of less than -36 dBm then a value of -36 dBm shall apply for offsets of up to (and including) 600 kHz from the carrier.

2) For offsets greater than 600 kHz up to (and including) 1800 kHz. if the table indicates that measurements performed in a 30 kHz bandwidth relative to a measurement in 30 kHz on the carrier would result in an absolute level measured in 30 kHz of less than -56 dBm then a value of -56 dBm shall apply.

3) For offsets greater than 1800 kHz if the table indicates that measurements performed in a 100 kHz bandwidth relative to a measurement in 30 kHz on the carrier would result in an absolute level measured in 100 kHz of less than -51 dBm then a value of -51 dBm shall apply.

For the power ramp sidebands of steps j) and k) the power levels must not exceed:

POWER	MAXIMUM LEVEL FOR VARIOUS OFFSETS FROM CARRIER FREQUENCY:

LEVEL 	400 kHz	 600 kHz	1200 kHz	1800 kHz

43 dBm 	 -9 dBm	 -21 dBm	 -21 dBm	 -24 dBm

41 dBm 	-11 dBm	 -21 dBm	 -21 dBm	 -24 dBm

39 dBm 	-13 dBm	 -21 dBm	 -21 dBm	 -24 dBm

37 dBm 	-15 dBm	 -21 dBm	 -21 dBm	 -24 dBm

35 dBm 	-17 dBm	 -21 dBm	 -21 dBm	 -24 dBm

33 dBm 	-19 dBm	 -21 dBm	 -21 dBm	 -24 dBm

31 dBm 	-21 dBm	 -23 dBm	 -23 dBm	 -26 dBm

29 dBm 	-23 dBm	 -25 dBm	 -25 dBm	 -28 dBm

27 dBm 	-23 dBm	 -26 dBm	 -27 dBm	 -30 dBm

25 dBm 	-23 dBm	 -26 dBm	 -29 dBm	 -32 dBm

23 dBm 	-23 dBm	 -26 dBm	 -31 dBm	 -34 dBm

<= +21 dBm 	-23 dBm	 -26 dBm	 -32 dBm	 -36 dBm

(8 dB allowance)                  (6 dB allowance) 	(3 dB allowance)

POWER	MAXIMUM LEVEL FOR VARIOUS OFFSETS FROM CARRIER

FREQUENCY:

LEVEL	 400 kHz 	600 kHz	1200 kHz	1800 kHz

30 dBm	 -22 dBm 	-24 dBm	 -24 dBm	 -27 dBm

24 dBm	 -23 dBm 	-26 dBm	 -30 dBm	 -33 dBm

<= +20 dBm	 -23 dBm 	-26 dBm	 -32 dBm	 -36 dBm

Note:	These figures are different from the requirements in GSM 05.05-DCS because of alignment with GSM 05.05 where it was found that at higher power levels it is the modulation spectrum which is being measured using a peak hold measurement. This The GSM 11.10 allowance has an impact down to the lower power levels of GSM 05.05-DCS resulting is given in the given table.

The results of the measurements on MSs with an antenna connector shall be in dBm.  The results of the measurements on equipment with no possibility to use an antenna connector shall be in dBs relative to the average transmitter carrier power (see section II.3.3) and then converted into dBm by subtracting this value from the average transmitter carrier power, taking the actual power level and filter bandwidths into account.

II.3.5 Intermodulation attenuation

II.3.5.1 	Definition

The intermodulation attenuation is a measure of the capability of an RF transmit equipment to inhibit the generation of signals in its non-linear elements caused by the presence of the carrier and an interfering signal reaching the equipment via its antenna.

The intermodulation attenuation is expressed as the ratio in dB of the output power level of the transmitter under test to the power level of the highest intermodulation component.

For further information see recommendation GSM 05.05-DCS section 4.7.3.

II.3.5.2	Methods of Measurement

Two methods of measurement are described separately for:

1) equipment fitted with an antenna connector and for

2) equipment fitted with an integral antenna, and which cannot be connected to an external antenna.

SFH shall be disabled during this measurement.

II.3.5.2.1	Method of measurement for equipment with an antenna connector

a)  The MS shall be connected to the SS. A call shall be originated by the SS to the MS and answered by the MS. The call shall be on a radio frequency in the range 690-710 (ARFCN), power control level set to maximum power.

b)  The antenna output of the MS shall be connected to the SS via a coupling device, presenting to the MS a load with an impedance of 50 ohms. The coupling device may consist of a circulator one port of which is to be connected by a coaxial cable to the output terminal of the MS, the second port is to be correctly terminated with 50 ohms into a selective measuring device (eg a spectrum analyser, peak hold) with a measurement bandwidth of 300 kHz. The third port of the circulator is to be connected to the unwanted test signal source by means of an isolator.

c)  The unwanted test signal shall be unmodulated and the frequency shall be 800 kHz above the frequency of the MS under test. The unwanted test signal power level shall be adjusted to be 40 dB below the power level of the MS.

d)  The power level of the test signal shall be measured at the antenna output end of the coaxial cable, when disconnected from the RF transmit equipment and then correctly matched into 50 ohm. The antenna output power of the MS shall be measured directly at the antenna output terminal connected to an artificial antenna.

e)  Any intermodulation components shall then be measured in the band indicated in table xx (of section II.3.5.2.3) by means of the selective measuring device.

f)  This measurement shall be repeated with the test signal at a frequency 800 kHz below the transmitted frequency.

NOTE:	When the above measurements are performed precautions must be taken, so that non-linearities in the selective measuring device do not influence the results appreciably. Furthermore it should be ensured that intermodulation components which may be generated by non-linear elements in the test equipment (eg signal generator, circulators, selective measuring device) are sufficiently reduced. The RF transmit equipment under test and the test signal source shall be physically separated in such a way that the measurement is not influenced by direct radiation.

II.3.5.2.2	Method of measurement for equipment with an integral antenna

a)  The MS shall be placed in the anechoic chamber as in section II.3.3.2.2.and the procedure of part a) of that section shall be repeated. The coupling factor F between the mobile and the test antenna shall be calculated as the difference in dB between Pac as defined in section II.3.3.2.2 and the power measured.

b)  The test antenna shall be connected to the SS via a coupling device, presenting to the antenna a load with an impedance of 50 ohm. The coupling device may consist of a circulator one port of which is to be connected by a coaxial cable to the antenna terminal, the second port is to be correctly terminated with 50 ohm into a selective measuring device (eg a spectrum analyser, peak hold) with a measurement bandwidth of 300 kHz. The third port of the circulator is to be connected to the unwanted test signal source by means of an isolator.

c)  The unwanted test signal shall be unmodulated and the frequency shall be 800 kHz above the frequency of the MS under test.The standard test signal Io, power level shall be adjusted to be 40 dB below the power level of the MS at the MS antenna. This is given by:

Pac(Tx dBm) - 40 + F    dBm

d)  The power level of the test signal shall be measured at the antenna output end of the coaxial cable, when disconnected from the RF transmit equipment and then correctly matched into 50 ohms.

e)  Any intermodulation components shall then be measured in the band indicated in table xx (of section II.3.5.2.3) by means of the selective measuring device.

f)  This measurement shall be repeated with the test signal at a frequency 800 kHz below the transmitted frequency.

NOTE: 	When the above measurements are performed precautions must be taken, so that non-linearities in the selective measuring device do not influence the results appreciably. Furthermore it should be ensured that intermodulation components which may be generated by non-linear elements in the test equipment (eg signal generator, circulators, selective measuring device) are sufficiently reduced. The RF transmit equipment under test and the test signal source shall be physically separated in such a way that the measurement is not influenced by direct radiation.

II.3.5.2.3	Requirements

The intermodulation limit is as indicated below under normal test conditions. The limits express the minimum intermodulation attenuation in dB.

Test Signal	Limit	Frequency Band

displacement (x)

0.8 MHz	50dB	1710-1785 MHz