GSM Protocol Stack

SolSA

Concept (First draft for discussion)

Confidential

| Author: | Condat AG |
| --- | --- |

| Date: | May 15, 2000 |
| --- | --- |
| ID: | xxxx.xxx.xx.xxx |
| Status: | Beeing processed |

Table of Contents

0	Document Control	3

0.1	Document History	3

0.2	References	3

0.3	Abbreviations	4

0.4	Terms	4

1	Introduction	5

2	Memory consumption	6

3	Changes to existing entities in G23	7

3.1	Changes to SIM	7

3.1.1	New SoLSA related data on the SIM	7

3.1.2	Changes to SIM entity in G23	7

3.2	Changes to ACI/MMI	8

3.3	Changes to RR	8

3.4	Changes to MM	9

3.5	Changes to GRR	9

4	New and changed Information Elements	10

4.1	MM Information	10

4.1.1	LSA Identifier	10

4.2	GMM Information	10

4.3	Mobile Station Classmark 2	10

4.4	System Information Type 4	11

4.4.1	PRIO_THR (3 bit field)	11

4.4.2	LSA_OFFSET (3 bit field)	11

4.4.3	MCC and MNC (24 bit field)	11

4.4.4	Cell Identity (16 bit field)	11

4.4.5	LSA_ID (24 bit field)	11

4.4.6	Short LSA_ID (10 bit field)	11

4.5	System Information Type 7	11

4.6	System Information Type 8	11

4.7	System Information Type 16	11

4.8	System Information Type 17	11

5	New and changed SAPs	12

5.1	RR_SOLSA_REQ	12

5.2	MMR_INFO_IND	12

5.3	GMMR_INFO_IND (or whatever this is called exactly)	12

5.4	SIM_READ_REQ/SIM_READ_CNF	12

5.5	SIM_MMI_INSERT_IND	12

5.6	SIM_MM_INSERT_IND	12

5.7	RR_ACTIVATE_REQ	12

5.8	RR_SYNC_REQ	12

5.9	RR_ACTIVATE_CNF/RR_ACTIVATE_IND	12

6	To be discussed	13

6.1	Idle mode support	13

6.2	LSA Descriptor files	13

6.3	Communication path between RR and SIM	13

# Document Control

ã Copyright Condat AG, 2000.

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Condat AG reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Condat AG. Condat AG accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a license agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Condat AG.

Condat AG

Alt Moabit 91d

10559 Berlin

Germany

Telephone:	+49.30.39094-0

Fax:	+49.30.39094-300

Internet:	www.condat.de

E-mail:	gsm@condat.de

## Document History

ID	Author	Date	Approval	Remarks

xxxx.xxx.xx.xxx	HM	May 8,2000		Initial

## References

[GSM 02.43]	ETSI TS 101 415: August 1999 (GSM 02.43 version 7.1.0 Release 1998)
Support of Localised Service Area (SoLSA); Service Description; Stage 1; ETSI

[GSM 03.03]	ETSI TS 100 927: December 1999 (GSM 03.03 version 7.3.0 Release 1998)
Numbering, addressing and identification; ETSI

[GSM 03.08]	ETSI TS 100 526: December 1999 (GSM 03.08 version 7.2.0 Release 1998)
Organization of subscriber data; ETSI

[GSM 03.22]	ETSI TS 100 930: July 1999 (GSM 03.22 version 7.1.0 Release 1998)
Functions related to Mobile Station (MS) in idle mode and group receive mode; ETSI

[GSM 03.73]	ETSI TS 101 416: August 1999 (GSM 03.73 version 7.1.0 Release 1998)
Support of Localised Service Area (SoLSA); Stage2; ETSI

[GSM 04.08]	ETSI EN 300 940: March 2000 (GSM 04.08 version 7.6.0 Release 1998)
Mobile interface layer 3 specification; ETSI

[GSM 04.60]	Available SMG only: November 1999 (GSM 04.60 version 8.1.0 Release 1999)
Mobile Station (MS) - Base Station (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol; ETSI

[GSM 05.08]	ETSI EN 300 911: July 1999 (GSM 05.08 version 7.1.0 Release 1998)
Radio subsystem link control; ETSI

[GSM 11.11]	ETSI TS 100 977: December 1999 (GSM 11.11 version 7.4.0 Release 1998)
Specification of the Subscriber Identity Module - Mobile Equipment (SIM - ME) interface; ETSI

In this references nothing is included which is unnecessary for SoLSA implementation, standardization of SoLSA really is that much scattered. As a general rule, Condat currently implements the G23 protocol stack according to the Release 97 of the recommendations. This is impossible with SoLSA as this feature is only standardized in Release 98 of the standards and above.

Normally it is expected that if a stage 1 and stage 2 standard of a feature exists, that also a stage 3 standard exists which explains the implementation in detail. The stage 1 standard and the stage 2 standard exists, but there is not yet a stage 3 standard describing the detailed implementation for SoLSA. Interesting is that normally the stage 1, stage 2 and stage 3 standard documents have the same number behind the dot. This is already not true for SoLSA with the stage 1 and stage 2 standard documents. Maybe this indicates that there will never a stage 3 document by ETSI as the stage 2 documentation is comprehensive.

## Abbreviations

ACI 	Application Control Interface (AT Commands)
BA	BCCH	Allocation
BCCH	Broadcast Control CHannel
BSIC	Base transceiver Station Identity Code
CI	Cell Identity
LAC	Location Area Code
LR	Location Register
LSA	Localised Service Area
ME	Mobile Equipment
MCC	Mobile Country Code
MM	Mobilty Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
OTA	Over the Air
RR	Radio Resource
SAP	Service Access Point
SAT	SIM Application Toolkit
SIM	Subscriber Identity Module
SoLSA	Support of Localised Service Area

## Terms

SoLSA exclusive access	Cells on which normal camping is allowed only for MS with Localised Service Area (LSA) subscription. (GSM 03.22 subclause 1.2)

Localised Service Area (LSA)	A localised service area consists of a cell or a number of cells. The cells constituting a LSA may not necessarily provide contiguous coverage. (GSM 03.22 subclause 1.2)

# Introduction

- G23 is a software package implementing Layers 2 and 3 of the ETSI-defined GSM air interface signalling protocol, and as such represents that part of a GSM mobile station’s protocol software which is both, platform and manufacturer independent. Therefore, G23 can be viewed as a building block providing standardised functionality through generic interfaces for easy integration.

The G23 suite of products consists of the following items:

- Layers 2 and 3 for speech & short message services,

- Layers 2 and 3 for fax & data services,

- Application Control Interface,

- Slim MMI [02.30] and

- Test and integration support tools.

This document is the concept for SoLSA support for the G23 development team. This document describes what has to be changed and implemented for the support of SoLSA in G23.

SoLSA is a new GSM Phase 2+ feature which allows the provisioning of locally restricted services. One of the services could be to make non-limited services at places which usually have a high traffic load only available for subscribers which have a special subscription, e.g. business users. This means, people who only have a standard subscription may only have limited service at locations like fair grounds, airports etc. With a mobile phone not supporting SoLSA or no SoLSA subscription it may be impossible to obtain full service in such areas.

Without SoLSA it is already possible to restrict roaming to some areas by network means, but this is only possible on a location area level. Without SoLSA it is impossible to restrict access on the cell level for a group of users aside cell barring for predetermined SIM access classes.

By SoLSA, it will be possible to implement things like "Home Zone", "City Zone" and maybe "Regional Zone" where the user will be offered special tariffs. In every different zone a distinct icon and/or information text may be displayed which will inform the user in which zone (LSA) the mobile phone currently is camped. This sounds like the propietary "Genion" feature which is offered by the german Viag Interkom company, but it should not be confused with this. SoLSA is technically able to offer the user the same features like the propietary "Genion", but technically these are two different solutions. SoLSA has the advantage to be public standardized by ETSI and thus may be globally supported.

For further details about the intentions behind SoLSA, reference shall be made to GSM 02.43 which makes things very clear here.

# Memory consumption

Recommendation GSM 11.11 lacks the determination of some limits for memory consumption. SoLSA is a feature which may need a lot of ME memory, it is somewhat difficult to determine the worst case memory consumtion from the standards as there are no reasonable limitations defined. The maximum size of the EF_SLL may become up to approximately 65000 bytes, so there can no reasonable limit derived by this elementary file. GSM 02.43 subclause 4.1 defines that a subriber can have up to 100 LSAs per PLMN. The subscription to LSAs is not restricted to only one PLMN.

GSM 03.08 subclause 2.4.17 defines that the maximum amount of LSA IDs per subscriber and PLMN shall not exceed 20 LSA IDs per PLMN. The problem with this limit is that a LSA may not only be given by the LSA ID, but also e.g. by a number of cell identities. So this limit is not really a hard limit given by the standards.

For now, we assume there are n records in EF_SLL (GSM 11.11 subclause 10.4.1.2), each defining a different LSA. n may theoretcally become greater than the above mentioned 100 LSAs, as there may be entries for more than only one PLMN.

For every LSA a different icon may be defined. The amount of icons is limited by the size of the Icon Identifier in EF_SLL, by one byte it is possible to distinguish 256 different values. No reasonable limit for the size of an icon could be found in GSM 11.11. It is not expected that an icon becomes too big, as it will be impossible to display such a big icon on a limited ME display, but this case has to be handled also. It is proposed that the icon itself remains in the SIM and is not copied into RAM until it has to be displayed. This will not affect the mobile's operation in any negative way and save a lot of ME memory.

The LSA name is limited to a length of 10 characters by GSM 03.73 subclause 9.1.2. Characters are not to be confused with bytes, as these may be UC2 characters and so every character may be a 16 bit value. It may be considered to hold all possible n LSA names in memory as the memory consumption is limited due to the limited size of a LSA name, assumed n doesn't become too big, but to conserve memory, it is proposed to leave the LSA name on the SIM until it has to be displayed like this should already be done with the icons.

For that, RR has to store the record index of the LSAs in the EF_SLL file and deliver this to the upper layers always it selects a LSA cell. The stored value itself will have no meaning for RR operation itself. The record index will be forwarded by MM to the ACI via the MMR SAP. With this information, ACI/MMI will be able to fetch the record in EF_SLL describing the selected LSA. The interesting fields are the LSA name, the Configuration parameters and the Icon identifier. By a second access on the SIM memory, ACI/MMI may fetch the Icon itself using the Icon Identifier. The icon can then be displayed on the ME's display.

Every one of the n LSA entries contains a LSA Descriptor File Identifier and a LSA Descriptor Record Identifier. The data structure is a forward chained list containing of up to 254 records. Theoretically, the amount of memory beeing used to describe a LSA may be up to approximately 65000 bytes, and this for each LSA the user has subscribed to. It has to mentionend that in informative Annex F of GSM 11.11 it is recommended that the record length in the LSA Descriptor files may not exceed 100 bytes.

It has to be mentioned that all these worst case assumptions may not become true as the physical memory is limited on existing phase 2+ SIMs, but it is difficult to predict where this limits will be in the future. There may be other limits, too.

So with limited ME RAM ressources it will be impossible to completely support the SoLSA features under all theoretically imaginable circumstances, reasonable limits have to be set by the ME manufacturer. As this will be arbitrary limits, these may be overstepped in the field.

It is important to understand that this concept paper was written under the assumption that SoLSA data structures may have an unpredictable size. For this, memory is not allocated statically at some places but dynamically.

This also means there is the danger that there may be so much memory used by the SoLSA related files in the SIM that the mobile will crash due to a lack of available memory. This has to be prevented.

# Changes to existing entities in G23

The following chapter describes the necessary changes and additions to the currently existing G23 protocol software.

## Changes to SIM

There are new elementary files on the SIM due to SoLSA support, but there are also changes in the SIM entity itself to support SoLSA. These will be described below.

### New SoLSA related data on the SIM

The following mandatory information elements shall be stored in the SIM for each service subscriber:

- A list of all the LSA Ids for the service subscriber (LSA description files via EF_SLL)

- The priority for each LSA (EF_SLL)

- The LSA only access indicator (EF_SAI)

The following optional information may be stored in the SIM for each service subscriber

- LSA only access indication text (text to be displayed when out of LSA) (EF_SAI)

The following optional information may also be stored for each subscribed LSA:

- A list of all the cell identities for the subscribed LSA  (LSA description files via EF_SLL)

- LSA name (A text string describing area and / or service. For example "ETSI building" (EF_SLL)

- LSA indication for idle mode (flag to activate/deactivate LSA indication in idle mode) (EF_SLL)

- Idle mode support (flag to activate/deactivate idle mode support in LSA) (EF_SLL)

The information shall be modified when corresponding HLR and BSS records are changed. (According to GSM 03.73 subclause 8.3.1 may this be performed over the air by SMS download)

It is to mention that GSM 03.73 and GSM 11.11 differ here. In GSM 11.11 the only optional field is the LSA name, all other fields are mandatory if SoLSA is supported. It is assumed that GSM 11.11 is more accurate here.

### Changes to SIM entity in G23

SIM has to inform MM about some of the SoLSA related informations. These are the LSA only access indicator and the Idle mode support. MM is not directly interested in these values, but RR is. In the G23 architecture it is not foreseen that RR communicates with SIM, this has to be done via MM. The interesting values are normally delivered by SIM to MM by the mean of the SIM_MM_INSERT_IND primitive. This primitive has to be enhanced. It is to mention that all the values regarding SoLSA may be changed by SAT activity. This may not have a direct impact on the SAT implementation in the SIM entitity, but will have be regarded for MM and RR. It may be discussed whether it is useful to retain the G23 architecture which doesn't allow dircect communication between SIM and RR in the future.

Also the primitive SIM_MMI_INSERT_IND has to be enhanced. The MMI is interested in the complete content of the EF_SAI field (SoLSA Access Indicator), this means the LSA only access indicator and the LSA only access indicator text. All other values which may be interesting for the ACI/MMI have to be fetched dynamicly by the ACI/MMI if a cell belonging to a LSA is selected.

Currently the SIM entity only supports a flat directory structure in the SAPs. This concept has to be extended for the introduction of SoLSA. There are now two files, the LSA Descriptor files, which have the file identification '4Fxx' and reside under DF_SOLSA, and the Image Instance Data Files, which also have the file identification '4Fxx' and reside under DF_Graphics. These files can not be distinguished by ranges, there have to be other means. It is proposed that the DF under which the desired elementary file resides also will be submitted in the requesting primitive in case of ambiguity.

## Changes to ACI/MMI

To support the SoLSA feature, the MMI has to be changed. If there is LSA exclusive access indicated, this fact has to be displayed. The respective indicator and message text will be submitted by the SIM_MMI_INSERT_IND primitive, which has to be enhanced for that. It may be necessary to display the LSA name or an associated icon stored in the SIM when the MS is camped on a cell within the LSA.

Also there has to be support for 3 digit MNC in this entities as this is a prerequisite for the implementation of SoLSA. The following table shows the data which has to be supported:

| Element | Description |
| --- | --- |
| LSA only access indicator | as of EF_SAI |
| LSA only access indication text | as of EF_SAI, assumed max. 10 characters UC2 |
| Record number | Record number of LSA in EF_SLL (submitted on LSA entrance) |
| LSA name | as of EF_SLL, max. 10 characters UC2 (fetched on LSA entrance) |
| Configuration parameters | as of EF_SLL (fetched on LSA entrance) |
| LSA indication for idle mode | as of EF_SLL (fetched on LSA entrance) |
| Icon data | as of EF_IMG (fetched on LSA entrance) |

Except the first two entries in the table, which can be realized by a memory consumption of 21 bytes, the trailing entries in the table should be fetched on demand from the SIM. This will conserve a lot of memory as mirroring this data may cause a lot memory consumtion. It is especially difficult to predict the size of the Icon data, which may become big, and there may be a lot of LSAs a user has subscription to. Also it should not do any harm to the ME's operation if on change of the LSA occasionaly the SIM is queried for the actual data. As ACI already communicates with the SIM entitity in the current G23 architecture, fetching the SIM data on LSA change should be a task of ACI.

## Changes to RR

The implementation of SoLSA support causes some extensive changes in the RR sublayer. This sublayer is the part of G23 which is affected most by the SoLSA feature.

GSM 03.22 subclause 3.2.1 requires that the mobile station should not camp on SoLSA exclusive cell to which the MS does not subscribe. GSM 03.22 subclause 3.3 describes the behaviour of a Localised Service Area which indicates exclusive access only. Cell re-selection algorithms are described in Recommendation GSM 05.08 chapter 6.6, these differ substantially for mobiles supporting SoLSA from those mobiles not supporting SoLSA. Thus cell re-selection algorithms have to be changed substantially in RR.

The following data structure has to be stored for each LSA in the RR sublayer. This data structure is needed to decide whether a cell belongs to a subscribed LSA or not. The size of the data structures are almost unpredictable as already explained above, it is proposed to allocate the memory for this dynamically. If no further checks in regard of memory consumtion are done, a SIM may cause a ME crash due to the lack of memory. There should be checks implemented so that it will be impossible for any SIM to cause the mobile to crash.

| Element | Description |
| --- | --- |
| Pointer to next defined LSA | Data structure of defined LSAs is a forward chained list, as it is almost unpredictable how much LSAs a user may have subscription to. |
| Record number of LSA in EF_SLL | The record number of the selected LSA will be enough information for ACI/MM to obtain the information about a specific LSA from the SIM. |
| LSA only access | LSA only access indicator from EF_SAI. If this is set, the mobile is restriced to LSA cells. |
| Idle mode support | Favour camping on LSA cell from EF_SLL. This is a little bit unclear (to me) as there is already a Priority field. |
| Priority | Priority of given LSA from EF_SLL. Used for cell selection. |
| PLMN | PLMN code of given LSA from EF_SLL |
| LSA descriptor type and number 1 | LSA descriptor type and number from LSA descriptor file |
| ... | ... |
| LSA descriptor type and number m | LSA descriptor type and number from LSA descriptor file |
| 0x00 | End of data structure marker |

As the architecture for G23 does currently not allow direct communication between SIM and RR, this data structure has to be build by various accesses to the SIM by MM and to be forwarded to RR by a newly to be introduced primitive. Otherwise means have to be introduced into G23 to let RR directly communicate with the SIM entitity to build the data structures. This should be discussed, as MM is not interested in the contents of these SIM files and serves only for forwarding here.

Due to the great number of LSAs and the size of the LSA describing data structure the memory consumption by RR may increase dramatically.

Because OTA management is foreseen for the SoLSA related files, SAT has to be supported for these files.

[There should really be some discussion about that all, some points of uncertainty. Memory consumption, architecture and so on.]

## Changes to MM

MM has to inform RR about some SoLSA related changes if no communication between RR and SIM will be established. Also MM may have to build some data structures (the ones described in the previous subclause) and to handle SoLSA related SAT activity. However, MM is not directly interested in any of the new SIM fields for its own operation.

Which primitives should be introduced (if any) for the communication between MM and RR should be discussed.

For SoLSA, MCC and MNC have both a width of 12 bit, which is equal to 3 digits, but G23 currently only supports 2 digit MNC. This means, before implementing SoLSA G23 has to be enhanced to support 3 digit MNC. This was already planned by Condat and affects also ACI, MMI and RR. Support of 3 digit MNC is described in GSM 03.22.

## Changes to GRR

The implementation of SoLSA support also causes changes in GRR. Recommendations GSM 05.08 subclause 10.1 describes the monitoring of PBCCH for mobile stations supporting SoLSA and the cell re-selection criteria. For further details, see the mentioned recommendation.

# New and changed Information Elements

## MM Information

The MM INFORMATION message, as defined in GSM 04.08 subclause 9.2.15a may contain the LSA identity. The contents of this IE indicate the LSA identity of the serving cell. MM is not interested in the contents of the MM Information message for its own functionality and forwards the informations received by this message to upper layers.

### LSA Identifier

This element uniquely identifies a LSA.

The LSA Identifier Information element is coded as shown below.

The LSA Identifier is a type 4 information element with a length of 2 or 5 octets. (GSM 04.08 subclause 10.5.3.11)

LSA identifier IEI

Length of LSA identifier contents

LSA ID

LSA ID cont.

LSA ID cont.

If the Length = 0, then no LSA ID is included. This is used to indicate that the MS has moved to an area where there is no LSA available for that MS.

Octets 3-5 are coded as specified in GSM 03.03, ‘Identification of Localised Service Area’. Bit 8 of octet 3 is the most significant bit.

The LSA Identifier is an optional information element only present in the MM INFORMATION message. (For a definition of the MM INFORMATION message, see Recommendation GSM 04.08 subclause 9.2.15a).

Recommendation GSM 03.03 chapter 11 explains the structure of the LSA ID.

Cells may be grouped into specific localised service areas. These localised service areas are identified by a localised service area identity (LSA ID). No restrictions are placed on what cells may be grouped into a given localised service area.

The LSA ID can either be a PLMN significant number or a universal identity. This shall be known both in the networks and in the SIM.

The LSA ID consists of 24 bits, numbered from 0 to 23 by order of significance, with bit 0 being the LSB. Bit 0 indicates if the LSA is a PLMN significant number or a universal LSA. If the bit is set to 0 the LSA is a PLMN significant number and if it is set to 1 it is a universal LSA.

The LSA ID shall be composed as shown in the figure below:

MSB	LSB

| 23 bits | 1 bit |
| --- | --- |

## GMM Information

This is similar to the MM information message.

## Mobile Station Classmark 2

In Mobile Station Classmark 2 the MS has to announce that it supports SoLSA. This is done by setting the bit 5 in octet 5.

## System Information Type 4

The SoLSA related information is broadcast in the SI 4 Rest Octets. These are defined by GSM 04.08 subclause 10.5.2.35. The relevant parameters for SoLSA are noted below (from GSM 04.08):

### PRIO_THR (3 bit field)

The PRIO_THR field is a signal threshold used by the mobile station to determine whether prioritised cell re-selection

shall apply. The use and coding of this parameters is defined in GSM 05.08.

### LSA_OFFSET (3 bit field)

The LSA_OFFSET field applies an offset for LSA reselection between cells with same LSA priorities. The use and coding of this parameters is defined in GSM 05.08.

### MCC and MNC (24 bit field)

If the escape PLMN is broadcast in SI3 and SI4 the cell is used for SoLSA exclusive access and the MCC and MNC field shall be included. The MS shall then for all purposes use the MCC and MNC values received in the LSA Parameters instead of the ones received in the Location Area information element in SI3 and 4, eg when deriving the PLMN identity, the Location Area Identity and Cell Global Identity broadcast by the cell. The MCC and MNC value field is coded as specified in Figure 10.5.3/GSM 04.08 and Table 10.5.3/GSM 04.08.

### Cell Identity (16 bit field)

The purpose of the Cell Identity is to identify a cell within a location area. The Cell Identity is coded as shown in figure 10.2/GSM 04.08 and table 10.5/GSM 04.08.

### LSA_ID (24 bit field)

The purpose of the LSA_ID field is to identify a LSA. The LSA ID value field is specified in GSM 03.03.

### Short LSA_ID (10 bit field)

The purpose of the Short LSA_ID field is to identify a LSA. The LSA ID defined by the Short LSA_ID is a LSA_ID as specified in GSM 03.03 with bit 0 set to “0” bit 1 to 10 set to the value of the Short LSA_ID field (LSB in bit 1, MSB in bit 10) and bit 11 to 23 set to “0”.

## System Information Type 7

The SoLSA related information is broadcast in the SI 7 Rest Octets. "The SI 7 Rest Octets information element is coded as the SI 4 Rest Octets." (GSM 04.08 subclause 10.5.2.36)

## System Information Type 8

The SoLSA related information is broadcast in the SI 8 Rest Octets. "The SI 8 Rest Octets information element is coded as the SI 4 Rest Octets." (GSM 04.08 subclause 10.5.2.37)

## System Information Type 16

"The SI 16 Rest Octets information element includes parameters which are used by the mobile station for cell selection

and reselection purposes." (GSM 04.08 subclause 10.5.2.37e)

The SI 16 Rest Octets carry the LSA parameters PRIO_THR, LSA_OFFSET, MCC and MNC and the LSA ID information. This means, the information provided by the SI 16 Rest Octets is a subset of these provided by the SI 4 Rest Octets.

## System Information Type 17

"The SI 17 Rest Octets information element is coded as the SI 16 Rest Octets." (GSM 04.08 subclause 10.5.37f)

# New and changed SAPs

This chapter describes the necessary new SAPs and the neccessary additions to already existing SAPs.

## RR_SOLSA_REQ

This primitive may be introduced to deliver the data structures which define each LSA to RR. Another way to obtain the information would be the introduction of a communication path between RR and SIM.

## MMR_INFO_IND

This primitive carries the informations received by the MM INFORMATION message. It has to extended to be able to transport the LSA identity which may be present in the MM INFORMATION message. It is somewhat unclear for what the LSA identity will be good for in this message, as the LSA identity may be already known information due to the system information messages RR receives. The proposed implementation of SoLSA in G23 is not so that the LSA identity may be usefull information for ACI/MMI. So this extension may be omitted.

## GMMR_INFO_IND (or whatever this is called exactly)

This is similar to the MMR_INFO_IND primitive.

## SIM_READ_REQ/SIM_READ_CNF

Currently there is no mean to distinguish elementary files which may have same file IDs (like the LSA descriptor files and the image instance data files), but reside in different diretories. So at least the SIM_READ_REQ primitive has to be enhanced to distinguish these. It is proposed that the directory of the files may be delivered in the primitive data as optional information in the future.

## SIM_MMI_INSERT_IND

This primitive has to be extended to carry the LSA access only indicator and the LSA only access indicator text.

## SIM_MM_INSERT_IND

This primitive has to be extended to carry the LSA access only indicator and the idle mode support indicator (if no communication path between RR and SIM will be introduced).

## RR_ACTIVATE_REQ

This primitive has to be extended to carry the LSA access only indicator and the idle mode support indicator (if no communication path between RR and SIM will be introduced).

## RR_SYNC_REQ

This primitive has to be extended to carry the LSA access only indicator and the idle mode support indicator (if no communication path between RR and SIM will be introduced). As the LSA access only indicator may change due to SAT activity (SMS download etc.), this is necessary.

## RR_ACTIVATE_CNF/RR_ACTIVATE_IND

This primitive has to be extended to carry the record index of the selected LSA. This index points into EF_SLL and describes the LSA for ACI/MMI. With this information, ACI/MMI will be able to obtain the necessary information it needs to inform the user about a selected LSA.

# To be discussed

## Idle mode support

GSM 11.11 subclause 10.4.1.2 defines the EF_SLL field, which contains the field "Configuration parameters", which contains the "Idle mode support" bit. Quotation: "Contents: The idle mode support is used to indicate whether the ME shall favour camping on the LSA cells in idle mode.". The question is: Isn't this controlled by the Priority field? This should be clarified.

## LSA Descriptor files

LSA Descriptor files are defined in GSM 11.11 subclause 10.4.1.3. The LSA may contain entries either containing the LSA ID, the LAC+CI, the CI alone or the LAC. There may be a problem with the CI alone. The CI is only valid within a given location area according to the GSM standars. So the CI alone may not be unique within a given PLMN. The magnitude of cells in the german GSM networks is around 10000 cells per network, there is room for around 65000 cells in the respective GSM 04.08 data structures, so it may be possible for the network operators to choose the cell ids in a way that they are unique within a given PLMN. This has to be considered as a network operator issue, implementation in ME cannot do anything here.

## Communication path between RR and SIM

SoLSA causes a lot of data delivery from SIM to RR. This is unlike the situation before where there was not much need for communication between SIM and RR. It should really be discussed if a new communication path between SIM and RR should be introduced, avoiding unneccessary involving of MM.