GSM Protocol Stack

SolSA

High level Design (draft)

Confidential

| Author: | Condat AG |
| --- | --- |

| Date: | October 2, 2001 |
| --- | --- |
| ID: | xxxx.xxx.xx.xxx |
| Status: | Being processed |

- Table of Contents

0	Document Control	3

0.1	Document History	3

0.2	References	3

0.3	Abbreviations	4

0.4	Terms	4

1	Introduction	5

2	Cell selection/reselection	7

2.1	Cell selection	7

2.2	Cell reselection	7

3	New and modified SAPs	9

3.1	SIM_RR_INSERT_IND / SIM_RR_INSERT_RES(new)	9

3.2	SIM_READ_RECORD_REQ / SIM_READ_RECORD_CNF	9

3.3	SIM_FILE_UPDATE_IND / SIM_FILE_UPDATE_RES	9

3.4	RR_ACTIVATE_CNF/ RR_ACTIVATE_IND	9

3.5	RR_SYNC_REQ	9

3.6	RR_SYNC_IND	9

3.7	MMR_REG_CNF	9

3.8	MMR_INFO_IND	9

4	Message Sequence Charts	10

4.1	Initialisation	10

4.2	Cell selection	11

4.3	Successful cell reselection	13

4.4	Change of LSA ID in the dedicated mode	14

4.5	Update of LSA data OTA (SIM REFRESH Command)	14

4.5.1	Mode: SIM Initialisation	15

4.5.2	Mode: File Change Notification	16

5	LSA Properties	18

5.1	SoLSA feature flag	18

5.2	Constants	18

5.3	Mobile Station Classmark 2	18

5.4	LSA-ONLY-ACCESS flag	19

5.5	LSA ID Information	19

5.5.1	LSA ID	19

5.6	LSA Element	20

5.6.1	Configuration parameters	20

5.6.2	Icon identifier	20

5.6.3	Priority	20

5.6.4	PLMN code	20

5.6.5	LSA ID	21

5.7	LSA Parameters (cell selection/reselection)	21

5.7.1	LSA Offset	21

5.7.2	Priority Threshold	21

5.7.3	Optional PLMN Code	22

# Document Control

ã Copyright Condat AG, 2000.

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Condat AG reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Condat AG. Condat AG accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a license agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Condat AG.

- Condat AG

Alt Moabit 91d

10559 Berlin

Germany

- Telephone:	+49.30.39094-0

Fax:	+49.30.39094-300

Internet:	www.condat.de

E-mail:	gsm@condat.de

## Document History

ID	Author	Date	Approval	Remarks

xxxx.xxx.xx.xxx	XBA	Feb 5, 2001		Initial

XBA	Oct 2, 2001		cell selection/reselection

## References

[GSM 02.43]	ETSI TS 101 415: August 1999 (GSM 02.43 version 7.1.0 Release 1998)
Support of Localised Service Area (SoLSA); Service Description; Stage 1; ETSI

[GSM 03.03]	ETSI TS 100 927: December 1999 (GSM 03.03 version 7.3.0 Release 1998)
Numbering, addressing and identification; ETSI

[GSM 03.08]	ETSI TS 100 526: December 1999 (GSM 03.08 version 7.2.0 Release 1998)
Organization of subscriber data; ETSI

[GSM 03.22]	ETSI TS 100 930: July 1999 (GSM 03.22 version 7.1.0 Release 1998)
Functions related to Mobile Station (MS) in idle mode and group receive mode; ETSI

[GSM 03.73]	ETSI TS 101 416: August 1999 (GSM 03.73 version 7.1.0 Release 1998)
Support of Localised Service Area (SoLSA); Stage2; ETSI

[GSM 04.08]	ETSI EN 300 940: March 2000 (GSM 04.08 version 7.6.0 Release 1998)
Mobile interface layer 3 specification; ETSI

[GSM 04.60]	Available SMG only: November 1999 (GSM 04.60 version 8.1.0 Release 1999)
Mobile Station (MS) - Base Station (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol; ETSI

[GSM 05.08]	ETSI EN 300 911: July 1999 (GSM 05.08 version 7.1.0 Release 1998)
Radio subsystem link control; ETSI

[GSM 11.11]	ETSI TS 100 977: December 1999 (GSM 11.11 version 7.4.0 Release 1998)
Specification of the Subscriber Identity Module - Mobile Equipment (SIM - ME) interface; ETSI

## Abbreviations

ACI 	Application Control Interface (AT Commands)
BA	BCCH	Allocation
BCCH	Broadcast Control CHannel
BSIC	Base transceiver Station Identity Code
CI	Cell Identity
LAC	Location Area Code
LR	Location Register
LSA	Localised Service Area
ME	Mobile Equipment
MCC	Mobile Country Code
MM	Mobilty Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
OTA	Over the Air
RR	Radio Resource
SAP	Service Access Point
SAT	SIM Application Toolkit
SIM	Subscriber Identity Module
SoLSA	Support of Localised Service Area

## Terms

SoLSA exclusive access	Cells on which normal camping is allowed only for MS with Localised Service Area (LSA) subscription. (GSM 03.22 sub-clause 1.2)

Localised Service Area (LSA)	A localised service area consists of a cell or a number of cells. The cells constituting an LSA may not necessarily provide contiguous coverage. (GSM 03.22 sub-clause 1.2)

# Introduction

SoLSA (Support of Localised Service Area) is a new GSM Phase 2+ feature which allows the provisioning of locally or regionally  restricted services. SoLSA provides a platform which enables Network operators to provide services tailored to the needs of  a subscriber or subscriber groups. These include

- the definition of special tariff packages (definition of Zones)

- the preferential allocation of resources e.g. in areas with high traffic intensity

- the definition of  private (virtual) mobile networks for corporate customers

This document aims at giving a very brief description of the design of SoLSA-integration into the existing G23 protocol stack.

The core of the SoLSA functionality lies in the ability of an MS to recognise and to select a cell, in which it can enjoy it’s subscribed privileges and also display certain features related to these privileges.

The following entities of the G23 protocol stack need to be modified as follows:

- MMI : to cater for the display of lsa-names and icons.

- MFW : to provide callback-functions and events for the display of lsa-names and icons in the MMI.

- ACI : to use the file identifier and record number received from RR to fetch the lsa-name and icons from the SIM and to send them to the upper layers for illustrative purposes.

- RR : to load all SoLSA information required by the cell selection/reselection processes from the SIM and to modify the cell selection/reselection processes accordingly  (i.e. to make them SoLSA-capable).

- SIM : to provide a SIM-SAP for the RR entity which is used to read Solsa data from the SIM and to provide functions for the OTA update of SoLSA information available on the SIM.

The following block-diagram illustrates the above points:

The letters in the diagram can be used to track a typical SoLSA scenario as follows:

- At initialisation, SoLSA relevant data is read through the new SAP from the SIM into RR and stored as a dynamic linked list.

- During cell selection/reselection, assuming the SoLSA flag is set, the MS makes extensive use of the solsa-linked-list to camp on a cell. It does so by comparing data ( lsa-id etc.) from system information elements on the BCCH with that in the solsa linked list. If the suitable cell is an LSA-cell, the file-identifier and record-number of it’s lsa-name/icon (i.e. on the SIM) is taken from the linked list and sent through the upper layers to the ACI.

- The Solsa interface in the ACI uses the file-identifier and record-number received from RR to load the lsa-name/icon from the SIM and sends it to the MFW.

- The MFW calls the solsa related functions to display lsa-name/icon in the MMI.

# Cell selection/reselection

## Cell selection

According to GSM 03.22, a cell must satisfy the ff. requirements before a SoLSA capable MS can perform normal camping on it:

i) It should be a cell of the selected PLMN;

ii) It should not be "barred" (see sub-clause 3.5.1 of GSM 03.22);

iii) It should not be in an LA which is in the list of "forbidden LAs for roaming";

iv) The radio path loss between MS and BTS must be below a threshold set by the PLMN operator. This is estimated

as shown in sub-clause 3.6 of GSM 03.22;

v) It should not be a SoLSA exclusive cell to which the MS does not subscribe.

Note: A SoLSA compatible MS which is not restricted by the “LSA only access” property can camp normally on any LSA-cell, to which it is has not even subscribed, with the exception of “exclusive access” cells. It can however camp in the limited service state on exclusive access cells.

There is the need therefore, for a SoLSA mobile to know that, it is in an exclusive access cell (in the case of cell selection) in order to declare it’s state as “limited-service-state”. This property of a cell is obtained from the System Information Messages(SI4, SI7, SI8, SI 16 SI 17).

Tip: After the SI messages have been read, consider functions like att_check_bcch_carrier() and insert the SoLSA conditions where the cell is checked for suitability (i.e. c1, priority, bar-status, etc).

## Cell reselection

According to GSM 05.08 clause 6.6.3, the following holds for cell reselection:

At least for every new sample or every second, whichever is the greatest, the MS calculate the value of C1, C2 and C4

for the serving cell and the non-serving cells. The MS shall make a cell reselection if:

i) The path loss criterion parameter (C1) for the serving cell falls below zero for a period of 5 seconds.

ii) A non-serving suitable cell (see GSM 03.22) is evaluated to be better than the serving cell for a period of 5

seconds. The best cell is

- the cell with the highest value of C2 + LSA_OFFSET among those cells that have highest LSA priority

among those that fulfil the criteria C4 0, or

- the cell with the highest value of C2 among all cells, if no cell fulfil the criterion C4 0.

LSA_OFFSET and LSA ID(s) are broadcast on BCCH. LSA priority is defined by the list of LSAs for the

subscriber stored on the SIM (see GSM 11.11). Cells with no LSA priority, eg non-LSA cells, are given LSA

priority lower than 0. If no LSA_OFFSET parameter is broadcast, LSA_OFFSET shall be set to 0.

When evaluating the best cell, the following hysteresis values shall be subtracted from the C2 value for the

neighbour cells:

- if the new cell is in the same location area: 0;

- if the new cell is in a different location area: CELL_RESELECT_HYSTERESIS, which is broadcast on BCCH of the serving cell.

- in case of a cell reselection occurred within the previous 15 seconds: 5 dB.

Cell reselection for any other reason (see GSM 03.22) shall take place immediately, but the cell that the MS was

camped on shall not be returned to within 5 seconds if another suitable cell can be found. If valid receive level averages are not available, the MS shall wait until these values are available and then perform the cell reselection if it is still required. The MS may accelerate the measurement procedure within the requirements in clause 6.6.1 of GSM 05.08 to minimise the cell reselection delay.

If no suitable cell is found within 10 seconds, the cell selection algorithm of GSM 03.22 shall be performed. Since

information concerning a number of channels is already known to the MS, it may assign high priority to measurements

on the strongest carriers from which it has not previously made attempts to obtain BCCH information, and omit

repeated measurements on the known ones.

The signal strength threshold criterion parameter C4 is used to determine whether prioritised LSA cell reselection shall

apply and is defined by :   C4 = A - PRIO_THR

where

A is defined as : A = RLA_C - RXLEV_ACCESS_MIN ( GSM 05.08 clause 6.4 )

and PRIO_THR is the signal threshold for applying LSA reselection. PRIO_THR is broadcast on

the BCCH. If the idle mode support is disabled for the LSA (see GSM 11.11) or if the cell does not belong to any LSA

to which the MS is subscribed or if no PRIO_THR parameter is broadcast, PRIO_THR shall be set to  (infinity).

# New and modified SAPs

## SIM_RR_INSERT_IND / SIM_RR_INSERT_RES(new)

These new primitives are part of the new SAP introduced between the SIM and RR. They serve the purpose of reading SoLSA information from the SIM into RR.

Important: This primitive pair will be called immediately after the call to the SIM_MMI_INSERT_IND primitive. After that, the SIM_SMS_INSERT_IND is called. Please note that this changes the sequence of primitive calls during the initialisation phase of the MS and therefore the SIM’s MSC document must be modified to reflect these changes.

## SIM_READ_RECORD_REQ / SIM_READ_RECORD_CNF

These new primitives are part of the new SAP introduced between the SIM and RR. They serve the purpose of reading SoLSA information from the SIM into RR.

## SIM_FILE_UPDATE_IND / SIM_FILE_UPDATE_RES

These primitives have to be extended to carry information  about new and changed LSAs after an update over the air.

## RR_ACTIVATE_CNF/ RR_ACTIVATE_IND

These primitives have to be extended to carry the file-identifier and record-number of the chosen LSA during cell selection and reselection.

## RR_SYNC_REQ

This primitive must be modified to carry the lsa-id received from the MM Information element in the dedicated mode.

## RR_SYNC_IND

This primitive must be extended to notify the upper layers of  a change in LSA.

## MMR_REG_CNF

This primitive must be extended to carry file-id and record number of lsa-name/icon.

## MMR_INFO_IND

This primitive must be extended to indicate a change of LSA in the dedicated mode.

# Message Sequence Charts

## Initialisation

MMI/SMS/ACI           MM                SIM              RR/DL

|                    |                 |                 |

|                    |                 |                 |

| SIM MMI INSERT IND                       			|

*<=====================================*

(MMI 1)               |                 |              	|

|                    |                 |                 |

|                    | 			SIM RR INSERT IND

|                    |			*================>*

|                    |                 |              (RR 1)

|                    |                 |                 |

- a) Solsa not allocated (SIM service table)

|                    |                 |                 |

|                    |                 SIM RR INSERT RES

|                    |                 *<================*

|                    |                (SIM 1) 		|

|                    |                 |                 |

|                    |                 |                 |

- b) Solsa allocated (SIM service table)

|                    |                 |                 |

|                    |                 SIM READ RECORD REQ

|                    |			*<================*

|                                     (SIM 2)            |

|                    |                 |                 |

|                    |                 |                 |

|                    |            	SIM READ RECORD CNF

|                    |			*================>*

|                    |                 |              (RR 2)

|                    |                 |                 |

|                    |                 |                 |

|		    |			SIM RR INSERT RES

|		    |			*<================*

|		    |		    (SIM 3)			|

|		    |			|			|

|                    |                 |                 |

| SIM SMS INSERT IND                       			|

*<=====================================*

(SMS 1)               |                 |              	|

|                    |                 |                 |

|                    |                 |                 |

(MMI 1)

During initialisation, the mobile goes through the usual sequence until the SIM_MMI_INSERT_IND primitive has been processed. This is followed immediately by the call to primitives responsible for loading SoLSA information from the SIM. Note that the sequence of the primitive calls has been changed to support SoLSA.

(RR 1)

The SIM_RR_INSERT_IND is used to inform the RR-entity about the presence of the SoLSA feature(i.e. whether SoLSA is supported or not) and whether the subscriber is restricted to only LSA cells or not. For this purpose, the SIM service table and the Elementary File Efsai on the SIM are read respectively.

(SIM 1)

If the SIM service table indicates that service 40(SoLSA functionality) is not available, SIM_RR_INSERT_RES returns an appropriate cause value(e.g. SolSA_NOT_ACTIVE).

(SIM 2)

If the SIM service table indicates that service 40(SoLSA functionality) is available, a request is made for the various SoLSA files on the SIM. The data requested by RR is used to build a dynamic linked list of LSA parameters. These are taken from the Efsll which consist of the configuration parameters, Priority, PLMN code, and the contents of the LSA descriptor files i.e. is the LSA-Ids. To save memory and to be consistent in the implementation, only the LSA-Ids are extracted from the descriptor files. Other contents of the descriptor files e.g. LAC+CI, CI and LAC are neglected completely. The LSA-name and Icon-identifier are not read during initialisation. These are read only when they are to be illustrated. This is done by the SoLSA module in the ACI.

Note: The file-id of the EFsll and the record-number of the respective LSAs are also stored in the linked list. These serve the purpose of backtracking an LSA-name or Icon-identifier on the SIM which has to be displayed after the Mobile has camped on a suitable and authorised LSA-cell.

(RR 2)

The SIM delivers the requested data to RR.

(SIM 3)

Depending on the outcome, RR signals a successful receipt of data or a failure.

(SMS 1)

The SIM_SMS_INSERT_IND is called after the SIM_RR_INSERT_IND.

## Cell selection

MMI/ACI               MM                SIM                RR

|                    |                 |                 |

|                    |                 |                 |

|    MMR REG REQ     |                 |                 |

*===================>*                 |                 |

|                 (MM 1)               |                 |

|                    |         RR ACTIVATE REQ           |

|                    *==================================>*

|                    |                 |               (RR 1)

|                    |                 |                 |

Successful cell selection         |         		            |                                   |

|                    |                 |                 |

|                    |                 |                 |

|                    |          RR ACTIVATE CNF          |

|                    *<==================================*

|                  (MM 2)              |                 |

|    MMR REG CNF     |                 |                 |

*<===================*                 |                 |

(MMI 1)                |                 |                 |

|                    |                 |                 |

|                    |                 |                 |

(MM 1)

MMI starts the cell search. No SoLSA information is sent along this path.

(RR 1)

The BCCH search for a network is started. Parameter of the SIM card like the IMSI are forwarded to RR. No SoLSA information is sent along this path.

Note: If the RR_ACTIVATE_REQ primitive is received at initialisation, and SoLSA data is still been read from the SIM, the primitive must be stored until all SoLSA information is read completely.

(MM 2)

RR signals a successful end of the cell search. If the cell is an LSA-cell, the file identifier and record index of it’s lsa-name/icon are forwarded to MM.

(MMI 1)

The file identifier and record index of the chosen LSA-cell is forwarded together with other data to the MMI/ACI.

## Successful cell reselection

MMI                  MM                SIM              RR/DL

|                    |                 |                 |

a)in idle mode

|                    |                 |                 |

|                    |          RR ACTIVATE IND          |

|                    *<==================================*

|                  (MM 1)              |                 |

|                    |                 |                 |

|                    |                 |                 |

b)from dedicated to idle mode

|                    |                 |                 |

|                    |          RR SYNC IND              |

|                    *<==================================*

|                  (MM 2)              |                 |

|                    |                 |                 |

|    MMR REG CNF     |                 |                 |

*<===================*                 |                 |

(MMI 1)                |                 |                 |

|                    |                 |                 |

|                    |                 |                 |

(MM 1)

After cell reselection in the idle mode, RR signals a successful cell reselection and forwards all information with the RR_ACTIVATE_IND primitive to MM.

(MM 2)

After cell reselection on return from the dedicated mode, RR forwards all information to MM with the RR_SYNC_IND primitive. The “BA List Pref” Information Element which is transmitted in the Channel Release message must be decoded if available. This is very important for SoLSA exclusive cells.

(MMI 1)

All LSA related information is forwarded with the MMR_REG_CNF primitive to the MMI.

## Change of LSA ID in the dedicated mode

MMI/ACI               MM                SIM              RR/DL

|                    |                 |                 |

|                    |            RR_DATA_IND            |

|                    |         (MM_INFORMATION)          |

|                    *<==================================*

| MMR_INFO_IND       |                 |                 |

*<===================*                 |                 |

|                 (MM 1)               |                 |

|                    |                 |                 |

a)LSA ID included in MM INFORMATION

|                    |                 |                 |

|                    |            RR SYNC REQ            |

|                    *==================================>*

|                    |                                (RR 1)

|                    |                 |                 |

|                    |            RR SYNC IND            |

|                    *<==================================*

|                  (MM 2)              |                 |

|    MMR REG CNF     |                 |                 |

*<===================*                 |                 |

(MMI 1)                |                 |                 |

|                    |                 |                 |

The network decides when to notify the MS about a change of current LSA (after a handover) in the dedicated mode. The information will be sent from the MSC to the MS and will contain the LSA ID, This is done by adding the LSA ID of the current (new) cell to the MM INFORMATION Message. If no LSA ID is included in the MM Information message the MS shall assume that the current cell does not belong to any of the allowed LSAs for the subscriber. The indication towards the user is optional and can be heard as e.g. a beep in the receiver or by displaying the stored LSA name that corresponds to the received LSA ID (GSM 03.73 clause 11.8.2).

(MM 1)

The network transmits the MM INFORMATION which may include the LSA ID. This is done so long as an RR connection with the network exists. The MMR_INFO_IND primitive carries the contents of the MM INFORMATION message to the MMI/ACI. Note that the LSA ID is not transferred to the MMI/ACI. In both cases of the inclusion and exclusion of the LSA ID in the MM INFORMATION message, only an appropriate flag is sent with the MMR_INFO_IND to the MMI/ACI to erase any displayed lsa-name or icon. This is so because an LSA ID cannot be used directly to trace an lsa-name/icon. It must first be sent to RR to find it’s corresponding file identity and record number.

(RR 1)

If an LSA ID is included in the MM INFORMATION, this is transferred to RR with the RR_SYNC_REQ primitive

(MM 2)

After the corresponding file identity and record number of the LSA ID have been extracted from the LSA linked list, they are sent to MM with the RR_SYNC_IND primitive.

(MMI 1)

The MMR_REG_CNF is used to inform the MMI/ACI of the current lsa-name/icon that must be displayed.

## Update of LSA data OTA (SIM REFRESH Command)

Data stored on the SIM  can be changed or extended through SIM application activity. This could also take place beyond the control of the MS when it is, for instance in idle or dedicated mode. Two interfaces must be considered as a result of this activity. The SIM-Network interface and the SIM-MS interface.

The SIM-Network interface: The SIM application functions responsible for updating data on the SIM, must be modified to consider SoLSA information. This is not treated in this document at the moment.

The SIM-MS interface: The SIM REFRESH Command enables the MS to be notified of the changes to the SIM configuration that have occurred as a result of  a SIM application activity. The modes of this command, that affect SoLSA-data loaded into the MS, are described below.

### Mode: SIM Initialisation

MMI/ACI               MM                SIM              RR/DL

|                    |                 |                 |

|                    |                 |                 |

|                    |                 |                 |

|                    | 			SIM RR INSERT IND

|                    |			*================>*

|                    |                 |              (RR 1)

|                    |                 |                 |

- a)SoLSA not allocated (SIM service table)

|                    |                 |                 |

|                    |                 SIM RR INSERT RES

|                    |                 *<================*

|                    |                (SIM 1) 		|

|                    |                 |                 |

|                    |                 |                 |

- b)SoLSA allocated (SIM service table)

|                    |                 |                 |

|                    |                 SIM READ RECORD REQ

|                    |			*<================*

|                                     (SIM 2)            |

|                    |                 |                 |

|                    |                 |                 |

|                    |            	SIM READ RECORD CNF

|                    |			*================>*

|                    |                 |              (RR 2)

|                    |                 |                 |

|                    |                 |                 |

|		    |			SIM RR INSERT RES

|		    |			*<================*

|		    |		     (SIM 3)		|

|		    |			|			|

The SIM Initialisation mode commands the MS to carry out SIM initialisation as defined in GSM 11.11 sub-clause 11.6.1 starting only after the CHV1 verification procedure. The MS shall not reset the SIM electrically.

(RR 1)

At initialisation, the SIM provides RR with required information about certain SoLSA related fields of the SIM card. This is to verify whether SoLSA features are available or not.

(SIM 1)

If the SIM service table indicates that service 40(SoLSA functionality) is neither activated nor allocated, SIM_RR_INSERT_RES returns an appropriate cause value(e.g. SolSA_NOT_ACTIVE).

(SIM 2)

If the SIM service table indicates that service 40 is allocated and activated, a request is made for the various SoLSA files on the SIM.

(RR 2)

The SIM delivers the requested SoLSA data to RR.

(SIM 3)

Depending on the outcome, RR signals a successful receipt of data or a failure.

### Mode: File Change Notification

MMI/ACI               MM                SIM               RR

|                    |                 |                 |

|                    | 			SIM FILE UPDATE IND

|                    |			*================>*

|                    |                 |              (RR 1)

|                    |                 |                 |

|                    |                 |                 |

|                    |                 SIM READ RECORD REQ

|                    |			*<================*

|                    |                (SIM 1)            |

|                    |                 |                 |

|                    |                 |                 |

|                    |            	SIM READ RECORD CNF

|                    |			*================>*

|                    |                 |              (RR 2)

|                    |                 |                 |

|                    |                 |                 |

|                    |                 SIM FILE UPDATE RES

|                    |                 *<================*

|                    |                (SIM 2) 		|

|                    |                 |			|

|                    |                 |                 |

|                    |     RR ACTIVATE IND/RR SYNC IND   |

|                    *<==================================*

|                  (MM 1)              |                 |

|    MMR REG CNF     |                 |                 |

*<===================*                 |                 |

(MMI 1)                |                 |                 |

|                    |                 |                 |

|                    |                 |                 |

The File Change Notification mode provides the MS with identities of the EFs (Elementary Files) that have been changed (in structure and/or content) in the SIM. This information is used by the MS to update the affected EFs whose contents form part of the LSA-linked-list in the MS.

(RR 1)

A list of LSA elementary files that have been updated beyond the control of the MS are delivered to RR for the update of the LSA linked list.

(SIM 1)

RR makes a request for all SoLSA files that must be modified.

(RR 2)

The SIM delivers the requested data to RR.

(SIM 2)

The SIM_FILE_UPDATE_RES is used by the RR entity to acknowledge the successful/unsuccessful update of the contents of the files indicated with the SIM_FILE_UPDATE_IND primitive.

(MM 1)

If the update of the LSA linked list triggers a cell reselection or any change that requires the notification of the upper layers, the RR_ACTIVATE_IND or RR_SYNC_IND primitives are used for this purpose. The following conditions could trigger a cell reselection:

- if a non-serving suitable cell according to GSM 05.08 sub-clause 6.6.3 is found to be better. For an LSA cell, the following conditions must hold: C4>=0, highest LSA-priority and highest value of C2 + LSA_OFFSET.

- if the subscriber attains/maintains an “LSA Only Acesss” status and the serving cell ceases to be part of an authorised LSA.

(MMI 1)

The MMR_REG_CNF primitives conveys any changed data due to update of the LSA list from MM to MMI/ACI.

# LSA Properties

## SoLSA feature flag

Description:

The SoLSA Flag indicates if the SoLSA feature should be considered or not.

Definition:

| short name | Comment |
| --- | --- |
| FF_SOLSA | SoLSA feature flag |

## Constants

Definition:

| name | value | comment |
| --- | --- | --- |
| MAX_LSA | 3 | maximum number of LSA IDs per cell |

## Mobile Station Classmark 2

Description:

The parameter defines the mobile station classmark 2 information.

Bit-4 of octet-5 in the classmark IE is used to indicate SoLSA support.

Definition:

| type | short name | Comment |
| --- | --- | --- |
| STRUCT | classmark_2 | Classmark 2 parameters |

Elements:

| long name | short name | type |
| --- | --- | --- |
| Revision level | rev_lev | UBYTE |
| Early classmark sending | es_ind | UBYTE |
| A5/1 | a5_1 | UBYTE |
| RF power capability | rf_pow_cap | UBYTE |
| Pseudo synchronized handover capability | ps_cap | UBYTE |
| SS screening indicator | ss_screen | UBYTE |
| Short message capability | sm_cap | UBYTE |
| VBS Notification Reception | vbs_not_rec | UBYTE |
| VGCS Notification Reception | vgcs_not_rec | UBYTE |
| Frequency capability | freq_cap | UBYTE |
| Additional classmark 3 information | class_3 | UBYTE |
| SoLSA Supported | solsa_supp | UBYTE |
| CMSP Supported | cmsp_supp | UBYTE |
| A5/3 | a5_3 | UBYTE |
| A5/2 | a5_2 | UBYTE |

SoLSA values:

| name | value | c-macro | comment |
| --- | --- | --- | --- |
| solsa_supp | 0 | SOLSA_SUPP_NO | SoLSA not supported |
|  | 1 | SOLSA_SUPP_YES | SoLSA supported |

## LSA-ONLY-ACCESS flag

Description:

The LSA-ONLY-ACCESS Flag indicates whether an MS (or more precisely a subscriber) is restricted to only LSA cells or not.

Definition:

| type | short name | Comment |
| --- | --- | --- |
| UBYTE | lsa_only_acc | LSA only access indicator |

Values:

| Name | value | c-macro | Comment |
| --- | --- | --- | --- |
| lsa_only_acc | 0 | LSA_ONLY_ACCESS_NO | LSA only access is not active |
|  | 1 | LSA_ONLY_ACCESS_YES | LSA only access is active |

## LSA ID Information

Description:

The LSA ID Information is part of SI 4, SI 7, SI 8, SI 16, and SI 17 rest octets.

Reference : GSM 04.08 section 10.5.2.35

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| LSA ID Information | lsa_id_info | 25-76 |

Elements:

| long name | short name | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| LSA ID | lsa_id | 24 | M |  |  |
| Additional LSA IDs | lsa_id_add | 24 | M | CSN1_S1 | [0..MAX_LSA_ID] |

### LSA ID

Description:

The parameter is used to identify the LSA.
Reference : GSM 04.08 section 10.5.2.35 (SI4 rest octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| LSA ID | lsa_id | 24 |
| Additional LSA IDs | lsa_id_add | 24 |

## LSA Element

Description:

LSA element is part of the elementary file EFsll (SoLSA LSA List) defined on the SIM card. These properties make up a significant part of the LSA linked list on the RR sub-layer. 
Reference : GSM 11.11 section 10.4.1.2.

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| LSA properties | lsa_element |  | 48 |

Elements:

| long name | short name | bit len | pres |
| --- | --- | --- | --- |
| Configuration parameters | config_param | 8 | M |
| Icon Identifier | icon_id | 8 | M |
| Priority | priority | 8 | M |
| PLMN code | lsa_plmn | 24 | M |
| LSA ID | lsa_id | 24 | M |

### Configuration parameters

Description:

This parameter contains fields which are used to determine whether the mobile shall favor camping on lsa-cells and how the lsa-name/icon should be displayed if present.
Reference : GSM 11.11 section 10.4.1.2.

### Icon identifier

Description:

This parameter contains the record number of an icon in the elementary file EFimg on the SIM card. 
Reference : GSM 11.11 section 10.4.1.2.

### Priority

Description:

This parameter contains a priority value which gives an LSA preference over others.
Reference : GSM 11.11 section 10.4.1.2.

### PLMN code

Description:

This parameter specifies the PLMN to which LSA belongs. 
Reference : GSM 11.11 section 10.4.1.2.

### LSA ID

Description:

This parameter specifies the identity of an LSA. 
Reference : GSM 11.11 section 10.4.1.2.

## LSA Parameters (cell selection/reselection)

Description:

The LSA ID Information is part of SI 4, SI 7, SI 8, SI 16, and SI 17 rest octets.

Reference : GSM 04.08 section 10.5.2.35, section 10.5.2.37e

Definition:

| long name | short name | type | bit len |
| --- | --- | --- | --- |
| LSA parameters | lsa_param |  | 20-32 |

Elements:

| long name | short name | bit len | pres | type | ctrl |
| --- | --- | --- | --- | --- | --- |
| Priority Threshold | Lsa_prio_thr | 3 | M |  |  |
| LSA Offset | lsa_offset | 3 | M |  |  |
| Optional PLMN | opt_plmn | 24 | O | CSN1_S1 |  |

### LSA Offset

Description:

The parameter defines an offset for LSA reselection between cells with same LSA priorities.
Reference : GSM 04.08 section 10.5.2.35 (SI4 rest octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| LSA Offset | lsa_offset | 3 |

### Priority Threshold

Description:

The parameter is used to determine whether the mobile shall apply prioritized cell re-selection.
Reference : GSM 04.08 section 10.5.2.35 (SI4 Rest Octets), section 10.5.2.37e (SI16 Rest Octets)

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Priority Threshold | lsa_prio_thr | 3 |

### Optional PLMN Code

Description:

The optional PLMN code is part of the LSA parameters in SI4 rest octets. It is used as a substitute PLMN,  if an “Escape- PLMN” is broadcast in the LAI IE for “SoLSA- exclusive access“ cells.

Reference : GSM 04.08 section 10.5.2.35

Definition:

| long name | short name | bit len |
| --- | --- | --- |
| Optional PLMN | opt_plmn | 24 |

Elements:

| long name | short name | bit len | pres | ctrl |
| --- | --- | --- | --- | --- |
| Mobile Country Code | mcc | 12 | M | BCD_NOFILL[3] |
| Mobile Network Code | mnc | 12 | M | BCD_MNC[2..3] |

Algorithms:

Cell selection

Case : normal camping

In addition to existing general requirements, a potential serving cell must satisfy the ff. condition for a SoLSA mobile:

It should not be a SoLSA exclusive cell to which an MS does not subscribe.

In a SoLSA exclusive cell, the MCC + MNC code of the LAI Information Element is replaced by a unique escape PLMN code (see GSM 03.73; GSM 03.22) not assigned to any PLMN in SI3 and SI4. The real PLMN code is still broadcast, but can only be found in the LSA Parameters of the SI4 Rest Octets.

If the escape PLMN is broadcast then a permission to the MS to perform “early classmark sending” procedure is also broadcast.