SIMLOCK HOW TO

| Department: | WTBU - Cellular Systems |  |  |
| --- | --- | --- | --- |
| Creation Date: | 2005-11-02 |  |  |
| Last Modified: | 2005-11-02 by Durgesh |  |  |
| ID: | SIML_Howto | Version: | 0.1 |
| Status: | Draft | ECCN: |  |

- Document Control

 2005 Texas Instruments Incorporated. All rights reserved.

Texas Instruments Incorporated and / or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and / or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and / or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and / or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and / or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and / or the licensing of software do not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of TI.

  - Export Control Statement

Recipient agrees that it will not knowingly export or re-export, directly or indirectly, any product or technical data (as defined by the U.S, EU and other Export Administration Regulations) including software, or any controlled product restricted by other applicable national regulations, received from Disclosing party under this Agreement, or any direct product of such technology, to any destination to which such export or re-export is restricted or prohibited by U.S or other applicable laws, without obtaining prior authorisation from U.S. Department of Commerce and other competent Government authorities to the extent required by those laws. This provision shall survive termination or expiration of this Agreement.

According to our best knowledge of the state and end-use of this product or technology, and in compliance with the export control regulations of dual-use goods in force in the origin and exporting countries, this technology is classified as given on the front page.

This product or technology may require export or re-export license for shipping it in compliance with certain countries regulations.

  - Document History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2005-Nov-02 | Durgesh S Alageri |  | 0.1 | Draft | 1 |

Notes:

- Initial version.

  - References, Abbreviations, Terms

| 1. | Personalization_LLD.doc |
| --- | --- |
| 2. | Personalization_HLD.doc |
| 3. | 3GPP TS 22.022 Personalization of ME |

Table of Contents

1	How to Create MEPD Raw Data	5

2	How to generate MEPD Bin file	5

3	How to Store MEPD in ME	5

3.1	Non Secured	5

3.2	Secured (For Locosto)	6

4	Details of Internal MEPD structure	6

4.1	Data sections	6

4.2	Global configuration	7

4.3	Failure count key	7

4.4	MEPD Configuration	7

4.4.1	Flags	8

4.4.2	AddNewIMSI	10

4.4.3	NumCategories	10

4.4.4	FC_Max	11

4.4.5	FC_Current	11

4.4.6	FC_Key_Length	11

4.4.7	FC__Reset_Fail_Max	11

4.4.8	FC_Reset_Fail_Current	12

4.4.9	FC__Reset_Success_Max	12

4.4.10	FC_Reset_Success_Current	12

4.4.11	MNC_Len	12

4.4.12	GID1_Len	12

4.4.13	GID2_Len	13

4.4.14	TypeAprovalSIM	13

4.4.15	TestCPHS	13

4.5	Category header	13

4.5.1	Status	14

4.5.2	Flags	14

4.5.3	Dependency	14

4.5.4	DataLen	15

4.5.5	KeyLength	15

4.6	Category Body	15

4.6.1	Network category:	16

4.6.2	Network Subset category:	18

4.6.3	SIM Code category:	22

4.6.4	Service Provider Category:	23

4.6.5	Corporate Category:	25

4.7	CTS-MEPD cross table	27

5	External MEPD Format	29

5.1	Data sections	29

Appendices	33

# How to Create MEPD Raw Data

Fill the data field in a text file and save it as <MEPD_File>.properties file. The details of the MEPD data fields to be entered are explained at length in the section 4. A sample of MEPD_File.properties (All the categories are disabled) file is attached below.

Note: This MEPD_File is created with all the personalization category disabled. It is for ease of team members don’t want to bother about personalization feature and want the mobile to boot up without any hassles of entering passwords etc.

# How to generate MEPD Bin file

It requires GenMEPD tool. The following steps are done to generate MEPD bin file

- Create the MEPD.properties file. (mentioned in section 1)

- Run the following commands in mepd_tool directory  to convert the MEPD_File.properties file to MEPD_File.bin

Java GenMEPD <MEPD_File>.properites MEPD_File.bin

A sample generated MEPD_File.bin is attached below (for the above <MEPD_File>.properties file):

# How to Store MEPD in ME

There are two ways of storing MEPD Data to mobile depending on usage of non secure or secured release of security driver. For non secure release of security driver MEPD Data can be written directly in the FFS. For secured release of security driver MEPD Data can be bind to the target using CSST tool.

## Non Secured

It require ETM tool. The following steps are done to write MEPD data to FFS.

- Copy  generated MEPD_File.bin to etm directory

- Run the command tmsh in etm directory.

- If the target build traces is through USB, open the tmsh shell by running the command tmsh –pn in etm directory where ‘n’ is the port number. ( E.g. : If USB is connected to COM4 then run tmsh –p4 in etm directory)

- If FFS is not formatted then board will not boot up. To format FFS  do the following steps

- Remove the sim card from the board

- Restart the board

- Wait until MMI asks Insert Sim

- Connect USB/ UART

- Do the step 4 or 5 depending on USB / UART

- Run the command mkfs –f   in tmsh shell

- Write the   MEPD_File.bin  to FFS by running following command in tmsh shell

fwr MEPD_File.bin  MEPD_File.bin

## Secured (For Locosto Release)

It requires CSST I-Sample Release 1.001 tool.

In order to bind an MEPD file to target, rename MEPD_File.bin to MEPD_File.raw. Start the CSST tool by choosing “Start All Programs  Texas Instruments  CSST I-Sample 1.001  CSST GUI” Then choose BOOT ROM in the toolbar, double click on the Download task in the left pane, and press the Bind tab. Specify the binary MEPD file and the start address and tick the “Enable encryption” checkbox. Note that the .raw extension must be used for binary files, as .bin is used for WinCE binaries in CSST. The start address is 0x066F0000 (i.e., the last flash sector before the FFS).

CSST I-Sample Release 1.001 can also be used for binding IMEI to Locosto GP devices even if the CSST GUI has not been updated yet. A hex editor is needed in which a binary file (.raw) can be created. If the IMEI is 1122334455667788 then the contents of the binary file must be 0x44 0x33 0x22 0x11 0x88 0x77 0x66 0x55. Finally, specify the newly created binary file and 0x066EFF00 as start address for the platform certificate and disable encryption on the Bind tab in CSST.

A sample of imei.raw file is attached below

# Details of Internal MEPD structure

This section describes the structure used internally by the security driver for the MEPD data. For the initial (unsecured) release of the driver, MEPD test data in this format can be written directly in the FFS. Once the secure version is available, the MEPD data will be stored in encrypted form outside the FFS.

## Data sections

The internal data is structured as a series of sections, each aligned to the nearest 8 byte boundary. The reason for this alignment is the nature of the used encryption algorithm.

The following table depicts the data sections as they come in the file, where “n” represents the total number of categories stored in the structure.

Internal sections

Global configuration

Failure count key

MEPD configuration

Category 0 (NW )

Category 1 (NS)

Category 2 (SP)

Category 3 (CP)

Category 4 (SIM)

Category 5 (AP)

Category X (n-6 times)

Each category is in turn divided in 3 subsections, each aligned to the nearest 8 byte boundary.

Category section

Category key

Category header

Category body

## Global configuration

This section is used internally by the security driver only. Its main use is to distinguish between 2 different formats; 1) The format coming from the operator that needs to be converted into the format described here. 2) The format as described here.

If this section is not initialized correctly, all API functions will return SEC_DRV_RETURN_NotPresent.

| Global configuration |  |  |
| --- | --- | --- |
| Type | Name | Value |
| UINT8 | firstboot_pattern | 0xAA |

## Failure count key

Each key (that is; not only this one) is structured as a C-string ending with a ‘\0’ termination, unless the length is at maximum in which case there is no room for the termination character. The maximum length is 16 chars.

| Key |  |  |
| --- | --- | --- |
| Type | Name | Value |
| char[16] | digit | String used as key. Should contain 16 valid chars at all times, as this is what first boot will calculate! |

## MEPD Configuration

The following table shows the layout of the MEPD configuration section:

| Offset (bytes) | Field | Size (bytes) | Description | Ref |
| --- | --- | --- | --- | --- |
| 0x0000 | Flags | 2 | Keeps flags for customer specific behavior | TR8.3, 5.2, 5.3, 8.1, 6.7, 6.8 |
| 0x0002 | AddNewIMSI | 2 | Describes the Personalization process | ETSI standard req, TR 5.6 |
| 0x0004 | NumCategories | 1 | Contains number of categories, stored in MEPD | TR1.1 |
| 0x0005 | FC_Max | 1 | Failure Counter maximum. 0xFF to disable the counter | TR10.1, 6.4, 6.5, 6.6 |
| 0x0006 | FC_Current | 1 | Current value of the Failure Counter | TR10.1, 6.4, 6.5, 6.6 |
| 0x0007 | FC_Key_Length | 1 | Length of generated FC Key | TR8.4 |
| 0x0008 | FC_Reset_Fail_Max1 | 1 | Maximum number of allowed failed FC resets. 0xFF to disable the counter. | TR10.1, 6.4, 6.5, 6.6 |
| 0x0009 | FC_Reset_Fail_Current1 | 1 | Current number of failed FC resets | TR10.1, 6.4, 6.5, 6.6 |
| 0x000a | FC_Reset_Success_Max1 | 1 | Maximum number of allowed successful FC resets. 0xFF to disable the counter | TR10.1, 6.4, 6.5, 6.6 |
| 0x000b | FC_Reset_Success_Current | 1 | Current number of successful FC resets | TR10.1, 6.4, 6.5, 6.6 |
| 0x000c | MNC_Len | 1 | Length of the MNC | TR11.2 |
| 0x000d | GID1_Len | 1 | Length of the GID1 | TR11.3 (was later reduced to 4) |
| 0x000e | GID2_Len | 1 | Length of the GID2 | TR11.3 (was later reduced to 4) |
| 0x000f | TypeAprovalSIM | 1 | Describes the behavior of the test SIM according to TR 4.1 | TR 4.1 |
| 0x0010 | TestCPHS | 1 | Describes the behavior of the test SIM according to TR 4.2 | TR 4.2 |

### Flags

| Name: Flags |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bit Nr | Name | Description | Values | Ref |
| 0 | Reserved |  |  |  |
| 1 | Reserved |  |  |  |
| 2 | SEC_DRV_HDR_FLAG_Truncation | Used to specify if Control Key shall be truncated during all SML control keys verification | 1: truncate key to 8 b 0: use full length | TR8.3 |
| 3 | SEC_DRV_HDR_FLAG_ETSI_Flag | Used to specify Lock behaviour (ETSI or Customer Specific) | 1: ETSI 0: Customer | TR5.2, 5.3 |
| 4 | SEC_DRV_HDR_FLAG_Spec_Lock_Key | Used to specify if specific lock key used | 1: Spec. key used 0: Spec. Key not used | TR8.1 |
| 5 | SEC_DRV_HDR_FLAG_LAM_Unlock | Used to specify LAM unlock (all unlock passwords are considered as wrong) | 1: LAM unlock active 0: LAM unlock off | TR6.7 |
| 6 | SEC_DRV_HDR_FLAG_No_SIM_Unlock | Used to set if ME de-personalization shall be allowed without SIM inserted | 1: If no SIM, no unlock possible 0: If no SIM, unlock possible | TR6.8 |
| 7 | SEC_DRV_HDR_FLAG_Airtel_Ind | Is used to indicate to MMI that Airtel behavior is active. This Flag is newer processed by ACI. Sub-flags as SEC_DRV_HDR_FLAG_No_SIM_Unlock or SEC_DRV_HDR_FLAG_Spec_Lock_Key always have first priority | 1: Airtel behavior 0: standard behavior | TR8.1, 6.8 |
| 8 | SEC_DRV_HDR_FLAG_Unlock_Timer | To inform MMI that there should be timer between Unlocking attempts | 1: Timer shall be used 0: No timer shall be used | TR10.3 |
| Examples |  |  |  |  |
| 0x08 – Truncation Off, ETSI behaviour, No Specific Lock Keys, LAM unlock Off Correct: 0x14 - Truncation On, Customer behaviour, Specific Lock Keys, LAM unlock Off, Airtel Off. Sub-flags for Specific lock or No SIM Unlock can be used separately 0x94 - Truncation On, Customer behaviour, Specific Lock Keys, LAM unlock Off, Airtel On. Sub-flags for Specific lock or No SIM Unlock can be used separately Incorrect 0x80 - Truncation Off, Customer behaviour, No Specific Lock Keys, LAM unlock Off, Airtel On. Airtel flag cannot be used separately from sub flags |  |  |  |  |

### AddNewIMSI

This field is used to specify if new code group shall be added to MEPD while doing ME Personalization process. Thus if we try to apply anti-theft protection (SIM Personalization) to ME, we need to add IMSI of currently inserted SIM into MEPD. Optionally customer can prevent adding new code group to MEPD. Thus, if bit 4 is set to 1, new IMSI will be added to MEPD

| Name: AddNewIMSI |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bit Nr | Name |  | Description | Values |
| 0 | ADD_NW_CODE |  | While locking ME add NW new code group to MEPD | 1: Add new code group |
|  |  |  |  | 0: Don’t add new code group |
| 1 | ADD_NS_CODE |  | While locking ME add NS new code group to MEPD | 1: Add new code group |
|  |  |  |  | 0: Don’t add new code group |
| 2 | ADD_SP_CODE |  | While locking ME add SP new code group to MEPD | 1: Add new code group |
|  |  |  |  | 0: Don’t add new code group |
| 3 | ADD_CP_CODE |  | While locking ME add CP new code group to MEPD | 1: Add new code group |
|  |  |  |  | 0: Don’t add new code group |
| 4 | ADD_SIM_CODE |  | While locking ME add SIM new code group to MEPD | 1: Add new code group |
|  |  |  |  | 0: Don’t add new code group |
| 5 to 15 |  |  | Describes the Personalization process behavior for proprietary categories | 1: Add new code group |
|  |  |  |  | 0: Don’t add new code group |
| Examples |  |  |  |  |
| 0x0000 |  | User can not add code groups of currently inserted SIM into MEPD |  |  |
| 0x1f |  | User can add code groups of currently inserted SIM into MEPD for all standard categories |  |  |
| 0x10 |  | User can add code group of currently inserted SIM into MEPD only for SIM personalization (most useful) |  |  |

### NumCategories

| Name: NumCategories |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | NumCategories |  | contains number of categories, stored in MEPD | For current implementation shall be always set to 5 |
| Examples |  |  |  |  |
| 0x05 |  | keeps 5 categories |  |  |

### FC_Max

| Name: FC_Max |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | FC_Max |  | Contains maximum value of failure counter | 0-0xff |
| Examples |  |  |  |  |
| 0x03 |  | User can try maximum of 3 attempts to enter correct key |  |  |

### FC_Current

| Name: FC_Current |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | FC_Current |  | Contains current value of failure counter | Shall be initially set to 0 |
| Examples |  |  |  |  |
| 0x02 |  | User entered the wrong key in 2 attempts |  |  |

### FC_Key_Length

| Name: FC_Key_Length |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | - FC_Key_Length |  | Specifies length of FC ke, that shall be generated | 6-16 |
| Examples |  |  |  |  |
| 0x08 |  | Length of generated FC key shall be 8 digits |  |  |

### FC__Reset_Fail_Max

| Name: FC_Reset_Fail_Max |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | FC_Reset_Fail_Max |  | Maximum number of allowed failed FC resets. 0xFF to disable the counter. | 0-0xff |
| Examples |  |  |  |  |
| 0x03 |  | User can try maximum of 3 attempts to reset FC with wrong key. |  |  |

### FC_Reset_Fail_Current

| Name: FC_Reset_Fail_Current |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | FC_Reset_Fail_Current |  | Current number of failed FC resets | Shall be initially set to 0 |
| Examples |  |  |  |  |
| 0x02 |  | User entered the wrong key in 2 attempts |  |  |

### FC__Reset_Success_Max

| Name: FC_Reset_Success_Max |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | FC_Reset_Success_Max |  | Maximum number of allowed successful FC resets. 0xFF to disable the counter | 0-0xff |
| Examples |  |  |  |  |
| 0x03 |  | User can try maximum of 3 attempts to reset FC with wrong key. |  |  |

### FC_Reset_Success_Current

| Name: FC_Reset_Success_Current |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | FC_Reset_Success_Current |  | Current number of successful FC resets | Shall be initially set to 0 |
| Examples |  |  |  |  |
| 0x02 |  | User entered the wrong key in 2 attempts |  |  |

### MNC_Len

| Name: MNC_Len |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | MNC_Len |  | Contains length of the MNC |  |
| Examples |  |  |  |  |
| 0x02 |  | MNC code has 2 digits |  |  |
| 0x03 |  | MNC code has 3 digits |  |  |

### GID1_Len

| Name: GID1_Len |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | GID1_Len |  | Contains length of the GID1 file, which shall be read from SIM |  |
| Examples |  |  |  |  |
| 0x04 |  | Length of the GID1 is 4 bytes |  |  |

### GID2_Len

| Name: GID2_Len |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | GID2_Len |  | Contains length of the GID2 file, which shall be read from SIM |  |
| Examples |  |  |  |  |
| 0x04 |  | Length of the GID2 is 4 bytes |  |  |

### TypeAprovalSIM

| Name: TypeAprovalSIM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | TypeAprovalSIM |  | Describes the behavior of the test SIM according to TR 4.1 | 0 - always accepted |
|  |  |  |  | 1 - always rejected |
|  |  |  |  | 2 - accepted until normal SIM is inserted |
| Examples |  |  |  |  |
| 0x00 |  | Test SIM is always accepted and no check is done |  |  |
| 0x01 |  | Test SIM is always rejected - if any lock category is active, Test SIM will be rejected |  |  |
| 0x02 |  | Accepted until normal SIM is inserted - as soon as normal SIM is inserted. |  |  |

### TestCPHS

| Name: TestCPHS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | TestCPHS |  | Describes the behavior of the test SIM according to TR 4.2 | 0 - always accepted |
|  |  |  |  | 1 - always rejected |
| Examples |  |  |  |  |
| 0x00 |  | Test SIM is Always accepted: there is no SIM-ME-LOCK check. The ME is never blocked on a category. |  |  |
| 0x01 |  | Test SIM is Always rejected: a SIM-ME-LOCK check is always performed. The ME goes in normal mode only if all personalized categories are unblocked |  |  |

## Category header

Category Header is represented by yellow part of MEPD Record in Fig. 2-2 of HLD Security Driver.

| Offset | Field | Size | Description | Ref |
| --- | --- | --- | --- | --- |
| 0x0000 | Status | 1 | Status defines for the different lock-categories | TR14.5, 5.x, 6.x |
| 0x0001 | Flags | 1 | Different flags for the lock-categories | TR6.2 |
| 0x0002 | Dependency | 2 | Each bit represents a category that is a child of this one. Children locks/unlocks with their parents. | TR6.2 |
| 0x0004 | DataLen | 2 | Number of bytes present in the category body. |  |
| 0x0006 | KeyLength | 1 | Length of generated control key | TR8.4 |

### Status

| Name: Status |  |  |  |  |
| --- | --- | --- | --- | --- |
| Size | Name |  | Description | Values |
| 1 | Status |  | Status defines for the different lock-categories | 0 - SEC_DRV_CAT_STAT_Unlocked |
|  |  |  |  | 2 - SEC_DRV_CAT_STAT_Locked |
| Examples |  |  |  |  |
| 0x00 |  | Category is unlocked. |  |  |
| 0x02 |  | Category is locked. Writing of category body no longer possible. |  |  |

### Flags

| Name: Flags |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bit Nr | Name |  | Description | Values |
| 1 | SEC_DRV_CAT_FLAG_LinkLocked |  | if the category is dependent on a parent. Disable locking/unlocking on its own! TR6.2 | 0 – category can be unlocked on its own |
|  |  |  |  | 1 – category can be only unlocked by unlocking parent category |
| 2 | SEC_DRV_CAT_FLAG_UseSeed |  | Use seed to generate key during first boot | 1 – A seed is needed for this category. |
| Examples |  |  |  |  |
| 0x01 |  | if the category is dependent on a parent. Disable locking/unlocking on its own! |  |  |
| 0x02 |  | Use seed to generate key during first boot |  |  |

### Dependency

Dependency flag is introduced to implement Linking between categories (TR6.2) Every bit in Dependency field points to chilled category, which is linked to actual one.

| Name: Dependency |  |  |  |
| --- | --- | --- | --- |
| Size | Name | Description | Values |
| 2 | Dependency | Each bit represents a category that is a child of this one. Children locks/unlocks with their parents. | 0/1 for every bit |
| Examples |  |  |  |
| If Dependency flag of SIM category is set to 0x08 (pointing to CP) and SEC_DRV_CAT_FLAG_LinkLocked bit of Flags field (please see “Flags” description) of CP category is set to 1, it would mean SIM-C lock (while unlocking SIM, CP will automatically be unlocked, CP cannot be unlocked separately from SIM) If Dependency flag of NS category is set to 0x04 (pointing to SP) and SEC_DRV_CAT_FLAG_LinkLocked bit of Flags field (please see “Flags” description) of SP category is set to 0, it would mean NS-SP lock (while unlocking NS, SP will automatically be unlocked, SP can be unlocked separately from NS) !!! Please be aware that by this mechanism customer can set any combination of linked/dependent personalization types. On the other hand it is in customer responsibility to guarantee that proper combination of Dependency/Flags field is stored on MEPD |  |  |  |

### DataLen

| Name: DataLen |  |  |  |
| --- | --- | --- | --- |
| Size | Name | Description | Values |
| 2 | DataLen | Number of bytes present in the category body. |  |
| Examples |  |  |  |
| 0x08 | Length of the category body is 8 bytes |  |  |

### KeyLength

| Name: KeyLength |  |  |  |
| --- | --- | --- | --- |
| Size | Name | Description | Values |
| 1 | KeyLength | Length of the key, which shall be generated by Security Driver on first boot in bytes | 6-16 |
| Examples |  |  |  |
| 0x08 | Length of the control key shall be 8 digits |  |  |

## Category Body

This is a sequence of bytes with no meaning to the security driver itself. These data are to be initialized and used by the ACI layer. The data however, can only be written if the corresponding category is unlocked.

The maximum number of bytes available is defined by the constant SEC_DRV_CAT_MAX_SIZE, currently set to 256 bytes.

When stored in the file/flash, these data must always occupy the maximum available space. That is, even though a category does not have any data, SEC_DRV_CAT_MAX_SIZE bytes should be reserved (but the DataLen field in the category header should be set to 0).

Following points should be noted for storing data in the MEPD.

- Network, Network Subset & MSIN codes shall be stored in BCD format, just as present in the IMSI value.

- GID 1 & GID2 values shall be stored just as stored in the Elementary file.

- The parity nibble shall be included when specifying the network code. (A parity nibble does not need to be considered when specifying regular expressions.)

- Regular expressions shall be stored in the ASCII format for simplicity of code and easy understanding.

- As mentioned in the structures below, the part above ‘Current user code index’ shall be configurable. The part from ‘Current user code index’ (shaded in blue) shall be maintained by ACI.

- Operator codes shall be identified by a code type:

0x0a indicating Normal code,

0x0b indicating Range,

0x0c indicating Regular Expression

0x0d indicating 8th digit intervals (Only for Network Subset category)

The following are the MEPD structures for each category:

### Network category:

A network code (MCC+MNC) consists of 5/6 digits and a parity nibble.

One normal network code would occupy 4 bytes for storage including parity nibble.

A Range would require 8 bytes:

4 bytes for Start value + 4 bytes for End value of range.

A Regular Expression would require 6 bytes

The following structure depicts how the Network category codes will be stored in the MEPD

|  | Bytes |
| --- | --- |
| Max number of User Codes (To be configured) | 1 |
| Number of Operator Codes (To be configured) | 1 |
| Operator Code-group length (To be set during configuration) where length = total number of bytes occupied by operator codes | 1 |
| Code type = 0x0a ( Type identifier for normal code) | 1 |
| Normal code (Including parity nibble) | 4 |
| Code type = 0x0b (Type identifier for range) | 1 |
| Start value (Including parity nibble) | 4 |
| End value (Including parity nibble) | 4 |
| Code type = 0x0c (Type identifier for regular expression) | 1 |
| Regular Expression | 6 |
| … |  |
| … |  |
| .. |  |
| Number of User Codes | 1 |
| Current user code index (for FIFO based addition of user codes) (should be set to 0xff if no user codes are present) | 1 |
| User code1 | 4 |
| User code 2 | 4 |
| … |  |
| … |  |
| … |  |
| User code n (n = max number of user codes) | 4 |

Example for storing the Normal network code.

For e.g 1.: A Normal network code “123456” will be stored as follows:

| 0x19 | Byte 1 |
| --- | --- |
| 0x32 | Byte 2 |
| 0x54 | Byte 3 |
| 0xf6 | Byte 4 |

Digit 9 in byte 1 indicate parity nibble. 0xf in byte 4 indicates unused nibble.

For e.g 2.: A Normal network code “12345” will be stored as follows:

| 0x19 | Byte 1 |
| --- | --- |
| 0x32 | Byte 2 |
| 0x54 | Byte 3 |
| 0xff | Byte 4 |

Range codes shall also be stored in a similar manner.

For e.g 3.: A regular expression “xx1234” will be stored as follows:

| ‘x’ | Byte 1 |
| --- | --- |
| ‘x’ | Byte 2 |
| ‘1’ | Byte 3 |
| ‘2’ | Byte 4 |
| ‘3’ | Byte 5 |
| ‘4’ | Byte 6 |

If no user-codes are present, all user-code nibbles should be set to 0xff

### Network Subset category:

The personalization check for this category involves Network code check + Network subset code check. Hence we shall store a combination of these 2 codes for this category.

i.e. If Network code is 123456 and NS code is 78, then we shall store 12345678 in the Network subset category.

Based on this, one normal network subset code shall need 5 bytes.

A range of network subset codes would occupy 6 bytes:

4 bytes for NW code + 1 byte for Start value + 1 byte for End value.

A regular expression would occupy 8 bytes. (including network code)

The following structure depicts how a Network subset code can be stored in the MEPD.

|  | Bytes |
| --- | --- |
| Max number of User Codes (To be configured) | 1 |
| Number of Operator Codes (To be configured) | 1 |
| Operator Code-group length (To be set during configuration) | 1 |
| Code type = 0x0a | 1 |
| Normal code | 5 |
| Code type = 0x0b | 1 |
| Network code | 4 |
| Start value of NS code | 1 |
| End value of NS code | 1 |
| Code type = 0x0c | 1 |
| Regular expression including Network + NS Code | 8 |
| Code type = 0x0d (8thdigit normal code/range) | 1 |
| Network code (Including parity nibble) | 4 |
| No. of 8th digit values for which normal codes and/or intervals are to be stored | 1 |
| 8th digit value | 1 |
| No. of normal codes to be associated with the digit (n1) | 1 |
| No. of intervals to be associated with the digit (n2) | 1 |
| Normal code 1 | 1 |
| … | 1 |
| … | 1 |
| Normal code n1 | 1 |
| Start value of interval 1 | 1 |
| End value of interval 1 | 1 |
| .. | 1 |
| .. | 1 |
| .. | 1 |
| Start value of interval n2 | 1 |
| End value of interval n2 | 1 |
| Number of User Codes | 1 |
| Current user code index (for FIFO based addition/deletion of user codes) (should be set to 0xff if no user codes are present) | 1 |
| User code1 | 5 |
| User code 2 | 5 |
| …. |  |
| … |  |
| … |  |
| User code n (n = Max number of user codes for NS) | 5 |

Examples for storing Network subset code:

For e.g 1.: A Normal network subset code “12345678” will be stored as follows:

| 0x19 | Byte 1 |
| --- | --- |
| 0x32 | Byte 2 |
| 0x54 | Byte 3 |
| 0x76 | Byte 4 |
| 0xf8 | Byte 5 |

Digit 9 in byte 1 indicate parity nibble. 0xf in byte 5 indicate unused nibble.

(The value of the parity nibble shall be as per the 3GPP standard TS 04.08.)

For e.g 2.: A Normal network subset code “1234567” will be stored as follows:

| 0x11 | Byte 1 |
| --- | --- |
| 0x32 | Byte 2 |
| 0x54 | Byte 3 |
| 0x76 | Byte 4 |
| 0xff | Byte 5 |

Range codes shall also be stored in a similar manner.

For e.g 3.: A regular expression “12345x1” will be stored as follows:

| ‘1’ | Byte 1 |
| --- | --- |
| ‘2’ | Byte 2 |
| ‘3’ | Byte 3 |
| ‘4’ | Byte 4 |
| ‘5’ | Byte 5 |
| ‘x’ | Byte 6 |
| ‘1’ | Byte 7 |

Regarding the 8th Digit interval:

Just as Code type = 0x0a indicates that the following code is a normal code,

Code type = 0x0d indicates that the following code is a normal code/range associated with the 8th digit  value. The 8th digit value could be any digit from 0 to 9. A normal code or range could be associated with any digit.

Range can also be specified for a few digit values say digit value 0 & digit value 1.

Consider the following example:

A CTS configuration of category codes (with associated 8th digit of IMSI) could be: 

Definition of category codes with : 
   MNC/MCC: 1 plmn 
   NS8D : values of 8th digit of the IMSI 
   NS: normal code or interval of NS (HLR) values linked to previous value of 8th digit of the IMSI 

MCC;262 
MNC;02 
   NS8D;0      NS;10,19 
   NS8D;1      NS;10,19 
   NS8D;1      NS;58,59 
   NS8D;1      NS;63 
   NS8D;1      NS;68 
   NS8D;1      NS;71 
   NS8D;2      NS;10,29 
   NS8D;2      NS;35,89 

This configuration define, for one PLMN (262 02) : 
   . 1 interval for NS8D=0 
   . 2 intervals and 3 normal code for NS8D=1 
   . 2 intervals for NS8D=2 

In this configuration, for example : 
   . SIM card with N=26202, NS=15 and value of 8th digit = 0 will be authorized 
   . SIM card with N=12345, NS=15 and value of 8th digit = 0 will be NOT authorized 
   . SIM card with N=26202, NS=20 and value of 8th digit = 0 will be NOT authorized 
   . SIM card with N=26202, NS=57 and value of 8th digit = 1 will be NOT authorized 
   . SIM card with N=26202, NS=59 and value of 8th digit = 1 will be authorized 
   . SIM card with N=26202, NS=63 and value of 8th digit = 1 will be authorized 
   . SIM card with N=26202, NS=64 and value of 8th digit = 1 will be NOT authorized 
   . SIM card with N=26202, NS=63 and value of 8th digit = 1 will be authorized 
   . SIM card with N=26202, NS=25 and value of 8th digit = 2 will be authorized 
   . SIM card with N=26202, NS=30 and value of 8th digit = 2 will be NOTauthorized

| 0x0d | Byte 1 | Identifier for 8th digit associated normal codes/range |
| --- | --- | --- |
| 0x21 | Byte 2 | Network code |
| 0x26 | Byte 3 |  |
| 0x20 | Byte 4 |  |
| 0xff | Byte 5 |  |
| 0x03 | Byte 6 | No of 8th digit values(In the given example, codes for 8th digit values 0,1 & 2 are to be specified. i.e. 3 digits) |
| 0x00 | Byte 7 | 8th digit value=0 |
| 0x00 | Byte 8 | No. of normal codes associated with digit value 0 |
| 0x01 | Byte 9 | No. of intervals associated with digit value 0 |
| 0x01 | Byte 10 | Packed BCD format of NS code 10 & 19 (Interval 1) |
| 0x91 | Byte 11 |  |
| 0x01 | Byte 12 | 8th digit value=1 |
| 0x03 | Byte 13 | No. of normal codes associated with digit value 1 |
| 0x02 | Byte 14 | No. of intervals associated with digit value 1 |
| 0x36 | Byte 15 | Packed BCD format of normal NS code 63 |
| 0x86 | Byte 16 | Packed BCD format of normal NS code 68 |
| 0x17 | Byte 17 | Packed BCD format of normal NS code 71 |
| 0x01 | Byte 18 | Packed BCD format of NS code 10 & 19 (Interval 1) |
| 0x91 | Byte 19 |  |
| 0x85 | Byte 20 | Packed BCD format of NS code 58 & 59 (Interval 2) |
| 0x95 | Byte 21 |  |
| 0x02 | Byte 22 | 8th digit value=2 |
| 0x00 | Byte 23 | No. of normal codes associated with digit value 2 |
| 0x02 | Byte 26 | No. of intervals associated with digit value 2 |
| 0x01 | Byte 27 | Packed BCD format of NS code 10 & 29 (Interval 1) |
| 0x92 | Byte 28 |  |
| 0x53 | Byte 29 | Packed BCD format of NS code 35 & 89 (Interval 2) |
| 0x98 | Byte 30 |  |

If no user-codes are present, all user-code nibbles should be set to 0xff

### SIM Code category:

The personalization check for this category involves the complete IMSI value check. Hence the complete IMSI value shall be stored for this category.

Based on this:

A normal SIM code shall occupy 8 bytes: (parity nibble + 15 digits)

A single range of SIM code would occupy 13bytes:

5 bytes for NW & NS code +

4 bytes for Start MSIN value + 4 bytes for End MSIN value.

A regular expression would occupy 15 bytes.

If no user-codes are present, all user-code nibbles should be set to 0xff

|  | Bytes |
| --- | --- |
| Max number of User Codes (To be configured) | 1 |
| Number of Operator Codes (To be configured) | 1 |
| Operator Code-group length (To be set during configuration) | 1 |
| Code type = 0x0a (Normal code) | 1 |
| Normal code1 | 8 |
| Code type = 0x0b (Range) | 1 |
| NW + NS code | 5 |
| Start MSIN | 4 |
| End MSIN | 4 |
| Code type = 0x0c (Regular Expression) | 1 |
| Regular Expression | 15 |
| Code type = 0x0c (Regular Expression) | 1 |
| … |  |
| … |  |
| Number of User Codes | 1 |
| Current user code index (for FIFO based addition/deletion of user codes) (should be set to 0xff if no user codes are present) | 1 |
| User code1 | 8 |
| User code 2 | 8 |
| …. | . |
| User code n (n = Max number of user codes for SIM category) | 8 |

### Service Provider Category:

The personalization check for this category involves Network code check + GID1 value check. Hence we shall store a combination of these 2 codes for this category. GID1 value can be provided in a maximum of 4 bytes.

Based on this, one Normal Service Provider subset code shall need 8 bytes:

4 bytes for Network code + 4 bytes for GID1 value

A single range would occupy 12 bytes:

4 bytes for NW code + 4 bytes for Start GID1 value + 4 bytes for End GID1 value.

A regular expression would occupy 10 bytes ( 6 bytes for NW code + 4 bytes for GID1 value)

|  | Bytes |
| --- | --- |
| Max number of User Codes (To be configured) | 1 |
| Number of Operator Codes (To be configured) | 1 |
| Operator Code-group length (To be set during configuration) | 1 |
| Code type = 0x0a ( Normal code) | 1 |
| Network code + GID1 value | 8 |
| Code type = 0x0b (Range) | 1 |
| Network code | 4 |
| Start GID1 | 4 |
| End GID1 | 4 |
| Code type = 0x0c (Regular Expression) | 1 |
| Regular Expression (including NW code and GID1 value) | 10 |
| Code type = 0x0c (Regular Expression) | 1 |
| .. |  |
| … |  |
| Number of User Codes | 1 |
| Current user code index (for FIFO based addition/deletion of user codes) (should be set to 0xff if no user codes are present) | 1 |
| User code1 | 8 |
| User code 2 | 8 |
| …. |  |
| … |  |
| User code n (n = max number of user codes for SP category) | 8 |

Example for storing SP codes:

For e.g 1.: For a NW code value of ”123456” & GID1 value of  0x1234, the code will be stored as follows:

| 0x19 | Byte 1 |
| --- | --- |
| 0x32 | Byte 2 |
| 0x54 | Byte 3 |
| 0xf6 | Byte 4 |
| 0x12 | Byte 5 |
| 0x34 | Byte 6 |
| 0xff | Byte 7 |
| 0xff | Byte 8 |

If no user-codes are present, all user-code nibbles should be set to 0xff

### Corporate Category:

The personalization check for this category involves Network code check + GID1 value + GID 2 value check. Hence we shall store a combination of these 3 codes for this category.

Based on this, one Normal Corporate code shall need 12 bytes:

4 bytes for Network code + 4 bytes for GID1 value + 4 bytes for GID2

A single range of Corporate code would occupy 16 bytes:

4 bytes for NW Code + 4 bytes for GID1 value +

4 bytes for GID2 start value + 4 bytes for GID2 End value.

A regular expression would occupy 14 bytes

(Considering 6 bytes for NW Code + 4 bytes for GID1 value + 4 bytes for reg. exp. for GID2/Corporate code.)

If no user-codes are present, all user-code nibbles should be set to 0xff

|  | Bytes |
| --- | --- |
| Max number of User Codes (To be configured) | 1 |
| Number of Operator Codes (To be configured) | 1 |
| Operator Code-group length (To be set during configuration) | 1 |
| Code type = 0x0a (Normal code) |  |
| Normal code | 12 |
| Code type = 0x0b (Range) | 1 |
| NW code | 4 |
| GID1 value | 4 |
| GID2 Start value | 4 |
| GID2 End value | 4 |
| Code type = 0x0c (Regular Expression) | 1 |
| Regular Expression | 14 |
| Code type = 0x0c | 1 |
| … |  |
| … |  |
| Number of User Codes | 1 |
| Current user code index (for FIFO based addition/deletion of user codes) (should be set to 0xff if no user codes are present) | 1 |
| User code1 | 12 |
| User code 2 | 12 |
| …. | . |
| User code n (n = Max number of user codes for CP category) | 12 |

## CTS-MEPD cross table

|  | SIM LOCK SETTINGS | Value 1 / Min | Value 2 / Max | Value 3 | Custom value ( Default value ) | MEPD Fi |
| --- | --- | --- | --- | --- | --- | --- |
| N | SIMLOCK NCK Key Length | From 8 | To 16 |  | 10 | “KeyLength” field of NW MEPD record. Key of “KeyLength”-bytes will be generated |
|  | SIMLOCK NCK Status | locked | unlocked |  | unlocked | “Status” field of NW MEPD record. |
|  | SIMLOCK NCK Automatic add on lock | Y | N |  | N | AddNewIMSI field of MEPD Configuration |
| NS | SIMLOCK NSCK Key Length | From 8 | To 16 |  | 10 | “KeyLength” field of NS MEPD record. Key of “KeyLength”-bytes will be generated |
|  | SIMLOCK NSCK Status | locked | unlocked |  | unlocked | “Status” field of NS MEPD record. |
|  | SIMLOCK NSCK slave | Y | N |  | N | “Dependency” field of MEPD Record |
|  | SIMLOCK NSCK Automatic add on lock | Y | N |  | N | AddNewIMSI field of MEPD Configuration |
| SP | SIMLOCK SPCK Key Length | From 8 | To 16 |  | 10 | “KeyLength” field of SP MEPD record. Key of “KeyLength”-bytes will be generated |
|  | SIMLOCK SPCK Status | locked | unlocked |  | unlocked | “Status” field of SP MEPD record. |
|  | SIMLOCK SPCK slave | Y | N |  | N | “Dependency” field of MEPD Record |
|  | SIMLOCK SPCK Automatic add on lock | Y | N |  | N | AddNewIMSI field of MEPD Configuration |
| SIM | SIMLOCK PCK Key Length | From 10 | To 16 |  | 10 | “KeyLength” field of SIM MEPD record. Key of “KeyLength”-bytes will be generated |
|  | SIMLOCK PCK Status | locked | unlocked |  | unlocked | “Status” field of SIM MEPD record. |
|  | SIMLOCK PCK slave | Y | N |  | N | “Dependency” field of MEPD Record |
|  | SIMLOCK PCK Automatic add on lock | Y | N |  | N | AddNewIMSI field of MEPD Configuration |
| C | SIMLOCK CCK Key Length | From 8 | To 16 |  | 10 | “KeyLength” field of CP MEPD record. Key of “KeyLength”-bytes will be generated |
|  | SIMLOCK CCK Status | locked | unlocked |  | unlocked | “Status” field of CP MEPD record. |
|  | SIMLOCK CCK slave | Y | N |  | N | “Dependency” field of MEPD Record |
|  | SIMLOCK CCK Automatic add on lock | Y | N |  | N | AddNewIMSI field of MEPD Configuration |
|  | SIMLOCK Key comparaison truncte to 8 digit | Y | N |  | N | SEC_DRV_HDR_FLAG_Truncation. If set, 8 bytes are compared, otherwise full length. Please be aware since TR specifies truncation only for OTA depersonalization and OTA depersonalization is out of TR, this flag is not used now. Please inform if it is needed to use that flag for other types of unlock |
|  | SIMLOCK Global failure counter | 1 | 0xff |  | 10 | FC_Max Field of MEPD Configuration. Disabled, if set to 0xff |
|  | SIMLOCK Global failure counter value | From 0 | To 254 | Removed | 10 | FC_Current Field of MEPD Configuration. |
|  | SIMLOCK Reset failure counter | 1 | 0xff |  | 10 | FC_Reset_Fail_Max Field of MEPD Configuration. Disabled, if set to 0xff |
|  | SIMLOCK Failure timer | Y | N |  | N | SEC_DRV_HDR_FLAG_Unlock_Timer |
|  | SIMLOCK Failed reset counter value | From 0 | To 254 | Removed | 10 | FC_Reset_Fail_Current Field of MEPD Configuration |
|  | SIMLOCK Used reset counter value | From 0 | To 254 | Removed | 10 | FC_Reset_Success_Current Field of MEPD Configuration. Please be aware that there is a FC_Reset_Success_Max, which specifies, how many times user allowed to reset FC succesfuly. Similar to FC_Reset_Fail_Max |
|  | SIMLOCK Category unlock disabling | Y | N |  | N | SEC_DRV_HDR_FLAG_LAM_Unlock |
|  | SIMLOCK behaviour | ALCATEL | ETSI |  | ALCATEL | SEC_DRV_HDR_FLAG_ETSI_Flag |
|  | SIMLOCK AIRTEL config | Y | N |  | N | SEC_DRV_HDR_FLAG_Airtel_Ind |
|  | SIMLOCK D2 config | Y | N |  | N | The 8th digit of IMSI associated with NS Code is configured by normal codes, rangeor their combination. |
|  | SIMLOCK TELEFONICA config | Y | N |  | N | Realized by “dependency” field and SEC_DRV_CAT_FLAG_LinkLocked flag of MEPD record |
|  | SIMLOCK MNC Key | 2 digits | 3 digits |  | 2 digits | MNC_Len |
|  | SIMLOCK GID1Size | 1 Byte | 4 Bytes |  | 1 | GID1_Len |
|  | SIMLOCK GID2 Size | 1 Byte | 4 Bytes |  | 1 | GID2_Len |
|  | GSM Test simcard accepted | Y | N | restricted* | Y | TR 4.1: typeApprovalSIM field of MEPD configuration |
|  | CPHS Test simcard accepted | Y | N |  | Y | TR 4.2: testCPHS field of MEPD configuration |

# External MEPD Format

This section describes the data structure used to represent encrypted MEPD data to be downloaded to the flash of the mobile phone. This format is converted by the first boot process to the structure described in chapter 1.

All fields of more than 8 bits in this format will need to be encoded using the same endianess as the handset.

## Data sections

The data is structured as a series of sections.

| External sections | Bits | Description |
| --- | --- | --- |
| Seed count | 8 | Number of seeds needed |
| DES key + Checksum | 1024 | RSA encrypted. |
| Size | 16 | Included in the checksum. |
| External lock structure | ? | Encrypted using the above random DES key. Included in the checksum. |

Seed count:

This is simply an unsigned integer, containing the number of categories in the external lock structure that needs a seed in order to generate a key. NOTE: A seed for the failure counter reset key is always included, so the seed count cannot be less than 1.

If the during first-boot scenario, this number doesn’t comply with the actual number of categories that have a set SEC_DRV_CAT_FLAG_UseSeed flag, then the handset will reset and act as if no lock file has been flashed.

DES key + checksum:

This is a simple structure as shown below

| DES key + checksum |  |  |
| --- | --- | --- |
| Type | Name | Value |
| UINT8 [8] | Key | These 8 bytes contain a random DES key, used to encrypt the external lock structure. |
| UINT8 [20] | Checksum | This is a 160 bit SHA-1 checksum over the size and the encrypted external lock structure. |
| UINT8 [100] | Padding | Padding due to the nature of the RSA algorithm. |

The structure is encrypted using the RSA algorithm using the manufacturer’s private key. NOTE: The 1024 bits stored are not packaged using PKCS#1 (ASN.1/BER) encoding. They are stored like the signatures in the firmware certificate.

Example: Consider the following DES key and SHA-1 digest:

| DES key | 31 32 33 34 35 36 37 38 |
| --- | --- |
| SHA-1 digest | 73 86 B1 21 25 5F 08 E2 7C 90 3A 1E 77 79 8C FF C8 EB E7 83 |

Suppose we concatenate these and encrypt the resulting block using the following RSA private key (padding consists of all zeros, key values listed below with least significant byte first):

| Exponent | 31 D6 04 EC 80 23 F8 B1 73 8D C6 24 1F 29 35 77 44 E1 12 3F E8 8E 02 06 DD 01 A6 D4 CC 3E 7F 8B 1E 51 F2 09 3B CF DE 6B 69 39 B3 0A CA 93 4B DC B4 0A C9 1C F2 2D 88 D5 1B 72 6A D0 B5 21 95 7B 0B 7A 0B C3 89 07 8D F3 8F 80 BC 5E 90 18 DB 18 53 FD D8 1E CF 3F 57 4E 71 BA F1 E2 C2 D8 7D F2 C9 A2 D5 F6 18 A5 8A 34 7F D3 57 11 85 D3 17 DB 10 34 E4 15 31 C3 83 FA 5B FA 37 41 EC 15 B0 67 |
| --- | --- |
| Modulus | 4F 33 66 9B 34 80 E5 C8 16 4B 49 90 76 9F EB 29 5F DB 22 0D 0E 8B 81 E6 4C E7 34 4D 5D 03 7F 80 28 3F 45 44 97 38 1A C4 38 9C 30 E3 76 59 23 E6 BE 0A 08 34 11 71 13 7C 04 57 84 3C 33 0A 1D 8A E1 E3 09 1D 11 16 0C 71 F3 C5 22 5E 70 62 81 43 10 24 ED 9F C4 CF 70 B1 EC EC 42 50 03 C2 31 3A 5D 81 BA DA 87 66 B3 EC EC A5 51 AC 3E 78 E6 8B 35 26 7F 73 88 8C 21 7D DC 5E 5E 52 C9 75 80 BE |

This will produce the following 128-bytes value to be included in the MEPD file:

5F 73 1E 6B DA 65 C2 47 47 7E 1D 86 53 D9 2C 65 B9 0E AF BB

53 0F C1 6D CD 80 CE F5 FD 0C FB 47 E0 C6 6C 51 B6 6E 1A 6D

37 36 6E A2 13 AB B1 84 C2 54 BC C2 93 15 00 94 A5 95 31 8E

BE DC 8B 87 93 81 E1 C1 6A 8E B8 BB 6E 8D BF F2 7E 54 D2 30

89 B8 E8 16 24 4D 45 2D 03 99 D2 60 82 0D 1A 8C 3F 24 99 6F

58 2A E9 59 13 CE C5 77 F9 30 F0 75 1C 30 07 76 FD 7A 75 CB

93 DB 8C F7 D9 45 F6 56

The example used padding of all zeros, but it is possible to use any values for the padding.

External lock structure:

This is the actual structure defining the individual lock categories. The layout of these data resembles part of the internal structure very closely. Size of each section must be padded to be a multiplum of 8 (size modulus 8 = 0).  The entire external lock structure is encrypted using the DES algorithm with the key contained in the RSA encrypted field, before the checksum is calculated.

External lock structure

MEPD configuration

Category 0 (NW )

Category 1 (NS)

Category 2 (SP)

Category 3 (CP)

Category 4 (SIM)

Category 5 (AP)

Category X (n-6 times)

Each category is in turn divided in 2 subsections. The size of each section here must also be padded to be a multiplum of 8 (size modulus 8 = 0).

Category section

Category header

Category data

The layout of each of these data sections is described in chapter 4.1.

Example: The following listing contains a simple external lock structure:

07 01 01 00 01 00 01 01 01 01 01 00 00 00 00 00 01 06 03 00

04 00 00 00 A1 B2 C3 D4 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

When encrypted using the DES key from the previous example, the result is the following block to be inserted in the MEPD file:

1A 55 BA 23 B6 80 71 BE A6 F0 A6 8A 2A 75 53 97 E0 2E C0 CC

2F 7A C6 56 AE 80 FE 03 D4 7E 26 26 3D 75 95 A9 8B FF 80 9D

3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9

8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D

3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9

8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D

3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9

8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D

3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9

8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D

3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9

8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D

3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9

8B FF 80 9D 3D 75 95 A9 8B FF 80 9D 3D 75 95 A9 8B FF 80 9D

When passed to the SHA-1 algorithm together with the size, this data produces the SHA-1 digest listed in the previous example.

# Appendices

  - Abbreviation and Acronyms

This is just an example!:

| ACI | Access Control Interface |
| --- | --- |
| DSP | Digital Signal Processor |
| DES |  |
| MMI | Man Machine Interface |
| FLUID | Flash Loader Utility Independent of Device |
| ME | Mobile Equipment |
| SIM | Subscriber Identity Module |
| GPF |  |
| OTP | One Time Programmable memory |
| RAM | Random Access Memory |
| MPK | Manufacturer Private Key |

  - Glossary

This is just an example!:

| Certificate | A certificate is a form of digital passport or credential that simplifies the task of establishing whether a public key truly belongs to the purported owner. |
| --- | --- |
| Firmware | Legacy code delivered by the manufacturer and permanently resident on the platform. The firmware is authenticated by the manufacturer’s certificate. |

Place will be reserved. API and code change will be provided on official CR from the customer.

Place will be reserved. API and code change will be provided on official CR from the customer.