Service Access Point

SIM Driver

| Department: | Aalborg Wireless Center |
| --- | --- |
| Creation Date: | 20 January, 2003 |
| Last Modified: | 10 June, 2004 by Flemming Dunker |
| ID and Version: | 8010.136.03.009 |
| Status: | Being Processed |

- Document Control

Copyright  2004 Texas Instruments, Inc.

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Texas Instruments reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Texas Instruments. Texas Instruments accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a license agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Texas Instruments.

  - Document History

ID	Author	Date	Status

8010.136.03.001	KKS	20 January, 2003	Accepted

Initial version.

8010.136.03.002	KKS	20 January, 2003	Accepted

Document based on 7510.126.02.10

8010.136.03.003	KKS	20 January, 2003	Accepted

Group column in function prototype removed

8010.136.03.004	KKS	26 May, 2003	Accepted

Links to constants added

8010.136.03.005	KKS	26 May, 2003	Accepted

sim_ in function names changed to sl2_.

8010.136.03.006	KKS	20 August, 2003	Accepted

Function names in MSCs in section 6 changed from sim_ to sl2_

8010.136.03.007	FK	20 November, 2003	Being processed

Revised

8010.136.03.008	FDU	3 May, 2004	Being Processed

Alignment according to review report.

8010.136.03.009	FDU	10 June, 2004	Being Processed

Alignment with agreements and review comments from meeting in Nice

  - References, Abbreviations, Terms

[TI 8010.801]	8010.801, References and Vocabulary, Texas Instruments

Table of Contents

1	Introduction	4

2	Constants	5

3	Primitives	6

3.1	SIMDRV_DUMMY_PRIM	6

4	Functions	7

4.1	simdrv_register	7

4.2	simdrv_poweroff	7

4.3	simdrv_reset	8

4.4	simdrv_xch_apdu	8

5	Parameters	10

5.1	Insert call-back function pointer	10

5.2	Remove call-back function pointer	10

5.3	SIM Card Information	10

5.4	Reset return value	11

5.5	Result Data	11

5.6	Length of Expected Data	12

5.7	Send Data	12

5.8	Status Words	13

5.9	Class byte	13

5.10	SIM Instruction Code	14

5.11	Command Header	14

5.12	Perform voltage selection	15

5.13	SIM Command Parameters	15

5.14	Configuration Characteristics Request	15

5.15	Configuration Characteristics	16

5.16	Reader Id	16

5.17	UICC Characteristics	17

6	MSCs	18

6.1	Successful activation	18

6.2	Unsuccessful activation	19

6.3	Power off	20

6.4	Issuing commands	20

6.5	Command  Types	21

# Introduction

The interface is function based only.

Throughout the document the term UICC is used to cover both a GSM SIM that contains one single application, the GSM application, and a multi-application UICC that contains one or more applications, maximum one GSM and one or more  UMTS applications.

Data stores are used to transfer information between  the SIM reader driver and the PS. Pointers to the data stores where the data is written to and read from are handed to the SIM reader driver by the PS. The format of the data stored in these stores are compliant  to the format specified in [3G 11.11], [3G 11.14], [3G 31.102], [3G 31.111] and [ETSI 102 221]. Therefore, the formatting of data  located in these stores are not described throughout this document. The same approach is used in regards to  status words, etc. Only in the case where SIM data is reformatted, will they be described in this document.

Two functions are not defined in this document and are the two only functions called from the SIM driver to the PS. These are the SIM insertion and SIM removal functions. The first call after power-on, the simdrv_register function, gives two function pointers to the SIM driver for these two functions. Else, all function calls are initiated by the PS.

The transmission protocol is handled autonomously by the SIM reader driver. This means that the interface between the PS and SIM reader driver  operates at APDU command level and that the PS has no knowledge of which protocol has been selected for communication between the SIM reader driver and the inserted UICC during the PPS procedure. Thus the PS is not required to perform the GET RESPONSE command, instead this is done, in case of protocol T=0, by the driver .

Figure 1 - Interface overview

Communication between the PS and the driver will always be initiated by the PS, with two exceptions to that rule.

- When the PS asks the driver to reset the UICC, the driver uses a call back function that is handed during initialisation to inform the PS.

- When the inserted UICC is physically removed from the PS, the driver calls another call back function that is handed during initialisation

# Constants

Description:

Defines maximum value used in parameter definitions

Pragma:

| Name | Value | Comment |
| --- | --- | --- |
| PREFIX | SIMDRV | Prefix for this document |
| ALLWAYS_ENUM_IN_VAL_FILE | YES | Adds enumerations in the .val file. |
| ENABLE_GROUP | YES | Enable h-file grouping |
| COMPATIBILITY_DEFINES | NO | Compatible to the old #defines |

Definition:

| Name | Value | Comment | Group |
| --- | --- | --- | --- |
| SIZE_ATR_INFO | 0x21 | The maximum length of the data returned from the ATR procedure | uicc |
| MIN_RESULT | 0x01 | Minimum size of the response of any given command | uicc |
| MAX_RESULT | 0x100 | Maximum size of the response of any given command | uicc |
| MIN_DATA_SIZE | 0x01 | Minimum length of a data element | uicc |
| MAX_DATA_SIZE | 0xFF | Maximum length of a data element | uicc |
| MAX_READERS | 0x02 | Maximum number of card readers supported. | uicc |

History:

20-February-02	FDU	Initial
30-October-2003	FK	'MAX_RESULT' corrected
03-May-04	FDU	PREFIX changed to SIMDRV

# Primitives

## SIMDRV_DUMMY_PRIM

Description:

Dummy.

Definition:

| Short Name | ID | Direction | Group |
| --- | --- | --- | --- |
| SIMDRV_DUMMY | 0x80FF009A | SIM -> DRIVER | uicc |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| Insert call-back function pointer | insert | PTR | 5.1 | STRUCT |
| Remove call-back function pointer | remove | PTR | 5.2 | STRUCT |
| SIM Card Info | atr_string_info | PTR | 5.3 | STRUCT |
| Return Value | reset_return_val |  | 5.4 | U8 |
| Result Buffer | result_info | PTR | 5.5 | STRUCT |
| Length of expected data | len |  | 5.6 | U16 |
| Update Data Element | data_info | PTR | 5.7 | STRUCT |
| Status Words | sw1_2 |  | 5.8 | U16 |
| Class Byte | cla |  | 5.9 | U8 |
| Instruction Code | ins |  | 5.10 | U8 |
| Transparent command header | cmd_header |  | 5.11 | STRUCT |
| Perform Voltage Selection | voltage_select |  | 5.12 | U8 |
| SIM Command Parameters | p1 |  | 5.13 | U8 |
| SIM Command Parameters | p2 |  | 5.13 | U8 |
| Configuration Characteristics requested | config_requested |  | 5.14 | U8 |
| Configuration Characteristics | config_characteristics | PTR | 5.15 | STRUCT |
| Reader Id | reader_id |  | 5.16 | U8 |
| UICC Characteristics | uicc_characteristics |  | 5.17 | U8 |

History:

9-December-2002	BRY	Initial version after merge.
20-November-2003	FK	New prefix; changes in section 5.
8-Dec-2003	FK	Revised
03-May-04	FDU	Aligned with contents of chapter 4 and chapter 5
10-June-04	FDU	Aligned with contents of chapter 4 and chapter 5

# Functions

## simdrv_register

Description:

The function is used to establish communication between the PS and the SIM reader driver. Two call back function pointers are passed to the driver. There is no return value.

The call back to 'insert()' is executed by the driver, when it, being called by the function 'simdrv_reset()', is able to successfully reset the SIM card.

The call back 'remove()' is executed by the driver at any time during a card session, when it detects that the SIM card cannot be accessed any more.

Definition:

| Short Name | ID | Direction | Group |
| --- | --- | --- | --- |
| void simdrv_register (void (* insert) (T_SIMDRV_atr_string_info *atr_string_info, U8 config_requested , T_SIMDRV_config_characteristics * config_characteristics), void (* remove)(void)) | InlineC | SIM->DRIVER | none |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| Insert call-back function pointer | insert | PTR | 5.1 | STRUCT |
| Remove call-back function pointer | remove | PTR | 5.2 | STRUCT |
| Answer To Reset string | atr_string_info | PTR | 5.3 | STRUCT |
| Configuration Characteristics requested | config_requested |  | 5.14 | U8 |
| Configuration Characteristics | config_characteristics | PTR | 5.15 | STRUCT |

History:

20-February-2002	FDU	Initial.
20-November-2003	FK	Parameters for callbacks added. 
03-May-04	FDU	function renamed, parameters aligned to review report
10-June-04	FDU	config_request parameters added.
19-Aug-04	FDU	(void) added to remove pointer.

## simdrv_poweroff

Description:

This function is used to inform the SIM reader driver that it shall deactivate. Nothing is passed to the SIM hardware driver, neither is something returned.

Definition:

| Short Name | ID | Direction | Group |
| --- | --- | --- | --- |
| void simdrv_poweroff (U8 reader_id) | InlineC | SIM->DRIVER | none |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| Reader Id | reader_id |  | 5.16 | U8 |

History:

20-February-2002	FDU	Initial.
20-November-2003	FK	Prefix changed; description extended. 
03-May-04	FDU	function renamed
10-June-04	FDU	reader id added

## simdrv_reset

Description:

The PS uses this function to make the driver attempt a reset of the inserted UICC and to start card session.

If the driver does not detect the presence of a SIM card, then it shall return the value SIM_NOT_INSERTED.

If the driver is able to successfully initialise the card and start a card session, it calls the call-back function 'insert()' with the appropriate parameters. After regaining control from the call-back function the driver finishes the reset function and returns the value SIM_INSERTED.

The function has a parameter which is used to indicate to the SIM reader driver whether or not the SIM reader driver shall reset the card performing the voltage selection procedure or reset the card using the current voltage class.

Definition:

| Short Name | ID | Direction | Group |
| --- | --- | --- | --- |
| U8 simdrv_reset (U8 reader_id , U8 voltage_select) | InlineC | SIM->DRIVER | none |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| Reader Id | reader_id |  | 5.16 | U8 |
| Perform voltage selection | voltage_select |  | 5.12 | U8 |
| Return Value | reset_return_val |  | 5.4 | U8 |

History:

20-February-2002 	FDU	Initial.
16-December-2002 	FDU	Parameters changed to pure struct pointers. 
03-May-04	FDU	function renamed, parameters aligned to review report
10-June-04	FDU	reader id added

## simdrv_xch_apdu

Description:

Used to transparently exchange APDU commands and result with any application, which may reside on the SIM. If the communication between SIM and ME is run by the protocol T=0, then the functions performs autonomously the GET RESPONSE Command observing the following rule:

- If the card responds with SW1=9F, then the command shall be coded as described in [GSM 11.11].

- If the card responds with SW1=61, then the command shall be coded as described in [ETSI TS 102 221].

The function complies with the following rules for the interpretation of its parameters:

- Access to a NULL pointer, either read or write, must not be performed. If this leads to a contradiction (i.e. a data count is given, but the null pointer prevents access to the data), then the function shall return SIMDRV_ERR_PARAM_WRONG.

- If parameter 'result_info' is set to ZERO or its element 'len' is set to ZERO, then the driver shall not read data from the card. If the card answers with SW1=61/9F, then this result code is passed to the caller, but GET RESPONSE is not performed.

- If the element 'len' of parameter 'result_info' is set to NON-ZERO, then the driver shall try to read up to 'len' bytes from the card. The data is stored in element 'result', the count element receives the number of actually read bytes. In case of protocol T=0 the element 'len' is identical with parameter P3, in case of protocol T=1 it is identical with parameter Le. If the UICC responds with SW1=6C and a requested size larger than indicated in ‘len’, it shall still  only return the amount of data indicated in ‘len’, discarding the remain.

- If the count element of parameter 'data_info' is set to NON-ZERO and its element 'data' is not set to ZERO, then the driver shall send the data stored in element 'data' up to the number given by the count element to the card. In case of protocol T=0 the count element is identical with parameter P3, in case of protocol T=1 it is identical with parameter Lc.

- If neither send nor receive data is given by the function parameters, then,. in case of protocol T=0, the function shall only send the command header to the card followed by parameter P3 set to ZERO. In case of protocol T=1 only the header is sent.

Definition:

| Short Name | ID | Direction | Group |
| --- | --- | --- | --- |
| U16 simdrv_xch_apdu (U8 reader_id , T_SIMDRV_cmd_header cmd_header, T_SIMDRV_data_info data_info, T_SIMDRV_result_info * result_info) | InlineC | SIM->DRIVER | none |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| Reader Id | reader_id |  | 5.16 | U8 |
| Transparent command header | cmd_header |  | 5.11 | STRUCT |
| Send Data Buffer | data_info | PTR | 5.7 | STRUCT |
| Result Buffer | result_info | PTR | 5.5 | STRUCT |
| Status Words | sw1_2 |  | 5.8 | U16 |

History:

30-October-2003	FK	Initial. 
03-May-04	FDU	function renamed, parameters aligned to review report
10-June-04	FDU	reader id added

# Parameters

## Insert call-back function pointer

Description:

The insert call back function to which the pointer refers shall be executed by the driver after the PS has executed the simdrv_reset() command call, if and only if the activation of the inserted UICC was successful. It has a two parameters. One used to indicate to the PS whether UICC configuration characteristics are required by the driver and one which points to a structure, where the PS can store the retrieved configuration characteristics. (See chapter 6)

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| STRUCT | insert | function pointer | uicc |

Elements:

| Long Name | Short Name | Ref | Type |
| --- | --- | --- | --- |
| insert pointer | insert_ptr |  | U32 |

History:

21-February-02	FDU	Initial
03-May-04	FDU	function renamed, parameters aligned to review report

## Remove call-back function pointer

Description:

The remove call back function to which the pointer refers, shall be executed by the driver when the driver detects that the inserted UICC has been removed. Either this detection is due to missing reply to command or due to hardware removal detection.

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| STRUCT | remove | function pointer | uicc |

Elements:

| Long Name | Short Name | Ref | Type |
| --- | --- | --- | --- |
| remove pointer | remove_ptr |  | U32 |

History:

21Feb-02	FDU	Initial
03-May-04	FDU	function renamed, parameters aligned to review report

## SIM Card Information

Description:

Contains the result of the ATR procedure, which the SIM hardware driver initiates during its start up sequence.

Definition:

| Type | Short Name | CTRL | Comment | Group |
| --- | --- | --- | --- | --- |
| STRUCT | atr_string_info | PTR | SIM Card Info | uicc |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| SIM Card Answer to reset string | atr_string | [2..SIZE_ATR_INFO] |  | U8 |

History:

20-February-02	FDU	Initial
16-December-02	FDU	Structure changed for element to a struct including a length byte
03-May-04	FDU	function renamed, parameters aligned to review report

## Reset return value

Description:

The return value is used by the SIM hardware driver to indicate to the PS whether a card is inserted or not and if it is fully functioning.

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| U8 | reset_return_val | Reset return value | uicc |

Values:

| Value | C-Macro | Comment |
| --- | --- | --- |
| 0 | SIM_INSERTED | A SIM is inserted and ATR/PPS was successful |
| 1 | SIM_NOT_INSERTED | No SIM inserted (hardware detected) |
| 2 | INVALID_CARD | Card is not responding or gives unintelligible answers, communications time out. |
| 3 | ME_FAILURE | Power management related problems, e.g. voltage selection |
| 4 | IMPROPER_CALL_BACK | simdrv_register function called with pointer improperly set, e.g. NULL. |
| 5 | ME_READER_NOT_AVAILABLE | The reader requested is not available |

History:

20-February-02	FDU	Initial
05-July-02	FDU	Meaning of byte changed
03-May-04	FDU	function renamed, parameters aligned to review report	
28-June-04	FDU	Added latter two return values after desk check.

## Result Data

Description:

The SIM driver uses this to return the response data retrieved when issuing a command.

Definition:

| Type | Short Name | CTRL | Comment | Group |
| --- | --- | --- | --- | --- |
| STRUCT | result_info | PTR | Result Buffer | uicc |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| Length of expected data | len |  | 5.6 | U16 |
| Result byte | result | DYN[MIN_RESULT..MAX_RESULT] |  | U8 |

History:

20-February-02	FDU	Initial
16-December-02	FDU	Structure changed for element to a struct including a length byte
03-May-04	FDU	function renamed, parameters aligned to review report

## Length of Expected Data

Description:

Used to indicate maximum length, in bytes, of data which the ME expects the UICC to return, e.g. during a READ_RECORD or a READ_BINARY command. In cases where the size of the return data is undeterminable, the value shall be LENGTH_UNKNOWN (See chapter 6)

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| U16 | len | Maximum length of expected data | uicc |

Values:

| Value | C-Macro | Comment |
| --- | --- | --- |
| 0x00-0x100 |  | Range of Length of Data |
| 0xFFFF | LENGTH_UNKNOWN | Indicating that the length expected is unknown. |

History:

21-February-02	FDU	Initial
03-May-04	FDU	function renamed, parameters aligned to review report

## Send Data

Description:

The Update data element is used by the PS to transfer the data which it wants to update to the SIM hardware driver.

Definition:

| Type | Short Name | CTRL | Comment | Group |
| --- | --- | --- | --- | --- |
| STRUCT | data_info | PTR | Data element info | uicc |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| Data element | data | DYN[MIN_DATA_SIZE..MAX_DATA_SIZE] |  | U8 |

History:

20-February-02	FDU	Initial
16-December-02	FDU	Structure changed for element to a struct including a length byte
03-May-04	FDU	function renamed, parameters aligned to review report

## Status Words

Description:

After each command issued by the ME to the UICC, the SIM hardware driver shall return the two status bytes from the UICC. The PS interprets these value and acts upon them on its own. The possible values of the status word can be found in [GSM 11.11] and [ETSI TS 102 221]. They are returned in a U16, SW1 occupies bits 8..15 and SW2 occupies bits 0..7 of the value.
Additionally there are a couple of driver error codes which are returned in case of unrecoverable communication problems between ME and UICC. Before the driver returns any of these error codes, it shall retry the causing operation at least once.

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| U16 | sw1_2 | Status Words | uicc |

Values:

| Value | C-Macro | Comment |
| --- | --- | --- |
| 0x0001 | ERR_NOCARD | No SIM inserted (hardware detected) |
| 0x0002 | ERR_NOT_RESET | A reset has not been performed on the driver with the requested Id. |
| 0x0003 | ERR_ME_FAIL | Unrecoverable ME failure (for instance interrupt fails to occur) |
| 0x0004 | ERR_RETRY_FAIL | ME/SIM communication failed after certain retries, SIM reset required |
| 0x0005 | ERR_PARAM_WRONG | A driver function is called with invalid parameters |

History:

21-February-02	FDU	Initial
8-Dec-2003	FK	Revised
03-May-04	FDU	function renamed, parameters aligned to review report

## Class byte

Description:

When communicating with an UMTS Application, the Class Byte shall be set according to [ETSI 102 221, 10.1.2], either code 0x00 (standard ISO 7816-4 Class Byte) or code 0x80 (for additional instructions defined in ETSI 102 221) dependent on the type of the selected command. For a GSM application according to GSM 11.11 / 3GPP 51.011 the code 0xA0 is used for all instructions.

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| U8 | cla | Class Byte | uicc |

Values:

| Value | C-Macro | Comment |
| --- | --- | --- |
| 0xA0 | GSM_CLASS_BYTE | GSM Class byte according to [GSM 11.11 / 3GPP 51.011] |
| 0x80 | UMTS_CLASS_BYTE | UMTS Class byte according to [ETSI 102 221 10.1.2] |
| 0x00 | UICC_CLASS_BYTE | UICC Class byte according to [ISO 7816-4] |

History:

4-July-2002	FDU	Initial
5-Dec-2003	FK	Revised
03-May-04	FDU	function renamed, parameters aligned to review report

## SIM Instruction Code

Description:

The parameter defines the SIM instruction code for SIM access.

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| U8 | ins | instruction code | uicc |

Values:

| Value | C-Macro | Comment |
| --- | --- | --- |
| 0xA4 | INS_SELECT | Select Instruction |
| 0xF2 | INS_STATUS | Status Instruction |
| 0xB0 | INS_READ_BINARY | Read Binary Instruction |
| 0xD6 | INS_UPDATE_BINARY | Update Binary Instruction |
| 0xB2 | INS_READ_RECORD | Read Record Instruction |
| 0xDC | INS_UPDATE_RECORD | Update Record Instruction |
| 0xA2 | INS_SEEK | Seek Instruction |
| 0x32 | INS_INCREASE | Increase Instruction |
| 0x20 | INS_VERIFY_CHV | Verify Chv Instruction |
| 0x24 | INS_CHANGE_CHV | Change Chv Instruction |
| 0x26 | INS_DISABLE_CHV | Disable Chv Instruction |
| 0x28 | INS_ENABLE_CHV | Ensable Chv Instruction |
| 0x2C | INS_UNBLOCK_CHV | Unblock Chv Instruction |
| 0x04 | INS_INVALIDATE | Invalidate Instruction |
| 0x44 | INS_REHABILITATE | Rehabilitate Instruction |
| 0x88 | INS_AUTHENTICATE | Run GSM Algorithm (2/2.5G) / Authenticate (3G) Instruction |
| 0x10 | INS_TERMINAL_PROFILE | Terminal Profile Instruction |
| 0xC2 | INS_ENVELOPE | Envelope Instruction |
| 0x12 | INS_FETCH | Fetch Instruction |
| 0x14 | INS_TERMINAL_RESPONSE | Terminal Response Instruction |
| 0xC0 | INS_GET_RESPONSE | Get Response Instruction |

History:

24-April-2002	JK	Initial
01-May-03	KBS	SIM_ prefix removed due to autoprefixing
8-Dec-2003	FK	Incorporated from SAP SIM

## Command Header

Description:

A struct pointer to create a command header for sending transparent commands to the UICC

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| STRUCT | cmd_header | Transparent command header | uicc |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| Class Byte value | cla |  | 5.9 | U8 |
| Instruction Code | ins |  | 5.10 | U8 |
| Parameter P1 | p1 |  | 5.13 | U8 |
| Parameter P2 | p2 |  | 5.13 | U8 |

History:

30-October-03	FK	Initial

## Perform voltage selection

Description:

Used to indicate to the SIM reader driver whether or not a voltage selection  shall be performed or not. If set to REQ_VOLTAGE_SEL, the SIM reader driver does a regular reset of the card in which it performs the voltage selection procedure.

If set to OMIT_VOLTAGE_SEL and the card has been activated with a simdrv_reset() call prior to this, thus having a used voltage already, it shall reset the card using only the currently used voltage. This also applies for UICC Characteristics.

This is a special case, where the SIM application toolkit requests a reset of the card, see [3G 11.14, 6.4.7]

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| U8 | voltage_select | Perform Voltage Selection | uicc |

Values:

| Value | C-Macro | Comment |
| --- | --- | --- |
| 0 | REQ_VOLTAGE_SEL | The SIM reader driver shall perform voltage selection |
| 1 | OMIT_VOLTAGE_SEL | The SIM reader driver shall restart with the current voltage |

History:

03-May-04	FDU	Initial

## SIM Command Parameters

Description:

The parameters P1 and P2 are individually coded in relevance to what type of command is to be performed. Coding according to [3G 11.11] and [ETSI 102 221]

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| U8 | p1 | Parameter 1 of the SIM APDU | uicc |
| U8 | p2 | Parameter 2 of the SIM APDU | uicc |

History:

03-May-04	FDU	function renamed, parameters aligned to review report

## Configuration Characteristics Request

Description:

This parameter is used by the driver to indicate that the PS must retrieve UICC characteristics from the MF (‘3F00’) and deliver them to the driver using the provided pointer reference (see chapter 5.15). This will occur in situations were the inserted cards does not deliver e.g. preferred clock stop level in the ATR. If the value of the parameter is SIMDRV_OMIT_CONFIG_CHARACTERISTICS, the PS must never try to access data using the provided pointer.

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| U8 | config_requested | Configuration Characteristics requested | uicc |

Values:

| Value | C-Macro | Comment |
| --- | --- | --- |
| 0x00 | OMIT_CONFIG_CHARACTERISTICS | PS Shall not retrieve Configuration Characteristics |
| 0x01 | REQUEST_CONFIG_CHARACTERISTICS | PS Shall retrieve Configuration Characteristics |

History:

10-June-04	FDU	function renamed, parameters aligned to review report

## Configuration Characteristics

Description:

The PS shall deliver the configuration characteristics coded according to [ETSI 102 221, 11.1.1.4.6.1].

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| STRUCT | config_characteristics | Configuration Characteristics | uicc |

Elements:

| Long Name | Short Name | CTRL | Ref | Type |
| --- | --- | --- | --- | --- |
| UICC Characteristics | uicc_characteristics |  | 5.17 | U8 |

History:

10-June-04	FDU	function renamed, parameters aligned to review report

## Reader Id

Description:

Used to indicate to the driver which reader the currently command is issued towards.

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| U8 | reader_id | Reader Id | uicc |

Values:

| Value | C-Macro | Comment |
| --- | --- | --- |
| 0x01-MAX_READERS |  | Range of Id’s of readers |

History:

10-June-04	FDU	function renamed, parameters aligned to review report

## UICC Characteristics

Description:

Used to deliver the UICC Characteristics of the MF to the driver

Definition:

| Type | Short Name | Comment | Group |
| --- | --- | --- | --- |
| U8 | uicc_characteristics | UICC Characteristics | uicc |

History:

10-June-04	FDU	function renamed, parameters aligned to review report

/*

# MSCs

## Successful activation

Figure 2 - Initial communication, successful activation

Figure 2 shows the initial communication between the PS and the driver. When the PS is initialised it registers the driver with the simdrv_register function. In this function call the driver is handed the two function pointers for respectively insert and remove functions.

When the PS is activated or wants to reset the inserted UICC it uses the simdrv_reset function to request  the driver to attempt to reset the inserted UICC.

Figure 3 - Initial communication, successful activation, configuration information required.

If the reset was successful, the driver uses the insert function pointer to execute the PS’s initialisation procedure (an example can be found on Figure 2).

In cases where the inserted UICC does not return any UICC characteristics, e.g. clock stop preferences as a part of the ATR, the driver will request these settings from the PS. This sequence will be that shown on Figure 3.

## Unsuccessful activation

If the reset fails, the driver returns a error code other than SIM_INSERTED and does not execute the insert function pointer.(an example can be found on Figure 4) .

Figure 4 - Unsuccessful initial communication

## Power off

At any time, the PS can perform a power off on the driver. This power off shall be performed according to [3G 11.11] and [ETSI102 221].

Figure 5 - Powering off

## Issuing commands

The PS uses the simdrv_xch_apdu() function to issue commands towards the reader with the Id given.

If the reader with the given Id has not been reset, a proper error code shall be returned and the driver shall not issue the requested command.

Figure 6 - Unsuccessful communication, missing reset

## Command  Types

The [ETSI 102 221] describes 4 different types of commands. As the access protocol is handled autonomously by the PS, one more case appears, case 5.

- No sending data, no receiving data.

- Sending data, no receiving data.

- No sending data, receiving data.

- Sending data, receiving data

- Sending/No sending of data, receiving data of unknown length

Each of the five cases is exemplified below.

Figure 7 - Case 1 command example

Figure 8 - Case 2 command example

Figure 9 - Case 3 command example

Figure 10 - Case 4 command example

Figure 11 - Case 5 command example

*/