Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document – Confidential

GPRS General Packet Radio Services

Message Sequence Charts

MAC

(Edit via File/Properties/Summary!)

| Document Number: | 8441.200.99.008 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.9 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1999-Mar-30 |
| Last changed: | 2003-May-19 by XGUTTEFE |
| File Name: | mac.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1999-Mar-30 | id, can |  | 0.1 |  | 1 |
| 1999-Jun-10 | can |  | 0.2 |  | 2 |
| 1999-Jun-10 | id |  | 0.3 |  | 3 |
| 1999-Jun-30 | id |  | 0.4 |  | 4 |
| 1999-Jul-21 | id |  | 0.5 |  | 5 |
| 2000-Mar-10 | sl |  | 0.6 |  | 6 |
| 2000-Jun-13 | ID |  | 0.7 |  | 7 |
| 2001-Oct-10 | id |  | 0.8 |  | 8 |
| 2003-May-19 | XGUTTEFE |  | 0.9 | Draft |  |

Notes:

- Initial version

- Changes in 04.60 Version 6.3.0 included

- New document header included

- LLC part, failure indication added

- GMM part added

- Update according to S2

- Update according to S3: (TBF establishment on CCCH, Paging on CCH: RR connection establishment and packet transfer for downlink)

- Remove GMMRR_ACCESS_BARRED_IND

Table of Contents

1.1	References	7

1.2	Abbreviations	9

1.3	Terms	12

2	Overview	12

2.1	GRR (RLC/MAC) – Radio Link Control/Medium Access Control	13

2.2	LLC – Logical Link Control	13

2.3	GMM – GPRS Mobility Management	13

2.4	SM – Session Management	13

2.5	SNDCP - Subnetwork Dependant Convergence Protocol	13

2.6	GACI – GPRS Application Control Interface	13

2.7	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	13

2.8	TOM – Tunnelling of Messages	13

3	Protocol	14

3.1	Configuration	14

3.1.1	TLLI assign	14

3.1.2	Enable/disable GPRS service	14

3.1.3	GMM state indication	14

3.2	TBF establishment initiated by the MS on PCCCH	16

3.2.1	Uplink TBF establishment initiated by the MS on PCCCH	16

3.2.2	Receiving packet paging request during packet access procedure	17

3.2.3	Stop the uplink TBF establishment	18

3.2.4	Packet assignment procedure (The MS has not indicated a measurement report)	19

3.2.5	Packet assignment procedure (The MS has indicated a measurement report)	22

3.2.6	Receipt of packet queuing notification	24

3.2.7	Packet polling procedure	26

3.2.8	Receipt of packet access reject message	28

3.2.9	Contention resolution completion/failure at one phase access	30

3.2.10	Timing Advance	31

3.2.11	TBF establishment using two phase access	32

3.2.12	Contention resolution at two phase access	33

3.2.13	Abnormal cases (muss noch gemacht werden)	34

3.3	Neighbour Cell Syncronisation	34

3.3.1	Defintion of BCCH allocation (BA_GPRS)	35

3.3.2	Synchronisation Request of ncells	35

3.3.3	Conformation of Synchronisation to  ncells	35

3.4	Performing measurements	36

3.4.1	Measurements on PCCCH	36

3.4.2	Measurements on CCCH	44

3.5	Cell Re-selection procedure in packet idle mode (PBCCH exists)	44

3.5.1	Read the pbcch of the new cell	45

3.5.2	Receiving of PSIs in the new cell	45

3.5.3	Reselect new cell in packet idle mode	46

3.5.4	Packet Access in the new Cell(must be modified  only in packet transfer mode)	48

3.6	TBF establishment initiated by the network on PCCCH	50

3.6.1	Packet downlink assignment	50

3.6.2	Packet polling procedure after receiving a packet downlink assignment	51

3.7	Procedures for sending measurement reports in packet idle mode	52

3.7.1	Measurement report sending initiated on PCCCH	52

3.7.2	Measurement report sending initiated on CCCH	54

3.8	Measurement order procedures in packet idle mode	55

3.8.1	Measurement order procedures initiated on PCCCH	55

3.8.2	Measurement order procedures initiated on CCCH	56

3.9	Measurement report in packet transfer mode	56

3.10	Measurement Order procedures in packet transfer mode	56

3.11	Cell change order procedure in packet idle mode	57

3.11.1	Cell change order procedure initiated on PCCCH	57

3.11.2	Cell change order procedure initiated on CCCH	58

3.12	TBF establishment initiated by the MS on CCCH	59

3.12.1	Mobile originated packet access –B	59

3.12.2	Mobile originated packet access –B	60

3.12.3	Mobile originated packet access –B	62

3.12.4	Mobile originated packet access –B	63

3.12.5	Mobile originated packet access –B	65

3.12.6	Mobile originated packet access –B	67

3.12.7	Mobile terminated downlink assignment –B	68

3.12.8	Mobile terminated downlink assignment –B	69

3.12.9	Abnormal cases	70

3.13	Packet downlink assignment procedure on CCCH	71

3.13.1	Packet downlink assignment completion	72

3.13.2	Packet downlink assignment abnormal cases	72

3.13.3	Sending an RLC/MAC control message: single block packet downlink assignment procedure	72

3.14	MAC procedures in uplink packet transfer mode	73

3.14.1	Resource Reallocation (Uplink)	73

3.14.2	Resource Reallocation for new LLC PDUs with different RLC mode and higher priority(Uplink)	74

3.14.3	On expiry if Timer T3168	75

3.14.4	Reaction of the network to the resource reallocation request	76

3.14.5	Abnormal Cases	77

3.14.6	Establishment of Downlink TBF	78

3.14.7	Abnormal cases during downlink assignment	79

3.14.8	Reallocation of the fixed Allocation	80

3.14.9	On expiry if Timer T3188	81

3.14.10	Network initiated release of uplink TBF	82

3.15	MAC procedures in downlink packet transfer mode	83

3.15.1	Downlink assignment	83

3.15.2	Polling for Downlink ACK/NACK	84

3.15.3	Suspending the Downlink TBF !!! REMOVED !!!	84

3.15.4	Resource Reassignment for Downlink TBF	84

3.15.5	Establishment of uplink TBF	85

3.15.6	Receiving of uplink resources	86

3.15.7	Receiving of packet access reject	87

3.15.8	On expiry if Timer T3168	87

3.15.9	Abnormal cases	88

3.15.10	Network initiated downlink TBF release	88

3.16	Packet PDCH Release	89

3.16.1	On reception of PACKET PDCH RELEASE	89

3.17	Cell change procedure in packet transfer mode	90

3.17.1	On reception of PACKET CELL CHANGE ORDER	90

3.17.2	Network controlled cell reselection completion	90

3.17.3	Failure on the new cell or Timeout of T3174	91

3.17.4	Timeout of Timer T3176	92

3.18	Packet Control Acknowledgement	92

3.19	Receiving a PACKET PAGING REQUEST	93

3.19.1	On PCCCH_GROUP	93

3.19.2	On PACCH	93

3.20	Cell indication	94

3.21	Abnormal Release	95

3.22	System information on PBCCH	96

3.22.1	Supervision of PBCCH_CHANGE_MARK and update of PBCCH information	97

3.22.2	Replacement of PBCCH	98

3.22.3	PSI1 reception failure	99

3.23	System information on BCCH	99

3.23.1	Supervision of BCCH_CHANGE_MARK and update of BCCH information	99

3.23.2	Establishment of PBCCH	100

3.23.3	SI13 reception failure	100

3.24	Page mode procedures on PCCCH	100

3.25	Page mode procedures on CCCH	102

3.25.1	Paging procedure for RR connection establishment	102

3.25.2	Paging procedure for downlink packet transfer	104

Appendices	105

A.	Acronyms	105

B.	Glossary	105

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 05.02 version 8.0.0 Release 1999
Digital cellular telecommunications system (Phase 2+);
Multiplexing and multiple access on the radio path

[2]	GSM 04.60 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station (MS) - Base Station System (BSS) interface;
Radio Link Control/ Medium Access Control (RLC/MAC) protocol

[3]	GSM 04.08 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface layer 3 specification

[4]	GSM 03.64 version 6.1.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Overall description of the GPRS radio interface; Stage 2

[5]	GSM 03.60 version 6.3.1 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Service description; Stage 2

[6]	GSM 04.07 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface signalling layer 3; General aspects

[7]	GSM 04.64 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station - Serving GPRS Support Node (MS-SGSN)
Logical Link Control (LLC) layer specification

[8]	GSM 05.08 version 6.4.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem link control

[9]	GSM 05.10 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem synchronization

[10]	GSM 03.20 TS 100 929: July 1998 (GSM 03.20 version 6.0.1)
Security related network functions, ETSI

[11]	Draft GSM 03.22: August 1998 (GSM 03.22 version 6.1.0)
Functions related to Mobile Station (MS) in idle mode and group receive mode, ETSI

[12]	GSM 04.65 V6.3.0: Subnetwork Dependant Convergence Protocol

ETSI, March 1999

[13]	ITU-T V42bis ITU-T, Recommendation V.42 bis 1990

[14]	GSM 09.60 GPRS Tunneling Protocol (GTP) across the Gn and Gp Interface

[15]	RFC 1661 IETF STD 51 July 1994
The Point-to-Point Protocol (PPP)

[16]	RFC 1662 IETF STD 51 July 1994
PPP in HDLC-like Framing

[17]	RFC 1570 January 1994
PPP LCP Extensions

[18]	RFC 1989 August 1996
PPP Link Quality Monitoring

[19]	RFC 1332 May 1992
The PPP Internet Protocol Control Protocol (IPCP)

[20]	RFC 1877 December 1995
PPP IPCP Extensions for Name Server Addresses

[21]	RFC 2153 May 1997
PPP Vendor Extensions

[22]	RFC 1334 October 1992
PPP Authentication Protocols (for Password Authentication Protocol only)

[23]	RFC 1994 August 1996
PPP Challenge Handshake Authentication Protocol (CHAP)

[24]	TIA/EIA-136-370
Packet-Data Services – Enhanced General Packet Radio for TIA/EIA-136 (EGPRS-136) - Overview, Telecommunications Industry Association

[25]	TIA/EIA-136-376
Packet-Data Services – EGPRS-136 Mobility Management, Telecommunications Industry Association

[26]	TIA/EIA-136-972
Packet-Data Services – Stage 2 Description, Telecommunications Industry Association

## Abbreviations

ACI	Application Control Interface
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CCI	Compression and Ciphering Interface
CHAP	Challenge Handshake Authentication Protocol
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DCOMP	Identifier of the user data compression algorithm used for the N-DPU
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

E	Extension bit
EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GACI	GPRS Application Control Interface
GMM	GPRS Mobility Management
GP	Guard Period
GRR	GPRS RR
GSM	Global System for Mobile Communication

HDLC	High-level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
IP	Internet Protocol
IPCP	Internet Protocol Control Protocol 
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Ciphering Key

L	Length Indicator
LAI	Location Area Information
LCP 	Link Control Protocol 
LISR	Low level Interrupt Service Routine
LLC	Logical Link Control
LPD	Link Protocol Discriminator
LQM	Link Quality Monitoring

M	More bit used to indicate the last segment of N-DPU
MAC	Medium Access Control
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MT	Mobile Termination

N(R)	Receive Number
N(S)	Send Number
NC	Network Control
NCC	National Colour Code
NCP 	Network Control Protocol
NECI	New Establishment Causes included
N-PDU	Network Protocol Data Unit
NSAPI	Network Layer Service Access Point Identifier

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PACCH	Packet Associated Control Channel
PAP	Password Authentication Protocol
PBCCH	Packet BCCH
PCCCH	Packet CCCH
PCOMP	Identifier of the protocol control information compression algorithm used for the N-DPU
PDCH	Packet Data Channel
PDP	Packet Data Protocol e.g. IP or X.25
PDTCH	Packet Data Traffic Channel
PRACH	Packet RACH
PSI	Packet System Information
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Data Unit
PL	Physical Layer
PLMN	Public Land Mobile Network
PPC	Packet Physical Convergence
PPP 	Point-to-Point Protocol
PTP	Point to Point

QoS	Quality of Service

RACH	Random Access Channel
REJ	Reject Frame
RLC	Radio Link Control
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Stand alone Dedicated Control Channel
SDU	Service Data Unit
SGSN	Serving GPRS Support Node
SIM	Subscriber Identity Module
SM	Session Management
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SNDCP	Subnetwork Dependant Convergence Protocol
SNSM	SNDCP-SM
SS	Supplementary Services

TAP	Test Application Program
TBF	Temporary Block Flow
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TCP	Transmission Control Protocol
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TFI	Temporary Flow Identifier
TLLI	Temporary Logical Link Identifier
TMSI	Temporary Mobile Subscriber Identity
TOM	Tunnelling of Messages
TQI	Temporary Queuing Identifier

UA	Unnumbered Acknowledgement Frame
UART	Universal Asynchronous Receiver Transmitter
UI	Unnumbered Information Frame
USF	Uplink State Flag

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visited Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for fax and data transmission consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity. The entity, which is described in this document, is coloured grey in the following figure :

- Figure 2-1: Architecture of the GSM/GPRS protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the GPRS protocol stack are:

## GRR (RLC/MAC) – Radio Link Control/Medium Access Control

This layer contains two functions: The Radio Link Control function provides a radio-solution-dependent reliable link. The Medium Access Control function controls the access signalling (request and grant) procedures for the radio channel, and the mapping of LLC frames onto the GSM physical channel.

## LLC – Logical Link Control

The LLC entity provides multiple highly reliable logical links for asynchronous data transfer between the MS and the network. It supports variable-length information frames, acknowledged and unacknowledged data transfer, flow and sequence control, error detection and recovery, notification of unrecoverable errors, user identity confidentiality, and ciphering of user and signaling data.

## GMM – GPRS Mobility Management

The GMM entity provides procedures for the mobility of the MS, such as informing the network of its present location, and user identity confidentiality. It manages the GMM context (attach, detach, routing area updating), supports security functions such as authentication of user and MS, controls ciphering of data, and initiates the response to paging messages.

## SM – Session Management

The main function of the session management (SM) is to support PDP context handling of the user terminal. Session Management activates, modifies and deletes the contexts for packet data protocols (PDP). Session Management services are provided at the SMREG-SAP and the SNSM-SAP for anonymous and non-anonymous access. The non-anonymous and anonymous access procedures for PDP context activation and PDP context deactivation are available at the SMREG-SAP. In addition there exists a PDP context modification for non-anonymous PDP contexts.

## SNDCP - Subnetwork Dependant Convergence Protocol

- SNDCP carries out all functions related to transfer of Network layer Protocol Data Units (N-PDUs) over GPRS in a transparent way. SNDCP helps to improve channel efficiency by means of compression techniques. The set of protocol entities above SNDCP consists of commonly used network protocols. They all use the same SNDCP entity, which then performs multiplexing of data coming from different sources to be sent using the service provided by the LLC layer.

## GACI – GPRS Application Control Interface

The GACI is the GPRS extension of the ACI. It is specified in GSM 07.07 and 07.60. It is responsible for processing of the GPRS related AT Commands to setup, activate and deactivate the PDP context parameter. It also provides functionality for the interworking between GMM/SM/SNDCP and a packet oriented protocol like PPP.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the GPRS. The driver has to be able to perform flow control.

## TOM – Tunnelling of Messages

The TOM entity is present if and only if HS136 is supported (the feature flag FF_HS136 is enabled).

The main function of TOM is to tunnel non-GSM signalling messages between the MS and the SGSN. The only non-GSM signalling which is currently supported by TOM is for the EGPRS-136 system (according to TIA/EIA-136-376). Data transfer in both uplink and downlink direction is possible. Two different priorities (high, low) of signalling data transfer are supported. TOM uses the unacknowledged mode of LLC and the acknowledged mode of GRR (RLC/MAC).

# Protocol

## Configuration

### TLLI assign

GMM                                  GRR

|                                    |

| GMMRR_ASSIGN_REQ                   |

*===================================>*

|                                  (GRR 1)

|                                    |

(GRR 1)

GMM assigns a new TLLI and GRR stores this value.

### Enable/disable GPRS service

a)GPRS enable

GMM                                  GRR

|                                    |

| GMMRR_ENABLE_REQ                   |

*===================================>*

|                                  (GRR 1)

|                                    |

(GRR 1)

GMM enables GPRS service, GRR saves this state.

b)GPRS enable

GMM                                  GRR

|                                    |

| GMMRR_DISABLE_REQ                  |

*===================================>*

|                                  (GRR 1)

|                                    |

(GRR 1)

GMM disables GPRS service, GRR saves this state.

### GMM state indication

a)GMM is in READY state

GMM                                  GRR

|                                    |

| GMMRR_CU_REQ                    |

*===================================>*

|                                  (GRR 1)

|                                    |

(GRR 1)

GMM enters state READY. The cell updating procedure is no longer blocked.

b)GMM enters the STANDBY state

GMM                                  GRR

|                                    |

| GMMRR_NO_CU_REQ                  |

*===================================>*

|                                  (GRR 1)

|                                    |

(GRR 1)

GMM enters state STANDBY. No cell updating procedure is performed.

## TBF establishment initiated by the MS on PCCCH

### Uplink TBF establishment initiated by the MS on PCCCH

LLC                GRR              RLC                     MAC                PL

|                  |                |                       |                  |

| GRR_DATA_REQ     |                |                       |                  |

or

| GRR_UNITDATA_REQ |                |                       |                  |

*=================>*                |                       |                  |

|                (GRR 1)            |                       |                  |

|                  |                |                       |                  |

a) Access to the network is not allowed

|                  |                |                       |                  |

| GRR_STATUS_IND   |                |                       |                  |

*<=================*                |                       |                  |

(LLC 1)             |                |                       |                  |

|                  |                |                       |                  |

b) Access to the network is allowed

|                  |                |                       |                  |

|                  |                |             MPHP_RA_REQ                  |

|                  |                |      (PACKET CHANNEL REQUEST)            |

|                  *==========================================================>*

|                  |                |                       |             (PL 1)

|                  |                |     MPHP_RA_CON       |                  |

|                  *<==========================================================*

|                (GRR 2)            |                       |                  |

|                  |                |                       |                  |

(GRR 1)

LLC wants to transmit LLC PDU. GRR checks whether the access to the network is allowed or not.

(LLC 1)

If the access to the network is not allowed, the GRR sends GRR_STATUS_IND which contains a cause field: Network access not allowed.

(PL 1)

If the upper layer wants to transmit LLC PDUs, then the GRR composes PACKET CHANNEL REQUEST message and posts it to the PL. The PL sends this message on PRACH corresponding to its PCCCH_GROUP. The GRR can send maximum MAX_RETRANS + 1 times PACKET CHANNEL REQUEST. The GRR set the Retry (R) bit into the appropriate value. This bit is used in subsequent MAC headers. The format of the PACKET CHANNEL REQUEST is either 8 bit or 11 bit. It is broadcast on PBCCH. For the next channel request attempt the GRR shall take PERSISTENCE_LEVEL into account. The purpose of the packet access shall be indicated in the message (Page Response, Cell Update, Mobility Management, Single Block without TBF).

(GRR 2)

The PL sends a confirmation to the GRR to indicate that the PACKET CHANNEL REQUEST message was sent.

Requirements:

<R.MAC.TBFPCCCH.M.00x>, <R.MAC.P_ACCESS.M.001-4, <R.MAC.P_ACCESS.M.009-13>, <R.MAC.ACCS_PER.M.00x>

### Receiving packet paging request during packet access procedure

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                       |          MPHP_DATA_IND           |

|                   | 	                      |    (PACKET PAGING REQUEST)       |

|                   |                       *<=================================*

|                   |                    (MAC 1)

a) Message indicates an RR connection establishment

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

b) Message indicates a non-RR connection establishment

(GRR 2)              |                       |                                  |

|                   |                       |                                  |

(MAC 1)

The MAC receives a PACKET PAGING REQUEST.

(GRR 1)

Class C MS ignores this message but decodes the PERSISTENCE_LEVEL, if indicated in the message.. Class A and B MSs shall respond to this message. Class B MS may aborts the packet access procedure and indicates this to RR.

(GRR 2)

This message will be ignored by all MS classes.

Requirements:

<R.MAC.P_ACCESS.M.005-8>

### Stop the uplink TBF establishment

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   | MPHP_TBF_RELEASE_REQ  |                                  |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                   | MPHP_TBF_RELEASE_CON  |                                  |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

(PL 1)

The GRR stops to send message on PRACH.

(GRR 1)

The PL confirms the received message.

Requirements:

defined in S921

### Packet assignment procedure (The MS has not indicated a measurement report)

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                       |          MPHP_DATA_IND           |

|                   |                       |    (PACKET UPLINK ASSIGNMENT)    |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

Stop T3170, if running                      |                                  |

|                   |     MPHP_RA_STOP_REQ  |                                  |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                   |     MPHP_RA_STOP_CON  |                                  |

*<=============================================================================*

(GRR 1)              |                       |                                  |

a) Message contains Single Block allocation structure

|                   |                       |       MPHP_SINGLE_BLOCK_REQ      |

|                   |                       |     (PACKET RESOURCE REQUEST)    |

|                   |                       *==================================>

|                   |                       |                             (PL 2)

|                   |                       |       MPHP_SINGLE_BLOCK_CON      |

|                   |                       *<=================================*

|                   |                    (MAC 2)                               |

b) The MS detects an invalid Frequency parameters IE in the assignment message

|                   |                       |                                  |

|                   |     MPHP_SCELL_PBCCH_REQ  (partial acquisition)          |

*=============================================================================>*

|                   |                       |                             (PL 3)

|                   |                 MPHP_DATA_IND                            |

|                   |             (PACKET SYSTEM INFO)                         |

*<=============================================================================*

(GRR 2)              |                       |                                  |

....

....

|                   |                 MPHP_DATA_IND                            |

|                   |             (PACKET SYSTEM INFO)                         |

*<=============================================================================*

(GRR 3)              |                       |                                  |

|                   |    MPHP_SCELL_PBCCH_STOP_REQ                             |

*=============================================================================>*

|                   |                       |                             (PL 4)

|                   |    MPHP_SCELL_PBCCH_STOP_CON                             |

*<=============================================================================*

(GRR 4)              |                       |                                  |

c) Message is received following a packet queuing notification

(GRR 5)              |                       |                                  |

Stop T3162          |                       |                                  |

d) Message contains dynamic or fixed allocation structure

|                   |                      (MAC 3)                             |

e) TBF starting time indicated in the message

(GRR 6)              |                       |                                  |

|                   |                       |

(PL 1)

The PL stops to sending channel requests.

(GRR 1)

The PL confirms that the sending of channel requests was stopped.

(MAC 1)

The PL sends the received PACKET UPLINK ASSIGNMENT message to MAC.

(PL 2)

The MAC sends PACKET RESOURCE REQUEST to start the two phase access. This message is sent on the channel which is indicated via Single Block Allocation structure.

(MAC 2)

The PL confirms that the PACKET RESOURCE REQUEST was sent.

(PL 3)

If the MS detects an invalid frequency parameters IE in the assignment message, the MS aborts the procedure, if required initiates a partial acquisition of PBCCH or BCCH and may reinitiate this procedure.

(GRR 2-3)

The PL indicates the read packet system information to the GRR.

(PL 4)

The GRR stops to read the packet system information.

(GRR 4)

The PL confirms the stop request.

(GRR 5)

The GRR stops timer T3162 and switches to the assigned PDCHs.

(MAC 3)

The MAC follows the one phase access procedure.

(GRR 6)

The GRR shall monitor PCCCH until the point in time denoted by the TBF starting time. Thereafter it shall switch to the assigned PDCHs, start timer T3164 and proceed with the contention resolution. While monitoring the PCCCH, if the MAC receives more than one PACKET UPLINK ASIGNMENT message, it shall act on the new one. This point is also valid for the situation on which the GRR has indicated a measurement report in the channel request message.

Requirements:

<R.MAC.REC_CHRQ.M.0xx>, <R.MAC.RC_RS_RQ.M.007>

### Packet assignment procedure (The MS has indicated a measurement report)

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                       |          MPHP_DATA_IND           |

|                   |                       |    (PACKET UPLINK ASSIGNMENT)    |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

Stop T3170, if running                       |                                  |

|                   |     MPHP_RA_STOP_REQ  |                                  |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                   |     MPHP_RA_STOP_CON  |                                  |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |       MPHP_SINGLE_BLOCK_REQ      |

|                   |                       |   (PACKET MEASUREMENT REPORT)    |

|                   |                       *==================================>

|                   |                       |                             (PL 2)

|                   |                       |       MPHP_SINGLE_BLOCK_CON      |

|                   |                       *<=================================*

|                   |                    (MAC 2)                               |

|                   |                       |                                  |

(PL 1)

The PL stops to sending channel requests.

(GRR 1)

The PL confirms that the sending of channel requests was stopped.

(MAC 1)

The MAC receives the PACKET UPLINK ASSIGNMENT message. This message contains the PDCH on which the measurement report should be send. The GRR stops the timer T3170.

(PL 2)

The MAC composes the PACKET MEASUREMENT REPORT structure and sends it on the assigned PDCH.

(MAC 2)

The PL confirms the sent of the message.

Requirements:

<R.MAC.REC_CHRQ.M.006>, <R.MAC.REC_CHRQ.M.014>

### Receipt of packet queuing notification

LLC             GRR                 RLC     MAC                                PL

|               |                   |       |                                  |

|               |                   |       |           MPHP_DATA_IND          |

|               |                   |       |   (PACKET QUEUING NOTIFICATION)  |

|               |                   |       *<=================================*

|               |                   |     (MAC 1)                              |

|               |                   |       |                                  |

a) Message corresponds to one of its last 3 packet channel requests

|               |                   |       |                                  |

|           Stop T3170              |       |                                  |

|           Start T3162             |       |                                  |

|               |    MPHP_RA_STOP_REQ (optional, can be implicit)              |

*=============================================================================>*

|               |                           |                             (PL 1)

|               |                   |       |           MPHP_DATA_IND          |

|               |                   |       |   (PACKET POLLING REQUEST)       |

|               |                   |       *<=================================*

|               |                   |     (MAC 2)                              |

|               |         MPHP_POLLING_REQ  |                                  |

*               |============================================================>*

|               |                   |       |                             (PL 2)

|               |               MPHP_POLLING_IND                               |

|               *<=============================================================*

|             (GRR 1)               |       |                                  |

b) Message is received while waiting for TBF starting time

|             (GRR 2)               |       |                                  |

|               |                   |       |                                  |

- C) Timer T3162 expires

| GRR_STATUS_IND|                   |       |                                  |

*<==============*                   |       |                                  |

(LLC 1)          |                   |       |                                  |

d) Reception of PACKET UPLINK ASSIGNMENT

|               |                   |       |           MPHP_DATA_IND          |

|               |                   |       |   (PACKET UPLINK ASSIGNMENT)     |

|               |                   |       *<=================================*

|               |                   |     (MAC 3)                              |

(MAC 1)

The MAC receives the PACKET QUEUING NOTIFICATION message.

(PL 1)

The GRR stops the timer T3170 if running and starts the timer T3162, and stop sending PACKET CHANNEL REQUEST messages. The GRR shall continue to listen to the PBCCH and PCCCH. If the timer T3162 expires the packet access procedure shall be aborted and a packet access failure is indicated to the upper layer (see LLC 1).

(MAC 2)

The MAC deliver a PACKET CONTROL ACKNOWLEDGEMENT within a MPHP_POLLING_RESPONSE_REQUESTS to PL.

(PL 2)

PL transmits PACKET CONTROL ACKNOWLEDGEMENT.

(GRR 1)

GRR restarts T3162.

(GRR 2)

The GRR ignores this message.

(MAC 2)

The PL indicates a PACKET POLLING REQUEST message. The timer T3162 should be restarted.

(LLC 1)

GRR sends to the LLC packet access failure indication via GRR_STATUS_IND. Cause field contains: Failure during packet access procedure, e.g. T3162 expired.

(MAC 3)

After reception of a PACKET UPLINK ASSIGNMENT the packet queueing procedure is finished, T3162 shall be stopped, T3164 started and GRR a should continue with TBF assignement procedure.

Requirements:

<R.MAC.ACCS_QUE.M.00X>

### Packet polling procedure

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                       |          MPHP_DATA_IND           |

|                   |                       |    (PACKET POLLING REQUEST)      |

|                   |                       *<=================================*

|                   |                     (MAC 1)                              |

a)The MS sends the PACKET CONTROL ACKNOWLEDGEMENT message as an RLC/MAC block. That means, the RRBP field in the MAC Header of the PACKET POLLING REQUEST message.

|                   |                       |       MPHP_SINGLE_BLOCK_REQ      |

|                   |                       | (PACKET CONTROL ACKNOWLEDGEMENT) |

|                   |                       *=================================>*

|                   |                       |                             (PL 1)

|                   |                       |       MPHP_SINGLE_BLOCK_CON      |

|                   |                       *<================================*|

|                   |                     (MAC 2)                              |

b)The MS sends the PACKET CONTROL ACKNOWLEDGEMENT message as 4 access bursts.

|                   |             MPHP_RA_REQ                                  |

|                   |   (PACKET CONTROL ACKNOWLEDGEMENT)                       |

*=============================================================================>*

|                   |                       |                             (PL 2)

|                   |     MPHP_RA_CON_REQ   |                                  |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |             MPHP_RA_REQ                                  |

|                   |   (PACKET CONTROL ACKNOWLEDGEMENT)                       |

*=============================================================================>*

|                   |                       |                             (PL 3)

|                   |     MPHP_RA_CON_REQ   |                                  |

*<=============================================================================*

(GRR 2)              |                       |                                  |

|                   |             MPHP_RA_REQ                                  |

|                   |   (PACKET CONTROL ACKNOWLEDGEMENT)                       |

*=============================================================================>*

|                   |                       |                             (PL 4)

|                   |     MPHP_RA_CON_REQ   |                                  |

*<=============================================================================*

(GRR 3)              |                       |                                  |

|                   |             MPHP_RA_REQ                                  |

|                   |   (PACKET CONTROL ACKNOWLEDGEMENT)                       |

*=============================================================================>*

|                   |                       |                             (PL 5)

|                   |     MPHP_RA_CON_REQ   |                                  |

*<=============================================================================*

(GRR 4)              |                       |                                  |

|                   |                       |                                  |

(MAC 1)

The PL indicates the PACKET POLLING REQUEST to the MAC. The timer T3162 should be restarted.

(PL 1)

If the CONTROL_ACK_TYPE (broadcast in PSI1 or (P)SI 13) indicates RLC/MAC control block, then the PACKET CONTROL ACKNOWLEDGEMENT message is sent within a RLC/MAC block. The MPHP_SINGLE_BLOCK_REQ is used to transfer the block.

(MAC 2)

The PL confirms that the single block.

(PL 2-5)

If the CONTROL_ACK_TYPE indicates 4 access bursts, the PACKET CONTROL ACKNOWLEDGEMENT message is sent as random access burst.

(GRR 1-4)

The PL confirms the random access request.

Requirements:

<R.MAC.P_POLL.M.003>

### Receipt of packet access reject message

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                       |          MPHP_DATA_IND           |

|                   |                       |     (PACKET ACCESS REJECT)       |

|                   |                       *<=================================*

|                   |                     (MAC 1)                              |

|                   |                       |                                  |

a) Message with WAIT_INDICATION field        |                                  |

(GRR 1)              |                       |                                  |

a.1) Additional packet access reject message was received

|                   |                       |          MPHP_DATA_IND           |

|                   |                       |     (PACKET ACCESS REJECT)       |

|                   |                       *<=================================*

|                   |                     (MAC 2)                              |

|                   |                       |                                  |

a.2) Packet uplink assignment was received which corresponds to one of the last 3 packet channel request messages.

|                   |                       |          MPHP_DATA_IND           |

|                   |                       |   (PACKET UPLINK ASSIGNMENT)     |

|                   |                       *<=================================*

|                   |                     (MAC 3)                              |

|                   |                       |                                  |

a.3) Timer T3162 expires.

|                   |                     (MAC 4)                              |

|                   |                       |                                  |

a.4) Before timer T3172 expires.

|                   |                     (MAC 5)                              |

|                   |                       |          MPHP_DATA_IND           |

|                   |                       |    (PACKET PAGING REQUEST)       |

|                   |                       *<=================================*

|                   |                     (MAC 6)                              |

|                   |                       |                                  |

a 5) Before timer T3162 expires, MAC receives erroneous PACKET UPLINK ASSIGNMENT message.

|                   |                     (MAC 7)                              |

|                   |                       |          MPHP_DATA_IND           |

|                   |                       |    (PACKET UPLINK ASSIGNMENT)    |

|                   |                       *<=================================*

b) Message without WAIT_INDICATION field     |                                  |

(GRR 2)              |                       |                                  |

|                   |                       |                                  |

(MAC 1)

The MAC receives the PACKET ACCESS REJECT message.

(GRR 1)

The PL indicates the PACKET ACCESS REJECT to the GRR. The message contains wait indication field. The GRR starts T3162 if it has not already been started, and starts T3172. The GRR shall listen to the PCCCH until timer T3162 expires

(MAC 2)

The MAC receives additional PACKET ACCESS REJECT message while waiting. The MAC ignores these messages.

(MAC 3)

The timers T3172 and T3162 should be stopped, and the same procedure as for receiving a PACKET UPLINK ASSIGNMENT message following a PACKET QUEUEING NOTIFICATION message should be followed.

(MAC 4)

If the timer T3162 expires the MS shall return to packet idle mode.

(MAC 5)

Before timer T3172 expires, it is not allowed to make a new attempt for packet access in the same cell, but may attempt packet access in a different cell after successful cell reselection. MS class A or B may attempt to enter dedicated mode in the same cell before T3172 expires.

(MAC 6)

The MAC shall ignore the received PACKET PAGING REQUEST messages except paging request to trigger RR connection establishment.

(MAC 7)

The MAC receives an erroneous PACKET UPLINK ASSIGNMENT message, reinitiate packet access procedure unless it has already been repeated 4 times, the MS shall return to packet idle mode.

(GRR 2)

The PL indicates the PACKET ACCESS REJECT to the GRR. The message does not contain wait indication field. Stop T3162, if running and return to packet idle mode. GRR shall decode the PACKET SYSTEM INFORMATION messages, if broadcast, or the SYSTEM INFORMATION messages before re-attempting the uplink TBF establishment.

Requirements:

<R.MAC.ACCS_REJ.M.00x>, <R.MAC.RC_RS_RQ.M.003-5>

### Contention resolution completion/failure at one phase access

LLC                 GRR                     MAC                                PL

|                   |                       |                                  |

|                   |                       |            RLC_uplink            |

|                   |                       | (RLC data block containing TLLI) |

|                   |                       *<---------------------------------*

|                   |                       |                             (PL 1)

|                   |                       |    MPHP_DATA_IND/RLC_DOWNLINK    |

|                   |                       |     (PACKET UPLINK ACK/NACK)     |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

|                   a) Message has the right TLLI                              |

|                 (GRR 1)                   |                                  |

|                   |                       |                                  |

|                   b) Message has the right TFI but another TLLI              |

|                 (GRR 2)                   |                                  |

|  GRR_STATUS_IND   |                       |                                  |

*<==================*                       |                                  |

(LLC 1)              |                       |                                  |

|                   c) N3104 has reached the maximum value                     |

|                 (GRR 3)                   |                                  |

|  GRR_STATUS_IND   |                       |                                  |

*<==================*                       |                                  |

(LLC 2)              |                       |                                  |

|                   |                       |                                  |

|                   d) T3166 expired        |                                  |

|                 (GRR 4)                   |                                  |

|  GRR_STATUS_IND   |                       |                                  |

*<==================*                       |                                  |

(LLC 3)              |                       |                                  |

|                   |                       |                                  |

(PL 1)

The MAC sends RLC data blocks. At sending of the first RLC data block T3164 shall be stopped, the counter N3104 shall be set to 1 and the timer T3166 shall be started

(MAC 1)

The MS receives a PACKET UPLINK ACK/NACK message.

(GRR 1)

Contention resolution completed. Stop T3166 and N3104.

(GRR 2, 3, 4)

Contention resolution fails. Reset of N3104, stop 3166, stop transmitting on this TBF.

(LLC 1,2,3)

GRR sends TBF failure to the LLC via GRR_STATUS_IND.

Requirements:

<R.MAC.CON_RES1.M.00x>, <R.MAC.ACC_COMP.A.001>

### Timing Advance

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                       |          MPHP_DATA_IND           |

|                   |                       |    (PACKET UPLINK ASSIGNMENT)    |

|                   |                       *<=================================*

|                   |                     (MAC 1)                              |

|                   |                       |                                  |

a) Timing Advance Index is included in the message

|                   |                       |                                  |

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

b) Timing Advance Index is not included in the message

|                   |                       |                                  |

(GRR 2)              |                       |                                  |

|                   |                       |                                  |

(MAC 1)

The MAC receives a PACKET UPLINK ASSIGNMENT message.

(GRR 1)

In this case the continuous update timing advance mechanism should be used.(Under construction!!!)

(GRR 2)

The continuous update timing advance mechanism shall not be used. The MS is not allowed to send normal bursts on the uplink until it receives a valid timing advance either through continuous timing advance procedure or packet power control/ timing advance message.

Requirements:

<R.MAC.TA.M.003-4>, <R.MAC.TA_2PH.M.00x>

### TBF establishment using two phase access

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                       |            MPHP_DATA_IND         |

|                   |                       |    (PACKET UPLINK ASSIGNMENT)    |

|                   |                      *<==================================*

|                   |                     (MAC 1)                              |

|                   |                       |       MPHP_SINGLE_BLOCK_REQ      |

|                   |                       |     (PACKET RESOURCE REQUEST)    |

|                   |                       *=================================>*

|                   |                       |                             (PL 1)

|                   |                       |       MPHP_SINGLE_BLOCK_CON      |

|                   |                       *<================================*|

|                   |                     (MAC 2)                              |

|                   |                       |                                  |

(MAC 1)

MAC receives PACKET UPLINK ASSIGMENT message which contains single block allocation structure.

(PL 1)

MAC composes a PACKET RESOURCE REQUEST message and sends it to the PL. Start timer T3168. The MS shall indicate in the RLC_OCTET_COUNT the number of octets of user data that it has to be transferred in the TBF.

(MAC 2)

The PL confirms that the requested message was sent.

Requirements:

<R.MAC.TBF_2PH.M.001>, <R.MAC.RES_REQ.M.00x>, <R.MAC.RC_RS_RQ.M.002>

### Contention resolution at two phase access

LLC                 GRR                     MAC                                PL

|                   |                       |                                  |

|                 <Timer 3168 expires before receiving UPLINK ASSIGNMENT>      |

|   GRR_STATUS_IND  |                       |                                  |

*<===================*                       |                                  |

(LLC 1)              |                       |                                  |

|                   |                       |                                  |

|                   |                       |            MPHP_DATA_IND         |

|                   |                       |    (PACKET UPLINK ASSIGNMENT)    |

|                   |                      *<==================================*

|                   |                     (MAC 1)                              |

|              b) (GRR 1) Assignment contains the same TLLI

|                   |                       |                                  |

|              c) (GRR 2) Assignment contains same TFI but different TLLI

|                   |                       |                                  |

|   GRR_STATUS_IND  |                       |                                  |

*<===================*                       |                                  |

(LLC 2)              |                       |                                  |

|                   |                       |                                  |

(LLC 1)

GRR indicates to the LLC via GRR_STATUS_IND that a RLC/MAC error has occurred (T3168 expired).

(MAC 1)

MAC receives PACKET UPLINK ASSIGMENT message which contains single block allocation structure.

(GRR 1)

MAC receives PACKET UPLINK ASSIGMENT message which contains the same TLLI as the MAC has included in the PACKET RESOURCE REQUEST message. The GRR stops T3168.

(GRR 2)

Contention resolution fails. Stop timer T3168, reinitiate packet access procedure unless it has already been repeated 4 times. In that case, TBF failure has occurred.

(LLC 2)

GRR indicates to the LLC via GRR_STATUS_IND that a TBF failure has occurred.

Requirements:

<R.MAC.CON_RES2.M.00x>, <R.MAC.AB_PCCCH.M.001>

### Abnormal cases (muss noch gemacht werden)

LLC                 GRR                     MAC                                PL

|                   |                       |                                  |

|                   |                       |            MPHP_DATA_IND         |

|                   |                       |    (PACKET UPLINK ASSIGNMENT)    |

|                   |                      *<==================================*

|                   |                     (MAC 1)                              |

|              a) (GRR 1) Assignment contains the same TLLI

|                   |                       |                                  |

|              b) (GRR 2) Assignment contains same TFI but different TLLI

|                   |                       |                                  |

|                   |                       |                                  |

|   GRR_STATUS_IND  |                       |                                  |

|                   |                       |                                  |

*<==================*                       |                                  |

(LLC 1)              |                       |                                  |

|                   |                       |                                  |

|                   |                       |                                  |

(MAC 1)

MAC receives PACKET UPLINK ASSIGMENT message which contains single block allocation structure.

(GRR 1)

MAC receives PACKET UPLINK ASSIGMENT message which contains the same TLLI as the MAC has included in the PACKET RESOURCE REQUEST message. The GRR stops T3168.

(GRR 2)

Contention resolution fails. Stop timer T3168, reinitiate packet access procedure unless it has already been repeated 4 times. In that case, TBF failure has occurred.

(LLC 1)

GRR indicates to the LLC via GRR_STATUS_IND that a TBF failure has occurred.

Requirements:

<R.MAC.AB_PCCCH.M.00x>

## Neighbour Cell Syncronisation

After cell indication the ms is in packet idle mode. For the neigbour cell following services are requested.

- Defintion of BCCH allocation(BA_GPRS)

- Synchronisation to neighbourcells

- Fieldstrength measurements

### Defintion of BCCH allocation (BA_GPRS)

GRR/MAC                              PPC                                       PL

|                                    |                                         |

|                                    |             MPHP_DATA_IND               |

|                                    *<=======================================*|

|                                  (PPC 1)                                     |

|           PPC_PSI_3_IND            |                                         |

|*<==================================*                                         |

(GRR 1)                               |                                         |

|                                    |                                         |

(PPC 1)

PPC receives an MPHP_DATA_IND including a psi 3 message.

(GRR 1)

GRR receives a psi 3 meassge, which includes the BCCH allocation (BA list for GPRS). These ncells are stored. 
If needed GRR starts synchronisation and/or measurements to ncells.

### Synchronisation Request of ncells

GRR                        RR         |                                         |

|                         |          |                                         |

| RRGRR_NCELL_SYNC_REQ    |          |                                         |

*========================>*          |                                         |

|                        (RR 1)      |                                         |

|                         |          |                                         |

(RR 1)

GRR requests the synchronisation  to ncells. RR starts its own procedures to synchronize the neighbour cells (see MSC document of TI++ chapter 5).

### Conformation of Synchronisation to  ncells

GRR				      RR

| RRGRR_NCELL_SYNC_IND    |          |                                         |

*<========================*          |                                         |

(GRR 1)                    |          |                                         |

- a) syncroisation successful with same bsic

|                         |          |                                         |

| RRGRR_NCELL_SYNC_IND    |          |                                         |

*<========================*          |                                         |

(GRR 2)                    |          |                                         |

- b) syncroisation successful with different bsic

|                         |          |                                         |

| RRGRR_NCELL_SYNC_IND    |          |                                         |

*<========================*          |                                         |

(GRR 1)                    |          |                                         |

- c) syncroisation failed

|                         |          |                                         |

(GRR 1)

RR indicates a successful syncronisation to ncells with the same bsic.

(GRR 2)

RR indicates a successful syncronisation to ncells with the different bsic. The carrier is treated as a new carrier.

(GRR 3)

RR indicates a failed syncronisation to ncell(s).

## Performing measurements

This part describes the measurement procedures performed via MPHP –on PCCCH- and MPHC –on CCCH- in GPRS mode. This part contains an interface named Packet Physical layer Convergence (PPC). This interface allows different physical layer software to use Condat layer 2&3 and vice versa.

### Measurements on PCCCH

The MS shall not use the discontinuous reception (DRX) mode of operation (i.e. powering itself down when it is not expecting paging messages from the network) while performing the cell selection algorithm defined in GSM 03.22.

If PBCCH does not exist, the criteria and algorithms defined in subclauses 10.1.2 and 10.1.3 in [05.08v6.8.0] shall also apply to cells for which the GPRS cell re-selection parameters are provided to the MS in a Packet Cell Change Order or Packet Measurement Order message. In this case, the MS may convert the idle mode cell re-selection parameters, received for the other cells according to clause 6, to GPRS cell re-selection parameters according to table 4 and use the same procedures, except that the MS may measure received signal strength in packet idle mode according to either subclause 6.6.1 or subclause 10.1.1.

#### Packet idle mode

The MS shall measure the received RF signal level on the BCCH carriers of the serving cell and the surrounding cells as indicated in the BA (GPRS) list and optionally the NC_FREQUENCY_LIST, and calculate the received level average (RLA_P) for each carrier.

GRR/MAC                              PPC                                       PL

|                                    |                                         |

| PPC_CELL_RESELECTION_MEAS_REQ      |                                         |

*===================================>*                                         |

|                                   (PPC 1)                                    |

|                                    |                                         |

|                                    |          MPHP_CR_MEAS_REQ               |

|                                    *=======================================>*|

|                                    |                                     (PL1)

|                                    |                                         |

|                                    |          MPHP_CR_MEAS_IND               |

|                                    *<=======================================*|

|                                   (PPC 2)                                    |

| PPC_CELL_RESELECTION_MEAS_IND      |                                         |

|*<==================================*                                         |

(GRR 1)                               |                                         |

|                                    |                                         |

|                                    |                                         |

|                                    |                                         |

|                                    |          MPHP_CR_MEAS_IND               |

|                                    *<=======================================*|

|                                   (PPC 3)                                    |

| PPC_CELL_RESELECTION_MEAS_IND      |                                         |

|*<==================================*                                         |

(GRR 2)                               |                                         |

|                                    |                                         |

|                                    |                                         |

|                                    |          MPHP_CR_MEAS_IND               |

|                                    *<=======================================*|

|                                   (PPC 4)                                    |

| PPC_CELL_RESELECTION_MEAS_IND      |                                         |

|*<==================================*                                         |

(GRR 3)                               |                                         |

|                                    |                                         |

|                                    |                                         |

|                                    |          MPHP_CR_MEAS_IND               |

|                                    *<=======================================*|

|                                   (PPC 5)                                    |

| PPC_CELL_RESELECTION_MEAS_IND      |                                         |

|*<==================================*                                         |

(GRR 4)                               |                                         |

|                                    |                                         |

|                                    |                                         |

|                                    |          MPHP_CR_MEAS_IND               |

|                                    *<=======================================*|

|                                   (PPC 6)                                    |

| PPC_CELL_RESELECTION_MEAS_IND      |                                         |

|*<==================================*                                         |

(GRR 5)                               |                                         |

|                                    |                                         |

<measurement continues and the list of 6 strongest non-serving carriers updated at least once per running average>

…

…

…

< at least every 14 consecutive paging blocks or 10 seconds>

<Timer 10 seconds>

|                                    |                                         |

|                        RR          |                                         |

|                         |          |                                         |

| RRGRR_CHECK_BSIC_REQ    |          |                                         |

*========================>*          |                                         |

|                        (RR 1)      |                                         |

|                         |          |                                         |

| RRGRR_CHECK_BSIC_CNF    |          |                                         |

*<========================*          |                                         |

(GRR 7)                    |          |                                         |

|                         |          |                                         |

…

<If the MS enters transfer mode >

|                                    |                                         |

| PPC_CELL_RESELECTION_MEAS_STOP_REQ |                                         |

*===================================>*                                         |

|                                    |         MPHP_CR_MEAS_STOP_REQ           |

|                                    *=======================================*>

|                                    |                                         |

(PPC 1)

PPC receives a request for performing measurements for a list of BCCH carriers for surrounding cells as indicated in BA (GPRS), e.g. send in PSI3/PSI3bis, and serving cell BCCH frequency that are used for cell reselection and optionally the NC_FREQUENCY_LIST. This list adds or deletes some frequencies from the BA (GPRS) list.

(PL 1)

PL receives the frequency list to perform measurements for cell reselection and network controlled cell reselection. PL performs measurements according to the rules/restrictions defined in [05.08v680]

(PPC 2)

PL sends received signal level measurement samples to PPC. PL takes the following constraints into account:

At least one received signal level measurement sample on each BCCH carrier shall be taken for each paging block monitored by the MS according to its current DRX mode and its paging group. As the minimum MS shall take one measurement for each BCCH carrier for every 4 second. As the maximum, the MS is however not required to take more than 1 sample per second for each BCCH carrier. Samples collected over a period of 5 s to Max {5s,  five  consecutive  paging  blocks  of  that MS}, and shall be maintained for each BCCH carrier. The same number of measurement samples shall be taken for all BCCH carriers, and the samples allocated to each carrier shall as far as possible be uniformly distributed over the evaluation period.

(GRR 1)

PPC sends the received signal level measurements (RX_LEV) to GRR and GRR saves the received last 5 RX_LEV to build a received level running average (RLA_P) value.

(GRR 2,3,4,5 and PPC 3,4,5,6)

See comments in GRR1 and PPC 2.

In addition the MS shall verify the BSIC of the BCCH carriers. Only cells with allowed BSIC shall be considered for re-selection. The allowed BSIC is either the BSIC broadcast for that carrier in the BA (GPRS) list or, for cells in BA (BCCH) where no BSIC is broadcast, a BSIC with allowed NCC part.

(RR 1)

GRR sends the BSIC of the 6 strongest non-serving cell BCCH carriers to check.

(GRR 7)

RR sends the result to GRR. If a change of BSIC is detected then the carrier shall be treated as a new carrier.

In the case of a multiband MS, the MS shall attempt to decode the BSIC, if any BCCH carrier with unknown BSIC is detected among the number of strongest BCCH carriers in each band as indicated by the parameter MULTIBAND_REPORTING, broadcast on PBCCH, or if PBCCH does not exist, on BCCH.

#### Packet transfer mode

GRR/MAC                              PPC                                       PL

|                                    |                                         |

| PPC_CELL_RESELECTION_MEAS_REQ      |                                         |

*===================================>*                                         |

|                                   (PPC 1)                                    |

|                                    |                                         |

|                                    |          MPHP_CR_MEAS_REQ               |

|                                    *=======================================>*|

|                                    |                                     (PL1)

|                                    |                                         |

|                                    |          MPHP_TCR_MEAS_IND              |

|                                    *<=======================================*|

|                                   (PPC 2)

|                                    |                                         |

| PPC_CELL_RESELECTION_MEAS_IND      |                                         |

*<===================================*                                         |

|                                    |                                         |

<5 times >

(GRR 2)

<continue >

…

…

<Timer 10 seconds>

(GRR 3)                               |                                         |

|                                    |                                         |

|                        RR          |                                         |

|                         |          |                                         |

| RRGRR_CHECK_BSIC_REQ    |          |                                         |

*========================>*          |                                         |

|                        (RR 1)      |                                         |

|                         |          |                                         |

| RRGRR_CHECK_BSIC_CNF    |          |                                         |

*<========================*          |                                         |

(GRR 4)                    |          |                                         |

|                         |          |                                         |

(GRR 5)                    |          |                                         |

…

- In case of multiband reporting

(GRR 6)                    |          |                                         |

<Timer 5 seconds>

(GRR 7)                               |                                         |

|                                    |                                         |

|                        RR          |                                         |

|                         |          |                                         |

| RRGRR_CHECK_BSIC_REQ    |          |                                         |

*========================>*          |                                         |

|                        (RR 2)      |                                         |

|                         |          |                                         |

| RRGRR_CHECK_BSIC_CNF    |          |                                         |

*<========================*          |                                         |

(GRR 8)                    |          |                                         |

<It is repeated the number of unknown BSIC times -in each band as indicated by the parameter MULTIBAND_REPORTING>

…

<Timeout 5 seconds>

(GRR 9)

a) BSIC decoding failed

|                                    |                                         |

|                        RR          |                                         |

|                         |          |                                         |

| RRGRR_CHECK_BSIC_REQ    |          |                                         |

*========================>*          |                                         |

|                         |          |                                         |

|                         |          |                                         |

| RRGRR_CHECK_BSIC_CNF    |          |                                         |

*<========================*          |                                         |

a1) Re-confirmed existing BSICs

<Timer 5 seconds>

|                                    |                                         |

|                        RR          |                                         |

|                         |          |                                         |

see notes following a1)

(PPC 1, PL 1)

After entering the packet transfer mode, GRR starts via PPC received signal level measurements.

(PPC 2, GRR 1)

PPC gets the measurement results and passes to GRR. The PL takes into account that samples collected over a period of 5 s, and shall be maintained for each BCCH carrier. The same number of measurement samples shall be taken for all BCCH carriers except, if the parameter PC_MEAS_CHAN indicates that the power control measurements shall be made on BCCH, for the serving cell where at least 6 measurement samples shall be taken per 52-multiframe. The samples allocated to each carrier shall as far as possible be uniformly distributed over the evaluation period.

(GRR 2)

At least 5 received signal level measurement samples are required for a valid RLA_P value, therefore GRR collects at least 5 results.

(GRR 3)

The MS shall attempt to check the BSIC for each of the 6 strongest non-serving cell BCCH carriers as often as possible, and at least every 10 seconds.

(RR 1)

GRR sends the BSIC of the 6 strongest non-serving cell BCCH carriers to check.

(GRR 4)

RR sends the result to GRR. If a change of BSIC is detected then the carrier shall be treated as a new carrier. A list containing BSIC and timing information for these strongest carriers at the accuracy required for accessing a cell including the absolute times derived from the parameters T1, T2, T3 shall be kept by the MS. This information may be used to schedule the decoding of BSIC and shall be used when re-selecting a new cell in order to keep the switching time at a minimum. When a BCCH carrier is found to be no longer among the 6 strongest, BSIC and timing information shall be retained for 10 seconds. (This is in case a cell re-selection command to this cell is received just after the MS has stopped reporting that cell).

(GRR 5)

Here an extract from GSM 05.08: “If an MS, performing a multislot uplink transfer with fixed allocation, is not able to perform BSIC decoding within the search frames according to its multislot class, the MS shall perform the BSIC decoding between allocations. The MS shall determine the necessary periods by not requiring uplink resources.

If an MS, performing a multislot downlink transfer with fixed allocation, is not able to perform BSIC decoding within the search frames according to its multislot class, the MS shall perform the BSIC decoding during inactivity periods. The MS shall request these inactivity periods from the network to allow for the required BSIC decoding (see GSM 04.60).” This situation should be taken into account on both Condat and layer 1 side. It is not clear enough!

(GRR 6)

In the case of a multiband MS, the MS shall attempt to decode the BSIC, if any BCCH carrier with unknown BSIC is detected among the number of strongest BCCH carriers in each band as indicated by the parameter MULTIBAND_REPORTING, broadcast on PBCCH, or if PBCCH does not exist, on BCCH.

(GRR 7,8 and RR 2)

Thus an MS shall, for a period of up to 5 seconds, devote all search frames to attempting to decode the BSICs for MULTIBAND_REPORTING.

(GRR 9a)

If decoding of BSIC for MULTIBAND_REPORTING fails then the MS shall return to confirming existing BSICs.

(GRR 9a1)

Having re-confirmed existing BSICs, if there are still BCCH carriers, among the six strongest, with unknown BSICs, then the decoding of these shall again be given priority for a further period of up to 5 seconds.

Notes:

- If either no BSIC can be decoded on a surrounding cell BCCH carrier, or the BSIC is not allowed, then the received signal level measurements on that channel shall be discarded and the MS shall continue to monitor that channel.

- If a change of BSIC is detected on a carrier, then any existing received signal level measurement shall be discarded and the carrier shall be treated as a new carrier.

- If the BSIC cannot be decoded at the next available opportunities re-attempts shall be made to decode this BSIC. If the BSIC is not decoded for more than three successive attempts it will be considered lost and any existing received signal level measurement shall be discarded and the MS shall continue to monitor that carrier.

### Measurements on CCCH

There are no PBCCH present in the cell.

GRR/MAC                                                                        PL

|                                                                              |

|            rlc_uplink/downlink                                               |

|   (PACKET CELL CHANGE ORDER/ PACKET MEASUREMENT ORDER)                       |

*<==============================================================================*

- a)                                                                              |

|                                                                              |

- b)                                                                              |

a)

GRR receives cell re-selection parameters in a Packet Cell Change Order or Packet Measurement Order message on PACCH, e.g. during transfer mode. GRR takes the cell selection and measurement procedures for GPRS into account

b)

GRR receives no GPRS relevant measurement or cell change messages. Perform cell reselection according the former procedures for GSM. Except, that the MS shall perform monitoring of full SI on BCCH, if indicated by change mark on BCCH or PACCH.

## Cell Re-selection procedure in packet idle mode (PBCCH exists)

GRR deceides to change the cell in packet idle mode. The decision depends on the fieldstrength measurements of the lower layer.

### Read the pbcch of the new cell

GRR                                  PPC                                       PL

|                                    |                                         |

|      PPC_READ_NCELL_PBCCH_REQ      |                                         |

*===================================>*                                         |

|                                   (PPC 1)                                    |

|                                    |                                         |

|                                    |          MPHP_NCELL_PBCCH_REQ           |

|                                    *=======================================>*|

|                                    |                                     (PL1)

|                                    |                                         |

(PPC 1)

GRR requests the reading of the potenally new cell.

(PL 1 )

PPC indicate the to L1 to read the indacted neighbourcell.

### Receiving of PSIs in the new cell

GRR                                  PPC                                       PL

|                                    |                                         |

|                                    |          MPHP_NCELL_PBCCH_IND           |

|                                    *<=======================================*|

|                                  (PL1)                                       |

|          PPC_PSI_X_IND             |                                         |

*<===================================*                                         |

(GRR 1)                              |                                         |

|                                    |                                         |

(PL 1)

L1  indicates the receiving of packet system information in the new cell.

(GRR 1 )

PPC indicates to GRR the received psi.

This procedure is possible foe different packet system information.

### Reselect new cell in packet idle mode

GRR                                  PPC                                       PL

|                                    |                                         |

|      PPC_STOP_PSI_READING_REQ      |                                         |

*===================================>*                                         |

|                                   (PPC 1)                                    |

|                                    |        MPHP_NCELL_PBCCH_STOP_REQ        |

|                                    *=======================================>*|

|                                    |                                    (PL 1)

|                                    |        MPHP_NCELL_PBCCH_STOP_CON        |

|                                    *<=======================================*|

|                                   (PPC 2)                                    |

|  PPC_NCELL_READING_STOP_IND        |                                         |

*<===================================*                                         |

(GRR 1)                              |                                         |

|        PPC_RECEIVE_PSI_REQ         |                                         |

*===================================>*                                         |

|                                   (PPC 3)                                    |

|                                    |                                         |

|                                    |          MPHP_SCELL_PBCCH_REQ           |

|                                    *=======================================>*|

|                                    |                                    (PL 2)

| PPC_UPDATE_STATIC_PCCCH_DATA_REQ   |                                         |

*===================================>*                                         |

|                                   (PPC 4)                                    |

|        PPC_READ_PCCCH_REQ          |                                         |

*===================================>*                                         |

|                                   (PPC 5)                                    |

|                                    |             MPHP_START_PCCCH_REQ        |

|                                    *=======================================>*|

|                                    |                                    (PL 3)

|                                    |                                         |

(PPC 1)

GRR requests the stop of reading of the new cell.

(PL 1 )

PPC forwards the request to L1.

(PPC 2)

L1 confirms that the reading of the neighbour cell is stopped.

(GRR 1 )

PPC forwards the confirm to GRR.

(PPC 2)

GRR  starts to assign the new cell to ppc.

(PL 2)

PPC forwards the request to L1 .

(PPC 4)

GRR  updates the PCCCH parameters.

(PPC 5)

GRR  requests to assign the PCCCH parameters to L1.

(PL 3)

PPC forwards the request to L1 .

### Packet Access in the new Cell(must be modified  only in packet transfer mode)

GRR                                  PPC                                       PL

|                                    |                                         |

|             PPC_PAP_START_REQ      |                                         |

*===================================>*                                         |

|                                   (PPC 1)                                    |

|           PPC_ASSIGN_TLLI_REQ      |                                         |

*===================================>*                                         |

|                                   (PPC 2)                                    |

|      PPC_STOP_PSI_READING_REQ      |                                         |

*===================================>*                                         |

|                                   (PPC 3)                                    |

|                                    |        MPHP_NCELL_PBCCH_STOP_REQ        |

|                                    *=======================================>*|

|                                    |                                    (PL 1)

| PPC_UPDATE_STATIC_PCCCH_DATA_REQ   |                                         |

*===================================>*                                         |

|                                   (PPC 4)                                    |

|        PPC_READ_PCCCH_REQ          |                                         |

*===================================>*                                         |

|                                   (PPC 5)                                    |

|                                    |             MPHP_START_PCCCH_REQ        |

|                                    *=======================================>*|

|                                    |                                    (PL 2)

|     PPC_SEND_ACCESS_BURST_REQ      |                                         |

*===================================>*                                         |

|                                   (PPC 6)                                    |

|                                    |                   MPHP_RA_REQ           |

|                                    *=======================================>*|

|                                    |                                    (PL 3)

|                                    |                   MPHP_RA_CNF           |

|                                    *<=======================================*|

|                                   (PPC 7)                                    |

|     PPC_SEND_ACCESS_BURST_CNF      |                                         |

*<===================================*                                         |

(GRR 1)                              |                                         |

|     PPC_SEND_ACCESS_BURST_REQ      |                                         |

*===================================>*                                         |

|                                   (PPC 8)                                    |

|                                    |                   MPHP_RA_REQ           |

|                                    *=======================================>*|

|                                    |                                    (PL 4)

|                                    |                                         |

(PPC 1)

GRR indicates the start of the packet access procedure.

(PPC 2 )

GRR assigns the TLLI to PPC.

(PPC 3)

GRR requests the stop of reading psis in the new cell.

(PL 1)

PPC forwards the request to L1.

(PPC 4)

GRR updates PCCCH parameters.

(PPC 5)

GRR requests to reorg the PCCCH of the new cell.

(PL 2)

PPC forwards the request to L1.

(PPC 6)

GRR builds a random access burst with cause "cell update" and sends it to PPC.

(PL 3)

PPC forwards the request to L1.

(PPC 7)

L1 confirms that the random access burst was sent.

(GRR 1)

PPC forwards the confirm to GRR.

(PPC 8)

GRR builds again a random access burst with cause "cell update" and sends it to PPC.

(PL 4)

PPC forwards the request to L1.

## TBF establishment initiated by the network on PCCCH

### Packet downlink assignment

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                       |            MPHP_DATA_IND         |

|                   |                       |   (PACKET DOWNLINK ASSIGNMENT)   |

|                   |                      *<==================================*

|                   |                      (MAC 1)                             |

|                   |                       |                                  |

- a) Poll bit is set in the message

|The same procedure as described in 4.2.7b is used. The PACKET CONTROL ACKNOWLEDGEMENT is sent as four access bursts.                             (PL 1)

a.1) Assignment message does not contains timing advance index

|                   |                       |                                  |

|                   |                       |            MPHP_DATA_IND         |

|                   |                     (PACKET POWER CONTROL/TIMING ADVANCE)|

|                   |                       *<=================================*

|                   |                     (MAC 2)                              |

|                   |                       |                                  |

a.2) Assignment message contains timing advance index

|                   |                       |                                  |

|                   |                     (MAC 3)                              |

|                   |                       |                                  |

(MAC 1)

MAC receives a PACKET DOWNLINK ASSIGNMENT message. If this message contains a TBF starting time, the GRR shall monitor PCCCH until the starting time occurs. The timer T3190 shall be started. The timer is reset when receiving the first valid RLC/MAC block. On expiry of T3190, the MS aborts the procedure and returns to packet idle mode.

(PL 1)

MAC sends the PACKET CONTROL ACKNOWLEDGEMENT message as four access bursts in the reserved uplink radio block specified by the RRBP field. The reserved block is considered as a one block PACCH allocation. The PACKET CONTROL ACKNOWLEDGEMENT, message is used to derive the timing advance.

(MAC 2)

MAC receives message to update the timing advance. The update of timing advance can be also done by continuous timing advance procedure, if timing advance index is included in the message.

(MAC 3)

The timing advance in the MS shall be updated via continuous timing advance procedure.

Requirements:

<R.MAC.P_DWN_AS.M.00x>, <R.MAC.AI_PCCCH.M.00x>

### Packet polling procedure after receiving a packet downlink assignment

This procedure is described in 4.2.7.

Requirements:

<R.MAC.I_P_POLL.M.00x>

## Procedures for sending measurement reports in packet idle mode

### Measurement report sending initiated on PCCCH

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |         MPHP_RA_REQ   |                                  |

|                   |   (PACKET CHANNEL REQUEST)                               |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                   |    MPHP_RA_CON        |                                  |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

a) Receipt of packet uplink assignment

|                   |                       |                                  |

|                   |                       |         MPHP_DATA_IND            |

|                   |                       |    (Packet uplink assignment)    |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

|                   |                       |       MPHP_SINGLE_BLOCK_REQ      |

|                   |                       |   (PACKET MEASUREMENT REPORT)    |

|                   |                       *=================================>*

|                   |                       |                             (PL 2)

|                   |                       |       MPHP_SINGLE_BLOCK_CON      |

|                   |                       *<================================*|

|                   |                     (MAC 2)                              |

|                   |                       |                                  |

b) Receipt of packet access reject

|                   |                       |                                  |

|                   |                       |          MPHP_DATA_IND           |

|                   |                       |      (Packet access reject)      |

|                   |                       *<=================================*

|                   |                    (MAC 3)                               |

|                   |                       |                                  |

c) Receipt of packet queuing notification or packet polling request

|                   |                       |                                  |

|                   |                       |          MPHP_DATA_IND           |

|                   |                       *<=================================*

|                   |                    (MAC 4)                               |

d) Timer T3170 expires. The measurement report for that period is canceled. The MS returns to packet idle mode.

|                   |                       |                                  |

(PL 1)

GRR composes a packet channel request message containing as cause “Single block without TBF establishment” and sends it on PRACH.

(GRR 1)

The PL confirms that the message was sent.

(MAC 1)

The PL indicates to the MAC that the packet uplink assignment was received. No TBF will be started. T3170 was not expired at the time on which the packet uplink assignment message was received. If T3170 expires, packet access procedure is cancelled and the transmission of the measurement report is cancelled for that measurement report period.

(PL 2)

The MAC composes packet measurement report either containing NC measurement report or EXT measurement report structure and sends it on the allocated single block.

(MAC 2)

The PL confirms that the PACKET MEASUREMENT message was sent.

(MAC 3)

The MAC receives the packet access reject message. The procedures in packet access reject message are done in this situation. If any of the measurement report interval timers T3158 or T3178 expires before any of the timers T3172 or T3162 expires. In this case no new measurement report shall be initiated but T3158 or T3178 shall be restarted.

(MAC 4)

The MAC receives either packet queuing notification or packet polling request and aborts the procedure.

Requirements:

<R.MAC.MREP_IDL.M.00x>, <R.MAC.MREP_SND.M.001>, <R.MAC.RC_PUPL.M.00x>, <R.MAC.RC_AC_RJ.M.00x>, <R.MAC.AB_MREP.M.00x>

### Measurement report sending initiated on CCCH

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |               MPHC_RA_REQ                                |

|                   |            (channel request)                             |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                   |               MPHC_RA_CON                                |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

- a) On receipt of an immediate assignment message

|                   |                       |                                  |

|                   |               MPHC_RA_REQ                                |

|                   |        (Immediate assignment)                            |

*<=============================================================================*

(GRR 2)              |                       |                                  |

|                   |                       |                                  |

|                   |                       |       MPHP_SINGLE_BLOCK_REQ      |

|                   |                       |   (PACKET MEASUREMENT REPORT)    |

|                   |                       *=================================>*

|                   |                       |                             (PL 2)

|                   |                       |       MPHP_SINGLE_BLOCK_CON      |

|                   |                       *<================================*|

|                   |                     (MAC 1)                              |

|                   |                       |                                  |

- b) On receipt of an immediate assignment reject message

|                   |                       |                                  |

|                   |               MPHC_RA_REQ                                |

|                   |        (Immediate assignment REJECT)                     |

*<=============================================================================*

(GRR 3)              |                       |                                  |

|                   |                       |                                  |

- c) Timers expires

(GRR 4)

|                   |                       |                                  |

(PL 1)

GRR sends a channel request message on RACH. This message indicates “Single block packet access”.

(GRR 1)

The PL confirms the send of the message.

(GRR 2)

GRR receives an immediate assignment message.

(PL 2)

The MAC composes a packet measurement report message and sends it on the assigned single block.

(MAC 3)

The PL confirms the send of the message.

(GRR 3)

The GRR receives an immediate assignment reject message. The measurement procedure shall be aborted.

(GRR 4)

If any of the measurement report interval timers T3158 or T3178 expires before any of the timers T3142 or T3146 expires. In this case no new measurement report shall be initiated but T3158 or T3178 shall be restarted.

Requirements:

<R.MAC.MRP_CCCH.M.00x>

## Measurement order procedures in packet idle mode

### Measurement order procedures initiated on PCCCH

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                       |           MPHP_DATA_IND          |

|                   |                       |    (Packet MEASUREMENT ORDER)    |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

|                   |                       |                                  |

(MAC 1)

The MAC receives a Packet MEASUREMENT ORDER message. Storing the measurement order parameters and immediately return to PCCCH.

Requirements:

<R.MAC.MORD_PCC.M.00x>

### Measurement order procedures initiated on CCCH

The procedure is the same procedure as in the measurement order initiated on PCCCH. The main difference is that the Packet MEASUREMENT ORDER message is sent on the PDCH which was assigned by immediate assignment message.

Requirements:

<R.MAC.MORD_CCC.M.00x>

## Measurement report in packet transfer mode

GRR                 RLC                     MAC                                PL

|                   |                       |            RLC_uplink  or        |

|                   |                       |       SINGLE BLOCK REQUEST       |

|                   |                       |    (PACKET MEASUREMRNT REPORT)   |

|                   |                       *=================================>*

|                   |                       |                             (PL 1)

|                   |                       |                                  |

(PL 1)

The MS sends the PACKET MEASUREMENT REPORT to the network in either the RLC_uplink or SINGLE BLOCK REQUEST.

Requirements:

<R.MAC.MREP_TRF.M.00x>

## Measurement Order procedures in packet transfer mode

GRR                 RLC                     MAC                                PL

|                   |                       |            RLC_downlink          |

|                   |                       |     (PACKET MESUREMENT ORDER)    |

|                   |                       *<---------------------------------*

|                   |                    (MAC 1)                               |

|                   |                       |                                  |

(MAC 1)

The network sends the PACKET MESUREMENT ORDER to the MS in the RLC_uplink.

Requirements:

<R.MAC.MORD_TRF.M.001>, <R.MAC.AB.M.004>

## Cell change order procedure in packet idle mode

### Cell change order procedure initiated on PCCCH

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                       |           MPHP_DATA_IND          |

|                   |                       |    (PACKET CELL CHANGE ORDER)    |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

(GRR 1)              |                       |                                  |

|                   |               MPH(P)C_RA_REQ                             |

|                   |         ((PACKET) channel request)                       |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                   |               MPH(P)C_RA_CON                             |

*<=============================================================================*

(GRR 2)              |                       |                                  |

a) Timer T3174 has not expired

|                   |                       |         MPH(P)C_DATA_IND         |

|                   |                    (Response to (packet) channel request)|

|                   |                       *<=================================*

|                   |                    (MAC 2)                               |

b) Timer T3174 has expired

(GRR 3)              |                       |                                  |

b.1) The MS was in packet idle mode or in downlink packet transfer mode. The Packet cell change failure is transmitted via a single block. In packet idle mode the MS has to request a single block (packet access procedure see 4.2.4).

|                   |                       |     MPHP_SINGLE_BLOCK_REQ        |

|                   |                       |  (Packet cell change failure)    |

|                   |                       *=================================>*

|                   |                       |                             (PL 2)

|                   |                       |     MPHP_SINGLE_BLOCK_CON        |

|                   |                       *<=================================*

|                   |                    (MAC 3)                               |

b.2)The MS was in uplink or both in up/downlink packet transfer mode

|                   |                       |           RLC_uplink             |

|                   |                       |  (Packet cell change failure)    |

|                   |                       *<---------------------------------*

|                   |                       |                             (PL 3)

|                   |                       |                                  |

(MAC 1)

The MAC receives packet cell change order message on the PCCCH blocks. The MAC stops all RLC/MAC timers and start timer T3174.

(GRR 1)

The MAC stops all RLC/MAC timers and start timer T3174. The MS shall switch to the specified new cell and obey the RLC/MAC relevant procedures on this new cell.

(PL 1)

The GRR sends packet channel request or channel request in the new cell and waits for an answer.

(GRR 2)

The PL confirms that the message was sent.

(MAC 2)

The MAC receives a response to packet channel request in the new cell. Stop timer T3174.

(GRR 3)

Timer T3174 has expired before a response to packet channel request. Start timer T3176 and return to the old cell.

(PL 2)

The GRR initiates a random access in the old cell with Single block without TBF establishment and sends a packet cell change failure to the network. Stop T3176. If T3176 expires before sending packet cell change failure, abnormal release with return to CCCH or PCCCH.

(MAC 3)

The PL confirms the single block request.

(PL 3)

The GRR initiates a new TBF and sends a packet cell change failure to the network. Stop T3176. If T3176 expires before sending packet cell change failure, abnormal release with random access shall be done.

Requirements:

<R.MAC.CC_PCCCH.M.00x>

### Cell change order procedure initiated on CCCH

The procedure is the same procedure as in the cell change order initiated on PCCCH. The main difference is that the packet cell change order message is sent on the PDCH which was assigned by immediate assignment message.

Requirements:

<R.MAC.CC_CCCH.M.00x>

## TBF establishment initiated by the MS on CCCH

### Mobile originated packet access –B

LLC            GRR             RR             SIM            PL

|              |               |              |              |

| GRR_DATA_REQ |

*=============>*

(GRR 1)

| RRGRR_CHANNEL_REQ

*==============>*

(RR 1)

| MPHC_RA_REQ                 |

*============================>*

(PL 1)

| MPHC_RA_CON                 |

*<============================*

(RR 2)

| MPHC_RA_REQ                 |

*============================>*

(PL 2)

| MPHC_RA_CON                 |

*<============================*

(RR 3)

<* start T3146 *>

|MPHC_DATA_IND (IMMED.ASS.)   |

*<============================*

(RR 4)

<* enter dedi mode*>

| RRGRR_IA_IND |

*<=============*

(GRR 2)

| RRGRR_STOP_CHANNEL_REQUEST_REQ

*=============>*

(RR 5)

<* stop T3146/ stop REORG / leave dedi*>|

|              |                             |

(GRR 1)

The LLC sends GRR a GRR_DATA_REQ primitive including LLC-pdu.

(RR 1)

With an RRGRR_CHANNEL_REQ primitive GRR orders RR to send RANDOM ACCESS REQUEST primitives to PL.

(PL 1)

The RR sends PL an MPHC_RA_REQ primitive. PL sends a CHANNEL REQUEST message on RACH.

(RR 2)

The PL sends RR an MPHC_RA_CON primitive, indicating that a CHANNEL REQUEST message has been sent out on RACH.

(PL 2)

The RR sends PL an MPHC_RA_REQ primitive. PL sends a CHANNEL REQUEST message on RACH. The RR will send RANDOM ACCESS REQUESTs until an IMMEDIATE ASSIGNMENT message arrives or max_retrans CHANNEL REQUEST are sent or timer T3146 expires

(RR 3)

The PL sends RR an MPHC_RA_CON primitive, indicating that a CHANNEL REQUEST message has been sent out on RACH. Timer T3146 is started.

(RR 4)

The PL sends RR an MPHC_DATA_IND primitive, including an IMMEDIATE ASSIGNMENT message. Dedicated mode is entered.

(GRR 2)

The RR sends GRR an RRGRR_IA_IND (immediate assignment) primitive.

(RR 5)

The GRR sends RR a RRGRR_STOP_CHANNEL_REQUEST_REQ primitive. Timer T3146 and REORG is stopped, dedicated mode is left.

Requirements:

### Mobile originated packet access –B

LLC            GRR             RR            SIM     DL      PL

|              |               |             |       |       |

| GRR_DATA_REQ |

*=============>*

(GRR 1)

| RRGRR_CHANNEL_REQ

*==============>*

(RR 1)

|MPHC_RA_REQ                  |

*============================>*

(PL 1)

|MPHC_RA_CON                  |

*<============================*

(RR 2)

|MPHC_RA_REQ                  |

*============================>*

(PL 2)

|MPHC_RA_CON                  |

*<============================*

(RR 3)

<* start T3146 *>

|MPHC_DATA_IND (IMMED.ASS.)   |

*<============================*

(RR 4)

<* enter dedi mode *>

| RRGRR_IA_IND |

*<=============*

(GRR 2)

| RRGRR_DATA_REQ

*=============>*

(RR 5)

| DL_ESTABLISH_REQ    |

*====================>*

(DL 1)

| DL_DATA_IND         |

*<====================*

(RR 6)

| RRGRR_DATA_IND

*<=============*

(GRR 3)

|              |                             |

(GRR 1)

The LLC sends GRR a GRR_DATA_REQ primitive including LLC-pdu.

(RR 1)

With an RRGRR_CHANNEL_REQ primitive GRR orders RR to send RANDOM ACCESS REQUEST primitives to PL.

(PL 1)

The RR sends PL an MPHC_RA_REQ primitive. PL sends a CHANNEL REQUEST message on RACH.

(RR 2)

The PL sends RR an MPHC_RA_CON primitive, indicating that a CHANNEL REQUEST message has been sent out on RACH.

(PL 2)

The RR sends PL an MPHC_RA_REQ primitive. PL sends a CHANNEL REQUEST message on RACH. The RR will send RANDOM ACCESS REQUESTs until an IMMEDIATE ASSIGNMENT message arrives arrives or  max_retrans CHANNEL REQUEST are sent or timer T3146 expires.

(RR 3)

The PL sends RR an MPHC_RA_CON primitive, indicating that a CHANNEL REQUEST message has been sent out on RACH. Timer T3146 is started.

(RR 4)

The PL sends RR an MPHC_DATA_IND primitive, including an IMMEDIATE ASSIGNMENT message. Dedicated mode is entered.

(GRR 2)

The RR sends GRR an RRGRR_IA_IND primitive.

(RR 5)

The GRR sends RR a RRGRR_DATA_REQ primitive including a RR INITIALISATION REQUEST

(DL 1)

RR establishes the dedicaded mode

(RR 6)

RR receives the PDCH ASSIGNMENT COMMAND

(GRR 3)

The RR sends GRR an RRGRR_DATA_IND primitive including a PDCH ASSIGNMENT COMMAND

Requirements:

### Mobile originated packet access –B

LLC            GRR            RR             SIM             PL

|              |              |              |               |

| GRR_DATA_REQ |

*=============>*

(GRR 1)

| RRGRR_CHANNEL_REQ

*==============>*

(RR 1)

|MPHC_RA_REQ                  |

*============================>*

(PL 1)

|MPHC_RA_CON                  |

*<============================*

(RR 2)

|MPHC_DATA_IND (IMMED.ASS.REJ)|

*<============================*

(RR 3)

<* start T3146/ stop ra req *>

| RRGRR_ASSIGNMENT_REJ_IND

*<==============*

(GRR 2)

<* start T3142 *>

<* expire T3146 / stop REORG*>

| RRGRR_STOP_CHANNEL_REQUEST_IND

*<==============*

(GRR 3)

<* expire T3142 *>

| RRGRR_CHANNEL_REQ

*==============>*

(RR 4)

|               |                             |

(GRR 1)

The LLC sends GRR a GRR_DATA_REQ primitive including LLC-pdu.

(RR 1)

With an RRGRR_CHANNEL_REQ primitive GRR orders RR to send RANDOM ACCESS REQUEST primitives.

(PL 1)

The RR sends PL an MPHC_RA_REQ primitive. PL sends a CHANNEL REQUEST message on RACH.

(RR 2)

The PL sends RR an MPHC_RA_CON primitive, indicating that a CHANNEL REQUEST message has been sent out on RACH.

(RR 3)

The PL sends RR an MPHC_DATA_IND primitive, including an IMMEDIATE ASSIGNMENT REJECTION message. Timer T3146 is started random access request is stopped.

(GRR 2)

The RR sends GRR an RRGRR_ASSINGMENT_REJ_IND (assignment rejection indication) primitive. Timer T3142 is started.

(GRR 3)

When timer T3146 expires, RR sends GRR a RRGRR_STOP_CHANNEL_REQUEST_IND primitive. REORG is stopped.

(RR 4)

When timer T3142 expires, GRR sends RR a new RRGRR_CHANNEL_REQ primitive.

Requirements:

### Mobile originated packet access –B

LLC            GRR             RR             SIM            PL

|              |               |              |              |

| GRR_DATA_REQ |

*=============>*

(GRR 1)

| RRGRR_CHANNEL_REQ

*==============>*

(RR 1)

|MPHC_RA_REQ                  |

*============================>*

(PL 1)

|MPHC_RA_CON                  |

*<============================*

(RR 2)

|MPHC_RA_REQ                  |

*============================>*

(PL 2)

|MPHC_RA_CON                  |

*<============================*

(RR 3)

<* start T3146 *>

|MPHC_DATA_IND (IMMED.ASS.)   |

| (wrong request reference)   |

| (or no IA received)         |

| (2nd part of two message IA  |

|  not received)              |

*<============================*

(RR 4)

<* expire T3146 *>                |

|RRGRR_STOP_CHANNEL_REQUEST_IND

*<==============*

(GRR 2)            |

- GRR_STATUS_IND|

*<=============*

(LLC 1)

|              |               |                             |

(GRR 1)

The LLC sends GRR a GRR_DATA_REQ primitive including LLC-pdu.

(RR 1)

With an RRGRR_CHANNEL_REQ primitive GRR orders RR to send RANDOM ACCESS REQUEST primitives.

(PL 1)

The RR sends PL an MPHC_RA_REQ primitive. PL sends a CHANNEL REQUEST message on RACH.

(RR 2)

The PL sends RR an MPHC_RA_CON primitive, indicating that a CHANNEL REQUEST message has been sent out on RACH.

(PL 2)

The RR sends PL an MPHC_RA_REQ primitive. PL sends a CHANNEL REQUEST message on RACH. The RR will send RANDOM ACCESS REQUESTs until an IMMEDIATE ASSIGNMENT message arrives.

(RR 3)

The PL sends RR an MPHC_RA_CON primitive, indicating that a CHANNEL REQUEST message has been sent out on RACH. Timer T3146 is started.

(RR 4)

The PL sends RR an MPHC_DATA_IND primitive, including an IMMEDIATE ASSIGNMENT message, but there was a wrong request reference or the second part of the message was not received.

(GRR 2)

When timer T3146 expires, RR sends GRR a RRGRR_STOP_CHANNEL_REQUEST_IND primitive.

(LLC 1)

The GRR sends LLC a GRR_STATUS_IND primitive which indicates an access failure

Requirements:

### Mobile originated packet access –B

LLC            GRR             RR             SIM            PL

|              |               |              |              |

| GRR_DATA_REQ |

*=============>*

(GRR 1)

| RRGRR_CHANNEL_REQ

*==============>*

(RR 1)

|MPHC_RA_REQ                  |

*============================>*

(PL 1)

|MPHC_RA_CON                  |

*<============================*

(RR 2)

|MPHC_RA_REQ                  |

*============================>*

(PL 2)

|MPHC_RA_CON                  |

*<============================*

(RR 3)

<* start T3146 *>

|MPHC_DATA_IND (IMMED.ASS.)   |

| (1st part)                   |

*<============================*

(RR 4)

<* start timer (2MFRMS)*>

|MPHC_DATA_IND (IMMED.ASS.)   |

| (2nd  part)                  |

*<============================*

(RR 5)

| RRGRR_IA_IND |

*<=============*

(GRR 2)

| RRGRR_IA_IND |

*<=============*

(GRR 3)

| RRGRR_STOP_CHANNEL_REQUEST_REQ

*=============>*

(RR 6)

|               |              |                             |

(GRR 1)

The LLC sends GRR a GRR_DATA_REQ primitive including LLC-pdu.

(RR 1)

With an RRGRR_CHANNEL_REQ primitive GRR orders RR to send RANDOM ACCESS REQUEST primitives.

(PL 1)

The RR sends PL an MPHC_RA_REQ primitive. PL sends a CHANNEL REQUEST message on RACH.

(RR 2)

The PL sends RR an MPHC_RA_CON primitive, indicating that a CHANNEL REQUEST message has been sent out on RACH.

(PL 2)

The RR sends PL an MPHC_RA_REQ primitive. PL sends a CHANNEL REQUEST message on RACH. The RR will send RANDOM ACCESS REQUESTs until an IMMEDIATE ASSIGNMENT message arrives.

(RR 3)

The PL sends RR an MPHC_RA_CON primitive, indicating that a CHANNEL REQUEST message has been sent out on RACH. Timer T3146 is started.

(RR 4)

The PL sends RR an MPHC_DATA_IND primitive, including the first part of an IMMEDIATE ASSIGNMENT message. Timer 2MFRMS is started.

(RR 5)

The PL sends RR a UNITDATA INDICATION primitive, including the second part of an IMMEDIATE ASSIGNMENT message.

(GRR 2)

The RR sends GRR an RRGRR_IA_IND (immediate assignment indication  1 part) primitive.

(GRR 3)

The RR sends GRR an RRGRR_IA_IND (immediate assignment indication  2 part) primitive.

(RR 6)

The GRR sends RR an RRGRR_STOP_CHANNEL_REQUEST_REQ primitive.

Requirements:

### Mobile originated packet access –B

LLC            GRR             RR             SIM            PL

|              |               |              |              |

| GRR_DATA_REQ |

*=============>*

(GRR 1)

| RRGRR_CHANNEL_REQ

*==============>*

(RR 1)

|MPHC_RA_REQ                  |

*============================>*

(PL 1)

|MPHC_RA_CON                  |

*<============================*

(RR 2)

|MPHC_RA_REQ                  |

*============================>*

(PL 2)

|MPHC_RA_CON                  |

*<============================*

(RR 3)

<* start T3146 *>

|MPHC_DATA_IND (IMMED.ASS.)   |

| (DL)                        |

*<============================*

(RR 4)

| RRGRR_IA_DOWNLINK_IND

*<============*

(GRR 2)

| RRGRR_STOP_CHANNEL_REQUEST_REQ

*=============>*

(RR 5)

|               |              |                             |

(GRR 1)

The LLC sends GRR a GRR_DATA_REQ primitive including LLC-pdu.

(RR 1)

With an RRGRR_CHANNEL_REQ primitive GRR orders RR to send RANDOM ACCESS REQUEST primitives.

(PL 1)

The RR sends PL an MPHC_RA_REQ primitive. PL sends a CHANNEL REQUEST message on RACH.

(RR 2)

The PL sends RR an MPHC_RA_CON primitive, indicating that a CHANNEL REQUEST message has been sent out on RACH.

(PL 2)

The RR sends PL an MPHC_RA_REQ primitive. PL sends a CHANNEL REQUEST message on RACH. The RR will send RANDOM ACCESS REQUESTs until an IMMEDIATE ASSIGNMENT message arrives.

(RR 3)

The PL sends RR an MPHC_RA_CON primitive, indicating that a CHANNEL REQUEST message has been sent out on RACH. Timer T3146 is started.

(RR 4)

The PL sends RR an MPHC_DATA_IND primitive, including an IMMEDIATE ASSIGNMENT (downlink) message.

(GRR 2)

The RR sends GRR an RRGRR_IA_DOWNLINK_IND (downlink immediate assignment) primitive.

(RR 5)

The GRR sends RR an RRGRR_STOP_CHANNEL_REQUEST_REQ primitive. . The uplink TBF will be established on the downlink TBF

Requirements:

### Mobile terminated downlink assignment –B

GRR           RR             SIM            PL

|             |              |              |

|MPHC_DATA_IND (IMMED.ASS.DL) |

*<============================*

(RR 1)

| RRGRR_IA_DOWNLINK_IND

*<=============*

(GRR 1)

|             |                             |

(RR 1)

The PL sends RR an MPHC_DATA_IND primitive including an IMMEDIATE ASSINGMENT (downlink) message.

(GRR 1)

The RR sends GRR an RRGRR_IA_DOWNLINK_IND (downlink immediate assignment) primitive.

Requirements:

### Mobile terminated downlink assignment –B

GRR           RR             SIM             PL

|             |              |              |

|MPHC_DATA_IND (IMMED.ASS.)   |

| (1st part DL)                |

*<============================*

(RR 1)

<* start timer (2MFRMS)/ start REORG*>

|MPHC_DATA_IND (IMMED.ASS.)   |

| (2nd part DL)                |

*<============================*

(RR 2)

<* page mode should be reset with page mode from last immed assign *>

| RRGRR_IA_DOWNLINK_IND

*<============*

(GRR 1)

| RRGRR_IA_DOWNLINK_IND

*<============*

(GRR 2)

|             |                             |

(RR 1)

The PL sends RR an MPHC_DATA_IND primitive including first part of a two part IMMEDIATE ASSINGMENT (downlink) message.

(RR 2)

The PL sends RR an MPHC_DATA_IND primitive including second part of a two part IMMEDIATE ASSINGMENT (downlink) message.

(GRR 1)

The RR sends GRR an RRGRR_IA_DOWNLINK_IND (downlink immediate assignment  1 part) primitive.

(GRR 2)

The RR sends GRR an RRGRR_IA_DOWNLINK_IND (downlink immediate assignment  2 part) primitive.

Requirements:

### Abnormal cases

LLC                 GRR                     MAC                                PL

|                   |                       |                                  |

- a) TLLI mismatch has occurred during the contention resolution procedure

|                   |                       |                                  |

| GRR_STATUS_IND    |                       |                                  |

*<===================*                       |                                  |

(LLC 1)              |                       |                                  |

|                   |                       |                                  |

|                   |                       |                                  |

- b) The MS receives an immediate assignment message

|                   |                       |                                  |

|                   |                  MPHC_DATA_IND                           |

|                   |             (IMMEDIATE ASSIGNMENT)                       |

|                   *<=========================================================*

|                 (GRR 1)                   |                                  |

|                   |                       |                                  |

b.1) Information in the MS does not satisfactorily define a PDCH

|                   |                       |                                  |

b.2) Mobile allocation indicates frequencies in more than one frequency band

|                   |                       |                                  |

b.3) The received message indicates a PDCH in a non supported frequency

|                   |                       |                                  |

(LLC 1)

TLLI mismatch has occurred. GRR informs the LLC via GRR_STATUS_IND. The cause parameter contains TBF establishment failure.

(GRR 1)

GRR receives an immediate assignment message. If one of the four abnormal cases occurs before the contention resolution procedure is completed, the MS returns to the packet idle mode and notifies upper layers.

Requirements:

<R.MAC.AB_IMMAS.M.00x>

## Packet downlink assignment procedure on CCCH

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                  MPHC_DATA_IND                           |

|                   |             (IMMEDIATE ASSIGNMENT)                       |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

- a) If the frequency hopping is applied, the network may use the indirect or the direct encoding of the frequency configuration.

|                   |                       |                                  |

- b) If the indirect encoding is used, the MS uses information received in SI or stored from a previous assignment to determine the frequency parameters.

|                   |                       |                                  |

- c) If the direct encoding is used, the MS uses the cell allocation defined for the cell to decode the mobile allocation.

|                   |                       |                                  |

- d) The message indicates a TBF starting time

|                   |                       |                                  |

d.1) The message is received before the starting time has expired

(GRR 2)              |                       |                                  |

|                   |                       |                                  |

d.2) The message is received after the starting time has expired

(GRR 3)              |                       |                                  |

|                   |                       |                                  |

- e) The message indicates a timing advance index (TAI)

(GRR 4)              |                       |                                  |

|                   |                       |                                  |

(GRR 1)

The GRR receives an immediate assignment message which indicates a downlink TBF. If the MS applies DRX mode, the message will be received on CCCH block corresponding to a paging group, otherwise on the CCCH timeslot corresponding to CCCH group the MS belongs to. The MS stops monitoring downlink CCCH and switches to the assigned PDCH and starts listening for downlink RLC/MAC blocks identified by the TFI; it starts the timer T3190.

(GRR 2)

The GRR waits until the frame number indicated by the TBF starting time, starts timer T3190 and switches to the assigned PDCH.

(GRR 3)

The GRR ignores the starting time and starts timer T3190 and switches to the assigned PDCH.

(GRR 4)

If there is a TAI indicated in the immediate assignment message, the GRR uses the continuous update timing advance procedure, using PTCCH in the same timeslot as the assigned PDCH. Otherwise, the GRR does not use the continuous update timing advance procedure.

Requirements:

<R.MAC.INT_PDWL.M.00x>

### Packet downlink assignment completion

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                   |                       |              RLC_downlink        |

|                   |                       *<---------------------------------*

|                   |                    (MAC 1)                               |

(MAC 1)

The packet downlink assignment procedure is completed when the MAC receives an RLC/MAC block identified by the assigned TFI. The GRR stops timer T3190.

Requirements:

<R.MAC.DWL_BLK.M.005>, <R.MAC.DWL_BLK.M.006>, <R.MAC.RR_DWL.M.004>

### Packet downlink assignment abnormal cases

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

a) T3190 expires

|                   |                     (MAC 1)                              |

(MAC 1)

The timer T3190 expires before receiving a valid RLC/MAC block. The MS returns to the packet idle mode, TBF is released. Other abnormal cases are the same cases which are listed for the process packet uplink assignment on CCCH.

Requirements:

<R.MAC.AB_DWL.M.00x>, <R.MAC.DWL_COMP.M.002>, <R.MAC.DWL_BLK.M.007>, <R.MAC.RR_DWL.M.005>

### Sending an RLC/MAC control message: single block packet downlink assignment procedure

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

(GRR 1)

(GRR 1)

Assumption: The GRR has received an immediate assignment message which contains a single block allocation structure. The GRR switches to the assigned PDCH and attempt to decode an RLC/MAC control message in the assigned downlink block. The actions done by having a TBF starting time are the same actions done in different procedures.

Requirements:

<R.MAC.SB_PDWL.M.00x>

## MAC procedures in uplink packet transfer mode

### Resource Reallocation (Uplink)

GRR                 RLC                     MAC                                PL

|                   |                       |            RLC_uplink            |

|                   |                       |    (PACKET RESOURCE REQUEST)     |

|                   |                       |    (PACKET DOWNLINK ACK/NACK)    |

|                   |                       *<---------------------------------*

|                   |                       |                             (PL 1)

|                   |                       |                                  |

(PL 1)

The MS requests new resources.

Requirements:

<R.MAC.UPL_RLCD.M.006-11>, <R.MAC.RR_UPL.M.001-4>, <R.MAC.RR_UPL.M.009>, <R.MAC.RR_UPL.M.0015>, <R.MAC.R_OE_TBF.M.001-2>, <R.MAC.BEGIN_FX.M.001-2>, <R.MAC.NC_PWR.M.001-3>

### Resource Reallocation for new LLC PDUs with different RLC mode and higher priority(Uplink)

GRR                 RLC                     MAC                                PL

|                   |                       |            RLC_uplink            |

|                   |                       |        (RLC DATA BLOCK)          |

|                   |                       *=================================>*

|                   |                       |                             (PL 1)

|                       MPHP_TBF_RELEASE_REQ(UL)                               |

*=============================================================================>*

|                   |                       |                             (PL 2)

|                       MPHP_TBF_RELEASE_CON                                   |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                                   MPHP_RA_REQ                                |

*=============================================================================>*

|                   |                       |                             (PL 3)

|                   |                       |                                  |

(PL 1)

The MS completes the transmission of the current LLC PDU.

(PL 2)

The MS releases the current TBF.

(GRR 1)

The MS confirms the release of the TBF.

(PL 3)

The MS starts a new TBF for the new LLC PDUs. When the sending of LLC PDUs with higher priority is complete the MS shall establish an uplink TBF for the remaining LLC PDUs.

Requirements:

<R.MAC.RR_UPL.M.005-7>

### On expiry if Timer T3168

GRR                 RLC                     MAC                                PL

|                   |                       *<Timeout T3168>                   |

|                   |                    (MAC 1)                               |

a)The Packet Resource Request message has not been sent 4 times.

|                   |                       |            RLC_uplink            |

|                   |                       |    (PACKET RESOURCE REQUEST)     |

|                   |                       *=================================>*

|                   |                       |                             (PL 1)

b) The Packet Resource Request message has already been sent 4 times.

|                       MPHP_TBF_RELEASE_REQ(UL)                               |

*=============================================================================>*

|                   |                       |                             (PL 2)

|                       MPHP_TBF_RELEASE_CON                                   |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

(MAC 1)

Timer T3168 expires.

(PL 1)

The MS retransmits the Packet Resource Request message.

(PL 2)

The MS performs an abnormal release with random access.

(GRR 1)

The MS confirms the release of the TBF.

Requirements:

<R.MAC.RR_UPL.M.010>, <R.MAC.R_OE_TBF.M.003

### Reaction of the network to the resource reallocation request

GRR                 RLC                     MAC                                PL

a) The MS receives a PACKET UPLINK ASSIGNMENT or PACKET TIMESLOT RECONFIGURE

|                   |                       |         MPHP_DATA_IND            |

|                   |                       |              ( - )               |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

b) The MS receives a Packet access reject

|                   |                       |         MPHP_DATA_IND            |

|                   |                       |      (PACKET ACCESS REJECT)      |

|                   |                       *<=================================*

|                   |                    (MAC 2)                               |

|                   |                       |                                  |

(MAC 1)

MAC receives new resources. It resumes as described in the packet uplink assignment procedure.

(MAC 2)

MAC receives a packet access reject message. It continues as described as in the packet access reject procedure.

Requirements:

<R.MAC.RR_UPL.M.009>, <R.MAC.RR_UPL.M.011-14>, <R.MAC.END_TBF.M.001>, <R.MAC.A_FX_UPL.M.003>

### Abnormal Cases

GRR                 RLC                     MAC                                PL

|                   |                       |          MPHP_DATA_IND           |

|                   |                       |     (PACKET UPLINK ASSIGNMENT)   |

|                   |                       |   (PACKET TIMESLOT RECONFIGURE)  |

|                   |                       |   (PACKET DOWNLINK ASSIGNMENT)   |

|                   |                       |      (PACKET UPLINK ACK/NACK)    |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

|                       MPHP_TBF_RELEASE_REQ(UL)                               |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                       MPHP_TBF_RELEASE_CON                                   |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

(MAC 1)

MAC receives new resources. Parameter error occurs.

(PL 1)

The MS performs an abnormal release and resumes with either system information reading or random access.

(GRR 1)

The MS confirms the release of the TBF.

Requirements:

<R.MAC.AB_PACCH.M.001-3>, <R.MAC.A_FX_UPL.M.001>, <R.MAC.A_FX_UPL.M.003-5>

### Establishment of Downlink TBF

GRR                 RLC                     MAC                                PL

|                   |                       |         MPHP_DATA_IND            |

|                   |                       |   (PACKET DOWNLINK ASSIGNMENT)   |

|                   |                       |  (PACKET TIMESLOT RECONFIGURE)   |

|                   |                       |      (PACKET UPLINK ACK/NACK)    |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

|                           MPHP_ASSIGNMENT_REQ                                |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                           MPHP_ASSIGNMENT_CON                                |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

(MAC 1)

MAC receives during the uplink a PACKET DOWNLINK ASSIGNMENT or a PACKET TIMESLOT RECONFIGURE.

(PL 1)

The downlink allocation is indicated to layer 1.

(GRR 1)

The downlink allocation is confirmed by layer 1.

Requirements:

<R.MAC.DWL_TBF.M.001>, <R.MAC.DWL_TBF.A.004-5>, <R.MAC.FX_DL_T.M.00x>, <R.MAC.A_FX_UPL.M.003>, <R.MAC.C_TBF_HD.M.00x>, <R.MAC.F2_R_DWL.M.001>, <R.MAC.Q_R_DWL.M.001>

### Abnormal cases during downlink assignment

GRR                 RLC                     MAC                                PL

|                   |                       |         MPHP_DATA_IND            |

|                   |                       |   (PACKET DOWNLINK ASSIGNMENT)   |

|                   |                       |  (PACKET TIMESLOT RECONFIGURE)   |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

a)if the MS receives a PACKET TIMESLOT RECONFIGURE message

|                       MPHP_TBF_RELEASE_REQ(UL)                               |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                       MPHP_TBF_RELEASE_CON                                   |

*<=============================================================================*

(GRR 1)              |                       |                                  |

b) if the MS receives a PACKET DOWNLINK ASSIGNMENT message

|                   |                       |                                  |

(MAC 1)

MAC receives during the uplink TBF a PACKET DOWNLINK ASSIGNMENT or a PACKET TIMESLOT RECONFIGURE.

(PL 1)

In case a) due to failures (see requirements) the uplink is released. A new random access is performed. In case b) the aborts or ignores the downlink assignment procedure and continues with uplink assignment.

(GRR 1)

The MS confirms the release of the TBF.

Requirements:

<R.MAC.AB_DWLTF.M.001-5>, <R.MAC.END_TBF.M.002>, <R.MAC.AB_TBF.M.00x>, <R.MAC.AB_FX_DL.M.00x> <R.MAC.AB.M.001>

### Reallocation of the fixed Allocation

GRR                 RLC                     MAC                                PL

|                   |                       |         MPHP_DATA_IND            |

|                   |                       |     (PACKET UPLINK ACK/NACK)     |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

|                 MPHP_REPEAT_UL_FIXED_ALLOC_REQ                               |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                 MPHP_REPEAT_UL_FIXED_ALLOC_CON                               |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

(MAC 1)

MAC receives during the uplink TBF with fixed mode an PACKET UPLINK ACK/NACK message indicating a repeat allocation.

(PL 1)

The reallocation is indicated to layer 1.

(GRR 1)

The reallocation is confirmed by layer 1.

Requirements:

<R.MAC.R_REA_RQ.M.004-6>, <R.MAC.EXH_ALOC.M.001-2>, <R.MAC.A_FX_UPL.M.003>

### On expiry if Timer T3188

GRR                 RLC                     MAC                                PL

|                   |                       *<Timeout T3188>                   |

|                   |                    (MAC 1)                               |

|                       MPHP_TBF_RELEASE_REQ(UL)                               |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                       MPHP_TBF_RELEASE_CON                                   |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

(MAC 1)

Timer T3188 expires.

(PL 1)

The MS performs an abnormal release with random access.

(GRR 1)

The MS confirms the release of the TBF.

Requirements:

<R.MAC.EXH_ALOC.003>

### Network initiated release of uplink TBF

GRR                 RLC                     MAC                                PL

|                   |                       |         MPHP_DATA_IND            |

|                   |                       |       (PACKET TBF RELEASE)       |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

a)If “normal release” is indicated and (extended) dynamic allocation is used, the TBF is continued to the next LLC PDU boundary. At the boundary the CV counts down to zero. After then the TBF is released(see (c)).

b)If “abnormal release” is indicated the TBF is immediately released. In this case

a random access procedure is followed.

|                       MPHP_TBF_RELEASE_REQ(UL)                               |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                       MPHP_TBF_RELEASE_CON                                   |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

(MAC 1)

The MS receives a PACKET TBF RELEASE message.

(PL 1)

The MS performs an abnormal release with random access.

(GRR 1)

The MS confirms the release of the TBF.

Requirements:

<R.MAC.NREL_TBF.I.001-4>

## MAC procedures in downlink packet transfer mode

### Downlink assignment

GRR                 RLC                     MAC                                PL

|                   |                       |   MPHP_DATA_IND/RLC_downlink     |

|                   |                       |  (PACKET DOWNLINK ASSIGNMENT)    |

|                   |                       |  (PACKET TIMESLOT RECONFIGURE)   |

|                   |                       *<=================================*

|                   |                       |                             (PL 1)

|                           MPHP_ASSIGNMENT_REQ                                |

*=============================================================================>*

|                   |                       |                             (PL 2)

|                           MPHP_ASSIGNMENT_CON                                |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

(PL 1)

The MS receives new downlink resources.

(PL 2)

The downlink allocation is indicated to layer 1.

(GRR 1)

The downlink allocation is confirmed by layer 1.

Requirements:

<R.MAC.DWL_BLK.M.001-4>

### Polling for Downlink ACK/NACK

GRR                 RLC                     MAC                                PL

|                   |                       |          RLC_downlink            |

|                   |                       |    (RLC DATA BLOCK valid RRBP)   |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

|                   |                       |       MPHP_SINGLE_BLOCK_REQ      |

|                   |                       |    (PACKET DOWNLINK ACK/NACK)    |

|                   |                       *=================================>*

|                   |                       |                             (PL 1)

|                   |                       |       MPHP_SINGLE_BLOCK_CON      |

|                   |                       *<=================================*

|                   |                    (MAC 2)                               |

|                   |                       |                                  |

(MAC 1)

MAC receives a RLC data block including a valid RRBP field(indicates the MS a single block for transmitting a PACKET DOWNLINK ACK/NACK message to the network).

(PL 1)

The PACKET DOWNLINK ACK/NACK message is sent by a single block request.

(MAC 2)

The single block transmission is confirmed by layer 1.

Requirements:

<R.MAC.PL_PNACK.M.001>, <R.MAC.PL_PNACK.M.003-6>, <R.MAC.PL_PNACK.A.008-9>

### Suspending the Downlink TBF !!! REMOVED !!!

### Resource Reassignment for Downlink TBF

GRR                 RLC                     MAC                                PL

|                   |                       |           RLC_downlink           |

|                   |                       |  (PACKET DOWNLINK ASSIGNMENT)    |

|                   |                       |  (PACKET TIMESLOT RECONFIGURE)   |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

|                   |                       |                                  |

(MAC 1)

The MS receives a change in resources in the current TBF.

Requirements:

<R.MAC.RR_DWL.M.001-3>

### Establishment of uplink TBF

GRR                 RLC                     MAC                                PL

|                   |                       |       MPHP_SINGLE_BLOCK_REQ      |

|                   |                       |    (PACKET DOWNLINK ACK/NACK)    |

|                   |                       *=================================>*

|                   |                       |                             (PL 1)

|                   |                       |       MPHP_SINGLE_BLOCK_CON      |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

|                   |                       |                                  |

(PL 1)

The MS requests a uplink TBF in the PACKET DOWNLINK ACK/NACK.

(MAC 1)

The single block transmission is confirmed by layer 1.

Requirements:

<R.MAC.DWL_UPL.M.001-5>, <R.MAC.DWL_UPL.M.015>, <R.MAC.C_TBF_HD.M.00x>, <R.MAC.F2_UPL.M.001>, <R.MAC.F2_R_UPL.M.003>, <R.MAC.RR_DL_DL.M.00x>

### Receiving of uplink resources

GRR                 RLC                     MAC                                PL

|                   |                       |           RLC_downlink           |

|                   |                       |    (PACKET UPLINK ASSIGNMENT)    |

|                   |                       |  (PACKET TIMESLOT RECONFIGURE)   |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

|                           MPHP_ASSIGNMENT_REQ                                |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                           MPHP_ASSIGNMENT_CON                                |

*<=============================================================================*

(GRR 1)              |                       |                                  |

|                   |                       |                                  |

|                   |                       |                                  |

(MAC 1)

The MS receives uplink resources.

(PL 1)

The uplink allocation is indicated to layer 1.

(GRR 1)

The uplink allocation is confirmed by layer 1.

Requirements:

<R.MAC.DWL_UPL.M.007-8>, <R.MAC.DWL_UPL.M.011-12>, <R.MAC.DWL_UPL.M.0016>, <R.MAC.Q_R_UPL.M.002>

### Receiving of packet access reject

GRR                 RLC                     MAC                                PL

|                   |                       |           RLC_downlink           |

|                   |                       |        (PACKET ACCESS REJECT)    |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

a)including a WAIT_INDICATION: wait for uplink assignment until expiry of T3172(WAIT_INDICATION value). On expiry of T3172 the continues with establishment of uplink assignment(either 4.15.5 or packet access procedure).

b)not including a WAIT_INDICATION: the MS performs a abnormal release with system information reading.

|                   |                       |                                  |

|                   |                       |                                  |

(MAC 1)

The MS receives PACKET ACCESS REJECT message.

Requirements:

<R.MAC.DWL_UPL.M.013-14>

### On expiry if Timer T3168

GRR                 RLC                     MAC                                PL

|                   |                       *<Timeout T3168>                   |

|                   |                   (MAC 1)                                |

- The MS retransmits the channel request description IE in the next PACKET DOWNLINK ACK/NACK message(see 4.15.5).

- If it has been transmitted four times the MS performs an abnormal release with random access.

|                   |                       |                                  |

(MAC 1)

Timer T3168expires.

Requirements:

<R.MAC.DWL_UPL.M.015>

### Abnormal cases

GRR                 RLC                     MAC                                PL

|                   |                       |           RLC_downlink           |

|                   |                       |    (PACKET UPLINK ASSIGNMENT)    |

|                   |                       |  (PACKET TIMESLOT RECONFIGURE)   |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

a)The MS ignores this message.

b)The MS performs an abnormal release with random access.

|                   |                       |                                  |

(MAC 1)

The MS receives uplink resources. A failure occurs.

Requirements:

<R.MAC.A_DWLUPL.M.00x>, <R.MAC.AB.M.002>, <R.MAC.RR_DL_AB.M.00x>

### Network initiated downlink TBF release

GRR                 RLC                     MAC                                PL

|                   |                       |            RLC_uplink or         |

|                   |                       |       SINGLE BLOCK REQUEST       |

|                   |                       |    (PACKET DOWNLINK ACK/NACK)    |

|                   |                       *=================================>*

|                   |                       |                             (PL 1)

|                   |                       |                                  |

(MAC 1)

The MS initiates the release of the downlink TBF by setting the TBF_RELEASE bit in the PACKET DOWNLINK ACK/NACK message.

Requirements:

<R.MAC.NT_DWL_R.M.001>

## Packet PDCH Release

### On reception of PACKET PDCH RELEASE

GRR                 RLC                     MAC                                PL

|                   |                       |          RLC_downlink            |

|                   |                       |      (PACKET PDCH RELEASE)       |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

a)The TIMESLOT_AVAILABLE parameter is indicated to layer 1.

|                           MPHP_PDCH_RELEASE_REQ                              |

*=============================================================================>*

|                   |                       |                             (PL 2)

|                           MPHP_PDCH_RELEASE_CON                              |

*<=============================================================================*

(GRR 1)              |                       |                                  |

b)If all uplink PDCHs are removed, the MS performs an abnormal release with random access. If all downlink PDCHs are removed, the MS returns to packet idle mode.

|                   |                       |                                  |

(PL 1)

The MS receives a PACKET PDCH RELEASE message.

(PL 2)

The TIMESLOT_AVAILABLE is indicated to layer 1.

(GRR 1)

The TIMESLOT_AVAILABLE is confirmed by layer 1.

Requirements:

<R.MAC.PDCH_REL.M.00x>

## Cell change procedure in packet transfer mode

### On reception of PACKET CELL CHANGE ORDER

GRR                 RLC                     MAC                                PL

|                   |                       |            RLC_downlink          |

|                   |                       |    (PACKET CELL CHANGE ORDER)    |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

Tasks:

1)abort any TBF

1)Read BCCH and PBCCH

2)If Uplink was in progress, the MS performs a random access for the remaining data. Otherwise it enters the packet idle mode.

|                   |                       |                                  |

(MAC 1)

The MS receives the PACKET CELL CHANGE ORDER message.

Requirements:

<R.MAC.CC_TRF.M.00x>

### Network controlled cell reselection completion

GRR                 RLC                     MAC                                PL

|                   |                       |    MPHP_DATA_IND/RLC_downlink    |

|                   |                       |      (Immediate Assignment)      |

|                   |                       |    (Packet Uplink Assignment)    |

|                   |                       *<=================================*

|                   |                    (MAC 1)                               |

|                   |                       |                                  |

(MAC 1)

The MS receives a response to its channel request or packet channel request.

Requirements:

<R.MAC.NC_CRES.M.001>

### Failure on the new cell or Timeout of T3174

GRR                 RLC                     MAC                                PL

a)Timer T3174 expires.

|                   |                       *<Timeout T3174>                   |

|                   |                    (MAC 1)                               |

b)The MS receives a reject in the new cell.

|                   |                       |            MPHP_DATA_IND         |

|                   |                       |   (Immediate Assignment Reject)  |

|                   |                       |       (Packet Access Reject)     |

|                   |                       *<=================================*

|                   |                    (MAC 2)                               |

c)Contention Resolution fails.

d)The MS is not capable to use the frequency indicated in the PACKET CELL CHANGE OREDER message.

If one of these cases occurs, the MS switches to the old cell.

|                   |                       |            RLC_uplink            |

|                   |                       |      SINGLE BLOCK REQUEST        |

|                   |                       |   (PACKET CELL CHANGE FAILURE)   |

|                   |                       *=================================>*

|                   |                       |                             (PL 1)

|                   |                       |                                  |

(MAC 1)

Timer T3174 expires.

(MAC 2)

The MS receives a negative response to its channel request or packet channel request.

(PL 1)

The MS transmits the PACKET CELL CHANGE FAILURE message to the network. If uplink was in progress, the message is transmitted in the RLC_uplink message. Other wise the MS starts random access procedure and request single block. On reception of single block assignment, the transmits the PACKET CELL CHANGE FAILURE message. Timer T3716 is started.

Requirements:

<R.MAC.NC_CRES.M.003-5>, <R.MAC.ABCC_TRF.M.001>

### Timeout of Timer T3176

GRR                 RLC                     MAC                                PL

|                   |                       *<Timeout T3176>                   |

|                   |                    (MAC 1)                               |

Tasks:

a)Uplink was in progress: The MS performs an abnormal release with random access.

b)No Uplink in progress: The MS performs an abnormal release and returns to the packet idle mode.

|                   |                       |                                  |

(MAC 1)

Timer T3176 expires.

Requirements:

<R.MAC.NC_CRES.M.006>

## Packet Control Acknowledgement

GRR                 RLC                     MAC                                PL

|                   |                       |            RLC_uplink            |

|                   |                       | (PACKET CONTROL ACKNOWLEDGEMENT) |

|                   |                       *<---------------------------------*

|                   |                       |                             (PL 1)

|                   |                       |                                  |

(PL 1)

The network sends the PACKET CONTROL ACKNOWLEDGEMENT message.

Requirements:

<R.MAC.PC_ACK.M.001>

## Receiving a PACKET PAGING REQUEST

### On PCCCH_GROUP

GMM                                         GRR                                PL

|                                           |                                  |

|                                           |     (PACKET PAGING REQUEST)      |

|                                           *<=================================*

|                                        (GRR 1)                               |

|                                           |                                  |

a) Indicating packet downlink establishment

|                                           |                                  |

|         (GMMRR_PAGE_IND)                  |                                  |

*<===========================================*                                  |

(GMM 1)                                     |                                  |

|                                           |                                  |

b) Indicating RR connection establishment

|                                        (GRR 2)                               |

|                                           |                                  |

|                                           |                                  |

(GRR 1)

The network sends the PACKET PAGING REQUEST to the MS in the PCCCH_GROUP.

(GMM 1)

The GRR indicates the paging request to the GMM.

(GRR 2)

The GRR indicates the paging request for RR connection establishment to the RR (except MSs in class C).

### On PACCH

GMM                                         GRR                                PL

|                                           |                                  |

|                                           |     (PACKET PAGING REQUEST)      |

|                                           *<=================================*

|                                           |      (RR connection est.)        |

|                                        (GRR 1)                               |

(GRR 2)

The GRR indicates the paging request for RR connection establishment to the RR (except MSs in class B or C).

## Cell indication

GMM                               GRR                     RR                  PL

|                                 |                       |                   |

|                                 |     (RRGRR_SI13_IND)  |                   |

|                                 *<======================*                   |

|                              (GRR 1)                    |                   |

|                                 |                       |                   |

- a) PBCCH present

|                                 |              MPHP_SCELL_PBCCH_REQ         |

|                                 |                 (read PSI1 and PSI2)      |

|                                 *==========================================>*

|                                 |                       |               (PL 1)

|                                 |                   MPHP_DATA_IND           |

|                                 |                 (PSI1 and/or PSI2)        |

|                                 *<==========================================*

|         (GMMRR_CELL_IND)      (GRR 2)                   |                   |

*<=================================*                       |                   |

(GMM 1)                           |                       |                   |

|                                 |                       |                   |

- b) PBCCH not present

|                                 |                       |                   |

|         (GMMRR_CELL_IND)        |                       |                   |

*<=================================*                       |                   |

(GMM 2)                           |                       |                   |

|                                 |                       |                   |

(GRR 1)

GRR receives RRGRR_SI13_IND from the RR.

(PL 1)

PL reads PSI1 and PSI2

(GRR 2)

GRR receives PSI1 and/or PSI2.

(GMM 1)

GRR indicates the cell information to the GMM. Note: If the Routing Area Code changes, GRR shall also send a GMMRR_CELL_IND to the GMM.

(GMM 2)

There is no PBCCH present, GRR sends GMMRR_CELL_IND to the GMM.

## Abnormal Release

GRR                 RLC                     MAC                                PL

|                   |                       |                                  |

|                    MPHP_TBF_RELEASE_REQ(UL & DL)                             |

*=============================================================================>*

|                   |                       |                             (PL 1)

|                    MPHP_TBF_RELEASE_CON                                      |

*<=============================================================================*

(GRR 1)              |                       |                                  |

Tasks:

a)Return to the packet idle mode.

b)Perform a new packet access procedure.

c)Return to BCCH and reread all relevant BCCH and/or PBCCH information

|                   |                       |                                  |

(PL 1)

The MS releases the downlink and/or uplink TBF, if it was in progress.

(GRR 1)

Layer 1 confirms the release of the uplink and/or downlink TBF .

Requirements:

<R.MAC.AB.M.00x>, <R.MAC.AB_R_CCH.M.00x>, <R.MAC.AB_R_RAC.M.001>, <R.MAC.AB_R_SYS.M.00x>

## System information on PBCCH

GRR                                                                            PL

|                                                                              |

|                    MPHP_SCELL_PBCCH_REQ (read all PSI)                       |

*=============================================================================>*

|                                                                         (PL 1)

|                                                                              |

|                    MPHP_DATA_IND (PSI1)                                      |

<==============================================================================*

(GRR 1)

|                                                                              |

|                    MPHP_DATA_IND (PSI2)                                      |

<==============================================================================*

(GRR 2)

|                                                                              |

...

|                                                                              |

|                    MPHP_DATA_IND (PSIN)                                      |

<==============================================================================*

(GRR N)

|                    MPHP_SCELL_PBCCH_STOP_REQ                                 |

*=============================================================================>*

|                                                                         (PL 1)

|                                                                              |

(PL 1)

GRR informs the PL to read all PSI messages (complete acquisition).

(GRR 1)

GRR receives PSI1 message. GRR starts two timers: 60 second and 30 second timer. These timers are restarted everytime if the GRR receives PSI1 or (P)SI13 messages on PACCH or PBCCH. GRR checks the acquisition state whether it should stop the PSI reading procedure or not.

(GRR 2)

GRR receives PSI2 message. GRR checks the acquisition state whether it should stop the PSI reading procedure or not.

(GRR N)

GRR receives PSIN message. GRR checks the acquisition state whether it should stop the PSI reading procedure or not. If the acquisition completed, then the GRR stops the process of reading PSI messages.

(PL 2)

PL stops reading PSI messages.

### Supervision of PBCCH_CHANGE_MARK and update of PBCCH information

GRR                                                                            PL

|                                                                              |

- Timer 30 second expires

|                    MPHP_SCELL_PBCCH_REQ ( read PSI1)                         |

*=============================================================================>*

|                                                                         (PL 1)

|                                                                              |

|                    MPHP_DATA_IND (PSI1)                                      |

<==============================================================================*

(GRR 1)

|                                                                              |

a) PBCCH_CHANGE_MARK has been incremented by one

|                    MPHP_SCELL_PBCCH_REQ ( partial acq.)                      |

*=============================================================================>*

|                                                                         (PL 2)

|                                                                              |

|                                                                              |

b) PBCCH_CHANGE_MARK has been incremented by more than one

|                    MPHP_SCELL_PBCCH_REQ ( complete acq.: read all)           |

*=============================================================================>*

|                                                                         (PL 3)

|                                                                              |

(PL 1)

GRR informs the PL to read PSI1 message.

(GRR 1)

GRR receives PSI1 message. GRR restarts 60 and 30 second timers. GRR supervises PBCCH_CHANGE_MARK.

(PL 2)

PL should perform partial acquisition. The PSI to acquire has been computed in GRR and passed to PL.

(PL 3)

PL should perform complete acquisition.

### Replacement of PBCCH

GRR                     RR/TIL

|                         |

- PSI1 or PSI13 indicates that the PBCCH is no longer valid in the cell

|    RRGRR_START_MON_BCCH_REQ

*========================>*

|                     (RR/TIL 1)

|                         |

|    RRGRR_SI13_IND       |

*<========================*

(GRR 1)                   |                                                   PL

|                         |                                                    |

|                         |                                                    |

a)SI13 contains PBCCH description: perform complete acq. of PBCCH

|                         |                                                    |

|                  MPHP_SCELL_PBCCH_REQ (read all PSI: complete acq.)          |

*=============================================================================>*

|                         |                                               (PL 1)

b)SI13 contains PBCCH description: perform complete acq. Of BCCH

|    RRGRR_START_MON_BCCH_REQ                                                  |

*========================>*                                                    |

|                     (RR/TIL 2)                                               |

|                         |                                                    |

|                         |                                                    |

|                         |                                                    |

(RR/TIL 1)

After receiving PSI1 or PSI13 GRR knows that the PBCCH has been no longer available. Therefore it starts to receive SI13 message via RR/TIL.

(GRR 1)

SI13 has been received

(PL 1)

SI13 contains a PBCCH description: GRR should perform a complete acq. of PSI messages.

(PL 1)

SI13 contains no PBCCH description: GRR should perform a complete acq. of SI messages on BCCH.

### PSI1 reception failure

If the PSI1 message has not been received within 60 seconds, perform cell reselection. To be done (TBD).

## System information on BCCH

GRR                     RR/TIL

|                         |

|                         |

|    RRGRR_SI13_IND       |

*<========================*

(GRR 1)                   |

|                         |

- SI13 without a PBCCH description: receive SI on BCCH

|                         |

| RRGRR_START_MON_BCCH_REQ|

*========================>*

|                     (RR/TIL 1)

|    RRGRR_SI13_IND       |

*<========================*

(GRR 2)                   |

|                         |

| RRGRR_SI_STATE_IND      |

*<========================*

(GRR 3)                   |

(GRR 1)

GRR receives SI13 which contains no PBCCH description. GRR should perform complete acq. of SI messages on BCCH.

(RR/TIL 1)

TIL has to perform complete acq. of SI messages.

(GRR 2)

RR/TIL may send SI13 to GRR..

(GRR 3)

GRR gets the information about SI acq. state (to allow packet access).

### Supervision of BCCH_CHANGE_MARK and update of BCCH information

Supervision BCCH_CHANGE_MARK has the same logical way as the supervision of PBCCH_CHANGE_MARK.

### Establishment of PBCCH

GRR                     RR/TIL

|                         |

|                         |

|    RRGRR_SI13_IND       |

*<========================*

(GRR 1)                   |

|                         |

SI13 with a PBCCH description: perform complete acq.

|                                                                           PL

|                       MPHP_SCELL_PBCCH_REQ  (perform complete acq.)        |

*===========================================================================>*

|                                                                            |

|                                                                            |

|                                                                            |

(GRR 1)

GRR receives SI13 which contains no PBCCH description. GRR should perform complete acq. of SI messages on BCCH.

### SI13 reception failure

If the SI13 or PSI13 message has not been received within 60 seconds, perform cell reselection (TBD).

## Page mode procedures on PCCCH

A mobile station in packet transfer mode shall not consider the page mode information received in any message.

GRR                                                                            PL

|                                                                              |

|                    MPHP_DATA_IND ( message corresponding to paging group)    |

*<=============================================================================*

(GRR 1)                                                                        |

|                                                                              |

- a) page mode has changed to normal paging

|                                                                              |

|                                                                              |

|                    MPHP_PCCCH_START_REQ (page mode=normal paging)            |

*=============================================================================>*

|                                                                         (PL 1)

|                                                                              |

|                                                                              |

- b) page mode has changed to extended paging

|                                                                              |

|                                                                              |

|                    MPHP_PCCCH_START_REQ (page mode=extended paging)          |

*=============================================================================>*

|                                                                         (PL 2)

|                                                                              |

|                                                                              |

- c) page mode has changed to paging reorganization

|                                                                              |

|                                                                              |

|                    MPHP_PCCCH_START_REQ (page mode= paging reorganization)   |

*=============================================================================>*

|                                                                         (PL 3)

|                                                                              |

|                                                                              |

- d) page mode has not changed = same as before: nothing to do

|                                                                              |

(GRR 1)

GRR receives any control message in the corresonding paging group. GRR takes the page mode information in this message into account.

(PL 1)

GRR starts paging procedure with normal paging in PL.

(PL 2)

GRR starts paging procedure with extended paging in PL.

(PL 3)

GRR starts paging procedure with paging reorganization PL.

## Page mode procedures on CCCH

This part describes the paging procedures on CCCH. It can be divided to two parts. One for initiation of RR connection establishment and one for downlink packet transfer.

### Paging procedure for RR connection establishment

This procedure has only meaning in the GRR part if the MS class is not CG. Because CG mobiles can not accept or receive RR establishment request. This type of MSs are only GPRS capable MSs

GMM                       GRR                       RR/TIL

|                         |                            |

|                         |                            |

|                         |    RRGRR_RR_EST_IND        |

|                         *<===========================|

|                        (GRR 1)                       |

|   GMMRR_CS_PAGE_IND     |                            |

*<=========================|                            |

(GMM 1)                                                                      |

|                                                                            |

|                         |                            |

|  GMMRR_CS_PAGE_RES      |                            |

|=======================>*|                            |

|                        (GRR 2)                       |

|                         |                            |

a.1) RR connection establishment accepted, suspension not needed

|                         |                            |

|                         |    RRGRR_RR_EST_RSP (no suspension needed)

|                         |==========================>*|

|                         |                          (RR/TIL 1)

|                         |                            |

a.2) RR connection establishment accepted, suspension needed

|                         |                            |

|                         |    RRGRR_RR_EST_RSP (suspension needed)

|                         |==========================>*|

|                         |                          (RR/TIL 2)

|                         |                            |

|                         |                            |

- b) RR connection establishment rejected

|                         |                            |

|                         |    RRGRR_RR_EST_RSP (rejected )

|                         |==========================>*|

|                         |                           (RR/TIL 3)

(GRR 1)

RR/TIL indicates to GRR that a RR connection establishment was received on CCCH channel. GRR has to infor the GMM.

(GMM 1)

GRR informs the GMM about the receiption of RR connection establishment.

(GRR 2)

GRR receives the answer to the former indication

(RR/TIL 1)

Establishment request was accepted, no suspension needed: (RR does not send GPRS Suspension Request message)

(RR/TIL 2)

Establishment request was accepted, suspension needed: (RR has to send GPRS Suspension Request message)

(RR/TIL 3)

Establishment request was rejected

### Paging procedure for downlink packet transfer

GMM                       GRR                       RR/TIL

|                         |                            |

|                         |                            |

|                         |    RRGRR_PACKET_PAGING_IND |

|                         *<===========================|

|                        (GRR 1)                       |

| GMMRR_PAGE_IND          |                            |

*<=========================|                            |

(GMM 1)                                                                      |

|                                                                            |

|                         |                            |

(GRR 1)

RR/TIL indicates to the GRR that the packet paging request was received. This information is in paging request type 1,2 or 3. This primitive indicates whether the paging type was IMSI or P-TMSI (GPRS TMSI). (GMM 1)

(GMM 1)

GRR informs the GMM about the receiption of paging request for downlink transfer.

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |