Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM Protocol Stack

Message Sequence Charts

TI++

(Edit via File/Properties/Summary!)

| Document Number: | 6301.200.98.105 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.7 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1997-Aug-20 |
| Last changed: | 2003-Jun-12 by XINTEGRA |
| File Name: | Til.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1997-Aug-20 | Stefan Lemke et al. |  | 0.1 |  | 1 |
| 1997-Sep-24 | Stefan Lemke et al. |  | 0.2 |  | 2 |
| 1997-Oct-13 | Stefan Lemke et al. |  | 0.3 |  | 3 |
| 1998-Jul-20 | Stefan Lemke et al. |  | 0.4 |  | 4 |
| 1998-Nov-20 | Stefan Lemke et al. |  | 0.5 |  | 5 |
| 1999-Jan-20 | Stefan Lemke et al. |  | 0.6 |  | 6 |
| 2003-Jun-10 | XINTEGRA |  | 0.7 | Draft |  |

Notes:

- Initial version

- Changes after Implementation / Test

- Changes after first Integration

- Triple Vocoder / DCS 1800 / PCS 1900 / Dualband

- Short Message Cell Broadcast

- Changes after Optimisations

Table of Contents

1.1	References	6

1.2	Abbreviations	10

1.3	Terms	11

3.1	Frequency Bands	13

3.2	Layer 1 Interface	16

3.3	Mobile Station Configuration	17

3.3.1	Default Settings	17

3.4	Integration of Layer 1 and TI++	19

3.4.1	Layer 1 / TI++ not combined	19

3.4.2	Layer 1 / TI++ combined	20

3.4.3	Layer 1 / TI++ combined with Access from Layer 1 Synchronous	21

4.1	Power Measurements with PCH Interruption	22

4.2	Power Measurements without PCH Interruption	23

4.3	BCCH Detection	24

4.4	BCCH carrier synchronization failed	25

4.5	Reading of BCCH messages failed	25

4.6	RR rejects BCCH carrier	26

4.7	RR selects BCCH	27

5.1	Initiation of Idle Mode	29

5.2	Page Mode Change	30

5.3	Measurement Reporting	32

5.4	BCCH Reading	34

5.5	Downlink Failure Detection	36

5.6	Paging Detection	37

5.7	Stop Idle Mode	38

6.1	Definition of BCCH Allocation	39

6.2	Measurement Reporting	40

6.3	Synchronisation to Neighbour Cells	45

6.4	Confirmation of Synchronisation to Neighbour Cells	46

6.5	Reading of Neighbour Cell BCCH	47

7.1	Reading of Neighbour Cell BCCH	50

7.2	End of Cell Reselection, cell is suitable	51

7.3	BCCH of Neighbour Cell is not suitable, next cell	51

7.4	BCCH of Neighbour Cell is not readable, next cell	52

7.5	BCCH Error, no further cell available	53

8.1	Start of Connection Establishment	54

8.2	Immediate Assignment Message for MS	55

8.3	T3126 Expiry in RR	56

8.4	Immediate Assignment Reject Message for MS	57

10.1	Initiation	60

10.2	Successfull Resumption	62

10.3	Resumption fails	64

10.4	Successfull Reconnection	65

10.5	Reconnection fails	66

11.1	Asynchronous Handover	67

11.2	Synchronous Handover	70

11.3	Pre-Synchronous Handover	73

11.4	Pseudo-Synchronous Handover	76

11.5	Timing Advance out of Range	79

11.6	Successfull Resumption	80

11.7	Resumption fails	81

11.8	Successfull Reconnection	82

11.9	Reconnection fails	83

12.1	SACCH Uplink Interface	84

12.2	SACCH Downlink Interface	86

12.3	FACCH/SDCCH Uplink Interface	87

12.4	FACCH/SDCCH Downlink Interface	88

16.1	Measurement Reporting	92

16.2	Update of Neighbour Cell List	95

16.3	Synchronization to Neighbour Cells	96

16.4	Confirmation of Synchronisation to Neighbour Cells	97

18.1	Configuration of CBCH	100

18.2	Start Reading of CBCH	101

18.3	Stop Reading of CBCH	102

18.4	Collection of CBCH Blocks	103

18.4.1	CBCH message, expected	103

18.4.2	CBCH message, not expected	105

18.4.3	Schedule message	106

18.4.4	Null message	112

18.5	Geographical Scope	113

19.1	TCH Loopback	115

19.2	DAI Testing	116

20.1	Volume Initialization	117

20.2	Volume Change	117

20.3	Speech On	118

20.4	Speech Off	118

20.5	Keypad Feedback	118

20.6	Tones	119

20.6.1	Tones without Audio Channel	120

20.6.2	Tones with Audio Channel	121

A.	Acronyms	123

B.	Glossary	123

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18a]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat GmbH

[39]	Service Access Point MNCC
	6147.101.96.100; Condat GmbH

[40]	Service Access Point MNSS
	6147.102.96.100; Condat GmbH

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat GmbH

[42]	Service Access Point MMCC
	6147.104.97.100; Condat GmbH

[43]	Service Access Point MMSS
	6147.105.97.100; Condat GmbH

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat GmbH

[45]	Service Access Point RR
	6147.107.97.100; Condat GmbH

[46]	Service Access Point SIM
	6147.108.97.100; Condat GmbH

[47]	Service Access Point MPH
	6147.109.96.100; Condat GmbH

[48]	Service Access Point DL
	6147.110.96.100; Condat GmbH

[49]	Service Access Point MDL
	6147.111.96.100; Condat GmbH

[50]	Service Access Point PH
	6147.112.97.100; Condat GmbH

[51]	Service Access Point MMI
	6147.113.96.100; Condat GmbH

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat GmbH

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat GmbH

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat GmbH

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat GmbH

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat GmbH

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat GmbH

[58]	Users Guide
	6147.300.96.100; Condat GmbH

[59]	Test Specification CC
	6147.400.97.100; Condat GmbH

[60]	Test Specification SS
	6147.401.97.100; Condat GmbH

[61]	Test Specification SMS
	6147.402.97.100; Condat GmbH

[62]	Test Specification MM
	6147.403.97.100; Condat GmbH

[63]	Test Specification RR
	6147.404.97.100; Condat GmbH

[64]	Test Specification DL
	6147.405.97.100; Condat GmbH

[65]	Test Specification CCD
	6147.406.97.100; Condat GmbH

[66]	SDL Specification CC
	6147.500.97.100; Condat GmbH

[67]	SDL Specification SS
	6147.501.97.100; Condat GmbH

[68]	SDL Specification SMS
	6147.502.97.100; Condat GmbH

[69]	SDL Specification MM
	6147.503.97.100; Condat GmbH

[70]	SDL Specification RR
	6147.504.97.100; Condat GmbH

[71]	SDL Specification DL
	6147.505.97.100; Condat GmbH

[72]	Message Specification CC
	6147.600.97.100; Condat GmbH

[73]	Message Specification SS
	6147.601.97.100; Condat GmbH

[74]	Message Specification SMS
	6147.602.97.100; Condat GmbH

[75]	Message Specification MM
	6147.603.97.100; Condat GmbH

[76]	Message Specification RR
	6147.604.97.100; Condat GmbH

[77]	Message Specification DL
	6147.605.97.100; Condat GmbH

[78]	Technical Documentation CC
	6147.700.97.100; Condat GmbH

[79]	Technical Documentation SS
	6147.701.97.100; Condat GmbH

[80]	Technical Documentation SMS
	6147.702.97.100; Condat GmbH

[81]	Technical Documentation MM
	6147.703.97.100; Condat GmbH

[82]	Technical Documentation RR
	6147.704.97.100; Condat GmbH

[83]	Technical Documentation DL
	6147.705.97.100; Condat GmbH

[84]	Technical Documentation CCD
	6147.706.97.100; Condat GmbH

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

This document describes the services needed for integration of TI layer 1 and Condat protocol stack.

# General Aspects

## Frequency Bands

The protocol stack supports different frequency bands:

- GSM 900

- DCS 1800

- PCS 1900

- Dualband GSM 900 / DCS 1800

- Dualband GSM 900 / E-GSM / DCS 1800

The different frequency ranges need some different algorithms and different storing of channel lists. Therefore a global variable std is introduced which is initialized in the PCM driver module. The value of this variable defines the supported frequency range. The following dependency exists:

| Frequency Range | Std |
| --- | --- |
| GSM 900 | 1 |
| PCS 1900 | 3 |
| DCS 1800 | 4 |
| Dualband GSM 900 / DCS 1800 | 5 |
| Dualband GSM 900 / E-GSM / DCS 1800 | 6 |

Frequency lists in TI++ have the following format:

GSM 900

channels 1 to 124 = 124 channels

| Offset | Channel |
| --- | --- |
| 0 | not used |
| 1 | 1 |
| ... | ... |
| 124 | 124 |

DCS 1800

channels 512 to 885 = 374 channels

| Offset | Channel |
| --- | --- |
| 0 | not used |
| 1 | 512 |
| ... | ... |
| 374 | 885 |

PCS 1900

channels 512 to 810 = 299 channels

| Offset | Channel |
| --- | --- |
| 0 | not used |
| 1 | 512 |
| ... | ... |
| 299 | 810 |

Dualband GSM 900 / DCS 1800

channels 1 to 124     = 124 channels

channels 512 to 885 = 374 channels

total                              498 channels

| Offset | Channel |
| --- | --- |
| 0 | not used |
| 1 | 1 |
| ... | ... |
| 124 | 124 |
| 125 | 512 |
| ... | ... |
| 498 | 885 |

Dualband GSM 900 / E-GSM / DCS 1800

channels 1 to 124              = 	124 channels

channels 975 to 1023 + 0  =	50 channels

channels 512 to 885          =	374 channels

total                                     	548 channels

| Offset | Channel |
| --- | --- |
| 0 | not used |
| 1 | 1 |
| ... | ... |
| 124 | 124 |
| 125 | 975 |
| ... | ... |
| 173 | 1023 |
| 174 | 0 |
| 175 | 512 |
| ... | ... |
| 548 | 885 |

## Layer 1 Interface

The structure of some primitives of layer 1 depend on the used frequency range. This is not supported by Condat´s generating tools. So different structures need different primitives. The following table shows the equivalent primitives. In this message sequence charts only the GSM 900 variant is described.

Full Power Measurement Interface

| Frequency Range | Primitive |
| --- | --- |
| GSM 900 | MPH5_RXLEV_CON |
| DCS 1800 | MPH5_RXLEV_CON_1800 |
| PCS 1900 | MPH5_RXLEV_CON_1900 |
| Dualband GSM 900 / DCS 1800 | MPH5_RXLEV_CON_900_1800 |
| Dualband GSM 900 / E-GSM / DCS 1800 | MPH5_RXLEV_CON_9_E_18 |

## Mobile Station Configuration

The protocol stack components read several records of the permanent memory configuration to get the actual configuration. The following records are read:

- IMEI	the international mobile equipment identity of the mobile station

- MSCAP	the mobile station capabilities

- CLASS2	the mobile station classmark 2 element

- CLASS3	the mobile station classmark 3 element.

Additionally records will be introduced with the MMI. The exact content is described in the PCM Driver Specification.

### Default Settings

The following default settings are used in the current implementation:

| Parameter | Frequency Range | PC-Version | Target-Version |
| --- | --- | --- | --- |
| MSCAP | GSM 900 | fullrate, no data capabilities | fullrate, halfrate, enhanced fullrate, NT Async, T Async, T fax |
|  | DCS 1800 | fullrate, halfrate, NT-Async | fullrate, halfrate, enhanced fullrate, NT Async, T Async, T fax |
|  | PCS 1900 | fullrate, halfrate, enhanced fullrate, NT Async, T Async, T fax | fullrate, halfrate, enhanced fullrate, NT Async, T Async, T fax |
|  | Dualband | fullrate, enhanced fullrate T Async, T fax | fullrate, halfrate, enhanced fullrate, NT Async, T Async, T fax |
|  | Dualband Extended | fullrate, enhanced fullrate T Async, T fax, 14.4 | fullrate, halfrate, enhanced fullrate, NT Async, T Async, T fax |

| Parameter | Frequency Range | PC-Version | Target-Version |
| --- | --- | --- | --- |
| CLASS2 | GSM 900 | class 2, A5/1 available early classmark sending phase 2 MS SMS, Pseudo-Sync Handover, no E-GSM No add info in class 3, no other cipher algorithm | class 4, A5/1 available early classmark sending phase 2 MS SMS, no Pseudo-Sync Handover, no E-GSM No add info in class 3, no other cipher algorithm |
|  | DCS 1800 | class 1, A5/1 available early classmark sending phase 2 MS SMS, no Pseudo-Sync Handover, no E-GSM noadd info in class 3, no other cipher algorithm | class 1, A5/1 available early classmark sending phase 2 MS SMS, no Pseudo-Sync Handover, no E-GSM No add info in class 3, no other cipher algorithm |
|  | PCS 1900 | class 1, A5/1 available early classmark sending phase 2 MS SMS, no Pseudo-Sync Handover, no E-GSM No add info in class 3, no other cipher algorithm | class 1, A5/1 available early classmark sending phase 2 MS SMS, no Pseudo-Sync Handover, no E-GSM No add info in class 3, no other cipher algorithm |
|  | Dualband | class 1, A5/1 available early classmark sending phase 2 MS SMS, no Pseudo-Sync Handover, no E-GSM add info in class 3, no other cipher algorithm | Class 1, A5/1 available early classmark sending phase 2 MS SMS, no Pseudo-Sync Handover, no E-GSM Add info in class 3, no other cipher algorithm |
|  | Dualband Extended | class 1, A5/1 available early classmark sending phase 2 MS SMS, no Pseudo-Sync Handover, E-GSM add info in class 3, no other cipher algorithm | Class 1, A5/1 available early classmark sending phase 2 MS SMS, no Pseudo-Sync Handover, E-GSM add info in class 3, no other cipher algorithm |
| CLASS3 | GSM 900 | no information | no information |
|  | DCS 1800 | no information | no information |
|  | PCS 1900 | no information | no information |
|  | Dualband | GSM 900 and DCS 1800 supported Class 2 DCS, Class 3 GSM | GSM 900 and DCS 1800 supported Class 1 DCS, Class 4 GSM |
|  | Dualband Extended | GSM 900, E-GSM, DCS 1800 supp. Class 2 DCS, Class 3 GSM | GSM 900, E-GSM, DCS 1800 supp. Class 1 DCS, Class 4 GSM |

## Integration of Layer 1 and TI++

Three scenarios are defined for the Layer 1 / TI++ integration.

### Layer 1 / TI++ not combined

G23, TI++ and Layer 1 are separate threads. They communicate via primitives and queues. This is the actual solution. The following picture shows the communication pathes:

G23

(RR/DL-Queue)

G23

vsi_c_send to PL

vsi_c_send to RR/DL

TI++

(PL-Queue)

TI++

vsi_c_send to PL

via send_sig

vsi_c_send to L1

L1

(L1-Queue)

Layer 1 Asynchronous

vsi_c_send to L1

via send_it_sig

Shared Memory

Layer 1 Synchronous

### Layer 1 / TI++ combined

TI++ and Layer 1 are combined to one thread. They communicate via primitives and queues. This combination saves power and RAM.

G23

(RR/DL-Queue)

G23

vsi_c_send to PL
via receive_sig

vsi_c_send to RR/DL

L1

(PL-Queue)

TI++

Direct access to pl_pei_primitive via send_sig

Function Access to Main Loop

Layer 1 Asynchronous

vsi_c_send to PL

via send_it_sig

Shared Memory

Layer 1 Synchronous

For this scenario the compile switch OPT_COMB must be set (TI++ Sources and DL1_MEM.C). The module DL1_PEI.C is not needed, the functionality is replaced by TIL_PEI.C.

The header pf_ent.h must be modified. TI++ (named PL) must be used as ACTIVE BODY, the stack size must be increased and the layer 1 entity (named L1) must be removed.

/*

* defines wich entities are a part of the Protocol stack

* and wich of them are active components.

* Last entry must be NULL, NULL, 0L, 0, 0, PASSIVE_BODY

*/

/*  name,  pei_createfunc,  stacksize, queueentries, priority, behaviour */

/*---------------------------------------------------------------------*/

"MMI", mmi_pei_create,  1436L,     10,           10,       PASSIVE_BODY,

"CC",  cc_pei_create,    924L,     10,           9,        PASSIVE_BODY,

"SIM", sim_pei_create,  1180L,     10,           8,        PASSIVE_BODY,

"SS",  ss_pei_create,    924L,     10,           11,       PASSIVE_BODY,

"SMS", sms_pei_create,  1624L,     10,           12,       PASSIVE_BODY,

"MM",  mm_pei_create,   1180L,     10,           6,        PASSIVE_BODY,

"RR",  rr_pei_create,   1436L,     10,           5,        PASSIVE_BODY,

"DL",  dl_pei_create,    924L,     10,           4,        PASSIVE_BODY,

"PL",  pl_pei_create,   2436L,     10,           3,        ACTIVE_BODY,

NULL,  NULL,               0L,      0,           0,        PASSIVE_BODY

### Layer 1 / TI++ combined with Access from Layer 1 Synchronous

TI++ and Layer 1 are combined to one thread. Some services of TI++ are requested directly by layer 1 synchronous. This combination saves power and RAM.

G23

(RR/DL-Queue)

G23

vsi_c_send to RR/DL

vsi_c_send to PL
via receive_sig

L1

(PL-Queue)

TI++

Function Access to Main Loop or vsi_c_send to PL

Direct access to some functions

Direct access to pl_pei_primitive via send_sig

Layer 1 Asynchronous

Shared Memory

vsi_c_send to PL

via send_it_sig

Layer 1 Synchronous

For this scenario the compile switch OPT_COMB and OPT_SYNC must be set (TI++ Sources and DL1_MEM.C). The module DL1_PEI.C is not needed, the functionality is replaced by TIL_PEI.C.

The header pf_ent.h must be modified. TI++ (named PL) must be used as ACTIVE BODY, the stack size must be increased and the layer 1 entity (named L1) must be removed.

/*

* defines wich entities are a part of the Protocol stack

* and wich of them are active components.

* Last entry must be NULL, NULL, 0L, 0, 0, PASSIVE_BODY

*/

/*  name,  pei_createfunc,  stacksize, queueentries, priority, behaviour */

/*---------------------------------------------------------------------*/

"MMI", mmi_pei_create,  1436L,     10,           10,       PASSIVE_BODY,

"CC",  cc_pei_create,    924L,     10,           9,        PASSIVE_BODY,

"SIM", sim_pei_create,  1180L,     10,           8,        PASSIVE_BODY,

"SS",  ss_pei_create,    924L,     10,           11,       PASSIVE_BODY,

"SMS", sms_pei_create,  1624L,     10,           12,       PASSIVE_BODY,

"MM",  mm_pei_create,   1180L,     10,           6,        PASSIVE_BODY,

"RR",  rr_pei_create,   1436L,     10,           5,        PASSIVE_BODY,

"DL",  dl_pei_create,    924L,     10,           4,        PASSIVE_BODY,

"PL",  pl_pei_create,   2436L,     10,           3,        ACTIVE_BODY,

NULL,  NULL,               0L,      0,           0,        PASSIVE_BODY

# Cell Selection

The cell selection is splitted into two parts. The power measurements and the BCCH detection phase.

## Power Measurements with PCH Interruption

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|    MPH_POWER_REQ            |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |                             |

|                             *<repeat 5 times>             |

|                             |                             |

|                             |       MPH5_RXLEV_REQ        |

|                             *============================>*

|                             |                          (PL 1)

|                             |       MPH5_RXLEV_CON        |

|                             *<============================*

|                          (TI 2)                           |

|    MPH_POWER_CNF            |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(TI 1)

The power measurements are initiated by RR. It is signalled that PCH listening shall be interrupted. All other activities of other parts like idle control etc. are stopped. The repeat counter for fieldstrength measurements is cleared. The power measurements are started. TI++ allocates a partition for the results of power measurements dynamically.

(PL 1)

Power measurement is requested to the physical layer.

(TI 2)

The result is received from PL. All fieldstrength values are added to the fieldstrength list. Note that the fieldstrength with index 0 is not used. The repeat counter is incremented. If the maximum is not reached the next measurement is triggered by a MPH5_RXLEV_REQ primitive.

(RR 1)

A list in decreasing fieldstrength order is created by TI++. A maximum of channels for each frequency band is selected. The dynamic allocated buffer is released. The resulting list is forwarded to RR to trigger the BCCH detection phase. A threshold value is introduced to check only channels with a minimum fieldstrength.

## Power Measurements without PCH Interruption

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|    MPH_POWER_REQ            |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |                             |

|                             *<repeat 5 times>             |

|                             |                             |

|                             |       MPH5_FULL_REQ         |

|                             *============================>*

|                             |                          (PL 1)

|                             |       MPH5_RXLEV_CON        |

|                             *<============================*

|                          (TI 2)                           |

|    MPH_POWER_CNF            |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(TI 1)

The power measurements are initiated by RR. It is signalled that PCH listening shall not be interrupted. The repeat counter for fieldstrength measurements is cleared. The power measurements are started. TI++ allocates a partition for the results of power measurements dynamically.

(PL 1)

Power measurement is requested to the physical layer.

(TI 2)

The result is received from PL. All fieldstrength values are added to the fieldstrength list. Note that the fieldstrength with index 0 is not used. The repeat counter is incremented. If the maximum is not reached the next measurement is triggered by a MPH5_RXLEV_REQ primitive.

(RR 1)

A list in decreasing fieldstrength order is created by TI++. A maximum of channels for each frequency band is selected. The dynamic allocated buffer is released. The resulting list is forwarded to RR to trigger the BCCH detection phase. A threshold value is used to check only channels with a minimum fieldstrength.

## BCCH Detection

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|        MPH_BSIC_REQ         |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |   MPH5_START_BCCH_READING   |

|                             *============================>*

|                             |                          (PL 1)

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 2)                           |

|        MPH_BSIC_CNF         |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |        MPH5_DATA_IND        |

|                             *<============================*

|                          (TI 3)                           |

|        MPH_UNITDATA_IND     |                             |

*<============================*                             |

(DL 1)                          |                             |

|                             |                             |

(TI 1)

RR requests BCCH detection for a channel. Typically this will be the channel with the highest fieldstrength from the previous power measurements.

(PL 1)

Synchronisation to frequency correction burst and synchron burst followed by reading the BCCH is triggered with the MPH5_START_BCCH_READING primitive. The parameter of the primitive is the channel number.

(TI 2)

PL has read the frequency correction burst and the synchron burst (indicated by the sb_flag in the MPH5_NCELL_SB_IND primitive). The bsic is stored for this channel.

(RR 1)

RR is informed about the BCCH carrier. Therefore a MPH_BSIC_CNF with the channel number and the bsic is sent to RR. The cause is set to CS_NO_ERROR. The cell selection process starts reading the BCCH.

(TI 3)

Layer 1 forwards subsequent received data blocks on BCCH. The content is valid.

(DL 1)

The content of the BCCH data block (stored in parameter l2_frame of the MPH5_DATA_IND) is forwarded directly to RR.

## BCCH carrier synchronization failed

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|        MPH_BSIC_REQ         |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |   MPH5_START_BCCH_READING   |

|                             *============================>*

|                             |                          (PL 1)

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 2)                           |

|        MPH_BSIC_CNF         |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(TI 1)

RR requests BCCH detection for a channel. Typically this will be the channel with the highest fieldstrength from the previous power measurements.

(PL 1)

Synchronisation to frequency correction burst and synchron burst followed by reading the BCCH is triggered with the MPH5_START_BCCH_READING primitive. The parameter of the primitive is the channel number.

(TI 2)

The sb_flag in the MPH5_NCELL_SB_IND primitive indicates that synchronisation to the frequency correction burst or the synchron burst has failed.

(RR 1)

RR is informed about the result. Therefore a MPH_BSIC_CNF with the channel number and the cause equal to CS_NO_BCCH_AVAIL is send to RR.

## Reading of BCCH messages failed

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             |        MPH5_DATA_IND        |

|                             *<============================*

|                          (TI 1)                           |

|        MPH_ERROR_IND        |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

a) other channels available

|                             |                             |

(TI 1)

The error_cause in the MPH5_DATA_IND primitive indicates that reading of the BCCH data has failed.

(RR 1)

RR is informed with the MPH_ERROR_IND primitive and the cause BCCH read error. The numbers of read errors may used by RR to decide rejection of the BCCH carrier.

## RR rejects BCCH carrier

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|        MPH_BSIC_REQ         |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |   MPH5_STOP_BCCH_READING    |

|                             *============================>*

|                             |                          (PL 1)

|                             |   MPH5_START_BCCH_READING   |

|                             *============================>*

|                             |                          (PL 2)

|                             |                             |

|                             |                             |

(TI 1)

RR rejects a BCCH carrier (cell barred, C1 <= 0, wrong plmn or too many read errors etc.) and requests BCCH detection for the next channel number. A channel number equal to 0xFFFF indicates that TI++ shall stop the BCCH reading of the previous channel and shall not start a new BCCH detection.

(PL 1)

Reading of the last BCCH is stopped.

(PL 2)

The BCCH for the next channel is searched.

## RR selects BCCH

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|      MPH_CLASSMARK_REQ      |                             |

*============================>*                             |

|                          (TI 1)                           |

|        MPH_IDLE_REQ         |                             |

*============================>*                             |

|                          (TI 2)                           |

|                             |   MPH5_STOP_BCCH_READING    |

|                             *============================>*

|                             |                          (PL 1)

|                             |    MPH5_START_NORMAL_PG     |

|                             *============================>*

|                             |                          (PL 2)

|        MPH_IDENTITY_REQ     |                             |

*============================>*                             |

|                          (TI 3)                           |

|        MPH_CBCH_REQ         |                             |

*============================>*                             |

|                          (TI 4)                           |

|                             |      MPH5_CONFIG_CBCH       |

|                             *============================>*

|                             |                          (PL 3)

|    MPH_NEIGHBOURCELL_REQ    |                             |

*============================>*                             |

|                          (TI 5)                           |

|                             |         MPH5_BA_LIST        |

|                             *============================>*

|                             |                          (PL 4)

|                             |                             |

(TI 1)

If RR configures the idle mode typical four primitives are send to the lower layer. The first one is MPH_CLASSMARK_REQ containing the power class (or power classes for dualband) of the mobile station. This information is stored for the RACH control process and is not used by the cell selection process.

(TI 2)

Configuration of the idle mode is carried out after receiving the MPH_IDLE_REQ from RR. The primitive contains the channel configuration and additional parameters to configure paging.

(PL 1)

BCCH reading is stopped.

(PL 2)

The layer 1 is configured for the idle mode. After configuration the cell selection process enters the idle mode and the idle control process is initiated. The layer 1 expects MPH5_START_NORMAL_PG primitive for normal paging. The primitive has the following parameters:

MPH5_START_NORMAL_PG	set from

bcch_combined	MPH_IDLE_REQ -> comb_ccch
bs_ag_blks_res	MPH_IDLE_REQ -> bs_ag_blks_res
bs_pa_mfrms	MPH_IDLE_REQ -> bs_pa_mfrms + 2
ccch_group	MPH_IDLE_REQ -> tn / 2
page_block_index	MPH_IDLE_REQ -> pg mod PAG_BLOCKS_PER_MFR
page_group	MPH_IDLE_REQ -> pg div PAG_BLOCKS_PER_MFR

with PAG_BLOCKS_PER_MFR equal to the number of paging blocks during on multiframe according to the following rules (see also GSM 5.02, section 7 table 5).

Combined BCCH	PAG_BLOCKS_PER_MFR = MAX (1, 3-bs_ag_blks_res)

Not combined BCCH	PAG_BLOCKS_PER_MFR = 9-bs_ag_blks_res

(TI 3)

RR sends the identities of the mobile (IMSI and TMSI) for the paging check in TI++. The mobile identities are forwarded to the paging sub-process of TI++.

(TI 4)

RR configures the CBCH channel.

(PL 3)

The configuration is forwarded to layer 1.

(TI 5)

The neighbourcell description received with the system information messages on BCCH is forwarded to the lower layers. This channel list is the base for the measurements in idle mode. This information is stored and used by the idle control process and not used by the cell selection process.

(PL 4)

The neighbourcell list is forwarded to layer 1.

# Idle Mode Serving Cell Procedures

After cell selection the mobile station is in idle mode. For the serving cell the following services supported by TI++:

- Initiation of Idle Mode

- Page Mode Change

- Measurement Reporting

- BCCH Reading

- Downlink Failure Detection

- Paging Detection

- Stop Idle Mode

## Initiation of Idle Mode

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|        MPH_IDLE_REQ         |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |   MPH5_STOP_BCCH_READING    |

|                             *============================>*

|                             |                          (PL 1)

|                             |    MPH5_START_NORMAL_PG     |

|                             *============================>*

|                             |                          (PL 2)

|                             |                             |

(TI 1)

The initiation of idle mode process is carried out by the MPH_IDLE_REQ primitive from RR at the end of the cell selection process. This is described in detail in chapter 4.7 RR selects BCCH.

Additional the downlink timeout value of the MPH_IDLE_REQ is send to the PCH control process to initialize the downlink failure detection.

## Page Mode Change

RR/DL                        TI-Add On                     TI-Layer 1

|                               |                             |

|                               |       MPH5_DATA_IND         |

|                               |    (PCH, EPCH, CCCH, BCCH)  |

|                               *<============================*

|                            (TI 1)                           |

|MPH_UNITDATA_IND/MPH_PAGING_IND|                             |

*<==============================*                             |

(RR 1)                            |                             |

|                               |                             |

a) swap to normal paging

|                               |                             |

|                               |     MPH5_START_NORMAL_PG    |

|                               *============================>*

|                               |                          (PL 1)

|                               |                             |

b) swap to extended paging

|                               |                             |

|                               |       MPH5_EXT_PG_READ      |

|                               *============================>*

|                               |                          (PL 2)

|                               |                             |

a) swap to paging reorganisation

|                               |                             |

|                               |     MPH5_START_REORG_PG     |

|                               *============================>*

|                               |                          (PL 3)

|                               |                             |

(TI 1)

Layer 1 listens to the unacknowledged channels BCCH and CCCH which is divided into AGCH and PCH. If the received data block is a PCH or extended PCH the result is forwarded to the PCH control process as input for downlink failure detection.

The page mode in the message is checked for CCCH messages. The following table shows the page mode change in detail:

| old page mode | new page mode | Action |
| --- | --- | --- |
| Paging reorganisation | normal paging | swap to normal paging |
|  | extended paging | None |
|  | paging reorganisation | None |
|  | same as before | None |
| normal paging | normal paging | None |
|  | extended paging | swap to extended paging |
|  | paging reorganisation | swap to paging reorganisation |
|  | same as before | None |
| extended paging | normal paging | none |
|  | extended paging | swap to extended paging |
|  | paging reorganisation | swap to paging reorganisation |
|  | same as before | swap to extended paging |

(RR 1)

Valid data blocks may receive on the BCCH, AGCH or CCCH.

BCCH messages contain no page mode information. TI++ removes the layer 2 pseudolength and forwards the message directly to RR, if the message content for a message has changed.

AGCH messages contain page mode information. TI++ removes the layer 2 pseudolength and forwards the message directly to RR, if the mobile is in connection establishment phase.

PCH messages contain page mode information. TI++ checks the content against the mobile identities and sends a paging information to RR, if the identities matches.

(PL 1)

Layer 1 swaps to normal paging after receiving a MPH5_START_NORMAL_PG primitive.

(PL 2)

Swapping to extended paging is carried out by sending a MPH5_EXT_PG_READ primitive. (Remark: For layer 1 with build number greater or equal 913 the primitive MPH5_START_NORMAL_PG with the parameter page_extended = 1 is used.)

(PL 3)

Paging reorganisation is initiated by sending a MPH5_START_REORG_PG primitive to layer 1.

## Measurement Reporting

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             |   MPH5_RXLEV_PERIODIC_IND   |

|                             *<============================*

|                          (TI 1)                           |

|                             *...                          |

|                          (TI 2)                           |

|     MPH_MEASUREMENT_IND     |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(TI 1)

During listening to its own paging subchannel, layer 1 performs measurements on the serving cell. The results are forwarded to the upper layer. The values are „GSM encoded“ but with a four times better resolution.

(TI 2)

The values are used to build an running average over at least five samples or a period of five seconds, whichever is the greater period.

(RR 1)

RR has the requirement to receive at least all five seconds a new measurement report from the lower layer after the initial period of five paging blocks or five seconds.

The periodic incoming MPH5_RXLEV_PERIODIC_IND primitives are used to determine the time for sending a measurement report to RR. The parameter fn_offset in the MPH_MEASUREMENT_IND is used to define the time between two measurement reports in TDMA frames. RR expects the fieldstrength values in the GSM range.

The first measurement report is send after receiving fieldstrength values from layer 1 for at least five seconds. The following measurement reports to RR are send for a time period just under five seconds. The following table shows the constants:

| bs_pa_mfrms | initial number of reports | number of reports from layer 1 to send a measurement report | TDMA frames since last measurement report to RR |
| --- | --- | --- | --- |
| 2 | 11 | 10 | 102 |
| 3 | 8 | 7 | 153 |
| 4 | 6 | 5 | 204 |
| 5 | 5 | 4 | 255 |
| 6 | 5 | 3 | 306 |
| 7 | 5 | 3 | 357 |
| 8 | 5 | 2 | 408 |
| 9 | 5 | 2 | 459 |

The measurement report contains only neighbourcells for which TI++ has read the frequency correction burst and the synchron burst (up to six, ordered after fieldstrength).

For the dualband version the multi-band parameter is used. It defines the order of neighbourcells in the measurement report. The following table shows the dependencies. For all single band variants the neighbourcells are ordered after fieldstrength.

| Multiband Parameter | Ordering of Neighbourcells |
| --- | --- |
| 0 | ordered by fieldstrength |
| 1 | one cell excluding the serving cell band rest reported from the serving cell band remaining ordered by fieldstrength |
| 2 | two cells excluding the serving cell band rest reported from the serving cell band remaining ordered by fieldstrength |
| 3 | three cells excluding the serving cell band rest reported from the serving cell band remaining ordered by fieldstrength |

## BCCH Reading

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             |   MPH5_RXLEV_PERIODIC_IND   |

|                             *<============================*

|                          (TI 1)                           |

|                             *...                          |

|                          (TI 2)                           |

|                             |     MPH5_SCELL_BCCH_READ    |

|                             *============================>*

|                             |                          (PL 1)

|                             |         MPH5_DATA_IND       |

|                             *<============================*

|                          (TI 3)                           |

|                             |                             |

a) no content changed

|                             |                             |

|                             *                             |

|                          (TI 4)                           |

|                             |                             |

b) content changed

|                             |                             |

|       MPH_UNITDATA_IND      |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

a) not all sys infos received

|                             |                             |

|                             |     MPH5_SCELL_BCCH_READ    |

|                             *============================>*

|                             |                          (PL 2)

|                             |                             |

(TI 1)

During listening to its own paging subchannel, layer 1 performs measurements on the serving cell. The results are forwarded to the upper layer. The number of received measurement results from layer 1 is the base for the decision to read the BCCH of the serving cell.

(TI 2)

The number of reports is counted. All thirty seconds the BCCH of the serving cell shall be read.

(PL 1)

The following table shows the number of incoming reports until BCCH reading of the serving cell is initiated.

| bs_pa_mfrms | number of reports from layer 1 between to successive BCCH readings |
| --- | --- |
| 2 | 63 |
| 3 | 42 |
| 4 | 31 |
| 5 | 25 |
| 6 | 21 |
| 7 | 18 |
| 8 | 15 |
| 9 | 14 |

The parameter si_bit_map of the MPH5_SCELL_BCCH_READ primitive indicates for which BCCH message the BCCH shall be read. It depends on the parameter bcch_ext of the previous MPH_IDLE_REQ.

The si_bit_map has the following format:

| BCCH type | si5bis | si2ter | si2bis | si8 | si7 | si6 | si5 | si4 | si3 | si2 | si1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| normal BCCH | - | - | x | - | - | - | - | x | x | x | x |
| extended BCCH | - | - | x | x | X | - | - | x | x | x | x |

(TI 3)

A BCCH data block is read by layer 1. The parameter tc in the received MPH5_SCELL_BCCH_IND primitive indicates the multiframe number modulo 8 in which the BCCH data block was read. The tc parameter corresponds to the received system information message. The corresponding bit in the si_bit_map is cleared according to the following table:

| tc | system info | Comment |
| --- | --- | --- |
| 0 | si1 | - |
| 1 | si2 | - |
| 2 | si3 | - |
| 3 | si4 or si8 | for extended bcch the message type is checked directly in the l2_frame |
| 4 | si2bis | not GSM 900 |
| 5 | si2ter | only dualband |
| 6 | si3 | - |
| 7 | si4 or si7 | for extended bcch the message type is checked directly in the l2_frame |

(TI 4)

If the message content has not changed, the message is not forwarded.

(RR 1)

If the BCCH data block is valid the message, TI++ removes the layer 2 pseudolength and forwards the message to RR.

(PL 2)

If the si_bit_map is not empty a new BCCH read request is started with the changed si_bit_map. Else the report counter is initialized.

## Downlink Failure Detection

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

a) Initialisation

|                             |                             |

|        MPH_IDLE_REQ         |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |                             |

b) receive valid PCH data block

|                             |                             |

|                             |         MPH5_DATA_IND       |

|                             |         (PCH or EPCH)       |

|                             *<============================*

|                          (TI 2)                           |

|                             |                             |

c) receive invalid PCH data block, downlink timeout not reached

|                             |                             |

|                             |         MPH5_DATA_IND       |

|                             |         (PCH or EPCH)       |

|                             *<============================*

|                          (TI 3)                           |

|                             |                             |

d) receive invalid PCH data block, downlink timeout reached

|                             |                             |

|                             |         MPH5_DATA_IND       |

|                             |         (PCH or EPCH)       |

|                             *<============================*

|                          (TI 4)                           |

|        MPH_ERROR_IND        |                             |

*<============================*                             |

(RR 2)                          |                             |

|                             |                             |

(TI 1)

During initialising the idle mode the downlink timeout counter is initialised by the MPH_IDLE_REQ primitive. This value is equal to the nearest integer of 90/bs_pa_mfrms and is calculated in RR.

(TI 2)

If a valid PCH data block receives the counter is incremented by one up to the initial value.

(TI 3)

If an invalid PCH data block receives the counter is decremented by four up to zero. The block is discharged.

(TI 4)

An invalid PCH data block received. The counter is decremented to zero. So the downlink failure criterion is met. The PCH data block is discharged.

(RR 2)

The downlink failure is signalled to RR. The downlink criterion counter is re-initialized.

## Paging Detection

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

a) Initialisation

|                             |                             |

|      MPH_IDENTITY_REQ       |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |                             |

b) receive valid PCH data block, not for MS

|                             |                             |

|                             |         MPH5_DATA_IND       |

|                             |         (PCH or EPCH)       |

|                             *<============================*

|                          (TI 2)                           |

|                             |                             |

c) receive valid PCH data block, for MS

|                             |                             |

|                             |         MPH5_DATA_IND       |

|                             |         (PCH or EPCH)       |

|                             *<============================*

|                          (TI 3)                           |

|        MPH_PAGING_IND       |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(TI 1)

Whenever RR has new information about IMSI / TMSI, it sends a MPH_IDENTITY_REQ to TI++. TI++ creates a pattern for the IMSI using the air-interface format and stores the TMSI.

(TI 2)

A valid PCH data block receives containing a paging request type 1, 2 or 3 message. The compare fails and the message is not longer used.

(TI 3)

A valid PCH data block receives containing a paging request type 1, 2 or 3 message. The compare matches. TI++ has calculated the mobile identity type which matches (IMSI or TMSI) and the needed channel depending on the position of the mobile identity in the paging request message.

(RR 1)

The paging indication is send to RR. RR starts mobile terminated connection establishment.

## Stop Idle Mode

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             *                             |

|                          (TI 1)                           |

|                             |       MPH5_STOP_IDLE        |

|                             *============================>*

|                             |                          (PL 1)

|                             |                             |

(TI 1)

Various events may stop the idle mode:

- Start of Dedicated Mode

- Start of Cell Selection

- ...

(PL 1)

The idle mode is stopped in layer 1 by sending a MPH5_STOP_IDLE primitive.

# Idle Mode Neighbour Cell Procedures

After cell selection the mobile station is in idle mode. For the neighbour cells the following services are requested by RR:

- Definition of BCCH allocation

- Fieldstrength measurements

- Synchronisation to neighbourcells

- Confirmation of synchronisation to neighbourcells

- BCCH reading for neighbourcells

## Definition of BCCH Allocation

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|    MPH_NEIGHBOURCELL_REQ    |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |         MPH5_BA_LIST        |

|                             *============================>*

|                             |                          (PL 1)

|                             |                             |

(TI 1)

A new neighbour cell description list is forwarded from RR to the lower layer.

(PL 1)

The serving cell is added to the neighbour cell list. The BCCH allocation list in the neighbour cell process is updated. That means all channels in the old BCCH allocation list, which are not member of the new list are removed. All channels in the new list, which are not member of the old list are added to the BCCH allocation list in the neighbour cell process. This entries are initialised with status FB_SB_SYNC for the serving cell and IDLE for all other cells.

The new BCCH allocation list is forwarded to PL. The parameter ba_id is used to identify incoming fieldstrength samples.

In the dualband version the multiband parameter is stored.

## Measurement Reporting

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             |   MPH5_RXLEV_PERIODIC_IND   |

|                             *<============================*

|                          (TI 1)                           |

|     MPH_MEASUREMENT_IND     |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(TI 1)

A single measurement report (MPH5_RXLEV_PERIODIC_IND) is returned to layer 3 after each PCH block monitor period. This report message contains a list of the 8 measurement results (arfcns measured with their corresponding power levels, in non clipped RXLEV format, i.e: 0 to 255). This list has a ba_id sequence number which matches the original MPH5_BA_LIST request. If the BA list contains less than 8 arfcns, measurements shall be repeated during the PCH block (e.g: if the BA list contained 3 arfcns (1,2,3), the first measurement period would return results for arfcns 1, 2, 3, 1, 2, 3, 1, 2 the next measurement returned would be 3, 1, 2, 3, 1, 2, 3, 1). For any BA list length, the measurements shall be made on a wrap-around basis (e.g: if the BA list contained 10 arfcns, the first 8 would be measured in the first PCH block period, the next PCH block period would measure the remaining 2 arfcns and then re-measure the first 6 again).

The results are added in the rxlev parameter. If five measurement samples are collected an average is calculated in the rxlev_average parameter.

For all members of the BCCH allocation a status is defined. The following values are possible:

INACTIVE	No field strength average value available.

IDLE	Only fieldstrength of the member is known. The mobile station is not synchronized and has no knowledge about the BCCH.

READ_FB_SB	This state is entered if a neighbour cell is one of the six strongest channels and the mobile station is not synchronized yet. It indicates that the mobile station shall read frequency correction burst and synchron burst as soon as possible.

FB_SB_SYNC	The channel is one of the six strongest neighbour cells and the mobile station is synchronized to the neighbour cell. RR is informed about the channel.

FB_SB_SYNC_2	The channel is one of the six strongest neighbour cells and the mobile station is synchronized to the neighbour cell. RR is not yet informed about the channel.

READ_SB	The channel is one of the six strongest neighbour cells and the mobile station must read the synchron burst again as soon as possible.

READ_BCCH	The channel is one of the six strongest neighbour cells, the mobile is synchronized to the channel and the BCCH of this channel shall be read as soon as possible.

IDLE_SYNC	The mobile station is synchronized to the channel, but the channel is not longer one of the six strongest neighbour cells.

FB_SB_FAILED	The attempt for synchronization failed, but the channel is one of the six strongest.

FB_SB_FAILED2	The attempt for synchronization failed and the channel is not one of the six strongest.

EXCLUDED	After eight synchronization attempts the channel is excluded from further attempts.

READ_SB_BCCH	All five minutes the neighbour cell BCCH must be read. This is started with a synchronisation attempt to the synchron burst followed by reading of the BCCH. The synchronisation attempt is started as soon as possible.

The status will be changed if a number of reports received from PL, or a primitive received from RR or the list of six strongest cells has changed. A counter is initialized to a value corresponding to the number of multiframes during the time period. After reception of a report from layer 1 the counter is decreased by BS_PA_MFRMS. If the value is less than BS_PA_MFRMS the time period has timed-out. This algorithm is independent from a change of BS_PA_MFRMS during the time period.

If the neighbour cell process is in state NC_IDLE the status will be changed and a procedure is started. In state NC_ACTIVE a specific procedure is running and only the status will be changed.

The possible status changes after receiving a fieldstrength report from PL are listed in the following table:

| old status | Condition | new status |
| --- | --- | --- |
| INACTIVE | 5 samples received | IDLE |
| IDLE | 1 of 6 best channels | READ_FB_SB |
| READ_FB_SB | not 1 of 6 best channels | IDLE |
| FB_SB_SYNC | not 1 of 6 best channels | IDLE_SYNC |
|  | more than 30 seconds | READ_SB |
|  | more than five minutes | READ_SB_BCCH |
| FB_SB_SYNC_2 | not 1 of 6 best channels | IDLE_SYNC |
|  | more than 30 seconds | READ_SB |
|  | more than five minutes | READ_SB_BCCH |
| READ_SB | not 1 of 6 best channels | IDLE |
|  | more than five minutes | READ_SB_BCCH |
| READ_BCCH | Not 1 of 6 best channels | IDLE_SYNC |
| IDLE_SYNC | More than 30 seconds | IDLE |
|  | 1 of 6 best channels | FB_SB_SYNC |
| FB_SB_FAILED | 1. to 4. Attempt, more than 10 seconds | READ_FB_SB |
|  | 5. to 8. Attempt, more than 20 seconds | READ_FB_SB |
|  | More than 8. Attempt | EXCLUDED |
|  | Not 1 of 6 best channels | FB_SB_FAILED2 |
|  | Reselection | ÎDLE |
|  | Fieldstrength jump +6dBm | IDLE |
| FB_SB_FAILED2 | 1 of 6 best channels, less than 30 seconds | FB_SB_FAILED |
|  | 1 of 6 best channels, more than 30 seconds | FB_SB_FAILED |
|  | Reselection | ÎDLE |
|  | Fieldstrength jump +6dBm | IDLE |
| EXCLUDED | Reselection | ÎDLE |
|  | Fieldstrength jump +6dBm | IDLE |

The conditions „more than 10/20/30 seconds“ are implemented by counters using BS_PA_MFRMS The criterion „1 of 6 best channels“ is calculated for all cells with a status unequal to INACTIVE. The calculation depends on the multi-band parameter in the dualband variant, else it depends on the fieldstrength.

(RR 1)

If a measurement report is send the neighbour cell process adds all neighbour cells for which synchronization was successfull (status FB_SB_SYNC, READ_SB or READ_BCCH) and the NCC permitted check was successfull.

The measurement report contains only cells for which national colour code (NCC) is available in the NCC permitted field of the BCCH data.

The NCC is part of the BSIC. The NCC permitted field is a bitmap of allowed NCCs in the measurement report and will be received by RR with the SYSTEM INFORMATION TYPE 2 message. The following figure shows in which case an NCC is valid. If it is not valid, the cell is not included in the measurement report:

NCC Check

## Synchronisation to Neighbour Cells

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             *                             |

|                          (TI 1)                           |

|                             |    MPH5_NCELL_FB_SB_READ    |

|                             *============================>*

|                             |                          (PL 1)

|                             |                             |

a) synchronisation failed

|                             |                             |

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 2)                           |

|                             |                             |

b) synchronisation successfull

|                             |                             |

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 3)                           |

|                             |                             |

(TI 1)

If the neighbour cell process is idle after processing a report of layer 1 or entering idle state after processing a specific procedure, it is checked whether the following conditions are true:

- At least one neighbour cell has the status READ_FB_SB and

- No neighbour cell has the status READ_SB, READ_BCCH or READ_SB_BCCH

(PL 1)

For the first neighbour cell with the status READ_FB_SB an attempt is initiated with the primitive MPH5_NCELL_FB_SB_READ. The neighbour cell process enters the state active

(TI 2)

Layer 1 indicates a failed synchronisation attempt. The cell gets the status FB_SB_FAILED or EXCLUDED (depending on the attempt counter), the c_attempt counter is incremented and the c_report variable is initialised with 10 or 20 seconds. For this time no new attempt is carried out for this channel although it may have excellent fieldstrength values. The neighbour cell process enters state idle.

(TI 3)

Layer 1 indicates a successfull synchronisation. The cell gets the status FB_SB_SYNC. The c_report variable is set to 30 seconds and the attempt counter is cleared. The frame offset, the time alignment and the bsic are stored. The neighbour cell process enters state idle.

## Confirmation of Synchronisation to Neighbour Cells

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             *                             |

|                          (TI 1)                           |

|                             |      MPH5_NCELL_SB_READ     |

|                             *============================>*

|                             |                          (PL 1)

|                             |                             |

a) synchronisation failed

|                             |                             |

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 2)                           |

|                             |                             |

b) synchronisation successfull, same bsic

|                             |                             |

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 3)                           |

|                             |                             |

c) synchronisation successfull, different bsic

|                             |                             |

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 4)                           |

|                             |                             |

(TI 1)

If the neighbour cell process is in state idle after processing a report of layer 1 or entering idle after processing a specific procedure, it is checked whether the following condition is true:

- From all cells with the status READ_SB, READ_BCCH or READ_SB_BCCH has the cell with the lowest c_report value the status READ_SB

(PL 1)

The procedure is initiated by a MPH5_NCELL_SB_READ primitive.

(TI 2)

Layer 1 indicates a failed reading attempt. The cell gets the status FB_SB_FAILED and the c_report variable is set to a value depending on the failed attempt counter. For this time period no new attempt is carried out for this channel although it may have excellent fieldstrength values. The neighbour cell process enters state idle.

(TI 3)

Layer 1 indicates a successful synchronisation with the same bsic. This indicates that the mobile station is synchronized to the same neighbour cell as the last time. The cell gets the status FB_SB_SYNC if the previous status was READ_SB. The c_report variable is set to 30 seconds. The frame offset and the time alignment are stored. The neighbour cell process enters state idle.

(TI 4)

Layer 1 indicates a successfull synchronisation with a different bsic. This indicates that the mobile station is synchronized to a different neighbour cell as the last time. The cell gets the status FB_SB_SYNC. The c_report variable is set to 30 seconds. The frame offset, the time alignment and the bsic are stored. The neighbour cell process enters state idle.

## Reading of Neighbour Cell BCCH

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             *                             |

|                          (TI 1)                           |

|                             |      MPH5_NCELL_SB_READ     |

|                             *============================>*

|                             |                          (PL 1)

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 2)                           |

|                             |    MPH5_NCELL_BCCH_READ     |

|                             *============================>*

|                             |                          (PL 2)

|                             |                             |

b) BCCH reading failed

|                             |                             |

|                             |       MPH5_DATA_IND         |

|                             *<============================*

|                          (TI 3)                           |

|        MPH_ERROR_IND        |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

c) BCCH reading successfull, same bsic

|                             |                             |

|                             |       MPH5_DATA_IND         |

|                             *<============================*

|                          (TI 4)                           |

|       MPH_UNITDATA_IND      |                             |

*<============================*                             |

(RR 2)                          |                             |

|                             |                             |

d) BCCH reading successfull, wrong bsic

|                             |                             |

|                             |       MPH5_DATA_IND         |

|                             *<============================*

|                          (TI 5)                           |

|        MPH_ERROR_IND        |                             |

*<============================*                             |

(RR 3)                          |                             |

|                             |                             |

(TI 1)

All five minutes TI++ requests reading of neighbourcell information. Only system information type 3 message shall be read including the cell reselection parameters. This request is carried out only for cells to which the mobile station is synchronized. The status of the requested cell is set to READ_SB_BCCH.

If the neighbour cell process is in state idle after processing a report of layer 1 or entering idle after processing a specific procedure, it is checked whether the following condition is true:

- From all cells with the status READ_SB, READ_BCCH or READ_SB_BCCH has the cell with the lowest c_report value the status READ_SB_BCCH

(PL 1)

The procedure is initiated by a MPH5_NCELL_SB_READ primitive.

(TI 2)

Layer 1 indicates a successful synchronisation with the same bsic. This indicates that the mobile station is synchronized to the same neighbour cell as the last time. The cell gets the status READ_BCCH. The frame offset and the time alignment are stored.

(TI 2)

If the neighbour cell process is in state idle after processing a report of layer 1 or entering idle after processing a specific procedure, it is checked whether the following condition is true:

- From all cells with the status READ_SB, READ_BCCH or READ_SB_BCCH has the cell with the highest c_report value the status READ_BCCH

(PL 2)

The procedure is initiated by a MPH5_NCELL_BCCH_READ primitive. The parameter si_bit_map is set to reading system information type 3 / 4 / 7 / 8 message.

(TI 3)

The BCCH data block indicates that reading has failed. The data block is discharged. The cell gets the status FB_SB_SYNC. The neighbour cell process enters state idle.

(RR 1)

An error indication is sent to RR with the cause BCCH decode error.

(TI 4)

The BCCH data block indicates that reading was successfull and the bsic is correct. Depending on the message type according the following table, BCCH reading is stopped or not. The cell gets the status FB_SB_SYNC. The c_report variable is resetted. The frame offset is stored. The neighbour cell process enters state idle.

| Message Type |  |
| --- | --- |
| System Information Type 3 | forward message and stop reading |
| System Information Type 4 | forward message and stop reading, if message indicates no additional information necessary |
| System Information Type 7 / 8 | forward message and stop reading if system information type 4 has indicated that additional information are necessary, else ignore. |

(RR 2)

The BCCH data block is forwarded to RR.

(TI 5)

The BCCH data block indicates that reading was successfull but the bsic is wrong. The cell gets the status FB_SB_SYNC. The frame offset and the bsic are stored. The neighbour cell process enters state idle.

(RR 3)

An error indication is sent to RR with the cause BCCH decode error.

# Cell Reselection

Cell Reselection is the change of the cell in idle mode. The decision is made by RR depending on the fieldstrength measurements of the lower layer.

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|      MPH_IDLE_REQ           |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |       MPH5_STOP_IDLE        |

|                             *============================>*

|                             |                          (PL 1)

|                             |   MPH5_START_RESELECT_REQ   |

|                             *============================>*

|                             |                          (PL 2)

|                             |                             |

(TI 1)

RR starts a cell reselection. The new channel description is send to TI++. One flag in the primitive indicates start of cell reselection.

(PL 1)

All running idle processes are stopped.

(PL 2)

Reselection is started in layer 1. It is necessary to check the BCCH of the neighbourcell. TI++ initiates reading of the full BCCH of the neighbour cell. Time alignment and frame offset are set by TI++.

## Reading of Neighbour Cell BCCH

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             |       MPH5_DATA_IND         |

|                             *<============================*

|                          (TI 1)                           |

|        MPH_UNITDATA_IND     |                             |

*<============================*                             |

(DL 1)                          |                             |

|                             |      MPH5_STOP_RESELECT     |

|                             *============================>*

|                             |                          (PL 1)

|                             |     MPH5_START_REORG_PG     |

|                             *============================>*

|                             |                          (PL 2)

|                             |       MPH5_DATA_IND         |

|                             *<============================*

|                          (TI 1)                           |

|        MPH_UNITDATA_IND     |                             |

*<============================*                             |

(DL 1)                          |                             |

|                             |                             |

(TI 1)

The BCCH data of the neighbour cell are read by layer 1. This is carried out for all BCCH blocks defined in the previous request.

(DL 1)

The valid BCCH blocks are send to RR.

(PL 1)

After reception of the first BCCH block, cell reselection as offered by the layer 1 interface is stopped.

(PL 2)

The MS switches to paging reorganization.

## End of Cell Reselection, cell is suitable

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|        MPH_IDLE_REQ         |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |       MPH5_STOP_IDLE        |

|                             *============================>*

|                             |                          (PL 1)

|                             |     MPH5_START_NORMAL_PG    |

|                             *============================>*

|                             |                          (PL 2)

|                             |                             |

(TI 1)

RR has decided that the neighbourcell is suitable. It forwards the new paging parameter to TI++.

(PL 1)

TI++ stops all previous idle mode activities like BCCH reading.

(PL 2)

TI++ configures the layer 1.

## BCCH of Neighbour Cell is not suitable, next cell

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|        MPH_IDLE_REQ         |                             |

*============================>*                             |

|                          (TI 2)                           |

|                             |       MPH5_STOP_IDLE        |

|                             *============================>*

|                             |                          (PL 1)

|                             |   MPH5_START_RESELECT_REQ   |

|                             *============================>*

|                             |                          (PL 2)

|                             |                             |

(TI 2)

RR has checked the incoming BCCH data for the neighbour cell. The cell is not suitable but there is another candidate for cell reselection.

(PL 1)

TI++ stops all idle mode activities like BCCH reading for the previous cell.

(PL 2)

The next attempt is started. The time alignment and frame offset for this neighbour cell are send to layer 1. Reading of the full BCCH is requested.

## BCCH of Neighbour Cell is not readable, next cell

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             |       MPH5_DATA_IND         |

|                             *<============================*

|                          (TI 1)                           |

|        MPH_ERROR_IND        |                             |

*<============================*                             |

(DL 1)                          |                             |

|        MPH_IDLE_REQ         |                             |

*============================>*                             |

|                          (TI 2)                           |

|                             |       MPH5_STOP_IDLE        |

|                             *============================>*

|                             |                          (PL 1)

|                             |   MPH5_START_RESELECT_REQ   |

|                             *============================>*

|                             |                          (PL 2)

|                             |                             |

(TI 1)

The BCCH data of the neighbour cell are read by layer 1. This is carried out for all BCCH blocks defined in the previous request.

(DL 1)

An invalid BCCH block is detected and an error is forwarded to RR.

(TI 2)

The cell is not suitable but there is another candidate for cell reselection.

(PL 1)

TI++ stops all idle mode activities like BCCH reading for the previous cell.

(PL 2)

The next attempt is started. The time alignment and frame offset for this neighbour cell are send to layer 1. Reading of the full BCCH is requested.

## BCCH Error, no further cell available

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             *                             |

|                          (TI 1)                           |

|       MPH_POWER_REQ         |                             |

*============================>*                             |

|                          (TI 2)                           |

|                             |      MPH5_STOP_RESELECT     |

|                             *============================>*

|                             |                          (PL 1)

|                             |        MPH5_RXLEV_REQ       |

|                             *============================>*

|                             |                          (PL 2)

|                             *...                          |

|                             |                             |

(TI 1)

A previous cell reselection attempt has failed.

(TI 2)

If RR has no further candidate for cell reselection and the old serving cell is not available (e.g. cell is barred) a Cell Selection is initiated by RR.

(PL 1)

The cell reselection process in layer 1 is stopped.

(PL 2)

Reading of fieldstrength values in layer 1 is triggered.

# Connection Establishment

The mobile station is in idle mode and starts connection establishment. This is possible for the mobile originated call by initiation of mobility management or for the mobile terminated call after reception of a paging message for the mobile station.

## Start of Connection Establishment

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|      MPH_CLASSMARK_REQ      |                             |

*============================>*                             |

|                          (TI 1)                           |

|         MPH_IDLE_REQ        |                             |

*============================>*                             |

|                          (TI 2)                           |

|   MPH_RANDOM_ACCESS_REQ     |                             |

*============================>*                             |

|                          (TI 3)                           |

|                             |     MPH5_STOP_IDLE_MODE     |

|                             *============================>*

|                             |                          (PL 1)

|                             |    MPH5_START_ACCESS_LINK   |

|                             *============================>*

|                             |                          (PL 2)

|                             |          MPH_RA_REQ         |

|                             *============================>*

|                             |                          (PL 3)

|                             |          MPH_RA_CNF         |

|                             *<============================*

|                          (TI 4)                           |

|   MPH_RANDOM_ACCESS_CNF     |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(TI 1)

During cell selection RR has send classmark information to the lower layer. The power class (or two power classes for dualband) of the mobile station is stored in the RACH control process. Note that the coding is different (RR 0 .. 4, PL 1 .. 5).

(TI 2)

During configuring the idle mode the MAX_TX_PWR_CCCH value from the system information messages is send to the lower layer. The value is stored in the RACH control process.

(TI 3)

The RACH control process is in state RA_NULL. RR starts the connection establishment. The primitive contains the number of random bursts (equal max_retrans + 1 from system information), the delta between to random bursts as multiple of TDMA frames and the channel request messages itself. The content of the primitive is stored.

(PL 1)

The idle mode is stopped.

(PL 2)

The access link is started by sending a MPH5_START_ACCESS_LINK primitive to PL. That means that in downlink direction the mobile station will send random burst on request and in uplink direction the full CCCH is listened. Therefore the RACH control process enters state RA_ACTIVE.

(PL 3)

The first random burst is send.

(TI 2)

Sending of the random burst is confirmed by layer 1. The primitive contains the absolute frame number. The short frame number FN = absolute frame number modulo 42432 is calculated. This frame number is splitted into the variables T1, T2 and T3:

- T1 = (FN div 1326) mod 32

- T2 = FN mod 26

- T3 = FN mod 51.

(RR 1)

The calculated values are forwarded to the upper layer.

This procedure is carried out for all channel request messages. After the last confirmation the RACH control process enters the state RA_LISTEN_TO_CCCH.

## Immediate Assignment Message for MS

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             |       MPH5_DATA_IND         |

|                             *<============================*

|                          (TI 1)                           |

|      MPH_UNITDATA_IND       |                             |

*<============================*                             |

(RR 1)                          |                             |

|      MPH_DEDICATED_REQ      |                             |

*============================>*                             |

|                          (TI 2)                           |

|                             |    MPH5_STOP_ACCESS_LINK    |

|                             *============================>*

|                             |                           (PL 1)

|                             |                             |

(TI 1)

After starting the access link, layer 1 listens to the downlink CCCH. All data blocks are forwarded to the upper layer.

(RR 1)

If the block is valid it will be forwarded to the upper layer.

(TI 2)

If it is a immediate assignment or immediate assignment extended message, RR checks whether the random reference and the frame number in this message is equal to one of the three last sended random bursts of the mobile station. If this matches RR configures the dedicated mode for SDCCH or FACCH depending on the assigned channel configuration. Handling of this is carried out by the SDCCH or FACCH control process.

(PL 1)

Configuration of dedicated mode means on the other hand the end of the RACH control process. Layer 1 is informed and the RACH control process enters state RA_NULL.

## T3126 Expiry in RR

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|       MPH_IDLE_REQ          |                             |

|       MPH_POWER_REQ         |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |    MPH5_STOP_ACCESS_LINK    |

|                             *============================>*

|                             |                           (PL 1)

|                             |                             |

(TI 1)

If all channel request messages are sent RR starts the timer T3126 defining the time in which an immediate assignment message may send from the infrastructure. After timeout RR goes back to idle mode (normally by MPH_IDLE_REQ). If no suitable cell is available a cell selection is started by power measurements with MPH_POWER_REQ.

(PL 1)

Going back to the idle mode or starting cell selection meas on the other hand the end of the RACH control process. Layer 1 is informed and the RACH control process enters state RA_NULL.

## Immediate Assignment Reject Message for MS

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             |       MPH5_DATA_IND         |

|                             *<============================*

|                          (TI 1)                           |

|        MPH_UNITDATA_IND     |                             |

*<============================*                             |

(RR 1)                          |                             |

|    MPH_RANDOM_ACCESS_REQ    |                             |

*============================>*                             |

|                          (TI 2)                           |

|                             |   MPH5_STOP_ACCESS_BURSTS   |

|                             *============================>*

|                             |                           (PL 1)

|         MPH_IDLE_REQ        |                             |

|         MPH_POWER_REQ       |                             |

|      MPH_DEDICATED_REQ      |                             |

*============================>*                             |

|                          (TI 3)                           |

|                             |    MPH5_STOP_ACCESS_LINK    |

|                             *============================>*

|                             |                           (PL 2)

|                             |                             |

(TI 1)

After starting the access link, layer 1 listens to the downlink CCCH. All data blocks are forwarded to the upper layer.

(RR 1)

If the block is valid it will be forwarded to the upper layer.

(TI 2)

If it is a immediate assignment reject message, RR checks whether the random reference and the frame number in this message is equal to one of the three last sended random bursts of the mobile station. If this matches RR stops sending of random burst if not all bursts are send. It starts T3126 and listens to the incoming CCCH messages.

(PL 1)

Sending of the remaining bursts is stopped and the RACH control process enters the state RA_LISTEN_TO_CCCH.

(TI 3)

After timeout of T3126 RR goes back to idle mode (normally by MPH_IDLE_REQ). If no suitable cell is available a cell selection is started with power measurements.

(PL 2)

Going back to the idle mode or starting cell selection meas on the other hand the end of the RACH control process. Layer 1 is informed and the RACH control process enters state RA_NULL.

# Immediate Assignment

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             |       MPH5_DATA_IND         |

|                             *<============================*

|                          (TI 1)                           |

|       MPH_UNITDATA_IND      |                             |

*<============================*                             |

(RR 1)                          |                             |

|      MPH_DEDICATED_REQ      |                             |

*============================>*                             |

|                          (TI 2)                           |

|                             |    MPH5_STOP_ACCESS_LINK    |

|                             *============================>*

|                             |                           (PL 1)

|                             |    MPH5_IMMED_ASSIGN_REQ    |

|                             *============================>*

|                             |                           (PL 2)

|                             |    MPH5_IMMED_ASSIGN_CNF    |

|                             *<============================*

|                          (TI 3)                           |

|      MPH_DEDICATED_CNF      |                             |

*<============================*                             |

(RR 2)                          |                             |

*<function call dll_read_ccch>|                             |

(DL 1)                          |                             |

|                             |                             *-> SABM (L3)

|                             |                           (PL 3)

|                             |                             *<- UA (L3)

|                             |                           (PL 4)

*<function call dll_dcch_downlink>                          |

(DL 2)                          |                             |

|                             |                             |

(TI 1)

After starting the access link, layer 1 listens to the downlink CCCH. All data blocks are forwarded to the upper layer.

(RR 1)

The incoming CCCH data block is an immediate assignment or immediate assignment extended message. The request reference inside the message matches to one of three last bursts send by the mobile station.

(TI 2)

The content of the immediate assignment (extended) message is forwarded to TI++).

(PL 1)

Sending of channel request bursts in uplink direction and listening to the CCCH in downlink direction is stopped.

(PL 2)

The new dedicated channel is set up in layer 1.

(TI 3)

Layer 1 signals that the dedicated channel is configured.

(RR 2)

The confirmation is forwarded to RR. RR starts establishment of the layer 2 connection. That means it sends a DL_ESTABLISH_REQ primitive to DL containing the initial layer 3 message (CM_SERVICE_REQ or PAGING_RESPONSE).

(DL 1)

Layer 1 starts transmission on SDCCH or FACCH in uplink direction. This is carried out by a function call to avoid timing problems. DL prepares a layer 2 frame with a SABM command including the layer 3 message.

(PL 3)

The SABM command for service access point 0 including the layer 3 message is send to the base station.

(PL 4)

The answer of the base station is received by layer 1. In the successfull case this will be an UA response for SAPI 0 including the layer 3 message previously send.

(DL 2)

The incoming frame is forwarded directly to DL via a function call to avoid timing problems. DL signals to RR the establishment of the layer 2 connection.

# Channel Assignment

## Initiation

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_dcch_downlink ASSIGNMENT COMMAND>       |

(DL 1)                          |                             |

|      MPH_DEDICATED_REQ      |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |    MPH5_CHANNEL_ASSIGN_REQ  |

|                             *============================>*

|                             |                           (PL 1)

|                             |    MPH5_CHANNEL_ASSIGN_CNF  |

|                             *<============================*

|                          (TI 2)                           |

|      MPH_DEDICATED_CNF      |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(DL 1)

DL receives an ASSIGNMENT COMMAND message. This message is forwarded to RR. RR checks the information elements of the message.

If a failure is detected by RR typical an ASSIGNMENT FAILURE message is sent to the network and the procedure is not started.

If the ASSIGNMENT COMMAND message is accepted, RR starts a local end release for the established connection. That means DL changes from multiple frame established state to the idle state.

(TI 1)

With the information from the ASSIGNMENT COMMAND message RR builds a MPH_DEDICATED_REQ and sends this to TI++.

(PL 1)

The incoming information is converted to the MPH5_CHANNEL_ASSIGN_REQ primitive to layer 1. The following table show the conversions and calculations from the ASSIGNMENT COMMAND to the layer 1 primitive.

| ASSIGNMENT COMMAND | MPH_DEDICATED_REQ | MPH5_CHANNEL_ ASSIGN_REQ | Remark |
| --- | --- | --- | --- |
| Channel Description 1 after time | ch_type | channel_desc_1 |  |
| Power Command | tr_para.power | txpwr |  |
| Frequency List after time | ch_type.ma | frequency_list | * 1) |
| Cell Channel Description | ---- | ---- | * 1) |
| Channel Mode 1 | tr_para.mode | channel_mode_1 | * 4) |
| Channel Description 2 after time | ---- | channel_desc_2 | * 2) |
| Channel Mode 2 | ---- | channel_mode_2 | * 2) |
| Mobile Allocation after time | ---- | ---- | * 1) |
| Starting Time | start | starting_time |  |
| Frequency List before time | ch_type2.ma | frequency_list_ _bef_sti | *3) |
| Channel Description 1 before time | ch_type2 | channel_desc_1_ _bef_sti | * 3) |
| Channel Description 2 before time | ---- | channel_desc_2_ _bef_sti | * 2) |
| Frequency Channel Sequence before time | ---- | ---- | * 3) |
| Mobile Allocation before time | ---- | ---- | * 3) |
| Cipher Mode Setting | ciph | cipher_mode a5_algorithm cipher_key |  |

*1 )	Only one frequency list is build by RR from the

Frequency List after time or
	Cell Channel Description together with Mobile Allocation after time.

*2)	The protocol stack supports only one connection at one time.

*3)	Only one frequency list is build by RR from the
	Frequency List before time or
	Frequency Channel Sequence befor time or
	Cell Channel Description together with Mobile Allocation before time.

Information elements for before the starting time are only valid if the starting time is present.

*4)	The channel mode must be converted according to the following rules:

channel mode	RR (Air-Interface)	Layer 1

Signalling Only	0	0
	Speech	1	1 (fullrate)
			2 (halfrate)
	Data 12.0	3	3
	Data 6.0	11	4 (fullrate)
			5 (halfrate)
	Data 3.6	19	6 (fullrate)
			7 (halfrate)
	Enhanced Fullrate	21	8
	Data 14.5	15	9

(TI 2)

Layer 1 confirms the configuration of the dedicated channel.

(RR 1)

RR is informed about the configuration of the dedicated channel and starts resumption of the layer 2 connection to the network.

## Successfull Resumption

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_read_dcch>|                             |

(DL 1)                          |                             |

|                             |                             *-> SABM

|                             |                           (PL 1)

|                             |                             *<- UA

|                             |                           (PL 2)

*<function call dll_dcch_downlink>                          |

(DL 2)                          |                             |

*<function call dll_read_dcch ASSIGNMENT COMPLETE>          |

(DL 3)                          |                             |

|                             |                             *-> I

|                             |                           (PL 3)

|                             |                             *<- RR

|                             |                           (PL 4)

*<function call dll_dcch_downlink>                          |

(DL 4)                          |                             |

|                             |                             |

(DL 1)

Resumption of the layer 2 connection is started by RR with the primitive DL_RESUME_REQ to DL including the ASSIGNMENT COMPLETE message. Layer 1 indicates with the function call dll_read_dcch that it expects data for the FACCH or SDCCH. DL fills a layer 2 frame with the SABM command.

(PL 1)

Layer 1 sends the SABM command via the air interface to the base station. This SABM command includes no message.

(PL 2)

The base station answers with an UA response frame.

(DL 2)

DL receives this message directly with a function call dll_dcch_downlink from layer 1.

(DL 3)

If the next sending slot is received layer 1 calls DL with the dll_read_dcch function and DL sends the ASSIGNMENT COMPLETE message with an I command. RR is informed about the successfull resumption of the layer 2 connection.

(PL 3)

The I command is sent to the base station.

(PL 4)

The base station answers with a RR response frame.

(DL 4)

DL receives the RR response frame from layer 1.

## Resumption fails

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_read_dcch>|                             |

(DL 1)                          |                             |

|                             |                             *-> SABM

|                             |                           (PL 1)

|                             |                             *<- DM

|                             |                           (PL 2)

*<function call dll_dcch_downlink>                          |

(DL 2)                          |                             |

|    MPH_DEDICATED_FAIL_REQ   |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |    MPH5_CHANNEL_ASSIGN_REQ  |

|                             *============================>*

|                             |                           (PL 3)

|                             |    MPH5_CHANNEL_ASSIGN_CNF  |

|                             *<============================*

|                          (TI 2)                           |

|    MPH_DEDICATED_FAIL_CNF   |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(DL 1)

Resumption of the layer 2 connection is started by RR with the primitive DL_RESUME_REQ to DL including the ASSIGNMENT COMPLETE message. Layer 1 indicates with the function call dll_read_dcch that it expects data for the FACCH or SDCCH. DL fills a layer 2 frame with the SABM command.

(PL 1)

Layer 1 sends the SABM command via the air interface to the base station. This SABM command includes no message.

(PL 2)

An error situation occurs. E.g. the base station answers with a DM frame or there is no answer in the T200 time of DL.

(DL 2)

DL detects that resumption of the layer 2 connection fails and sends a DL_RELEASE_IND primitive to RR.

(TI 1)

RR starts switching back to the old channel and sends a MPH_DEDICATED_FAIL_REQ to TI++.

(PL 3)

TI++ configures the old channel for layer 1.

(TI 2)

Layer 1 confirms the configuration of the old channel.

(RR 1)

The confirmation is forwarded to RR and RR starts the reconnection attempt for the layer 2 connection.

## Successfull Reconnection

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_read_dcch>|                             |

(DL 1)                          |                             |

|                             |                             *-> SABM

|                             |                           (PL 1)

|                             |                             *<- UA

|                             |                           (PL 2)

*<function call dll_dcch_downlink>                          |

(DL 2)                          |                             |

*<function call dll_read_dcch ASSIGNMENT FAILURE>           |

(DL 3)                          |                             |

|                             |                             *-> I

|                             |                           (PL 3)

|                             |                             *<- RR

|                             |                           (PL 4)

*<function call dll_dcch_downlink>                          |

(DL 4)                          |                             |

|                             |                             |

(DL 1)

Reconnection of the layer 2 connection is started by RR with the primitive DL_RECONNECT_REQ to DL including the ASSIGNMENT FAILURE message. Layer 1 indicates with the function call dll_read_dcch that it expects data for the FACCH or SDCCH. DL fills a layer 2 frame with the SABM command.

(PL 1)

Layer 1 sends the SABM command via the air interface to the base station. This SABM command includes no message.

(PL 2)

The base station answers with an UA response frame.

(DL 2)

DL receives this message directly with a function call dll_dcch_downlink from layer 1.

(DL 3)

If the next sending slot is received layer 1 calls DL with the dll_read_dcch function and DL sends the ASSIGNMENT FAILURE message with an I command. RR is informed about the successfull reconnection of the layer 2 connection.

(PL 3)

The I command is sent to the base station.

(PL 4)

The base station answers with a RR response frame.

(DL 4)

DL receives the RR response frame from layer 1.

## Reconnection fails

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_read_dcch>|                             |

(DL 1)                          |                             |

|                             |                             *-> SABM

|                             |                           (PL 1)

|                             |                             *<- DM

|                             |                           (PL 2)

*<function call dll_dcch_downlink>                          |

(DL 2)                          |                             |

|                             |                             |

*.... back to idle mode       |                             |

|                             |                             |

(DL 1)

Reconnection of the layer 2 connection is started by RR with the primitive DL_RECONNECT_REQ to DL including the ASSIGNMENT FAILURE message. Layer 1 indicates with the function call dll_read_dcch that it expects data for the FACCH or SDCCH. DL fills a layer 2 frame with the SABM command.

(PL 1)

Layer 1 sends the SABM command via the air interface to the base station. This SABM command includes no message.

(PL 2)

An error situation occurs. E.g. the base station answers with a DM frame or there is no answer in the T200 time of DL.

(DL 2)

DL detects that reconnection of the layer 2 connection fails and sends a DL_RELEASE_IND primitive to RR. This leads to an radio link failure situation and the mobile station comes back to idle mode.

# Handover

## Asynchronous Handover

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_dcch_downlink HANDOVER COMMAND>         |

(DL 1)                          |                             |

|      MPH_DEDICATED_REQ      |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |      MPH5_ASYNC_HO_REQ      |

|                             *============================>*

|                             |                           (PL 1)

|                             |      MPH5_ASYNC_HO_CNF      |

|                             *<============================*

|                          (TI 2)                           |

|                             |    MPH5_HANDOVER_FINISHED   |

|                             *<============================*

|                          (TI 3)                           |

|      MPH_DEDICATED_CNF      |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(DL 1)

DL receives a HANDOVER COMMAND message. This message is forwarded to RR. RR checks the information elements of the message.

If a failure is detected by RR typical a HANDOVER FAILURE message is sent to the network and the procedure is not started.

(TI 1)

With the information from the HANDOVER COMMAND message RR builds a MPH_DEDICATED_REQ and sends this to TI++.

(PL 1)

The handover type indicates an asynchronous handover.

The incoming information is converted to the MPH5_ASYNC_HO_REQ primitive to layer 1. The following table show the conversions and calculations from the HANDOVER COMMAND to the layer 1 primitive.

| HANDOVER COMMAND | MPH_DEDICATED_REQ | MPH5_ASYNC_HO_REQ | Remark |
| --- | --- | --- | --- |
| Cell Description | bsic arfcn | handover_command. cell_description |  |
| Channel Description 1 after time | ch_type | handover_command. channel_desc_1 |  |
| Handover Reference | ho_para.ho_ref | handover_command. ho_acc |  |
| Power Command and Access Type | ho_para.ho_pow ho_para.ho_acc_type | handover_command. txpwr | *1 ) |
| Sychronization Indication | mode | ---- | *2 ) |
| Frequency Short List after time | ch_type.ma | handover_command. frequency_list | *4 ) |
| Frequency List after time | ---- | ---- | *4 ) |
| Cell Channel Description | ---- | ---- | *4 ) |
| Channel Mode 1 | tr_para.mode | handover_command. channel_mode_1 | *7 ) |
| Channel Description 2 after time | ---- | handover_command. channel_desc_2 | *3 ) |
| Channel Mode 2 | ---- | handover_command. channel_mode_2 | *3 ) |
| Frequency Channel Sequence after time | ---- | ---- | *4 ) |
| Mobile Allocation after time | ---- | ---- | *4 ) |
| Starting Time | start | handover_command. starting_time |  |
| Real Time Difference | ---- | ---- | *5 ) |
| Timing Advance | ---- | ---- | *5 ) |
| Frequency Short List before time | ch_type2.ma | handover_command. frequency_list_bef_sti | *6 ) |
| Frequency List before time | ---- | ---- | *6 ) |
| Channel Description 1 before time | ch_type2 | handover_command. channel_desc_1_bef_sti | *6 ) |
| Channel Description 2 before time | ---- | handover_command. channel_desc_2_bef_sti | *3 ) |
| Frequency Channel Sequence before time | ---- | ---- | *6 ) |
| Mobile Allocation before time | ---- | ---- | *6 ) |
| Cipher Mode Setting | ciph | handover_command. cipher_mode handover_command. a5_algorithm cipher_key |  |

*1 )	Access Type is not supported from layer 1.

*2 )	The type of handover is defined by the primitive type.

*3)	The protocol stack supports only one connection at one time.

*4 )	Only one frequency list is build by RR from the

Frequency Short List after time or
	Frequency List after time or
	Frequency Channel Sequence after time or
	Cell Channel Description together with Mobile Allocation after time.

*5)	For asynchronous handover no timing advance is available.

*6)	Only one frequency list is build by RR from the
	Frequency Short List before time or
	Frequency List before time or
	Frequency Channel Sequence befor time or
	Cell Channel Description together with Mobile Allocation before time.

Information elements for before the starting time are only valid if the starting time is present.

*7)	The channel mode must be converted according to the following rules:

channel mode	RR (Air-Interface)	Layer 1

Signalling Only	0	0
	Speech	1	1 (fullrate)
			2 (halfrate)
	Data 12.0	3	3
	Data 6.0	11	4 (fullrate)
			5 (halfrate)
	Data 3.6	19	6 (fullrate)
			7 (halfrate)
	Enhanced Fullrate	21	8
	Data 14.5	15	9

(TI 2)

Layer 1 confirms the configuration of the dedicated channel. It starts sending continously HANDOVER ACCESS messages as random bursts on the dedicated channel. Timer T3124 is started.

(TI 3)

During T3124 is running the layer 1 listens on the downlink DCCH. After reception of the PHYSICAL INFORMATION message layer 1 stops T3214 and configures the received timing advance.

If the handover fails the same primitive is used with the error cause timeout.

(RR 1)

RR is informed about the successfull handover and starts resumption of the layer 2 connection to the network. If the handover fails RR is informed with the error cause timeout and reconnection is started.

## Synchronous Handover

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_dcch_downlink HANDOVER COMMAND>         |

(DL 1)                          |                             |

|      MPH_DEDICATED_REQ      |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |       MPH5_SYNC_HO_REQ      |

|                             *============================>*

|                             |                           (PL 1)

|                             |       MPH5_SYNC_HO_CNF      |

|                             *<============================*

|                          (TI 2)                           |

|                             |    MPH5_HANDOVER_FINISHED   |

|                             *<============================*

|                          (TI 3)                           |

|      MPH_DEDICATED_CNF      |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(DL 1)

DL receives a HANDOVER COMMAND message. This message is forwarded to RR. RR checks the information elements of the message.

If a failure is detected by RR typical a HANDOVER FAILURE message is sent to the network and the procedure is not started.

(TI 1)

With the information from the HANDOVER COMMAND message RR builds a MPH_DEDICATED_REQ and sends this to TI++.

(PL 1)

The handover type indicates an synchronous handover.

The incoming information is converted to the MPH5_SYNC_HO_REQ primitive to layer 1. The following table show the conversions and calculations from the HANDOVER COMMAND to the layer 1 primitive.

| HANDOVER COMMAND | MPH_DEDICATED_REQ | MPH5_ASYNC_HO_REQ | Remark |
| --- | --- | --- | --- |
| Cell Description | bsic arfcn | handover_command. cell_description |  |
| Channel Description 1 after time | ch_type | handover_command. channel_desc_1 |  |
| Handover Reference | ho_para.ho_ref | handover_command. ho_acc |  |
| Power Command and Access Type | ho_para.ho_pow ho_para.ho_acc_type | handover_command. txpwr | *1 ) |
| Sychronization Indication | mode ho_para.ho_nci | nci | *2 ) |
| Frequency Short List after time | ch_type.ma | handover_command. frequency_list | *4 ) |
| Frequency List after time | ---- | ---- | *4 ) |
| Cell Channel Description | ---- | ---- | *4 ) |
| Channel Mode 1 | tr_para.mode | handover_command. channel_mode_1 | *7 ) |
| Channel Description 2 after time | ---- | handover_command. channel_desc_2 | *3 ) |
| Channel Mode 2 | ---- | handover_command. channel_mode_2 | *3 ) |
| Frequency Channel Sequence after time | ---- | ---- | *4 ) |
| Mobile Allocation after time | ---- | ---- | *4 ) |
| Starting Time | start | handover_command. starting_time |  |
| Real Time Difference | ---- | ---- | *5 ) |
| Timing Advance | ---- | ---- | *5 ) |
| Frequency Short List before time | ch_type2.ma | handover_command. frequency_list_bef_sti | *6 ) |
| Frequency List before time | ---- | ---- | *6 ) |
| Channel Description 1 before time | ch_type2 | handover_command. channel_desc_1_bef_sti | *6 ) |
| Channel Description 2 before time | ---- | handover_command. channel_desc_2_bef_sti | *3 ) |
| Frequency Channel Sequence before time | ---- | ---- | *6 ) |
| Mobile Allocation before time | ---- | ---- | *6 ) |
| Cipher Mode Setting | ciph | handover_command. cipher_mode handover_command. a5_algorithm cipher_key |  |

*1 )	Access Type is not supported from layer 1.

*2 )	The type of handover is defined by the primitive type.

*3)	The protocol stack supports only one connection at one time.

*4 )	Only one frequency list is build by RR from the

Frequency Short List after time or
	Frequency List after time or
	Frequency Channel Sequence after time or
	Cell Channel Description together with Mobile Allocation after time.

*5)	For synchronous handover no timing advance is available.

*6)	Only one frequency list is build by RR from the
	Frequency Short List before time or
	Frequency List before time or
	Frequency Channel Sequence befor time or
	Cell Channel Description together with Mobile Allocation before time.

Information elements for before the starting time are only valid if the starting time is present.

*7)	The channel mode must be converted according to the following rules:

channel mode	RR (Air-Interface)	Layer 1

Signalling Only	0	0
	Speech	1	1 (fullrate)
			2 (halfrate)
	Data 12.0	3	3
	Data 6.0	11	4 (fullrate)
			5 (halfrate)
	Data 3.6	19	6 (fullrate)
			7 (halfrate)
	Enhanced Fullrate	21	8
	Data 14.5	15	9

(TI 2)

Layer 1 confirms the configuration of the dedicated channel. It starts sending four HANDOVER ACCESS messages as random bursts on the dedicated channel.

(TI 3)

After sending the four handover access bursts the completion of the synchronous handover. If the layer 1 detects a timing advance out of range error this will be reported with the MPH5_TA_FAIL_IND primitve. In this case no bursts are send and no change to the new channel has occured.

(RR 1)

RR is informed about the successfull handover and starts resumption of the layer 2 connection to the network. If the handover fails RR is informed with the error cause TA out of range.

## Pre-Synchronous Handover

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_dcch_downlink HANDOVER COMMAND>         |

(DL 1)                          |                             |

|      MPH_DEDICATED_REQ      |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |     MPH5_PRE_SYNC_HO_REQ    |

|                             *============================>*

|                             |                           (PL 1)

|                             |     MPH5_PRE_SYNC_HO_CNF    |

|                             *<============================*

|                          (TI 2)                           |

|                             |    MPH5_HANDOVER_FINISHED   |

|                             *<============================*

|                          (TI 3)                           |

|      MPH_DEDICATED_CNF      |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(DL 1)

DL receives a HANDOVER COMMAND message. This message is forwarded to RR. RR checks the information elements of the message.

If a failure is detected by RR typical a HANDOVER FAILURE message is sent to the network and the procedure is not started.

(TI 1)

With the information from the HANDOVER COMMAND message RR builds a MPH_DEDICATED_REQ and sends this to TI++.

(PL 1)

The handover type indicates an pre-synchronous handover.

The incoming information is converted to the MPH5_PRE_SYNC_HO_REQ primitive to layer 1. The following table show the conversions and calculations from the HANDOVER COMMAND to the layer 1 primitive.

| HANDOVER COMMAND | MPH_DEDICATED_REQ | MPH5_ASYNC_HO_REQ | Remark |
| --- | --- | --- | --- |
| Cell Description | bsic arfcn | handover_command. cell_description |  |
| Channel Description 1 after time | ch_type | handover_command. channel_desc_1 |  |
| Handover Reference | ho_para.ho_ref | handover_command. ho_acc |  |
| Power Command and Access Type | ho_para.ho_pow ho_para.ho_acc_type | handover_command. txpwr | *1 ) |
| Sychronization Indication | mode ho_para.ho_nci | nci | *2 ) |
| Frequency Short List after time | ch_type.ma | handover_command. frequency_list | *4 ) |
| Frequency List after time | ---- | ---- | *4 ) |
| Cell Channel Description | ---- | ---- | *4 ) |
| Channel Mode 1 | tr_para.mode | handover_command. channel_mode_1 | * 7) |
| Channel Description 2 after time | ---- | handover_command. channel_desc_2 | *3 ) |
| Channel Mode 2 | ---- | handover_command. channel_mode_2 | *3 ) |
| Frequency Channel Sequence after time | ---- | ---- | *4 ) |
| Mobile Allocation after time | ---- | ---- | *4 ) |
| Starting Time | start | handover_command. starting_time |  |
| Real Time Difference | ---- | ---- | *5 ) |
| Timing Advance | tr_para.tav | timing_advance_valid timing_advance |  |
| Frequency Short List before time | ch_type2.ma | handover_command. frequency_list_bef_sti | *6 ) |
| Frequency List before time | ---- | ---- | *6 ) |
| Channel Description 1 before time | ---- | handover_command. channel_desc_1_bef_sti | *6 ) |
| Channel Description 2 before time | ch_type2 | handover_command. channel_desc_2_bef_sti | *3 ) |
| Frequency Channel Sequence before time | ---- | ---- | *6 ) |
| Mobile Allocation before time | ---- | ---- | *6 ) |
| Cipher Mode Setting | ciph | handover_command. cipher_mode handover_command. a5_algorithm cipher_key |  |

*1 )	Access Type is not supported from layer 1.

*2 )	The type of handover is defined by the primitive type.

*3)	The protocol stack supports only one connection at one time.

*4 )	Only one frequency list is build by RR from the

Frequency Short List after time or
	Frequency List after time or
	Frequency Channel Sequence after time or
	Cell Channel Description together with Mobile Allocation after time.

*5)	For pre-synchronous handover the timing advance is used from the HANDOVER COMMAND if 
	available.

*6)	Only one frequency list is build by RR from the
	Frequency Short List before time or
	Frequency List before time or
	Frequency Channel Sequence befor time or
	Cell Channel Description together with Mobile Allocation before time.

Information elements for before the starting time are only valid if the starting time is present.

*7)	The channel mode must be converted according to the following rules:

channel mode	RR (Air-Interface)	Layer 1

Signalling Only	0	0
	Speech	1	1 (fullrate)
			2 (halfrate)
	Data 12.0	3	3
	Data 6.0	11	4 (fullrate)
			5 (halfrate)
	Data 3.6	19	6 (fullrate)
			7 (halfrate)
	Enhanced Fullrate	21	8
	Data 14.5	15	9

(TI 2)

Layer 1 confirms the configuration of the dedicated channel. It starts sending four HANDOVER ACCESS messages as random bursts on the dedicated channel.

(TI 3)

After sending the four handover access bursts the completion of the pre-synchronous handover. If the layer 1 detects a timing advance out of range error this will be reported with the MPH5_TA_FAIL_IND primitve. In this case no bursts are send and no change to the new channel has occured.

(RR 1)

RR is informed about the successfull handover and starts resumption of the layer 2 connection to the network. If the handover fails RR is informed with the error cause TA out of range.

## Pseudo-Synchronous Handover

Pseudo-Synchronized Handover is not supported by layer 1.

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_dcch_downlink HANDOVER COMMAND>         |

(DL 1)                          |                             |

|      MPH_DEDICATED_REQ      |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |    MPH5_PSEUDO_SYNC_HO_REQ  |

|                             *============================>*

|                             |                           (PL 1)

|                             |    MPH5_PSEUDO_SYNC_HO_CNF  |

|                             *<============================*

|                          (TI 2)                           |

|                             |    MPH5_HANDOVER_FINISHED   |

|                             *<============================*

|                          (TI 3)                           |

|      MPH_DEDICATED_CNF      |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(DL 1)

DL receives a HANDOVER COMMAND message. This message is forwarded to RR. RR checks the information elements of the message.

If a failure is detected by RR typical a HANDOVER FAILURE message is sent to the network and the procedure is not started.

(TI 1)

With the information from the HANDOVER COMMAND message RR builds a MPH_DEDICATED_REQ and sends this to TI++.

(PL 1)

The handover type indicates a pseudo-synchronous handover.

The incoming information is converted to the MPH5_PSEUDO_SYNC_HO_REQ primitive to layer 1. The following table show the conversions and calculations from the HANDOVER COMMAND to the layer 1 primitive.

| HANDOVER COMMAND | MPH_DEDICATED_REQ | MPH5_ASYNC_HO_REQ | Remark |
| --- | --- | --- | --- |
| Cell Description | bsic arfcn | handover_command. cell_description |  |
| Channel Description 1 after time | ch_type | handover_command. channel_desc_1 |  |
| Handover Reference | ho_para.ho_ref | handover_command. ho_acc |  |
| Power Command and Access Type | ho_para.ho_pow ho_para.ho_acc_type | handover_command. txpwr | *1 ) |
| Sychronization Indication | mode ho_para.ho_nci | nci | *2 ) |
| Frequency Short List after time | ch_type.ma | handover_command. frequency_list | *4 ) |
| Frequency List after time | ---- | ---- | *4 ) |
| Cell Channel Description | ---- | ---- | *4 ) |
| Channel Mode 1 | tr_para.mode | handover_command. channel_mode_1 | *7 ) |
| Channel Description 2 after time | ---- | handover_command. channel_desc_2 | *3 ) |
| Channel Mode 2 | ---- | handover_command. channel_mode_2 | *3 ) |
| Frequency Channel Sequence after time | ---- | ---- | *4 ) |
| Mobile Allocation after time | ---- | ---- | *4 ) |
| Starting Time | start | handover_command. starting_time |  |
| Real Time Difference | tr_para.tav | real_time_difference |  |
| Timing Advance | ---- | ---- | *5 ) |
| Frequency Short List before time | ch_type2.ma | handover_command. frequency_list_bef_sti | *6 ) |
| Frequency List before time | ---- | ---- | *6 ) |
| Channel Description 1 before time | ch_type2 | handover_command. channel_desc_1_bef_sti | *6 ) |
| Channel Description 2 before time | ---- | handover_command. channel_desc_2_bef_sti | *3 ) |
| Frequency Channel Sequence before time | ---- | ---- | *6 ) |
| Mobile Allocation before time | ---- | ---- | *6 ) |
| Cipher Mode Setting | ciph | handover_command. cipher_mode handover_command. a5_algorithm cipher_key |  |

*1 )	Access Type is not supported from layer 1.

*2 )	The type of handover is defined by the primitive type.

*3)	The protocol stack supports only one connection at one time.

*4 )	Only one frequency list is build by RR from the

Frequency Short List after time or
	Frequency List after time or
	Frequency Channel Sequence after time or
	Cell Channel Description together with Mobile Allocation after time.

*5)	For pseudo-synchronous handover the real time difference is calculated by RR.

*6)	Only one frequency list is build by RR from the
	Frequency Short List before time or
	Frequency List before time or
	Frequency Channel Sequence befor time or
	Cell Channel Description together with Mobile Allocation before time.

Information elements for before the starting time are only valid if the starting time is present.

*7)	The channel mode must be converted according to the following rules:

channel mode	RR (Air-Interface)	Layer 1

Signalling Only	0	0
	Speech	1	1 (fullrate)
			2 (halfrate)
	Data 12.0	3	3
	Data 6.0	11	4 (fullrate)
			5 (halfrate)
	Data 3.6	19	6 (fullrate)
			7 (halfrate)
	Enhanced Fullrate	21	8
	Data 14.5	15	9

(TI 2)

Layer 1 confirms the configuration of the dedicated channel. It starts sending four HANDOVER ACCESS messages as random bursts on the dedicated channel.

(TI 3)

After sending the four handover access bursts the completion of the pseudo-synchronous handover. If the layer 1 detects a timing advance out of range error this will be reported with the MPH5_TA_FAIL_IND primitve. In this case no bursts are send and no change to the new channel has occured.

(RR 1)

RR is informed about the successfull handover and starts resumption of the layer 2 connection to the network. If the handover fails RR is informed with the error cause TA out of range.

## Timing Advance out of Range

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_read_dcch HANDOVER FAILURE>             |

(DL 1)                          |                             |

|                             |                             *-> I

|                             |                           (PL 1)

|                             |                             *<- RR

|                             |                           (PL 2)

*<function call dll_dcch_downlink>                          |

(DL 2)                          |                             |

|                             |                             |

(DL 1)

If TI++ has signalled a timing advance out of range problem, RR sends a HANDOVER FAILURE message to DL with the cause timing advance out of range.

If the next sending slot is received layer 1 calls DL with the dll_read_dcch function and DL sends the HANDOVER FAILURE message with an I command.

(PL 1)

The I command is sent to the base station.

(PL 2)

The base station answers with a RR response frame.

(DL 2)

DL receives the RR response frame from layer 1.

## Successfull Resumption

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_read_dcch>|                             |

(DL 1)                          |                             |

|                             |                             *-> SABM

|                             |                           (PL 1)

|                             |                             *<- UA

|                             |                           (PL 2)

*<function call dll_dcch_downlink>                          |

(DL 2)                          |                             |

*<function call dll_read_dcch HANDOVER COMPLETE>            |

(DL 3)                          |                             |

|                             |                             *-> I

|                             |                           (PL 3)

|                             |                             *<- RR

|                             |                           (PL 4)

*<function call dll_dcch_downlink>                          |

(DL 4)                          |                             |

|                             |                             |

(DL 1)

RR starts a local end release for the old established connection. That means DL changes from multiple frame established state to the idle state.

Resumption of the layer 2 connection is started by RR with the primitive DL_RESUME_REQ to DL including the HANDOVER COMPLETE message. Layer 1 indicates with the function call dll_read_dcch that it expects data for the FACCH or SDCCH. DL fills a layer 2 frame with the SABM command.

(PL 1)

Layer 1 sends the SABM command via the air interface to the base station. This SABM command includes no message.

(PL 2)

The base station answers with an UA response frame.

(DL 2)

DL receives this message directly with a function call dll_dcch_downlink from layer 1.

(DL 3)

If the next sending slot is received layer 1 calls DL with the dll_read_dcch function and DL sends the HANDOVER COMPLETE message with an I command. RR is informed about the successfull resumption of the layer 2 connection.

(PL 3)

The I command is sent to the base station.

(PL 4)

The base station answers with a RR response frame.

(DL 4)

DL receives the RR response frame from layer 1.

## Resumption fails

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_read_dcch>|                             |

(DL 1)                          |                             |

|                             |                             *-> SABM

|                             |                           (PL 1)

|                             |                             *<- DM

|                             |                           (PL 2)

*<function call dll_dcch_downlink>                          |

(DL 2)                          |                             |

|    MPH_DEDICATED_FAIL_REQ   |                             |

*============================>*                             |

|                          (TI 1)                           |

|                             |    MPH5_HANDOVER_FAIL_REQ   |

|                             *============================>*

|                             |                           (PL 3)

|                             |    MPH5_HANDOVER_FAIL_CNF   |

|                             *<============================*

|                          (TI 2)                           |

|    MPH_DEDICATED_FAIL_CNF   |                             |

*<============================*                             |

(RR 1)                          |                             |

|                             |                             |

(DL 1)

Resumption of the layer 2 connection is started by RR with the primitive DL_RESUME_REQ to DL including the HANDOVER COMPLETE message. Layer 1 indicates with the function call dll_read_dcch that it expects data for the FACCH or SDCCH. DL fills a layer 2 frame with the SABM command.

(PL 1)

Layer 1 sends the SABM command via the air interface to the base station. This SABM command includes no message.

(PL 2)

An error situation occurs. E.g. the base station answers with a DM frame or there is no answer in the T200 time of DL.

(DL 2)

DL detects that resumption of the layer 2 connection fails and sends a DL_RELEASE_IND primitive to RR.

(TI 1)

RR starts switching back to the old channel and sends a MPH_DEDICATED_FAIL_REQ to TI++.

(PL 3)

TI++ forward the trigger to layer 1.

(TI 2)

Layer 1 confirms the configuration of the old channel.

(RR 1)

The confirmation is forwarded to RR and RR starts the reconnection attempt for the layer 2 connection.

## Successfull Reconnection

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_read_dcch>|                             |

(DL 1)                          |                             |

|                             |                             *-> SABM

|                             |                           (PL 1)

|                             |                             *<- UA

|                             |                           (PL 2)

*<function call dll_dcch_downlink>                          |

(DL 2)                          |                             |

*<function call dll_read_dcch HANDOVER FAILURE>           |

(DL 3)                          |                             |

|                             |                             *-> I

|                             |                           (PL 3)

|                             |                             *<- RR

|                             |                           (PL 4)

*<function call dll_dcch_downlink>                          |

(DL 4)                          |                             |

|                             |                             |

(DL 1)

Reconnection of the layer 2 connection is started by RR with the primitive DL_RECONNECT_REQ to DL including the HANDOVER FAILURE message. Layer 1 indicates with the function call dll_read_dcch that it expects data for the FACCH or SDCCH. DL fills a layer 2 frame with the SABM command.

(PL 1)

Layer 1 sends the SABM command via the air interface to the base station. This SABM command includes no message.

(PL 2)

The base station answers with an UA response frame.

(DL 2)

DL receives this message directly with a function call dll_dcch_downlink from layer 1.

(DL 3)

If the next sending slot is received layer 1 calls DL with the dll_read_dcch function and DL sends the HANDOVER FAILURE message with an I command. RR is informed about the successfull reconnection of the layer 2 connection.

(PL 3)

The I command is sent to the base station.

(PL 4)

The base station answers with a RR response frame.

(DL 4)

DL receives the RR response frame from layer 1.

## Reconnection fails

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

*<function call dll_read_dcch>|                             |

(DL 1)                          |                             |

|                             |                             *-> SABM

|                             |                           (PL 1)

|                             |                             *<- DM

|                             |                           (PL 2)

*<function call dll_dcch_downlink>                          |

(DL 2)                          |                             |

|                             |                             |

*.... back to idle mode       |                             |

|                             |                             |

(DL 1)

Reconnection of the layer 2 connection is started by RR with the primitive DL_RECONNECT_REQ to DL including the HANDOVER FAILURE message. Layer 1 indicates with the function call dll_read_dcch that it expects data for the FACCH or SDCCH. DL fills a layer 2 frame with the SABM command.

(PL 1)

Layer 1 sends the SABM command via the air interface to the base station. This SABM command includes no message.

(PL 2)

An error situation occurs. E.g. the base station answers with a DM frame or there is no answer in the T200 time of DL.

(DL 2)

DL detects that reconnection of the layer 2 connection fails and sends a DL_RELEASE_IND primitive to RR. This leads to an radio link failure situation and the mobile station comes back to idle mode.

# Message Interfaces

The message interfaces are build not by primitives. Instead function calls used to fullfill the timing requirements. These function calls are initiated from an interrupt service routine in layer 1 . Therefore it is necessary to avoid trace outputs. Nevertheless for testing into a simulation variant a primitive interface is used as well for data link layer.

## SACCH Uplink Interface

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

*<function call dll_read_sacch>                          |

(DL 1)                          |                          |

|                             |                          |

a) No SAPI 0 information available

|                             |                          |

|                             |                          *-> Empty Frame

|                             |                       (PL 1)

|                             |                          |

b) SAPI 0 information available

|                             |                          |

|                             |                          *-> UI (SAPI 0)

|                             |                       (PL 2)

|                             |                          |

c) SAPI 3 information available

|                             |                          |

|                             |                          *-> I (SAPI 3)

|                             |                       (PL 3)

|                             |                          |

d) SAPI 0 information available, last frame was SAPI 3

|                             |                          |

|                             |                          *-> UI (SAPI 0)

|                             |                       (PL 4)

|                             |                          |

(DL 1)

Layer 1 indicates with the function call dll_read_sacch that it expects data for the SACCH. On SACCH uplink direction measurement reports as UI frames for SAPI 0 are send and short message service messages as I frames for SAPI 3 if the main dedicated channel is FACCH.

The prototype for the dll_read_sacch function is:

T_RADIO_FRAME * dll_read_sacch (T_CHANNEL_MODE chn_mode)

The structure T_RADIO_FRAME is an array of 23 bytes.

(PL 1)

If no information for SAPI 0 is availabe, layer 2 sends an empty frame to layer 1. This may happens in start phase of a connection if RR has not build a first measurement report message. A DL UNITDATA CNF primitive is given to RR to request sending of new measurement report data. An empty frame has the following structure:

0   0   0   0   0   0   0   1	Address field
0   0   0   0   0   0   1   1	Control field	(UI frame)
0   0   0   0   0   0   0   1	Length field	(0 bytes)
0   0   1   0   1   0   1   1
...
0   0   1   0   1   0   1   1

The length of empty frames is 21 bytes for SACCH and 23 bytes for SDCCH. Frames for SACCH starts at position 2, that means byte 0 and 1 are free for the layer 1 header.

(PL 2)

New measurement report data are send with a DL UNITDATA REQ primitive from RR to DL. DL builds with this data an unnumbered information frame for SAPI 0 and sends this by the return value of the function to layer 1. New measurement report data is requested from RR by sending a DL UNITDATA CNF primitive to RR.

(PL 3)

If SAPI 3 information is available for SACCH, DL builds an I frame or a SABM frame during establishment of the layer 3 connection. This frame is given by the return value of the function to layer 1.

(PL 4)

If SAPI 3 information is available for SACCH and the last transmitted frame on SACCH was a SAPI 3 frame transmission of SAPI 0 data is processed. This is done to fullfill the requirement that at least each second frame must be a measurement report (equal SAPI 0 frames).

## SACCH Downlink Interface

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

|                             |        MPH5_DATA_IND     |

|                             *<=========================*

|                          (TI 1)                        |

|                             |                          |

a) valid SACCH block (acknowledged information)

|                             |                          |

|          PH_DATA_IND        |                          |

*<===========================*                          |

(DL 1)                          |                          |

|                             |                          |

b) valid SACCH block (unacknowledged information)

|                             |                          |

|        MPH_UNITDATA_IND     |                          |

*<===========================*                          |

(RR 1)                          |                          |

|                             |                          |

c) invalid SACCH block

|                             |                          |

|                             *                          |

|                          (TI 2)                        |

|                             |                          |

d) invalid SACCH block, radio link failure

|                             |                          |

|                             *                          |

|                          (TI 3)                        |

|        MPH_ERROR_IND        |                          |

*<===========================*                          |

(RR 2)                          |                          |

|                             |                          |

(TI 1)

After reception of a SACCH downlink frame layer 1 sends this frame to TI++ using the MPH5 DATA IND primitive. On SACCH downlink the SYS INFO TYPE 5 and 6 and SMS layer 2 frames are send.

(DL 1)

If it is a valid block the radio link counter is incremented if it is less than the inital value. The frame is forwarded to DL with the primitive PH DATA IND in the windows environment and with a function call in the target system if it is an acknowledged frame (that means SMS layer 2 frame).

(RR 1)

If it is unacknowledged information the frame is forwarded to RR directly.

(TI 2)

An invalid block has received. The block is discharged and the radio link failure counter is decreased. The radio link failure criterion is not reached.

(TI 3)

An invalid block has received. The block is discharged and the radio link failure counter is decreased. The radio link failure criterion is reached.

(RR 2)

An error indication is send to RR.

## FACCH/SDCCH Uplink Interface

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

*<function call dll_read_dcch>|                          |

(DL 1)                          |                          |

|                             |                          |

a) No information available, Signalling only

|                             |                          |

|                             |                          *-> Empty Frame

|                             |                       (PL 1)

|                             |                          |

b) information available

|                             |                          |

|                             |                          *-> I (SAPI 0)

|                             |                       (PL 2)

|                             |                          |

c) No information available, not Signalling only

|                             |                          |

|                             |                          *

|                             |                       (PL 3)

|                             |                          |

(DL 1)

Layer 1 indicates with the function call dll_read_dcch that it expects data for the FACCH or SDCCH. On FACCH and SDCCH layer 3 messages with SAPI 0 from RR, MM, CC and SS are send to the base station. If the channel is SDCCH additional SMS messages with SAPI 3 are send.

The prototype for the dll_read_dcch function is:

T_RADIO_FRAME * dll_read_dcch (T_CHANNEL_MODE chn_mode)

The structure T_RADIO_FRAME is an array of 23 bytes.

(PL 1)

The actual channel mode is signalling only. Layer 2 sends fillframes which are discharged on the network side. Therefore the frame structure of 23 bytes is filled with a fill byte pattern (0x2B) and forwarded with the return value to layer 1.

(PL 2)

DL has stored information for the base station. If fills the frame structure with the data and the appropriate layer 2 command (e.g. SABM or I or RR frame) and sends it by the return value of the function to layer 1.

(PL 3)

If the channel mode is unequal to signalling only and no data is available for transmission a NULL pointer is given back by the function. This indicates that the layer 1 has nothing to do.

## FACCH/SDCCH Downlink Interface

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

|                             *<function call dll_dcch_downlink>

|                          (TI 1)                        |

|                             |                          |

a) No information available

|                             |                          |

|                             *                          |

|                          (TI 2)                        |

|                             |                          |

b) information available, invalid, radio link failure not reached

|                             |                          |

|                             *                          |

|                          (TI 3)                        |

|                             |                          |

c) information available, invalid, radio link failure reached

|                             |                          |

|        MPH_ERROR_IND        |                          |

*<============================*                          |

(RR 1)                          |                          |

|                             |                          |

d) information available, valid

|                             |                          |

|        PH_DATA_IND          |                          |

*<============================*                          |

(DL 1)                          |                          |

|                             |                          |

(TI 1)

Layer 1 indicates with the function call dll_dcch_downlink that it has read FACCH or SDCCH downlink. On FACCH and SDCCH layer 3 messages with SAPI 0 from RR, MM, CC and SS are send by the base station. If the channel is SDCCH additional SMS messages with SAPI 3 are send.

The prototype for the dll_dcch_downlink function is:

void dll_dcch_downlink (T_RADIO_FRAME * frame, BOOL valid)

The structure T_RADIO_FRAME is an array of 23 bytes.

(TI 2)

If layer 1 has not received information the frame pointer is set to NULL. There is no reaction by the upper layer.

(TI 3)

Layer 1 indicates reception of an invalid block. The frame is ignored and the radio link counter is decremented. The radio link error condition is not fullfilled.

(RR 1)

Layer 1 indicates reception of an invalid block. The frame is ignored and the radio link counter is decremented. The radio link error condition is fullfilled and an error indication is send to RR.

(DL 1)

Layer 1 indicates reception of a valid block. The frame is forwarded to DL with the primitive PH DATA IND in the windows environment and with a function call in the target system..

# Frequency Redefinition

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

*                             |                          |

(RR 1)                          |                          |

|     MPH_FREQ_REDEF_REQ      |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             |MPH5_CHANGE_FREQUENCY_REQ |

|                             *=========================>*

|                             |                       (PL 1)

|                             |MPH5_CHANGE_FREQUENCY_CNF |

|                             *<=========================*

|                          (TI 2)                        |

|                             |                          |

(RR 1)

RR has received a frequency redefinition message. The new frequencies are calculated.

(TI 1)

The new frequencies and a start time are forwarded to TI++. The parameters are converted according to the following table:

| FREQUENCY REDEFINTION | MPH_FREQ_REDEF_REQ | MPH5_CHANNEL_FREQUENCY_REQ | Remark |
| --- | --- | --- | --- |
| Channel Description | chan_desc | channel_desc |  |
| Mobile Allocation | chan_desc.ma | frequency_list | * 1) |
| Starting Time | start | starting_time |  |
| Cell Channel Description | ----- | ----- | * 1) |

*1 )	The frequency list is build from mobile allocation and cell channel description.

(PL 1)

Layer 1 receives the new frequency list and switches to the new dedicated channel.

(TI 2)

A confirmation is send to TI++.

# Channel Mode Modify

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

*                             |                          |

(RR 1)                          |                          |

|     MPH_CHANNEL_MODE_REQ    |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             |MPH5_CHAN_MODE_MODIFY_REQ |

|                             *=========================>*

|                             |                       (PL 1)

|                             |MPH5_CHAN_MODE_MODIFY_CNF |

|                             *<=========================*

|                          (TI 2)                        |

|                             |                          |

(RR 1)

RR has received a channel mode modify message.

(TI 1)

The new channel mode is forwarded to TI++. The parameters are converted according to the following table:

| CHANNEL MODE MODIFY | MPH_CHANNEL_MODE_REQ | MPH5_CHANNEL_MODE_MODIFY_REQ | Remark |
| --- | --- | --- | --- |
| Channel Description | ----- | subchannel | *1 ) |
| Channel Mode | mode | channel_mode | *2 ) |

*1 )	The protocol stack supports only one channel.

*2 )	The channel mode must be converted according to the following rules:

channel mode	RR (Air-Interface)	Layer 1

Signalling Only	0	0
	Speech	1	1 (fullrate)
			2 (halfrate)
	Data 12.0	3	3
	Data 6.0	11	4 (fullrate)
			5 (halfrate)
	Data 3.6	19	6 (fullrate)
			7 (halfrate)
	Enhanced Fullrate	21	8
	Data 14.5	15	9

(PL 1)

Layer 1 receives the new channel mode.

(TI 2)

A confirmation is send to TI++.

# Cipher Mode Setting

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

*                             |                          |

(RR 1)                          |                          |

|     MPH_CIPHERING_REQ       |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             |  MPH5_SET_CIPHERING_REQ  |

|                             *=========================>*

|                             |                       (PL 1)

|                             |  MPH5_SET_CIPHERING_CNF  |

|                             *<=========================*

|                          (TI 2)                        |

|                             |                          |

(RR 1)

RR has received a cipher mode setting message.

(TI 1)

The new cipher parameters are forwarded to TI++. The parameters are converted according to the following table:

| CIPHER MODE SETTING | MPH_CIPHERING_REQ | MPH5_SET_CIPHERING_REQ | Remark |
| --- | --- | --- | --- |
| Cipher Mode Setting | stat algo kc | cipher_mode a5_algorithm new_ciph_param |  |
| Cipher Response | ----- | ----- | *1 ) |

*1 )	Cipher Response is carried out by RR.

(PL 1)

Layer 1 receives the new cipher parameters.

(TI 2)

A confirmation is send to TI++.

# Dedicated Mode Neighbour Cell Procedures

## Measurement Reporting

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

|                             |                          |

|                             |     MPH5_MEAS_REPORT     |

|                             *<=========================*

|                          (TI 1)                        |

|                             |                          |

a) measurements invalid

|                             |                          |

|                             *                          |

|                          (TI 2)                        |

|                             |                          |

b) measurements valid

|                             |                          |

|     MPH_MEASUREMENT_IND     |                          |

*<============================*                          |

(RR 1)                          |                          |

|                             |                          |

(TI 1)

Layer 1 sends measurement values to TI++.

(TI 2)

The valid flag in the measurement values indicates invalid data. TI++ deletes the measurement values.

(RR 1)

If the measurement values are valid, TI++ builds a MPH MEASUREMENT IND primitive and sends the data to RR.

For all members of the BCCH allocation a status is defined. The status of idle mode is used as initial value for the dedicated mode. The status READ_BCCH is not possible in dedicated mode. It is handled like READ_SB. The neighbour cell process switches to state NCELL_DEDICATED. The following values are possible:

NULL	No field strength average value available.

IDLE	Only fieldstrength of the member is known. The mobile station is not synchronized and has no knowledge about the BCCH.

READ_FB_SB	This state is entered if a neighbour cell is one of the six strongest channels and the mobile station is not synchronized yet. It indicates that the mobile station shall read frequency correction burst and synchron burst as fast as possible.

FB_SB_SYNC	The channel is one of the six strongest neighbour cells and the mobile station is synchronized to the neighbour cell

READ_SB	The channel is one of the six strongest neighbour cells and the mobile station must read the synchron burst again as fast as possible.

IDLE_SYNC	The mobile station is synchronized to the channel, but the channel is not longer one of the six strongest neighbour cells.

FB_SB_FAILED	The attempt for synchronization failed

The status will be changed if a number of reports received from PL, or a primitive received from RR or the list of six strongest cells has changed. Cells which not passing the NCC permitted check are not member of the measurement report.

If the neighbour cell process is in state NC_DEDICATED the status will be changed and a procedure is started. In state NC_DEDICATED_ACTIVE a specific procedure is running and only the status will be changed.

The possible status changes after receiving a fieldstrength report from PL are listed in the following table:

| old status | condition | new status |
| --- | --- | --- |
| NULL | 1 sample received | IDLE |
| IDLE | 1 of 6 best channels | READ_FB_SB |
| READ_FB_SB | not 1 of 6 best channels | IDLE |
| FB_SB_SYNC | not 1 of 6 best channels | IDLE_SYNC |
|  | more than 8 seconds | READ_SB |
| READ_SB | not 1 of 6 best channels | IDLE |
| IDLE_SYNC | more than 8 seconds | IDLE |
|  | 1 of 6 best channels | FB_SB_SYNC |
| FB_SB_FAILED | more than 8 seconds | IDLE |

The condition „more than 8 seconds“ is implemented by counting the number of reports (Variable c_report). Layer 1 sends a each 104 TDMA frames (102 TDMA frames for SDCCH) one measurement report. The criterion „1 of 6 best channels“ is calculated for all cells with a status unequal to NULL. It depends on the fieldstrength of the cells and additionally on the multiband period in the dualband variant.

If a measurement report is send the neighbour cell process adds all neighbour cells for which synchronization was successfull (status FB_SB_SYNC, READ_SB or READ_BCCH).

| MPH5_MEAS_REPORT | MPH_MEASUREMENT_IND | Relationship |
| --- | --- | --- |
| --- | arfcn | 1) |
| --- | fn_offset | 2) |
| dtx_used | dtx | 3) |
| meas_valid | --- |  |
| rxlev_full_acc | rxlev_full | 4) |
| rxlev_full_nbr_meas | --- |  |
| rxlev_sub_acc | rxlev_sub | 5) |
| rxlev_sub_nbr_meas | --- |  |
| rxqual_full_acc_errors | rxqual_full | 6) |
| rxqual_full_nbr_bits | --- |  |
| rxqual_sub_acc_errors | rxqual_sub | 7) |
| rxqual_sub_nbr_bits | --- |  |
| no_of_ncell_meas | ncells.no_of_ncells | 8) |
| ncell_meas.A[33].bcch_freq | ncells.arfcn[6] | 9) |
| ncell_meas.A[33]rxlev_acc | ncells.rxlev[6] | 10) |
| --- | ncells.bsic[6] | 11) |
| --- | ncells.tav[6] | 12) |
| ncell_meas.A[33]rxlev_nbr_meas | --- |  |
| ba_id |  |  |
| timing_advance | tav | 13) |
| txpwr_used | --- |  |

1) The channel number of the current serving cell is stored in the arfcn parameter.

- The frame offset defines the number of TDMA frames since the last measurement report to RR. It is 104 for FACCH and 102 for SDCCH.

- The Parameter dtx defines whether discontinous transmission was used or not. The value is taken directly from the layer 1 measurement report.

- The Parameter rxlev_full is the receiving signal strength on the serving cell., measured on all slots. Layer 1 indicates a value with is the sum of n attempts. Each layer 1 value is in the range 0 to 255. So the value is divided by the number of attempts and 4.

- The Parameter rxlev_sub is the receiving signal strength on the serving cell., measured on a subset of slots. Layer 1 indicates a value with is the sum of n attempts. Each layer 1 value is in the range 0 to 255. So the value is divided by the number of attempts and 4.

- The Parameter rxqual_full is calculated from the values rxqual_full_acc and rxqual_full_nbr_bits. The number of detected error bits is multiplied with 500 and divided by the number of checked bits. This leads to the following table:

| RX Quality | Bit Error Rate | Calculated Value |
| --- | --- | --- |
| 0 | < 0.2 % | 0 |
| 1 | 0.2 % .. < 0.4 % | 1 |
| 2 | 0.4 % .. < 0.8 % | 2-3 |
| 3 | 0.8 % .. < 1.6 % | 4-7 |
| 4 | 1.6 % .. < 3.2 % | 8-15 |
| 5 | 3.2 % .. < 6.4 % | 16-31 |
| 6 | 6.4 % .. < 12.8 % | 32-63 |
| 7 | 12.8 % ... | > 63 |

- The Parameter rxqual_sub is calculated from the values rxqual_sub_acc and rxqual_sub_nbr_bits. The number of detected error bits is multiplied with 500 and divided by the number of checked bits.

- The number of neighbourcells is filled with the number of synchronized neighbourcells up to the maximum of six strongest cells.

- The parameter defines the channel number of the neighbourcell.

- The parameter defines the fieldstrength of the neighbourcell. It is the average of ncell_meas.A[cell].rxlev_acc calculated with ncell_meas.A[cell].rxlev_nbr_meas and divided by four to have the GSM range of 0 to 63.

- The BSIC of the neighbourcell is taken from the result of the synchronization attempt.

- The relative timing advance of the neighbourcell to the serving cell is taken from the result of the synchronization attempt.

- The timing advance of the serving cell is taken directly from the layer 1 report.

## Update of Neighbour Cell List

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

*                             |                          |

(RR 1)                          |                          |

|   MPH_NEIGHBOURCELL_REQ     |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             |   MPH5_UPDATE_BA_LIST    |

|                             *=========================>*

|                             |                        (PL 1)

|                             |                          |

(RR 1)

RR receives a System Information Type 5 message. It is a option for the base station to have different BCCH allocations in idle and in dedicated mode. If the System Information Type 5 message contains a neighbour cell description different from the BCCH allocation in idle mode, this will be detected by RR.

(TI 1)

The updated neighbour cell list is forwarded to TI++. If the serving cell is not included yet, TI++ includes it.

(PL 1)

The updated BCCH allocation is forwarded to layer 1. It will be used immediately. To avoid inconsistency the next measurement report to TI++ is signed as invalid.

## Synchronization to Neighbour Cells

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             *                             |

|                          (TI 1)                           |

|                             |    MPH5_NCELL_FB_SB_READ    |

|                             *============================>*

|                             |                          (PL 1)

|                             |                             |

a) synchronisation failed

|                             |                             |

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 2)                           |

|                             |                             |

b) synchronisation successfull

|                             |                             |

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 3)                           |

|                             |                             |

(TI 1)

If the neighbour cell process is in state NC_DEDICATED after processing a report of layer 1 or entering NC_DEDICATED after processing a specific procedure, it is checked whether the following conditions are true:

- At least one neighbour cell has the status READ_FB_SB and

- No neighbour cell has the status READ_SB or READ_BCCH

(PL 1)

For the first neighbour cell with the status READ_FB_SB an attempt is initiated with the primitive MPH5_NCELL_FB_SB_READ. The neighbour cell process enters the state NC_DEDICATED_ACTIVE.

(TI 2)

Layer 1 indicates a failed synchronisation attempt. The cell gets the status FB_SB_FAILED and the c_report variable is resetted. For the next eight seconds no new attempt is carried out for this channel although it may have excellent fieldstrength values. The neighbour cell process enters state NC_DEDICATED.

(TI 3)

Layer 1 indicates a successfull synchronisation. The cell gets the status FB_SB_SYNC. The c_report variable is resetted. The frame offset, the time alignment and the bsic are stored. The neighbour cell process enters state NC_DEDICATED.

## Confirmation of Synchronisation to Neighbour Cells

RR/DL                      TI-Add On                     TI-Layer 1

|                             |                             |

|                             *                             |

|                          (TI 1)                           |

|                             |      MPH5_NCELL_SB_READ     |

|                             *============================>*

|                             |                          (PL 1)

|                             |                             |

a) synchronisation failed

|                             |                             |

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 2)                           |

|                             |                             |

b) synchronisation successfull, same bsic

|                             |                             |

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 3)                           |

|                             |                             |

c) synchronisation successfull, different bsic

|                             |                             |

|                             |      MPH5_NCELL_SB_IND      |

|                             *<============================*

|                          (TI 4)                           |

|                             |                             |

(TI 1)

If the neighbour cell process is in state NC_DEDICATED after processing a report of layer 1 or entering NC_DEDICATED after processing a specific procedure, it is checked whether the following condition is true:

- From all cells with the status READ_SB or READ_BCCH has the cell with the highest c_report value the status READ_SB

(PL 1)

The procedure is initiated by a MPH5_NCELL_SB_READ primitive.

(TI 2)

Layer 1 indicates a failed reading attempt. The cell gets the status FB_SB_FAILED and the c_report variable is resetted. For the next eight seconds no new attempt is carried out for this channel although it may have excellent fieldstrength values. The neighbour cell process enters state NC_DEDICATED.

(TI 3)

Layer 1 indicates a successful synchronisation with the same bsic. This indicates that the mobile station is synchronized to the same neighbour cell as the last time. The cell gets the status FB_SB_SYNC if the previous status was READ_SB. If RR has requested BCCH reading no status change occur. The c_report variable is resetted. The frame offset and the time alignment are stored. The neighbour cell process enters state NC_DEDICATED.

(TI 4)

Layer 1 indicates a successfull synchronisation with a different bsic. This indicates that the mobile station is synchronized to a different neighbour cell as the last time. The cell gets the status FB_SB_SYNC. The c_report variable is resetted. The frame offset, the time alignment and the bsic are stored. The neighbour cell process enters state NC_DEDICATED.

# Channel Release

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

*                             |                          |

(RR 1)                          |                          |

*                             |                          |

(DL 1)                          |                          |

|       MPH_IDLE_REQ          |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             |   MPH_STOP_DEDICATED     |

|                             *=========================>*

|                             |                       (PL 1)

|                             |                          |

- neighbour cell is available

|                             |                          |

|                             |   MPH_START_RESELECT_REQ |

|                             *=========================>*

|                             |                       (PL 2)

|                             |                          |

b) no neighbour cell is available

|                             |                          |

|                             |  MPH_START_NORMAL_PG_REQ |

|                             *=========================>*

|                             |                       (PL 3)

|                             |                          |

(RR 1)

RR receives a Channel Release message.

(DL 1)

The layer 2 connection is disconnected.

(TI 1)

RR returns to idle mode and configures the lower layer.

(PL 1)

The channel release is signalled to layer 1.

(PL 2)

A cell reselection is started.

(PL 3)

Normal paging is configured.

# Short Message Cell Broadcast

The CBCH procedure is located in TI++. The following aspects are handled:

- Configuration of CBCH

- Collection of CBCH blocks

- Request of information (Message Id, Data Coding Scheme, Mode)

- Reading of CBCH Messages

- Suppression of multiple received CBCH messages

- Handling of Schedule Messages

- Handling of Null Message

CBCH-SMS messages may be received approximately every 2 seconds from the network. They are unacknowledged by the mobile. A CBCH channel occupies a SDCCH block and has a cycle of 8 multi frames a 51 TDMA frames. Only 4 multi frames contain CBCH information. This time is called a "CBCH slot".

The infrastructure may send schedule message. A schedule message includes information about a number of immediately following consecutive CB message, planned for a cell. The length of time covered by the CB messages referred to in a schedule message is called the "schedule period" of that message.

The schedule message contains a message description for each CB message to be broadcast during the scheduling period, in order of transmission. The position of a CB message is called the "message slot number" of the CB message, and it indicates the position of the CB message within the schedule period.

Each Message Description includes various information, including for SMSCB messages directly or indirectly all or part of their message identifier, and whether an occurrence is repetition or not.

Each Schedule message includes a begin slot number field and an end slot number field. The end slot number field indicates the length of the schedule period (i.e., specifically the number of CB message slots about which information is provided). In the case where the network uses schedule messages to describe all message slots in advance, the first schedule message of the next schedule period will be transmitted in the message slot pointed by end slot number plus 1.

The begin slot number is defined to allow the network to broadcast several scheduling message referring to the same schedule period. The begin slot number field indicates the message slot number of the CB message following the received schedule message.

The network may send unscheduled messages during empty message slots. The network need only update the begin slot number in an unscheduled schedule message to reflect the current offset within the schedule message of the next message to be transmitted.

## Configuration of CBCH

RR                 TI++                     L1                 Network

|                    |                       |                     |

|    MPH_CBCH_REQ    |                       |                     |

*===================>*                       |                     |

|                 (TI 1)                     |                     |

|                    |                       |                     |

a) CBCH available, no request from MMI

|                    |                       |                     |

|                    |    MPH5_CONFIG_CBCH   |                     |

|                    *======================>*                     |

|                    |                    (L1 1)                   |

|                    |                       |                     |

|                    |                       |                     |

b) no CBCH available

|                    |                       |                     |

|                    |    MPH5_STOP_CBCH     |                     |

|                    *======================>*                     |

|                    |                    (L1 2)                   |

|                    |                       |                     |

c) CBCH available, Request from MMI

|                    |                       |                     |

|                    |    MPH5_CONFIG_CBCH   |                     |

|                    *======================>*                     |

|                    |                    (L1 1)                   |

|                    |MPH5_START_CBCH_READING|                     |

|                    *======================>*                     |

|                    |                    (L1 3)                   |

|                    |                       |                     |

(TI 1)

As part of the idle mode configuration the CBCH is configured by RR.

(L1 1)

RR indicates that a CBCH is available. There was no request from MMI for reading CB message. TI++ configures the CBCH process in layer 1.

(L1 2)

RR indicates that the current cell has no support of CBCH. It stops any older configuration of the CBCH process in layer 1.

(L1 3)

A previous request of CB message reading is stored in TI++. After configuration of the CBCH process in layer 1 reading of the first block of CB message is started. The parameter of the primitive to layer 1 is set to zero indicating that each first block shall be read.

## Start Reading of CBCH

MMI                 TI++                     L1                 Network

|                    |                       |                     |

|    MMI_CBCH_REQ    |                       |                     |

*===================>*                       |                     |

|                 (TI 1)                     |                     |

|                    |                       |                     |

a) no CBCH available

|                    |                       |                     |

|                    *                       |                     |

|                 (TI 2)                     |                     |

|                    |                       |                     |

b) CBCH available

|                    |                       |                     |

|                    |MPH5_START_CBCH_READING|                     |

|                    *======================>*                     |

|                    |                    (L1 1)                   |

|                    |                       |                     |

(TI 1)

MMI requests reading of CB message. The primitive contains a list of message identifier ranges , a list  of data coding scheme ranges and the used mode. Two subsequent values define the lower and the upper boundary of a range. The value 0xFFFF indicates the end of the list. The mode defines whether all CB message shall be read which are inside the ranges or outside.

An example: CB messages  with message identifiers 2, 4-50 and 99 shall be read. This leads to the following primitive content:

- Message identifier	2,2

4,50

99,99

0xFFFF, ....

- Data Coding Scheme	0xFFFF, ....

- Mode	Inside the ranges

(TI 2)

If no CBCH is available, the parameters are stored.

(L1 1)

Reading of all slots is requested by layer 1. The parameter for permanent reading is zero.

## Stop Reading of CBCH

MMI                 TI++                     L1                 Network

|                    |                       |                     |

|    MMI_CBCH_REQ    |                       |                     |

*===================>*                       |                     |

|                 (TI 1)                     |                     |

|                    |                       |                     |

a) no CBCH available

|                    |                       |                     |

|                    *                       |                     |

|                 (TI 2)                     |                     |

|                    |                       |                     |

b) CBCH available

|                    |                       |                     |

|                    |     MPH5_STOP_CBCH    |                     |

|                    *======================>*                     |

|                    |                    (L1 1)                   |

|                    |                       |                     |

(TI 1)

MMI stops CBCH reading with the mode set to 0xFF.

(TI 2)

If no CBCH is available, previous stored parameters are cleared.

(L1 1)

If a CBCH is available, the CBCH process in layer 1 is stopped and all previous stored parameters are cleared.

## Collection of CBCH Blocks

### CBCH message, expected

MMI                 TI++                     L1                 Network

|                    |                       |                     |

|                    |                  CB message                 |

|                    |                 (first block)               |

|                    *<============================================*

|                 (TI 1)                     |                     |

|                    |MPH5_NEXT3_BCCH_READING|                     |

|                    *======================>*                     |

|                    |                    (L1 1)                   |

|                    |                  CB message                 |

|                    |                 (second block)              |

|                    *<============================================*

|                 (TI 2)                     |                     |

|                    |                  CB message                 |

|                    |                 (third block)               |

|                    *<============================================*

|                 (TI 3)                     |                     |

|                    |                  CB message                 |

|                    |                 (fourth block)              |

|                    *<============================================*

|                 (TI 4)                     |                     |

|                    |                       |                     |

|    MMI_CBCH_IND    |                       |                     |

*<===================*                       |                     |

(MMI 1)              |                       |                     |

|                    |MPH5_START_CBCH_READING|                     |

|                    *======================>*                     |

|                    |                    (L1 2)                   |

|                    |                       |                     |

|                    |                       |                     |

(TI 1)

TI ++ receives a CBCH block. The first byte has the following structure:

| 8 | 7 6 | 5 | 4 3 2 1 |
| --- | --- | --- | --- |
| Spare 0 | LPD 0 1 | LB | Sequence Number |

The sequence number indicates "first block" of a CBCH message. The following 22 bytes are the first quarter of the CBCH message.

The first block of a CBCH message has the following structure:

| Octet No. | Meaning |
| --- | --- |
| 1-2 | Serial Number |
| 3-4 | Message Identifier |
| 5 | Data Coding Scheme |
| 6 | Page Parameter |
| 7 .. | Content of Message |

TI++ checks whether the message identifier and the data coding scheme matches to the requested CBCH messages. An additional check is made for the serial number and the page parameter against already received messages.

(L1 1)

If both checks have the result that this is an expected not yet received message, the remaining three blocks of the message are requested.

(TI 2)

The second block of the message receives. The block content is stored.

(TI 3)

The third block of the message receives. The block content is stored.

(TI 4)

The fourth block of the message receives. The block content is stored.

(MMI 1)

The message is forwarded to MMI. The CBCH message parameter (serial number, message identifier, data coding scheme and page parameter) are stored in the list of the already received CBCH messages.

(L1 2)

Reading of the next CBCH message is triggered.

### CBCH message, not expected

MMI                 TI++                     L1                 Network

|                    |                       |                     |

|                    |                  CB message                 |

|                    |                 (first block)               |

|                    *<============================================*

|                 (TI 1)                     |                     |

|                    |                       |                     |

(TI 1)

TI ++ receives a CBCH block. The first byte has the following structure:

| 8 | 7 6 | 5 | 4 3 2 1 |
| --- | --- | --- | --- |
| Spare 0 | LPD 0 1 | LB | Sequence Number |

The sequence number indicates "first block" of a CBCH message. The following 22 bytes are the first quarter of the CBCH message.

The first block of a CBCH message has the following structure:

| Octet No. | Meaning |
| --- | --- |
| 1-2 | Serial Number |
| 3-4 | Message Identifier |
| 5 | Data Coding Scheme |
| 6 | Page Parameter |
| 7 .. | Content of Message |

TI++ checks whether the message identifier and the data coding scheme matches to the requested CBCH messages. An additional check is made for the serial number and the page parameter against already received messages.

### Schedule message

MMI                 TI++                     L1                 Network

|                    |                       |                     |

|                    |                  CB message                 |

|                    |            (first schedule block)           |

|                    *<============================================*

|                 (TI 1)                     |                     |

|                    |MPH5_NEXT3_BCCH_READING|                     |

|                    *======================>*                     |

|                    |                    (L1 1)                   |

|                    |                  CB message                 |

|                    |                 (second block)              |

|                    *<============================================*

|                 (TI 2)                     |                     |

|                    |                  CB message                 |

|                    |                 (third block)               |

|                    *<============================================*

|                 (TI 3)                     |                     |

|                    |                  CB message                 |

|                    |                 (fourth block)              |

|                    *<============================================*

|                 (TI 4)                     |                     |

|                    |MPH5_START_CBCH_READING|                     |

|                    *======================>*                     |

|                    |                    (L1 2)                   |

|                    |                       |                     |

(TI 1)

TI ++ receives a schedule message. The first byte has the following structure:

| 8 | 7 6 | 5 | 4 3 2 1 |
| --- | --- | --- | --- |
| Spare 0 | LPD 0 1 | LB | Sequence Number |

The sequence number indicates "first schedule block" of a schedule message. The following 22 bytes are the first quarter of the schedule message.

The schedule message has the following structure:

| Octet No. | Meaning |
| --- | --- |
| 1 | Begin Slot Number |
| 2 | End Slot Number |
| 3-8 | New CBSMS Message Bitmap |
| 9 - 2m | New CBSMS Message Description |
| (2m+1)-n | Other Message Description |

(L1 1)

TI++ requests reading of the remaining three blocks of the schedule message.

(TI 2)

TI++ receives the second block of the schedule message. The message block is stored.

(TI 3)

TI++ receives the third block of the schedule message. The message block is stored.

(TI 4)

TI++ receives the fourth block of the schedule message. The message block is stored.

Now the forecast for the next scheduling period is calculated. The behaviour of TI++ depends on the DRX mode of the CBCH process. There are three modes defined:

NO DRX

This is the mode before receiving the schedule message. In this mode all first blocks of the message slots are read and the remaining three blocks if the first block indicates that this is useful.

FIRST DRX

The MS has received a schedule message applicable to the next message slot. But either did not receive the first schedule message of the schedule period previous to the current one, or has not received all messages sent in that period (during that period or before).

SECOND DRX

The MS knows all the messages of interest sent in the schedule period previous to the current one, and has received the first schedule message of the current period.

The begin slot number indicates the begin of the scheduling period from the view of the incoming schedule message. If the begin slot number is 1, this is the first schedule message of that schedule period.

The end slot number indicates the length of the scheduling period.

The new CBSMS message bitmap indicates the new CBCH messages. The bitmap is valid from bit 1 to the bit with the number of the end slot number. All bits in this range with the value 1 are new CBCH messages or define free slots which must be read by the mobile.

All bits in this range with the value 0 are free slots, which must not be read by the mobile or which contains repetitions of the CBCH message, which were new in previous scheduling periods.

New CBCH message must be read in FIRST and SECOND DRX mode if the message identifier matches to the requested message identifiers.

Free Slots with bit set to 1 must be read in FIRST and SECOND DRX mode.

Old CBCH message must be read in FIRST DRX mode if the message identifier matches to the requested message identifiers.

Free Slots with bit set to 0 must not be read.

After the new CBSMS message bitmap message descriptions are defined for the messages with bit set to 1 in the bitmap. At least message descriptions for messages with bit set to 0 are added.

Using this informations TI++ indicates to layer 1 which slots shall be read by layer 1. The parameter of the MPH5_START_CBCH_READING primitive has the following meanings:

0	Read all subsequent slots

1	Read the next slot and stop reading

2	Don´t read the next slot but the second slot and stop reading

and so on.

Four possible message descriptions are defined:

First transmission of an SMSCB within the Schedule Period

| Octet 1 | MDT 1 |  | Message Identifier (high part) |
| --- | --- | --- | --- |
| Octet 2 |  | Message Identifier (low part) |  |

Retransmission Indication

| Octet 1 | MDT 0 0 | Repeated Message Slot Number |
| --- | --- | --- |

Free Message Slot, reading optional

| Octet 1 | MDT 0 1 | 0 |
| --- | --- | --- |

Free Message Slot, reading advised

| Octet 1 | MDT 0 1 | 1 |
| --- | --- | --- |

An example:

MMI has requested reading of the CB messages with the message identifiers 3, 11 and 7. The infrastructure has defined a slot period of 19 slots. Depending on the DRX mode of the mobile different behaviour of TI++ is implemented.

The infrastructure sends a schedule message in slot 0 and slot 10. Repeated (old) CB message are send in the slots 2 and 16. New CB messages are send in the slots 4 and 14. An unscheduled message is send in the slot 7 and 17.

If the mobile is in NO DRX mode it starts after configuration of CBCH at slot 5 (random slot number).

Read CBCH means reading of a CB message.

Read Schedule means reading of the schedule message.

Start with n=x means sending of the MPH5_START_CBCH_READING primitive with the parameter n as defined before.

Schedule (x,y) means schedule message with begin slot number x and end slot number y.

Old CBCH (x) means repeated CBCH message with message identifier x.

New CBCH (x) means new CBCH message with message identifier x.

Unscheduled CBCH (x) means CBCH message with message identifier x which was not indicated by the schedule message.

In slot 18 a repetition of the old CBCH message with message identifier 3 is send.

| Slot No | CBCH activity | NO DRX | FIRST DRX | SECOND DRX |
| --- | --- | --- | --- | --- |
| 0 | Schedule (0/18) | - | Read Schedule, Start with n = 2 | Read Schedule, Start with n = 7 |
| 1 | NULL | - | - | - |
| 2 | Old CBCH (3) | - | Read CBCH, Start with n= 5 | - |
| 3 | NULL | - | - | - |
| 4 | New CBCH (5) | - | - | - |
| 5 | NULL | Start with n = 0 | - | - |
| 6 | NULL | - | - | - |
| 7 | Unscheduled CBCH (11) | Read CBCH, Start with n = 0 | Read CBCH, Start with n = 7 | Read CBCH, Start with n = 7 |
| 8 | NULL | - | - | - |
| 9 | NULL | - | - | - |
| 10 | Schedule (11 / 18) | Read Schedule, Start with n = 4 | - | - |
| 11 | NULL | - | - | - |
| 12 | NULL | - | - | - |
| 13 | NULL | - | - | - |
| 14 | New CBCH (7) | Read CBCH, Start with n = 5 | Read CBCH, Start with n = 3 | Read CBCH, Start with n = 3 |
| 15 | NULL | - | - | - |
| 16 | Old CBCH (8) | - | - | - |
| 17 | Unscheduled CBCH (14) | - | Start with n = 2 | Start with n = 2 |
| 18 | Old CBCH (3) Repeat | - | - |  |
| Next DRX mode |  | FIRST DRX | SECOND DRX | SECOND DRX |

The schedule message in slot 0 has the following layout.

01	Begin Slot Number 1
12	End Slot Number 18
12	New CBSMS Message Bitmap (NM 4 and NM 7)
04	NM 14
80	NM 17
00
00
00
80 05	New Message Indication: Message Identifier 5 (NM 4 = 1)
41	Free Message Slot, reading advised (NM 7 = 1)
80 07	New Message Indication: Message Identifier 7 (NM 14 = 1)
41	Free Message Slot, reading advised (NM 17 = 1)
40	Free Message Slot, reading optional (NM 1 = 0)
80 03	Old Message Indication: Message Identifier 3 (NM 2 = 0)
40	Free Message Slot, reading optional (NM 3 = 0)
40	Free Message Slot, reading optional (NM 5 = 0)
40	Free Message Slot, reading optional (NM 6 = 0)
40	Free Message Slot, reading optional (NM 8 = 0)
40	Free Message Slot, reading optional (NM 9 = 0)
40	Free Message Slot, reading optional (NM 10 = 0)
40	Free Message Slot, reading optional (NM 11 = 0)
40	Free Message Slot, reading optional (NM 12 = 0)
40	Free Message Slot, reading optional (NM 13 = 0)
40	Free Message Slot, reading optional (NM 15 = 0)
80 08	Old Message Indication: Message Identifier 8 (NM 16 = 0)
02	Old Message Indication: Repeat Message Identifier 3 from Slot 2 (NM 18 = 0)

The unscheduled schedule message from slot 10 has the same layout, only the begin slot number is set to 11.

### Null message

MMI                 TI++                     L1                 Network

|                    |                       |                     |

|                    |                  CB message                 |

|                    |                 (first block)               |

|                    *<============================================*

|                 (TI 1)                     |                     |

|                    |MPH5_START_CBCH_READING|                     |

|                    *======================>*                     |

|                    |                    (L1 1)                   |

|                    |                       |                     |

(TI 1)

TI ++ receives a null message. The first byte has the following structure:

| 8 | 7 6 | 5 | 4 3 2 1 |
| --- | --- | --- | --- |
| Spare 0 | LPD 0 1 | LB | Sequence Number |

The sequence number indicates "null message". The message is ignored.

(L1 1)

If the mobile is in FIRST or SECOND DRX mode CBCH reading is restarted in layer 1.

## Geographical Scope

The Geographical Scope (GS) indicates the geographical area over which the Message Code is unique, and the display mode. In particular, the Geographical Scope tells the mobile if the message is only cell wide (which means that any message if received in the next cell is regarded as "new"), or PLMN wide (which means that the Message Code and/or Update Number must change in the next cell for the message to be "new"), or location area wide (which means that a message with the same Message Code and Update Number may or may not be "new" in the next cell according to whether the next cell is in the same location area as the current cell).

Serial number (16 bit)

Octet 1		 Octet 2

| 7 6 | 5 4 3 2 1 0 7 6 5 4 | 3 2 1 0 |
| --- | --- | --- |
| GS | Message Code | Update Number |

The display mode indicates whether the message is supposed to be on the display all the time ("immediate") or only when the user wants to see it ("normal").The coding of this field that indicates the scope of the message is:

CODE DISPLAY MODE GEOGRAPHICAL SCOPE

00	Immediate Cell wide

01	Normal PLMN wide

10	Normal Location Area wide

11	Normal Cell wide

Immediate = default direct display;

Normal = default display under user interaction.

|  | GS=00 (Immediate) | GS=11 (Normal) | GS=10 |  | GS=01 |
| --- | --- | --- | --- | --- | --- |
|  | cell wide cbch message | cell wide cbch message | Location Area cbch message |  | PLMN cbch message |
| Cell wide has changed | X | X | O | O |  |
| Location Area wide has changed | X | X | X | O |  |
| PLMN has changed | X | X | X | X |  |
| "Special Testfeature" | O | O | O | O |  |

X => have to clear the message

O => just keep the message

The table shows the connection between the geographical area and the particular cbch messages. The changing of the geographical area is available from bcch sys_info_3 and the GS of the cbch message is standing in the serial number of each cbch messages.

The "Special Testfeature" when the pattern 0x100 for MCC and 0x10 for MNC does happen. It means just to keep the all cbch messages.

# Special Testfeatures

## TCH Loopback

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

*                             |                          |

(RR 1)                          |                          |

|     MPH_TCH_LOOP_REQ        |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             | OML1_CLOSE_TCH_LOOP_REQ  |

|                             *=========================>*

|                             |                       (PL 1)

|                             | OML1_CLOSE_TCH_LOOP_CNF  |

|                             *<=========================*

|                          (TI 2)                        |

*                             |                          |

(RR 2)                          |                          |

|     MPH_TCH_LOOP_REQ        |                          |

*============================>*                          |

|                          (TI 3)                        |

|                             |  OML1_OPEN_TCH_LOOP_REQ  |

|                             *=========================>*

|                             |                       (PL 2)

|                             |  OML1_OPEN_TCH_LOOP_CNF  |

|                             *<=========================*

|                          (TI 4)                        |

|                             |                          |

(RR 1)

RR receives a Close TCH Loop Command message.

(TI 1)

The parameter are forwarded to TI++ if a test sim is available.

(PL 1)

Closing of the TCH loop is configured for layer 1.

(TI 2)

A confirmation is given to TI++.

(RR 2)

RR receives an Open TCH Loop Command message.

(TI 3)

The parameter are forwarded to TI++ if a test sim is available.

(PL 2)

Opening of the TCH loop is configured for layer 1.

(TI 4)

A confirmation is given to TI++.

## DAI Testing

RR/DL                      TI-Add On                  TI-Layer 1

|                             |                          |

*                             |                          |

(RR 1)                          |                          |

|         MPH_DAI_REQ         |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             | OML1_START_DAI_TEST_REQ  |

|                             *=========================>*

|                             |                       (PL 1)

|                             | OML1_START_DAI_TEST_CNF  |

|                             *<=========================*

|                          (TI 2)                        |

*                             |                          |

(RR 2)                          |                          |

|          MPH_DAI_REQ        |                          |

*============================>*                          |

|                          (TI 3)                        |

|                             | OML1_STOP_DAI_TEST_REQ   |

|                             *=========================>*

|                             |                       (PL 2)

|                             | OML1_STOP_DAI_TEST_CNF   |

|                             *<=========================*

|                          (TI 4)                        |

|                             |                          |

(RR 1)

RR receives a Test Interface message.

(TI 1)

The parameter are forwarded to TI++ if a test sim is available.

(PL 1)

Starting of DAI testing is configured for layer 1.

(TI 2)

A confirmation is given to TI++.

(RR 2)

RR receives a Test Interface message indicating no further tests.

(TI 3)

The parameter are forwarded to TI++ if a test sim is available.

(PL 2)

Stopping of DAI testing is configured for layer 1.

(TI 4)

A confirmation is given to TI++.

# Audio Control

## Volume Initialization

MMI                        TI-Add On                  TI-Layer 1

|                             |                          |

|     MMI_AUDIO_INPUT_REQ     |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             *<VG_UlVolume>             |

|                          (TI 2)                        |

|     MMI_AUDIO_OUTPUT_REQ    |                          |

*============================>*                          |

|                          (TI 3)                        |

|                             *<VG_DlVolume>             |

|                          (TI 4)                        |

|                             |                          |

(TI 1)

After power-on the MMI reads the input volume and forwards it to TI++.

(TI 2)

TI++ forwards the uplink microphone volume to the hardware using the driver call VG_UlVolume.

(TI 3)

After power-on the MMI reads the out volume and forwards it to TI++.

(TI 4)

TI++ forwards the downlink speaker volume to the hardware using the driver call VG_DlVolume.

## Volume Change

MMI                        TI-Add On                  TI-Layer 1

|                             |                          |

|     MMI_AUDIO_INPUT_REQ     |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             *<VG_UlVolume>             |

|                          (TI 2)                        |

|     MMI_AUDIO_OUTPUT_REQ    |                          |

*============================>*                          |

|                          (TI 3)                        |

|                             *<VG_DlVolume>             |

|                          (TI 4)                        |

|                             |                          |

(TI 1)

If the input volume changes, MMI forwards the new value to TI++.

(TI 2)

TI++ forwards the uplink microphone volume to the hardware using the driver call VG_UlVolume.

(TI 3)

If the output volume changes, MMI forwards the new value to TI++.

(TI 4)

TI++ forwards the downlink speaker volume to the hardware using the driver call VG_DlVolume.

## Speech On

MMI                        TI-Add On                  TI-Layer 1

|                             |                          |

|     MMI_SPEECH_MODE_REQ     |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             *<VG_DlMute(OFF)>          |

|                          (TI 2)                        |

|                             |                          |

(TI 1)

MMI requests switching on of the speech channel.

(TI 2)

TI++ uses the driver call VG_DlMute with the parameter set to OFF to disable muting of the speech channel.

## Speech Off

MMI                        TI-Add On                  TI-Layer 1

|                             |                          |

|     MMI_SPEECH_MODE_REQ     |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             *<VG_DlMute(ON)>           |

|                          (TI 2)                        |

|                             |                          |

(TI 1)

MMI requests switching off of the speech channel.

(TI 2)

TI++ uses the driver call VG_DlMute with the parameter set to ON to enable muting of the speech channel.

## Keypad Feedback

MMI                        TI-Add On                  TI-Layer 1

|                             |                          |

|                             *<key_pressed>             |

|                          (TI 1)                        |

|        MMI_KEYPAD_IND       |                          |

*<============================*                          |

(MMI 1)                         |                          |

|                             |     MMI_KEYBEEP_REQ      |

|                             *=========================>*

|                             |                        (PL 1)

|                             |                          |

(TI 1)

A key is pressed on the keypad.

(MMI 1)

The pressed key is forwarded to MMI.

(PL 1)

If it is configured a keybeep is given to the user as a feedback to key pressing.

## Tones

For each possible tone a describing data structure is defined. This data structure describes whether it is a tone created by the buzzer or imposed to the audio channel.

If it is a buzzer tone the tone itself is described in a list with the entries command and length. The following commands are possible:

0	the buzzer is switched off for the time length in TDMA frames.

> 0	the buzzer is switched on with the value (equal to frequency) for the time length in TDMA frames.

FFFE	the buzzer is switched on continously.

FFFF	end of list

Example:	Subscriber Busy	Command	Length	Comment
		425	108	500 ms On
		0	108	500 ms Off
		FFFF	0	end of list

If the tone is imposed to the Audio Channel, the tone is described by up to two frequencies and optional a silence time. The tone is described in a list with the entries command_1, command_2 and length. The following commands are possible:

0	the tone is switched off for the time length in 20 ms units.

> 0	the tone is switched on with the value (corresponding to a frequency and amplitude) for the time length 
in 20 ms units.

FFFF	end of list

Example:	DTMF Tone 1	Command_1	Command_2	Length	Comment
		F_697	F_1209	13	260 ms On
		FFFF	FFFF	0	end of list

If the tone is requested by MMI the options volume and mode are possible. The parameter volume defines the volume of the tone. The parameter mode has the following options:

- single tone	the tone is created one time

- repeated tone	the tone is repeated

- silent tone	the tone is not created.

### Tones without Audio Channel

Tones without Audio Channel are produced with the buzzer. The following tones are defined according to 
GSM 2.40:

| Type | Tone | Silence |
| --- | --- | --- |
| Ringing *1) | 10 x (55 ms 800 Hz, 55 ms 900 Hz) | 4000 ms |
| Normal Clearing | Continous 425 Hz | - |
| Subscriber Busy | 500 ms 425 Hz | 500 ms |
| Congestion | 200 ms 425 Hz | 200 ms |
| Radio Path Acknowledgement *2) | 200 ms 425 Hz | - |
| Radio Path not available | 3 periods with 200 ms 425 Hz | 200 ms |
| Error/Special Information | tripple tone 330 ms 940 Hz 330 ms 1400 Hz 330 ms 1800 Hz | 1000 ms |

*1) Additional 15 custom specific ringing tones

*2) Optional

MMI                        TI-Add On                  TI-Layer 1

|                             |                          |

|     MMI_AUDIO_TONE_REQ      |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             *<BZ_Enable>               |

|                          (TI 2)                        |

|                             *<BZ_Tone>...              |

|                          (TI 3)                        |

|                             *<BZ_Disable>              |

|                          (TI 4)                        |

|                             |                          |

(TI 1)

MMI requests to generate a buzzer tone.

(TI 2)

The buzzer is enabled using the BS_Enable driver call.

(TI 3)

According to the tone description list the frequency of the tone is configured using the driver call BZ_Tone. A timer is started to supervise the length of the tone.

(TI 4)

After timeout the next entry of the tone description list is used or if the list is finished the buzzer is disabled with the driver call BZ_Disable.

### Tones with Audio Channel

Tones with Audio Channel are produced by layer1. The tone is mixed with the speech data. The following tones are defined according to GSM 2.40:

| Type | Tone | Silence |
| --- | --- | --- |
| Call Waiting Tone | 200 ms 425 Hz 200 ms 425 Hz 200 ms 425 Hz 200 ms 425 Hz | 600 ms 3000 ms 600 ms |
| DTMF ´1´ | 260 ms 697 Hz / 1209 Hz | - |
| DTMF ´2´ | 260 ms 697 Hz / 1336 Hz | - |
| DTMF ´3´ | 260 ms 697 Hz / 1477 Hz | - |
| DTMF ´a´ | 260 ms 697 Hz / 1633 Hz | - |
| DTMF ´4´ | 260 ms 770 Hz / 1209 Hz | - |
| DTMF ´5´ | 260 ms 770 Hz / 1336 Hz | - |
| DTMF ´6´ | 260 ms 770 Hz / 1477 Hz | - |
| DTMF ´c´ | 260 ms 770 Hz / 1633 Hz | - |
| DTMF ´7´ | 260 ms 852 Hz / 1209 Hz | - |
| DTMF ´8´ | 260 ms 852 Hz / 1336 Hz | - |
| DTMF ´9´ | 260 ms 852 Hz / 1477 Hz | - |
| DTMF ´e´ | 260 ms 852 Hz / 1633 Hz | - |
| DTMF ´*´ | 260 ms 941 Hz / 1209 Hz | - |
| DTMF ´0´ | 260 ms 941 Hz / 1336 Hz | - |
| DTMF ´#´ | 260 ms 941 Hz / 1477 Hz | - |
| DTMF ´^´ | 260 ms 941 Hz / 1633 Hz | - |

MMI                        TI-Add On                  TI-Layer 1

|                             |                          |

|     MMI_AUDIO_TONE_REQ      |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             |       MMI_TONE_REQ       |

|                             *=========================>*

|                             |                       (PL 1)

|                             |                          |

(TI 1)

MMI requests generation of a tone imposed to the audio channel.

(PL 1)

The parameters are set and forwarded to layer 1. If it is necessary a timer is started for the next tone generation according to the tone description list.

# Deactivation

RR                        TI-Add On                  TI-Layer 1

|                             |                          |

|     MPH_DEACTIVATE_REQ      |                          |

*============================>*                          |

|                          (TI 1)                        |

|                             |                          |

- Cell Selection or Idle Mode running

|                             |                          |

|                             |      MPH5_STOP_IDLE      |

|                             *=========================>*

|                             |                       (PL 1)

|                             |                          |

- Cell Reselection running

|                             |                          |

|                             |      MPH5_STOP_IDLE      |

|                             *=========================>*

|                             |                       (PL 2)

|                             |     MPH5_STOP_RESELECT   |

|                             *=========================>*

|                             |                       (PL 3)

|                             |                          |

|                             |                          |

- Connection Establishment running

|                             |                          |

|                             |   MPH5_STOP_ACCESS_LINK  |

|                             *=========================>*

|                             |                       (PL 4)

|                             |                          |

- Connection Establishment running

|                             |                          |

|                             |   MPH5_STOP_DEDICATED    |

|                             *=========================>*

|                             |                       (PL 5)

|                             |                          |

(TI 1)

After power-off the lower layer are deactivated.

(PL 1)

During cell selection or in idle mode the idle mode tasks in layer 1 are deactivated.

(PL 2)

During cell reselection the idle mode tasks are deactivated.

(PL 3)

The cell reselection process in layer 1 is stopped.

(PL 4)

During connection establishment sending of random bursts and downlink listen to CCCH is stopped.

(PL 5)

During dedicated mode the dedicated mode tasks are stopped.

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |