Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM Protocol Stack

Service Access Point

TRA

(Edit via File/Properties/Summary!)

| Document Number: | 8411.106.01.002 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.13 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2001-Mar-26 |
| Last changed: | 2003-Jun-16 by XGUTTEFE |
| File Name: | tra.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2001-Mar-26 | MG/SLO |  | 0.1 |  | 1 |
| 2002-Jun-10 | TVO |  | 0.2 |  | 2 |
| 2003-Jun-16 | XGUTTEFE |  | 0.3 | Draft |  |

Notes:

- Initial version

- Adaptation to use of DTI2 SAP

Table of Contents

1.1	References	5

1.2	Abbreviations	7

1.3	Terms	9

2	Overview	9

2.1	RA - Rate Adaptation	9

2.2	RLP - Radio Link Protocol	10

2.3	L2R - Layer 2 Relay Functionality	10

2.4	ACI - AT Command Interpreter	10

2.5	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	10

3	Constants	10

4	Primitives	11

4.1	TRA_ACTIVATE_REQ	11

4.2	TRA_ACTIVATE_CNF	11

4.3	TRA_DEACTIVATE_REQ	11

4.4	TRA_DEACTIVATE_CNF	12

4.5	TRA_DTI_REQ	12

4.6	TRA_DTI_CNF	13

4.7	TRA_DTI_IND	13

5	Parameters	13

5.1	Entity Name	14

5.2	Acknowledge Flag	14

5.3	Link Identifier	14

5.4	DTI direction	15

5.5	DTI connect	15

Appendices	16

A.	Acronyms	16

B.	Glossary	16

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	Rec. T.4 Standardisation of group 3 facsimile apparatus for document transmission;
(CCITT-T.4, 1984)

[2]	ITU-T Recommendation T.30; Series T: Terminal equipments and protocols for telematic services;
Procedures for document facsimile transmission in the general switched
telephone network;
(ITU-T.30, 1996)

[3]	ITU-T Recommendation T.31; Terminals for telematic services;
Asynchronous facsimile DCE control - service class 1
(ITU-T.31, 1995)

[4]	ITU-T Recommendation T.32; Terminals for telematic services;
Asynchronous facsimile DCE control - service class 2
(ITU-T.32, 1995)

[5]	Rec. T.35; Terminal equipment and protocols for telematic services;
Procedures for the allocation of CCITT definde codes for non-standard facilities;
(CCITT-T.35, 1991)

[6]	ITU-T Recommendation V.25 ter; Series V: data communication over the telephone network;
Interfaces and voiceband modems; Serial asynchronous automatic dialling and control
(ITU-T V.25 ter, 1997)

[7]	Rec. V.42 bis Data compression procedures for data circuit terminating equipment (DCE) using error correction procedures;
(CCITT-V.42 bis, 1990)

[8]	Rec. V.110 (Blue book, Vol. VIII, Fascicle VIII.1) Support of data terminal equipments (DTEs) with V-series type interfaces by an integrated services digital network (ISDN);
(CCITT-V.110, 1988)

[9]	European digital cellular telecommunications system (Phase 2);
GSM Public Land Mobile Network (PLMN) connection types;
(GSM 3.10, September 1994, version 4.3.1)

[10]	European digital cellular telecommunications system (Phase 2);
Technical realisation of facsimile group 3 transparent;
(GSM 3.45, September 1995, version 4.5.0)

[11]	Digital cellular telecommunications system (Phase 2);
Mobile radio interface layer 3 specification;
(GSM 4.08, November 1996, version 4.17.0)

[12]	European digital cellular telecommunications system (Phase 2);
Rate adaptation on the Mobile Station - Base Station System (MS - BSS) Interface;
(GSM 4.21, May 1995, version 4.6.0)

[13]	European digital cellular telecommunications system (Phase 2);
Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-service Switching Centre (BSS - MSC) interface
(GSM 4.22, September 1994, version 4.3.0)

[14]	European digital cellular telecommunications system (Phase 2);
Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-service Switching Centre (BSS - MSC) interface
(Amendment prA1 for GSM 4.22, version 4.3.0)
(GSM 4.22, March 1995, version 4.4.0)

[15]	European digital cellular telecommunications system (Phase 2);
General on Terminal Adaptation Functions (TAF) for Mobile Stations (MS);
(GSM 7.01, December 1995, version 4.10.0)

[16]	European digital cellular telecommunications system (Phase 2);
Terminal Adaptation Functions (TAF) for services using asynchronous bearer capabilities;
(GSM 7.02, September 1994, version 4.5.1)

[17]	European digital cellular telecommunications system (Phase 2);
Terminal Adaptation Functions (TAF) for services using synchronous bearer capabilities;
(GSM 7.03, September 1994, version 4.5.1)

[18]	Digital cellular telecommunications system (Phase 2);
Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Services (CBS);
(GSM 7.05, November 1996, version 4.8.0)

[19]	Digital cellular telecommunications system (Phase 2);
AT command set for GSM Mobile Equipment (ME)
(GSM 7.07, May 1996, version 4.1.0)

[20]	Digital cellular telecommunication system (Phase 2);
Mobile Station (MS) conformance specification;
Part 1: Conformance specification
(GSM 11.10-1, November 1996, version 4.17.0)

[21]	Digital cellular telecommunications system (Phase 2);
Mobile Station (MS) conformance specification;
Part 2: Protocol Implementation Conformance Statement (PICS)
proforma specification
(GSM 11.10-2, May 1996, version 4.15.0)

[22]	Digital cellular telecommunications system (Phase 2);
Mobile Station (MS) conformance specification;
Part 3: Layer 3 (L3) Abstract Test Suite (ATS)
(GSM 11.10-3, November 1996, version 4.17.0)

[23]	Proposal for Rate Adaptation implemented on a DSP;
(C. Bianconi, Texas Instruments, January 1998, version 1.0)

[24]	MCU-DSP Interfaces for Data Applications;
Specification S844
(C. Bianconi, Texas Instruments, March 1998, version 0.1)

[25]	Users Guide
6147.300.96.100; Condat GmbH

[26]	Service Access Point RA
8411.100.98.100; Condat GmbH

[27]	Service Access Point L2R
8411.102.98.100; Condat GmbH

[28]	Service Access Point ACI
8411.105.98.100; Condat GmbH

[29]	Message Sequence Charts TRA
8411.202.98.100; Condat GmbH

[30]	Message Sequence Charts ACI
8411.205.98.100; Condat GmbH

[31]	Test Specification TRA
8411.402.98.100; Condat GmbH

[32]	Test Specification ACI
8411.405.98.100; Condat GmbH

[33]	SDL Specification TRA
8411.502.98.100; Condat GmbH

[34]	SDL Specification ACI
8411.505.98.100; Condat GmbH

[35]	Technical Documentation TRA
8411.702.98.100; Condat GmbH

## Abbreviations

ACI	AT Command Interpreter
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BCS	Binary Coded Signals
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface
EOL	End Of Line

F	Final Bit
F&D	Fax and Data Protocol Stack
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HDLC	High level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LISR	Low level Interrupt Service Routine
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MSG	Message phase in the GSM 3.45 protocol

N(R)	Receive Number
N(S)	Send Number
NCC	National Colour Code
NECI	New Establishment Causes included

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Description Unit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

T.4	CCITT Standardisation for Document coding of Group 3 Facsimile Apparatus
TAP	Test Application Program
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visiting Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for fax and data transmission consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity. The entity, which is described in this document, is coloured grey in the following figure :

Figure 2-1: Architecture of the fax and data protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the fax and data protocol stack are:

## RA - Rate Adaptation

This entity performs an adaptation between an asynchronous or synchronous data stream with several bit rates on to the fixed bit rate used at the TCH. This is performed by the rate adaptation functions RA1’ and RA0 described in GSM 04.21.

## RLP - Radio Link Protocol

This entity provides a Layer 2 protocol for asynchronous reliable data transfer as specified in GSM 04.22. It includes error correction, sequence numbers and a mechanism for repeating corrupted and lost messages.

## L2R - Layer 2 Relay Functionality

The L2R provides relay functions in order to adapt the character-oriented data received from the TE via USART to the bit-oriented RLP protocol.

## ACI - AT Command Interpreter

The ACI is specified in GSM 07.07. It is responsible for call establishment via the GSM voice protocol stack and terminal adaptation for asynchronous transparent character-oriented data transmission. The ACI is able to receive AT commands and send the replies over the USART driver to a remote PC. This makes it possible to control the voice and data protocol stack from a remote application running on a PC. The ACI also provides a unique interface for an internal MMI in the MS.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the F&D. The driver has to be able to perform flow control.

# Constants

Definition:

| name | value | comment |
| --- | --- | --- |

History:

28-Mar-00	SLO	Initial

# Primitives

## TRA_ACTIVATE_REQ

Description:

The TRA_ACTIVATE_REQ is used to initialise the TRA entity. This is the first primitive, which should be passed to L2R after starting the TRA entity. It contains all parameters necessary for initialisation. Those parameters, which belong to the TRA layer are just passed on to the TRA entity.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| TRA_ACTIVATE_REQ | 0x80000019 | ACI -> L2R |

Elements:

| long name | short name | Ctrl | Ref | Type |
| --- | --- | --- | --- | --- |

History:

23 February 98	MG	Initial

## TRA_ACTIVATE_CNF

Description:

The TRA_ACTIVATE_CNF acknowledges the previous TRA_ACTIVATE_REQ. The acknowledge can be positive (ACK) or negative (NAK). It is negative, when the parameters of the TRA_ACTIVATE_REQ are invalid.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| TRA_ACTIVATE_CNF | 0x80004019 | TRA -> ACI |

Elements:

| long name | short name | ref | type |
| --- | --- | --- | --- |
| acknowledge flag | ack_flg | 6.2 | UBYTE |

History:

22 March 01	MG/SLO	Initial

## TRA_DEACTIVATE_REQ

Description:

The TRA_DEACTIVATE_REQ is used to deactivate the TRA entity. This primitive has no parameters.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| TRA_DEACTIVATE_REQ | 0x80010019 | ACI -> TRA |

Elements:

| long name | short name | ref | type |
| --- | --- | --- | --- |

History:

22 March 01	MG/SLO	Initial

## TRA_DEACTIVATE_CNF

Description:

The TRA_DEACTIVATE_CNF acknowledges the previous TRA_DEACTIVATE_REQ. The acknowledge is always positive.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| TRA_DEACTIVATE_CNF | 0x80014019 | TRA -> ACI |

Elements:

| long name | short name | ref | type |
| --- | --- | --- | --- |

History:

22 March 01	MG/SLO	Initial

## TRA_DTI_REQ

Description:

TRA_DTI_REQ is used to start data transfer between TRA and another entity via DTI. If this primitive is received the existing DTI connection will be removed and the new connection will be established.

Definition:

| Short name | ID | direction |
| --- | --- | --- |
| TRA_DTI_REQ | 0x80020019 | MMI -> TRA |

Elements:

| long name | short name | Ctrl | ref | type |
| --- | --- | --- | --- | --- |
| DTI connect | dti_conn |  | 6.5 | UBYTE |
| Link identifier of DTI | link_id |  | 6.3 | ULONG |
| Communication entity name | entity_name | [TRA_ENTITY_NAME_LEN] | 6.1 | UBYTE |
| DTI direction | dti_direction |  | 6.4 | UBYTE |

History:

17-Aug-2000	SLO	Initial

## TRA_DTI_CNF

Description:

TRA_DTI_CNF is the response primitive to TRA_DTI_REQ.

The primitive does not have parameters.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| TRA_DTI_CNF | 0x80024019 | UART -> MMI |

Elements:

| long name | short name | ref | type |
| --- | --- | --- | --- |
| DTI connect | dti_conn | 6.5 | UBYTE |
| Link identifier | link_id | 6.3 | ULONG |

History:

22 March 01	SLO	Initial

## TRA_DTI_IND

Description:

The TRA_DTI_IND is used by TRA to pass the control of the DTI back to the upper layer after detecting the escape sequence (standard: ‘+++’). The parameter link_id can be used to identify the DTI link.

Definition:

| short name | ID | Direction |
| --- | --- | --- |
| TRA_DTI_IND | 0x80034019 | TRA -> CTL |

Elements:

| long name | short name | ref | type |
| --- | --- | --- | --- |
| link identifier | link_id | 6.3 | ULONG |

History:

14 October 98	MG	Initial

22 March 01	MG	UART replaced by DTI

# Parameters

Parameters shall be part of the primitives described below and if applied the parameters shall contain the values specified here. These values are selected to correspond element values used in the air interface protocol.

## Entity Name

Description:

This parameter contains a string (zero terminated) which is the name of a neighbour entity, i.e. the entity handling a higher or lower protocol. The parameter is used to open the communication channel to this entity.

Definition:

| type | short name | Ctrl | Comment |
| --- | --- | --- | --- |
| UBYTE | entity_name | [TRA_ENTITY_NAME_LEN] | name of an neighbour entity |

Values:

| value | c-macro | Comment |
| --- | --- | --- |
| 6 | TRA_ENTITY_NAME_LEN | Max name length |

History:

22 March 01	MG/SLO	Initial

## Acknowledge Flag

Description:

The parameter identifies the acknowledge flag in response and confirm primitives.

Definition:

| type | short name | Comment |
| --- | --- | --- |
| UBYTE | ack_flg | acknowledge flag |

Values:

| value | c-macro | Comment |
| --- | --- | --- |
| 0 | TRA_ACK | acknowledged |
| 1 | TRA_NAK | not acknowledged |

History:

22 March 01	MG/SLO	Initial

## Link Identifier

Description:

This parameter is used to identify the affected channel in case of multiplexing.

Definition:

| type | short name | Comment |
| --- | --- | --- |
| ULONG | link_id | link identifier |

History:

22 March 01	MG/SLO	Initial

10 June 02	TVO	Changed to link id

## DTI direction

Description:

This parameter controls if TRA uses the DTI in the normal way or if the primitives are inverted. In the normal way data are sent as DTI_DATA_IND and received as DTI_DATA_REQ. In the inverted mode data are send as DTI_DATA_REQ and received as DTI_DATA_IND (relay functionality).

Definition:

| type | short name | Comment |
| --- | --- | --- |
| UBYTE | dti_direction | DTI direction |

Values:

| value | c-macro | Comment |
| --- | --- | --- |
| 0 | TRA_DTI_NORMAL | DTI used normally |
| 1 | TRA_DTI_INVERTED | DTI is inverted |

History:

22 March 01	MG	Initial

## DTI connect

Description:

The parameter defines, if TRA shall be connected via the DTI to the upper layer or not.

Definition:

| type | short name | Comment |
| --- | --- | --- |
| UBYTE | dti_conn | DTI connect |

Values:

| value | c-macro | Comment |
| --- | --- | --- |
| 0 | TRA_CONNECT_DTI | Connect DTI to TRA |
| 1 | TRA_DISCONNECT_DTI | Connect DTI to upper layer |

History:

14 October 98	MG	Added

… (Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |