| Date: | 1. Juni 2004 |
| --- | --- |
| ID: | 8462.101.03.001 |
| Status: | Planned |

| Author: | Texas Instruments Berlin AG Alt-Moabit 90a 10559 Berlin Germany |
| --- | --- |

| Condat Proprietary Information NDA - Confidential Do Not Copy |  |
| --- | --- |

G23-GPRS Protocol Stack

DCM

Service Access Point

GPRS

Table of Contents

0	Document Control	3

0.1	Document History	3

0.2	References, Abbreviations, Terms	3

1	Overview	4

2	Constants	5

3	Primitives	6

3.1	DCM_OPEN_CONN_REQ	6

3.2	DCM_OPEN_CONN_CNF	6

3.3	DCM_CLOSE_CONN_REQ	7

3.4	DCM_CLOSE_CONN_CNF	7

3.5	DCM_GET_CURRENT_CONN_REQ	7

3.6	DCM_GET_CURRENT_CONN_CNF	8

3.7	DCM_ERROR_IND	8

4	Parameters	9

4.1	API instance	9

4.2	Bearer handle	9

4.3	Requested Bearer type	9

4.4	Authentication type	10

4.5	Profile number	11

4.6	Connection Information	11

4.7	Result	12

4.8	Error cause	12

# Document Control

ã Copyright Condat AG, 2003

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Condat AG reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Condat AG. Condat AG accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a license agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Condat AG.

Condat AG

Alt Moabit 90a

10559 Berlin

Germany

Telephone:	+49.30.39 49 0

Fax:		+49.30.39 49 1300

Internet:	www.condat.de

## Document History

ID	Author	Date	Status
8462.101.03.001	STW	3 September, 2003	Planned

Initial version.

## References, Abbreviations, Terms

[C_7010.801]	7010.801, References and Vocabulary, Condat AG

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

# Constants

Definition:

| name | value | comment |
| --- | --- | --- |
| CDCM_APN_MAX_LEN | 100 | Maximum length of an access point name |
| CDCM_PHONE_NR_LEN | 84 | Maximum length of a CSD phone number |
| CDCM_PDP_MAX_LEN | 20 | Maximum length of PDP address |
| CDCM_GATE_MAX_LEN | 20 | Maximum length of gateway address |
| CDCM_USER_MAX_LEN | 25 | Maximum length of user name |
| CDCM_PASSWORD_MAX_LEN | 25 | Maximum length of password |
| CDCM_DNS_MAX_LEN | 20 | Maximum length of DNS addresses |

History:

03-Sep-2003	STW	Initial

# Primitives

## DCM_OPEN_CONN_REQ

Description:

The primitive allows the user to open a data connection depending on the selected bearer.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| DCM_OPEN_CONN_REQ | 0x8000401c | APP -> DCM |

Elements:

| long name | short name | Ctrl | ref | type |
| --- | --- | --- | --- | --- |
| Api instance | api_instance |  | 5.1 | U32 |
| Bearer type | bearer_select |  | 5.3 | S32 |
| profile number | profile_number |  | 5.5 | U16 |
| Paramater used for bearer connection | dcm_info_conn |  | 5.6 | STRUCT |

History:

03-Sep-2003	STW	Initial

## DCM_OPEN_CONN_CNF

Description:

The primitive informs user of result about DCM_OPEN_CONN_REQ.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| DCM_OPEN_CONN_CNF | 0x8000001c | DCM -> APP |

Elements:

| long name | short name | Ctrl | ref | type |
| --- | --- | --- | --- | --- |
| Result of the operation | result |  | 5.7 | S32 |
| Bearer handle | bearer_handle |  | 5.2 | U16 |

History:

03-Sep-2003	STW	Initial

## DCM_CLOSE_CONN_REQ

Description:

The primitive allows the user to close an opened data connection.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| DCM_CLOSE_CONN_REQ | 0x8001401c | APP -> DCM |

Elements:

| long name | short name | Ctrl | ref | type |
| --- | --- | --- | --- | --- |
| Api instance | api_instance |  | 5.1 | U32 |
| Beare handle | bearer_handle |  | 5.2 | U16 |

History:

03-Sep-2003	STW	Initial

## DCM_CLOSE_CONN_CNF

Description:

The primitive informs user of result about DCM_CLOSE_CONN_REQ.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| DCM_CLOSE_CONN_CNF | 0x8001001c | DCM -> APP |

Elements:

| long name | short name | Ctrl | ref | type |
| --- | --- | --- | --- | --- |
| Result of the operation | result |  | 5.7 | S32 |

History:

03-Sep-2003	STW	Initial

## DCM_GET_CURRENT_CONN_REQ

Description:

The primitive allows the user to get information upon current active connection.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| DCM_GET_CURRENT_CONN_REQ | 0x8002401c | APP -> DCM |

Elements:

| long name | short name | Ctrl | ref | type |
| --- | --- | --- | --- | --- |
| Api instance | api_instance |  | 5.1 | U32 |
| Beare handle | bearer_handle |  | 5.2 | U16 |

History:

03-Sep-2003	STW	Initial

## DCM_GET_CURRENT_CONN_CNF

Description:

The primitive informs user of result about DCM_GET_CURRENT_CONN_REQ and current active connection information.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| DCM_GET_CURRENT_CONN_CNF | 0x8002001c | DCM -> APP |

Elements:

| long name | short name | Ctrl | ref | type |
| --- | --- | --- | --- | --- |
| Result of the operation | result |  | 5.7 | S32 |
| Connection information | dcm_info_conn |  | 5.6 | STRUCT |

History:

03-Sep-2003	STW	Initial

## DCM_ERROR_IND

Description:

The primitive allows the user to receive error and event indications from DCM.

Definition:

| short name | ID | direction |
| --- | --- | --- |
| DCM_ERROR_IND | 0x8003001c | DCM -> APP |

Elements:

| long name | short name | Ctrl | ref | type |
| --- | --- | --- | --- | --- |
| Error value | dcm_err |  | 5.8 | S32 |
| Result of the operation | result |  | 5.7 | S32 |

History:

03-Sep-2003	STW	Initial

# Parameters

## API instance

Description:

The api_instance value returned by sock_api_initialize()..

Definition:

| type | short name | Comment |
| --- | --- | --- |
| U32 | api_instance | The api_instance value returned by sock_api_initialize(). |

History:

03-Sep-2003	STW	Initial

## Bearer handle

Description:

The bearer_handle value is returned with the primitive DCM_OPEN_CONN_REQ.

Definition:

| type | short name | Comment |
| --- | --- | --- |
| U16 | bearer_handle | Bearer handle |

History:

03-Sep-2003	STW	Initial

## Requested Bearer type

Description:

Bearer type used from DCFG or can take values as described below.

Definition:

| type | short name | Comment |
| --- | --- | --- |
| S32 | bearer_select | Bearer type |

Values:

| value | c-macro | Comment |
| --- | --- | --- |
| 1 | DCM_SOCK_BEARER_ANY | DCM will decide which connection type to be used |
| 2 | DCM_SOCK_BEARER_GPRS | Use a GPRS context as bearer, DCM will decide which GPRS settings to be used |
| 3 | DCM_SOCK_BEARER_GSM | Use GSM data connection as bearer DCM will decide which GSM settings to be used |
| 4 | DCM_SOCK_BEARER_USE_PROFILE | Use a specific data account for this connection |
| 5 | DCM_SOCK_BEARER_AS_SPECIFIED | Use the data account information which is which is delivered within this signal |

History:

03-Sep-2003	STW	Initial

## Authentication type

Description:

The authentication type defines the authentication protocol to be used.

Definition:

| type | short name | Comment |
| --- | --- | --- |
| U16 | authtype | Authentication type |

Values:

| value | c-macro | Comment |
| --- | --- | --- |
| 1 | DCM_SOCK_AUTH_PAP | PAP authentification protocol |
| 2 | DCM_SOCK_AUTH_CHAP | CHAP authentification protocol !!! NOT SUPPORTED |
| 3 | DCM_SOCK_AUTH_NO | No authentication |

History:

03-Sep-2003	STW	Initial

## Profile number

Description:

Number of the selected profile with a bearer selection of SOCK_BEARER_USE_PROFILE. Unused in other cases..

Definition:

| type | short name | Comment |
| --- | --- | --- |
| U16 | profile_number | Profile ID for bearer selection |

History:

06-Dec-2003	NI	Initial

## Connection Information

Description:

The parameter contains information about the current active connection.

Definition:

| type | short name | Comment |
| --- | --- | --- |
| STRUCT | dcm_info_conn | Connection information |

Elements:

| long name | short name | Ctrl | type |
| --- | --- | --- | --- |
| Systemwide bearer handle | bearer_handle |  | U16 |
| Comm handle of application | app_handle |  | U16 |
| Used bearer type | bearer_type |  | U8 |
| Flag for apn validation | apn_valid |  | U8 |
| Access Point Name | apn | [CDCM_APN_MAX_LEN + 1] | U8 |
| Flag for phone number validation | phone_number_valid |  | U8 |
| CSD dial up phone number | phone_number | [CDCM_PHONE_NR_LEN + 1] | U8 |
| Flag for user ID validation | user_id_valid |  | U8 |
| User ID | user_id | [CDCM_USER_MAX_LEN + 1] | U8 |
| Flag for password validation | password_valid |  | U8 |
| Password | password | [CDCM_PASSWORD_MAX_LEN + 1] | U8 |
| GPRS context ID | cid |  | U16 |
| Used IP address | ip_address |  | U32 |
| First domain name server | dns1 |  | U32 |
| Second domain name server | dns2 |  | U32 |
| Gateway address | gateway |  | U32 |
| Type of authentication | auth_type |  | U16 |
| Flag for data compression | data_compr |  | U8 |
| Flag for header compression | header_compr |  | U8 |
| GPRS precedence class | precedence |  | U16 |
| GPRS delay class | delay |  | U16 |
| GPRS reliability class | reliability |  | U16 |
| GPRS peak throughput | peak_throughput |  | U16 |
| GPRS mean throughput | mean_throughput |  | U16 |
| Flag for sharing requested bearer conn | shareable |  | U8 |

History:

03-Sep-2003	STW	Initial

## Result

Description:

The parameter contains the result of an operation requested by an application.

Definition:

| type | short name | Ctrl | Comment |
| --- | --- | --- | --- |
| S32 | result |  | Result of the operation |

Values:

| value | c-macro | Comment |
| --- | --- | --- |
| 0 | DCM_RET_OK | The action has been performed successfully. |
| 1 | DCM_RET_NOT_READY | The command can not be processed now. |
| 2 | DCM_RET_ALREADY_ACTIVATED | Connection refused because already actuvated. |
| 3 | DCM_RET_UNKNOWN_EVENT | Unknown receipt event. |
| 4 | DCM_RET_INVALID_PARAMETER | A parameter is wrong. |
| 5 | DCM_RET_CMD_PENDING | There is a pending command yet. |
| 6 | DCM_RET_PS_CONN_BROKEN | Loose of bearer connection. |

History:

03-Sep-2003	STW	Initial

## Error cause

Description:

The parameter contains errors received from PS (ETSI Spec 07.07) or values described below.

Definition:

| type | short name | Ctrl | Comment |
| --- | --- | --- | --- |
| S32 | dcm_err |  | Error value |

History:

03-Sep-2003	STW	Initial