Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document – Confidential

GSM Protocol Stack

Test Specification

EOTD-Controller

(Edit via File/Properties/Summary!)

| Document Number: | 8443.406.02.001 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.2 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2002-Sep-23 |
| Last changed: | 2003-May-19 by XGUTTEFE |
| File Name: | lc.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2002-Sep-23 | MNC |  | 0.1 |  | 1 |
| 2003-May-19 | XGUTTEFE |  | 0.2 | Draft |  |

Notes:

- Initial version

Table of Contents

1.1	References	4

1.2	Abbreviations	4

1.3	Terms	5

2	Overview	5

3	Parameters	6

4	TEST CASES	17

4.1	Internal Routing	17

4.1.1	LC000: Configure internal routing and PCO view	17

4.1.2	LC001: Configure internal routing and PCO view	18

4.2	Position Request	18

4.2.1	LC002: EOTD-Controller1: Position calculation (only for test)	18

4.2.2	LC003: EOTD-Controller1: (only for test)	20

4.2.3	LC004: EOTD-Controller1: Position calculation	22

4.2.4	LC005: EOTD-Controller1: RRLP request no measurement data	23

4.2.5	LC006: EOTD-Controller1: Position calculation	24

4.2.6	LC007: EOTD-Controller1: RRLP request location method not supported	26

4.2.7	LC008: EOTD-Controller1: RRLP request serving BTS not reference BTS	27

4.2.8	LC009: EOTD-Controller1: RRLP request handover during measurements.	28

4.2.9	LC010: EOTD-Controller1: Position calculation	28

4.2.10	LC011: EOTD-Controller1: RRLP request no measurement data	29

Appendices	32

A.	Acronyms	32

B.	Glossary	32

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[3GPP_03.71]	Digital cellular telecommunications system (Phase 2+), Location services (LCS), (Functional description) - Stage 2, (V8.5.0 Release 1999)

[3GPP_04.18]	3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; Mobile radio interface layer 3 specification, Radio Resource Control Protocol, (V8.13.0 Release 1999)

[3GPP_04.31]	3rd Generation Partnership Project; Technical Specification Group GSM EDGE Radio Access Network; Location Services (LCS); Mobile Station (MS) – Serving Mobile Location Centre (SMLC); Radio Resource LCS Protocol (RRLP), (V8.8.0 Release 1999)

[3GPP_24.007]	3rd Generation Partnership Project; Technical Specification Group Core Network; Mobile radio interface signalling layer 3; (V3.8.0 Release 1999)

[3GPP_24.030]	3rd Generation Partnership Project; Technical Specification Group Core Network; Location Services (LCS); Supplementary service operations - Stage 3), (V3.3.0 Release 1999)

[3GPP_22.071]	3rd Generation Partnership Project; Technical Specification Group services and System aspects; Location services (LCS); Service description; Stage 1 (V3.4.0 Release 1999)

http://www.fcc.gov/911/enhanced/	For further information of E911 requirements.

[X.209]		ITU-T Recommendation X.209: "Specification of Abstract Syntax Notation One (ASN.1)

[CU 0107]	CPS - GSM Cursor TM   Mobile Terminal Requirements, Full Specification – Issue 4.2

[CU 0115]	CPS - GSM Cursor TM   Mobile Terminal Requirements, MMI Specification – Issue 1.1

[CU 0126]	CPS - GSM Cursor TM   Mobile Terminal Requirements, AT Commands for Feature Control – Issue 3.3

[CU 0149]	CPS - GSM Cursor TM   Enhanced Observed Time Difference, (Software implementation project) – Issue 3.0

[CU 0305]	CPS - GSM Cursor TM   Mobile Terminal Requirements, Cursor TM   Task Bridge Description – Issue 1.3

[TI S922]		L1/RR interface for neighbour cells monitoring (Idle mode) and EOTD

[8353.507]	UELCS Requirements for Location Services in a UE – Requirement Analysis

[4711.005.02]	G23-GPRS Protocol Stack Release 1.4.0, Product Specification, May 2002

[4711.020.02]	G23-GPRS Protocol, E-OTD Lower Level Design LC, August 2002

[4711.021.02]	G23-GPRS Protocol, E-OTD Lower Level Design RR, August 2002

[TI 7010.801]	7010.801, References and Vocabulary, Texas Instruments

[C_7010.801]	7010.801, References and Vocabulary, Condat AG

## Abbreviations

ACI 	Application Control Interface (AT Commands)

AL	Application Layer

APDU	Application Protocol Data Unit

CBCH	Common Broadcast CHannel

CPS	Cambridge Positioning System

DTAP	Direct Transfer Application Part

E-OTD	Enhanced Observed Time Difference positioning method

LCS	Location Services

LC	Location Service handling Entity

LR	Location Request

NA-ESRD	North American Emergency Services Routing Digits

NA-ESRK	North American Emergency Services Routing Key

PS	Protocol Stack

RRLP	Radio Resource Location Service Protocol

SMLC	Serving Mobile Location Center

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

Figure 1: LC details

Figure 1 shows the details of the Cursor TM   task as provided by CPS. It consists of two parts, the Cursor TM   Algorithm and the Cursor TM   Bridge code. To have a more generic approach for further developments, the LC controller co-ordinates the positioning requests to the appropriate implementation, e.g. E-OTD or GPS. Texas Instruments will develop the LC controller and the RRLP. The Cursor TM   Bridge code is available in source code, representing the interface between the LC controller and the Cursor TM   Algorithm. It is used to exemplify the data handling between the modules but can be adapted to the Texas Instruments needs. The Cursor TM   Algorithm module is provided in object code, receiving the correlation data and BTS identifier and calculates precise OTD values. The Cursor TM   Algorithm remains property of CPS and will not be part of the Protocol Stack software release. The data handling is taking place inside RRLP or ACI (only during test phase 1 & 2 or for further implementations of assistance data). All point-to-point data, e.g. assistance data from the network and the location response from the MS, are handled by RRLP. General positioning data, e.g. assistance data to all mobiles in a cell, might be distributed via CBCH and is then transported via ACI to the LC. The interface between LC controller and RRLP is primitive based with functional character in order to run RRLP in another task context, e.g. RR.

# Parameters

/* DECLARATION  */

DECLARATION (BCD_NUMBER)	/* */

DECLARATION (MAX_BCD_LENGTH)	/* */

DECLARATION (SMS_TEXT)	/* */

DECLARATION (EOTD_SC_RES)	/* */

DECLARATION (EOTD_SC_RES1)	/* */

DECLARATION (EOTD_NC_RES_1)	/* */

DECLARATION (EOTD_NC_RES_2)	/* */

DECLARATION (EOTD_NC_RES_3)	/* */

DECLARATION (EOTD_NC_RES_4)	/* */

DECLARATION (EOTD_NC_RES_5)	/* */

DECLARATION (EOTD_NC_RES_6)	/* */

DECLARATION (EOTD_NC_RES_7)	/* */

DECLARATION (EOTD_NC_RES_8)	/* */

DECLARATION (EOTD_NC_RES_9)	/* */

DECLARATION (EOTD_NC_RES_10)	/* */

DECLARATION (EOTD_NC_RES_11)	/* */

DECLARATION (EOTD_NC_RES_12)	/* */

DECLARATION (ASSIST_DATA_1)	/* */

DECLARATION (ASSIST_DATA_2)	/* */

DECLARATION (ASSIST_DATA_3)	/* */

DECLARATION (ASSIST_DATA_4)	/* */

DECLARATION (ASSIST_DATA_5)	/* */

DECLARATION (ASSIST_DATA_6)	/* */

DECLARATION (ASSIST_DATA_7)	/* */

DECLARATION (ASSIST_DATA_8)	/* */

DECLARATION (ASSIST_DATA_9)	/* */

DECLARATION (ASSIST_DATA_10)	/* */

DECLARATION (ASSIST_DATA_11)	/* */

DECLARATION (ASSIST_DATA_12)	/* */

DECLARATION (ASSIST_DATA_13)	/* */

DECLARATION (ASSIST_DATA_14)	/* */

DECLARATION (ASSIST_DATA_15)	/* */

DECLARATION (TIMING_DATA)	/* */

DECLARATION (EXOR_NO)	/**/

DECLARATION (MNC)	/* */

DECLARATION (MCC)	/* */

DECLARATION (EOTD_NC_RES)	/* */

DECLARATION (ASSIST_DATA)	/* */

DECLARATION (REF_BTS_DATA)	/* */

/* Constants */

/* callreference */
SHORT	CALL_REF	0 		/* */

/* CursorDataValid */

BYTE	DATA_VALID	0 		/* TRUE */

/* CursorDataValid */

BYTE	DATA_VALID1	1 		/* TRUE */

/* sb_flag */

BYTE	SB_FLAG		0 		/* TRUE */

/* tav */

BYTE	TAV		 0xFF 		/* NO TAV */

/* bsic */

BYTE	BSIC		 0 		/* Base Station Code */

/* arfcn */

SHORT	ARFCN		 1 		/* Channal number */

/* rssi */

LONG	RSSI		 2 		/* RSSI */

/* time tag */

LONG	TIME_TAG	3		/* TIME_TAG */

/* fn */

LONG	FN		4 		/* TIME_TAG */

/* number length */
BYTE	NUMBER_LENGTH  10 		/* */

/* lac */
SHORT	LAC	5			/*Location Area Code */

/* cell_id */
SHORT	CELL_ID	6			/*Cell identification */

/* req_id */
SHORT	REQ_ID	0			/* */

/* req_id */
SHORT	REQ_id	1			/* */

/* req_id */
SHORT	REQ_ID_NOT	0		/* */

/* mfrm_offset*/

BYTE	MFRM_OFFSET	8	/* Offset in multiframes between the Scell and the

speceficed cell */

/* otd type*/

BYTE	OTD_TYPE	9	/* 0 = ROUGH_RTD */

/* uncertainty*/

BYTE	UNCERTAINTY	10	/* Max 2 Bit */

/* exp otd*/

SHORT	EXP_OTD 	11		/* Expected OTD from network */

/* rough rtd*/

SHORT	ROUGH_RTD	12		/* Rough RTD from network */

/* reference_relation */

BYTE	REFERENCE_RELATION	0		/* */

/* sb_flag */

BYTE	SB_FLAG		14 		/* TRUE */

/* std_resolution */

BYTE	STD_RESOLUTION		0		/* */

/* 	 */

BYTE	NUM_MEASUREMENT_SETS		0		/* */

/* num_reference_cells*/

BYTE	NUM_REFERENCE_CELLS	0		/* */

/* loc_method */

BYTE	LOC_METHOD		 1	/* */

/* pos_method */

BYTE	POS_METHOD		 1 		/* */

/* loc_method */

BYTE	LOC_MET		 0	/* */

/* pos_method */

BYTE	POS_MET		 0 		/* */

/* time_slot */

BYTE	TIME_SLOT	0		/* */

/* cause*/

BYTE	RRLC_ERROR_REQ_CAUSE		7		/*7 rrlp_metohod invalid  */

/* cause*/

BYTE	RRLC_ERROR_REQ_CAUSE_1	10		/* */

/* cause*/

BYTE	RRLC_ERROR_REQ_CAUSE_2	20		/* */

/* ta_correction_present */

BYTE	TA_CORRECTION_PRESENT		0		/* */

/* ta_correction */

LONG	TA_CORRECTION		0 		/* */

/* 	 */

BYTE	NUM_MEASUREMENTS		0		/* */

/* quality */

BYTE	QUALITY		0		/* */

/* timing_offset */

BYTE	TIMING_OFFSET		0		/* */

/* total_neigh_bts */

BYTE	TOTAL_NEIGH_BTS		0		/* */

/* bts_data */

BYTE	BTS_DATA		0		/* */

/* frame_number */

BYTE	FRAME_NUMBER		0		/* */

/* timing_advance */

LONG	TIMING_ADVANCE		0 		/* TIME_TAG */

/* t_mcc */

LONG	T_MCC		0 		/* mcc */

/* t_mnc */

LONG	T_MNC		0 		/* mnc */

BEGIN_PSTRUCT ("called_party_bcd_num", BCD_NUMBER) 	
	SET_COMP ("number_length", NUMBER_LENGTH)
	SET_COMP ("bcd", MAX_BCD_LENGTH)
ENDSTRUCT

/* Bcd */
BEGINARRAY_PART (MAX_BCD_LENGTH, 10) 	
	0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0x0A
ENDARRAY

BEGIN_PSTRUCT ("eotd_sc_res", EOTD_SC_RES)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO)	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

/* eotd_crosscor */
BEGINARRAY_PART (EXOR_NO, 18)			 
	0x01, 0x02, 0x03, 0x04, 0x05, 0x06,

0x07, 0x08, 0x09, 0x0A, 0x0B, 0x0C,

0x0D, 0x0E, 0x0F, 0x10, 0x11, 0x12
ENDARRAY

/* mnc */
BEGINARRAY_PART (MNC, 3) 			/* Mobile network code*/
	0x01, 0x02, 0x03
ENDARRAY

/* mcc */
BEGINARRAY_PART (MCC, 3) 			/* Mobile country code*/
	0x01, 0x02, 0x03
ENDARRAY

BEGIN_PSTRUCT ("eotd_sc_res1", EOTD_SC_RES1)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO)	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

/* sms text*/
BEGINARRAY_PART (SMS_TEXT, 0) 			/* */
	0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_1)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_2)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_3)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_4)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_5)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_6)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_7)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_8)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_9)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_10)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_11)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_12)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_13)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_14)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_PSTRUCT ("assist_data", ASSIST_DATA_15)

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("mfrm_offset",MFRM_OFFSET)	/* */

SET_COMP ("otd_type", OTD_TYPE)	/* */

SET_COMP ("exp_otd",EXP_OTD)		/* */

SET_COMP ("uncertainty",UNCERTAINTY)	/* */

SET_COMP ("rough_rtd",ROUGH_RTD)	/* */

ENDSTRUCT

BEGIN_STRUCT_ARRAY (ASSIST_DATA, 15)	/* ASSIST_DATA */

ASSIST_DATA_1,

ASSIST_DATA_2,

ASSIST_DATA_3,

ASSIST_DATA_4,

ASSIST_DATA_5,

ASSIST_DATA_6,

ASSIST_DATA_7,

ASSIST_DATA_8,

ASSIST_DATA_9,

ASSIST_DATA_10,

ASSIST_DATA_11,

ASSIST_DATA_12,

ASSIST_DATA_13,

ASSIST_DATA_14,

ASSIST_DATA_15

ENDARRAY

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_1)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */

SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/
SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_2) 	/* */

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_3)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_4)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_5)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_6)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_7)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_8)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_9)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_10)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_11)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_12)

SET_COMP ("sb_flag",SB_FLAG)		/* Data Valid Flag */
SET_COMP ("bsic",BSIC)			/* Base Station Identity Code*/

SET_COMP ("arfcn",ARFCN)		/* Channal Number*/

SET_COMP ("eotd_crosscor", EXOR_NO) 	/* */

SET_COMP ("d_eotd_nrj",RSSI)		/* RSSI*/

SET_COMP ("time_tag",TIME_TAG)		/* Time Tag */

ENDSTRUCT

BEGIN_STRUCT_ARRAY (EOTD_NC_RES, 12)	/* NC Measurements*/

EOTD_NC_RES_1,

EOTD_NC_RES_2,

EOTD_NC_RES_3,

EOTD_NC_RES_4,

EOTD_NC_RES_5,

EOTD_NC_RES_6,

EOTD_NC_RES_7,

EOTD_NC_RES_8,

EOTD_NC_RES_9,

EOTD_NC_RES_10,

EOTD_NC_RES_11,

EOTD_NC_RES_12

ENDARRAY

BEGIN_PSTRUCT ("timing_data", TIMING_DATA)

SET_COMP ("mcc",T_MCC)						/* */
SET_COMP ("mnc",T_MNC)						/* */

SET_COMP ("reference_relation",REFERENCE_RELATION)		/* */

SET_COMP ("time_slot",TIME_SLOT) 				/* */

SET_COMP ("std_resolution",STD_RESOLUTION)			/* */

SET_COMP ("num_measurement_sets",NUM_MEASUREMENT_SETS)	/* */

SET_COMP ("num_reference_cells",NUM_REFERENCE_CELLS)		/* */
SET_COMP ("ta_correction_present",TA_CORRECTION_PRESENT)	/* */

SET_COMP ("ta_correction",TA_CORRECTION)			/* */
SET_COMP ("ref_bts_data",REF_BTS_DATA)				/* */

ENDSTRUCT

BEGIN_PSTRUCT ("ref_bts_data",REF_BTS_DATA)

SET_COMP ("cell_id",CELL_ID)					/* */

SET_COMP ("lac",LAC)						/* */

SET_COMP ("quality",QUALITY)					/* */

SET_COMP ("num_measurements",NUM_MEASUREMENTS)		/* */

SET_COMP ("timing_offset",TIMING_OFFSET)			/* */

SET_COMP ("total_neigh_bts",TOTAL_NEIGH_BTS)			/* */

SET_COMP ("bts_data",BTS_DATA)					/* */

SET_COMP ("frame_number",FRAME_NUMBER)			/* */

SET_COMP ("timing_advance",TIMING_ADVANCE)			/* */

ENDSTRUCT

# TEST CASES

## Internal Routing

### LC000: Configure internal routing and PCO view

Description:	Internal routing is configured and the duplication of primitives for performing the component tests with TAP and PCO view is carried out

Preamble:	None

MMI		LC		RR 
	|		|		|
COMMAND (LC RESET)
COMMAND (MMI RESET)
COMMAND (RR RESET)
	|		|		|

COMMAND (TAP REDIRECT CLEAR)
COMMAND (LC REDIRECT CLEAR) 
COMMAND (TAP REDIRECT TAP LC)

COMMAND (LC REDIRECT MMI TAP)

COMMAND (LC REDIRECT RR TAP)
	 |	                                                             |		  |

Parametrization

- Primitive	Parameter	Value

History:	23.09.02	MNC	Initial

### LC001: Configure internal routing and PCO view

Description:	Internal routing is configured and the duplication of primitives for performing the component tests with TAP and PCO view is carried out

Preamble:	None

RRLP		LC		RR 
	|		|		|
COMMAND (LC RESET)
COMMAND (RRLP RESET)
COMMAND (RR RESET)
	|		|		|

COMMAND (TAP REDIRECT CLEAR)
COMMAND (LC REDIRECT CLEAR) 
COMMAND (TAP REDIRECT TAP LC)

COMMAND (LC REDIRECT RRLP TAP)

COMMAND (LC REDIRECT RR TAP)
	 |	                                                             |		  |

Parametrization

- Primitive	Parameter	Value

History:	16.10.02	MNC	Initial

## Position Request

### LC002: EOTD-Controller1: Position calculation (only for test)

Description:	LC make 2 position calculations.

Preamble:	LC000

	MMI				  LC				  RR
	 |				  |				  |
 (1)	 |				  |	RRLC_MEAS_IND		  |
	 |				  *<==========================*

(2)	 |	MNLC_SMS_MEAS_REQ	  |				  |	
	 *==========================>*				  |
 (3)	 |				  |	RRLC_MEAS_REQ		  |

|				 *===========================>*
 (4)	 |				  |	RRLC_MEAS_IND		  |
	 |				  *<==========================*
 (5)	 |	MNLC_SMS_MEAS_CNF	  |				  |
	 *<==========================*				  |

(6)  	 |	MNLC_SMS_MEAS_REQ	  |				  |	
	 *==========================>*				  |
 (7)	 |				  |	RRLC_MEAS_REQ		  |

|				 *===========================>*
 (8)	 |				  |	RRLC_MEAS_IND		  |
	 |				  *<==========================*
 (9)	 |	MNLC_SMS_MEAS_CNF	  |				  |
	 *<==========================*				  |
 (10)	 |				  |	RRLC_MEAS_IND		  |

|				  *<==========================*

|				  |				  |

Parametrization

- Primitive	Parameter	Value

- RRLC_MEAS_IND
		mcc	MCC
		mnc	MNC
		lac	LAC
		cell_id	CELL_ID
		eotd_mode	NOT_USED
		req_id	REQ_ID_NOT
		tav	TAV
		fn	FN
		eotd_sc_res	EOTD_SC_RES
		eotd_sc_res1	EOTD_SC_RES1
		eotd_nc_res	EOTD_NC_RES

- MNLC_SMS_MEAS_REQ
		reference 	CALL_REF
		called_party_bcd_num	BCD_NUMBER

- RRLC_MEAS_REQ
		req_id	REQ_ID

- arfcn	NOT_USED

- bsic	NOT_USED

- assist_data	NOT_USED

- RRLC_MEAS_IND
		mcc	MCC
		mnc	MNC
		lac	LAC
		cell_id	CELL_ID
		eotd_mode	NOT_USED
		req_id	REQ_ID
		tav	TAV
		fn	FN
		eotd_sc_res	EOTD_SC_RES
		eotd_sc_res1	EOTD_SC_RES1
		eotd_nc_res	EOTD_NC_RES

- MNLC_SMS_MEAS_CNF
		reference	CALL_REF
		data_valid	DATA_VALID
		sms_text	SMS_TEXT

- MNLC_SMS_MEAS_REQ
		reference 	CALL_REF
		called_party_bcd_num	BCD_NUMBER

- RRLC_MEAS_REQ
		req_id	REQ_id

- arfcn	NOT_USED

- bsic	NOT_USED

- assist_data	NOT_USED

- RRLC_MEAS_IND
		mcc	MCC
		mnc	MNC
		lac	LAC
		cell_id	CELL_ID
		eotd_mode	NOT_USED
		req_id	REQ_id
		tav	TAV
		fn	FN
		eotd_sc_res	EOTD_SC_RES
		eotd_sc_res1	EOTD_SC_RES1
		eotd_nc_res	EOTD_NC_RES

- MNLC_SMS_MEAS_CNF
		reference	CALL_REF
		data_valid	DATA_VALID
		sms_text	SMS_TEXT

- RRLC_MEAS_IND
		mcc	MCC
		mnc	MNC
		lac	LAC
		cell_id	CELL_ID
		eotd_mode	NOT_USED
		req_id	REQ_ID_NOT
		tav	TAV
		fn	FN
		eotd_sc_res	EOTD_SC_RES
		eotd_sc_res1	EOTD_SC_RES1
		eotd_nc_res	EOTD_NC_RES

History:	23.10.02	MNC	Initial

### LC003: EOTD-Controller1: (only for test)

Description:	Test if it is possible for LC to send and receive a signal.

Preamble:	LC000
	MMI				  LC				  RR
	 |				  |				  |
 (1)	 |	MNLC_SMS_MEAS_REQ	  |				  |
	 *==========================>*				  |

(2)	 |				  |	RRLC_MEAS_REQ		  |

|				 *===========================>*

(3)	 |				  |	RRLC_MEAS_IND		  |

|				 *<===========================*
 (4)	 |				  |	RRLC_MEAS_IND		  |
	 |				 *<===========================*
 (5)	 |	MNLC_SMS_MEAS_CNF	  |				  |
	 *<==========================*				  |
	 |				  |				  |

Parametrization

- Primitive	Parameter	Value

- MNLC_SMS_MEAS_REQ
		reference 	NOT_USED
		called_party_bcd_num	NOT_USED

- RRLC_MEAS_REQ
		req_id	REQ_ID

- arfcn	NOT_USED

- bsic	NOT_USED

- assist_data	NOT_USED

- RRLC_MEAS_IND
		mcc	MCC
		mnc	MNC
		lac	LAC
		cell_id	CELL_ID
		eotd_mode	NOT_USED
		req_id	REQ_ID_NOT
		tav	TAV
		fn	FN
		eotd_sc_res	EOTD_SC_RES
		eotd_sc_res1	EOTD_SC_RES1
		eotd_nc_res	EOTD_NC_RES

- RRLC_MEAS_IND
		mcc	MCC
		mnc	MNC
		lac	LAC
		cell_id	CELL_ID
		eotd_mode	NOT_USED
		req_id	REQ_ID
		tav	TAV
		fn	FN
		eotd_sc_res	EOTD_SC_RES
		eotd_sc_res1	EOTD_SC_RES1
		eotd_nc_res	EOTD_NC_RES

- MNLC_SMS_MEAS_CNF
		reference	NOT_USED
		data_valid	NOT_USED
		sms_text	NOT_USED

History:	23.09.02	MNC	Initial

### LC004: EOTD-Controller1: Position calculation

Description:	LC receive a MNLC_SMS_SMS_REQ and ask RR for new position data (RRLC_MEAS_REQ) LC receive new position data (RRLC_MEAS_IND) and send them to the algorithm. When the LC receive the calculated data from the algorithm it send the data to MMI in the MNLC_SMS_MEAS_CNF.

Preamble:	LC000

MMI				  LC				  RR
 (1)	 |	MNLC_SMS_MEAS_REQ	  |				  |

*==========================>*				  |
 (2)	 |				  |	RRLC_MEAS_REQ		  |
	 |				  *==========================>*
 (3)	 |				  |	RRLC_MEAS_IND		  |
	 |				  *<==========================*
 (4)	 |	MNLC_SMS_MEAS_CNF	  |				  |
	 *<==========================*				  |
	 |				  |				  |

Parametrization

- Primitive	Parameter	Value

- MNLC_SMS_MEAS_REQ
		reference 	CALL_REF
		called_party_bcd_num	BCD_NUMBER

- RRLC_MEAS_REQ
		req_id	REQ_ID

- arfcn	NOT_USED

- bsic	NOT_USED

- assist_data	NOT_USED

- RRLC_MEAS_IND
		mcc	MCC
		mnc	MNC
		lac	LAC
		cell_id	CELL_ID
		eotd_mode	NOT_USED
		req_id	REQ_ID
		tav	TAV
		fn	FN
		eotd_sc_res	EOTD_SC_RES
		eotd_sc_res1	EOTD_SC_RES1
		eotd_nc_res	EOTD_NC_RES

- MNLC_SMS_MEAS_CNF
		reference	CALL_REF
		data_valid	DATA_VALID
		sms_text	SMS_TEXT

History:	23.09.02	MNC	Initial

### LC005: EOTD-Controller1: RRLP request no measurement data

Description:	LC receive a RRLP_POS_IND and ask RR for new position data (RRLC_MEAS_REQ) LC receive new position data (RRLC_MEAS_IND) and send them to the algorithm. When the LC receive the calculated data from the algorithm it send the data to RRLP in the RRLP_POS_RES.

Preamble:	LC001

RRLP				  LC				  RR
 (1)	 |	RRLP_POS_IND		  |				  |

*==========================>*				  |
 (2)	 |				  |	RRLC_MEAS_REQ		  |
	 |				  *==========================>*
 (3)	 |				  |	RRLC_MEAS_IND		  |
	 |				  *<==========================*
 (4)	 |	RRLP_POS_RES		  |				  |
	 *<==========================*				  |
	 |				  |				  |

Parametrization

- Primitive	Parameter	Value

- RRLP_POS_IND
	assist_data	ASSIST_DATA
	loc_method	LOC_MET
	pos_method	POS_MET
	arfcn	ARFCN
	bsic	BSIC

- RRLC_MEAS_REQ
	req_id	REQ_ID
	arfcn	NOT_USED
	bsic	NOT_USED
	assist_data	NOT_USED

- RRLC_MEAS_IND
	mcc	MCC
	mnc	MNC
	lac	LAC
	cell_id	CELL_ID
	eotd_mode	NOT_USED
	req_id	REQ_ID
	tav	TAV
	fn	FN
	eotd_sc_res	EOTD_SC_RES
	eotd_sc_res1	EOTD_SC_RES1
	eotd_nc_res	EOTD_NC_RES

- RRLP_POS_RES
	timing_data	NOT_USED

History:	23.09.02	MNC	Initial

### LC006: EOTD-Controller1: Position calculation

Description:	LC has enough information by the three RRLC_MEAS_IND to make a position calculation then RRLP ask for a position.

Preamble:	LC001

	RRLP				  LC				  RR
	 |				  |				  |
 (1)	 |				  |	RRLC_MEAS_IND		  |
	 |				  *<==========================*

(2)	 |	RRLP_POS_IND	  	  |				  |	
	 *==========================>*				  |
 (3)	 |				  |	RRLC_MEAS_REQ		  |

|				 *===========================>*
 (4)	 |				  |	RRLC_MEAS_IND		  |
	 |				  *<==========================*
 (5)	 |	RRLP_POS_RES		  |				  |
	 *<==========================*				  |

(6)  	 |	RRLP_POS_IND		  |				  |	
	 *==========================>*				  |
 (7)	 |				  |	RRLC_MEAS_REQ		  |

|				 *===========================>*
 (8)	 |				  |	RRLC_MEAS_IND		  |
	 |				  *<==========================*
 (9)	 |	RRLP_POS_RES		  |				  |
	 *<==========================*				  |
 (10)	 |				  |	RRLC_MEAS_IND		  |

|				  *<==========================*

|				  |				  |

Parametrization

- Primitive	Parameter	Value

- RRLC_MEAS_IND
		mcc	MCC
		mnc	MNC
		lac	LAC
		cell_id	CELL_ID
		eotd_mode	NOT_USED
		req_id	REQ_ID_NOT
		tav	TAV
		fn	FN
		eotd_sc_res	EOTD_SC_RES
		eotd_sc_res1	EOTD_SC_RES1
		eotd_nc_res	EOTD_NC_RES

- RRLP_POS_IND
	assist_data	ASSIST_DATA
	loc_method	LOC_MET
	pos_method	POS_MET
	arfcn	ARFCN
	bsic	BSIC

- RRLC_MEAS_REQ
		req_id	REQ_ID

- arfcn	NOT_USED

- bsic	NOT_USED

- assist_data	NOT_USED

- RRLC_MEAS_IND
		mcc	MCC
		mnc	MNC
		lac	LAC
		cell_id	CELL_ID
		eotd_mode	NOT_USED
		req_id	REQ_ID
		tav	TAV
		fn	FN
		eotd_sc_res	EOTD_SC_RES
		eotd_sc_res1	EOTD_SC_RES1
		eotd_nc_res	EOTD_NC_RES

- RRLP_POS_RES
		timing_data	NOT_USED

- RRLP_POS_IND
	assist_data	ASSIST_DATA
	loc_method	LOC_MET
	pos_method	POS_MET
	arfcn	ARFCN
	bsic	BSIC

- RRLC_MEAS_REQ
		req_id	REQ_id

- arfcn	NOT_USED

- bsic	NOT_USED

- assist_data	NOT_USED

- RRLC_MEAS_IND
		mcc	MCC
		mnc	MNC
		lac	LAC
		cell_id	CELL_ID
		eotd_mode	NOT_USED
		req_id	REQ_id
		tav	TAV
		fn	FN
		eotd_sc_res	EOTD_SC_RES
		eotd_sc_res1	EOTD_SC_RES1
		eotd_nc_res	EOTD_NC_RES

- RRLP_POS_RES
		timing_data	NOT_USED

- RRLC_MEAS_IND
	mcc	MCC
	mnc	MNC
	lac	LAC
	cell_id	CELL_ID
	eotd_mode	NOT_USED
	req_id	REQ_ID_NOT
	tav	TAV
	fn	FN
	eotd_sc_res	EOTD_SC_RES
	eotd_sc_res1	EOTD_SC_RES1
	eotd_nc_res	EOTD_NC_RES

History:	23.09.02	MNC	Initial

### LC007: EOTD-Controller1: RRLP request location method not supported

Description:	The location method requested to be used by SMLC is not supported.

Preamble:	LC001

	RRLP				  LC				  RR
	 |				  |				  |
 (1)	 |	RRLP_POS_IND		  |				  |
	 *==========================>*				  |
 (2)	 |	RRLP_ERROR_REQ	  |				  |
	 *<==========================*				  |
	 |				  |				  |

Parametrization

- Primitive	Parameter	Value

- RRLP_POS_IND
	assist_data	ASSIST_DATA
	loc_method	LOC_METHOD
	pos_method	POS_METHOD
	arfcn	ARFCN
	bsic	BSIC

- RRLP_ERROR_REQ

- cause	RRLC_ERROR_REQ_CAUSE

History:	23.09.02	MNC	Initial

### LC008: EOTD-Controller1: RRLP request serving BTS not reference BTS

Description:	The reference BTS received in the assistance data differs from the serving BTS.

Preamble:	LC001

RRLP				  LC				  RR
 (1)	 |	RRLP_POS_IND		  |				  |

*==========================>*				  |
 (2)	 |				  |	RRLC_MEAS_REQ		  |
	 |				  *==========================>*
 (3)	 |				  |	RRLC_ERROR_IND		  |
	 |				  *<==========================*
 (4)	 |	RRLP_ERROR_REQ	  |				  |
	 *<==========================*				  |
	 |				  |				  |

Parametrization

- Primitive	Parameter	Value

- RRLP_POS_IND
	assist_data	ASSIST_DATA
	loc_method	LOC_MET
	pos_method	POS_MET
	arfcn	ARFCN
	bsic	BSIC

- RRLC_MEAS_REQ
	req_id	REQ_ID
	arfcn	NOT_USED
	bsic	NOT_USED
	assist_data	NOT_USED

- RRLC_ERROR_IND
	cause	RRLC_ERROR_REQ_CAUSE_1

- RRLP_ERROR_REQ
	cause	RRLC_ERROR_REQ_CAUSE_1

History:	23.09.02	MNC	Initial

### LC009: EOTD-Controller1: RRLP request handover during measurements.

Description:	A handover occurs before the measurements are completed.

Preamble:	LC001

RRLP				  LC				  RR
 (1)	 |	RRLP_POS_IND		  |				  |

*==========================>*				  |
 (2)	 |				  |	RRLC_MEAS_REQ		  |
	 |				  *==========================>*
 (3)	 |				  |	RRLC_ERROR_IND		  |
	 |				  *<==========================*
 (4)	 |	RRLP_ERROR_REQ	  |				  |
	 *<==========================*				  |
	 |				  |				  |

Parametrization

- Primitive	Parameter	Value

- RRLP_POS_IND
	assist_data	ASSIST_DATA
	loc_method	LOC_MET
	pos_method	POS_MET
	arfcn	ARFCN
	bsic	BSIC

- RRLC_MEAS_REQ
		req_id	REQ_ID
		arfcn	NOT_USED
		bsic	NOT_USED
		assist_data	NOT_USED

- RRLC_ERROR_IND
		cause	RRLC_ERROR_REQ_CAUSE_2

- RRLP_ERROR_REQ
		cause	RRLC_ERROR_REQ_CAUSE_2

History:	23.09.02	MNC	Initial

### LC010: EOTD-Controller1: Position calculation

Description:	LC receive a MNLC_SMS_SMS_REQ and ask RR for new position data (RRLC_MEAS_REQ) LC receive new position data (RRLC_MEAS_IND) and send them to the algorithm. When the LC receive the calculated data from the algorithm it send the data to MMI in the MNLC_SMS_MEAS_CNF.

Preamble:	LC000

MMI				  LC				  RR
 (1)	 |	MNLC_SMS_MEAS_REQ	  |				  |

*==========================>*				  |
 (2)	 |				  |	RRLC_MEAS_REQ		  |
	 |				  *==========================>*
 (3)	 |				  |	RRLC_ERROR_IND		  |
	 |				  *<==========================*
 (4)	 |	MNLC_SMS_MEAS_CNF	  |				  |
	 *<==========================*				  |
	 |				  |				  |

Parametrization

- Primitive	Parameter	Value

- MNLC_SMS_MEAS_REQ
		reference 	CALL_REF
		called_party_bcd_num	BCD_NUMBER

- RRLC_MEAS_REQ
		req_id	REQ_ID

- arfcn	NOT_USED

- bsic	NOT_USED

- assist_data	NOT_USED

- RRLC_ERROR_IND
		cause	RRLC_ERROR_REQ_CAUSE_2

- MNLC_SMS_MEAS_CNF
		reference	NOT_USED
		data_valid	DATA_VALID1
		sms_text	NOT_USED

History:	23.09.02	MNC	Initial

### LC011: EOTD-Controller1: RRLP request no measurement data

Description:	LC receive a RRLP_POS_IND and ask RR for new position data (RRLC_MEAS_REQ) LC receive new position data (RRLC_MEAS_IND) and send them to the algorithm. When the LC receive the calculated data from the algorithm it send the data to RRLP in the RRLP_POS_RES.

Preamble:	LC001

RRLP				  LC				  RR
 (1)	 |	RRLP_POS_IND		  |				  |

*==========================>*				  |
 (2)	 |				  |	RRLC_MEAS_REQ		  |
	 |				  *==========================>*

(3)	 |	RRLP_POS_IND		  |				  |

*==========================>*				  |

(4)	 |				  |	RRLC_MEAS_REQ		  |

|				  *==========================>*

(5)	 |				  |	RRLC_MEAS_IND		  |
	 |				  *<==========================*

(6)	 |				  |	RRLC_MEAS_IND		  |

|				  *<==========================*
 (7)	 |	RRLP_POS_RES		  |				  |
	 *<==========================*				  |
	 |				  |				  |

Parametrization

- Primitive	Parameter	Value

- RRLP_POS_IND
	assist_data	ASSIST_DATA
	loc_method	LOC_MET
	pos_method	POS_MET
	arfcn	ARFCN
	bsic	BSIC

- RRLC_MEAS_REQ
	req_id	REQ_ID
	arfcn	NOT_USED
	bsic	NOT_USED
	assist_data	NOT_USED

- RRLP_POS_IND
	assist_data	ASSIST_DATA
	loc_method	LOC_MET
	pos_method	POS_MET
	arfcn	ARFCN
	bsic	BSIC

- RRLC_MEAS_REQ
		req_id	REQ_id
		arfcn	NOT_USED
		bsic	NOT_USED
		assist_data	NOT_USED

- RRLC_MEAS_IND
	mcc	MCC
	mnc	MNC
	lac	LAC
	cell_id	CELL_ID
	eotd_mode	NOT_USED
	req_id	REQ_ID
	tav	TAV
	fn	FN
	eotd_sc_res	EOTD_SC_RES
	eotd_sc_res1	EOTD_SC_RES1
	eotd_nc_res	EOTD_NC_RES

- RRLC_MEAS_IND
	mcc	MCC
	mnc	MNC
	lac	LAC
	cell_id	CELL_ID
	eotd_mode	NOT_USED
	req_id	REQ_id
	tav	TAV
	fn	FN
	eotd_sc_res	EOTD_SC_RES
	eotd_sc_res1	EOTD_SC_RES1
	eotd_nc_res	EOTD_NC_RES

- RRLP_POS_RES
		timing_data	NOT_USED

History:	23.09.02	MNC	Initial

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |