Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM General Packet Radio Services

Test Specification

SM

(Edit via File/Properties/Summary!)

| Document Number: | 8441.405.99.006 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.8 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1999-Nov-11 |
| Last changed: | 2003-Jun-16 by XINTEGRA |
| File Name: | Sm.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1999-Nov-11 | HK |  | 0.1 |  | 1 |
| 2000-Jun-02 | HK |  | 0.2 |  | 2 |
| 2000-Oct-11 | HK |  | 0.3 |  | 3 |
| 2000-Oct-26 | HK |  | 0.4 |  | 4 |
| 2000-Nov-24 | HK |  | 0.5 |  | 5 |
| 2001-Jan-04 | HK |  | 0.6 |  | 6 |
| 2002-Apr-15 | HK |  | 0.7 |  | 7 |
| 2003-Jun-11 | XINTEGRA |  | 0.8 | Draft |  |

Notes:

- Initial version

- S3

- Fill ‘NOT_USED’ params.

- Routing Area Update.

- Improved ti handling.

- Add SMREG_PDP_ACT_REJ_RES.

- Test rest of T3380, T3390.

Table of Contents

1.1	References	8

1.2	Abbreviations	10

1.3	Terms	13

2.1	GRR (RLC/MAC) – Radio Link Control/Medium Access Control	14

2.2	LLC – Logical Link Control	14

2.3	GMM – GPRS Mobility Management	15

2.4	SM – Session Management	15

2.5	SNDCP - Subnetwork Dependant Convergence Protocol	15

2.6	GACI – GPRS Application Control Interface	15

2.7	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	15

2.8	TOM – Tunnelling of Messages	15

3.1	primitive elements	15

3.1.1	Fields	16

3.2	Declarations	18

3.2.1	Message Struct Declarations	18

3.2.2	Primitive Struct Declarations	19

3.2.3	Message Array Declarations	19

3.3	Message Arrays	19

3.4	Primitive Struct Arrays	20

3.5	Message Structs	21

3.6	Primitive Structs	24

4.1	Routing (internal) (SM100)	25

4.1.1	SM100: Setup Routing and PCO View for SM Tests	25

4.2	Identified PDP context activation	26

4.2.1	SM201: PDP context activation, MS initiated, requested NSAPI not yet used	26

4.2.2	SM202: PDP context activation, MS initiated, all possible 7 tis are used	27

4.2.3	SM203: PDP context activation, MS initiated, requested NSAPI already used	30

4.2.4	SM204: PDP context activation, MS initiated, no more transaction identifiers available	32

4.2.5	SM205: PDP context activation, MS initiated, GMM attach rejected	36

4.2.6	SM206: PDP context activation, MS initiated, GMM attach rejected, multiple contexts waiting	37

4.2.7	SM207: PDP context activation, MS initiated, new attach accepted by the network or MS already attached	38

4.2.8	SM208: PDP context activation, MS initiated, new attach accepted, multiple instances	39

4.2.9	SM209: PDP context activation, receiving ACTIVATE_PDP_CONTEXT_ACCEPT	41

4.2.10	SM210: PDP context activation, NSAPI now in use	42

4.2.11	SM211: PDP context activation, timer expiring up to four times	43

4.2.12	SM212: PDP context activation, timer expiring for the fifth time	44

4.2.13	SM213: PDP context activation, network initiated	46

4.2.14	SM214: PDP context activation, network initiated, ACI accepts	47

4.2.15	SM229: PDP context activation, network initiated, ACI accepts	48

4.2.16	SM216: Receiving ACTIVATE_PDP_CONTEXT_ACCEPT after collision	49

4.2.17	SM217: GMM establishment requested, timer expiring for the fifth time	50

4.2.18	SM218: PDP context activation, requested QoS “subscribed”	51

4.2.19	SM219: PDP context activation, minimum QoS “subscribed”	52

4.2.20	SM220: PDP context activation, MS initiated, new attach accepted by the network or MS already attached	53

4.2.21	SM221: PDP context activation, requested QoS “subscribed”receiving ACTIVATE_PDP_CONTEXT_ACCEPT	54

4.2.22	SM222: PDP context activation, network initiated, ACI does not accept	55

4.2.23	SM223: PDP context activation, NSAPI now in use	56

4.2.24	SM224: PDP context activation, receiving ACTIVATE_PDP_CONTEXT_REJECT	58

4.2.25	SM226: PDP context activation, MS initiated, header compresion	59

4.2.26	SM227: PDP context activation, MS initiated, new attach accepted by the network or MS already attached, header compression	60

4.2.27	SM228: PDP context activation, receiving ACTIVATE_PDP_CONTEXT_ACCEPT, header compression	61

4.2.28	SM230: PDP context activation, receiving ACTIVATE_PDP_CONTEXT_ACCEPT	62

4.2.29	SM231: PDP context activation, NSAPI now in use	63

4.2.30	SM232: PDP context activation, receiving ACTIVATE_PDP_CONTEXT_ACCEPT	64

4.2.31	SM233: PDP context activation, MS initiated, dynamic address allocation requested	65

4.2.32	SM234: PDP context activation, MS initiated, new attach accepted by the network or MS already attached	66

4.2.33	SM235: 2nd PDP context activation after dynamic one, MS initiated, APN not given in request: not comparable with possible MT activation	67

4.2.34	SM236: 2nd PDP context activation after dynamic one, PDP context activation, MS initiated, APN not given in request: not comparable with possible MT activation	68

4.2.35	SM237: 2nd PDP context activation after dynamic one, MS initiated comparable with possible MT activation	69

4.2.36	SM238: 2nd PDP context activation after dynamic one, PDP context activation, MS initiated, comparable with possible MT activation	70

4.2.37	SM239: PDP context activation, NSAPI now in use, header compression rejected by SNDCP	71

4.2.38	SM240: PDP context activation, timer expiring for the third time, the ACTIVATE_PDP_ACC, then deactivation and reactivation attempt with 5 expiries of T3380	72

4.3	Context deactivation	78

4.3.1	SM301: PDP context deactivation, MS initiated	78

4.3.2	SM302: PDP context deactivation, MS initiated, network accepts	79

4.3.3	SM303: PDP context deactivation, MS initiated, NSAPI released	80

4.3.4	SM304: PDP context deactivation, network initiated	81

4.3.5	SM305: PDP context deactivation, network initiated, NSAPI released	82

4.3.6	SM306: PDP context deactivation, timer expiring up to four times	83

4.3.7	SM307: PDP context deactivation, timer expiring for the fifth time	84

4.3.8	SM308: PDP context deactivation, network initiated	85

4.3.9	SM309: PDP context deactivation, MS initiated after SNSM_STATUS_REQ in state PDP_ACTIVE	87

4.3.10	SM310: PDP context deactivation after SNSM_STATUS_REQ, MS initiated, network accepts	88

4.3.11	SM311: PDP context deactivation, MS initiated, NSAPI released	89

4.3.12	SM312: PDP context deactivation after SNSM_STATUS_REQ, timer expiring up to four times	89

4.3.13	SM313: PDP context deactivation after SNSM_STATUS_REQ, timer expiring for the fifth time	90

4.3.14	SM314: PDP context deactivation, locally after GMMSM_RELEASE_IND in  state PDP_ACTIVE	92

4.3.15	SM315: SNSM_STATUS_REQ with cause “no peer response”	92

4.3.16	SM316: After SNSM_STATUS_REQ with cause "DM received" only ack contexts deactivated	93

4.3.17	SM317: GMMSM_RELEASE_IND in state CO_PDP_INACTIVE	94

4.3.18	SM318: GMMSM_RELEASE_IND in state CO_PDP_INACTIVE_WAIT_SNDCP	94

4.3.19	SM319: GMMSM_RELEASE_IND in state CO_AWAIT_GMMSM_ESTABLISH_CNF	95

4.3.20	SM320: GMMSM_RELEASE_IND in state CO_PDP_ACTIVE_PENDING	95

4.3.21	SM321: PDP context deactivation after GMMSM_RELEASE_IND in state CO_PDP_ACTIVE_AWAIT_SNDCP	96

4.3.22	SM322: GMMSM_RELEASE_IND in state CO_PDP_INACTIVE_PENDING	97

4.3.23	SM323: GMMSM_RELEASE_IND in state CO_PDP_STATUS_PENDING	97

4.3.24	SM324: PDP context deactivation, MS initiated after SNSM_STATUS_REQ in state PDP_ACTIVE_WAIT_SNDCP	98

4.3.25	SM325: PDP context deactivation, MS initiated and local	99

4.3.26	SM326: PDP context deactivation, MS initiated and local	99

4.3.27	SM327: PDP context deactivation local, MS initiated, NSAPI released	100

4.3.28	SM328: PDP context deactivation, MS initiated, NSAPI released after 5th expiry of T3390 and response from SNDCP	101

4.3.29	SM329: PDP context deactivation, MS initiated and local in intermediate states	102

4.3.30	SM330: PDP context deactivation, MS initiated and local	103

4.3.31	SM331: PDP context deactivation, timer expiring for the fifth time after deactivation (3 expiries and 1 DEACT_PDP_ACC) and reactivation	103

4.4	Context modification	108

4.4.1	SM401: PDP context modification initiated by the network, MS accepts QoS and SAPI	108

4.4.2	SM402: PDP context modification initiated by the network, MS does not accept QoS and SAPI	109

4.4.3	SM403: PDP context modification, network initiated, SNDCP finished requested changes	110

4.4.4	SM404: PDP context modification initiated by the network in state PDP_ACTIVE_AWAIT_SNDCP, MS accepts QoS and SAPI	111

4.4.5	SM405: PDP context modification in state PDP_ACTIVE_AWAIT_SNDCP, network initiated, SNDCP finished requested changes	112

4.5	Handling of unknown, unforeseen and erroneous protocol data	113

4.5.1	SM501: Message other than SM_STATUS with TI value “111”	113

4.5.2	SM502: Invalid transaction identifier value	114

4.5.3	SM503: Unknown message type	115

4.5.4	SM504: Unforeseen message type, not compatible with protocol state	116

4.5.5	SM505: Non_semantical mandatory element errors, not in list GSM 4.08, 8.5	117

4.5.6	SM506: Non_semantical mandatory element errors, DEACT_PDP_REQ	118

4.5.7	SM507: Non_semantical mandatory element errors, REQ_PDP_ACT	119

4.5.8	SM508: TI flag set to 1 with	120

4.5.9	SM509: ACTIVATE_PDP_ACC with invalid SAPI number	120

4.5.10	SM510: MOD_PDP_REQ with invalid SAPI number	121

4.5.11	SM511: Message not compatible with protocol state, ACTIVATE_PDP_ACC	122

4.5.12	SM512: Message not compatible with protocol state, ACTIVATE_PDP_REJ	124

4.5.13	SM513: Message not compatible with protocol state, DEACT_PDP_ACC	125

4.5.14	SM514: Message not compatible with protocol state, MOD_PDP_REQ	126

4.5.15	SM515: MOD_PDP_REQ for non existing context	127

4.5.16	SM516: Invalid ti, ACTIVATE_PDP_ACC	128

4.5.17	SM517: Invalid ti, ACTIVATE_PDP_REJ	129

4.5.18	SM518: Invalid ti, DEACT_PDP_ACC	130

4.5.19	SM519: Message not compatible with protocol state, MOD_PDP_REQ	131

4.6	Routing Area update	132

4.6.1	SM601: Receiving GMMSM_SEQUENCE_IND	132

4.6.2	SM602: Receiving SNSM_SEQUENCE_RES	133

4.6.3	SM603: Receiving GMMSM_SEQUENCE_IND with unused nsapi	133

4.6.4	SM604: Receiving SNSM_SEQUENCE_RES	134

4.6.5	SM605: Receiving GMMSM_SEQUENCE_IND with empty npdu number list	135

4.6.6	SM606: Receiving GMMSM_SEQUENCE_IND, npdu number list for invalid nsapi value	135

4.7	Config Primitives	136

4.7.1	SM701: Config prim NO MT	136

4.8	Collision of MT and MO activation	137

4.8.1	SM801: Collision of MT and MO activation, all MO activations use dynamic address allocation	137

4.8.2	SM802: Collision of MT and MO activation, 1 MO activations uses dynamic address allocation, 1 has not requested APN (i.e. not comparable)	138

4.8.3	SM803: Collision of MT and MO activation, 1 MO activations uses dynamic address allocation, 1 is  comparable and equal to MT activation	139

4.8.4	SM804: Collision of MT and MO activation, 1 MO activations uses dynamic address allocation, 1 is  comparable and different from MT activation	140

4.8.5	SM805: Collison of MO and MT context activation, pdp type is not IP 4	141

A.	Acronyms	143

B.	Glossary	143

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 05.02 version 8.0.0 Release 1999
Digital cellular telecommunications system (Phase 2+);
Multiplexing and multiple access on the radio path

[2]	GSM 04.60 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station (MS) - Base Station System (BSS) interface;
Radio Link Control/ Medium Access Control (RLC/MAC) protocol

[3]	GSM 04.08 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface layer 3 specification

[4]	GSM 03.64 version 6.1.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Overall description of the GPRS radio interface; Stage 2

[5]	GSM 03.60 version 6.3.1 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Service description; Stage 2

[6]	GSM 04.07 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface signalling layer 3; General aspects

[7]	GSM 04.64 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station - Serving GPRS Support Node (MS-SGSN)
Logical Link Control (LLC) layer specification

[8]	GSM 05.08 version 6.4.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem link control

[9]	GSM 05.10 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem synchronization

[10]	GSM 03.20 TS 100 929: July 1998 (GSM 03.20 version 6.0.1)
Security related network functions, ETSI

[11]	Draft GSM 03.22: August 1998 (GSM 03.22 version 6.1.0)
Functions related to Mobile Station (MS) in idle mode and group receive mode, ETSI

[12]	GSM 04.65 V6.3.0: Subnetwork Dependant Convergence Protocol

ETSI, March 1999

[13]	ITU-T V42bis ITU-T, Recommendation V.42 bis 1990

[14]	GSM 09.60 GPRS Tunneling Protocol (GTP) across the Gn and Gp Interface

[15]	RFC 1661 IETF STD 51 July 1994
The Point-to-Point Protocol (PPP)

[16]	RFC 1662 IETF STD 51 July 1994
PPP in HDLC-like Framing

[17]	RFC 1570 January 1994
PPP LCP Extensions

[18]	RFC 1989 August 1996
PPP Link Quality Monitoring

[19]	RFC 1332 May 1992
The PPP Internet Protocol Control Protocol (IPCP)

[20]	RFC 1877 December 1995
PPP IPCP Extensions for Name Server Addresses

[21]	RFC 2153 May 1997
PPP Vendor Extensions

[22]	RFC 1334 October 1992
PPP Authentication Protocols (for Password Authentication Protocol only)

[23]	RFC 1994 August 1996
PPP Challenge Handshake Authentication Protocol (CHAP)

[24]	TIA/EIA-136-370
Packet-Data Services – Enhanced General Packet Radio for TIA/EIA-136 (EGPRS-136) - Overview, Telecommunications Industry Association

[25]	TIA/EIA-136-376
Packet-Data Services – EGPRS-136 Mobility Management, Telecommunications Industry Association

[26]	TIA/EIA-136-972
Packet-Data Services – Stage 2 Description, Telecommunications Industry Association

## Abbreviations

ACI	Application Control Interface
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CCI	Compression and Ciphering Interface
CHAP	Challenge Handshake Authentication Protocol
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DCOMP	Identifier of the user data compression algorithm used for the N-DPU
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

E	Extension bit
EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GACI	GPRS Application Control Interface
GMM	GPRS Mobility Management
GP	Guard Period
GRR	GPRS RR
GSM	Global System for Mobile Communication

HDLC	High-level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
IP	Internet Protocol
IPCP	Internet Protocol Control Protocol 
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Ciphering Key

L	Length Indicator
LAI	Location Area Information
LCP 	Link Control Protocol 
LISR	Low level Interrupt Service Routine
LLC	Logical Link Control
LPD	Link Protocol Discriminator
LQM	Link Quality Monitoring

M	More bit used to indicate the last segment of N-DPU
MAC	Medium Access Control
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MT	Mobile Termination

N(R)	Receive Number
N(S)	Send Number
NC	Network Control
NCC	National Colour Code
NCP 	Network Control Protocol
NECI	New Establishment Causes included
N-PDU	Network Protocol Data Unit
NSAPI	Network Layer Service Access Point Identifier

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PACCH	Packet Associated Control Channel
PAP	Password Authentication Protocol
PBCCH	Packet BCCH
PCCCH	Packet CCCH
PCOMP	Identifier of the protocol control information compression algorithm used for the N-DPU
PDCH	Packet Data Channel
PDP	Packet Data Protocol e.g. IP or X.25
PDTCH	Packet Data Traffic Channel
PRACH	Packet RACH
PSI	Packet System Information
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Data Unit
PL	Physical Layer
PLMN	Public Land Mobile Network
PPC	Packet Physical Convergence
PPP 	Point-to-Point Protocol
PTP	Point to Point

QoS	Quality of Service

RACH	Random Access Channel
REJ	Reject Frame
RLC	Radio Link Control
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Stand alone Dedicated Control Channel
SDU	Service Data Unit
SGSN	Serving GPRS Support Node
SIM	Subscriber Identity Module
SM	Session Management
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SNDCP	Subnetwork Dependant Convergence Protocol
SNSM	SNDCP-SM
SS	Supplementary Services

TAP	Test Application Program
TBF	Temporary Block Flow
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TCP	Transmission Control Protocol
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TFI	Temporary Flow Identifier
TLLI	Temporary Logical Link Identifier
TMSI	Temporary Mobile Subscriber Identity
TOM	Tunnelling of Messages
TQI	Temporary Queuing Identifier

UA	Unnumbered Acknowledgement Frame
UART	Universal Asynchronous Receiver Transmitter
UI	Unnumbered Information Frame
USF	Uplink State Flag

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visited Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for GPRS consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity.

- Figure 2-1: Architecture of the GSM/GPRS protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the GPRS protocol stack are:

## GRR (RLC/MAC) – Radio Link Control/Medium Access Control

This layer contains two functions: The Radio Link Control function provides a radio-solution-dependent reliable link. The Medium Access Control function controls the access signalling (request and grant) procedures for the radio channel, and the mapping of LLC frames onto the GSM physical channel.

## LLC – Logical Link Control

The LLC entity provides multiple highly reliable logical links for asynchronous data transfer between the MS and the network. It supports variable-length information frames, acknowledged and unacknowledged data transfer, flow and sequence control, error detection and recovery, notification of unrecoverable errors, user identity confidentiality, and ciphering of user and signaling data.

## GMM – GPRS Mobility Management

The GMM entity provides procedures for the mobility of the MS, such as informing the network of its present location, and user identity confidentiality. It manages the GMM context (attach, detach, routing area updating), supports security functions such as authentication of user and MS, controls ciphering of data, and initiates the response to paging messages.

## SM – Session Management

The main function of the session management (SM) is to support PDP context handling of the user terminal. Session Management activates, modifies and deletes the contexts for packet data protocols (PDP). Session Management services are provided at the SMREG-SAP and the SNSM-SAP for anonymous and non-anonymous access. The non-anonymous and anonymous access procedures for PDP context activation and PDP context deactivation are available at the SMREG-SAP. In addition there exists a PDP context modification for non-anonymous PDP contexts.

## SNDCP - Subnetwork Dependant Convergence Protocol

- SNDCP carries out all functions related to transfer of Network layer Protocol Data Units (N-PDUs) over GPRS in a transparent way. SNDCP helps to improve channel efficiency by means of compression techniques. The set of protocol entities above SNDCP consists of commonly used network protocols. They all use the same SNDCP entity, which then performs multiplexing of data coming from different sources to be sent using the service provided by the LLC layer.

## GACI – GPRS Application Control Interface

The GACI is the GPRS extension of the ACI. It is specified in GSM 07.07 and 07.60. It is responsible for processing of the GPRS related AT Commands to setup, activate and deactivate the PDP context parameter. It also provides functionality for the interworking between GMM/SM/SNDCP and a packet oriented protocol like PPP.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the GPRS. The driver has to be able to perform flow control.

## TOM – Tunnelling of Messages

The TOM entity is present if and only if HS136 is supported (the feature flag FF_HS136 is enabled).

The main function of TOM is to tunnel non-GSM signalling messages between the MS and the SGSN. The only non-GSM signalling which is currently supported by TOM is for the EGPRS-136 system (according to TIA/EIA-136-376). Data transfer in both uplink and downlink direction is possible. Two different priorities (high, low) of signalling data transfer are supported. TOM uses the unacknowledged mode of LLC and the acknowledged mode of GRR (RLC/MAC).

# Parameters

/*

## primitive elements

*/

- BYTE MSID_0 0x00

- BYTE MSID_4 0x04

- BYTE NSAPI_2 0x2

- BYTE NSAPI_8 0x8

BYTE PDP_TYPE_IP_V_4 0x21

BYTE PDP_TYPE_IP_V_6 0x57

BYTE TI_0 0x0

BYTE TI_1 0x1

BYTE TI_8 0x8

BYTE TI_111 0x7

BYTE TI_9 0x9

BYTE TI_1_000 0x8

BYTE TI_NOT_USED 0xff

SHORT NSAPI_SET_2 0x4 /* bit mask with 2**2 set to 1*/

SHORT REC_NO_AA 0xaa

LONG DTI_LINKID_ABCD 0xabcdabcd

LONG TIMER_2_SEC 2

LONG DTI_NEIGHBOR_ABCD 0xabcdabcd

/*

### Fields

*/

- FIELD(ADDRESS_BUF_1)

0x01, 0x02, 0x03, 0x04

- ENDFIELD(ADDRESS_BUF_1,4)

- FIELD(ADDRESS_BUF_2)

0x00, 0x08, 0x01, 0x05

- ENDFIELD(ADDRESS_BUF_2,4)

/*

* Unexpected ACT_AA_PDP_ACC (also not correctly structured).

*/

- FIELD (ACT_AA_PDP_ACC_UNEX)

/* length */

0x81, 0,

/* offset */

0x18, 0,

/* */

0, 0, 0, 0x0a,

/* msg type */

0x81,

/* NOT_USED radio prio + half spare */

0,

/* SAPI */

0x3

/* mandatory IE ‘QoS’ missing! */

- ENDFIELD(ACT_AA_PDP_ACC_UNEX, 11)

/*

* MOD_PDP_REQ message with mandatory IE ‘QoS’ missing.

*/

- FIELD (MOD_PDP_REQ_IE_MISS)

/* length */

0x20, 0,

/* offset */

0x18, 0,

/* */

0, 0, 0, 0x0a,

/* msg type */

0x48,

/* NOT_USED radio prio + half spare */

0,

/* SAPI */

0x3

/* mandatory IE ‘QoS’ missing! */

- ENDFIELD(MOD_PDP_REQ_IE_MISS, 11)

/*

* DEACT_PDP_REQ message with mandatory IE ‘sm_cause’ missing.

*/

- FIELD (DEACT_PDP_REQ_IE_MISS)

/* length */

0x10, 0,

/* offset */

0x18, 0,

/* */

0, 0, 0, 0x8a,

/* msg type */

0x46

/* mandatory IE  ‘sm_cause’ missing! */

- ENDFIELD(DEACT_PDP_REQ_IE_MISS, 11)

- FIELD (PCO_123456789)

/* length */

0x48, 0,

/* offset */

0x08, 0x00,

/* offset ENCODE_OFFSET */

0x00,

/* pco data */

1, 2, 3, 4, 5, 6, 7, 8, 9

- ENDFIELD(PCO_123456789, 14)

- FIELD (PCO_987654321)

/* length */

0x48, 0,

/* offset */

0x0, 0x0,

/* pco data */

9, 8, 7, 6, 5, 4, 3, 2, 1

- ENDFIELD(PCO_987654321, 13)

/*

* REQ_PDP_ACT message with mandatory IE ‘address’ missing.

*/

- FIELD (REQ_PDP_ACT_IE_MISS)

/* length */

0x10, 0,

/* offset */

0x18, 0,

/* */

0, 0, 0, 0x0a,

/* msg type */

0x44

/* mandatory IE  ‘address’ missing! */

- ENDFIELD(REQ_PDP_ACT_IE_MISS, 9)

/*

## Declarations

*/

/*

### Message Struct Declarations

*/

- DECLARATION(ADDRESS_BUF_DYN)

- DECLARATION(AIR_ADDRESS_DYN)

- DECLARATION(AIR_ADDRESS_IP6)

- DECLARATION(AIR_ADDRESS_1)

- DECLARATION(AIR_ADDRESS_2)

- DECLARATION(AIR_ADD_INFO_DYN)

- DECLARATION(AIR_ADD_INFO_1)

- DECLARATION(AIR_ADD_INFO_2)

- DECLARATION(APN_EMPTY_FIELD)

- DECLARATION(APN_TEST_FIELD)

- DECLARATION(APN_1234_FIELD)

- DECLARATION(APN_WELL_FORMED_FIELD)

- DECLARATION(AIR_APN_TEST)

- DECLARATION(AIR_APN_1234)

- DECLARATION(AIR_APN_EMPTY)

- DECLARATION(AIR_APN_WELL_FORMED)

- DECLARATION(AIR_APN_MT)

- DECLARATION(AIR_PCO_MT)

- DECLARATION(AIR_PCO_987654321)

- DECLARATION(AIR_PCO_123456789)

- DECLARATION(AIR_RADIO_PRIO_1)

- DECLARATION(AIR_READY_TIMER_10)

- DECLARATION(AIR_SM_CAUSE_GGSN_ACT_REJ)

- DECLARATION(AIR_SM_CAUSE_UNK_PDP)

- DECLARATION(AIR_SM_CAUSE_LLC_OR_SNDCP)

- DECLARATION(AIR_SM_CAUSE_NET_FAIL)

- DECLARATION(AIR_SM_CAUSE_INSUF_RES)

- DECLARATION(AIR_SM_CAUSE_TI_INVAL)

- DECLARATION(AIR_SM_CAUSE_MSG_T_NON_X)

- DECLARATION(AIR_SM_CAUSE_MSG_T_NOT_COMP)

- DECLARATION(AIR_SM_CAUSE_MAND_INF_INVAL)

- DECLARATION(AIR_SM_CAUSE_REGULAR)

- DECLARATION(AIR_SM_CAUSE_UNSPEC)

- DECLARATION(AIR_SM_CAUSE_QOS_NOT_ACC)

- DECLARATION(AIR_NSAPI_2)

- DECLARATION(AIR_NSAPI_8)

- DECLARATION(AIR_NSAPI_5)

- DECLARATION(AIR_LLC_SAPI_3)

- DECLARATION(AIR_LLC_SAPI_17)

- DECLARATION(AIR_QOS_1)

- DECLARATION(AIR_QOS_ACK)

- DECLARATION(AIR_QOS_SUBSCRIBED)

- DECLARATION(AIR_QOS_UNACCEPTABLE)

- DECLARATION(LLC_SAPI_3)

- DECLARATION(SM_CAUSE_REGULAR)

- DECLARATION (NPDU_LIST_VAL_1)

/*

### Primitive Struct Declarations

*/

DECLARATION(SNSM_QOS_0)
DECLARATION(SNSM_QOS_ACK)
DECLARATION(SMREG_QOS_0)
DECLARATION(SMREG_QOS_ACK)
DECLARATION(SMREG_QOS_SUBSCRIBED)
DECLARATION(SMREG_APN_EMPTY) 
DECLARATION(SMREG_APN_TEST) 
DECLARATION(SMREG_APN_1234) 
DECLARATION(SMREG_APN_WELL_FORMED) 
DECLARATION(PDP_ADDRESS_DYN)
DECLARATION(PDP_ADDRESS_1)
DECLARATION(PDP_ADDRESS_2)

- DECLARATION (NPDU_LIST_OLD)

- DECLARATION (NPDU_LIST_EMPTY)

- DECLARATION (NPDU_LIST_INVALID)

- DECLARATION (NPDU_LIST_INVAL)

/*

### Message Array Declarations

*/

- DECLARATION(AIR_PCO_RA_123456789)

- DECLARATION(AIR_PCO_RA_987654321)

/*

## Message Arrays

*/

- BEGINARRAY (AIR_ADD_INFO_DYN, 0)

- 0xff

- ENDARRAY

- BEGINARRAY (AIR_ADD_INFO_1, 4)

0x1, 0x2, 0x3, 0x4

- ENDARRAY

- BEGINARRAY (AIR_ADD_INFO_2, 4)

0x0, 0x8, 0x1, 0x5

- ENDARRAY

BEGINARRAY (APN_1234_FIELD, 4)

0x1, 0x2, 0x3, 0x4

- ENDARRAY

BEGINARRAY (APN_TEST_FIELD, 12)

0x7, 0x74, 0x2d, 0x65, 0x2d, 0x73, 0x2d, 0x74, 0x3, 0x41, 0x50, 0x4e

- ENDARRAY

BEGINARRAY (APN_EMPTY_FIELD, 0)

0xff

- ENDARRAY

BEGINARRAY (APN_WELL_FORMED_FIELD, 17)

0x8, 0x69, 0x6e, 0x74, 0x65, 0x72, 0x6e, 0x65, 0x74,

0x4, 0x74, 0x2d, 0x64, 0x31, 0x2, 0x64, 0x65

- ENDARRAY

BEGINARRAY (AIR_PCO_RA_123456789, 9)

0x1, 0x2, 0x3, 0x4, 0x5, 0x6, 0x7, 0x8, 0x9

- ENDARRAY

BEGINARRAY (AIR_PCO_RA_987654321, 9)

0x9, 0x8, 0x7, 0x6, 0x5, 0x4, 0x3, 0x2, 0x1

- ENDARRAY

- BEGIN_STRUCT_ARRAY (NPDU_LIST_OLD, 1)

- NPDU_LIST_VAL_1

- ENDARRAY

- BEGIN_STRUCT_ARRAY (NPDU_LIST_INVALID, 1)

- NPDU_LIST_INVAL

- ENDARRAY

- BEGIN_STRUCT_ARRAY (NPDU_LIST_EMPTY, 0)

- 0xff

- ENDARRAY

BEGIN_MSTRUCT ("receive_n_pdu_number_list_val", NPDU_LIST_VAL_1)

SET_COMP ("nsapi", 0x07)

SET_COMP ("receive_n_pdu_number_val", 0xAA)

- ENDSTRUCT

BEGIN_MSTRUCT ("receive_n_pdu_number_list_val", NPDU_LIST_INVAL)

SET_COMP ("nsapi", 0x16)

SET_COMP ("receive_n_pdu_number_val", 0xAA)

- ENDSTRUCT

- /*

## Primitive Struct Arrays

- */

BEGINARRAY (ADDRESS_BUF_DYN, 0)

0xff

- ENDARRAY

/*

## Message Structs

*/

BEGIN_MSTRUCT("address", AIR_ADDRESS_1)

SET_COMP("pdp_type_org", IETF_ORG)

SET_COMP("pdp_type_no", PDP_TYPE_IP_V_4)

SET_COMP("add_info", AIR_ADD_INFO_1)

- ENDSTRUCT

BEGIN_MSTRUCT("address", AIR_ADDRESS_IP6)

SET_COMP("pdp_type_org", IETF_ORG)

SET_COMP("pdp_type_no", PDP_TYPE_IP_V_6)

SKIP_COMP("add_info")

- ENDSTRUCT

BEGIN_MSTRUCT("address", AIR_ADDRESS_DYN)

SET_COMP("pdp_type_org", IETF_ORG)

SET_COMP("pdp_type_no", PDP_TYPE_IP_V_4)

SET_COMP("add_info", AIR_ADD_INFO_DYN)

- ENDSTRUCT

BEGIN_MSTRUCT("address", AIR_ADDRESS_2)

SET_COMP("pdp_type_org", IETF_ORG)

SET_COMP("pdp_type_no", PDP_TYPE_IP_V_4)

SET_COMP("add_info", AIR_ADD_INFO_2)

- ENDSTRUCT

BEGIN_MSTRUCT("apn", AIR_APN_1234)

SET_COMP("apn_value", APN_1234_FIELD)

- ENDSTRUCT

BEGIN_MSTRUCT("apn", AIR_APN_TEST)

SET_COMP("apn_value", APN_TEST_FIELD)

- ENDSTRUCT

BEGIN_MSTRUCT("apn", AIR_APN_EMPTY)

SET_COMP("apn_value", APN_EMPTY_FIELD)

- ENDSTRUCT

BEGIN_MSTRUCT("apn", AIR_APN_WELL_FORMED)

SET_COMP("apn_value", APN_WELL_FORMED_FIELD)

- ENDSTRUCT

BEGIN_MSTRUCT("llc_sapi", AIR_LLC_SAPI_3)

SET_COMP("sapi", 0x3)

- ENDSTRUCT

BEGIN_MSTRUCT("llc_sapi", AIR_LLC_SAPI_17)

SET_COMP("sapi", 0x11)

- ENDSTRUCT

BEGIN_MSTRUCT("apn", AIR_APN_MT)

SKIP_COMP("apn_value")

- ENDSTRUCT

BEGIN_MSTRUCT("pco", AIR_PCO_987654321)

SET_COMP("pco_value", AIR_PCO_RA_987654321)

- ENDSTRUCT

BEGIN_MSTRUCT("pco", AIR_PCO_123456789)

SET_COMP("pco_value", AIR_PCO_RA_123456789)

- ENDSTRUCT

BEGIN_MSTRUCT("radio_prio", AIR_RADIO_PRIO_1)

SET_COMP("radio_prio_val", 1)

- ENDSTRUCT

BEGIN_MSTRUCT("ready_timer", AIR_READY_TIMER_10)

SET_COMP("timer_unit", TIMER_2_SEC)

SET_COMP("timer_value", 5)

- ENDSTRUCT

BEGIN_MSTRUCT("nsapi", AIR_NSAPI_2)

SET_COMP("nsapi_val", SMREG_NSAPI_2)

- ENDSTRUCT

BEGIN_MSTRUCT("nsapi", AIR_NSAPI_8)

SET_COMP("nsapi_val", SMREG_NSAPI_8)

- ENDSTRUCT

BEGIN_MSTRUCT("nsapi", AIR_NSAPI_5)

SET_COMP("nsapi_val", SMREG_NSAPI_5)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_GGSN_ACT_REJ)

SET_COMP("sm_cause_val", GGSN_ACT_REJ)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_UNK_PDP)

SET_COMP("sm_cause_val", UNK_PDP)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_LLC_OR_SNDCP)

SET_COMP("sm_cause_val", LLC_OR_SNDCP)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_NET_FAIL)

SET_COMP("sm_cause_val", NET_FAIL)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_INSUF_RES)

SET_COMP("sm_cause_val", INSUF_RES)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_TI_INVAL)

SET_COMP("sm_cause_val", TI_INVAL)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_MSG_T_NON_X)

SET_COMP("sm_cause_val", MSG_T_NON_X)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_MSG_T_NOT_COMP)

SET_COMP("sm_cause_val", MSG_T_NOT_COMP)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_MAND_INF_INVAL)

SET_COMP("sm_cause_val", MAND_INF_INVAL)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_REGULAR)

SET_COMP("sm_cause_val", REGULAR)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_UNSPEC)

SET_COMP("sm_cause_val", UNSPEC)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", AIR_SM_CAUSE_QOS_NOT_ACC)

SET_COMP("sm_cause_val", QOS_NOT_ACC)

- ENDSTRUCT

BEGIN_MSTRUCT("qos", AIR_QOS_1)

SET_COMP("delay", DELAY_1)

SET_COMP("reliability", RELY_3)

SET_COMP("peak", PEAK_1000)

SET_COMP("precedence", PRECED_HIGH)

SET_COMP("mean", MEAN_100)

- ENDSTRUCT

BEGIN_MSTRUCT("qos", AIR_QOS_ACK)

SET_COMP("delay", DELAY_1)

SET_COMP("reliability", RELY_2)

SET_COMP("peak", PEAK_1000)

SET_COMP("precedence", PRECED_HIGH)

SET_COMP("mean", MEAN_100)

- ENDSTRUCT

BEGIN_MSTRUCT("qos", AIR_QOS_SUBSCRIBED)

SET_COMP("delay", DELAY_SUB)

SET_COMP("reliability", RELY_SUB)

SET_COMP("peak", PEAK_SUB)

SET_COMP("precedence", PRECED_SUB)

SET_COMP("mean", MEAN_SUB)

ENDSTRUCT

BEGIN_MSTRUCT("qos", AIR_QOS_UNACCEPTABLE)

SET_COMP("delay", DELAY_4)

SET_COMP("reliability", RELY_3)

SET_COMP("peak", PEAK_1000)

SET_COMP("precedence", PRECED_HIGH)

SET_COMP("mean", MEAN_100)

- ENDSTRUCT

BEGIN_MSTRUCT("sm_cause", SM_CAUSE_REGULAR)

SET_COMP("sm_cause_val", REGULAR)

- ENDSTRUCT

BEGIN_MSTRUCT("llc_sapi", LLC_SAPI_3)

SET_COMP("sapi", SAPI_3)

- ENDSTRUCT

/*

## Primitive Structs

*/

BEGIN_PSTRUCT("snsm_qos", SNSM_QOS_0)

SET_COMP("delay", SNSM_DELAY_1)

SET_COMP("relclass", SNSM_RLC_PROT)

SET_COMP("peak", SNSM_PEAK_1K)

SET_COMP("preced", SNSM_PRIO_HIGH)

SET_COMP("mean", SNSM_MEAN_100)

- ENDSTRUCT

BEGIN_PSTRUCT("snsm_qos", SNSM_QOS_ACK)

SET_COMP("delay", SNSM_DELAY_1)

SET_COMP("relclass", SNSM_LLC_RLC_PROT)

SET_COMP("peak", SNSM_PEAK_1K)

SET_COMP("preced", SNSM_PRIO_HIGH)

SET_COMP("mean", SNSM_MEAN_100)

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_apn", SMREG_APN_1234)

SET_COMP("buffer", APN_1234_FIELD) /* this is not a valid apn */

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_apn", SMREG_APN_TEST)

SET_COMP("buffer", APN_TEST_FIELD) /* this is a valid apn */

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_apn", SMREG_APN_EMPTY)

SET_COMP("buffer", APN_EMPTY_FIELD) /* no APN requested */

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_apn", SMREG_APN_WELL_FORMED)

SET_COMP("buffer", APN_WELL_FORMED_FIELD) /* this is a valid apn */

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_qos", SMREG_QOS_0)

SET_COMP("delay", SMREG_DELAY_1)

SET_COMP("relclass", SMREG_RLC_PROT)

SET_COMP("peak", SMREG_PEAK_1K)

SET_COMP("preced", SMREG_PRIO_HIGH)

SET_COMP("mean", SMREG_MEAN_100)

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_qos", SMREG_QOS_ACK)

SET_COMP("delay", SMREG_DELAY_1)

SET_COMP("relclass", SMREG_LLC_RLC_PROT)

SET_COMP("peak", SMREG_PEAK_1K)

SET_COMP("preced", SMREG_PRIO_HIGH)

SET_COMP("mean", SMREG_MEAN_100)

- ENDSTRUCT

BEGIN_PSTRUCT("smreg_qos", SMREG_QOS_SUBSCRIBED)

SET_COMP("delay", SNSM_DELAY_SUB)

SET_COMP("relclass", SMREG_RELCLASS_SUB)

SET_COMP("peak", SNSM_PEAK_SUB)

SET_COMP("preced", SNSM_PRECED_SUB)

SET_COMP("mean", SNSM_MEAN_SUB)

ENDSTRUCT

BEGIN_PSTRUCT("pdp_address", PDP_ADDRESS_DYN)

SET_COMP("buff", ADDRESS_BUF_DYN)

- ENDSTRUCT

BEGIN_PSTRUCT("pdp_address", PDP_ADDRESS_1)

SET_COMP("buff", ADDRESS_BUF_1)

- ENDSTRUCT

BEGIN_PSTRUCT("pdp_address", PDP_ADDRESS_2)

SET_COMP("buff", ADDRESS_BUF_2)

- ENDSTRUCT

# TEST CASES

## Routing (internal) (SM100)

### SM100: Setup Routing and PCO View for SM Tests

Description:

Routings for SM tests are set.

Preamble:

None

SNDCP/GACI                      SM                         GMM

|                           |                           |

- COMMAND (TAP RESET)

- COMMAND (MMI RESET)

- COMMAND (SND RESET)

- COMMAND (SM RESET)

- COMMAND (GMM RESET)

|                           |                           |

- COMMAND (TAP REDIRECT CLEAR)

- COMMAND (MMI REDIRECT CLEAR)

- COMMAND (SND REDIRECT CLEAR)

- COMMAND (SM REDIRECT CLEAR)

- COMMAND (GMM REDIRECT CLEAR)

|                           |                           |

- COMMAND (MMI REDIRECT SM NULL)

- COMMAND (SND REDIRECT SM NULL)

- COMMAND (GMM REDIRECT SM NULL)

|                           |                           |

- COMMAND (SM REDIRECT MMI TAP)

- COMMAND (SM REDIRECT SND TAP)

- COMMAND (SM REDIRECT GMM TAP)

|                           |                           |

- COMMAND (TAP REDIRECT TAP SM)

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

History:

06-Nov-1999	HK	Initial

## Identified PDP context activation

### SM201: PDP context activation, MS initiated, requested NSAPI not yet used

Description:

(MSC 3.1.1)
(SM 1)

SM is in state PDP_INACTIVE <R.SM.PDP_INAC.M.001>, <R.SM.MS_CONT.M.001>. The requested NSAPI is not yet used <R.SM.MS_ACT_S.M.005>. ACI sends an SMREG_PDP_ACTIVATE_REQ to initiate a PDP context activation.

(LLC 1)

SM starts timer T3380 <R.SM.MS_ACT_S.M.003>. SM sends a GMMSM_ESTABLISH_REQ to force a setup of GMM connection <R.SM.MS_CONT.M.002>. (If the MS has already been attached GMM will just send back a confirmation.)

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM202: PDP context activation, MS initiated, all possible 7 tis are used

Description:

(no MSC, multiplex issue)
(SM 1)

SM is in state PDP_INACTIVE <R.SM.PDP_INAC.M.001>. The user tries to activate all possible 7 MS initiated contexts.

(ACI 1)

SM sends a GMMSM_ESTABLISH_REQ for each attempt.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(3)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(4)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(5)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(6)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(7)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(8)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(9)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(10) |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(11) | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(12) |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(13) | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(14) |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_0
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_1
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_3
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_4
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_6
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM203: PDP context activation, MS initiated, requested NSAPI already used

Description:

(MSC 3.1.2)
(SM 1)

SM is in state PDP_INACTIVE <R.SM.PDP_INAC.M.001>. The requested NSAPI is already in use.

(ACI 1)

SM sends an SMREG_PDP_ACTIVATE_REJ to GMM with cause SMREG_RC_NSAPI_ALREADY_USED <R.SM.MS_ACT_S.M.005>.

.

Preamble:

- SM201

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  | SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_NSAPI_ALREADY_USED
	smreg_nsapi	SMREG_NSAPI_2

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM204: PDP context activation, MS initiated, no more transaction identifiers available

Description:

(MSC 3.1.2)
(SM 1)

SM is in state PDP_INACTIVE <R.SM.PDP_INAC.M.001>. All available tis are already in use.

(ACI 1)

SM sends an SMREG_PDP_ACTIVATE_REJ to GMM with cause SMREG_RC_INSUF_RES <R.SM.MS_ACT_S.M.005>.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(3)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(4)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(5)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(6)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(7)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(8)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(9)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(10) |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(11) | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(12) |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(13) | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(14) |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(15) | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(16) | SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_0
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_1
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_3
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_4
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_6
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_7
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_INSUF_RES
	smreg_nsapi	SMREG_NSAPI_7

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM205: PDP context activation, MS initiated, GMM attach rejected

Description:

(not mentioned in MSCs)
(SM 1)

A pdp context is in state CO_AWAIT_GMMSM_ESTABLISH_CNF. Instead of an expected GMMSM_ESTABLISH_CNF a GMMSM_ESTABLISH_REJ is received. The NSAPI of the pending context has number 0.

(ACI 1)

SM sends an SMREG_PDP_ACTIVATE_REJ to GMM with the cause given in the reject primitive and NSAPI number 0.

.

Preamble:

- SM201

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_ESTABLISH_REJ       |

|                           *<==========================*
(2)  | SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_ESTABLISH_REJ
	sm_cause	SM_ERRCS_NET_FAIL

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SM_ERRCS_NET_FAIL
	smreg_nsapi	SMREG_NSAPI_2

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM206: PDP context activation, MS initiated, GMM attach rejected, multiple contexts waiting

Description:

(not mentioned in MSCs)
(SM 1)

A pdp context is in state CO_AWAIT_GMMSM_ESTABLISH_CNF. Instead of an expected GMMSM_ESTABLISH_CNF a GMMSM_ESTABLISH_REJ is received. The NSAPIs of the pending contexts have numbers 0...7.

(ACI 1...)

SM sends an SMREG_PDP_ACTIVATE_REJ to GMM with the cause given in the reject primitive and NSAPI numbers for all affected contexts.

.

Preamble:

- SM202

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_ESTABLISH_REJ       |

|                           *<==========================*
(2)  | SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

(3)  | SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

(4)  | SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

(5)  | SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

(6)  | SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

(7)  | SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

(8)  | SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_ESTABLISH_REJ
	sm_cause	SM_ERRCS_NET_FAIL

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SM_ERRCS_NET_FAIL
	smreg_nsapi	SMREG_NSAPI_0

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SM_ERRCS_NET_FAIL
	smreg_nsapi	SMREG_NSAPI_1

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SM_ERRCS_NET_FAIL
	smreg_nsapi	SMREG_NSAPI_2

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SM_ERRCS_NET_FAIL
	smreg_nsapi	SMREG_NSAPI_3

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SM_ERRCS_NET_FAIL
	smreg_nsapi	SMREG_NSAPI_4

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SM_ERRCS_NET_FAIL
	smreg_nsapi	SMREG_NSAPI_5

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SM_ERRCS_NET_FAIL
	smreg_nsapi	SMREG_NSAPI_6

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM207: PDP context activation, MS initiated, new attach accepted by the network or MS already attached

Description:

(MSC 3.1.3)
(SM 1)

SM is in state PDP_INACTIVE <R.SM.PDP_INAC.M.001>, but has sent a GMMSM_ESTABLISH_REQ to initiate a new GMM connection. The MS has already been attached or the network has sent the ATTACH ACCEPT message to the MS, the indirect attach was successful. GMM sends a GMMSM_ESTABLISH_CNF <R.SM.MS_CONT.M.003>. Now session management can proceed with PDP context activation.

(GMM 1)

SM sends an ACTIVATE_PDP_CONTEXT_REQ message to activate the requested PDP context <R.SM.MS_ACT_S.M.001>, <R.SM.MS_CONT.M.004> and changes to state PDP_ACTIVE_PENDING <R.SM.PDP_A_P.M.001>, <R.SM.MS_ACT_S.M.002>. The message contains the selected NSAPI, PDP type and, if th MS requests a static address, the PDP address <R.SM.MS_ACT_S.M.004>.

.

Preamble:

- SM201

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_ESTABLISH_CNF

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM208: PDP context activation, MS initiated, new attach accepted, multiple instances

Description:

(MSC 3.1.3 with multiple instances)
(SM 1)

SM is in state PDP_INACTIVE <R.SM.PDP_INAC.M.001>, but has sent a GMMSM_ESTABLISH_REQ to initiate a new GMM connection. The MS has already been attached or the network has sent the ATTACH ACCEPT message to the MS, the indirect attach was successful. GMM sends a GMMSM_ESTABLISH_CNF <R.SM.MS_CONT.M.003>. Now session management can proceed with PDP context activation.

(GMM 1)

SM sends an ACTIVATE_PDP_CONTEXT_REQ message to activate the requested PDP context <R.SM.MS_ACT_S.M.001>, <R.SM.MS_CONT.M.004> and changes to state PDP_ACTIVE_PENDING <R.SM.PDP_A_P.M.001>, <R.SM.MS_ACT_S.M.002>. The message contains the selected NSAPI, PDP type and, if th MS requests a static address, the PDP address <R.SM.MS_ACT_S.M.004>.

.

Preamble:

- SM202

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

(3)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

(4)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

(5)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

(6)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

(7)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

(8)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_ESTABLISH_CNF

- GMMSM_UNITDATA_REQ
	sdu	NOT_USED

- GMMSM_UNITDATA_REQ
	sdu	NOT_USED

- GMMSM_UNITDATA_REQ
	sdu	NOT_USED

- GMMSM_UNITDATA_REQ
	sdu	NOT_USED

- GMMSM_UNITDATA_REQ
	sdu	NOT_USED

- GMMSM_UNITDATA_REQ
	sdu	NOT_USED

- GMMSM_UNITDATA_REQ
	sdu	NOT_USED

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM209: PDP context activation, receiving ACTIVATE_PDP_CONTEXT_ACCEPT

Description:

(MSC 3.1.4)
(SM 1)

SM is in state PDP_ACTIVE_PENDING <R.SM.PDP_A_P.M.001>. SM receives an ACTIVATE_PDP_CONTEXT_ACCEPT message indicating that the MS indicated PDP context activation succeeded <R.SM.MS_ACT_S.M.009>, <R.SM.MS_CONT.M.009>.

(SNDCP 1)

SM stops timer T3380 <R.SM.MS_ACT_S.M.010>, <R.SM.MS_CONT.M.010>. SM changes to state PDP_ACTIVE <R.SM.PDP_ACT.M.011>, <R.SM.MS_ACT_S.M.009>. SM sends an SNSM_ACTIVATE_IND to SNDCP <R.SM.MS_CONT.M.011>.

.

Preamble:

- SM207

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SNSM_ACTIVATE_IND         |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_8
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_1
	pco	AIR_PCO_123456789
	}

- SNSM_ACTIVATE_IND
	tlli	NOT_USED
	nsapi	NSAPI_2
	snsm_qos	SNSM_QOS_0
	sapi	SAPI_3
	radio_prio	SNSM_RADIO_PRIO_1
	ppp_hc	SMREG_VAN_NOT_USED
	msid	NOT_USED
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SNSM_HOME

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM210: PDP context activation, NSAPI now in use

Description:

(MSC 3.1.5)
(SM 1)

SM is in state PDP_ACTIVE <R.SM.PDP_ACT.M.001> and SNDCP has sent an SNSM_ACTIVATE_RES to inform the SM entity that the indicated NSAPI is now in use and that the acknowledged peer-to-peer LLC operation for the indicated SAPI is established, if necessary <R.SM.MS_CONT.M.012>.

(ACI 1)

SM sends an SMREG_PDP_ACTIVATE_CNF to ACI to confirm that the PDP context is active <R.SM.MS_CONT.M.013>.

.

Preamble:

- SM209

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_ACTIVATE_RES         |                           |

*==========================>*                           |

(2)  | SMREG_PDP_ACTIVATE_CNF    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_ACTIVATE_RES
	tlli	NOT_USED
	nsapi	SNSM_NSAPI_2
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	pdp_address	PDP_ADDRESS_1
	sdu	PCO_123456789

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM211: PDP context activation, timer expiring up to four times

Description:

(MSC 3.1.9)
(GMM 1)

SM is in state PDP_ACTIVE_PENDING. Instead of receiving the expected ACTIVATE_PDP_CONTEXT_ACCEPT message the timer T3380 expires (for a first, second, third or fourth time) <R.SM.ACT_ABNO.M.001>. Then the MS resends the ACTIVATE_PDP_CONTEXT_REQ message <R.SM.ACT_ABNO.M.002> and resets and restarts T3380 <R.SM.ACT_ABNO.M.003>.

.

Preamble:

- SM207

SNDCP/GACI                     SM                          GMM

|                           |                           |

- TIMEOUT (30000)

(1)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}
History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM212: PDP context activation, timer expiring for the fifth time

Description:

(MSC 3.1.10)
(GMM 1)

SM is in state PDP_ACTIVE_PENDING. Instead of receiving the expected ACTIVATE_PDP_CONTEXT_ACCEPT message the timer T3380 expires for the fifth time <R.SM.ACT_ABNO.M.004>. Then the MS releases all resources possibly allocated for this invocation <R.SM.ACT_ABNO.M.005> and aborts the procedure without re-attempting to activate the context <R.SM.ACT_ABNO.M.006>.

(ACI 1)

SM sends an SMREG_PDP_ACTIVATE_REJ with cause "Network failure". [HK: primitive not explicitely mentioned in GSM 4.08, 6.1.3.1.5, cause "Network failure" is just assumed here.].

.

Preamble:

- SM207

SNDCP/GACI                     SM                          GMM

|                           |                           |

- TIMEOUT (30000)

(1)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (30000)

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (30000)

(3)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (30000)

(4)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (30000)

(5)  |  SMREG_PDP_ACTIVATE_REJ   |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_NETWORK_FAILURE
	smreg_nsapi	SMREG_NSAPI_2

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM213: PDP context activation, network initiated

Description:

(MSC 3.1.7)

(SM 1)

SM is in state PDP_INACTIVE  <R.SM.PDP_INAC.M.001>. SM receives a REQUEST_PDP_CONTEXT_ACTIVATION message to be informed about a network initiated PDP context activation <R.SM.N_ACT_S.M.003>, <R.SM.NET_CONT.M.004>.

(ACI 1)

SM sends an SMREG_PDP_ACTIVATE_IND to inform ACI about the requested connection. <R.SM.NET_CONT.M.005>.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SMREG_PDP_ACTIVATE_IND    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	REQ_PDP_ACT
	ti	TI_0
	address	AIR_ADDRESS_1
	apn	AIR_APN_WELL_FORMED
	}

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	NOT_USED
	smreg_ti	TI_8
	smreg_apn	SMREG_APN_WELL_FORMED
	pdp_type	PDP_TYPE_IP_V_4
	pdp_address	PDP_ADDRESS_1

History:

10-May-2000	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM214: PDP context activation, network initiated, ACI accepts

Description:

(MSC 3.1.8)
(SM 1)

SM has sent an SMREG_PDP_ACTIVATE_IND to ACI and receives an SMREG_PDP_ACTIVATE_REQ <R.SM.NET_CONT.M.006>.

(GMM 1)

SM starts timer3380and sends GMMSM_ESTABLISH_REQ

.

Preamble:

- SM213

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MT
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_5
	smreg_ti	TI_8
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

History:

10-May-2000	HK	Initial

04-Oct-2000	HK	Replaced NOT_USEDs.

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

08-Feb-2001	HK	GMMSM_UNITDATA_REQ replaced by GMMSM_ESTABLISH_REQ.

### SM229: PDP context activation, network initiated, ACI accepts

Description:

(MSC 3.1.8)
(SM 1)

SM has sent an SMREG_PDP_ACTIVATE_IND to ACI and receives an SMREG_PDP_ACTIVATE_REQ <R.SM.NET_CONT.M.006>.

(GMM 1)

SM starts timer3380 <R.SM.NET_CONT.M.007>. SM sends an ACTIVATE_PDP_CONTEXT_REQ message and changes to state PDP_ACTIVE_PENDING <R.SM.PDP_A_P.M.001>, <R.SM.NET_CONT.M.008>. The ACTIVATE_PDP_CONTEXT_REQ message sent by the MS in order to initiate the PDP context activation shall contain the PDP address requested by the network in the REQUEST_PDP_CONTEXT_ACTIVATION message <R.SM.N_ACT_S.M.007>. The rest of the activation procedure is the same for MS initiated and network initiated context activation. <R.SM.N_ACT_S.A.009>

.

Preamble:

- SM214

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*
(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_ESTABLISH_CNF

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_8
	nsapi	AIR_NSAPI_5
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

History:

10-May-2000	HK	Initial

04-Oct-2000	HK	Replaced NOT_USEDs.

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM216: Receiving ACTIVATE_PDP_CONTEXT_ACCEPT after collision

Description:

(similar to SM209)
There has been a collision of MS ond MT context activation. The MT activation was discarded and the accept from the network is expected. Now in this case this accept arrives.

.

Preamble:

- SM803a

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SNSM_ACTIVATE_IND         |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_8
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_1
	pco	AIR_PCO_123456789
	}

- SNSM_ACTIVATE_IND
	tlli	NOT_USED
	nsapi	NSAPI_2
	snsm_qos	SNSM_QOS_0
	sapi	SAPI_3
	radio_prio	SNSM_RADIO_PRIO_1
	ppp_hc	SMREG_VAN_NOT_USED
	msid	NOT_USED
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SNSM_HOME

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM217: GMM establishment requested, timer expiring for the fifth time

Description:

In preamble 201 a first GMMSM_ESTABLISH_REQ has been sent, but not been confirmed. Now 4 other requestes are also not confirmed and the context activation is rejected.

Preamble:

- SM201

SNDCP/GACI                     SM                          GMM

|                           |                           |

- TIMEOUT (30000)

(1)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

- TIMEOUT (30000)

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

- TIMEOUT (30000)

(3)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

- TIMEOUT (30000)

(4)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

- TIMEOUT (30000)

(5)  |  SMREG_PDP_ACTIVATE_REJ   |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_ESTABLISH_REQ

- GMMSM_ESTABLISH_REQ

- GMMSM_ESTABLISH_REQ

- GMMSM_ESTABLISH_REQ

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_NETWORK_FAILURE
	smreg_nsapi	SMREG_NSAPI_2

History:

12-Oct-2000	HK	Initial

### SM218: PDP context activation, requested QoS “subscribed”

Description:

A contex is requested by the user. The special issue of this case is to test the correct handling of “subscribed” QoS parameters: if the user requests subscribed QoS parameters, any answer from the net shall to be accepted. In this case the requested QoS is set to subscribed values.

.

Preamble:

SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

|                           |                           |

Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_SUBSCRIBED
	smreg_min_qos	SMREG_QOS_SUBSCRIBED
	smreg_nsapi	SMREG_NSAPI_2
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

History:

17-Nov-2000	HK	Initial

### SM219: PDP context activation, minimum QoS “subscribed”

Description:

A contex is requested by the user. The special issue of this case is to test the correct handling of “subscribed” QoS parameters: if the user requests subscribed QoS parameters, any answer from the net shall to be accepted. In this case the minimum QoS is set to subscribed values.

.

Preamble:

SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

|                           |                           |

Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_SUBSCRIBED
	smreg_nsapi	SMREG_NSAPI_2
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

History:

17-Nov-2000	HK	Initial

### SM220: PDP context activation, MS initiated, new attach accepted by the network or MS already attached

Description:

A context activation has been requested by the user. The quality of service has “subscribed” values for <A> the requested and minimum QoS or <B> the minimum QoS.

.

Variants:

<A>...<B>

Preamble:

<A>SM218

<B>SM219

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

|                           |                           |

Parametrization:

Primitive	Parameter	Value

- GMMSM_ESTABLISH_CNF

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
<A>	qos	AIR_QOS_SUBSCRIBED
<B>	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

History:

17-Nov-2000	HK	Initial

### SM221: PDP context activation, requested QoS “subscribed”receiving ACTIVATE_PDP_CONTEXT_ACCEPT

Description:

A context activation with requested QoS set to “subscribed” values has been requested by SMREG user. The network responds with a set of QoS values which must be accepted because the requested QoS was all “subscribed”

.

Preamble:

SM220A

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SNSM_ACTIVATE_IND         |                           |

*<==========================*                           |

|                           |                           |

Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_8
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_1
	pco	AIR_PCO_123456789
	}

- SNSM_ACTIVATE_IND
	tlli	NOT_USED
	nsapi	NSAPI_2
	snsm_qos	SNSM_QOS_0
	sapi	SAPI_3
	radio_prio	SNSM_RADIO_PRIO_1
	ppp_hc	SMREG_VAN_NOT_USED
	msid	NOT_USED
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SNSM_HOME

History:

17-Nov-2000	HK	Initial

24-Nov-2000	HK	Improved ti handling.

### SM222: PDP context activation, network initiated, ACI does not accept

Description:

An MT context activation has been indicated to ACI. ACI does not accept. SM sends a REQ_PDP_ACT_REJ message to the network.

Note: SMREG_PDP_ACTIVATE_RES will be renamed to SMREG_PDP_ACT_REJ_RES when changes from Aalborg are submitted!!!

.

Preamble:

- SM213

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_RES    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_RES
	smreg_ti	TI_8
	smreg_cause	SMREG_RC_INSUF_RES

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	REQ_PDP_ACT_REJ
	ti	TI_8
	sm_cause	AIR_SM_CAUSE_INSUF_RES
	}

History:

04-Jan-2001	HK	Initial

### SM223: PDP context activation, NSAPI now in use

Description:

In preamble SM210 an unacknowledged context for NSAPI 2 has been successfully activated. Now in this case a second context for NSAPI 8 is activated, but this time one with acknowledged LLC operation mode.

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(1)  |                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SNSM_ACTIVATE_IND         |                           |

*<==========================*                           |

(1)  | SNSM_ACTIVATE_RES         |                           |

*==========================>*                           |

(2)  | SMREG_PDP_ACTIVATE_CNF    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_ACK
	smreg_min_qos	SMREG_QOS_ACK
	smreg_nsapi	SMREG_NSAPI_8
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_2
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- GMMSM_ESTABLISH_CNF

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_1
	nsapi	AIR_NSAPI_8
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_ACK
	address	AIR_ADDRESS_2
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_9
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_ACK
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_2
	pco	AIR_PCO_123456789
	}

- SNSM_ACTIVATE_IND
	tlli	NOT_USED
	nsapi	NSAPI_8
	snsm_qos	SNSM_QOS_ACK
	sapi	SAPI_3
	radio_prio	SNSM_RADIO_PRIO_1
	ppp_hc	SMREG_VAN_NOT_USED
	msid	NOT_USED
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SNSM_HOME

- SNSM_ACTIVATE_RES
	tlli	NOT_USED
	nsapi	SNSM_NSAPI_8
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_ACK
	smreg_nsapi	SMREG_NSAPI_8
	pdp_address	PDP_ADDRESS_2
	sdu	PCO_123456789

History:

08-Jan-2001	HK	Initial

### SM224: PDP context activation, receiving ACTIVATE_PDP_CONTEXT_REJECT

Description:

After reception of an ACTIVATE_PDP_CONTEXT_REJECT the activation is rejected.

.

Preamble:

- SM207

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_REJ
	ti	TI_8
	sm_cause	AIR_SM_CAUSE_GGSN_ACT_REJ
	pco	AIR_PCO_123456789
	}

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_GGSN_ACT_REJ
	smreg_nsapi	SMREG_NSAPI_2

History:

18-Jan-2001	HK	Initial

### SM226: PDP context activation, MS initiated, header compresion

Description:

Like test case 201, but with header compression

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_4
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_BOTH_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

History:

02-Feb-2001	HK	Initial

### SM227: PDP context activation, MS initiated, new attach accepted by the network or MS already attached, header compression

Description:

Like test case 207, but with header compression

.

Preamble:

- SM226

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_ESTABLISH_CNF

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

History:

02-Feb-2001	HK	Initial

### SM228: PDP context activation, receiving ACTIVATE_PDP_CONTEXT_ACCEPT, header compression

Description:

Like test case 209, but with header compression

.

Preamble:

- SM227

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SNSM_ACTIVATE_IND         |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_8
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_1
	pco	AIR_PCO_123456789
	}

- SNSM_ACTIVATE_IND
	tlli	NOT_USED
	nsapi	NSAPI_2
	snsm_qos	SNSM_QOS_0
	sapi	SAPI_3
	radio_prio	SNSM_RADIO_PRIO_1
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_4
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_BOTH_DIRECT
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SNSM_HOME

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM230: PDP context activation, receiving ACTIVATE_PDP_CONTEXT_ACCEPT

Description:

Like SM209, but in the SMREG_PDP_ACTIVATE_REQ PDP_ADDRESS_1 was requested and the ACTIVATE_PDP_ACC comes back with PDP_ADDRESS_2. In SM209 the requested address was the same as the accepted one.

.

Preamble:

- SM207

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SNSM_ACTIVATE_IND         |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_8
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_2
	pco	AIR_PCO_123456789
	}

- SNSM_ACTIVATE_IND
	tlli	NOT_USED
	nsapi	NSAPI_2
	snsm_qos	SNSM_QOS_0
	sapi	SAPI_3
	radio_prio	SNSM_RADIO_PRIO_1
	ppp_hc	SMREG_VAN_NOT_USED
	msid	NOT_USED
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SNSM_HOME

History:

19-Apr-2001	HK	Initial

### SM231: PDP context activation, NSAPI now in use

Description:

Like SM209, but in the SMREG_PDP_ACTIVATE_REQ PDP_ADDRESS_1 was requested and the ACTIVATE_PDP_ACC comes back with PDP_ADDRESS_2. In SM209 the requested address was the same as the accepted one.

.

Preamble:

- SM230

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_ACTIVATE_RES         |                           |

*==========================>*                           |

(2)  | SMREG_PDP_ACTIVATE_CNF    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_ACTIVATE_RES
	tlli	NOT_USED
	nsapi	SNSM_NSAPI_2
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	pdp_address	PDP_ADDRESS_2
	sdu	PCO_123456789

History:

19-Apr-2001	HK	Initial

### SM232: PDP context activation, receiving ACTIVATE_PDP_CONTEXT_ACCEPT

Description:

Networks accepts ACTIVATE_PDP_REQ during MT activation procedure.

.

Preamble:

- SM229

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SNSM_ACTIVATE_IND         |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_0
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_1
	pco	AIR_PCO_123456789
	}

- SNSM_ACTIVATE_IND
	tlli	NOT_USED
	nsapi	NSAPI_5
	snsm_qos	SNSM_QOS_0
	sapi	SAPI_3
	radio_prio	SNSM_RADIO_PRIO_1
	ppp_hc	SMREG_VAN_NOT_USED
	msid	NOT_USED
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SNSM_HOME

History:

02-may-2001	HK	Initial

### SM233: PDP context activation, MS initiated, dynamic address allocation requested

Description:

An MO context activation requests dynamic pdp address allocation.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_DYN
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

History:

17-May-2001	HK	Initial

### SM234: PDP context activation, MS initiated, new attach accepted by the network or MS already attached

Description:

An MO context activation requests dynamic pdp address allocation. In this case the GMMSM_ESTABLISH_CNF is received.

Preamble:

- SM233

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_ESTABLISH_CNF

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_DYN
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

History:

17-May-2001	HK	Initial

### SM235: 2nd PDP context activation after dynamic one, MS initiated, APN not given in request: not comparable with possible MT activation

Description:

An MO context activation does not explicitly request an APN.

.

Preamble:

- SM234

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_8
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_EMPTY
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

History:

17-May-2001	HK	Initial

### SM236: 2nd PDP context activation after dynamic one, PDP context activation, MS initiated, APN not given in request: not comparable with possible MT activation

Description:

An MO context activation does not explicitly request an APN.

In this case the GMMSM_ESTABLISH_CNF is received.

Preamble:

- SM235

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_ESTABLISH_CNF

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_1
	nsapi	AIR_NSAPI_8
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	NOT_USED
	pco	AIR_PCO_987654321
	}

History:

17-May-2001	HK	Initial

### SM237: 2nd PDP context activation after dynamic one, MS initiated comparable with possible MT activation

Description:

Type, address and APN given.

.

Preamble:

- SM234

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(2)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_8
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_WELL_FORMED
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

History:

17-May-2001	HK	Initial

### SM238: 2nd PDP context activation after dynamic one, PDP context activation, MS initiated, comparable with possible MT activation

Description:

Type, address and APN given.

In this case the GMMSM_ESTABLISH_CNF is received.

Preamble:

- SM237

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ       |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_ESTABLISH_CNF

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_1
	nsapi	AIR_NSAPI_8
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_WELL_FORMED
	pco	AIR_PCO_987654321
	}

History:

17-May-2001	HK	Initial

### SM239: PDP context activation, NSAPI now in use, header compression rejected by SNDCP

Description:

An SNSM_ACTIVATE_IND has been sent to SNDCP with header compression requested. Now in this case SNDCP responds, but rejects the header compression.

.

Preamble:

- SM228

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_ACTIVATE_RES         |                           |

*==========================>*                           |

(2)  | SMREG_PDP_ACTIVATE_CNF    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_ACTIVATE_RES
	tlli	NOT_USED
	nsapi	SNSM_NSAPI_2
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	pdp_address	PDP_ADDRESS_1
	sdu	PCO_123456789

History:

29-Oct-1999	HK	Initial

### SM240: PDP context activation, timer expiring for the third time, the ACTIVATE_PDP_ACC, then deactivation and reactivation attempt with 5 expiries of T3380

Description:

A context activation is started, timer T3380 expires 3 times, activation is accepted by the network, the context is deactivated. Reactivation is attempted, the T3380 expires 5 times and the SMREG_PDP_ACTIVATE_REQ is rejected.

Preamble:

- SM207

SNDCP/GACI                     SM                          GMM

|                           |                           |

- TIMEOUT (4500)

(1)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (4500)

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (4500)

(3)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(4)  | SNSM_ACTIVATE_IND         |                           |

*<==========================*                           |

(5)  | SNSM_ACTIVATE_RES         |                           |

*==========================>*                           |

(6)  | SMREG_PDP_ACTIVATE_CNF    |                           |

*<==========================*                           |

(7)  | SMREG_PDP_DEACTIVATE_REQ  |                           |

*==========================>*                           |

- TIMEOUT (8000)

(8)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(9)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(10) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

(11) |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(12) | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

TIMEOUT (1000)

(13) | SNSM_DEACTIVATE_RES       |                           |

*==========================>*                           |

(14) | SMREG_PDP_DEACTIVATE_CNF  |                           |

*<==========================*                           |

(15) | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(16) |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(17) |                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*

(18) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (4500)

(19) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (4500)

(20) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (4500)

(21) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (4500)

(22) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (4500)

(23) |  SMREG_PDP_ACTIVATE_REJ   |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_8
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_1
	pco	AIR_PCO_123456789
	}

- SNSM_ACTIVATE_IND
	tlli	NOT_USED
	nsapi	NSAPI_2
	snsm_qos	SNSM_QOS_0
	sapi	SAPI_3
	radio_prio	SNSM_RADIO_PRIO_1
	ppp_hc	SMREG_VAN_NOT_USED
	msid	NOT_USED
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SNSM_HOME

- SNSM_ACTIVATE_RES
	tlli	NOT_USED
	nsapi	SNSM_NSAPI_2
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT

- SMREG_PDP_ACTIVATE_CNF
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	pdp_address	PDP_ADDRESS_1
	sdu	PCO_123456789

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_2
	smreg_local	SMREG_NONLOCAL

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	DEACT_PDP_ACC
	ti	TI_8
	}

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_NO

- SNSM_DEACTIVATE_RES
	tlli	NOT_USED
	nsapi	SMREG_NSAPI_2

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_2

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- GMMSM_ESTABLISH_CNF

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_NETWORK_FAILURE
	smreg_nsapi	SMREG_NSAPI_2

History:

12-Apr-2002	HK	Initial

## Context deactivation

### SM301: PDP context deactivation, MS initiated

Description:

(MSC 3.3.1)
(SM 1)

SM is in state PDP_ACTIVE <R.SM.PDP_ACT.M.001>. ACI sends an SMREG_PDP_DEACTIVATE_REQ to iniate the anonymous PDP context deactivation <R.SM.PDP_DE_M.M.001>.

(GMM 1)

SM starts timer T3390 <R.SM.MS_DEACT.M.003>, <R.SM.PDP_DE_M.M.002>. SM enters state PDP_INACTIVE_PENDING <R.SM.PDP_I_P.M.001><R.SM.MS_DEACT.M.002> and sends a DEACTIVATE_PDP_CONTEXT_REQ message <R.SM.MS_DEACT.M.001>, <R.SM.PDP_DE_M.M.003> containing the transaction identifier in use for the PDP context to be deactivated and a cause code that typically indicates one of the following causes: #26 insufficient resources, #36 regular PDP context deactivation or #37 QoS not accepted.

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_DEACTIVATE_REQ  |                           |

*==========================>*                           |

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_2
	smreg_local	SMREG_NONLOCAL

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}	
History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM302: PDP context deactivation, MS initiated, network accepts

Description:

(MSC 3.3.2)
(SM 1)

SM is in state PDP_INACTIVE_PENDING <R.SM.PDP_I_P.M.001>. The network sends a DEACTIVATE_PDP_CONTEXT_ACCEPT message to inform the MS that the deactivation request is accepted <R.SM.MS_DEACT.M.007>, <R.SM.PDP_DE_M.M.007>.

(SNDCP 1)

SM stops timer T3390 <R.SM.MS_DEACT.M.008>, <R.SM.PDP_DE_M.M.008>. SM enters state PDP_INACTIVE <R.SM.PDP_INAC.M.001>. SM sends an SNSM_DEACTIVATE_IND to inform SNDCP that the NSAPI in question can be de-allocated <R.SM.PDP_DE_M.M.009>. (SNDCP will do so only if the LLC link in question is not used by any other PDP context using acknowledged mode <R.SM.MS_DEACT.M.009>.

.

Preamble:

- SM301

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	DEACT_PDP_ACC
	ti	TI_8
	}

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_NO

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM303: PDP context deactivation, MS initiated, NSAPI released

Description:

(MSC 3.3.3)
(SM 1)

SM is in state PDP_INACTIVE <R.SM.PDP_INAC.M.001>. SNDCP sends an SNSM_DEACTIVATE_RES to inform SM that the NSAPI in question has been released <R.SM.PDP_DE_M.M.012>.

(ACI 1)

SM sends an SMREG_PDP_DEACTIVATE_CNF to ACI to confirm that the PDP context has been deactivated <R.SM.PDP_DE_M.M.013>.

.

Preamble:

- SM302

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_DEACTIVATE_RES       |                           |

*==========================>*                           |

(2)  | SMREG_PDP_DEACTIVATE_CNF  |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_DEACTIVATE_RES
	tlli	NOT_USED
	nsapi	SMREG_NSAPI_2

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_2

History:

12-Nov-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM304: PDP context deactivation, network initiated

Description:

(MSC 3.3.4)

(SM 1)

SM is in state PDP_ACTIVE <R.SM.PDP_ACT.M.001>. The network sends a DEACTIVATE_PDP_CONTEXT_REQ message to initiate a PDP context deactivation <R.SM.N_DEACT.I.001>, <R.SM.N_DEACT.M.004>, <R.SM.PDP_DE_N.M.004>. The message contains the transaction identifier inn use for the PDP context to be deactivated and a cause code that typically indicates one of the following causes: #36 regular PDP context deactivation, #38 network failure or #39 reactivation requested. .

(GMM 1)

SM enters state PDP_INACTIVE <R.SM.PDP_INAC.M.001>. SM answers with a DEACTIVATE_PDP_CONTEXT_ACCEPT message <R.SM.N_DEACT.M.005>, <R.SM.PDP_DE_N.M.005>.

(SNDCP 1)

SM sends an SNSM_DEACTIVATE_IND to SNDCP to inform the SNDCP entity that an NSAPI has been deallocated and cannot be used by the SNDCP entity anymore <R.SM.PDP_DE_N.M.006>. All buffered N-PDUs corresponding to this NSAPI are deleted. SNDCP will release the affected LLC link if it is not used by any other PDP context using acknowledged mode.

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

(3)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	DEACT_PDP_REQ
	ti	TI_8
	sm_cause	AIR_SM_CAUSE_NET_FAIL
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_ACC
	ti	TI_0
	}

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_NO

History:

7-Dec-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM305: PDP context deactivation, network initiated, NSAPI released

Description:

(MSC 3.3.5)
(SM 1)

SM is in state PDP_INACTIVE <R.SM.PDP_INAC.M.001>. SNDCP sends an SNSM_DEACTIVATE_RES to inform SM that the NSAPI in question has been released <R.SM.PDP_DE_N.M.010>.

(ACI 1)

SM sends an SMREG_PDP_DEACTIVATE_IND to ACI to confirm that the PDP context has been deactivated <R.SM.PDP_DE_N.M.011>.

.

Preamble:

- SM304

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_DEACTIVATE_RES       |                           |

*==========================>*                           |

(2)  | SMREG_PDP_DEACTIVATE_IND  |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_DEACTIVATE_RES
	tlli	NOT_USED
	nsapi	NSAPI_2

- SMREG_PDP_DEACTIVATE_IND
	nsapi_set	NSAPI_SET_2

History:

8-Dec-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM306: PDP context deactivation, timer expiring up to four times

Description:

(MSC 3.3.6)
(GMM 1)

SM is in state PDP_INACTIVE_PENDING. Instead of receiving the expected DEACTIVATE_PDP_CONTEXT_ACCEPT message the timer T3390 expires (for a first, second, third or fourth time) <R.SM.DE_ABNO.M.001>. Then the MS resends the DEACTIVATE_PDP_CONTEXT_REQ message <R.SM.DE_ABNO.M.002> and resets and restarts T3390 <R.SM.DE_ABNO.M.003>.

.

Preamble:

- SM301

SNDCP/GACI                     SM                          GMM

|                           |                           |

- TIMEOUT (8000)

(1)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}		
History:

9-Dec-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM307: PDP context deactivation, timer expiring for the fifth time

Description:

(MSC 3.3.7)
(GMM 1)

SM is in state PDP_INACTIVE_PENDING. Instead of receiving the expected DEACTIVATE_PDP_CONTEXT_ACCEPT message the timer T3390 expires for the fifth time <R.SM.DE_ABNO.M.004>.

(SNDCP 1)

Then the MS releases all resources possibly allocated for this invocation <R.SM.DE_ABNO.M.005> and erases the PDP context related data <R.SM.DE_ABNO.M.006>.

Note: the context is deactivated locally because the net does not seem to be available.

.

Preamble:

- SM301

SNDCP/GACI                     SM                          GMM

|                           |                           |

- TIMEOUT (8000)

(1)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(3)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(4)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(5)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_YES
History:

9-Dec-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM308: PDP context deactivation, network initiated

Description: 
(MSC 3.3.8)
(SM 1)

SM is in state PDP_INACTIVE_PENDING after having sent a DEACTIVATE_PDP_CONTEXT_REQ message. Instead of the expected DEACTIVATE_PDP_CONTEXT_ACCEPT a DEACTIVATE_PDP_CONTEXT_REQ is received (collision) <R.SM.DE_ABNO.A.012>.

(GMM 1)

SM replies with the message DECATIVATE_PDP_CONTEXT_ACCEPT <R.SM.DE_ABNO.M.013>. SM stops timer T3390 <R.SM.DE_ABNO.M.014>.

(SNDCP 1)

SM sends an SNSM_DEACTIVATE_IND to SNDCP to inform the SNDCP entity that an NSAPI has been deallocated and cannot be used by the SNDCP entity anymore. All buffered N-PDUs corresponding to this NSAPI are deleted.

Preamble:

- SM301

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

(3)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_8
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_ACC
	ti	TI_0
	}

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_NO
History:

10-May-2000	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM309: PDP context deactivation, MS initiated after SNSM_STATUS_REQ in state PDP_ACTIVE

Description:

SNDCP sends an SNSM_STATUS_REQ and SM deactivates all contexts using the affected SAPI.

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |    SNSM_STATUS_REQ        |                           |

*==========================>*                           |

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_STATUS_REQ
	tlli	NOT_USED
	sapi	SAPI_3
	status_cause	LL_RELCS_NORMAL

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_LLC_OR_SNDCP
	}	
History:

13-Oct-2000	HK	Initial

### SM310: PDP context deactivation after SNSM_STATUS_REQ, MS initiated, network accepts

Description:

The network accepts a contexts deactivation resulting from an SNSM_STATUS_REQ.

.

Preamble:

- SM309

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	DEACT_PDP_ACC
	ti	TI_8
	}

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_NO

History:

13-Oct-2000	HK	Initial

24-Nov-2000	HK	Improved ti handling.

### SM311: PDP context deactivation, MS initiated, NSAPI released

Description:

In the course of a context deactivation caused by an SNSM_STATUS_REQ SNDCP now responds and the user is notified with an SMREG_PDP_DEACTIVATE_IND.

Preamble:

- SM310

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_DEACTIVATE_RES       |                           |

*==========================>*                           |

(2)  | SMREG_PDP_DEACTIVATE_IND  |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_DEACTIVATE_RES
	tlli	NOT_USED
	nsapi	SMREG_NSAPI_2

- SMREG_PDP_DEACTIVATE_IND
	nsapi_set	NSAPI_SET_2

History:

13-Oct-2000	HK	Initial

### SM312: PDP context deactivation after SNSM_STATUS_REQ, timer expiring up to four times

Description:

In the course of a context deactivation due to SNSM_STATUS_REQ the DEACT_PDP_REQ mesage has been sent to the network but is not answered. The timer T3390 expires and the message is sent again.

Preamble:

- SM309

SNDCP/GACI                     SM                          GMM

|                           |                           |

- TIMEOUT (8000)

(1)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_LLC_OR_SNDCP
	}		
History:

3-Oct-2000	HK	Initial

### SM313: PDP context deactivation after SNSM_STATUS_REQ, timer expiring for the fifth time

Description:

In the course of a context deactivation caused by an SNSM_STATUS_REQ SNDCP now responds and the user is notified with an SMREG_PDP_DEACTIVATE_IND. After the 5th expiry the context will be deactivated locally.

Preamble:

- SM309

SNDCP/GACI                     SM                          GMM

|                           |                           |

- TIMEOUT (8000)

(1)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(3)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(4)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(5)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_LLC_OR_SNDCP
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_LLC_OR_SNDCP
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_LLC_OR_SNDCP
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_LLC_OR_SNDCP
	}

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_YES
History:

13-Oct-2000	HK	Initial

### SM314: PDP context deactivation, locally after GMMSM_RELEASE_IND in  state PDP_ACTIVE

Description:

After a GMMSM_RELEASE_IND all contexts are deactivated locally.

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_RELEASE_IND         |

|                           *<==========================*

(2)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_RELEASE_IND

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_YES

History:

13-Oct-2000	HK	Initial

### SM315: SNSM_STATUS_REQ with cause “no peer response”

Description:

SNDCP sends an SNSM_STATUS_REQ with cause “no peer response” and SM does not deactivate all contexts using the affected SAPI since in case of this cause the context will not be deactivated (GSM 4.65, 6.2.1.4).

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |    SNSM_STATUS_REQ        |                           |

*==========================>*                           |

MUTE (3000)

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_STATUS_REQ
	tlli	NOT_USED
	sapi	SAPI_3
	status_cause	SNSM_RELCS_NO_PEER_RES

- History:

04-Jan-2001	HK	Initial

### SM316: After SNSM_STATUS_REQ with cause "DM received" only ack contexts deactivated

Description:

In preamble SM223 an unacknowledged and an acknowledged context have been activated. Now in this case an SNSM_STATUS_REQ with cause  "DM received" is received. According to GSM 4.65, 6.2.1.4 SM shall then deactivate only the acknowledged contexts.

.

Preamble:

- SM223

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |    SNSM_STATUS_REQ        |                           |

*==========================>*                           |

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_STATUS_REQ
	tlli	NOT_USED
	sapi	SAPI_3
	status_cause	LL_RELCS_DM_RECEIVED

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_1
	sm_cause	AIR_SM_CAUSE_LLC_OR_SNDCP
	}	
History:

08-Jan-2001	HK	Initial

### SM317: GMMSM_RELEASE_IND in state CO_PDP_INACTIVE

Description:

A GMMSM_RELEASE_IND in state CO_PDP_INACTIVE will be ignored.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_RELEASE_IND         |

|                           *<==========================*

MUTE (3000)

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_RELEASE_IND

History:

16-Jan-2001	HK	Initial

### SM318: GMMSM_RELEASE_IND in state CO_PDP_INACTIVE_WAIT_SNDCP

Description:

A GMMSM_RELEASE_IND in state CO_PDP_INACTIVE_WAIT_SNDCP will be ignored.

.

Preamble:

- SM302

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_RELEASE_IND         |

|                           *<==========================*

MUTE (3000)

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_RELEASE_IND

History:

16-Jan-2001	HK	Initial

### SM319: GMMSM_RELEASE_IND in state CO_AWAIT_GMMSM_ESTABLISH_CNF

Description:

A GMMSM_RELEASE_IND in state CO_AWAIT_GMMSM_ESTABLISH_CNF will result in an SMREG_PDP_ACTIVATE_REJ after next expiry of timer T3380.

.

Preamble:

- SM201

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_RELEASE_IND         |

|                           *<==========================*

- TIMEOUT (30000)

(2)  |  SMREG_PDP_ACTIVATE_REJ   |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_RELEASE_IND

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_NETWORK_FAILURE
	smreg_nsapi	SMREG_NSAPI_2

History:

16-Jan-2001	HK	Initial

### SM320: GMMSM_RELEASE_IND in state CO_PDP_ACTIVE_PENDING

Description:

A GMMSM_RELEASE_IND in state CO_PDP_ACTIVE_PENDING will result in an SMREG_PDP_ACTIVATE_REJ after next expiry of timer T3380.

.

Preamble:

- SM207

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_RELEASE_IND         |

|                           *<==========================*

- TIMEOUT (30000)

(2)  |  SMREG_PDP_ACTIVATE_REJ   |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_RELEASE_IND

- SMREG_PDP_ACTIVATE_REJ
	smreg_cause	SMREG_RC_NETWORK_FAILURE
	smreg_nsapi	SMREG_NSAPI_2

History:

16-Jan-2001	HK	Initial

### SM321: PDP context deactivation after GMMSM_RELEASE_IND in state CO_PDP_ACTIVE_AWAIT_SNDCP

Description:

After a GMMSM_RELEASE_IND all contexts are deactivated.

.

Preamble:

- SM209

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_RELEASE_IND         |

|                           *<==========================*

(2)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_RELEASE_IND

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_YES

History:

16-Jan-2001	HK	Initial

### SM322: GMMSM_RELEASE_IND in state CO_PDP_INACTIVE_PENDING

Description:

A GMMSM_RELEASE_IND in state CO_PDP_INACTIVE_PENDING will result in an SNSM_DEACTIVATE_IND after next expiry of timer T3390.

.

Preamble:

- SM301

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_RELEASE_IND         |

|                           *<==========================*

- TIMEOUT (30000)

(2)  |  SNSM_DEACTIVATE_IND      |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_RELEASE_IND

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_YES

History:

16-Jan-2001	HK	Initial

### SM323: GMMSM_RELEASE_IND in state CO_PDP_STATUS_PENDING

Description:

A GMMSM_RELEASE_IND in state CO_PDP_STATUS_PENDING will result in an SNSM_DEACTIVATE_IND after next expiry of timer T3390.

.

Preamble:

- SM309

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_RELEASE_IND         |

|                           *<==========================*

- TIMEOUT (8000)

(2)  |  SNSM_DEACTIVATE_IND      |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_RELEASE_IND

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_YES

History:

16-Jan-2001	HK	Initial

### SM324: PDP context deactivation, MS initiated after SNSM_STATUS_REQ in state PDP_ACTIVE_WAIT_SNDCP

Description:

SNDCP sends an SNSM_STATUS_REQ and SM deactivates all contexts useing the affected SAPI.

.

Preamble:

- SM209

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |    SNSM_STATUS_REQ        |                           |

*==========================>*                           |

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_STATUS_REQ
	tlli	NOT_USED
	sapi	SAPI_3
	status_cause	LL_RELCS_NORMAL

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_LLC_OR_SNDCP
	}	
History:

30-Jan-2001	HK	Initial

### SM325: PDP context deactivation, MS initiated and local

Description:

A context is deactivate locally.

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_DEACTIVATE_REQ  |                           |

*==========================>*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_2
	smreg_local	SMREG_LOCAL

- History:

07-May-2001	HK	Initial

### SM326: PDP context deactivation, MS initiated and local

Description:

A context is deactivate locally.

.

Preamble:

- SM325

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_YES

History:

07-May-1999	HK	Initial

### SM327: PDP context deactivation local, MS initiated, NSAPI released

Description:

A context is deactivated locally, SNSM responds.

.

Preamble:

- SM326

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_DEACTIVATE_RES       |                           |

*==========================>*                           |

(2)  | SMREG_PDP_DEACTIVATE_CNF  |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_DEACTIVATE_RES
	tlli	NOT_USED
	nsapi	SMREG_NSAPI_2

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_2

History:

07-May-1999	HK	Initial

### SM328: PDP context deactivation, MS initiated, NSAPI released after 5th expiry of T3390 and response from SNDCP

Description:

After 5th expiry of T3390 the deactivation of a context has been indicated to SNDCP. Now in this case SNDCP responds.

.

Preamble:

- SM307

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_DEACTIVATE_RES       |                           |

*==========================>*                           |

(2)  | SMREG_PDP_DEACTIVATE_CNF  |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_DEACTIVATE_RES
	tlli	NOT_USED
	nsapi	SMREG_NSAPI_2

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_2

History:

30-Oct-2001	HK	Initial

### SM329: PDP context deactivation, MS initiated and local in intermediate states

Description:

A context is deactivated locally while SM is in intermediate states.

CO_PDP_INACTIVE			no variant

CO_AWAIT_GMMSM_ESTABLISH_CNF    <A>

CO_PDP_ACTIVE_PENDING                  <B>

CO_PDP_ACTIVE_AWAIT_SNDCP           <C>

CO_PDP_ACTIVE                                no variant

CO_PDP_INACTIVE_PENDING               <D>

CO_PDP_INACTIVE_WAIT_SNDCP          <E>

CO_PDP_INACTIVE_WAIT_SNDCP_MT    <F>

CO_PDP_STATUS_PENDING                 <G>

.

Variants:

<A>...<G>

Preamble:

<A>SM201

<B>SM207

<C>SM209

<D>SM301

<E>SM302

<F>SM304

<G>SM309

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SMREG_PDP_DEACTIVATE_REQ  |                           |

*==========================>*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_2
	smreg_local	SMREG_LOCAL

- History:

21-Nov-2001	HK	Initial

### SM330: PDP context deactivation, MS initiated and local

Description:

A context is deactivate locally while the machine is in intermediate states.

.

Variants:

<A>...<G>

Preamble:

<A>SM329a

<B>SM329b

<C>SM329c

<D>SM329d

<E>SM329e

<F>SM329f

<G>SM329g

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_YES

History:

21-Nov -1999	HK	Initial

### SM331: PDP context deactivation, timer expiring for the fifth time after deactivation (3 expiries and 1 DEACT_PDP_ACC) and reactivation

Description:

A context has been activated, deactivated after 3 times expiry of T3390 and reception of 1 DEACT_PDP_ACC message, re-activated (SNDCP has not responded yet). Now in this case the context is deactivated once again, also with 5 times expiring of T3390 and then local deactivation.

.

Preamble:

- SM301

SNDCP/GACI                     SM                          GMM

|                           |                           |

- TIMEOUT (8000)

(1)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(3)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

(4)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(5)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

TIMEOUT (1000)

(6)  | SNSM_DEACTIVATE_RES       |                           |

*==========================>*                           |

(7)  | SMREG_PDP_DEACTIVATE_CNF  |                           |

*<==========================*                           |

(8)  | SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

(9)  |                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

(10) |                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*

(11) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

(12) |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(13) | SNSM_ACTIVATE_IND         |                           |

*<==========================*                           |

(14) | SMREG_PDP_DEACTIVATE_REQ  |                           |

*==========================>*                           |

(15) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(16) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(17) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(18) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(19) |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

- TIMEOUT (8000)

(20) | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	DEACT_PDP_ACC
	ti	TI_8
	}

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_NO

- SNSM_DEACTIVATE_RES
	tlli	NOT_USED
	nsapi	SMREG_NSAPI_2

- SMREG_PDP_DEACTIVATE_CNF
	nsapi_set	NSAPI_SET_2

- SMREG_PDP_ACTIVATE_REQ
	direc	DIREC_MO
	ppp_hc	SMREG_VAN_NOT_USED
	msid	MSID_0
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	pdp_type	PDP_TYPE_IP_V_4
	smreg_qos	SMREG_QOS_0
	smreg_min_qos	SMREG_QOS_0
	smreg_nsapi	SMREG_NSAPI_2
	smreg_ti	TI_NOT_USED
	pdp_address	PDP_ADDRESS_1
	smreg_apn	SMREG_APN_TEST
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SMREG_HOME
	sdu	PCO_987654321

- GMMSM_ESTABLISH_REQ

- GMMSM_ESTABLISH_CNF

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	ACTIVATE_PDP_REQ
	ti	TI_0
	nsapi	AIR_NSAPI_2
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	pco	AIR_PCO_987654321
	}

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_8
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_1
	pco	AIR_PCO_123456789
	}

- SNSM_ACTIVATE_IND
	tlli	NOT_USED
	nsapi	NSAPI_2
	snsm_qos	SNSM_QOS_0
	sapi	SAPI_3
	radio_prio	SNSM_RADIO_PRIO_1
	ppp_hc	SMREG_VAN_NOT_USED
	msid	NOT_USED
	dcomp	SMREG_COMP_NEITHER_DIRECT
	hcomp	SMREG_COMP_NEITHER_DIRECT
	dti_linkid	DTI_LINKID_ABCD
	dti_neighbor	DTI_NEIGHBOR_ABCD
	dti_direction	SNSM_HOME

- SMREG_PDP_DEACTIVATE_REQ
	nsapi_set	NSAPI_SET_2
	smreg_local	SMREG_NONLOCAL

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	SM_CAUSE_REGULAR
	}

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_YES
History:

11-Apr-2002	HK	Initial

## Context modification

### SM401: PDP context modification initiated by the network, MS accepts QoS and SAPI

Description:

(MSC 3.5.1)

(SM 1)

SM is in state PDP_ACTIVE <R.SM.PDP_ACT.M.001>. The network sends a MODIFY_PDP_CONTEXT_REQ message to initiate a PDP context modification <R.SM.CONT_MOD.M.005>, <R.SM.CO_MOD.M.004>, <R.SM.RAU_INTE.M.006>.

(GMM 1)

The MS accepts the proposed new QoS an the indicated LLC SAPI <R.SM.CONT_MOD.M.006> (That means the requested new QoS corresponds with the “minimum QoS given by GACI during activation). SM answers with a MODIFY_PDP_CONTEXT_ACCEPT message <R.SM.CONT_MOD.M.006>, <R.SM.CONT_MOD.M.007>, <R.SM.CO_MOD.M.005>, <R.SM.RAU_INTE.M.007>.

(SNDCP 1)

SM sends an SNSM_MODIFY_IND to SNDCP to to trigger change of the QoS profile (see GSM 04.08) for an NSAPI and indication of the SAPI to be used <R.SM.CO_MOD.M.006>, <R.SM.RAU_INTE.M.008>. It is also used by the SM entity in the SGSN to inform the SNDCP entity that an NSAPI shall be created, together with the (re-)negotiated QoS profile, the SAPI assigned, and, in the MS, the radio priority level to be used by RLC/MAC.

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

(3)  | SNSM_MODIFY_IND           |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	MOD_PDP_REQ
	ti	TI_8
	radio_prio	AIR_RADIO_PRIO_1
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	MOD_PDP_ACC
	ti	TI_0
	}

- SNSM_MODIFY_IND
	tlli	NOT_USED
	nsapi	NSAPI_2
	snsm_qos	SNSM_QOS_0
	sapi	SAPI_3
	radio_prio	SNSM_RADIO_PRIO_1
	send_no	NOT_USED
	rec_no	NOT_USED
History:

10-May-2000	HK	Initial.

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM402: PDP context modification initiated by the network, MS does not accept QoS and SAPI

Description:

(MSC 3.5.2)

(SM 1)

SM is in state PDP_ACTIVE <R.SM.PDP_ACT.M.001>. The network sends a MODIFY_PDP_CONTEXT_REQ message to initiate a PDP context modification <R.SM.CONT_MOD.M.005>.

(GMM 1)

The MS does not accept the new QoS and the indicated LLC SAPI <R.SM.CONT_MOD.M.008> and initiates the PDP context deactivation procedure by sending a DEACTIVATE_PDP_CONTEXT_REQ message <R.SM.CONT_MOD.M.009>.

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	MOD_PDP_REQ
	ti	TI_8
	radio_prio	AIR_RADIO_PRIO_1
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_UNACCEPTABLE
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	DEACT_PDP_REQ
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_QOS_NOT_ACC
	}

- History:

10-May-2000	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM403: PDP context modification, network initiated, SNDCP finished requested changes

Description:

(MSC 3.5.3)
(SM 1)

SM is in state PDP_INACTIVE <R.SM.PDP_INAC.M.001>. SNDCP sends an SNSM_DEACTIVATE_RES to inform SM that the NSAPI in question has been released <R.SM.CO_MOD.M.010>.

(ACI 1)

SM sends an SMREG_PDP_MODIFY_IND to ACI to confirm that the PDP context has been deactivated <R.SM.CO_MOD.M.011>.

.

Preamble:

- SM401

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_MODIFY_RES           |                           |

*==========================>*                           |

(2)  | SMREG_PDP_MODIFY_IND      |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_MODIFY_RES
	tlli	NOT_USED
	nsapi	NSAPI_2

- SMREG_PDP_MODIFY_IND
	smreg_qos	SMREG_QOS_0
	nsapi_set	NSAPI_SET_2

History:

8-Dec-1999	HK	Initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM404: PDP context modification initiated by the network in state PDP_ACTIVE_AWAIT_SNDCP, MS accepts QoS and SAPI

Description:

Like case 401, but SM is in state PDP_ACTIVE_AWAIT_SNDCP instead PDP_ACTIVE.

.

Preamble:

- SM209

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

(3)  | SNSM_MODIFY_IND           |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	MOD_PDP_REQ
	ti	TI_8
	radio_prio	AIR_RADIO_PRIO_1
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	MOD_PDP_ACC
	ti	TI_0
	}

- SNSM_MODIFY_IND
	tlli	NOT_USED
	nsapi	NSAPI_2
	snsm_qos	SNSM_QOS_0
	sapi	SAPI_3
	radio_prio	SNSM_RADIO_PRIO_1
	send_no	NOT_USED
	rec_no	NOT_USED
History:

10-Jan-2001	HK	Initial.

### SM405: PDP context modification in state PDP_ACTIVE_AWAIT_SNDCP, network initiated, SNDCP finished requested changes

Description:

Like case 403 but SM is in state PDP_ACTIVE_AWAIT_SNDCP instead PDP_ACTIVE.

Preamble:

- SM404

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_MODIFY_RES           |                           |

*==========================>*                           |

(2)  | SMREG_PDP_MODIFY_IND      |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_MODIFY_RES
	tlli	NOT_USED
	nsapi	NSAPI_2

- SMREG_PDP_MODIFY_IND
	smreg_qos	SMREG_QOS_0
	nsapi_set	NSAPI_SET_2

History:

10-Jan-2001	HK	Initial

## Handling of unknown, unforeseen and erroneous protocol data

### SM501: Message other than SM_STATUS with TI value “111”

Description:

(MSC 3.6.2)

(SM 1)

SM is in any state. It receives a message other than SM_STATUS and having a TI value “111” <R.SM.UN_TI.A.001>.

(GMM 1)

SM shall immediately send an SM_STATUS message with TI value “111”.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	MOD_PDP_REQ
	ti	TI_111
	radio_prio	AIR_RADIO_PRIO_1
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_UNACCEPTABLE
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_111
	sm_cause	AIR_SM_CAUSE_TI_INVAL
	}

- History:

10-May-2000 	HK	initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM502: Invalid transaction identifier value

Description:

(MSC 3.6.3)

A session management message except REQUEST_PDP_CONTEXT_ACTIVATION or SM_STATUS

is received by the MS specifying a transaction identifier which is not recognized as relating to an

active context or to a context that is in the process of activation or deactivation or has been [recently]

deactivated <R.SM.UN_TI.M.006>.

(GMM 1)

SM sends an SM_STATUS message with cause #81 “Invalid transaction identifier value” using the received transaction identifier value <R.SM.UN_TI.M.007>.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	MOD_PDP_REQ
	ti	TI_8
	radio_prio	AIR_RADIO_PRIO_1
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_UNACCEPTABLE
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_TI_INVAL
	}

- History:

10-May-2000 	HK	initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM503: Unknown message type

Description:

(MSC 3.6.5)

(SM 1)

If a mobile station receives an SM message with message type not defined for the PD or not implemented by the receiver<R.SM.UN_MSG_T.M.001>, then

(GMM 1)

it shall return a status message (SM STATUS) with cause # 97 "message type non-existent or not implemented"<R.SM.UN_MSG_T.M.002>.

Note: Here a ACT_AA_PDP_ACC is received. The anonymous access is not implemented.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu	ACT_AA_PDP_ACC_UNEX

- GMMSM_UNITDATA_REQ
	sdu	
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_8
	sm_cause	AIR_SM_CAUSE_MSG_T_NON_X
	}

- History:

10-May-2000 	HK	initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM504: Unforeseen message type, not compatible with protocol state

Description:

(MSC 3.6.6)

(SM 1)

If a mobile station receives an SM message with message type not compatible with the protocol state <R.SM.UN_MSG_T.M.006> and an RR connection exists <R.SM.UN_MSG_T.M.007>, then

(GMM 1)

it shall return a status message (SM STATUS) with cause # 98 " Message type not compatible with protocol state” <R.SM.UN_MSG_T.M.008>. (If no RR connection existed, the message is just ignored.

Note: Here an ACTIVATE_PDP_ACC is sent for a context that is already activated.

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_8
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_1
	pco	AIR_PCO_123456789
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_MSG_T_NOT_COMP
	}

- History:

10-May-2000 	HK	initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

24-Nov-2000	HK	Improved ti handling.

### SM505: Non_semantical mandatory element errors, not in list GSM 4.08, 8.5

Description:

(MSC 3.6.7)

(SM 1)

The mobile station receives an SM message with Non-semantical mandatory information element errors and the message is not in the list in [GSM 04.65, 8.5] <R.SM.MAND_ERR.M.005> and an RR connection exists <R.SM.MAND_ERR.M.002>

(GMM 1)

The MS shall return a status message (SM-STATUS) with cause # 96 "invalid mandatory information"<R.SM.MAND_ERR.M.003>. If no RR connection exists the message is just ignored <R.SM.MAND_ERR.M.004>.

Note: in this case a MOD_PDP_REQ with missing IE ‘QoS’ is received.

.

Preamble:

- SM209

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu	MOD_PDP_REQ_IE_MISS

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_8
	sm_cause	AIR_SM_CAUSE_MAND_INF_INVAL
	}

- History:

10-May-2000 	HK	initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM506: Non_semantical mandatory element errors, DEACT_PDP_REQ

Description:

(MSC 3.6.8)

(SM 1)

The mobile station receives an SM message with Non-semantical mandatory information element errors and the message is DEACTIVATE PDP CONTEXT REQUEST <R.SM.MAND_ERR.M.006>.

(GMM 1)

The MS shall return a DEACTIVATE PDP CONTEXT ACCEPT message. All resources allocated for that context shall be released.

.

Preamble:

- SM209

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

(3)  | SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu	DEACT_PDP_REQ_IE_MISS

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	DEACT_PDP_ACC
	ti	TI_0
	}

- SNSM_DEACTIVATE_IND
	tlli	NOT_USED
	nsapi_set	NSAPI_SET_2
	rel_ind	REL_IND_NO

- History:

10-May-2000 	HK	initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM507: Non_semantical mandatory element errors, REQ_PDP_ACT

Description:

(MSC 3.6.9)

(SM 1)

The mobile station receives an SM message with Non-semantical mandatory information element errors and the message is REQUEST_PDP_CONTEXT_ACTIVATION <R.SM.MAND_ERR.M.008>.

(GMM 1)

The MS shall return a REQUEST_PDP_CONTEXT_ACTIVATION_REJECT message with cause  # 96 "Invalid mandatory information".

.

Preamble:

- SM209

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu	REQ_PDP_ACT_IE_MISS

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	REQ_PDP_ACT_REJ
	ti	TI_8
	sm_cause	AIR_SM_CAUSE_MAND_INF_INVAL
	}

- History:

10-May-2000 	HK	initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM508: TI flag set to 1 with

Description:

(SM 1)

When an ACTIVATE AA PDP CONTEXT REQUEST or REQUEST PDP CONTEXT ACTIVATION message

is received with a transaction identifier flag set to "1", this message shall be ignored.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

MUTE (3000)

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	REQ_PDP_ACT
	ti	TI_1_000
	address	AIR_ADDRESS_1
	apn	AIR_APN_TEST
	}

- History:

20-May-2000	HK	initial

11-Oct-2000	HK	Fill ‘NOT_USED’ params.

### SM509: ACTIVATE_PDP_ACC with invalid SAPI number

Description:

An ACTIVATE_PDP_ACC message with a SAPI number not equal to 3, 5, 9, 11 arrives. According to GSM 4.08, 8.5 an SM_STATUS_REQ is sent.

.

Preamble:

- SM207

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_8
	llc_sapi	AIR_LLC_SAPI_17
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_1
	pco	AIR_PCO_123456789
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_MAND_INF_INVAL
	}

- History:

04-Jan-2001 	HK	initial

15-Mar-2002	HK	Now SM_STATUS is sent instead of DEACT_PDP_REQ.

### SM510: MOD_PDP_REQ with invalid SAPI number

Description:

A MOD_PDP_REQ message with a SAPI number not equal to 3, 5, 9, 11 arrives. According to GSM 4.08, 8.5 an SM_STATUS_REQ is sent.

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	MOD_PDP_REQ
	ti	TI_8
	radio_prio	AIR_RADIO_PRIO_1
	llc_sapi	AIR_LLC_SAPI_17
	qos	AIR_QOS_1
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_MAND_INF_INVAL
	}

- History:

04-Jan-2001 	HK	initial

15-Mar-2002	HK	Now SM_STATUS is sent instead of DEACT_PDP_REQ.

### SM511: Message not compatible with protocol state, ACTIVATE_PDP_ACC

Description:

An ACTIVATE_PDP_ACC message is received in a state where it is not expected. An SM_STATUS is sent. Pending timers are stopped.
<A>: CO_ACTIVE_AWAIT_SNDCP
<B>: CO_PDP_ACTIVE
<C>: CO_PDP_INACTIVE_PENDING
<D>: CO_PDP_STATUS_PENDING
<E>: CO_PDP_STATUS_WAIT_SNDCP

.

Variants:

<A>...<E>

Preamble:

- <A>SM209

- <B>SM210

- <C>SM301
<D>SM309
<E>SM310

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_8
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_1
	pco	AIR_PCO_123456789
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_MSG_T_NOT_COMP
	}

- History:

31-Jan-2001 	HK	initial

### SM512: Message not compatible with protocol state, ACTIVATE_PDP_REJ

Description:

An ACTIVATE_PDP_ACC message is received in a state where it is not expected. An SM_STATUS is sent. Pending timers are stopped.
<A>: CO_ACTIVE_AWAIT_SNDCP
<B>: CO_PDP_ACTIVE
<C>: CO_PDP_INACTIVE_PENDING
<D>: CO_PDP_STATUS_PENDING
<E>: CO_PDP_STATUS_WAIT_SNDCP

.

Variants:

<A>...<E>

Preamble:

- <A>SM209

- <B>SM210

- <C>SM301
<D>SM309
<E>SM310

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_REJ
	ti	TI_8
	sm_cause	AIR_SM_CAUSE_GGSN_ACT_REJ
	pco	AIR_PCO_123456789
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_MSG_T_NOT_COMP
	}

- History:

31-Jan-2001 	HK	initial

### SM513: Message not compatible with protocol state, DEACT_PDP_ACC

Description:

An ACTIVATE_PDP_ACC message is received in a state where it is not expected. An SM_STATUS is sent. Pending timers are stopped.
<A>: CO_ACTIVE_AWAIT_SNDCP
<B>: CO_PDP_ACTIVE
<C>: CO_PDP_ACTIVE_PENDING
<D>: CO_PDP_STATUS_WAIT_SNDCP

.

Variants:

<A>...<D>

Preamble:

- <A>SM209

- <B>SM210

- <C>SM207
<D>SM310

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	DEACT_PDP_ACC
	ti	TI_8
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_MSG_T_NOT_COMP
	}

- History:

31-Jan-2001 	HK	initial

### SM514: Message not compatible with protocol state, MOD_PDP_REQ

Description:

An ACTIVATE_PDP_ACC message is received in a state where it is not expected. An SM_STATUS is sent. Pending timers are stopped.
<A>: CO_PDP_INACTIVE_PENDING
<B>: CO_PDP_STATUS_PENDING
<C>: CO_PDP_STATUS_WAIT_SNDCP
<D>: CO_PDP_ACTIVE_PENDING

.

Variants:

<A>...<D>

Preamble:

- <A>SM301
<B>SM309
<C>SM310
<D>SM207

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	MOD_PDP_REQ
	ti	TI_8
	radio_prio	AIR_RADIO_PRIO_1
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_MSG_T_NOT_COMP
	}

- History:

31-Jan-2001 	HK	initial

15-Mar-2002	HK	Change sapi in accet message from 17

### SM515: MOD_PDP_REQ for non existing context

Description:

A MOD_PDP_REQ message for a non existing context is received.

.

Preamble:

- SM313

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_DEACTIVATE_RES       |                           |

*==========================>*                           |

(2)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(3)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_DEACTIVATE_RES
	tlli	NOT_USED
	nsapi	SMREG_NSAPI_2

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	MOD_PDP_REQ
	ti	TI_8
	radio_prio	AIR_RADIO_PRIO_1
	llc_sapi	AIR_LLC_SAPI_17
	qos	AIR_QOS_1
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_TI_INVAL
	}

- History:

18-Jul-2001 	HK	initial

### SM516: Invalid ti, ACTIVATE_PDP_ACC

Description:

An ACTIVATE_PDP_ACC message is received for an unused ti. An SM_STATUS is sent. Pending timers are stopped.
<A>: CO_PDP_INACTIVE_WAIT_SNDCP
<B>: CO_PDP_INACTIVE_WAIT_SNDCP_MT
<C>: CO_AWAIT_GMMSM_ESTABLISH_CNF

.

Variants:

<A>...<C>

Preamble:

- <A>SM302

- <B>SM304

- <C>SM201

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_ACC
	ti	TI_8
	llc_sapi	LLC_SAPI_3
	qos	AIR_QOS_1
	radio_prio	AIR_RADIO_PRIO_1
	address	AIR_ADDRESS_1
	pco	AIR_PCO_123456789
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_TI_INVAL
	}

- History:

20-Jul-2001 	HK	initial

### SM517: Invalid ti, ACTIVATE_PDP_REJ

Description:

An ACTIVATE_PDP_REJ message is received in a state where it is not expected. An SM_STATUS is sent. Pending timers are stopped.
<A>: CO_PDP_INACTIVE_WAIT_SNDCP
<B>: CO_PDP_INACTIVE_WAIT_SNDCP_MT
<C>: CO_AWAIT_GMMSM_ESTABLISH_CNF

.

Variants:

<A>...<C>

Preamble:

- <A>SM302

- <B>SM304

- <C>SM201

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	ACTIVATE_PDP_REJ
	ti	TI_8
	sm_cause	AIR_SM_CAUSE_GGSN_ACT_REJ
	pco	AIR_PCO_123456789
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_TI_INVAL
	}

- History:

31-Jan-2001 	HK	initial

### SM518: Invalid ti, DEACT_PDP_ACC

Description:

An ACTIVATE_PDP_ACC message is received in a state where it is not expected. An SM_STATUS is sent. Pending timers are stopped.
<A>: CO_PDP_INACTIVE_WAIT_SNDCP
<B>: CO_PDP_INACTIVE_WAIT_SNDCP_MT
<C>: CO_AWAIT_GMMSM_ESTABLISH_CNF

.

Variants:

<A>...<C>

Preamble:

- <A>SM302

- <B>SM304

- <C>SM201

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	DEACT_PDP_ACC
	ti	TI_8
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_TI_INVAL
	}

- History:

31-Jan-2001 	HK	initial

### SM519: Message not compatible with protocol state, MOD_PDP_REQ

Description:

A MOD_PDP_REQ message is received in a state where it is not expected. An SM_STATUS is sent. Pending timers are stopped.
<A>: CO_PDP_INACTIVE_WAIT_SNDCP
<B>: CO_PDP_INACTIVE_WAIT_SNDCP_MT
<C>: CO_AWAIT_GMMSM_ESTABLISH_CNF

.

Variants:

<A>...<C>

Preamble:

- <A>SM302

- <B>SM304

- <C>SM201

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	MOD_PDP_REQ
	ti	TI_8
	radio_prio	AIR_RADIO_PRIO_1
	llc_sapi	AIR_LLC_SAPI_3
	qos	AIR_QOS_1
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	SM_STATUS
	ti	TI_0
	sm_cause	AIR_SM_CAUSE_TI_INVAL
	}

- History:

31-Jan-2001 	HK	initial

## Routing Area update

### SM601: Receiving GMMSM_SEQUENCE_IND

Description:

In the course of a Routing Area Update SM receives a GMMSM_SEQUENCE_IND and passes it in form of SNSM_SEQUENCE_IND to SNDCP.

.

Preamble:

- SM210

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_SEQUENCE_IND        |

|                           *<==========================*

(2)  | SNSM_SEQUENCE_IND         |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_SEQUENCE_IND
	npdu_list	NPDU_LIST_OLD

- SNSM_SEQUENCE_IND
	tlli	NOT_USED
	nsapi	NSAPI_7
	rec_no	REC_NO_AA

History:

26-Oct-2000	HK	Initial

### SM602: Receiving SNSM_SEQUENCE_RES

Description:

In the course of a Routing Area Update SM receives an SNSM_SEQUENCE_RES and passes it in form of GMMSM_SEQUENCE_RES to GMM.

.

Preamble:

- SM601

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_SEQUENCE_RES         |                           |

*==========================>*                           |

(2)  |                           | GMMSM_SEQUENCE_RES        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_SEQUENCE_RES
	tlli	NOT_USED
	nsapi	NSAPI_7
	rec_no	REC_NO_AA

- GMMSM_SEQUENCE_RES
	npdu_list	NPDU_LIST_OLD

History:

26-Oct-2000	HK	Initial

### SM603: Receiving GMMSM_SEQUENCE_IND with unused nsapi

Description:

In the course of a Routing Area Update SM receives a GMMSM_SEQUENCE_IND and passes it in form of SNSM_SEQUENCE_IND to SNDCP. The nsapi given is not used.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_SEQUENCE_IND        |

|                           *<==========================*

(2)  | SNSM_SEQUENCE_IND         |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_SEQUENCE_IND
	npdu_list	NPDU_LIST_OLD

- SNSM_SEQUENCE_IND
	tlli	NOT_USED
	nsapi	NSAPI_7
	rec_no	REC_NO_AA

History:

26-Mar-2002	HK	Initial

### SM604: Receiving SNSM_SEQUENCE_RES

Description:

In the course of a Routing Area Update SM receives an SNSM_SEQUENCE_RES and passes it in form of GMMSM_SEQUENCE_RES to GMM. The affected nsapi is not used.

.

Preamble:

- SM603

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  | SNSM_SEQUENCE_RES         |                           |

*==========================>*                           |

(2)  |                           | GMMSM_SEQUENCE_RES        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- SNSM_SEQUENCE_RES
	tlli	NOT_USED
	nsapi	NSAPI_7
	rec_no	REC_NO_AA

- GMMSM_SEQUENCE_RES
	npdu_list	NPDU_LIST_OLD

History:

26-Mar-2002	HK	Initial

### SM605: Receiving GMMSM_SEQUENCE_IND with empty npdu number list

Description:

In the course of a Routing Area Update SM receives a GMMSM_SEQUENCE_IND. The list of npdu numbers is empty. A GMMSM_SEQUENCE_RES is sent immediately.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_SEQUENCE_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_SEQUENCE_RES        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_SEQUENCE_IND
	npdu_list	NPDU_LIST_EMPTY

- GMMSM_SEQUENCE_RES
	npdu_list	NPDU_LIST_EMPTY

History:

04-Apr-2002	HK	Initial

### SM606: Receiving GMMSM_SEQUENCE_IND, npdu number list for invalid nsapi value

Description:

In the course of a Routing Area Update SM receives a GMMSM_SEQUENCE_IND. The affected nsapi for the list of npdu numbers is an invalid value (valid nsapis values go from 0 to 15). A GMMSM_SEQUENCE_RES is sent immediately.

.

Preamble:

- SM100

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_SEQUENCE_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_SEQUENCE_RES        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_SEQUENCE_IND
	npdu_list	NPDU_LIST_INVALID

- GMMSM_SEQUENCE_RES
	npdu_list	NPDU_LIST_EMPTY

History:

04-Apr-2002	HK	Initial

## Config Primitives

### SM701: Config prim NO MT

Description:

The Config prim NO MT results in an air message ACTIVATE_PDP_REJ with cause INSUF_RES.

Preamble:

- SM207

SNDCP/GACI                     SM                          GMM

|                           |                           |

COMMAND (SM CONFIG NO MT 8)

(1)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	REQ_PDP_ACT_REJ
	ti	TI_8
	sm_cause	AIR_SM_CAUSE_INSUF_RES
	}

History:

10-May-2001	HK	Initial

## Collision of MT and MO activation

### SM801: Collision of MT and MO activation, all MO activations use dynamic address allocation

Description:

An MT activation collides with an MO activation. The MO activation uses dynamic address allocation. The collision is ignored and the MT activation handled like normal.
 .

Variants:

<A>...<B>

Preamble:

<A>SM233

<B>SM234

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SMREG_PDP_ACTIVATE_IND    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	REQ_PDP_ACT
	ti	TI_0
	address	AIR_ADDRESS_1
	apn	AIR_APN_WELL_FORMED
	}

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	NOT_USED
	smreg_ti	TI_8
	smreg_apn	SMREG_APN_WELL_FORMED
	pdp_type	PDP_TYPE_IP_V_4
	pdp_address	PDP_ADDRESS_1

History:

17-May-2001	HK	Initial

### SM802: Collision of MT and MO activation, 1 MO activations uses dynamic address allocation, 1 has not requested APN (i.e. not comparable)

Description:

An MT activation collides with two MO activations. The 1st MO activation uses dynamic address allocation. The 2nd has not requested the APN, i.e. is not comparable. The REQ_PDP_ACT is rejected.
 .

Variants:

<A>...<B>

Preamble:

<A>SM235

<B>SM236

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	REQ_PDP_ACT
	ti	TI_0
	address	AIR_ADDRESS_1
	apn	AIR_APN_WELL_FORMED
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	REQ_PDP_ACT_REJ
	ti	TI_8
	sm_cause	AIR_SM_CAUSE_INSUF_RES
	}

History:

17-May-2001	HK	Initial

### SM803: Collision of MT and MO activation, 1 MO activations uses dynamic address allocation, 1 is  comparable and equal to MT activation

Description:

An MT activation collides with two MO activations. The 1st MO activation uses dynamic address allocation. The 2nd is comparable with the MT activation and uses same type, adress, APN. The REQ_PDP_ACT is ignored.

Variants:

<A>...<B>

Preamble:

<A>SM237

<B>SM238

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

MUTE (3000)

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	REQ_PDP_ACT
	ti	TI_0
	address	AIR_ADDRESS_1
	apn	AIR_APN_WELL_FORMED
	}

History:

17-May-2001	HK	Initial

### SM804: Collision of MT and MO activation, 1 MO activations uses dynamic address allocation, 1 is  comparable and different from MT activation

Description:

An MT activation collides with two MO activations. The 1st MO activation uses dynamic address allocation. The 2nd is comparable with the MT activation and uses different adress or APN. The REQ_PDP_ACT is accepted and computed. Different type is also checked, but leads to rejection in the current implementation!) Different address in <A>, <B>, different APN in <C>, <D>.

Variants:

<A>...<D>

Preamble:

<A>SM237

<B>SM238

<C>SM237

<D>SM238

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  | SMREG_PDP_ACTIVATE_IND    |                           |

*<==========================*                           |

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	REQ_PDP_ACT
	ti	TI_0
<A>	address	AIR_ADDRESS_2
<B>	address	AIR_ADDRESS_2
<C>	address	AIR_ADDRESS_1
<D>	address	AIR_ADDRESS_1
<A>	apn	AIR_APN_WELL_FORMED
<B>	apn	AIR_APN_WELL_FORMED
<C>	apn	AIR_APN_TEST
<D>	apn	AIR_APN_TEST
	}

- SMREG_PDP_ACTIVATE_IND
	smreg_qos	NOT_USED
	smreg_ti	TI_8
<A>	smreg_apn	SMREG_APN_WELL_FORMED
<B>	smreg_apn	SMREG_APN_WELL_FORMED
<C>	smreg_apn	SMREG_APN_TEST
<D>	smreg_apn	SMREG_APN_TEST
	pdp_type	PDP_TYPE_IP_V_4
<A>	pdp_address	PDP_ADDRESS_2
<B>	pdp_address	PDP_ADDRESS_2
<C>	pdp_address	PDP_ADDRESS_1
<D>	pdp_address	PDP_ADDRESS_1

History:

17-May-2001	HK	Initial

History:

17-May-2001	HK	Initial

### SM805: Collison of MO and MT context activation, pdp type is not IP 4

Description:

During two MO activations with an IP version 4 addresses an MT activation with IP version 6 comes in. The activation is rejected because IP 6 is not supported yet.

Preamble:

- SM237

SNDCP/GACI                     SM                          GMM

|                           |                           |

(1)  |                           | GMMSM_UNITDATA_IND        |

|                           *<==========================*

(2)  |                           | GMMSM_UNITDATA_REQ        |

|                           *==========================>*

|                           |                           |

- Parametrization:

Primitive	Parameter	Value

- GMMSM_UNITDATA_IND
	sdu		
	{
	component	SM
	direction	DOWNLINK
	pd	REQ_PDP_ACT
	ti	TI_0
	address	AIR_ADDRESS_IP6
	apn	AIR_APN_TEST
	}

- GMMSM_UNITDATA_REQ
	sdu		
	{
	component	SM
	direction	UPLINK
	pd	REQ_PDP_ACT_REJ
	ti	TI_8
	sm_cause	AIR_SM_CAUSE_UNK_PDP
	}

History:

20-May-2001	HK	Initial

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |