Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM Protocol Stack

G23

GSI – General Serial Interface

(Edit via File/Properties/Summary!)

| Document Number: | 8434.104.01.002(Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.3 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2001-Jun-15 |
| Last changed: | 2003-Sep-12 by XINTEGRA |
| File Name: | Gsi_api.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2001-Jun-15 | STW |  | 0.1 |  | 1 |
| 2001-Nov-12 | STW |  | 0.2 |  | 2 |
| 2003-May-20 | XINTEGRA |  | 0.3 | Draft |  |

Notes:

- Initial version

- Better description, correction of type errors

Table of Contents

1.1	References	3

3.1	Data types	4

3.1.1	T_GSI_DCB – Device Control Block	4

3.2	Constants	6

3.3	Functions	8

3.3.1	GSI_Init – Driver Initialization	9

3.3.2	GSI_Exit – Termination of the driver	10

3.3.3	GSI_Read - Read data from the driver	11

3.3.4	GSI_Write – Transmit data via the serial interface	13

3.3.5	GSI_Look – Look at received data of the driver	14

3.3.6	GSI_Clear – Clear internal driver buffers	15

3.3.7	GSI_Flush – Flush internal driver buffer	16

3.3.8	GSI_SetSignal – Setup a Signal	17

3.3.9	GSI_ResetSignal – Remove a Signal	21

3.3.10	GSI_SetConfig – Setup a device configuration	22

3.3.11	GSI_GetConfig – Retrieve a driver configuration	23

3.3.12	GSI_CallBack – Callback entry for the driver	24

A.	Acronyms	25

B.	Glossary	25

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[C_8415.0026]	8415.026.99.012; March 19, 1999
Generic Driver Interface – Functional Specification; Condat

# Introduction

- G23 is a software package implementing Layers 2 and 3 of the ETSI-defined GSM air interface signaling protocol, and as such represents the part of a GSM mobile station’s protocol software which is both, platform and manufacturer independent. Therefore, G23 can be viewed as a building block providing standardized functionality through generic interfaces for easy integration.

The G23 suite of products consists of the following items:

- Layers 2 and 3 for speech & short message services,

- Layers 2 and 3 for fax & data services,

- Application Control Interface, AT Command Interface

- MMI  and MMI Framework (MFW)

- Test and integration support tools.

This document describes the functional interface of the G23 Standard Serial Receiver Transmitter (GSI) driver, API. This driver can be used for common communication purposes and includes UART, USART and IRDA devices. This API of the driver is derived from the generic driver interface specification [C_8415.0026].

NOTE: The driver devices need to be configured after initialization. Only the following functions can be called while the driver is not configured: GSI_Clear(), GSI_SetSignal() and GSI_SetConfig(). All other functions return DRV_NOTCONFIGURED. After the Initialization of the driver the device has to be configured with the GSI_SetConfig() function.

# Interface description of the GSI driver

## Data types

Name	Description

UBYTE	unsigned  8 bit integer data type

BYTE	signed 8 bit integer data type
USHORT	unsigned 16 bit integer data type

SHORT	signed 16 bit integer data type

ULONG	unsigned 32 bit integer data type

LONG	signed 32 bit integer data type

T_GSI_DCB	Device Control Block

### T_GSI_DCB – Device Control Block

Definition:

typedef struct T_GSI_DCB
{
	USHORT	Baud
	UBYTE	DataBits
	UBYTE	StopBits
	UBYTE	Parity
	UBYTE	RxFlowControl
	UBYTE	TxFlowControl
	USHORT	RxBufferSize
	USHORT	TxBufferSize
	USHORT	RxThreshold
	USHORT	TxThreshold
	UBYTE	XON
	UBYTE	XOFF
	UBYTE	EscChar
	USHORT	GuardPeriod
} ;

Description:

The device control block data type contains all parameters used to configure a serial device. The following table contains a list of the data elements and brief descriptions of them.

| Data element | Description |
| --- | --- |
| Baud | Transmission rate in bits/sec. See the following corresponding table (Table 1). The GSI_BAUD_AUTO should be used for auto detection of the baud rate and the framing format (if supported by the driver). The expected character for automatic detection is an "A" or "a". After successful detection of the speed and character framing the driver should work with the measured values. |
| DataBits | Number of bits per character. See the following corresponding table (Table 2). |
| StopBits | Number of stop bits attached to each character. See the following corresponding table (Table 3). |
| Parity | Type of Parity checking. See the following corresponding table (Table 4). |
| RxFlowControl | Type of flow control for receive direction. See the following corresponding table (Table 5). |
| TxFlowControl | Type of flow control for transmit direction. See the following corresponding table (Table 5). |
| RxBufferSize | Size of the receiver buffer in bytes. |
| TxBufferSize | Size of the transmitter buffer in bytes. |
| RxThreshold | Amount of received bytes that triggers the signal DRV_SIGTYPE_READ. |
| TxThreshold | Low watermark of the TX buffer that triggers the signal DRV_SIGTYPE_WRITE. |
| XON | ASCII code of the character which should be detected/send as XON. This parameter should be ignored if non of the flow control modes is set to Software-Flow-Control. |
| XOFF | ASCII code of the character which should be detected/send as XOFF. This parameter should be ignored if non of the flow control modes is set to Software-Flow-Control. |
| EscChar | ASCII character which could appear three times as an escape sequence. This parameter should be ignored if escape sequence detection is deactivated. |
| GuardPeriod | Denotes the minimal duration of the rest before the first and after the last character of the escape sequence, and the maximal receiving duration of the whole escape string. The unit of this parameter should be milliseconds. If this character is set to zero the driver will not detect escape sequences. |

## Constants

Name	Description

DRV_OK	Return value indicating the function completed successfully

DRV_INITIALIZED	Device is already initialized
DRV_NOTCONFIGURED	Device is not configured
DRV_BUFFER_FULL	The internal buffer is exhausted
DRV_INPROCESS	The requested function is currently being executed
DRV_INVALID_PARAMS	One or more parameters are out of range or invalid
DRV_SIGFCT_NOTAVAILABLE	The requested event signaling functionality is not available
DRV_INTERNAL_ERROR	Unspecified internal driver error

| Possible transmission rate | Description |
| --- | --- |
| GSI_BAUD_812500 | Transmission rate of 812500 bits/sec. |
| GSI_BAUD_406250 | Transmission rate of 406250 bits/sec. |
| GSI_BAUD_203125 | Transmission rate of 203125 bits/sec. |
| GSI_BAUD_115200 | Transmission rate of 115200 bits/sec. |
| GSI_BAUD_57600 | Transmission rate of 57600 bits/sec. |
| GSI_BAUD_38400 | Transmission rate of 38400 bits/sec. |
| GSI_BAUD_33900 | Transmission rate of 33900 bits/sec. |
| GSI_BAUD_28800 | Transmission rate of 28800 bits/sec. |
| GSI_BAUD_19200 | Transmission rate of 19200 bits/sec. |
| GSI_BAUD_14400 | Transmission rate of 14400 bits/sec. |
| GSI_BAUD_9600 | Transmission rate of 9600 bits/sec. |
| GSI_BAUD_7200 | Transmission rate of 7200 bits/sec. |
| GSI_BAUD_4800 | Transmission rate of 4800 bits/sec. |
| GSI_BAUD_2400 | Transmission rate of 2400 bits/sec. |
| GSI_BAUD_1200 | Transmission rate of 1200 bits/sec. |
| GSI_BAUD_600 | Transmission rate of 600 bits/sec. |
| GSI_BAUD_300 | Transmission rate of 300 bits/sec. |
| GSI_BAUD_150 | Transmission rate of 150 bits/sec. |
| GSI_BAUD_75 | Transmission rate of 75 bits/sec. |
| GSI_BAUD_AUTO | Automatic detection. |

Table 1

| Data bits | Description |
| --- | --- |
| GSI_CHAR5 | Send 5 bits per character. |
| GSI_CHAR6 | Send 6 bits per character. |
| GSI_CHAR7 | Send 7 bits per character. |
| GSI_CHAR8 | Send 8 bits per character. |

Table 2

| Stop bits | Description |
| --- | --- |
| GSI_STOP1 | Send 1 stop bit. |
| GSI_STOP15 | Send 1.5 stop bits. |
| GSI_STOP2 | Send 2 stop bits. |

Table 3

| Parity | Description |
| --- | --- |
| GSI_PARITYNO | Don't send a parity bit. |
| GSI_PARITYODD | Send an odd parity bit. |
| GSI_PARITYEVEN | Send an even parity bit. |
| GSI_PARITYSPACE | Send a space for parity bit. |

Table 4

| Flow Controls | Description |
| --- | --- |
| GSI_FLOWNO | No Flow Control. |
| GSI_FLOWHW | The status lines RTS/CTS (CT105/CT106) are used for Flow Control. |
| GSI_FLOWSW | The characters XON/XOFF are used for Flow Control. |

Table 5

## Functions

Name	Description

GSI_Init	Initialization of the serial communication driver
GSI_Exit	De-initialization of serial communication driver
GSI_Read	Read data received via the serial interface 
GSI_Write	Send data via the serial interface 
GSI_Look	Read data received via the serial interface, but leave data unchanged
GSI_Clear	Re-initialize all buffers
GSI_Flush	Flush transmit buffer
GSI_SetSignal	Define a signal used to indicated special events
GSI_ResetSignal	Un-define a signal the driver uses to indicate an event
GSI_SetConfig	Set a device configuration 
GSI_GetConfig	Retrieve a device configuration
GSI_CallBack	Callback entry for the driver

### GSI_Init – Driver Initialization

Definition:

USHORT GSI_Init
(
	UBYTE	DeviceNo,
	USHORT	DrvHandle,
	T_DRV_CB_FUNC	CallbackFunc,
	T_DRV_EXPORT**	DrvInfo
);

Parameters:

Name	Description

DeviceNo	serial device number

DrvHandle	unique handle for this device

CallbackFunc	This parameter points to the function that is called at the time an event that is to be signaled occurs. This value can be set to NULL if event signaling should not be possible.

DrvInfo	pointer to the driver parameters (see GDI specification document for a description of T_DRV_EXPORT)

Return values:

Name	Description

DRV_OK	Initialization successful
DRV_INVALID_PARAMS	The specified device does not exist
DRV_INTERNAL_ERROR	Internal driver error
DRV_INITIALIZED	Device already initialized

Description

The function initializes the module and the connected serial device.

The driver stores the DrvHandle and passes it in the T_DRV_SIGNAL structure of the Signal parameter to the calling process every time the callback function is called.

The driver exports its properties like its name, the functions to access driver functionality and a bitfield called flags by the parameter DrvInfo. If the driver does not support this property export it returns NULL in the DrvInfo parameter.

The function returns DRV_INITIALIZED if the device has already been initialized and is ready to be used or is already in use.

The driver uses the following default configuration until the function GSI_SetConfig() have been called:

- Transmission rate of 19200 bits/sec

- Send 8 bits per character

- Send 1 stop bit

- Don't send a parity bit

- No Flow Control for Rx and Tx direction

- No escape sequence detection

### GSI_Exit – Termination of the driver

Definition:

void GSI_Exit
(
	UBYTE	DeviceNo
);

Parameters:

Name	Description

DeviceNo	serial device number

Return values:

Name	Description

-	-

Description

The function is called when the device functionality is no longer required. The function “de-allocates” the resources (interrupts, buffers, etc.). The driver terminates the device regardless of any outstanding data to be sent.

### GSI_Read - Read data from the driver

Definition:

USHORT GSI_Read
(
	UBYTE	DeviceNo,
	void*	Buffer,
	USHORT*	Length,
	ULONG*	State
);

Parameters:

Name	Description

DeviceNo	serial device number

Buffer	This parameter points to the buffer wherein the data is to be copied.

Length	On call: Size of Buffer.
On return: If the function returns DRV_OK, it contains the number of characters read. Otherwise it contains 0.

State	Line states of the serial connection (see table below)

Return values:

Name	Description

DRV_OK	Function successful.
DRV_INVALID_PARAMS	The specified device does not exist.
DRV_INTERNAL_ERROR	Internal driver error.
DRV_NOTCONFIGURED	The device is not yet configured.

Description

This function reads out data from the driver. It copies received data into a caller provided buffer and it returns the line states. It should always return immediately after copying the data, without waiting for any more data.

The parameter *Length contains the size of the buffer in characters. When the function returns the parameter *Length contains the number of read characters.

Each copied character is cleared in the buffer of the driver. The driver only keeps the data available when calling the function GSI_Look().

If the device is not configured, the function returns DRV_ NOTCONFIGURED.

The parameter State is used in GSI_Read() and in GSI_Write(). The bits of this bit field are:

| signal | bitpos | read/write | meaning |
| --- | --- | --- | --- |
| SA | 31 | read/write | state of SA bit (Device ready) |
| SB | 30 | read/write | state of SB bit (Data valid) |
| X | 29 | read/write | state of X bit (Flow control) |
| RING | 28 | write | RING indicator |
| ESC | 27 | read | escape sequence detected |
| DISC | 26 | read | link disconnected |
| BRK | 25 | read/write | break received/to be send |
| BRKLEN | 0-7 | read/write | length of the break signal in characters |

The ESC bit is set to 1 to indicate a detected escape sequence.

The DISC bit is set to 1 to indicate a disconnected link.

The BRK bit is set to 1 when the driver has received/to send a break signal. The BRKLEN value is only valid if the BRK bit is set to 1.

The bits SA, SB, X and RING are mapped onto the corresponding lines by the driver. They are provided to give the user of the driver a more abstract view of the status lines, particularly of flow control. The mapping depends on the mode of flow control used:

| signal |  | mode of flow control |  |  |
| --- | --- | --- | --- | --- |
|  |  | GSI_FLOWNO | GSI_FLOWHW | GSI_FLOWSW |
| SA | write | CT107 = DSR | CT107 = DSR | CT107 = DSR |
|  | read | CT108 = DTR | CT108 = DTR | CT108 = DTR |
| SB | write | CT109 = DCD | CT109 = DCD | CT109 = DCD |
|  | read | CT105 = RTS | ON = 0 | CT105 = RTS |
| X | write | ignore | CT106 = CTS | mapped onto XON/XOFF |
|  | read | ON = 0 | CT105 = RTS |  |
| RING | write | CT125 = RI | CT125 = RI | CT125 = RI |

The SA bit is always mapped onto DSR/DTR. The bit is set to 0 when the device is ready to communicate.

The SB bit is always mapped onto DCD/RTS unless the mode of flow control is GSI_FLOWHW. In this case RTS is used for flow control and the driver returns always 0 (ON) as SB bit. The bit is set to 0 when the data are valid.

The X bit is mapped on CTS/RTS in case of outband flow control or on XON/XOFF in case of inband flow control. When no flow control is used, the driver ignores any setting of the X bit and returns 0 (ON) as X bit. The bit is set to 0 when the device is ready to receive data.

The RING bit is always mapped onto RI. The bit is set to 1 to indicate an incomming call.

Any unused line must be held in ON condition (0) by the driver.

Any unused bits must be set to 0 by the driver.

NOTE: When calling the function with a buffer size of 0, the function stores the number of characters available in the RX buffer of the driver in the parameter *Length. In this case, Buffer can be set to NULL and the function will return DRV_OK. The line states will be delivered anyway.

### GSI_Write – Transmit data via the serial interface

Definition:

USHORT GSI_Write
(
	UBYTE	DeviceNo,
	void*	Buffer,
	USHORT*	Length,
	ULONG	State,
	ULONG	Mask
);

Parameters:

Name	Description

DeviceNo	serial device number

Buffer	This parameter points to the buffer that is passed to the driver for further processing.

Length	On call: number of characters to write.
On return: If the function returns DRV_OK, it contains the number of characters written. Otherwise it contains 0.

State	is a bit field containing the current status bits. Only the lines which are marked with the „set“-access can be used to change the state of this line. The parameter is defined in more detail along with the function GSI_Read().

Mask	is a bit field with the same structure as State. Each bit in State corresponds to a bit in Mask. Only those status bits are manipulated by the driver, which are marked by a 1 in the Mask bit field and which are settable according to the table in the specification of GSI_Read().

Return values:

Name	Description

DRV_OK	Function successful.
DRV_INVALID_PARAMS	The specified device does not exist.
DRV_INTERNAL_ERROR	Internal driver error.
DRV_NOTCONFIGURED	The device is not yet configured.

Description

This functions writes data into the driver. It copies the provided data into the buffer of the driver and sets the line states. This function must return immediately after copying, even if there is not enough space in the driver buffer.

The parameter *Length contains the number of characters to write. When the function returns the parameter *Length contains the number of written characters.

In the case of a successful completion, the function returns DRV_OK.

If the device is not configured, the function returns DRV_ NOTCONFIGURED.

NOTE:	When calling the function with a buffer size of 0, the function will return the number of characters that can be written into the TX driver buffer in the parameter *Length. In this case, Buffer can be set to NULL and the function will return DRV_OK. The line states will be set anyway.

### GSI_Look – Look at received data of the driver

Definition:

USHORT GSI_Look
(
	UBYTE	DeviceNo,
	void*	Buffer,
	USHORT*	Length,
	ULONG*	State
) ;

Parameters:

Name	Description

DeviceNo	serial device number

Buffer	This parameter points to the buffer wherein the data is to be copied.

Length	On call: Size of Buffer.
On return: If the function returns DRV_OK, it contains the number of characters read. Otherwise it contains 0.

State	Line states of the serial connection (see function GSI_Read() )

Return values:

Name	Description

DRV_OK	Function successful.
DRV_INVALID_PARAMS	The specified device does not exist.
DRV_INTERNAL_ERROR	Internal driver error.
DRV_NOTCONFIGURED	The device is not yet configured.

Description

This function reads data from the driver without delete the data in the driver buffer. It copies received data into a buffer, which is provided by the caller, and it returns the line states. It should always return immediately after copying the data, without waiting for any more data.

The parameter *Length contains the size of the buffer in characters. When the function returns the parameter *Length contains the number of copied characters.

Each copied character is still held in the buffer of the driver. Consecutive calls to GSI_Look() will lead to identical results, except may be line states. The driver only deletes the data after copy when calling the function GSI_Read().

If the device is not configured, the function returns DRV_ NOTCONFIGURED.

NOTE: When calling the function with a buffer size of 0, the function stores the number of characters available in the RX buffer of the driver in the parameter *Length. In this case, Buffer can be set to NULL and the function will return DRV_OK. The line states will be delivered anyway.

### GSI_Clear – Clear internal driver buffers

Definition:

USHORT GSI_Clear
(
	UBYTE	DeviceNo,
	USHORT	BufferType
);

Parameters:

Name	Description

DeviceNo	serial device number

BufferType	Bit-mask used to specify which buffer must be cleared

Return values:

Name	Description

DRV_OK	Function successful
DRV_INVALID_PARAMS	The specified device does not exist
DRV_INTERNAL_ERROR	Internal driver error
DRV_INPROCESS	The driver is busy clearing the buffers.

Description

This function is used to clear the device internal buffers. It should always return immediately after changing internal states and resetting internal values, without waiting for any outstanding events.

The parameter BufferType is used to specify which buffer is to be cleared. The value of BufferType can be one of the values or a combination of the values defined in [C_8415.0026]. DRV_BUFTYPE_READ and DRV_BUFTYPE_WRITE can be combined to clear the read and write buffers at once.

In the case of a successful completion, the function returns DRV_OK.

If the driver could not complete the clearance of the buffers at once the function returns DRV_INPROCESS. The driver will send a signal to the protocol stack when the buffers are cleared completly.

### GSI_Flush – Flush internal driver buffer

Definition:

USHORT GSI_Flush
(
	UBYTE	DeviceNo
);

Parameters:

Name	Description

DeviceNo	serial device number

Return values:

Name	Description

DRV_OK	Function successful.
DRV_INVALID_PARAMS	The specified device does not exist.
DRV_INTERNAL_ERROR	Internal driver error.
DRV_INPROCESS	The driver is busy flushing the buffer.
DRV_NOTCONFIGURED	The device is not yet configured.

Description

With this function the driver is requested to inform the protocol stack, when all data have been written successfully to the serial device. That means all buffers including buffer in the HW are empty. It considers only TX data. The RX direction has no influence on this function.

This function can be used by the protocol stack to ensure that no more data is to be sent (e.g. before switching between command mode and data mode or before changing the settings of the driver).

In case the TX buffers are already empty or could be written at once, the function returns DRV_OK.

If the driver could not complete flushing the buffers at once the function returns DRV_INPROCESS. The driver will send a signal to the protocol stack when the buffers are flushed completly.

If the driver is not configured, the function returns DRV_ NOTCONFIGURED.

### GSI_SetSignal – Setup a Signal

Definition:

USHORT GSI_SetSignal
(
	UBYTE	DeviceNo,
	USHORT	SignalType
);

Parameters:

Name	Description

DeviceNo	serial device number

SignalType	Signal type to be set

Return values:

Name	Description

DRV_OK	Function completed successfully
DRV_INVALID_PARAMS	One or more parameters are out of range or invalid
DRV_INTERNAL_ERROR	Internal driver error
DRV_SIGFCT_NOTAVAILABLE	Event signaling functionality is not available

Description

This function is used to define a single or multiple signals that is/are indicated to the process when the event identified in the signal information data type as SignalType occurs. The driver uses only the standard signals defined in [C_8415.0026].

To remove a signal, call the function GSI_ResetSignal().

If one of the parameters of the signal information data is invalid, the function returns DRV_INVALID_PARAMS.

If no signal callback function has been defined at the time of initialization, the driver returns DRV_SIGFCT_NOTAVAILABLE.

GSI_Read(): To inform the protocol stack about the availability of data or about a change of the line states, the driver has to call the callback function (provided with the GSI_Init() call) with the SignalType DRV_SIGTYPE_READ. The values UserData and DataLength of the T_DRV_SIGNAL stucture are not used for this signal type. The driver should call this function every time

- when the threshold (RxThreshold of T_GSI_DCB) is reached or

- when a line status is changed.

If all available data has been read out successfully, the driver returns the number of processed bytes.

driver		Callback Function		Protocol Stack

|		|		|

|	CallbackFunc	|		|

|	(DRV_SIGTYPE_READ)	|		|

*	=====================>	*	PSIGNAL	|

|		|	(READY_FOR_READ)	|

|		*	=====================>	*

|		|		|

|		GSI_Read()		|

*	<=============================================	*

|		returns No. of Bytes		|

If the provided buffer is filled completely then the callback function does not have to be called again with DRV_SIGTYPE_READ even if new data is received by the driver. In this case GSI_Read() must be called again, after space becomes available. A change of the line states however should always be reported with callback.

driver		Callback Function		Protocol Stack

|		|		|

|	CallbackFunc	|		|

|	(DRV_SIGTYPE_READ)	|		|

*	=====================>	*	PSIGNAL	|

|		|	(READY_FOR_READ)	|

|		*	=====================>	*

|		|		|

|		GSI_Read()		|

*	<=============================================	*

|		returns buffer size		|

|		|		|

|		|		|

|		GSI_Read()		|

*	<=============================================	*

|		returns No. of Bytes		|

GSI_Write(): If all data could be copied successfully, then the return value is equal to the buffer size. In this case the protocol stack may call the function at any time again, when there is new data to be sent.

driver		Callback Function		Protocol Stack

|		|		|

|		GSI_Write()		|

*	<=============================================	*

|		returns buffer size		|

|		|		|

|		GSI_WriteData()		|

*	<=============================================	*

|		returns buffer size		|

|		|		|

If the driver can not take over all data then the return value is smaller then the buffer size. In this case the driver must inform the protocol stack, when the amount of buffered data drops below the low watermark (TxThreshold of T_GSI_DCB). In order to do this the driver has to call the callback function (provided with the GSI_Init() call) with DRV_SIGTYPE_WRITE. Then the function GSI_Write() must be called again by the protocol stack. The values UserData and DataLength of the T_DRV_SIGNAL stucture are not used for this signal type.

GSI_Write() may be called by the protocol stack at any time, even if not all data has been copied at the last call and DRV_SIGTYPE_WRITE has not been signalled by the driver yet.

driver		Callback Function		Protocol Stack

|		|		|

|		GSI_Write()		|

*	<=============================================	*

|		(No. of Bytes returned		|

|		< buffer size)		|

|		|		|

|	CallbackFunc	|		|

|	(DRV_SIGTYPE_WRITE)	|		|

*	=====================>	*	PSIGNAL	|

|		|	(READY_FOR_WRITE)	|

|		*	=====================>	*

|		|		|

|		GSI_Write()		|

*	<=============================================	*

|		returns buffer size		|

|		|		|

GSI_Clear(): After all internal buffers are completely cleared the driver calls the callback function (provided with the GSI_Init() call) with DRV_SIGTYPE_CLEAR to inform the protocol stack about the successful completion. The values UserData and DataLength of the T_DRV_SIGNAL stucture are not used for this signal type.

driver		Callback Function		Protocol Stack

|		|		|

|		GSI_Clear()		|

*	<=============================================	*

|		returns DRV_INPROCESS		|

|		|		|

|	CallbackFunc	|		|

|	(DRV_SIGTYPE_CLEAR)	|		|

*	=====================>	*	PSIGNAL	|

|		|	(BUFFERS_CLEARED)	|

|		*	=====================>	*

|		|		|

GSI_Flush(): After all data is completely written to the serial interface the driver calls the callback function (provided with the GSI_Init() call) with DRV_SIGTYPE_FLUSH to inform the protocol stack about the successful completion. The values UserData and DataLength of the T_DRV_SIGNAL stucture are not used for this signal type.

The driver must call the callback function only after all data are written and all buffers including in the HW are empty.

driver		Callback Function		Protocol Stack

|		|		|

|		GSI_Flush()		|

*	<=============================================	*

|		returns DRV_INPROCESS		|

|		|		|

|	CallbackFunc	|		|

|	(DRV_SIGTYPE_FLUSH)	|		|

*	=====================>	*	PSIGNAL	|

|		|	(DATA_FLUSHED)	|

|		*	=====================>	*

|		|		|

### GSI_ResetSignal – Remove a Signal

Definition:

USHORT GSI_ResetSignal
(
	UBYTE	DeviceNo,
	USHORT	SignalType
);

Parameters:

Name	Description

DeviceNo	serial device number

SignalType	Signal type to be reset

Return values:

Name	Description

DRV_OK	Function completed successfully.
DRV_INVALID_PARAMS	One or more parameters are out of range or invalid.
DRV_INTERNAL_ERROR	Internal driver error.
DRV_NOTCONFIGURED	The device is not yet configured.
DRV_SIGFCT_NOTAVAILABLE	Event signaling functionality is not available.

Description

This function is used to remove previously set single or multiple signals. The signals that are removed are identified by the SignalType. If the provided SignalType can not be located, the function returns DRV_INVALID_PARAMS.

If the driver is not configured, the function returns DRV_ NOTCONFIGURED.

If no signal callback function has been defined at the time of initialization, the driver returns DRV_SIGFCT_NOTAVAILABLE.

### GSI_SetConfig – Setup a device configuration

Definition:

USHORT GSI_SetConfig
(
	UBYTE	DeviceNo,
	T_GSI_DCB*	DCBPtr
);

Parameters:

Name	Description

DeviceNo	serial device number

DCBPtr	Pointer to the device control block

Return values:

Name	Description

DRV_OK	Function successfully completed
DRV_INVALID_PARAMS	One or more values are out of range or invalid in that combination
DRV_INTERNAL_ERROR	Internal driver error

Description

This function is used to configure the serial device (port, transmission rate, flow control, etc). The device can be configured at any time. The parameters that can be configured are included in the device control block T_GSI_DCB. For detailed information about the contents of the device control block, refer to Chapter 3.1.1.

If any value of the configuration is out of range, not supported or invalid in combination with any other value of the configuration, the function returns DRV_INVALID_PARAMS.

Call the GSI_GetConfig() function to retrieve the driver’s configuration.

The driver needs to be configured after initialization. Only the following functions can be called while the driver is not configured: GSI_Clear(), GSI_SetSignal() and GSI_SetConfig(). All other functions return DRV_NOTCONFIGURED.

### GSI_GetConfig – Retrieve a driver configuration

Definition:

USHORT GSI_GetConfig
(
	UBYTE	DeviceNo,
	T_GSI_DCB*	DCBPtr
);

Parameters:

Name	Description

DeviceNo	serial device number

DCBPtr	Pointer to the device control block

Return values:

Name	Description

DRV_OK	Function successfully completed.
DRV_INTERNAL_ERROR	Internal driver error.
DRV_NOTCONFIGURED	The device is not yet configured.

Description

This function is used to retrieve the configuration of the serial device. The configuration is returned in the device control block to which the provided pointer DCBPtr points. For detailed information about the contents of the device control block, refer to Chapter 3.1.1.

If the driver is not configured, the function returns DRV_ NOTCONFIGURED.

### GSI_CallBack – Callback entry for the driver

Definition:

void GSI_CallBack
(
	T_DRV_SIGNAL	* Signal
);

Parameters:

Name	Description

Signal	Pointer to the signal information data

Return values:

Name	Description

-	-

Description

This function must not be confused with the parameter CallbackFunc passed to GSI_Init().

This function is only needed for cascaded drivers where the lower layer driver calls the callback function of the upper layer driver via the frame.

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |