Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

GSM Protocol Stack

GPF

xPanel – MMI Test Application (PC)

User guide

(Edit via File/Properties/Summary!)

| Document Number: | 06-03-36-UDO-005 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.5 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2000-Oct-26 |
| Last changed: | 2004-May-13 by Ronny Kiessling |
| File Name: | Xpan_userguide.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Table of Contents

0	Document Control	4

0.1	Change History	4

0.2	List of Figures and Tables	4

0.3	List of References	4

0.4	Abbreviations	5

0.5	Terms	5

1	Introduction	6

2	Application Manual	7

2.1	Environment / Installation	7

2.2	Getting started	8

2.3	Command line parameters and ini-files	9

2.4	Working with xPanel	10

2.4.1	Sending of SYSTEM-Primitives	10

2.4.2	Input of AT-Commands	10

2.4.3	Using graphical screen with color table	11

3	Layout development	12

3.1	Basics	12

3.2	Editing the template	12

4	Known problems and future tasks	13

4.1	Known bugs	13

4.2	„Soon implemented“	13

4.3	„Nice to have “	13

# Document Control

## Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2000-Oct-26 | RK et al. |  | 0.1 |  | 1 |
| 2001-Nov-02 | RK |  | 0.2 |  | 2 |
| 2002-Sep-30 | RK |  | 0.3 |  | 2 |
| 2003-May-21 | XINTEGRA |  | 0.4 | Draft |  |
| 2003-Aug-18 | RK |  | 0.5 | Draft | 3 |

Notes:

- Initial version

- Updated

- New Document ID introduced

## List of Figures and Tables

## List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |
| [GSM 2.30] | ETS 300 511: July 1995 (GSM 02.30 version 4.13.0) Man-Machine Interface (MMI) of the Mobile Station (MS), ETSI |
| [PANEL] | 8415.014.00.105, February 7, 2000, Panel – PC Test Application |
| [XPAN+] | xPanel_plus_design_spec.doc |
| [XPAN_DD] | 06-03-36-SLL, xPanel Developer Description (xpan_description.doc) |
| [MOAN] | 06-03-53-UDO, MoanBtn – Instant GUI-problem Informer (mbtn_userguide.doc) |
| [XM] | 06-03-55-UDO, XM –GUI-frontend for GPF m.bat (xm_userguide.doc) |
| [PCO2] | 06-03-35-UDO, PCO2 – Tracing Environment (pco_userguide.doc) |

## Abbreviations

ACI 	Application Control Interface (AT Commands)

G23	The Condat implementation of Layers 2 and 3 of the GSM Protocol Stack

G23 Target System	Hardware which executes G23

LCD	Liquid Crystal Display

MM	Mobility Management

MMI	Man Machine Interface

MOC	Mobile Originated Call

MTC	Mobile Terminated Call

PC	Personal Computer

PCO	Point of Control and Observation

PIN	Personal Identification Number

RS232	Serial Communication Standard

Target System	Shortened form of 'G23 Target System'

## Terms

Entity	Program which executes the functions of a layer

Message	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Introduction

- G23 is a software package implementing Layers 2 and 3 of the ETSI-defined GSM air interface signaling protocol, and as such represents the part of a GSM mobile station's protocol software which is both, platform and manufacturer independent. Therefore, G23 can be viewed as a building block providing standardized functionality through generic interfaces for easy integration.

The G23 suite of products consists of the following items:

- Layers 2 and 3 for speech & short message services,

- Layers 2 and 3 for fax & data services,

- Application Control Interface/AT Command Interface,

- MMI  and MMI Framework (MFW) and

- Test and integration support tools.

This userguide is meant to give an introduction of the functionality of xPanel and to explain several important issues in detail. It is based on the assumption that the user is familiar with the basic operation principles of mobile phones. The operation is implemented in accordance with the SMI (Slim Man Machine Interface).

The new eXtended panel has the functionality of the former „Panel“ (see [PANEL]) and the „Gameboy“ insight. That means in brief it provides a mobile-like keypad and a screen for display response. Furthermore the layout is configurable. Customers themselves should be able to do configuration and creation of new layout files (see 3).

The current xpanel.exe supports serial port communications and simulated communications using shared memory to access a stack.

Unlike the old panel xPanel does not include a PCO for watching traces anymore. To do that you have to use the PCO2 environment (see [PCO2]) together with xPanel.

XPanel does also include moan button support (see [MOAN]), if you got the moanbtn.dll with the package (internal CONDAT workers with ClearCase automatically get it).

# Application Manual

## Environment / Installation

To use xPanel some environmental constraints have to be taken into account.

For use under Windows:

At first you’ll have to make sure that several DLL-files are available to the system. In the Condat development directory structure you can find them in „<View>/GPF/Bin” and „<View>/GPF/tools/bin”:

- Frame-DLLs (frame.dll, tif.dll, misc.dll)

- cms.dll, ipc.dll

- tk84.dll, tcl84.dll

So just make sure „<View>/GPF/Bin” and “<View>/GPF/gnu/bin” are in your PATH-variable. (use e.g. the “initvars.bat” in „<View>/GPF”)

From the MS-Explorer you can directly double click on the “<View>/GPF/bin/xpan.bat”, which will solve the PATH-problem for you.

Furthermore the Tcl-interpreter expects some files in subdirectories of a lib-folder relative to the start-directory -> “../lib”. At Condat all the necessary files can be found in “<View>/GPF/Lib” :

- “tk8.4”-directory

- “tcl8.4”-directory

At least you need to have xpanel.tcl in the same directory as the executable.

For your special needs you may have to change the default ini-file of xPanel (“<view>/GPF/cfg/xpan.ini”). An explanation of the options is given in 2.3. 
In “<view>/GPF/cfg” you can find some example ini-s like one for the usage with the TI-Multiplexer (“<view>/GPF/cfg/xpan.ini.ti_mux”).

To start meaningful work you additionally need a layout-file –adapted to your needs (see 2.3 and 3.1 for layout usage details). You should use PCO (see [PCO2]) together with xPanel, to watch traces and redirected primitives.

## Getting started

To start xpanel the first time just start your protocol stack and call “xpanel” (or click “xpan.bat”). Now you should see the xPanel-window with at least a menu. If no layout has been loaded, you can select “file/load layout …” and select one.

(there are some example-layouts in “<View>/GPF/cfg” and “<View>/GPF/util/xpanel/layouts”)

Now you can press the buttons and if communication with stack works its display requests should result in a new screen-picture in the xPanel-window.

If not you might have to switch the display-type (textual, graphical or off).

In the “Display” menu you can set the size of the graphical screen, too.

If communication doesn’t work you can try to change communication mode from the “Config”-menu. Therefore you have to enable test interface configuration at first. (Per default xPanel does not configure it but lets this work do other tools like PCO)

Then the mode may be chosen inside a dedicated dialog:

Currently the Frame supports Sockets, real USART using a COM-port and simulated USART via shared memory. For each mode you can specify special parameters and decide, whether PCON shall be used (if your stack is using PCON for communication)

For real USART you may e.g. select the COM-port to use and more parameters like baudrate and flow control value.

All parameters can be specified in the ini-file, too (see 2.3).

If communication over serial port stops working you can try to reset the serial driver from the “Config”-menu. Another possibility is to restart the whole xPanel application.

All settings you choose while working with xPanel (even the position of the window) will be stored in the used ini-file (see 2.3) when you exit xPanel.

## Command line parameters and ini-files

Inside a ClearCase vob the convenience batch file xpan.bat may be used with command line parameters as follows:

- h	… display all parameters

- tst_com{1|2}	… load default configuration files for communication via test interface on com port 1/2

- at_com{1|2}	… load default configuration files for communication via at interface on com port 1/2

- sim	… load default configuration files for communication via shared memory

- ti_mux	… load default configuration files for communication via TI multiplexer

At the command line of xpanel.exe itself you can specify some environmental options for xPanel. You can select between the following possibilities:

- -h 	-> usage-message

- -l <layout name>	-> program will use the specified layout (see 3.1)

- -m <usart-mode>	-> program will use the specified usart mode (REAL|SIM)

- -i <ini file>	-> program will parse the specified ini file

- -s[end] <key code> [<press time in ms>]
-> program will send specified key stroke to the test interface (finally the PS) … optionally the milliseconds between press/release can be specified

- -c[lean]	-> cleans the text display

If you don’t give any parameters the default ini-file “xpan.ini” will be parsed if one exists (searched in current directory and “..\cfg”). In an ini-file only lines of the following format will be interpreted:

<parameter><whitespace><value>

An overview of the available parameters follows:

- layoutdir	<layout directory>

- layout	<layout name> (see 3.1)

- initpri	<file with initial primitives to execute at beginning> (see )

- zoom	<zoom level> (the higher the bigger the display will be)

- set-comm	{0|1} (specify here if xpanel shall set tst configurations)

- STX		<STX-mode> (0 or 1)

- comport	<COM-port number>

- usartmode	{REAL|SIM|SOCKET}

- hostname	<name of the host for socket connection> (can be empty)

- socket_port	<port for socket connection> (default: 20612)

- flowctrl	Defines the mode of flow control. The characters 'N', 'R', 'D', 'P' and 'V' are accepted.

| <control flow> | Type of flow control |
| --- | --- |
| N | None |
| R | RTS/CTS |
| D | DTR/DSR |
| P | None, DTR line enabled, serial interface provides power supply for external devices |
| V | None, DTR and RTS lines enabled, serial interface provides power supply for external devices |

- windim	.. set automatically

- palette	.. file containing a color table (see 2.4.3)

- pcon	<PCON-mode> (0 or 1)

- extdspl_data_frequency
 		.. frequency in ms xPanel will request graphical display data (see [XPAN+])

You can find examples in the standard xpan.ini files in gpf/cfg or in gpf/util/xpanel/cfg.

## Working with xPanel

### Sending of SYSTEM-Primitives

In the “Cmd”-menu you have to points for sending SYSTEM-primitives – either directly or by loading them from a file.

A SYSTEM-primitive consists of a receiver and a primitive body. You can input both directly by choosing the Menu entry ”Cmd/Input primitve …“.

Under ”Cmd/Load primitve …“ it is possible to load primitives from an ASCII-file formatted like this:

<receiver>

<prim body>

<receiver>

<prim body>

:

For example:

MMI

CONFIG KEY_SEQUENCE= LEFT

MMI

CONFIG KEY_SEQUENCE= DOWN

### Input of AT-Commands

In the “Cmd”-menu you have to points for sending AT-commands – either directly or by loading them from a file.

An AT-command is just a string like “AT+…”.

You can input such a line directly by choosing the Menu entry „Cmd/Input AT command …“.

Under „Cmd/Load AT commands …“ it is possible to load AT-lines from an ASCII-file formatted like this:

<at-string>

<at-string>

<at-string>

:

For example:

AT+CSCA=491710760000*145

AT+CMGS=491712002400*145*Test__das_ist_MO_SMS

### Using graphical screen with color table

XPanel supports the display of graphical screen informations received from the the connected target. See [XPAN+] for detailed information about the supported interface.

Depending on the used bits per pixel it either takes colors from a color table or calculates true color values (bpp>=16, see [XPAN+] for detailed description of bit-usage). A color table can be specified in the ini-file (see 2.3). Currently only the JASC-format is supported as it can be exported e.g. from the PaintShop application. Such a „.pal“-file is just an ASCII-file of the following style:

JASC-PAL	… fix string

0100	… fix number

<count>	… count of colors in file (e.g. 256)

<RGB-entry 0>

<RGB-entry 1>	… -count- entries with RedGreenBlue-information separated by blanks (e.g. 0 255 23)

:

<RGB-entry count>

All numbers are in decimal format.

If no color table is specified xPanel will try to use „default256.pal“.

If no table can be found anyway, levels of grey (between RGB(0,0,0) and RGB(255,255,255)) will be used.

# Layout development

## Basics

To develop a special mobile layout you have to get detailed informations about all the keys – their location, the text/graphics on them and the associated function.

The next step is to modify the template tcl-file (xpan_lo_tmpl.tcl) and rename it to <name>_lo.tcl where <name> stands e.g. for the name of the company you develop the layout for.

Now this name can just be passed to xPanel as a parameter (e.g. xpan –l <name>) or specified in your ini-file (see 2.3) and the new layout will be used. Another option is to load the layout from the menu of xPanel (file/load layout …).

## Editing the template

After copying the „xpan_lo_tmpl.tcl” to “<name>_lo.tcl” you can modify the constants in the following parts of the layout file:

- Display colors

Here you can specify the fore-and background colors of the text display.

- Key count

Here you can specify the count of keys in a row/column of your mobile.

- Key IDs

Here you can specify the mapping of key names to IDs. Usually no changes are necessary.

- key distribution

Here you can specify how the key are distributed on your mobile. Where no key exists use $KEY_NOKEY.

- Key labels

Here you can specify strings as labels of your keys. Where no key exists or where you want to provide an image use an empty string.

- Optional Key images

Here you can specify names of image files (Bitmaps) which will be uses instead of a string as labels of your keys. Where no key exists or where you want use a label string specify an empty string.

- Key sizes

Here you can specify the sizes of your keys. Just look at the given examples and “trial and error” until you are satisfied. Where no key exists use “0” (it will be ignored anyway).

- Key positions

Here you can specify the relative positions of your keys using sticky-strings. You can choose from “w” (west), “e” (east), “n” (north) and “s” (south) to indicate that a key should stick at a side, or you can center it by using “”. Where no key exists use an empty string (it will be ignored anyway).

- Key reliefs

Here you can specify the relief type of the keys. One of the following constants may be used: raised, sunken, flat, ridge, solid, groove. Just try and decide what you like.

# Known problems and future tasks

This paragraph is meant to show which bugs are already found (but not removed yet) and to provide an impression of future plans concerning this product.

## Known bugs

- XPanel does not remember its position/size correctly sometimes

## „Soon implemented“

- the file dialogs should „remember“ the last chosen path

- Online-Help

## „Nice to have “

- more extensive possibilities of command execution – more automatization

- a recording of button-presses shall be possible

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |