Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Documentation

Protocol Stack

GPF

SNS – SAP Numbering Scheme

(Edit via File/Properties/Summary!)

| Document Number: | 89_03_00_00684 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.18(Edit via File/Properties/Summary!) |
| Status: | (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2001-Aug-22 |
| Last changed: | 2005-May-02 by Ronny Kiessling |
| File Name: | sap_numbering_scheme.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2001-Aug-22 | MP et al. |  | 0.1 |  | 1 |
| 2002-Apr-18 | SBK |  | 0.2 |  | 2 |
| 2002-Sep-30 | MP |  | 0.3 |  | 3 |
| 2002-Nov-15 | MP |  | 0.4 |  | 4 |
| 2002-Nov-18 | FR |  | 0.5 |  | 5 |
| 2002-Nov-28 | MP |  | 0.6 |  | 6 |
| 2003-May-20 | XINTEGRA |  | 0.7 | Draft |  |
| 2003-Sep-08 | MP |  | 0.8 |  | 7 |
| 2003-Dec-15 | MP |  | 0.9 |  | 8 |
| 2004-Mai-13 | HCH, SIJ |  | 0.10 |  | 9 |
| 2004-06-08 | SIJ |  | 0.11 |  | 10 |
| 2004-Jul-02 | SPRK |  | 0.12 |  | 11 |
| 2004-Jul-14 | SIJ |  | 0.13 |  | 12 |
| 2004-Jul-20 | MP |  | 0.14 |  | 13 |
| 2004-Aug-25 | MP |  | 0.15 |  | 14 |
| 2004-Oct-05 | KSP |  | 0.16 |  | 15 |
| 2005-Feb-05 | ROZ |  | 0.17 |  | 16 |
| 2005-Apr-21 | MP |  | 0.18 |  | 17 |

Notes:

- Initial version

- Replace 16bit opcodes by 32bit opcodes for GSM voice&sms where possible (no “external” SAPs)

- New opcodes for EOTD and new SAP PKTIO Replace 16bit opcodes by 32bit opcodes for GSM fax&data

- New opcode for UDP

- New opcode for AAA

- Update UMTS SAP numbers

- New SAP DMI, TCPIP, DCM

- New SAP CL

- Added: GPSDRV, GPSM and GPSC SAP

- Updated document number, change-history and page titles

- New SAPs: L2TEST; L1/ESF-Mapping-SAPs; Virtual SAPs for tracing

- Added L1/ESF; changed number for L2TEST; Updated to the status of umts and  added GMMAA, DRRC, MPHI, SN, UPM, AUDIO SAP and removed RRRRC and GRRRC.

- Added flag description in 32bit opc for bit 30 and 29

- TCSD and PSI  added

- FFS SAP added.

- ATLAS SAP’s added

- ETM SAP added

Table of Contents

1.1	Abbreviations	4

3.1	G23/GPRS	6

3.2	G23Net	7

3.3	UMTS	9

A.	Acronyms	11

B.	Glossary	11

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## Abbreviations

SAP 	Service Access Point

OPC	Operation Code

# Introduction

Currently the opc in the primitive header consists of 16 bits. They have the following meaning:

| SysPrim | UL/DL | SAP | Primitive Number |
| --- | --- | --- | --- |
| Bit 15 | Bit 14 | Bit 8...13 | Bit 0...7 |

SysPrim = 0		Protocol Primitives

SysPrim = 1		System Primitives

UL/DL = 0		uplink, that means in the direction to lower layers  (XXX_REQ and XXX_RES)

UL/DL = 1		downlink, that means in the direction to the upper layers (XXX_IND and XXX_CNF)

SAP			XX decimal entity number (see following table)

Primitive Number	incrementing index of the primitives for one SAP, separated by uplink and downlink, starting with 0

The SAP number coding with 6 bits will not be sufficient for future applications. Therefor the opc will be extended to 32 bit corresponding to the following scheme. The meaning of the lower 16 bits will remain unchanged to be downwards compatible.

| Flags | Reserved | Primitive Number | SysPrim | UL/DL | SAP |
| --- | --- | --- | --- | --- | --- |
| Bit 31...26 | Bit 25...24 | Bit 23...16 | Bit 15 | Bit 14 | Bit 13...0 |

Flags Bit 31 = 1		32 bit opc used

Flags Bit 31 = 0		16 bit opc used

Flags Bit 30 = 1		virtual opc (GPF internal, not passed through PCON)

Flags Bit 30 = 0		‘normal opc’

Flags Bit 29 = 1		primitive contains MEM entity handle

Flags Bit 29 = 0		primitive does not contain MEM entity handle

Flags Bit 28…26	unused

SysPrim = 0		Protocol Primitives

SysPrim = 1		System Primitives

UL/DL = 0		uplink, that means in the direction to lower layers  (XXX_REQ and XXX_RES)

UL/DL = 1		downlink, that means in the direction to the upper layers (XXX_IND and XXX_CNF)

SAP			XX decimal entity number (see following table)

Primitive Number	incrementing index of the primitives for one SAP, separated by uplink and downlink, starting with 0

# SAP Numbers

## G23/GPRS

Currently for some SAPs (mainly GPRS) 16 bit opc is used whilst for others 32 bit opc is used.

| SAP Number | SAP Name |  | opc/UL(hex) | opc/DL(hex) |
| --- | --- | --- | --- | --- |
| 0 | MPHC |  | 0x00xx |  |
| 1 | PH |  | 0x01xx | 0x41xx |
| 2 | MPHP |  | 0x02xx |  |
| 3 | DL |  | 0x80xx0003 | 0x80xx4003 |
| 4 | MDL |  | 0x80xx0004 | 0x80xx4004 |
| 5 | SIM |  | 0x80xx0005 | 0x80xx4005 |
| 6 | RR |  | 0x80xx0006 | 0x80xx4006 |
| 7 | MMCC |  | 0x80xx0007 | 0x80xx4007 |
| 8 | MMSS |  | 0x80xx0008 | 0x80xx4008 |
| 9 | MMSMS |  | 0x80xx0009 | 0x80xx4009 |
| 10 | MMREG |  | 0x80xx000A | 0x80xx400A |
| 11 | MNCC |  | 0x80xx000B | 0x80xx400B |
| 12 | MNSS |  | 0x80xx000C | 0x80xx400C |
| 13 | MNSMS |  | 0x80xx000D | 0x80xx400D |
| 14 | MMI |  | 0x0Exx | 0x4Exx |
| 15 | MON |  | 0x0Fxx |  |
| 16 | RA |  | 0x80xx0010 | 0x80xx4010 |
| 17 | RLP |  | 0x80xx0011 | 0x80xx4011 |
| 18 | L2R |  | 0x80xx0012 | 0x80xx4012 |
| 19 | FAD |  | 0x80xx0013 | 0x80xx4013 |
| 20 | T30 |  | 0x80xx0014 | 0x80xx4014 |
| 21 | ACI |  | 0x15xx | 0x55xx |
| 22 | CST |  | 0x16xx | 0x56xx |
| 23 | MPH |  | 0x17xx | 0x57xx |
| 24 | TB |  | 0x18xx |  |
| 25 | TRA |  | 0x80xx0019 | 0x80xx4019 |
| 26 | DMI |  | 0x80xx001A | 0x80xx401A |
| 27 | IDA |  | 0x80xx001B | 0x80xx401B |
| 28 | DCM |  | 0x80xx001C | 0x80xx401C |
| 29 | PSI |  | 0x80xx001D | 0x80xx401D |
| 30 | FRM |  | 0x80xx001E | 0x80xx401E |
| 31 | GMMRR |  | 0x1Fxx | 0x5Fxx |
| 32 | GRR |  | 0x20xx | 0x60xx |
| 33 | LLGMM |  | 0x21xx | 0x61xx |
| 34 | LL |  | 0x22xx | 0x62xx |
| 35 | GMMSMS |  | 0x23xx | 0x63xx |
| 36 | GMMSM |  | 0x24xx | 0x64xx |
| 37 | GMMAA |  | 0x25xx | 0x65xx |
| 38 | SMREG |  | 0x26xx | 0x66xx |
| 39 | SNSM |  | 0x27xx | 0x67xx |
| 40 | SN |  | 0x28xx | 0x68xx |
| 41 | GSIM, to be removed |  | 0x29xx | 0x69xx |
| 42 | TCSD |  | 0x80xx002A | 0x80xx402A |
| 43 | ETM |  | 0x80xx002B | 0x80xx402B |
| 44 | unused |  |  |  |
| 45 | RRGRR |  | 0x2Dxx | 0x6Dxx |
| 46 | MMGMM |  | 0x2Exx | 0x6Exx |
| 47 | unused |  |  |  |
| 48 | unused |  |  |  |
| 49 | unused |  |  |  |
| 50 | MAC |  | 0x32xx | 0x72xx |
| 51 | GMMREG |  | 0x33xx | 0x73xx |
| 52 | UART |  | 0x34xx | 0x74xx |
| 53 | PPP |  | 0x35xx | 0x75xx |
| 54 | CCI |  | 0x36xx | 0x76xx |
| 55 | DTI |  | 0x37xx | 0x77xx |
| 56 | PPC |  | 0x38xx | 0x78xx |
| 57 | TOM/IP |  | 0x39xx | 0x79xx |
| 58 | BTP |  | 0x3Axx | 0x7Axx |
| 59 | UDPA |  | 0x3Bxx | 0x7Bxx |
| 60 | IPA |  | 0x3Cxx | 0x7Cxx |
| 61 | WAP |  | 0x3Dxx | 0x7Dxx |
| 62 | EM |  | 0x3Exx | 0x7Exx |
| 63 | EXTDSPL/GTI |  | 0x3Fxx | 0x7Fxx |
| 64 | RRLC |  | 0x80xx0040 | 0x80xx4040 |
| 65 | RRRRLP |  | 0x80xx0041 | 0x80xx4041 |
| 66 | RRLP |  | 0x80xx0042 | 0x80xx4042 |
| 67 | CSRLC |  | 0x80xx0043 | 0x80xx4043 |
| 68 | MNLC |  | 0x80xx0044 | 0x80xx4044 |
| 69 | PKTIO |  | 0x80xx0045 | 0x80xx4045 |
| 70 | UDP |  | 0x80xx0046 | 0x80xx4046 |
| 71 | AAA |  | 0x80xx0047 | 0x80xx4047 |
| 72 | TCPIP |  | 0x80xx0048 | 0x80xx4048 |
| 73 | GPSDRV |  | 0x80xx0049 | 0x80xx4049 |
| 74 | GPSM |  | 0x80xx004A | 0x80xx404A |
| 75 | GPSC |  | 0x80xx004B | 0x80xx404B |
| 76 | L2TEST |  | 0x80xx004C | 0x80xx404C?? |
| 77 | unused |  |  |  |
| 78 | unused |  |  |  |
| 79 | unused |  |  |  |
| 80 | customer 6379 |  | 0x80xx0050 | 0x80xx4050 |
| 81 | customer 6379 |  | 0x80xx0051 | 0x80xx4051 |
| 82 | customer 6379 |  | 0x80xx0052 | 0x80xx4051 |
| 83 | customer 6379 |  | 0x80xx0053 | 0x80xx4053 |
| 84 | customer 6379 |  | 0x80xx0054 | 0x80xx4054 |
| 85 | customer 6379 |  | 0x80xx0055 | 0x80xx4055 |
| 86 | customer 6379 |  | 0x80xx0056 | 0x80xx4056 |
| 87 | customer 6379 |  | 0x80xx0057 | 0x80xx4057 |
| 88 | customer 6379 |  | 0x80xx0058 | 0x80xx4058 |
| 89 | customer 6379 |  | 0x80xx0059 | 0x80xx4059 |
| 90 | customer 6379 |  | 0x80xx005A | 0x80xx405A |
| ... |  |  |  |  |

## G23Net

Currently only 16 bit opc is used.

| SAP Number | SAP Name | opc(16bit)/UL(hex) | opc(16bit)/DL(hex) |
| --- | --- | --- | --- |
| 2 | MMCC | 0x02xx | 0x42xx |
| 3 | RM | 0x03xx | 0x43xx |
| 4 | MMSS | 0x04xx | 0x44xx |
| 6 | RR | 0x06xx | 0x46xx |
| 7 | MNCC | 0x07xx | 0x47xx |
| 8 | MNSS | 0x08xx | 0x48xx |
| 9 | MNMM | 0x09xx | 0x49xx |
| 10 | LAPD | 0x0Axx | 0x4Axx |
| 11 | HO | 0x0Bxx | 0x4Bxx |
| 13 | OMHO | 0x0Dxx | 0x4Dxx |
| 14 | OMRM | 0x0Exx | 0x4Exx |
| 15 | OMRMBTSM | 0x0Fxx | 0x4Fxx |
| 16 | OMMM | 0x10xx | 0x50xx |
| 17 | OMCC | 0x11xx | 0x51xx |
| 18 | MMSM | 0x12xx | 0x52xx |
| 19 | MNSMS | 0x13xx | 0x53xx |
| 20 | SMRL | 0x14xx | 0x54xx |
| 21 | SMGT | 0x15xx | 0x55xx |
| 22 | SMGVM | 0x16xx | 0x56xx |
| 23 | OMSMC | 0x17xx | 0x57xx |
| 24 | OMSMR | 0x18xx | 0x58xx |
| 25 | OMSMG | 0x19xx | 0x59xx |
| 26 | CBS | 0x1Axx | 0x5Axx |
| 27 | RA | 0x1Bxx | 0x5Bxx |
| 28 | RLP | 0x1Cxx | 0x5Cxx |
| 29 | L2R | 0x1Dxx | 0x5Dxx |
| 30 | DTI | 0x1Exx | 0x5Exx |
| 31 | IM | 0x1Fxx | 0x5Fxx |

## UMTS

32 bit opc will be used:

| SAP Number | SAP Name | opc(32bit)/UL(hex) | opc(32bit)/DL(hex) |
| --- | --- | --- | --- |
|  | CPHY | 0x80xx0080 | 0x80xx4080 |
|  | PHY | 0x80xx0081 | 0x80xx4081 |
|  | CUMAC | 0x80xx0082 | 0x80xx4082 |
|  | UMAC | 0x80xx0083 | 0x80xx4083 |
|  | CRLC | 0x80xx0084 | 0x80xx4084 |
|  | RLC | 0x80xx0085 | 0x80xx4085 |
|  | CBM | 0x80xx0086 | 0x80xx4086 |
|  | HC | 0x80xx0087 | 0x80xx4087 |
|  | CPDCP | 0x80xx0088 | 0x80xx4088 |
|  | PDCP | 0x80xx0089 | 0x80xx4089 |
|  | RRC | 0x80xx008A | 0x80xx408A |
|  | MEM | 0x80xx008B | 0x80xx408B |
|  | DRRC | 0x80xx008C | 0x80xx408C |
|  | MPHI | 0x80xx008D | 0x80xx408D |
|  | RCM | 0x80xx008E | 0x80xx408E |
|  | GMMRABM | 0x80xx008F | 0x80xx408F |
|  | SM | 0x80xx0090 | 0x80xx4090 |
|  | PMMSMS | 0x80xx0091 | 0x80xx4091 |
|  | PHYSTUB | 0x80xx0092 | 0x80xx4092 |
|  | PHYTEST | 0x80xx0093 | 0x80xx4093 |
|  | MMREG | 0x80xx0094 | 0x80xx4094 |
|  | MMCM | 0x80xx0095 | 0x80xx4095 |
|  | MMPM | 0x80xx0096 | 0x80xx4096 |
|  | GRLC | 0x80xx0097 | 0x80xx4097 |
|  | CGRLC | 0x80xx0098 | 0x80xx4098 |
|  | EINFO | 0x80xx0099 | 0x80xx4099 |
|  | SL2 | 0x80xx009A | 0x80xx409A |
|  | L1TEST | 0x80xx009B | 0x80xx409B |
|  | CL or COMLIB | 0x80xx009C | 0x80xx409C |
|  | UPM | 0x80xx009D | 0x80xx409D |
|  | SN | 0x80xx009E | 0x80xx409E |
| 159 | AUDIO | 0x80xx009F | 0x80xx409F |
| 160 | FFS | 0x80xx00A0 | 0x80xx40A0 |
| ... |  |  |  |
| 1024 | range reserved | 0x80xx0400 | 0x80xx4400 |
| To | for |  |  |
| 2047 | L1/ESF | 0x80xx04FF | 0x80xx44FF |
| 2303 | range reserved | 0x80xx08FF | 0x80xx48FF |
| To | for |  |  |
| 23559 | Atlas (DIN) | 0x80xx09FF | 0x80xx49FF |

## Virtual SAPs

32 bit opc will be used:

| SAP Number | SAP Name | opc(32bit)/UL(hex) | opc(32bit)/DL(hex) |
| --- | --- | --- | --- |
| 2048 | Binary Traces | 0xC0xx0800 | 0xC0xx4800 |
| 2049 | ASCII Traces | 0xC0xx0801 | 0xC0xx4801 |
| 2050 | PCON-Filter | 0x80xx0802 | 0x80xx4802 |

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |