Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

High Level Design Description

Secure ME Personalization - Architecture overview

(Edit via File/Properties/Summary!)

| Document Number: | 8462.747.05.001 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 1.0 (Edit via File/Properties/Summary!) |
| Status: | Approved (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2005-Jul-22 |
| Last changed: | 2005-Oct-31 by Oleksiy Kulish |
| File Name: | input.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2005-Jul-22 | Oleksiy Kulish |  | 0.1 | Draft | 1 |
| 2005-Oct-17 | Oleksiy Kulish |  | 0.2 | Draft |  |
| 2026-Jul-28 | Oleksiy Kulish | Thomas Luettig | 1 | Approved |  |

Notes:

- Initial version

Table of Contents

Secure ME Personalization - Architecture overview	1

List of Figures and Tables	5

List of References	5

1	Introduction	6

2	Functional requirements	6

3	Architecture	12

3.1	Current architecture	12

3.2	Enhanced Architecture	12

3.3	Platform Security Implementation	14

4	Use Cases	15

4.1	MMI Use cases	15

4.2	ACI Use Case	19

4.3	Modem Security Driver	20

5	MEPD Structure	23

Appendices	25

A.	Acronyms	25

B.	Glossary	25

- List of Figures and Tables

- List of References

- This is just an example!:

| 1. | ETSI TS 122 022 V3.1.0 (2000-07) Personalization of Mobile Equipment (ME); Mobile functionality specification (3G TS 22.022 V3.1.0 Release 1999) |
| --- | --- |
| 2. | ETSI TS 127 007 V3.9.0 (2001-06) AT command set for User Equipment (UE) (3G TS 27.007 version 3.9.0 Release 1999) |
| 3. | ETSI TS 151 011 V4.11.0 (2004-03) Specification of the Subscriber Identity Module -Mobile Equipment (SIM-ME) interface (3GPP TS 51.011 version 4.11.0 Release 4) |
| 4. | 8462.600.02 - ACI – ME PERSONALIZATION INTERFACE DESCRIPTION |

# Introduction

As per end of 2004 TI protocol stack (e.g. TCS2.1 2.1.1.8) contains a very basic personalization module (SIM Lock module), which covers only requirements, described by [1]. It was mainly caused by the fact that many ME manufacturers have very specific HW, SW, security requirements. Thus TI was supplying only default SIM Lock implementation, which should serve as an example for implementation of customized SIM Lock.

To avoid customer problems in future and make TI Stack more attractive it was decided to implement powerful SIM Lock module which covers most use cases, provides high level of security against intrusion and can be completely used by customers without or with minor changes

Proposed implementation is based on requirements, collected from [1] and different customer requests, coming from previous issues. Some of requirements are shown in Chapter 2.

# Functional requirements

Functional requirements are shown in Table 2.1

| # | Requirement | Comments |
| --- | --- | --- |
| TR-0 | General Requirements |  |
|  | ME shall be fully compatible to all mandatory requirements of 22.022 (v.5.0.0. R. 5) |  |
| TR-1 | Personalization Categories |  |
| TR1.1 | NW, NS, SP, CP, SIM Personalization shall be supported |  |
| TR-1.2 | Auto personalization shall be supported |  |
| TR-1.3 | Extension to Network Subset personalization shall be supported: |  |
|  | - subset code can be defined by two other consecutive digits from IMSI | e.g. it is configured that instead of 6th & 7th, the 7th & 8th shall be checked |
|  | - instead of one network subset code, it is possible to define an interval of authorized network subset codes |  |
|  | - the 8th digit of the IMSI can be associated with a network subset code or a network subset code interval (defined by 6th and 7th) | e.g. if it is specified that 6th& 7th are in range (20..30) for 8th = 5, imsi 11111255xxxx is accepted, 11111111xxxx is not |
| TR-1.4 | Extension to Service Provider personalization shall be supported: |  |
|  | - SP code can be defined by the first two, three or four bytes of the GID1 file |  |
|  | - interval of authorized SP code shall be supported |  |
| TR-1.5 | Extension to Corporate Personalization shall be supported |  |
|  | - CP code can be defined by the first two, three or four bytes of the GID2 file |  |
|  | - interval of authorized CP code shall be supported |  |
| TR-1.6 | The ME may optionally be personalized to multiple networks, network subsets, SP’s, Corporate, SIMs or any combinations thereof | it means, number of IMSIs or other personalization code shall be >1 (15 in current implementation) |
| TR-2 | Auto Personalization |  |
| TR-2.1 | Auto Personalization shall be supported |  |
| TR-2.2 | Test SIMs shall not be treated as a candidate for Auto Lock | It shall never be possible to personalize ME to test SIM |
| TR-2.3 | All categories defined in TR-1.1 shall be available for auto Personalization |  |
| TR-2.4 | Auto Personalization shall be configurable | e.g. Lock ME on NW and SP |
| TR-2.5 | Configuration of Auto Personalization shall be a part of MEPD and shall not be changed by the user | Configuration shall be done by config tool only and stored in MEPD. No functional interface for changing of configuration shall be provided |
| TR-3 | Personalization Verification |  |
| TR-3.1 | Personalization shall be checked for every supported category every time ME is powered on or new SIM is inserted |  |
| TR-3.2 | If personalization check fails even for one category ME goes to limited mode (Emergency Calls). Otherwise to full mode |  |
| TR-3.4 | If personalization fails user shall automatically be asked to enter keys for each category | That shall be done within MMI. API will be provided for personalization status requesting |
| TR-3.5 | CPIN? Shall give currently used lock (first found) and request entering required control key. Entering correct key shall unlock ME for one session (till next restart) |  |
| TR-3.4 | If SIM Personalization category is verified, IMSI is only accepted if NW and NS Personalization passes | e.g. if NW and/or NS locks are set, they must be checked first, then if check passes, SIM is checked |
| TR-4 | Test SIM |  |
| TR-4.1 | Test SIMs shall be treated - always accepted, always rejected and accepted until once normal SIM is inserted. ME must be configurable for all three Test SIM behaviour models |  |
|  | - always accepted - no check is done |  |
|  | - always rejected - if any lock category is active, Test SIM will be rejected |  |
|  | - accepted until normal SIM is inserted - as soon as normal SIM is inserted, autopersonalization applied to ME |  |
| TR-4.2 | CPHS Type test SIMs (IMSI starts with 00101) acceptance shall be configurable | Accepted/Rejected |
|  | - - Always accepted : there is no SIM-ME-LOCK check. The ME is never blocked on a category. |  |
|  | - - Always rejected : a SIM-ME-LOCK check is always performed. The ME goes in normal mode only if all personalized categories are unblocked |  |
| TR-4.3 | Configuration of Test SIM behaviour shall be a part of MEPD and shall not be changed by the user | Configuration shall be done by config tool only and stored in MEPD. No functional interface for changing of configuration shall be provided |
| TR-5 | Personalization Process |  |
| TR-5.1 | Personalization cycle types according to ETSI spec shall be supported (via keypad, SIM info, manufacturer defined etc) |  |
|  | - personalization over modem SW shall be provided | standard CLCK commands |
|  | - personalization over direct access to MEPD with help of config Tool shall be provided | manufacturer personalization |
|  | - API for personalization over the user interface shall be provided |  |
| TR-5.2 | It shall be possible to personalize ME to all categories by entering a new key | ETSI behavior |
| TR-5.3 | It shall be possible to personalize ME to any category, which already contains PD and key by asking user to enter a key. Personalization succeeds if entered and stored keys match. If no key was stored, entered key shall be stored without comparison. | Proprietary behavior |
| TR-5.4 | Configuration of Personalization process behaviour shall be a part of MEPD and shall not be changed by the user | Configuration shall be done by config tool only and stored in MEPD. No functional interface for changing of configuration shall be provided |
| TR-5.5 | API shall provide the possibility to personalize ME only on standard categories. All extended categories are treated as standard | e.g. lock on range, on 7th&8th digits shall be locked/unlocked as NS |
| TR-5.6 | While executing SIM Personalization, new IMSI shall be added to the list of SIM personalization code group | e.g. user sets up anti theft, protection the IMSI of currently inserted SIM is added to MEPD |
| TR-5.7 | It shall not be possible to personalize ME if no SIM card inserted |  |
| TR-6 | De-personalization Process |  |
| TR-6.1 | De-personalization for every single category shall be supported | API for all de-personalization categories shall be provided |
| TR-6.2 | De-personalization for combined category shall be supported | e.g. If ME was personalized to combined categories, unlocking one of them shall automatically mean unlocking another. It is not secure and disagrees TS22.022 - thus unlocking all categories with one PW would mean that PW is same for all categories. But 22.022 specifies that PW shall be unique. |
|  | - slave categories preserve their own control key and can be independently locked or unlocked by the user |  |
|  | - slave categories (SP or C) cannot be independently locked/unlocked by the user |  |
| TR-6.3 | De-personalization process shall be done via control keys comparing |  |
| TR-6.4 | For each failed de-personalization process failure counter shall be increased |  |
| TR-6.5 | For each succeeded de-personalization process failure counter shall be reset |  |
| TR-6.6 | Re-setting of failure counter shall be done similar to de-personalization process | e.g. By comparing entered and stored personalization key |
| TR-6.7 | It shall be configurable to forbid all unlocking/changing password attempts by user/modem SW, as if entered control keys were wrong | LAM requirement - if the user tries to unlock and enters the key, even if key is correct, action is ignored and "wrong key" message returned |
| TR-6.8 | It shall not be possible to de-personalize ME if no SIM card inserted |  |
| TR-7 | Data Storage |  |
| TR-7.1 | MEPD format shall be specified by TI during design phase |  |
| TR-7.2 | Size of MEPD shall be limited to 8Kbytes |  |
| TR-7.3 | Secure part of MEPD shall be stored encrypted and shall be protected from unauthorized access | Shall be dependent on Modem Security Driver implementation |
| TR-8 | Personalization Keys |  |
| TR-8.1 | Specific locking keys could be used for locking unlocking. Behaviour shall be configurable | lock key shall be different from unlock. Up to TI, which way to use, separate storing or generating from unlock key. Consider Config tool dependency |
| TR-8.2 | Keys generating | covered by separate doc |
| TR-8.3 | Key truncation to 8 byte | e.g. it shall be configurable which length shall be checked - standard or 8 bytes |
| TR-8.4 | Standard Key length shall be 6-16 bytes for PCK and 8-16 for other control keys |  |
| TR-9 | Security Requirements |  |
| TR-9.1 | MEPD Data should be secured with confidentiality | Exact implementation is up to customer implementation of Modem Security Driver |
| TR-9.2 | Secure personalization info (e.g control keys) shall be kept separately from non-secure data (e.g. timer value etc) and shall not be accessed by ACI directly | Exact implementation is up to customer implementation of Modem Security Driver |
| TR-10 | Supplementary Information |  |
| TR-10.1 | Failure Counter concept shall be used to limit number of failing de-personalization attempts - every time de-personalization fails, counter is increased. When counter reaches its maximal value, de-personalization process cannot be run anymore. ME shall be returned to factory for unlock. |  |
| TR-10.2 | Max Value of the counter shall be stored together with other MEPD and thus is configurable |  |
| TR-10.3 | Handling of MMI behaviour of the counter (e.g. Increasing timer between de-personalization attempts) shall be provided | shall be provided by customer MMI, if required |
| TR-10.4 | Failure counter shall be also configurable for unlimited value |  |
| TR-10.5 | It shall be possible to interrogate Supplementary Information | API for retrieving any supplementary info values. Concept of flexible API is developed in LLD |
| TR-11 | SW Requirements |  |
| TR-11.1 | MMI shall have no direct access to MEPD | API for retrieving only certain information will be provided |
| TR-11.2 | MNC of length 3 shall be supported |  |
| TR-11.3 | GIDx at least of length 5 shall be supported | for proprietary locks it must be necessary to check more than 1 byte of GIDx |
| TR-11.4 | Number of attempts to reset Failure Counter shall be configurable only by config tool | No API will be provided |
| TR-11.5 | Following functionality will be required for ACI-MFW communication |  |
|  | - +CLCK: for lock/unlock/interrogate standard categories |  |
|  | - %XXXX: for lock/unlock/interrogate proprietary categories |  |
|  | - %XXXX: for get supplementary info |  |
|  | - +CPWD: for password change for standard categories |  |
|  | - %XXXX: for password change for proprietary categories |  |
|  | - +CPIN: for passing required Control key for unlock for one session |  |
|  | During design phase interface can be extended/changed |  |
| TR-11.6 | Maximal number of personalization code groups (IMSI, MNC/MCC etc) for each category shall be limited to 20 for NS, 5 SP and 1 CP |  |
| TR-11.7 | If MEPD contains already maximal number of personalization codes and user wants to personalize ME to new SIM card with new code, the oldest (first added) code will be replaced with newest one | FIFO way |
| TR-12 | Memory Requirements |  |
| TR-12.1 | For memory optimization the max size of MEPD can be decreased during design/implementation phase |  |
| TR-13 | Documentation Requirements |  |
| TR-13.1 | LLD or comparable documents for following modules shall be provided: ACI, MSD, MFW, MMI, Config Tool |  |
| TR-14 | User Interface Requirements |  |
| TR-14.1 | UI for SIM personalization shall be provided |  |
| TR-14.2 | UI for entering a control keys for each locked category to unblock ME for one session | e.g. as a response for CPIN |
| TR-14.6 | MMI shall provide following features: - UI for ME locking to NW, NS, SP, CP, SIM - UI for ME unlocking NW, NS, SP, CP, SIM - UI for status check of NW, NS, SP, CP, SIM locks - UI for password changing of NW, NS, SP, CP, SIM locks - UI for failure counter reset |  |
| TR-15 | Config Tools |  |
| TR-15.1 | Config tool shall be able to create MEPD and save it into local File |  |
| TR-15.2 | Output MEPD format of Config tool shall be synchronous with MEPD format used internally by low-level entities |  |

# Architecture

## Current architecture

As it was already mentioned, that current SIM Lock implementation does not cover majority of use cases, has serious security problem and serves actually as an example for customized SIM Lock implementation.

Default SIM Lock module architecture is shown on Fig. 3.1. For detailed description please refer [4].

## Enhanced Architecture

It is proposed to enhance SIM Lock functionality by re-designing SIM Lock module.

Enhanced SIM Lock architecture is shown on Fig. 3.2

Following constraints shall be applied to the architecture:

- All MMI applications shall run in MMI space

- Communication of MMI to ACI shall be done through command handlers (e.g. no direct SIM Lock module calls shall be done from MMI)

- SIM Lock Module shall consists of SIM Lock Module itself (which provides generalized part) and SIM Lock Module Extension (which contains the requirements specific logic and can be re-implemented by customers)

- Both SIM Lock and SIM Lock Ext module can communicate to other entities (e.g. SIM)

- It is desirable that only SIM Lock Ext (not a SIM Lock) communicates to Secure Driver

- Exceptionally SIM Lock and SIM Lock Ext might be accessed directly from ATI Extension for implementing some specific AT commands to avoid ACI changes.

- Secure driver is responsible for delivering Personalization Data to SIM Lock Module, updating its status according to SIM Lock request, doing all Security procedures such as MEPD decoding, Control Keys comparison etc. For more details please see 3.3

- Personalizing Tool is a mean to save Personalization data into non-volatile memory

Functional Blocks:

- MEPD Database – Customer will store settings for ME personalization in its own database in its own format.

- MEPD converter – to convert ME Personalization settings from customer format to MEPD format, used by Modem Security Driver, MEPD converting tool will be needed. It shall be created by the customer after clarifying all requirements to ME Personalization functionality and TI MEPD format.

- Personalizing Tool – tool used to store MEPD on ME storage (FFS, secure ROM etc)

- Platform Security Implementation – set of SW and HW means to manage storage, access and altering MEPD on different TI platforms.

- Modem Security Driver – SW module to provide access from L23 (mostly ACI) to MEPD. MSD is platform and Customer requirements dependent. Thus customer has always a possibility to create his own MSD, realizing required encryption algorithm.

- SIM Lock Module Ext – extension of ME Personalization module which contains major logic of ME personalization. Extension provides an API towards SIM Lock Module itself. Customer has always possibility to re-implement SIM Lock Ext to fit his requirements, leaving other parts of SW untouched

- SIM Lock module – mostly this is an adapter between SIM Lock Module Ext and other modules of L23

- ACI – Access Controller Interface, SW module for communication to upper layers

- L23 – Other modules of Layer 2,3

- MMI – Upper layer, realizing interaction to user/applications

## Platform Security Implementation

New implementation shall be used on different platforms, used by TI, with minor changes. To separate L23 from platform dependency, Modem Security Driver shall be introduced. Thus, Modem Security Driver will not only serve as module to encrypt/decrypt MEPD data, stored in ME, but will be responsible for usage of Platform Security Means properly.

Some features of different platforms are shown in Table 3.3

Table 3.3

| Platform | Specifics |
| --- | --- |
| Calypso+, Locosto | secure ROM available |
| Calypso | security is provided by usage of key, “generated” by DSP |
| Sedna | two processors are connected by HW interface, thus one of the tasks of Secure Driver could be to copy MEPD from ARM11 to ARM9 into MEPD’; |
| Neptune | one processor solution |
| P2 | communication is provided via Shared Memory |

Platform Security Implementation for different chipsets are shown on Fig. 3.4

# Use Cases

To understand functionality of ME Personalization and make design easier, following Chapter describes Use Cases for main blocks. Please keep in mind that Use Cases may differ from real implementation and could be refined after complete integration of ME Personalization.

## MMI Use cases

Following general MMI Use Cases shall be supported:

- ME Lock

- ME Unlock

- Permanent Unlock (extension of ME Unlock) – currently not considered

- Status Interrogation

- Password Changing

- Failure Counter Reset

- Powering On

- Supplementary Info Request/Setup

All other actions/features (lock on range, on GID etc.) shall not be accessible from UI and be available only through MEPD configuration or Flashing Tool access.

MMI Use Case Diagram is shown on Fig. 4.1.1

Fig 4.1.2 gives MMI Use Case Description:

To implement above mentioned use cases in MMI, MFW shall provide access to ACI API. Thus, MFW shall map all ACI APIs into format, accepted by MMI and grant the access to MMI. Syntax of MFW API will be developed during ACI/MFW/BMI design phase.

## ACI Use Case

ACI Use Cases can be interpreted as AT commands actions (API description). Fig. 4.2.1 shows AT Commands based MMI-ACI interaction.

ACI API description is shown on Fig. 4.2.2

The +CLCK and +CPWD commands are standard AT commands, described in [1]. The commands %CLCK and %CPWD are proprietary TI commands for proprietary locks handling. It is also possible to extend +CLCK and +CPWD instead of introducing new commands – to be finalized in ACI LLD.

The most challenging part of ACI is internal logic, how the different kind of locks are handled. It will be defined in ACI LLD.

## Modem Security Driver

Although almost all the Personalization logic is kept inside of ACI, because of security reasons there are quite a lot of actions, to be performed in Modem Security Driver. Fig. 4.3.1 shows basic Use Cases for Modem Security Driver. For more details refer Modem Security Driver related documentation.

Corresponding Use Case description is given on Fig. 4.3.2. Thus, Modem Security Driver will need to know

# MEPD Structure

Proposed MEPD structure is shown on Fig. 5.1

MEPD is spited in three informative areas:

- MEPD configuration

- MEPD Records

- MEPD Keys

And three access areas:

- Full access

- Read Only

- No Access

ACI can see Read Only area “as-is”, but cannot change anything there. MEPD configuration and secure part of MEPD record (status, category dependency) are stored there

Full access area stores the information that is allowed to be freely changed by ACI – code groups.

“No access” area is only accessed by Secure Driver. All control keys are stored there and verified only by Modem Secure Driver. ACI will receive only the result of comparison.

Please refer MSD documentation for more details.

Appendices

  - Acronyms

This is just an example!:

| ACI | Access Control Interface |
| --- | --- |
| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| DSP | Digital Signal Processor |
| FFS | Flash File System |
| IMSI | International Mobile Subscriber Identifier |
| ME | Mobile Equipment |
| MEPD | ME Personalization Data |
| MFW | MMI Framework |
| MMI | Man Machine Interface |
| MSD | Modem Security Driver |
| OTA | Over the Air |
| RAM | Random Access Memory |
| ROM | Read Only Memory |
| SIM | Subscriber Identity Module |

  - Glossary

This is just an example!:

| SIM Lock | Module/Functionality providing ME Personalization features, described in [3] |
| --- | --- |
| Default SIM Lock | SIM Lock implementation, used in G23 SW as per end of 2003 |
| Enhanced SIM Lock | Newly developed SIM Lock functionality |
| Customer | Consumer of TI Protocol Stack and ME Personalization Functionality (Phone developer, Operator, Service Centre etc) |
| User | End User of ME |