LLD Automatic Call Repeat

| Project | G23-GSM Protocol Stack |
| --- | --- |
| Document Type | AT Command Interface Description |
| Title | LLD Automatic Call Repeat |
| Author | TI Employee |
| Creation Date | 6 November, 2003 |
| Last Modified | 6 November, 2003 |
| ID and Version | 8462.719.03.001 |
| Status | Being Processed |

Copyright  2001 Texas Instruments, Inc. All rights reserved.

Texas Instruments Proprietary Information – Strictly Private

Document Control

ã Copyright Texas Instruments, Inc. 2001

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Texas Instruments reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Texas Instruments. Texas Instruments accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a license agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Texas Instruments.

  - Document History

ID	Author	Date	Status

8462.719.03.001	RM	6 November, 2003	Being Processed

  - References, Abbreviations, Terms

[1]	7010.801, References and Vocabulary, Texas Instruments.

[2]       GSM 2.07, Mobile Stations (MS) features

Table of Contents

Introduction	3

1	General requirements in ACI	4

2	Effort in ACI	5

3	Error indication vs. call repeat attempts	6

4	AT-command syntax	7

4.1	AT%RDL	7

4.2	AT%RDLB	7

5	MSC’s for ACI support of automatic calling repeat	9

5.1	MSC’s of AT command AT%RDL	9

5.1.1	Set automatic calling repeat mode	9

5.1.2	Query automatic calling repeat mode	9

5.2	MSC’s of AT command AT%RDLB	10

5.2.1	Deleting of black list	10

5.2.2	Query black list entries	10

5.3	Further MSC’s	11

5.3.1	Check Causes of unsuccessful Call Attempts	11

5.3.2	Elapsing of Redial Timer	12

5.3.3	Check Redial State for MOC, MTC, ATH and AT+CHUP	12

6	Modifications in ACI	14

6.1	Extension of Existing Data Types	14

6.1.1	Module ati_cmd.c	14

6.1.1.1	Table cmds[]	14

6.1.2	Module aci_cmh.h	14

6.1.2.1	Type T_ACI_AT_CMD	14

6.1.3	Module  cmh.h	14

6.1.3.1	Type RAT_ID	14

6.1.4	Module psa_cc.h	14

6.1.4.1	Type T_CC_CALL_TBL	14

6.1.4.2	Type T_CC_CLTP	15

6.2	Modification of Existing Functions	15

6.2.1	Module cmh_ccf.c	15

6.2.1.1	cmhCC_SndDiscRsn ()	15

6.2.2	Module cmh_ccr.c	15

6.2.2.1	cmhCC_CallReleased ()	15

6.2.3	Module cmh_ccq.c	15

6.2.3.1	qAT_PlusCLCC()	15

6.2.4	Module cmh_ccs.c	15

6.2.4.1	cmhCC_Dial ()	15

6.2.4.2	sAT_H ()	15

6.2.4.3	sAT_PlusCHUP ()	16

6.2.5	Module cmh_f.c	16

6.2.5.1	cmh_timeout ()	16

6.2.6	Module cmh_mmis.c	16

6.2.6.1	cmhMM_handleAudioTone ()	16

6.2.7	Module aci_pei.c	16

6.2.7.1	pei_init ()	16

6.2.7.2	pei_exit ()	16

6.2.8	Module psa_ccf.c	16

6.2.8.1	psaCC_InitCtbNtry ()	16

6.2.9	Module psa_ccp.c	16

6.2.9.1	psa_mncc_disconnect_ind ()	16

6.2.9.2	psa_mncc_reject_ind ()	16

6.2.9.3	psa_mncc_release_cnf ()	16

6.2.9.4	psa_mncc_release_ind ()	16

6.2.9.5	psa_mncc_setup_cnf ()	16

6.2.9.6	psa_mncc_setup_cnf ()	17

6.2.9.7	psa_mncc_setup_req ()	17

7	New Functions and Data Structures	18

7.1	Predefined Values	18

7.1.1	Module l4_tim.h	18

7.1.1.1	Timer	18

7.1.2	Module aci_cmh.h	18

7.1.2.1	Constants	18

7.2	New Data Types	18

7.2.1	Module aci_cmh.h	18

7.2.1.1	T_ACI_CC_BLACKL_ENTRY	18

7.2.1.2	T_ACI_CC_RDL_BLACKL_STATE	18

7.2.1.3	T_ACI_CC_RDL_FFS	19

7.2.1.4	T_ACI_CC_REDIAL_BLACKL	19

7.2.1.5	T_ACI_CC_REDIAL_BLMODE	19

7.2.1.6	T_ACI_CC_REDIAL_MODE	19

7.2.1.7	T_ACI_CC_REDIAL_NOTIF	19

7.2.1.8	T_ACI_CC_REDIAL_STATE	19

7.2.1.9	T_ACI_CC_REDIAL_PAR	20

7.2.2	Extending of FFS	20

7.2.2.1	RedialMode	20

7.3	New Functions	20

7.3.1	Module ati_cc.c	20

7.3.1.1	setatPercentRDL	20

7.3.1.2	qeatPercentRDL	21

7.3.1.3	setatPercentRDLB	21

7.3.1.4	qeatPercentRDLB	21

7.3.2	Module ati_ret.c	21

7.3.2.1	rCI_PercentRDL	21

7.3.2.2	rCI_PercentRDLB	22

7.3.3	Module cmh_ccf.c	22

7.3.3.1	cmhCC_redialCheck	22

7.3.3.2	cmhCC_redialChkBlackl	22

7.3.3.3	cmhCC_rd_mode_FFS	22

7.3.4	Module cmh_ccr.c	23

7.3.4.1	cmhCC_redialTimeout	23

7.3.5	Module cmh_ccs.c	23

7.3.5.1	rdlPrm_exit	23

7.3.5.2	rdlPrm_init	23

7.3.5.3	sAT_PercentRDL	23

7.3.5.4	sAT_PercentRDLB	24

7.3.6	Module cmh_ccq.c	24

7.3.6.1	qAT_PercentRDL	24

7.3.6.2	qAT_PercentRDLB	24

7.3.7	Module smi_dmy.c	25

7.3.7.1	rAT_PercentRDL	25

7.3.7.2	rAT_PercentRDLB	25

8	Test Strategy	26

8.1	Tests under Windows	26

8.2	Tests on target	26

# Introduction

This document contains the needed effort  for the  ACI support of  the feature “Automatic Calling Repeat”. The behaviour for “Automatic calling repeat call attempt restrictions” is described in GSM 02.07, Annex A.  Up to now this feature was implemented in MFW.

# General requirements in ACI

For support of automatic calling repeat of voice calls two new AT commands are necessary:

- Setting redial mode ON/OFF of automatic call repeat:  AT%RDL

- Show /delete of blacklist of phone numbers:  AT%RDLB

The redialling of the last unsuccessful outgoing phone number depends on error indications described in [2]. These indications requests different redialling procedures.

The ACI has to manage a “black list” of outgoing phone numbers (at least 8). Their use is forbidden until the user deletes the list. If the black list is full the automatic calling repeat procedure is stopped until deleting of entries.

The entry in the black list depends on both parameters redial counter and call attempt error indication. The counter is reset after successful connection and after phone reset. The black list is deleted after phone switch on.

For the handling of different redialling procedures ACI needs some different redial timer described in [2].

Each outgoing call is checked against the black list entries.

The redialling procedure is stopped for:

- Each outgoing call (if not black listed) activated by user

- Each incoming call

- User set redial mode OFF

- User switchs off redialling via ATH and via AT+CHUP

The user can decide whether he want to get redialling state indications via the both new AT commands.

ACI has to store/restore the redial mode in FFS.

Note: SAT related redialling procedures are not focussed.

# Effort in ACI

| Description | New Commands/Functions | Comments |
| --- | --- | --- |
| Implementation AT commands Redial mode ON/OFF, unsolicited indication on/off | AT%RDL setatPercentRDL sAT_PercentRDL qetatPercentRDL qAT_PercentRDL | Set/Test/Query functions ATI, CMH Testcases |
| Implementation AT commands Show / Delete black list entries, unsolicited indication on/off | AT%RDLB setatPercentRDLB sAT_PercentRDLB qetatPercentRDLB qAT_PercentRDLB | Set/Test/Query functions ATI, CMH Testcases |
| Evaluation of unsuccessful call attempts, add phone number in black list, start redial timer | cmhCC_redialCheck | Check a lot of error indications considering outgoing calls in PSA |
| Implementation of redial timer administration | cmhCC_redialTimeout | New timer handles and function in CMH |
| Check black list against outgoing phone number; abort redialling procedure in case of an outgoing call |  | Check is to be implemented for each mobile outgoing call in CMH |
| Send if requested redial state indication to TE |  |  |
| Check redial timer for received mobile terminated calls |  | Check is to be implemented for each mobile incoming call in PSA |
| User aborts redialling mobile outgoing call via AT%RDL |  | Check call type CT_MOC_RDL in CMH |
| Store of redial mode in FFS | cmhCC_rd_mode_FFS | Test only on target possible |

# Error indication vs. call repeat attempts

The relevant error indications of unsuccessful call attempts are[2]:

1. "Busy destination":

Cause number		17	User busy

2. "Unobtainable destination - temporary":

Cause number		18	No user responding

19	User alerting, no answer

27	Destination out of order

34	No circuit/channel available

41	Temporary failure

42	Switching Equipment congestion

44	Requested circuit/channel not available

47	Resources unavailable, unspecified

3. "Unobtainable destination - permanent/long term":

Cause number		1	Unassigned (unallocated) number

3	No route to destination

22	Number changed

28	Invalid number format (uncomplete number)

- Network out of order.

This table describes the different redialling procedures:

Note: For the categories 1 and 2 above, n shall be 10;  for category 3, n shall be 1.

| call attempts | Minimum duration between Call attempt |
| --- | --- |
| Initial call attempt | - |
| 1st repeat attempt | 5 sec |
| 2nd repeat attempt | 1 min |
| 3rd repeat attempt | 1 min |
| 4th repeat attempt | 1 min |
| 5th repeat attempt | 3 min |
|  | . |
|  | . |
| nth repeat attempt | 3 min |

The conditions for the entry of a number in the black list are the following:

- Error indications of category 1 + 2:  the number of 10 unsuccessful call attempts is achieved  and the black list is not full yet

- Error indications of category 3: the first call repeat was unsuccessful and the black list is not full yet

# AT-command syntax

## AT%RDL

| Command | Possible response(s) |
| --- | --- |
| %RDL=[<mode>,[<n>]] | +CME ERROR: <err> |
| %RDL? | %RDL: <mode>, <n> |
| %RDL =? | %RDL: (supported < mode >, supported <n>s) |

Description

This function allows the user to set and reset the automatic calling repeat of unsuccessful outgoing call attempts.

When <n>=1 the unsolicited result %RDL: <state> is sent to TE.

Defined values:

<mode> :

- Disable automatic calling repeat

- Enable automatic calling repeat

<n>:

- No redial state indication to TE

- Redial state indication to TE

<state>:

- Redialling timer is started

- Automatic call repeat is started

- Call attempt fails

- Call attempt successful

- Automatic call repeat is stopped

## AT%RDLB

| Command | Possible response(s) |
| --- | --- |
| %RDLB =[<mode>,[<n>]] | +CME ERROR: <err> |
| %RDLB? | %RDLB: <n> %RDLB: <counter>, <number>[[…] <CR>><LF>%RDLB: <number>]] +CME ERROR: <err> |
| %RDLB =? | %RDLB: (list of supported <max counter >, supported <n>s) |

Description

This function allows the user to show and delete the entries of the black list. 
When <n>=1 the unsolicited result %RDLB: <state> is sent to TE.

Defined values:

<mode> :

- No delete black list

- Delete black list

<n>:

- No black list state indication to TE

- Black list state indication to TE

<counter> :

integer type values : number of black list entries

<number>:

string type: phone number

<max counter >

integer type values : maximal number of black list entries

<state>:

- Black list is full

- Phone number entry is set in black list

- Phone number is an black list entry

# MSC’s for ACI support of automatic calling repeat

## MSC’s of AT command AT%RDL

### Set automatic calling repeat mode

With the set AT command %RDL the chosen mode and notification state will be stored into the CC redial parameter and in the Flash File System  (FFS). Furthermore memory of blacklist is allocated if it is does not exist yet.

PSA

ATI

CMH

setatPercentRDL

(*string, srcId)

sAT_PercentRDL

(srcId , mode, notification)

store param mode in FFS

allocate memory of blacklist if necessary

OK/CME error

### Query automatic calling repeat mode

With the query AT command %RDL the chosen mode and notification status will be interrogated.

PSA

ATI

CMH

queatPercentRDL

(*string, srcId)

qAT_PercentRDL

(srcId ,* mode, * notification)

OK/CME error

## MSC’s of AT command AT%RDLB

### Deleting of black list

With the AT command %RDLB the user can delete all of phone number entries in the black list and or set the notification status of black list

PSA

ATI

CMH

setatPercentRDLB

(*string, srcId)

sAT_PercentRDLB

(srcId, notification)

Delete the black list entries

OK/CME error

### Query black list entries

With the query AT command %RDLB the user gets all of phone number entries of the black list and the notification status.

PSA

ATI

CMH

queatPercentRDLB

(*string, srcId)

copy phonebook entries of the black list in the output list

qAT_PercentRDLB

(srcId, *blacklist,

* notification)

OK/CME error

## Further MSC’s

### Check Causes of unsuccessful Call Attempts

For the redial procedure the normal and reject causes of unsuccessful outgoing call attempts are checked if the following primitives are received in ACI.

PSA

ATI

CMH

MNCC_DISCONNECT_IND

cmhCC_redialCheck(cId)

%RDL : 2

%RDL :  4

if max counter is reached

%RDLB: 2

or

%RDLB:  3

call release procedure

PSA

MNCC_RELEASE_IND

ATI

CMH

cmhCC_redialCheck(callId)

%RDL : 2

%RDL :  4

if max counter is reached

%RDLB: 2

or

%RDLB:  3

call release procedure

PSA

ATI

CMH

MNCC_REJECT_IND

cmhCC_redialCheck(callId)

call release procedure

%RDL : 2

%RDL :  4

if max counter is reached

%RDLB: 2

or

%RDLB:  3

### Elapsing of Redial Timer

If the redial timer is elapsed ACI starts a new outgoing call. It is successful.

PSA

ATI

CMH

cmhCC_redialTimeout()

MNCC_SETUP_REQ

%RDL : 3

MNCC_SETUP_CNF

%RDL : 5

### Check Redial State for MOC, MTC, ATH and AT+CHUP

During redial timer is running user starts a new outgoing call. The timer is stopped immediately. In case the new call number is already set in the black list, the new outgoing call is canceled.

PSA

ATI

CMH

MNCC_SETUP_REQ

Stop redial timer

Check against black list,

%RDL : 6

if number is in blacklist

%RDLB: 4

During redial timer is running ACI receives an incoming call indication. The timer is stopped immediately.

PSA

ATI

CMH

MNCC_SETUP_IND

Stop redial timer

%RDL : 6

User want to disconnect an outgoing call.

PSA

ATI

CMH

Outgoing call is running

ATH/AT+CHUP

Stop redial timer

%RDL : 6

call release procedure

# Modifications in ACI

## Extension of Existing Data Types

### Module ati_cmd.c

#### Table cmds[]

- Add new entries:  setatPercentRDL, qeatPercentRDL, setatPercentRDLB, qeatPercentRDLB

### Module aci_cmh.h

#### Type T_ACI_AT_CMD

typedef enum {

…

AT_CMD_RDL,        	 /* %RDL command, process redial mode */

AT_CMD_RDLB,        	 /* %RDLB command, process black list in redial mode */

AT_CMD_MAX                  	/* maximum command id */

} T_ACI_AT_CMD;

### Module  cmh.h

#### Type RAT_ID

typedef enum {

…

RAT_RDL,

RAT_RDLB	 /* %RDLB command, process black list in redial mode */

} RAT_ID;

### Module psa_cc.h

#### Type T_CC_CALL_TBL

typedef struct CCCallTabl{

…

UBYTE  rdlCnt;                       	 /* redial counter */

UBYTE  rdlTimIndex;             	/* redial timer index for 5th and more attempts */

} T_CC_CALL_TBL;

Description:

For the state of redial counter refering to the callId the new parameter rdlCnt is to be introduced.

#### Type T_CC_CLTP

typedef enum{

NO_VLD_CT = 0,                  	/* not a valid call type */

CT_MOC,                              	 /* mobile originated call */

CT_MTC,                               	/* mobile terminated call */

CT_NI_MOC ,                       	/* network initiated mobile originated call */

CT_MOC_RDL           	 /* redialling mobile originatied call  */

} T_CC_CLTP;

Description:

For distinction from usual mobile originated call the new call type CT_ MOC_RDL is to be introduced.

## Modification of Existing Functions

### Module cmh_ccf.c

#### cmhCC_SndDiscRsn ()

- If redialling is running calltable entries are not changed

### Module cmh_ccr.c

#### cmhCC_CallReleased ()

- If redialling is running calltable redial call Id entry is not changed

### Module cmh_ccq.c

#### qAT_PlusCLCC()

- Add check of call type CT_MOC_RDL  for call table output

- Enhance for state CS_ACT_REQ call type CT_MOC_RDL  output during redial attempt

### Module cmh_ccs.c

#### cmhCC_Dial ()

- Add comparison of black list entries against outgoing phone number

- Add stop of redial timer and  reset of redial parameters if necessary

#### sAT_H ()

- Add check of call type CT_MOC_RDL for stop of redial procedure and reset redial parameter if user aborts redialling mobile originated call

#### sAT_PlusCHUP ()

- Add check of call type CT_MOC_RDL in state CS_ACT_REQ for stop of redial procedure and reset redial parameter if user aborts redialling mobile originated call; inform user

### Module cmh_f.c

#### cmh_timeout ()

- Call new function cmhCC_redialTimeout()

### Module cmh_mmis.c

#### cmhMM_handleAudioTone ()

- for redialling BUSY tone is started several times

### Module aci_pei.c

#### pei_init ()

- Add function rdlPrm_init() for redial parameter

- Add function cmhCC_rd_mode_FFS() reading of redial mode from FFS

#### pei_exit ()

- Add function rdlPrm_exit() for redial parameter , freeing of black list

### Module psa_ccf.c

#### psaCC_InitCtbNtry ()

- Add initializing redial parameter

### Module psa_ccp.c

#### psa_mncc_disconnect_ind ()

- Add call new function cmhCC_redialCheck ()

#### psa_mncc_reject_ind ()

- Add call new function cmhCC_redialCheck ()

#### psa_mncc_release_cnf ()

- Add call new function cmhCC_redialCheck () ; no change calltable entry if redialling time is running

#### psa_mncc_release_ind ()

- Add call new function cmhCC_redialCheck (); no change calltable entry if redialling time is running

#### psa_mncc_setup_cnf ()

- For successful mobile originated calls add reset redial parameters; change call type CT_MOC_RDL to CT_MOC; inform user, reset redial parameter

#### psa_mncc_setup_cnf ()

- For successful mobile originated calls add reset redial parameters; change call type CT_MOC_RDL to CT_MOC; inform user, reset redial parameter

#### psa_mncc_setup_req ()

- add check call type CT_MOC_RDL

# New Functions and Data Structures

## Predefined Values

### Module l4_tim.h

#### Timer

/* time in milliseconds */

ACI_REPEAT          	5000		/* 5 sec for waiting for 1th redialing        */

ACI_REPEAT_2_4	60000		/* 1 min for waiting for 2th to 4st redialing  */

/* timer index */

ACI_REPEAT_HND

### Module aci_cmh.h

#### Constants

MAX_NUM_REPEAT_ATT 10                    /* maximal number of call repeat attempts */

MAX_NUM_ENTR_BLACK_LIST 10        /* maximal number of black list phone numbers */

## New Data Types

### Module aci_cmh.h

#### T_ACI_CC_BLACKL_ENTRY

typedef struct{

UBYTE             numb_len;

CHAR               number[MAX_CC_CALLED_NUMBER];

T_ACI_TOA     type;

}T_ACI_CC_BLACKL_ENTRY;

#### T_ACI_CC_RDL_BLACKL_STATE

typedef enum{

BLACKLIST_FULL = 2,                /* black list is full */

ENTRY_BLACKLISTED,              /* phone number set in black list */

ENTRY_IN_BLACKLIST              /* phone number is in black list */

}T_ACI_CC_RDL_BLACKL_STATE

#### T_ACI_CC_RDL_FFS

typedef enum

{

READ_RDLmode = 0,              	/* read redial mode from FFS */

WRITE_RDLmode                  	/* write redial mode in FFS */

}T_ACI_CC_RDL_FFS;

#### T_ACI_CC_REDIAL_BLACKL

typedef struct CCblacklist{

UBYTE blCount;                  	/* counter of black list entries */

T_ACI_CC_BLACKL_ENTRY blNum[MAX_NUM_ENTR_BLACK_LIST];

/* table with blacklisted phone number */

}T_ACI_CC_REDIAL_BLACKL;

Description:

For blacklist of the outgoing phone numbers is to be introduced. It is possible to save maximal 10 numbers.

#### T_ACI_CC_REDIAL_BLMODE

typedef enum{

BLMODE_NO_PRESENT = -1,        	/* parameter is not given */

BL_NO_DELETE,

BL_DELETE                     		 /* blacklist is deleted */

}T_ACI_CC_REDIAL_BLMODE;

#### T_ACI_CC_REDIAL_MODE

typedef enum{

AUTOM_REP_NOT_PRESENT = -1,      /* parameter is not given */

AUTOM_REPEAT_OFF = 0,               	/* automatic calling repeat is switched off */

AUTOM_REPEAT_ON                       	/* automatic calling repeat is switched on */

} T_ ACI_CC_REDIAL_MODE;

#### T_ACI_CC_REDIAL_NOTIF

typedef enum{

NOTIF_NO_PRESENT = -1,        	/* parameter is not given */

NO_NOTIF_USER,                	/* no notification to user */

NOTIF_USER                    	/* notification to user */

} T_ACI_CC_REDIAL_NOTIF;

#### T_ACI_CC_REDIAL_STATE

typedef enum{

REDIAL_TIM_START = 2,                   	/* redial timer starts */

REDIAL_ATT_START,                 	 /* start redialling attempt */

CALL_ATTEMPT_FAILED,

CALL_ATTEMPT_SUCCESSFUL,

REDIAL_STOP                  	 /* redialling finished */

}T_ACI_CC_REDIAL_STATE;

#### T_ACI_CC_REDIAL_PAR

typedef struct {

T_ ACI_CC_REDIAL_MODE rdlMod;      	 /* redial mode */

T_ACI_CC_REDIAL_NOTIF rdlModN;    	 /*  redial state notification */

SHORT rdlcId;                           	 /* call id for redialling */

BOOL rdlState;                          	  /* internal redial state */

T_ACI_CC_REDIAL_NOTIF rdlBlN;          	  /* black list state notification */

T_ACI_CC_REDIAL_BLMODE rdlBlMod;      /* black list mode */

}T_ACI_CC_REDIAL_ PAR;

Description:

The global structure contains all needed redial parameter.

### Extending of FFS

#### RedialMode

Pathname: /GSM/COM/; fixed object

BYTE			 redialMode

0            			 /* default value: automatic calling repeat switch off  */

1            			 /* automatic calling repeat switch on */

## New Functions

### Module ati_cc.c

#### setatPercentRDL

Prototype:

GLOBAL T_ATI_RSLT setatPercentRDL (char *cl, UBYTE srcId)

Parameters:

cl		command string

srcId		source of AT command

Return:

ATI_FAIL  	wrong parameter or set redial mode fails

ATI_CMPL 	set redial mode successful

Description:

This function checks the AT command string and calls sAT_PercentRDL.

#### queatPercentRDL

Prototype:

GLOBAL T_ATI_RSLT queatPercentRDL (char *cl, UBYTE srcId)

Parameters:

cl		command string

srcId		source of AT command

Return:

ATI_FAIL  	wrong parameter or query redial mode fails

ATI_CMPL 	query redial mode successful

Description:

This function checks the AT command string and calls qAT_PercentRDL.

#### setatPercentRDLB

Prototype:

GLOBAL T_ATI_RSLT setatPercentRDLB (char *cl, UBYTE srcId)

Parameters:

cl		command string

srcId		source of AT command

Return:

ATI_FAIL  	wrong parameter or delete black list fails

ATI_CMPL 	delete black list successful

Description:

This function checks the AT command string and calls sAT_PercentRDLB.

#### queatPercentRDLB

Prototype:

GLOBAL T_ATI_RSLT queatPercentRDLB (char *cl, UBYTE srcId)

Parameters:

cl		command string

srcId		source of AT command

Return:

ATI_FAIL  	wrong parameter or empty black list

ATI_CMPL 	show black list successful

Description:

This function checks the AT command string and calls qAT_PercentRDLB.

### Module ati_ret.c

#### rCI_PercentRDL

Prototype:

GLOBAL void rCI_PercentRDL ( T_ACI_CC_REDIAL_STATE state )

Parameters:

state	 WAITING /  REDIAL_START/  CALL_ATTEMPT_FAILED
CALL_ATTEMPT_SUCCESSFUL/ REDIAL_STOPPED

Description:

This call back function sends the response string with user notification about redial state in ATI if the user notification is requested by user.

#### rCI_PercentRDLB

Prototype:

GLOBAL void rCI_PercentRDLB ( T_ACI_CC_REDIAL_STATE state )

Parameters:

state	 BLACKLIST_FULL/ ENTRY_BLACKLISTED

Description:

Thiscall back function sends the response string with user notification about black list state in ATI if the user notification is requested by user.

### Module cmh_ccf.c

#### cmhCC_redialCheck

Prototype:

GLOBAL void cmhCC_redialCheck (SHORT cId)

Parameters:

cId		call Id

Return:

-

Description:

If an outgoing call attempt was not successful this function evaluates the received error indications. It checks the redial counter with respect to the error indication and starts the appropriate redial timer.  If necessary the function adds entries in the black list.  If requested the function informs the user about the redial and/or black list state.

#### cmhCC_redialChkBlackl

Prototype:

GLOBAL void cmhCC_redialChkBlackl (SHORT cId)

Parameters:

cId		call Id

Return:

-

Description:

The function checks the desired MOC number against the black list entries.

#### cmhCC_rd_mode_FFS

Prototype:

GLOBAL T_ACI_RETURN cmhCC_rd_mode_FFS(T_ACI_CC_REDIAL_MODE rdl_mode,

T_ACI_CC_RDL_FFS ffs_mode)

Parameters:

redial_mode		AUTOM_REPEAT_ON/ AUTOM_REPEAT_OFF

ffs_mode		READ_RDLmode / WRITE_RDLmode

Return:

ATI_FAIL  	set redial mode fails

ATI_CMPL 	set redial mode successful

Description:

The function writes the redial mode in FFS.

### Module cmh_ccr.c

#### cmhCC_redialTimeout

Prototype:

GLOBAL void cmhCC_redialTimeout ( void )

Parameters:

-

Return:

-

Description:

This function checks the current redial mode and starts a call repeat. It increases the redial counter of the related call Id.

### Module cmh_ccs.c

#### rdlPrm_exit

Prototype:

GLOBAL void rdlPrm_exit(void)

Description:

This function resets all redial parameter of structure rdlPrm and give the black list memory free.

#### rdlPrm_init

Prototype:

GLOBAL void rdlPrm_init(void)

Description:

This function initializes all redial parameter of structure rdlPrm.

#### sAT_PercentRDL

Prototype:

GLOBAL T_ACI_RETURN sAT_ PercentRDL ( T_ACI_CMD_SRC  srcId,

T_ACI_CC_REDIAL_MODE redial_mode,

T_ACI_CC_REDIAL_NOTIF notification)

Parameters:

srcId			source of AT command

redial_mode		AUTOM_REPEAT_ON/ AUTOM_REPEAT_OFF
notification		NO_NOTIF_USER /NOTIF_USER

Return:

ATI_FAIL  	set redial mode fails

ATI_CMPL 	set redial mode successful

Description:

This function saves the redial mode and user notification state in the structure T_CC_REDIAL_PAR rdlPrm. Furthermore it stores the mode in the FFS. If the chosen redial mode is AUTOM_REPEAT_ON the function allocates the black list if it does not exist.  In case AUTOM_REPEAT_OFF the function resets the redial parameter, stops the redial timer and frees the call table entry (if necessary).

#### sAT_PercentRDLB

Prototype:

GLOBAL T_ACI_RETURN sAT_ PercentRDLB ( T_ACI_CMD_SRC   srcId,

T_ACI_CC_REDIAL_BLMODE blacklist_mode,

T_ACI_CC_REDIAL_NOTIF notification)

Parameters:

srcId			source of AT command

blacklist_mode                  BL_NO_DELETE/BL_DELETE

notification		NO_NOTIF_USER /NOTIF_USER

Return:

ATI_FAIL  	delete black list fails

ATI_CMPL 	delete black list successful

Description:

This function clears the black list if blackl_clear is TRUE and stores the state of user notification.

### Module cmh_ccq.c

#### qAT_PercentRDL

Prototype:

GLOBAL T_ACI_RETURN qAT_ PercentRDL ( T_ACI_CMD_SRC   srcId,

T_ACI_CC_REDIAL_MODE * redial_mode,

T_ACI_CC_REDIAL_NOTIF *notification)

Parameters:

srcId			source of AT command

redial_mode		AUTOM_REPEAT_ON/ AUTOM_REPEAT_OFF

notification		NO_NOTIF_USER /NOTIF_USER

Return:

ATI_FAIL  	query redial mode fails

ATI_CMPL 	query redial mode successful

Description:

This function informs about the state of redial mode and user notification.

#### qAT_PercentRDLB

Prototype:

GLOBAL T_ACI_RETURN qAT_PercentRDLB ( T_ACI_CMD_SRC  srcId,

T_ACI_CC_REDIAL_BLACKL *blackl,

T_ACI_CC_REDIAL_NOTIF *notification )

Parameters:

srcId		source of AT command

blackl		list of blacklisted phone numbers

notification	NO_NOTIF_USER /NOTIF_USER

Return:

ATI_FAIL  	show black list fails

ATI_CMPL 	show black list successful

Description:

This function returns the black list entries and the state of user notification to the user.

### Module smi_dmy.c

#### rAT_PercentRDL

Prototype:

GLOBAL void rAT_PercentRDL (T_ACI_CC_REDIAL_STATE state)

Parameters:

state	 WAITING /  REDIAL_START/  CALL_ATTEMPT_FAILED
CALL_ATTEMPT_SUCCESSFUL/ REDIAL_STOPPED

Description:

This is the callback dummy function in SMI.

#### rAT_PercentRDLB

Prototype:

GLOBAL void rAT_PercentRDLB (T_ACI_CC_REDIAL_STATE state)

Parameters:

state				 BLACKLIST_FULL/ ENTRY_BLACKLISTED

Description:

This is the callback dummy function in SMI.

# Test Strategy

## Tests under Windows

Add new testcases:

- for AT%RDL with correct and incorrect parameters

- for AT%RDLB with correct and incorrect parameters; delete black list entries

- Evaluation of all kind of related error indications, inform user

- Set phone numbers in black list until the list is full, inform user about black list

- Check blacklist entries against outgoing phone number; inform user

- Check redialling status for incoming calls; inform user

- If redialling is successful user is informed

- Redialling if timer is elapsed; inform user

- Test ATH and AT+CHUP for redialling ; Test AT+CLCC

- Abort redialling procedure by user with AT%RDL = 0

- Test with four window stacks

## Tests on target

- Store/Restore redial mode in FFS: target test

- Test the both new AT Commands: set correct and incorrect parameters

- Abort redialling procedure by user with AT%RDL = 0, x

- Evaluation of error indication USER BUSY

- Set entries in blacklist, delete entries in black list

- Check blacklist entries against outgoing phone number

- Check redialling status for incoming call

- Redialling if timer is elapsed

- Control responses via ATI

- Test with some images