Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

LLD SixTIes SIM

Low Level Design

(Edit via File/Properties/Summary!)

| Document Number: | xxxx.xxx.xx.xxx (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 1 (Edit via File/Properties/Summary!) |
| Status: | Submitted (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2004-Aug-10 |
| Last changed: | 2004-Sep-29 by Gareth Cleeves TI Employee |
| File Name: | input.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2004-Aug-10 | Gareth Cleeves |  | 0.1 | Draft | 1 |
| 2004-Aug-30 | Gareth Cleeves | Comments from Martin Schroeder | 1 | Draft |  |

Notes:

- Initial version

Table of Contents

1	Introduction	5

2	The Refresh Proactive SIM Command	6

3	New Messages	8

4	Implementing the New Messages	9

4.1	New cust_mode field in the SIM_ACTIVATE_REQ primitive	9

4.2	New message SIM_REFRESH_USER_RES	11

5	Implementing the SAT Refresh Command	13

6	Testing with Windows Simulation	15

A.	Appendices	16

B.	Acronyms	16

C.	Glossary	16

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |
|  | SixTIes SAT Command Support High Level Design |

# Introduction

This document provides the low-level design for the modification to the SIM entity to be compatible with the  SixTIes application.

An overview of the high-level design is provided in the HLD (reference xxxxx).

The modification to the SIM entity is restricted to the processing of the SAT Refresh command.

The SixTIes requirement is for the MMI to be informed about any SIM Refresh command and have the option to disallow the command.

There are no variants of the SIM entity. Therefore, on power up the ACI will  inform the SIM entity if it is required to support the SixTIes functionality.

If SixTIes functionality is required then the Refresh command is forwarded to the ACI (it previously wasn’t) and a response expected. The response will inform the SIM entity if the user accepts the Refresh or not.

If the user accepts the refresh command, the SIM entity proceeds as before by sending the appropriate command to the ACI, MM and SMS entities.

If the user rejects the refresh command, the SIM Entity will return an “ME currently unable to process command” Terminal Response to the SIM. The SIM will resend the Refresh Command to the SIM Entity, although the details of exactly how frequently are SIM specific.

# The Refresh Proactive SIM Command

- SIM initialization <extra in 11.14 spec. see below>	0x00 	soft reset	11.14 mode 3

- File Change notification				0x01	soft reset	11.14 mode 1

- SIM initialization and file change notification		0x02	soft reset	11.14 mode 2

- SIM initialization and full file change notification	0x03	soft reset	11.14 mode 0

- SIM Reset						0x04	hard reset	11.14 mode 4

In contrast to the present behavior, SixTIes wants to be notified of all refresh processing:

Additionally SixTIes wants to have the possibility to allow the user to confirm or reject the refresh, which means, if the user rejects the refresh (including a SIM reset) a Terminal Response will be returned to the SIM with the value “ME currently unable to process command’

For each refresh type the original PDU must be delivered, in a SIM_TOOLKIT_IND, to the modem part and forwarded, in a %SATI indication, to the SixTIes MMI.

6.4.7 REFRESH <From 11.14> The purpose of this command is to enable the ME to be notified of the changes to the SIM configuration that have occurred as the result of a SIM application activity. It is up to the SIM application to ensure that this is done correctly. The command supports five different modes: - SIM Initialization. This mode tells the ME to carry out SIM initialization as it is defined in TS 11.11 [20], starting after the CHV1 verification procedure. The ME shall not reset the SIM electrically. - File Change Notification. This mode advises the ME of the identity of the EFs that have been changed (in structure and/or contents) in the SIM. This information can be used by the ME if there is an image of SIM EFs (e.g. the ADN file) in the ME's memory, to determine whether it needs to update this image. - SIM Initialization and File Change Notification. This is a combination of the first two modes above. - SIM Initialization and Full File Change Notification. This mode causes the ME to perform the SIM initialization procedure of the first mode above and advises the ME that several EFs have been changed (in structure or contents) in the SIM. If there is an image of SIM EFs in the ME's memory, the ME shall completely update this image. - SIM Reset. This mode causes the ME to run the GSM session termination procedure and to deactivate the SIM in accordance with TS 11.11 [20]. Subsequently, the ME activates the SIM again and starts a new card session. In case of a 3 Volt technology ME, the ME shall restart the SIM with the same supply voltage as in the previous session, if the ME can ensure that the SIM has not been changed in between. Otherwise, the ME shall perform the supply voltage switching in accordance with TS 11.12 [21]. The ME shall not send the TERMINAL RESPONSE; this is an exception from the normal procedure, where TERMINAL RESPONSE is sent after completion of the command. The SIM Application shall interpret a new activation of the contacts of the SIM as an implicit TERMINAL RESPONSE. The SIM Reset mode is used when a SIM application requires ATR or complete SIM initialization procedures to be performed. SIM Applications should take into account that early implementations of SIM Application Toolkit in some MEs may send a TERMINAL RESPONSE after performing the REFRESH command involving resetting the SIM electrically. If the ME performs the REFRESH command successfully for only those EFs indicated in the mode, the ME shall inform the SIM using TERMINAL RESPONSE (OK), after it has completed its refreshing. For REFRESH commands with mode other than "SIM Reset", it is permissible for the ME, as part of its execution of the REFRESH command, to read EFs in addition to those notified by the SIM, or to perform a SIM initialisation, provided that the procedure executed wholly encompasses the mode requested by the SIM. The ME shall not electrically reset the SIM. If the ME does the refreshing successfully, it shall inform the SIM using TERMINAL RESPONSE (Refresh performed with additional EFs read), after the ME has completed its refreshing. It should be noted that reading additional EFs will lengthen the refresh procedure. If the ME receives a REFRESH command while in a state where execution of the command would be unacceptable, upsetting the current user operation (e.g. notification during a call that the IMSI has changed), the ME shall inform the SIM using TERMINAL RESPONSE (ME currently unable to process command - currently busy on call) or TERMINAL RESPONSE (ME currently unable to process command - screen is busy) as appropriate. NOTE: Many MEs copy an image of the SIM's memory to the ME at initialization to speed up access to these fields during a GSM session. One of the purposes of this coding of the REFRESH command is to enable MEs to change such an image efficiently. If, on receipt of the REFRESH command, the ME replies that it is busy (e.g. in call or navigating menus), the toolkit application may shorten the polling interval utilising the POLL INTERVAL command in order to resend the REFRESH command more frequently. It is recommended for the ME to minimise the use of sending temporary problem TERMINAL RESPONSE, as during the period between the SIM issuing a REFRESH command and the ME performing the refresh procedure, there may be inconsistencies between data held in the ME and in the SIM. However, responsibility for retrying of all pro-active commands lies with the SIM Application. 12.6 Command details Byte(s); Description; Length. 1; Command details tag; 1. 2; Length = '03'; 1. 3; Command number; 1. 4; Type of command; 1. 5; Command Qualifier; 1. - Command number For contents and coding, see subclause 6.5.1. - Type of command: Contents: The Type of Command specifies the required interpretation of the data objects which follow, and the required ME procedure. Coding: See section 13.4 The ME shall respond to reserved values (i.e. values not listed) with the result "Command type not understood". - Command Qualifier: Contents: Qualifiers specific to the command. Coding: - REFRESH; '00' =SIM Initialization and Full File Change Notification; '01' = File Change Notification; '02' = SIM Initialization and File Change Notification; '03' = SIM Initialization; '04' = SIM Reset; '05' to 'FF' = reserved values.

# New Messages

Two new messages are required (a) to inform the SIM Entity on Power up if SixTIes support is required and (b) to indicated the user response to the request for the SIM Refresh command.

- To inform the SIM Entity on Power up if SixTIes support is required:-
This information is provided by adding an additional field to the SIM_ACTIVATE_REQ primative. The new field is cust_mode (short for custom mode). Cust_mode=1, SixTIes Support. Cust_mode=0, normal behaviour to apply.

- To indicated the user response to the request for the SIM Refresh command:-
SIM_REFRESH_USER_RES. This primitive will be sent from the ACI to the SIM.
Primitive contains a Field:- BOOL user_accepts. TRUE if user accepted the refresh command, FALSE if user has rejected it.
The primitive also contains an additional field:- stk_cmd. In which the MMI sends a pdu if the user rejects the refresh command. A Terminal Response is then generated from the pdu and sent to the SIM card.

# Implementing the New Messages

## New cust_mode field in the SIM_ACTIVATE_REQ primitive

The structure of the SIM_ACTIVATE_REQ primitive is defined by T_SIM_ACTIVE_REQ in ms\CDGINC\p_sim.h

The additional field, cust_mode, in the SIM_ACTIVATE_REQ primitive will be appended to the current structure:-

typedef struct

{

U8                        proc;                     /*<  0:  1> procedure type       */

U8                        mmi_pro_file;             /*<  1:  1> MMI profile          */

U8                        stk_pro_file[MAX_STK_PRF]; /*<  2: 20> SIM toolkit profile */

U8                        cust_mode;

U8                        _align0;                  /*< 22:  1> alignment            */

} T_SIM_ACTIVATE_REQ;

This primitive is processed by app_sim_activate_req() in sim_app.c :-

When sim_activate_req.proc is SIM_INITIALISATION read the cust_mode value and store it in sim_data.

i.e.

if sim_activate_req.proc EQ SIM_INITIALISATION

then sim_data.cust_mode =  sim_activate_req.cust_mode

The cust_mode value is held globally in sim_data structure .

sim_data is a data structure of type T_SIM_DATA which is defined in sim.h .

The additional field cust_mode, is added to the current structure:-

typedef struct

{

// General SIM Variables

ULONG          flags;                   // contains several flags for internal handling

USHORT         act_directory;           // last selected directory

USHORT         act_field;               // last selected elementary field

T_TIME         status_time;             // status poll time

USHORT         act_length;              // actual file or record length

USHORT         sim_data_len;            // response data len

USHORT         dir_status_len;          // size of directory status information

USHORT         remove_error;            // provides error code for SIM Remove

UBYTE          sim_phase;               // sim card phase

UBYTE          last_requested_pin_no;   // last requested pin number

UBYTE          act_access;              // actual access condition

UBYTE          max_record;              // records of actual file

UBYTE          field_type;              // of the last selected record

UBYTE          sw1;                     // last status code SW1 from SIM

UBYTE          sw2;                     // last status code SW2 from SIM

// SIM Toolkit Variables

#ifdef SIM_TOOLKIT

UBYTE          file_change_resp;        // tracks response to FILE CHANGE

SHORT          proactive_sim_data_len;

USHORT         stk_resp_len;

UBYTE          stk_response[16];        // save TERMINAL RESPONSE

UBYTE          stk_profile[MAX_STK_PRF];

USHORT         cell_identity;

T_loc_info     location_info;

T_SAT_TIMER    timer[MAX_SAT_TIMER];

UBYTE          sat_session;             /* SAT session started */

UBYTE          ext_sat_cmd;             /* indicator for external SAT commands */

UBYTE          term_resp_sent;          /* indicator if Terminal Response was sent */

UBYTE          idle_polling;            /* poll SIM in idle mode */

UBYTE          chk_sat_avail;           /* check SAT command to be fetched */

#ifdef FF_SAT_E

UBYTE          dti_connection_state;    /* state of DTI connection */

UBYTE          dti_rx_state;            /* state of DTI reception */

UBYTE          dti_tx_state;            /* state of DTI transmission */

UBYTE          event_data_avail;        /* Data available event */

UBYTE          bip_ch_id;               /* BIP channel identifier */

UBYTE          bip_suspend;             /* BIP suspension state */

UBYTE          bip_state;               /* state of BIP channel */

UBYTE          bip_rx_state;            /* state of BIP reception */

UBYTE          bip_tx_state;            /* state of BIP transmission */

UBYTE          bip_timer_state;         /* state of BIP release timer */

UBYTE          bip_general_result;      /* general result code for on demand link */

UBYTE          bip_add_info_result;     /* additional information for on demand link */

UBYTE          con_type;                /* connection type */

USHORT         received_data_pos;       /* start of not yet delivered data */

DTI_HANDLE     hDTI;                    /* DTILIB handle */

T_TIME         bip_release_time;        /* BIP release time */

ULONG          link_id;                 /* DTI link identifer */

T_desc_list2   data_to_send;            /* DTI data to send */

T_desc_list2   prev_data_to_send;       /* last values of DTI data to send */

T_desc_list2   received_data;           /* received DTI data */

#ifdef WAP

T_SRC_DES      udp_parameters;          /* ports and IP addresses for UDP */

#endif /* WAP */

T_SIM_DTI_REQ* sim_dti_req;             /* storage of SIM_DTI_REQ primitive */

T_cmd_details  bip_rx_cmd_details;      /* command details for RX */

T_cmd_details  bip_tx_cmd_details;      /* command details for TX */

BUF_cmd_prms   bip_cmd_prms;            /* parameter buffer for RX and TX */

#endif /* FF_SAT_E */

UBYTE          cust_mode;               /* custom: 0=default, 1=SixTIes*/

#endif /* SIM_TOOLKIT */

// Windows Simulation Variables

#if defined (_SIMULATION_)

USHORT         mode;

#endif /* _SIMULATION_ */

} T_SIM_DATA;

NOTE: The field cust_mode in sim_data is not required if a flag is used to represent the cust_mode value received in the SIM_ACTIVATE_REQ primitive

## New message SIM_REFRESH_USER_RES

This new primative indicates the user response to the request for the SIM Refresh command.
SIM_REFRESH_USER_RES primitive is sent from the MMI to the SIM.

The structure of the SIM_REFRESH_USER_RES primitive is defined by T_SIM_REFRESH_USER_RES in ms\CDGINC\p_sim.h

typedef struct

{

U8                       user_accepts;      /*<  0:  1> 1 = accepted, 0 = rejected   */

U8                        _align0;          /*<  1:  1> alignment                    */

U8                        _align1;          /*<  2:  1> alignment                    */

U8                        _align2;          /*<  3:  1> alignment                    */

T_stk_cmd                 stk_cmd;

} T_SIM_REFRESH_USER_RES;

The new primitive is processed by stk_sim_refresh_user_res() in sim_stk.c

An entry into sim_table[] (ms\src\sim_pei.c) is required to facilitate this processing :-

LOCAL const T_FUNC sim_table[] = {

MAK_FUNC_0 (app_sim_read_req,          SIM_READ_REQ            ), /* 4500 */

MAK_FUNC_0 (app_sim_update_req,        SIM_UPDATE_REQ          ), /* 4501 */

MAK_FUNC_0 (app_sim_read_record_req,   SIM_READ_RECORD_REQ     ), /* 4502 */

MAK_FUNC_N (pei_not_supported,         0                       ), /* 4503 */

MAK_FUNC_0 (app_sim_update_record_req, SIM_UPDATE_RECORD_REQ   ), /* 4504 */

MAK_FUNC_N (pei_not_supported,         0                       ), /* 4505 */

MAK_FUNC_N (pei_not_supported,         0                       ), /* 4506 */

#if defined SIM_TOOLKIT

MAK_FUNC_0 (stk_sim_refresh_user_res,   SIM_REFRESH_USER_RES  ), /*  */

#else /* SIM_TOOLKIT */

MAK_FUNC_N (pei_not_supported,         0                       ), /* 4507 */

#endif /* else SIM_TOOLKIT */

MAK_FUNC_0 (app_sim_increment_req,     SIM_INCREMENT_REQ       ), /* 4508 */

MAK_FUNC_0 (app_sim_verify_pin_req,    SIM_VERIFY_PIN_REQ      ), /* 4509 */

MAK_FUNC_0 (app_sim_change_pin_req,    SIM_CHANGE_PIN_REQ      ), /* 450a */

MAK_FUNC_0 (app_sim_disable_pin_req,   SIM_DISABLE_PIN_REQ     ), /* 450b */

MAK_FUNC_0 (app_sim_enable_pin_req,    SIM_ENABLE_PIN_REQ      ), /* 450c */

MAK_FUNC_0 (app_sim_unblock_req,       SIM_UNBLOCK_REQ         ), /* 450d */

MAK_FUNC_0 (app_sim_auth_req,          SIM_AUTHENTICATION_REQ  ), /* 450e */

MAK_FUNC_0 (app_sim_mm_update_req,     SIM_MM_UPDATE_REQ       ), /* 450f */

MAK_FUNC_0 (app_sim_sync_req,          SIM_SYNC_REQ            ), /* 4510 */

MAK_FUNC_0 (app_sim_activate_req,      SIM_ACTIVATE_REQ        ), /* 4511 */

#if defined SIM_TOOLKIT

MAK_FUNC_0 (stk_sim_toolkit_req,       SIM_TOOLKIT_REQ         ), /* 4512 */

MAK_FUNC_0 (stk_sim_toolkit_res,       SIM_TOOLKIT_RES         ), /* 4513 */

#else /* SIM_TOOLKIT */

MAK_FUNC_N (pei_not_supported,         0                       ), /* 4512 */

MAK_FUNC_N (pei_not_supported,         0                       ), /* 4513 */

#endif /* else SIM_TOOLKIT */

MAK_FUNC_0 (app_sim_access_req,        SIM_ACCESS_REQ          ), /* 4514 */

#if defined SIM_TOOLKIT

MAK_FUNC_0 (stk_file_update_res,       SIM_FILE_UPDATE_RES     ), /* 4515 */

#else /* SIM_TOOLKIT */

MAK_FUNC_N (pei_not_supported,         0                       ), /* 4515 */

#endif /* else SIM_TOOLKIT */

#if defined GPRS

MAK_FUNC_0 (gprs_sim_gmm_update_req,   SIM_GMM_UPDATE_REQ      ), /* 4516 */

#else /* GPRS */

MAK_FUNC_N (pei_not_supported,         0                       ), /* 4516 */

#endif /* else GPRS */

#if defined FF_SAT_E

MAK_FUNC_0 (stk_sim_dti_req,           SIM_DTI_REQ             ), /* 4517 */

MAK_FUNC_0 (stk_sim_eventlist_req,     SIM_EVENTLIST_REQ       ), /* 4518 */

#else

MAK_FUNC_N (pei_not_supported,         0                       ), /* 4517 */

MAK_FUNC_N (pei_not_supported,         0                       ), /* 4518 */

#endif /* FF_SAT_E */

};

GLOBAL void stk_sim_refresh_user_res (T_SIM_REFRESH_USER_RES * sim_refresh_user_res)

{

TRACE_FUNCTION ("stk_sim_refresh_user_res()");

/*check primitive is expected */

<If cust_mode NEQ 1 then log error and exit>

<IF sim_data.user_confirmation_expected NEQ TRUE then log error and exit>

if ((sim_refresh_user_res ->user_accepts) EQ FALSE)

{

/*user rejected Refresh Command Request      */

/*Send Terminal_Resp to SIM                  */

FKT_TerminalResponse (sim_toolkit_res->stk_cmd.cmd,

(USHORT)(sim_toolkit_res->stk_cmd.l_cmd>>3));

/*terminate */

<check any Messages to Free ??????>

}

else

{

/*user accepted Refresh Command Request */

<Process as before>

<Call procedure to process Refresh Command>

<NOTE:Ensure Messages are freed correctly>

}

sim_data.user_confirmation_expected = FALSE;

PFREE (sim_refresh_user_res);

}

The above function, in sim_app.c, is called to process the SIM_REFRESH_USER_RES primitive.

If the user accepts the Refresh Request (user_accepts  = 1) then this function continues with the processing of the Refresh Command that used to be handled in the stk_proactive_polling() function.

If the user rejects the Refresh Request (user_accepts  = 0, stk_cmd = pdu from MMI) a Terminal Response is sent to the SIM built from the the pdu received from the MMI.

# Implementing the SAT Refresh Command

The SAT Refresh Command is processed within stk_proactive_polling() as well as in

stk_sim_refresh_user_res()  (see previous section).

The stk_proactive_polling() function is in sim_stk.c .

The stk_proactive_polling() code needs to be modified to reflect the new functionality:-

If cust_mode EQ 1

Then send Refresh command to MMI and terminate

If cust_mode EQ 0

Process Refresh Command as before

Modifications to stk_proactive_polling() function are :-

Global void stk_proactive_polling (void)

{

-------

PALLOC (sim_toolkit_ind, SIM_TOOLKIT_IND)

<clear primitive>

-------

-------

switch (cmd_type)

{

case STK_REFRESH:

If (sim_data.cust_mode EQ 0)

{

/*default operation*/

call function for processing Refresh Commands

}

ElseIf (sim_data.cust_mode EQ 1)

{

/*sixTIes operation*/

sim_data.user_confirmation_expected = TRUE;

/*send Refresh command to MMI*/

PALLOC (sim_toolkit_ind, SIM_TOOLKIT_IND);

memset (sim_toolkit_ind, 0, sizeof (T_SIM_TOOLKIT_IND));

/* copy PDU into message ???????? */

PSENDX (MMI, sim_toolkit_ind);

/*REFRESH COMMAND NOW PROCESSED IN stk_sim_refresh_user_res() */

/*Terminate Command                                                 */

break;

}

ElseIf (sim_data.cust_mode EQ 2)

{

/*Next Custom Requirement*/

}

< PROCESS AS BEFORE >

NOTE: This code for processing the Refresh Commands will be placed in a separate

Function.

switch (cmd_qual)

{

-------

-------

}/*end of switch (cmd_qual)*/

-------

-------

-------

PFREE (sim_toolkit_ind)

Break; /* end of case STK_REFRESH */

Add STK_RES_SCREEN_IS_BUSY 0x01 (11.14 12.12.2 ) to sim_stk.c

#define STK_RES_EXT_DEF                  0x00

#define STK_RES_SCREEN_IS_BUSY           0x01

#define STK_RES_EXT_BUSY_CALL            0x02

#define STK_RES_EXT_NO_SERVICE           0x04

# Testing with Windows Simulation

The following tests need to be performed on the SIM entity:-

1) SIM_ACTIVATE_REQ primitive to be sent with cust_mode = 0.

Ensure that all current tests pass.

2) SIM_ ACTIVATE_REQ primitive to be sent with cust_mode=1.

Test Refresh File Change Notification

Test Refresh Reset

Test Refresh Sim Initialization and File Change Notification

3) Test error conditions. e.g.:-

SIM_REFRESH_USER_RES primitive received when cust_mode=0

SIM_REFRESH_USER_RES primitive received when cust_mode=1 but a Refresh command has not occurred (i.e. user_confirmation_expected EQ FALSE)

  - Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |