Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document – Confidential

GSM General Packet Radio Services

Message Sequence Charts

MMGMM

(Edit via File/Properties/Summary!)

| Document Number: | 8441.253.99.001 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.2 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1999-Jul-06 |
| Last changed: | 2003-May-15 by XGUTTEFE |
| File Name: | gsm-gprs.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1999-Jul-06 | DB, ANS |  | 0.1 |  | 1 |
| 2003-May-15 | XGUTTEFE |  | 0.2 | Draft |  |

Notes:

- Initial version

Table of Contents

1.1	References	5

1.2	Abbreviations	7

1.3	Terms	10

2	Overview	10

2.1	GRR (RLC/MAC) – Radio Link Control/Medium Access Control	11

2.2	LLC – Logical Link Control	11

2.3	GMM – GPRS Mobility Management	11

2.4	SM – Session Management	11

2.5	SNDCP - Subnetwork Dependant Convergence Protocol	11

2.6	GACI – GPRS AT Command Interpreter	11

2.7	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	11

3	Introduction	12

4	Protocol	12

5	Registration	12

5.1	Power ON	12

5.1.1	MGM000: Activation of MS as class A, BC, BG, CG mobile	12

5.1.2	MGM003: Activation of MS as a circuit switched only mobile	13

5.1.3	MGM001: Positive End of Registration Attempt (Limited Service)	14

5.1.4	MGM002: SIM inserted - initiate cell selection	14

5.2	Cell found – mobile class BG	15

5.2.1	MGM010: IMSI Cell found	15

5.2.2	MGM011: GPRS Cell not found	16

5.2.3	MGM012: GPRS Cell found in state GPRS deactivated	17

5.2.4	MGM013: GPRS Cell found in state GPRS deactivated – net mode III	19

5.2.5	MGM014: Change of net mode  from III to II	21

5.2.6	MGM015: Change of net mode  from III to I	23

5.2.7	MGM016 Change of net mode  from I to II	24

5.2.8	MGM017 Change of net mode  from I to III	27

5.2.9	MGM018: Automatic networck selection mode	29

5.3	Cell found – mobile class BC	30

5.3.1	MGM200: IMSI Cell found	30

5.3.2	MGM201: GPRS Cell not found	31

5.3.3	MGM202: GPRS Cell found in state GPRS deactivated	32

5.3.4	MGM203: GPRS Cell found in state GPRS deaktivated – net mode III	33

5.3.5	MGM206: Change of net mode  from III to a different one	34

5.3.6	MGM204: Change of net mode  from III to II	35

5.3.7	MGM205: Change of net mode from III to I	37

5.3.8	MGM207 Change of net mode  from I to II	38

5.3.9	MGM208 Change of net mode  from I to III	38

5.4	Mobile originated call	41

5.4.1	MGM300 Mobile originated call indicated – BG	41

5.4.2	MGM301 Mobile originated call indicated - BG	42

5.4.3	MGM302 Mobile originated call indicated – BC	44

5.4.4	MGM303 Mobile originated call indicated - BC	45

5.5	Mobile terminated call	46

MGM400: Mobile terminated call indicated - BG	46

MGM401: Mobile terminated call indicated - BG	48

MGM402: Mobile terminated call indicated - BC	50

5.5.1	MGM501: Cell Selection	52

5.6	Packet Access using CCCH	54

MGM403: Mobile originated packet access - B	54

5.6.1	MGM404: Mobile originated packet access - B	57

5.6.2	MGM405: Mobile originated packet access - B	60

5.6.3	MGM404: Mobile originated packet access - B	63

Packet Paging using CCCH	66

MGM407: Mobile terminated packet access – B??	66

Cell Response Procedure	66

5.6.4	MGM800: Normal Cell Change	66

5.6.5	MGM801: RA changed	67

5.6.6	MGM802: RAU w/o Cell Change	70

5.6.7	MGM803: Periodic RAU  is interupted by RAU change	72

5.6.8	MGM804: Periodic RAU  is interupted by LAU change	74

Appendices	77

A.	Acronyms	77

B.	Glossary	77

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 05.02 version 8.0.0 Release 1999
Digital cellular telecommunications system (Phase 2+);
Multiplexing and multiple access on the radio path

[2]	GSM 04.60 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station (MS) - Base Station System (BSS) interface;
Radio Link Control/ Medium Access Control (RLC/MAC) protocol

[3]	GSM 04.08 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface layer 3 specification

[4]	GSM 03.64 version 6.1.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Overall description of the GPRS radio interface; Stage 2

[5]	GSM 03.60 version 6.3.1 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Service description; Stage 2

[6]	GSM 04.07 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface signalling layer 3; General aspects

[7]	GSM 04.64 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station - Serving GPRS Support Node (MS-SGSN)
Logical Link Control (LLC) layer specification

[8]	GSM 05.08 version 6.4.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem link control

[9]	GSM 05.10 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem synchronization

[10]	GSM 03.20 TS 100 929: July 1998 (GSM 03.20 version 6.0.1)
Security related network functions, ETSI

[11]	Draft GSM 03.22: August 1998 (GSM 03.22 version 6.1.0)
Functions related to Mobile Station (MS) in idle mode and group receive mode, ETSI

[12]	GSM 04.65 V6.3.0: Subnetwork Dependant Convergence Protocol

ETSI, March 1999

[13]	ITU-T V42bis ITU-T, Recommendation V.42 bis 1990

[14]	GSM 09.60 GPRS Tunneling Protocol (GTP) across the Gn and Gp Interface

[15]	RFC 1661 IETF STD 51 July 1994
The Point-to-Point Protocol (PPP)

[16]	RFC 1662 IETF STD 51 July 1994
PPP in HDLC-like Framing

[17]	RFC 1570 January 1994
PPP LCP Extensions

[18]	RFC 1989 August 1996
PPP Link Quality Monitoring

[19]	RFC 1332 May 1992
The PPP Internet Protocol Control Protocol (IPCP)

[20]	RFC 1877 December 1995
PPP IPCP Extensions for Name Server Addresses

[21]	RFC 2153 May 1997
PPP Vendor Extensions

[22]	RFC 1334 October 1992
PPP Authentication Protocols (for Password Authentication Protocol only)

[23]	RFC 1994 August 1996
PPP Challenge Handshake Authentication Protocol (CHAP)

## Abbreviations

AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CHAP	Challenge Handshake Authentication Protocol
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DCOMP	Identifier of the user data compression algorithm used for the N-DPU
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

E	Extension bit
EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GACI	GPRS AT Command Interpreter
GMM	GPRS Mobility Management
GP	Guard Period
GRR	GPRS RR
GSM	Global System for Mobile Communication

HDLC	High-level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
IP	Internet Protocol
IPCP	Internet Protocol Control Protocol 
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Ciphering Key

L	Length Indicator
LAI	Location Area Information
LCP 	Link Control Protocol 
LISR	Low level Interrupt Service Routine
LLC	Logical Link Control
LPD	Link Protocol Discriminator
LQM	Link Quality Monitoring

M	More bit used to indicate the last segment of N-DPU
MAC	Medium Access Control
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MT	Mobile Termination

N(R)	Receive Number
N(S)	Send Number
NC	Network Control
NCC	National Colour Code
NCP 	Network Control Protocol
NECI	New Establishment Causes included
N-PDU	Network Protocol Data Unit
NSAPI	Network Layer Service Access Point Identifier

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PACCH	Packet Associated Control Channel
PAP	Password Authentication Protocol
PBCCH	Packet BCCH
PCCCH	Packet CCCH
PCOMP	Identifier of the protocol control information compression algorithm used for the N-DPU
PDCH	Packet Data Channel
PDP	Packet Data Protocol e.g. IP or X.25
PDTCH	Packet Data Traffic Channel
PRACH	Packet RACH
PSI	Packet System Information
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Data Unit
PL	Physical Layer
PLMN	Public Land Mobile Network
PPP 	Point-to-Point Protocol
PTP	Point to Point

QoS	Quality of Service

RACH	Random Access Channel
REJ	Reject Frame
RLC	Radio Link Control
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SDU	Service Data Unit
SGSN	Serving GPRS Support Node
SIM	Subscriber Identity Module
SM	Session Management
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SNDCP	Subnetwork Dependant Convergence Protocol
SNSM	SNDCP-SM
SS	Supplementary Services

TAP	Test Application Program
TBF	Temporary Block Flow
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TCP	Transmission Control Protocol
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TFI	Temporary Flow Identifier
TLLI	Temporary Logical Link Identifier
TMSI	Temporary Mobile Subscriber Identity
TQI	Temporary Queuing Identifier

UA	Unnumbered Acknowledgement Frame
UART	Universal Asynchronous Receiver Transmitter
UI	Unnumbered Information Frame
USF	Uplink State Flag

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visiting Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for GPRS consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity.

Figure 2-1: Architecture of the GSM/GPRS protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the GPRS protocol stack are:

## GRR (RLC/MAC) – Radio Link Control/Medium Access Control

This layer contains two functions: The Radio Link Control function provides a radio-solution-dependent reliable link. The Medium Access Control function controls the access signalling (request and grant) procedures for the radio channel, and the mapping of LLC frames onto the GSM physical channel.

## LLC – Logical Link Control

The LLC entity provides multiple highly reliable logical links for asynchronous data transfer between the MS and the network. It supports variable-length information frames, acknowledged and unacknowledged data transfer, flow and sequence control, error detection and recovery, notification of unrecoverable errors, user identity confidentiality, and ciphering of user data.

## GMM – GPRS Mobility Management

The GMM entity provides procedures for the mobility of the MS, such as informing the network of its present location, and user identity confidentiality. It manages the GMM context (attach, detach, routing area updating), supports security functions such as authentication of user and MS, controls ciphering of user data, and initiates the response to paging messages.

## SM – Session Management

The main function of the session management (SM) is to support PDP context handling of the user terminal. Session Management activates, modifies and deletes the contexts for packet data protocols (PDP). Session Management services are provided at the SMREG-SAP and the SNSM-SAP for anonymous and non-anonymous access. The non-anonymous and anonymous access procedures for PDP context activation and PDP context deactivation are available at the SMREG-SAP. In addition there exists a PDP context modification for non-anonymous PDP contexts.

## SNDCP - Subnetwork Dependant Convergence Protocol

- SNDCP carries out all functions related to transfer of Network layer Protocol Data Units (N-PDUs) over GPRS in a transparent way. SNDCP helps to improve channel efficiency by means of compression techniques. The set of protocol entities above SNDCP consists of commonly used network protocols. They all use the same SNDCP entity, which then performs multiplexing of data coming from different sources to be sent using the service provided by the LLC layer.

## GACI – GPRS AT Command Interpreter

The ACI is the GPRS extension of the ACI. It is specified in GSM 07.60. It is responsible for processing of the GPRS related AT Commands to setup, activate and deactivate the PDP context parameter. It also provides funtionallity for the interworking between GMM/SM/SNDCP and a packet oriented protocol like PPP.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the GPRS. The driver has to be able to perform flow control.

# Introduction

# Protocol

# Registration

## Power ON

### MGM000: Activation of MS as class A, BC, BG, CG mobile

Description:

GMM is in state GMM-NULL.NO-IMSI. MMI activates the MS as an mobile with the appropriate class.

Variant A: mobile class A

Variant B: mobile class BC (class B mobile which reverts to CS in net mode III)

Variant C: mobile class BG (class B mobile which reverts to GPRS in net mode III)

Variant D: mobile class CG (GPRS-only mobile)

Variant E: mobile class CC (CS-only mobile)

Variants:

<A>....<E>

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

(1)|GMMREG_ATTACH_REQ (mobile class)                                         |

*============>*|                                            |

(2)|              |MM_ACTIVATE_RR_REQ                          |

|              *=============>*

(3)|                             |RR_ACTIVATE_REQ (GPRS)                      |

|                             *===========================================>*

(4)|              |GMMRR_ENABLE_REQ                            |

|              *===========================================>*

|              |              |              |              |              |              |              |

- GMMREG_ATTACH_REQ
<A>	mobile_class	GMMREG_AT_CLASS_A
<B>	mobile_class	GMMREG_AT_CLASS_BC
<C>	mobile_class	GMMREG_AT_CLASS_BG
<D>	mobile_class	GMMREG_AT_CLASS_CG
	t3314_ready_val	T3314
	t3312_standby_rau_val	T3312

- MM_ACTIVATE_RR_REQ

- RR_ACTIVATE_REQ
	plmn	PLMN_NO_ID
	op	OP_MODE_NO_SIM_NO_SERV
	cksn	CKSN_RES
	kcv	KC_DELETED
	accc	ACC_CLASS_0000
	imsi	MOB_ID_NO_ID
	tmsi	MOB_ID_NO_ID
	thplmn	THPLMN_FF
	bcch_info	NOT_USED
	gprs	RR_GPRS_ON

- GMMRR_ENABLE_REQGMMREG_ATTACH_REQ
<A>	mobile_class	GMMREG_AT_CLASS_A
<B>	mobile_class	GMMREG_AT_CLASS_BC
<C>	mobile_class	GMMREG_AT_CLASS_BG
<D>	mobile_class	GMMREG_AT_CLASS_CG

History:

10-Apr-00	ANS	Initial

25-Apr-00	ANS	Class CC removed.

### MGM003: Activation of MS as a circuit switched only mobile

Description:

GMM is in state GMM-NULL.NO-IMSI. MMI activates the MS as an CS-only mobile.

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

(1)|GMMREG_ATTACH_REQ (mobile class CC)                                       |

*============>*|                                                           |

(2)|              |MMGMM_REG_REQ ( REG_GPRS_INACTIVE )                                         |

|              *=============>*                                            |

|                             |RR_ACTIVATE_REQ                             |

|                             *===========================================>*

|              |              |              |              |              |              |              |

- GMMREG_ATTACH_REQ
	mobile_class	GMMREG_AT_CLASS_CC
	t3314_ready_val	T3314
	t3312_standby_rau_val	T3312

- MM_ACTIVATE_RR_REQ

- RR_ACTIVATE_REQ
	plmn	PLMN_NO_ID
	op	OP_MODE_NO_SIM_NO_SERV
	cksn	CKSN_RES
	kcv	KC_DELETED
	accc	ACC_CLASS_0000
	imsi	MOB_ID_NO_ID
	tmsi	MOB_ID_NO_ID
	thplmn	THPLMN_FF
	bcch_info	NOT_USED

- GMMRR_ENABLE_REQGMMREG_ATTACH_REQ
	mobile_class	GMMREG_AT_CLASS_CC

History:

25-Apr-00	ANS	Initial

### MGM001: Positive End of Registration Attempt (Limited Service)

Description:

MS class BG.

MM is informed by means of a RR-ACTIVATE confirmation primitive that the mobile station is synchronous to a cell. The network identification if forwarded to GMM and furter to MMI as part of a GMMREG_ATTACH_REJ indication primitive.

Preamble:

MGM000b

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                             |RR_ACTIVATE_CNF (limited service, no SIM )  |

|                             *<===========================================*

|              |MMGMM_NREG_IND (LIMITED)

|              *<=============*

|GMMREG_ATTACH_REJ (CAUSE IMSI LIMITED ONLY)

*<=============*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

13-Apr-00	DB	Corrected direction of MM_ACTIVATE_RR_CNF

### MGM002: SIM inserted - initiate cell selection

Description:

MS class BG.

GMM is in state GMM-DEREGISTERED.NO-IMSI. GMM receives the indication that a SIM has been inserted and that the SIM is valid. GMM resets the GPRS attach attempt counter. GMM enters state GMM-DEREGISTERED.PLMN-SEARCH. SIM sends the stored temporary data, if any, and the update states to MM and GMM.

Preamble:

MGM001

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                             |SIM_MM_INSERT_IND                                          |

|                             *<==========================================================*

|              |GMM_SIM_INSERTED_IND                                                      |

|              *<=========================================================================*

|              |MMGMM_ACTIVATE_RR_REQ                                   |

|              *=============>*                                         |

|                             |RR_ACTIVATE_REQ ( GPRS )                 |

|                             *========================================>*

History:

10-Apr-00	ANS	Initial

## Cell found – mobile class BG

### MGM010: IMSI Cell found

Description:

MS class BG.

Successful conclusion of cell selection is signalled by the receipt of a RR-ACTIVATE confirmation primitive. MM forwards the PLMN identification to GMM and GMM to MMI in the form of a GMMREG_ATTACH_IND confirmation primitive.

Preamble:

MGM002

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                             |RR_ACTIVATE_CNF                             |

|                             *<===========================================*

|              |MMGMM_ACTIVATE_RR_CNF                                      |

|              *<=============*

History:

10-Apr-00	ANS	Initial

### MGM011: GPRS Cell not found

Description:

MS class BG.

No GPRS cell was found by RR. RR informs GRR that the current cell does not support GPRS. GRR passes this information to GMM and GMM switches the main actvities to the GSM side with the MM_ACTIVATE_MM_REQ primitve. MM has to ast as an GSM-only mobile untlil the MM_DEACTVATE_MM_REQ primitive is received. That means, MM has to start the attach procedure on its own. After successful location update (LAU) MM informs GMM and not MMI about the result. GMM forwards this information to MMI.

If the LAI is changed, MM forwards GMM the LAI.

Preamble:

MGM010

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                                                           |RRGRR_GPRS_IND (GPRS NOT SUPPORTED)         |

|                                                           *<=============*

|              |GMMRR_NO_CELL_IND                           |

|              *<===========================================*

|              |MMGMM_REG_REQ (REG_GPRS_INACTIVE)

|              *=============>*

*<   LAU   >

|              |MMGMM_REG_CNF |

|              *<=============*

|GMMREG_ATTACH_IND (IMSI)

*<=============*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

13-Apr-00	DB	Corrected GMMREG_ATTACH_IND

### MGM012: GPRS Cell found in state GPRS deactivated

Description:

MS class BG.

RR informs GRR about finding a GPRS cell.

Preamble:

MGM011

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                                                           |RRGRR_GPRS_IND (GPRS)

|                                                           *<=============*

|                                                           |RRGRR_SI13_IND  (PBCCH)

|                                                           *<=============*

|                                                        (GRR 1)

|                                                           |RRGRR_STOP_MON_BCCH_REQ

|                                                           *=============>*

|                                                        (GRR 2)

|                                                           |              |  MPH_MON_CTRL_REQ(BCCH)     |

|                                                           |              *============================>*

|                                                           |            (RR 1)

|                                                           |RRGRR_STOP_MON_CCCH_REQ

|                                                           *=============>*

|                                                        (GRR 3)

|                                                           |              |  MPH_MON_CTRL_REQ(CCCH)     |

|                                                           |              *============================>*

|                                                           |            (RR 2)

|                                                           |MPHP_SCELL_PBCCH_REQ                        |

|                                                           *===========================================>*

|                                                           |MPHP_DATA_IND (PSI1)                        |

|                                                           *<===========================================*

|                                                           |MPHP_DATA_IND (PSI2)                        |

|                                                           *<===========================================*

|              |              |              |              |              |              |              |

(GRR 1)

RR sends the SI13 to GRR. The SI13 indicates that a PBCCH is available.

(GRR 2)

GRR stops BCCH reading in idle mode.

(RR 1)

tbd

(GRR 3)

GRR stops CCCH listening in idle mode.

(RR 2)

tbd

History:

10-Apr-00	ANS	Initial

24-Jul-00	MPA	revised

### MGM013: GPRS Cell found in state GPRS deactivated – net mode III

Description:

MS class BG.

GRR informs GMM that a cell in network mode of operation III was found.

MM is deactivated and is requested to start its own IMSI detach procedure. When the detach procedure is finished MM forwards the result to GMM and MMI is informed. GMM starts the normal attach procedure.

Preamble:

MGM012

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|        	      |GMMRR_CELL_IND (net mode III)               |

|              *<===========================================*

|        	      |GMMRR_SUSPEND_REQ ( deactivate while DETACH)|

|              *===========================================>*

|                                                           |RRGRR_ACTIVATE_REQ

|                                                           *=============>*

|        	      |GMMRR_SUSPEND_CNF                           |

|              *<===========================================*

|              |MMGMM_NREG_REQ (detach not done)

|              *=============>*

|              |MMGMM_NREG_CNF

|              *<=============*

|GMMREG_DETACH_IND (nothing attached)

*<=============*

|        	      |GMMRR_RESUME_REQ                            |

|              *===========================================>*

|                                                           |RRGRR_SI13_IND(PBCCH)

|                                                           *<=============*

|                                                           |RRGRR_STOP_MON_REQ

|                                                           *=============>*

|                                                           |MPHP_SCELL_PBCCH_REQ                        |

|                                                           *===========================================>*

|                                                           |MPHP_DATA_IND (PSI1)                        |

|                                                           *<===========================================*

|                                                           |MPHP_DATA_IND (PSI2)                        |

|                                                           *<===========================================*

|        	      |GMMRR_CELL_IND (net mode not changed)       |

|              *<===========================================*

|              |GMMRR_ASSIGN_REQ (random TLLI, PTMSI, TMSI)

|              *===========================================>*

|              |LLGMM_ASSIGN_REQ (random TLLI)

|              *============================>*

|              |              |              |              |              |              |              |

|              |LL_UNITDATA_REQ (ATTACH_REQUEST (GPRS only))

|              *============================>*

|              |LL_UNITDATA_IND (ATTACH_CONFIRM)

|              *<============================*

|              |              |              |              |              |              |              |

|              |GSIM_UPDATE_REQ  (GU1, PTMSI)                                             |

|              *=========================================================================>*

|              |GMMRR_ASSIGN_REQ ( new TLLI, PTMSI,TMSI)    |

|              *===========================================>*

|              |LLGMM_ASSIGN_REQ ( new TLLI )|

|              *============================>*

|GMMEG_ATTACH_IND (GPRS only)

*<=============*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

24-Jul-00	MPA	revised

### MGM014: Change of net mode  from III to II

Description:

MS class BG.

GPRS-only attached.

GRR changes the routing area and the network mode of operation changes from III to II.  GMM suspends GRR and requestes MM to perform the IMSI attach proceduere with indicating the result to GMM at the end. MM is still in state DEACTIVATED. After location update proceudre GRR is resumed, GRR selects all GPRS cell relevant data and passes this data with GMMRR_CELL_IND to GMM. GMM performs the normal routing area update procedure and indicates the result to MMI.

Preamble:

MGM013

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

....(GlumPs)

|        	                     | RR_ACCTIVATE_IND                           |

|                             *<===========================================*

|              |MMGMM_ACTIVATE_IND

|              *<=============*

|        	      |GMMRR_CELL_IND ( net mode II)               |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ ( unassign TLLI )

|              *============================>*

|        	      |GMMRR_SUSPEND_REQ ( deactivate while LAU )  |

|              *===========================================>*

|                                                           |RRGRR_ACTIVATE_REQ

|                                                           *=============>*

|        	      |GMMRR_SUSPEND_CNF                           |

|              *<===========================================*

|              |MMGMM_REG_REQ (REG_REMOTE_CONTROLLED)

|              *=============>*

|              *< LAU >

|              |MMGMM_REG_CNF

|              *<=============*

|        	      |GMMRR_RESUME_REQ                            |

|              *===========================================>*

|                                                           | RRGRR_SI13_IND (BCCH)

|                                                           *<=============*

|                                                           | RRGRR_START_MON_BCCH_REQ

|                                                           *=============>*

|        	      |GMMRR_CELL_IND ( net mode NOT changed)      |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ ( assign last TLLI again )

|              *============================>*

|              |LL_UNITDATA_REQ (Normal RAU) |

|              *============================>*

|              |LL_UNITDATA_REQ (RAU_CNF (PTMSI))

|              *<============================*

|              |GSIM_UPDATE_REQ  (GU1, PTMSI)                                             |

|              *=========================================================================>*

|              |GMMRR_ASSIGN_REQ ( new TLLI generated from PTMSI, PTMSI,TMSI)

|              *===========================================>*

|              |LLGMM_ASSIGN_REQ ( new TLLI )|

|              *============================>*

|GMMREG_ATTACH_CNF (GPRS and CS attached)

*<=============*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

25-May-00	ANS	??_activate_ind added

24-Jul-00	MPA	revised

### MGM015: Change of net mode  from III to I

Description:

MS class BG.

GPRS-only attached.

GRR changes the routing area and the network mode of operation changes from III to I.  GMM performs the combined routing area update procedure and indicates the result to MM and MMI. MM sets the update state to U1 resets the location update attempt counter and gets the new TMSI. The TMSI is passed to SIM and the PTMSI is passed to GSIM.

Preamble:

MGM013

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

... (GlumPs)

|        	                     | RR_ACCTIVATE_IND                           |

|                             *<===========================================*

|              |MMGMM_ACTIVATE_IND

|              *<=============*

|        	      |GMMRR_CELL_IND (net mode I)                 |

|              *<===========================================*

|              |MMGMM_ATTACH_STARTED_REQ

|              *=============>*

|              |LLGMM_SUSPEND_REQ            |

|              *============================>*

|              |LL_UNITDATA_REQ (Combined RAU with IMSI attach)

|              *============================>*

|              |LL_UNITDATA_REQ (RAU_CNF (PTMSI))

|              *<============================*

|              |LLGMM_RESUME_REQ             |

|              *============================>*

|              |GSIM_UPDATE_REQ  (GU1, PTMSI)                                             |

|              *=========================================================================>*

|              |GMMRR_ASSIGN_REQ ( new TLLI, PTMSI,TMSI)    |

|              *===========================================>*

|              |LLGMM_ASSIGN_REQ ( new TLLI )|

|              *============================>*

|              |MMGMM_ATTACH_ACC_REQ ( new TMSI)

|              *=============>*

|                             |SIM_MM_UPDATE_REQ ( TMSI)

|                             *==========================================================>*

|GMMREG_ATTACH_CNF (GPRS and CS attached)

*<=============*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

25-May-00	ANS	??_activate_ind added

### MGM016 Change of net mode  from I to II

Description:

MS class BG.

net mode I.

Combined attached.

GRR changes the routing area and the network mode of operation changes from I to II.  GMM suspends GRR to request MM to perform location update on its own procedures. After finishing location update GMM resumes GRR and performs normal routing area update.

Preamble:

MGM015

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

... (GlumPs)

|        	      |GMMRR_CELL_IND (net mode II)                |

|              *<===========================================*

|        	                     | RR_ACCTIVATE_IND                           |

|                             *<===========================================*

|              |MMGMM_ACTIVATE_IND

|              *<=============*

|              |LLGMM_ASSIGN_REQ ( unassign TLLI )

|              *============================>*

|        	      |GMMRR_SUSPEND_REQ ( deactivate while LAU )  |

|              *===========================================>*

|                                                           |RRGRR_ACTIVATE_REQ

|                                                           *=============>*

|        	      |GMMRR_SUSPEND_CNF                           |

|              *<===========================================*

|              |MMGMM_REG_REQ (REG_REMOTE_CONTROLLED)

|              *=============>*

|              *< LAU >

|              |MMGMM_REG_CNF

|              *<=============*

|        	      |GMMRR_RESUME_REQ                            |

|              *===========================================>*

|                                                           |RRGRR_SI13_IND (BCCH)

|                                                           *<=============*

|                                                           | RRGRR_START_MON_BCCH_REQ

|                                                           *=============>*

|        	      |GMMRR_CELL_IND ( net mode NOT changed)      |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ ( assign last TLLI again )

|              *============================>*

|              |LL_UNITDATA_REQ (Normal RAU) |

|              *============================>*

|              |LL_UNITDATA_REQ (RAU_CNF (PTMSI))

|              *<============================*

|              |GSIM_UPDATE_REQ  (GU1, PTMSI)                                             |

|              *=========================================================================>*

|              |GMMRR_ASSIGN_REQ ( new TLLI, PTMSI,TMSI)    |

|              *===========================================>*

|              |LLGMM_ASSIGN_REQ ( new TLLI )|

|              *============================>*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

25-May-00	ANS	??_activate_ind added

24-Jul-00	MPA	revised

### MGM017 Change of net mode  from I to III

Description:

MS class BG.

net mode I.

Combined attached.

GRR changes the routing area and the network mode of operation changes from I to III.  GMM suspends GRR to request MM to perform location update by its own procedures. After finishing location update GMM resumes GRR and performs normal routing area update.

Preamble:

MGM015

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                                                           |MPHP_DATA_IND (PSI1)                        |

|                                                           *<===========================================*

|                                                           |MPHP_DATA_IND (PSI2)                        |

|                                                           *<===========================================*

|        	      |GMMRR_CELL_IND (net mode III)               |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ ( unassign TLLI )

|              *============================>*

|        	      |GMMRR_SUSPEND_REQ ( deactivate while LAU)   |

|              *===========================================>*

|                                                           |RRGRR_ACTIVATE_REQ

|                                                           *=============>*

|                                                           |

|        	      |GMMRR_SUSPEND_CNF                           |

|              *<===========================================*

|              |MM	GMM_NREG_REQ ( MMGMM_DISABLE, MMGMM_PERFORM_DETACH )

|              *=============>*

|              *< IMSI DETACH >

|              |MMGMM_NREG_IND

|              *<=============*

|        	      |GMMRR_RESUME_REQ                            |

|              *===========================================>*

|                                                           |RRGRR_SI13_IND (PBCCH)

|                                                           *<=============*

|                                                           |RRGRR_STOP_MON_BCCH_REQ

|                                                           *=============>*

|                                                           |RRGRR_STOP_MON_CCCH_REQ

|                                                           *=============>*

|                                                           |MPHP_DATA_IND (PSI1)                        |

|                                                           *<===========================================*

|                                                           |MPHP_DATA_IND (PSI2)                        |

|                                                           *<===========================================*

|        	      |GMMRR_CELL_IND ( net mode NOT changed)      |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ ( assign last TLLI again )

|              *============================>*

|              |LL_UNITDATA_REQ (Normal RAU) |

|              *============================>*

|              |LL_UNITDATA_REQ (RAU_CNF PTMSI)

|              *<============================*

|              |GSIM_UPDATE_REQ  (GU1, PTMSI)                                             |

|              *=========================================================================>*

|              |GMMRR_ASSIGN_REQ ( new TLLI, PTMSI,TMSI)    |

|              *===========================================>*

|              |LLGMM_ASSIGN_REQ ( new TLLI )|

|              *============================>*

|GMMREG_ATTACH_CNF (GPRS attached)

*<=============*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

24-Jul-00 	MPA	revised

### MGM018: Automatic networck selection mode

Description:

MS class BG.

Preamble:

MGM002

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                             |RR_ABORT_IND  (PLMN list)                   |

|                             *<===========================================*

|                             |RR_ACTIVATE_REQ ( first PLMN )              |

|                             *===========================================>*

|                             |RR_ACTIVATE_CNF                             |

|                             *<===========================================*

|              |MMGMM_ACTIVATE_CNF                                         |

|              *<=============*

<attach>

|              |MMGMM_NET_REQ ( take next plmn)

|              *=============>*

|                             |RR_ACTIVATE_REQ (second PLMN )              |

|                             *===========================================>*

|                             |RR_ACTIVATE_CNF                             |

|                             *<===========================================*

|              |MMGMM_ACTIVATE_CNF                                         |

|              *<=============*

<attach>

|              |MMGMM_NET_REQ ( take next plmn)

|              *=============>*

|              |MMGMM_NREG_IND|

|              *<=============*

|GMMREG_ATTACH_REJ

*<=============*

|

History:

26-Apr-00	ANS	Initial

## Cell found – mobile class BC

### MGM200: IMSI Cell found

Description:

MMI switches to class BC mobile.

Successful conclusion of cell selection is signalled by the receipt of a RR-ACTIVATE confirmation primitive.

Preamble:

MGM013

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|GMMREG_ATTACH_REQ  (Class BC)                              |

*============>*|                                            |

|              |MMGMM_ACTIVATE_REQ

|              *=============>*

|                             |RR_ACTIVATE_REQ (GPRS)                      |

|                             *===========================================>*

|                             |RR_ACTIVATE_CNF                             |

|                             *<===========================================*

|              |MMGMM_ACTIVATE_CNF                                         |

|              *<=============*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

### MGM201: GPRS Cell not found

Description:

MS class BC.

NO GPRS cell was found by RR. RR informs GRR that the current cell does not support GPRS. GRR passes this information to GMM and GMM switches the main actvities to the GSM side with the MM_ACTIVATE_MM_REQ primitve. MM has to ast as an GSM-only mobile untlil the MM_DEACTVATE_MM_REQ primitive is received. That means, MM has to start the attach procedure on its own. After successful location update (LAU) MM informs GMM and not MMI about the result. GMM forwards this information to MMI.

If the LAI is changed, MM forwards GMM the LAI.

Preamble:

MGM200

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                                                           |RRGRR_GPRS_IND (GPRS NOT SUPPORTED)         |

|                                                           *<=============*

|              |GMMRR_NO_CELL_IND                           |

|              *<===========================================*

|              |MMGMM_REG_REQ (REG_GPRS_INACTIVE)

|              *=============>*

*<   LAU   >

|              |MMGMM_REG_CNF    |

|              *<=============*

|GMMEG_ATTACH_IND (IMSI)

*<=============*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

### MGM202: GPRS Cell found in state GPRS deactivated

Description:

MS class BC.

IMSI only attached.

RR informs GRR about finding a GPRS cell. GMM requests GRR to search for a cell which supports a different net mode than net mode III.

Preamble:

MGM201

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                             |RR_ACTIVATE_IND (LAI)                       |

|                             *<===========================================*

|              |MMGMM_ACTIVATE_IND (LAI)                                   |

|              *<=============*

|        	      |GMMRR_RESUME_REQ (search net mode different from III)

|              *===========================================>*

|                                                           |MPHP_DATA_IND (GPRS_RESUME)                 |

|                                                           *===========================================>*

|                                                           |GPRS_IND (GPRS supported)

|                                                           *<=============*

|                                                           |RRGRR_SI13_IND (PBCCH)

|                                                           *<=============*

|                                                           |RRGRR_STOP_MON_BCCCH_REQ

|                                                           *=============>*

|                                                           |RRGRR_STOP_MON_CCCCH_REQ

|                                                           *=============>*

|                                                           |MPHP_DATA_IND (PSI1)                        |

|                                                           *<===========================================*

|                                                           |MPHP_DATA_IND (PSI2)                        |

|                                                           *<===========================================*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

15-May-00	ANS	MM_DEACTIVATE_REQ removed.

### MGM203: GPRS Cell found in state GPRS deaktivated – net mode III

Description:

MS class BC.

IMSI only attached.

GRR informs GMM that a cell in network mode of operation III was found.

Preamble:

MGM202

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|        	      |GMMRR_CELL_IND ( net mode III)              |

|              *<===========================================*

|              |GMMRR_SUSPEND_REQ ( net mode III )

|              *===========================================>*

|                                                           |RRGRR_START_MON_REQ

|                                                           *=============>*

|        	      |GMMRR_SUSPEND_CNF                           |

|              *<===========================================*

|              |MMGMM_REG_REQ (REG_REMOTE_CONTROLLED)

|              *=============>*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

15-May-00	ANS	MM_ACTIVATE_MM replaced with MMGMM_REG_REQ

### MGM206: Change of net mode  from III to a different one

Description:

MS class BC.

IMSI-only attached.

RR indicates that a new cell was selected. MM forwards the new LAI to GMM. If the LAI changed then GMM  resumes GRR with search for network of operation request. GRR responses with GMMRR_CELL_IND.

Variant A:	net mode I found

Variant B:	net mode II found

Preamble:

MGM203

Variants:

<A>....<B>

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                             |RR_ACTIVATE_IND                             |

|                             *<===========================================*

|              |MMGMM_ACTIVATE_IND (LAI)                                   |

|              *<=============*

|        	      |GMMRR_RESUME_REQ (search net mode different from III)

|              *===========================================>*

|                                                           |GPRS_IND (GPRS supported)

|                                                           *<=============*

|                                                           |RRGRR_SI13_IND (PCCCH)

|                                                           *<=============*

|                                                           |RRGRR_STOP_MON_REQ

|                                                           *=============>*

|                                                           |MPHP_DATA_IND (PSI1)                        |

|                                                           *<===========================================*

|                                                           |MPHP_DATA_IND (PSI2)                        |

|                                                           *<===========================================*

|        	      |GMMRR_CELL_IND ( net mode )                 |

|              *<===========================================*

History:

10-Apr-00	ANS	Initial

15-May-00	ANS	MM_DEACTIVATE_MM removed

### MGM204: Change of net mode  from III to II

Description:

MS class BC.

IMSI-only attached.

MM deactivated.

GRR has changed the routing area and the network mode of operation changes from III to II. GMM suspends GRR to request MM to perform location update procedure. At the end of this procedure GMM switches on GRR again an perform th GPRS attach procedure.

Preamble:

MGM206B

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|        	      |GMMRR_SUSPEND_REQ ( deactivate while LAU )  |

|              *===========================================>*

|                                                           |MPHP_DATA_IND (GPRS_SUSPEND)                |

|                                                           *===========================================>*

|                                                           |RRGRR_START_MON_REQ

|                                                           *=============>*

|                                                           |

|        	      |GMMRR_SUSPEND_CNF                           |

|              *<===========================================*

|              |MMGMM_REG_REQ (REG_REMOTE_CONTROLLED)

|              *=============>*

|              *< LAU >

|              |MMGMM_REG_CNF

|              *<=============*

|        	      |GMMRR_RESUME_REQ                            |

|              *===========================================>*

|                                                           |RRGRR_STOP_MON_REQ

|                                                           *=============>*

|                                                           |RRGRR_SI13_IND (CCCH)

|                                                           *<=============*

|                                                           |MPHP_DATA_IND (PSI1)                        |

|                                                           *<===========================================*

|                                                           |MPHP_DATA_IND (PSI2)                        |

|                                                           *<===========================================*

|        	      |GMMRR_CELL_IND ( net mode NOT changed)      |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ ( foreign TLLI )

|              *============================>*

|              |LL_UNITDATA_REQ (Normal ATTACH)

|              *============================>*

|              |LL_UNITDATA_REQ (ATTACH_CNF PTMSI)

|              *<============================*

|              |GSIM_UPDATE_REQ  (GU1, PTMSI)                                             |

|              *=========================================================================>*

|              |GMMRR_ASSIGN_REQ ( new TLLI, PTMSI,TMSI)    |

|              *===========================================>*

|              |LLGMM_ASSIGN_REQ ( new TLLI )|

|              *============================>*

|GMMREG_ATTACH_CNF (combined ATTACHED)

*<=============*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

15-May-00	ANS	MMGMM_REG_ substituted

### MGM205: Change of net mode from III to I

Description:

MS class BC.

IMSI-only attached.

MM deactivated.

GRR has changed the routing area and the network mode of operation changes from III to I.  GMM performs the combined routing area update procedure and indicates the result to MM and MMI. MM sets the update state to U1 resets the location update attempt counter and gets the new TMSI. The TMSI is passed to SIM and the PTMSI is passed to GSIM.

Preamble:

MGM206A

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|              |MMGMM_ATTACH_STARTED_REQ

|              *=============>*

|              |LL_UNITDATA_REQ (Combined RAU with IMSI attach)

|              *============================>*

|              |LL_UNITDATA_REQ (RAU_CNF)    |

|              *<============================*

|              |GMMRR_ASSIGN_REQ ( new TLLI, PTMSI,TMSI)    |

|              *===========================================>*

|              |GSIM_UPDATE_REQ  (GU1, PTMSI)                                             |

|              *=========================================================================>*

|              |LLGMM_ASSIGN_REQ ( new TLLI )|

|              *============================>*

|              |MMGMM_ATTACH_ACC_REQ ( new TMSI)

|              *=============>*

|                             |SIM_MM_UPDATE_REQ (U1, TMSI)

|                             *==========================================================>*

|GMMREG_ATTACH_CNF (combined ATTACHED)

*<=============*

|              |              |              |              |              |              |              |

History:

10-Apr-00	ANS	Initial

15-May-00	ANS	attach_started and attach_acc included.

### MGM207 Change of net mode  from I to II

Description:

MS class BG.

net mode I.

Combined attached.

see MGM016.

### MGM208 Change of net mode  from I to III

Description:

MS class BC.

net mode I.

Combined attached.

GRR changes the routing area and the network mode of operation changes from I to III.  GMM suspends GRR to request MM to perform location update by its own procedures. After finishing location update GMM resumes GRR and performs normal detach procedure.

Preamble:

MGM205

MMI            GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                                                           |RRGRR_SI13_IND (CCCH)

|                                                           *<=============*

|                                                           |MPHP_DATA_IND (PSI1)                        |

|                                                           *<===========================================*

|                                                           |MPHP_DATA_IND (PSI2)                        |

|                                                           *<===========================================*

|        	      |GMMRR_CELL_IND (net mode III)               |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ ( unassign TLLI )

|              *============================>*

|        	      |GMMRR_SUSPEND_REQ ( deactivate while LAU )  |

|              *===========================================>*

|                                                           |MPHP_DATA_IND (GPRS_SUSPEND)                |

|                                                           *===========================================>*

|                                                           |RRGRR_START_MON_REQ

|                                                           *=============>*

|                                                           |

|        	      |GMMRR_SUSPEND_CNF                           |

|              *<===========================================*

|              |MMGMM_REG_REQ (REG_REMOTE_CONTROLLED)

|              *=============>*

|              *< LAU >

|              |MMGMM_REG_CNF

|              *<=============*

|        	      |GMMRR_RESUME_REQ                            |

|              *===========================================>*

|                                                           |RRGRR_STOP_MON_REQ

|                                                           *=============>*

|                                                           |RRGRR_SI13_IND (CCCH)

|                                                           *<=============*

|                                                           |MPHP_DATA_IND (PSI1)                        |

|                                                           *<===========================================*

|                                                           |MPHP_DATA_IND (PSI2)                        |

|                                                           *<===========================================*

|        	      |GMMRR_CELL_IND ( net mode NOT changed)      |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ ( assign old TLLI again )

|              *============================>*

|              |LL_UNITDATA_REQ (Normal DETACH)

|              *============================>*

|              |LL_UNITDATA_REQ (DETACH_CNF) |

|              *<============================*

|              |LLGMM_ASSIGN_REQ ( unassign TLLI )

|              *============================>*

|GMMREG_DETACH_IND (IMSI attached, cause net mode III)

*<=============*

|              |MMGMM_REG_REQ (REG_GPRS_INACTIVE)

|              *=============>*

|              |              |              |              |              |              |              |

History:

11-Apr-00	ANS	Initial

15-May-00	ANS	MMGMM_REG_REQ appended.

## Mobile originated call

### MGM300 Mobile originated call indicated – BG

Description:

MS class BG.

net mode III.

GPRS attached.

The trigger from connection management is forwarded to GMM. GMM rejeccts the establish request, because of the mobile class in net mode III.

Preamble:

MGM013

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|MMSMS_ESTABLISH_REQ          |

|MMSS_ESTABLISH_REQ           |

|MMCC_ESTABLISH_REQ           |

*============================>*

|              |MMGMM_CM_ESTABLISH_IND

|              *<=============*

|              |MMGMM_CM_ESTABLISH_REJ

|              *=============>*

|MMSMS_RELEASE_IND            |

|MMSS_RELEASE_IND             |

|MMCC_RELEASE_IND             |

*<============================*

|              |              |              |              |              |              |              |

History:

11-Apr-00	ANS	Initial

### MGM301 Mobile originated call indicated - BG

Description:

MS class BG.

net mode II.

Combined attached.

The trigger from connection management is forwarded to GMM. GMM suspends the GPRS side and switches to GSM by confirming the cm_establich primitive.

Preamble:

MGM014

MMI/CM           GMM             MM            LLC            GRR            RR             SIM     DL      PL

|              |              |              |              |              |              |       |      |

|MMSMS_ESTABLISH_REQ          |

|MMSS_ESTABLISH_REQ           |

|MMCC_ESTABLISH_REQ           |

*============================>*

|              |MMGMM_CM_ESTABLISH_IND

|              *<=============*

|GMMREG_SUSPEND_IND (call started)

*<=============*

|              |LLGMM_ASSIGN_REQ ( unassign TLLI )

|              *============================>*

|        	      |GMMRR_SUSPEND_REQ ( call started )          |

|              *===========================================>*

a)

|                                                           | RRGRR_STOP_MON_BCCH_REQ

|                                                           *==============>*

b)

|                                                           | MPHP_STOP_PBCCH_REQ

|                                                           *==============>*

|                                                           | RRGRR_ACTIVATE_REQ

|                                                           *==============>*

|        	      |GMMRR_SUSPEND_CNF                           |

|              *<===========================================*

|              |MMGMM_CM_ESTABLISH_RES

|              *=============>*

|                             |RR_ESTABLISH_REQ (CM SERVICE REQUEST)       |

|                             *============================================*

|                             |                                          <* start connection establishement *>

|                             |                                            | DL_ESTABLISH_REQ

|                                                                          *=======================>*

|                             |                                            | DL_ESTABLISH_CNF

|                                                                          *<=======================*

|                             | RR ESTABLISH CNF                           |

|                             *<===========================================*

|                             |                                            | DL_DATA_REQ(GPRS SUSPENSION)

|                                                                          *=======================>*

|                             | RR DATA IND (CM SERVICE ACCEPT)            |

|                             *<===========================================*

|                             | RR SYNC IND (ciphering on)                 |

|                             *<===========================================*

|MMSMS_ESTABLISH_CNF          |

|MMSS_ESTABLISH_CNF           |

|MMCC_ESTABLISH_CNF           |

*<============================*

|MMSMS_RELEASE_REQ            |

|MMSS_RELEASE_REQ             |

|MMCC_RELEASE_REQ             | RR_RELEASE_IND (GlumPs ?GPRS resumption?)  |

*============================>*<===========================================*

|MMSMS_RELEASE_IND            |

|MMSS_RELEASE_IND             |

|MMCC_RELEASE_IND             |

*<============================*

|                             | MDL_RELEASE_REQ                                                          |

|                             *=========================================================================>*

channel release ... GlumPs

|              |MMGMM_CM_RELEASE_IND

|              *<=============*

|        	      |GMMRR_RESUME_REQ                            |

|              *===========================================>*

|                                                           |RRGRR_SI13_IND (BCCH)

|                                                           *<=============*

|        	      |GMMRR_CELL_IND ( net mode not changed)      |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ ( assign last TLLI again)

|              *============================>*

|GMMREG_RESUME_IND (call ended)

*<=============*

|              |              |              |              |              |              |              |

History:

11-Apr-00	ANS	Initial

### MGM302 Mobile originated call indicated – BC

Description:

MS class BC.

net mode III.

IMSI attached.

The trigger from connection management is forwarded to GMM. GMM confirms MM the establish request and MM perform the mobile originated call procedure.

Preamble:

MGM203

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|MMSMS_ESTABLISH_REQ          |

|MMSS_ESTABLISH_REQ           |

|MMCC_ESTABLISH_REQ           |

*============================>*

|              |MMGMM_CM_ESTABLISH_IND

|              *<=============*

|              |MMGMM_CM_ESTABLISH_RES

|              *=============>*

|                             |RR_ESTABLISH_REQ (CM SERVICE REQUEST)       |

|                             *============================================*

|                             | RR_ESTABLISH_CNF                           |

|                             *<===========================================*

|                             | RR_DATA_IND (CM SERVICE ACCEPT)            |

|                             *<===========================================*

|                             | RR_SYNC_IND (ciphering on)                 |

|                             *<===========================================*

|MMSMS_ESTABLISH_CNF          |

|MMSS_ESTABLISH_CNF           |

|MMCC_ESTABLISH_CNF           |

*<============================*

|MMSMS_RELEASE_REQ            |

|MMSS_RELEASE_REQ             |

|MMCC_RELEASE_REQ             | RR_RELEASE_IND                             |

*============================>*<===========================================*

|MMSMS_RELEASE_IND            |

|MMSS_RELEASE_IND             |

|MMCC_RELEASE_IND             |

*<============================*

|                             | MDL_RELEASE_REQ                                                          |

|                             *=========================================================================>*

|              |MMGMM_CM_RELEASE_IND

|              *<=============*

|              |              |              |              |              |              |              |

History:

12-Apr-00	ANS	Initial

### MGM303 Mobile originated call indicated - BC

Description:

MS class BC.

net mode II.

Combined attached.

equivalent to MGM301.

Preamble:

MGM204

History:

12-Apr-00	ANS	Initial

## Mobile terminated call

### MGM400: Mobile terminated call indicated - BG

Description:

MS class BG.

net mode I.

Combined attached.

GRR receives a paging request for CS and indicates it to GMM. GMM suspends the GPRS side and confirms the paging request. RR follows the normal CS paging procedure.

Preamble:

MGM015

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                                                           |MPHP_DATA_IND(TMSI)                         |

|                                                           *<===========================================*

|              |GMMRR_CS_PAGE_IND                           |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ (unassign TLLI)

|              *============================>*

|              |GMMRR_CS_PAGE_RES (suspend)                 |

|              *===========================================>*

|              |GMMRR_SUSPEND_CNF                           |

|              *<===========================================*

|GMMREG_SUSPEND_IND

*<=============*

|                                                           |RRGRR_ACTIVATE_REQ

|                                                           *=============>*

|                                                           |RRGRR_RR_EST_REQ

|                                                           *=============>*

|                             | RR_ESTABLISH_IND                           |

|                             *<===========================================*

|                             | RR_DATA_IND (CM SERVICE ACCEPT)            |

|                             *<===========================================*

|                             | RR_SYNC_IND (ciphering on)                 |

|                             *<===========================================*

|MMSMS_ESTABLISH_CNF          |

|MMSS_ESTABLISH_CNF           |

|MMCC_ESTABLISH_CNF           |

*<============================*

|MMSMS_RELEASE_REQ            |

|MMSS_RELEASE_REQ             |

|MMCC_RELEASE_REQ             | RR_RELEASE_IND (GPRS RESUMPTION OK)        |

*============================>*<===========================================*

|MMSMS_RELEASE_IND            |

|MMSS_RELEASE_IND             |

|MMCC_RELEASE_IND             |

*<============================*

|                             | MDL_RELEASE_REQ                                                          |

|                             *=========================================================================>*

|              |MMGMM_CM_RELEASE_IND (GPRS Resumption OK)

|              *<=============*

|        	      |GMMRR_RESUME_REQ                            |

|              *===========================================>*

|                                                           |RRGRR_SI13_IND (PCCCH)

|                                                           *<=============*

|                                                           |RRGRR_STOP_MON_BCCH_REQ

|                                                           *=============>*

|                                                           |RRGRR_STOP_MON_CCCH_REQ

|                                                           *=============>*

|                                                           |MPHP_DATA_IND (PSI1)                        |

|                                                           *<===========================================*

|                                                           |MPHP_DATA_IND (PSI2)                        |

|                                                           *<===========================================*

|        	      |GMMRR_CELL_IND ( net mode )                 |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ ( assign old TLLI again)

|              *============================>*

|GMMREG_RESUME_IND (call ended)

*<=============*

|              |              |              |              |              |              |              |

History:

12-Apr-00	ANS	Initial

### MGM401: Mobile terminated call indicated - BG

Description:

MS class BG.

net mode II.

Combined attached.

RR receives a paging request for CS. Before continuing RR waits for the paging response. GMM suspends the GPRS side and confirms the paging request. RR follows the normal CS paging procedure.

Preamble:

MGM015

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                                                                          |MPH_PAGING_IND (CS paging)   |

|                                                                          *<============================*

|                                                           |RRGRR_RR_EST_IND

|                                                           *<=============*

|              |GMMRR_CS_PAGE_IND                           |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ (unassign TLLI)

|              *============================>*

|              |GMMRR_CS_PAGE_RES (suspend)                 |

|              *===========================================>*

|              |GMMRR_SUSPEND_CNF                           |

|              *<===========================================*

|GMMREG_SUSPEND_IND

*<=============*

|                                                           |RRGRR_RR_EST_RSP

|                                                           *=============>*

|                                                                          |MPH RANDOM ACCESS REQ        |

|                                                                          *============================>*

|                                                                          |MPH RANDOM ACCESS CNF        |

|                                                                          *<============================*

|                                                                          |MPH UNITDATA IND (IMMED.ASS.)|

|                                                                          *<============================*

|                                                                          |MPH DEDICATED REQ            |

|                                                                          *============================>*

|                                                                          |MPH DEDICATED CNF            |

|                                                                          *<============================*

|                                                                          |DL ESTABLISH REQ             |

|                                                                          *============================>*

|                                                                          |DL ESTABLISH CNF             |

|                                                                          *<============================*

|                             | RR ESTABLISH IND                           |

|                             *<===========================================*

|                             | RR DATA IND (CM SERVICE ACCEPT)            |

|                             *<===========================================*

|                             | RR SYNC IND (ciphering on)                 |

|                             *<===========================================*

|MMSMS_ESTABLISH_CNF          |

|MMSS_ESTABLISH_CNF           |

|MMCC_ESTABLISH_CNF           |

*<============================*

|MMSMS_RELEASE_REQ            |

|MMSS_RELEASE_REQ             |

|MMCC_RELEASE_REQ             | RR_RELEASE_IND (GPRS RESUMPTION OK)        |

*============================>*<===========================================*

|MMSMS_RELEASE_IND            |

|MMSS_RELEASE_IND             |

|MMCC_RELEASE_IND             |

*<============================*

|                             | MDL_RELEASE_REQ                                                          |

|                             *=========================================================================>*

|              |MMGMM_CM_RELEASE_IND (GPRS Resumption OK)

|              *<=============*

|        	      |GMMRR_RESUME_REQ                            |

|              *===========================================>*

|                                                           |RRGRR_SI13_IND (CCCH)

|                                                           *<=============*

|        	      |GMMRR_CELL_IND ( net mode )                 |

|              *<===========================================*

|              |LLGMM_ASSIGN_REQ ( assign old TLLI again)

|              *============================>*

|GMMREG_RESUME_IND (call ended)

*<=============*

|              |              |              |              |              |              |              |

History:

12-Apr-00	ANS	Initial

### MGM402: Mobile terminated call indicated - BC

Description:

MS class BC.

net mode III.

IMSI attached.

RR receives a paging request for CS. Before continuing RR waits for the paging response. If GMM decides to react on the paging request RR should follow the normal CS paging procedure.

Preamble:

MGM203

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|                                                                          |MPH_PAGING_IND (CS paging)    |

|                                                                          *<============================*

|                                                           |RRGRR_RR_EST_IND

|                                                           *<=============*

|              |GMMRR_CS_PAGE_IND                           |

|              *<===========================================*

|              |GMMRR_CS_PAGE_RES                           |

|              *===========================================>*

|                                                           |RRGRR_RR_EST_RSP

|                                                           *=============>*

|                                                                          |MPH RANDOM ACCESS REQ        |

|                                                                          *============================>*

|                                                                          |MPH RANDOM ACCESS CNF        |

|                                                                          *<============================*

|                                                                          |MPH UNITDATA IND (IMMED.ASS.)|

|                                                                          *<============================*

|                                                                          |MPH DEDICATED REQ            |

|                                                                          *============================>*

|                                                                          |MPH DEDICATED CNF            |

|                                                                          *<============================*

|                                                                          |DL ESTABLISH REQ             |

|                                                                          *============================>*

|                                                                          |DL ESTABLISH CNF             |

|                                                                          *<============================*

|                             |RR_ESTABLISH_IND                            |

|                             *<===========================================*

|                             | RR DATA IND (CM SERVICE ACCEPT)            |

|                             *<===========================================*

|                             | RR SYNC IND (ciphering on)                 |

|                             *<===========================================*

|MMSMS_ESTABLISH_CNF          |

|MMSS_ESTABLISH_CNF           |

|MMCC_ESTABLISH_CNF           |

*<============================*

|MMSMS_RELEASE_REQ            |

|MMSS_RELEASE_REQ             |

|MMCC_RELEASE_REQ             | RR_RELEASE_IND                             |

*============================>*<===========================================*

|MMSMS_RELEASE_IND            |

|MMSS_RELEASE_IND             |

|MMCC_RELEASE_IND             |

*<============================*

|                             | MDL_RELEASE_REQ                                                          |

|                             *=========================================================================>*

|              |              |              |              |              |              |              |

History:

12-Apr-00	ANS	Initial

### MGM501: Cell Selection

Description:

MS class B.

net mode II.

IMSI attached.

The MS does a cell selection and finds a GPRS cell.

Preamble:

MGM

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|             |              |              |              |               |              |              |

|                                                                          | MPH_POWER_REQ               |

|                                                                          *============================>*

|                                                                          | MPH_POWER_CNF               |

|                                                                          *<============================*

|                                                                          | MPH_BSIC_REQ                |

|                                                                          *============================>*

|                                                                          | MPH_BSIC_CNF                |

|                                                                          *<============================*

|                                                                          | MPH_UNITDATA_IND(SYS_INFO_3)|

|                                                                          *<============================*

|                                                          | RRGRR_GPRS_IND|

|                                                          *<==============*

|                                                                          | MPH_MON_CTRL_REQ(EBCCH,SI13)|

|                                                                          *============================>*

|                                                                          | MPH_UNITDATA_IND(SYS_INFO_1)|

|                                                                          *<============================*

|                                                                          | MPH_UNITDATA_IND(SYS_INFO_4)|

|                                                                          *<============================*

|                                                                          | MPH_UNITDATA_IND(SYS_INFO_13)|

|                                                                          *<============================*

|                                                          | RRGRR_SI13_IND|

|                                                          *<==============*

|                                                          | RRGRR_START_MON_BCCH_REQ(COMPLETE_SI)

|                                                          *=============>*

|                                                          | RRGRR_SI_STATE_IND

|                                                          *<==============*

|                                                                          | MPH_CLASSMARK_REQ            |

|                                                                          | MPH_IDLE_REQ            |

## Packet Access using CCCH

### MGM403: Mobile originated packet access - B

Description:

MS class B.

net mode II.

IMSI attached.

The MS successfully allocates a PDCH to start TBF.

Preamble:

MGM

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|             |              |              |              |               |              |              |

|                                           	| GRR_DATA_REQ |

|                                           *=============>*

|                                                          | RRGRR_CHANNEL_REQ

|                                                          *==============>*

|                                                                        (RR 1)

|                                                                          |MPH_RANDOM_ACCESS_REQ        |

|                                                                          *============================>*

|                                                                          |MPH_RANDOM_ACCESS_CNF        |

|                                                                          *<============================*

|                                                                          |MPH_RANDOM_ACCESS_CNF        |

|                                                                          *<============================*

|                                                                        (RR 2)

|                                                                          |MPH_UNITDATA_IND (IMMED.ASS.)|

|                                                                          *<============================*

|                                                                        (RR 3)

|                                                           | RRGRR_IA_IND

|                                                           *<=============*

|                                                           | RRGRR_STOP_CHANNEL_REQ

|                                                           *=======================>*

|                                                                        (RR 4)

|                                                                          | MPH_MON_CTRL_REQ(STOP CCCH) |

|                                                                          *============================>*

|                                                                      <* enter GPRS_DEDI *>

a)

|                                                           | RRGRR_START_CCCH_REQ

|                                                           *=====================>*

|                                                           | RRGRR_START_BCCH_REQ

|                                                           *=====================>*

(RR 5)

b)

|                                                           | RRGRR_CELL_RESELECTION_REQ

|                                                           *=====================>*

|                                                                        (RR 5)

|                                                           |              |                             |

(RR 1)

RR receives the channel request from GRR. RR starts the access procedure on the CCCH. RR doesnot need to check if there is a call active(GMM/GRR know this) or if the access is allowed(also checked by GRR). The wait indication(T3122) needs not to be checked for packet access and T3142 is checked by GRR.

(RR 2)

T3146 is started after sending all random bursts. T3146 uses T3126(the GSM equivalent) as they are the same.

(RR 3)

On receipt of an Immediate Assignment message RR checks if the Message is either a TBF or dedicated mode assignment (If the Immediate Assignment message was part of a TMA(two message assignment) is by from RR, as PL handles the TMA and sends only the relevant part to RR). RR checks if the IA is valid by comparing the random access reference. For a valid TBF assignment RR informs GRR via the RRGRR_IA_IND(there is also only 1 RRGRR_IA_IND for a two message assignment with all the relevant parts merged). For a dedicated mode assignment RR sends a RRGRR_IA_IND if GRR requested a channel else the normal RR procedures are used. T3146 is stopped on receipt of a valid IA.

(RR 4)

RR waits either for a RRGRR_STOP_CHANNEL_REQ in the case of a PDCH assignment or for a RRGRR_DATA_REQ in the case of a dedicated mode assignment. On receipt of a RRGRR_STOP_CHANNEL_REQ RR stops CCCH listening in PL and enters the GPRS_DEDI state waiting to resume normal operation(ie cell reselection after leaving the PDCH). On receipt of a RRGRR_DATA_REQ RR uses this message as SABM message and enters normal GSM dedicated mode. XXX This has to checkedif this really works(ie. If the data req takes too long time, there is a network timer waiting for the SABM, alsothe RR procedure has to be totally reworked. It would be better if we already get the DATA_REQ before receivingthe IA, if possible).

(RR 5)

Entering idle mode. XXX is still open for discussion.

History:

12-Apr-00	MPA	Initial

22-Jul-00	MPA	revised

### MGM404: Mobile originated packet access - B

Description:

MS class B.

net mode II.

IMSI attached.

The MS successfully allocates a SDCCH. For comments see MSC above.

Preamble:

MGM

MMI/CM           GMM             MM            LLC            GRR            RR             SIM      DL    PL

|             |              |              |              |               |              |         |     |

|                                           	| GRR_DATA_REQ |

|                                           *=============>*

|                                                          | RRGRR_CHANNEL_REQ

|                                                          *==============>*

|                                                                        (RR 1)

|                                                                          |MPH_RANDOM_ACCESS_REQ        |

|                                                                          *============================>*

|                                                                          |MPH_RANDOM_ACCESS_CNF        |

|                                                                          *<============================*

|                                                                          |MPH_RANDOM_ACCESS_CNF        |

|                                                                          *<============================*

|                                                                        (RR 2)

|                                                                          |MPH_UNITDATA_IND (IMMED.ASS.)|

|                                                                          *<============================*

|                                                                        (RR 3)

|                                                           | RRGRR_IA_IND |

|                                                           *<=============*

|                                                           | RRGRR_DATA_REQ

|                                                           *=============>*

|                                                                        (RR 4)

|                                                                          |MPH_DEDICATED_REQ            |

|                                                                          *============================>*

|                                                                          |MPH_DEDICATED_CNF            |

|                                                                          *<============================*

|                                                                          | DL_ESTABLISH_REQ    |

|                                                                          *====================>*

|                                                                          | DL_DATA_IND         |

|                                                                          *<====================*

|                                                           | RRGRR_DATA_IND

|                                                           *<=============*

|                                                           |              |                             |

History:

12-Apr-00	MPA	Initial

### MGM405: Mobile originated packet access - B

Description:

MS class B.

net mode II.

IMSI attached.

The MS tries to allocate a PDCH but receives a Immediate Assignment Reject.

Preamble:

MGM

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|             |              |              |              |              |              |              |

|                                           	| GRR_DATA_REQ |

|                                           *=============>*

|                                                          | RRGRR_CHANNEL_REQ

|                                                          *==============>*

|                                                                          |MPH_RANDOM_ACCESS_REQ        |

|                                                                          *============================>*

|                                                                          |MPH_RANDOM_ACCESS_CNF        |

|                                                                          *<============================*

|                                                                        (RR 1)

|                                                                          |MPH_UNITDATA_IND (IMMED.ASS. REJ)|

|                                                                          *<============================*

|                                                                        (RR 2)

|                                                          | RRGRR_ASSIGNMENT_REJ_IND

|                                                          *<==============*

|                                                        (GRR 1)

|                                                                      <* expire T3146 *>                |

|                                                                        (RR 3)

|                                                          | RRGRR_STOP_CHANNEL_IND

|                                                          *<==============*

|                                                                          |MPH_IDLE_REQ(CELL_RESEL)     |

|                                                                          *============================>*

|                                                        (GRR 2)

|                                                          | RRGRR_CHANNEL_REQ

|                                                          *==============>*

|                                                          |               |                             |

(RR 1)

RR starts timer T3146 after sending the last random burst.

(RR 2)

RR receives a Immediate Assignment Reject message. RR checks if the IA is valid by comparing the random access reference and informs GRR via a RRGRR_ASSIGNMENT_REJ_IND.

(GRR 1)

GRR starts the wait indication timer(T3142).

(RR 3)

If timer T3146 expires RR aborts the Packet Access procedure. RR informs GRR via the RRGRR_STOP_CHANNEL_IND and starts a cell reselection to enter idle mode.

(GRR 2)

After T3143 expires GRR can retry the packet access procedure. If there is a GRR_RR_EST_REQ from RR while T3142 is running, the timer can be ignored as it only should prevent packet access.

History:

12-Apr-00	MPA	Initial

23-Jul-00	MPA	revised

### MGM404: Mobile originated packet access - B

Description:

MS class B.

net mode II.

IMSI attached.

The MS tries to allocate a PDCH but no valid IA is received before T3146 expires.

Preamble:

MGM

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|             |              |              |              |              |              |              |

|                            	               | GRR_DATA_REQ |

|                                           *=============>*

|                                                          | RRGRR_CHANNEL_REQ

|                                                          *==============>*

|                                                                          |MPH RANDOM ACCESS REQ        |

|                                                                          *============================>*

|                                                                          |MPH RANDOM ACCESS CNF        |

|                                                                          *<============================*

|                                                                          |MPH RANDOM ACCESS CNF        |

|                                                                          *<============================*

|                                                                        (RR 1)

a)

|                                                                          |MPH UNITDATA IND (IMMED.ASS.)|

|                                                                          | (wrong request reference)   |

|                                                                          *<============================*

|                                                                        (RR 2)

b)

|                                                                          | <* or no IA received *>     |

|                                                                        (RR 3)

c)

|                                                                          | (2nd part of two message IA  |

|                                                                          |  not received)              |

|                                                                        (RR 4)

|                                                                      <* expire T3146 *>                |

|                                                                        (RR 5)

|                                                           | RRGRR_STOP_CHANNEL_IND

|                                                           *<=======================*

|                                                                          |MPH_IDLE_REQ(CELL_RESEL)     |

|                                                                          *============================>*

|                                           | GRR_STATUS_IND|

|                                           *<==============*

|                                           |               |              |                             |

(RR 1)

RR starts timer T3146 after sending the last random burst.

(RR 2)

RR receives Immediate Assignment message with a wrong request reference. This Immediate Assignment is ignored.

(RR 3)

RR receives no Immediate Assignment.

(RR 4)

If RR receives no Immediate Assignment message it could also be that only 2nd part of a two message Immediate Assignment was not received because PL handles the correct reception of a TMA and sends only the relevant parts to RR.

(RR 5)

If the Packet Access Timer T3146 expires GRR is informed via the RRGRR_STOP_CHANNEL_IND and RR starts a cell reselection.

History:

12-Apr-00	MPA	Initial

## Packet Paging using CCCH

### MGM407: Mobile terminated packet access – B??

Description: (under construction/DG 25.7.2000)

MS class B

net mode II.

IMSI attached.

The MS receives packet paging on CCCH. RR informs GRR.

Preamble:

MGM

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|             |              |              |              |              |              |                |

|                                                                         |MPH PAGING IND (packet paging) |

|                                                                         *<==============================*

|                                                          | RRGRR_PACKET_PAGING_IND

|                                                          *<=============*

## Cell Response Procedure

- GMM has to response to GMMRR_CELL_IND finally with GMMRR_CELL_RES.

- GMM may receive more than one GMMRR_CELL_IND before responding with GMMRR_CELL_RES.

- If LLC is suspended with LLGMM_RAU LLC should at least the first time send GRR_SUSPEND_REQ to GRR to ask GRR for GRR_SUSPEND_READY_IND.

- Upon receiving GMMRR_CELL_RES GRR should send GMMRR_READY_IND instead of GMMRR_SUSPEND_READY_IND.

- Upon receiving LLGMM_RESUME_REQ LLC should listen to GMMRR_READY_IND instead of GMMRR_SUSPEND_READY_IND.

- If LLC is in supended state it should expect LLGMM_SUSPEND_REQ if cause is changed, i.e. if the cause changed to LLGMM_RAU.

- LLC should not send GRR_SUSPEND_REQ to GRR on causes different from LLGMM_RAU.

### MGM800: Normal Cell Change

Description:

MS class B.

For normal cell changes LLC is not suspended. GMM responses immediately with GMMRR_CELL_RES.

Preamble:

none

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|              |   GMMRR_CELL_IND                           |

|              |  (	RAI not changed)                         |

|              |  (i.e. Cell Changed or Resumtion)          |

|              *<===========================================*

(GMM 1)

|              |   GMMRR_CELL_RES                           |

|              *===========================================>*

(GRR 1)

|              |              |              |              |              |              |              |

(GMM 1)

GRR sends cell ind either because cell has changed or just only as an answer to GMMRR_RESUME_REQ.

(GMM 1)

No Suspension is needed.

History:

6-Mar-02	ANS	Initial

### MGM801: RA changed

Description:

MS class B.

If RA is changed LLC is suspended with LLGMM_RAU. LLC is forwarding suspension request to GRR to ask for GMMRR_SUSPEND_READY_IND. When RAU is finished GRR is informed via GMMR_CELL_RES and LLC is resumed.

Preamble:

none

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

|              |   GMMRR_CELL_IND                           |

|              |  (	RAI changed)                             |

|              *<===========================================*

(GMM 1)

|              |   LLGMM_SUSPEND_REQ         |

|              |      (	LLGMM_RAU)            |

|              *============================>*

(LLC 1)

|                                            | GRR_SUSPEND_REQ |

|                                            *=============>*

(GRR 1)

|                                            | GRR_SUSPEND_READY_IND |

|                                            *<============*

(GRR 2)

|              |   LLGMM_ASSIGN_REQ          |

|              |    (foreign TLLI)           |

|              *============================>*

(LLC 2)

|              |   GMMRR_ASSIGN_REQ                         |

|              |   (	foreign TLLI)                           |

|              *===========================================>*

(GRR 3)

|              |   GMMRR_ATTACH_STARTED_REQ                 |

|              *===========================================>*

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU REQUEST)            |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_SUSPEND_READY_IND |

|                                            *<============*

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU REQUEST)            |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|                                            | GRR_SUSPEND_READY_IND |

|                                            *<============*

|                                            | GRR_DATA_IND |

|                                            *<=============*

|              |   LL_UNIT_DATA_IND          |

|              |    (RAU REJ)                |

|              *============================>*

|              |   GMMRR_ATTACH_FINISHED_REQ                |

|              *===========================================>*

|              |   GMMRR_ATTACH_STARTED_REQ                 |

|              *===========================================>*

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU REQUEST)            |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_SUSPEND_READY_IND |

|                                            *<============*

|                                            | GRR_DATA_IND |

|                                            *<=============*

|              |   LL_UNIT_DATA_IND          |

|              |    (RAU ACCEPT)             |

|              *============================>*

|              |   GMMRR_ATTACH_FINISHED_REQ                |

|              *===========================================>*

|              |   LLGMM_ASSIGN_REQ          |

|              |  (new local TLLI)           |

|              *============================>*

(LLC 2)

|              |   GMMRR_ASSIGN_REQ                         |

|              |   (new local TLLI)                         |

|              *===========================================>*

(GRR 3)

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU COMPLETE)           |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_SUSPEND_READY_IND |

|                                            *<============*

|              |   GMMRR_CELL_RES                           |

|              *===========================================>*

|              |   LLGMM_RESUME__REQ          |

|              *============================>*

|                                            | GRR_READY_IND |

|                                            *<============*

|              |              |              |              |              |              |              |

(GMM 1)

GRR sends cell ind and the RAU is crossed.

(LLC 1)

LLC is suspended. LLC has to inform GRR.

(GRR 1/2)

GRR switches to GMM queue and sends GRR_SUSPEND_READY to LLC.

(LLC 2/GRR3)

Foreign TLLI is assigned to the mobile

History:

6-Mar-02	ANS	Initial

### MGM802: RAU w/o Cell Change

Description:

MS class B.

Routing area update should be done, but no cell is changed. LLC is suspended with different cause than LLGMM_RAU. LLC must not suspend GRR. If RAU is finished LLC is resumed by GMM.

Preamble:

none

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

* RAU requested (due to timeout or different reasons)

(GMM 1)

|              |   LLGMM_SUSPEND_REQ         |

|              |  (	LLGMM_PERIODIC_RAU)       |

|              *============================>*

(LLC 1)

|              |   GMMRR_ATTACH_STARTED_REQ                 |

|              *===========================================>*

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU REQUEST)            |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_READY_IND |

|                                            *<============*

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU REQUEST)            |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|                                            | GRR_READY_IND |

|                                            *<============*

|                                            | GRR_DATA_IND |

|                                            *<=============*

|              |   LL_UNIT_DATA_IND          |

|              |    (RAU ACCEPT)             |

|              *============================>*

|              |   GMMRR_ATTACH_FINISHED_REQ                |

|              *===========================================>*

|              |   LLGMM_ASSIGN_REQ          |

|              |  (new local TLLI)           |

|              *============================>*

(LLC 2)

|              |   GMMRR_ASSIGN_REQ                         |

|              |   (new local TLLI)                         |

|              *===========================================>*

(GRR 3)

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU COMPLETE)           |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_READY_IND |

|                                            *<============*

|              |   LLGMM_RESUME__REQ          |

|              *============================>*

|              |              |              |              |              |              |              |

History:

6-Mar-02	ANS	Initial

### MGM803: Periodic RAU  is interupted by RAU change

Description:

MS class B.

Routing area update should be done, but no cell is changed. LLC is suspended with different cause than LLGMM_RAU. LLC must not suspend GRR. If RAU is finished LLC is resumed by GMM. RAU procedure is interupted by RA change. GMM requests LLC to suspend GRR via LLGMM_SUSPEND_REQ (LLGMM_RAU). RAU is reinitialized and restarted. When RAU is finished GRR is informed via GMMR_CELL_RES and LLC is resumed.

Preamble:

none

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

* RAU requested (due to timeout or different reasons)

(GMM 1)

|              |   LLGMM_SUSPEND_REQ         |

|              |  (	LLGMM_PERIODIC_RAU)       |

|              *============================>*

(LLC 1)

|              |   GMMRR_ATTACH_STARTED_REQ                 |

|              *===========================================>*

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU REQUEST)            |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_READY_IND |

|                                            *<============*

|              |   GMMRR_CELL_IND                           |

|              |    (	RAI changed)                           |

|              *<===========================================*

(GMM 1)

|              |   LLGMM_SUSPEND_REQ         |

|              |      (	LLGMM_RAU)            |

|              *============================>*

(LLC 1)

|                                            | GRR_SUSPEND_REQ |

|                                            *=============>*

(GRR 1)

|                                            | GRR_SUSPEND_READY_IND |

|                                            *<============*

(GRR 2)

|              |   LLGMM_ASSIGN_REQ          |

|              |    (foreign TLLI)           |

|              *============================>*

(LLC 2)

|              |   GMMRR_ASSIGN_REQ                         |

|              |   (	foreign TLLI)                           |

|              *===========================================>*

(GRR 3)

|              |   GMMRR_ATTACH_STARTED_REQ                 |

|              *===========================================>*

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU REQUEST)            |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_SUSPEND_READY_IND |

|                                            *<============*

|                                            | GRR_DATA_IND |

|                                            *<=============*

|              |   LL_UNIT_DATA_IND          |

|              |    (RAU ACCEPT)             |

|              *============================>*

|              |   GMMRR_ATTACH_FINISHED_REQ                |

|              *===========================================>*

|              |   LLGMM_ASSIGN_REQ          |

|              |  (new local TLLI)           |

|              *============================>*

(LLC 2)

|              |   GMMRR_ASSIGN_REQ                         |

|              |   (new local TLLI)                         |

|              *===========================================>*

(GRR 3)

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU COMPLETE)           |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_SUSPEND_READY_IND |

|                                            *<============*

|              |   GMMRR_CELL_RES                           |

|              *===========================================>*

|              |   LLGMM_RESUME__REQ          |

|              *============================>*

History:

6-Mar-02	ANS	Initial

### MGM804: Periodic RAU  is interupted by LAU change

Description:

MS class B.

Routing area update should be done, but no cell is changed. LLC is suspended with different cause than LLGMM_RAU. LLC must not suspend GRR. If RAU is finished LLC is resumed by GMM. RAU procedure is interupted by LA change. GMM requests LLC to suspend GRR via LLGMM_SUSPEND_REQ (LLGMM_RAU). RAU is reinitialized and restarted. When RAU is finished GRR is informed via GMMR_CELL_RES and LLC is resumed.

Preamble:

none

MMI/CM           GMM             MM            LLC            GRR            RR             SIM             PL

|              |              |              |              |              |              |              |

* RAU requested (due to timeout or different reasons)

(GMM 1)

|              |   LLGMM_SUSPEND_REQ         |

|              |  (	LLGMM_PERIODIC_RAU)       |

|              *============================>*

(LLC 1)

|              |   GMMRR_ATTACH_STARTED_REQ                 |

|              *===========================================>*

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU REQUEST)            |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_READY_IND |

|                                            *<============*

|              |   GMMRR_CELL_IND                           |

|              |    (	LAI changed)                           |

|              *<===========================================*

|              |   LLGMM_SUSPEND_REQ         |

|              |     (LLGMM_CALL)            |

|              *============================>*

|        	      |   GMMRR_SUSPEND_REQ

|              |  (deactivate while LAU )                   |

|              *===========================================>*

|        	      |   GMMRR_SUSPEND_CNF                        |

|              *<===========================================*

|              |MMGMM_REG_REQ

|              |(perform LAU)

|              *=============>*

|              *< LAU >

|              |MMGMM_REG_CNF

|              *<=============*

|        	      |   GMMRR_RESUME_REQ                         |

|              *===========================================>*

|        	      |   GMMRR_CELL_IND                           |

|              *<===========================================*

|              |   LLGMM_SUSPEND_REQ         |

|              |      (	LLGMM_RAU)            |

|              *============================>*

(LLC 1)

|                                            | GRR_SUSPEND_REQ |

|                                            *=============>*

(GRR 1)

|                                            | GRR_SUSPEND_READY_IND |

|                                            *<============*

(GRR 2)

|              |   LLGMM_ASSIGN_REQ          |

|              |    (foreign TLLI)           |

|              *============================>*

(LLC 2)

|              |   GMMRR_ASSIGN_REQ                         |

|              |   (	foreign TLLI)                           |

|              *===========================================>*

(GRR 3)

|              |   GMMRR_ATTACH_STARTED_REQ                 |

|              *===========================================>*

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU REQUEST)            |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_SUSPEND_READY_IND |

|                                            *<============*

|                                            | GRR_DATA_IND |

|                                            *<=============*

|              |   LL_UNIT_DATA_IND          |

|              |    (RAU ACCEPT)             |

|              *============================>*

|              |   GMMRR_ATTACH_FINISHED_REQ                |

|              *===========================================>*

|              |   LLGMM_ASSIGN_REQ          |

|              |  (new local TLLI)           |

|              *============================>*

(LLC 2)

|              |   GMMRR_ASSIGN_REQ                         |

|              |   (new local TLLI)                         |

|              *===========================================>*

(GRR 3)

|              |   LL_UNIT_DATA_REQ          |

|              |    (RAU COMPLETE)           |

|              *============================>*

|                                            | GRR_DATA_REQ |

|                                            *=============>*

|              |   LLGMM_TRIGGER_IND         |

|              *<============================*

|                                            | GRR_SUSPEND_READY_IND |

|                                            *<============*

|              |   GMMRR_CELL_RES                           |

|              *===========================================>*

|              |   LLGMM_RESUME__REQ          |

|              *============================>*

History:

6-Mar-02	ANS	Initial

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |