Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

GSM Data Services

Message Sequence Charts

RA1´

(Edit via File/Properties/Summary!)

| Document Number: | 6148.203.97.100 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.2 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1997-Mar-12 |
| Last changed: | 2003-May-26 by XGUTTEFE |
| File Name: | ra1.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1997-Mar-12 | LE |  | 0.1 |  | 1 |
| 2003-May-26 | XGUTTEFE |  | 0.2 | Draft |  |

Notes:

- Initial version

Table of Contents

1	General	4

1.1	References	4

1.2	Abbreviations	8

1.3	Terms	9

2	Overview	10

3	Introduction	12

3.1	Frame Formats	12

3.2	Extension Bits	13

3.3	Status Bits	15

4	Common Procedures	16

4.1	Activation / Deactivation	16

4.2	Setting of Baudrate	16

4.3	Status Change	17

5	Transmission of data	18

6	Reception of data	19

Appendices	20

A.	Acronyms	20

B.	Glossary	20

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

# General

## References

[1]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat GmbH, December 1996

[39]	Service Access Point MNCC
	6147.101.96.100; Condat GmbH, December 1996

[40]	Service Access Point MNSS
	6147.102.96.100; Condat GmbH, December 1996

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat GmbH, December 1996

[42]	Service Access Point MMCC
	6147.104.97.100; Condat GmbH

[43]	Service Access Point MMSS
	6147.105.97.100; Condat GmbH

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat GmbH

[45]	Service Access Point RR
	6147.107.97.100; Condat GmbH

[46]	Service Access Point SIM
	6147.108.97.100; Condat GmbH

[47]	Service Access Point MPH
	6147.109.96.100; Condat GmbH, December 1996

[48]	Service Access Point DL
	6147.110.96.100; Condat GmbH, December 1996

[49]	Service Access Point MDL
	6147.111.96.100; Condat GmbH, December 1996

[50]	Service Access Point PH
	6147.112.96.100; Condat GmbH, December 1996

[51]	Service Access Point MMI
	6147.113.96.100; Condat GmbH, December 1996

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat GmbH

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat GmbH

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat GmbH

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat GmbH

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat GmbH, December 1996

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat GmbH, December 1996

[58]	Users Guide
	6147.300.96.100; Condat GmbH, December 1996

[59]	Test Specification CC
	6147.400.97.100; Condat GmbH

[60]	Test Specification SS
	6147.401.97.100; Condat GmbH

[61]	Test Specification SMS
	6147.402.97.100; Condat GmbH

[62]	Test Specification MM
	6147.403.97.100; Condat GmbH

[63]	Test Specification RR
	6147.404.97.100; Condat GmbH

[64]	Test Specification DL
	6147.405.97.100; Condat GmbH

[65]	Test Specification CCD
	6147.406.97.100; Condat GmbH

[66]	SDL Specification CC
	6147.500.97.100; Condat GmbH

[67]	SDL Specification SS
	6147.501.97.100; Condat GmbH

[68]	SDL Specification SMS
	6147.502.97.100; Condat GmbH

[69]	SDL Specification MM
	6147.503.97.100; Condat GmbH

[70]	SDL Specification RR
	6147.504.97.100; Condat GmbH

[71]	SDL Specification DL
	6147.505.97.100; Condat GmbH

[72]	Message Specification CC
	6147.600.97.100; Condat GmbH

[73]	Message Specification SS
	6147.601.97.100; Condat GmbH

[74]	Message Specification SMS
	6147.602.97.100; Condat GmbH

[75]	Message Specification MM
	6147.603.97.100; Condat GmbH

[76]	Message Specification RR
	6147.604.97.100; Condat GmbH

[77]	Message Specification DL
	6147.605.97.100; Condat GmbH

[78]	Technical Documentation CC
	6147.700.97.100; Condat GmbH

[79]	Technical Documentation SS
	6147.701.97.100; Condat GmbH

[80]	Technical Documentation SMS
	6147.702.97.100; Condat GmbH

[81]	Technical Documentation MM
	6147.703.97.100; Condat GmbH

[82]	Technical Documentation RR
	6147.704.97.100; Condat GmbH

[83]	Technical Documentation DL
	6147.705.97.100; Condat GmbH

[84]	Technical Documentation CCD
	6147.706.97.100; Condat GmbH

## Abbreviations

AGCH	Access Grant Channel

## Terms

Entity	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and

Figure 1: Data Services protocol architecture

This document describes the services offered by the radio link protocol.

# Introduction

## Frame Formats

The channel codec requests in regular times data from the rate adaption function RA1´.

- For 9600 bit/s 60 bit CCITT V.110 frames are send to the channel codec.

- For 4800 bit/s 60 bit CCITT V.110 frames are send to the channel codec.

- For 2400 bit/s 36 bit CCITT V.110 frames are send to the channel codec.

The number of blocks send to channel codec depends on the time interval and the baudrate:

|  |  | Time Interval |  |  |
| --- | --- | --- | --- | --- |
| Baudrate | 5 ms | 10 ms | 20 ms | Type of Frame |
| 9600 bit/s | 1 | 2 | 4 | 60 bit V.110 |
| 4800 bit/s | - | 1 | 2 | 60 bit V.110 |
| 2400 bit/s | - | 1 | 2 | 36 bit V.110 |

If the channel codec requests all 10 ms data at 9600 bit/s the rate adaption function RA1´ sends 100 times per second two 60 bit V.110 frames. This is an air interface rate of 12000 bit/s. The data inside one 60 bit V.110 frame is 48 bits. So 100 * 2 * 48 data bits are send per second. This is equal to the baudrate of 9600 bits/s.

Using longer intervals reduces communication between rate adaption function RA1´ and channel codec but prolongs the reaction time for in-band flow control (status bits).

If the number of stored bits for sending to channel codec is lower than a threshold the rate adaption function RA1´ requests new data from it´s source (RLP, RA0 or fax adapter).

The following frame formats are defined:

9600 bit/s and 4800 bit/s (60 bit CCITT V.110 frames)

| Offset | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | D7 | S1 | D6 | D5 | D4 | D3 | D2 | D1 |
| 1 | D14 | D13 | x | D12 | D11 | D10 | D9 | D8 |
| 2 | D21 | D20 | D19 | S3 | D18 | D17 | D16 | D15 |
| 3 | E7 | E6 | E5 | E4 | S4 | D24 | D23 | D22 |
| 4 | D31 | S6 | D30 | D29 | D28 | D27 | D26 | D25 |
| 5 | D38 | D37 | x | D36 | D35 | D34 | D33 | D32 |
| 6 | D45 | D44 | D43 | S8 | D42 | D41 | D40 | D39 |
| 7 | 0 | 0 | 0 | 0 | S9 | D48 | D47 | D46 |

2400 bit/s (blocks of 36 bit CCITT V.110 frames)

| Offset | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | x | D6 | D5 | D4 | S1 | D3 | D2 | D1 |
| 1 | S4 | D12 | D11 | D10 | S3 | D9 | D8 | D7 |
| 2 | S6 | D15 | D14 | D13 | E7 | E6 | E5 | E4 |
| 3 | S8 | D21 | D20 | D19 | x | D18 | D17 | D16 |
| 4 | 0 | 0 | 0 | 0 | S9 | D24 | D23 | D22 |

## Extension Bits

The extension bits E4, E5, E6and E7 are inserted in each V.110 frames. E4, E5 and E6 are used as clocking bits and E7 is the multiframe bit.

Synchronous data signals received by the MT from the DTE at the MS or by IWF from the modem on the PSTN may not be synchronized to the PLMN. The following method shall be used to enable transfer of those data signals and the corresponding bit timing information via the V.110 frames. Such a situation would exist where the signals received from the modem at the IWF require its own clock or where the signals received from the DTE at the MS employs ist own network independent clock.

The transmitting end of the GSM PLMN connection shall establish a multiframe structure utilising bit E7 consisting of four frames by setting E7 in every fourth frame to binary 0. This structure is identical to all user rates. The frame synchronization will be achieved and maintained during the entire call so that corrections for the network independent clocking by the receiving end of the GSM PLMN connection can be easily recognized and applied based on the code words (in c1, c2, c3, c4 and c5) positioned in bits E4, E5 and E6 of two consecutive V.110 frames as depicted in the next figure. Thus, the multiframe structure allows for one 5-bit code words to be transmitted every two V.110 frames for the purposes of network independent clocking. The two code-words may be different from each other within the multiframe.

| Frame | E4 | E5 | E6 | E7 |
| --- | --- | --- | --- | --- |
| Multiframe 0a | c1 | c2 | 1 | 0 |
| Multiframe 1a | c3 | c4 | c5 | 1 |
| Multiframe 0b | c1 | c2 | 1 | 1 |
| Multiframe 1b | c3 | c4 | c5 | 1 |

Once multiframe synchronization is achieved, each code word is independently evaluated to determine the compensation needed, if any. The compensation is carried out after reception of multiframe 1a or 1b.

The V.110 transmitter will use the following 5-bit code words, as shown in the next figure, to indicate the four possible states of compensation required for network independent clocking.

|  | c1 | c2 | c3 | c4 | c5 |
| --- | --- | --- | --- | --- | --- |
| No compensation | 1 | 1 | 1 | 1 | 1 |
| Negative compensation | 1 | 0 | 0 | 1 | 0 |
| Positive compensation of a zero | 0 | 1 | 0 | 0 | 1 |
| Positive compensation of one | 0 | 0 | 1 | 0 | 0 |

When negative compensation is indicated, one less user data bit than normal is transported in the accected frame (multiframe 1a or multiframe 1b). A negative compensation shall cause the receiver to delete the user data bit occupied by bit position D25, since the transmitter sets this to binary 1 and does not utilize this position for user data. At those user data rates where the user data bit repeated, all copies of D25 shall be discarded.

When a positive compensation is indicated, one additional user data bit is transferred by meand of the code word. At the receiver, a positive compensation will cause a user data bit of binary 0 or 1, as indicated by the code word, to be inserted between the user data bits carried in bit positions D24 and D25 (in multiframe 1a or 1b) of the V.110 frame.

When no compensation is necessary or when NIC is applied, the values of E4, E5, E6 and E7 on the four multi frame scheme is

| Frame | E4 | E5 | E6 | E7 |
| --- | --- | --- | --- | --- |
| Multiframe 0a | 1 | 1 | 1 | 0 |
| Multiframe 1a | 1 | 1 | 1 | 1 |
| Multiframe 0b | 1 | 1 | 1 | 1 |
| Multiframe 1b | 1 | 1 | 1 | 1 |

When NIC is not applicable the MS and the IWF shall disregard the received value of bits E4, E5, E6 and E7 in the data transmission phase. NIC is not applicable in the following cases:

- transparent asynchronous bearer services

- the facsimile teleservices in transparent mode

- every transparent bearer services when interworking with an UDI information transfer capability

## Status Bits

The bits S and X are used to convey channel status information associated with the data bits in the data transfer state. The S-Bits are put in two groups SA and SB to carry the condition of two interchange circuits. The X-bit is used to control the condition of circuit 106.

For the S and X bits, a binary zero corresponds to ON condition, a binary one to the OFF condition.

The status SA corresponds with S1, S3, S6 and S8. Status SB is defined by S4 and S9. The following figure shows the use of SA, SB and X and the corresponding flow control signals:

Terminal Equipment	Interworking Function	Terminal Equipment

DTR		SA		DSR

RTS		SB		DCD

CTS		X		CTS

The mobile station uses DTR to signal that is is ready for operation. The RTS signal is used to indicate that the mobile station will send data. The CTS signal is used to indicate whether it is possible to receive data or not.

The interworking function uses DSR to indicate that the other party is connected. DCD is used to indicate that data will be send to the mobile station. CTS is used by the interworking function to indicate that it is ready for receiving data.

# Common Procedures

## Activation / Deactivation

FAD/RLP/RA0                  RA1´                       CHC

|                           |                          |

|        RA1_INFO_REQ       |                          |

*==========================>*                          |

|                        (RA1 1)                       |

|                           |       CHC_INFO_REQ       |

|                           *=========================>*

|                           |                       (CHC 1)

|                           |                          |

(RA1 1)

The rate adaption function 1´ is activated or deactivated.

(CHC 1)

The information is forwarded to the channel codec.

## Setting of Baudrate

FAD/RLP/RA0                  RA1´                       CHC

|                           |                          |

|        RA1_INFO_REQ       |                          |

*==========================>*                          |

|                        (RA1 1)                       |

|                           |       CHC_INFO_REQ       |

|                           *=========================>*

|                           |                       (CHC 1)

|                           |                          |

(RA1 1)

A new speed rate is defined.

(CHC 1)

The information is forwarded to the channel codec.

## Status Change

AT-CMD                       RA1´

|                           |

|       RA1_STATUS_REQ      |

*==========================>*

|                        (RA1 1)

|                           |

|       RA1_STATUS_IND      |

*<==========================*

(AT 1)                        |

|                           |

(RA1 1)

The AT command interpreter indicates a change of status. This results in change of setting in S and X bits in the following V.110 frames.

(AT 1)

After receiving information with V.100 frames the status bits have changed. The new status is signalled to the AT command interpreter.

# Transmission of data

Transmission of data is only processed, when the rate adaption function RA1´ is activated.

FAD/RLP/RA0                  RA1´                       CHC

|                           |                          |

|                           |     CHC_READY_TO_SEND    |

|                           *<=========================*

|                        (RA1 1)                       |

|                           |                          |

a) IWF is ready and able to receive

|                           |                          |

|                           |       CHC_DATA_REQ       |

|                           *=========================>*

|                           |                       (CHC 1)

|                           |                          |

b) IWF is not ready or able to receive

|                           |                          |

|                           |       CHC_DATA_REQ       |

|                           *=========================>*

|                           |                       (CHC 2)

|                           |                          |

c) stored data is lower than threshold

|                           |                          |

|     RA1_READY_TO_SEND     |                          |

*<==========================*                          |

(S 1)                         |                          |

|       RA1_DATA_REQ        |                          |

*==========================>*                          |

|                        (RA1 2)                       |

|                           |                          |

(RA1 1)

The channel codec requests new data.

(CHC 1)

From previous received V.110 frames it is detected that the interworking function is not able to receive data. Rate adaption function RA1´ retransmits the last sended V.110 frame to the interworking function to avoid loss of synchronisation and to forward it´s own status.

(CHC 2)

From previous received V.110 frames it is detected that the interworking function is able to receive data. Rate adaption function RA1´ builds the next frame(s) and sends it to the channel coded.

(S 1)

If the remaining number of bits is lower than a configurable threshold value, the rate adaption function RA1´ requests new data from it´s source. This is the FAD, RLP or RA0.

(RA1 2)

New data is stored at rate adaption function 1´.

# Reception of data

Reception of data is only processed, when the rate adaption function RA1´ is activated.

FAD/RLP/RA0                  RA1´                       CHC

|                           |                          |

|                           |       CHC_DATA_IND       |

|                           *<=========================*

|                        (RA1 1)                       |

|                           |                          |

a) RA1´ is ready and able to receive

|                           |                          |

|       RA1_DATA_IND        |                          |

*<==========================*                          |

(S 1)                         |                          |

|                           |                          |

b) RA1´ is not ready or able to receive

|                           |                          |

|                           |       CHC_DATA_IND       |

|                           *<=========================*

|                        (RA1 2)                       |

|                           |                          |

(RA1 1)

The channel codec sends V.110 frames to the rate adaption function RA1´. The status and extension bits are extracted.

(S 1)

If the rate adaption function is able to receive data, the data is forwarded to the source. The source is the fax adapter, the radio link protocol or the rate adaption function 0.

(RA1 2)

The channel codec sends V.110 frames to the rate adaption function RA1´. The status and extension bits are extracted. If the rate adaption function is unable to receive data, the data is erased.

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |