Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM Protocol Stack

Message Sequence Charts

SIM

(Edit via File/Properties/Summary!)

| Document Number: | 6301.201.98.102 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.4 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1997-Oct-16 |
| Last changed: | 2003-Jun-16 by XINTEGRA |
| File Name: | Sim.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1997-Oct-16 | Stefan Lemke et al. |  | 0.1 |  | 1 |
| 1998-Oct-27 | Stefan Lemke et al. |  | 0.2 |  | 2 |
| 2002-Mar-14 | STW |  | 0.3 |  | 3 |
| 2003-Jun-11 | XINTEGRA |  | 0.4 | Draft |  |

Notes:

- Initial version

- SIM Toolkit, Phase 2+

- SAT class "c/e"

Table of Contents

1.1	References	5

1.2	Abbreviations	8

1.3	Terms	9

3.1	SIM Driver	11

4.1	Error Causes	13

4.2	Access Conditions	14

5.1	Read Binary File	15

5.2	Read Record File	16

5.3	Update Binary File	18

5.4	Update Record File	19

5.5	Increment	20

5.6	Verify PIN	21

5.7	Change PIN	22

5.8	Disable PIN	23

5.9	Enable PIN	24

5.10	Unblock PIN	25

5.11	Authentication	26

6.1	SIM Insertion	27

6.1.1	Access to Main Directory	27

6.1.2	Check of PIN / PUK counter	29

6.1.3	Reading of SIM Parameters	30

6.1.4	Read MM Parameter	32

6.1.5	Read MMI Parameter	33

6.1.6	Read SMS Parameter	33

6.1.7	FDN/BDN Procedures	34

6.2	No SIM Inserted	35

6.3	SIM Removing	36

6.4	SIM Updating	36

6.5	SIM Status	37

6.6	Change Operation Mode to Restricted Operation	38

6.7	Change Operation Mode to Unrestricted Operation	39

7.1	Profile Download	40

7.2	Pro-active Commands	41

7.2.1	Display Text	41

7.2.2	Get Inkey	41

7.2.3	Get Input	42

7.2.4	Play Tone	42

7.2.5	Refresh	42

7.2.6	Set Up Menu	43

7.2.7	Select Item	43

7.2.8	Send SMS	43

7.2.9	Send SS	44

7.2.10	Set Up Call	44

7.2.11	Launch Browser	44

7.2.12	Set Up Event List	46

7.2.13	More Time	46

7.2.14	Poll Intervall	46

7.2.15	Polling Off	47

7.2.16	Provide Local Information	47

7.3	Terminal Response	48

7.4	Envelope	48

7.5	SAT class e	49

7.5.1	Open Channel	49

7.5.2	Close Channel	54

7.5.3	Receive Data	56

7.5.4	Send Data	57

7.5.5	Get Channel Status	58

7.5.6	Forward of Alpha identifier and Icon identifier	58

7.5.7	Set Up Event List	59

7.5.8	Suspend and Resume channel	59

8.1	SIM Test modes	60

A.	Acronyms	63

B.	Glossary	63

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18a]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat GmbH

[39]	Service Access Point MNCC
	6147.101.96.100; Condat GmbH

[40]	Service Access Point MNSS
	6147.102.96.100; Condat GmbH

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat GmbH

[42]	Service Access Point MMCC
	6147.104.97.100; Condat GmbH

[43]	Service Access Point MMSS
	6147.105.97.100; Condat GmbH

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat GmbH

[45]	Service Access Point RR
	6147.107.97.100; Condat GmbH

[46]	Service Access Point SIM
	6147.108.97.100; Condat GmbH

[47]	Service Access Point MPH
	6147.109.96.100; Condat GmbH

[48]	Service Access Point DL
	6147.110.96.100; Condat GmbH

[49]	Service Access Point MDL
	6147.111.96.100; Condat GmbH

[50]	Service Access Point PH
	6147.112.97.100; Condat GmbH

[51]	Service Access Point MMI
	6147.113.96.100; Condat GmbH

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat GmbH

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat GmbH

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat GmbH

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat GmbH

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat GmbH

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat GmbH

[58]	Users Guide
	6147.300.96.100; Condat GmbH

[59]	Test Specification CC
	6147.400.97.100; Condat GmbH

[60]	Test Specification SS
	6147.401.97.100; Condat GmbH

[61]	Test Specification SMS
	6147.402.97.100; Condat GmbH

[62]	Test Specification MM
	6147.403.97.100; Condat GmbH

[63]	Test Specification RR
	6147.404.97.100; Condat GmbH

[64]	Test Specification DL
	6147.405.97.100; Condat GmbH

[65]	Test Specification CCD
	6147.406.97.100; Condat GmbH

[66]	SDL Specification CC
	6147.500.97.100; Condat GmbH

[67]	SDL Specification SS
	6147.501.97.100; Condat GmbH

[68]	SDL Specification SMS
	6147.502.97.100; Condat GmbH

[69]	SDL Specification MM
	6147.503.97.100; Condat GmbH

[70]	SDL Specification RR
	6147.504.97.100; Condat GmbH

[71]	SDL Specification DL
	6147.505.97.100; Condat GmbH

[72]	Message Specification CC
	6147.600.97.100; Condat GmbH

[73]	Message Specification SS
	6147.601.97.100; Condat GmbH

[74]	Message Specification SMS
	6147.602.97.100; Condat GmbH

[75]	Message Specification MM
	6147.603.97.100; Condat GmbH

[76]	Message Specification RR
	6147.604.97.100; Condat GmbH

[77]	Message Specification DL
	6147.605.97.100; Condat GmbH

[78]	Technical Documentation CC
	6147.700.97.100; Condat GmbH

[79]	Technical Documentation SS
	6147.701.97.100; Condat GmbH

[80]	Technical Documentation SMS
	6147.702.97.100; Condat GmbH

[81]	Technical Documentation MM
	6147.703.97.100; Condat GmbH

[82]	Technical Documentation RR
	6147.704.97.100; Condat GmbH

[83]	Technical Documentation DL
	6147.705.97.100; Condat GmbH

[84]	Technical Documentation CCD
	6147.706.97.100; Condat GmbH

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The SIM card used by MM, MMI and SMS in several ways. This document describes the services needed for the SIM application.

# Structure

The SIM software is divided in the following parts:

Application

(sim_app.c)

SIM Toolkit

(sim_stk.c)

PEI-
Interface
(sim_pei.c)

Wrapping Functions

(sim_fkt.c)

T=0 driver

(sim_csf.c or TI driver)

The SIM application contains the primitive interface to MMI, MM and SMS. It contains several structured services based on the basic functions of the SIM driver.

The wrapping functions in module sim_fkt.c are use to wrap the original SIM driver functions. It contains converting of the original SIM driver error codes.

The interface of the SIM functionality to the target frame is handled in the PEI interface module sim_pei.c. It contains the PEI interface, primitive distribution and timeout handling.

The SIM Driver has a functional interface and covers the functions according to GSM 11.11. This includes the T=0 protocol and the hardware control. In the target version this part of software is delivered by the customer. In the PC-environment this part is the module sim_csf.c. It contains a SIM driver simulation with predefined contents and a lot of test modes for multilayer tests and component tests of the SIM application.

Optional the SIM toolkit functionality can be added to the SIM application. It is located in the sim_stk.c module. If SIM toolkit is integrated, all other modules must be compiled with the constant SIM_TOOLKIT.

## SIM Driver

The SIM driver contains the T=0 protocol and has several function calls mapped to GSM 11.11.

SIM_Init

Initializes the SIM driver. This function must be called before any other access to the SIM.

SIM_Reset

The SIM driver reads the ATR signal and checks whether a SIM card is available.

SIM_Select

This function selects a file in the SIM. After a successful selection the record pointer in a linear fixed file is undefined. The record pointer in a cyclic file shall address the last record which has been updated or increased.

SIM_Status

This function returns information concerning the current directory. A current elementary field is not affected by the STATUS function.

##### SIM_ReadBinary

This function reads a string of bytes from the current transparent elementary field. This function shall only be performed if the READ access condition for this elementary field is satisfied.

SIM_UpdateBinary

This function updates the current transparent elementary field with a string of bytes. This function shall only be performed if the UPDATE access condition for this elementary field is satisfied. An update can be considered as a replacement of the string already present in the elementary by the string given in the update command.

SIM_ReadRecord

This function reads one complete record in the current linear fixed or cyclic elementary field. The record to be read is described by the modes below. This function shall only be performed if the READ access condition for this elementary field is satisfied. The record pointer shall not be changed by an unsuccessful READ RECORD function. Only the absolute mode is used.

SIM_UpdateRecord

This function updates one complete record in the current linear fixed or cyclic elementary field. This function shall only be performed if the UPDATE access condition for this elementary field is satisfied. The UPDATE can be considered as a replacement of the relevant record data of the elementary field by the record data given in the command. The record pointer shall not be changed by an unsuccessful UPDATE RECORD function. Only the absolute mode is used.

SIM_Increase

This function adds the value given by the mobile station to the value of the last increased/updated record of the current cyclic elementary field, and stores the result into the oldest record. The record pointer is set to this record and this record becomes record number 1. This function shall be used only if this elementary field has an INCREASE access condition assigned and this condition is fulfilled (see bytes 8 and 10 in the response parameters/data of the current elementary field). The SIM shall not perform the increase if the result would exceed the maximum value of the record (represented by all bytes set to 'FF').

SIM_VerifyCHV

This function verifies the CHV presented by the mobile station by comparing it with the relevant one stored in the SIM. The verification process is subject to the following conditions being fulfilled:

- CHV is not disabled;

- CHV is not blocked.

If the access condition for a function to be performed on the last selected file is CHV1 or CHV2, then a successful verification of the relevant CHV is required prior to the use of the function on this file unless the CHV is disabled.

If the CHV presented is correct, the number of remaining CHV attempts for that CHV shall be reset to its initial value 3.

If the CHV presented is false, the number of remaining CHV attempts for that CHV shall be decremented. After 3 consecutive false CHV presentations, not necessarily in the same card session, the respective CHV shall be blocked and the access condition can never be fulfilled until the UNBLOCK CHV function has been successfully performed on the respective CHV.

SIM_ChangeCHV

This function assigns a new value to the relevant CHV subject to the following conditions being fulfilled:

- CHV is not disabled;

- CHV is not blocked.

The old and new CHV shall be presented.

If the old CHV presented is correct, the number of remaining CHV attempts for that CHV shall be reset to its initial value 3 and the new value for the CHV becomes valid.

If the old CHV presented is false, the number of remaining CHV attempts for that CHV shall be decremented and the value of the CHV is unchanged. After 3 consecutive false CHV presentations, not necessarily in the same card session, the respective CHV shall be blocked and the access condition can never be fulfilled until the UNBLOCK CHV function has been performed successfully on the respective CHV.

SIM_DisableCHV

This function may only be applied to CHV1. The successful execution of this function has the effect that files protected by CHV1 are now accessible as if they were marked "ALWAYS". The function DISABLE CHV shall not be executed by the SIM when CHV1 is already disabled or blocked.

If the CHV1 presented is correct, the number of remaining CHV1 attempts shall be reset to its initial value 3 and CHV1 shall be disabled.

If the CHV1 presented is false, the number of remaining CHV1 attempts shall be decremented and CHV1 remains enabled. After 3 consecutive false CHV1 presentations, not necessarily in the same card session, CHV1 shall be blocked and the access condition can never be fulfilled until the UNBLOCK CHV function has been successfully performed on CHV1.

SIM_EnableCHV

This function may only be applied to CHV1. It is the reverse function of DISABLE CHV. The function ENABLE CHV shall not be executed by the SIM when CHV1 is already enabled or blocked.

If the CHV1 presented is correct, the number of remaining CHV1 attempts shall be reset to its initial value 3 and CHV1 shall be enabled.

If the CHV1 presented is false, the number of remaining CHV1 attempts shall be decremented and CHV1 remains disabled. After 3 consecutive false CHV1 presentations, not necessarily in the same card session, CHV1 shall be blocked and the access condition can never be fulfilled until the UNBLOCK CHV function has been successfully performed on CHV1.

SIM_UnblockCHV

This function unblocks a CHV which has been blocked by 3 consecutive wrong CHV presentations. This function may be performed whether or not the relevant CHV is blocked.

If the UNBLOCK CHV presented is correct, the value of the CHV, presented together with the UNBLOCK CHV, is assigned to that CHV, the number of remaining UNBLOCK CHV attempts for that UNBLOCK CHV is reset to its initial value 10 and the number of remaining CHV attempts for that CHV is reset to its initial value 3. After a successful unblocking attempt the CHV is enabled and the relevant access condition level is satisfied.

If the presented UNBLOCK CHV is false, the number of remaining UNBLOCK CHV attempts for that UNBLOCK CHV shall be decremented. After 10 consecutive false UNBLOCK CHV presentations, not necessarily in the same card session, the respective UNBLOCK CHV shall be blocked. A false UNBLOCK CHV shall have no effect on the status of the respective CHV itself.

SIM_Invalidate

This function invalidates the current elementary field. After an INVALIDATE function the respective flag in the file status shall be changed accordingly. This function shall only be performed if the INVALIDATE access condition for the current elementary field is satisfied.

An invalidated file shall no longer be available within the application for any function except for the SELECT and the REHABILITATE functions.

SIM_Rehabilitate

This function rehabilitates the invalidated current elementary field. After a REHABILITATE function the respective flag in the file status shall be changed accordingly. This function shall only be performed if the REHABILITATE access condition for the current elementary field is satisfied.

SIM_RunGSMAlgo

This function is used during the procedure for authenticating the SIM to a GSM network and to calculate a cipher key. The card runs the specified algorithms A3 and A8 using a 16 byte random number and the subscriber authentication key Ki, which is stored in the SIM. The function returns the calculated response SRES and the cipher key Kc.

The function shall not be executable unless DFGSM has been selected as the Current Directory and a successful CHV1 verification procedure has been performed.

The contents of Kc shall be presented to algorithm A5 by the mobile station in its full 64 bit format as delivered by the SIM.

SIM_GetResponse

The response data depends on the preceding command. Response data is available after the commands RUN GSM ALGORITHM, SEEK (type 2), SELECT, and INCREASE. If the command GET RESPONSE is executed, it is required that it is executed immediately after the command it is related to (no other command shall come between the command/response pair and the command GET RESPONSE). If the sequence is not respected, the SIM shall send the status information "technical problem with no diagnostic given" as a reaction to the GET RESPONSE.

Since the master file is implicitly selected after activation of the SIM, GET RESPONSE is also allowed as the first command after activation.

SIM_PowerOff

The driver call is used to switch off the SIM driver during power off.

SIM_TerminalProfile (SIM Toolkit only)

This function is used by the ME to transmit to the SIM its capabilities concerning the SIM Application Toolkit functionality.

SIM_TerminalResponse (SIM Toolkit only)

This function is used to transfer from the ME to the SIM the response to a previously fetched SIM Application Toolkit command.

SIM_Fetch (SIM Toolkit only)

This function is used to transfer an Application Toolkit command from the SIM to the ME.

SIM_Envelope (SIM Toolkit only)

This function is used to transfer data to the SIM Application Toolkit applications in the SIM.

# General Aspects

The Application has a primitive interface to the protocol stack. It uses one or a combination of SIM driver calls for the requested services by the protocol stack. It is mapping the result codes of the SIM driver and handles error conditions.

## Error Causes

Each driver call has a result buffer. The two last bytes of the result buffer contains the error variables SW1 and SW2. The following table defines the mapping of this variables to the error codes of the SIM application:

| SW 1 | SW 2 | Error Code | Remark |
| --- | --- | --- | --- |
| 0x90 | - | No error |  |
| 0x94 | 0,4 | Unknown data field |  |
|  | 2 | Invalid offset |  |
|  | 8 | Invalid access |  |
| 0x98 | 2,4,8 | Invalid PIN | See table 1 |
|  | 0x40 | Invalid PUK | See table 2 |
|  | 0x50 | Max value |  |
| 0x91 | - | No error Fatal error | If proactive SIM card SW 2 Bytes Proactive SIM data available If no proactive SIM card |
| 0x9F | - | No error | SW 2 Byte Response data available |
| ELSE | - | Fatal error |  |

If an invalid PIN is detected the error code depends on the last requested PIN number according to table 1:

| Last requested PIN number | Error code |
| --- | --- |
| None | Invalid PIN 1 |
| PIN 1 | Invalid PIN 1 |
| PIN 2 | Invalid PIN 2 |
| PUK 1 | Invalid PUK 1 |
| PUK 2 | Invalid PUK 2 |

If an invalid PUK (unblocking key) is detected the error code depends on the last requested PIN number according to table 2:

| Last requested PIN number | Error code |
| --- | --- |
| None | Invalid PUK 1 |
| PIN 1 | Invalid PUK 1 |
| PIN 2 | Invalid PUK 2 |
| PUK 1 | Invalid card |
| PUK 2 | Invalid card |

## Access Conditions

For each SIM field the GSM 11.11 defines access conditions for read and update functions. The following access conditions are possible:

ALWAYS	The access to the SIM field is possible without restrictions.

PIN 1	The access to the SIM field is possible after entering PIN 1. This is normally done after Power on.

PIN 2	The access to the SIM field is possible after entering PIN 2.

PIN 1 or 2	The access condition must be detemined by the SIM application (using GET RESPONSE). The needed PIN number is signalled to the MMI. The use of PIN 1 or 2 is a choice of the SIM card manufacturer.

NEVER	The access to the SIM field is not allowed.

The following access conditions are used:

| SIM field | Read Operation | Update Operation |
| --- | --- | --- |
| Preferred Language | ALWAYS | PIN 1 |
| Authentication Parameter Kc | PIN 1 | PIN 1 |
| Cell Broadcast Message Identifier | PIN 1 | PIN 1 |
| BCCH information | PIN 1 | PIN 1 |
| Forbidden PLMN list | PIN 1 | PIN 1 |
| Preferred PLMN list | PIN 1 | PIN 1 |
| Automatic Answer for eMLPP Service | PIN 1 | PIN 1 |
| Cell broadcast message identifier range selection | PIN 1 | PIN 1 |
| Short messages | PIN 1 | PIN 1 |
| Capability configuration parameters | PIN 1 | PIN 1 |
| MSISDN | PIN 1 | PIN 1 |
| Short message service parameters | PIN 1 | PIN 1 |
| SMS status | PIN 1 | PIN 1 |
| Last number dialled | PIN 1 | PIN 1 |
| Extension1 | PIN 1 | PIN 1 |
| Location information | PIN 1 | PIN 1 |
| Abbreviated dialling numbers | PIN 1 | PIN 1 |
| ACM maximum value | PIN 1 | PIN 1 or 2 |
| Price per unit and currency table | PIN 1 | PIN 1 or 2 |
| Accumulated call meter | PIN 1 | PIN 1 or 2 |
| Service Provider Name | ALWAYS | NEVER |
| Phase identification | ALWAYS | NEVER |
| Administrative data | ALWAYS | NEVER |
| Emergency Call Codes | ALWAYS | NEVER |
| Fixed dialling numbers | PIN 1 | PIN 2 |
| Extension 2 | PIN 1 | PIN 2 |
| Extension 4 | PIN 1 | PIN 2 |
| Barred dialling numbers | PIN 1 | PIN 2 |
| Else | PIN 1 | NEVER |

# Basic Functions

## Read Binary File

MMI/SMS/MM                     SIM

|                             |

|        SIM READ REQ         |

*============================>*

|                          (SIM 1)

|                             |

a) No SIM inserted

|                             |

|        SIM READ CNF         |

*<============================*

(PS 1)                          |

|                             |

b) no access rights

|                             |

|                             *<SIM_Select    Directory>

|                             *<SIM_Select    Field>

|                          (SIM 2)

|                             |

|        SIM READ CNF         |

*<============================*

(PS 2)                          |

|                             |

c) SIM inserted

|                             |

|                             *<SIM_Select    Directory>

|                             *<SIM_Select    Field>

|                             *<SIM_ReadBinary>

|                          (SIM 3)

|                             |

|        SIM READ CNF         |

*<============================*

(PS 3)                          |

|                             |

(SIM 1)

A protocol stack component requests reading of a data field.

(PS 1)

If no SIM is inserted, the SIM application rejects this request with an error cause SIM_FATAL_ERROR.

(SIM 2)

The requested data field is selected using the SIM driver calls SIM_Select. If the actual directory differs from the needed directory a SIM_Select is carried out for the needed directory. If the actual field differs from the needed field, a SIM_Select is carried out for the needed field. The dependency between field and directory is hardcoded. The access rights are checked. If it is not allowed to read the field the reading is not processed.

(PS 2)

The result of the reading attempt is forwarded to the requesting procotol stack component (error cause SIM_INVALID_ACCESS).

(SIM 3)

The requested data field is selected using the SIM driver calls SIM_Select. If the actual directory differs from the needed directory a SIM_Select is carried out for the needed directory. If the actual field differs from the needed field, a SIM_Select is carried out for the needed field. The dependency between field and directory is hardcoded. After successfull selection the content of the field is read with the SIM driver call SIM_ReadBinary.

(PS 3)

The result of the reading attempt is forwarded to the requesting procotol stack component.

## Read Record File

MMI/SMS/MM                     SIM

|                             |

|     SIM READ RECORD REQ     |

*============================>*

|                          (SIM 1)

|                             |

a) No SIM inserted

|                             |

|     SIM READ RECORD CNF     |

*<============================*

(PS 1)                          |

|                             |

b) no access rights

|                             |

|                             *<SIM_Select    Directory>

|                             *<SIM_Select    Field>

|                          (SIM 2)

|                             |

|     SIM READ RECORD CNF     |

*<============================*

(PS 2)                          |

|                             |

c) SIM inserted

|                             |

|                             *<SIM_Select    Directory>

|                             *<SIM_Select    Field>

|                             *<SIM_GetResponse>

|                             *<SIM_ReadBinary>

|                          (SIM 3)

|                             |

|     SIM READ RECORD CNF     |

*<============================*

(PS 3)                          |

|                             |

(SIM 1)

A protocol stack component requests reading of a record of a cyclic or linear fixed data field.

(PS 1)

If no SIM is inserted, the SIM application rejects this request with an error cause SIM_FATAL_ERROR.

(SIM 2)

The requested data field is selected using the SIM driver calls SIM_Select. If the actual directory differs from the needed directory a SIM_Select is carried out for the needed directory. If the actual field differs from the needed field, a SIM_Select is carried out for the needed field. The dependency between field and directory is hardcoded. The access rights are checked. If it is not allowed to read the field the reading is not processed.

(PS 2)

The result of the reading attempt is forwarded to the requesting procotol stack component (error cause SIM_INVALID_ACCESS).

(SIM 3)

The requested data field is selected using the SIM driver calls SIM_Select. If the actual directory differs from the needed directory a SIM_Select is carried out for the needed directory. If the actual field differs from the needed field, a SIM_Select is carried out for the needed field. The dependency between field and directory is hardcoded.

If the record number is one, that means the first record, additionally the maximum number of records will be calculated. Therefore the SIM_Get_Response driver call is used. The maximum number of records will be calculated and is part of the response to the requesting component. Else the maximum number of records will not be calculated and is set to zero in the response.

After successfull selection the content of the record  is read with the SIM driver call SIM_ReadRecord.

(PS 3)

The result of the reading attempt is forwarded to the requesting procotol stack component.

## Update Binary File

MMI/SMS/MM                     SIM

|                             |

|      SIM UPDATE REQ         |

*============================>*

|                          (SIM 1)

|                             |

a) No SIM inserted

|                             |

|      SIM UPDATE CNF         |

*<============================*

(PS 1)                          |

|                             |

b) no access rights

|                             |

|                             *<SIM_Select    Directory>

|                             *<SIM_Select    Field>

|                          (SIM 2)

|                             |

|      SIM UPDATE CNF         |

*<============================*

(PS 2)                          |

|                             |

c) SIM inserted

|                             |

|                             *<SIM_Select    Directory>

|                             *<SIM_Select    Field>

|                             *<SIM_UpdateBinary>

|                          (SIM 3)

|                             |

|      SIM UPDATE CNF         |

*<============================*

(PS 3)                          |

|                             |

(SIM 1)

A protocol stack component requests updating of a data field.

(PS 1)

If no SIM is inserted, the SIM application rejects this request with an error cause SIM_FATAL_ERROR.

(SIM 2)

The requested data field is selected using the SIM driver calls SIM_Select. If the actual directory differs from the needed directory a SIM_Select is carried out for the needed directory. If the actual field differs from the needed field, a SIM_Select is carried out for the needed field. The dependency between field and directory is hardcoded.  The access rights are checked. If it is not allowed to update the field the updating is not processed.

(PS 2)

The result of the updating attempt is forwarded to the requesting procotol stack component (error cause SIM_INVALID_ACCESS).

(SIM 3)

The requested data field is selected using the SIM driver calls SIM_Select. If the actual directory differs from the needed directory a SIM_Select is carried out for the needed directory. If the actual field differs from the needed field, a SIM_Select is carried out for the needed field. The dependency between field and directory is hardcoded. After successfull selection the content of the field is updated with the SIM driver call SIM_UpdateBinary.

(PS 3)

The result of the updating attempt is forwarded to the requesting procotol stack component.

## Update Record File

MMI/SMS/MM                     SIM

|                             |

|   SIM UPDATE RECORD REQ     |

*============================>*

|                          (SIM 1)

|                             |

a) No SIM inserted

|                             |

|   SIM UPDATE RECORD CNF     |

*<============================*

(PS 1)                          |

|                             |

b) no access rights

|                             |

|                             *<SIM_Select    Directory>

|                             *<SIM_Select    Field>

|                          (SIM 2)

|                             |

|   SIM UPDATE RECORD CNF     |

*<============================*

(PS 2)                          |

|                             |

c) SIM inserted

|                             |

|                             *<SIM_Select    Directory>

|                             *<SIM_Select    Field>

|                             *<SIM_UpdateBinary>

|                          (SIM 3)

|                             |

|   SIM UPDATE RECORD CNF     |

*<============================*

(PS 3)                          |

|                             |

(SIM 1)

A protocol stack component requests update of a record of a cyclic or linear fixed data field.

(PS 1)

If no SIM is inserted, the SIM application rejects this request with an error cause SIM_FATAL_ERROR.

(SIM 2)

The requested data field is selected using the SIM driver calls SIM_Select. If the actual directory differs from the needed directory a SIM_Select is carried out for the needed directory. If the actual field differs from the needed field, a SIM_Select is carried out for the needed field. The dependency between field and directory is hardcoded. The access rights are checked. If it is not allowed to update the field the updating is not processed.

(PS 2)

The result of the updating attempt is forwarded to the requesting procotol stack component (error cause SIM_INVALID_ACCESS).

(SIM 3)

The requested data field is selected using the SIM driver calls SIM_Select. If the actual directory differs from the needed directory a SIM_Select is carried out for the needed directory. If the actual field differs from the needed field, a SIM_Select is carried out for the needed field. The dependency between field and directory is hardcoded.

After successfull selection the content of the record  is updated with the SIM driver call SIM_UpdateRecord.

(PS 3)

The result of the updating attempt is forwarded to the requesting procotol stack component.

## Increment

MMI/SMS/MM                     SIM

|                             |

|      SIM INCREMENT REQ      |

*============================>*

|                          (SIM 1)

|                             |

a) No SIM inserted

|                             |

|      SIM INCREMENT CNF      |

*<============================*

(PS 1)                          |

|                             |

b) SIM inserted

|                             |

|                             *<SIM_Select        Directory>

|                             *<SIM_Select        Field>

|                             *<SIM_Increase>

|                             *<SIM_GetResponse>

|                          (SIM 2)

|                             |

|      SIM INCREMENT CNF      |

*<============================*

(PS 2)                          |

|                             |

(SIM 1)

One protocol stack component requests incrementing of a cyclic data field.

(PS 1)

If no SIM is inserted, the SIM application rejects this request with an error cause.

(SIM 2)

The requested data field is selected using the SIM driver calls SIM_Select. After successfull selection the field is incremented using the SIM driver call SIM_Increase. The result of the operation is requested by the SIM driver call SIM_GetResponse.

(PS 2)

The result of the operation is forwarded to the requesting procotol stack component.

## Verify PIN

MMI/SMS/MM                     SIM

|                             |

|      SIM VERIFY PIN REQ     |

*============================>*

|                          (SIM 1)

|                             |

a) SIM not inserted

|                             |

|      SIM VERIFY PIN CNF     |

*<============================*

(PS 1)                          |

|                             |

b) SIM inserted

|                             *<SIM_VerifyCHV>

|                          (SIM 2)

|      SIM VERIFY PIN CNF     |

*<============================*

(PS 2)                          |

|                             |

|                             |

c) PIN ok and SIM just inserted

|                             |

|                             *<read SIM parameters>

|                          (SIM 3)

|                             |

(SIM 1)

MMI requests verification of the PIN.

(PS 1)

If no SIM is inserted, the SIM application rejects this request with an error cause.

(SIM 2)

The PIN is verified using the SIM driver call SIM_VerifyCHV.

(PS 2)

The result of the operation is forwarded to the requesting procotol stack component.

(SIM 3)

If the PIN verify is successfull and the SIM was just inserted, the SIM application starts reading the SIM parameters as described into another chapter of this specification.

## Change PIN

MMI/SMS/MM                     SIM

|                             |

|      SIM CHANGE PIN REQ     |

*============================>*

|                          (SIM 1)

|                             |

b) No SIM inserted

|                             |

|      SIM CHANGE PIN CNF     |

*<============================*

(PS 1)                          |

|                             |

b) SIM inserted

|                             |

|                             *<SIM_ChangeCHV>

|                          (SIM 2)

|      SIM CHANGE PIN CNF     |

*<============================*

(PS 2)                          |

|                             |

(SIM 1)

MMI requests the change of a PIN.

(PS 1)

If no SIM is inserted, the SIM application rejects this request with an error cause.

(SIM 2)

The PIN is changed using the SIM driver call SIM_ChangeCHV.

(PS 2)

The result of the operation is forwarded to the requesting procotol stack component.

## Disable PIN

MMI/SMS/MM                     SIM

|                             |

|      SIM DISABLE PIN REQ    |

*============================>*

|                          (SIM 1)

|                             |

b) No SIM inserted or PIN is disabled

|                             |

|      SIM DISABLE PIN CNF    |

*<============================*

(PS 1)                          |

|                             |

b) SIM inserted

|                             |

|                             *<SIM_DisableCHV>

|                          (SIM 2)

|      SIM DISABLE PIN CNF    |

*<============================*

(PS 2)                          |

|                             |

(SIM 1)

MMI requests disabling of a PIN.

(PS 1)

If no SIM is inserted, the SIM application rejects this request with an error cause. If a SIM is inserted and the card is already disabled, the procedure ends successful immediately.

(SIM 2)

The PIN is disabled using the SIM driver call SIM_DisableCHV.

(PS 2)

The result of the operation is forwarded to the requesting procotol stack component.

## Enable PIN

MMI/SMS/MM                     SIM

|                             |

|      SIM ENABLE PIN REQ     |

*============================>*

|                          (SIM 1)

|                             |

b) No SIM inserted or card is enabled

|                             |

|      SIM ENABLE PIN CNF     |

*<============================*

(PS 1)                          |

|                             |

b) SIM inserted

|                             |

|                             *<SIM_EnableCHV>

|                          (SIM 2)

|      SIM ENABLE PIN CNF     |

*<============================*

(PS 2)                          |

|                             |

(SIM 1)

MMI requests enabling of a PIN.

(PS 1)

If no SIM is inserted, the SIM application rejects this request with an error cause. If a SIM card is inserted and the card is enabled, the procedure ends successful immediately.

(SIM 2)

The PIN is enabled using the SIM driver call SIM_EnableCHV.

(PS 2)

The result of the operation is forwarded to the requesting procotol stack component.

## Unblock PIN

MMI/SMS/MM                     SIM

|                             |

|      SIM UNBLOCK REQ        |

*============================>*

|                          (SIM 1)

|                             |

b) No SIM inserted

|                             |

|      SIM UNBLOCK CNF        |

*<============================*

(PS 1)                          |

|                             |

b) SIM inserted

|                             |

|                             *<SIM_UnblockCHV>

|                          (SIM 2)

|      SIM UNBLOCK CNF        |

*<============================*

(PS 2)                          |

|                             |

c) PUK 1 ok and SIM just inserted

|                             |

|                             *<read SIM parameters>

|                          (SIM 3)

|                             |

(SIM 1)

MMI requests unblocking of a PIN with the PUK.

(PS 1)

If no SIM is inserted, the SIM application rejects this request with an error cause.

(SIM 2)

The PIN is unblocked using the SIM driver call SIM_UnblockCHV.

(PS 2)

The result of the operation is forwarded to the requesting procotol stack component.

(SIM 3)

If the PUK verify is successfull and the SIM was just inserted, the SIM application starts reading the SIM parameters as described into another chapter of this specification.

## Authentication

MMI/SMS/MM                     SIM

|                             |

|   SIM AUTHENTICATION REQ    |

*============================>*

|                          (SIM 1)

|                             |

b) No SIM inserted

|                             |

|   SIM AUTHENTICATION CNF    |

*<============================*

(PS 1)                          |

|                             |

b) SIM inserted

|                             |

|                             *<SIM_Select        GSM/DCS 1800 Directory>

|                             *<SIM_RunGSMAlgo>

|                             *<SIM_GetResponse>

|                          (SIM 2)

|   SIM AUTHENTICATION CNF    |

*<============================*

(PS 2)                          |

|                             *<SIM_Select        GSM/DCS 1800 Directory>

|                             *<SIM_Select        KC_N>

|                             *<SIM_UpdateBinary  KC_N>

|                          (SIM 3)

|                             |

(SIM 1)

MM requests running of the GSM algorithm.

(PS 1)

If no SIM is inserted, the SIM application rejects this request with an error cause.

(SIM 2)

The GSM directory is selected by the SIM driver call SIM_Select. After successfull selection the GSM algoritm is triggered using the SIM driver call SIM_RunGSMAlgo. The result parameters are requested by the SIM driver call SIM_GetResponse.

(PS 2)

The result of the operation (authentication parameter Sres and Kc) is forwarded to the requesting procotol stack component.

(SIM 3)

The calculated Kc value is stored on the SIM card. Therefore the KC_N data field is selected using the SIM driver calls SIM_Select and written by the SIM driver call SIM_UpdateBinary.

# Structured Procedures

## SIM Insertion

### Access to Main Directory

MMI/SMS/MM                     SIM

|                             |

|       SIM ACTIVATE REQ      |

*============================>*

|                             *<SIM_Reset>

|                          (SIM 1)

|                             *<SIM Insertion signal>

|                          (SIM 2)

|                             *<SIM_Select        MF Directory>

|                          (SIM 3)

|                             |

a) Selection Main Directory unsuccessfull

|                             |

|       SIM ACTIVATE CNF      |

|        (invalid card)       |

*<============================*

(MMI 1)                         |

|                             |

b) Selection Main Directory successfull

|                             |

|                             *<SIM_Select        GSM Directory>

|                             *<SIM_GetResponse>

|                          (SIM 4)

|                             |

c) Selection GSM Directory unsuccessfull

|                             |

|                             *<SIM_Select        DCS 1800 Directory>

|                             *<SIM_GetResponse>

|                          (SIM 5)

|                             |

d) Selection DCS 1800 Directory unsuccessfull

|                             |

|       SIM ACTIVATE CNF      |

|        (invalid card)       |

*<============================*

(MMI 2)                         |

|                             |

(SIM 1)

MMI sends the primitive to start activation of the SIM card. The SIM driver is resetted. The callback functions for SIM insertion and SIM remove are installed.

(SIM 2)

The SIM driver has detected a SIM card and the callback function for SIM insertion is called by the SIM driver.

(SIM 3)

The SIM application selectes the Main File Directory.

(MMI 1)

If is it impossible to select the Main File Directory, the SIM card is invalid and the MMI is informed.

(SIM 4)

After successful selection of the Main File Directory the SIM application selects the GSM Directory. If this is successful it reads the response data of this field.

(SIM 5)

If the selection of the GSM directory is unsuccessful, the SIM application starts the selection of the DCS 1800 directory instead. This is done for backward compatibility for Phase 1 cards for DCS 1800.

(MMI 2)

If also the selection of the DCS 1800 directory fails the SIM card is invalid and this is signalled to the MMI.

### Check of PIN / PUK counter

MMI/SMS/MM                     SIM

|                             |

a) SIM card is blocked, unblocking attempts available

|                             |

|                             *<SIM_Select        GSM/DCS 1800 Directory>

|                             *<SIM_Select        Phase>

|                             *<SIM_ReadBinary    Phase>

|                             *<SIM_Select        Emergency Codes>

|                             *<SIM_ReadBinary    Emergency Codes>

|                             *<SIM_Select        Preferred Languages>

|                             *<SIM_ReadBinary    Preferred Languages>

|                          (SIM 1)

|       SIM ACTIVATE CNF      |

|        (invalid PUK 1)      |

*<============================*

(MMI 1)                         |

|                             |

b) SIM card is blocked, no unblocking attempts available

|                             |

|       SIM ACTIVATE CNF      |

|        (invalid card)       |

*<============================*

(MMI 2)                         |

|                             |

c) SIM card is okay, no PIN entering

|                             |

|                             *<SIM_Select        GSM/DCS 1800 Directory>

|                             *<SIM_Select        Phase>

|                             *<SIM_ReadBinary    Phase>

|                             *<SIM_Select        Emergency Codes>

|                             *<SIM_ReadBinary    Emergency Codes>

|                             *<SIM_Select        Preferred Languages>

|                             *<SIM_ReadBinary    Preferred Languages>

|                          (SIM 2)

|       SIM ACTIVATE CNF      |

|         (no error)          |

*<============================*

(MMI 3)                         |

|                             |

d) SIM card needs PIN 1, attempts available

|                             |

|                             *<SIM_Select        GSM/DCS 1800 Directory>

|                             *<SIM_Select        Phase>

|                             *<SIM_ReadBinary    Phase>

|                             *<SIM_Select        Emergency Codes>

|                             *<SIM_ReadBinary    Emergency Codes>

|                             *<SIM_Select        Preferred Languages>

|                             *<SIM_ReadBinary    Preferred Languages>

|                          (SIM 3)

|       SIM ACTIVATE CNF      |

|        (invalid PIN 1)      |

*<============================*

(MMI 4)                         |

(SIM 1)

The response data from selecting the GSM / DCS 1800 directory contains the blocking status and the number of remaining PIN / PUK attempts. If the SIM card is blocked, but unblocking attempts are still available, the SIM application reads the elementary fields PHASE, EMERGENCY CODES and PREFERRED LANGUAGES. The field PHASE must be available, the two other fields are optional.

(MMI 1)

The MMI is informed that the SIM card is blocked with the cause Invalid PUK 1. This is the trigger for entering PUK 1 in the MMI. The primitive to MMI contains additional the parameters read from the SIM card.

(MMI 2)

The response data from selecting the GSM / DCS 1800 directory contains the blocking status and the number of remaining PIN / PUK attempts. The SIM card is blocked and no unblocking attempts are available. The SIM card is invalid. This is signalled to MMI.

(SIM 2)

The response data from selecting the GSM / DCS 1800 directory contains the PIN / PUK status and the number of remaining PIN / PUK attempts. If the SIM card is unblocked and needs no PIN entering, the SIM application reads the elementary fields PHASE, EMERGENCY CODES and PREFERRED LANGUAGES. The field PHASE must be available, the two other fields are optional.

(MMI 3)

The MMI is informed that the SIM card is okay and no PIN entering is needed with the cause No Error. This is the trigger for starting registration in the MMI. The primitive to MMI contains additional the parameters read from the SIM card.

(SIM 3)

The response data from selecting the GSM / DCS 1800 directory contains the PIN / PUK status and the number of remaining PIN / PUK attempts. If the SIM card is unblocked but  PIN 1 entering is needed, the SIM application reads the elementary fields PHASE, EMERGENCY CODES and PREFERRED LANGUAGES. The field PHASE must be available, the two other fields are optional.

(MMI 4)

The MMI is informed that the SIM card is okay and PIN 1 entering is needed with the cause Invalid Pin 1. This is the trigger for  requesting PIN 1 in the MMI. The primitive to MMI contains additional the parameters read from the SIM card.

### Reading of SIM Parameters

The SIM parameters are read

- after activation, if PIN entering is disabled, or

- after successful verification of PIN1 or

- after successful unblocking the SIM card with PUK 1

MMI/SMS/MM                     SIM

|                             |

a) Phase 1 card

|                             |

|                             *<Read MM Parameter >

|                             *<Read MMI Parameter>

|                             *<Read SMS Parameter>

|                          (SIM 1)

|                             |

b) Phase 2 card

|                             |

|                             *<Read MM Parameter >

|                             *<Process FDN/BDN procedures>

|                             *<Read MMI Parameter>

|                             *<Read SMS Parameter>

|                          (SIM 2)

|                             |

c) Phase 2+ card

|                             |

|                             *<Read MM Parameter >

|                             *<Perform Profile Download>

|                             *<Process FDN/BFDN procedures>

|                             *<Read MMI Parameter>

|                             *<Read SMS Parameter>

|                          (SIM 3)

|                             |

(SIM 1)

If the SIM card is a phase 1 card, the SIM application reads the MM parameter and forwards them to MM, reads the MMI parameter and forwards them to MMI and reads the SMS parameter and forwards them to SMS.

(SIM 2)

If the SIM card is a phase 2 card, the SIM application reads the MM parameter and forwards them to MM. Then it checks the capabilities of restricted or unrestricted operation as described in the chapter Process FDN / BDN procedures. Then it reads the MMI parameter and forwards them to MMI and reads the SMS parameter and forwards them to SMS.

(SIM 3)

A phase 2+ card processes the same procedures like a phase 2 card. It additionally process the profile download for SIM toolkit.

### Read MM Parameter

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM_Select        GSM/DCS 1800 Directory>

|                             *<SIM_Select        Administrative Data>

|                             *<SIM_ReadBinary    Administrative Data>

|                             *<SIM_Select        SIM Service Table>

|                             *<SIM_ReadBinary    SIM Service Table>

|                             *<SIM_Select        IMSI>

|                             *<SIM_ReadBinary    IMSI>

|                             *<SIM_Select        Location Information>

|                             *<SIM_ReadBinary    Location Information>

|                             *<SIM_Select        Access Control Classes>

|                             *<SIM_ReadBinary    Access Control Classes>

|                             *<SIM_Select        BCCH Information>

|                             *<SIM_ReadBinary    BCCH Information>

|                             *<SIM_Select        Kc>

|                             *<SIM_ReadBinary    Kc>

|                             *<SIM_Select        Preferred PLMN>

|                             *<SIM_ReadBinary    Preferred PLMN>

|                             *<SIM_Select        Forbidden PLMN>

|                             *<SIM_ReadBinary    Forbidden PLMN>

|                             *<SIM_Select        HPLMN (only > phase 1)>

|                             *<SIM_ReadBinary    HPLMN (only > phase 1)>

|                          (SIM 1)

|       SIM MM INSERT IND     |

*<============================*

(MM 1)                          |

|                             |

(SIM 1)

MM needs the content of several fields of the SIM card. This data are used for registration purposes in the protocol stack. The SIM application reads the following fields: ADMINISTRATIVE DATA, SIM SERVICE TABLE, IMSI, LOCATION INFORMATION, ACCESS CONTROL CLASSES, BCCH INFORMATION, KC, PREFERRED PLMN LIST, FORBIDDEN PLMN and HPLMN if the SIM card is not a phase 1 card.

(MM 1)

All parameters are filled in a SIM MM INSERT IND primitive and forwarded to MM.

### Read MMI Parameter

MMI/SMS/MM                     SIM

|                             |

|                             *<copy MM parameter>

|                          (SIM 1)

|                             *<SIM_Select        GSM/DCS 1800 Directory>

|                             *<SIM_Select        CB Message Identifier>

|                             *<SIM_ReadBinary    CB Message Identifier>

|                          (SIM 2)

|                             *<SIM_Select        GSM/DCS 1800 Directory>

|                             *<SIM_Select        De-personalisation CK>

|                             *<SIM_ReadBinary    De-personalisation CK>

|                          (SIM 3)

|      SIM MMI INSERT IND     |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Some of the MM parameters are also needed by MMI. The SIM application copies this parameters to the SIM MMI INSERT IND primitive.

(SIM 2)

If the SIM service table indicates that service 14 is activated and allocated, the SIM application selects the cell broadcast message identifier field and reads the content.

(SIM 3)

If the SIM service table indicates that service 33 is activated and allocated, the SIM application selects the de-personalisation key field and reads the content.

(MMI 1)

The content of the several datafields relevant to MMI are send with the primitive SIM MMI INSERT IND to MMI.

### Read SMS Parameter

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM_Select        GSM/DCS 1800 Directory>

|                             *<SIM_Select        SMS Status>

|                             *<SIM_ReadBinary    SMS Status>

|                          (SIM 1)

|                             *<set download SMS flag>

|                          (SIM 2)

|      SIM SMS INSERT IND     |

*<============================*

(SMS 1)                         |

|                             |

(SIM 1)

The SIM application reads the SMS status field for the initialization of SMS. This field contains the last used message reference and a flag whether short message memory is available on the SIM card.

(SIM 2)

The download SMS flag indicates SMS whether the SIM card supports Data Download to the SIM card via SMS. This is only supported if

- The SIM card is a phase 2+ card,

- SIM toolkit is supported by the mobile and

- Service 26 of the SIM service table is allocated and activated.

(SMS 1)

The parameters are forwarded to SMS.

### FDN/BDN Procedures

This chapter describes the implementation of FDN / BDN checking procedures according to annex B of GSM 11.11.

The result of this procedure is the actual operation mode of the mobile. The following operation modes are possible:

ADN ENABLED	The SIM card has enabled unrestricted operation (abbreviated dialling). Fixed dialling and barred dialling are not available or de-activated.

ADN/BDN ENABLED	The SIM card has enabled unrestricted operation (abbreviated dialling). Fixed dialling is not available or de-activated. Additionally barred dialling is activated.

FDN ENABLED	The SIM card has enabled restricted operation (fixed dialling). Abbreviated dialling and barred dialling are not available or de-activated.

FDN/BDN ENABLED	The SIM card has enabled restricted operation (fixed dialling). Abbreviated dialling is not available or de-activated. Additionally barred dialling is activated.

NO OPERATION	The SIM card is invalid and no operation is allowed.

The first check is IMSI LOCI VALIDATION. Therefore the invalidation flag of the response after selection of IMSI and Location Information is checked. The result is TRUE or FALSE. If the result is TRUE, the operation mode of the SIM card is ADN ENABLED.

If this check fails then the rest of the procedure must be carried out as described in the following:

The BDN and FDN capability are checked.

The BDN CAPABILITY REQUEST means whether the SIM card supports barred dialling and whether it is enabled or disabled. The following results are possible NO BDN SIM, BDN ENABLED or BDN DISABLED. If service 31 of the SIM service table is unequal allocated and activates the result is NO BDN SIM. Else the BDN field is selected and the response data are read. Any failure during this procedure leads to the result NO BDN SIM. Then the validation flag of the response data is checked. If the validation flag is 0 the result is BDN DISABLED else BDN ENABLED.

The FDN CAPABILITY REQUEST means whether the SIM card supports fixed dialling and whether it is enabled or disabled. The following results are possible NO FDN SIM, FDN ENABLED or FDN DISABLED. If service 3 of the SIM service table is unequal allocated and activates the result is NO FDN SIM. If the abbreviated dialling field (service 2) is not allocated and activated the result is FDN ENABLED. Else the ADN field is selected and the response data are read. Any failure during this procedure leads to the result FDN ENABLED. Then the validation flag of the response data is checked. If the validation flag is 0 the result is FDN ENABLED else FDN DISABLED.

During checking the operation mode it is may be necessary to try rehabilitation of IMSI and Location Information. Therefore the IMSI field is selected and rehabilitated (using SIM driver call SIM_Rehabilitate). After this the same is carried out for the field Location Information. Any failure during this procedure leads to the result FALSE. Else the result of this procedure is TRUE.

The resulting operation mode for mobile with support of call control with SIM toolkit is given by the following rules:

- If the mobile doesn´t support BDN, but the SIM card supports BDN, the resulting operation mode is NO OPERATION.

- If the mobile doesn´t support FDN, but the SIM card supports FDN, the resulting operation mode is NO OPERATION.

- If the rehabilitation of IMSI and location information fails, the resulting operation mode is NO OPERATION.

- If the rehabilitation of IMSI and location information is successful, the resulting operation mode is set according to the following table:

|  | NO BDN SIM | BDN ENABLED | BDN DISABLED |
| --- | --- | --- | --- |
| NO FDN SIM | ADN ENABLED | ADN BDN ENABLED | ADN ENABLED |
| FDN ENABLED | FDN ENABLED | FDN BDN ENABLED | FDN ENABLED |
| FDN DISABLED | ADN ENABLED | ADN BDN ENABLED | ADN ENABLED |

The resulting operation mode for mobiles without supporting call control with the SIM card is given by the following rules:

- If the mobile doesn´t support FDN or FDN is not supported by the SIM the resulting operation mode is NO OPERATION.

- If rehabilitation of IMSI and location information fails, the resulting operation mode is NO OPERATION.

- If rehabilitation of IMSI and location information is successful, the resulting operation mode is FDN ENABLED.

## No SIM Inserted

MMI/SMS/MM                     SIM

|                             |

|       SIM ACTIVATE REQ      |

*============================>*

|                             *<SIM_Reset>

|                          (SIM 1)

|                             *<timeout>

|                          (SIM 2)

|       SIM ACTIVATE CNF      |

|        (fatal error)        |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

MMI sends the primitive to start activation of the SIM card. The SIM driver is resetted. The callback functions for SIM insertion and SIM remove are installed.

(SIM 2)

The SIM driver has not detected a SIM card and a timer in the SIM application times-out.

(MMI 1)

The MMI is informed with the cause fatal error about the absence of a SIM card.

## SIM Removing

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Remove signal>

|                          (SIM 1)

|        SIM REMOVE IND       |

*<============================*

(PS 1)                          |

|                             |

(SIM 1)

The SIM driver sends a signal that the SIM card is removed.

(PS 1)

Mobility management is informed about removing of the SIM card and starts de-registration.

## SIM Updating

MMI/SMS/MM                     SIM

|                             |

|      SIM MM UPDATE REQ      |

*============================>*

|                          (SIM 1)

|                             |

a) at power off

|                             |

|        SIM SYNC REQ         |

*============================>*

|                          (SIM 2)

|                             *<SIM_Select        GSM Directory>

|                             *<SIM_Select        Location Information>

|                             *<SIM_UpdateBinary  Location Information>

|                             *<SIM_Select        GSM Directory>

|                             *<SIM_Select        BCCH Information>

|                             *<SIM_UpdateBinary  BCCH Information>

|                             *<SIM_Select        GSM Directory>

|                             *<SIM_Select        Forbidden PLMNs>

|                             *<SIM_UpdateBinary  Forbidden PLMNs>

|                             *<SIM_Select        GSM Directory>

|                             *<SIM_Select        Kc_N>

|                             *<SIM_UpdateBinary  Kc_N>

|                          (SIM 3)

|        SIM SYNC CNF         |

*<============================*

(PS 1)                          |

|                             |

(SIM 1)

Whenever mobility management has changed location information, changed BCCH information, a changed forbidden PLMN list or a changed Kc or ciphering key sequence number it updates the SIM card content.

To avoid attempt to the SIM-Hardware the changes are stored only in the SIM application.

If SIM toolkit is supported by the mobile additionally the cell identity is stored by the SIM application.

(SIM 2)

If the mobile station is switched off, MMI sends a SIM SYNC REQ to the SIM application. This triggers the SIM application to update this data fields on the SIM card.

(PS 1)

If the synchronisation is finished a confirmation is send to the MMI.

## SIM Status

MMI/SMS/MM                     SIM

|                             |

|                             *<timeout>

|                             *<SIM Status>

|                          (SIM 1)

|                             |

a) successfull request

|                             *<start timer again>

|                          (SIM 2)

|                             |

b) unsuccessfull request

|       SIM REMOVE IND        |

*<============================*

(MM 1)                          |

|                             |

(SIM 1)

All thirty seconds the status timer times-out. The SIM driver call SIM status is used to check the presence of the SIM card.

(SIM 2)

If the request is sucessful, the timer is started again.

(MMI 1)

An unsuccessful request leads to the remove indication to MM.

## Change Operation Mode to Restricted Operation

MMI/SMS/MM                     SIM

|                             |

|       SIM ACTIVATE REQ      |

|         (fdn enable)        |

*============================>*

|                          (SIM 1)

|                             |

|                             *<SIM Select       ADN>

|                             *<SIM Invalidate>

|                          (SIM 2)

|                             |

a) successfull operation

|                             |

|       SIM ACTIVATE CNF      |

|          (no error)         |

*<============================*

(MMI 1)                         |

|                             |

b) unsuccessfull operation

|                             |

|       SIM ACTIVATE CNF      |

|         (invalidate)        |

*<============================*

(MMI 2)                         |

|                             |

(SIM 1)

MMI requests the change to restricted operation. This will be done only if

- The mobile is in unrestricted operation and

- Service 3 is allocated and activated (that means restricted operation is possible)

(SIM 2)

The SIM application selects the ADN field and invalidates it. This is implicit the rehabilitation of FDN and restricted operation.

(MMI 1)

The successful end of the procedure is signalled with the cause no error to MMI. The SIM service table inside of SIM application is changed.

(MMI 2)

An unsuccessful request leads to the error cause invalidate to MMI.

## Change Operation Mode to Unrestricted Operation

MMI/SMS/MM                     SIM

|                             |

|       SIM ACTIVATE REQ      |

|         (fdn disable)       |

*============================>*

|                          (SIM 1)

|                             |

|                             *<SIM Select       ADN>

|                             *<SIM Rehabilitate>

|                          (SIM 2)

|                             |

a) successfull operation

|                             |

|       SIM ACTIVATE CNF      |

|          (no error)         |

*<============================*

(MMI 1)                         |

|                             |

b) unsuccessfull operation

|                             |

|       SIM ACTIVATE CNF      |

|         (invalidate)        |

*<============================*

(MMI 2)                         |

|                             |

(SIM 1)

MMI requests the change to unrestricted operation. This will be done only if

- The mobile is in restricted operation and

- Service 2 is allocated and activated (that means unrestricted operation is possible)

(SIM 2)

The SIM application selects the ADN field and rehabilitates it.

(MMI 1)

The successful end of the procedure is signalled with the cause no error to MMI. The SIM service table inside of SIM application is changed.

(MMI 2)

An unsuccessful request leads to the error cause invalidate to MMI.

# SIM Toolkit

The SIM toolkit package is an upgrade of the protocol stack to support the SIM toolkit functionality according GSM 11.11. To include this functionality the SIM application must be compiled with the compile switch SIM_TOOLKIT and the sim toolkit object sim_stk.obj must be linked.

Additional support must be available by the MMI.

The following procedures / pro-active commands / SIM Toolkit Commands are supported:

- Profile Download

- REFRESH

- MORE TIME

- POLL INTERVALL

- POLLING OFF

- SET UP CALL

- SEND SS

- SEND SMS

- PLAY TONE

- DISPLAY TEXT

- GET INKEY

- GET INPUT

- SELECT ITEM

- SET UP MENU

- PROVIDE LOCAL INFO

- LAUNCH BROWSER

- OPEN CHANNEL

- CLOSE CHANNEL

- RECEIVE DATA

- SEND DATA

- GET CHANNEL STATUS

- Terminal Response

- Envelope

## Profile Download

MMI/SMS/MM                     SIM

|                             |

|       SIM ACTIVATE REQ      |

|       (Initialisation)      |

*============================>*

|                          (SIM 1)

|                             *<SIM TerminalProfile>

|                          (SIM 2)

|                             |

(SIM 1)

During Activation of the mobile MMI requests the initialisation of the SIM application. One parameter of the initiating SIM ACTIVATE REQ primitive is the SIM toolkit capability parameter. This parameter is a bitmap according GSM 11.11 describing the SIM toolkit capabilities of the mobile.

(SIM 2)

If a proactive SIM card is detected (phase 2+ card) during initialisation of the SIM application the profile download is performed. Therefore the mobile capabilities for SIM toolkit are forwarded to the SIM card using the SIM TerminalProfile driver call.

If this operation is successfull, the SIM application supports SIM toolkit during this session.

## Pro-active Commands

MMI/SMS/MM                     SIM

|                             |

|                             *<any SIM driver call>

|                          (SIM 1)

|                             *<SIM Fetch>

|                          (SIM 2)

|                             *....

|                             |

(SIM 1)

Any SIM driver Call has two result parameters SW1 and SW2. If the parameter SW1 is 0x91 or 0x9F this is an indication that the pro-active SIM card has a SIM Toolkit command to send. In this case the parameter SW2 indicates the length of the data.

(SIM 2)

Then the pro-active command is read from the SIM card using the driver call SIM Fetch. The following procedure depends on the pro-command type.

### Display Text

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the DISPLAY TEXT command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used to display text in the MMI display.

### Get Inkey

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the GET INKEY command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used to request a key input from the user.

### Get Input

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the GET INPUT command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used to get input (string) from the user.

### Play Tone

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the PLAY TONE command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used to play a tone in the earpiece.

### Refresh

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the Refresh command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used to initiate a refresh of some SIM fields up to a complete reset of the mobile.

### Set Up Menu

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the SET UP MENU command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used to install a menu in the MMI menu system.

### Select Item

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the SELECT ITEM command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used forward a item list to the MMI to select one of the items by the user.

### Send SMS

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the SEND SMS command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used to send a short message initiated from the SIM card.

### Send SS

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the SEND SS command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used to initiate a call independent supplementary service transaction by the SIM card.

### Set Up Call

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the SET UP CALL command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used to initiate a call establishment by the SIM card.

### Launch Browser

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the LAUNCH BROWSER command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used to load a page on the WAP browser.

### Set Up Event List

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|       SIM TOOLKIT IND       |

*<============================*

(MMI 1)                         |

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the SET UP EVENT LIST command. This is processed by MMI.

(MMI 1)

The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. A response of MMI is expected as described later. The command is used to define requested ME events.

### More Time

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|                             *<SIM TerminalResponse>

|                          (SIM 2)

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the MORE TIME command. This is processed by the SIM application internally.

(SIM 2)

The MORE TIME command indicates that the SIM card needs more time for operation. This has no affect to the SIM application. Using the SIM driver call SIM TerminalResponse an OK indication is send to the SIM card.

### Poll Intervall

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|                             *<SIM TerminalResponse>

|                          (SIM 2)

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the POLL INTERVALL command. This is processed by the SIM application internally.

(SIM 2)

The POLL INTERVALL command indicates that the SIM card will change the interval for the status requests. The new time is send with the unit minutes, seconds or ten seconds and will be calculated to the internal time unit. Using the SIM driver call SIM TerminalResponse an OK indication is send to the SIM card.

### Polling Off

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the POLLING OFF command. This is processed by the SIM application internally.

(SIM 2)

The POLLING OFF command indicates that the SIM shall use the default value of thirty seconds between to status requests. The used timer value in the SIM application is resetted to thirty seconds.

### Provide Local Information

MMI/SMS/MM                     SIM

|                             |

|                             *<SIM Fetch>

|                          (SIM 1)

|                             |

a) request location information

|                             |

|                             *<SIM TerminalResponse>

|                          (SIM 2)

|                             |

b) request IMEI

|                             |

|                             *<SIM TerminalResponse>

|                          (SIM 3)

|                             |

c) not supported information request

|                             |

|                             *<SIM TerminalResponse>

|                          (SIM 4)

|                             |

(SIM 1)

Using the SIM driver call SIM Fetch the pro-active command is read from the SIM card. It is the PROVIDE LOCAL INFORMATION command. This is processed by the SIM application internally.

(SIM 2)

The PROVIDE LOCAL INFORMATION command indicates that the SIM card requests location information. The SIM application builds a terminal response to the SIM card containing the mobile country code, the mobile network code, the location area identification and the cell identity.

(SIM 3)

The PROVIDE LOCAL INFORMATION command indicates that the SIM card requests the IMEI. The IMEI is read from the permanent configuration memory. The SIM application builds a terminal response to the SIM card containing the IMEI.

(SIM 4)

The PROVIDE LOCAL INFORMATION command indicates that the SIM card requests a not supported information. The SIM application builds a terminal response to the SIM card indicating that the SIM application has no capability to provide the requested information.

## Terminal Response

MMI/SMS/MM                     SIM

|                             |

|       SIM TOOLKIT RES       |

*============================>*

|                          (SIM 1)

|                             *<SIM TerminalResponse>

|                          (SIM 2)

|                             |

(SIM 1)

Most of the pro-active commands are forwarded to MMI and expect a response of the MMI. This response is send by MMI to the SIM application using the SIM TOOLKIT RES primitive.

(SIM 2)

The content of the primitive is forwarded to the SIM card using the SIM driver call SIM TerminalResponse.

## Envelope

MMI/SMS/MM                     SIM

|                             |

|       SIM TOOLKIT REQ       |

*============================>*

|                          (SIM 1)

|                             *<SIM Envelope>

|                          (SIM 2)

|                             |

a) response data from the SIM card is available

|                             |

|                             *<SIM GetResponse>

|                          (SIM 3)

|       SIM TOOLKIT CNF       |

*<============================*

(MMI/SMS 1)                     |

|                             |

(SIM 1)

MMI or SMS requests a SIM toolkit operation. An example is the Data Download via SMS to the SIM card.

(SIM 2)

The requested operation is forwarded to the SIM card using the SIM driver call SIM Envelope.

(SIM 3)

The result code of this Envelope SIM driver call (SW1/SW2) indicates whether response data of the SIM card are available. If this happens the data are read with the SIM driver call SIM GetResponse. Else the response data are not read and will be cleared in the response primitive to the requesting layer.

(MMI/SMS 1)

A response primitive is build by the SIM application and send to the requesting layer (MMI or SMS).

## SAT class e

### Open Channel

There are two types of link establishment:

- immediate link establishment

- on demand link establishment

If immediate link establishment is requested, MMI sets up the link, connects the SIM entity via DTI with an appropriate entity and sends a TERMINAL RESPONSE message to SIM.

If link establishment on demand is requested, MMI just stores the data and generates a TERMINAL RESPONSE message for SIM. The actual link establishment will be done if SIM sends a SEND DATA message to MMI with the information "send data immediately". Link establishment includes DTI configuration.

If MMI requests a DTI connection to UDP entity then the UDP SAP is also used by SIM entity.

#### Immediate link establishment

MMI                           SIM                          Card

|                             |                             |

|                             |        OPEN CHANNEL         |

|                             |         (immediate)         |

|                             *<============================*

|       SIM TOOLKIT IND       |                             |

|       (OPEN CHANNEL)        |                             |

*<============================*                             |

|                          (SIM 1)                          |

|         SIM_DTI_REQ         |                             |

|     (DTI configuration)     |                             |

|    (channel identifier)     |                             |

*============================>*                             |

|         SIM_DTI_CNF         |                             |

*<============================*                             |

|                          (SIM 2)                          |

|       SIM TOOLKIT RES       |                             |

|     (TERMINAL RESONSE)      |                             |

*============================>*                             |

|                             |      TERMINAL RESONSE       |

|                             *============================>*

|                          (SIM 3)                          |

|                             |                             |

(SIM 1)

SIM receives an OPEN CHANNEL command. The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. The command is used to open a channel for bearer independent protocol.

(SIM 2)

MMI requests a DTI connection and a bearer independent protocol channel (bits SIM_DTI_CONNECT and SIM_BIP_OPEN_CHANNEL are set in dti_conn parameter). It provides all necessary information including BIP channel identifier. If a UDP connection is requested (con_type parameter is set to SIM_CON_TYPE_UDP) then MMI also provides UDP relevant information (local_ip, destination_ip, destination_port). SIM opens the DTI connection and the BIP channel. After that SIM sends a confirm primitive to MMI.

(SIM 3)

MMI sends a TERMINAL RESPONSE message. The content of the primitive is forwarded to the SIM card using the SIM driver call SIM TerminalResponse.

#### Immediate link establishment fails

MMI                           SIM                          Card

|                             |                             |

|                             |        OPEN CHANNEL         |

|                             |         (immediate)         |

|                             *<============================*

|       SIM TOOLKIT IND       |                             |

|       (OPEN CHANNEL)        |                             |

*<============================*                             |

|                          (SIM 1)                          |

|       SIM TOOLKIT RES       |                             |

|     (TERMINAL RESONSE)      |                             |

*============================>*                             |

|                             |      TERMINAL RESONSE       |

|                             | (unable to process command) |

|                             *============================>*

|                          (SIM 2)                          |

|                             |                             |

(SIM 1)

SIM receives an OPEN CHANNEL command. The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. The command is used to open a channel for bearer independent protocol.

(SIM 2)

MMI sends a TERMINAL RESPONSE message because it is not able to establish the link. The content of the primitive is forwarded to the SIM card using the SIM driver call SIM TerminalResponse.

#### Link establishment on demand

MMI                           SIM                          Card  DTI peer

|                             |                             |      |

|                             |        OPEN CHANNEL         |      |

|                             |         (on demand)         |      |

|                             *<============================*      |

|       SIM TOOLKIT IND       |                             |      |

|       (OPEN CHANNEL)        |                             |      |

*<============================*                             |      |

|                          (SIM 1)                          |      |

|         SIM_DTI_REQ         |                             |      |

|    (channel identifier)     |                             |      |

*============================>*                             |      |

|         SIM_DTI_CNF         |                             |      |

*<============================*                             |      |

|                          (SIM 2)                          |      |

|       SIM TOOLKIT RES       |                             |      |

|     (TERMINAL RESONSE)      |                             |      |

*============================>*                             |      |

|                             |      TERMINAL RESONSE       |      |

|                             *============================>*      |

|                          (SIM 3)                          |      |

|                             |          SEND DATA          |      |

|                             |           (store)           |      |

|                             *<============================*      |

|                             |      TERMINAL RESONSE       |      |

|                             *============================>*      |

|                             |          SEND DATA          |      |

|                             |           (store)           |      |

|                             *<============================*      |

|                             |      TERMINAL RESONSE       |      |

|                             *============================>*      |

|                          (SIM 4)                          |      |

|                             |          SEND DATA          |      |

|                             |         (immediate)         |      |

|                             *<============================*      |

|       SIM TOOLKIT IND       |                             |      |

|         (SEND DATA)         |                             |      |

*<============================*                             |      |

|                          (SIM 5)                          |      |

|         SIM_DTI_REQ         |                             |      |

|     (DTI configuration)     |                             |      |

*============================>*                             |      |

|         SIM_DTI_CNF         |                             |      |

*<============================*                             |      |

|                          (SIM 6)                          |      |

|                             |           DTI_READY_IND            |

|                             *<===================================*

|                             |            DTI_DATA_REQ            |

|                             *===================================>*

|                             |      TERMINAL RESONSE       |      |

|                             *============================>*      |

|                          (SIM 7)                          |      |

|                             |                             |      |

(SIM 1)

SIM receives an OPEN CHANNEL command. The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. The command is used to open a channel for bearer independent protocol.

(SIM 2)

MMI requests a bearer independent protocol channel (bit SIM_BIP_OPEN_CHANNEL is set in dti_conn parameter). It provides the BIP channel identifier. SIM opens the BIP channel. After that SIM sends a confirm primitive to MMI.

(SIM 3)

MMI sends a TERMINAL RESPONSE message. The content of the primitive is forwarded to the SIM card using the SIM driver call SIM TerminalResponse.

(SIM 4)

The SIM card gives data to send to the SIM entity. The SIM entity stores the data and expects more data because of the "store" flag. The SIM entity informs about the number of remaining bytes in the send buffer by a SIM TerminalResponse call.

(SIM 5)

If SIM gets a SEND DATA command with a set "immediate" flag then it forwards this message to MMI to indicate the need of link establishment.

(SIM 6)

MMI requests a DTI connection (bit SIM_DTI_CONNECT is set in dti_conn parameter). It provides all necessary information. If a UDP connection is requested (con_type parameter is set to SIM_CON_TYPE_UDP) then MMI also provides UDP relevant information (local_ip, destination_ip and destination_port). SIM opens the DTI connection. After that SIM sends a confirm primitive to MMI.

(SIM 7)

SIM receives a DTI flow control primtive from the DTI peer. Now it is able to send the stored data to the DTI peer. After that SIM informs about a successful link establishment and data sending by a SIM TerminalResponse call.

#### Link establishment on demand fails

MMI                           SIM                          Card

|                             |                             |

|                             |        OPEN CHANNEL         |

|                             |         (on demand)         |

|                             *<============================*

|       SIM TOOLKIT IND       |                             |

|       (OPEN CHANNEL)        |                             |

*<============================*                             |

|                          (SIM 1)                          |

|         SIM_DTI_REQ         |                             |

|    (channel identifier)     |                             |

*============================>*                             |

|         SIM_DTI_CNF         |                             |

*<============================*                             |

|                          (SIM 2)                          |

|       SIM TOOLKIT RES       |                             |

|     (TERMINAL RESONSE)      |                             |

*============================>*                             |

|                             |      TERMINAL RESONSE       |

|                             *============================>*

|                          (SIM 3)                          |

|                             |          SEND DATA          |

|                             |           (store)           |

|                             *<============================*

|                             |      TERMINAL RESONSE       |

|                             *============================>*

|                             |          SEND DATA          |

|                             |           (store)           |

|                             *<============================*

|                             |      TERMINAL RESONSE       |

|                             *============================>*

|                          (SIM 4)                          |

|                             |          SEND DATA          |

|                             |         (immediate)         |

|                             *<============================*

|       SIM TOOLKIT IND       |                             |

|         (SEND DATA)         |                             |

*<============================*                             |

|                          (SIM 5)                          |

|         SIM_DTI_REQ         |                             |

|       (close channel)       |                             |

*============================>*                             |

|                             |      TERMINAL RESONSE       |

|                             |      (protocol error)       |

|                             *============================>*

|         SIM_DTI_CNF         |                             |

*<============================*                             |

|                          (SIM 6)                          |

|                             |                             |

(SIM 1)

SIM receives an OPEN CHANNEL command. The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. The command is used to open a channel for bearer independent protocol.

(SIM 2)

MMI requests a bearer independent protocol channel (bit SIM_BIP_OPEN_CHANNEL is set in dti_conn parameter). It provides the BIP channel identifier. SIM opens the BIP channel. After that SIM sends a confirm primitive to MMI.

(SIM 3)

MMI sends a TERMINAL RESPONSE message. The content of the primitive is forwarded to the SIM card using the SIM driver call SIM TerminalResponse.

(SIM 4)

The SIM card gives data to send to the SIM entity. The SIM entity stores the data and expects more data because of the "store" flag. The SIM entity informs about the number of remaining bytes in the send buffer by a SIM TerminalResponse call.

(SIM 5)

If SIM gets a SEND DATA command with a set "immediate" flag then it forwards this message to MMI to indicate the need of link establishment.

(SIM 6)

MMI is not able to establish the link. It requests a close of a bearer independent protocol channel (bit SIM_BIP_CLOSE_CHANNEL is set in dti_conn parameter). It provides the BIP channel identifier and the result codes. SIM closes the BIP channel and sends a Terminal Response message to the SIM card. After that SIM sends a confirm primitive to MMI.

### Close Channel

#### SIM card initiated

MMI                           SIM                          Card

|                             |                             |

|                             |        CLOSE CHANNEL        |

|                             *<============================*

|       SIM TOOLKIT IND       |                             |

|       (CLOSE CHANNEL)       |                             |

*<============================*                             |

|                          (SIM 1)                          |

|         SIM_DTI_REQ         |                             |

|       (diconnect DTI)       |                             |

|       (close channel)       |                             |

*============================>*                             |

|         SIM_DTI_CNF         |                             |

*<============================*                             |

|                          (SIM 2)                          |

|       SIM TOOLKIT RES       |                             |

|     (TERMINAL RESONSE)      |                             |

*============================>*                             |

|                             |      TERMINAL RESONSE       |

|                             *============================>*

|                          (SIM 3)                          |

|                             |                             |

(SIM 1)

SIM receives a CLOSE CHANNEL command. The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. The command is used to close a channel for bearer independent protocol.

(SIM 2)

MMI requests a DTI disconnection and a close of a bearer independent protocol channel (bits SIM_DTI_DISCONNECT and SIM_BIP_CLOSE_CHANNEL are set in dti_conn parameter). It provides the BIP channel identifier. SIM closes the DTI connection and the BIP channel. After that SIM sends a confirm primitive to MMI.

(SIM 3)

MMI sends a TERMINAL RESPONSE message. The content of the primitive is forwarded to the SIM card using the SIM driver call SIM TerminalResponse.

#### SIM entity initiated

The release timer is used to release the link if there is no data exchange on the link. The value of the release timer is given in the SIM_DTI_REQ primitive. The use of the release timer can be turned off.

MMI                           SIM                          Card

|                             |                             |

|                             *<==<release timer>           |

|         SIM_DTI_IND         |                             |

|       (diconnect DTI)       |                             |

|       (close channel)       |                             |

*<============================*                             |

|                          (SIM 1)                          |

|                             |        SEND DATA or         |

|                             |        RECEIVE DATA         |

|                             *<============================*

|                             |      TERMINAL RESONSE       |

|                             |      (channel closed)       |

|                             *============================>*

|                          (SIM 2)                          |

|                             |                             |

(SIM 1)

The release timer in SIM expirers and SIM closes the DTI connection and the bearer independent protocol (BIP) channel. After that SIM sends an indication primitive to MMI to inform about the closed DTI connection and the colsed BIP channel (bits SIM_DTI_DISCONNECT and SIM_BIP_CLOSE_CHANNEL are set in dti_conn parameter). It also provides the BIP channel identifier.

(SIM 2)

SIM receives a SEND DATA command or a RECEIVE DATA command from the SIM card. It answers with a TERMINAL RESPONSE message containing the information that the link is already closed an not usable any more.

#### MMI initiated

MMI                           SIM                          Card

|                             |                             |

|         SIM_DTI_REQ         |                             |

|       (diconnect DTI)       |                             |

|       (close channel)       |                             |

*============================>*                             |

|         SIM_DTI_CNF         |                             |

*<============================*                             |

|                          (SIM 1)                          |

|                             |        SEND DATA or         |

|                             |        RECEIVE DATA         |

|                             *<============================*

|                             |      TERMINAL RESONSE       |

|                             |      (channel closed)       |

|                             *============================>*

|                          (SIM 2)                          |

|                             |                             |

(SIM 1)

MMI requests a DTI disconnection and a close of a bearer independent protocol channel (bits SIM_DTI_DISCONNECT and SIM_BIP_CLOSE_CHANNEL are set in dti_conn parameter). It provides the BIP channel identifier. SIM closes the DTI connection and the BIP channel. After that SIM sends a confirm primitive to MMI.

(SIM 2)

SIM receives a SEND DATA command or a RECEIVE DATA command from the SIM card. It answers with a TERMINAL RESPONSE message containing the information that the link is already closed an not usable any more.

### Receive Data

DTI peer                       SIM                          Card

|                             |                             |

|       DTI_GETDATA_REQ       |                             |

*<============================*                             |

|                             *==><release timer>           |

(DTI 1)                         |                             |

|        DTI_DATA_IND         |                             |

*============================>*                             |

|                             *==I<release timer>           |

|                             |          ENVELOPE           |

|                             |      (Data available)       |

|                             *============================>*

|                          (SIM 1)                          |

|                             |        RECEIVE DATA         |

|                             |        (buffe size)         |

|                             *<============================*

|                             |      TERMINAL RESPONSE      |

|                             |       (Data, Remain)        |

|                             *============================>*

|                             |        RECEIVE DATA         |

|                             |        (buffe size)         |

|                             *<============================*

|                             |      TERMINAL RESPONSE      |

|                             |       (Data, Remain)        |

|                             *============================>*

|                          (SIM 2)                          |

|                             |        RECEIVE DATA         |

|                             |        (buffe size)         |

|                             *<============================*

|                             |      TERMINAL RESPONSE      |

|                             |      (Data, Remain=0)       |

|                             *============================>*

|       DTI_GETDATA_REQ       |                             |

*<============================*                             |

|                             *==><release timer>           |

|                          (SIM 3)                          |

|                             |                             |

(DTI 1)

SIM indicates to the DTI peer that it is ready to receive data. It starts the release timer to watch for the data exchange.

(SIM 1)

The DTI peer entity sends bearer independent protocol data to SIM.  SIM stops the release timer, because of the received data. If possible SIM informs the SIM application on the SIM card about available data by calling the driver function SIM Envelope.

(SIM 2)

The SIM card requests the received data and provides its internal buffer size by a RECEIVE DATA message. The received data is forwarded to the SIM card in a SIM TerminalResponse call. The number of remaining bytes in the reception buffer is also forwarded, if it is not possible to forward all data in this function call.

(SIM 3)

The SIM card requests the rest of the received data by a RECEIVE DATA message. The rest of the received data is forwarded to the SIM card in a SIM TerminalResponse call. The number of remaining bytes is 0 to indicate that the reption buffer is empty now. SIM indicates to the DTI peer that it is ready to receive data again.  It starts the release timer again to watch for the data exchange.

### Send Data

DTI peer                       SIM                          Card

|                             |                             |

|                             |          SEND DATA          |

|                             |        (Data, store)        |

|                             *<============================*

|                             |      TERMINAL RESPONSE      |

|                             |           (Space)           |

|                             *============================>*

|                             |          SEND DATA          |

|                             |        (Data, store)        |

|                             *<============================*

|                             |      TERMINAL RESPONSE      |

|                             |           (Space)           |

|                             *============================>*

|                             |          SEND DATA          |

|                             |      (Data, immediate)      |

|                             *<============================*

|                             *==><release timer>           |

|                          (SIM 1)                          |

|        DTI_READY_IND        |                             |

*============================>*                             |

|        DTI_DATA_REQ         |                             |

*<============================*                             |

|                             *==I<release timer>           |

|                             |      TERMINAL RESPONSE      |

|                             |     (Space=buffer size)     |

|                             *============================>*

|                          (SIM 2)                          |

|                             |                             |

(SIM 1)

The SIM card gives data to send to the SIM entity by a SEND DATA message. The SIM entity stores the data and expects more data because of the "store" flag. The SIM entity informs about the number of bytes of empty space in the send buffer by a SIM TerminalResponse call. If the "immediate" flag is set in a SEND DATA message then the SIM entity stores the data, but does not expect more data. SIM does not call SIM TerminalResponse until the stored data is sent. It starts the release timer to watch for the data exchange.

(SIM 2)

The DTI peer entity indicates to SIM that it is ready to receive data. SIM stops the release timer, forwards the whole send buffer content to the DTI peer entity and calls SIM TerminalResponse to indicate a successful send operation and an empty send buffer to the SIM card.

### Get Channel Status

MMI                           SIM                          Card

|                             |                             |

|                             |     GET CHANNEL STATUS      |

|                             *<============================*

|       SIM TOOLKIT IND       |                             |

|    (GET CHANNEL STATUS)     |                             |

*<============================*                             |

|                          (SIM 1)                          |

|       SIM TOOLKIT RES       |                             |

|     (TERMINAL RESONSE)      |                             |

*============================>*                             |

|                             |      TERMINAL RESONSE       |

|                             |      (CHANNEL STATUS)       |

|                             *============================>*

|                          (SIM 2)                          |

|                             |                             |

(SIM 1)

SIM receives a GET CHANNEL STATUS command. The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. The command is used to get the current channel status.

(SIM 2)

MMI sends a TERMINAL RESPONSE message. The content of the primitive is forwarded to the SIM card using the SIM driver call SIM TerminalResponse.

### Forward of Alpha identifier and Icon identifier

MMI                           SIM                          Card

|                             |                             |

|                             |        SEND DATA or         |

|                             |        RECEIVE DATA         |

|                             |  (alpha id and/or icon id)  |

|                             *<============================*

|       SIM TOOLKIT IND       |                             |

*<============================*                             |

|                          (SIM 1)                          |

|                             |                             |

(SIM 1)

If the SIM entity receives a SEND DATA message or a RECEIVE DATA message that contains an Alpha identifier or an Icon identifier then a duplicate of this message is forwarded to MMI.

### Set Up Event List

MMI                           SIM                          Card

|                             |                             |

|                             |      SET UP EVENT LIST      |

|                             *<============================*

|       SIM TOOLKIT IND       |                             |

|     (SET UP EVENT LIST)     |                             |

*<============================*                             |

|                          (SIM 1)                          |

|      SIM_EVENTLIST_REQ      |                             |

|     (enable or disable      |                             |

|    Data available event)    |                             |

*============================>*                             |

|      SIM_EVENTLIST_CNF      |                             |

*<============================*                             |

|                          (SIM 2)                          |

|       SIM TOOLKIT RES       |                             |

|     (TERMINAL RESONSE)      |                             |

*============================>*                             |

|                             |      TERMINAL RESONSE       |

|                             *============================>*

|                          (SIM 3)                          |

|                             |                             |

(SIM 1)

SIM receives a SET UP EVENT LIST command. The content of the pro-active command is forwarded to MMI with the SIM TOOLKIT IND primitive. The command requests notification of the SIM card on specific events in the ME.

(SIM 2)

MMI informs about a change of the status of the Data available event. SIM stores the new status and sends a confirm primitive to MMI.

(SIM 3)

MMI sends a TERMINAL RESPONSE message. The content of the primitive is forwarded to the SIM card using the SIM driver call SIM TerminalResponse.

### Suspend and Resume channel

The release timer is used to release the link if there is no data exchange on the link. The value of the release timer is given in the SIM_DTI_REQ primitive. The use of the release timer can be turned off. MMI informs SIM if the link is suspended. In this case SIM stops the release timer.

MMI                           SIM                          Card

|                             |                             |

|     SIM_BIP_SUSPEND_REQ     |                             |

|     (suspend or resume      |                             |

|        BIP channel)         |                             |

*============================>*                             |

|                             *<start or stop release timer>|

|                          (SIM 2)                          |

|                             |                             |

(SIM 1)

MMI informs about a change of the status of a bearer independent protocol (BIP) channel. SIM reacts with a start or stop of the release timer.

# SIM Driver Simulation

The SIM driver simulation replaces the SIM driver in the Windows-Environment. Using the principle of dynamic configuration a lot of modes are defined to have a specific behaviour of the SIM driver / SIM card to test all procedures of the SIM application.

A test mode is configured using the dynamic configuration for the SIM:

SIM CONFIG MODE=n

The parameter n indicates the test mode.

The SIM driver simulation is located in the module sim_csf.c. This object will be linked in the windows environment to the executable and replaces the SIM driver of the target version. The SIM driver simulation is not linked to the target version

## SIM Test modes

The following table show the different test modes. Default values are:

- PIN enabled

- Phase 2 card

- Emergency Call Codes available

- Preferred Languages available

- SIM Service Table Phase 2

- IMSI and Location Information validated

- No SIM toolkit capability

| No | Test Mode | Remark |
| --- | --- | --- |
| 0 | PIN disabled | Default mode |
| 1 |  |  |
| 2 | Blocked SIM Card |  |
| 3 |  |  |
| 4 | PIN disabled, Phase 1, no Emergency Call Codes, no Preferred Languages, SIM Service Table Phase 1 |  |
| 5 | Phase 1, no Emergency Call Codes, no Preferred Languages, SIM Service Table Phase 1 |  |
| 6 | Phase 1, no Emergency Call Codes, no Preferred Languages, SIM Service Table Phase 1 |  |
| 7 |  |  |
| 8 | PIN disabled, Phase 2+, SIM Service Table Phase 2+ |  |
| 9 | Phase 2+, SIM Service Table Phase 2+ |  |
| 10 | IMSI/Location Information invalidated, SIM Service Table Phase 2 with FDN |  |
| 11 | IMSI/Location Information invalidated |  |
| 12 | IMSI/Location Information invalidated, SIM Service Table Phase 2 with FDN |  |

| No | Test Mode | Remark |
| --- | --- | --- |
| 14 | IMSI/Location Information invalidated, Phase 2+, SIM Service Table Phase 2 with FDN |  |
| 15 | IMSI/Location Information invalidated, Phase 2+, SIM Service Table Phase 2 with FDN and BDN |  |
| 16 | IMSI/Location Information invalidated, Phase 2+ |  |
| 17 | IMSI/Location Information invalidated, Phase 2+, SIM Service Table Phase 2 with ADN and BDN |  |
| 18 | IMSI/Location Information invalidated, Phase 2+, SIM Service Table Phase 2 with FDN |  |
| 19 | IMSI/Location Information invalidated, Phase 2+, SIM Service Table Phase 2 with FDN and BDN |  |
| 20 | IMSI/Location Information invalidated, Phase 2+, SIM Service Table Phase 2 with FDN and BDN |  |
| 21 | IMSI/Location Information invalidated, Phase 2+, SIM Service Table Phase 2 with FDN and BDN |  |
| 22 | IMSI/Location Information invalidated, Phase 2+, SIM Service Table Phase 2 with FDN, rehabilitation fails |  |
| 23 | IMSI/Location Information invalidated, Phase 2+, SIM Service Table Phase 2 with FDN and BDN, rehabilitation fails |  |
| 24 | PIN disabled, SIM Service Table Phase 2 with ADN and FDN |  |
| 25 | Negative status request |  |
| 26 | PIN disabled, Phase 2+, SIM Service Table Phase 2+, SIM-Toolkit enabled |  |
| 27 | Pro-active Command Display text (short) | Only with SIM-Toolkit |
| 28 | Pro-active Command Display text (long) | Only with SIM-Toolkit |
| 29 | Pro-active Command Get Inkey | Only with SIM-Toolkit |
| 30 | Pro-active Command Get Input | Only with SIM-Toolkit |
| 31 | Pro-active Command More Time, TerminalResponse with OK expected | Only with SIM-Toolkit |
| 32 | Pro-active Command Play Tone | Only with SIM-Toolkit |
| 33 | Pro-active Command Poll Interval TerminalResponse with OK and duration expected | Only with SIM-Toolkit |
| 34 | Pro-active Command Polling Off | Only with SIM-Toolkit |
| 35 | Pro-active Command Refresh | Only with SIM-Toolkit |

| No | Test Mode | Remark |
| --- | --- | --- |
| 36 | Pro-active Command Set Up Menu | Only with SIM-Toolkit |
| 37 | Pro-active Command Select Item | Only with SIM-Toolkit |
| 38 | Pro-active Command Send SMS | Only with SIM-Toolkit |
| 39 | Pro-active Command Send SS | Only with SIM-Toolkit |
| 40 | Pro-active Command Set Up Call | Only with SIM-Toolkit |
| 41 | Pro-active Command Provide Local Information (location information), TerminalResponse with OK and location information expected | Only with SIM-Toolkit |
| 42 | Pro-active Command Provide Local Information (location information), TerminalResponse with negative result for location information expected | Only with SIM-Toolkit |
| 43 | Pro-active Command Provide Local Information (IMEI), TerminalResponse with OK and IMEI expected | Only with SIM-Toolkit |
| 44 | Pro-active Command Provide Local Information (not supported), TerminalResponse with negative result no capability expected | Only with SIM-Toolkit |
| 45 | Blocked SIM card, no unblock attempts |  |
| 46 | Response data for Envelope Command | Only with SIM-Toolkit |
| 47 | Pro-active Command Display Text (short), TerminalResponse with OK expected | Only with SIM-Toolkit |

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |