Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM General Packet Radio Services

Message Sequence Charts

SM

(Edit via File/Properties/Summary!)

| Document Number: | 8441.205.99.006 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.7 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1999-Apr-20 |
| Last changed: | 2004-Oct-21 by HK |
| File Name: | Sm.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1999-Apr-20 | Harald Krug |  | 0.1 |  | 1 |
| 2000-Jun-02 | Harald Krug |  | 0.2 |  | 2 |
| 2000-Jun-20 | Harald Krug |  | 0.3 |  | 3 |
| 2000-Sep-13 | Harald Krug |  | 0.4 |  | 4 |
| 2001-Jan-04 | Harald Krug |  | 0.5 |  | 5 |
| 2001-Sep-18 | Harald Krug |  | 0.6 |  | 6 |
| 2003-Jun-11 | TI Employee |  | 0.7 | Draft |  |
| 2004-Oct-21 | Harald Krug |  | 0.8 |  |  |

Notes:

- Initial version

- S3

- Added param ‘apn’ to REQ_PDP_ACT_REJ

- Cleaned chapter 2.

- Add reaction to SMREG_PDP_ACTIVATE_REJ_RES.

- Review, skip AA.

Table of Contents

1.1	References	6

1.2	Abbreviations	8

1.3	Terms	11

GRR (RLC/MAC) – Radio Link Control/Medium Access Control	13

LLC – Logical Link Control	13

GMM – GPRS Mobility Management	14

SM – Session Management	14

SNDCP - Subnetwork Dependant Convergence Protocol	14

GACI – GPRS Application Control Interface	14

USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	14

TOM – Tunnelling of Messages	14

Flow Control	15

Identified PDP context activation	16

PDP context activation, MS initiated, requested NSAPI not yet used	16

PDP context activation, MS initiated, requested NSAPI already used	16

PDP context activation, MS initiated, new attach accepted by the network or MS already attached	17

PDP context activation, receiving ACTIVATE_PDP_CONTEXT_ACCEPT	17

PDP context activation, NSAPI now in use	18

Unsuccessful PDP context activation, MS initiated	18

PDP context activation, network initiated	18

PDP context activation, network initiated, ACI accepts	19

PDP context activation, timer expiring up to four times	19

PDP context activation, timer expiring for the fifth time	19

Collision of MS iniated and network requested PDP context activation, comparable	20

Receiving activation reject messages in state PDP_ACTIVE_PENDING	20

Receiving activation reject messages in state other than PDP_ACTIVE_PENDING	21

PDP context activation, network initiated, ACI does not accept	21

Collision of MS iniated and network requested PDP context activation, not comparable	21

Anonymous PDP context activation	21

PDP context deactivation	22

PDP context deactivation, MS initiated	22

PDP context deactivation, MS initiated, network accepts	22

PDP context deactivation, MS initiated, NSAPI released	23

PDP context deactivation, network initiated	23

PDP context deactivation, network initiated, NSAPI released	24

PDP context deactivation, timer expiring up to four times	24

PDP context deactivation, timer expiring for the fifth time	24

PDP context deactivation, collision of deactivation requests	25

Anonymous PDP context deactivation	25

PDP context modification	25

PDP context modification initiated by the network, MS accepts QoS and SAPI	25

PDP context modification initiated by the network, MS does not accept QoS and SAPI	26

PDP context modification, network initiated, SNDCP finished requested changes	26

Handling of unknown, unforeseen and erroneous protocol data	27

Message too short	27

Message other than SM_STATUS with TI value “111”	27

Invalid transaction identifier value	27

REQUEST_PDP_CONTEXT_ACTIVATION message with TI flag set to “1”	28

Unknown message type	28

Unforeseen message type	28

Non-semantical mandatory information element errors, not in list 4.08, 8.5	29

Non-semantical mandatory information element errors, DEACTIVATE_PDP_CONTEXT_REQUEST	29

Non-semantical mandatory information element errors, REQUEST PDP CONTEXT ACTIVATION	30

Non-semantical mandatory information element errors, ACTIVATE PDP CONTEXT REQUEST	30

Detach	31

MS intiated detach, GPRS only	31

Network intiated detach, GPRS only	31

A.	Acronyms	32

B.	Glossary	32

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 05.02 version 8.0.0 Release 1999
Digital cellular telecommunications system (Phase 2+);
Multiplexing and multiple access on the radio path

[2]	GSM 04.60 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station (MS) - Base Station System (BSS) interface;
Radio Link Control/ Medium Access Control (RLC/MAC) protocol

[3]	GSM 04.08 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface layer 3 specification

[4]	GSM 03.64 version 6.1.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Overall description of the GPRS radio interface; Stage 2

[5]	GSM 03.60 version 6.3.1 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Service description; Stage 2

[6]	GSM 04.07 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Mobile radio interface signalling layer 3; General aspects

[7]	GSM 04.64 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
General Packet Radio Service (GPRS);
Mobile Station - Serving GPRS Support Node (MS-SGSN)
Logical Link Control (LLC) layer specification

[8]	GSM 05.08 version 6.4.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem link control

[9]	GSM 05.10 version 6.3.0 Release 1997
Digital cellular telecommunications system (Phase 2+);
Radio subsystem synchronization

[10]	GSM 03.20 TS 100 929: July 1998 (GSM 03.20 version 6.0.1)
Security related network functions, ETSI

[11]	Draft GSM 03.22: August 1998 (GSM 03.22 version 6.1.0)
Functions related to Mobile Station (MS) in idle mode and group receive mode, ETSI

[12]	GSM 04.65 V6.3.0: Subnetwork Dependant Convergence Protocol

ETSI, March 1999

[13]	ITU-T V42bis ITU-T, Recommendation V.42 bis 1990

[14]	GSM 09.60 GPRS Tunneling Protocol (GTP) across the Gn and Gp Interface

[15]	RFC 1661 IETF STD 51 July 1994
The Point-to-Point Protocol (PPP)

[16]	RFC 1662 IETF STD 51 July 1994
PPP in HDLC-like Framing

[17]	RFC 1570 January 1994
PPP LCP Extensions

[18]	RFC 1989 August 1996
PPP Link Quality Monitoring

[19]	RFC 1332 May 1992
The PPP Internet Protocol Control Protocol (IPCP)

[20]	RFC 1877 December 1995
PPP IPCP Extensions for Name Server Addresses

[21]	RFC 2153 May 1997
PPP Vendor Extensions

[22]	RFC 1334 October 1992
PPP Authentication Protocols (for Password Authentication Protocol only)

[23]	RFC 1994 August 1996
PPP Challenge Handshake Authentication Protocol (CHAP)

[24]	TIA/EIA-136-370
Packet-Data Services – Enhanced General Packet Radio for TIA/EIA-136 (EGPRS-136) - Overview, Telecommunications Industry Association

[25]	TIA/EIA-136-376
Packet-Data Services – EGPRS-136 Mobility Management, Telecommunications Industry Association

[26]	TIA/EIA-136-972
Packet-Data Services – Stage 2 Description, Telecommunications Industry Association

## Abbreviations

ACI	Application Control Interface
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CCI	Compression and Ciphering Interface
CHAP	Challenge Handshake Authentication Protocol
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DCOMP	Identifier of the user data compression algorithm used for the N-DPU
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

E	Extension bit
EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GACI	GPRS Application Control Interface
GMM	GPRS Mobility Management
GP	Guard Period
GRR	GPRS RR
GSM	Global System for Mobile Communication

HDLC	High-level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
IP	Internet Protocol
IPCP	Internet Protocol Control Protocol 
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Ciphering Key

L	Length Indicator
LAI	Location Area Information
LCP 	Link Control Protocol 
LISR	Low level Interrupt Service Routine
LLC	Logical Link Control
LPD	Link Protocol Discriminator
LQM	Link Quality Monitoring

M	More bit used to indicate the last segment of N-DPU
MAC	Medium Access Control
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MT	Mobile Termination

N(R)	Receive Number
N(S)	Send Number
NC	Network Control
NCC	National Colour Code
NCP 	Network Control Protocol
NECI	New Establishment Causes included
N-PDU	Network Protocol Data Unit
NSAPI	Network Layer Service Access Point Identifier

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PACCH	Packet Associated Control Channel
PAP	Password Authentication Protocol
PBCCH	Packet BCCH
PCCCH	Packet CCCH
PCOMP	Identifier of the protocol control information compression algorithm used for the N-DPU
PDCH	Packet Data Channel
PDP	Packet Data Protocol e.g. IP or X.25
PDTCH	Packet Data Traffic Channel
PRACH	Packet RACH
PSI	Packet System Information
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Data Unit
PL	Physical Layer
PLMN	Public Land Mobile Network
PPC	Packet Physical Convergence
PPP 	Point-to-Point Protocol
PTP	Point to Point

QoS	Quality of Service

RACH	Random Access Channel
REJ	Reject Frame
RLC	Radio Link Control
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Stand alone Dedicated Control Channel
SDU	Service Data Unit
SGSN	Serving GPRS Support Node
SIM	Subscriber Identity Module
SM	Session Management
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SNDCP	Subnetwork Dependant Convergence Protocol
SNSM	SNDCP-SM
SS	Supplementary Services

TAP	Test Application Program
TBF	Temporary Block Flow
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TCP	Transmission Control Protocol
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TFI	Temporary Flow Identifier
TLLI	Temporary Logical Link Identifier
TMSI	Temporary Mobile Subscriber Identity
TOM	Tunnelling of Messages
TQI	Temporary Queuing Identifier

UA	Unnumbered Acknowledgement Frame
UART	Universal Asynchronous Receiver Transmitter
UI	Unnumbered Information Frame
USF	Uplink State Flag

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visited Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for fax and data transmission consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity. The entity, which is described in this document, is coloured grey in the following figure :

Figure 0-1: Architecture of the GSM/GPRS protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the GPRS protocol stack are:

## GRR (RLC/MAC) – Radio Link Control/Medium Access Control

This layer contains two functions: The Radio Link Control function provides a radio-solution-dependent reliable link. The Medium Access Control function controls the access signalling (request and grant) procedures for the radio channel, and the mapping of LLC frames onto the GSM physical channel.

## LLC – Logical Link Control

The LLC entity provides multiple highly reliable logical links for asynchronous data transfer between the MS and the network. It supports variable-length information frames, acknowledged and unacknowledged data transfer, flow and sequence control, error detection and recovery, notification of unrecoverable errors, user identity confidentiality, and ciphering of user and signaling data.

## GMM – GPRS Mobility Management

The GMM entity provides procedures for the mobility of the MS, such as informing the network of its present location, and user identity confidentiality. It manages the GMM context (attach, detach, routing area updating), supports security functions such as authentication of user and MS, controls ciphering of data, and initiates the response to paging messages.

## SM – Session Management

The main function of the session management (SM) is to support PDP context handling of the user terminal. Session Management activates, modifies and deletes the contexts for packet data protocols (PDP). Session Management services are provided at the SMREG-SAP and the SNSM-SAP for anonymous and non-anonymous access. The non-anonymous and anonymous access procedures for PDP context activation and PDP context deactivation are available at the SMREG-SAP. In addition there exists a PDP context modification for non-anonymous PDP contexts.

## SNDCP - Subnetwork Dependant Convergence Protocol

- SNDCP carries out all functions related to transfer of Network layer Protocol Data Units (N-PDUs) over GPRS in a transparent way. SNDCP helps to improve channel efficiency by means of compression techniques. The set of protocol entities above SNDCP consists of commonly used network protocols. They all use the same SNDCP entity, which then performs multiplexing of data coming from different sources to be sent using the service provided by the LLC layer.

## GACI – GPRS Application Control Interface

The GACI is the GPRS extension of the ACI. It is specified in GSM 07.07 and 07.60. It is responsible for processing of the GPRS related AT Commands to setup, activate and deactivate the PDP context parameter. It also provides functionality for the interworking between GMM/SM/SNDCP and a packet oriented protocol like PPP.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the GPRS. The driver has to be able to perform flow control.

## TOM – Tunnelling of Messages

The TOM entity is present if and only if HS136 is supported (the feature flag FF_HS136 is enabled).

The main function of TOM is to tunnel non-GSM signalling messages between the MS and the SGSN. The only non-GSM signalling which is currently supported by TOM is for the EGPRS-136 system (according to TIA/EIA-136-376). Data transfer in both uplink and downlink direction is possible. Two different priorities (high, low) of signalling data transfer are supported. TOM uses the unacknowledged mode of LLC and the acknowledged mode of GRR (RLC/MAC).

# Introduction

[GSM 04.65, 7]

## Flow Control

Between SM and GMM no flow control has been established.

# Protocol

## Identified PDP context activation

<R.SM.SERVICES.A.001>

### PDP context activation, MS initiated, requested NSAPI not yet used

ACI                         SM                          GMM

|                           |                           |

| SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

|                         (SM 1)                        |

|                           | GMMSM_ESTABLISH_REQ       |

|                           *==========================>*

|                           |                         (GMM 1)

|                           |                           |

(SM 1)

SM is in state PDP_INACTIVE <R.SM.PDP_INAC.M.001>, <R.SM.MS_CONT.M.001>. The requested NSAPI is not yet used <R.SM.MS_ACT_S.M.005>. ACI sends an SMREG_PDP_ACTIVATE_REQ to initiate a PDP context activation.

(GMM 1)

SM starts timer T3380 <R.SM.MS_ACT_S.M.003>. SM sends a GMMSM_ESTABLISH_REQ to force a setup of GMM connection <R.SM.MS_CONT.M.002>and enters the state AWAIT_GMMSM_ESTABLISH_CNF. (If the MS has already been attached GMM will just send back a confirmation.)

### PDP context activation, MS initiated, All the transaction identifiers are in use

ACI                         SM                          GMM

|                           |                           |

| SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

|                         (SM 1)                        |

|                           |                           |

| SMREG_PDP_ACTIVATE_REJ    |                           |

|  (SMREG_RC_INSUF_RES)     |                           |

*<==========================*                           |

(ACI 1)                       |                           |

|                           |                           |

(SM 1)

SM is in state PDP_INACTIVE <R.SM.PDP_INAC.M.001>, <R.SM.MS_CONT.M.001>. The requested NSAPI is not yet used <R.SM.MS_ACT_S.M.005>. ACI sends an SMREG_PDP_ACTIVATE_REQ to initiate a PDP context activation.

(ACI 1)

SM sends SMREG_PDP_ACTIVATE_REJ primitive with the cause RC_INSUF_RES to the ACI informing that all the TI’s are in use .

### PDP context activation, MS initiated, requested NSAPI already used

ACI                         SM                          GMM

|                           |                           |

| SMREG_PDP_ACTIVATE_REQ    |                           |

*==========================>*                           |

|                         (SM 1)                        |

| SMREG_PDP_ACTIVATE_REJ    |                           |

| (RC_NSAPI_ALREADY_USED)   |                           |

*<==========================*                           |

(ACI 1)                      |                           |

|                           |                           |

(SM 1)

SM receives an SMREG_PDP_ACTIVATE_REQ. The requested NSAPI is already in use.

(ACI 1)

SM sends an SMREG_PDP_ACTIVATE_REJ to GMM with cause RC_NSAPI_ALREADY_USED <R.SM.MS_ACT_S.M.005>.

### PDP context activation, MS initiated, new attach accepted by the network or MS already attached

ACI                         SM                          GMM

|                           |                           |

|                           | GMMSM_ESTABLISH_CNF       |

|                           *<==========================*

|                          (SM 1)                       |

|                           | GMMSM_UNITDATA_REQ        |

|                           | (ACTIVATE_PDP_CONTEXT_REQ)|

|                           *==========================>|

|                           |                         (GMM 1)

|                           |                           |

(SM 1)

SM is in state AWAIT_GMMSM_ESTABLISH_CNF, but has sent a GMMSM_ESTABLISH_REQ to initiate a new GMM connection. The MS has already been attached or the network has sent the ATTACH ACCEPT message to the MS, the indirect attach was successful. GMM sends a GMMSM_ESTABLISH_CNF <R.SM.MS_CONT.M.003>. Now session management can proceed with PDP context activation.

(GMM 1)

SM sends an ACTIVATE_PDP_CONTEXT_REQ message to activate the requested PDP context <R.SM.MS_ACT_S.M.001>, <R.SM.MS_CONT.M.004> and changes to state PDP_ACTIVE_PENDING <R.SM.PDP_A_P.M.001>, <R.SM.MS_ACT_S.M.002>. The message contains the selected NSAPI, PDP type and, if the MS requests a static address, the PDP address <R.SM.MS_ACT_S.M.004>.

### PDP context activation, receiving ACTIVATE_PDP_CONTEXT_ACCEPT

SNDCP                         SM                            GMM

|                             |                             |

|                             | GMMSM_UNITDATA_IND          |

|                             |(ACTIVATE_PDP_CONTEXT_ACCEPT)|

|                             *<============================*

|                            (SM 1)                         |

| SNSM_ACTIVATE_IND           |                             |

*<============================*                             |

(SNDCP 1)                      |                             |

|                             |                             |

(SM 1)

SM is in state PDP_ACTIVE_PENDING <R.SM.PDP_A_P.M.001>. SM receives an ACTIVATE_PDP_CONTEXT_ACCEPT message indicating that the MS indicated PDP context activation succeeded <R.SM.MS_ACT_S.M.009>, <R.SM.MS_CONT.M.009>.

(SNDCP 1)

SM stops timer T3380 <R.SM.MS_ACT_S.M.010>, <R.SM.MS_CONT.M.010>. SM changes to state PDP_ACTIVE_AWAIT_SNDCP. SM sends an SNSM_ACTIVATE_IND to SNDCP if the QOS are acceptable <R.SM.MS_CONT.M.011>.

### PDP context activation, If QOS is not acceptable

SNDCP                         SM                            GMM

|                             |                             |

|                             |   GMMSM_UNITDATA_IND        |

|                             |(ACTIVATE_PDP_CONTEXT_ACCEPT)|

|                             *<============================*

|                            (SM 1)                         |

|                             | GMMSM_UNITDATA_REQ          |

|                             |(DEACTIVATE_PDP_CONTEXT_REQ) |

|                             *============================>|

|                             |                         (GMM 1)

|                                                                              |                                                                              |

(SM1)

SM is in state PDP_ACTIVE_PENDING <R.SM.PDP_A_P.M.001>. SM receives an ACTIVATE_PDP_CONTEXT_ACCEPT message indicating that the MS indicated PDP context activation succeeded <R.SM.MS_ACT_S.M.009>, <R.SM.MS_CONT.M.009>.

(GMM 1)

SM Rejects the PDP context when QOS is not acceptable and sends DEACTIVATE_PDP_CONTEXT_REQ to inform GMM of the deactivation stops the Timer 3380 and starts Timer 3390 and enters into state PDP_INACTIVE_PENDING.

### PDP context activation, NSAPI now in use

ACI                          SNDCP                           SM

|                             |                             |

|                             | SNSM_ACTIVATE_RES           |

|                             *============================>*

|                             |                            (SM 1)

|               SMREG_PDP_ACTIVATE_CNF                      |

*<==========================================================*

(ACI 1)                        |                             |

|                             |                             |

(SM 1)

SM is in state PDP_ACTIVE_AWAIT_SNDCP and SNDCP has sent an SNSM_ACTIVATE_RES to inform the SM entity that the indicated NSAPI is now in use and that the acknowledged peer-to-peer LLC operation for the indicated SAPI is established, if necessary <R.SM.MS_CONT.M.012>.

(ACI 1)

SM sends an SMREG_PDP_ACTIVATE_CNF to ACI to confirm that the PDP context is active <R.SM.MS_CONT.M.013>.

### Unsuccessful PDP context activation, MS initiated

<R.SM.MS_ACT_U.M.004>

ACI                           SM                            GMM

|                             |                             |

|                             | GMMSM_UNITDATA_IND          |

|                             |(ACTIVATE_PDP_CONTEXT_REJ)   |

|                             *<============================*

|                           (SM 1)                          |

|                             |                             |

|SMREG_PDP_ACTIVATE_REJ       |                             |

*<============================*                             |

(ACI 1)                        |                             |

|                             |                             |

(SM 1)

SM is in state PDP_ACTIVE_PENDING <R.SM.PDP_A_P.M.001> and waiting for an ACTIVATE_PDP_CONTEXT_ACC message, but receives an ACTIVATE_PDP_CONTEXT_REJ instead <R.SM.N_ACT_U.M.004>.. It shall stop timer T3380 <R.SM.N_ACT_U.M.005> in turn informs ACI about the Deactivation by SMREG_PDP_ACTIVATE_REJ Message and enter state PDP_INACTIVE <R.SM.N_ACT_U.M.006>.

### PDP context activation, network initiated

ACI                              SM                               GMM

|                                |                                |

|                                |    GMMSM_UNITDATA_IND          |

|                                |(REQUEST_PDP_CONTEXT_ACTIVATION)|

|                                *<===============================*

|                              (SM 1)                             |

| SMREG_PDP_ACTIVATE_IND         |                                |

*<===============================*                                |

(ACI 1)                           |                                |

|                                |                                |

(SM 1)

SM is in state PDP_INACTIVE  <R.SM.PDP_INAC.M.001>. SM receives a REQUEST_PDP_CONTEXT_ACTIVATION message to be informed about a network initiated PDP context activation <R.SM.N_ACT_S.M.003>, <R.SM.NET_CONT.M.004>.

(ACI 1)

SM sends an SMREG_PDP_ACTIVATE_IND to inform ACI about the requested connection. <R.SM.NET_CONT.M.005>.

### PDP context activation, network initiated, ACI accepts

ACI                              SM                               GMM

|                                |                                |

| SMREG_PDP_ACTIVATE_REQ         |                                |

*===============================>*                                |

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                | (ACTIVATE_PDP_CONTEXT_REQ)     |

|                                *===============================>*

|                                |                              (GMM 1)

|                                |                                |

(SM 1)

SM has sent an SMREG_PDP_ACTIVATE_IND to ACI and receives an SMREG_PDP_ACTIVATE_REQ <R.SM.NET_CONT.M.006>.

(GMM 1)

SM starts timer3380 <R.SM.NET_CONT.M.007>. SM sends an ACTIVATE_PDP_CONTEXT_REQ message and changes to state PDP_ACTIVE_PENDING <R.SM.PDP_A_P.M.001>, <R.SM.NET_CONT.M.008>. The ACTIVATE_PDP_CONTEXT_REQ message sent by the MS in order to initiate the PDP context activation shall contain the PDP address requested by the network in the REQUEST_PDP_CONTEXT_ACTIVATION message <R.SM.N_ACT_S.M.007>. The rest of the activation procedure is the same for MS initiated and network initiated context activation. <R.SM.N_ACT_S.A.009>

### PDP context activation, timer expiring up to four times

ACI                         SM                          GMM

|                           |                           |

*<Timeout T3380>            |                           |

|                           | GMMSM_UNITDATA_REQ        |

|                           | (ACTIVATE_PDP_CONTEXT_REQ)|

|                           *==========================>|

|                           |                         (GMM 1)

|                           |                           |

(GMM 1)

SM is in state PDP_ACTIVE_PENDING. Instead of receiving the expected ACTIVATE_PDP_CONTEXT_ACCEPT message the timer T3380 expires (for a first, second, third or fourth time) <R.SM.ACT_ABNO.M.001>. Then the MS resends the ACTIVATE_PDP_CONTEXT_REQ message <R.SM.ACT_ABNO.M.002> and resets and restarts T3380 <R.SM.ACT_ABNO.M.003>.

### PDP context activation, timer expiring for the fifth time

ACI                         SM                          GMM

|                           |                           |

*<Timeout T3380>            |                           |

| SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

|                           |                           |

(GMM 1)

SM is in state PDP_ACTIVE_PENDING. Instead of receiving the expected ACTIVATE_PDP_CONTEXT_ACCEPT message the timer T3380 expires for the fifth time <R.SM.ACT_ABNO.M.004>. Then the MS releases all resources possibly allocated for this invocation <R.SM.ACT_ABNO.M.005> and aborts the procedure without re-attempting to activate the context <R.SM.ACT_ABNO.M.006>.

(ACI 1)

SM sends an SMREG_PDP_ACTIVATE_REJ with cause "Network failure". [HK: primitive not explicitely mentioned in GSM 4.08, 6.1.3.1.5, cause "Network failure" is just assumed here!!!].

### Collision of MS iniated and network requested PDP context activation, comparable

ACI                         SM                          GMM

|                           |                           |

|                           | GMMSM_UNITDATA_IND        |

|                           |(REQUEST_PDP_CONTEXT_ACTIVATION)|

|                           *<==========================*

|                          (SM 1)                       |

|                           |                           |

(SM 1)

SM is in state PDP_ACTIVE_PENDING after having sent an ACTIVATE_PDP_CONTEXT_REQ message, receives a REQUEST_PDP_CONTEXT_ACTIVATION message, both messages contain the same PDP address and the request from the MS has neither been accepted nor rejected. <R.SM.ACT_ABNO.M.013>. The MS initiated PDP context activation shall take precedence over the network requested PDP context activation <R.SM.ACT_ABNO.A.014>, i.e. the MS discards the received REQUEST_PDP_CONTEXT_ACTIVATION message <R.SM.ACT_ABNO.M.017> and waits for an ACTIVATE_PDP_CONTEXT_ACCEPT message <R.SM.ACT_ABNO.M.018>.

### Receiving activation reject messages in state PDP_ACTIVE_PENDING

ACI                         SM                          GMM

|                           |                           |

|                           | GMMSM_UNITDATA_IND        |

|                           |(ACTIVATE_PDP_CONTEXT_REJ) |

|                           *<==========================*

|                          (SM 1)                       |

|                           |                           |

|                           |                           |

| SMREG_PDP_ACTIVATE_REJ    |                           |

*<==========================*                           |

(ACI 1)                       |                           |

|                           |                           |

(SM 1)

SM is in state PDP_ACTIVE_PENDING and receives an ACTIVATE_PDP_CONTEXT_REJ <R.SM.MS_ACT_U.M.004>. The MS stops timer T3380 <R.SM.MS_ACT_U.M.005> and enters state PDP_INACTIVE <R.SM.MS_ACT_U.M.006>.

(ACI 1)

SM  in turn informs ACI about the rejection by sending SMREG_PDP_ACTIVATE_REJ  primitive to it.

### Receiving activation reject messages in state other than PDP_ACTIVE_PENDING

ACI                         SM                          GMM

|                           |                           |

|                           | GMMSM_UNITDATA_IND        |

|                           |(ACTIVATE_PDP_CONTEXT_REJ) |

|                           *<==========================*

|                          (SM 1)                       |

|                           |                           |

|                           |                           |

|                           | GMMSM_UNITDATA_REQ        |

|                           | (SM_STATUS)               |

|                           *==========================>*

|                           |                         (GMM1)

|                           |                           |

(SM 1)

SM is in state PDP_INACTIVE, PDP_ACTIVE or PDP_INACTIVE_PENDING and receives an ACTIVATE_PDP_CONTEXT_REJ.

(GMM 1)

The MS sends Status message with specified cause value.

### PDP context activation, network initiated, ACI does not accept

ACI                              SM                               GMM

|                                |                                |

| SMREG_PDP_ACTIVATE_REJ_RES     |                                |

*===============================>*                                |

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                | (REQ_PDP_ACT_REJ)              |

|                                *===============================>*

|                                |                              (GMM 1)

|                                |                                |

(SM 1)

GACI does not accept an MT activation indicated by SM.

(GMM 1)

SM sends a REQ_PDP_ACT_REJ message to reject the MT activation.

### Collision of MS initiated and network requested PDP context activation, not comparable

ACI                         SM                          GMM

|                           |                           |

|                           | GMMSM_UNITDATA_IND        |

|                           |(REQUEST_PDP_CONTEXT_ACTIVATION)|

|                           *<==========================*

|                          (SM 1)                       |

|                           | GMMSM_UNITDATA_REQ        |

|                           |(REQUEST_PDP_CONTEXT_ACTIVATION_REJECT)|

|                           *==========================>|

|                           |                         (GMM 1)

|                           |                           |

(SM 1)

SM is in state PDP_ACTIVE_PENDING after having sent an ACTIVATE_PDP_CONTEXT_REQ message, receives a REQUEST_PDP_CONTEXT_ACTIVATION message, the request from the MS has neither been accepted nor rejected. <R.SM.ACT_ABNO.M.013>. The address and APN in the REQ_PDP_ACT and ACTIVATE_PDP_REQ message are not comparable.

(GMM 1)

SM rejects with cause “Insufficient Resources”.

### GMM Sends Receive N-PDU number in the form of SEQUENCE primitive

SNDCP                         SM                            GMM

|                             |                             |

|                             |                             |

|                             | GMMSM_SEQUENCE_IND        (GMM1)

|                             *<============================*

|                            (SM 1)                         |

|   SNSM_SEQUENCE_IND         |                             |

*<============================*                             |

(SNDCP 1)                      |                             |

|                             |                             |

| SNSM_SEQUENCE_RES           |                             |

*============================>*                             |

|                             | GMMSM_SEQUENCE_RES          |

|                                                                              *====================================>*

|                                                                             (SM 2)                                                                    (GMM 2)

(GMM 1)

Network sends the Receive N-PDU number in the form of GMMSM_SEQUENCE_IND to SM.

(SM 1)

SM will forward this message in a primitive to SNDCP.

(SNDCP 1)

SNDCP after receiving the Receive N-PDU number from N/w puts the current N-PDU number of MS SNDCP’s in SNSM_SEQUENCE_RES primitive to SM.

(SM 2)

SM will in turn sends Receive N-PDU number of SNDCP to GMM through GMMSM_SEQUENCE_RES.

### SNDCP sends STATUS primitive to SM to Deactivate particular PDP Context

SNDCP                         SM                            GMM

|                             |                             |

|   SNSM_STATUS_REQ          (SM 1)                         |

*============================>*                             |

|                             |                             |

|                             |  GMMSM_UNITDATA_REQ         |

|                             |   (DEACT_PDP_REQ)           |

|                                                                              *====================================>*

|                                                                              |                                                                           (GMM 1)

|                                                                              |                                                                              |

( SM 1)

SM is in PDP_ACTIVE  state and SNDCP sends SNSM_STATUS_REQ primitive to SM to Deactivate particular PDP Context.

(GMM 1)

SM deactivates particular contexts using the affected nsapi for both Acknowledge and Unacknowledged mode, Starts Timer 3390 and enters into the State PDP_INACTIVE_PENDING .

## PDP context deactivation

<R.SM.SERVICES.A.001>

### PDP context deactivation, MS initiated

ACI                              SM                               GMM

|                                |                                |

| SMREG_PDP_DEACTIVATE_REQ       |                                |

*===============================>*                                |

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                | (DEACTIVATE_PDP_CONTEXT_REQ)   |

|                                *===============================>*

|                                |                              (GMM 1)

|                                |                                |

(SM 1)

SM is in state PDP_ACTIVE <R.SM.PDP_ACT.M.001>. ACI sends an SMREG_PDP_DEACTIVATE_REQ to initiate the anonymous PDP context deactivation <R.SM.PDP_DE_M.M.001>.

(GMM 1)

SM starts timer T3390 <R.SM.MS_DEACT.M.003>, <R.SM.PDP_DE_M.M.002>. SM enters state PDP_INACTIVE_PENDING <R.SM.PDP_I_P.M.001><R.SM.MS_DEACT.M.002> and sends a DEACTIVATE_PDP_CONTEXT_REQ message <R.SM.MS_DEACT.M.001>, <R.SM.PDP_DE_M.M.003> containing the transaction identifier in use for the PDP context to be deactivated and a cause code that typically indicates one of the following causes: #26 insufficient resources, #36 regular PDP context deactivation or #37 QoS not accepted.

### PDP context deactivation, MS initiated, network accepts

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                | (DEACTIVATE_PDP_CONTEXT_ACCEPT)|

|                                *<===============================*

|                               (SM 1)                            |

| SNSM_DEACTIVATE_IND            |                                |

*<===============================*                                |

(SNDCP 1)                         |                                |

|                                |                                |

(SM 1)

SM is in state PDP_INACTIVE_PENDING <R.SM.PDP_I_P.M.001>. The network sends a DEACTIVATE_PDP_CONTEXT_ACCEPT message to inform the MS that the deactivation request is accepted <R.SM.MS_DEACT.M.007>, <R.SM.PDP_DE_M.M.007>.

(SNDCP 1)

SM stops timer T3390 <R.SM.MS_DEACT.M.008>, <R.SM.PDP_DE_M.M.008>. SM enters state PDP_INACTIVE_WAIT_SNDCP. SM sends an SNSM_DEACTIVATE_IND to inform SNDCP that the NSAPI in question can be de-allocated <R.SM.PDP_DE_M.M.009>. (SNDCP will do so only if the LLC link in question is not used by any other PDP context using acknowledged mode <R.SM.MS_DEACT.M.009>.

### PDP context deactivation, MS initiated, NSAPI released

ACI               SNDCP                              SM

|                   |                                |

|                   | SNSM_DEACTIVATE_RES            |

|                   *===============================>*

|                   |                              (SM 1)

|      SMREG_PDP_DEACTIVATE_CNF                      |

*<===================================================*

(ACI 1)              |                                |

|                   |                                |

(SM 1)

SM is in state PDP_INACTIVE_WAIT_SNDCP. SNDCP sends an SNSM_DEACTIVATE_RES to inform SM that the NSAPI in question has been released <R.SM.PDP_DE_M.M.012>.

(ACI 1)

SM sends an SMREG_PDP_DEACTIVATE_CNF to ACI to confirm that the PDP context has been deactivated <R.SM.PDP_DE_M.M.013>.

### PDP context deactivation, network initiated

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |(DEACTIVATE_PDP_CONTEXT_REQ)    |

|                                *<===============================*

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                |(DEACTIVATE_PDP_CONTEXT_ACCEPT) |

|                                *===============================>*

|                                |                              (GMM 1)

| SNSM_DEACTIVATE_IND            |                                |

*<===============================*                                |

(SNDCP 1)                         |                                |

|                                |                                |

(SM 1)

SM is in state PDP_ACTIVE <R.SM.PDP_ACT.M.001>. The network sends a DEACTIVATE_PDP_CONTEXT_REQ message to initiate a PDP context deactivation <R.SM.N_DEACT.I.001>, <R.SM.N_DEACT.M.004>, <R.SM.PDP_DE_N.M.004>. The message contains the transaction identifier in use for the PDP context to be deactivated and a cause code that typically indicates one of the following causes: #36 regular PDP context deactivation, #38 network failure or #39 reactivation requested. .

(GMM 1)

SM enters state PDP_INACTIVE_WAIT_SNDCP_MT. SM answers with a DEACTIVATE_PDP_CONTEXT_ACCEPT message <R.SM.N_DEACT.M.005>, <R.SM.PDP_DE_N.M.005>.

(SNDCP 1)

SM sends an SNSM_DEACTIVATE_IND to SNDCP to inform the SNDCP entity that an NSAPI has been deallocated and cannot be used by the SNDCP entity anymore <R.SM.PDP_DE_N.M.006>. All buffered N-PDUs corresponding to this NSAPI are deleted. SNDCP will release the affected LLC link if it is not used by any other PDP context using acknowledged mode.

### PDP context deactivation, network initiated, NSAPI released

ACI               SNDCP                              SM

|                   |                                |

|                   | SNSM_DEACTIVATE_RES            |

|                   *===============================>*

|                   |                              (SM 1)

|      SMREG_PDP_DEACTIVATE_IND                      |

*<===================================================*

(ACI 1)              |                                |

|                   |                                |

(SM 1)

SM is in state PDP_INACTIVE_WAIT_SNDCP_MT. SNDCP sends an SNSM_DEACTIVATE_RES to inform SM that the NSAPI in question has been released <R.SM.PDP_DE_N.M.010>.

(ACI 1)

SM sends an SMREG_PDP_DEACTIVATE_IND to ACI to confirm that the PDP context has been deactivated <R.SM.PDP_DE_N.M.011> and enters the state PDP_INACTIVE.

### PDP context deactivation, timer expiring up to four times

ACI                         SM                          GMM

|                           |                           |

*<Timeout T3390>            |                           |

|                           | GMMSM_UNITDATA_REQ        |

|                           |(DEACTIVATE_PDP_CONTEXT_REQ)|

|                           *==========================>|

|                           |                         (GMM 1)

|                           |                           |

(GMM 1)

SM is in state PDP_INACTIVE_PENDING. Instead of receiving the expected DEACTIVATE_PDP_CONTEXT_ACCEPT message the timer T3390 expires (for a first, second, third or fourth time) <R.SM.DE_ABNO.M.001>. Then the MS resends the DEACTIVATE_PDP_CONTEXT_REQ message <R.SM.DE_ABNO.M.002> and resets and restarts T3390 <R.SM.DE_ABNO.M.003>.

### PDP context deactivation, timer expiring for the fifth time

SNDCP                       SM                          GMM

|                           |                           |

*<Timeout T3390>            |                           |

| SNSM_DEACTIVATE_IND       |                           |

*<==========================*                           |

(SNDCP)                      |                           |

|                           |                           |

(GMM 1)

SM is in state PDP_INACTIVE_PENDING. Instead of receiving the expected DEACTIVATE_PDP_CONTEXT_ACCEPT message the timer T3390 expires for the fifth time <R.SM.DE_ABNO.M.004>.

(SNDCP 1)

Then the MS releases all resources possibly allocated for this invocation <R.SM.DE_ABNO.M.005> and erases the PDP context related data <R.SM.DE_ABNO.M.006>. This is initiated by sending an SNSM_DEACTIVATE_IND and enters the state PDP_INACTIVE_WAIT_SNDCP.

### PDP context deactivation, collision of deactivation requests

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |(DEACTIVATE_PDP_CONTEXT_REQ)    |

|                                *<===============================*

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                |(DEACTIVATE_PDP_CONTEXT_ACCEPT) |

|                                *===============================>*

|                                |                              (GMM 1)

| SNSM_DEACTIVATE_IND            |                                |

*<===============================*                                |

(SNDCP 1)                         |                                |

|                                |                                |

(SM 1)

SM is in state PDP_INACTIVE_PENDING after having sent a DEACTIVATE_PDP_CONTEXT_REQ message. Instead of the expected DEACTIVATE_PDP_CONTEXT_ACCEPT a DEACTIVATE_PDP_CONTEXT_REQ is received (collision) <R.SM.DE_ABNO.A.012>.

(GMM 1)

SM replies with the message DECATIVATE_PDP_CONTEXT_ACCEPT <R.SM.DE_ABNO.M.013>. SM stops timer T3390 <R.SM.DE_ABNO.M.014>.

(SNDCP 1)

SM sends an SNSM_DEACTIVATE_IND to SNDCP to inform the SNDCP entity that an NSAPI has been deallocated and cannot be used by the SNDCP entity anymore. All buffered N-PDUs corresponding to this NSAPI are deleted.

## PDP context modification

<R.SM.SERVICES.A.001>

### PDP context modification initiated by the network, MS accepts QoS and SAPI

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |(MODIFY_PDP_CONTEXT_REQ)        |

|                                *<===============================*

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                |(MODIFY_PDP_CONTEXT_ACCEPT)     |

|                                *===============================>*

|                                |                              (GMM 1)

| SNSM_MODIFY_IND                |                                |

*<===============================*                                |

(SNDCP 1)                         |                                |

|                                |                                |

(SM 1)

SM is in state PDP_ACTIVE <R.SM.PDP_ACT.M.001>. The network sends a MODIFY_PDP_CONTEXT_REQ message to initiate a PDP context modification <R.SM.CONT_MOD.M.005>, <R.SM.CO_MOD.M.004>, <R.SM.RAU_INTE.M.006>.

(GMM 1)

The MS accepts the proposed new QoS an the indicated LLC SAPI <R.SM.CONT_MOD.M.006> (That means the requested new QoS corresponds with the “minimum QoS given by GACI during activation). SM answers with a MODIFY_PDP_CONTEXT_ACCEPT message <R.SM.CONT_MOD.M.006>, <R.SM.CONT_MOD.M.007>, <R.SM.CO_MOD.M.005>, <R.SM.RAU_INTE.M.007>.

(SNDCP 1)

SM sends an SNSM_MODIFY_IND to SNDCP to to trigger change of the QoS profile (see GSM 04.08) for an NSAPI and indication of the SAPI to be used <R.SM.CO_MOD.M.006>, <R.SM.RAU_INTE.M.008>. It is also used by the SM entity in the SGSN to inform the SNDCP entity that an NSAPI shall be created, together with the (re-)negotiated QoS profile, the SAPI assigned, and, in the MS, the radio priority level to be used by RLC/MAC.

### PDP context modification initiated by the network, MS does not accept QoS and SAPI

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |(MODIFY_PDP_CONTEXT_REQ)        |

|                                *<===============================*

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                |(DEACTIVATE_PDP_CONTEXT_REQ)    |

|                                *===============================>*

|                                |                              (GMM 1)

|                                |                                |

(SM 1)

SM is in state PDP_ACTIVE <R.SM.PDP_ACT.M.001>. The network sends a MODIFY_PDP_CONTEXT_REQ message to initiate a PDP context modification <R.SM.CONT_MOD.M.005>.

(GMM 1)

The MS does not accept the new QoS and the indicated LLC SAPI <R.SM.CONT_MOD.M.008> and initiates the PDP context deactivation procedure by sending a DEACTIVATE_PDP_CONTEXT_REQ message <R.SM.CONT_MOD.M.009> and starts the Timer 3390 and enters into PDP_INACTIVE_PENDING state.

### PDP context modification, network initiated, SNDCP finished requested changes

ACI               SNDCP                              SM

|                   |                                |

|                   | SNSM_MODIFY_RES                |

|                   *===============================>*

|                   |                              (SM 1)

|      SMREG_PDP_MODIFY_IND                          |

*<===================================================*

(ACI 1)              |                                |

|                   |                                |

(SM 1)

SM is in state PDP_ACTIVE <R.SM.PDP_INAC.M.001>. SNDCP sends an SNSM_DEACTIVATE_RES to inform SM that the NSAPI in question has been released <R.SM.CO_MOD.M.010>.

(ACI 1)

SM sends an SMREG_PDP_MODIFY_IND to ACI to confirm that the PDP context has been deactivated <R.SM.CO_MOD.M.011>.

## Handling of unknown, unforeseen and erroneous protocol data

### Message too short

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |(Any message)                   |

|                                *<===============================*

|                               (SM 1)                            |

|                                |                                |

(SM 1)

SM is in any state. It receives a message too short to contain a complete message type IE. <R.SM.MSG.A.001>. That message is ignored <R.SM.MSG.A.002>.

### Message other than SM_STATUS with TI value “111”

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |(message other than SM_STATUS TI=111)|

|                                *<===============================*

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                |(SM_STATUS TI=111)              |

|                                *===============================>*

|                                |                              (GMM 1)

|                                |                                |

(SM 1)

SM is in any state. It receives a message other than SM_STATUS and having a TI value “111” <R.SM.UN_TI.A.001>.

(GMM 1)

SM shall immediately send an SM_STATUS message with TI value “111”.

### Invalid transaction identifier value

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |(message other than ACTIVATE_PDP_CONTEXT_REQ

|                                | or SM_STATUS with invalid TI ) |

|                                *<===============================*

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                |(SM_STATUS cause # 81)          |

|                                *===============================>*

|                                |                              (GMM 1)

|                                |                                |

(SM 1)

A session management message except REQUEST_PDP_CONTEXT_ACTIVATION or SM_STATUS

is received by the MS specifying a transaction identifier which is not recognized as relating to an

active context or to a context that is in the process of activation or deactivation or has been [recently]

deactivated <R.SM.UN_TI.M.006>.

(GMM 1)

SM sends an SM_STATUS message with cause #81 “Invalid transaction identifier value” using the received transaction identifier value <R.SM.UN_TI.M.007> .

### REQUEST_PDP_CONTEXT_ACTIVATION message with TI flag set to “1”

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |(REQUEST_PDP_CONTEXT_ACTIVATION |

|                                |  TI flag = 1)                  |

|                                *<===============================*

|                               (SM 1)                            |

|                                |                                |

(SM 1)

a REQUEST_PDP_CONTEXT_ACTIVATION is received with transaction identifier set to “1” <R.SM.UN_TI.M.009>. This message is ignored <R.SM.UN_TI.M.010>.

### Unknown message type

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |(unknown message type)          |

|                                *<===============================*

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                |(SM_STATUS cause # 97)          |

|                                *===============================>*

|                                |                              (GMM 1)

|                                |                                |

(SM 1)

If a mobile station receives an SM message with message type not defined for the PD or not implemented by the receiver<R.SM.UN_MSG_T.M.001>, then

(GMM 1)

it shall return a status message (SM STATUS) with cause # 97 "message type non-existent or not implemented"<R.SM.UN_MSG_T.M.002>.

### Unforeseen message type

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |(unforeseen message type)       |

|                                *<===============================*

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                |(SM_STATUS cause # 98)          |

|                                *===============================>*

|                                |                              (GMM 1)

|                                |                                |

(SM 1)

If a mobile station receives an SM message with message type not compatible with the protocol state <R.SM.UN_MSG_T.M.006> and an RR connection exists <R.SM.UN_MSG_T.M.007>, then

(GMM 1)

it shall return a status message (SM STATUS) with cause # 98 " Message type not compatible with protocol state” <R.SM.UN_MSG_T.M.008>.

### Non-semantical mandatory information element errors, not in list 4.08, 8.5

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |( Non-semantical mandatory      |

|                                | information element errors,    |

|                                | not in list in 4.08, 8.5)      |

|                                *<===============================*

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                |(SM_STATUS cause # 96)          |

|                                *===============================>*

|                                |                              (GMM 1)

|                                |                                |

(SM 1)

The mobile station receives an SM message with Non-semantical mandatory information element errors and the message is not in the list in [GSM 04.65, 8.5] <R.SM.MAND_ERR.M.005> and an RR connection exists <R.SM.MAND_ERR.M.002>

(GMM 1)

The MS shall return a status message (SM-STATUS) with cause # 96 "invalid mandatory information"<R.SM.MAND_ERR.M.003>. If no RR connection exists the message is just ignored <R.SM.MAND_ERR.M.004>.

### Non-semantical mandatory information element errors, DEACTIVATE_PDP_CONTEXT_REQUEST

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_UNITDATA_IND             |

|                                |(DEACTIVATE_PDP_CONTEXT_REQUEST)|

|                                *<===============================*

|                               (SM 1)                            |

|                                | GMMSM_UNITDATA_REQ             |

|                                |(DEACTIVATE_PDP_CONTEXT_ACCEPT) |

|                                *===============================>*

|                                |                              (GMM 1)

|                                |                                |

(SM 1)

The mobile station receives an SM message with Non-semantical mandatory information element errors and the message is DEACTIVATE PDP CONTEXT REQUEST <R.SM.MAND_ERR.M.006>.

(GMM 1)

The MS shall return a DEACTIVATE PDP CONTEXT ACCEPT message. All resources allocated for that context shall be released.

### Non-semantical mandatory information element errors, REQUEST PDP CONTEXT ACTIVATION

SNDCP                            SM                                  GMM

|                                |                                   |

|                                | GMMSM_UNITDATA_IND                |

|                                |(REQUEST_PDP_CONTEXT_ACTIVATION)   |

|                                *<==================================*

|                               (SM 1)                               |

|                                | GMMSM_UNITDATA_REQ                |

|                                |( REQUEST_PDP_CONTEXTACTIVATION_REJECT)|

|                                *==================================>*

|                                |                                (GMM 1)

|                                |                                   |

(SM 1)

The mobile station receives an SM message with Non-semantical mandatory information element errors and the message is REQUEST_PDP_CONTEXT_ACTIVATION <R.SM.MAND_ERR.M.008>.

(GMM 1)

The MS shall return a REQUEST_PDP_CONTEXT_ACTIVATION_REJECT message with cause  # 96 "Invalid mandatory information".

### Non-semantical mandatory information element errors, ACTIVATE PDP CONTEXT REQUEST

SNDCP                            SM                                  GMM

|                                |                                   |

|                                | GMMSM_UNITDATA_IND                |

|                                |( ACTIVATE PDP CONTEXT REQUEST)    |

|                                *<==================================*

|                               (SM 1)                               |

|                                | GMMSM_UNITDATA_REQ                |

|                                |( ACTIVATE_PDP_CONTEXT_REJECT)      |

|                                *==================================>*

|                                |                                (GMM 1)

|                                |                                   |

(SM 1)

The mobile station receives an SM message with Non-semantical mandatory information element errors and the message is REQUEST_PDP_CONTEXT_ACTIVATION <R.SM.MAND_ERR.M.008>.

(GMM 1)

The MS shall return a REQUEST_PDP_CONTEXT_ACTIVATION_REJECT message with cause  # 96 "Invalid mandatory information".

## Detach

### MS intiated detach, GPRS only

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_RELEASE_IND              |

|                                *<===============================*

|                               (SM 1)                            |

| SNSM_DEACTIVATE_IND            |                                |

*<===============================*                                |

(SNDCP 1)                         |                                |

|                                |                                |

(SM 1)

SM receives a GMMSM_RELEASE_IND from GMM <R.SM.MS_DET.A.001>.

(SNDCP 1)

SM initiates the release of the logical link by sending an SNSM_DEACTIVATE_IND <R.SM.MS_DET.A.002>.

### Network intiated detach, GPRS only

SNDCP                            SM                               GMM

|                                |                                |

|                                | GMMSM_RELEASE_IND              |

|                                *<===============================*

|                               (SM 1)                            |

| SNSM_DEACTIVATE_IND            |                                |

*<===============================*                                |

(SNDCP 1)                         |                                |

|                                |                                |

(SM 1)

SM receives a GMMSM_RELEASE_IND from GMM <R.SM.NET_DET.A.001>.

(SNDCP 1)

SM initiates the release of the logical link by sending an SNSM_DEACTIVATE_IND <R.SM.NET_DET.A.002>.

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |