Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

GSM Protocol Stack

Message Sequence Charts

SMS

(Edit via File/Properties/Summary!)

| Document Number: | 6147.202.98.101 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.2 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | xx |
| Last changed: | 2006-Feb-13 by Girish Hegade (xhegadeg) |
| File Name: | Sms.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2003-Jun-16 | TI Employee |  | 0.1 | Draft | 1 |
| 2006-Jan-20 | xhegadeg |  | 0.2 | Draft | 2 |

Notes:

- Initial version

- Reading of Status report message stored in SIM or ME.

Table of Contents

1	General	4

1.1	References	4

1.2	Abbreviations	8

1.3	Terms	10

Overview	11

Introduction	12

Dynamic Configuration	12

Timer Modes	12

Initialisation	13

Initial Parameters from the SIM card	13

Reading SMS from SIM card	13

Reading SMS from Mobile Memory	15

Mobile Originated Short Message Service	16

Establishment Phase	16

Initiation by Man Machine Interface	16

Confirmation of SMS Connection	17

No SMS Connection established	17

Timeout TR1M	18

Multiple Request  by Man Machine Interface	18

Confirmation to CP DATA Message	19

Wrong Message received	19

CP Error received	19

Lower Layer Failure	21

Release of MM-Connection	21

Timeout TC1M	22

Timeout TR1M (SMS connection established)	23

Active Phase	24

Acknowledge of the Infrastructure	24

Error signalled by the Infrastructure	25

Wrong Message signalled by the Infrastructure (CP layer)	26

Wrong Message signalled by the Infrastructure (RL layer)	27

Timeout TR1M	28

Lower Layer Failure	29

Release of MM-Connection	29

Mobile Originated Short Message Command	30

Mobile Terminated Short Message Service	32

Reception of a Short Message	34

Reception of CP ERROR message	35

Reception of CP ACK or unknown message	35

Second Mobile Terminated Connection	36

Additional Message for  Mobile Terminated Connection	37

Display Message	38

Store Message in ME memory	39

No Storage available on ME memory, no other opportunities	41

Store Message in SIM memory	42

Storage on SIM available, writing successful	42

Storage on SIM available, writing not successful	44

No Storage on SIM available	45

No Storage available on SIM memory, no other opportunities	46

Replace Short Message	47

Replace in Mobile Station Memory	47

Mobile Terminated Short Message Status	48

SIM Toolkit Data Download	49

SMS - Depersonalisation	50

Short Message Service Procedures	51

Storing of Short Messages	51

Changing of Short Messages	52

Reading of Short Messages	53

Deleting of Short Messages	54

Data Formats	55

Mobile Terminated Short Message (Air-Interface)	55

Mobile Terminated Short Message (SIM)	56

Mobile Originated Short Message (Air-Interface)	57

Mobile Originated Short Message (SIM)	58

Appendices	59

A.	Acronyms	59

B.	Glossary	59

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

# General

## References

[1]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat GmbH, December 1996

[39]	Service Access Point MNCC
	6147.101.96.100; Condat GmbH, December 1996

[40]	Service Access Point MNSS
	6147.102.96.100; Condat GmbH, December 1996

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat GmbH, December 1996

[42]	Service Access Point MMCC
	6147.104.97.100; Condat GmbH

[43]	Service Access Point MMSS
	6147.105.97.100; Condat GmbH

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat GmbH

[45]	Service Access Point RR
	6147.107.97.100; Condat GmbH

[46]	Service Access Point SIM
	6147.108.97.100; Condat GmbH

[47]	Service Access Point MPH
	6147.109.96.100; Condat GmbH, December 1996

[48]	Service Access Point DL
	6147.110.96.100; Condat GmbH, December 1996

[49]	Service Access Point MDL
	6147.111.96.100; Condat GmbH, December 1996

[50]	Service Access Point PH
	6147.112.96.100; Condat GmbH, December 1996

[51]	Service Access Point MMI
	6147.113.96.100; Condat GmbH, December 1996

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat GmbH

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat GmbH

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat GmbH

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat GmbH

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat GmbH, December 1996

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat GmbH, December 1996

[58]	Users Guide
	6147.300.96.100; Condat GmbH, December 1996

[59]	Test Specification CC
	6147.400.97.100; Condat GmbH

[60]	Test Specification SS
	6147.401.97.100; Condat GmbH

[61]	Test Specification SMS
	6147.402.97.100; Condat GmbH

[62]	Test Specification MM
	6147.403.97.100; Condat GmbH

[63]	Test Specification RR
	6147.404.97.100; Condat GmbH

[64]	Test Specification DL
	6147.405.97.100; Condat GmbH

[65]	Test Specification CCD
	6147.406.97.100; Condat GmbH

[66]	SDL Specification CC
	6147.500.97.100; Condat GmbH

[67]	SDL Specification SS
	6147.501.97.100; Condat GmbH

[68]	SDL Specification SMS
	6147.502.97.100; Condat GmbH

[69]	SDL Specification MM
	6147.503.97.100; Condat GmbH

[70]	SDL Specification RR
	6147.504.97.100; Condat GmbH

[71]	SDL Specification DL
	6147.505.97.100; Condat GmbH

[72]	Message Specification CC
	6147.600.97.100; Condat GmbH

[73]	Message Specification SS
	6147.601.97.100; Condat GmbH

[74]	Message Specification SMS
	6147.602.97.100; Condat GmbH

[75]	Message Specification MM
	6147.603.97.100; Condat GmbH

[76]	Message Specification RR
	6147.604.97.100; Condat GmbH

[77]	Message Specification DL
	6147.605.97.100; Condat GmbH

[78]	Technical Documentation CC
	6147.700.97.100; Condat GmbH

[79]	Technical Documentation SS
	6147.701.97.100; Condat GmbH

[80]	Technical Documentation SMS
	6147.702.97.100; Condat GmbH

[81]	Technical Documentation MM
	6147.703.97.100; Condat GmbH

[82]	Technical Documentation RR
	6147.704.97.100; Condat GmbH

[83]	Technical Documentation DL
	6147.705.97.100; Condat GmbH

[84]	Technical Documentation CCD
	6147.706.97.100; Condat GmbH

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Commmunication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

Different layers in the mobile station Protocol Stacks. It is also intended to offer an overview of the intersystem interface. The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The GSM specifications do not set out any implementation of the Protocol Stack. The following diagrams show the implementation described in all these documents for the mobile station. All entities except the Man Machine Interface and Physical Layer are implemented as part of the Protocol Stack.

This document describes the services offered by the short message service.

# Introduction

## Dynamic Configuration

Void

## Timer Modes

Void

# Initialisation

## Initial Parameters from the SIM card

MMI                   SMS                       SIM

|                    |                         |

|                    |   SIM SMS INSERT IND    |

|                    *<========================*

|                 (SMS 1)                      |

|                    |                         |

(SMS 1)

The SIM application sends to SMS the initial parameters from the SIM card. These are the SMS status whether Short message storage capacity is available on the SIM card, the last used message reference for mobile originated short message service and the SMS download flag. The SMS download flag defines whether data download via SMS for SIM toolkit is possible or not.

The SMS initialisation procedure starts now reading the SMS messages from the SIM card and the mobile memory.

## Reading SMS from SIM card

MMI                   SMS                       SIM

|                    |                         |

|                    |   SIM READ RECORD REQ   |

|                    *========================>*

|                    |                       (SIM 1)

|                    |   SIM READ RECORD CNF   |

|                    *<========================*

|                 (SMS 1)                      |

a) free record

|                    |                         |

|                    *                         |

|                 (SMS 2)                      |

|                    |                         |

b) mobile originated message

|                    |                         |

|  MNSMS MESSAGE IND |                         |

*<===================*                         |

(MMI 1)                |                         |

|                    |                         |

c) mobile terminated message

|                    |                         |

|  MNSMS MESSAGE IND |                         |

*<===================*                         |

(MMI 2)                |                         |

|                    |                         |

d) mobile terminated status report message

|                    |                         |

|  MNSMS MESSAGE IND |                         |

*<===================*                         |

(MMI 3)                |                         |

|                    |                         |

(SIM 1)

Starting with record 1 all SMS records are read. If the answer from the SIM application indicates that all records are read this procedure is finished.

(SMS 1)

The SIM application has read the requested record of the SIM card.

(SMS 2)

If it is a free record the status of the SMS message is stored.

(MMI 1)

If it is a mobile originated message, the status and the protocol identifier are stored from the SMS message. An indication is send to the MMI.

(MMI 2)

If it is a mobile terminated message, the status and the protocol identifier are stored from the SMS message. An indication is send to the MMI.

The status and protocol identifier backup for all SMS messages is used by SMS for faster access to the SIM card.

(MMI 3)

If it is a mobile terminated status report message, the status and the protocol identifier are not stored for this message, as only reading and deleting of the Status report message is allowed and not replacing of the message. An indication is sent to the MMI.

## Reading SMS from Mobile Memory

MMI                   SMS                       SIM

|                    |                         |

|                    *<pcm_read_record>        |

|                 (SMS 1)                      |

a) free record

|                    |                         |

|                    *                         |

|                 (SMS 2)                      |

|                    |                         |

b) mobile originated message

|                    |                         |

|  MNSMS MESSAGE IND |                         |

*<===================*                         |

(MMI 1)                |                         |

|                    |                         |

c) mobile terminated message

|                    |                         |

|  MNSMS MESSAGE IND |                         |

*<===================*                         |

(MMI 2)                |                         |

|                    |                         |

d) mobile terminated status report message

|                    |                         |

|  MNSMS MESSAGE IND |                         |

*<===================*                         |

(MMI 3)                |                         |

|                    |                         |

(SMS 1)

Starting with record 1 all SMS records are read from the permanent configuration memory. If the answer from the permanent configuration memory indicates that all records are read this procedure is finished.

(SMS 2)

If it is a free record the status of the SMS message is stored.

(MMI 1)

If it is a mobile originated message, the status and the protocol identifier are stored from the SMS message. An indication is send to the MMI.

(MMI 2)

If it is a mobile terminated message, the status and the protocol identifier are stored from the SMS message. An indication is send to the MMI.

The status and protocol identifier backup for all SMS messages is used by SMS for faster access to the permanent configuration memory.

(MMI 3)

If it is a mobile terminated status report message, the status and the protocol identifier are not stored for this message, as only reading and deleting of the Status report message is allowed and not replacing of the message. An indication is sent to the MMI.

# Mobile Originated Short Message Service

The mobile originated short message service enables the user to send a text message to the infrastructure. The receiving side is the so-called service centre (SC), which handles sending of the message to the destination address.

## Establishment Phase

### Initiation by Man Machine Interface

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

| MNSMS SUBMIT REQ   |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|                    |          SIM UPDATE REQ           |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |cp_establish_req |                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |       MMSMS ESTABLISH REQ    |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |                 |                 |            |

(SMS 1)

The user starts sending of a short message.

(SIM 1)

Each short message contains a message reference. This number is used to identify this short message for later transactions. The message reference is stored on the SIM card, if it is a phase 2/2+ SIM card.

(SMS 2)

The RP DATA message is created and forwarded to the Control Protocol layer. The timer TR1M is started.

(MM 1)

The RP DATA message is stored and the establishment of the SMS-Connection is requested by MM.

### Confirmation of SMS Connection

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |       MMSMS ESTABLISH CNF    |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |         MMSMS DATA REQ       |

|                    |                 |             (CP DATA)        |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |                 |                 |            |

(SMS 1)

MM confirms the establishnment of the SMS connection.

(MM 1)

The control protocol builds a CP DATA message containing the stored RP DATA message. The message is forwarded to MM. The timer T1CM is started to control the answer of the network. The retransmission counter is resetted.

### No SMS Connection established

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |       MMSMS RELEASE IND      |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_error_ind  |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

MM indicates that the SMS-connection has not been established.

(SMS 2)

The control protocol indicates the error to the relay layer.

(MMI 1)

The error is reported to the user.

### Timeout TR1M

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    *<timeout TR1M>   |                 |            |

|                 (SMS 1)              |                 |            |

|                    |   cp_abort_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

In the relay layer the timer TR1M times-out. That means that no answer has received from the infrastructure.

(SMS 2)

The control protocol is informed about the abort.

(MM 1)

The requested SMS connection is released.

(MMI 1)

The user is informed about the abort of the mobile originated short message service.

### Multiple Request  by Man Machine Interface

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|  MNSMS SUBMIT REQ  |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

The user tries to send an additional mobile originated short message althougth the current transaction has not finished. At one time only one transaction is allowed.

(MMI 1)

The user is informed about rejection of the mobile originated short message request.

### Confirmation to CP DATA Message

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |           MMSMS DATA IND     |

|                    |                 |              (CP ACK)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |                 |            |

(SMS 1)

Control Protocol is in the state Wait for CP ACK. That means it waits for the acknowledgement of the previous CP-DATA message. This acknowlegment receives. The timer T1CM is stopped and Control Protocol enters the state SMS CONNECTION ESTABLISHED.

### Wrong Message received

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |           MMSMS DATA IND     |

|                    |                 |       (CP DATA or unknown)   |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |           MMSMS DATA REQ     |

|                    |                 |             (CP ERROR)       |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |   rl_error_ind  |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

Control Protocol expects a CP ACK message as response to its own CP DATA message. Instead of this a CP DATA or unknown message receives. The timer T1CM is stopped.

(MM 1)

Control Protocol answers with a CP ERROR message and the cause #98 for a CP DATA message or #97 for an unknown message.

(SMS 2)

Relay Layer is informed about the error.

(MMI 1)

The user is informed about the error.

(SMS 3)

The SMS-Connection is not longer necessary and will be released.

(MM 2)

MM is informed about the release of the SMS connection.

### CP Error received

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |           MMSMS DATA IND     |

|                    |                 |             (CP ERROR)       |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_error_ind  |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |                 |                 |            |

(SMS 1)

Control Protocol exspects a CP ACK message as response of its own previous CP DATA message. Instead a CP ERROR message receives. The timer T1CM is stopped.

(SMS 2)

The error cause is forwarded to the relay layer.

(MMI 1)

The user is informed about the error.

(SMS 3)

The release of the SMS connection is requested.

(MM 1)

MM is informed about the release of the SMS connection.

### Lower Layer Failure

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |           MMSMS ERROR IND    |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    | rl_release_ind  |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

A lower layer is detected. The SMS connection is not longer valid.

(SMS 2)

Relay layer is informed about the loss of connection.

(MMI 1)

The user is informed about the error.

### Release of MM-Connection

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |       MMSMS RELEASE IND      |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    | rl_release_ind  |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

The lower layer have released the connection. Control Protocol is informed.

(SMS 2)

This information is forwarded to relay layer.

(MMI 1)

The user is informed about the error.

### Timeout TC1M

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 *<timeout TC1M>   |            |

|                    |              (SMS 1)              |            |

|                    |                 |                 |            |

a) max retransmission not reached

|                    |                 |                 |            |

|                    |                 |          MMSMS DATA REQ      |

|                    |                 |              (CP DATA)       |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |                 |                 |            |

b) max retransmission reached

|                    |                 |                 |            |

|                    |   rl_error_ind  |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |         MMSMS RELEASE REQ    |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

The timer T1CM of the control protocol times-out.

(MM 1)

If this happens the first time, the CP DATA message is retransmitted.

(SMS 2)

The timeout occurs a second time. So the maximum of retransmissions is reached and the error is forwarded to the relay layer.

(MMI 1)

The user is informed about the error.

(SMS 3)

The SMS connection is not longer necessary. Relay layer requests the release of SMS connection.

(MM 2)

Control Protocol releases the SMS connection.

### Timeout TR1M (SMS connection established)

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    *<timeout TR1M>   |                 |            |

|                 (SMS 1)              |                 |            |

|                    |   cp_abort_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |          MMSMS DATA REQ      |

|                    |                 |      (CP ERROR, Cause #111)  |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |         MMSMS RELEASE REQ    |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

A SMS connection is established. Then the timer T1RM of the relay layer expires.

(SMS 2)

The control protocol is informed about the abort.

(MM 1)

Control Protocol sends a CP ERROR message with the cause #111 to the infrastructure.

(SMS 3)

The relay layer requests the release of the SMS connection.

(MM 2)

The SMS connection is released.

## Active Phase

### Acknowledge of the Infrastructure

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |           MMSMS DATA IND     |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_data_ind   |                 |            |

|                    |     (RP_ACK)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |           MMSMS DATA REQ     |

|                    |                 |               (CP ACK)       |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

A SMS connection is established and the response of the network receives. It is a CP DATA message containing a RP ACK message.

(SMS 2)

The RP ACK message is forwarded to the relay layer.

(MMI 1)

The user is informed about the positive end of procedure.

(MM 1)

After reception of the CP DATA message control protocol sends a CP ACK message as response to the infrastructure.

(SMS 2)

Relay Layer releases the SMS connection.

(MM 2)

The release of SMS connection is requested to MM.

### Error signalled by the Infrastructure

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |           MMSMS DATA IND     |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_data_ind   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |                 |                 |            |

(SMS 1)

A SMS connection is established and the response of the network receives. It is a CP DATA message containing a RP ERROR message.

(SMS 2)

The RP ERROR message is forwarded to the relay layer.

(MMI 1)

The user is informed about the negative end of procedure.

(SMS 2)

Relay Layer releases the SMS connection.

(MM 1)

The release of SMS connection is requested to MM.

### Wrong Message signalled by the Infrastructure (CP layer)

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |           MMSMS DATA IND     |

|                    |                 |        (CP ACK or unknown)   |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_error_ind  |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |           MMSMS DATA REQ     |

|                    |                 |              (CP ERROR)      |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

A SMS connection is established and the response of the network receives. It is a CP ACK or an unknown message.

(SMS 2)

The error is forwarded to the relay layer.

(MMI 1)

The user is informed about the negative end of procedure.

(MM 1)

Control protocol sends a CP ERROR message as response to the infrastructure. The used cause is #97 for an unknown message and #98 for a CP ACK message.

(SMS 2)

Relay Layer releases the SMS connection.

(MM 2)

The release of SMS connection is requested to MM.

### Wrong Message signalled by the Infrastructure (RL layer)

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |           MMSMS DATA IND     |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_data_ind   |                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |           MMSMS_DATA_REQ     |

|                    |                 |             (CP DATA)        |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 4)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |                 |                 |            |

(SMS 1)

A response of the infrastructure is expected. Control Protocol receives a CP DATA message.

(SMS 2)

This CP DATA message contains a RP DATA message instead of the expected RP ACK message.

(SMS 3)

Relay Layer builds a RP ERROR message and forwards it to the control protocol.

(MM 1)

The RP ERROR message is included into a CP DATA message and sent to the infrastructure.

(MMI 1)

The error is reported to the user.

(SMS 4)

Relay Layer requests the release of the SMS connection.

(MM 1)

The SMS connection is released.

### Timeout TR1M

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    *<timeout TR1M>   |                 |            |

|                 (SMS 1)              |                 |            |

|                    |  cp_abort_req   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |           MMSMS DATA REQ     |

|                    |                 |          (CP ERROR, #111)    |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

SMS expects a response of the infrastructure. In the relay layer the timer TR1M timeout.

(SMS 2)

The abort of the SMS connection is requested to the control protocol.

(MM 1)

A CP ERROR message with the cause #111 is send to the infrastructure.

(SMS 2)

The SMS connection is not longer necessary.

(MM 2)

The release of the SMS connection is requested to MM.

(MMI 1)

The user is informed about the error.

### Lower Layer Failure

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |         MMSMS ERROR IND      |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |  rl_release_ind |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

SMS expects a response of the infrastructure. Instead a lower layer failure is indicated.

(SMS 2)

The release indication is forwarded to the relay layer.

(MMI 1)

The user is informed about the negative end of procedure.

### Release of MM-Connection

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |         MMSMS RELEASE IND    |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |  rl_release_ind |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

| MNSMS REPORT IND   |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

SMS exspects the response of the infrastructure. Instead the lower layer indicate the release of connection.

(SMS 2)

The release indication is forwarded to the relay layer.

(MMI 1)

The user is informed about the negative end of procedure.

## Mobile Originated Short Message Command

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|  MNSMS COMMAND REQ |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|                    |             SIM UPDATE REQ        |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    | cp_establish_req|                 |            |

|                    |     (RP DATA)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |      MMSMS ESTABLISH REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |                 |      MMSMS ESTABLISH CNF     |

|                    |                 *<=============================*

|                    |              (SMS 3)              |            |

|                    |                 |         MMSMS DATA REQ       |

|                    |                 |             (CP DATA)        |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |         MMSMS DATA IND       |

|                    |                 |              (CP ACK)        |

|                    |                 *<=============================*

|                    |              (SMS 4)              |            |

|                    |                 |         MMSMS DATA IND       |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 5)              |            |

|                    |   rl_data_ind   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 6)              |                 |            |

| MNSMS REPORT IND   |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |  cp_release_req |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 7)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 3)

|                    |                 |                 |            |

(SMS 1)

The mobile originated short message command procedure is used to send commands to the service centre for previous sent short messages. The procedure differs from a mobile originated short message service procedure only in the different initial primitive.

The mobile station starts sending of a short message command. The following commands are available:

Status request for a short message

Delete of status report request for a short message and

Delete of a short message.

The named short message is identified by the message reference used for the short message

(SIM 1)

Each short message command has its own message reference. This number is used if other short message commands are related to this short message command. The message reference is incremented by one. If a phase 2 SIM is available the message reference is stored on the SIM card.

(SMS 2)

The relay layer builds a RP DATA message containing the short message command. The message is forwarded to the control protocol. The timer TR1M is started to supervise the response of the infrastructure.

(MM 1)

Control Protocol requests establishment of the SMS connection by MM.

(SMS 3)

MM confirms establishment of the SMS connection.

(MM 2)

The CP DATA message containing the RP DATA message of the relay layer is send to the infrastructure. The timer T1CM is started to supervise response of the infrastructure.

(SMS 4)

Control Protocol receives the response of the infrastructure. It is a CP ACK message. It now waits for the response for the relay layer.

(SMS 5)

A CP DATA message receives. The content for the relay layer is decoded.

(SMS 6)

The content of the message is a RP ACK message.

(MMI 1)

The user is informed about the positive end of procedure.

(SMS 7)

The connection release is requested.

(MM 3)

The SMS connection is released.

# Mobile Terminated Short Message Service

With the mobile terminated short message service it is possible to send a textual message from the infrastructure to the mobile station. This message contains the data coding scheme describing the message class.

Additionally it is possible to configure the preferred memory (SIM or mobile equipment (ME) memory) and the used mode (MT = 0 .. 3) according the AT command +CNMI. If nothing is configured SMS prefers SIM card memory and MT is set to 4.

The following tables show the handling in the relay layer of short message service with the following meanings:

- DISPLAY	forward directly to MMI

- MEM_ME	store in ME memory

- MEM_SIM	store in SIM memory

- NOTHING	do nothing

SMS differs between the following data coding schemes:

- NO CLASS

- CLASS 0

- CLASS 1

- CLASS 2

- CLASS 3

- DISCARD		Mail-Waiting Indicator, discard message

- STORE		Mail-Waiting Indicator, store message

The first action if the preferred memory is ME memory is carried out according the following table:

| MT | NO CLASS | CLASS 0 | CLASS 1 | CLASS 2 | CLASS 3 | DISCARD | STORE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MEM_ME | DISPLAY | MEM_ME | MEM_SM | MEM_ME | DISPLAY | MEM_ME |
| 1 | MEM_ME | DISPLAY | MEM_ME | MEM_SM | MEM_ME | DISPLAY | MEM_ME |
| 2 | DISPLAY | DISPLAY | DISPLAY | MEM_SM | DISPLAY | DISPLAY | MEM_ME |
| 3 | MEM_ME | DISPLAY | MEM_ME | MEM_SM | DISPLAY | DISPLAY | MEM_ME |
| 4 (default) | MEM_SM | DISPLAY | MEM_ME | MEM_SM | MEM_SM | DISPLAY | MEM_SM |

The second action if the preferred memory is ME memory is carried out according the following table:

| MT | NO CLASS | CLASS 0 | CLASS 1 | CLASS 2 | CLASS 3 | DISCARD | STORE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | NOTHING | NOTHING | MEM_SM | MEM_ME | MEM_SM | NOTHING | NOTHING |
| 1 | NOTHING | NOTHING | MEM_SM | MEM_ME | MEM_SM | NOTHING | NOTHING |
| 2 | NOTHING | NOTHING | NOTHING | MEM_ME | NOTHING | NOTHING | NOTHING |
| 3 | NOTHING | NOTHING | MEM_SM | MEM_ME | NOTHING | NOTHING | NOTHING |
| 4 (default) | NOTHING | NOTHING | MEM_SM | MEM_ME | MEM_ME | NOTHING | NOTHING |

The first action if the preferred memory is SIM memory is carried out according the following table:

| MT | NO CLASS | CLASS 0 | CLASS 1 | CLASS 2 | CLASS 3 | DISCARD | STORE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MEM_SM | DISPLAY | MEM_SM | MEM_SM | MEM_SM | DISPLAY | MEM_SM |
| 1 | MEM_SM | DISPLAY | MEM_SM | MEM_SM | MEM_SM | DISPLAY | MEM_SM |
| 2 | DISPLAY | DISPLAY | DISPLAY | MEM_SM | DISPLAY | DISPLAY | MEM_SM |
| 3 | MEM_SM | DISPLAY | MEM_SM | MEM_SM | DISPLAY | DISPLAY | MEM_SM |
| 4 (default) | MEM_SM | DISPLAY | MEM_ME | MEM_SM | MEM_SM | DISPLAY | MEM_SM |

The second action if the preferred memory is SIM memory is carried out according the following table:

| MT | NO CLASS | CLASS 0 | CLASS 1 | CLASS 2 | CLASS 3 | DISCARD | STORE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | NOTHING | NOTHING | MEM_ME | MEM_ME | MEM_ME | NOTHING | NOTHING |
| 1 | NOTHING | NOTHING | MEM_ME | MEM_ME | MEM_ME | NOTHING | NOTHING |
| 2 | NOTHING | NOTHING | NOTHING | MEM_ME | NOTHING | NOTHING | NOTHING |
| 3 | NOTHING | NOTHING | MEM_ME | MEM_ME | NOTHING | NOTHING | NOTHING |
| 4 (default) | NOTHING | NOTHING | MEM_SM | MEM_ME | MEM_ME | NOTHING | NOTHING |

## Reception of a Short Message

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|MNSMS CONFIGURE REQ |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|                    |                 |       MMSMS ESTABLISH IND    |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 2)              |            |

|                    | rl_establish_ind|                 |            |

|                    |     (RP_DATA)   |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 3)              |                 |            |

|                    |                 |          MMSMS DATA REQ      |

|                    |                 |             (CP ACK)         |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                 ...*                 |                 |            |

|                 (SMS 4)              |                 |            |

|                    |                 |                 |            |

(SMS 1)

MMI configures the reception modes for mobile terminated short messages.

(SMS 2)

MM indicates establishment of a SMS connection. This initial primitive contains a CP DATA message.

(SMS 3)

The content of the CP DATA message is a RP DATA message that is forwarded to the Relay Layer.

(MM 1)

Control Procotol sends a CP ACK message as acknowledgement to the infrastructure.

(SMS 4)

The following handling of the mobile terminated short message depends on the protocol identifier and the data coding scheme of the message and of the configured modes by MMI.

## Reception of CP ERROR message

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |       MMSMS ESTABLISH IND    |

|                    |                 |             (CP ERROR)       |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |                 |                 |            |

(SMS 1)

MM indicates a new SMS connection. The incoming message is a CP ERROR message. The message is ignored.

(MM 1)

The SMS connection is released.

## Reception of CP ACK or unknown message

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |       MMSMS ESTABLISH IND    |

|                    |                 |       (CP ACK or unknown)    |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |                 |          MMSMS DATA REQ      |

|                    |                 |             (CP ERROR)       |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

A new SMS connection is indicated by MM. The incoming message is CP ACK or unknown.

(MM 1)

Control Protocol builds a CP ERROR message with cause #97 for an unknown message and #98 for a CP ACK message.

(MM 2)

The SMS Connection is released.

## Second Mobile Terminated Connection

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |        MMSMS ESTABLISH IND   |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |          MMSMS DATA REQ      |

|                    |                 |             (CP DATA)        |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 4)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

MM indicates a second mobile terminated short message connection with a CP DATA message.

(SMS 2)

The content of the CP DATA message is forwarded to the Relay Layer.

(SMS 3)

Only one terminated transaction is allowed in parallel. So a RP ERROR message is build by the Relay Layer and forwarded to the Control Protocol.

(MM 1)

The message is included into a CP DATA message and sent to the infrastructure.

(SMS 4)

The Relay Layer releases the SMS connection.

(MM 2)

The release request is forwarded to MM.

## Additional Message for  Mobile Terminated Connection

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |          MMSMS DATA IND      |

|                    |                 |             (CP DATA)        |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    |   rl_data_ind   |                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |          MMSMS DATA REQ      |

|                    |                 |             (CP DATA)        |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    |                 |                 |            |

(SMS 1)

Control Protocol receives additional CP DATA messages for an existing transaction.

(SMS 2)

The content of this CP DATA message is a RP DATA message. It is forwarded to the Relay Layer.

(SMS 3)

The relay layer builds a RP ERROR message and sends it back to the Control Protocol.

(MM 1)

This message is part of a CP DATA message that is forwarded to the infrastructure.

## Display Message

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|  MNSMS MESSAGE IND |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |            MMSMS DATA REQ    |

|                    |                 |               (CP DATA)      |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message receives in the Relay Layer. Depending on the protocol identifier, the data coding scheme and the configured modes SMS decides that the message shall be displayed immediately and not stored.

(MMI 1)

The message is displayed to the user.

(SMS 2)

Relay Layer builds a RP ACK message and sent it to Control Protocol.

(MM 1)

The message is part of a CP DATA message that is build by Control Protocol. The message is sent to the infrastructure.

(SMS 3)

The Relay Layer releases the SMS connection.

(MM 2)

The release of SMS connection is forwarded to MM.

## Store Message in ME memory

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|                    |                 |                 |            |

a) storage capacity available in mobile station memory

|                    |                 |                 |            |

|  MNSMS MESSAGE IND |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |            MMSMS DATA REQ    |

|                    |                 |               (CP DATA)      |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

b) no storage capacity available in mobile station memory

|                    |                 |                 |            |

|                    *...              |                 |            |

|                 (SMS 4)              |                 |            |

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message receives in the Relay Layer. Depending on the protocol identifier, the data coding scheme and the configured modes SMS decides that the message shall be stored in the ME memory.

(MMI 1)

The message is stored in the memory of the mobile station. An indication is sent to the user. The user can read the message with the SMS read procedure.

(SMS 2)

Relay Layer codes a RP ACK message and forwards it to the Control Protocol.

(MM 1)

The RP ACK message is part of the CP DATA message that is sent to the infrastructure.

(SMS 3)

The existing SMS-Connection is not longer necessary. The Relay Layer requests the release.

(MM 2)

The release of SMS connection is send to MM.

(SMS 4)

If no free entry is available in the mobile station memory, the relay layer starts the second action defined for this type of message or sends an error indication to the network as described in the next chapter.

### No Storage available on ME memory, no other opportunities

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 1)              |            |

|                    |                 |            MMSMS DATA REQ    |

|                    |                 |               (CP DATA)      |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

The error, that no memory is available is coded in a RP ERROR message and forwarded to the CP layer.

(MM 1)

The RP ERROR message is filled in a CP DATA message and forwarded to the infrastructure.

(SMS 2)

The RL layer will release the SMS connection.

(MM 2)

MM is informed about the release of the SMS connection.

## Store Message in SIM memory

### Storage on SIM available, writing successful

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|                    |       SIM UPDATE RECORD REQ       |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |       SIM UPDATE RECORD CNF       |            |

|                    *<==================================*            |

|                 (SMS 2)              |                 |            |

|   MNSMS ALERT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |            MMSMS DATA REQ    |

|                    |                 |               (CP DATA)      |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 4)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message receives in the Relay Layer. Depending on the protocol identifier, the data coding scheme and the configured modes SMS decides that the message shall be stored in the SIM memory. Using the backup of SMS status bytes from the SIM card a free record is found.

(SIM 1)

The SMS message is written to this record.

(SMS 2)

SMS receives the confirmation of writing the SMS message to the SIM memory.

(MMI 1)

An indication is send to the user. The user can read the message with the SMS read procedure.

(SMS 3)

Relay Layer codes a RP ACK message and forwards it to the Control Protocol.

(MM 1)

The RP ACK message is part of the CP DATA message that is sent to the infrastructure.

(SMS 4)

The existing SMS-Connection is not longer necessary. The Relay Layer requests the release.

(MM 2)

The release of SMS connection is send to MM.

### Storage on SIM available, writing not successful

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|                    |       SIM UPDATE RECORD REQ       |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |       SIM UPDATE RECORD CNF       |            |

|                    *<==================================*            |

|                 (SMS 2)              |                 |            |

|                    |          SIM UPDATE REQ           |            |

|                    *==================================>*            |

|                    |                 |              (SIM 2)         |

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message receives in the Relay Layer. Depending on the protocol identifier, the data coding scheme and the configured modes SMS decides that the message shall be stored in the SIM memory. Using the backup of SMS status bytes from the SIM card a free record is found.

(SIM 1)

The SMS message is written to this record.

(SMS 2)

SMS indicates a write error during updating the record.

(SIM 2)

A memory capacity exceeded message will be send to the infrastructure to indicate that the SIM memory is not accessible. Therefore the message number in this mobile originated short message attempt is incremented.

### No Storage on SIM available

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|                    |          SIM UPDATE REQ           |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message receives in the Relay Layer. Depending on the protocol identifier, the data coding scheme and the configured modes SMS decides that the message shall be stored in the SIM memory. Using the backup of SMS status bytes from the SIM card SMS finds no free entry.

(SIM 1)

A memory capacity exceeded message will be send to the infrastructure to indicate that the SIM memory is not accessible. Therefore the message number in this mobile originated short message attempt is incremented.

### No Storage available on SIM memory, no other opportunities

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |    (RP ERROR)   |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 1)              |            |

|                    |                 |          MMSMS DATA REQ      |

|                    |                 |             (CP DATA)        |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

The error, that no memory is available is coded in a RP ERROR message and forwarded to the CP layer.

(MM 1)

The RP ERROR message is filled in a CP DATA message and forwarded to the infrastructure.

(SMS 2)

The RL layer will release the SMS connection.

(MM 2)

MM is informed about the release of the SMS connection.

## Replace Short Message

### Replace in Mobile Station Memory

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |      MMSMS ESTABLISH IND     |

|                    |                 |            (CP DATA)         |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|                    *<replace message in memory>        |            |

|                 (SMS 3)              |                 |            |

|                    *<replace message on SIM card>      |            |

|                 (SMS 4)              |                 |            |

|                    |                 |                 |            |

(SMS 1)

MM indicates the reception of a mobile terminated short message.

(SMS 2)

The protocol identifier of the incoming short message indicates that it is a replace short message. That means it shall replace a short message that is already stored in the mobile station.

(SMS 3)

Using the ME backup of status and protocol identifier of all SMS messages in the ME memory a candidate is selected by SMS for replacing. This SMS message must be mobile terminated and must have the same protocol identifier. If such a SMS message is available, SMS reads the whole message from the permanent configuration memory and compares originator address and service centre address. If all matches the new message is stored in the mobile memory replacing the old one.

(SMS 4)

If replacing in the mobile memory has failed, the same is carried out for the SIM card memory. A pre-selection of candidates is carried out using the stored status and protocol identifier of the SMS messages stored on the SIM card. Then a possible record is read from the SIM card and the originator address and the service centre address are checked. If this parameter are also matching with the received message the SMS record on the SIM card is replaced.

If both checks are failed, normal storing of the incoming message is carried out.

## Mobile Terminated Short Message Status

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 |      MMSMS ESTABLISH IND     |

|                    |                 |            (CP DATA)         |

|                    |                 *<=============================*

|                    |              (SMS 1)              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 2)              |                 |            |

|  MNSMS STATUS IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 2)              |            |

|                    |                 |            MMSMS DATA REQ    |

|                    |                 |               (CP DATA)      |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

MM indicates the reception of a mobile terminated short message.

(SMS 2)

It is a SMS STATUS message.

(MMI 1)

The content of the SMS STATUS message is forwarded to MMI.

(SMS 2)

The reception of the RP DATA message is acknowledged by the RP ACK message.

(MM 1)

The message is forwarded to the infrastructure.

(SMS 3)

The RL layer releases the SMS connection.

(MM 2)

MM is informed about the release of SMS connection.

## SIM Toolkit Data Download

MMI                 SMS(RL)           SMS(CP)            SIM          MM

|                    |                 |                 |            |

|                    |                 *...              |            |

|                    | rl_establish_ind|                 |            |

|                    |    (RP DATA)    |                 |            |

|                    *<----------------*                 |            |

|                 (SMS 1)              |                 |            |

|                    |           SIM_TOOLKIT_REQ         |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |           SIM_TOOLKIT_CNF         |            |

|                    *==================================>*            |

|                 (SMS 2)              |                 |            |

|                    |   cp_data_req   |                 |            |

|                    |     (RP ACK)    |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 3)              |            |

|                    |                 |            MMSMS DATA REQ    |

|                    |                 |               (CP DATA)      |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 1)

|                    | cp_release_req  |                 |            |

|                    *---------------->*                 |            |

|                    |              (SMS 4)              |            |

|                    |                 |        MMSMS RELEASE REQ     |

|                    |                 *=============================>*

|                    |                 |                 |         (MM 2)

|                    |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message receives in the Relay Layer. The protocol identifier indicates SIM Data Download.

(SIM 1)

If the sms download flag indicates support of this feature, the message content is forwarded to the SIM application.

(SMS 2)

The SIM application response receives.

(SMS 3)

Relay Layer builds a RP ACK message that may contain response data from the SIM application and sent it to Control Protocol.

(MM 1)

The message is part of a CP DATA message that is build by Control Protocol. The message is sent to the infrastructure.

(SMS 4)

The Relay Layer will release the SMS connection.

(MM 2)

The release of SMS connection is forwarded to MM.

## SMS - Depersonalisation

MMI                  SMS(RL)           SMS(CP)            SIM          MM

|                     |                 |                 |            |

|                     |                 *...              |            |

|                     | rl_establish_ind|                 |            |

|                     |    (RP DATA)    |                 |            |

|                     *<----------------*                 |            |

|                  (SMS 1)              |                 |            |

|   MNSMS_MESSAGE_IND |                 |                 |            |

*<====================*                 |                 |            |

(MMI 1)                 |                 |                 |            |

| MNSMS_ACK_RES       |                 |                 |            |

*====================>*                 |                 |            |

|                  (SMS 2)              |                 |            |

|                     |   cp_data_req   |                 |            |

|                     |     (RP ACK)    |                 |            |

|                     *---------------->*                 |            |

|                     |              (SMS 3)              |            |

|                     |                 |            MMSMS DATA REQ    |

|                     |                 |               (CP DATA)      |

|                     |                 *=============================>*

|                     |                 |                 |        (MM 1)

|                     | cp_release_req  |                 |            |

|                     *---------------->*                 |            |

|                     |              (SMS 4)              |            |

|                     |                 |        MMSMS RELEASE REQ     |

|                     |                 *=============================>*

|                     |                 |                 |        (MM 2)

|                     |                 |                 |            |

(SMS 1)

The incoming mobile terminated short message receives in the Relay Layer. The protocol identifier indicates SMS De-personalisation.

(MMI 1)

The SMS message is forwarded to MMI.

(SMS 2)

MMI sends the response to SMS.

(SMS 3)

Relay Layer builds a RP ACK message containing the DELIVER REPORT from MMI and sent it to Control Protocol.

(MM 1)

The message is part of a CP DATA message that is build by Control Protocol. The message is sent to the infrastructure.

(SMS 4)

The Relay Layer will release the SMS connection.

(MM 2)

The release of SMS connection is forwarded to MM.

# Short Message Service Procedures

## Storing of Short Messages

MMI                 SMS(RL)           SMS(CP)            SIM           MM

|                    |                 |                 |            |

|  MNSMS STORE REQ   |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|                    |         SIM UPDATE RECORD REQ     |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |         SIM UPDATE RECORD CNF     |            |

|                    *<==================================*            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

MMI will store a mobile originated short message.

(SIM 1)

The memory type indicates storing on the SIM card. Therefore the SMS record on the SIM card is updated. If the memory type indicates storing in the mobile memory, the SMS message is stored in the mobile memory.

(SMS 2)

The SIM application indicates the successful operation of updating a SMS record.

(MMI 1)

An indication about the result of the operation is forwarded to the MMI.

## Changing of Short Messages

MMI                 SMS(RL)           SMS(CP)            SIM           MM

|                    |                 |                 |            |

|  MNSMS STORE REQ   |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|                    |         SIM UPDATE RECORD REQ     |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |         SIM UPDATE RECORD CNF     |            |

|                    *<==================================*            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

MMI will change a short message. Therefore it has read the short message before.

(SIM 1)

The memory type indicates storing on the SIM card. Therefore the SMS record on the SIM card is updated. If the memory type indicates storing in the mobile memory, the SMS message is stored in the mobile memory.

(SMS 2)

The SIM application indicates the successful operation of updating a SMS record.

(MMI 1)

An indication about the result of the operation is forwarded to the MMI.

## Reading of Short Messages

MMI                 SMS(RL)           SMS(CP)            SIM           MM

|                    |                 |                 |            |

|  MNSMS READ REQ    |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|                    |          SIM READ RECORD REQ      |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |          SIM READ RECORD CNF      |            |

|                    *<==================================*            |

|                 (SMS 2)              |                 |            |

|  MNSMS MO/MT IND   |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

MMI will read a short message.

(SIM 1)

If the memory type indicates reading from the SIM card, reading of a SMS record is requested from the SIM card. If the memory type mobile memory is selected SMS reads the record from the permanent configuration memory.

(SMS 2)

The SIM application returns the content of the record.

(MMI 1)

The mobile originated or terminated short message is forwarded to the MMI.

## Deleting of Short Messages

MMI                 SMS(RL)           SMS(CP)            SIM           MM

|                    |                 |                 |            |

|  MNSMS DELETE REQ  |                 |                 |            |

*===================>*                 |                 |            |

|                 (SMS 1)              |                 |            |

|                    |         SIM UPDATE RECORD REQ     |            |

|                    *==================================>*            |

|                    |                 |              (SIM 1)         |

|                    |         SIM UPDATE RECORD CNF     |            |

|                    *<==================================*            |

|                 (SMS 2)              |                 |            |

|  MNSMS REPORT IND  |                 |                 |            |

*<===================*                 |                 |            |

(MMI 1)                |                 |                 |            |

|                    |                 |                 |            |

(SMS 1)

MMI will delete a short message. Therefore it has read the short message before.

(SIM 1)

If the memory type indicates SIM card memory, the SIM application is requested to updated the content of the actual record, that means to write a free record content to this record. If mobile memory is selected, the message is stored directly and the response is given as described at (MMI 1).

(SMS 2)

The SIM application indicates the successful operation.

(MMI 1)

An indication about the result of the operation is forwarded to the MMI.

# Data Formats

## Mobile Terminated Short Message (Air-Interface)

| Information Element Identifier | Message Structure |  |  |  |  | Example |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Transaction Identifier | Protocol Discriminator |  |  |  |  |  |  |
|  | Message Type |  |  |  |  |  |  |  |
| CP User Data | Length User Data |  |  |  |  |  |  |  |
| RP-Data | 0 |  | mti |  | 01 |  |  |  |
| Message Reference | Message Reference |  |  |  |  |  |  |  |
| RP-Originator Address | Length Originator Address |  |  |  |  | 04 |  |  |
|  | Ton & Npi |  |  |  |  | 81 |  |  |
|  | Digits |  |  |  |  | 21 |  |  |
|  |  |  |  |  |  | 43 |  |  |
|  |  |  |  |  |  | F5 |  |  |
| RP-Destination Address | Not used |  |  |  |  | 00 |  |  |
| RP-User Data | Length RP User Data |  |  |  |  |  |  |  |
| RP-Data | Flags |  | mti |  |  |  |  |  |
| TP-Originator Address | Length TP-Originator Address |  |  |  |  | 04 |  |  |
|  | Ton & Npi |  |  |  |  | 81 |  |  |
|  | Digits |  |  |  |  | 89 |  |  |
|  |  |  |  |  |  | 67 |  |  |
|  |  |  |  |  |  | 45 |  |  |
| Protocol Identifier | Protocol Identifier |  |  |  |  | 40 |  |  |
| Data Coding Scheme | Data Coding Scheme |  |  |  |  | F0 |  |  |
| Service Centre Time Stamp | Service Centre Time Stamp |  |  |  |  | 89 |  |  |
|  |  |  |  |  |  | 10 |  |  |
|  |  |  |  |  |  | 70 |  |  |
|  |  |  |  |  |  | 21 |  |  |
|  |  |  |  |  |  | 43 |  |  |
|  |  |  |  |  |  | 65 |  |  |
|  |  |  |  |  |  | 04 |  |  |
| TP-User Data | Length TP-User Data |  |  |  |  | 08 |  |  |
|  |  |  |  |  |  | 41 |  |  |
|  |  |  |  |  |  | E1 |  |  |
|  |  |  |  |  |  | 90 |  |  |
|  |  |  |  |  |  | 58 |  |  |
|  |  |  |  |  |  | 34 |  |  |
|  |  |  |  |  |  | 1E |  |  |
|  |  |  |  |  |  | 91 |  |  |
|  |  |  |  |  |  | 49 |  |  |

## Mobile Terminated Short Message (SIM)

| Information Element Identifier | Message Structure |  |  |  | Example |
| --- | --- | --- | --- | --- | --- |
| SMS Status | status |  |  |  |  |
| Service Centre Address | Length Service Centre Address |  |  |  | 04 |
|  | Ton & Npi |  |  |  | 81 |
|  | Digits |  |  |  | 21 |
|  |  |  |  |  | 43 |
|  |  |  |  |  | 65 |
| TP-Message Type | Flags | Mti |  |  |  |
| TP- Originator Address | Length TP-Originator Address |  |  |  | 04 |
|  | Ton & Npi |  |  |  | 81 |
|  | Digits |  |  |  | 56 |
|  |  |  |  |  | 31 |
|  |  |  |  |  | 21 |
| Protocol Identifier | Protocol Identifier |  |  |  | 40 |
| Data Coding Scheme | Data Coding Scheme |  |  |  | F0 |
| Service Centre Time Stamp | Service Centre Time Stamp |  |  |  | 89 |
|  |  |  |  |  | 10 |
|  |  |  |  |  | 70 |
|  |  |  |  |  | 21 |
|  |  |  |  |  | 43 |
|  |  |  |  |  | 65 |
|  |  |  |  |  | 04 |
| TP-User Data | Length TP-User Data |  |  |  | 06 |
|  |  |  |  |  | 41 |
|  |  |  |  |  | E1 |
|  |  |  |  |  | 90 |
|  |  |  |  |  | 58 |
|  |  |  |  |  | 34 |
|  |  |  |  |  | 02 |

## Mobile Originated Short Message (Air-Interface)

| Information Element Identifier | Message Structure |  |  |  |  | Example |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Transaction Identifier | Protocol Discriminator |  |  |  |  |  |  |
|  | Message Type |  |  |  |  |  |  |  |
| CP User Data | Length User Data |  |  |  |  |  |  |  |
| RP-Data | 0 |  | mti |  | 00 |  |  |  |
| Message Reference | Message Reference |  |  |  |  |  |  |  |
| RP-Originator Address | Not used |  |  |  |  | 00 |  |  |
| RP-Destination Address | Length Destination Address |  |  |  |  | 04 |  |  |
|  | Ton & Npi |  |  |  |  | 81 |  |  |
|  | Digits |  |  |  |  | 21 |  |  |
|  |  |  |  |  |  | 43 |  |  |
|  |  |  |  |  |  | 65 |  |  |
| RP-User Data | Length RP User Data |  |  |  |  |  |  |  |
| RP-Data | Flags |  | Mti |  |  |  |  |  |
| TP-Message Reference | Message Reference |  |  |  |  |  |  |  |
| TP-Destination Address | Length TP-Destination Address |  |  |  |  | 04 |  |  |
|  | Ton & Npi |  |  |  |  | 81 |  |  |
|  | Digits |  |  |  |  | 56 |  |  |
|  |  |  |  |  |  | 31 |  |  |
|  |  |  |  |  |  | 21 |  |  |
| Protocol Identifier | Protocol Identifier |  |  |  |  | 40 |  |  |
| Data Coding Scheme | Data Coding Scheme |  |  |  |  | F0 |  |  |
| Validity Period | Validity Period |  |  |  |  | 89 |  |  |
|  |  |  |  |  |  | 10 |  |  |
|  |  |  |  |  |  | 70 |  |  |
|  |  |  |  |  |  | 21 |  |  |
|  |  |  |  |  |  | 43 |  |  |
|  |  |  |  |  |  | 65 |  |  |
|  |  |  |  |  |  | 04 |  |  |
| TP-User Data | Length TP-User Data |  |  |  |  | 06 |  |  |
|  |  |  |  |  |  | 41 |  |  |
|  |  |  |  |  |  | E1 |  |  |
|  |  |  |  |  |  | 90 |  |  |
|  |  |  |  |  |  | 58 |  |  |
|  |  |  |  |  |  | 34 |  |  |
|  |  |  |  |  |  | 02 |  |  |

## Mobile Originated Short Message (SIM)

| Information Element Identifier | Message Structure |  |  |  | Example |
| --- | --- | --- | --- | --- | --- |
| SMS Status | status |  |  |  |  |
| Service Centre Address | Length Service Centre Address |  |  |  | 04 |
|  | Ton & Npi |  |  |  | 81 |
|  | Digits |  |  |  | 21 |
|  |  |  |  |  | 43 |
|  |  |  |  |  | 65 |
| TP-Message Type | Flags | Mti |  |  |  |
| TP-Message Reference | Message Reference |  |  |  |  |
| TP- Destination Address | Length TP-Destination Address |  |  |  | 04 |
|  | Ton & Npi |  |  |  | 81 |
|  | Digits |  |  |  | 56 |
|  |  |  |  |  | 31 |
|  |  |  |  |  | 21 |
| Protocol Identifier | Protocol Identifier |  |  |  | 40 |
| Data Coding Scheme | Data Coding Scheme |  |  |  | F0 |
| Validity Period | Validity Period |  |  |  | 89 |
|  |  |  |  |  | 10 |
|  |  |  |  |  | 70 |
|  |  |  |  |  | 21 |
|  |  |  |  |  | 43 |
|  |  |  |  |  | 65 |
|  |  |  |  |  | 04 |
| TP-User Data | Length TP-User Data |  |  |  | 06 |
|  |  |  |  |  | 41 |
|  |  |  |  |  | E1 |
|  |  |  |  |  | 90 |
|  |  |  |  |  | 58 |
|  |  |  |  |  | 34 |
|  |  |  |  |  | 02 |

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |