Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document - Confidential

GSM Protocol Stack

Message Sequence Charts

Supplementary Services

(Edit via File/Properties/Summary!)

| Document Number: | 6301.209.98.100 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.2 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1998-Aug-07 |
| Last changed: | 2003-Jun-12 by XINTEGRA |
| File Name: | ss.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1998-Aug-07 | Stefan Lemke et al. |  | 0.1 |  | 1 |
| 2003-Jun-10 | XINTEGRA |  | 0.2 | Draft |  |

Notes:

- Initial version

Table of Contents

1.1	References	3

1.2	Abbreviations	7

1.3	Terms	8

4.1	Connection Establishment	11

4.1.1	Mobile Originated Connection Establishment	11

4.1.2	Mobile Terminated Connection Establishment	13

4.2	Connected	14

4.2.1	Exchange of Facility Messages	14

4.3	Connection Release	15

4.3.1	Call Reestablishment	15

4.3.2	Release by Mobility Management	16

4.3.3	Mobile Originated Connection Release	17

4.3.4	Mobile Terminated Connection Release	18

A.	Acronyms	19

B.	Glossary	19

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18a]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat GmbH

[39]	Service Access Point MNCC
	6147.101.96.100; Condat GmbH

[40]	Service Access Point MNSS
	6147.102.96.100; Condat GmbH

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat GmbH

[42]	Service Access Point MMCC
	6147.104.97.100; Condat GmbH

[43]	Service Access Point MMSS
	6147.105.97.100; Condat GmbH

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat GmbH

[45]	Service Access Point RR
	6147.107.97.100; Condat GmbH

[46]	Service Access Point SIM
	6147.108.97.100; Condat GmbH

[47]	Service Access Point MPH
	6147.109.96.100; Condat GmbH

[48]	Service Access Point DL
	6147.110.96.100; Condat GmbH

[49]	Service Access Point MDL
	6147.111.96.100; Condat GmbH

[50]	Service Access Point PH
	6147.112.97.100; Condat GmbH

[51]	Service Access Point MMI
	6147.113.96.100; Condat GmbH

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat GmbH

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat GmbH

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat GmbH

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat GmbH

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat GmbH

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat GmbH

[58]	Users Guide
	6147.300.96.100; Condat GmbH

[59]	Test Specification CC
	6147.400.97.100; Condat GmbH

[60]	Test Specification SS
	6147.401.97.100; Condat GmbH

[61]	Test Specification SMS
	6147.402.97.100; Condat GmbH

[62]	Test Specification MM
	6147.403.97.100; Condat GmbH

[63]	Test Specification RR
	6147.404.97.100; Condat GmbH

[64]	Test Specification DL
	6147.405.97.100; Condat GmbH

[65]	Test Specification CCD
	6147.406.97.100; Condat GmbH

[66]	SDL Specification CC
	6147.500.97.100; Condat GmbH

[67]	SDL Specification SS
	6147.501.97.100; Condat GmbH

[68]	SDL Specification SMS
	6147.502.97.100; Condat GmbH

[69]	SDL Specification MM
	6147.503.97.100; Condat GmbH

[70]	SDL Specification RR
	6147.504.97.100; Condat GmbH

[71]	SDL Specification DL
	6147.505.97.100; Condat GmbH

[72]	Message Specification CC
	6147.600.97.100; Condat GmbH

[73]	Message Specification SS
	6147.601.97.100; Condat GmbH

[74]	Message Specification SMS
	6147.602.97.100; Condat GmbH

[75]	Message Specification MM
	6147.603.97.100; Condat GmbH

[76]	Message Specification RR
	6147.604.97.100; Condat GmbH

[77]	Message Specification DL
	6147.605.97.100; Condat GmbH

[78]	Technical Documentation CC
	6147.700.97.100; Condat GmbH

[79]	Technical Documentation SS
	6147.701.97.100; Condat GmbH

[80]	Technical Documentation SMS
	6147.702.97.100; Condat GmbH

[81]	Technical Documentation MM
	6147.703.97.100; Condat GmbH

[82]	Technical Documentation RR
	6147.704.97.100; Condat GmbH

[83]	Technical Documentation DL
	6147.705.97.100; Condat GmbH

[84]	Technical Documentation CCD
	6147.706.97.100; Condat GmbH

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The SIM card used by MM, MMI and SMS in several ways. This document describes the services needed for the SIM application.

# Overview

The protocol entity Supplementary Services (SS) is used to transport facility elements from and to the infrastructure for all call independent supplementary services.

It has the following states:

- IDLE	there is no existing connection

- MM CONNECTION PENDING	SS has requested a MM connection, but it is not confirmed

- CONNECTED	SS has a MM connection.

# Procedures

## Connection Establishment

### Mobile Originated Connection Establishment

MMI                    SS                   MM                    Um

|                     |                    |                    |

|  MNSS BEGIN REQ     |                    |                    |

*====================>*                    |                    |

|                  (SS 1)                  |                    |

|                     | MMSS ESTABLISH REQ |                    |

|                     *===================>*                    |

|                     |                 (MM 1)                  |

|                     |                    |                    |

a) establishment of MM connection is successful

|                     |                    |                    |

|                     | MMSS ESTABLISH CNF |                    |

|                     *===================>*                    |

|                  (SS 2)                  |                    |

|                     |    MMSS DATA REQ   |                    |

|                     |      (REGISTER)    |       REGISTER     |

|                     *===================>*===================>*

|                     |                 (MM 2)                  |

|                     |                    |                    |

b) establishment of MM connection fails

|                     |                    |                    |

|                     |  MMSS RELEASE IND  |                    |

|                     *===================>*                    |

|                  (SS 3)                  |                    |

|    MNSS END IND     |                    |                    |

*====================>*                    |                    |

(MMI 1)                 |                    |                    |

|                     |                    |                    |

(SS 1)

MMI starts establishment of a connection. A facility information element and the SS version info are forwarded to SS. SS checks the transaction identifier. It is not expected that this leads to a problem, because the MS shall make no internal errors. If this check fails, the request is ignored.

(MM 1)

SS stores the facility and the SS version information element and requests a MM connection by MM. The new state of SS is MM CONNECTION PENDING. In this state SS awaits the confirmation for the MM connection.

(SS 2)

MM has established a MM connection successfully and confirms this to SS.

(MM 2)

Using the stored information elements for facility and SS version, SS codes a SS REGISTER message and sends this message to the infrastructure. Any SS transaction starts with the SS REGISTER message. SS enters the state CONNECTED.

(SS 3)

MM was not able to establish a MM connection. This is signalled to SS.

(MMI 1)

MMI is informed about the release.

### Mobile Terminated Connection Establishment

MMI                    SS                   MM                    Um

|                     |                    |                    |

|                     | MMSS ESTABLISH IND |                    |

|                     |      (REGISTER)    |       REGISTER     |

|                     *<===================*<===================*

|                  (SS 1)                  |                    |

|                     |                    |                    |

a) message and transaction identifier ok

|                     |                    |                    |

|  MNSS BEGIN IND     |                    |                    |

*<====================*                    |                    |

(MMI 1)                 |                    |                    |

|                     |                    |                    |

b) message or transaction identifier not ok

|                     |                    |                    |

|                     |  MMSS RELEASE REQ  |                    |

|                     *===================>*                    |

|                     |                 (MM 1)                  |

|                     |                    |                    |

(SS 1)

SS receives a message. It must be a SS REGISTER message, because it is the first message of this transaction.

(MMI 1)

SS checks the message type and the transaction identifier. If it is a SS REGISTER message and the transaction identifier is not reserved and in the range for mobile terminated transactions, SS forwards the content (facility information) to MMI.

(MM 1)

If this check fails, SS ignores the message and releases the connection. MM is informed about this.

## Connected

### Exchange of Facility Messages

MMI                    SS                   MM                    Um

|                     |                    |                    |

|                     |   MMSS DATA IND    |                    |

|                     |     (FACILITY)     |       FACILITY     |

|                     *<===================*<===================*

|                  (SS 1)                  |                    |

|  MNSS FACILITY IND  |                    |                    |

*<====================*                    |                    |

(MMI 1)                 |                    |                    |

|                     |                    |                    |

|  MNSS FACILITY REQ  |                    |                    |

*====================>*                    |                    |

|                  (SS 2)                  |                    |

|                     |    MMSS DATA REQ   |                    |

|                     |     (FACILITY)     |       FACILITY     |

|                     *===================>*===================>*

|                     |                 (MM 1)                  |

|                     |                    |                    |

During a transaction (SS has the state CONNECTED) the mobile station and the infrastructure may exchange facility information elements with SS FACIILITY messages.

(SS 1)

The infrastructure sends a SS FACILITY message to the mobile station.

(MMI 1)

SS decodes the message and forwards the facility information element to MMI.

(SS 2)

MMI will send a facility information to the infrastructure without releasing the connection.

(MM 1)

SS code the facility information element into a SS FACILITY message and forwards it to the infrastructure.

## Connection Release

### Call Reestablishment

MMI                    SS                   MM                    Um

|                     |                    |                    |

|                     |   MMSS ERROR IND   |                    |

|                     *<===================*                    |

|                  (SS 1)                  |                    |

|     MNSS END IND    |                    |                    |

*<====================*                    |                    |

(MMI 1)                 |                    |                    |

|                     |  MMSS RELEASE REQ  |                    |

|                     *===================>*                    |

|                     |                 (MM 1)                  |

|                     |                    |                    |

(SS 1)

MM indicates an error in the lower layer with the possibility to reestablishment in the cell.

(MMI 1)

SS will not process reestablishment in general. MMI is informed about the end of the transaction.

(MM 1)

MM is informed about the end of transaction.

### Release by Mobility Management

MMI                    SS                   MM                    Um

|                     |                    |                    |

|                     |  MMSS RELEASE IND  |                    |

|                     *<===================*                    |

|                  (SS 1)                  |                    |

|     MNSS END IND    |                    |                    |

*<====================*                    |                    |

(MMI 1)                 |                    |                    |

|                     |                    |                    |

(SS 1)

MM indicates the connection release.

(MMI 1)

MMI is informed about the end of transaction.

### Mobile Originated Connection Release

MMI                    SS                   MM                    Um

|                     |                    |                    |

|    MNSS END REQ     |                    |                    |

*====================>*                    |                    |

|                  (SS 1)                  |                    |

|                     |    MMSS DATA REQ   |                    |

|                     | (RELEASE COMPLETE) |  RELEASE COMPLETE  |

|                     *===================>*===================>*

|                     |                 (MM 1)                  |

|                     |  MMSS RELEASE REQ  |                    |

|                     *===================>*                    |

|                     |                 (MM 2)                  |

|                     |                    |                    |

(SS 1)

MMI will release the transaction. The last message of a SS transaction is the SS RELEASE COMPLETE message.

(MM 1)

SS codes the message and forwards it to the infrastructure.

(MM 2)

MM is informed about the end of transaction.

### Mobile Terminated Connection Release

MMI                    SS                   MM                    Um

|                     |                    |                    |

|                     |    MMSS DATA IND   |                    |

|                     | (RELEASE COMPLETE) |  RELEASE COMPLETE  |

|                     *<===================*<===================*

|                  (SS 1)                  |                    |

|    MNSS END IND     |                    |                    |

*<====================*                    |                    |

(MMI 1)                 |                    |                    |

|                     |  MMSS RELEASE REQ  |                    |

|                     *===================>*                    |

|                     |                 (MM 1)                  |

|                     |                    |                    |

(SS 1)

The infrastructure will release the transaction. The last message of a SS transaction is the SS RELEASE COMPLETE message.

(MM 1)

SS decodes the message and forwards the content to MMI.

(MM 2)

MM is informed about the end of transaction.

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |