Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Documentation – Confidential

GSM Protocol Stack

MMI

(Edit via File/Properties/Summary!)

| Document Number: | 6147.708.97.100 |
| --- | --- |
| Version: | 0.2 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1997-Dec-11 |
| Last changed: | 2003-May-20 by XGUTTEFE |
| File Name: | mmi.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1997-Dec-11 | Stefan Lemke et al |  | 0.1 |  | 1 |
| 2003-May-20 | XGUTTEFE |  | 0.2 | Draft |  |

Notes:

- Initial version

Table of Contents

1.1	References	4

1.2	Abbreviations	7

1.3	Terms	9

2	Overview	9

3	Structure	11

4	Modules	13

5	Header	15

6	Configuration	18

6.1	Static Configuration	18

6.2	Dynamic Configuration	20

6.3	Custom Specific Functions	23

7	Monitoring	24

8	Resources	25

8.1	Timer	25

8.2	Memory	25

Appendices	26

A.	Acronyms	26

B.	Glossary	26

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	GSM 2.81, Line Identification Supplementary Services - Stage 1
	ETS 300 514, ETSI, September 1994

[2]	GSM 2.82, Call Forwarding Supplementary Services - Stage 1
	ETS 300 515, ETSI, September 1994

[3]	GSM 2.83, Call Waiting and Call Hold Supplementary Services - Stage 1
	ETS 300 516, ETSI, September 1994

[4]	GSM 2.84, Multi Party Supplementary Services - Stage 1
	ETS 300 517, ETSI, September 1994

[5]	GSM 2.85, Closed User Group Supplementary Services - Stage 1
	ETS 300 518, ETSI, September 1994

[6]	GSM 2.86, Advice of Charge Supplementary Services - Stage 1
	ETS 300 519, ETSI, September 1994

[7]	GSM 2.88, Call Barring Supplementary Services - Stage 1
	ETS 300 520, ETSI, September 1994

[8]	GSM 3.14, Support of Dual Tone Multi Frequency Signalling via the GSM System
	ETS 300 532, ETSI, April 1994

[9]	GSM 3.40, Technical Realization of the Short Message Service Point-to-Point
	ETS 300 536, ETSI, January 1996

[10]	GSM 3.41, Technical Realization of Short Message Service Cell Broadcast
	ETS 300 537, ETSI, June 1995

[11]	GSM 3.81, Line Identification Supplementary Services - Stage 2
	ETS 300 542, ETSI, February 1995

[12]	GSM 3.82, Call Forwarding Supplementary Services - Stage 2
	ETS 300 543, ETSI, February 1995

[13]	GSM 3.83, Call Waiting and Call Hold Supplementary Services - Stage 2
	ETS 300 544, ETSI, November 1994

[14]	GSM 3.84, Multi Party Supplementary Services - Stage 2
	ETS 300 545, ETSI, November 1994

[15]	GSM 3.85, Closed User Group Supplementary Services - Stage 2
	ETS 300 546, ETSI, January 1996

[16]	GSM 3.86, Advice of Charge Supplementary Services - Stage 2
	ETS 300 547, ETSI, March 1995

[17]	GSM 3.88, Call Barring Supplementary Services - Stage 2
	ETS 300 548, ETSI, November 1994

[18]	GSM 4.01, MS-BSS Interface General Aspects and Principles
	ETS 300 550, ETSI, September 1994

[18a]	GSM 4.03, MS-BSS Interface Channel Structures and Access Capabilities
	ETS 300 552, ETSI, September 1994

[19]	GSM 4.05, Data Link Layer General Aspects
	ETS 300 554, ETSI, September 1994

[20]	GSM 4.06, MS-BSS Interface Data Link Layer Specification
	ETS 300 555, ETSI, September 1994

[21]	GSM 4.07, Mobile Radio Interface Signalling Layer 3 General Aspects
	ETS 300 556, ETSI, February 1995

[22]	GSM 4.08, Mobile Radio Interface Layer 3 Specification
	ETS 300 557, ETSI, January 1996

[23]	GSM 4.10, Mobile Radio Interface Layer 3 Supplementary Services Specification 
	General Aspects
	ETS 300 558, ETSI, February 1995

[24]	GSM 4.11, Point-to-Point Short Message Service Support on Mobile Radio Interface
	ETS 300 559, ETSI, October 1995

[25]	GSM 4.12, Short Message Service Cell Broadcast Support on Mobile Radio Interface
	ETS 300 560, ETSI, January 1996

[26]	GSM 4.80, Mobile Radio Interface Supplementary Services Specification Formats and Coding
	ETS 300 564, ETSI, February 1995

[27]	GSM 4.81, Line Identification Supplementary Services - Stage 3
	ETS 300 565, ETSI, February 1995

[28]	GSM 4.82, Call Forwarding Supplementary Services - Stage 3
	ETS 300 566, ETSI, February 1995

[29]	GSM 4.83, Call Waiting and Call Hold Supplementary Services - Stage 3
	ETS 300 567, ETSI, February 1995

[30]	GSM 4.84, Multi Party Supplementary Services - Stage 3
	ETS 300 568, ETSI, February 1995

[31]	GSM 4.85, Closed User Group Supplementary Services - Stage 3
	ETS 300 569, ETSI, February 1995

[32]	GSM 4.86, Advice of Charge Supplementary Services - Stage 3
	ETS 300 570, ETSI, February 1995

[33]	GSM 4.88, Call Barring Supplementary Services - Stage 3
	ETS 300 571, ETSI, February 1995

[34]	GSM 5.01, Physical Layer on the Radio Path General Description
	ETS 300 573, ETSI, October 1995

[35]	GSM 5.02, Multiplexing and Multiple Access on the Radio Path
	ETS 300 574, ETSI, January 1996

[36]	GSM 5.08, Radio Sub-system Link Control
	ETS 300 578, ETSI, January 1996

[37]	GSM 5.10, Radio Sub-system Synchronisation
	ETS 300 579, ETSI, October 1995

[38]	Service Access Point MMREG
	6147.100.96.100; Condat GmbH

[39]	Service Access Point MNCC
	6147.101.96.100; Condat GmbH

[40]	Service Access Point MNSS
	6147.102.96.100; Condat GmbH

[41]	Service Access Point MNSMS
	6147.103.96.100; Condat GmbH

[42]	Service Access Point MMCC
	6147.104.97.100; Condat GmbH

[43]	Service Access Point MMSS
	6147.105.97.100; Condat GmbH

[44]	Service Access Point MMSMS
	6147.106.97.100; Condat GmbH

[45]	Service Access Point RR
	6147.107.97.100; Condat GmbH

[46]	Service Access Point SIM
	6147.108.97.100; Condat GmbH

[47]	Service Access Point MPH
	6147.109.96.100; Condat GmbH

[48]	Service Access Point DL
	6147.110.96.100; Condat GmbH

[49]	Service Access Point MDL
	6147.111.96.100; Condat GmbH

[50]	Service Access Point PH
	6147.112.97.100; Condat GmbH

[51]	Service Access Point MMI
	6147.113.96.100; Condat GmbH

[52]	Message Sequence Charts CC
	6147.200.97.100; Condat GmbH

[53]	Message Sequence Charts SS
	6147.201.97.100; Condat GmbH

[54]	Message Sequence Charts SMS
	6147.202.97.100; Condat GmbH

[55]	Message Sequence Charts MM
	6147.203.97.100; Condat GmbH

[56]	Message Sequence Charts RR
	6147.204.96.100; Condat GmbH

[57]	Message Sequence Charts DL
	6147.205.96.100; Condat GmbH

[58]	Users Guide
	6147.300.96.100; Condat GmbH

[59]	Test Specification CC
	6147.400.97.100; Condat GmbH

[60]	Test Specification SS
	6147.401.97.100; Condat GmbH

[61]	Test Specification SMS
	6147.402.97.100; Condat GmbH

[62]	Test Specification MM
	6147.403.97.100; Condat GmbH

[63]	Test Specification RR
	6147.404.97.100; Condat GmbH

[64]	Test Specification DL
	6147.405.97.100; Condat GmbH

[65]	Test Specification CCD
	6147.406.97.100; Condat GmbH

[66]	SDL Specification CC
	6147.500.97.100; Condat GmbH

[67]	SDL Specification SS
	6147.501.97.100; Condat GmbH

[68]	SDL Specification SMS
	6147.502.97.100; Condat GmbH

[69]	SDL Specification MM
	6147.503.97.100; Condat GmbH

[70]	SDL Specification RR
	6147.504.97.100; Condat GmbH

[71]	SDL Specification DL
	6147.505.97.100; Condat GmbH

[72]	Message Specification CC
	6147.600.97.100; Condat GmbH

[73]	Message Specification SS
	6147.601.97.100; Condat GmbH

[74]	Message Specification SMS
	6147.602.97.100; Condat GmbH

[75]	Message Specification MM
	6147.603.97.100; Condat GmbH

[76]	Message Specification RR
	6147.604.97.100; Condat GmbH

[77]	Message Specification DL
	6147.605.97.100; Condat GmbH

[78]	Technical Documentation CC
	6147.700.97.100; Condat GmbH

[79]	Technical Documentation SS
	6147.701.97.100; Condat GmbH

[80]	Technical Documentation SMS
	6147.702.97.100; Condat GmbH

[81]	Technical Documentation MM
	6147.703.97.100; Condat GmbH

[82]	Technical Documentation RR
	6147.704.97.100; Condat GmbH

[83]	Technical Documentation DL
	6147.705.97.100; Condat GmbH

[84]	Technical Documentation CCD
	6147.706.97.100; Condat GmbH

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Response
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The GSM specification do not set out any implementation of the Protocol Stack. The following diagrams show the implementation described in all these documents for the mobile station. All entities except the Man Machine Interface and Physical Layer are implemented as part of the Protocol Stack.

Figure 1: Mobile-station protocol architecture

This document is the technical documentation for the Man Machine Interface (MMI).

# Structure

Figure 3: Structure of Protocol Stack Entity

Each protocol stack entity has the same structure and is build by the following modules

- Frame

- Protocol Stack Entity

- Virtual System Interface and

- Condat Coder Decoder

Frame

The frame carries out the work depending on the operating system, like startup or shutdown of a entity. This module is implemented by the customer and linked to the protocol stack entity. The interface between both is the protocol stack entity interface (PEI).

Protocol Stack Entity

The protocol stack entities are implemented without any dependencies on the underlying operating system. And they contain the protocol stack functionality as described into the ETSI Recommendations. They are sub-divided in the three modules protocol stack entity interface, custom specific functions and  finite state machine.

The protocol stack interface module defines the interface to the frame. Custom specific functions are used to configure the protocol stack at run-time with custom specific data from a non-erasable memory for example. The finite state machine modules implement the protocol stack logic.

Virtual System Interface

The protocol stack entities requires operation system functionality like communication and timer. The requirements are low and independent from the operating system, so the system interface is virtual from the view of protocol stack entities. The module virtual system interface implements this interface. This implementation is carried out by pre-processor definitions or by function calls.

Condat Coder Decoder

GSM message at the air-interface are bit streams. The most efficient method to handle this GSM messages into a protocol stack entity is to convert this bit streams into corresponding C-Structures. This is done by the module Condat Coder Decoder. It makes sense to integrate this module as a server into the target system because this module is needed by more than one entity.

Figure 4: Generating a Protocol Stack Entity

Generating a protocol stack entity is carried out in two steps:

- Generating primitives and message structures

- Compiling / linking  for target system

The message definition files (MDF) and primitive definition files (PDF) contain a description for all messages and primitives independent from a target system. The CCD-Compiler generates target dependent header files and tables. Parameters of the CCD-Compiler are Alignment and Byte-ordering of the target system.

The generated header files and the delivered header and source files are compiled and linked for the target system to obtain a executable for the target system.

# Modules

The protocol stack entity DL for the mobile station is divided in the following modules:

- dl_ccch.c

- dl_com.c

- dl_csf.c

- dl_dph.c

- dl_drr.c

- dl_facch.c

- dl_pei.c

- dl_sacch.c

- dl_sdcc3.c

- dl_sdcch.c

- dl_tim.c

- tok.c

dl_ccch.c

This module defines the message handling for the common control channels and the cell broadcast control channel. The received message format from PL is converted to the expected format of RR.

dl_com.c

This module defines common used functions for building frames etc.

dl_csf.c

The customer specific functions are carried out in this module. The module contains the functions needed for the timer pool.

dl_dph.c

Incoming frames of the PL are checked and distributed by this module. Errors are detected and MM is informed.

dl_drr.c

Incoming primitives from RR are checked and distributed according to the given channel and service access point identifier.

dl_facch.c

The protocol for channel FACCH and service access point identifier 0 is located in this module.

dl_pei.c

The module implements the interface to the process frame. That means initialising of protocol stack entity data, open of communication and timer resources and routing of incoming primitives. Handling of time-outs, dynamic configuration and monitoring are located in this module.

dl_sacch.c

The protocol for channel SACCH and service access point identifier 0 is located in this module. Additionally reception and sending of unacknowledged data is processed by the module.

dl_sdcc3.c

The protocol for channel SDCCH and service access point identifier 3 is located in this module.

dl_sdcch.c

The protocol for channel SDCCH and service access point identifier 0 is located in this module.

dl_tim.c

The module contains functions for the timer handling. That means allocation of a timer from the timer pool, start, stop of configure of a timer and release of an allocated timer.

tok.c

This module defines some parse functions for the dynamic configuration strings.

# Header

The modules include several header files. Header files which are changeable by the user are marked (*). These header files are used to integrate the protocol stack entities in a specific target system.

- ccdapi.h (*)

- dl.h

- cnf_dl.h (*)

- custom.h (*)

- cus_dl.h (*)

- gsm.h

- mconst.cdg

- message.h

- mon_dl.h

- pconst.cdg

- pei.h (*)

- prim.h

- p_ph.h

- p_dl.h

- p_mdl.h

- stddefs.h

- string.h

- tok.h

- vsi.h (*)

ccdapi.h

The header defines the prototypes and some constants for the Condat Coder Decoder (CCD).

dl.h

The header contains constants for the data link layer and the prototypes of the component.

cnf_dl.h

Constants for dynamic configuration of DL are defined in this header. It is allowed to change the commands and the parameter names for the supported dynamic configurations.

custom.h

The header defines global constants for the integration of the protocol stack entity into a specific target system. The user may define the identifier of the communication resource, the supported traces, the communication method (by copying primitives or by exchanging references of primitives) , the custom specific primitive header and some more.

cus_dl.h

Custom specific definitions for the protocol stack entity are located in this header. Timer values and identifier are changeable. A version identifier is defined.

gsm.h

The header contains global definitions for all protocol stack entities. Depending on the definitions in custom.h many options and traces are defines in this header.

mconst.cdg

This header is generated by the CCD compiler. It includes all message identifier and some constants needed by the entities.

message.h

Constants for messages are defined. DL messages are coded and decoded by low level functions of CCD.

mon_dl.h

Constants for monitoring of DL are defined in this header.

pconst.cdg

This header is generated by the CCD compiler. It includes all primitive identifier and some constants needed by the entities.

pei.h

Prototypes for the protocol stack entity interface are defined in this header. Some parameter and the return type of this function are changeable by the user.

prim.h

Constants for primitives are defined and service access point dependent primitive header files are included (p_ph.h., p_dl.h and p_mdl.h).

p_dl.h

This header is generated by the CCD compiler. It includes the C-struct type definitions for primitives of the service access point DL. The header is included by prim.h

p_mdl.h

This header is generated by the CCD compiler. It includes the C-struct type definitions for primitives of the service access points MDL. The header is included by prim.h

p_ph.h

This header is generated by the CCD compiler. It includes the C-struct type definitions for primitives of the service access points PH. The header is included by prim.h

stddefs.h

The header contains several standard definitions used by the protocol stack entities.

string.h

This header is a the standard string header from the target compiler. It defines string and memory functions.

tok.h

The prototypes and some constants for the parse function of the TOK module are defined in this header.

vsi.h

Prototypes for the virtual system interface are defined in this header. Some parameter and the return types of these function are changeable by the user for integration in a specific target system.

# Configuration

The protocol stack software is independent from a specific target system. The step from the independent protocol stack software to a target system is carried out by configuration and by custom specific functions. There are three ways to configure the protocol stack entity:

- static configuration

- dynamic configuration

- custom specific functions

## Static Configuration

Static configuration is carried out by setting constants or types in some header files before compiling the sources. The following header files are changeable by the user:

- ccdapi.h

- cnf_dl.h

- custom.h

- cus_dl.h

- pei.h

- vsi.h

Some configurations are valid for all protocol stack entities (defined in ccdapi.h, custom.h, pei.h and vsi.h). They are described in the Users guide. The entity specific configurations are defined in cnf_dl.h and cus_dl.h and described in the following:

TIMER_VALUES (cus_dl.h)

The constants define the timer values for the DL timer T200 depending on the various channel types and service access point identifiers. If your target system uses other units please change the values.

TIMER IDENTIFIER (cus_dl.h)

The constants define the identifier of the timer resources of DL.

MAX_DL_TIMER (cus_dl.h)

The constant define the number of timer available in the timer pool.

PRIMITIVE SIZES (cus_dl.h)

The primitive size of PH_DATA_REQ depends on the used channel. It is defined separately to achieve an optimum of transmitted bytes to physical layer. The default value which is defined here is calculated without any alignment bits.

E.g. SACCH	N201	=	18
	l2 header	=	3
	sdu offset	=	2
	sdu length	=	2
	ch_type	=	1
	dummy	=	1
	----------------------
	total	=	27

FRAME_OFFSET_ZERO (cus_dl.h)

If the constant is set, PL ensures that all frames are send with offset zero to DL. This enables a faster access on frame header parameter.

MAX_QUEUED_MESSAGES (cus_dl.h)

The constant define the maximum number of stored layer 3 messages in data link layer.

VERSION (cus_dl.h)

The constants define the type and the value of a version identification. The version is part of the monitor struct.

CONFIGURATION PARAMETER (cnf_dl.h)

The constants define the commands for dynamic configuration proposals.

## Dynamic Configuration

Dynamic configuration means to change the behaviour of the protocol stack entity at run-time. This is carried out by sending a string with a dedicated format as described in the Users Guide. An additionally feature is to request the old configuration. This feature is may be switched off by an option defined in custom.h.

The dynamic configuration string is a parameter of the pei_config () function which is part of the protocol stack entity interface (PEI). The keywords for the dynamic configurations are changeable by the static configuration mechanism.

RESET

The dynamic configuration RESET is used to make a warm start of DL belonging the GSM functionality.

TIMER_SET =<timer, value>

The timer mode TIMER_SET defines a new timer value instead of the origin start value.

TIMER_RESET =timer

The default timer mode is TIMER_RESET which affects in no manipulation of the start value.

TIMER_SPEED_UP =<timer, factor>

TIMER_SPEED_UP is used to speed up a timer by the given factor. The start value is divided by the factor. The minimum time is one unit.

TIMER_SLOW_DOWN = <timer, factor>

The opposite mode is TIMER_SLOW_DOWN. The start value is increased by the given factor.

TIMER_SUPPRESS = <timer>

TIMER_SUPPRESS is used to suppress the timer start.

N200 = <chan_sapi, value>

The configuration N200 is used to redefine the retransmission counter N200. The default value depends on the used channel.

VS = <chan_sapi, value>	single instance system
VS = <inst, chan_sapi, value>	multi instance system

The configuration VS manipulates the send variable V(S) on the defined channel. For multi instance systems this configuration is carried out only for the defined instance. This is used to force error situations on data link layer.

VR = <chan_sapi, value>	single instance system
VR = <inst, chan_sapi, value>	multi instance system

The configuration VR manipulates the receive variable V(R) on the defined channel. For multi instance systems this configuration is carried out only for the defined instance. This is used to force error situations on data link layer.

VA = <chan_sapi, value>	single instance system
VA = <inst, chan_sapi, value>	multi instance system

The configuration VA manipulates the acknowledge variable V(A) on the defined channel. For multi instance systems this configuration is carried out only for the defined instance. This is used to force error situations on data link layer.

UA_IGNORE = <chan_sapi, value>	single instance system
UA_IGNORE = <inst, chan_sapi, value>	multi instance system

To force repetition of sending SABM or DISC frames it is possible to ignore incoming UA frames on the defined channel. The number of ignored frames is defined by the parameter value. For multi instance systems this configuration is carried out only for the defined instance.

RR_IGNORE = <chan_sapi, value>	single instance system
RR_IGNORE = <inst, chan_sapi, value>	multi instance system

To force repetition of sending I frames it is possible to ignore incoming RR frames on the defined channel. The number of ignored frames is defined by the parameter value. For multi instance systems this configuration is carried out only for the defined instance.

I_IGNORE = <chan_sapi, value>	single instance system
I_IGNORE = <inst, chan_sapi, value>	multi instance system

To force repetition of received frames it is possible to ignore incoming I frames on the defined channel. The number of ignored frames is defined by the parameter value. For multi instance systems this configuration is carried out only for the defined instance.

ADDR_FIELD = value	single instance system
ADDR_FIELD = <inst, value>	multi instance system

To force error situation it is possible to create a frame content, which is send with the right timing on the defined channel (see configuration SEND_FRAME). With this configuration it is possible to define the address field of this frame. For multi instance systems this configuration is carried out only for the defined instance.

CTRL_FIELD = value	single instance system
CTRL_FIELD = <inst, value>	multi instance system

To force error situation it is possible to create a frame content, which is send with the right timing on the defined channel (see configuration SEND_FRAME). With this configuration it is possible to define the control field of this frame. For multi instance systems this configuration is carried out only for the defined instance.

LENGTH_FIELD = value	single instance system
LENGTH_FIELD = <inst, value>	multi instance system

To force error situation it is possible to create a frame content, which is send with the right timing on the defined channel (see configuration SEND_FRAME). With this configuration it is possible to define the length field of this frame. For multi instance systems this configuration is carried out only for the defined instance.

INFO_FIELD = value	single instance system
INFO_FIELD = <inst, value>	multi instance system

To force error situation it is possible to create a frame content, which is send with the right timing on the defined channel (see configuration SEND_FRAME). With this configuration it is possible to define the information field of this frame. For multi instance systems this configuration is carried out only for the defined instance.

SEND_FRAME = chan	single instance system
SEND_FRAME = <inst, chan>	multi instance system

To force error situation it is possible to create a frame content, which is send with the right timing on the defined channel. For multi instance systems this configuration is carried out only for the defined instance.

## Custom Specific Functions

Custom specific functions are implemented in the module dl_csf.c. It is allowed to replace the functions in this module by functions of the customer. It is not allowed to change parameters of the functions.

The sense of this custom specific functions is to have a mechanism to configure the protocol stack entity at run-time by a source outside the protocol stack entity, for example a non erasable memory.

GLOBAL BOOL csf_init_timer (void)

The function initialises the timer pool. The timer pool allocates a number of timer resources. This timer resources are allocated to instances on demand.

GLOBAL void csf_close_timer (void)

All timer resources are closed. This function is carried during shutdown or reset.

GLOBAL void csf_alloc_timer (UBYTE id, T_DL_DATA * dl_data, T_VSI_TVALUE value)

The function allocates one timer of the timer pool and starts the timer.

GLOBAL void csf_free_timer (T_VSI_THANDLE handle)

The function frees one timer. The timer is stopped and given back to the timer pool.

GLOBAL BOOL csf_vdb_timeout (T_VSI_THANDLE handle, T_DL_DATA ** dl_data, 
USHORT * timer)

After time-out the according instance is searched. The timer is given back to the timer pool.

# Monitoring

The monitor struct includes relevant physical parameter of the protocol stack entity. The parameters are updated at every time. By this way the environment has always the possibility of access to parameters of the protocol stack. This parameters are used to create monitor reports to a display or a test system, to create statistical data or other things outside the functionality of a protocol stack but with access to protocol stack parameters. It is allowed to read the parameters of the monitor struct and it is strictly forbidden to write to the monitor struct. The first parameter of the monitor struct is the version of the protocol stack entity.

For the protocol stack entity the following monitor struct is defined:

typedef struct

{

T_VERSION	*	version;

} T_MONITOR;

# Resources

## Timer

Four timers are defined for each DL instance. In parallel a maximum of two timers per instance is possible (to control service access point 0 and 3).

## Memory

The following sizes are measured with a TURBO-C 2.0 Compiler for a INTEL-PC. Options and traces are set as defined in the header at delivery. The size in a target system depends on the controller, the operating system, the compiler and the used options. Consider that a lot of trace output is included at delivery.

| Module | Code | Data | Bss |
| --- | --- | --- | --- |
| DL_CCCH.C | 208 | 91 | 0 |
| DL_COM.C | 3019 | 416 | 0 |
| DL_CSF.C | 784 | 88 | 6 |
| DL_DPH.C | 4264 | 401 | 0 |
| DL_DRR.C | 1006 | 197 | 0 |
| DL_FACCH.C | 6768 | 410 | 0 |
| DL_PEI.C | 2373 | 1670 | 1970 |
| DL_SACCH.C | 6086 | 378 | 0 |
| DL_SDCC3.C | 6127 | 334 | 0 |
| DL_SDCCH.C | 6786 | 429 | 0 |
| DL_TIM.C | 836 | 227 | 26 |
| Total | 38257 | 4641 | 2002 |

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |