Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

GSM Fax & Data Services

Test Specification

ACIDTI

(Edit via File/Properties/Summary!)

| Document Number: | 8411.417.02.101 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.3 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 2002-Aug-15 |
| Last changed: | 2003-May-12 by XGUTTEFE |
| File Name: | acidti.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 2002-Aug-15 | TLU |  | 0.1 |  | 1 |
| 2003-Jan-21 | RM |  | 0.2 |  | 2 |
| 2003-May-12 | XGUTTEFE |  | 0.3 | Draft |  |

Notes:

- Initial version

- add testcases 080-084

Table of Contents

1.1	References	5

1.2	Abbreviations	8

1.3	Terms	10

2	Overview	10

2.1	RA - Rate Adaptation	10

2.2	RLP - Radio Link Protocol	11

2.3	L2R - Layer 2 Relay Functionality	11

2.4	FAD 03.45 - Fax Adaptation Protocol	11

2.5	T.30 - Fax Protocol Entity	11

2.6	ACI - AT Command Interpreter	11

2.7	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	11

3	Parameters	11

4	TEST CASES	25

4.1	Initialisation	25

4.1.1	ACIDTI000: Preamble	25

4.1.2	ACIDTI001: UART Init	26

4.1.3	ACIDTI002: Deactivate echoing	28

4.2	Simple Command Handling	29

4.2.1	ACIDTI011: Receive a command	29

4.2.2	ACIDTI012: Repeat last command	31

4.2.3	ACIDTI013: Receive wrong command.	31

4.3	Multiplexer Mode	32

4.3.1	ACIDTI021: Start multiplexer	32

4.3.2	ACIDTI022: Start a second channel - multiplexer	35

4.3.3	ACIDTI023: Send a command ("AT") on second channel	36

4.3.4	ACIDTI024: Try with AT+CFUN on the second channel	37

4.3.5	ACIDTI025: Try to start a second channel - get error when asking for a DTI channel	40

4.3.6	ACIDTI026: Start multiplexer and 2 channels - get error	41

4.3.7	ACIDTI027: Close multiplexer	44

4.4	Transparent Data Transfer	46

4.4.1	ACIDTI031: Transparent Data call over DTI (part 1)	46

4.4.2	ACIDTI032: Transparent Data call over DTI (part 2)	49

4.4.3	ACIDTI033: Network breaks communication - Disconnect Ind	50

4.4.4	ACIDTI034: Network breaks communication - Release Ind	53

4.4.5	ACIDTI035: TRA gives channel back to ACI when an escape sequence detected	56

4.4.6	ACIDTI036: User terminates call (ATH)	57

4.4.7	ACIDTI037: Return to data mode (ATO)	59

4.4.8	ACIDTI038: DTR detected (AT&D0)	60

4.4.9	ACIDTI039: DTR detected (AT&D1)	61

4.4.10	ACIDTI040: DTR detected (AT&D2)	64

4.4.11	ACIDTI041: Escape sequence detected and ATO (flush data from send queue)	67

4.5	Non-Transparent Data Transfer	70

4.5.1	ACIDTI051: Non-Transparent Data call over DTI (part 1)	70

4.5.2	ACIDTI052: Non-Transparent Data call over DTI (part 2)	74

4.5.3	ACIDTI053: L2R breaks communication	75

4.5.4	ACIDTI054: Network breaks communication - Disconnect Ind	77

4.5.5	ACIDTI055: Network breaks communication - Release Ind	80

4.5.6	ACIDTI056: LR2 gives channel back to ACI when an escape sequence detected	82

4.5.7	ACIDTI057: User terminates call (ATH)	84

4.5.8	ACIDTI058: Return to data mode (ATO)	86

4.5.9	ACIDTI059: DTR detected (AT&D1)	88

4.5.10	ACIDTI060: DTR detected (AT&D2)	90

4.6	FAX Transfer	93

4.6.1	ACIDTI071: MO FAX call (transmit 2 pages complete)	93

4.6.2	ACIDTI072: MT FAX call (receive 2 pages complete)	105

4.7	AT commands to data flow	117

4.7.1	ACIDTI080: Entity PKTIO registers in ACI, successfull	117

4.7.2	ACIDTI081:Set Data Flow for packet data	118

4.7.3	ACIDTI082:Query Data Flow for packet data	119

4.7.4	ACIDTI083:Query Data Flow for packet data, no redirection has been set	120

4.7.5	ACIDTI084: Show data channel information for current channel	121

Appendices	122

A.	Acronyms	122

B.	Glossary	122

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[1]	Rec. T.4 Standardisation of group 3 facsimile apparatus for document transmission;
(CCITT-T.4, 1984)

[2]	ITU-T Recommendation T.30; Series T: Terminal equipments and protocols for telematic services;
Procedures for document facsimile transmission in the general switched
telephone network;
(ITU-T.30, 1996)

[3]	ITU-T Recommendation T.31; Terminals for telematic services;
Asynchronous facsimile DCE control - service class 1
(ITU-T.31, 1995)

[4]	ITU-T Recommendation T.32; Terminals for telematic services;
Asynchronous facsimile DCE control - service class 2
(ITU-T.32, 1995)

[5]	Rec. T.35; Terminal equipment and protocols for telematic services;
Procedures for the allocation of CCITT definde codes for non-standard facilities;
(CCITT-T.35, 1991)

[6]	ITU-T Recommendation V.25 ter; Series V: data communication over the telephone network;
Interfaces and voiceband modems; Serial asynchronous automatic dialling and control
(ITU-T V.25 ter, 1997)

[7]	Rec. V.42 bis Data compression procedures for data circuit terminating equipment (DCE) using error correction procedures;
(CCITT-V.42 bis, 1990)

[8]	Rec. V.110 (Blue book, Vol. VIII, Fascicle VIII.1) Support of data terminal equipments (DTEs) with V-series type interfaces by an integrated services digital network (ISDN);
(CCITT-V.110, 1988)

[9]	European digital cellular telecommunications system (Phase 2);
GSM Public Land Mobile Network (PLMN) connection types;
(GSM 3.10, September 1994, version 4.3.1)

[10]	European digital cellular telecommunications system (Phase 2);
Technical realisation of facsimile group 3 transparent;
(GSM 3.45, September 1995, version 4.5.0)

[11]	Digital cellular telecommunications system (Phase 2);
Mobile radio interface layer 3 specification;
(GSM 4.08, November 1996, version 4.17.0)

[12]	European digital cellular telecommunications system (Phase 2);
Rate adaptation on the Mobile Station - Base Station System (MS - BSS) Interface;
(GSM 4.21, May 1995, version 4.6.0)

[13]	European digital cellular telecommunications system (Phase 2);
Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-service Switching Centre (BSS - MSC) interface
(GSM 4.22, September 1994, version 4.3.0)

[14]	European digital cellular telecommunications system (Phase 2);
Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-service Switching Centre (BSS - MSC) interface
(Amendment prA1 for GSM 4.22, version 4.3.0)
(GSM 4.22, March 1995, version 4.4.0)

[15]	European digital cellular telecommunications system (Phase 2);
General on Terminal Adaptation Functions (TAF) for Mobile Stations (MS);
(GSM 7.01, December 1995, version 4.10.0)

[16]	European digital cellular telecommunications system (Phase 2);
Terminal Adaptation Functions (TAF) for services using asynchronous bearer capabilities;
(GSM 7.02, September 1994, version 4.5.1)

[17]	European digital cellular telecommunications system (Phase 2);
Terminal Adaptation Functions (TAF) for services using synchronous bearer capabilities;
(GSM 7.03, September 1994, version 4.5.1)

[18]	Digital cellular telecommunications system (Phase 2);
Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Services (CBS);
(GSM 7.05, November 1996, version 4.8.0)

[19]	Digital cellular telecommunications system (Phase 2);
AT command set for GSM Mobile Equipment (ME)
(GSM 7.07, May 1996, version 4.1.0)

[20]	Digital cellular telecommunication system (Phase 2);
Mobile Station (MS) conformance specification;
Part 1: Conformance specification
(GSM 11.10-1, November 1996, version 4.17.0)

[21]	Digital cellular telecommunications system (Phase 2);
Mobile Station (MS) conformance specification;
Part 2: Protocol Implementation Conformance Statement (PICS)
proforma specification
(GSM 11.10-2, May 1996, version 4.15.0)

[22]	Digital cellular telecommunications system (Phase 2);
Mobile Station (MS) conformance specification;
Part 3: Layer 3 (L3) Abstract Test Suite (ATS)
(GSM 11.10-3, November 1996, version 4.17.0)

[23]	Proposal for Rate Adaptation implemented on a DSP;
(C. Bianconi, Texas Instruments, January 1998, version 1.0)

[24]	MCU-DSP Interfaces for Data Applications;
Specification S844
(C. Bianconi, Texas Instruments, March 1998, version 0.1)

[25]	Users Guide
6147.300.96.100; Condat GmbH

[26]	Service Access Point RA
8411.100.98.100; Condat GmbH

[27]	Service Access Point RLP
8411.101.98.100; Condat GmbH

[28]	Service Access Point L2R
8411.102.98.100; Condat GmbH

[29]	Service Access Point FAD
8411.103.98.100; Condat GmbH

[30]	Service Access Point T30
8411.104.98.100; Condat GmbH

[31]	Service Access Point ACI
8411.105.98.100; Condat GmbH

[32]	Message Sequence Charts RLP
8411.201.98.100; Condat GmbH

[33]	Message Sequence Charts L2R
8411.202.98.100; Condat GmbH

[34]	Message Sequence Charts FAD
8411.203.98.100; Condat GmbH

[35]	Message Sequence Charts T30
8411.204.98.100; Condat GmbH

[36]	Message Sequence Charts ACI
8411.205.98.100; Condat GmbH

[37]	Proposal for Fax & Data Integration; March 1998
8411.300.98.100; Condat GmbH

[38]	Test Specification RLP
8411.401.98.100; Condat GmbH

[39]	Test Specification L2R
8411.402.98.100; Condat GmbH

[40]	Test Specification FAD
8411.403.98.100; Condat GmbH

[41]	Test Specification T30
8411.404.98.100; Condat GmbH

[42]	Test Specification ACI
8411.405.98.100; Condat GmbH

[43]	SDL Specification RLP
8411.501.98.100; Condat GmbH

[44]	SDL Specification L2R
8411.502.98.100; Condat GmbH

[45]	SDL Specification FAD
8411.503.98.100; Condat GmbH

[46]	SDL Specification T30
8411.504.98.100; Condat GmbH

[47]	SDL Specification ACI
8411.505.98.100; Condat GmbH

[48]	Technical Documentation RLP
8411.701.98.100; Condat GmbH

[49]	Technical Documentation L2R
8411.702.98.100; Condat GmbH

[50]	Technical Documentation FAD
8411.703.98.100; Condat GmbH

[51]	Technical Documentation T30
8411.704.98.100; Condat GmbH

[52]	Technical Documentation ACI
8411.705.98.100; Condat GmbH

## Abbreviations

ACI	AT Command Interpreter
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BCS	Binary Coded Signals
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface
EOL	End Of Line

F	Final Bit
F&D	Fax and Data Protocol Stack
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HDLC	High level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LISR	Low level Interrupt Service Routine
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MSG	Message phase in the GSM 3.45 protocol

N®	Receive Number
N(S)	Send Number
NCC	National Colour Code
NECI	New Establishment Causes included

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Description Unit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

- T.4	CCITT Standardisation for Document coding of Group 3 Facsimile Apparatus
TAP	Test Application Program
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

V(A)	Acknowledgement State Variable
V®	Receive State Variable
V(S)	Send State Variable
VPLMN	Visiting Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for fax and data transmission consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity. The entity, which is described in this document, is coloured grey in the following figure :

- Figure 2-1: Architecture of the fax and data protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the fax and data protocol stack are:

## RA - Rate Adaptation

This entity performs an adaptation between an asynchronous or synchronous data stream with several bit rates on to the fixed bit rate used at the TCH. This is performed by the rate adaptation functions RA1’ and RA0 described in GSM 04.21.

## RLP - Radio Link Protocol

This entity provides a Layer 2 protocol for asynchronous reliable data transfer as specified in GSM 04.22. It includes error correction, sequence numbers and a mechanism for repeating corrupted and lost messages.

## L2R - Layer 2 Relay Functionality

The L2R provides relay functions in order to adapt the character-oriented data received from the TE via USART to the bit-oriented RLP protocol.

## FAD 03.45 - Fax Adaptation Protocol

The fax adaptation protocol, as specified in GSM 03.45, provides synchronisation with the BCS and MSG modems of the peer entity. It uses byte repetition in conjunction with a voting algorithm to handle corruption on the TCH data stream. The non-transparent fax protocol in accordance with GSM 03.46 is not part of this implementation.

The fax adapter enables T.30 to send BCS at 300 BPS and T.4 MSG in 2400, 4800, 7200 and 9600 BPS.

## T.30 - Fax Protocol Entity

The protocol uses binary coded signals packed in HDLC frames to set up and release a connection in the message phase of the FAX transmission. This entity is specified in the ITU-T.30. The main tasks of this unit are:

- Building the HDLC frames with CRC.

- Performing bit stuffing/de-stuffing.

- Executing a sequence of 5 phases: 1.) set up, 2.) pre-message procedures, 3.) transmission/reception, 4.) post message procedures, 5.) waiting for call release.

## ACI - AT Command Interpreter

The ACI is specified in GSM 07.07. It is responsible for call establishment via the GSM voice protocol stack and terminal adaptation for asynchronous transparent character-oriented data transmission. The ACI is able to receive AT commands and send the replies over the USART driver to a remote PC. This makes it possible to control the voice and data protocol stack from a remote application running on a PC. The ACI also provides a unique interface for an internal MMI in the MS.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the F&D. The driver has to be able to perform flow control.

# Parameters

/* array declarations */

DECLARATION (F_AT_BLUP)
DECLARATION (F_AT_E0)
DECLARATION (F_AT_E1)
DECLARATION (F_AT_CMUX)
DECLARATION (F_AT_AND_D0)
DECLARATION (F_AT_AND_D1)
DECLARATION (F_AT_AND_D2)
DECLARATION (F_AT)
DECLARATION (F_AT_CF)
DECLARATION (F_AT_UN1)
DECLARATION (F_AT_H)
DECLARATION (F_AT_O)
DECLARATION (F_AT_REP)
DECLARATION (F_AT_CBST_NONTRA)
DECLARATION (F_AT_CBST_TRA)
DECLARATION (F_AT_D_DAT)
DECLARATION (F_AT_PLUS_CMOD_FAX)
DECLARATION (F_AT_PLUS_FCLASS_2_0)
DECLARATION (F_AT_PLUS_FNR)
DECLARATION (F_AT_PLUS_FIS3)
DECLARATION (F_AT_D_DAT2)
DECLARATION (F_AT_PLUS_FDT)
DECLARATION (F_C_ATA)
DECLARATION (F_AT_PLUS_FDR)
DECLARATION (F_AT_PERC_DATA)
DECLARATION (F_AT_PERC_DATA_Q)
DECLARATION (F_AT_PERC_DINF_Q)

DECLARATION (F_RESP_OK)
DECLARATION (F_RESP_ERROR)
DECLARATION (F_RESP_EXT_I)
DECLARATION (F_RESP_A)
DECLARATION (F_RESP_T)
DECLARATION (F_RESP_E)
DECLARATION (F_RESP_0)
DECLARATION (F_RESP_CR)
DECLARATION (F_RESP_CONNECT)
DECLARATION (F_RESP_NO_CARRIER)
DECLARATION (F_RESP_PLUS_FCO)
DECLARATION (F_RESP_PLUS_FIS)
DECLARATION (F_RESP_PLUS_FCS13)
DECLARATION (F_RESP_PLUS_FHS)
DECLARATION (F_RESP_PLUS_FPS)
DECLARATION (F_RESP_PLUS_FET0)
DECLARATION (F_RESP_PLUS_FET2)
DECLARATION (F_RESP_PERC_DATA)
DECLARATION (F_RESP_PERC_DATA_NO_RED)
DECLARATION (F_RESP_PERC_DINF)
DECLARATION (F_RING)

DECLARATION (A_ECC_FIELD )
DECLARATION (A_AD_FIELD_CI_DISABLED )
DECLARATION (A_CLD_NUM )
DECLARATION (A_CLG_NUM )

/* structure declarations */

DECLARATION (S_BS_DAT_9600_ASY_TRA)
DECLARATION (S_BS_DAT_9600_ASY_NON_TRA)
DECLARATION (S_BS_NOT_PRESENT)
DECLARATION (S_CLD_PARTY)
DECLARATION (S_CLD_PARTY_SUB)
DECLARATION (S_CHN_FULL_9600)
DECLARATION (S_CLG_PARTY)
DECLARATION (S_CLG_PARTY_SUB)
DECLARATION (S_BS_FAX)
DECLARATION (S_BS_VOICE)
DECLARATION (S_HDLC_DIS_9600)
DECLARATION (S_DIS_9600)
DECLARATION (S_HDLC_DCS)
DECLARATION (S_DCS)
DECLARATION (DEFAULT_DCB_FROM_DIO_1)
DECLARATION (DEFAULT_DCB_FROM_DIO_2)
DECLARATION (DEFAULT_DCB_FROM_ACI_1)
DECLARATION (DEFAULT_DCB_FROM_ACI_2)

/* Number definitions */

BYTE NUM_0 0
BYTE NUM_1 1
BYTE NUM_2 2
BYTE NUM_3 3
BYTE NUM_4 4
BYTE NUM_5 5
BYTE NUM_6 6
BYTE NUM_7 7
BYTE NUM_8 8
BYTE NUM_9 9
BYTE NUM_10 10
BYTE NUM_12 12
BYTE NUM_18 18
BYTE NUM_20 20
BYTE NUM_27 27
BYTE NUM_30 30
BYTE NUM_50 50
BYTE NUM_DEC_90 90
BYTE NUM_512 512

SHORT NUM_4800 4800
SHORT NUM_9600 9600

SHORT	EOL_0	0x0000

LONG ACI_DTI_LINK_ID_1	0x00000100

LONG ACI_DTI_LINK_ID_2	0x00000200

/* PKTIO data */

LONG	LONG_0	0x00000000
BYTE DIO_CONV_PACKET 0x02

BYTE DIO_CONV_FAIL 0x09

BYTE DIO_MODE_DATA 0x02

BYTE DIO_SLEEP_DISABLE 0x02

LONG DIO_BAUD_812500 0x80000

LONG DIO_BAUD_14400 0x0400

BYTE DIO_CHAR_8 0x80

BYTE DIO_STOP_1 0x01

BYTE DIO_PARITY_NO 0x01

SHORT DIO_FLOW_HW_HW 0x100

BYTE DIO_ESC_OFF 0x00

SHORT PKTCS_SUCCESS 0xD200

BYTE NORMAL_WAY 0x01

BYTE DTI_ID_15 0x15

/*
 * Maximum transmission unit
 */

SHORT	MTU_1500	1500

/* All baud rates allowed on shared memory except DIO_BAUD_AUTO
 */
LONG	DIO_BAUD_SHAREDMEM	0x000FFFFE

LONG	DIO_BAUD_TEST_1	0x000005FE

/*---------------------------------------- AT commands from/to Test Src---------------------- */

/*

- Command: 	&D

DTR behaviour

*/
STRING(C_AND_D0, "AT&D0" )
STRING(C_AND_D1, "AT&D1" )
STRING(C_AND_D2, "AT&D2" )
BYTE LC_AND_D 5

/*

- Message: 	OK

successful operation

*/
STRING(M_OK, "OK" )
BYTE LM_OK 2

/*

message:	RING

*/

STRING(M_RING, "RING" )

BYTE LM_RING 4

/*---------------------------------------- AT commands ------------------------------------------ */

/* AT cmd AT+BLUP field */

BEGINARRAY (F_AT_BLUP, 12) 0x40, 0x00, 0x00, 0x00, 0x41, 0x54, 0x2B, 0x42, 0x4C, 0x55, 0x50, 0x0D

ENDARRAY

/* AT cmd ATE0 field */

BEGINARRAY (F_AT_E0, 9) 0x28, 0x00, 0x00, 0x00, 0x41, 0x54, 0x45, 0x30, 0x0D

ENDARRAY

/* AT cmd ATE1 field */

BEGINARRAY (F_AT_E1, 9) 0x28, 0x00, 0x00, 0x00, 0x41, 0x54, 0x45, 0x31, 0x0D

ENDARRAY

/* AT cmd AT+CMUX field */

BEGINARRAY (F_AT_CMUX, 14) 0x50, 0x00, 0x00, 0x00, 0x41, 0x54, 0x2B, 0x43, 0x4D, 0x55, 0x58, 0x3D, 0x31, 0x0D

ENDARRAY

/* AT cmd AT&D0 field */

BEGINARRAY (F_AT_AND_D0, 10) 0x30, 0x00, 0x00, 0x00, 0x41, 0x54, 0x26, 0x44, 0x30, 0x0D

ENDARRAY

/* AT cmd AT&D1 field */

BEGINARRAY (F_AT_AND_D1, 10) 0x30, 0x00, 0x00, 0x00, 0x41, 0x54, 0x26, 0x44, 0x31, 0x0D

ENDARRAY

/* AT cmd AT&D2 field */

BEGINARRAY (F_AT_AND_D2, 10) 0x30, 0x00, 0x00, 0x00, 0x41, 0x54, 0x26, 0x44, 0x32, 0x0D

ENDARRAY

/* AT cmd AT field */

BEGINARRAY (F_AT, 7) 0x18, 0x00, 0x00, 0x00, 0x41, 0x54, 0x0D

ENDARRAY

/* AT cmd GPAT+CF field */

BEGINARRAY (F_AT_CF, 11) 0x38, 0x00, 0x00, 0x00, 0x47, 0x50, 0x41, 0x54, 0x2B, 0x43, 0x46

ENDARRAY

/* AT cmd AT_UN1 field */

BEGINARRAY (F_AT_UN1, 9) 0x28, 0x00, 0x00, 0x00, 0x55, 0x4E, 0x3D, 0x31, 0x0D

ENDARRAY

/* AT cmd ATH field */

BEGINARRAY (F_AT_H, 8) 0x20, 0x00, 0x00, 0x00, 0x41, 0x54, 0x48, 0x0D

ENDARRAY

/* AT cmd ATO field */

BEGINARRAY (F_AT_O, 8) 0x20, 0x00, 0x00, 0x00, 0x41, 0x54, 0x4F, 0x0D

ENDARRAY

/* AT cmd A/ field */

BEGINARRAY (F_AT_REP, 7) 0x18, 0x00, 0x00, 0x00, 0x41, 0x2F, 0x0D

ENDARRAY

/* AT cmd AT+CBST=71,0,1 field */

BEGINARRAY (F_AT_CBST_NONTRA, 19) 0x78, 0x00, 0x00, 0x00, 0x41, 0x54, 0x2B, 0x43, 0x42, 0x53, 0x54, 0x3D, 0x37, 0x31, 0x2C, 0x30, 0x2C, 0x31, 0x0D

- ENDARRAY

/* AT cmd AT+CBST=71,0,0 field */

BEGINARRAY (F_AT_CBST_TRA, 19) 0x78, 0x00, 0x00, 0x00, 0x41, 0x54, 0x2B, 0x43, 0x42, 0x53, 0x54, 0x3D, 0x37, 0x31, 0x2C, 0x30, 0x2C, 0x30, 0x0D

- ENDARRAY

/* AT cmd ATD123456 field */

BEGINARRAY (F_AT_D_DAT, 19) 0x78, 0x00, 0x00, 0x00, 0x41, 0x54, 0x44, 0x30, 0x33, 0x30, 0x33, 0x39, 0x30, 0x39, 0x34, 0x34, 0x34, 0x34, 0x0D

- ENDARRAY

/* AT+CBST=7,0,0;+CMOD=1^M */

BEGINARRAY (F_AT_PLUS_CMOD_FAX, 26)
0xB0, 0x00, 0x00, 0x00,
0x41, 0x54, 0x2B, 0x43, 0x42,
0x53, 0x54, 0x3D, 0x37, 0x2C, 0x30, 0x2C, 0x30,
0x3B, 0x2B, 0x43, 0x4D, 0x4F, 0x44, 0x3D, 0x31,
0x0D

ENDARRAY

/* AT+FCLASS=2.0^M */

BEGINARRAY (F_AT_PLUS_FCLASS_2_0, 18)
0x70, 0x00, 0x00, 0x00,
0x41, 0x54, 0x2B, 0x46, 0x43, 0x4C, 0x41, 0x53, 0x53, 0x3D, 0x32, 0x2E, 0x30, 0x0D

ENDARRAY

/* AT+FNR=1,1,1,0^M */

BEGINARRAY (F_AT_PLUS_FNR, 19)
0x78, 0x00, 0x00, 0x00,
0x41, 0x54, 0x2B, 0x46, 0x4E, 0x52, 0x3D, 0x31, 0x2C, 0x31, 0x2C, 0x31, 0x2C, 0x30, 0x0D

ENDARRAY

BEGINARRAY (F_AT_PLUS_FIS3, 27)
0xB8, 0x00, 0x00, 0x00,
0x41, 0x54, 0x2B, 0x46, 0x49, 0x53,
0x3D, 0x31, 0x2C, 0x33, 0x2C, 0x30, 0x2C, 0x32, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x30, 0x0D

ENDARRAY

/* ATDT(030)39094444^M */

BEGINARRAY (F_AT_D_DAT2, 22)
0x90, 0x00, 0x00, 0x00,
0x41, 0x54, 0x44, 0x54, 0x28, 0x30, 0x33, 0x30, 0x29, 0x33, 0x39, 0x30, 0x39, 0x34, 0x34, 0x34, 0x34, 0x0D

ENDARRAY

/* AT+FDT */

BEGINARRAY (F_AT_PLUS_FDT, 11)
0x38, 0x00, 0x00, 0x00,
0x41, 0x54, 0x2B, 0x46, 0x44, 0x54, 0x0D

ENDARRAY

/* ATA */

BEGINARRAY (F_C_ATA, 8)

0x20, 0x00, 0x00, 0x00,

0x41, 0x54, 0x41, 0x0D

ENDARRAY

/* AT+FDR */

BEGINARRAY (F_AT_PLUS_FDR, 11)
0x38, 0x00, 0x00, 0x00,
0x41, 0x54, 0x2B, 0x46, 0x44, 0x52, 0x0D

ENDARRAY

/* AT%DATA=2,”PKTIO”,0,0,”PKT”,”UART”,0,0,1 */

BEGINARRAY (F_AT_PERC_DATA, 45)
0x48, 0x01, 0x00, 0x00,
0x41, 0x54, 0x25, 0x44, 0x41, 0x54, 0x41, 0x3D, 0x32, 0x2C, 0x22, 0x50, 0x4B, 0x54, 0x49,
0x4F, 0x22, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x22, 0x50, 0x4B, 0x54,
0x22, 0x2C, 0x22, 0x55, 0x41, 0x52, 0x54, 0x22, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x31,
0x0D

ENDARRAY

/* AT%DATA? */

BEGINARRAY (F_AT_PERC_DATA_Q, 13)
0x48, 0x00, 0x00, 0x00,
0x41, 0x54, 0x25, 0x44, 0x41, 0x54, 0x41, 0x3F, 0x0D

ENDARRAY

/* AT%DINF=0 */

BEGINARRAY (F_AT_PERC_DINF_Q, 14)
0x50, 0x00, 0x00, 0x00,
0x41, 0x54, 0x25, 0x44, 0x49, 0x4E, 0x46, 0x3D, 0x30, 0x0D

ENDARRAY

/*---------------------------------------- AT command responses -------------------------------------- */

/* OK Response */

BEGINARRAY (F_RESP_OK, 6)
0x10, 0x00, 0x00, 0x00, 0x4F, 0x4B

ENDARRAY

/* "Error" Response */

BEGINARRAY (F_RESP_ERROR, 9) 0x28, 0x00, 0x00, 0x00, 0x45, 0x52, 0x52, 0x4f, 0x52

ENDARRAY

/* "EXT I" Response */

BEGINARRAY (F_RESP_EXT_I, 10) 0x30, 0x00, 0x00, 0x00, 0x45, 0x58, 0x54, 0x3A, 0x20, 0x49

ENDARRAY

/* Echo ATE0 */

/* */

- BEGINARRAY (F_RESP_A, 5) 0x08, 0x00, 0x00, 0x00, 0x41

ENDARRAY

/* */

- BEGINARRAY (F_RESP_T, 5) 0x08, 0x00, 0x00, 0x00, 0x54

ENDARRAY

/* */

- BEGINARRAY (F_RESP_E, 5) 0x08, 0x00, 0x00, 0x00, 0x45

ENDARRAY

/* */

- BEGINARRAY (F_RESP_0, 5) 0x08, 0x00, 0x00, 0x00, 0x30

ENDARRAY

/* _ */

BEGINARRAY (F_RESP_CR, 5)

0x08, 0x00, 0x00, 0x00, 0x0D

ENDARRAY

/* CONNECT Response */

BEGINARRAY (F_RESP_CONNECT, 11) 0x38, 0x00, 0x00, 0x00, 0x43, 0x4F, 0x4E, 0x4E, 0x45, 0x43, 0x54

ENDARRAY

/* NO CARRIER Response */

BEGINARRAY (F_RESP_NO_CARRIER, 11) 0x50, 0x00, 0x00, 0x00, 0x4E, 0x4F, 0x20, 0x43, 0x41, 0x52, 0x52, 0x49, 0x45, 0x52

- ENDARRAY

/* ^M+FCO */

BEGINARRAY (F_RESP_PLUS_FCO, 8)
0x20, 0x00, 0x00, 0x00,
0x2B, 0x46, 0x43, 0x4F

ENDARRAY

BEGINARRAY (F_RESP_PLUS_FIS, 26)
0xB0, 0x00, 0x00, 0x00,
0x2B, 0x46, 0x49, 0x53,
0x3A, 0x31, 0x2C, 0x33, 0x2C, 0x30, 0x2C, 0x32, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x30

ENDARRAY

BEGINARRAY (F_RESP_PLUS_FCS13, 26)
0xB0, 0x00, 0x00, 0x00,
0x2B, 0x46, 0x43, 0x53,
0x3A, 0x31, 0x2C, 0x33, 0x2C, 0x30, 0x2C, 0x32, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x30

ENDARRAY

/* +FHS:00 */

BEGINARRAY (F_RESP_PLUS_FHS, 11)

0x38, 0x00, 0x00, 0x00,

0x2B, 0x46, 0x48, 0x53, 0x3A, 0x30, 0x30

ENDARRAY

/* +FPS:1,0,0,0,0 */

BEGINARRAY (F_RESP_PLUS_FPS, 18)

0x70, 0x00, 0x00, 0x00,

0x2B, 0x46, 0x50, 0x53, 0x3A, 0x31, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x30

ENDARRAY

/* +FET:0 */

BEGINARRAY (F_RESP_PLUS_FET0, 10)

0x30, 0x00, 0x00, 0x00,

0x2B, 0x46, 0x45, 0x54, 0x3A, 0x30

ENDARRAY

/* +FET:2 */

BEGINARRAY (F_RESP_PLUS_FET2, 10)

0x30, 0x00, 0x00, 0x00,

0x2B, 0x46, 0x45, 0x54, 0x3A, 0x32

ENDARRAY

/* RING */

BEGINARRAY (F_RING, 8)

0x20, 0x00, 0x00, 0x00,

0x52, 0x49, 0x4E, 0x47

ENDARRAY

/* %DATA:2,"PKTIO",0,0,"PKT",”UART”,0,0,1 */

BEGINARRAY (F_RESP_PERC_DATA, 42)

0x30, 0x01, 0x00, 0x00,

0x25, 0x44, 0x41, 0x54, 0x41, 0x3A, 0x32, 0x2C, 0x22, 0x50, 0x4B, 0x54, 0x49, 0x4F, 0x22, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x22, 0x50, 0x4B, 0x54, 0x22, 0x2C, 0x22, 0x55, 0x41, 0x52, 0x54, 0x22, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x31

ENDARRAY

/* %DATA:0,,0,0,,,0,0,0 */

BEGINARRAY (F_RESP_PERC_DATA_NO_RED, 24)

0xA0, 0x00, 0x00, 0x00,

0x25, 0x44, 0x41, 0x54, 0x41, 0x3A, 0x30, 0x2C, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x2C, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x30

ENDARRAY

/* %DINF:”UART”,0,0,“CMD,SER“,“CMD“,0 */

BEGINARRAY (F_RESP_PERC_DINF, 38)

0x10, 0x01, 0x00, 0x00,

0x25, 0x44, 0x49, 0x4E, 0x46, 0x3A, 0x22, 0x55, 0x41, 0x52, 0x54, 0x22, 0x2C, 0x30, 0x2C, 0x30, 0x2C, 0x22, 0x43, 0x4D, 0x44, 0x2C, 0x53, 0x45, 0x52, 0x22, 0x2C, 0x22, 0x43, 0x4D, 0x44, 0x22, 0x2C, 0x30

ENDARRAY

/*------------------------------------------ Arrays ---------------------------------------- */

/* EF ECC field array */

BEGINARRAY (A_ECC_FIELD,12) 0x11, 0xF2, 0xFF, 0x99, 0xF9, 0xFF, 0x21, 0x43, 0x65, 0xFF, 0xFF, 0xFF ENDARRAY

/* EF AD field array , disable CI */

BEGINARRAY_PART (A_AD_FIELD_CI_DISABLED,4) 0x00, 0x00, 0x00, 0x02 ENDARRAY

/* called number array */

BEGINARRAY_PART (A_CLD_NUM,11) 0x0, 0x3, 0x0, 0x3, 0x9, 0x0, 0x9, 0x4, 0x4, 0x4, 0x4 ENDARRAY

BYTE LA_CLD_NUM 11

/* calling number array */

BEGINARRAY_PART (A_CLG_NUM,11) 0x0, 0x3, 0x0, 0x3, 0x9, 0x0, 0x9, 0x4, 0x2, 0x2, 0x3 ENDARRAY

BYTE LA_CLG_NUM 11

/*---------------------------------------- Structures ---------------------------------------- */

/* bearer service transparent data 9600 */
 BEGIN_PSTRUCT ("bcpara", S_BS_DAT_9600_ASY_TRA)
	SET_COMP ("rate",	UR_9_6_KBIT)
	SET_COMP ("bearer_serv",	BEARER_SERV_ASYNC)
	SET_COMP ("conn_elem",	CONN_ELEM_TRANS)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)
	SET_COMP ("modem_type",	MT_NONE)
ENDSTRUCT

/* bearer service non transparent data 9600 */
 BEGIN_PSTRUCT ("bcpara", S_BS_DAT_9600_ASY_NON_TRA)
	SET_COMP ("rate",	UR_9_6_KBIT)
	SET_COMP ("bearer_serv",	BEARER_SERV_ASYNC)
	SET_COMP ("conn_elem",	CONN_ELEM_NON_TRANS)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)
	SET_COMP ("modem_type",	MT_NONE)
ENDSTRUCT

/* bearer service not present */
 BEGIN_PSTRUCT ("bcpara", S_BS_NOT_PRESENT)
	SET_COMP ("rate",	UR_NOT_PRES)
	SET_COMP ("bearer_serv",	BEARER_SERV_NOT_PRES)
	SET_COMP ("conn_elem",	CONN_ELEM_NOT_PRES)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)
	SET_COMP ("modem_type",	MT_NONE)
ENDSTRUCT

/* called party address national*/
 BEGIN_PSTRUCT ("called_party", S_CLD_PARTY)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num",	LA_CLD_NUM)
	SET_COMP ("called_num",	A_CLD_NUM)
ENDSTRUCT

/* called party sub address */
 BEGIN_PSTRUCT ("called_party_sub", S_CLD_PARTY_SUB)
	SET_COMP ("tos",	TOS_NOT_PRES)
	SET_COMP ("odd_even",	OE_EVEN)
	SET_COMP ("c_subaddr",	NUM_0)
	SKIP_COMP ("subaddr")
ENDSTRUCT

/* data full rate 9600 */
 BEGIN_PSTRUCT ("chm", S_CHN_FULL_9600)
	SET_COMP ("ch_type",	CH_TCH_F)
	SET_COMP ("ch_mode",	CHM_DATA_9_6)
ENDSTRUCT

/* calling party address */
 BEGIN_PSTRUCT ("calling_party", S_CLG_PARTY)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("present",	PRES_PRES_ALLOW)
	SET_COMP ("screen",	SCREEN_IND_NOT_PRES)
	SET_COMP ("c_num",	LA_CLG_NUM)
	SET_COMP ("num",	A_CLG_NUM)
ENDSTRUCT

/* calling party sub address */
 BEGIN_PSTRUCT ("calling_party_sub", S_CLG_PARTY_SUB)
	SET_COMP ("tos",	TOS_NOT_PRES)
	SET_COMP ("odd_even",	OE_EVEN)
	SET_COMP ("c_subaddr",	NUM_0)
	SKIP_COMP ("subaddr")
ENDSTRUCT

/* bearer service FAX 9600 */

BEGIN_PSTRUCT ("bcpara", S_BS_FAX)
	SET_COMP ("rate",	UR_9_6_KBIT)
	SET_COMP ("bearer_serv",	BEARER_SERV_FAX)
	SET_COMP ("conn_elem",	CONN_ELEM_TRANS)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)
	SET_COMP ("modem_type",	MT_NONE)

ENDSTRUCT

/* bearer service voice */

BEGIN_PSTRUCT ("bcpara", S_BS_VOICE)
	SET_COMP ("rate",	UR_NOT_PRES)
	SET_COMP ("bearer_serv",	BEARER_SERV_SPEECH)
	SET_COMP ("conn_elem",	CONN_ELEM_NOT_PRES)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)

SET_COMP ("modem_type",	MT_NONE)

ENDSTRUCT

BEGIN_PSTRUCT ("hdlc_info", S_HDLC_DIS_9600)
	SET_COMP ("crp",	NUM_0)
	SET_COMP ("c_pwd",	NUM_0)
	SKIP_COMP ("pwd")
	SET_COMP ("c_sub",	NUM_0)
	SKIP_COMP ("sub")
	SET_COMP ("c_sep",	NUM_0)
	SKIP_COMP ("sep")
	SET_COMP ("c_nsc",	NUM_0)
	SKIP_COMP ("nsc")
	SET_COMP ("c_nsf",	NUM_0)
	SKIP_COMP ("nsf")
	SET_COMP ("c_tsi",	NUM_0)
	SKIP_COMP ("tsi")
	SET_COMP ("c_cig",	NUM_0)
	SKIP_COMP ("cig")
	SET_COMP ("c_csi",	NUM_0)
	SKIP_COMP ("csi")
	SET_COMP ("c_nss",	NUM_0)
	SKIP_COMP ("nss")
	SET_COMP ("v_dis",	NUM_1)
	SET_COMP ("dis",	S_DIS_9600)
	SET_COMP ("v_dcs",	NUM_0)
	SKIP_COMP ("dcs")
	SET_COMP ("v_dtc",	NUM_0)
	SKIP_COMP ("dtc")

ENDSTRUCT

/* DIS frame – 9600 bps */

BEGIN_PSTRUCT ("dis", S_DIS_9600)
	SKIP_COMP ("v8")
	SKIP_COMP ("n_byte")
	SKIP_COMP ("ready_tx_fax")
	SKIP_COMP ("rec_fax_op")
	SET_COMP ("data_sig_rate", NUM_8)
	SET_COMP ("R8_lines_pels", NUM_1)
	SKIP_COMP ("two_dim_coding")
	SKIP_COMP ("rec_width")
	SET_COMP ("max_rec_len", NUM_1)
	SET_COMP ("min_scan_time", NUM_7)
	SKIP_COMP ("uncomp_mode")
	SKIP_COMP ("err_corr_mode")
	SKIP_COMP ("frame_size")
	SKIP_COMP ("t6_coding")
	SKIP_COMP ("R8_lines")
	SKIP_COMP ("r_300_pels")
	SKIP_COMP ("R16_lines_pels")
	SKIP_COMP ("resolution_type")
	SKIP_COMP ("i_res_pref")
	SKIP_COMP ("m_res_pref")
	SKIP_COMP ("min_scan_time_hr")
	SKIP_COMP ("sel_polling")
	SKIP_COMP ("subaddr")
	SKIP_COMP ("password")
	SKIP_COMP ("ready_tx_doc")
	SKIP_COMP ("bft")
	SKIP_COMP ("dtm")
	SKIP_COMP ("edi")
	SKIP_COMP ("btm")
	SKIP_COMP ("ready_tx_mixed")
	SKIP_COMP ("char_mode")
	SKIP_COMP ("mixed_mode")
	SKIP_COMP ("proc_mode_26")
	SKIP_COMP ("dig_network_cap")
	SKIP_COMP ("duplex")
	SKIP_COMP ("jpeg")
	SKIP_COMP ("full_colour")
	SKIP_COMP ("huffman_tables")
	SKIP_COMP ("r_12_bits_pel_comp")
	SKIP_COMP ("no_subsamp")
	SKIP_COMP ("cust_illum")
	SKIP_COMP ("cust_gamut")
	SKIP_COMP ("na_letter")
	SKIP_COMP ("na_legal")
	SKIP_COMP ("sing_prog_seq_coding_basic")
	SKIP_COMP ("sing_prog_seq_coding_L0")

ENDSTRUCT

/* HDLC frame */

BEGIN_PSTRUCT ("hdlc_info", S_HDLC_DCS)
	SET_COMP ("crp",	NUM_0)
	SET_COMP ("c_pwd",	NUM_0)
	SKIP_COMP ("pwd")
	SET_COMP ("c_sub",	NUM_0)
	SKIP_COMP ("sub")
	SET_COMP ("c_sep",	NUM_0)
	SKIP_COMP ("sep")
	SET_COMP ("c_nsc",	NUM_0)
	SKIP_COMP ("nsc")
	SET_COMP ("c_nsf",	NUM_0)
	SKIP_COMP ("nsf")
	SET_COMP ("c_tsi",	NUM_0)
	SKIP_COMP ("tsi")
	SET_COMP ("c_cig",	NUM_0)
	SKIP_COMP ("cig")
	SET_COMP ("c_csi",	NUM_0)
	SKIP_COMP ("csi")
	SET_COMP ("c_nss",	NUM_0)
	SKIP_COMP ("nss")
	SET_COMP ("v_dis",	NUM_0)
	SKIP_COMP ("dis")
	SET_COMP ("v_dcs",	NUM_1)
	SET_COMP ("dcs",	S_DCS)
	SET_COMP ("v_dtc",	NUM_0)
	SKIP_COMP ("dtc")

ENDSTRUCT

/* DCS frame */

BEGIN_PSTRUCT ("dcs", S_DCS)
	SKIP_COMP ("v8")
	SKIP_COMP ("n_byte")
	SKIP_COMP ("ready_tx_fax")
	SKIP_COMP ("rec_fax_op")
	SKIP_COMP ("data_sig_rate")
	SKIP_COMP ("R8_lines_pels")
	SKIP_COMP ("two_dim_coding")
	SKIP_COMP ("rec_width")
	SKIP_COMP ("max_rec_len")
	SKIP_COMP ("min_scan_time")
	SKIP_COMP ("uncomp_mode")
	SKIP_COMP ("err_corr_mode")
	SKIP_COMP ("frame_size")
	SKIP_COMP ("t6_coding")
	SKIP_COMP ("R8_lines")
	SKIP_COMP ("r_300_pels")
	SKIP_COMP ("R16_lines_pels")
	SKIP_COMP ("resolution_type")
	SKIP_COMP ("i_res_pref")
	SKIP_COMP ("m_res_pref")
	SKIP_COMP ("min_scan_time_hr")
	SKIP_COMP ("sel_polling")
	SKIP_COMP ("subaddr")
	SKIP_COMP ("password")
	SKIP_COMP ("ready_tx_doc")
	SKIP_COMP ("bft")
	SKIP_COMP ("dtm")
	SKIP_COMP ("edi")
	SKIP_COMP ("btm")
	SKIP_COMP ("ready_tx_mixed")
	SKIP_COMP ("char_mode")
	SKIP_COMP ("mixed_mode")
	SKIP_COMP ("proc_mode_26")
	SKIP_COMP ("dig_network_cap")
	SKIP_COMP ("duplex")
	SKIP_COMP ("jpeg")
	SKIP_COMP ("full_colour")
	SKIP_COMP ("huffman_tables")
	SKIP_COMP ("r_12_bits_pel_comp")
	SKIP_COMP ("no_subsamp")
	SKIP_COMP ("cust_illum")
	SKIP_COMP ("cust_gamut")
	SKIP_COMP ("na_letter")
	SKIP_COMP ("na_legal")
	SKIP_COMP ("sing_prog_seq_coding_basic")
	SKIP_COMP ("sing_prog_seq_coding_L0")

ENDSTRUCT

/*DIO Capabilities from DIO */

BEGIN_PSTRUCT ("dio_dcb", DEFAULT_DCB_FROM_DIO_1)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", NUM_0)
	SET_COMP ("n2", NUM_0)
	SET_COMP ("t1", NUM_0)
	SET_COMP ("t2", NUM_0)
	SET_COMP ("t3", NUM_0)
	SET_COMP ("k", NUM_0)
	SET_COMP ("mtu", MTU_1500)
	SET_COMP ("baud", DIO_BAUD_SHAREDMEM)
	SET_COMP ("data_bits", DIO_CHAR_8) /* Some defaults */
	SET_COMP ("stop_bits", DIO_STOP_1) /* Some defaults */
	SET_COMP ("parity", DIO_PARITY_NO) /* Some defaults */
	SET_COMP ("flow_control", DIO_FLOW_HW_HW) /* Shared memory is HW */
	SET_COMP ("xon", NUM_0)
	SET_COMP ("xoff", NUM_0)
	SET_COMP ("esc_char", NUM_0)
	SET_COMP ("guard_period", DIO_ESC_OFF) /* No guard pattern detection */
ENDSTRUCT

BEGIN_PSTRUCT ("dio_dcb", DEFAULT_DCB_FROM_DIO_2)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", NUM_0)
	SET_COMP ("n2", NUM_0)
	SET_COMP ("t1", NUM_0)
	SET_COMP ("t2", NUM_0)
	SET_COMP ("t3", NUM_0)
	SET_COMP ("k", NUM_0)
	SET_COMP ("mtu", MTU_1500)
	SET_COMP ("baud", DIO_BAUD_TEST_1)
	SET_COMP ("data_bits", DIO_CHAR_8) /* Some defaults */
	SET_COMP ("stop_bits", DIO_STOP_1) /* Some defaults */
	SET_COMP ("parity", DIO_PARITY_NO) /* Some defaults */
	SET_COMP ("flow_control", DIO_FLOW_HW_HW) /* Shared memory is HW */
	SET_COMP ("xon", NUM_0)
	SET_COMP ("xoff", NUM_0)
	SET_COMP ("esc_char", NUM_0)
	SET_COMP ("guard_period", DIO_ESC_OFF) /* No guard pattern detection */
ENDSTRUCT

/*DIO Capabilities from ACI */

BEGIN_PSTRUCT ("dio_dcb", DEFAULT_DCB_FROM_ACI_1)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", NUM_0)
	SET_COMP ("n2", NUM_0)
	SET_COMP ("t1", NUM_0)
	SET_COMP ("t2", NUM_0)
	SET_COMP ("t3", NUM_0)
	SET_COMP ("k", NUM_0)
	SET_COMP ("mtu", MTU_1500)
	SET_COMP ("baud", DIO_BAUD_812500) /* Negotiated highest common */
	SET_COMP ("data_bits", DIO_CHAR_8)
	SET_COMP ("stop_bits", DIO_STOP_1)
	SET_COMP ("parity", DIO_PARITY_NO)
	SET_COMP ("flow_control", DIO_FLOW_HW_HW)
	SET_COMP ("xon", NUM_0)
	SET_COMP ("xoff", NUM_0)
	SET_COMP ("esc_char", NUM_0)
	SET_COMP ("guard_period", DIO_ESC_OFF)
ENDSTRUCT

BEGIN_PSTRUCT ("dio_dcb", DEFAULT_DCB_FROM_ACI_2)
	SET_COMP ("convergence", DIO_CONV_PACKET)
	SET_COMP ("data_mode", DIO_MODE_DATA)
	SET_COMP ("sleep_mode", DIO_SLEEP_DISABLE)
	SET_COMP ("mux_configuration", LONG_0)
	SET_COMP ("n1", NUM_0)
	SET_COMP ("n2", NUM_0)
	SET_COMP ("t1", NUM_0)
	SET_COMP ("t2", NUM_0)
	SET_COMP ("t3", NUM_0)
	SET_COMP ("k", NUM_0)
	SET_COMP ("mtu", MTU_1500)
	SET_COMP ("baud", DIO_BAUD_14400) /* Negotiated highest common */
	SET_COMP ("data_bits", DIO_CHAR_8)
	SET_COMP ("stop_bits", DIO_STOP_1)
	SET_COMP ("parity", DIO_PARITY_NO)
	SET_COMP ("flow_control", DIO_FLOW_HW_HW)
	SET_COMP ("xon", NUM_0)
	SET_COMP ("xoff", NUM_0)
	SET_COMP ("esc_char", NUM_0)
	SET_COMP ("guard_period", DIO_ESC_OFF)
ENDSTRUCT

# TEST CASES

## Initialisation

### ACIDTI000: Preamble

Description:

Preamble:

None

APL		ACI		PS

|		|		|

COMMAND (TAP RESET)
COMMAND (CC RESET)
COMMAND (MM RESET)
COMMAND (SIM RESET)
COMMAND (SS RESET)
COMMAND (SMS RESET)

|		|		|

COMMAND (UART REDIRECT MMI NULL)

|		|		|
COMMAND (UART RESET)
COMMAND (MMI RESET)
COMMAND (PL RESET)
COMMAND (PKT RESET)
	 |		|		|
COMMAND (TAP REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)
COMMAND (UART REDIRECT CLEAR)
COMMAND (PL REDIRECT CLEAR)
COMMAND (PKT REDIRECT CLEAR)
	 |		|		|
COMMAND (MMI REDIRECT CC TAP)
COMMAND (MMI REDIRECT MM TAP)
COMMAND (MMI REDIRECT SIM TAP)
COMMAND (MMI REDIRECT SS TAP)
COMMAND (MMI REDIRECT MMI TAP)
COMMAND (MMI REDIRECT SMS TAP)
COMMAND (MMI REDIRECT T30 TAP)
COMMAND (MMI REDIRECT L2R TAP)
COMMAND (MMI REDIRECT UART TAP)
COMMAND (MMI REDIRECT RA TAP)
COMMAND (MMI REDIRECT PKT TAP)
COMMAND (PL REDIRECT MMI NULL)

COMMAND (UART REDIRECT MMI NULL)
	|		|		|
COMMAND (TAP REDIRECT TAP MMI)
COMMAND (MMI REDIRECT MMI TAP)
	|		|		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

History:	15.02.01	Clb	Initial

### ACIDTI001: UART Init

Description: MMI reset is done after in the initialization... Means, every primitive that has to be sent at the very beginning should be here defined.

.

Preamble:

- ACIDTI000

APL		ACI		PS

|		|		|

- |		|	UART_PARAMETERS_IND	|
	|		*	<==========================	*

- |		|	UART_PARAMETERS_REQ	|
	|		*	==========================>	*

- |		|	UART_PARAMETERS_CNF	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- UART_PARAMETERS_IND
	uart_instances	NUM_1

- UART_PARAMETERS_REQ
	device	NUM_0
	comPar	NOT_USED

- UART_PARAMETERS_CNF
	device	NUM_0

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- UART_DCD_REQ
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- UART_DCD_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		15.02.01	Clb	Initial
		29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI002: Deactivate echoing

Description:  receive the command: ATE0

Preamble:

- ACIDTI001

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: ATE0)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: ATE0)	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_E0

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_E0

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ 	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		15.02.01	Clb	Initial
		29.07.02	Tlu 	Adaptation to DTI2

## Simple Command Handling

### ACIDTI011: Receive a command

Description:

receive the command: AT+CFUN=1

Preamble:

- ACIDTI002

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+CF)	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: UN=1)	|
	|		*	<==========================	*

- |		| 	SIM_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	SIM_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		| 	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*

- |		| 	SIM_READ_REQ	|
	|		*	==========================>	*

- |		| 	SIM_READ_CNF	|
	|		*	<==========================	*

- |		| 	SIM_READ_REQ	|
	|		*	==========================>	*

- |		| 	SIM_READ_CNF	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_CF

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_UN1

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	NOT_USED
	stk_pro_file	NOT_USED

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_9
	pin2_cnt	NUM_3
	puk2_cnt	NUM_9
	ec_code	NOT_USED
	pref_lang	NOT_USED

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	NOT_USED
	imsi_field	NOT_USED
	pref_plmn	NOT_USED
	phase	PHASE_2_SIM
	access_acm	NOT_USED
	access_acmmax	NOT_USED
	access_puct	NOT_USED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_AD
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_AD
	cause	SIM_NO_ERROR
	length	NUM_4
	trans_data	A_AD_FIELD_CI_DISABLED

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	
	link_id	ACI_DTI_LINK_ID_1

History:		15.02.01	Clb	Initial
		29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI012: Repeat last command

Description:

receive the command: A/

Preamble:

- ACIDTI011

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: A/)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_REP

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

History:		15.02.01	Clb	Initial
		29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI013: Receive wrong command.

Description:

receive the command: AT+BLUP

Preamble:

- ACIDTI011

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+BLUP)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: EXT: I)	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: ERROR)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_BLUP

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_EXT_I

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_ERROR

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		15.02.01	Clb	Initial
		29.07.02	Tlu 	Adaptation to DTI2

## Multiplexer Mode

### ACIDTI021: Start multiplexer

Description:

test AT+CMUX

Preamble:

- ACIDTI002

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+CMUX)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		| 	DTI2_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_MUX_START_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	UART_MUX_DLC_ESTABLISH_IND	|
	|		*	<==========================	*

- |		|	UART_MUX_DLC_ESTABLISH_RES	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_CMUX

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_DISCONNECT_REQ	link_id	ACI_DTI_LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_MUX_START_REQ
	device	NUM_0
	mode	UART_MUX_MODE_ADVANCED
	frame_type	UART_MUX_FRAME_UIH
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	t1	UART_MUX_T1_DEFAULT
	n2	UART_MUX_N2_DEFAULT
	t2	UART_MUX_T2_DEFAULT
	t3	UART_MUX_T3_DEFAULT

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- UART_MUX_DLC_ESTABLISH_IND
	device	NUM_0
	dlci	NUM_1
	convergence	UART_MUX_CONVERGENCE_DEFAULT
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	service	UART_MUX_SERVICE_AT

- UART_MUX_DLC_ESTABLISH_RES
	device	NUM_0
	dlci	NUM_1
	n1	UART_MUX_N1_ADVANCED_DEFAULT

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	NUM_1
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	NUM_1

- UART_DCD_REQ
	device	NUM_0
	dlci	NUM_1
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	NUM_1

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		15.02.01	Clb	Initial
		29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI022: Start a second channel - multiplexer

Description:

- Preamble: ACIDTI021

APL		ACI		PS

|		|		|

- |		|	UART_MUX_DLC_ESTABLISH_IND	|
	|		*	<==========================	*

- |		|	UART_MUX_DLC_ESTABLISH_RES	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- UART_MUX_DLC_ESTABLISH_IND
	device	NUM_0
	dlci	NUM_2
	convergence	UART_MUX_CONVERGENCE_DEFAULT
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	service	UART_MUX_SERVICE_AT

- UART_MUX_DLC_ESTABLISH_RES
	device	NUM_0
	dlci	NUM_2
	n1	UART_MUX_N1_ADVANCED_DEFAULT

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_2
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	NUM_2
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_2
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_2
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	NUM_2

- UART_DCD_REQ
	device	NUM_0
	dlci	NUM_2
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	NUM_2

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_2

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_2

History:		27.03.01	clb	Initial
		29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI023: Send a command ("AT") on second channel

Description: Send a command ("AT") on the new multiplex channel.

- Preamble: ACIDTI022

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: AT)	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_2
	parameters	NOT_USED
	sdu	F_AT

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_2
	parameters	NOT_USED
	sdu	F_AT

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_2

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_2

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_2
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_2

History:	27.03.01	clb	Initial
	29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI024: Try with AT+CFUN on the second channel

Description: Deactivate echoing before send an AT Command on the second channel.

- Preamble: ACIDTI022

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: ATE0)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: ATE0)	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+CF)	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: UN=1)	|
	|		*	<==========================	*

- |		| 	SIM_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	SIM_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		| 	SIM_MMI_INSERT_IND	|
	|		*	<==========================	*

- |		| 	SIM_READ_REQ	|
	|		*	==========================>	*

- |		| 	SIM_READ_CNF	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_2
	parameters	NOT_USED
	sdu	F_AT_E0

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_2
	parameters	NOT_USED
	sdu	F_AT_E0

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_2

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_2

- DTI2_DATA_TEST_REQ 	link_id	ACI_DTI_LINK_ID_2
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_2

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_2
	parameters	NOT_USED
	sdu	F_AT_CF

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_2

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_2
	parameters	NOT_USED
	sdu	F_AT_UN1

- SIM_ACTIVATE_REQ
	proc	SIM_INITIALISATION
	mmi_pro_file	NOT_USED
	stk_pro_file	NOT_USED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_2

- SIM_ACTIVATE_CNF
	cause	SIM_NO_ERROR
	pin_cnt	NUM_3
	puk_cnt	NUM_9
	pin2_cnt	NUM_3
	puk2_cnt	NUM_9
	ec_code	NOT_USED
	pref_lang	NOT_USED

- SIM_MMI_INSERT_IND
	func	SIM_ADN_ENABLED
	sim_serv	NOT_USED
	imsi_field	NOT_USED
	pref_plmn	NOT_USED
	phase	PHASE_2_SIM
	access_acm	NOT_USED
	access_acmmax	NOT_USED
	access_puct	NOT_USED

- SIM_READ_REQ
	source	SRC_MMI
	offset	NUM_0
	datafield	SIM_ECC
	length	NOT_PRESENT_8BIT
	max_length	NUM_0

- SIM_READ_CNF
	datafield	SIM_ECC
	cause	SIM_NO_ERROR
	length	NUM_12
	trans_data	A_ECC_FIELD

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_2
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_2

History:	27.03.01	clb	Initial
	29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI025: Try to start a second channel - get error when asking for a DTI channel

Description:

- Preamble: ACIDTI021

APL		ACI		PS

|		|		|

- |		|	UART_MUX_DLC_ESTABLISH_IND	|
	|		*	<==========================	*

- |		|	UART_MUX_DLC_ESTABLISH_RES	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	UART_ERROR_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- UART_MUX_DLC_ESTABLISH_IND
	device	NUM_0
	dlci	NUM_2
	convergence	UART_MUX_CONVERGENCE_DEFAULT
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	service	UART_MUX_SERVICE_AT

- UART_MUX_DLC_ESTABLISH_RES
	device	NUM_0
	dlci	NUM_2
	n1	UART_MUX_N1_ADVANCED_DEFAULT

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_2
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	NUM_2
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_2
	entity_name	NOT_USED

- UART_ERROR_IND
	device	NUM_0
	dlci	NUM_2
	error	UART_ERROR_NO_CHANNEL

History:		27.03.01	clb	Initial
		29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI026: Start multiplexer and 2 channels - get error

Description:

.test AT+CMUX

Preamble:

- ACIDTI002

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+CMUX)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		| 	DTI2_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_MUX_START_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	UART_MUX_DLC_ESTABLISH_IND	|
	|		*	<==========================	*

- |		|	UART_MUX_DLC_ESTABLISH_RES	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	UART_ERROR_IND	|
	|		*	<==========================	*

- |		|	UART_MUX_DLC_ESTABLISH_IND	|
	|		*	<==========================	*

- |		|	UART_MUX_DLC_ESTABLISH_RES	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_CMUX

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_DISCONNECT_REQ	link_id	ACI_DTI_LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_MUX_START_REQ
	device	NUM_0
	mode	UART_MUX_MODE_ADVANCED
	frame_type	UART_MUX_FRAME_UIH
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	t1	UART_MUX_T1_DEFAULT
	n2	UART_MUX_N2_DEFAULT
	t2	UART_MUX_T2_DEFAULT
	t3	UART_MUX_T3_DEFAULT

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- UART_MUX_DLC_ESTABLISH_IND
	device	NUM_0
	dlci	NUM_1
	convergence	UART_MUX_CONVERGENCE_DEFAULT
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	service	UART_MUX_SERVICE_AT

- UART_MUX_DLC_ESTABLISH_RES
	device	NUM_0
	dlci	NUM_1
	n1	UART_MUX_N1_ADVANCED_DEFAULT

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	NUM_1
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- UART_ERROR_IND
	device	NUM_0
	dlci	NUM_1
	error	UART_ERROR_NO_CHANNEL

- UART_MUX_DLC_ESTABLISH_IND
	device	NUM_0
	dlci	NUM_2
	convergence	UART_MUX_CONVERGENCE_DEFAULT
	n1	UART_MUX_N1_ADVANCED_DEFAULT
	service	UART_MUX_SERVICE_AT

- UART_MUX_DLC_ESTABLISH_RES
	device	NUM_0
	dlci	NUM_2
	n1	UART_MUX_N1_ADVANCED_DEFAULT

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_2
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	NUM_2
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_2
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_2
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	NUM_2

- UART_DCD_REQ
	device	NUM_0
	dlci	NUM_2
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	NUM_1

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_2

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_2

History:	15.02.01	Clb	Initial
	30.07.02	Tlu 	Adaptation to DTI2

### ACIDTI027: Close multiplexer

Description:

Preamble:

- ACIDTI022

APL		ACI		PS

|		|		|

- |		|	UART_MUX_CLOSE_IND	|
	|		*	<==========================	*

- |		|	UART_PARAMETERS_REQ	|
	|		*	==========================>	*

- |		|	UART_PARAMETERS_CNF	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- UART_MUX_CLOSE_IND
	device	NUM_0

- UART_PARAMETERS_REQ
	device	NUM_0
	comPar	NOT_USED

- UART_PARAMETERS_CNF
	device	NUM_0

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- UART_DCD_REQ
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:	15.02.01	Clb	Initial
	30.07.02	Tlu 	Adaptation to DTI2

## Transparent Data Transfer

### ACIDTI031: Transparent Data call over DTI (part 1)

Description:

.test connect from DTI

Preamble:

- ACIDTI002

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+CBST=71,0,0)	|
	|		*	<==========================	*

- |		| 	MNCC_CONFIGURE_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: ATD03039094444)	|
	|		*	<==========================	*

- |		| 	MNCC_SETUP_REQ	|
	|		*	==========================>	*

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_CALL_PROCEED_IND	|
	|		*	<==========================	*

- |		| 	MNCC_SYNC_IND	|
	|		*	<==========================	*

- |		| 	MNCC_SETUP_CNF	|
	|		*	<==========================	*

- |		| 	RA_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	RA_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		| 	TRA_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	TRA_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: CONNECT)	|
	|		*	==========================>	*

- |		| 	DTI2_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|		|

- |		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_CBST_TRA

- MNCC_CONFIGURE_REQ
	called_party_sub	NOT_USED
	bcpara	S_BS_DAT_9600_ASY_TRA
	sns_mode	SNS_MODE_VOICE
	ctm_ena	CTM_DISABLED

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_D_DAT

- MNCC_SETUP_REQ
	ti	NUM_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	S_BS_DAT_9600_ASY_TRA
	bcpara2	S_BS_NOT_PRESENT
	called_party	S_CLD_PARTY
	called_party_sub	S_CLD_PARTY_SUB
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- MNCC_CALL_PROCEED_IND
	ti	NUM_0
	progress_desc	PROG_NOT_PRES
	ri	NOT_PRESENT_8BIT
	bcpara	S_BS_DAT_9600_ASY_TRA
	bcpara2	S_BS_NOT_PRESENT

- MNCC_SYNC_IND
	ti	NOT_PRESENT_8BIT
	cause	MNCC_CAUSE_NO_MS_CAUSE
	chm	S_CHN_FULL_9600

- MNCC_SETUP_CNF
	ti	NUM_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	S_CLG_PARTY
	connected_number_sub	S_CLG_PARTY_SUB

- RA_ACTIVATE_REQ
	model	RA_MODEL_TRANS
	tra_rate	TRA_FULLRATE_9600
	user_rate	URA_9600
	ndb	NUM_8
	nsb	NOT_USED

- RA_ACTIVATE_CNF
	ack_flg	RA_ACK

- TRA_ACTIVATE_REQ

- TRA_ACTIVATE_CNF
	ack_flg	NOT_USED

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_CONNECT

- DTI2_DISCONNECT_REQ	link_id	ACI_DTI_LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

History:	15.02.01	Clb	Initial
	30.07.02	Tlu 	Adaptation to DTI2

### ACIDTI032: Transparent Data call over DTI (part 2)

Description:

Preamble:

- ACIDTI031

APL		ACI		PS

|		|		|

- |		| 	TRA_DTI_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		| 	TRA_DTI_CNF	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- TRA_DTI_REQ
	dti_conn	TRA_CONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED
	dti_direction	NOT_USED

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- TRA_DTI_CNF
	dti_conn	TRA_CONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

History:	27.03.01	CLB	Initial
	30.07.02	Tlu 	Adaptation to DTI2

### ACIDTI033: Network breaks communication - Disconnect Ind

Description:

- Preamble: ACIDTI032

APL		ACI		PS

|		|		|

- |		| 	MNCC_DISCONNECT_IND	|
	|		*	<==========================	*

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_RELEASE_CNF	|
	|		*	<==========================	*

- |		| 	TRA_DTI_REQ	|
	|		*	==========================>	*

- |		| 	TRA_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		| 	TRA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		| 	TRA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		| 	RA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	RA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: NO CARRIER)	|
	|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_DISCONNECT_IND
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR
	diagnostic	NOT_PRESENT_8BIT
	progress_desc	PROG_NOT_PRES

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_RELEASE_CNF
	ti	NOT_USED
	cause	NOT_USED

- TRA_DTI_REQ
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED
	dti_direction	NOT_USED

- TRA_DTI_CNF
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- TRA_DEACTIVATE_REQ

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- TRA_DEACTIVATE_CNF

- UART_DCD_REQ
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- RA_DEACTIVATE_REQ

- RA_DEACTIVATE_CNF

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_NO_CARRIER

History:	27.03.01	clb	Initial
	30.07.02	Tlu 	Adaptation to DTI2

### ACIDTI034: Network breaks communication - Release Ind

Description:

- Preamble: ACIDTI032

APL		ACI		PS

|		|		|

- |		| 	MNCC_RELEASE_IND	|
	|		*	<==========================	*

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		| 	TRA_DTI_REQ	|
	|		*	==========================>	*

- |		| 	TRA_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		| 	TRA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		| 	TRA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		| 	RA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	RA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: NO CARRIER)	|
	|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_RELEASE_IND
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- TRA_DTI_REQ
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED
	dti_direction	NOT_USED

- TRA_DTI_CNF
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- TRA_DEACTIVATE_REQ

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- TRA_DEACTIVATE_CNF

- UART_DCD_REQ
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- RA_DEACTIVATE_REQ

- RA_DEACTIVATE_CNF

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_NO_CARRIER

History:	27.03.01	clb	Initial
	29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI035: TRA gives channel back to ACI when an escape sequence detected

Description: escape sequence detected.

- Preamble: ACIDTI032

APL		ACI		PS

|		|		|

- |		| 	UART_DETECTED_IND	|
	|		*	<==========================	*

- |		|	TRA_DTI_REQ	|
	|		*	==========================>	*

- |		|	TRA_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- UART_DETECTED_IND
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	cause	UART_DETECT_ESC

- TRA_DTI_REQ
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED
	dti_direction	NOT_USED

- TRA_DTI_CNF
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		27.03.01	clb	Initial
		30.07.02	Tlu 	Adaptation to DTI2

### ACIDTI036: User terminates call (ATH)

Description:

- Preamble: ACIDTI035

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: ATH)	|
	|		*	<==========================	*

- |		| 	TRA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	TRA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		| 	RA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	RA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_RELEASE_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_H

- TRA_DEACTIVATE_REQ

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- TRA_DEACTIVATE_CNF

- UART_DCD_REQ
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- RA_DEACTIVATE_REQ

- RA_DEACTIVATE_CNF

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		27.03.01	clb	Initial
		30.07.02	Tlu 	Adaptation to DTI2

### ACIDTI037: Return to data mode (ATO)

Description:

- Preamble: ACIDTI035

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: ATO)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: CONNECT)	|
	|		*	==========================>	*

- |		| 	DTI2_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		| 	TRA_DTI_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		| 	TRA_DTI_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_O

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_CONNECT

- DTI2_DISCONNECT_REQ	link_id	ACI_DTI_LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- TRA_DTI_REQ
	dti_conn	TRA_CONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED
	dti_direction	NOT_USED

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- TRA_DTI_CNF
	dti_conn	TRA_CONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

History:		27.03.01	clb	Initial
		30.07.02	Tlu 	Adaptation to DTI2

### ACIDTI038: DTR detected (AT&D0)

Description: DTR detected, when DTR behavior is set to 0 (DCE ignores DTR).

- Preamble: ACIDTI032

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: AT&D0)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|

- |		| 	UART_DETECTED_IND	|
	|		|	(UART_DETECT_DTR)	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_AND_D
	cmd_seq	C_AND_D0

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- UART_DETECTED_IND
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	cause	UART_DETECT_DTR

History:		30.07.02	Tlu 	Initial

### ACIDTI039: DTR detected (AT&D1)

Description: DTR detected, when DTR behavior is set to 1 (on reception of DTR the DCE enters command state and issues an OK result code).

- Preamble: ACIDTI032

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: AT&D1)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|

- |		| 	UART_DETECTED_IND	|
	|		|	(UART_DETECT_DTR)	|
	|		*	<==========================	*

- |		|	TRA_DTI_REQ	|
	|		*	==========================>	*

- |		|	TRA_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_AND_D
	cmd_seq	C_AND_D1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- UART_DETECTED_IND
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	cause	UART_DETECT_DTR

- TRA_DTI_REQ
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED
	dti_direction	NOT_USED

- TRA_DTI_CNF
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		30.07.02	Tlu 	Initial

### ACIDTI040: DTR detected (AT&D2)

Description: DTR detected, when DTR behavior is set to 2 (DCE perfoms a cleardown of the call (ATH)).

- Preamble: ACIDTI032

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: AT&D2)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|

- |		| 	UART_DETECTED_IND	|
	|		|	(UART_DETECT_DTR)	|
	|		*	<==========================	*

- |		|	TRA_DTI_REQ	|
	|		*	==========================>	*

- |		|	TRA_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	TRA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		| 	TRA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		| 	RA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	RA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_RELEASE_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: NO CARRIER)	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_AND_D
	cmd_seq	C_AND_D2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- UART_DETECTED_IND
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	cause	UART_DETECT_DTR

- TRA_DTI_REQ
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED
	dti_direction	NOT_USED

- TRA_DTI_CNF
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- TRA_DEACTIVATE_REQ

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- TRA_DEACTIVATE_CNF

- UART_DCD_REQ
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- RA_DEACTIVATE_REQ

- RA_DEACTIVATE_CNF

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_NO_CARRIER

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		30.07.02	Tlu 	Initial

### ACIDTI041: Escape sequence detected and ATO (flush data from send queue)

Description: Escape sequence was detected. Close connection between ACI and UART (with ATO). Data ("CONNECT") is to be flushed from send queue when conncetion is closing.

- Preamble: ACIDTI032

APL		ACI		PS

|		|		|

- |		| 	UART_DETECTED_IND	|
	|		*	<==========================	*

- |		|	TRA_DTI_REQ	|
	|		*	==========================>	*

- |		|	TRA_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: ATO)	|
	|		*	<==========================	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: CONNECT)	|
	|		*	==========================>	*

- |		| 	DTI2_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		| 	TRA_DTI_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		| 	TRA_DTI_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- UART_DETECTED_IND
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	cause	UART_DETECT_ESC

- TRA_DTI_REQ
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED
	dti_direction	NOT_USED

- TRA_DTI_CNF
	dti_conn	TRA_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_O

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_CONNECT

- DTI2_DISCONNECT_REQ	link_id	ACI_DTI_LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- TRA_DTI_REQ
	dti_conn	TRA_CONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED
	dti_direction	NOT_USED

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- TRA_DTI_CNF
	dti_conn	TRA_CONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

History:		05.09.02	Tlu 	Initial

## Non-Transparent Data Transfer

### ACIDTI051: Non-Transparent Data call over DTI (part 1)

Description:

.test connect from DTI

Preamble:

- ACIDTI002

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+CBST=71,0,1)	|
	|		*	<==========================	*

- |		| 	MNCC_CONFIGURE_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: ATD03039094444)	|
	|		*	<==========================	*

- |		| 	MNCC_SETUP_REQ	|
	|		*	==========================>	*

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_CALL_PROCEED_IND	|
	|		*	<==========================	*

- |		| 	MNCC_SYNC_IND	|
	|		*	<==========================	*

- |		| 	MNCC_SETUP_CNF	|
	|		*	<==========================	*

- |		| 	RA_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	RA_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		| 	L2R_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	L2R_XID_IND	|
	|		*	<==========================	*

- |		| 	L2R_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		| 	L2R_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	L2R_CONNECT_CNF	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: CONNECT)	|
	|		*	==========================>	*

- |		| 	DTI2_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_CBST_NONTRA

- MNCC_CONFIGURE_REQ
	called_party_sub	NOT_USED
	bcpara	S_BS_DAT_9600_ASY_NON_TRA
	sns_mode	SNS_MODE_VOICE
	ctm_ena	CTM_DISABLED

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_D_DAT

- MNCC_SETUP_REQ
	ti	NUM_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	S_BS_DAT_9600_ASY_NON_TRA
	bcpara2	S_BS_NOT_PRESENT
	called_party	S_CLD_PARTY
	called_party_sub	S_CLD_PARTY_SUB
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- MNCC_CALL_PROCEED_IND
	ti	NUM_0
	progress_desc	PROG_NOT_PRES
	ri	NOT_PRESENT_8BIT
	bcpara	S_BS_DAT_9600_ASY_NON_TRA
	bcpara2	S_BS_NOT_PRESENT

- MNCC_SYNC_IND
	ti	NOT_PRESENT_8BIT
	cause	MNCC_CAUSE_NO_MS_CAUSE
	chm	S_CHN_FULL_9600

- MNCC_SETUP_CNF
	ti	NUM_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	S_CLG_PARTY
	connected_number_sub	S_CLG_PARTY_SUB

- RA_ACTIVATE_REQ
	model	RA_MODEL_RLP
	tra_rate	TRA_FULLRATE_9600
	user_rate	URA_9600
	ndb	NUM_8
	nsb	NOT_USED

- RA_ACTIVATE_CNF
	ack_flg	RA_ACK

- L2R_ACTIVATE_REQ
	k_ms_iwf	NOT_USED
	k_iwf_ms	NOT_USED
	t1	NOT_USED
	t2	NOT_USED
	n2	NOT_USED
	pt	L2R_COMPR_TYPE_V42BIS
	p0	L2R_COMP_DIR_NONE
	p1	NOT_USED
	p2	NOT_USED
	uil2p	L2R_ISO6429
	bytes_per_prim	NOT_USED
	buffer_size	NOT_USED
	rate	L2R_FULLRATE_9600

- L2R_XID_IND
	rlp_vers	NUM_1
	k_ms_iwf	NOT_USED
	k_iwf_ms	NOT_USED
	t1	NOT_USED
	t2	NOT_USED
	n2	NOT_USED
	pt	L2R_COMPR_TYPE_V42BIS
	p0	L2R_COMP_DIR_NONE
	p1	NUM_512
	p2	NUM_6

- L2R_ACTIVATE_CNF
	ack_flg	L2R_ACK

- L2R_CONNECT_REQ

- L2R_CONNECT_CNF
	ack_flg	L2R_ACK

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_CONNECT

- DTI2_DISCONNECT_REQ	link_id	ACI_DTI_LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

History:	15.02.01	Clb	Initial
	29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI052: Non-Transparent Data call over DTI (part 2)

Description:

Preamble:

- ACIDTI051

APL		ACI		PS

|		|		|

- |		| 	L2R_DTI_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		| 	L2R_DTI_CNF	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- L2R_DTI_REQ
	dti_conn	L2R_CONNECT_DTI
	entity_name	NOT_USED
	link_id	ACI_DTI_LINK_ID_1
	dti_direction	L2R_DTI_INVERTED

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- L2R_DTI_CNF
	dti_conn	L2R_CONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

History:		27.03.01	CLB	Initial
		29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI053: L2R breaks communication

Description:

- Preamble: ACIDTI052

APL		ACI		PS

|		|		|

- |		| 	L2R_DISC_IND	|
	|		*	<==========================	*

- |		| 	L2R_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	L2R_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		| 	RA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		| 	RA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_RELEASE_IND	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: NO CARRIER)	|
	|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- L2R_DISC_IND

- L2R_DEACTIVATE_REQ

- L2R_DEACTIVATE_CNF

- UART_DCD_REQ
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- RA_DEACTIVATE_REQ

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- RA_DEACTIVATE_CNF

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	NUM_0

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_NO_CARRIER

History:		27.03.01	clb	Initial
		29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI054: Network breaks communication - Disconnect Ind

Description:

- Preamble: ACIDTI052

APL		ACI		PS

|		|		|

- |		| 	MNCC_DISCONNECT_IND	|
	|		*	<==========================	*

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_RELEASE_CNF	|
	|		*	<==========================	*

- |		| 	L2R_DTI_REQ	|
	|		*	==========================>	*

- |		| 	L2R_DTI_CNF	|
	|		*	<==========================	*

- |		| 	L2R_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		| 	L2R_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		| 	RA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	RA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: NO CARRIER)	|
	|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_DISCONNECT_IND
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR
	diagnostic	NOT_PRESENT_8BIT
	progress_desc	PROG_NOT_PRES

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_RELEASE_CNF
	ti	NOT_USED
	cause	NOT_USED

- L2R_DTI_REQ
	dti_conn	L2R_DISCONNECT_DTI
	entity_name	NOT_USED
	link_id	ACI_DTI_LINK_ID_1
	dti_direction	L2R_DTI_NORMAL

- L2R_DTI_CNF
	dti_conn	L2R_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- L2R_DEACTIVATE_REQ

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- L2R_DEACTIVATE_CNF

- UART_DCD_REQ
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- RA_DEACTIVATE_REQ

- RA_DEACTIVATE_CNF

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_NO_CARRIER

History:	27.03.01	clb	Initial
	29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI055: Network breaks communication - Release Ind

Description:

- Preamble: ACIDTI052

APL		ACI		PS

|		|		|

- |		| 	MNCC_RELEASE_IND	|
	|		*	<==========================	*

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		| 	L2R_DTI_REQ	|
	|		*	==========================>	*

- |		| 	L2R_DTI_CNF	|
	|		*	<==========================	*

- |		| 	L2R_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	L2R_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		| 	RA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		| 	RA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: NO CARRIER)	|
	|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNCC_RELEASE_IND
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- L2R_DTI_REQ
	dti_conn	L2R_DISCONNECT_DTI
	entity_name	NOT_USED
	link_id	ACI_DTI_LINK_ID_1
	dti_direction	L2R_DTI_NORMAL

- L2R_DTI_CNF
	dti_conn	L2R_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- L2R_DEACTIVATE_REQ

- L2R_DEACTIVATE_CNF

- UART_DCD_REQ
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- RA_DEACTIVATE_REQ

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- RA_DEACTIVATE_CNF

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_NO_CARRIER

History:	27.03.01	clb	Initial
	29.07.02	Tlu 	Adaptation to DTI2

### ACIDTI056: LR2 gives channel back to ACI when an escape sequence detected

Description: escape sequence detected.

- Preamble: ACIDTI052

APL		ACI		PS

|		|		|

- |		| 	UART_DETECTED_IND	|
	|		|	(UART_DETECT_ESC)	|
	|		*	<==========================	*

- |		|	L2R_DTI_REQ	|
	|		*	==========================>	*

- |		|	L2R_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- UART_DETECTED_IND
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	cause	UART_DETECT_ESC

- L2R_DTI_REQ
	dti_conn	L2R_DISCONNECT_DTI
	entity_name	NOT_USED
	link_id	ACI_DTI_LINK_ID_1
	dti_direction	NOT_USED

- L2R_DTI_CNF
	dti_conn	L2R_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		30.07.02	Tlu 	Initial

### ACIDTI057: User terminates call (ATH)

Description:

- Preamble: ACIDTI056

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: ATH)	|
	|		*	<==========================	*

- |		| 	L2R_DISC_REQ	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	L2R_DISC_CNF	|
	|		*	<==========================	*

- |		| 	L2R_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	L2R_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		| 	RA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	RA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_RELEASE_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_H

- L2R_DISC_REQ

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- L2R_DISC_CNF

- L2R_DEACTIVATE_REQ

- L2R_DEACTIVATE_CNF

- UART_DCD_REQ
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- RA_DEACTIVATE_REQ

- RA_DEACTIVATE_CNF

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		27.03.01	clb	Initial
		30.07.02	Tlu 	Adaptation to DTI2

### ACIDTI058: Return to data mode (ATO)

Description:

- Preamble: ACIDTI056

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: ATO)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: CONNECT)	|
	|		*	==========================>	*

- |		| 	DTI2_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		| 	L2R_DTI_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		| 	L2R_DTI_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_O

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_CONNECT

- DTI2_DISCONNECT_REQ	link_id	ACI_DTI_LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- L2R_DTI_REQ
	dti_conn	L2R_CONNECT_DTI
	entity_name	NOT_USED
	link_id	ACI_DTI_LINK_ID_1
	dti_direction	L2R_DTI_INVERTED

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- L2R_DTI_CNF
	dti_conn	L2R_CONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

History:		27.03.01	clb	Initial
		30.07.02	Tlu 	Adaptation to DTI2

### ACIDTI059: DTR detected (AT&D1)

Description: DTR detected, when DTR behavior is set to 1 (on reception of DTR the DCE enters command state and issues an OK result code).

- Preamble: ACIDTI052

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: AT&D1)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|

- |		| 	UART_DETECTED_IND	|
	|		|	(UART_DETECT_DTR)	|
	|		*	<==========================	*

- |		|	L2R_DTI_REQ	|
	|		*	==========================>	*

- |		|	L2R_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_AND_D
	cmd_seq	C_AND_D1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- UART_DETECTED_IND
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	cause	UART_DETECT_DTR

- L2R_DTI_REQ
	dti_conn	L2R_DISCONNECT_DTI
	entity_name	NOT_USED
	link_id	ACI_DTI_LINK_ID_1
	dti_direction	NOT_USED

- L2R_DTI_CNF
	dti_conn	L2R_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		30.07.02	Tlu 	Initial

### ACIDTI060: DTR detected (AT&D2)

Description: DTR detected, when DTR behavior is set to 2 (DCE perfoms a cleardown of the call (ATH)).

- Preamble: ACIDTI052

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: AT&D2)	|		|
	*	==========================>	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|

- |		| 	UART_DETECTED_IND	|
	|		|	(UART_DETECT_DTR)	|
	|		*	<==========================	*

- |		|	L2R_DTI_REQ	|
	|		*	==========================>	*

- |		|	L2R_DTI_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_REQ	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		| 	L2R_DISC_REQ	|
	|		*	==========================>	*

- |		| 	UART_DTI_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

- |		| 	L2R_DISC_CNF	|
	|		*	<==========================	*

- |		| 	L2R_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	L2R_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_DCD_CNF	|
	|		*	<==========================	*

- |		| 	RA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		| 	RA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		| 	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		| 	MNCC_RELEASE_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: NO CARRIER)	|
	|		*	==========================>	*

- |		| 	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_AND_D
	cmd_seq	C_AND_D2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- UART_DETECTED_IND
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	cause	UART_DETECT_DTR

- L2R_DTI_REQ
	dti_conn	L2R_DISCONNECT_DTI
	entity_name	NOT_USED
	link_id	ACI_DTI_LINK_ID_1
	dti_direction	NOT_USED

- L2R_DTI_CNF
	dti_conn	L2R_DISCONNECT_DTI
	link_id	ACI_DTI_LINK_ID_1

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	NUM_0
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- L2R_DISC_REQ

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ
	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

- L2R_DISC_CNF

- L2R_DEACTIVATE_REQ

- L2R_DEACTIVATE_CNF

- UART_DCD_REQ
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_DCD_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- RA_DEACTIVATE_REQ

- RA_DEACTIVATE_CNF

- SIM_SYNC_REQ
	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND
	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_NO_CARRIER

- DTI2_READY_IND
	link_id	ACI_DTI_LINK_ID_1

History:		30.07.02	Tlu 	Initial

## FAX Transfer

### ACIDTI071: MO FAX call (transmit 2 pages complete)

Description:

Mobile originated FAX call (transmit 2 pages complete)

Preamble:

ACIDTI002

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(ATE0;+CBST=7,0,0;+CMOD=1)	|
	|		*	<==========================	*

- |		|	MNCC_CONFIGURE_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+FCLASS=2.0)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+FNR=1,1,1,0)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(AT+FIS=1,x,0,2,0,0,0,0)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: ATD123456)	|
	|		*	<==========================	*

- |		|	MNCC_SETUP_REQ	|
	|		*	==========================>	*

- |		|	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	SIM_SYNC_CNF	|
	|		*	<==========================	*

- |		|	MNCC_CALL_PROCEED_IND	|
	|		*	<==========================	*

- |		|	MNCC_PROGRESS_IND	|
	|		*	<==========================	*

- |		|	MNCC_ALERT_IND	|
	|		*	<==========================	*

- |		|	MNCC_SYNC_IND	|
	|		*	<==========================	*

- |		|	MNCC_SETUP_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	RA_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	RA_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	T30_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	T30_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_ESCAPE_REQ	|
	|		*	==========================>	*

- |		|	T30_CONFIG_REQ	|
	|		*	==========================>	*

- |		|	T30_PREAMBLE_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: +FCO)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	T30_CAP_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(+FIS:1,3,0,2,0,0,0,0)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+FDT)	|
	|		*	<==========================	*

- |		|	T30_CAP_REQ	|
	|		*	==========================>	*

- |		|	T30_MODIFY_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(+FCS:1,3,0,2,0,0,0,0,0)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	T30_PHASE_IND	|
	|		|	(MSG)	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: CONNECT)	|
	|		*	==========================>	*

- |		|	DTI2_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	UART_DTI_REQ	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	UART_DTI_CNF	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	T30_DTI_REQ	|
	|		|	(T30_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	T30_DTI_CNF	|
	|		|	(T30_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	T30_PHASE_IND	|
	|		|	(BCS)	|
	|		*	<==========================	*

- |		|	T30_DTI_REQ	|
	|		|	(T30_DISCONNECT_DTI)	|
	|		*	==========================>	*

- |		|	T30_DTI_CNF	|
	|		|	(T30_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_REQ	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_CNF	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	T30_SGN_IND	|
	|		|	(SGN_MPS)	|
	|		*	<==========================	*

- |		|	T30_SGN_REQ	|
	|		|	(cmd: SGN_MPS)	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)		|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+FDT)	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	T30_PHASE_IND	|
	|		|	(MSG)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: CONNECT)	|
	|		*	==========================>	*

- |		|	DTI2_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	UART_DTI_REQ	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	T30_DTI_REQ	|
	|		|	(T30_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	UART_DTI_CNF	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	T30_DTI_CNF	|
	|		|	(T30_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	T30_PHASE_IND	|
	|		|	(BCS)	|
	|		*	<==========================	*

- |		|	T30_DTI_REQ	|
	|		|	(T30_DISCONNECT_DTI)	|
	|		*	==========================>	*

- |		|	T30_DTI_CNF	|
	|		|	(T30_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_REQ	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_CNF	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	T30_SGN_IND	|
	|		|	(SGN_EOP)	|
	|		*	<==========================	*

- |		|	T30_SGN_REQ	|
	|		|	(SGN_EOP)	|
	|		*	==========================>	*

- |		|	T30_CMPL_IND	|
	|		|	(CMPL_EOP)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: +FHS:00)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	T30_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	T30_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_ESCAPE_REQ	|
	|		*	==========================>	*

- |		|	RA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	RA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		|	MNCC_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	MNCC_RELEASE_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(AT)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PLUS_CMOD_FAX

- MNCC_CONFIGURE_REQ	called_party_sub	NOT_USED
	bcpara	NOT_USED
	sns_mode	NOT_USED
	ctm_ena	CTM_DISABLED

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PLUS_FCLASS_2_0

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PLUS_FNR

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PLUS_FIS3

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_D_DAT2

- MNCC_SETUP_REQ	ti	NUM_0
	prio	PRIO_NORM_CALL
	ri	RI_CIRCULAR
	bcpara	S_BS_FAX
	bcpara2	S_BS_VOICE
	called_party	S_CLD_PARTY
	called_party_sub	S_CLD_PARTY_SUB
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ	synccs	SYNC_START_CALL

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- SIM_SYNC_CNF	cause	SIM_NO_ERROR

- MNCC_CALL_PROCEED_IND	ti	NUM_0
	progress_desc	PROG_NOT_PRES
	ri	NOT_PRESENT_8BIT
	bcpara	S_BS_FAX
	bcpara2	S_BS_NOT_PRESENT

- MNCC_PROGRESS_IND	ti	NUM_0
	progress_desc	PROG_NOT_PRES

- MNCC_ALERT_IND	ti	NUM_0
	progress_desc	PROG_NOT_PRES

- MNCC_SYNC_IND	ti	NOT_PRESENT_8BIT
	cause	MNCC_CAUSE_NO_MS_CAUSE
	chm	S_CHN_FULL_9600

- MNCC_SETUP_CNF	ti	NUM_0
	cause	MNCC_CAUSE_SUCCESS
	progress_desc	PROG_NOT_PRES
	connected_number	S_CLG_PARTY
	connected_number_sub	S_CLG_PARTY_SUB

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- RA_ACTIVATE_REQ	model	RA_MODEL_FAX
	tra_rate	TRA_FULLRATE_9600
	user_rate	URA_9600
	ndb	NUM_8
	nsb	NUM_1

- RA_ACTIVATE_CNF	ack_flg	RA_ACK

- T30_ACTIVATE_REQ	trans_rate	NUM_9600
	half_rate	NUM_0
	threshold	NUM_DEC_90
	frames_per_prim	NUM_3
	bitorder	NUM_0

- T30_ACTIVATE_CNF	buf_size_rx	NUM_4800
	buf_size_tx	NUM_4800

- UART_DCD_REQ	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_ON

- UART_ESCAPE_REQ	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	detection	UART_ESC_DETECTION_OFF

- T30_CONFIG_REQ	hdlc_report	NUM_1
	test_mode	NOT_USED

- T30_PREAMBLE_IND

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PLUS_FCO

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- T30_CAP_IND	hdlc_info	S_HDLC_DIS_9600

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PLUS_FIS

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PLUS_FDT

- T30_CAP_REQ	hdlc_info	NOT_USED

- T30_MODIFY_REQ	trans_rate	NUM_9600
	half_rate	NUM_0

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PLUS_FCS13

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- T30_PHASE_IND	phase	MSG_PHASE

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_CONNECT

- DTI2_DISCONNECT_REQ	link_id	ACI_DTI_LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	FALSE
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- T30_DTI_REQ	dti_conn	T30_CONNECT_DTI
	entity_name	NOT_USED
	link_id	NOT_USED
	dti_direction	T30_DTI_INVERTED

- T30_DTI_CNF	dti_conn	T30_CONNECT_DTI

- T30_PHASE_IND	phase	BCS_PHASE

- T30_DTI_REQ	dti_conn	T30_DISCONNECT_DTI
	entity_name	NOT_USED
	link_id	NOT_USED
	dti_direction	T30_DTI_INVERTED

- T30_DTI_CNF	dti_conn	T30_DISCONNECT_DTI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	FALSE
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- T30_SGN_IND	sgn	SGN_MPS

- T30_SGN_REQ	sgn	SGN_MPS

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PLUS_FDT

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- T30_PHASE_IND	phase	MSG_PHASE

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_CONNECT

- DTI2_DISCONNECT_REQ	link_id	ACI_DTI_LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	FALSE
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- T30_DTI_REQ	dti_conn	T30_CONNECT_DTI
	entity_name	NOT_USED
	link_id	NOT_USED
	dti_direction	T30_DTI_INVERTED

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- T30_DTI_CNF	dti_conn	T30_CONNECT_DTI

- T30_PHASE_IND	phase	BCS_PHASE

- T30_DTI_REQ	dti_conn	T30_DISCONNECT_DTI
	entity_name	NOT_USED
	link_id	NOT_USED
	dti_direction	T30_DTI_INVERTED

- T30_DTI_CNF	dti_conn	T30_DISCONNECT_DTI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	FALSE
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- T30_SGN_IND	sgn	SGN_EOP

- T30_SGN_REQ	sgn	SGN_EOP

- T30_CMPL_IND	cmpl	CMPL_EOP

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PLUS_FHS

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- T30_DEACTIVATE_REQ

- T30_DEACTIVATE_CNF

- UART_DCD_REQ	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_ESCAPE_REQ	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	detection	UART_ESC_DETECTION_ON

- RA_DEACTIVATE_REQ

- RA_DEACTIVATE_CNF

- SIM_SYNC_REQ	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND	ti	NUM_0
	cause	MNCC_CAUSE_CALL_CLEAR

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

History:		27.06.2001		KJF	Initial
		06.09.2002		TLU	UART_ESCAPE_REQ inserted

07.10.2002		TLU	"OK" added at the end of the TC

### ACIDTI072: MT FAX call (receive 2 pages complete)

Description:

Mobile terminated FAX call (receive 2 pages complete)

Preamble:

ACIDTI002

APL		ACI		PS

|		|		|

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(ATE0;+CBST=7,0,0;+CMOD=1)	|
	|		*	<==========================	*

- |		|	MNCC_CONFIGURE_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	MNCC_SETUP_IND	|
	|		*	<==========================	*

- |		|	MNCC_ALERT_REQ	|
	|		*	==========================>	*

- |		|	UART_RING_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: RING)	|
	|		*	==========================>	*

- |	ACI_CMD_IND	|		|
	|	(msg: RING)	|		|
	*	<==========================	*		|

- |		|	UART_RING_CNF	|
	|		*	<==========================	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: ATA)	|
	|		*	<==========================	*

- |		|	MNCC_SETUP_RES	|
	|		*	==========================>	*

- |		|	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		|	MNCC_SYNC_IND	|
	|		*	<==========================	*

- |		|	MNCC_SETUP_COMPL_IND	|
	|		*	<==========================	*

- |		|	UART_RING_REQ	|
	|		*	==========================>	*

- |		|	UART_RING_CNF	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	RA_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	RA_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	T30_ACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	T30_ACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_ESCAPE_REQ	|
	|		*	==========================>	*

- |		|	T30_CONFIG_REQ	|
	|		*	==========================>	*

- |		|	T30_CAP_REQ	|
	|		*	==========================>	*

- |		|	T30_PREAMBLE_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: +FCO)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	T30_CAP_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+FDR)	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	T30_PHASE_IND	|
	|		|	(MSG)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: CONNECT)	|
	|		*	==========================>	*

- |		|	DTI2_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	UART_DTI_REQ	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	T30_DTI_REQ	|
	|		|	(T30_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	UART_DTI_CNF	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	T30_DTI_CNF	|
	|		|	(T30_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	T30_PHASE_IND	|
	|		|	(BCS)	|
	|		*	<==========================	*

- |		|	T30_DTI_REQ	|
	|		|	(T30_DISCONNECT_DTI)	|
	|		*	==========================>	*

- |		|	T30_DTI_CNF	|
	|		|	(T30_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_REQ	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_CNF	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	T30_EOL_IND	|
	|		|	(eol)	|
	|		*	<==========================	*

- |		|	T30_SGN_IND	|
	|		|	(SGN_MPS)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: +FPS:1,0,0,0,0)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: +FET:0)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+FDR)	|
	|		*	<==========================	*

- |		|	T30_SGN_REQ	|
	|		|	(SGN_MCF)	|
	|		*	==========================>	*

- |		|	T30_PHASE_IND	|
	|		|	(MSG)	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: CONNECT)	|
	|		*	==========================>	*

- |		|	DTI2_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	UART_DTI_REQ	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	UART_DTI_CNF	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	T30_DTI_REQ	|
	|		|	(T30_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	T30_DTI_CNF	|
	|		|	(T30_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	T30_PHASE_IND	|
	|		|	(BCS)	|
	|		*	<==========================	*

- |		|	T30_DTI_REQ	|
	|		|	(T30_DISCONNECT_DTI)	|
	|		*	==========================>	*

- |		|	T30_DTI_CNF	|
	|		|	(T30_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	UART_DTI_IND	|
	|		|	(UART_DISCONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_CONNECT_REQ	|
	|		*	==========================>	*

- |		|	UART_DTI_REQ	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	==========================>	*

- |		|	DTI2_CONNECT_CNF	|
	|		*	<==========================	*

- |		|	UART_DTI_CNF	|
	|		|	(UART_CONNECT_DTI)	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	T30_EOL_IND	|
	|		|	(eol)	|
	|		*	<==========================	*

- |		|	T30_SGN_IND	|
	|		|	(SGN_EOP)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: +FPS:1,0,0,0,0)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: +FET:2)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT+FDR)	|
	|		*	<==========================	*

- |		|	T30_SGN_REQ	|
	|		|	(SGN_MCF)	|
	|		*	==========================>	*

- |		|	T30_CMPL_IND	|
	|		|	(CMPL_EOP)	|
	|		*	<==========================	*

- |		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: +FHS:00)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	T30_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	T30_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	UART_DCD_REQ	|
	|		*	==========================>	*

- |		|	UART_ESCAPE_REQ	|
	|		*	==========================>	*

- |		|	RA_DEACTIVATE_REQ	|
	|		*	==========================>	*

- |		|	RA_DEACTIVATE_CNF	|
	|		*	<==========================	*

- |		|	SIM_SYNC_REQ	|
	|		*	==========================>	*

- |		|	MNCC_DISCONNECT_REQ	|
	|		*	==========================>	*

- |		|	MNCC_RELEASE_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

- |		|	DTI2_READY_IND	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_IND	|
	|		|	(AT)	|
	|		*	<==========================	*

- |		|	DTI2_DATA_TEST_REQ	|
	|		|	(msg: OK)	|
	|		*	==========================>	*

|		|		|

Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PLUS_CMOD_FAX

- MNCC_CONFIGURE_REQ	called_party_sub	NOT_USED
	bcpara	NOT_USED
	sns_mode	NOT_USED
	ctm_ena	CTM_DISABLED

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- MNCC_SETUP_IND	ti	NUM_8
	ri	RI_CIRCULAR
	bcpara	S_BS_FAX
	bcpara2	S_BS_VOICE
	progress_desc	PROG_NOT_PRES
	sig	SIG_RING_BACK_TONE_ON
	calling_party	S_CLG_PARTY
	calling_party_sub	S_CLG_PARTY_SUB
	called_party	S_CLD_PARTY
	called_party_sub	S_CLD_PARTY_SUB
	redirecting_party	NOT_USED
	redirecting_party_sub	NOT_USED

- MNCC_ALERT_REQ	ti	NUM_8

- UART_RING_REQ	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_ON

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RING

- ACI_CMD_IND	cmd_len	LM_RING
	cmd_seq	M_RING

- UART_RING_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_C_ATA

- MNCC_SETUP_RES	ti	NUM_8

- SIM_SYNC_REQ	synccs	SYNC_START_CALL

- MNCC_SYNC_IND	ti	NOT_PRESENT_8BIT
	cause	MNCC_CAUSE_NO_MS_CAUSE
	chm	S_CHN_FULL_9600

- MNCC_SETUP_COMPL_IND	ti	NUM_8
	cause	MNCC_CAUSE_SUCCESS

- UART_RING_REQ	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_RING_CNF
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- RA_ACTIVATE_REQ	model	RA_MODEL_FAX
	tra_rate	TRA_FULLRATE_9600
	user_rate	URA_9600
	ndb	NUM_8
	nsb	NUM_1

- RA_ACTIVATE_CNF	ack_flg	RA_ACK

- T30_ACTIVATE_REQ	trans_rate	NUM_9600
	half_rate	NUM_0
	threshold	NUM_DEC_90
	frames_per_prim	NUM_3
	bitorder	NUM_0

- T30_ACTIVATE_CNF	buf_size_rx	NUM_4800
	buf_size_tx	NUM_4800

- UART_DCD_REQ	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_ON

- UART_ESCAPE_REQ	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	detection	UART_ESC_DETECTION_OFF

- T30_CONFIG_REQ	hdlc_report	NUM_1
	test_mode	NOT_USED

- T30_CAP_REQ	hdlc_info	NOT_USED

- T30_PREAMBLE_IND

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PLUS_FCO

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- T30_CAP_IND	hdlc_info	S_HDLC_DCS

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PLUS_FDR

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- T30_PHASE_IND	phase	MSG_PHASE

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_CONNECT

- DTI2_DISCONNECT_REQ	link_id	ACI_DTI_LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	FALSE
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- T30_DTI_REQ	dti_conn	T30_CONNECT_DTI
	entity_name	NOT_USED
	link_id	NOT_USED
	dti_direction	T30_DTI_INVERTED

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- T30_DTI_CNF	dti_conn	T30_CONNECT_DTI

- T30_PHASE_IND	phase	BCS_PHASE

- T30_DTI_REQ	dti_conn	T30_DISCONNECT_DTI
	entity_name	NOT_USED
	link_id	NOT_USED
	dti_direction	T30_DTI_INVERTED

- T30_DTI_CNF	dti_conn	T30_DISCONNECT_DTI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	FALSE
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- T30_EOL_IND	eol	EOL_0

- T30_SGN_IND	sgn	SGN_MPS

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PLUS_FPS

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PLUS_FET0

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PLUS_FDR

- T30_SGN_REQ	sgn	SGN_MCF

- T30_PHASE_IND	phase	MSG_PHASE

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_CONNECT

- DTI2_DISCONNECT_REQ	link_id	ACI_DTI_LINK_ID_1
	cause	DTI_CAUSE_NORMAL_CLOSE

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	FALSE
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- T30_DTI_REQ	dti_conn	T30_CONNECT_DTI
	entity_name	NOT_USED
	link_id	NOT_USED
	dti_direction	T30_DTI_INVERTED

- T30_DTI_CNF	dti_conn	T30_CONNECT_DTI

- T30_PHASE_IND	phase	BCS_PHASE

- T30_DTI_REQ	dti_conn	T30_DISCONNECT_DTI
	entity_name	NOT_USED
	link_id	NOT_USED
	dti_direction	T30_DTI_INVERTED

- T30_DTI_CNF	dti_conn	T30_DISCONNECT_DTI

- UART_DTI_IND
	dti_conn	UART_DISCONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_CONNECT_REQ
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_REQ
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	direction	FALSE
	link_id	ACI_DTI_LINK_ID_1
	entity_name	NOT_USED

- DTI2_CONNECT_CNF
	link_id	ACI_DTI_LINK_ID_1
	version	DTI_VERSION_10

- UART_DTI_CNF
	dti_conn	UART_CONNECT_DTI
	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- T30_EOL_IND	eol	EOL_0

- T30_SGN_IND	sgn	SGN_EOP

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PLUS_FPS

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PLUS_FET2

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PLUS_FDR

- T30_SGN_REQ	sgn	SGN_MCF

- T30_CMPL_IND	cmpl	CMPL_EOP

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PLUS_FHS

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- T30_DEACTIVATE_REQ

- T30_DEACTIVATE_CNF

- UART_DCD_REQ	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	line_state	UART_LINE_OFF

- UART_ESCAPE_REQ	device	NUM_0
	dlci	UART_DLCI_NOT_MULTIPLEXED
	detection	UART_ESC_DETECTION_ON

- RA_DEACTIVATE_REQ

- RA_DEACTIVATE_CNF

- SIM_SYNC_REQ	synccs	SYNC_STOP_CALL

- MNCC_DISCONNECT_REQ	ti	NUM_8
	cause	MNCC_CAUSE_CALL_CLEAR
	fac_inf	NOT_USED
	ss_version	NOT_USED

- MNCC_RELEASE_IND	ti	NUM_8
	cause	MNCC_CAUSE_CALL_CLEAR

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_IND
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT

- DTI2_DATA_TEST_REQ	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

History:		27.06.2001	ACI	Initial
		07.10.2002	TLU	UART_ESCAPE_REQ inserted
		07.10.2002	TLU	"OK" added at the end of the TC

## AT commands to data flow

### ACIDTI080: Entity PKTIO registers in ACI, successfull

Description:

Variants:

- <A>...<B>

Preamble:

- ACIDTI002

APL		ACI		PS

|		|		|

(1)	|		|	PKT_CONNECT_IND	|
	|		*	<==========================	*

(2)	|		|	PKT_CONNECT_RES	|
	|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- PKT_CONNECT_IND
	device_no	NUM_0
<A>	dio_dcb	DEFAULT_DCB_FROM_DIO_1
<B>	dio_dcb	DEFAULT_DCB_FROM_DIO_2

- PKT_CONNECT_RES
	device_no	NUM_0
<A>	dio_dcb	DEFAULT_DCB_FROM_ACI_1
<B>	dio_dcb	DEFAULT_DCB_FROM_ACI_2

History:	11.11.02	RM	Initial

### ACIDTI081:Set Data Flow for packet data

Description:

Preamble:

- ACIDTI080A

APL		ACI		PS

|		|		|

- (1)	|		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT%DATA)	|
	|		*	<==========================	*

- (2)	|		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- (3)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- (4)	|		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PERC_DATA

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

History:	20.01.03	RM	Initial

### ACIDTI082:Query Data Flow for packet data

Description:

Preamble:

- ACIDTI081

APL		ACI		PS

|		|		|

- (1)	|		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT%DATA?)	|
	|		*	<==========================	*

- (2)	|		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: %DATA:..)	|
	|		*	==========================>	*

- (3)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- (4)	|		|	DTI2_READY_IND	|
	|		*	<==========================	*

- (5)	|		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- (6)	|		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PERC_DATA_Q

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PERC_DATA

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

History:	20.01.03	RM	Initial

### ACIDTI083:Query Data Flow for packet data, no redirection has been set

Description: in difference to ACIDTI082 we use here another preamble, so that there has not been set a   redirection.

Preamble:

- ACIDTI002

APL		ACI		PS

|		|		|

- (1)	|		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT%DATA?)	|
	|		*	<==========================	*

- (2)	|		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: %DATA:..)	|
	|		*	==========================>	*

- (3)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- (4)	|		|	DTI2_READY_IND	|
	|		*	<==========================	*

- (5)	|		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- (6)	|		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PERC_DATA_Q

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PERC_DATA_NO_RED

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

History:	20.01.03	RM	Initial

### ACIDTI084: Show data channel information for current channel

Description: with ACIDTI002 we initialised a UART, so we have an AT cmd channel

Preamble:

- ACIDTI002

APL		ACI		PS

|		|		|

- (1)	|		|	DTI2_DATA_TEST_IND	|
	|		|	(cmd: AT%DINF=0)	|
	|		*	<==========================	*

- (2)	|		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: %DINF:..)	|
	|		*	==========================>	*

- (3)	|		|	DTI2_GETDATA_REQ	|
	|		*	==========================>	*

- (4)	|		|	DTI2_READY_IND	|
	|		*	<==========================	*

- (5)	|		|	DTI2_DATA_TEST_REQ	|
	|		|	(cmd: OK)	|
	|		*	==========================>	*

- (6)	|		|	DTI2_READY_IND	|
	|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- DTI2_DATA_TEST_IND	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_AT_PERC_DINF_Q

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_PERC_DINF

- DTI2_GETDATA_REQ	link_id	ACI_DTI_LINK_ID_1

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

- DTI2_DATA_TEST_REQ
	link_id	ACI_DTI_LINK_ID_1
	parameters	NOT_USED
	sdu	F_RESP_OK

- DTI2_READY_IND	link_id	ACI_DTI_LINK_ID_1

History:	20.01.03	RM	Initial

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |