| Date: | 19 June, 2002 |
| --- | --- |
| ID: | 8411.417.98.003 |
| Status: | Being Processed |

| Author: | Condat AG Alt Moabit 90a 10559 Berlin Germany |
| --- | --- |

| Condat Proprietary Information NDA - Confidential Do Not Copy |  |
| --- | --- |

G23-GSM Protocol Stack

ACI SS

Entity Test Specification

GSM

- Table of Contents

0	Document Control	5

0.1	Document History	5

0.2	References	6

0.3	Abbreviations	9

0.4	Terms	11

1	Overview	12

1.1	RA - Rate Adaptation	12

1.2	RLP - Radio Link Protocol	12

1.3	L2R - Layer 2 Relay Functionality	12

1.4	FAD 03.45 - Fax Adaptation Protocol	13

1.5	T.30 - Fax Protocol Entity	13

1.6	ACI - AT Command Interpreter	13

1.7	USART - Universal Synchronous Asynchronous Receiver Transmitter Driver	13

2	Parameters	14

3	TEST CASES	96

3.1	Routing (internal) (ACI001 – ACI010)	96

3.1.1	ACISS001:  Setup the Routing and the PCO view for the ACI test	96

3.2	Line identification supplementary services (ACI011 – ACI099)	97

3.2.1	ACISS011:  Interrogate CLIP status	97

3.2.2	ACISS012:  Interrogate CLIR status	97

3.2.3	ACISS013:  Interrogate COLP status	98

3.2.4	ACISS020:  Successful interrogation of CLIP, provisioned	98

3.2.5	ACISS021:  Successful interrogation of CLIR	99

3.3	Call Waiting (ACI040-ACI059)	100

3.3.1	ACISS040:  Interrogate Call waiting status	100

3.3.2	ACISS041:  Activate Call waiting status	101

3.3.3	ACISS042:  Deactivate Call Waiting Voice/Data/Fax	102

3.3.4	ACISS043:  Deactivate Call Waiting Voice/Data/Fax: error case	104

3.4	Call Forwarding (ACI060-ACI079)	105

3.4.1	ACISS060:  Interrogate Call Forwarding status	105

3.4.2	ACISS061:  Register Call Forwarding Voice/Data/Fax	105

3.4.3	ACISS062:  Erase Call Forwarding Voice/Data/Fax	106

3.4.4	ACISS063:  Activate Call Forwarding Voice/Data/Fax	107

3.4.5	ACISS064:  Deactivate Call Forwarding Voice/Data/Fax	108

3.4.6	ACISS065:  Successful interrogation of Call Forwarding CFU	109

3.4.7	ACISS066:  Successful interrogation of Call Forwarding CFB	110

3.4.8	ACISS067:  Successful interrogation of Call Forwarding CFNRY	111

3.4.9	ACISS068:  Successful interrogation of Call Forwarding	111

3.5	SS Notification (ACI080-ACI099)	112

3.5.1	ACISS080:  SS Notify	112

3.5.2	ACISS081:  Forward Check SS Indication	114

3.5.3	ACISS082:  SS Notify for ECT and override case for Redirection Number (RDN)	116

3.5.4	ACISS083:  SS Notify: MT call FTA 31.2.1.7.1.1	117

3.6	Closed User Group (ACI0100-ACI109)	119

3.6.1	ACISS100:  CUG with temporary mode enabled                        FAILS: to be analyzed	119

3.6.2	ACISS101:  CUG with default parameters                        FAILS: to be analyzed	120

3.7	SS Password (ACI0110-ACI119)	121

3.7.1	ACISS110:  Change Password, successful attempt	121

3.7.2	ACISS111:  Change Password, subscription violation	122

3.7.3	ACISS112:  Change Password, negative password check	123

3.7.4	ACISS113:  Change Password, new password mismatch	124

3.8	Call Barring (ACI120-ACI139)	125

3.8.1	ACISS120:  Interrogate Call Barring status	125

3.8.2	ACISS121:  Activate Call Forwarding Voice/Data/Fax	126

3.8.3	ACISS122:  Deactivate Call Barring Voice/Data/Fax	128

3.8.4	ACISS123:  Activate Call Forwarding Voice/Data/Fax/SMS	131

3.8.5	ACISS124:  Deactivate Call Barring Voice/Data/Fax/SMS	133

3.8.6	ACISS125:  Activate Call Barring no class entered	135

3.9	Unstructured SS (ACI0140-ACI149)	137

3.9.1	ACISS140:  Unstructured SS notify	137

3.9.2	ACISS141:  Unstructured SS request	138

3.9.3	ACISS142:  Process Unstructured SS request, no network request involved	139

3.9.4	ACISS143:  Process Unstructured SS request, with network request involved	141

3.9.5	ACISS144:  Process Unstructured SS request, network supports only version 1 protocol	144

3.9.6	ACISS145:  Process Unstructured SS request, network does not support USSD	145

3.9.7	ACISS146:  Unstructured SS control string, network supports only version 1 protocol	147

3.9.8	ACISS147:  Unstructured SS operation in parallel to other SS operation. Network initiated	148

3.10	PIN Modification (ACI0150-ACI169)	149

3.10.1	ACISS150:  Change PIN 1	149

3.10.2	ACISS151:  Change PIN 2	150

3.10.3	ACISS152:  enable PIN 1	151

3.10.4	ACISS153:  disable PIN 1	152

3.10.5	ACISS154:  Query Enabled PIN 1	153

3.10.6	ACISS155:  Query Disabled PIN 1	153

3.10.7	ACISS156:  Query Unknown PIN 1 Status	154

3.11	Keystroke Sequences (ACI170 – ACI249)	155

3.11.1	ACISS170:  Register CFNRy 31.2.1.1.1	155

3.11.2	ACISS171:  Register CFU 31.2.1.1.1	156

3.11.3	ACISS172:  Register CFU 31.2.1.1.2	157

3.11.4	ACISS173:  Register CF 31.2.1.1.2	158

3.11.5	ACISS174:  Erase CFC 31.2.1.2.1	158

3.11.6	ACISS175:  Erase CFNRC 31.2.1.2.1	159

3.11.7	ACISS176:  Erase CFU 31.2.1.2.2	160

3.11.8	ACISS177:  Erase CFNRY 31.2.1.2.2	161

3.11.9	ACISS178:  Activate CF 31.2.1.3	162

3.11.10	ACISS179:  Activate CFU 31.2.1.3	163

3.11.11	ACISS180:  Deactivate CFC 31.2.1.4	164

3.11.12	ACISS181:  Deactivate CFNRC 31.2.1.4	165

3.11.13	ACISS182:  Interrogate CFB 31.2.1.6.1, 1 CCFC	166

3.11.14	ACISS183:  Interrogate CFB 31.2.1.6.1, 3 CCFC             FAILS !!!! (to be processed)	167

3.11.15	ACISS184:  Interrogate CFNRY, Speech 31.2.1.6.1	168

3.11.16	ACISS185:  Interrogate CFNRC 31.2.1.6.2	169

3.11.17	ACISS186:  Interrogate CFB FAX 31.2.1.6.2	170

3.11.18	ACISS190:  Register Password CB all 31.8.1.1	171

3.11.19	ACISS191:  Activate BAOC  31.8.3.1	173

3.11.20	ACISS192:  Activate BICR  31.8.3.1	174

3.11.21	ACISS193:  Activate BOIC  31.8.3.2.1	175

3.11.22	ACISS194:  Activate BAIC  31.8.3.2.2	176

3.11.23	ACISS195:  Deactivate all barring services  31.8.4.1	176

3.11.24	ACISS196:  Deactivate all outgoing barring services  31.8.4.1	177

3.11.25	ACISS197:  Deactivate all incoming barring services  31.8.4.2.1	178

3.11.26	ACISS198:  Deactivate BOIC except Home  31.8.4.2.2	179

3.11.27	ACISS199:  Interrogate BAIC  31.8.6.1, 1 CLCK	180

3.11.28	ACISS200:  Interrogate BAIC  31.8.6.1, 3 CLCKs	181

3.11.29	ACISS201:  Interrogate BOIC except Home 31.8.6.1	183

3.11.30	ACISS202:  Interrogate BIC roaming 31.8.6.2	184

3.11.31	ACISS203:  Interrogate BOIC  31.8.6.2	185

3.11.32	ACISS204:  Unstructured SS  31.9	185

3.11.33	ACISS210:  Change PIN 1 via Key Sequence	186

3.11.34	ACISS211:  Unblock PIN1 via Key Sequence	187

3.11.35	ACISS212:  Unblock PIN2 via Key Sequence	188

3.11.36	ACISS213:  Present IMEI via Key Sequence	189

3.11.37	ACISS214:  Deactivate BAOC	190

3.11.38	ACISS215:  Interrogate CLIR	191

3.11.39	ACISS216:  Register Password, unsuccessful attempt CB all	192

3.11.40	ACISS217:  Activate CW	193

3.11.41	ACISS218:  Temporary CLIR Modification During Call Setup	194

3.11.42	ACISS219:  Erase CCBS Entry	195

3.11.43	ACISS230:  Interrogate CCBS Entries: success        FAILS !!!!!	196

3.11.44	ACISS231:  Interrogate CCBS Entries: error	198

3.11.45	ACISS249:  Key Sequence	199

3.12	Call Completion Busy Subscriber (250 - 269)	200

3.12.1	ACISS250:  Clear CCBS Entries        FAILS !!!	200

3.12.2	ACISS251:  Interrogate CCBS Entries, 3 entries are registered    FAILS !!!	201

3.12.3	ACISS252:  Interrogate CCBS Entries, no entries are registered, status is displayed     FAILS !!!	202

# Document Control

ã Copyright Condat AG, 2002

All rights reserved.

Every effort has been made to ensure that the information contained in this document is accurate at the time of printing. However, the software described in this document is subject to continuous development and improvement. Condat AG reserves the right to change the specification of the software. Information in this document is subject to change without notice and does not represent a commitment on the part of Condat AG. Condat AG accepts no liability for any loss or damage arising from the use of any information contained in this document.

The software described in this document is furnished under a license agreement and may be used or copied only in accordance with the terms of the agreement. It is an offence to copy the software in any way except as specifically set out in the agreement. No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Condat AG.

- Condat AG

Alt Moabit 90a

10559 Berlin

Germany

- Telephone:	+49.30.39 49 0

Fax:		+49.30.39 49 1300

Internet:	www.condat.de

## Document History

ID	Author	Date	Status
8411.417.98.001	AK	08 January, 1998	Being Processed

Initial version.

8411.417.98.002	SBK	14 January, 2002	Being Processed

8411.417.98.003	CLB	14 January, 2002	Being Processed

Adapted e.g. due to ACI-FIX-1666

## References

[1]	Rec. T.4 Standardisation of group 3 facsimile apparatus for document transmission;
(CCITT-T.4, 1984)

[2]	ITU-T Recommendation T.30; Series T: Terminal equipments and protocols for telematic services;
Procedures for document facsimile transmission in the general switched
telephone network;
(ITU-T.30, 1996)

[3]	ITU-T Recommendation T.31; Terminals for telematic services;
Asynchronous facsimile DCE control - service class 1
(ITU-T.31, 1995)

[4]	ITU-T Recommendation T.32; Terminals for telematic services;
Asynchronous facsimile DCE control - service class 2
(ITU-T.32, 1995)

[5]	Rec. T.35; Terminal equipment and protocols for telematic services;
Procedures for the allocation of CCITT definde codes for non-standard facilities;
(CCITT-T.35, 1991)

[6]	ITU-T Recommendation V.25 ter; Series V: data communication over the telephone network;
Interfaces and voiceband modems; Serial asynchronous automatic dialling and control
(ITU-T V.25 ter, 1997)

[7]	Rec. V.42 bis Data compression procedures for data circuit terminating equipment (DCE) using error correction procedures;
(CCITT-V.42 bis, 1990)

[8]	Rec. V.110 (Blue book, Vol. VIII, Fascicle VIII.1) Support of data terminal equipments (DTEs) with V-series type interfaces by an integrated services digital network (ISDN);
(CCITT-V.110, 1988)

[9]	European digital cellular telecommunications system (Phase 2);
GSM Public Land Mobile Network (PLMN) connection types;
(GSM 3.10, September 1994, version 4.3.1)

[10]	European digital cellular telecommunications system (Phase 2);
Technical realisation of facsimile group 3 transparent;
(GSM 3.45, September 1995, version 4.5.0)

[11]	Digital cellular telecommunications system (Phase 2);
Mobile radio interface layer 3 specification;
(GSM 4.08, November 1996, version 4.17.0)

[12]	European digital cellular telecommunications system (Phase 2);
Rate adaptation on the Mobile Station - Base Station System (MS - BSS) Interface;
(GSM 4.21, May 1995, version 4.6.0)

[13]	European digital cellular telecommunications system (Phase 2);
Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-service Switching Centre (BSS - MSC) interface
(GSM 4.22, September 1994, version 4.3.0)

[14]	European digital cellular telecommunications system (Phase 2);
Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-service Switching Centre (BSS - MSC) interface
(Amendment prA1 for GSM 4.22, version 4.3.0)
(GSM 4.22, March 1995, version 4.4.0)

[15]	European digital cellular telecommunications system (Phase 2);
General on Terminal Adaptation Functions (TAF) for Mobile Stations (MS);
(GSM 7.01, December 1995, version 4.10.0)

[16]	European digital cellular telecommunications system (Phase 2);
Terminal Adaptation Functions (TAF) for services using asynchronous bearer capabilities;
(GSM 7.02, September 1994, version 4.5.1)

[17]	European digital cellular telecommunications system (Phase 2);
Terminal Adaptation Functions (TAF) for services using synchronous bearer capabilities;
(GSM 7.03, September 1994, version 4.5.1)

[18]	Digital cellular telecommunications system (Phase 2);
Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Services (CBS);
(GSM 7.05, November 1996, version 4.8.0)

[19]	Digital cellular telecommunications system (Phase 2);
AT command set for GSM Mobile Equipment (ME)
(GSM 7.07, May 1996, version 4.1.0)

[20]	Digital cellular telecommunication system (Phase 2);
Mobile Station (MS) conformance specification;
Part 1: Conformance specification
(GSM 11.10-1, November 1996, version 4.17.0)

[21]	Digital cellular telecommunications system (Phase 2);
Mobile Station (MS) conformance specification;
Part 2: Protocol Implementation Conformance Statement (PICS)
proforma specification
(GSM 11.10-2, May 1996, version 4.15.0)

[22]	Digital cellular telecommunications system (Phase 2);
Mobile Station (MS) conformance specification;
Part 3: Layer 3 (L3) Abstract Test Suite (ATS)
(GSM 11.10-3, November 1996, version 4.17.0)

[23]	Proposal for Rate Adaptation implemented on a DSP;
(C. Bianconi, Texas Instruments, January 1998, version 1.0)

[24]	MCU-DSP Interfaces for Data Applications;
Specification S844
(C. Bianconi, Texas Instruments, March 1998, version 0.1)

[25]	Users Guide
6147.300.96.100; Condat GmbH

[26]	Service Access Point RA
8411.100.98.100; Condat GmbH

[27]	Service Access Point RLP
8411.101.98.100; Condat GmbH

[28]	Service Access Point L2R
8411.102.98.100; Condat GmbH

[29]	Service Access Point FAD
8411.103.98.100; Condat GmbH

[30]	Service Access Point T30
8411.104.98.100; Condat GmbH

[31]	Service Access Point ACI
8411.105.98.100; Condat GmbH

[32]	Message Sequence Charts RLP
8411.201.98.100; Condat GmbH

[33]	Message Sequence Charts L2R
8411.202.98.100; Condat GmbH

[34]	Message Sequence Charts FAD
8411.203.98.100; Condat GmbH

[35]	Message Sequence Charts T30
8411.204.98.100; Condat GmbH

[36]	Message Sequence Charts ACI
8411.205.98.100; Condat GmbH

[37]	Proposal for Fax & Data Integration; March 1998
8411.300.98.100; Condat GmbH

[38]	Test Specification RLP
8411.401.98.100; Condat GmbH

[39]	Test Specification L2R
8411.402.98.100; Condat GmbH

[40]	Test Specification FAD
8411.403.98.100; Condat GmbH

[41]	Test Specification T30
8411.404.98.100; Condat GmbH

[42]	Test Specification ACI
8411.405.98.100; Condat GmbH

[43]	SDL Specification RLP
8411.501.98.100; Condat GmbH

[44]	SDL Specification L2R
8411.502.98.100; Condat GmbH

[45]	SDL Specification FAD
8411.503.98.100; Condat GmbH

[46]	SDL Specification T30
8411.504.98.100; Condat GmbH

[47]	SDL Specification ACI
8411.505.98.100; Condat GmbH

[48]	Technical Documentation RLP
8411.701.98.100; Condat GmbH

[49]	Technical Documentation L2R
8411.702.98.100; Condat GmbH

[50]	Technical Documentation FAD
8411.703.98.100; Condat GmbH

[51]	Technical Documentation T30
8411.704.98.100; Condat GmbH

[52]	Technical Documentation ACI
8411.705.98.100; Condat GmbH

## Abbreviations

ACI	AT Command Interpreter
AGCH	Access Grant Channel
AT	Attention sequence "AT" to indicate valid commands of the ACI

BCCH	Broadcast Control Channel
BCS	Binary Coded Signals
BS	Base Station
BSIC	Base Station Identification Code

C/R	Command/Response
C1	Path Loss Criterion
C2	Reselection Criterion
CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
CRC	Cyclic Redundancy Check

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame
DTX	Discontinuous Transmission

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface
EOL	End Of Line

F	Final Bit
F&D	Fax and Data Protocol Stack
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HDLC	High level Data Link Control
HISR	High level Interrupt Service Routine
HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity
ITU	International Telecommunication Union
IWF	Interworking Function

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LISR	Low level Interrupt Service Routine
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station
MSG	Message phase in the GSM 3.45 protocol

N(R)	Receive Number
N(S)	Send Number
NCC	National Colour Code
NECI	New Establishment Causes included

OTD	Observed Time Difference

P	Poll Bit
P/F	Poll/Final Bit
PCH	Paging Channel
PCO	Point of Control and Observation
PDU	Protocol Description Unit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference
RTOS	Real Time Operating System

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

- T.4	CCITT Standardisation for Document coding of Group 3 Facsimile Apparatus
TAP	Test Application Program
TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TE	Terminal Equipment - e. g. a PC
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable
VPLMN	Visiting Public Land Mobile Network

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Overview

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The protocol stack for fax and data transmission consists of several entities. Each entity has one ore more service access points, over which the entity provides a service for the upper entity. The entity, which is described in this document, is coloured grey in the following figure :

- Figure 1-1: Architecture of the fax and data protocol stack

The information units passed via the SAPs are called primitives and consists of an operation code and several parameters. See the Users Guide for details.

The entities of the fax and data protocol stack are:

## RA - Rate Adaptation

This entity performs an adaptation between an asynchronous or synchronous data stream with several bit rates on to the fixed bit rate used at the TCH. This is performed by the rate adaptation functions RA1’ and RA0 described in GSM 04.21.

## RLP - Radio Link Protocol

This entity provides a Layer 2 protocol for asynchronous reliable data transfer as specified in GSM 04.22. It includes error correction, sequence numbers and a mechanism for repeating corrupted and lost messages.

## L2R - Layer 2 Relay Functionality

The L2R provides relay functions in order to adapt the character-oriented data received from the TE via USART to the bit-oriented RLP protocol.

## FAD 03.45 - Fax Adaptation Protocol

The fax adaptation protocol, as specified in GSM 03.45, provides synchronisation with the BCS and MSG modems of the peer entity. It uses byte repetition in conjunction with a voting algorithm to handle corruption on the TCH data stream. The non-transparent fax protocol in accordance with GSM 03.46 is not part of this implementation.

The fax adapter enables T.30 to send BCS at 300 BPS and T.4 MSG in 2400, 4800, 7200 and 9600 BPS.

## T.30 - Fax Protocol Entity

The protocol uses binary coded signals packed in HDLC frames to set up and release a connection in the message phase of the FAX transmission. This entity is specified in the ITU-T.30. The main tasks of this unit are:

- Building the HDLC frames with CRC.

- Performing bit stuffing/de-stuffing.

- Executing a sequence of 5 phases: 1.) set up, 2.) pre-message procedures, 3.) transmission/reception, 4.) post message procedures, 5.) waiting for call release.

## ACI - AT Command Interpreter

The ACI is specified in GSM 07.07. It is responsible for call establishment via the GSM voice protocol stack and terminal adaptation for asynchronous transparent character-oriented data transmission. The ACI is able to receive AT commands and send the replies over the USART driver to a remote PC. This makes it possible to control the voice and data protocol stack from a remote application running on a PC. The ACI also provides a unique interface for an internal MMI in the MS.

## USART - Universal Synchronous Asynchronous Receiver Transmitter Driver

The USART is a hardware component that facilitates a connection between the mobile station and terminal equipment (e.g. a PC). This interface uses some of the circuits described in V.24.

The data exchange provided by this unit is serial and asynchronous (synchronous communication is not in the scope of this document). A driver that uses interrupts to manage a circular buffer for the sending and receiving direction is necessary in order to use this component in the F&D. The driver has to be able to perform flow control.

# Parameters

/*---------------------------------------- array declarations ------------------------------------------*/

- DECLARATION (F_ICC)

- DECLARATION (F_CUR_PIN)

- DECLARATION (F_NEW_PIN)

- DECLARATION (F_PUK)

- DECLARATION (F_STK_PRF)

- DECLARATION ( A_CLG_NUM )

- DECLARATION ( A_CLD_NUM )

- DECLARATION ( A_CLD_NUM_B )

- DECLARATION ( A_CLD_NUM_NINT )

- DECLARATION ( A_CLD_NUM_MDFY )

- DECLARATION ( A_CLD_NUM_INT )

- DECLARATION ( A_CLD_NUM_ECC )

- DECLARATION ( A_CLD_SUB_MDFY )

- DECLARATION ( A_SS_VER_1 )

- DECLARATION ( A_SS_VER_1_CONTENT )

- DECLARATION ( A_SS_VER_2 )

- DECLARATION ( A_SS_VER_2_CONTENT )

- DECLARATION ( A_FAC_AOC )

- DECLARATION ( A_FAC_AOC_CONTENT )

- DECLARATION ( A_FAC_CLIP_IRGT )

- DECLARATION ( A_FAC_CLIP_IRGT_CONTENT )

- DECLARATION ( A_FAC_CLIR_IRGT )

- DECLARATION ( A_FAC_CLIR_IRGT_CONTENT )

- DECLARATION ( A_FAC_COLP_IRGT )

- DECLARATION ( A_FAC_COLP_IRGT_CONTENT )

- DECLARATION ( A_FAC_COLR_IRGT )

- DECLARATION ( A_FAC_COLR_IRGT_CONTENT )

- DECLARATION ( A_FAC_CCWA_IRGT )

- DECLARATION ( A_FAC_CCWA_IRGT_CONTENT )

- DECLARATION ( A_FAC_CCWA_ACT_V )

- DECLARATION ( A_FAC_CCWA_ACT_V_CONTENT )

- DECLARATION ( A_FAC_CCWA_ACT_V_RES )

- DECLARATION ( A_FAC_CCWA_ACT_V_RES_CONTENT )

- DECLARATION ( A_FAC_CCWA_ACT_V_RES2 )

- DECLARATION ( A_FAC_CCWA_ACT_V_RES2_CONTENT )

- DECLARATION ( A_FAC_CCWA_DEACT_VF )

- DECLARATION ( A_FAC_CCWA_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCWA_DEACT_VF_RES )

- DECLARATION ( A_FAC_CCWA_DEACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CCWA_DEACT_VF_ERR_RES )

- DECLARATION ( A_FAC_CCWA_DEACT_VF_ERR_RES_CONTENT )

- DECLARATION ( A_FAC_CCWA_DEACT_D )

- DECLARATION ( A_FAC_CCWA_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CCWA_DEACT_D_RES )

- DECLARATION ( A_FAC_CCWA_DEACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CCWA_ACT_D )

- DECLARATION ( A_FAC_CCWA_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CCWA_ACT_D_RES )

- DECLARATION ( A_FAC_CCWA_ACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CCWA_IRGT_RES )

- DECLARATION ( A_FAC_CCWA_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_CCWA_IRGT_RES2 )

- DECLARATION ( A_FAC_CCWA_IRGT_RES_CONTENT2 )

- DECLARATION ( A_FAC_CCFC_CFU_IRGT )

- DECLARATION ( A_FAC_CCFC_CFU_IRGT_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFB_IRGT )

- DECLARATION ( A_FAC_CCFC_CFB_IRGT_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRY_IRGT )

- DECLARATION ( A_FAC_CCFC_CFNRY_IRGT_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRC_IRGT )

- DECLARATION ( A_FAC_CCFC_CFNRC_IRGT_CONTENT )

- DECLARATION ( A_FAC_CLIP_IRGT_RES_PROV )

- DECLARATION ( A_FAC_CLIP_IRGT_RES_PROV_CONTENT )

- DECLARATION ( A_FAC_CLIP_IRGT_RES_NOTPROV )

- DECLARATION ( A_FAC_CLIP_IRGT_RES_NOTPROV_CONTENT )

- DECLARATION ( A_FAC_CLIP_IRGT_RES_UNKNOWN )

- DECLARATION ( A_FAC_CLIP_IRGT_RES_UNKNOWN_CONTENT )

- DECLARATION ( A_FAC_CLIR_IRGT_RES_PVTMAL )

- DECLARATION ( A_FAC_CLIR_IRGT_RES_PVTMAL_CONTENT )

- DECLARATION ( A_FAC_CLIR_IRGT_RES_PVTMAL2 )

- DECLARATION ( A_FAC_CLIR_IRGT_RES_PVTMAL_CONTENT2 )

- DECLARATION ( A_FAC_CCFC_CFU_REG_VF )

- DECLARATION ( A_FAC_CCFC_CFU_REG_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFU_REG_D )

- DECLARATION ( A_FAC_CCFC_CFU_REG_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFB_REG_VF )

- DECLARATION ( A_FAC_CCFC_CFB_REG_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFB_REG_D )

- DECLARATION ( A_FAC_CCFC_CFB_REG_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRY_REG_VF )

- DECLARATION ( A_FAC_CCFC_CFNRY_REG_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRY_REG_D )

- DECLARATION ( A_FAC_CCFC_CFNRY_REG_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRC_REG_VF )

- DECLARATION ( A_FAC_CCFC_CFNRC_REG_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRC_REG_D )

- DECLARATION ( A_FAC_CCFC_CFNRC_REG_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCF_REG_VF )

- DECLARATION ( A_FAC_CCFC_ALLCF_REG_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCF_REG_D )

- DECLARATION ( A_FAC_CCFC_ALLCF_REG_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCFC_REG_VF )

- DECLARATION ( A_FAC_CCFC_ALLCFC_REG_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCFC_REG_D )

- DECLARATION ( A_FAC_CCFC_ALLCFC_REG_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFU_ERS_VF )

- DECLARATION ( A_FAC_CCFC_CFU_ERS_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFU_ERS_D )

- DECLARATION ( A_FAC_CCFC_CFU_ERS_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFB_ERS_VF )

- DECLARATION ( A_FAC_CCFC_CFB_ERS_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFB_ERS_D )

- DECLARATION ( A_FAC_CCFC_CFB_ERS_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRY_ERS_VF )

- DECLARATION ( A_FAC_CCFC_CFNRY_ERS_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRY_ERS_D )

- DECLARATION ( A_FAC_CCFC_CFNRY_ERS_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRC_ERS_VF )

- DECLARATION ( A_FAC_CCFC_CFNRC_ERS_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRC_ERS_D )

- DECLARATION ( A_FAC_CCFC_CFNRC_ERS_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCF_ERS_VF )

- DECLARATION ( A_FAC_CCFC_ALLCF_ERS_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCF_ERS_D )

- DECLARATION ( A_FAC_CCFC_ALLCF_ERS_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCFC_ERS_VF )

- DECLARATION ( A_FAC_CCFC_ALLCFC_ERS_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCFC_ERS_D )

- DECLARATION ( A_FAC_CCFC_ALLCFC_ERS_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFU_ACT_VF )

- DECLARATION ( A_FAC_CCFC_CFU_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFU_ACT_D )

- DECLARATION ( A_FAC_CCFC_CFU_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFB_ACT_VF )

- DECLARATION ( A_FAC_CCFC_CFB_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFB_ACT_D )

- DECLARATION ( A_FAC_CCFC_CFB_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRY_ACT_VF )

- DECLARATION ( A_FAC_CCFC_CFNRY_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRY_ACT_D )

- DECLARATION ( A_FAC_CCFC_CFNRY_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCF_ACT_VF )

- DECLARATION ( A_FAC_CCFC_ALLCF_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCF_ACT_D )

- DECLARATION ( A_FAC_CCFC_ALLCF_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCFC_ACT_VF )

- DECLARATION ( A_FAC_CCFC_ALLCFC_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCFC_ACT_D )

- DECLARATION ( A_FAC_CCFC_ALLCFC_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFU_DEACT_VF )

- DECLARATION ( A_FAC_CCFC_CFU_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFU_DEACT_D )

- DECLARATION ( A_FAC_CCFC_CFU_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFB_DEACT_VF )

- DECLARATION ( A_FAC_CCFC_CFB_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFB_DEACT_D )

- DECLARATION ( A_FAC_CCFC_CFB_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRY_DEACT_VF )

- DECLARATION ( A_FAC_CCFC_CFNRY_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRY_DEACT_D )

- DECLARATION ( A_FAC_CCFC_CFNRY_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRC_DEACT_VF )

- DECLARATION ( A_FAC_CCFC_CFNRC_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRC_ACT_D )

- DECLARATION ( A_FAC_CCFC_CFNRC_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRC_DEACT_D )

- DECLARATION ( A_FAC_CCFC_CFNRC_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCF_DEACT_VF )

- DECLARATION ( A_FAC_CCFC_ALLCF_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCF_DEACT_D )

- DECLARATION ( A_FAC_CCFC_ALLCF_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCFC_DEACT_VF )

- DECLARATION ( A_FAC_CCFC_ALLCFC_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CCFC_ALLCFC_DEACT_D )

- DECLARATION ( A_FAC_CCFC_ALLCFC_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFU_IRGT_RES )

- DECLARATION ( A_FAC_CCFC_CFU_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFB_IRGT_RES )

- DECLARATION ( A_FAC_CCFC_CFB_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRY_IRGT_RES )

- DECLARATION ( A_FAC_CCFC_CFNRY_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRC_IRGT_RES )

- DECLARATION ( A_FAC_CCFC_CFNRC_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_CCFC_CFNRC_ACT_VF )

- DECLARATION ( A_FAC_CCFC_CFNRC_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_NTFY_SS_1 )

- DECLARATION ( A_FAC_NTFY_SS_1_CONTENT )

- DECLARATION ( A_FAC_NTFY_SS_ECT_RSTR )

- DECLARATION ( A_FAC_NTFY_SS_ECT_RSTR_CONTENT )

- DECLARATION ( A_FAC_NTFY_SS_FD)

- DECLARATION ( A_FAC_NTFY_SS_FD_CONTENT )

- DECLARATION ( A_FAC_CHECK_SS )

- DECLARATION ( A_FAC_CHECK_SS_CONTENT )

- DECLARATION ( A_FAC_CUG_SS_1 )

- DECLARATION ( A_FAC_CUG_SS_1_CONTENT )

- DECLARATION ( A_FAC_CUG_SS_2 )

- DECLARATION ( A_FAC_CUG_SS_2_CONTENT )

- DECLARATION ( A_FAC_CPWD_ALLCB_REG )

- DECLARATION ( A_FAC_CPWD_ALLCB_REG_CONTENT )

- DECLARATION ( A_FAC_ENTER_PWD_REQ )

- DECLARATION ( A_FAC_ENTER_PWD_REQ_CONTENT )

- DECLARATION ( A_FAC_ENTER_PWD_RES )

- DECLARATION ( A_FAC_ENTER_PWD_RES_CONTENT )

- DECLARATION ( A_FAC_ENTER_PWD_REQ_2 )

- DECLARATION ( A_FAC_ENTER_PWD_REQ_2_CONTENT )

- DECLARATION ( A_FAC_ENTER_PWD_RES_2 )

- DECLARATION ( A_FAC_ENTER_PWD_RES_2_CONTENT )

- DECLARATION ( A_FAC_NEW_PWD_REQ )

- DECLARATION ( A_FAC_NEW_PWD_REQ_CONTENT )

- DECLARATION ( A_FAC_NEW_PWD_RES )

- DECLARATION ( A_FAC_NEW_PWD_RES_CONTENT )

- DECLARATION ( A_FAC_NEWAGN_PWD_REQ )

- DECLARATION ( A_FAC_NEWAGN_PWD_REQ_CONTENT )

- DECLARATION ( A_FAC_NEWAGN_PWD_RES )

- DECLARATION ( A_FAC_NEWAGN_PWD_RES_CONTENT )

- DECLARATION ( A_FAC_CPWD_ALLCB_RES )

- DECLARATION ( A_FAC_CPWD_ALLCB_RES_CONTENT )

- DECLARATION ( A_FAC_CPWD_ALLCB_ERR_1 )

- DECLARATION ( A_FAC_CPWD_ALLCB_ERR_1_CONTENT )

- DECLARATION ( A_FAC_CPWD_ALLCB_ERR_2 )

- DECLARATION ( A_FAC_CPWD_ALLCB_ERR_2_CONTENT )

- DECLARATION ( A_FAC_CPWD_ALLCB_ERR_3 )

- DECLARATION ( A_FAC_CPWD_ALLCB_ERR_3_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLCB_DEACT_VF )

- DECLARATION ( A_FAC_CLCK_ALLCB_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLCB_DEACT_D )

- DECLARATION ( A_FAC_CLCK_ALLCB_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLIN_DEACT_VF )

- DECLARATION ( A_FAC_CLCK_ALLIN_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLIN_DEACT_D )

- DECLARATION ( A_FAC_CLCK_ALLIN_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLOUT_DEACT_VF )

- DECLARATION ( A_FAC_CLCK_ALLOUT_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLOUT_DEACT_D )

- DECLARATION ( A_FAC_CLCK_ALLOUT_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_ACT_D_RES )

- DECLARATION ( A_FAC_CLCK_BAIC_ACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_DEACT_VF )

- DECLARATION ( A_FAC_CLCK_BAOC_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_ALL_DEACT_VF )

- DECLARATION ( A_FAC_CLCK_BAOC_ALL_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_DEACT_D )

- DECLARATION ( A_FAC_CLCK_BAOC_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_DEACT_VF )

- DECLARATION ( A_FAC_CLCK_BOIC_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_ALL_DEACT_VF )

- DECLARATION ( A_FAC_CLCK_BOIC_ALL_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_DEACT_D )

- DECLARATION ( A_FAC_CLCK_BOIC_DEACT_D_CONTENT )

DECLARATION ( A_FAC_CLCK_BOICxHC_DEACT_VF )

DECLARATION ( A_FAC_CLCK_BOICxHC_DEACT_VF_CONTENT )

DECLARATION ( A_FAC_CLCK_BOICxHC_ALL_DEACT_VF )

DECLARATION ( A_FAC_CLCK_BOICxHC_ALL_DEACT_VF_CONTENT )

DECLARATION ( A_FAC_CLCK_BOICxHC_DEACT_D )

DECLARATION ( A_FAC_CLCK_BOICxHC_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_DEACT_VF )

- DECLARATION ( A_FAC_CLCK_BAIC_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_ALL_DEACT_VF )

- DECLARATION ( A_FAC_CLCK_BAIC_ALL_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_DEACT_D )

- DECLARATION ( A_FAC_CLCK_BAIC_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_DEACT_VF )

- DECLARATION ( A_FAC_CLCK_BICR_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_ALL_DEACT_VF )

- DECLARATION ( A_FAC_CLCK_BICR_ALL_DEACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_DEACT_D )

- DECLARATION ( A_FAC_CLCK_BICR_DEACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLCB_ACT_VF )

- DECLARATION ( A_FAC_CLCK_ALLCB_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLCB_ACT_D )

- DECLARATION ( A_FAC_CLCK_ALLCB_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLIN_ACT_VF )

- DECLARATION ( A_FAC_CLCK_ALLIN_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLIN_ACT_D )

- DECLARATION ( A_FAC_CLCK_ALLIN_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLOUT_ACT_VF )

- DECLARATION ( A_FAC_CLCK_ALLOUT_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLOUT_ACT_D )

- DECLARATION ( A_FAC_CLCK_ALLOUT_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT_VF )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_ALL_ACT_VF )

- DECLARATION ( A_FAC_CLCK_BAOC_ALL_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT_D )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_ACT )

- DECLARATION ( A_FAC_CLCK_BOIC_ACT_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_ACT_VF )

- DECLARATION ( A_FAC_CLCK_BOIC_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_ALL_ACT_VF )

- DECLARATION ( A_FAC_CLCK_BOIC_ALL_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_ACT_D )

- DECLARATION ( A_FAC_CLCK_BOIC_ACT_D_CONTENT )

DECLARATION ( A_FAC_CLCK_BOICxHC_ACT )

DECLARATION ( A_FAC_CLCK_BOICxHC_ACT_CONTENT )

DECLARATION ( A_FAC_CLCK_BOICxHC_ACT_VF )

DECLARATION ( A_FAC_CLCK_BOICxHC_ACT_VF_CONTENT )

DECLARATION ( A_FAC_CLCK_BOICxHC_ALL_ACT_VF )

DECLARATION ( A_FAC_CLCK_BOICxHC_ALL_ACT_VF_CONTENT )

DECLARATION ( A_FAC_CLCK_BOICxHC_ACT_D )

DECLARATION ( A_FAC_CLCK_BOICxHC_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_ACT )

- DECLARATION ( A_FAC_CLCK_BAIC_ACT_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_ACT_VF )

- DECLARATION ( A_FAC_CLCK_BAIC_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_ALL_ACT_VF )

- DECLARATION ( A_FAC_CLCK_BAIC_ALL_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_ACT_D )

- DECLARATION ( A_FAC_CLCK_BAIC_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_ACT )

- DECLARATION ( A_FAC_CLCK_BICR_ACT_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_ACT_VF )

- DECLARATION ( A_FAC_CLCK_BICR_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_ALL_ACT_VF )

- DECLARATION ( A_FAC_CLCK_BICR_ALL_ACT_VF_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_ACT_D )

- DECLARATION ( A_FAC_CLCK_BICR_ACT_D_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_IRGT )

- DECLARATION ( A_FAC_CLCK_BAOC_IRGT_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_IRGT )

- DECLARATION ( A_FAC_CLCK_BOIC_IRGT_CONTENT )

DECLARATION ( A_FAC_CLCK_BOICxHC_IRGT )

DECLARATION ( A_FAC_CLCK_BOICxHC_IRGT_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_IRGT )

- DECLARATION ( A_FAC_CLCK_BAIC_IRGT_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_IRGT )

- DECLARATION ( A_FAC_CLCK_BICR_IRGT_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLCB_ACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_ALLCB_ACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLCB_ACT_D_RES )

- DECLARATION ( A_FAC_CLCK_ALLCB_ACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLIN_ACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_ALLIN_ACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLIN_ACT_D_RES )

- DECLARATION ( A_FAC_CLCK_ALLIN_ACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLOUT_ACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_ALLOUT_ACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLOUT_ACT_D_RES )

- DECLARATION ( A_FAC_CLCK_ALLOUT_ACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT_RES )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_ALL_ACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BAOC_ALL_ACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT_D_RES )

- DECLARATION ( A_FAC_CLCK_BAOC_ACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_ACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BOIC_ACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_ALL_ACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BOIC_ALL_ACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_ACT_D_RES )

- DECLARATION ( A_FAC_CLCK_BOIC_ACT_D_RES_CONTENT )

DECLARATION ( A_FAC_CLCK_BOICxHC_ACT_VF_RES )

DECLARATION ( A_FAC_CLCK_BOICxHC_ACT_VF_RES_CONTENT )

DECLARATION ( A_FAC_CLCK_BOICxHC_ALL_ACT_VF_RES )

DECLARATION ( A_FAC_CLCK_BOICxHC_ALL_ACT_VF_RES_CONTENT )

DECLARATION ( A_FAC_CLCK_BOICxHC_ACT_D_RES )

DECLARATION ( A_FAC_CLCK_BOICxHC_ACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_ACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BAIC_ACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_ALL_ACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BAIC_ALL_ACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_ACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BICR_ACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_ALL_ACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BICR_ALL_ACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_ACT_D_RES )

- DECLARATION ( A_FAC_CLCK_BICR_ACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLCB_DEACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_ALLCB_DEACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLCB_DEACT_D_RES )

- DECLARATION ( A_FAC_CLCK_ALLCB_DEACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLOUT_DEACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_ALLOUT_DEACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLOUT_DEACT_D_RES )

- DECLARATION ( A_FAC_CLCK_ALLOUT_DEACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLIN_DEACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_ALLIN_DEACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_ALLIN_DEACT_D_RES )

- DECLARATION ( A_FAC_CLCK_ALLIN_DEACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_DEACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BAOC_DEACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_ALL_DEACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BAOC_ALL_DEACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAOC_DEACT_D_RES )

- DECLARATION ( A_FAC_CLCK_BAOC_DEACT_D_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_DEACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BOIC_DEACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_ALL_DEACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BOIC_ALL_DEACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BOIC_DEACT_D_RES )

- DECLARATION ( A_FAC_CLCK_BOIC_DEACT_D_RES_CONTENT )

DECLARATION (A_FAC_CLCK_BOICxHC_DACT_VF_RES)

DECLARATION (A_FAC_CLCK_BOICxHC_DACT_VF_RES_CONTENT)

DECLARATION (A_FAC_CLCK_BOICxHC_ALL_DACT_VF_RES)

DECLARATION (A_FAC_CLCK_BOICxHC_ALL_DACT_VF_RES_CONTENT)

DECLARATION (A_FAC_CLCK_BOICxHC_DACT_D_RES)

DECLARATION (A_FAC_CLCK_BOICxHC_DACT_D_RES_CONTENT)

- DECLARATION ( A_FAC_CLCK_BAIC_DEACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BAIC_DEACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BAIC_ALL_DEACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BAIC_ALL_DEACT_VF_RES_CONTENT )

- DECLARATION (A_FAC_CLCK_BAIC_DEACT_D_RES)

- DECLARATION (A_FAC_CLCK_BAIC_DEACT_D_RES_CONTENT)

- DECLARATION ( A_FAC_CLCK_BICR_DEACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BICR_DEACT_VF_RES_CONTENT )

- DECLARATION ( A_FAC_CLCK_BICR_ALL_DEACT_VF_RES )

- DECLARATION ( A_FAC_CLCK_BICR_ALL_DEACT_VF_RES_CONTENT )

- DECLARATION (A_FAC_CLCK_BICR_DEACT_D_RES)

- DECLARATION (A_FAC_CLCK_BICR_DEACT_D_RES_CONTENT)

- DECLARATION ( A_FAC_CCBS_ERS_1 )

- DECLARATION ( A_FAC_CCBS_ERS_1_CONTENT )

- DECLARATION ( A_FAC_CCBS_ERS_ALL )

- DECLARATION ( A_FAC_CCBS_ERS_ALL_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRY_REG_VB )

- DECLARATION ( A_FAC_KSD_CFNRY_REG_VB_CONTENT )

- DECLARATION ( A_FAC_CCBS_ITRG )

- DECLARATION ( A_FAC_CCBS_ITRG_CONTENT )

- DECLARATION ( A_FAC_CCBS_ERS_1_RES )

- DECLARATION ( A_FAC_CCBS_ERS_1_RES_CONTENT )

- DECLARATION ( A_FAC_CCBS_ERS_ALL_RES )

- DECLARATION ( A_FAC_CCBS_ERS_ALL_RES_CONTENT )

- DECLARATION ( A_FAC_CCBS_ITRG_3N_RES )

- DECLARATION ( A_FAC_CCBS_ITRG_3N_RES_CONTENT )

- DECLARATION ( A_FAC_CCBS_ITRG_P_RES )

- DECLARATION ( A_FAC_CCBS_ITRG_P_RES_CONTENT )

- DECLARATION ( A_FAC_CCBS_ITRG_NP_RES )

- DECLARATION ( A_FAC_CCBS_ITRG_NP_RES_CONTENT )

- DECLARATION ( A_FAC_USSD_REQ )

- DECLARATION ( A_FAC_USSD_REQ_CONTENT )

- DECLARATION ( A_FAC_USSD_NTFY )

- DECLARATION ( A_FAC_USSD_NTFY_CONTENT )

- DECLARATION ( A_FAC_USSD_PROC )

- DECLARATION ( A_FAC_USSD_PROC_CONTENT )

- DECLARATION ( A_FAC_USSD_PROC_KSD )

- DECLARATION ( A_FAC_USSD_PROC_KSD_CONTENT )

- DECLARATION ( A_FAC_USSD_REQ_RES )

- DECLARATION ( A_FAC_USSD_REQ_RES_CONTENT )

- DECLARATION ( A_FAC_USSD_NTFY_RES )

- DECLARATION ( A_FAC_USSD_NTFY_RES_CONTENT )

- DECLARATION ( A_FAC_USSD_PROC_RES )

- DECLARATION ( A_FAC_USSD_PROC_RES_CONTENT )

- DECLARATION ( A_FAC_USSD_DAT_RES )

- DECLARATION ( A_FAC_USSD_DAT_RES_CONTENT )

- DECLARATION ( A_FAC_USSD_PROC_REJ )

- DECLARATION ( A_FAC_USSD_PROC_REJ_CONTENT )

- DECLARATION ( A_FAC_USSD_PROC_IA5 )

- DECLARATION ( A_FAC_USSD_PROC_IA5_CONTENT )

- DECLARATION ( A_FAC_USSD_PROC_KSD_IA5 )

- DECLARATION ( A_FAC_USSD_PROC_KSD_IA5_CONTENT )

- DECLARATION ( A_FAC_USSD_BUSY_ERR )

- DECLARATION ( A_FAC_USSD_BUSY_ERR_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRY_REG_V )

- DECLARATION ( A_FAC_KSD_CFNRY_REG_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFU_REG_V )

- DECLARATION ( A_FAC_KSD_CFU_REG_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFB_REG_V )

- DECLARATION ( A_FAC_KSD_CFB_REG_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CF_REG_V )

- DECLARATION ( A_FAC_KSD_CF_REG_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFC_ERS_V )

- DECLARATION ( A_FAC_KSD_CFC_ERS_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRC_ERS_V )

- DECLARATION ( A_FAC_KSD_CFNRC_ERS_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFU_ERS_V )

- DECLARATION ( A_FAC_KSD_CFU_ERS_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRY_ERS_V )

- DECLARATION ( A_FAC_KSD_CFNRY_ERS_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CF_ACT_V )

- DECLARATION ( A_FAC_KSD_CF_ACT_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFU_ACT_V )

- DECLARATION ( A_FAC_KSD_CFU_ACT_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFC_DEACT_V )

- DECLARATION ( A_FAC_KSD_CFC_DEACT_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRC_DEACT_V )

- DECLARATION ( A_FAC_KSD_CFNRC_DEACT_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFB_IRGT_V )

- DECLARATION ( A_FAC_KSD_CFB_IRGT_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRY_IRGT_V )

- DECLARATION ( A_FAC_KSD_CFNRY_IRGT_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRC_IRGT_V )

- DECLARATION ( A_FAC_KSD_CFNRC_IRGT_V_CONTENT )

- DECLARATION ( A_FAC_KSD_CFB_IRGT_F )

- DECLARATION ( A_FAC_KSD_CFB_IRGT_F_CONTENT )

- DECLARATION ( A_FAC_KSD_ALLCB_PWD )

- DECLARATION ( A_FAC_KSD_ALLCB_PWD_CONTENT )

- DECLARATION ( A_FAC_KSD_BAOC_ACT )

- DECLARATION ( A_FAC_KSD_BAOC_ACT_CONTENT )

- DECLARATION ( A_FAC_KSD_BICR_ACT )

- DECLARATION ( A_FAC_KSD_BICR_ACT_CONTENT )

- DECLARATION ( A_FAC_KSD_BOIC_ACT )

- DECLARATION ( A_FAC_KSD_BOIC_ACT_CONTENT )

- DECLARATION ( A_FAC_KSD_BAIC_ACT )

- DECLARATION ( A_FAC_KSD_BAIC_ACT_CONTENT )

- DECLARATION ( A_FAC_KSD_ALLCB_DEACT )

- DECLARATION ( A_FAC_KSD_ALLCB_DEACT_CONTENT )

- DECLARATION ( A_FAC_KSD_ALLOUT_DEACT )

- DECLARATION ( A_FAC_KSD_ALLOUT_DEACT_CONTENT )

- DECLARATION ( A_FAC_KSD_ALLIN_DEACT )

- DECLARATION ( A_FAC_KSD_ALLIN_DEACT_CONTENT )

DECLARATION ( A_FAC_KSD_BOICxHC_DEACT )

DECLARATION ( A_FAC_KSD_BOICxHC_DEACT_CONTENT )

- DECLARATION ( A_FAC_KSD_BAOC_DEACT )

- DECLARATION ( A_FAC_KSD_BAOC_DEACT_CONTENT )

- DECLARATION ( A_FAC_KSD_BAIC_IRGT )

- DECLARATION ( A_FAC_KSD_BAIC_IRGT_CONTENT )

DECLARATION ( A_FAC_KSD_BOICxHC_IRGT )

DECLARATION ( A_FAC_KSD_BOICxHC_IRGT_CONTENT )

- DECLARATION ( A_FAC_KSD_BICR_IRGT )

- DECLARATION ( A_FAC_KSD_BICR_IRGT_CONTENT )

- DECLARATION ( A_FAC_KSD_BOIC_IRGT )

- DECLARATION ( A_FAC_KSD_BOIC_IRGT_CONTENT )

- DECLARATION ( A_FAC_KSD_CLIR_IRGT )

- DECLARATION ( A_FAC_KSD_CLIR_IRGT_CONTENT )

- DECLARATION ( A_FAC_KSD_CW_ACT )

- DECLARATION ( A_FAC_KSD_CW_ACT_CONTENT )

- DECLARATION ( A_FAC_KSD_USSD_PROC )

- DECLARATION ( A_FAC_KSD_USSD_PROC_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRY_REG_RES_A )

- DECLARATION ( A_FAC_KSD_CFNRY_REG_RES_A_CONTENT )

- DECLARATION ( A_FAC_KSD_CCBS_ERS_1 )

- DECLARATION ( A_FAC_KSD_CCBS_ERS_1_CONTENT )

- DECLARATION ( A_FAC_KSD_CCBS_ERS_ALL )

- DECLARATION ( A_FAC_KSD_CCBS_ERS_ALL_CONTENT )

- DECLARATION ( A_FAC_KSD_CCBS_IRGT )

- DECLARATION ( A_FAC_KSD_CCBS_IRGT_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRY_REG_RES_B )

- DECLARATION ( A_FAC_KSD_CFNRY_REG_RES_B_CONTENT )

- DECLARATION ( A_FAC_KSD_CFU_REG_RES )

- DECLARATION ( A_FAC_KSD_CFU_REG_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFB_REG_RES )

- DECLARATION ( A_FAC_KSD_CFB_REG_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CF_REG_RES )

- DECLARATION ( A_FAC_KSD_CF_REG_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFC_ERS_RES )

- DECLARATION ( A_FAC_KSD_CFC_ERS_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRC_ERS_RES )

- DECLARATION ( A_FAC_KSD_CFNRC_ERS_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFU_ERS_RES )

- DECLARATION ( A_FAC_KSD_CFU_ERS_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRY_ERS_RES )

- DECLARATION ( A_FAC_KSD_CFNRY_ERS_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CF_ACT_RES )

- DECLARATION ( A_FAC_KSD_CF_ACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFU_ACT_RES )

- DECLARATION ( A_FAC_KSD_CFU_ACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFC_DEACT_RES )

- DECLARATION ( A_FAC_KSD_CFC_DEACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRC_DEACT_RES )

- DECLARATION ( A_FAC_KSD_CFNRC_DEACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_BAOC_DEACT_RES )

- DECLARATION ( A_FAC_KSD_BAOC_DEACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFB_IRGT_RES )

- DECLARATION ( A_FAC_KSD_CFB_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFB_IRGT_RES2 )

- DECLARATION ( A_FAC_KSD_CFB_IRGT_RES2_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRY_IRGT_RES )

- DECLARATION ( A_FAC_KSD_CFNRY_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFNRC_IRGT_RES )

- DECLARATION ( A_FAC_KSD_CFNRC_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CFB_IRGT_F_RES )

- DECLARATION ( A_FAC_KSD_CFB_IRGT_F_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_BAOC_ACT_RES )

- DECLARATION ( A_FAC_KSD_BAOC_ACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_BICR_ACT_RES )

- DECLARATION ( A_FAC_KSD_BICR_ACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_BOIC_ACT_RES )

- DECLARATION ( A_FAC_KSD_BOIC_ACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_BAIC_ACT_RES )

- DECLARATION ( A_FAC_KSD_BAIC_ACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_ALLCB_DEACT_RES )

- DECLARATION ( A_FAC_KSD_ALLCB_DEACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_ALLOUT_DEACT_RES )

- DECLARATION ( A_FAC_KSD_ALLOUT_DEACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_ALLIN_DEACT_RES )

- DECLARATION ( A_FAC_KSD_ALLIN_DEACT_RES_CONTENT )

DECLARATION ( A_FAC_KSD_BOICxHC_DEACT_RES )

DECLARATION ( A_FAC_KSD_BOICxHC_DEACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_BAIC_IRGT_RES_A )

- DECLARATION ( A_FAC_KSD_BAIC_IRGT_RES_A_CONTENT )

- DECLARATION ( A_FAC_KSD_BAIC_IRGT_RES_B )

- DECLARATION ( A_FAC_KSD_BAIC_IRGT_RES_B_CONTENT )

- DECLARATION ( A_FAC_KSD_BAIC_IRGT_RES_C )

- DECLARATION ( A_FAC_KSD_BAIC_IRGT_RES_C_CONTENT )

DECLARATION ( A_FAC_KSD_BOICxHC_IRGT_RES )

DECLARATION ( A_FAC_KSD_BOICxHC_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_BICR_IRGT_RES )

- DECLARATION ( A_FAC_KSD_BICR_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_BOIC_IRGT_RES )

- DECLARATION ( A_FAC_KSD_BOIC_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CLIR_IRGT_RES )

- DECLARATION ( A_FAC_KSD_CLIR_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CW_ACT_RES )

- DECLARATION ( A_FAC_KSD_CW_ACT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CW_ACT_RES2 )

- DECLARATION ( A_FAC_KSD_CW_ACT_RES2_CONTENT )

- DECLARATION ( A_FAC_KSD_USSD_PROC_RES )

- DECLARATION ( A_FAC_KSD_USSD_PROC_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CCBS_ERS_RES )

- DECLARATION ( A_FAC_KSD_CCBS_ERS_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CCBS_IRGT_RES )

- DECLARATION ( A_FAC_KSD_CCBS_IRGT_RES_CONTENT )

- DECLARATION ( A_FAC_KSD_CCBS_IRGT_ERR )

- DECLARATION ( A_FAC_KSD_CCBS_IRGT_ERR_CONTENT )

- DECLARATION ( A_FAC_KSD_EMPTY_RES_INV_ID0 )

- DECLARATION ( A_FAC_KSD_EMPTY_RES_INV_ID0_CONTENT )

- DECLARATION (NUM_654321)

DECLARATION (S_CLG_PARTY)
DECLARATION (S_CLD_PARTY_2)

- DECLARATION (S_CLG_PARTY_SUB)
DECLARATION (S_CLG_EMERG_SUB)

/*----------------------------------------  structure declarations ------------------------------------ */

DECLARATION ( S_FAC_AOC )
DECLARATION ( S_FAC_CLIP_IRGT )

DECLARATION (S_CLD_PARTY)
DECLARATION (S_CLD_PARTY_SUB)
DECLARATION (S_BS_NOT_PRESENT)
DECLARATION (S_BS_V)
DECLARATION (S_BS_VOICE)
DECLARATION (S_CHN_FULL_9600)
DECLARATION (S_CLD_PARTY_ECC)
DECLARATION (S_CLD_PARTY_MDFY)
DECLARATION (S_CLD_PARTY_INT)
DECLARATION (S_CLD_PARTY_SUB_MDFY)
DECLARATION (S_CLD_PARTY_NINT)

/*----------------------------------------  definitions ------------------------------------------------ */

- BYTE NUM_0 0

- BYTE NUM_1 1

BYTE NUM_2 2

BYTE NUM_3 3

BYTE NUM_4 4

BYTE NUM_5 5

BYTE NUM_6 6

BYTE NUM_7 7

BYTE NUM_8 8

BYTE NUM_9 9

BYTE NUM_10 10

- SHORT NUM_9600 9600

- SHORT NUM_4800 4800

BYTE BCD_LEN_6	0x06

BYTE PRES_ALLOWED 0

BYTE SCR_USER_UNSCREENED 0

BYTE V_PLMN_PRES 1

/*---------------------------------------- AT commands and messages ----------------------------- */

/* AT final results */

/*

- Message: 	OK

successful operation

*/

- STRING(M_OK, "OK" )

- BYTE LM_OK 2

/*message:	RING

alerting*/

STRING(M_RING, "RING" )

/*

- Message: 	IMEI

*/

- STRING(M_IMEI, "13579024681122278" )

- BYTE LM_IMEI 17

/*

- Message: 	NO CARRIER

carrier lost

*/

- STRING(M_NO_CARRIER, "NO CARRIER" )

- BYTE LM_NO_CARRIER 10

/*

- Message: 	CONNECT

successful data call connection

*/

- STRING(M_CONNECT_9600, "CONNECT 9600" )

- BYTE LM_CONNECT_9600 12

/*

- Message: 	ERROR

error result code

*/

- STRING(M_ERROR, "ERROR" )

- BYTE LM_ERROR 5

/*

- Message: 	+CME

error result code

*/

STRING(M_ERR_PIN_REQ, "+CME ERROR: SIM PIN required" )

- BYTE LM_ERR_PIN_REQ 28

STRING(M_ERR_SIM_FATAL, "+CME ERROR: SIM not inserted" )

- BYTE LM_ERR_SIM_FATAL 28

STRING(M_ERR_SIM_BLCK, "+CME ERROR: SIM PUK required" )

- BYTE LM_ERR_SIM_BLCK 28

STRING(M_ERR_SIM_FAIL, "+CME ERROR: SIM failure" )

- BYTE LM_ERR_SIM_FAIL 23

STRING(M_ERR_NO_NTW_SRV, "+CME ERROR: no network service" )

- BYTE LM_ERR_NO_NTW_SRV 30

STRING(M_ERR_UKN, "+CME ERROR: unknown" )

- BYTE LM_ERR_UKN 19

/* AT commands */

/*

- Command: 	+CEER

extended error report

*/

- STRING(C_PLUS_CEER, "AT+CEER " )

- BYTE LC_PLUS_CEER 7

/*

- Command: 	+CMEE

error report mode

*/

- STRING(C_PLUS_CMEE_VERB, "AT+CMEE=2 " )

- BYTE LC_PLUS_CMEE_VERB 9

/*

- Command:	A

accept call

*/

- STRING(C_A, "ATA" )

- BYTE LC_A 3

/*

- Command:	CHLD

Call hold

*/

- STRING(C_CHLD_2, "AT+CHLD=2" )

- BYTE LC_CHLD_2 9

/*

- Command: 	+CCWA

Call waiting report mode

*/

- STRING(C_CCWA_ENABLE, "AT+CCWA=1" )

- BYTE LC_CCWA_ENABLE 9

/*

Message: 	+CCWA:

Indication of call waiting

*/

- STRING(M_CCWA_IND, "+CCWA: \"654321\",168,1" )

- BYTE LM_CCWA_IND 21

/*

- Command:	D

Dial a number

*/

- STRING(C_D_DAT, "ATD03039094444" )

- BYTE LC_D_DAT 14

STRING(C_D_VOICE, "ATD03039094444;" )

- BYTE LC_D_VOICE 15

STRING(C_D_VOICE_CUG, "ATD03039094444G;" )

- BYTE LC_D_VOICE_CUG 16

STRING(C_D_KSD_1, "ATD**61*00431234*11*5#" )

- BYTE LC_D_KSD_1 22

STRING(C_D_KSD_1B, "ATD**61*+431234*11*5#" )

- BYTE LC_D_KSD_1B 22

STRING(C_D_KSD_2, "ATD**21*00431234*13#" )

- BYTE LC_D_KSD_2 20

STRING(C_D_KSD_3, "ATD**67*00431234*21#" )

- BYTE LC_D_KSD_3 20

STRING(C_D_KSD_4, "ATD**002*00431234*13#" )

- BYTE LC_D_KSD_4 21

STRING(C_D_KSD_5, "ATD##004**13#" )

- BYTE LC_D_KSD_5 13

STRING(C_D_KSD_6, "ATD##62#" )

- BYTE LC_D_KSD_6 8

STRING(C_D_KSD_7, "ATD##21**11#" )

- BYTE LC_D_KSD_7 12

STRING(C_D_KSD_8, "ATD##61**13#" )

- BYTE LC_D_KSD_8 12

STRING(C_D_KSD_9, "ATD*002**22#" )

- BYTE LC_D_KSD_9 12

STRING(C_D_KSD_10, "ATD*21#" )

- BYTE LC_D_KSD_10 7

STRING(C_D_KSD_11, "ATD#004**11#" )

- BYTE LC_D_KSD_11 12

STRING(C_D_KSD_12, "ATD#62**13#" )

- BYTE LC_D_KSD_12 11

STRING(C_D_KSD_13, "ATD*#67#" )

- BYTE LC_D_KSD_13 8

STRING(C_D_KSD_14, "ATD*#61**11#" )

- BYTE LC_D_KSD_14 12

STRING(C_D_KSD_15, "ATD*#62#" )

- BYTE LC_D_KSD_15 8

STRING(C_D_KSD_16, "ATD*#67**13#" )

- BYTE LC_D_KSD_16 12

STRING(C_D_KSD_17, "ATD**03*330*1234*9876*9876#" )

- BYTE LC_D_KSD_17 27

STRING(C_D_KSD_18, "ATD*33*1234*22#" )

- BYTE LC_D_KSD_18 15

STRING(C_D_KSD_19, "ATD*351#" )

- BYTE LC_D_KSD_19 8

STRING(C_D_KSD_20, "ATD*331#" )

- BYTE LC_D_KSD_20 8

STRING(C_D_KSD_21, "ATD*35#" )

- BYTE LC_D_KSD_21 7

STRING(C_D_KSD_22, "ATD#330**11#" )

- BYTE LC_D_KSD_22 12

STRING(C_D_KSD_23, "ATD#333**13#" )

- BYTE LC_D_KSD_23 12

STRING(C_D_KSD_24, "ATD#353#" )

- BYTE LC_D_KSD_24 8

STRING(C_D_KSD_25, "ATD#332#" )

- BYTE LC_D_KSD_25 8

STRING(C_D_KSD_26, "ATD*#35#" )

- BYTE LC_D_KSD_26 8

STRING(C_D_KSD_27, "ATD*#332#" )

- BYTE LC_D_KSD_27 9

STRING(C_D_KSD_28, "ATD*#351#" )

- BYTE LC_D_KSD_28 9

STRING(C_D_KSD_29, "ATD*#331#" )

- BYTE LC_D_KSD_29 9

STRING(C_D_KSD_30, "ATD**00#" )

- BYTE LC_D_KSD_30 8

STRING(C_D_KSD_31, "ATD**04*1234*9876*9876#" )

- BYTE LC_D_KSD_31 23

STRING(C_D_KSD_32, "ATD*#06#" )

- BYTE LC_D_KSD_32 8

STRING(C_D_KSD_33, "ATD**05*87654321*9876*9876#" )

- BYTE LC_D_KSD_33 27

STRING(C_D_KSD_34, "ATD**052*87654321*9876*9876#" )

- BYTE LC_D_KSD_34 28

STRING(C_D_KSD_35, "ATD#33**10#" )

- BYTE LC_D_KSD_35 11

STRING(C_D_KSD_36, "ATD*#31#" )

- BYTE LC_D_KSD_36 8

STRING(C_D_KSD_37, "ATD*43#" )

- BYTE LC_D_KSD_37 7

STRING(C_D_KSD_38, "ATD*31#00493039094444;" )

- BYTE LC_D_KSD_38 22

STRING(C_D_KSD_39, "ATD#31#00493039094444;" )

- BYTE LC_D_KSD_39 22

STRING(C_D_KSD_40, "ATD*31#+493039094444;" )

- BYTE LC_D_KSD_40 21

STRING(C_D_KSD_41, "ATD#31#+493039094444;" )

- BYTE LC_D_KSD_41 21

STRING(C_D_KSD_42, "ATD##37*1#" )

- BYTE LC_D_KSD_42 10

STRING(C_D_KSD_43, "ATD##37#" )

- BYTE LC_D_KSD_43 8

STRING(C_D_KSD_44, "ATD*#37#" )

- BYTE LC_D_KSD_44 8

STRING(C_D_KSD_99, "ATD**03*330*0105*1014*1014#" )

- BYTE LC_D_KSD_99 27

STRING(C_D_KSD_100, "ATD*100#" )

- BYTE LC_D_KSD_100 8

/*

- Command:	+CCWA

Call waiting indication

*/

- STRING(C_PLUS_CCWA_QUERY, "AT+CCWA=,2 " )

- BYTE LC_PLUS_CCWA_QUERY 10

- STRING(C_PLUS_CCWA_QUERY2, "AT+CCWA=1,2 " )

- BYTE LC_PLUS_CCWA_QUERY2 11

STRING(C_PLUS_CCWA_ACT_V, "AT+CCWA=,1,1 " )

- BYTE LC_PLUS_CCWA_ACT_V 12

STRING(C_PLUS_CCWA_ACT_D, "AT+CCWA=,1,2 " )

- BYTE LC_PLUS_CCWA_ACT_D 12

STRING(C_PLUS_CCWA_ACT_F, "AT+CCWA=,1,4 " )

- BYTE LC_PLUS_CCWA_ACT_D 12

STRING(C_PLUS_CCWA_DEACT, "AT+CCWA=,0,7 " )

- BYTE LC_PLUS_CCWA_DEACT 12

/*

message:	+CCWA

call waiting presentation

*/

- STRING(M_PLUS_CCWA, "+CCWA: \"03039094223\",129,1," )

BYTE LM_PLUS_CCWA 27
STRING(M_PLUS_CCWA_V, "+CCWA: 1,1" )

BYTE LM_PLUS_CCWA_V 10
STRING(M_PLUS_CCWA_D, "+CCWA: 1,2" )

BYTE LM_PLUS_CCWA_D 10
STRING(M_PLUS_CCWA_F, "+CCWA: 1,4" )

BYTE LM_PLUS_CCWA_F 10

/*

- Command:	+CCFC

Call Forwarding

*/

- STRING(C_PLUS_CCFC_CFU_QUERY, "AT+CCFC=0,2 " )

- BYTE LC_PLUS_CCFC_CFU_QUERY 11

STRING(C_PLUS_CCFC_CFB_QUERY, "AT+CCFC=1,2 " )

- BYTE LC_PLUS_CCFC_CFB_QUERY 11

STRING(C_PLUS_CCFC_CFNRY_QUERY, "AT+CCFC=2,2 " )

- BYTE LC_PLUS_CCFC_CFNRY_QUERY 11

STRING(C_PLUS_CCFC_CFNRC_QUERY, "AT+CCFC=3,2 " )

- BYTE LC_PLUS_CCFC_CFNRC_QUERY 11

STRING(C_PLUS_CCFC_ALLCF_QUERY, "AT+CCFC=4,2 " )

- BYTE LC_PLUS_CCFC_ALLCF_QUERY 11

STRING(C_PLUS_CCFC_ALLCFC_QUERY, "AT+CCFC=5,2 " )

- BYTE LC_PLUS_CCFC_ALLCFC_QUERY 11

STRING(C_PLUS_CCFC_CFU_REG, "AT+CCFC=0,3,\"03039094223\",,7,\"123456\" " )

- BYTE LC_PLUS_CCFC_CFU_REG 38

STRING(C_PLUS_CCFC_CFB_REG, "AT+CCFC=1,3,\"03039094223\",,7,\"123456\" " )

- BYTE LC_PLUS_CCFC_CFB_REG 38

STRING(C_PLUS_CCFC_CFNRY_REG, "AT+CCFC=2,3,\"03039094223\",,7,\"123456\",,16 " )

- BYTE LC_PLUS_CCFC_CFNRY_REG 42

STRING(C_PLUS_CCFC_CFNRC_REG, "AT+CCFC=3,3,\"03039094223\",,7,\"123456\" " )

- BYTE LC_PLUS_CCFC_CFNRC_REG 38

STRING(C_PLUS_CCFC_ALLCF_REG, "AT+CCFC=4,3,\"03039094223\",,7,\"123456\" " )

- BYTE LC_PLUS_CCFC_ALLCF_REG 38

STRING(C_PLUS_CCFC_ALLCFC_REG, "AT+CCFC=5,3,\"03039094223\",,7,\"123456\" " )

- BYTE LC_PLUS_CCFC_ALLCFC_REG 38

STRING(C_PLUS_CCFC_CFU_ERS, "AT+CCFC=0,4,,,7" )

- BYTE LC_PLUS_CCFC_CFU_ERS 15

STRING(C_PLUS_CCFC_CFB_ERS, "AT+CCFC=1,4,,,7" )

- BYTE LC_PLUS_CCFC_CFB_ERS 15

STRING(C_PLUS_CCFC_CFNRY_ERS, "AT+CCFC=2,4,,,7" )

- BYTE LC_PLUS_CCFC_CFNRY_ERS 15

STRING(C_PLUS_CCFC_CFNRC_ERS, "AT+CCFC=3,4,,,7" )

- BYTE LC_PLUS_CCFC_CFNRC_ERS 15

STRING(C_PLUS_CCFC_ALLCF_ERS, "AT+CCFC=4,4,,,7" )

- BYTE LC_PLUS_CCFC_ALLCF_ERS 15

STRING(C_PLUS_CCFC_ALLCFC_ERS, "AT+CCFC=5,4,,,7" )

- BYTE LC_PLUS_CCFC_ALLCFC_ERS 15

STRING(C_PLUS_CCFC_CFU_ACT, "AT+CCFC=0,1,,,7" )

- BYTE LC_PLUS_CCFC_CFU_ACT 15

STRING(C_PLUS_CCFC_CFB_ACT, "AT+CCFC=1,1,,,7" )

- BYTE LC_PLUS_CCFC_CFB_ACT 15

STRING(C_PLUS_CCFC_CFNRY_ACT, "AT+CCFC=2,1,,,7" )

- BYTE LC_PLUS_CCFC_CFNRY_ACT 15

STRING(C_PLUS_CCFC_CFNRC_ACT, "AT+CCFC=3,1,,,7" )

- BYTE LC_PLUS_CCFC_CFNRC_ACT 15

STRING(C_PLUS_CCFC_ALLCF_ACT, "AT+CCFC=4,1,,,7" )

- BYTE LC_PLUS_CCFC_ALLCF_ACT 15

STRING(C_PLUS_CCFC_ALLCFC_ACT, "AT+CCFC=5,1,,,7" )

- BYTE LC_PLUS_CCFC_ALLCFC_ACT 15

STRING(C_PLUS_CCFC_CFU_DEACT, "AT+CCFC=0,0,,,7" )

- BYTE LC_PLUS_CCFC_CFU_DEACT 15

STRING(C_PLUS_CCFC_CFB_DEACT, "AT+CCFC=1,0,,,7" )

- BYTE LC_PLUS_CCFC_CFB_DEACT 15

STRING(C_PLUS_CCFC_CFNRY_DEACT, "AT+CCFC=2,0,,,7" )

- BYTE LC_PLUS_CCFC_CFNRY_DEACT 15

STRING(C_PLUS_CCFC_CFNRC_DEACT, "AT+CCFC=3,0,,,7" )

- BYTE LC_PLUS_CCFC_CFNRC_DEACT 15

STRING(C_PLUS_CCFC_ALLCF_DEACT, "AT+CCFC=4,0,,,7" )

- BYTE LC_PLUS_CCFC_ALLCF_DEACT 15

STRING(C_PLUS_CCFC_ALLCFC_DEACT, "AT+CCFC=5,0,,,7" )

- BYTE LC_PLUS_CCFC_ALLCFC_DEACT 15

/*

message:	+CCFC

call forwarding presentation

*/

- STRING(M_PLUS_CCFC_CFU, "+CCFC: 0,7" )

- BYTE LM_PLUS_CCFC_CFU 10

STRING(M_PLUS_CCFC_CFB_V, "+CCFC: 1,1,\"493039094444\",145" )

- BYTE LM_PLUS_CCFC_CFB_V 29

STRING(M_PLUS_CCFC_CFB_D, "+CCFC: 1,2,\"03039094223\",129" )

- BYTE LM_PLUS_CCFC_CFB_D 28

STRING(M_PLUS_CCFC_CFNRY_V, "+CCFC: 1,1,\"493039094444\",145,,,10" )

- BYTE LM_PLUS_CCFC_CFNRY_V 34

STRING(M_PLUS_CCFC_CFNRY_F, "+CCFC: 1,4" )

- BYTE LM_PLUS_CCFC_CFNRY_F 10

STRING(M_PLUS_CCFC_CFNRY_KSD_V, "+CCFC: 1,1,\"431234\",145" )

- BYTE LM_PLUS_CCFC_CFNRY_KSD_V 23

STRING(M_PLUS_CCFC_CFNRC, "+CCFC: 0,7" )

- BYTE LM_PLUS_CCFC_CFNRC 10

/*

- Command:	+COLP

Calling line presentation mode

*/

- STRING(C_PLUS_COLP_ON, "AT+COLP=1 " )

- BYTE LC_PLUS_COLP_ON 9

STRING(C_PLUS_COLP_QUERY, "AT+COLP? " )

- BYTE LC_PLUS_COLP_QUERY 8

/*

- Command: 	+CFUN

set phone functionality

*/

- STRING(C_PLUS_CFUN_FULL, "AT+CFUN=1 " )

- BYTE LC_PLUS_CFUN_FULL 9

/*

message:	+COLP

calling line presentation

*/

- STRING(M_PLUS_COLP_NUM, "+COLP: \"03039094223\",129,,," )

- BYTE LM_PLUS_COLP_NUM 27

/*

- Command:	+CLIR

Calling line restriction mode

*/

- STRING(C_PLUS_CLIR_SUP, "AT+CLIR=2 " )

- BYTE LC_PLUS_CLIR_SUP 9

STRING(C_PLUS_CLIR_QUERY, "AT+CLIR? " )

- BYTE LC_PLUS_CLIR_QUERY 8

/*

- Message:	+CLIR

Calling line restriction mode

*/

- STRING(M_PLUS_CLIR_PVTMAL, "+CLIR: 0,4" )

- BYTE LM_PLUS_CLIR_PVTMAL 10

/*

- Message:	+CLIR

Calling line restriction mode (2)

*/

- STRING(M_PLUS_CLIR_PVTMAL2, "+CLIR: 0,0" )

- BYTE LM_PLUS_CLIR_PVTMAL2 10

/*

- Command:	+CLIP

Calling line presentation mode

*/

- STRING(C_PLUS_CLIP_QUERY, "AT+CLIP? " )

- BYTE LC_PLUS_CLIP_QUERY 8

/*

- Message:	+CLIP

Calling line presentation mode

*/

- STRING(M_PLUS_CLIP_PROV, "+CLIP: 0,1" )

- BYTE LM_PLUS_CLIP_PROV 10

STRING(M_PLUS_CLIP_NOTPROV, "+CLIP: 0,0" )

- BYTE LM_PLUS_CLIP_NOTPROV 10

STRING(M_PLUS_CLIP_UNKNOWN, "+CLIP: 0,2" )

- BYTE LM_PLUS_CLIP_UNKNOWN 10

/*

- Command:	+CHLD

Call on hold

*/

- STRING(C_PLUS_CHLD_0, "AT+CHLD=0 " )

- BYTE LC_PLUS_CHLD_0 9

STRING(C_PLUS_CHLD_1, "AT+CHLD=1 " )

- BYTE LC_PLUS_CHLD_1 9

STRING(C_PLUS_CHLD_2, "AT+CHLD=2 " )

- BYTE LC_PLUS_CHLD_2 9

STRING(C_PLUS_CHLD_13, "AT+CHLD=13 " )

- BYTE LC_PLUS_CHLD_13 10

/*

- Command:	+CSSN

Supplementary Service Notifications

*/

- STRING(C_PLUS_CSSN_ON, "AT+CSSN=1,1 " )

- BYTE LC_PLUS_CSSN_ON 11

STRING(C_PLUS_CSSN_QUERY, "AT+CSSN? " )

- BYTE LC_PLUS_CSSN_QUERY 8

/*

message:	+CSSN

Supplementary Service Notifications

*/

- STRING(M_PLUS_CSSI_CFU, "+CSSI: 0")

- BYTE LM_PLUS_CSSI_CFU 8

STRING(M_PLUS_CSSI_CW, "+CSSI: 3")

- BYTE LM_PLUS_CSSI_CW 8

STRING(M_PLUS_CSSI_CUG, "+CSSI: 4,5")

- BYTE LM_PLUS_CSSI_CUG 10

STRING(M_PLUS_CSSI_CLIR, "+CSSI: 7")

- BYTE LM_PLUS_CSSI_CLIR 8

- STRING(M_PLUS_CSSU_FWD, “+CSSU: 0”)

STRING(M_PLUS_CSSU_HLD, "+CSSU: 2")

- BYTE LM_PLUS_CSSU_HLD 8

STRING(M_PLUS_CSSU_MPTY, "+CSSU: 4")

- BYTE LM_PLUS_CSSU_MPTY 8

STRING(M_PLUS_CSSU_ECT_ALRT, "+CSSU: 7,,\"03039094223\",129,\"123456\",128")

- BYTE LM_PLUS_CSSU_ECT_ALRT 40

STRING(M_PLUS_CSSU_CHK, "+CSSU: 6" )

- BYTE LM_PLUS_CSSU_CHK 8

/*

- Command:	+CCUG

Closed User Group

*/

- STRING(C_PLUS_CCUG_ON, "AT+CCUG=1,5,3 " )

- BYTE LC_PLUS_CCUG_ON 13

/*

- Command:	+CPWD

Change Password

*/

- STRING(C_PLUS_CPWD_CBALL, "AT+CPWD=\"AB\",\"1234\",\"9876\"" )

- BYTE LC_PLUS_CPWD_CBALL 26

STRING(C_PLUS_CPWD_PIN1, "AT+CPWD=\"SC\",\"1234\",\"9876\"" )

- BYTE LC_PLUS_CPWD_PIN1 26

STRING(C_PLUS_CPWD_PIN2, "AT+CPWD=\"P2\",\"1234\",\"9876\"" )

- BYTE LC_PLUS_CPWD_PIN2 26

/*

- Command:	+CLCK

Facility lock

*/

- STRING(C_PLUS_CLCK_ALLCB_ACT, "AT+CLCK=\"AB\",1,\"1234\"" )

- BYTE LC_PLUS_CLCK_ALLCB_ACT 21

STRING(C_PLUS_CLCK_BAOC_ACT, "AT+CLCK=\"AO\",1,\"1234\",7" )

- BYTE LC_PLUS_CLCK_BAOC_ACT 23

STRING(C_PLUS_CLCK_BAOC_ALL_ACT, "AT+CLCK=\"AO\",1,\"1234\",15" )

- BYTE LC_PLUS_CLCK_BAOC_ALL_ACT 24

STRING(C_PLUS_CLCK_BAOC_DEF_ACT, "AT+CLCK=\"AO\",1,\"1234\"" )

- BYTE LC_PLUS_CLCK_BAOC_DEF_ACT 21

STRING(C_PLUS_CLCK_BOIC_ACT, "AT+CLCK=\"OI\",1,\"1234\",7" )

- BYTE LC_PLUS_CLCK_BOIC_ACT 23

STRING(C_PLUS_CLCK_BOIC_ALL_ACT, "AT+CLCK=\"OI\",1,\"1234\",15" )

- BYTE LC_PLUS_CLCK_BOIC_ALL_ACT 24

STRING(C_PLUS_CLCK_BOIC_DEF_ACT, "AT+CLCK=\"OI\",1,\"1234\"" )

- BYTE LC_PLUS_CLCK_BOIC_DEF_ACT 21

STRING(C_PLUS_CLCK_BOICxHC_ACT, "AT+CLCK=\"OX\",1,\"1234\",7" )

BYTE LC_PLUS_CLCK_BOICxHC_ACT 23

STRING(C_PLUS_CLCK_BOICxHC_ALL_ACT, "AT+CLCK=\"OX\",1,\"1234\",15" )

BYTE LC_PLUS_CLCK_BOICxHC_ALL_ACT 24

STRING(C_PLUS_CLCK_BOICxHC_DEF_ACT, "AT+CLCK=\"OX\",1,\"1234\"" )

- BYTE LC_PLUS_CLCK_BOICxHC_DEF_ACT 21

STRING(C_PLUS_CLCK_BAIC_ACT, "AT+CLCK=\"AI\",1,\"1234\",7" )

- BYTE LC_PLUS_CLCK_BAIC_ACT 23

STRING(C_PLUS_CLCK_BAIC_ALL_ACT, "AT+CLCK=\"AI\",1,\"1234\",15" )

- BYTE LC_PLUS_CLCK_BAIC_ALL_ACT 24

STRING(C_PLUS_CLCK_BAIC_DEF_ACT, "AT+CLCK=\"AI\",1,\"1234\"" )

- BYTE LC_PLUS_CLCK_BAIC_DEF_ACT 21

STRING(C_PLUS_CLCK_BICR_ACT, "AT+CLCK=\"IR\",1,\"1234\",7" )

- BYTE LC_PLUS_CLCK_BICR_ACT 23

STRING(C_PLUS_CLCK_BICR_ALL_ACT, "AT+CLCK=\"IR\",1,\"1234\",15" )

- BYTE LC_PLUS_CLCK_BICR_ALL_ACT 24

STRING(C_PLUS_CLCK_BICR_DEF_ACT, "AT+CLCK=\"IR\",1,\"1234\"" )

- BYTE LC_PLUS_CLCK_BICR_DEF_ACT 21

STRING(C_PLUS_CLCK_ALLOUT_ACT, "AT+CLCK=\"AG\",1,\"1234\",7" )

- BYTE LC_PLUS_CLCK_ALLOUT_ACT 23

STRING(C_PLUS_CLCK_ALLIN_ACT, "AT+CLCK=\"AC\",1,\"1234\",7" )

- BYTE LC_PLUS_CLCK_ALLIN_ACT 23

STRING(C_PLUS_CLCK_PIN1_ACT, "AT+CLCK=\"SC\",1,\"1234\"" )

- BYTE LC_PLUS_CLCK_PIN1_ACT 21

STRING(C_PLUS_CLCK_ALLCB_DEACT, "AT+CLCK=\"AB\",0,\"1234\",7" )

- BYTE LC_PLUS_CLCK_ALLCB_DEACT 23

STRING(C_PLUS_CLCK_BAOC_DEACT, "AT+CLCK=\"AO\",0,\"1234\",7" )

- BYTE LC_PLUS_CLCK_BAOC_DEACT 23

STRING(C_PLUS_CLCK_BAOC_ALL_DEACT, "AT+CLCK=\"AO\",0,\"1234\",15" )

- BYTE LC_PLUS_CLCK_BAOC_ALL_DEACT 24

STRING(C_PLUS_CLCK_BOIC_DEACT, "AT+CLCK=\"OI\",0,\"1234\",7" )

- BYTE LC_PLUS_CLCK_BOIC_DEACT 23

STRING(C_PLUS_CLCK_BOIC_ALL_DEACT, "AT+CLCK=\"OI\",0,\"1234\",15" )

- BYTE LC_PLUS_CLCK_BOIC_ALL_DEACT 24

STRING(C_PLUS_CLCK_BOICxHC_DEACT, "AT+CLCK=\"OX\",0,\"1234\",7" )

BYTE LC_PLUS_CLCK_BOICxHC_DEACT 23

STRING(C_PLUS_CLCK_BOICxHC_ALL_DEACT, "AT+CLCK=\"OX\",0,\"1234\",15" )

BYTE LC_PLUS_CLCK_BOICxHC_ALL_DEACT 24

STRING(C_PLUS_CLCK_BAIC_DEACT, "AT+CLCK=\"AI\",0,\"1234\",7" )

- BYTE LC_PLUS_CLCK_BAIC_DEACT 23

STRING(C_PLUS_CLCK_BAIC_ALL_DEACT, "AT+CLCK=\"AI\",0,\"1234\",15" )

- BYTE LC_PLUS_CLCK_BAIC_ALL_DEACT 24

STRING(C_PLUS_CLCK_BICR_DEACT, "AT+CLCK=\"IR\",0,\"1234\",7" )

- BYTE LC_PLUS_CLCK_BICR_DEACT 23

STRING(C_PLUS_CLCK_BICR_ALL_DEACT, "AT+CLCK=\"IR\",0,\"1234\",15" )

- BYTE LC_PLUS_CLCK_BICR_ALL_DEACT 24

STRING(C_PLUS_CLCK_ALLOUT_DEACT, "AT+CLCK=\"AG\",0,\"1234\",7" )

- BYTE LC_PLUS_CLCK_ALLOUT_DEACT 23

STRING(C_PLUS_CLCK_ALLIN_DEACT, "AT+CLCK=\"AC\",0,\"1234\",7" )

- BYTE LC_PLUS_CLCK_ALLIN_DEACT 23

STRING(C_PLUS_CLCK_PIN1_DEACT, "AT+CLCK=\"SC\",0,\"1234\"" )

- BYTE LC_PLUS_CLCK_PIN1_DEACT 21

STRING(C_PLUS_CLCK_ALLCB_QUERY, "AT+CLCK=\"AB\",2" )

- BYTE LC_PLUS_CLCK_ALLCB_QUERY 14

STRING(C_PLUS_CLCK_BAOC_QUERY, "AT+CLCK=\"AO\",2" )

- BYTE LC_PLUS_CLCK_BAOC_QUERY 14

STRING(C_PLUS_CLCK_BOIC_QUERY, "AT+CLCK=\"OI\",2" )

- BYTE LC_PLUS_CLCK_BOIC_QUERY 14

STRING(C_PLUS_CLCK_BOICxHC_QUERY, "AT+CLCK=\"OX\",2" )

BYTE LC_PLUS_CLCK_BOICxHC_QUERY 14

STRING(C_PLUS_CLCK_BAIC_QUERY, "AT+CLCK=\"AI\",2" )

- BYTE LC_PLUS_CLCK_BAIC_QUERY 14

STRING(C_PLUS_CLCK_BICR_QUERY, "AT+CLCK=\"IR\",2" )

- BYTE LC_PLUS_CLCK_BICR_QUERY 14

STRING(C_PLUS_CLCK_ALLOUT_QUERY, "AT+CLCK=\"AG\",2" )

- BYTE LC_PLUS_CLCK_ALLOUT_QUERY 14

STRING(C_PLUS_CLCK_ALLIN_QUERY, "AT+CLCK=\"AC\",2" )

- BYTE LC_PLUS_CLCK_ALLIN_QUERY 14

STRING(C_PLUS_CLCK_PIN1_QUERY, "AT+CLCK=\"SC\",2" )

- BYTE LC_PLUS_CLCK_PIN1_QUERY 14

/*

- Message:	+CLCK

Facility lock

*/

- STRING(M_PLUS_CLCK_PIN1_ENA, "+CLCK: 1" )

- BYTE LM_PLUS_CLCK_PIN1_ENA 8

STRING(M_PLUS_CLCK_PIN1_DIS, "+CLCK: 0" )

- BYTE LM_PLUS_CLCK_PIN1_DIS 8

STRING(M_PLUS_CLCK_IND_0_15, "+CLCK: 0,15" )

- BYTE LM_PLUS_CLCK_IND_0_15 11

STRING(M_PLUS_CLCK_IND_1_1, "+CLCK: 1,1" )

- BYTE LM_PLUS_CLCK_IND_1_1 10

STRING(M_PLUS_CLCK_IND_1_2, "+CLCK: 1,2" )

- BYTE LM_PLUS_CLCK_IND_1_2 10

STRING(M_PLUS_CLCK_IND_1_4, "+CLCK: 1,4" )

- BYTE LM_PLUS_CLCK_IND_1_4 10

STRING(M_PLUS_CLCK_IND_1_8, "+CLCK: 1,8" )

- BYTE LM_PLUS_CLCK_IND_1_8 10

/*

- Command:	+CUSD

Unstructured Supplementary Service Data

*/

- STRING(C_PLUS_CUSD_ON, "AT+CUSD=1" )

- BYTE LC_PLUS_CUSD_ON 9

STRING(C_PLUS_CUSD_SEND, "AT+CUSD=1,\"ABCDEFGHIJKLMNOPQRSTUVWXYZ@$\",0" )

- BYTE LC_PLUS_CUSD_SEND 42

/*

message:	+CUSD

Unstructured Supplementary Service Data

*/

STRING(M_PLUS_CUSD_NTFY, "+CUSD: 0,\"abcdefghijklmnopqrstuvwxyz\",165" )

- BYTE LM_PLUS_CUSD_NTFY 41

STRING(M_PLUS_CUSD_PROC_RES, "+CUSD: 0,\"abcdefghijklmnopqrstuvwxyz\",165" )

- BYTE LM_PLUS_CUSD_PROC_RES 41

STRING(M_PLUS_CUSD_DAT_RES, "+CUSD: 0,\"abcdefghijklmnopqrstuvwxyz\",0" )

- BYTE LM_PLUS_CUSD_DAT_RES 39

STRING(M_PLUS_CUSD_ATD_RES, "+CUSD: 0,\"abcdefghijklmnopqrstuvwxyz\"" )

- BYTE LM_PLUS_CUSD_ATD_RES 37

STRING(M_PLUS_CUSD_DEFAULT_DCS_PROC_RES, "+CUSD: 0,\"*#100#\",15" )

- BYTE LM_PLUS_CUSD_DEFAULT_DCS_PROC_RES 20

STRING(M_PLUS_CUSD_REQ, "+CUSD: 1,\"abcdefghijklmnopqrstuvwxyz\",165" )

- BYTE LM_PLUS_CUSD_REQ 41

/*

- Command: 	%CCBS

Call Completion Busy Subscriber

*/

- STRING(C_PERCENT_CCBS_ERS_1, "AT%CCBS=,1" )

- BYTE LC_PERCENT_CCBS_ERS_1 10

STRING(C_PERCENT_CCBS_ERS_ALL, "AT%CCBS=,0" )

- BYTE LC_PERCENT_CCBS_ERS_ALL 10

STRING(C_PERCENT_CCBS_ITRG, "AT%CCBS?" )

- BYTE LC_PERCENT_CCBS_ITRG 8

/*

Message: 	+CCWA:

Indication of Call Completion Busy Subscriber

*/

- STRING(M_PERCENT_CCBS_ITRG_N1, "%CCBS: 2,1,\"493039094123\",145,,,1" )

- BYTE LM_PERCENT_CCBS_ITRG_N1 33

STRING(M_PERCENT_CCBS_ITRG_N2, "%CCBS: 2,2,\"493039094456\",145,,,4" )

- BYTE LM_PERCENT_CCBS_ITRG_N2 33

STRING(M_PERCENT_CCBS_ITRG_N3, "%CCBS: 2,4,\"493039094789\",145,,,2" )

- BYTE LM_PERCENT_CCBS_ITRG_N3 33

STRING(M_PERCENT_CCBS_ITRG_P, "%CCBS: 1" )

- BYTE LM_PERCENT_CCBS_ITRG_P 8

STRING(M_PERCENT_CCBS_ITRG_NP, "%CCBS: 0" )

- BYTE LM_PERCENT_CCBS_ITRG_NP 8

/*----------------------------------------  fields ------------------------------------------------------- */

/* chip card identification field */

BEGINARRAY (F_ICC, 10)

0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09, 0x00
ENDARRAY

/* current PIN */

BEGINARRAY (F_CUR_PIN, 8) 0x31, 0x32, 0x33, 0x34, 0xFF, 0xFF, 0xFF, 0xFF ENDARRAY

/* new PIN */

BEGINARRAY (F_NEW_PIN, 8) 0x39, 0x38, 0x37, 0x36, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF ENDARRAY

/* PUK */

BEGINARRAY (F_PUK, 8) 0x38, 0x37, 0x36, 0x35, 0x34, 0x33, 0x32, 0x31, 0xFF ENDARRAY

/* SIM toolkit profile */

BEGINARRAY (F_STK_PRF, 11)

0x0A, 0x00, 0x00, 0x00, 0x00, 0x00,0x00, 0x00, 0x00,0x00,0x00
ENDARRAY

/*----------------------------------------  arrays ------------------------------------------------------ */

BEGINARRAY_PART (NUM_654321, 6) 0x06, 0x05, 0x04, 0x03, 0x02, 0x01 ENDARRAY

/* called number array */

BEGINARRAY_PART (A_CLD_NUM,11) 0x0, 0x3, 0x0, 0x3, 0x9, 0x0, 0x9, 0x4, 0x4, 0x4, 0x4 ENDARRAY

BYTE LA_CLD_NUM 11

BEGINARRAY_PART (A_CLD_NUM_B,7) 0x0, 0x3, 0x0, 0x3, 0x9, 0x0, 0x9 ENDARRAY

BYTE LA_CLD_NUM_B 7

/* SS version */

BEGIN_PSTRUCT ("ss_ver", A_SS_VER_2)

SET_COMP ("len", 0x01)

SET_COMP ("ver", A_SS_VER_2_CONTENT)

- ENDSTRUCT

- BEGINARRAY_PART (A_SS_VER_2_CONTENT, 3)  0x00, 0x00, 0x00 ENDARRAY

BEGIN_PSTRUCT ("ss_ver", A_SS_VER_1)

SET_COMP ("len", 0x00)

SET_COMP ("ver", A_SS_VER_1_CONTENT)

- ENDSTRUCT

- BEGINARRAY_PART (A_SS_VER_1_CONTENT, 3) 0x00, 0x00, 0x00 ENDARRAY

/* forward Aoc FIE */

BEGINARRAY (A_FAC_AOC,45) 0xA1, 0x2B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x7D, 0x30, 0x80, 0x80, 0x01, 0x72, 0xA1, 0x1C, 0x81, 0x02, 0x00, 0x3C, 0x82, 0x02, 0x00, 0x8C, 0x83, 0x02, 0x00, 0x64, 0x84, 0x02, 0x00, 0xFA, 0x85, 0x02, 0x00, 0x00, 0x86, 0x02, 0x00, 0x00, 0x87, 0x02, 0x02, 0x58, 0x00, 0x00 ENDARRAY

- BYTE LA_FAC_AOC 360

/*----------------------------------------  arrays ------------------------------------------------------ */

/* calling number array */

- BEGINARRAY_PART (A_CLG_NUM,11) 0x0, 0x3, 0x0, 0x3, 0x9, 0x0, 0x9, 0x4, 0x2, 0x2, 0x3 ENDARRAY

BYTE LA_CLG_NUM 11

/* called number modified array */

- BEGINARRAY_PART (A_CLD_NUM_MDFY,11) 0x0, 0x3, 0x0, 0x3, 0x9, 0x0, 0x9, 0x4, 0x2, 0x2, 0x3 ENDARRAY

BYTE LA_CLD_NUM_MDFY 11

/* called subaddress modified array */

- BEGINARRAY_PART (A_CLD_SUB_MDFY,7) 0x1, 0x2, 0x3, 0x4, 0x5, 0x6, 0x7 ENDARRAY

BYTE LA_CLD_SUB_MDFY 7

/* international called number array */

BEGINARRAY_PART (A_CLD_NUM_INT,12)

0x4, 0x9, 0x3, 0x0, 0x3, 0x9, 0x0, 0x9, 0x4, 0x4, 0x4, 0x4

- ENDARRAY

- BYTE LA_CLD_NUM_INT 12

- BEGINARRAY_PART (A_CLD_NUM_NINT,14)

0x0, 0x0, 0x4, 0x9, 0x3, 0x0, 0x3, 0x9, 0x0, 0x9, 0x4, 0x4, 0x4, 0x4

- ENDARRAY

- BYTE LA_CLD_NUM_NINT 14

/* international called number array */

- BEGINARRAY_PART (A_CLD_NUM_ECC,11) 0x1, 0x1, 0x2, 0x3, 0x9, 0x0, 0x9, 0x4, 0x4, 0x4, 0x4 ENDARRAY

BYTE LA_CLD_NUM_ECC 3

/*---------------------------------------- Line identification SS ------------------------------------------ */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLIP_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLIP_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLIP_IRGT_CONTENT,13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x11

- ENDARRAY

/* interrogate CLIR FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLIR_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLIR_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLIR_IRGT_CONTENT,13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x12

- ENDARRAY

/* interrogate COLP FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_COLP_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_COLP_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_COLP_IRGT_CONTENT,13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x13

- ENDARRAY

/* interrogate COLR FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_COLR_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_COLR_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_COLR_IRGT_CONTENT,13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x14

- ENDARRAY

/* interrogate result CLIP provisioned FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLIP_IRGT_RES_PROV)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLIP_IRGT_RES_PROV_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLIP_IRGT_RES_PROV_CONTENT,13)

0xA2, 0x0B, 0x02, 0x01, 0x00, 0x30, 0x06, 0x02, 0x01, 0x0E, 0x80, 0x01, 0x05

- ENDARRAY

/* interrogate result CLIP not provisioned FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLIP_IRGT_RES_NOTPROV)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLIP_IRGT_RES_NOTPROV_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLIP_IRGT_RES_NOTPROV_CONTENT,13)

0xA2, 0x0B, 0x02, 0x01, 0x00, 0x30, 0x06, 0x02, 0x01, 0x0E, 0x80, 0x01, 0x00

- ENDARRAY

/* interrogate result CLIP unknown FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLIP_IRGT_RES_UNKNOWN)

SET_COMP ("l_fac", 0x0050)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLIP_IRGT_RES_UNKNOWN_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLIP_IRGT_RES_UNKNOWN_CONTENT,10)

0xA2, 0x0B, 0x02, 0x01, 0x00, 0x30, 0x06, 0x02, 0x01, 0x0E

- ENDARRAY

/* interrogate result CLIR provisioned temp allowed FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLIR_IRGT_RES_PVTMAL)

SET_COMP ("l_fac", 0x0090)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLIR_IRGT_RES_PVTMAL_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLIR_IRGT_RES_PVTMAL_CONTENT,18)

0xA2, 0x10, 0x02, 0x01, 0x00, 0x30, 0x0B, 0x02, 0x01, 0x0E, 0xA4, 0x06, 0x04, 0x01, 0x05, 0x0A, 	0x01, 0x02

- ENDARRAY

/* interrogate result CLIR provisioned temp allowed FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLIR_IRGT_RES_PVTMAL2)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLIR_IRGT_RES_PVTMAL_CONTENT2)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLIR_IRGT_RES_PVTMAL_CONTENT2,13)

0xA2, 0x0B, 0x02, 0x01, 0x00, 0x30, 0x06, 0x02, 0x01, 0x0E, 0x80, 0x01, 0x00

- ENDARRAY

/*----------------------------------------  Call waiting SS------------------------------------------------ */

/* interrogate CCWA FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_IRGT_CONTENT,13)

0xA1, 0x0B, /* Invoke Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

0x30, 0x03, /* Sequence Tag, Length */

0x04, 0x01, 0x41 /* SS Code Tag (Octet String Tag), Length, Value (Call Waiting) */

/*

* No specification of optional basic Service! Note that Condat decided to no longer use

* 2 specific enquiries but one general one, see ACI-FIX-1666

*/

- ENDARRAY

/* interrogate result CCWA FIE, one FIE for both teleservices and bearer services */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_IRGT_RES)

SET_COMP ("l_fac", 168)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_IRGT_RES_CONTENT,21)

0xA2, 0x13, /* Return Result Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x30, 0x0E, /* Sequence Tag, Length */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

/* begin of parameters */

0xA2, 0x09, /* BasicServiceGroupList Tag, Length */

0x83, 0x01, 0x10, /* TeleService Tag, Length, Value (all speech transmission services) */

0x83, 0x01, 0x60, /* TeleService Tag, Length, Value (all FAX transmission services) */

0x82, 0x01, 0x10 /* BearerService Tag, Length, Value (all data CDA services) */

- ENDARRAY

/* interrogate result CCWA FIE, one FIE for both teleservices and bearer services, length of parameters unknown */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_IRGT_RES2)

SET_COMP ("l_fac", 184)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_IRGT_RES_CONTENT2)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_IRGT_RES_CONTENT2,21)

0xA2, 0x15, /* Return Result Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x30, 0x10, /* Sequence Tag, Length */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

/* begin of parameters */

0xA2, 0x80, /* BasicServiceGroupList Tag, Length (unknown) */

0x83, 0x01, 0x10, /* TeleService Tag, Length, Value (all speech transmission services) */

0x83, 0x01, 0x60, /* TeleService Tag, Length, Value (all FAX transmission services) */

0x82, 0x01, 0x10, /* BearerService Tag, Length, Value (all data CDA services) */

0x00, 0x00 /* end */

- ENDARRAY

/* activate CCWA voice FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_ACT_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_ACT_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_ACT_V_CONTENT,16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x41, 0x83, 0x01, 0x10

- ENDARRAY

/* activate result CCWA voice FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_ACT_V_RES)

SET_COMP ("l_fac", 0x00B8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_ACT_V_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_ACT_V_RES_CONTENT,23)

0xA2, 0x15, 0x02, 0x01, 0x00, 0x30, 0x10, 0x02, 0x01, 0x0C, 0xA3, 0x0B, 0x04, 0x01, 0x41, 0x84, 	0x01, 0x05, 0x30, 0x03, 0x83, 0x01, 0x10

- ENDARRAY

/* activate result CCWA voice FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_ACT_V_RES2)

SET_COMP ("l_fac", 0x0050)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_ACT_V_RES2_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_ACT_V_RES2_CONTENT,10)

0xA2, 0x08, 0x02, 0x01, 0x00, 0x30, 0x03, 0x02, 0x01, 0x0C

- ENDARRAY

/* deactivate CCWA voice/fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_DEACT_VF )

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_DEACT_VF_CONTENT,16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x41, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate result CCWA voice/fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00B8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_DEACT_VF_RES_CONTENT )

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_DEACT_VF_RES_CONTENT ,23)

0xA2, 0x15, 0x02, 0x01, 0x00, 0x30, 0x10, 0x02, 0x01, 0x0D, 0xA3, 0x0B, 0x04, 0x01, 0x41, 0x84, 	0x01, 0x04, 0x30, 0x03, 0x83, 0x01, 0x10

- ENDARRAY

/* deactivate error result CCWA voice/fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_DEACT_VF_ERR_RES )

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_DEACT_VF_ERR_RES_CONTENT )

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_DEACT_VF_ERR_RES_CONTENT ,8)

0xA3, 0x06, 0x02, 0x01, 0x00, 0x02, 0x01, 0x24

ENDARRAY

/* deactivate CCWA data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_DEACT_D )

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_DEACT_D_CONTENT,16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x41, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate result CCWA voice/fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_DEACT_D_RES)

SET_COMP ("l_fac", 0x00B8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_DEACT_D_RES_CONTENT )

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_DEACT_D_RES_CONTENT ,23)

0xA2, 0x15, 0x02, 0x01, 0x01, 0x30, 0x10, 0x02, 0x01, 0x0D, 0xA3, 0x0B, 0x04, 0x01, 0x41, 0x84, 	0x01, 0x04, 0x30, 0x03, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CCWA data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_ACT_D_CONTENT,16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x41, 0x82, 0x01, 0x00

- ENDARRAY

/* activate result CCWA data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCWA_ACT_D_RES)

SET_COMP ("l_fac", 0x00B8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCWA_ACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCWA_ACT_D_RES_CONTENT,23)

0xA2, 0x15, 0x02, 0x01, 0x00, 0x30, 0x10, 0x02, 0x01, 0x0C, 0xA3, 0x0B, 0x04, 0x01, 0x41, 0x84, 	0x01, 0x05, 0x30, 0x03, 0x82, 0x01, 0x00

- ENDARRAY

/*----------------------------------------  Call forwarding SS -------------------------------------------- */

/* interrogate CCFC CFU FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFU_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFU_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFU_IRGT_CONTENT,13)

0xA1, 0x0B, /* Invoke Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

0x30, 0x03, /* Sequence Tag, Length */

0x04, 0x01, 0x21 /* SS Code Tag (Octet String Tag), Length, Value (CFU) */

/*

* No specification of optional basic Service! Note that Condat decided to no longer use

* 2 specific enquiries but one general one, see ACI-FIX-1666

*/

- ENDARRAY

/* interrogate CCFC CFB FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFB_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFB_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFB_IRGT_CONTENT,13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x29

- ENDARRAY

/* interrogate CCFC CFNRY FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRY_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRY_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRY_IRGT_CONTENT,13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x2A

- ENDARRAY

/* interrogate CCFC CFNRC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRC_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRC_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRC_IRGT_CONTENT,13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x2B

- ENDARRAY

/* interrogate result CCFC CFU FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFU_IRGT_RES)

SET_COMP ("l_fac", 36 * 8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFU_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFU_IRGT_RES_CONTENT,36)

0xA2, 0x22, /* Return Result Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x30, 0x1D, /* Sequence Tag, Length */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

/* begin of parameters */

0xA3, 0x18, /* forwardingFeatureList */

0x30, 0x06, /* Sequence Tag, Length */

0x83, 0x01, 0x10, /* TeleService Tag, Length, Value (all speech transmission services) */

0x84, 0x01, 0x04, /* ss-Status Tag, Length, Value (provisioned) */

0x30, 0x06, /* Sequence Tag, Length */

0x83, 0x01, 0x60, /* TeleService Tag, Length, Value (all FAX transmission services) */

0x84, 0x01, 0x04, /* ss-Status Tag, Length, Value (provisioned) */

0x30, 0x06, /* Sequence Tag, Length */

0x82, 0x01, 0x10, /* BearerService Tag, Length, Value (all data CDA services) */

0x84, 0x01, 0x04 /* ss-Status Tag, Length, Value (provisioned) */

- ENDARRAY

/* interrogate result CCFC CFB FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFB_IRGT_RES)

SET_COMP ("l_fac", 60 * 8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFB_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFB_IRGT_RES_CONTENT,60)

0xA2, 0x3A,

0x02, 0x01, 0x00,

0x30, 0x35,

0x02, 0x01, 0x0E,

/* begin of parameters */

0xA3, 0x30,

0x30, 0x12,

0x83, 0x01, 0x10, /* TeleService Tag, Length, Value (all speech transmission services) */

0x84, 0x01, 0x07, /* ss-Status Tag, Length, Value (provisioned, active, registered) */

0x85, 0x07, 0x91, 0x94, 0x03, 0x93, 0x90, 0x44, 0x44, /* forwarded to number */

0x86, 0x01, 0x04, /* forwarding options */

0x30, 0x06,

0x83, 0x01, 0x60, /* TeleService Tag, Length, Value (all FAX transmission services) */

0x84, 0x01, 0x00, /* ss-Status Tag, Length, Value (nothing, e.g. not provisioned) */

0x30, 0x12,

0x82, 0x01, 0x10, /* BearerService Tag, Length, Value (all data CDA services) */

0x84, 0x01, 0x07, /* ss-Status Tag, Length, Value (provisioned, active, registered) */

0x85, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, /* forwarded to number */

0x86, 0x01, 0x04 /* forwarding options */

- ENDARRAY

/* interrogate result CCFC CFNRY FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRY_IRGT_RES)

SET_COMP ("l_fac", 51 * 8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRY_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRY_IRGT_RES_CONTENT,51)

0xA2, 0x31,

0x02, 0x01, 0x00,

0x30, 0x2C,

0x02, 0x01, 0x0E,

/* begin of parameters */

0xA3, 0x27, /* forwardingFeatureList */

0x30, 0x15,

0x83, 0x01, 0x10, /* TeleService Tag, Length, Value (all speech transmission services) */

0x84, 0x01, 0x07, /* ss-Status Tag, Length, Value (provisioned, active, registered) */

0x85, 0x07, 0x91, 0x94, 0x03, 0x93, 0x90, 0x44, 0x44, /* forwarded to number */

0x86, 0x01, 0x04, /* forwarding options  */

0x87, 0x01, 0x0A, /* noReplyConditionTime */

0x30, 0x06,

0x83, 0x01, 0x60,  /* TeleService Tag, Length, Value (all FAX transmission services) */

0x84, 0x01, 0x07, /* ss-Status Tag, Length, Value (provisioned, active, registered) */

/* note that nevertheless no forwarded to number is present, although this is contradicting 04.82 */

/* this test vector seems to test if ACI implementation is robust if networks erroneously stick to */

/* ASN.1 purely where ALL elements are syntactically optional and it tests the 07.07 implied */

/* syntax of the CCFC response command for the query case which reads that the forwarded to number */

/* and all following parameters can be omitted */

0x30, 0x06,

0x82, 0x01, 0x10, /* BearerService Tag, Length, Value (all data CDA services) */

0x84, 0x01, 0x04 /* ss-Status Tag, Length, Value (provisioned, not registered) */

- ENDARRAY

/* interrogate result CCFC CFNRC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRC_IRGT_RES)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRC_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRC_IRGT_RES_CONTENT,13)

0xA2, 0x0B, /* Return Result Component Tag, Length */

0x02, 0x01, 0x01, /* Invoke ID Tag, Length, Value */

0x30, 0x06, /* Sequence Tag, Length */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

0x80, 0x01, 0x04 /* ss-Status Tag, Length, Value (provisioned, not registered) */

- ENDARRAY

/* register CCFC CFU Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFU_REG_VF)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFU_REG_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFU_REG_VF_CONTENT, 31)

0xA1, 0x1D, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x15, 0x04, 0x01, 0x21, 0x83, 0x01, 0x80, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65

- ENDARRAY

/* register CCFC CFU Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFU_REG_D)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFU_REG_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFU_REG_D_CONTENT, 31)

0xA1, 0x1D, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0A, 0x30, 0x15, 0x04, 0x01, 0x21, 0x82, 0x01, 0x00, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65

- ENDARRAY

/* register CCFC CFB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFB_REG_VF)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFB_REG_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFB_REG_VF_CONTENT, 31)

0xA1, 0x1D, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x15, 0x04, 0x01, 0x29, 0x83, 0x01, 0x80, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65

- ENDARRAY

/* register CCFC CFB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFB_REG_D)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFB_REG_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFB_REG_D_CONTENT, 31)

0xA1, 0x1D, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0A, 0x30, 0x15, 0x04, 0x01, 0x29, 0x82, 0x01, 0x00, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65

- ENDARRAY

/* register CCFC CFNRY Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRY_REG_VF)

SET_COMP ("l_fac", 0x0110)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRY_REG_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRY_REG_VF_CONTENT, 34)

0xA1, 0x20, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x18, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x80, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65, 0x85, 0x01, 	0x10

- ENDARRAY

/* register CCFC CFNRY Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRY_REG_D)

SET_COMP ("l_fac", 0x0110)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRY_REG_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRY_REG_D_CONTENT, 34)

0xA1, 0x20, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0A, 0x30, 0x18, 0x04, 0x01, 0x2A, 0x82, 0x01, 0x00, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65, 0x85, 0x01, 	0x10

- ENDARRAY

/* register CCFC CFNRC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRC_REG_VF)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRC_REG_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRC_REG_VF_CONTENT, 31)

0xA1, 0x1D, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x15, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x80, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65

- ENDARRAY

/* register CCFC CFNRC Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRC_REG_D)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRC_REG_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRC_REG_D_CONTENT, 31)

0xA1, 0x1D, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0A, 0x30, 0x15, 0x04, 0x01, 0x2B, 0x82, 0x01, 0x00, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65

- ENDARRAY

/* register CCFC all CF Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCF_REG_VF)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCF_REG_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCF_REG_VF_CONTENT, 31)

0xA1, 0x1D, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x15, 0x04, 0x01, 0x20, 0x83, 0x01, 0x80, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65

- ENDARRAY

/* register CCFC all CF Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCF_REG_D)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCF_REG_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCF_REG_D_CONTENT, 31)

0xA1, 0x1D, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0A, 0x30, 0x15, 0x04, 0x01, 0x20, 0x82, 0x01, 0x00, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65

- ENDARRAY

/* register CCFC all CFC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCFC_REG_VF)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCFC_REG_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCFC_REG_VF_CONTENT, 31)

0xA1, 0x1D, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x15, 0x04, 0x01, 0x28, 0x83, 0x01, 0x80, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65

- ENDARRAY

/* register CCFC all CFC Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCFC_REG_D)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCFC_REG_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCFC_REG_D_CONTENT, 31)

0xA1, 0x1D, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0A, 0x30, 0x15, 0x04, 0x01, 0x28, 0x82, 0x01, 0x00, 	0x84, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, 0x86, 0x04, 0x80, 0x21, 0x43, 0x65

- ENDARRAY

/* erase CCFC CFU Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFU_ERS_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFU_ERS_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFU_ERS_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x21, 0x83, 0x01, 0x80

- ENDARRAY

/* erase CCFC CFU Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFU_ERS_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFU_ERS_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFU_ERS_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x21, 0x83, 0x01, 0x80

- ENDARRAY

/* erase CCFC CFB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFB_ERS_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFB_ERS_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFB_ERS_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x29, 0x83, 0x01, 0x80

- ENDARRAY

/* erase CCFC CFB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFB_ERS_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFB_ERS_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFB_ERS_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x29, 0x82, 0x01, 0x00

- ENDARRAY

/* erase CCFC CFNRY Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRY_ERS_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRY_ERS_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRY_ERS_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x80

- ENDARRAY

/* erase CCFC CFNRY Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRY_ERS_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRY_ERS_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRY_ERS_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x82, 0x01, 0x00

- ENDARRAY

/* erase CCFC CFNRC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRC_ERS_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRC_ERS_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRC_ERS_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x80

- ENDARRAY

/* erase CCFC CFNRC Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRC_ERS_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRC_ERS_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRC_ERS_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x82, 0x01, 0x00

- ENDARRAY

/* erase CCFC all CF Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCF_ERS_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCF_ERS_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCF_ERS_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x20, 0x83, 0x01, 0x80

- ENDARRAY

/* erase CCFC all CF Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCF_ERS_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCF_ERS_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCF_ERS_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x20, 0x82, 0x01, 0x00

- ENDARRAY

/* erase CCFC all CFC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCFC_ERS_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCFC_ERS_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCFC_ERS_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x28, 0x83, 0x01, 0x80

- ENDARRAY

/* erase CCFC all CFC Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCFC_ERS_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCFC_ERS_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCFC_ERS_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x28, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CCFC CFU Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFU_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFU_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFU_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x21, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CCFC CFU Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFU_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFU_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFU_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x21, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CCFC CFB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFB_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFB_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFB_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x29, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CCFC CFB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFB_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFB_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFB_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x29, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CCFC CFNRY Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRY_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRY_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRY_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CCFC CFNRY Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRY_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRY_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRY_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CCFC CFNRC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRC_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRC_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRC_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CCFC CFNRC Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRC_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRC_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRC_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CCFC all CF Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCF_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCF_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCF_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x20, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CCFC all CF Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCF_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCF_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCF_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x20, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CCFC all CFC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCFC_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCFC_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCFC_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x28, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CCFC all CFC Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCFC_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCFC_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCFC_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x28, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CCFC CFU Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFU_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFU_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFU_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x21, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CCFC CFU Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFU_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFU_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFU_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x21, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CCFC CFB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFB_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFB_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFB_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x29, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CCFC CFB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFB_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFB_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFB_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x29, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CCFC CFNRY Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRY_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRY_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRY_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CCFC CFNRY Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRY_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRY_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRY_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CCFC CFNRC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRC_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRC_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRC_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CCFC CFNRC Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_CFNRC_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_CFNRC_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_CFNRC_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CCFC all CF Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCF_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCF_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCF_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x20, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CCFC all CF Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCF_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCF_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCF_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x20, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CCFC all CFC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCFC_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCFC_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCFC_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x28, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CCFC all CFC Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCFC_ALLCFC_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCFC_ALLCFC_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCFC_ALLCFC_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x28, 0x82, 0x01, 0x00

- ENDARRAY

/*---------------------------------------- SS Notify --------------------------------------------------- */

/* SS notify  FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_NTFY_SS_1)

SET_COMP ("l_fac", 0x01A8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_NTFY_SS_1_CONTENT)

- ENDSTRUCT

BEGINARRAY (A_FAC_NTFY_SS_1_CONTENT, 53)

0xA1, 0x33, /* Invoke Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x02, 0x01, 0x10, /* Operation Code Tag (NotifySS), Length, Value */

0x30, 0x2B, /* Sequence Tag, Length */

0x81, 0x01, 0x21, /* SS Code Tag, Length, Value */

0x84, 0x01, 0x07, /* SS Status Tag, Length, Value */

0x8E, 0x00, /* Call Waiting Indicator Tag, Length */

0x8F, 0x01, 0x01, /* Call Hold Indicator Tag, Length, Value */

0x90, 0x00, /* MPTY Indicator Tag, Length */

0x91, 0x02, 0x00, 0x05, /* CUG Index Tag, Length Value */

0x92, 0x00, /* CLIR Suppression Indicator Tag, Length */

0xB3, 0x16, /* ECT-Indicator Tag, Length */

0x80, 0x01, 0x00, /* ECT Call State Tag, Length Value */

0xA1, 0x11, /* RDN Tag, Length */

0xA0, 0x0F, /* Presentation Allowed Tag, Length */

0x80, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, /* PartyNumber Tag, Length, Value */

0x81, 0x04, 0x80, 0x21, 0x43, 0x65 /* PartySubaddress Tag, Length, Value */

ENDARRAY

/* SS notify  FIE for ECT in alerting state and override case, ACISS082*/

BEGIN_PSTRUCT ("fac_inf", A_FAC_NTFY_SS_FD)

SET_COMP ("l_fac", 8*18)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_NTFY_SS_FD_CONTENT)

- ENDSTRUCT

BEGINARRAY (A_FAC_NTFY_SS_FD_CONTENT, 18)

0xA1, 0x10, /* Invoke Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x02, 0x01, 0x10, /* Operation Code Tag (NotifySS), Length, Value */

0x30, 0x80, /* Sequence Tag, Length */

0x81, 0x01, 0x29,

0x85, 0x01, 0x02,

0x00, 0x00

ENDARRAY

/* SS notify  FIE for ECT in alerting state and override case, ACISS082*/

BEGIN_PSTRUCT ("fac_inf", A_FAC_NTFY_SS_ECT_RSTR)

SET_COMP ("l_fac", 0x0110)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_NTFY_SS_ECT_RSTR_CONTENT)

- ENDSTRUCT

BEGINARRAY (A_FAC_NTFY_SS_ECT_RSTR_CONTENT, 34)

0xA1, 0x20, /* Invoke Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x02, 0x01, 0x10, /* Operation Code Tag (NotifySS), Length, Value */

0x30, 0x18, /* Sequence Tag, Length */

0xB3, 0x16, /* ECT-Indicator Tag, Length */

0x80, 0x01, 0x00, /* ECT Call State Tag, Length Value */

0xA1, 0x11, /* RDN Tag, Length */

0xA3, 0x0F, /* Presentation Restricted Tag, Length */

0x80, 0x07, 0x81, 0x30, 0x30, 0x09, 0x49, 0x22, 0xF3, /* PartyNumber Tag, Length, Value */

0x81, 0x04, 0x80, 0x21, 0x43, 0x65 /* PartySubaddress Tag, Length, Value */

ENDARRAY

/* check SS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CHECK_SS)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CHECK_SS_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CHECK_SS_CONTENT, 8)

0xA1, 0x06, 0x02, 0x01, 0x00, 0x02, 0x01, 0x26

- ENDARRAY

/*---------------------------------------- CUG control --------------------------------------------------- */

/* CUG index 5 suppress both FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CUG_SS_1)

SET_COMP ("l_fac", 0x0090)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CUG_SS_1_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CUG_SS_1_CONTENT, 18)

0xA1, 0x10, 0x02, 0x01, 0x00, 0x02, 0x01, 0x78, 0x30, 0x08, 0x80, 0x02, 0x00, 0x05, 0x81, 0x00, 	0x82, 0x00

- ENDARRAY

/* CUG defaults FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CUG_SS_2)

SET_COMP ("l_fac", 0x0070)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CUG_SS_2_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CUG_SS_2_CONTENT, 14)

0xA1, 0x0C, 0x02, 0x01, 0x00, 0x02, 0x01, 0x78, 0x30, 0x04, 0x80, 0x02, 0x00, 0x00

- ENDARRAY

/*---------------------------------------- Register Password --------------------------------------------------- */

/* invoke register PWD FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CPWD_ALLCB_REG)

SET_COMP ("l_fac", 0x0058)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CPWD_ALLCB_REG_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CPWD_ALLCB_REG_CONTENT, 11)

0xA1, 0x09, 0x02, 0x01, 0x00, 0x02, 0x01, 0x11, 0x04, 0x01, 0x90

- ENDARRAY

/* invoke get PWD, enter PWD FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_ENTER_PWD_REQ)

SET_COMP ("l_fac", 0x0070)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_ENTER_PWD_REQ_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_ENTER_PWD_REQ_CONTENT, 14)

0xA1, 0x0C, /* Invoke Component Tag, Length */

0x02, 0x01, 0x01, /* Invoke ID Tag, Length, Value */

0x80, 0x01, 0x00, /* Linked ID Tag, Length, Value */

0x02, 0x01, 0x12, /* Operation Code Tag (GetPassword), Length, Value */

0x0A, 0x01, 0x00 /* GuidanceInfo Tag, Length, Value (enterPW) */

- ENDARRAY

/* 2nd invoke get PWD, enter PWD FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_ENTER_PWD_REQ_2)

SET_COMP ("l_fac", 0x0070)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_ENTER_PWD_REQ_2_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_ENTER_PWD_REQ_2_CONTENT, 14)

0xA1, 0x0C, 0x02, 0x01, 0x81, 0x80, 0x01, 0x00, 0x02, 0x01, 0x12, 0x0A, 0x01, 0x00

- ENDARRAY

/* result get PWD, enter PWD FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_ENTER_PWD_RES)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_ENTER_PWD_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_ENTER_PWD_RES_CONTENT, 16)

0xA2, 0x0E, 0x02, 0x01, 0x01, 0x30, 0x09, 0x02, 0x01, 0x12, 0x12, 0x04, 0x31, 0x32, 0x33, 0x34

- ENDARRAY

/* 2nd result get PWD, enter PWD FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_ENTER_PWD_RES_2)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_ENTER_PWD_RES_2_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_ENTER_PWD_RES_2_CONTENT, 16)

0xA2, 0x0E, 0x02, 0x01, 0x81, 0x30, 0x09, 0x02, 0x01, 0x12, 0x12, 0x04, 0x31, 0x32, 0x33, 0x34

- ENDARRAY

/* invoke get PWD, enter new PWD FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_NEW_PWD_REQ)

SET_COMP ("l_fac", 0x0070)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_NEW_PWD_REQ_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_NEW_PWD_REQ_CONTENT, 14)

0xA1, 0x0C, 0x02, 0x01, 0x81, 0x80, 0x01, 0x00, 0x02, 0x01, 0x12, 0x0A, 0x01, 0x01

- ENDARRAY

/* result get PWD, enter new PWD FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_NEW_PWD_RES)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_NEW_PWD_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_NEW_PWD_RES_CONTENT, 16)

0xA2, 0x0E, 0x02, 0x01, 0x81, 0x30, 0x09, 0x02, 0x01, 0x12, 0x12, 0x04, 0x39, 0x38, 0x37, 0x36

- ENDARRAY

/* invoke get PWD, enter new PWD again FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_NEWAGN_PWD_REQ)

SET_COMP ("l_fac", 0x0070)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_NEWAGN_PWD_REQ_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_NEWAGN_PWD_REQ_CONTENT, 14)

0xA1, 0x0C, 0x02, 0x01, 0x82, 0x80, 0x01, 0x00, 0x02, 0x01, 0x12, 0x0A, 0x01, 0x02

- ENDARRAY

/* result get PWD, enter new PWD again FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_NEWAGN_PWD_RES)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_NEWAGN_PWD_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_NEWAGN_PWD_RES_CONTENT, 16)

0xA2, 0x0E, 0x02, 0x01, 0x82, 0x30, 0x09, 0x02, 0x01, 0x12, 0x12, 0x04, 0x39, 0x38, 0x37, 0x36

- ENDARRAY

/* result register PWD FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CPWD_ALLCB_RES)

SET_COMP ("l_fac", 0x00A0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CPWD_ALLCB_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CPWD_ALLCB_RES_CONTENT, 20)

0xA2, 0x80, 0x02, 0x01, 0x00, 0x30, 0x80, 0x02, 0x01, 0x11, 0x12, 0x04, 0x31, 0x32, 0x33, 0x34, 	0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* error register PWD FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CPWD_ALLCB_ERR_1)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CPWD_ALLCB_ERR_1_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CPWD_ALLCB_ERR_1_CONTENT, 8)

0xA3, 0x06, 0x02, 0x01, 0x00, 0x02, 0x01, 0x13

- ENDARRAY

/* error register PWD FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CPWD_ALLCB_ERR_2)

SET_COMP ("l_fac", 0x0050)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CPWD_ALLCB_ERR_2_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CPWD_ALLCB_ERR_2_CONTENT, 10)

0xA3, 0x80, 0x02, 0x01, 0x00, 0x02, 0x01, 0x26, 0x00, 0x00

- ENDARRAY

/* error register PWD FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CPWD_ALLCB_ERR_3)

SET_COMP ("l_fac", 0x0058)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CPWD_ALLCB_ERR_3_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CPWD_ALLCB_ERR_3_CONTENT, 11)

0xA3, 0x09, 0x02, 0x01, 0x00, 0x02, 0x01, 0x25, 0x0A, 0x01, 0x02

- ENDARRAY

/*----------------------------------------  Call barring SS -------------------------------------------- */

/* interrogate CLCK BAOC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_IRGT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x92

- ENDARRAY

/* interrogate CLCK BOIC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_IRGT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x93

- ENDARRAY

/* interrogate CLCK BAOC ex HC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_IRGT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x94

- ENDARRAY

/* interrogate CLCK BAIC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_IRGT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x9A

- ENDARRAY

/* interrogate CLCK BICR FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_IRGT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x9B

- ENDARRAY

/* activate CLCK all CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLCB_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLCB_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLCB_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x90, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CLCK all CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLCB_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLCB_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLCB_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x90, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CLCK all out CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLOUT_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLOUT_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLOUT_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x91, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CLCK all out CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLOUT_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLOUT_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLOUT_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x91, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CLCK all in CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLIN_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLIN_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLIN_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x99, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CLCK all in CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLIN_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLIN_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLIN_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x99, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CLCK BAOC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_ACT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_ACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_ACT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x92

- ENDARRAY

/* activate CLCK BAOC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x92, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CLCK BAOC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_ALL_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_ALL_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_ALL_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x92, 0x83, 0x01, 0x00

- ENDARRAY

/* activate CLCK BAOC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x92, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CLCK BOIC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_ACT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_ACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_ACT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x93

- ENDARRAY

/* activate CLCK BOIC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x93, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CLCK BOIC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_ALL_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_ALL_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_ALL_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x93, 0x83, 0x01, 0x00

- ENDARRAY

/* activate CLCK BOIC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x93, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CLCK BOIC ex HC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_ACT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_ACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_ACT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x94

- ENDARRAY

/* activate CLCK BOIC ex HC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x94, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CLCK BOIC ex HC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_ALL_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_ALL_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_ALL_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x94, 0x83, 0x01, 0x00

- ENDARRAY

/* activate CLCK BOIC ex HC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x94, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CLCK BAIC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_ACT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_ACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_ACT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x9A

- ENDARRAY

/* activate CLCK BAIC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x9A, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CLCK BAIC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_ALL_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_ALL_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_ALL_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x9A, 0x83, 0x01, 0x00

- ENDARRAY

/* activate CLCK BAIC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x9A, 0x82, 0x01, 0x00

- ENDARRAY

/* activate CLCK BICR FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_ACT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_ACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_ACT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x9B

- ENDARRAY

/* activate CLCK BICR Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x9B, 0x83, 0x01, 0x80

- ENDARRAY

/* activate CLCK BICR Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_ALL_ACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_ALL_ACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_ALL_ACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x9B, 0x83, 0x01, 0x00

- ENDARRAY

/* activate CLCK BICR CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_ACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_ACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_ACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x9B, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK all CB Voice/Fax FIE */ //ENZ

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLCB_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLCB_DEACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLCB_DEACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x90, 0x30, 	0x08, 0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK all CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLCB_DEACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLCB_DEACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLCB_DEACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x01, 0x30, 0x13, 0x02, 0x01, 0x0D, 0x30, 0x0D, 0x04, 0x01, 0x90, 0x30, 	0x08, 0x30, 0x06, 0x82, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK all out CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLOUT_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLOUT_DEACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLOUT_DEACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x91, 0x30, 	0x08, 0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK all out CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLOUT_DEACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLOUT_DEACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLOUT_DEACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x91, 0x30, 	0x08, 0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK all in CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLIN_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLIN_DEACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLIN_DEACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x99, 0x30, 	0x08, 0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK all in CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLIN_DEACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLIN_DEACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLIN_DEACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x99, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BAOC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_DEACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_DEACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x92, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BAOC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_ALL_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_ALL_DEACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_ALL_DEACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x92, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BAOC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_DEACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_DEACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_DEACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x92, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BOIC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_DEACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_DEACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x93, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BOIC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_ALL_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_ALL_DEACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_ALL_DEACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x93, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BOIC Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_DEACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_DEACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_DEACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x93, 0x30, 	0x08,0x30, 0x06, 0x82, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BOIC ex HC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_DACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_DACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_DACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x94, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BOIC ex HC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_ALL_DACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_ALL_DACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_ALL_DACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x94, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BOIC ex HC Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_DACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_DACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_DACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x94, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BAIC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_DEACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_DEACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x9A, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BAIC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_ALL_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_ALL_DEACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_ALL_DEACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x9A, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BAIC Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_DEACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_DEACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_DEACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x9A, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BICR Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_DEACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_DEACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x9B, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BICR Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_ALL_DEACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_ALL_DEACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_ALL_DEACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x9B, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK BICR Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_DEACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_DEACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_DEACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x9B, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* deactivate CLCK all CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLCB_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLCB_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLCB_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x90, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CLCK all CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLCB_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLCB_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLCB_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x90, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK all out CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLOUT_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLOUT_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLOUT_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x91, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CLCK all out CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLOUT_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLOUT_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLOUT_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x91, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK all in CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLIN_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLIN_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLIN_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x99, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CLCK all in CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLIN_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLIN_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLIN_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x99, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK BAOC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x92, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CLCK BAOC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_ALL_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_ALL_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_ALL_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x92, 0x83, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK BAOC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x92, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK BOIC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x93, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CLCK BOIC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_ALL_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_ALL_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_ALL_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x93, 0x83, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK BOIC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x93, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK BOIC ex HC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x94, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CLCK BOIC ex HC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_ALL_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_ALL_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_ALL_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x94, 0x83, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK BOIC ex HC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x94, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK BAIC Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x9A, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CLCK BAIC Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_ALL_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_ALL_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_ALL_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x9A, 0x83, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK BAIC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x9A, 0x82, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK BICR Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x9B, 0x83, 0x01, 0x80

- ENDARRAY

/* deactivate CLCK BICR Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_ALL_DEACT_VF)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_ALL_DEACT_VF_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_ALL_DEACT_VF_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x9B, 0x83, 0x01, 0x00

- ENDARRAY

/* deactivate CLCK BICR CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_DEACT_D)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_DEACT_D_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_DEACT_D_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x01, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x9B, 0x82, 0x01, 0x00

- ENDARRAY

/* activate result CLCK all CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLCB_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLCB_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLCB_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x90, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK all CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLCB_ACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLCB_ACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLCB_ACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x01, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x90, 0x30, 	0x08,0x30, 0x06, 0x82, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK all out CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLOUT_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLOUT_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLOUT_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x91, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK all out CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLOUT_ACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLOUT_ACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLOUT_ACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x01, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x91, 0x30, 	0x08,0x30, 0x06, 0x82, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK all in CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLIN_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLIN_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLIN_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x99, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK all in CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_ALLIN_ACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_ALLIN_ACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_ALLIN_ACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x01, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x99, 0x30, 	0x08,0x30, 0x06, 0x82, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BAOC CB FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_ACT_RES)

SET_COMP ("l_fac", 0x00B8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_ACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_ACT_RES_CONTENT, 22)

0xA2, 0x15, 0x02, 0x01, 0x00, 0x30, 0x10, 0x02, 0x01, 0x0C, 0xA1, 0x0A, 0x04, 0x01, 0x92, 0x30, 	0x05,0x30, 0x03, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BAOC CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x92, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BAOC CB Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_ALL_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_ALL_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_ALL_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x92, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BAOC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAOC_ACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAOC_ACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAOC_ACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x01, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x92, 0x30,	0x08,0x30, 0x06, 0x82, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BOIC CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x93, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BOIC CB Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_ALL_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_ALL_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_ALL_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x93, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BOIC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOIC_ACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOIC_ACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOIC_ACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x01, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x93, 0x30, 	0x08,0x30, 0x06, 0x82, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BOICxHC CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x94, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BOICxHC CB Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_ALL_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_ALL_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_ALL_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x94, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BOIC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BOICxHC_ACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BOICxHC_ACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BOICxHC_ACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x01, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x94, 0x30, 	0x08,0x30, 0x06, 0x82, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BAIC CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x9A, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BAIC CB Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_ALL_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_ALL_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_ALL_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x9A, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BAIC CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BAIC_ACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BAIC_ACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BAIC_ACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x01, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x9A, 0x30, 	0x08,0x30, 0x06, 0x82, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BICR CB Voice/Fax FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x9B, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x80, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BICR CB Voice/Fax/SMS FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_ALL_ACT_VF_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_ALL_ACT_VF_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_ALL_ACT_VF_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x9B, 0x30, 	0x08,0x30, 0x06, 0x83, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/* activate result CLCK BICR CB Data FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CLCK_BICR_ACT_D_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CLCK_BICR_ACT_D_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CLCK_BICR_ACT_D_RES_CONTENT, 25)

0xA2, 0x18, 0x02, 0x01, 0x01, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0D, 0x04, 0x01, 0x9B, 0x30, 	0x08,0x30, 0x06, 0x82, 0x01, 0x00, 0x84, 0x01, 0x07

- ENDARRAY

/*----------------------------------------  unstructured SS data -------------------------------------------- */

/* unstructured SS request FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_REQ)

SET_COMP ("l_fac", 0x0130)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_REQ_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_REQ_CONTENT, 38)

0xA1, 0x24, 0x02, 0x01, 0x80, 0x02, 0x01, 0x3C, 0x30, 0x1C, 0x04, 0x01, 0xA5, 0x04, 0x17, 0x61, 0xF1, 	0x98, 0x5C, 0x36, 0x9F, 0xD1, 0x69, 0xF5, 0x9A, 0xDD, 0x76, 0xBF, 0xE1, 0x71, 0xF9, 0x9C, 0x5E, 0xB7, 	0xDF, 0xF1, 0x79, 0x3D

- ENDARRAY

/* unstructured SS notify FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_NTFY)

SET_COMP ("l_fac", 0x0130)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_NTFY_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_NTFY_CONTENT, 38)

0xA1, 0x24, 0x02, 0x01, 0x80, 0x02, 0x01, 0x3D, 0x30, 0x1C, 0x04, 0x01, 0xA5, 0x04, 0x17, 0x61, 	0xF1, 0x98, 0x5C, 0x36, 0x9F, 0xD1, 0x69, 0xF5, 0x9A, 0xDD, 0x76, 0xBF, 0xE1, 0x71, 0xF9, 0x9C, 0x5E, 	0xB7, 0xDF, 0xF1, 0x79, 0x3D

- ENDARRAY

/* process unstructured SS request FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_PROC)

SET_COMP ("l_fac", 0x0140)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_PROC_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_PROC_CONTENT, 40)

0xA1, 0x26, 0x02, 0x01, 0x00, 0x02, 0x01, 0x3B, 0x30, 0x1E, 0x04, 0x01, 0x00, 0x04, 0x19, 0x41, 	0xE1, 0x90, 0x58, 0x34, 0x1E, 0x91, 0x49, 0xE5, 0x92, 0xD9, 0x74, 0x3E, 0xA1, 0x51, 0xE9, 0x94, 0x5A, 	0xB5, 0x5E, 0xB1, 0x59, 0x2D, 0x40, 0x00

- ENDARRAY

/* process unstructured SS request FIE initiated by KSD */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_PROC_KSD)

SET_COMP ("l_fac", 0x00A0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_PROC_KSD_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_PROC_KSD_CONTENT, 20)

0xA1, 0x12, 0x02, 0x01, 0x00, 0x02, 0x01, 0x3B, 0x30, 0x0A, 0x04, 0x01, 0x0F, 0x04, 0x05, 0x2A, 	0x15, 0x0C, 0x36, 0x02

- ENDARRAY

/* unstructured SS request result FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_REQ_RES)

SET_COMP ("l_fac", 0x0150)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_REQ_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_REQ_RES_CONTENT, 42)

0xA2, 0x28, 0x02, 0x01, 0x80, 0x30, 0x23, 0x02, 0x01, 0x3C, 0x30, 0x1E, 0x04, 0x01, 0x00, 0x04, 	0x19, 0x41, 0xE1, 0x90, 0x58, 0x34, 0x1E, 0x91, 0x49, 0xE5, 0x92, 0xD9, 0x74, 0x3E, 0xA1, 0x51, 0xE9, 	0x94, 0x5A, 0xB5, 0x5E, 0xB1, 0x59, 0x2D, 0x40, 0x00

- ENDARRAY

/* unstructured SS data result FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_DAT_RES)

SET_COMP ("l_fac", 0x0138)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_DAT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_DAT_RES_CONTENT, 39)

0xA2, 0x25, 0x02, 0x01, 0x01, 0x30, 0x1F, 0x02, 0x01, 0x13, 0x16, 0x1A, 0x61, 0x62, 0x63, 0x64, 	0x65, 0x66, 0x67, 0x68, 0x69, 0x6A, 0x6B, 0x6C, 0x6D, 0x6E, 0x6F, 0x70, 0x71, 0x72, 0x73, 0x74, 	0x75, 0x76, 0x77, 0x78, 0x79, 0x7A

- ENDARRAY

/* unstructured SS notify result FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_NTFY_RES)

SET_COMP ("l_fac", 0x0028)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_NTFY_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_NTFY_RES_CONTENT, 5)

0xA2, 0x03, 0x02, 0x01, 0x80

- ENDARRAY

/* process unstructured SS request result  FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_PROC_RES)

SET_COMP ("l_fac", 0x0158)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_PROC_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_PROC_RES_CONTENT, 40)

0xA2, 0x26, 0x02, 0x01, 0x00, 0x30, 0x24, 0x02, 0x01, 0x3B, 0x30, 0x1C, 0x04, 0x01, 0xA5, 0x04, 	0x17, 0x61, 0xF1, 0x98, 0x5C, 0x36, 0x9F, 0xD1, 0x69, 0xF5, 0x9A, 0xDD, 0x76, 0xBF, 0xE1, 0x71, 0xF9, 	0x9C, 0x5E, 0xB7, 0xDF, 0xF1, 0x79, 0x3D

- ENDARRAY

/* process unstructured SS request FIE IA5 coded */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_PROC_IA5)

SET_COMP ("l_fac", 0x0130)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_PROC_IA5_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_PROC_IA5_CONTENT, 38)

0xA1, 0x24, 0x02, 0x01, 0x01, 0x02, 0x01, 0x13, 0x16, 0x1C, 0x41, 0x42, 0x43, 0x44, 0x45, 0x46, 	0x47, 0x48, 0x49, 0x4A, 0x4B, 0x4C, 0x4D, 0x4E, 0x4F, 0x50, 0x51, 0x52, 0x53, 0x54, 0x55, 0x56, 	0x57, 0x58, 0x59, 0x5A, 0x40, 0x24

- ENDARRAY

/* process unstructured SS request FIE inititated by KSD IA5 coded */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_PROC_KSD_IA5)

SET_COMP ("l_fac", 0x0078)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_PROC_KSD_IA5_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_PROC_KSD_IA5_CONTENT, 15)

0xA1, 0x0D, 0x02, 0x01, 0x01, 0x02, 0x01, 0x13, 0x16, 0x05, 0x2A, 0x2A, 0x30, 0x30, 0x23

- ENDARRAY

/* process unstructured SS reject FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_PROC_REJ)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_PROC_REJ_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_PROC_REJ_CONTENT, 8)

0xA4, 0x06, 0x02, 0x01, 0x00, 0x81, 0x01, 0x01

- ENDARRAY

/* process unstructured SS busy error FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_USSD_BUSY_ERR)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_USSD_BUSY_ERR_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_USSD_BUSY_ERR_CONTENT, 8)

0xA3, 0x06, 0x02, 0x01, 0x80, 0x02, 0x01, 0x48

- ENDARRAY

/*----------------------------------------  call completion busy subscriber --------------------------------- */

/* erase CCBS entry 1 FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCBS_ERS_1)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCBS_ERS_1_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCBS_ERS_1_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x4D, 0x30, 0x06, 0x80, 0x01, 0x43, 0x81, 0x01, 0x01

- ENDARRAY

/* erase CCBS all entries FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCBS_ERS_ALL)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCBS_ERS_ALL_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCBS_ERS_ALL_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x4D, 0x30, 0x03, 0x80, 0x01, 0x43

- ENDARRAY

/* erase CCBS all entries FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCBS_ERS_1_RES)

SET_COMP ("l_fac", 0x0090)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCBS_ERS_1_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCBS_ERS_1_RES_CONTENT, 18)

0xA2, 0x0B, 0x02, 0x01, 0x00, 0x30, 0x08, 0x02, 0x01, 0x4D, 0x30, 0x03, 0x80, 0x01, 0x43

- ENDARRAY

/* erase CCBS all entries FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCBS_ERS_ALL_RES)

SET_COMP ("l_fac", 0x0078)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCBS_ERS_ALL_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCBS_ERS_ALL_RES_CONTENT, 15)

0xA2, 0x0D, 0x02, 0x01, 0x00, 0x30, 0x08, 0x02, 0x01, 0x4D

- ENDARRAY

/* interrogate CCBS entry FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCBS_ITRG)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCBS_ITRG_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCBS_ITRG_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x43

- ENDARRAY

/* interrogate result CCBS entry FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCBS_ITRG_3N_RES)

SET_COMP ("l_fac", 0x0250)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCBS_ITRG_3N_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCBS_ITRG_3N_RES_CONTENT, 74)

0xA2, 0x48, /* Return Result Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x30, 0x43, /* Sequence Tag, Length */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

/* begin of parameters */

0xA4, 0x3E, /* genericServiceInfo Tag, Length */

0x04, 0x01, 0x07, /* ss-Status: provisioned, registered, active */

0xA2, 0x39, /* ccbs-FeatureList */

0x30, 0x11, /* Sequence Tag, Length, entry #1 */

0x80, 0x01, 0x01, /* ccbs-Index 1 */

0x81, 0x07, 0x91, 0x94, 0x03, 0x93, 0x90, 0x14, 0x32, /* b-subscriber number */

0xA3, 0x03, /* Basic Service Group Tag, Length */

0x83, 0x01, 0x10, /* TeleService Tag, Length, Value (all speech) */

0x30, 0x11, /* Sequence Tag, Length, entry #2 */

0x80, 0x01, 0x02, /* ccbs-Index 2 */

0x81, 0x07, 0x91, 0x94, 0x03, 0x93, 0x90, 0x44, 0x65, /* b-subscriber number */

0xA3, 0x03, /* Basic Service Group Tag, Length */

0x83, 0x01, 0x60, /* TeleService Tag, Length, Value (all fax) */

0x30, 0x11, /* Sequence Tag, Length, entry #3 */

0x80, 0x01, 0x04, /* ccbs-Index 4 */

0x81, 0x07, 0x91, 0x94, 0x03, 0x93, 0x90, 0x74, 0x98, /* b-subscriber number */

0xA3, 0x03, /* Basic Service Group Tag, Length */

0x82, 0x01, 0x10

- ENDARRAY

/* interrogate result CCBS entry FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCBS_ITRG_P_RES)

SET_COMP ("l_fac", 0x0078)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCBS_ITRG_P_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCBS_ITRG_P_RES_CONTENT, 15)

0xA2, 0x0D, 0x02, 0x01, 0x00, 0x30, 0x08, 0x02, 0x01, 0x0E,

0xA4, 0x03,

0x04, 0x01, 0x04 /* ss-status provisioned */

- ENDARRAY

/* interrogate result CCBS entry FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_CCBS_ITRG_NP_RES)

SET_COMP ("l_fac", 0x0078)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_CCBS_ITRG_NP_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_CCBS_ITRG_NP_RES_CONTENT, 15)

0xA2, 0x0D, 0x02, 0x01, 0x00, 0x30, 0x08, 0x02, 0x01, 0x0E,

0xA4, 0x03,

0x04, 0x01, 0x00 /* ss-status nothing, i.e. not provisioned */

- ENDARRAY

/*----------------------------------------  keystroke sequences  -------------------------------------------- */

/* register CFNRY Voice FIE 31.2.1.1.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRY_REG_V)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRY_REG_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRY_REG_V_CONTENT, 26)

0xA1, 0x18,

0x02, 0x01, 0x00,

0x02, 0x01, 0x0A,

0x30, 0x10,

0x04, 0x01, 0x2A,

0x83, 0x01, 0x10,

0x84, 0x05, 0x81, 0x00, 0x34, 0x21, 0x43,

0x85, 0x01, 0x05

- ENDARRAY

/* register CFNRY Voice FIE 31.2.1.1.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRY_REG_VB)

SET_COMP ("l_fac", 0x00C8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRY_REG_VB_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRY_REG_VB_CONTENT, 25)

0xA1, 0x17, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0F, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x10, 	0x84, 0x04, 0x91, 0x34, 0x21, 0x43, 0x85, 0x01, 0x05

- ENDARRAY

/* result register CFNRY Voice FIE 31.2.1.1.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRY_REG_RES_A)

SET_COMP ("l_fac", 0x0110)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRY_REG_RES_A_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRY_REG_RES_A_CONTENT, 34)

0xA2, 0x20, 0x02, 0x01, 0x00, 0x30, 0x1B, 0x02, 0x01, 0x0A, 0xA0, 0x16, 0x04, 0x01, 0x2A, 0x30, 	0x11, 0x30, 0x0F, 0x83, 0x01, 0x11, 0x84, 0x01, 0x07, 0x85, 0x04, 0x91, 0x34, 0x21, 0x43, 0x87, 0x01, 	0x05

- ENDARRAY

/* result register CFNRY Voice FIE unlimited length coding 31.2.1.1.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRY_REG_RES_B)

SET_COMP ("l_fac", 0x0168)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRY_REG_RES_B_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRY_REG_RES_B_CONTENT, 45)

0xA2, 0x80, 0x02, 0x01, 0x00, 0x30, 0x80, 0x02, 0x01, 0x0A, 0xA0, 0x80, 0x04, 0x01, 0x2A, 0x30, 	0x80, 0x30, 0x80, 0x83, 0x01, 0x11, 0x84, 0x01, 0x07, 0x85, 0x05, 0x81, 0x00, 0x34, 0x21, 0x43, 	0x87, 0x01, 0x05, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* register CFU Voice FIE 31.2.1.1.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFU_REG_V)

SET_COMP ("l_fac", 0x00B8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFU_REG_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFU_REG_V_CONTENT, 23)

0xA1, 0x15, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0D, 0x04, 0x01, 0x21, 0x83, 0x01, 0x60, 	0x84, 0x05, 0x81, 0x00, 0x34, 0x21, 0x43

- ENDARRAY

/* result register CFU Voice FIE 31.2.1.1.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFU_REG_RES)

SET_COMP ("l_fac", 0x0148)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFU_REG_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFU_REG_RES_CONTENT, 41)

0xA2, 0x80, 0x02, 0x01, 0x00, 0x30, 0x80, 0x02, 0x01, 0x0A, 0xA0, 0x80, 0x04, 0x01, 0x21, 0x30, 	0x80, 0x30, 0x80, 0x83, 0x01, 0x60, 0x84, 0x01, 0x07, 0x85, 0x04, 0x91, 0x34, 0x21, 0x43, 0x00, 	0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* register CFB Voice FIE 31.2.1.1.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFB_REG_V)

SET_COMP ("l_fac", 0x00B8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFB_REG_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFB_REG_V_CONTENT, 22)

0xA1, 0x15, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0D, 0x04, 0x01, 0x29, 0x82, 0x01, 0x60, 	0x84, 0x05, 0x81, 0x00, 0x34, 0x21, 0x43

- ENDARRAY

/* result register CFB Voice FIE 31.2.1.1.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFB_REG_RES)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFB_REG_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFB_REG_RES_CONTENT, 8)

0xA3, 0x06, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A

- ENDARRAY

/* register CF Voice FIE 31.2.1.1.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CF_REG_V)

SET_COMP ("l_fac", 0x00B8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CF_REG_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CF_REG_V_CONTENT, 22)

0xA1, 0x15, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0A, 0x30, 0x0D, 0x04, 0x01, 0x20, 0x83, 0x01, 0x60, 	0x84, 0x05, 0x81, 0x00, 0x34, 0x21, 0x43

- ENDARRAY

/* result register CF Voice FIE 31.2.1.1.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CF_REG_RES)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CF_REG_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CF_REG_RES_CONTENT, 8)

0xA4, 0x06, 0x02, 0x01, 0x00, 0x81, 0x01, 0x03

- ENDARRAY

/* erase CFC Voice FIE 31.2.1.2.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFC_ERS_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFC_ERS_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFC_ERS_V_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x28, 0x83, 0x01, 0x60

- ENDARRAY

/* result erase CFC Voice FIE 31.2.1.2.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFC_ERS_RES)

SET_COMP ("l_fac", 0x00F8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFC_ERS_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFC_ERS_RES_CONTENT, 31)

0xA2, 0x1D, 0x02, 0x01, 0x00, 0x30, 0x18, 0x02, 0x01, 0x0B, 0xA0, 0x80, 0x04, 0x01, 0x28, 0x30, 	0x80, 0x30, 0x80, 0x83, 0x01, 0x60, 0x84, 0x01, 0x04, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* erase CFNRC Voice FIE 31.2.1.2.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRC_ERS_V)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRC_ERS_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRC_ERS_V_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x03, 0x04, 0x01, 0x2B

- ENDARRAY

/* result erase CFNRC Voice FIE 31.2.1.2.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRC_ERS_RES)

SET_COMP ("l_fac", 0x00B0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRC_ERS_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRC_ERS_RES_CONTENT, 22)

0xA2, 0x14, 0x02, 0x01, 0x00, 0x30, 0x0F, 0x02, 0x01, 0x0B, 0xA0, 0x0A, 0x04, 0x01, 0x2B, 0x30, 	0x05, 0x30, 0x03, 0x84, 0x01, 0x04

- ENDARRAY

/* erase CFU Voice FIE 31.2.1.2.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFU_ERS_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFU_ERS_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFU_ERS_V_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x21, 0x83, 0x01, 0x10

- ENDARRAY

/* result erase CFU Voice FIE 31.2.1.2.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFU_ERS_RES)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFU_ERS_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFU_ERS_RES_CONTENT, 8)

0xA3, 0x06, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B

- ENDARRAY

/* erase CFNRY Voice FIE 31.2.1.2.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRY_ERS_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRY_ERS_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRY_ERS_V_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0B, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x60

- ENDARRAY

/* result erase CFNRY Voice FIE 31.2.1.2.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRY_ERS_RES)

SET_COMP ("l_fac", 0x0050)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRY_ERS_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRY_ERS_RES_CONTENT, 10)

0xA4, 0x80, 0x02, 0x01, 0x00, 0x81, 0x01, 0x03, 0x00, 0x00

- ENDARRAY

/* activate CF Voice FIE 31.2.1.3 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CF_ACT_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CF_ACT_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CF_ACT_V_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x20, 0x82, 0x01, 0x68

- ENDARRAY

/* result activate CF Voice FIE 31.2.1.3 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CF_ACT_RES)

SET_COMP ("l_fac", 0x00E8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CF_ACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CF_ACT_RES_CONTENT, 29)

0xA2, 0x80, 0x02, 0x01, 0x00, 0x30, 0x14, 0x02, 0x01, 0x0C, 0xA0, 0x80, 0x04, 0x01, 0x20, 0x30, 	0x08, 0x30, 0x06, 0x82, 0x01, 0x68, 0x84, 0x01, 0x07, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* activate CFU Voice FIE 31.2.1.3 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFU_ACT_V)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFU_ACT_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFU_ACT_V_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x21

- ENDARRAY

/* result activate CFU Voice FIE 31.2.1.3 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFU_ACT_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFU_ACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFU_ACT_RES_CONTENT, 26)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x80, 0x02, 0x01, 0x0C, 0xA0, 0x80, 0x04, 0x01, 0x21, 0x30, 	0x05, 0x30, 0x03, 0x84, 0x01, 0x07, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* deactivate CFC Voice FIE 31.2.1.4 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFC_DEACT_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFC_DEACT_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFC_DEACT_V_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x28, 0x83, 0x01, 0x10

- ENDARRAY

/* result deactivate CFC Voice FIE 31.2.1.4 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFC_DEACT_RES)

SET_COMP ("l_fac", 0x00E8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFC_DEACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFC_DEACT_RES_CONTENT, 29)

0xA2, 0x1B, 0x02, 0x01, 0x00, 0x30, 0x16, 0x02, 0x01, 0x0D, 0xA0, 0x80, 0x04, 0x01, 0x28, 0x30, 	0x80, 0x30, 0x06, 0x83, 0x01, 0x11, 0x84, 0x01, 0x06, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* deactivate CFNRC Voice FIE 31.2.1.4 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRC_DEACT_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRC_DEACT_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRC_DEACT_V_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x2B, 0x83, 0x01, 0x60

- ENDARRAY

/* result deactivate CFNRC Voice FIE 31.2.1.4 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRC_DEACT_RES)

SET_COMP ("l_fac", 0x00D8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRC_DEACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRC_DEACT_RES_CONTENT, 27)

0xA2, 0x19, 0x02, 0x01, 0x00, 0x30, 0x14, 0x02, 0x01, 0x0D, 0xA0, 0x0F, 0x04, 0x01, 0x2B, 0x30, 	0x0A, 0x30, 0x80, 0x83, 0x01, 0x60, 0x84, 0x01, 0x06, 0x00, 0x00

- ENDARRAY

/* interrogate CFB Voice FIE 31.2.1.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFB_IRGT_V)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFB_IRGT_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFB_IRGT_V_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x29

- ENDARRAY

/* result interrogate CFB Voice FIE 31.2.1.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFB_IRGT_RES)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFB_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFB_IRGT_RES_CONTENT, 13)

0xA2, 0x0B, /* Return Result Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x30, 0x06, /* Sequence Tag, Length */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

/* begin of parameters */

0x80, 0x01, 0x04 /* ss-Status Tag, Length, Value (provisioned, not active) */

- ENDARRAY

/* result interrogate CFB FIE, 2nd version  */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFB_IRGT_RES2)

SET_COMP ("l_fac", 0x01F0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFB_IRGT_RES2_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFB_IRGT_RES2_CONTENT, 62)

0xA2, 0x3C,

0x02, 0x01, 0x00,

0x30, 0x37,

0x02, 0x01, 0x0E,

0xA3, 0x32,

0x30, 0x06,

0x83, 0x01, 0x80, /* TeleService Tag, Length, Value (all fax and speech transmission services) */

0x84, 0x01, 0x07, /* provisioned, active, registered */

/* note no forwarded to number */

0x30, 0x0B,

0x83, 0x01, 0x60, /* TeleService Tag, Length, Value (all fax transmission services) */

0x84, 0x01, 0x07, /* provisioned, active, registered */

0x85, 0x03, 0xA1, 0x65, 0x87, /* forwarded to number */

0x30, 0x06,

0x82, 0x01, 0x60, /* BearerService Tag, Length, Value (all async transmission services) */

0x84, 0x01, 0x06, /* */

0x30, 0x0B,

0x82, 0x01, 0x50, /* BearerService Tag, Length, Value (all data async transmission services) */

0x84, 0x01, 0x07, /* provisioned, active, registered */

0x85, 0x03, 0xA1, 0x65, 0x87, /* forwarded to number */

0x30, 0x0B,

0x82, 0x01, 0x20, /* BearerService Tag, Length, Value (all data PAD services transmission services) */

0x84, 0x01, 0x07 /* provisioned, active, registered */

/* note no forwarded to number */

- ENDARRAY

/* interrogate CFNRY Voice FIE 31.2.1.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRY_IRGT_V)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRY_IRGT_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRY_IRGT_V_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x2A, 0x83, 0x01, 0x10

- ENDARRAY

/* result interrogate CFNRY Voice FIE 31.2.1.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRY_IRGT_RES)

SET_COMP ("l_fac", 0x00D0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRY_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRY_IRGT_RES_CONTENT, 26)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0E, 0xA3, 0x0E, 0x30, 0x0C, 0x83, 0x01, 	0x11, 0x84, 0x01, 0x0F, 0x85, 0x04, 0x91, 0x34, 0x21, 0x43

- ENDARRAY

/* interrogate CFNRC Voice FIE 31.2.1.6.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRC_IRGT_V)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRC_IRGT_V_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRC_IRGT_V_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x2B

- ENDARRAY

/* result interrogate CFNRC Voice FIE 31.2.1.6.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFNRC_IRGT_RES)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFNRC_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFNRC_IRGT_RES_CONTENT, 8)

0xA3, 0x06, 0x02, 0x01, 0x00, 0x02, 0x01, 0x12

- ENDARRAY

/* interrogate CFB Fax FIE 31.2.1.6.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFB_IRGT_F)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFB_IRGT_F_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFB_IRGT_F_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x06, 0x04, 0x01, 0x29, 0x83, 0x01, 0x60

- ENDARRAY

/* interrogate CFB Fax FIE 31.2.1.6.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CFB_IRGT_F_RES)

SET_COMP ("l_fac", 0x0050)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CFB_IRGT_F_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CFB_IRGT_F_RES_CONTENT, 10)

0xA4, 0x80, 0x02, 0x01, 0x00, 0x81, 0x01, 0x03, 0x00, 0x00

- ENDARRAY

/* password CB all  FIE 31.8.1.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_ALLCB_PWD)

SET_COMP ("l_fac", 0x0058)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_ALLCB_PWD_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_ALLCB_PWD_CONTENT, 11)

0xA1, 0x09, 0x02, 0x01, 0x00, 0x02, 0x01, 0x11,  0x04, 0x01, 0x90

- ENDARRAY

/* activate BAOC  FIE 31.8.3.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BAOC_ACT)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BAOC_ACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BAOC_ACT_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x06, 0x04, 0x01, 0x92, 0x82, 0x01, 0x68

- ENDARRAY

/* result activate BAOC  FIE 31.8.3.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BAOC_ACT_RES)

SET_COMP ("l_fac", 0x00E8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BAOC_ACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BAOC_ACT_RES_CONTENT, 29)

0xA2, 0x1B, 0x02, 0x01, 0x00, 0x30, 0x80, 0x02, 0x01, 0x0C, 0xA1, 0x80, 0x04, 0x01, 0x92, 0x30, 	0x08, 0x30, 0x06, 0x82, 0x01, 0x68, 0x84, 0x01, 0x07, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* activate BICR  FIE 31.8.3.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BICR_ACT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BICR_ACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BICR_ACT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x9B

- ENDARRAY

/* result activate BICR  FIE 31.8.3.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BICR_ACT_RES)

SET_COMP ("l_fac", 0x00F0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BICR_ACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BICR_ACT_RES_CONTENT, 26)

0xA2, 0x18, 0x02, 0x01, 0x00, 0x30, 0x13, 0x02, 0x01, 0x0C, 0xA1, 0x0E, 0x04, 0x01, 0x9B, 0x30, 	0x80, 0x30, 0x80, 0x84, 0x01, 0x07, 0x00, 0x00, 0x00, 0x00

- ENDARRAY

/* activate BOIC  FIE 31.8.3.2.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BOIC_ACT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BOIC_ACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BOIC_ACT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x93

- ENDARRAY

/* result activate BOIC  FIE 31.8.3.2.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BOIC_ACT_RES)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BOIC_ACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BOIC_ACT_RES_CONTENT, 8)

0xA3, 0x06, 0x02, 0x01, 0x00, 0x02, 0x01, 0x13

- ENDARRAY

/* activate BAIC  FIE 31.8.3.2.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BAIC_ACT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BAIC_ACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BAIC_ACT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x9A

- ENDARRAY

/* result activate BAIC  FIE 31.8.3.2.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BAIC_ACT_RES)

SET_COMP ("l_fac", 0x0050)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BAIC_ACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BAIC_ACT_RES_CONTENT, 10)

0xA3, 0x80, 0x02, 0x01, 0x00, 0x02, 0x01, 0x26, 0x00, 0x00

- ENDARRAY

/* deactivate all CB  FIE 31.8.4.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_ALLCB_DEACT)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_ALLCB_DEACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_ALLCB_DEACT_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x90, 0x83, 0x01, 0x10

- ENDARRAY

/* result deactivate all CB  FIE 31.8.4.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_ALLCB_DEACT_RES)

SET_COMP ("l_fac", 0x00A8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_ALLCB_DEACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_ALLCB_DEACT_RES_CONTENT, 21)

0xA2, 0x80, 0x02, 0x01, 0x00, 0x30, 0x0C, 0x02, 0x01, 0x0D, 0xA1, 0x07, 0x30, 0x05, 0x30, 0x03, 	0x83, 0x01, 0x11, 0x00, 0x00

- ENDARRAY

/* deactivate all CB out  FIE 31.8.4.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_ALLOUT_DEACT)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_ALLOUT_DEACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_ALLOUT_DEACT_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x91, 0x83, 0x01, 0x60

- ENDARRAY

/* result deactivate all CB out  FIE 31.8.4.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_ALLOUT_DEACT_RES)

SET_COMP ("l_fac", 0x00A8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_ALLOUT_DEACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_ALLOUT_DEACT_RES_CONTENT, 21)

0xA2, 0x80, 0x02, 0x01, 0x00, 0x30, 0x0C, 0x02, 0x01, 0x0D, 0xA1, 0x07, 0x30, 0x05, 0x30, 0x03, 	0x83, 0x01, 0x60, 0x00, 0x00

- ENDARRAY

/* deactivate all CB in  FIE 31.8.4.2.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_ALLIN_DEACT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_ALLIN_DEACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_ALLIN_DEACT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x03, 0x04, 0x01, 0x99

- ENDARRAY

/* result deactivate all CB in  FIE 31.8.4.2.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_ALLIN_DEACT_RES)

SET_COMP ("l_fac", 0x0050)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_ALLIN_DEACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_ALLIN_DEACT_RES_CONTENT, 10)

0xA3, 0x80, 0x02, 0x01, 0x00, 0x02, 0x01, 0x13, 0x00, 0x00

- ENDARRAY

/* deactivate BOIX ex Home  FIE 31.8.4.2.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BOICxHC_DEACT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BOICxHC_DEACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BOICxHC_DEACT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x03, 0x04, 0x01, 0x94

- ENDARRAY

/* result deactivate BOIX ex Home  FIE 31.8.4.2.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BOICxHC_DEACT_RES)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BOICxHC_DEACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BOICxHC_DEACT_RES_CONTENT, 8)

0xA3, 0x06, 0x02, 0x01, 0x00, 0x02, 0x01, 0x26

- ENDARRAY

/* interrogate BAIC  FIE 31.8.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BAIC_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BAIC_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BAIC_IRGT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x9A

- ENDARRAY

/* result A interrogate BAIC  FIE 31.8.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BAIC_IRGT_RES_A)

SET_COMP ("l_fac", 0x0078)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BAIC_IRGT_RES_A_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BAIC_IRGT_RES_A_CONTENT, 15)

0xA2, 0x0D, /* Return Result Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x30, 0x08, /* Sequence Tag, Length */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

/* begin of parameters */

0xA2, 0x03, /* basicServiceGroupList */

0x83, 0x01, 0x11 /* TeleService Tag, Length, Value (telephony) */

- ENDARRAY

/* result B interrogate BAIC  FIE 31.8.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BAIC_IRGT_RES_B)

SET_COMP ("l_fac", 21 * 8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BAIC_IRGT_RES_B_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BAIC_IRGT_RES_B_CONTENT, 21)

0xA2, 0x13, /* Return Result Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x30, 0x0E, /* Sequence Tag, Length */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

0xA2, 0x80, /* basicServiceGroupList */

0x83, 0x01, 0x10, /* TeleService Tag, Length, Value (all speech) */

0x82, 0x01, 0x60, /* BearerService Tag, Length, Value (all asynchronous services) */

0x83, 0x01, 0x20 /* TeleService Tag, Length, Value (all SMS services) */

ENDARRAY

/* result C interrogate BAIC  FIE 31.8.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BAIC_IRGT_RES_C)

SET_COMP ("l_fac", 21 * 8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BAIC_IRGT_RES_C_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BAIC_IRGT_RES_C_CONTENT, 21)

0xA2, 0x13, /* Return Result Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x30, 0x0E, /* Sequence Tag, Length */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

0xA2, 0x80, /* basicServiceGroupList */

0x83, 0x01, 0x10, /* TeleService Tag, Length, Value (all speech) */

0x83, 0x01, 0x60, /* TeleService Tag, Length, Value (all fax) */

0x82, 0x01, 0x10  /* BearerService Tag, Length, Value (all data CDA services) */

- ENDARRAY

/* interrogate BOIC ex Home  FIE 31.8.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BOICxHC_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BOICxHC_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BOICxHC_IRGT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x94

- ENDARRAY

/* result interrogate BOIC ex Home  FIE 31.8.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BOICxHC_IRGT_RES)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BOICxHC_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BOICxHC_IRGT_RES_CONTENT, 13)

0xA2, 0x0B, 0x02, 0x01, 0x00, 0x30, 0x06, 0x02, 0x01, 0x0E,

0x80,  0x01, 0x04 /* ss-Status: provisioned */

- ENDARRAY

/* interrogate BICR  FIE 31.8.6.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BICR_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BICR_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BICR_IRGT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x9B

- ENDARRAY

/* result interrogate BICR  FIE 31.8.6.2 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BICR_IRGT_RES)

SET_COMP ("l_fac", 0x0050)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BICR_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BICR_IRGT_RES_CONTENT, 10)

0xA3, 0x80, 0x02, 0x01, 0x00, 0x02, 0x01, 0x12, 0x00, 0x00

- ENDARRAY

/* interrogate BOIC  FIE 31.8.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BOIC_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BOIC_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BOIC_IRGT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x93

- ENDARRAY

/* result interrogate BOIC  FIE 31.8.6.1 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BOIC_IRGT_RES)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BOIC_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BOIC_IRGT_RES_CONTENT, 8)

0xA4, 0x06, 0x02, 0x01, 0x00, 0x81, 0x01, 0x03

- ENDARRAY

/* process USSD  FIE 31.9 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_USSD_PROC)

SET_COMP ("l_fac", 0x00A0)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_USSD_PROC_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_USSD_PROC_CONTENT, 20)

0xA1, 0x12, 0x02, 0x01, 0x00, 0x02, 0x01, 0x3B, 0x30, 0x0A, 0x04, 0x01, 0x0F, 0x04, 0x05, 0x2A, 	0x15, 0x0C, 0x36, 0x02

- ENDARRAY

/* result process USSD  FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_USSD_PROC_RES)

SET_COMP ("l_fac", 0x00B8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_USSD_PROC_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_USSD_PROC_RES_CONTENT, 23)

0xA2, 0x15, /* Return Result Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x30, 0x10, /* Sequence Tag, Length */

0x02, 0x01, 0x3B, /* Operation Code Tag (ProcessUnstructuredSS-Request), Length, Value */

/* begin of parameters */

0x30, 0x0B, /* Sequence Tag, Length */

0x04, 0x01, 0x0F, /* ussd-DataCodingScheme (default, unspecified language) */

0x04, 0x06, 0xAA, 0x51, 0x0C, 0x06, 0x1B, 0x01 /* ussd-String */

- ENDARRAY

/* deactivate BAOC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BAOC_DEACT)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BAOC_DEACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BAOC_DEACT_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0D, 0x30, 0x06, 0x04, 0x01, 0x92, 0x83, 0x01, 0x00

- ENDARRAY

/* result deactivate BAOC FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_BAOC_DEACT_RES)

SET_COMP ("l_fac", 0x00C8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_BAOC_DEACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_BAOC_DEACT_RES_CONTENT, 25)

0xA2, 0x17, 0x02, 0x01, 0x01, 0x30, 0x12, 0x02, 0x01, 0x0D, 0xA1, 0x0D, 0x04, 0x01, 0x92, 0x30, 	0x08, 0x30, 0x06, 0x83, 0x01, 0x00, 0x84, 0x01, 0x01

- ENDARRAY

/* interrogate CLIR FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CLIR_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CLIR_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CLIR_IRGT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0E, 0x30, 0x03, 0x04, 0x01, 0x12

- ENDARRAY

/* result interrogate CLIR FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CLIR_IRGT_RES)

SET_COMP ("l_fac", 0x0090)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CLIR_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CLIR_IRGT_RES_CONTENT, 18)

0xA2, 0x10,

0x02, 0x01, 0x01,

0x30, 0x0B,

0x02, 0x01, 0x0E,

0xA4, 0x06, /* genericServiceInfo Tag, length */

0x04, 0x01, 0x05, /* ss-Status Tag, length, value (Provisioned,Active) */

0x0A, 0x01, 0x02 /* cliRestrictionOption Tag, Length, Value (temporaryDefaultAllowed) */

- ENDARRAY

/* activate CCWA all FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CW_ACT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CW_ACT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CW_ACT_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x0C, 0x30, 0x03, 0x04, 0x01, 0x41

- ENDARRAY

/* activate result CCWA voice FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CW_ACT_RES)

SET_COMP ("l_fac", 0x0090)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CW_ACT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CW_ACT_RES_CONTENT, 18)

0xA2, 0x10, 0x02, 0x01, 0x00, 0x30, 0x0B, 0x02, 0x01, 0x0C, 0xA3, 0x06, 0x04, 0x01, 0x41, 0x84, 	0x01, 0x05

- ENDARRAY

/* activate result CCWA voice FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CW_ACT_RES2)

SET_COMP ("l_fac", 0x0050)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CW_ACT_RES2_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CW_ACT_RES2_CONTENT, 10)

0xA2, 0x08, 0x02, 0x01, 0x00, 0x30, 0x03, 0x02, 0x01, 0x0C

- ENDARRAY

/* activate result CCWA voice FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CCBS_ERS_RES)

SET_COMP ("l_fac", 0x0090)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CCBS_ERS_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CCBS_ERS_RES_CONTENT, 18)

0xA2, 0x10, 0x02, 0x01, 0x00, 0x30, 0x0B, 0x02, 0x01, 0x4D, 0x30, 0x06, 0x80, 0x01, 0x43, 0x81, 	0x01, 0x04

- ENDARRAY

/* erase CCBS entry 1 FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CCBS_ERS_1)

SET_COMP ("l_fac", 0x0080)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CCBS_ERS_1_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CCBS_ERS_1_CONTENT, 16)

0xA1, 0x0E, 0x02, 0x01, 0x00, 0x02, 0x01, 0x4D, 0x30, 0x06, 0x80, 0x01, 0x43, 0x81, 0x01, 0x01

- ENDARRAY

/* erase CCBS all entries FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CCBS_ERS_ALL)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CCBS_ERS_ALL_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CCBS_ERS_ALL_CONTENT, 13)

0xA1, 0x0B, 0x02, 0x01, 0x00, 0x02, 0x01, 0x4D, 0x30, 0x03, 0x80, 0x01, 0x43

- ENDARRAY

/* interrogate CCBS entry FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CCBS_IRGT)

SET_COMP ("l_fac", 0x0068)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CCBS_IRGT_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CCBS_IRGT_CONTENT, 13)

0xA1, 0x0B, /* Invoke Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

0x30, 0x03, /* Sequence Tag, Length */

0x04, 0x01, 0x43 /* SS Code Tag (Octet String Tag), Length, Value (CCBS, originating side) */

- ENDARRAY

/* interrogate result CCBS entry FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CCBS_IRGT_RES)

SET_COMP ("l_fac", 0x0250)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CCBS_IRGT_RES_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CCBS_IRGT_RES_CONTENT, 74)

0xA2, 0x48, /* Return Result Component Tag, Length */

0x02, 0x01, 0x00, /* Invoke ID Tag, Length, Value */

0x30, 0x43, /* Sequence Tag, Length */

0x02, 0x01, 0x0E, /* Operation Code Tag (InterrogateSS), Length, Value */

/* begin of parameters */

0xA4, 0x3E, /* genericServiceInfo Tag, Length */

0x04, 0x01, 0x07, /* ss-Status: provisioned, registered, active */

0xA2, 0x39, /* ccbs-FeatureList */

0x30, 0x11, /* Sequence Tag, Length, entry #1 */

0x80, 0x01, 0x01, /* ccbs-Index 1 */

0x81, 0x07, 0x91, 0x94, 0x03, 0x93, 0x90, 0x14, 0x32, /* b-subscriber number */

0xA3, 0x03, /* Basic Service Group Tag, Length */

0x83, 0x01, 0x10, /* TeleService Tag, Length, Value (all speech) */

0x30, 0x11, /* Sequence Tag, Length, entry #2 */

0x80, 0x01, 0x02, /* ccbs-Index 2 */

0x81, 0x07, 0x91, 0x94, 0x03, 0x93, 0x90, 0x44, 0x65, /* b-subscriber number */

0xA3, 0x03, /* Basic Service Group Tag, Length */

0x83, 0x01, 0x60, /* TeleService Tag, Length, Value (all fax) */

0x30, 0x11, /* Sequence Tag, Length, entry #3 */

0x80, 0x01, 0x04, /* ccbs-Index 4 */

0x81, 0x07, 0x91, 0x94, 0x03, 0x93, 0x90, 0x74, 0x98, /* b-subscriber number */

0xA3, 0x03, /* Basic Service Group Tag, Length */

0x82, 0x01, 0x10 /* BearerService Tag, Length, Value (all data CDA services) */

- ENDARRAY

/* interrogate error CCBS entry FIE */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_CCBS_IRGT_ERR)

SET_COMP ("l_fac", 0x0040)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_CCBS_IRGT_ERR_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_CCBS_IRGT_ERR_CONTENT, 8)

0xA3, 0x06, 0x02, 0x01, 0x00, 0x02, 0x01, 0x10

- ENDARRAY

/* return result FIE, empty except for invoke ID being 0 */

BEGIN_PSTRUCT ("fac_inf", A_FAC_KSD_EMPTY_RES_INV_ID0)

SET_COMP ("l_fac", 5 * 8)

SET_COMP ("o_fac", 0x0000)

SET_COMP ("fac", A_FAC_KSD_EMPTY_RES_INV_ID0_CONTENT)

- ENDSTRUCT

BEGINARRAY_PART (A_FAC_KSD_EMPTY_RES_INV_ID0_CONTENT, 5)

0xA2, 0x03, /* Return Result Component Tag, Length */

0x02, 0x01, 0x00 /* Invoke ID Tag, Length, Value */

- ENDARRAY

/*----------------------------------------  structs ----------------------------------------------------- */

/* forward advice of charge info */

BEGIN_PSTRUCT ("fac_inf", S_FAC_AOC)
	SET_COMP ("l_fac",	LA_FAC_AOC)
	SET_COMP ("o_fac",	NUM_0)
	SET_COMP ("fac",	A_FAC_AOC)
ENDSTRUCT

/* bearer service not present */
 BEGIN_PSTRUCT ("bcpara", S_BS_NOT_PRESENT)
	SET_COMP ("rate",	UR_NOT_PRES)
	SET_COMP ("bearer_serv",	BEARER_SERV_NOT_PRES)
	SET_COMP ("conn_elem",	CONN_ELEM_NOT_PRES)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)
	SKIP_COMP ("modem_type")
ENDSTRUCT

/* bearer service voice */
 BEGIN_PSTRUCT ("bcpara", S_BS_VOICE)
	SET_COMP ("rate",	UR_NOT_PRES)
	SET_COMP ("bearer_serv",	BEARER_SERV_SPEECH)
	SET_COMP ("conn_elem",	CONN_ELEM_NOT_PRES)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)
	SKIP_COMP ("modem_type")
ENDSTRUCT

BEGIN_PSTRUCT ("bcpara", S_BS_V)
	SET_COMP ("rate",	UR_NOT_PRES)
	SET_COMP ("bearer_serv",	BEARER_SERV_SPEECH)
	SET_COMP ("conn_elem",	CONN_ELEM_NOT_PRES)
	SET_COMP ("stop_bits",	STOP_1_BIT)
	SET_COMP ("data_bits",	DATA_8_BIT)
	SET_COMP ("parity",	PARITY_NONE)
	SET_COMP ("flow_control",	NO_FLOW_CONTROL)
	SKIP_COMP ("modem_type")
ENDSTRUCT

/* called party address national modified*/
 BEGIN_PSTRUCT ("called_party", S_CLD_PARTY_MDFY)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num",	LA_CLD_NUM_MDFY)
	SET_COMP ("called_num",	A_CLD_NUM_MDFY)
ENDSTRUCT

/* called party address international*/
 BEGIN_PSTRUCT ("called_party", S_CLD_PARTY_INT)
	SET_COMP ("ton",	TON_INT_NUMB)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num",	LA_CLD_NUM_INT)
	SET_COMP ("called_num",	A_CLD_NUM_INT)
ENDSTRUCT

/* called party address unknown type of number*/
 BEGIN_PSTRUCT ("called_party", S_CLD_PARTY_NINT)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num",	LA_CLD_NUM_NINT)
	SET_COMP ("called_num",	A_CLD_NUM_NINT)
ENDSTRUCT

/* called party address emergency*/
 BEGIN_PSTRUCT ("called_party", S_CLD_PARTY_ECC)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num",	LA_CLD_NUM_ECC)
	SET_COMP ("called_num",	A_CLD_NUM_ECC)
ENDSTRUCT

/* calling party sub address modified */
 BEGIN_PSTRUCT ("called_party_sub", S_CLD_PARTY_SUB_MDFY)
	SET_COMP ("tos",	TOS_NSAP)
	SET_COMP ("odd_even",	OE_ODD)
	SET_COMP ("c_subaddr",	LA_CLD_SUB_MDFY)
	SET_COMP ("subaddr",	A_CLD_SUB_MDFY)
ENDSTRUCT

/* called party address national*/
 BEGIN_PSTRUCT ("called_party", S_CLD_PARTY)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("c_called_num",	LA_CLD_NUM)
	SET_COMP ("called_num",	A_CLD_NUM)
ENDSTRUCT

/* called party sub address */
 BEGIN_PSTRUCT ("called_party_sub", S_CLD_PARTY_SUB)
	SET_COMP ("tos",	TOS_NOT_PRES)
	SET_COMP ("odd_even",	OE_EVEN)
	SET_COMP ("c_subaddr",	NUM_0)
	SKIP_COMP ("subaddr")
ENDSTRUCT

/* data full rate 9600 */
 BEGIN_PSTRUCT ("chm", S_CHN_FULL_9600)
	SET_COMP ("ch_type",	CH_TCH_F)
	SET_COMP ("ch_mode",	CHM_DATA_9_6)
ENDSTRUCT

/* bearer service not present */
/* calling party address */
 BEGIN_PSTRUCT ("calling_party", S_CLG_PARTY)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_ISDN_TEL_NUMB_PLAN)
	SET_COMP ("present",	PRES_PRES_ALLOW)
	SET_COMP ("screen",	SCREEN_IND_NOT_PRES)
	SET_COMP ("c_num",	LA_CLG_NUM)
	SET_COMP ("num",	A_CLG_NUM)
ENDSTRUCT

/* calling party sub address */
 BEGIN_PSTRUCT ("calling_party_sub", S_CLG_PARTY_SUB)
	SET_COMP ("tos",	TOS_NOT_PRES)
	SET_COMP ("odd_even",	OE_EVEN)
	SET_COMP ("c_subaddr",	NUM_0)
	SKIP_COMP ("subaddr")
ENDSTRUCT

/* called party address national*/
/* called party address national*/
 BEGIN_PSTRUCT ("called_party", S_CLD_PARTY_2)
	SET_COMP ("ton",	TON_UNKNOWN)
	SET_COMP ("npi",	NPI_UNKNOW)
	SET_COMP ("c_called_num",	LA_CLD_NUM)
	SET_COMP ("called_num",	A_CLD_NUM)
ENDSTRUCT

# TEST CASES

## Routing (internal) (ACI001 – ACI010)

### ACISS001:  Setup the Routing and the PCO view for the ACI test

Description:

Routings for the ACI tests are set.

Preamble:

None

APL		ACI		PS

|		|		|

COMMAND (TAP RESET)
COMMAND (CC RESET)

COMMAND (MM RESET)
COMMAND (SIM RESET)
COMMAND (SS RESET)
COMMAND (MMI RESET)
COMMAND (SMS RESET)
COMMAND (RA RESET)
COMMAND (T30 RESET)
	 |		|		|
COMMAND (TAP REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)
COMMAND (RA REDIRECT CLEAR)
COMMAND (T30 REDIRECT CLEAR)
	 |		|		|
COMMAND (MMI REDIRECT CC TAP)
COMMAND (MMI REDIRECT MM TAP)
COMMAND (MMI REDIRECT SIM TAP)
COMMAND (MMI REDIRECT SS TAP)
COMMAND (MMI REDIRECT MMI TAP)
COMMAND (MMI REDIRECT SMS TAP)
COMMAND (MMI REDIRECT RA TAP)
COMMAND (MMI REDIRECT T30 TAP)
	|		|		|

COMMAND (TAP REDIRECT TAP MMI)
COMMAND (MMI REDIRECT MMI TAP)
	|		|		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

History:	14.12.98	AK	Initial

## Line identification supplementary services (ACI011 – ACI099)

### ACISS011:  Interrogate CLIP status

Description:

interrogate network for CLIP status.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CLIP?)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	NUM_ELEMENTS(C_PLUS_CLIP_QUERY)
	cmd_seq	C_PLUS_CLIP_QUERY

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CLIP_IRGT
	ss_ver	NOT_USED

History:	10.08.98	ACI	Initial

### ACISS012:  Interrogate CLIR status

Description:

interrogate network for CLIR status.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CLIR?)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLIR_QUERY
	cmd_seq	C_PLUS_CLIR_QUERY

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CLIR_IRGT
	ss_ver	A_SS_VER_2

History:	10.08.98	ACI	Initial

### ACISS013:  Interrogate COLP status

Description:

interrogate network for COLP status.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +COLP?)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_COLP_QUERY
	cmd_seq	C_PLUS_COLP_QUERY

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_COLP_IRGT
	ss_ver	A_SS_VER_2

History:	10.08.98	ACI	Initial

### ACISS020:  Successful interrogation of CLIP, provisioned

Description:

network returns current status of CLIP. CLIP is provisioned

Preamble:

- ACISS011

APL		ACI		PS

|		|		|

(1)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(2)	|	ACI_CMD_IND	|		|

|	(msg: +CLIP: n,m)	|		|

*	<==========================	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CLIP_IRGT_RES_PROV

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLIP_PROV
	cmd_seq	M_PLUS_CLIP_PROV

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS021:  Successful interrogation of CLIR

Description:

network returns current status of CLIR.

Variant A: CLIR is provisioned, temporary mode presentation allowed

Variant B: CLIR is not provisioned

Preamble:

- ACISS012

Variants:      <A>....<B>

APL		ACI		PS

|		|		|

(1)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(2)	|	ACI_CMD_IND	|		|

|	(msg: +CLIR: n,m)	|		|

*	<==========================	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CLIR_IRGT_RES_PVTMAL
<B>	fac_inf	A_FAC_CLIR_IRGT_RES_PVTMAL2

- ACI_CMD_IND
<A>	cmd_len	LM_PLUS_CLIR_PVTMAL
<B>	cmd_len	LM_PLUS_CLIR_PVTMAL2
<A>	cmd_seq	M_PLUS_CLIR_PVTMAL
<B>	cmd_seq	M_PLUS_CLIR_PVTMAL2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

## Call Waiting (ACI040-ACI059)

### ACISS040:  Interrogate Call waiting status

Description:

interrogate network for CW status.

Preamble:

- ACISS001

Variants:      <A>....<B>

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CCWA=,2)	|		|
	*	==========================>	*		|

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: +CCWA: n,m)	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: +CCWA: n,m)	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: +CCWA: n,m)	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CCWA_QUERY
<B>	cmd_len	LC_PLUS_CCWA_QUERY2
<A>	cmd_seq	C_PLUS_CCWA_QUERY
<B>	cmd_seq	C_PLUS_CCWA_QUERY2

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CCWA_IRGT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CCWA_IRGT_RES
<B>	fac_inf	A_FAC_CCWA_IRGT_RES2

- ACI_CMD_IND
	cmd_len	LM_PLUS_CCWA_V
	cmd_seq	M_PLUS_CCWA_V

- ACI_CMD_IND
	cmd_len	LM_PLUS_CCWA_D
	cmd_seq	M_PLUS_CCWA_D

- ACI_CMD_IND
	cmd_len	LM_PLUS_CCWA_F
	cmd_seq	M_PLUS_CCWA_F

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial
	04.01.02	SBK	Update due to ACI-FIX-1666
	25.04.02	TLU	Variant B added

### ACISS041:  Activate Call waiting status

Description:

activate CW supplementary service.

Preamble:

- ACISS001

Variants:      <A>....<C>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CCWA=,1,x)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CCWA_ACT_V
<B>	cmd_len	LC_PLUS_CCWA_ACT_D
<C>	cmd_len	LC_PLUS_CCWA_ACT_V
<A>	cmd_seq	C_PLUS_CCWA_ACT_V
<B>	cmd_seq	C_PLUS_CCWA_ACT_D
<C>	cmd_seq	C_PLUS_CCWA_ACT_V

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CCWA_ACT_V
<B>	fac_inf	A_FAC_CCWA_ACT_D
<C>	fac_inf	A_FAC_CCWA_ACT_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CCWA_ACT_V_RES
<B>	fac_inf	A_FAC_CCWA_ACT_D_RES
<C>	fac_inf	A_FAC_CCWA_ACT_V_RES2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:		10.08.98	ACI	Initial

### ACISS042:  Deactivate Call Waiting Voice/Data/Fax

Description:

Deactivate Call Waiting Voice/Data/Fax: ok case

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CCWA=x,0)	|		|
	*	==========================>	*		|

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CCWA_DEACT
	cmd_seq	C_PLUS_CCWA_DEACT

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CCWA_DEACT_VF
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CCWA_DEACT_VF_RES

- MNSS_BEGIN_REQ
	ti	NUM_1
	fac_inf	A_FAC_CCWA_DEACT_D
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_1
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CCWA_DEACT_D_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	06.05.02	ACI	Initial

### ACISS043:  Deactivate Call Waiting Voice/Data/Fax: error case

Description:

Deactivate Call Waiting Voice/Data/Fax: Error case

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CCWA=x,0)	|		|
	*	==========================>	*		|

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: Error)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CCWA_DEACT
	cmd_seq	C_PLUS_CCWA_DEACT

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CCWA_DEACT_VF
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CCWA_DEACT_VF_ERR_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	06.05.02	ACI	Initial

## Call Forwarding (ACI060-ACI079)

### ACISS060:  Interrogate Call Forwarding status

Description:

interrogate network for CF status.

Preamble:

- ACISS001

Variants:      <A>....<D>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CCFC=x,2)	|		|
	*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CCFC_CFU_QUERY
<B>	cmd_len	LC_PLUS_CCFC_CFB_QUERY
<C>	cmd_len	LC_PLUS_CCFC_CFNRY_QUERY
<D>	cmd_len	LC_PLUS_CCFC_CFNRC_QUERY
<A>	cmd_seq	C_PLUS_CCFC_CFU_QUERY
<B>	cmd_seq	C_PLUS_CCFC_CFB_QUERY
<C>	cmd_seq	C_PLUS_CCFC_CFNRY_QUERY
<D>	cmd_seq	C_PLUS_CCFC_CFNRC_QUERY

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CCFC_CFU_IRGT
<B>	fac_inf	A_FAC_CCFC_CFB_IRGT
<C>	fac_inf	A_FAC_CCFC_CFNRY_IRGT
<D>	fac_inf	A_FAC_CCFC_CFNRC_IRGT
	ss_ver	NOT_USED

History:	18.05.99	ACI	Initial

04.02.01	SBK	Adapted due to ACI-FIX-1666

### ACISS061:  Register Call Forwarding Voice/Data/Fax

Description:

Register call forwarding SS for voice, data and fax.

Preamble:

- ACISS001

Variants:      <A>....<F>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CCFC=x,3)	|		|
	*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|
	|		*	==========================>	*
	|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CCFC_CFU_REG
<B>	cmd_len	LC_PLUS_CCFC_CFB_REG
<C>	cmd_len	LC_PLUS_CCFC_CFNRY_REG
<D>	cmd_len	LC_PLUS_CCFC_CFNRC_REG
<E>	cmd_len	LC_PLUS_CCFC_ALLCF_REG
<F>	cmd_len	LC_PLUS_CCFC_ALLCFC_REG
<A>	cmd_seq	C_PLUS_CCFC_CFU_REG
<B>	cmd_seq	C_PLUS_CCFC_CFB_REG
<C>	cmd_seq	C_PLUS_CCFC_CFNRY_REG
<D>	cmd_seq	C_PLUS_CCFC_CFNRC_REG
<E>	cmd_seq	C_PLUS_CCFC_ALLCF_REG
<F>	cmd_seq	C_PLUS_CCFC_ALLCFC_REG

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CCFC_CFU_REG_VF
<B>	fac_inf	A_FAC_CCFC_CFB_REG_VF
<C>	fac_inf	A_FAC_CCFC_CFNRY_REG_VF
<D>	fac_inf	A_FAC_CCFC_CFNRC_REG_VF
<E>	fac_inf	A_FAC_CCFC_ALLCF_REG_VF
<F>	fac_inf	A_FAC_CCFC_ALLCFC_REG_VF
	ss_ver	NOT_USED

History:	18.05.99	ACI	Initial

### ACISS062:  Erase Call Forwarding Voice/Data/Fax

Description:

Erase call forwarding SS for voice, data and fax.

Preamble:

- ACISS001

Variants:      <A>....<F>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CCFC=x,4)	|		|
	*	==========================>	*		|
(2)	|		| 	MNSS_BEGIN_REQ	|
	|		*	==========================>	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CCFC_CFU_ERS
<B>	cmd_len	LC_PLUS_CCFC_CFB_ERS
<C>	cmd_len	LC_PLUS_CCFC_CFNRY_ERS
<D>	cmd_len	LC_PLUS_CCFC_CFNRC_ERS
<E>	cmd_len	LC_PLUS_CCFC_ALLCF_ERS
<F>	cmd_len	LC_PLUS_CCFC_ALLCFC_ERS
<A>	cmd_seq	C_PLUS_CCFC_CFU_ERS
<B>	cmd_seq	C_PLUS_CCFC_CFB_ERS
<C>	cmd_seq	C_PLUS_CCFC_CFNRY_ERS
<D>	cmd_seq	C_PLUS_CCFC_CFNRC_ERS
<E>	cmd_seq	C_PLUS_CCFC_ALLCF_ERS
<F>	cmd_seq	C_PLUS_CCFC_ALLCFC_ERS

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CCFC_CFU_ERS_VF
<B>	fac_inf	A_FAC_CCFC_CFB_ERS_VF
<C>	fac_inf	A_FAC_CCFC_CFNRY_ERS_VF
<D>	fac_inf	A_FAC_CCFC_CFNRC_ERS_VF
<E>	fac_inf	A_FAC_CCFC_ALLCF_ERS_VF
<F>	fac_inf	A_FAC_CCFC_ALLCFC_ERS_VF
	ss_ver	NOT_USED

History:	18.05.99	ACI	Initial

### ACISS063:  Activate Call Forwarding Voice/Data/Fax

Description:

Activate call forwarding SS for voice, data and fax.

Preamble:

- ACISS001

Variants:      <A>....<F>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CCFC=x,1)	|		|
	*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|
	|		*	==========================>	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CCFC_CFU_ACT
<B>	cmd_len	LC_PLUS_CCFC_CFB_ACT
<C>	cmd_len	LC_PLUS_CCFC_CFNRY_ACT
<D>	cmd_len	LC_PLUS_CCFC_CFNRC_ACT
<E>	cmd_len	LC_PLUS_CCFC_ALLCF_ACT
<F>	cmd_len	LC_PLUS_CCFC_ALLCFC_ACT
<A>	cmd_seq	C_PLUS_CCFC_CFU_ACT
<B>	cmd_seq	C_PLUS_CCFC_CFB_ACT
<C>	cmd_seq	C_PLUS_CCFC_CFNRY_ACT
<D>	cmd_seq	C_PLUS_CCFC_CFNRC_ACT
<E>	cmd_seq	C_PLUS_CCFC_ALLCF_ACT
<F>	cmd_seq	C_PLUS_CCFC_ALLCFC_ACT

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CCFC_CFU_ACT_VF
<B>	fac_inf	A_FAC_CCFC_CFB_ACT_VF
<C>	fac_inf	A_FAC_CCFC_CFNRY_ACT_VF
<D>	fac_inf	A_FAC_CCFC_CFNRC_ACT_VF
<E>	fac_inf	A_FAC_CCFC_ALLCF_ACT_VF
<F>	fac_inf	A_FAC_CCFC_ALLCFC_ACT_VF
	ss_ver	NOT_USED

History:	18.05.99	ACI	Initial

### ACISS064:  Deactivate Call Forwarding Voice/Data/Fax

Description:

Deactivate call forwarding SS for voice, data and fax.

Preamble:

- ACISS001

Variants:      <A>....<F>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CCFC=x,0)	|		|
	*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|
	|		*	==========================>	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CCFC_CFU_DEACT
<B>	cmd_len	LC_PLUS_CCFC_CFB_DEACT
<C>	cmd_len	LC_PLUS_CCFC_CFNRY_DEACT
<D>	cmd_len	LC_PLUS_CCFC_CFNRC_DEACT
<E>	cmd_len	LC_PLUS_CCFC_ALLCF_DEACT
<F>	cmd_len	LC_PLUS_CCFC_ALLCFC_DEACT
<A>	cmd_seq	C_PLUS_CCFC_CFU_DEACT
<B>	cmd_seq	C_PLUS_CCFC_CFB_DEACT
<C>	cmd_seq	C_PLUS_CCFC_CFNRY_DEACT
<D>	cmd_seq	C_PLUS_CCFC_CFNRC_DEACT
<E>	cmd_seq	C_PLUS_CCFC_ALLCF_DEACT
<F>	cmd_seq	C_PLUS_CCFC_ALLCFC_DEACT

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CCFC_CFU_DEACT_VF
<B>	fac_inf	A_FAC_CCFC_CFB_DEACT_VF
<C>	fac_inf	A_FAC_CCFC_CFNRY_DEACT_VF
<D>	fac_inf	A_FAC_CCFC_CFNRC_DEACT_VF
<E>	fac_inf	A_FAC_CCFC_ALLCF_DEACT_VF
<F>	fac_inf	A_FAC_CCFC_ALLCFC_DEACT_VF
	ss_ver	NOT_USED

History:	18.05.99	ACI	Initial

### ACISS065:  Successful interrogation of Call Forwarding CFU

Description:

network returns current status of call forwarding SS

Preamble:

- ACISS060A

APL		ACI		PS

|		|		|

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: +CCFC:n,m)	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CCFC_CFU_IRGT_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CCFC_CFU
	cmd_seq	M_PLUS_CCFC_CFU

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

04.02.01	SBK	Adapted due to ACI-FIX-1666

### ACISS066:  Successful interrogation of Call Forwarding CFB

Description:

network returns current status of call forwarding SS

Preamble:

- ACISS060B

APL		ACI		PS

|		|		|

- |		| 	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: +CCFC:n,m)	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: +CCFC:n,m)	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CCFC_CFB_IRGT_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CCFC_CFB_V
	cmd_seq	M_PLUS_CCFC_CFB_V

- ACI_CMD_IND
	cmd_len	LM_PLUS_CCFC_CFB_D
	cmd_seq	M_PLUS_CCFC_CFB_D

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

04.02.01		       SBK	                               Adapted due to ACI-FIX-1666

### ACISS067:  Successful interrogation of Call Forwarding CFNRY

Description:

network returns current status of call forwarding SS

Preamble:

- ACISS060C

APL		ACI		PS

|		|		|

- |		| 	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: +CCFC:n,m)	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: +CCFC:n,m)	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CCFC_CFNRY_IRGT_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CCFC_CFNRY_V
	cmd_seq	M_PLUS_CCFC_CFNRY_V

- ACI_CMD_IND
	cmd_len	LM_PLUS_CCFC_CFNRY_F
	cmd_seq	M_PLUS_CCFC_CFNRY_F

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

04.02.01		       SBK	                               Adapted due to ACI-FIX-1666

### ACISS068:  Successful interrogation of Call Forwarding

Description:

network returns current status of call forwarding SS

Preamble:

- ACISS060D

APL		ACI		PS

|		|		|

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: +CCFC:n,m)	|		|
	*	<==========================	*		|

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CCFC_CFNRC_IRGT_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CCFC_CFNRC
	cmd_seq	M_PLUS_CCFC_CFNRC

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial
	04.02.01	SBK	Adapted due to ACI-FIX-1666

## SS Notification (ACI080-ACI099)

### ACISS080:  SS Notify

Description:

Receive SS notify during call setup.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CSSN=1,1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: D03039094444;)	|		|

*	==========================>	*		|

(4)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(5)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|		| 	MNCC_FACILITY_IND	|

|		*	<==========================	*

(8)	|	ACI_CMD_IND	|		|

|	(msg: +CSSI)	|		|

*	<==========================	*		|

(9)	|	ACI_CMD_IND	|		|

|	(msg: +CSSI)	|		|

*	<==========================	*		|

(10)	|	ACI_CMD_IND	|		|

|	(msg: +CSSU)	|		|

*	<==========================	*		|

(11)	|	ACI_CMD_IND	|		|

|	(msg: +CSSU)	|		|

*	<==========================	*		|

(12)	|	ACI_CMD_IND	|		|

|	(msg: +CSSI)	|		|

*	<==========================	*		|

(13)	|	ACI_CMD_IND	|		|

|	(msg: +CSSI)	|		|

*	<==========================	*		|

(14)	|	ACI_CMD_IND	|		|

|	(msg: +CSSU)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CSSN_ON
	cmd_seq	C_PLUS_CSSN_ON

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_VOICE
	cmd_seq	C_D_VOICE

- MNCC_SETUP_REQ
	ti	NUM_0
	prio	PRIO_NORM_CALL
	ri	RI_NOT_PRES
	bcpara	S_BS_VOICE
	bcpara2	S_BS_NOT_PRESENT
	called_party	S_CLD_PARTY
	called_party_sub	S_CLD_PARTY_SUB
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNCC_FACILITY_IND
	ti	NUM_0
	fac_context	NOT_USED
	fac_inf	A_FAC_NTFY_SS_1

- ACI_CMD_IND
	cmd_len	LM_PLUS_CSSI_CFU
	cmd_seq	M_PLUS_CSSI_CFU

- ACI_CMD_IND
	cmd_len	LM_PLUS_CSSI_CW
	cmd_seq	M_PLUS_CSSI_CW

- ACI_CMD_IND
	cmd_len	LM_PLUS_CSSU_HLD
	cmd_seq	M_PLUS_CSSU_HLD

- ACI_CMD_IND
	cmd_len	LM_PLUS_CSSU_MPTY
	cmd_seq	M_PLUS_CSSU_MPTY

- ACI_CMD_IND
	cmd_len	LM_PLUS_CSSI_CUG
	cmd_seq	M_PLUS_CSSI_CUG

- ACI_CMD_IND
	cmd_len	LM_PLUS_CSSI_CLIR
	cmd_seq	M_PLUS_CSSI_CLIR

- ACI_CMD_IND
	cmd_len	LM_PLUS_CSSU_ECT_ALRT
	cmd_seq	M_PLUS_CSSU_ECT_ALRT

History:	18.05.99	ACI	Initial

### ACISS081:  Forward Check SS Indication

Description:

Receive forward check SS indication during call setup.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CSSN=1,1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: D03039094444;)	|		|

*	==========================>	*		|

(4)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(5)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|		| 	MNCC_FACILITY_IND	|

|		*	<==========================	*

(8)	|	ACI_CMD_IND	|		|

|	(msg: +CSSI)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CSSN_ON
	cmd_seq	C_PLUS_CSSN_ON

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_VOICE
	cmd_seq	C_D_VOICE

- MNCC_SETUP_REQ
	ti	NUM_0
	prio	PRIO_NORM_CALL
	ri	RI_NOT_PRES
	bcpara	S_BS_VOICE
	bcpara2	S_BS_NOT_PRESENT
	called_party	S_CLD_PARTY
	called_party_sub	S_CLD_PARTY_SUB
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNCC_FACILITY_IND
	ti	NUM_0
	fac_context	NOT_USED
	fac_inf	A_FAC_CHECK_SS

- ACI_CMD_IND
	cmd_len	LM_PLUS_CSSU_CHK
	cmd_seq	M_PLUS_CSSU_CHK

History:	18.05.99	ACI	Initial

### ACISS082:  SS Notify for ECT and override case for Redirection Number (RDN)

Description:

Receive Notify SS indication for ECT and the redirection number being present as Presentation Restricted. This is the override case, i.e. when the redirected-to-party belongs to a special override authority allowing to override / overrule the wish of the redirecting-party for being anonymous .

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CSSN=1,1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: D03039094444;)	|		|

*	==========================>	*		|

(4)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

(5)	|		| 	SIM_SYNC_REQ	|

|		*	==========================>	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(7)	|		| 	MNCC_FACILITY_IND	|

|		*	<==========================	*

(8)	|	ACI_CMD_IND	|		|

|	(msg: +CSSU)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CSSN_ON
	cmd_seq	C_PLUS_CSSN_ON

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_VOICE
	cmd_seq	C_D_VOICE

- MNCC_SETUP_REQ
	ti	NUM_0
	prio	PRIO_NORM_CALL
	ri	RI_NOT_PRES
	bcpara	S_BS_V
	bcpara2	S_BS_NOT_PRESENT
	called_party	S_CLD_PARTY
	called_party_sub	S_CLD_PARTY_SUB
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	NOT_USED

- SIM_SYNC_REQ
	synccs	SYNC_START_CALL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNCC_FACILITY_IND
	ti	NUM_0
	fac_context	NOT_USED
	fac_inf	A_FAC_NTFY_SS_ECT_RSTR

- ACI_CMD_IND
	cmd_len	LM_PLUS_CSSU_ECT_ALRT
	cmd_seq	M_PLUS_CSSU_ECT_ALRT

History:	25.07.01	SBK	Initial, due to (G)ACI-FIX-1434

### ACISS083:  SS Notify: MT call FTA 31.2.1.7.1.1

Description:

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CSSN=1,1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|		| 	MNCC_SETUP_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: RING)	|		|

*	<==========================	*		|

(5)	|		| 	MNCC_ALERT_REQ	|

|		*	==========================>	*

(6)	|		| 	MNCC_FACILITY_IND	|

|		*	<==========================	*

(7)	|	ACI_CMD_IND	|		|

|	(msg: +CSSU)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CSSN_ON
	cmd_seq	C_PLUS_CSSN_ON

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNCC_SETUP_IND
	ti	NUM_0
	ri	RI_NOT_PRES
	bcpara	S_BS_VOICE
	bcpara2	S_BS_NOT_PRESENT
	progress_desc	PROG_NOT_PRES
	sig	SIG_RING_BACK_TONE_ON
	calling_party	S_CLG_PARTY
	calling_party_sub	S_CLG_PARTY_SUB
	called_party	S_CLD_PARTY
	called_party_sub	S_CLD_PARTY_SUB
	redirecting_party	NOT_USED
	redirecting_party_sub	NOT_USED

- ACI_CMD_IND
	cmd_len	NUM_ELEMENTS(M_RING)
	cmd_seq	M_RING

- MNCC_ALERT_REQ
	ti	NUM_0

- MNCC_FACILITY_IND
	ti	NUM_0
	fac_context	NOT_USED
	fac_inf	A_FAC_NTFY_SS_FD

- ACI_CMD_IND
	cmd_len	NUM_ELEMENTS(M_PLUS_CSSU_FWD)
	cmd_seq	M_PLUS_CSSU_FWD

History:	27.02.03	CLB	Initial, due to FTA fails 31.2.1.7.1.1

07.04.03	SKA	M_PLUS_CSSU_FWD added

## Closed User Group (ACI0100-ACI109)

### ACISS100:  CUG with temporary mode enabled                        FAILS: to be analyzed

Description:

Enable CUG temporary mode. Send CUG SS during call setup.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CCUG=1,x,x)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: D03039094444)	|		|

*	==========================>	*		|

(4)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CCUG_ON
	cmd_seq	C_PLUS_CCUG_ON

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_VOICE
	cmd_seq	C_D_VOICE

- MNCC_SETUP_REQ
	ti	NUM_0
	prio	PRIO_NORM_CALL
	ri	RI_NOT_PRES
	bcpara	S_BS_VOICE
	bcpara2	S_BS_NOT_PRESENT
	called_party	S_CLD_PARTY
	called_party_sub	S_CLD_PARTY_SUB
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	A_FAC_CUG_SS_1

History:	21.05.99	ACI	Initial

### ACISS101:  CUG with default parameters                        FAILS: to be analyzed

Description:

Establish a call using CUG default values. Send CUG SS during call setup.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: D03039094444G;)	|		|

*	==========================>	*		|

(2)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_VOICE_CUG
	cmd_seq	C_D_VOICE_CUG

- MNCC_SETUP_REQ
	ti	NUM_0
	prio	PRIO_NORM_CALL
	ri	RI_NOT_PRES
	bcpara	S_BS_VOICE
	bcpara2	S_BS_NOT_PRESENT
	called_party	S_CLD_PARTY
	called_party_sub	S_CLD_PARTY_SUB
	clir_sup	NOT_PRESENT_8BIT
	fac_inf	A_FAC_CUG_SS_2

History:	21.05.99	ACI	Initial

## SS Password (ACI0110-ACI119)

### ACISS110:  Change Password, successful attempt

Description:

Change the password for all call barring services. Command is successful

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPWD="AB",x,y)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(4)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(5)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(6)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(7)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(8)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(9)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(10)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CPWD_CBALL
	cmd_seq	C_PLUS_CPWD_CBALL

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CPWD_ALLCB_REG
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_NEW_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_NEW_PWD_RES
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_NEWAGN_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_NEWAGN_PWD_RES
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CPWD_ALLCB_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	25.05.99	ACI	Initial

### ACISS111:  Change Password, subscription violation

Description:

Change the password for all call barring services. Failure due to subscription violation.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPWD="AB",x,y)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CPWD_CBALL
	cmd_seq	C_PLUS_CPWD_CBALL

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CPWD_ALLCB_REG
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CPWD_ALLCB_ERR_1

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	25.05.99	ACI	Initial

### ACISS112:  Change Password, negative password check

Description:

Change the password for all call barring services. Password check fails.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPWD="AB",x,y)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(4)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(5)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CPWD_CBALL
	cmd_seq	C_PLUS_CPWD_CBALL

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CPWD_ALLCB_REG
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CPWD_ALLCB_ERR_2

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	25.05.99	ACI	Initial

### ACISS113:  Change Password, new password mismatch

Description:

Change the password for all call barring services. failed due to new password mismatch.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPWD="AB",x,y)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(4)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(5)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(6)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(7)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(8)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(9)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(10)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CPWD_CBALL
	cmd_seq	C_PLUS_CPWD_CBALL

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CPWD_ALLCB_REG
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_NEW_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_NEW_PWD_RES
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_NEWAGN_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_NEWAGN_PWD_RES
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CPWD_ALLCB_ERR_3

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	25.05.99	ACI	Initial

## Call Barring (ACI120-ACI139)

### ACISS120:  Interrogate Call Barring status

Description:

interrogate network for CB status.

Preamble:

- ACISS001

Variants:      <A>....<E>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|
	|	(cmd: +CLCK=x,2)	|		|
	*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|
	|		*	==========================>	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CLCK_BAOC_QUERY
<B>	cmd_len	LC_PLUS_CLCK_BOIC_QUERY
<C>	cmd_len	LC_PLUS_CLCK_BOICxHC_QUERY
<D>	cmd_len	LC_PLUS_CLCK_BAIC_QUERY
<E>	cmd_len	LC_PLUS_CLCK_BICR_QUERY
<A>	cmd_seq	C_PLUS_CLCK_BAOC_QUERY
<B>	cmd_seq	C_PLUS_CLCK_BOIC_QUERY
<C>	cmd_seq	C_PLUS_CLCK_BOICxHC_QUERY
<D>	cmd_seq	C_PLUS_CLCK_BAIC_QUERY
<E>	cmd_seq	C_PLUS_CLCK_BICR_QUERY

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CLCK_BAOC_IRGT
<B>	fac_inf	A_FAC_CLCK_BOIC_IRGT
<C>	fac_inf	A_FAC_CLCK_BOICxHC_IRGT
<D>	fac_inf	A_FAC_CLCK_BAIC_IRGT
<E>	fac_inf	A_FAC_CLCK_BICR_IRGT
	ss_ver	NOT_USED

History:	18.05.99	ACI	Initial

### ACISS121:  Activate Call Forwarding Voice/Data/Fax

Description:

Activate call forwarding SS for voice, data and fax.

Preamble:

- ACISS001

Variants:      <A>....<E>

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CLCK=x,0,...)	|		|
	*	==========================>	*		|

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_FACILITY_IND	|
	|		*	<==========================	*

- |		|	MNSS_FACILITY_REQ	|
	|		*	==========================>	*

- |		|	MNSS_FACILITY_IND	|
	|		*	<==========================	*

- |		|	MNSS_FACILITY_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CLCK_BAOC_ACT
<B>	cmd_len	LC_PLUS_CLCK_BOIC_ACT
<C>	cmd_len	LC_PLUS_CLCK_BOICxHC_ACT
<D>	cmd_len	LC_PLUS_CLCK_BAIC_ACT
<E>	cmd_len	LC_PLUS_CLCK_BICR_ACT
<A>	cmd_seq	C_PLUS_CLCK_BAOC_ACT
<B>	cmd_seq	C_PLUS_CLCK_BOIC_ACT
<C>	cmd_seq	C_PLUS_CLCK_BOICxHC_ACT
<D>	cmd_seq	C_PLUS_CLCK_BAIC_ACT
<E>	cmd_seq	C_PLUS_CLCK_BICR_ACT

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CLCK_BAOC_ACT_VF
<B>	fac_inf	A_FAC_CLCK_BOIC_ACT_VF
<C>	fac_inf	A_FAC_CLCK_BOICxHC_ACT_VF
<D>	fac_inf	A_FAC_CLCK_BAIC_ACT_VF
<E>	fac_inf	A_FAC_CLCK_BICR_ACT_VF
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_1
	fac_inf	A_FAC_ENTER_PWD_REQ_2

- MNSS_FACILITY_REQ
	ti	NUM_1
	fac_inf	A_FAC_ENTER_PWD_RES_2
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CLCK_BAOC_ACT_VF_RES
<B>	fac_inf	A_FAC_CLCK_BOIC_ACT_VF_RES
<C>	fac_inf	A_FAC_CLCK_BOICxHC_ACT_VF_RES
<D>	fac_inf	A_FAC_CLCK_BAIC_ACT_VF_RES
<E>	fac_inf	A_FAC_CLCK_BICR_ACT_VF_RES

- MNSS_BEGIN_REQ
	ti	NUM_1
<A>	fac_inf	A_FAC_CLCK_BAOC_ACT_D
<B>	fac_inf	A_FAC_CLCK_BOIC_ACT_D
<C>	fac_inf	A_FAC_CLCK_BOICxHC_ACT_D
<D>	fac_inf	A_FAC_CLCK_BAIC_ACT_D
<E>	fac_inf	A_FAC_CLCK_BICR_ACT_D
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_1
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CLCK_BAOC_ACT_D_RES
<B>	fac_inf	A_FAC_CLCK_BOIC_ACT_D_RES
<C>	fac_inf	A_FAC_CLCK_BOICxHC_ACT_D_RES
<D>	fac_inf	A_FAC_CLCK_BAIC_ACT_D_RES
<E>	fac_inf	A_FAC_CLCK_BICR_ACT_D_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	18.05.99	ACI	Initial

### ACISS122:  Deactivate Call Barring Voice/Data/Fax

Description:

Deactivate call barring SS for voice, data and fax.

Preamble:

- ACISS001

Variants:      <A>....<H>

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CLCK=x,1,...)	|		|
	*	==========================>	*		|

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_FACILITY_IND	|
	|		*	<==========================	*

- |		|	MNSS_FACILITY_REQ	|
	|		*	==========================>	*

- |		|	MNSS_FACILITY_IND	|
	|		*	<==========================	*

- |		|	MNSS_FACILITY_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |		| 	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CLCK_ALLCB_DEACT
<B>	cmd_len	LC_PLUS_CLCK_ALLOUT_DEACT
<C>	cmd_len	LC_PLUS_CLCK_ALLIN_DEACT
<D>	cmd_len	LC_PLUS_CLCK_BAOC_DEACT
<E>	cmd_len	LC_PLUS_CLCK_BOIC_DEACT
<F>	cmd_len	LC_PLUS_CLCK_BOICxHC_DEACT
<G>	cmd_len	LC_PLUS_CLCK_BAIC_DEACT
<H>	cmd_len	LC_PLUS_CLCK_BICR_DEACT
<A>	cmd_seq	C_PLUS_CLCK_ALLCB_DEACT
<B>	cmd_seq	C_PLUS_CLCK_ALLOUT_DEACT
<C>	cmd_seq	C_PLUS_CLCK_ALLIN_DEACT
<D>	cmd_seq	C_PLUS_CLCK_BAOC_DEACT
<E>	cmd_seq	C_PLUS_CLCK_BOIC_DEACT
<F>	cmd_seq	C_PLUS_CLCK_BOICxHC_DEACT
<G>	cmd_seq	C_PLUS_CLCK_BAIC_DEACT
<H>	cmd_seq	C_PLUS_CLCK_BICR_DEACT

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CLCK_ALLCB_DEACT_VF
<B>	fac_inf	A_FAC_CLCK_ALLOUT_DEACT_VF
<C>	fac_inf	A_FAC_CLCK_ALLIN_DEACT_VF
<D>	fac_inf	A_FAC_CLCK_BAOC_DEACT_VF
<E>	fac_inf	A_FAC_CLCK_BOIC_DEACT_VF
<F>	fac_inf	A_FAC_CLCK_BOICxHC_DEACT_VF
<G>	fac_inf	A_FAC_CLCK_BAIC_DEACT_VF
<H>	fac_inf	A_FAC_CLCK_BICR_DEACT_VF
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_1
	fac_inf	A_FAC_ENTER_PWD_REQ_2

- MNSS_FACILITY_REQ
	ti	NUM_1
	fac_inf	A_FAC_ENTER_PWD_RES_2
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CLCK_ALLCB_DEACT_VF_RES
<B>	fac_inf	A_FAC_CLCK_ALLOUT_DEACT_VF_RES
<C>	fac_inf	A_FAC_CLCK_ALLIN_DEACT_VF_RES
<D>	fac_inf	A_FAC_CLCK_BAOC_DEACT_VF_RES
<E>	fac_inf	A_FAC_CLCK_BOIC_DEACT_VF_RES
<F>	fac_inf	A_FAC_CLCK_BOICxHC_DACT_VF_RES
<G>	fac_inf	A_FAC_CLCK_BAIC_DEACT_VF_RES
<H>	fac_inf	A_FAC_CLCK_BICR_DEACT_VF_RES

- MNSS_BEGIN_REQ
	ti	NUM_1
<A>	fac_inf	A_FAC_CLCK_ALLCB_DEACT_D
<B>	fac_inf	A_FAC_CLCK_ALLOUT_DEACT_D
<C>	fac_inf	A_FAC_CLCK_ALLIN_DEACT_D
<D>	fac_inf	A_FAC_CLCK_BAOC_DEACT_D
<E>	fac_inf	A_FAC_CLCK_BOIC_DEACT_D
<F>	fac_inf	A_FAC_CLCK_BOICxHC_DEACT_D
<G>	fac_inf	A_FAC_CLCK_BAIC_DEACT_D
<H>	fac_inf	A_FAC_CLCK_BICR_DEACT_D
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_1
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CLCK_ALLCB_DEACT_D_RES
<B>	fac_inf	A_FAC_CLCK_ALLOUT_DEACT_D_RES
<C>	fac_inf	A_FAC_CLCK_ALLIN_DEACT_D_RES
<D>	fac_inf	A_FAC_CLCK_BAOC_DEACT_D_RES
<E>	fac_inf	A_FAC_CLCK_BOIC_DEACT_D_RES
<F>	fac_inf	A_FAC_CLCK_BOICxHC_DACT_D_RES
<G>	fac_inf	A_FAC_CLCK_BAIC_DEACT_D_RES
<H>	fac_inf	A_FAC_CLCK_BICR_DEACT_D_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- History:	18.05.99	ACI	Initial

### ACISS123:  Activate Call Forwarding Voice/Data/Fax/SMS

Description:

Activate call forwarding SS for voice, data, fax and SMS.

Preamble:

- ACISS001

Variants:      <A>....<E>

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CLCK=x,0,...)	|		|
	*	==========================>	*		|

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_FACILITY_IND	|
	|		*	<==========================	*

- |		|	MNSS_FACILITY_REQ	|
	|		*	==========================>	*

- |		|	MNSS_FACILITY_IND	|
	|		*	<==========================	*

- |		|	MNSS_FACILITY_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CLCK_BAOC_ALL_ACT
<B>	cmd_len	LC_PLUS_CLCK_BOIC_ALL_ACT
<C>	cmd_len	LC_PLUS_CLCK_BOICxHC_ALL_ACT
<D>	cmd_len	LC_PLUS_CLCK_BAIC_ALL_ACT
<E>	cmd_len	LC_PLUS_CLCK_BICR_ALL_ACT
<A>	cmd_seq	C_PLUS_CLCK_BAOC_ALL_ACT
<B>	cmd_seq	C_PLUS_CLCK_BOIC_ALL_ACT
<C>	cmd_seq	C_PLUS_CLCK_BOICxHC_ALL_ACT
<D>	cmd_seq	C_PLUS_CLCK_BAIC_ALL_ACT
<E>	cmd_seq	C_PLUS_CLCK_BICR_ALL_ACT

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CLCK_BAOC_ALL_ACT_VF
<B>	fac_inf	A_FAC_CLCK_BOIC_ALL_ACT_VF
<C>	fac_inf	A_FAC_CLCK_BOICxHC_ALL_ACT_VF
<D>	fac_inf	A_FAC_CLCK_BAIC_ALL_ACT_VF
<E>	fac_inf	A_FAC_CLCK_BICR_ALL_ACT_VF
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_1
	fac_inf	A_FAC_ENTER_PWD_REQ_2

- MNSS_FACILITY_REQ
	ti	NUM_1
	fac_inf	A_FAC_ENTER_PWD_RES_2
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CLCK_BAOC_ALL_ACT_VF_RES
<B>	fac_inf	A_FAC_CLCK_BOIC_ALL_ACT_VF_RES
<C>	fac_inf	A_FAC_CLCK_BOICxHC_ALL_ACT_VF_RES
<D>	fac_inf	A_FAC_CLCK_BAIC_ALL_ACT_VF_RES
<E>	fac_inf	A_FAC_CLCK_BICR_ALL_ACT_VF_RES

- MNSS_BEGIN_REQ
	ti	NUM_1
<A>	fac_inf	A_FAC_CLCK_BAOC_ACT_D
<B>	fac_inf	A_FAC_CLCK_BOIC_ACT_D
<C>	fac_inf	A_FAC_CLCK_BOICxHC_ACT_D
<D>	fac_inf	A_FAC_CLCK_BAIC_ACT_D
<E>	fac_inf	A_FAC_CLCK_BICR_ACT_D
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_1
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CLCK_BAOC_ACT_D_RES
<B>	fac_inf	A_FAC_CLCK_BOIC_ACT_D_RES
<C>	fac_inf	A_FAC_CLCK_BOICxHC_ACT_D_RES
<D>	fac_inf	A_FAC_CLCK_BAIC_ACT_D_RES
<E>	fac_inf	A_FAC_CLCK_BICR_ACT_D_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	16.01.02	KGT	Initial

### ACISS124:  Deactivate Call Barring Voice/Data/Fax/SMS

Description:

Deactivate call barring SS for voice, data and fax.

Preamble:

- ACISS001

Variants:      <A>....<E>

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CLCK=x,1,...)	|		|
	*	==========================>	*		|

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_FACILITY_IND	|
	|		*	<==========================	*

- |		|	MNSS_FACILITY_REQ	|
	|		*	==========================>	*

- |		|	MNSS_FACILITY_IND	|
	|		*	<==========================	*

- |		|	MNSS_FACILITY_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |		| 	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CLCK_BAOC_ALL_DEACT
<B>	cmd_len	LC_PLUS_CLCK_BOIC_ALL_DEACT
<C>	cmd_len	LC_PLUS_CLCK_BOICxHC_ALL_DEACT
<D>	cmd_len	LC_PLUS_CLCK_BAIC_ALL_DEACT
<E>	cmd_len	LC_PLUS_CLCK_BICR_ALL_DEACT
<A>	cmd_seq	C_PLUS_CLCK_BAOC_ALL_DEACT
<B>	cmd_seq	C_PLUS_CLCK_BOIC_ALL_DEACT
<C>	cmd_seq	C_PLUS_CLCK_BOICxHC_ALL_DEACT
<D>	cmd_seq	C_PLUS_CLCK_BAIC_ALL_DEACT
<E>	cmd_seq	C_PLUS_CLCK_BICR_ALL_DEACT

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CLCK_BAOC_ALL_DEACT_VF
<B>	fac_inf	A_FAC_CLCK_BOIC_ALL_DEACT_VF
<C>	fac_inf	A_FAC_CLCK_BOICxHC_ALL_DEACT_VF
<D>	fac_inf	A_FAC_CLCK_BAIC_ALL_DEACT_VF
<E>	fac_inf	A_FAC_CLCK_BICR_ALL_DEACT_VF
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_1
	fac_inf	A_FAC_ENTER_PWD_REQ_2

- MNSS_FACILITY_REQ
	ti	NUM_1
	fac_inf	A_FAC_ENTER_PWD_RES_2
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CLCK_BAOC_ALL_DEACT_VF_RES
<B>	fac_inf	A_FAC_CLCK_BOIC_ALL_DEACT_VF_RES
<C>	fac_inf	A_FAC_CLCK_BOICxHC_DACT_VF_RES
<D>	fac_inf	A_FAC_CLCK_BAIC_ALL_DEACT_VF_RES
<E>	fac_inf	A_FAC_CLCK_BICR_ALL_DEACT_VF_RES

- MNSS_BEGIN_REQ
	ti	NUM_1
<A>	fac_inf	A_FAC_CLCK_BAOC_DEACT_D
<B>	fac_inf	A_FAC_CLCK_BOIC_DEACT_D
<C>	fac_inf	A_FAC_CLCK_BOICxHC_DEACT_D
<D>	fac_inf	A_FAC_CLCK_BAIC_DEACT_D
<E>	fac_inf	A_FAC_CLCK_BICR_DEACT_D
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_1
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CLCK_BAOC_DEACT_D_RES
<B>	fac_inf	A_FAC_CLCK_BOIC_DEACT_D_RES
<C>	fac_inf	A_FAC_CLCK_BOICxHC_DACT_D_RES
<D>	fac_inf	A_FAC_CLCK_BAIC_DEACT_D_RES
<E>	fac_inf	A_FAC_CLCK_BICR_DEACT_D_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- History:	21.01.02	KGT	Initial

### ACISS125:  Activate Call Barring no class entered

Description:

Activate call barring SS for no class entered.

Preamble:

- ACISS001

Variants:      <A>....<E>

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CLCK=x,0,...)	|		|
	*	==========================>	*		|

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_FACILITY_IND	|
	|		*	<==========================	*

- |		|	MNSS_FACILITY_REQ	|
	|		*	==========================>	*

- |		|	MNSS_FACILITY_IND	|
	|		*	<==========================	*

- |		|	MNSS_FACILITY_REQ	|
	|		*	==========================>	*

- |		|	MNSS_END_IND	|
	|		*	<==========================	*

- |	ACI_CMD_IND	|		|
	|	(msg: OK)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PLUS_CLCK_BAOC_DEF_ACT
<B>	cmd_len	LC_PLUS_CLCK_BOIC_DEF_ACT
<C>	cmd_len	LC_PLUS_CLCK_BOICxHC_DEF_ACT
<D>	cmd_len	LC_PLUS_CLCK_BAIC_DEF_ACT
<E>	cmd_len	LC_PLUS_CLCK_BICR_DEF_ACT
<A>	cmd_seq	C_PLUS_CLCK_BAOC_DEF_ACT
<B>	cmd_seq	C_PLUS_CLCK_BOIC_DEF_ACT
<C>	cmd_seq	C_PLUS_CLCK_BOICxHC_DEF_ACT
<D>	cmd_seq	C_PLUS_CLCK_BAIC_DEF_ACT
<E>	cmd_seq	C_PLUS_CLCK_BICR_DEF_ACT

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CLCK_BAOC_ACT
<B>	fac_inf	A_FAC_CLCK_BOIC_ACT
<C>	fac_inf	A_FAC_CLCK_BOICxHC_ACT
<D>	fac_inf	A_FAC_CLCK_BAIC_ACT
<E>	fac_inf	A_FAC_CLCK_BICR_ACT
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ_2

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES_2
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CLCK_BAOC_ACT_VF_RES
<B>	fac_inf	A_FAC_CLCK_BOIC_ACT_VF_RES
<C>	fac_inf	A_FAC_CLCK_BOICxHC_ACT_VF_RES
<D>	fac_inf	A_FAC_CLCK_BAIC_ACT_VF_RES
<E>	fac_inf	A_FAC_CLCK_BICR_ACT_VF_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	21.01.02	KGT	Initial

## Unstructured SS (ACI0140-ACI149)

### ACISS140:  Unstructured SS notify

Description:

Network initiated unstructured SS notification.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|		| 	MNSS_BEGIN_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CUSD: 0,...)	|		|

*	<==========================	*		|

(5)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_ON
	cmd_seq	C_PLUS_CUSD_ON

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNSS_BEGIN_IND
	ti	NUM_8
	fac_inf	A_FAC_USSD_NTFY

- ACI_CMD_IND
	cmd_len	LM_PLUS_CUSD_NTFY
	cmd_seq	M_PLUS_CUSD_NTFY

- MNSS_FACILITY_REQ
	ti	NUM_8
	fac_inf	A_FAC_USSD_NTFY_RES
	ss_ver	NOT_USED

History:	25.05.99	ACI	Initial

### ACISS141:  Unstructured SS request

Description:

Network initiated unstructured SS request.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|		| 	MNSS_BEGIN_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CUSD: 1,...)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=,...)	|		|

*	==========================>	*		|

(6)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(8)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(9)	|	ACI_CMD_IND	|		|

|	(msg: +CUSD: 1,...)	|		|

*	<==========================	*		|

(10)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=,...)	|		|

*	==========================>	*		|

(11)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(12)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_ON
	cmd_seq	C_PLUS_CUSD_ON

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNSS_BEGIN_IND
	ti	NUM_8
	fac_inf	A_FAC_USSD_REQ

- ACI_CMD_IND
	cmd_len	LM_PLUS_CUSD_REQ
	cmd_seq	M_PLUS_CUSD_REQ

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_SEND
	cmd_seq	C_PLUS_CUSD_SEND

- MNSS_FACILITY_REQ
	ti	NUM_8
	fac_inf	A_FAC_USSD_REQ_RES
	ss_ver	NOT_USED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNSS_FACILITY_IND
	ti	NUM_8
	fac_inf	A_FAC_USSD_REQ

- ACI_CMD_IND
	cmd_len	LM_PLUS_CUSD_REQ
	cmd_seq	M_PLUS_CUSD_REQ

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_SEND
	cmd_seq	C_PLUS_CUSD_SEND

- MNSS_FACILITY_REQ
	ti	NUM_8
	fac_inf	A_FAC_USSD_REQ_RES
	ss_ver	NOT_USED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	25.05.99	ACI	Initial

### ACISS142:  Process Unstructured SS request, no network request involved

Description:

Mobile initiated process unstructured SS request. The network does not request USSD during transaction.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=,...)	|		|

*	==========================>	*		|

(4)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(5)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: +CUSD: 0,...)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_ON
	cmd_seq	C_PLUS_CUSD_ON

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_SEND
	cmd_seq	C_PLUS_CUSD_SEND

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_USSD_PROC
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_USSD_PROC_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CUSD_PROC_RES
	cmd_seq	M_PLUS_CUSD_PROC_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	25.05.99	ACI	Initial

### ACISS143:  Process Unstructured SS request, with network request involved

Description:

Mobile initiated process unstructured SS request. The network does request USSD during transaction.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=,...)	|		|

*	==========================>	*		|

(4)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(5)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: +CUSD: 1,...)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=,...)	|		|

*	==========================>	*		|

(4)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(5)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: +CUSD: 1,...)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=,...)	|		|

*	==========================>	*		|

(4)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(5)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: +CUSD: 0,...)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_ON
	cmd_seq	C_PLUS_CUSD_ON

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_SEND
	cmd_seq	C_PLUS_CUSD_SEND

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_USSD_PROC
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_USSD_REQ

- ACI_CMD_IND
	cmd_len	LM_PLUS_CUSD_REQ
	cmd_seq	M_PLUS_CUSD_REQ

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_SEND
	cmd_seq	C_PLUS_CUSD_SEND

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_USSD_REQ_RES
	ss_ver	NOT_USED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_USSD_REQ

- ACI_CMD_IND
	cmd_len	LM_PLUS_CUSD_REQ
	cmd_seq	M_PLUS_CUSD_REQ

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_SEND
	cmd_seq	C_PLUS_CUSD_SEND

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_USSD_REQ_RES
	ss_ver	NOT_USED

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_USSD_PROC_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CUSD_PROC_RES
	cmd_seq	M_PLUS_CUSD_PROC_RES

History:	25.05.99	ACI	Initial

### ACISS144:  Process Unstructured SS request, network supports only version 1 protocol

Description:

Mobile initiated process unstructured SS request. The network does not support version 2 protocol. Message has to be repeated using version 1 protocol. Successful outcome.

Preamble:

- ACISS001

Variants:	<A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=,...)	|		|

*	==========================>	*		|

(4)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(5)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(6)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(7)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(8)	|	ACI_CMD_IND	|		|

|	(msg: +CUSD: 0,...)	|		|

*	<==========================	*		|

(9)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_ON
	cmd_seq	C_PLUS_CUSD_ON

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_SEND
	cmd_seq	C_PLUS_CUSD_SEND

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_USSD_PROC
	ss_ver	A_SS_VER_2

- MNSS_END_IND
	ti	NUM_0
<A>	cause	MNSS_CAUSE_NO_NET_CAUSE
<B>	cause	MNSS_CAUSE_FACILITY_REJECT
<A>	fac_inf	A_FAC_USSD_PROC_REJ
<B>	fac_inf	NOT_USED

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_USSD_PROC_IA5
	ss_ver	A_SS_VER_1

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_USSD_DAT_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CUSD_DAT_RES
	cmd_seq	M_PLUS_CUSD_DAT_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	25.05.99	ACI	Initial

### ACISS145:  Process Unstructured SS request, network does not support USSD

Description:

Mobile initiated process unstructured SS request. The network does not support any version of the USSD protocol. Unsuccessful outcome.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=1)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CUSD=,...)	|		|

*	==========================>	*		|

(4)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(5)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(6)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(7)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(8)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_ON
	cmd_seq	C_PLUS_CUSD_ON

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CUSD_SEND
	cmd_seq	C_PLUS_CUSD_SEND

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_USSD_PROC
	ss_ver	A_SS_VER_2

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_USSD_PROC_REJ

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_USSD_PROC_IA5
	ss_ver	A_SS_VER_1

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_USSD_PROC_REJ

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	25.05.99	ACI	Initial

### ACISS146:  Unstructured SS control string, network supports only version 1 protocol

Description:

Mobile initiated process unstructured SS request. The network does not support version 2 protocol. Message has to be repeated using version 1 protocol. Successful outcome.

Preamble:

- ACISS001

Variants:	<A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(5)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: +CUSD:...)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_30
	cmd_seq	C_D_KSD_30

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_USSD_PROC_KSD
	ss_ver	A_SS_VER_2

- MNSS_END_IND
	ti	NUM_0
<A>	cause	MNSS_CAUSE_NO_NET_CAUSE
<B>	cause	MNSS_CAUSE_FACILITY_REJECT
<A>	fac_inf	A_FAC_USSD_PROC_REJ
<B>	fac_inf	NOT_USED

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_USSD_PROC_KSD_IA5
	ss_ver	A_SS_VER_1

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_USSD_DAT_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CUSD_ATD_RES
	cmd_seq	M_PLUS_CUSD_ATD_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	25.05.99	ACI	Initial

07.01.02	SBK	Adapted due to ACI-FIX-1666

### ACISS147:  Unstructured SS operation in parallel to other SS operation. Network initiated

Description:

The network initiates a USSD operation while another operation is in progress. MS rejects the USSD operation with cause USSD busy.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

- |	ACI_CMD_REQ	|		|
	|	(cmd: +CCFC=x,4)	|		|
	*	==========================>	*		|

- |		|	MNSS_BEGIN_REQ	|
	|		*	==========================>	*

- |		|	MNSS_BEGIN_IND	|
	|		*	<==========================	*

- |		|	MNSS_END_REQ	|
	|		*	==========================>	*
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CCFC_CFU_ERS
	cmd_seq	C_PLUS_CCFC_CFU_ERS

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CCFC_CFU_ERS_VF
	ss_ver	NOT_USED

- MNSS_BEGIN_IND
	ti	NUM_8
	fac_inf	A_FAC_USSD_NTFY

- MNSS_END_REQ
	ti	NUM_8
	fac_inf	A_FAC_USSD_BUSY_ERR

History:	26.08.99	ACI	Initial

## PIN Modification (ACI0150-ACI169)

### ACISS150:  Change PIN 1

Description:

Change PIN 1.

Preamble:

- ACISS001

Variants:      <A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPWD="SC",...)	|		|

*	==========================>	*		|

(2)	|		| 	SIM_CHANGE_PIN_REQ	|

|		*	==========================>	*

(3)	|		| 	SIM_CHANGE_PIN_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: result)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CPWD_PIN1
	cmd_seq	C_PLUS_CPWD_PIN1

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	F_CUR_PIN
	new_pin	F_NEW_PIN
	pin_id	PHASE_2_PIN_1

- SIM_CHANGE_PIN_CNF
<A>	cause	SIM_NO_ERROR
<B>	cause	SIM_CAUSE_PIN1_REMAIN2
	pin_id	PHASE_2_PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- ACI_CMD_IND
<A>	cmd_len	LM_OK
<B>	cmd_len	LM_ERROR
<A>	cmd_seq	M_OK
<B>	cmd_seq	M_ERROR

History:	03.06.99	ACI	Initial

### ACISS151:  Change PIN 2

Description:

Change PIN 2.

Preamble:

- ACISS001

Variants:      <A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CPWD="P2",...)	|		|

*	==========================>	*		|

(2)	|		| 	SIM_CHANGE_PIN_REQ	|

|		*	==========================>	*

(3)	|		| 	SIM_CHANGE_PIN_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: result)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CPWD_PIN2
	cmd_seq	C_PLUS_CPWD_PIN2

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	F_CUR_PIN
	new_pin	F_NEW_PIN
	pin_id	PHASE_2_PIN_2

- SIM_CHANGE_PIN_CNF
<A>	cause	SIM_NO_ERROR
<B>	cause	SIM_CAUSE_PIN2_REMAIN2
	pin_id	PHASE_2_PIN_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- ACI_CMD_IND
<A>	cmd_len	LM_OK
<B>	cmd_len	LM_ERROR
<A>	cmd_seq	M_OK
<B>	cmd_seq	M_ERROR

History:	03.06.99	ACI	Initial

### ACISS152:  enable PIN 1

Description:

Enable PIN 1.

Preamble:

- ACISS001

Variants:      <A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CLCK="SC",...)	|		|

*	==========================>	*		|

(2)	|		| 	SIM_ENABLE_PIN_REQ	|

|		*	==========================>	*

(3)	|		| 	SIM_ENABLE_PIN_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: result)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLCK_PIN1_ACT
	cmd_seq	C_PLUS_CLCK_PIN1_ACT

- SIM_ENABLE_PIN_REQ
	source	SRC_MMI
	pin	F_CUR_PIN

- SIM_ENABLE_PIN_CNF
<A>	cause	SIM_NO_ERROR
<B>	cause	SIM_CAUSE_PIN1_REMAIN2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- ACI_CMD_IND
<A>	cmd_len	LM_OK
<B>	cmd_len	LM_ERROR
<A>	cmd_seq	M_OK
<B>	cmd_seq	M_ERROR

History:	03.06.99	ACI	Initial

### ACISS153:  disable PIN 1

Description:

Disable PIN 1.

Preamble:

- ACISS001

Variants:      <A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CLCK="SC",...)	|		|

*	==========================>	*		|

(2)	|		| 	SIM_DISABLE_PIN_REQ	|

|		*	==========================>	*

(3)	|		| 	SIM_DISABLE_PIN_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: result)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLCK_PIN1_DEACT
	cmd_seq	C_PLUS_CLCK_PIN1_DEACT

- SIM_DISABLE_PIN_REQ
	source	SRC_MMI
	pin	F_CUR_PIN

- SIM_DISABLE_PIN_CNF
<A>	cause	SIM_NO_ERROR
<B>	cause	SIM_CAUSE_PIN1_REMAIN2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- ACI_CMD_IND
<A>	cmd_len	LM_OK
<B>	cmd_len	LM_ERROR
<A>	cmd_seq	M_OK
<B>	cmd_seq	M_ERROR

History:	03.06.99	ACI	Initial

### ACISS154:  Query Enabled PIN 1

Description:

Query enabled PIN 1.

Preamble:

- ACISS152A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CLCK="SC",2)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CLCK: 1)	|		|

*	<==========================	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLCK_PIN1_QUERY
	cmd_seq	C_PLUS_CLCK_PIN1_QUERY

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLCK_PIN1_ENA
	cmd_seq	M_PLUS_CLCK_PIN1_ENA

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	15.09.99	ACI	Initial

### ACISS155:  Query Disabled PIN 1

Description:

Query disabled PIN 1.

Preamble:

- ACISS153A

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CLCK="SC",2)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CLCK: 0)	|		|

*	<==========================	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLCK_PIN1_QUERY
	cmd_seq	C_PLUS_CLCK_PIN1_QUERY

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLCK_PIN1_DIS
	cmd_seq	M_PLUS_CLCK_PIN1_DIS

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	15.09.99	ACI	Initial

### ACISS156:  Query Unknown PIN 1 Status

Description:

Query unknown PIN 1 status.

Preamble:

- ACISS153B

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: +CMEE=2)	|		|

*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

(3)	|	ACI_CMD_REQ	|		|

|	(cmd: +CLCK="SC",2)	|		|

*	==========================>	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CME:...)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CMEE_VERB
	cmd_seq	C_PLUS_CMEE_VERB

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PLUS_CLCK_PIN1_QUERY
	cmd_seq	C_PLUS_CLCK_PIN1_QUERY

- ACI_CMD_IND
	cmd_len	LM_ERR_SIM_FAIL
	cmd_seq	M_ERR_SIM_FAIL

History:	15.09.99	ACI	Initial

07.01.02	SBK	Expect +CME ERROR: SIM failure

## Keystroke Sequences (ACI170 – ACI249)

### ACISS170:  Register CFNRy 31.2.1.1.1

Description:

Register CFNRy.

Preamble:

- ACISS001

Variants:	<A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_D_KSD_1
<B>	cmd_len	LC_D_KSD_1B
<A>	cmd_seq	C_D_KSD_1
<B>	cmd_seq	C_D_KSD_1B

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_KSD_CFNRY_REG_V
<B>	fac_inf	A_FAC_KSD_CFNRY_REG_VB
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_KSD_CFNRY_REG_RES_A
<B>	fac_inf	A_FAC_KSD_CFNRY_REG_RES_B

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS171:  Register CFU 31.2.1.1.1

Description:

Register CFU.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		|	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		|	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_2
	cmd_seq	C_D_KSD_2

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFU_REG_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFU_REG_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS172:  Register CFU 31.2.1.1.2

Description:

Register CFB.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_3
	cmd_seq	C_D_KSD_3

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFB_REG_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFB_REG_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS173:  Register CF 31.2.1.1.2

Description:

Register CF.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_4
	cmd_seq	C_D_KSD_4

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CF_REG_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CF_REG_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS174:  Erase CFC 31.2.1.2.1

Description:

Erase CFC.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		|	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		|	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_5
	cmd_seq	C_D_KSD_5

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFC_ERS_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFC_ERS_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS175:  Erase CFNRC 31.2.1.2.1

Description:

Erase CFNRC.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_6
	cmd_seq	C_D_KSD_6

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFNRC_ERS_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFNRC_ERS_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS176:  Erase CFU 31.2.1.2.2

Description:

Erase CFU.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_7
	cmd_seq	C_D_KSD_7

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFU_ERS_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFU_ERS_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS177:  Erase CFNRY 31.2.1.2.2

Description:

Erase CFNRY.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_8
	cmd_seq	C_D_KSD_8

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFNRY_ERS_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFNRY_ERS_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS178:  Activate CF 31.2.1.3

Description:

Activate CF.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_9
	cmd_seq	C_D_KSD_9

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CF_ACT_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CF_ACT_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS179:  Activate CFU 31.2.1.3

Description:

Activate CFU.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_10
	cmd_seq	C_D_KSD_10

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFU_ACT_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFU_ACT_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS180:  Deactivate CFC 31.2.1.4

Description:

Deactivate CFC.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_11
	cmd_seq	C_D_KSD_11

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFC_DEACT_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFC_DEACT_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS181:  Deactivate CFNRC 31.2.1.4

Description:

Deactivate CFNRC.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_12
	cmd_seq	C_D_KSD_12

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFNRC_DEACT_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFNRC_DEACT_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS182:  Interrogate CFB 31.2.1.6.1, 1 CCFC

Description:

Interrogate CFB with successful network response indicating not active and no bearer service or teleservice is present.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CCFC:...)	|		|

*	<==========================	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_13
	cmd_seq	C_D_KSD_13

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFB_IRGT_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFB_IRGT_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CCFC_CFU
	cmd_seq	M_PLUS_CCFC_CFU

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

07.01.02	SBK	Expect +CCFC:

### ACISS183:  Interrogate CFB 31.2.1.6.1, 3 CCFC             FAILS !!!! (to be processed)

Description:

Interrogate CFB with successful network response indicating different stati for different teleservices and bearer services (see data definition for details).

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_13
	cmd_seq	C_D_KSD_13

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFB_IRGT_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFB_IRGT_RES2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	07.01.02	SBK	Derived from former ACISS182B

### ACISS184:  Interrogate CFNRY, Speech 31.2.1.6.1

Description:

Interrogate CFNRY.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CCFC:...)	|		|

*	<==========================	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_14
	cmd_seq	C_D_KSD_14

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFNRY_IRGT_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFNRY_IRGT_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CCFC_CFNRY_KSD_V
	cmd_seq	M_PLUS_CCFC_CFNRY_KSD_V

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS185:  Interrogate CFNRC 31.2.1.6.2

Description:

Interrogate CFNRC.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_15
	cmd_seq	C_D_KSD_15

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFNRC_IRGT_V
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFNRC_IRGT_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS186:  Interrogate CFB FAX 31.2.1.6.2

Description:

Interrogate CFB FAX.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_16
	cmd_seq	C_D_KSD_16

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CFB_IRGT_F
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CFB_IRGT_F_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS190:  Register Password CB all 31.8.1.1

Description:

Register Password all CB.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(4)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(5)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(6)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(7)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(8)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(9)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(10)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_17
	cmd_seq	C_D_KSD_17

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_ALLCB_PWD
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_NEW_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_NEW_PWD_RES
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_NEWAGN_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_NEWAGN_PWD_RES
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CPWD_ALLCB_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS191:  Activate BAOC  31.8.3.1

Description:

Activate BAOC.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(4)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(5)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_18
	cmd_seq	C_D_KSD_18

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_BAOC_ACT
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_BAOC_ACT_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS192:  Activate BICR  31.8.3.1

Description:

Activate BIC roaming.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_19
	cmd_seq	C_D_KSD_19

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_BICR_ACT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_BICR_ACT_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS193:  Activate BOIC  31.8.3.2.1

Description:

Activate BOIC.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_20
	cmd_seq	C_D_KSD_20

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_BOIC_ACT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_BOIC_ACT_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS194:  Activate BAIC  31.8.3.2.2

Description:

Activate BAIC.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_21
	cmd_seq	C_D_KSD_21

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_BAIC_ACT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_BAIC_ACT_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS195:  Deactivate all barring services  31.8.4.1

Description:

Deactivate all barring services.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_22
	cmd_seq	C_D_KSD_22

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_ALLCB_DEACT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_ALLCB_DEACT_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS196:  Deactivate all outgoing barring services  31.8.4.1

Description:

Deactivate all outgoing barring services.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_23
	cmd_seq	C_D_KSD_23

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_ALLOUT_DEACT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_ALLOUT_DEACT_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS197:  Deactivate all incoming barring services  31.8.4.2.1

Description:

Deactivate all incoming barring services.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_24
	cmd_seq	C_D_KSD_24

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_ALLIN_DEACT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_ALLIN_DEACT_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS198:  Deactivate BOIC except Home  31.8.4.2.2

Description:

Deactivate BOIC except Home.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_25
	cmd_seq	C_D_KSD_25

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_BOICxHC_DEACT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_BOICxHC_DEACT_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS199:  Interrogate BAIC  31.8.6.1, 1 CLCK

Description:

Interrogate BAIC.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CLCK:…)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_26
	cmd_seq	C_D_KSD_26

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_BAIC_IRGT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_BAIC_IRGT_RES_A

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLCK_IND_1_1
	cmd_seq	M_PLUS_CLCK_IND_1_1

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

07.01.02	SBK	Expect +CLCK

### ACISS200:  Interrogate BAIC  31.8.6.1, 3 CLCKs

Description:

Interrogate BAIC.

Preamble:

- ACISS001

Variants:	<A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CLCK:…)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(msg: +CLCK:…)	|		|

*	<==========================	*		|

(6)	|	ACI_CMD_IND	|		|

|	(msg: +CLCK:…)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_26
	cmd_seq	C_D_KSD_26

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_BAIC_IRGT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_KSD_BAIC_IRGT_RES_B
<B>	fac_inf	A_FAC_KSD_BAIC_IRGT_RES_C

- ACI_CMD_IND
<A>	cmd_len	LM_PLUS_CLCK_IND_1_1
<B>	cmd_len	LM_PLUS_CLCK_IND_1_1
<A>	cmd_seq	M_PLUS_CLCK_IND_1_1
<B>	cmd_seq	M_PLUS_CLCK_IND_1_1

- ACI_CMD_IND
<A>	cmd_len	LM_PLUS_CLCK_IND_1_8
<B>	cmd_len	LM_PLUS_CLCK_IND_1_4
<A>	cmd_seq	M_PLUS_CLCK_IND_1_8
<B>	cmd_seq	M_PLUS_CLCK_IND_1_4

- ACI_CMD_IND
<A>	cmd_len	LM_PLUS_CLCK_IND_1_2
<B>	cmd_len	LM_PLUS_CLCK_IND_1_2
<A>	cmd_seq	M_PLUS_CLCK_IND_1_2
<B>	cmd_seq	M_PLUS_CLCK_IND_1_2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	07.01.02	SBK	Derived from former 195B,C

### ACISS201:  Interrogate BOIC except Home 31.8.6.1

Description:

Interrogate BOIC except home.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CLCK:…)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_27
	cmd_seq	C_D_KSD_27

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_BOICxHC_IRGT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_BOICxHC_IRGT_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLCK_IND_0_15
	cmd_seq	M_PLUS_CLCK_IND_0_15

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

07.01.02	SBK	Expect +CLCK

### ACISS202:  Interrogate BIC roaming 31.8.6.2

Description:

Interrogate BIC roaming.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_28
	cmd_seq	C_D_KSD_28

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_BICR_IRGT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_BICR_IRGT_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS203:  Interrogate BOIC  31.8.6.2

Description:

Interrogate BOIC.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_29
	cmd_seq	C_D_KSD_29

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_BOIC_IRGT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_BOIC_IRGT_RES

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	10.08.98	ACI	Initial

### ACISS204:  Unstructured SS  31.9

Description:

Send Unstructured SS.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CUSD:...)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_30
	cmd_seq	C_D_KSD_30

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_USSD_PROC
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_USSD_PROC_RES

- ACI_CMD_IND
	cmd_len LM_PLUS_CUSD_DEFAULT_DCS_PROC_RES
	cmd_seq M_PLUS_CUSD_DEFAULT_DCS_PROC_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

07.12.02	SBK	Adapted (issue +CUSD)

### ACISS210:  Change PIN 1 via Key Sequence

Description:

Change PIN 1.

Preamble:

- ACISS001

Variants:      <A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	SIM_CHANGE_PIN_REQ	|

|		*	==========================>	*

(3)	|		| 	SIM_CHANGE_PIN_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: result)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_31
	cmd_seq	C_D_KSD_31

- SIM_CHANGE_PIN_REQ
	source	SRC_MMI
	old_pin	F_CUR_PIN
	new_pin	F_NEW_PIN
	pin_id	PHASE_2_PIN_1

- SIM_CHANGE_PIN_CNF
<A>	cause	SIM_NO_ERROR
<B>	cause	SIM_CAUSE_PIN1_REMAIN2
	pin_id	PHASE_2_PIN_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- ACI_CMD_IND
<A>	cmd_len	LM_OK
<B>	cmd_len	LM_ERROR
<A>	cmd_seq	M_OK
<B>	cmd_seq	M_ERROR

History:	03.06.99	ACI	Initial

### ACISS211:  Unblock PIN1 via Key Sequence

Description:

Unblock PIN 1.

Preamble:

- ACISS001

Variants:      <A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	SIM_UNBLOCK_REQ	|

|		*	==========================>	*

(3)	|		| 	SIM_UNBLOCK_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: result)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_33
	cmd_seq	C_D_KSD_33

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	F_PUK
	pin	F_NEW_PIN
	pin_id	PHASE_2_PUK_1

- SIM_UNBLOCK_CNF
<A>	cause	SIM_NO_ERROR
<B>	cause	SIM_CAUSE_PUK1_REMAIN9
	pin_id	PHASE_2_PUK_1
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- ACI_CMD_IND
<A>	cmd_len	LM_OK
<B>	cmd_len	LM_ERROR
<A>	cmd_seq	M_OK
<B>	cmd_seq	M_ERROR

- History:	02.08.99	ACI	Initial

### ACISS212:  Unblock PIN2 via Key Sequence

Description:

Unblock PIN 2.

Preamble:

- ACISS001

Variants:      <A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	SIM_UNBLOCK_REQ	|

|		*	==========================>	*

(3)	|		| 	SIM_UNBLOCK_CNF	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: result)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_34
	cmd_seq	C_D_KSD_34

- SIM_UNBLOCK_REQ
	source	SRC_MMI
	unblock_key	F_PUK
	pin	F_NEW_PIN
	pin_id	PHASE_2_PUK_2

- SIM_UNBLOCK_CNF
<A>	cause	SIM_NO_ERROR
<B>	cause	SIM_CAUSE_PUK2_REMAIN9
	pin_id	PHASE_2_PUK_2
	pin_cnt	NUM_3
	puk_cnt	NUM_10
	pin2_cnt	NUM_3
	puk2_cnt	NUM_10

- ACI_CMD_IND
<A>	cmd_len	LM_OK
<B>	cmd_len	LM_ERROR
<A>	cmd_seq	M_OK
<B>	cmd_seq	M_ERROR

- History:	02.08.99	ACI	Initial

### ACISS213:  Present IMEI via Key Sequence

Description:

Present IMEI out of PCM.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|
	|	(cmd: ATD...)	|		|
	*	==========================>	*		|

(2)	|	ACI_CMD_IND	|		|
	|	(msg: IMEI )	|		|
	*	<==========================	*		|

(3)	|	ACI_CMD_IND	|		|
	|	(msg: result)	|		|
	*	<==========================	*		|
	|		|		|

Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_32
	cmd_seq	C_D_KSD_32

- ACI_CMD_IND
	cmd_len	LM_IMEI
	cmd_seq	M_IMEI

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	02.08.99	ACI	Initial

### ACISS214:  Deactivate BAOC

Description:

Deactivate BAOC

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_35
	cmd_seq	C_D_KSD_35

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_BAOC_DEACT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_BAOC_DEACT_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS215:  Interrogate CLIR

Description:

Interrogate CLIR.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: +CLIR:...)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_36
	cmd_seq	C_D_KSD_36

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CLIR_IRGT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CLIR_IRGT_RES

- ACI_CMD_IND
	cmd_len	LM_PLUS_CLIR_PVTMAL
	cmd_seq	M_PLUS_CLIR_PVTMAL

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS216:  Register Password, unsuccessful attempt CB all

Description:

Register Password all CB. Password given by the user was wrong.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_FACILITY_IND	|

|		*	<==========================	*

(4)	|		| 	MNSS_FACILITY_REQ	|

|		*	==========================>	*

(5)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(6)	|	ACI_CMD_IND	|		|

|	(msg: ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_17
	cmd_seq	C_D_KSD_17

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_ALLCB_PWD
	ss_ver	NOT_USED

- MNSS_FACILITY_IND
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_REQ

- MNSS_FACILITY_REQ
	ti	NUM_0
	fac_inf	A_FAC_ENTER_PWD_RES
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CPWD_ALLCB_ERR_2

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	01.10.99	ACI	Initial

### ACISS217:  Activate CW

Description:

Activate CW.

Preamble:

- ACISS001

Variants: 	<A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_37
	cmd_seq	C_D_KSD_37

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CW_ACT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_KSD_CW_ACT_RES
<B>	fac_inf	A_FAC_KSD_CW_ACT_RES2

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	10.08.98	ACI	Initial

### ACISS218:  Temporary CLIR Modification During Call Setup

Description:

The dial string contains a key sequence, which modifies the CLIR setting for this call temporarily.

Preamble:

- ACISS001

Variants: 	<A>....<D>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNCC_SETUP_REQ	|

|		*	==========================>	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_D_KSD_38
<B>	cmd_len	LC_D_KSD_39
<C>	cmd_len	LC_D_KSD_40
<D>	cmd_len	LC_D_KSD_41
<A>	cmd_seq	C_D_KSD_38
<B>	cmd_seq	C_D_KSD_39
<C>	cmd_seq	C_D_KSD_40
<D>	cmd_seq	C_D_KSD_41

- MNCC_SETUP_REQ
	ti	NUM_0
	prio	PRIO_NORM_CALL
	ri	NOT_PRESENT_8BIT
	bcpara	S_BS_VOICE
	bcpara2	S_BS_NOT_PRESENT
<A>	called_party	S_CLD_PARTY_NINT
<B>	called_party	S_CLD_PARTY_NINT
<C>	called_party	S_CLD_PARTY_INT
<D>	called_party	S_CLD_PARTY_INT
	called_party_sub	S_CLD_PARTY_SUB
<A>	clir_sup	CLR_SUP
<B>	clir_sup	CLR_SUP_NOT
<C>	clir_sup	CLR_SUP
<D>	clir_sup	CLR_SUP_NOT
	fac_inf	NOT_USED

History:	01.10.99	ACI	Initial

### ACISS219:  Erase CCBS Entry

Description:

Erase a registered CCBS entry. Variant A erases only CCBS Request 1. Variant B erases all pending CCBS requests.

Preamble:

- ACISS001

Variants:	<A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_D_KSD_42
<B>	cmd_len	LC_D_KSD_43
<A>	cmd_seq	C_D_KSD_42
<B>	cmd_seq	C_D_KSD_43

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_KSD_CCBS_ERS_1
<B>	fac_inf	A_FAC_KSD_CCBS_ERS_ALL
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CCBS_ERS_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	05.04.00	ACI	Initial

### ACISS230:  Interrogate CCBS Entries: success        FAILS !!!!!

Description:

Interrogate registered CCBS entries.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: %CCBS: ...)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(msg: %CCBS: ...)	|		|

*	<==========================	*		|

(6)	|	ACI_CMD_IND	|		|

|	(msg: %CCBS: ...)	|		|

*	<==========================	*		|

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_44
	cmd_seq	C_D_KSD_44

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CCBS_ITRG
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CCBS_ITRG_3N_RES

- ACI_CMD_IND
	cmd_len	LM_PERCENT_CCBS_ITRG_N1
	cmd_seq	M_PERCENT_CCBS_ITRG_N1

- ACI_CMD_IND
	cmd_len	LM_PERCENT_CCBS_ITRG_N2
	cmd_seq	M_PERCENT_CCBS_ITRG_N2

- ACI_CMD_IND
	cmd_len	LM_PERCENT_CCBS_ITRG_N3
	cmd_seq	M_PERCENT_CCBS_ITRG_N3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	06.04.00	ACI	Initial

### ACISS231:  Interrogate CCBS Entries: error

Description:

Interrogate registered CCBS entries.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK/ERROR)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_D_KSD_44
	cmd_seq	C_D_KSD_44

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_KSD_CCBS_IRGT
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_CCBS_IRGT_ERR

- ACI_CMD_IND
	cmd_len	LM_ERROR
	cmd_seq	M_ERROR

History:	06.04.00	ACI	Initial

### ACISS249:  Key Sequence

Description:

Send variable key sequence. This test case or any variant of it are not intended to act as regression test but can be used to see with the debugger / tapcaller / debug output files in a simple way what the results of a certain key sequence like *100# are, i.e. if they are treated as USSD or standard GSM SS procedure (with its dedicated stage 3 signalling) etc. They could be used for white box testing. Note that for this reason the contents of the MNSS_BEGIN_REQ is kept unspecified (NOT_USED). The MNSS_END_IND is an empty return result except for the invoke id being 0. As some key sequences result in output (unsolicited, e.g. +CUSD) prior to the final result code (e.g. OK), no primitive is expected after the MNSS_END_IND.

Preamble:

- ACISS001

Variants:	<A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: ATD...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_D_KSD_99
<B>	cmd_len	LC_D_KSD_100
<A>	cmd_seq	C_D_KSD_99
<B>	cmd_seq	C_D_KSD_100

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	NOT_USED
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_KSD_EMPTY_RES_INV_ID0

History:	10.08.98	ACI	Initial

07.01.02	SBK	Removed expected OK, clarified use

## Call Completion Busy Subscriber (250 - 269)

### ACISS250:  Clear CCBS Entries        FAILS !!!

Description:

Clear registered CCBS entries.

Preamble:

- ACISS001

Variants:	<A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT%CCBS=...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
<A>	cmd_len	LC_PERCENT_CCBS_ERS_1
<B>	cmd_len	LC_PERCENT_CCBS_ERS_ALL
<A>	cmd_seq	C_PERCENT_CCBS_ERS_1
<B>	cmd_seq	C_PERCENT_CCBS_ERS_ALL

- MNSS_BEGIN_REQ
	ti	NUM_0
<A>	fac_inf	A_FAC_CCBS_ERS_1
<B>	fac_inf	A_FAC_CCBS_ERS_ALL
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CCBS_ERS_1_RES
<B>	fac_inf	A_FAC_CCBS_ERS_ALL_RES

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	06.04.00	ACI	Initial

### ACISS251:  Interrogate CCBS Entries, 3 entries are registered    FAILS !!!

Description:

Interrogate registered CCBS entries. 3 entries are registered and will be displayed.

Preamble:

- ACISS001

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT%CCBS=...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: %CCBS: ...)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(msg: %CCBS: ...)	|		|

*	<==========================	*		|

(6)	|	ACI_CMD_IND	|		|

|	(msg: %CCBS: ...)	|		|

*	<==========================	*		|

(7)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PERCENT_CCBS_ITRG
	cmd_seq	C_PERCENT_CCBS_ITRG

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CCBS_ITRG
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
	fac_inf	A_FAC_CCBS_ITRG_3N_RES

- ACI_CMD_IND
	cmd_len	LM_PERCENT_CCBS_ITRG_N1
	cmd_seq	M_PERCENT_CCBS_ITRG_N1

- ACI_CMD_IND
	cmd_len	LM_PERCENT_CCBS_ITRG_N2
	cmd_seq	M_PERCENT_CCBS_ITRG_N2

- ACI_CMD_IND
	cmd_len	LM_PERCENT_CCBS_ITRG_N3
	cmd_seq	M_PERCENT_CCBS_ITRG_N3

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	07.04.00	ACI	Initial

### ACISS252:  Interrogate CCBS Entries, no entries are registered, status is displayed     FAILS !!!

Description:

Interrogate registered CCBS entries. No entries are registered but status is displayed.

Preamble:

- ACISS001

Variants:	<A>....<B>

APL		ACI		PS

|		|		|

(1)	|	ACI_CMD_REQ	|		|

|	(cmd: AT%CCBS=...)	|		|

*	==========================>	*		|

(2)	|		| 	MNSS_BEGIN_REQ	|

|		*	==========================>	*

(3)	|		| 	MNSS_END_IND	|

|		*	<==========================	*

(4)	|	ACI_CMD_IND	|		|

|	(msg: %CCBS: ...)	|		|

*	<==========================	*		|

(5)	|	ACI_CMD_IND	|		|

|	(msg: OK)	|		|

*	<==========================	*		|

|		|		|

- Parametrization:

Primitive	Parameter	Value

- ACI_CMD_REQ
	cmd_src	CMD_SRC_EXT
	cmd_len	LC_PERCENT_CCBS_ITRG
	cmd_seq	C_PERCENT_CCBS_ITRG

- MNSS_BEGIN_REQ
	ti	NUM_0
	fac_inf	A_FAC_CCBS_ITRG
	ss_ver	NOT_USED

- MNSS_END_IND
	ti	NUM_0
	cause	MNSS_CAUSE_NO_NET_CAUSE
<A>	fac_inf	A_FAC_CCBS_ITRG_P_RES
<B>	fac_inf	A_FAC_CCBS_ITRG_NP_RES

- ACI_CMD_IND
<A>	cmd_len	LM_PERCENT_CCBS_ITRG_P
<B>	cmd_len	LM_PERCENT_CCBS_ITRG_NP
<A>	cmd_seq	M_PERCENT_CCBS_ITRG_P
<B>	cmd_seq	M_PERCENT_CCBS_ITRG_NP

- ACI_CMD_IND
	cmd_len	LM_OK
	cmd_seq	M_OK

History:	07.04.00	ACI	Initial