Some document information in this template is displayed by fields (e.g. the title). If you want to change such information, open the Properties dialog box (File menu) and change the information that is currently entered in. The new information won’t be visible before the field has been updated. To update all fields in a document, click Select All on the Edit menu, and then press F9. For fields in headers and footers you have to repeat the procedure after opening them.

Technical Document

G23-GSM Protocol Stack

ALR

Entity Test Specification

(Edit via File/Properties/Summary!)

| Document Number: | 6301.400.02.110 (Edit via File/Properties/Summary!) |
| --- | --- |
| Version: | 0.13 (Edit via File/Properties/Summary!) |
| Status: | Draft (Edit via File/Properties/Summary!) |
| Approval Authority: |  |
| Creation Date: | 1999-Sep-22 |
| Last changed: | 2003-May-12 by XGUTTEFE |
| File Name: | alr.doc |

Important Notice

Texas Instruments Incorporated and/or its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products, software and services at any time and to discontinue any product, software or service without notice. Customers should obtain the latest relevant information during product design and before placing orders and should verify that such information is current and complete.

All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment. TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI products, software and/or services. To minimize the risks associated with customer products and applications, customers should provide adequate design, testing and operating safeguards.

Any access to and/or use of TI software described in this document is subject to Customers entering into formal license agreements and payment of associated license fees. TI software may solely be used and/or copied subject to and strictly in accordance with all the terms of such license agreements.

Customer acknowledges and agrees that TI products and/or software may be based on or implement industry recognized standards and that certain third parties may claim intellectual property rights therein. The supply of products and/or the licensing of software does not convey a license from TI to any third party intellectual property rights and TI expressly disclaims liability for infringement of third party intellectual property rights.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products, software or services are used.

Information published by TI regarding third–party products, software or services does not constitute a license from TI to use such products, software or services or a warranty, endorsement thereof or statement regarding their availability. Use of such information, products, software or services may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

No part of this document may be reproduced or transmitted in any form or by any means, electronically or mechanically, including photocopying and recording, for any purpose without the express written permission of TI.

Change History

| Date | Changed by | Approved by | Version | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1999-Sep-22 | MPA |  | 0.1 | Being Processed | 1 |
| 2001-Jun-20 | MSB |  | 0.2 | Being Processed |  |
| 2001-Jul-18 | MSB |  | 0.3 | Being Processed |  |
| 2001-Nov-02 | SBK |  | 0.4 | Being Processed |  |
| 2002-Feb-08 | LG |  | 0.5 | Being Processed |  |
| 2002-Feb-18 | OT |  | 0.6 | Being Processed |  |
| 2002-May-17 | MSB |  | 0.7 | Being Processed |  |
| 2002-Jun-28 | MSB |  | 0.8 | Being Processed |  |
| 2002-Sep-26 | DL |  | 0.9 | Being Processed |  |
| 2002-Dec-12 | VK |  | 0.10 | Being Processed |  |
| 2003-Feb-04 | LG |  | 0.11 | Being Processed |  |
| 2003-Apr-07 | MSB |  | 0.12 | Being Processed |  |
| 2003-May-12 | XGUTTEFE |  | 0.13 | Draft |  |

Notes:

- Initial version

Table of Contents

1.1	References	8

1.2	Abbreviations	8

1.3	Terms	9

2	Introduction	9

3	Parameters	10

4	TEST CASES	154

4.1	Configuration	154

4.1.1	ALR000: Filter and Routings (GSM 900)	154

4.1.2	ALR200: Filter and Routings (DCS 1800)	156

4.1.3	ALR400: Filter and Routings (PCS 1900)	157

4.1.4	ALR600: Filter and Routings (Dualband GSM 900 / DCS 1800)	158

4.1.5	ALR849: Filter and Routings (Dualband GSM 900 / E-GSM / DCS 1800)	159

4.2	Cell Selection (GSM 900)	161

4.2.1	ALR001: Initiation by RR, 4 channels available	161

4.2.2	ALR002: Initiation by RR, no channels available	162

4.2.3	ALR003: Find BCCH carrier, first channel	164

4.2.4	ALR004: Find BCCH carrier, first channel failed, then second channel	165

4.2.5	ALR005: Find BCCH carrier, all channels failed, then error indication	166

4.2.6	ALR006: Find BCCH carrier, second channel	167

4.2.7	ALR007: Read BCCH data	168

4.2.8	ALR008: Read failed BCCH data	170

4.2.9	ALR009: RR rejects BCCH carrier, try third channel	171

4.2.10	ALR010: RR rejects BCCH carrier, try fourth channel	172

4.2.11	ALR011: RR rejects BCCH carrier, no further channel available	174

4.2.12	ALR012: RR select second channel	174

4.2.13	ALR013: RR select first channel	176

4.2.14	ALR015: Re-Initiation of Cell Selection during measurements	178

4.2.15	ALR017: Re-Initiation of Cell Selection during BCCH reading	180

4.2.16	ALR044: Stop Idle Mode by Normal Cell Selection	181

4.2.17	ALR018: ALR008 - Engineering mode	182

4.2.18	ALR019: Search for BCCH carrier takes too long	183

4.3	PLMN Selection(GSM 900)	184

4.3.1	ALR084: Init Ncell data, 4 channels available	184

4.3.2	ALR085: Initiation by RR, 4 channels available	190

4.3.3	ALR086: Initiation by RR, no channels available	191

4.3.4	ALR088: Find BCCH carrier, second channel	192

4.3.5	ALR089: Find BCCH carrier, all channels failed, then error indication	193

4.3.6	ALR090: Find BCCH carrier, second channel	195

4.3.7	ALR091: Read BCCH data	196

4.3.8	ALR093: RR rejects BCCH carrier, try third channel	198

4.3.9	ALR094: RR rejects BCCH carrier, try fourth channel	200

4.3.10	ALR095: RR rejects BCCH carrier, no further channel available	202

4.4	Cell Selection(DCS 1800)	203

4.4.1	ALR201: Initiation by RR, 4 channels available	203

4.4.2	ALR202: Initiation by RR, no channels available	204

4.4.3	ALR203: Find BCCH carrier, first channel	205

4.4.4	ALR204: Find BCCH carrier, first channel failed, then second channel	207

4.4.5	ALR205: Find BCCH carrier, all channels failed, then error indication	208

4.4.6	ALR206: Find BCCH carrier, second channel	209

4.4.7	ALR207: Read BCCH data	210

4.4.8	ALR208: Read failed BCCH data	212

4.4.9	ALR209: RR rejects BCCH carrier, try third channel	213

4.4.10	ALR210: RR rejects BCCH carrier, try fourth channel	214

4.4.11	ALR211: RR rejects BCCH carrier, no further channel available	216

4.4.12	ALR212: RR select second channel	216

4.4.13	ALR213: RR select first channel	218

4.4.14	ALR215: Re-Initiation of Cell Selection during measurements	219

4.4.15	ALR217: Re-Initiation of Cell Selection during BCCH reading	221

4.4.16	ALR244: Stop Idle Mode by Normal Cell Selection	223

4.5	Cell Selection (Dualband GSM 900 / DCS 1800)	224

4.5.1	ALR601: Initiation by RR, 8 channels available	224

4.5.2	ALR602: Initiation by RR, no channels available	225

4.5.3	ALR603: Find BCCH carrier, first channel	227

4.5.4	ALR604: Find BCCH carrier, first channel failed, then second channel	228

4.5.5	ALR605: Find BCCH carrier, all channels failed, then error indication	229

4.5.6	ALR606: Find BCCH carrier, second channel	232

4.5.7	ALR607: Read BCCH data	233

4.5.8	ALR608: Read failed BCCH data	235

4.5.9	ALR609: RR rejects BCCH carrier, try third channel	235

4.5.10	ALR610: RR rejects BCCH carrier, try fourth channel	237

4.5.11	ALR611: RR rejects BCCH carrier, no further channel available	238

4.5.12	ALR612: RR select second channel	239

4.5.13	ALR613: RR select first channel	240

4.5.14	ALR615: Re-Initiation of Cell Selection during measurements	242

4.5.15	ALR617: Re-Initiation of Cell Selection during BCCH reading	244

4.5.16	ALR644: Stop Idle Mode by Normal Cell Selection	245

4.6	Page Mode Change	246

4.6.1	ALR020: Initiation with Paging Reorganisation	246

4.6.2	ALR022: Page Mode Change, Paging Reorganisation	248

4.6.3	ALR023: Page Mode Change, Normal Paging to Extended Paging	248

4.6.4	ALR024: Page Mode Change, Normal Paging to Paging Reorganisation	249

4.6.5	ALR025: Page Mode Change, Back to Normal Paging	250

4.6.6	ALR026: Page Mode Change, Normal Paging	250

4.6.7	ALR027: Page Mode Change, Extended Paging a second time	251

4.6.8	ALR028: Page Mode Change, Extended Paging to Paging Reorganisation	252

4.6.9	ALR029: Page Mode Change, Extended Paging to Normal Paging	253

4.6.10	ALR700: Page Mode Change according 26.6.2.3.1	255

4.7	Paging	256

4.7.1	ALR070: Paging Req 1, Empty Paging Message	256

4.7.2	ALR071: Paging Req 1, IMSI / TMSI for MS	256

4.7.3	ALR072: Paging Req 1, Not for MS	257

4.7.4	ALR073: Paging Req 1, Short IMSI / TMSI for MS	258

4.7.5	ALR074: Paging Req 2, Empty Paging Message	259

4.7.6	ALR075: Paging Req 2, TMSI for MS (Mobile Identity 1 or 2)	260

4.7.7	ALR076: Paging Req 2, Not for MS	261

4.7.8	ALR077: Paging Req 2, IMSI / TMSI for MS (Mobile Identity 3)	262

4.7.9	ALR078: Paging Req 2, Short IMSI / TMSI for MS (Mobile Identity 3)	263

4.7.10	ALR079: Paging Req 3, Empty Paging Message	264

4.7.11	ALR080: Paging Req 3, TMSI for MS	265

4.7.12	ALR081: Paging Req 3, Not for MS	267

4.8	Measurement Reporting	267

4.8.1	ALR030: Measurement Reporting, Configuration	267

4.8.2	ALR031: Measurement Reporting, BS_PA_MFRMS = 2	269

4.8.3	ALR032: Measurement Reporting, BS_PA_MFRMS = 3	272

4.8.4	ALR033: Measurement Reporting, BS_PA_MFRMS = 4	275

4.8.5	ALR034: Measurement Reporting, BS_PA_MFRMS = 5	277

4.8.6	ALR035: Measurement Reporting, BS_PA_MFRMS = 6	279

4.8.7	ALR036: Measurement Reporting, BS_PA_MFRMS = 7	281

4.8.8	ALR037: Measurement Reporting, BS_PA_MFRMS = 8	283

4.8.9	ALR038: Measurement Reporting, BS_PA_MFRMS = 9	285

4.8.10	ALR062: Extended Measurement Reporting, start procedure	286

4.8.11	ALR063: Extended Measurement Reporting, successful power measurement	287

4.8.12	ALR064: Extended Measurement Reporting, successful end	288

4.8.13	ALR065: Extended Measurement Reporting, premature end during power measurement	289

4.8.14	ALR066: Extended Measurement Reporting, premature end during BSIC scan	289

4.9	BCCH Reading	290

4.9.1	ALR039: BCCH Reading, BS_PA_MFRMS = 5	290

4.9.2	ALR040: BCCH Reading, BS_PA_MFRMS = 7	295

4.9.3	ALR041: BCCH Reading, BS_PA_MFRMS = 9	299

4.9.4	ALR082: BCCH Reading, Changed Sys Infos	302

4.10	Connection Establishment	307

4.10.1	ALR055: Start of Sending Channel Request Messages (GSM 900)	307

4.10.2	ALR056: Immediate Assignment for the Mobile Station	308

4.10.3	ALR057: Immediate Assignment Reject for the Mobile Station	310

4.10.4	ALR058: T3126 Expiry, Back to Idle Mode	312

4.11	EMO - Extended Measurement on SACCH	313

4.11.1	ALR100: EMO, normal case	313

4.11.2	ALR101: EMO, delayed, invalid and duplicated ba_id values	315

4.12	Handover & Assignment	317

4.12.1	ALR150: Non-synchronized Handover	317

4.12.2	ALR151: FTA 26.6.13.3	318

4.12.3	ALR152: FTA 26.6.13.8	322

4.12.4	ALR153: SACCH Downlink Messages	323

4.12.5	ALR154: Handover, Serving Cell Parameter	326

4.13	Channel mode modify	327

4.13.1	ALR701: Channel mode modify request – AMR half rate	327

4.14	Downlink Failure Detection	327

4.14.1	ALR042: Receive Invalid Paging Messages	327

4.14.2	ALR043: Test Upper Limit of Downlink Timeout Value	328

4.15	Cell Reselection	330

4.15.1	ALR900: Successful Case	330

4.15.2	ALR901: Cell Reselection after dedicated mode	335

4.15.3	ALR920: PDCH Assignment	338

4.15.4	ALR921: PDCH Assignment, Success, Start TBF establishment	339

4.15.5	ALR922: PDCH Assignment, Error during TBB establishment	339

4.15.6	ALR925: Cell Selection after dedicated mode	340

4.15.7	ALR930: Network Coltrolled Change Cell	345

4.16	Idle Mode Neighbourcells Procedures	348

4.16.1	ALR046: Definition of BCCH Allocation	348

4.16.2	ALR047: Synchronisation to Neighbour Cells successful	350

4.16.3	ALR048: Synchronisation to Neighbour Cells failed	362

4.16.4	ALR053: Reading of Neighbour Cell BCCH, failed	376

4.16.5	ALR061: Ncell-Synch, NCC permitted Check	384

4.16.6	ALR650: Multiband = 0, Serving Cell is GSM 900, 8 channels	390

4.16.7	ALR651: Multiband = 0, Serving Cell is DCS 1800, 8 channels	401

4.16.8	ALR652: Multiband = 1, Serving Cell is GSM 900, 8 channels	413

4.16.9	ALR653: Multiband = 1, Serving Cell is GSM 900, 4 channels	424

4.16.10	ALR654: Multiband = 1, Serving Cell is DCS 1800, 8 channels	432

4.16.11	ALR655: Multiband = 1, Serving Cell is DCS 1800, 4 channels	444

4.16.12	ALR902: Synchronisation to Neighbour Cells successful (sys info 4)	453

4.16.13	ALR903: Synchronisation to Neighbour Cells successful (sys info 4 and 7)	464

4.16.14	ALR904: Synchronisation to Neighbour Cells successful (sys info 4 and 8)	476

4.16.15	ALR905: Synchronisation to Neighbour Cells successful (unexpected msg and sys info 3)	491

4.16.16	ALR906: Synchronisation to Neighbour Cells successful (read error and sys info 3)	503

4.16.17	ALR907: Synchronisation to Neighbour Cells successful (sys info 3)	516

4.17	Short Message Cell Broadcast	527

4.17.1	ALR800: Configuration CBCH followed by MMI Request	527

4.17.2	ALR801: MMI Request followed by Configuration of CBCH	529

4.17.3	ALR802: Reception of NULL Message	531

4.17.4	ALR803: Reception of expected CBCH Message	531

4.17.5	ALR804: Reception of unexpected CBCH Message	532

4.17.6	ALR805: Reception of unscheduled SCHEDULE Message	533

4.18	E-OTD	535

4.18.1	ALR950: Power measurement on 3 Neighbour Cells – E-OTD off (Idle Mode)	535

4.18.2	ALR952: Power measurement with change of BS – E-OTD off (Idle Mode)	541

4.18.3	ALR954: Power measurement on 8 Neighbour Cells – E-OTD off (Idle Mode)	546

4.18.4	ALR960: Power measurement – E-OTD on (Idle Mode)	558

4.18.5	ALR962: Power measurement – E-OTD on (Idle Mode)	570

4.18.6	ALR964: Power measurement – E-OTD on – no response from Layer 1 (Idle Mode)	575

4.18.7	ALR966: E-OTD measurement requested by upper layers (IDLE Mode)	576

4.18.8	ALR968: Cell Reselection started during E-OTD measurement (Idle Mode)	576

4.18.9	ALR970: Connection established (E-OTD -DEDICATED Mode)	578

4.18.10	ALR972: E-OTD measurement started (DEDICATED Mode)	581

4.18.11	ALR974: E-OTD measurement halted due to commencement of Handover	582

4.18.12	ALR976: E-OTD Measurement performed  (DEDICATED Mode)	583

4.18.13	ALR978: E-OTD Measurement performed – unknown ARFCNs (DEDICATED Mode)	587

Appendices	593

A.	Acronyms	593

B.	Glossary	593

List of Figures and Tables

List of References

This is just an example!:

| [ISO 9000:2000] | International Organization for Standardization. Quality management systems - Fundamentals and vocabulary. December 2000 |
| --- | --- |

## References

[C_7010.801]	7010.801, References and Vocabulary, Condat AG

[CDD]	6147.706, Technical Documentation CCD, Condat AG

[TI.S922]	Upper layers / L1 Circuit Switched interfaces, TI Nice

[SAP_RR]	6147.107, Service Access Point RR, Condat AG

[MSG_RR]	6147.604, Message Specification RR, Condat AG

## Abbreviations

AGCH	Access Grant Channel

BCCH	Broadcast Control Channel
BS	Base Station
BSIC	Base Station Identification Code

CBCH	Cell Broadcast Channel
CBQ	Cell Bar Qualify
CC	Call Control
CCCH	Common Control Channel
CCD	Condat Coder Decoder
CKSN	Ciphering Key Sequence Number
C/R	Command / Result
C1	Path Loss Criterion
C2	Reselection Criterion

DCCH	Dedicated Control Channel
DISC	Disconnect Frame
DL	Data Link Layer
DM	Disconnected Mode Frame

EA	Extension Bit Address Field
EL	Extension Bit Length Field
EMMI	Electrical Man Machine Interface

F	Final Bit
FACCH	Fast Associated Control Channel
FHO	Forced Handover

GP	Guard Period
GSM	Global System for Mobile Communication

HPLMN	Home Public Land Mobile Network

I	Information Frame
IMEI	International Mobile Equipment Identity
IMSI	International Mobile Subscriber Identity

Kc	Authentication Key

L	Length Indicator
LAI	Location Area Information
LPD	Link Protocol Discriminator

M	More Data Bit
MCC	Mobile Country Code
MM	Mobility Management
MMI	Man Machine Interface
MNC	Mobile Network Code
MS	Mobile Station

NCC	National Colour Code
NECI	New Establishment Causes included
N(R)	Receive Number
N(S)	Send Number

OTD	Observed Time Difference

P	Poll Bit
PCH	Paging Channel
PDU	Protocol Description Unit
P/F	Poll / Final Bit
PL	Physical Layer
PLMN	Public Land Mobile Network

RACH	Random Access Channel
REJ	Reject Frame
RNR	Receive Not Ready Frame
RR	Radio Resource Management
RR	Receive Ready Frame
RTD	Real Time Difference

SABM	Set Asynchronous Balanced Mode
SACCH	Slow Associated Control Channel
SAP	Service Access Point
SAPI	Service Access Point Identifier
SDCCH	Slow Dedicated Control Channel
SIM	Subscriber Identity Module
SMS	Short Message Service
SMSCB	Short Message Service Cell Broadcast
SS	Supplementary Services

TCH	Traffic Channel
TCH/F	Traffic Channel Full Rate
TCH/H	Traffic Channel Half Rate
TDMA	Time Division Multiple Access
TMSI	Temporary Mobile Subscriber Identity

UA	Unnumbered Acknowledgement Frame
UI	Unnumbered Information Frame

VPLMN	Visiting Public Land Mobile Network
V(A)	Acknowledgement State Variable
V(R)	Receive State Variable
V(S)	Send State Variable

## Terms

Entity:	Program which executes the functions of a layer

Message:	A message is a data unit which is transferred between the entities of the same layer (peer-to-peer) of the mobile and infrastructure side. Message is used as a synonym to protocol data unit (PDU). A message may contain several information elements.

Primitive:	A primitive is a data unit which is transferred between layers on one component (mobile station or infrastructure). The primitive has an operation code which identifies the primitive and its parameters.

Service Access Point:	A Service Access Point is a data interface between two layers on one component (mobile station or infrastructure).

# Introduction

The Protocol Stacks are used to define the functionality of the GSM protocols for interfaces. The GSM specifications are normative when used to describe the functionality of interfaces, but the stacks and the subdivision of protocol layers does not imply or restrict any implementation.

The base of the Protocol Stack rests on the physical layer.

The Data Link Layer (DL) is used to handle an acknowledged connection between mobile and base station. The LAPDm protocol is used.

Radio Resource (RR) manages the resources of the air-interface. That means configuration of physical layer, cell selection and cell reselection, data transfer, RR-Connection handling.

Mobility Management (MM) handles registration aspects for the mobile station. It detects changes of location areas and updates a mobile station in the new location area.

Call Control (CC) provides the call functionality. This includes call establishment, call maintenance procedures like Hold, Retrieve or Modify, and call disconnection.

Supplementary Services (SS) handles all call independent supplementary services like call forwarding or call barring.

Short Message Services (SMS) is used for sending and receiving point-to-point short messages. Additionally the reception of cell broadcast short messages is included.

The man machine interface (MMI) is the interface to the user. Normally it is connected with a keypad as input device and a display as output device.

Between the several entities data interfaces are defined. These data interfaces are called Service Access Points (SAPs), indicating that an upper layer uses the services of a lower layer.

The GSM specification do not set out any implementation of the Protocol Stack. The following diagrams show the implementation described in all these documents for the mobile station. All entities except the Man Machine Interface and Physical Layer are implemented as part of the Protocol Stack.

This document describes the tests for the ALR functionality.

# Parameters

#define RR	CCDENT_RR

DECLARATION (ARFCN_11b)

DECLARATION (ARFCN_124a)

DECLARATION (ARFCN_124b)

DECLARATION (ARFCN_14a)

DECLARATION (ARFCN_14b)

DECLARATION (ARFCN_1a)

DECLARATION (ARFCN_1b)

DECLARATION (ARFCN_23_14_124_1)

DECLARATION (ARFCN_23_14_124_1_WITH_STD)

DECLARATION (ARFCN_23a)

DECLARATION (ARFCN_25a)

DECLARATION (ARFCN_25b)

DECLARATION (ARFCN_512a)

DECLARATION (ARFCN_512b)

DECLARATION (ARFCN_580a)

DECLARATION (ARFCN_637_580_810_512)

DECLARATION (ARFCN_637_580_810_512_WITH_STD)

DECLARATION (ARFCN_637_580_885_512)

DECLARATION (ARFCN_637_580_885_512_WITH_STD)

DECLARATION (ARFCN_EXT_MEAS_000)

DECLARATION (ARFCN_EXT_MEAS_SORT_000)

DECLARATION (ARFCN_637a)

DECLARATION (ARFCN_87b)

DECLARATION (ARFCN_885a)

DECLARATION (ARFCN_885b)

DECLARATION (ARFCN_DUAL)

DECLARATION (ARFCN_DUAL_WITH_STD)

DECLARATION (ARFCN_EGSM)

DECLARATION (ASYNC_HO_CMD)

DECLARATION (ASYNC_HO_CMD_2)

DECLARATION (ASYNC_HO_CMD_3)

DECLARATION (BURST_DELTA)

DECLARATION (BURST_RACH)

DECLARATION (CBCH_1_11)

DECLARATION (CBCH_1_11_CONTENT)

DECLARATION (CBCH_1_12)

DECLARATION (CBCH_1_12_CONTENT)

DECLARATION (CBCH_1_13)

DECLARATION (CBCH_1_13_CONTENT)

DECLARATION (CBCH_1_14)

DECLARATION (CBCH_1_14_CONTENT)

DECLARATION (CBCH_1_2)

DECLARATION (CBCH_1_2_CONTENT)

DECLARATION (CBCH_1_3)

DECLARATION (CBCH_1_3_CONTENT)

DECLARATION (CBCH_1_7)

DECLARATION (CBCH_1_7_B)

DECLARATION (CBCH_1_7_B_CONTENT)

DECLARATION (CBCH_1_7_CONTENT)

DECLARATION (CBCH_1_8)

DECLARATION (CBCH_1_8_CONTENT)

DECLARATION (CBCH_2)

DECLARATION (CBCH_2_CONTENT)

DECLARATION (CBCH_3)

DECLARATION (CBCH_3_CONTENT)

DECLARATION (CBCH_4)

DECLARATION (CBCH_4_CONTENT)

DECLARATION (CBCH_DESCRIPTION_4)

DECLARATION (CBCH_DESCRIPTION_4_MA)

DECLARATION (CBCH_DESCRIPTION_8)

DECLARATION (CBCH_DESCRIPTION_8_MA)

DECLARATION (CBCH_MSG_11)

DECLARATION (CBCH_MSG_12)

DECLARATION (CBCH_MSG_13)

DECLARATION (CBCH_MSG_3)

DECLARATION (CBCH_MSG_7)

DECLARATION (CBCH_MSG_7_B)

DECLARATION (CELL_CHAN_DESC_1)

DECLARATION (CELL_CHAN_DESC_1_NEW)

DECLARATION (CELL_DESCRIPTION_2)

DECLARATION (CELL_DESCRIPTION_3)

DECLARATION (CELL_OPT_BCCH_1)

DECLARATION (CELL_OPT_SACCH_1)

DECLARATION (CELL_SELECT_1)

DECLARATION (CELL_SELECT_2)

DECLARATION (CH_TYPE_ASS_AFTER)

DECLARATION (CH_TYPE_ASS_BEFORE)

DECLARATION (CH_TYPE_FREQ_REDEF)

DECLARATION (CH_TYPE_HOP)

DECLARATION (CH_TYPE_IMM_ASS)

DECLARATION (CH_TYPE_SDCCH2)

DECLARATION (CH_TYPE_SDCCH3)

DECLARATION (CH_TYPE_SDCCH4)

DECLARATION (CH_TYPE_TCH2)

DECLARATION (CH_TYPE_TCH2_MA)

DECLARATION (CH_TYPE_TCH3)

DECLARATION (CH_TYPE_TCH3_MA)

DECLARATION (CH_TYPE2)

DECLARATION (CHAN_DESC_1)

DECLARATION (CHAN_DESC_2)

DECLARATION (CHAN_SEL_1)

DECLARATION (CHAN_SEL_10)

DECLARATION (CHAN_SEL_11)

DECLARATION (CHAN_SEL_12)

DECLARATION (CHAN_SEL_2)

DECLARATION (CHAN_SEL_3)

DECLARATION (CHAN_SEL_4)

DECLARATION (CHAN_SEL_5)

DECLARATION (CHAN_SEL_6)

DECLARATION (CHAN_SEL_7)

DECLARATION (CHAN_SEL_8)

DECLARATION (CHAN_SEL_9)

DECLARATION (CHANGED_SYS_INFO_6)

DECLARATION (CHANNEL_DESC_1)

DECLARATION (CHANNEL_DESC_2)

DECLARATION (CHANNEL_DESC_2_TCH)

DECLARATION (CHANNEL_DESC_3)

DECLARATION (CHANNEL_DESC_3_TCH)

DECLARATION (CHANNEL_DESC_4)

DECLARATION (CHANNEL_DESC_ASS_AFTER)

DECLARATION (CHANNEL_DESC_ASS_BEFORE)

DECLARATION (CHANNEL_DESC_CBCH_4)

DECLARATION (CHANNEL_DESC_CBCH_8)

DECLARATION (CHANNEL_DESC_FR)

DECLARATION (CHANNEL_DESC_IA)

DECLARATION (CHLIST_0)

DECLARATION (CHLIST_1_11_14_25_87_124_512_885_FFFF)

DECLARATION (CHLIST_1_11_14_25_87_512)

DECLARATION (CHLIST_1_124)

DECLARATION (CHLIST_1_14)

DECLARATION (CHLIST_1_14_0_124_512_580_1023_975)

DECLARATION (CHLIST_1_14_0_124_512_580_1023_FFFF)

DECLARATION (CHLIST_1_14_124)

DECLARATION (CHLIST_1_14_124_23)

DECLARATION (CHLIST_1_14_124_512_580_637_1023_0)

DECLARATION (CHLIST_1_14_124_512_580_637_1023_FFFF)

DECLARATION (CHLIST_1_14_124_512_580_637_885_FFFF)

DECLARATION (CHLIST_1_14_124_FFFF)

DECLARATION (CHLIST_1_14_15)

DECLARATION (CHLIST_1_14_23_124_512_580_885_FFFF)

DECLARATION (CHLIST_1_14_23_124_FFFF)

DECLARATION (CHLIST_1_14_25_124_512_580_637_885_FFFF)

DECLARATION (CHLIST_1_14_512_513_600_700_810_885_FFFF)

DECLARATION (CHLIST_1_14_512_885)

DECLARATION (CHLIST_1_14_512_885_FFFF)

DECLARATION (CHLIST_1_15_FFFF)

DECLARATION (CHLIST_2_30_FFFF)

DECLARATION (CHLIST_10_20_40_80_90_100_110_120)

DECLARATION (CHLIST_10_20_40_80_90_100_110_120_FFFF)

DECLARATION (CHLIST_10_52_59_73_108_114)

DECLARATION (CHLIST_10_52_59_73_108_114_FFFF)

DECLARATION (CHLIST_10_FFFF)

DECLARATION (CHLIST_14)

DECLARATION (CHLIST_14_124_FFFF)

DECLARATION (CHLIST_14_2_8)

DECLARATION (CHLIST_14_23_1_124)

DECLARATION (CHLIST_14_23_69_115)

DECLARATION (CHLIST_14_23_69_115_FFFF)

DECLARATION (CHLIST_14_25_512_580_637_885)

DECLARATION (CHLIST_14_512_637_885_FFFF)

DECLARATION (CHLIST_14_513_600_700_810_885)

DECLARATION (CHLIST_2_8_FFFF)

DECLARATION (CHLIST_23)

DECLARATION (CHLIST_1_11_14_25_87_124_512_885_23)

DECLARATION (CHLIST_23_1_124_FFFF)

DECLARATION (CHLIST_23_1_14_124)

DECLARATION (CHLIST_23_1_14_124_512_580_637_885)

DECLARATION (CHLIST_23_1_14_124_FFFF)

DECLARATION (CHLIST_1_14_25_124_512_580_637_885_23)

DECLARATION (CHLIST_23_1_14_512_885)

DECLARATION (CHLIST_23_1_15)

DECLARATION (CHLIST_42_2_30)

DECLARATION (CHLIST_23_10)

DECLARATION (CHLIST_26_34_42_52_59_FFFF)

DECLARATION (CHLIST_46_52_59_66_73_74)

DECLARATION (CHLIST_46_52_59_66_73_74_FFFF)

DECLARATION (CHLIST_512_580_810_637)

DECLARATION (CHLIST_512_580_810_FFFF)

DECLARATION (CHLIST_512_580_885_FFFF)

DECLARATION (CHLIST_512_637_810_FFFF)

DECLARATION (CHLIST_512_637_885_FFFF)

DECLARATION (CHLIST_512_810_FFFF)

DECLARATION (CHLIST_512_885_FFFF)

DECLARATION (CHLIST_1_14_25_124_512_580_637_885_578)

DECLARATION (CHLIST_580)

DECLARATION (CHLIST_580_512_637_810)

DECLARATION (CHLIST_580_512_637_885)

DECLARATION (CHLIST_637)

DECLARATION (CHLIST_637_1_14_23_124_512_580_885)

DECLARATION (CHLIST_1_14_512_513_600_700_810_885_637)

DECLARATION (CHLIST_637_1_14_512_885)

DECLARATION (CHLIST_637_512_580_885)

DECLARATION (CHLIST_73_74_75_76)

DECLARATION (CHLIST_73_74_75_76_FFFF)

DECLARATION (CHLIST_975)

DECLARATION (CIPH_PARAM)

DECLARATION (CIPH_PARAM_KC)

DECLARATION (CLASS_DCS_1800)

DECLARATION (CLASS_DUAL)

DECLARATION (CLASS_GSM_1800)

DECLARATION (CLASS_GSM_1900)

DECLARATION (CLASS_GSM_900)

DECLARATION (CTRL_CHAN_DESC_1)

DECLARATION (DCS_ID_EMPTY)

DECLARATION (EMPTY_FRAME)

DECLARATION (EMPTY_NCELL_LIST)

DECLARATION (EMPTY_SCELL_NBCCH)

DECLARATION (FREQ_LIST)

DECLARATION (FREQ_LIST_ASS_AFTER)

DECLARATION (FREQ_LIST_ASS_AFTER_NO)

DECLARATION (FREQ_LIST_ASS_BEFORE)

DECLARATION (FREQ_LIST_ASS_BEFORE_NO)

DECLARATION (FREQ_LIST_FR)

DECLARATION (FREQ_LIST_FR_NO)

DECLARATION (FREQ_LIST_IA)

DECLARATION (FREQ_LIST_IA_FREQ)

DECLARATION (FREQ_LIST_IA_NO)

DECLARATION (FREQ_LIST_NO)

DECLARATION (FULL_READ)

DECLARATION (FULL_READ_ARRAY)

DECLARATION (HO_PARAM)

DECLARATION (HO_PARAM_1)

DECLARATION (I_SMS)

DECLARATION (IMM_ASS)

DECLARATION (IMM_ASS_HOP)

DECLARATION (IMM_ASS_REJ)

DECLARATION (IMSI)

DECLARATION (IMSI2)

DECLARATION (L2_CHANGED_SYS_INFO_6)

DECLARATION (L2_CHANGED_SYS_INFO_6_ARRAY)

DECLARATION (L2_I_SMS)

DECLARATION (L2_I_SMS_ARRAY)

DECLARATION (L2_IMM_ASS)

DECLARATION (L2_IMM_ASS_ARRAY)

DECLARATION (L2_IMM_ASS_EXT_REO)

DECLARATION (L2_IMM_ASS_EXT_REO_ARRAY)

DECLARATION (L2_IMM_ASS_HOP)

DECLARATION (L2_IMM_ASS_HOP_ARRAY)

DECLARATION (L2_IMM_ASS_REJ)

DECLARATION (L2_IMM_ASS_REJ_ARRAY)

DECLARATION (L2_NO_CONTENT)

DECLARATION (L2_PAG_1_EMPTY)

DECLARATION (L2_PAG_1_EMPTY_ARRAY)

DECLARATION (L2_PAG_1_I1_A)

DECLARATION (L2_PAG_1_I1_A_ARRAY)

DECLARATION (L2_PAG_1_I1_S)

DECLARATION (L2_PAG_1_I1_S_ARRAY)

DECLARATION (L2_PAG_1_I2_A)

DECLARATION (L2_PAG_1_I2_A_ARRAY)

DECLARATION (L2_PAG_1_I2_S)

DECLARATION (L2_PAG_1_I2_S_ARRAY)

DECLARATION (L2_PAG_1_SI1_S)

DECLARATION (L2_PAG_1_SI1_S_ARRAY)

DECLARATION (L2_PAG_1_SI2_D)

DECLARATION (L2_PAG_1_SI2_D_ARRAY)

DECLARATION (L2_PAG_1_SI2_T4)

DECLARATION (L2_PAG_1_SI2_T4_ARRAY)

DECLARATION (L2_PAG_1_ST1_A1)

DECLARATION (L2_PAG_1_ST1_A1_ARRAY)

DECLARATION (L2_PAG_1_ST1_A2)

DECLARATION (L2_PAG_1_ST1_A2_ARRAY)

DECLARATION (L2_PAG_1_ST2_T3)

DECLARATION (L2_PAG_1_ST2_T3_ARRAY)

DECLARATION (L2_PAG_1_ST2_T4)

DECLARATION (L2_PAG_1_ST2_T4_ARRAY)

DECLARATION (L2_PAG_1_T1_D)

DECLARATION (L2_PAG_1_T1_D_ARRAY)

DECLARATION (L2_PAG_1_T1_T)

DECLARATION (L2_PAG_1_T1_T_ARRAY)

DECLARATION (L2_PAG_1_T2_D)

DECLARATION (L2_PAG_1_T2_D_ARRAY)

DECLARATION (L2_PAG_1_T2_T)

DECLARATION (L2_PAG_1_T2_T_ARRAY)

DECLARATION (L2_PAG_1_WI1)

DECLARATION (L2_PAG_1_WI1_ARRAY)

DECLARATION (L2_PAG_1_WI2)

DECLARATION (L2_PAG_1_WI2_ARRAY)

DECLARATION (L2_PAG_1_WT1)

DECLARATION (L2_PAG_1_WT1_ARRAY)

DECLARATION (L2_PAG_1_WT2)

DECLARATION (L2_PAG_1_WT2_ARRAY)

DECLARATION (L2_PAG_1_WTYPE)

DECLARATION (L2_PAG_1_WTYPE_ARRAY)

DECLARATION (L2_PAG_2_EMPTY)

DECLARATION (L2_PAG_2_EMPTY_ARRAY)

DECLARATION (L2_PAG_2_I3_A)

DECLARATION (L2_PAG_2_I3_A_ARRAY)

DECLARATION (L2_PAG_2_I3_D)

DECLARATION (L2_PAG_2_I3_D_ARRAY)

DECLARATION (L2_PAG_2_I3_N)

DECLARATION (L2_PAG_2_I3_N_ARRAY)

DECLARATION (L2_PAG_2_I3_S)

DECLARATION (L2_PAG_2_I3_S_ARRAY)

DECLARATION (L2_PAG_2_I3_T)

DECLARATION (L2_PAG_2_I3_T_ARRAY)

DECLARATION (L2_PAG_2_SI3_A)

DECLARATION (L2_PAG_2_SI3_A_ARRAY)

DECLARATION (L2_PAG_2_ST3_D)

DECLARATION (L2_PAG_2_ST3_D_ARRAY)

DECLARATION (L2_PAG_2_ST3_N)

DECLARATION (L2_PAG_2_ST3_N_ARRAY)

DECLARATION (L2_PAG_2_ST3_S)

DECLARATION (L2_PAG_2_ST3_S_ARRAY)

DECLARATION (L2_PAG_2_ST3_T)

DECLARATION (L2_PAG_2_ST3_T_ARRAY)

DECLARATION (L2_PAG_2_T1_A)

DECLARATION (L2_PAG_2_T1_A_ARRAY)

DECLARATION (L2_PAG_2_T1_D)

DECLARATION (L2_PAG_2_T1_D_ARRAY)

DECLARATION (L2_PAG_2_T1_S)

DECLARATION (L2_PAG_2_T1_S_ARRAY)

DECLARATION (L2_PAG_2_T1_T)

DECLARATION (L2_PAG_2_T1_T_ARRAY)

DECLARATION (L2_PAG_2_T2_A)

DECLARATION (L2_PAG_2_T2_A_ARRAY)

DECLARATION (L2_PAG_2_T2_D)

DECLARATION (L2_PAG_2_T2_D_ARRAY)

DECLARATION (L2_PAG_2_T2_S)

DECLARATION (L2_PAG_2_T2_S_ARRAY)

DECLARATION (L2_PAG_2_T2_T)

DECLARATION (L2_PAG_2_T2_T_ARRAY)

DECLARATION (L2_PAG_2_T3_A)

DECLARATION (L2_PAG_2_T3_A_ARRAY)

DECLARATION (L2_PAG_2_T3_D)

DECLARATION (L2_PAG_2_T3_D_ARRAY)

DECLARATION (L2_PAG_2_T3_N)

DECLARATION (L2_PAG_2_T3_N_ARRAY)

DECLARATION (L2_PAG_2_T3_S)

DECLARATION (L2_PAG_2_T3_S_ARRAY)

DECLARATION (L2_PAG_2_T3_T)

DECLARATION (L2_PAG_2_T3_T_ARRAY)

DECLARATION (L2_PAG_2_WRONG)

DECLARATION (L2_PAG_2_WRONG_ARRAY)

DECLARATION (L2_PAG_3_EMPTY)

DECLARATION (L2_PAG_3_EMPTY_ARRAY)

DECLARATION (L2_PAG_3_T1_A)

DECLARATION (L2_PAG_3_T1_A_ARRAY)

DECLARATION (L2_PAG_3_T1_D)

DECLARATION (L2_PAG_3_T1_D_ARRAY)

DECLARATION (L2_PAG_3_T1_S)

DECLARATION (L2_PAG_3_T1_S_ARRAY)

DECLARATION (L2_PAG_3_T1_T)

DECLARATION (L2_PAG_3_T1_T_ARRAY)

DECLARATION (L2_PAG_3_T2_A)

DECLARATION (L2_PAG_3_T2_A_ARRAY)

DECLARATION (L2_PAG_3_T2_D)

DECLARATION (L2_PAG_3_T2_D_ARRAY)

DECLARATION (L2_PAG_3_T2_S)

DECLARATION (L2_PAG_3_T2_S_ARRAY)

DECLARATION (L2_PAG_3_T2_T)

DECLARATION (L2_PAG_3_T2_T_ARRAY)

DECLARATION (L2_PAG_3_T3_A)

DECLARATION (L2_PAG_3_T3_A_ARRAY)

DECLARATION (L2_PAG_3_T3_D)

DECLARATION (L2_PAG_3_T3_D_ARRAY)

DECLARATION (L2_PAG_3_T3_N)

DECLARATION (L2_PAG_3_T3_N_ARRAY)

DECLARATION (L2_PAG_3_T3_S)

DECLARATION (L2_PAG_3_T3_S_ARRAY)

DECLARATION (L2_PAG_3_T3_T)

DECLARATION (L2_PAG_3_T3_T_ARRAY)

DECLARATION (L2_PAG_3_T4_A)

DECLARATION (L2_PAG_3_T4_A_ARRAY)

DECLARATION (L2_PAG_3_T4_D)

DECLARATION (L2_PAG_3_T4_D_ARRAY)

DECLARATION (L2_PAG_3_T4_N)

DECLARATION (L2_PAG_3_T4_N_ARRAY)

DECLARATION (L2_PAG_3_T4_S)

DECLARATION (L2_PAG_3_T4_S_ARRAY)

DECLARATION (L2_PAG_3_T4_T)

DECLARATION (L2_PAG_3_T4_T_ARRAY)

DECLARATION (L2_PAG_3_WRONG)

DECLARATION (L2_PAG_3_WRONG_ARRAY)

DECLARATION (L2_PAGING_REO_1)

DECLARATION (L2_PAGING_REO_1_ARRAY)

DECLARATION (L2_PAGING_REQ_1)

DECLARATION (L2_PAGING_REQ_1_ARRAY)

DECLARATION (L2_PAGING_REQ_1_EXT)

DECLARATION (L2_PAGING_REQ_1_EXT_ARRAY)

DECLARATION (L2_PAGING_REQ_1_REO)

DECLARATION (L2_PAGING_REQ_1_REO_ARRAY)

DECLARATION (L2_PAGING_REQ_1_SAB)

DECLARATION (L2_PAGING_REQ_1_SAB_ARRAY)

DECLARATION (L2_SYS_INFO_1)

DECLARATION (L2_SYS_INFO_1_ARRAY)

DECLARATION (L2_SYS_INFO_1_NEW)

DECLARATION (L2_SYS_INFO_1_NEW_ARRAY)

DECLARATION (L2_SYS_INFO_2)

DECLARATION (L2_SYS_INFO_2_ARRAY)

DECLARATION (L2_SYS_INFO_3)

DECLARATION (L2_SYS_INFO_3_ARRAY)

DECLARATION (L2_SYS_INFO_4)

DECLARATION (L2_SYS_INFO_4_ACS)

DECLARATION (L2_SYS_INFO_4_ACS_ARRAY)

DECLARATION (L2_SYS_INFO_4_ARRAY)

DECLARATION (L2_SYS_INFO_5)

DECLARATION (L2_SYS_INFO_5_ARRAY)

DECLARATION (L2_SYS_INFO_5BIS)

DECLARATION (L2_SYS_INFO_5BIS_ARRAY)

DECLARATION (L2_SYS_INFO_6)

DECLARATION (L2_SYS_INFO_6_ARRAY)

DECLARATION (L2_SYS_INFO_7)

DECLARATION (L2_SYS_INFO_7_ARRAY)

DECLARATION (L2_SYS_INFO_8)

DECLARATION (L2_SYS_INFO_8_ARRAY)

DECLARATION (LOC_AREA_IDENT_1)

DECLARATION (LOC_AREA_IDENT_2)

DECLARATION (MAC_1)

DECLARATION (MCC_1)

DECLARATION (MCC_2)

DECLARATION (MNC_1)

DECLARATION (MNC_2)

DECLARATION (MOB_ALLOC_1)

DECLARATION (MS_ID_IMSI_TMSI)

DECLARATION (MS_ID_SHORT_IMSI_TMSI)

DECLARATION (MSG_ID_1_TO_20)

DECLARATION (MSG_ID_3_7_11_TO_13)

DECLARATION (NCELL_1_L1_23_CONTENT)

DECLARATION (NCELL_1_L1_N23_CONTENT)

DECLARATION (NCELL_BCCH_BITMAP)

DECLARATION (NCELL_RES_124)

DECLARATION (NCELL_RES_124a)

DECLARATION (NCELL_RES_124y)

DECLARATION (NCELL_RES_14)

DECLARATION (NCELL_RES_14a)

DECLARATION (NCELL_RES_14c)

DECLARATION (NCELL_RES_14d)

DECLARATION (NCELL_RES_14y)

DECLARATION (NCELL_RES_14z)

DECLARATION (NCELL_RES_1c)

DECLARATION (NCELL_RES_1d)

DECLARATION (NCELL_RES_1z)

DECLARATION (NCELL_RES_23)

DECLARATION (NCELL_RES_23a)

DECLARATION (NCELL_RES_25)

DECLARATION (NCELL_RES_25a)

DECLARATION (NCELL_RES_25y)

DECLARATION (NCELL_RES_512)

DECLARATION (NCELL_RES_512a)

DECLARATION (NCELL_RES_512c)

DECLARATION (NCELL_RES_512d)

DECLARATION (NCELL_RES_512y)

DECLARATION (NCELL_RES_512z)

DECLARATION (NCELL_RES_513d)

DECLARATION (NCELL_RES_578y)

DECLARATION (NCELL_RES_580)

DECLARATION (NCELL_RES_580a)

DECLARATION (NCELL_RES_580y)

DECLARATION (NCELL_RES_600d)

DECLARATION (NCELL_RES_637)

DECLARATION (NCELL_RES_637_4_1)

DECLARATION (NCELL_RES_637a)

DECLARATION (NCELL_RES_637y)

DECLARATION (NCELL_RES_700d)

DECLARATION (NCELL_RES_810d)

DECLARATION (NCELL_RES_885)

DECLARATION (NCELL_RES_885a)

DECLARATION (NCELL_RES_885c)

DECLARATION (NCELL_RES_885d)

DECLARATION (NCELL_RES_885y)

DECLARATION (NCELL_RES_885z)

DECLARATION (NCELL_RES_SC_23_4_1)

DECLARATION (NCELL_RES_SC_23_8)

DECLARATION (NCELL_RES_SC_23_8_1)

DECLARATION (NCELL_RES_SC_23_8_1_CONTENT)

DECLARATION (NCELL_RES_SC_578_8)

DECLARATION (NCELL_RES_SC_637_4_1)

DECLARATION (NCELL_RES_SC_637_8_1)

DECLARATION (NCELL_RESULT_1)

DECLARATION (NCELL_RESULT_124a)

DECLARATION (NCELL_RESULT_14a)

DECLARATION (NCELL_RESULT_1a)

DECLARATION (NCELL_RESULT_23a)

DECLARATION (NCELL_RESULT_NO_CONTENT)

DECLARATION (NCELL_RESULT_NO_CONTENT_1)

DECLARATION (NCELLS_1_124)

DECLARATION (NCELLS_1_124_BSICS)

DECLARATION (NCELLS_1_124_FO)

DECLARATION (NCELLS_1_124_RXLEVS)

DECLARATION (NCELLS_1_124_TA)

DECLARATION (NCELLS_1_14_124)

DECLARATION (NCELLS_1_14_124_BSICS)

DECLARATION (NCELLS_1_14_124_FO)

DECLARATION (NCELLS_1_14_124_RXLEVS)

DECLARATION (NCELLS_1_14_124_TA)

DECLARATION (NCELLS_1_14_NCC)

DECLARATION (NCELLS_1_14_NCC_BSICS)

DECLARATION (NCELLS_1_14_NCC_FO)

DECLARATION (NCELLS_1_14_NCC_RXLEVS)

DECLARATION (NCELLS_1_14_NCC_TA)

DECLARATION (NCELLS_NO_CONTENT)

DECLARATION (NCELLS_RES_SC_23_16_2A)

DECLARATION (NCELLS_RES_SC_23_16_2B)

DECLARATION (NCELLS_SC_1800_4_1)

DECLARATION (NCELLS_SC_1800_4_1_BSICS)

DECLARATION (NCELLS_SC_1800_4_1_FO)

DECLARATION (NCELLS_SC_1800_4_1_RXLEVS)

DECLARATION (NCELLS_SC_1800_4_1_TA)

DECLARATION (NCELLS_SC_1800_8_1)

DECLARATION (NCELLS_SC_1800_8_1_BSICS)

DECLARATION (NCELLS_SC_1800_8_1_FO)

DECLARATION (NCELLS_SC_1800_8_1_RXLEVS)

DECLARATION (NCELLS_SC_1800_8_1_TA)

DECLARATION (NCELLS_SC_23_16_2)

DECLARATION (NCELLS_SC_900_16_2)

DECLARATION (NCELLS_SC_900_4_1)

DECLARATION (NCELLS_SC_900_4_1_BSICS)

DECLARATION (NCELLS_SC_900_4_1_FO)

DECLARATION (NCELLS_SC_900_4_1_RXLEVS)

DECLARATION (NCELLS_SC_900_4_1_TA)

DECLARATION (NCELLS_SC_900_8)

DECLARATION (NCELLS_SC_900_8_1)

DECLARATION (NCELLS_SC_900_8_1_BSICS)

DECLARATION (NCELLS_SC_900_8_1_FO)

DECLARATION (NCELLS_SC_900_8_1_RXLEVS)

DECLARATION (NCELLS_SC_900_8_1_TA)

DECLARATION (NCELLS_SC_900_8_BSICS)

DECLARATION (NCELLS_SC_900_8_FO)

DECLARATION (NCELLS_SC_900_8_RXLEVS)

DECLARATION (NCELLS_SC_900_8_TA)

DECLARATION (NEIGH_CELL_DESC_1)

DECLARATION (NEIGH_CELL_DESC_2)

DECLARATION (NO_CBCH)

DECLARATION (NO_NCELLS)

DECLARATION (NO_STARTING_TIME)

DECLARATION (NULL_MESSAGE)

DECLARATION (NULL_MESSAGE_CONTENT)

DECLARATION (PAGE_MODE_1)

DECLARATION (PAGING_REQ_1)

DECLARATION (PAGING_REQ_1_EXT)

DECLARATION (PAGING_REQ_1_REO)

DECLARATION (PAGING_REQ_1_SAB)

DECLARATION (PCH_NCH_INFO_1)

DECLARATION (PCK_CHAN_DESC_1)

DECLARATION (PERIODIC_SCELL_BCCH_ARRAY)

DECLARATION (RACH_CTRL_1)

DECLARATION (REQ_REF_1)

DECLARATION (RF_1_14_0_124_512_580_1023_975)

DECLARATION (RF_1_14_0_124_512_580_1023_FFFF)

DECLARATION (RF_1_14_124_23)

DECLARATION (RF_1_14_124_512_580_637_1023_0)

DECLARATION (RF_1_14_124_512_580_637_1023_FFFF)

DECLARATION (RF_1_14_15)

DECLARATION (RF_1_14_23_124_FFFF)

DECLARATION (RF_10_20_40_80_90_100_110_120)

DECLARATION (RF_10_20_40_80_90_100_110_120_FFFF)

DECLARATION (RF_10_FFFF)

DECLARATION (RF_14)

DECLARATION (RF_14_2_8)

DECLARATION (RF_14_23_1_124)

DECLARATION (RF_2_8_FFFF)

DECLARATION (RF_23)

DECLARATION (RF_1_11_14_25_87_124_512_885_23)

DECLARATION (RF_23_1_14_124)

DECLARATION (RF_23_1_14_124_512_580_637_885)

DECLARATION (RF_1_14_25_124_512_580_637_885_23)

DECLARATION (RF_23_1_14_512_885)

DECLARATION (RF_23_1_15)

DECLARATION (RF_42_2_30)

DECLARATION (RF_23_10)

DECLARATION (RF_512_580_810_637)

DECLARATION (RF_512_580_810_FFFF)

DECLARATION (RF_512_637_810_FFFF)

DECLARATION (RF_1_14_25_124_512_580_637_885_578)

DECLARATION (RF_580)

DECLARATION (RF_580_512_637_810)

DECLARATION (RF_580_512_637_885)

DECLARATION (RF_637)

DECLARATION (RF_637_1_14_23_124_512_580_885)

DECLARATION (RF_1_14_512_513_600_700_810_885_637)

DECLARATION (RF_637_1_14_512_885)

DECLARATION (RF_637_512_580_885)

DECLARATION (RF_975)

DECLARATION (RF_CHANNEL_1)

DECLARATION (RF_CHANNEL_10)

DECLARATION (RF_CHANNEL_11)

DECLARATION (RF_CHANNEL_12)

DECLARATION (RF_CHANNEL_2)

DECLARATION (RF_CHANNEL_3)

DECLARATION (RF_CHANNEL_4)

DECLARATION (RF_CHANNEL_5)

DECLARATION (RF_CHANNEL_6)

DECLARATION (RF_CHANNEL_7)

DECLARATION (RF_CHANNEL_8)

DECLARATION (RF_CHANNEL_9)

DECLARATION (RXLEV_23_14_124_1)

DECLARATION (RXLEV_637_580_810_512)

DECLARATION (RXLEV_637_580_885_512)

DECLARATION (RXLEV_DUAL)

DECLARATION (RXLEV_EGSM)

DECLARATION (RXLEV_EXT_MEAS_000)

DECLARATION (S_TIME_MPHC)

DECLARATION (S_TIME_MPHC_ST)

DECLARATION (S_TIME_T1)

DECLARATION (S_TIME_T1_ST)

DECLARATION (S_TIME_T2)

DECLARATION (S_TIME_T2_ST)

DECLARATION (SCELL_TC0)

DECLARATION (SCELL_TC1)

DECLARATION (SCELL_TC2)

DECLARATION (SCELL_TC3)

DECLARATION (SCELL_TC4)

DECLARATION (SCELL_TC5)

DECLARATION (SCELL_TC6)

DECLARATION (SCELL_TC7)

DECLARATION (SCHEDULE_1_A)

DECLARATION (SCHEDULE_1_A_CONTENT)

DECLARATION (SCHEDULE_1_B)

DECLARATION (SCHEDULE_1_B_CONTENT)

DECLARATION (SCHEDULE_2)

DECLARATION (SCHEDULE_2_CONTENT)

DECLARATION (SCHEDULE_3)

DECLARATION (SCHEDULE_3_CONTENT)

DECLARATION (SCHEDULE_4)

DECLARATION (SCHEDULE_4_CONTENT)

DECLARATION (SI6_REST_OCT_1)

DECLARATION (SI7_REST_OCT_1)

DECLARATION (SI8_REST_OCT_1)

DECLARATION (STARTING_TIME)

DECLARATION (STARTING_TIME_T1)

DECLARATION (STARTING_TIME_T2)

DECLARATION (STOP_ARRAY_1)

DECLARATION (STOP_ARRAY_11)

DECLARATION (STOP_ARRAY_124)

DECLARATION (STOP_ARRAY_14)

DECLARATION (STOP_ARRAY_25)

DECLARATION (STOP_ARRAY_512)

DECLARATION (STOP_ARRAY_513)

DECLARATION (STOP_ARRAY_580)

DECLARATION (STOP_ARRAY_600)

DECLARATION (STOP_ARRAY_637)

DECLARATION (STOP_ARRAY_700)

DECLARATION (STOP_ARRAY_810)

DECLARATION (STOP_ARRAY_87)

DECLARATION (STOP_ARRAY_885)

DECLARATION (STOP_ARRAY_EMPTY_6)

DECLARATION (STOP_ARRAY_EMPTY_12)

DECLARATION (STOP_BURSTS)

DECLARATION (SYS_INFO_4_ACS)

DECLARATION (SYS_INFO_6)

DECLARATION (SYS_INFO_7)

DECLARATION (SYS_INFO_8)

DECLARATION (T123_BURST_1)

DECLARATION (TIME_ADVANCE_1)

DECLARATION (TIME_ADVANCE_2)

DECLARATION (TMSI)

DECLARATION (TMSI2)

DECLARATION (TR_PARAM)

DECLARATION (TWO_BURSTS)

DECLARATION (VBS_VGCS_OPT_1)

/* AMR */

DECLARATION (S_AMR_conf_4_ICMI)
DECLARATION (SA_COD_PROP_4)
DECLARATION (S_COD_PROP_1)
DECLARATION (S_COD_PROP_2)
DECLARATION (S_COD_PROP_3)
DECLARATION (S_amr_configuration)

DECLARATION (A_THRESHOLD)

DECLARATION (A_HYSTERESIS)

/* EMO */

DECLARATION (A_ARFCN_EMO_000)

DECLARATION (CHAN_LIST_100)

DECLARATION (CHAN_LIST_101)

DECLARATION (RADIO_FREQ_100)

DECLARATION (RADIO_FREQ_101)

DECLARATION (NCELL_MEAS_100)

DECLARATION (NCELL_MEAS_102)

DECLARATION (S_EMO_MEAS_RES_000)

DECLARATION (S_EMO_MEAS_RES_000_000)

DECLARATION (S_EMO_MEAS_RES_000_001)

DECLARATION (S_EMO_MEAS_RES_000_002)

DECLARATION (S_EMO_MEAS_RES_000_003)

DECLARATION (S_EMO_MEAS_RES_000_004)

DECLARATION (RES_LIST_100_000)

DECLARATION (RES_LIST_100_001)

DECLARATION (RES_LIST_100_002)

DECLARATION (RES_LIST_100_003)

DECLARATION (RES_LIST_100_004)

DECLARATION (RES_LIST_100_NULL)

DECLARATION (RES_LIST_100)

DECLARATION (RES_LIST_102_000)

DECLARATION (RES_LIST_102_001)

DECLARATION (RES_LIST_102_002)

DECLARATION (RES_LIST_102_003)

DECLARATION (RES_LIST_102_004)

DECLARATION (RES_LIST_102_NULL)

DECLARATION (RES_LIST_102)

/* DL/26.09.02: EOTD */

/* DECLARATIONS: arrays */

DECLARATION (eotd_crosscor_6)

DECLARATION (eotd_crosscor_9)

DECLARATION (eotd_crosscor_12)

DECLARATION (NCELL_LIST_EOTD)

DECLARATION (ARFCN_LIST_EOTD_1)

DECLARATION (RX_LEV_LIST_EOTD_1)

DECLARATION (BSIC_LIST_EOTD_1)

DECLARATION (TIME_ALIGN_LIST_EOTD_1)

DECLARATION (FRAME_OFS_LIST_EOTD_1)

DECLARATION (ARFCN_LIST_EOTD_2)

DECLARATION (RX_LEV_LIST_EOTD_2)

DECLARATION (BSIC_LIST_EOTD_2)

DECLARATION (TIME_ALIGN_LIST_EOTD_2)

DECLARATION (FRAME_OFS_LIST_EOTD_2)

DECLARATION (NCELLS_SC_900_8_FO_EOTD)

/* DECLARATIONS: structs – primitives */

DECLARATION (CHAN_LIST_EOTD)

DECLARATION (NCELL_RESULT_1023a)

DECLARATION (NCELL_RESULT_114a)

DECLARATION (NCELL_RESULT_46a)

DECLARATION (NCELL_RESULT_22a)

DECLARATION (NCELL_RESULT_15a)

DECLARATION (NCELL_RESULT_74a)

DECLARATION (NCELL_RESULT_30a)

DECLARATION (NCELL_RESULT_526a)

DECLARATION (ARFCN_23_EOTD)

DECLARATION (ARFCN_637_EOTD)

DECLARATION (ARFCN_25_EOTD)

DECLARATION (ARFCN_14_EOTD)

DECLARATION (ARFCN_512_EOTD)

DECLARATION (ARFCN_516_EOTD)

DECLARATION (ARFCN_525_EOTD)

DECLARATION (ARFCN_580_EOTD)

DECLARATION (ARFCN_885_EOTD)

DECLARATION (eotd_sc_res_ok)

DECLARATION (eotd_sc_res1_ok)

DECLARATION (EOTD_NC_RES_637)

DECLARATION (EOTD_NC_RES_25)

DECLARATION (EOTD_NC_RES_14)

DECLARATION (EOTD_NC_RES_512)

DECLARATION (EOTD_NC_RES_516)

DECLARATION (EOTD_NC_RES_525)

DECLARATION (EOTD_NC_RES_580)

DECLARATION (EOTD_NC_RES_885)

DECLARATION (NCELL_ARFCN_1)

DECLARATION (NCELL_ARFCN_14)

DECLARATION (NCELL_ARFCN_23)

DECLARATION (NCELL_ARFCN_25)

DECLARATION (NCELL_ARFCN_124)

DECLARATION (NCELL_ARFCN_512)

DECLARATION (NCELL_ARFCN_516)

DECLARATION (NCELL_ARFCN_525)

DECLARATION (NCELL_ARFCN_580)

DECLARATION (NCELL_ARFCN_637)

DECLARATION (NCELL_ARFCN_885)

DECLARATION (NCELL_ARFCN_EMPTY)

/* DECLARATIONS: struct arrays */

DECLARATION (NCELLS_SC_900_EOTD)

DECLARATION (NCELL_EOTD_7)

DECLARATION (NCELL_EOTD_9)

DECLARATION (EOTD_RESULT_6)

DECLARATION (EOTD_RESULT_8)

DECLARATION (NCELL_LIST_1_EOTD)

DECLARATION (NCELL_LIST_3_EOTD)

DECLARATION (NCELL_LIST_5_EOTD_IDLE)

DECLARATION (NCELL_LIST_6_EOTD_IDLE)

DECLARATION (NCELL_LIST_6_EOTD_DEDI)

DECLARATION (NCELL_LIST_8_EOTD_DEDI)

DECLARATION (NCELL_RESULT_EOTD_1)

DECLARATION (NCELL_RESULT_EOTD_2)

DECLARATION (NCELLS_EOTD_BSIC_1)

DECLARATION (NCELLS_EOTD_BSIC_2)

/*

BYTE	TV_INVALID_TIMING_INFO	0

BYTE	TV_VALID_TIMING_INFO	2

BYTE	SM_WIDE_MODE		0

BYTE	L2_CHANNEL_NBCCH		6

BYTE	PGM_NORMAL			0

BYTE	PGM_EXTENDED			1

BYTE	PGM_REORG			2

SHORT	NCELL_BCCH_SI_3_4		0x00CC

SHORT	NCELL_BCCH_SI_2_3_4		0x00CE

SHORT	NCELL_BCCH_SI_2		0x0002

SHORT	NCELL_BCCH_SI_3_7_8		0x8844

BYTE	NO_OF_CELLS_4		4

BYTE	HO_COMPLETE			0

BYTE	HO_TIMEOUT			1

BYTE	HO_TIMEOUT			1

BYTE    CBCH_READ_NORM		0

BYTE    CBCH_READ_EXT		1

BYTE	CBCH_NORM_BLOCK234		7

BYTE	MAX_SCHED_SIZE		10

*/

BYTE	NO_OF_CELLS_3		3

BYTE 	RAND_BURSTS_2		2

BYTE	TI_0	0

BYTE	TI_1	1

BYTE	TI_2	2

BYTE	TI_3	3

BYTE	TI_4	4

BYTE	TI_5	5

BYTE	TI_6	6

BYTE	TI_7	7

BYTE	TI_8	8

BYTE	TI_9	9

BYTE 	NCC_PERMIT_1	0x01

BYTE 	NCC_PERMIT_2	0x00

SHORT	CELL_IDENT_1	0x3748

SHORT  CELL_IDENT_2 	0x0004

SHORT 	CELL_IDENT_3	0x0104

SHORT	ACC_0005	0x0040

BYTE	PAGE_NORM	0

BYTE	MODE_CBCH_CONFIG	10

BYTE	CBCH_ACCEPT	0

BYTE	CBCH_READ_SUCCESS	1

BYTE	CH_TYPE_CBCH	7

BYTE	HO_REF	23

BYTE	HO_POW	10

BYTE	RXLEV_IDX_1	0
BYTE	RXLEV_IDX_2	1
BYTE	RXLEV_IDX_3_1800	2
BYTE	RXLEV_IDX_DUAL	3
BYTE	RXLEV_IDX_2_DUAL	1
BYTE	RXLEV_IDX_7	7

BYTE	SCHED_SIZE_1	1
BYTE	SCHED_LEN_0	0
BYTE	CHAN_LIST_IDX_0	0
SHORT	ONE_ELEM	1
SHORT	PERIODIC_SCELL_BCCH_ARRAY_SIZE	8

BYTE	CBCH_LEN_22	22
BYTE	CBCH_LEN_44	44
BYTE	CBCH_LEN_66	66
BYTE	CBCH_LEN_88	88

BYTE 	TEST 	0x7C

BYTE	ENTITY	0x01

LONG	Bitm_L	0x0040
SHORT	Bitm_H	0x0000

/* Constants */

SHORT	ARFCN_0	0
SHORT	ARFCN_1	1
SHORT	ARFCN_1_STD_900	2049 /* 1 | (1<<11) */
SHORT	ARFCN_1_STD_DUAL	10241 /* 1 | (5<<11) */
SHORT	ARFCN_2	2
SHORT	ARFCN_3	3
SHORT	ARFCN_4	4
SHORT	ARFCN_5	5
SHORT	ARFCN_6	6
SHORT	ARFCN_7	7
SHORT	ARFCN_8	8
SHORT	ARFCN_9	9
SHORT	ARFCN_10	10
SHORT	ARFCN_11	11
SHORT	ARFCN_12	12
SHORT	ARFCN_13	13
SHORT	ARFCN_14	14
SHORT	ARFCN_14_STD_900	2062 /* 14 | (1<<11) */
SHORT	ARFCN_14_STD_DUAL	10254 /* 14 | (5<<11) */
SHORT	ARFCN_15	15
SHORT	ARFCN_16	16
SHORT	ARFCN_17	17
SHORT	ARFCN_18	18
SHORT	ARFCN_19	19
SHORT	ARFCN_20	20
SHORT	ARFCN_21	21
SHORT	ARFCN_22	22
SHORT	ARFCN_23	23
SHORT	ARFCN_23_STD_900	2071 /* 23 | (1<<11) */
SHORT	ARFCN_23_STD_DUAL	10263 /* 23 | (5<<11) */
SHORT	ARFCN_24	24
SHORT	ARFCN_24_STD_900	2072 /* 24 | (1<<11) */
SHORT	ARFCN_24_STD_DUAL	10264 /* 24 | (5<<11) */
SHORT	ARFCN_25	25
SHORT	ARFCN_26	26
SHORT	ARFCN_27	27
SHORT	ARFCN_28	28
SHORT	ARFCN_29	29
SHORT	ARFCN_30	30
SHORT	ARFCN_32	32
SHORT	ARFCN_34	34
SHORT	ARFCN_40	40
SHORT	ARFCN_42	42
SHORT	ARFCN_42_STD_900	2090 /* 42 | (1<<11) */
SHORT	ARFCN_46	46
SHORT	ARFCN_52	52
SHORT	ARFCN_59	59
SHORT	ARFCN_64	64
SHORT	ARFCN_66	66
SHORT	ARFCN_69	69
SHORT	ARFCN_73	73
SHORT	ARFCN_74	74
SHORT	ARFCN_75	75
SHORT	ARFCN_76	76
SHORT	ARFCN_80	80
SHORT	ARFCN_87	87
SHORT	ARFCN_90	90
SHORT	ARFCN_100	100
SHORT	ARFCN_108	108
SHORT	ARFCN_110	110
SHORT	ARFCN_114	114
SHORT	ARFCN_115	115
SHORT	ARFCN_120	120
SHORT	ARFCN_124	124
SHORT	ARFCN_124_STD_900	2172 /* 124 | (1<<11) */
SHORT	ARFCN_124_STD_DUAL	10364 /* 124 | (5<<11) */
SHORT	ARFCN_512	512
SHORT	ARFCN_512_STD_1800	8704 /* 512 | (4<<11) */
SHORT	ARFCN_512_STD_DUAL	10752 /* 512 | (5<<11) */
SHORT	ARFCN_513	513
SHORT	ARFCN_514	514
SHORT	ARFCN_515	515
SHORT	ARFCN_516	516
SHORT	ARFCN_517	517
SHORT	ARFCN_518	518
SHORT	ARFCN_519	519
SHORT	ARFCN_520	520
SHORT	ARFCN_521	521
SHORT	ARFCN_522	522
SHORT	ARFCN_523	523
SHORT	ARFCN_524	524
SHORT	ARFCN_525	525
SHORT	ARFCN_526	526
SHORT	ARFCN_527	527
SHORT	ARFCN_528	528
SHORT	ARFCN_529	529
SHORT	ARFCN_530	530
SHORT	ARFCN_531	531
SHORT	ARFCN_532	532
SHORT	ARFCN_533	533
SHORT	ARFCN_534	534
SHORT	ARFCN_535	535
SHORT	ARFCN_536	536
SHORT	ARFCN_537	537
SHORT	ARFCN_538	538
SHORT	ARFCN_539	539
SHORT	ARFCN_540	540
SHORT	ARFCN_541	541
SHORT	ARFCN_542	542
SHORT	ARFCN_543	543
SHORT	ARFCN_544	544
SHORT	ARFCN_545	545
SHORT	ARFCN_546	546
SHORT	ARFCN_547	547
SHORT	ARFCN_548	548
SHORT	ARFCN_549	549
SHORT	ARFCN_550	550
SHORT	ARFCN_551	551
SHORT	ARFCN_570	570
SHORT	ARFCN_578	578
SHORT	ARFCN_580	580
SHORT	ARFCN_580_STD_1800	8772 /* 580 | (4<<11) */
SHORT	ARFCN_580_STD_DUAL	10820 /* 580 | (5<<11) */
SHORT	ARFCN_588	588
SHORT	ARFCN_600	600
SHORT	ARFCN_637	637
SHORT	ARFCN_637_STD_1800	8829 /* 637 | (4<<11) */
SHORT	ARFCN_637_STD_DUAL	10877 /* 637 | (5<<11) */
SHORT	ARFCN_687	687
SHORT	ARFCN_700	700
SHORT	ARFCN_810	810
SHORT	ARFCN_810_STD_1800	9002 /* 810 | (4<<11) */
SHORT	ARFCN_810_STD_DUAL	11050 /* 810 | (5<<11) */
SHORT	ARFCN_853	853
SHORT	ARFCN_883	883
SHORT	ARFCN_885	885
SHORT	ARFCN_885_STD_1800	9077 /* 885 | (4<<11) */
SHORT	ARFCN_885_STD_DUAL	11125 /* 885 | (5<<11) */
SHORT	ARFCN_975	975
SHORT	ARFCN_1023	1023
BYTE	BA_ID_1	1
BYTE	BA_ID_2	2
BYTE	BA_ID_3	3
BYTE	BA_ID_4	4
BYTE	BS_AG_BLKS_RES_2	2
BYTE	BS_AG_BLKS_RES_3	3
BYTE	BS_AG_BLKS_RES_5	5
BYTE	BS_AG_BLKS_RES_7	7
BYTE	BS_PA_MFRMS_0	0
BYTE	BS_PA_MFRMS_1	1
BYTE	BS_PA_MFRMS_2	2
BYTE	BS_PA_MFRMS_3	3
BYTE	BS_PA_MFRMS_4	4
BYTE	BS_PA_MFRMS_5	5
BYTE	BS_PA_MFRMS_6	6
BYTE	BS_PA_MFRMS_7	7
BYTE	BS_PA_MFRMS_8	8
BYTE	BS_PA_MFRMS_9	9
BYTE	BSIC_0	0
BYTE	BSIC_1	1
BYTE	BSIC_2	2
BYTE	BSIC_10	10
BYTE	BSIC_16	16
BYTE	CCCH_GROUP_0	0
BYTE	CCCH_GROUP_2	2
BYTE	CHANNEL_REQUEST_1	0xA0
BYTE	CHANNEL_REQUEST_2	0xA3
BYTE	CHANNELS_0	0
BYTE	CHANNELS_1	1
BYTE	CHANNELS_2	2
BYTE	CHANNELS_3	3
BYTE	CHANNELS_4	4
BYTE	CHANNELS_5	5

BYTE	CHANNELS_8	8
BYTE	CHANNELS_9	9
BYTE	CHANNELS_17	17
BYTE	CHANNELS_10	10

SHORT	CONST_0	0

SHORT	CONST_1	1

SHORT	CONST_2	2

SHORT	CONST_3	3

SHORT	CONST_4	4

SHORT	CONST_5	5

SHORT	CONST_6	6

SHORT	CONST_7	7

SHORT	CONST_8	8

SHORT	CONST_9	9

SHORT	CONST_10	10

SHORT	CONST_11	11

SHORT	CONST_12	12

SHORT	CONST_13	13

SHORT	CONST_14	14

SHORT	CONST_15	15

SHORT	CONST_16	16

SHORT	CONST_17	17

SHORT	CONST_17	17

SHORT	CONST_18	18

SHORT	CONST_19	19

SHORT	CONST_20	20

SHORT	CONST_21	21
SHORT	CONST_22	22
SHORT	CONST_23	23
SHORT	CONST_24	24
SHORT	CONST_25	25
SHORT	CONST_26	26
SHORT	CONST_27	27
SHORT	CONST_28	28
SHORT	CONST_29	29
SHORT	CONST_30	30
SHORT	CONST_31	31
SHORT	CONST_32	32
SHORT	CONST_33	33

SHORT	CONST_34	34

SHORT	CONST_35	35

SHORT	CONST_36	36

SHORT	CONST_37	37

SHORT	CONST_38	38

SHORT	CONST_39	39

SHORT	CONST_40	40

SHORT	CONST_41	41
SHORT	CONST_42	42
SHORT	CONST_43	43
SHORT	CONST_44	44
SHORT	CONST_45	45
SHORT	CONST_46	46
SHORT	CONST_47	47
SHORT	CONST_48	48
SHORT	CONST_49	49
SHORT	CONST_50	50
SHORT	CONST_51	51
SHORT	CONST_52	52
SHORT	CONST_53	53
SHORT	CONST_54	54
SHORT	CONST_55	55
SHORT	CONST_56	56
SHORT	CONST_57	57
SHORT	CONST_58	58
SHORT	CONST_59	59
SHORT	CONST_60	60
SHORT	CONST_61	61
SHORT	CONST_62	62
SHORT	CONST_63	63
SHORT	CONST_75	75
SHORT	CONST_123	123

SHORT	CONST_124	124

SHORT	CONST_125	125

SHORT	CONST_126	126

SHORT	CONST_127	127

SHORT	CONST_128	128

SHORT	CONST_129	129

SHORT	CONST_130	130

SHORT	CONST_131	131

SHORT	CONST_132	132

SHORT	CONST_133	133

SHORT	CONST_134	134

SHORT	CONST_135	135

SHORT	CONST_136	136

SHORT	CONST_137	137

SHORT	CONST_138	138

SHORT	CONST_139	130

SHORT	CONST_974	974

SHORT	CONST_975	975

SHORT	CONST_1022	1022

SHORT	CONST_1023	1023

BYTE	DLT_10	10
BYTE	DLT_24	24
SHORT	FN_BURST_1	61434
SHORT	FN_OFFSET_0	0
SHORT	FN_OFFSET_1	101
SHORT	FN_OFF_1	1
SHORT	FN_OFF_2	2
SHORT	FN_OFF_3	3
SHORT	FN_OFF_4	4
SHORT	FN_OFF_16	16
SHORT	FN_OFFSET_14	114
SHORT	FN_OFFSET_102	10*102
SHORT	FN_OFFSET_124	224
SHORT	FN_OFFSET_153	7*153
SHORT	FN_OFFSET_204	5*204
SHORT	FN_OFFSET_255	4*255
SHORT	FN_OFFSET_306	3*306
SHORT	FN_OFFSET_357	3*357
SHORT	FN_OFFSET_408	2*408
SHORT	FN_OFFSET_459	2*459
SHORT	FN_OFFSET_10	10
SHORT	FN_OFFSET_11	11
SHORT	FN_OFFSET_23	23
SHORT	FN_OFFSET_25	25
SHORT	FN_OFFSET_87	87
SHORT	FN_OFFSET_512	512
SHORT	FN_OFFSET_513	513
SHORT	FN_OFFSET_540	540
SHORT	FN_OFFSET_570	570
SHORT	FN_OFFSET_580	580
SHORT	FN_OFFSET_600	600
SHORT	FN_OFFSET_637	637
SHORT	FN_OFFSET_700	700
SHORT	FN_OFFSET_810	810

SHORT	FN_OFFSET_816	816
SHORT	FN_OFFSET_874	874

SHORT	FN_OFFSET_885	885

SHORT	FN_OFFSET_918	918

SHORT	FN_OFFSET_1020	1020

SHORT	FN_OFFSET_1071	1071

BYTE	MF51_0	0
BYTE	MF51_2	2
BYTE	MF51_3	3
BYTE	MF51_4	4
BYTE	MF51_5	5
BYTE	MF51_7	7
BYTE	NCC_PERMITTED_4	4
BYTE	PBI_0	0
BYTE	PBI_1	1
BYTE	PBI_2	2
BYTE	PBI_4	4
BYTE	PG_0	0
BYTE	PG_1	1
BYTE	PG_11	11
BYTE	PG_20	20
BYTE	PG_23	23
BYTE	PGROUP_1	1
BYTE	PGROUP_3	3
BYTE	POWER_12	12
BYTE	POWER_1	1
/*
BYTE	POWER_CLASS_2	2
BYTE	POWER_CLASS_3	3
BYTE	POWER_CLASS_4	4
BYTE	POWER_CLASS_5	5
*/
BYTE	RAND_BURST_1	6
BYTE	RAND_BURST_2	8
BYTE	RXLEV_11	11
BYTE	RXLEV_12	12
BYTE	RXLEV_23	23
BYTE	RXLEV_25	25
BYTE	RXLEV_43	43
BYTE	RXLEV_44	44
BYTE	RXLEV_55	55
BYTE	RXLEV_56	56
SHORT	NORMAL_BCCH_BITMAP	0x00FF
SHORT	SYS_12342BIS	0x030F
SHORT	SYS_2342BIS	0x030E
SHORT	SYS_34	0x000C
BYTE	TC_0	0
BYTE	TC_1	1
BYTE	TC_2	2
BYTE	TC_3	3
BYTE	TC_4	4
BYTE	TC_5	5
BYTE	TC_6	6
BYTE	TC_7	7
LONG	TIME_ALIGNMT_0	0
LONG	TIME_ALIGNMT_1	1
SHORT	TIME_ALIGNMT_2	2
SHORT	TIME_ALIGNMT_3	3
SHORT	TIME_ALIGNMT_4	4
SHORT	TIME_ALIGNMT_10	10
SHORT	TIME_ALIGNMT_11	11
SHORT	TIME_ALIGNMT_14	14
SHORT	TIME_ALIGNMT_16	16
SHORT	TIME_ALIGNMT_124	124
SHORT	TIME_ALIGNMT_23	23
SHORT	TIME_ALIGNMT_25	25
SHORT	TIME_ALIGNMT_87	87
SHORT	TIME_ALIGNMT_512	512
SHORT	TIME_ALIGNMT_513	513
SHORT	TIME_ALIGNMT_540	540
SHORT	TIME_ALIGNMT_570	570
SHORT	TIME_ALIGNMT_580	580
SHORT	TIME_ALIGNMT_600	600
SHORT	TIME_ALIGNMT_637	637
SHORT	TIME_ALIGNMT_700	700
SHORT	TIME_ALIGNMT_810	810
SHORT	TIME_ALIGNMT_885	885
BYTE	TIMING_ADVANCE	0x1E
BYTE	TN_0	0
BYTE	TN_4	4

BYTE	LOOP_0	0
BYTE	LOOP_2	2
BYTE	LOOP_4	4
BYTE	LOOP_6	6
BYTE	LOOP_8	8
BYTE	LOOP_10	10
BYTE	LOOP_12	12
BYTE	LOOP_14	14
BYTE	LOOP_16	16
BYTE	LOOP_18	18
BYTE	LOOP_20	20
BYTE	LOOP_22	22
BYTE	LOOP_24	24
BYTE	LOOP_26	26
BYTE	LOOP_28	28
BYTE	LOOP_30	30
BYTE	LOOP_A	0
BYTE	LOOP_B	1
BYTE	LOOP_C	2
BYTE	LOOP_D	3
BYTE	LOOP_E	4
BYTE	LOOP_F	5
BYTE	SUB_CHANNEL_0	0
BYTE	TXPWR0			10
BYTE	VALID_REPORT		1
BYTE	TV_VALID_TIMING_INFO_SB	2
BYTE	STOP_SIZE_1		1
BYTE	STOP_SIZE_0		0

LONG 	FIRST_BLOCKS_0_B	0x032042
BYTE 	SCHED_LEN_5		5

LONG 	FIRST_BLOCKS_1_B	0

BEGIN_STRUCT_ARRAY (NCELL_LIST_1_EOTD, 12)

NCELL_ARFCN_14,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY

ENDARRAY

BEGIN_STRUCT_ARRAY (NCELL_LIST_3_EOTD, 12)

NCELL_ARFCN_1,

NCELL_ARFCN_14,

NCELL_ARFCN_124,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY

ENDARRAY

BEGIN_STRUCT_ARRAY (NCELL_LIST_5_EOTD_IDLE, 12)

NCELL_ARFCN_14,

NCELL_ARFCN_25,

NCELL_ARFCN_512,

NCELL_ARFCN_580,

NCELL_ARFCN_637,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY

ENDARRAY

BEGIN_STRUCT_ARRAY (NCELL_LIST_6_EOTD_IDLE, 12)

NCELL_ARFCN_14,

NCELL_ARFCN_25,

NCELL_ARFCN_512,

NCELL_ARFCN_580,

NCELL_ARFCN_637,

NCELL_ARFCN_885,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY

ENDARRAY

BEGIN_STRUCT_ARRAY (NCELL_LIST_6_EOTD_DEDI, 12)

NCELL_ARFCN_637,

NCELL_ARFCN_25,

NCELL_ARFCN_14,

NCELL_ARFCN_512,

NCELL_ARFCN_580,

NCELL_ARFCN_885,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY

ENDARRAY

BEGIN_STRUCT_ARRAY (NCELL_LIST_8_EOTD_DEDI, 12)

NCELL_ARFCN_637,

NCELL_ARFCN_25,

NCELL_ARFCN_14,

NCELL_ARFCN_512,

NCELL_ARFCN_580,

NCELL_ARFCN_885,

NCELL_ARFCN_516,

NCELL_ARFCN_525,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY,

NCELL_ARFCN_EMPTY

ENDARRAY

/* used for stopping NCELL BCCH and SYNC requests */

BEGIN_SHORT_ARRAY(STOP_ARRAY_EMPTY_6, 6)	0, 0, 0, 0, 0, 0	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_EMPTY_12, 12)
	0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0
ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_14, 6) 	ARFCN_14,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_124, 6) 	ARFCN_124,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_25,6)	ARFCN_25,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_512, 6)	ARFCN_512,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_580,6)	ARFCN_580,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_637,6)	ARFCN_637,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_885,6)	ARFCN_885,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_87,6)	ARFCN_87,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_11,6)	ARFCN_11,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_700,6)	ARFCN_700,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_600,6)	ARFCN_600,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_513,6)	ARFCN_513,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_810,6)	ARFCN_810,0,0,0,0,0 	ENDARRAY

BEGIN_SHORT_ARRAY(STOP_ARRAY_1,6)	ARFCN_1,0,0,0,0,0 	ENDARRAY

/* AMR */

BYTE	CHANNEL_MODE_AMR	0x41
BYTE	CHM_AFS	0x0B
BYTE	CHM_AHS	0x0A
BYTE	NSCB	0x00
BYTE	ICMI_S	0x01
BYTE	ST_MODE_1	0x01
BYTE	ACS_4	0x0F
BYTE	V_COD_PROP	0x01
BYTE	C_COD_PROP_4	0x03

BEGINARRAY(a_threshold,3)
	0x01,0x03,0x05
ENDARRAY

BEGINARRAY(a_hysteresis,3)
	0x01,0x03,0x05
ENDARRAY

BEGIN_PSTRUCT_ARRAY (SA_cod_prop_4, 3)
	S_COD_PROP_1,
	S_COD_PROP_2,
	S_COD_PROP_3
ENDARRAY

BEGIN_PSTRUCT ("cod_prop", S_COD_PROP_1)
	SET_COMP ("codec_thr",	0x01)
	SET_COMP ("codec_hyst",	0x01)
ENDSTRUCT

BEGIN_PSTRUCT ("cod_prop", S_COD_PROP_2)
	SET_COMP ("codec_thr",	0x03)
	SET_COMP ("codec_hyst",	0x03)
ENDSTRUCT

BEGIN_PSTRUCT ("cod_prop", S_COD_PROP_3)
	SET_COMP ("codec_thr",	0x05)
	SET_COMP ("codec_hyst",	0x05)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_conf", S_AMR_conf_4_ICMI)
	SET_COMP ("nscb",	nscb)
	SET_COMP ("icmi",	ICMI_S)
	SET_COMP ("st_mode",	ST_MODE_1)
	SET_COMP ("acs",	ACS_4)
	SET_COMP ("v_cod_prop",	v_cod_prop)
	SET_COMP ("c_cod_prop",	C_cod_prop_4)
	SET_COMP ("cod_prop",	SA_cod_prop_4)
ENDSTRUCT

BEGIN_PSTRUCT ("amr_configuration", S_amr_configuration)
	SET_COMP ("noise_suppression_control_bit",	nscb)
	SET_COMP ("initial_codec_mode_indicator",	ICMI_S)
	SET_COMP ("initial_codec_mode",	ST_MODE_1)
	SET_COMP ("active_codec_set",	ACS_4)
	SET_COMP ("threshold",	a_threshold)
	SET_COMP ("hysteresis",	a_hysteresis)
ENDSTRUCT

/* requests full SCELL BCCH reading */

BEGIN_PSTRUCT_ARRAY(FULL_READ, MAX_SCHED_SIZE)
	FULL_READ_ARRAY,
	EMPTY_SCELL_NBCCH,
	EMPTY_SCELL_NBCCH,
	EMPTY_SCELL_NBCCH,
	EMPTY_SCELL_NBCCH,
	EMPTY_SCELL_NBCCH,
	EMPTY_SCELL_NBCCH,
	EMPTY_SCELL_NBCCH,
	EMPTY_SCELL_NBCCH,
	EMPTY_SCELL_NBCCH
ENDARRAY

BEGIN_PSTRUCT("schedule_array", FULL_READ_ARRAY)
	SET_COMP("modulus", 1)
	SET_COMP("relative_position", 0)
ENDSTRUCT

BEGIN_PSTRUCT("schedule_array", EMPTY_SCELL_NBCCH)
	SET_COMP("modulus", 0)
	SET_COMP("relative_position", 0)
ENDSTRUCT

/* requests periodic 30sec SCELL BCCH reading */

BEGIN_PSTRUCT_ARRAY(PERIODIC_SCELL_BCCH_ARRAY, MAX_SCHED_SIZE)
	SCELL_TC0,
	SCELL_TC1,
	SCELL_TC2,
	SCELL_TC3,
	SCELL_TC4,
	SCELL_TC5,
	SCELL_TC6,
	SCELL_TC7,
	EMPTY_SCELL_NBCCH,
	EMPTY_SCELL_NBCCH
ENDARRAY

BEGIN_PSTRUCT("schedule_array", SCELL_TC0)
	SET_COMP("modulus", 128)
	SET_COMP("relative_position", 0)
ENDSTRUCT

BEGIN_PSTRUCT("schedule_array", SCELL_TC1)
	SET_COMP("modulus", 128)
	SET_COMP("relative_position", 1)
ENDSTRUCT

BEGIN_PSTRUCT("schedule_array", SCELL_TC2)
	SET_COMP("modulus", 128)
	SET_COMP("relative_position", 2)
ENDSTRUCT

BEGIN_PSTRUCT("schedule_array", SCELL_TC3)
	SET_COMP("modulus", 128)
	SET_COMP("relative_position", 3)
ENDSTRUCT

BEGIN_PSTRUCT("schedule_array", SCELL_TC4)
	SET_COMP("modulus", 128)
	SET_COMP("relative_position", 4)
ENDSTRUCT

BEGIN_PSTRUCT("schedule_array", SCELL_TC5)
	SET_COMP("modulus", 128)
	SET_COMP("relative_position", 5)
ENDSTRUCT

BEGIN_PSTRUCT("schedule_array", SCELL_TC6)
	SET_COMP("modulus", 128)
	SET_COMP("relative_position", 6)
ENDSTRUCT

BEGIN_PSTRUCT("schedule_array", SCELL_TC7)
	SET_COMP("modulus", 128)
	SET_COMP("relative_position", 7)
ENDSTRUCT

BEGINARRAY(IMSI,15)
	0,0,1,0,1,3,8,2,3,3,0,5,9,4,5
ENDARRAY

LONG TMSI 0x05223366L

BEGIN_PSTRUCT ("mid", MS_ID_IMSI_TMSI)
	SET_COMP("len_imsi",15)
	SET_COMP("imsi", IMSI)
	SET_COMP("v_tmsi", 1)
	SET_COMP("tmsi",TMSI)
	SKIP_COMP("v_ptmsi")
	SKIP_COMP("ptmsi")
ENDSTRUCT

BEGINARRAY(IMSI2,15)
	0,0,1,0,1,3,8,2,3,3,0xF,0xF,0xF,0xF,0xF
ENDARRAY

LONG TMSI2 142L

BEGIN_PSTRUCT ("mid", MS_ID_SHORT_IMSI_TMSI)
	SET_COMP("len_imsi",10)
	SET_COMP("imsi", IMSI2)
	SET_COMP("v_tmsi", 1)
	SET_COMP("tmsi", TMSI2)
	SKIP_COMP("v_ptmsi")
	SKIP_COMP("ptmsi")
ENDSTRUCT

/*

| 0x04 | Power Class GSM 900 |
| --- | --- |
| 0x00 | Power Class DCS 1800 (not used) |

*/

BEGIN_PSTRUCT("classmark", CLASS_GSM_900)
	SET_COMP("pclass", 4)
	SET_COMP("pclass2", 0)
ENDSTRUCT

/*

| 0x04 | Power Class DCS 1800 |
| --- | --- |
| 0x00 | (not used) |

*/

BEGIN_PSTRUCT("classmark", CLASS_GSM_1800)
	SET_COMP("pclass", 2)
	SET_COMP("pclass2", 0)
ENDSTRUCT

/*

| 0x03 | Power Class PCS 1900 |
| --- | --- |
| 0x00 | (not used) |

*/

BEGIN_PSTRUCT("classmark", CLASS_GSM_1900)
	SET_COMP("pclass", 3)
	SET_COMP("pclass2", 0)
ENDSTRUCT

/*

| 0x03 | Power Class GSM 900 |
| --- | --- |
| 0x01 | Power Class DCS 1800 |

*/

BEGIN_PSTRUCT("classmark", CLASS_DUAL)
	SET_COMP("pclass", CLASS_4)
	SET_COMP("pclass2", CLASS_2)
ENDSTRUCT

/*

| 0x03, 0x03 | message identifier for CBCH |
| --- | --- |
| 0x07, 0x07 | message identifier for CBCH |
| 0x0B, 0x0D | message identifier for CBCH |
| 0xFF, ... | other fields not used |

*/

BEGINARRAY (MSG_ID_3_7_11_TO_13, 40)
	3, 0, 3, 0,
	7, 0, 7, 0,
	11, 0, 13, 0,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF
ENDARRAY

BEGINARRAY (MSG_ID_1_TO_20,  40)
	1, 0, 20, 0,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF
ENDARRAY

/*

| 0xFF, 0xFF ... | data coding scheme for CBCH |
| --- | --- |
| 0xFF, ... | other fields not used |

*/

BEGINARRAY (DCS_ID_EMPTY, 40)
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF,
	0xFF, 0xFF, 0xFF, 0xFF
ENDARRAY

/*

| 0x00 | First block |
| --- | --- |
| 0x00, 0x00 | Serial number |
| 0x00, 0x05 | Message identifier |
| 0x00 | Data coding scheme |
| 0x01 | Page |
| 0xAA, ... | Content |

*/

BEGIN_PSTRUCT ("l2_frame", CBCH_1_7)

SET_COMP ("content", CBCH_1_7_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_1_7_CONTENT, 23)
	0x00,
	0x00, 0x00,
	0x00, 0x07,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", CBCH_1_7_B)

SET_COMP ("content", CBCH_1_7_B_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_1_7_B_CONTENT, 23)
	0x00,
	0x00, 0x01,
	0x00, 0x07,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", CBCH_1_2)

SET_COMP ("content", CBCH_1_2_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_1_2_CONTENT, 23)
	0x00,
	0x00, 0x00,
	0x00, 0x02,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", CBCH_1_3)

SET_COMP ("content", CBCH_1_3_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_1_3_CONTENT, 23)
	0x00,
	0x00, 0x00,
	0x00, 0x03,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", CBCH_1_8)

SET_COMP ("content", CBCH_1_8_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_1_8_CONTENT, 23)
	0x00,
	0x00, 0x00,
	0x00, 0x08,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", CBCH_1_11)

SET_COMP ("content", CBCH_1_11_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_1_11_CONTENT, 23)
	0x00,
	0x00, 0x00,
	0x00, 0x0B,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", CBCH_1_12)

SET_COMP ("content", CBCH_1_12_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_1_12_CONTENT, 23)
	0x00,
	0x00, 0x00,
	0x00, 0x0C,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", CBCH_1_13)

SET_COMP ("content", CBCH_1_13_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_1_13_CONTENT, 23)
	0x00,
	0x00, 0x00,
	0x00, 0x0D,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", CBCH_1_14)

SET_COMP ("content", CBCH_1_14_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_1_14_CONTENT, 23)
	0x00,
	0x00, 0x00,
	0x00, 0x0E,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", NULL_MESSAGE)

SET_COMP ("content", NULL_MESSAGE_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (NULL_MESSAGE_CONTENT, 23)
	0x0F,
	0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", SCHEDULE_1_A)

SET_COMP ("content", SCHEDULE_1_A_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (SCHEDULE_1_A_CONTENT, 23)
	0x08,
	0x01, 0x12,
	0x12, 0x04, 0x80,
	0x00, 0x00, 0x00,
	0x80, 0x05,
	0x41,
	0x80, 0x07,
	0x41,
	0x40,
	0x80, 0x03,
	0x40, 0x40, 0x40, 0x40, 0x40
ENDARRAY

/* unscheduled schedule msg
	beg_sched: 11
	end_sched: 18
	msg map:	4, msg_id 5 wrong msg_id	-> not read
			7, optional, read advised	-> read
			14, msg_id 7			-> read
			17 optional, read advised	-> read
if old msgs are also read:
	msg map: 	2, msg_id 3	 		-> read
			16, msg_id 8 wrong id		-> not read
			18, repeat of 2			-> read
*/

BEGIN_PSTRUCT ("l2_frame", SCHEDULE_1_B)

SET_COMP ("content", SCHEDULE_1_B_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (SCHEDULE_1_B_CONTENT, 23)
	0x08,
	0x0B, 0x12,
	0x12, 0x04, 0x80,
	0x00, 0x00, 0x00,
	0x80, 0x05,
	0x41,
	0x80, 0x07,
	0x41,
	0x40,
	0x80, 0x03,
	0x40, 0x40, 0x40, 0x40, 0x40
ENDARRAY

/*

| 0x01 | Second block |
| --- | --- |
| 0xBA, ... | Content |

*/

BEGIN_PSTRUCT ("l2_frame", CBCH_2)

SET_COMP ("content", CBCH_2_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_2_CONTENT, 23)
	0x01,
	0xBA, 0xBB, 0xBC, 0xBD,
	0xBE, 0xBF, 0xC0, 0xC1,
	0xC2, 0xC3, 0xC4, 0xC5,
	0xC6, 0xC7, 0xC8, 0xC9,
	0xCA, 0xCB, 0xCC, 0xCD,
	0xCE, 0xCF
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", SCHEDULE_2)

SET_COMP ("content", SCHEDULE_2_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (SCHEDULE_2_CONTENT, 23)
	0x01,
	0x40, 0x40, 0x40, 0x40, 0x40,
	0x80, 0x08,
	0x02,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x02 | third block |
| --- | --- |
| 0xD0, ... | content |

*/

BEGIN_PSTRUCT ("l2_frame", CBCH_3)

SET_COMP ("content", CBCH_3_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_3_CONTENT, 23)
	0x02,
	0xD0, 0xD1, 0xD2, 0xD3,
	0xD4, 0xD5, 0xD6, 0xD7,
	0xD8, 0xD9, 0xDA, 0xDB,
	0xDC, 0xDD, 0xDE, 0xDF,
	0xE0, 0xE1, 0xE2, 0xE3,
	0xE4, 0xE5
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", SCHEDULE_3)

SET_COMP ("content", SCHEDULE_3_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (SCHEDULE_3_CONTENT, 23)
	0x02,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B
ENDARRAY

/*

| 0x03 | fourth block |
| --- | --- |
| 0xE6, ... | content |

*/

BEGIN_PSTRUCT ("l2_frame", CBCH_4)

SET_COMP ("content", CBCH_4_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (CBCH_4_CONTENT, 23)
	0x03,
	0xE6, 0xE7, 0xE8, 0xE9,
	0xEA, 0xEB, 0xEC, 0xED,
	0xEE, 0xEF, 0xF0, 0xF1,
	0xF2, 0xF3, 0xF4, 0xF5,
	0xF6, 0xF7, 0xF8, 0xF9,
	0xFA, 0xFB
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", SCHEDULE_4)

SET_COMP ("content", SCHEDULE_4_CONTENT)

ENDSTRUCT

BEGINARRAY_PART (SCHEDULE_4_CONTENT, 23)
	0x13,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B
ENDARRAY

/*

| 0x58, 0x00, 0x00 | Length of field serial number |
| --- | --- |
| 0x00, 0x05 | message identifier |
| 0x00 | data coding scheme |
| 0x01 | Page |
| 0xAA, ... | Content |

*/

BEGINARRAY (CBCH_MSG_3, 88)
	0x00, 0x00,
	0x00, 0x03,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9,
	0xBA, 0xBB, 0xBC, 0xBD,
	0xBE, 0xBF, 0xC0, 0xC1,
	0xC2, 0xC3, 0xC4, 0xC5,
	0xC6, 0xC7, 0xC8, 0xC9,
	0xCA, 0xCB, 0xCC, 0xCD,
	0xCE, 0xCF,
	0xD0, 0xD1, 0xD2, 0xD3,
	0xD4, 0xD5, 0xD6, 0xD7,
	0xD8, 0xD9, 0xDA, 0xDB,
	0xDC, 0xDD, 0xDE, 0xDF,
	0xE0, 0xE1, 0xE2, 0xE3,
	0xE4, 0xE5,
	0xE6, 0xE7, 0xE8, 0xE9,
	0xEA, 0xEB, 0xEC, 0xED,
	0xEE, 0xEF, 0xF0, 0xF1,
	0xF2, 0xF3, 0xF4, 0xF5,
	0xF6, 0xF7, 0xF8, 0xF9,
	0xFA, 0xFB
ENDARRAY

BEGINARRAY (CBCH_MSG_7, 88)
	0x00, 0x00,
	0x00, 0x07,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9,
	0xBA, 0xBB, 0xBC, 0xBD,
	0xBE, 0xBF, 0xC0, 0xC1,
	0xC2, 0xC3, 0xC4, 0xC5,
	0xC6, 0xC7, 0xC8, 0xC9,
	0xCA, 0xCB, 0xCC, 0xCD,
	0xCE, 0xCF,
	0xD0, 0xD1, 0xD2, 0xD3,
	0xD4, 0xD5, 0xD6, 0xD7,
	0xD8, 0xD9, 0xDA, 0xDB,
	0xDC, 0xDD, 0xDE, 0xDF,
	0xE0, 0xE1, 0xE2, 0xE3,
	0xE4, 0xE5,
	0xE6, 0xE7, 0xE8, 0xE9,
	0xEA, 0xEB, 0xEC, 0xED,
	0xEE, 0xEF, 0xF0, 0xF1,
	0xF2, 0xF3, 0xF4, 0xF5,
	0xF6, 0xF7, 0xF8, 0xF9,
	0xFA, 0xFB
ENDARRAY

BEGINARRAY (CBCH_MSG_7_B, 88)
	0x00, 0x01,
	0x00, 0x07,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9,
	0xBA, 0xBB, 0xBC, 0xBD,
	0xBE, 0xBF, 0xC0, 0xC1,
	0xC2, 0xC3, 0xC4, 0xC5,
	0xC6, 0xC7, 0xC8, 0xC9,
	0xCA, 0xCB, 0xCC, 0xCD,
	0xCE, 0xCF,
	0xD0, 0xD1, 0xD2, 0xD3,
	0xD4, 0xD5, 0xD6, 0xD7,
	0xD8, 0xD9, 0xDA, 0xDB,
	0xDC, 0xDD, 0xDE, 0xDF,
	0xE0, 0xE1, 0xE2, 0xE3,
	0xE4, 0xE5,
	0xE6, 0xE7, 0xE8, 0xE9,
	0xEA, 0xEB, 0xEC, 0xED,
	0xEE, 0xEF, 0xF0, 0xF1,
	0xF2, 0xF3, 0xF4, 0xF5,
	0xF6, 0xF7, 0xF8, 0xF9,
	0xFA, 0xFB
ENDARRAY

BEGINARRAY (CBCH_MSG_11, 88)
	0x00, 0x00,
	0x00, 0x0B,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9,
	0xBA, 0xBB, 0xBC, 0xBD,
	0xBE, 0xBF, 0xC0, 0xC1,
	0xC2, 0xC3, 0xC4, 0xC5,
	0xC6, 0xC7, 0xC8, 0xC9,
	0xCA, 0xCB, 0xCC, 0xCD,
	0xCE, 0xCF,
	0xD0, 0xD1, 0xD2, 0xD3,
	0xD4, 0xD5, 0xD6, 0xD7,
	0xD8, 0xD9, 0xDA, 0xDB,
	0xDC, 0xDD, 0xDE, 0xDF,
	0xE0, 0xE1, 0xE2, 0xE3,
	0xE4, 0xE5,
	0xE6, 0xE7, 0xE8, 0xE9,
	0xEA, 0xEB, 0xEC, 0xED,
	0xEE, 0xEF, 0xF0, 0xF1,
	0xF2, 0xF3, 0xF4, 0xF5,
	0xF6, 0xF7, 0xF8, 0xF9,
	0xFA, 0xFB
ENDARRAY

BEGINARRAY (CBCH_MSG_12, 88)
	0x00, 0x00,
	0x00, 0x0C,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9,
	0xBA, 0xBB, 0xBC, 0xBD,
	0xBE, 0xBF, 0xC0, 0xC1,
	0xC2, 0xC3, 0xC4, 0xC5,
	0xC6, 0xC7, 0xC8, 0xC9,
	0xCA, 0xCB, 0xCC, 0xCD,
	0xCE, 0xCF,
	0xD0, 0xD1, 0xD2, 0xD3,
	0xD4, 0xD5, 0xD6, 0xD7,
	0xD8, 0xD9, 0xDA, 0xDB,
	0xDC, 0xDD, 0xDE, 0xDF,
	0xE0, 0xE1, 0xE2, 0xE3,
	0xE4, 0xE5,
	0xE6, 0xE7, 0xE8, 0xE9,
	0xEA, 0xEB, 0xEC, 0xED,
	0xEE, 0xEF, 0xF0, 0xF1,
	0xF2, 0xF3, 0xF4, 0xF5,
	0xF6, 0xF7, 0xF8, 0xF9,
	0xFA, 0xFB
ENDARRAY

BEGINARRAY (CBCH_MSG_13, 88)
	0x00, 0x00,
	0x00, 0x0D,
	0x00,
	0x01,
	0xAA, 0xAB, 0xAC, 0xAD,
	0xAE, 0xAF, 0xB0, 0xB1,
	0xB2, 0xB3, 0xB4, 0xB5,
	0xB6, 0xB7, 0xB8, 0xB9,
	0xBA, 0xBB, 0xBC, 0xBD,
	0xBE, 0xBF, 0xC0, 0xC1,
	0xC2, 0xC3, 0xC4, 0xC5,
	0xC6, 0xC7, 0xC8, 0xC9,
	0xCA, 0xCB, 0xCC, 0xCD,
	0xCE, 0xCF,
	0xD0, 0xD1, 0xD2, 0xD3,
	0xD4, 0xD5, 0xD6, 0xD7,
	0xD8, 0xD9, 0xDA, 0xDB,
	0xDC, 0xDD, 0xDE, 0xDF,
	0xE0, 0xE1, 0xE2, 0xE3,
	0xE4, 0xE5,
	0xE6, 0xE7, 0xE8, 0xE9,
	0xEA, 0xEB, 0xEC, 0xED,
	0xEE, 0xEF, 0xF0, 0xF1,
	0xF2, 0xF3, 0xF4, 0xF5,
	0xF6, 0xF7, 0xF8, 0xF9,
	0xFA, 0xFB
ENDARRAY

BEGIN_SHORT_ARRAY_PART (ARFCN_23_14_124_1, 4)

ARFCN_23, ARFCN_14, ARFCN_124, ARFCN_1
ENDARRAY

BEGIN_SHORT_ARRAY_PART (ARFCN_23_14_124_1_WITH_STD, 4)

ARFCN_23_STD_900, ARFCN_14_STD_900, ARFCN_124_STD_900, ARFCN_1_STD_900
ENDARRAY

BEGINARRAY_PART (RXLEV_23_14_124_1, 4)

56, 44, 25, 21
ENDARRAY

BEGIN_SHORT_ARRAY_PART (ARFCN_637_580_885_512, 4)

ARFCN_637,
	ARFCN_580,
	ARFCN_885,
	ARFCN_512
ENDARRAY

BEGIN_SHORT_ARRAY_PART (ARFCN_637_580_885_512_WITH_STD, 4)

ARFCN_637_STD_1800,
	ARFCN_580_STD_1800,
	ARFCN_885_STD_1800,
	ARFCN_512_STD_1800
ENDARRAY

BEGIN_SHORT_ARRAY_PART (ARFCN_EXT_MEAS_000,10)

CONST_20,

CONST_0,

CONST_974,

CONST_10,

CONST_1,

CONST_1023,

CONST_124,

CONST_1022,

CONST_123,

CONST_975

ENDARRAY

BEGIN_SHORT_ARRAY_PART (ARFCN_EXT_MEAS_SORT_000,10)

(CONST_1023|0x1000),

(CONST_1022|0x1000),

(CONST_0|0x1000),

(CONST_974|0x1000),

(CONST_124|0x1000),

(CONST_123|0x1000),

(CONST_20|0x1000),

(CONST_10|0x1000),

(CONST_1|0x1000),

(CONST_975|0x1000)

ENDARRAY

BEGINARRAY_PART (RXLEV_EXT_MEAS_000,10)

80,70,60,50,40,30,20,10,0,-10
ENDARRAY

BEGINARRAY_PART (RXLEV_637_580_885_512,4)

56, 44, 25, 21
ENDARRAY

BEGIN_SHORT_ARRAY_PART (ARFCN_637_580_810_512_WITH_STD, 4)

ARFCN_637_STD_1800,
	ARFCN_580_STD_1800,
	ARFCN_810_STD_1800,
	ARFCN_512_STD_1800
ENDARRAY

BEGINARRAY_PART (RXLEV_637_580_810_512,4)

56, 44, 25, 12
ENDARRAY

BEGIN_SHORT_ARRAY_PART (ARFCN_DUAL, 8)

ARFCN_23,
	ARFCN_637,
	ARFCN_14,
	ARFCN_580,
	ARFCN_124,
	ARFCN_885,
	ARFCN_1,
	ARFCN_512
ENDARRAY

BEGIN_SHORT_ARRAY_PART (ARFCN_DUAL_WITH_STD, 8)

ARFCN_23_STD_DUAL,
	ARFCN_637_STD_DUAL,
	ARFCN_14_STD_DUAL,
	ARFCN_580_STD_DUAL,
	ARFCN_124_STD_DUAL,
	ARFCN_885_STD_DUAL,
	ARFCN_1_STD_DUAL,
	ARFCN_512_STD_DUAL
ENDARRAY

BEGINARRAY_PART (RXLEV_DUAL, 8)

56, 55, 44, 43, 25, 23, 21, 11
ENDARRAY

BEGIN_SHORT_ARRAY_PART (ARFCN_EGSM, 8)

0,
	ARFCN_975,
	ARFCN_1023,
	ARFCN_580,
	ARFCN_124,
	ARFCN_885,
	ARFCN_1,
	ARFCN_512
ENDARRAY

BEGINARRAY (RXLEV_EGSM, 8)

56, 55, 44, 43, 25, 23, 12, 11
ENDARRAY

/*

| 0x59 | l2 pseudo length (=22 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x19 | message type |
| 0x00, ... | cell channel description |
| 0x00, ... |  |
| 0x00, | rach control parameter |
| 0x2B | rest octet |

*/

BEGIN_PSTRUCT("l2_frame", L2_SYS_INFO_1)
	SET_COMP("content", L2_SYS_INFO_1_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_SYS_INFO_1_ARRAY, 23)
	0x59,
	0x06,
	0x19,
	0x00, 0x04, 0x00, 0x00, 0x00, 0x02, 0x00, 0x01,
	0x00, 0x08, 0x00, 0x00, 0x81, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x40,
	0x2B
ENDARRAY

BEGIN_PSTRUCT("l2_frame", L2_SYS_INFO_1_NEW)
	SET_COMP("content", L2_SYS_INFO_1_NEW_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_SYS_INFO_1_NEW_ARRAY,23)
	0x59,
	0x06,
	0x19,
	0x00, 0x04, 0x00, 0x00, 0x00, 0x02, 0x00, 0x01,
	0x00, 0x18, 0x00, 0x00, 0x81, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x40,
	0x2B
ENDARRAY

/*

| 0x59 | l2 pseudo length (=22 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x1A | message type |
| 0x00, ... | neighbour cell description |
| 0x00, ... |  |
| 0x01, | ncc permitted |
| 0x00, | rach control parameter |

*/

BEGIN_PSTRUCT("l2_frame", L2_SYS_INFO_2)
	SET_COMP("content", L2_SYS_INFO_2_ARRAY)

ENDSTRUCT

BEGINARRAY(L2_SYS_INFO_2_ARRAY,23)
	0x59,
	0x06,
	0x1A,
	0x00, 0x00, 0x00, 0x04, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x04,
	0x01,
	0x00, 0x00, 0x40
ENDARRAY

/*

| 0x59 | l2 pseudo length (=22 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x1B | message type |
| 0x37,0x48 | cell identity |
| 0x21, ... | location area identification |
| 0x28, ... | control channel description |
| 0x5F | cell options BCCH |
| 0x42, 0x56 | cell selection parameter |
| 0x00, | rach control parameter |
| 0x2B | rest octets |

*/

BEGIN_PSTRUCT("l2_frame", L2_SYS_INFO_3)
	SET_COMP("content", L2_SYS_INFO_3_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_SYS_INFO_3_ARRAY, 23)
	0x59,
	0x06,
	0x1B,
	0x37, 0x48, 
	0x21, 0xF3, 0x33, 0x21, 0x47,
	0x28, 0x02, 0x06,
	0x5F,
	0x42, 0x56,
	0x00, 0x00, 0x40,
	0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x59 | l2 pseudo length (=22 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x1C | message type |
| 0x21, ... | location area identification |
| 0x42, 0x56 | cell selection parameter |
| 0x00, | rach control parameter |
| 0x2B | rest octets |

*/

BEGIN_PSTRUCT("l2_frame", L2_SYS_INFO_4)
	SET_COMP("content", L2_SYS_INFO_4_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_SYS_INFO_4_ARRAY, 23)
	0x59,
	0x06,
	0x1C,
	0x21, 0xF3, 0x33, 0x21, 0x47,
	0x42, 0x56,
	0x00, 0x00, 0x40,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT("l2_frame", L2_SYS_INFO_4_ACS)
	SET_COMP("content", L2_SYS_INFO_4_ACS_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_SYS_INFO_4_ACS_ARRAY, 23)
	0x59,
	0x06,
	0x1C,
	0x21, 0xF3, 0x33, 0x21, 0x47,
	0x42, 0xD6,
	0x00, 0x00, 0x40,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT("l2_frame", L2_SYS_INFO_5)
	SET_COMP("content", L2_SYS_INFO_5_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_SYS_INFO_5_ARRAY, 23)
	0xFF, 0xAA,
	0x03, 0x03, 0x01,
	0x06,
	0x1D,
	0x00, 0x04, 0x00, 0x00, 0x00, 0x02, 0x00, 0x01,
	0x00, 0x08, 0x00, 0x00, 0x81, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT("l2_frame", L2_SYS_INFO_5BIS)
	SET_COMP("content", L2_SYS_INFO_5BIS_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_SYS_INFO_5BIS_ARRAY,23)
	0xFF, 0xAA,
	0x03, 0x03, 0x01,
	0x06,
	0x05,
	0x00, 0x04, 0x00, 0x00, 0x00, 0x02, 0x00, 0x01,
	0x00, 0x08, 0x00, 0x00, 0x81, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT("l2_frame", L2_SYS_INFO_6)
	SET_COMP("content", L2_SYS_INFO_6_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_SYS_INFO_6_ARRAY,23)
	0xFF, 0xAA,
	0x03, 0x03, 0x01,
	0x06,
	0x1E,
	0x00, 0x04, 0x00, 0x00, 0x00, 0x02, 0x00, 0x01,
	0x00, 0x08, 0x00, 0x00, 0x81, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT("l2_frame", L2_CHANGED_SYS_INFO_6)
	SET_COMP("content", L2_CHANGED_SYS_INFO_6_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_CHANGED_SYS_INFO_6_ARRAY,23)
	0xFF, 0xAA,
	0x03, 0x03, 0x01,
	0x06,
	0x1E,
	0x01, 0x04, 0x00, 0x00, 0x00, 0x02, 0x00, 0x01,
	0x00, 0x08, 0x00, 0x00, 0x81, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_PSTRUCT("l2_frame", L2_SYS_INFO_7)
	SET_COMP("content", L2_SYS_INFO_7_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_SYS_INFO_7_ARRAY, 23)
	0x59,
	0x06,
	0x1F,
	0x2b, 0x2b, 0x2b, 0x2b, 0x2b,
	0x2b, 0x2b,
	0x2b, 0x2b, 0x2b,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT("l2_frame", L2_SYS_INFO_8)
	SET_COMP("content", L2_SYS_INFO_8_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_SYS_INFO_8_ARRAY, 23)
	0x59,
	0x06,
	0x18,
	0x2b, 0x2b, 0x2b, 0x2b, 0x2b,
	0x2b, 0x2b,
	0x2b, 0x2b, 0x2b,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT("l2_frame", L2_I_SMS)
	SET_COMP("content", L2_I_SMS_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_I_SMS_ARRAY,12)
	0xFF, 0xAA,
	0x0F,
	0x02,
	0x19,
	0x3A, 0x3B, 0x3C, 0x3D, 0x3E, 0x3F,
	0x2B
ENDARRAY

BEGIN_MSTRUCT ("loc_area_ident", LOC_AREA_IDENT_2)

SET_COMP ("mcc", MCC_2)
	SET_COMP ("mnc", MNC_2)
	SET_COMP ("lac", 0x0200)
ENDSTRUCT

BEGINARRAY (MCC_2, 3)

0, 0, 0

ENDARRAY

BEGINARRAY (MNC_2, 3)

0, 0, 0

ENDARRAY

BEGIN_MSTRUCT ("cell_opt_sacch", CELL_OPT_SACCH_1)

SET_COMP ("dtx2_s",  0)
	SET_COMP ("pow_ctrl", POW_CTRL_NO)
	SET_COMP ("dtx_s", S_DTX_F_MAYUSE_H_DONTUSE)
	SET_COMP ("rlt", 1)
ENDSTRUCT

BEGIN_MSTRUCT ("si6_rest_oct", SI6_REST_OCT_1)

SET_COMP ("pch_nch_info",  PCH_NCH_INFO_1)

SET_COMP ("vbs_vgcs_opt",  VBS_VGCS_OPT_1)

ENDSTRUCT

BEGIN_MSTRUCT ("pch_nch_info", PCH_NCH_INFO_1)

SET_COMP ("pch_restruct", PCH_RESTRUCT_NO)

SET_COMP ("nln_sacch", 0)

SET_COMP ("call_prio", CALL_PRIO_LEV_3)

SET_COMP ("nln_status", 0)

ENDSTRUCT

BEGIN_MSTRUCT ("vbs_vgcs_opt",  VBS_VGCS_OPT_1)

SET_COMP ("inband_not", IN_BAND_NOT_NCH)

SET_COMP ("inband_pag", IN_BAND_PAG_PCH)

ENDSTRUCT

BEGINARRAY (CHANGED_SYS_INFO_6, 23)
	0x90, 0x00,
	0x08, 0x00,
	0x00,
	0x06,
	0x1E,
	0x01, 0x04, 0x00, 0x00, 0x00, 0x02, 0x00, 0x01,
	0x00, 0x08, 0x00, 0x00, 0x81, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (I_SMS, 16)
	0xA8, 0x00,
	0x10, 0x00,
	0xFF, 0xAA,
	0x0F,
	0x02,
	0x19,
	0x3A, 0x3B, 0x3C, 0x3D, 0x3E, 0x3F,
	0x2B
ENDARRAY

/*

| 0xB0,0x00 | Length in bits |
| --- | --- |
| 0x08, 0x00 | Offset in bits |
| 0x59 | l2 pseudo length (=22 Byte) |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x19 | Message type |
| 0x00, ... | cell channel description |
| 0x00, ... |  |
| 0x00, | rach control parameter |
| 0x2B | rest octet |

*/

SET_BITBUF ("cell_chan_desc", CELL_CHAN_DESC_1, 128)

0x00, 0x04, 0x00, 0x00, 0x00, 0x02, 0x00, 0x01,
	0x00, 0x08, 0x00, 0x00, 0x81, 0x00, 0x00, 0x00
ENDBITBUF

BEGIN_MSTRUCT ("rach_ctrl", RACH_CTRL_1)
	SET_COMP ("max_retrans",	MAX_RETRANS_1)
	SET_COMP ("tx_integer",	SPREAD_TRANS_3)
	SET_COMP ("cell_bar_access",	BARRED_NO)
	SET_COMP ("re",	REESTAB_YES)
	SET_COMP ("ac",	ACC_0005)
ENDSTRUCT

SET_BITBUF ("cell_chan_desc", CELL_CHAN_DESC_1_NEW, 128)

0x00, 0x04, 0x00, 0x00, 0x00, 0x02, 0x00, 0x01,
	0x00, 0x18, 0x00, 0x00, 0x81, 0x00, 0x00, 0x00
ENDBITBUF

/*

| 0xB0,0x00 | length in bits |
| --- | --- |
| 0x08, 0x00 | offset in bits |
| 0x59 | l2 pseudo length (=22 Byte) |
| 0x06 | protocol discriminator, transaction identifier |
| 0x1A | message type |
| 0x00, ... | neighbour cell description |
| 0x00, ... |  |
| 0x01, | ncc permitted |
| 0x00, | rach control parameter |

*/

SET_BITBUF ("neigh_cell_desc", NEIGH_CELL_DESC_1, 128)

0x00, 0x00, 0x00, 0x04, 0x00, 0x00, 0x00, 0x00,
	0x00, 0x00, 0x00, 0x00, 0x00, 0x80, 0x00, 0x04
ENDBITBUF

SET_BITBUF ("neigh_cell_desc", NEIGH_CELL_DESC_2, 128)

0x00, 0x04, 0x00, 0x00, 0x00, 0x02, 0x00, 0x01,
	0x00, 0x08, 0x00, 0x00, 0x81, 0x00, 0x00, 0x00
ENDBITBUF

/*

| 0xB0,0x00 | length in bits |
| --- | --- |
| 0x08, 0x00 | offset in bits |
| 0x59 | l2 pseudo length (=22 Byte) |
| 0x06 | protocol discriminator, transaction identifier |
| 0x1B | message type |
| 0x37,0x48 | cell identity |
| 0x21, ... | location area identification |
| 0x28, ... | control channel description |
| 0x5F | cell options BCCH |
| 0x42, 0x56 | cell selection parameter |
| 0x00, | rach control parameter |
| 0x2B | rest octets |

*/

BEGIN_MSTRUCT ("loc_area_ident", LOC_AREA_IDENT_1)

SET_COMP ("mcc", MCC_1)
	SET_COMP ("mnc", MNC_1)
	SET_COMP ("lac", 0x2147)
ENDSTRUCT

BEGINARRAY (MCC_1, 3)

1,2,3

ENDARRAY

BEGINARRAY (MNC_1, 2)

3,3

ENDARRAY

BEGIN_MSTRUCT ("ctrl_chan_desc", CTRL_CHAN_DESC_1)

SET_COMP ("att", CCD_ATT_NO)
	SET_COMP ("bs_ag_blks_res", 5	)
	SET_COMP ("ccch_conf", CCD_CCCH_1_NOT_COMB	)
	SET_COMP ("bs_pa_mfrms", 2)
	SET_COMP ("t3212", 6)
ENDSTRUCT

BEGIN_MSTRUCT ("cell_opt_bcch", CELL_OPT_BCCH_1)

SET_COMP ("pow_ctrl",  POW_CTRL_YES)
	SET_COMP ("dtx_b", B_DTX_SHALL_USE)
	SET_COMP ("rlt", 15)
ENDSTRUCT

BEGIN_MSTRUCT ("cell_select", CELL_SELECT_1)

SET_COMP ("cell_resel_hyst", CELL_HYST_4_DB)
	SET_COMP ("ms_txpwr_max_cch", 2)
	SET_COMP ("acs", ACS_USE_SI4	)
	SET_COMP ("neci", NECI_YES)
	SET_COMP ("rxlev_access_min", 22)
ENDSTRUCT

BEGIN_MSTRUCT ("cell_select", CELL_SELECT_2)

SET_COMP ("cell_resel_hyst", CELL_HYST_4_DB)
	SET_COMP ("ms_txpwr_max_cch", 2)
	SET_COMP ("acs", ACS_USE_SI7	)
	SET_COMP ("neci", NECI_YES)
	SET_COMP ("rxlev_access_min", 22)
ENDSTRUCT

BEGIN_MSTRUCT ("si7_rest_oct", SI7_REST_OCT_1)

SKIP_COMP ("opt_sel_par")
	SKIP_COMP ("pow_offs")
	SKIP_COMP ("gprs_indic")
	SKIP_COMP ("lsa_param")
	SKIP_COMP ("cell_ident")

SKIP_COMP ("lsa_id_info")

ENDSTRUCT

BEGIN_MSTRUCT ("si8_rest_oct", SI8_REST_OCT_1)

SKIP_COMP ("opt_sel_par")
	SKIP_COMP ("pow_offs")
	SKIP_COMP ("gprs_indic")
	SKIP_COMP ("lsa_param")
	SKIP_COMP ("cell_ident")

SKIP_COMP ("lsa_id_info")

ENDSTRUCT

/*

| 0x19 | l2 pseudo length (= 6 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x21 | message type |
| PAGING_NORM | page mode normal |
| 0x00 | not empty mobile identity |

*/

BEGIN_PSTRUCT("l2_frame", L2_PAGING_REQ_1)
	SET_COMP("content", L2_PAGING_REQ_1_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAGING_REQ_1_ARRAY, 23)
	0x11,
	0x06,
	0x21,
	PAGING_NORM,
	0x00,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x19 | l2 pseudo length (= 6 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x21 | message type |
| PAGING_SAME | page mode same as before |
| 0x00 | not empty mobile identity |

*/

BEGIN_PSTRUCT("l2_frame", L2_PAGING_REQ_1_SAB)
	SET_COMP("content", L2_PAGING_REQ_1_SAB_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAGING_REQ_1_SAB_ARRAY, 23)
	0x11,
	0x06,
	0x21,
	PAGING_SAME,
	0x00,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B 
ENDARRAY

/*

| 0x19 | l2 pseudo length (= 6 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x21 | message type |
| PAGING_EXT | page mode extended |
| 0x00 | not empty mobile identity |

*/

BEGIN_PSTRUCT("l2_frame", L2_PAGING_REQ_1_EXT)
	SET_COMP("content", L2_PAGING_REQ_1_EXT_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAGING_REQ_1_EXT_ARRAY, 23)
	0x11,
	0x06,
	0x21,
	PAGING_EXT,
	0x00,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B 
ENDARRAY

/*

| 0x19 | l2 pseudo length (= 6 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x21 | message type |
| PAGING_REORG | page mode reorganization |
| 0x00 | not empty mobile identity |

*/

BEGIN_PSTRUCT("l2_frame", L2_PAGING_REO_1)
	SET_COMP("content", L2_PAGING_REO_1_ARRAY)

ENDSTRUCT

BEGIN_PSTRUCT ("l2_frame", L2_PAGING_REQ_1_REO)

SET_COMP ("content", L2_PAGING_REQ_1_REO_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAGING_REQ_1_REO_ARRAY, 23)
	0x11,
	0x06,
	0x21,
	PAGING_REORG,
	0x00,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0xB0,0x00 | length in bits |
| --- | --- |
| 0x08, 0x00 | offset in bits |
| 0x19 | l2 pseudo length (= 6 Byte) |
| 0x06 | protocol discriminator, transaction identifier |
| 0x21 | message type |
| 0x00 | page mode normal |
| 0x00 | not empty mobile identity |

*/

BEGINARRAY (PAGING_REQ_1, 9)
	0xB0, 0x00,
	0x08, 0x00,
	0x19,
	0x06,
	0x21,
	0x00,
	0x00
ENDARRAY

/*

| 0xB0,0x00 | length in bits |
| --- | --- |
| 0x08, 0x00 | offset in bits |
| 0x19 | l2 pseudo length (= 6 Byte) |
| 0x06 | protocol discriminator, transaction identifier |
| 0x21 | message type |
| 0x01 | page mode extended |
| 0x00 | not empty mobile identity |

*/

BEGINARRAY (PAGING_REQ_1_EXT, 9)
	0xB0, 0x00,
	0x08, 0x00,
	0x19,
	0x06,
	0x21,
	0x01,
	0x00
ENDARRAY

/*

| 0xB0,0x00 | length in bits |
| --- | --- |
| 0x08, 0x00 | offset in bits |
| 0x19 | l2 pseudo length (= 6 Byte) |
| 0x06 | protocol discriminator, transaction identifier |
| 0x21 | message type |
| 0x02 | page mode reorganization |
| 0x00 | empty mobile identity |

*/

BEGINARRAY (PAGING_REQ_1_REO, 9)
	0xB0, 0x00,
	0x08, 0x00,
	0x19,
	0x06,
	0x21,
	0x02,
	0x00
ENDARRAY

/*

| 0xB0,0x00 | length in bits |
| --- | --- |
| 0x08, 0x00 | offset in bits |
| 0x19 | l2 pseudo length (= 6 Byte) |
| 0x06 | protocol discriminator, transaction identifier |
| 0x21 | message type |
| 0x03 | page mode same as before |
| 0x00 | empty mobile identity |

*/

BEGINARRAY (PAGING_REQ_1_SAB, 9)
	0xB0, 0x00,
	0x08, 0x00,
	0x19,
	0x06,
	0x21,
	0x03,
	0x00
ENDARRAY

/*

| 0x11 | l2 pseudo length (= 4 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x21 | message type |
| 0x00 | page mode normal |
| 0x00 | empty mobile identity |

*/

BEGIN_PSTRUCT("l2_frame", L2_PAG_1_EMPTY)
	SET_COMP("content", L2_PAG_1_EMPTY_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_EMPTY_ARRAY, 23)
	0x11,
	0x06,
	0x21,
	0x00,
	0x00,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x31 | l2 pseudo length (= 12 Bytes) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x21 | message type |
| 0x00 | page mode normal |
| 0x09 | mobile identity 1 |

*/

BEGIN_PSTRUCT("l2_frame", L2_PAG_1_I1_A)
	SET_COMP("content", L2_PAG_1_I1_A_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_I1_A_ARRAY, 23)
	0x31,
	0x06,
	0x21,
	0x00,
	0x08,
	0x09, 0x10, 0x10, 0x83, 0x32,
	0x03, 0x95, 0x54, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x59 | l2 pseudo length (= 22 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x00 | Page mode normal |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT("l2_frame", L2_PAG_1_I2_A)
	SET_COMP("content", L2_PAG_1_I2_A_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_I2_A_ARRAY, 23)
	0x59,
	0x06,
	0x21,
	0x00,
	0x08,
	0x09, 0x10, 0x10, 0x83, 0x23,
	0x03, 0x95, 0x54, 0x17, 0x08,
	0x09, 0x10, 0x10, 0x83, 0x32,
	0x03, 0x95, 0x54
ENDARRAY

/*

| 0x1D | l2 pseudo length (= 7 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x23 | normal page mode, TCH F |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT("l2_frame", L2_PAG_1_T1_T)
	SET_COMP("content", L2_PAG_1_T1_T_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_T1_T_ARRAY, 23)
	0x1D,
	0x06,
	0x21,
	0x20,
	0x05,
	0xF4, 0x05, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x41 | l2 pseudo length (= 16 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x80 | normal page mode , TCH F |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT("l2_frame", L2_PAG_1_T2_T)
	SET_COMP("content", L2_PAG_1_T2_T_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_T2_T_ARRAY, 23)
	0x41,
	0x06,
	0x21,
	0x80,
	0x05,
	0xF4, 0x06, 0x22, 0x33, 0x66,
	0x17, 0x05, 0xF4, 0x05, 0x22,
	0x33, 0x66, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x31 | l2 pseudo length (= 12 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x01 | page mode normal, SDCCH |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_I1_S)

SET_COMP ("content", L2_PAG_1_I1_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_I1_S_ARRAY, 23)
	0x31,
	0x06,
	0x21,
	0x10,
	0x08,
	0x09, 0x10, 0x10, 0x83, 0x32,
	0x03, 0x95, 0x54, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x59 | l2 pseudo length (= 22 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x04 | Page mode normal, SDCCH |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_I2_S)

SET_COMP ("content", L2_PAG_1_I2_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_I2_S_ARRAY, 23)
	0x59,
	0x06,
	0x21,
	0x40,
	0x08,
	0x09, 0x10, 0x10, 0x83, 0x23,
	0x03, 0x95, 0x54, 0x17, 0x08,
	0x09, 0x10, 0x10, 0x83, 0x32,
	0x03, 0x95, 0x54
ENDARRAY

/*

| 0x1D | l2 pseudo length (= 7 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x30 | page mode normal, Dual |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_T1_D)

SET_COMP ("content", L2_PAG_1_T1_D_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_T1_D_ARRAY, 23)
	0x1D,
	0x06,
	0x21,
	0x30,
	0x05,
	0xF4, 0x05, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x41 | l2 pseudo length (= 16 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0xC0 | page mode normal, Dual |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_T2_D)

SET_COMP ("content", L2_PAG_1_T2_D_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_T2_D_ARRAY, 23)
	0x41,
	0x06,
	0x21,
	0xC0,
	0x05,
	0xF4, 0x06, 0x22, 0x33, 0x66,
	0x17, 0x05, 0xF4, 0x05, 0x22,
	0x33, 0x66, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x31 | l2 pseudo length (= 12 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x01 | page mode normal, SDCCH |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_WI1)

SET_COMP ("content", L2_PAG_1_WI1_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_WI1_ARRAY, 23)
	0x31,
	0x06,
	0x21,
	0x10,
	0x08,
	0x09, 0x12, 0x10, 0x83, 0x32,
	0x03, 0x95, 0x54, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x59 | l2 pseudo length (= 22 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x04 | Page mode normal, SDCCH |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_WI2)

SET_COMP ("content", L2_PAG_1_WI2_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_WI2_ARRAY, 23)
	0x59,
	0x06,
	0x21,
	0x40,
	0x08,
	0x09, 0x10, 0x10, 0x83, 0x23,
	0x03, 0x95, 0x54, 0x17, 0x08,
	0x09, 0x12, 0x10, 0x83, 0x32,
	0x03, 0x95, 0x54
ENDARRAY

/*

| 0x1D | l2 pseudo length (= 7 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x30 | page mode normal, Dual |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_WT1)

SET_COMP ("content", L2_PAG_1_WT1_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_WT1_ARRAY, 23)
	0x1D,
	0x06,
	0x21,
	0x30,
	0x05,
	0xF4, 0x06, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x41 | l2 pseudo length (= 16 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0xC0 | page mode normal, Dual |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_WT2)

SET_COMP ("content", L2_PAG_1_WT2_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_WT2_ARRAY, 23)
	0x41,
	0x06,
	0x21,
	0xC0,
	0x05,
	0xF4, 0x06, 0x22, 0x33, 0x66,
	0x17, 0x05, 0xF4, 0x05, 0x23,
	0x33, 0x66, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x31 | l2 pseudo length (= 12 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x01 | page mode normal, SDCCH |
| 0x0B | Mobile identity 1 (unknown type) |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_WTYPE)

SET_COMP ("content", L2_PAG_1_WTYPE_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_WTYPE_ARRAY, 23)
	0x31,
	0x06,
	0x21,
	0x10,
	0x08,
	0x0B, 0x12, 0x10, 0x83, 0x32,
	0x03, 0x95, 0x54, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x19 | l2 pseudo length (= 6 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0xC0 | page mode normal, any channel |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_ST1_A1)

SET_COMP ("content", L2_PAG_1_ST1_A1_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_ST1_A1_ARRAY, 23)
	0x19,
	0x06,
	0x21,
	0x00,
	0x02,
	0xF4, 142, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x2D | l2 pseudo length (= 11 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x80 | page mode normal, TCH F |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_ST2_T3)

SET_COMP ("content", L2_PAG_1_ST2_T3_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_ST2_T3_ARRAY, 23)
	0x2D,
	0x06,
	0x21,
	0x80,
	0x02,
	0xF4, 143, 0x17, 0x03, 0xF4,
	0x00, 142, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x29 | l2 pseudo length (= 10 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0xC0 | page mode normal, Dual |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_SI1_S)

SET_COMP ("content", L2_PAG_1_SI1_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_SI1_S_ARRAY, 23)
	0x29,
	0x06,
	0x21,
	0x10,
	0x06,
	0x01, 0x10, 0x10, 0x83, 0x32,
	0xF3, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x39 | l2 pseudo length (= 14 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0xC0 | page mode normal, Dual |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_SI2_D)

SET_COMP ("content", L2_PAG_1_SI2_D_ARRAY)

ENDSTRUCT

BEGINARRAY(L2_PAG_1_SI2_D_ARRAY, 23)
	0x39,
	0x06,
	0x21,
	0xC0,
	0x02,
	0xF4, 143, 0x17, 0x06, 0x01,
	0x10, 0x10, 0x83, 0x32, 0xF3,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x1D | l2 pseudo length (= 7 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x00 | page mode normal, any channel |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_ST1_A2)

SET_COMP ("content", L2_PAG_1_ST1_A2_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_ST1_A2_ARRAY, 23)
	0x1D,
	0x06,
	0x21,
	0x00,
	0x03,
	0xF4, 0, 142, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B
ENDARRAY

/*

| 0x35 | l2 pseudo length (= 13 Bytes) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x21 | Message type |
| 0x80 | page mode normal, TCH F |
| 0x09 | Mobile identity 1 |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_1_ST2_T4)

SET_COMP ("content", L2_PAG_1_ST2_T4_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_1_ST2_T4_ARRAY, 23)
	0x35,
	0x06,
	0x21,
	0x80,
	0x02,
	0xF4, 143, 0x17, 0x05, 0xF4,
	0x00, 0x00, 0x00, 142, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x11 | l2 pseudo length (= 4 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x22 | message type |
| 0x00 | page mode normal |
| 0x00 | empty mobile identity |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_EMPTY)

SET_COMP ("content", L2_PAG_2_EMPTY_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_EMPTY_ARRAY, 23)
	0x11,
	0x06,
	0x22,
	0x00,
	0x00,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x1D | l2 pseudo length (= 7 Byte) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x22 | Message type |
| 0x00 | page mode normal |
| 0x00 | Mobile identity |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T1_A)

SET_COMP ("content", L2_PAG_2_T1_A_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T1_A_ARRAY, 23)
	0x1D,
	0x06,
	0x22,
	0x00,
	0x05, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T1_S)

SET_COMP ("content", L2_PAG_2_T1_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T1_S_ARRAY, 23)
	0x1D,
	0x06,
	0x22,
	0x10,
	0x05, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T1_T)

SET_COMP ("content", L2_PAG_2_T1_T_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T1_T_ARRAY, 23)
	0x1D,
	0x06,
	0x22,
	0x20,
	0x05, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T1_D)

SET_COMP ("content", L2_PAG_2_T1_D_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T1_D_ARRAY, 23)
	0x1D,
	0x06,
	0x22,
	0x30,
	0x05, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x2D | l2 pseudo length (= 11 Byte) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x22 | Message type |
| 0x00 | page mode normal |
| 0x00 | Mobile identity |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T2_A)

SET_COMP ("content", L2_PAG_2_T2_A_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T2_A_ARRAY, 23)
	0x2D,
	0x06,
	0x22,
	0x00,
	0x06, 0x22, 0x33, 0x66,
	0x05, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T2_S)

SET_COMP ("content", L2_PAG_2_T2_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T2_S_ARRAY, 23)
	0x2D,
	0x06,
	0x22,
	0x40,
	0x05, 0x23, 0x33, 0x66,
	0x05, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T2_T)

SET_COMP ("content", L2_PAG_2_T2_T_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T2_T_ARRAY, 23)
	0x2D,
	0x06,
	0x22,
	0x80,
	0x05, 0x22, 0x31, 0x66,
	0x05, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T2_D)

SET_COMP ("content", L2_PAG_2_T2_D_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T2_D_ARRAY, 23)
	0x2D,
	0x06,
	0x22,
	0xC0,
	0x05, 0x22, 0x33, 0x65,
	0x05, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_WRONG)

SET_COMP ("content", L2_PAG_2_WRONG_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_WRONG_ARRAY, 23)
	0x2D,
	0x06,
	0x22,
	0x30,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B
ENDARRAY

/*

| 0x55 | l2 pseudo length (= 11 Byte) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x22 | Message type |
| 0x00 | page mode normal |
| 0x00 | Mobile identity |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_I3_A)

SET_COMP ("content", L2_PAG_2_I3_A_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_I3_A_ARRAY, 23)
	0x55,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x08, 0x09, 0x10, 0x10,
	0x83, 0x32, 0x03, 0x95, 0x54,
	0x8B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_I3_S)

SET_COMP ("content", L2_PAG_2_I3_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_I3_S_ARRAY, 23)
	0x55,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x08, 0x09, 0x10, 0x10,
	0x83, 0x32, 0x03, 0x95, 0x54,
	0xAB
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_I3_T)

SET_COMP ("content", L2_PAG_2_I3_T_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_I3_T_ARRAY, 23)
	0x55,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x08, 0x09, 0x10, 0x10,
	0x83, 0x32, 0x03, 0x95, 0x54,
	0xCB
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_I3_D)

SET_COMP ("content", L2_PAG_2_I3_D_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_I3_D_ARRAY, 23)
	0x55,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x08, 0x09, 0x10, 0x10,
	0x83, 0x32, 0x03, 0x95, 0x54,
	0xEB
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_I3_N)

SET_COMP ("content", L2_PAG_2_I3_N_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_I3_N_ARRAY, 23)
	0x55,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x08, 0x09, 0x10, 0x10,
	0x83, 0x32, 0x03, 0x95, 0x54,
	0x2B
ENDARRAY

/*

| 0x49 | l2 pseudo length (= 18 Byte) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x22 | Message type |
| 0x00 | Page mode normal |
| 0x00 | Mobile identity |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T3_A)

SET_COMP ("content", L2_PAG_2_T3_A_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T3_A_ARRAY, 23)
	0x49,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x05, 0xF4, 0x05, 0x22,
	0x33, 0x66, 0x8B, 0x2B, 0x2B,
	0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T3_S)

SET_COMP ("content", L2_PAG_2_T3_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T3_S_ARRAY, 23)
	0x49,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x05, 0xF4, 0x05, 0x22,
	0x33, 0x66, 0xAB, 0x2B, 0x2B,
	0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T3_T)

SET_COMP ("content", L2_PAG_2_T3_T_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T3_T_ARRAY, 23)
	0x49,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x05, 0xF4, 0x05, 0x22,
	0x33, 0x66, 0xCB, 0x2B, 0x2B,
	0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T3_D)

SET_COMP ("content", L2_PAG_2_T3_D_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T3_D_ARRAY, 23)
	0x49,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x05, 0xF4, 0x05, 0x22,
	0x33, 0x66, 0xEB, 0x2B, 0x2B,
	0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_T3_N)

SET_COMP ("content", L2_PAG_2_T3_N_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_T3_N_ARRAY, 23)
	0x49,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x05, 0xF4, 0x05, 0x22,
	0x33, 0x66, 0x2B, 0x2B, 0x2B,
	0x2B
ENDARRAY

/*

| 0x4D | l2 pseudo length (= 19 Byte) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x22 | Message type |
| 0x00 | page mode normal |
| 0x00 | Mobile identity |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_SI3_A)

SET_COMP ("content", L2_PAG_2_SI3_A_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_SI3_A_ARRAY, 23)
	0x4D,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x06, 0x01, 0x10, 0x10,
	0x83, 0x32, 0xF3, 0x8B, 0x2B,
	0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_ST3_S)

SET_COMP ("content", L2_PAG_2_ST3_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_ST3_S_ARRAY, 23)
	0x3D,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x02, 0xF4, 142, 0xAB,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_ST3_T)

SET_COMP ("content", L2_PAG_2_ST3_T_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_ST3_T_ARRAY, 23)
	0x41,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x03, 0xF4, 0, 142, 0xCB,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_ST3_D)

SET_COMP ("content", L2_PAG_2_ST3_D_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_ST3_D_ARRAY, 23)
	0x45,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x04, 0xF4, 0,0,142, 0xEB,
	0x2B, 0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_2_ST3_N)

SET_COMP ("content", L2_PAG_2_ST3_N_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_2_ST3_N_ARRAY, 23)
	0x49,
	0x06,
	0x22,
	0x00,
	0x01, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x17, 0x05, 0xF4, 0, 0,0,142, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x11 | l2 pseudo length (= 4 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x24 | message type |
| 0x00 | page mode normal |
| 0x00 | empty mobile identity |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_EMPTY)

SET_COMP ("content", L2_PAG_3_EMPTY_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_EMPTY_ARRAY, 23)
	0x11,
	0x06,
	0x24,
	0x00,
	0x00,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B, 0x2B, 0x2B,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x4D | l2 pseudo length (= 19 Byte) |
| --- | --- |
| 0x06 | Protocol discriminator, transaction identifier |
| 0x24 | Message type |
| 0x00 | Page mode normal |
| 0x00 | Mobile identity |

*/

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T1_A)

SET_COMP ("content", L2_PAG_3_T1_A_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T1_A_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x00,
	0x05, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x03, 0x22, 0x33, 0x65,
	0x02, 0x22, 0x33, 0x65,
	0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T1_S)

SET_COMP ("content", L2_PAG_3_T1_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T1_S_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x10,
	0x05, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x03, 0x22, 0x33, 0x65,
	0x02, 0x22, 0x33, 0x65,
	0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T1_T)

SET_COMP ("content", L2_PAG_3_T1_T_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T1_T_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x20,
	0x05, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x03, 0x22, 0x33, 0x65,
	0x02, 0x22, 0x33, 0x65,
	0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T1_D)

SET_COMP ("content", L2_PAG_3_T1_D_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T1_D_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x30,
	0x05, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x03, 0x22, 0x33, 0x65,
	0x02, 0x22, 0x33, 0x65,
	0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T2_A)

SET_COMP ("content", L2_PAG_3_T2_A_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T2_A_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x00,
	0x04, 0x22, 0x33, 0x66,
	0x05, 0x22, 0x33, 0x66,
	0x03, 0x22, 0x33, 0x65,
	0x02, 0x22, 0x33, 0x65,
	0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T2_S)

SET_COMP ("content", L2_PAG_3_T2_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T2_S_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x40,
	0x04, 0x22, 0x33, 0x66,
	0x05, 0x22, 0x33, 0x66,
	0x03, 0x22, 0x33, 0x65,
	0x02, 0x22, 0x33, 0x65,
	0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T2_T)

SET_COMP ("content", L2_PAG_3_T2_T_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T2_T_ARRAY, 23)

0x4D,
	0x06,
	0x24,
	0x80,
	0x04, 0x22, 0x33, 0x66,
	0x05, 0x22, 0x33, 0x66,
	0x03, 0x22, 0x33, 0x65,
	0x02, 0x22, 0x33, 0x65,
	0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T2_D)

SET_COMP ("content", L2_PAG_3_T2_D_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T2_D_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0xC0,
	0x04, 0x22, 0x33, 0x66,
	0x05, 0x22, 0x33, 0x66,
	0x03, 0x22, 0x33, 0x65,
	0x02, 0x22, 0x33, 0x65,
	0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T3_A)

SET_COMP ("content", L2_PAG_3_T3_A_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T3_A_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x00,
	0x04, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x05, 0x22, 0x33, 0x66,
	0x02, 0x22, 0x33, 0x65,
	0x83, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T3_S)

SET_COMP ("content", L2_PAG_3_T3_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T3_S_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x00,
	0x04, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x05, 0x22, 0x33, 0x66,
	0x02, 0x22, 0x33, 0x65,
	0xA3, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T3_T)

SET_COMP ("content", L2_PAG_3_T3_T_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T3_T_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x00,
	0x04, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x05, 0x22, 0x33, 0x66,
	0x02, 0x22, 0x33, 0x65,
	0xC3, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T3_D)

SET_COMP ("content", L2_PAG_3_T3_D_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T3_D_ARRAY, 23)

0x4D,
	0x06,
	0x24,
	0x00,
	0x04, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x05, 0x22, 0x33, 0x66,
	0x02, 0x22, 0x33, 0x65,
	0xE3, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T3_N)

SET_COMP ("content", L2_PAG_3_T3_N_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T3_N_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x10,
	0x04, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x05, 0x22, 0x33, 0x66,
	0x02, 0x22, 0x33, 0x65,
	0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T4_A)

SET_COMP ("content", L2_PAG_3_T4_A_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T4_A_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x00,
	0x04, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x04, 0x22, 0x33, 0x67,
	0x05, 0x22, 0x33, 0x66,
	0xA3, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T4_S)

SET_COMP ("content", L2_PAG_3_T4_S_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T4_S_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x00,
	0x04, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x04, 0x22, 0x33, 0x67,
	0x05, 0x22, 0x33, 0x66,
	0x8B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T4_T)

SET_COMP ("content", L2_PAG_3_T4_T_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T4_T_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x00,
	0x04, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x04, 0x22, 0x33, 0x67,
	0x05, 0x22, 0x33, 0x66,
	0x93, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T4_D)

SET_COMP ("content", L2_PAG_3_T4_D_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T4_D_ARRAY, 23)

0x4D,
	0x06,
	0x24,
	0x00,
	0x04, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x04, 0x22, 0x33, 0x67,
	0x05, 0x22, 0x33, 0x66,
	0x9B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_T4_N)

SET_COMP ("content", L2_PAG_3_T4_N_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_T4_N_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x50,
	0x04, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x04, 0x22, 0x33, 0x67,
	0x05, 0x22, 0x33, 0x66,
	0x2B, 0x2B, 0x2B
ENDARRAY

BEGIN_PSTRUCT ("l2_frame", L2_PAG_3_WRONG)

SET_COMP ("content", L2_PAG_3_WRONG_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_PAG_3_WRONG_ARRAY, 23)
	0x4D,
	0x06,
	0x24,
	0x50,
	0x04, 0x22, 0x33, 0x66,
	0x04, 0x22, 0x33, 0x65,
	0x04, 0x22, 0x33, 0x67,
	0x02, 0x22, 0x33, 0x65,
	0x2B, 0x2B, 0x2B
ENDARRAY

/*

| 0x31 | l2 pseudo length (= 12 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x3F | message type |
| 0x00 | page mode |
| 0x2B, .. | channel description |
| 0xA0 | request reference |
| 0x1B | timing advance |
| 0x01, 0x16 | mobile allocation |

*/

BEGIN_PSTRUCT ("l2_frame", L2_IMM_ASS)

SET_COMP ("content", L2_IMM_ASS_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_IMM_ASS_ARRAY, 13)
	0x31,
	0x06,
	0x3F,
	0x00,
	0x2B, 0x54, 0x94,
	0xFB, 0x21, 0x06,
	0x1B,
	0x01, 0x16
ENDARRAY

/*

| 0x49 | l2 pseudo length (= 18 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x39 | message type |
| 0x02 | page mode |
| 0x20, .. | channel description |
| 0x00 | request reference |
| 0x00 | timing advance |
| 0x01, 0x16 | mobile allocation |

*/

BEGIN_PSTRUCT ("l2_frame", L2_IMM_ASS_EXT_REO)

SET_COMP ("content", L2_IMM_ASS_EXT_REO_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_IMM_ASS_EXT_REO_ARRAY, 23)
	0x49,
	0x06,
	0x39,
	0x02,
	0x20, 0xA0, 0x14,
	0x00, 0x00, 0x00,
	0x00,
	0x20, 0xA0, 0x14,
	0x00, 0x00, 0x00,
	0x00,
	0x00,
	0x2b,0x2b,0x2b,0x2b
ENDARRAY

/*

| 0x31 | l2 pseudo length (= 12 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x3F | message type |
| 0x00 | page mode |
| 0x40, 0x10, 0x40 | channel description (hopping) |
| 0xA0 | request reference (???) |
| 0x1E | timing advance (30 bits) |
| 0x01, 0x16 | mobile allocation (00010110) (->Freq 1,2,4) (0x01 = length of ma) |
| 0x2B,.. | spare bits |
| 0x2B,.. | spare bits |

*/

BEGIN_PSTRUCT ("l2_frame", L2_IMM_ASS_HOP)

SET_COMP ("content", L2_IMM_ASS_HOP_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_IMM_ASS_HOP_ARRAY, 23)
	0x31,
	0x06,
	0x3F,
	0x00,
	0x40, 0x10, 0x40,
	0xFB, 0x21, 0x06,
	0x1E,
	0x01, 0x16,
	0x2B,0x2B,0x2B,0x2B,0x2B,
	0x2B,0x2B,0x2B,0x2B,0x2B
ENDARRAY

/*

| 0xB0,0x00 | length in bits (184/8 = 23 bytes) |
| --- | --- |
| 0x08, 0x00 | offset in bits |
| 0x31 | l2 pseudo length (= 12 Byte) |
| 0x06 | protocol discriminator, transaction identifier |
| 0x3F | message type |
| 0x00 | page mode |
| 0x40, .. | channel description |
| 0xFB, .. | request reference |
| 0x1E | timing advance |
| 0x16 | mobile allocation |
| 0x2B,.. | spare bits |
| 0x2B,.. | spare bits |

*/

BEGIN_MSTRUCT ("page_mode", PAGE_MODE_1)

SET_COMP ("pm", PAGING_NORM)

ENDSTRUCT

BEGIN_MSTRUCT ("chan_desc", CHAN_DESC_1)

SET_COMP ("chan_type", SDCCH_8_S0)

SET_COMP ("tn", 0)

SET_COMP ("tsc", 0)

SET_COMP ("hop", HOP_YES)

SKIP_COMP ("arfcn")

SET_COMP ("maio", 1)
	SET_COMP ("hsn", 0)

ENDSTRUCT

BEGIN_MSTRUCT ("pck_chan_desc", PCK_CHAN_DESC_1)

SET_COMP ("pck_chan_type",1)

SET_COMP ("tn",1)

SET_COMP ("tsc",5)

SET_COMP ("hop", 0)

SKIP_COMP ("indir")

SKIP_COMP ("arfcn")

SKIP_COMP ("maio")
	SKIP_COMP ("ma_num")
	SKIP_COMP ("flag")

SKIP_COMP ("ch_mark1")
	SKIP_COMP ("hsn")

ENDSTRUCT

BEGIN_MSTRUCT ("req_ref", REQ_REF_1)

SET_COMP ("ra", 0xFB)

SET_COMP ("t1", 4)

SET_COMP ("t3", 8)

SET_COMP ("t2", 6)

ENDSTRUCT

BEGIN_MSTRUCT ("time_advance", TIME_ADVANCE_1)

SET_COMP ("ta", 0x1E)

ENDSTRUCT

BEGIN_MSTRUCT ("mob_alloc", MOB_ALLOC_1)

SET_COMP ("mac", MAC_1)

ENDSTRUCT

BEGINARRAY_PART (MAC_1, 1)

0x16

ENDARRAY

/*

| 0x31 | l2 pseudo length (= 12 Byte) |
| --- | --- |
| 0x06 | protocol discriminator, transaction identifier |
| 0x3F | message type |
| 0x00 | page mode |
| 0x2B, .. | any content |

*/

BEGIN_PSTRUCT ("l2_frame", L2_IMM_ASS_REJ)

SET_COMP ("content", L2_IMM_ASS_REJ_ARRAY)

ENDSTRUCT

BEGINARRAY (L2_IMM_ASS_REJ_ARRAY, 13)
	0x31,
	0x06,
	0x3F,
	0x00,
	0x2B, 0x54, 0x94,
	0xFB, 0x21, 0x06,
	0x1B,
	0x01, 0x16
ENDARRAY

/*

| 0xB0,0x00 | length in bits |
| --- | --- |
| 0x08, 0x00 | offset in bits |
| 0x31 | l2 pseudo length (= 12 Byte) |
| 0x06 | protocol discriminator, transaction identifier |
| 0x3F | message type |
| 0x00 | page mode |
| 0x2B, .. | any content |

*/

BEGIN_MSTRUCT ("chan_desc", CHAN_DESC_2)

SET_COMP ("chan_type", SDCCH_4_S1)

SET_COMP ("tn", 3)

SET_COMP ("tsc", 2)

SET_COMP ("hop", HOP_YES)

SKIP_COMP ("arfcn")

SET_COMP ("maio", 18)
	SET_COMP ("hsn", 20)

ENDSTRUCT

BEGIN_MSTRUCT ("time_advance", TIME_ADVANCE_2)

SET_COMP ("ta", 0x1B)

ENDSTRUCT

/*

| 14, 0 | ncell 14 |
| --- | --- |
| 124, 0 | ncell 124 |
| 0xFF, 0xFF | end of actual description list |

*/

BEGIN_SHORT_ARRAY (CHLIST_14_124_FFFF, 3)
	14,
	124,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 0, 2 | ncell 512 |
| --- | --- |
| 0x75, 3 | ncell 885 |
| 0xFF, 0xFF | end of actual description list |

*/

BEGIN_SHORT_ARRAY (CHLIST_512_885_FFFF, 3)
	ARFCN_512,
	ARFCN_885,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 0, 2 | ncell 512 |
| --- | --- |
| 0x2a, 3 | ncell 810 |
| 0xFF, 0xFF | end of actual description list |

*/

BEGIN_SHORT_ARRAY (CHLIST_512_810_FFFF, 3)
	ARFCN_512,
	ARFCN_810,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 14, 0 | ncell 14 |
| --- | --- |
| 0,2 | ncell 512 |
| 0x7d,2 | ncell 637 |
| 0x75, 3 | ncell 885 |
| 0xFF, 0xFF | end of actual description list |

*/

BEGIN_SHORT_ARRAY (CHLIST_14_512_637_885_FFFF, 5)
	ARFCN_14,
	ARFCN_512,
	ARFCN_637,
	ARFCN_885,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 1, 0 | ncell 1 |
| --- | --- |
| 14, 0 | ncell 14 |
| 124, 0 | ncell 124 |
| 0xFF, 0xFF | end of neighbour cell list |

*/

BEGIN_SHORT_ARRAY_PART (CHLIST_1_14_124_FFFF, 4)
	ARFCN_1,
	ARFCN_14,
	ARFCN_124,
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_1_14_23_124_FFFF)

SET_COMP("radio_freq", RF_1_14_23_124_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_1_14_23_124_FFFF, 5)
	ARFCN_1,
	ARFCN_14,
	ARFCN_23,
	ARFCN_124,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 23, 0 | serving cell 23 |
| --- | --- |
| 1, 0 | ncell 1 |
| 14, 0 | ncell 14 |
| 124,0 | ncell 124 |

*/

BEGIN_PSTRUCT ("chan_list", CHLIST_23_1_14_124)

SET_COMP("radio_freq", RF_23_1_14_124)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_23_1_14_124, 4)
	ARFCN_23,
	ARFCN_1,
	ARFCN_14,
	ARFCN_124
ENDARRAY

/*

| 23, 0 | Ncell 23 |
| --- | --- |
| 1, 0 | Ncell 1 |
| 124, 0 | Ncell 124 |
| 0xFF, 0xFF | end of neighbour cell list |

*/

BEGIN_SHORT_ARRAY_PART (CHLIST_23_1_124_FFFF, 4)
	ARFCN_23,
	ARFCN_1,
	ARFCN_124,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 0, 2 | ncell 512 |
| --- | --- |
| 0x44, 2 | ncell 580 |
| 0x75, 3 | ncell 885 |
| 0xFF, 0xFF | end of neighbour cell list |

*/

BEGIN_SHORT_ARRAY_PART (CHLIST_512_580_885_FFFF, 4)
	ARFCN_512,
	ARFCN_580,
	ARFCN_885,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 0x7D, 2 | serving cell 637 |
| --- | --- |
| 0, 2 | ncell 512 |
| 0x44, 2 | ncell 580 |
| 0x75, 3 | ncell 885 |

*/

BEGIN_PSTRUCT ("chan_list", CHLIST_637_512_580_885)

SET_COMP ("radio_freq", RF_637_512_580_885)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_637_512_580_885, 4)
	ARFCN_637,

ARFCN_512,
	ARFCN_580,
	ARFCN_885
ENDARRAY

/*

| 0,2 | ncell 518 |
| --- | --- |
| 0x7d,2 | ncell 637 |
| 0x75,3 | ncell 885 |
| 0xFF, 0xFF | end of neighbour cell list |

*/

BEGIN_SHORT_ARRAY_PART (CHLIST_512_637_885_FFFF, 4)
	ARFCN_512,
	ARFCN_637,
	ARFCN_885,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 0, 2 | ncell 512 |
| --- | --- |
| 0x44, 2 | ncell 580 |
| 0x2a, 3 | ncell 810 |
| 0xFF, 0xFF | end of neighbour cell list |

*/

BEGIN_PSTRUCT ("chan_list", CHLIST_512_580_810_FFFF)

SET_COMP("radio_freq", RF_512_580_810_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_512_580_810_FFFF, 4)
	ARFCN_512,
	ARFCN_580,
	ARFCN_810,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 0, 2 | ncell 512 |
| --- | --- |
| 0x44, 2 | ncell 580 |
| 0x2a, 3 | ncell 810 |
| 0x7D, 2 | serving cell 637 |

*/

BEGIN_PSTRUCT ("chan_list", CHLIST_512_580_810_637)

SET_COMP("radio_freq", RF_512_580_810_637)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_512_580_810_637, 4)
	ARFCN_512,
	ARFCN_580,
	ARFCN_810,
	ARFCN_637
ENDARRAY

/*

| 0,2 | ncell 518 |
| --- | --- |
| 0x7d,2 | ncell 637 |
| 0x2a,3 | ncell 810 |
| 0xFF, 0xFF | end of neighbour cell list |

*/

BEGIN_PSTRUCT ("chan_list", CHLIST_512_637_810_FFFF)

SET_COMP("radio_freq", RF_512_637_810_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_512_637_810_FFFF, 4)
	ARFCN_512,
	ARFCN_637,
	ARFCN_810,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 1, 0 | ncell 1 |
| --- | --- |
| 14, 0 | ncell 14 |
| 124, 0 | ncell 124 |
| 0,2 | ncell 512 |
| 0x44, 2 | ncell 580 |
| 0x7d,2 | ncell 637 |
| 0x75, 3 | ncell 885 |
| 0xFF, 0xFF | end of neighbour cell list |

*/

BEGIN_SHORT_ARRAY_PART (CHLIST_1_14_124_512_580_637_885_FFFF, 8)
	ARFCN_1,
	ARFCN_14,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_637,
	ARFCN_885,
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_1_14_124_512_580_637_1023_FFFF)

SET_COMP("radio_freq", RF_1_14_124_512_580_637_1023_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_1_14_124_512_580_637_1023_FFFF, 8)
	ARFCN_1,
	ARFCN_14,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_637,
	ARFCN_1023,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 1, 0 | ncell 1 |
| --- | --- |
| 14, 0 | ncell 14 |
| 124, 0 | ncell 124 |
| 0,2 | ncell 512 |
| 0x44, 2 | ncell 580 |
| 0x7d,2 | ncell 637 |
| 0x75, 3 | ncell 885 |
| 23,0 | serving cell 23 |

*/

BEGIN_PSTRUCT ("chan_list", CHLIST_23_1_14_124_512_580_637_885)

SET_COMP("radio_freq", RF_23_1_14_124_512_580_637_885)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_23_1_14_124_512_580_637_885, 8)
	ARFCN_23,
	ARFCN_1,
	ARFCN_14,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_637,
	ARFCN_885

ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_1_14_124_512_580_637_1023_0)

SET_COMP("radio_freq", RF_1_14_124_512_580_637_1023_0)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_1_14_124_512_580_637_1023_0, 8)
	ARFCN_1,
	ARFCN_14,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_637,
	ARFCN_1023,
	ARFCN_0
ENDARRAY

/*

| 1, 0 | ncell 1 |
| --- | --- |
| 14, 0 | ncell 14 |
| 23, 0 | ncell 23 |
| 124, 0 | ncell 124 |
| 0,2 | ncell 512 |
| 0x44, 2 | ncell 580 |
| 0x75, 3 | ncell 885 |
| 0xFF, 0xFF | end of neighbour cell list |

*/

BEGIN_SHORT_ARRAY_PART (CHLIST_1_14_23_124_512_580_885_FFFF, 8)
	ARFCN_1,
	ARFCN_14,
	ARFCN_23,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_885,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 1, 0 | ncell 1 |
| --- | --- |
| 14, 0 | ncell 14 |
| 0, 0 | ncell 0 |
| 124, 0 | ncell 124 |
| 0,2 | ncell 512 |
| 0x44, 2 | ncell 580 |
| 0xFF, 3 | ncell 1023 |
| 0xFF, 0xFF | end of neighbour cell list |

*/

BEGIN_PSTRUCT ("chan_list", CHLIST_1_14_0_124_512_580_1023_FFFF)

SET_COMP("radio_freq", RF_1_14_0_124_512_580_1023_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_1_14_0_124_512_580_1023_FFFF, 8)
	ARFCN_1,
	ARFCN_14,
	ARFCN_0,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_1023,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 0x7d, 2 | serving cell 637 |
| --- | --- |
| 1, 0 | ncell 1 |
| 14, 0 | ncell 14 |
| 23, 0 | ncell 23 |
| 124, 0 | ncell 124 |
| 0,2 | ncell 512 |
| 0x44, 2 | ncell 580 |
| 0x75, 3 | ncell 885 |

*/

BEGIN_PSTRUCT ("chan_list", CHLIST_637_1_14_23_124_512_580_885)

SET_COMP ("radio_freq", RF_637_1_14_23_124_512_580_885)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_637_1_14_23_124_512_580_885, 8)
	ARFCN_637,
	ARFCN_1, 
	ARFCN_14, 
	ARFCN_23,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_885
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_1_14_0_124_512_580_1023_975)

SET_COMP("radio_freq", RF_1_14_0_124_512_580_1023_975)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_1_14_0_124_512_580_1023_975, 8)
	ARFCN_1,
	ARFCN_14,
	ARFCN_0,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_1023,
	ARFCN_975
ENDARRAY

BEGIN_SHORT_ARRAY_PART (EMPTY_NCELL_LIST,1)
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_10_20_40_80_90_100_110_120_FFFF)

SET_COMP("radio_freq", RF_10_20_40_80_90_100_110_120_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_10_20_40_80_90_100_110_120_FFFF, 9)
	ARFCN_10,
	ARFCN_20,
	ARFCN_40,
	ARFCN_80,
	ARFCN_90, 
	ARFCN_100,
	ARFCN_110,
	ARFCN_120,
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_SHORT_ARRAY_PART (CHLIST_1_15_FFFF, 3)
	ARFCN_1,
	ARFCN_15,
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_SHORT_ARRAY_PART (CHLIST_2_30_FFFF, 3)
	ARFCN_2,
	ARFCN_30,
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_10_20_40_80_90_100_110_120)

SET_COMP("radio_freq", RF_10_20_40_80_90_100_110_120)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_10_20_40_80_90_100_110_120, 8)
	ARFCN_10,
	ARFCN_20,
	ARFCN_40,
	ARFCN_80,
	ARFCN_90, 
	ARFCN_100,
	ARFCN_110,
	ARFCN_120
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_1_14_15)

SET_COMP("radio_freq", RF_1_14_15)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_1_14_15, 3)
	ARFCN_1,
	ARFCN_14,
	ARFCN_15
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_23_1_15)

SET_COMP("radio_freq", RF_23_1_15)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_list", CHLIST_42_2_30)

SET_COMP("radio_freq", RF_42_2_30)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_23_1_15, 3)
	ARFCN_23,
	ARFCN_1,
	ARFCN_15
ENDARRAY

BEGIN_SHORT_ARRAY_PART (RF_42_2_30, 3)
	ARFCN_42,
	ARFCN_2,
	ARFCN_30
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_1_14_124_23)

SET_COMP("radio_freq", RF_1_14_124_23)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_1_14_124_23, 4)
	ARFCN_1,
	ARFCN_14,
	ARFCN_124,
	ARFCN_23
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_10_FFFF)

SET_COMP("radio_freq", RF_10_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_10_FFFF, 2)
	ARFCN_10,
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_23_10)

SET_COMP("radio_freq", RF_23_10)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_23_10, 2)
	ARFCN_23,
	ARFCN_10
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_2_8_FFFF)

SET_COMP("radio_freq", RF_2_8_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_2_8_FFFF, 3)
	ARFCN_2,
	ARFCN_8,
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_14_2_8)

SET_COMP("radio_freq", RF_14_2_8)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_14_2_8, 3)	
	ARFCN_14,
	ARFCN_2,
	ARFCN_8
ENDARRAY

BEGIN_PSTRUCT("chan_list", CHLIST_14_23_1_124)

SET_COMP("radio_freq", RF_14_23_1_124)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_14_23_1_124, 4)
	ARFCN_14,
	ARFCN_23,
	ARFCN_1,
	ARFCN_124
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_14)

SET_COMP("radio_freq", RF_14)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_14, 1)
	ARFCN_14
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_975)

SET_COMP("radio_freq", RF_975)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_975, 1)
	ARFCN_975
ENDARRAY

/*

| 0,2 | ncell 512 |
| --- | --- |
| 0x7d, 2 | ncell 637 |
| 0x75, 3 | ncell 885 |
| 0x44, 2 | serving cell 580 |

*/

BEGIN_PSTRUCT ("chan_list", CHLIST_580_512_637_885)

SET_COMP ("radio_freq", RF_580_512_637_885)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_580_512_637_885, 4)
	ARFCN_580,

ARFCN_512,
	ARFCN_637,
	ARFCN_885
ENDARRAY

/*

| 0x44, 2 | serving cell 580 |
| --- | --- |
| 0,2 | ncell 512 |
| 0x7d, 2 | ncell 637 |
| 0x2a, 3 | ncell 810 |

*/

BEGIN_PSTRUCT ("chan_list", CHLIST_580_512_637_810)

SET_COMP("radio_freq", RF_580_512_637_810)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_580_512_637_810, 4)
	ARFCN_580,

ARFCN_512,
	ARFCN_637,
	ARFCN_810	
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_580)

SET_COMP ("radio_freq", RF_580)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_580, 1)
	ARFCN_580
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_23)

SET_COMP ("radio_freq", RF_23)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_23, 1)
	ARFCN_23
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_637)

SET_COMP ("radio_freq", RF_637)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_637, 1)
	ARFCN_637
ENDARRAY

BEGIN_SHORT_ARRAY_PART (CHLIST_23_1_14_124_FFFF, 5)
	ARFCN_23,
	ARFCN_1,
	ARFCN_14,
	ARFCN_124,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 1, 0 | ncell 1 |
| --- | --- |
| 14, 0 | ncell 14 |
| 25, 0 | ncell 25 |
| 124, 0 | ncell 124 |
| 0, 2 | ncell 512 |
| 0x44, 2 | ncell 580 |
| 0x7D, 2 | ncell 637 |
| 0x75, 3 | ncell 885 |
| 0xFF, 0xFF | end of neighbour cell list |

*/

BEGIN_SHORT_ARRAY (CHLIST_1_14_25_124_512_580_637_885_FFFF, 9)
	ARFCN_1,
	ARFCN_14,
	ARFCN_25,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_637,
	ARFCN_885,
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_1_14_25_124_512_580_637_885_23)

SET_COMP ("radio_freq", RF_1_14_25_124_512_580_637_885_23)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_1_14_25_124_512_580_637_885_23, 9)
	ARFCN_1,
	ARFCN_14,
	ARFCN_25,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_637,
	ARFCN_885,
	ARFCN_23
ENDARRAY

BEGIN_PSTRUCT("result", NCELL_RES_23a)
	SET_COMP("radio_freq", ARFCN_23)
	SET_COMP("rxlev", 30)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_14a)
	SET_COMP("radio_freq", ARFCN_14)
	SET_COMP("rxlev", 44)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_25a)
	SET_COMP("radio_freq", ARFCN_25)
	SET_COMP("rxlev", 55)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_124a)
	SET_COMP("radio_freq", ARFCN_124)
	SET_COMP("rxlev", 12)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_512a)
	SET_COMP("radio_freq", ARFCN_512)
	SET_COMP("rxlev", 43)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_580a)
	SET_COMP("radio_freq", ARFCN_580)
	SET_COMP("rxlev", 25)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_637a)
	SET_COMP("radio_freq", ARFCN_637)
	SET_COMP("rxlev", 56)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_885a)
	SET_COMP("radio_freq", ARFCN_885)
	SET_COMP("rxlev", 25)
ENDSTRUCT

BEGIN_PSTRUCT_ARRAY (NCELL_RES_SC_23_8, 8)
	NCELL_RES_23a,
	NCELL_RES_14a,
	NCELL_RES_25a,
	NCELL_RES_124a,
	NCELL_RES_512a,
	NCELL_RES_580a,
	NCELL_RES_637a,
	NCELL_RES_885a

ENDARRAY

BEGIN_SHORT_ARRAY (CHLIST_14_25_512_580_637_885, 6)
	ARFCN_14,
	ARFCN_25,
	ARFCN_512,
	ARFCN_580,
	ARFCN_637,
	ARFCN_885
ENDARRAY

BEGINARRAY(NCELLS_SC_900_8_RXLEVS, 6)
	44, 55, 43, 25, 56, 25
ENDARRAY

BEGINARRAY(NCELLS_SC_900_8_BSICS, 6)
	1, 1, 1, 1, 1, 1
ENDARRAY

BEGIN_LONG_ARRAY(NCELLS_SC_900_8_TA,6)
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14
ENDARRAY

BEGIN_LONG_ARRAY(NCELLS_SC_900_8_FO,6)
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14
ENDARRAY

BEGIN_LONG_ARRAY(NCELLS_SC_900_8_FO_EOTD,6)
	FN_OFFSET_124,
	FN_OFFSET_102,
	FN_OFFSET_153,
	FN_OFFSET_204,
	FN_OFFSET_14,
	FN_OFFSET_255
ENDARRAY

BEGIN_PSTRUCT("ncells", NCELLS_SC_900_8)
	SET_COMP("no_of_ncells", 6)

SET_COMP("arfcn", CHLIST_14_25_512_580_637_885)
	SET_COMP("rx_lev", NCELLS_SC_900_8_RXLEVS)
	SET_COMP("bsic", NCELLS_SC_900_8_BSICS)
	SET_COMP("time_alignmt", NCELLS_SC_900_8_TA)
	SET_COMP("frame_offset", NCELLS_SC_900_8_FO)
ENDSTRUCT

BEGIN_PSTRUCT("ncells", NCELLS_SC_900_EOTD)
	SET_COMP("no_of_ncells", 6)

SET_COMP("arfcn", CHLIST_14_25_512_580_637_885)
	SET_COMP("rx_lev", NCELLS_SC_900_8_RXLEVS)
	SET_COMP("bsic", NCELLS_SC_900_8_BSICS)
	SET_COMP("time_alignmt", NCELLS_SC_900_8_TA)
	SET_COMP("frame_offset", NCELLS_SC_900_8_FO_EOTD)
ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CHLIST_1_11_14_25_87_124_512_885_FFFF, 9)
	ARFCN_1,
	ARFCN_11,
	ARFCN_14,
	ARFCN_25,
	ARFCN_87,
	ARFCN_124,
	ARFCN_512,
	ARFCN_885,
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_PSTRUCT ("chan_list",  CHLIST_1_11_14_25_87_124_512_885_23)

SET_COMP ("radio_freq", RF_1_11_14_25_87_124_512_885_23)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_1_11_14_25_87_124_512_885_23, 9)
	ARFCN_1,
	ARFCN_11,
	ARFCN_14,
	ARFCN_25,
	ARFCN_87,
	ARFCN_124,
	ARFCN_512,
	ARFCN_885,
	ARFCN_23
ENDARRAY

BEGIN_PSTRUCT("result", ARFCN_14b)
	SET_COMP("radio_freq", ARFCN_14)
	SET_COMP("rxlev", 44)
ENDSTRUCT

BEGIN_PSTRUCT("result", ARFCN_1b)
	SET_COMP("radio_freq", ARFCN_1)
	SET_COMP("rxlev", 11)
ENDSTRUCT

BEGIN_PSTRUCT("result", ARFCN_11b)
	SET_COMP("radio_freq", ARFCN_11)
	SET_COMP("rxlev", 22)
ENDSTRUCT

BEGIN_PSTRUCT("result", ARFCN_25b)
	SET_COMP("radio_freq", ARFCN_25)
	SET_COMP("rxlev", 15)
ENDSTRUCT

BEGIN_PSTRUCT("result", ARFCN_87b)
	SET_COMP("radio_freq", ARFCN_87)
	SET_COMP("rxlev", 21)
ENDSTRUCT

BEGIN_PSTRUCT("result", ARFCN_124b)
	SET_COMP("radio_freq", ARFCN_124)
	SET_COMP("rxlev", 3)
ENDSTRUCT

BEGIN_PSTRUCT("result", ARFCN_512b)
	SET_COMP("radio_freq", ARFCN_512)
	SET_COMP("rxlev", 43)
ENDSTRUCT

BEGIN_PSTRUCT("result", ARFCN_885b)
	SET_COMP("radio_freq", ARFCN_885)
	SET_COMP("rxlev", 23)
ENDSTRUCT

BEGIN_PSTRUCT_ARRAY(NCELL_RES_SC_23_8_1, 8)
	ARFCN_14b,
	ARFCN_1b,
	ARFCN_11b,
	ARFCN_25b,
	ARFCN_87b,
	ARFCN_124b,
	ARFCN_512b,
	ARFCN_885b
ENDARRAY

BEGIN_SHORT_ARRAY(CHLIST_1_11_14_25_87_512, 6)
	ARFCN_1,
	ARFCN_11,
	ARFCN_14,
	ARFCN_25,

ARFCN_87,
	ARFCN_512
ENDARRAY

BEGINARRAY(NCELLS_SC_900_8_1_RXLEVS, 6)
	11, 22, 44, 15, 21, 43
ENDARRAY

BEGINARRAY(NCELLS_SC_900_8_1_BSICS, 6)
	1, 1, 1, 1, 1, 1
ENDARRAY

BEGIN_LONG_ARRAY(NCELLS_SC_900_8_1_TA,6)
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14
ENDARRAY

BEGIN_LONG_ARRAY(NCELLS_SC_900_8_1_FO,6)
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14
ENDARRAY

BEGIN_PSTRUCT("ncells", NCELLS_SC_900_8_1)
	SET_COMP("no_of_ncells", 6)

SET_COMP("arfcn", CHLIST_1_11_14_25_87_512)
	SET_COMP("rx_lev", NCELLS_SC_900_8_1_RXLEVS)
	SET_COMP("bsic", NCELLS_SC_900_8_1_BSICS)
	SET_COMP("time_alignmt", NCELLS_SC_900_8_1_TA)
	SET_COMP("frame_offset", NCELLS_SC_900_8_1_FO)
ENDSTRUCT

BEGIN_SHORT_ARRAY (CHLIST_1_14_512_885_FFFF, 5)
	ARFCN_1,
	ARFCN_14,
	ARFCN_512,
	ARFCN_885,
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_23_1_14_512_885)

SET_COMP ("radio_freq", RF_23_1_14_512_885)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_23_1_14_512_885, 5)
	ARFCN_23,
	ARFCN_1,
	ARFCN_14,
	ARFCN_512,
	ARFCN_885
ENDARRAY

BEGIN_PSTRUCT("result", NCELL_RES_1c)
	SET_COMP("radio_freq", ARFCN_1)
	SET_COMP("rxlev", 11)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_14c)
	SET_COMP("radio_freq", ARFCN_14)
	SET_COMP("rxlev", 44)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_512c)
	SET_COMP("radio_freq", ARFCN_512)
	SET_COMP("rxlev", 43)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_885c)
	SET_COMP("radio_freq", ARFCN_885)
	SET_COMP("rxlev", 23)
ENDSTRUCT

BEGIN_PSTRUCT_ARRAY (NCELL_RES_SC_23_4_1, 8)
	NCELL_RES_1c,

NCELL_RES_14c,
	NCELL_RES_512c,
	NCELL_RES_885c,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1

ENDARRAY

BEGIN_SHORT_ARRAY_PART (CHLIST_1_14_512_885, 4)
	ARFCN_1,
	ARFCN_14,
	ARFCN_512,
	ARFCN_885
ENDARRAY

BEGINARRAY_PART (NCELLS_SC_900_4_1_RXLEVS, 4)
	11, 44, 43, 23
ENDARRAY

BEGINARRAY_PART (NCELLS_SC_900_4_1_BSICS, 4)
	1, 1, 1, 1, 1, 1
ENDARRAY

BEGIN_LONG_ARRAY_PART (NCELLS_SC_900_4_1_TA, 4)
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14
ENDARRAY

BEGIN_LONG_ARRAY_PART (NCELLS_SC_900_4_1_FO,4)
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14
ENDARRAY

BEGIN_PSTRUCT("ncells", NCELLS_SC_900_4_1)
	SET_COMP("no_of_ncells", 4)

SET_COMP("arfcn", CHLIST_1_14_512_885)
	SET_COMP("rx_lev", NCELLS_SC_900_4_1_RXLEVS)
	SET_COMP("bsic", NCELLS_SC_900_4_1_BSICS)
	SET_COMP("time_alignmt", NCELLS_SC_900_4_1_TA)
	SET_COMP("frame_offset", NCELLS_SC_900_4_1_FO)
ENDSTRUCT

BEGIN_SHORT_ARRAY (CHLIST_1_14_512_513_600_700_810_885_FFFF, 9)
	ARFCN_1,
	ARFCN_14,
	ARFCN_512,
	ARFCN_513,
	ARFCN_600,
	ARFCN_700,
	ARFCN_810,
	ARFCN_885,
	NOT_PRESENT_16BIT
ENDARRAY

BEGIN_PSTRUCT ("chan_list", CHLIST_1_14_512_513_600_700_810_885_637)

SET_COMP ("radio_freq", RF_1_14_512_513_600_700_810_885_637)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_1_14_512_513_600_700_810_885_637, 9)
	ARFCN_1,
	ARFCN_14,
	ARFCN_512,
	ARFCN_513,
	ARFCN_600,
	ARFCN_700,
	ARFCN_810,
	ARFCN_885,
	ARFCN_637
ENDARRAY

BEGIN_PSTRUCT("result", NCELL_RES_1d)
	SET_COMP("radio_freq", ARFCN_1)
	SET_COMP("rxlev", 33)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_14d)
	SET_COMP("radio_freq", ARFCN_14)
	SET_COMP("rxlev", 44)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_512d)
	SET_COMP("radio_freq", ARFCN_512)
	SET_COMP("rxlev", 12)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_513d)
	SET_COMP("radio_freq", ARFCN_513)
	SET_COMP("rxlev", 21)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_600d)
	SET_COMP("radio_freq", ARFCN_600)
	SET_COMP("rxlev", 24)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_700d)
	SET_COMP("radio_freq", ARFCN_700)
	SET_COMP("rxlev", 27)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_810d)
	SET_COMP("radio_freq", ARFCN_810)
	SET_COMP("rxlev", 18)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_885d)
	SET_COMP("radio_freq", ARFCN_885)
	SET_COMP("rxlev", 15)
ENDSTRUCT

BEGIN_PSTRUCT_ARRAY(NCELL_RES_SC_637_8_1, 8)
	NCELL_RES_1d,
	NCELL_RES_14d,
	NCELL_RES_512d,
	NCELL_RES_513d,
	NCELL_RES_600d,
	NCELL_RES_700d,
	NCELL_RES_810d,
	NCELL_RES_885d
ENDARRAY

BEGIN_SHORT_ARRAY(CHLIST_14_513_600_700_810_885,6)
	ARFCN_14,
	ARFCN_513,
	ARFCN_600,
	ARFCN_700,
	ARFCN_810,
	ARFCN_885
ENDARRAY

BEGINARRAY(NCELLS_SC_1800_8_1_RXLEVS,6)
	44,21,24,27,18,15
ENDARRAY

BEGINARRAY(NCELLS_SC_1800_8_1_BSICS,6)
	1,1,1,1,1,1
ENDARRAY

BEGIN_LONG_ARRAY(NCELLS_SC_1800_8_1_TA,6)
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14
ENDARRAY

BEGIN_LONG_ARRAY(NCELLS_SC_1800_8_1_FO,6)
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14
ENDARRAY

BEGIN_PSTRUCT("ncells", NCELLS_SC_1800_8_1)
	SET_COMP("no_of_ncells", 6)
	SET_COMP("arfcn",CHLIST_14_513_600_700_810_885)
	SET_COMP("rx_lev", NCELLS_SC_1800_8_1_RXLEVS)
	SET_COMP("bsic",NCELLS_SC_1800_8_1_BSICS)
	SET_COMP("time_alignmt",NCELLS_SC_1800_8_1_TA)
	SET_COMP("frame_offset",NCELLS_SC_1800_8_1_FO)
ENDSTRUCT

BEGIN_PSTRUCT ("chan_list", CHLIST_637_1_14_512_885)

SET_COMP ("radio_freq", RF_637_1_14_512_885)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_637_1_14_512_885,5)
	ARFCN_637,
	ARFCN_1,
	ARFCN_14,
	ARFCN_512,
	ARFCN_885
ENDARRAY

/*

| 14, 0 | ncell 14 |
| --- | --- |
| 44 | rxlev 44 |
| 1, 0 | ncell 1 |
| 11 | rxlev 11 |
| 0, 2 | ncell 512 |
| 43 | rxlev 43 |
| 0x75, 3 | ncell 885 |
| 23 | rxlev 23 |
| 0, 0, ... | not used |

*/

BEGIN_PSTRUCT("result", NCELL_RES_14z)
	SET_COMP("radio_freq", ARFCN_14)
	SET_COMP("rxlev", 44)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_1z)
	SET_COMP("radio_freq", ARFCN_1)
	SET_COMP("rxlev", 11)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_512z)
	SET_COMP("radio_freq", ARFCN_512)
	SET_COMP("rxlev", 43)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_885z)
	SET_COMP("radio_freq", ARFCN_885)
	SET_COMP("rxlev", 23)
ENDSTRUCT

BEGIN_PSTRUCT_ARRAY (NCELL_RES_SC_637_4_1, 8)
	NCELL_RES_14z,
	NCELL_RES_1z,
	NCELL_RES_512z,
	NCELL_RES_885z,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1
ENDARRAY

BEGINARRAY_PART (NCELLS_SC_1800_4_1_RXLEVS, 4)
	11,44,43,23
ENDARRAY

BEGINARRAY_PART (NCELLS_SC_1800_4_1_BSICS, 4)
	1,1,1,1

ENDARRAY

BEGIN_LONG_ARRAY_PART (NCELLS_SC_1800_4_1_TA, 4)
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14,
	TIME_ALIGNMT_14
ENDARRAY

BEGIN_LONG_ARRAY_PART (NCELLS_SC_1800_4_1_FO, 4)
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14,
	FN_OFFSET_14
ENDARRAY

BEGIN_PSTRUCT("ncells", NCELLS_SC_1800_4_1)

SET_COMP("no_of_ncells", 4)
	SET_COMP("arfcn", CHLIST_1_14_512_885)
	SET_COMP("rx_lev", NCELLS_SC_1800_4_1_RXLEVS)
	SET_COMP("bsic", NCELLS_SC_1800_4_1_BSICS)
	SET_COMP("time_alignmt", NCELLS_SC_1800_4_1_TA)
	SET_COMP("frame_offset", NCELLS_SC_1800_4_1_FO)
ENDSTRUCT

BEGIN_PSTRUCT ("chan_list", CHLIST_1_14_25_124_512_580_637_885_578)

SET_COMP("radio_freq", RF_1_14_25_124_512_580_637_885_578)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (RF_1_14_25_124_512_580_637_885_578, 9)
	ARFCN_1,
	ARFCN_14,
	ARFCN_25,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_637,
	ARFCN_885,
	ARFCN_578
ENDARRAY

/*

| 0x42, 2 | serving cell 578 |
| --- | --- |
| 30 | rxlev 30 |
| 14, 0 | ncell 14 |
| 44 | rxlev 44 |
| 25, 0 | ncell 25 |
| 55 | rxlev 55 |
| 124, 0 | ncell 124 |
| 12 | rxlev 12 |
| 0, 2 | ncell 512 |
| 43 | rxlev 43 |
| 0x44, 2 | ncell 580 |
| 25 | rxlev 25 |
| 0x7d, 2 | ncell 637 |
| 56 | rxlev 56 |
| 0x75, 3 | ncell 885 |
| 25 | rxlev 25 |

*/

BEGIN_PSTRUCT("result", NCELL_RES_578y)
	SET_COMP("radio_freq", ARFCN_578)
	SET_COMP("rxlev", 30)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_14y)
	SET_COMP("radio_freq", ARFCN_14)
	SET_COMP("rxlev", 44)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_25y)
	SET_COMP("radio_freq", ARFCN_25)
	SET_COMP("rxlev", 55)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_124y)
	SET_COMP("radio_freq", ARFCN_124)
	SET_COMP("rxlev", 12)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_512y)
	SET_COMP("radio_freq", ARFCN_512)
	SET_COMP("rxlev", 43)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_580y)
	SET_COMP("radio_freq", ARFCN_580)
	SET_COMP("rxlev", 25)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_637y)
	SET_COMP("radio_freq", ARFCN_637)
	SET_COMP("rxlev", 56)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RES_885y)
	SET_COMP("radio_freq", ARFCN_885)
	SET_COMP("rxlev", 25)
ENDSTRUCT

BEGIN_PSTRUCT_ARRAY(NCELL_RES_SC_578_8,8)
	NCELL_RES_578y,
	NCELL_RES_14y,
	NCELL_RES_25y,
	NCELL_RES_124y,
	NCELL_RES_512y,
	NCELL_RES_580y,
	NCELL_RES_637y,
	NCELL_RES_885y
ENDARRAY

/*

| 0 | status inactive |
| --- | --- |

*/

BEGIN_PSTRUCT ("cbch", NO_CBCH)

SKIP_COMP ("stat")

SKIP_COMP ("ch")

SKIP_COMP ("tn")

SKIP_COMP ("tsc")

SKIP_COMP ("h")

SKIP_COMP ("arfcn")

SKIP_COMP ("maio")

SKIP_COMP ("hsn")

SKIP_COMP ("ma")

ENDSTRUCT

/*

| 1 | status active |
| --- | --- |
| 8 | channel type SDCCH/8(5) |
| 3 | timeslot number |
| 3 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 1 | mobile allocation index offset |
| 0 | hopping sequence number |
| 0E,00,... | mobile allocation list |

*/

BEGIN_PSTRUCT ("cbch", CBCH_DESCRIPTION_8)

SET_COMP ("stat", STAT_ACT)

SET_COMP ("ch", CH_SDCCH_8_0)

SET_COMP ("tn", 3)

SET_COMP ("tsc", 3)

SET_COMP ("h", H_FREQ)

SET_COMP ("arfcn", 0)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

SET_COMP ("ma", CBCH_DESCRIPTION_8_MA)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CBCH_DESCRIPTION_8_MA, 5)
	0x0E,
	0x17,
	0x45,
	0x73,
	NOT_PRESENT_16BIT
ENDARRAY

/*

| 1 | status active |
| --- | --- |
| 4 | channel type SDCCH/4(3) |
| 0 | timeslot number |
| 3 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 1 | mobile allocation index offset |
| 0 | hopping sequence number |
| 0E,00,... | mobile allocation list |

*/

BEGIN_PSTRUCT ("cbch", CBCH_DESCRIPTION_4)

SET_COMP ("stat", STAT_ACT)

SET_COMP ("ch", 4 )    /*CH_SDCCH_4_0) */

SET_COMP ("tn", 0)

SET_COMP ("tsc", 3)

SET_COMP ("h", H_FREQ)

SET_COMP ("arfcn", 0)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

SET_COMP ("ma", CBCH_DESCRIPTION_4_MA)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CBCH_DESCRIPTION_4_MA, 5)
	0x0E, 0x17, 0x45, 0x73, NOT_PRESENT_16BIT
ENDARRAY

/*

| 0 | empty neighbour cell results |
| --- | --- |

*/

BEGIN_PSTRUCT ("l2_frame", L2_NO_CONTENT)

SET_COMP ("content", EMPTY_FRAME)

ENDSTRUCT

BEGINARRAY_PART (EMPTY_FRAME, 1)

0

ENDARRAY

BEGIN_PSTRUCT ("ncells", NCELLS_NO_CONTENT)

SKIP_COMP("no_of_ncells")

SKIP_COMP("arfcn")

SKIP_COMP("rx_lev")

SKIP_COMP("bsic")

SKIP_COMP("time_alignmt")

SKIP_COMP("frame_offset")

ENDSTRUCT

/*

| 23, 0 | serving cell 23 |
| --- | --- |
| 56 | rxlev 56 |
| 1, 0 | ncell 1 |
| 12 | rxlev 12 |
| 14, 0 | ncell 14 |
| 44 | rxlev 44 |
| 124, 0 | ncell 124 |
| 25 | rxlev 25 |
| 23, 0 | serving cell 23 |
| 56 | rxlev 56 |
| 1, 0 | ncell 1 |
| 12 | rxlev 12 |
| 14, 0 | ncell 14 |
| 44 | rxlev 44 |
| 124, 0 | ncell 124 |
| 25 | rxlev 25 |

*/

BEGIN_PSTRUCT("result", NCELL_RESULT_NO_CONTENT_1)
	SKIP_COMP("radio_freq")
	SKIP_COMP("rxlev")
ENDSTRUCT

BEGIN_PSTRUCT_ARRAY (NCELL_RESULT_NO_CONTENT, 8)

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1,

NCELL_RESULT_NO_CONTENT_1

ENDARRAY

BEGIN_PSTRUCT("result", NCELL_RESULT_23a)
	SET_COMP("radio_freq", ARFCN_23)
	SET_COMP("rxlev", 56)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RESULT_1a)
	SET_COMP("radio_freq", ARFCN_1)
	SET_COMP("rxlev", 12)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RESULT_14a)
	SET_COMP("radio_freq", ARFCN_14)
	SET_COMP("rxlev", 44)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RESULT_124a)
	SET_COMP("radio_freq", ARFCN_124)
	SET_COMP("rxlev", 25)
ENDSTRUCT

BEGIN_PSTRUCT_ARRAY (NCELL_RESULT_1, 8)
	NCELL_RESULT_23a,
	NCELL_RESULT_1a,
	NCELL_RESULT_14a,
	NCELL_RESULT_124a,
	NCELL_RESULT_23a,
	NCELL_RESULT_1a,
	NCELL_RESULT_14a,
	NCELL_RESULT_124a
ENDARRAY

/*

| 3 | number of neighbour cells |
| --- | --- |
| 1, 0 | ncell 1 |
| 14, 0 | ncell 14 |
| 124, 0 | ncell 124 |
| 0, 0, ... | not used (three channel numbers) |
| 12, 44, 25 | rxlev ncell 1, 14, 124 |
| 0, 0, 0 | not used (rxlev three channel numbers) |
| 1, 1, 1 | bsic ncell 1, 14, 124 |
| 0, 0, 0 | not used (bsic three channel numbers) |
| 1, 0, 14, 0, 124, 0 | timing advance ncell 1, 14, 124 |
| 0, 0, 0, 0, 0, 0 | not used (timing advance three channel numbers) |

*/

BEGIN_SHORT_ARRAY_PART (CHLIST_1_14_124, 3)
	ARFCN_1,
	ARFCN_14,
	ARFCN_124
ENDARRAY

BEGINARRAY_PART(NCELLS_1_14_124_RXLEVS, 3)
	12,44,25
ENDARRAY

BEGINARRAY_PART(NCELLS_1_14_124_BSICS, 3)
	1,1,1
ENDARRAY

BEGIN_LONG_ARRAY(NCELLS_1_14_124_TA, 6)
	1,14,124,0,0,0
ENDARRAY

BEGIN_LONG_ARRAY(NCELLS_1_14_124_FO, 6)
	0x65,0x72,0xE0,0x00,0x00,0x00
ENDARRAY

BEGIN_PSTRUCT("ncells", NCELLS_1_14_124)
	SET_COMP("no_of_ncells", NO_OF_CELLS_3)
	SET_COMP("arfcn", CHLIST_1_14_124)
	SET_COMP("rx_lev", NCELLS_1_14_124_RXLEVS)
	SET_COMP("bsic", NCELLS_1_14_124_BSICS)
	SET_COMP("time_alignmt", NCELLS_1_14_124_TA)
	SET_COMP("frame_offset", NCELLS_1_14_124_FO)
ENDSTRUCT

BEGIN_PSTRUCT("ncells", NO_NCELLS)
	SET_COMP("no_of_ncells", 0)
	SKIP_COMP("arfcn")
	SKIP_COMP("rx_lev")
	SKIP_COMP("bsic")
	SKIP_COMP("time_alignmt")
	SKIP_COMP("frame_offset")
ENDSTRUCT

/*

| 2 | number of neighbour cells |
| --- | --- |
| 1, 0 | ncell 1 |
| 14, 0 | ncell 14 |
| 0, 0, ... | not used (four channel numbers) |
| 12, 44 | rxlev ncell 1, 14 |
| 0, 0, 0, 0 | not used (rxlev four channel numbers) |
| 16, 16 | bsic ncell 1, 14 |
| 0, 0, 0, 0 | not used (bsic four channel numbers) |
| 1, 0, 14, 0 | timing advance ncell 1, 14 |
| 0, 0, ... | not used (timing advance four channel numbers) |

*/

BEGIN_SHORT_ARRAY_PART(CHLIST_1_14, 2)
	ARFCN_1,
	ARFCN_14
ENDARRAY

BEGINARRAY_PART(NCELLS_1_14_NCC_RXLEVS, 2)
	12, 44
ENDARRAY

BEGINARRAY_PART(NCELLS_1_14_NCC_BSICS, 2)
	16,16
ENDARRAY

BEGIN_LONG_ARRAY_PART(NCELLS_1_14_NCC_TA, 2)
	1, 14
ENDARRAY

BEGIN_LONG_ARRAY_PART(NCELLS_1_14_NCC_FO, 2)
	101, 114      /* not interpreted in old TAP  */
ENDARRAY

BEGIN_PSTRUCT("ncells", NCELLS_1_14_NCC)
	SET_COMP("no_of_ncells", 2)
	SET_COMP("arfcn", CHLIST_1_14)
	SET_COMP("rx_lev", NCELLS_1_14_NCC_RXLEVS)
	SET_COMP("bsic", NCELLS_1_14_NCC_BSICS)
	SET_COMP("time_alignmt", NCELLS_1_14_NCC_TA)
	SET_COMP("frame_offset", NCELLS_1_14_NCC_FO)
ENDSTRUCT

/*

| 2 | number of neighbour cells |
| --- | --- |
| 1, 0, 124, 0 | ncell 1 and 124 |
| 0, 0, ... | not used (four channel numbers) |
| 12, 25 | rxlev ncell 1 and 124 |
| 0, 0, ... | not used (rxlev four channel numbers) |
| 1,1 | bsic ncell 1 and 124 |
| 0, 0, ... | not used (bsic four channel numbers) |
| 1, 0, 124, 0 | timing advance ncell 1 and 124 |
| 0, 0, ... | not used (timing advance four channel numbers) |

*/

BEGIN_SHORT_ARRAY_PART(CHLIST_1_124, 2)
	ARFCN_1,
	ARFCN_124
ENDARRAY

BEGINARRAY_PART(NCELLS_1_124_RXLEVS, 2)
	12, 25
ENDARRAY

BEGINARRAY_PART(NCELLS_1_124_BSICS, 2)
	1,1
ENDARRAY

BEGIN_LONG_ARRAY_PART(NCELLS_1_124_TA, 2)
	1, 124
ENDARRAY

BEGIN_LONG_ARRAY_PART(NCELLS_1_124_FO, 2)
	101, 224               /* not interpreted in old tap  wrong field length */
ENDARRAY

BEGIN_PSTRUCT("ncells", NCELLS_1_124)
	SET_COMP("no_of_ncells", 2)
	SET_COMP("arfcn", CHLIST_1_124)
	SET_COMP("rx_lev", NCELLS_1_124_RXLEVS)
	SET_COMP("bsic", NCELLS_1_124_BSICS)
	SET_COMP("time_alignmt", NCELLS_1_124_TA)
	SET_COMP("frame_offset", NCELLS_1_124_FO)
ENDSTRUCT

/*

| 0 | not continous |
| --- | --- |
| 2 | 2 random bursts |
| 5, 7 | delta (two random bursts) |
| 0, 0, ... | not used (six random bursts) |
| 0xA0, 0xA3 | channel request content (two random bursts) |
| 0, 0, ... | not used (six random bursts) |

*/

BEGIN_PSTRUCT ("send_mode", TWO_BURSTS)

SKIP_COMP ("c")

SET_COMP ("no", RAND_BURSTS_2)

SET_COMP ("delta", BURST_DELTA)

SET_COMP ("rach", BURST_RACH)

ENDSTRUCT

BEGINARRAY (BURST_DELTA, 8)
	5,7,0,0,0,0,0,0
ENDARRAY

BEGINARRAY (BURST_RACH, 8)

0xA0,0xA3,0,0,0,0,0,0
ENDARRAY

/*

| 0 | no starting time present |
| --- | --- |
| 0 | n32 |
| 0 | n51 |
| 0 | n26 |

*/

BEGIN_PSTRUCT ("starting_time", NO_STARTING_TIME)

SET_COMP ("start_time_present", 0)

SKIP_COMP ("start_time")

ENDSTRUCT

/*

| 1 | starting time present |
| --- | --- |
| 11 | t1 |
| 22 | t2 |
| 33 | t3 |

*/

BEGIN_PSTRUCT ("start", STARTING_TIME)

SET_COMP ("v_start", 1)

SET_COMP ("t1", 11)

SET_COMP ("t2", 22)

SET_COMP ("t3", 33)

ENDSTRUCT

/*

| 1 | starting time present |
| --- | --- |
| 11 | n32 |
| 33 | n51 |
| 22 | n26 |

*/

BEGIN_PSTRUCT ("starting_time", S_TIME_MPHC)

SET_COMP ("start_time_present", 1)

SET_COMP ("start_time", S_TIME_MPHC_ST)

ENDSTRUCT

BEGIN_PSTRUCT ("start_time", S_TIME_MPHC_ST)

SET_COMP ("t1", 11)

SET_COMP ("t3", 33)

SET_COMP ("t2", 22)

ENDSTRUCT

/*

| 1 | starting time present |
| --- | --- |
| 11 | t1 |
| 22 | t2 |
| 33 | t3 |

*/

BEGIN_PSTRUCT ("start", STARTING_TIME_T1)

SET_COMP ("v_start", 1)

SET_COMP ("t1", 11)

SET_COMP ("t2", 22)

SET_COMP ("t3", 33)

ENDSTRUCT

/*

| 1 | starting time present |
| --- | --- |
| 11 | n32 |
| 33 | n51 |
| 22 | n26 |

*/

BEGIN_PSTRUCT ("starting_time", S_TIME_T1)

SET_COMP ("start_time_present", 1)

SET_COMP ("start_time", S_TIME_T1_ST)

ENDSTRUCT

BEGIN_PSTRUCT ("start_time", S_TIME_T1_ST)

SET_COMP ("t1", 11)

SET_COMP ("t3", 33)

SET_COMP ("t2", 22)

ENDSTRUCT

/*

| 1 | starting time present |
| --- | --- |
| 45 | t1 |
| 22 | t2 |
| 33 | t3 |

*/

BEGIN_PSTRUCT ("start", STARTING_TIME_T2)

SET_COMP ("v_start", 1)

SET_COMP ("t1", 45)

SET_COMP ("t2", 22)

SET_COMP ("t3", 33)

ENDSTRUCT

/*

| 1 | starting time present |
| --- | --- |
| 45 | n32 |
| 33 | n51 |
| 22 | n26 |

*/

BEGIN_PSTRUCT ("starting_time", S_TIME_T2)

SET_COMP ("start_time_present", 1)

SET_COMP ("start_time", S_TIME_T2_ST)

ENDSTRUCT

BEGIN_PSTRUCT ("start_time", S_TIME_T2_ST)

SET_COMP ("t1", 45)

SET_COMP ("t3", 33)

SET_COMP ("t2", 22)

ENDSTRUCT

/*

| 0 | not continous |
| --- | --- |
| 0 | 0 random bursts ( equal stop burst sending) |
| 0, 0, ... | not used (eight random bursts) |
| 0, 0, ... | not used (eight random bursts) |

*/

BEGIN_PSTRUCT ("send_mode", STOP_BURSTS)

SKIP_COMP ("c")

SKIP_COMP ("no")

SKIP_COMP ("delta")

SKIP_COMP ("rach")

ENDSTRUCT

/*

| 14 | T1 |
| --- | --- |
| 22 | T2 |
| 30 | T3 |

*/

BEGIN_PSTRUCT ("frame_no", T123_BURST_1)
	SET_COMP ("t1", 14)
	SET_COMP ("t2", 22)

SET_COMP ("t3", 30)
ENDSTRUCT

/*

| 0x0D | channel type SDCCH/8(5)  SDCCH/8(0) used |
| --- | --- |
| 0 | timeslot number |
| 0 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 1 | mobile allocation index offset |
| 0 | hopping sequence number |
| 0E,00,... | mobile allocation list |

*/

BEGIN_PSTRUCT ("ch_type", CH_TYPE_HOP)

SET_COMP ("ch", CH_SDCCH_8_0)    	/* see TABLE */

SET_COMP ("tn", 0)

SET_COMP ("tsc", 0)

SET_COMP ("h", 1)

SET_COMP ("arfcn", 0)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

SET_COMP ("ma", CHLIST_14_23_69_115_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CHLIST_14_23_69_115_FFFF, 5)
	ARFCN_14,

ARFCN_23,

ARFCN_69,

ARFCN_115,

NOT_PRESENT_16BIT
ENDARRAY

/*

| 0x0D | channel type SDCCH/8(5) |
| --- | --- |
| 1 | timeslot number |
| 0 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 1 | mobile allocation index offset |
| 0 | hopping sequence number |
| 0E,00,... | mobile allocation list |

*/

BEGINARRAY (CH_TYPE_SDCCH2, 138)
	8,1,0,1,0,0,1,0,
	0x0E,0x00,
	0x17,0x00,
	0x45,0x00,
	0x73,0x00,
	0xFF,0xFF,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00
ENDARRAY

/*

| 0x0D | channel type SDCCH/8(5) |
| --- | --- |
| 3 | timeslot number |
| 0 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 1 | mobile allocation index offset |
| 0 | hopping sequence number |
| 0E,00,... | mobile allocation list |

*/

BEGINARRAY (CH_TYPE_SDCCH3, 138)
	8,3,0,1,0,0,1,0,
	0x0E,0x00,
	0x17,0x00,
	0x45,0x00,
	0x73,0x00,
	0xFF,0xFF,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,
	0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00
ENDARRAY

/*

| 8 | channel type SDCCH/8 |
| --- | --- |
| 4 | timeslot number |
| 0 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 1 | mobile allocation index offset |
| 0 | hopping sequence number |
| 0E,00,... | mobile allocation list |

*/

BEGIN_PSTRUCT ("ch_type", CH_TYPE_SDCCH4)

SET_COMP ("ch", CH_SDCCH_8_0)    	/* see TABLE */

SET_COMP ("tn", 4)

SET_COMP ("tsc", 0)

SET_COMP ("h", 1)

SET_COMP ("arfcn", 0)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

SET_COMP ("ma", CHLIST_14_23_69_115_FFFF)

ENDSTRUCT

/*

| 0x01 | channel type TCH |
| --- | --- |
| 1 | timeslot number |
| 0 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 1 | mobile allocation index offset |
| 0 | hopping sequence number |
| 0E,00,... | mobile allocation list |

*/

BEGIN_PSTRUCT ("ch_type", CH_TYPE_TCH2)

SET_COMP ("ch", 0x01)

SET_COMP ("tn", 1)

SET_COMP ("tsc", 0)

SET_COMP ("h", 1)

SET_COMP ("arfcn", 0)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

SET_COMP ("ma", CHLIST_14_23_69_115_FFFF)

ENDSTRUCT

/*

| 0x1 | channel type TCH |
| --- | --- |
| 3 | timeslot number |
| 0 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 1 | mobile allocation index offset |
| 0 | hopping sequence number |
| 0E,00,... | mobile allocation list |

*/

BEGIN_PSTRUCT ("ch_type", CH_TYPE_TCH3)

SET_COMP ("ch", 0x01)

SET_COMP ("tn", 3)

SET_COMP ("tsc", 0)

SET_COMP ("h", 1)

SET_COMP ("arfcn", 0)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

SET_COMP ("ma", CHLIST_14_23_69_115_FFFF)

ENDSTRUCT

/*

| 0x0 | channel type |
| --- | --- |
| 3 | timeslot number |
| 6 | training sequence code |
| 0 | hopping |
| 0 | channel number (only h=0) |
| 0 | mobile allocation index offset |
| 0 | hopping sequence number |
| 00,00,... | mobile allocation list |

*/

BEGIN_PSTRUCT ("ch_type", CH_TYPE2)

SET_COMP ("ch", 0x00)

SET_COMP ("tn", 3)

SET_COMP ("tsc", 6)

SET_COMP ("h", 0)

SET_COMP ("arfcn", 0)

SET_COMP ("maio", 0)

SET_COMP ("hsn", 0)

SET_COMP ("ma", CHLIST_0)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CHLIST_0, 1)
	0x00
ENDARRAY

/*

| 1 | channel type TCH |
| --- | --- |
| 1 | timeslot number |
| 5 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 0 | mobile allocation index offset |
| 1 | hopping sequence number |
| .... | mobile allocation list |

*/

BEGIN_PSTRUCT ("ch_type", CH_TYPE_IMM_ASS)

SET_COMP ("ch", CH_TCH_F)

SET_COMP ("tn", 1)

SET_COMP ("tsc", 5)

SET_COMP ("h", H_FREQ)

SET_COMP ("arfcn", 0)

SET_COMP ("maio", 0)

SET_COMP ("hsn", 1)

SET_COMP ("ma", CHLIST_26_34_42_52_59_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CHLIST_26_34_42_52_59_FFFF, 6)
	ARFCN_26,

ARFCN_34,

ARFCN_42,

ARFCN_52,

ARFCN_59,

NOT_PRESENT_16BIT

ENDARRAY

/*

| 1 | channel type TCH |
| --- | --- |
| 1 | timeslot number |
| 5 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 1 | mobile allocation index offset |
| 1 | hopping sequence number |
| .... | mobile allocation list |

*/

BEGIN_PSTRUCT ("ch_type", CH_TYPE_FREQ_REDEF)

SET_COMP ("ch", CH_TCH_F)

SET_COMP ("tn", 1)

SET_COMP ("tsc", 5)

SET_COMP ("h", H_FREQ)

SET_COMP ("arfcn", 0)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 1)

SET_COMP ("ma", CHLIST_10_52_59_73_108_114_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CHLIST_10_52_59_73_108_114_FFFF, 7)
	ARFCN_10,

ARFCN_52,

ARFCN_59,

ARFCN_73,

ARFCN_108,

ARFCN_114,

NOT_PRESENT_16BIT

ENDARRAY

/*

| 1 | channel type TCH |
| --- | --- |
| 0 | timeslot number |
| 5 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 0 | mobile allocation index offset |
| 1 | hopping sequence number |
| .... | mobile allocation list |

*/

BEGIN_PSTRUCT ("ch_type", CH_TYPE_ASS_AFTER)

SET_COMP ("ch", CH_TCH_F)

SET_COMP ("tn", 0)

SET_COMP ("tsc", 5)

SET_COMP ("h", H_FREQ)

SET_COMP ("arfcn", 0)

SET_COMP ("maio", 0)

SET_COMP ("hsn", 1)

SET_COMP ("ma", CHLIST_73_74_75_76_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CHLIST_73_74_75_76_FFFF, 5)
	ARFCN_73,

ARFCN_74,

ARFCN_75,

ARFCN_76,

NOT_PRESENT_16BIT

ENDARRAY

/*

| 1 | channel type TCH |
| --- | --- |
| 0 | timeslot number |
| 5 | training sequence code |
| 1 | hopping |
| 0,0 | channel number (only h=0) |
| 1 | mobile allocation index offset |
| 7 | hopping sequence number |
| .... | mobile allocation list |

*/

BEGIN_PSTRUCT ("ch_type", CH_TYPE_ASS_BEFORE)

SET_COMP ("ch", CH_TCH_F)

SET_COMP ("tn", 0)

SET_COMP ("tsc", 5)

SET_COMP ("h", H_FREQ)

SET_COMP ("arfcn", 0)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 7)

SET_COMP ("ma", CHLIST_46_52_59_66_73_74_FFFF)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CHLIST_46_52_59_66_73_74_FFFF, 7)
	ARFCN_46,

ARFCN_52,

ARFCN_59,

ARFCN_66,

ARFCN_73,

ARFCN_74,

NOT_PRESENT_16BIT

ENDARRAY

/*

| 0 | ho_ref |
| --- | --- |
| 0 | ho_pow |
| 0 | ho_acc_type |
| 0 | ho_nci |

*/

BEGIN_PSTRUCT ("ho_param", HO_PARAM)

SET_COMP ("ho_ref", NCI_TA_OUT_OF_RANGE_OK)

SET_COMP ("ho_pow", 0)

SET_COMP ("ho_acc_type", 0)

SET_COMP ("ho_nci", 0)

ENDSTRUCT

/*

| 23 | ho_ref |
| --- | --- |
| 10 | ho_pow |
| 0 | ho_acc_type |
| 0 | ho_nci |

*/

BEGIN_PSTRUCT ("ho_param", HO_PARAM_1)

SET_COMP ("ho_ref", HO_REF)

SET_COMP ("ho_pow", HO_POW)

SET_COMP ("ho_acc_type", 0)

SET_COMP ("ho_nci", 0)

ENDSTRUCT

/*

| 10 | power |
| --- | --- |
| 0 | dtx |
| 5 | rlt |
| 30 | tav |
| 0 | pwrc |
| MODE_SPEECH_VER1 | mode |

*/

BEGIN_PSTRUCT ("tr_para", TR_PARAM)

SET_COMP ("power", 10)

SET_COMP ("dtx", DTX_NOT_USED)

SET_COMP ("rlt", 5)

SET_COMP ("tav", 30)

SET_COMP ("pwrc", PWRC_NOT_SET)

SET_COMP ("mode", MODE_SPEECH_VER1)

ENDSTRUCT

/*

| 0 | stat |
| --- | --- |
| 0 | algo |
| 0,1,2,3,4,5,6,7 | kc |

*/

BEGIN_PSTRUCT ("ciph", CIPH_PARAM)

SET_COMP ("stat", STAT_CIPH_OFF)

SET_COMP ("algo", ALGO_A5_1)

SET_COMP ("kc", CIPH_PARAM_KC)

ENDSTRUCT

BEGINARRAY (CIPH_PARAM_KC, 8)
	0,1,2,3,4,5,6,7
ENDARRAY

/*

| 0x01 | h |
| --- | --- |
| 0x01 | maio |
| 0x00 | hsn |
| 0x04 | channel_type (SDCCH/8) |
| 0x05 | sub_channel 5 |
| 0x03 | timeslot_no |
| 0x06 | tsc |

*/

BEGIN_PSTRUCT ("channel_desc", CHANNEL_DESC_1)

SET_COMP ("chan_sel", CHAN_SEL_1)

SET_COMP ("channel_type",  0x04)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 0)

SET_COMP ("tsc", 0)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_1)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_1)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_1)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

ENDSTRUCT

/*

| 0x01 | h |
| --- | --- |
| 0x01 | maio |
| 0x00 | hsn |
| 0x04 | channel_type (SDCCH/8) |
| 0x00 | sub_channel 0 |
| 0x01 | timeslot_no |
| 0x00 | tsc |

*/

BEGIN_PSTRUCT ("channel_desc", CHANNEL_DESC_2)

SET_COMP ("chan_sel", CHAN_SEL_10)

SET_COMP ("channel_type",  0x04)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 1)

SET_COMP ("tsc", 0)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_10)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_10)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_10)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

ENDSTRUCT

/*

| 0x01 | h |
| --- | --- |
| 0x01 | maio |
| 0x00 | hsn |
| 0x04 | channel_type (SDCCH/8) |
| 0x00 | sub_channel 0 |
| 0x03 | timeslot_no |
| 0x00 | tsc |

*/

BEGIN_PSTRUCT ("channel_desc", CHANNEL_DESC_3)

SET_COMP ("chan_sel", CHAN_SEL_9)

SET_COMP ("channel_type",  0x04)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 3)

SET_COMP ("tsc", 0)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_9)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_9)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_9)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

ENDSTRUCT

/*

| 0x01 | h |
| --- | --- |
| 0x01 | maio |
| 0x00 | hsn |
| 0x04 | channel_type (SDCCH/8) |
| 0x00 | sub_channel 0 |
| 0x04 | timeslot_no |
| 0x00 | tsc |

*/

BEGIN_PSTRUCT ("channel_desc", CHANNEL_DESC_4)

SET_COMP ("chan_sel", CHAN_SEL_8)

SET_COMP ("channel_type",  0x04)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 4)

SET_COMP ("tsc", 0)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_8)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_8)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_8)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

ENDSTRUCT

/*

| 0x01 | h |
| --- | --- |
| 0x01 | maio |
| 0x00 | hsn |
| 0x01 | channel_type (TCH) |
| 0x00 | sub_channel 0 |
| 0x01 | timeslot_no |
| 0x00 | tsc |

*/

BEGIN_PSTRUCT ("channel_desc", CHANNEL_DESC_2_TCH)

SET_COMP ("chan_sel", CHAN_SEL_7)

SET_COMP ("channel_type",  0x01)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 1)

SET_COMP ("tsc", 0)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_7)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_7)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_7)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

ENDSTRUCT

/*

| 0x01 | h |
| --- | --- |
| 0x01 | maio |
| 0x00 | hsn |
| 0x01 | channel_type (TCH) |
| 0x00 | sub_channel 0 |
| 0x03 | timeslot_no |
| 0x00 | tsc |

*/

BEGIN_PSTRUCT ("channel_desc", CHANNEL_DESC_3_TCH)

SET_COMP ("chan_sel", CHAN_SEL_11)

SET_COMP ("channel_type",  0x01)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 3)

SET_COMP ("tsc", 0)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_11)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_11)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_11)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

ENDSTRUCT

/*

| 0x01 | h |
| --- | --- |
| 0x01 | maio |
| 0x00 | hsn |
| 0x04 | channel_type (SDCCH/8) |
| 0x00 | sub_channel 0 |
| 0x03 | timeslot_no |
| 0x03 | tsc |

*/

BEGIN_PSTRUCT ("cbch_desc", CHANNEL_DESC_CBCH_8)

SET_COMP ("chan_sel", CHAN_SEL_4 )

SET_COMP ("channel_type", 0x04)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 3)

SET_COMP ("tsc", 3)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_4)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_4)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_4)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

ENDSTRUCT

/*

| 0x01 | h |
| --- | --- |
| 0x01 | maio |
| 0x00 | hsn |
| 0x03 | channel_type (SDCCH/4) |
| 0x00 | sub_channel 0 |
| 0x00 | timeslot_no |
| 0x03 | tsc |

*/

BEGIN_PSTRUCT ("cbch_desc", CHANNEL_DESC_CBCH_4)

SET_COMP ("chan_sel", CHAN_SEL_5)

SET_COMP ("channel_type", 0x03)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 0)

SET_COMP ("tsc", 3)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_5)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_5)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_5)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 0)

ENDSTRUCT

/*

| 0 | ncc |
| --- | --- |
| 1 | mcc |
| 14,0 | arfcn |

*/

BEGIN_PSTRUCT ("cell_description", CELL_DESCRIPTION_2)

SET_COMP ("ncc", 0)

SET_COMP ("bcc", 1)

SET_COMP ("bcch_carrier", 14)

ENDSTRUCT

BEGIN_PSTRUCT ("cell_description", CELL_DESCRIPTION_3)

SET_COMP ("ncc", 0)

SET_COMP ("bcc", 1)

SET_COMP ("bcch_carrier", 30)

ENDSTRUCT

/*

| 0x01 | h |
| --- | --- |
| 0x00 | maio |
| 0x01 | hsn |
| 0x01 | channel_type (TCH) |
| 0x00 | sub_channel 0 |
| 0x01 | timeslot_no |
| 0x05 | tsc |

*/

BEGIN_PSTRUCT ("channel_desc", CHANNEL_DESC_IA)

SET_COMP ("chan_sel", CHAN_SEL_2)

SET_COMP ("channel_type", CH_TCH_F)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 1)

SET_COMP ("tsc", 5)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_2)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_2)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_2)

SET_COMP ("maio", 0)

SET_COMP ("hsn", 1)

ENDSTRUCT

/*

| 5 | # channels |
| --- | --- |
| 26, 0 | 1st hopping channel |
| 34 ,0 | 2nd hopping channel |
| 42, 0 | 3rd hopping channel |
| 52, 0 | 4th hopping channel |
| 59, 0 | 5th hopping channel |
| 0, ... | rest of list |

*/

BEGIN_PSTRUCT ("frequency_list", FREQ_LIST_IA)

SET_COMP ("rf_chan_cnt", 5)

SET_COMP ("rf_chan_no", FREQ_LIST_IA_NO)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_chan_no", FREQ_LIST_IA_NO)

SET_COMP ("radio_freq", FREQ_LIST_IA_FREQ)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (FREQ_LIST_IA_FREQ, 5)
	26, 34, 	42, 52, 	59

ENDARRAY

/*

| 0x01 | h |
| --- | --- |
| 0x01 | maio |
| 0x01 | hsn |
| 0x01 | channel_type (TCH) |
| 0x00 | sub_channel 0 |
| 0x01 | timeslot_no |
| 0x05 | tsc |

*/

BEGIN_PSTRUCT ("channel_desc", CHANNEL_DESC_FR)

SET_COMP ("chan_sel", CHAN_SEL_3)

SET_COMP ("channel_type", CH_TCH_F)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 1)

SET_COMP ("tsc", 5)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_3)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_3)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_3)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 1)

ENDSTRUCT

/*

| 6 | # channels |
| --- | --- |
| 10, 0 | 1st hopping channel |
| 52 ,0 | 2nd hopping channel |
| 59, 0 | 3rd hopping channel |
| 73, 0 | 4th hopping channel |
| 108, 0 | 5th hopping channel |
| 114, 0 | 6th hopping channel |
| 0, ... | rest of list |

*/

BEGIN_PSTRUCT ("frequency_list", FREQ_LIST_FR)

SET_COMP ("rf_chan_cnt", 6)

SET_COMP ("rf_chan_no", FREQ_LIST_FR_NO)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_chan_no", FREQ_LIST_FR_NO)

SET_COMP ("radio_freq", CHLIST_10_52_59_73_108_114)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CHLIST_10_52_59_73_108_114, 6)
	ARFCN_10,

ARFCN_52,

ARFCN_59,

ARFCN_73,

ARFCN_108,

ARFCN_114

ENDARRAY

/*

| 0x01 | h |
| --- | --- |
| 0x00 | maio |
| 0x01 | hsn |
| 0x01 | channel_type (TCH) |
| 0x00 | sub_channel 0 |
| 0x00 | timeslot_no |
| 0x05 | tsc |

*/

BEGIN_PSTRUCT ("cbch_desc", CHANNEL_DESC_ASS_AFTER)

SET_COMP ("chan_sel", CHAN_SEL_6 )

SET_COMP ("channel_type", CH_TCH_F)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 0)

SET_COMP ("tsc", 5)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_6)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_6)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_6)

SET_COMP ("maio", 0)

SET_COMP ("hsn", 1)

ENDSTRUCT

/*

| 4 | # channels |
| --- | --- |
| 73, 0 | 1st hopping channel |
| 74 ,0 | 2nd hopping channel |
| 75, 0 | 3rd hopping channel |
| 76, 0 | 4th hopping channel |
| 0, ... | rest of list |

*/

BEGIN_PSTRUCT ("frequency_list", FREQ_LIST_ASS_AFTER)

SET_COMP ("rf_chan_cnt", 4)

SET_COMP ("rf_chan_no", FREQ_LIST_ASS_AFTER_NO)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_chan_no", FREQ_LIST_ASS_AFTER_NO)

SET_COMP ("radio_freq", CHLIST_73_74_75_76)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CHLIST_73_74_75_76, 4)
	ARFCN_73,

ARFCN_74,

ARFCN_75,

ARFCN_76
ENDARRAY

/*

| 0x01 | h |
| --- | --- |
| 0x01 | maio |
| 0x07 | hsn |
| 0x01 | channel_type (TCH) |
| 0x00 | sub_channel 0 |
| 0x00 | timeslot_no |
| 0x05 | tsc |

*/

BEGIN_PSTRUCT ("channel_desc", CHANNEL_DESC_ASS_BEFORE)

SET_COMP ("chan_sel", CHAN_SEL_12)

SET_COMP ("channel_type", CH_TCH_F)

SET_COMP ("sub_channel", 0)

SET_COMP ("timeslot_no", 0)

SET_COMP ("tsc", 5)

ENDSTRUCT

BEGIN_PSTRUCT ("chan_sel", CHAN_SEL_12)

SET_COMP ("h", 1)

SET_COMP ("rf_channel", RF_CHANNEL_12)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_channel", RF_CHANNEL_12)

SET_COMP ("maio", 1)

SET_COMP ("hsn", 7)

ENDSTRUCT

/*

| 6 | # channels |
| --- | --- |
| 46, 0 | 1st hopping channel |
| 52 ,0 | 2nd hopping channel |
| 59, 0 | 3rd hopping channel |
| 66, 0 | 4th hopping channel |
| 73, 0 | 5th hopping channel |
| 74, 0 | 6th hopping channel |
| 0, ... | rest of list |

*/

BEGIN_PSTRUCT ("frequency_list", FREQ_LIST_ASS_BEFORE)

SET_COMP ("rf_chan_cnt", 6)

SET_COMP ("rf_chan_no", FREQ_LIST_ASS_BEFORE_NO)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_chan_no", FREQ_LIST_ASS_BEFORE_NO)

SET_COMP ("radio_freq", CHLIST_46_52_59_66_73_74)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CHLIST_46_52_59_66_73_74, 6)
	ARFCN_46,

ARFCN_52,

ARFCN_59,

ARFCN_66,

ARFCN_73,

ARFCN_74
ENDARRAY

/*

| 0x04 | # channels |
| --- | --- |
| 0x01,0x00 | 1st hopping channel |
| 0x17,0x00 | 2nd hopping channel |
| 0x45,0x00 | 3rd hopping channel |
| 0x73,0x00 | 4th hopping channel |
| 0x00,... | rest of list |

*/

BEGIN_PSTRUCT ("frequency_list", FREQ_LIST)

SET_COMP ("rf_chan_cnt", 0x04)

SET_COMP ("rf_chan_no", FREQ_LIST_NO)

ENDSTRUCT

BEGIN_PSTRUCT ("rf_chan_no", FREQ_LIST_NO)

SET_COMP ("radio_freq", CHLIST_14_23_69_115)

ENDSTRUCT

BEGIN_SHORT_ARRAY_PART (CHLIST_14_23_69_115, 4)

ARFCN_14,

ARFCN_23,

ARFCN_69,

ARFCN_115
ENDARRAY

BEGIN_PSTRUCT ("handover_command", ASYNC_HO_CMD)
	SET_COMP	("cell_description",	CELL_DESCRIPTION_2	)
	SET_COMP	("channel_desc_1",	CHANNEL_DESC_2_TCH	)
	SET_COMP	("channel_mode_1",	MODE_SPEECH_VER1	)
	SKIP_COMP	("starting_time"				)
	SET_COMP	("ho_acc",		HO_REF			)
	SET_COMP	("txpwr",		HO_POW		)
	SKIP_COMP	("report_time_diff"				)
	SET_COMP	("frequency_list",	FREQ_LIST		)
	SKIP_COMP	("channel_desc_2"				)
	SKIP_COMP	("channel_mode_2"				)
	SKIP_COMP	("frequency_list_bef_sti"			)
	SKIP_COMP	("channel_desc_1_bef_sti"			)
	SKIP_COMP	("channel_desc_2_bef_sti"			)
	SKIP_COMP	("cipher_mode"					)
	SKIP_COMP	("a5_algorithm"				)
ENDSTRUCT

BEGIN_PSTRUCT ("handover_command", ASYNC_HO_CMD_2)
	SET_COMP ("cell_description", CELL_DESCRIPTION_2)
	SET_COMP ("channel_desc_1", CHANNEL_DESC_2_TCH)
	SET_COMP ("channel_mode_1", MODE_SPEECH_VER1)
	SET_COMP ("starting_time",	S_TIME_MPHC)
	SET_COMP ("ho_acc",	HO_REF)
	SET_COMP ("txpwr",	HO_POW)
	SKIP_COMP ("report_time_diff")
	SET_COMP ("frequency_list", FREQ_LIST)
	SKIP_COMP ("channel_desc_2")
	SKIP_COMP ("channel_mode_2")
	SET_COMP ("frequency_list_bef_sti", FREQ_LIST)
	SET_COMP ("channel_desc_1_bef_sti", CHANNEL_DESC_3_TCH)
	SKIP_COMP ("channel_desc_2_bef_sti")
	SKIP_COMP ("cipher_mode")
	SKIP_COMP ("a5_algorithm")
ENDSTRUCT

BEGIN_PSTRUCT ("handover_command", ASYNC_HO_CMD_3)
	SET_COMP ("cell_description", CELL_DESCRIPTION_3)
	SET_COMP ("channel_desc_1", CHANNEL_DESC_2_TCH)
	SET_COMP ("channel_mode_1", MODE_SPEECH_VER1)
	SET_COMP ("starting_time",	S_TIME_MPHC)
	SET_COMP ("ho_acc",	HO_REF)
	SET_COMP ("txpwr",	HO_POW)
	SKIP_COMP ("report_time_diff")
	SET_COMP ("frequency_list", FREQ_LIST)
	SKIP_COMP ("channel_desc_2")
	SKIP_COMP ("channel_mode_2")
	SET_COMP ("frequency_list_bef_sti", FREQ_LIST)
	SET_COMP ("channel_desc_1_bef_sti", CHANNEL_DESC_3_TCH)
	SKIP_COMP ("channel_desc_2_bef_sti")
	SKIP_COMP ("cipher_mode")
	SKIP_COMP ("a5_algorithm")
ENDSTRUCT

BEGIN_PSTRUCT ("classmark", CLASS_DCS_1800)

SET_COMP ("pclass", CLASS_1)

SET_COMP ("pclass2", CLASS_1)

ENDSTRUCT

/* DL/26.09.02: EOTD */

/* BYTES */

BYTE	NO_NCELLS_1	0x01

BYTE	NO_NCELLS_2	0x02

BYTE	NO_NCELLS_3	0x03

BYTE	NO_NCELLS_5	0x05

BYTE	NO_NCELLS_6	0x06

BYTE	NO_NCELLS_8	0x08

BYTE	SB_TRUE	0x01

BYTE	STOP_SIZE_12	0x0C

BYTE	mfrm_offset_44	0x2C

BYTE	SB_TRUE	0x01

BYTE	TIME_ALIGN_0	0x00

BYTE	EOTD_00	0x00

BYTE	RX_0	0x00

/* SHORTS */

SHORT	REQ_ID_0	0x0000

SHORT	REQ_ID_1	0x0001

SHORT	REQ_ID_FFFF	0xFFFF

SHORT	REQ_ID_8	0x0008

SHORT	EOTD_DELAY_8	0x0008

SHORT	EOTD_DELAY_16	0x0010

SHORT	OTD_666	0x029A

SHORT	EOTD_0000	0x0000

SHORT	OTD_0	0x0000

/* LONGS */

LONG	time_alignment_1000	0x000003E8

LONG	time_alignment_2000	0x000007D0

LONG	nom_pos_1000	0x000003E8

LONG	nom_pos_2000	0x000007D0

LONG	SUM_Amplitude_128	0x00000080

LONG	RSSI_61440	0x0000F000

LONG	RSSI_61952	0x0000F200

LONG	EOTD_0L	0x00000000

LONG	FN_OFFSET_114	0x00000072

LONG	TIME_ALIGN_14	0x0000000E

LONG	TIME_ALIGN_2664	0x00000A68

/* frame number */

LONG	FN_OFFSET_2	0x00000002

LONG	FN_OFFSET_3	0x00000003

LONG	FN_OFFSET_4	0x00000004

LONG	FN_OFFSET_5	0x00000005

LONG	FN_OFFSET_6	0x00000006

LONG	FN_OFFSET_7	0x00000007

/* Arrays */

BEGIN_SHORT_ARRAY (eotd_crosscor_6, 18)
	0x01FF, 0x00FF, 0x007F, 0x003F, 0x001F, 0x000F, 0x0000, 0x0000,
	0x0000, 0x0000, 0x0000, 0x0000, 0x0000, 0x0000, 0x0000, 0x0000,
	0x0000, 0x0000
ENDARRAY

BEGIN_SHORT_ARRAY (eotd_crosscor_9, 18)
	0x000F, 0x07FF, 0x001F, 0x007F, 0x003F, 0x00FF, 0x03FF, 0x01FF,
	0x0FFF, 0x0000, 0x0000, 0x0000, 0x0000, 0x0000, 0x0000, 0x0000,
	0x0000, 0x0000
ENDARRAY

BEGIN_SHORT_ARRAY (eotd_crosscor_12, 18)
	0x000F, 0x001F, 0x003F, 0x007F, 0x00FF, 0x01FF, 0x03FF, 0x07FF,
	0x0FFF, 0x1FFF, 0x3FFF, 0x7FFF, 0x0000, 0x0000, 0x0000, 0x0000,
	0x0000, 0x0000
ENDARRAY

BEGIN_SHORT_ARRAY (ARFCN_LIST_EOTD_1, 6)
	ARFCN_14, 0x0000, 0x0000, 0x0000, 0x0000, 0x0000
ENDARRAY

BEGIN_SHORT_ARRAY (ARFCN_LIST_EOTD_2, 6)
	ARFCN_14, ARFCN_124, 0x0000, 0x0000, 0x0000, 0x0000
ENDARRAY

BEGINARRAY (RX_LEV_LIST_EOTD_1, 6)
	RXLEV_44, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (RX_LEV_LIST_EOTD_2, 6)
	RXLEV_44, RXLEV_25, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (BSIC_LIST_EOTD_1, 6)
	BSIC_1, 0x00, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGINARRAY (BSIC_LIST_EOTD_2, 6)
	BSIC_1, BSIC_2, 0x00, 0x00, 0x00, 0x00
ENDARRAY

BEGIN_LONG_ARRAY (TIME_ALIGN_LIST_EOTD_1, 6)
	TIME_ALIGNMT_14, 0x00000000, 0x00000000, 0x00000000, 0x00000000, 0x00000000
ENDARRAY

BEGIN_LONG_ARRAY (TIME_ALIGN_LIST_EOTD_2, 6)
	TIME_ALIGNMT_14, TIME_ALIGNMT_14, 0x00000000, 0x00000000, 0x00000000, 0x00000000
ENDARRAY

BEGIN_LONG_ARRAY (FRAME_OFS_LIST_EOTD_1, 6)
	FN_OFFSET_14, 0x00000000, 0x00000000, 0x00000000, 0x00000000, 0x00000000
ENDARRAY

BEGIN_LONG_ARRAY (FRAME_OFS_LIST_EOTD_2, 6)
	FN_OFFSET_14, FN_OFFSET_14, 0x00000000, 0x00000000, 0x00000000, 0x00000000
ENDARRAY

BEGIN_SHORT_ARRAY (NCELL_LIST_EOTD, 9)
	ARFCN_1,
	ARFCN_14,
	ARFCN_25,
	ARFCN_124,
	ARFCN_512,
	ARFCN_580,
	ARFCN_637,
	ARFCN_885,
	NOT_PRESENT_16BIT
ENDARRAY

/* Primitive Structures */

BEGIN_PSTRUCT ("chan_list", CHAN_LIST_EOTD)

SET_COMP ("radio_freq", RF_1_14_25_124_512_580_637_885_23)

ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RESULT_1023a)
	SET_COMP("radio_freq", ARFCN_1023)
	SET_COMP("rxlev", 6)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RESULT_114a)
	SET_COMP("radio_freq", ARFCN_114)
	SET_COMP("rxlev", 5)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RESULT_46a)
	SET_COMP("radio_freq", ARFCN_46)
	SET_COMP("rxlev", 11)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RESULT_22a)
	SET_COMP("radio_freq", ARFCN_22)
	SET_COMP("rxlev", 3)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RESULT_15a)
	SET_COMP("radio_freq", ARFCN_15)
	SET_COMP("rxlev", 9)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RESULT_74a)
	SET_COMP("radio_freq", ARFCN_74)
	SET_COMP("rxlev", 5)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RESULT_30a)
	SET_COMP("radio_freq", ARFCN_30)
	SET_COMP("rxlev", 7)
ENDSTRUCT

BEGIN_PSTRUCT("result", NCELL_RESULT_526a)
	SET_COMP("radio_freq", ARFCN_526)
	SET_COMP("rxlev", 8)
ENDSTRUCT

BEGIN_PSTRUCT ("ncell_eotd", ARFCN_23_EOTD)
	SET_COMP ("arfcn",	ARFCN_23)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("ncell_eotd", ARFCN_637_EOTD)
	SET_COMP ("arfcn",	ARFCN_637)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("ncell_eotd", ARFCN_25_EOTD)
	SET_COMP ("arfcn",	ARFCN_25)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("ncell_eotd", ARFCN_14_EOTD)
	SET_COMP ("arfcn",	ARFCN_14)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("ncell_eotd", ARFCN_512_EOTD)
	SET_COMP ("arfcn",	ARFCN_512)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("ncell_eotd", ARFCN_580_EOTD)
	SET_COMP ("arfcn",	ARFCN_580)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("ncell_eotd", ARFCN_885_EOTD)
	SET_COMP ("arfcn",	ARFCN_885)
	SET_COMP ("bsic",	BSIC_2)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("ncell_eotd", ARFCN_516_EOTD)
	SET_COMP ("arfcn",	ARFCN_516)
	SET_COMP ("bsic",	BSIC_10)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("ncell_eotd", ARFCN_525_EOTD)
	SET_COMP ("arfcn",	ARFCN_525)
	SET_COMP ("bsic",	BSIC_10)
	SET_COMP ("mfrm_offset",	mfrm_offset_44)
	SET_COMP ("otd_type",	EXPECTED_OTD)
	SET_COMP ("exp_otd",	OTD_666)
	SET_COMP ("uncertainty",	UNC_MAX_2BIT)
	SET_COMP ("rough_rtd",	OTD_666)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_sc_res", eotd_sc_res_ok)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("arfcn",	ARFCN_23)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_12)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_sc_res1", eotd_sc_res1_ok)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("arfcn",	ARFCN_23)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_12)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_2000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_637)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("arfcn",	ARFCN_637)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_9)
	SET_COMP ("d_eotd_nrj",	RSSI_61952)
	SET_COMP ("time_tag",	nom_pos_2000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_25)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("arfcn",	ARFCN_25)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_6)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_2000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_14)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("arfcn",	ARFCN_14)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_9)
	SET_COMP ("d_eotd_nrj",	RSSI_61952)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_512)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("arfcn",	ARFCN_512)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_6)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_2000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_516)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_10)
	SET_COMP ("arfcn",	ARFCN_516)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_6)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_525)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_10)
	SET_COMP ("arfcn",	ARFCN_525)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_9)
	SET_COMP ("d_eotd_nrj",	RSSI_61952)
	SET_COMP ("time_tag",	nom_pos_2000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_580)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_1)
	SET_COMP ("arfcn",	ARFCN_580)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_9)
	SET_COMP ("d_eotd_nrj",	RSSI_61952)
	SET_COMP ("time_tag",	nom_pos_1000)
ENDSTRUCT

BEGIN_PSTRUCT ("eotd_nc_res", EOTD_NC_RES_885)
	SET_COMP ("sb_flag",	SB_TRUE)
	SET_COMP ("bsic",	BSIC_2)
	SET_COMP ("arfcn",	ARFCN_885)
	SET_COMP ("eotd_crosscor",	eotd_crosscor_6)
	SET_COMP ("d_eotd_nrj",	RSSI_61440)
	SET_COMP ("time_tag",	nom_pos_2000)
ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_1)

SET_COMP ("radio_freq",	ARFCN_1)

SET_COMP ("fn_offset",	FN_OFFSET_124)

SET_COMP ("time_alignment",	TIME_ALIGN_14)

SET_COMP ("timing_validity",	TV_VALID_TIMING_INFO)

ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_14)

SET_COMP ("radio_freq",	ARFCN_14)

SET_COMP ("fn_offset",	FN_OFFSET_124)

SET_COMP ("time_alignment",	TIME_ALIGN_14)

SET_COMP ("timing_validity",	TV_VALID_TIMING_INFO)

ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_23)

SET_COMP ("radio_freq",	ARFCN_23)

SET_COMP ("fn_offset",	FN_OFFSET_114)

SET_COMP ("time_alignment",	TIME_ALIGN_14)

SET_COMP ("timing_validity",	TV_VALID_TIMING_INFO)

ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_124)

SET_COMP ("radio_freq",	ARFCN_124)

SET_COMP ("fn_offset",	FN_OFFSET_114)

SET_COMP ("time_alignment",	TIME_ALIGN_14)

SET_COMP ("timing_validity",	TV_VALID_TIMING_INFO)

ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_25)

SET_COMP ("radio_freq",	ARFCN_25)

SET_COMP ("fn_offset",	FN_OFFSET_102)

SET_COMP ("time_alignment",	TIME_ALIGN_14)

SET_COMP ("timing_validity",	TV_VALID_TIMING_INFO)

ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_512)

SET_COMP ("radio_freq",	ARFCN_512)

SET_COMP ("fn_offset",	FN_OFFSET_153)

SET_COMP ("time_alignment",	TIME_ALIGN_14)

SET_COMP ("timing_validity",	TV_VALID_TIMING_INFO)

ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_516)

SET_COMP ("radio_freq",	ARFCN_516)

SET_COMP ("fn_offset",	FN_OFFSET_874)

SET_COMP ("time_alignment",	TIME_ALIGN_2664)

SET_COMP ("timing_validity",	TV_VALID_TIMING_INFO)

ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_525)

SET_COMP ("radio_freq",	ARFCN_525)

SET_COMP ("fn_offset",	FN_OFFSET_874)

SET_COMP ("time_alignment",	TIME_ALIGN_2664)

SET_COMP ("timing_validity",	TV_VALID_TIMING_INFO)

ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_580)

SET_COMP ("radio_freq",	ARFCN_580)

SET_COMP ("fn_offset",	FN_OFFSET_204)

SET_COMP ("time_alignment",	TIME_ALIGN_14)

SET_COMP ("timing_validity",	TV_VALID_TIMING_INFO)

ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_637)

SET_COMP ("radio_freq",	ARFCN_637)

SET_COMP ("fn_offset",	FN_OFFSET_14)

SET_COMP ("time_alignment",	TIME_ALIGN_14)

SET_COMP ("timing_validity",	TV_VALID_TIMING_INFO)

ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_885)

SET_COMP ("radio_freq",	ARFCN_885)

SET_COMP ("fn_offset",	FN_OFFSET_255)

SET_COMP ("time_alignment",	TIME_ALIGN_14)

SET_COMP ("timing_validity",	TV_VALID_TIMING_INFO)

ENDSTRUCT

BEGIN_PSTRUCT ("ncell_list", NCELL_ARFCN_EMPTY)

SET_COMP ("radio_freq",	0)

SET_COMP ("fn_offset",	0)

SET_COMP ("time_alignment",	0)

SET_COMP ("timing_validity",	0)

ENDSTRUCT

BEGIN_PSTRUCT ("ncells", NCELLS_EOTD_BSIC_1)

SET_COMP ("no_of_ncells",	NO_NCELLS_1)

SET_COMP ("arfcn",	ARFCN_LIST_EOTD_1)

SET_COMP ("rx_lev",	RX_LEV_LIST_EOTD_1)

SET_COMP ("bsic",	BSIC_LIST_EOTD_1)

SET_COMP ("time_alignmt",	TIME_ALIGN_LIST_EOTD_1)

SET_COMP ("frame_offset",	FRAME_OFS_LIST_EOTD_1)

ENDSTRUCT

BEGIN_PSTRUCT ("ncells", NCELLS_EOTD_BSIC_2)

SET_COMP ("no_of_ncells",	NO_NCELLS_2)

SET_COMP ("arfcn",	ARFCN_LIST_EOTD_2)

SET_COMP ("rx_lev",	RX_LEV_LIST_EOTD_2)

SET_COMP ("bsic",	BSIC_LIST_EOTD_2)

SET_COMP ("time_alignmt",	TIME_ALIGN_LIST_EOTD_2)

SET_COMP ("frame_offset",	FRAME_OFS_LIST_EOTD_2)

ENDSTRUCT

/* Primitive Arrays */

BEGIN_STRUCT_ARRAY (NCELL_EOTD_7, 7)
	ARFCN_23_EOTD,
	ARFCN_637_EOTD,
	ARFCN_25_EOTD,
	ARFCN_14_EOTD,
	ARFCN_512_EOTD,
	ARFCN_580_EOTD,
	ARFCN_885_EOTD
ENDARRAY

BEGIN_STRUCT_ARRAY (NCELL_EOTD_9, 9)
	ARFCN_23_EOTD,
	ARFCN_637_EOTD,
	ARFCN_25_EOTD,
	ARFCN_14_EOTD,
	ARFCN_512_EOTD,
	ARFCN_516_EOTD,
	ARFCN_525_EOTD,
	ARFCN_580_EOTD,
	ARFCN_885_EOTD
ENDARRAY

BEGIN_STRUCT_ARRAY (EOTD_RESULT_6, 6)
	EOTD_NC_RES_637,
	EOTD_NC_RES_25,
	EOTD_NC_RES_14,
	EOTD_NC_RES_512,
	EOTD_NC_RES_580,
	EOTD_NC_RES_885
ENDARRAY

BEGIN_STRUCT_ARRAY (EOTD_RESULT_8, 8)
	EOTD_NC_RES_637,
	EOTD_NC_RES_25,
	EOTD_NC_RES_14,
	EOTD_NC_RES_512,
	EOTD_NC_RES_580,
	EOTD_NC_RES_885,
	EOTD_NC_RES_516,
	EOTD_NC_RES_525
ENDARRAY

BEGIN_PSTRUCT_ARRAY (NCELL_RESULT_EOTD_1, 8)
	NCELL_RESULT_23a,
	NCELL_RESULT_1a,
	NCELL_RESULT_14a,
	NCELL_RESULT_124a,
	NCELL_RESULT_1023a,
	NCELL_RESULT_114a,
	NCELL_RESULT_46a,
	NCELL_RESULT_22a
ENDARRAY

BEGIN_PSTRUCT_ARRAY (NCELL_RESULT_EOTD_2, 8)
	NCELL_RESULT_15a,
	NCELL_RESULT_74a,
	NCELL_RESULT_30a,
	NCELL_RESULT_526a,
	NCELL_RESULT_23a,
	NCELL_RESULT_1a,
	NCELL_RESULT_14a,
	NCELL_RESULT_124a
ENDARRAY

- BEGIN_SHORT_ARRAY (A_ARFCN_EMO_000,4)

1,46,47,124
ENDARRAY

- BEGIN_SHORT_ARRAY (RADIO_FREQ_100,33)

1,46,47,124,23,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,

0
ENDARRAY

BEGIN_PSTRUCT("chan_list",CHAN_LIST_100)

SET_COMP ("radio_freq", RADIO_FREQ_100)

ENDSTRUCT

- BEGIN_SHORT_ARRAY (RADIO_FREQ_101,33)

23,1,14,124,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,
	0,0,0,0,0,0,0,0,

0
ENDARRAY

BEGIN_PSTRUCT("chan_list",CHAN_LIST_101)

SET_COMP ("radio_freq", RADIO_FREQ_101)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_100_000)

SET_COMP ("bcch_freq", CONST_1)

SET_COMP ("rxlev_acc", CONST_30)

SET_COMP ("rxlev_nbr_meas", CONST_1)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_100_001)

SET_COMP ("bcch_freq", CONST_46)

SET_COMP ("rxlev_acc", CONST_6)

SET_COMP ("rxlev_nbr_meas", CONST_1)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_100_002)

SET_COMP ("bcch_freq", CONST_47)

SET_COMP ("rxlev_acc", CONST_53)

SET_COMP ("rxlev_nbr_meas", CONST_1)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_100_003)

SET_COMP ("bcch_freq", CONST_124)

SET_COMP ("rxlev_acc", CONST_13)

SET_COMP ("rxlev_nbr_meas", CONST_1)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_100_004)

SET_COMP ("bcch_freq", CONST_23)

SET_COMP ("rxlev_acc", CONST_1)

SET_COMP ("rxlev_nbr_meas", CONST_1)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_100_NULL)

SET_COMP ("bcch_freq", CONST_0)

SET_COMP ("rxlev_acc", CONST_0)

SET_COMP ("rxlev_nbr_meas", CONST_0)

ENDSTRUCT

BEGIN_PSTRUCT_ARRAY (RES_LIST_100, 33)

RES_LIST_100_000, RES_LIST_100_001, RES_LIST_100_002,RES_LIST_100_003,

RES_LIST_100_004,

RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL,
	RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL,
	RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL,
	RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL,
	RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL,
	RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL,
	RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL, RES_LIST_100_NULL
ENDARRAY

BEGIN_PSTRUCT("ncell_meas",NCELL_MEAS_100)

SET_COMP ("res_list", RES_LIST_100)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_102_000)

SET_COMP ("bcch_freq", CONST_1)

SET_COMP ("rxlev_acc", CONST_30)

SET_COMP ("rxlev_nbr_meas", CONST_1)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_102_001)

SET_COMP ("bcch_freq", CONST_46)

SET_COMP ("rxlev_acc", CONST_6)

SET_COMP ("rxlev_nbr_meas", CONST_1)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_102_002)

SET_COMP ("bcch_freq", CONST_47)

SET_COMP ("rxlev_acc", CONST_53)

SET_COMP ("rxlev_nbr_meas", CONST_1)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_102_003)

SET_COMP ("bcch_freq", CONST_124)

SET_COMP ("rxlev_acc", CONST_13)

SET_COMP ("rxlev_nbr_meas", CONST_1)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_102_004)

SET_COMP ("bcch_freq", CONST_23)

SET_COMP ("rxlev_acc", CONST_1)

SET_COMP ("rxlev_nbr_meas", CONST_1)

ENDSTRUCT

BEGIN_PSTRUCT("res_list",RES_LIST_102_NULL)

SET_COMP ("bcch_freq", CONST_0)

SET_COMP ("rxlev_acc", CONST_0)

SET_COMP ("rxlev_nbr_meas", CONST_0)

ENDSTRUCT

BEGIN_PSTRUCT_ARRAY (RES_LIST_102, 33)

RES_LIST_102_000, RES_LIST_102_001, RES_LIST_102_002,RES_LIST_102_003,

RES_LIST_102_004,

RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL,
	RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL,
	RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL,
	RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL,
	RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL,
	RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL,
	RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL, RES_LIST_102_NULL
ENDARRAY

BEGIN_PSTRUCT("ncell_meas",NCELL_MEAS_102)

SET_COMP ("res_list", RES_LIST_102)

ENDSTRUCT

BEGIN_PSTRUCT ("meas_results", S_EMO_MEAS_RES_000_000)
	SET_COMP ("arfcn",	CONST_1)

SET_COMP ("rx_lev",	CONST_30)

- ENDSTRUCT

BEGIN_PSTRUCT ("meas_results", S_EMO_MEAS_RES_000_001)
	SET_COMP ("arfcn",	CONST_46)

SET_COMP ("rx_lev",	CONST_6)

- ENDSTRUCT

BEGIN_PSTRUCT ("meas_results", S_EMO_MEAS_RES_000_002)
	SET_COMP ("arfcn",	CONST_47)

SET_COMP ("rx_lev",	CONST_53)

- ENDSTRUCT

BEGIN_PSTRUCT ("meas_results", S_EMO_MEAS_RES_000_003)
	SET_COMP ("arfcn",	CONST_124)

SET_COMP ("rx_lev",	CONST_13)

- ENDSTRUCT

BEGIN_PSTRUCT ("meas_results", S_EMO_MEAS_RES_000_004)
	SET_COMP ("arfcn",	CONST_23)

SET_COMP ("rx_lev",	CONST_1)

- ENDSTRUCT

- BEGIN_STRUCT_ARRAY (S_EMO_MEAS_RES_000,5)

- S_EMO_MEAS_RES_000_000,
	S_EMO_MEAS_RES_000_001,
	S_EMO_MEAS_RES_000_002,
	S_EMO_MEAS_RES_000_003,
	S_EMO_MEAS_RES_000_004
ENDARRAY

# TEST CASES

## Configuration

### ALR000: Filter and Routings (GSM 900)

Description:	The ALR is configured.

Preamble:	None

RR/DL		 ALR		 PL
	 |		  |		  |
COMMAND (TAP RESET)
COMMAND (MMI RESET)
COMMAND (CC RESET)
COMMAND (SS RESET)
COMMAND (SMS RESET)
COMMAND (MM RESET)
COMMAND (RR RESET)
COMMAND (DL RESET)
COMMAND (SIM RESET)
COMMAND (PL RESET)
	 |		  |		  |
COMMAND (TAP REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (RR REDIRECT CLEAR)
COMMAND (DL REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (PL REDIRECT CLEAR)
	 |		  |		  |
COMMAND (MMI REDIRECT MM NULL)
COMMAND (MMI REDIRECT CC NULL)
COMMAND (MMI REDIRECT SS NULL)
COMMAND (MMI REDIRECT SMS NULL)
COMMAND (MMI REDIRECT PL NULL)
	 |		  |		  |
COMMAND (CC REDIRECT MMI NULL)
COMMAND (CC REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SS REDIRECT MMI NULL)
COMMAND (SS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SMS REDIRECT MMI NULL)
COMMAND (SMS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (MM REDIRECT MMI NULL)
COMMAND (MM REDIRECT CC NULL)
COMMAND (MM REDIRECT SS NULL)
COMMAND (MM REDIRECT SMS NULL)
COMMAND (MM REDIRECT SIM NULL)
COMMAND (MM REDIRECT RR NULL)
COMMAND (MM REDIRECT DL NULL)
	 |		  |		  |
COMMAND (RR REDIRECT PL NULL)
COMMAND (RR REDIRECT DL NULL)
COMMAND (RR REDIRECT MM NULL)
	 |		  |		  |
COMMAND (DL REDIRECT RR NULL)
COMMAND (DL REDIRECT MM NULL)
COMMAND (DL REDIRECT PL NULL)
	 |		  |		  |
COMMAND (PL REDIRECT RR TAP)
COMMAND (PL REDIRECT DL TAP)
COMMAND (PL REDIRECT MMI TAP)
COMMAND (PL REDIRECT L1 TAP)
	 |		  |		  |
COMMAND (SIM REDIRECT MM NULL)
	 |		  |		  |
COMMAND (TAP REDIRECT TAP PL)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

History:	22.09.99	MPA	Initial

### ALR200: Filter and Routings (DCS 1800)

Description:	The ALR is configured for DCS 1800.

Preamble:	None

RR/DL		 ALR		 PL
	 |		  |		  |
COMMAND (TAP RESET)
COMMAND (MMI RESET)
COMMAND (CC RESET)
COMMAND (SS RESET)
COMMAND (SMS RESET)
COMMAND (MM RESET)
COMMAND (RR RESET)
COMMAND (DL RESET)
COMMAND (SIM RESET)
COMMAND (PL RESET)
	 |		  |		  |
COMMAND (TAP REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (RR REDIRECT CLEAR)
COMMAND (DL REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (PL REDIRECT CLEAR)
	 |		  |		  |
COMMAND (MMI REDIRECT MM NULL)
COMMAND (MMI REDIRECT CC NULL)
COMMAND (MMI REDIRECT SS NULL)
COMMAND (MMI REDIRECT SMS NULL)
COMMAND (MMI REDIRECT PL NULL)
	 |		  |		  |
COMMAND (CC REDIRECT MMI NULL)
COMMAND (CC REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SS REDIRECT MMI NULL)
COMMAND (SS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SMS REDIRECT MMI NULL)
COMMAND (SMS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (MM REDIRECT MMI NULL)
COMMAND (MM REDIRECT CC NULL)
COMMAND (MM REDIRECT SS NULL)
COMMAND (MM REDIRECT SMS NULL)
COMMAND (MM REDIRECT SIM NULL)
COMMAND (MM REDIRECT RR NULL)
COMMAND (MM REDIRECT DL NULL)
	 |		  |		  |
COMMAND (RR REDIRECT PL NULL)
COMMAND (RR REDIRECT DL NULL)
COMMAND (RR REDIRECT MM NULL)
	 |		  |		  |
COMMAND (DL REDIRECT RR NULL)
COMMAND (DL REDIRECT MM NULL)
COMMAND (DL REDIRECT PL NULL)
	 |		  |		  |
COMMAND (PL REDIRECT RR TAP)
COMMAND (PL REDIRECT DL TAP)
COMMAND (PL REDIRECT MMI TAP)
COMMAND (PL REDIRECT L1 TAP)
	 |		  |		  |
COMMAND (SIM REDIRECT MM NULL)
	 |		  |		  |
COMMAND (TAP REDIRECT TAP PL)
	 |		  |		  |
COMMAND (PL CONFIG STD=4)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

History:	22.09.99	MPA	Initial

### ALR400: Filter and Routings (PCS 1900)

Description:	The ALR is configured for PCS 1900.

Preamble:	None

RR/DL		 ALR		 PL
	 |		  |		  |
COMMAND (TAP RESET)
COMMAND (MMI RESET)
COMMAND (CC RESET)
COMMAND (SS RESET)
COMMAND (SMS RESET)
COMMAND (MM RESET)
COMMAND (RR RESET)
COMMAND (DL RESET)
COMMAND (SIM RESET)
COMMAND (PL RESET)
	 |		  |		  |
COMMAND (TAP REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (RR REDIRECT CLEAR)
COMMAND (DL REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (PL REDIRECT CLEAR)
	 |		  |		  |
COMMAND (MMI REDIRECT MM NULL)
COMMAND (MMI REDIRECT CC NULL)
COMMAND (MMI REDIRECT SS NULL)
COMMAND (MMI REDIRECT SMS NULL)
COMMAND (MMI REDIRECT PL NULL)
	 |		  |		  |
COMMAND (CC REDIRECT MMI NULL)
COMMAND (CC REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SS REDIRECT MMI NULL)
COMMAND (SS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SMS REDIRECT MMI NULL)
COMMAND (SMS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (MM REDIRECT MMI NULL)
COMMAND (MM REDIRECT CC NULL)
COMMAND (MM REDIRECT SS NULL)
COMMAND (MM REDIRECT SMS NULL)
COMMAND (MM REDIRECT SIM NULL)
COMMAND (MM REDIRECT RR NULL)
COMMAND (MM REDIRECT DL NULL)
	 |		  |		  |
COMMAND (RR REDIRECT PL NULL)
COMMAND (RR REDIRECT DL NULL)
COMMAND (RR REDIRECT MM NULL)
	 |		  |		  |
COMMAND (DL REDIRECT RR NULL)
COMMAND (DL REDIRECT MM NULL)
COMMAND (DL REDIRECT PL NULL)
	 |		  |		  |
COMMAND (PL REDIRECT RR TAP)
COMMAND (PL REDIRECT DL TAP)
COMMAND (PL REDIRECT MMI TAP)
COMMAND (PL REDIRECT L1 TAP)
	 |		  |		  |
COMMAND (SIM REDIRECT MM NULL)
	 |		  |		  |
COMMAND (TAP REDIRECT TAP PL)
	 |		  |		  |
COMMAND (PL CONFIG STD=3)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

History:	22.09.99	MPA	Initial

### ALR600: Filter and Routings (Dualband GSM 900 / DCS 1800)

Description:	The ALR is configured for Dualband GSM 900 / DCS 1800.

Preamble:	None

RR/DL		 ALR		 PL
	 |		  |		  |
COMMAND (TAP RESET)
COMMAND (MMI RESET)
COMMAND (CC RESET)
COMMAND (SS RESET)
COMMAND (SMS RESET)
COMMAND (MM RESET)
COMMAND (RR RESET)
COMMAND (DL RESET)
COMMAND (SIM RESET)
COMMAND (PL RESET)
	 |		  |		  |
COMMAND (TAP REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (RR REDIRECT CLEAR)
COMMAND (DL REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (PL REDIRECT CLEAR)
	 |		  |		  |
COMMAND (MMI REDIRECT MM NULL)
COMMAND (MMI REDIRECT CC NULL)
COMMAND (MMI REDIRECT SS NULL)
COMMAND (MMI REDIRECT SMS NULL)
COMMAND (MMI REDIRECT PL NULL)
	 |		  |		  |
COMMAND (CC REDIRECT MMI NULL)
COMMAND (CC REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SS REDIRECT MMI NULL)
COMMAND (SS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SMS REDIRECT MMI NULL)
COMMAND (SMS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (MM REDIRECT MMI NULL)
COMMAND (MM REDIRECT CC NULL)
COMMAND (MM REDIRECT SS NULL)
COMMAND (MM REDIRECT SMS NULL)
COMMAND (MM REDIRECT SIM NULL)
COMMAND (MM REDIRECT RR NULL)
COMMAND (MM REDIRECT DL NULL)
	 |		  |		  |
COMMAND (RR REDIRECT PL NULL)
COMMAND (RR REDIRECT DL NULL)
COMMAND (RR REDIRECT MM NULL)
	 |		  |		  |
COMMAND (DL REDIRECT RR NULL)
COMMAND (DL REDIRECT MM NULL)
COMMAND (DL REDIRECT PL NULL)
	 |		  |		  |
COMMAND (PL REDIRECT RR TAP)
COMMAND (PL REDIRECT DL TAP)
COMMAND (PL REDIRECT MMI TAP)
COMMAND (PL REDIRECT L1 TAP)
	 |		  |		  |
COMMAND (SIM REDIRECT MM NULL)
	 |		  |		  |
COMMAND (TAP REDIRECT TAP PL)
	 |		  |		  |
COMMAND (PL CONFIG STD=5)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

History:	22.09.99	MPA	Initial

### ALR849: Filter and Routings (Dualband GSM 900 / E-GSM / DCS 1800)

Description:	The ALR is configured for Dualband GSM 900 / E-GSM / DCS 1800.

Preamble:	None

RR/DL		 ALR		 PL
	 |		  |		  |
COMMAND (TAP RESET)
COMMAND (MMI RESET)
COMMAND (CC RESET)
COMMAND (SS RESET)
COMMAND (SMS RESET)
COMMAND (MM RESET)
COMMAND (RR RESET)
COMMAND (DL RESET)
COMMAND (SIM RESET)
COMMAND (PL RESET)
	 |		  |		  |
COMMAND (TAP REDIRECT CLEAR)
COMMAND (MMI REDIRECT CLEAR)
COMMAND (CC REDIRECT CLEAR)
COMMAND (SS REDIRECT CLEAR)
COMMAND (SMS REDIRECT CLEAR)
COMMAND (MM REDIRECT CLEAR)
COMMAND (RR REDIRECT CLEAR)
COMMAND (DL REDIRECT CLEAR)
COMMAND (SIM REDIRECT CLEAR)
COMMAND (PL REDIRECT CLEAR)
	 |		  |		  |
COMMAND (MMI REDIRECT MM NULL)
COMMAND (MMI REDIRECT CC NULL)
COMMAND (MMI REDIRECT SS NULL)
COMMAND (MMI REDIRECT SMS NULL)
COMMAND (MMI REDIRECT PL NULL)
	 |		  |		  |
COMMAND (CC REDIRECT MMI NULL)
COMMAND (CC REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SS REDIRECT MMI NULL)
COMMAND (SS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (SMS REDIRECT MMI NULL)
COMMAND (SMS REDIRECT MM NULL)
	 |		  |		  |
COMMAND (MM REDIRECT MMI NULL)
COMMAND (MM REDIRECT CC NULL)
COMMAND (MM REDIRECT SS NULL)
COMMAND (MM REDIRECT SMS NULL)
COMMAND (MM REDIRECT SIM NULL)
COMMAND (MM REDIRECT RR NULL)
COMMAND (MM REDIRECT DL NULL)
	 |		  |		  |
COMMAND (RR REDIRECT PL NULL)
COMMAND (RR REDIRECT DL NULL)
COMMAND (RR REDIRECT MM NULL)
	 |		  |		  |
COMMAND (DL REDIRECT RR NULL)
COMMAND (DL REDIRECT MM NULL)
COMMAND (DL REDIRECT PL NULL)
	 |		  |		  |
COMMAND (PL REDIRECT RR TAP)
COMMAND (PL REDIRECT DL TAP)
COMMAND (PL REDIRECT MMI TAP)
COMMAND (PL REDIRECT L1 TAP)
	 |		  |		  |
COMMAND (SIM REDIRECT MM NULL)
	 |		  |		  |
COMMAND (TAP REDIRECT TAP PL)
	 |		  |		  |
COMMAND (PL CONFIG STD=6)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

History:	22.09.99	MPA	Initial

## Cell Selection (GSM 900)

### ALR001: Initiation by RR, 4 channels available

Description:	RR starts a cell selection with power measurements with interruption. Four channels in the order 23, 14, 124, 1 have acceptable fieldstrength values.

Preamble:	ALR000

RR/DL		ALR		PL
	|		|		|

- |	MPH_POWER_REQ	|		|
	*=============================>*		|

- |		|	MPHC_INIT_L1_REQ	|
	|		*==============================>*

- |		|	MPHC_INIT_L1_CON	|
	|		*<==============================*

- |		|	MPHC_RXLEV_REQ	|
	|		*==============================>*

- |		|	MPHC_RXLEV_IND	|
	|		*<==============================*

- |		|	MPHC_RXLEV_REQ	|
	|		*==============================>*

- |		|	MPHC_RXLEV_IND	|
	|		*<==============================*

- |		|	MPHC_RXLEV_REQ	|
	|		*==============================>*

- |		|	MPHC_RXLEV_IND	|
	|		*<==============================*

- |		|	MPHC_RXLEV_REQ	|
	|		*==============================>*

- |		|	MPHC_RXLEV_IND	|
	|		*<==============================*

- |		|	MPHC_RXLEV_REQ	|
	|		*==============================>*

- |		|	MPHC_RXLEV_IND	|
	|		*<==============================*

- |	MPH_POWER_CNF	|		|
	*<=============================*		|

|		|		|

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_GSM_900

- MPHC_INIT_L1_REQ
		radio_band_config	STD_900

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_1

- MPH_POWER_CNF
		num_of_chan	CHANNELS_4
		arfcn	NOT_USED
		rx_lev	NOT_USED

History:	22.09.99	MPA	Initial
	17.05.02	MSB	add MPHC_INIT_L1_REQ/CON

### ALR002: Initiation by RR, no channels available

Description:	RR starts a cell selection. No suitable channel is available.

Preamble:	ALR000

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(9)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(14)	 |	MPH_POWER_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_GSM_900

- MPHC_INIT_L1_REQ
		radio_band_config	STD_900

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPH_POWER_CNF
		num_of_chan	CHANNELS_0
		arfcn	NOT_USED
		rx_lev	NOT_USED

History:	22.09.99	MPA	Initial
	17.05.02	MSB	add MPHC_INIT_L1_REQ/CON

### ALR003: Find BCCH carrier, first channel

Description:	The carrier with the highest fieldstrength (channel 23) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR001

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |    MPHC_NETWORK_SYNC_REQ     |
	 |		  *=============================>*	
(3)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(4)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(5)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_23_STD_900

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_23
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_23
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_1

- MPH_BSIC_CNF
		arfcn	ARFCN_23_STD_900
		bsic	BSIC_1
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
 		radio_freq	ARFCN_23
 		fn_offset	FN_OFFSET_0
 		time_alignment	TIME_ALIGNMT_0
 		tsc	BSIC_1

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	FULL_READ

History:	22.09.99	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ included

### ALR004: Find BCCH carrier, first channel failed, then second channel

Description:	The carrier with the highest fieldstrength (channel 23) is selected for synchronizing to frequency correction burst and synchron burst. The attempt failed. A second request is started for the next strongest channel (channel 14).

Preamble:	ALR001

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(4)	 |		  |	MPHC_NETWORK_SYNC_REQ     |
	 |		  *=============================>*	
(5)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(8)	 |		  |	MPHC_NETWORK_SYNC_REQ     |
	 |		  *=============================>*	
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_23_STD_900

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_23
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_23
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_1

- MPH_BSIC_CNF
		arfcn	ARFCN_23_STD_900
		bsic	BSIC_1
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_14_STD_900

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

History:	22.09.99	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR005: Find BCCH carrier, all channels failed, then error indication

Description:	All synchronization attempts to the available channels are failed. RR is informed by an error indication with the cause no BCCH available.

Preamble:	ALR004

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(3)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(4)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*	
(5)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*	
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(8)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*	
(9)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*	
(10)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_14_STD_900
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124_STD_900

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_124
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_124_STD_900
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_1_STD_900

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_1
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_1_STD_900
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

History:	22.09.99	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR006: Find BCCH carrier, second channel

Description:	The next best channel (channel 14) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR004

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(3)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_14_STD_900
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
 		radio_freq	ARFCN_14
 		fn_offset	FN_OFFSET_0
 		time_alignment	TIME_ALIGNMT_0
 		tsc	BSIC_0

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	FULL_READ

History:	23.09.99	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ included

### ALR007: Read BCCH data

Description:	The BCCH data blocks are read for the channel 23.

Preamble:	ALR003

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(3)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(4)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(5)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(8)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NEIGH_CELL_DESC_1
		ncc_permit	NCC_PERMIT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4
		tc	TC_3
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

History:	23.09.99	MPA	Initial

### ALR008: Read failed BCCH data

Description:	A failed BCCH data block is read.

Preamble:	ALR007

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_ERROR_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_3
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_23

History:	23.09.99	MPA	Initial

### ALR009: RR rejects BCCH carrier, try third channel

Description:	RR rejects the BCCH carrier. The next best channel (channel 124) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR006

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(11)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_124_STD_900

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_124
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_124_STD_900
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
 		radio_freq	ARFCN_124
 		fn_offset	FN_OFFSET_0
 		time_alignment	TIME_ALIGNMT_0
 		tsc	BSIC_0

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	FULL_READ

- MPHC_DATA_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

History:	23.09.9	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR010: RR rejects BCCH carrier, try fourth channel

Description:	RR rejects the BCCH carrier. The next best channel (channel 1) is selected for synchronising to frequency correction burst and synchron burst.

Preamble:	ALR009

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(11)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_1_STD_900

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_1
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_1_STD_900
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		tsc	BSIC_0

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPHC_DATA_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

History:	23.09.99	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR011: RR rejects BCCH carrier, no further channel available

Description:	RR rejects the BCCH carrier. No further channel is available.

Preamble:	ALR010

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	NOT_PRESENT_16BIT

History:	23.09.99	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR012: RR select second channel

Description:	RR selects the channel 14 after reading the BCCH carrier.
Variant A: with neighbourcell list
Variant B: with empty neighbour cell list
Variant C: like A, but non-combined CCCH

Preamble:	ALR006

Variants:	<A> .. <C>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_STOP_SCELL_BCCH_CON	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(6) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(7)	 |	MPH_IDENTITY_REQ	  |		  |
	 *===============================>*		  |
(8)	 |	MPH_CBCH_REQ	  |		  |
	 *===============================>*		  |
(9)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(10)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_GSM_900

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_14
		ext_bcch	NOT_USED
<A>	comb_ccch	COMB_CCCH_COMB
<B>	comb_ccch	COMB_CCCH_COMB
<C>	comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_STOP_SCELL_BCCH_CON
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
<A>	bcch_combined	COMB_CCCH_COMB
<B>	bcch_combined	COMB_CCCH_COMB
<C>	bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
<A>	page_block_index	PBI_0
<B>	page_block_index	PBI_0
<C>	page_block_index	PBI_2
 		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_IDENTITY_REQ
		mid	MS_ID_IMSI_TMSI

- MPH_CBCH_REQ
		cbch	NO_CBCH

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
<A>	arfcn	CHLIST_23_1_124_FFFF

- <B>	arfcn	EMPTY_NCELL_LIST
<C>	arfcn	CHLIST_23_1_124_FFFF

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
<A>	chan_list	CHLIST_14_23_1_124

- <B>	chan_list	CHLIST_14
<C>	chan_list	CHLIST_14_23_1_124

- <A>	num_of_chans	CHANNELS_4
<B>	num_of_chans	CHANNELS_1
<C>	num_of_chans	CHANNELS_4
		ba_id	BA_ID_1
 		next_radio_freq_measured	CHAN_LIST_IDX_0

History:	23.09.99	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

19.07.01	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ included.
				MPHC_STOP_SCELL_BCCH_CON included.

07.02.02	LG	changed value for ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ included

### ALR013: RR select first channel

Description:	RR selects the channel 23 after reading the BCCH carrier.

Preamble:	ALR007

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(6) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(7)	 |	MPH_IDENTITY_REQ	  |		  |
	 *===============================>*		  |
(8)	 |	MPH_CBCH_REQ	  |		  |
	 *===============================>*		  |
(9)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(10)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_GSM_900

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
 		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
 		page_block_index	PBI_0
 		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	FULL_READ

- MPH_IDENTITY_REQ
		mid	MS_ID_IMSI_TMSI

- MPH_CBCH_REQ
		cbch	NO_CBCH

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	CHLIST_1_14_124_FFFF
		sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_23_1_14_124	
		num_of_chans	CHANNELS_4
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

History:	23.09.99	MPA	Initial

12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

21.06.01	MSB	sequence and parameter changed
			for MPHC_SCELL_NBCCH_REQ:
			after pagemode set to REORG, full read is 				necessary

19.07.01	MSB	CHLIST_23_1_14_124 changed

07.02.02	LG	changed value for ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ included

### ALR015: Re-Initiation of Cell Selection during measurements

Description:	RR has started a cell selection. During power measurements a new activation of cell selection is started by RR. The power measurement is restarted.

Preamble:	ALR000

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(7)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(8)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(9)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(16)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(17)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(19)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(20)	 |	MPH_POWER_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_GSM_900

- MPHC_INIT_L1_REQ
		radio_band_config	STD_900

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_GSM_900

- MPHC_INIT_L1_REQ
		radio_band_config	STD_900

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_1

- MPH_POWER_CNF
		num_of_chan	CHANNELS_4
		arfcn	ARFCN_23_14_124_1_WITH_STD
		rx_lev	RXLEV_23_14_124_1

History:	23.09.99	MPA	Initial
	17.05.02	MSB	add MPHC_INIT_L1_REQ/CON

### ALR017: Re-Initiation of Cell Selection during BCCH reading

Description:	During BCCH reading RR restarts a cell selection. This leads to a new full power measurement cycle.

Preamble:	ALR007

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_SYNC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(3)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(4)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(9)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(16)	 |	MPH_POWER_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_SYNC_REQ
		cs	CS_STOP_BCCH_READING

- MPHC_STOP_SCELL_BCCH_REQ 
		param	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_GSM_900

- MPHC_INIT_L1_REQ
		radio_band_config	STD_900

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_1

- MPH_POWER_CNF
		num_of_chan	CHANNELS_4
		arfcn	ARFCN_23_14_124_1_WITH_STD
		rx_lev	RXLEV_23_14_124_1

History:	23.09.99	MPA	Initial
	17.05.02	MSB	add MPHC_INIT_L1_REQ/CON, change arfcn´s

### ALR044: Stop Idle Mode by Normal Cell Selection

Description:	The idle mode is stopped if a normal cell selection is initiated.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(3)	 |		  |  MPHC_STOP_CCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  | MPHC_STOP_RXLEV_PERIODIC_REQ  |
	 |		  *=============================>*
(5)	 |		  | MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_GSM_900

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_STOP_CCCH_REQ
		param	NOT_USED

- MPHC_STOP_RXLEV_PERIODIC_REQ
 		param	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_INIT_L1_REQ
		radio_band_config	STD_900

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

History:	23.09.99	MPA	Initial
	17.05.02	MSB	add parameter ‘freq_band’

29.10.02	DL	MPHC_STOP_NCELL_SYNC/BCCH_REQ removed

### ALR018: ALR008 - Engineering mode

Description:	A failed BCCH data block is read.

Preamble:	ALR007

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	EM_L1_EVENT_REQ	  |		  |
	 *===============================>*		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_ERROR_IND	  |		  |
	 *<===============================*		  |
(3)	 |	EM_DATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- EM_L1_EVENT_REQ
		bitmask_l1_h	Bitm_H
		bitmask_l1_l	Bitm_L

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_3
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_23

- EM_DATA_IND
		entity	ENTITY

History:	23-Oct-01	OT	Initial

### ALR019: Search for BCCH carrier takes too long

Description:	A request is made to sync to a network. There is a timeout when no response is received by ALR.

After informing RR that there is no BCCH, RR instructs ALR to check the next BCCH. This again, fails.

This ensures the timer is correctly set up for subsequent BCCH search’s.

Preamble:	ALR001

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |    MPHC_NETWORK_SYNC_REQ     |
	 |		  *=============================>*	
TIMEOUT (10000) 
(3)	 |		  |    MPHC_STOP_NETWORK_SYNC_REQ|
	 |		  *=============================>*	
(4)	 |		  |    MPHC_STOP_NETWORK_SYNC_CON|
	 |		  *<=============================*	
(5)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(6)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(7)	 |		  |    MPHC_NETWORK_SYNC_REQ     |
	 |		  *=============================>*	
TIMEOUT (10000) 
(8)	 |		  |    MPHC_STOP_NETWORK_SYNC_REQ|
	 |		  *=============================>*	
(9)	 |		  |    MPHC_STOP_NETWORK_SYNC_CON|
	 |		  *<=============================*	
(10)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_23_STD_900

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_23
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_STOP_NETWORK_SYNC_REQ
		param	NOT_USED

- MPHC_STOP_NETWORK_SYNC_CON
		param	NOT_USED

- MPH_BSIC_CNF
		arfcn	NOT_USED
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_24_STD_900

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_24
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_STOP_NETWORK_SYNC_REQ
		param	NOT_USED

- MPHC_STOP_NETWORK_SYNC_CON
		param	NOT_USED

- MPH_BSIC_CNF
		arfcn	NOT_USED
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

History:	14.02.03	ZMM	Initial
	19.03.03	ZMM	Sent MPH_BSIC_CNF back to RR after timeout

## PLMN Selection(GSM 900)

### ALR084: Init Ncell data, 4 channels available

Description:	RR starts ncell measurements for cells 23, 14, 124, 1.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(13)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(15)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(16)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(17)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(18)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(20)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(21)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(22)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(23)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(24)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_816
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_124

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_408
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

History:	24.09.99	MPA	Initial

07.02.02	LG	changed value for ba_id
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR085: Initiation by RR, 4 channels available

Description:	RR starts a full list power measurement with power measurements. Four channels in the order 23, 14, 124, 1 have acceptable fieldstrength values.

Preamble:	ALR084

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(4)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(12)	 |	MPH_POWER_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	BAND_GSM_900

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_1

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_1

- MPH_POWER_CNF
		num_of_chan	CHANNELS_4
		arfcn	ARFCN_23_14_124_1_WITH_STD
		rx_lev	RXLEV_23_14_124_1

History:	24.09.99	MPA	Initial
	17.05.02	MSB	change arfcn´s

29.10.02	DL	MPHC_STOP_NCELL_SYNC/BCCH_REQ removed

### ALR086: Initiation by RR, no channels available

Description:	RR starts a full list power measurement. No suitable channel is available.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(9)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(12)	 |	MPH_POWER_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	BAND_GSM_900

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPH_POWER_CNF
		num_of_chan	CHANNELS_0
		arfcn	NOT_USED
		rx_lev	NOT_USED

History:	22.09.99	MPA	Initial

29.10.02	DL	MPHC_STOP_NCELL_SYNC/BCCH_REQ removed

### ALR088: Find BCCH carrier, second channel

Description:	The non-serving cell carrier with the highest fieldstrength (channel 14) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR085

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*	
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_14

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

History:	24.09.99	MPA	Initial

### ALR089: Find BCCH carrier, all channels failed, then error indication

Description:	All synchronization attempts to the available channels fail. RR is informed by an error indication with the cause no BCCH available.

Preamble:	ALR088

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(3)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*	
(5)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(8)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*	
(9)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(10)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPH_BSIC_CNF
		arfcn	ARFCN_14
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_124
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_1
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPH_BSIC_CNF
		arfcn	ARFCN_1
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

History:	24.09.99	MPA	Initial
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR090: Find BCCH carrier, second channel

Description:	The next best channel (channel 14) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR088

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(3)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPH_BSIC_CNF
		arfcn	ARFCN_14
		bsic	BSIC_1
		cs	CS_NO_ERROR

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_2_3_4
		gprs_prio	NOT_USED

History:	24.09.99	MPA	Initial
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR091: Read BCCH data

Description:	The BCCH data blocks are read for the channel 14.

Preamble:	ALR090

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(3)	 |		  |  MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
(5)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
(6)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(7)	 |	MPH_ERROR_IND	  |		  |
	 *<===============================*		  |
(8)	 |		  |  MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
(10)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
(11)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(12)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(13)	 |		  |  MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_1
		fn	FN_OFFSET_14

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NEIGH_CELL_DESC_1
		ncc_permit	NCC_PERMIT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	STOP_SIZE_1
		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	STOP_SIZE_1
		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	STOP_SIZE_1
		radio_freq_array	STOP_ARRAY_14

History:	24.09.99	MPA	Initial

### ALR093: RR rejects BCCH carrier, try third channel

Description:	RR rejects the BCCH carrier. The next best channel (channel 124) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR091

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*	
(3)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(4)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(5)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
(6)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(7)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(8)	 |		  |  MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
(9)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_124

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_124
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPH_BSIC_CNF
		arfcn	ARFCN_124
		bsic	BSIC_1
		cs	CS_NO_ERROR

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_2_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_0
		fn	FN_OFFSET_124

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	STOP_SIZE_1
		radio_freq_array	STOP_ARRAY_124

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_2
		gprs_prio	NOT_USED

History:	24.09.99	MPA	Initial
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR094: RR rejects BCCH carrier, try fourth channel

Description:	RR rejects the BCCH carrier. The next best channel (channel 1) is selected for synchronising to frequency correction burst and synchron burst.

Preamble:	ALR093

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
 (4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*	
(5)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
(8)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(9)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(10)	 |		  |  MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
(11)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_1

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	STOP_SIZE_1
		radio_freq_array	STOP_ARRAY_124

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_1
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPH_BSIC_CNF
		arfcn	ARFCN_1
		bsic	BSIC_1
		cs	CS_NO_ERROR

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_2_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_0
		fn	FN_OFFSET_1

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NEIGH_CELL_DESC_1
		ncc_permit	NCC_PERMIT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	STOP_SIZE_1
		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

History:	24.09.99	MPA	Initial
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR095: RR rejects BCCH carrier, no further channel available

Description:	RR rejects the BCCH carrier. No further channel is available.

Preamble:	ALR094

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_SYNC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*	
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_SYNC_REQ
 		cs	CS_STOP_PLMN_SEARCH

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

History:	24.09.99	MPA	Initial

## Cell Selection(DCS 1800)

### ALR201: Initiation by RR, 4 channels available

Description:	RR starts a cell selection. Four channels in the order 637, 580, 885, 512 have acceptable fieldstrength values.

Preamble:	ALR200

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(14)	 |	MPH_POWER_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DCS_1800

- MPHC_INIT_L1_REQ
		radio_band_config	STD_1800

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_3_1800

- MPH_POWER_CNF
		num_of_chan	CHANNELS_4
		arfcn	ARFCN_637_580_885_512_WITH_STD
		rx_lev	RXLEV_637_580_885_512

History:	22.09.99	MPA	Initial
	17.05.02	MSB	add MPHC_INIT_L1_REQ/CON, change arfcn´s

### ALR202: Initiation by RR, no channels available

Description:	RR starts a cell selection. No channel is available. After thirty attempts a failure is signalled to RR.

Preamble:	ALR200

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(9)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(14)	 |	MPH_POWER_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DCS_1800

- MPHC_INIT_L1_REQ
		radio_band_config	STD_1800

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2

- MPH_POWER_CNF
		num_of_chan	CHANNELS_0
		arfcn	NOT_USED
		rx_lev	NOT_USED

History:	22.09.99	MPA	Initial
	17.05.02	MSB	add MPHC_INIT_L1_REQ/CON

### ALR203: Find BCCH carrier, first channel

Description:	The carrier with the highest fieldstrength (channel 637) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR201

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |    MPHC_NETWORK_SYNC_REQ     |
	 |		  *=============================>*	
(3)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(4)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(5)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_637_STD_1800

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_637
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_637
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_1

- MPH_BSIC_CNF
		arfcn	ARFCN_637_STD_1800
		bsic	BSIC_1
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
		radio_freq	ARFCN_637
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		tsc	BSIC_1

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	FULL_READ

History:	22.09.99	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ included

### ALR204: Find BCCH carrier, first channel failed, then second channel

Description:	The carrier with the highest fieldstrength (channel 637) is selected for synchronizing to frequency correction burst and synchron burst. The attempt failed. A second request is started for the next strongest channel (channel 580).

Preamble:	ALR201

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_NETWORK_SYNC_REQ     |
	 |		  *=============================>*	
(3)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(4)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(5)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(6)	 |		  |	MPHC_NETWORK_SYNC_REQ     |
	 |		  *=============================>*	
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_637_STD_1800

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_637
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_637
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_637_STD_1800
		bsic	BSIC_0
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_580_STD_1800

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_580
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

History:	22.09.99	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR205: Find BCCH carrier, all channels failed, then error indication

Description:	All synchronization attempts to the available channels are failed. RR is informed by an error indication with the cause no BCCH available.

Preamble:	ALR204

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(3)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(4)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*	
(5)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*	
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(8)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*	
(9)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*	
(10)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_580
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_580_STD_1800
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_885_STD_1800

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_885
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_885
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_885_STD_1800
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_512_STD_1800

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_512
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_512
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_512_STD_1800
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

History:	22.09.99	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR206: Find BCCH carrier, second channel

Description:	The next best channel (channel 14) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR204

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(3)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_580
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_580_STD_1800
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
		radio_freq	ARFCN_580
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		tsc	BSIC_0

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	FULL_READ

History:	23.09.99	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ included

### ALR207: Read BCCH data

Description:	The BCCH data blocks are read for the channel 23.

Preamble:	ALR203

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(3)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(4)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(5)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(8)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_637
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_637
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_637
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_637
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NEIGH_CELL_DESC_1
		ncc_permit	NCC_PERMIT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_637
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_637
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_637
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4
		tc	TC_3
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_637
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

History:	23.09.99	MPA	Initial

### ALR208: Read failed BCCH data

Description:	A failed BCCH data block is read.

Preamble:	ALR207

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_ERROR_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_637
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_3
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_637

History:	23.09.99	MPA	Initial

### ALR209: RR rejects BCCH carrier, try third channel

Description:	RR rejects the BCCH carrier. The next best channel (channel 124) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR206

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(11)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_885_STD_1800

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_885
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_885
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_885_STD_1800
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
		radio_freq	ARFCN_885
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		tsc	BSIC_0

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	FULL_READ

- MPHC_DATA_IND
		radio_freq	ARFCN_885
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_885
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

History:	23.09.9	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR210: RR rejects BCCH carrier, try fourth channel

Description:	RR rejects the BCCH carrier. The next best channel (channel 1) is selected for synchronising to frequency correction burst and synchron burst.

Preamble:	ALR209

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(11)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_512_STD_1800

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_512
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_512
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_512_STD_1800
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
		radio_freq	ARFCN_512
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		tsc	BSIC_0

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPHC_DATA_IND
		radio_freq	ARFCN_512
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_512
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

History:	23.09.9	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR211: RR rejects BCCH carrier, no further channel available

Description:	RR rejects the BCCH carrier. No further channel is available.

Preamble:	ALR210

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	NOT_PRESENT_16BIT

History:	23.09.99	MPA	Initial

### ALR212: RR select second channel

Description:	RR selects the channel 14 after reading the BCCH carrier.
Variant A: with neighbourcell list
Variant B: with empty neighbour cell list
Variant C: like A, but non-combined CCCH

Preamble:	ALR206

Variants:	<A> .. <C>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_IDENTITY_REQ	  |		  |
	 *===============================>*		  |
(7)	 |	MPH_CBCH_REQ	  |		  |
	 *===============================>*		  |
(8)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(9)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_DCS_1800

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_580
		ext_bcch	NOT_USED
<A>	comb_ccch	COMB_CCCH_COMB
<B>	comb_ccch	COMB_CCCH_COMB
<C>	comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
<A>	bcch_combined	COMB_CCCH_COMB
<B>	bcch_combined	COMB_CCCH_COMB
<C>	bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
<A>	page_block_index	PBI_0
<B>	page_block_index	PBI_0
<C>	page_block_index	PBI_2
 		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_IDENTITY_REQ
		mid	MS_ID_IMSI_TMSI

- MPH_CBCH_REQ
		cbch	NO_CBCH

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
<A>	arfcn	CHLIST_512_637_885_FFFF

- <B>	arfcn	EMPTY_NCELL_LIST
<C>	arfcn	CHLIST_512_637_885_FFFF

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
<A>	chan_list	CHLIST_580_512_637_885
<B>	chan_list	CHLIST_580
<C>	chan_list	CHLIST_580_512_637_885
<A>	num_of_chans	CHANNELS_4
<B>	num_of_chans	CHANNELS_1
<C>	num_of_chans	CHANNELS_4
		ba_id	BA_ID_1
 		next_radio_freq_measured	CHAN_LIST_IDX_0

History:	23.09.99	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

20.07.01	MSB	channel list adapted

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ included

### ALR213: RR select first channel

Description:	RR selects the channel 23 after reading the BCCH carrier.

Preamble:	ALR203

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_IDENTITY_REQ	  |		  |
	 *===============================>*		  |
(7)	 |	MPH_CBCH_REQ	  |		  |
	 *===============================>*		  |
(8)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(9)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_GSM_900

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_637
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
 		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
 		page_block_index	PBI_0
 		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_IDENTITY_REQ
		mid	MS_ID_IMSI_TMSI

- MPH_CBCH_REQ
		cbch	NO_CBCH

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	CHLIST_512_580_885_FFFF					sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_637_512_580_885					num_of_chans	CHANNELS_4
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

History:	23.09.99	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

20.07.01	MSB	channel list adapted

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ included

### ALR215: Re-Initiation of Cell Selection during measurements

Description:	RR has started a cell selection. During power measurements a new activation of cell selection is started by RR. The power measurement is restarted.

Preamble:	ALR200

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(7)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(8)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(9)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(16)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(17)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(19)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(20)	 |	MPH_POWER_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DCS_1800

- MPHC_INIT_L1_REQ
		radio_band_config	STD_1800

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DCS_1800

- MPHC_INIT_L1_REQ
		radio_band_config	STD_1800

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_3_1800

- MPH_POWER_CNF
		num_of_chan	CHANNELS_4
		arfcn	ARFCN_637_580_885_512_WITH_STD
		rx_lev	RXLEV_637_580_885_512

History:	23.09.99	MPA	Initial
	17.05.02	MSB	add MPHC_INIT_L1_REQ/CON, change arfcn´s

### ALR217: Re-Initiation of Cell Selection during BCCH reading

Description:	During BCCH reading RR restarts a cell selection. This leads to a new full power measurement cycle.

Preamble:	ALR207

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_SYNC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(5)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(13)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(15)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(16)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(17)	 |	MPH_POWER_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_SYNC_REQ
 		cs	CS_STOP_BCCH_READING

- MPHC_STOP_SCELL_BCCH_REQ 
		param	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DCS_1800

- MPHC_INIT_L1_REQ
		radio_band_config	STD_1800

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_3_1800

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND	
		shared_ptr	RXLEV_IDX_3_1800

- MPH_POWER_CNF
		num_of_chan	CHANNELS_4
		arfcn	ARFCN_637_580_885_512_WITH_STD
		rx_lev	RXLEV_637_580_885_512

History:	23.09.99	MPA	Initial
	17.05.02	MSB	add MPHC_INIT_L1_REQ/CON, change arfcn´s

### ALR244: Stop Idle Mode by Normal Cell Selection

Description:	The idle mode is stopped if a normal cell selection is initiated.

Preamble:	ALR213

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(3)	 |		  |  MPHC_STOP_CCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  | MPHC_STOP_RXLEV_PERIODIC_REQ |
	 |		  *=============================>*
(5)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DCS_1800

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_STOP_CCCH_REQ
 		param	NOT_USED

- MPHC_STOP_RXLEV_PERIODIC_REQ
		param	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_INIT_L1_REQ
		radio_band_config	STD_1800

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

History:	23.09.99	MPA	Initial
	17.05.02	MSB	add parameter ‘freq_band’

29.10.02	DL	MPHC_STOP_NCELL_SYNC/BCCH_REQ removed

## Cell Selection (Dualband GSM 900 / DCS 1800)

### ALR601: Initiation by RR, 8 channels available

Description:	RR starts a cell selection. Eight channels in the order 23, 637, 14, 580, 124, 885, 1, 512 have acceptable fieldstrength values.

Preamble:	ALR600

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(6)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPHC_RXLEV_IND	 |
	 |		  *<==============================*
(8)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(10)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(14)	 |	MPH_POWER_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DUAL

- MPHC_INIT_L1_REQ
		radio_band_config	STD_DUAL

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPH_POWER_CNF
		num_of_chan	CHANNELS_8
		arfcn	ARFCN_DUAL_WITH_STD
		rx_lev	RXLEV_DUAL

History:	24.01.00	MPA	Initial
	17.05.02	MSB	change arfcn´s

### ALR602: Initiation by RR, no channels available

Description:	RR starts a cell selection. No channel is available.

Preamble:	ALR600

RR/DL		 ALR			 PL
	 |		  |			  |
(1)	 |	MPH_POWER_REQ	  |			  |
	 *===============================>*			  |
(2)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(9)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(14)	 |	MPH_POWER_CNF	  |			  |
	 *<===============================*			  |
	 |		  |			  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DUAL

- MPHC_INIT_L1_REQ
		radio_band_config	STD_DUAL

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_2_DUAL

- MPH_POWER_CNF
		num_of_chan	CHANNELS_0
		arfcn	NOT_USED
		rx_lev	NOT_USED

History:	24.01.00	MPA	Initial

### ALR603: Find BCCH carrier, first channel

Description:	The carrier with the highest fieldstrength (channel 23) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR601

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |    MPHC_NETWORK_SYNC_REQ     |
	 |		  *=============================>*	
(3)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(4)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(5)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_23_STD_DUAL

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_23
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_23
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_1

- MPH_BSIC_CNF
		arfcn	ARFCN_23_STD_DUAL
		bsic	BSIC_1
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
		radio_freq	ARFCN_23
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		tsc	BSIC_1

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	FULL_READ

History:	24.01.00	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR604: Find BCCH carrier, first channel failed, then second channel

Description:	The carrier with the highest fieldstrength (channel 23) is selected for synchronizing to frequency correction burst and synchron burst. The attempt failed. A second request is started for the next strongest channel (channel 637).

Preamble:	ALR601

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_NETWORK_SYNC_REQ     |
	 |		  *=============================>*	
(3)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(4)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(5)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(6)	 |		  |	MPHC_NETWORK_SYNC_REQ     |
	 |		  *=============================>*	
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_23_STD_DUAL

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_23
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_23
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_1

- MPH_BSIC_CNF
		arfcn	ARFCN_23_STD_DUAL
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_637_STD_DUAL

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_637
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

History:	24.01.00	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR605: Find BCCH carrier, all channels failed, then error indication

Description:	All synchronization attempts to the available channels are failed. RR is informed by an error indication with the cause no BCCH available.

Preamble:	ALR604

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(3)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(4)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*	
(5)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*	
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(8)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*	
(9)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*	
(10)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(11)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(12)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*	
(13)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*	
(14)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(15)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(16)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*	
(17)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*	
(18)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(19)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(20)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*	
(21)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*	
(21)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_637
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_637_STD_DUAL
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_14_STD_DUAL

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_14_STD_DUAL
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_580_STD_DUAL

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_580
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_580
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_580_STD_DUAL
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_124_STD_DUAL

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_124
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_124_STD_DUAL
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_885_STD_DUAL

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_885
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_885
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_885_STD_DUAL
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

- MPH_BSIC_REQ
		arfcn	ARFCN_512_STD_DUAL

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_512
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_512
		sb_flag	NO_SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_512_STD_DUAL
		bsic	NOT_USED
		cs	CS_NO_BCCH_AVAIL

History:	24.01.00	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR606: Find BCCH carrier, second channel

Description:	The next best channel (channel 637) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR604

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(3)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_637
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_637_STD_DUAL
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
		radio_freq	ARFCN_637
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		tsc	BSIC_0

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	FULL_READ

History:	24.01.00	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR607: Read BCCH data

Description:	The BCCH data blocks are read for the channel 23.

Preamble:	ALR603

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(3)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(4)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(5)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(8)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NEIGH_CELL_DESC_1
		ncc_permit	NCC_PERMIT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4
		tc	TC_3
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

History:	24.01.00	MPA	Initial

### ALR608: Read failed BCCH data

Description:	A failed BCCH data block is read.

Preamble:	ALR607

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_ERROR_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_3
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_ERROR_IND
		cs	CS_BCCH_READ_ERROR
		arfcn	ARFCN_23

History:	22.01.00	MPA	Initial

### ALR609: RR rejects BCCH carrier, try third channel

Description:	RR rejects the BCCH carrier. The next best channel (channel 14) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR606

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(11)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_14_STD_DUAL

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_14_STD_DUAL
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		tsc	BSIC_0

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	FULL_READ

- MPHC_DATA_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

History:	24.01.00	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR610: RR rejects BCCH carrier, try fourth channel

Description:	RR rejects the BCCH carrier. The next best channel (channel 580) is selected for synchronizing to frequency correction burst and synchron burst.

Preamble:	ALR609

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NETWORK_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_NETWORK_SYNC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_BSIC_CNF	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(11)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_580_STD_DUAL

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_580
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_580
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_0

- MPH_BSIC_CNF
		arfcn	ARFCN_580_STD_DUAL
		bsic	BSIC_0
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
		radio_freq	ARFCN_580
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		tsc	BSIC_0

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPHC_DATA_IND
		radio_freq	ARFCN_580
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_580
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

History:	24.01.00	MPA	Initial
	17.05.02	MSB	adapt MPH_BSIC_REQ/CNF to multiband

### ALR611: RR rejects BCCH carrier, no further channel available

Description:	RR rejects the BCCH carrier. No further channel is available.

Preamble:	ALR610

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_BSIC_REQ	  |		  |
	 *===============================>*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	NOT_PRESENT_16BIT

History:	24.01.00	MPA	Initial

### ALR612: RR select second channel

Description:	RR selects the channel 637 after reading the BCCH carrier.
Variant A: with neighbourcell list
Variant B: with empty neighbour cell list

Preamble:	ALR606

Variants:	<A> .. <B>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_IDENTITY_REQ	  |		  |
	 *===============================>*		  |
(7)	 |	MPH_CBCH_REQ	  |		  |
	 *===============================>*		  |
(8)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(9)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_DUAL

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_637
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_IDENTITY_REQ
		mid	MS_ID_IMSI_TMSI

- MPH_CBCH_REQ
		cbch	NO_CBCH

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_1
<A>	arfcn	CHLIST_1_14_23_124_512_580_885_FFFF

- <B>	arfcn	EMPTY_NCELL_LIST

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
<A>	chan_list	CHLIST_637_1_14_23_124_512_580_885

- <B>	chan_list	CHLIST_637
<A>	num_of_chans	CHANNELS_8
<B>	num_of_chans	CHANNELS_1
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

History:	24.01.00	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

20.07.01	MSB	channel list adapted

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ included

### ALR613: RR select first channel

Description:	RR selects the channel 23 after reading the BCCH carrier.

Preamble:	ALR603

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_IDENTITY_REQ	  |		  |
	 *===============================>*		  |
(7)	 |	MPH_CBCH_REQ	  |		  |
	 *===============================>*		  |
(8)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(9)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_DUAL

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_IDENTITY_REQ
		mid	MS_ID_IMSI_TMSI

- MPH_CBCH_REQ
		cbch	NO_CBCH

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_2
		arfcn	CHLIST_1_14_124_512_580_637_885_FFFF			sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_23_1_14_124_512_580_637_885			num_of_chans	CHANNELS_8
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

History:	24.01.00	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

20.07.01	MSB	channel list adapted

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ included

### ALR615: Re-Initiation of Cell Selection during measurements

Description:	RR has started a cell selection. During power measurements a new activation of cell selection is started by RR. The power measurement is restarted.

Preamble:	ALR600

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(7)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(8)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(9)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(11)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(12)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	MPHC_RXLEV_IND	 |
	 |		  *<==============================*
(16)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(17)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<==============================*
(20)	 |	MPH_POWER_CNF	  |		  |
	 *===============================>*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DUAL

- MPHC_INIT_L1_REQ
		radio_band_config	STD_DUAL

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DUAL

- MPHC_INIT_L1_REQ
		radio_band_config	STD_DUAL

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPH_POWER_CNF
		num_of_chan	CHANNELS_8
		arfcn	ARFCN_DUAL_WITH_STD
		rx_lev	RXLEV_DUAL

History:	24.01.00	MPA	Initial
	17.05.02	MSB	change arfcn´s

21.05.02	MSB	add MPHC_INIT_L1_REQ/CON

### ALR617: Re-Initiation of Cell Selection during BCCH reading

Description:	During BCCH reading RR restarts a cell selection. This leads to a new full power measurement cycle.

Preamble:	ALR607

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_SYNC_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(5)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(13)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(15)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
(16)	 |		  |	MPHC_RXLEV_IND	  |
	 |		  *<=============================*
(17)	 |	MPH_POWER_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_SYNC_REQ
 		cs	CS_STOP_BCCH_READING

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DUAL

- MPHC_INIT_L1_REQ
		radio_band_config	STD_DUAL

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_DUAL

- MPH_POWER_CNF
		num_of_chan	CHANNELS_8
		arfcn	ARFCN_DUAL_WITH_STD
		rx_lev	RXLEV_DUAL

History:	24.01.00	MPA	Initial
	17.05.02	MSB	change arfcn´s

21.05.02	MSB	add MPHC_INIT_L1_REQ/CON

### ALR644: Stop Idle Mode by Normal Cell Selection

Description:	The idle mode is stopped if a normal cell selection is initiated.

Preamble:	ALR613

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_POWER_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(3)	 |		  |  MPHC_STOP_CCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  | MPHC_STOP_RXLEV_PERIODIC_REQ |
	 |		  *=============================>*
(5)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_INIT_L1_REQ	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_INIT_L1_CON	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_POWER_REQ
		pch_interrupt	PCH_INTERRUPT
		freq_bands	BAND_DUAL

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_STOP_CCCH_REQ
 		param	NOT_USED

- MPHC_STOP_RXLEV_PERIODIC_REQ
		param	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_INIT_L1_REQ
		radio_band_config	STD_DUAL

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

History:	24.01.00	MPA	Initial
	17.05.02	MSB	add parameter ‘freq_bands’

21.05.02	MSB	add MPHC_INIT_L1_REQ/CON

29.10.02	DL	MPHC_STOP_NCELL_SYNC/BCCH_REQ removed

## Page Mode Change

### ALR020: Initiation with Paging Reorganisation

Description:	The idle mode is configured by RR. Layer 1 is configured with MPHC_START_CCCH_REQ.

Preamble:	ALR007

Variants:	<A>..<B>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
 	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_COMB
<A>	tn	TN_0
<B>	tn	TN_4
		dlt	DLT_24
		pg	PG_23
		bs_ag_blocks_res	BS_AG_BLKS_RES_2
		bs_pa_mfrms	BS_PA_MFRMS_5
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_7
		bs_ag_blks_res	BS_AG_BLKS_RES_2
		bcch_combined	COMB_CCCH_COMB
<A>	ccch_group	CCCH_GROUP_0
<B>	ccch_group	CCCH_GROUP_2
		page_group	PG_23
		page_block_index	PBI_0
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

History:	07.10.99	MPA	Initial

19.07.01	MSB	use full sc bcch read MPHC_SCELL_NBCCH_REQ
			instead of periodical
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR022: Page Mode Change, Paging Reorganisation

Description:	The idle mode is configured with Paging Reorganisation. After reception of a new paging mode this information is forwarded to PL. The message contain the following page modi:

Variant A: Extended Paging
Variant B: Paging Reorganisation
Variant C: Same as before

No reaction is expected.

Preamble:	ALR020A

Variants:	<A>...<C>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
MUTE (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
<A>	l2_frame	L2_PAGING_REQ_1_EXT
<B>	l2_frame	L2_PAGING_REQ_1_REO
<C>	l2_frame	L2_PAGING_REQ_1_SAB
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	07.10.99	MPA	Initial

### ALR023: Page Mode Change, Normal Paging to Extended Paging

Description:	The idle mode is configured with Normal Paging. After reception of the new paging mode Extended Paging this information is forwarded to the lower layer. The new paging mode is configured for layer 1.

Preamble:	ALR025

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(3) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1_EXT
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_7
		bs_ag_blks_res	BS_AG_BLKS_RES_2
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_2
		page_group	PG_23
		page_block_index	PBI_0
		page_mode	PGM_EXTENDED

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

History:	07.10.99	MPA	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR024: Page Mode Change, Normal Paging to Paging Reorganisation

Description:	The idle mode is configured with Normal Paging. After reception of the new paging mode Paging Reorganisation this information is forwarded to PL. The new paging mode is configured for layer 1.

Preamble:	ALR025

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(6) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1_REO
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_2
		bs_ag_blks_res	BS_AG_BLKS_RES_7
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_0
		page_block_index	PBI_0
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

History:	07.10.99	MPA	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR025: Page Mode Change, Back to Normal Paging

Description:	The idle mode has been configured for normal paging. After a swap to Paging Reorganisation RR re-configures normal paging.

Preamble:	ALR020B

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_7
		bs_ag_blks_res	BS_AG_BLKS_RES_2
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_2
		page_group	PG_23
		page_block_index	PBI_0
		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

History:	07.10.99	MPA	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR026: Page Mode Change, Normal Paging

Description:	The idle mode is configured with Normal Paging. The messages contain the following page modi:

Variant A: Normal Paging
Variant B: Same as before.

No reaction is exspected.

Preamble:	ALR025

Variants:	<A>...<B>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
MUTE (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
<A>	l2_frame	L2_PAGING_REQ_1
<B>	l2_frame	L2_PAGING_REQ_1_SAB
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	07.10.99	MPA	Initial

### ALR027: Page Mode Change, Extended Paging a second time

Description:	The idle mode has been configured for normal paging. After a swap to Extended Paging this mode is configured a second time.

Variant A: Extended Paging

Variant B: Same as before

Layer 1 must not configured the second time, because it´s a continues process in ALR(MPHC).

Preamble:	ALR025

Variants:	<A>...<B>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(3) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
MUTE (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1_EXT
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_7
		bs_ag_blks_res	BS_AG_BLKS_RES_2
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_2
		page_group	PG_23
		page_block_index	PBI_0
		page_mode	PGM_EXTENDED

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
	<A>	l2_frame	L2_PAGING_REQ_1_EXT
	<B>	l2_frame	L2_PAGING_REQ_1_SAB
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	07.10.99	MPA	Initial

20.06.01	MSB	Layer 1 must not configured the second time

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR028: Page Mode Change, Extended Paging to Paging Reorganisation

Description:	The idle mode has been configured for normal paging. After a swap to extended paging paging reorganisation is configured.

Preamble:	ALR025

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(3) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(6) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1_EXT
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_7
		bs_ag_blks_res	BS_AG_BLKS_RES_2
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_2
		page_group	PG_23
		page_block_index	PBI_0
		page_mode	PGM_EXTENDED

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1_REO
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_2
		bs_ag_blks_res	BS_AG_BLKS_RES_7
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_0
		page_block_index	PBI_0
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

History:	07.10.99	MPA	Initial

20.06.01	MSB	Normal paging will be configure with 025

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR029: Page Mode Change, Extended Paging to Normal Paging

Description:	The idle mode has been configured for normal paging. After reception of the new paging mode normal paging this mode is configured.

Preamble:	ALR025

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(3) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(6) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1_EXT
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_7
		bs_ag_blks_res	BS_AG_BLKS_RES_2
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_2
		page_group	PG_23
		page_block_index	PBI_0
		page_mode	PGM_EXTENDED

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_7
		bs_ag_blks_res	BS_AG_BLKS_RES_2
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_2
		page_group	PG_23
		page_block_index	PBI_0
		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

History:	07.10.99	MPA	Initial

20.06.01	MSB	reconfiguration to normal paging

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR700: Page Mode Change according 26.6.2.3.1

Description:	The idle mode is configured with Paging Reorganisation. After reception of the paging mode Paging Reorganisation no change occurs (received with an immediate assignment extended message. Then the mobile is paged with a paging request 2 message. It is expected that the paging is detected by the mobile.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(3)	 |	MPH_PAGING_IND	  |		  |
	 *<===============================*		  |
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_IMM_ASS_EXT_REO
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAG_2_T1_A
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_PAGING_IND
		identity_type	ID_TMSI
		channel_needed	CN_ANY_CHAN

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_0
		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

History:	07.10.99	MPA	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

## Paging

### ALR070: Paging Req 1, Empty Paging Message

Description:	The MS is in idle mode. It receives an empty paging message. The message content is not forwarded to RR.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAG_1_EMPTY
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	08.10.99	MPA	Initial

### ALR071: Paging Req 1, IMSI / TMSI for MS

Description:	The MS is in idle mode. It receives a paging message corresponding to the actual IMSI or TMSI. A paging indication is forwarded to RR.
Variant A: IMSI mobile identity 1, any channel needed
Variant B: IMSI mobile identity 2, any channel needed
Variant C: TMSI mobile identity 1, TCH/F channel needed
Variant D: TMSI mobile identity 2, TCH/F channel needed
Variant E: IMSI mobile identity 1, SDCCH channel needed
Variant F: IMSI mobile identity 2, SDCCH channel needed
Variant G: TMSI mobile identity 1, Dual Rate channel needed
Variant H: TMSI mobile identity 2, Dual Rate channel needed

Preamble:	ALR013

Variants:	<A>..<H>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_PAGING_IND	  |		  |
	 *<===============================*		  |
(3)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
<A>	l2_frame	L2_PAG_1_I1_A
<B>	l2_frame	L2_PAG_1_I2_A
<C>	l2_frame	L2_PAG_1_T1_T
<D>	l2_frame	L2_PAG_1_T2_T
<E>	l2_frame	L2_PAG_1_I1_S
<F>	l2_frame	L2_PAG_1_I2_S
<G>	l2_frame	L2_PAG_1_T1_D
<H>	l2_frame	L2_PAG_1_T2_D
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_PAGING_IND
<A>	identity_type	ID_TYPE_IMSI
<B>	identity_type	ID_TYPE_IMSI
<C>	identity_type	ID_TYPE_TMSI
<D>	identity_type	ID_TYPE_TMSI
<E>	identity_type	ID_TYPE_IMSI
<F>	identity_type	ID_TYPE_IMSI
<G>	identity_type	ID_TYPE_TMSI
<H>	identity_type	ID_TYPE_TMSI
<A>	channel_needed	CN_ANY_CHAN
<B>	channel_needed	CN_ANY_CHAN
<C>	channel_needed	CN_TCH_F
<D>	channel_needed	CN_TCH_F
<E>	channel_needed	CN_SDCCH
<F>	channel_needed	CN_SDCCH
<G>	channel_needed	CN_TCH
<H>	channel_needed	CN_TCH

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_0
		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

History:	08.10.99	MPA	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR072: Paging Req 1, Not for MS

Description:	The MS is in idle mode. It receives paging request type 1 messages. The message content is not forwarded to RR, because the message content doesn´t match to the MS identities.
Variant A: wrong IMSI mobile identity 1
Variant B: wrong TMSI mobile identity 1
Variant C: wrong IMSI mobile identity 2
Variant D: wrong TMSI mobile identity 2
Variant E: wrong type mobile identity 2

Preamble:	ALR013

Variants:	<A>..<E>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
<A>	l2_frame	L2_PAG_1_WI1
<B>	l2_frame	L2_PAG_1_WT1
<C>	l2_frame	L2_PAG_1_WI2
<D>	l2_frame	L2_PAG_1_WT2
<E>	l2_frame	L2_PAG_1_WTYPE
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	08.10.99	MPA	Initial

### ALR073: Paging Req 1, Short IMSI / TMSI for MS

Description:	The MS is in idle mode. It receives a paging message corresponding to the actual IMSI or TMSI. A paging indication is forwarded to RR. The IMSI is less than 15 digits and the TMSI has only one valid byte, but is signalled in different ways by the infrastructure
Variant A: Short TMSI mobile identity 1, any channel needed, 1 byte length from the network
Variant B: Short TMSI mobile identity 2, TCH/F needed, 3 bytes length from the network
Variant C: Short IMSI mobile identity 1, SDCCH needed, 10 digits
Variant D: Short IMSI mobile identity 2, Dual Rate needed, 10 digits
Variant E: Short TMSI mobile identity 1, any channel needed, 2 bytes length from the network
Variant F: Short TMSI mobile identity 2, TCH/F needed, 4 bytes length from the network

Preamble:	ALR013

Variants:	<A>..<F>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_IDENTITY_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(3)	 |	MPH_PAGING_IND	  |		  |
	 *<===============================*		  |
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_IDENTITY_REQ
		mid	MS_ID_SHORT_IMSI_TMSI

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
<A>	l2_frame	L2_PAG_1_ST1_A1
<B>	l2_frame	L2_PAG_1_ST2_T3
<C>	l2_frame	L2_PAG_1_SI1_S
<D>	l2_frame	L2_PAG_1_SI2_D
<E>	l2_frame	L2_PAG_1_ST1_A2
<F>	l2_frame	L2_PAG_1_ST2_T4
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_PAGING_IND
<A>	identity_type	ID_TYPE_TMSI
<B>	identity_type	ID_TYPE_TMSI
<C>	identity_type	ID_TYPE_IMSI
<D>	identity_type	ID_TYPE_IMSI
<E>	identity_type	ID_TYPE_TMSI
<F>	identity_type	ID_TYPE_TMSI
<A>	channel_needed	CN_ANY_CHAN
<B>	channel_needed	CN_TCH_F
<C>	channel_needed	CN_SDCCH
<D>	channel_needed	CN_TCH
<E>	channel_needed	CN_ANY_CHAN
<F>	channel_needed	CN_TCH_F

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_0
		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

History:	08.10.99	MPA	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ moved

### ALR074: Paging Req 2, Empty Paging Message

Description:	The MS is in idle mode. It receives an empty paging message. The message content is not forwarded to RR.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAG_2_EMPTY
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	08.10.99	MPA	Initial

### ALR075: Paging Req 2, TMSI for MS (Mobile Identity 1 or 2)

Description:	The MS is in idle mode. It receives a paging message corresponding to the actual TMSI in the information elements mobile identity 1 or 2. A paging indication is forwarded to RR.
Variant A: TMSI mobile identity 1, any channel needed
Variant B: TMSI mobile identity 1, SDCCH needed
Variant C: TMSI mobile identity 1, TCH/F needed
Variant D: TMSI mobile identity 1, Dual Rate needed
Variant E: TMSI mobile identity 2, any channel needed
Variant F: TMSI mobile identity 2, SDCCH needed
Variant G: TMSI mobile identity 2, TCH/F needed
Variant H: TMSI mobile identity 2, Dual Rate needed

Preamble:	ALR013

Variants:	<A>..<H>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_PAGING_IND	  |		  |
	 *<===============================*		  |
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
 (5) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
<A>	l2_frame	L2_PAG_2_T1_A
<B>	l2_frame	L2_PAG_2_T1_S
<C>	l2_frame	L2_PAG_2_T1_T
<D>	l2_frame	L2_PAG_2_T1_D
<E>	l2_frame	L2_PAG_2_T2_A
<F>	l2_frame	L2_PAG_2_T2_S
<G>	l2_frame	L2_PAG_2_T2_T
<H>	l2_frame	L2_PAG_2_T2_D
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_PAGING_IND
		identity_type	ID_TYPE_TMSI
<A>	channel_needed	CN_ANY_CHAN
<B>	channel_needed	CN_SDCCH
<C>	channel_needed	CN_TCH_F
<D>	channel_needed	CN_TCH
<E>	channel_needed	CN_ANY_CHAN
<F>	channel_needed	CN_SDCCH
<G>	channel_needed	CN_TCH_F
<H>	channel_needed	CN_TCH

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_0
		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

History:	08.10.99	MPA	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
				MPHC_START_CCCH_REQ moved

### ALR076: Paging Req 2, Not for MS

Description:	The MS is in idle mode. It receives paging request type 2 messages. The message content is not forwarded to RR, because the message content doesn´t match to the MS identities.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAG_2_WRONG
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	08.10.99	MPA	Initial

### ALR077: Paging Req 2, IMSI / TMSI for MS (Mobile Identity 3)

Description:	The MS is in idle mode. It receives a paging request type 2 message corresponding to the actual IMSI or TMSI in information element in the information element mobile identity 3. A paging indication is forwarded to RR. 
Variant A: IMSI mobile identity 3, any channel needed 
Variant B: IMSI mobile identity 3, SDCCH needed
Variant C: IMSI mobile identity 3, TCH/F needed 
Variant D: IMSI mobile identity 3, Dual Rate needed
Variant E: IMSI mobile identity 3, no channel indication
Variant F: TMSI mobile identity 3, any channel needed
Variant G: TMSI mobile identity 3, SDCCH needed,
Variant H: TMSI mobile identity 3, TCH/F needed
Variant I: TMSI mobile identity 3, Dual Rate needed
Variant J: TMSI mobile identity 3, no channel indication

Preamble:	ALR013

Variants:	<A>..<J>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_PAGING_IND	  |		  |
	 *<===============================*		  |
(3)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(4) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
<A>	l2_frame	L2_PAG_2_I3_A
<B>	l2_frame	L2_PAG_2_I3_S
<C>	l2_frame	L2_PAG_2_I3_T
<D>	l2_frame	L2_PAG_2_I3_D
<E>	l2_frame	L2_PAG_2_I3_N
<F>	l2_frame	L2_PAG_2_T3_A
<G>	l2_frame	L2_PAG_2_T3_S
<H>	l2_frame	L2_PAG_2_T3_T
<I>		l2_frame	L2_PAG_2_T3_D
<J>	l2_frame	L2_PAG_2_T3_N
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_PAGING_IND
<A>	identity_type	ID_TYPE_IMSI
<B>	identity_type	ID_TYPE_IMSI
<C>	identity_type	ID_TYPE_IMSI
<D>	identity_type	ID_TYPE_IMSI
<E>	identity_type	ID_TYPE_IMSI
<F>	identity_type	ID_TYPE_TMSI
<G>	identity_type	ID_TYPE_TMSI
<H>	identity_type	ID_TYPE_TMSI
<I>		identity_type	ID_TYPE_TMSI
<J>	identity_type	ID_TYPE_TMSI
<A>	channel_needed	CN_ANY_CHAN
<B>	channel_needed	CN_SDCCH
<C>	channel_needed	CN_TCH_F
<D>	channel_needed	CN_TCH
<E>	channel_needed	CN_ANY_CHAN
<F>	channel_needed	CN_ANY_CHAN
<G>	channel_needed	CN_SDCCH
<H>	channel_needed	CN_TCH_F
<I>		channel_needed	CN_TCH
<J>	channel_needed	CN_ANY_CHAN

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_0
		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

History:	08.10.99	MPA	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR078: Paging Req 2, Short IMSI / TMSI for MS (Mobile Identity 3)

Description:	The MS is in idle mode. It receives a paging request type 2 message corresponding to the actual IMSI or TMSI in information element in the information element mobile identity 3. A paging indication is forwarded to RR. The IMSI is less than 15 digits and the TMSI has only one valid byte, but is signalled in different ways by the infrastructure
Variant A: Short IMSI mobile identity 3, any channel needed, 10 digits
Variant B: Short TMSI mobile identity 3, SDCCH needed, 1 byte from the network
Variant C: Short TMSI mobile identity 3, TCH/F needed, 2 bytes from the network
Variant D: Short TMSI mobile identity 3, Dual Rate needed, 3 bytes from the network
Variant E: Short TMSI mobile identity 3, No channel indication, 4 bytes from the network

Preamble:	ALR013

Variants:	<A>..<E>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_IDENTITY_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(3)	 |	MPH_PAGING_IND	  |		  |
	 *<===============================*		  |
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_IDENTITY_REQ
		mid	MS_ID_SHORT_IMSI_TMSI

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
<A>	l2_frame	L2_PAG_2_SI3_A
<B>	l2_frame	L2_PAG_2_ST3_S
<C>	l2_frame	L2_PAG_2_ST3_T
<D>	l2_frame	L2_PAG_2_ST3_D
<E>	l2_frame	L2_PAG_2_ST3_N
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_PAGING_IND
<A>	identity_type	ID_TYPE_IMSI
<B>	identity_type	ID_TYPE_TMSI
<C>	identity_type	ID_TYPE_TMSI
<D>	identity_type	ID_TYPE_TMSI
<E>	identity_type	ID_TYPE_TMSI
<A>	channel_needed	CN_ANY_CHAN
<B>	channel_needed	CN_SDCCH
<C>	channel_needed	CN_TCH_F
<D>	channel_needed	CN_TCH
<E>	channel_needed	CN_ANY_CHAN

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_0
		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

History:	08.10.99	MPA	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR079: Paging Req 3, Empty Paging Message

Description:	The MS is in idle mode. It receives an empty paging message. The message content is not forwarded to RR.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAG_3_EMPTY
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	08.10.99	MPA	Initial

### ALR080: Paging Req 3, TMSI for MS

Description:	The MS is in idle mode. It receives a paging message corresponding to the actual TMSI in the information elements mobile identity 1 to 4. A paging indication is forwarded to RR.
Variant A: TMSI mobile identity 1, any channel needed
Variant B: TMSI mobile identity 1, SDCCH needed
Variant C: TMSI mobile identity 1, TCH/F needed
Variant D: TMSI mobile identity 1, Dual Rate needed
Variant E: TMSI mobile identity 2, any channel needed
Variant F: TMSI mobile identity 2, SDCCH needed
Variant G: TMSI mobile identity 2, TCH/F needed
Variant H: TMSI mobile identity 2, Dual Rate needed
Variant I: TMSI mobile identity 3, any channel needed
Variant J: TMSI mobile identity 3, SDCCH needed
Variant K: TMSI mobile identity 3, TCH/F needed
Variant L: TMSI mobile identity 3, Dual Rate needed
Variant M: TMSI mobile identity 3, no channel indication
Variant N: TMSI mobile identity 4, any channel needed
Variant O: TMSI mobile identity 4, SDCCH needed
Variant P: TMSI mobile identity 4, TCH/F needed
Variant Q: TMSI mobile identity 4, Dual Rate needed
Variant R: TMSI mobile identity 4, no channel indication

Preamble:	ALR013

Variants:	<A>..<R>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_PAGING_IND	  |		  |
	 *<===============================*		  |
(3)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
<A>	l2_frame	L2_PAG_3_T1_A
<B>	l2_frame	L2_PAG_3_T1_S
<C>	l2_frame	L2_PAG_3_T1_T
<D>	l2_frame	L2_PAG_3_T1_D
<E>	l2_frame	L2_PAG_3_T2_A
<F>	l2_frame	L2_PAG_3_T2_S
<G>	l2_frame	L2_PAG_3_T2_T
<H>	l2_frame	L2_PAG_3_T2_D
<I>		l2_frame	L2_PAG_3_T3_A
<J>	l2_frame	L2_PAG_3_T3_S
<K>	l2_frame	L2_PAG_3_T3_T
<L>	l2_frame	L2_PAG_3_T3_D
<M>	l2_frame	L2_PAG_3_T3_N
<N>	l2_frame	L2_PAG_3_T4_A
<O>	l2_frame	L2_PAG_3_T4_S
<P>	l2_frame	L2_PAG_3_T4_T
<Q>	l2_frame	L2_PAG_3_T4_D
<R>	l2_frame	L2_PAG_3_T4_N
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_PAGING_IND
		identity_type	ID_TYPE_TMSI
<A>	channel_needed	CN_ANY_CHAN
<B>	channel_needed	CN_SDCCH
<C>	channel_needed	CN_TCH_F
<D>	channel_needed	CN_TCH
<E>	channel_needed	CN_ANY_CHAN
<F>	channel_needed	CN_SDCCH
<G>	channel_needed	CN_TCH_F
<H>	channel_needed	CN_TCH
<I>		channel_needed	CN_ANY_CHAN
<J>	channel_needed	CN_SDCCH
<K>	channel_needed	CN_TCH_F
<L>	channel_needed	CN_TCH
<M>	channel_needed	CN_ANY_CHAN
<N>	channel_needed	CN_ANY_CHAN
<O>	channel_needed	CN_SDCCH
<P>	channel_needed	CN_TCH_F
<Q>	channel_needed	CN_TCH
<R>	channel_needed	CN_ANY_CHAN

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_0
		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

History:	08.10.99	MPA	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR081: Paging Req 3, Not for MS

Description:	The MS is in idle mode. It receives paging request type 3 messages. The message content is not forwarded to RR, because the message content doesn´t match to the MS identities.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAG_3_WRONG
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	08.10.99	MPA	Initial

## Measurement Reporting

### ALR030: Measurement Reporting, Configuration

Description:	The idle mode is configured for different multiframe periods.

Variant A:	bs_pa_mfrms = 2
Variant B:	bs_pa_mfrms = 3
Variant C:	bs_pa_mfrms = 4
Variant D:	bs_pa_mfrms = 5
Variant E:	bs_pa_mfrms = 6
Variant F:	bs_pa_mfrms = 7
Variant G:	bs_pa_mfrms = 8
Variant H:	bs_pa_mfrms = 9

Preamble:	ALR007

Variants:	<A>...<H>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(5)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(6)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_IDLE_REQ
		mod	NOT_USED
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_24
		pg	PG_11
		bs_ag_blocks_res	BS_AG_BLKS_RES_2
<A>	bs_pa_mfrms	BS_PA_MFRMS_0
<B>	bs_pa_mfrms	BS_PA_MFRMS_1
<C>	bs_pa_mfrms	BS_PA_MFRMS_2
<D>	bs_pa_mfrms	BS_PA_MFRMS_3
<E>	bs_pa_mfrms	BS_PA_MFRMS_4
<F>	bs_pa_mfrms	BS_PA_MFRMS_5
<G>	bs_pa_mfrms	BS_PA_MFRMS_6
<H>	bs_pa_mfrms	BS_PA_MFRMS_7
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
<A>	bs_pa_mfrms	BS_PA_MFRMS_2
<B>	bs_pa_mfrms	BS_PA_MFRMS_3
<C>	bs_pa_mfrms	BS_PA_MFRMS_4
<D>	bs_pa_mfrms	BS_PA_MFRMS_5
<E>	bs_pa_mfrms	BS_PA_MFRMS_6
<F>	bs_pa_mfrms	BS_PA_MFRMS_7
<G>	bs_pa_mfrms	BS_PA_MFRMS_8
<H>	bs_pa_mfrms	BS_PA_MFRMS_9
		bs_ag_blks_res	BS_AG_BLKS_RES_2
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_11
		page_block_index	PBI_4
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	EMPTY_NCELL_LIST

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_23
		num_of_chans	CHANNELS_1
		ba_id	BA_ID_1
 		next_radio_freq_measured	CHAN_LIST_IDX_0

History:	08.10.99	MPA	Initial

07.02.02	LG	changed value for ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR031: Measurement Reporting, BS_PA_MFRMS = 2

Description:	Measurement reporting for the serving cell is tested. The multiframe period is set to two multiframes. It is exspected that the initial report to RR is send after eleven reports of layer 1 and successive reports are send to RR after ten reports. The number of TDMA frames between measurement reports to RR is 102 TDMA frames which is equal to two multiframes.

Preamble:	ALR030A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(12)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(13)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(15)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(16)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(20)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(21)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(23)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_1020
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_102
		ncells	NOT_USED
		gprs_sync	NOT_USED

History:	08.10.99	MPA	Initial

08.06.01	MSB	fn_offset in (12) corrected

07.02.02	LG	changed value for ba_id

### ALR032: Measurement Reporting, BS_PA_MFRMS = 3

Description:	Measurement reporting for the serving cell is tested. The multiframe period is set to three multiframes. It is exspected that the initial report to RR is send after eight reports of layer 1 and successive reports are send to RR after seven reports. The number of TDMA frames between measurement reports to RR is 153 TDMA frames which is equal to three multiframes.

Preamble:	ALR030B

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(13)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(15)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(16)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(17)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_1071
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_153
		ncells	NOT_USED
		gprs_sync	NOT_USED

History:	08.10.99	MPA	Initial

08.06.01	MSB	fn_offset in (9) corrected

07.02.02	LG	changed value for ba_id

### ALR033: Measurement Reporting, BS_PA_MFRMS = 4

Description:	Measurement reporting for the serving cell is tested. The multiframe period is set to four multiframes. It is exspected that the initial report to RR is send after six reports of layer 1 and successive reports are send to RR after five reports. The number of TDMA frames between measurement reports to RR is 204 TDMA frames which is equal to four multiframes.

Preamble:	ALR030C

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(7)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(13)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_1020
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_204
		ncells	NOT_USED
		gprs_sync	NOT_USED

History:	08.10.99	MPA	Initial

08.06.01	MSB	fn_offset in (7) corrected

07.02.02	LG	changed value for ba_id

### ALR034: Measurement Reporting, BS_PA_MFRMS = 5

Description:	Measurement reporting for the serving cell is tested. The multiframe period is set to five multiframes. It is exspected that the initial report to RR is send after five reports of layer 1 and successive reports are send to RR after four reports. The number of TDMA frames between measurement reports to RR is 255 TDMA frames which is equal to five multiframes.

Preamble:	ALR030D

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_1020
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_255
		ncells	NOT_USED
		gprs_sync	NOT_USED

History:	08.10.99	MPA	Initial

08.06.01	MSB	fn_offset in (6) corrected

07.02.02	LG	changed value for ba_id

### ALR035: Measurement Reporting, BS_PA_MFRMS = 6

Description:	Measurement reporting for the serving cell is tested. The multiframe period is set to six multiframes. It is exspected that the initial report to RR is send after five reports of layer 1 and successive reports are send to RR after three reports. The number of TDMA frames between measurement reports to RR is 306 TDMA frames which is equal to six multiframes.

Preamble:	ALR030E

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(10)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NOT_USED
		gprs_sync	NOT_USED

History:	08.10.99	MPA	Initial

08.06.01	MSB	fn_offset in (6) corrected

07.02.02	LG	changed value for ba_id

### ALR036: Measurement Reporting, BS_PA_MFRMS = 7

Description:	Measurement reporting for the serving cell is tested. The multiframe period is set to seven multiframes. It is exspected that the initial report to RR is send after five reports of layer 1 and successive reports are send to RR after three reports. The number of TDMA frames between measurement reports to RR is 357 TDMA frames which is equal to seven multiframes.

Preamble:	ALR030F

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(10)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_1071
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_357
		ncells	NOT_USED
		gprs_sync	NOT_USED

History:	08.10.99	MPA	Initial

08.06.01	MSB	fn_offset in (6) corrected

07.02.02	LG	changed value for ba_id

### ALR037: Measurement Reporting, BS_PA_MFRMS = 8

Description:	Measurement reporting for the serving cell is tested. The multiframe period is set to eight multiframes. It is exspected that the initial report to RR is send after five reports of layer 1 and successive reports are send to RR after two reports. The number of TDMA frames between measurement reports to RR is 408 TDMA frames which is equal to eight multiframes.

Preamble:	ALR030G

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_816
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_408
		ncells	NOT_USED
		gprs_sync	NOT_USED

History:	08.10.99	MPA	Initial

08.06.01	MSB	fn_offset in (6) corrected

07.02.02	LG	changed value for ba_id

### ALR038: Measurement Reporting, BS_PA_MFRMS = 9

Description:	Measurement reporting for the serving cell is tested. The multiframe period is set to nine multiframes. It is exspected that the initial report to RR is send after five reports of layer 1 and successive reports are send to RR after two reports. The number of TDMA frames between measurement reports to RR is 459 TDMA frames which is equal to nine multiframes.

Preamble:	ALR030H

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_459
		ncells	NOT_USED
		gprs_sync	NOT_USED

History:	08.10.99	MPA	Initial

08.06.01	MSB	fn_offset in (6) corrected

07.02.02	LG	changed value for ba_id

### ALR062: Extended Measurement Reporting, start procedure

Description:	The extended measurement procedure according GSM 05.08, section 10.1.5 is started.

Preamble:	ALR030H

RR/DL		ALR		PL
	|		|		|

- |	MPH_EXT_MEAS_REQ	|		|
	*=============================>*		|

- |		|	MPHC_INIT_L1_REQ	|
	|		*==============================>*

- |		|	MPHC_INIT_L1_CON	|
	|		*<==============================*

- |		|	MPHC_RXLEV_REQ	|
	|		*==============================>*

|		|		|

Parametrization

- Primitive	Parameter	Value

- MPH_EXT_MEAS_REQ
		pch_interrupt	NO_PCH_INTERRUPT
		freq_bands	BAND_E_GSM
		num_of_chan	CHANNELS_4
		arfcn	ARFCN_EXT_MEAS_000

- MPHC_INIT_L1_REQ
		radio_band_config	STD_EGSM

- MPHC_INIT_L1_CON
		param	NOT_USED

- MPHC_RXLEV_REQ
		shared_ptr	NOT_USED

History:	12.04.02	VK	Initial

29.10.02	DL	MPHC_STOP_NCELL_SYNC/BCCH_REQ removed

### ALR063: Extended Measurement Reporting, successful power measurement

Description:	The extended measurement procedure passes the step of the power measurement.

Preamble:	ALR062

RR/DL		ALR		PL
	|		|		|

- |		|	MPHC_RXLEV_IND	|
	|		*<==============================*

- |	MPH_EXT_MEAS_CNF	|		|
	*<=============================*		|

- |	MPH_BSIC_REQ	|		|
	*=============================>*		|

- |		|	MPHC_NCELL_SYNC_REQ	|
	|		*==============================>*

|		|		|

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_7

- MPH_EXT_MEAS_CNF
		num_of_chan	CHANNELS_10
		arfcn	ARFCN_EXT_MEAS_SORT_000
		rx_lev	RXLEV_EXT_MEAS_000

- MPH_BSIC_REQ
		arfcn	CONST_1023

- MPHC_NCELL_SYNC_REQ
		radio_freq	CONST_1023
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

History:	15.04.02	VK	Initial

### ALR064: Extended Measurement Reporting, successful end

Description:	During the extended measurement procedure the BSIC of a carrier is successfully obtained.

Preamble:	ALR063

RR/DL		ALR		PL
	|		|		|

- |		|	MPHC_NCELL_SYNC_IND	|
	|		*<==============================*

- |	MPH_BSIC_CNF	|		|
	*<=============================*		|

- |	MPH_SYNC_REQ	|		|
	*=============================>*		|

- |	MPH_SYNC_IND	|		|
	*<=============================*		|

|		|		|

Parametrization

- Primitive	Parameter	Value

- MPHC_NCELL_SYNC_IND
		radio_freq	CONST_1023
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	CONST_12
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPH_BSIC_CNF
		arfcn	CONST_1023
		bsic	CONST_12
		cs	CS_NO_ERROR

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- MPH_SYNC_IND
		cs	CS_STOP_PLMN_SEARCH
		arfcn	NOT_USED

History:	15.04.02	VK	Initial
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR065: Extended Measurement Reporting, premature end during power measurement

Description:	The extended measurement procedure is stopped during the power maesurement.

Preamble:	ALR062

RR/DL		ALR		PL
	|		|		|

- |	MPH_SYNC_REQ	|		|
	*=============================>*		|

- |		|	MPHC_RXLEV_IND	|
	|		*<==============================*

- |	MPH_SYNC_IND	|		|
	*<=============================*		|

|		|		|

Parametrization

- Primitive	Parameter	Value

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- MPHC_RXLEV_IND
		shared_ptr	RXLEV_IDX_7

- MPH_SYNC_IND
		cs	CS_STOP_PLMN_SEARCH
		arfcn	NOT_USED

History:	15.04.02	VK	Initial

### ALR066: Extended Measurement Reporting, premature end during BSIC scan

Description:	The extended measurement procedure is stopped during the carrier synchronosation attempt.

Preamble:	ALR063

RR/DL		ALR		PL
	|		|		|

- |	MPH_SYNC_REQ	|		|
	*=============================>*		|

- |		|	MPHC_NCELL_SYNC_IND	|
	|		*<==============================*

- |	MPH_SYNC_IND	|		|
	*<=============================*		|

|		|		|

Parametrization

- Primitive	Parameter	Value

- MPH_SYNC_REQ
		cs	CS_STOP_PLMN_SEARCH

- MPHC_NCELL_SYNC_IND
		radio_freq	CONST_1023
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	CONST_12
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPH_SYNC_IND
		cs	CS_STOP_PLMN_SEARCH
		arfcn	NOT_USED

History:	15.04.02	VK	Initial
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

## BCCH Reading

### ALR039: BCCH Reading, BS_PA_MFRMS = 5

Description:	For a multiframe period of five multiframes it is expected that all 25 reports from layer 1 reading of BCCH information is started. Nine reports are received in the preamble. All four reports a measurement report is send to RR.

Preamble:	ALR034

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(5)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(6)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(10)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(11)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(13)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(15)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(16)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(20)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(21)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(23)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(24)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(26)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(27)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(28)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_255
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_255
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_255
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_255
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_2
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4
		tc	TC_3
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_4
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
 		tc	TC_5
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_6
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_7
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	14.10.99	MPA	Initial

07.02.02	LG	changed value for ba_id

### ALR040: BCCH Reading, BS_PA_MFRMS = 7

Description:	For a multiframe period of seven multiframes it is expected that all 18 reports from layer 1 reading of BCCH information is started. Eight reports are received in the preamble. All three reports a measurement report is send to RR.

Preamble:	ALR036

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(4)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(9)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(12)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(13)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(15)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(16)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(17)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(20)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(21)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_357
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_357
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_357
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_2
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4
		tc	TC_3
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_4
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_5
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_6
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_7
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	14.10.99	MPA	Initial

07.02.02	LG	changed value for ba_id

### ALR041: BCCH Reading, BS_PA_MFRMS = 9

Description:	For a multiframe period of nine multiframes it is expected that all 14 reports from layer 1 reading of BCCH information is started. Seven reports are received in the preamble. All two reports a measurement report is send to RR.

Preamble:	ALR038

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(4)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(13)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(15)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(16)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(17)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_459
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_459
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_459
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_2
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4
		tc	TC_3
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_4
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_5
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_6
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_7
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	14.10.99	MPA	Initial

07.02.02	LG	changed value for ba_id

### ALR082: BCCH Reading, Changed Sys Infos

Description:	For a multiframe period of five multiframes it is expected that all 25 reports from layer 1 reading of BCCH information is started. Nine reports are received in the preamble. All four reports a measurement report is send to RR. Changed System Information Messages are forwarded to RR.

Preamble:	ALR034

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(5)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(6)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(10)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(11)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(13)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(15)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(16)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(20)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(21)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(23)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(24)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(25)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(26)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(27)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(28)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(29)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(30)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_255
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_255
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_255
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_NO_CONTENT
		nbr_of_carriers	CHANNELS_0
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_255
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1_NEW
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1_NEW
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4
		tc	TC_3
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_4
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_6
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_7
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	14.10.99	MPA	Initial

07.02.02	LG	changed value for ba_id

## Connection Establishment

### ALR055: Start of Sending Channel Request Messages (GSM 900)

Description:	RR starts the sending of channel request messages. Idle mode is stopped and the access mode is started. There is one retransmission configured.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_RANDOM_ACCESS_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_STOP_RXLEV_PERIODIC_REQ	  |
	 |		  *===============================>*
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *===============================>*
(4)	 |		  |	MPHC_RA_REQ	  |
	 |		  *===============================>*
(5)	 |		  |	MPHC_RA_CON	  |
	 |		  *<===============================*
(6)	 |	MPH_RANDOM_ACCESS_CNF	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RA_REQ	  |
	 |		  *===============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_REQ
		send_mode	TWO_BURSTS

- MPHC_STOP_RXLEV_PERIODIC_REQ
 		param	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_RA_REQ
		txpwr	POWER_12
		rand	RAND_BURST_1
		channel_request	CHANNEL_REQUEST_1
		powerclass_gsm	POWER_CLASS_5
		powerclass_dcs	NOT_USED

- MPHC_RA_CON
		fn	FN_BURST_1
		channel_request	CHANNEL_REQUEST_1

- MPH_RANDOM_ACCESS_CNF
		frame_no	T123_BURST_1

- MPHC_RA_REQ
		txpwr	POWER_12
		rand	RAND_BURST_2
		channel_request	CHANNEL_REQUEST_2
		powerclass_gsm	POWER_CLASS_5
		powerclass_dcs	NOT_USED

History:	10.11.99	LE	Initial

07.02.02	LG	removed MPHC_START_CCCH_REQ (ALR-FIX-4650)

29.10.02	DL	MPHC_STOP_NCELL_SYNC/BCCH_REQ removed

### ALR056: Immediate Assignment for the Mobile Station

Description:	An immediate assignment for the mobile station receives. Configuration of the dedicated mode is processed by RR. This stops the access link mode.

Preamble:	ALR055

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(3)	 |	MPH_DEDICATED_REQ	  |		  |
	 *===============================>*		  |
(4)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_STOP_CCCH_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_STOP_RA_REQ	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_IMMED_ASSIGN_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_IMMED_ASSIGN_CON	  |
	 |		  *<=============================*
(9)	 |	MPH_DEDICATED_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_IMM_ASS_HOP
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	TMA_0
		dl	DL_0
		d_t	D_T_DED
		page_mode	PAGE_MODE_1
		chan_desc	CHAN_DESC_1
		pck_chan_desc	NOT_USED
		req_ref	REQ_REF_1
		time_advance	TIME_ADVANCE_1
		mob_alloc	MOB_ALLOC_1
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	STARTING_TIME
		ch_type	CH_TYPE_HOP
		ch_type2	CH_TYPE2
		arfcn	ARFCN_23
		bsic	BSIC_1
		ho_param	HO_PARAM
		tr_para	TR_PARAM
		ciph	CIPH_PARAM
		amr_conf	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_STOP_CCCH_REQ
		param	NOT_USED

- MPHC_STOP_RA_REQ
		param	NOT_USED

- MPHC_IMMED_ASSIGN_REQ
		channel_desc	CHANNEL_DESC_1
		timing_advance	TIMING_ADVANCE
		frequency_list	FREQ_LIST
		starting_time	S_TIME_MPHC
		frequency_list_bef_sti	NOT_USED
		maio_bef_sti	NOT_USED
		dtx_allowed	NOT_USED
		bcch_allocation	NOT_USED
		ba_id	NOT_USED
		pwrc	NOT_USED

- MPHC_IMMED_ASSIGN_CON
		param	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

History:	10.11.99	LE	Initial

18.Feb.02	OT	Adaptation for AMR

### ALR057: Immediate Assignment Reject for the Mobile Station

Description:	An immediate assignment reject for the mobile station receives. Sending of channel request bursts is stopped. The mobile station listen to the downlink CCCH. Thereafter an immediate assignmment message receives.

Preamble:	ALR055

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(3)	 |	MPH_RANDOM_ACCESS_REQ	  |		  |
	 *===============================>*		  |
(4)	 |		  |	MPHC_STOP_RA_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(7)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(8)	 |	MPH_DEDICATED_REQ	  |		  |
	 *===============================>*		  |
(9)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_STOP_CCCH_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_IMMED_ASSIGN_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_IMMED_ASSIGN_CON	  |
	 |		  *<=============================*
(15)	 |	MPH_DEDICATED_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_IMM_ASS_REJ
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	TMA_0
		dl	DL_0
		d_t	D_T_DED
		page_mode	PAGE_MODE_1				chan_desc	CHAN_DESC_2
		pck_chan_desc	NOT_USED
		req_ref	REQ_REF_1
		time_advance	TIME_ADVANCE_2
		mob_alloc	MOB_ALLOC_1
		}

- MPH_RANDOM_ACCESS_REQ
		send_mode	STOP_BURSTS

- MPHC_STOP_RA_REQ
		param	NOT_USED

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_IMM_ASS
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	TMA_0
		dl	DL_0
		d_t	D_T_DED
		page_mode	PAGE_MODE_1				chan_desc	CHAN_DESC_2
		pck_chan_desc	NOT_USED
		req_ref	REQ_REF_1
		time_advance	TIME_ADVANCE_2
		mob_alloc	MOB_ALLOC_1
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	STARTING_TIME
		ch_type	CH_TYPE_HOP
		ch_type2	CH_TYPE2
		arfcn	ARFCN_23
		bsic	BSIC_1
		ho_param	HO_PARAM
		tr_para	TR_PARAM
		ciph	CIPH_PARAM
		amr_conf	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_STOP_CCCH_REQ
		param	NOT_USED

- MPHC_IMMED_ASSIGN_REQ
		channel_desc	CHANNEL_DESC_1
		timing_advance	TIMING_ADVANCE
		frequency_list	FREQ_LIST
		starting_time	S_TIME_MPHC
		frequency_list_bef_sti	NOT_USED
		maio_bef_sti	NOT_USED
		dtx_allowed	NOT_USED
		bcch_allocation	NOT_USED
		ba_id	NOT_USED
		pwrc	NOT_USED

- MPHC_IMMED_ASSIGN_CON
		param	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

History:	10.11.99	LE	Initial
	26-Feb-02	OT	Adaptations for AMR integration

### ALR058: T3126 Expiry, Back to Idle Mode

Description:	After timeout of T3126 the mobile station goes back to idle mode.

Preamble:	ALR055

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RA_CON	  |
	 |		  *<=============================*
(2)	 |	MPH_RANDOM_ACCESS_CNF	  |		  |
	 *<===============================*		  |
(3)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(4)	 |		  |	MPHC_STOP_RA_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RA_CON
		fn	FN_BURST_1
		channel_request	CHANNEL_REQUEST_1

- MPH_RANDOM_ACCESS_CNF
		frame_no	T123_BURST_1

- MPH_IDLE_REQ
		mod	NOT_USED
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_RA_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
 		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

History:	10.11.99	LE	Initial
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

## EMO - Extended Measurement on SACCH

### ALR100: EMO, normal case

Description:	The successful case of the EMO procedure is performed. No interfering signals are received.

Preamble:	ALR056

RR/DL		 ALR		 PL
	|		  |		  |

- |	MPH_EMO_REQ	  |		  |
	*===============================>*		  |

- |		  |	MPHC_UPDATE_BA_LIST	  |
	|		  *=============================>*

- |		  |	MPHC_MEAS_REPORT	  |
	|		  *<=============================*

- |	MPH_EMO_MEAS_IND	  |		  |
	*<===============================*		  |

- |	MPH_NEIGHBOURCELL_REQ	  |		  |
	*===============================>*		  |

- |		  |	MPHC_UPDATE_BA_LIST	  |
	|		  *=============================>*

|		  |		  |
Parametrization

- Primitive	Parameter	Value

- MPH_EMO_REQ
	ba_id	CONST_129
	arfcn	A_ARFCN_EMO_000

- MPHC_UPDATE_BA_LIST
	num_of_chans	CONST_5
	chan_list	CHAN_LIST_100
	pwrc	PWRC_NOT_SET
	dtx_allowed	DTX_NOTALLOWED
	ba_id	CONST_129

- MPHC_MEAS_REPORT
	dtx_used	DTX_USED
	meas_valid	MEAS_ARE_VALID
	rxlev_full_acc	CONST_1
	rxlev_full_nbr_meas	CONST_1
	rxlev_sub_acc	CONST_1
	rxlev_sub_nbr_meas	CONST_1
	rxqual_full_acc_errors	CONST_1
	rxqual_full_nbr_bits	CONST_1
	rxqual_sub_acc_errors	CONST_1
	rxqual_sub_nbr_bits	CONST_1
	no_of_ncells_meas	CONST_5
	ncell_meas	NCELL_MEAS_100
	ba_id	CONST_129
	timing_advance	CONST_1
	txpwr_used	CONST_1

- MPH_EMO_MEAS_IND
	ba_id	CONST_129
	dtx	CONST_1
	meas_results	S_EMO_MEAS_RES_000

- MPH_NEIGHBOURCELL_REQ
	multi_band	MULTI_BAND_0
	arfcn	CHLIST_1_14_124_FFFF
	sync_only	NOT_USED

- MPHC_UPDATE_BA_LIST
	num_of_chans	CONST_4
	chan_list	CHAN_LIST_101
	pwrc	PWRC_NOT_SET
	dtx_allowed	DTX_NOTALLOWED
	ba_id	CONST_2

History:	07.11.02	VK	Initial

### ALR101: EMO, delayed, invalid and duplicated ba_id values

Description:	The successful case of the EMO procedure is performed.

Preamble:	ALR056

RR/DL		 ALR		 PL
	|		  |		  |

- |	MPH_EMO_REQ	  |		  |
	*===============================>*		  |

- |		  |	MPHC_UPDATE_BA_LIST	  |
	|		  *=============================>*

- |		  |	MPHC_MEAS_REPORT	  |
	|		  *<=============================*

- |		  |	MPHC_NCELL_SYNC_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_NCELL_SYNC_REQ	  |
	|		  *=============================>*

- |	MPH_MEASUREMENT_IND	  |		  |
	*<===============================*		  |

- |	MPH_EMO_REQ	  |		  |
	*===============================>*		  |

- |		  |	MPHC_UPDATE_BA_LIST	  |
	|		  *=============================>*

- |		  |	MPHC_MEAS_REPORT	  |
	|		  *<=============================*

- |		  |	MPHC_MEAS_REPORT	  |
	|		  *<=============================*

- |	MPH_EMO_MEAS_IND	  |		  |
	*<===============================*		  |

- |	MPH_NEIGHBOURCELL_REQ	  |		  |
	*===============================>*		  |

- |		  |	MPHC_UPDATE_BA_LIST	  |
	|		  *=============================>*

|		  |		  |
Parametrization

- Primitive	Parameter	Value

- MPH_EMO_REQ
	ba_id	CONST_129
	arfcn	A_ARFCN_EMO_000

- MPHC_UPDATE_BA_LIST
	num_of_chans	CONST_5
	chan_list	CHAN_LIST_100
	pwrc	PWRC_NOT_SET
	dtx_allowed	DTX_NOTALLOWED
	ba_id	CONST_129

- MPHC_MEAS_REPORT
	dtx_used	DTX_USED
	meas_valid	MEAS_ARE_VALID
	rxlev_full_acc	CONST_63
	rxlev_full_nbr_meas	CONST_1
	rxlev_sub_acc	CONST_63
	rxlev_sub_nbr_meas	CONST_1
	rxqual_full_acc_errors	CONST_1
	rxqual_full_nbr_bits	CONST_1
	rxqual_sub_acc_errors	CONST_1
	rxqual_sub_nbr_bits	CONST_1
	no_of_ncells_meas	CONST_5
	ncell_meas	NCELL_MEAS_102
	ba_id	CONST_1
	timing_advance	CONST_1
	txpwr_used	CONST_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPH_MEASUREMENT_IND
	arfcn	CONST_23
	rx_lev_full	CONST_63
	rx_lev_sub	NOT_USED
	rx_qual_full	NOT_USED
	rx_qual_sub	NOT_USED
	dtx	NOT_USED
	otd	NOT_USED
	valid	NOT_USED
	fn_offset	NOT_USED
	ncells	NOT_USED
	gprs_sync	NOT_USED

- MPH_EMO_REQ
	ba_id	CONST_130
	arfcn	A_ARFCN_EMO_000

- MPHC_UPDATE_BA_LIST
	num_of_chans	CONST_5
	chan_list	CHAN_LIST_100
	pwrc	PWRC_NOT_SET
	dtx_allowed	DTX_NOTALLOWED
	ba_id	CONST_130

- MPHC_MEAS_REPORT
	dtx_used	DTX_USED
	meas_valid	MEAS_ARE_VALID
	rxlev_full_acc	CONST_1
	rxlev_full_nbr_meas	CONST_1
	rxlev_sub_acc	CONST_1
	rxlev_sub_nbr_meas	CONST_1
	rxqual_full_acc_errors	CONST_1
	rxqual_full_nbr_bits	CONST_1
	rxqual_sub_acc_errors	CONST_1
	rxqual_sub_nbr_bits	CONST_1
	no_of_ncells_meas	CONST_5
	ncell_meas	NCELL_MEAS_100
	ba_id	CONST_129
	timing_advance	CONST_1
	txpwr_used	CONST_1

- MPHC_MEAS_REPORT
	dtx_used	DTX_USED
	meas_valid	MEAS_ARE_VALID
	rxlev_full_acc	CONST_1
	rxlev_full_nbr_meas	CONST_1
	rxlev_sub_acc	CONST_1
	rxlev_sub_nbr_meas	CONST_1
	rxqual_full_acc_errors	CONST_1
	rxqual_full_nbr_bits	CONST_1
	rxqual_sub_acc_errors	CONST_1
	rxqual_sub_nbr_bits	CONST_1
	no_of_ncells_meas	CONST_5
	ncell_meas	NCELL_MEAS_100
	ba_id	CONST_130
	timing_advance	CONST_1
	txpwr_used	CONST_1

- MPH_EMO_MEAS_IND
	ba_id	CONST_130
	dtx	CONST_1
	meas_results	S_EMO_MEAS_RES_000

- MPH_NEIGHBOURCELL_REQ
	multi_band	MULTI_BAND_0
	arfcn	CHLIST_1_14_124_FFFF
	sync_only	NOT_USED

- MPHC_UPDATE_BA_LIST
	num_of_chans	CONST_4
	chan_list	CHAN_LIST_101
	pwrc	PWRC_NOT_SET
	dtx_allowed	DTX_NOTALLOWED
	ba_id	CONST_2

History:	07.11.02	VK	Initial

## Handover & Assignment

### ALR150: Non-synchronized Handover

Description:	A non-synchronized handover for the mobile station is configured without starting time.

Preamble:	ALR056

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_DEDICATED_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_ASYNC_HO_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_DEDICATED_REQ
		mod	MODE_ASYNC_HANDOVER
		start	NOT_USED
		ch_type	CH_TYPE_TCH2
		ch_type2	NOT_USED
		arfcn	ARFCN_14
		bsic	BSIC_1
		ho_param	HO_PARAM_1
		tr_para	TR_PARAM
		ciph	CIPH_PARAM
		amr_conf	NOT_USED

- MPHC_ASYNC_HO_REQ	handover_command	ASYNC_HO_CMD
		fn_offset	NOT_USED
		time_alignmnt	NOT_USED
		cipher_key	NOT_USED
		amr_configuration	NOT_USED

History:	10.11.99	LE	Initial
	26-Feb-02	OT	Adaptations for AMR integration

### ALR151: FTA 26.6.13.3

Description:	The type approval testcase 26.6.13.3 is performed.

Preamble:	ALR012A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_RANDOM_ACCESS_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_STOP_RXLEV_PERIODIC_REQ	  |
	 |		  *================================>*
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *================================>*
(4)	 |		  |	MPHC_RA_REQ	  |
	 |		  *================================>*
(5)	 |		  |	MPHC_RA_CON	  |
	 |		  *<================================*
(6)	 |	MPH_RANDOM_ACCESS_CNF	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RA_REQ	  |
	 |		  *================================>*
(8)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<================================*
(9)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(10)	 |	MPH_DEDICATED_REQ	  |		  |
	 *===============================>*		  |
(11)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *================================>*
(12)	 |		  |	MPHC_STOP_CCCH_REQ	  |
	 |		  *================================>*
(13)	 |		  |	MPHC_STOP_RA_REQ	  |
	 |		  *================================>*
(14)	 |		  |	MPHC_IMMED_ASSIGN_REQ	  |
	 |		  *================================>*
(15)	 |		  |	MPHC_IMMED_ASSIGN_CON	  |
	 |		  *<================================*
(16)	 |	MPH_DEDICATED_CNF	  |		  |
	 *<===============================*		  |
(17)	 |	MPH_FREQ_REDEF_REQ	  |		  |
	 *===============================>*		  |
(18)	 |		  |	MPHC_CHANGE_FREQUENCY	  |
	 |		  *================================>*
(19)	 |	MPH_DEDICATED_REQ	  |		  |
	 *===============================>*		  |
(20)	 |		  |	MPHC_CHANNEL_ASSIGN_REQ	  |
	 |		  *================================>*
(21)	 |		  |	MPHC_CHANNEL_ASSIGN_CON	  |
	 |		  *<================================*
(22)	 |	MPH_DEDICATED_CNF	  |		  |
	 *<===============================*		  |
(23)	 |	MPH_DEDICATED_FAIL_REQ	  |		  |
	 *===============================>*		  |
(24)	 |		  |	MPHC_CHANNEL_ASSIGN_REQ	  |
	 |		  *================================>*
(25)	 |		  |	MPHC_CHANNEL_ASSIGN_CON	  |
	 |		  *<================================*
(26)	 |	MPH_DEDICATED_FAIL_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_REQ
		send_mode	TWO_BURSTS

- MPHC_STOP_RXLEV_PERIODIC_REQ
 		param	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_RA_REQ
		txpwr	POWER_12
		rand	RAND_BURST_1
		channel_request	CHANNEL_REQUEST_1
		powerclass_gsm	POWER_CLASS_5
		powerclass_dcs	NOT_USED

- MPHC_RA_CON
		fn	FN_BURST_1
		channel_request	CHANNEL_REQUEST_1

- MPH_RANDOM_ACCESS_CNF
		frame_no	T123_BURST_1

- MPHC_RA_REQ
		txpwr	POWER_12
		rand	RAND_BURST_2
		channel_request	CHANNEL_REQUEST_2
		powerclass_gsm	POWER_CLASS_5
		powerclass_dcs	NOT_USED

- MPHC_DATA_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_CCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_IMM_ASS_HOP
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	TMA_0
		dl	DL_0
		d_t	D_T_DED
		page_mode	PAGE_MODE_1
		chan_desc	CHAN_DESC_1
		pck_chan_desc	NOT_USED
		req_ref	REQ_REF_1
		time_advance	TIME_ADVANCE_1
		mob_alloc	MOB_ALLOC_1
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	NO_STARTING_TIME
		ch_type	CH_TYPE_IMM_ASS
		ch_type2	NOT_USED
		arfcn	ARFCN_14
		bsic	BSIC_1
		ho_param	HO_PARAM
		tr_para	TR_PARAM
		ciph	CIPH_PARAM
		amr_conf	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_STOP_CCCH_REQ
		param	NOT_USED

- MPHC_STOP_RA_REQ
		param	NOT_USED

- MPHC_IMMED_ASSIGN_REQ
		channel_desc	CHANNEL_DESC_IA
		timing_advance	TIMING_ADVANCE
		frequency_list	FREQ_LIST_IA
		starting_time	NO_STARTING_TIME
		frequency_list_bef_sti	NOT_USED
		maio_bef_sti	NOT_USED
		dtx_allowed	NOT_USED
		bcch_allocation	NOT_USED
		ba_id	NOT_USED
		pwrc	NOT_USED

- MPHC_IMMED_ASSIGN_CON
		param	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

- MPH_FREQ_REDEF_REQ
		start	STARTING_TIME_T1
		ch_type	CH_TYPE_FREQ_REDEF

- MPHC_CHANGE_FREQUENCY
		channel_desc	CHANNEL_DESC_FR
		frequency_list	FREQ_LIST_FR
		starting_time	S_TIME_T1

- MPH_DEDICATED_REQ
		mod	MODE_CHAN_ASSIGN
		start	STARTING_TIME_T2
		ch_type	CH_TYPE_ASS_AFTER
		ch_type2	CH_TYPE_ASS_BEFORE
		arfcn	ARFCN_14
		bsic	BSIC_1
		ho_param	NOT_USED
		tr_para	TR_PARAM
		ciph	CIPH_PARAM
		amr_conf	NOT_USED

- MPHC_CHANNEL_ASSIGN_REQ
		channel_desc_1	CHANNEL_DESC_ASS_AFTER
		channel_mode_1	NOT_USED
		txpwr	TXPWR0
		frequency_list	FREQ_LIST_ASS_AFTER
		starting_time	S_TIME_T2
		channel_desc_2	NOT_USED
		channel_mode_2	NOT_USED
		frequency_list_bef_sti	FREQ_LIST_ASS_BEFORE
		channel_desc_1_bef_sti	CHANNEL_DESC_ASS_BEFORE
		channel_desc_2_bef_sti	NOT_USED
		cipher_mode	NOT_USED
		a5_algorithm	NOT_USED
		cipher_key	NOT_USED
		dtx_allowed	NOT_USED
		amr_configuration	NOT_USED

- MPHC_CHANNEL_ASSIGN_CON
		param	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

- MPH_DEDICATED_FAIL_REQ
		param	NOT_USED

- MPHC_CHANNEL_ASSIGN_REQ
		channel_desc_1	CHANNEL_DESC_FR
		channel_mode_1	NOT_USED
		txpwr	POWER_12
		frequency_list	FREQ_LIST_FR
		starting_time	S_TIME_T1
		channel_desc_2	NOT_USED
		channel_mode_2	NOT_USED
		frequency_list_bef_sti	FREQ_LIST_IA
		channel_desc_1_bef_sti	CHANNEL_DESC_IA
		channel_desc_2_bef_sti	NOT_USED
		cipher_mode	NOT_USED
		a5_algorithm	NOT_USED
		cipher_key	NOT_USED
		dtx_allowed	NOT_USED
		amr_configuration	NOT_USED

- MPHC_CHANNEL_ASSIGN_CON
		param	NOT_USED

- MPH_DEDICATED_FAIL_CNF
		param	NOT_USED

History:	10.11.99	LE	Initial

07.02.02	LG	removed MPHC_START_CCCH_REQ (ALR-FIX-4650)
	26-Feb-02	OT	Adaptations for AMR integration

29.10.02	DL	MPHC_STOP_NCELL_SYNC/BCCH_REQ removed

### ALR152: FTA 26.6.13.8

Description:	The type approval testcase 26.6.13.8 is performed.

Preamble:	ALR056

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_FREQ_REDEF_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_CHANGE_FREQUENCY	  |
	 |		  *=============================>*
(3)	 |	MPH_DEDICATED_REQ	  |		  |
	 *===============================>*		  |
(4)	 |		  |	MPHC_ASYNC_HO_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_ASYNC_HO_CON	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_HANDOVER_FINISHED	  |
	 |		  *<=============================*
(7)	 |	MPH_DEDICATED_CNF	  |		  |
	 *<===============================*		  |
(8)	 |	MPH_DEDICATED_FAIL_REQ	  |		  |
	 *===============================>*		  |
(9)	 |		  |	MPHC_HANDOVER_FAIL_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_HANDOVER_FAIL_CON	  |
	 |		  *<=============================*
(11)	 |	MPH_DEDICATED_FAIL_CNF	  |		  |
	 *<===============================*		  |
(12)	 |		  |	MPHC_CHANGE_FREQUENCY_CON	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_FREQ_REDEF_REQ
		start	STARTING_TIME
		ch_type	CH_TYPE_SDCCH4

- MPHC_CHANGE_FREQUENCY
		channel_desc	CHANNEL_DESC_4
		frequency_list	FREQ_LIST
		starting_time	S_TIME_MPHC

- MPH_DEDICATED_REQ
		mod	MODE_ASYNC_HANDOVER
		start	STARTING_TIME
		ch_type	CH_TYPE_TCH2
		ch_type2	CH_TYPE_TCH3
		arfcn	ARFCN_14
		bsic	BSIC_1
		ho_param	HO_PARAM_1
		tr_para	TR_PARAM
		ciph	CIPH_PARAM
		amr_conf	NOT_USED

- MPHC_ASYNC_HO_REQ
		handover_command	ASYNC_HO_CMD_2
		fn_offset	NOT_USED
		time_alignmnt	NOT_USED
		cipher_key	NOT_USED
		amr_configuration	NOT_USED

- MPHC_ASYNC_HO_CON
		param	NOT_USED

- MPHC_HANDOVER_FINISHED
		cause	HO_COMPLETE

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

- MPH_DEDICATED_FAIL_REQ
		param	NOT_USED

- MPHC_HANDOVER_FAIL_REQ
		param	NOT_USED

- MPHC_HANDOVER_FAIL_CON
		param	NOT_USED

- MPH_DEDICATED_FAIL_CNF
		param	NOT_USED

- MPHC_CHANGE_FREQUENCY_CON
		param	NOT_USED

History:	10.11.99	LE	Initial
	26-Feb-02	OT	Adaptations for AMR integration

### ALR153: SACCH Downlink Messages

Description:	The MS is in dedicated mode. It receives a system info type 5 message (forwarded to RR), a system info type 5bis message (forwarded to RR), a system info type 6 message (forwarded to RR), an I-frame for SMS (forwarded to DL), the first system info type 5 message again (compared and not forwarded by ALR) and a changed system info type 6 message (forwarded to RR).

Preamble:	ALR151

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(SYS INFO 5)	  |
	 |		  *<=============================*
(2)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(3)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(SYS INFO 5bis)	  |
	 |		  *<=============================*
(4)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(5)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(SYS INFO 6)	  |
	 |		  *<=============================*
(6)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(I frame)	  |
	 |		  *<=============================*
(8)	 |	PH_DATA_IND	  |		  |
	 *<===============================*		  |
(9)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(SYS INFO 5)	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(changed SYS INFO 6)	  |
	 |		  *<=============================*
(11)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_SACCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_5
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5
		ti	TI_0
		neigh_cell_desc	NEIGH_CELL_DESC_2
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_SACCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_5BIS
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_5BIS
		ti	TI_0
		neigh_cell_desc	NEIGH_CELL_DESC_2
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_SACCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_6
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_6
		ti	TI_0
		cell_ident	CELL_IDENT_2
		loc_area_ident	LOC_AREA_IDENT_2
		cell_opt_sacch	CELL_OPT_SACCH_1
		ncc_permit	NCC_PERMIT_2
		si6_rest_oct	NOT_USED		
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_SACCH
		error_flag	VALID_BLOCK
		l2_frame	L2_I_SMS
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- PH_DATA_IND
		ch_type	CH_TYPE_SACCH
		dummy	NOT_USED
		sdu	I_SMS

- MPHC_DATA_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_SACCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_5
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_SACCH
		error_flag	VALID_BLOCK
		l2_frame	L2_CHANGED_SYS_INFO_6
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_6
		ti	TI_0
		cell_ident	CELL_IDENT_3
		loc_area_ident	LOC_AREA_IDENT_2
		cell_opt_sacch	CELL_OPT_SACCH_1
		ncc_permit	NCC_PERMIT_2
		si6_rest_oct	NOT_USED		
		}

History:	10.11.99	LE	Initial

### ALR154: Handover, Serving Cell Parameter

Description:	A handover is performed. It is checked that the serving cell channel numbers are correctly set.

<A>:	AMR not signalled
<B>:	AMR signalled

Variants:	<A>....<B>

Preamble:	ALR056

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_DEDICATED_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_ASYNC_HO_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_ASYNC_HO_CON	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_HANDOVER_FINISHED	  |
	 |		  *<=============================*
(5)	 |	MPH_DEDICATED_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_DEDICATED_REQ
		mod	MODE_ASYNC_HANDOVER
		start	STARTING_TIME
		ch_type	CH_TYPE_TCH2
		ch_type2	CH_TYPE_TCH3
		arfcn	ARFCN_30
		bsic	BSIC_1
		ho_param	HO_PARAM_1
		tr_para	TR_PARAM
		ciph	CIPH_PARAM
<A>	amr_conf	NOT_USED
<B>	amr_conf	S_AMR_conf_4_ICMI

- MPHC_ASYNC_HO_REQ
		handover_command	ASYNC_HO_CMD_3
		fn_offset	NOT_USED
		time_alignmnt	NOT_USED
		cipher_key	NOT_USED
<A>	amr_configuration	NOT_USED
<B>	amr_configuration	S_amr_configuration

- MPHC_ASYNC_HO_CON
		param	NOT_USED

- MPHC_HANDOVER_FINISHED
		cause	HO_COMPLETE

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

History:	10.11.99	LE	Initial

30-Nov-01	OT	Variants <A>..<B> introduced, <B> for AMR

## Channel mode modify

### ALR701: Channel mode modify request – AMR half rate

Description:	A handover is performed. It is checked that the serving cell channel numbers are correctly set.

Preamble:	ALR056

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CHANNEL_MODE_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_CHANNEL_MODE_MODIFY_REQ	  |
	 |		  *================================>*
(3)	 |		  |	MPHC_CHANNEL_MODE_MODIFY_CON	  |
	 |		  *<================================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CHANNEL_MODE_REQ
		mode	CHANNEL_MODE_AMR
		ch	CHANNEL_0
		amr_conf	S_AMR_conf_4_ICMI

- MPHC_CHANNEL_MODE_MODIFY_REQ
		sub_channel	CHANNEL_0
		channel_mode	CHM_AHS
		amr_configuration	S_amr_configuration

- MPHC_CHANNEL_MODE_MODIFY_CON
		param	NOT_USED

History:	30-Nov-01	OT	Initial

## Downlink Failure Detection

### ALR042: Receive Invalid Paging Messages

Description:	The downlink timeout value is initialised with a value of 10 (bs_pa_mfrms equal to 9). For each invalid paging message the counter is decremented by four. After three invalid messages a downlink error shall be indicated to RR.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(4)	 |	MPH_ERROR_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_ERROR_IND
		cs	CS_DOWN_LINK_FAIL
		arfcn	ARFCN_23

History:	10.11.99	LE	Initial

### ALR043: Test Upper Limit of Downlink Timeout Value

Description:	The downlink timeout value is initialised with a value of 10 (bs_pa_mfrms equal to 9). For each invalid paging message the counter is decremented by four. For each valid paging message the counter is incremented by one. It is checked that the counter is not incremented more than the initial value.

Preamble:	ALR013

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(9)	 |	MPH_ERROR_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
 		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
 		page_block_index	PBI_0
 		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_ERROR_IND
		cs	CS_DOWN_LINK_FAIL
		arfcn	ARFCN_23

History:	10.11.99	LE	Initial

## Cell Reselection

### ALR900: Successful Case

Description:	The current serving cell is 23, the neighbourcells are 1,14 and 124. RR starts a cell reselection to neighbourcell 14. After the end of cell reselection a new neighbourcell list is forwarded to ALR with the neighbourcells 2 and 15. It is expected that layer 1 is configured with the new serving cell 14 and the neighbourcells 2 and 15.

Preamble:	ALR047

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |  MPHC_STOP_RXLEV_PERIODIC_REQ |
	 |		  *=============================>*
(3)	 |		  |  MPHC_STOP_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(4)	 |		  |  MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(9)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(11)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(12)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(15)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(16)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(17)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(18)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(19)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(20)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(19)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(20)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(22)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
(23)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(24)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_14
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_RXLEV_PERIODIC_REQ
		param	NOT_USED

- MPHC_STOP_NCELL_SYNC_REQ
		radio_freq_array_size	STOP_SIZE_0
		radio_freq_array	NOT_USED

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	STOP_SIZE_0
		radio_freq_array	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_NEW_SCELL_REQ
 		radio_freq	ARFCN_14
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		tsc	BSIC_1

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_2
		bs_ag_blks_res	BS_AG_BLKS_RES_7
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_0
		page_block_index	PBI_0
 		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	NOT_USED

- MPHC_DATA_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_14

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_4
		bs_ag_blks_res	BS_AG_BLKS_RES_5
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_1
		page_block_index	PBI_1
 		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPHC_DATA_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_14

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_4
		bs_ag_blks_res	BS_AG_BLKS_RES_5
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_1
		page_block_index	PBI_1
 		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

- MPHC_DATA_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_14

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NEIGH_CELL_DESC_1
		ncc_permit	NCC_PERMIT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_14
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_1
		bs_ag_blocks_res	BS_AG_BLKS_RES_5
		bs_pa_mfrms	BS_PA_MFRMS_2
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_23_1_14_124

- num_of_chans	CHANNELS_4
		ba_id	BA_ID_2
 		next_radio_freq_measured	CHAN_LIST_IDX_0

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	CHLIST_1_15_FFFF

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_1_14_15
		num_of_chans	CHANNELS_3
		ba_id	BA_ID_3
 		next_radio_freq_measured	CHAN_LIST_IDX_0

History:	10.11.99	LE	Initial

08.02.02	LG	handling of SI3 inserted (ALR-FIX-4650)

28.06.02	MSB	faster switch to normal paging leads to normal 
			reading of SC BCCH with 
			MPHC_SCELL_NBCCH_REQ (sched_size=8)
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR901: Cell Reselection after dedicated mode

Description:	The mobile leaves the dedicated mode to the current serving cell 23. A cell reselection is performed and the idle mode is configured.

Preamble:	ALR056

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(2)	 |		  |	MPHC_STOP_DEDICATED	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(6)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(8)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(9)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(10)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(11)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(12)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
(16)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(17)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_DEDICATED
		param	NOT_USED

- MPHC_NEW_SCELL_REQ
 		radio_freq	ARFCN_23
 		fn_offset	FN_OFFSET_0
 		time_alignment	TIME_ALIGNMT_0
 		tsc	BSIC_1

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_2
		bs_ag_blks_res	BS_AG_BLKS_RES_7
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_0
		page_block_index	PBI_0
 		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	NOT_USED

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	FN_OFFSET_0
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	FN_OFFSET_0
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NEIGH_CELL_DESC_1
		ncc_permit	NCC_PERMIT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
 		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	FULL_READ

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_23_1_14_124

- num_of_chans	CHANNELS_4
		ba_id	BA_ID_2
 		next_radio_freq_measured	CHAN_LIST_IDX_0

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	CHLIST_1_15_FFFF

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_23_1_15
		num_of_chans	CHANNELS_3
		ba_id	BA_ID_3
 		next_radio_freq_measured	CHAN_LIST_IDX_0

History:	10.11.99	LE	Initial

21.06.02	MSB	sequence and parameter changed
			for MPHC_SCELL_NBCCH_REQ:
			after page mode set to REORG, 
			full read is necessary
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR920: PDCH Assignment

Description:	In dedicated mode ALR is suspended during a PDCH Assignment procedure. 
Reference: TS04.08 V8.10.0, section 3.4.19 Assignment to a Packet Data channel.

Preamble:	ALR056

RR/DL		 ALR		 PL
	|		  |		  |

- |	MPH_DEDICATED_REQ	  |		  |
	 *===============================>*		  |

- |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_STOP_CCCH_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_STOP_DEDICATED	  |
	|		  *=============================>*

- |	MPH_DEDICATED_CNF	  |		  |
	*<================================*		  |

|		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_DEDICATED_REQ
		mod	MODE_PDCH_ASSIGN
		start	NOT_USED
		ch_type	NOT_USED
		ch_type2	NOT_USED
		arfcn	NOT_USED
		bsic	NOT_USED
		ho_param	NOT_USED
		tr_para	NOT_USED
		ciph	NOT_USED
		amr_conf	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_STOP_CCCH_REQ
		param	NOT_USED

- MPHC_STOP_DEDICATED
		param	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

History:	12.08.01	VK	Initial (copied from ALR930 and adapted)
		26-Feb-02	OT	Adaptations for AMR integration

### ALR921: PDCH Assignment, Success, Start TBF establishment

Description:	In dedicated mode ALR is suspended during a PDCH Assignment procedure. After a successful TBF establishment, ALR is requested to enter a mode which is compatible to Packet Transfer Mode.

Preamble:	ALR920

RR/DL		 ALR		 PL
	|		  |		  |

- |	MPH_IDLE_REQ	  |		  |
	*================================>*		  |

- |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_STOP_CCCH_REQ	  |
	|		  *=============================>*

|		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_IDLE_REQ
		mod	MODE_PACKET_TRANSFER
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_STOP_CCCH_REQ
		param	NOT_USED

History:	13.08.01	VK	Initial
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

### ALR922: PDCH Assignment, Error during TBB establishment

Description:	After an unsuccessful TBF establishment, ALR is requested to switch back to the 'old' channel.

Preamble:	ALR921

RR/DL		 ALR		 PL
	|		  |		  |

- |	MPH_DEDICATED_FAIL_REQ	  |		  |
	*================================>*		  |

- |		  |	MPHC_IMMED_ASSIGN_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_IMMED_ASSIGN_CON	  |
	|		  *<=============================*

- |		  |	MPHC_CHANNEL_ASSIGN_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_CHANNEL_ASSIGN_CON	  |
	|		  *<=============================*

- |	MPH_DEDICATED_FAIL_CNF	  |		  |
	*<================================*		  |

|		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_DEDICATED_FAIL_REQ
		param	NOT_USED

- MPHC_IMMED_ASSIGN_REQ
		channel_desc	CHANNEL_DESC_1
		timing_advance	TIMING_ADVANCE
		frequency_list	FREQ_LIST
		starting_time	NOT_USED
		frequency_list_bef_sti	NOT_USED
		maio_bef_sti	NOT_USED
		dtx_allowed	NOT_USED
		bcch_allocation	NOT_USED
		ba_id	NOT_USED
		pwrc	NOT_USED

- MPHC_IMMED_ASSIGN_CON
		param	NOT_USED

- MPHC_CHANNEL_ASSIGN_REQ
		channel_desc_1	NOT_USED
		channel_mode_1	NOT_USED
		txpwr	NOT_USED
		frequency_list	NOT_USED
		starting_time	NOT_USED
		channel_desc_2	NOT_USED
		channel_mode_2	NOT_USED
		frequency_list_bef_sti	NOT_USED
		channel_desc_1_bef_sti	NOT_USED
		channel_desc_2_bef_sti	NOT_USED
		cipher_mode	NOT_USED
		a5_algorithm	NOT_USED
		cipher_key	NOT_USED
		dtx_allowed	NOT_USED
		amr_configuration	NOT_USED

- MPHC_CHANNEL_ASSIGN_CON
		param	NOT_USED

- MPH_DEDICATED_FAIL_CNF
		param	NOT_USED

History:	14.08.01	VK	Initial4650)
	26-Feb-02	OT	Adaptations for AMR integration

### ALR925: Cell Selection after dedicated mode

Description:	The mobile leaves the dedicated mode after some moments to the current serving cell 23. A cell selection is performed and the idle mode is configured.

Preamble:	ALR020B

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(3) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_RANDOM_ACCESS_REQ	  |		  |
	 *===============================>*		  |
(7)	 |		  |	MPHC_STOP_RXLEV_PERIODIC_REQ	  |
	 |		  *===============================>*
(8)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *===============================>*
(9)	 |		  |	MPHC_RA_REQ	  |
	 |		  *===============================>*
(10)	 |		  |	MPHC_RA_CON	  |
	 |		  *<===============================*
(11)	 |	MPH_RANDOM_ACCESS_CNF	  |		  |
	 *<===============================*		  |
(12)	 |		  |	MPHC_RA_REQ	  |
	 |		  *===============================>*
(13)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(14)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(15)	 |	MPH_DEDICATED_REQ	  |		  |
	 *===============================>*		  |
(16)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(17)	 |		  |	MPHC_STOP_CCCH_REQ	  |
	 |		  *=============================>*
(18)	 |		  |	MPHC_STOP_RA_REQ	  |
	 |		  *=============================>*
(19)	 |		  |	MPHC_IMMED_ASSIGN_REQ	  |
	 |		  *=============================>*
(20)	 |		  |	MPHC_IMMED_ASSIGN_CON	  |
	 |		  *<=============================*
(21)	 |	MPH_DEDICATED_CNF	  |		  |
	 *<===============================*		  |
(22)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(23)	 |		  |	MPHC_STOP_DEDICATED	  |
	 |		  *=============================>*
(24)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(25)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(26)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_7
		bs_ag_blks_res	BS_AG_BLKS_RES_2
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_2
		page_group	PG_23
		page_block_index	PBI_0
		page_mode	PGM_NORMAL

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	PERIODIC_SCELL_BCCH_ARRAY_SIZE
 		schedule_array	PERIODIC_SCELL_BCCH_ARRAY

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_PCH
		error_flag	VALID_BLOCK
		l2_frame	L2_PAGING_REQ_1_EXT
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_7
		bs_ag_blks_res	BS_AG_BLKS_RES_2
		bcch_combined	COMB_CCCH_COMB
		ccch_group	CCCH_GROUP_2
		page_group	PG_23
		page_block_index	PBI_0
		page_mode	PGM_EXTENDED

- MPH_RANDOM_ACCESS_REQ
		send_mode	TWO_BURSTS

- MPHC_STOP_RXLEV_PERIODIC_REQ
 		param	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_RA_REQ
		txpwr	POWER_12
		rand	RAND_BURST_1
		channel_request	CHANNEL_REQUEST_1
		powerclass_gsm	NOT_USED
		powerclass_dcs	NOT_USED

- MPHC_RA_CON
		fn	FN_BURST_1
		channel_request	CHANNEL_REQUEST_1

- MPH_RANDOM_ACCESS_CNF
		frame_no	T123_BURST_1

- MPHC_RA_REQ
		txpwr	POWER_12
		rand	RAND_BURST_2
		channel_request	CHANNEL_REQUEST_2
		powerclass_gsm	NOT_USED
		powerclass_dcs	NOT_USED

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_IMM_ASS_HOP
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	TMA_0
		dl	DL_0
		d_t	D_T_DED
		page_mode	PAGE_MODE_1
		chan_desc	CHAN_DESC_1
		pck_chan_desc	NOT_USED
		req_ref	REQ_REF_1
		time_advance	TIME_ADVANCE_1
		mob_alloc	MOB_ALLOC_1
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	STARTING_TIME
		ch_type	CH_TYPE_HOP
		ch_type2	CH_TYPE2
		arfcn	ARFCN_23
		bsic	BSIC_1
		ho_param	HO_PARAM
		tr_para	TR_PARAM
		ciph	CIPH_PARAM
		amr_conf	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_STOP_CCCH_REQ
		param	NOT_USED

- MPHC_STOP_RA_REQ
		param	NOT_USED

- MPHC_IMMED_ASSIGN_REQ
		channel_desc	CHANNEL_DESC_1
		timing_advance	TIMING_ADVANCE
		frequency_list	FREQ_LIST
		starting_time	S_TIME_MPHC
		frequency_list_bef_sti	NOT_USED
		maio_bef_sti	NOT_USED
		dtx_allowed	NOT_USED
		bcch_allocation	NOT_USED
		ba_id	NOT_USED
		pwrc	NOT_USED

- MPHC_IMMED_ASSIGN_CON
		param	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_DEDICATED
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_2
		bs_ag_blks_res	BS_AG_BLKS_RES_7
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_0
		page_block_index	PBI_0
 		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	NOT_USED
		num_of_chans	NOT_USED
		ba_id	NOT_USED
		next_radio_freq_measured	NOT_USED

History:  	12.02.03	ZMM	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR930: Network Coltrolled Change Cell

Description:	In dedicated mode ALR is requested to synchronize to a new cell. The new cell ARFCN and BSIC is provided by the network.

Preamble:	ALR056

RR/DL		 ALR		 PL
	|		  |		  |

- |	MPH_BSIC_REQ	  |		  |
	*===============================>*		  |

- |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_STOP_CCCH_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_STOP_RXLEV_PERIODIC_REQ	|
	|		  *=============================>*

- |		  |	MPHC_STOP_DEDICATED	  |
	|		  *=============================>*

- |		  |	MPHC_NETWORK_SYNC_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_NETWORK_SYNC_IND	  |
	|		  *<=============================*

- |	MPH_BSIC_CNF	  |		  |
	*<===============================*		  |

- |		  |	MPHC_NEW_SCELL_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_NEW_SCELL_CON	  |
	|		  *<=============================*

- |		  |	MPHC_SCELL_NBCCH_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_DATA_IND	  |
	|		  *<=============================*

- |	MPH_UNITDATA_IND	  |		  |
	*<===============================*		  |

- |		  |	MPHC_DATA_IND	  |
	|		  *<=============================*

- |	MPH_UNITDATA_IND	  |		  |
	*<===============================*		  |

- |	MPH_IDLE_REQ	  |		  |
	*===============================>*		  |

- |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_START_CCCH_REQ	  |
	|		  *=============================>*

- |		  |	MPHC_SCELL_NBCCH_REQ	  |
	|		  *=============================>*

- |	MPH_NEIGHBOURCELL_REQ	  |		  |
	*===============================>*		  |

- |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	|		  *=============================>*

|		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_BSIC_REQ
		arfcn	ARFCN_42

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_STOP_CCCH_REQ
		param	NOT_USED

- MPHC_STOP_RXLEV_PERIODIC_REQ
		param	NOT_USED

- MPHC_STOP_DEDICATED
		param	NOT_USED

- MPHC_NETWORK_SYNC_REQ
		radio_freq	ARFCN_42
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO
 		search_mode	SM_WIDE_MODE

- MPHC_NETWORK_SYNC_IND
		radio_freq	ARFCN_42
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGNMT_0
		bsic	BSIC_1

- MPH_BSIC_CNF
		arfcn	ARFCN_42_STD_900
		bsic	BSIC_1
		cs	CS_NO_ERROR

- MPHC_NEW_SCELL_REQ
 		radio_freq	ARFCN_42
 		fn_offset	FN_OFFSET_0
 		time_alignment	TIME_ALIGNMT_0
 		tsc	BSIC_1

- MPHC_NEW_SCELL_CON
 		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	FULL_READ

- MPHC_DATA_IND
		radio_freq	ARFCN_42
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_1
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_42
		fn	FN_OFFSET_0
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_1
		ti	TI_0
		cell_chan_desc	CELL_CHAN_DESC_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_DATA_IND
		radio_freq	ARFCN_42
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_1
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_42
		fn	FN_OFFSET_0
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_2
		ti	TI_0
		neigh_cell_desc	NEIGH_CELL_DESC_1
		ncc_permit	NCC_PERMIT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_42
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
 		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	FULL_READ

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	CHLIST_2_30_FFFF
		sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_42_2_30
		num_of_chans	CHANNELS_3
		ba_id	BA_ID_2
 		next_radio_freq_measured	CHAN_LIST_IDX_0

History:	08.08.01	VK	Initial (copied from ALR901)

28.06.02	MSB	change ba_id in the last message to 2
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

## Idle Mode Neighbourcells Procedures

### ALR046: Definition of BCCH Allocation

Description:	RR selects the channel 23 after reading the BCCH carrier.
Variant A: with neighbourcell list (serving cell not included)
Variant B: with neighbourcell list (serving cell included)
Variant C: with empty neighbour cell list.
Variant D: same like A, but with ncc_permitted = 4

Preamble:	ALR003

Variants:	<A> .. <D>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_IDENTITY_REQ	  |		  |
	 *===============================>*		  |
(7)	 |	MPH_CBCH_REQ	  |		  |
	 *===============================>*		  |
(8)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(9)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_GSM_900

- MPH_IDLE_REQ
		mod	NOT_USED
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_4
		power	POWER_12
<A>	ncc_permitted	NOT_PRESENT_8BIT
<B>	ncc_permitted	NOT_PRESENT_8BIT
<C>	ncc_permitted	NOT_PRESENT_8BIT
<D>	ncc_permitted	NCC_PERMITTED_4
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_6
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
 		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_IDENTITY_REQ
		mid	MS_ID_IMSI_TMSI

- MPH_CBCH_REQ
		cbch	NO_CBCH

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
<A>	arfcn	CHLIST_1_14_124_FFFF

- <B>	arfcn	CHLIST_23_1_14_124_FFFF

- <C>	arfcn	EMPTY_NCELL_LIST
<D>	arfcn	CHLIST_1_14_124_FFFF

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
<A>	chan_list	CHLIST_23_1_14_124

- <B>	chan_list	CHLIST_23_1_14_124

- <C>	chan_list	CHLIST_23
<D>	chan_list	CHLIST_23_1_14_124

- <A>	num_of_chans	CHANNELS_4
<B>	num_of_chans	CHANNELS_4
<C>	num_of_chans	CHANNELS_1
<D>	num_of_chans	CHANNELS_4
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

History:	10.11.99	LE	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	07.02.02	LG	changed value for ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included
	07.04.03	MSB	Use preamble ALR003 for arfcn=23 as serving cell

### ALR047: Synchronisation to Neighbour Cells successful

Description:	The BA list contains the serving cell 23 and the neighbour cells 1, 14 and 124. The fieldstrength is 56 for channel 23, 12 for channel 1, 44 for channel 14 and 25 for channel 124 (all values in GSM range). The ranking for the neighbour cells is 14, 124 and channel 1. Each reports contains two fieldstrength values per channel. The multiframe period is set to 6. The first measurement report is send to RR after five reports from PL. Then after each three reports from PL a measurement report is send to RR.

Preamble:	ALR046A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(13)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(16)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(17)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(19)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(20)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(22)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(23)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(24)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(25)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(26)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(27)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(28)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(29)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(30)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(32)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(33)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(34)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(35)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(36)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(37)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(38)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(39)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(40)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(41)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(42)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(43)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(44)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(45)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(46)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(47)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(48)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(49)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(50)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(51)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(52)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(53)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(54)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(55)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(56)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(57)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(58)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(59)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(60)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(61)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(62)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(63)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_14

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_124

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		timing_validity	TV_VALID_TIMING_INFO

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		timing_validity	TV_VALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		timing_validity	TV_VALID_TIMING_INFO

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

History:	24.09.99	MPA	Initial

13.06.01	MSB	fn_offset in (6) corrected

20.06.01	MSB	numbering of msc corrected

07.02.02	LG	changed value for ba_id
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR048: Synchronisation to Neighbour Cells failed

Description:	The BA list contains the serving cell 23 and the neighbour cells 1, 14 and 124. The fieldstrength is 56 for channel 23, 12 for channel 1, 44 for channel 14 and 25 for channel 124 (all values in GSM range). The ranking for the neighbour cells is 14, 124 and at least channel 1. Each reports contains two fieldstrength values per channel. The multiframe period is set to 6. The first measurement report is send to RR after five reports from PL. Then after each three reports from PL a measurement report is send to RR. Synchronisation to channel 14 fails.

Preamble:	ALR046A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(13)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(15)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(16)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(17)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(18)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(19)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(20)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(21)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(22)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(23)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(24)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(25)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(26)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(27)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(28)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(29)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(30)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(31)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(32)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(33)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(34)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(35)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(36)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(37)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(38)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(39)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(40)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(41)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(42)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(43)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(44)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(45)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(46)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(47)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(48)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(49)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(50)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(51)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(52)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(55)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(56)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(57)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(58)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(59)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(60)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(61)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(62)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(63)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(64)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(65)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(66)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(67)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(68)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(69)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(70)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(71)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(72)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(73)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(74)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	NO_SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	ONE_ELEM
 		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	ONE_ELEM
 		radio_freq_array	STOP_ARRAY_124

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_124
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	ONE_ELEM
 		radio_freq_array	STOP_ARRAY_14

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		timing_validity	TV_VALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		timing_validity	TV_VALID_TIMING_INFO

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		timing_validity	TV_VALID_TIMING_INFO

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

History:	24.09.99	MPA	Initial

20.06.01	MSB	fn_offset in (6) corrected,

20.06.01	MSB	numbering of msc corrected

07.02.02	LG	changed value for ba_id
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR053: Reading of Neighbour Cell BCCH, failed

Description:	ALR requests successive reading of BCCH for all neighbour cells. This failed for channel 14.

Preamble:	ALR046A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<==============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<==============================*
(9)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<==============================*
(11)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(12)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<==============================*
(14)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(15)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(17)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<==============================*
(18)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(20)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<==============================*
(21)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(22)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(23)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<==============================*		  |
(24)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<==============================*
(25)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(26)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<==============================*
(27)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(28)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(29)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(30)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(31)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<==============================*		  |
(32)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(33)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<==============================*
(34)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(35)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(36)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<==============================*
(37)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *==============================>*
(38)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(39)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
(40)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<==============================*		  |
(41)	 |	MPH_UNITDATA_IND	  |		  |
	 *<==============================*		  |
(42)	 |	MPH_UNITDATA_IND	  |		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	ONE_ELEM
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	ONE_ELEM
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	ONE_ELEM
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	ONE_ELEM
 		radio_freq_array	STOP_ARRAY_14

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	ONE_ELEM
 		radio_freq_array	STOP_ARRAY_124

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	ONE_ELEM
 		radio_freq_array	STOP_ARRAY_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_124
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

History:	08.12.99	MPA	Initial

20.06.01	MSB	fn_offset in (6) corrected

07.02.02	LG	changed value for ba_id
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR061: Ncell-Synch, NCC permitted Check

Description:	The BA list contains the serving cell 23 and the neighbour cells 1, 14 and 124. The fieldstrength is 56 for channel 23, 12 for channel 1, 44 for channel 14 and 25 for channel 124 (all values in GSM range). The ranking for the neighbour cells is 14, 124 and channel 1. Each reports contains two fieldstrength values per channel. The multiframe period is set to 6. The first measurement report is send to RR after five reports from PL. Then after each three reports from PL a measurement report is send to RR. The NCC permitted check for neighbour cell 124 fails. The cell shall be excluded from further attempts.

Preamble:	ALR046D

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(13)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(15)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(16)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(17)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(20)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(24)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(26)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(27)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(28)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(29)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(30)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(30)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_16
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_0
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_16
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_0
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_NCC
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_NCC
		gprs_sync	NOT_USED

History:	08.12.99	MPA	Initial

20.06.01	MSB	fn_offset in (6) corrected

07.02.02	LG	changed value for ba_id
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR650: Multiband = 0, Serving Cell is GSM 900, 8 channels

Description:	The multiband parameter is set to 0, that means the neighbourcells are ranked after the fieldstrength. The serving cell is 23 (that means in the GSM 900 frequency band). The neighbourcell list contains eight channels : 1, 14, 25, 124, 512 580, 637 and 885. It is expected, that ALR starts synchronization to the six strongest cells 637, 25, 14, 512, 580, 885. This channels must be included in the measurement report to RR.

Preamble:	ALR607

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(16)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(17)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(19)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(20)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(23)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(24)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(25)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(26)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(27)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(28)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(29)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(30)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(32)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(33)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(34)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(35)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(36)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(37)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(38)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(39)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(40)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(41)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(42)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(43)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(44)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(45)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(46)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(47)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(48)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(49)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(50)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(51)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(52)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_DUAL

- MPH_IDLE_REQ
		mod	NOT_USED
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_4
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_6
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	CHLIST_1_14_25_124_512_580_637_885_FFFF

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_1_14_25_124_512_580_637_885_23
		num_of_chans	CHANNELS_9
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_637
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_25
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_512
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_580
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_885
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NOT_USED
		gprs_sync	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_637
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_637
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_25
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_25
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
 		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_512
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_512
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_580
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_580
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_885
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_885
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_637
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_637

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_25
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_25

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_512
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_512

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_580
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_580

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_885
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_885

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_8
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_25
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_512
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_580
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_637
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_885
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_8
		gprs_sync	NOT_USED

History:	22.01.00	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

20.06.01	MSB	fn_offset in (6) corrected

20.07.01	MSB	channel list adapted

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ / 
			MPHC_NCELL_SYNC_IND)

### ALR651: Multiband = 0, Serving Cell is DCS 1800, 8 channels

Description:	The multiband parameter is set to 0, that means the neighbourcells are ranked after the fieldstrength. The serving cell is 578 (that means in the DCS 1800 frequency band). The neighbourcell list contains eight channels : 1, 14, 25, 124, 512 580, 637 and 885. It is expected, that ALR starts synchronization to the six strongest cells 14, 25, 512, 580, 637, 885. This channels must be included in the measurement report to RR.

Preamble:	ALR607

RR/DL		 ALR		 PL
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(16)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(17)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(19)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(20)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(23)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(24)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(25)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(26)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(27)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(28)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(29)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(30)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(32)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(33)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(34)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(35)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>* 
(36)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(37)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(38)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(39)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(40)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(41)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(42)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(43)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(44)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(45)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(46)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(47)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(48)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(49)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(50)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(51)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(52)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_DUAL

- MPH_IDLE_REQ
		mod	NOT_USED
		arfcn	ARFCN_578
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_4
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_6
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	CHLIST_1_14_25_124_512_580_637_885_FFFF

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_1_14_25_124_512_580_637_885_578
		num_of_chans	CHANNELS_9
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_578_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_637
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_25
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_512
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_580
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_885
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_578_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_578
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_637
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_637
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_25
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_25
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_512
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_512
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_580
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_580
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_885
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_885
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_578_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_637
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_637

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_25
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_25

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_512
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_512

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_580
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_580

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_885
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_885

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_578_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_578_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_578
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_8
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_25
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_512
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_580
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_637
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_885
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_578_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_578_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_578_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_578
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_8
		gprs_sync	NOT_USED

History:	24.01.00	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

20.06.01	MSB	fn_offset in (6) corrected

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ / 
			MPHC_NCELL_SYNC_IND)

### ALR652: Multiband = 1, Serving Cell is GSM 900, 8 channels

Description:	The multiband parameter is set to 1, that means at least one of the neighbourcells is member of the non-serving cell band (that means in the DCS 1800 frequency band). The serving cell is 23 (that means in the GSM 900 frequency band). The neighbourcell list contains eight channels : 1, 11, 14, 25,87, 124, 512 and 885. It is expected, that ALR starts synchronization to the six cells 14, 87, 25, 11, 1 and 512. This channels must be included in the measurement report to RR. ALR shall not synchronize to 885, although the cell has a better fieldstrength than most channels of the GSM 900 frequency band.

Preamble:	ALR607

RR/DL		 ALR		 PL
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(16)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(17)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(18)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(20)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(21)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(22)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(23)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(24)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(26)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(27)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(28)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(29)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(30)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(32)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(33)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(34)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(35)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(36)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>* 
(37)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(38)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(39)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(40)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(41)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(42)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(43)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(44)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(45)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(46)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(47)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(48)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(49)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(50)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(51)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(52)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(55)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_DUAL

- MPH_IDLE_REQ
		mod	NOT_USED
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_4
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_6
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_1
		arfcn	CHLIST_1_11_14_25_87_124_512_885_FFFF

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_1_11_14_25_87_124_512_885_23
		num_of_chans	CHANNELS_9
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_512
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_885
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_11
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_87
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_25
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_87
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_87
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_25
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_25
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_11
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_11
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_512
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_512
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_87
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_87

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_25
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_25

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_11
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_11

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_512
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_512

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_8_1
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_11
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_25
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_87
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_512
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_8_1
		gprs_sync	NOT_USED

History:	24.01.00	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

20.06.01	MSB	fn_offset in (6) corrected

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ / 
				MPHC_NCELL_SYNC_IND)

29.10.02	DL	Order of NCELL-SYN requests corrected

### ALR653: Multiband = 1, Serving Cell is GSM 900, 4 channels

Description:	The multiband parameter is set to 1, that at least one of the neighbourcells is member of the non-serving cell band (that means in the DCS 1800 frequency band). The serving cell is 23 (that means in the GSM 900 frequency band). The neighbourcell list contains four channels : 1, 14, 512 and 885. It is expected, that ALR starts synchronization to the six cells 1, 14, 512 and 885. This channels must be included in the measurement report to RR.

Preamble:	ALR607

RR/DL		 ALR		 PL
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(14)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(15)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(16)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(17)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(19)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(20)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(23)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(24)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(26)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(27)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(28)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(29)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>* 
(30)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(32)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(33)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(34)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(35)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(36)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(37)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(38)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(39)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(40)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(41)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(42)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_DUAL

- MPH_IDLE_REQ
		mod	NOT_USED
		arfcn	ARFCN_23
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_4
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_6
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_1
		arfcn	CHLIST_1_14_512_885_FFFF

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_23_1_14_512_885
		num_of_chans	CHANNELS_5
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_512
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_885
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_512
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_512
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_885
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_885
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_512
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_512

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_885
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_885

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_4_1
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_512
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_885
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_4_1
		gprs_sync	NOT_USED

History:	24.01.00	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

20.06.01	MSB	fn_offset in (6) corrected

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ / 
			MPHC_NCELL_SYNC_IND)

### ALR654: Multiband = 1, Serving Cell is DCS 1800, 8 channels

Description:	The multiband parameter is set to 1, that at least one of the neighbourcells is member of the non-serving cell band (that means in the GSM 900 frequency band). The serving cell is 637 (that means in the DCS 1800 frequency band). The neighbourcell list contains eight channels : 1, 14, 512, 513, 600, 700, 810 and 885. It is expected, that ALR starts synchronization to the six cells 14, 513, 600, 700, 810 and 885. This channels must be included in the measurement report to RR. ALR shall not synchronize to 1, although the cell has a better fieldstrength than the most channels of the DCS 1800 frequency band.

Preamble:	ALR607

RR/DL		 ALR		 PL
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(16)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(18)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(19)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(20)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(23)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(24)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(25)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(26)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(27)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(28)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(29)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(30)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(31)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(32)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(33)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(34)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(35)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(36)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(37)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>* 
(38)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(39)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(40)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(41)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(42)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(43)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(44)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(45)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(46)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(47)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(48)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(49)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(50)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(51)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(52)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(55)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(56)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_DUAL

- MPH_IDLE_REQ
		mod	NOT_USED
		arfcn	ARFCN_637
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_4
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_6
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_1
		arfcn	CHLIST_1_14_512_513_600_700_810_885_FFFF

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_1_14_512_513_600_700_810_885_637
		num_of_chans	CHANNELS_9
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_700
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_600
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_513
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_810
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_885
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_512
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_637
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_700
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_700
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_600
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_600
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_513
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_513
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_810
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_810
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_885
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_885
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_700
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_700

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_600
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_600

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_513
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_513

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_810
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_810

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_885
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_885

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_14

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_637
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_1800_8_1
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_513
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_600
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_700
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_810
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_885
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_8_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_637
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_1800_8_1
		gprs_sync	NOT_USED

History:	24.01.00	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

20.06.01	MSB	fn_offset corrected in (16)

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ / 
				MPHC_NCELL_SYNC_IND)

29.10.02	DL	Order of NCELL-SYN requests corrected;
			NCELL-SYN requests added (E-OTD changes)

### ALR655: Multiband = 1, Serving Cell is DCS 1800, 4 channels

Description:	The multiband parameter is set to 1, that at least one of the neighbourcells is member of the non-serving cell band (that means in the GSM 900 frequency band). The serving cell is 637 (that means in the DCS 1800 frequency band). The neighbourcell list contains four channels : 1, 14, 512 and 885. It is expected, that ALR starts synchronization to the six cells 1, 14, 512 and 885. This channels must be included in the measurement report to RR.

Preamble:	ALR607

RR/DL		 ALR		 PL
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(14)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(15)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(16)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(17)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(19)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(20)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(23)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(24)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(26)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(27)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*

(28)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(29)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>* 
(30)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*

(32)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(33)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(34)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(35)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(36)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |

(37)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |

(38)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |

(39)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(40)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(41)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(42)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_DUAL

- MPH_IDLE_REQ
		mod	NOT_USED
		arfcn	ARFCN_637
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_4
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_6
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_1
		arfcn	CHLIST_1_14_512_885_FFFF

- sync_only	NOT_USED

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHLIST_637_1_14_512_885
		num_of_chans	CHANNELS_5
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_512
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_885
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_637
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_512
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_512
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_885
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_885
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_512
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_512

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_885
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_885

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_637
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_1800_4_1
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_512
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_885
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_637_4_1
		nbr_of_carriers	CHANNELS_4
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_637
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_1800_4_1
		gprs_sync	NOT_USED

History:	24.01.00	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)

20.06.01	MSB	fn_offset corrected in (14)

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ / 
			MPHC_NCELL_SYNC_IND)

### ALR902: Synchronisation to Neighbour Cells successful (sys info 4)

Description:	The BA list contains the serving cell 23 and the neighbour cells 1, 14 and 124. The fieldstrength is 56 for channel 23, 12 for channel 1, 44 for channel 14 and 25 for channel 124 (all values in GSM range). The ranking for the neighbour cells is 14, 124 and channel 1. Each report contains two fieldstrength values per channel. The multiframe period is set to 6. The first measurement report is send to RR after five reports from PL. Then after each three reports from PL a measurement report is send to RR.

Preamble:	ALR046A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(16)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(24)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(27)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(28)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(29)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(30)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(31)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(32)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(33)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(34)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(35)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(36)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(37)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(38)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(39)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(40)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(41)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(42)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(43)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(44)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(45)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(46)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(47)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(48)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(49)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(50)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(51)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(52)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(55)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(56)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(57)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(58)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(59)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(60)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(61)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(62)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(63)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_124

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

History:	24.09.99	MPA	Initial

20.06.01	MSB	fn_offset corrected in (6)

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR903: Synchronisation to Neighbour Cells successful (sys info 4 and 7)

Description:	The BA list contains the serving cell 23 and the neighbour cells 1, 14 and 124. The fieldstrength is 56 for channel 23, 12 for channel 1, 44 for channel 14 and 25 for channel 124 (all values in GSM range). The ranking for the neighbour cells is 14, 124 and channel 1. Each report contains two fieldstrength values per channel. The multiframe period is set to 6. The first measurement report is send to RR after five reports from PL. Then after each three reports from PL a measurement report is send to RR.

Preamble:	ALR046A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(16)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(22)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(23)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(25)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(26)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(29)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(30)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(32)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(34)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(35)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(37)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(38)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(40)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(41)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(42)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(43)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(44)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(45)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(46)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(47)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(48)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(49)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(50)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(51)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(52)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(55)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(56)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(57)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(58)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(59)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(60)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(61)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(62)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(63)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(64)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(65)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(66)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(67)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(68)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(69)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(70)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(71)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(72)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(73)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(74)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(75)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(76)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(77)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(78)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4_ACS
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_7_8
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4_ACS
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_7_8
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4_ACS
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_124

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_7_8
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_7
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_7
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_7
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_124

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_2
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_7
		ti	TI_0
		si7_rest_oct	SI7_REST_OCT_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_2
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_7
		ti	TI_0
		si7_rest_oct	SI7_REST_OCT_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_2
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_7
		ti	TI_0
		si7_rest_oct	SI7_REST_OCT_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

History:	24.09.99	MPA	Initial

20.06.01	MSB	fn_offset corrected in (6)

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR904: Synchronisation to Neighbour Cells successful (sys info 4 and 8)

Description:	The BA list contains the serving cell 23 and the neighbour cells 1, 14 and 124. The fieldstrength is 56 for channel 23, 12 for channel 1, 44 for channel 14 and 25 for channel 124 (all values in GSM range). The ranking for the neighbour cells is 14, 124 and channel 1. Each report contains two fieldstrength values per channel. The multiframe period is set to 6. The first measurement report is send to RR after five reports from PL. Then after each three reports from PL a measurement report is send to RR.

Preamble:	ALR046A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(16)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(22)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(23)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(25)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(26)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(29)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(30)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(32)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(33)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(34)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(36)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(37)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(39)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(40)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(42)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(43)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(44)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(45)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(46)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(47)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(48)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(49)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(50)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(51)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(52)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(55)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(56)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(57)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(58)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(59)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(60)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(61)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(62)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(63)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(64)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(65)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(66)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(67)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(68)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(69)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(70)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(71)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(72)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(73)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(74)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(75)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(76)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(77)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(78)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(79)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(80)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(81)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(82)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4_ACS
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_7_8
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4_ACS
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_7_8
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_4_ACS
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_124

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_7_8
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_8
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_8
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_8
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_124

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_2
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_8
		ti	TI_0
		si8_rest_oct	SI8_REST_OCT_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_2
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_8
		ti	TI_0
		si8_rest_oct	SI8_REST_OCT_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_4
		ti	TI_0
		loc_area_ident	LOC_AREA_IDENT_1
		cell_select	CELL_SELECT_2
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_8
		ti	TI_0
		si8_rest_oct	SI8_REST_OCT_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

History:	24.09.99	MPA	Initial

20.06.01	MSB	fn_offset corrected in (6)

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR905: Synchronisation to Neighbour Cells successful (unexpected msg and sys info 3)

Description:	The BA list contains the serving cell 23 and the neighbour cells 1, 14 and 124. The fieldstrength is 56 for channel 23, 12 for channel 1, 44 for channel 14 and 25 for channel 124 (all values in GSM range). The ranking for the neighbour cells is 14, 124 and channel 1. Each report contains two fieldstrength values per channel. The multiframe period is set to 6. The first measurement report is send to RR after five reports from PL. Then after each three reports from PL a measurement report is send to RR.

Preamble:	ALR046A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(16)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(22)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(23)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(25)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(26)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(29)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(30)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(32)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(33)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(34)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(36)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(37)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(39)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(40)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(42)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(43)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(44)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(45)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(46)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(47)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(48)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(49)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(50)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(51)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(52)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(55)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(56)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(57)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(58)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(59)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(60)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(61)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(62)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(63)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(64)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(65)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(66)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(67)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(68)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(69)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(70)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(71)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(72)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(73)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(74)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(75)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(76)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(77)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(78)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(79)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_2
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_7
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_8
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_124

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_124

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

History:	24.09.99	MPA	Initial

20.06.01	MSB	fn_offset corrected in (6)

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR906: Synchronisation to Neighbour Cells successful (read error and sys info 3)

Description:	The BA list contains the serving cell 23 and the neighbour cells 1, 14 and 124. The fieldstrength is 56 for channel 23, 12 for channel 1, 44 for channel 14 and 25 for channel 124 (all values in GSM range). The ranking for the neighbour cells is 14, 124 and channel 1. Each report contains two fieldstrength values per channel. The multiframe period is set to 6. The first measurement report is send to RR after five reports from PL. Then after each three reports from PL a measurement report is send to RR.

Preamble:	ALR046A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(16)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(22)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(23)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(25)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(26)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(29)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(30)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(32)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(33)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(34)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(36)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(37)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(39)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(40)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(42)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(43)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(44)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(45)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(46)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(47)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(48)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(49)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(50)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(51)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(52)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(55)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(56)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(57)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(58)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(59)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(60)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(61)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(62)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(63)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(64)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(65)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(66)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(67)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(68)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(69)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(70)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(71)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(72)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(73)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(74)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(75)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(76)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(77)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(78)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(79)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	INVALID_BLOCK
		l2_frame	L2_NO_CONTENT
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_124

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_124

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

History:	24.09.99	MPA	Initial

20.06.01	MSB	fn_offset corrected in (6)

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

### ALR907: Synchronisation to Neighbour Cells successful (sys info 3)

Description:	The BA list contains the serving cell 23 and the neighbour cells 1, 14 and 124. The fieldstrength is 56 for channel 23, 12 for channel 1, 44 for channel 14 and 25 for channel 124 (all values in GSM range). The ranking for the neighbour cells is 14, 124 and channel 1. Each reports contains two fieldstrength values per channel. The multiframe period is set to 6. The first measurement report is send to RR after five reports from PL. Then after each three reports from PL a measurement report is send to RR.

Preamble:	ALR046A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(6)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(11)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(16)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(18)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(24)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  *=============================>*
(27)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(28)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(29)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(30)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(31)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(32)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(33)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(34)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(35)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(36)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(37)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(38)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(39)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(40)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(41)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(42)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(43)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(44)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(45)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(46)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(47)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(48)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(49)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(50)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(51)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(52)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(55)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(56)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
(57)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(58)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(59)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(60)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(61)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(62)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(63)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_14
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_14
 		time_alignment	TIME_ALIGNMT_14
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_124
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_124
 		time_alignment	TIME_ALIGNMT_124
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_124
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_NCELL_SYNC_IND
 		radio_freq	ARFCN_1
 		sb_flag	SB_FOUND
 		fn_offset	FN_OFFSET_1
 		time_alignment	TIME_ALIGNMT_1
 		bsic	BSIC_1
		neigh_id	NOT_USED
		attempt	NOT_USED
		pm	NOT_USED
		toa	NOT_USED
		angle	NOT_USED
		snr	NOT_USED
		eodt_data_valid	EOTD_NOT_PRES
		mode	NOT_USED
		d_eotd_first	NOT_USED
		d_eotd_max	NOT_USED
		d_eotd_nrj	NOT_USED
		a_eotd_crosscor	NOT_USED
		time_tag	NOT_USED
		fn_sb_neigh	NOT_USED
		fn_in_sb	NOT_USED
		toa_correction	NOT_USED
		delta_fn	NOT_USED
		delta_qbit	NOT_USED

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_1
		time_alignment	TIME_ALIGNMT_1
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NO_NCELLS
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_1

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
 		radio_freq_array_size	STOP_SIZE_1
 		radio_freq_array	STOP_ARRAY_124

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
 		fn_offset	NOT_USED
 		time_alignment	NOT_USED
 		timing_validity	TV_VALID_TIMING_INFO_SB

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	NOT_USED
		rx_qual_full	NOT_USED
		rx_qual_sub	NOT_USED
		dtx	NOT_USED
		otd	NOT_USED
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_1_14_124
		gprs_sync	NOT_USED

History:	24.09.99	MPA	Initial

20.06.01	MSB	fn_offset corrected in (16)

07.02.02	LG	changed value of ba_id
	26.09.02	DL	E-OTD changes (MPHC_NCELL_SYNC_IND)

## Short Message Cell Broadcast

### ALR800: Configuration CBCH followed by MMI Request

Description:	The CBCH channel is configured. Then MMI requests reading of CBCH for the message identifier 3, 7 and 11 to 13.

Preamble:	ALR006

Variants:	<A>..<B>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |  MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(6) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(7)	 |	MPH_CBCH_REQ	  |		  |
	 *===============================>*		  |
(8)	 |	MMI_CBCH_REQ	  |		  |
	 *===============================>*		  |
(10)	 |		  |	MPHC_CONFIG_CBCH_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPHC_CBCH_SCHEDULE_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_CBCH_SCHEDULE_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_GSM_900

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_14
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_CBCH_REQ
<A>	cbch	CBCH_DESCRIPTION_4
<B>	cbch	CBCH_DESCRIPTION_8

- MMI_CBCH_REQ
		msg_id	MSG_ID_3_7_11_TO_13
		dcs_id	DCS_ID_EMPTY
		modus	CBCH_ACCEPT

- MPHC_CONFIG_CBCH_REQ
<A>	cbch_desc	CHANNEL_DESC_CBCH_4
<B>	cbch_desc	CHANNEL_DESC_CBCH_8
		cbch_freq_list	FREQ_LIST

- MPHC_CBCH_SCHEDULE_REQ
		cbch_select	CBCH_READ_NORM
 		schedule_length 	SCHED_LEN_0
 		first_blocks_0	NOT_USED
 		first_blocks_1	NOT_USED

- MPHC_CBCH_SCHEDULE_REQ
		cbch_select	CBCH_READ_EXT
 		schedule_length 	SCHED_LEN_0
 		first_blocks_0	NOT_USED
 		first_blocks_1	NOT_USED

History:	10.01.00	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR801: MMI Request followed by Configuration of CBCH

Description:	MMI requests reading of CBCH for the message identifier 3, 7 and 11 to 13. Then the CBCH channel is configured and reading of CBCH starts.

Preamble:	ALR006

Variants:	<A>..<C>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(6) 	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(7)	 |	MMI_CBCH_REQ	  |		  |
	 *===============================>*		  |
(8)	 |	MPH_CBCH_REQ	  |		  |
	 *===============================>*		  |
(9)	 |		  |	MPHC_CONFIG_CBCH_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPHC_CBCH_SCHEDULE_REQ	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_CBCH_SCHEDULE_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_GSM_900

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_14
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_8
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MMI_CBCH_REQ
<A>	msg_id	MSG_ID_3_7_11_TO_13
<B>	msg_id	MSG_ID_3_7_11_TO_13
<C>	msg_id	MSG_ID_1_TO_20
		dcs_id	DCS_ID_EMPTY
		modus	CBCH_ACCEPT

- MPH_CBCH_REQ
<A>	cbch	CBCH_DESCRIPTION_4
<B>	cbch	CBCH_DESCRIPTION_8
<C>	cbch	CBCH_DESCRIPTION_4

- MPHC_CONFIG_CBCH_REQ
<A>	cbch_desc	CHANNEL_DESC_CBCH_4
<B>	cbch_desc	CHANNEL_DESC_CBCH_8
<C>	cbch_desc	CHANNEL_DESC_CBCH_4
		cbch_freq_list	FREQ_LIST

- MPHC_CBCH_SCHEDULE_REQ
		cbch_select	CBCH_READ_NORM
 		schedule_length 	SCHED_LEN_0
 		first_blocks_0	NOT_USED
 		first_blocks_1	NOT_USED

- MPHC_CBCH_SCHEDULE_REQ
		cbch_select	CBCH_READ_EXT
 		schedule_length 	SCHED_LEN_0
 		first_blocks_0	NOT_USED
 		first_blocks_1	NOT_USED

History:	13.1.00	MPA	Initial
	12.07.00	DG	MPH_CLASSMARK_REQ:
				class changed into classmark
				(Forum G23M / No 0057)
	26.09.02	DL	E-OTD changes (MPH_IDLE_REQ)

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR802: Reception of NULL Message

Description:	ALR receives a NULL message. No further reaction is expected.

Preamble:	ALR801A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(NULL Message)	  |
	 |		  *<=============================*
MUTE (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CBCH
		error_flag	VALID_BLOCK
		l2_frame	NULL_MESSAGE
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	13.1.00	MPA	Initial

### ALR803: Reception of expected CBCH Message

Description:	ALR receives an expected CBCH message. The content is forwarded to MMI.
Variant A: single message identifier (7)
Variant B: message identifier specified by range (11), lower boundary
Variant C: message identifier specified by range (12), middle of the range
Variant D: message identifier specified by range (13), upper boundary

Preamble:	ALR801A

Variants:	<A>..<D>

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(CBCH first Block)	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_CBCH_INFO_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(CBCH second Block)	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(CBCH third Block)	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(CBCH fourth Block)	  |
	 |		  *<=============================*
(6)	 |	MMI_CBCH_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CBCH
		error_flag	VALID_BLOCK
<A>	l2_frame	CBCH_1_7
<B>	l2_frame	CBCH_1_11
<C>	l2_frame	CBCH_1_12
<D>	l2_frame	CBCH_1_13
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_CBCH_INFO_REQ
		tb_bitmap	CBCH_NORM_BLOCK234

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CBCH
		error_flag	VALID_BLOCK
		l2_frame	CBCH_2
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CBCH
		error_flag	VALID_BLOCK
		l2_frame	CBCH_3
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CBCH
		error_flag	VALID_BLOCK
		l2_frame	CBCH_4
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MMI_CBCH_IND
<A>	cbch_msg	CBCH_MSG_7
<B>	cbch_msg	CBCH_MSG_11
<C>	cbch_msg	CBCH_MSG_12
<D>	cbch_msg	CBCH_MSG_13
		cbch_len	CBCH_LEN_88

History:	13.1.00	MPA	Initial

### ALR804: Reception of unexpected CBCH Message

Description:	ALR receives an unexpected CBCH message. No reaction is expected after receiving the first block.

Preamble:	ALR801A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(CBCH first Block)	  |
	 |		  *<=============================*
TIMEOUT (3000)
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CBCH
		error_flag	VALID_BLOCK
		l2_frame	CBCH_1_8
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

History:	13.1.00	MPA	Initial

### ALR805: Reception of unscheduled SCHEDULE Message

Description:	ALR receives an unscheduled SCHEDULE message. The content is used for going into DRX mode.

Preamble:	ALR801A

RR/DL		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(SCHEDULE first Block)	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_CBCH_INFO_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(SCHEDULE second Block)	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(SCHEDULE third Block)	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_DATA_IND	  |
	 |		  |	(SCHEDULE fourth Block)	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_CBCH_SCHEDULE_REQ	  |
	 |		  *=============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CBCH
		error_flag	VALID_BLOCK
		l2_frame	SCHEDULE_1_B
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_CBCH_INFO_REQ
		tb_bitmap	CBCH_NORM_BLOCK234

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CBCH
		error_flag	VALID_BLOCK
		l2_frame	SCHEDULE_2
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CBCH
		error_flag	VALID_BLOCK
		l2_frame	SCHEDULE_3
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CBCH
		error_flag	VALID_BLOCK
		l2_frame	SCHEDULE_4
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPHC_CBCH_SCHEDULE_REQ
		cbch_select	CBCH_READ_NORM
		schedule_length	SCHED_LEN_5
		first_blocks_0	FIRST_BLOCKS_0_B
		first_blocks_1	FIRST_BLOCKS_1_B

History:	13.1.00	MPA	Initial

## E-OTD

### ALR950: Power measurement on 3 Neighbour Cells – E-OTD off (Idle Mode)

Description:	After receiving the power levels of the neighbour cells (MPHC-RXLEV-PERIODIC indication primitive), synchronization of neighbour cells begins (MPHC-NCELL-SYNC request/indication primitives). After 30 seconds (third expiry of TIM_NCSYNC timer), synchronization is halted and a list of neighbour cells for synchronization is sent to Layer 1 (MPHC-NCELL-LIST-SYNC request primitive), following which synchronization is recommenced.

Preamble:	ALR013

RR		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
START_TIMEOUT (29500)
(2)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *==============================>*
(3)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 124)	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 124)	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 1)	  |
	 |		  *=============================>*
(7)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 1)	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *=============================>*
(9)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 124)	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 124)	  |
	 |		  *<=============================*
(12)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 1)	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 1)	  |
	 |		  *<=============================*
(14)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *==============================>*
(15)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 124)	  |
	 |		  *==============================>*
(16)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 1)	  |
	 |		  *==============================>*
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(18)	 |	MPH_MEASUREMENT_IND	  |		  |
	 |	(SC - ARFCN 23)	  |		  |
	 *<==============================*		  |
(19)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 1)	  |		  |
	 *<==============================*		  |
(20)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 14)	  |		  |
	 *<==============================*		  |
(21)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 124)	  |		  |
	 *<==============================*		  |
WAIT_TIMEOUT
START_TIMEOUT (30000)
(22)	 |		  |	MPHC_NCELL_LIST_SYNC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_124
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_1
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_124
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_124

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	RX_0
		rx_qual_full	RX_0
		rx_qual_sub	RX_0
		dtx	RX_0
		otd	OTD_0
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_816
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NORMAL_MEAS_REP

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	FN_OFFSET_124
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	FN_OFFSET_124
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_124
		fn	FN_OFFSET_114
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_NCELL_LIST_SYNC_REQ
		eotd	NC_MON
		list_size	NO_NCELLS_3
		ncell_list	NCELL_LIST_3_EOTD

History:	26 September 2002	DL	Initial

### ALR952: Power measurement with change of BS – E-OTD off (Idle Mode)

Description:	After receiving the power levels of the neighbour cells (MPHC-RXLEV-PERIODIC indication primitive), synchronization of neighbour cells begins (MPHC-NCELL-SYNC request/indication primitives). After 30 seconds (third expiry of TIM_NCSYNC timer), synchronization is halted and a list of neighbour cells for synchronization is sent to Layer 1 (MPHC-NCELL-LIST-SYNC request primitive), following which synchronization is recommenced.
Cell reselection is then requested by RR (MPH-IDLE request primitive), nominating a cell which has been reassigned to a different BTS. ALR deselects the current serving cell (MPHC-STOP-SCELL_BCCH request primitive and selects the new serving cell (MPHC-NEW-SCELL request primitive).

Preamble:	ALR950

RR		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *<==============================*
(2)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 124)	  |
	 |		  *<==============================*
(3)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 1 – BS 2)	  |
	 |		  *<==============================*
(4)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 1)	  |
	 |		  *==============================>*
(5)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 1)	  |
	 |		  *<==============================*
(6)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 1)	  |
	 |		  *==============================>*
(7)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<==============================*
(9)	 |	MPH_MEASUREMENT_IND	  |		  |
	 *<==============================*		  |
(10)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 1)	  |		  |
	 *<==============================*		  |
WAIT_TIMEOUT
START_TIMEOUT (30000)
(11)	 |		  |	MPHC_NCELL_LIST_SYNC_REQ	  |
	 |		  *==============================>*
(12)	 |	MPH_IDLE_REQ	  |		  |
	 |	(ARFCN 1)	  |		  |
	 *==============================>*		  |
(13)	 |		  | MPHC_STOP_RXLEV_PERIODIC_REQ	  |
	 |		  *==============================>*
(14)	 |		  |	MPHC_STOP_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
(15)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *==============================>*
(16)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  |	(ARFCN 1 – BS 2)	  |
	 |		  *==============================>*
(17)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<==============================*
(18)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *==============================>*
(19)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_124
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_1
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_2
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_2
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_1
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_124

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RESULT_1
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	RX_0
		rx_qual_full	RX_0
		rx_qual_sub	RX_0
		dtx	RX_0
		otd	OTD_0
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_816
		ncells	NCELLS_NO_CONTENT
		gprs_sync	NORMAL_MEAS_REP

- MPH_UNITDATA_IND
		arfcn	ARFCN_1
		fn	FN_OFFSET_124
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_NCELL_LIST_SYNC_REQ
		eotd	NC_MON
		list_size	NO_NCELLS_3
		ncell_list	NCELL_LIST_3_EOTD

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_1
		ext_bcch	NORMAL_PGM
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_4
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_RXLEV_PERIODIC_REQ
		param	NOT_USED

- MPHC_STOP_NCELL_SYNC_REQ
		radio_freq_array_size	STOP_SIZE_12
		radio_freq_array	STOP_ARRAY_EMPTY_12

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_NEW_SCELL_REQ
		radio_freq	ARFCN_1
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGN_14
		tsc	BSIC_2

- MPHC_NEW_SCELL_CON
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	NOT_USED
		bs_ag_blks_res	NOT_USED
		bcch_combined	NOT_USED
		ccch_group	NOT_USED
		page_group	NOT_USED
		page_block_index	NOT_USED
		page_mode	NOT_USED

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	FULL_READ

History:	15 October 2002	DL	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR954: Power measurement on 8 Neighbour Cells – E-OTD off (Idle Mode)

Description:	RR selects Channel 23 as serving cell after reading the BCCH carrier.
After receiving the power levels of the neighbour cells (MPHC-RXLEV-PERIODIC indication primitive), synchronization of the six strongest neighbour cells begins (MPHC-NCELL-SYNC request/indication primitives). After 30 seconds (third expiry of TIM_NCSYNC timer), synchronization is halted and a list of neighbour cells for synchronization is sent to Layer 1 (MPHC-NCELL-LIST-SYNC request primitive), following which synchronization is recommenced.

Preamble:	ALR607

RR		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
START_TIMEOUT (29500)
(9)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 124)	  |
	 |		  *=============================>*
(16)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(17)	 |	MPH_MEASUREMENT_IND	  |		  |
	 |	(ARFCN 23 - SC)	  |		  |
	 *<===============================*		  |
(18)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *<=============================*
(19)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *=============================>*
(20)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *<=============================*
(21)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *=============================>*
(22)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *<=============================*
(23)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *=============================>*
(24)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *<=============================*
(25)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *=============================>*
(26)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *<=============================*
(27)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *=============================>*
(28)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *<=============================*
(29)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *=============================>*
(30)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(31)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *<=============================*
(32)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *=============================>*
(33)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *<=============================*
(34)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *=============================>*
(35)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *<=============================*
(36)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *=============================>*
(37)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *<=============================*
(38)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *=============================>*
(39)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *<=============================*
(40)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *=============================>*
(41)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *<=============================*
(42)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *=============================>*
(43)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(44)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(45)	 |	MPH_MEASUREMENT_IND	  |		  |
	 |	(ARFCN 23 - SC)	  |		  |
	 *<===============================*		  |
(46)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 14)	  |		  |
	 *<===============================*		  |
(47)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 25)	  |		  |
	 *<===============================*		  |
(48)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 512)	  |		  |
	 *<===============================*		  |
(49)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 580)	  |		  |
	 *<===============================*		  |
(50)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 637)	  |		  |
	 *<===============================*		  |
(51)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 885)	  |		  |
	 *<===============================*		  |
(52)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(55)	 |	MPH_MEASUREMENT_IND	  |		  |
	 |	(ARFCN 23 - SC)	  |		  |
	 *<===============================*		  |
WAIT_TIMEOUT
(56)	 |		  |	MPHC_STOP_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
(57)	 |		  |	MPHC_STOP_NCELL_SYNC_CON	  |
	 |		  *<==============================*
START_TIMEOUT (30000)
(58)	 |		  |	MPHC_NCELL_LIST_SYNC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_DUAL

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_23
		ext_bcch	NORMAL_PGM
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_4
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_NOT_PRES
		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_6
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
 		schedule_array_size	SCHED_SIZE_1
 		schedule_array	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	NCELL_LIST_EOTD
		sync_only	NORMAL_BA

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHAN_LIST_EOTD
		num_of_chans	CHANNELS_9
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_637
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_25
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_512
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_580
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_885
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	RX_0
		rx_qual_full	RX_0
		rx_qual_sub	RX_0
		dtx	RX_0
		otd	OTD_0
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NOT_USED
		gprs_sync	NORMAL_MEAS_REP

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_637
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_637
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_25
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_102
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_25
		fn_offset	FN_OFFSET_102
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_512
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_153
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_512
		fn_offset	FN_OFFSET_153
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_580
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_204
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_580
		fn_offset	FN_OFFSET_204
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_885
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_255
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_NOT_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_885
		fn_offset	FN_OFFSET_255
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_637
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_637

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_25
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_25

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_512
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_512

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_580
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_580

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_885
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_885

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	RX_0
		rx_qual_full	RX_0
		rx_qual_sub	RX_0
		dtx	RX_0
		otd	OTD_0
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_EOTD
		gprs_sync	NORMAL_MEAS_REP

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	FN_OFFSET_14
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_25
		fn	FN_OFFSET_14
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_512
		fn	FN_OFFSET_14
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_580
		fn	FN_OFFSET_14
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_637
		fn	FN_OFFSET_14
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_885
		fn	FN_OFFSET_14
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	RX_0
		rx_qual_full	RX_0
		rx_qual_sub	RX_0
		dtx	RX_0
		otd	OTD_0
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_EOTD
		gprs_sync	NORMAL_MEAS_REP

- MPHC_STOP_NCELL_SYNC_REQ
		radio_freq_array_size	STOP_SIZE_12
		radio_freq_array	STOP_ARRAY_EMPTY_12

- MPHC_STOP_NCELL_SYNC_CON
		param	NOT_USED

- MPHC_NCELL_LIST_SYNC_REQ
		eotd	NC_MON
		list_size	NO_NCELLS_6
		ncell_list	NCELL_LIST_6_EOTD_IDLE

History:		14 October 2002	DL	Initial

### ALR960: Power measurement – E-OTD on (Idle Mode)

Description:	RR selects Channel 23 as serving cell after reading the BCCH carrier.
On receipt of a list of neighbour cells (MPH-NEIGHBOURCELL request primitive) followed by the power measurements of these cells (MPHC-RXLEV-PERIODIC indication primitive), ALR starts a 10-second timer and requests Layer 1 to commence synchronization of these cells.
On expiry of this timer, ALR requests Layer 1 to stop synchronization of these cells and forwards a list of all synchronized neighbour cells to be measured to Layer 1 (MPHC-NCELL-LIST-SYNC request primitive).

Preamble:	ALR607

RR		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_CLASSMARK_REQ	  |		  |
	 *===============================>*		  |
(2)	 |	MPH_IDLE_REQ	  |		  |
	 *===============================>*		  |
(3)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *=============================>*
(5)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *=============================>*
(6)	 |	MPH_NEIGHBOURCELL_REQ	  |		  |
	 *===============================>*		  |
(7)	 |		  |	MPHC_RXLEV_PERIODIC_REQ	  |
	 |		  *=============================>*
(8)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
START_TIMEOUT (9500)
(9)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *=============================>*
(10)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *=============================>*
(11)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *=============================>*
(12)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *=============================>*
(13)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *=============================>*
(14)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *=============================>*
(15)	 |		  |	MPHC_NCELL_SYNC_REQ	  |
	 |		  |	(ARFCN 124)	  |
	 |		  *=============================>*
(16)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 23 - SC)	  |
	 |		  *<=============================*
(17)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(18)	 |	MPH_MEASUREMENT_IND	  |		  |
	 |	(ARFCN 23 - SC)	  |		  |
	 *<===============================*		  |
(19)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *<=============================*
(20)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *=============================>*
(21)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *<=============================*
(22)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *=============================>*
(23)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *<=============================*
(24)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *=============================>*
(25)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *<=============================*
(26)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *=============================>*
(27)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *<=============================*
(28)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *=============================>*
(29)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *<=============================*
(30)	 |		  |	MPHC_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *=============================>*
(31)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(32)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *<=============================*
(33)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *=============================>*
(34)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *<=============================*
(35)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *=============================>*
(36)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *<=============================*
(37)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *=============================>*
(38)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *<=============================*
(39)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *=============================>*
(40)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *<=============================*
(41)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *=============================>*
(42)	 |		  |	MPHC_NCELL_BCCH_IND	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *<=============================*
(43)	 |		  |	MPHC_STOP_NCELL_BCCH_REQ	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *=============================>*
(44)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(45)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(46)	 |	MPH_MEASUREMENT_IND	  |		  |
	 |	(ARFCN 23 - SC)	  |		  |
	 *<===============================*		  |
(47)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 14)	  |		  |
	 *<===============================*		  |
(48)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 25)	  |		  |
	 *<===============================*		  |
(49)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 512)	  |		  |
	 *<===============================*		  |
(50)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 580)	  |		  |
	 *<===============================*		  |
(51)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 637)	  |		  |
	 *<===============================*		  |
(52)	 |	MPH_UNITDATA_IND	  |		  |
	 |	(ARFCN 885)	  |		  |
	 *<===============================*		  |
(53)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(54)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(55)	 |		  |	MPHC_RXLEV_PERIODIC_IND	  |
	 |		  *<=============================*
(56)	 |	MPH_MEASUREMENT_IND	  |		  |
	 |	(ARFCN 23 - SC)	  |		  |
	 *<===============================*		  |
WAIT_TIMEOUT
(57)	 |		  |	MPHC_STOP_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
(58)	 |		  |	MPHC_STOP_NCELL_SYNC_CON	  |
	 |		  *<==============================*
START_TIMEOUT (9500)
(59)	 |		  |	MPHC_NCELL_LIST_SYNC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_CLASSMARK_REQ
		classmark	CLASS_DUAL

- MPH_IDLE_REQ
		mod	MODE_CELL_SELECTION
		arfcn	ARFCN_23
		ext_bcch	NORMAL_PGM
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_4
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_PRES
 		gprs_support	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	BS_PA_MFRMS_6
		bs_ag_blks_res	BS_AG_BLKS_RES_3
		bcch_combined	COMB_CCCH_NOT_COMB
		ccch_group	CCCH_GROUP_0
		page_group	PG_20
		page_block_index	PBI_2
		page_mode	PGM_REORG

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	NOT_USED

- MPH_NEIGHBOURCELL_REQ
		multi_band	MULTI_BAND_0
		arfcn	NCELL_LIST_EOTD
		sync_only	NORMAL_BA

- MPHC_RXLEV_PERIODIC_REQ
		chan_list	CHAN_LIST_EOTD
		num_of_chans	CHANNELS_9
		ba_id	BA_ID_1
		next_radio_freq_measured	CHAN_LIST_IDX_0

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_637
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_25
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_512
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_580
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_885
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_REQ
		radio_freq	ARFCN_124
		fn_offset	FN_OFFSET_0
		time_alignment	TIME_ALIGN_0
		timing_validity	TV_INVALID_TIMING_INFO

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_23
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_918
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	RX_0
		rx_qual_full	RX_0
		rx_qual_sub	RX_0
		dtx	RX_0
		otd	OTD_0
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_918
		ncells	NOT_USED
		gprs_sync	NORMAL_MEAS_REP

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_637
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_637
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_25
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_102
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_25
		fn_offset	FN_OFFSET_102
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_512
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_153
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_512
		fn_offset	FN_OFFSET_153
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_580
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_204
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_580
		fn_offset	FN_OFFSET_204
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_885
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_255
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	EOTD_0L
		a_eotd_crosscor	NOT_USED
		time_tag	EOTD_0L
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_BCCH_REQ
		radio_freq	ARFCN_885
		fn_offset	FN_OFFSET_255
		time_alignment	TIME_ALIGNMT_14
		tsc	BSIC_1
		bcch_blocks_required	NCELL_BCCH_SI_3_4
		gprs_prio	GPRS_PRIO_NORM

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_637
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_637

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_25
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_25

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_14
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_14

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_512
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_512

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_580
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_580

- MPHC_NCELL_BCCH_IND
		radio_freq	ARFCN_885
		l2_channel	L2_CHANNEL_NBCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_SYS_INFO_3
		tc	TC_2
		fn	FN_OFFSET_14

- MPHC_STOP_NCELL_BCCH_REQ
		radio_freq_array_size	ONE_ELEM
		radio_freq_array	STOP_ARRAY_885

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	RX_0
		rx_qual_full	RX_0
		rx_qual_sub	RX_0
		dtx	RX_0
		otd	OTD_0
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_EOTD
		gprs_sync	NORMAL_MEAS_REP

- MPH_UNITDATA_IND
		arfcn	ARFCN_14
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_25
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_512
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_580
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_637
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPH_UNITDATA_IND
		arfcn	ARFCN_885
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_SYS_INFO_3
		ti	TI_0
		cell_ident	CELL_IDENT_1
		loc_area_ident	LOC_AREA_IDENT_1
		ctrl_chan_desc	CTRL_CHAN_DESC_1
		cell_opt_bcch	CELL_OPT_BCCH_1
		cell_select	CELL_SELECT_1
		rach_ctrl	RACH_CTRL_1
		}

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPHC_RXLEV_PERIODIC_IND
		result	NCELL_RES_SC_23_8
		nbr_of_carriers	CHANNELS_8
		s_rxlev	RXLEV_56
		ba_id	BA_ID_1

- MPH_MEASUREMENT_IND
		arfcn	ARFCN_23
		rx_lev_full	RXLEV_56
		rx_lev_sub	RX_0
		rx_qual_full	RX_0
		rx_qual_sub	RX_0
		dtx	RX_0
		otd	OTD_0
		valid	VALID_REPORT
		fn_offset	FN_OFFSET_306
		ncells	NCELLS_SC_900_EOTD
		gprs_sync	NORMAL_MEAS_REP

- MPHC_STOP_NCELL_SYNC_REQ
		radio_freq_array_size	STOP_SIZE_12
		radio_freq_array	STOP_ARRAY_EMPTY_12

- MPHC_STOP_NCELL_SYNC_CON
		param	NOT_USED

- MPHC_NCELL_LIST_SYNC_REQ
		eotd	EOTD_MON
		list_size	NO_NCELLS_6
		ncell_list	NCELL_LIST_6_EOTD_IDLE

History:	26 September 2002	DL	Initial

16 October 2002		DL	Revised

### ALR962: Power measurement – E-OTD on (Idle Mode)

Description:	Completion of the E-OTD cell monitoring procedure is signalled by the receipt of a second MPHC-NCELL-SYNC indication primitive for the serving cell, whereupon the measurement results are forwarded to RR (MPH-NCELL-POS indication primitive).

Preamble:	ALR960

RR		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 23 - SC)	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 23 - SC)	  |
	 |		  *<=============================*
(9)	 |	MPH_NCELL_POS_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_23
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_918
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_12
		time_tag	nom_pos_1000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_637
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61952
		a_eotd_crosscor	eotd_crosscor_9
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_25
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_6
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61952
		a_eotd_crosscor	eotd_crosscor_9
		time_tag	nom_pos_1000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_512
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_6
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_580
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61952
		a_eotd_crosscor	eotd_crosscor_9
		time_tag	nom_pos_1000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_885
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_2
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_6
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_23
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_918
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_12
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPH_NCELL_POS_IND
		req_id	REQ_ID_FFFF
		eotd_res	EOTD_SUCC
		ta	TA_NOT_AVAIL
		fn	FN_OFFSET_0
		eotd_sc_res	eotd_sc_res_ok
		eotd_sc_res1	eotd_sc_res1_ok
		eotd_nc_res	EOTD_RESULT_6

History:	16 October 2002	DL	Initial

### ALR964: Power measurement – E-OTD on – no response from Layer 1 (Idle Mode)

Description:	ALR requests Layer 1 to stop synchronization of neighbour cells and starts a 10-second timer. As Layer 1 fails to respond the request is repeated on each expiry of the 10-second TIM_NCSYNC timer.

Preamble:	ALR960

RR		 ALR		 PL
	 |		  |		  |
WAIT_TIMEOUT
(1)	 |		  |	MPHC_STOP_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
START_TIMEOUT (9500)
(2)	 |		  |	MPHC_NCELL_LIST_SYNC_REQ	  |
	 |		  *==============================>*
WAIT_TIMEOUT
(3)	 |		  |	MPHC_STOP_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
START_TIMEOUT (9500)
(4)	 |		  |	MPHC_NCELL_LIST_SYNC_REQ	  |
	 |		  *==============================>*
WAIT_TIMEOUT
(5)	 |		  |	MPHC_STOP_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
START_TIMEOUT (9500)
(6)	 |		  |	MPHC_NCELL_LIST_SYNC_REQ	  |
	 |		  *==============================>*
WAIT_TIMEOUT
(7)	 |		  |	MPHC_STOP_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
START_TIMEOUT (9500)
(8)	 |		  |	MPHC_NCELL_LIST_SYNC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_STOP_NCELL_SYNC_REQ
		radio_freq_array_size	STOP_SIZE_12
		radio_freq_array	STOP_ARRAY_EMPTY_12

- MPHC_NCELL_LIST_SYNC_REQ
		eotd	EOTD_MON
		list_size	NO_NCELLS_6
		ncell_list	NCELL_LIST_6_EOTD_IDLE

- MPHC_STOP_NCELL_SYNC_REQ
		radio_freq_array_size	STOP_SIZE_12
		radio_freq_array	STOP_ARRAY_EMPTY_12

- MPHC_NCELL_LIST_SYNC_REQ
		eotd	EOTD_MON
		list_size	NO_NCELLS_6
		ncell_list	NCELL_LIST_6_EOTD_IDLE

- MPHC_STOP_NCELL_SYNC_REQ
		radio_freq_array_size	STOP_SIZE_12
		radio_freq_array	STOP_ARRAY_EMPTY_12

- MPHC_NCELL_LIST_SYNC_REQ
		eotd	EOTD_MON
		list_size	NO_NCELLS_6
		ncell_list	NCELL_LIST_6_EOTD_IDLE

- MPHC_STOP_NCELL_SYNC_REQ
		radio_freq_array_size	STOP_SIZE_12
		radio_freq_array	STOP_ARRAY_EMPTY_12

- MPHC_NCELL_LIST_SYNC_REQ
		eotd	EOTD_MON
		list_size	NO_NCELLS_6
		ncell_list	NCELL_LIST_6_EOTD_IDLE

History:	26 September 2002	DL	Initial

### ALR966: E-OTD measurement requested by upper layers (IDLE Mode)

Description:	RR requests E-OTD measurement of a list of cells forwarded in the MPH-NCELL-POS request primitive. However, as no dedicated channel is available, ALR rejects this request (MPH-NCELL-POS indication primitive with the eotd_res IE set to EOTD_REF).

Preamble:	ALR960

RR		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_NCELL_POS_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |	MPH_NCELL_POS_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_NCELL_POS_REQ
		req_id	REQ_ID_8
		ncell_eotd	NCELL_EOTD_7

- MPH_NCELL_POS_IND
		req_id	REQ_ID_8
		eotd_res	EOTD_REF
		ta	NOT_USED
		fn	NOT_USED
		eotd_sc_res	NOT_USED
		eotd_sc_res1	NOT_USED
		eotd_nc_res	NOT_USED

History:		18 October 2002	DL	Initial

### ALR968: Cell Reselection started during E-OTD measurement (Idle Mode)

Description:	RR request cell reselection (MPH-IDLE request primitive) following commencement of E-OTD measurement. E-OTD measurement is halted (MPHC-STOP-NCELL-SYNC request primitive), the current serving cell is deselected (MPHC-STOP-SCELL-BCCH request primitive) and ALR requests the new serving cell (MPHC-NEW-SCELL request primitive).

Preamble:	ALR960

RR		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_IDLE_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  | MPHC_STOP_RXLEV_PERIODIC_REQ  |
	 |		  *==============================>*
(3)	 |		  |	MPHC_STOP_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
(4)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPHC_NEW_SCELL_REQ	  |
	 |		  *==============================>*
(6)	 |		  |	MPHC_NEW_SCELL_CON	  |
	 |		  *<==============================*
(7)	 |		  |	MPHC_START_CCCH_REQ	  |
	 |		  *==============================>*
(8)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_IDLE_REQ
		mod	MODE_CELL_RESELECTION
		arfcn	ARFCN_14
		ext_bcch	NOT_USED
		comb_ccch	COMB_CCCH_NOT_COMB
		tn	TN_0
		dlt	DLT_10
		pg	PG_20
		bs_ag_blocks_res	BS_AG_BLKS_RES_3
		bs_pa_mfrms	BS_PA_MFRMS_6
		power	POWER_12
		ncc_permitted	NOT_PRESENT_8BIT
		reorg_only	NOT_USED
		eotd_avail	EOTD_PRES
		gprs_support	NOT_USED

- MPHC_STOP_RXLEV_PERIODIC_REQ
		param	NOT_USED

- MPHC_STOP_NCELL_SYNC_REQ
		radio_freq_array_size	STOP_SIZE_12
		radio_freq_array	STOP_ARRAY_EMPTY_12

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_NEW_SCELL_REQ
		radio_freq	ARFCN_14
		fn_offset	FN_OFFSET_124
		time_alignment	TIME_ALIGN_14
		tsc	BSIC_1

- MPHC_NEW_SCELL_CON
		param	NOT_USED

- MPHC_START_CCCH_REQ
		bs_pa_mfrms	NOT_USED
		bs_ag_blks_res	NOT_USED
		bcch_combined	NOT_USED
		ccch_group	NOT_USED
		page_group	NOT_USED
		page_block_index	NOT_USED
		page_mode	NOT_USED

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	NOT_USED

History:	26.09.02	DL	Initial

07.04.03	MSB	MPHC_SCELL_NBCCH_REQ after
			MPHC_START_CCCH_REQ included

### ALR970: Connection established (E-OTD -DEDICATED Mode)

Description:	A dedicated channel is assigned in preparation for E-OTD measurement.

Preamble:	ALR960

RR		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_RANDOM_ACCESS_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  | MPHC_STOP_RXLEV_PERIODIC_REQ	  |
	 |		  *==============================>*
(3)	 |		  | MPHC_STOP_RXLEV_PERIODIC_CON	  |
	 |		  *<==============================*
(4)	 |		  |	MPHC_STOP_NCELL_SYNC_REQ	  |
	 |		  *==============================>*
(5)	 |		  |	MPHC_STOP_NCELL_SYNC_CON	  |
	 |		  *<==============================*
(6)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *==============================>*
(7)	 |		  |	MPHC_STOP_SCELL_BCCH_CON	  |
	 |		  *<==============================*
(8)	 |		  |	MPHC_SCELL_NBCCH_REQ	  |
	 |		  *==============================>*
(9)	 |		  |	MPHC_RA_REQ	  |
	 |		  *==============================>*
(10)	 |		  |	MPHC_RA_CON	  |
	 |		  *<==============================*
(11)	 |	MPH_RANDOM_ACCESS_CNF	  |		  |
	 *<==============================*		  |
(12)	 |		  |	MPHC_RA_REQ	  |
	 |		  *==============================>*
(13)	 |		  |	MPHC_DATA_IND	  |
	 |		  *<=============================*
(14)	 |	MPH_UNITDATA_IND	  |		  |
	 *<===============================*		  |
(15)	 |	MPH_DEDICATED_REQ	  |		  |
	 *===============================>*		  |
(16)	 |		  |	MPHC_STOP_SCELL_BCCH_REQ	  |
	 |		  *=============================>*
(17)	 |		  |	MPHC_STOP_CCCH_REQ	  |
	 |		  *=============================>*
(18)	 |		  |	MPHC_STOP_RA_REQ	  |
	 |		  *=============================>*
(19)	 |		  |	MPHC_IMMED_ASSIGN_REQ	  |
	 |		  *=============================>*
(20)	 |		  |	MPHC_IMMED_ASSIGN_CON	  |
	 |		  *<=============================*
(21)	 |	MPH_DEDICATED_CNF	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_RANDOM_ACCESS_REQ
		send_mode	TWO_BURSTS

- MPHC_STOP_RXLEV_PERIODIC_REQ
		param	NOT_USED

- MPHC_STOP_RXLEV_PERIODIC_CON
		param	NOT_USED

- MPHC_STOP_NCELL_SYNC_REQ
		radio_freq_array_size	STOP_SIZE_12
		radio_freq_array	STOP_ARRAY_EMPTY_12

- MPHC_STOP_NCELL_SYNC_CON
		param	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
		param	NOT_USED

- MPHC_STOP_SCELL_BCCH_CON
		param	NOT_USED

- MPHC_SCELL_NBCCH_REQ
		schedule_array_size	SCHED_SIZE_1
		schedule_array	NOT_USED

- MPHC_RA_REQ
		txpwr	POWER_12
		rand	RAND_BURST_1
		channel_request	CHANNEL_REQUEST_1
		powerclass_gsm	POWER_CLASS_4
		powerclass_dcs	POW_CLASS_2

- MPHC_RA_CON
		fn	FN_BURST_1
		channel_request	CHANNEL_REQUEST_1

- MPH_RANDOM_ACCESS_CNF
		frame_no	T123_BURST_1

- MPHC_RA_REQ
		txpwr	POWER_12
		rand	RAND_BURST_2
		channel_request	CHANNEL_REQUEST_2
		powerclass_gsm	POWER_CLASS_4
		powerclass_dcs	POW_CLASS_2

- MPHC_DATA_IND
		radio_freq	ARFCN_23
		l2_channel	L2_CHANNEL_CCCH
		error_flag	VALID_BLOCK
		l2_frame	L2_IMM_ASS_HOP
		tc	TC_0
		ccch_lev	NOT_USED
		fn	FN_OFFSET_0

- MPH_UNITDATA_IND
		arfcn	ARFCN_23
		fn	NOT_USED
		sdu	
		{
		component	RR
		direction	DOWNLINK
		pd	D_IMM_ASSIGN
		ti	TI_0
		tma	TMA_0
		dl	DL_0
		d_t	D_T_DED
		page_mode	PAGE_MODE_1
		chan_desc	CHAN_DESC_1
		pck_chan_desc	NOT_USED
		req_ref	REQ_REF_1
		time_advance	TIME_ADVANCE_1
		mob_alloc	MOB_ALLOC_1
		}

- MPH_DEDICATED_REQ
		mod	MODE_IMM_ASSIGN
		start	STARTING_TIME
		ch_type	CH_TYPE_HOP
		ch_type2	CH_TYPE2
		arfcn	ARFCN_23
		bsic	BSIC_1
		ho_param	HO_PARAM
		tr_para	TR_PARAM
		ciph	CIPH_PARAM
		amr_conf	NOT_USED

- MPHC_STOP_SCELL_BCCH_REQ
 		param	NOT_USED

- MPHC_STOP_CCCH_REQ
		param	NOT_USED

- MPHC_STOP_RA_REQ
		param	NOT_USED

- MPHC_IMMED_ASSIGN_REQ
		channel_desc	CHANNEL_DESC_1
		timing_advance	TIMING_ADVANCE
		frequency_list	FREQ_LIST
		starting_time	S_TIME_MPHC
		frequency_list_bef_sti	NOT_USED
		maio_bef_sti	NOT_USED
		dtx_allowed	NOT_USED
		bcch_allocation	NOT_USED
		ba_id	NOT_USED
		pwrc	NOT_USED

- MPHC_IMMED_ASSIGN_CON
		param	NOT_USED

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

History:		26 September 2002	DL	Initial

### ALR972: E-OTD measurement started (DEDICATED Mode)

Description:	RR requests E-OTD measurement of a list of cells forwarded in the MPH-NCELL-POS request primitive, whereupon ALR forwards this list to Layer 1 (MPHC-NCELL-LIST-SYNC request primitive).
Note: in Variant A all cells in the list are current neighbour cells and are thus known to ALR, whereas Variant B contains two cell (arfcn 516 and 525 respectively) which are unknown.

Variants:	<A>....<B>

Preamble:	ALR970

RR		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_NCELL_POS_REQ	  | 		  |
	 *==============================>*		  |
(2)	 |		  |	MPHC_NCELL_LIST_SYNC_REQ	  |
	 |		  *==============================>*
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_NCELL_POS_REQ
<A>	req_id	REQ_ID_1
<B>	req_id	REQ_ID_8
<A>	ncell_eotd	NCELL_EOTD_7
<B>	ncell_eotd	NCELL_EOTD_9

- MPHC_NCELL_LIST_SYNC_REQ
		eotd	EOTD_MON
<A>	list_size	NO_NCELLS_6
<B>	list_size	NO_NCELLS_8
<A>	ncell_list	NCELL_LIST_6_EOTD_DEDI
<B>	ncell_list	NCELL_LIST_8_EOTD_DEDI

History:		26 September 2002	DL	Initial

### ALR974: E-OTD measurement halted due to commencement of Handover

Description:	Following commencement of handover (receipt of MPH-DEDICATED request primitive), ALR immediately requests Layer 1 to cease E-OTD measurement (MPHC-STOP-NCELL-SYNC request primitive) and proceeds with the handover.

Preamble:	ALR972A

RR		 ALR		 PL
	 |		  |		  |
(1)	 |	MPH_DEDICATED_REQ	  |		  |
	 *==============================>*		  |
(2)	 |		  |	MPHC_STOP_NCELL_SYNC_REQ	  |
	 |		  *=============================>*
(3)	 |		  |	MPHC_ASYNC_HO_REQ	  |
	 |		  *=============================>*
(4)	 |		  |	MPHC_ASYNC_HO_CON	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_HANDOVER_FINISHED	  |
	 |		  *<=============================*
(6)	 |	MPH_DEDICATED_CNF	  | 		  |
	 *<==============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPH_DEDICATED_REQ
		mod	MODE_ASYNC_HANDOVER
		start	NOT_USED
		ch_type	CH_TYPE_TCH2
		ch_type2	NOT_USED
		arfcn	ARFCN_14
		bsic	BSIC_1
		ho_param	HO_PARAM_1
		tr_para	TR_PARAM
		ciph	CIPH_PARAM
		amr_conf	NOT_USED

- MPHC_STOP_NCELL_SYNC_REQ
		radio_freq_array_size	STOP_SIZE_12
		radio_freq_array	STOP_ARRAY_EMPTY_12

- MPHC_ASYNC_HO_REQ
		handover_command	ASYNC_HO_CMD
		fn_offset	NOT_USED
		time_alignmnt	NOT_USED
		cipher_key	NOT_USED
		amr_configuration	NOT_USED

- MPHC_ASYNC_HO_CON
		param	NOT_USED

- MPHC_HANDOVER_FINISHED
		cause	HO_COMPLETE

- MPH_DEDICATED_CNF
		dedi_res	DEDI_RES_OK

History:		26 September 2002	DL	Initial

### ALR976: E-OTD Measurement performed  (DEDICATED Mode)

Description:	On confirmation from Layer 1 that sychronization of neighbour cells has ceased (reception of MPHC-STOP-NCELL-SYNC confirmation primitive) ALR forwards a list of the neighbour cells selected for E-OTD measurement to Layer 1 (MPHC-NCELL-LIST-SYNC request primitive). Layer 1 subsequently forwards the measurements for the serving cell and the neighbour cells requested (MPHC-NCELL-SYNC indication primitive). The process is complete when a second measurement for the serving cell is received, whereupon ALR forwards the measurement results to RR (MPH-NCELL-POS indication primitive).

Preamble:	ALR972A

RR		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 23 - SC)	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 23 - SC)	  |
	 |		  *<=============================*
(9)	 |	MPH_NCELL_POS_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_23
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_918
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_12
		time_tag	nom_pos_1000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_637
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61952
		a_eotd_crosscor	eotd_crosscor_9
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_25
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_6
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61952
		a_eotd_crosscor	eotd_crosscor_9
		time_tag	nom_pos_1000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_512
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_6
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_580
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61952
		a_eotd_crosscor	eotd_crosscor_9
		time_tag	nom_pos_1000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_885
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_2
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_6
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_23
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_918
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_12
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPH_NCELL_POS_IND
		req_id	REQ_ID_1
		eotd_res	EOTD_SUCC
		ta	NOT_USED
		fn	FN_OFFSET_0
		eotd_sc_res	eotd_sc_res_ok
		eotd_sc_res1	eotd_sc_res1_ok
		eotd_nc_res	EOTD_RESULT_6

History:		26 September 2002	DL	Initial

### ALR978: E-OTD Measurement performed – unknown ARFCNs (DEDICATED Mode)

Description:	On confirmation from Layer 1 that sychronization of neighbour cells has ceased (reception of MPHC-STOP-NCELL-SYNC confirmation primitive) ALR forwards a list of the neighbour cells selected for E-OTD measurement to Layer 1 (MPHC-NCELL-LIST-SYNC request primitive). Layer 1 subsequently forwards the measurements for the serving cell and the neighbour cells requested (MPHC-NCELL-SYNC indication primitive). The process is complete when a second measurement for the serving cell is received, whereupon ALR forwards the measurement results to RR (MPH-NCELL-POS indication primitive).

Preamble:	ALR972B

RR		 ALR		 PL
	 |		  |		  |
(1)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 23 - SC)	  |
	 |		  *<=============================*
(2)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 637)	  |
	 |		  *<=============================*
(3)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 25)	  |
	 |		  *<=============================*
(4)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 14)	  |
	 |		  *<=============================*
(5)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 512)	  |
	 |		  *<=============================*
(6)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 580)	  |
	 |		  *<=============================*
(7)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 885)	  |
	 |		  *<=============================*
(8)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 516)	  |
	 |		  *<=============================*
(9)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 525)	  |
	 |		  *<=============================*
(10)	 |		  |	MPHC_NCELL_SYNC_IND	  |
	 |		  |	(ARFCN 23 - SC)	  |
	 |		  *<=============================*
(11)	 |	MPH_NCELL_POS_IND	  |		  |
	 *<===============================*		  |
	 |		  |		  |

Parametrization

- Primitive	Parameter	Value

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_23
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_918
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_12
		time_tag	nom_pos_1000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_637
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61952
		a_eotd_crosscor	eotd_crosscor_9
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_25
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_6
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_14
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61952
		a_eotd_crosscor	eotd_crosscor_9
		time_tag	nom_pos_1000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_512
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_6
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_580
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61952
		a_eotd_crosscor	eotd_crosscor_9
		time_tag	nom_pos_1000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_885
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_14
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_2
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_6
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_516
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_102
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_10
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_6
		time_tag	nom_pos_1000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_525
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_114
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_10
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61952
		a_eotd_crosscor	eotd_crosscor_9
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPHC_NCELL_SYNC_IND
		radio_freq	ARFCN_23
		sb_flag	SB_FOUND
		fn_offset	FN_OFFSET_918
		time_alignment	TIME_ALIGNMT_14
		bsic	BSIC_1
		neigh_id	EOTD_00
		attempt	EOTD_00
		pm	EOTD_0L
		toa	EOTD_0L
		angle	EOTD_0L
		snr	EOTD_0L
		eodt_data_valid	EOTD_PRES
		mode	EOTD_00
		d_eotd_first	EOTD_0000
		d_eotd_max	EOTD_0000
		d_eotd_nrj	RSSI_61440
		a_eotd_crosscor	eotd_crosscor_12
		time_tag	nom_pos_2000
		fn_sb_neigh	EOTD_0L
		fn_in_sb	EOTD_0L
		delta_fn	EOTD_0L
		delta_qbit	EOTD_0L

- MPH_NCELL_POS_IND
		req_id	REQ_ID_8
		eotd_res	EOTD_SUCC
		ta	NOT_USED
		fn	FN_OFFSET_0
		eotd_sc_res	eotd_sc_res_ok
		eotd_sc_res1	eotd_sc_res1_ok
		eotd_nc_res	EOTD_RESULT_8

History:		25 October 2002	DL	Initial

(Provide a short description of this document.)

Appendices

  - Acronyms

This is just an example!:

| DS-WCDMA | Direct Sequence/Spread Wideband Code Division Multiple Access |
| --- | --- |

  - Glossary

This is just an example!:

| International Mobile Telecommunication 2000 (IMT-2000/ITU-2000) | Formerly referred to as FPLMTS (Future Public Land-Mobile Telephone System), this is the ITU's specification/family of standards for 3G. This initiative provides a global infrastructure through both satellite and terrestrial systems, for fixed and mobile phone users. The family of standards is a framework comprising a mix/blend of systems providing global roaming. <URL: http://www.imt-2000.org/> |
| --- | --- |